ISO/IEC JTC 1/SC 29/WG 11 	N19370

ISO/IEC JTC 1/SC 29/WG 11

Coding of moving pictures and audio

Convenorship: UNI (Italy)

Document type: 	Approved WG 11 document

Title: 	Working Draft 1 of Conformance Testing for Visual Volumetric Video-based Coding (V3C) with Video-based Point Cloud Compression (V-PCC)

Status:	Draft

Date of document:	2020-04-24

Source: 	3DG

Expected action:

No. of pages: 	14

Email of convenor: leonardo@chiariglione.org

Committee URL: mpeg.chiariglione.org

ISO/IEC 23090-20:2020(E)

ISO/IEC JTC 1/SC 29/WG 11

Secretariat: JISC

Information technology — Coded Representation of Immersive Media — Part 20: Conformance Testing for Visual Volumetric Video-based Coding (V3C) with Video-based Point Cloud Compression (V-PCC)

WD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO/IEC 2020

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Fax: +41 22 749 09 47

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	v

Introduction	vi

1	Scope	1

2	Normative reference	1

2.1	General	1

3	Terms and definitions	1

4	Abbreviations	2

5	Conventions	2

6	Conformance for ISO/IEC 23090-5	2

6.1	Introduction	2

6.2	Bitstream conformance	3

6.3	Decoder conformance	3

6.4	Reconstruction conformance	3

6.5	Procedure to test bitstreams	4

6.6	Procedure to test decoder conformance	5

6.6.1	Conformance bitstreams	5

6.6.2	Contents of the bitstream file	5

6.6.3	Checksum computations for atlas bitstream conformance testing	6

6.6.4	Requirements on output of the decoding process, reconstruction process, and timing	7

6.6.5	Bitstream validation	7

6.6.6	Recommendations (informative)	7

6.7	Specification of the test bitstreams	7

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Subcommittee 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO/IEC 23090 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

Advances in 3D capturing and rendering technologies have unleashed a new wave of innovation in Virtual/Augmented/Mixed reality (VR/AR/MR) content creation and communication. Point clouds have arisen as one of the main representations for such applications. A point cloud frame consists of a set of individual 3D points. Each point, in addition to having a 3D position, i.e., spatial attribute, may also be associated with a number of other attributes such as colour, reflectance, surface normal, etc. A point cloud consists of a sequence of point cloud frames. The number of points, their positions, and their attributes may vary from one frame to another. Such representations require a large amount of data, which can be costly in terms of storage and transmission. Therefore, the ISO/IEC Moving Picture Experts Group (MPEG) developed a new International Standard, which aims at efficiently compressing point cloud representations.

Information technology — Coded Representation of Immersive Media — Part 20: Conformance Testing for Video-based Point Cloud Compression (V-PCC)

# Scope

This International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in ISO/IEC 23090-5.

# Normative reference

# 2.1	General

The following International Standards contain provisions which, through reference in this text, constitute provisions of this International Standard. At the time of publication, the editions indicated were valid. All Standards are subject to revision, and parties to agreements based on this International Standard are encouraged to investigate the possibility of applying the most recent edition of the Standards listed below. Members of ISO maintain registers of currently valid International Standards.

# Terms and definitions

For the purposes of this International Standard, the terms, definitions, abbreviations and symbols specified in ISO/IEC 23090-5. apply. The following terms are further clarified for purposes herein as follows.

bitstream

a ISO/IEC 23090-5 V3C bitstream.

decoder

a ISO/IEC DIS 23090-5 V3C decoder, i.e., an embodiment of the decoding process specified by ISO/IEC 23090-5. The decoder does not include the display process, which is outside the scope of this  International Standard.

encoder

an embodiment of a process, not specified in this International Standard (except in respect to identification of the reference software encoder), that produces a bitstream.

bitstream conformance

An embodiment of a process specified in annex E.4 of ISO/IEC 23090-5.

decoder conformance

an embodiment of a process specified in annex E.5 of ISO/IEC 23090-5.

reference software decoder

a software decoder provided in ISO/IEC 23090-19.

reference software encoder

a software encoder provided in ISO/IEC 23090-19.

V3C component

atlas, occupancy map, geometry, or attribute if present, of a particular type that is associated with a V3C V-PCC volumetric content representation.

[Ed. (VZ/ATA/DBG): it is a minimum requirement that the atlas component of the V3C bitstream is present for conformance testing]

V-PCC reconstruction

an ISO/IEC DIS 23090-5 V3C V-PCC content, i.e., a recommendation of the reconstruction process specified by Annex H in ISO/IEC 23090-5.

# Abbreviations

For the purposes of this International Standard, relevant abbreviations and acronyms are specified in clause 4 of ISO/IEC 23090-5.

# Conventions

For the purposes of this International Standard, relevant conventions are specified in clause 5 of ISO/IEC 23090-5.

# Conformance for ISO/IEC 23090-5

## Introduction

The following clauses specify normative tests for verifying the conformance of V3C bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this International Standard and the reference software decoder specified in ISO/IEC 23090-5.

-	Define conformance testing procedure for conformance point A and soft conformance testing procedure for conformance point B.

[Ed. (VZ/ATA/DBG): Figure 1 indicates the conformance points A and B interpretations]

[Ed. (VZ/ATA/DBG): The conformance points A and B are specified by Annex A.2 of ISO/IEC 23090-5, in addition at the conformance point A the block to volume information may be reconstructed in a form of volumetric boxes that corresponds to the block to patch information].

-	Compile a list of the type of conformance tests that need to be performed.

-	Set up a repository for conformance bitstream exchange.

Note. It is a requirement of this specification that the checksums calculated independently for the Tiles and the Atlas frame.

[Ed. (VZ/ATA/DBG): During the projection process from 2D into 3D information the 2D tiles shall be used, the 3D tile definition mechanism seems to be redundant and can be handled by the encoder, in addition the Volumetric Annotation SEI message defines the 3d bounding box].

## Bitstream conformance

The bitstream conformance of ISO/IEC 23090-5 is specified by clause E.4 of ISO/IEC 23090-5.

For conducting conformance testing, we should define and elaborate on the followings:

-	Bitstream conformance testing at the conformance point A.

-	Decoder conformance at conformance point A and soft conformance at the conformance point B.

-	Bitstreams generation and verification

## Decoder conformance

The decoder conformance of ISO/IEC 23090-5 is specified by clause E.5 of ISO/IEC 23090-5.

## Reconstruction conformance

The reconstruction conformance of ISO/IEC 23090-5 is specified by clause H.9 of ISO/IEC 23090-5.

The V3C reconstruction decoder for the V-PCC conformance for components as frame rate normalization, resolution conversion, chroma format conversion and attribute transfer processes are specified by annex B of ISO/IEC 23090-5.

The decoded synchronization conformance for the decoded V3C components of ISO/IEC 23090-5 is specified by Annex B of ISO/IEC 23090-5.

[Ed. (VZ/ATA/DBG): the synchronization of the components is not yet defined in the ISO/IEC 23090-5]

It is the recommendation of this specification that the decoded V3C output units are stored in an intermediate buffer, for reconstruction purposes. The V3C output units may be placed into the intermediate buffer at the output time of the corresponding V3C output unit processing, and the reconstruction process can start at the time when all required V3C output units are available for processing.

Figure 1 – V3C bitstream conformance evaluation for V-PCC content

## Procedure to test bitstreams

A bitstream that claims conformance with ISO/IEC 23090-5 shall pass the following normative test:

The bitstream shall be decoded by processing it with the decoder. When processed by the decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to ISO/IEC 23090-5.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in ISO/IEC 23090-5 including the hypothetical reference decoder (HRD) conformance (based on Annexes E	, F and G). These complementary tests may be performed using other point cloud bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

[Ed. (VZ): This maps to 23090-5 annexes E – HRD, F – SEI, G – VUI]

ISO/IEC 23090-5 contains several informative recommendations that are not an integral part of that International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check the correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in ISO/IEC 23090-5.

A verifier may not necessarily perform all stages of the decoding process specified in ISO/IEC 23090-5 to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream has values of ptl_profile_codec_group_idc and ptl_profile_pcc_toolset_idc, and ptl_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile and level is required in Annex A of ISO/IEC 23090-5 to properly perform the decoding process.

### Contents of the bitstream file

The conformance bitstreams are included in this International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

- short description of the bitstream. A “textual” description could, for example, be: “All tiles are coded as I_TILE. Each atlas frame contains one tile”, and a “functional stage such as “Test the reconstruction process of I_TILE”.

- output picture log (*.opl) file which contains, one row, for each output picture (e.g. occupancy, geometry and attribute, if present) in the bitstream, in output order:

  - PicOrderCntVal;

  - pic_width_max_in_luma_samples;

  - pic_height_max_in_luma_samples;

  - MD5 checksum for the i-th colour plane.

- a trace file to show the generated results during the bitstream decoding process, an output atlas frame.

[Ed. (DBG): what is the content of this trace file? Would it dump the values of the patches? Should we specify a certain expected format? What is the output atlas frame, and how do we distinguish this from the output tile group?]

[Ed. (VZ): We have a trace functionality in the TMC2 software, it contains most of the required information, MD5 checksums can be added on top of this functionality]

- An output tile log (*.otl) file which contains, one row for each output tile in the bitstream, in output order. Each row contains the following information, as CAS.

  - AtlasFrameOrderCntVal

  - TopLeftTileColumn

  - TopLeftTileRow

  - tile_width_max_in_luma_samples (BottomRightTileCoulmn)

  - tile_height_max_in_luma_samples (BottomRightTileRow)

  - MD5 checksum for decoded tile group

[Ed. (DBG): How to calculate the MD5 for the decoded tile group?]

[Ed. (VZ): I suggest we use the text description of the decoded tile in a text format for the patch, the patches are in the precedence order, we also need to explicitly indicate the origin of the tile within an atlas frame.]

- An output atlas log (*.ofl) file which contains, a number of rows for each output tile group in the atlas frame in the bitstream, in output order. Each row contains the following information, as CAS.

  - AtlasFrameOrderCntVal

  - atlas_frame_width_max_in_luma_samples

  - atlas_frame_height_max_in_luma_samples

  - MD5 checksum for decoded atlas frame

  - MD5 checksum for the derived block to patch per atlas frame

  - MD5 checksum for the patch to volume per atlas frame

- An output point cloud frame log (*.opcl) file which contains, one row for each output point cloud frame in the bitstream, in output order. Each row contains the following information, as CAS.

  - PointCloudFrameOrderCntVal

  - MD5 checksum MD5 for decoded point cloud frame provided.

In cases where the decoded pictures or hashes of decoded pictures are not available, the reference software decoder shall be used to generate the necessary reference decoded pictures from the bitstream.

Table 1 – List of the files for conformance testing

| Item | Extension | Description | Requirement |
| --- | --- | --- | --- |
| 1 | *.bit | coded bitstream | mandatory |
| 2 | *.txt | a short description of the bitstream | mandatory |
| 3 | *.trc | trace file | optional |
| 4 | *.cfg | the configuration file used to generate bitstream with a software encoder provided in ISO/IEC 23090-20. [Ed. (DBG): we usually use multiple configurations, how to handle this?] | mandatory |
| 5 | *.md5 | MD5 checksum of the bitstream file | mandatory |
| 6 | *.opl | output picture log for occupancy map, geometry and attribute (depends on input configuration). | optional |
| 7 | *_atl.ofl | output atlas log | mandatory* |
| 8 | *_atl.otl | output tile group log | mandatory* |
| 9 | *.opcl | output point cloud frame log | mandatory |
| 10 | *_dec.vid | decoded image for occupancy map, geometry and attribute (depends on input configuration). | optional |
| 11 | *_dec.atl | decoded atlas frame | mandatory |
| 12 | *_dec.tig | decoded tile group | optional |
| 13 | *_dec.ply | unordered reconstructed decoded point cloud frames. The decoded point cloud does not necessarily need to be ordered. However, for performing conformance test voxels in the point cloud need to be sorted in a certain order (e.g. using a generalized Morton based order sorting). | soft/optional |

* it is defined by the particular test if the atlas frame of the collection for the separate tile checksums are required

[Ed. (VZ/ATA/DBG): Use annex B for reconstruction process. Rec0. What should we follow for the reconstruction and to generate the MD5 checksum for the decoded point cloud frame ply?]

### Checksum computations for atlas bitstream conformance testing

Conformance test needs to be done directly on the patch information, and by processing the patch information in tile and patch coding order. That, as well as comparing essential high-level syntax information that allows appropriate interpretation of the tile and patch information (e.g. tile position within the atlas, use of interleaving, etc.)  In case of specifying a hash SEI message that could be used for the verification of an atlas decoder, such hash value should be computed by going through all elements associated with each patch, e.g. for non EOM and RAW patches the variables Patch2dPosX[ p ], Patch2dPosY[ p ], Patch2dSizeX[ p ], Patch2dSizeY[ p ], Patch3dPosX[ p ], Patch3dPosY[ p ], Patch3dPosMinZ[ p ], PatchOrientationIndex[ p ], PatchLoDScaleX[ p ], and PatchLoDScaleY[ p ], in a prespecified order. Patches should be processed in tile followed by patch coding order. Adding onto the hash computation HLS information essential for reconstruction, e.g. position of each tile on the atlas.

### Requirements on output of the decoding process, reconstruction process, and timing

The output of the decoding process is specified in clauses 8, 9, and Annexes E, G, and H of ISO/IEC 23090-5. The input to this process is specified in the Annex B of ISO/IEC 23090-5.

For output order conformance, it is a requirement that all of the decoded V3C component frames (point cloud frames) specified for output in Annex E of ISO/IEC 23090-5 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of ISO/IEC 23090-5.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex E of ISO/IEC 23090-5.

For the reconstruction conformance, it is a requirement that all of the decoded components are normalized to the nominal frame rate as specified in Annex B of ISO/IEC 23090-5.

The display process, which ordinarily follows the output of the reconstruction process, is outside the scope of this International Standard.

### Bitstream validation

The uploaded bitstreams should be validated and verified by at least two organizations and the following aspects should be verified.

- Confirm if the bitstream is decoded with the perfect match

[Ed. (ATA): Shall we be more explicit as to what do we mean by “exact match”?]

- Confirm if all intended features are included in the bitstream

The output for the conformance point A, comprised of the V3C components that form a point cloud frame at a given time instance are used as an input for the decoder reconstruction process that specifies soft conformance check for point B.

### Recommendations (informative)

This clause does not form an integral part of this International Standard.

In addition to the requirements, conforming decoders should implement various informative recommendations specified in ISO/IEC 23090-5 that are not an integral part of that International Standard. This clause lists some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases, it is possible to resume decoding at the next V3C slice unit header. It is recommended that a conforming decoder be able to perform concealment for the patches or V3C packets for which all the coded data of the V3C components has not been received.

## Specification of the test bitstreams

Some characteristics of each bitstream listed in Tables 1 are specified in this clause.

The following naming convention for zip file shall be used:

- CodecGroup_Toolset_Feature_Reconstruction_Source_Version

- CodecGroup refers to a candidate codecGroup from table A-1 of V3C specification.

- Toolset refers to a candidate toolset from Table A-3 and A-4 of V3C specification

- The feature is summarized in Table 1.

- Reconstruction refers to the reconstructions from Table A-5 of V3C specification.

- The source is the name of the volunteer organization that provided the bitstream.

| Categories | Tool description | Feature Name | Bitstream features | Submitter |
| --- | --- | --- | --- | --- |
| Patch type | Inter patch type | INTERP | Test inter patch |  |
|  | Intra patch type | INTRAP | Test intra patch |  |
| Tile structure | Uniform tile | UT | Test uniform tile structure |  |
| Essential SEIs, type A | Type A Essential SEI messages | ESEIA |  |  |

- Version is the version number of the file, in case a bitstream must be replaced because of an identified problem (starting from 1).

[Ed (VZ/ATA/DBG): come up with a table to test Toolset. Example: tiles, intra tiles, uniform tiles, could be a very big table. Provide some fundamental things (categories) that need to be tested, and request volunteers to provide different types of bitstreams]

Basic features can be combines into bitstreams to reduce the number of conformances bitstreams.

Two volunteers for each category is desirable. At least one person should be a volunteer to generate bitstream from a company.

This International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The bitstreams can also be downloaded from the ITU-T Test Signal Database.

While the reference software decoder project is in development, the Test Model shall be used in place of the reference software decoder, the MD5 checksum of the V3C output units shall be used for the conformance testing.

It is also similar for EOM/RAW patches depending also on whether these are related to the regular or the auxiliary streams. The example is just for illustration purposes