ISO/IEC JTC 1/SC 29/WG 03 N0253

ISO/IEC JTC 1/SC 29/WG 03

MPEG Systems 
Convenorship: KATS (Korea, Republic of)

ISO/IEC JTC 1/SC 29/WG 03

MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	WD of ISO/IEC 23000-19:2019 AMD 3 8K HEVC, 4K HFR HEVC and Chroma Location for CMAF

Status:	Approved

Date of document:	2021-05-27

Source:	ISO/IEC JTC 1/SC 29/WG 03

Expected action:	ACT

Action due date:

No. of pages:	10 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL: https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N0253

April 2021, Virtual

| Title | WD of ISO/IEC 23000-19:2019 AMD 3 8K HEVC, 4K HFR HEVC and Chroma Location for CMAF |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 20315 |

ISO 23000-19:####(X)

ISO/IEC JTC1 /SC 29 /WG 03 /N0224

Secretariat: XXXX

Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media, AMENDMENT 3: 8K HEVC, 4K HFR HEVC and Chroma Location for CMAF

WD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO 2020

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO's member body in the country of the requester.

ISO copyright office

Case postale 56  CH-1211 Geneva 20

Tel.  + 41 22 749 01 11

Fax  + 41 22 749 09 47

E-mail  copyright@iso.org

Web  www.iso.org

Published in Switzerland.

# Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2.  www.iso.org/directives

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received.  www.iso.org/patents

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation on the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the WTO principles in the Technical Barriers to Trade (TBT) see the following URL: Foreword - Supplementary information

The committee responsible for this document is ISO/IEC JTC1 SC29.

# Introduction to Amendment 2

This amendment adds support for

- Media Profile for 8K HEVC,

- Media Profile for 4K HFR HEVC,

- HEVC Clarifications on Chroma Sample Location

This amendment also addresses several clarifcations and improvements.

Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media, AMENDMENT 3: 8K HEVC, 4K HFR HEVC and Chroma Location for CMAF

# Change 1: 8K HEVC Media Profile definition

Add 2.	4K-HFR HEVC Media Profile definition

Table B.1 — HEVC video CMAF media profiles

| Media Profile | Codec | Profile | Level | colour_ primaries in VUI | transfer_ characteristics in VUI | matrix_ coefficients in VUI | Max Frame Height | Max Frame Width | Max Frame Rate | CMAF File Brand |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8K10 | HEVC | Main10 MainTier 10-bit | 6.1 | 9 (BT.2100) [Note 5] | 16 (BT.2100 Table 4 PQ EOTF) [Note 3, 6] 18 (BT.2100 Table 5 HLG OETF) [Note 8] 14 (BT.2020 OETF) [Note2] | 9 (BT.2100 Table 6 Y′C′BC′R) [Note 7] | 4320 | 7680 | 60 | 'c8k0' |

Add an extra entry for the UHDTV2 resolution definition to Table C.1 (marked in red)

Table C.1 — Resolution operating point sets

|  | Vertical Size (lines) | Horizontal Size (pixels) | aspect_ ratio_idc (see ISO/IEC 23008-2, Table E.1) | Display Aspect Ratio | Production Format |
| --- | --- | --- | --- | --- | --- |
|  | 4320 | 7680 | 1 | 16:9 | UHDTV2 |
|  | 2160 | 3840 | 1 | 16:9 | UHDTV1 |
|  | 1080 | 1920 | 1 | 16:9 | HDTV |

# Change 2: 4K-HFR HEVC Media Profile definition

## Replace clause B.6

- B.6 High frame rate HD

HEVC Levels 5.0 and 5.1 support greater than 60 Hz, e.g. when the resolution is not at the maximum. These profiles relax the frame rate constraints of Table B.1 profiles “UHD8”, UHD10”, “HDR10”, and “HLG10” to allow what Levels 5.0 and 5.1 permit. The profiles defined in Table B.2 are identical to the corresponding Table B.1 CMAF media profiles, except for the “Max Frame Rate” column in Table B.1. These new profiles can support the maximum resolution or maximum frame rate but not both concurrently due to HEVC level constraints.

Table B.2 — HEVC video CMAF media profiles – High frame rate

| Media Profile | Corresponding Table B.1 Media Profile | Max Frame Rate | CMAF File Brand |
| --- | --- | --- | --- |
| UHD8H | UHD8 | 120 | 'cud9' |
| UHD10H | UHD10 | 120 | 'cud2' |
| HDR10H | HDR10 | 120 | 'chd2' |
| HLG10H | HLG10 | 120 | 'clg2' |
| HLG10H | HLG10 | 120 | 'clg2' |

## With

- B.6 High frame rate

HEVC Levels 5.0,  5.1 and 6.1 support greater than 60 Hz, e.g. when the resolution is not at the maximum. These profiles relax the frame rate constraints of Table B.1 profiles “UHD8”, UHD10”, “HDR10”, “HLG10” and “8K10” to allow what Levels 5.0, 5.1 and 6.1 permit. The profiles defined in Table B.2 are identical to the corresponding Table B.1 CMAF media profiles, except for the “Max Frame Rate” column in Table B.1. These new profiles can support the maximum resolution or maximum frame rate but not both concurrently due to HEVC level constraints.

For a CMAF track to comply with one of the CMAF media profiles in Table B.2, it:

—	shall conform to the profile in column “Corresponding Table B.1 Media Profile” except for the Max Frame Rate indicated in Table B.1.

—	shall not exceed the frame rate listed in Table B.2 “Max Frame Rate” column, even if the HEVC level would permit higher values.

—	should include the CMAF File Brand listed in its CMAF header

NOTE	CMAF tracks conforming to the Media Profile listed in the Second column of Table B.2 automatically conform to the associated media profile listed in the same row in the first column.

Table B.2 — HEVC video CMAF media profiles – High frame rate

| Media Profile | Corresponding Table B.1 Media Profile | Max Frame Rate | CMAF File Brand |
| --- | --- | --- | --- |
| UHD8H | UHD8 | 120 | 'cud9' |
| UHD10H | UHD10 | 120 | 'cud2' |
| HDR10H | HDR10 | 120 | 'chd2' |
| HLG10H | HLG10 | 120 | 'clg2' |
| 8K10H | 8K10 | 120 | 'c8k1' |

# Change 3: Chroma location info in HEVC CMAF Media profiles

## Replace clause B.3.3.4.2

- B.3.3.4.2 Visual Usability Information (VUI) parameters

VUI parameters that occur within a CMAF HEVC track shall conform to ISO/IEC 23008-2 with the following additional constraints.

—	The following fields shall have pre-determined values as follows.

—	aspect_ratio_info_present_flag shall be set to 1.

—	video_full_range_flag shall be set to 0.

—	The following fields have the following values.

—	colour_description_present_flag should be set to 1.

NOTE	As defined in ISO/IEC 23008-2, if the colour_description_present_flag is set to 1, the colour_primaries, transfer_characteristics and matrix_coefficients fields are present in the VUI.

—	If colour_description_present_flag is set to 1, then colour_primaries, transfer_characteristics and matrix_coefficients shall be set to one of the values permitted for the media profile (see Table B.1).

—	If colour_description_present_flag is set to 0, this shall indicate the following values are to be assumed:

—	colour_primaries = 1;

—	transfer_characteristics = 1;

—	matrix_coefficients = 1.

—	overscan_info_present_flag shall be set to 0, therefore overscan_appropriate shall not be present.

—	The values of the following fields shall not change throughout a CMAF track and CMAF switching set.

—	low_delay_hrd_flag

—	colour_description_present_flag

—	colour_primaries, when present

—	transfer_characteristics, when present

—	matrix_coeffs, when present

—	The values of the following fields should not change throughout a CMAF track.

—	vui_time_scale

—	vui_num_units_in_tick

—	If BT.2020 or BT.2100 is signalled by colour_primaries = 9,

—	chroma_loc_info_present_flag should be set to 1, indicating the presence of chroma location types.

—	If chroma_loc_info_present_flag is set to 1 then,

—	chroma_sample_loc_type_top_field shall be set to 2

—	chroma_sample_loc_type_bottom_field shall be set to 2

NOTE	If BT.2020 or BT.2100 is signalled by colour_primaries = 9 and the chroma_loc_info_present_flag is set to 0 then receivers are expected to rely on the BT.2020 and BT.2100 definitions. For more information, refers to the HEVC specficiation clause E.X.