# ISO/IEC JTC 1/SC 29/WG 03 N0333

ISO/IEC JTC 1/SC 29/WG 03

MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Technologies under Consideration for Dynamic Adaptive Streaming over HTTP 23009, Parts 1, 3, 4, 5, 6 and 8

Status:	Approved

Date of document:	2021-07-23

Source:	ISO/IEC JTC 1/SC 29/WG 03

Expected Action:	ACT

Action due date:	2021-07-23

No. of pages:	112 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N 0333

July 2021, Virtual

| Source | MPEG Systems WG |
| --- | --- |
| Status | Output |
| Title | Technologies under Consideration for Dynamic Adaptive Streaming over HTTP 23009, Parts 1, 3, 4, 5, 6 and 8 |
| Editor | Iraj Sodagar |
| Serial Number | 20587 |

This Technology under Consideration document collects candidate technologies for inclusion into new amendments of ISO/IEC 23009 Dynamic Adaptive Streaming over HTTP.

Note that any section that is not indicated otherwise relates to Part 1.

- Contents

ISO/IEC JTC 1/SC 29/WG 03 N0333	1

1	Annotation and Client Model for Content Selection	8

1.1	Introduction	8

1.2	Proposed for Part 3: Adaptation Set Labeling Options for Selection	8

1.3	Proposed for Part 1: Content Model	15

1.4	Signalling Requirements and Recommendations	17

1.4.1	General	17

1.4.2	Alternative Content Signalling	17

1.4.3	Associated Content Signalling	18

1.4.4	Media-type Independent Target Version Annotation	18

1.4.5	Video Target Version Annotation	18

1.4.6	Audio Target Version Annotation	19

1.4.7	Subtitle Target Version Annotation	20

1.4.8	Other Annotation, Auxiliary Data	20

1.5	Client Processing Reference Model	21

1.5.1	Introduction	21

1.5.2	Client Model	21

1.5.3	Generic Processing Model	22

2	Part 5: CORS Considerations for SAND	26

3	Ad insertion	27

3.1	Use-cases	27

3.2	The proposed solutions so far	27

3.3	Discussion at MPEG127th (m48861, m49263, m49607, m49374, m49608)	27

3.4	Discussion at MPEG128th (m50916, m51006, m51021)	27

3.5	Discussion at MPEG129th (based on m52438 )	28

3.6	Proposal: Content and Ad Server Communication (updated with m47557 @MPEG 126)	29

3.6.1	Problem Statement	29

3.6.2	High-level Proposed Solution	32

3.6.3	Targeted Ads in Live Services	33

3.7	Proposal: Flexible Ad insertion with existing tools in TuC and minor additions (updated with m47195 @MPEG126 and m48861@MPEG127)	38

3.7.1	Introduction	38

3.7.2	Client side/Server guided ad insertion architecture	38

3.7.3	Use cases	39

3.7.4	DASH Principle	39

3.7.5	Early Termination of ads	40

3.7.6	New tools needed for standard implementation	41

3.8	Proposal: Content Replacement and Ad Insertion Event (m54500)	44

3.8.1	Introduction	44

3.8.2	Relevant Reference Architecture	44

3.8.3	Content Replacement Event	44

3.9	Proposal: Xlink caching processing model and signaling (m54502)	46

3.9.1	Introduction	46

3.9.2	The most common OnRequest xlink Usecase	46

3.9.3	Deficiency of DASH xlink processing model	47

3.9.4	Proposed solutions in MPEG#130	48

3.9.5	Is the resolution consistency challenge unique to xlink?	49

3.9.6	General local caching model for remote elements	50

3.9.7	HTTP Cache processing model	50

3.9.8	Cache processing model for DASH client	51

3.9.9	Media time vs Wall Clock time	52

3.9.10	Cache model for implicit signaling proposed by other proposals	53

3.9.11	Difficulties with implicit cache signaling	54

3.9.12	Why signaling in the MPD and not in HTTP headers?	54

3.9.13	Proposal	54

4	Adding haptics functionality (m54495- updated at MPEG#132)	56

4.1	Purpose	56

4.2	References	56

4.3	Haptics in MPEG-DASH	57

4.3.1	New Media Component Content Type - ‘haptics’	57

4.3.2	Haptic Channel Configuration	57

4.4	Patent Rights Declaration	58

4.5	Informative Annex A: Mobile Phone to Home Use Case	58

4.6	Informative Annex B: Mobile Phone to Car Use Case	60

4.7	Informative Annex C: Haptics Headphones Use Case	61

4.8	Informative Annex D: Companion Device (ATSC 3.0) Use Case	62

4.9	Informative Annex E: Live streaming of Games Use Case	63

4.10	Informative Annex F: Haptics from Different Perspectives Use Case	64

4.11	Informative Annex G: Haptics on Connected Devices Use Case	65

4.12	Informative Annex H: Haptics-Enabled Ads Use Case	66

4.13	Informative Annex I: Tactile Essence-Based Haptic Track Use Case	66

5	Failover in multi-origin linear deployments (m54725)	67

5.1	Introduction	67

5.2	Proposal	68

5.2.1	Approach	68

5.2.2	Proposed syntax and semantics	68

6	Track Derivation based Server-Side Dynamic Adaptation in DASH (m55222, m56824 and m57449)	70

6.1	Introduction	70

6.2	Derived Visual Tracks and Those for Track Selection and Switching	70

6.3	Track Derivation based Server-Side Dynamic Adaptation in DASH	71

6.4	Track Selection and Switching	74

6.5	Viewport and Viewpoint Selection	75

6.6	Temporal Adaptations for Join Live and Turn-in Fast	76

6.7	Proposal	76

6.8	References	77

6.9	Discussion at MPEG#132 meeting	77

6.10	Discussion at MPEG#134 meeting	77

7	Extensions for Service Description (m56093)	78

7.1	Introduction	78

7.2	Background	78

7.3	Key Issues for DASH	83

7.4	Proposal	84

8	MSE implementation of inband events (m56684)	84

A.1.1	Sample inband event processing using MSE data model	84

A.1.1.1	Process@append  rule	84

A.1.1.2	Dispatch buffer timing model	84

A.1.1.3	Implementation	84

A.1.1.4	Algorithms	86

A.1.1.4.1	Initialization	86

9	Determination and Handling of Duplicate MPD Events(m56503)	88

9.1	Introduction	88

9.2	Use cases	88

9.2.1	Events with unknown duration	88

9.2.2	Joining just before or during an ad break	88

9.2.3	Cancelation and update	89

9.3	Notes from the MPEG#134	89

10	Fast Fine Tune in	89

10.1	Introduction	89

10.2	Minimizing initialization delay in live streaming (initially in m56673, refined in m57429)	89

10.2.1	Abstract	89

10.2.2	Introduction	90

10.2.3	Proposal	91

10.2.4	Simulations	91

10.2.5	Questions and answers	92

10.2.6	Suggestions from the 135th MPEG meeting	92

10.3	Shortening tune-in time (m56798)	93

10.3.1	Introduction	93

10.3.2	Proposal	94

10.4	Potentially finer-grain random access in DASH by using streams with gradual refresh and recovery_point() SEIs (m56891)	95

10.4.1	Summary	96

10.4.2	Gradual refresh and Recovery Point SEIs in MPEG and ITU-T video standards	96

10.4.3	Potential application of this functionality in MPEG-DASH	99

10.4.4	References	101

11	External Stream Representation (ESR) for support of EDRAP based video streaming (initially in m56674, refined in m57430)	102

11.1	Abstract	102

11.2	Introduction	102

11.3	Proposal summary	102

11.4	Proposed spec text changes	103

11.4.1	Definitions	103

11.4.2	Semantics of AdaptationSet element	104

11.4.3	5.3.3.3 XML syntax	105

11.5	Questions/comments and answers/responses	106

11.6	Suggestions from the 135th MPEG meeting	107

12	Clarifying pre-roll signaling for seamless content splicing across MPEG-DASH Periods (m56890)	107

12.1	Introduction	107

12.2	Related contributions	107

12.3	Discussion	108

12.3.1	Pre-roll context	108

12.4	Use-cases	108

12.4.1	Ad insertion using multiple Periods	108

12.4.2	Open-GOPs splicing	109

12.5	Discussions	109

12.6	Conclusion	110

13	Support of picture-in-picture services in DASH (m57431)	110

13.1	Abstract	110

13.2	Introduction	110

13.3	Proposal	111

13.4	Discussions at the 135th MPEG meeting	112

# Annotation and Client Model for Content Selection

## Introduction

Beyond the ability to provide multiple Representations of the same media component in one Adaptation Set, DASH MPDs also provide the functionality to annotate Adaptation Sets, such that clients can typically select at most one Adaptation Set for each media type, based on the encoding and description provided in the MPD. The selection is based on client capabilities, client preferences, user preferences and possibly also interactive signalling with the user. Typically, the signalling and selection is independent of the codec in use. This clause provides requirements and recommendations for labelling Adaptation Sets, if multiple tracks are offered. Note that there may be cases that multiple Representations from different Adaptation Sets per media type are chosen for playback, for example if there is a dependency across Representations. In other cases, a DASH client may be asked to select more than one Adaptation Set per media type based on application decisions.

Multiple Adaptation Sets may be offered to provide the same content in different encodings, for example different codecs; or different source formats, for example one Adaptation Set encoded from a standard dynamic range master and another encoded from a high dynamic range video master. Alternatively, Adaptation Sets may describe different content, for example different languages, or different camera views of the same event that are provided in a synchronized presentation in one MPD.

Proper labelling of Adaptation Sets in MPDs conforming to DASH-IF IOPs is essential in order to enable consistent client implementations. In addition, also a model is needed on how the client makes use of the annotation for a content authors to understand the expected effect of the labelling on playback.

## Proposed for Part 3: Adaptation Set Labeling Options for Selection

DASH in ISO/IEC 23009-1 [1] provides many options for labelling Adaptation Sets. In order to provide more consistency in the context of DASH-IF, Table 2 provides a restricted subset of labels for which DASH-IF IOPS provide interoperability, i.e. on how they are expected to be used by the content authors and how they are expected to be used by clients. The table provides information specific for each media type.

It is expected that DASH clients following the DASH IOPs recognize the descriptors, elements, and attributes as documented in Table 2.

Other organizations may define additional descriptors or elements, as well as processing models for clients.

Table 2 Adaptation Set Attributes and Elements and Usage in DASH-IF IOPs (see ISO/IEC 23009-1 [1])

| Attribute or Element | Use for media type | Detailed Usage in DASH-IF IOPs |
| --- | --- | --- |
| General Attributes and Elements for any media type |  |  |
| @profiles | O | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. If not present, it is inherited from the MPD or Period. This may be used for example to signal extensions for new media profiles in the MPD. At least one of the values defined in Table 1 and Table 2 of this document shall be present, or inferred from MPD or Period higher-level. |
| @group | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. The attribute may be used and shall be different at least for different media type. If present, the value shall be greater than 0. For all Adaptation Sets in the same group, the Group shall be the same. Only one Representation in a Group is intended to be presented at a time. However, two or multiple groups of the same media type may exist, if the content author expects simultaneous presentation of two or more Representation of the same media type. |
| @selectionPriority | OD default=1 | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This attribute should be used to dis-ambiguate Adaptation Sets within one group for selection and expresses the preference of the MPD author on selecting Adaptation Sets for which the DASH client does make a decision otherwise. Examples include two video codecs providing the same content, but one of the two provides higher compression efficiency and is therefore preferred by the MPD author. |
| ContentProtection | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. If this element is present, then the content is protected. If not present, no content protection is applied. For details and usage please refer to clause 7. |
| ###### Label | ###### 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This element enables to provide a textual description of the content. This element should be used if content author expects that clients supports UI for selection. However, this element must not be used as the sole differentiating element as at start-up no user interaction is available. |
| ###### EssentialProperty | ###### 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9 specifies information about the containing element that is considered essential by the Media Presentation author for processing the containing element. The following schemes are expected to be recognized by a DASH-IF client independent of the media type: - http://dashif.org/guidelines/trickmode (see clause 3.2.9) |
| ###### SupplementalProperty |  | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9 specifies information about the containing element that is considered supplemental by the Media Presentation author for processing the containing element. In no case this information is used for differentiation, the information may used by a DASH client for improved operation. The following schemes are expected to be recognized by a DASH-IF client independent of the media type: - urn:mpeg:dash:adaptation-set-switching:2016 (see clause 3.8) - http://dashif.org/guidelines/trickmode (see clause 3.2.9) - urn:mpeg:dash:period-continuity:2015 (see clause 3.2.12) - urn:mpeg:dash:period-connectivity:2015 (see clause 3.2.12) |
| ###### Viewpoint | ###### 0 … N | Provides the ability to indicate that media differentiates by a different ViewPoint. If not present, no view point is assigned and no differentiation is taken. For detailed usage of this descriptor, see below. |
| Attributes and Elements for media type “Video” |  |  |
| @mimeType | M | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. Shall be set to "video/mp4". |
| @codecs | M | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This provides the codec that is used for the Adaptation Set. It expresses the codec that is necessary to playback all Representations in one Adaption Set. The following codecs are expected to be recognized by a DASH-IF client: - Codecs in Table 16 - Codecs in Table 18 |
| @par | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. shall be present, if the display aspect ratio is a differentiating parameter in the MPD. |
| @maxWidth | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. This attribute should be present to express the maximum width in samples after decoder sample cropping of any Representation contained in the Adaptation Set. The value should be the maximum horizontal sample count of any SPS in the contained bitstream. |
| @maxHeight | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. This attribute should be present to express the maximum height in pixel of any Representation contained in the Adaptation Set. The value should be the maximum horizontal sample count of any SPS in the contained bitstream. |
| @maxFrameRate | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. This attribute should be present to express the maximum frame rate, i.e. the maximum value of any entry in the Decoder configuration record of the signaled frame rate, if constant frame rate is provided. contained in the Adaptation Set. |
| @scanType | OD Default: progressive | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. This value is expected to be not present. If present, it is expected to be set to "progressive". |
| EssentialProperty | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. specifies information about the containing element that is considered essential by the Media Presentation author for processing the containing element. The following schemes are expected to be recognized by a DASH-IF client for video: - urn:mpeg:mpegB:cicp:<Parameter> as defined in ISO/IEC 23001-8 Error! Reference source not found. and <Parameter> one of the following: ColourPrimaries, TransferCharacteristics, or MatrixCoefficients |
| Accessibility | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. In DASH-IF IOPs two schemes for accessibility are defined. - the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “video” together with the Accessibility descriptor: - sign - captions - the scheme when CEA-608 is used as defined in clause 6.4.3.3, with @schemeIdURI set to "urn:scte:dash:cc:cea-608:2015" |
| Role | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. In DASH-IF IOPs only the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “video” together with the Role descriptor: - caption - subtitle - main - alternate - supplementary - sign - emergency #### If not present, the role is assumed to be main |
| Rating | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. #### DASH-IF IOPs do not define a Rating scheme. If present, Adaptation Sets using this descriptor may be ignored by the DASH-IF IOP clients. |
| Attributes and Elements for media type “Audio” |  |  |
| @mimeType | M | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. Shall set to "audio/mp4". |
| @codecs | M | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This provides the codec that is used for the Adaptation Set. It expresses the codec that is necessary to playback all Representations in one Adaption Set. The following codecs are expected to be recognized by a DASH-IF client: - Codecs in Table 19 - Codecs in Table 22 - Codecs in Table 23 - Codecs in Table 24 - Codecs in Table 25 - Codecs in Table 26 Note: additional values need to be added with new codecs being added |
| @lang | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. The language should be present. If not present, the language is unknown or no language applies. |
| @audioSamplingRate | O | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This attribute may be present to support output devices that may only be able to render specific values. |
| AudioChannelConfiguration | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. specifies information about the Audio channel configuration.The following schemes are expected to be recognized by a DASH-IF client for audio: - urn:mpeg:dash:23003:3:audio_channel_configuration:2011 as defined in ISO/IEC 23009-1 [1], 5.8.5.4 - urn:mpeg:mpegB:cicp:<Parameter> as defined in ISO/IEC 23001-8 Error! Reference source not found. and <Parameter> one of the following: OutputChannelPosition or ChannelConfiguration - tag:dolby.com,2014:dash:audio_channel_configuration:2011 as defined at http://dashif.org/identifiers/audio-source-data/ Note: Annotation may be different for other codecs and may be updated |
| EssentialProperty | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. specifies information about the containing element that is considered essential by the Media Presentation author for processing the containing element. The following schemes are expected to be recognized by a DASH-IF client for audio: - urn:mpeg:dash:audio-receiver-mix:2014 as defined in ISO/IEC 23009-1, clause 5.8.5.7. |
| Accessibility | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. In DASH-IF IOPs only the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “audio” together with the Accessibility descriptor: - description - enhanced-audio-intelligibility |
| Role | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. In DASH-IF IOPs only the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “audio” together with the Accessibility descriptor: - main - alternate - supplementary - commentary - dub - emergency #### If not present, the role is assumed to be main |
| Rating | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. DASH-IF IOPs do not define a Rating scheme. If present, Adaptation Sets using this descriptor may be ignored by the DASH-IF IOP clients. |
| #### Attributes and Elements for media type “Subtitle” |  |  |
| @mimeType | M | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. Shall set to "application/mp4" or "application/ttml+xml" |
| @codecs | O | See ISO/IEC 23009-1 [1], clause 5.3.7.2 Table 9. This provides the codec that is used for the Adaptation Set. It expresses the codec that is necessary to playback all Representations in one Adaption Set. The following codecs are expected to be recognized by a DASH-IF client: - Codecs in Table 20 Note: more need to be added with new codecs being added. |
| @lang | O | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. The language should be present. If not present, the language is unknown or no language applies. |
| Accessibility | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. In DASH-IF IOPs only the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “subtitle” together with the Accessibility descriptor: - caption - sign |
| Role | 0 … N | See ISO/IEC 23009-1 [1], clause 5.3.3.2 Table 5. In DASH-IF IOPs only the Role scheme as defined by MPEG-DASH should be used as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 The DASH role scheme and with the following values is expected to be recognized by a DASH-IF client for media type “subtitle” together with the Accessibility descriptor: - main - alternate - subtitle - supplementary - commentary - dub - description - emergency #### If not present, the role is assumed to be main |

## Proposed for Part 1: Content Model

In order to support the content author in providing content in a consistent manner, Figure 1 provides a conceptual content model for DASH content in one Period of an MPD. The content may be described by an Asset Identifier as a whole and may contain different media types, video, audio, subtitle and application types. Signalling of media types is out of scope for this section, for details refer to section 3.2.12.

Figure 2 Content Model for DASH Multitrack

Within each media type, the content author may want to offer different alternative content that are time-aligned, but each alternative represents different content. Automatic selection of the alternative content is not expected to be done by the DASH client as the client would not have sufficient information to make such decisions. However, the selection is expected to be done by communication with an application or the user, typically using a user interface appropriate for selection.

However, in the absence of this external communication, or at startup, the DASH client still needs to playback content and therefore benefits from information of what is the default content. Such signalling should be provided by the content author. Such default content is referred to as main content, whereas any content that is not main is referred to as alternative. There may be multiple alternatives which may need to be distinguished. We define main and alternative content. Examples for such are synchronized camera views of one master content. The main camera view is provided as main content, all other views as alternative content.

Furthermore, it may be that content of different media type is linked by the content author, to express that two content of different media type are preferably played together. We define associated content for this purpose. As an example, there may be a main commentator associated with the main camera view, but for a different camera view, a different associated commentary is provided.

In addition to semantical content level differentiation, each alternative content may be prepared with different target versions, based on content preparation properties (downmix, subsampling, translation, suitable for trick mode, etc.), client preferences (decoding or rendering preferences, e.g. codec), client capabilities (DASH profile support, decoding capabilities, rendering capabilities) or user preferences (accessibility, language, etc.). In simple AV playout and in the absence of guidance from an application, a content author expects that the DASH client selects at most one target version for each Group taking into account its capabilities and preferences and the capabilities and preferences of the media subsystem. However, an application may obviously select multiple Groups and playout different video Adaptation Sets to support for example picture-in-picture, multi-angle and so on.

In addition, the content author may also provide priorities for target versions, if the receivers support multiple of those. Typical examples are that the content is prepared for H.264/AVC and H.265/HEVC capable receivers, and the content author prefers the selection of the H.265/HEVC version as its distribution is more efficient. A device supporting both decoders may then choose the one with higher priority signalled by the content author. In a similar version, the same content may be provided in different languages. In this case, it can still be expected that the language can be automatically selected by the client, so it is assigned to a target version. Again, a content author may express priorities on languages, for example preferring the native language over a dubbed one. Languages may be considered as alternative content as well, but as long as automatic selection can be provided, it may be considered as different target versions. Hence for each content of one media type, different target versions may exist and the annotation of the content expressed that it is expected that automated selection can be done. Each target version is preferably accumulated in one Adaptation Set, with exceptions such as scalable codecs.

Finally, in the content model, each of the target version typically has multiple Representations that are prepared to enable dynamic switching. This aspect is outside the scope of this section as switching by the client is expected to be done independent of the media type as well as the target version, primarily using the bandwidth and possibly abstract quality information. However, the signalling on the target versions may provide information on how to distribute the available bitrate across different media types.

Based on this content model and the available elements, attributes and descriptors from Table 2, requirements and recommendations are provided for Adaptation Set Signalling to address main and alternative content, associated content as well as different target versions. Based on the signalling, a client decision model is developed that may serve a content provider as a reference client to test if the annotation provided in the MPD provides the proper results.

## Signalling Requirements and Recommendations

### General

Assuming the content author can map its content to the above content model, this section provides signalling requirements and recommendations for such content, such that the content author can expect proper playback of its content for DASH-IF IOP clients

In general, if multiple Adaptation Sets for one media types are provided, sufficient information should be provided such that a DASH client make proper selections, either automatically communicating with it s platform or in communication with the application/user.

### Alternative Content Signalling

If a Period contains alternative content for one media type, then the alternatives should be differentiated. In addition, one of the alternatives should be provided as main content. The main content is intended to be selected by the client in the absence of any other information, e.g. at startup or if the annotation of the content cannot be used.

Main content is signaled by using the Role descriptor with Role scheme as defined by MPEG-DASH in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with value set to main. Alternative content is signaled by using the Role descriptor with Role scheme as defined by MPEG-DASH in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with value set to alternative. If an Adaptation Set does not include either of the two signals, it is assumed to be main content.

The alternative content may be selected by the client, if the client does have the capability to select alternatives, typically by either communicating with the application or with the user. If main and alternative content is provided in the Media Presentation, then alternative content shall be signaled by at least one of the two:

- a ViewPoint descriptor. If  ViewPoint is used for differentiation, then at least each alternative Adaptation Set of the same media type shall include a ViewPoint with the same value for @schemeIdURI. The content is differentiated by different values for the @value attribute in the descriptor for different content.

- a Label element. If Label is used for differentiation, then at least each alternative Adaptation Set shall include a Label with the same value for @id. The content is differentiated by different values for the Label element.

A ViewPoint descriptor is typically used if a target application (identified by the the value for @schemeIdURI) is expected that can make use of the values in the ViewPoint descriptor. A Label element is typically used if the DASH client can provide a user interaction.

### Associated Content Signalling

For associated content of different media types, the ViewPoint descriptor is used. If different media types all belong to one alternative content, they share the same View Point descriptor, i.e. the same value for @schemeIdURI and for @value. Note also that even if the DASH client does not understand the value for @schemeIdURI it would stll obey the rules for associated selection. The DASH client may for example use the labels of different video alternatives for selection, and play the audio according to ViewPoint association.

### Media-type Independent Target Version Annotation

Adaptation Sets within one media type and alternative content shall differ by at least by one of the following annotation labels

- @profiles,

- ContentProtection  (need to provide some details on what the options are: present, not-present, different schemes)  work with content protection task force

- EssentialProperty (not-present, trickmode, a media type specific value, unknown value, which may be extended)

- Any of those documented in section 3.10.4.5 for media type video, section 3.10.4.6 for media type audio and 3.10.4.7 for media type subtitle.

Adaptation Sets with elements EssentialProperty not using any of the permitted values in this document should not be present.

In addition, Adaptation Sets within one media type and alternative content should differ by different values of @selectionPriority. If not present or non-differentiating values are provided, then the content author should expect a random selection of Adaptation Sets in case it is able to handle multiple Adaptation Sets within one media type and alternative content.

### Video Target Version Annotation

Video Adaptation Sets of one alternative content shall differ by at least by one of the following annotation labels:

- @codecs: specifies the codecs present within the Representation. The codecs parameters shall also include the profile and level information where applicable.

- @maxWidth and @maxHeight specifies the horizontal and vertical visual presentation size of the video media type

- @maxFrameRate specifies the maximum frame rate of the video media type

- EssentialProperty: specifies information about the containing element that is considered essential by the Media Presentation author selecting this component.

  - The following different options exist: not-present; generic parameters from above; list in Table 1; unknown value, which may be extended

- Accessibility descriptor with

  - Role scheme as defined by MPEG-DASH in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with value set to sign, caption or subtitle. The presence of caption or subtitle signals open (“burned in”) captions or subtitles.

  - the scheme when CEA-608 is used as defined in clause 6.4.3.3, with @schemeIdURI set to "urn:scte:dash:cc:cea-608:2015" indicating the use of CEA-608 captions carried in SEI messages.

Adaptation Sets with elements Rating and FramePacking as well with @scanType not set to progressive should not be present.

The content author should use the @selectionPriority attribute in order to express preference for video selection. If captions are burned in with video Adaptation Set, and other video Adaptation Sets are available as well, the content author should use the @selectionPriority to indicate the selection priority of this Adaptation Set compared to others without burned in captions.

### Audio Target Version Annotation

Audio Adaptation Sets of one alternative content shall differ by at least by one of the following annotation labels:

- @codecs: specifies the codecs present within the Representation. The codecs parameters shall also include the profile and level information where applicable.

- @lang: specifies the dominant language of the audio

  - If not present, the language is unknown or no language applies

- @audioSamplingRate specifies the maximum sampling rate of the content

  - If not present, the audio sampling rate is unknown

- The AudioChannelConfiguration specifies support for output devices that may only be able to render specific values. This element should be present.

  - If no AudioChannelConfiguration is present, then this value is unknown.

  - If the codec is anyone in Table 19, Table 24, Table 25 or Table 26, then any of the following may be used

    - urn:mpeg:dash:23003:3:audio_channel_configuration:2011	as defined in ISO/IEC 23009-1 [1], 5.8.5.4

    - urn:mpeg:mpegB:cicp:<Parameter> as defined in ISO/IEC 23001-8 Error! Reference source not found. and <Parameter> one of the following: OutputChannelPosition or ChannelConfiguration

  - If the codec is ec-3 or ac-4 according to Table 22, then the following shall be used

    - tag:dolby.com,2014:dash:audio_channel_configuration:2011" as defined at http://dashif.org/identifiers/audio-source-data/  (see section 9.2.1.2)

  - If the codec is anyone in Table 23, then refer to DTS specification 9302K62400 Error! Reference source not found.

- EssentialProperty: specifies information about the containing element that is considered essential by the Media Presentation author selecting this component.

  - The following different options exist: not-present; generic parameters from above; unknown value, which may be extended

- Accessibility descriptor with Role scheme as defined by MPEG-DASH in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with value set to description or enhanced-audio-intelligibility.

Note that Adaptation Sets with element Rating may be ignored by the client and should therefore only be used if the content provider has knowledge that clients can process the applied Rating scheme.

### Subtitle Target Version Annotation

Subtitle Adaptation Sets of one alternative content shall differ by at least by one of the following annotation labels:

- @codecs: specifies the codecs present within the Representation. The codecs parameters shall also include the profile and level information where applicable.

- @lang: specifies the language of the subtitle

  - If not present, the language is unknown or no language applies

- EssentialProperty: specifies information about the containing element that is considered essential by the Media Presentation author selecting this component.

  - The following different options exist: not-present; generic parameters from above; unknown value, which may be extended

- Accessibility descriptor with Role scheme as defined by MPEG-DASH in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with value set to description or caption.

### Other Annotation, Auxiliary Data

In addition to selection relevant data, the Adaptation Set may also signal additional auxiliary information. Auxiliary information is expressed by

- The Role descriptor with the Role scheme as defined by MPEG-DASH as defined in ISO/IEC 23009-1, 5.8.5.5, urn:mpeg:dash:role:2011 with the following values:

  - caption

  - subtitle

  - main

  - alternate

  - supplementary

  - sign

  - emergency

  - dub

- The Supplemental descriptor with the @schemeIdURI and @value pairs:

  - Trickmode: @schemeIdURI set to "http://dashif.org/guidelines/trickmode" and the @value the value of @id attribute of the Adaptation Set to which these trick mode Representations belong.

  - Period-continuous Adaptation Sets by using Aa @schemeIdUri set to "urn:mpeg:dash:period-continuity:2015" with the @value of the descriptor matching the value of an @id of a Adaptation Set that is contained in the MPD,

  - Period-connected Adaptation Sets by using Aa @schemeIdUri set to "urn:mpeg:dash:period-connectivity:2015" with the @value of the descriptor matching the value of an @id of a Adaptation Set that is contained in the MPD,

  - Switching across Adaptation Sets: @schemeIdUri set to urn:mpeg:dash:adaptation-set-switching:2016 and the @value is a comma-separated list of Adaptation Set IDs that may be seamlessly switched to from this Adaptation Set.

## Client Processing Reference Model

### Introduction

The following client model serves two purposes:

- In the absence of other information, the following client model may be implemented in a DASH client for the purpose of selection of Adaptation Set for playout

- A content author may use the model to verify that the annotation is properly done in order to get the desired client behaviour.

In the model it is assumed that the client can get sufficient information on at least the following properties:

- For each codec in the @codecs string, the DASH client can get information if the media playback platform can decode the codec as described in the string. The answer should be yes or no.

- For each DRM system in the XXX string, the DASH client can get information if the media playback platform can handle this DRM as described in the string. The answer should be yes or no.

- the DASH client can get information on the media playback platform and rendering capabilities in terms of

  - the maximum spatial resolution for video that can be handled

  - the maximum frame rate for video that can be handled

  - the audio channel configuration of the audio system

  - the audio sampling rate of the audio system

- the preferred language of the system

- Accessibility settings for captions, subtitles, audio description, enhanced audio intelligibility,

- Potentially preferences on media playback and rendering of the platform

Note of any of these functionalities are not fulfilled, then it may still be functional, but it may not result in the full experience as provided by the content author. As an example, if the DASH client cannot determine the preferred language, it may just use the selection priority for language selection.

### Client Model

Figure 5 provides a basic overview of a DASH based distribution. Content is offered in the cloud in with different Adaptation Sets and Representations and the content is described in the MPD. The MPD is exposed to the client which uses the information for content selection and finally for downloading and switching. Selection is done based on Adaptation Set or Preselection annotation. Whereas download and switching within an Adaptation Set is part of the DASH client, the selection needs to be done based on information that the DASH client collects from surrounding system functions such as system capabilities, applications or possible even a user interface. Selection may be done statically at startup or may be done dynamically based on changes in the application, based on user input, or other actions. Downloaded segments are passed to the file format parser and finally to the media decoders.

The MPD (on interface 1) is a well defined format based on the DASH specification. The interface 3 towards the file format is well defined through DASH segments, file format structures as well as possible CMAF. The selection is not fully defined. It is also noted the MPD typically extracts information from the ISO BMFF Movie Header and Track Headers in order to support the selection. Hence the data is semantically the same in the file format as well as in the DASH MPD.

.

Figure 5 Client Logic

The selection logic needs to extract relevant information from the MPD (or file format) and provide this to the system and application and dynamically communicate on updates of the selection. In order to leverage the synergies with file format (and possibly other system standards), it is beneficial that the selection logic is abstracted and common high-level API or set of APIs is developed that abstract the selection process.

Typical API calls are:

- isSupported(feature)

- preferenceAmongAlternatives (list of alternatives)

- dynamicSelection(list of altenatives) with callback and notification

### Generic Processing Model

The DASH client uses the MPD and finds the Period that it likes to join, typically the first one for On-Demand content and the one at the live edge for live content. In order to select the media to be played, the DASH client assumes that the content is offered according to the content model above.

- The DASH client looks for main content, i.e. any Adaptation Set with annotation  Role@schemeIdURI="urn:mpeg:dash:role:2011" and  Role@value="alternative" is excluded initially for selection. Note that in this model it is assumed that immediate startup is desired. If the DASH client wants to go over the alternatives upfront before starting the service, then the sequence is slightly different, but still follows the remaining principles.

- DASH Client checks each Adaptation Set for the supported capabilities of the platform

  - Codec support

  - DRM support

  - Rendering capabilities

If any of the capabilities are not supported, then the Adaptation Set is excluded from the selection process.

- The DASH client checks if it supports for CEA-608 rendering as defined in 6.4.3.3. If not supported, any accessibility descriptor with is @schemeIdURI="urn:scte:dash:cc:cea-608:2015" removed. Note that the Adaptation Set is maintained as it may used for regular video decoding.

- DASH Client checks is there are any specific settings for accessibility in the user preferences

  - If captions are requested by the system, the DASH client extracts

    - all video Adaptation Sets that have an Accessibility descriptor assigned with either the @schemeIdURI=" urn:mpeg:dash:role:2011 and @value="caption" or  @schemeIdURI=" urn:scte:dash:cc:cea-608:2015" (burned-in captions and SEI-based), as well as

      - all subtitle Adaptation Sets that have an Accessibility descriptor assigned with either the @schemeIdURI=" urn:mpeg:dash:role:2011 and @value="caption"

      - and makes those available for Adaptation Sets that can be selected by the DASH client for caption support.

    - If multiple caption Adaptation Sets remain, the DASH client removes all Adaptation Sets from the selection that are not in the preferred language, if language settings are provided in the system. If no language settings in the system are provided, or none of the Adaptation Sets meets the preferred languages, none of the Adaptation Sets are removed from the selection. Any Adaptation Sets that do not contain language annotation are removed, if any of the remaining Adaptation Sets provides proper language settings.

    - If still multiple caption Adaptation Sets remain, then the ones with the highest value of @selectionPriority is chosen.

    - If still multiple caption Adaptation Sets remain, then the DASH client makes a random choice on which caption to enable.

  - else if no captions are requested

    - the Accessibility element signaling captions may be removed from the Adaptation Set before continuing the selection.

  - If sign language is requested

    - all video Adaptation Sets that have an Accessibility descriptor assigned with @schemeIdURI="urn:mpeg:dash:role:2011" and @value="sign" are made available for sign language support.

  - else if no sign language is requested

    - the Adaptation Set signaling sign language with the  Accessibility element may be removed from the Adaptation Set before continuing the selection.

  - If audio descriptions are requested

    - all video Adaptation Sets that have an Accessibility descriptor assigned with @schemeIdURI="urn:mpeg:dash:role:2011" and @value="description" are made available for audio description support.

  - else if no audio descriptions are requested

    - the Adaptation Set signaling audio descriptions with the  Accessibility element may be removed from the Adaptation Set before continuing the selection.

  - If enhanced audio intelligibility is requested

    - all audio Adaptation Sets that have an Accessibility descriptor assigned with @schemeIdURI="urn:mpeg:dash:role:2011" and @value="enhanced-audio-intelligibility" are made available for enhanced audio intelligibility support.

  - else if no enhanced audio intelligibility is requested

    - the Accessibility element may be removed from the Adaptation Set before continuing the selection.

- If video rendering is enabled, based on the remaining video Adaptation Sets the client selects one as follows:

  - Any Adaptation Set for which an Essential Descriptor is present for which the scheme or value is not understood by the DASH client, is excluded from the selection

  - Any Adaptation Set for which an Essential Descriptor is present for which the scheme is http://dashif.org/guidelines/trickmode, is excluded from the initial selection

  - If still multiple video Adaptation Sets remain, then the ones with the highest value of @selectionPriority is chosen.

  - If still multiple video Adaptation Sets remain, then the DASH client makes a choice for itself, possibly on a random basis.

  - Note that an Adaptation Set selection may include multiple Adaptation Sets, if Adaptation Set Switching is signaled. However, the selection is done for only one Adaptation Set.

- If audio rendering is enabled, based on the remaining audio Adaptation Sets the client selects one as follows:

  - Any Adaptation Set for which an Essential Descriptor is present for which the scheme or value is not understood by the DASH client, is excluded from the selection

  - If multiple audio Adaptation Sets remain, the DASH client removes all Adaptation Sets from the selection that are not in the preferred language, if language settings are provided in the system. If no language settings in the system are provided, or none of the Adaptation Sets meets the preferred languages, none of the Adaptation Sets are removed from the selection. Any Adaptation Set that does not contain language annotation are removed, if any of the remaining Adaptation Sets provides proper language settings.

  - If still multiple audio Adaptation Sets remain, then the ones with the highest value of @selectionPriority is chosen.

  - If still multiple audio Adaptation Sets remain, then the DASH client makes a choice for itself, possibly on a random basis.

  - Note that an Adaptation Set may include multiple Adaptation Sets, if Adaptation Set Switching or receiver mix is signaled. However, the selection is done for only one Adaptation Set.

- If subtitle rendering is enabled, based on the subtitle Adaptation Sets the client selects one as follows:

  - Any Adaptation Set for which an Essential Descriptor is present for which the scheme or value is not understood by the DASH client, is excluded from the selection

  - If multiple subtitle Adaptation Sets remain, the DASH client removes all Adaptation Sets from the selection that are not in the preferred language, if language settings are provided in the system. If no language settings in the system are provided, or none of the Adaptation Sets meets the preferred languages, none of the Adaptation Sets are removed from the selection. Any Adaptation Set that does not contain language annotation are removed, if any of the remaining Adaptation Sets provides proper language settings.

  - If still multiple subtitle Adaptation Sets remain, then the ones with the highest value of @selectionPriority is chosen.

  - If still multiple subtitle Adaptation Sets remain, then the DASH client makes a choice for itself, possibly on a random basis.

- If the DASH client has the ability to possibly switch to alternative content, then alternative content may be selected either through the Label function or the ViewPoint functionality. This selection may be done dynamically during playout and the DASH client is expected to switch to the alternative content. Once all alternative content is selected, the procedures following from step 2 onwards apply.

At Period boundary a DASH client initially looks for a Period continuity or connectivity, i.e. does the Period include an Adaptation Set that is a continuation of the existing one. If not present it will go back to step 1 and execute the decision logic.

# Part 5: CORS Considerations for SAND

Note: This section will be added to the next amendment of Part 5.

SAND may be used in the context of 3GPP for different purposes. A DANE may sit in the operator’s network and assist the client to achieve an optimal streaming experience for both the client and also in terms of network resource efficiency. It can also be used in the context of MBMS client to DASH application communication.

SAND defines a main communication channel using connection triggers, through insertion of an HTTP header field in the response. Upon detecting this header field, the client will connect to the provided URL to fetch the SAND message. Alternatively, a WebSocket channel may be indicated by the MPD, in which case the client connects to a global DANE server to receive SAND messages. Both methods have their pros and cons. However, both methods are subject to the CORS and mixed content restrictions.

In particular, the URL of the DANE’s SAND message that is provided in the “MPEG-DASH-SAND” message or as part of the WebSocket URL must allow the origin of this request. It also has to match the same security environment (i.e. usage or not usage of TLS consistently).

It is therefore recommended that the DANE servers shall support CORS and allow the origin of the MPD in all cases. DANE’s shall also support the same security configuration to avoid the mixed content problem.

# Ad insertion

This section consists of few use-cases and few proposals. The discussion has been going on for few meetings. The goal is to drive to a single set of tools for ad-insertion that satisfies the use-cases.

## Use-cases

The following use-cases are considered by TuC:

- Live content with early termination ads

- Switching between different live feeds

## The proposed solutions so far

- Existing tools (Xlink + MPD update) with additional signaling (4.7)

- Content replacement event (4.6)

## Discussion at MPEG127th (m48861, m49263, m49607, m49374, m49608)

Several solutions has been proposed.  To summarize, we have the following use-cases:

- Live with early termination

  - Signaling of possible early termination

    - Period with early termination: no need to signal if a period may terminate early

    - Termination at segment boundary: recommendation

    - Acceptable transitions: -

    - Others

    - Xlink

      - We need to develop a more detailed processing model for remote period resolution (currently identified as xlink).

      - Potentially the remote period resolution enables the client to play the content independently.

      - The content represented by this remote period may require conditioning.

  - MPD update mechanism:

    - How the client receive signal for updating the MPD while playing the ad, as it is not connected to the live server.

The DASH ad-hoc has the mandate to develop a single solution for above use-cases and address above questions for MPEG128th.

## Discussion at MPEG128th (m50916, m51006, m51021)

We reached consensus on our approach to ad insertion:

  - We need a processing model for resolution of a hyperlink

  - The hyperlink should point to a valid playable MPD

  - We will study how a new period is introduced to the client and the client/ad ins/live can consistently work with this new period

  - Not every period is a replacement opportunity. We study on how we need to carry splice point info to the client for client side ad insertion.

## Discussion at MPEG129th (based on m52438 )

### We need a processing model for resolution of a hyperlink

In order to process a hyperlink consistently, we believe:

- Any resolution of a hyperlink for ad-insertion use cases should result in the complete period.

- The solution should support the resolution of hyperlink when the client gets close to the ad break. This may require additional signaling in the case of xlink. For instance, a range of time can be given to the client as the guidelines for xlink resolution.

- The resolution should result in a valid timeline, the combined timeline after the resolution can be described by an MPD.

- The client needs to resolve the hyperlink once or if it does multiple times, they all should result in one consistent resolution.

- In the case of random access to the middle of the ad, the ad could be played depending on the use-case.

- In the case of replay, the ad could be replaced with a new ad, or the same ad could be played depending on the use-case.

Requirements 4 and 6 suggest that the client when requesting a hyperlink may need to signal ad-insertion entity that which state it is at, prior to playing the ad for the first time or playback the ad again.

We believe xlink element as long as its resolution is constrained with the above requirement is adequate for hyperlinking.

### We will study how a new period is introduced to the client and the client/ad ins/live can consistently work with this new period

We observe:

- The introduction of a period as long as it is consistent with the MPD update should not create any problem for the client.

- Splice conditioning of the period transition is a property of the content. Signaling the properties of the content and required capabilities by the client informs the client whether it can work consistently cross period transitions.

- Period insertion in the client is similar to period insertion in the MPD manipulator since there is no segment manipulation is possible in either case.

- As long as the hyperlink rules are obeyed by the client, a hyperlink period is the same as an explicit period.

### Not every period is a replacement opportunity. We study on how we need to carry splice point info to the client for client side ad insertion.

There is no universal signaling for ad replacement opportunities. SCTE35 is just one of them. Ad replacement opportunity may include: 1) The timing of the ad replacement: in and out moments and 2) splice conditioning.

In the case of server-side ad-insertion, the MPD manipulator can drive the conditions from the ad-replacement opportunity signaling and from MPD itself (in case of splice conditioning.

For the server guided ad insertion, ad opportunity should be turned into ad-replacement instruction. In this case, the conditioning of the live content (specifically transition to) could be signaled to the client.  This can be done in two levels:

- What the client could expect (e.g. transitioning always at the boundary)

- What the client is required to address (e.g. transition in the middle of segment)

## Proposal: Content and Ad Server Communication (updated with m47557 @MPEG 126)

### Problem Statement

The DASH spec deals with the case that the program author has full knowledge on the content authoring and controls both, the main content generation and the ad content generation and splicing. However, in many cases ad content is provided on a separate library and independent of the main content. While ads may be considered “annoying” in general, there is a significant improvement in the viewing experience if the quality of the ad matches to at least the following:

- The quality of the ad, matched to the device capabilities.

- The features of the ad, for example whether interactivity is supported or not.

- The quality of the ad, matched to the currently playing content. E.g. avoiding any HDMI resets, source buffer reinitialization, unnecessary black frames due to content splicing, encryption and protected content mismatches, different audio codec configurations, etc. This can be further refined by taking a combination of the played content and the device capabilities, for example for certain capabilities the ad performance can be improved.

- The encoding requirements (timing, splicing) of the ad at the ad splice point.

- The personalization of the ad to the user, for example based on user ids, geolocation and so on.

- The duration of the ad slot.

- Other issues that permit targeting the ad.

Especially the third issue has been ignored largely because many workflows are quite vertical and content providers also condition their ads. However, with more and more content formats, device capabilities and different formats, there is a benefit in combining the above for a consistent way and providing the ad server as much information as possible in order to provide a “good” spliced ad.

The ad may also be providing instructions on what its conditions are in order for the application/player to optimize the user experience by using proper playback instructions.

Yet another aspect is the issue that the receiver may or may not be able to add a second SourceBuffer (media decoding elements), one for ad playback and one for the main content, depending upon the device capabilities.

A proposed call flow is shown in the below Figure:

Figure 1: Call flow for ad insertion

- The content server provides a content manifest and the manifest generator uses this information to generate a manifest for the player.

- The player requests the manifest and,

- based on the available content and its capabilities, the ABR player will select the proper content for playout. Examples of the choices include

  - The chosen codec,

  - Spatial and temporal resolution of the display, the HDR capabilities, etc.

  - Audio codec and rendering capabilities, etc.

- At some point in time, the Content Server provides an ad insertion opportunity to the manifest generator and,

- it also adds a signal into the content in order to expire the MPD in hand.

- Subsequently, the ABR player requests a manifest update. This manifest update will now include a sub-manifest that points the player to the ad insertion server. The player request may include additional parameters such that the ad insertion server is aware of the currently playing content, personalization information etc, as well as the capabilities of the player.

A way to inform on the playing content is for example:

- The Movie Header/Initialization Segment/CMAF Header of each playing track. This provider a good overview on the codecs, formats and so on. Other means to provide such information may be provided, for example manifest parameters, mime types with codecs sub-parameters, etc.

- In addition, the presentation time of the last presented sample may be provided in order to provide a continuous presentation.

Another issue that is generally observed is the problem of splicing content for which two media types, typically audio and video, do have different “end times” before the splice point, or do have different starting times. In this case the transition for each media type needs to be done carefully and individually. This may include that for example one of the media types has a gap, or content may overlap. Overlap may be difficult in a single source buffer as the splice point is ambiguous. Gaps may cause issues because the playback gets stalled or interrupted.

While this is generally a bigger problem in an on-demand environment, it is still mostly a non-issue in the OTT environment. This is because the SCTE-35 signalling allows for explicit cut point creation in the raw stream in addition to periodic segment cuts. The only real time difference you observe then is a handful of milliseconds difference due to frame size but the gap is too small to be perceived in playout.

### High-level Proposed Solution

In the solution to this problem, it is proposed that the client provides information to the ad server such that the ad server can provide content that enables the device the playout of the ad content in a suitable and adjusted manner.

This includes that the content may be provisioned (specifically encoded or at least selected) to the main content to be displayable within one source buffer.

The key issue is that the ad server receives sufficient knowledge on the capability of the device as well as on the currently playing content. Based on this knowledge the ad insertion server may properly prepare and provide the content such that playback is seamless.

As one example, the combination of the following information may be provided

- Device capabilities, for example how to handle ad insertion. Some options are:

  - Single source buffer with static initialization.

  - HDMI properties

  - Multiple source buffers

  - Regular MSE operation

- The ad insertion slot time. This provides the ad server the information to accurately select the ad and the ad boundaries.

- The properties of the played content, for example CMAF Header of the played content of each media component. This allows the ad server to select proper content that matches the playback.

- The last played timing of the main content for each media component. This allows the ad server to splice content properly, for example to avoid overlaps or to exactly splice

The signaling may for example be supported by

- Adding a query string to a sub-manifest request in step 6 of the call flow in Figure 1 with the properties of one or more of the content property, the device capabilities and the timing.

- Adding a dedicated HTTP extension header to the request of the manifest

- Adding a URL in the request where the ad server can obtain the information

An example is the construction of a URL using the Flexible URL Parameter algorithm in ISO/IEC 23009-1, Annex I.

Detailed signaling needs is tbd.

### Targeted Ads in Live Services

#### Problem Statement

The insertion of ads into DASH or general ABR live service is a major opportunity, but also challenge. The ISO/IEC 23009-1 standard provides sufficient means to insert ads in On-demand content. It also permits to easily add targeted ads into live services.

A common approach is the use of Periods with xlink. The Period element will provide a URL to an ad server and the resolution of this request provides a fully compatible Period structure that is placed in the timeline of the main live service.

Live services are supported by extending the timeline with each segment. This is typically verified by information of the live service, i.e. by an updated manifest or by the absence of inband event message that terminates the event or triggers an MPD/manifest update.

For live services, if the ad slots are accurately timed and the ads exactly match the times of the provided time slot (both when leaving the main content and transition to the ad, as well as when exiting the ad and going back to the main content), this usage of regular DASH Periods, that are placed in the time line, create no problem. The DASH client plays the received Period content and plays this without even recognizing the playback is an Ad. At the end of the ad, the return to the live service is supported by providing a new Period that enables to return to the live service.

However, there are several cases for which exact timing and placement of the ad is not supported and the placement of Periods in the timeline is not following the DASH timing model. Some cases are listed below:

- An ad is offered during the live service, but the targeted ad is not exactly matching the time of the provided timeline. In this case, the Period of the ad does not create a proper continuous timeline for the client and the client needs to act to compensate this.

- An ad opportunity is offered with a defined duration, and an ad Period is inserted matching this duration, but the live service that continues in the background wants the DASH client to terminate the ad playback early and return to the live service. This is typically done such that the MPD packager sends an MPD update that includes a period that terminates the ad. An example may be that the live event resumes earlier than expected.

In all of these cases, the main issue is that the ad Period cannot be exactly placed and inserted in the live timeline.

This issue had been addressed to some extent by providing two media decoding pipelines that are both running in parallel, one for the live service and one for the ad decoding and processing. On presentation level, only the ad is presented and the live service is hidden during Ad presentation. In addition, there exist methods that this signaling needs a separated processor (such as SCTE-35 clients) on the playback device that interprets the logic and the ad placement.

However, all of this results in the problem, that two media pipelines need to be supported. This is either not possible at all for some devices, and even when it is, it places significant processing and resource burden on resource constrained platforms, especially for mobile devices. In addition, proper signaling to realize all the abovementioned cases is not supported currently in DASH.

#### Proposed Solution

##### Overview

Based on the discussion in clause 3.1, a proposal is sketched that addresses the issues. This includes proper signaling on DASH level, combines the signaling and processing with the conditioning in clause 2, as well as the ability to use a single media pipeline for playback of main content and the Ad.

On high-level the following is proposed:

- Periods are not added in the timeline of the MPD received by the client, but the DASH client places a properly constrained "Period" that contains the ad on the playback timeline.

- The URL to the remote server providing the Period, the start time, the considered duration as well as other information is provided in a timed DASH event, possibly being added in an MPD Event or in an inband event. The Event is parsed by the DASH client and the DASH client acts and downloads the Period and places the Ad Period timeline on top of the continuing live service.

- The requested Period may be properly conditioned, possibly taking into account dynamic information sent in the request as defined in clause 2.

- The DASH client continues to consume the live service in a sense that at least it updates the MPD and/or observes event streams for MPD updates. This allows that the packager can signal for example the early termination of the ad, or the extension of the ad, or other related information.

- As the Period is properly conditioned, the DASH client is able to splice the ad in the same media element/Source Buffers for playback.

The following aspects are defined:

- A DASH event that signals the placement of a remote Period on top of the live service. Some details are provided in clause 3.2.2.

- A signal that overlays the ad at the time when the service is joined for the preroll ad. This is be covered by the same DASH event with a specific time signal.

- The response format of the remote Period.

- The conditioning of the remote period as defined in clause 2.

- The client processing model for splicing the ad on the client.

##### DASH Event for Remote Period Overlay Playback

An Event which signals a remote period provides at least the following information

- The start time of the ad insertion opportunity.

- The planned duration of the ad insertion opportunity.

- The URL from where the client can retrieve the remote Period.

The above information is fully processed by the DASH client and is not needed to be passed to an application. This avoids any additional processing.

The start time maps to the media timeline. The start time may be conditioned such that it coincides with the end of a segment or a Period to simplify insertion.

A specific value for the event time may defined that indicates that the Event is to be played is when the client joins. Alternatively, the Event time may be at the start of the first period and the duration of the event is infinite or long such that it ensures that the event still starts even after joining.

The Event may also contain additional information that supports the ad insertion process.

##### Remote Period format

The remote period contains a regular remote period format as defined in ISO/IEC 23009-1.

In addition, it may contain information, e.g. if the playback of the Ad may be shortened without impacting the user experience, for example indication of a time during the ad when the audio is mute for a while and has black frames. This permits for easier conditioning.

##### Ad Playback Placement by the client

The client uses the remote period start time and adds this as a regular Period in its playback. It requests the ad following the URL, potentially with a condition. It plays the ad according to the nominal duration, but still continues to monitor the live service for potential new events or changes on the playback. At the end of the ad, the client transitions back to the live service by assuming that the live service creates a new Period after Period duration of the ad. The DASH client may adjust the duration of the ad if permitted to simplify the transitions.

#### Detailed Specification Text

Change 1: Add the following attribute on Period level

|  |  | @earlyTermination | OD Default: 0 | if present and the @duration attribute is present, this value specifies that the Period may be terminated earlier by at most the value of the attribute in the playback than signaled by the @duration. Note that this may for example be the case when the content fades to black at the end and audio is muted. Content in such Periods may be used preferably when content of different length need to be spliced. |
| --- | --- | --- | --- | --- |

Change 2: Add the following

#### 5.10.4.6	DASH Content Replacement Event

5.10.4.6.1		General

DASH Content Replacement events are instructions in the content that the DASH client replaces content from a specific media time onwards for some time by a Period that is provided from remote. These event schemes are identified by the URN "urn:mpeg:dash:event:replacement:2019".

A content author may use such an event for replacing certain part of the content.

A content author may also use such an event for terminating this event earlier. This is achieved by adding a URN into the event with the value "urn:mpeg:dash:event:replacement:main:2019"

A DASH client is expected to follow this event stream as well as regular MPD updates while playing back the replacement content.

5.10.4.6.2		Semantics

| #### Key | #### Description |
| --- | --- |
| #### schemeIdURI | #### Set to urn:mpeg:dash:event:replacement:2019 |
| #### start_time | #### Provides the nominal start time of the replacement content in the media timeline. |
| #### value | #### 1: indicates that the remote Period is played from the time that matches since the start of the event has passed. #### 2: indicates that the remote Period is played from the start, regardless at what time you join the event and played until end. |
| #### duration | #### Provide the range for which the event may be started after the start time. If set to 0, then only when media time with value start_time is played the content replacement happens. If greater 0, the content replacement can also happen in a media time later start_time, maximally at start_time + duration. #### For preroll content replacement start_time is set to the presentation time offset of the period and duration is set to a very large value, i.e. the expected duration of the period. The value is set to 2. |
| #### message | #### Provides a URL to a remote Period, possibly extended with query parameters to signal the duration of the content replacement slot as well as other parameters. #### If this is set to "urn:mpeg:dash:event:replacement:main:2019" then the Period that includes the event stream is played. The return to the main content happens earliest at start_time, and latest at start_time + duration. |

The above values are mapped to presentation time, duration and the message of the event and may be carried in the MPD or inband.

5.10.4.6.3		Requirements for Remote Period

A remote period referenced in a content replacement event shall follow the following format:

-	The Period@duration attribute is provided

-	The @availalilityTimeOffset is set to INF

-	All content in the Period is filled, i.e. no gaps at start or end

-	The @earlyTermination attribute may be present to indicate that the content can be terminated earlier than indicated by @duration.

<more to be added>

5.10.4.6.4		Remote Period Query Parameters

The following query parameters should be present in a remote Period

- Duration: provides the nominal duration of the content replacement slot.

- MaxDurationDelta: provides the maximum duration delta of the content replacement slot.

- MinDurationDelta: provides the minimum duration delta of the content replacement slot

The client may add the following query parameters

- Conditioning Parameters:

  - The Initialization Sets for the content currently played back

- The desired value of the @start attribute of the Period in the MPD.

5.10.4.6.5		Client Processing Model

When subscribed to a content replacement event stream, the client is expected to act as follows:

- If an Event is received with a new ID, then the client acts as follows:

  - The client uses the start time of the event to determine when the replacement content will happen.

  - If the start time has passed, but the event duration indicates that it is still within the duration, then the replacement content is played, depending on the value being set.

  - If the message is set to "urn:mpeg:dash:event:replacement:main:2019" and the DASH client is playing the main content, it continues to play the main content.

  - If the message is set to "urn:mpeg:dash:event:replacement:main:2019" and the DASH client is playing the replacement content, the DASH client returns to the main content at start_time of the main content. If the start time has passed, but the media time is still within the duration of the event then the return to main content happens as soon as possible.

  - If the message is set to a regular URL, then the DASH client issues a request for the content. The DASH client may add additional query parameters as defined in 5.10.4.6.4.

  - Once received the remote Period the DASH client places this Period into its playback timeline as follows:

    - The Period@start is set such that it matches the presentation time of the event in the main content.

    - The remote period is played following the @duration attribute, possibly using the @earlyTermination, and after playing, it returns to the main content at the presentation time that matches the Period duration. This is equivalent and playing the main content with presentationTimeOffset set to start_time + the duration of the ad. The exact transition may be adjusted of earlyTermination is provided.

  - Anytime a new Event is received, the replacement happens, i.e. even if replacement content is already played, a new event results in a new replacement.

- Else the event is ignored.

## Proposal: Flexible Ad insertion with existing tools in TuC and minor additions (updated with m47195 @MPEG126 and m48861@MPEG127)

### Introduction

This contribution proposes the additional signalling needed for the client-side ad-insertion in DASH standard.

### Client side/Server guided ad insertion architecture

The following figures shows the interfaces between origin server, client and the ad server:

Figure 1: DASH-IF Ad Insertion Architecture

As shown in this figure, there is no direct interface between the origin server and ad server. Interfaces IF-5 and IF-7 indicates the interfaces between origin server, DASH client and the ad server.

### Use cases

We cover the following use-case in this contribution:

- Dynamic ad insertion with fixed duration

- Dynamic ad insertion with possibility of early termination by content server

Case 1 is already supported by existing tools. Case 2 is the use-case that is difficult due to early termination of the ad by the origin server.

### DASH Principle

#### MPD as the holder of truth

The current DASH design for live services is built on this concept that the MPD at origin server at any moment of time holds the latest update of the program, i.e. that version of MPD includes the presentation with correct period breakpoints and duration and reflect the best knowledge of the program.

We argue that since the MPD is published for possibly millions of clients and different clients may join the live program at any moment, the MPD request may happen at any time but the client or by the CDNs to maintain a copy of the latest MPD update in its cache, the MPD in the origin server must reflect the most accurate information known, i.e. if a ad-breaks is being planned or if an ad period will be shorten, the MPD must reflect this information, so that if any client access the MPD at the time, the client can play the content correctly.

Therefore, we argue that even if the ad-server shorten an ad period, the origin server must be informed and it may possibly reflect the new duration in its MPD update, if it decides that the shorten of duration is applied to all clients.

#### MPD as the entry point for media presentation

The current DASH client implementations rely on the MPD as the single point of entry for playing back the content, i.e. they build their timeline by parsing the MPD and building a corresponding timeline. Creating multiple timelines or switching between timelines while conceptually are possible, it requires multiple DASH client working in parallel.

#### Maintaining single timeline and not switching between timelines

The current DASH clients work on single timeline. The idea of switching between two timelines might conceptually be ok, but in reality two DASH player and an application is needed to manage these timelines. It is very much similar to playlist concept in which the playlist calls DASH client and passing the MPDs for playback of each and combine the output together.

#### Fallback methods

In DASH streaming, many clients with different features are deployed. An application should rely on the most common denominating features among DASH client, that is the application can use advanced features, but it is preferred that a client not supporting the advance feature still be able to operate using the basic features.

#### MPD acquisition

Since the MPD at origin server always carries the latest truth, the client needs to monitor if the MPD is updated when an MPD update is a possibility. In DASH, we have 3 techniques to inform the client about an MPD update:

- MPD pulling

- Inband MPD validity expiration event

- MPD expiration SAND message

As long as the client uses one of the above methods to maintain the updates of the MPD with the frequency that is required, the client must be able to operate correctly

#### Communication through DASH client

In the above diagram, IF3 interface shows the communication between the origin server and ad server. While this is possible, the most trusted commination between origin and ad server is through the client, since the content might be cached in various CDNs.

### Early Termination of ads

Figure 2 shows the sequence of actions for an ad that is early terminated:

Figure 2: sequence of actions in early termination of ad in live streaming

The key is that during the playback of the ad (Period P1 in MPD1), the origin servers decides to cut the duration of ad earlier than it was originally planned. It will update the MPD, from MPD1 to MPD2. The client needs to update the MPD by any of the means established for MPD update:

- MPD pulling

- Inband event to update the MPD

- Out of channel signalling (such as SAND).

In order for the client to use one or more of the above tools to update the MPD, the client should be aware of the possibility of early termination for the ad period. Therefore, we introduce signalling of those possibilities in MPD in the next section.

### New tools needed for standard implementation

#### Signaling a possible early termination

Either an explicit attribute or a supplementary descriptor should be defined to signal early termination for a given period.

As example of supplemental descriptor can be found in the MPD below:

<Period xlink:href="remote.period" xlink:actuate="onLoad"> xmlns:xlink="http://www.w3.org/1999/xlink"> <SupplamentalProperty schemeIdUri="urn:mpeg:dash:earlytermination" /> … … … </Period>

Figure 2. Signaling the possibility of cutting ad durations using descirptor

Alternatively, an attribute can be defined for the Period element:

| Element or Attribute Name |  | Use | Description |
| --- | --- | --- | --- |
| Period |  |  | specifies the information of a Period. |
|  | @earlyTermination | O Default = “False” | A “True” value indicate this Period duration might be updated to a shorter value. This value is only allowed If the MPD@type is "dynamic", MPD@MPDLocation exist, and the period is a Xlink period, with default period. It is recommended that client updates the MPD during streaming of this period if this value is set to “true” either by frequent pulling of the MPD or by streaming any representation that carries inband MPD validity expiry event. |

Figure 3. Signaling the possibility of cutting ad durations using explicit attribute

#### Possibility of early termination at Period level

This signalling informs the client that this period may end earlier than expected. We use supplemental Property descriptor to signal:

- The period may be early terminated

- The supported tools that provides the early termination information:

  - MPD pulling

  - Inband event in a given adaptation set or representation

  - Out of band signalling (e.g. SAND).

The event and values name for the above can be:

- schemeIdUri: uri:mpeg:dash:earlyTerminating:2019 - value: sum of any combinations of the following values - 1: MPD pulling - 2: Inband event - 4: SAND

Table 2: Descriptor at Period level

DASH client seeing this descriptor, knows how to obtain the information about MPD update, i.e. through MPD pulling, Inband events or SAND. Note that these tools are not exclusive, and the origin sever can signal it may provide the MPD update through any number of them.

#### Additional improvement 1: Shortest eArly termination Duration (SAD)

There are benefits for the ad-server to know that there is a minimum duration for early termination notice.  The benefits are:

- Ad server can provide content with short segment duration, at least with segment duration comparable with the Shortest eArly termination Duration (SAD)

- Ad server may provide content that is more appropriate if they cut short

The query parameter can be used to provide such parameter. This parameter is provided to ad-server as a query with xlink request.

#### Additional improvement 2: Guaranteed Ad Duration (GAD)

In order to guarantee a minimum duration of ad, for the sake of the ad-server, we introduce another optional parameter that signals the ad-server the minimum duration of ad promised to be play. In this case, an early termination signal may come after this minimum duration (GAD) only.

It would be beneficial to signal GAD to the ad server and DASH client using query parameters.

The value of GAD can be signaled in MPD using the query parameter insertion.

#### Allowing cutting the duration

The origin server ads a UrlQueryInfo element of DASH specification (ISO/IEC 23009-1) Annex I in the remote period, signaling the duration of this period can be shortened. A queryString such as cutDurationAllowed long with a periodId are used to signal such conditions. Additionally, an ExtUrlInfo element of Annex I is used to request the query info to be added to any xlink request. Figure 4 shows such example.

<Period xlink:href="remote.period" xlink:actuate="onLoad"> xmlns:xlink="http://www.w3.org/1999/xlink"> <SupplamentalProperty schemeIdUri="urn:mpeg:dash:urlparam:2016" xmlns:up="urn:mpeg:dash:schema:urlparam:2016"> <up:UrlQueryInfo queryString="cutDurationAllowed&periodId=ABCXDN"/> <up:ExtUrlInfo includeInRequests="2"/> </SupplamentalProperty> … … … </Period>

Figure 4. Signaling the possibility of cutting ad durations

A client receiving such manifest adds the cutDurationAllowed string to the query along with the xlink request to the ad-server. The ad-server parsing this query knows it can shorten the ad-duration.

#### Manifest Update Event with new duration

The ad-server resolve the xlink with the ad period. The ad period consists of:

- The new shortened duration

- An optional MPD event, including an MPD update event.

- A UrlQueryInfo element, signaling addition of the duration value as part of query for MPD update request.

An example of such period is shown in Figure 4.

<Period id="1" duration="PT20S" xmlns="urn:mpeg:dash:schema:mpd:2011"> <EventStream schemeIdUri="urn:mpeg:dash:mpdevent:2018" value="1"> <Event presentationTime="0" duration="10000" id="0"> </EventStream> <SupplamentalProperty schemeIdUri="urn:mpeg:dash:urlparam:2016" xmlns:up="urn:mpeg:dash:schema:urlparam:2016"> <up:UrlQueryInfo queryString="newDuration=PT20S&periodId=ABCXDN"/> <up:ExtUrlInfo includeInRequests="3"/> </SupplamentalProperty> … … … </Period>

Figure 5. Signaling the new duration of the remote ad

Note that in this example, the duration is 20 seconds. Therefore, the query string attribute includes newDuration=PT20S&periodId=ABCXDN used to the URLQueryInfo provided in original MPD to identify the period. In each case, the exact duration is included in this value.

#### New Manifest event for MPD update

In order to support this solution, we propose a new DASH MPD event: MPD update. This event is similar to in-band MPD validity expiration event, with the difference that it is included in manifest and not in-band. It has a different URL of its scheme identification, but its values are same as the values of MPD validity expiration events.

In this case of signaling, the @presentationTime are set to zero (default), so that the client immediately to request the MPD update.

## Proposal: Content Replacement and Ad Insertion Event (m54500)

### Introduction

This section proposes a simple DASH logic that permits to do content replacement of main content based on an Event.

### Relevant Reference Architecture

The following architecture is assumed.

### Content Replacement Event

#### 5.10.4.7	DASH Content Replacement Event

5.10.4.7.1		General

DASH Content Replacement events are instructions in the content that the DASH client replaces content from a specific media time onwards for some time by a Period that is provided from remote. These event schemes are identified by the URN "urn:mpeg:dash:event:replacement:2020".

A content author may use such an event for replacing certain part of the content.

A content author may also use such an event for terminating this event earlier. This is achieved by adding a URN into the event with the value "urn:mpeg:dash:event:replacement:main:2020"

A DASH client is expected to follow this event stream as well as regular MPD updates while playing back the replacement content.

5.10.4.6.2		Semantics

| #### Key | #### Description |
| --- | --- |
| #### schemeIdURI | #### Set to urn:mpeg:dash:event:replacement:2020 |
| #### start_time | #### Provides the nominal start time of the replacement content in the media timeline. |
| #### value | #### 1: indicates that the remote Period is played from the time that matches since the start of the event has passed. #### 2: indicates that the remote Period is played from the start, regardless at what time you join the event and played until end. |
| #### duration | #### Provide the range for which the event may be started after the start time. If set to 0, then only when media time with value start_time is played the content replacement happens. If greater 0, the content replacement can also happen in a media time later start_time, maximally at start_time + duration. |
| #### message | #### Provides a URL to a remote Period, possibly extended with query parameters to signal the duration of the content replacement slot as well as other parameters. #### If this is set to "urn:mpeg:dash:event:replacement:main:2020" then the Period that includes the event stream is played. The return to the main content happens earliest at start_time, and latest at start_time + duration. |

The above values are mapped to presentation time, duration and the message of the event and may be carried in the MPD or inband.

5.10.4.6.3		Requirements for Remote Period

A remote period referenced in a content replacement event shall follow the following format:

-	The Period@duration attribute is provided

-	The @availalilityTimeOffset is set to INF

-	All content in the Period is filled, i.e. no gaps at start or end

-	The content follows the DASH Ad Insertion Parameters

5.10.4.6.4		Remote Period Query Parameters

The following query parameters should be present in a remote Period

- Duration: provides the nominal duration of the content replacement slot.

- MaxDurationDelta: provides the maximum duration delta of the content replacement slot.

- MinDurationDelta: provides the minimum duration delta of the content replacement slot

The client may add the following query parameters

- Conditioning Parameters:

  - The Initialization Sets for the content currently played back

- The desired value of the @start attribute of the Period in the MPD.

5.10.4.6.5		Client Processing Model

When subscribed to a content replacement event stream, the client is expected to act as follows:

- If an Event is received with a new ID, then the client acts as follows:

  - The client uses the start time of the event to determine when the replacement content will happen.

  - If the start time has passed, but the event duration indicates that it is still within the duration, then the replacement content is played, depending on the value being set.

  - If the message is set to "urn:mpeg:dash:event:replacement:main:2019" and the DASH client is playing the main content, it continues to play the main content.

  - If the message is set to "urn:mpeg:dash:event:replacement:main:2019" and the DASH client is playing the replacement content, the DASH client returns to the main content at start_time of the main content. If the start time has passed, but the media time is still within the duration of the event then the return to main content happens as soon as possible.

  - If the message is set to a regular URL, then the DASH client issues a request for the content. The DASH client may add additional query parameters as defined in 5.10.4.6.4.

  - Once received the remote Period the DASH client places this Period into its playback timeline as follows:

    - The Period@start is set such that it matches the presentation time of the event in the main content.

    - The remote period is played following the @duration attribute, possibly using the @earlyTermination, and after playing, it returns to the main content at the presentation time that matches the Period duration. This is equivalent and playing the main content with presentationTimeOffset set to start_time + the duration of the ad. The exact transition may be adjusted of earlyTermination is provided.

  - Anytime a new Event is received, the replacement happens, i.e. even if replacement content is already played, a new event results in a new replacement.

- Else the event is ignored.

## Proposal: Xlink caching processing model and signaling (m54502)

### Introduction

This section argues that the xlink processing model defined in XML and DASH specification are not adequate to address the media playback, and a cache processing model is needed.

It further demonstrates that this problem is not limited to xlink and any hyperlink with possible re-resolution during the same session needs a caching model for consistent playback by a client.

Finally, it shows how the HTTP caching model can be used in DASH and proposed the same model and similar signaling to be used for any time-dependent hyperlink in MPD.

### The most common OnRequest xlink Usecase

The most common usecase is target-based ads for live content with just in time resolution. The current solution in MPEG DASH is onRequest xlink.

In this usecase, various questions arise:

- When is the best time for the client to resolve the remote link? (not answered in this contribution)

- How long is a resolved xlink valid?

- What if due to the MPD update process, the client attempts to resolve the remote link again during the playback of the remote period?

- Is the client expected to resolve the xlink again when it is directed to random access the ad to the beginning, or just before the remote period or should it play the same resolved period?

- Is the client expected to resolve the xlink again when the played ad is stopped and started again from the beginning of the period? Is there any notion of a session for the resolved period that should be maintained?

### Deficiency of DASH xlink processing model

The W3C xlink recommendation defines “onRequest” as the following:

An application should traverse from the starting resource to the ending resource only on a post loading event triggered for the purpose of traversal. An example of such an event might be when a user clicks on the presentation of the starting resource, or a software module finishes a countdown that precedes a redirect.

And according to ISO/IEC 23009-1:

onRequest (default): according to W3C Xlink, an application should dereference the remote element entity only on a post-loading event triggered for the purpose of dereferencing. In the context of this document, the application dereferences the link only for those resources it needs (or anticipates it probably will need) and at the time when it needs the content of the remote element entity for playout. Examples include dereferencing a link in a Period element when the play-time is expected to enter that Period, dereferencing an Adaptation Set link when it appears to contain Representations that will be needed, and so on.

And for the processing, ISO/IEC 23009-1 states:

The remote element entity referenced from within an MPD (referred to as appropriate targets) shall be embedded into the MPD by applying the following rules: 1) If the remote element entity is empty, all @xlink attributes shall be removed from the element in the MPD and the remaining attributes and child elements shall not be changed. 2) If the remote element entity is non-empty, the original element in the MPD that contains @xlink:href shall be replaced with the content in the remote element entity. If multiple top-level elements are obtained from the remote element entity, the elements shall be in appropriate order and the first element shall replace the MPD element. All other top-level elements shall be inserted immediately after this element in the order in which they are declared. 3) All XLINK attributes initially present in the MPD shall be removed after dereferencing is completed. 4) All resources in the remote element entity referenced by @xlink:href shall have an availability end time as specified by MPD@availabilityEndTime.

Comments from Paul Higgs during ad-hoc discussion:

XLINK is used to create well formed XML instances based on information that is currently known or will become known (or created) when needed. Once that information is known (i.e. in the XML instance) then 'job done'. You should not be looking at XLINK if you want something like an MPD version of a Dynamic HTML DOM There is no cache in XLINK, when a local resource (with the xlink:actuate) needs to be dereferenced, the remote resource replaces the local resource entirely.

Considering all above the processing model for MPD with xlink with OnRequest mode is the following:

- The client receives the MPD.

- At the time of processing the xlink element, it requests the xlink and replaces the element in MPD with the resolution is received.

- The client parses MPD and put it in DOM.

- The client play the presentation using DOM

In the case of receiving an MPD update, it is not clear whether the MPD update rules applies to MPD before or after resolving the xlink:

- If it is before, then requesting the xlink may result to a different resolution, and therefore an update MPD results in a different period in the middle of play of the previously resolved-currently played period.

- If it is after xlink resolution, then once the xlink is resolved, the same resolution should be used in all MPD updates, i.e. the MPD packager should guarantee that the link always resolved to the same resolution.

XML xlink is designed for one-time resolution of the resource (traversal). XLink works in MPD if the resolution only occurs once. If the resolution needs to be updated, XML processing model neither addresses the use-case, nor is adequate for consistent playback by all clients.

### Proposed solutions in MPEG#130

Because of the above problems, three proposals were submitted to MPEG#130.

#### From m52959: implicit caching policy

When a resolution event occurs, the internal cache is updated, and the appropriate replacement is executed. This cache captures resolution results until either the presentation ends or the element containing the resolution identifier falls completely out of the availability window.”

“each remote resolution entity would be assigned a unique identifier by the DASH client (resolution connected Period sets have a single identifier) and the client uses this identifier to internally cache the resolution results it has obtained.”

#### From m52955: implicit caching policy

"When de-referencing a period xlink with xlink:actuate value not equal to "onLoad" and the parent MPD @type attribute is set to dynamic, the de-referencing must be done when the period is about to become active and must not be re-evaluated during subsequent MPD updates happening while that period is active.

NOTE: this implies that a dynamic MPD processor has some internal caching mechanism of resolved XLink for past and current periods."

#### From m52954: explicit caching signaling

“For each remote element in the manifest, the following additional parameters are described for caching purposes:

- Maximum age: the duration in seconds for the cached resolved resource is valid after resolution. The cached resolved resource will expire after this duration and must be purged.

- Revalidate flag: the flag to purge the resolved resource in cache if exists, and resolve it again.

- Extend flag: the flag to extend the lifetime of resolved resource in the cache if it is accessed again before its expiration”

### Is the resolution consistency challenge unique to xlink?

One suggestion could be to use xlink for static content and use a different hyperlink for dynamic MPDs. Would this approach be free of the resolution consistency issues?

We argue that any hyperlink which may be resolved multiple times and the result of its resolution would be dependent to the history and timing of the request, must have a state that is preserved in either:

- the server serving the resolution, or

- the local cache in the client

The reason for this is simple. When a DASH client plays a live MPD, the MPD may get updated. The DASH client may use MPD pulling or MPD update events to get the MPD update. If the MPD includes of a just-in-time hyperlink, such a  OnRequest xlink, then the hyperlink must be resolved at the time the DASH client wants to play the relevant element. That element may be a period for instance. Since the DASH client gets an update of MPD, it has to either have strict rules to not resolving the previously resolved hyperlink, or it has to have explicit signaling on re-resolving the link.

An example of this characteristic is to use the hyperlink provided by an event at the period for replacing the main content as shown in Figure 4:

time

timeShiftbuffer

Duration of event

Pr (Alternative period resolved by a hyperlink in an event)

Event1

Figure 2. hyperlink resolution of a link carried in an event

Let’s assume that the manifest includes an event with start time equal to Event1@start, which provides a link. When the client requests the link, an alternative Pr period is provided by the server. The client is expected to play Pr instead of the original content.   If the client passes through Event1@start point more than once, such as during an MPD update, the client would request the link for Pr more than once.  Such a situation can occur in the following cases:

- The client received an MPD with Event1.

- It parses the MPD. If an OnRequest behavior is desired, the client requests the hyperlink at Event1 start time.

- If the client plays the period again due to random access, does it request the link again? If it doesn’t, then the client is holding a caching age for the resolved xlink. If the client resolves it again and the resolution is a new period, is it ok that the client plays a different content due to random access?

- In the case of an MPD update in the middle of the Pr period, the client parses updated MPD in DOM. How does the client know not to request the hyperlink at this point? Does it have a caching model?

Therefore, in general:

  - The responding server must maintain a state for the hyperlink, so it provides the same or a different Pr depending the case, or

  - The client must maintain a local cache and use the previously resolved Pr.

### General local caching model for remote elements

Figure 1 describes the local cache processing model for remote elements. The interfaces between two content servers and a DASH client with a local cache are shown by IF1 and IF2, respectively. The DASH client has an internal cache for remote link resolutions.

DASH Client with cache

Content Server 1

Content Server 2

DASH Client

IF1

IF2

Remote link

local cache

Figure 1. Local cache model

The local cache of the DASH client operates similar to the HTTP caches. The internal cache purging process removes the items that are expired. The cache as an internal clock (which could be the same clock as DASH client) and manages the purging based on the clock.

### HTTP Cache processing model

According to RFC2616:

“The basic cache mechanisms in HTTP/1.1 (server-specified expiration times and validators) are implicit directives to caches. In some cases, a server or client might need to provide explicit directives to the HTTP caches. We use the Cache-Control header for this purpose.”

The cache control of a local browser by the server is shown in the following figure:

Standard Cache-Control directives that can be used by the server in an HTTP response.

- Cache-Control: must-revalidate

- Cache-Control: no-cache

- Cache-Control: no-store

- Cache-Control: no-transform

- Cache-Control: public

- Cache-Control: private

- Cache-Control: proxy-revalidate

- Cache-Control: max-age=<seconds>

- Cache-Control: s-maxage=<seconds>

Definition of a few directives are listed here:

no-store

The response may not be stored in any cache. Although other directives may be set, this alone is the only directive you need in preventing cached responses on modern browsers. max-age=0 is already implied. Setting must-revalidate does not make sense because in order to go through revalidation you need the response to be stored in a cache, which no-store prevent

must-revalidate

Indicates that once a resource becomes stale, caches must not use their stale copy without successful validation on the origin server.

max-age=<seconds>

The maximum amount of time a resource is considered fresh. Unlike Expires, this directive is relative to the time of the request.

s-maxage=<seconds>

Overrides max-age or the Expires header, but only for shared caches (e.g., proxies). Ignored by private caches.

### Cache processing model for DASH client

In the HTTP cache model, when a client parses a remote element in the received manifest, the cache receives the following parameters from the client:

- URL to access the remote element

- Period@id

- MaxAge: the maximum age of the resolved element

Receive xlink request:

- URL

- Period@id

- Max-age

URl entry exists in cache?

Yes

No

Expired?

Cache entry:

- URL

- id

- Resource

- Expiration time

- Extend flag

No

Provide resource to DASH client

Request xlink

Create/update cache entry

Yes

Figure 3. Cache flowchart

The local cache operates in the following steps as shown in Figure 3:

- Checks whether the URL exists in the cache. If the entry doesn’t exist, it goes to Step 3, otherwise continue.

- Checks if the cache entry’s expiration time has passed. If the time is expired, go to Step 3, otherwise go to step 5.

- It sends the URL request and receives the resolved resource.

- It creates/update the entry in the cache with the following items:

  - URL

  - Id

  - Resolved resource

  - Expiration time

- Provide the resolved resource to the DASH client.

This is identical to the HTTP cache control model.

### Media time vs Wall Clock time

#### On-demand content (static MPD)

Let’s consider the case of static MPD. The timeline for a static MPD is shown in the following figure:

Media time

P0

P1 (remote element)

PeriodStart

Period@duration

Pn

MaxAge

Figure 4. The timeline for static MPD

In this case, the cache clock is on the media timeline. Therefore, the Maxage of an item in the cache is defined based on this timeline. As an example, if the maxAge of a resolved P1 element is set to that Period’s duration, the resolution would be expired after playing the entire P1 period. On the other hand, if the MaxAge is set to the program duration, the resolved P1 would new get expired.

#### Live content (dynamic MPD)

Let’s consider the case of dynamic MPD. The timeline for a dynamic MPD is shown in the following figure:

Media time

P0

P1 (remote element)

PeriodStart

Period@duration

Pn

MaxAge

Wall clock time

AST

PeriodStart

Figure 5. The timeline for dynamic MPD

In this case, the media time is anchored to the wall clock time using the AST and the P0 PeriodStart time. Therefore, any Period’s start time can be mapped to the wall clock time.

Similarly, the cache clock can work the wall clock time. As an example, if the maxAge of a resolved P1 element is set to that Period’s duration, the resolution would be expired at a certain moment in the wall clock time (P2 start time).

### Cache model for implicit signaling proposed by other proposals

“When a resolution event occurs, the internal cache is updated, and the appropriate replacement is executed. This cache captures resolution results until either the presentation ends or the element containing the resolution identifier falls completely out of the availability window.”

The following figure demonstrates the cache model for the above implicit signaling:

time

P0

P1  (remote element)

PeriodStart

timeShiftbuffer

Period@duration

MaxAge

Figure 6. The cache model for implicit signaling

"When de-referencing a period xlink with xlink:actuate value not equal to "onLoad" and the parent MPD @type attribute is set to dynamic, the de-referencing must be done when the period is about to become active and must not be re-evaluated during subsequent MPD updates happening while that period is active.”

In this case, MaxAge = Period@duration.

### Difficulties with implicit cache signaling

In the case of implicit cache signaling, the max-age of an item in the cache, or commands such as must-revalidate must be derived based on the state and timing of the client with the respect to an MPD. While it is possible to derive such rules for each specific case, developing it per case is difficult and not future proof. Explicit cache signaling is simple and clear for a client. It knows what exactly to do with each item in the cache and therefore its behavior can be assured to the service provider. Also, a service provider can use the cache-signaling to address its new use-cases, which might not be the same as the default one.

### Why signaling in the MPD and not in HTTP headers?

It is much easier to add the cache-control rules to each remote element of MPD, rather than to keep the information outside of the MPD and then provide with the resolution of the link. Furthermore, the entity resolving the link resolution might be different than the one providing the MPD. Therefore, the two servers may not have the means of sharing these caching information.

### Proposal

Any hyperlinked element in MPD that may get resolved multiple times needs a state. Therefore, we propose in any solution we adopt, we include a caching model that has clear signaling on the resolved item in the cache. We propose an explicit model so that all clients have the same behavior with any resolved hyperlink.

The element may have a set of attribute similar to the HTTP cache:

<Element href=”http://example.com/blahblah” actuate= “onRequest” other(s) >

Therefore, a following semantics is proposed to be added to ISO/IEC 23009-1:

Table 1 — Additional attributes for MPD element with remote link capability

| Element or Attribute Name | Use | Description |
| --- | --- | --- |
| (Element name) |  |  |
| @href | O | specifies a reference to a remote element entity that is either empty or contains one or multiple top-level elements of type (Element name) |
| @actuate | OD default: onRequest | specifies the processing instructions, which can be either "onLoad" or "onRequest". This attribute shall not be present if the @href attribute is not present. |
| @mustRevalidate | O | If ‘true’, any stored resolution is not valid and the link shall be resolved again. |
| @nostore | O | If ‘true’, the resolution of the link is expected not to be stored in any cache for future use. |
| @maxAge | O | duration for which the resolution stays valid: If the value is a <number>, the resolved link expires after <number> seconds. If the value is ‘forever’, the resolved link never expires. If the value is ‘current’, the resolved link expires as soon as any content is accessed from any period but the current one that is being played. If the value ‘Insidetimeshift’, the resolved link expires the current period is outside of the timeShiftBuffer. For MPD@type=’static’, the reference clock for this value is the DASH player’s media timeline clock. For MPD@type=’dynamic, the reference clock for this value is the DASH player’s wall clock. |
| @expires | O | The time when the resolved link expires. For MPD@type=’static’, the reference clock for this value is the DASH player’s media timeline clock. For MPD@type=’dynamic, the reference clock for this value is the DASH player’s wall clock. |
| @extend | O | duration for which the resolution validity is extended: If the value is a <number>, the resolved link is extended by <number> seconds. If the value is ‘forever’, the resolved link never expires. If the value is ‘current’, the resolved link expires as soon as any content is accessed from any period other than the current one that is being played. If string = ‘Insidetimeshift’, the resolved link expires the current period is outside of the timeshift buffer. For MPD@type=’static’, the reference clock for this value is the DASH player’s media timeline clock. For MPD@type=’dynamic, the reference clock for this value is the DASH player’s wall clock. |

# Adding haptics functionality (m54495- updated at MPEG#132)

## Purpose

This section contains proposed changes to the ISO/IEC 23009-1 (MPEG-DASH Part 1) standard to enable playback of haptics in streaming media content. Haptics has been proposed as a new first-order media type in the ISOBMFF standard. That proposal has moved to DAM at MPEG132.

## References

- MPEG-DASH Part 1 Standard (ISO/IEC 23009-1: MPD and Segment Formats)

  - https://www.iso.org/standard/79329.html

  - Hereby referred to as MPEG-DASH

- MPEG4 Part-12 Standard (ISO/IEC 14496-12 ISO Base Media File Format)

      - https://www.iso.org/standard/71851.html

      - Hereby referred to as ISOBMFF

    - IETF RFC 6838:2013, Media Type Specifications and Registration Procedures

      - https://tools.ietf.org/html/rfc6838

      - Hereby referred to as RFC6838

    - IETF Haptics Standards Track RFC Internet Draft

      - https://datatracker.ietf.org/doc/draft-muthusamy-dispatch-haptics/

      - Hereby referred to as IETF Haptics ID

## Haptics in MPEG-DASH

Haptics provide an additional layer of entertainment and sensory immersion to the user. Therefore, the user experience and enjoyment of streaming media content such as ATSC 3.0 broadcasts, streaming games, and mobile advertisements can be significantly enhanced by the judicious addition of haptics to the audio/video streams. Since haptics will be part of the ISOBMFF standard, haptic tracks will be part of the .mp4 Segments that are included in the MPEG-DASH  Representations. Therefore, we do not envision any changes to the three ISOBMFF profiles (Main, On-Demand, Live) that are currently included in MPEG-DASH.

However, we do propose the following necessary changes to MPEG-DASH to support haptics.

### New Media Component Content Type - ‘haptics’

A mechanism to signal haptics as a new media component type (like audio, video, subtitle, etc.) is needed. Currently, text, image, audio, video, application, and font are the recognized content types in MPEG-DASH, as per RFC6838. We have proposed adding ‘haptics’ as a new media content component type to RFC6838. An IETF Haptics ID containing that proposal is currently under review by the IETF Dispatch WG.

### Haptic Channel Configuration

Contribution m55200 (Haptics CICP) proposed a table that contains a reference set of haptic channel configuration schema similar to the audio channel configuration schema provided in Table 3 of ISO/IEC 23091-3. We envision this table to be included in (a new) Part 5 of the ISO/IEC 23091 (Haptics CICP) standard that defines various code points and fields that establish properties of a haptics stream that are independent of the compression and bit rate.

The @schemeIdUri attribute of an EssentialProperty descriptor can be used to identify the haptic channel configuration scheme employed. Multiple EssentialProperty elements may be present indicating that the DASH Representation supports multiple haptic channel configurations. For example, it may describe a Representation that includes supporting both single actuator vibration devices (like mobile phones) as well as three-actuator vibration devices (like a haptic seat).

NOTE: If the scheme or the value for this descriptor is not recognized, the DASH Client is expected to ignore the descriptor.

## Patent Rights Declaration

Immersion Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

## Informative Annex A: Mobile Phone to Home Use Case

Mobile Phone to Home

A user is watching a streaming video on their phone using phone speakers/headphones and feeling haptics using the phone’s actuator. The user gets home, and the phone automatically connects to the home devices via Bluetooth. As a result: - Audio is now routed to the in-home entertainment speakers - Haptics is transferred to the in-seat haptic display (if available) - Video becomes available on the home TV In case of multiple entertainment setups in the home (family room system, bedroom system, media room system) the player provides the option to select the system of choice, or automatically detects it based on Bluetooth proximity sensing.

Overlap with other use cases

Required features

An EssentialProperty descriptor in the DASH MPD provides the available channel configurations in the current AdaptationSet. This attribute shall enable the player engine to select an alternate Representation at the next segment boundary after the user has connected to his/her home entertainment system. For example, while viewing the media on the mobile phone, the EssentialProperty descriptor will have a @value = 1 (notation: 1.0.0) or 2 (1.1.0), as per Table 1 in m55200. Once the user’s phone connects to their home theater system, the player will look for a Representation with a @value = 5 (x.x.0; actuator array) for the in-seat haptic display. If the AdaptationSet does not have a Representation with a @value = 5 choice, the player can transcode the single channel mobile haptic signal onto the actuator array in the haptic seat at home.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex B: Mobile Phone to Car Use Case

Mobile Phone to Car

A user is watching a streaming video on their phone using phone speakers/headphones and feeling haptics using the phone’s actuator. The user gets into their car and connects to the car systems via Bluetooth. As a result: - Audio is routed to the vehicle speaker systems - Haptics is transferred to the in-seat haptic display - Video becomes available on the car’s video display (when available and subject to safety regulations, e.g., car must be fully autonomous)

Overlap with other use cases

Required features

An EssentialProperty descriptor in the DASH MPD provides the available channel configurations in the current AdaptationSet. This descriptor shall enable the player engine to select an alternate Representation at the next period boundary after the user has connected to the vehicle. While viewing the media on the mobile phone, the EssentialProperty descriptor will have a @value = 1 (notation: 1.0.0) or 2 (1.1.0), as per Table 1 in m55200. Once the user’s phone connects to their vehicle, the player will look for a Representation with a @value = 5 (x.x.0; actuator array) for the vehicle in-seat display. If the AdaptationSet has a Representation that includes an attribute for an actuator array, this may be selected to render on the user’s seat in the vehicle. If the AdaptationSet does not have this attribute choice, the player may choose to transcode the single channel mobile haptic signal onto the actuator array in the vehicle or to render nothing.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex C: Haptics Headphones Use Case

Haptic Headphones

A user is watching a streaming video on their phone using phone speakers and feeling haptics using the phone’s actuator. They don a pair of Bluetooth connected headphones that include haptic feedback functionality for each ear. The two haptic actuators (left and right) become available to the DASH player during playback as independent haptic output devices.

Overlap with other use cases

Required features

The basic media encoding can have alternate haptic tracks in the same stream as the audio/video and rely on the player to parse and allocate these alternate tracks depending on the type and number of available haptic devices. While viewing the media on the mobile phone, the EssentialProperty descriptor will have a @value = 1 (notation: 1.0.0) or 2 (1.1.0), as per Table 1 in m55200. Once the user’s phone connects to their Bluetooth headphones, the player will look for a Representation with a @value = 4 (0.2.0; Dual HD vibration in left/right configuration) for the haptic headphones. The DASH MPD may also describe these distinct choices of haptic tracks as different Representations for a particular media stream that a user can select or that the player can select at the next segment boundary given the change in endpoint device configuration.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex D: Companion Device (ATSC 3.0) Use Case

ATSC 3.0 “Companion Device” Viewing

A user is watching ATSC 3.0 broadcast content on their home TV (“primary device” in ATSC 3.0 parlance). The user decides to watch the same content on his/her mobile phone or tablet (“companion device” in ATSC 3.0 parlance) as well, mainly because of the possibility of supplemental content on the companion device. As a result: - Audio is routed to the TV or home entertainment speakers (as before) as well as the companion device speakers - Video is on both the TV and the companion device - Haptics track is on the companion device and playing in sync with the media on the companion device. It could also be on the in-seat haptic display, if available. - The user is also able to configure the haptics experience based on the content type (e.g., live sports or action video content) as well as his/her personal preferences.

Overlap with other use cases

Required features

An EssentialProperty descriptor in the DASH MPD provides the available channel configurations in the current AdaptationSet. This descriptor element will enable the player engine to select an alternate Representation at the next segment boundary after the user has connected to the companion device. For mobile companion devices, the EssentialProperty descriptor will have a @value = 1 (notation: 1.0.0) or 2 (1.1.0), as per Table 1 in m55200. If the AdaptationSet has a Representation that includes an attribute for an actuator array (@value = 5), this may be selected to render on the user’s in-seat haptic display. If the AdaptationSet does not have this attribute choice, the player can transcode the single channel mobile haptic signal onto the actuator array in the user’s seat. If a haptic seat is not available, the single channel mobile haptic signal is rendered as-is to the user’s companion device.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex E: Live streaming of Games Use Case

Live Streaming of eSports and Interactive Cloud Gaming

Streaming A user is watching a live broadcast of an eSports event on their mobile device. The players are using single-channel haptic devices and this stream of 2-SD actuator haptic data is available. The player on the user’s mobile device transcodes the 2-channel haptic data into a single channel and renders it locally on the user’s mobile. Interactive Cloud Gaming A user is playing a cloud-based game using a local haptic controller. The downstream media stream is generated in real-time by the server and is bandwidth-optimized in real-time. The game tries to preserve haptics at a higher fidelity than video and audio streams. The user is using a DualShock style controller with two SD haptic actuators or a DualShock controller with two HD actuators and two kinesthetic triggers (e.g., Sony PS5).

Overlap with other use cases

Required features

The DASH MPD may be dynamic in this case: @type="dynamic" to enable changes in controllers during the game. The InitializationSet and subsequent AdaptationSet elements should specify either @value = 3 or 6 for the EssentialProperty descriptor (i.e., 2.2.0 or 2.2.2; dual vibration motors or dual vibration and trigger, respectively, using the proposed notation in Table 1 in m55200). They player may additionally have algorithms to enable basic transcoding such as mixing of haptic Representation data from multi-channel to single-channel output devices if appropriate (e.g., watching dual vibration content on a single vibration endpoint such as a mobile phone).

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex F: Haptics from Different Perspectives Use Case

Haptics from Different Perspectives and Viewpoints

A user is watching a video stream on their phone augmented with haptics felt through the mobile device, or on a screen at home with the haptics displayed through an array of actuators embedded in wearable devices and furniture. The user has the option to exclusively experience a specific haptic track related to a specific feature of the content. The features include: specific content events (e.g., bumps, gunshots, kicks received etc.), specific characters (e.g., the fighter in the red corner in a MMA stream), specific objects (e.g., a car’s exhaust rumbling in a NASCAR race) and specific locations in the scene (e.g., touchdown area on a football field). The DASH player would then route the chosen track to the haptic playback system. The user can also choose to simultaneously experience multiple haptic tracks, each one related to a specific feature of the content, by selecting these features. The DASH player would then combine and mix the different tracks related to the different selected features, into a single track to be routed to the haptic playback system. Moreover, the user can associate specific tracks/features with one or more specific actuators, in the case of an array of actuators. The DASH player would then combine and mix the tracks dedicated to each actuator separately and re-route the resulting tracks to the specific actuators separately.

Overlap with other use cases

Required features

The basic media encoding can have multiple haptic tracks for the same audio/video stream, related each to a specific feature of the content (i.e. event, character, location). The user may select which features to feel and the player would then mix the selected features tracks and display the mixed track. The stream also contains a default pre-mixed haptic track encompassing all features. The DASH MPD may contain distinct available choices of haptic Representations in the current AdaptationSet. The @schemeIdUri attribute of an EssentialProperty descriptor will specify the different haptic channel configurations available. This will enable the player engine to select an alternate Representation at the next period boundary after the user modifies his haptic features and playback devices scheme, forcing the player potentially to partially mix some haptic tracks (i.e., mix a subset of) and to re-route them to alternate playback actuators. For example, while viewing the media on the mobile phone, the EssentialProperty descriptor will have a @value = 1 or 2 (mobile phone) in the case of content being played back similarly across all available actuators. Once the user decides to play different tracks on the two available actuators, the player will choose a Representation with a @value = 3 for the EssentialProperty descriptor. While viewing the media on a screen at home, a Representation with @value = 11 (denoting multiple vibration and kinesthetic playback devices) will be selected.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex G: Haptics on Connected Devices Use Case

Personal Haptic Device Connected to a Nearby AV(H) Device

A user is carrying a personal haptic device (e.g., mobile phone, wearable watch or band, haptic glasses, haptic vest, etc.) then she walks into a space area (e.g. bar, restaurant, living room), where a multimedia device is displaying audio visual information, possible haptic information also, although the user might not be capable of knowing as she is not in contact with the device. When the user walks into the new space, the haptic personal device might connect to the local Wi-Fi of the space or Bluetooth corresponding to the AV(H) device. As a result: - The haptic track is transmitted to the personal haptic device, either directly from the broadcasting station or from the local AV(H) device. - The haptic track can contain information about the entire event (e.g., sports) or simple alerts to the user when important events happen. The AV tracks could also be routed to the personal haptic device if the user requests that, and if the device can play back such tracks. The different personal haptic devices can play a haptic track tailored to the features of each specific device. Alternatively, each device could play alerts or an entire track for the AV content depending on configuration of the device or the event, or user preferences.

Overlap with other use cases

Required features

The basic media encoding may have multiple haptic tracks for the same audio/video stream, related each to a specific personal haptic device. The user can select which features to feel in each specific device. The DASH MPD may describe these distinct choices of haptic tracks as different Representations for a particular media stream that a user can select or that the player can select at the next segment boundary given the change in endpoint device configuration.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex H: Haptics-Enabled Ads Use Case

Haptic Ads

A user is watching a live streaming broadcast on their phone and the broadcaster inserts a haptic ad mid-roll dynamically based on the broadcast content.

Overlap with other use cases

Required features

The DASH MPD should be dynamic in this case: @type="dynamic" to enable mid-roll advertising. The server pushes an update to the MPD sufficiently in advance of the end of the current period that contains a new collection of AdaptationSet elements for the static ad content. Within these AdaptationSet elements are one or more Representations that have EssentialProperty descriptors representing available haptic configurations (typically @value = 0 or 1 for generic ad content; see Table 1 in m55200). Since the user is watching the live broadcast on their mobile phone, the player may select the segment that contains a single SD or HD vibration track for the ad content and then render this along with the ad video/audio content.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

## Informative Annex I: Tactile Essence-Based Haptic Track Use Case

Tactile Essence

A live event is utilizing sensors that capture tactile related data consistent with SMPTE st2100-1-2017 (Coding of Tactile Essence). This sensor data is a set of 3-axis linear accelerometers for individual players in a football game. The broadcaster would like to make available user-selectable tactile experiences for each available player’s tactile essence. The server transforms the tactile essence for each player in real-time to generate a set of 1-dimensional tactile signals (one signal per player). These signals are exposed to the client through dynamic updates to the MPD.

Overlap with other use cases

Informative Annex F: Haptics from Different Perspectives Use Case

Required features

The server shall be able to transform multi-axis tactile essence signals into 1-dimensional tactile tracks, encoded as ISOBMFF (Live profile) streams, in real-time and with appropriate synchronization to related audio and video streams.

Identified Gaps and Optimization Potentials

Potential Requirements and Specifications

The MPD shall include one or more EssentialProperty descriptor elements whose @schemeIdUri attribute identifies the haptic channel configuration scheme employed . Table 1 in m55200 contains the labels and values for different haptic channel configurations.

Failover in multi-origin linear deployments (m54725)

## Introduction

In many cases multiple origins are used to serve the same content. In particular, in a geo-redundancy case same channel is transcoded by different transcoders and published to different origins. In case one origin fails, the player can seamlessly or near-seamlessly switch to a different origin.

In the naïve implementation, the client will quit on an origin failure and start with a new URL pointing to a different origin. This solution is unacceptable, as it incurs a significant delay due to buffering, tearing down the current DRM session, getting a new URL from the back-end, downloading the MPD and initialization segments, reinitializing the decoder, requesting new DRM license, setting up the new DRM context, etc. The overall effect can be very noticeable, as the player will be down for e.g. 30 seconds. Moreover, an origin failure affecting hundreds of thousands or millions of concurrent sessions will create a “thundering herd” effect on the licensing servers as hundreds of thousands or millions of license requests will come at the same time, taxing the license server infrastructure.

DASH provides two mechanisms to reduce the downtime: Fallback MPD chaining and MPD reset.

Fallback MPD chaining provides the client with an alternative MPD it can use in case of a catastrophic failure such as failure to do normal MPD update. MPD reset tells the client that it has to switch to playback of a new MPD at a given time. In both cases, the player can finish playing its current buffer and switch to the new MPD. While this is an improvement over the naïve implementation, the DRM issue remains.

In case we continue viewing the same channel, there typically is no need to request a new license – if the viewer was authorized to view a channel from one origin, then he/she will be authorized to view it from a different origin. As long as the origins have matching encryption keys (even if `pssh` is different), there is no need to re-request them or do anything to the secure rendering pipeline.

A different issue is preserving user experience. For example, if the MPD offers multiple views of the same content, we want to continue watching the same view we were watching prior to the failover. In case two MPDs have same adaptation set IDs, we can switch to precisely the same view.

These types of continuity need to be somehow signaled to the player which currently cannot tell whether it is switching to a slate with an error message or to a very similar MPD with different segment URLs and difference in segment timing. This type of signaling is proposed below.

## Proposal

### Approach

We define several types of continuity:

- Asset continuity: both MPDs show the same visual content and have approximately the same live point. It is hard to get to precisely same live point when the encoders are at significantly different geographic locations and same frame enters different transcoders at a different time due to speed of light.

- Service continuity: the adaptation set and representation IDs are identical, so it is possible to simplify the switch logic to follow the currently playing IDs as opposed to starting from a “clean slate”. Additionally, the same events are synchronized (i.e., the events are the same in their semantics), which is important in terms of advertising logic and SCTE 35.

- Content protection continuity: as long as same key IDs are used, there is no need to re-request the DRM license

Note that the structure here is not a perfect “onion structure” – content protection continuity requires asset continuity, but not service continuity.

We propose a new element to explicitly provide such signaling and explicitly call out different types of continuity.

### Proposed syntax and semantics

The Continuity element describes various aspects which are maintained across representations described by successive MPDs. This element is embedded inside a SupplementalProperty or EssentialProperty element with @schemeIdUri attribute values of urn:mpeg:dash:fallback:2016 or urn:mpeg:dash:reset:2016.

Table 25 — Semantics of Continuity element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Continuity |  |  |  |
|  |  |  | @asset | OD default=false | If true, content of the new presentation is perceptually identical to the content of the current presentation, i.e. the “live point” of one presentation is within 250ms from the other |
|  |  |  | @service | OD default=false | If true, adaptation set, representation, and sub-representation IDs are identical across the new and old representation |
|  |  |  | @contentProtection | OD default=false | If true, the DRM license delivered for segments in the old presentation, is still true for segments in the new presentation with same key IDs |
| Key For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory For elements: <minOccurs>..<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |

# Track Derivation based Server-Side Dynamic Adaptation in DASH (m55222, m56824 and m57449)

## Introduction

The revised CD text [1] of MPEG-B part 16, “Derived Visual Tracks in ISOBMFF”, contains a preliminary set of derivation transformations: Identity, Dissolve, Crop, Rotate, Mirror, Scaling, Region-of-interest, and Track Grid. More derivation transformation candidates are documented in the TuC [2], including more composition and immersive media processing related ones.

To this 132nd MPEG meeting, contribution [9] proposes derivation transformations for track derivations resulting from track selection and switching, with applications targeting at selective playback and adaptive streaming, where tracks are typically grouped into alternate and switch groups, respectively.

This contribution proposes server-side dynamic adaptation (SSDA) in DASH using a single representation in an AdaptationSet corresponding to a derived visual track for switching among a set of switchable visual tracks as proposed in contribution [9].

## Derived Visual Tracks and Those for Track Selection and Switching

Derived visual tracks are designed to enable defining a timed sequence of visual transformation operations to be applied to input still images and/or samples of timed sequence of images. It is built using tools defined in the ISO base media file format (ISO/IEC 14496-12). The latest CD [3] specifies the core design and a set of transformation operations (e.g., identity, rotation, scaling and grid composition), and the latest TuC [4] includes some generic transformation operations such as static and dynamic overlays as well as some immersive media processing related transformations such as stitching, projection, packing and viewport.

In essence, derived visual tracks whose samples are derivation transformations can be used to generate, from input tracks or images, an output visual track with visual samples, as follows:

Thus, if a derived visual track is of a track selection transformation, its track derivation is to select a desired visual track (e.g., according to a list of parameters) from its input visual tracks, and if a derived visual track is of track switching transformation, it track derivation is to switch among input visual tracks to a desired visual track.  This kind of capability leads to its applications to implementations of dynamic adaptation in adaptive streaming systems, especially those server-based using DASH.

A selection-based track derivation as proposed in contribution [9] allows selection of samples of a track from an alternate or switch group at the time of derivation, and it has the following advantages:

- It provides a track encapsulation for track samples selected or switched from an alternate or switch group, which is what the existing track selection mechanism using alternate and switch groups does not provide.

- This track encapsulation allows straightforward association of metadata about a selected or switched track with its track encapsulation itself, rather than with a track group from which the track is selected or switched. For instance, in order to say a track selected from a track group at run time has a region of interest (ROI), it becomes very easy and nature to signal the ROI in the metadata box ('meta') of the derived track when the ROI is static, and use a timed metadata track to reference the derived track, using reference type'cdsc', when the ROI is dynamic. In contrast, there is no direct way to signal the ROI metadata without a derived track: signaling a static ROI in the metadata box of every track in an alternate or switch group does not convey the same meaning as it means that every track has the static ROI, and having a timed metadata track representing a dynamic ROI to reference an alternate or switch group needs to define a new track reference type, as the existing track reference in the track reference box (sub-clause 8.3.3 in [3]) states, when it applies to referencing a track group, “the track reference applies to each track of the referenced track group individually”, which is also not the desired meaning.

- The derived track encapsulation enables definitions and executions of track-based media processing workflows, such as in network based media processing [5], to use derived tracks not just as outputs but also intermediate inputs in the workflows.

- The derived track encapsulation allows it possible for track selection or switching to be transparent to clients of dynamic adaptive streaming, such as DASH [6], and carried out at corresponding servers or within distribution networks, for instance, implemented in conjunction SAND [7]. This can help simplifying client logics and implementations with respective to shifting dynamic content adaptation from the streaming manifest level to the file format derived track level, for instance, based on the descriptive and differentiating attributes defined in sub-clause 8.3.3 in [3]. With selection-based derived tracks, DASH clients [6] and DASH aware network elements (DANE) [7] can just provide values of attributes (e.g., codec 'cdec', screen size 'scsz', bitrate 'bitr') required in the derived tracks (according to their definitions), and let media origin servers and CND’s provide content selection and switching from a group of available media tracks. This may then result in eliminating use of AdaptationSet or restricting its use to just containing a single Representation in DASH.

## Track Derivation based Server-Side Dynamic Adaptation in DASH

This section provides more details on how derived selection and switch tracks can be used to implement server-side streaming adaptation (SSSA), as opposed to client-side streaming adaptation (CSSA), in adaptive streaming systems.

In a generic architecture of an adaptive streaming system as shown in the diagram below [6],

it has been the Streaming Client that implements some Adaptation Logic for streaming adaptation, including static as well as dynamic adaptation, in selecting segments from different media streams according to some Adaptation Parameters (i.e., “MPD Selection Metadata” in [6] such as bitrate), depicted as follows:

More intuitively in an end-to-end streaming media processing diagram, it is the Client that performs streaming adaptation in terms of selecting (encrypted) segments from a set of available streams.

At a high level, this kind of CSDA workflow consists of the following steps between the Client and Server (or CDN):

Now thanks to derived selection and switch tracks proposed in [9] that enable track selection and switching, at run time, from an alternate track group and a switch track group, respectively, streaming adaptation can be performed at the server side, instead of the client side, to simplify Streaming Client implementation, as follows:

It should be noted that it is feasible to use derived selection and switch tracks to implement CSSD, but may not be efficient in term of distributing tracks (or segments), as all but one will end up with not being used.

Thus, in an end-to-end streaming media processing diagram, it is now the Server that performs streaming adaptation in terms of selecting (encrypted) segments from a set of available streams.

At a high level, this kind of (SSSA) workflow consists of the following steps between the Client and Server (or CDN):

When derived switch tracks are used to implement SSSA, the workflow above can be modified as follows; namely the segment request can be made from a derived switch track:

It should be noted that in the SSSA scheme, the client can still make some static selection (such as those related to video codec profile, screen size and encryption algorithm), and only leave dynamic adaptation (such as those related to video bitrate, network bandwidth) to the server, by collecting and passing dynamic adaptation parameters needed for Adaptation Logic to the server as part of (http) segment requests. The communication of these adaptation parameters can be implemented in anyone of the following mechanisms:

- URL query parameters

- HTTP  header parameters

- SAND messages (carrying adaptation parameters collected by the client and other DANE’s) [7].

## Track Selection and Switching

The list of parameters is the list of differentiating attributes specified in clause 8.10.3, “track selection box”, in the latest ISOBMFF specification [2] for the purpose of track selection or switching:

| Name | Attribute | Pointer |
| --- | --- | --- |
| Codec | 'cdec' | Sample Entry (in SampleDescriptionBox of media track) |
| Screen size | 'scsz' | Width and height fields of VisualSampleEntry. |
| Max packet size | 'mpsz' | Maxpacketsize field in RtpHintSampleEntry |
| Media type | 'mtyp' | Handlertype in HandlerBox (of media track) |
| Media language | 'mela' | Language field in MediaHeaderBox |
| Bitrate | 'bitr' | Total size of the samples in the track divided by the duration in the TrackHeaderBox |
| Frame rate | 'frar' | Number of samples in the track divided by duration in the TrackHeaderBox |
| Number of views | 'nvws' | Number of views in the track |

It is agreed that most of these parameters are used in DASH, possibly with different names. Nevertheless, they are in the DASH namespace.

It is true that a DASH device selects “selects the tracks based on its screen size”, but that is the client-side dynamic adaptation. For server-side dynamic adaptation, the server would need to know client’s screen size.

## Viewport and Viewpoint Selection

The list below is a collection of viewport/viewpoint/spatial-object related data structure attributes from OMAF [3] (spherical region), V3CD [4] (cuboid region) and DASH [5] (planar region).

| Name | Attribute | Pointer |
| --- | --- | --- |
| Azimuth | 'azim' | Azimuth component of a spherical viewport [OMAF] |
| Elevation | 'elev' | Elevation component of a spherical viewport [OMAF] |
| Azimuth range | 'azim' | Azimuth range of a spherical viewport [OMAF] |
| Elevation range | 'elev' | Elevation range of a spherical viewport [OMAF] |
| Position x | 'posx' | x coordinate of a position in a reference coordinate system, for a viewpoint, viewport or camera [V3CD] |
| Position y | 'posy' | y coordinate of a position in a reference coordinate system, for a viewpoint, viewport or camera [V3CD] |
| Position z | 'posz' | z coordinate of a position in a reference coordinate system, for a viewpoint, viewport or camera [V3CD] |
| Quaternion x | 'qutx' | x component of the rotation of a viewport or camera using the quaternion representation [V3CD] |
| Quaternion y | 'quty' | y component of the rotation of a viewport or camera using the quaternion representation [V3CD] |
| Quaternion z | 'qutz' | z component of the rotation of a viewport or camera using the quaternion representation [V3CD] |
| Object x | ‘objx’ | non-negative integer in decimal representation expressing the horizontal position of the top-left corner of the Spatial Object in arbitrary units [DASH] |
| Object y | ‘objy’ | non-negative integer in decimal representation expressing the vertical position of the top-left corner of the Spatial Object in arbitrary units [DASH] |
| Object width | ‘objw’ | non-negative integer in decimal representation expressing the width of the Spatial Object in arbitrary units |
| Object height | ‘objh’ | non-negative integer in decimal representation expressing the height of the Spatial Object in arbitrary units [DASH] |
| Total width | ‘totw’ | optional non-negative integer in decimal representation expressing the width of the reference space in arbitrary units [DASH] |
| Total height | ‘toth’ | optional non-negative integer in decimal representation expressing the height of the reference space in arbitrary units [DASH] |

## Temporal Adaptations for Join Live and Turn-in Fast

The list below is a collection of temporal adaptation related attributes for use cases where the client needs to indicate to the server if a media request is for turning into a live event (or channel) [7] or joining fast into a stream [8]. They allow the server to respond accordingly for the purposes:

- low latency: e.g., by adaptively returning a sub-segment or a CMAF chunk on a live edge [9];

- on-demand: e.g., by adaptively returning a regular segment for on-demand content, when the attribute “Join Live” is either omitted or set to be FALSE;

- fast start-up: e.g., by adaptively returning one or more low-quality initial segments, possibly in conjunction with an initialization segment; and

- good-experience start-up: e.g., by adaptively returning one or more high-quality initial segments to ensure a good viewing experience from the very beginning, when the attribute “Join Fast” is either omitted or set to be FALSE.

| Name | Attribute | Pointer |
| --- | --- | --- |
| Join Live | 'jilv' | Indication of the media request for joining into a live event, due to initial join and seeking to the live edge of the event. |
| Turn-in Fast | 'tift' | Indication of the media request for turning into a stream as fast as possible. |

Note. These attributes are different from the start up key 	'stup' in CMCD [7], as the latter is described as follows:

“Key is included without a value if the object is needed urgently due to startup, seeking or recovery after a buffer-empty event. The media SHOULD not be rendering when this request is made. This key MUST not be sent if it is FALSE.”

## Proposal

A few issues to be considered and worked on:

- Namespace for these adaptation parameters, as a whole set or in the following categories:

  - Bitrate adaptation

  - Temporal adaptation

  - Spatial adaptation and

  - Content adaptation

- Additional adaptation parameters for

  - Bitrate adaptation (such as dynamic transcoding),

  - Temporal adaptation (such as trick plays),

  - Spatial adaptation (such as user-selected viewport/viewpoint dependent media processing), and

  - Content adaptation (such as pre-rendering and storyline selection in NBMP).

- Mechanisms for communication of these adaptation parameters:

  - URL query parameters

  - HTTP  header parameters

  - SAND messages (carrying adaptation parameters collected by the client and other DANE’s).

  - Json objects (e.g., CTA CMCD [7])

## References

- WG03_N0189_20251. “DASH TuC”. ISO/IEC JTC 1/SC 29/WG 3. April 2021.

- W18855. “Text of ISO/IEC 14496-12 6th edition”. October 2019. Geneva, CH.

- MDS19786_WG03_N00072. “Information technology — Coded representation of immersive media — Part 2: Omnidirectional MediA Format (OMAF) 2nd Edition”.

- WG03N0163. “Draft text of ISOIEC FDIS 23090-10 Carriage of Visual Volumetric Video-based Coding Data”. ISO/IEC JTC 1/SC 29/WG 3. January 2021.

- MDS19973_WG03_N00140. “Draft text of ISO/IEC 23009-1 5th edition”. January 2021.

- CTA Specification. “Web Application Video Ecosystem -Common Media Client Data”. CTA-5004.

- m56798. “Shortening tune-in time”. April 2021.

- m56673. “Minimizing initialization delay in live streaming”. April 2021.

- AWS Media Blog. “Lower latency with AWS Elemental MediaStore chunked object transfer”. https://aws.amazon.com/blogs/media/lower-latency-with-aws-elemental-mediastore-chunked-object-transfer/

## Discussion at MPEG#132 meeting

Note that the DASH client may not need any special signaling, and therefore there may no need for any standardization in DASH for this use-case.

## Discussion at MPEG#134 meeting

- CTA WAVE has started Common Server Metadata. We propose to work on these topics there and not duplicate the efforts in MPEG.

- We believe CMCD does not restrict its usage and you can use CMCD for server-side dynamic adaptation.

- These parameters seems to be extensions (new keys) to the CMCD spec.

# Extensions for Service Description (m56093)

## Introduction

This document provides proposed updates to Service Description. The background for this is the usage of Service Description as part of 5G Media Streaming.

## Background

3GPP has specified a system for 5G Media Streaming that enables a mobile network operator to offer a level of service that goes beyond "best effort" over-the-top IP-based media streaming. 5G Media Streaming services offered by a 5GMS System are provisioned by a third-party actor referred to as the 5GMS Application Provider for use by an application running on the User Equipment (UE) referred to as a 5GMS-Aware Application. The reference architecture and basic functional procedures are defined in TS 26.501 [X] and the detailed protocols are specified in TS 26.512 [Y]. The baseline video codecs and packaging standards that compliant UEs must support as a minimum are specified is TS 26.511 [Z].

In 3GPP Release 16, the scope of these specifications is restricted to unicast media streaming only. A Content Hosting capability is defined that resembles a Content Delivery Network (CDN). Later releases may add support for more complex media hosting and manipulation features. Ongoing Release 17 studies and normative work seek to add multicast/broadcast distribution mechanisms to the 5G System as well as edge computing capabilities.

The following high-level features are specified for 5G Media Streaming in Release 16. Each feature is optional and only available to a 5GMS-Aware Application if explicitly provisioned by a 5GMS Application Provider:

- Content Hosting. This may be deployed inside the 5G Core network in the form of an Operator CDN. Alternatively, an external third-party CDN may be integrated into the 5G Media Streaming system.

- Media Consumption Reporting. A random subset of 5GMS Clients can be configured to periodically report media session usage information to the 5GMS System.

- QoE Metrics Reporting. A random subset of 5GMS Clients can be configured to periodically report Quality of Experience metrics to the 5GMS System. These may be relayed to the 5GMS Application Provider.

- Dynamic Network QoS Policies. Specific network QoS policies are provisioned in advance, expressed as Policy Templates. During streaming sessions these Policy Templates can then be instantiated on demand by individual 5GMS Clients. The 5GMS Application Function negotiates with the Policy and Charging Function (PCF) in the 5G Core to apply the requested QoS policy to the relevant 5GMS packet flow.

- Network Assistance. Two forms of assistance are currently defined. Neither requires any special configuration at the provisioning stage.

  - The 5GMS Client can interrogate the network to find out what downlink network capacity is currently available to it. This can be used to influence the Media Player's choice of media representations to best ensure an uninterrupted streaming experience.

  - The 5GMS Client can request a temporary "boost" to its network Quality of Service, for example to speed up a background download (network resources permitting).

Figure X Reference architecture for 5G Media Downlink Streaming (see TS 26.501 [X])

The reference architecture for 5G Media Streaming as shown in Figure X defines the following functions to support the abovementioned features:

- A 5GMS Application Function deployed in the 5G Core or in an External Data Network that manages a 5GMS System. This logical function embodies the control plane aspects of the system, such as provisioning, configuration and reporting:

  - A 5GMS Application Provider provisions 5GMS functions using a RESTful HTTP-based provisioning interface at reference point M1.

  - Another RESTful HTTP-based configuration and reporting interface is exposed to 5GMS Clients at reference point M5.

- A 5GMS Application Server deployed in the 5G Core or in an External Data Network that provides 5G Media Streaming services to 5GMS Clients. This logical function embodies the data plane aspects of the system that deal with media content:

  - Content is ingested from 5GMS Application Providers at reference point M2. Both push- and pull-based ingest methods are supported, based on HTTP.

  - Content is distributed to 5GMS Clients at reference point M4 (after possible manipulation by the 5GMS Application Server function). Standard pull-based content retrieval protocols (e.g. DASH) are supported at this reference point.

- A 5GMS Client deployed in the UE that consumes 5G Media Streaming services. The 3GPP specifications are silent on whether this logical function is realised as shared UE middleware components or provided piecemeal by individual applications.

  - A Media Session Handler subcomponent first retrieves its configuration ("Service Access Information") from the 5GMS Application Function at reference point M5 and then uses this configuration information to activate and exploit the currently provisioned 5GMS features. The 5GMS-Aware Application controls the Media Session Handler via a UE-internal API defined at reference point M6. This reference point could, for example, be realised as a Javascript API in a web browser.

  - A Media Player subcomponent consumes media from the 5GMS Application Server at reference point M4. The 5GMS-Aware Application controls the Media Player via a UE-internal API defined at reference point M7. This reference point could also be realised as a Javascript API in a web browser, for example.

The basic procedures for 5G Media Streaming are shown in Figure Y.

Figure Y Basic procedures for 5G Media Downlink Streaming

According to TS 26.501 [X], Downlink Media Streaming provides the ability for content to be distributed using procedures and protocols defined by 5G Media Streaming as shown in Figure Z. The detailed procedures for the interfaces and APIs for 5G Media Streaming are defined in TS 26.512 [Y].

Figure Z <caption>

5G Media Streaming segment formats are defined based on the Common Media Application Format (CMAF) in ISO/IEC 23000-19 [A]. By using this format, 5G Media Streaming is compatible with a broad set of segment-based streaming protocols including Dynamic Streaming over HTTP (DASH) and HTTP Live Streaming (HLS). For example, ISO/IEC 23009-1 [B] defines a detailed DASH profile for delivering CMAF content within a DASH Media Presentation using a converged format for segmented media content.

5GMS media profiles for video, audio and subtitles based on the general constraints of ISO/IEC 23000-19 [A] are defined in TS 26.511 [Y]. However, 5G Downlink Media Streaming is not restricted to the media profiles defined in [Y]. Any CMAF media profile, for example for codecs defined in DVB specifications, may be used and distributed within 5G Downlink Media Streaming.

A more detailed client-centric approach is shown in Figure 1.

Figure 1 Client centric architecture

An important concept in 5G Media Streaming are Service Operation points and policy templates. The details are shown in Figure 2. An overview is provided in the following.

- Service Operation Points

  - Service Operation points define long lived profiles that will be used by streaming sessions as references.

  - Policy templates represent long term agreements made between the AP and the MNO.

  - Filtering: AP can limit what traffic and which users are allowed to use a specific policy template (FQDN-based)

  - Streaming session uses at most one of the allowed policy templates at any point in time

  - MSH may pre-cache or retrieve periodically or on request the list of allowed operation points for a specific AP

- Option 1: Define each Service Operation Point as a Slice

  - concept of Network Slice as a Service (NSaaS) is defined in TS 28.530 [10].

  - NSaaS can be offered by an MNO to third-party providers in the form of a service.

  - UE establishes or modifies the PDU session that will be used for the traffic based on URSP rules

  - Alternatively, network may also trigger the establishment or modification of the PDU session

- Option 2: Define each Service Operation Point as a QoS Flow

  - Flow description(s) of the transport session established in the step (see TS 23.502 [3]), e.g. the 5-tuple

Figure 2 Operation Points in 5G Media Streaming

In particular, clause 12 of TS 26.512 defines details on how to obtain status information. Table 13.2.6-1 provides a list of dynamically changing status information that can be obtained from the client.

Table 13.2.6-1: Dynamic Status information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| AverageThroughput | float | none | Current average throughput computed in the ABR logic in bit/s. |
| BufferLength | float | MediaType "video", "audio" and "subtitle" | Current length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type. |
| liveLatency | float | none | Current live stream latency in seconds based on the latency measurement. |
| MediaSetting[] | MPDAdaptationSet | MediaType "video", "audio" and "subtitle" | Current media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type. |
| MediaTime | float | None | Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered. |
| PlaybackRate | float | None | The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00. |
| availableServiceDescriptions[] | Provides the available service descriptions |  | Provides the list of available selectable service descriptions with an id to select from. Those are either configured ones or the ones in the MPD. |
| availableMediaOptions[] | List of Adaptation Set or Preselection ids | MediaType "video", "audio" "subtitle" "all" | Provides the list of available media options that can be selected by the application based on the capability discovery and the subset information. |
| Metrics[][] | Metrics |  | A data blob of metrics for each defined metrics collecting scheme. |

Table 13.2.6-2 provides a list of configured operation point information that can be obtained from the client. Any change to a parameter below shall be announced with a notification OPERATION_POINT_CHANGED.

Table 13.2.6-2: Operation Point Information

| OperationPoint |  |  | Operation Point Parameters | The currently configured operation point parameters according to which the DASH client is operating. |
| --- | --- | --- | --- | --- |
|  | mode |  | Enum | The following operation modes are defined: live: The DASH client operates to maintain configured target latencies using playback rate adjustments and possibly resync. vod: The DASH client operates without latency requirements and rebuffering may result in additional latencies |
|  | maxBufferTime |  | Integer | maximum buffer time in milliseconds for the service. |
|  | switchBufferTime |  | Integer | buffer time threshold below which the DASH clients attempts to switch Representations. |
|  | Latency |  |  | Defines the latency parameters used by the DASH client when operating in live mode. |
|  |  | target | Integer | The target latency for the service in milliseconds. |
|  |  | max | Integer | The maximum latency for the service in milliseconds. |
|  |  | min | Integer | The maximum latency for the service in milliseconds. |
|  | PlaybackRate |  | MediaType audio, video, all | Defines the playback rate parameters used by the DASH client for catchup mode and deceleration to avoid buffer underruns and maintaining target latencies. |
|  |  | max | Real | The maximum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  |  | min | Real | The minimum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  | Bandwidth |  |  | Defines the operating bandwidth parameters used by the DASH client used for a specific media type or aggregated. The values are on IP level. |
|  |  | target | Integer | The target bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | max | Integer | The maximum bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | min | Integer | The minimum bandwidth for the service in bit/s that the client is configured to consume. |
|  | PlayerSpecificParameters |  |  | Player specific parameters may be provided, for example about the used algorithm, etc. |

As seen, the parameters above align with today’s DASH Service Description to most extent.

## Key Issues for DASH

In order to make full use of the above functionality, the details on operation points can be carried in the application space. However, for better interoperability, a significant benefit is the addition on Operation Points associated to content in the MPD. The Service Description in DASH is exactly built for this.

We believe we should solicit input from Service Providers on static and dynamic service parameters that can be mapped to delivery optimizations.

We also believe that different operation points need to be mapped to content options, for example

- HD content (define the associated media and the required operation points)

- FullHD content (define the associated media and the required operation points)

## Proposal

It is proposed to extend service description to additional relevant information that can be used in order to define the delivery requirements and to also define content profiles. Some additional requirements should be collected.

MSE implementation of inband events (m56684)

Note: This section is included in TuC for study and is not intended for any publication by MPEG.

    - Sample inband event processing using MSE data model

This clause demonstrates an extended W3C Media Source Extension (MSE) model for inband event processing.

Note: The current MSE specification does not support the processing of inband events and this clause is an illustrative design on how MSE can be extended to support such functionality.

      - Process@append  rule

The process@append rule means that the inband events of a segment are processed, i.e. parsed, and dispatched or scheduled to dispatch at the time of appending the segment to the Media Source buffer.

In the case of an inband event with the on_receive dispatch mode:

- If the event end time is not smaller than the current playback position, and

- If this event or an equivalent has not been dispatched before,

Then the dispatcher dispatches the event immediately.

In the case of an inband event with the on_start dispatch mode:

- If the current playback position is  not smaller than the event start time, and

- If the current playback position is not equal or larger than the event end time, and

- If this event or an equivalent has not been dispatched before,

Then the event is stored in a dispatch buffer for dispatching at the event start time.

      - Dispatch buffer timing model

Figure A.13-7 demonstrates an inband event with the on_start dispatch mode relative to the MSE timing model.

Figure A.13-7- media source and inband event  dispatch buffers

      - Implementation

Figure A.13.8 demonstrates an example of the overlapping events with on-start dispatch mode.

S0

Event dispatch buffer

Wall clock

Media buffer

S1

S2

S3

E0

E1

E2

S0

S1

E0

E2

E1

Event

Media Segment

E1

E2

Event dispatch buffer

E0

E1

S0

S1

S2

Media buffer

Figure A.13.8—event buffer model example for on_start events

As is shown above, emsgs E0, E1, and E2 are mapped to the dispatch buffer. With the initial appending of the S1 media segment to the media buffer, the ranges between the event’s start and the event’s end are marked in the dispatch buffer for E0 and E1.

When S2 is appended to the media buffer, in this case since E2 overlaps with E1 in the dispatch buffer, the corresponding range in the dispatched first is divided into 3 subranges, as shown in the figure.

Figure A.13.9 demonstrates an example of an overwrite, in which the segment S2 is overwritten by a new segment S2’ (that does not contain any emsgs) and has a duration that only covers a portion of S2 in the media buffer.

S0

Event dispatch buffer

Wall clock

Media buffer

S1

S2

S3

E0

E1

E2

S0

S1

S2

Event

Media Segment

S2’

E0

E2

E1

E1

E2

Figure 13. 9—Overwrite of a part of a segment with events having on_start dispatch mode

As shown, since the event E2 has the on_start dispatch mode, its range in the dispatch buffer is unchanged.

      - Algorithms

        - Initialization

- Applications inputs to DASH client

  - Subscribe SchemeIdURI/value

  - Provide dispatch mode

- Event buffer initialization:

  - Event dispatch (range of event purge may go beyond media buffer)

- Set Presentation Time Offset (PTO)

A.1.13.2.2	Append

- Parse media samples

- Append media samples to media buffer, calculate its start in media source buffer (segment_start)

- For each emsg

  - Parse emsg

  - Calculate event start and end times, and compare it to the current playback position.

  - If

    - event’s end time < current playback position or

    - its emsg.id exists in the “already-dispatched” table

then stop processing this emsg and go to Step 3 to start processing the next emsg.

  - If dispatch_mode = on_receive

    - Dispatch the event

    - Add its emsg.id to the “already-dispatched” table

    - Go to Step 3 to start processing the next emsg.

  - Otherwise (dispatch_mode = on_start):

    - Calculate the event dispatch range:

      - For emsg v0 (esmg.version =0): dispatch_range_start =  segment_start+ emsg.presentation_time_delta/emsg.time_scale

      - For emsg v1 (emsg.version =1): dispatch_range_start = emsg.presentation_time/emsg.timescale

      - dispatch_range_end = dispatch_range_start + emsg.duration/emsg.timescale

    - If there is already existing event in the dispatch range:

      - Divide the range to subranges, such that each subrange is either

        - empty (no event), or

        - is occupied with one or more events

      - For each subrange:

        - Add a new event constaining:

- Emsg.scheme_id_uri

- emsg.value

- emsg.id

- dispatch_range_start

- dispatch_duration = emsg.duration/emsg.timescale

- emsg.message_data()

A.1.13.2.4	Dispatch

- Find the events occurring in the dispatch buffer at the playback position.

- For each event

  - If  its emsg.id is not in the “already-dispatched” table,

    - Dispatch the event

    - Remove the event from the dispatch buffer

    - Add its emsg.id to the “already-dispatched” table

  - Otherwise, remove the event from the dispatch buffer

A.1.13.2.5  Purge

In a purge operation, either a range from the start or a range from the end of the media buffer is purged. This range is referred to as the “purged-range” in this subclause.

- If any event in the dispatch buffer overlaps with the purged-range

  - Split the event into two events around the purge-range boundary.

- Remove the purged-range from the dispatch buffer

# Determination and Handling of Duplicate MPD Events(m56503)

## Introduction

In case of SCTE 35 events the expected event handling sometimes differs from what was envisioned in case of DASH events. This contribution goes over some cases where the same event needs to appear multiple times and its subsequent appearances may be essential for correct functioning of the application.

The key difference is that an SCTE 35 event may describe some time period, and this description can sometimes be modified (e.g. terminated or extended). This contribution revisits two use cases where it is unclear how to handle SCTE 35 messages or where the typical implementations are inefficient.

## Use cases

### Events with unknown duration

An SCTE 35 message can signal a start of a time interval and keep it “open” until a different SCTE 35 message closes it. There is a need for an application to be aware that it is within this interval, as this lets the programmer / distributor use at last a part of an ad break if the viewer tunes into the channel during a break.

Current implementations using an MPD event keep the Period element containing the first SCTE 35 message even if the period has no segments left, and remove empty periods after the time interval closes. The result is much larger MPDs than needed.

The 5th edition event model lets us handle the use case, albeit in a somewhat awkward way: we need to use random id values and on-receive method. The application will have a special SCTE 35 processing mechanism which will parse the SCTE 35 message and maintain state given the id specified in the message itself (which is identical for the first and the second message).

The above implementation is workable, but it is very specific to SCTE 35. It is possible to have other events (e.g. events intended for UI for a football game), moreover the HLS EXT-X-DATERANGE tag allows such functionality for any event.

Secondly, the timing information in this case may sometimes be lost. If we are repeating an event (e.g. copying an event to the current Period element in order to remove empty periods), the presentation time of a copy event can not be earlier than the EPT of the segment carrying it (i.e., LAT ≤ ST, in the 5th ed. terminology). The timing information in the SCTE 35 message itself is expressed as a PTS offset, and the PTSs of the media representations are not necessarily preserved when ISO-BMFF segments are generated. This way we cannot let the application know when did the event start.

### Joining just before or during an ad break

For the duration of the ad break the linear channel has some “default” content. This way, in case the client starts playback too late and inband events are used, it will miss the original event and show default content. The desirable functionality is to always attempt using an ad break or at least a part of it.

This way, a periodic (e.g. each segment boundary) repetition of the event is needed. With that said, the start time of the event is in the past, which cannot be accommodated by either the current event syntax or the model in Annex A.

### Cancelation and update

In some cases, a follow-up SCTE 35 message is sent to update a previous opportunity or to cancel it altogether.  This again can only be implemented at the application level and done specifically for SCTE 35 – a second event with identical id will be ignored by the system

## Notes from the MPEG#134

- Why do we need to signal it in the event envelope and not address it in the messageData?

- Do these parameters create states for events?

- Definition of cancelation, updating, ..?

- Clear processing model?

- Backward compatibility?

Events are getting overloaded with features. There might be a different solution. We should investigate the use-case.

One usecase is that for MPD event instance that started prior to the period containing it. One solution is the presentationtimeoffset.

Fast Fine Tune in

## Introduction

At MPEG#134, 3 different proposal was submitted on fast tuning. At MPEG#135, one of the contributions was followed up with an update. This section of TuC summarizes these contributions. The goal is to compare these solutions with each other using a set of common metrics that need to be developed first, and eventually develop a fast tune in feature for DASH.

## Minimizing initialization delay in live streaming (initially in m56673, refined in m57429)

### Abstract

This contribution is a follow-up of m56673 (of the 134th MPEG meeting in April 2021), which was accepted into the TuC for DASH in MDS20251_WG03_N00189.

The following aspects are proposed for minimizing initialization delay in live streaming scenarios wherein the Segment duration is not constant:

- To have the "now" Media Segment (MS) available at a Representation-level constant URL, such that clients can tuning-in into a live streaming session with an earlier pre-fetched MPD.

- The definition of Tuning-In Media Segment (TIMS), which is either the concatenation of an Initialization Segment (IS) and the "now" MS, or just the "now" MS. If the IS is not included in the MPD, it can then be included in the TIMS to save a round-trip time in the initialization delay.

- The definition of Live Streaming Tuning-In Event, for signalling of the Segment number of the Tuning-In Media Segment using the 'emsg' box, to enable clients figuring out the URLs of the subsequent MSs after tuning-in (assuming the use of the Segment-number-based URL template).

The proposed specification text changes are included in the attachment to the contribution m57429, with change marks relative to the 5th edition FDIS text in m57094.

### Introduction

In live streaming services, the initialization delay refers to the delay between the time the user clicks a button to join the live session and the time when the first picture is displayed to the user. This delay is one of the most important factors that affect the live streaming use experience. There for continuously improving live streaming services deployed worldwide, live streaming service provides, including Bytedance, have spent quite some efforts trying to optimize the initialization delay.

For a user to tune into a live session, the client needs to obtain 1) the MPD, 2) the Initialization Segment (IS), and 3) the "now" Media Segment (MS), and then continues from there.

Ideally, the simplest way, which is also optimal for initialization delay, is 1) to have constant Segment durations, 2) use the Segment-number-based URL template, and 3) have the MPD (possibly together with the IS) pre-fetched before tunning. This way, if the IS is pre-fetched together with the MPD, only one HTTP request for the "now" MS is needed for tuning into the live session; and if the IS is not pre-fetched together with the MPD, two HTTP requests, one for the IS and one for the "now" MS are needed.

However, in live streaming, particularly when the live "broadcasters" are users using all kinds of mobile devices (e.g., in TikTok and Douyin), it is often difficult to ensure constant Segment durations. The video camera of the device may capture video at different varying frame rates. The video encoder may skip a frame from time to time due to computing resource issues. The video encoder may also apply encoding optimizations based on scene cuts, which results in different RAP periods. As a result, it is not always possible to use the simple and nice approach based on the @duration attribute that specifies the constant approximate Segment duration. Consequently, many live streaming services are forced to use the SegmentTimeline element. BTW, if constant Segment duration can always be ensured, we would not have needed the SegmentTimeline element at the first place.

However, using SegmentTimeline often requires a client to request the latest MPD to figure out the URL of the "now" MS for tuning into a live streaming session. Basically, the client firstly requests the latest MPD to obtain the URL information of the "now" MS, then, if the IS is not embedded in the MPD, it requests the IS, and then requests the "now" MS, and continues from there.

This need of multiple roundtrips and multiple requests causes additional initialization delay.

In this contribution, we propose a set of changes, as summarized in the abstract, to minimize the initialization delay in live streaming scenarios with non-constant Segment durations.

The proposed changes enable, for live streaming scenarios with non-constant Segment durations, to have the MPD prefetched and to use only one HTTP request to tune into a live streaming session. Simulations have been performed to compare to the existing capability of embedding the IS with the MPD. For cold streams, the average reductions of the initialization delay are 38.4% and 17.9%, respectively, for the cases without and with keep-alive, and for hot streams, the numbers are 44.0% and 12.2%, respectively.

### Proposal

The proposed specification text changes are included in the attachment to the contribution m57429, with change marks relative the 5th edition FDIS text in m57094.

There are mainly three parts in the proposal that can be separately considered:

- Part 1: Using a constant URL for obtaining the "now" MS.

- Part 2: Combining the IS with the "now" MS.

- Part 3: The indication of the Segment number of the "now" MS.

Noe that the target application scenario is wherein Segment durations in a Representation cannot be kept constant enough to ensure correct derivation of the Segment URLs based on the Segment@duration attribute and thus the SegmentTimeline element is in use.

The key point of Part 1 is that it enables live streaming tuning in with pre-fetched MPD for the target application scenario. To our knowledge, this capability not possible with any existing DASH design, including the existing capability of embedding the IS with the MPD.

For Part 2, in this contribution we made the combination of the IS with the "now" MS optional, such that if the IS is embedded in the MPD then it is not combined with the "now" MS.

Part 3 is a further step on top of Part 1, to enable figuring out the URLs of the subsequent MSs after tunning for continuing the live streaming session, assuming that the use of the Segment-number-based URL template.

### Simulations

To address a comment received from the discussion of the previous proposal in m56673, on whether the proposal improves on the existing capability of embedding the IS with the MPD, we performed a set of simulations with 100 runs for each simulation, for both some cold streams and some hot streams. The average initialization delay values are listed below:

| Cold streams |  |  | Hot streams |  |  |
| --- | --- | --- | --- | --- | --- |
| Anchor w/o keep-alive | Anchor with keep-alive | Proposal | Anchor w/o keep-alive | Anchor with keep-alive | Proposal |
| 868 ms | 652 ms | 535 ms | 630 ms | 402 ms | 353 ms |

As can be seen, for cold streams, the average reductions of the initialization delay are 38.4% and 17.9%, respectively, for the cases without and with keep-alive, and for hot streams, the numbers are 44.0% and 12.2%, respectively.

The reduction mainly comes from the saving of the request for the MPD, as in the proposal approach the MPD can be pre-fetched, while in the anchor approaches the MPD has to be fetched immediately before requesting the "now" MS to figure out the URL of the "now" MS. Note again that the target application scenario is wherein the Segment durations in a Representation cannot be kept constant enough to ensure correct derivation of the Segment URLs based on the Segment@duration attribute and thus the SegmentTimeline element is in use.

### Questions and answers

Some discussions of m56673 can be found herein: http://mpegx.int-evry.fr/software/MPEG/Systems/DASH/spec/-/issues/155. Below are our answers/responses to the questions/comments in addition to those included in the above link and expressed orally at the 134th MPEG meeting.

- Q: There is already a possibility of inlining the initialization Segment into the MPD using data URIs. This saves 2-3 extra HTTP GET requests (audio, video, possibly subtitles). Does the proposal improve on this result?

A: See Section 3.

- Q: If I understand correctly your proposal, you need to have a concatenated IS+seg for you tune-in Segment, which will either require:

- duplication of all "tune-in" Segments at the origin server, (potentially all of them)

- logic at the edge server to convert the request for such Segments into the concatenated version.

Is this correct?

A: It works as follows, described separately for the origin server and edge servers.

At the original server, each time when a new MS is generated for the live streaming service, the new MS is duplicated, one as the URL using the Segment-number-based URL template, the other as the constant URL. When the next MS is generated, the constant URL now represents this MS. Therefore, for one particular Representation, at any moment there is only one duplication of the latest available MS.

The expiration time for the Tuning-In MS is set to be very short, e.g., less than or equal to it duration, avoiding clients fetching and using of a cached staled version from an edge server. Although it is possible for an implementation of an edge server to generate the Tuning-In MS, in our implementations we did not do that, i.e., if the edge server does not have a un-expired version of a Tuning-In MS available, it just lets the request go to the next upper-level server.

### Suggestions from the 135th MPEG meeting

Discussions of m57429 at the 135th MPEG meeting can be found herein: http://mpegx.int-evry.fr/software/MPEG/Systems/DASH/spec/-/issues/189.

Key suggestions/issues pointed out:

- The shorter the time a TIMS is cached, the higher the chance of cache miss. If the edge doesn't do the generation but let the request flow upward in the CDN, request overloading up to the origin might be a potential problem, particularly at the beginning for big live streaming events.

- CDNs might not follow the TTL (i.e., the expiration time) set for the Tuning-In MS.

- Consider using a) multipart MIME messages, b) in deployments only use tune in for one Representation, and c) no emsg as either an alternative approach or to be used in combination with (some of) the proposed pieces for optimization of the initialization delay.

## Shortening tune-in time (m56798)

### Introduction

Any encoding scheme using inter-frame prediction introduces a tune-in delay. In particular, when GOP coding structure is used, joining a live session is only possible at a GOP boundary. This creates a trade-off: shorter GOPs result in loss of compression efficiency; longer GOPs result in longer tune-in delays. Moreover, these delays become hard to predict if variable GOP durations are used – typically GOP boundaries aligning with scene cuts produce better results than fixed-duration GOP structures.

One approach to get faster tune-in and hence finer grain random access is the Resync element. Resync will let the player estimate locations of switch points inside a segment, i.e. subsegments (CMAF chunks) which start with a key frame. In the best case the player is able to guess the location well and issue a byte-range request to get close to the switch point's previous key frame and start decoding from there.

The limitations of the Resync approach are (a) expectation of a regular cadence of key frames (while variable segment duration is known to be beneficial), (b) lack of byte range support, and (c) reliance on relatively frequent key frames which negatively impacts compression efficiency, especially when sub-500ms frequency is needed.

Fast join and frame-accurate transitions can be achieved by using different adaptation sets for joining or transitioning and for normal playback. This has been discussed as a part of the DASH-IF low latency work. The “join” adaptation set(s) will have shorter segments than the “main” adaptation sets and will be arranged such that each “main” adaptation set boundary corresponds to “join” adaptation set boundary. The client starts from downloading segments from the “join” adaptation set until it reaches the boundary of the “main” adaptation set. This can be formalized as a “ladder” of special-purpose join adaptation sets, as shown in Figure 1 below.

Figure 1: Ladder-based fast join

In the example below there are two “join” adaptation sets, J0 and J1 aligned such that for any segment in J1 with EPT=X, there will be a segment with the same value of EPT in J0. The segment duration in J0 is very short (e.g. a single I frame, or a 4-frame IBBB structure), and J1 has longer segments (e.g. 8 or 16 frames). Lastly, there is the “main” adaptation set M, which is aligned with the “join” adaptation sets such that for every segment in M with EPT=Y, there will be segments with EPT=Y in both J0 and J1

At the start of playback, the client will start with a number of segments from J0 and switch to J1 whenever the boundary of a J1 segment in is encountered. Similarly, the client will switch to a segment from adaptation set M whenever the segment start is encountered.

“Dirty switches” (described in contribution m56085 sec 3.6) may be beneficial from the bandwidth perspective, as J representations will inevitably be less efficient from the standpoint of compression efficiency.

### Proposal

All the necessary tools already exist in DASH. The only missing part is signalling the ladder-style segment alignment across adaptation sets.

This can be signalled as an EssentialProperty with a @schemeIdUri of urn:mpeg:dash:adaptation-set-ladder:2021 and a @value specifying the list of adaptation sets with a ladder time alignment. E.g., the @value for adaptation set M will list J0 and J1. The value for J1 will only list J0.

Additionally, the ladder switching requires either presence of SegmentTimeline or precise SegmentTemplate@duration (i.e. each segment except for the last contains fixed number of frames). This is needed for “climbing up the ladder” since there are multiple segments in the “join” representation for the timespan of a single “main” segment, and the number of “join” segments per each “main” segment is not constant.

The above doesn’t work well in conjunction with low latency, as it relies on MPD signalling. In order to accommodate the use case we propose introducing inband event with schemeIdUri of urn:mpeg:dash:adaptation-set-ladder:2021

The event payload will consist of an adaptation set switch table listing one or more switching point for a given presentation time. In some cases (such as chunked CMAF), a CMAF fragment may comprise multiple subsegments, hence in addition to switch time we may need byte offset to address the particular subsegment.

The adaptation set switch table would be structured as follows:

aligned(8) struct AdaptationSetSwitchingTable {

unsigned int(8) num_adaptation_sets

struct {

string  adaptation_set_id;

int(3)  SAP_type;

int(1)  subsegment_flag

int(4)  reserved = 0

if (subsegment_flag) {

int(64) subsegment_offset;

}

} [ num_adaptation_sets];

}

Semantics:

adaptation_set_id: AdaptationSet@id of an adaptation set which has a SAP of a given type at presentation time of the event.

SAP_type: SAP type, per ISO/IEC 14496-12. For any SAP type other than 1 or 2, the switch is a “dirty switch” (as defined in contribution m56085 sec 3.6). Error propagation and artefacts may occur after such a switch, Encoder-level measures such as periodic intra refresh, joins at mini-GOP boundaries, and reducing age of references may be used to reduce error propagation.

subsegment_offset: offset from the start of the segment containing the presentation time to the first byte following the last byte of the `mdat` box of the previous segment (i.e., combined size of all preceding subsegments)

In order to accommodate “dirty” switches, we may need additional signaling to indicate maximum reference age (similar to the definition of SRAP in SCTE 128) in order for the client to make sure that correct pictures will be in the decoded picture buffer by the time the switch is made. This can be signalled as an EssentialProperty with a @schemeIdUri of urn:mpeg:dash:max-reference-age:2021 and a @value specifying the minimum amount of samples (in unit of timescale) needed to guarantee that the decoded picture buffer contains all reference frames needed for decoding any sample within any segment will already be in the decoded picture buffer.

## Potentially finer-grain random access in DASH by using streams with gradual refresh and recovery_point() SEIs (m56891)

### Summary

This contribution brings information about a somewhat less known feature of MPEG video standards, known as recovery_point SEIs, and explains how they can be potentially used in MPEG-DASH for the purpose of providing random access capability without inserting additional IDR frames.

### Gradual refresh and Recovery Point SEIs in MPEG and ITU-T video standards

It is well known that gradual intra-refresh was a feature of pretty much all video codecs since ITU-T H.263 and MPEG-1 standards. For example, most remember the requirement to refresh each macroblock as intra coded one once in at most 256 frames in those early standards.  In practice, such refresh was often enforced to occur even more often.

The initial need in gradual refresh functionality was driven by lossy media or networks used for video storage and transmission, and where by introducing more intra-coded macroblocks the streams were made more “resilient” to such errors. Additional reasoning for intra refresh was coming from non-exact definition of IDCT and desire to minimize potential drift that may be caused by different decoder implementations.

But moving forward to video standards today (MPEG-4 AVC and HEVC standard), we discover that gradual refresh is still supported, and in fact, often used in practice. New common applications of this functionality are streams coded for satellite TV transmission, low-latency contribution feeds, sports events production workflows, etc.

Furthermore, we find that modern standards offer specific SEIs, so-called Recovery Point SEIs, that allow decoders to join streams at certain points where there is no IDR frame inserted, but where is known that the following sequence of frames (P- or B-coded) has a sufficient density and placement of gradually updated picture elements (macroblocks or CUs) that would enable successful decoding and joining the stream. Furthermore, the semantics of recovery point SEIs provides exact instructions for decoder implementations on how to treat missing references and other stream elements such that decoding will be successful. In other words, the behavior of decoders in joining streams at Recovery Point SEIs becomes fully defined by standards. Additional details about syntax and semantics of recovery point SEIs in modern standards is provided in Appendix A at the end of this contribution.

As mentioned earlier, we have observed that there are many existing deployments of video systems using this functionality, and that most commercial video equipment nowadays is supporting it. This includes modular receiver-decoders, contribution encoders, distribution encoders/transcoders, etc.

An example of a production stream using this functionality and the resulting decoder behavior is illustrated in a sequence of figures below. All figures have been produced as screenshots of elementary stream analyzer (Elecard StreamEye) used to decode and visualize the sequence:

Fig 1. First frame after Recovery Point SEI. It is seen that only a vertical bar of about 5 macroblocks in width are Intra-coded, while rest of picture represents coded residual.

Fig 2. Second frame after Recovery Point SEI. It is seen that now about 10 macroblocks at the left are cleanly decoded.

Fig 3. Frame 14 after Recovery Point SEI. It is seen that now about half of frame is cleanly decoded.

Fig 4. Frame 28 after Recovery Point SEI. It is seen that now the entire frame is fully reconstructed and clean decoding follows from this moment on.

As can be easily grasped from the above example, the Recovery Point SEIs do provide an alternative random-access functionality in video streams. This method is well supported by existing commercial encoders and decoders in broadcast industry.

### Potential application of this functionality in MPEG-DASH

We believe that streams with gradual refresh and Recovery Point SEIs could potentially be used in MPEG DASH. The random-access points corresponding to Recovery Point SEIs can possibly be defined as SAP type 4 random access points.

But of course, a mixed use of IDR-frame based (SAP type 1) and gradual refresh (SAP type 4) can also be envisioned and discussed.

Appendix A. Syntax and Semantics of the recovery point SEI

Below is the definition of syntax and semantic of recovery_point() SEI in MPEG-4 AVC/ITU-T Rec. H.264 standard [2]. The MPEG-H HEVC / ITU-T Rec. H.265 standard [3] provides essentially the same definitions.

D.2.8 Recovery point SEI message semantics

The recovery point SEI message assists a decoder in determining when the decoding process will produce acceptable pictures for display after the decoder initiates random access or after the encoder indicates a broken link in the coded video sequence. When the decoding process is started with the access unit in decoding order associated with the recovery point SEI message, all decoded pictures at or subsequent to the recovery point in output order specified in this SEI message are indicated to be correct or approximately correct in content. Decoded pictures produced by random access at or before the picture associated with the recovery point SEI message need not be correct in content until the indicated recovery point, and the operation of the decoding process starting at the picture associated with the recovery point SEI message may contain references to pictures not available in the decoded picture buffer.

In addition, by use of the broken_link_flag, the recovery point SEI message can indicate to the decoder the location of some pictures in the bitstream that can result in serious visual artefacts when displayed, even when the decoding process was begun at the location of a previous IDR access unit in decoding order.

NOTE 1 – The broken_link_flag can be used by encoders to indicate the location of a point after which the decoding process for the decoding of some pictures may cause references to pictures that, though available for use in the decoding process, are not the pictures that were used for reference when the bitstream was originally encoded (e.g., due to a splicing operation performed during the generation of the bitstream).

The recovery point is specified as a count in units of frame_num increments subsequent to the frame_num of the current access unit at the position of the SEI message.

NOTE 2 – When HRD information is present in the bitstream, a buffering period SEI message should be associated with the access unit associated with the recovery point SEI message in order to establish initialization of the HRD buffer model after a random access.

Any picture parameter set RBSP that is referred to by a picture associated with a recovery point SEI message or by any picture following such a picture in decoding order shall be available to the decoding process prior to its activation, regardless of whether or not the decoding process is started at the beginning of the bitstream or with the access unit, in decoding order, that is associated with the recovery point SEI message.

Any sequence parameter set RBSP that is referred to by a picture associated with a recovery point SEI message or by any picture following such a picture in decoding order shall be available to the decoding process prior to its activation, regardless of whether or not the decoding process is started at the beginning of the bitstream or with the access unit, in decoding order, that is associated with the recovery point SEI message.

recovery_frame_cnt specifies the recovery point of output pictures in output order. All decoded pictures in output order are indicated to be correct or approximately correct in content starting at the output order position of the reference picture having the frame_num equal to the frame_num of the VCL NAL units for the current access unit incremented by recovery_frame_cnt in modulo MaxFrameNum arithmetic. recovery_frame_cnt shall be in the range of 0 to MaxFrameNum − 1, inclusive.

exact_match_flag indicates whether decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the recovery point SEI message shall be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous IDR access unit in the NAL unit stream. The value 0 indicates that the match need not be exact and the value 1 indicates that the match shall be exact.

When decoding starts from the location of the recovery point SEI message, all references to not available reference pictures shall be inferred as references to pictures containing only macroblocks coded using Intra macroblock prediction modes and having sample values given by Y samples equal to ( 1 << ( BitDepthY − 1 ) ), Cb samples equal to ( 1 << ( BitDepthC − 1 ) ), and Cr samples equal to ( 1 << ( BitDepthC − 1 ) ) (mid-level grey) for purposes of determining the conformance of the value of exact_match_flag.

NOTE 3 – When performing random access, decoders should infer all references to not available reference pictures as references to pictures containing only intra macroblocks and having sample values given by Y equal to ( 1 << ( BitDepthY − 1 ) ), Cb equal to ( 1 << ( BitDepthC − 1 ) ), and Cr equal to ( 1 << ( BitDepthC − 1 ) ) (mid-level grey), regardless of the value of exact_match_flag.

When exact_match_flag is equal to 0, the quality of the approximation at the recovery point is chosen by the encoding process and is not specified by this Recommendation | International Standard.

NOTE 4 – Under some circumstances, the decoding process of pictures depends on the difference DiffPicOrderCnt( picA, picB ) between the PicOrderCnt( ) values for two pictures picA and picB. However, no particular values of TopFieldOrderCnt and BottomFieldOrderCnt (as applicable) are specified to be assigned to the reference pictures that are not available due to the initiation of random access at the location of a picture associated with a recovery point SEI message. Also, no particular value has been specified for initialization (for random access purposes) of the related variables prevPicOrderCntMsb, prevPicOrderCntLsb, prevFraneNumOffset, and prevFrameNum. Thus, any values for these variables may be assigned that could hypothetically have resulted from operation of the decoding process starting with a hypothetical preceding IDR picture in decoding order, although such values may not be the same as the values that would have been obtained if the decoding process had started with the actual preceding IDR picture in the bitstream. When performing random access at a picture associated with a recovery point SEI message, it is suggested that decoders should derive the picture order count variables TopFieldOrderCnt and BottomFieldOrderCnt according to the following method:

– A bit range greater than 32 bits should be allocated for the variables TopFieldOrderCnt and BottomFieldOrderCnt for each current picture to be decoded, as well as for the intermediate variables used for deriving these variables as specified in clause 8.2.1. (Due to the lack of assurance of correspondence of the values used for initialization of the related variables when random access is performed to the values that would be obtained if the decoding process had begun with the preceding IDR picture in decoding order, the calculations involving these variables in the decoding process of subsequent pictures may result in violation of the 32 bit range.)

– Any value within in the range of −231 to 231 − 1, inclusive, may be assigned to the values of the variables TopFieldOrderCnt and BottomFieldOrderCnt of the reference pictures that are not available due to the random access operation. For example, the value 0 may be assigned to these variables.

– For the derivation of the picture order count variables for the picture at which random access is performed, prevPicOrderCntMsb may be set equal to any integer multiple of MaxPicOrderCntLsb in the range of −231 to 231 − 1, inclusive, prevPicOrderCntLsb may be set equal to any value in the range of 0 to MaxPicOrderCntLsb − 1, inclusive, prevFrameNumOffset may be set equal to any integer multiple of MaxFrameNum in the range of 0 to 231 − 1, inclusive, and prevFrameNum may be set equal to any value in the range of 0 to MaxFrameNum − 1, inclusive. For example, the value 0 may be assigned to all of the variables prevPicOrderCntMsb, prevPicOrderCntLsb, prevFrameNumOffset, and prevFrameNum.

When exact_match_flag is equal to 1, it is a requirement of bitstream conformance that the values of the samples in the decoded pictures at or subsequent to the recovery point in output order shall be independent of the values that a decoder assigns to the variables prevPicOrderCntMsb, prevPicOrderCntLsb, prevFrameNumOffset, and prevFrameNum used in clause 8.2.1 for deriving the picture order count variables for the initialization of the decoding process at the picture associated with the recovery point SEI message, and of the values that are assigned to the TopFieldOrderCnt and BottomFieldOrderCnt variables of the reference pictures that are not available due to the random access operation.

broken_link_flag indicates the presence or absence of a broken link in the NAL unit stream at the location of the recovery point SEI message and is assigned further semantics as follows:

– If broken_link_flag is equal to 1, pictures produced by starting the decoding process at the location of a previous IDR access unit may contain undesirable visual artefacts to the extent that decoded pictures at and subsequent to the access unit associated with the recovery point SEI message in decoding order should not be displayed until the specified recovery point in output order.

– Otherwise (broken_link_flag is equal to 0), no indication is given regarding any potential presence of visual artefacts.

Regardless of the value of the broken_link_flag, pictures subsequent to the specified recovery point in output order are specified to be correct or approximately correct in content.

NOTE 5 – When a sub-sequence information SEI message is present in conjunction with a recovery point SEI message in which broken_link_flag is equal to 1 and when sub_seq_layer_num is equal to 0, sub_seq_id should be different from the latest sub_seq_id for sub_seq_layer_num equal to 0 that was decoded prior to the location of the recovery point SEI message. When broken_link_flag is equal to 0, the sub_seq_id in sub-sequence layer 0 should remain unchanged.

changing_slice_group_idc equal to 0 indicates that decoded pictures are correct or approximately correct in content at and subsequent to the recovery point in output order when all macroblocks of the primary coded pictures are decoded within the changing slice group period, i.e., the period between the access unit associated with the recovery point SEI message (inclusive) and the specified recovery point (inclusive) in decoding order. changing_slice_group_idc shall be equal to 0 when num_slice_groups_minus1 is equal to 0 in any primary coded picture within the changing slice group period.

When changing_slice_group_idc is equal to 1 or 2, num_slice_groups_minus1 shall be equal to 1 and the macroblock-to-slice-group map type 3, 4, or 5 shall be applied in each primary coded picture in the changing slice group period.

changing_slice_group_idc equal to 1 indicates that within the changing slice group period no sample values outside the decoded macroblocks covered by slice group 0 are used for inter prediction of any macroblock within slice group 0. In addition, changing_slice_group_idc equal to 1 indicates that when all macroblocks in slice group 0 within the changing slice group period are decoded, decoded pictures will be correct or approximately correct in content at and subsequent to the specified recovery point in output order regardless of whether any macroblock in slice group 1 within the changing slice group period is decoded.

changing_slice_group_idc equal to 2 indicates that within the changing slice group period no sample values outside the decoded macroblocks covered by slice group 1 are used for inter prediction of any macroblock within slice group 1. In addition, changing_slice_group_idc equal to 2 indicates that when all macroblocks in slice group 1 within the changing slice group period are decoded, decoded pictures will be correct or approximately correct in content at and subsequent to the specified recovery point in output order regardless of whether any macroblock in slice group 0 within the changing slice group period is decoded.

changing_slice_group_idc shall be in the range of 0 to 2, inclusive.

### References

- ITU-T Rec. H.263, Feb 1998.

- ISO/IEC 13818-2 | ITU-T Rec. H.262, MPEG-2 Video, Jul. 1995.

- ISO/IEC 14496-10 | ITU-T Rec. H.264, Advanced Video Coding, Dec. 2003.

- ISO/IEC 23090-3 | ITU-T Rec. H.266, Versatile Video Coding, DIS, Jul 2020.

External Stream Representation (ESR) for support of EDRAP based video streaming (initially in m56674, refined in m57430)

## Abstract

This contribution is a follow-up of m56674 (of the 134th MPEG meeting in April 2021), which was accepted into the TuC for DASH in MDS20251_WG03_N00189.

A design of an External Stream Representation (ESR) is proposed, which is asserted to be the missing DASH piece that is needed for support of extended dependent random access point (EDRAP) based video streaming, as the pieces at both the video codec level and at the file format level have been in place.

An ESR is time-synchronized with an associated Main Stream Representation (MSR), the "normal" Representation. An ESR contains only the random access point (RAP) pictures additionally needed when random accessing from an time-synced EDRAP picture/sample in the MSR.

The attached slides illustrate the concept.

Compared to m56674, the following updates are included in this contribution:

- A summary of the proposal has been added.

- A definition of the EDRAP picture, based on ISOBMFF sample groups, has been added.

- To make sure that legacy DASH clients won't select and consume an ESR, an EssentialProperty descriptor is defined to be included in Adaptation Sets containing ESRs.

- A section for questions and answers has been added.

## Introduction

At the 133rd MPEG meeting, the extended dependent random access point (EDRAP) sample group was agreed based on the proposal in m56020. Due to that there was no ISOBMFF amendment being worked on, it was included into the ISOBMFF TuC (in WG3N107_N19940). At the same time, at the 21st JVET meeting, the EDRAP indication SEI message based on the proposal in JVET-U0084 was adopted into the VSEI specification. The above two items pave the basis for support of the so-called external decoding refresh (EDR) based video streaming, also referred to as EDRAP based video streaming.

See the slides attached to the contributions m56674 and m57430 for an introduction to the concept.

## Proposal summary

The proposal is summarized as follows:

- Five definitions, for the terms EDRAP picture, external elementary stream, external picture, External Stream Representation (ESR), and Main Stream Representation (MSR), are proposed.

- An optional Adaptation Set level attribute, named @esasFlag, is proposed to indicate whether the Representations in the Adaptation Set are ESRs or MSRs.

- As part of the semantics of the @esasFlag attribute, the following are proposed:

  - The association of an ESR to an MSR through existing Representation attributes @associationId and @associationType, based a newly specified association type value 'aest' ("associated external stream track"; the same 4CC as the ISOBMFF track reference type).

  - A new EssentialProperty descriptor is proposed to be included in Adaptation Sets containing ESRs, to indicate that a Representation in such an Adaptation Set cannot be consumed or played back by itself without other video Representations.

  - Some constraints for simplifying the EDRAP based streaming operations:

    - Each EDRAP picture in an MSR shall be the first picture in a Segment.

    - For an MSR and an ESR associated with each other, the following constraints apply:

      - For each Segment in the MSR that starts with an EDRAP picture, there shall be a Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR, wherein the Segment in the ESR carries the external pictures needed for decoding of that EDRAP picture and the subsequent pictures in decoding order in the bitstream carried in the MSR.

      - For each Segment in the MSR that does not start with an EDRAP picture, there shall be no Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR.

## Proposed spec text changes

The proposed text changes are provided below.

### Definitions

### extended dependent random access point (EDRAP) picture

picture in a sample that is a member of an EDRAP or DRAP sample group in an ISOBMFF track

### external elementary stream

elementary stream containing access units with external pictures

### external picture

picture that is in the external elementary stream in an ESR and is needed for inter prediction reference in decoding of the elementary stream in the MSR when random accessing from certain EDRAP pictures in the MSR

### External Stream Representation (ESR)

Representation containing an external elementary stream

### Main Stream Representation (MSR)

Representation containing a video elementary stream

### Semantics of AdaptationSet element

Table 5 — Semantics of AdaptationSet element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | AdaptationSet |  |  | Adaptation Set description |
|  |  |  | ... |  |  |
|  |  |  | @subsegmentStartsWithSAP | OD default: 0 | when greater than 0, specifies that each Subsegment with SAP_type greater than 0 starts with a SAP of type less than or equal to the value of @subsegmentStartsWithSAP. A Subsegment starts with SAP when the Subsegment contains a SAP, and for the first SAP, ISAU is the index of the first access unit that follows ISAP, and ISAP is contained in the Subsegment. The semantics of @subsegmentStartsWithSAP equal to 0 are unspecified. |
|  |  |  | @esasFlag | OD default: false | #### when not set to 'false', this specifies that all Representations in the Adaptation Set are Main Stream Representations (MSRs). The value set to 'true' specifies that all Representations in the Adaptation Sets are External Stream Representations (ESRs). When not present, the value of this attribute is inferred to be equal to 'false'. When the value of this attribute is equal to 'true' for an Adaptation Set ( i.e., all Representations in this Adaptation Set are ESRs), each Representation in this Adaptation Set shall be associated with an MSR through the Representation attributes @associationId and @associationType in the MSR, i.e., the @id of each Representation in this Adaptation Set shall be referred to by a value contained in the attribute @associationId corresponding to a value in the attribute @associationType equal to 'aest', where @associationId and @associationType are Representation attributes of an MSR. When the value of this attribute is equal to 'true' for an Adaptation Set, the Adaptation Set shall have an EssentialProperty descriptor with a @schemeIdUri of urn:mpeg:dash:adaptation-set-external-stream:2021; the presence of this EssentialProperty indicates that a Representation in the Adaptation Set shall not be consumed or played back by itself without other video Representations. When the value of this attribute is equal to 'false' for an Adaptation Set ( i.e., all Representations in this Adaptation Set are MSRs), each EDRAP picture in any Representation in this Adaptation Set shall be the first picture in a Segment. For an MSR and an ESR associated with each other through the Representation attributes @associationId and @associationType in the MSR, the following constraints apply: - For each Segment in the MSR that starts with an EDRAP picture, there shall be a Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR, wherein the Segment in the ESR carries the external pictures needed for decoding of that EDRAP picture and the subsequent pictures in decoding order in the bitstream carried in the MSR. - For each Segment in the MSR that does not start with an EDRAP picture, there shall be no Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR. |
|  |  |  | ... |  |  |

### 5.3.3.3	XML syntax

<!-- Adaptation Set --> <xs:complexType name="AdaptationSetType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentComponent" type="ContentComponentType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="Representation" type="RepresentationType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="group" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="IETF RFC6838ContentTypeType"/> <xs:attribute name="par" type="RatioType"/> <xs:attribute name="minBandwidth" type="xs:unsignedInt"/> <xs:attribute name="maxBandwidth" type="xs:unsignedInt"/> <xs:attribute name="minWidth" type="xs:unsignedInt"/> <xs:attribute name="maxWidth" type="xs:unsignedInt"/> <xs:attribute name="minHeight" type="xs:unsignedInt"/> <xs:attribute name="maxHeight" type="xs:unsignedInt"/> <xs:attribute name="minFrameRate" type="FrameRateType"/> <xs:attribute name="maxFrameRate" type="FrameRateType"/> <xs:attribute name="segmentAlignment" type="xs:boolean" default="false"/> <xs:attribute name="subsegmentAlignment" type="xs:boolean" default="false"/> <xs:attribute name="subsegmentStartsWithSAP" type="SAPType" default="0"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean"/> <xs:attribute name="esasFlag" type="xs:boolean" default="false"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Ratio Type for sar and par --> <xs:simpleType name="RatioType"> <xs:restriction base="xs:string"> <xs:pattern value="[0-9]*:[0-9]*"/> </xs:restriction> </xs:simpleType> <!-- Type for Frame Rate --> <xs:simpleType name="FrameRateType"> <xs:restriction base="xs:string"> <xs:pattern value="[0-9]*[0-9](/[0-9]*[0-9])?"/> </xs:restriction> </xs:simpleType> <!-- Type for IETF RFC6838 Content Type --> <xs:simpleType name="IETF RFC6838ContentTypeType"> <xs:restriction base="xs:string"> <xs:enumeration value="text" /> <xs:enumeration value="image" /> <xs:enumeration value="audio" /> <xs:enumeration value="video" /> <xs:enumeration value="application" /> <xs:enumeration value="font" /> </xs:restriction> </simpleType>

## Questions/comments and answers/responses

Discussions of m56674 at the 134th MPEG meeting can be found herein: http://mpegx.int-evry.fr/software/MPEG/Systems/DASH/spec/-/issues/156.

Below are the proponents' additional answers/responses to the questions/comments, particularly for those that they thought additional clarifications would likely be helpful for better understanding.

- Comment: I believe I understand the concept. However, I cannot imagine that a single flag can solve the issue.

Response: Firstly the proposal builds on the basis of the ISOBMFF EDRAP and DRAP sample groups. Secondly, in addition to the flag attribute, the proposal also contained, as part of the semantics, 1) the association of an ESR to an MSR through existing Representation attributes @associationId and @associationType based a newly specified association type value; and 2) some constraints for simplifying the EDRAP based streaming operations. Lastly, at this meeting, we further added a new EssentialProperty descriptor to avoid DASH clients selecting and playing back an ESR by itself.

- How does the random access work with a player using MSE, i.e. when a random access is made, what segments of the external representation to put in MSE, and how continue with the segments from the main representation such that the decoder continues to decode?

Response: When random accessing from a segment in an MSR, only the time-synchronized segment in the associated ESR needs to be added immediately before that MSR segment, then the decoder continues decoding and output pictures as usual. The continuation decoding of any following segment in the MSR is also as usual (insertion of an ESR segment is only needed for random access from the time-synchronized MSR segment, not needed for continuation decoding, as shown in page 5 of the slides attached to the contribution document).

In MSE terms: When random accessing from a segment in an MSR, the following steps apply:

  - Firstly the time-synchronized segment in the associated ESR is inserted to MSE source buffer immediately before the media data of the MSR segment.

  - Then everything works as usual, including that the media data of the MSR continues to flow into the source buffer.

## Suggestions from the 135th MPEG meeting

Discussions of m57430 at the 135th MPEG meeting can be found herein: http://mpegx.int-evry.fr/software/MPEG/Systems/DASH/spec/-/issues/190.

Two key suggestions:

- Explore the resync signaling for the location of EDRAP chunks.

- Explore how to apply common encryption to these tracks.

Clarifying pre-roll signaling for seamless content splicing across MPEG-DASH Periods (m56890)

Note: the issue of audio prerolling during period transitioning is being discussed in ISOBMFF as well as CMAF. We will investigate how it should be the best to address.

## Introduction

In our experiments on splicing, DASH-ing, and playing content back we have encountered issues to provide the required information to the playback device to provide a perfect experience (from a Systems perspective). These issues may happen with audio, video, and other media as well.

Splicing points may be mandated whenever the most suitable from a system-level perspective i.e. at segment boundaries or when SAP types 1 or 2 occur. However, this is not always possible or desirable (e.g. because it would require to re-process the content or e.g. because for historical reasons SAP Types are not well signaled).

For these use-cases we realized that MPEG-DASH (and other HTTP Adaptive Streaming (HAS) technologies) is missing signaling to allow a player to get some context information that is needed to reconstruct the container-advertised content.

## Related contributions

Contributions m55420 (October 2020) and m55478 (liaison letter from DVB) raise questions around the correct usage of DASH multi-period with regards with content continuity. Both contributions raise pragmatic questions. The present contribution aims at apprehending the same category of issues from a more generic perspective.

## Discussion

### Pre-roll context

Issues when splicing occur when the splice point doesn’t respect the SAP Type 1 or 2 constraints. Then the effective presentation duration may differ from the presentation duration advertised at the container and MPD levels because the player (in particular an elementary stream decoder) doesn’t have the decoding context which would allow to fully decode the segment, ensuring a perfect playback experience.

The difference between the decoded data duration and the presentation duration is usually handled using edit lists. In this case the edit list entries serve both to compensate some introduced delays as well as adjusting the duration (hence discarding data that is not intended to be presented).

Usually, a single decoder is used to decode a single content. Hence a single initialization segment can contain some general information about the processing of this non-presented data.

As SAP Types postdates the introduction of encoding technologies using pre-roll some streams (typically audio ones) are still commonly signaled as SAP Type 1 while this is not correct.

The pre-roll information should be present inside the container in conformance to the applicable standards.

## Use-cases

### Ad insertion using multiple Periods

In this use case, advertisement content (Period #2) is inserted in the middle of a main content (Periods #1 and #3). When switching back from the ad to the main content some players may need to reinitialize themselves. In this case the pre-roll information and associated data shall be present to ensure a perfect playback experience.

In current HAS deployments, this pre-roll data is located in a previous segment whose URL is unknown at the HAS layer, as it requires identifying the pre-roll count of the first sample of the new segment to play.

### Open-GOPs splicing

Open-GOPs have been largely avoided by the industry due to the lack of proper description of how to handle them. As codecs using similar mechanisms become more widespread, the information about how to reconstruct a full signal over the advertised period is more necessary.

## Discussions

There are two possible approaches to solve this issue:

- Identify pre-roll data as part of the HAS manifest.

- Embed pre-roll data in media segments.

Working at the manifest level has the following benefits:

- No changes required to media segments.

Working at the manifest level has the following drawbacks:

- Pre-roll URL needs to be indicated for each switching point, i.e. potentially for each segment.

- The system is dependent of the length of the time-shift buffer and requires the past segment containing the pre-roll content to be available; this is usually not the case in broadcast environments such as ROUTE.

- The solution will have to be declined for each possible manifest formats.

- The URL might not be fine-grained enough, resulting in fetching more data than needed (potentially a full segment or subsegment), i.e. the moof+mdat identified is much larger than the pre-roll data. Reducing the duration of subsegments to match the pre-roll duration is possible but will increase the overhead (for AAC, this could mean one moof+mdat per AAC frame).

- It requires the initialization segment (in the case of ISOBMFF) to be the same between the pre-roll segment and the actual segment, or a fetch of the previous init segment to parse the pre-roll data.

Working at the segment level has the following benefits:

- Independent from the HAS solution (DASH, HLS, pure CMAF).

- Only the required pre-roll data is copied in the media segment, lowering overhead when switching.

Working at the segment level has the following drawbacks:

- It requires update to the media segment parsing.

- It may require updates of the media pipeline, or an adaptation layer reformatting the segment at switch point.

- The solution will have to be declined for each possible segment formats

- Duplication of media data.

A third approach, specific to MPEG-DASH, could be to use the BitstreamSwitching segment, currently not defined for ISOBMFF. Such a bitstream switching segment would only contain the pre-roll AUs, formatted according to the current init segment. It will still result in media duplication, but does not require any modification to the media segment.

A fourth approach, only valid for splice-out case (main content resume), would be to not use period continuity signaling and copy over the pre-roll data in the first segment of the new period.

## Conclusion

We believe it is MPEG responsibility to address these issues urgently, and kindly request WG3 to investigate possible actions to define guidelines or technical proposals towards a unified, decoding-exact handling of splicing and non-SAP1 switching.

Support of picture-in-picture services in DASH (m57431)

## Abstract

This contribution proposes an extension to the Preselection design for support of the picture-in-picture experience.

## Introduction

Picture-in-picture services offer the ability to include a picture with a small resolution within a picture with a bigger resolution. Such a service may be beneficial to show two videos to a user at the same time, whereby the video with bigger resolution is considered as the main video and the video with a smaller resolution is considered as the supplementary video. Such a picture-in-picture service can be used to offer accessibility services where the main video is supplemented by another video, e.g., a signage video.

VVC subpictures can be used for picture-in-picture services by using both the extraction and merging properties of VVC subpictures. For such a service, the main video is encoded using a number of subpictures, one of them of the same size as a supplementary video, located at the exact location where the supplementary video is intended to be composited into the main video and coded independently to enable extraction. If a user chooses to view the version of the service that includes the supplementary video, the subpicture that corresponds to the picture-in-picture area of the main video is extracted from the main video bitstream, and the supplementary video bitstream is merged with the main video bitstream in its place, as illustrated in the following figure:

Fig. 1: An example of picture-in-picture support based on VVC subpictures

When both main and supplementary bitstreams are available via a DASH-based delivery system, the Preselection may be used to signal main and supplementary bitstreams that are intended to be merged and rendered together.

However, when it comes to use of Preselection for support of the picture-in-picture experience, the following issues have been observed:

- While it is possible to use a Preselection for the picture-in-picture experience, there lacks an indication of such a purpose.

- While it is possible to use VVC subpictures for the picture-in-picture experience e.g., as discussed above, it is also possible to use other codecs and methods without being able to replace the coded video data units representing the target picture-in-picture region in the main video with the corresponding video data units of the supplementary video. Therefore, it should be able to indicate whether such replacement is possible.

- When the above replacement is possible, the client needs to know which coded video data units in each picture of the main video represent the target picture-in-picture region, to be able to perform the replacement. Therefore, this information needs to be signalled.

- For content selection purpose and possibly other purposes as well, it would be useful to signal the position and size of the target picture-in-picture region in the main video.

This contribution proposes some extensions to the Preselection design for support of the picture-in-picture experience.

## Proposal

The proposal is summarized as follows:

- Specify the value "PicInPic" for the @tag attribute of the Preselection element (through the CommonAttributesElements) for indicating the purpose of the Preselection being for providing the picture-in-picture experience.

- Add an optional element Preselection.Picnpic that contains the following attributes:

  - An optional attribute @dataUnitsReplacable for indicating whether the coded video data units representing the target picture-in-picture region in the main video can be replaced by the corresponding video data units of the supplementary video.

  - An optional attribute @regionIds for indicating which coded video data units in each picture of the main video represent the target picture-in-picture region.

    - In the case of VVC, the region IDs are subpicture IDs and coded video data units are VCL NAL units.

  - Four optional attributes @x, @y, @width, and @height for signalling the position and size in the main video for embedding/overlaying the supplementary video.

- Add a subclause as part of the Preselection subclause for introduction of picture-in-picture services and the use of the above information for support of the picture-in-picture experience.

The exact specification text changes are provided in an attachment in the contribution m57431.

## Discussions at the 135th MPEG meeting

Details of the discussions can be found herein: http://mpegx.int-evry.fr/software/MPEG/Systems/DASH/spec/-/issues/191.

Key points:

- Currently in the DASH spec there is a Role value "sign" defined for accessibility. The closed signing (see ATSC A/300 clause 5.3.5) as a Role value seemed relevant.

- For simple cases, e.g., when there is only one "main" video Adaptation Set and only one "sign" video Adaptation Set, implicit association of the two video Adaptation Sets using the Role values "main" and "sign" can be sufficient for associating the two video Adaptation Sets.

- Regarding the indication of the purpose of a Preselection being for "picture-in-picture", it was commented that tags are application-specific and are not defined in DASH. Thus defining a specific Role value would be better for this.

- Regarding the attribute @dataUnitsReplacable, it was suggested to define different Role schemes for this or a descriptor instead of using an attribute as this is application logic. Similarly, for the attributes @regionIds, @x, @y, @width, and @height, defining descriptor scheme(s) was also suggested for a similar reason.

  - It was suggested that if such design principles/guidelines are not yet clearly documented in the spec, it would probably be good to document them somewhere in the DASH spec.

- If possible, consider finding a better term than "picture-in-picture".