# ISO/IEC JTC 1/SC 29/WG 03 N0425

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	WD of ISO/IEC 23009-1 5th edition AMD2 EDRAP streaming and other extensions

Status:	Approved

Date of document:	2021-11-02

Source:	ISO/IEC JTC 1/SC 29/WG 03

Expected action:		   None

Action due date:		   None

No. of pages:	4 (with cover page)

Email of Convenor:	young.L @ samsung . com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 03 N 0425

October 2021, Virtual

| Title | WD of ISO/IEC 23009-1 5th edition AMD2 EDRAP streaming and other extensions |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 21030 |

[Ed. (YK): Per the DASH BoG meeting minutes in m58396, the following updates should be made for improving the text:

- The text needs to be updated with the DASH level terminology and avoid elementary bitstream terms.

- The DASH client operation needs to be added.

- Show the relationship between EDRAP ES and EDRAP sample entry in ISOBMFF WD.]

# In subclause 3.1, add the following definitions:

### extended dependent random access point (EDRAP) picture

picture in a sample that is a member of an EDRAP or DRAP sample group in an ISOBMFF track

### external elementary stream

elementary stream containing access units with external pictures

### external picture

picture that is in the external elementary stream in an ESR and is needed for inter prediction reference in decoding of the elementary stream in the MSR when random accessing from certain EDRAP pictures in the MSR

### external stream Representation (ESR)

Representation containing an external elementary stream

### main stream Representation (MSR)

Representation containing a video elementary stream

# In subclause 3.2, add the following abbreviations:

| EDRAP | extended dependent random access point |
| --- | --- |
| ESR | external stream Representatio |
| MSR | acknowledgement message in TCP |

# Add subclause 5.8.5.15 as follows:

#### 5.8.5.15	MSR and ESR descriptors

An Adaptation Set may have an EssentialProperty descriptor with @schemeIdUri equal to urn:mpeg:dash:msr:2021. This descriptor is referred to as the MSR descriptor. The presence of this EssentialProperty indicates that each Representation in this Adaptation Set is an MSR.

The following applies for MSRs:

- Each SAP in an MSR Representation in the Adaptation Set can be used for accessing the content in the Representation provided that the time-synced sample, when present in the track carried in the associated ESR, is made available to the client.

- Each EDRAP picture in an MSR shall be the first picture in a Segment (i.e., each EDRAP picture shall start a Segment).

An Adaptation Set may have an EssentialProperty descriptor with @schemeIdUri equal to urn:mpeg:dash:esr:2021. This descriptor is referred to as the ESR descriptor. The presence of this EssentialProperty indicates that each Representation in this Adaptation Set is an ESR. An ESR shall not be consumed or played back by itself without other video Representations.

Each MSR shall be associated with an MSR through the (existing) Representation-level attributes @associationId and @associationType in the MSR as follows: the @id of the associated ESR shall be referred to by a value contained in the attribute @associationId for which the corresponding value in the attribute @associationType is equal to 'aest'.

For an MSR and an ESR associated with each other through the Representation attributes @associationId and @associationType in the MSR, the following constraints apply:

- For each Segment in the MSR that starts with an EDRAP picture, there shall be a Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR, wherein the Segment in the ESR carries the external pictures needed for decoding of that EDRAP picture and the subsequent pictures in decoding order in the bitstream carried in the MSR.

- For each Segment in the MSR that does not start with an EDRAP picture, there shall be no Segment in the ESR having the same Segment start time derived from the MPD as the Segment in the MSR.