ISO/IEC JTC 1/SC 29 N 00381

Date: 2022-06-03

ISO/IEC 23001-17:202X(E)

ISO/IEC JTC 1/SC 29/WG 3

Secretariat: JISC

Information technology — MPEG Systems technologies — Part17: Uncompressed video in ISO Base Media File Format

Document type: Committee Draft International Standard

Document subtype:

Document language: E

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manager of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	6

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

3.1	sample	1

3.2	component	2

3.3	uncompressed video	2

3.4	frame	2

3.5	pixel	2

3.6	interleaving	2

3.7	tile	2

3.8	row	2

3.9	block	2

4	Technical overview	2

5	Uncompressed video formats	3

5.1	Overview	3

5.2	Uncompressed video	4

5.2.1	Generic uncompressed video	4

5.2.2	Generic uncompressed video configuration	4

5.3	Profiles for uncompressed video configurations	17

5.3.1	Overview	17

5.3.2	Predefined configurations	17

5.4	MIME type sub-parameters	19

6	Video sample entry extensions	19

6.1.1	Overview	19

6.1.2	Palette configuration	20

6.1.3	Pattern Definition	21

6.1.4	Reference Level	22

6.1.5	Polarization Pattern Definition	24

6.1.6	Sensor Non-Uniformity Correction	25

6.1.7	Sensor Bad Pixels Map	26

6.1.8	Chroma Location	27

6.1.9	Frame Packing Information	28

6.1.10	Disparity Information	28

6.1.11	Depth Mapping Information	29

7	Sample group descriptions	29

7.1	Field Interlace Type	29

7.1.1	Definition	29

7.1.2	Syntax	30

7.1.3	Semantics	30

8	Image Item properties	30

8.1	Field Interlace Property	30

8.1.1	Definition	30

8.1.2	Syntax	31

8.1.3	Semantics	31

9	Multiple track storage	31

9.1	Overview	31

9.2	Component video track group	31

9.3	Image tiling using ISOBMFF tracks and items	32

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents) or the IEC list of patent declarations received (see http://patents.iec.ch).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see www.iso.org/iso/foreword.html.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information Technology, Subcommittee SC 29, Coding of audio, picture, multimedia, and hypermedia.

A list of all parts in the ISO/IEC 23001 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Information technology — MPEG Systems technologies — Part 17: Uncompressed video in ISO Base Media File Format

# Scope

This document defines how uncompressed 2D image or video data is carried in files in the family of standards based on the ISO base media file format. This includes but is not limited to monochromatic data, colour data, transparency (alpha) information, depth information.

The primary goal of this specification is to allow exchange of uncompressed video or image data while relying on the information set provided by the ISO base media file format, such as timing, colour space, sample aspect ratio to specify the interpretation and/or display of that video or image data.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format

ISO/IEC 23008-12, Information technology – High efficiency coding and media delivery in heterogeneous environments – Part 12: Image File Format (HEIF)

ISO/IEC 23091-2, Information technology —Coding-independent code points — Part 2: Video

ISO/IEC 23002-3, Information technology — MPEG video technologies — Part 3: Representation of auxiliary video and supplemental information

IEEE 754, IEEE Std 754™-2008, IEEE Standard for Floating-Point Arithmetic

# Terms and definitions

For the purposes of this document, the terms and definitions given in ISO/IEC 14496-12 and the following apply.

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at https://www.iso.org/obp

—	IEC Electropedia: available at http://www.electropedia.org/

sample

media sample when the uncompressed data is described by a media track, or the payload of an item when the uncompressed data is described by an image item

Note 1 to entry: Media sample as defined in 14496-12.

Note 2 to entry: Payload of an item as defined in 14496-12.

component

visual or non-visual information

Note to entry: In this specification, a component may describe visible information such as luminance or chroma, or other information usually not intended for direct display such as depth or transparency.

uncompressed video

A sequence of one or more frames for which each value of each component is coded independently from any other component value in the same frame or any other frame

Note to entry: In this specification, the uncompressed term is used with some video formats applying sub-sampling of some components for the purpose of data reduction; however, data access to each individual component for such formats is still independent from other components or frames.

frame

two-dimensional rectangular array of pixels contained within a sample

pixel

point associated with one or more components

interleaving

pixel and component order within the sample data of component values

tile

two-dimensional rectangular array of pixels within a frame

row

horizonal line of pixels within a frame or a tile

block

consecutive bytes within the sample data containing one or more component values

# Technical overview

ISO/IEC 14496-12 shall be used as framework for the video extensions described in this document.

ISO/IEC 14496-12 or ISO/IEC 23008-12 shall be used as framework for the image extensions described in this document.

The extensions included in this document provide the necessary specifications to carry uncompressed video and images in these frameworks.

Media tracks and media samples may be associated with meta-data information using the various tools defined in ISO/IEC 14496-12, such as sample group descriptions, MetaBox, metadata tracks and sample auxiliary information.

Media items may be associated with meta-data information using the various tools defined in ISO/IEC 14496-12 or ISO/IEC 23008-12, such as descriptive item properties.

# Uncompressed video formats

## Overview

Uncompressed video formats may be stored in ISO base media files using a generic uncompressed video description defined hereafter.

For media tracks, uncompressed video formats shall be described by a sample entry extending the VisualSampleEntry. The handler type associated with the track is usually 'vide', 'auxv' or 'pict' but derived specifications may introduce new handler types. The width and height fields of the sample entry shall document the exact frame dimension, in pixels, of any sample of the video stream that is described by this sample entry. Consequently, if the frame dimension changes within a video track, multiple sample entries must be used.

For image items, uncompressed images shall be associated with a ImageSpatialExtentsProperty whose image_width and image_height fields shall document the exact frame dimension, in pixels, of the reconstructed image, i.e. the size of the image before applying any associated transformative properties.

The tools defined in ISO/IEC 14496-12 and ISO/IEC 23008-12 should be used whenever applicable, namely to specify pixel aspect ratio, colour information, clean aperture, content light level, mirror and rotate properties or track header matrix, etc.

An uncompressed video media sample or item consists of one frame. Each frame is organized as a set of one or more rectangular, non-overlapping areas called tiles.

The first byte of the sample data shall be the first byte of the block containing the first component of the top-left pixel of the frame. The size in bytes of the associated media sample or item shall be at least the size in bytes required to store all the components values documented by the uncompressed video configuration as defined in subclause 5.2.2.

For media tracks, each uncompressed video sample is a sync sample. Consequently, the SyncSampleBox (‘stss’) should be absent from the track.

Media tracks containing only non-visual components should be marked as not present in the presentation, i.e. track_in_movie and track_in_preview flags should not be present.

Image items containing only non-visual components should be marked them as hidden items, i.e. have (flags & 1) equal to 1 in their ItemInfoEntry.

## Uncompressed video

### Generic uncompressed video

Generic uncompressed video media compliant to this specification shall use a video sample entry with codingname equal to 'uncv' containing one UncompressedVideoConfigBox. The compressorname field of an uncompressed video sample entry should be set to all 0 (empty string). The depth field of an uncompressed video sample entry shall be ignored and should be set to 0, the bit depth per component being indicated by the UncompressedVideoConfigBox .

A generic uncompressed image item shall use the item_type 'uncv' and an associated UncompressedVideoConfigBox essential item property, i.e., essential shall be equal to 1 for an UncompressedVideoConfigBox item property associated with an image item of type 'uncv'.

### Generic uncompressed video configuration

#### Definition

##### Overview

Box Type:	'uncC' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	Yes, if codingname of the sample entry is 'uncv' or if the item_type of the item is 'uncv'
Quantity:	Zero or one per video sample entry or per item

The UncompressedVideoConfigBox is used to document the format of the image data. This box appears in a video sample entry of type 'uncv', or is associated with an item of type 'uncv' in the ItemPropertyAssociationBox.

The UncompressedVideoConfigBox may indicate a profile for this description, allowing faster identification of the class of video data used. Profiles are defined in subclause 5.3.

The UncompressedVideoConfigBox describes uncompressed video frames that are composed of one or more components. RGB or YUV formats are typical examples of such uncompressed video samples for which each colour component is a component of the uncompressed video sample. Other component types can also be described, such as non-visual information (e.g., disparity, transparency) or non-visible information (e.g., infra-red).

Component values may be absolute values or indexes into a colour palette, with adjustable black and white reference levels. Pattern-based sensor data, such as Bayer, can be described through user-defined patterns, together with various sensor information (polarisation, non-uniformity correction, broken pixels, etc.).

Component values may be interleaved per component, per pixel, per row or per tile. Component values may also be grouped together in blocks, typically to respect endianness constraints.

##### Component type

Each pixel in a frame is made of one or more components, where each component is assigned a type (e.g., ‘Y’, ‘U’, ‘V’), as detailed in Table 1.

NOTE1	In some cases, two or more components in a UncompressedVideoConfigBox may have the same type (e.g., two IR components representing different portions of the “mid-wave infrared” portion of the electromagnetic spectrum), and file readers may require additional information to process the pixel data. How such information is provided to the file reader is out of scope of this specification.

The order in which components occur in the UncompressedVideoConfigBox indicates the order in which components are stored within the sample data.

Some formats, such as YUV video, do not always use the same 2D resolution for each component of the frame. This is indicated by the sampling_type field.

Some formats, such as Bayer image data, contain a 2D array of single-component values, with individual component value assigned to component types using a fixed pattern. Such formats are described as mono-component data with no subsampling, use a component type of 11 (‘FA’). There shall be at most one component with type 11 (‘FA’) present in the component list. There may be additional components of other type present, for example to associate an alpha component with a Bayer image. If a component with type 11 (‘FA’) is present, there shall be an UncompressedPatternDefinitionBox in the video sample entry to indicate the pattern of component values.

Some formats code colours according to a set of predefined colours, or palette. Such formats are described as single-component with no subsampling, use a component type of 10 (‘P’). There shall be at most one component with type 10 (‘P’) present in the component list. There may be additional components of other type present, for example to associate per-pixel alpha component with a palette image. If a component with type 10 (‘P’) is present, there shall be an UncompressedPaletteBox in the video sample entry to indicate the palette values.

Table 1 – Component types

| Value | Description |
| --- | --- |
| 0 | Visible monochrome component |
| 1 | Luma component (Y) |
| 2 | Chroma component (Cb / U) |
| 3 | Chroma component (Cr / V) |
| 4 | Red component (R) |
| 5 | Green component (G) |
| 6 | Blue component(B) |
| 7 | Alpha/transparency component (A) |
| 8 | Depth component (D) |
| 9 | Disparity component (Disp) |
| 10 | Palette component (P) The component_format value for this component shall be 0. |
| 11 | Filter Array component such as Bayer, RGBW, etc. (FA) |
| 12 | Padded component (unused bits/bytes) |
| 13 | Gamma |
| 14 | X-ray |
| 15 | Vacuum Ultraviolet (VUV) |
| 16 | Ultraviolet C (UVC) |
| 17 | Ultraviolet B (UVB) |
| 18 | Ultraviolet A (UVA) |
| 19 | Near Infra-Red (NIR) |
| 20 | Short Wavelength Infra-Red (SWIR) |
| 21 | Middle Wavelength Infra-Red (MWIR) |
| 22 | Long Wavelength Infra-Red (LWIR) |
| 23 | Synthetic Aperture Radar (SAR) – Complex Data The component_format value for this component shall be 2. |
| 24 | Synthetic Aperture Radar (SAR) – Magnitude |
| 25 | Synthetic Aperture Radar (SAR) – Phase |
| 26 | Inverse Synthetic Aperture Radar (ISAR) – Complex Data The component_format value for this component shall be 2. |
| 27 | Inverse Synthetic Aperture Radar (ISAR) – Magnitude |
| 28 | Inverse Synthetic Aperture Radar (ISAR) – Phase |
| 29-0xFFFE | ISO/IEC reserved |
| 0xFFFF | User-defined |

NOTE 2	Component types reflect only a nominal characterization of the pixel data; precise bandpass limits, for example, are not implied and may differ from image to image. The signalling of such information is out of scope of this specification.

##### Component size and numerical format

The variable component_bit_depth for a component is defined as (component_bit_depth_minus_one + 1).

For a given component, the binary representation of each value is given by component_bit_depth (the size in bits of each component value) and component_format. The possible values for component_format field is defined in Table 2.

Table 2 - Component formats

| Value | Description |
| --- | --- |
| 0 | Component value is an unsigned integer coded on component_bit_depth bits. |
| 1 | Component value is an IEEE 754 binary float number coded on component_bit_depth bits (e.g., if component_bit_depth is 16, then the component value is coded as IEEE 754 “binary16”). For this component format, component_bit_depth values shall be 16, 32, 64, 128 or 256; other values are forbidden. |
| 2 | Component value is a complex number coded on component_bit_depth bits, where the first component_bit_depth/2 bits represent the real part and the next component_bit_depth/2 bits represent the imaginary part. Each part shall be coded as an IEEE 754 binary float of the size component_bit_depth/2. For this component format, component_bit_depth values shall be 32, 64, 128 or 256; other values are forbidden. |
| 3 – 255 | ISO/IEC reserved for future definition |

##### Tiling

Frame data is structured in one or more tiles. Tiles allow grouping together the component values of pixels close to each-other (i.e., in the same spatial region of the frame).

All tiles have the same width and height. The tile width is w/(num_tile_cols_minus_one+1), with w the frame width, and the tile height is h/(num_tile_rows_minus_one+1), with h the frame height.

The frame width (resp. height) shall be a multiple of num_tile_cols_minus_one+1 (resp. num_tile_rows_minus_one+1).

If the frame width (resp. height) in a source image is not an integer multiple of num_tile_cols_minus_one+1, (resp. num_tile_rows_minus_one+1), an application creating the tiled image shall pad the source image with an appropriate number of columns to the right (resp. rows to the bottom), and the original frame dimension shall be documented using a CleanApertureBox for media tracks or a clean aperture transformative item property for image items.

Tiles shall be stored in raster-scan order: the top-left tile is stored first, followed by the tile to its right, and the first tile of a tile row is stored after the last tile of the previous tile row.

Within a tile, component values shall be stored in raster-scan order, left-to-right and top to bottom: for a given component, the value for pixel {x+1,y} is stored after (but possibly not contiguous with) the value for pixel {x,y}, and the value for pixel {x,y+1} is stored after (but possibly not contiguous with) the value for pixel {x, y}.

##### Sampling type

All components in a frame either have the same dimensions or use pre-defined sampling modes, indicated by the sampling_type field. Possible values for this field are described in Table 3.

NOTE1 Sampling type values restrict the possible interleaving modes since the number of values is not the same for each component.

NOTE2 Derived specifications may further restrict the usage of sampling types with tiling.

Table 3 – Sampling type values

| Value | Definition |
| --- | --- |
| 0 | No subsampling All components have the same 2D size, given by the sample entry. |
| 1 | YCbCr 4:2:2 subsampling This value shall only be set if all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list and if the tile width is a multiple of 2. If this value is used, the interleave_type field shall be set to 0, 2 or 5. The 2D size of the ‘Y’ component is given by the sample entry. The height of the ‘U’ and ‘V’ component is the same as the height of the ‘Y’ component. The width of the ‘U’ and ‘V’ component is half the width of the ‘Y’ component. Components of other types may be present, and have the same dimension as the ‘Y’ component. Pixels {x,y} and {x+1,y}, with x%2==0, share the same component values ‘U’ and ‘V’. If row_align_size is not 0 and interleave_type is 0: - row_align_size shall be a multiple of 2 - the size of rows for components other than ‘U’ and ‘V’ shall be row_align_size bytes - the size of rows for ‘U’ and ‘V’ components shall be row_align_size/2 bytes |
| 2 | YCbCr 4:2:0 subsampling This value shall only be set if all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list and if both tile width and height are multiple of 2. If this value is used, the interleave_type field shall be set to 0 or 2. The 2D size of the ‘Y’ component is given by the sample entry. The width of the ‘U’ and ‘V’ component is half the width of the ‘Y’ component’. The height of the ‘U’ and ‘V’ component is half the height of the ‘Y’ component. Components of other types may be present, and have the same dimension as the ‘Y’ component. Pixels {x,y}, {x+1,y}, {x,y+1} and {x+1,y+1} with x%2==0 and y%2==0, share the same component values ‘U’ and ‘V’. If row_align_size is not 0 and interleave_type is 0: - row_align_size shall be a multiple of 2 - the size of rows for components other than ‘U’ and ‘V’ shall be row_align_size bytes - the size of rows for ‘U’ and ‘V’ components shall be row_align_size/2 bytes |
| 3 | YCbCr 4:1:1 subsampling This value shall only be set if all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list and if tile width is a multiple of 4. If this value is used, the interleave_type field shall be set to 0, 2 or 5. The 2D size of the ‘Y’ component is given by the sample entry. The height of the ‘U’ and ‘V’ component is the same as the height of the ‘Y’ component. The width of the ‘U’ and ‘V’ component is the width of the ‘Y’ component divided by 4. Components of other types may be present, and have the same dimension as the ‘Y’ component. Pixels {x,y} , {x+1,y} , {x+2,y} , {x+3,y}, with x%4==0, share the same component values ‘U’ and ‘V’. If row_align_size is not 0 and interleave_type is 0: - row_align_size shall be a multiple of 4 - the size of rows for components other than ‘U’ and ‘V’ shall be row_align_size bytes - the size of rows for ‘U’ and ‘V’ components shall be row_align_size/4 bytes |
| 4-0xFF | reserved |

##### Interleaving

The interleaving of pixels within a tile (or withing the frame if a single tile is used) is indicated by interleave_type, as defined in Table 4. The interleave_type shall apply to all listed components unless stated otherwise in Table 4.

NOTE The interleaving describes the layout of values within a sample. Positioning of sample data within the container file is done according to ISO/IEC 14496-12.

Table 4 - Interleaving types

| Value | Definition |
| --- | --- |
| 0 | Component Interleaving For a given component, values for all pixels of a tile are located sequentially in the sample data. Component values are located in the order the components were declared. This mode can be used with a single tile, as illustrated in Figure 1, or with multiple tiles, as illustrated in Figure 2. In both examples, R is the first component, G is the second and B is the third. Figure 1 Single tile Component Interleaving example Figure 2 Multiple tiles Component Interleaving example |
| 1 | Pixel Interleaving For a given pixel, the component values are located one after the other, in the order the components are declared, with the first component of a pixel following the last component of the previous pixel. Pixel Interleaving is only permitted when all components are at the same resolution. This mode can be used with a single tile, as illustrated in Figure 3, or with multiple tiles, as illustrated in Figure 4. Figure 3 Single tile Pixel Interleaving example Figure 4 Multiple tiles Pixel Interleaving example |
| 2 | Mixed Interleaving This value shall only be used if sampling_type field value is not 0, if sampling_type value explicitly allows this interleaving mode and if the ‘U’ and ‘V’ components are consecutive in the component list, i.e. ‘V’ component immediately follows ‘U’ component or ‘U’ component immediately follows ‘V’ component, For all components other than ‘U’ and ‘V’, component values for all pixels are located as specified for Component Interleaving in the order they are declared. The ‘U’ and ‘V’ component values are located as specified by Pixel Interleaving. NOTE This mode is typically used to store YUV 420 data with all Y components followed by interleaved U and V components. Figure 5 illustrates this mode for sampling_type=2 and Figure 6 illustrates this mode for sampling_type=1. Figure 5 Mixed Interleaving for YUV 420 example Figure 6 Mixed Inter leaving for YUV422 example |
| 3 | Row Interleaving For a given row, component values for each component are located sequentially, in the order the components are declared. Each row continues after the previous row. If multiple tiles are used, the first component of the first row of a tile is located after the last component of the last row of the previous tile. Row Interleaving is only permitted when all components are at the same resolution. This mode can be used with a single tile, as illustrated in Figure 7, or with multiple tiles, as illustrated in Figure 8. Figure 7 Single tile Row Interleaving example Figure 8 Multiple tiles Row Interleaving example |
| 4 | Tile-Component Interleaving For each component, in the order they are declared, the values for each component for all tiles are located sequentially, with values within the tile located per each individual tile. This mode shall not be used if only a single tile is defined. This mode is illustrated in Figure 9. Figure 9 Tile-Component Interleaving example |
| 5 | Multi-Y Pixel Interleaving Component values are organized as with interleave_type=1 (Pixel Interleaved), but the list of components contains multiple ‘Y’ components representing the multiple ‘Y’ component values associated with a single pair of ‘U’ and ’V’ component values. The ‘U’ and ‘V’ components shall only be listed once in the list of components. If the chrominance components are subsampled according to 4:2:2 subsampling (sampling_type=1), the component list describes two horizontally consecutive pixels, and thus shall include two ‘Y’ components, representing the two ‘Y’ values associated with a single pair of ‘U’ and ‘V’ component values. For example, {‘U’, ‘Y’, ‘V’, ‘Y’} for sampling_type=1 (YUV 4:2:2) indicates that the Y value of the second pixel is located after the ‘V’ component value. Figure 10: ‘U’, ‘Y’, ‘V’, ‘Y’ 4:2:2 example If the chrominance components are subsampled according to 4:1:1 subsampling, the component list describes four horizontally consecutive pixels, and thus shall include four Y components, representing the four Y values associated with a single pair of U and V component values. For example, {‘U’, ‘Y’, ‘Y’, ‘V’, ‘Y’, ‘Y’} for sampling_type=3 (YUV 4:1:1) indicates that the 'Y’ value of the second pixel is located after the ‘Y’ value of the first pixel, that the 'Y’ value of the third pixel is located after the ‘V’ component value and that the 'Y’ value of the fourth pixel is located after the ‘Y’ value of the third pixel. Figure 11: ‘U’, ‘Y’, ‘Y’, ‘V’, ‘Y’, ‘Y’ 4:1:1 example This interleave mode shall not be used for other forms of chrominance subsampling. |
| 6-0xFF | Reserved |

##### Alignment of values to byte boundaries

Padding bits can be included within the data to ensure that component values, rows or tiles are aligned to specific byte boundaries. This may be to simply ensure that individual component values are CPU addressable (e.g., not split across two 32-bit words), or in more complicated scenarios, to ensure that rows of the image align to memory pages or solid-state drive blocks.

Component values are stored within blocks, the first value of the first component of the top-left pixel of the frame being stored in the first block. The first byte of the first block is the first byte of the sample data.

The block size in bytes is specified by the block_size field.

If the block size is 0,

- no padding is performed between values, except for the last value in a row if row_align_size is set

- both block_pad_last and block_little_endian shall be 0

- values are stored most-significant byte first.

If the block size is not 0, each block is made of exactly block_size bytes and contains exactly one or more component values, i.e. the bits representing a component value are always contained within a single block. If the block size in bits is greater than the size in bits of component values present in the block, padding bits are present. All padding bits shall be ignored and should be set to 0.

NOTE1 Padding bits may be more than 8 (one byte).

When block_pad_last is 0, padding bits are the first bits of the block, i.e. located in the most significant bits of the block. Otherwise (block_pad_last is 1), padding bits are the last bits of the block, i.e. located in the least significant bits of the block.

If block_little_endian is 1,

- the first byte of the block in the sample data is the least significant byte and the last byte of the block is the most significant byte, i.e. bytes are stored in reverse order from big-endian ordering,

- the first component value in the block is located in the least significant bits (minus padding bits) and the last component value in the block is located in the most significant bits (minus padding bits).

Otherwise (block_little_endian is 0),

- bytes of the blocks are stored in big-endian order in the sample data

- the first component value in the block is located in the most significant bits (minus padding bits) and the last component value in the block is located in the least significant bits (minus padding bits).

The following figures illustrate storage of three components (‘R’ on 9 bits , ‘G’ on 10 bits , ‘B’ on 9 bits) in 32 bit words (block_size = 4), with “byte 0” being the first byte of the block in the sample data, the most significant bit of each byte being the first starting from the left:

- Figure 12 illustrates a block with block_little_endian=0 and block_pad_last =0.

- Figure 13 illustrates a block with block_little_endian=0 and block_pad_last =1

- Figure 14 illustrates a block with block_little_endian=1 and block_pad_last =0

- Figure 15 illustrates a block with block_little_endian=1 and block_pad_last =1

Figure 12 - Example of big-endian right aligned block

Figure 13 - Example of big-endian left aligned block

Figure 14 - Example of little-endian right aligned block

Figure 15 - Example of little endian left-aligned block

NOTE2 Block alignment allows groups of consecutive component values, regardless of interleaving type, to be aligned within an easily addressable value; e.g., multiple components can be packed into 32-bit values but no component value spans two consecutive 32-bit values. For example, a 10-bit RGBA image using pixel interleaving with no component padding and 32-bit (4-byte) block padding on the right will produce a repeated pattern of four 32-bits blocks describing 3 pixels [{R,G,B}, {A, R, G}, {B, A, R}, {G, B, A}]. If tile padding (resp. row) padding is also specified to be 32-bit (4-byte) then the start the next tile (resp. row) will start at the next 4-byte boundary, regardless of whether space for the next component value was available in the previous block. For example, with 4-byte tile alignment, if the width of the tile is not a multiple of 3, the last block of the line will only contain 1 or 2 component values but all 4-bytes of the last block will be present in the sample data.

EDITOR’S NOTE: The group is investigating support for component block alignment (eg "14 bits aligned on 2 bytes, 17 bits aligned on 3 bytes” within a single 5 bytes block), as well as pixel-alignment, and welcomes NB comments and contributions on that topic.

Rows of tiles shall be byte-aligned at the end of the row:

- if the block size is 0, this means that padding bits until byte alignment may be present after the last component value of the last pixel of each row,

- Otherwise, this means that the last block (containing the last component value of the last pixel of each row) may contain more padding bits than previous blocks.

If row_align_size is 0, no padding is present at the end of rows of tiles. Otherwise, let N be the number of bytes required to contain:

- all values of all components for the current row if interleave_type is 1 or 5 or if interleave_type is 2 and component type is ‘U’ or ‘V’,

- all values of the current component for the current row otherwise.

row_align_size shall be equal to or greater than N, and (N - row_align_size) bytes of padding are present after the last byte of the row.

If tile_align_size is 0, no padding bits are present at the end of tiles. Otherwise, let N be the number of bytes required to contain:

- all values of the current component for all rows of the current tile if interleave_type is 4,

- all values of all components for all rows of the current tile otherwise.

tile_align_size shall be equal to or greater than N, and (N - tile_align_size) bytes of padding are present after the last byte of the last row of the tile. This value shall be 0 if a single tile is used.

#### Syntax

aligned(8) class UncompressedVideoConfigBox extends FullBox('uncC', 0, 0) {
	unsigned int(32) profile;	
	unsigned int(16) component_count;	
	{
		unsigned int(16) component_type;
		if (component_type==0xFFFF) {
			utf8string component_type_uri;
		}
		unsigned int(8) component_bit_depth_minus_one;
		unsigned int(8) component_format;
	} [component_count];
	unsigned int(8) sampling_type;
	unsigned int(8) interleave_type;
	unsigned int(8) block_size;
	bit(1) block_pad_last;
	bit(1) block_little_endian;
	bit(6) reserved = 0;
	unsigned int(32) row_align_size;
	unsigned int(32) tile_align_size;
	unsigned int(32) num_tile_cols_minus_one;
	unsigned int(32) num_tile_rows_minus_one;
}

#### Semantics

profile indicates the predefined configuration for this uncompressed sample entry, as defined in Table 6. Value 0 indicates this sample entry may or may not be compliant to a profile. If not 0, all remaining fields in the box shall have the values indicated In Table 6 for this profile. The four-character code 'gene' is reserved and shall be interpreted as a value of 0.

component_count indicates the number of components present in the samples referencing this sample entry

component_type indicates the type of the component, as defined in Table 1

component_type_uri indicates a URI describing the user-defined component type

component_bit_depth_minus_one indicates the size in bits minus one of the values for the component

component_format indicates the format of the component, as defined in Table 2

sampling_type indicates the sampling mode as defined in Table 3

interleave_type indicates the interleaving type of the components, as defined in Table 4

block_pad_last indicates padding bits location, as described in subclause 5.2.2.1.7

block_little_endian indicates block endianness, as described in subclause 5.2.2.1.7

block_size indicates the size in bytes of blocks, as described in subclause 5.2.2.1.7

row_align_size indicates the number of bytes between rows, as described in subclause 5.2.2.1.7

tile_align_size indicates the number of bytes between tiles, as described in subclause 5.2.2.1.7

num_tile_cols_minus_one plus one indicates the horizontal number of tiles in the frame

num_tile_rows_minus_one plus one indicates the vertical number of tiles in the frame

## Profiles for uncompressed video configurations

### Overview

The four-character codes defined in this subclause identify common video formats for which a profile is defined. This profile may be indicated in the UncompressedVideoConfigBox to simplify pixel format detection by file processors.

Profiles only provide identification of pixel format used in frame. Image reconstruction might need additional information, such as colour information or chroma location, present in child boxes of the sample entry.

NOTE	These profiles are introduced to document existing practices in the industry prior to the definition of this specification.

### Predefined configurations

In Table 6 below, the values for the UncompressedVideoConfigBox are listed as:

{profile, [{component_type, component_bit_depth_minus_1}], sampling_type, interleave_type}

The array of {component_type, component_bit_depth_minus_1} is given in component order, and gives the value for component_count.

Unless indicated otherwise, all other fields of UncompressedVideoConfigBox have a value of 0.

Table 6 – Predefined uncompressed video formats

| Profile identifier | Description | Field values for UncompressedVideoConfigBox |
| --- | --- | --- |
| ‘2vuy’ | 8 bits YUV 422 packed Cb Y0 Cr Y1 | {‘2vuy’, 4, [{2,7},{1,7},{3,7},{1,7}], 1, 5} |
| 'yuv2' | 8 bits YUV 422 packed Y0 Cb Y1 Cr | {'yuv2', 4, [{1,7},{2,7},{1,7},{3,7}], 1, 5} |
| 'yvyu' | 8 bits YUV 422 packed Y0 Cr Y1 Cb | {'yvyu', 4, [{1,7},{3,7},{1,7},{2,7}], 1, 5} |
| 'vyuy' | 8 bits YUV 422 packed Cr Y0 Cb Y1 | {'vyuy', 4, [{3,7},{1,7},{2,7},{1,7}], 1, 5} |
| 'yuv1' | 8 bits YUV 411 packed Y0 Y1 Cb Y2 Y3 Cr | {'yuv1', 6, [{1,7},{1,7},{2,7},{1,7},{1,7},{3,7}], 3, 5} |
| 'v308' | 8 bits YUV 444 packed Cr Y Cb | {'v308', 3, [{3,7},{1,7},{2,7}], 0, 1} |
| 'v408' | 8 bits YUVA 444 packed Cb Y Cr A | {'v408', 4, [{2,7},{1,7},{3,7},{7, 7}], 0, 1} |
| 'y210' | 10 bits YUV 422 packed LE Y0 Cb Y1 Cr | {'y210', 4, [{1,9},{2,9},{1,9},{3,9}], 1, 5, 2} block_size shall be 2. block_little_endian shall be 1. block_pad_last shall be 1. |
| 'v410' | 10 bits YUV 444 packed CbYCr, 2 unused bits at LSB byte 0 | {'v410', 3, [{2,9},{1,9},{3,9}], 0, 1} block_size shall be 4. block_little_endian shall be 1. block_pad_last shall be 1. |
| 'v210' | YUV 422 10 bits packed CbYCr | {‘v210’, 4, [{2,9},{1,9},{3,9},{1,9}], 1, 5} block_size shall be 4. block_little_endian shall be 1. block_pad_last shall be 0. Image width shall be a multiple of 48. row_align_size shall be 0 or a multiple of 128. |
| 'rgb3' | RGB 24 bits packed | {'rgb3', 3, [{4,7},{5,7},{6,7}], 0, 1} |
| 'i420' | YUV 420 8 bits planar YCbCr | {'i420', 3, [{1,7},{2,7},{3,7}], 2, 0} |
| 'nv12' | YUV 420 8 bits semi-planar YCbCr | {'nv12', 3, [{1,7},{2,7},{3,7}], 2, 2} |
| 'nv21' | YUV 420 8 bits semi-planar YCrCb | {'nv21', 3, [{1,7},{3,7},{2,7}], 2, 2} |
| 'rgba' | RGBA 32bits packed | {'rgba', 4, [{4,7},{5,7},{6,7},{7,7}], 0, 1} |
| 'abgr' | RGBA 32bits packed | {'abgr', 4, [ {7,7}, {6,7},{5,7},{4,7}], 0, 1} |

EDITOR’S NOTE: decide which one we keep, add other configurations, finalize profile names. NB comments on the topic are welcome.

## MIME type sub-parameters

When the ‘codecs’ parameter of a MIME type is used, as defined in RFC 6381, its value shall be formatted, using field values of the UncompressedVideoConfigBox, as follows (each element in the value is separated from the previous one using a dot ‘.’):

- The first element of the value shall be set to ‘uncv’

- If the profile field is not 0, an element equal to the profile four-character code string (case sensitive) is added

- Otherwise, the string ‘gene’ (case sensitive) is added

- Optionally if profile field is not 0, mandatory otherwise:

  - an element equal to the sampling_type expressed in hexadecimal is added

  - an element equal to the interleave_type expressed in hexadecimal is added

  - an element equal to the block_size expressed in hexadecimal is added

  - an element equal to ‘cTr’ is added, with c the number of tile columns in hexadecimal and r the number of tile rows in hexadecimal

  - for each component, an element equal to ‘aLb’ is added, with a the hexadecimal value of component_type and b the hexadecimal value of component_bit_depth

Hexadecimal value shall not be prefixed with ‘0x’, and leading zeros should be omitted.

Examples:

- For an uncompressed video with profile yvyu: codecs=uncv.yvyu

- For an uncompressed video with monochrome and alpha, same sampling (sampling_type=0), component interleaving (interleave_type=0) with each value stored on 10 bits with 2 bytes blocks little-endian left-padded: codecs=uncv.gene.0.0.2.1T1.0LA.7LA

- For an uncompressed video with 2 bits alpha, 10 bits R, B, G, same sampling (sampling_type=0), pixel interleaving (interleave_type=1) with no specific block alignment (block_size=0) and a tiling of 4 horizontal for 3 vertical tiles: codecs=uncv.gene.0.1.0.4T3.7L2.4LA.5LA.6LA

# Video sample entry extensions

### Overview

The boxes defined in this subclause can be used to provide further information on one or more components present in the video format. Presence of these boxes may be mandated by the video format used.

Each of the defined boxes can be added to a video sample entry for media tracks or associated with an image item.

The syntaxes in this section are given for a video sample entry container and the defined boxes therefore extend Box (resp. FullBox). When used in an ItemPropertyContainerBox, the same syntax applies but the defined boxes extend ItemProperty (resp. ItemFullProperty).

NOTE Some of the boxes defined in this subclause may apply to both uncompressed video formats or compressed video formats. Derive specifications may allow usage of these boxes in for compressed formats.

### Palette configuration

#### Definition

Box Type:	'upal' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The UncompressedPaletteBox allows describing image data coded through a palette. This box shall be present if, in the associated UncompressedVideoConfigBox, there is a component defined with a component_type value of 10 (‘P’).

The component value of each pixel gives the index in the palette, and shall be an integer between 0 and values_count-1, with value 0 indicating the first entry in the list of pixel values of the palette.

#### Syntax

aligned(8) class UncompressedPaletteBox extends FullBox('upal', 0, 0) {
	unsigned int(16) palette_components_count;	
	{
		unsigned int(16) component_type;
		if (component_type==0xFFFF) {
			utf8string component_type_uri;
		}
		unsigned int(8) component_bit_depth_minus_one;
		unsigned int(8) component_format;
	} [palette_components_count];

	unsigned int(32) values_count;
	for (i=0; i<values_count; i++) {
		for (j=0; j<palette_components_count; j++) {
			bit(X) component_value;
			aligned(8) bit(0) unused_bits_zero;
		}
	}
}

#### Semantics

palette_components_count indicates the number of components described by the palette

component_type indicates the type of the Nth component, as defined in Table 1. This value shall be different from 11 (‘FA’) and from any component type listed in the associated UncompressedVideoConfigBox.

component_type_uri indicates a URI describing the user-defined component type

component_bit_depth_minus_one indicates the size in bits minus one of values of the Nth component

component_format indicates the format of the Nth component, as defined in Table 2

values_count indicates the number of pixel values following

component_value indicates the value of the Nth component for the given entry. This field is coded on X bits with X = component_bit_depth_minus_one + 1 of the Nth component

unused_bits_zero represents the number of unused bits following the component value to achieve byte alignment. These unused bits shall be ignored by the file reader and should be set to 0.

### Pattern Definition

#### Definition

Box Type:	'upat' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The UncompressedPatternDefinitionBox allows describing filter array patterns such as Bayer. This box shall be present if, in the associated UncompressedVideoConfigBox, there is a component defined with a component_type value of 11 (‘FA’).

The pattern is used to assign the final component type to each value. The pattern is defined at the top-left pixel of the image, and is repeated to cover the entire image. For a pixel position {x, y} in the image, with {0,0} being the top-left pixel, the coded value for that position represents the component type given by position {width%pattern_width, height%pattern_height} in the UncompressedPatternDefinitionBox, and other component values for that pixel, as defined in the UncompressedPatternDefinitionBox, are not present.

The tile width (resp. height) shall be a multiple of pattern_width (resp. pattern_height).

Examples:

- a BGGR Bayer image will be represented by a single component 11 (‘FA’) and a pattern with 2 rows, 2 columns indicating components [6,5,5,4].

- a GRBG Bayer image will be represented by a single component 11 (‘FA’) and a pattern with 2 rows, 2 columns indicating components [5,4,6,5].

- a BGGR Bayer image with an alpha plane following the Bayer data will be represented by two components 11 (‘FA’) and 7 (‘A’), an interleave_type of 0 and a pattern with 2 rows, 2 columns indicating components [6,5,5,4].

- a Red-White-Green-Blue sensor image will be represented by a single component 11 (“FA”) and a pattern with 4 rows, 4 columns indicating the R, G, B and monochrome components order.

#### Syntax

aligned(8) class UncompressedPatternDefinitionBox extends FullBox('upat', 0, 0) {
	unsigned int(16) pattern_width;
	unsigned int(16) pattern_height;
	for (i=0; i< pattern_height; i++) {
		for (j=0; j< pattern_width; j++) {
			unsigned int(16) component_type;
			if (component_type==0xFFFF) {
				utf8string component_type_uri;
			}
			double(32) component_gain;
		}
	}
}

#### Semantics

pattern_width indicates the width in pixels of the pattern

pattern_height indicates the height in pixels of the pattern

component_type indicates the type of the Nth component, as defined in Table 1. This value shall be different from 10 (‘P’) and from any component type listed in the associated UncompressedVideoConfigBox.

component_type_uri indicates a URI describing the user-defined component type

component_gain indicates a gain to be applied to the specific component, such as for white balance correction with Bayer imagery, etc. The value shall be coded as an IEEE 754 “binary32”.

### Reference Level

#### Definition

Box Type:	'ulev' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The UncompressedReferenceLevelBox allows describing the minimum and maximum values for components present in the image data. When this box is absent or a component type is not listed in this box, reference white and black values for this component type are derived from the ColourInformationBox of the sample entry. If the ColourInformationBox is absent, the levels for the desired component type are derived as follows:

- For components of type Y, U or V with 8 bits depth, reference black is 16 and reference white is 235

- For components of type Y, U or V with 10 bits depth, reference black is 64 and reference white is for 940

- Otherwise, reference black is 0 and reference white is maximum value for component bit depth.

When UncompressedReferenceLevelBox and ColourInformationBox are both present and document reference levels for the same component types, information from the ColourInformationBox shall be used.

NOTE If the ColourInformationBox is present with unspecified matrix_coefficients and has full_range_flag set to 1, full range is assumed.

Reference levels shall be ignored for non-integer component types.

There shall be at least as many components of a given type A in the associated UncompressedVideoConfigBox as there are entries for that type A in the UncompressedReferenceLevelBox; the first reference level for component type A corresponds to the first component with type A in the associated UncompressedReferenceLevelBox., the second reference level for component type A corresponds to the second component with type A and so on.

If clip_range is set to 1, black_level (resp. white_level) indicates the minimum (resp. maximum value) for the component, and any value less than black_level (resp. greater than white_level) shall be set to black_level (resp. white_level).

If clip_range is set to 0, the value N coded on k bits results in a component value of black_level+N*(white_level-black_level)/(2k-1).

#### Syntax

aligned(8) class UncompressedReferenceLevelBox extends FullBox('ulev', 0, 0) {
	unsigned int(16) level_count;

{
		unsigned int(16) component_type;
		if (component_type==0xFFFF) {
			utf8string component_type_uri;
		}
		unsigned int(1) clip_range;
		bits(7) reserved = 0;
		signed int(32) black_level;	
		signed int(32) white_level;	
	} [level_count];
}

#### Semantics

level_count indicates the number of components for which levels are described

component_type indicates the type of the Nth component, as defined in Table 1

component_type_uri indicates a URI describing the user-defined component type

clip_range indicates if the levels indicate a clip range or a final range

black_level indicates the black level for the component

white_level indicates the white level for the component; this value shall be greater than the black_level value.

### Polarization Pattern Definition

#### Definition

Box Type:	'plzp' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The PolarizationPatternDefinitionBox allows describing a filter array pattern for sensors that have polarization patterns implemented on the image sensor. The pattern dimension is not required to match the pattern dimension given in the UncompressedPatternDefinitionBox.

The pattern is used to assign polarization values at the pixel level. The pattern is defined at the top-left pixel of the image, and is repeated to cover the entire image. For a pixel position {x, y} in the image, with {0,0} being the top-left pixel, the polarization value for each pixel is given by position {width%pattern_width, height%pattern_height} in the PolarizationPatternDefinitionBox.

The tile width (resp. height) shall be a multiple of pattern_width (resp. pattern_height).

Figure 10 illustrates an RGGB Bayer filter array pattern superimposed on a 4x4 pixel grid (filter array pattern with values of ‘FA1’ through ‘FA16’) with a 2x2 polarization pattern with angles of 90, 45, 135, and 0 degrees.

Figure 16 Polarization example with a filter array pattern component

#### Syntax

aligned(8) class PolarizationPatternDefinitionBox extends FullBox('plzp', 0, 0) {
	unsigned int(16) pattern_width;
	unsigned int(16) pattern_height;
	for (i=0; i< pattern_height; i++) {
		for (j=0; j< pattern_width; j++) {
			double(32) polarization_angle;
		}
	}
}

#### Semantics

pattern_width indicates the width in pixels of the pattern

pattern_height indicates the height in pixels of the pattern

polarization_angle indicates the polarization angle of the pixel at the defined pattern location. Value is coded as an IEEE 754 “binary32” number, and shall be either ‘NaN’, indicating that no polarization filter is present, or a value greater than or equal to 0.0 and strictly less than 360.0.

### Sensor Non-Uniformity Correction

#### Definition

Box Type:	'snuc' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The SensorNonUniformityCorrectionBox allows describing pixel specific gain and offset corrections, as well as a global gain term.

The Non-Uniform Correction (NUC) nuc_gain and nuc_offset attributes provide non-uniformity corrections for a sensor. This is commonly used with radiometric imagery. The correction equation is linear: 𝑦 = nuc_gain *𝑥+ nuc_offset, where x is an input component value as stored in the file. The minimum value of y saturates at a value of 0. In situations where the same component value range is to be maintained after application of the NUCs, the maximum value of y saturates at a level defined by the number of bits in the component value, for instance, 65535 for a 16-bit component.

The NUC coefficients are assigned per pixel through the full image. The NUC gain and offset tables are aligned with the entire image, resulting in a table width equal to image_width and table height equal to image_height. For a pixel position {x, y} in the image, with {0,0} being the top-left pixel, the NUC coefficients for each pixel are given by position {x,y} in the SensorNonUniformityCorrectionBox.

#### Syntax

aligned(8) class SensorNonUniformityCorrectionBox extends FullBox('snuc', 0, 0) {
	unsigned int(32) image_width;
	unsigned int(32) image_height;
	for (i=0; i< image_height; i++) {
		for (j=0; j< image_width; j++) {
			double(32) nuc_gain;
		}
	}
   for (i=0; i< image_height; i++) {
		for (j=0; j< image_width; j++) {
			double(32) nuc_offset;
			}
		}
	}

}

#### Semantics

image_width indicates the width in pixels of the NUC tables, shall be equal to image width

image_height indicates the height in pixels of the NUC tables, shall be equal to image height

nuc_gain specifies the Non-Uniform Correction (NUC) gain. The value shall be coded as an IEEE 754 “binary32”

nuc_offset specifies the Non-Uniform Correction (NUC) offset. The value shall be coded as an IEEE 754 “binary32”

### Sensor Bad Pixels Map

#### Definition

Box Type:	'sbpm'
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The SensorBadPixelsMapBox allows identifying bad pixels on a sensor, i.e. pixels for which at least one component value is corrupted.

#### Syntax

aligned(8) class SensorBadPixelsMapBox extends FullBox('sbpm', 0, 0) {
	unsigned int(32) num_bad_rows;

unsigned int(32) num_bad_cols;

unsigned int(32) num_bad_pixels;

for (i=0; i< num_bad_rows; i++) {

unsigned int(32) bad_row;

}

for (i=0; i< num_bad_cols; i++) {

unsigned int(32) bad_column;

}
	for (i=0; i< num_bad_pixels; i++) {
		unsigned int(32) bad_pixel_row;

unsigned int(32) bad_pixel_column;

}
}

#### Semantics

num_bad_rows indicates the number of full rows in the image that are bad.

num_bad_columns indicates the number of full columns in the image that are bad.

num_bad_pixels indicates the number of individual bad pixels. This does not include bad pixels that are identified as being part of an entire bad row or bad column.

bad_row a row number for which all pixels are bad. Value 0 corresponds to the top row. The value shall be less than image height minus 1.

bad_column a column number for which all pixels are bad. Value 0 corresponds to the left column. The value shall be less than image width minus 1.

row the row index of the coordinate pair identifying the location of a bad pixel. Value 0 corresponds to the top row. The value shall be less than image height minus 1.

column the column index of a coordinate pair identifying the location of a bad pixel. Value 0 corresponds to the left column. The value shall be less than image width minus 1.

### Chroma Location

#### Definition

Box Type:	'cloc' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or per item

The ChromaLocationBox may be used to describe the chroma subsampling location method used for uncompressed video using a sampling_type other than 0 and ‘Y’, ‘U’, and ‘V’ components. It shall not be present for other uncompressed video configurations.

For an uncompressed video using YUV 4:2:2 or YUV 4:1:1 subsampling, the indicated Chroma420SampleLocType as defined in ISO/IEC 23091-2 shall have VerticalOffsetC equal to 0.

If ChromaLocationBox is not present for an uncompressed video using a sampling_type other than 0 and ‘Y’, ‘U’, and ‘V’ components, the associated Chroma420SampleLocType is 0.

If ChromaLocationBox is used as an item property, it shall be marked as essential.

#### Syntax

aligned(8) class ChromaLocationBox extends FullBox('cloc', 0, 0) {
	unsigned int(8) chroma_location;
}

#### Semantics

chroma_location indicates a Chroma420SampleLocType value as defined in ISO/IEC 23091-2, or the value 6. Value 6 is defined, following Chroma420SampleLocType semantics as defined in ISO/IEC 23091-2, with: VerticalOffsetC=0, HorizontalOffsetC=0 for Cr and HorizontalOffsetC=1 for Cb (ex: DV PAL).

### Frame Packing Information

#### Definition

Box Type:	'pack' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or per item

The FramePackingInformationBox can be used to describe how two pictures are packed into an uncompressed video frame for stereoscopic imagery.

If not present, each uncompressed video frame consists in a single picture.

If FramePackingInformationBox is used as an item property, it shall be marked as essential.

#### Syntax

class FramePackingInfoBox extends FullBox('pack', 0, 0) {
	unsigned int(4) video_frame_packing;
	unsigned int(4) PackedContentInterpretationType
	unsigned int(1) QuincunxSamplingFlag;
	unsigned int(7) reserved = 0;
}

#### Semantics

video_frame_packing, QuincunxSamplingFlag, PackedContentInterpretationType have the same semantics as defined in ISO/IEC 23091-2.

### Disparity Information

#### Definition

Box Type:	'disi' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or per item

The DisparityInformationBox can be used to describe how values of a disparity map should be interpreted. If not present, the mapping from disparity value to distance in meters uses the default values (parallax_zero=2N-1, parallax_scale=256, dref=300, wref=100) as defined in ISO/IEC 23002-3.

If this box is used as an item property, it shall be marked as essential.

#### Syntax

class DisparityInformationBox extends FullBox('disi', 0, 0) {
	unsigned int(16) parallax_zero;
	unsigned int(16) parallax_scale;
	unsigned int(16) dref;
	unsigned int(16) wref;
}

#### Semantics

parallax_zero, parallax_zero, parallax_zero, parallax_zero: have the same semantics as defined in ISO/IEC 23002-3.

### Depth Mapping Information

#### Definition

Box Type:	'depi' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or per item

The DepthMappingInformationBox may be used to describe how values in a depth map are transformed into distance values. If not present, the mapping from depth value to distance values follow the default values (nknear=128, nkfar=128) as defined in ISO/IEC 23002-3.

If this box is used as an item property, it shall be marked as essential.

#### Syntax

class DepthInfoBox extends FullBox('depi', 0, 0) {
	unsigned int(8) nknear;
	unsigned int(8) nkfar;
}

#### Semantics

nknear, nkfar near and far distances have the same semantics as defined in ISO/IEC 23002-3.

# Sample group descriptions

## Field Interlace Type

### Definition

The FieldInterlaceType sample group description allows describing the field layout of a sample in case of interlaced video content. This sample group description is identified by the 'ilce' grouping_type.

If no FieldInterlaceType sample group description is associated to a sample, whether because the sample group description is not present or because the sample is not mapped to any sample group description of this type, the sample data is assumed to be a progressive frame.

### Syntax

aligned(8) class FieldInterlaceDescription {
	unsigned int(2) interlace_type;
	unsigned int(1) interleaved;
	unsigned int(1) first_field_type;
	unsigned int(1) bottom_first;
	bits(3) reserved=0;
}

class FieldInterlaceType extends VisualSampleGroupEntry ('ilce') {
	FieldInterlaceDescription interlace_description;
}

### Semantics

interlace_type indicates the interlacing type. The following values are defined:

- 0: progressive frame

- 1: the sample data contains both top and bottom fields. The duration of each field is half the duration of the associated sample.

- 2: the sample data contains a single field. The video sample entry height shall be twice the number of lines in the field. The duration of the field is the duration of the associated sample.

- 3: the sample data contains both top and bottom fields, but the fields are co-incident in time (sample is a progressive segmented frame).

interleaved if set to 1, indicates that lines of each field are interleaved in the sample data, i.e. first line of first field followed by first line of second field followed by second line of first field etc.). Otherwise (interleaved set to 0), the first line of the second field follows the last line of the first field in the sample data. This value shall be 0 for interlace_type values other than 1 and 3.

first_field_type if set to 1, indicates that the first field appearing in the sample data is the bottom field. If a single field is present in the sample data, then this field is the bottom field. This value shall be 0 if interlace_type value is 0.

bottom_first if set to 1, indicates that the first field to present is the bottom field. This value shall be 0 if interlace_type value is not 1.

EDITOR’s NOTE: We could also use the reserved bits to indicate source origin/copy types for content resulting from telecine (2:2, 3:2 pulldown) if needed. NB Comments and contributions on the topic are welcome.

# Image Item properties

## Field Interlace Property

### Definition

Box Type:	'ilcp' 
Container:	ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per item

The FieldInterlaceProperty allows describing the field layout of an image item in case of interlaced video content. In not present, the image item is assumed to be a progressive frame. If present, the property shall be marked as essential.

### Syntax

class FieldInterlaceProperty extends ItemProperty ('ilce') {
	FieldInterlaceDescription interlace_description;
}

### Semantics

The semantics are identical to the FieldInterlaceType sample group description semantics.

# Multiple track storage

## Overview

Multiple track storage allows describing uncompressed video streams whose components are in multiple tracks. This process may be used in various cases:

- simplify editing, by modifying only one component (e.g. alpha or depth) without modifying the other components (colour)

- provide players a way to play a subpart of the components (e.g. colour) instead of failing playback due to unsupported configuration

- enable efficient disk and network usage for partial access (spatial, temporal) of a subset of the components. A typical use case is to store all or a subset of samples for first component followed by samples of second component etc. by using ISOBMFF sample data layout tools.

- use components with different spatial resolutions

## Component video track group

Video tracks containing components related to the same media source may be grouped using a TrackGroupBox with the track_group_type value of 'scvg'. The pair of track_group_id and track_group_type identifies a track group within the file. The tracks that contain a particular TrackGroupTypeBox having the same value of track_group_id and track_group_type belong to the same track group.

Tracks belonging to such a group may have different resolutions, but shall have:

- The same aspect ratio,

- The same temporal layout, i.e. there shall not be any presentation time for which no sample is defined for one or more tracks of the group while a sample is defined for other tracks of the group.

NOTE The component video track group is mainly intended for storage of uncompressed video with components in different tracks, but can be used for a mix of compressed and uncompressed video (e.g. colour compressed, alpha / depth uncompressed). Derived specification may further restrict the kind of components allowed in 'scvg' track groups, the timescale used, the sample entry format, the visual dimension of the tracks, etc. The display or processing of (part of) a track group with track_group_type value of 'scvg' is application specific.

EDITOR’S NOTE: we don’t have any proposal for multiple item storage, maybe Entity Groups could be used if feature is needed. Contributions on the topic are welcome.

## Image tiling using ISOBMFF tracks and items

Source image data may be spatially split in different rectangular images, or tiles, each resulting tile being stored in its own track or image item rather than storing the entire image data as a single track or item.

A typical use case is to allow a temporal interleaving of tiles different from what can be achieved with tiling as defined in 5.2.2.1.4, for which all tiles of a frame have to be in the same sample.

Video tracks containing tiles of the same media source may be grouped using a TrackGroupBox with the track_group_type value of 'svtg'.

The pair of track_group_id and track_group_type identifies a track group within the file. The tracks that contain a particular TrackGroupTypeBox having the same value of track_group_id and track_group_type belong to the same track group, i.e. they contain different tiles of the same source image. Tile layouts must be indicated using the width, height and matrix of the track header. Tracks belonging to the same track group with type 'svtg' shall not spatially overlap each other.

For image items, tile layouts can be indicated using the grid derived image item.

NOTE Derived specification may further restrict the combination of tiles within the uncompressed video tracks and multi-track tiling.