# ISO/IEC JTC 1/SC 29/WG 03 N0847

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	WD of ISO/IEC 14496-22 5th edition Open Font Format

Status:	Approved

Date of document:	2023-04-28

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	935 (with cover page)

Email of Convenor:	young.L @ samsung . com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N0847

April 2023, Antalya, TR

| Title | WD of ISO/IEC 14496-22 5th edition Open Font Format |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 22589 |

ISO/IEC 14496-22:202x(5th edition)_WD

ISO JTC1/SC 29/WG 3

Secretariat: XXXX

Information technology — Coding of audio-visual objects —

Part 22: Open Font Format

WD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO 2022

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents	Page

Foreword	ix

Introduction	x

1	Scope	1

2	Normative references	1

3	Terms, definitions and abbreviated terms	2

3.1	Terms and definitions	2

3.2	Abbreviated terms	2

4	The Open Font file format	4

4.1	Description	4

4.2	Filenames	4

4.3	Data types	4

4.4	Table version numbers	6

4.5	Top-level OFF organization	7

4.5.1	Table directory	7

4.5.2	Table records	8

4.5.3	Calculating checksums	9

4.6	Font collections	10

4.6.1	The Font Collection overview	10

4.6.2	The Font Collection file structure	10

4.6.3	TTC header	11

5	Open font tables	12

5.1	Required common tables	12

5.1.1	List of required tables	12

5.1.2	cmap—Character to glyph index mapping table	13

5.1.3	head—Font header	30

5.1.4	hhea—Horizontal header	33

5.1.5	hmtx—Horizontal metrics	35

5.1.6	maxp—Maximum profile	37

5.1.7	name—Naming table	38

5.1.8	OS/2 — OS/2 and Windows metrics table	62

5.1.9	post—PostScript	93

5.2	Tables related to TrueType outlines	96

5.2.1	List of TrueType outlines tables	96

5.2.2	cvt—Control value table	97

5.2.3	fpgm—Font program	97

5.2.4	glyf—Glyf data	97

5.2.5	loca—Index to location	106

5.2.6	prep—Control value program	107

5.2.7	gasp—Grid-fitting and scan-conversion procedure table	107

5.3	Tables related to CFF outlines	110

5.3.1	List of CFF outline tables	110

5.3.2	CFF—Compact Font Format table (version 1)	111

5.3.3	CFF2—Compact Font Format (version 2) table	111

5.3.4	VORG—Vertical origin table	157

5.4	Table for SVG glyph outlines	158

5.4.1	SVG—The SVG (Scalable Vector Graphics) table	158

5.4.2	SVG Documents	160

5.4.3	Color and color palettes	162

5.4.4	Glyph Identifiers	164

5.4.5	Glyph semantics and text layout processing	164

5.4.6	Animations	166

5.4.7	SVG glyph examples	166

5.5	Tables related to bitmap glyphs	173

5.5.1	List of bitmap glyph tables	173

5.5.2	EBDT—Embedded bitmap data table	173

5.5.3	EBLC—Embedded bitmap location table	177

5.5.4	EBSC—Embedded bitmap scaling table	185

5.5.5	CBDT—Color bitmap data table	186

5.5.6	CBLC—Color bitmap location table	189

5.5.7	sbix—Standard bitmap graphics table	190

5.6	Optional tables	193

5.6.1	DSIG—Digital signature table	193

5.6.2	hdmx—Horizontal device metrics	196

5.6.3	kern—Kerning	197

5.6.4	LTSH—Linear threshold	200

5.6.5	MERG—Merge table	201

5.6.6	meta—Metadata table	206

5.6.7	PCLT—PCL 5 table	210

5.6.8	VDMX—Vertical device metrics	218

5.6.9	vhea—Vertical header table	221

5.6.10	vmtx—Vertical metric table	226

5.6.11	COLR—Color Table	228

5.6.12	CPAL—Palette Table	297

6	Advanced Open Font layout tables	303

6.1	Advanced Open Font layout extensions	303

6.1.1	Overview of advanced typographic layout extensions	303

6.1.2	TrueType versus OFF layout	304

6.1.3	OFF layout terminology	305

6.1.4	Text processing with OFF layout	308

6.1.5	OFF layout and Font variations	309

6.2	OFF layout common table formats	310

6.2.1	Overview	310

6.2.2	OFF layout and Font variations	311

6.2.3	Table organization	312

6.2.4	Scripts and languages	313

6.2.5	Features and lookups	316

6.2.6	Coverage table	321

6.2.7	Class definition table	323

6.2.8	Device and VariationIndex tables	324

6.2.9	Feature variations	327

6.2.10	Common table examples	331

6.3	Advanced typographic tables	340

6.3.1	BASE—Baseline table	340

6.3.2	GDEF—The glyph definition table	364

6.3.3	GPOS—The glyph positioning table	380

6.3.4	GSUB—The glyph substitution table	446

6.3.5	JSTF—The justification table	489

6.3.6	MATH—The mathematical typesetting table	501

6.4	Layout tag registry	522

6.4.1	Scripts tags	522

6.4.2	Language tags	529

6.4.3	Feature tags	554

6.4.4	Baseline tags	634

7	OFF font variations	639

7.1	Font variations overview	639

7.1.1	General	639

7.1.2	Terminology	642

7.1.3	Variation space, default instances and adjustment deltas	644

7.1.4	Coordinate scales and normalization	648

7.1.5	Variation data	653

7.1.6	Variation data tables and miscellaneous requirements	663

7.1.7	Algorithm for interpolation of instance values	665

7.1.8	Interpolation example	669

7.1.9	Dynamic generation of static instance fonts	675

7.2	Font variations common table formats	676

7.2.1	Overview	676

7.2.2	Tuple variation store	677

7.2.3	Item variation stores	687

7.2.4	Design-variation axis tag registry	695

7.3	Font variations tables	702

7.3.1	avar—Axis variations table	702

7.3.2	cvar—CVT variations table	705

7.3.3	fvar—Font variations table	708

7.3.4	gvar—Glyph variations table	717

7.3.5	HVAR—Horizontal metrics variations table	729

7.3.6	MVAR—Metrics variations table	731

7.3.7	STAT—Style attributes table	737

7.3.8	VVAR—Vertical metrics variations table	753

8	Recommendations for OFF fonts	755

8.1	Byte ordering	755

8.2	Mixing outline formats	755

8.3	Filenames	755

8.4	Table alignment and length	756

8.5	Glyph 0: the .notdef glyph	756

8.6	'BASE' table	756

8.7	'cmap' table	757

8.8	'cvt' table	757

8.9	'fpgm' table	757

8.10	'glyf' table	757

8.11	'hdmx' table	758

8.12	'head' table	758

8.13	'hhea' table	758

8.14	'hmtx' table	758

8.15	'kern' table	758

8.16	'loca' table	759

8.17	'LTSH' table	759

8.18	'maxp' table	759

8.19	'name' table	759

8.20	'OS/2' table	762

8.21	'post' table	763

8.22	'prep' table	763

8.23	'VDMX' table	763

8.24	TrueType Collections	763

9	General recommendations	763

9.1	Optimized table ordering	763

9.2	Non-standard (Symbol) fonts	764

9.3	Baseline to baseline distances	764

9.4	Style bits	765

9.5	Drop-out control	766

9.6	Embedded bitmaps	766

9.7	OFF CJK font guidelines	766

9.8	Stroke reduction in variable fonts	767

9.9	Families with optical size variants	767

Annex A (informative)  IBM font family classifications	768

A.1	General	768

A.2	Class ID=0 No Classification	768

A.3	Class ID=1 Oldstyle Serifs	768

A.4	Class ID=2 Transitional Serifs	770

A.5	Class ID=3 Modern Serifs	770

A.6	Class ID=4 Clarendon Serifs	771

A.7	Class ID=5 Slab Serifs	772

A.8	Class ID=6 (reserved for future use)	774

A.9	Class ID=7 Freeform Serifs	774

A.10	Class ID=8 Sans Serifs	774

A.11	Class ID=9 Ornamentals	776

A.12	Class ID=10 Scripts	777

A.13	Class ID=11 (reserved for future use)	778

A.14	Class ID=12 Symbolic	778

A.15	Class ID=13 Reserved	779

A.16	Class ID=14 Reserved	779

Annex B (informative) Earlier versions of OS/2 – OS/2 and Windows metrics	780

B.1	OS/2 - OS/2 and Windows metrics (version 0)	780

B.2	OS/2 - OS/2 and Windows metrics (version 1)	800

B.3	OS/2 - OS/2 and Windows metrics (version 2)	826

B.4	OS/2 - OS/2 and Windows metrics (version 3)	849

B.5	OS/2 - OS/2 and Windows metrics (version 4)	872

Annex C (informative)  OFF Mirroring Pairs List	897

Annex D (informative)  Example CFF2 Font	904

Annex E (informative)  Registration of Media Type: application/font-sfnt	910

# Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents) or the IEC list of patent declarations received (see http://patents.iec.ch).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see www.iso.org/iso/foreword.html.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

This fifth edition cancels and replaces the fourth edition (ISO/IEC 14496-22:2019), which has been technically revised. It also incorporates the Amendments ISO/IEC 14496-22:2019/Amd.1:2020 and ISO/IEC 14496-22:2019/Amd.2:2022.

The main changes compared to the previous edition are as follows:

—	new technology clauses were added;

—	many existing clauses, subclauses, tables, figures and annexes were editorially revised.

A list of all parts in the ISO/IEC 14496 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

# Introduction

Multimedia applications require a broad range of media-related standards. In addition to the typical audio and video applications, multimedia presentations include scalable 2D graphics and text supporting all languages of the world. Faithful reproduction of scalable multimedia content requires additional components including scalable font technology. The Open Font Format is a widely-supported format for font data with a rich set of capabilities for digital typography. It is based on the OpenType® font format, which was originally developed as an extension of the TrueType® font format, using the same 'sfnt' container structure, and maintaining compatibility for fonts created following the original TrueType specification. But several additional capabilities are also supported, including the following:

- Glyph outline data can use the CFF or CFF version 2 (“CFF2”) formats, as well as the TrueType glyph format.

- Multicolour glyph presentation is supported using embedded colour bitmaps or SVG documents, or using 2D graphic compositions defined in the font using a binary format that combines outline-format glyphs with various graphics operations.

- All Unicode® characters can be supported, including supplementary-plane characters, as well as Unicode variation sequences.

- Advanced Open Font Layout tables provide the advanced typographic capabilities needed for high-quality typography as well as for international text using the wide variety of scripts supported in The Unicode Standard®.

- The mathematical typesetting table allows a font to include data required for layout of complex, math formulas.

- Open Font collection files enable multiple fonts that share common data to be housed within a single file, allowing for de-duplication of data. This is especially useful, for example, for sets of CJK (Chinese, Japanese, Korean) fonts of the same design that share most glyphs in common but that vary with locale-specific glyphs for certain characters.

- Font variations (“variable fonts”) enable glyph outlines or other font data to be variable based on one or more design-axis parameters. Whereas a collection file can contain multiple discrete, static font resources, a variable font can provide continuous variation in design along each of its axes. This can provide great flexibility for content authors and designers while also allowing the font data for an entire font family to be represented in an efficient format.

This specification is intended to be used in conjunction with other specifications.

- While various legacy character encoding standards are supported, it is primarily designed for use with The Unicode Standard, which provides the universal encoding for written characters and symbols, as well as specifications for how text in different scripts is to be represented.

- This specification defines Advanced Open Font Layout tables and low-level glyph substitution and positioning operations needed for high-quality typography and for correct display of Unicode text in various scripts. (See 6.1.1.) The OpenType Layout feature registry defines various features that represent specific typographic capabilities that may be supported in a font, and that are used to activate those capabilities in a given font. Many features expose optional capabilities that authors and typographers can choose to use at their discretion; for example, small cap forms, or kerning. But many other features are used to activate capabilities that are required for correct display of text; for example, required ligatures for Arabic script, or positioning of mark glyphs. Many scripts supported in Unicode have complex structural behaviors that require non-trivial operations, implemented in applications or in text-layout and “shaping” libraries, to derive a correct sequence of positioned glyphs for presentation of an underlying Unicode string. The feature registry will include features that can be used in those operations. However, complete specification of shaping algorithms for different scripts is beyond the scope of this specification. Such algorithms can be proprietary, “closed-source” implementations in particular applications; or they can be defined in vendor-specific specifications or in other industry specifications.

- Support for mathematical text involving complex formulas requires a content format to describe text semantics combined with layout and presentation capabilities. This specification defines the font-specific data that would be needed for presentation. In this way, the Open Font Format can be used to implement presentation capabilities for other document format specifications, such as TEX or MathML. Certain features can be defined in the Advanced Open Font Layout feature tag registry to support math layout operations.

- Text layout involves operations within individual lines of text but also control over arrangement of lines into larger blocks within a page or similar context. This specification defines certain data that is used in block-level layout, such as default line metrics (ascent, descent, leading). Also, certain features can be defined in the Advanced Open Font Layout feature tag registry to support layout of text blocks for either horizontal or vertical layout orientation. Complete specification of block level layout is beyond the scope of this specification, however. It can be used in conjunction with other specifications, such as Unicode Standard Annex #50: Unicode Vertical Text Layout.

Some font capabilities can be subject to tailoring by applications or other higher-level protocols. For example, the TrueType instructions defined in this specification define operations used in rasterization of glyph outlines, but applications can supplement their own algorithms for final rasterization (for example, using over-sampling, or control of sub-pixel display elements) to optimize legibility of glyphs. Also, while the descriptions of features in the Advanced Open Font Layout feature tag registry can specify expected usage, applications may tailor their usage of features subject to other specifications or according to their own needs.

The International Organization for Standardization (ISO) and International Electrotechnical Commission (IEC) draw attention to the fact that it is claimed that compliance with this document may involve the use of patents.

ISO and IEC take no position concerning the evidence, validity and scope of these patent rights. The holders of these patent rights have assured ISO and IEC that they are willing to negotiate licences under reasonable and non-discriminatory terms and conditions with applicants throughout the world. In this respect, the statement of the holders of these patent rights is registered with ISO and IEC.

Information may be obtained from:

Apple Inc.

1 Infinite Loop MS 3-PAT

US-Cupertino, CA 95014-2084

Tel.: +1 408 974 9453

Email: iplaw@apple.com

Microsoft Corporation

Interoperability Group 3460 157th Avenue NE

US-Redmond, WA 98052

Tel.: +1 425 882 80 80

Monotype

600 Unicorn Park Drive

US-Woburn, MA 01801

Tel.: +1 781-970-6000

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights other than those identified above. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Information technology — Coding of audio-visual objects —

Part 22:
Open Font Format

# Scope

This document specifies the Open Font Format (OFF) specification, including the TrueType and Compact Font Format (CFF) outline formats. Many references to both TrueType and PostScript exist throughout this document, as Open Font Format fonts combine the two technologies. The document defines data structures for various font tables, and provides the necessary details for developers to build a font rendering and text layout/shaping engines in compliance with this document.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 10646, Information technology — Universal Coded Character Set (UCS)

ISO/IEC 14496-18, Information technology — Coding of audio-visual objects — Part 18: Font compression and streaming

ISO/IEC 15948, Information technology — Computer graphics and image processing — Portable Network Graphics: Functional specification

IEC 61966-2-1/Amd 1:2003: Multimedia systems and equipment — Colour measurement and management — Part 2-1: Colour management — Default RGB colour space — sRGB.

TrueType Instruction Set, http://www.microsoft.com/typography/otspec/ttinst.htm

The Unicode Standard, Version 15.0.0, http://www.unicode.org/versions/Unicode15.0.0/

Scalable Vector Graphics (SVG) 1.1 (2nd edition), W3C Recommendation, 16 August 2011 http://www.w3.org/TR/SVG11/

IETF BCP 47 specification, “Tags for Identifying Languages”. https://www.rfc-editor.org/info/bcp47

# Terms, definitions and abbreviated terms

## Terms and definitions

No terms and definitions are listed in this document.

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at https://www.iso.org/obp

—	IEC Electropedia: available at http://www.electropedia.org/

## Abbreviated terms

ACF 	average character face

ANSI 	American National Standards Institute

ASCII 	American Standard Code for Information Interchange

BMP 	[Unicode] basic multilingual plane

BTBD 	baseline to baseline distance

CFF 	compact font format

CID 		character identifier

CJK 		Chinese Japanese Korean [characters, ideographs, fonts, etc.]

CJKV 	Chinese Japanese Korean and Vietnamese

CV 		control value

CVT 	control value table

DLL 	dynamically linked library

FDEF 	function definition

GID 		glyph ID

ICF 		ideographic character face

IDEF 	instruction definition

IETF 	Internet Engineering Task Force

JIS 		Japanese Industrial Standard

LTR 	left to right

NLC 	National Language Council of Japan

OFF 	open font format

OMPL 	OFF mirroring pairs list

OTF	OpenType font

PCL 	printer control language

PPM, PPEM 	pixels per em

RTL 	right to left

TTC 	TrueType collection

TTF 	TrueType font

UCS 	universal character set

UTF 	Unicode transformation format

UVS 	Unicode variation sequence

VM 		virtual memory

W3C 	world wide web consortium

# The Open Font file format

## Description

An Open Font file contains data, in table format, used for rendering of text. Portions of the data are used by applications to calculate the layout of text using the font; that is, the selection of glyphs and their placement within a line. Other data provide descriptions of glyphs as TrueType or Compact Font Format (CFF) outlines. Still other data can provide monochromatic or colour bitmaps, SVG documents or binary, 2D vector graphics compositions as alternate glyph descriptions. The font data also includes meta-information, such as name strings that can used to present the font as an available choice in a font-picker user interface. Each of these types of data is stored in one or more tables each designed for a particular purpose.

References to the Universal Coded Character Set and the Unicode Standard are used throughout this document; the users of the OFF cannot meet the requirements of this document without strict adherence to these standards.

## Filenames

OFF font files may have the extension .OTF, .TTF, .OTC or .TTC. (The extension may be upper or lower case.) The extensions .OTC and .TTC should only be used for font collection files. For additional information on filename extension conventions, see subclause 8.3.

## Data types

The following data types are used in the OFF font file. All OFF fonts use big-endian (network) byte order:

| Data Type | Description |
| --- | --- |
| uint8 | 8-bit unsigned integer. |
| int8 | 8-bit signed integer. |
| uint16 | 16-bit unsigned integer. |
| int16 | 16-bit signed integer. |
| uint24 | 24-bit unsigned integer. |
| uint32 | 32-bit unsigned integer. |
| int32 | 32-bit signed integer. |
| Fixed | 32-bit signed fixed-point number (16.16) |
| FWORD | int16 that describes a quantity in font design units. |
| UFWORD | uint16 that describes a quantity in font design units. |
| F2DOT14 | 16-bit signed fixed number with the low 14 bits of fraction (2.14). |
| LONGDATETIME | Date and time represented in number of seconds since 12:00 midnight, January 1, 1904, UTC. The value is represented as a signed 64-bit integer. |
| Tag | Array of four uint8s (length = 32 bits) used to identify a table, design-variation axis, script, language system, feature, or baseline. |
| Offset8 | 8-bit offset to a table, same as uint8, NULL offset = 0x00 |
| Offset16 | Short offset to a table, same as uint16, NULL offset = 0x0000 |
| Offset24 | 24-bit offset to a table, same as uint24, NULL offset = 0x000000 |
| Offset32 | Long offset to a table, same as uint32, NULL offset = 0x00000000 |
| Version16Dot16 | Packed 32-bit value with major and minor version numbers. (See 4.4.) |

The F2DOT14 format consists of a signed, 2's complement integer and an unsigned fraction. To compute the actual value, take the integer and add the fraction. Examples of 2.14 values are:

| Decimal Value | Hex Value | Integer | Fraction |
| --- | --- | --- | --- |
| 1.999939 | 0x7fff | 1 | 16383/16384 |
| 1.75 | 0x7000 | 1 | 12288/16384 |
| 0.000061 | 0x0001 | 0 | 1/16384 |
| 0.0 | 0x0000 | 0 | 0/16384 |
| -0.000061 | 0xffff | -1 | 16383/16384 |
| -2.0 | 0x8000 | -2 | 0/16384 |

A Tag value is a uint8 array. Each byte within the array shall have a value in the range 0x20 to 0x7E. This corresponds to the range of values of Unicode Basic Latin characters in UTF-8 encoding, which is the same as the printable ASCII characters. As a result, a Tag value can be re-interpreted as a four-character sequence, which is conventionally how they are referred to. Formally, however, the value is a byte array.

When re-interpreted as characters, the Tag value is case sensitive. It shall have one to four non-space characters, padded with trailing spaces (byte value 0x20). A space character shall not be followed by a non-space character.

Within this specification, many structures are defined in terms of the data types listed above. Structures are characterized as either records or tables. The distinction between records and tables is based on these general criteria:

- Tables are referenced by offsets. If a table contains an offset to a sub-structure, the offset is normally from the start of that table.

- Records occur sequentially within a parent structure, either within a sequence of table fields or within an array of records of a given type. If a record contains an offset to a sub-structure, that structure is logically a subtable of the record’s parent table and the offset is normally from the start of the parent table.

In some cases, fields for subtable offsets are documented as permitting NULL values when the given subtable is optional. For example, in the ‘BASE’ table header, the horizAxisOffset and vertAxisOffset fields may be NULL, meaning that either subtable (or both) is optional. A NULL subtable offset always indicates that the given subtable is not present. Applications shall never interpret a NULL offset value as the offset to subtable data. For cases in which subtable offset fields are not documented as permitting NULL values, font compilers shall include a subtable of the indicated format, even if it is a header stub without further data (for example, a coverage table with no glyph IDs). Applications parsing font data should, however, anticipate non-conformant font data that has a NULL subtable offset where only a non-NULL value is expected.

## Table version numbers

Most tables have version numbers, and the version number for the entire font is contained in the table directory. Note that there are five different table version number types, each with its own numbering scheme.

- A single uint16 field. This is used in a number of tables, usually with versions starting at zero (0).

- Separate, uint16 major and minor version fields. This is used in a number of tables, usually with versions starting at 1.0.

- A uint32 field with enumerated values.

- A uint32 field with a numeric value. This is used only in the 'DSIG' and 'meta' tables.

- A Version16Dot16 field for major/minor version numbers. This is used only in the 'maxp', 'post' and 'vhea' tables.

The Version16Dot16 type is used for the version field of certain tables, and only for reasons of backward compatibility. (In earlier editions, these fields were documented as using a Fixed value, but had minor version numbers that did not follow the definition of the Fixed type.) Version16Dot16 is a packed value: the upper 16 bits comprise a major version number, and the lower 16 bits, a minor version. Non-zero minor version numbers are represented using digits 0 to 9 in the highest-order nibbles of the lower 16 bits. For example, the version field of 'maxp' table version 0.5 is 0x00005000, and that of 'vhea' table version 1.1 is 0x00011000. This type is used only in the 'maxp', 'post' and 'vhea' tables, and will not be used for any other tables in the future.

The table directory uses a uint32 field, sfntVersion, with an enumeration of defined values, some of which represent four-character tags; see subclause 4.5 (Top-level OFF organization) for details.

Implementations reading tables must include code to check version numbers so that, if and when the format and version number changes, older implementations will handle newer versions gracefully.

Minor version number changes always imply format changes that are compatible extensions. If an implementation understands a major version number, then it can safely proceed with reading the table. If the minor version is greater than the latest version recognized by the implementation, then the extension fields will be undetectable to the implementation.

For purposes of compatibility, version numbers that are represented using a single uint16 or uint32 value are treated like a minor version number, and any format changes are compatible extensions.

Note that some field values that were undefined or reserved in an earlier revision may be assigned meanings in a minor version change. Implementations should never make assumptions regarding reserved or unassigned values or bits in bit fields, and can ignore them if encountered. When writing font data, tools should always write zero for reserved fields or bits. Validators should warn of any non-zero values for fields or bits that are not defined for the given version against which data is being validated.

If the major version is not recognized, the implementation must not read the table as it can make no assumptions regarding interpretation of the binary data. The implementation should treat the table as missing.

## Top-level OFF organization

### Table directory

A key characteristic of the OFF format is the TrueType sfnt "wrapper", which provides organization for a collection of tables in a general and extensible manner.

The OFF font starts with the table directory, which is a directory of the top-level tables in a font. If the font file contains only one font, the table directory will begin at byte 0 of the file. If the font file is a font collection (see 4.6), the beginning point of the table directory for each font is indicated in the TTCHeader.

TableDirectory :

| Type | Name | Description |
| --- | --- | --- |
| uint32 | sfntVersion | 0x00010000 or 0x4F54544F ('OTTO') – see below. |
| uint16 | numTables | Number of tables. |
| uint16 | searchRange | Maximum power of 2 less than or equal to numTables, times 16 ((2**floor(log2(numTables))) * 16, where “**” is an exponentiation operator). |
| uint16 | entrySelector | Log2 of the maximum power of 2 less than or equal to numTables (log2(searchRange/16), which is equal to floor(log2(numTables))). |
| uint16 | rangeShift | numTables times 16, minus searchRange ((numTables * 16) - searchRange). |
| TableRecord | tableRecords[numTables] | Table records array—one for each top-level table in the font. |

NOTE 1	In the preceding table, the symbol “**” is an exponentiation operator, as used in several programming languages.

OFF fonts that contain TrueType outlines should use the value of 0x00010000 for sfntVersion. OFF fonts containing CFF data (version 1 or 2) should use 0x4F54544F ('OTTO', when re-interpreted as a Tag) for sfntVersion.

NOTE 2	Apple's TrueType Reference Manual allows for 'true' and 'typ1' for sfntVersion. These version tags should not be used for OFF fonts.

The table directory format allows for a large number of tables. To assist in quick binary searches, the searchRange, entrySelector and rangeShift fields are included as parameters that may be used in configuring search algorithms. Binary search is optimal when the number of entries as a power of two. The searchRange field provides the largest number of items that can be searched with that constraint (maximum power of two). The rangeShift field provides the remaining number of items that would also need to be searched. The entrySelector field indicates the maximum number of levels into the binary tree will need to be entered. Values are multiplied by 16, which is the size of each TableRecord.

In early implementations on devices with limited hardware capabilities, optimizations provided by the searchRange, entrySelector and rangeShift fields were of high importance. They have less importance on modern devices, but may still be used in some implementations. However, incorrect values could potentially be used as an attack vector against some implementations. Since these values can be derived from the numTables field when the file is parsed, it is strongly recommended that parsing implementations not rely on the searchRange, entrySelector and rangeShift fields in the font but derive them independently from numTables. Font files, however, should continue to provide valid values for these fields to maintain compatibility with all existing implementations.

### Table records

The table directory ends with the tableRecords array. The TableRecord format is as follows.

Table Record:

| Type | Name | Description |
| --- | --- | --- |
| Tag | tableTag | Table identifier. |
| uint32 | checksum | Checksum for this table. |
| Offset32 | offset | Offset from beginning of OFF font file. |
| uint32 | length | Length of this table. |

Table tags are the names given to tables in the OFF font file. The tableRecords array makes it possible for a given font to contain only those tables it actually needs. As a result, there is no standard value for numTables. The records in the array shall be sorted in ascending order by tag.

For requirements of Tag values, see 4.3 (Data types). Several tags and their associated table formats are defined in this specification. For table tags defined in this specification, a font resource should have at most one table record using a given tag. If a font resource does contain more than one table of a given type, behaviour is unpredictable: apps or platforms may select one of the tables arbitrarily, or may reject the font as invalid.

Platform vendors may define additional tables and associated tags to provide platform-specific functionality. See, for example, Apple’s TrueType Reference Manual [7] which defines various tables with associated tags not defined in this document. Some font development tools may also define special tables. Fonts that contain such additional tables can still qualify as Open Font Format fonts if they satisfy requirements of this document. For custom tables defined outside this document, an external specification of such a table may permit multiple tables of that type within a single font resource.

NOTE 1	The OpenType specification recommends that vendors notify Microsoft when they define custom tags to ensure forward compatibility in the event of future expansion of OpenType.

NOTE 2	Apple’s TrueType specification [7] stipulates that tag names consisting of all lower case letters are reserved for Apple’s use.

All tables shall begin on four-byte boundaries, and any remaining space between tables shall be padded with zeros. The length of each table should be recorded in the table record with the actual length of data, not the padded length.

NOTE 3	The requirement for four-byte alignment applies to top-level tables only and does not extend to sub-table offsets, records, or fields within tables or records.

Some tables have an internal structure with subtables located at specified offsets, and as a result, it is possible to construct a font with data for different tables interleaved. In general, tables should be arranged contiguously without overlapping the ranges of distinct tables. In any case, however, table lengths measure a contiguous range of bytes that encompasses all of the data for a table. This applies to any subtables as well as to top-level tables.

### Calculating checksums

Table checksums are the unsigned sum of the uint32 units of a given table. In C, the following function can be used to determine a checksum:

uint32

CalcTableChecksum(uint32 *Table, uint32 Length)

{

uint32 Sum = 0L;

uint32 *Endptr = Table+((Length+3) & ~3) / sizeof(uint32);

while (Table < EndPtr)

Sum += *Table++;

return Sum;

}

NOTE	This function assumes that the length of any table is a multiple of four bytes, or that tables are padded with zero to four-byte aligned offsets. Actual table lengths recorded in the table directory should not include padding, however. To accommodate data with a length that is not a multiple of four, the above algorithm must be modified to treat the data as though it contains zero padding to a length that is a multiple of four.

The 'head' table is a special case in checksum calculations, as it includes a checksumAdjustment field that is calculated and written after the table’s checksum is calculated and written into the table directory entry, necessarily invalidating that checksum value.

When generating font data, to calculate and write the 'head' table checksum and checksumAdjustment field, do the following:

- Set the checksumAdjustment field to 0.

- Calculate the checksum for all tables including the 'head' table and enter the value for each table into the corresponding record in the table directory.

- Calculate the checksum for the entire font.

- Subtract that value from 0xB1B0AFBA.

- Store the result in the ‘head’ table checksumAdjustment field.

An application attempting to verify that the 'head' table has not changed should calculate the checksum for that table assuming that the checksumAdjustment value is zero, rather than the actual value in the font, before comparing the result with the ‘head’ table record in the table directory.

Within a font collection file (see 4.6), table checksums shall reflect the tables as they are in the collection file. The checksumAdjustment field in the 'head' table is not used for collection files and may be set to zero.

## Font collections

### The Font Collection overview

A Font Collection (either TTC or OTC, formerly known as TrueType Collection) is a means of delivering multiple OFF font resources in a single file structure. The format for font collections allows font tables that are identical between two or more fonts to be shared. Font collections containing outline glyph data (TrueType, CFF, CFF2 or SVG) are most useful when the fonts to be delivered together share many glyphs in common. By allowing multiple fonts to share glyph sets and other common font tables, font collections can result in a significant saving of file space.

For example, a group of Japanese fonts may each have their own designs for the kana glyphs, but share identical designs for the kanji. With ordinary OFF font files, the only way to include the common kanji glyphs is to copy their glyph data into each font. Since the kanji represent much more data than the kana, this results in a great deal of wasteful duplication of glyph data. Font collections were defined to solve this problem.

NOTE 1	Even though the original definition of Font Collection (as part of the TrueType specification) was intended to be used with fonts containing TrueType outlines, and this constraint was maintained in earlier editions of OFF, this is no longer a constraint in OFF. Font Collection files may contain various types of outlines (or a mix of them), regardless of whether or not fonts have layout tables present. For backward compatibility and simplicity, the description of the font collection file structure is using the term "TrueType Collection", or TTC, though it is understood that it is used to identify a generic font collection structure containing any type of outline tables.

NOTE 2	A variable font is functionally equivalent to multiple non-variable fonts. Variable fonts do not need to be contained within a collection file. A collection file can include one or even multiple variable fonts, however, and may even combine variable and non-variable fonts.

### The Font Collection file structure

A font collection file consists of a single TTC header table, one or more table directories (each corresponding to a different font resource), and a number of OFF tables. The TTC header shall be located at the beginning of the TTC file.

The TTC file shall contain a complete table directory for each font resource. The same TableDirectory format is used for each font resource in a collection file as is used in a non-collection file. The table offsets in all table directories within a TTC file are measured from the beginning of the TTC file.

Each OFF table in a TTC file is referenced through the table directory of each font which uses that table. Some of the OFF tables must appear multiple times, once for each font included in the TTC; while other tables may be shared by multiple fonts in the TTC.

As an example, consider a TTC file which combines two Japanese fonts (Font1 and Font2). The fonts have different kana designs (Kana1 and Kana2) but use the same design for kanji. The TTC file contains a single 'glyf' table which includes both designs of kana together with the kanji; both fonts' table directories point to this 'glyf' table. But each font's table directory points to a different 'cmap' table, which identifies the glyph set to use. Font1's 'cmap' table points to the Kana1 region of the 'loca' and 'glyf' tables for kana glyphs, and to the kanji region for the kanji. Font2's 'cmap' table points to the Kana2 region of the 'loca' and 'glyf' tables for kana glyphs, and to the same kanji region for the kanji.

The tables that should have a unique copy per font are those that are used by the system in identifying the font and its character mapping, including 'cmap', 'name', and 'OS/2'. The tables that should be shared by fonts in the TTC are those that define glyph and instruction data or use glyph indices to access data: 'glyf', 'loca', 'hmtx', 'hdmx', 'LTSH', 'cvt ', 'fpgm', 'prep', 'EBLC', 'EBDT', 'EBSC', 'maxp', and so on. In practice, any tables which have identical data for two or more fonts may be shared.

NOTE	When building a collection file from separate font files, close attention needs to be paid to the issue of glyph renumbering in a font and the side effects that can result in the 'cmap' table and elsewhere. The fonts to be merged also need to have compatible TrueType instructions; that is, their preprograms, function definitions, and control values cannot conflict.

Collection files containing TrueType glyph outlines should use the filename suffix .TTC. Collection files containing CFF or CFF2 outlines should use the file extension .OTC.

### TTC header

There are two versions of the TTC header: Version 1.0 has been used for TTC files without digital signatures. Version 2.0 can be used for TTC files with or without digital signatures — if there's no signature, then the last three fields of the version 2.0 header are left null.

If a digital signature is used, the DSIG table for the file shall be located at the end of the TTC file, following any other font tables. Signatures in a TTC file are expected to be Format 1 signatures.

The purpose of the TTC header table is to locate the different table directories within a TTC file. The TTC header is located at the beginning of the TTC file (offset = 0). It consists of an identification tag, a version number, a count of the number of OFF fonts in the file, and an array of offsets to each table directory.

TTCHeader Version 1.0:

| Type | Name | Description |
| --- | --- | --- |
| Tag | ttcTag | Font Collection ID string: 'ttcf' (used for fonts with CFF or CFF2 outlines, as well as TrueType outlines) |
| uint16 | majorVersion | Major version of the TTCHeader, = 1. |
| uint16 | minorVersion | Minor version of the TTCHeader, = 0. |
| uint32 | numFonts | Number of fonts in TTC |
| Offset32 | tableDirectoryOffsets [numFonts] | Array of offsets to the TableDirectory for each font from the beginning of the file |

TTCHeader Version 2.0:

| Type | Name | Description |
| --- | --- | --- |
| Tag | ttcTag | Font Collection ID string: 'ttcf' |
| uint16 | majorVersion | Major version of the TTCHeader, = 2. |
| uint16 | minorVersion | Minor version of the TTCHeader, = 0. |
| uint32 | numFonts | Number of fonts in TTC |
| Offset32 | tableDirectoryOffsets [numFonts] | Array of offsets to the TableDirectory for each font from the beginning of the file |
| uint32 | dsigTag | Tag indicating that a DSIG table exists, 0x44534947 ('DSIG') (null if no signature) |
| uint32 | dsigLength | The length (in bytes) of the DSIG table (null if no signature) |
| uint32 | dsigOffset | The offset (in bytes) of the DSIG table from the beginning of the TTC file (null if no signature) |

# Open font tables

## Required common tables

### List of required tables

Whether TrueType or CFF outlines are used in an OFF font, the following tables are required for the font to function correctly:

| Tag | Name |
| --- | --- |
| cmap | Character to glyph mapping |
| head | Font header |
| hhea | Horizontal header |
| hmtx | Horizontal metrics |
| maxp | Maximum profile |
| name | Naming table |
| OS/2 | OS/2 and Windows specific metrics |
| post | PostScript information |

### cmap—Character to glyph index mapping table

#### Table overview

This table defines the mapping of character codes to a default glyph index. Different subtables may be defined that each contain mappings for different character encoding schemes. The table header indicates the character encodings for which subtables are present.

Regardless of the encoding scheme, character codes that do not correspond to any glyph in the font should be mapped to glyph index 0. The glyph at this location must be a special glyph representing a missing character, commonly known as .notdef.

Each subtable is in one of seven possible formats and begins with a format field indicating the format used. The first four formats — formats 0, 2, 4 and 6 — were originally defined prior to Unicode 2.0. These formats allow for 8-bit single-byte, 8-bit multi-byte, and 16-bit encodings. With the introduction of supplementary planes in Unicode 2.0, the Unicode addressable code space extends beyond 16 bits. To accommodate this, three additional formats were added — formats 8, 10 and 12 — that allow for 32-bit encoding schemes.

Other enhancements in Unicode led to the addition of other subtable formats. Subtable format 13 allows for an efficient mapping of many characters to a single glyph; this is useful for “last-resort” fonts that provide fallback rendering for all possible Unicode characters with a distinct fallback glyph for different Unicode ranges. Subtable format 14 provides a unified mechanism for supporting Unicode variation sequences.

Of the seven available formats, not all are commonly used today. Formats 4 or 12 are appropriate for most new fonts, depending on the Unicode character repertoire supported. Format 14 is used in many applications for support of Unicode variation sequences. Some platforms also make use for format 13 for a last-resort fallback font. Other subtable formats are not recommended for use in new fonts. Application developers, however, should anticipate that any of the formats may be used in fonts.

NOTE	The 'cmap' table version number remains at 0x0000 for fonts that make use of the newer subtable formats.

#### 'cmap' Header

The Character to Glyph Index Mapping Table is organized as follows.

'cmap' Header:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number (0) |
| uint16 | numTables | Number of encoding tables that follow |
| EncodingRecord | encodingRecords[numTables] |  |

#### Encoding records and encodings

The array of encoding records specifies particular encodings and the offset to the subtable for each encoding.

EncodingRecord:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | platformID | Platform ID. |
| uint16 | encodingID | Platform-specific encoding ID. |
| Offset32 | subtableOffset | Byte offset from beginning of table to the subtable for this encoding. |

The platform ID and platform-specific encoding ID in the encoding record are used to specify a particular character encoding. In the case of the Macintosh platform, a language field within the mapping subtable is also used for this purpose.

The encoding record entries in the 'cmap' header must be sorted first by platform ID, then by platform-specific encoding ID, and then by the language field in the corresponding subtable. Each platform ID, platform-specific encoding ID, and subtable language combination may appear only once in the 'cmap' table.

Apart from a format 14 subtable, all other subtables are exclusive: applications should select and use one and ignore the others. If a Unicode subtable is used (platform 0, or platform 3 / encoding 1 or 10), then a format 14 subtable using platform 0/encoding 5 can also be supplemented for mapping Unicode variation sequences.

If a font includes Unicode subtables for both 16-bit encoding (typically, format 4) and also 32-bit encoding (formats 10 or 12), then the characters supported by the subtable for 32-bit encoding should be a superset of the characters supported by the subtable for 16-bit encoding, and the 32-bit encoding should be used by applications.

Fonts should not include 16-bit Unicode subtables using both format 4 and format 6; format 4 should be used. Similarly, fonts should not include 32-bit Unicode subtables using both format 10 and format 12; format 12 should be used.

If a font includes encoding records for Unicode subtables of the same format but with different platform IDs, an application may choose which to select, but should make this selection consistently each time the font is used.

#### Platform IDs

The following platform IDs are defined:

| Platform ID | Platform name | Platform-specific encoding IDs |
| --- | --- | --- |
| 0 | Unicode | Various |
| 1 | Macintosh | Script manager code |
| 2 | ISO [deprecated] | ISO encoding [deprecated] |
| 3 | Windows | Windows encoding |
| 4 | Custom | Custom |

Platform ID values 240 through 255 are reserved for user-defined platforms, and shall never be assigned to a registered platform.

Unicode platform (platform ID = 0)

The following encoding IDs are defined for use with the Unicode platform:

| Encoding ID | Description |
| --- | --- |
| 0 | Unicode 1.0 semantics—deprecated |
| 1 | Unicode 1.1 semantics—deprecated |
| 2 | ISO/IEC 10646 semantics—deprecated |
| 3 | Unicode 2.0 and onwards semantics, Unicode BMP only |
| 4 | Unicode 2.0 and onwards semantics, Unicode full repertoire |
| 5 | Unicode variation sequences—for use with subtable format 14 |
| 6 | Unicode full repertoire—for use with subtable format 13 |

Use of encoding IDs 0, 1 or 2 is deprecated.

Encoding ID 3 should be used in conjunction with 'cmap' subtable formats 4 or 6. Encoding ID 4 should be used in conjunction with subtable formats 10 or 12.

Unicode variation sequences supported by the font should be specified in the 'cmap' table using a format 14 subtable. A format 14 subtable shall only be used under platform ID 0 and encoding ID 5; and encoding ID 5 should only be used with a format 14 subtable.

Encoding ID 6 should only be used in conjunction with 'cmap' subtable format 13; and subtable format 13 should only be used under platform ID 0 and encoding ID 6.

Macintosh platform (platform ID = 1)

Older Macintosh versions required fonts to have a 'cmap' subtable for platform ID 1. For current Apple platforms, use of platform ID 1 is discouraged. See 5.1.7.3 for details regarding encoding IDs defined for the Macintosh platform.

ISO platform (platform ID=2)

Use of this platform ID is deprecated.

The following encoding IDs are defined for use with ISO platform:

| Code | ISO encoding |
| --- | --- |
| 0 | 7-bit ASCII |
| 1 | ISO 10646 |
| 2 | ISO 8859-1 |

Windows platform (platform ID = 3)

The Windows platform supports several encodings. When creating fonts for Windows, Unicode 'cmap' subtables should always be used—platform ID 3 with encoding ID 1 or encoding ID 10. See below for additional details.

The following encoding IDs are supported on the Windows platform:

| Platform ID | Encoding ID | Description |
| --- | --- | --- |
| 3 | 0 | Symbol |
| 3 | 1 | Unicode BMP |
| 3 | 2 | ShiftJIS |
| 3 | 3 | PRC |
| 3 | 4 | Big5 |
| 3 | 5 | Wansung |
| 3 | 6 | Johab |
| 3 | 7 | Reserved |
| 3 | 8 | Reserved |
| 3 | 9 | Reserved |
| 3 | 10 | Unicode full repertoire |

Fonts that support only Unicode BMP characters (U+0000 to U+FFFF) on the Windows platform shall use encoding 1 with a format 4 subtable. This encoding shall not be used to support Unicode supplementary-plane characters.

Fonts that support Unicode supplementary-plane characters (U+10000 to U+10FFFF) on the Windows platform shall use encoding 10 with a format 12 subtable.

The symbol encoding was created to support fonts with arbitrary ornaments or symbols not supported in Unicode or other standard encodings. A format 4 subtable would be used, typically with up to 224 graphic characters assigned at code positions beginning with 0xF020. This corresponds to a sub-range within the Unicode Private-Use Area (PUA), though this is not a Unicode encoding. In legacy usage, some applications would represent the symbol characters in text using a single-byte encoding, and then map 0x20 to the OS/2.usFirstCharIndex value in the font. In new fonts, symbols or characters not in Unicode should be encoded using PUA code points in a Unicode 'cmap' subtable.

See the recommendations in 8.7 for additional information.

Custom platform (platform ID = 4) and OTF Windows NT compatibility mapping

Platform ID 4 is a legacy platform that was created to provide compatibility of older applications with OFF fonts that had been adapted from older Type 1 fonts. This platform is not commonly used today, and should not be used in new fonts.

| ID | Custom encoding |
| --- | --- |
| 0-255 | OTF Windows NT compatibility mapping |

This 'cmap' platform provides a compatibility mechanism for non-Unicode applications that use the font as if it were Windows ANSI encoded. Non-Windows ANSI Type 1 fonts, such as Cyrillic and Central European fonts that Adobe shipped in the past, had “0” (Windows ANSI) recorded in the CharSet field of the .PFM file; Adobe Type Manager for Windows 9x ignored the CharSet altogether. Adobe provides this compatibility 'cmap' encoding in every OFF font converted from a Type1 font in which the Encoding is not StandardEncoding.

When platform ID 4 is used, the encoding ID shall be set to the Windows charset value (in the range 0 to 255, inclusive) present in the .PFM file of the original Type 1 font.

If a platform ID 4, encoding ID 0 – 255 'cmap' encoding is present in an OFF font with CFF outlines, then the OTF font driver in Windows NT will: (a) superimpose the glyphs encoded at character codes 0-255 in the encoding on the corresponding Windows ANSI (code page 1252) Unicode values in the Unicode encoding it reports to the system; (b) add Windows ANSI (CharSet 0) to the list of CharSets supported by the font; and (c) consider the value of the encoding ID to be a Windows CharSet value and add it to the list of CharSets supported by the font. Note that the 'cmap' subtable needs to use Format 0 or 6 for its subtable, and the encoding needs to be identical to the CFF’s encoding.

#### cmap subtable formats

##### Use of the language field in 'cmap' subtables

All ‘cmap’ subtable formats include a language field. The language field shall be set to zero for all cmap subtables whose platform IDs are other than Macintosh (platform ID 1). For cmap subtables whose platform IDs are Macintosh, set this field to the Macintosh language ID of the cmap subtable plus one, or to zero if the cmap subtable is not language-specific. For example, a Mac OS Turkish cmap subtable must set this field to 18, since the Macintosh language ID for Turkish is 17. A Mac OS Roman cmap subtable must set this field to 0, since Mac OS Roman is not a language-specific encoding.

##### Format 0: Byte encoding table

Format 0 was the standard mapping subtable used on older Macintosh platforms (see Reference [7] for details) but is not required on newer Apple platforms.

'cmap' Subtable Format 0:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format number is set to 0. |
| uint16 | length | This is the length in bytes of the subtable. |
| uint16 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint8 | glyphIdArray[256] | An array that maps character codes to glyph index values. |

This is a simple 1 to 1 mapping of character codes to glyph indices. The glyph set is limited to 256. If this format is used to index into a larger glyph set, only the first 256 glyphs will be accessible.

##### Format 2: High byte mapping through table

This subtable format was created for “double-byte” encodings following national character code standards used for Japanese, Chinese, and Korean characters. These code standards use a mixed 8-/16-bit encoding. This format is not commonly used today.

In these mixed 8-/16-bit encodings, certain byte values signal the first byte of a 2-byte character. (These byte values are also valid as the second byte of a 2-byte character.) In addition, even for the 2-byte characters, the mapping of character codes to glyph index values depends heavily on the first byte. Consequently, the table begins with an array that maps the first byte to a SubHeader record. For 2-byte character codes, the SubHeader is used to map the second byte's value into a sub-range of a glyph index array—a sub-array—, as described below. When processing mixed 8/16-bit text, SubHeader 0 is special: it is used for single-byte character codes. When SubHeader 0 is used, a second byte is not needed; the single byte value is mapped through the specified sub-array.

'cmap' Subtable Format 2:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format number is set to 2. |
| uint16 | length | This is the length in bytes of the subtable. |
| uint16 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint16 | subHeaderKeys[256] | Array that maps high bytes into the subHeaders array: value is subHeader index × 8. |
| SubHeader | subHeaders[ ] | Variable-length array of SubHeader records. |
| uint16 | glyphIdArray[ ] | Variable-length array containing sub-arrays used for mapping the low byte of 2-byte characters. |

The SubHeader record is structured as follows:

SubHeader Record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | firstCode | First valid low byte for this SubHeader. |
| uint16 | entryCount | Number of valid low bytes for this SubHeader. |
| int16 | idDelta | See text below. |
| uint16 | idRangeOffset | See text below. |

The firstCode and entryCount values specify a subrange that begins at firstCode and has a length equal to the value of entryCount. This subrange stays within the 0-255 range of the byte being mapped. Bytes outside of this subrange are mapped to glyph index 0 (missing glyph). The offset of the byte within this subrange is then used as index into a corresponding sub-array of glyphIdArray. This sub-array is also of length entryCount. The value of the idRangeOffset is the number of bytes past the actual location of the idRangeOffset word where the glyphIdArray element corresponding to firstCode appears.

Finally, if the value obtained from the sub-array is not 0 (which indicates the missing glyph), you should add idDelta to it in order to get the glyphIndex. The value idDelta permits the same sub-array to be used for several different subheaders. The idDelta arithmetic is modulo 65536. If the result after adding idDelta to the value from the sub-array is less than zero, add 65536 to obtain a valid glyph ID.

##### Format 4: Segment mapping to delta values

This is the standard character-to-glyph-index mapping subtable for fonts that support only Unicode Basic Multilingual Plane characters (U+0000 to U+FFFF).

NOTE	To support Unicode supplementary-plane characters, format 12 should be used.

This format is used when the character codes for the characters represented by a font fall into several contiguous ranges, possibly with holes in some or all of the ranges (that is, some of the codes in a range may not have a representation in the font). The format-dependent data is divided into three parts, which shall occur in the following order:

- A four-word header gives parameters for an optimized search of the segment list.

- Four parallel arrays describe the segments (one segment for each contiguous range of codes).

- A variable-length array of glyph IDs (unsigned words).

'cmap' Subtable Format 4:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format number is set to 4. |
| uint16 | length | This is the length in bytes of the subtable. |
| uint16 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint16 | segCountX2 | 2 × segCount. |
| uint16 | searchRange | Maximum power of 2 less than or equal to segCount, times 2 ((2**floor(log2(segCount))) * 2, where “**” is an exponentiation operator) |
| uint16 | entrySelector | Log2 of the maximum power of 2 less than or equal to segCount (log2(searchRange/2), which is equal to floor(log2(segCount))) |
| uint16 | rangeShift | segCount times 2, minus searchRange ((segCount * 2) - searchRange) |
| uint16 | endCode[segCount] | End characterCode for each segment, last=0xFFFF. |
| uint16 | reservedPad | Set to 0. |
| uint16 | startCode[segCount] | Start character code for each segment. |
| int16 | idDelta[segCount] | Delta for all character codes in segment. |
| uint16 | idRangeOffset[segCount] | Offsets into glyphIdArray or 0 |
| uint16 | glyphIdArray[ ] | Glyph index array (arbitrary length) |

The number of segments is specified by segCount, which is not given directly in the header but is readily derived from segCountX2. All of the other header parameters are derived from it. The searchRange value is twice the largest power of 2 that is less than or equal to segCount. For example, if segCount=39, we have the following:

| segCountX2 | 78 |
| --- | --- |
| searchRange | 64 | (= 2 × (largest power of 2 <=39)) |
| entrySelector | 5 | (= log232) |
| rangeShift | 14 | (= 2 × 39 - 64) |

To assist in quick binary searches, the searchRange, entrySelector and rangeShift fields are included as parameters that can be used in configuring search algorithms. In particular, binary search is optimal when the number of entries is a power of two. The searchRange field provides the largest number of items that can be searched with that constraint (maximum power of two). The rangeShift field provides the remaining number of items that would also need to be searched. The entrySelector field indicates the maximum number of levels into the binary tree will need to be entered.

In early implementations on devices with limited hardware capabilities, optimizations provided by the searchRange, entrySelector and rangeShift fields were of high importance. They have less importance on modern devices, but could still be used in some implementations. However, incorrect values could potentially be used as an attack vector against some implementations. Since these values can be derived from the segCountX2 field when the file is parsed, it is strongly recommended that parsing implementations not rely on the searchRange, entrySelector and rangeShift fields in the font but derive them independently from segCountX2. Font files, however, should continue to provide valid values for these fields to maintain compatibility with all existing implementations.

Each segment is described by a startCode and endCode, along with an idDelta and an idRangeOffset, which are used for mapping the character codes in the segment. The segments are sorted in order of increasing endCode values, and the segment values are specified in four parallel arrays. You search for the first endCode that is greater than or equal to the character code you want to map. If the corresponding startCode is less than or equal to the character code, then you use the corresponding idDelta and idRangeOffset to map the character code to a glyph index (otherwise, the missingGlyph is returned). For the search to terminate, the final startCode and endCode values shall be 0xFFFF. This segment need not contain any valid mappings. (It can just map the single character code 0xFFFF to missingGlyph). However, the segment must be present.

If the idRangeOffset value for the segment is not 0, the mapping of character codes relies on glyphIdArray. The character code offset from startCode is added to the idRangeOffset value. This sum is used as an offset from the current location within idRangeOffset itself to index out the correct glyphIdArray value. This obscure indexing trick works because glyphIdArray immediately follows idRangeOffset in the font file. The C expression that yields the glyph index is:

glyphId = *(idRangeOffset[i]/2

+ (c - startCode[i])

+ &idRangeOffset[i])

The value c is the character code in question, and i is the segment index in which c appears. If the value obtained from the indexing operation is not 0 (which indicates missingGlyph), idDelta[i] is added to it to get the glyph index. The idDelta arithmetic is modulo 65536.

If the idRangeOffset is 0, the idDelta value is added directly to the character code offset (i.e. idDelta[i] + c) to get the corresponding glyph index. Again, the idDelta arithmetic is modulo 65536. If the result after adding idDelta[i] + c is less than zero, add 65536 to obtain a valid glyph ID.

As an example, the variant part of the table to map characters 10-20, 30-90, and 153-480 onto a contiguous range of glyph indices may look like this:

| segCountX2: | 8 |  |  |
| --- | --- | --- | --- |
| searchRange: | 8 |  |  |
| entrySelector: | 4 |  |  |
| rangeShift: | 0 |  |  |
| endCode: | 20 | 90 | 480 | 0xffff |
| reservedPad: | 0 |  |  |  |
| startCode: | 10 | 30 | 153 | 0xffff |
| idDelta: | -9 | -18 | -80 | 1 |
| idRangeOffset: | 0 | 0 | 0 | 0 |

This table yields the following mappings:

10 ⇒ 10 - 9 = 1

20 ⇒ 20 - 9 = 11

30 ⇒ 30 - 18 = 12

90 ⇒ 90 - 18 = 72

153 ⇒ 153 - 80 = 73

480 ⇒ 480 - 80 = 400

0xffff ⇒ 0

Note that the delta values could be reworked so as to reorder the segments.

##### Format 6: Trimmed table mapping

Format 6 was designed to map 16-bit characters to glyph indexes when the character codes for a font fall into a single contiguous range.

'cmap' Subtable Format 6:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format number is set to 6. |
| uint16 | length | This is the length in bytes of the subtable. |
| uint16 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint16 | firstCode | First character code of subrange. |
| uint16 | entryCount | Number of character codes in subrange. |
| uint16 | glyphIdArray [entryCount] | Array of glyph index values for character codes in the range. |

The firstCode and entryCount values specify a subrange (beginning at firstCode, length = entryCount) within the range of possible character codes. Codes outside of this subrange are mapped to glyph index 0. The offset of the code (from the first code) within this subrange is used as index to the glyphIdArray, which provides the glyph index value.

##### Format 8: mixed 16-bit and 32-bit coverage

Subtable format 8 was designed to support Unicode supplementary-plane characters in UTF-16 encoding, though it is not commonly used. Format 8 is similar to format 2 in that it provides for mixed-length character codes. Instead of allowing for 8- and 16-bit character codes, however, it allows for 16- and 32-bit character codes.

If a font contains Unicode supplementary-plane characters (U+10000 to U+10FFFF), then it's likely that it will also include Unicode BMP characters (U+0000 to U+FFFF) as well. Hence, there is a need to map a mixture of 16-bit and 32-bit character codes. A simplifying assumption is made: namely, that there are no 32-bit character codes which share the same first 16 bits as any 16-bit character code. (Since the Unicode code space extends only to U+10FFFF, a potential conflict exists only for characters U+0000 to U+0010, which are non-printing control characters.) This means that the determination as to whether a particular 16-bit value is a standalone character code or the start of a 32-bit character code can be made by looking at the 16-bit value directly, with no further information required.

'cmap' Subtable Format 8:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Subtable format; set to 8. |
| uint16 | reserved | Reserved; set to 0 |
| uint32 | length | Byte length of this subtable (including the header) |
| uint32 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint8 | is32[8192] | Tightly packed array of bits (8K bytes total) indicating whether the particular 16-bit (index) value is the start of a 32-bit character code |
| uint32 | numGroups | Number of groupings which follow |
| SequentialMapGroup | groups[numGroups] | Array of SequentialMapGroup records. |

Each sequential map group record specifies a character range and the starting glyph ID mapped from the first character. Glyph IDs for subsequent characters follow in sequence.

SequentialMapGroup Record:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | startCharCode | First character code in this group; note that if this group is for one or more 16-bit character codes (which is determined from the is32 array), this 32-bit value will have the high 16-bits set to zero |
| uint32 | endCharCode | Last character code in this group; same condition as listed above for the startCharCode |
| uint32 | startGlyphID | Glyph index corresponding to the starting character code |

A few notes here. The endCharCode is used, rather than a count, because comparisons for group matching are usually done on an existing character code, and having the endCharCode be there explicitly saves the necessity of an addition per group. Groups shall be sorted by increasing startCharCode. A group's endCharCode shall be less than the startCharCode of the following group, if any.

To determine if a particular word (cp) is the first half of 32-bit code points, one can use an expression such as ( is32[ cp / 8 ] & ( 1 << ( 7 - ( cp % 8 ) ) ) ). If this is non-zero, then the word is the first half of a 32-bit code point.

0 is not a special value for the high word of a 32-bit code point. A font may not have both a glyph for the code point 0x0000 and glyphs for code points with a high word of 0x0000.

The presence of the packed array of bits indicating whether a particular 16-bit value is the start of a 32-bit character code is useful even when the font contains no glyphs for a particular 16-bit start value. This is because the system software often needs to know how many bytes ahead the next character begins, even if the current character maps to the missing glyph. By including this information explicitly in this table, no "secret" knowledge needs to be encoded into the OS.

Although this format was created to support Unicode supplementary-plane characters, it is not widely supported or used. Also, no character encoding other than Unicode uses mixed 16-/32-bit characters. The use of this format is discouraged.

##### Format 10: Trimmed array

Subtable format 10 was designed to support Unicode supplementary-plane characters, though it is not commonly used. Format 10 is similar to format 6, in that it defines a trimmed array for a tight range of character codes. It differs, however, in that it uses 32-bit character codes.

'cmap' Subtable Format 10:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Subtable format; set to 10. |
| uint16 | reserved | Reserved; set to 0 |
| uint32 | length | Byte length of this subtable (including the header) |
| uint32 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint32 | startCharCode | First character code covered |
| uint32 | numChars | Number of character codes covered |
| uint16 | glyphIdArray [] | Array of glyph indices for the character codes covered |

This format is not widely used and is not supported on Windows platforms. It would be most suitable for fonts that support only a contiguous range of Unicode supplementary-plane characters, but such fonts are rare.

##### Format 12: Segmented coverage

This is the standard character-to-glyph-index mapping subtable for fonts supporting Unicode character repertoires that include supplementary-plane characters (U+10000 to U+10FFFF).

Fonts that include a format 12 subtable can also include a format 4 subtable for compatibility with older applications. This is not required, however. See the recommendations in 8.7 for additional information.

Format 12 is similar to format 4 in that it defines segments for sparse representation. It differs, however, in that it uses 32-bit character codes.

'cmap' Subtable Format 12:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Subtable format; set to 12. |
| uint16 | reserved | Reserved; set to 0 |
| uint32 | length | Byte length of this subtable (including the header) |
| uint32 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint32 | numGroups | Number of groupings which follow |
| SequentialMapGroup | groups[numGroups] | Array of SequentialMapGroup records. |

The sequential map group record is the same format as is used for the format 8 subtable. The qualifications regarding 16-bit character codes does not apply here, however, since characters codes are uniformly 32-bit.

SequentialMapGroup Record:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | startCharCode | First character code in this group |
| uint32 | endCharCode | Last character code in this group |
| uint32 | startGlyphID | Glyph index corresponding to the starting character code |

Groups shall be sorted by increasing startCharCode. A group's endCharCode shall be less than the startCharCode of the following group, if any. The endCharCode is used, rather than a count, because comparisons for group matching are usually done on an existing character code, and having the endCharCode be there explicitly saves the necessity of an addition per group.

##### Format 13: Many-to-one range mappings

This subtable provides for situations in which the same glyph is used for hundreds or even thousands of consecutive characters spanning across multiple ranges of the code space. This subtable format may be useful for “last resort” fonts, although these fonts may use other suitable subtable formats as well. (For "last resort" fonts, see also the 'head' table flags in subclause 5.1.3, bit 14).

NOTE	Subtable formats 13 has the same data structure as format 12; it differs only in the interpretation of the startGlyphID/glyphID fields.

'cmap' Subtable Format 13:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Subtable format; set to 13. |
| uint16 | reserved | Reserved; set to 0 |
| uint32 | length | Byte length of this subtable (including the header) |
| uint32 | language | For requirements on use of the language field, see subclause 5.1.2.5.1. |
| uint32 | numGroups | Number of groupings which follow |
| ConstantMapGroup | groups[numGroups] | Array of ConstantMapGroup records. |

The constant map group record has the same structure as the sequential map group record, with start and end character codes and a mapped glyph ID. However, the same glyph ID applies to all characters in the specified range rather than sequential glyph IDs.

ConstantMapGroup Record:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | startCharCode | First character code in this group |
| uint32 | endCharCode | Last character code in this group |
| uint32 | glyphID | Glyph index to be used for all the characters in the group's range. |

##### Format 14: Unicode variation sequences

Subtable format 14 specifies the Unicode variation sequences (UVSes) supported by the font. A variation sequence, according to the Unicode Standard, comprises a base character followed by a variation selector. For example, <U+82A6, U+E0101>.

This subtable format shall only be used under platform ID 0 and encoding ID 5.

The subtable partitions the UVSes supported by the font into two categories: “default” and “non-default” UVSes. Given a UVS, if the glyph obtained by looking up the base character of that sequence in the Unicode 'cmap' subtable (i.e., the BMP subtable or BMP + supplementary-planes subtable) is the glyph to use for that sequence, then the sequence is a “default” UVS. Otherwise, it is a “non-default” UVS, and the glyph to use for that sequence is specified in the format 14 subtable itself.

The example at the end of this subclause shows how a font vendor can use format 14 for a JIS-2004–aware font.

'cmap' Subtable Format 14:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Subtable format; set to 14. |
| uint32 | length | Byte length of this subtable (including this header) |
| uint32 | numVarSelectorRecords | Number of variation Selector Records |
| VariationSelector | varSelector[numVarSelectorRecords] | Array of VariationSelector records. |

Each VariationSelector record specifies a variation selector character, and offsets to “default” and “non-default” tables used to map variation sequences using that variation selector.

VariationSelector Record:

| Type | Name | Description |
| --- | --- | --- |
| uint24 | varSelector | Variation selector |
| Offset32 | defaultUVSOffset | Offset from the start of the format 14 subtable to Default UVS table. May be 0. |
| Offset32 | nonDefaultUVSOffset | Offset from the start of the format 14 subtable to Non-Default UVS table. May be 0. |

The VariationSelector records are sorted in increasing order of varSelector. No two records may have the same varSelector value.

A VariationSelector record and the data its offsets point to specify those UVSes supported by the font for which the variation selector is the varSelector value of the record. The base characters of the UVSes are stored in the tables pointed to by the offsets. The UVSes are partitioned by whether they are default or non-default UVSes.

Glyph IDs to be used for non-default UVSes are specified in the Non-Default UVS table.

Default UVS table

A Default UVS table is simply a range-compressed list of Unicode scalar values, representing the base characters of the default UVSes which use the varSelector of the associated VariationSelector record.

Default UVS Table:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | numUnicodeValueRanges | Number of Unicode character ranges. |
| UnicodeRange | ranges[numUnicodeValueRanges] | Array of UnicodeRange records. |

Each Unicode range record specifies a contiguous range of Unicode values.

UnicodeRange Record:

| Type | Name | Description |
| --- | --- | --- |
| uint24 | startUnicodeValue | First value in this range |
| uint8 | additionalCount | Number of additional values in this range |

For example, the range U+4E4D – U+4E4F (3 values) will set startUnicodeValue to 0x004E4D and additionalCount to 2. A singleton range will set additionalCount to 0.

The sum (startUnicodeValue + additionalCount) shall not exceed 0xFFFFFF.

The Unicode Value Ranges are sorted in increasing order of startUnicodeValue. The ranges must not overlap; i.e., (startUnicodeValue + additionalCount) must be less than the startUnicodeValue of the following range (if any).

All code points listed in the ranges array should have corresponding entries in a Unicode 'cmap' subtable. Applications could encounter fonts in which this is not the case, however.

Non-default UVS table

A Non-Default UVS table is a list of pairs of Unicode scalar values and glyph IDs. The Unicode values represent the base characters of all non-default UVSes which use the varSelector of the associated VariationSelector record, and the glyph IDs specify the glyph IDs to use for the UVSes.

Non-Default UVS Table:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | numUVSMappings | Number of UVS Mappings that follow |
| UVSMapping | uvsMappings[numUVSMappings] | Array of UVSMapping records. |

Each UVSMapping record provides a glyph ID mapping for one base Unicode character, when that base character is used in a variation sequence with the current variation selector.

UVSMapping Record:

| Type | Name | Description |
| --- | --- | --- |
| uint24 | unicodeValue | Base Unicode value of the UVS |
| uint16 | glyphID | Glyph ID of the UVS |

The UVS Mappings are sorted in increasing order of unicodeValue. No two mappings in this table may have the same unicodeValue values.

Typically, code points listed in the uvsMappings array will have corresponding entries in a Unicode 'cmap' subtable. This is not required, however. For example, it might not be the case if a font is intended for use with content in which a given Unicode character only occurs in a variation sequence.

Example

Here is an example of how a format 14 'cmap' subtable may be used in a font that is aware of JIS-2004 variant glyphs. The CIDs (character IDs) in this example refer to those in the Adobe Character Collection “Adobe-Japan1”, and can be assumed to be identical to the glyph IDs in the font in our example.

JIS-2004 changed the default glyph variants for some of its code points. For example:

JIS-90: U+82A6 ⇒ CID 1142 
JIS-2004: U+82A6 ⇒ CID 7961

Both of these glyph variants are supported through the use of Unicode variation sequences, as the following examples from Unicode’s UVS registry show:

U+82A6 U+E0100 ⇒ CID 1142 
U+82A6 U+E0101 ⇒ CID 7961

If the font wants to support the JIS-2004 variants by default, it will:

- encode glyph ID 7961 at U+82A6 in the Unicode 'cmap' subtable,

- specify <U+82A6, U+E0101> in the UVS 'cmap' subtable’s Default UVS table (varSelector will be 0x0E0101 and defaultUVSOffset will point to data containing a 0x0082A6 Unicode value)

- specify <U+82A6, U+E0100> ⇒ glyph ID 1142 in the UVS 'cmap' subtable’s Non-Default UVS table (varSelector will be 0x0E0100 and nonDefaultBaseUVSOffset will point to data containing a unicodeValue 0x0082A6 and glyphID 1142).

If, however, the font wants to support the JIS-90 variants by default, it will:

- encode glyph ID 1142 at U+82A6 in the Unicode 'cmap'subtable,

- specify <U+82A6, U+E0100> in the UVS 'cmap'subtable’s Default UVS Table

- specify <U+82A6, U+E0101> ⇒ glyph ID 7961 in the UVS 'cmap'subtable’s Non-Default UVS Table

### head—Font header

This table gives global information about the font. The bounding box values should be computed using only glyphs that have contours. Glyphs with no contours should be ignored for the purposes of these calculations.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the font header table — set to 1. |
| uint16 | minorVersion | Minor version number of the font header table — set to 0. |
| Fixed | fontRevision | Set by font manufacturer. |
| uint32 | checksumAdjustment | To compute: set it to 0, sum the entire font as uint32, then store 0xB1B0AFBA - sum. If the font is used as a component in a font collection file, the value of this field will be invalidated by changes to the file structure and font table directory, and must be ignored. |
| uint32 | magicNumber | Set to 0x5F0F3CF5. |
| uint16 | flags | Bit 0: Baseline for font at y=0. Bit 1: Left sidebearing point at x=0 (relevant only for TrueType rasterizers) — see additional information below regarding variable fonts. Bit 2: Instructions may depend on point size. Bit 3: Force ppem to integer values for all internal scaler math; may use fractional ppem sizes if this bit is clear. It is strongly recommended that this be set in hinted fonts. Bit 4: Instructions may alter advance width (the advance widths might not scale linearly). It is strongly recommended that this be set in hinted fonts. Bit 5: This bit is not used in OFF, and should not be set in order to ensure compatible behavior on all platforms. If set, it may result in different behavior for vertical layout in some platforms. (See the 'head' table specification of "Apple's TrueType Reference Manual" [7] for details regarding behavior in Apple platforms.) Bits 6 – 10: These bits are not used in OFF and should always be cleared. (See the 'head' table specification of "Apple's TrueType Reference Manual" [7] for details regarding legacy use in Apple platforms.) Bit 11: Font data is “lossless” as a result of having been subjected to optimizing transformation and/or compression (such as font compression mechanisms defined by ISO/IEC 14496-18, MicroType® Express, WOFF2 [24], or similar) where the original font functionality and features are retained but the binary compatibility between input and output font files is not guaranteed. As a result of the applied transform, the ‘DSIG’ table may also be invalidated. Bit 12: Font converted (produce compatible metrics). Bit 13: Font optimized for ClearType®. Note, fonts that rely on embedded bitmaps (EBDT) for rendering should not be considered optimized for ClearType, and therefore should keep this bit cleared. Bit 14: Last Resort font. If set, indicates that the glyphs encoded in the 'cmap' subtables are simply generic symbolic representations of code point ranges and do not truly represent support for those code points. If unset, indicates that the glyphs encoded in the cmap subtables represent proper support for those code points. Bit 15: Reserved, set to 0. |
| uint16 | unitsPerEm | Set to a value from 16 to 16384. Any value in this range is valid. In fonts that have TrueType outlines, a power of 2 is recommended as this allows performance optimization in some rasterizers. |
| LONGDATETIME | created | Number of seconds since 12:00 midnight that started January 1st, 1904, in GMT/UTC time zone. |
| LONGDATETIME | modified | Number of seconds since 12:00 midnight that started January 1st, 1904, in GMT/UTC time zone. |
| int16 | xMin | Minimum x coordinate across all glyph bounding boxes. |
| int16 | yMin | Minimum y coordinate across all glyph bounding boxes. |
| int16 | xMax | Maximum x coordinate across all glyph bounding boxes. |
| int16 | yMax | Maximum y coordinate across all glyph bounding boxes. |
| uint16 | macStyle | Bit 0: Bold (if set to 1); Bit 1: Italic (if set to 1) Bit 2: Underline (if set to 1) Bit 3: Outline (if set to 1) Bit 4: Shadow (if set to 1) Bit 5: Condensed (if set to 1) Bit 6: Extended (if set to 1) Bits 7 – 15: Reserved (set to 0). |
| uint16 | lowestRecPPEM | Smallest readable size in pixels. |
| int16 | fontDirectionHint a | Deprecated (Set to 2). 0: Fully mixed directional glyphs; 1: Only strongly left to right; 2: Like 1 but also contains neutrals; -1: Only strongly right to left; -2: Like -1 but also contains neutrals. |
| int16 | indexToLocFormat | 0 for short offsets (Offset16), 1 for long (Offset32). |
| int16 | glyphDataFormat | 0 for current format. |

a The fontDirectionHint field was intended for support of right-to-left scripts. Strong left to right, strong right to left, and neutral are properties of characters. A neutral character has no inherent directionality; it is not a character with zero (0) width. Spaces and punctuation are examples of neutral characters. Non-neutral characters are those with inherent directionality. For example, Roman letters (left-to-right) and Arabic letters (right-to-left) have strong directionality. In a "normal" Roman font where spaces and punctuation are present, the font direction hints should be set to two (2).

The unitsPerEm value determines the granularity of the font’s coordinate grid at which coordinates can be specified for visual elements such as outline control points or mark-attachment anchor positions.

A glyph’s bounding box is the smallest rectangle that contains all of the glyph’s control points. (See 5.2.4 for more information.) The xMin, yMin, xMax and yMax values provide a bounding box that will contain all glyphs from the font. Glyphs without contours are ignored.

The macStyle bits shall agree with the fsSelection bits the 'OS/2' table(5.1.8.20). The fsSelection bits are used over the macStyle bits in Windows. The PANOSE values and 'post' table values are ignored for determining bold or italic fonts.

For historical reasons, the fontRevision value contained in this table is not used by Windows to determine the version of a font. Instead, Windows evaluates the version string (ID 5) in the 'name' table.

The left sidebearing point for each glyph corresponds to the first “phantom” point (“pp1” below) and relates to the glyph left sidebearing (lsb) and xMin as follows:

pp1 = xMin - lsb

If bit 1 of the flags field is set, then pp1 = 0 for all glyphs and the xMin and left sidebearing for each glyph shall be equal.

In a variable font with TrueType outlines, the left side bearing for each glyph shall equal xMin, and bit 1 in the flags field shall be set. Also, bit 5 shall be cleared in all variable fonts. For general information on OFF Font Variations, see subclause 7.1.

Also, in a variable font, the minimum or maximum x or y values of control points can vary, and a tight bounding rectangle containing the outline or all points for an instance of any given glyph can be smaller or larger than for the default instance of that glyph. The xMin, yMin, xMax and yMax values in this table might or might not encompass the derived glyph outlines for an instance. In addition, variation deltas are not provided for these values. If an application requires a bounding rectangle that encompasses the glyphs for a non-default instance of the font, the derived glyph outlines for the instance should be processed to determine a bounding rectangle.

### hhea—Horizontal header

This table contains information for horizontal layout. The values in the minRightSidebearing, minLeftSideBearing and xMaxExtent should be computed using only glyphs that have contours. Glyphs with no contours should be ignored for the purposes of these calculations. All reserved areas shall be set to 0.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the horizontal header table — set to 1. |
| uint16 | minorVersion | Minor version number of the horizontal header table — set to 0. |
| FWORD | ascender | Typographic ascent—see remarks below. |
| FWORD | descender | Typographic descent—see remarks below. |
| FWORD | lineGap | Typographic line gap. Negative lineGap values are treated as zero in some legacy platform implementations. |
| UFWORD | advanceWidthMax | Maximum advance width value in 'hmtx' table. |
| FWORD | minLeftSideBearing | Minimum left sidebearing value in 'hmtx' table for glyphs with contours (empty glyphs should be ignored). |
| FWORD | minRightSideBearing | Minimum right sidebearing value; calculated as min(aw - (lsb + xMax - xMin)) for glyphs with contours (empty glyphs should be ignored). |
| FWORD | xMaxExtent | Max(lsb + (xMax - xMin)). |
| int16 | caretSlopeRise | Used to calculate the slope of the cursor (rise/run); 1 for vertical. |
| int16 | caretSlopeRun | 0 for vertical. |
| int16 | caretOffset | The amount by which a slanted highlight on a glyph needs to be shifted to produce the best appearance. Set to 0 for non-slanted fonts |
| int16 | (reserved) | set to 0 |
| int16 | (reserved) | set to 0 |
| int16 | (reserved) | set to 0 |
| int16 | (reserved) | set to 0 |
| int16 | metricDataFormat | 0 for current format. |
| uint16 | numberOfHMetrics | Number of hMetric entries in 'hmtx' table |

The ascender, descender and linegap values in this table are Apple specific; see the ‘hhea’ table specification of Apple’s TrueType Reference Manual [7] for details regarding Apple platforms. The sTypoAscender, sTypoDescender and sTypoLineGap fields in the OS/2 table are used on the Windows platform, and are recommended for new text-layout implementations. Font developers should evaluate behavior in target applications that may use fields in this table or in the OS/2 table to ensure consistent layout. See the descriptions of the OS/2 fields (5.1.8.23 – 5.1.8.25) for additional details.

'hhea' Table and OFF Font Variations

In a variable font, various font-metric values within the horizontal header table may need to be adjusted for different variation instances. Variation data for 'hhea' entries can be provided in the metrics variations ('MVAR') table. Different 'hhea' entries are associated with particular variation data in the 'MVAR' table using value tags, as follows:

| 'hhea' entry | Tag |
| --- | --- |
| caretOffset | 'hcof' |
| caretSlopeRise | 'hcrs' |
| caretSlopeRun | 'hcrn' |

For general information on OFF Font Variations, see subclause 7.1.

### hmtx—Horizontal metrics

Glyph metrics used for horizontal text layout include glyph advance widths, side bearings and X-direction min and max values (xMin, xMax). These are derived using a combination of the glyph outline data ('glyf', 'CFF ' or 'CFF2') and the horizontal metrics table. The horizontal metrics ('hmtx') table provides glyph advance widths and left side bearings.

In a font with TrueType outline data, the 'glyf' table provides xMin and xMax values, but not advance widths or side bearings. The advance width is always obtained from the 'hmtx' table. In some fonts, depending on the state of flags in the 'head' table, the left side bearings may be the same as the xMin values in the 'glyf' table, though this is not true for all fonts. (See the description of bit 1 of the flags field in the 'head' table.) For this reason, left side bearings are provided in the 'hmtx' table. The right side bearing is always derived using advance width and left side bearing values from the 'hmtx' table, plus bounding-box information in the glyph description — see below for more details.

In a variable font with TrueType outline data, the left side bearing value in the 'hmtx' table must always be equal to xMin (bit 1 of the 'head' flags field must be set). Hence, these values can also be derived directly from the 'glyf' table. Note that these values apply only to the default instance of the variable font: non-default instances may have different side bearing values. These can be derived from interpolated “phantom point” coordinates using the 'gvar' table (see below for additional details), or by applying variation data in the 'HVAR' table to default-instance values from the 'glyf' or 'hmtx' table.

In a font with CFF version 1 outline data, the 'CFF ' table does include advance widths. These values are used by PostScript processors, but are not used in OFF layout. In an OFF context, the 'hmtx' table is required and must be used for advance widths. Note that fonts in a Font Collection file that share a 'CFF ' table may specify different advance widths in font-specific 'hmtx' tables for a particular glyph index. Also note that the 'CFF2' table does not include advance widths. In addition, for either 'CFF ' or 'CFF2' data, there are no explicit xMin and xMax values; side bearings are implicitly contained within the CharString data, and can be obtained from the CFF / CFF2 rasterizer. Some layout engines may use left side bearing values in the 'hmtx' table, however; hence, font production tools should ensure that the left side bearing values in the 'hmtx' table match the implicit xMin values reflected in the CharString data. In a variable font with CFF2 outline data, left side bearing and advance width values for non-default instances should be obtained by combining information from the 'hmtx' and 'HVAR' tables.

The table uses a LongHorMetric record to give the advance width and left side bearing of a glyph. Records are indexed by glyph ID. As an optimization, the number of records can be less than the number of glyphs, in which case the advance width value of the last record applies to all remaining glyph IDs. This can be useful in monospaced fonts, or in fonts that have a large number of glyphs with the same advance width (provided the glyphs are ordered appropriately). The number of LongHorMetric records is determined by the numberOfHMetrics field in the 'hhea' table.

If numberOfHMetrics is less than the total number of glyphs, then the hMetrics array is followed by an array for the left side bearing values of the remaining glyphs. The number of elements in the leftSideBbearing array will be derived from the numGlyphs field in the 'maxp' table minus numberOfHMetrics.

Horizontal Metrics Table:

| Type | Name | Description |
| --- | --- | --- |
| LongHorMetric | hMetrics [numberOfHMetrics] | Paired advance width and left side bearing values for each glyph. Records are indexed by glyph ID. |
| FWORD | leftSideBearings [numGlyphs - numberOfHMetrics] | Left side bearings for glyph IDs greater than or equal to numberOfHMetrics. |

LongHorMetric Record:

| Type | Name | Description |
| --- | --- | --- |
| UFWORD | advanceWidth | Advance width, in font design units. |
| FWORD | lsb | Glyph left side bearing, in font design units. |

In a font with TrueType outlines, xMin and xMax values for each glyph are given in the 'glyf' table. The advance width (“aw”) and left side bearing (“lsb”) can be derived from the glyph “phantom points”, which are computed by the TrueType rasterizer; or they can be obtained from the 'hmtx' table. In a font with CFF or CFF2 outlines, xMin (= left side bearing) and xMax values can be obtained from the CFF / CFF2 rasterizer. From those values, the right side bearing (“rsb”) is calculated as follows:

rsb = aw - (lsb + xMax - xMin)

If pp1 and pp2 are TrueType phantom points used to control lsb and rsb, their initial position in the X-direction is calculated as follows:

pp1 = xMin - lsb

pp2 = pp1 + aw

If a glyph has no contours, xMax/xMin are not defined. The left side bearing indicated in the 'hmtx' table for such glyphs should be zero.

### maxp—Maximum profile

This table establishes the memory requirements for this font. Fonts with CFF or CFF2 outlines shall use Version 0.5 of this table, specifying only the numGlyphs field. Fonts with TrueType outlines must use Version 1.0 of this table, where all data is required.

Version 0.5

| Type | Name | Description |
| --- | --- | --- |
| Version16Dot16 | version | 0x00005000 for version 0.5 |
| uint16 | numGlyphs | The number of glyphs in the font. |

Version 1.0

| Type | Name | Description |
| --- | --- | --- |
| Version16Dot16 | version | 0x00010000 for version 1.0. |
| uint16 | numGlyphs | The number of glyphs in the font. |
| uint16 | maxPoints | Maximum points in a non-composite glyph. |
| uint16 | maxContours | Maximum contours in a non-composite glyph. |
| uint16 | maxCompositePoints | Maximum points in a composite glyph. |
| uint16 | maxCompositeContours | Maximum contours in a composite glyph. |
| uint16 | maxZones | 1 if instructions do not use the twilight zone (Z0), or 2 if instructions do use Z0; should be set to 2 in most cases. |
| uint16 | maxTwilightPoints | Maximum points used in Z0. |
| uint16 | maxStorage | Number of Storage Area locations. |
| uint16 | maxFunctionDefs | Number of FDEFs, equal to the highest function number + 1. |
| uint16 | maxInstructionDefs | Number of IDEFs. |
| uint16 | maxStackElements | Maximum stack depth across Font Program ('fpgm' table), CVT Program ('prep' table) and all glyph instructions (in the 'glyf' table). |
| uint16 | maxSizeOfInstructions | Maximum byte count for glyph instructions. |
| uint16 | maxComponentElements | Maximum number of components referenced at "top level" for any composite glyph. |
| uint16 | maxComponentDepth | Maximum levels of recursion; 1 for simple components. |

### name—Naming table

#### Table structure

The naming table allows multilingual strings to be associated with the OFF font. These strings can represent copyright notices, font names, family names, style names, and so on. To keep this table short, the font manufacturer may wish to make a limited set of entries in some small set of languages; later, the font can be "localized" and the strings translated or added. Other parts of the OFF font that require these strings can refer to them using a language-independent name ID. In addition to language variants, the table also allows for platform-specific character-encoding variants. Applications that need a particular string can look it up by its platform ID, encoding ID, language ID and name ID. Note that different platforms may have different requirements for the encoding of strings.

Many newer platforms can use strings intended for different platforms if a font does not include strings for that platform. Some applications might display incorrect strings, however, if strings for the current platform are not included.

Naming table header

There are two versions of the Naming Table. Version 0 uses platform-specific, numeric language identifiers. Version 1 allows for use of language-tag strings to indicate the language of strings. Both versions include variable-size string-data storage, and an array of name records that are used to identify the type of string (name ID), platform, encoding and language variants of the string, and the location within the storage.

Naming table version 0

The version 0 naming table is organized as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number (=0). |
| uint16 | count | Number of name records. |
| Offset16 | storageOffset | Offset to start of string storage (from start of table). |
| NameRecord | nameRecord[count] | The name records where count is the number of records. |
| (Variable) |  | Storage for the actual string data. |

Version 0 differs from version 1 in regard to handling of language identification: it uses only numeric language IDs, which generally are values less than 0x8000 and have platform-specific interpretations. See subclause 5.1.7.2 for more details.

Naming table version 1

The version 1 naming table adds additional elements, as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number (=1). |
| uint16 | count | Number of name records. |
| Offset16 | storageOffset | Offset to start of string storage (from start of table). |
| NameRecord | nameRecord[count] | The name records where count is the number of records. |
| uint16 | langTagCount | Number of language-tag records. |
| LangTagRecord | langTagRecord[langTagCount] | The language-tag records where langTagCount is the number of records. |
| (Variable) |  | Storage for the actual string data. |

When version 1 is used, the language IDs in name records can be less than or greater than 0x8000. If a language ID is less than 0x8000, it has a platform-specific interpretation as with a version 0 naming table. If a language ID is equal to or greater than 0x8000, it is associated with a language-tag record (LangTagRecord) that references a language-tag string. In this way, the language ID is associated with a language-tag string that specifies the language for name records using that language ID, regardless of the platform. These can be used for any platform that supports this language-tag mechanism.

A font using a version 1 naming table may use a combination of platform-specific language IDs as well as language-tag records for a given platform and encoding.

Each LangTagRecord is organized as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | length | Language-tag string length (in bytes). |
| Offset16 | langTagOffset | Language-tag string offset from start of storage area (in bytes). |

Language-tag strings stored in the naming table must be encoded in UTF-16BE. The language tags shall conform to IETF specification BCP 47. This provides tags such as "en", "fr-CA" and "zh-Hant" to identify languages, including dialects, written form and other language variants.

The language-tag records are associated sequentially with language IDs starting with 0x8000. Each language-tag record corresponds to a language ID one greater than that for the previous language-tag record. Thus, language IDs associated with language-tag records must be within the range 0x8000 to 0x8000 + langTagCount - 1. If a name record uses a language ID that is greater than this, the identity of the language is unknown; such name records should not be used.

For example, suppose a font has two language-tag records referencing strings in the storage: the first references the string "en", and the second references the string "zh-Hant-HK" In this case, the language ID 0x8000 is used in name records to index English-language strings. The language ID 0x8001 is used in name records to index strings in Traditional Chinese as used in Hong Kong.

#### Name records

Each string in the string storage is referenced by a name record. The name record has a multi-part key, to identify the logical type of string and its language or platform-specific implementation variants, plus the location of the string in the string storage.

Each NameRecord is organized as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | platformID | Platform ID. |
| uint16 | encodingID | Platform-specific encoding ID. |
| uint16 | languageID | Language ID. |
| uint16 | nameID | Name ID. |
| uint16 | length | String length (in bytes). |
| Offset16 | stringOffset | String offset from start of storage area (in bytes). |

The name ID identifies a logical string category, such as family name or copyright. Name IDs are the same for all platforms and languages; these are described in detail below. The other three elements of the key allow for platform-specific implementations: a platform ID, a platform-specific encoding ID, and a language ID.

As with encoding records in the 'cmap' table (subclause 5.1.2), name records shall be sorted first by platform ID, then by platform-specific encoding ID, then by language ID, and then by name ID. Descriptions of the various IDs follow.

#### Platform, encoding and language

The platform, encoding and language IDs of a name record allow for platform-specific implementations. Different platforms can support different encodings, and different languages. All encoding IDs are platform-specific. Language IDs are similarly platform-specific, except in the case of IDs used in conjunction with the language-tag mechanism of naming table version 1, described above.

NOTE 	Platform IDs, platform-specific encoding IDs and, in some cases, platform-specific language IDs are also used in the 'cmap' table (see subclause 5.1.2)

Language IDs refer to a value that identifies the language in which a particular string is written. Values less than 0x8000 are defined on a platform-specific basis. A version 0 naming table shall use only language ID values less than 0x8000 from the platform-specific enumerations given below. (Exceptions to this requirement are permitted, however, in the case of user-defined platforms — platform IDs 240 to 255.) Values greater than or equal to 0x8000 may be used in a version 1 naming table in conjunction with language-tag records, as described above. Not all platforms have platform-specific language IDs, and not all platforms support language-tag records.

Tables with detailed listings of platform, encoding and language IDs used within the 'name' table are provided in 5.1.7.5.

#### Name IDs

The following name IDs are pre-defined and apply to all platforms unless indicated otherwise. Name IDs 26 to 255, inclusive, are reserved for future standard names. Name IDs 256 to 32767, inclusive, are reserved for font-specific names such as those referenced by a font's layout features.

| ID | Meaning |
| --- | --- |
| 0 | Copyright notice. |
| 1 | Font Family name. The Font Family name is used in combination with Font Subfamily name (name ID 2), and should be shared among at most four fonts that differ only in weight or style (italic/oblique), as described below. This four-way distinction should also be reflected in the OS/2.fsSelection field, using bits 0 and 5. While some platforms or applications do not have this constraint, many existing applications that use this pair of names assume that a Font Family name is shared by at most four fonts that form a font style-linking group: regular, italic (or oblique), bold, and bold italic (or bold oblique). To be compatible with the broadest range of platforms and applications, it is strongly recommended that fonts limit use of Font Family name in this manner. For extended typographic families that includes fonts other than the four basic styles (regular, italic, bold, bold italic), it is strongly recommended that name IDs 16 and 17 be used in fonts to create an extended, typographic grouping. (See examples provided below.) It is also strongly recommended that applications support extended typographic-family groupings using name IDs 16 and 17. Note that variable fonts can include a large number of named instances, each of which will use a shared typographic family name (name ID 16) and will have a typographic subfamily name (equivalent to name ID 17). Applications that assume a four-style family grouping based on name IDs 1 and 2 are likely to provide a poor user experience with variable fonts. For fonts within an extended typographic family that fall outside the basic four-way distinction, the distinguishing attributes should be reflected in the Font Family name so that those fonts appear as a separate font family in applications that support only four-member families. For example, the Font Family name for the Arial Narrow font is “Arial Narrow”; the Font Family name for the Arial Black font is “Arial Black”. Note that, in such cases, name ID 16 should also be included with a shared name (e.g., "Arial") that reflects the full, typographic family. |
| 2 | Font Subfamily name. The Font Subfamily is used in combination with Font Family name (name ID 1), and distinguishes the fonts in a group with the same Font Family name. This should be used for weight and style (italic/oblique) variants only, as described below. This four-way distinction should also be reflected in the OS/2.fsSelection field, using bits 0 and 5. While some platforms or applications do not have this constraint, many existing applications that use name IDs 1 and 2 assume that a Font Family name is shared by at most four fonts that form a font style-linking group, and that Font Subfamily names would reflect one of the four basic styles, regular, italic (or oblique), bold, and bold italic (or bold oblique). To be compatible with the broadest range of platforms and applications, it is strongly recommended that fonts should limit use of Font Subfamily in this manner. For extended typographic families that includes fonts other than the four basic styles (regular, italic, bold, bold italic), it is strongly recommended that name IDs 16 and 17 be used in fonts to create an extended, typographic grouping. Within an extended typographic family that includes fonts beyond regular, bold, italic, or bold italic, distinctions for these other fonts are made in the Font Family name so that fonts appear to be in separate families. In some cases, this can lead to specifying a Subfamily name of “Regular” for a font that might not otherwise be considered a regular font. For example, the Arial Black font has a Font Family name of “Arial Black” and a Subfamily name of “Regular”. Note that, in such cases, name IDs 16 and 17 should also be included, using a shared value for name ID 16 (e.g., "Arial") that reflects the full typographic family, and values for name ID 17 that appropriately reflect the actual design variant of each font. Fonts that are not part of an extended typographic family and with no distinctive weight or style (e.g., medium weight, not italic) should use "Regular" as the Font Subfamily name (for English). |
| 3 | Unique font identifier |
| 4 | Full font name that reflects all family and relevant subfamily descriptors. The full font name is generally a combination of name IDs 1 and 2, or of name IDs 16 and 17, or a similar human-readable variant. For fonts in extended typographic families (that is, families that include more than regular, italic, bold, and bold italic variants), values for name IDs 1 and 2 are normally chosen to provide compatibility with certain applications that assume a family has at most four style-linked fonts. In that case, some fonts may end up with a Subfamily name (name ID 2) of “Regular” even though the font would not be considered, typographically, a regular font. For such non-regular fonts in which name ID 2 is specified as “Regular”, the “Regular” descriptor would generally be omitted from name ID 4. For example, the Arial Black font has a Font Family name (name ID 1) of “Arial Black” and a Subfamily name (name ID 2) of “Regular”, but has a full font name (name ID 4) of “Arial Black”. Note that name IDs 16 and 17 should also be included in these fonts, and that name ID 4 would typically be a combination of name IDs 16 and 17, without needing any additional qualifications regarding “Regular”. |
| 5 | Version string. Should begin with the pattern 'Version <number>.<number>' (upper case, lower case, or mixed, with a space between "Version" and the number). The string must contain a version number of the following form: one or more digits (0-9) of value less than 65,535, followed by a period, followed by one or more digits of value less than 65,535. Any character other than a digit will terminate the minor number. A character such as ";" is helpful to separate different pieces of version information. The first such match in the string can be used by installation software to compare font versions. Some installers could require the string to start with "Version ", followed by a version number as above. |
| 6 | PostScript name for the font; Name ID 6 specifies a string which is used to invoke a PostScript language font that corresponds to this OFF font. When translated to ASCII, the name string must be no longer than 63 characters and restricted to the printable ASCII subset, codes 33 to 126, except for the 10 characters '[', ']', '(', ')', '{', '}', '<', '>', '/', '%'. In a CFF OFF font, there is no requirement that this name be the same as the font name in the CFF’s Name INDEX. Thus, the same CFF may be shared among multiple font components in an OFF Font Collection. See subclause 8.19 for additional information. |
| 7 | Trademark; this is used to save any trademark notice/information for this font. Such information should be based on legal advice. This is distinctly separate from the copyright. |
| 8 | Manufacturer Name. |
| 9 | Designer; name of the designer of the typeface. |
| 10 | Description; description of the typeface. Can contain revision information, usage recommendations, history, features, etc. |
| 11 | URL Vendor; URL of font vendor (with protocol, e.g., http://, ftp://). If a unique serial number is embedded in the URL, it can be used to register the font. |
| 12 | URL Designer; URL of typeface designer (with protocol, e.g., http://, ftp://). |
| 13 | License Description; description of the license or licenses under which the font is provided. This could be a reference to a named license agreement (e.g., a common open source licenses), identification of a software-use license under which a font is bundled, information about where to locate an external license (see also name ID 14), a summary of permitted uses, or the full legal text of a license agreement. It is prudent to seek legal advice on the content of this name ID to avoid possible conflict of interpretation between it and the license(s). |
| 14 | License Info URL; URL where additional licensing information can be found. |
| 15 | Reserved. |
| 16 | Typographic Family name: The typographic family grouping doesn't impose any constraints on the number of faces within it, in contrast with the 4-style family grouping (ID 1), which is present both for historical reasons and to express style linking groups. If name ID 16 is absent, then name ID 1 is considered to be the typographic family name. (In earlier versions of the specification, name ID 16 was known as "Preferred Family".) |
| 17 | Typographic Subfamily name: This allows font designers to specify a subfamily name within the typographic family grouping. This string must be unique within a particular typographic family. If it is absent, then name ID 2 is considered to be the typographic subfamily name. (In earlier versions of the specification, name ID 17 was known as "Preferred Subfamily".) |
| 18 | Compatible Full (Macintosh only); On the Macintosh, the menu name is constructed using the FOND resource. This usually matches the Full Name. If you want the name of the font to appear differently than the Full Name, you can insert the Compatible Full Name in ID 18. |
| 19 | Sample text; This can be the font name, or any other text that the designer thinks is the best sample to display the font in. |
| 20 | PostScript CID findfont name; Its presence in a font means that the nameID 6 holds a PostScript font name that is meant to be used with the "composefont" invocation in order to invoke the font in a PostScript interpreter. See the definition of name ID 6. The value held in the name ID 20 string is interpreted as a PostScript font name that is meant to be used with the "findfont" invocation, in order to invoke the font in a PostScript interpreter. When translated to ASCII, this name string must be restricted to the printable ASCII subset, codes 33 through 126, except for the 10 characters: '[', ']', '(', ')', '{', '}', '<', '>', '/', '%'. See subclause 8.19 for additional information. |
| 21 | WWS Family Name. Used to provide a WWS-conformant family name in case the entries for IDs 16 and 17 do not conform to the WWS model. (That is, in case the entry for ID 17 includes qualifiers for some attribute other than weight, width or slope.) If bit 8 of the OS/2 fsSelection field is set, a WWS Family Name entry should not be needed and should not be included. Conversely, if an entry for this ID is included, bit 8 should not be set. (See 5.1.8.20 for details.) Examples of name ID 21: “Minion Pro Caption” and “Minion Pro Display”. (Name ID 16 would be "Minion Pro" for these examples.) See additional remarks regarding IDs 21 and 22 below. |
| 22 | WWS Subfamily Name. Used in conjunction with ID 21, this ID provides a WWS-conformant subfamily name (reflecting only weight, width and slope attributes) in case the entries for IDs 16 and 17 do not conform to the WWS model. As in the case of ID 21, use of this ID should correlate inversely with bit 8 of the OS/2 fsSelection field being set. Examples of name ID 22: “Semibold Italic”, “Bold Condensed”. (Name ID 17 could be "Semibold Italic Caption", or "Bold Condensed Display", for example.) See additional remarks regarding IDs 21 and 22 below. |
| 23 | Light Background Palette. This ID, if used in the CPAL table’s Palette Labels Array, specifies that the corresponding color palette in the CPAL table is appropriate to use with the font when displaying it on a light background such as white. Strings for this ID are for use as user interface strings associated with this palette. |
| 24 | Dark Background Palette. This ID, if used in the CPAL table’s Palette Labels Array, specifies that the corresponding color palette in the CPAL table is appropriate to use with the font when displaying it on a dark background such as black. Strings for this ID are for use as user interface strings associated with this palette. |
| 25 | Variations PostScript Name Prefix. If present in a variable font, it may be used as the family prefix in the PostScript Name Generation for Variation Fonts algorithm. The character set is restricted to ASCII-range uppercase Latin letters, lowercase Latin letters, and digits. All name strings for name ID 25 within a font, when converted to ASCII, must be identical. See Reference [27] for reasons to include name ID 25 in a font, and for examples. For general information on OFF Font Variations, see subclause 7.1. |

For a typographic family that includes member faces that differ from Regular in relation to attributes other than weight, width or slope, there may also be some member faces that differ only in relation to these three attributes. For example, the Minion Pro family includes Minion Pro Display, but also includes Minion Pro Bold and Minion Pro Italic. IDs 21 and 22 should be used only in those fonts that differ from the Regular face in terms of an attribute other than weight, width or slope. For example, IDs 21 and 22 should be used in Minion Pro Display, but not in Minion Pro Bold or Minion Pro Italic.

NOTE	While both Apple and Microsoft support the same set of name strings, the interpretations may be somewhat different. But since name strings are stored by platform, encoding and language (placing separate strings for both Apple and MS platforms), this should not present a problem.

The key information for this table for Microsoft platforms relates to the use of name IDs 1, 2, 4, 16 and 17. Note that some newer applications will use name IDs 16 and 17, while some legacy applications require name IDs 1 and 2 and also assume certain limitations on these values (see descriptions of name IDs 1 and 2 above). Fonts should include all of these strings for the broadest application compatibility. To better understand how to set values for these name IDs, some examples of name usage, weight class and style flags have been created.

Name ID examples

The following is an example of how name strings would be made for the Arial family:

| Font | Name ID 1 | Name ID 2 | Name ID 4 | Name ID 16 | Name ID 17 |
| --- | --- | --- | --- | --- | --- |
| Arial Narrow | Arial Narrow | Regular | Arial Narrow | Arial | Narrow |
| Arial Narrow Italic | Arial Narrow | Italic | Arial Narrow Italic | Arial | Narrow Italic |
| Arial Narrow Bold | Arial Narrow | Bold | Arial Narrow Bold | Arial | Narrow Bold |
| Arial Narrow Bold Italic | Arial Narrow | Bold Italic | Arial Narrow Bold Italic | Arial | Narrow Bold Italic |
| Arial | Arial | Regular | Arial | Arial |  |
| Arial Italic | Arial | Italic | Arial Italic | Arial | Italic |
| Arial Bold | Arial | Bold | Arial Bold | Arial | Bold |
| Arial Bold Italic | Arial | Bold Italic | Arial Bold Italic | Arial | Bold Italic |
| Arial Black | Arial Black | Regular | Arial Black | Arial | Black |
| Arial Black Italic | Arial Black | Italic | Arial Black Italic | Arial | Black Italic |

In addition to name strings, OS/2.usWeightClass, OS/2.usWidthClass, OS/2.fsSelection style bits, and head.macStyle bits are shown. These settings allow the fonts to fit together into a single family of varying weight and compression/expansion.

| Font | OS/2 usWeightClass | OS/2 usWidthClass | OS/2 fsSelection Italic | OS/2 fsSelection Bold | OS/2 fsSelection Regular | head macStyle Bold | head macStyle Italic | head macStyle Condensed | head macStyle Extended |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arial Narrow | 400 | 3 |  |  | x |  |  | x |  |
| Arial Narrow Italic | 400 | 3 | x |  |  |  | x | x |  |
| Arial Narrow Bold | 700 | 3 |  | x |  | x |  | x |  |
| Arial Narrow Bold Italic | 700 | 3 | x | x |  | x | x | x |  |
| Arial | 400 | 5 |  |  | x |  |  |  |  |
| Arial Italic | 400 | 5 | x |  |  |  | x |  |  |
| Arial Bold | 700 | 5 |  | x |  | x |  |  |  |
| Arial Bold Italic | 700 | 5 | x | x |  | x | x |  |  |
| Arial Black | 900 | 5 |  | x |  | x |  |  |  |
| Arial Black Italic | 900 | 5 | x | x |  | x | x |  |  |

The following are examples of how these strings might be defined, based on Times New Roman Bold:

0. The copyright string from the font vendor. © Copyright the Monotype Corporation plc, 1990

1. The name the user sees. Times New Roman

2. The name of the style. Bold

3. A unique identifier that applications can store to identify the font being used. Monotype: Times New Roman Bold: 1990

4. The complete, unique, human readable name of the font. This name is used by Windows. Times New Roman Bold

5. Release and version information from the font vendor. Version 1.00 June 1, 1990, initial release

6. The name the font will be known by on a PostScript printer. TimesNewRoman-Bold

7. Trademark string. Times New Roman is a registered trademark of the Monotype Corporation.

8. Manufacturer. Monotype Corporation

9. Designer. Stanley Morison

10. Description. Designed in 1932 for the Times of London newspaper. Excellent readability and a narrow overall width, allowing more words per line than most fonts.

11. URL of Vendor. http://www.monotype.com

12. URL of Designer. http://www.monotype.com

13. License Description. This font may be installed on all of your machines and printers, but you may not sell or give these fonts to anyone else.

14. License Info URL. http://www.monotype.com/license/

15. Reserved.

16. Preferred Family. No name string present, since it is the same as name ID 1 (Font Family name).

17. Preferred Subfamily. No name string present, since it is the same as name ID 2 (Font Subfamily name).

18. Compatible Full (Macintosh only). No name string present, since it is the same as name ID 4 (Full name).

19. Sample text. The quick brown fox jumps over the lazy dog.

20. PostScript CID findfont name. No name string present. Thus, the PostScript Name defined by name ID 6 should be used with the "findfont" invocation for locating the font in the context of a PostScript interpreter.

21. WWS family name: Since Times New Roman is a WWS font, this field does not need to be specified. If the font contained styles such as "caption", "display", "handwriting", etc., that would be noted here.

22. WWS subfamily name: Since Times New Roman is a WWS font, this field does not need to be specified.

23. Light background palette name. No name string present, since this is not a color font.

24. Dark background palette name. No name string present, since this is not a color font.

25. Variations PostScript name prefix. No name string present, since this is not a variable font.

The following is an example of only name IDs 6 and 20 in the CFF OFF Japanese font Kozuka Mincho Std Regular (other name IDs are also present in this font):

6. PostScript name: KozMinStd-Regular. Since a name ID 20 is present in the font (see below), then the PostScript name defined by name ID 6 should be used with the "composefont" invocation for locating the font in the context of a PostScript interpreter.

20. PostScript CID findfont name: KozMinStd-Regular-83pv-RKSJ-H, in a name record of Platform 1 [Macintosh], Platform-specific script 1 [Japanese], Language: 0xFFFF [English]. This name string is a PostScript name that should be used with the "findfont" invocation for locating the font in the context of a PostScript interpreter, and is associated with the encoding specified by the following 'cmap' subtable, which must be present in the font: Platform: 1 [Macintosh]; Platform-specific encoding: 1 [Japanese]; Language: 0 [not language-specific].

The following is an example of family/subfamily naming for an extended, WWS-only family. Consider Adobe Caslon Pro, with six members: upright and italic versions of regular, semibold and bold weights. (Bit 8 of the fsSelection field of the OS/2 table, version 4, should be set for all six fonts, and none should include ‘name’ entries for IDs 21 or 22.)

Adobe Caslon Pro Regular: 
Name ID 1: Adobe Caslon Pro 
Name ID 2: Regular

Adobe Caslon Pro Italic: 
Name ID 1: Adobe Caslon Pro 
Name ID 2: Italic

Adobe Caslon Pro Semibold: 
Name ID 1: Adobe Caslon Pro 
Name ID 2: Bold 
Name ID 16: Adobe Caslon Pro 
Name ID 17: Semibold

Adobe Caslon Pro Semibold Italic: 
Name ID 1: Adobe Caslon Pro 
Name ID 2: Bold Italic 
Name ID 16: Adobe Caslon Pro 
Name ID 17: Semibold Italic

Adobe Caslon Pro Bold: 
Name ID 1: Adobe Caslon Pro Bold 
Name ID 2: Regular 
Name ID 16: Adobe Caslon Pro 
Name ID 17: Bold

Adobe Caslon Pro Bold Italic: 
Name ID 1: Adobe Caslon Pro Bold 
Name ID 2: Italic 
Name ID 16: Adobe Caslon Pro 
Name ID 17: Bold Italic

The following is an example of family/subfamily naming for an extended, non-WWS family. Consider Minion Pro Opticals, with 32 member fonts: upright and italic versions of regular, medium, semibold and bold weights in each of four optical sizes: regular, caption, display and subhead. The following show names for a sampling of the fonts in this family. (Bit 8 of the fsSelection field in the OS/2 table, version 4, should be set in those fonts that do not include ‘name’ entries for IDs 21 or 22, and only in those fonts.)

Minion Pro Regular: 
Name ID 1: Minion Pro 
Name ID 2: Regular

Minion Pro Italic: 
Name ID 1: Minion Pro 
Name ID 2: Italic

Minion Pro Semibold: 
Name ID 1: Minion Pro SmBd 
Name ID 2: Regular 
Name ID 16: Minion Pro 
Name ID 17: Semibold

Minion Pro Semibold Italic: 
Name ID 1: Minion Pro SmBd 
Name ID 2: Italic 
Name ID 16: Minion Pro 
Name ID 17: Semibold Italic

Minion Pro Caption: 
Name ID 1: Minion Pro Capt 
Name ID 2: Regular 
Name ID 16: Minion Pro 
Name ID 17: Caption 
Name ID 21: Minion Pro Caption 
Name ID 22: Regular

Minion Pro Semibold Italic Caption: 
Name ID 1: Minion Pro SmBd Capt 
Name ID 2: Italic 
Name ID 16: Minion Pro 
Name ID 17: Semibold Italic Caption 
Name ID 21: Minion Pro Caption 
Name ID 22: Semibold Italic

#### Platform, encoding and language IDs

The following sections provide details regarding platform IDs, platform-specific encoding IDs, and platform-specific language IDs used in the 'name' table. For details regarding platform, encoding or language IDs used in the 'cmap' table, see subclause 5.1.2.1.

Platform IDs

The following platform IDs may be used in the 'name' table:

| Platform ID | Platform name | Platform-specific encoding IDs | Language IDs |
| --- | --- | --- | --- |
| 0 | Unicode | Various | None |
| 1 | Macintosh | Script manager code | Various |
| 3 | Windows | Windows encoding | Various |

Note that other platform IDs have been defined for use only in the 'cmap' table.

Unicode platform (platform ID = 0)

The following encoding IDs for the Unicode platform may be used in the 'name' table:

| Encoding ID | Description |
| --- | --- |
| 0 | Unicode 1.0 semantics—deprecated |
| 1 | Unicode 1.1 semantics—deprecated |
| 2 | ISO/IEC 10646 semantics—deprecated |
| 3 | Unicode 2.0 and onwards semantics, Unicode BMP only |
| 4 | Unicode 2.0 and onwards semantics, Unicode full repertoire |

Use of encoding IDs 0, 1 or 2 is deprecated. Either encoding ID 3 or 4 may be used for 'name' entries.

Note that other platform IDs have been defined for use only in the 'cmap' table. Also note that a new encoding ID for the Unicode platform may sometimes be assigned when new 'cmap' subtable formats are defined, and these may also be appropriate for use in the 'name' table. For example, when 'cmap' subtable formats 10 and 12 were added to the specification, encoding ID 4 was added as well.

There are no platform-specific language IDs defined for the Unicode platform. Language ID = 0 may be used for Unicode-platform strings, but this does not indicate any particular language. Language IDs greater than or equal to 0x8000 may be used together with language-tag records, as described above.

Strings for the Unicode platform shall be encoded in UTF-16BE.

Macintosh platform (platform ID = 1)

Macintosh encoding IDs (script manager codes)

The following encoding IDs are defined for use with the Macintosh platform:

| Encoding ID | Script | Encoding ID | Script |
| --- | --- | --- | --- |
| 0 | Roman | 17 | Malayalam |
| 1 | Japanese | 18 | Sinhalese |
| 2 | Chinese (Traditional) | 19 | Burmese |
| 3 | Korean | 20 | Khmer |
| 4 | Arabic | 21 | Thai |
| 5 | Hebrew | 22 | Laotian |
| 6 | Greek | 23 | Georgian |
| 7 | Russian | 24 | Armenian |
| 8 | RSymbol | 25 | Chinese (Simplified) |
| 9 | Devanagari | 26 | Tibetan |
| 10 | Gurmukhi | 27 | Mongolian |
| 11 | Gujarati | 28 | Geez |
| 12 | Oriya | 29 | Slavic |
| 13 | Bengali | 30 | Vietnamese |
| 14 | Tamil | 31 | Sindhi |
| 15 | Telugu | 32 | Uninterpreted |
| 16 | Kannada |  |  |

Strings for the Macintosh platform (platform ID 1) use platform-specific single- or double-byte encodings according to the specified encoding ID for a given name record.

Macintosh language IDs

The following are platform-specific language IDs assigned by Apple:

| Language ID | Language | Language ID | Language |
| --- | --- | --- | --- |
| 0 | English | 59 | Pashto |
| 1 | French | 60 | Kurdish |
| 2 | German | 61 | Kashmiri |
| 3 | Italian | 62 | Sindhi |
| 4 | Dutch | 63 | Tibetan |
| 5 | Swedish | 64 | Nepali |
| 6 | Spanish | 65 | Sanskrit |
| 7 | Danish | 66 | Marathi |
| 8 | Portuguese | 67 | Bengali |
| 9 | Norwegian | 68 | Assamese |
| 10 | Hebrew | 69 | Gujarati |
| 11 | Japanese | 70 | Punjabi |
| 12 | Arabic | 71 | Oriya |
| 13 | Finnish | 72 | Malayalam |
| 14 | Greek | 73 | Kannada |
| 15 | Icelandic | 74 | Tamil |
| 16 | Maltese | 75 | Telugu |
| 17 | Turkish | 76 | Sinhalese |
| 18 | Croatian | 77 | Burmese |
| 19 | Chinese (Traditional) | 78 | Khmer |
| 20 | Urdu | 79 | Lao |
| 21 | Hindi | 80 | Vietnamese |
| 22 | Thai | 81 | Indonesian |
| 23 | Korean | 82 | Tagalog |
| 24 | Lithuanian | 83 | Malay (Roman script) |
| 25 | Polish | 84 | Malay (Arabic script) |
| 26 | Hungarian | 85 | Amharic |
| 27 | Estonian | 86 | Tigrinya |
| 28 | Latvian | 87 | Galla |
| 29 | Sami | 88 | Somali |
| 30 | Faroese | 89 | Swahili |
| 31 | Farsi/Persian | 90 | Kinyarwanda/Ruanda |
| 32 | Russian | 91 | Rundi |
| 33 | Chinese (Simplified) | 92 | Nyanja/Chewa |
| 34 | Flemish | 93 | Malagasy |
| 35 | Irish Gaelic | 94 | Esperanto |
| 36 | Albanian | 128 | Welsh |
| 37 | Romanian | 129 | Basque |
| 38 | Czech | 130 | Catalan |
| 39 | Slovak | 131 | Latin |
| 40 | Slovenian | 132 | Quechua |
| 41 | Yiddish | 133 | Guarani |
| 42 | Serbian | 134 | Aymara |
| 43 | Macedonian | 135 | Tatar |
| 44 | Bulgarian | 136 | Uighur |
| 45 | Ukrainian | 137 | Dzongkha |
| 46 | Byelorussian | 138 | Javanese (Roman script) |
| 47 | Uzbek | 139 | Sundanese (Roman script) |
| 48 | Kazakh | 140 | Galician |
| 49 | Azerbaijani (Cyrillic script) | 141 | Afrikaans |
| 50 | Azerbaijani (Arabic script) | 142 | Breton |
| 51 | Armenian | 143 | Inuktitut |
| 52 | Georgian | 144 | Scottish Gaelic |
| 53 | Moldavian | 145 | Manx Gaelic |
| 54 | Kirghiz | 146 | Irish Gaelic (with dot above) |
| 55 | Tajiki | 147 | Tongan |
| 56 | Turkmen | 148 | Greek (polytonic) |
| 57 | Mongolian (Mongolian script) | 149 | Greenlandic |
| 58 | Mongolian (Cyrillic script) | 150 | Azerbaijani (Roman script) |

Windows platform (platform ID= 3)

Windows encoding IDs

The following encoding IDs are defined for use with the Windows platform:

| Encoding ID | Description |
| --- | --- |
| 0 | Symbol |
| 1 | Unicode BMP |
| 2 | ShiftJIS |
| 3 | PRC |
| 4 | Big5 |
| 5 | Wansung |
| 6 | Johab |
| 7 | Reserved |
| 8 | Reserved |
| 9 | Reserved |
| 10 | Unicode full repertoire |

Encoding IDs for platform 3 'name' entries should match the encoding IDs used for platform 3 subtables in the 'cmap' table. When building a Unicode font for Windows, the platform ID should be 3 and the encoding ID should be 1. When building a symbol font for Windows, the platform ID should be 3 and the encoding ID should be 0. If a font has records for encoding IDs 3, 4 or 5, the corresponding string data should be encoded using code pages 936, 950 and 949, respectively. Otherwise, all string data for platform 3 must be encoded in UTF-16BE.

NOTE	Some legacy Traditional Chinese fonts had name entries for platform 3, encoding ID 4 (Big5) with some string data encoded using code page 950 but with the string data for name ID 2 (font subfamily) encoded, instead, in UTF-16BE. For example, this was the case in the MingLi font included in the Traditional Chinese edition of Windows 95. Some older software implementations, including Windows GDI, allow for this exception.

Windows language IDs

The following are platform-specific language IDs assigned by Microsoft:

| Primary Language | Region | Language ID (hexadecimal) |
| --- | --- | --- |
| Afrikaans | South Africa | 0436 |
| Albanian | Albania | 041C |
| Alsatian | France | 0484 |
| Amharic | Ethiopia | 045E |
| Arabic | Algeria | 1401 |
| Arabic | Bahrain | 3C01 |
| Arabic | Egypt | 0C01 |
| Arabic | Iraq | 0801 |
| Arabic | Jordan | 2C01 |
| Arabic | Kuwait | 3401 |
| Arabic | Lebanon | 3001 |
| Arabic | Libya | 1001 |
| Arabic | Morocco | 1801 |
| Arabic | Oman | 2001 |
| Arabic | Qatar | 4001 |
| Arabic | Saudi Arabia | 0401 |
| Arabic | Syria | 2801 |
| Arabic | Tunisia | 1C01 |
| Arabic | U.A.E. | 3801 |
| Arabic | Yemen | 2401 |
| Armenian | Armenia | 042B |
| Assamese | India | 044D |
| Azeri (Cyrillic) | Azerbaijan | 082C |
| Azeri (Latin) | Azerbaijan | 042C |
| Bashkir | Russia | 046D |
| Basque | Basque | 042D |
| Belarusian | Belarus | 0423 |
| Bengali | Bangladesh | 0845 |
| Bengali | India | 0445 |
| Bosnian (Cyrillic) | Bosnia and Herzegovina | 201A |
| Bosnian (Latin) | Bosnia and Herzegovina | 141A |
| Breton | France | 047E |
| Bulgarian | Bulgaria | 0402 |
| Catalan | Catalan | 0403 |
| Chinese | Hong Kong S.A.R. | 0C04 |
| Chinese | Macao S.A.R. | 1404 |
| Chinese | People's Republic of China | 0804 |
| Chinese | Singapore | 1004 |
| Chinese | Taiwan | 0404 |
| Corsican | France | 0483 |
| Croatian | Croatia | 041A |
| Croatian (Latin) | Bosnia and Herzegovina | 101A |
| Czech | Czech Republic | 0405 |
| Danish | Denmark | 0406 |
| Dari | Afghanistan | 048C |
| Divehi | Maldives | 0465 |
| Dutch | Belgium | 0813 |
| Dutch | Netherlands | 0413 |
| English | Australia | 0C09 |
| English | Belize | 2809 |
| English | Canada | 1009 |
| English | Caribbean | 2409 |
| English | India | 4009 |
| English | Ireland | 1809 |
| English | Jamaica | 2009 |
| English | Malaysia | 4409 |
| English | New Zealand | 1409 |
| English | Republic of the Philippines | 3409 |
| English | Singapore | 4809 |
| English | South Africa | 1C09 |
| English | Trinidad and Tobago | 2C09 |
| English | United Kingdom | 0809 |
| English | United States | 0409 |
| English | Zimbabwe | 3009 |
| Estonian | Estonia | 0425 |
| Faroese | Faroe Islands | 0438 |
| Filipino | Philippines | 0464 |
| Finnish | Finland | 040B |
| French | Belgium | 080C |
| French | Canada | 0C0C |
| French | France | 040C |
| French | Luxembourg | 140c |
| French | Principality of Monaco | 180C |
| French | Switzerland | 100C |
| Frisian | Netherlands | 0462 |
| Galician | Galician | 0456 |
| Georgian | Georgia | 0437 |
| German | Austria | 0C07 |
| German | Germany | 0407 |
| German | Liechtenstein | 1407 |
| German | Luxembourg | 1007 |
| German | Switzerland | 0807 |
| Greek | Greece | 0408 |
| Greenlandic | Greenland | 046F |
| Gujarati | India | 0447 |
| Hausa (Latin) | Nigeria | 0468 |
| Hebrew | Israel | 040D |
| Hindi | India | 0439 |
| Hungarian | Hungary | 040E |
| Icelandic | Iceland | 040F |
| Igbo | Nigeria | 0470 |
| Indonesian | Indonesia | 0421 |
| Inuktitut | Canada | 045D |
| Inuktitut (Latin) | Canada | 085D |
| Irish | Ireland | 083C |
| isiXhosa | South Africa | 0434 |
| isiZulu | South Africa | 0435 |
| Italian | Italy | 0410 |
| Italian | Switzerland | 0810 |
| Japanese | Japan | 0411 |
| Kannada | India | 044B |
| Kazakh | Kazakhstan | 043F |
| Khmer | Cambodia | 0453 |
| K'iche | Guatemala | 0486 |
| Kinyarwanda | Rwanda | 0487 |
| Kiswahili | Kenya | 0441 |
| Konkani | India | 0457 |
| Korean | Korea | 0412 |
| Kyrgyz | Kyrgyzstan | 0440 |
| Lao | Lao P.D.R. | 0454 |
| Latvian | Latvia | 0426 |
| Lithuanian | Lithuania | 0427 |
| Lower Sorbian | Germany | 082E |
| Luxembourgish | Luxembourg | 046E |
| Macedonian | North Macedonia | 042F |
| Malay | Brunei Darussalam | 083E |
| Malay | Malaysia | 043E |
| Malayalam | India | 044C |
| Maltese | Malta | 043A |
| Maori | New Zealand | 0481 |
| Mapudungun | Chile | 047A |
| Marathi | India | 044E |
| Mohawk | Mohawk | 047C |
| Mongolian (Cyrillic) | Mongolia | 0450 |
| Mongolian (Traditional) | People's Republic of China | 0850 |
| Nepali | Nepal | 0461 |
| Norwegian (Bokmal) | Norway | 0414 |
| Norwegian (Nynorsk) | Norway | 0814 |
| Occitan | France | 0482 |
| Odia (formerly Oriya) | India | 0448 |
| Pashto | Afghanistan | 0463 |
| Polish | Poland | 0415 |
| Portuguese | Brazil | 0416 |
| Portuguese | Portugal | 0816 |
| Punjabi | India | 0446 |
| Quechua | Bolivia | 046B |
| Quechua | Ecuador | 086B |
| Quechua | Peru | 0C6B |
| Romanian | Romania | 0418 |
| Romansh | Switzerland | 0417 |
| Russian | Russia | 0419 |
| Sami (Inari) | Finland | 243B |
| Sami (Lule) | Norway | 103B |
| Sami (Lule) | Sweden | 143B |
| Sami (Northern) | Finland | 0C3B |
| Sami (Northern) | Norway | 043B |
| Sami (Northern) | Sweden | 083B |
| Sami (Skolt) | Finland | 203B |
| Sami (Southern) | Norway | 183B |
| Sami (Southern) | Sweden | 1C3B |
| Sanskrit | India | 044F |
| Serbian (Cyrillic) | Bosnia and Herzegovina | 1C1A |
| Serbian (Cyrillic) | Serbia | 0C1A |
| Serbian (Latin) | Bosnia and Herzegovina | 181A |
| Serbian (Latin) | Serbia | 081A |
| Sesotho sa Leboa | South Africa | 046C |
| Setswana | South Africa | 0432 |
| Sinhala | Sri Lanka | 045B |
| Slovak | Slovakia | 041B |
| Slovenian | Slovenia | 0424 |
| Spanish | Argentina | 2C0A |
| Spanish | Bolivia | 400A |
| Spanish | Chile | 340A |
| Spanish | Colombia | 240A |
| Spanish | Costa Rica | 140A |
| Spanish | Dominican Republic | 1C0A |
| Spanish | Ecuador | 300A |
| Spanish | El Salvador | 440A |
| Spanish | Guatemala | 100A |
| Spanish | Honduras | 480A |
| Spanish | Mexico | 080A |
| Spanish | Nicaragua | 4C0A |
| Spanish | Panama | 180A |
| Spanish | Paraguay | 3C0A |
| Spanish | Peru | 280A |
| Spanish | Puerto Rico | 500A |
| Spanish (Modern Sort) | Spain | 0C0A |
| Spanish (Traditional Sort) | Spain | 040A |
| Spanish | United States | 540A |
| Spanish | Uruguay | 380A |
| Spanish | Venezuela | 200A |
| Swedish | Finland | 081D |
| Swedish | Sweden | 041D |
| Syriac | Syria | 045A |
| Tajik (Cyrillic) | Tajikistan | 0428 |
| Tamazight (Latin) | Algeria | 085F |
| Tamil | India | 0449 |
| Tatar | Russia | 0444 |
| Telugu | India | 044A |
| Thai | Thailand | 041E |
| Tibetan | PRC | 0451 |
| Turkish | Turkey | 041F |
| Turkmen | Turkmenistan | 0442 |
| Uighur | PRC | 0480 |
| Ukrainian | Ukraine | 0422 |
| Upper Sorbian | Germany | 042E |
| Urdu | Islamic Republic of Pakistan | 0420 |
| Uzbek (Cyrillic) | Uzbekistan | 0843 |
| Uzbek (Latin) | Uzbekistan | 0443 |
| Vietnamese | Vietnam | 042A |
| Welsh | United Kingdom | 0452 |
| Wolof | Senegal | 0488 |
| Yakut | Russia | 0485 |
| Yi | PRC | 0478 |
| Yoruba | Nigeria | 046A |

For more information on Windows language IDs and corresponding BCP 47 language tags, see Windows Language Code Identifier (LCID) Reference [1].

### OS/2 — OS/2 and Windows metrics table

The OS/2 table consists of a set of metrics and other data that are required in OFF fonts.

OS/2 Table formats

Six versions of the OS/2 table have been defined: versions 0 to 5. All versions are supported, but use of version 4 or later is strongly recommended. Descriptions of OS/2 versions 4 and earlier are provided in Annex B. The format of version 5 is as follows:

| Type | Name of Entry | Comments |
| --- | --- | --- |
| uint16 | version | 0x0005 |
| FWORD | xAvgCharWidth |  |
| uint16 | usWeightClass |  |
| uint16 | usWidthClass |  |
| uint16 | fsType |  |
| FWORD | ySubscriptXSize |  |
| FWORD | ySubscriptYSize |  |
| FWORD | ySubscriptXOffset |  |
| FWORD | ySubscriptYOffset |  |
| FWORD | ySuperscriptXSize |  |
| FWORD | ySuperscriptYSize |  |
| FWORD | ySuperscriptXOffset |  |
| FWORD | ySuperscriptYOffset |  |
| FWORD | yStrikeoutSize |  |
| FWORD | yStrikeoutPosition |  |
| int16 | sFamilyClass |  |
| uint8 | panose[10] |  |
| uint32 | ulUnicodeRange1 | Bits 0–31 |
| uint32 | ulUnicodeRange2 | Bits 32–63 version 0x0001 and later |
| uint32 | ulUnicodeRange3 | Bits 64–95 version 0x0001 and later |
| uint32 | ulUnicodeRange4 | Bits 96–127 version 0x0001 and later |
| Tag | achVendID |  |
| uint16 | fsSelection |  |
| uint16 | usFirstCharIndex |  |
| uint16 | usLastCharIndex |  |
| FWORD | sTypoAscender |  |
| FWORD | sTypoDescender |  |
| FWORD | sTypoLineGap |  |
| UFWORD | usWinAscent |  |
| UFWORD | usWinDescent |  |
| uint32 | ulCodePageRange1 | Bits 0–31 version 0x0001 and later |
| uint32 | ulCodePageRange2 | Bits 32–63 version 0x0001 and later |
| FWORD | sxHeight | version 0x0002 and later |
| FWORD | sCapHeight | version 0x0002 and later |
| uint16 | usDefaultChar | version 0x0002 and later |
| uint16 | usBreakChar | version 0x0002 and later |
| uint16 | usMaxContext | version 0x0002 and later |
| uint16 | usLowerOpticalPointSize | version 0x0005 and later |
| uint16 | usUpperOpticalPointSize | version 0x0005 and later |

OS/2 field details are provided below.

#### version

| Format: | uint16 |
| --- | --- |
| Units: | n/a |
| Title: | OS/2 table version number. |
| Description: | The version number for the OS/2 table: 0x0000 to 0x0005. |
| Comments: | The version number allows for identification of the precise contents and layout for the OS/2 table. |

#### xAvgCharWidth

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Average weighted escapement. |
| Description: | The Average Character Width field specifies the arithmetic average of the escapement (width) of all non-zero width glyphs in the font. |
| Comments: | The value for xAvgCharWidth is calculated by obtaining the arithmetic average of the width of all non-zero width glyphs in the font. It is strongly recommended that implementers do not rely on this value for computing layout for lines of text, especially for cases where complex scripts are used. NOTE 1 Some fonts have xAvgCharWidth values that do not conform to this calculation. Fonts could have been originally created with a version 0 to version 2 OS/2 table using the legacy calculation described below, then later updated with a newer OS/2 table version but without changing the xAvgCharWidth value. In some CJK fonts, the xAvgCharWidth can be almost half the value that would be expected by the above calculation. Applications should not use xAvgCharWidth for determining actual glyph advance widths. NOTE 2 The calculation used in versions 0 to 2 of the OS/2 table differs from the above. See details in Annex B. |

#### usWeightClass

| Format: | uint16 |
| --- | --- |
| Title: | Weight class. |
| Description: | Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font. Values from 1 to 1000 are valid. |
| Comments: | usWeightClass values use the same scale as the 'wght' axis that is used in the 'fvar' table (7.3.3) of variable fonts and in the 'STAT' table (7.3.7). While integer values from 1 to 1000 are supported, some legacy platforms could have limitations on supported values. The following are commonly used values: |

| Value | Description | C Definition (from windows.h) |
| --- | --- | --- |
| 100 | Thin | FW_THIN |
| 200 | Extra-light (Ultra-light) | FW_EXTRALIGHT |
| 300 | Light | FW_LIGHT |
| 400 | Normal (Regular) | FW_NORMAL |
| 500 | Medium | FW_MEDIUM |
| 600 | Semi-bold (Demi-bold) | FW_SEMIBOLD |
| 700 | Bold | FW_BOLD |
| 800 | Extra-bold (Ultra-bold) | FW_EXTRABOLD |
| 900 | Black (Heavy) | FW_BLACK |

#### usWidthClass

| Format: | uint16 |
| --- | --- |
| Title: | Width class. |
| Description: | Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font. |
| Comments: | Although every glyph in a font can have a different numeric aspect ratio, each glyph in a font of normal width is considered to have a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the glyphs in the new font is some percentage greater or less than the corresponding glyphs in the normal font—it is this difference that this field specifies. The valid usWidthClass values are shown in the following table. Note that the usWidthClass values are related to but distinct from the scale for the 'wdth' axis that is used in the 'fvar' table (7.3.3) of variable fonts and in the 'STAT' table (7.3.7). The “% of normal” column in the following table provides a mapping from usWidthClass values 1 – 9 to 'wdth' values. |

| Value | Description | C Definition | % of normal |
| --- | --- | --- | --- |
| 1 | Ultra-condensed | FWIDTH_ULTRA_CONDENSED | 50 |
| 2 | Extra-condensed | FWIDTH_EXTRA_CONDENSED | 62.5 |
| 3 | Condensed | FWIDTH_CONDENSED | 75 |
| 4 | Semi-condensed | FWIDTH_SEMI_CONDENSED | 87.5 |
| 5 | Medium (normal) | FWIDTH_NORMAL | 100 |
| 6 | Semi-expanded | FWIDTH_SEMI_EXPANDED | 112.5 |
| 7 | Expanded | FWIDTH_EXPANDED | 125 |
| 8 | Extra-expanded | FWIDTH_EXTRA_EXPANDED | 150 |
| 9 | Ultra-expanded | FWIDTH_ULTRA_EXPANDED | 200 |

#### fsType

| Format: | uint16 |
| --- | --- |
| Title: | Type flags. |
| Description: | Indicates font embedding licensing rights for the font. The interpretation of flags is as follows: |

| Bits | Mask | Description |
| --- | --- | --- |
| 0 – 3 | 0x000F | Usage permissions. Valid fonts shall set at most one of bits 1, 2 or 3; bit 0 is permanently reserved and shall be zero. Valid values for this sub-field are 0, 2, 4 or 8. The meaning of these values is as follows: 0: Installable embedding: the font may be embedded, and may be permanently installed for use on a remote systems, or for use by other users. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser. 2: Restricted License embedding: the font shall not be modified, embedded or exchanged in any manner without first obtaining explicit permission of the legal owner. 4: Preview & Print embedding: the font may be embedded, and may be temporarily loaded on other systems for purposes of viewing or printing the document. Documents containing Preview & Print fonts shall be opened “read-only”; no edits may be applied to the document. 8: Editable embedding: the font may be embedded, and may be temporarily loaded on other systems. As with Preview & Print embedding, documents containing Editable fonts may be opened for reading. In addition, editing is permitted, including ability to format new text using the embedded font, and changes may be saved. |
| 4 – 7 | 0x00F0 | Reserved, must be zero. |
| 8 | 0x0100 | No subsetting: When this bit is set, the font may not be subsetted prior to embedding. Other embedding restrictions specified in bits 0 – 3 and bit 9 also apply. |
| 9 | 0x0200 | Bitmap embedding only: When this bit is set, only bitmaps contained in the font may be embedded. No outline data may be embedded. If there are no bitmaps available in the font, then the font is considered unembeddable and the embedding services will fail. Other embedding restrictions specified in bits 0-3 and 8 also apply. |
| 10 – 15 | 0xFC00 | Reserved, must be zero. |

Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font.

Applications that implement support for font embedding shall not embed fonts which are not licensed to permit embedding. Also, when embedding a font into a document, applications shall not modify the embedding permissions and restrictions indicated in this field. In addition, applications loading embedded fonts for temporary use (Preview & Print or Editable embedding) shall delete the fonts when the document containing the embedded font is closed.

Bits 0 to 3 (the embedding permissions sub-field) are mutually exclusive: fonts should never have more than one of these bits set. Note that, if two or more bits are set, some applications could assume the least-restrictive permission indicated. (See version differences for more discussion.) Caution: Font vendors are responsible to set these bits correctly to obtain the desired application behaviors. For Restricted License embedding to take effect, the Embedding permissions sub-field shall have the value 2 (that is, only bit 1 is set).

NOTE 	Apple’s TrueType Reference Manual[7] specifies bit 1 of the fsType field, and only bit 1, as having an assigned semantic. This originated from a pre-release draft specification for the OS/2 table. However, the final specification for version 0 of the OS/2 table defined bits 0 to 3. Also, some early font implementations mistakenly used the value 1 (bit 0 set), leading to problems of non-interoperability. For this reason, bit 0 was specified as reserved in the final specification for version 0. Bit 0 is permanently reserved, and its use is deprecated.

#### ySubscriptXSize

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for subscripts for this font. Should be > 0. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal size of subscript glyphs associated with this font. If a font does not include all required subscript glyphs for an application, and the application can substitute glyphs by scaling the glyphs of a font or by substituting glyphs from another font, this field specifies the recommended nominal width for those subscript glyphs. For example, if the em for a font is 2048 units and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript glyph would be 1/10th the size of the normal glyph. |

NOTE	This was implemented in the OS/2 platform as a signed value, even though only values > 0 would be expected.

#### ySubscriptYSize

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript vertical font size. |
| Description: | The recommended vertical size in font design units for subscripts for this font. Should be > 0. |
| Comments: | If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the em size of the font being used for a subscript. The vertical font size specifies a font designer's recommendation for vertical size of subscript glyphs associated with this font. If a font does not include all required subscript glyphs for an application, and the application can substitute glyphs by scaling the glyphs in a font or by substituting glyphs from another font, this field specifies the recommended nominal height for those subscript glyphs. For example, if the em for a font is 2048 units and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript glyph would be 1/10th the size of the normal glyph. |

NOTE	This was implemented in the OS/2 platform as a signed value, even though only values > 0 would be expected.

#### ySubscriptXOffset

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript x offset. |
| Description: | The recommended horizontal offset in font design units for subscripts for this font. |
| Comments: | The ySubscriptXOffset field specifies a font designer's recommended horizontal offset—from the glyph origin to the glyph origin of the subscript’s glyph—for subscript glyphs associated with this font. If a font does not include all required subscript glyphs for an application, and the application can substitute glyphs, this field specifies the recommended horizontal position from the glyph escapement point of the last glyph before the first subscript glyph. For upright glyphs, this value is usually zero; however, if the glyphs of a font have an incline (italic or slant), the reference point for subscript glyphs is usually adjusted to compensate for the angle of incline. |

#### ySubscriptYOffset

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for subscripts for this font. |
| Comments: | The ySubscriptYOffset field specifies a font designer's recommended vertical offset from the glyph baseline to the glyph baseline for subscript glyphs associated with this font. Values are expressed as a positive offset below the glyph baseline. If a font does not include all required subscript glyphs for an application, this field specifies the recommended vertical distance below the glyph baseline for alternate subscript glyphs supplied by the application . |

#### ySuperscriptXSize

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for superscripts for this font. Should be > 0. |
| Comments: | If a font has two recommended sizes for superscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em size of the font being used for a superscript. The horizontal font size specifies a font designer's recommended horizontal size for superscript glyphs associated with this font. If a font does not include all required superscript glyphs for an application, and the application can substitute glyphs by scaling the glyphs of a font or by substituting glyphs from another font, this field specifies the recommended nominal width for those superscript glyphs. For example, if the em for a font is 2048 units and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript glyph would be 1/10th the size of the normal glyph. |

NOTE	This was implemented in the OS/2 platform as a signed value, even though only values > 0 would be expected.

#### ySuperscriptYSize

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript vertical font size. |
| Description: | The recommended vertical size in font design units for superscripts for this font. Should be > 0. |
| Comments: | If a font has two recommended sizes for superscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em size of the font being used for a superscript. The vertical font size specifies a font designer's recommended vertical size for superscript glyphs associated with this font. If a font does not include all required superscript glyphs for an application, and the application can substitute glyphs by scaling the glyphs of a font or by substituting glyphs from another font, this field specifies the recommended nominal height for those superscript glyphs. For example, if the em for a font is 2048 units and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript glyph would be 1/10th the size of the normal glyph. |

NOTE	This was implemented in the OS/2 platform as a signed value, even though only values > 0 would be expected.

#### ySuperscriptXOffset

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript x offset. |
| Description: | The recommended horizontal offset in font design units for superscripts for this font. |
| Comments: | The ySuperscriptXOffset field specifies a font designer's recommended horizontal offset—from the glyph’s origin to the superscript glyph’s origin for the superscript glyphs associated with this font. If a font does not include all required superscript glyphs for an application, this field specifies the recommended horizontal position from the escapement point of the glyph before the first superscript glyph. For upright glyphs, this value is usually zero; however, if the glyphs of a font have an incline (italic or slant), the reference point for superscript glyphs is usually adjusted to compensate for the angle of incline. |

#### ySuperscriptYOffset

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for superscripts for this font. |
| Comments: | The ySuperscriptYOffset field specifies a font designer's recommended vertical offset—from the glyph’s baseline to the superscript glyph’s baseline associated with this font. Values for this field are expressed as a positive offset above the glyph baseline. If a font does not include all required superscript glyphs for an application, this field specifies the recommended vertical distance above the glyph baseline for alternate superscript glyphs supplied by the application. |

#### yStrikeoutSize

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout size. |
| Description: | Thickness of the strikeout stroke in font design units. Should be > 0. |
| Comments: | This field should normally be the thickness of the em dash for the current font, and should also match the underline thickness, which is specified in the 'post' table (5.1.9). |

NOTE	This was implemented in the OS/2 platform as a signed value, even though only values > 0 would be expected.

#### yStrikeoutPosition

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout position. |
| Description: | The position of the top of the strikeout stroke relative to the baseline in font design units. |
| Comments: | Positive values represent distances above the baseline; negative values represent distances below the baseline. Aligning the strikeout position with the em dash is suggested. Note, however, that the strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. |

#### sFamilyClass

| Format: | int16 |
| --- | --- |
| Title: | Font-family class and subclass. |
| Description: | This field provides a classification of font-family design. |
| Comments: | The family class and subclass are assigned to a font based on the family design. This field is intended for use in selecting an alternate font when the requested font is not available. The class and subclass provide a two-level classification. The high byte of this field contains the family class, while the low byte contains the family subclass. The interpretation of subclass values depends on the class value. Registered class and subclass values were originally defined by IBM. For details on registered values, see Annex A. |

#### Panose

| Format: | uint8[10] |
| --- | --- |
| Title: | PANOSE classification number |
| International: | Additional specifications are required for PANOSE to classify non-Latin character sets. |
| Description: | This 10-byte array of numbers is used to describe the visual characteristics of a given typeface. These characteristics are then used to associate the font with other fonts of similar appearance having different names. The variables for each digit are listed below. |
| Comments: | The PANOSE definition contains ten bytes, each of which can have multiple possible values. The PANOSE values are fully described in the PANOSE Classification Metrics Guide[14]. Note that the first byte is used for a high-level classification, “Family Kind”, and that the interpretation of the remaining bytes is contingent on the value of the first byte. For example, if the Family Kind value is 2 (Latin Text), then the next byte specifies “Serif Style”; but if the Family Kind value is 3 (Latin Hand Written), then the next byte specifies “Tool Kind”. Some applications might support only certain Family Kind values. The following table gives the interpretation for the panose array when the Family Kind is Latin Text: |

| Type | Name |
| --- | --- |
| uint8 | bFamilyType |
| uint8 | bSerifStyle |
| uint8 | bWeight |
| uint8 | bProportion |
| uint8 | bContrast |
| uint8 | bStrokeVariation |
| uint8 | bArmStyle |
| uint8 | bLetterform |
| uint8 | bMidline |
| uint8 | bXHeight |

Some applications can use the PANOSE values for font selection, to select a font matching certain parameters. For example, Proportion (for Family Kind = Latin Text) might be used to determine if a font is monospaced; or Serif Style might be use to determine if a font falls into generic serif or sans serif classes. Some applications will use Family Kind = 5 (Latin Symbol) to identify symbol fonts, which might affect font selection or fallback behaviors. There are no requirements for how applications should use the PANOSE values.

In a variable font that uses OpenType Font Variation mechanisms, there is no way to represent different PANOSE values for different instances supported by the font. The PANOSE values can be set based on the default instance.

#### UnicodeRange

ulUnicodeRange1 (Bits 0 – 31)

ulUnicodeRange2 (Bits 32 – 63)

ulUnicodeRange3 (Bits 64 – 95)

ulUnicodeRange4 (Bits 96 – 127)

| Format: | uint32[4] (128 bits total) |
| --- | --- |
| Title: | Unicode Character Range |
| Description: | This field is used to specify the Unicode blocks or ranges encompassed by the font file in 'cmap' subtables for platform 3, encoding ID 1 (Microsoft platform, Unicode BMP) and platform 3, encoding ID 10 (Microsoft platform, Unicode full repertoire). If a bit is set (1), then the Unicode ranges assigned to that bit are considered functional. If the bit is clear (0), then the range is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by ranges if at all possible. All reserved fields must be zero. Each uint32 is in Big-Endian form. |

| Bit | Unicode Range | Block range |
| --- | --- | --- |
| 0 | Basic Latin | 0000-007F |
| 1 | Latin-1 Supplement | 0080-00FF |
| 2 | Latin Extended-A | 0100-017F |
| 3 | Latin Extended-B | 0180-024F |
| 4 | IPA Extensions | 0250-02AF |
|  | Phonetic Extensions | 1D00-1D7F |
|  | Phonetic Extensions Supplement | 1D80-1DBF |
| 5 | Spacing Modifier Letters | 02B0-02FF |
|  | Modifier Tone Letters | A700-A71F |
| 6 | Combining Diacritical Marks | 0300-036F |
|  | Combining Diacritical Marks Supplement | 1DC0-1DFF |
| 7 | Greek and Coptic | 0370-03FF |
| 8 | Coptic | 2C80-2CFF |
| 9 | Cyrillic | 0400-04FF |
|  | Cyrillic Supplement | 0500-052F |
|  | Cyrillic Extended-A | 2DE0-2DFF |
|  | Cyrillic Extended-B | A640-A69F |
| 10 | Armenian | 0530-058F |
| 11 | Hebrew | 0590-05FF |
| 12 | Vai | A500-A63F |
| 13 | Arabic | 0600-06FF |
|  | Arabic Supplement | 0750-077F |
| 14 | NKo | 07C0-07FF |
| 15 | Devanagari | 0900-097F |
| 16 | Bengali | 0980-09FF |
| 17 | Gurmukhi | 0A00-0A7F |
| 18 | Gujarati | 0A80-0AFF |
| 19 | Oriya | 0B00-0B7F |
| 20 | Tamil | 0B80-0BFF |
| 21 | Telugu | 0C00-0C7F |
| 22 | Kannada | 0C80-0CFF |
| 23 | Malayalam | 0D00-0D7F |
| 24 | Thai | 0E00-0E7F |
| 25 | Lao | 0E80-0EFF |
| 26 | Georgian | 10A0-10FF |
|  | Georgian Supplement | 2D00-2D2F |
| 27 | Balinese | 1B00-1B7F |
| 28 | Hangul Jamo | 1100-11FF |
| 29 | Latin Extended Additional | 1E00-1EFF |
|  | Latin Extended-C | 2C60-2C7F |
|  | Latin Extended-D | A720-A7FF |
| 30 | Greek Extended | 1F00-1FFF |
| 31 | General Punctuation | 2000-206F |
|  | Supplemental Punctuation | 2E00-2E7F |
| 32 | Superscripts And Subscripts | 2070-209F |
| 33 | Currency Symbols | 20A0-20CF |
| 34 | Combining Diacritical Marks For Symbols | 20D0-20FF |
| 35 | Letterlike Symbols | 2100-214F |
| 36 | Number Forms | 2150-218F |
| 37 | Arrows | 2190-21FF |
|  | Supplemental Arrows-A | 27F0-27FF |
|  | Supplemental Arrows-B | 2900-297F |
|  | Miscellaneous Symbols and Arrows | 2B00-2BFF |
| 38 | Mathematical Operators | 2200-22FF |
|  | Supplemental Mathematical Operators | 2A00-2AFF |
|  | Miscellaneous Mathematical Symbols-A | 27C0-27EF |
|  | Miscellaneous Mathematical Symbols-B | 2980-29FF |
| 39 | Miscellaneous Technical | 2300-23FF |
| 40 | Control Pictures | 2400-243F |
| 41 | Optical Character Recognition | 2440-245F |
| 42 | Enclosed Alphanumerics | 2460-24FF |
| 43 | Box Drawing | 2500-257F |
| 44 | Block Elements | 2580-259F |
| 45 | Geometric Shapes | 25A0-25FF |
| 46 | Miscellaneous Symbols | 2600-26FF |
| 47 | Dingbats | 2700-27BF |
| 48 | CJK Symbols And Punctuation | 3000-303F |
| 49 | Hiragana | 3040-309F |
| 50 | Katakana | 30A0-30FF |
|  | Katakana Phonetic Extensions | 31F0-31FF |
| 51 | Bopomofo | 3100-312F |
|  | Bopomofo Extended | 31A0-31BF |
| 52 | Hangul Compatibility Jamo | 3130-318F |
| 53 | Phags-pa | A840-A87F |
| 54 | Enclosed CJK Letters And Months | 3200-32FF |
| 55 | CJK Compatibility | 3300-33FF |
| 56 | Hangul Syllables | AC00-D7AF |
| 57 | Non-Plane 0 a | 10000-10FFFF |
| 58 | Phoenician | 10900-1091F |
| 59 | CJK Unified Ideographs | 4E00-9FFF |
|  | CJK Radicals Supplement | 2E80-2EFF |
|  | Kangxi Radicals | 2F00-2FDF |
|  | Ideographic Description Characters | 2FF0-2FFF |
|  | CJK Unified Ideographs Extension A | 3400-4DBF |
|  | CJK Unified Ideographs Extension B | 20000-2A6DF |
|  | Kanbun | 3190-319F |
| 60 | Private Use Area (plane 0) | E000-F8FF |
| 61 | CJK Strokes | 31C0-31EF |
|  | CJK Compatibility Ideographs | F900-FAFF |
|  | CJK Compatibility Ideographs Supplement | 2F800-2FA1F |
| 62 | Alphabetic Presentation Forms | FB00-FB4F |
| 63 | Arabic Presentation Forms-A | FB50-FDFF |
| 64 | Combining Half Marks | FE20-FE2F |
| 65 | Vertical Forms | FE10-FE1F |
|  | CJK Compatibility Forms | FE30-FE4F |
| 66 | Small Form Variants | FE50-FE6F |
| 67 | Arabic Presentation Forms-B | FE70-FEFF |
| 68 | Halfwidth And Fullwidth Forms | FF00-FFEF |
| 69 | Specials | FFF0-FFFF |
| 70 | Tibetan | 0F00-0FFF |
| 71 | Syriac | 0700-074F |
| 72 | Thaana | 0780-07BF |
| 73 | Sinhala | 0D80-0DFF |
| 74 | Myanmar | 1000-109F |
| 75 | Ethiopic | 1200-137F |
|  | Ethiopic Supplement | 1380-139F |
|  | Ethiopic Extended | 2D80-2DDF |
| 76 | Cherokee | 13A0-13FF |
| 77 | Unified Canadian Aboriginal Syllabics | 1400-167F |
| 78 | Ogham | 1680-169F |
| 79 | Runic | 16A0-16FF |
| 80 | Khmer | 1780-17FF |
|  | Khmer Symbols | 19E0-19FF |
| 81 | Mongolian | 1800-18AF |
| 82 | Braille Patterns | 2800-28FF |
| 83 | Yi Syllables | A000-A48F |
|  | Yi Radicals | A490-A4CF |
| 84 | Tagalog | 1700-171F |
|  | Hanunoo | 1720-173F |
|  | Buhid | 1740-175F |
|  | Tagbanwa | 1760-177F |
| 85 | Old Italic | 10300-1032F |
| 86 | Gothic | 10330-1034F |
| 87 | Deseret | 10400-1044F |
| 88 | Byzantine Musical Symbols | 1D000-1D0FF |
|  | Musical Symbols | 1D100-1D1FF |
|  | Ancient Greek Musical Notation | 1D200-1D24F |
| 89 | Mathematical Alphanumeric Symbols | 1D400-1D7FF |
| 90 | Private Use (plane 15) | F0000-FFFFD |
|  | Private Use (plane 16) | 100000-10FFFD |
| 91 | Variation Selectors | FE00-FE0F |
|  | Variation Selectors Supplement | E0100-E01EF |
| 92 | Tags | E0000-E007F |
| 93 | Limbu | 1900-194F |
| 94 | Tai Le | 1950-197F |
| 95 | New Tai Lue | 1980-19DF |
| 96 | Buginese | 1A00-1A1F |
| 97 | Glagolitic | 2C00-2C5F |
| 98 | Tifinagh | 2D30-2D7F |
| 99 | Yijing Hexagram Symbols | 4DC0-4DFF |
| 100 | Syloti Nagri | A800-A82F |
| 101 | Linear B Syllabary | 10000-1007F |
|  | Linear B Ideograms | 10080-100FF |
|  | Aegean Numbers | 10100-1013F |
| 102 | Ancient Greek Numbers | 10140-1018F |
| 103 | Ugaritic | 10380-1039F |
| 104 | Old Persian | 103A0-103DF |
| 105 | Shavian | 10450-1047F |
| 106 | Osmanya | 10480-104AF |
| 107 | Cypriot Syllabary | 10800-1083F |
| 108 | Kharoshthi | 10A00-10A5F |
| 109 | Tai Xuan Jing Symbols | 1D300-1D35F |
| 110 | Cuneiform | 12000-123FF |
|  | Cuneiform Numbers and Punctuation | 12400-1247F |
| 111 | Counting Rod Numerals | 1D360-1D37F |
| 112 | Sundanese | 1B80-1BBF |
| 113 | Lepcha | 1C00-1C4F |
| 114 | Ol Chiki | 1C50-1C7F |
| 115 | Saurashtra | A880-A8DF |
| 116 | Kayah Li | A900-A92F |
| 117 | Rejang | A930-A95F |
| 118 | Cham | AA00-AA5F |
| 119 | Ancient Symbols | 10190-101CF |
| 120 | Phaistos Disc | 101D0-101FF |
| 121 | Carian | 102A0-102DF |
|  | Lycian | 10280-1029F |
|  | Lydian | 10920-1093F |
| 122 | Domino Tiles | 1F030-1F09F |
|  | Mahjong Tiles | 1F000-1F02F |
| 123-127 | Reserved |  |  |  |
| a Setting bit 57 implies there is at least one codepoint beyond the Basic Multilingual Plane supported by this font. |  |  |

NOTE 	All available bits were exhausted as of Unicode 5.1. The bit assignments were last updated for OS/2 version 4. There are many additional ranges supported in the current version of Unicode that are not supported by this field in the OS/2 table. See the 'dlng' and 'slng' tags in the 'meta' table (5.7.6) for an alternate mechanism to declare what scripts or languages a font can support or is designed for.

#### achVendID

| Format: | Tag |
| --- | --- |
| Title: | Font Vendor Identification |
| Description: | The four character identifier for the vendor of the given type face. |
| Comments: | This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. For example, there can be multiple vendors of ITC Zapf Dingbats, with some vendors providing differentiating benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor’s type to be used over another, possibly inferior, font file. A registry of vendor IDs is maintained by Microsoft. Registered IDs shall be unique to a single vendor. Non-registered IDs may also be used, but are discouraged: vendors are strongly encouraged to register an ID to ensure that there are no conflicts between different vendors in use of a given ID, and that customers are able to find vendor contact information for a given font. This field can also be left blank (set to null, or a tag comprised of four space characters). All vendor IDs use the Tag data type, which is equivalent to a four-character string composed of a limited set of ASCII characters. For details regarding the Tag data type, see 4.3. By convention, only registered tags should be comprised of only uppercase letters (or space). For a list of registered Vendor IDs, or for details on registering a vendor ID or updating vendor information, see Reference [6]. |

#### fsSelection

| Format: | uint16 |
| --- | --- |
| Title: | Font selection flags. |
| Description: | Contains information concerning the nature of the font patterns, as follows: |

| Bit # | macStyle bit | C definition | Description |
| --- | --- | --- | --- |
| 0 | bit 1 | ITALIC | Font contains italic or oblique glyphs, otherwise they are upright. |
| 1 |  | UNDERSCORE | Glyphs are underscored. |
| 2 |  | NEGATIVE | Glyphs have their foreground and background reversed. |
| 3 |  | OUTLINED | Outline (hollow) glyphs, otherwise they are solid. |
| 4 |  | STRIKEOUT | Glyphs are overstruck. |
| 5 | bit 0 | BOLD | Glyphs are emboldened. |
| 6 |  | REGULAR | Glyphs are in the standard weight/style for the font. |
| 7 |  | USE_TYPO_METRICS | If set, it is strongly recommended that applications use OS/2.sTypoAscender - OS/2.sTypoDescender + OS/2.sTypoLineGap as the default line spacing for this font. (OS/2 version 4 and later) |
| 8 |  | WWS | The font has 'name' table strings consistent with a weight/width/slope family without requiring use of name IDs 21 and 22. (See a more detailed description below.) (OS/2 version 4 and later) |
| 9 |  | OBLIQUE | Font contains oblique glyphs. (OS/2 version 4 and later) |
| 10 – 15 |  | <reserved> | Reserved; set to 0. |

| Comments: | All undefined bits must be zero. The fsSelection field was originally created for compatibility with the OS/2 platform. Some style description bits, such as bit 0, are used by modern applications, while other style description bits are not. Bit 0: The setting of bits 0 shall match the setting of bit 1 in the macStyle field of the 'head' table (5.1.3). Bits 1 – 4: Bits 1 – 4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. Bit 5: The setting of bit 5 shall match the settings of bit 0 in the macStyle field of the 'head' table (5.1.3). Bit 6: Bit 6 is not used in widely used in modern applications. If bit 6 is set, then bits 0 and 5 shall be clear, else the behavior is undefined. Note that, if both bit 0 and bit 5 are clear, that does not give any indication as to whether or not bit 6 will be clear. For example, Arial Light is not the regular style of Arial and would have all bits cleared. In extended font families, bit 6 does not need to be set for fonts other than the regular style that, nonetheless, use "Regular" for name ID 2. (See 5.1.7.4 for more information.) Bit 7: Bit 7 was defined in version 4 and is used in many modern applications. For new fonts, vendors are encouraged to use a version 4 or later OS/2 table and to have bit 7 set. If a font was created with an earlier version of the OS/2 table and is updated to the current version of the OS/2 table, then setting bit 7 could create potential for reflow of existing documents which use the fonts. To minimize such risk, the bit would be set only if using the OS/2.usWin* metrics for line height would yield significantly inferior results than using the OS/2.sTypo* values. Bit 8: If bit 8 is set, then 'name' table strings for family and subfamily are provided that are consistent with a weight/width/slope family model without requiring the use of name IDs 21 or 22. Many typographic families contain faces that differ only in one or more of the attributes weight, width and slope. Even though a family might have a large number of member faces, if the variations are in these attributes only, then family and subfamily names provided in the 'name' table using IDs 1 and 2 or 16 and 17 will be consistent with a weight/width/slope family model. In this case, bit 8 should be set, and 'name' entries for name IDs 21 and 22 should not be included. Some typographic families include faces that differ in attributes other than weight, width or slope. For example, a family might include variations for “handwriting”, “caption”, “display”, “optical size”, etc. In this case, some of the member faces could differ from the Regular face only in weight, width or slope attributes, while other members will differ in relation to other attributes. Fonts for those member faces that differ from Regular only in weight, width or slope should have bit 8 set, and should not use name ID 21 or 22. But the fonts for those member faces that differ from Regular in terms of other attributes should not have bit 8 set, and they should use name IDs 21 and 22 to map these faces into a WWS-conformant family model. Thus, if a font has a version 4 or later OS/2 table, bit 8 should be set if and only if 'name' entries for IDs 16 and 17 are consistent with the WWS model and entries for IDs 21 and 22 are not included. Conversely, if bit 8 is not set, that will be interpreted to mean that the names provided by IDs 16 and 17 are not consistent with the WWS model and that 'name' entries for IDs 21 and 22 are included. In this context, “typographic family” is the Microsoft Unicode string for name ID 16, if present, else the Microsoft Unicode string for name ID 1; “weight” is OS/2.usWeightClass; “width” is OS/2.usWidthClass; “slope” is OS/2.fsSelection bit 0 (ITALIC) and bit 9 (OBLIQUE). Bit 9: If bit 9 is set, then this font is to be considered an “oblique” style by processes which make a distinction between oblique and italic styles, such as Cascading Style Sheets font matching. For example, a font created by algorithmically slanting an upright face will set this bit. If a font has a version 4 or later OS/2 table and this bit is not set, then this font is not to be considered an “oblique” style. For example, a font that has a classic italic design will not set this bit. This bit, unlike the ITALIC bit (bit 0), is not related to style-linking in applications that assume a four-member font-family model comprised of regular, italic, bold and bold italic. It may be set or unset independently of the ITALIC bit. In most cases, if OBLIQUE is set, then ITALIC will also be set, though this is not required. Bit 15: Bit 15 is permanently reserved. It has been used in some legacy implementations and could result in special behavior in some implementations. Use of this bit is deprecated. |
| --- | --- |

#### usFirstCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF if the minimum index value is a supplementary character. |

#### usLastCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF. |

#### sTypoAscender

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Description: | The typographic ascender for this font. This field should be combined with the sTypoDescender and sTypoLineGap values to determine default line spacing. This field is similar to the ascender field in the 'hhea' table (5.1.4) as well as to the usWinAscent field in this table. However, legacy platform implementations used those fields with platform-specific behaviors. As a result, those fields are constrained by backward-compatibility requirements, and they do not ensure consistent layout across implementations. The sTypoAscender, sTypoDescender and sTypoLineGap fields are intended to allow applications to lay out documents in a typographically-correct and portable fashion. The USE_TYPO_METRICS flag (bit 7) of the fsSelection field is used to choose between using sTypo* values or usWin* values for default line metrics. See fsSelection for additional details. It is not a general requirement that sTypoAscender - sTypoDescender be equal to unitsPerEm. These values should be set to provide default line spacing appropriate for the primary languages the font is designed to support. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical (as well as horizontal) layout, the required value for sTypoAscender is that which describes the top of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000 × 1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the recommendations in 8.20 for more on this field. |

#### sTypoDescender

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Description: | The typographic descender for this font. This field should be combined with the sTypoAscender and sTypoLineGap values to determine default line spacing. This field is similar to the descender field in the 'hhea' table (5.1.4) as well as to the usWinDescent field in this table. However, legacy platform implementations used those fields with platform-specific behaviors. As a result, those fields are constrained by backward-compatibility requirements, and they do not ensure consistent layout across implementations. The sTypoAscender, sTypoDescender and sTypoLineGap fields are intended to allow applications to lay out documents in a typographically-correct and portable fashion. The USE_TYPO_METRICS flag (bit 7) of the fsSelection field is used to choose between using sTypo* values or usWin* values for default line metrics. See fsSelection for additional details. It is not a general requirement that sTypoAscender - sTypoDescender be equal to unitsPerEm. These values should be set to provide default line spacing appropriate for the primary languages the font is designed to support. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical (as well as horizontal) layout, the required value for sTypoDescender is that which describes the bottom of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000 × 1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the recommendations in 8.20 for more on this field. |

#### sTypoLineGap

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Description: | The typographic line gap for this font. This field should be combined with the sTypoAscender and sTypoDescender values to determine default line spacing. This field is similar to the lineGap field in the 'hhea' table. However, legacy platform implementations treat that field with platform-specific behaviors. As a result, that field is constrained by backward-compatibility requirements, and does not ensure consistent layout across implementations. The sTypoAscender, sTypoDescender and sTypoLineGap fields are intended to allow applications to lay out documents in a typographically-correct and portable fashion. The USE_TYPO_METRICS flag (bit 7) of the fsSelection field is used to choose between using sTypo* values or usWin* values for default line metrics. See fsSelection for additional details. |

#### usWinAscent

| Format: | UFWORD |
| --- | --- |
| Units: | Font design units |
| Description: | The “Windows ascender” metric. This should be used to specify the height above the baseline for a clipping region. This is similar to the sTypoAscender field, and also to the ascender field in the 'hhea' table (5.1.4). There are important differences between these, however. In the Windows GDI implementation, the usWinAscent and usWinDescent values have been used to determine the size of the bitmap surface in the TrueType rasterizer. Windows GDI will clip any portion of a TrueType glyph outline that appears above the usWinAscent value. If any clipping is unacceptable, then the value should be set greater than or equal to yMax. NOTE This pertains to the default position of glyphs, not their final position in layout after data from the GPOS or 'kern' table has been applied. Also, this clipping behavior interacts with the VDMX table: if a VDMX table is present and there is data for the current device aspect ratio and rasterization size, then the VDMX data will supersede the usWinAscent and usWinDescent values. Some legacy applications use the usWinAscent and usWinDescent values to determine default line spacing. This is strongly discouraged. The sTypo* fields should be used for this purpose. Note that some applications use either the usWin* values or the sTypo* values to determine default line spacing, depending on whether the USE_TYPO_METRICS flag (bit 7) of the fsSelection field is set. This could be useful to provide compatibility with legacy documents using older fonts, while also providing better and more-portable layout using newer fonts. See fsSelection for additional details. Applications that use the sTypo* fields for default line spacing can use the usWin* values to determine the size of a clipping region. Some applications use a clipping region for editing scenarios to determine what portion of the display surface to re-draw when text is edited, or how large a selection rectangle to draw when text is selected. This is an appropriate use for the usWin* values. Early specifications suggested that the usWinAscent value be computed as the yMax for all characters in the Windows “ANSI” character set. For new fonts, the value should be determined based on the primary languages the font is designed to support, and should take into consideration additional height that could be required to accommodate tall glyphs or mark positioning. |

#### usWinDescent

| Format: | UFWORD |
| --- | --- |
| Units: | Font design units |
| Description: | The “Windows descender” metric. This should be used to specify the vertical extent below the baseline for a clipping region. This is similar to the sTypoDescender field, and also to the descender field in the 'hhea' table (5.1.4). There are important differences between these, however. Some of these differences are described below. In addition, the usWinDescent value treats distances below the baseline as positive values; thus, usWinDescent is usually a positive value, while sTypoDescender and hhea.descender are usually negative. In the Windows GDI implementation, the usWinDescent and usWinAscent values have been used to determine the size of the bitmap surface in the TrueType rasterizer. Windows GDI will clip any portion of a TrueType glyph outline that appears below (-1 × usWinDescent). If any clipping is unacceptable, then the value should be set greater than or equal to (-yMin). NOTE This pertains to the default position of glyphs, not their final position in layout after data from the GPOS or 'kern' table has been applied. Also, this clipping behavior interacts with the VDMX table: if a VDMX table is present and there is data for the current device aspect ratio and rasterization size, then the VDMX data will supersede the usWinAscent and usWinDescent values. Some legacy applications use the usWinAscent and usWinDescent values to determine default line spacing. This is strongly discouraged. The sTypo* fields should be used for this purpose. Note that some applications use either the usWin* values or the sTypo* values to determine default line spacing, depending on whether the USE_TYPO_METRICS flag (bit 7) of the fsSelection field is set. This could be useful to provide compatibility with legacy documents using older fonts, while also providing better and more-portable layout using newer fonts. See fsSelection for additional details. Applications that use the sTypo* fields for default line spacing can use the usWin* values to determine the size of a clipping region. Some applications use a clipping region for editing scenarios to determine what portion of the display surface to re-draw when text is edited, or how large a selection rectangle to draw when text is selected. This is an appropriate use for the usWin* values. Early specifications suggested that the usWinDescent value be computed as -yMin for all characters in the Windows “ANSI” character set. For new fonts, the value should be determined based on the primary languages the font is designed to support, and should take into consideration additional vertical extent that could be required to accommodate glyphs with low descenders or mark positioning. |

#### ulCodePageRange

ulCodePageRange1 Bits 0 – 31
ulCodePageRange2 Bits 32 – 63

| Format: | uint32[2] (64 bits total) |
| --- | --- |
| Title: | Code Page Character Range |
| Description: | This field is used to specify the code pages encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform, Unicode BMP). If the font file is encoding ID 0, then the Symbol Character Set bit should be set. If a given bit is set (1), then the code page is considered functional. If the bit is clear (0) then the code page is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by code pages if at all possible. Symbol character sets have a special meaning. If the symbol bit (31) is set, and the font file contains a 'cmap' subtable for platform of 3 and encoding ID of 1, then all of the characters in the Unicode range 0xF000 - 0xF0FF (inclusive) will be used to enumerate the symbol character set. If the bit is not set, any characters present in that range will not be enumerated as a symbol character set. All reserved fields shall be zero. Each uint32 is in Big-Endian form. |

| Bit | Code Page | Description |
| --- | --- | --- |
| 0 | 1252 | Latin 1 |
| 1 | 1250 | Latin 2: Eastern Europe |
| 2 | 1251 | Cyrillic |
| 3 | 1253 | Greek |
| 4 | 1254 | Turkish |
| 5 | 1255 | Hebrew |
| 6 | 1256 | Arabic |
| 7 | 1257 | Windows Baltic |
| 8 | 1258 | Vietnamese |
| 9 – 15 |  | Reserved for Alternate ANSI |
| 16 | 874 | Thai |
| 17 | 932 | JIS/Japan |
| 18 | 936 | Chinese: Simplified chars—PRC and Singapore |
| 19 | 949 | Korean Wansung |
| 20 | 950 | Chinese: Traditional chars—Taiwan and Hong Kong |
| 21 | 1361 | Korean Johab |
| 22 – 28 |  | Reserved for Alternate ANSI & OEM |
| 29 |  | Macintosh Character Set (US Roman) |
| 30 |  | OEM Character Set |
| 31 |  | Symbol Character Set |
| 32 – 47 |  | Reserved for OEM |
| 48 | 869 | IBM Greek |
| 49 | 866 | MS-DOS Russian |
| 50 | 865 | MS-DOS Nordic |
| 51 | 864 | Arabic |
| 52 | 863 | MS-DOS Canadian French |
| 53 | 862 | Hebrew |
| 54 | 861 | MS-DOS Icelandic |
| 55 | 860 | MS-DOS Portuguese |
| 56 | 857 | IBM Turkish |
| 57 | 855 | IBM Cyrillic; primarily Russian |
| 58 | 852 | Latin 2 |
| 59 | 775 | MS-DOS Baltic |
| 60 | 737 | Greek; former 437 G |
| 61 | 708 | Arabic; ASMO 708 |
| 62 | 850 | WE/Latin 1 |
| 63 | 437 | US |

#### sxHeight

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Description: | This metric specifies the distance between the baseline and the approximate height of non-ascending lowercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0078 (LATIN SMALL LETTER X). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in font substitution: the xHeight value of one font can be scaled to approximate the apparent size of another. |

#### sCapHeight

| Format: | FWORD |
| --- | --- |
| Units: | Font design units |
| Description: | This metric specifies the distance between the baseline and the approximate height of uppercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0048 (LATIN CAPITAL LETTER H). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in systems that specify type size by capital height measured in millimeters. It can also be used as an alignment metric; the top of a drop capital, for instance, can be aligned to the sCapHeight metric of the first line of text. |

#### usDefaultChar

| Format: | uint16 |
| --- | --- |
| Description: | This is the Unicode code point, in UTF-16 encoding, of a character that can be used for a default glyph if a requested character is not supported in the font. If the value of this field is zero, glyph ID 0 is to be used for the default character. This field cannot represent supplementary-plane character values (code points greater than 0xFFFF), and so applications are strongly discouraged from using this field. |

#### usBreakChar

| Format: | uint16 |
| --- | --- |
| Description: | This is the Unicode code point, in UTF-16 encoding, of a character that can be used as a default break character. The break character is used to separate words and justify text. Most fonts specify U+0020 SPACE as the break character. This field cannot represent supplementary-plane character values (code points greater than 0xFFFF), and so applications are strongly discouraged from using this field. |

#### usMaxContext

| Format: | uint16 |
| --- | --- |
| Description: | The maximum length of a target glyph context for any feature in this font. For example, a font which has only a pair kerning feature should set this field to 2. If the font also has a ligature feature in which the glyph sequence 'f f i' is substituted by the ligature 'ffi', then this field should be set to 3. This field could be useful to sophisticated line-breaking engines in determining how far they should look ahead to test whether something could change that affects the line breaking. For chaining contextual lookups, the length of the string (covered glyph) + (input sequence) + (lookahead sequence) should be considered. |

#### usLowerOpticalPointSize

| Format: | uint16 |
| --- | --- |
| Units: | TWIPs |
| Description: | This field is used for fonts with multiple optical styles. This value is the lower value of the size range for which this font has been designed. The units for this field are TWIPs (one-twentieth of a point, or 1440 per inch). The value is inclusive — meaning that that font was designed to work best at this point size through, but not including, the point size indicated by usUpperOpticalPointSize. When used with other optical-size-variant fonts within a typographic family that also specify usLowerOpticalPointSize and usUpperOpticalPointSize values, it would be expected that another font has the usUpperOpticalPointSize field set to the same value as the value in this field, unless this font is designed for the lowest size range among the fonts in the family. The smallest font in an optical-size set should set this value to 0. When working across multiple optical-size-variant fonts within a typographic family, there should be no intentional gaps or overlaps in the ranges. The usLowerOpticalPointSize value shall be less than usUpperOpticalPointSize. The maximum valid value is 0xFFFE. For fonts that were not designed for multiple optical-size variants, this field should be set to 0 (zero), and usUpperOpticalPointSize should be set to 0xFFFF. NOTE Use of this field has been superseded by the STAT table. See recommendations in subclause 9.9 for more information. |

#### usUpperOpticalPointSize

| Format: | uint16 |
| --- | --- |
| Units: | TWIPs |
| Description: | This field is used for fonts with multiple optical styles. This value is the upper value of the size range for which this font has been designed. The units for this field are TWIPs (one-twentieth of a point, or 1440 per inch). The value is exclusive — meaning that that font was designed to work best below this point size down to the usLowerOpticalPointSize threshold. When used with other optical-size-variant fonts within a typographic family that also specify usLowerOpticalPointSize and usUpperOpticalPointSize values, it would be expected that another font has the usLowerOpticalPointSize field set to the same value as the value in this field, unless this font is designed for the highest size range among the fonts in the family. The largest font in an optical-size set should set this value to 0xFFFF, which is interpreted as infinity. When working across multiple optical-size-variant fonts within a typographic family, there should be no intentional gaps or overlaps left in the ranges. The usUpperOpticalPointSize value shall be greater than usLowerOpticalPointSize. The minimum valid value for this field is 2 (two). The largest possible inclusive point size represented by this field is 3276.65 points; any higher values would be represented as infinity. For fonts that were not designed for multiple optical-size variants, this field should be set to 0xFFFF, and usLowerOpticalPointSize should be set to 0 (zero). NOTE Use of this field has been superseded by the 'STAT' table. See recommendations in subclause 9.9 for more information. |

#### OS/2 Table and Font Variations

In variable fonts, default line metrics should always be set using the sTypoAscender, sTypoDescender and sTypoLineGap values, and the USE_TYPO_METRICS flag in the fsSelection field should be set. The ascender, descender and lineGap fields in the 'hhea' table should be set to the same values as sTypoAscender, sTypoDescender and sTypoLineGap. The usWinAscent and usWinDescent fields should be used to specify a recommended clipping rectangle.

In a variable font, various font-metric values within the OS/2 table might need to be adjusted for different variation instances. Variation data for OS/2 entries can be provided in the metrics variations ('MVAR') table. Different OS/2 entries are associated with particular variation data in the 'MVAR' table using value tags, as follows:

| OS/2 entry | Tag |
| --- | --- |
| sCapHeight | 'cpht' |
| sTypoAscender | 'hasc' |
| sTypoDescender | 'hdsc' |
| sTypoLineGap | 'hlgp' |
| sxHeight | 'xhgt' |
| usWinAscent | 'hcla' |
| usWinDescent | 'hcld' |
| yStrikeoutPosition | 'stro' |
| yStrikeoutSize | 'strs' |
| ySubscriptXOffset | 'sbxo' |
| ySubScriptXSize | 'sbxs' |
| ySubscriptYOffset | 'sbyo' |
| ySubscriptYSize | 'sbys' |
| ySuperscriptXOffset | 'spxo' |
| ySuperscriptXSize | 'spxs' |
| ySuperscriptYOffset | 'spyo' |
| ySuperscriptYSize | 'spys' |

NOTE 1	The usWeightClass and usWidthClass values are not adjusted by variation data since these correspond to 'wght' and 'wdth' variation axes that can be used to define a font’s variation space. Appropriate usWeightClass and usWidthClass values for a variation instance can be derived from 'wght' and 'wdth' user coordinates that are used to select a particular variation instance. For 'wdth' values greater than 200, the usWidthClass value is clamped to 9. See the discussion of the 'wght' and 'wdth' axes in 7.2.4.2for details on the relationship between these OS/2 fields and the corresponding design axes.

NOTE 2	The usLowerOpticalPointSize and usUpperOpticalPointSize values are not adjusted by variation data. These values (now superseded by the 'STAT' table) are used to indicate a range of sizes for which a given font has been designed. It is assumed that variation that targets different sizes will be implemented using the 'opsz' variation axis. If a variable font supports 'opsz' as an axis of variation, then the usLowerOpticalPointSize and usUpperOpticalPointSize fields can be set to the same values as the minValue and maxValue fields for the 'opsz' axis in the 'fvar' table.

To have variable line metrics in a variable font, the 'hasc', 'hdsc' and 'hlgp' value tags should be used in the 'MVAR' table to vary the ascender, descender and line gap values from defaults specified in the sTypoAscender, sTypoDescender and sTypoLineGap fields. The 'hcla' and 'hcld' value tags can be used in addition to vary the size of a clipping region from the default specified in the winAscent and winDescent fields. Other metrics can be varied using value tags listed above.

For general information on OFF Font Variations, see subclause 7.1.

### post—PostScript

This table contains additional information needed to use OFF fonts on PostScript printers. This includes certain data found in the FontInfo dictionary of Type 1 fonts and the PostScript names of all the glyphs. For more information about PostScript names, see the Adobe Glyph List Specification[3].

Versions 1.0, 2.0, and 2.5 of the 'post' table are used only in fonts with TrueType or CFF version 2 outline data. Fonts with TrueType or CFF version 2 data may also use a version 3.0 'post' table. OFF fonts with CFF version 1 outline data shall only use a version 3.0 'post' table.

The table begins as follows:

| Type | Name | Description |
| --- | --- | --- |
| Version16Dot16 | version | 0x00010000 for version 1.0 0x00020000 for version 2.0 0x00025000 for version 2.5 (deprecated) 0x00030000 for version 3.0 |
| Fixed | italicAngle | Italic angle in counter-clockwise degrees from the vertical. Zero for upright text, negative for text that leans to the right (forward). |
| FWORD | underlinePosition | This is the suggested distance of the top of the underline from the baseline (negative values indicate below baseline). The PostScript definition of this FontInfo dictionary key (the y coordinate of the center of the stroke) is not used for historical reasons. The value of the PostScript key may be calculated by subtracting half the underlineThickness from the value of this field. |
| FWORD | underlineThickness | Suggested values for the underline thickness. In general, the underline thickness should match the thickness of the underscore character (U+005F LOW LINE), and should also match the strikeout thickness, which is specified in the OS/2 table (5.1.8.14). |
| uint32 | isFixedPitch | Set to 0 if the font is proportionally spaced, non-zero if the font is not proportionally spaced (i.e. monospaced). |
| uint32 | minMemType42 | Minimum memory usage when an OFF font is downloaded. |
| uint32 | maxMemType42 | Maximum memory usage when an OFF font is downloaded. |
| uint32 | minMemType1 | Minimum memory usage when an OFF font is downloaded as a Type 1 font. |
| uint32 | maxMemType1 | Maximum memory usage when an OFF font is downloaded as a Type 1 font. |

The last four entries in the table are present because PostScript drivers can do better memory management if the virtual memory (VM) requirements of a downloadable OFF font are known before the font is downloaded. This information should be supplied if known. If it is not known, set the value to zero. The driver will still work but will be less efficient.

Maximum memory usage is minimum memory usage plus maximum runtime memory use. Maximum runtime memory use depends on the maximum band size of any bitmap potentially rasterized by the font scaler. Runtime memory usage could be calculated by rendering characters at different point sizes and comparing memory use.

If the table version is 1.0 or 3.0, the table ends here. The additional entries for versions 2.0 and 2.5 are shown below. A version 4.0 is defined by Apple for use in their platforms (see Reference [2]), but is not supported in OFF.

#### Version 1.0

This version is used in order to supply PostScript glyph names when the font file contains exactly the 258 glyphs in the standard Macintosh TrueType font file (see 'post' Format 1 in Reference [2] for a list of the 258 Macintosh glyph names), and the font does not otherwise supply glyph names. As a result, the glyph names are taken from the system with no storage required by the font.

#### Version 2.0

Version 2.0 is used for fonts that use glyph names that are not in the set of Macintosh glyph names. A given font may map some of its glyphs to the standard Macintosh glyph names, and some to its own custom names. A version 2.0 'post' table can be used in fonts with TrueType or CFF version 2 outlines.

For version 2.0, the following fields are appended at the end of the header:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | numGlyphs | Number of glyphs (this should be the same as numGlyphs in 'maxp' table). |
| uint16 | glyphNameIndex[numGlyphs] | Array of indices into the string data. See below for details. |
| uint8 | stringData[variable] | Storage for the string data. |

This font file contains glyphs not in the standard Macintosh set, or the ordering of the glyphs in the font file differs from the standard Macintosh set. The glyphNameIndex array maps glyph IDs to a glyph name index. If the glyph name index is between 0 and 257 (inclusive), treat that index as a glyph index in the Macintosh standard glyph set and use the Macintosh glyph name. If the glyph name index is between 258 and 65535, then subtract 258 and use that to index into the list of Pascal strings at the end of the table.

For example, suppose glyphNameIndex[302] (for glyph ID 302) is 217: since that glyph name index is less than 258, the glyph name is the Macintosh glyph name for glyph ID 217. Suppose glyphNameIndex[408] is 262: subtracting 258, the difference is 4; the glyph name for that glyph is the fifth string (index 4, base 0) in the string data.

The string data extends from the last version 2.0 header field to the end of the table. Strings are in Pascal string format, meaning that the first byte of a given string is a length: the number of characters in that string. The length byte is not included; for example, a length byte of 8 indicates that the 8 bytes following the length byte comprise the string character data. To find the string for a given glyph name index, start with the first length byte, advance that number of bytes to find the length byte for the next string entry, and so on.

Glyph name strings are encoded in ASCII. Valid characters are limited to A–Z, a–z, 0–9, “.” (FULL STOP), and “_” (LOW LINE). Names shall be no longer than 63 characters; some older implementations can assume a length limit of 31 characters.

If you do not want to associate a PostScript name with a particular glyph, use 0, which refers to the name .notdef, as the glyphNameIndex entry for that glyph ID.

#### Version 2.5

Version 2.5 of the 'post' table is deprecated.

This version provides a space-saving table for fonts containing TrueType outlines which contain a pure subset of, or a simple reordering of, the standard Macintosh glyph set.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | numGlyphs | Number of glyphs. |
| int8 | offset[numGlyphs] | Difference between glyph index and the standard order of the glyph. |

This version has been used in some legacy fonts with TrueType outlines that contain only glyphs in the standard Macintosh glyph set but that have those glyphs arranged in a non-standard order or that are missing some glyphs. The table contains one byte for each glyph in the font file. The byte is treated as a signed offset that maps the glyph index used in the font into the standard glyph index. For example, assuming that the font contains the three glyphs A, B, and C which are the 37th, 38th, and 39th glyphs in the standard ordering, the 'post'; table would contain the bytes +36, +36, +36.

#### Version 3.0

This version makes it possible to create a font that is not burdened with a large set of glyph names. A version 3.0 'post' table can be used by OFF fonts with TrueType or CFF (version 1 or 2) data.

This version specifies that no PostScript name information is provided for the glyphs in this font file. The printing behavior of this version on PostScript printers is unspecified, except that it should not result in a fatal or unrecoverable error. Some drivers may print nothing; other drivers may attempt to print using a default naming scheme.

NOTE 	Windows makes use of the italic angle value in the 'post' table but does not actually require any glyph names to be stored as Pascal strings.

#### 'post' Table and OFF Font Variations

In a variable font, various font-metric values within the 'post' table may need to be adjusted for different variation instances. Variation data for 'post' entries can be provided in the metrics variations (MVAR) table (see subclause 7.3.6). Different 'post' entries are associated with particular variation data in the 'MVAR' table using value tags, as follows:

| 'post' entry | Tag |
| --- | --- |
| underlinePosition | 'undo' |
| underlineThickness | 'unds' |

NOTE	The italicAngle value is not adjusted by variation data since this corresponds to the 'slnt' variation axis that can be used to define a font’s variation space. Appropriate post.italicAngle values for a variation instance can be derived from the 'slnt' user coordinates that are used to select a particular variation instance. See the discussion of the 'slnt' axis in subclause 7.2.4 for details on the relationship between italicAngle and the 'slnt' axis.

For general information on OFF Font Variations, see subclause 7.1.

## Tables related to TrueType outlines

### List of TrueType outlines tables

For OFF fonts based on TrueType outlines, the following tables are used:

| Tag | Name |
| --- | --- |
| cvt | Control Value Table (optional table) |
| fpgm | Font program (optional table) |
| glyf | Glyph data |
| loca | Index to location |
| prep | Control Value Program (optional table) |
| gasp | Grid-fitting/Scan-conversion (optional table) |

### cvt—Control value table

This table contains a list of values that can be referenced by instructions. They can be used, among other things, to control characteristics for different glyphs. The length of the table must be an integral number of FWORD units.

| Type | Description |
| --- | --- |
| FWORD[ n ] | List of n values referenceable by instructions. n is the number of FWORD items that fit in the size of the table. |

### fpgm—Font program

This table is similar to the Control Value (CV) Program, except that it is only run once, when the font is first used. It is used only for FDEFs and IDEFs. Thus the CV Program need not contain function definitions. However, the CV Program may redefine existing FDEFs or IDEFs.

This table is optional.

| Type | Description |
| --- | --- |
| uint8[ n ] | Instructions. n is the number of uint8 items that fit in the size of the table. |

### glyf—Glyf data

#### Table structure

This table contains information that describes the glyphs in the font in the TrueType outline format. Information regarding the rasterizer (scaler) refers to the TrueType rasterizer. For details regarding scaling, grid-fitting and rasterization of TrueType outlines, see TrueType Fundamentals [35].

Table organization

The 'glyf' table is comprised of a list of glyph data blocks, each of which provides the description for a single glyph. Glyphs are referenced by identifiers (glyph IDs), which are sequential integers beginning at zero. The total number of glyphs is specified by the numGlyphs field in the 'maxp' table. The 'glyf' table does not include any overall table header or records providing offsets to glyph data blocks. Rather, the 'loca' table provides an array of offsets, indexed by glyph IDs, which provide the location of each glyph data block within the 'glyf' table. Note that the 'glyf' table must always be used in conjunction with the 'loca' and 'maxp' tables. The size of each glyph data block is inferred from the difference between two consecutive offsets in the 'loca' table (with one extra offset provided to give the size of the last glyph data block). As a result of the 'loca' format, glyph data blocks within the 'glyf' table must be in glyph ID order.

Each glyph description uses one of two formats:

- Simple glyph descriptions specify a glyph outline directly using Bezier control points.

- Composite glyph descriptions specify a glyph outline indirectly by referencing one or more glyph IDs to use as components.

In both cases, the glyph description begins with a glyph header that is common to both types of glyph description.

Glyph headers

Each glyph description begins with a header:

Glyph Header

| Type | Name | Description |
| --- | --- | --- |
| int16 | numberOfContours | If the number of contours is greater than or equal to zero, this is a simple glyph. If negative, this is a composite glyph—the value -1 should be used for composite glyphs. |
| int16 | xMin | Minimum x for coordinate data. |
| int16 | yMin | Minimum y for coordinate data. |
| int16 | xMax | Maximum x for coordinate data. |
| int16 | yMax | Maximum y for coordinate data. |

The bounding rectangle from each character is defined as the rectangle with a lower left corner of (xMin, yMin) and an upper right corner of (xMax, yMax). These values are obtained directly from the point coordinate data for the glyph, comparing all on-curve and off-curve points. Phantom points computed by the rasterizer are not relevant. Note that the bounding box defined by control points is guaranteed to contain the outline, but might not be tight to the outline.

The scaler will perform better if the glyph coordinates have been created such that the xMin is equal to the lsb. For example, if the lsb is 123, then xMin for the glyph should be 123. If the lsb is -12 then the xMin should be -12. If the lsb is 0 then xMin is 0. If all glyphs are done like this, set bit 1 of flags field in the 'head' table.

NOTE	The glyph descriptions do not include side bearing information. Left side bearings are provided in the 'hmtx' table, and right side bearings are inferred from the advance width (also provided in the 'hmtx' table) and the bounding box coordinates provided in the 'glyf' table. For vertical layout, top side bearings are provided in the 'vmtx' table, and bottom side bearings are inferred. The rasterizer will generate a representation of side bearings in the form of “phantom” points, which are added as four additional points at the end of the glyph description and which can be referenced and manipulated by glyph instructions. See Reference [36] for more background on phantom points.

In a variable font, the minimum and maximum x or y values of control points can vary, and a tight bounding rectangle containing the outline or all points for an instance could be smaller or larger than for the default instance (that is, for the glyph description in this table). The xMin, yMin, xMax and yMax values might or might not encompass the derived outline for an instance. Also, the 'gvar' table does not provide deltas for these values. If an application requires a bounding rectangle for a non-default instance of a glyph, the derived point data (with deltas applied) should be processed to determine a bounding rectangle.

##### Simple glyph description

This is the information used to describe a glyph if numberOfContours is greater than or equal to zero—that is, a glyph is not a composite. Note that point numbers are base-zero indices that are numbered sequentially across all of the contours for a glyph; that is, the first point number of each contour (except the first) is one greater than the last point number of the preceding contour.

Simple Glyph table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | endPtsOfContours[numberOfContours] | Array of point indices for the last point of each contour, in increasing numeric order. |
| uint16 | instructionLength | Total number of bytes for instructions. If instructionLength is zero, no instructions are present for this glyph, and this field is followed directly by the flags field. |
| uint8 | instructions[instructionLength] | Array of instruction byte code for the glyph. |
| uint8 | flags[variable] | Array of flag elements. See below for details regarding the number of flag array elements. |
| uint8 or int16 | xCoordinates[variable] | Contour point x-coordinates. See below for details regarding the number of coordinate array elements. Coordinate for the first point is relative to (0,0); others are relative to previous point. |
| uint8 or int16 | yCoordinates[variable] | Contour point y-coordinates. See below for details regarding the number of coordinate array elements. Coordinate for the first point is relative to (0,0); others are relative to previous point. |

NOTE 1	In the 'glyf' table, the position of a point is not stored in absolute terms but as a vector relative to the previous point. The delta-x and delta-y vectors represent these (often small) changes in position. Coordinate values are in font design units, as defined by the unitsPerEm field in the 'head' table. Note that smaller unitsPerEm values will make it more likely that delta-x and delta-y values can fit in a smaller representation (8-bit rather than 16-bit), though with a trade-off in the level or precision that can be used for describing an outline.

Each element in the flags array is a single byte, each of which has multiple flag bits with distinct meanings, as shown below.

In logical terms, there is one flag byte element, one x-coordinate, and one y-coordinate for each point. The number of points is determined by the last entry in the endPtsOfContours array. Note, however, that the flag byte elements and the coordinate arrays use packed representations. In particular, if a logical sequence of flag elements or sequence of x- or y-coordinates is repeated, then the actual flag byte element or coordinate value can be given in a single entry, with special flags used to indicate that this value is repeated for subsequent logical entries. The actual stored size of the flags or coordinate arrays must be determined by parsing the flags array entries. See the flag descriptions below for details.

Simple Glyph flags

| Mask | Name | Description |
| --- | --- | --- |
| 0x01 | ON_CURVE_POINT | Bit 0: If set, the point is on the curve; otherwise, it is off the curve. |
| 0x02 | X_SHORT_VECTOR | Bit 1: If set, the corresponding x-coordinate is 1 byte long, and the sign is determined by the X_IS_SAME_OR_POSITIVE_X_SHORT_VECTOR flag. If not set, its interpretation depends on the X_IS_SAME_OR_POSITIVE_X_SHORT_VECTOR flag: If that other flag is set, the x-coordinate is the same as the previous x-coordinate, and no element is added to the xCoordinates array. If both flags are not set, the corresponding element in the xCoordinates array is two bytes and interpreted as a signed integer. See the description of the X_IS_SAME_OR_POSITIVE_X_SHORT_VECTOR flag for additional information. |
| 0x04 | Y_SHORT_VECTOR | Bit 2: If set, the corresponding y-coordinate is 1 byte long, and the sign is determined by the Y_IS_SAME_OR_POSITIVE_Y_SHORT_VECTOR flag. If not set, its interpretation depends on the Y_IS_SAME_OR_POSITIVE_Y_SHORT_VECTOR flag: If that other flag is set, the y-coordinate is the same as the previous y-coordinate, and no element is added to the yCoordinates array. If both flags are not set, the corresponding element in the yCoordinates array is two bytes and interpreted as a signed integer. See the description of the Y_IS_SAME_OR_POSITIVE_Y_SHORT_VECTOR flag for additional information. |
| 0x08 | REPEAT_FLAG | Bit 3: If set, the next byte (read as unsigned) specifies the number of additional times this flag byte is to be repeated in the logical flags array—that is, the number of additional logical flag entries inserted after this entry. (In the expanded logical array this bit is ignored.) In this way, the number of flags listed can be smaller than the number of points in the glyph description. |
| 0x10 | X_IS_SAME_OR_POSITIVE_X_SHORT_VECTOR | Bit 4: This flag has two meanings, depending on how the X_SHORT_VECTOR flag is set. If X_SHORT_VECTOR is set, this bit describes the sign of the value, with 1 equaling positive and 0 negative. If X_SHORT_VECTOR is not set and this bit is set, then the current x-coordinate is the same as the previous x-coordinate. If X_SHORT_VECTOR is not set and this bit is also not set, the current x-coordinate is a signed 16-bit delta vector. |
| 0x20 | Y_IS_SAME_OR_POSITIVE_Y_SHORT_VECTOR | Bit 5: This flag has two meanings, depending on how the Y_SHORT_VECTOR flag is set. If Y_SHORT_VECTOR is set, this bit describes the sign of the value, with 1 equaling positive and 0 negative. If Y_SHORT_VECTOR is not set and this bit is set, then the current y-coordinate is the same as the previous y-coordinate. If Y_SHORT_VECTOR is not set and this bit is also not set, the current y-coordinate is a signed 16-bit delta vector. |
| 0x40 | OVERLAP_SIMPLE | Bit 6: If set, contours in the glyph description may overlap. Use of this flag is not required in OFF—that is, it is valid to have contours overlap without having this flag set. It may affect behaviors in some platforms, however. (See the discussion of “Overlapping contours” in Apple’s specification [7] for details regarding behavior in Apple platforms.) When used, it shall be set on the first flag byte for the glyph. See additional details below. |
| 0x80 | Reserved | Bit 7 is reserved: set to zero. |

A non-zero-fill algorithm is needed to avoid dropouts when contours overlap. The OVERLAP_SIMPLE flag is used by some rasterizer implementations to ensure that a non-zero-fill algorithm is used rather than an even-odd-fill algorithm. Implementations that always use a non-zero-fill algorithm will ignore this flag. Note that some implementations might check this flag specifically in non-variable fonts, but always use a non-zero-fill algorithm for variable fonts. This flag can be used in order to provide broad interoperability of fonts — particularly non-variable fonts — when glyphs have overlapping contours.

Variable fonts often make use of overlapping contours. This has implications for tools that generate static-font data for a specific instance of a variable font, if broad interoperability of the derived font is desired:  if a glyph has overlapping contours in the given instance, then the tool should either set the OVERLAP_SIMPLE flag in the derived glyph data, or else should merge contours to remove overlap of separate contours.

NOTE 2 	The OVERLAP_COMPOUND flag, described below, has a similar purpose in relation to composite glyphs. The same considerations described for the OVERLAP_SIMPLE flag also apply to the OVERLAP_COMPOUND flag.

##### Composite glyph description

If numberOfContours is negative, a composite glyph description is used.

NOTE 1	A numberOfContours value of -1 is recommended to indicate a composite glyph.

A composite , or compound, glyph describes an outline indirectly by referencing other glyphs that get incorporated into the composite glyph as components. This is useful when the same contours are needed for multiple glyphs as it provides consistency for contours that are repeated across multiple glyphs and can also provide significant size reduction.

To add clarity in explaining composite glyphs, the terms parent and child will be used, a composite glyph description being the parent, and the other glyphs referenced as components being the children.

Composite glyphs may be nested within other composite glyphs—that is, a composite glyph parent may include other composite glyphs as child components. Thus, a composite glyph description is a directed graph. This graph shall be acyclic, with every path through the graph leading to a simple glyph as a leaf node. The maxComponentDepth field in the 'maxp' table is set to indicate the maximum nesting depth across all composite glyphs in a font. There is no minimum nesting depth that must be supported. For fonts to be compatible with the widest range of implementations, nesting of composites should be avoided.

NOTE 2	Some PostScript devices (and possibly other implementations) do not correctly render glyphs that have nested composite descriptions. A composite glyph description that has nested composites can be flattened to reference only simple glyphs as child components. This can lose some benefits of de-duplication of information, but can still retain significant size-saving benefits as well as providing broader compatibility.

The data for a composite glyph description is comprised of a sequence of data blocks for each child component glyph. A flag within the data for each component is used to indicate if there are additional components in the sequence. The sequence is processed in the order given, with the contours from each child glyph incorporated into the parent. As the contours from a child are incorporated, its control points are renumbered to follow sequentially after all points previously incorporated into the parent.

Each glyph has an outline positioned within the font design grid based on the x and y coordinates of its control points. When incorporated as a child into a composite glyph, the parent can control placement of the child outline within the parent’s design grid. This can be done in two different ways: by specifying a vector offset added to (x, y) coordinates of the child’s control points, or by specifying one control point from the child’s outline that is aligned with a specified control point in the parent. The second mechanism assumes some outlines have already been incorporated into the parent, so cannot be used for the first component glyph.

The parent can also specify a scale or other affine transform to be applied to a child glyph as it is incorporated into the parent. The transform can affect an offset vector used to position the child glyph; see below for additional details.

Each component glyph can include instructions that apply to its outline. A parent composite glyph description can include instructions that apply to the composite as a whole, after instructions for each child have been performed. Instructions for the parent composite apply to the accumulated contour data from components with points renumbered, as described above.

Before each child is incorporated into the parent, it is processed, with phantom points defined and hinting instructions performed. Thus, if placement of the child is done by alignment of points, the child’s phantom points can be used for this alignment, and instructions in child glyphs that affected their points will already have been performed.

The data block for each child component starts with two uint16 values: a flags field, and a glyph ID. These are followed by two argument fields, though the size and interpretation of the arguments varies according to the flags that are set. Optional fields describing a transformation can follow the arguments, depending on the flags.

Component glyph record

| Type | Name | Description |
| --- | --- | --- |
| uint16 | flags | component flag |
| uint16 | glyphIndex | glyph index of component |
| uint8, int8, uint16 or int16 | argument1 | x-offset for component or point number; type depends on bits 0 and 1 in component flags |
| uint8, int8, uint16 or int16 | argument2 | y-offset for component or point number; type depends on bits 0 and 1 in component flags |
| [transform data] |  | optional transform data—see below |

The C pseudo-code fragment below shows how the sequence of composite glyph records is stored and parsed; definitions for the flag bits follow this fragment:

do {

uint16 flags;

uint16 glyphIndex;

if ( flags & ARG_1_AND_2_ARE_WORDS) {

(int16 or FWORD) argument1;

(int16 or FWORD) argument2;

} else {

uint16 arg1and2; /* (arg1 << 8) | arg2 */

}

if ( flags & WE_HAVE_A_SCALE ) {

F2DOT14  scale;    /* Format 2.14 */

} else if ( flags & WE_HAVE_AN_X_AND_Y_SCALE ) {

F2DOT14  xscale;    /* Format 2.14 */

F2DOT14  yscale;    /* Format 2.14 */

} else if ( flags & WE_HAVE_A_TWO_BY_TWO ) {

F2DOT14  xscale;    /* Format 2.14 */

F2DOT14  scale01;   /* Format 2.14 */

F2DOT14  scale10;   /* Format 2.14 */

F2DOT14  yscale;    /* Format 2.14 */

}

} while ( flags & MORE_COMPONENTS )

if (flags & WE_HAVE_INSTRUCTIONS){

uint16 numInstr

uint8 instr[numInstr]

The following composite glyph flags are defined:

| Mask | Flags | Description |
| --- | --- | --- |
| 0x0001 | ARG_1_AND_2_ARE_WORDS | Bit 0: If this is set, the arguments are 16-bit (uint16 or int16); otherwise, they are bytes (uint8 or int8). |
| 0x0002 | ARGS_ARE_XY_VALUES | Bit 1: If this is set, the arguments are signed xy values; otherwise, they are unsigned point numbers. |
| 0x0004 | ROUND_XY_TO_GRID | Bit 2: If set and ARGS_ARE_XY_VALUES is also set, the xy values are rounded to the nearest grid line. Ignored if ARGS_ARE_XY_VALUES is not set. |
| 0x0008 | WE_HAVE_A_SCALE | Bit 3: This indicates that there is a simple scale for the component. Otherwise, scale = 1.0. |
| 0x0020 | MORE_COMPONENTS | Bit 5: Indicates at least one more glyph after this one. |
| 0x0040 | WE_HAVE_AN_X_AND_Y_SCALE | Bit 6: The x direction will use a different scale from the y direction. |
| 0x0080 | WE_HAVE_A_TWO_BY_TWO | Bit 7: There is a 2 by 2 transformation that will be used to scale the component. |
| 0x0100 | WE_HAVE_INSTRUCTIONS | Bit 8: Following the last component are instructions for the composite glyph. |
| 0x0200 | USE_MY_METRICS | Bit 9: If set, this forces the aw and lsb (and rsb) for the composite to be equal to those from this component glyph. This works for hinted and unhinted glyphs. |
| 0x0400 | OVERLAP_COMPOUND | Bit 10: If set, the components of this compound glyph overlap. Use of this flag is not required in OFF—that is, component glyphs may overlap without having this flag set. It can affect behaviors in some platforms, however. (See Apple’s specification [7] for details regarding behavior in Apple platforms.) When used, it shall be set on the flag word for the first component. See additional remarks, above, for the similar OVERLAP_SIMPLE flag used in simple-glyph descriptions. |
| 0x0800 | SCALED_COMPONENT_OFFSET | Bit 11: The composite is designed to have the component offset scaled. Ignored if ARGS_ARE_XY_VALUES is not set. |
| 0x1000 | UNSCALED_COMPONENT_OFFSET | Bit 12: The composite is designed not to have the component offset scaled. Ignored if ARGS_ARE_XY_VALUES is not set. |
| 0xE010 | RESERVED | Bits 4, 13, 14 and 15 are reserved: set to 0. |

The argument1 and argument2 fields of the component glyph record are used to determine the placement of the child component glyph within the parent composite glyph. They are interpreted either as an offset vector or as points from the parent and the child, according to whether the ARGS_ARE_XY_VALUES flag is set. This flag must always be set for the first component of a composite glyph.

If ARGS_ARE_XY_VALUES is set, then argument1 and argument2 are interpreted as units in the design coordinate system and an offset vector (x, y) = (argument1, argument2) is added to the coordinates of each control point of the component glyph. In a variable font, the offset vector can be modified by deltas in the 'gvar' table; see 7.3.4.3 for details. If a scale or transform matrix is provided, the offset vector might or might not be subject to the transformation; see the discussion below of the SCALED_COMPONENT_OFFSET and UNSCALED_COMPONENT_OFFSET flags for details.

If ARGS_ARE_XY_VALUES is set and the ROUND_XY_TO_GRID flag is also set, the offset vector (after any transformation and variation deltas are applied) is grid-fitted, with the x and y values rounded to the nearest pixel grid line.

If ARGS_ARE_XY_VALUES is not set, then argument1 is a point number in the parent glyph (from contours incoporated and re-numbered from previous component glyphs); and argument2 is a point number (prior to re-numbering) from the child component glyph. Phantom points from the parent or the child may be referenced. The child component glyph is positioned within the parent glyph by aligning the two points. If a scale or transform matrix is provided, the transformation is applied to the child’s point before the points are aligned.

In a variable font, when a component is positioned by alignment of points, deltas are applied to component glyphs before this alignment is done. Any deltas specified for the parent composite glyph to be applied to components positioned by point alignment are ignored. See 7.3.4.3 for details.

The WE_HAVE_A_SCALE, WE_HAVE_AN_X_AND_Y_SCALE and WE_HAVE_A_TWO_BY_TWO flags are mutually exclusive: no more than one of these may be set. If WE_HAVE_A_SCALE is set, one additional F2DOT14 value is appended to the component glyph data; if WE_HAVE_AN_X_AND_Y_SCALE is set, two F2DOT14 values are appended; if WE_HAVE_A_TWO_BY_TWO, four F2DOT14 values are appended. The child component glyph is transformed as it is incorporated into the parent composite glyph, prior to grid-fitting of the parent. The transform can affect an offset vector used to position the child; see discussion below of the SCALED_COMPONENT_OFFSET and UNSCALED_COMPONENT_OFFSET flags for details.

The WE_HAVE_INSTRUCTIONS flag is used to indicate that the parent composite glyph has instructions, in addition to instructions for any of the child component glyphs. If the flag is set on any component glyph, then a uint16 value is read immediately after the last component glyph to get the byte length for instructions.

The purpose of USE_MY_METRICS is to force the lsb and rsb to take on values obtained from the component glyph. For example, an i-circumflex (U+00EF) is often composed of the circumflex and a dotless-i. In order to force the composite to have the same metrics as the dotless-i, set USE_MY_METRICS for the dotless-i component of the composite. Without this bit, the rsb and lsb would be calculated from the 'hmtx' entry for the composite (or would need to be explicitly set with TrueType instructions).

Note that the behavior of the USE_MY_METRICS operation is undefined for rotated component glyphs.

The SCALED_COMPONENT_OFFSET and UNSCALED_COMPONENT_OFFSET flags are used to determine how x and y offset values are to be interpreted when the component glyph is scaled. If the SCALED_COMPONENT_OFFSET flag is set, then the x and y offset values are deemed to be in the component glyph’s coordinate system, and the scale transformation is applied to both values. If the UNSCALED_COMPONENT_OFFSET flag is set, then the x and y offset values are deemed to be in the current glyph’s coordinate system, and the scale transformation is not applied to either value. If neither flag is set, then the rasterizer may apply a default behavior. On Microsoft and Apple platforms, the default behavior is the same as when the UNSCALED_COMPONENT_OFFSET flag is set; this behavior is recommended for all rasterizer implementations. If a font has both flags set, this is invalid; the rasterizer should use its default behavior for this case.

### loca—Index to location

The index to location table provides a mapping from glyph indices to offsets, which are the locations of TrueType glyph descriptions in the 'glyf' table (5.2.4), relative to the beginning of that table (“local” offsets). In order to compute the length of the last glyph description, there is an extra entry: after the entry pointing to the last valid glyph description, there is a final entry that points to the end of the glyph data.

The table is an array of n offsets, where n is the number of glyphs in the font plus one. There are two formats for the array that use different sizes for the offsets, as described below. The format used is determined by the indexToLocFormat field in the 'head' table (5.1.3). The total size of the array must equal the size of the offsets times the value of the numGlyphs field in the 'maxp' table (5.1.6) plus one.

Offsets must be two-byte aligned and must be in ascending order, with loca[n] <= loca[n+1].

By definition, glyph index zero points to the "missing character", which is the glyph that appears if a character is not found in the font. The missing character is commonly represented by a blank box or a space. If the font does not contain an outline for the missing character glyph, then the first and second offsets should have the same value. This also applies to any other glyphs without an outline, such as the space character: if a glyph has no outline, then loca[n] = loca[n+1]. In the particular case of the last entry, loca[numGlyphs] will equal the length of the glyph data ('glyf') table.

There are two formats of this table: the short format, and the long format. The format is specified in the indexToLocFormat entry in the 'head' table.

Short format

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | offsets[numGlyphs + 1] | The local offset divided by 2 is stored. |

Long format

| Type | Name | Description |
| --- | --- | --- |
| Offset32 | offsets[numGlyphs + 1] | The actual local offset is stored. |

### prep—Control value program

The Control Value (CV) Program consists of a set of TrueType instructions that can be used to make font-wide changes in the Control Value Table. Any instruction is valid in the CV Program but since no glyph is associated with it, instructions intended to move points within a particular glyph outline have no effect in the CV Program.

The CV Program can potentially be executed before any glyph is processed, as determined by the rasterizer implementation. The CV Program will always be executed before the first glyph is processed after a change in font, point size, global transformation matrix, or other values that can be obtained from the GET INFOrmation [36] instruction. The CV Program is not re-executed for components in a composite glyph, including cases in which a composite glyph description applies a transform to a component.

NOTE	The tag 'prep' is anachronistic: in earlier specifications, the table was referred to as the Pre Program table.

| Type | Description |
| --- | --- |
| uint8[ n ] | Set of instructions executed whenever point size or font or transformation change. n is the number of uint8 items that fit in the size of the table. |

### gasp—Grid-fitting and scan-conversion procedure table

This table contains information which describes the preferred rasterization techniques for the typeface when it is rendered on grayscale-capable devices. This table also has some use for monochrome devices, which may use the table to turn off hinting at very large or small sizes, to improve performance.

At very small sizes, the best appearance on grayscale devices can usually be achieved by rendering the glyphs in grayscale without using hints. At intermediate sizes, hinting and monochrome rendering will usually produce the best appearance. At large sizes, the combination of hinting and grayscale rendering will typically produce the best appearance.

If the 'gasp' table is not present in a typeface, the rasterizer may apply default rules to decide how to render the glyphs on grayscale devices.

Two versions of the 'gasp' table have been defined: both versions use the same formats, except that two new flags were defined in version 1, as documented below. All new fonts and applications should use version 1.

The 'gasp' table consists of a header followed by groupings of 'gasp' records:

'gasp' Header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Version number (0 or 1—set to 1 in new fonts) |
| uint16 | numRanges | Number of records to follow |
| GaspRange | gaspRanges[numRanges] | Sorted by ppem |

The array of GaspRange records provides recommended behaviors for various ppem sizes.

GaspRange Record

| Type | Name | Description |
| --- | --- | --- |
| uint16 | rangeMaxPPEM | Upper limit of range, in PPEM |
| uint16 | rangeGaspBehavior | Flags describing desired rasterizer behavior. |

There are four RangeGaspBehavior flags defined.

RangeGaspBehavior flags

| Mask | Name | Description |
| --- | --- | --- |
| 0x0001 | GASP_GRIDFIT | Use gridfitting |
| 0x0002 | GASP_DOGRAY | Use grayscale rendering |
| 0x0004 | GASP_SYMMETRIC_GRIDFIT | Use gridfitting with ClearType symmetric smoothing Only supported in version 1 |
| 0x0008 | GASP_SYMMETRIC_ SMOOTHING | Use smoothing along multiple axes with ClearType® Only supported in version 1 |
| 0xFFF0 | Reserved | Reserved flags – set to 0. |

The set of bit flags may be extended in the future. The first two bit flags operate independently of the following two bit flags. If font smoothing is enabled, then the first two bit flags are used. If ClearType is enabled, then the following two bit flags are used. The seven currently defined values of rangeGaspBehavior would have the following uses:

| Flag | Value | Meaning |
| --- | --- | --- |
| GASP_DOGRAY | 0x0002 | small sizes, typically ppem < 9 |
| GASP_GRIDFIT | 0x0001 | medium sizes, typically 9 <= ppem <= 16 |
| GASP_DOGRAY ¦ GASP_GRIDFIT | 0x0003 | large sizes, typically ppem > 16 |
| GASP_SYMMETRIC_GRIDFIT | 0x0004 | typically always enabled |
| GASP_SYMMETRIC_SMOOTHING | 0x0008 | larger screen sizes, typically ppem > 15, most commonly used with the gridfit flag. |
| GASP_SYMMETRIC_SMOOTHING ¦ GASP_ SYMMETRIC_GRIDFIT | 0x000C | Large screen sizes, typically ppem>15 |
| (neither) | 0x0000 | optional for very large sizes, typically ppem>2048 |

The records in the gaspRange[] array must be sorted in order of increasing rangeMaxPPEM value. The last record should use 0xFFFF as a sentinel value for rangeMaxPPEM and should describe the behavior desired at all sizes larger than the previous record's upper limit. If the only entry in 'gasp' is the 0xFFFF sentinel value, the behavior described will be used for all sizes.

'gasp' Table and OFF Font Variations

In a variable font, the threshold sizes at which rasterizer behaviors are changed may need to be adjusted for different variation instances. Variation data for adjusting the rangeMaxPPEM value of up to ten GaspRange records can be provided in the metrics variations ('MVAR') table (7.3.6), referenced using value tags 'gsp0' to 'gsp9'. Note that the rasterizer behavior for a given GaspRange record cannot be changed for different variation instances; only the rangeMaxPPEM value can be adjusted.

The last GaspRange record in a ‘gasp’ table is assumed to have a rangeMaxPPEM value of 0xFFFF (effectively infinity). The rangeMaxPPEM value of the last record is never adjusted for different instances; the number of value records in the 'MVAR' table that are associated with 'gasp' entries must never be more than numRanges minus one.

For general information on OFF Font Variations, see subclause 7.1.

Sample 'gasp' table

| Field | Value | Font smoothing meaning | ClearType with symmetric smoothing meaning |
| --- | --- | --- | --- |
| version | 0x0001 |  |  |
| numRanges | 0x0004 |  |  |
| gaspRanges[0] |  |  |  |
| rangeMaxPPEM | 0x0008 | ppem <= 8 | ppem <= 8 |
| rangeGaspBehavior | 0x000A | grayscale only | symmetric ClearType only |
| gaspRanges[1] |  |  |  |
| rangeMaxPPEM | 0x0010 | 9 <= ppem <= 16 | 9 <= ppem <= 16 |
| rangeGaspBehavior | 0x0005 | gridfit only | symmetric gridfit only |
| gaspRanges[2] |  |  |  |
| rangeMaxPPEM | 0x0013 | 17 <= ppem <= 19 | 17 <= ppem <= 19 |
| rangeGaspBehavior | 0x0007 | gridfit and grayscale | symmetric gridfit |
| gaspRanges[3] |  |  |  |
| rangeMaxPPEM | 0xFFFF | 20 <= ppem | 20 <= ppem |
| rangeGaspBehavior | 0x000F | gridfit and grayscale | symmetric gridfit and symmetric smoothing |

## Tables related to CFF outlines

### List of CFF outline tables

For OFF fonts based on CFF outlines, the following tables are used:

| Tag | Name |
| --- | --- |
| CFF | Compact Font Format 1.0 |
| CFF2 | Compact Font Format 2.0 |
| VORG | Vertical Origin (optional table) |

It is strongly recommended that CFF fonts that are used for vertical writing include a Vertical Origin ('VORG') table.

### CFF—Compact Font Format table (version 1)

This table contains a Compact Font Format (CFF) font representation (also known as a PostScript Type 1 font, or CID Font) and is structured according to Adobe Technical Note #5176: "The Compact Font Format Specification" [5] and Adobe Technical Note #5177: "Type 2 Charstring Format" [4].

NOTE	See Reference [5] for details regarding the CFF INDEX, DICT and FontSet structures mentioned below.

The CFF font format existed as an independent font format prior to the development of OFF. When incorporated into the OFF format, certain essential font information continued to be represented using mechanisms defined in the CFF format, duplicating other data representations available within OFF for use in fonts with TrueType outlines. For example, CFF incorporates glyph widths into glyph outline descriptions, though these could otherwise be provided in the 'hmtx' table (5.1.5). OFF fonts with TrueType outlines use a glyph index in conjunction with offsets in the 'loca' table (5.2.5) to access glyph data within the 'glyf’ table (5.2.4). This concept is retained in OFF CFF fonts, except that glyph data is accessed through a CharStrings INDEX structure within the 'CFF ' table.

See Annex C for a summary of significant differences between the 'glyf', 'CFF ' and 'CFF2' tables.

The Name INDEX in the 'CFF ' table shall contain only one entry; that is, there must be only one font in the CFF FontSet. It is not a requirement that this name be the same as name ID 6 entries in the ‘name’ table (5.1.7). Note that, in an OFF Font Collection file, a single 'CFF ' table can be shared across multiple fonts; names used by applications shall be those provided in the 'name' table, not the Name INDEX entry.

The CFF Top DICT shall specify a CharstringType value of 2.

The numGlyphs field in the 'maxp' table shall be the same as the number of entries in the CFF's CharStrings INDEX. The OFF font glyph index is the same as the CFF glyph index for all glyphs in the font.

### CFF2—Compact Font Format (version 2) table

#### Introduction

The Compact Font Format table, version 2 (CFF2), is used for describing glyphs in an OFF font. It is an alternative to the ‘glyf’ table (subclause 5.3.4) using an efficient format to represent glyph outlines that has origins in the Adobe® Postscript® language.

In the CFF2 table, sequences of cubic (3rd-order) Bézier curves and straight lines are used to define glyph outlines. CFF2 data can also include “blend” operations, controlled by the OFF Font Variations mechanisms to change the shapes of glyphs. A rasterization fill rule is used to provide the opaque, monochrome shape of each glyph. CFF2 data can include “hint” operations that influence this rasterization. When combined with the COLR (subclause 5.7.11) and CPAL (subclause 5.7.12) tables, the CFF2 table can be used to represent multi-color glyphs.

CFF2 is the successor to the CFF table (subclause 5.4.2) and glyph format. Because of the origins of CFF in the Postscript language, the data in a complete CFF table can be used as a stand-alone font. When used within an OFF font, however, that aspect of CFF results in redundancy. CFF2 avoids redundancy by relying on data in other font tables. CFF2 also adds new operators used in variable fonts and uses a new CFF2 CharString specification.

See Annex C for a summary of significant differences between the 'glyf', 'CFF ' and 'CFF2' tables.

#### Organization of the CFF2 table

The CFF2 table is comprised of various required and optional subtables. The following description summarizes the overall structure of the CFF2 table.

Two common, generic structures are used for various subtables:

- A DICT structure is a binary dictionary format with one or more key-value pairs. This format is used for TopDICT, FontDICT and LocalDICT subtables.

- An INDEX structure contains an array of one or more data objects of various types. This format is used for CharStringINDEX, GlobalSubrINDEX, LocalSubrINDEX and FontDictINDEX subtables.

The DICT and INDEX formats and the various subtables that use them are described in subsequent parts of this document.

The CFF2 table begins with a short header (5.3.3.6). The header is followed by the TopDICT subtable (5.3.3.7), which stores offsets to other subtables. The TopDICT subtable is followed by the GlobalSubrINDEX subtable, which stores CharString data (5.3.3.9) that can be re-used in multiple glyph descriptions.

The first three structures—Header, TopDICT and GlobalSubrINDEX—must occur in that order at the start of the CFF2 table. Other subtables may occur in any order at offsets indicated in the TopDICT subtable and elsewhere.

The CharStringINDEX subtable stores CharString data. A CharString is an encoded representation of a glyph, including the glyph outline data as well as hinting and variation data specific to the glyph. Within the CharStringINDEX, there is one CharString for each glyph in the font. CharStrings that have encoded data in common with other CharStrings may use Subrs (subroutines) to save space. Subroutines are stored in the GlobalSubrINDEX subtable or in LocalSubrINDEX data within a LocalDICT subtable.

Each CharString is dependent on metadata related to hinting and subroutines that is stored in a FontDICT / LocalDICT pair. (The metadata is stored in a LocalDICT; a corresponding FontDICT provides the location of the LocalDICT.) A CFF2 table only requires a single FontDICT / LocalDICT pair. Multiple FontDICT / LocalDICT pairs may be used, however. In particular, if some CharStrings require a different set of metadata from other CharStrings, then multiple FontDICT / LocalDICT pairs may be defined. In such cases, a FontDICTSelect subtable is included to specify which FontDICT / LocalDICT pair is used by each CharString.

The following table illustrates organization of the various subtables that can occur in a CFF2 table. Required subtables are shown in bold type.

CFF2 Data layout

| Data Block | Required | Offset from start of CFF2 table |
| --- | --- | --- |
| Header | Yes | 0 |
| TopDICT | Yes | 5 |
| GlobalSubrINDEX | Yes | 5 + Header.TopDictLength |
| CharStringINDEX | Yes | TopDICT: CharStringINDEXOffset |
| FontDICTSelect | No | TopDICT: FontDICTSelectOffset |
| FontDICTINDEX | Yes | TopDICT: FontDICTINDEXOffset |
| FontDICT#0 | Yes | +FontDICTINDEX.offsets[0] |
| FontDICT#1 | - | +FontDICTINDEX.offsets[1] |
| … |  |  |
| FontDICT#n | - | +FontDICTINDEX.offsets[n] |
| LocalDICT#0 | Yes | FontDICT#0: LocalDICTOffset |
| LocalDICT#1 | - | FontDICT#1: LocalDICTOffset |
| … |  |  |
| LocalDICT#n | - | FontDICT#n: LocalDICTOffset |
| VariationStore | No | TopDICT: VariationStoreOffset |

An annotated example of a CFF2 table is shown in 5.3.3.14.

Subtables and data types used within a CFF2 table are not subject to any byte-alignment requirements.

DICT and CharString data are structured, binary data that use certain encoded representations for numbers, along with certain operators that determine the interpretation of the numeric values. For each operator, there is a defined syntactic sequence for numeric arguments. A stack is used by a CFF2 decoder for processing the binary operator/argument data in DICT and CharString data blocks. The various DICT and CharString operators are described in the clauses below that describe those two types of structured data. The encoded representations of numbers and the decoding stack are described in 5.3.3.3 and 5.3.3.4.

#### Encoded numbers

##### Encoded number formats

Various CFF2 structures represent numeric values using the standard OpenType data types, such as uint8 and Fixed (see 4.3). DICT and CharString data (including CharString subroutines) use special encoded representations for numbers.

During decoding, byte sequences representing numbers or operators are encountered. Numbers have various encodings that use one or more bytes to represent a numeric value, depending on their magnitude and whether they are integers. The initial bytes used for encoding of numbers do not overlap with the inital bytes used for operators. Hence, for all number encodings, a byte sequence can be identified as being a number by the initial byte of the encoded representation

.

Most number encodings may be used in both DICT and Charstring data, though there are exceptions. The following table shows how numbers are encoded based on an initial byte, and also indicates whether each encoding is used in DICT or CharString data only, or both. Each encoding uses an initial byte, b0, interpreted as uint8, and possible subsequent bytes b1, b2, b3, etc. In the 2-byte formats, b1 is also interpreted as uint8.

Numerical Value Encoding

| Initial byte b0 | Value range | Value calculation | Size in bytes | Usage |
| --- | --- | --- | --- | --- |
| 32 to 246 | -107 to 107 | b0 - 139 | 1 | both |
| 247 to 250 | 108 to 1131 | (b0 - 247) * 256 + b1 + 108 | 2 | both |
| 251 to 254 | -1131 to -108 | -(b0 - 251) * 256 - b1 - 108 | 2 | both |
| 28 | -32768 to 32767 | interpret b1 and b2 as int16 | 3 | both |
| 255 | -32768 to (32768 - 1/65536) | interpret b1 to b4 as Fixed | 5 | CharString only |
| 29 | -(2^31) to (2^31 - 1) | interpret b1 to b4 as int32 | 5 | DICT only |
| 30 | any real number | following bytes are Binary Coded Decimal | unlimited | DICT only |

NOTE	For some of these encodings, b0 serves only to indicate the encoding used. For some, however, b0 is used both to indicate the encoding used and also in the calculation of the numeric value.

The Usage column indicates whether each encoded format may be used in DICT data, in CharString data, or in both. Numbers encoded as int32 (b0 = 29) or binary coded decimal (b0 = 30) are only permitted in DICT data. Numbers encoded as Fixed (b0 = 255) are only permitted in CharString data. It is not possible to represent numbers >= 32768 or < -32768 in CharString data.

If the initial byte b0 is not in this list, then either the byte encodes an operator or the data is invalid.

##### Binary Coded Decimal

Real numeric values of arbitrary precision can be represented in a binary coded decimal form. This representation uses a binary encoding of a decimal numeric expression, such as “123.456”. The numeric expression can optionally use decimal exponential notation, such as “1.23456 × 10^2”. Binary coded decimal numbers may only be used in DICT data.

The representation of binary coded decimal numbers begins with a prefix byte (b0) value of 30. This is followed by a byte sequence in which each 4-bit nibble represents an element. The two nibbles of each byte are interpreted in big-endian order: the first element is stored in the most significant 4 bits, and the second element is stored in the least significant 4 bits. The sequence is terminated with a nibble value of 0xf (hexadecimal).

After the prefix byte of 30 is recognized, the value of the binary coded decimal number is obtained by stepping through the nibbles, building up the decimal expression of the number, until the termination nibble is encountered. Each nibble value is interpreted according to the following table. Converted to ASCII, the string can then be converted to a high-precision floating point number using standard functions in most programming languages.

Nibble definitions

| Nibble value | Nibble value (hex) | Represents in ASCII |
| --- | --- | --- |
| 0 to 9 | 0 to 9 | 0 to 9 |
| 10 | a | . (decimal point) |
| 11 | b | E |
| 12 | c | E- |
| 13 | d | <reserved> |
| 14 | e | - (minus) |
| 15 | f | end of number |

If the terminating 0xf nibble is the first nibble of a byte pair, then an additional 0xf shall be appended so that the encoded representation is always a whole number of bytes.

Negative exponents, as in the example 3×10^-5 (=0.00003), shall be represented using the nibble value 0xc, not the nibble sequence 0xb followed by 0xe.

While the binary encoded decimal format can represent values with arbitrary precision, the maximum degree of effective precision obtainable is implementation dependent.

Examples:

- the value -2.25 is encoded by the byte 30 (decimal) followed by the byte sequence 0xe2 0xa2 0x5f (hexadecimal)

- the value 0.140541E-3 is encoded by 30 (decimal) followed by the byte sequence 0x0a 0x14 0x05 0x41 0xc3 0xff (hexadecimal).

The following regular expression (using POSIX ERE or Perl Compatible RE syntax) validates a binary coded decimal value represented as ASCII:

-?([1-9][0-9]*|0)?(\.[0-9]*)?(E-?[1-9][0-9]*)?

The following table shows some edge cases (represented as ASCII) and their decoded values:

| Input | Value |
| --- | --- |
| [empty] | 0 |
| . | 0 |
| .5 | 0.5 |
| 2. | 2 |
| e5 | invalid |
| 05 | invalid |
| e05 | invalid |

#### Decoding DICT and CharString data using a stack

##### Introduction

Two fundamental data encoding schemes in the CFF2 table are the DICT and the CharString. Each is a binary data block that represents a sequence of encoded numbers and operators. In order to interpret those sequences, CFF2 decoders use a stack.

A stack is an array conceived as a physical stack of items, which may be manipulated in only two ways:

- place one new item on the top (“push an item onto the stack”);

- remove one item from the top (“pop an item from the stack”).

When an item is popped from the stack, the item removed shall be the last item that was added.

##### The stack-based decoder in CFF2

A CFF2 decoder is expected to implement a stack in a suitable manner. The CFF2 stack stores only numbers.

Starting with an empty stack, a decoder of DICT and CharString data processes bytes sequentially from the start to the end, decoding numbers and operators.

Numbers in DICT and CharString data are represented using an encoding scheme of one or more bytes, as described in  5.3.3.3.

Decoded numbers are immediately pushed to the stack, ready for use as operands for operators that follow.

Operators are encoded with one or two bytes. The byte ranges used for operators do not overlap with the encodings used for numbers. All two-byte operators begin with 0x0c. A few operators also encode data in succeeding bytes.

Decoded operators pop one or more numbers from the stack, then store or process them, and in some cases push new numbers back onto the stack. The maximum number of operands on the CFF2 stack is 513.

The function of operators in a DICT subtable is to create key-value pairs. During decoding, the DICT key is identified by the operator, and the DICT value is obtained by popping items (one or more numbers) from the stack. In well-formed CFF2 data, the number of operands preceding a DICT key operator shall be exactly the number required for that operator; hence, the stack will be empty after the operator is processed. The DICT blend operator is exceptional, neither creating a key-value pair nor leaving the stack empty. Instead, its function is to modify operand values before they are assigned to keys. See 5.3.3.12 for details on the blend operator.

The function of operators in CharString data is the representation of glyph outlines, hints and variations. During decoding, and assuming valid CharString data, most CharString operators pop all the numbers on the stack and do not push any new numbers to the stack. Thus, in CharString decoding, most operators will leave the stack empty. There are three exceptions:

- blend: Pops operands from the stack (but not necessarily all the operands on the stack) and pushes processed numbers back to the stack.

- callsubr: Pops a number from the stack and invokes a local subroutine (subroutines are not restricted regarding the state of the stack).

- callgsubr: Pops a number from the stack and invokes a global subroutine.

These operators are described in detail below (see 5.3.3.10).

##### Stack notation

In specifications for DICT and CharString, this document uses the following notation to describe the state of the stack when particular operators are encountered.

[ — represents the bottom of the stack

] — represents the top of the stack

x, dx, etc. — values on the stack referred to in the description

<number> — a number, either an integer or real number

<integer> — an integer

( ) — delimiters for a group of items that is repeated or optional

* — an item or group occurs 0 or more times

+ — an item or group occurs 1 or more times

... — items at the bottom of the stack that are not involved in the current operation

The following are some expressions using this notation:

[ <number> ] — the stack contains one number

[ <integer> ] — the stack contains one integer

[ <number>+ ] — the stack contains one or more numbers

[ (<number> <number>)+ ] — the stack contains an even number of numbers (using the regular expression-style parentheses for grouping)

[... <number>* <integer> ] — at the top of the stack are 0 or more numbers, followed by an integer

This notation is similarly used to describe the operands interpreted by operators. For example:

- The stack notation specified for the CharString hmoveto operator is [ dx ], indicating that a single operand, dx, is expected.

- The stack notation specified for the CharString rlineto operator is [ (dx dy)+ ], indicating that one or more pairs of operands, dx dy, are expected.

#### Common formats: DICT and INDEX

DICT and INDEX are two common, generic structures that are used for various subtables. This section describes the generic specifications common across various subtables; details specific to particular subtables will follow in later sections.

##### DICT data

A DICT subtable defines a dictionary data structure consisting of key-value pairs. DICTs are used in CFF2 to store offsets to other data blocks, and metadata used across multiple glyphs. In each key-value pair:

- The key is a string from a predefined vocabulary encoded as an operator of one or two bytes. Each type of DICT subtable has a different set of valid keys. A given key shall not be defined more than once in any DICT.

- The value is a number or a sequence of numbers.

DICT data is decoded using a stack, as described in 5.3.3.4. Starting from the beginning of the DICT data, numbers and operators are parsed from the data sequentially until the end of DICT data is reached. Decoded numbers are pushed onto the stack. DICT operators pop numbers from the stack and assign those numbers to the key identified by the operator itself. The number of operands that are popped is specific to each operator.

With the exception of the blend operator, after a DICT operator and its values have been decoded, the stack must be empty. If it is not empty, the subtable is invalid.

While decoding a DICT, if an operator is encountered that is not defined for the current DICT type, the behavior is unspecified.

The DICT format is used for TopDICT (5.3.3.7), FontDICT (5.3.3.8) and LocalDICT (5.3.3.9) subtables. In general, the key-value pairs for a DICT may be specified in any order. However, some ordering rules are specified for the LocalDICT subtable. For each subtable type, default values are specified for certain keys. For keys that have a specified default value, that default value must be used if the corresponding operator for the key is not present with the DICT. Details for each of these subtables is provided below.

Multiple DICT subtables may share encoded data with overlapping byte ranges within the CFF2 table. In this way, one DICT could represent a subset of the data of another DICT.

#### INDEX data

An INDEX is an array of data objects of some type. Objects in the array are accessed by a 0-based index. Each object in the array may be of arbitrary size. Objects in the array are stored contiguously and in order. An offset array provides the location and size of each object, which may be of arbitrary size.

The INDEX format is used in CharStringINDEX, GlobalSubrINDEX and LocalSubrINDEX (5.3.3.10), and FontDICTINDEX (5.3.3.8) subtables. These are described below.

The format of an INDEX is as follows.

INDEX table

| Type | Name | Description |
| --- | --- | --- |
| uint32 | count | Number of objects stored in INDEX |
| uint8 | offsetSize | Offset array element size |
| Offset8 or Offset16 or Offset24 or Offset32 | offsets[count+1] | Array of offsets — offsets are from byte preceding object data. |
| uint8 | data[variable] | Object data — total length of data is the last offset - 1 |

The offsetSize field specifies the number of bytes used to store each offset: 1 for Offset8, 2 for Offset16, 3 for Offset24, or 4 for Offset32.

The offsets array specifies the starting location for each data object according to its index. Offsets in the offset array are relative to the byte that precedes the object data. The first element of the offset array must always be 1. Data objects are stored contiguously and in order, therefore each offset must be less than or equal to the following offset.

An object is retrieved by using the object index to look up an offset in the offsets array and fetching the binary data at the specified offset. The size for each object is determined by subtracting the offset for a given object from the next offset in the offset array. The offset array has count + 1 elements, which provides a length for the last object. Hence, every object has a non-zero offset and a size (which may be zero).

An empty INDEX is represented by a count field with a 0 value and no additional fields. Thus, the total size of an empty INDEX is 4 bytes.

The total size of a non-empty INDEX is:

4 + 1 + offsetSize × (count + 1) + offsets[count] - 1

When creating INDEX data blocks, the smallest possible representation for offsets should be used. For example, if offsets[count] is greater than 255 and less than 65536, then all offsets can be represented as Offset16, and so offsetSize should be set to 2.

The following table shows the layout of INDEX data.

INDEX Data Format

| Type | Name | Description |
| --- | --- | --- |
| uint32 | count | Number of objects stored in INDEX |
| uint8 | offSize | Offset array element size (1, 2, 3 or 4) |
| uint8 or uint16 or uint24 or uint32 | offsets [count+1] | Array of offsets – offsets are from byte preceding object data. |
| uint8 | data [<unlimited>] | total length of data is the last offset - 1 |

#### Decoding DICT and CharString data using a stack

Two of the fundamental data encoding schemes in CFF2 are the DICT and the CharString. Each is a binary data block that represents a sequence of encoded numbers and operators. In order to interpret those sequences, CFF2 uses the abstract data structure of a stack.

##### Description of a stack

A stack is a kind of array conceived as a physical stack of items, which may be manipulated in only two ways:

- place one new item on the top (“push an item to the stack”);

- remove one item from the top (“pop an item from the stack”).

A CFF2 decoder is expected to implement a stack in a suitable manner. The CFF2 stack stores only numbers.

##### The stack-based decoder in CFF2

Starting with an empty stack, a decoder of DICT and CharString data processes bytes sequentially from the start to the end, decoding numbers and operators.

Numbers

Numbers are represented using an encoding scheme of 1 or 2 bytes for integers >= -1131 and <= 1131, a range that is very common in fonts, and additional bytes for integers of larger magnitude and non-integers. For these number encodings, see subclause 5.4.3.3.

Decoded numbers are immediately pushed to the stack, ready for use as operands for forthcoming operators.

Operators

Operators are encoded with 1 or 2 bytes, specified so that they do not overlap with the encodings allocated for numbers. All two-byte operators start with the byte 0x0c. A few operators also encode data in succeeding bytes.

Decoded operators pop one or more numbers from the stack, then store or process them, and in some cases push new numbers back to the stack. The maximum number of operands on the CFF2 stack is 513.

The function of DICT operators is to create key-value pairs. During decoding, the DICT key is identified by the operator, and the DICT value is obtained by popping all the items from the stack (that is, one or more numbers), after which the stack is empty. The DICT operator blend is exceptional, neither creating a key-value pair nor leaving the stack empty. Instead, its function is to modify values before they are assigned to DICT keys. See subclause 5.4.3.13 for details on the blend operator.

The function of CharString operators is to decode data describing glyph outlines, hints and variations. During decoding, most CharString operators pop all the numbers on the stack and do not push any new numbers to the stack; thus CharString operators leave the stack empty. There are three exceptions:

- blend: pops operands from the stack (but not necessarily all the operands on the stack) and pushes processed numbers back to the stack;

- callsubr: pops a number from the stack which invokes a local subroutine (subroutines are not restricted regarding the state of the stack);

- callgsubr: invokes a global subroutine.

##### Stack notation

In specifications for DICT and CharString, this document uses the following notation to describe the state of the stack when particular operators are encountered.

[ — represents the bottom of the stack

] — represents the top of the stack

x, dx, etc. — values on the stack referred to in the description

<number> — a number, either an integer or real number

<integer> — an integer

( ) — delimiters for a group of items that is repeated or optional

* — an item or group occurs 0 or more times

+ — an item or group occurs 1 or more times

... — items at the bottom of the stack that are not involved in the current operation

##### Examples

[ <number> ] — the stack contains one number

[ <integer> ] — the stack contains one integer

[ <number>+ ] — the stack contains 1 or more numbers

[ (<number> <number>)+ ] — the stack contains an even number of numbers (using the regular expression-style parentheses for grouping)

[... <number>* <integer> ] — at the top of the stack are 0 or more numbers, followed by an integer ##

#### Header

The CFF2 table begins with a header having the following format.

CFF2 header

| Type | Name | Description |
| --- | --- | --- |
| uint8 | majorVersion | Format major version (set to 2). |
| uint8 | minorVersion | Format minor version (set to 0). |
| uint8 | headerSize | Header size (set to 5) |
| uint16 | topDictLength | Length of TopDICT subtable. |

The TopDICT subtable shall always start immediately after the header—for this version, at offset 5 from the start of the CFF2 table.

The headerSize field shall be used when locating the start of the TopDICT subtable. It is provided so that future versions of the format can introduce additional data between the topDictLength field and the TopDICT subtable in a manner that is compatible with older implementations.

The sum headerSize + topDICTLength is the location within the CFF2 table of the required GlobalSubrINDEX subtable.

#### DICT data

Used in and with: TopDICT, FontDICT, PrivateDICT, FontDICTINDEX and FontDICTSelect

A DICT subtable defines a “dictionary” data structure consisting of key-value pairs. DICTs are used in CFF2 to store offsets to other data blocks, and metadata used across multiple glyphs. In each key-value pair:

- The key is a string from a predefined vocabulary, and is encoded by an operator of one or two bytes. Each type of DICT subtable has a different set of keys which are valid. A specific key may not be defined more than once in any DICT.

- The value is a number or a sequence of numbers.

DICT data is decoded using a stack: see subclause 5.4.3.5. Starting from the beginning of the DICT data, numbers and operators are parsed from the data sequentially until the end of DICT data is reached. Decoded numbers are pushed onto the stack. DICT operators pop numbers from the stack, and assign those numbers to the key identified by the operator itself.

With the exception of the blend operator, after a DICT operator and its values have been decoded, the stack should be empty. If it is not empty, the subtable is invalid.

In general, the key-value pairs for a DICT may be specified in any order. However, some ordering rules are specified for PrivateDICT.

While decoding a DICT, if an operator is encountered that is not defined for the current DICT type, the behavior is unspecified.

It is permitted for multiple DICTs to share the same encoded data. In this way, one DICT could represent a subset of the data of another DICT.

Offsets and lengths of DICT data blocks

The methods to determine the offset and the length of the three different types of DICT are given in the following table.

| DICT type | Offset | Length |
| --- | --- | --- |
| TopDICT | 5 | stored in Header as TopDICTLength |
| FontDICT | stored in FontDICTINDEX | stored in FontDICTINDEX |
| PrivateDICT | stored in the PrivateDICTOffset key of a FontDICT | stored in the PrivateDICTOffset key of a FontDICT |

#### TopDICT

The TopDICT is a DICT that provides offsets to various parts of the CFF2 table, and also provides values relating to unitsPerEm in the head table (subclause 5.2.3). TopDICT data immediately follows Header data (see subclause 5.4.3.6) in a CFF2 table. Every CFF2 table requires one TopDICT. For a description of DICTs, see subclause 5.4.3.7.

##### TopDICT operator summary

The following table lists the 5 operators allowed in a TopDICT, the bytes by which they are encoded in DICT data as hexadecimal and decimal, and whether or not they are required.

Top DICT Operator Entries

| Hex Value | Dec Value | Name | Required | Default |
| --- | --- | --- | --- | --- |
| 0x11 | 17 | CharStringINDEXOffset | yes | – |
| 0x18 | 24 | VariationStoreOffset | only for fonts with variations | – |
| 0x0c24 | 12,36 | FontDICTINDEXOffset | yes | – |
| 0x0c25 | 12,37 | FontDICTSelectOffset | no | – |
| 0x0c07 | 12,7 | FontMatrix | no | 0.001 0 0 0.001 0 0 |

##### TopDICT operator specifications

The following specifies the 5 keys that are valid in a TopDICT.

CharStringINDEXOffset: 0x11 (17)

Stack: <integer>
Required. This entry defines the offset to the CharStringINDEX data, from the start of the CFF2 table.

VariationStoreOffset: 0x18 (24)

Stack: <integer>
Required if the font uses variations. Forbidden in non-variable fonts. This entry defines the offset to the VariationStore data, from the start of the CFF2 table.

FontDICTINDEXOffset: 0x0c24 (12, 36)

Stack: <integer>
Required. This entry defines the offset to the FontDICTINDEX data, from the start of the CFF2 table.

FontDICTSelectOffset: 0x0c25 (12, 37)

Stack: <integer>
Optional. This entry defines the offset to the FontDICTSelect data, from the start of the CFF2 table. If the CFF2 has only one FontDICT, and therefore no need for FontDICTSelect data, then FontDICTSelectOffset must not be defined.

FontMatrix: 0x0c07 (12, 7)

Stack: <number> 0 0 <number> 0 0
Required if unitsPerEm is not equal to 1000. This entry defines the scale factor for glyph coordinates within the em square, similar to the unitsPerEm field in the head table (subclause 5.2.3). However, a reciprocal value is used (thus 1 / unitsPerEm) and this value occurs twice in the 6 operands. For the common case where unitsPerEm is 1000, a default key-value pair is created with the value “0.001 0 0 0.001 0 0” and the FontMatrix operator should be omitted.

The Binary Coded Decimal number format (see subclause 5.4.3.3) is used to represent 1 / unitsPerEm. For example, if unitsPerEm is 2000, then the value 0.0005 must be represented as Binary Coded Decimal.

NOTE 	The origin of the TopDICT FontMatrix key is a 2×3 transformation matrix. However, in CFF2 only matrices with uniform horizontal and vertical scaling without translation are permitted, thus the 1st and 4th entries must be identical and the remaining entries must be zero.

#### FontDICT

A FontDICT is a DICT (see subclause 5.4.3.7) data block that provides offsets to a related PrivateDICT (subclause 5.4.3.10), which in turn provides metadata relating to hinting, variations and subroutines for a subset of CharStrings. Each CharString in a CFF2 table requires access to a PrivateDICT, and this is provided by assigning a FontDICT to each CharString.

A FontDICT exists only to point to a PrivateDICT. Hence there is only one operator allowed in a FontDICT, namely PrivateDICTOffset. FontDICTs are stored in FontDICTINDEX.

It is common for all CharStrings in a font to use the same PrivateDICT, and thus the same FontDICT. When there is only one FontDICT in a CFF2 table, all CharStrings are assigned to it. When there are multiple FontDICTs, the FontDICTSelect structure is required, in order to assign FontDICTs to CharStrings.

There is no specified limit to the number of FontDICTs.

PrivateDICTOffset: 0x12 (18)

Stack: <integer> <integer>
Required. The two numbers are, respectively, the size and offset of the related PrivateDICT data block. The offset is from the start of the CFF2 table. This is the only operator allowed in a FontDICT. If the related PrivateDICT is empty, then the numbers 0 0 must be used.

##### FontDICTINDEX

The FontDICTINDEX is an INDEX (subclause 5.4.3.4) data block that stores all the FontDICT data blocks in the CFF2 table. At least one FontDICT is required, therefore FontDICTINDEX requires at least one entry. The location of FontDICTINDEX in the CFF2 table is given by the FontDICTINDEXOffset key in TopDICT.

If the CFF2 table has more than one FontDICT, then a FontDICTSELECT structure is required, in order to assign glyphs to FontDICTs.

An upper limit to the number of FontDICT structures in a CFF2 table is not specified.

##### FontDICTSelect

The FontDICTSelect data block assigns each CharString (i.e. each glyph) in the CFF2 table to a particular FontDICT in FontDICTINDEX. The location of FontDICTSelect in the CFF2 table is given by the FontDICTSelectOffset key in TopDICT.

In many fonts all CharStrings use the same FontDICT (and therefore the same PrivateDICT). Such fonts do not store FontDICTSelect data and their TopDICT omits the FontDICTSelectOffset key.

There are three formats defined for FontDICTSelect: format 0, format 3 and format 4. All formats provide numGlyphs mappings from a CharString identifier (gid) to a FontDICT identifier (fd), where numGlyphs is the number of CharStrings in CharStringINDEX. Formats 3 and 4 map ranges of gids onto a single fd, which often makes those formats a better choice for efficiency.

Format 0

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 0 |
| uint8 | fds [numGlyphs] | FontDICT selector array |

Each element of the fds array represents the FD index of a Font DICT in the FDArray. This format should be used when the FD indices are in a fairly random order. The number of glyphs (numGlyphs) is the value of the count field in the CharStrings INDEX.

Format 3

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 3 |
| uint16 | nRanges | Number of ranges |
| Range3 | range3[nRanges] | Array of Range3 records (see below) |
| uint16 | sentinel | Sentinel gid |

The format of a Range3 record is as follows:

Range3 Record Format

| Type | Name | Description |
| --- | --- | --- |
| uint16 | first | First gid in range |
| uint8 | fd | FontDICT index for all glyphs in range |

Each Range3 describes a group of sequential gids that have the same fd index. Each range includes gids from the first gid in the range record up to, but not including, the first gid of the next range record. Records in the Range3 array must be in increasing order of first gid. The first range must have a first gid of 0. A sentinel gid follows the last range element and serves to delimit the last range in the array. The sentinel gid is set equal to numGlyphs, the number of glyphs in the font. That is, its value is 1 greater than the last gid in the font. The largest value for fd is 255.

NOTE 1 	Since the sentinel gid delimits the last range in the array, its value, encoded as a uint16, cannot exceed the value 65535. Therefore, the last gid encoded when using FontDICTSelect Format 3 cannot exceed 65534.

Format 4

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 4 |
| uint32 | nRanges | Number of ranges |
| Range4 | range4[nRanges] | Array of Range4 records (see below) |
| uint32 | sentinel | Sentinel gid |

Format 4 is identical to Format 3, except that it accommodates more than 65534 glyphs by using a uint32 type for the nRanges and sentinel fields, and a Range4 record array, which accommodates FontDICTINDEXes of a length up to 65535.

The format of a Range4 record is as follows:

Range4 Record Format

| Type | Name | Description |
| --- | --- | --- |
| uint32 | first | First gid in range |
| uint16 | fd | FontDICT index for all glyphs in range |

The Range4 format differs from the Range3 only in that it accommodates more than 65536 glyphs by using a uint32 for the first gid field, and accommodates more than 255 FontDICTs by using a uint16 field for the fd index.

NOTE 2 	While FontDICTSelect format 4 allows for more than 65535 glyphs, other parts of the OFF format, such as the numGlyphs field of the ‘maxp’ table, are still constrained to 65535 glyphs.

#### PrivateDICT

A PrivateDICT is a DICT (subclause 5.4.3.7) subtable providing metadata relating to hinting, subroutines and variations that are used by a subset of CharStrings. Many fonts use only one PrivateDICT, whose metadata therefore applies to all CharStrings. Access to a PrivateDICT, in terms of length and offset of the subtable, is provided by the PrivateDICTOffset key in a FontDICT.

When different subsets of CharStrings use different PrivateDICTs, multiple FontDICTs must be defined in FontDICTINDEX, each pointing to a PrivateDICT. In such cases a FontDICTSelect structure is also required in order to allocate FontDICTs to CharStrings. For more information on FontDICTs, see subclause 5.4.3.9.

A PrivateDICT may be empty, requiring no storage. In this case, the PrivateDICTOffset key of FontDICT must set size and offset to *0*, and default values will be used for vsindex, BlueScale, BlueShift, BlueFuzz, LanguageGroup and ExpansionFactor, as set out in the table below.

Some PrivateDICT values relating to hinting can undergo interpolation with font variations using the blend operator. These are identified as “blendable” in the table below.

NOTE	Non-integer numbers in DICT data, such as the values for BlueScale and ExpansionFactor, are represented as Binary Coded Decimal bytes.

When to use multiple PrivateDICTs

A CFF2 table requires at least one pair of FontDICT and PrivateDICT (even if the PrivateDICT is empty), and many fonts only have one pair. However, it may be beneficial to use more than one FontDICT (and hence more than one PrivateDICT) when the glyphs in the font are of multiple distinct styles; for example, when the font contains a Latin set plus an Emoji set, or a Latin set plus a Korean Hangul set. In such cases, specifying hint metadata for each glyph style in separate PrivateDICT structures will likely allow more compact and efficient CharString encoding, with better use of hinting zones, smaller index values in callsubr subroutine calls, and fewer operands in blend operations — resulting in less data in total. For similar reasons, the facility for multiple FontDICT/PrivateDICT pairs allow easier merging of multiple fonts whose glyphs already have hints, variation data and subroutines.

##### PrivateDICT operator summary

The following table lists all PrivateDICT operators alongside their encoding as hexadecimal and decimal bytes, their default value (if any), their general purpose, and whether they may be blended.

| Hex Value | Dec Value | Name | Default | Purpose | Blendable |
| --- | --- | --- | --- | --- | --- |
| 0x13 | 19 | LocalSubrINDEXOffset | – | subroutines | no |
| 0x16 | 22 | vsindex | 0 | variation | no |
| 0x17 | 23 | blend | – | variation | yes |
| 0x06 | 6 | BlueValues | – | hinting | yes |
| 0x07 | 7 | OtherBlues | – | hinting | yes |
| 0x08 | 8 | FamilyBlues | – | hinting | no |
| 0x09 | 9 | FamilyOtherBlues | – | hinting | no |
| 0x0c09 | 12,9 | BlueScale | 0.039625 | hinting | no |
| 0x0c0a | 12,10 | BlueShift | 7 | hinting | no |
| 0x0c0b | 12,11 | BlueFuzz | 1 | hinting | no |
| 0x0a | 10 | StdHW | – | hinting | yes |
| 0x0b | 11 | StdVW | – | hinting | yes |
| 0x0c0c | 12,12 | StemSnapH | – | hinting | yes |
| 0x0c0d | 12,13 | StemSnapV | – | hinting | yes |
| 0x0c11 | 12,17 | LanguageGroup | 0 | hinting | no |
| 0x0c12 | 12,18 | ExpansionFactor | 0.06 | hinting | no |

##### Subroutine operator

LocalSubrINDEXOffset: 0x13 (19)

Stack: [ <integer> ]
Defines a location for the local subroutines that CharStrings may access.

This specifies the offset, from the start of the PrivateDICT, to the LocalSubrINDEX. The LocalSubrINDEX stores all the local subroutines that may be used by CharStrings associated with this PrivateDICT, using the callsubr operator. If there are no local subroutines, then the LocalSubrINDEXOffset operator must be omitted. For more information on subroutines, see subclause 5.4.3.11.

##### Variation operators

Two variation operators are defined that depend on various other parts of the CFF2 table and on other tables in the OFF font. For their full descriptions, see Font Variations in CFF2 (subclause 5.4.3.13). A summary of their function is given below.

vsindex: 0x16 (22)

Stack: [ <integer> ]
Activates a particular list of variation regions from VariationStore.

When this operator is used in a PrivateDICT, then the active list of regions (also known as an ItemVariationData) is used not only within the PrivateDICT, but also for all CharStrings that reference that PrivateDICT. However, a vsindex operator used inside a CharString always takes precedence.

NOTE 	The vsindex operator is encoded as 0x16 in PrivateDICT data, unlike in CharString data where it is encoded as 0x0f.

blend: 0x17 (23)

Stack: [... <number>+ <integer> ]
Pops a certain number of operands from the stack, processes them according to the font variations interpolation algorithm, then pushes processed numbers back onto the stack.

Blending hinting values is useful, for example, to interpolate BlueZones between those required for the default Regular instance and those required for the Bold instance of a font.

For the operation of blend, see CFF2 Variation Store operators (subclause 5.4.3.13.4).

NOTE 	The blend operator is encoded as 0x17 in PrivateDICT data, unlike in CharString data where it is encoded as 0x10.

##### Hinting operators

BlueValues: 0x06 (6)

Stack: [ <deltaArray> ]
Font-wide vertical alignment zones.
Blendable: yes

The value represented by BlueValues is an array containing an even number of integers taken in pairs, and which follow a small number of rules:

- The first integer in each pair is less than or equal to the second integer in that pair.

- The first pair is the baseline overshoot position and the baseline. This is a bottom-zone.

- All subsequent pairs describe top-zones, that is, alignment zones for the tops of character features, for example, x-height and x-height overshoot position, ascender-height and ascender-height overshoot position, cap-height and cap-height overshoot position, figure-height and figure-height overshoot position.

- Up to seven pairs may be given in the BlueValues array; the first pair must be the baseline pair.

- Different pairs must be at least 3 units apart from each other and from pairs in OtherBlues, as described in the following section. (This minimum distance can be modified by the optional BlueFuzz entry in the PrivateDICT; see the definition of BlueFuzz, that follows.)

- The maximum difference between values in one pair is constrained as described under the description of BlueScale, that follows.

Despite the names often given to the various alignment zones described by the BlueValues, renderers have no built-in notions of which parameters apply to which glyphs. Each zone helps to control the alignment of any and all characters with character level hints that fall within the zone.

Example

Consider the following array that represents three alignment zones in a typeface, being baseline (at 0 with negative overshoot at -15), cap-height (at 700 with overshoot at 715) and x-height (at 547 with overshoot at 559), thus:

[-15 0 700 715 547 559]

However, because this must be stored as a deltaArray, the encoded numbers in the PrivateDICT would be, in fact:

[-15 15 700 15 -168 12]

Notice that this saves 2 bytes compared with storing the absolute numbers, because smaller numbers are encoded with fewer bytes in DICT data.

OtherBlues: 0x07 (7)

Stack: [ <deltaArray> ]
Additional bottom alignment zones.
Blendable: yes

The optional OtherBlues entry in the PrivateDICT is associated with an array of pairs of integers similar to the BlueValues array. However, the OtherBlues array describes bottom-zones only. For example, these may include: descender-depth overshoot position and descender-depth, superior baseline overshoot position and superior baseline, and ordinal baseline overshoot position and ordinal baseline. Up to five pairs (10 integers) may be specified in the OtherBlues array. Numbers in a pair must be in ascending order, with the same restriction on the maximum difference in a pair. Pairs must be at least 3 units apart from all other pairs, including those in the BlueValues array. (This minimum distance can be modified by the optional BlueFuzz entry in the PrivateDICT.)

FamilyBlues: 0x08 (8)

Stack: [ <deltaArray> ]
Family-wide vertical alignment zones.
Blendable: no

When different styles of the same font family are mixed in text, it is often desirable to coordinate their x-heights, cap-heights, and other alignments so that they will be the same at small sizes. For example, at 72 pixels per inch, the x-height of a 10-point roman face might be 5.4 pixels while the boldface x-height might be 5.6 pixels. If the roman face is the regular for the family, the renderer can render both faces with an x-height of 5 pixels instead of letting the boldface jump to 6 while the roman is still at 5. However, at 100 points, the roman x-height will be 54 pixels and the bold x-height will be 56.

You can include information about the dominant alignment zones in a font family so that this consistency can be enforced. When enabled, if the difference between a font’s alignment and its family’s standard alignment is less than 1 pixel, then the renderer will use the standard alignment instead of the normal alignment for that font. Thus at 10 points in the previous example, the difference is 5.6 − 5.4 = 0.2 pixels so the standard is used. At 100 points, the difference is 56 − 54 = 2, so the specific x-height for the font is used. Family alignment values are identical to individual font alignment values; i.e., they are things like x-height, x-height overshoot, etc. The PrivateDICT entries are as follows.

- The value associated with FamilyBlues is an array containing an even number of integers taken in pairs. The rules governing the contents of this array are analogous to those of the BlueValues array.

- The value associated with FamilyOtherBlues is an array containing an even number of integers taken in pairs. The rules governing the contents of this array are analogous to those of the OtherBlues array.

Typically, the FamilyBlues and FamilyOtherBlues entries will simply be copied from the BlueValues and OtherBlues of the regular face in the family. Each font in a family (except the regular) must have these entries if it is to have family alignment properties. Of course, if these entries are not present, then only a font’s own alignment hints will be considered.

FamilyOtherBlues: 0x09 (9)

Stack: [ <deltaArray> ]
Family-wide bottom alignment zones.
Blendable: no

See FamilyBlues for description.

BlueScale: 0x0c09 (12,9)

Stack: [ <number> ]
Related to point size at which to deactivate overshoot suppression. Blendable: no

The optional BlueScale entry in the PrivateDICT controls the point size at which overshoot suppression ceases. This point size varies with the number of device pixels per inch available on the device where the font is being rendered.

- For point sizes that occupy fewer device pixels than the BlueScale value results in for a given device, overshoot suppression is performed. All features falling in an alignment zone are rendered at the same pixel height.

- For point sizes that occupy the same number or a greater number of device pixels than the BlueScale value results in, overshoot suppression is turned off, thus allowing overshoots to occur. (This behavior may be modified by the optional BlueShift setting; see the definition of BlueShift, that follows.)

The BlueScale value is a number directly related to the number of pixels tall that one-character space unit will be before overshoot suppression is turned off. The default value of BlueScale is 0.039625, which corresponds to 10 points at 300 dpi. (Thus if that value is acceptable, a PrivateDICT does not need to define BlueScale.) A simple formula that relates point size as rendered on a 300-dpi device to the BlueScale value is:

BlueScale = (pointsize − 0.49) / 240

The formula provides a convenient number that font producers can use to determine at what integer point size overshoot suppression should be off. However, the exact point size at which overshoot suppression ceases is actually 0.49 points less (at 9.51 points using the default value of BlueScale) than the value of pointsize used in the formula. Adobe recommends using the adjustment shown in the formula so that the change in overshoot suppression behavior occurs at an exact point size unlikely to be used in practice.

For example, if you wish overshoot suppression to turn off at 11 points on a 300-dpi device, you should set BlueScale to (11 − 0.49) / 240 or 0.04379. With this one setting of BlueScale, overshoot suppression will turn off at proportionately smaller point sizes on higher resolution output devices or larger point sizes on lower-resolution devices such as displays.

BlueShift: 0x0c0a (12,10)

Stack: [ <integer> ]
Overshoot enforcement. Optional, but relevant even if Flex is not used.
Blendable: no

The optional BlueShift entry in the PrivateDICT adds another capability to the treatment of overshoot behavior. The value of BlueShift is an integer that indicates a character space distance beyond the flat position of alignment zones at which overshoot enforcement for character features occurs. The default value of BlueShift is 7. The single setting of BlueShift applies to all alignment zones, regardless of where their over- shoot positions lie.

When a character’s size is less than that expressed by BlueScale, character features that fall within alignment zones have their overshoots suppressed. For characters larger than the BlueScale size, character features that fall beyond the flat position of an alignment zone (above for top-zones, below for bottom-zones) by a character space distance equal to or greater than the value of BlueShift will overshoot, while character features closer to the flat position than the BlueShift value will overshoot only if their device space distance is at least one-half pixel.

If one or more flex operators occurs in any CharStrings using the current PrivateDICT (that is, any of flex, hflex, hflex1, flex1), then the BlueShift value must be greater than flex depth. Since the default value of BlueShift is 7, this entry must be set explicitly if flex depth is greater than 6. For example, if flex depth is 8, then set BlueShift to 9. If flex depth is 6 or less, then BlueShift may be omitted.

BlueFuzz: 0x0c0b (12,11)

Stack: [ <integer> ]
Extends the range of alignment zones. Optional.
Blendable: no

The optional BlueFuzz entry in the PrivateDICT is an integer value that specifies the number of character space units to extend (in both directions) the effect of an alignment zone on a horizontal stem. If the top of a horizontal stem is within BlueFuzz units (in character space) outside of a top-zone, the interpreter will act as if the stem top were actually within the zone; the same holds for the bottoms of horizontal stems in bottom-zones. The default value of BlueFuzz is 1. BlueFuzz has been a convenient means for compensating for slightly inaccurate coordinate data. The effect of a non-zero value for BlueFuzz can usually be better achieved by adjusting the sizes of the alignment zones. Adobe suggests that new fonts not rely on it and disable the feature by explicitly setting BlueFuzz to 0 in the PrivateDICT.

NOTE 	Because a non-zero value for BlueFuzz extends the range of alignment zones, alignment zones must be declared at least (2 × BlueFuzz + 1) units apart from each other. Therefore, a default BlueFuzz value of 1 implies that alignment zones should be at least 3 units apart from each other.

StdHW: 0x0a (10)

Stack: [ <number> ]
Dominant horizontal stem width.
Blendable: yes

StdVW: 0x0b (11)

Stack: [ <number> ]
Dominant vertical stem width.
Blendable: yes

StemSnapH: 0x0c0c (12,12)

Stack: [ <deltaArray> ]
Array of common horizontal stem widths.
Blendable: yes

StemSnapV: 0x0c0d (12,13)

Stack: [ <deltaArray> ]
Array of common vertical stem widths.
Blendable: yes

Stem Width Information, controlled by these 4 operators, is a mechanism to tell the renderer about standard stem widths in a font so that the renderer can ensure consistency at small sizes. If a particular stem is slightly wider or narrower than standard, either by design or as a result of a small error in creating the font, then at small sizes where a single pixel difference would be very noticeable, the renderer can render the stem as though it had the standard width. However, at large sizes where a single pixel difference will produce only a subtle visual effect, the stem will be allowed to deviate from the standard.

When the difference between a standard stem width and a particular stem width is small, the standard width is used. For example, if at 10 points a standard stem width corresponds to 1.4 pixels wide and a particular stem is 1.6 pixels wide, both can be rendered as a 1-pixel wide stem. However, at 100 points the standard stem would be rendered as 14 pixels wide and the particular stem would be rendered as 16 pixels wide. The information that the renderer needs appears in the following PrivateDICT entries.

- StdHW takes a number expressing the dominant width of horizontal stems (measured vertically in FUnits).

- StdVW takes a number expressing the dominant width of vertical stems (measured horizontally in FUnits). Typically, this will be the width of straight stems in lower case letters. (For an italic font, give the width of the vertical stem measured at an angle perpendicular to the stem direction.)

- StemSnapH is an array of up to 12 numbers of the most common widths (including the dominant width given in StdHW) for horizontal stems (measured vertically). These widths must be sorted in increasing order.

- StemSnapV is an array of up to 12 numbers of the most common widths (including the dominant width given in the StdVW) for vertical stems (measured horizontally). These widths must be sorted in increasing order. For example, you might include widths for straight and curved stems in upper and lower case letters. For an italic font, this array should be empty.

If these Stem Width Information hints are not present in the PrivateDICT, then each stem is rendered according to its own definition in the CharString (as modified by any other CharString hints).

LanguageGroup: 0x0c11 (12,17)

Stack: [ <integer> ]
Identifies language group of font.
Blendable: no

Certain groups of written languages share broad aesthetic characteristics. Identification of such language groups can prove useful for accurate character rendering.

The value of the entry LanguageGroup is an integer that indicates the language group of the CharStrings (glyphs) using this PrivateDICT. If the PrivateDICT does not contain this entry, or if the given value is not recognized, then the value of LanguageGroup defaults to zero.

Two language groups are defined:

- group 0 consists of languages that use Latin, Greek, Cyrillic, and similar alphabets. Since the value of the LanguageGroup entry defaults to 0, a PrivateDICT containing glyphs corresponding to one of these languages does not need to contain this entry.

- group 1 consists of Chinese ideographs and similar character sets, including Japanese Kanji and Korean Hangul. A PrivateDICT that contains glyphs corresponding to one of these languages should define LanguageGroup.

ExpansionFactor: 0x0c12 (12,18)

Stack: [ <number> ]
Provides control over rendering of counters.
Blendable: no

The optional ExpansionFactor entry is a real number that gives a limit for changing the size of a character bounding box during the processing that adjusts the sizes of counters in fonts of LanguageGroup 1. The default value of ExpansionFactor is 0.06. At small point sizes or low resolutions, the system may have to accept irregular counters rather than violate this limit. Bar code fonts or logos that need counter control may benefit by setting LanguageGroup to 1 and increasing the ExpansionFactor limit to a larger amount such as 0.5 or more.

##### Operator ordering restrictions

The OtherBlues operator, if used, must occur after the BlueValues operator.

The FamilyOtherBlues operator, if used, must occur after the FamilyBlues operator.

#### CharString

The CharString format provides a method for compact encoding of glyphs, namely path data, hint data and variation data. The CFF2 CharString specification updates previous definitions of CharString and is intended for use only with a CFF2 font table in an OFF font file.

Like DICT data (subclause 5.4.3.7), the CharString is a binary data block that consists of a sequence of encoded numbers and operators, and is intended to be decoded using a stack. See subclause 5.4.3.5 for details.

##### CharStringINDEX

All CharStrings in a CFF2 table are stored in the CharStringINDEX data block. This is located at the offset specified by the CharStringINDEXOffset key in TopDICT. Indices in CharStringINDEX are used universally as glyph identifiers elsewhere in the OFF specification.

##### CharString operator summary

With three exceptions, every operator processes all operands from the stack and leaves the stack empty. Thus, arguments are generally supplied only for the next operator. The exceptions are the variation operator blend and the subroutine operators callsubr and callgsubr, which all process only a single item or a limited number of items at the top of the stack. If an unrecognized operator is encountered, the behavior is unspecified.

The following table lists all CharString operators alongside their encoding as hexadecimal and decimal bytes, the type of operator, and whether or not it clears the stack. All 2-byte operators start with the byte 0x0c (12). For convenience the bytes introducing numbers are also shown; for the full specification of numbers in CharStrings, see subclause 5.4.3.3.

| Hex Value | Dec Value | Operator | Type | Clears stack |
| --- | --- | --- | --- | --- |
| 0x01 | 1 | hstem | hinting | yes |
| 0x03 | 3 | vstem | hinting | yes |
| 0x04 | 4 | vmoveto | path | yes |
| 0x05 | 5 | rlineto | path | yes |
| 0x06 | 6 | hlineto | path | yes |
| 0x07 | 7 | vlineto | path | yes |
| 0x08 | 8 | rrcurveto | path | yes |
| 0x0a | 10 | callsubr | subroutine | no |
| 0x0f | 15 | vsindex | variation | yes |
| 0x10 | 16 | blend | variation | no |
| 0x12 | 18 | hstemhm | hinting | yes |
| 0x13 | 19 | hintmask | hinting | yes |
| 0x14 | 20 | cntrmask | hinting | yes |
| 0x15 | 21 | rmoveto | path | yes |
| 0x16 | 22 | hmoveto | path | yes |
| 0x17 | 23 | vstemhm | hinting | yes |
| 0x18 | 24 | rcurveline | path | yes |
| 0x19 | 25 | rlinecurve | path | yes |
| 0x1a | 26 | vvcurveto | path | yes |
| 0x1b | 27 | hhcurveto | path | yes |
| 0x1d | 29 | callgsubr | subroutine | no |
| 0x1e | 30 | vhcurveto | path | yes |
| 0x1f | 31 | hvcurveto | path | yes |
| 0x0c22 | 12,34 | hflex | path | yes |
| 0x0c23 | 12,35 | flex | path | yes |
| 0x0c24 | 12,36 | hflex1 | path | yes |
| 0x0c25 | 12,37 | flex1 | path | yes |
| 0x1c | 28 | int16 value | number | — |
| 0x20..0xf6 | -107..107 | 1-byte number | number | — |
| 0xf7..0xfa | 108..1131 | 2-byte number | number | — |
| 0xfb..0xfe | -1131..-108 | 2-byte number | number | — |
| 0xff | 255 | Fixed format value | number | — |

##### CharString organization

A complete CharString, including its subroutines, has the following structure:

- (hstem or hstemhm)*
Any number of horizontal stem hints, followed by

- (vstem or vstemhm)*
Any number of vertical stem hints, followed by

- (cntrmask)*
Any number of counter mask hints, followed by

- path data
A path consisting of any number of contours. Each contour consists of one of the moveto operators followed by any number of line and curve operators. Within the path data there can be any number of hintmask operators.

The blend operator may be used in all of the path operators and in all of the hinting operators except hintmask and cntrmask. If the vsindex operator is used, it must be used before the first blend operator. Only one set of VariationRegions may be used within a particular CharString.

Any part of CharString data may be packaged as a subroutine.

##### Subroutines

A subroutine is a portion of CharString data that is invoked (or “called”) from within other CharString data. Multiple CharStrings may efficiently call the same subroutine, thereby saving significant amounts of space.

An example of a subroutine could be data that describes a diacritic or serif shape that is identical in multiple glyphs. In practice, the identification of portions of CharString data suitable for becoming subroutines is usually automated. There are no restrictions on how CharString data may be split into subroutines; subroutines do not need to represent complete contours, or even a complete set of operands with their operator.

There are two types of subroutine: * global subroutines are stored in GlobalSubrINDEX and may be called by any CharString; * local subroutines are stored in a LocalSubrINDEX and may be called only by CharStrings that have access to that LocalSubrINDEX via their PrivateDICT.

A single CharString may use both types of subroutine.

Every CFF2 table includes a GlobalSubrINDEX. If there are no global subroutines to store, then an empty GlobalSubrINDEX is required. See subclause 5.4.3.4 for decoding and storing empty INDEX structures.

CFF2 compilers use subroutines in order to minimize file size. However, using subroutines is not mandatory. Avoiding subroutines when creating CFF2 tables may result in faster encoding and potentially a smaller file size in subsequent file compression steps.

Nesting subroutines

Nesting of subroutines (one subroutine calling another subroutine) is allowed up to a maximum of 10 levels. Recursion (one subroutine calling itself, or any other subroutine in its nested subroutines) is not allowed.

Subroutine bias

The operand supplied to a callsubr or callgsubr operator must be added to the subroutine bias number before being used as an index into LocalSubrINDEX or GlobalSubrINDEX. The bias is calculated from nSubrs, the number of subroutines in the given INDEX:

- If nSubrs is in the range [0,1239] then bias = 107.

- If nSubrs is in the range [1240,33899] then bias = 1131.

- If nSubrs is >= 33900 then bias = 32768.

This technique allows subroutine identifiers to be specified using negative as well as positive numbers, efficiently utilizing the available number ranges.

Subroutines and the stack

The subroutine operators callsubr and callgsubr, unlike most other operators, do not clear the stack. For example, several numbers can be pushed onto the stack, then a subroutine call occurs, and the subroutine pops the operands for a path drawing operation.

Implementing subroutines with a “return stack”

Implementing subroutines in a CFF2 decoder requires keeping track of the location to which the subroutine should return after it has completed. The return location is in fact the byte immediately after that of the operator (callsubr or callgsubr) that called the subroutine. Because of potential nesting, it is recommended to implement a “return stack” so that multiple return locations can be stored. Care must be taken to check if the return location is beyond the end of the CharString or subroutine data from which the subroutine was called, in which case the decoder must perform one or more extra “return” steps (if it was called from another subroutine) or terminate decoding (if it was called from a CharString).

##### Paths

Summary

A path decoded from a CharString consists of one or more contours, where each contour consists of a start point followed by one or more line segments and cubic Bézier curves. Line segments are defined by an implicit start point and provided x and y coordinates. Curves are defined by an implicit start point and 3 provided sets of x and y coordinates.

Decoding paths

Decoding CharString data into a path involves keeping track of the “current point”, initially at (0, 0), and processing three basic types of operator:

- moveto operators start a new contour at a provided point;

- lineto operators define a line segment from the current point to a provided point;

- curveto operators define a cubic Bézier curve which starts at the current point, has two provided off-curve control points, and a provided end-point.

All moveto, lineto and curveto operators use relative coordinates: each point is defined as a relative coordinate offset from the point before it. This rule also applies to the three points of a curve operation, where the first provided point is relative to the current point, the second is relative to the first, and the third is relative to the second. After any path construction operation, the current point is then set to the last of the given points.

Operators for move, line and curve exist in multiple forms to avoid explicit representation of the number 0, which is a common relative measurement in paths. Also, the number of items on the stack can trigger multiple sequential path operations of the same type. Some operators combine curve and line specifications in a single operation.

Path direction

A contour that is to be filled must be defined in a counterclockwise orientation. A contour that is to be left unfilled must be defined in a clockwise orientation. If you imagine walking along a contour in the direction it is defined, then a filled area should be on your left.

NOTE 	These orientations are opposite to those specified in the ‘glyf’ table (subclause 5.3.4).

Overlapping contours

Overlapping contours are supported in CFF2 fonts. CFF2 renderers must use the non-zero winding number rule, rather than the even-odd rule. Thus one counter-clockwise contour completely inside another counter-clockwise contour does not introduce any transparency; both counters are rendered opaque. This behavior is necessary in order to support variable font data, where overlapping contours are common. Care must be taken therefore when converting CFF2 CharString data into CFF CharString data (or any other outline format that uses the even-odd rendering rule), for example by using an overlap removal operation.

Non-printing glyphs

Non-printing glyphs, such as the space character, require no path, hints or variation data. Therefore, no CharString data is stored. In the CharStringINDEX, the offset for such an empty CharString is identical to the offset for the subsequent CharString.

NOTE 	Horizontal and vertical metrics for non-printing glyphs are defined and varied using the usual methods.

Contours that end with a curve

Unlike contours in the ‘glyf’ table (subclause 5.3.4), CFF2 CharString contours are always self-closed with a straight line segment, even if it is of zero length. Thus, if the last explicit operand in a contour is a curve, a line segment of zero length is automatically inserted between it and the contour’s start-point. In static fonts this implicit line segment is invisible and causes no difficulty. However, in variable fonts, care must be taken when the start-point of a contour is blended, since the contour’s final explicit point does not necessarily move in the same way.

For example, consider a glyph contour representing an approximation of a circle. This can be constructed from 4 cubic Bézier curves, thus 12 control points. In CFF2, we may represent this contour as one rmoveto operation (with 2 operands) followed by four rrcurveto operations (each with 6 operands). Notice that 26 operands are used in total rather than 24, the last two denoting the same x and y coordinates as the start-point defined by rmoveto. When variations for this contour are defined using the blend operator, particular care must be taken to match the movement between the start-point and the last point of the last rrcurveto; otherwise, a line segment will appear to be inserted into the contour, linking the last point to the start-point.

Implementation limits

The total number of contours in a glyph is unlimited. The number of line and curve operations in a contour is also unlimited.

Flex

The four flex operators define Bézier curves that could be represented with two rrcurveto operators, except that they also define interaction with the rasterizer such that the curve should — when it is very small in device coordinates — be treated as a straight line.

Resolution

Coordinates for path operations are typically integers. Fixed 16.16 format coordinates are permitted, but require significantly more CharString data.

##### Metrics

CharStrings do not contain any representation of horizontal or vertical glyph metrics. Metrics are stored in the hmtx (subclause 5.2.5) and vmtx (subclause 5.7.10) tables and undergo font variations using the HVAR (subclause 7.3.5) and VVAR (subclause 7.3.8) tables.

##### Hints

In a CharString, hints specify horizontal and vertical regions that should be treated with special techniques by the rasterizer, resulting in a more regular appearance. Hints must be specified in the following order:

- horizontal stems and edges using the hstem or hstemhm operators,

- vertical stems and edges using the vstem or vstemhm operators,

- counters (gaps between stems), specified in terms of stems with the cntrmask operator.

Each of these types of hint is optional. However, if a counter hint is provided, it requires two stems in its orientation to be defined.

Hint encoding

The hint stem operators (hstem, vstem, hstemhm and vstemhm) define horizontal or vertical hints for stems and edges. Each stem is defined as a pair of numbers: its bottom or left edge followed by its top or right edge. Stems for a particular orientation are encoded by:

- sorting the pairs by bottom coordinate (for horizontal stems) or left coordinate (for vertical stems);

- concatenating the pairs into a sequence of numbers;

- using relative values: the first number is relative to 0, and all other numbers are relative to the previous number.

Edge hints

Edge hints are defined as if they were stem hints with special negative thickness:

- a thickness of -21 defines a left edge or a bottom edge;

- a thickness of -20 defines a top edge or a right edge.

Because the thickness of an edge hint represents “left”, “bottom”, “top” or “right” rather than an actual measurement, edge hints are never said to overlap with other hints. For the purposes of sorting, the first coordinate implied by an edge hint is considered its bottom or left coordinate. An example of a glyph with edge hints is shown in Figure 5.1 below.

Figure 5.1 – Encoding of edge hints

Overlapping hints

In some glyphs two or more stems overlap, potentially causing problems for a rasterizer deciding how to control outline points on the pixel grid. In such cases, the hstemhm and vstemhm stem definition operators must be used for all hints in the CharString instead of hstem and vstem. The hintmask must then be placed at various points within path construction data to control which hints are active for subsequent path construction operators. An example of overlapping hints, illustrated in Figure 5.2 below, is encoded thus:

280 100 -70 40 hstemhm

Figure 5.2 – Encoding of overlapping hints

Implied vstem and vstemhm operators

The vstem and vstemhm operators have the unusual property that in special cases they may be omitted from a CharString to save one byte of data. These cases (which are in fact relatively common in fonts) require both horizontal and vertical stems to be defined, and for those definitions to be followed immediately by a hintmask or cntrmask operator.

Example of edge hints

Consider a sans-serif glyph E with the cap-height of 700 and with three horizontal stems whose y coordinates are: 0 and 80, 310 and 390, 620 and 700. These stems would be defined as:

0 80 230 80 230 80 hstem

In the same font, we also want to control the parts of other glyphs that align vertically with the stems of the E, but do not have a suitable horizontal stem — the glyph “L” is often such a case. We can represent its bottom stem like the “E”, but its top edge requires an edge hint, thus it needs two stems whose y coordinates are: 0 and 80, 700 and 680 (=700-20). Its encoding would be:

0 80 620 -20 hstem

Likewise, an “F” glyph in the same font would benefit from its bottom edge being controlled with an edge hint, thus the encoding:

21 -21 700 -20 hstem

Example of overlapping hints and hint masking

Figure 5.3 shows a part of a glyph that has horizontal and vertical stems, and where the horizontal stems overlap. The horizontal stems are from 280 to 380 (hstem1) and from 310 to 350 (hstem2). Because they overlap, use of the hintmask operator will be required to select one or the other as active. The glyph also has a vertical stem, from 400 to 450 (vstem1).

Figure 5.3 – Hint encoding example

Suppose that only these three hints are defined, and that the outline is being constructed in a counterclockwise direction, beginning with the horizontal line segment ending at (400, 310). The three bits required to represent the three stems fit into a single byte, so hintmask will be followed by a bitmask of a single byte. In this case, the first hint group to be active is hstem2 and vstem1: these are the second and third hints to be defined, so are represented respectively as 0x40 and 0x20, yielding a byte of 0x60 or 0b01100000). The hints and hint mask are thus specified as follows:

280 100 -70 40 hstemhm 400 50 vstemhm hintmask 0x60

Note that hstemhm is used to indicate a forthcoming hintmask. The hints are defined in the order hstem1, hstem2, vstem1. The vstemhm operator is implicitly invoked by hintmask, so the following encoding is therefore equivalent:

280 100 -70 40 hstemhm 400 50 hintmask 0x60

##### Subroutine operators

callsubr: 0x0a (10)

Stack: [... <integer>]
Call the subroutine in LocalSubrINDEX with the index determined by the operand. Note that the operand must be added to the subroutine bias number before being used as the index. See Subroutine bias for calculation of this number.

The LocalSubrINDEX key in the PrivateDICT of this CharString provides the location of the LocalSubrINDEX.

If callsubr is used within a global subroutine, care must be taken to invoke the local subroutine in the LocalSubrINDEX of the CharString being processed.

callgsubr: 0x1d (29)

Stack: [... <integer>]
Call the subroutine in GlobalSubrINDEX with the index determined by the operand. Note that the operand must be added to the subroutine bias number before being used as the index. See Subroutine bias for calculation of this number.

NOTE 	The callsubr and callgsubr operators pop from the stack the operand needed to determine the subroutine index, and leave the stack otherwise unchanged. The subroutine itself may pop and push operands freely.

##### Path operators that move the current point and start a new contour

rmoveto: 0x15 (21)

Stack: [ dx dy ]
Moves the current point to a position at the relative coordinates (dx, dy) and starts a new contour.

hmoveto: 0x16 (22)

Stack: [ dx ]
Moves the current point dx units in the horizontal direction and starts a new contour.

vmoveto: 0x04 (4)

Stack: [ dy ]
Moves the current point dy units in the vertical direction and starts a new contour.

##### Path operators that draw line segments

rlineto: 0x05 (5)

Stack: [ (dx dy)+ ]
Appends a line segment from the current point to a position at the relative coordinates (dx, dy). Additional rlineto operations are performed for all subsequent argument pairs. The number of lines is determined from the number of arguments on the stack.

hlineto: 0x06 (6)

Stack: [ (d)+ ]
Appends a horizontal line segment from the current point to a position at the relative coordinates (d, 0). When multiple arguments are used, the orientation alternates for each argument, thus the second argument is a vertical line to the relative coordinates (0, d), the third argument is a horizontal line to the relative coordinates (d, 0), and so on. The number of line segments is determined from the number of arguments on the stack. A contour that consists of only horizontal and vertical lines can be constructed using one of the moveto operators and a single hlineto operator, with the number of line segments limited only by the size of the number stack.

vlineto: 0x07 (7)

Stack: [ (d)+ ]
Appends a vertical line segment from the current point to a position at the relative coordinates (0, d). As with hlineto, when multiple arguments are used, the orientation alternates for each argument.

##### Path operators that draw curves

rrcurveto: 0x08 (8)

Stack: [ (dxa dya dxb dyb dxc dyc)+ ]
Appends a cubic Bézier curve, defined by the points p0, p1, p2, p3 where p0 is located at the current point, p1 is given by the relative coordinates (dxa, dya), p2 is given by the relative coordinates (dxb, dyb) and p3 is given by the relative coordinates (dxc, dyc). The location of each point is defined relative to the preceding one. For each subsequent set of six arguments, an additional curve is appended to the current point. The number of curve segments is determined from the number of arguments on the number stack and is limited only by the size of the number stack.

hhcurveto: 0x1b (27)

Stack: [ dy1? (dxa dxb dyb dxc)+ ]
Appends one or more Bézier curves, as described by the dxa…dxc set(s) of arguments, to the current point. If the argument count is a multiple of four, the curve starts and ends horizontal. If the argument count is one more than a multiple of four, the first curve does not begin with a horizontal tangent and its dy1 value is given by the first argument. This operator is the horizontal counterpart to vvcurveto.

vvcurveto: 0x1a (26)

Stack: [ dx1? (dya dxb dyb dyc)+ ]
Appends one or more Bézier curves, as described by the dya…dyc set(s) of arguments, to the current point. If the argument count is a multiple of four, the curve starts and ends vertical. If the argument count is one more than a multiple of four, the first curve does not begin with a vertical tangent and its dx1 value is given by the first argument. This operator is the vertical counterpart to hhcurveto.

hvcurveto: 0x1f (31)

Stack: [ dx1 dx2 dy2 dy3 (dya dxb dyb dxc dxd dxe dye dyf)* dxf? ]
Stack: [ (dxa dxb dyb dyc dyd dxe dye dxf)+ dyf? ]
Appends one or more Bézier curves to the current point with tangents that alternate between horizontal and vertical. The first curve’s first tangent is horizontal and its second tangent is vertical. If a second curve is defined, its first tangent is vertical and its second tangent is horizontal. A third curve, if defined, is oriented like the first. The tangent of the final curve can be overridden freely by using the optional extra argument to define a non-zero coordinate. The equivalent operator that starts with a vertical tangent is vhcurveto.

vhcurveto: 0x1e (30)

Stack: [ dy1 dx2 dy2 dx3 (dxa dxb dyb dyc dyd dxe dye dxf)* dyf? ]
Stack: [ (dya dxb dyb dxc dxd dxe dye dyf)+ dxf? ]
Appends one or more Bézier curves to the current point with tangents that alternate between vertical and horizontal. The first curve’s first tangent is vertical and its second tangent is horizontal. If a second curve is defined, its first tangent is horizontal and its second tangent is vertical. A third curve, if defined, is oriented like the first. The tangent of the final curve can be overridden freely by using the optional extra argument to define a non-zero coordinate. The equivalent operator that starts with a horizontal tangent is hvcurveto.

rcurveline: 0x18 (24)

Stack: [ (dxa dya dxb dyb dxc dyc)+ dxd dyd ]
Appends a sequence of Bézier curves followed by a line. This is equivalent to one rrcurveto for each set of six arguments dxa…dyc, followed by exactly one rlineto using the dxd, dyd arguments. The number of curves is determined from the count on the argument stack.

rlinecurve: 0x19 (25)

Stack: [ (dxa dya)+ dxb dyb dxc dyc dxd dyd ]
Appends a sequence of lines followed by a Bézier curve. This is equivalent to one rlineto for each pair of arguments preceding the six arguments dxb…dyd needed for the one rrcurveto operation. The number of lines is determined from the count of items on the argument stack.

flex: 0x0c23 (12,35)

Stack: [ dx1 dy1 dx2 dy2 dx3 dy3 dx4 dy4 dx5 dy5 dx6 dy6 fd ]
Causes two Bézier curves, as described by the arguments (as shown in Figure 5.4 below), to be rendered as a straight line when the flex depth is less than fd/100 device pixels, and as curved lines when the flex depth is greater than or equal to fd/100 device pixels.

Figure 5.4 – Flex hint example

The flex depth for a horizontal curve, as shown in Figure 5.4, is the distance from the join point to the line connecting the start and end points on the curve. If the curve is not exactly horizontal or vertical, determination of flex depth uses a method that depends on whether the curve is more horizontal or vertical by the method described in the flex1 description, below, and as illustrated in Figure 5.5.

Figure 5.5 – Flex depth calculations

NOTE 	In cases where some of the points have the same x or y coordinate as other points in the curves, arguments may be omitted by using one of the following forms of the flex operator: hflex, hflex1 or flex1.

hflex: 0x0c22 (12,34)

Stack: [ dx1 dx2 dy2 dx3 dx4 dx5 dx6 ]
Causes the two curves described by the arguments dx1…dx6 to be rendered as a straight line when the flex depth is less than 0.5 device pixels (that is, fd < 50), and as curved lines when the flex depth is greater than or equal to 0.5 device pixels.

hflex is used when the following are all true:

- The starting point, the ending point, the first control point and the last control point all have the same y value (that is, y = dy1 = dy5 = dy6).

- The joining point and the neighboring control points have the same y value (that is, dy2 = dy3 = dy4).

- The flex depth is 50.

hflex1: 0x0c24 (12,36)

Stack: [ dx1 dy1 dx2 dy2 dx3 dx4 dx5 dy5 dx6 ]
Causes the two curves described by the arguments to be rendered as a straight line when the flex depth is less than 0.5 device pixels, and as curved lines when the flex depth is greater than or equal to 0.5 device pixels.

hflex1 is used if the conditions for hflex are not met but all of the following are true:

- The starting and ending points have the same y value.

- The joining point and the neighboring control points have the same y value.

- The flex depth is 50.

flex1: 0x0c25 (12,37)

Stack: [ dx1 dy1 dx2 dy2 dx3 dy3 dx4 dy4 dx5 dy5 d6 ]
Causes the two curves described by the arguments to be rendered as a straight line when the flex depth is less than 0.5 device pixels, and as curved lines when the flex depth is greater than or equal to 0.5 device pixels.

The d6 argument will be either a dx or dy value, depending on the curve (see Figure 3). To determine the correct value, compute the distance from the starting point (x, y), the first point of the first curve, to the last flex control point (dx5, dy5) by summing all the arguments except d6; call this (dx, dy). If abs(dx) > abs(dy), then the last point’s x-value is given by d6, and its y-value is equal to y. Otherwise, the last point’s x-value is equal to x and its y-value is given by d6.

flex1 is used if the conditions for hflex and hflex1 are not met but all of the following are true:

- The starting and ending points have the same x or y value.

- The flex depth is 50.

##### Variation operators

vsindex: 0x0f (15)

Stack: [ <integer> ]
Activates a particular list of variation regions from VariationStore.

When used in a CharString, vsindex sets the active list of variation regions (also known as an ItemVariationData), overriding both the default list (index 0) and any list of regions that may be defined (using the PrivateDICT version of the vsindex operator) in the PrivateDICT of this this CharString.

NOTE 	The vsindex operator is encoded as 0x0f in CharString data, unlike in PrivateDICT data where it is encoded as 0x16.

blend: 0x10 (16)

Stack: [... <n default numbers> <n * k delta numbers> <n> ]
Pops n + (n*k) + 1 operands from the stack, processes them according to the font variations interpolation algorithm, then pushes n processed numbers back onto the stack.

For the operation of blend, see CFF2 Variation Store operators (subclause 5.4.3.13.4).

NOTE 	The blend operator is encoded as 0x10 in CharString data, unlike in PrivateDICT data where it is encoded as 0x17.

##### Hint operators

hstem: 0x01 (1)

Stack: [ (<number> <number>)+ ]

Defines a sequence of non-overlapping horizontal stems for the CharString using pairs of numbers.

vstem: 0x03 (3)

Stack: [ (<number> <number>)+ ]

Defines a sequence of non-overlapping vertical stems for the CharString using pairs of numbers.

hstemhm: 0x12 (18)

Stack: [ (<number> <number>)+ ]

Defines a sequence of potentially overlapping horizontal stems for the CharString using pairs of numbers.

vstemhm: 0x17 (23)

Stack: [ (<number> <number>)+ ]

Defines a sequence of potentially overlapping vertical stems for the CharString using pairs of numbers.

hintmask: 0x13 (19)

Stack: [ ]

Activates and deactivates stem hints within this CharString, in order that overlapping hints do not cause ambiguities for the rasterizer.

If any hints overlap, hintmask must be used to establish a non-overlapping subset of hints for a portion of CharString data. Path operators occurring after a hintmask are influenced by the new set of active hints. The hintmask operator can be used any number times within one CharString.

hintmask does not pop any operands from the stack. Instead it consumes bytes that come after it in the CharString data, interpreting them as a bit field that flags each stem as active or inactive. The position of a bit in the bit field determines the index of the stem it controls, treating all horizontal and vertical stem as part of a single array of stems. The number of bytes used depends on the total number of horizontal and vertical stem, numStems, and is given by the formula: 1 + floor ( ( numStems - 1) /8 ). All unused bits must be 0.

hintmask may not be used unless the CharString also defines stems using hstemhm and/or vstemhm.

Example

A hintmask activating hints 1, 3 and 9 in a CharString that has 17 hints in total requires the bit field: 01010000 01000000 00000000. Thus the three bytes 0x504000 immediately follow the hintmask operator.

cntrmask: 0x14 (20)

Stack: [ ]

Specifies the counter spaces to be controlled, and their relative priority.

Counter definitions depend on stem definitions (already defined using hstem, vstem, hstemhm and vstemhm). Counters are defined by specifying groups of stems that delimit them. More than two stems may be specified in order to delimit multiple counters.

Multiple cntrmask operators may be used where sets of counters are to be treated with different priority. Counters controlled by the first cntrmask have top priority; subsequent cntrmask commands specify lower priority counters.

cntrmask does not pop any operands from the stack. Instead it consumes bytes that come after it that flag stems as active or inactive. See hintmask for the formatting of these bytes.

Any cntrmask operators in a CharString must be placed immediately after the stem hint definitions. A CharString without defined stems may not use cntrmask.

Example

Figure 5.6 shows a glyph with a complete list of stems thus:

H1 H2 H3 H4 H5 H6 H7 H8 V1 V2 V3 V4 V5

We may wish to assert that the counters in the right half of the glyph should maintain their vertical proportions as far as possible, and that the two principal horizontal counter widths should also maintain their relative proportion. These counters are represented with the following set of stems:

H1 H3 H4 H6 H8 V1 V2 V3 V5

We may also assert that the counters in the left half of the glyph should maintain their vertical proportions as far as possible, but not at the cost of affecting previously defined counters. That secondary representation of counters, via stems, is:

H2 H5 H7

The following cntrmask commands each use 13 bits (thus 2 bytes) to encode these requirements:

cntrmask 0xB5 0xE8

cntrmask 0x4A 0x00

Notice that the V4 stem does not delimit a useful counter space, and hence is not referenced in this example.

Figure 5.6 – Counter control example

#### Hinting in CFF2 fonts

This information is split between hinting in PrivateDICTs and hinting in CharStrings

Please refer to the description of PrivateDICT (subclause 5.4.3.10) and CharString (subclause 5.4.3.11) for details.

#### Font Variations in CFF2

##### Introduction

The CFF2 table supports font variations as described in the chapter Font Variations Overview (subclause 7.1), and this section assumes familiarity with that chapter. In CFF2 tables, variation axis settings may adjust:

- positions of outline points defined in CharStrings,

- hinting values defined in CharStrings,

- hinting values defined in PrivateDICTs.

In general, to support variation of glyphs or other font data, font variations require a set of default values for the particular data item, a set of delta adjustment values used to modify the default values, and a set of regions within the font’s variation space over which the different delta values apply. In CFF2, the calculation of new values for particular instances — based on default values, delta values, regions and current axis settings — is called blending.

Blending is defined only for CharString and PrivateDICT data. In each case, two operators control variation:

- vsindex selects the active list of variation regions,

- blend updates numbers on the stack using delta values also provided on the stack.

The function of these operators is explained below.

A CFF2 table supporting font variations requires a VariationStore structure. This defines a global list of all the variation regions used in the font, and also multiple subsets of that list.

##### VariationStore

The VariationStore starts with a uint16 field that specifies a length, followed by an Item Variation Store structure of the specified length.

| Offset | Type | Field |
| --- | --- | --- |
| 0 | uint16 | length (of Item Variation Store) |
| 2 | <length> bytes | Item Variation Store |

For the specification of Item Variation Store, see Font Variations Common Table Formats (subclause 7.2). A brief summary is provided below.

Summary of Item Variation Store

An Item Variation Store consists of two arrays: an array of VariationRegions and an array of ItemVariationData structures.

- A VariationRegion is defined in terms of three locations on each of the variation axes in the font. The three locations are the start, the peak and the end of the active region in each axis, all recorded as normalized axis values (i.e. between -1 and 1).

- The VariationRegionList is the complete list of VariationRegions used in the CFF2 table. All the VariationRegions used in the CFF2 font must be defined, even if they are used only in a single CharString or a single PrivateDICT. A CFF2 table that supports variations has exactly one VariationRegionList, which contains at least one VariationRegion.

- An ItemVariationData structure defines a regionIndexes array. Each item in regionIndexes identifies a VariationRegion via its 0-based index in VariationRegionList. (In the discussion of the blend operator, the length of the active regionIndexes array is referred to as k.)

- The Item Variation Store contains an array of ItemVariationData structures. Often the Item Variation Store contains only one ItemVariationData structure, hence only one set of regions is used by all glyphs.

Other considerations for the CFF2 Variation Store

Although the ItemVariationData structure in general allows the specification of delta values within itself, CFF2 does not use this facility. Instead, in CFF2 delta values are stored as operands to the blend operator. Therefore, the itemCount and shortDeltaCount fields in each ItemVariationData must be set to 0. Since itemCount is 0, there are no deltaSets to store, so the deltaSets field is not stored at all.

Like all Item Variation Stores, that used in CFF2 VariationStore is dependent on the fvar (subclause 7.3.3) and avar (subclause 7.3.1) tables that define the font’s variation space.

The location of the VariationStore in the CFF2 table is given by the VariationStoreOffset key of TopDICT. If there is no such key in a CFF2 table, the variations are not defined, the VariationStore data block must be omitted, and the blend operator must not be used.

NOTE 	The CFF2 VariationStore structure does not share data with Item Variation Store structures in other parts of the font. For example, if the VariationRegionList used in the CFF2 table is identical with that in the MVAR table, it must be defined explicitly in both places.

##### CFF2 Variation Store operators

The vsindex operator

vsindex: 0x16 (22) in PrivateDICT data, 0x0f (15) in CharString data

Stack: [ <integer> ]
Activates a particular list of VariationRegions from VariationStore.

If multiple ItemVariationData structures are defined, then PrivateDICTs and CharStrings need to be able to select an active list of VariationRegions. The operator vsindex is provided to select the active ItemVariationData via its index in Item Variation Store, and hence sets the active list of regions. The default vsindex is 0, thus fonts with only a single list of regions have no need for the vsindex operator.

When a PrivateDICT specifies an active list of VariationRegions using the vsindex operator, it applies not only to any blending that occurs within the PrivateDICT but also to blending in all of the CharStrings that use that PrivateDICT. However, using the vsindex operator in a CharString overrides the set of regions set using PrivateDICT. Within a given PrivateDICT or CharString only one list of regions is active throughout, thus multiple occurrences of vsindex in a PrivateDICT or CharString are forbidden. If used in a CharString, then vsindex must occur before any use of blend.

The blend operator

blend: 0x17 (23) in PrivateDICT data, 0x10 (16) in CharString data

Stack: [... <n default numbers> <n * k delta values> <n> ]
Pops n + (n*k) + 1 operands from the stack, processes them according to the font variations interpolation algorithm, then pushes n “blended” numbers back onto the stack (where n is the integer at the top of the stack, and k is the number of VariationRegions in the active ItemVariationData).

The blend operator is CFF2’s mechanism for interpolating default values with delta values. Interpolation happens in the contexts of:

- an active ItemVariationData structure that provides a list of k VariationRegions (the active ItemVariationData structure is controlled using the vsindex operator);

- a set of normalized axis settings based on axis settings selected by a user or system.

Based on the axis settings and the configuration of the VariationRegions, the Font Variations system supplies a scalar (a real number in the range [0,1]) for each of the k VariationRegions, yielding an array of k scalars.

The n+(n*k)+1 arguments to the blend operator are organized in three groups:

- n default values;

- n * k delta values, organized into n groups each of k delta values, one group for each of the n default values;

- n itself.

To calculate the blended values, the n default values are treated in turn. The k delta values for a particular default value are respectively multiplied by the k scalars. These k products are then all added to the default value, resulting in its final blended value. Once all n default values have been blended, they are pushed onto the stack for use by other operators.

Examples

Consider a value of 120 used in a hlineto operator that has a delta of 52 FUnits in the context of an ItemVariationData that has just one variation region active (k = 1). After the single default value (120) we also require n * k delta values, and finally n itself. Since n =1 and k =1, the number of delta values is 1 * 1 = 1. So the complete sequence of numbers and operators is as follows:

120 52 1 blend hlineto

For an instance where axis settings mean that the scalar for the one active region is 0.75, we can calculate the blended value by multiplying the delta value by that scalar, and then adding it to the default value, thus in this case 120 + 52 * 0.75 = 159. So in this instance, we effectively have a hlineto line segment as if a static font had the following sequence of numbers and operators:

159 hlineto

Advanced uses of blend

Although the blend operator always works in conjunction with a subsequent operator expecting precalculated values (such as rlineto or BlueValues), and is ultimately required to leave operands on the stack ready for that static operator, it is not required to process all of those operands in a single blend operation. Thus the number n in a blend need not equal the number of operands expected by the subsequent static operator, nStatic.

When n < nStatic, only a subset of the operands for the static operator are blended. For example, the two operands to rlineto would normally be blended in a single blend with n=2, but using two separate blend operations, each with n=1 and each placed immediately after the relevant operand, is also valid. Also, if only one of the operands has variation deltas, then a blend with n=1 immediately after that operand can save some data.

Furthermore, by means of multiple blend operations before a static operator and also by pushing more values onto the stack than might be expected, values already blended once can be blended multiple times. Only in the very last blend operation directly before the static operator are the blended offsets added to the default operands. This technique can be used to provide higher order interpolation using a single variation axis, for example to control the variation of a point along a quadratic, cubic or quartic curve rather than along a straight line.

Why permit multiple lists of variation regions?

It may reasonably be asked why the Item Variation Store structure provides access to the global list of variation regions only via multiple ItemVariationData structures that each contains a list of indices into the global list. Why not keep all variation regions permanently active?

Indeed, fonts that have consistent variation behavior in all glyphs should use a single ItemVariationData structure, whose regionIndexes array simply relists all the VariationRegion items in the VariationRegionList.

However, fonts that have heterogeneous variation behavior across their glyphs benefit from multiple ItemVariationData structures. Consider a 1-axis font where a special glyph uses 20 intermediate regions, while 100 other glyphs use only 1 region based on default and maximum axis settings. Thus 21 VariationRegions must be specified in VariationRegionList. Yet if all 21 regions exist in a single ItemVariationData structure, then the blend operations in the 100 glyphs, in addition to the delta that controls their intended variation, must also specify 20 deltas with value 0 for all the intermediate regions to ensure they are inactive. With two ItemVariationData structures (the first containing 1 region, the second containing 20 regions) the special glyph can use the vsindex operator to select the second list of regions, and all blend operators can avoid using deltas of 0. In a large font, the data saved by this technique can be considerable.

##### Comparison of CFF2 variations with ‘gvar’ variations

Metrics

When processing fonts that use a gvar table for font variations, the rasterizer appends four “phantom points” to the array of glyph points, based on metrics data from the hmtx (subclause 5.2.6) and vmtx (subclause 5.7.10) tables. These phantom points represent glyph metrics, and they can then be varied using gvar deltas as if they were points on the glyph outline. The CFF2 rasterizer also uses the hmtx and vmtx tables for metrics, but it does not create phantom points for manipulation in CharStrings. Instead CFF2 depends on the HVAR (subclause 7.3.5) and VVAR (subclause 7.3.8) tables for variation of glyph metrics.

Variation regions

In gvar, a global array of variation regions is defined in the sharedTuples array. A record for each glyph defines a custom array of regions, where each region is either selected from the global array or defined only for that glyph. CFF2, by requiring all regions to be defined globally, and also by predefining the order of regions when referenced in a blend operation, requires less data overall for specifying regions.

Inferred deltas

The gvar variations mechanism includes functionality for “inferred deltas”. By moving only certain points on a glyph outline, deltas can be inferred for other points, saving data overall. See “Inferred deltas for un-referenced point numbers” (subclause 7.3.4.4). 
Inferred deltas are not supported in CFF2 fonts.

Moving contours

Although CFF2 lacks functionality for inferred deltas, an entire contour may still be efficiently translated by variable x and y offsets by applying variation deltas to a single point on the contour to be moved. In gvar, any point on the contour may be chosen, the other points moving identically via inferred deltas. In CFF2, since all points of a contour are defined relative to the contour’s initial rmoveto operation, it is only the operands of that rmoveto that should be blended. The locations of subsequent points on the contour, having deltas of zero relative to the initial point, can avoid blend operations entirely. Care must be taken to disable unintended variation on subsequent contours in the CharString, by means of opposing deltas on the operands of the subsequent rmoveto.

#### Comparing CFF2 with CFF and glyf

The three formats intended for storing monochrome glyph outlines in OFF fonts are the ‘glyf’ table (subclause 5.3.4), the CFF table (subclause 5.4.2), and the CFF2 table.

CFF2 and CFF use cubic (3rd order) Bézier curves to represent glyph outlines, whereas the ‘glyf’ table uses quadratic (2nd order) Bézier curves. CFF2 and CFF also use a different conceptual model for “hints” than the ‘glyf’ table. The three tables also differ in relation to support of variations and in how variation data is stored.

The following table provides a summary comparison of the CFF2, ‘CFF’ and ‘glyf’ tables. Note that some of these differences might not be exposed in high-level font editing software or in runtime programming interfaces.

| Consideration | glyf | CFF | CFF2 |
| --- | --- | --- | --- |
| curves | quadratic (2nd order) | cubic (3rd order) | cubic (3rd order) |
| coordinate precision | 1 FUnit | 1/65536 FUnit | 1/65536 FUnit |
| hinting | TrueType instructions move outline points by controlled amounts | alignment zones apply to all glyphs, stem locations are declared in each glyph | alignment zones apply to all glyphs, stem locations are declared in each glyph |
| decoding | not stack-based (except TrueType instructions) | mostly stack-based | mostly stack-based |
| Font variations | yes: outline variation data is stored in ‘gvar’ (subclause 7.3.4); hint variation data is stored in ‘cvar’ (subclause 7.3.2) | no | yes: variation data for outlines and hints is stored within the CFF2 table |
| data redundancy | low | moderate | low |
| overlapping contours | yes | no | yes |

#### CFF2 dependencies on other OFF data

An important factor in the design of the CFF2 specification was the avoidance of recording data already specified elsewhere in an OFF font file. Consequently, CFF2 is highly dependent on data in other OFF tables. OFF tables that do not refer to specific glyph formats are intended to function as specified, including ‘cmap‘ (subclause 5.2.2), ‘hhea‘ (subclause 5.2.4), ‘name‘ (subclause 5.2.7), ‘post‘ (subclause 5.2.9), ‘OS/2‘ (subclause 5.2.8), ‘GPOS‘ (subclause 6.3.3), ‘GSUB‘ (subclause 6.3.4), the color tables ‘COLR‘ (subclause 5.7.11) and ‘CPAL‘ (subclause 5.7.12), and the variation tables ‘fvar‘ (subclause 7.3.3), ‘avar‘ (subclause 7.3.1), ‘STAT‘ (subclause 7.3.7), ‘MVAR‘ (subclause 7.3.6), ‘HVAR‘ (subclause 7.3.5) and ‘VVAR‘ (subclause 7.3.8).

Here follow some requirements and considerations for specific tables.

##### SFNT Header

sfntVersion

The sfntVersion field, the first four bytes of an OFF font file, must be 0x4F54544F (“OTTO”) for OFF fonts with glyphs defined in a CFF2 table.

##### ‘maxp’ table

version

CFF2 fonts must use version 0.5 (encoded as 0x00005000)

numGlyphs

This must be identical to the number of CharStrings in the CFF2 table (the number of items in CharStringsINDEX).

This version of the maxp table is only 6 bytes in total and avoids specifying values not used by CFF2.

##### ‘head’ table

The specification should be followed for all fields, with the following notes.

unitsPerEm

This is respected, and must correspond with the value of the TopDICT FontMatrix key in the CFF2 table. FontMatrix uses the reciprocal of unitsPerEm in a specific sequence of operands. For example, if unitsPerEm is 2000, having reciprocal 1/2000 = 0.0005, then FontMatrix must be set to 0.0005 0 0 0.0005 0 0. If unitsPerEm is equal to 1000, then the FontMatrix key may be omitted from the TopDICT.

indexToLocFormat

Set to 0.

glyphDataFormat

Set to 0.

##### ‘hmtx’ and ‘HVAR’ tables

Vertical metrics for each glyph are stored in the ‘hmtx’ table (subclause 5.2.5). Variation of horizontal metrics is controlled by the ‘HVAR’ table (subclause 7.3.5).

##### ‘vmtx’ and ‘VVAR’ tables

Vertical metrics for each glyph are stored in the ‘vmtx’ table (subclause 5.7.10). Variation of vertical metrics is controlled by the ‘VVAR’ table (subclause 7.3.8).

##### ‘fvar’ table

An ‘fvar’ (subclause 7.3.3) table is required if a CFF2 table uses font variations. In other words, if a VariationStore structure (subclause 7.2.3) is defined within the CFF2 table, an ‘fvar’ table must also be defined. This table specifies the number of axes, axisCount, as well as their order. For each axis it specifies the fields axisTag, nameID, defaultValue, minValue, and maxValue, which are all necessary for any implementation of font variations.

The CFF2 VariationStore data structure (see subclause 5.4.3.13.2) and the blend operators refer to variation axes according to their ordering in the ‘fvar’ table.

##### ‘gvar’ table

The ‘gvar’ table (subclause 7.3.4) is not used in OFF fonts with CFF2-encoded glyphs. Variation in glyph outlines and glyph hinting are controlled directly in CharString data. Variation of glyph metrics is controlled in the ‘HVAR’ (subclause 7.3.5) and ‘VVAR’ tables (subclause 7.3.8).

### VORG—Vertical origin table

This optional table specifies the y coordinate of the vertical origin of every glyph in the font.

This table may be optionally present only in CFF OFF fonts. If present in TrueType OFF fonts it must be ignored by font clients, just as any other unrecognized table would be. This is because this table is not needed for TrueType OFF fonts: the Vertical Metrics ('vmtx') and Glyph Data ('glyf') tables in TrueType OFF fonts provide all the information necessary to accurately calculate the y-coordinate of a glyph's vertical origin. See the "Vertical Origin and Advance Height" in the 'vmtx' table specification for more details.

The 'vmtx' and Vertical Header ('vhea') tables continue to be required for all OFF fonts that support vertical writing. Advance heights must continue to be obtained from the 'vmtx' table; that is the only place they are stored.

If a 'VORG' table is present in a CFF OFF font, a font client may choose to obtain the y coordinate of a glyph's vertical origin either:

- directly from the 'VORG', or:

- by first calculating the top of the glyph's bounding box from the CFF charstring data and then adding to it the glyph's top side bearing from the 'vmtx' table.

The former method offers the advantage of increased accuracy and efficiency, since bounding box results calculated from the CFF charstring as in the latter method can differ depending on the rounding decisions and data types of the bounding box algorithm. The latter method provides compatibility for font clients who are either unaware of or choose not to support the 'VORG'.

Thus, the 'VORG' doesn't add any new font metric values per se; it simply improves accuracy and efficiency for CFF OFF font clients, since the intermediate step of calculating bounding boxes from the CFF charstring is rendered unnecessary.

See subclause9.7 "OFF CJK Font Guidelines" for more information about constructing CJK (Chinese, Japanese, and Korean) fonts.

Vertical Origin Table Format

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version (starting at 1). Set to 1. |
| uint16 | minorVersion | Minor version (starting at 0). Set to 0. |
| int16 | defaultVertOriginY | The y coordinate of a glyph's vertical origin, in the font's design coordinate system, to be used if no entry is present for the glyph in the vertOriginYMetrics array. |
| uint16 | numVertOriginYMetrics | Number of elements in the vertOriginYMetrics array. |

This is immediately followed by the vertOriginYMetrics array (if numVertOriginYMetrics is non-zero), which has numVertOriginYMetrics elements of the following format:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphIndex | Glyph index. |
| int16 | vertOriginY | Y coordinate, in the font's design coordinate system, of the vertical origin of glyph with index glyphIndex. |

This array must be sorted by increasing glyphIndex, and must not have more than one element with the same glyphIndex. In a size-optimized implementation, glyphs whose vertical origin's y coordinate equals defaultVertOriginY will not have an entry in this array.

If all glyphs in a font share the same defaultVertOriginY value, the length of the 'VORG' table will be 8 bytes in a size-optimized implementation, since the vertOriginYMetrics array will be absent.

Typically only the full-width Latin glyphs in an East Asian font will have entries in the vertOriginYMetrics array. Glyphs rotated for vertical writing, as used in the Vertical Alternates and Rotation ('vrt2') feature, for example, can take advantage of the default value if they are designed appropriately.

In the following example of a complete 'VORG' table for a 1000-unit-em font, every glyph in the font is specified as having a vertOriginY of 880 except for glyphs with glyph indexes 10, 12, and 13:

majorVersion         =1

minorVersion         =0

defaultVertOriginY   =880

numVertOriginYMetrics=3

--- vertOriginYMetrics[index]=(glyphIndex,vertOriginY)

[0]=(10,889)

[1]=(12,861)

[2]=(13,849)

## Table for SVG glyph outlines

### SVG—The SVG (Scalable Vector Graphics) table

This table contains SVG descriptions for some or all of the glyphs in the font.

OFF provides various formats for color fonts, one of which is the SVG table. The SVG table provides the benefits of supporting scalable color graphics using the Scalable Vector Graphics markup language, a vector graphics file format that is widely used on the Web and that provides rich graphics capabilities, such as gradients.

SVG was developed for use in environments that allow for a rich set of functionality, including leveraging the full functionality of Cascading Style Sheets for styling, and programmatic manipulation of graphics objects using the SVG Document Object Model. Adoption of SVG for use in OpenType does not entail wholesale incorporation of all SVG capabilities. Text-rendering engines typically have more stringent security, performance and architectural requirements than general-purpose SVG engines. For this reason, when used within OFF fonts, the expressiveness of the language is limited and simplified to be appropriate for environments in which font processing and text layout occurs.

The SVG table is optional, and may be used in OFF fonts with TrueType, CFF or CFF2 outlines. For every SVG glyph description, there must be a corresponding TrueType, CFF or CFF2 glyph description in the font.

SVG Table Header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version (starting at 0). Set to 0. |
| Offset32 | offsetToSVGDocumentList | Offset to the SVG Documents List, from the start of the SVG table. Must be non-zero. |
| uint32 | reserved | Set to 0. |

SVG Document List

The SVG document list provides a set of SVG documents, each of which defines one or more glyph descriptions.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | numEntries | Number of SVG Document Index Entries. Must be non-zero. |
| SVGDocumentRecord | documentRecords [numEntries] | Array of SVG document records. |

SVGDocumentRecord

Each SVG document record specifies a range of glyph IDs (from startGlyphID to endGlyphID, inclusive), and the location of its associated SVG document in the SVG table.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | startGlyphID | The first glyph ID for the range covered by this record. |
| uint16 | endGlyphID | The last glyph ID for the range covered by this record. |
| Offset32 | svgDocOffset | Offset from the beginning of the SVGDocumentList to an SVG document. Must be non-zero. |
| uint32 | svgDocLength | Length of the SVG document data. Must be non-zero. |

Records must be sorted in order of increasing startGlyphID. For any given record, the startGlyphID must be less than or equal to the endGlyphID of that record, and also must be greater than the endGlyphID of any previous record.

NOTE:	Two or more records can point to the same SVG document. In this way, a single SVG document can provide glyph descriptions for discontinuous glyph ID ranges. See Example 1 in §5.5.6

### SVG Documents

SVG specification

The SVG markup language used in the SVG table shall be as defined in the Scalable Vector Graphics (SVG) 1.1 (2nd edition), W3C Recommendation, 16 August 2011. Any SVG features beyond those in the SVG 1.1 specification are not supported, unless explicitly indicated otherwise.

Previous versions of this specification allowed use of context-fill and other context-* property values, which are defined in the draft SVG 2 specification. Use of these properties is deprecated: conforming implementations may support these properties, but support is not required or recommended, and use of these properties in fonts is strongly discouraged.

Document encoding and format

SVG documents within an OFF SVG table may either be plain text or gzip-encoded, and applications that support the SVG table shall support both.

The gzip format is defined in the RFC 1952, “GZIP file format specification version 4.3”, [31]. Within a gzip-encoded SVG document, the deflate compression method (defined by the RFC 1951) must be used. Thus, the first three bytes of the gzip-encoded document header must be 0x1F, 0x8B, 0x08.

Whether compressed or plain-text transfer encoding is used, the SVGDocLength field of the SVG document record specifies the length of the encoded data, not the decoded document.

The encoding of the (uncompressed) SVG document must be UTF-8.

While SVG 1.1 is defined as an XML application, some SVG implementations for the Web use an “HTML dialect”. The “HTML dialect” differs from the XML-based definition in various ways, including being case-incensitive (XML is case-sensitive), and not requiring an xmlns attribute on the SVG root element. Applications that support the OFF SVG table shall support the XML-based definition for SVG 1.1. Applications may use SVG-parsing libraries that also support the “HTML dialect”. However, SVG documents within the OFF fonts must always conform to the XML-based definition.

While SVG 1.1 requires conforming interpreters to support XML namespace constructs, applications that support the OpenType SVG table are not required to have full support for XML namespaces. The root element of each SVG document must declare SVG as the default namespace:

<svg version=”1.1” xmlns=”http://www.w3.org/2000/svg”>

If the XLink href attribute is used, the root must also declare “xlink” as a namespace in the root element:

<svg version=”1.1” 	xmlns=”http://www.w3.org/2000/svg”

xmlns:xlink=”http://www.w3.org/1999/xlink”>

No other XLink attributes or other mechanisms may be used anywhere in the document. Also, no other namespace declarations should be made in any element.

SVG capability requirements and restrictions

Most SVG 1.1 capabilities are supported in OFF, and should be supported in all OFF applications that support the SVG table. Some SVG 1.1 capabilities are not required and may be optionally supported in applications. Certain other capabilities are not supported in OFF and must not be used in SVG documents within OFF fonts.

The following capabilities are restricted from use in OFF and must not be used in conforming fonts. If use of associated elements is encountered within a font, conforming applications must ignore and not render those elements.

- <text>, <font>, and associated elements

- <foreignObject> elements

- <switch> elements

- <script> elements

- <a> elements

- <view> elements

- XSL processing

- Use of relative units em, ex

- Use of SVG data within <image> elements

- Use of color profiles (the <icccolor> data type, the <color-profile> element, the color-profile property, or the CSS @color-profile rule)

- Use of the contentStyleType attribute

- Use of CSS2 system color keywords

SVG documents may include <desc>, <metadata> or <title> elements, but these are ignored by implementations.

Support for the following capabilities is not required in conforming implementations, though some applications may support them. Font developers should evaluate support for these capabilities in the target environments in which their fonts will be used before using them in fonts. To ensure interoperability across the broadest range of environments, use of these capabilities should be avoided.

- Internal CSS stylesheets (expressed using the <style> element)

- CSS inline styles (expressed using the style attribute)

- CSS variables (custom properties) — but see further qualifications below

- CSS media queries, calc() or animations

- SVG animations

- SVG child elements

- <filter> elements and associated attributes, including enableBackground

- <pattern> elements

- <mask> elements

- <marker> elements

- <symbol> elements

- Use of XML entities

- Use of image data within <image> elements in formats other than JPEG or PNG

- Interactivity capabilities (event attributes, zoomAndPan attributes, the cursor property, or <cursor> elements)

NOTE 1 	In fonts intended for broad distribution, use of XML presentation attributes for styling is recommended over CSS styling as that will have the widest support across implementations.

NOTE 2 	Use of media queries to react to environment changes within a glyph description is not recommended, even when fonts are used in applications that provide CSS media query support. Instead, a higher-level presentation framework should handle environment changes. The higher-level framework can interact with options implemented within the font using OpenType mechanisms such as glyph substitution, or selection of color palettes.

While supporting the use of CSS variables is optional, it is strongly recommended that all implementations support the CSS var() function for color variables defined in the CPAL table. Fonts should not define any variables within an SVG document; var() should only be used in attributes or properties that accept a color value, and should only occur as the first item in the value. See §5.5.3 “Color and color palettes” below for more information.

While support for patterns and masks is not required, all conforming implementations must support gradients (<linearGradient> and <radialGradient> elements), clipping paths and opacity properties.

Conforming implementations must support all other capabilities of SVG 1.1 that are not listed above as restricted or as optional and best avoided for broad interoperability.

### Color and color palettes

In SVG 1.1, color values can be specified in various ways. For some of these, special considerations apply when used in the SVG table. Also, OFF provides a mechanism for alternate, user-selectable color palettes that can be used within SVG glyph descriptions.

Colors

Implementations must support numerical RGB specifications; for example, "#ffbb00", or "rgb(255,187,0)". Implementations must also support all of the recognized color keywords supported in SVG 1.1. However, CSS2 system color keywords are not supported and must not be used.

Some implementations may use graphics engines that happen to support RGBA specifications using the rgba() function. This is not supported in OFF, however, and rgba() specifications must not be used in conforming fonts. Note that SVG 1.1 provides opacity properties that can achieve the same effects.

Implementations must also support the “currentColor” keyword. The initial value must be set by the text-layout engine or application environment. This can be set in whatever way is considered most appropriate for the application. In general, it is recommended that this be set to the text foreground color applied to a given run of text.

NOTE 	Within an SVG document, the value of “currentColor” for any element is the current color property value for that element. If a color property is set explicitly on an element, it will reset the “currentColor” value for that element and its children. Doing so will override the value set by the host environment. In SVG documents within the SVG table, there is no scenario in which it would be necessary to set a color property value since any effects can be achieved in other ways. Setting a color property value should be avoided.

Color palettes

Implementations can optionally support color palettes defined in the §5.7.12 CPAL table. The CPAL table allows the font designer to define one or more palettes, each containing a number of colors. All palettes defined in the font have the same number of colors, which are referenced by base-zero index. Within an SVG document in the SVG table, colors in a CPAL palette are referenced as implementation-defined CSS variables (custom properties), using the var() function.

Support for the CPAL table and palettes in implementations is strongly recommended. Implementations that support palettes must support the CSS var() function for purposes of referencing palette entries as custom properties. Fonts should only use custom properties and the var() function to reference CPAL palette entries. Fonts should not define any variables within an SVG document. The var() function should only be used in attributes or properties that accept a color value, and should only occur as the first item in the value.

NOTE 	Even if an implementation does not support CPAL palettes, it is strongly recommended that the var() function be supported, and that the implementation is able to apply a fallback value specified as a second var() argument if the first argument (the color variable) is not supported. This will allow fonts intended for wide distribution to include use of the CPAL table but to be able to specify fallback colors in case CPAL palettes are not supported in some applications.

The text-layout engine or application defines a custom property for each palette entry and assigns color values to each one. Custom color properties should only be defined for fonts that include a CPAL table. In general, the values of the custom properties should be set using palette entries from the CPAL table, though applications can assign values derived by other means, such as user input. When assigning values from CPAL palette entries, the first palette should normally be used by default. If the font has palettes marked with the USABLE_WITH_LIGHT_BACKGROUND or USABLE_WITH_DARK_BACKGROUND flag, however, one of these palettes can be used as the default instead.

However the values are assigned, the number of custom properties defined must be numPaletteEntries, as specified in the CPAL table header. The custom-property names must be of the form “--color<num>”, where <num> is a non-zero-padded decimal number in the range [0, numPaletteEntries-1]. For example, “--color0”, “--color1”, and so on.

The following illustrates how a color variable might be used in an SVG glyph description:

<path fill="var(--color0, yellow)" d="..."/>

In implementations that do support color variables and palettes, the color value assigned to the variable will be applied. If an implementation does not support color variables and palettes, however, the color variable will be ignored, and the fallback color value, yellow, will be applied.

Palette entries in the CPAL table are specified as BGRA values. (CPAL alpha values are in the range 0 to 255, where 0 is fully transparent and 255 is fully opaque.) Note that SVG 1.1 supports RGB color values, but not RGBA/BGRA color values. As noted above, use of rgba() color values within SVG documents in the SVG table is not supported and must not be used in conforming fonts. Alpha values in CPAL entries are supported, however. When a CPAL color entry is applied to a fill or stroke property of a shape element, to the stop-color of a gradient stop element, or to the flood-color property of an feFlood filter element, then the alpha value from that palette entry must be converted to a value in the range [0.0 – 1.0] and multiplied into the corresponding fill-opacity, stroke-opacity, stop-opacity or flood-opacity property of the same element. If an implementation supports feDiffuseLighting or feSpecularLighting filters and a palette entry is applied to the lighting-color property, then the alpha value is ignored. When the alpha value is applied in this way to an opacity property of an element, it is the original opacity property value that is inherited by child elements, not the computed result of applying the alpha value to the opacity property. The alpha value is inherited as a component of the color-related property (fill, stroke, etc.), however.

### Glyph Identifiers

Each SVG document defines one or more glyph description. For each glyph ID in the glyph ID range of a document record within the SVG Document list, the associated SVG document shall contain an element with ID “glyph<glyphID>”, where <glyphID> is the glyph ID expressed as a non–zero-padded decimal value. This element functions as the SVG glyph description for the given glyph ID.

For example, suppose a font with 100 glyphs (glyph IDs 0 – 99) has SVG glyph definitions only for its last 5 glyphs. Suppose also that the last SVG glyph definition has its own SVG document, but that the other four glyphs are defined in a single SVG document (to take advantage of shared graphical elements, for instance). There will be two document records, the first with glyph ID range [95, 98]; and the second, with glyph ID range [99, 99]. The SVG document referenced by the first record will contain elements with id “glyph95”, “glyph96”, “glyph97”, and “glyph98”. The SVG document referenced by the second record will contain an element with id “glyph99”.

Glyph identifiers may appear deep within an SVG element hierarchy, but SVG itself does not define how partial SVG documents are to be rendered. Thus, font engines shall render an element designated in this way as the glyph description for a given glyph ID according to SVG’s <use> tag behavior, as though the given element and its content were specified in a <defs> tag and then referenced as the graphic content of an SVG document. For example, consider the following SVG document, which defines two glyphs:

<svg version="1.1" xmlns="http://www.w3.org/2000/svg">

<defs>...</defs>

<g id="glyph13">...</g>

<g id="glyph14">...</g>

</svg>

NOTE 	The <g> element in SVG is a container for grouping of elements, not a “glyph” element.

When a font engine renders glyph 14, the result shall be the same as rendering the following SVG document:

<svg version="1.1" xmlns="http://www.w3.org/2000/svg" xmlns:xlink="http://www.w3.org/1999/xlink">

<defs>

<defs>...</defs>

<g id="glyph14">...</g>

</defs>

<use xlink:href="#glyph14" />

</svg>

### Glyph semantics and text layout processing

An SVG glyph description in the SVG table is an alternate to the corresponding glyph description with the same glyph ID in the 'glyf', 'CFF ' or CFF2 table. The SVG glyph description must provide a depiction of the same abstract glyph as the corresponding TrueType/CFF glyph description.

When SVG glyph descriptions are used, text layout is done in the same manner, using the 'cmap', 'hmtx', GSUB and other tables. This results in an array of final glyph IDs arranged at particular x,y positions on a surface (along with applicable scale/rotation matrices). After this layout processing is done, available SVG descriptions are used in rendering, instead of the TrueType/CFF descriptions. For each glyph ID in the final glyph array, if an SVG glyph description is available for that glyph ID, then it is rendered using an SVG engine; otherwise, the TrueType or CFF glyph description is rendered. Glyph advance widths or heights are the same for SVG glyphs as for TrueType/CFF glyphs, though there may be small differences in glyph ink bounding boxes. Because advances are the same, switching between SVG and non-SVG rendering should not require re-layout of lines unless the line layout depends on bounding boxes.

Coordinate Systems and Glyph Metrics

The default SVG coordinate system is vertically mirrored in comparison to the TrueType/CFF design grid: the positive y-axis points downward, rather than usual convention for OpenType of the positive y-axis pointing upward. In other respects, the default coordinate system of an SVG document corresponds to the TrueType/CFF design grid: the default units for the SVG document are equivalent to font design units; the SVG origin (0,0) is aligned with the origin in the TrueType/CFF design grid; and y = 0 is the default horizontal baseline used for text layout.

The size of the initial viewport for the SVG document is the em square: height and width both equal to head.unitsPerEm. If a viewBox attribute is specified on the <svg> element with width or height values different from the unitsPerEm value, this will have the effect of a scale transformation on the SVG user coordinate system. Similarly, if height or width attributes are specified on the <svg> element, this will also have the effect of a scale transformation on the coordinate system.

Although the initial viewport size is the em square, the viewport must not be clipped. The <svg> element is assumed to have a clip property value of auto, and an overflow property value of visible. A font should not specify clip or overflow properties on the <svg> element. If clip or overflow properties are specified on the <svg> element with any other values, they must be ignored.

NOTE 	Because SVG uses a y-down coordinate system, then by default, glyphs will often be drawn in the +x –y quadrant of the SVG coordinate system. (See Example 2.) In many other environments, however, graphic elements may need to be in the +x +y quadrant to be visible. Font development tools should provide an appropriate transfer between a design environment and the representation within the font’s SVG table. In Example 3, a viewBox attribute is used to shift the viewport up. In Example 4, a translate transform is used on container elements to shift the graphic elements that comprise the glyph descriptions.

Glyph advance widths are specified in the 'hmtx' table; advance heights are specified in the 'vmtx' table. Note that glyph advances are static and cannot be animated.

As with CFF glyphs, no explicit glyph bounding boxes are recorded. Note that left side bearing values in the 'hmtx' table, top side bearings in the 'vmtx' table, and bit 1 in the flags field of the 'head' table are not used for SVG glyph descriptions. The “ink” bounding box of the rendered SVG glyph should be used if a bounding box is desired; this box may be different for animated versus static renderings of the glyph.

Glyph advances and positions can be adjusted by data in the GPOS or 'kern' tables. Note that data in the GPOS and kern table use the y-up coordinate system, as with TrueType or CFF glyph descriptions. When applied to SVG glyph descriptions, applications must handle the translation between the y-up coordinate system and the y-down coordinates used for the SVG glyph descriptions.

### Animations

Some implementations may support use of animations—either SVG animation or CSS animation. Note that support for animation is optional and is not recommended in fonts intended for wide distribution.

Applications that support animations may, in some cases, require a static rendering for glyphs that include animations. This may be needed, for example, when printing. Note that a static rendering is obtained by ignoring and not running any animations in the SVG document, not by allowing animations to run and capturing the initial frame at time = 0.

Note that glyph advance widths and advance heights are defined in the 'hmtx' and 'vmtx' tables, and cannot be animated. A glyph’s bounding box may change during animation, but should remain within the glyph advance width/height and the font’s default line metrics to avoid collision with other text elements.

### SVG glyph examples

The SVG code in these examples is presented exactly as could be used in the SVG documents of an OFF font with SVG glyph descriptions. The code is not optimized for brevity.

Example 1: SVG table header and document list

This example shows an SVG table header and document list.

In the SVG Document List, multiple SVG Document Records can point to the same SVG document. In this way, a single SVG document can provide glyph descriptions for a discontinuous range of glyph IDs. This example shows multiple records in the Document List pointing to the same SVG document.

Example 1:

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | SVGHeader |  |
| 0000 | Version | Table version = 0 |
| 0000000A | offsetToSVGDocumentlist | Offset to document list |
| 00000000 | Reserved | Offset to AttachList table |
|  | SVGDocumentList |  |
| 0005 | numEntries | Five documentRecord entries, index 0 to 4 |
|  | documentRecords[0] | Document record for glyph ID range [1,1] |
| 0001 | startGlyphID |  |
| 0001 | endGlyphID |  |
| 0000003E | svgDocOffset | Offset to SVG document for glyph1 |
| 0000019F | svgDocLength | Length of SVG document for glyph1 |
|  | documentRecords[1] | Document record for glyph ID range [2,2] |
| 0002 | startGlyphID |  |
| 0002 | endGlyphID |  |
| 000001DD | svgDocOffset | Offset to SVG document for glyph2. The same SVG document is also used for glyph13 and glyph14. |
| 000002FF | svgDocLength | Length of SVG document for glyph2 – length of the entire SVG document, covering glyphs 2, 13 and 14. |
|  | documentRecords[2] | Document record for glyph ID range [3,12] |
| 0003 | startGlyphID |  |
| 000C | endGlyphID |  |
| 000004DC | svgDocOffset | Offset to SVG document for glyphs 3 to 12 |
| 000006F4 | svgDocLength | Length of SVG document for glyphs 3 to 12 |
|  | documentRecords[3] | Document record for glyph ID range [13,14] |
| 000D | startGlyphID |  |
| 000E | endGlyphID |  |
| 000001DD | svgDocOffset | Offset to SVG document for glyphs 2, 13 and 14. Offset is the same as for documentRecords[1]. |
| 000002FF | svgDocLength | Length of SVG document for glyphs 2, 13 and 14. Length is the same as for documentRecords[1]. |
|  | documentRecords[4] | Document record for glyph ID range [15,19] |
| 000F | startGlyphID |  |
| 0013 | endGlyphID |  |
| 00000BD0 | svgDocOffset | Offset to SVG document for glyphs 15 to 19 |
| 00000376 | svgDocLength | Length of SVG document for glyphs 15 to 19 |

Example 4 illustrates an SVG document with glyph descriptions for the discontinuous ranges [2, 2], [13, 14].

Example 2: Glyph specified directly in expected position

In this example, the letter “i” is drawn directly in the +x –y quadrant of the SVG coordinate system, upright, with its baseline on the x axis. Note that the y attribute of the <rect> elements specifies the top edge, with the height of the rectangle below that.

<svg id="glyph7" version="1.1" xmlns="http://www.w3.org/2000/svg">

<defs>

<linearGradient id="grad" x1="0%" y1="0%" x2="0%" y2="100%">

<stop offset="0%" stop-color="darkblue" stop-opacity="1" />

<stop offset="100%" stop-color="#00aab3" stop-opacity="1" />

</linearGradient>

</defs>

<rect x="100" y="-430" width="200" height="430" fill="url(#grad)" />

<rect x="100" y="-635" width="200" height="135" fill="darkblue" />

</svg>

The following image shows the visual result:

Example 3: Glyph shifted up with viewBox

When designing in an SVG illustration application, it may be most natural to draw objects in the +x +y quadrant of the SVG coordinate system. In this example, the glyph description of the letter “i” is specified with upright orientation in the +x +y quadrant as though the baseline were at y=1000 in the SVG coordinate system. A viewBox in the <svg> element is then used to shift the viewport down by 1000 units so that the actual baseline aligns with the design’s baseline.

NOTE 	When using a viewBox attribute on the <svg> element, it is important to specify unitsPerEm for width and height values to avoid a scaling effect. See “Coordinate Systems and Glyph Metrics” above for more information.

<svg id="glyph7" version="1.1" xmlns="http://www.w3.org/2000/svg" viewBox="0 1000 1000 1000">

<defs>

<linearGradient id="grad" x1="0%" y1="0%" x2="0%" y2="100%">

<stop offset="0%" stop-color="darkblue" stop-opacity="1" />

<stop offset="100%" stop-color="#00aab3" stop-opacity="1" />

</linearGradient>

</defs>

<rect x="100" y="570" width="200" height="430" fill="url(#grad)" />

<rect x="100" y="365" width="200" height="135" fill="darkblue" />

</svg>

The visual result is the same as that shown for Example 2.

Example 4: Common elements shared across glyphs in same SVG doc

In this example, the base of the letter 'i' is specified as a component within the <defs> element so that it can be re-used across three glyphs. This shared component is referenced using the identifier “i-base”. In glyph2, the component is used alone to comprise the dotless 'i'. In glyph13, a dot is added on top. In glyph14, an acute accent is adds a dot on top. Glyph ID 14 adds an acute accent on top.

This example also illustrates the use of a translate transform to shift elements drawn in the +x +y quadrant of the SVG coordinate system so that they appear in the +x –y quadrant, above the baseline.

<svg version="1.1" xmlns="http://www.w3.org/2000/svg" xmlns:xlink="http://www.w3.org/1999/xlink">

<defs>

<linearGradient id="grad" x1="0%" y1="0%" x2="0%" y2="100%">

<stop offset="0%" stop-color="darkblue" stop-opacity="1" />

<stop offset="100%" stop-color="#00aab3" stop-opacity="1" />

</linearGradient>

<g id="i-base">

<rect x="100" y="570" width="200" height="430" fill="url(#grad)" />

</g>

</defs>

<g id="glyph2" transform="translate(0,-1000)">

<use xlink:href="#i-base" />

</g>

<g id="glyph13" transform="translate(0,-1000)">

<use xlink:href="#i-base" />

<rect x="100" y="365" width="200" height="135" fill="darkblue" />

</g>

<g id="glyph14" transform="translate(0,-1000)">

<use xlink:href="#i-base" />

<polygon fill="darkblue" points="120,500 280,500 435,342 208,342 "/>

</g>

</svg>

The following image shows the visual results for glyph IDs 2, 13, and 14, from left to right.

Example 5: Specifying current text color in glyphs

This example uses the same glyph description for “i” as in Example 2 with one modification: the “darkblue” color value for the dot of the “i” is replaced with the “currentColor” keyword. The application sets the color value for currentColor, typically with the text foreground color.

<svg id="glyph7" version="1.1" xmlns="http://www.w3.org/2000/svg" viewBox="0 1000 1000 1000">

<defs>

<linearGradient id="grad" x1="0%" y1="0%" x2="0%" y2="100%">

<stop offset="0%" stop-color="darkblue" stop-opacity="1" />

<stop offset="100%" stop-color="#00aab3" stop-opacity="1" />

</linearGradient>

</defs>

<rect x="100" y="570" width="200" height="430" fill="url(#grad)" />

<rect x="100" y="365" width="200" height="135" fill="currentColor" />

</svg>

The following image illustrates visual results with currentColor set to two different color values by the application: black (left), and red (right).

Example 6: Specifying color palette variables in glyphs

This example uses the same glyph description for “i” as in Example 2, but with a modification: the stop colors of the linear gradient are specified using color variables --color0 and --color1.

The font in this example includes a CPAL table. The value for the color variables is set by the application, typically using CPAL entries. The CPAL table assumed in this example has two palettes, each with two entries, with BGRA color values as follows:

- palette 0: {8B0000FF, B3AA00FF}

- palette 1: {800080FF, D670DAFF}

(SVG equivalents for colors in palette 0 would be {darkblue, #00aab3}. SVG equivalents for colors in palette 1 would be {purple, orchid}.)

In each of the var() invocations used to reference the color variables, a second parameter for a fallback color value has been specified. The values match the values used in palette 0. These fallback values will be used if the application does not support the CPAL table.

<svg id="glyph7" version="1.1" xmlns="http://www.w3.org/2000/svg" viewBox="0 1000 1000 1000">

<defs>

<linearGradient id="grad" x1="0%" y1="0%" x2="0%" y2="100%">

<stop offset="0%" stop-color="var(--color0,darkblue)" stop-opacity="1" />

<stop offset="100%" stop-color="var(--color1,#00aab3)" stop-opacity="1" />

</linearGradient>

</defs>

<rect x="100" y="570" width="200" height="430" fill="url(#grad)" />

<rect x="100" y="365" width="200" height="135" fill="darkblue" />

</svg>

The following images show the visual results in three situations, from left to right:

The first case, on left, shows the result when the value of the color variables has been set using palette 0 from the CPAL table. The second case, in the middle, shows the result when values have been set using palette 1.

The third case, on the right, shows a result in which the application has set the values of the color variables using a custom palette with user-specified colors:

- --color0: red

- --color1: orange

Note that, in all three cases, the dot of the “i” is still dark blue, since this is hard coded in the glyph description and not controlled by a color variable.

If the application has not set values for --color0 and --color1 (because it does not support the CPAL table, for example), then the fallback values provided in the var() functions (darkblue and #00aab3, respectively) are used. Note that these are in fact the same colors as in the first (default) CPAL color palette, which means the glyph will render as in the first case shown above.

Example 7: Embedding a PNG in an SVG glyph

In this example, PNG data is embedded within an <image> element.

A typical use case for embedding PNG data is detailed artwork in a lettering font.

<svg id="glyph2" version="1.1" xmlns="http://www.w3.org/2000/svg" xmlns:xlink="http://www.w3.org/1999/xlink" viewBox="0 1000 1000 1000">

<image x="100" y="365" width="200" height="635"

xlink:href="data:image/png;base64,

iVBORw0KGgoAAAANSUhEUgAAAMgAAAJ7CAYAAACmmd5sAAAFZklEQVR42u3XsQ3D

MBAEQUpw9ypahrMPGGwiwcFMCQQW9zzWuu4FbJ2eAAQCAgGBgEBAICAQEAgIBAQC

CAQEAgIBgYBAQCAgEBAICAQEAggEBAICAYGAQEAgIBAQCAgEEAgIBAQCAgGBgEBA

ICAQEAgIBBAICAQEAgIBgYBAQCAgEBAIIBAQCAgEBAICAYGAQEAgIBAQCCAQEAgI

BAQCAgGBgEBAICAQQCAgEBAICAQEAgIBgYBAQCAgEEAgIBAQCAgEBAICAYGAQEAg

IBBPAAIBgYBAQCAgEBAICAQEAgIBBAICAYGAQEAgIBAQCAgEBAICAQQCAgGBgEBA

ICAQEAgIBAQCCAQEAgIBgYBAQCAgEBAICAQEAggEBAICAYGAQEAgIBAQCAgEAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAA4DHHWtftGWDv80sE2Ds9AQgEBAL+IPBuIAoBJxYIBAQCPukgEHBigUBAIOAP

AlgQiAtiQsCCgEDAJx0sCFgQsCAgEHBigQUB5oKYELAgIBDwSQcLAhYELAgIBJxY

YEEACwItEIWAEwucWGBBwIKABQGBgBMLLAhYEMCCQFwQEwJOLHBigQUBCwICAScW

WBCwIGBBAIFAPbHcWGBBwCcdLAgIBJxYYEHAgoAFAYEA88RyY4EFAZ90sCAgEBAI

+IOAQMCJBQIBBALxD+ITAj7p4MQCgYBAwB8EBAJOLBAICATwB4EYiELAiQUCAYGA

TzoIBJxYIBAQCPiDABYE4oKYELAgIBDwSQcLAhYELAgIBJxYYEGAuSAmBCwICAR8

0sGCgAUBCwICAScWWBDAgkALRCHgxAInFlgQsCBgQUAg4MQCCwIWBLAgEBfEhIAT

C5xYYEHAgoBAwIkFFgQsCFgQQCBQTyw3FlgQ8EkHCwICAScWWBCwIGBBQCDAPLHc

WGBBwCcdLAgIBAQC/iAgEHBigUAAgUD8g/iEgE86OLFAICAQ8AcBgYATCwQCAgH8

QSAGohBwYoFAQCDgkw4CAScWCAQEAv4ggAWBuCAmBCwICAR80sGCgAUBCwICAScW

WBBgLogJAQsCAgGfdLAgYEHAgoBAwIkFFgSwINACUQg4scCJBRYELAhYEBAIOLHA

goAFASwIxAUxIeDEAicWWBCwICAQcGKBBQELAhYEEAjUE8uNBRYEfNLBgoBAwIkF

FgQsCFgQEAgwTyw3FlgQ8EkHCwICAYGAPwgIBJxYIBBAIBD/ID4h4JMOTiwQCAgE

/EFAIODEAoGAQAB/EIiBKAScWCAQEAj4pINAwIkFAgGBgD8IYEEgLogJAQsCAgGf

dLAgYEHAgoBAwIkFFgSYC2JCwIKAQMAnHSwIWBCwICAQcGKBBQEsCLRAFAJOLHBi

gQUBCwIWBAQCTiywIGBBAAsCcUFMCDixwIkFFgQsCAgEnFhgQcCCgAUBBAL1xHJj

gQUBn3SwICAQcGKBBQELAhYEBALME8uNBRYEfNLBgoBAQCDgDwICAScWCAQQCMQ/

iE8I+KSDEwsEAgIBfxAQCDixQCAgEMAfBGIgCgEnFggEBAI+6SAQcGKBQEAg4A8C

WBCIC2JCwIKAQMAnHSwIWBCwICAQcGKBBQHmgpgQsCAgEPBJBwsCFgQsCAgEnFhg

QQALAi0QhYATC5xYYEHAgoAFAYGAEwssCFgQwIJAXBATAk4scGKBBQELAgIBJxZY

ELAgYEEAgUA9sdxYYEHAJx0sCAgEnFhgQcCCgAUBgQDzxHJjgQUBn3SwICAQEAj4

g4BAwIkFAgEEAvEP4hMCPungxAKBgEDgH3wBrUwJtCBGuc0AAAAASUVORK5CYII=

"/>

</svg>

The following image shows the visual result:

## Tables related to bitmap glyphs

### List of bitmap glyph tables

OFF fonts may also contain bitmaps of glyphs, in addition to outlines. Hand-tuned bitmaps are especially useful in OFF fonts for representing complex glyphs at very small sizes. If a bitmap for a particular size is provided in a font, it will be used by the system instead of the outline when rendering the glyph.

NOTE	Adobe Type Manager does not currently support hinted bitmaps in OFF fonts.

| Tag | Name |
| --- | --- |
| EBDT | Embedded bitmap data |
| EBLC | Embedded bitmap location data |
| EBSC | Embedded bitmap scaling data |
| CBDT | Color bitmap data |
| CBLC | Color bitmap location data |

### EBDT—Embedded bitmap data table

The 'EBDT' table is used to embed monochrome or grayscale bitmap glyph data. It is used together with the 'EBLC' table, which provides embedded bitmap locators, and the 'EBSC' table, which provides embedded bitmap scaling information.

OFF embedded bitmaps are also called 'sbits' (for “scaler bitmaps”). A set of bitmaps for a face at a given size is called a strike.

#### Table structure

The 'EBLC' table identifies the sizes and glyph ranges of the sbits, and keeps offsets to glyph bitmap data in IndexSubTables. The 'EBDT' table then stores the glyph bitmap data, in a number of different possible formats. Glyph metrics information may be stored in either the 'EBLC' or 'EBDT' table, depending upon the IndexSubTable and glyph bitmap data formats. The 'EBSC' table identifies sizes that will be handled by scaling up or scaling down other sbit sizes.

The 'EBDT' table begins with a header containing simply the table version number.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the EBDT table, = 2. |
| uint16 | minorVersion | Minor version of the EBDT table, = 0. |

The rest of the 'EBDT' table is a collection of bitmap data. The data can be in a number of possible formats, indicated by information in the 'EBLC' table. Some of the formats contain metric information plus image data, and other formats contain only the image data. Long word alignment is not required for these sub tables; byte alignment is sufficient.

There are also two different formats for glyph metrics: big glyph metrics and small glyph metrics. Big glyph metrics define metrics information for both horizontal and vertical layouts. This is important in fonts (such as Kanji) where both types of layout may be used. Small glyph metrics define metrics information for one layout direction only. Which direction applies, horizontal or vertical, is determined by the 'flags' field in the BitmapSize table within the 'EBLC' table.

BigGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | horiBearingX |
| int8 | horiBearingY |
| uint8 | horiAdvance |
| int8 | vertBearingX |
| int8 | vertBearingY |
| uint8 | vertAdvance |

SmallGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | bearingX |
| int8 | bearingY |
| uint8 | advance |

#### Glyph bitmap data formats

The nine different formats currently defined for glyph bitmap data are listed and described below. Different formats are better for different purposes.

In all formats, if the bitDepth is greater than 1, all of a pixel's bits are stored consecutively in memory, and all of a row's pixels are stored consecutively.

NOTE	Each of these formats can contain black/white or grayscale bitmaps depending on the setting of the bitDepth field in the 'EBLC' table. For performance reasons, we recommend using a byte-aligned format for embedded bitmaps with bitDepth of 8.

##### Format 1: small metrics, byte-aligned data

| Type | Name | Description |
| --- | --- | --- |
| SmallGlyphMetrics | smallMetrics | Metrics information for the glyph |
| VARIABLE | imageData | Byte-aligned bitmap data |

Glyph bitmap format 1 consists of small metrics records (either horizontal or vertical depending on the flags field of the BitmapSize table within the 'EBLC' table) followed by byte aligned bitmap data. The bitmap data begins with the most significant bit of the first byte corresponding to the top-left pixel of the bounding box, proceeding through succeeding bits moving left to right. The data for each row is padded to a byte boundary, so the next row begins with the most significant bit of a new byte. 1 bits correspond to black, and 0 bits to white.

##### Format 2: small metrics, bit-aligned data

| Type | Name | Description |
| --- | --- | --- |
| SmallGlyphMetrics | smallMetrics | Metrics information for the glyph |
| VARIABLE | imageData | Bit-aligned bitmap data |

Glyph bitmap format 2 is the same as format 1 except that the bitmap data is bit aligned. This means that the data for a new row will begin with the bit immediately following the last bit of the previous row. The start of each glyph must be byte aligned, so the last row of a glyph may require padding. This format takes a little more time to parse, but saves file space compared to format 1.

##### Format 3: (obsolete)

This format is not supported in OFF.

##### Format 4: metrics in EBLC, compressed data

NOTE	Glyph bitmap format 4 is a compressed format used by Macintosh platform in some of the East Asian fonts.

##### Format 5: metrics in EBLC, bit-aligned image data only

| Type | Name | Description |
| --- | --- | --- |
| VARIABLE | imageData | Bit-aligned bitmap data |

Glyph bitmap format 5 is similar to format 2 except that no metrics information is included, just the bit aligned data. This format is for use with 'EBLC' IndexSubTable format 2 or format 5, which will contain the metrics information for all glyphs. It works well for Kanji fonts.

The rasterizer recalculates sbit metrics for Format 5 bitmap data, allowing Windows to report correct ABC widths, even if the bitmaps have white space on either side of the bitmap image. This allows fonts to store monospaced bitmap glyphs in the efficient Format 5 without breaking Windows GetABCWidths call.

##### Format 6: big metrics, byte-aligned data

| Type | Name | Description |
| --- | --- | --- |
| BigGlyphMetrics | bigMetrics | Metrics information for the glyph |
| VARIABLE | imageData | Byte-aligned bitmap data |

Glyph bitmap format 6 is the same as format 1 except that is uses big glyph metrics instead of small.

##### Format7: big metrics, bit-aligned data

| Type | Name | Description |
| --- | --- | --- |
| BigGlyphMetrics | bigMetrics | Metrics information for the glyph |
| VARIABLE | imageData | Bit-aligned bitmap data |

Glyph bitmap format 7 is the same as format 2 except that is uses big glyph metrics instead of small.

EbdtComponent Record

The EbdtComponent record is used in glyph bitmap data formats 8 and 9.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Component glyph ID |
| int8 | xOffset | Position of component left |
| int8 | yOffset | Position of component top |

The EbdtComponent record contains the glyph ID of the component, which can be used to look up the location of component glyph data in the 'EBLC' table, as well as xOffset and yOffset values which specify where to position the top-left corner of the component in the composite. Nested composites (a composite of composites) are allowed, and the number of nesting levels is determined by implementation stack space.

##### Format 8: small metrics, component data

| Type | Name | Description |
| --- | --- | --- |
| SmallGlyphMetrics | smallMetrics | Metrics information for the glyph |
| uint8 | pad | Pad to short boundary |
| uint16 | numComponents | Number of components |
| EbdtComponent | components[numComponents] | Array of EbdtComponent records |

##### Format 9: big metrics, component data

| Type | Name | Description |
| --- | --- | --- |
| BigGlyphMetrics | bigMetrics | Metrics information for the glyph |
| uint16 | numComponents | Number of components |
| EbdtComponent | components[numComponents] | Array of EbdtComponent records |

Glyph bitmap formats 8 and 9 are used for composite bitmaps. For accented characters and other composite glyphs, it may be more efficient to store a copy of each component separately, and then use a composite description to construct the finished glyph. The composite formats allow for any number of components, and allow the components to be positioned anywhere in the finished glyph. Format 8 uses small metrics, and format 9 uses big metrics.

### EBLC—Embedded bitmap location table

The EBLC provides embedded bitmap locators. It is used together with the EDBT table, which provides embedded, monochrome or grayscale bitmap glyph data, and the EBSC table, which provided embedded bitmap scaling information.

OFF embedded bitmaps are also called 'sbits' (for “scaler bitmaps”). A set of bitmaps for a face at a given size is called a strike.

#### Table structure and data types

The 'EBLC' table identifies the sizes and glyph ranges of the sbits, and keeps offsets to glyph bitmap data in IndexSubTables. The 'EBDT' table then stores the glyph bitmap data, also in a number of different possible formats. Glyph metrics information may be stored in either the 'EBLC' or 'EBDT' table, depending upon the IndexSubTable and glyph bitmap formats. The 'EBSC' table identifies sizes that will be handled by scaling up or scaling down other sbit sizes.

The 'EBLC' table begins with a header containing the table version and number of strikes. An OFF font may have one or more strikes embedded in the 'EBDT' table.

EblcHeader

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the EBLC table, = 2. |
| uint16 | minorVersion | Minor version of the EBLC table, = 0. |
| uint32 | numSizes | Number of BitmapSize tables. |

The EblcHeader is followed immediately by the BitmapSize table array(s). The numSizes field in the EblcHeader indicates the number of BitmapSize tables in the array. Each strike is defined by one BitmapSize table.

BitmapSize table

| Type | Name | Description |
| --- | --- | --- |
| Offset32 | indexSubTableArrayOffset | Offset to IndexSubtableArray, from beginning of EBLC. |
| uint32 | indexTablesSize | Number of bytes in corresponding index subtables and array. |
| uint32 | numberOfIndexSubTables | There is an index subtable for each range or format change. |
| uint32 | colorRef | Not used; set to 0. |
| SbitLineMetrics | hori | Line metrics for text rendered horizontally. |
| SbitLineMetrics | vert | Line metrics for text rendered vertically. |
| uint16 | startGlyphIndex | Lowest glyph index for this size. |
| uint16 | endGlyphIndex | Highest glyph index for this size. |
| uint8 | ppemX | Horizontal pixels per em. |
| uint8 | ppemY | Vertical pixels per em. |
| uint8 | bitDepth | The following bitDepth values are supported, as described below: 1, 2, 4, and 8. |
| int8 | flags | Vertical or horizontal (see Bitmap Flags) |

The indexSubTableArrayOffset is the offset from the beginning of the 'EBLC' table to the IndexSubTableArray. Each strike has one of these arrays to support various formats and discontinuous ranges of bitmaps. The indexTablesSize is the total number of bytes in the IndexSubTableArray and the associated IndexSubTables. The numberOfIndexSubTables is a count of the IndexSubTables for this strike.

#### Description of table entries

The horizontal and vertical line metrics contain the ascender, descender, linegap, and advance information for the strike. The line metrics format is described in the following table:

SbitLineMetrics

| Type | Name |
| --- | --- |
| int8 | ascender |
| int8 | descender |
| uint8 | widthMax |
| int8 | caretSlopeNumerator |
| int8 | caretSlopeDenominator |
| int8 | caretOffset |
| int8 | minOriginSB |
| int8 | minAdvanceSB |
| int8 | maxBeforeBL |
| int8 | minAfterBL |
| int8 | pad1 |
| int8 | pad2 |

The caret slope determines the angle at which the caret is drawn, and the offset is the number of pixels (+ or -) to move the caret. This is a signed char since we are dealing with integer metrics. The minOriginSB, minAdvanceSB, maxBeforeBL, and minAfterBL are described in the diagrams below. The main need for these numbers is for scalers that may need to pre-allocate memory and/or need more metric information to position glyphs. All of the line metrics are one byte in length. The line metrics are not used directly by the rasterizer, but are available to clients who want to parse the 'EBLC' table.

The startGlyphIndex and endGlyphIndex describe the minimum and maximum glyph IDs in the strike, but a strike does not necessarily contain bitmaps for all glyph IDs in this range. The IndexSubTables determine which glyphs are actually present in the 'EBDT' table.

The ppemX and ppemY fields describe the size of the strike in pixels per Em. The ppem measurement is equivalent to point size on a 72 dots per inch device. Typically, ppemX will be equal to ppemY for devices with 'square pixels'. To accommodate devices with rectangular pixels, and to allow for bitmaps with other aspect ratios, ppemX and ppemY may differ.

The bitDepth field is used to specify the number of levels of gray used in the embedded bitmaps. The Windows rasterizer v.1.7 or greater support the following values.

Bit Depth

The bitDepth field of the BitmapSize table is used to specify the number of levels of gray used in the embedded bitmaps. The following values are supported.

| Value | Description |
| --- | --- |
| 1 | black/white |
| 2 | 4 levels of gray |
| 4 | 16 levels of gray |
| 8 | 256 levels of gray |

The 'flags' byte contains two bits to indicate the direction of small glyph metrics: horizontal or vertical. The remaining bits are reserved.

Bitmap Flags

| Mask | Name | Description |
| --- | --- | --- |
| 0x01 | HORIZONTAL_METRICS | Horizontal |
| 0x02 | VERTICAL_METRICS | Vertical |
| 0xFC | Reserved | Reserved for future use – set to 0. |

The colorRef and bitDepth fields are reserved for future enhancements. For monochrome bitmaps they should have the values colorRef=0 and bitDepth=1.

Figure 5.7 – Horizontal text

Figure 5.8 – Vertical text

Associated with the image data for every glyph in a strike is a set of glyph metrics. These glyph metrics describe bounding box height and width, as well as side bearing and advance width information. The glyph metrics can be found in one of two places. For ranges of glyphs (not necessarily the whole strike) whose metrics may be different for each glyph, the glyph metrics are stored along with the glyph image data in the 'EBDT' table. Details of how this is done is described in 'EBDT'. For ranges of glyphs whose metrics are identical for every glyph, we save significant space by storing a single copy of the glyph metrics in the IndexSubTable in the 'EBLC'.

There are also two different formats for glyph metrics: big glyph metrics and small glyph metrics. Big glyph metrics define metrics information for both horizontal and vertical layouts. This is important in fonts (such as Kanji) where both types of layout may be used. Small glyph metrics define metrics information for one layout direction only. Which direction applies, horizontal or vertical, is determined by the 'flags' field in the BitmapSize table.

BigGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | horiBearingX |
| int8 | horiBearingY |
| uint8 | horiAdvance |
| int8 | vertBearingX |
| int8 | vertBearingY |
| uint8 | vertAdvance |

SmallGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | bearingX |
| int8 | bearingY |
| uint8 | advance |

The following diagram illustrates the meaning of the glyph metrics.

Figure 5.9 – Glyph metrics

The BitmapSize table for each strike contains the offset to an array of IndexSubTableArray elements. Each element describes a glyph ID range and an offset to the IndexSubTable for that range. This allows a strike to contain multiple glyph ID ranges and to be represented in multiple index formats if desirable.

IndexSubTableArray

| Type | Name | Description |
| --- | --- | --- |
| uint16 | firstGlyphIndex | First glyph ID of this range |
| uint16 | lastGlyphIndex | Last glyph ID of this range (inclusive) |
| Offset32 | additionalOffsetToIndexSubtable | Add to indexSubTableArrayOffset to get offset from beginning of 'EBLC' |

After determining the strike, the rasterizer searches this array for the range containing the given glyph ID. When the range is found, the additionalOffsetToIndexSubtable is added to the indexSubTableArrayOffset to get the offset of the IndexSubTable in the 'EBLC'.

The first IndexSubTableArray is located after the last bitmapSizeSubTable entry. Then the IndexSubTables for the strike follow. Another IndexSubTableArray (if more than one strike) and its IndexSubTables are next. The 'EBLC' continues with an array and IndexSubTables for each strike.

We now have the offset to the IndexSubTable. All IndexSubTable formats begin with an IndexSubHeader which identifies the IndexSubTable format, the format of the 'EBDT' image data, and the offset from the beginning of the 'EBDT' table to the beginning of the image data for this range.

IndexSubHeader

| Type | Name | Description |
| --- | --- | --- |
| uint16 | indexFormat | Format of this IndexSubTable |
| uint16 | imageFormat | Format of 'EBDT' image data |
| Offset32 | imageDataOffset | Offset to image data in 'EBDT' table |

IndexSubTables

There are currently five different formats used for the IndexSubTable, depending upon the size and type of bitmap data in the glyph ID range.

The choice of which IndexSubTable format to use is up to the font manufacturer, but should be made with the aim of minimizing the size of the font file. Ranges of glyphs with variable metrics - that is, where glyphs may differ from each other in bounding box height, width, side bearings or advance - must use format 1, 3 or 4. Ranges of glyphs with constant metrics can save space by using format 2 or 5, which keep a single copy of the metrics information in the IndexSubTable rather than a copy per glyph in the 'EBDT' table. In some monospaced fonts it makes sense to store extra white space around some of the glyphs to keep all metrics identical, thus permitting the use of format 2 or 5.

Structures for each IndexSubTable format are listed below.

IndexSubTable1: variable-metrics glyphs with 4-byte offsets

| Type | Name | Description |
| --- | --- | --- |
| IndexSubHeader | header | Header info. |
| Offset32 | offsetArray[] | offsetArray[glyphIndex] + imageDataOffset = glyphData sizeOfArray = (lastGlyph – firstGlyph + 1) + 1 + 1, pad if needed |

IndexSubTable2: all glyphs have identical metrics

| Type | Name | Description |
| --- | --- | --- |
| IndexSubHeader | header | Header info |
| uint32 | imageSize | All the glyphs are of the same size. |
| BigGlyphMetrics | bigMetrics | All glyphs have the same metrics; glyph data may be compressed, byte-aligned, or bit-aligned. |

IndexSubTable3: variable-metrics glyphs with 2-byte offsets

| Type | Name | Description |
| --- | --- | --- |
| IndexSubHeader | header | Header info. |
| Offset16 | offsetArray[] | offsetArray[glyphIndex] + imageDataOffset = glyphData sizeOfArray = (lastGlyph – firstGlyph + 1) + 1 + 1, pad if needed |

IndexSubTable4: variable-metrics glyphs with sparse glyph IDs

| Type | Name | Description |
| --- | --- | --- |
| IndexSubHeader | header | Header info. |
| uint32 | numGlyphs | Array length |
| GlyphIdOffsetPair | glyphArray[] | One per glyph; sizeOfArray = numGlyphs+1 |

GlyphIdOffsetPair record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Glyph ID of glyph present. |
| Offset16 | offset | Location in EBDT |

IndexSubTable5: constant-metrics glyphs with sparse glyph IDs

| Type | Name | Description |
| --- | --- | --- |
| IndexSubHeader | header | Header info. |
| uint32 | imageSize | All glyphs have the same data size |
| BigGlyphMetrics | bigMetrics | All glyphs have the same metrics |
| uint32 | numGlyphs | Array length |
| uint16 | glyphIDArray[] | One per glyph, sorted by glyph ID; sizeOfArray=numGlyphs |

The size of the 'EBDT' image data can be calculated from the IndexSubTable information. For the constant metrics formats (2 and 5) the image data size is constant, and is given in the imageSize field. For the variable metrics formats (1, 3, and 4) image data must be stored contiguously and in glyph ID order, so the image data size may be calculated by subtracting the offset for the current glyph from the offset of the next glyph. Because of this, it is necessary to store one extra element in the OffsetArray pointing just past the end of the range's image data. This will allow the correct calculation of the image data size for the last glyph in the range.

Contiguous, or nearly contiguous, ranges of glyph IDs are handled best by formats 1, 2, and 3 which store an offset for every glyph ID in the range. Very sparse ranges of glyph IDs should use format 4 or 5 which explicitly call out the glyph IDs represented in the range. A small number of missing glyphs can be efficiently represented in formats 1 or 3 by having the offset for the missing glyph be followed by the same offset for the next glyph, thus indicating a data size of zero.

The only difference between formats 1 and 3 is the size of the OffsetArray elements: format 1 uses uint32's while format 3 uses uint16's. Therefore, format 1 can cover a greater range (> 64k bytes) while format 3 saves more space in the 'EBLC' table. Since the OffsetArray elements are added to the imageDataOffset base address in the IndexSubHeader, a very large set of glyph bitmap data could be addressed by splitting it into multiple ranges, each less than 64k bytes in size, allowing the use of the more efficient format 3.

The 'EBLC' table specification requires double word (uint32) alignment for all subtables. This occurs naturally for IndexSubTable formats 1, 2, and 4, but may not for formats 3 and 5, since they include arrays of type uint16. When there are an odd number of elements in these arrays it is necessary to add an extra padding element to maintain proper alignment.

### EBSC—Embedded bitmap scaling table

The 'EBSC' table provides a mechanism for describing embedded bitmaps which are created by scaling other embedded bitmaps. While this is the sort of thing that outline font technologies were invented to avoid, there are cases (small sizes of Kanji, for example) where scaling a bitmap produces a more legible glyph than scan-converting an outline. For this reason, the 'EBSC' table allows a font to define a bitmap strike as a scaled version of another strike.

The EBSC table is used together with the EBDT table, which provides embedded monochrome or grayscale bitmap data, and the EBLC table, which provides embedded bitmap locators.

EBSC Header

The 'EBSC' table begins with a header containing the table version and number of strikes.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the EBSC table, = 2. |
| uint16 | minorVersion | Minor version of the EBSC table, = 0. |
| uint32 | numSizes |  |

The header is followed immediately by the BitmapScale table array. The numSizes field in the header indicates the number of BitmapScale tables in the array. Each strike is defined by one BitmapScale table.

BitmapScale Table

| Type | Name | Description |
| --- | --- | --- |
| SbitLineMetrics | hori | line metrics |
| SbitLineMetrics | vert | line metrics |
| uint8 | ppemX | target horizontal pixels per Em |
| uint8 | ppemY | target vertical pixels per Em |
| uint8 | substitutePpemX | use bitmaps of this size |
| uint8 | substitutePpemY | use bitmaps of this size |

The line metrics have the same meaning as those in the BitmapSize table, and refer to font wide metrics after scaling. The ppemX and ppemY values describe the size of the font after scaling. The substitutePpemX and substitutePpemY values describe the size of a strike that exists as an sbit in the 'EBLC' and 'EBDT', and that will be scaled up or down to generate the new strike.

Notice that scaling in the x direction is independent of scaling in the y direction, and their scaling values may differ. A square aspect-ratio strike could be scaled to a non-square aspect ratio. Glyph metrics are scaled by the same factor as the pixels per Em (in the appropriate direction), and are rounded to the nearest integer pixel.

### CBDT—Color bitmap data table

The 'CBDT' table is used to embed color bitmap glyph data. It is used together with the 'CBLC' table (see subclause 5.6.6), which provides embedded bitmap locators. The formats of these two tables are backward compatible with the 'EBDT' (subclause 5.6.2) and 'EBLC' (subclause 5.6.3) tables used for embedded monochrome and grayscale bitmaps.

#### Table structure

The 'CBDT' table begins with a header containing simply the table version number.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the CBDT table, = 3. |
| uint16 | minorVersion | Minor version of the CBDT table, = 0. |

The rest of the 'CBDT' table is a collection of bitmap data. The data can be presented in three possible formats, indicated by information in the 'CBLC' table. Some of the formats contain metric information plus image data, and other formats contain only the image data. Long word alignment is not required for these subtables; byte alignment is sufficient.

Uncompressed color bitmaps

The value ‘32’ of the bitDepth field of BitmapSize table struct defined in the CBLC table, to identify color bitmaps with 8-bit blue/green/red/alpha (“BGRA”) channels per pixel, encoded in that order for each pixel. The color channels represent pre-multiplied color and shall encode colors in the sRGB colorspace, as specified in IEC 61966-2-1/Amd 1:2003. For example, the color “full-green with half translucency” is encoded as \x00\x80\x00\x80, and not \x00\xFF\x00\x80.

All bitmap image data formats defined in the EBDT / EBLC tables are valid for use with the CBDT / CBLC tables.

Compressed color bitmaps

Images for each individual glyph are stored as straight PNG data, and shall be as specified in ISO/IEC 15948. Only the following chunks are allowed in such PNG data: IHDR, PLTE, tRNS, sRGB, IDAT, and IEND. If other chunks are present, the behavior is undefined. The image data shall be in the sRGB colorspace, regardless of color information that may be present in other chunks in the PNG data. The individual images must have the same size as expected by the table in the bitmap metrics.

Glyph Metrics

There are also two different formats for glyph metrics: big glyph metrics and small glyph metrics. Big glyph metrics define metrics information for both horizontal and vertical layouts. This is important in fonts (such as Kanji) where both types of layout may be used. Small glyph metrics define metrics information for one layout direction only. Which direction applies, horizontal or vertical, is determined by the 'flags' field in the BitmapSize tables within the 'CBLC' table.

BigGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | horiBearingX |
| int8 | horiBearingY |
| uint8 | horiAdvance |
| int8 | vertBearingX |
| int8 | vertBearingY |
| uint8 | vertAdvance |

SmallGlyphMetrics

| Type | Name |
| --- | --- |
| uint8 | height |
| uint8 | width |
| int8 | bearingX |
| int8 | bearingY |
| uint8 | advance |

#### Glyph bitmap data formats

In addition to nine different formats already defined for glyph bitmap data in the EBDT table, there are three new formats described below.

##### Format 17: small metrics, PNG image data

| Type | Name | Description |
| --- | --- | --- |
| smallGlyphMetrics | glyphMetrics | Metrics information for the glyph |
| uint32 | dataLen | Length of data in bytes |
| VARIABLE | data | Raw PNG data |

##### Format 18: big metrics, PNG image data

| Type | Name | Description |
| --- | --- | --- |
| BigGlyphMetrics | glyphMetrics | Metrics information for the glyph |
| uint32 | dataLen | Length of data in bytes |
| VARIABLE | data | Raw PNG data |

##### Format 19: metrics in CBLC, PNG image data

| Type | Name | Description |
| --- | --- | --- |
| uint32 | dataLen | Length of data in bytes |
| VARIABLE | data | Raw PNG data |

Scaling behavior

Applications using these glyphs may need to scale them to fit the raster size available for display. How this scaling takes place is application dependent. It is recommended that where possible the application downscale using the closest sized bitmap that is larger than the desired end raster size.

### CBLC—Color bitmap location table

The 'CBLC' table provides embedded bitmap locators. It is used together with the 'CBDT' table (subclause 5.6.5), which provides embedded, color bitmap glyph data. The formats of these two tables are backward compatible with the 'EBDT' (subclause 5.6.2) and 'EBLC' (subclause 5.6.3) tables used for embedded monochrome and grayscale bitmaps.

The 'CBLC' table begins with a header containing the table version and number of strikes. An OFF font may have one or more strikes embedded in the 'CBDT' table.

CblcHeader

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the 'CBLC' table, = 3. |
| uint16 | minorVersion | Minor version of the 'CBLC' table, = 0. |
| uint32 | numSizes | Number of BitmapSize tables |

The CblcHeader is followed immediately by the BitmapSize table array(s). The numSizes in the cblcHeader indicates the number of BitmapSize tables in the array. Each strike is defined by one BitmapSize table.

BitmapSize table

| Type | Name | Description |
| --- | --- | --- |
| Offset32 | indexSubTableArrayOffset | Offset to index subtable from beginning of CBLC. |
| uint32 | indexTablesSize | Number of bytes in corresponding index subtables and array |
| uint32 | numberOfIndexSubTables | There is an index subtable for each range or format change |
| uint32 | colorRef | Not used; set to 0. |
| SbitLineMetrics | hori | Line metrics for text rendered horizontally |
| SbitLineMetrics | vert | Line metrics for text rendered vertically |
| uint16 | startGlyphIndex | Lowest glyph index for this size |
| uint16 | endGlyphIndex | Highest glyph index for this size |
| uint8 | ppemX | Horizontal pixels per Em |
| uint8 | ppemY | Vertical pixels per Em |
| uint8 | bitDepth | In addition to already defined bitDepth values 1, 2, 4, and 8 supported by existing implementations, the value of 32 is used to identify color bitmaps with 8 bit per pixel RGBA channels. |
| int8 | flags | Vertical or horizontal (see Bitmap Flags section of subclause 5.6.3) |

The indexSubTableArrayOffset is the offset from the beginning of the 'CBLC' table to the IndexSubTableArray. Each strike has one of these arrays to support various formats and discontinuous ranges of bitmaps. The indexTablesSize is the total number of bytes in the IndexSubTableArray and the associated IndexSubTables. The numberOfIndexSubTables is a count of the IndexSubTables for this strike.

The rest of the CBLC table structure is identical to one already defined for EBLC, see subclause 5.6.3 for details.

### sbix—Standard bitmap graphics table

This table provides access to bitmap data in a standard graphics format, such as PNG, JPEG or TIFF.

The 'sbix' table has functionality somewhat similar to the 'EBDT' table in that both provide bitmap data for glyph presentation. They are different in three important respects, however. First, whereas the 'EBDT' table supports only black/white or grayscale bitmaps, the 'sbix' table supports color bitmaps. Secondly, whereas the 'EBDT' table uses formats specific to the OFF specification, the 'sbix' table uses standard bitmap graphics formats that are in common use. Thirdly, whereas the 'EBDT' table must be used in conjunction with other tables ('EBLC' and 'EBSC') for processing the bitmap data, the 'sbix' table contains complete data required for processing bitmaps.

A font that includes an 'sbix' table may also include outline glyph data in a 'glyf' or 'CFF ' table. An 'sbix' table can provide bitmap data for all glyph IDs, or for only a subset of glyph IDs. A font can also include different bitmap data for different sizes (“strikes”), and the glyph coverage for one size can be different from that for another size.

#### Header

'sbix' Header:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number – set to 1 |
| uint16 | flags | Bit 0: Set to 1. Bit 1: Draw outlines. Bits 2 to 15: reserved (set to 0). |
| uint32 | numStrikes | Number of bitmap strikes. |
| Offset32 | strikeOffsets[numStrikes] | Offsets from the beginning of the 'sbix' table to data for each individual bitmap strike. |

For historical reasons, bit 0 of the flags field should always be set to 1.

If bit 1 of the flags field is clear, then the application is instructed to draw only the bitmaps for each glyph supported in the 'sbix' table. If bit 1 is set, then the application is instructed to draw the bitmap and the outline, in that order (that is, with the outline overlaid on top of the bitmap). If the 'sbix' table does not contain bitmap data for a glyph, then the outline is always drawn, regardless of the state of bit 1.

NOTE	Application support for bit 1 of the flags field is optional. To ensure the best compatibility, set this bit to 0.

#### Strikes

Each strike includes a header and the glyph bitmap data. The header has the following format:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | ppem | The PPEM size for which this strike was designed. |
| uint16 | ppi | The device pixel density (in PPI) for which this strike was designed. (E.g., 96 PPI, 192 PPI.) |
| Offset32 | glyphDataOffsets[numGlyphs+1] | Offset from the beginning of the strike data header to bitmap data for an individual glyph ID. |

The glyphDataOffset array includes offsets for every glyph ID, plus one extra. The number of glyphs is determined from the 'maxp' table. The length of the bitmap data for each glyph is variable, and can be determined from the difference between two consecutive offsets. Hence, the length of data for glyph N is glyphDataOffset[N+1] – glyphDataOffset[N]. If this is zero, there is no bitmap data for that glyph in this strike. There is one extra offset in the array in order to provide the length of data for the last glyph.

NOTE	The length of data for non-printing glyphs, such as space, should always be zero.

A strike does not need to include data for every glyph, and does not need to include data for the same set of glyphs as other strikes. If the application is using bitmap data to draw text and there is bitmap data for a glyph in any strike, then the glyph must be drawn using a bitmap from some strike. If the exact size is not available, implementations may choose a bitmap based on the closest available larger size, or the closest available integer-multiple larger size, or on some other basis. The only cases in which a glyph is not drawn using a bitmap are if the application has not requested that text be drawn using bitmap data or if there is no bitmap data for the glyph in any strike.

Each strike targets a specific PPEM size and device pixel density (PPI). Thus, a font can contain two strikes for the same PPEM but different pixel densities, or two strikes for the same pixel density but different PPEMs. Note that some platforms may not support targeting of strikes for particular pixel densities.

#### Glyph data

The data for each glyph includes a header and the actual, embedded graphic data, with the following format:

| Type | Name | Description |
| --- | --- | --- |
| int16 | originOffsetX | The horizontal (x-axis) offset from the left edge of the graphic to the glyph’s origin. That is, the x-coordinate of the point on the baseline at the left edge of the glyph. |
| int16 | originOffsetY | The vertical (y-axis) offset from the bottom edge of the graphic to the glyph’s origin. That is, the y-coordinate of the point on the baseline at the left edge of the glyph. |
| Tag | graphicType | Indicates the format of the embedded graphic data: one of 'jpg ', 'png ' or 'tiff', or the special format 'dupe'. |
| uint8 | data[] | The actual embedded graphic data. The total length is inferred from sequential entries in the glyphDataOffsets array and the fixed size (8 bytes) of the preceding fields. |

The graphicType field indicates the format of the embedded graphic data. Three standard formats are supported: JPEG, PNG and TIFF; these are indicated using tag values 'jpg ', 'png ' and 'tiff', respectively.

The special graphicType of 'dupe' indicates that the data field contains a two-byte, big-endian glyph ID. The bitmap data for the indicated glyph should be used for the current glyph.

NOTE	Apple’s specification for TrueType fonts allows for a graphicType tag value of 'pdf ' or 'mask'. These values are not supported in the OFF specification, however.

#### Table dependencies

The glyph count is derived from the 'maxp' table. Advance and side-bearing glyph metrics are stored in the 'hmtx' table for horizontal layout, and the 'vmtx' table for vertical layout.

## Optional tables

| Tag | Name |
| --- | --- |
| DSIG | Digital signature |
| hdmx | Horizontal device metrics |
| Kern | Kerning |
| LTSH | Linear threshold data |
| PCLT | PCL 5 data |
| VDMX | Vertical device metrics |
| vhea | Vertical Metrics header |
| vmtx | Vertical Metrics |
| COLR | Color table |
| CPAL | Color palette table |

### DSIG—Digital signature table

The DSIG table contains the digital signature of the OFF font. Signature formats are widely documented and rely on a key pair architecture. Software developers, or publishers posting material on the Internet, create signatures using a private key. Operating systems or applications authenticate the signature using a public key.

The W3C and major software and operating system developers have specified security standards that describe signature formats, specify secure collections of web objects, and recommend authentication architecture. OFF fonts with signatures will support these standards.

OFF fonts offer many security features:

- Operating systems and browsing applications can identify the source and integrity of font files before using them,

- Font developers can specify embedding restrictions in OFF fonts, and these restrictions cannot be altered in a font signed by the developer.

The enforcement of signatures is an administrative policy that may be supported by the host environment in which fonts are used. Systems may restrict use of unsigned fonts, or may allow policy to be controlled by a system administrator.

Anyone can obtain identity certificates and encryption keys from a certifying agency, such as Verisign or GTE's Cybertrust, free or at a very low cost.

The DSIG table is organized as follows. The first portion of the table is the header.

DSIG Header

| Type | Name | Description |
| --- | --- | --- |
| uint32 | version | Version number of the DSIG table (0x00000001) |
| uint16 | numSignatures | Number of signatures in the table |
| uint16 | flags | Shall be set to 0x0001 |
| SignatureRecord | signatureRecords[numSignatures] | Array of signature records |

The version of the DSIG table is expressed as a uint32, beginning at 0. The version of the DSIG table currently used is version 1 (0x00000001).

Permission bit 0 allows a party signing the font to prevent any other parties from also signing the font (counter-signatures). If this bit is set to zero (0) the font may have a signature applied over the existing digital signature(s). A party who wants to ensure that their signature is the last signature can set this bit.

The DSIG header has an array of signature records that specify the format and offset of signature blocks.

SignatureRecord

| Type | Name | Description |
| --- | --- | --- |
| uint32 | format | Format of the signature |
| uint32 | length | Length of signature in bytes |
| Offset32 | signatureBlockOffset | Offset to the signature block from the beginning of the table |

Signatures are contained in one or more signature blocks. Signature blocks may have various formats; currently one format is defined. The format identifier specifies both the format of the signature block, as well as the hashing algorithm used to create and authenticate the signature.

Signature Block Format 1

| Type | Name | Description |
| --- | --- | --- |
| uint16 | reserved1 | Reserved for future use; set to zero. |
| uint16 | reserved2 | Reserved for future use; set to zero. |
| uint32 | signatureLength | Length (in bytes) of the PKCS#7 packet in the signature field. |
| uint8 | signature[signatureLength] | PKCS#7 packet |

For more information about PKCS#7 signatures see Reference [12].

For more information about counter-signatures, see Reference [13].

Format 1: For whole fonts, with either TrueType outlines and/or CFF data

PKCS#7 or PKCS#9. The signed content digest is created as follows:

- If there is an existing DSIG table in the font:

  - Remove the DSIG table from font.

  - Remove the DSIG table entry from the table directory.

  - Adjust table offsets as necessary.

  - Recalculate the checksumAdjustment in the ‘head’ table.

- Hash the revised font data using a secure one-way hash (such as MD5) to create the content digest.

- Create the PKCS#7 signature block using the content digest.

- Create a new DSIG table containing the signature block.

- Add the DSIG table to the font, adjusting table offsets as necessary.

- Add a DSIG table entry to the table directory.

- Recalculate the checksumAdjustment in the ‘head’ table.

Validation of a signature in a font is done by repeating steps 1 – 4 in an in-memory copy of the font file. Note that changing the checksumAdjustment in the last step does not break the signature because verification is done on an in-memory copy with these changes.

Prior to signing a font file, ensure that all the following attributes are true:

- The magic number in the ‘head’ table is correct.

- Given the numTables value in the Table Directory, the other values in the Table Directory are consistent.

- The table records in the Table Directory are ordered alphabetically by the table tags, and there are no duplicate tags.

- The offset of each table is a multiple of 4. (That is, tables are long word aligned.)

- The first actual table in the file comes immediately after the directory of tables.

- If the tables are sorted by offset, then for all tables i (where index 0 means the table with the smallest offset), Offset[i] + Length[i] <= Offset[i+1] and Offset[i] + Length[i] >= Offset[i+1] - 3. In other words, the tables do not overlap, and there are at most 3 bytes of padding between tables.

- The pad bytes between tables are all zeros.

- The offset of the last table in the file plus its length is not greater than the size of the file.

- The checksums of all tables are correct.

- The ‘head’ table's checksumAdjustment field is correct.

Signatures for Font Collections

The DSIG table for a Font Collection (TTC) shall be the last table in the TTC file. The offset to the table is put in the TTCHeader (version 2). Signatures of TTC files are expected to be Format 1 signatures.

The signature of a TTC file applies to the entire file, not to the individual fonts contained within the TTC. Signing the TTC file ensures that other contents are not added to the TTC.

Individual fonts included in a font collection should not be individually signed as the process of making the TTC could invalidate the signature on the font.

When DSIG table is created for a collection file, the steps given above are used, with these revisions:

- In step 1: if there is an existing DSIG table referenced in a version 2.0 TTC header, the DSIG table is removed, and the DSIG fields in the header is set to NULL. No recalculation of a checksumAdjustment is required.

- In steps 6 and 7: the DSIG table is added to the file, not to any individual font within the collection. A version 2.0 TTC header is required, with the DSIG fields in the header set to reference the DSIG table.

- Step 8 is not applicable.

See the TTC Header description (subclause 4.6.3) for related information.

### hdmx—Horizontal device metrics

The hdmx table relates to OFF fonts with TrueType outlines. The Horizontal Device Metrics table stores integer advance widths scaled to particular pixel sizes. This allows the font manager to build integer width tables without calling the scaler for each glyph. Typically, this table contains only selected screen sizes. This table is sorted by pixel size. The checksum for this table applies to both subtables listed.

NOTE	For non-square pixel grids, the character width (in pixels) will be used to determine which device record to use. For example, a 12-point character on a device with a resolution of 72x96 would be 12 pixels high and 16 pixels wide. The hdmx device record for 16 pixel characters would be used.

If bit 4 of the flag field in the 'head' table is not set, then it is assumed that the font scales linearly; in this case an 'hdmx' table is not necessary and should not be built. If bit 4 of the flag field is set, then one or more glyphs in the font are assumed to scale nonlinearly. In this case, performance can be improved by including the 'hdmx' table with one or more important DeviceRecord's for important sizes. See Clause 7 "Recommendations for OFF Fonts" for more detail.

The table begins as follows:

| hdmx Header |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint16 | version | Table version number (0) |
| int16 | numRecords | Number of device records. |
| int32 | sizeDeviceRecord | Size of a device record, long aligned. |
| DeviceRecord | records[numRecords] | Array of device records. |

Each DeviceRecord for format 0 looks like this.

| Device Record |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint8 | pixelSize | Pixel size for following widths (as ppem). |
| uint8 | maxWidth | Maximum width. |
| uint8 | widths[numGlyphs] | Array of widths (numGlyphs is from the 'maxp' table). |

Each DeviceRecord is padded with 0's to make it long word aligned.

Each Width value is the width of the particular glyph, in pixels, at the pixels per em (ppem) size listed at the start of the DeviceRecord.

The ppem sizes are measured along the y axis.

### kern—Kerning

The kerning table contains the values that control the inter-character spacing for the glyphs in a font.

Each subtable varies in format, and can contain information for vertical or horizontal text, and can contain kerning values or minimum values. Kerning values are used to adjust inter-character spacing, and minimum values are used to limit the amount of adjustment that the scaler applies by the combination of kerning and tracking. Because the adjustments are additive, the order of the subtables containing kerning values is not important. However, tables containing minimum values should usually be placed last, so that they can be used to limit the total effect of other subtables.

The kerning table in the OFF font file has a header, which contains the format number and the number of subtables present, and the subtables themselves.

| Type | Field | Description |
| --- | --- | --- |
| uint16 | version | Table version number (0) |
| uint16 | nTables | Number of subtables in the kerning table. |

Kerning subtables will share the same header format. This header is used to identify the format of the subtable and the kind of information it contains:

| Type | Field | Description |
| --- | --- | --- |
| uint16 | version | Kern subtable version number (0) |
| uint16 | length | Length of the subtable, in bytes (including this header). |
| uint16 | coverage | What type of information is contained in this table. |

The coverage field is divided into the following sub-fields, with sizes given in bits:

| Sub-field | Bits #'s | Size | Description |
| --- | --- | --- | --- |
| horizontal | 0 | 1 | 1 if table has horizontal data, 0 if vertical. |
| minimum | 1 | 1 | If this bit is set to 1, the table has minimum values. If set to 0, the table has kerning values. |
| cross-stream | 2 | 1 | If set to 1, kerning is perpendicular to the flow of the text. If the text is normally written horizontally, kerning will be done in the up and down directions. If kerning values are positive, the text will be kerned upwards; if they are negative, the text will be kerned downwards. If the text is normally written vertically, kerning will be done in the left and right directions. If kerning values are positive, the text will be kerned to the right; if they are negative, the text will be kerned to the left. The value 0x8000 in the kerning data resets the cross-stream kerning back to 0. |
| override | 3 | 1 | If this bit is set to 1 the value in this table should replace the value currently being accumulated. |
| reserved1 | 4-7 | 4 | Reserved. This should be set to zero. |
| format | 8-15 | 8 | Format of the subtable. Only formats 0 and 2 have been defined. Formats 1 and 3 through 255 are reserved for future use. |

Format 0

Format 0 is the only subtable format supported by Windows.

This subtable is a sorted list of kerning pairs and values. The list is preceded by information which makes it possible to make an efficient binary search of the list:

| Type | Field | Description |
| --- | --- | --- |
| uint16 | nPairs | This gives the number of kerning pairs in the table. |
| uint16 | searchRange | The largest power of two less than or equal to the value of nPairs, multiplied by the size in bytes of an entry in the table. |
| uint16 | entrySelector | This is calculated as log2 of the largest power of two less than or equal to the value of nPairs. This value indicates how many iterations of the search loop will have to be made. (For example, in a list of eight items, there would have to be three iterations of the loop). |
| uint16 | rangeShift | The value of nPairs minus the largest power of two less than or equal to nPairs, and then multiplied by the size in bytes of an entry in the table. |

This is followed by the list of kerning pairs and values. Each has the following format:

| Type | Field | Description |
| --- | --- | --- |
| uint16 | left | The glyph index for the left-hand glyph in the kerning pair. |
| uint16 | right | The glyph index for the right-hand glyph in the kerning pair. |
| FWORD | value | The kerning value for the above pair, in font design units. If this value is greater than zero, the characters will be moved apart. If this value is less than zero, the character will be moved closer together. |

The left and right halves of the kerning pair make an unsigned 32-bit number, which is then used to order the kerning pairs numerically.

A binary search is most efficiently coded if the search range is a power of two. The search range can be reduced by half by shifting instead of dividing. In general, the number of kerning pairs, nPairs, will not be a power of two. The value of the search range, searchRange, should be the largest power of two less than or equal to nPairs. The number of pairs not covered by searchRange (that is, nPairs - searchRange) is the value rangeShift.

Format 2

This subtable is a two-dimensional array of kerning values. The glyphs are mapped to classes, using a different mapping for left- and right-hand glyphs. This allows glyphs that have similar right- or left-side shapes to be handled together. Each similar right- or left-hand shape is said to be single class.

Each row in the kerning array represents one left-hand glyph class, each column represents one right-hand glyph class, and each cell contains a kerning value. Row and column 0 always represent glyphs that do not kern and contain all zeros.

The values in the right class table are stored pre-multiplied by the number of bytes in a single kerning value, and the values in the left class table are stored pre-multiplied by the number of bytes in one row. This eliminates needing to multiply the row and column values together to determine the location of the kerning value. The array can be indexed by doing the right- and left-hand class mappings, adding the class values to the address of the array, and fetching the kerning value to which the new address points.

The header for the simple array has the following format:

| Type | Field | Description |
| --- | --- | --- |
| uint16 | rowWidth | The width, in bytes, of a row in the table. |
| Offset16 | leftClassTable | Offset from beginning of this subtable to left-hand class table. |
| Offset16 | rightClassTable | Offset from beginning of this subtable to right-hand class table. |
| Offset16 | array | Offset from beginning of this subtable to the start of the kerning array. |

Each class table has the following header:

| Type | Field | Description |
| --- | --- | --- |
| uint16 | firstGlyph | First glyph in class range. |
| uint16 | nGlyphs | Number of glyph in class range. |

This header is followed by nGlyphs number of class values, which are in uint16 format. Entries for glyphs that don't participate in kerning should point to the row or column at position zero.

The array itself is a left by right array of kerning values, which are FWords, where left is the number of left-hand classes and R is the number of right-hand classes. The array is stored by row.

NOTE	This format is the quickest to process since each lookup requires only a few index operations. The table can be quite large since it will contain the number of cells equal to the product of the number of right-hand classes and the number of left-hand classes, even though many of these classes do not kern with each other.

### LTSH—Linear threshold

The LTSH table relates to OFF fonts containing TrueType outlines. There are noticeable improvements to fonts on the screen when instructions are carefully applied to the sidebearings. The gain in readability is offset by the necessity for the OS to grid fit the glyphs in order to find the actual advance width for the glyphs (since instructions may be moving the sidebearing points). The TrueType outline format already has two mechanisms to side step the speed issues: the 'hdmx' table, where precomputed advance widths may be saved for selected ppem sizes, and the 'vdmx' table, where precomputed vertical advance widths may be saved for selected ppem sizes. The 'LTSH' table (Linear ThreSHold) is a second, complementary method.

The LTSH table defines the point at which it is reasonable to assume linearly scaled advance widths on a glyph-by-glyph basis. This table should not be included unless bit 4 of the "flags" field in the 'head' table is set. The criteria for linear scaling is:

- (ppem size is ≥ 50) AND (difference between the rounded linear width and the rounded instructed width ≤ 2% of the rounded linear width) or

- Linear width == Instructed width

The LTSH table records the ppem for each glyph at which the scaling becomes linear again, despite instructions effecting the advance width. It is a requirement that, at and above the recorded threshold size, the glyph remain linear in its scaling (i.e., not legal to set threshold at 55 ppem if glyph becomes nonlinear again at 90 ppem). The format for the table is:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Version number (starts at 0). |
| uint16 | numGlyphs | Number of glyphs (from "numGlyphs" in 'maxp' table). |
| uint8 | yPels[numGlyphs] | The vertical pel height at which the glyph can be assumed to scale linearly. On a per glyph basis. |

NOTE	Glyphs which do not have instructions on their sidebearings have yPels = 1; i.e., always scales linearly.

### MERG—Merge table

The 'MERG' table enables a font to specify whether antialiasing filtering of glyphs within a glyph run can be performed separately for each glyph, or whether certain glyph pairs or sequences should be composed together – or merged – before antialiasing is performed.

When glyphs are composed together after antialiasing has been performed, that can result in rendering artifacts in some cases in which glyphs touch or overlap. (This is true of any per-primitive antialiasing.) Merging glyphs together before antialiasing eliminates those artifacts, but it also adds a significant performance cost. A font can use the MERG table to indicate specific glyph pairs or sequences for which pre-antialiasing merging is required in order to avoid the risk of rendering artifacts, while implicitly declaring that other glyph pairs or sequences do not require pre-antialiasing merging.

NOTE 1	Hereafter, “merging” will be used to refer to composing of glyphs together prior to antialiasing filtering.

NOTE 2	Some implementations may use caching of glyph-rendering results as a means of performance optimization. If merging of glyph sequences is not required, then the cached renderings may be composed without a need to render glyphs or the glyph sequence again.

The approach used is to give a positive declaration of cases in which merging should be performed. If a 'MERG' table is provided but there no declarations are made for any pairs, then the intended interpretation is that no merging is necessary. (Some implementations may still merge glyphs before antialiasing, however.) If no 'MERG' table is provided, then implementations should always merge glyphs before filtering in order to avoid artifacts.

Data is provided for pairs of glyph classes. The first and second glyph elements in a pair correspond to logical ordering of glyphs in a run. Since glyphs are processed in logical order but may be presented in visual left-to-right or right-to-left order, it is possible to give separate recommendations for either left-to-right or right-to-left order.

In some cases, it may be necessary to consider interaction of sequences of more than two glyphs rather than simply a pair. The data format allows the font developer to specify sequences that need to be treated together in merging; this is explained further below.

Determination of whether or not merging should be done is a design consideration on the part of the font developer. Glyphs may touch or overlap, but there might not be any perceptible artifacts in visual results. If the designer determines that merging for a particular pair or sequence is not needed to provide adequate visual results, then there will be performance benefits from not declaring that pair or sequence as requiring merging.

Note that different platforms may support different rendering techniques, and may or may not support the 'MERG' table. On some platforms, glyphs sequences might always be merged before antialiasing is performed, regardless of whether 'MERG' data is provided in the font. Similarly, in some other platforms, glyphs might always be composed after antialiasing is performed, regardless of whether 'MERG' data is provided in the font. Font developers should consult developer documentation for the different platforms on which fonts will be used to determine what benefits a 'MERG' table may provide, and should evaluate rendered results on relevant platforms to determine which glyph pairs or sequences should be declared as requiring merging.

To construct a 'MERG' table, the first step is to classify glyphs based on desired merging behavior such that each glyph has an associated merge class (represented by a zero-based index). The system of classification and number of classes will depend on the font and the font developer’s discretion, but could take into account such properties as a glyph’s general shape and whether it connects to other glyphs. After assigning glyphs into classes, one then selects a recommended merging behavior for each pair of merge classes.

NOTE	The number of pairs for which data is provided is the square of the number of merge classes. Therefore, the number of merge classes should be as small as possible.

#### Grouping of glyphs

In some cases, a sequence of glyphs may need to be treated as a unit for purposes of merging. For example, a glyph for a combining accent might not typographically interact at all with following glyphs, yet it might come in logical order between glyphs that do interact and that may require merging. This is illustrated in the following Figure 5.10 (assume left-to-right visual order).

Figure 5.10: Accent glyph logically between two typographically-interacting glyphs

In this case, the accent and the base “e” glyph can be treated as a unit for purposes of evaluating the required merge behavior with the following “f” glyph. Note that, if merging of the “e” and “f” glyphs is required, then the accent will also need to be included in the sequence that gets antialiased together.

The merge data entries that specify how pairs of glyphs should be handled include values that indicate that the pair of glyphs should be grouped together as a unit, without specifying whether merging is required. Whether or not merging is needed will get determined only as this group is compared with other glyphs. In the above figure 5.10, the accent is grouped together with the “e” glyph, but whether or not any merging is required will be determined by comparing that combination with the following “f” glyph.

Whenever a pair of glyphs is grouped or merged, then one or the other will be most relevant when the combination is evaluated in relation to the following glyph. In the example above, the required behavior for the “e”-plus-accent combination when it interacts with the following glyph can be determined by the “e”. In a different example, it may be the second of a pair of glyphs that is most relevant for purposes of interaction with subsequent glyphs. Thus, whenever a pair of glyphs is grouped or merged, the data indicates whether the merge class of the sequence takes on the class of the first or the second element of the pair. This is indicated by use of a flag: in the unmarked case (flag is not set), the sequence takes the merge class of the second glyph. But if a second is subordinate flag is set, then the sequence takes on the merge class of the first element of the pair.

See the following sections for complete details regarding the merge entry values and how they are processed.

#### Table formats

The 'MERG' table is comprised by a header, a set of class-definition tables, and an array of merge-entry data. The format of the header is as follows:

Merge header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Version number of the merge table — set to 0. |
| uint16 | mergeClassCount | The number of merge classes. |
| Offset16 | mergeDataOffset | Offset to the array of merge-entry data. |
| uint16 | classDefCount | The number of class definition tables. |
| Offset16 | offsetToClassDefOffsets | Offset to an array of offsets to class definition tables — in bytes from the start of the 'MERG' table. |

The offsetToClassDefOffsets field provides an offset to the start of an array of offsets. Each element in the array is an offset (unsigned 16-bit) from the start of the 'MERG' table to a class definition table. The classDefCount field gives the number of elements in the offsets array, and the number of class definition tables.

NOTE 1	A given class definition table can be used to assign different glyphs into multiple classes. The number of class definition tables does not determine the number of merge classes. Rather, the mergeClassCount field determines the number of classes that can be referenced by the merge-entry data. Specifically, merge entries are provided for merge classes 0 to mergeClassCount - 1. If any glyph is assigned to a class ID greater than or equal to mergeClassCount, there will be no merge entries for pairs involving that class, which effectively means that merging of that glyph with other glyphs is never required.

The class definition tables use the same formats as are used in OFF Layout tables. Both ClassDefFormat1 and ClassDefFormat2 may be used. For details on class definition table formats, see the Class definition table section of subclause 6.2.

NOTE 2	A class definition table gives an explicit assignment of glyphs to specific class IDs. Any glyph that is not assigned to a class are implicitly assigned to class zero.

Any given glyph must be assigned to at most one class. Moreover, as the class definition tables are read in order, glyph ID references must be in strictly increasing order. If glyph IDs are given out of order, the 'MERG' table is invalid and is ignored.

The merge-entry data array is a 2D table of entries for glyph-class pairs. Each entry is a uint8 value, and the total size of the data is mergeClassCount^2. The data are organized as mergeClassCount number of rows each having mergeClassCount number of column entries.

MergeEntry table

| Type | Name | Description |
| --- | --- | --- |
| MergeEntryRow | mergeEntryRows [mergeClassCount] | Array of merge-entry rows. |

MergeEntryRow record:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | mergeEntries [mergeClassCount] | Array of merge entries. |

Each merge entry specifies a behavior for a pair of merge classes: the row index represents the class of the first element, in logical order, and the column index represents the class of the second element.

Each merge entry is a bit field with six flags defined. These describe three different processing behaviors for both left-to-right and right-to-left visual orders. The flags are assigned as follows:

Merge Entry Flags

| Mask | Name | Description |
| --- | --- | --- |
| 0x01 | MERGE_LTR | Merge glyphs, for LTR visual order. |
| 0x02 | GROUP_LTR | Group glyphs, for LTR visual order. |
| 0x04 | SECOND_IS_SUBORDINATE_LTR | Second glyph is subordinate to the first glyph, for LTR visual order. |
| 0x08 | Reserved | Flag reserved for future use — set to 0. |
| 0x10 | MERGE_RTL | Merge glyphs, for RTL visual order. |
| 0x20 | GROUP_RTL | Group glyphs, for RTL visual order. |
| 0x40 | SECOND_IS_SUBORDINATE_RTL | Second glyph is subordinate to the first glyph, for RTL visual order. |
| 0x80 | Reserved | Flag reserved for future use — set to 0. |

The Merge flags (MergeLTR, MergeRTL) indicate that the pair of items should be merged prior to antialiasing.

The Group flags, described in the previous section, indicate that the pair should be treated as a unit, without indicating whether or not merging is required — that will be determine be evaluating the combination in relation to other glyphs.

The SecondIsSubordinate flags, also described in the previous section, are used only if the Merge or Group flag for the same visual order was set. These indicate whether the class for the merged or grouped sequence should be that of the first or second item of the pair. If a SecondIsSubordinate flag is set but neither the Merge or Group flag for the same visual order was set, then it is ignored.

A detailed description of handling of the Merge, Group and SecondIsSubordinate flags is provided in the following subclause.

#### Processing

The merge entries are used while processing glyphs in a glyph run to determine which sequences of glyphs require merging before antialiasing filtering is performed. The following description is given in a way that is generic with regard to visual order. So, for instance “the Merge flag” refers to the MergeLTR flag if the visual order is LTR, or to the MergeRTL flag if the visual order is RTL.

In the following description, a merge group is a sequence of one or more glyphs that are processed as a unit. In addition to the glyph sequence, a merge group has a boolean mergeRequired property that is set by default to false. The group also has a mergeClass property, that is set as described below.

Merge processing proceeds as follows:

- Start: The start state for the processing algorithm is one in which the current glyph did not need to be merged with a preceding glyph or glyph sequence. (This includes the start of a glyph run.) The current glyph is the start of a new merge group with group.mergeRequired = false.

- Determine the merge class of the current glyph. Set the group.mergeClass to this class ID.

- Process next glyph: Determine the merge class of the next glyph.

- Using group.mergeClass as a row index and the merge class of the next glyph as a column index, retrieve the merge entry for the given row and column.

- If the merge entry is zero, or if the merge class for either the current or next glyph was greater or equal to mergeClassCount, then the next glyph does not need to be merged into the current merge sequence. Do not add the next glyph into the merge group, but proceed to step 10.

- Else, if the merge entry has the Merge flag set, then the next glyph is added to the current merge group, and group.mergeRequired is set to true. Proceed to step 8.

- Else, if the merge entry has the Group flag set, then the next glyph is added to the current merge group. The group.mergeRequired property is not changed. Proceed to step 8.

- Determine the new merge class for the group:

  - If the merge entry has the SecondIsSubordinate flag set then the group.mergeClass property is not changed.

  - Else (the SecondIsSubordinate flag is clear), then set the group.mergeClass property to be the merge class of the next glyph.

- The merge group has been extended; proceed to the next glyph: next becomes current, and the group properties remain as set in steps 6 – 8. Return to step 3.

- The merge group is terminated:

  - If group.mergeRequired is true, then merge all of the glyphs in this merge group prior to antialiasing filtering.

  - Else (group.mergeRequired is false), then merging is not required for any of the glyphs in the merge group.

  - Proceed to next glyph (next becomes current) and return to the start state, step 1.

### meta—Metadata table

The metadata table contains various metadata values for the font. Different categories of metadata are identified by four-character tags. Values for different categories can be either binary or text.

#### Table formats

The metadata table begins with a header, structured as follows:

Metadata header

| Type | Name | Description |
| --- | --- | --- |
| uint32 | version | Version number of the metadata table – set to 1. |
| uint32 | flags | Flags – currently unused; set to 0. |
| uint32 | Reserved | Not used; set to 0. |
| uint32 | dataMapsCount | The number of data maps in the table. |
| DataMap | dataMaps[dataMapsCount] | Array of data map records. |

NOTE	The reserved field was originally documented in Apple TrueType specification as a data offset. This was redundant, since DataMap records include offsets from the start of the 'meta' table, and therefore not used.

The data map record has the following format:

DataMap record

| Type | Name | Description |
| --- | --- | --- |
| Tag | tag | A tag indicating the type of metadata. |
| Offset32 | dataOffset | Offset in bytes from the beginning of the metadata table to the data for this tag. |
| uint32 | dataLength | Length of the data, in bytes. The data is not required to be padded to any byte boundary. |

The data for a given record may be either textual or binary. The representation format is specified for each tag. Depending on the tag, multiple records for a given tag or multiple, delimited values in a record may or may not be permitted, as specified for each tag. If only one record or value is permitted for a tag, then any instances after the first may be ignored.

#### Metadata Tags

Metadata tags identify the category of information provided and representation format used for a given metadata value. A registry of commonly-used tags is maintained, but private, vendor-determined tags can also be used.

Like other OFF tags, metadata tags are four unsigned bytes that can equivalently be interpreted as a string of four ASCII characters. Metadata tags shall begin with a letter (0x41 to 0x5A, 0x61 to 0x7A) and must use only letters, digits (0x30 to 0x39) or space (0x20). Space characters must only occur as trailing characters in tags that have fewer than four letters or digits.

Privately-defined metadata tags shall begin with an uppercase letter (0x41 to 0x5A), and shall use only uppercase letters or digits. Registered axis tags must not use that pattern, but can be any other valid pattern.

Every registered tag defines the semantics of the associated metadata values, and the representation format of those values. Values for registered tags may be may be either textual or binary. If textual, it will be in UTF-8 encoding unless explicitly indicated otherwise.

The following registered tags are defined or reserved at this time:

| Tag | Name |  | Description |
| --- | --- | --- | --- |
| appl | (reserved) |  | Reserved — used by Apple. |
| bild | (reserved) |  | Reserved — used by Apple. |
| dlng | Design languages | Text, using only Basic Latin (ASCII) characters. | Indicates languages and/or scripts for the user audiences that the font was primarily designed for. Only one instance is used. See below for additional details. |
| slng | Supported languages | Text, using only Basic Latin (ASCII) characters. | Indicates languages and/or scripts that the font is declared to be capable of supporting. Only one instance See below for additional details. |

The values for 'dlng' and 'slng' are comprised of a series of comma-separated ScriptLangTags, which are described in detail below. Spaces may follow the comma delimiters and are ignored. Each ScriptLangTag identifies a language or script. A list of tags is interpreted to imply that all of the languages or scripts are included.

The 'dlng' value is used to indicate the languages or scripts of the primary user audiences for which the font was designed. This value may be useful for selecting default font formatting based on content language, for presenting filtered font options based on user language preferences, or similar applications involving the language or script of content or user settings.

The 'slng' value is used to declare languages or scripts that the font is capable of supported. This value may be useful for font fallback mechanisms or other applications involving the language or script of content or user settings.

NOTE 	Implementations that use 'slng' values in a font may choose to ignore Unicode-range bits set in the OS/2 table.

Some examples will help to understand the distinction between design and supported languages:

- Consider the case of accented Latin letters: Although the accents are used in common by a number of languages, the precise shape of the accents can depend on the typographic traditions of a specific language. Polish, for example, prefers steeper accents than French. A font that was designed with accents specifically for Polish would declare Polish as a design language, but might declare support for any language using Latin script.

- Fonts designed for East Asian markets will generally include glyphs for Latin, Greek and Cyrillic because these characters are included in important East Asian character set standards, but using East Asian fonts for languages that are written with those scripts is generally unsatisfactory. Such fonts would therefore include these scripts in the 'slng' value, but not in their 'dlng' value.

- There are some systematic differences in glyph design for the characters shared by simplified and traditional Chinese, such as the way the “bone” radical is drawn in all characters using it. A font specifically designed for use with simplified Chinese can usually be used to display traditional Chinese, but any character with the “bone” radical will look wrong to readers of traditional Chinese. Such a font would include simplified Chinese 'dlng' value, but both simplified and traditional Chinese in its 'slng' value.

#### ScriptLangTag Values

The 'dlng' and 'slng' metadata use ScriptLangTag values, defined here.

A ScriptLangTag denotes a particular language or script associated with a font. These are adapted from the IETF BCP 47 specification [25].

BCP 47 language tags can include various subtags that provide different types of qualifiers, such as language, script or region. In a BCP 47 language tag, a language subtag element is mandatory and other subtags are optional. ScriptLangTag values used for 'dlng' and 'slng' metadata values use a modification of the BCP 47 syntax: a script subtag is mandatory and other subtags are optional. The following augmented BNF syntax, adapted from BCP 47, is used:

ScriptLangTag = [language "-"]
                    script
                    ["-" region]
                    *("-" variant)
                    *("-" extension)
                    ["-" privateuse]

The expansion of the elements and the intended semantics associated with each are as defined in BCP 47. Script subtags are taken from ISO 15924. At present, no extensions are defined for use in ScriptLangTags, and any extension will be ignored. Private-use elements, which are prefixed with “-x”, are defined by private agreement between the source and recipient and may be ignored.

Subtags must be valid for use in BCP 47 and contained in the Language Subtag Registry [26] maintained by IANA. See also section 3 of BCP 47 for details.

NOTE	OFF Layout script and language system tags are not the same as those used in BCP 47 and should not be referenced when creating or processing ScriptLangTags.

Any ScriptLangTag value not conforming to these specifications is ignored.

A ScriptLangTag can denote fairly specific information; for example, 'en-Latn-IN' would represent "Latin script as used for the English language in India". In most cases, however, generic tags should be used, and it is anticipated that most tags used in 'dlng' and 'slng' metadata declarations will consist only of a script subtag. Language or other subtags can be included, however, and may be appropriate in some cases. Implementations must allow for ScriptLangTags that include additional subtags, but they may also choose to interpret only the script subtag and ignore other subtags.

Examples:

- 'Latn' denotes Latin script (and any language or writing system using Latin script).

- 'Cyrl' denotes Cyrillic script.

- 'sr-Cyrl' denotes Cyrillic script as used for writing the Serbian language; a font that has this property value may not be suitable for displaying text in Russian or other languages written using Cyrillic script.

- 'en-Dsrt' denotes English written with the Deseret script.

- 'Hant' denotes Traditional Chinese.

- 'Hant-HK' denotes Traditional Chinese as used in China.

- 'Jpan' denotes Japanese writing – ISO 15924 defines 'Jpan' as an alias for Han + Hiragana + Katakana.

- 'Kore' denotes Korean writing – ISO 15924 defines 'Kore' as an alias for Hangul + Han.

- 'Hang' denotes Hangul script (exclusively – Hanja are not implied by 'Hang').

The Unicode Standard uses the ISO 15924 identifiers 'Zinh' ("inherited") and 'Zyyy' ("undetermined"). These should not be used in ScriptLangTags. Similarly, 'Zxxx' ("unwritten document") and 'Zzzz' ("unencoded script") should never be used.

On the other hand, 'Zmth' ("Mathematical notation") and 'Zsym' ("Symbols”) are not used in the Unicode Standard, yet they may be very useful as declarations in font files. (They were, in fact, added to ISO 15924 for use in relation to fonts.)

In relation to East Asian scripts, a declaration of 'Jpan' can be used to cover hiragana, katakana and kanji. Similarly, 'Kore' can be used to cover Hangul and hanja, though a Korean font with only Hangul support should use 'Hang'. For Chinese fonts, 'Hans' and 'Hant' should normally be used to distinguish between Simplified and Traditional orthographies rather than the more generic declaration 'Hani'. Region-specific variations such as 'Hant-HK' can also be declared. In some cases, it may be appropriate to describe a font capability (but probably not design target) using the generic declaration 'Hani' (denoting ‘Han / Hanzi / Kanji / Hanja’).

The BCP 47 specification for region subtags allows for continental and sub-continental regions. For example, “039” can be used to denote Southern Europe. Use of such extended-region subtags in ScriptLangTag values is not recommended as software implementations may not have the logic to make appropriate correlations to more specific regions or languages associated with those regions.

### PCLT—PCL 5 table

The 'PCLT' table is strongly discouraged for OFF fonts with TrueType outlines. Extra information on many of these fields can be found in the HP PCL 5 Printer Language Technical Reference Manual available from Hewlett-Packard Boise Printer Division.

The format for the table is:

| Type | Name of Entry |
| --- | --- |
| uint16 | majorVersion |
| uint16 | minorVersion |
| uint32 | FontNumber |
| uint16 | Pitch |
| uint16 | xHeight |
| uint16 | Style |
| uint16 | TypeFamily |
| uint16 | CapHeight |
| uint16 | SymbolSet |
| int8 | Typeface[16] |
| int8 | CharacterComplement[8] |
| int8 | FileName[6] |
| int8 | StrokeWeight |
| int8 | WidthType |
| uint8 | SerifStyle |
| uint8 | Reserved (pad) |

Major and Minor Version

The current PCLT table version is 1.0.

FontNumber

This 32-bit number is segmented in two parts. The most significant bit indicates native versus converted format. Only font vendors should create fonts with this bit zeroed. The 7 next most significant bits are assigned by Hewlett-Packard Boise Printer Division to major font vendors. The least significant 24 bits are assigned by the vendor. Font vendors should attempt to ensure that each of their fonts are marked with unique values.

| Code | Vendor |
| --- | --- |
| A | Adobe Systems |
| B | Bitstream Inc. |
| C | Agfa Corporation |
| H | Bigelow & Holmes |
| L | Linotype Company |
| M | Monotype Typography Ltd. |

Pitch

The width of the space in font design units (font design units are described by the unitsPerEm field of the 'head' table). Monospace fonts derive the width of all characters from this field.

xHeight

The height of the optical line describing the height of the lowercase x in font design units. This might not be the same as the measured height of the lowercase x.

Style

The most significant 6 bits are reserved. The 5 next most significant bits encode structure. The next 3 most significant bits encode appearance width. The 2 least significant bits encode posture.

Structure (bits 5-9)

| 0 | Solid (normal, black) |
| --- | --- |
| 1 | Outline (hollow) |
| 2 | Inline (incised, engraved) |
| 3 | Contour, edged (antique, distressed) |
| 4 | Solid with shadow |
| 5 | Outline with shadow |
| 6 | Inline with shadow |
| 7 | Contour, or edged, with shadow |
| 8 | Pattern filled |
| 9 | Pattern filled #1 (when more than one pattern) |
| 10 | Pattern filled #2 (when more than two patterns) |
| 11 | Pattern filled #3 (when more than three patterns) |
| 12 | Pattern filled with shadow |
| 13 | Pattern filled with shadow #1 (when more than one pattern or shadow) |
| 14 | Pattern filled with shadow #2 (when more than two patterns or shadows) |
| 15 | Pattern filled with shadow #3 (when more than three patterns or shadows) |
| 16 | Inverse |
| 17 | Inverse with border |
| 18-31 | reserved |

Width (bits 2-4)

| 0 | normal |
| --- | --- |
| 1 | condensed |
| 2 | compressed, extra condensed |
| 3 | extra compressed |
| 4 | ultra compressed |
| 5 | reserved |
| 6 | expanded, extended |
| 7 | extra expanded, extra extended |

Posture (bits 0-1)

| 0 | upright |
| --- | --- |
| 1 | oblique, italic |
| 2 | alternate italic (backslanted, cursive, swash) |
| 3 | reserved |

TypeFamily

The 4 most significant bits are font vendor codes. The 12 least significant bits are typeface family codes. Both are assigned by HP Boise Division.

Vendor Codes (bits 12-15)

| 0 | reserved |
| --- | --- |
| 1 | Agfa Corporation |
| 2 | Bitstream Inc. |
| 3 | Linotype Company |
| 4 | Monotype Typography Ltd. |
| 5 | Adobe Systems |
| 6 | font repackagers |
| 7 | vendors of unique typefaces |
| 8-15 | reserved |

CapHeight

The height of the optical line describing the top of the uppercase H in font design units. This might not be the same as the measured height of the uppercase H.

SymbolSet

The most significant 11 bits are the value of the symbol set "number" field. The value of the least significant 5 bits, when added to 64, is the ASCII value of the symbol set "ID" field. Symbol set values are assigned by HP Boise Division. Unbound fonts, or "typefaces" should have a symbol set value of 0. See the PCL 5 Printer Language Technical Reference Manual or the PCL 5 Comparison Guide for the most recent published list of codes.

Examples

|  | PCL | decimal |
| --- | --- | --- |
| Windows 3.1 "ANSI" | 19U | 629 |
| Windows 3.0 "ANSI" | 9U | 309 |
| Adobe "Symbol" | 19M | 621 |
| Macintosh | 12J | 394 |
| PostScript ISO Latin 1 | 11J | 362 |
| PostScript Std. Encoding | 10J | 330 |
| Code Page 1004 | 9J | 298 |
| DeskTop | 7J | 234 |

TypeFace

This 16-byte ASCII string appears in the "font print" of PCL printers. Care should be taken to ensure that the base string for all typefaces of a family are consistent, and that the designators for bold, italic, etc. are standardized.

Example

| Times New |  |
| --- | --- |
| Times New | Bd |
| Times New | It |
| Times New | BdIt |
| Courier New |  |
| Courier New | Bd |
| Courier New | It |
| Courier New | BdIt |

CharacterComplement

This 8-byte field identifies the symbol collections provided by the font, each bit identifies a symbol collection and is independently interpreted. Symbol set bound fonts should have this field set to all F's (except bit 0).

Example

| DOS/PCL Complement | 0xFFFFFFFF003FFFFE |
| --- | --- |
| Windows 3.1 "ANSI" | 0xFFFFFFFF37FFFFFE |
| Macintosh | 0xFFFFFFFF36FFFFFE |
| ISO 8859-1 Latin 1 | 0xFFFFFFFF3BFFFFFE |
| ISO 8859-1,2,9 Latin 1,2,5 | 0xFFFFFFFF0BFFFFFE |

The character collections identified by each bit are as follows:

| 31 | ASCII (supports several standard interpretations) |
| --- | --- |
| 30 | Latin 1 extensions |
| 29 | Latin 2 extensions |
| 28 | Latin 5 extensions |
| 27 | Desktop Publishing Extensions |
| 26 | Accent Extensions (East and West Europe) |
| 25 | PCL Extensions |
| 24 | Macintosh Extensions |
| 23 | PostScript Extensions |
| 22 | Code Page Extensions |

The character complement field also indicates the index mechanism used with an unbound font. Bit 0 must always be cleared when the font elements are provided in Unicode order.

FileName

This 6-byte field is composed of 3 parts. The first 3 bytes are an industry standard typeface family string. The fourth byte is a treatment character, such as R, B, I. The last two characters are either zeroes for an unbound font or a two character mnemonic for a symbol set if symbol set found.

Examples

| TNRR00 | Times New (text weight, upright) |
| --- | --- |
| TNRI00 | Times New Italic |
| TNRB00 | Times New Bold |
| TNRJ00 | Times New Bold Italic |
| COUR00 | Courier |
| COUI00 | Courier Italic |
| COUB00 | Courier Bold |
| COUJ00 | Courier Bold Italic |

Treatment Flags

| R | Text, normal, book, etc. |
| --- | --- |
| I | Italic, oblique, slanted, etc. |
| B | Bold |
| J | Bold Italic, Bold Oblique |
| D | Demibold |
| E | Demibold Italic, Demibold Oblique |
| K | Black |
| G | Black Italic, Black Oblique |
| L | Light |
| P | Light Italic, Light Oblique |
| C | Condensed |
| A | Condensed Italic, Condensed Oblique |
| F | Bold Condensed |
| H | Bold Condensed Italic, Bold Condensed Oblique |
| S | Semibold (lighter than demibold) |
| T | Semibold Italic, Semibold Oblique |

Other treatment flags are assigned over time.

StrokeWeight

This signed 1-byte field contains the PCL stroke weight value. Only values in the range -7 to 7 are valid:

| -7 | Ultra Thin |
| --- | --- |
| -6 | Extra Thin |
| -5 | Thin |
| -4 | Extra Light |
| -3 | Light |
| -2 | Demilight |
| -1 | Semilight |
| 0 | Book, text, regular, etc. |
| 1 | Semibold (Medium, when darker than Book) |
| 2 | Demibold |
| 3 | Bold |
| 4 | Extra Bold |
| 5 | Black |
| 6 | Extra Black |
| 7 | Ultra Black, or Ultra |

Type designers often use interesting names for weights or combinations of weights and styles, such as Heavy, Compact, Inserat, Bold No. 2, etc. PCL stroke weights are assigned on the basis of the entire family and use of the faces. Typically, display faces don't have a "text" weight assignment.

WidthType

This signed 1-byte field contains the PCL appearance width value. The values are not directly related to those in the appearance with field of the style word above. Only values in the range -5 to 5 are valid.

| -5 | Ultra Compressed |
| --- | --- |
| -4 | Extra Compressed |
| -3 | Compressed, or Extra Condensed |
| -2 | Condensed |
| 0 | Normal |
| 2 | Expanded |
| 3 | Extra Expanded |

SerifStyle

This uint8 field contains the PCL serif style value. The most significant 2 bits of this byte specify the serif/sans or contrast/monoline characteristics of the typeface.

Bottom 6 bit values:

| 0 | Sans Serif Square |
| --- | --- |
| 1 | Sans Serif Round |
| 2 | Serif Line |
| 3 | Serif Triangle |
| 4 | Serif Swath |
| 5 | Serif Block |
| 6 | Serif Bracket |
| 7 | Rounded Bracket |
| 8 | Flair Serif, Modified Sans |
| 9 | Script Nonconnecting |
| 10 | Script Joining |
| 11 | Script Calligraphic |
| 12 | Script Broken Letter |

Top 2 bit values:

| 0 | reserved |
| --- | --- |
| 1 | Sans Serif/Monoline |
| 2 | Serif/Contrasting |
| 3 | reserved |

Reserved

Should be set to zero.

### VDMX—Vertical device metrics

The VDMX table relates to OFF fonts with TrueType outlines. Under Windows, the usWinAscent and usWinDescent values from the 'OS/2' table will be used to determine the maximum black height for a font at any given size. Windows calls this distance the Font Height. Because TrueType instructions can lead to Font Heights that differ from the actual scaled and rounded values, basing the Font Height strictly on the yMax and yMin can result in "lost pixels". Windows will clip any pixels that extend above the yMax or below the yMin. In order to avoid grid fitting the entire font to determine the correct height, the VDMX table has been defined.

The VDMX table consists of a header followed by groupings of VDMX records:

| VDMX Header |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint16 | version | Version number (0 or 1). |
| uint16 | numRecs | Number of VDMX groups present |
| uint16 | numRatios | Number of aspect ratio groupings |
| RatioRange | ratRange[numRatios] | Ratio record array |
| Offset16 | Offset[numRatios] | Offset from start of this table to the VDMX group for a corresponding RatioRange record. |
| Vdmx | groups | The actual VDMX groupings (documented below) |

| RatioRange Record |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint8 | bCharSet | Character set (see below). |
| uint8 | xRatio | Value to use for x-Ratio |
| uint8 | yStartRatio | Starting y-Ratio value. |
| uint8 | yEndRatio | Ending y-Ratio value. |

Ratios are set up as follows:

| For a 1:1 aspect ratio | Ratios.xRatio = 1; Ratios.yStartRatio = 1; Ratios.yEndRatio = 1; |
| --- | --- |
| For 1:1 through 2:1 ratio | Ratios.xRatio = 2; Ratios.yStartRatio = 1; Ratios.yEndRatio = 2; |
| For 1.33:1 ratio | Ratios.xRatio = 4; Ratios.yStartRatio = 3; Ratios.yEndRatio = 3; |
| For all aspect ratios | Ratio.xRatio = 0; Ratio.yStartRatio = 0; Ratio.yEndRatio = 0; |

All values set to zero signal the default grouping to use; if present, this must be the last Ratio group in the table. Ratios of 2:2 are the same as 1:1.

Aspect ratios are matched against the target device by normalizing the entire ratio range record based on the current X resolution and performing a range check of Y resolutions for each record after normalization. Once a match is found, the search stops. If the 0,0,0 group is encountered during the search, it is used (therefore if this group is not at the end of the ratio groupings, no group that follows it will be used). If there is not a match and there is no 0,0,0 record, then there is no VDMX data for that aspect ratio.

NOTE	Range checks are conceptually performed as follows:

(deviceXRatio == Ratio.xRatio) && (deviceYRatio >= Ratio.yStartRatio) && (deviceYRatio <= Ratio.yEndRatio)

Each ratio grouping refers to a specific VDMX record group; there must be at least 1 VDMX group in the table.

The bCharSet value is used to denote cases where the VDMX group was computed based on a subset of the glyphs present in the font file. The semantics of bCharSet is different based on the version of the VDMX table. It is recommended that VDMX version 1 be used. The currently defined values for character set are:

| Character Set Values - Version 0 |  |
| --- | --- |
| Value | Description |
| 0 | No subset; the VDMX group applies to all glyphs in the font. This is used for symbol or dingbat fonts. |
| 1 | Windows ANSI subset; the VDMX group was computed using only the glyphs required to complete the Windows ANSI character set. Windows will ignore any VDMX entries that are not for the ANSI subset (i.e. ANSI_CHARSET). |

| Character Set Values - Version 1 |  |
| --- | --- |
| Value | Description |
| 0 | No subset; the VDMX group applies to all glyphs in the font. If adding new character sets to existing font, add this flag and the groups necessary to support it. This should only be used in conjunction with ANSI_CHARSET. |
| 1 | No subset; the VDMX group applies to all glyphs in the font. Used when creating a new font for Windows. No need to support SYMBOL_CHARSET. |

VDMX groups immediately follow the table header. Each set of records (there need only be one set) has the following layout:

| VDMX Group |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint16 | recs | Number of height records in this group |
| uint8 | startsz | Starting yPelHeight |
| uint8 | endsz | Ending yPelHeight |
| vTable | entry[recs] | The VDMX records |

| vTable Record |  |  |
| --- | --- | --- |
| Type | Name | Description |
| uint16 | yPelHeight | yPelHeight to which values apply. |
| int16 | yMax | Maximum value (in pels) for this yPelHeight. |
| int16 | yMin | Minimum value (in pels) for this yPelHeight. |

This table must appear in sorted order (sorted by yPelHeight), but need not be continuous. It should have an entry for every pel height where the yMax and yMin do not scale linearly, where linearly scaled heights are defined as:

Hinted yMax and yMin are identical to scaled/rounded yMax and yMin.

It is assumed that once yPelHeight reaches 255, all heights will be linear, or at least close enough to linear that it no longer matters. Please note that while the Ratios structure can only support ppem sizes up to 255, the vTable structure can support much larger pel heights (up to 65535). The choice of int16 and uint16 for vTable is dictated by the requirement that yMax and yMin be signed values (and 127 to -128 is too small a range) and the desire to word-align the vTable elements.

### vhea—Vertical header table

The vertical header table (tag name: 'vhea') contains information needed for vertical fonts. The glyphs of vertical fonts are written either top to bottom or bottom to top. This table contains information that is general to the font as a whole. Information that pertains to specific glyphs is given in the vertical metrics table (tag name: 'vmtx') described separately. The formats of these tables are similar to those for horizontal metrics (hhea and hmtx).

Data in the vertical header table must be consistent with data that appears in the vertical metrics table. The advance height and top sidebearing values in the vertical metrics table must correspond with the maximum advance height and minimum bottom sidebearing values in the vertical header table.

See the clause 6 "OFF CJK Font Guidelines" for more information about constructing CJK (Chinese, Japanese, and Korean) fonts.

The difference between version 1.0 and version 1.1 is the name and definition of the following fields:

- ascender becomes vertTypoAscender

- descender becomes vertTypoDescender

- lineGap becomes vertTypoLineGap

Version 1.0 of the vertical header table format is as follows:

Vertical Header Table v1.0

| Version 1.0 Type | Name | Description |
| --- | --- | --- |
| Version16Dot16 | version | Version number of the vertical header table; 0x00010000 for version 1.0 |
| int16 | ascent | Distance in font design units from the centerline to the previous line’s descent. |
| int16 | descent | Distance in font design units from the centerline to the next line’s ascent. |
| int16 | lineGap | Reserved; set to 0 |
| int16 | advanceHeightMax | The maximum advance height measurement -in font design units found in the font. This value must be consistent with the entries in the vertical metrics table. |
| int16 | minTop SideBearing | The minimum top sidebearing measurement found in the font, in font design units. This value must be consistent with the entries in the vertical metrics table. |
| int16 | minBottom SideBearing | The minimum bottom sidebearing measurement found in the font, in font design units. This value must be consistent with the entries in the vertical metrics table. |
| int16 | yMaxExtent | Defined as max(tsb + (yMax-yMin)) |
| int16 | caretSlopeRise | The value of the caretSlopeRise field divided by the value of the caretSlopeRun Field determines the slope of the caret. A value of 0 for the rise and a value of 1 for the run specifies a horizontal caret. A value of 1 for the rise and a value of 0 for the run specifies a vertical caret. Intermediate values are desirable for fonts whose glyphs are oblique or italic. For a vertical font, a horizontal caret is best. |
| int16 | caretSlopeRun | See the caretSlopeRise field. Value=1 for nonslanted vertical fonts. |
| int16 | caretOffset | The amount by which the highlight on a slanted glyph needs to be shifted away from the glyph in order to produce the best appearance. Set value equal to 0 for nonslanted fonts. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | metricDataFormat | Set to 0. |
| uint16 | numOf LongVerMetrics | Number of advance heights in the vertical metrics table. |

Version 1.1 of the vertical header table format is as follows:

Vertical Header Table v1.1

| Version 1.1 Type | Name | Description |
| --- | --- | --- |
| Version16Dot16 | version | Version number of the vertical header table; 0x00011000 for version 1.1 The representation of a non-zero fractional part, in Fixed numbers. |
| int16 | vertTypoAscender | The vertical typographic ascender for this font. It is the distance in FUnits from the ideographic em-box center baseline for the vertical axis to the right edge of the ideographic em-box. It is usually set to (head.unitsPerEm)/2. For example, a font with an em of 1000 FUnits will set this field to 500. See the §6.4.4. Baseline Tags of the OFF Layout Tag Registry for a description of the ideographic em-box. |
| int16 | vertTypoDescender | The vertical typographic descender for this font. It is the distance in FUnits from the ideographic em-box center baseline for the vertical axis to the left edge of the ideographic em-box. It is usually set to (head.unitsPerEm)/2. For example, a font with an em of 1000 FUnits will set this field to -500. |
| int16 | vertTypoLineGap | The vertical typographic gap for this font. An application can determine the recommended line spacing for single spaced vertical text for an OFF font by the following expression: ideo embox width + vhea.vertTypoLineGap |
| int16 | advanceHeightMax | The maximum advance height measurement -in font design units found in the font. This value must be consistent with the entries in the vertical metrics table. |
| int16 | minTop SideBearing | The minimum top sidebearing measurement found in the font, in font design units. This value must be consistent with the entries in the vertical metrics table. |
| int16 | minBottom SideBearing | The minimum bottom sidebearing measurement found in the font, in font design units. This value must be consistent with the entries in the vertical metrics table. |
| int16 | yMaxExtent | Defined as max(tsb + (yMax-yMin)) |
| int16 | caretSlopeRise | The value of the caretSlopeRise field divided by the value of the caretSlopeRun Field determines the slope of the caret. A value of 0 for the rise and a value of 1 for the run specifies a horizontal caret. A value of 1 for the rise and a value of 0 for the run specifies a vertical caret. Intermediate values are desirable for fonts whose glyphs are oblique or italic. For a vertical font, a horizontal caret is best. |
| int16 | caretSlopeRun | See the caretSlopeRise field. Value=1 for nonslanted vertical fonts. |
| int16 | caretOffset | The amount by which the highlight on a slanted glyph needs to be shifted away from the glyph in order to produce the best appearance. Set value equal to 0 for nonslanted fonts. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | reserved | Set to 0. |
| int16 | metricDataFormat | Set to 0. |
| uint16 | numOf LongVerMetrics | Number of advance heights in the vertical metrics table. |

'vhea' Table and OFF Font Variations

In a variable font, various font-metric values within the 'vhea' table may need to be adjusted for different variation instances. Variation data for 'vhea' entries can be provided in the metrics variations ('MVAR') table. Different 'post' entries are associated with particular variation data in the 'MVAR' table using value tags, as follows:

| 'vhea' entry | Tag |
| --- | --- |
| ascent | 'vasc' |
| caretOffset | 'vcof' |
| caretSlopeRun | 'vcrn' |
| caretSlopeRise | 'vcrs' |
| descent | 'vdsc' |
| lineGap | 'vlgp' |

For general information on OFF Font Variations, see subclause 7.1.

Vertical Header Table Example

| Offset/ length | Value | Name | Comment |
| --- | --- | --- | --- |
| 0/4 | 0x00011000 | version | Version number of the vertical header table, in fixed-point format, is 1.1 |
| 4/2 | 1024 | vertTypoAscender | Half the em-square height. |
| 6/2 | -1024 | vertTypoDescender | Minus half the em-square height. |
| 8/2 | 0 | vertTypoLineGap | Typographic line gap is 0 font design units. |
| 10/2 | 2079 | advanceHeightMax | The maximum advance height measurement found in the font is 2079 font design units. |
| 12/2 | -342 | minTopSideBearing | The minimum top sidebearing measurement found in the font is -342 font design units. |
| 14/2 | -333 | minBottomSideBearing | The minimum bottom sidebearing measurement found in the font is -333 font design units. |
| 16/2 | 2036 | yMaxExtent | max (tsb + (yMax - yMin)) = 2036. |
| 18/2 | 0 | caretSlopeRise | The caret slope rise of 0 and a caret slope run of 1 indicate a horizontal caret for a vertical font. |
| 20/2 | 1 | caretSlopeRun | The caret slope rise of 0 and a caret slope run of 1 indicate a horizontal caret for a vertical font. |
| 22/2 | 0 | caretOffset | Value set to 0 for nonslanted fonts. |
| 24/4 | 0 | reserved | Set to 0. |
| 26/2 | 0 | reserved | Set to 0. |
| 28/2 | 0 | reserved | Set to 0. |
| 30/2 | 0 | reserved | Set to 0. |
| 32/2 | 0 | metricDataFormat | Set to 0. |
| 34/2 | 258 | numOfLongVerMetrics | Number of advance heights in the vertical metrics table is 258. |

### vmtx—Vertical metric table

The vertical metrics table allows you to specify the vertical spacing for each glyph in a vertical font. This table consists of either one or two arrays that contain metric information (the advance heights and top sidebearings) for the vertical layout of each of the glyphs in the font. The vertical metrics coordinate system is shown below.

Figure 5.11 – Vertical Metrics

OFF vertical fonts require both a vertical header table ('vhea') and the vertical metrics table discussed below. The vertical header table contains information that is general to the font as a whole. The vertical metrics table contains information that pertains to specific glyphs. The formats of these tables are similar to those for horizontal metrics (hhea and hmtx).

See Clause 6 (OFF CJK Font Guidelines) for more information about constructing CJK (Chinese, Japanese, and Korean) fonts.

Vertical Origin and Advance Height

The y coordinate of a glyph's vertical origin is specified as the sum of the glyph's top side bearing (recorded in the 'vmtx' table) and the top (i.e. maximum y) of the glyph's bounding box.

TrueType OFF fonts contain glyph bounding box information in the Glyph Data ('glyf') table. CFF OFF fonts do not contain glyph bounding box information, and so for these fonts the top of the glyph's bounding box shall be calculated from the charstring data in the Compact Font Format ('CFF ') table.

OpenType 1.3 introduced the optional Vertical Origin ('VORG') table for CFF OFF fonts, which records the y coordinate of glyphs' vertical origins directly, thus obviating the need to calculate bounding boxes as an intermediate step. This improves accuracy and efficiency for CFF OFF clients.

The x coordinate of a glyph's vertical origin is not specified in the 'vmtx' table. Vertical writing implementations may make use of the baseline values in the Baseline ('BASE') table, if present, in order to align the glyphs in the x direction as appropriate to the desired vertical baseline.

The advance height of a glyph starts from the y coordinate of the glyph's vertical origin and advances downwards. Its endpoint is at the y coordinate of the vertical origin of the next glyph in the run, by default. Metric-adjustment OFF layout features such as Vertical Kerning ('vkrn') could modify the vertical advances in a manner similar to kerning in horizontal mode.

Vertical Metrics Table Format

The overall structure of the vertical metrics table consists of two arrays shown below: the vMetrics array followed by an array of top side bearings. The top side bearing is measured relative to the top of the origin of glyphs, for vertical composition of ideographic glyphs.

This table does not have a header, but does require that the number of glyphs included in the two arrays equals the total number of glyphs in the font.

The number of entries in the vMetrics array is determined by the value of the numOfLongVerMetrics field of the vertical header table.

The vMetrics array contains two values for each entry. These are the advance height and the top sidebearing for each glyph included in the array.

In monospaced fonts, such as Courier or Kanji, all glyphs have the same advance height. If the font is monospaced, only one entry need be in the first array, but that one entry is required.

The format of an entry in the vertical metrics array is given below.

| Type | Name | Description |
| --- | --- | --- |
| uint16 | advanceHeight | The advance height of the glyph. Unsigned integer in font design units |
| int16 | topSideBearing | The top sidebearing of the glyph. Signed integer in font design units. |

The second array is optional and generally is used for a run of monospaced glyphs in the font. Only one such run is allowed per font, and it shall be located at the end of the font. This array contains the top sidebearings of glyphs not represented in the first array, and all the glyphs in this array shall have the same advance height as the last entry in the vMetrics array. All entries in this array are therefore monospaced.

The number of entries in this array is calculated by subtracting the value of numOfLongVerMetrics from the number of glyphs in the font. The sum of glyphs represented in the first array plus the glyphs represented in the second array therefore equals the number of glyphs in the font. The format of the top sidebearing array is given below.

| Type | Name | Description |
| --- | --- | --- |
| int16 | topSideBearing[] | The top sidebearing of the glyph. Signed integer in font design units. |

### COLR—Color Table

The COLR table adds support for multi-colored glyphs in a manner that integrates with the rasterizers of existing text engines and that is designed to be easy to support with current OpenType font files.

The COLR table defines color presentations for glyphs. The color presentation of a glyph is specified as a graphic composition using other glyphs, such as a layered arrangement of glyphs, each with a different color. The term “color glyph” is used informally to refer to such a graphic composition defined in the COLR table; and the term “base glyph” is used to refer to a glyph for which a color glyph is provided. Processing of the COLR table is done on glyph sequences after text layout processing is completed and prior to final presentation of glyphs. Typically, a base glyph is a glyph that may occur in a sequence that results from the text layout process.

For example, the Unicode character U+1F600 is the grinning face emoji. Suppose in an emoji font the ‘cmap’ table maps U+1F600 to glyph ID 718. Assuming no glyph substitutions, glyph ID 718 would be considered the base glyph. Suppose the COLR table has data describing a color presentation for this using a layered arrangement of other glyphs with different colors assigned: that description and its presentation result would be considered the corresponding color glyph.

Two versions of the COLR table are defined.

Version 0 allows for a simple composition of colored elements: a linear sequence of glyphs that are stacked vertically as layers in bottom-up z-order. Each layer combines a glyph outline from the ‘glyf’, CFF or CFF2 table (referenced by glyph ID) with a solid color fill. These capabilities are sufficient to define color glyphs such as those illustrated in figure 5.12.

Figure 5.12 Examples of the graphic capabilities of COLR version 0

Version 1 supports additional graphic capabilities. In addition to solid colors, gradient fills can be used, as well as more complex fills using other graphic operations, including affine transformations and various blending modes. Version 1 capabilities allow for color glyphs such as those illustrated in figure 5.13:

Figure 5.13 Examples of the graphic capabilities of COLR version 1

Version 1 also extends capabilities in variable fonts. A COLR version 0 table can be used in variable fonts with glyph outlines being variable, but no other aspect of the color composition being variable. In version 1, all of the new constructs for which it could be relevant have been designed to be variable; for example, the placement of color stops in a gradient, or the alpha values applied to colors. The graphic capabilities supported in version 0 and in version 1 are described in more detail below.

The COLR table is used in combination with the CPAL table (5.7.12): all color values are specified as entries in color palettes defined in the CPAL table. If the COLR table is present in a font but no CPAL table exists, then the COLR table is ignored.

#### Graphic compositions

The graphic compositions in a color glyph definition use a set of 2D graphic concepts and constructs:

- Shapes (or geometries)

- Fills (or shadings)

- Layering—a z-order—of elements

- Composition and blending modes—different ways that the content of a layer is combined with the content of layers above or below it

- Affine transformations

For both version 0 and version 1, shapes are obtained from glyph outlines in the ‘glyf’, ‘CFF’ or CFF2 table, referenced by glyph ID. Colors used in fills are obtained from the CPAL table.

The simplest color glyphs use just a few of the concepts above: shapes, solid color fills, and layering. This is the set of capabilities provided by version 0 of the COLR table. In version 0, a base glyph record specifies the color glyph for a given base glyph as a sequence of layers. Each layer is specified in a layer record and has a shape (a glyph ID) and a solid color fill (a CPAL palette entry). The filled shapes in the layer stack are composed using only alpha blending.

Figure 5.14 illustrates the version 0 capabilities: three shapes are in a layered stack: a blue square in the bottom layer, an opaque green circle in the next layer, and a red triangle with some transparency in the top layer.

Figure 5.14 Basic graphic capabilities of COLR version 0

The basic concepts also apply to color glyphs defined using the version 1 formats: shapes have fills and can be arranged in layers. But the additional formats of version 1 support much richer capabilities. In a version 1 color glyph, graphic constructs and capabilities are represented primarily in Paint tables, which are linked together in a directed, acyclic graph. Several different Paint formats are defined, each describing a particular type of graphic operation:

- A PaintColrLayers table provides a layering structure used for creating a color glyph from layered elements. A PaintColrLayers table can be used at the root of the graph, providing a base layering structure for the entire color glyph definition. A PaintColrLayers table can also be nested within the graph, providing a set of layers to define some graphic sub-component within the color glyph.

- The PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, and PaintVarSweepGradient tables provide basic fills, using color entries from the CPAL table.

- The PaintGlyph table provides glyph outlines as the basic shapes.

- The PaintTransform and PaintVarTransform tables are used to apply an affine transformation matrix to a sub-graph of paint tables, and the graphic operations they represent. Several Paint formats are also provided for specific transformation types: translate, scale, rotate, or skew, with additional variants of these formats for variations and other options.

- The PaintComposite table supports alternate compositing and blending modes for two sub-graphs.

- The PaintColrGlyph table allows a color glyph definition, referenced by a base glyph ID, to be re-used as a sub-graph within multiple color glyphs.

NOTE	Some paint formats come in Paint* and PaintVar* pairs. In these cases, the latter format supports variations in variable fonts, while the former provides a more compact representation for the same graphic capability but without variation capability.

In a simple color glyph description, a PaintGlyph table might be linked to a PaintSolid table, for example, representing a glyph outline filled using a basic solid color fill. But the PaintGlyph table could instead be linked to a much more complex sub-graph of Paint tables, representing a shape that gets filled using the more-complex set of operations described by the sub-graph of Paint tables.

The graphic capabilities are described in more detail in 5.7.11.1.1 – 5.7.11.1.9. The formats used for each are specified in 5.7.11.2.

##### Colors and solid color fills

All colors are specified as a base zero index into CPAL (5.7.12) palette entries. A font can define alternate palettes in its CPAL table; it is up to the application to determine which palette is used. A palette entry index value of 0xFFFF is a special case indicating that the text foreground color (defined by the application) should be used, and shall not be treated as an actual index into the CPAL ColorRecord array.

The CPAL color data includes alpha information, as well as RGB values. In the COLR version 0 formats, a color reference is made in a LayerRecord as a palette entry index alone. In the formats added for COLR version 1, color references include a palette entry index and a separate alpha value within the COLR structure for a solid color fill or gradient color stop (described below). Separation of alpha from palette entries in version 1 allows use of transparency in a color glyph definition independent of the choice of palette. The alpha value in the COLR structure is multiplied into the alpha value given in the CPAL color entry. If the palette entry index is 0xFFFF, the alpha value in the COLR structure is multiplied into the alpha value of the text foreground color.

In version 1, a solid color fill is specified using a PaintVarSolid or PaintSolid table, with or without variation support, respectively. See 5.7.11.2.6.2 for format details.

See 5.7.11.1.3 for details on how fills are applied to a shape.

##### Gradients

###### General

COLR version 1 supports three types of gradients: linear gradients, radial gradients, and sweep gradients. For each type, non-variable and variable formats are defined. Each type of gradient is specified using a color line.

###### Color Lines

A color line is a function that maps real numbers to color values to define a one-dimensional gradation of colors, to be used in the definition of linear, radial, or sweep gradients. A color line is defined as a set of one or more color stops, each of which maps a particular real number to a specific color.

On its own, a color line has no positioning, orientation or size within a design grid. The definition of a linear, radial, or sweep gradient will reference a color line and map it onto the design grid by specifying positions in the design grid that correspond to the real values 0 and 1 in the color line. The specification for linear, radial and sweep gradients also include rules for where to draw interpolated colors of the color line, following from the placement of 0 and 1.

A color stop is defined by a real number, the stop offset, and a color. Stop offsets are represented using F2DOT14 values, therefore color stops can only be specified within the range [-2, 2). In a variable font, however, stop offsets can be variable, and with the application of deltas can lie outside that range. (See 5.7.11.2.5 for format details.) A color line requires at least one color stop to paint any color values. If the color line does not have any color stops, then transparent black is used for the entire color line. If only one color stop is specified, that color is used for the entire color line. At least two color stops are needed to create color gradation.

Color gradation is defined over the interval from the color stop with the minimum offset, through the successive color stops, to the color stop with the maximum offset. Between numerically-adjacent color stops, color values are linearly interpolated. See Interpolation of Colors in 5.7.12 for requirements on how colors are interpolated.

Color stops shall be used in their stop offset order, which can be different from the order in which they are defined in the font. Also, in a variable font, stops shall be ordered according to offset values after variation deltas are applied. Therefore, the ordering of colors in the color line definition can change between one variation instance and another instance.

Color values outside the defined interval are determined by the color line’s extend mode, described below. In this way, colors are defined for all stop offset values, from negative infinity to positive infinity.

For example, a gradient color line could be defined with two color stops at 0.2 and 1.5. Colors for offsets between 0.2 and 1.5 are interpolated. Colors for offsets above 1.5 and below 0.2 are determined by the color line’s extend mode.

If there are multiple color stops defined for the same stop offset, the first one given in the font shall be used for computing color values on the color line below that stop offset, and the last one shall be used for computing color values at or above that stop offset.  All other color stops for that stop offset shall be ignored. In a variable font, if two color stops have different default offset values but vary to have the same offset value for a given variation instance, the stops shall be applied to the color line in the order in which they are given in the font, as described above.

The color patterns outside the defined interval are determined by the color line’s extend mode. Three extend modes are supported:

- Pad: outside the defined interval, the color of the closest color stop is used. Using a sequence of letters as an analogy, given a sequence “ABC”, it is extended to “…AA ABC CC…”.

- Repeat: The color line is repeated over repeated multiples of the defined interval. For example, if color stops are specified for a defined interval of [0.2, 1.5], then the pattern is repeated above the defined interval for intervals (1.5, 2.8], (2.8, 4.1], etc.; and also repeated below the defined interval for intervals [-1.1, 0.2), [-2.4, -1.1), etc. In each repeated interval, the first color is that of the farthest defined color stop. By analogy, given a sequence “ABC”, it is extended to “…ABC ABC ABC…”.

- Reflect: The color line is repeated over repeated intervals, as for the repeat mode. However, in each repeated interval, the ordering of color stops is the reverse of the adjacent interval. By analogy, given a sequence “ABC”, it is extended to “…ABC CBA ABC CBA ABC…”.

Figures 5.15 – 5.17 illustrate the different color line extend modes. The figures show the color line extended over a limited interval, but the extension is unbounded in either direction.

Figure 5.15 Color gradation extended using pad mode

Figure 5.16 Color gradation extended using repeat mode

Figure 5.17 Color gradation extended using reflect mode

NOTE 1	The extend modes are the same as the spreadMethod attribute used for linear and radial gradients in the Scalable Vector Graphics (SVG) 1.1 (2nd Edition) specification.

If all color stops have the same offset and the extend mode is pad, the color line has two colors: the first stop is used below that offset, and the last stop is used at and above that offset. However, if the extend mode is repeat or reflect, the color line is ill-formed and nothing shall be rendered.

When combining a color line with the geometry of a particular gradient definition, one might want to achieve a certain number of repetitions of the gradient pattern over a particular geometric range. Assuming that geometric range will correspond to placement of stop offsets 0 and 1, the following steps can be used:

- In order to get a certain number of repetitions of the gradient pattern (without reflection), divide 1 by the number of desired repetitions, use the result as the maximum stop offset for specified color stops, and set the extend mode to repeat.

- In order to get a certain number of repetitions of the reflected gradient pattern, divide 1 by two times the number of desired repetitions, use the result as the maximum stop offset for specified color stops, and set the extend mode to reflect.

NOTE 2	Given the above specifications for how color lines are defined, and also allowing for the application of variation deltas, the definition of a color line can span an interval greater than, narrower than, or in other ways different from the interval [0, 1]. However, some application platforms that provide an implementation for gradients can require a color line to be defined for exactly the interval [0, 1]. Implementations can work around such constraints by normalizing the offsets of the defined color stops. For example, if constrained to the range [0, 1], map the lowest stop offset to 0, map the largest offset to 1, and linearly interpolate normalized offsets for other stops in between. Then, for correct alignment to the geometry of a gradient definition, interpolate or extrapolate alternate geometric values to align with the normalized 0 and 1 stop offsets. For radial gradients, special geometry considerations can also apply; see 5.7.11.1.2.4 for more information.

Color lines are specified using color line tables, which contain arrays of color stop records. Two color line table and two color stop record formats are defined:

- ColorLine table and ColorStop record

- VarColorLine table and VarColorStop record

The VarColorLine and VarColorStop formats can be used in variable fonts and allow for stop offsets and color alpha values to be variable. The ColorLine and ColorStop formats provide a more compact representation when variation is not required. See 5.7.11.2.5 for format details.

###### Linear gradients

A linear gradient provides gradation of colors along a straight line. The gradient is defined by three points, p₀, p₁ and p₂, plus a color line. The color line is positioned in the design grid with stop offset 0 aligned to p₀ and stop offset 1.0 aligned to p₁. (The line passing through p₀ and p₁ will be referred to as line p₀p₁.) Colors at each position on line p₀p₁ are interpolated using the color line. For each position along line p₀p₁, the color at that position is projected on either side of the line.

The additional point, p₂, is used to rotate the gradient orientation in the space on either side of the line p₀p₁. The line passing through points p₀ and p₂ (line p₀p₂) determines the direction in which colors are projected on either side of the color line. That is, for each position on line p₀p₁, the line that passes through that position on line p₀p₁ and that is parallel to line p₀p₂ will have the color for that position on line p₀p₁.

NOTE 1	For convenience, point p₂ can be referred to as the rotation point, and the vector from p₀ to p₂ can be referred to as the rotation vector. However, neither the magnitude of the vector nor the direction (from p₀ to p₂, versus from p₂ to p₀) has significance.

If either point p₁ or p₂ is the same as point p₀, the gradient is ill-formed and shall not be rendered.

If line p₀p₂ is parallel to line p₀p₁ (or near-parallel for an implementation-determined definition), then the gradient is ill-formed and shall not be rendered.

NOTE 2	An implementation can derive a single vector, from p₀ to a point p₃, by computing the orthogonal projection of the vector from p₀ to p₁ onto a line perpendicular to line p₀p₂ and passing through p₀ to obtain point p₃. The linear gradient defined using p₀, p₁ and p₂ as described above is functionally equivalent to a linear gradient defined by aligning stop offset 0 to p₀ and aligning stop offset 1.0 to p₃, with each color projecting on either side of that line in a perpendicular direction. This specification uses three points, p₀, p₁ and p₂, as that provides greater flexibility in controlling the placement and rotation of the gradient, as well as variations thereof.

Figures 5.18 – 5.20 illustrate linear gradients using the three different color line extend modes. Each figure illustrates linear gradients with two different rotation vectors. In each case, three color stops are specified: red at 0.0, yellow at 0.5, and blue at 1.0.

Figure 5.18 Linear gradients with different rotations using the pad extend mode

Figure 5.19 Linear gradients with different rotations using the repeat extend mode

Figure 5.20 Linear gradients with different rotations using the reflect extend mode

NOTE 3	When a linear gradient is combined with a transformation (see 5.7.11.1.5), the appearance will be the same as if the gradient were defined using the transformed positions of points p₀, p₁ and p₂.

Linear gradients are specified using a PaintVarLinearGradient or PaintLinearGradient table, with or without variation support, respectively. See 5.7.11.2.6.3 for format details.

See 5.7.11.1.3 for details on how fills are applied to a shape.

Note on implementation of linear gradients

Some applications platform can provide an implementation for linear gradients that requires a color line to be specified for exactly the interval [0, 1]. If an application is using such a platform implementation but the font has a linear gradient with a color line specified over a different interval, the application can work around the platform constraint by normalizing the color line to the interval [0, 1]. This would require deriving alternate geometric points for the linear gradient definition. For the defined points p₀, p₁ and p₂, alternate points p₀′, p₁′ and p₂′ can be derived as follows: if omin is the minimum stop offset in the defined color line, omax is the maximum offset, and vector v = p₁ - p₀, then let

- p₀′ = p₀ + ominv

- p₁′ = p₀ + omaxv

- p₂′ = p₂ + ominv

If an implementation derives a single vector with point p₃, as described above, then alternate points p₀ and p₃ can be derived as follows: with vector w = p₃ - p₀, let

- p₀′ = p₀ + ominw

- p₃′ = p₀ + omaxw

###### Radial gradients

A radial gradient provides gradation of colors along a cylinder defined by two circles. The gradient is defined by circles with center c₀ and radius r₀, and with center c₁ and radius r₁, plus a color line. The color line aligns with the two circles by associating stop offset 0 with the first circle (with center c₀) and aligning stop offset 1.0 with the second circle (with center c₁).

NOTE 1	The term “radial gradient” is used in some contexts for more limited capabilities. In some contexts, the type of gradient defined here is referred to as a “two point conical” gradient.

The drawing algorithm for radial gradients follows the HTML WHATWG Canvas specification for createRadialGradient() [32], but adapted with alternate color line extend modes and more flexibility for color stop offsets. Radial gradients shall be rendered with results that match the results produced by the following steps.

With circle center points c₀ and c₁ defined as c₀ = (x₀, y₀) and c₁ = (x₁, y₁):

- If c₀ = c₁ and r₀ = r₁ then paint nothing and return.

- For real values of ω: Let x(ω) = (x₁-x₀)ω + x₀ Let y(ω) = (y₁-y₀)ω + y₀ Let r(ω) = (r₁-r₀)ω + r₀ Let the color at ω be the color at position ω on the color line.

- For all values of ω where r(ω) > 0, starting with the value of ω nearest to positive infinity and ending with the value of ω nearest to negative infinity, draw the circular line with radius r(ω) centered at position (x(ω), y(ω)), with the color at ω, but only painting on the parts of the bitmap that have not yet been painted on in this step of the algorithm for earlier values of ω.

The algorithm provides results in various cases as follows:

- When the circles are identical, then nothing is painted.

- When both radii are 0 (r₀ = r₁ = 0), then r(ω) is always 0 and nothing is painted.

- If the centers of the circles are distinct, the radii of the circles are different, and neither circle is entirely contained within the radius of the other circle, then the resulting shape resembles a cone that is open to one side. The surface outside the cone is not painted. (See figures 5.21 – 5.23.)

- If the centers of the circles are distinct but the radii are the same, and neither circle is contained within the other, then the result will be a strip, similar to the flattened projection of a circular cylinder. The surface outside the strip is not painted. (See figures 5.24 – 5.26.)

- If the radii of the circles are different but one circle is entirely contained within the radius of the other circle, the gradient will radiate in all directions from the inner circle, and the entire surface will be painted. (See figures 5.30 – 5.32.)

Figures 5.21 – 5.23 illustrate radial gradients using the three different color line extend modes. The color line is defined with stops for the interval [0, 1]: red at 0.0, yellow at 0.5, and blue at 1.0. Note that the circles that define the gradient are not stroked as part of the gradient itself. Stroked circles have been overlaid in the figure to illustrate the color line and the region that is painted in relation to the two circles.

Figure 5.21 Radial gradient using pad extend mode.

Figure 5.22 Radial gradient using repeat extend mode.

Figure 5.23 Radial gradient using reflect extend mode.

Figures 5.24 – 5.26 illustrate the case in which the circles have distinct centers but the same radii, and neither circle is contained within the other, giving the appearance of a strip. The color stops are as in the previous figures.

Figure 5.24 Radial gradient with same-size circles appearing as a strip, using pad extend mode.

Figure 5.25 Radial gradient with same-size circles appearing as a strip, using repeat extend mode.

Figure 5.26 Radial gradient with same-size circles appearing as a strip, using reflect extend mode.

Because the rendering algorithm progresses ω in a particular direction, from positive infinity to negative infinity, and because pixels are not re-painted as ω progresses, the appearance will be affected by which circle is considered circle 0 and which is circle 1. This is illustrated in figures 5.27 – 5.29. The gradient in figure 5.27 is the same as that in figure 5.21, using the pad extend mode. In this gradient, circle 0 is the small circle, on the left. In figure 5.28, the start and end circles are reversed: circle 0 is the large circle, on the right. The color line is kept the same, and so the red end starts at circle 0, now on the right. In figure 5.29, the order of stops in the color line is also reversed to put red on the left. The key difference to notice between the gradients in these figures is the way that colors are painted in the interior: when the two circles are not overlapping, the arcs of constant color bend in the same direction as the near side of circle 1.

NOTE 2	This difference does not exist if one circle is entirely contained within the other: in that case, the arcs of constant color are complete circles.

Figure 5.27 Cone-shaped radial gradient with circle 0 on the left.

Figure 5.28 Cone-shaped radial gradient with start and end circles swapped.

Figure 5.29 Cone-shaped radial gradient with start and end circles swapped and color line reversed.

When one circle is contained within the other, the extension of the gradient beyond the larger circle will fill the entire surface. Colors in the areas inside the inner circle and outside the outer circle are determined by the extend mode. Figures 5.30 – 5.32 illustrate this for the different extend modes.

Figure 5.30 Radial gradient with one circle contained within the other, pad extend mode.

Figure 5.31 Radial gradient with one circle contained within the other, repeat extend mode.

Figure 5.32 Radial gradient with one circle contained within the other, reflect extend mode.

NOTE 3	When a radial gradient is combined with a transformation (see 5.7.11.1.5), the appearance will be the same as if the geometry of the two circles were transformed and step 3 of the algorithm were performed by interpolating the shapes derived from the two transformed circles. For the condition r(ω) > 0, the pre-transformation values of r(ω) can be used.

NOTE 4	A scale transformation can flatten shapes to resemble lines. If a radial gradient is nested in the child sub-graph of a transformation that flattens the circles so that they are nearly lines, the centers could still be separated by some distance. In that case, the radial gradient would appear as a strip or a cone filled with a linear gradient.

If a radial gradient is nested in the sub-graph of a transformation that flattens the circles so that they form a single line (or nearly a line, for an implementation-determined definition), with both centers on that line, then the resulting gradient is degenerate and shall not be rendered.

NOTE 5	As seen in the figures above, the gradient fills the space when one circle is contained within the other, but not when neither circle is contained within the other. In a variable font, if the placement or radii of the circles vary, then a sharp transition can occur if the variation results in one circle being contained within the other for some instances but not for other instances. This transition will occur when the inner circle just touches the outer circle (i.e., they have exactly one point in common). In this case, the gradient will fill exactly one half of the space. This is illustrated in figure 5.33 using the pad extend mode.

Figure 5.33 Radial gradient with inner circle just touching the outer circle, pad extend mode

When the repeat or reflect extend modes are used, having the two circles in very close proximity results in very high spatial-frequency transitions that can lead to Moiré patterns or other display artifacts. This is illustrated in figure 5.34, which shows the display result, for one particular rendering context, of a radial gradient defined using nearly-identical circles and the reflect extend mode.

Figure 5.34 Radial gradient defined using nearly-identical circles, showing interference patterns

The artifacts seen can be affected by a combination of several factors, such as image scaling, sub-pixel rendering, display technology, and limitations in software implementation or display capabilities. For this reason, the appearance can be very different in different situations. Font designers should exercise caution if the circles are in close proximity (either in a static design or for some variable font instances), and should not rely on these display artifacts to obtain a particular pattern.

While the Paint table formats use unsigned values to define the circle radii, application of deltas in variable fonts can result in negative radii values. While a negative radius does not seem to make sense intuitively, negative radii are not a problem for the rendering algorithm described above: given the formula r(ω) = (r₁-r₀)ω + r₀, r(ω) will have positive and negative values for different values of ω when the values of r₀ and r₁ are both positive, but also if either r₀ or r₁, or both, are negative. For example, if r₀ = -1 and r₁ = -2, then r(ω) = ω - 2, and r(ω) ≥ 0 for ω ≥ 2. There is one notable change, however, in the case of r₀ = r₁ (hence, r(ω) = r₀): if the values are positive, a cylinder is painted (since r₀ > 0), but if the values are negative, then nothing is painted (since r₀ < 0).

NOTE 6	If r₀ is different from r₁, the geometry defined by the two circles can be understood as a 2D projection of a cone. The cone has a tip where r(ω) = 0, and the cone is painted on the side of the tip for which r(ω) ≥ 0. In the general case,
•	ωtip = r₀/(r₀ - r₁)
•	if r₁ > r₀, r(ω) > 0 for all ω > r₀/(r₀ - r₁)
•	if r₁ < r₀, r(ω) > 0 for all ω < r₀/(r₀ - r₁)

Radial gradients are specified using a PaintVarRadialGradient or PaintRadialGradient table, with or without variation support, respectively. See 5.7.11.2.6.4 for format details.

See 5.7.11.1.3 for details on how fills are applied to a shape.

Note on implementation of radial gradients

Some application platforms can provide an implementation for radial gradients that require the circle radii to be non-negative. If an application is using such a platform implementation but a variable font instance has a radial gradient with a negative circle radius, the application can work around the platform constraint by deriving an alternate gradient definition that uses only circles with non-negative radii and that produces the same visual appearance. This would also require deriving an alternate color line definition that preserves the same color line extend mode behaviors.

Similarly, if the platform provides an implementation for radial gradients that requires color stops to be defined for exactly the interval [0, 1] but the font has a radial gradient with a color line specified over a different interval, the application can work around that platform constraint by normalizing the color line to the interval [0, 1]. But the normalized color line would require deriving alternate circle specifications, and a simple normalization can result in a circle with a negative radius.

Thus, handling of negative circle radii and normalization of the color line need to be considered together. The following observations will be helpful:

- If r₀ and r₁ are positive and r₀ = r₁, a simple normalization of the color line can be used.

- If the color stops that define the color line all have offsets ω such that r(ω) ≥ 0, then a simple normalization of the color line can be used with alternate circles that have non-negative radii.

- For pad extend mode, if some color stops have offsets ω such that r(ω) < 0, the same visual result can be obtained by using a truncated color line definition in which all stops with offsets ω such that r(ω) < 0 are replaced by a single stop for ωtip with an interpolated color value. (See Interpolation of Colors in 5.7.12 for requirements on how colors are interpolated.)

- For pad and repeat extend modes, the extended color line has a repeating pattern with offset wavelength λ, and any color line definition that is a slice of the color line of length λ will produce the same visual results. Thus, the application can derive an alternate color line definition in which all stops have offsets ω such that r(ω) ≥ 0, normalize those stop offsets to [0, 1], then derive alternate circles that align to the normalized 0 and 1 offsets.

###### Sweep gradients

A sweep gradient provides a gradation of colors that sweep around a center point. For a given color on a color line, that color projects as a ray from the center point in a given direction. This is illustrated in figure 5.35.

NOTE 1	The following figures illustrate sweep gradients clipped to a region. Sweep gradients are not bounded, however, and fill the entire space.

Figure 5.35 A sweep gradient

NOTE 2	In some contexts, this type of gradient is referred to as a “conic” gradient, or as an “angular” gradient.

A sweep gradient is defined by a center point, start and end angles, and a color line. The angles are expressed in counter-clockwise degrees from the direction of the positive x-axis on the design grid.

For sampling and interpolating colors, the color line is aligned to an infinitely spiraling circular arc around the center point, with arbitrary radius. The position 0° on the arc means the direction of the positive x-axis, and 360° means one full counter-clockwise rotation. The ColorLine’s stop offset 0 is aligned with the start angle, and stop offset 1 is aligned with the end angle, independent of which angle is larger.

Outside the defined interval of the color line, the color value of a position on the color line is filled in depending on its extend mode. See 5.7.11.1.2.1 Color Lines for more details. In effect, this means that the spiraling circular arc can be sampled for colors outside the defined color line interval.

To draw the sweep gradient, for each position along the circular arc starting from 0° up to but not including 360°, a ray from the center outward is painted with the color of the color line where the ray intersects the circular arc at that particular angle. Angle positions below 0° and equal to or above 360° are not sampled for drawing the rays.

If the color line’s extend mode is reflect or repeat and start and end angle are equal, nothing shall be drawn.

NOTE 3	When the sweep gradient’s color line uses the repeat or reflect extend mode and the angular distance between start and end angle is small, this results in very high spatial-frequency transitions that can lead to Moiré patterns or other display artifacts. (See Figure 5.34 where this effect is shown for a similar case involving radial gradients).

Figure 5.36 illustrates a sweep gradient with the drawing direction progressing from 0° to 360°. The gradient is specified with a start angle of 110° and end angle of 230°. The color line is specified with a yellow stop at offset 0 (aligned to the start angle) and a red stop at offset 1 (aligned to the end angle). The pad extend mode is used, hence the color for angles below 110° is yellow, and the color for angles above 150° is red:

Figure 5.36 A sweep gradient with start angle of 110° and an end angle of 230°, using a color line with color stops 0 and 1 for yellow and red and pad extend mode

If the start angle is less than or equal to the end angle, the color line progresses from smaller to larger offsets in the counter-clockwise direction along the circular arc. If the start angle is larger than the end angle, however, the color line is inverted and progresses in the clockwise direction, as illustrated in figure 5.37.

Figure 5.37 A sweep gradient, from yellow to red, with start angle of 210° and end angle of 110°, showing the inversion of the color line when the start angle is larger than the end angle

Not more than one full rotation is drawn and there is no overlap in drawing for angles outside the 0° and 360° range. However, start and end angles may be positioned at angles below 0° or above 360°. Through that, and through how wide the color line interval is defined, color stops can lie outside the 0° to 360° circle. This has an effect on the computation of the gradient colors inside the interval of 0° to 360°, but colors are not sampled from outside this interval. See figure 5.38 for an example: The color line goes from yellow to red with a start angle of 330° and an end angle of 400°. The color for angles lower than 330° is yellow due to the pad extend mode, then the color line transitions from yellow at 330° to red at 400°, but only the color values up until 360° are sampled for drawing.

Figure 5.38 A sweep gradient with start angle of 330° and an end angle of 400°, using a color line with color stops 0 and 1 for yellow and red and extend mode pad

NOTE 4	Because the sweep gradient is drawn from 0° to 360° a sharp transition can occur at 0°. This can be mitigated by adjusting the color stops at the 0° and 360° positions on the arc to have the same color. The location of the transition axis can also be shifted by nesting the sweep gradient inside a rotation transformation (see 5.7.11.1.5).

NOTE 5	When a sweep gradient is combined with a transformation (see 5.7.11.1.5), the appearance will be the same as if (i) a circle of some non-zero radius was constructed with points on the circle computed from the start and end angles; (ii) the center point, circle and points on the circle were transformed; (iii) the color line was aligned to the transformed circle; and then (iv) a gradient was derived from the result, with rays from the transformed center point passing through the transformed color circle. When aligning the color line to the transformed circle, stop offset 0 would be aligned to the transformed point derived from the start angle, with stop offset 1 aligned to the transformed point derived from the end angle. Thus, a transform can result in the color line progressing in the opposite direction compared to the non-transformed gradient.

Sweep gradients are specified using a PaintVarSweepGradient or PaintSweepGradient table, with or without variation support, respectively. See 5.7.11.2.6.5 for format details.

See 5.7.11.1.3 for details on how fills are applied to a shape.

##### Filling shapes

All basic shapes used in a color glyph are obtained from glyph outlines, referenced using a glyph ID. In a color glyph description, a PaintGlyph table is used to represent a basic shape.

NOTE	Shapes can also be derived using PaintGlyph tables in combination with other tables, such as PaintTransform (see 5.7.11.1.5) or PaintComposite (see 5.7.11.1.6).

The PaintGlyph table has a field for the glyph ID, plus an offset to a child paint table that is used as the fill for the shape. The glyph outline is not rendered; only the fill is rendered.

Any of the basic fill formats (PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, PaintVarSweepGradient) can be used as the child paint table. This is illustrated in figure 5.39: a PaintGlyph table has a glyph ID for an outline in the shape of a triangle, and it links to a child PaintLinearGradient table. The combination is used to represent a triangle filled with the linear gradient.

Figure 5.39 PaintGlyph and PaintLinearGradient tables used to fill a shape with a linear gradient.

The child of a PaintGlyph table is not, however, limited to one of the basic fill formats. Rather, the child can be the root of a sub-graph that describes some graphic composition that is used as a fill. Another way to describe the relationship between a PaintGlyph table and its child sub-graph is that the glyph outline specified by the PaintGlyph table defines a bounds, or clip region, that is applied to the fill composition defined by the child sub-graph.

To illustrate this, the example in figure 5.39 is extended in figure 5.40 so that a PaintGlyph table links to a second PaintGlyph that links to a PaintLinearGradient: the parent PaintGlyph will clip the filled shape described by the child sub-graph.

Figure 5.40 A PaintGlyph table defines a clip region for the composition defined by its child sub-graph.

A PaintGlyph table on its own does not add content: if there is no child paint table, then the graph is not well formed. See 5.7.11.1.9 for details regarding well-formedness and validity of the graph.

##### Layering

Layering of visual elements was introduced above, in the introduction to 5.7.11.1. Both version 0 and version 1 support use of multiple layers, though in different ways.

For version 0, layers are fundamental: they are the sole way in which separate elements are composed into a color glyph. An array of LayerRecords is created, with each LayerRecord specifying a glyph ID and a CPAL entry (a shape and solid color fill). Each color glyph definition is a slice from that array (that is, a contiguous sub-sequence), specified in a BaseGlyphRecord for a particular base glyph. Within a given slice, the first record specifies the content of the bottom layer, and each subsequent record specifies content that overlays the preceding content (increasing z-order). A single array is used for defining all color glyphs. The LayerRecord slices for two base glyphs may overlap, though often will not overlap.

Figure 5.41 illustrates layers using version 0 formats.

Figure 5.41 Version 0: Color glyphs are defined by slices of a layer records array.

When using version 1 formats, use of multiple layers is supported but is optional. For example, a simple glyph description need not use any layering, as illustrated in figure 5.42:

Figure 5.42 Complete color glyph definition without use of layers.

The version 1 formats define a color glyph as a directed, acyclic graph of paint tables, and the concept of layering corresponds roughly to the number of distinct leaf nodes in the graph. (See 5.7.11.1.9.) The basic fill formats (PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, PaintVarSweepGradient) do not have child paint tables and so can only be leaf nodes in the graph. Some paint tables, such as the PaintGlyph table, have only a single child, so can be used within a layer but do not provide any means of adding additional layers. Increasing the number of layers requires paint tables that have two or more children, creating a fork in the graph.

The version 1 formats include two paint formats that have two or more children, and so can increase the number of layers in the graph:

- The PaintComposite table allows two sub-graphs to be composed together using different compositing or blending modes.

- The PaintColrLayers table supports defining a sequence of several layers.

NOTE 1	The PaintColrGlyph table provides a means of incorporating the graph of one color glyph as a sub-graph in the definition of another color glyph. In this way, PaintColrGlyph provides an indirect means of introducing additional layers into a color glyph definition: forks in the resulting graph do not come from the PaintColrGlyph table itself, but can come from PaintColrLayers or PaintComposite tables that are nested in the incorporated sub-graph. See 5.7.11.1.7.4 for a description of the PaintColrGlyph table.

While the PaintComposite table only combines two sub-graphs, other PaintComposite tables can be nested to provide additional layers. The primary purpose of PaintComposite is to support compositing or blending modes other than simple alpha blending. The PaintComposite table is covered in more detail in 5.7.11.1.6. The remainder of this clause will focus on the PaintColrLayers table.

The PaintColrLayers table is used to define a bottom-up z-order sequence of layers. Similar to version 0, it defines a layer set as a slice in an array, but in this case the array is an array of offsets to paint tables, contained in a LayerList table. Each referenced paint table is the root of a sub-graph of paint tables that specifies a graphic composition to be used as a layer. Within a given slice, the first offset provides the content for the bottom layer, and each subsequent offset provides content that overlays the preceding content. Definition of a layer set—a slice within the layer list—is given in a PaintColrLayers table.

Figure 5.43 illustrates the organizational relationship between PaintColrLayers tables, the LayerList, and referenced paint tables that are roots of sub-graphs.

Figure 5.43 Version 1: PaintColrLayers tables specify slices within the LayerList, providing a layering of content defined in sub-graphs.

NOTE 2	Paint table offsets in the LayerList table are only used in conjunction with PaintColrLayers tables. If a paint table does not need to be referenced via a PaintColrLayers table, its offset does not need to be included in the LayerList array.

A PaintColrLayers table can be used as the root of a color glyph definition, providing a base layering structure for the color glyph. In this usage, the PaintColrLayers table is referenced by a BaseGlyphPaintRecord, which specifies the root of the graph of a color glyph definition for a given base glyph. This is illustrated in figure 5.44.

Figure 5.44 PaintColrLayers table used as the root of a color glyph definition.

A PaintColrLayers table can also be nested more deeply within the graph, providing a layer structure to define some component within a larger color glyph definition. (See 5.7.11.1.7.3 for more information.) The ability to nest a PaintColrLayers table within a graph creates the potential to introduce a cycle within the graph, which would be invalid (see 5.7.11.1.9).

##### Transformations

A 2 × 3 transformation matrix can be used within a color glyph description to apply an affine transformation to a sub-graph. Affine transformations supported by a matrix can be a combination of scale, skew, mirror, rotate, or translate. The transformation is applied to all nested paints in the child sub-graph.

A transformation matrix is specified using a PaintVarTransform or PaintTransform table, with or without variation support, respectively. See 5.7.11.2.6.8 for format details.

The effect of a transformation is illustrated in figure 5.45: a PaintTransform table is used to specify a rotation, and both the glyph outline and gradient in the sub-graph are rotated.

Figure 5.45 A rotation transformation rotates the fill content defined by the child sub-graph.

If the sub-graph of a transformation table contains another nested transformation table, then the second transformation also applies to its child sub-graph. For the sub-sub-graph, the two transformations are combined. To illustrate this, the example in figure 5.45 is extended in figure 5.46 by inserting a mirroring transformation between the PaintGlyph and PaintLinearGradient tables: the glyph outline is rotated as before, but the gradient is mirrored in its (pre-rotation) y-axis as well as being rotated. Notice that both visible elements—the shape and the gradient fill—are affected by the rotation, but only the gradient is affected by the mirroring.

Figure 5.46 Combined effects of a transformation nested within the child sub-graph of another transformation.

While the PaintTransform and PaintVarTransform tables support several types of transforms, additional paint formats are defined to support specific transformations:

- PaintTranslate and PaintVarTranslate support translation only, without or with variation support, respectively. See 5.7.11.2.6.9 for format details.

- PaintScale and PaintVarScale support scaling only, without or with variation support. These two formats scale relative to the origin, and allow for different scale factors in X and Y directions. PaintScaleAroundCenter, PaintVarScaleAroundCenter, PaintScaleUniform, PaintVarScaleUniform, PaintScaleUniformAroundCenter, and PaintVarScaleUniformAroundCenter support scaling relative to a different center, scaling uniformly in both X and Y directions, or both. See 5.7.11.2.6.10 for format details.

- PaintRotate and PaintVarRotate support rotation only, without or with variation support. These two formats rotate around the origin; the PaintRotateAroundCenter and PaintVarRotateAroundCenter formats support rotation around a different center. See 5.7.11.2.6.11 for format details.

- PaintSkew and PaintVarSkew support skew only, without or with variation support. These two formats skew using the origin as a center for the skew rotation; the PaintSkewAroundCenter and PaintVarSkewAroundCenter formats support skews using a different center. See 5.7.11.2.6.12 for format details.

NOTE 1	Horizontal mirroring is done by scaling using a scale factor in the x direction of -1. Vertical mirroring is done by scaling with a -1 scale factor in the y direction.

When only one of these specific types of transformation is required, these formats provide a more compact representation than the PaintTransform or PaintVarTransform formats. Another significant difference of the rotation and skew formats is that the rotations and skews are specified as angles, in counter-clockwise degrees.

NOTE 2	Specifying the rotation or skew as an angle can have a significant benefit in variable fonts if an angle of skew or rotation needs to vary, since it is easier to implement variation of angles when specified directly rather than as matrix elements. This is because the matrix elements for a rotation or skew are the sine, cosine or tangent of the rotation angle, which do not change in linear proportion to the angle. To achieve a linear variation of rotation using matrix elements would require approximating the variation using multiple delta sets.

The rotations and skews specified using PaintRotate, PaintSkew and their variants can also be represented as a matrix using a PaintTransform or PaintVarTransform table. The behavior for the PaintRotate or PaintSkew formats and their variants shall be the same as if the rotation or skew were represented using an equivalent matrix. See 5.7.11.2.6.11 for details regarding the matrix equivalent for a rotation expressed as an angle; and see 5.7.11.2.6.12 for similar details in relation to skews.

##### Compositing and blending

When a color glyph has overlapping content in two layers, the pixels in the two layers must be combined in some way. If the content in the top layer has full opacity, then normally the pixels from that layer are shown, occluding overlapping pixels from lower layers. If the top layer has some transparency (some portion has alpha less than 1.0), then blending of colors for overlapping pixels occurs by default. The default interaction between layers uses simple alpha compositing, as described in Compositing and Blending Level 1 [33].

A PaintComposite table can be used to get other compositing or blending effects. The PaintComposite table combines content defined by two sub-graphs: a source sub-graph; and a destination, or backdrop, sub-graph. First, the paint operations for the backdrop sub-graph are executed, then the drawing operations for the source sub-graph are executed and combined with backdrop using a specified compositing or blending mode. The available modes are given in the CompositeModes enumeration (see 5.7.11.2.6.13). The effect and processing rule of each mode are specified in Compositing and Blending Level 1 [33].

The available modes fall into two general types: compositing modes, also referred to as “Porter-Duff” modes; and blending modes. In rough terms, the Porter-Duff modes determine how much effect pixels from the source and the backdrop each contribute in the result, while blending modes determine how color values for pixels from the source and backdrop are combined. These are illustrated with examples in figures 5.47 and 5.48: in each case, red and blue rectangles are the source and backdrop content.

Figure 5.47 shows the effect of a Porter-Duff mode, XOR, which has the effect that only non-overlapping pixels contribute to the result.

Figure 5.47 Two content elements combined using the Porter-Duff XOR mode.

Figure 5.48 shows the effect of a lighten blending mode, which has the effect that the R, G, and B color components for each pixel in the result is the greater of the R, G, and B values from corresponding pixels in the source and backdrop.

Figure 5.48 Two content elements combined using the lighten blending mode.

For complete details on each of the Porter-Duff and blending modes, see the Compositing and Blending Level 1 specification [33].

Figure 5.49 illustrates how the PaintComposite table is used in combination with content sub-graphs to implement an alternate compositing effect. The source sub-graph defines a green capital A; the backdrop sub-graph defines a black circle. The compositing mode used is Source Out, which has the effect that the source content punches out a hole in the backdrop. (For this mode, the fill color of the source is irrelevant; a black or yellow ″A″ would have the same effect.) A red rectangle is included as a lower layer to show that the backdrop has been punched out by the source, making that portion of the lower layer visible.

Figure 5.49 A color glyph using a PaintComposite table to punch out a shape from the fill of a circle.

NOTE	In figure 5.49, the ″A″ is filled with green to illustrate that the color of the fill has no affect for the Source Out composite mode. Because that is the case, the black or red PaintSolid could have been re-used instead of adding a separate PaintSolid table. See 5.7.11.1.7.2 for more information on re-use of paint tables for such situations.

Scalable Vector Graphics (SVG) supports alpha channel masking using the <mask> element. The same effects can be implemented in COLR version 1 using a PaintComposite table by setting a pattern of alpha values in the source sub-graph and selecting the Source In composite mode. This is illustrated in figure 5.50.

Figure 5.50 An alpha mask implemented using a PaintComposite table and the Source In mode.

##### Re-usable components

###### Overview

Within a color font, many color glyphs might share components in common. For example, in emoji fonts, many different “smilies” or clock faces share a common background. This can be seen in figure 5.51, which shows color glyphs for three emoji clock faces.

Figure 5.51 Emoji clock faces for 12 o’clock, 1 o’clock and 2 o’clock.

Several components are shared between these color glyphs: the entire face, with a gradient background and dots at the 3, 6, 9 and 12 positions; the minute hand pointing to the 12 position; and the circles in the center. Also, note that the four dots have the same shape and fill, and differ only in their position. In addition, the hour hands have the same shape and fill, and differ only in their orientation.

There are several ways in which elements of a color glyph description can be re-used:

- Reference to shared subtables

- Use of a PaintColrLayers table

- Use of a PaintColrGlyph table

The PaintColrLayers and PaintColrGlyph table formats create a potential for introducing cycles within the graph of a color glyph, which would be invalid (see 5.7.11.1.9).

###### Re-use by referencing shared subtables

Several of the paint table formats link to a child paint table using a forward offset within the file:

- PaintGlyph

- PaintComposite

- PaintTransform, PaintVarTransform

- PaintTranslate, PaintVarTranslate

- PaintScale, PaintVarScale, and the other variant scaling paint formats

- PaintRotate, PaintVarRotate, PaintRotateAroundCenter, PaintVarRotateAroundCenter

- PaintSkew, PaintVarSkew, PaintSkewAroundCenter, PaintVarSkewAroundCenter

A child subtable can be shared by several tables of these formats. For example, several PaintGlyph tables might link to the same PaintSolid table, or to the same node for a sub-graph describing a more complex fill. The only limitation is that child paint tables are referenced using a forward offset from the start of the referencing table, so a re-used paint table can only occur later in the file than any of the paint tables that use it.

The clock faces shown in figure 5.51 provide an example of how PaintRotate tables can be combined with re-use of a sub-graph. As noted above, the hour hands have the same shape and fill, but have a different orientation. The glyph outline could point to the 12 position, then in color glyph descriptions for other times, PaintRotate tables could link to the same glyph/fill sub-graph, re-using that component but rotated as needed.

This is illustrated in the figures 5.52 and 5.53. Figure 5.52 shows a sub-graph defining the hour hand, with upright orientation, using a PaintGlyph and a PaintSolid table. Example file offsets for the tables are indicated.

Figure 5.52 A PaintGlyph and PaintSolid table are used to define the clock hour hand pointing to 12.

Figure 5.53 shows this sub-graph of paint tables being re-used, in some cases linked from PaintRotate tables that rotate the hour hand to point to different clock positions as needed. All of the paint tables that reference this sub-graph occur earlier in the file.

Figure 5.53 The sub-graph for the hour hand is re-used with PaintRotate tables to point to different hours.

###### Re-use using PaintColrLayers

As described above (see 5.7.11.1.4), a PaintColrLayers table defines a set of paint sub-graphs arranged in bottom-up z-order layers, and an example was given of a PaintColrLayers table used as the root of a color glyph definition. A PaintColrLayers table can also be nested more deeply within the graph of a color glyph. One purpose for doing this is to reference a re-usable component defined as a contiguous set of layers in the LayerList table.

This is readily explained using the clock faces as an example. As described above, each clock face shares several elements in common. Some of these form a contiguous set of layers. Suppose four sub-graphs for shared clock face elements are given in the LayerList as contiguous layers, as shown in figure 5.54. (For brevity, the visual result for each sub-graph is shown, but not the paint details.)

Figure 5.54 Common clock face elements given as a slice within the LayerList table.

A PaintColrLayers table can reference any contiguous slice of layers in the LayerList table. Thus, the set of layers shown in figure 5.54 can be referenced by PaintColrLayers tables anywhere in the graph of any color glyph. In this way, this set of layers can be re-used in multiple clock face color glyph definitions.

This is illustrated in figure 5.55: The color glyph definition for the one o’clock emoji has a PaintColrLayers table as its root, referencing a slice of three layers in the LayerList table. The upper two layers are the hour hand, which is specific to this color glyph; and the cap over the pivot for the minute and hour hands, which is common to other clock emoji but in a layer that is not contiguous with other common layers. The bottom layer of these three layers is the composition for all the remaining common layers. It is represented using a nested PaintColrLayers table that references the slice within the LayerList for the common clock face elements shown in figure 5.54.

Figure 5.55 A PaintColrLayers table is used to reference a set of layers that define a shared clock face composition.

The color glyphs for other clock face emoji could be structured in exactly the same way, using a nested PaintColrLayers table to re-use the layer composition of the common clock face elements.

###### Re-use using PaintColrGlyph

A third way to re-use components in color glyph definitions is to use a nested PaintColrGlyph table. This format references a base glyph ID, which is used to access a corresponding BaseGlyphPaintRecord. That record will provide the offset of a paint table that is the root of a graph for a color glyph definition. That graph can potentially be used as an independent color glyph, but it can also define a shared composition that gets re-used in multiple color glyphs. Each time the shared composition is to be re-used, it is referenced by its base glyph ID using a PaintColrGlyph table. The graph of the referenced color glyph is thereby incorporated into the graph of the PaintColrGlyph table as its child sub-graph.

When a PaintColrGlyph table is used, a BaseGlyphPaintRecord with the specified glyph ID is expected. If no BaseGlyphPaintRecord with that glyph ID is found, the color glyph is not well formed. See 5.7.11.1.9 for details regarding well-formedness and validity of the graph.

The example from 5.7.11.1.7.3 is modified to illustrate use of a PaintColrGlyph table. In figure 5.56, a PaintColrLayers table references a slice within the LayerList that defines the shared component. Now, however, this PaintColrLayers table is treated as the root of a color glyph definition for base glyph ID 63163. The color glyph for the one o’clock emoji is defined with three layers, as before, but now the bottom layer uses a PaintColrGlyph table that references the color glyph definition for glyph ID 63163.

Figure 5.56 A PaintColrGlyph table is used to reference the shared clock face composition via a glyph ID.

While the PaintColrGlyph and PaintColrLayers tables are similar in being able to reference a layer set as a re-usable component, they could be handled differently in implementations. In particular, an implementation could process and cache the result of the color glyph description for a given base glyph ID. In that case, subsequent references to that base glyph ID using a PaintColrGlyph table would not require the corresponding graph of paint tables to be re-processed. As a result, using a PaintColrGlyph for re-used graphic components could provide performance benefits.

##### Glyph metrics and boundedness

###### Metrics for color glyphs using version 0 formats

For color glyphs using version 0 formats, the advance width of glyphs used for each layer shall be the same as the advance width of the base glyph. If the font has vertical metrics, the glyphs used for each layer shall also have the same advance height and vertical Y origin as the base glyph.

###### Metrics and boundedness of color glyphs using version 1 formats

For color glyphs using version 1 formats, the advance width of the base glyph shall be used as the advance width for the color glyph. If the font has vertical metrics, the advance height and vertical Y origin of the base glyph shall be used for the color glyph. The advance width and height of glyphs referenced by PaintGlyph tables are not required to be the same as that of the base glyph and are ignored.

A valid color glyph definition shall define a bounded region—that is, it shall paint within a region for which a finite bounding box could be defined. A clip box can be specified to set overall bounds for a color glyph (see below). Otherwise, boundedness is determined by the graph of paint tables that describe the color glyph content. The different paint formats have different boundedness characteristics:

- PaintGlyph is inherently bounded.

- PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, and PaintVarSweepGradient are inherently unbounded.

- PaintColrLayers is bounded if and only if all referenced sub-graphs are bounded.

- PaintColrGlyph is bounded if and only if the color glyph definition for the referenced base glyph ID is bounded.

- Paint formats for transformations (PaintTransform, PaintVarTransform, PaintTranslate, PaintScale, etc.)  are bounded if and only if the referenced sub-graph is bounded.

- PaintComposite is either bounded or unbounded, according to the composite mode used and the boundedness of the referenced sub-graphs. See 5.7.11.2.6.13 for details.

A ClipBox table (5.7.11.2.4) may be associated with a color glyph to define overall bounds for the color glyph. The clip box may vary in a variable font. If a clip box is provided for a color glyph, the color glyph is bounded, and no inspection of the Paint graph is required to determine boundedness. If no clip box is defined for a color glyph, however, applications shall confirm that the color glyph definition is bounded, and shall not render the color glyph if the defining graph is not bounded.

NOTE 1	If present, the clip box for a color glyph can be used to allocate a drawing surface without needing to traverse the graph of the color glyph definition.

NOTE 2	If no ClipBox table is present but a bounding box is required by the implementation, it can be computed for a given color glyph by traversing the graph of Paint tables that defines that color glyph.

To ensure that rendering implementations do not clip any part of a color glyph, the clip box needs to be large enough to encompass the entire color glyph composition. In a variable font, glyph outlines can vary, but transformations in a color glyph description can also vary, affecting the portions of the design grid to be painted. For example, a filled rectangle that is wide but not tall for one variation instance can be variably rotated to be tall but not wide for other instances. The clip box either should be large enough to encompass the color glyph for all instances, or should itself vary such that each instance of the clip box encompasses the instance color glyph.

##### Color glyphs as a directed acyclic graph

When using version 1 formats, a color glyph is defined by a directed, acyclic graph of linked paint tables. For each BaseGlyphPaintRecord, the paint table referenced by that record is the root of a graph defining a color glyph composition.

The graph for a given color glyph is made up of all paint tables reachable from the BaseGlyphPaintRecord. The BaseGlyphPaintRecord and several paint table formats use direct links; that is, they include a forward offset to a paint subtable. Two paint formats make indirect links:

- A PaintColrLayers table references a slice of offsets within the LayerList. The paint tables referenced by those offsets are considered to be linked within the graph as children of the PaintColrLayers table.

- A PaintColrGlyph table references a base glyph ID, for which a corresponding BaseGlyphPaintRecord is expected. That record points to the root of a graph that is a complete color glyph definition on its own. But when referenced in this way by a PaintColrGlyph table, that entire graph is considered to be a child sub-graph of the PaintColrGlyph table, and a continuation of the graph of which the PaintColrGlyph table is a part.

The graph for a color glyph is a combination of paint tables using any of the paint table formats. The simplest color glyph definition would consist of a PaintGlyph table linked to a basic fill table (PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, PaintVarSweepGradient). But the graph can be arbitrarily complex, with an arbitrary depth of paint nodes (to the limits inherent in the formats).

The graph can define a visual element in a single layer, or many elements in many layers. The concept of layers, as distinct visual elements stacked in a z-order, is not precisely defined in relation to the complexity of the graph. Each separate visual element requires a leaf node, but nodes in the graph, including leaf nodes, can be re-used (see 5.7.11.1.7). Also, each separate visual element requires a fork in the graph, and a separate root-to-leaf path, but not all paths necessarily result in a distinct visual element. For example, a gradient mask effect can be created with a gradient with gradation of alpha values, and then using that as the source of a PaintComposite table with the Source In compositing mode. In that case, the leaf has a visual affect but does not result in a distinct visual element. This was illustrated in figure 5.50, repeated here as figure 5.57: the PaintLinearGradient is a leaf node in the graph and creates a masking effect but does not add a distinct visual element.

Figure 5.57 Graph with a leaf node that isn’t a distinct visual element.

Thus, the generalization that can be made regarding the relationship between the number of layers and the nature of the graph is that the number of distinct root-to-leaf paths will be greater than or equal to the number of layers.

The following are necessary for the graph to be well-formed and valid:

- All subtable links shall satisfy the following criteria:

  - Forward offsets are within the COLR table bounds.

  - If a PaintColrLayers table is present, then a LayerList is also present, and the referenced slice is within the length of the LayerList.

  - If a PaintColrGlyph table is present, there is a BaseGlyphPaintRecord for the referenced base glyph ID.

- The graph shall be acyclic.

NOTE 1	These constraints imply that all leaf nodes will be one of PaintSolid, PaintVarSolid, PaintLinearGradient, PaintVarLinearGradient, PaintRadialGradient, PaintVarRadialGradient, PaintSweepGradient, or PaintVarSweepGradient.

For the graph to be acyclic, no paint table shall have any child or descendent paint table that is also its parent or ancestor within the graph. In particular, because the PaintColrLayers and PaintColrGlyph tables use indirect child references rather than forward offsets, they provide a possibility for introducing cycles. Applications should track paint tables within a path in the graph, checking whether any paint table was already encountered within that path. The following pseudo-code algorithm can be used:

// called initially with the root paint and an empty set pathPaints
    function paintIsAcyclic(paint, pathPaints)
        if paint is in pathPaints
          return false // cycle detected
        add paint to pathPaints
        for each childPaint referenced by paint as a child subtable
          call paintIsAcyclic(childPaint, pathPaints)
        remove paint from pathPaints

For the graph to be valid, it shall also be visually bounded, as described in 5.7.11.1.8.2.

NOTE 2	Implementations can combine testing for cycles and other well-formedness or validity requirements together with other processing for rendering the color glyph.

If the graph contains a cycle or is otherwise not well formed or valid, the paint table at which the error occurs should be ignored, that sub-graph should not be rendered, and that node in the graph should be considered to be visually bounded. The application should attempt to render the remainder of the graph, if well-formed and valid.

Future minor version updates of the COLR table could introduce new paint formats. If a paint table with an unrecognized format is encountered, it and its sub-graph should similarly be ignored, the node should be considered to be visually bounded, and the application should attempt to render the remainder of the graph.

If an application is not able to recover from errors while traversing the graph, it may ignore the color glyph entirely. If the base glyph ID has an outline, that may be rendered as a non-color glyph instead.

#### COLR table formats

##### Overview

Various table and record formats are defined for COLR version 0 and version 1. Several values contained within the version 1 formats are variable.

For items that vary in a variable font, the variation data is contained in an ItemVariationStore table (7.2.3). To associate each variable item with the corresponding variation data, a DeltaSetIndexMap table (7.2.3.1) is used. Within a given table that has variable items, a base/sequence scheme is used to index into the mapping data. See 5.7.11.4 for details.

Future minor version updates of the COLR table could introduce new formats that extend the capabilities for color glyph descriptions using version 1 formats. Unrecognized formats should be ignored. See 5.7.11.1.9 for more information.

All table offsets are from the start of the parent table in which the offset is given, unless otherwise indicated.

The COLR table begins with a header. Two versions have been defined. Offsets in the header are from the start of the table.

##### COLR header

###### COLR version 0

COLR version 0:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number—set to 0. |
| uint16 | numBaseGlyphRecords | Number of BaseGlyph records. |
| Offset32 | baseGlyphRecordsOffset | Offset to baseGlyphRecords array. |
| Offset32 | layerRecordsOffset | Offset to layerRecords array. |
| uint16 | numLayerRecords | Number of Layer records. |

NOTE	For fonts that use COLR version 0, some early Windows implementations of the COLR table require glyph ID 1 to be the .null glyph.

###### COLR version 1

COLR version 1:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number—set to 1. |
| uint16 | numBaseGlyphRecords | Number of BaseGlyph records; may be 0 in a version 1 table. |
| Offset32 | baseGlyphRecordsOffset | Offset to baseGlyphRecords array (may be NULL). |
| Offset32 | layerRecordsOffset | Offset to layerRecords array (may be NULL). |
| uint16 | numLayerRecords | Number of Layer records; may be 0 in a version 1 table. |
| Offset32 | baseGlyphListOffset | Offset to BaseGlyphList table. |
| Offset32 | layerListOffset | Offset to LayerList table (may be NULL). |
| Offset32 | clipListOffset | Offset to ClipList table (may be NULL). |
| Offset32 | varIndexMapOffset | Offset to DeltaSetIndexMap table (may be NULL). |
| Offset32 | itemVariationStoreOffset | Offset to ItemVariationStore (may be NULL). |

The BaseGlyphList and its subtables are only used in COLR version 1.

The LayerList is only used in conjunction with the BaseGlyphList and, specifically, with PaintColrLayers tables (5.7.11.2.6.1); it is not required if no color glyphs use a PaintColrLayers table. If not used, set layerListOffset to NULL.

The ClipList is only used in conjunction with the BaseGlyphList. If not used, set clipListOffset to NULL.

The ItemVariationStore (7.2.3) is used in conjunction with a BaseGlyphList and its subtables, but only in variable fonts. If it is not used, set itemVariationStoreOffset to NULL.

The DeltaSetIndexMap table is described in 7.2.3.1. Within the COLR table, either format 0 or format 1 of the DeltaSetIndexMap can be used. A DeltaSetIndexMap is used in conjunction with the ItemVariationStore in a variable font. The DeltaSetIndexMap is optional: if an ItemVariationStore is present but a DeltaSetIndexMap is not included (varIndexMapOffset is NULL), then an implicit mapping is used. See 5.7.11.4 for details.

###### Mixing version 0 and version 1 formats

A font that uses COLR version 1 and that includes a BaseGlyphList can also include BaseGlyph and Layer records for compatibility with applications that only support COLR version 0.

Color glyphs that can be implemented in COLR version 0 using BaseGlyph and Layer records can also be implemented using the version 1 BaseGlyphList and subtables. Thus, a font that uses the version 1 formats does not need to use the version 0 BaseGlyph and Layer records. However, a font may use the version 1 structures for some base glyphs and the version 0 structures for other base glyphs. A font may also include a version 1 color glyph definition for a given base glyph ID that is equivalent to a version 0 definition, though this should never be needed.

A font may define a color glyph for a given base glyph ID using version 0 formats, and also define a different color glyph for the same base glyph ID using version 1 formats. Applications that support COLR version 1 should give preference to the version 1 color glyph.

For applications that support COLR version 1, the application should search for a base glyph ID first in the BaseGlyphList. Then, if not found, search in the baseGlyphRecords array, if present.

##### BaseGlyph and Layer records

BaseGlyph and Layer records are required for COLR version 0, but optional for version 1. (See 5.7.11.2.2.3.)

A BaseGlyph record is used to map a base glyph to a sequence of layer records that define the corresponding color glyph. The BaseGlyph record includes a base glyph index, an index into the layerRecords array, and the number of layers.

BaseGlyph record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Glyph ID of the base glyph. |
| uint16 | firstLayerIndex | Index (base 0) into the layerRecords array. |
| uint16 | numLayers | Number of color layers associated with this glyph. |

The glyph ID shall be less than the numGlyphs value in the ‘maxp’ table (5.2.6).

The BaseGlyph records shall be sorted in increasing glyphID order. It is assumed that a binary search can be used to find a matching BaseGlyph record for a specific glyphID.

The color glyph for a given base glyph is defined by the consecutive records in the layerRecords array for the specified number of layers, starting with the record indicated by firstLayerIndex. The first record in this sequence is the bottom layer in the z-order, and each subsequent layer is stacked on top of the previous layer.

The layer record sequences for two different base glyphs may overlap, with some layer records used in multiple color glyph definitions.

The Layer record specifies the glyph used as the graphic element for a layer and the solid color fill.

Layer record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Glyph ID of the glyph used for a given layer. |
| uint16 | paletteIndex | Index (base 0) for a palette entry in the CPAL table. |

The glyphID in a Layer record shall be less than the numGlyphs value in the ‘maxp’ table. That is, it shall be a valid glyph with outline data in the ‘glyf’ (5.3.4), ‘CFF’ (5.4.2) or CFF2 (5.4.3) table. See 5.7.11.1.8.2 for requirements regarding glyph metrics of referenced glyphs.

The paletteIndex value shall be less than the numPaletteEntries value in the CPAL table (5.7.12). A paletteIndex value of 0xFFFF is a special case, indicating that the text foreground color (as determined by the application) is to be used.

##### BaseGlyphList, LayerList and ClipList

The BaseGlyphList table is, conceptually, similar to the baseGlyphRecords array in COLR version 0, providing records that map a base glyph to a color glyph definition. The color glyph definitions that each refer to are significantly different, however—see 5.7.11.1.

BaseGlyphList table:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | numBaseGlyphPaintRecords |  |
| BaseGlyphPaintRecord | baseGlyphPaintRecords[numBaseGlyphPaintRecords] |  |

BaseGlyphPaintRecord:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Glyph ID of the base glyph. |
| Offset32 | paintOffset | Offset to a Paint table. |

The glyphID value shall be less than the numGlyphs value in the ‘maxp’ table (5.2.6).

The records in the baseGlyphPaintRecords array shall be sorted in increasing glyphID order. It is intended that a binary search can be used to find a matching BaseGlyphPaintRecord for a specific glyphID.

The paint table referenced by the BaseGlyphPaintRecord is the root of the graph for a color glyph definition.

NOTE 1	Often the paint table that is the root of the graph for the color glyph definition will be a PaintColrLayers table, though this is not required. See 5.7.11.1.9 for more information regarding the graph of a color glyph, and 5.7.11.1.4 for background information regarding the PaintColrLayers table.

A LayerList table is used in conjunction with PaintColrLayers tables to represent layer structures. A single LayerList is defined and can be used by multiple PaintColrLayers tables, each of which references a slice of the layer list.

LayerList table:

| Type | Name | Description |
| --- | --- | --- |
| uint32 | numLayers |  |
| Offset32 | paintOffsets[numLayers] | Offsets to Paint tables. |

The sequence of offsets to paint tables corresponds to a bottom-up z-order layering of the graphic compositions defined by the sub-graph of each referenced paint table graph. For a given slice of the list, the sub-graph of the first paint table defines the element at the bottom of the z-order, and the sub-graph of each subsequent paint table defines an element that is layered on top of the previous element. As each element is a composition defined in a sub-graph, one of these elements may itself be multi-layered. In that case, the layers of this element are stacked above all previous layers, and layers of following elements are stacked above the top layer of this element.

Offsets for paint tables not referenced by any PaintColrLayers table should not be included in the paintOffsets array.

A ClipList table is used to provide precomputed clip boxes for color glyphs. It contains an array of Clip records, each of which associates a range of base glyph IDs with a ClipBox table. The ClipBox table provides a precomputed clip box for the associated color glyphs. Clip boxes are optional: a font may provide clip boxes for some color glyphs but not others.

ClipList table:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 1. |
| uint32 | numClips | Number of Clip records. |
| Clip | clips[numClips] | Clip records. Sorted by startGlyphID. |

Clip record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | startGlyphID | First glyph ID in the range. |
| uint16 | endGlyphID | Last glyph ID in the range. |
| Offset32 | clipBoxOffset | Offset to a ClipBox table. |

Within a ClipList table, the glyph ID ranges of Clip records shall not overlap.

Two Clipbox table formats are defined: format 1 for clip boxes without variation, and format 2 allowing for clip boxes that can vary in a variable font.

ClipBoxFormat1 table, static clip box:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 1. |
| FWORD | xMin | Minimum x of clip box. |
| FWORD | yMin | Minimum y of clip box. |
| FWORD | xMax | Maximum x of clip box. |
| FWORD | yMax | Maximum y of clip box. |

ClipBoxFormat2 table, variable clip box:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 2. |
| FWORD | xMin | Minimum x of clip box. For variation, use varIndexBase + 0. |
| FWORD | yMin | Minimum y of clip box. For variation, use varIndexBase + 1. |
| FWORD | xMax | Maximum x of clip box. For variation, use varIndexBase + 2. |
| FWORD | yMax | Maximum y of clip box. For variation, use varIndexBase + 3. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

Any content drawn outside the clip box shall not render.

The clip box is not required to be a tight bounding box around the content. As it may be used by implementations to allocate resources, however, it should not be unnecessarily large.

NOTE 2	At runtime, when computing a variable ClipBox, compute the min/max coordinates using floating point values and then round to integer values such that the clip box expands. That is, round xMin and yMin towards negative infinity and round xMax and yMax towards positive infinity.

For variable data, a base/sequence scheme is used to index into variation mapping data. See 5.7.11.4 for details.

##### Color references, ColorStop and ColorLine

Colors are used in solid color fills for graphic elements, or as stops in a color line used to define a gradient. Colors are defined by reference to palette entries in the CPAL table (5.7.12). While CPAL entries include an alpha component, formats for COLR version 1 that reference palette entries also includes a separate alpha specification to allow different graphic elements to use the same color but with different alpha values, and to allow for variation of the alpha in variable fonts.

A paletteIndex value of 0xFFFF is a special case, indicating that the text foreground color (as determined by the application) is to be used.

The alpha value is always set explicitly. Values for alpha outside the range [0., 1.] (inclusive) are reserved; values outside this range shall be clamped. A value of zero means no opacity (fully transparent); 1.0 means fully opaque (no transparency). The alpha indicated in this record is multiplied with the alpha component of the CPAL entry (converted to float—divide by 255). Note that the resulting alpha value can be combined with and does not supersede alpha or opacity attributes set in higher-level, application-defined contexts.

See 5.7.11.1.1 for more information regarding color references and solid color fills. Solid color fills are defined using a PaintSolid or PaintVarSolid table, described below—see 5.7.11.2.6.2.

Gradients are defined using a color line. A color line is a mapping of real numbers to color values, defined using color stops. See 5.7.11.1.2.2 for an overview and additional details.

Two color-stop record formats are defined: one that allows for variation of stop offset position or of alpha, and one that does not.

ColorStop record:

| Type | Name | Description |
| --- | --- | --- |
| F2DOT14 | stopOffset | Position on a color line. |
| uint16 | paletteIndex | Index for a CPAL palette entry. |
| F2DOT14 | alpha | Alpha value. |

VarColorStop record:

| Type | Name | Description |
| --- | --- | --- |
| F2DOT14 | stopOffset | Position on a color line. For variation, use varIndexBase + 0. |
| uint16 | paletteIndex | Index for a CPAL palette entry. |
| F2DOT14 | alpha | Alpha value. For variation, use varIndexBase + 1. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The VarColorStop format uses a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

A color line is defined by an array of color stop records plus an extend mode. Two color-line table formats are defined: one that allows for variation of color stop offsets positions or of alpha values, and one that does not. Different paint table formats for gradients use one or the other of the color line formats.

ColorLine table:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | extend | An Extend enum value. |
| uint16 | numStops | Number of ColorStop records. |
| ColorStop | colorStops[numStops] |  |

VarColorLine table:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | extend | An Extend enum value. |
| uint16 | numStops | Number of ColorStop records. |
| VarColorStop | colorStops[numStops] | Allows for variations. |

Applications shall apply the colorStops entries in increasing stopOffset order. Within a variable font, the stopOffset values can vary, and the relative orderings of color stop records along the color line can change as a result of variation. With a variable font, the colorStops entries shall be ordered after the instance values for the stop offsets have been derived.

A color line defines stops for only certain positions along the line, but the color line extends infinitely in either direction. The extend field is used to indicate how the color line is extended. The same behavior is used for extension in both directions. The extend field uses the following enumeration:

Extend enumeration:

| Value | Name | Description |
| --- | --- | --- |
| 0 | EXTEND_PAD | Use nearest color stop. |
| 1 | EXTEND_REPEAT | Repeat from farthest color stop. |
| 2 | EXTEND_REFLECT | Mirror color line from nearest end. |

The extend mode behaviors are described in detail in 5.7.11.1.2.2. If a ColorLine in a font has an unrecognized extend value, applications should use EXTEND_PAD by default.

##### Paint tables

Paint tables are used for COLR version 1 color glyph definitions. Thirty-two paint table formats are defined (formats 1 to 32). Some formats come in non-variable and variable pairs, but otherwise, each provides different graphic capability for defining the composition for a color glyph. The graphic capability of each format and the manner in which they are combined to represent a color glyph has been described above—see 5.7.11.1.

Each paint table format has a one-byte format field as the first field. When parsing font data, the format field can be read first to determine the format of the table.

###### Format 1: PaintColrLayers

Format 1 is used to define a vector of layers. The layers are a slice of layers from the LayerList table. The first layer is the bottom of the z-order, and subsequent layers are composited on top using the COMPOSITE_SRC_OVER composition mode (see 5.7.11.2.6.13).

For general information on the PaintColrLayers table, see 5.7.11.1.4. For information about its use for shared, re-usable components, see 5.7.11.1.7.3.

PaintColrLayers table (format 1):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 1. |
| uint8 | numLayers | Number of offsets to paint tables to read from LayerList. |
| uint32 | firstLayerIndex | Index (base 0) into the LayerList. |

NOTE	An 8-bit value is used for numLayers to minimize size for common scenarios. If more than 256 layers are needed, then two or more PaintColrLayers tables can be combined in a tree using a PaintComposite table or another PaintColrLayers table to combine them.

###### Formats 2 and 3: PaintSolid, PaintVarSolid

Formats 2 and 3 are used to specify a solid color fill. Format 3 allows for variation of alpha in a variable font; format 2 provides a more compact representation when variation is not required. Format 3 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

For general information about specifying color values, see 5.7.11.1.1. For information about applying a fill to a shape, see 5.7.11.1.3.

PaintSolid table (format 2):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 2. |
| uint16 | paletteIndex | Index for a CPAL palette entry. |
| F2DOT14 | alpha | Alpha value. |

PaintVarSolid table (format 3):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 3. |
| uint16 | paletteIndex | Index for a CPAL palette entry. |
| F2DOT14 | alpha | Alpha value. For variation, use varIndexBase + 0. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarSolid format uses a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

###### Formats 4 and 5: PaintLinearGradient, PaintVarLinearGradient

Formats 4 and 5 are used to specify a linear gradient fill. Format 5 allows for variation of color stop positions or of alpha in a variable font; format 4 provides a more compact representation when variation is not required. Format 5 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

For general information about linear gradients, see 5.7.11.1.2.3. For information about applying a fill to a shape, see 5.7.11.1.3.

The PaintLinearGradient and PaintVarLinearGradient tables have a ColorLine and VarColorLine subtable, respectively. For the ColorLine and VarColorLine table formats, see 5.7.11.2.5. For background information on the color line, see 5.7.11.1.2.2.

PaintLinearGradient table (format 4):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 4. |
| Offset24 | colorLineOffset | Offset to ColorLine table. |
| FWORD | x0 | Start point (p₀) x coordinate. |
| FWORD | y0 | Start point (p₀) y coordinate. |
| FWORD | x1 | End point (p₁) x coordinate. |
| FWORD | y1 | End point (p₁) y coordinate. |
| FWORD | x2 | Rotation point (p₂) x coordinate. |
| FWORD | y2 | Rotation point (p₂) y coordinate. |

PaintVarLinearGradient table (format 5):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 5. |
| Offset24 | colorLineOffset | Offset to VarColorLine table. |
| FWORD | x0 | Start point (p₀) x coordinate. For variation, use varIndexBase + 0. |
| FWORD | y0 | Start point (p₀) y coordinate. For variation, use varIndexBase + 1. |
| FWORD | x1 | End point (p₁) x coordinate. For variation, use varIndexBase + 2. |
| FWORD | y1 | End point (p₁) y coordinate. For variation, use varIndexBase + 3. |
| FWORD | x2 | Rotation point (p₂) x coordinate. For variation, use varIndexBase + 4. |
| FWORD | y2 | Rotation point (p₂) y coordinate. For variation, use varIndexBase + 5. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarLinearGradient format uses a base/sequence scheme to index into mapping data; see 5.7.11.4 for details.

###### Formats 6 and 7: PaintRadialGradient, PaintVarRadialGradient

Formats 6 and 7 are used to specify a radial gradient fill. Format 7 allows for variation of color stop positions or of alpha in a variable font; format 6 provides a more compact representation when variation is not required. Format 7 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

For general information about radial gradients supported in COLR version 1, see 5.7.11.1.2.4. For information about applying a fill to a shape, see 5.7.11.1.3.

The PaintRadialGradient and PaintVarRadialGradient tables have a ColorLine and VarColorLine subtable, respectively. For the ColorLine and VarColorLine table formats, see in 5.7.11.2.5. For background information on the color line, see 5.7.11.1.2.2.

PaintRadialGradient table (format 6):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 6. |
| Offset24 | colorLineOffset | Offset to ColorLine table. |
| FWORD | x0 | Start circle center x coordinate. |
| FWORD | y0 | Start circle center y coordinate. |
| UFWORD | radius0 | Start circle radius. |
| FWORD | x1 | End circle center x coordinate. |
| FWORD | y1 | End circle center y coordinate. |
| UFWORD | radius1 | End circle radius. |

PaintVarRadialGradient table (format 7):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 7. |
| Offset24 | colorLineOffset | Offset to VarColorLine table. |
| FWORD | x0 | Start circle center x coordinate. For variation, use varIndexBase + 0. |
| FWORD | y0 | Start circle center y coordinate. For variation, use varIndexBase + 1. |
| UFWORD | radius0 | Start circle radius. For variation, use varIndexBase + 2. |
| FWORD | x1 | End circle center x coordinate. For variation, use varIndexBase + 3. |
| FWORD | y1 | End circle center y coordinate. For variation, use varIndexBase + 4. |
| UFWORD | radius1 | End circle radius. For variation, use varIndexBase + 5. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarRadialGradient format uses a base/sequence scheme to index into mapping data; see 5.7.11.4 for details.

###### Formats 8 and 9: PaintSweepGradient, PaintVarSweepGradient

Formats 8 and 9 are used to specify a sweep gradient fill. Format 9 allows for variation of color stop positions or of alpha in a variable font; format 8 provides a more compact representation when variation is not required. Format 9 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

For general information about sweep gradients, see 5.7.11.1.2.5. For information about applying a fill to a shape, see 5.7.11.1.3.

The PaintSweepGradient and PaintVarSweepGradient table have a ColorLine and VarColorLine subtable, respectively. For the ColorLine and VarColorLine table formats, see 5.7.11.2.5. For background information on the color line, see 5.7.11.1.2.2.

PaintSweepGradient table (format 8):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 8. |
| Offset24 | colorLineOffset | Offset to ColorLine table. |
| FWORD | centerX | Center x coordinate. |
| FWORD | centerY | Center y coordinate. |
| F2DOT14 | startAngle | Start of the angular range of the gradient: add 1.0 and multiply by 180° to retrieve counter-clockwise degrees. |
| F2DOT14 | endAngle | End of the angular range of the gradient: add 1.0 and multiply by 180° to retrieve counter-clockwise degrees. |

PaintVarSweepGradient table (format 9):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 9. |
| Offset24 | colorLineOffset | Offset to VarColorLine table. |
| FWORD | centerX | Center x coordinate. For variation, use varIndexBase + 0. |
| FWORD | centerY | Center y coordinate. For variation, use varIndexBase + 1. |
| F2DOT14 | startAngle | Start of the angular range of the gradient: add 1.0 and multiply by 180° to retrieve counter-clockwise degrees. For variation, use varIndexBase + 2. |
| F2DOT14 | endAngle | End of the angular range of the gradient: add 1.0 and multiply by 180° to retrieve counter-clockwise degrees. For variation, use varIndexBase + 3. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarSweepGradient format uses a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

Angles are expressed in counter-clockwise degrees from the direction of the positive x-axis in the design grid.

NOTE	To allow for a representation of +360°, a bias of 1.0 is used in the representation of start and end angles of sweep gradients. For example, an F2DOT14 value of -2.0 (0x8000) represents -180°; an F2DOT14 value of 0.0 (0x0000) represents +180°; an F2DOT14 value of 0.25 (0x1000) represents +225°; an F2DOT14 value of 1.0 (0x4000) represents +360°. However, a bias is not used for representation of angles in rotate or skew transforms (5.7.11.2.5.11, 5.7.11.2.5.12).

###### Format 10: PaintGlyph

Format 10 is used to specify a glyph outline to use as a shape to be filled or, equivalently, a clip region. The outline sets a clip region that constrains the content of a separate paint subtable and the sub-graph linked from that subtable.

For information about applying a fill to a shape, see 5.7.11.1.3.

PaintGlyph table (format 10):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 10. |
| Offset24 | paintOffset | Offset to a Paint table. |
| uint16 | glyphID | Glyph ID for the source outline. |

The glyphID value shall be less than the numGlyphs value in the ‘maxp’ table (5.2.6). That is, it shall be a valid glyph with outline data in the ‘glyf’ (5.3.4), ‘CFF’ (5.4.2) or CFF2 (5.4.3) table. Only that outline data is used. In particular, if this glyph ID has a description in the COLR table (glyphID appears in a COLR BaseGlyph record or the BaseGlyphList), that COLR data is not relevant for purposes of the PaintGlyph table.

###### Format 11: PaintColrGlyph

Format 11 is used to allow a color glyph definition from the BaseGlyphList to be a re-usable component that can be incorporated into multiple color glyph definitions. See 5.7.11.1.7.4 for more information.

PaintColrGlyph table (format 11):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 11. |
| uint16 | glyphID | Glyph ID for a BaseGlyphList base glyph. |

The glyphID value shall be a glyphID found in a BaseGlyphPaintRecord within the BaseGlyphList. The BaseGlyphPaintRecord provides an offset to a paint table; that paint table and the graph linked from it are incorporated as a child sub-graph of the PaintColrGlyph table within the current color glyph definition.

###### Formats 12 and 13: PaintTransform, PaintVarTransform

Formats 12 and 13 are used to apply an affine transformation to a sub-graph. The paint table that is the root of the sub-graph is linked as a child.

Format 13 allows for variation of the transformation in a variable font; format 12 provides a more compact representation when variation is not required. Format 13 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

For general information regarding transformations in a color glyph definition, see 5.7.11.1.5.

PaintTransform table (format 12):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 12. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| Offset24 | transformOffset | Offset to an Affine2x3 table. |

PaintVarTransform table (format 13):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 13. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| Offset24 | transformOffset | Offset to a VarAffine2x3 table. |

The affine transformation is defined by a 2×3 matrix, specified in an Affine2x3 or VarAffine2x3 record. The 2×3 matrix supports scale, skew, reflection, rotation, and translation transformations. The VarAffine2x3 table supports mapping into variation data, allowing the transform definition to be variable in a variable font.

Affine2x3 table:

| Type | Name | Description |
| --- | --- | --- |
| Fixed | xx | x-component of transformed x-basis vector. |
| Fixed | yx | y-component of transformed x-basis vector. |
| Fixed | xy | x-component of transformed y-basis vector. |
| Fixed | yy | y-component of transformed y-basis vector. |
| Fixed | dx | Translation in x direction. |
| Fixed | dy | Translation in y direction. |

VarAffine2x3 table:

| Type | Name | Description |
| --- | --- | --- |
| Fixed | xx | x-component of transformed x-basis vector. For variation, use varIndexBase + 0. |
| Fixed | yx | y-component of transformed x-basis vector. For variation, use varIndexBase + 1. |
| Fixed | xy | x-component of transformed y-basis vector. For variation, use varIndexBase + 2. |
| Fixed | yy | y-component of transformed y-basis vector. For variation, use varIndexBase + 3. |
| Fixed | dx | Translation in x direction. For variation, use varIndexBase + 4. |
| Fixed | dy | Translation in y direction. For variation, use varIndexBase + 5. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The VarAffine2x3 format uses a base/sequence scheme to index into mapping data. Also, applying variation deltas to a Fixed value requires special handling. See 5.7.11.4 for details.

For a pre-transformation position (x, y), the post-transformation position (x′, y′) is calculated as follows:

x′ = xx * x + xy * y + dx
y′ = yx * x + yy * y + dy

NOTE	It is helpful to understand linear transformations by their effect on x- and y-basis vectors î = (1, 0) and ĵ = (0, 1). The transform described by the Affine2x3 or VarAffine2x3 table maps the basis vectors to î′ = (xx, yx) and ĵ′ = (xy, yy), and translates the origin to (dx, dy).

When the transformed composition from the referenced paint table (and its sub-graph) is composed into the destination (represented by the parent of this table), the source design grid origin is aligned to the destination design grid origin. The transform can translate the source such that a pre-transform position (0,0) is moved elsewhere. The post-transform origin, (0,0), is aligned to the destination origin.

###### Formats 14 and 15: PaintTranslate, PaintVarTranslate

Formats 14 and 15 are used to apply a translation to a sub-graph. The paint table that is the root of the sub-graph is linked as a child.

Format 15 allows for variation of the translation in a variable font; format 14 provides a more compact representation when variation is not required. Format 15 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

These tables use reduced precision for compactness. Where higher precision is required use PaintTransform/PaintVarTransform.

For general information regarding transformations in a color glyph definition, see 5.7.11.1.5.

PaintTranslate table (format 14):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 14. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| FWORD | dx | Translation in x direction. |
| FWORD | dy | Translation in y direction. |

PaintVarTranslate table (format 15):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 15. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| FWORD | dx | Translation in x direction. For variation, use varIndexBase + 0. |
| FWORD | dy | Translation in y direction. For variation, use varIndexBase + 1. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarTranslate format uses a base/sequence scheme to index into mapping data; see 5.7.11.4 for details.

NOTE	Pure translation can also be represented using the PaintTransform or PaintVarTransform table by setting xx = 1, yy = 1, xy and yx = 0, and setting dx and dy to the translation values. The PaintTranslate or PaintVarTranslate table provides a more compact representation when only translation is required.

The translation will result in the pre-transform position (0,0) being moved elsewhere. See 5.7.11.2.6.8 regarding alignment of the transformed content with the destination.

###### Formats 16 to 23: PaintScale and variant scaling formats

Formats 16 to 23 are used to scale a sub-graph. The paint table that is the root of the sub-graph is linked as a child. Several variant formats are provided:

- Formats 16 and 17: scale in x or y directions relative to the origin. Format 17 allows for variation of the x and y scale factors in a variable font; format 16 provides a more compact representation when variation is not required.

- Formats 18 and 19: scale in x or y directions relative to a specified center. Format 19 allows for variation of the x and y scale factors or of the center position; format 18 provides a more compact representation when variation is not required.

- Formats 20 and 21: scale uniformly in x and y directions relative to the origin. Format 21 allows for variation of the scale factor in a variable font; format 20 provides a more compact representation when variation is not required.

- Formats 22 and 23: scale uniformly in x and y directions relative to a specified center. Format 23 allows for variation of the scale factor or of the center position; format 22 provides a more compact representation when variation is not required.

Formats 17, 19, 21 and 23 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

These tables use reduced precision for compactness. Where higher precision is required use PaintTransform/PaintVarTransform.

For general information regarding transformations in a color glyph definition, see 5.7.11.1.5.

PaintScale table (format 16):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 16. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scaleX | Scale factor in x direction. |
| F2DOT14 | scaleY | Scale factor in y direction. |

PaintVarScale table (format 17):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 17. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scaleX | Scale factor in x direction. For variation, use varIndexBase + 0. |
| F2DOT14 | scaleY | Scale factor in y direction. For variation, use varIndexBase + 1. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

PaintScaleAroundCenter table (format 18):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 18. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scaleX | Scale factor in x direction. |
| F2DOT14 | scaleY | Scale factor in y direction. |
| FWORD | centerX | x coordinate for the center of scaling. |
| FWORD | centerY | y coordinate for the center of scaling. |

PaintVarScaleAroundCenter table (format 19):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 19. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scaleX | Scale factor in x direction. For variation, use varIndexBase + 0. |
| F2DOT14 | scaleY | Scale factor in y direction. For variation, use varIndexBase + 1. |
| FWORD | centerX | x coordinate for the center of scaling. For variation, use varIndexBase + 2. |
| FWORD | centerY | y coordinate for the center of scaling. For variation, use varIndexBase + 3. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

PaintScaleUniform table (format 20):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 20. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scale | Scale factor in x and y directions. |

PaintVarScaleUniform table (format 21):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 21. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scaleX | Scale factor in x and y directions. For variation, use varIndexBase + 0. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

PaintScaleUniformAroundCenter table (format 22):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 22. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scale | Scale factor in x and y directions. |
| FWORD | centerX | x coordinate for the center of scaling. |
| FWORD | centerY | y coordinate for the center of scaling. |

PaintVarScaleUniformAroundCenter table (format 23):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 23. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | scale | Scale factor in x and y directions. For variation, use varIndexBase + 0. |
| FWORD | centerX | x coordinate for the center of scaling. For variation, use varIndexBase + 1. |
| FWORD | centerY | y coordinate for the center of scaling. For variation, use varIndexBase + 2. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarScale, PaintVarScaleAroundCenter, PaintVarScaleUniform, and PaintVarScaleUniformAroundCenter formats use a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

NOTE	Pure scaling can also be represented using the PaintTransform or PaintVarTransform table. For scaling about the origin, this could be done by setting xx and yy to x and y scale factors, and setting xy, yx, dx and dy = 0. The PaintScale table and variants provide more compact representation when only scaling is required.

###### Formats 24 to 27: PaintRotate, PaintVarRotate, PaintRotateAroundCenter, PaintVarRotateAroundCenter

Formats 24 to 27 are used to apply a rotation to a sub-graph. The paint table that is the root of the sub-graph is linked as a child. The amount of rotation is expressed directly as an angle, using a floating point value where 1.0 represents an angle of 180°.

Formats 24 and 25 apply rotations using the origin as the center of rotation. Format 25 allows for variation of the rotation in a variable font; format 24 provides a more compact representation when variation is not required.

Formats 26 and 27 apply rotations around a specified center of rotation. Format 27 allows for variation of the rotation or of the position of the center of rotation in a variable font; format 26 provides a more compact representation when variation is not required.

Formats 25 and 27 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

These tables use reduced precision for compactness. Where higher precision is required use PaintTransform/PaintVarTransform.

For general information regarding transformations in a color glyph definition, see 5.7.11.1.5.

PaintRotate table (format 24):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 24. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | angle | Rotation angle, 180° in counter-clockwise degrees per 1.0 of value. |

PaintVarRotate table (format 25):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 25. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | angle | Rotation angle, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 0. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

PaintRotateAroundCenter table (format 26):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 26. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | angle | Rotation angle, 180° in counter-clockwise degrees per 1.0 of value. |
| FWORD | centerX | x coordinate for the center of rotation. |
| FWORD | centerY | y coordinate for the center of rotation. |

PaintVarRotateAroundCenter table (format 27):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 27. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | angle | Rotation angle, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 0. |
| FWORD | centerX | x coordinate for the center of rotation. For variation, use varIndexBase + 1. |
| FWORD | centerY | y coordinate for the center of rotation. For variation, use varIndexBase + 2. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarRotate and PaintVarRotateAroundCenter formats use a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

NOTE 1	Pure rotation about a point can also be represented using the PaintTransform or PaintVarTransform table. For rotation about the origin, this could be done by setting matrix values as follows for angle θ:

- xx = cos(θ)

- yx = sin(θ)

- xy = -sin(θ)

- yy = cos(θ)

- dx = dy = 0

The important difference of the PaintRotate table and its variants is in allowing an angle to be specified directly in degrees, rather than as changes to basis vectors. In variable fonts, if a rotation angle needs to vary, it is easier to get smooth variation if an angle is specified directly than when using trigonometric functions to derive matrix elements.

NOTE 2	The rotation angle is represented using an F2DOT14 value, which supports values in the range [-2, 2). Since each 1.0 unit represents a change of 180°, rotation angles of [-360, 360) can be represented directly. Variations of the rotation angle are not limited to that range, however.

NOTE 3	If representation of rotation directly as an angle is preferred but higher precision is required to specify a center of rotation, a chained sequence of transforms can be used. For example, a PaintTransform can be used to align the origin to the desired center of rotation, then PaintRotate can be used for the desired rotation, and a second PaintTransform can be used to reset the origin.

When combining the transform effect of a PaintRotate table (or variants) with other transforms, the result shall be the same as if the rotation were represented using an equivalent matrix or sequence of matrices.

A rotation can result in the pre-transform position (0, 0) being moved elsewhere. See 5.7.11.2.6.8 regarding alignment of the transformed content with the destination.

###### Format 28 to 31: PaintSkew, PaintVarSkew, PaintSkewAroundCenter, PaintVarSkewAroundCenter

Formats 28 to 31 are used to apply a skew to a sub-graph. The paint table that is the root of the sub-graph is linked as a child. The amounts of skew in the x or y direction are expressed directly as angles, using floating point values where 1.0 represents an angle of 180°.

Formats 28 and 29 apply skews using the origin as the center of rotation for the skew. Format 29 allows for variation of the rotation in a variable font; format 28 provides a more compact representation when variation is not required.

Formats 30 and 31 apply skews around a specified center of rotation. Format 31 allows for variation of the rotation or of the position of the center of rotation in a variable font; format 30 provides a more compact representation when variation is not required.

Formats 29 and 31 shall not be used in non-variable fonts or if the COLR table does not have an ItemVariationStore subtable.

These tables use reduced precision for compactness. Where higher precision is required use PaintTransform/PaintVarTransform.

For general information regarding transformations in a color glyph definition, see 5.7.11.1.5.

PaintSkew table (format 28):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 28. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | xSkewAngle | Angle of skew in the direction of the x-axis, 180° in counter-clockwise degrees per 1.0 of value. |
| F2DOT14 | ySkewAngle | Angle of skew in the direction of the y-axis, 180° in counter-clockwise degrees per 1.0 of value. |

PaintVarSkew table (format 29):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 29. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | xSkewAngle | Angle of skew in the direction of the x-axis, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 0. |
| F2DOT14 | ySkewAngle | Angle of skew in the direction of the y-axis, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 1. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

PaintSkewAroundCenter table (format 30):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 30. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | xSkewAngle | Angle of skew in the direction of the x-axis, 180° in counter-clockwise degrees per 1.0 of value. |
| F2DOT14 | ySkewAngle | Angle of skew in the direction of the y-axis, 180° in counter-clockwise degrees per 1.0 of value. |
| FWORD | centerX | x coordinate for the center of rotation. |
| FWORD | centerY | y coordinate for the center of rotation. |

PaintVarSkewAroundCenter table (format 31):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 31. |
| Offset24 | paintOffset | Offset to a Paint subtable. |
| F2DOT14 | xSkewAngle | Angle of skew in the direction of the x-axis, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 0. |
| F2DOT14 | ySkewAngle | Angle of skew in the direction of the y-axis, 180° in counter-clockwise degrees per 1.0 of value. For variation, use varIndexBase + 1. |
| FWORD | centerX | x coordinate for the center of rotation. For variation, use varIndexBase + 2. |
| FWORD | centerY | y coordinate for the center of rotation. For variation, use varIndexBase + 3. |
| uint32 | varIndexBase | Base index into DeltaSetIndexMap. |

The PaintVarSkew and PaintVarSkewAroundCenter formats use a base/sequence scheme to index into mapping data. Also, applying variation deltas to an F2DOT14 value requires special handling. See 5.7.11.4 for details.

NOTE 1	Pure skews about a point can also be represented using the PaintTransform or PaintVarTransform table. For skews about the origin, this could be done by setting matrix values as follows for x skew angle φ and y skew angle ψ:

- xx = yy = 1

- yx = tan(ψ)

- xy = -tan(φ)

- dx = dy = 0

The important difference of the PaintSkew table and its variants is in being able to specify skew as an angle, rather than as changes to basis vectors. In variable fonts, if a skew angle needs to vary, it is easier to get smooth variation if an angle is specified directly than when using trigonometric functions to derive matrix elements.

NOTE 2	The skew angles are represented using F2DOT14 values, which support values in the range [-2, 2). Since each 1.0 unit represents a change of 180°, skew angles of [-360, 360) can be represented directly. Variations of the skew angle are not limited to that range, however.

NOTE 3	If representation of skew directly as an angle is preferred but higher precision is required to specify a center of rotation, a chained sequence of transforms can be used. For example, a PaintTransform can be used to align the origin to the desired center of rotation, then PaintSkew can be used for the desired skew rotation, and a second PaintTransform can be used to reset the origin.

When combining the transform effect of a PaintSkew table (or variants) with other transforms, the result shall be the same as if the skew were represented using an equivalent matrix or sequence of matrices.

A skew can result in the pre-transform position (0, 0) being moved elsewhere. See 5.7.11.2.6.8 regarding alignment of the transformed content with the destination.

###### Format 32: PaintComposite

Format 32 is used to combine two layered compositions, referred to as source and backdrop, using different compositing or blending modes. The available compositing and blending modes are defined in an enumeration. For general information and examples, see 5.7.11.1.6.

NOTE	The backdrop is also referred to as the “destination”.

PaintComposite table (format 32):

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | Set to 32. |
| Offset24 | sourcePaintOffset | Offset to a source Paint table. |
| uint8 | compositeMode | A CompositeMode enumeration value. |
| Offset24 | backdropPaintOffset | Offset to a backdrop Paint table. |

The compositeMode value shall be one of the values defined in the CompositeMode enumeration, which are taken from the W3C Compositing and Blending Level 1 specification [33]. Details on each mode, including specifications of the required calculations using pixel color and alpha values, are provided in that specification. If an unrecognized value is encountered, COMPOSITE_CLEAR shall be used.

CompositeMode enumeration:

| Value | Name | Description |
| --- | --- | --- |
|  | Porter-Duff modes |  |
| 0 | COMPOSITE_CLEAR | Clear |
| 1 | COMPOSITE_SRC | Source ("Copy" in [33]) |
| 2 | COMPOSITE_DEST | Destination |
| 3 | COMPOSITE_SRC_OVER | Source Over |
| 4 | COMPOSITE_DEST_OVER | Destination Over |
| 5 | COMPOSITE_SRC_IN | Source In |
| 6 | COMPOSITE_DEST_IN | Destination In |
| 7 | COMPOSITE_SRC_OUT | Source Out |
| 8 | COMPOSITE_DEST_OUT | Destination Out |
| 9 | COMPOSITE_SRC_ATOP | Source Atop |
| 10 | COMPOSITE_DEST_ATOP | Destination Atop |
| 11 | COMPOSITE_XOR | XOR |
| 12 | COMPOSITE_PLUS | Plus (“Lighter” in [33]) |
|  | Separable color blend modes: |  |
| 13 | COMPOSITE_SCREEN | screen |
| 14 | COMPOSITE_OVERLAY | overlay |
| 15 | COMPOSITE_DARKEN | darken |
| 16 | COMPOSITE_LIGHTEN | lighten |
| 17 | COMPOSITE_COLOR_DODGE | color-dodge |
| 18 | COMPOSITE_COLOR_BURN | color-burn |
| 19 | COMPOSITE_HARD_LIGHT | hard-light |
| 20 | COMPOSITE_SOFT_LIGHT | soft-light |
| 21 | COMPOSITE_DIFFERENCE | difference |
| 22 | COMPOSITE_EXCLUSION | exclusion |
| 23 | COMPOSITE_MULTIPLY | multiply |
|  | Non-separable color blend modes: |  |
| 24 | COMPOSITE_HSL_HUE | hue |
| 25 | COMPOSITE_HSL_SATURATION | saturation |
| 26 | COMPOSITE_HSL_COLOR | color |
| 27 | COMPOSITE_HSL_LUMINOSITY | luminosity |

The graphic compositions are defined by the source and backdrop paint tables and their respective sub-graphs. Conceptually, they are rendered into bitmaps, and the source is composited or blended into the backdrop using the specified composite mode.

While color values obtained from the CPAL table are represented in sRGB using the non-linear transfer function defined in the sRGB specification, the compositing and blending calculations are done after applying the inverse transfer function to derive linear-light RGB values. For more information regarding the non-linear and linear-light representations for sRGB, see Interpolation of Colors in 5.7.12.

As mentioned in 5.7.11.1.8.2, a color glyph definition shall be bounded. A sub-graph that has PaintComposite as its root is either bounded or unbounded, depending on the mode used and the boundedness of the source and backdrop sub-graphs. For each mode, boundedness is determined by the boundedness of the source and backdrop as follows:

- Always bounded:

  - COMPOSITE_CLEAR

- Bounded if and only if the source is bounded:

  - COMPOSITE_SRC

  - COMPOSITE_SRC_OUT

- Bounded if and only if the backdrop is bounded:

  - COMPOSITE_DEST

  - COMPOSITE_DEST_OUT

- Bounded if and only if either the source or backdrop is bounded:

  - COMPOSITE_SRC_IN

  - COMPOSITE_DEST_IN

- Bounded if and only if both the source and backdrop are bounded:

  - All other modes

#### COLR version 1 rendering algorithm

The various graphic concepts represented by COLR version 1 formats were individually described in 5.7.11.1, and the various formats were described in 5.7.11.2. Together, these provide most of the necessary details regarding how a color glyph is rendered. The following provides a comprehensive description of the rendering process, considering the graph as a whole.

The following algorithm can be used to render color glyphs defined using version 1 formats. Applications are not required to implement rendering using this algorithm, but shall produce equivalent results.

NOTE	Checks for well-formedness and validity, as described in 5.7.11.1.9, are not repeated here. Actual implementations can integrate such checks with rendering processing.

- Start with an initial drawing surface. As mentioned in 5.7.11.1.8.2, if a clip box is provided, it can be used to determine the size. Otherwise, the graph can be traversed to compute a required size.

- Traverse the graph of a color glyph definition, starting with the root paint table referenced by a BaseGlyphPaintRecord, using the following pseudo-code function.

// render a paint table and its sub-graph
function renderPaint(paint)

  if format 1: // PaintColrLayers
    for each referenced child paint table, in bottom-up z-order:
      // for ordering, see 5.7.11.1.4, 5.7.11.2.6.1
      call renderPaint() passing the child paint table
      compose the returned graphic onto the surface using simple alpha blending

  if format 2 or 3: // PaintSolid, PaintVarSolid
    paint the specified color onto the surface

  if format 4, 5, 6, 7, 8 or 9: 
    // PaintLinearGradient, PaintVarLinearGradient
    // PaintRadialGradient, PaintVarRadialGradient
    // PaintSweepGradient, PaintVarSweepGradient
    paint the gradient onto the surface following the gradient algorithm

  if format 10: // PaintGlyph
    apply the outline of the referenced glyph to the clip region
      // take the intersection of clip regions—see 5.7.11.1.3
    call renderPaint() passing the child paint table
    restore the previous clip region

  if format 11: // PaintColrGlyph
    call renderPaint() passing the paint table referenced by the base glyph ID

  if format 12 to 31: 
    // PaintTransform, PaintVarTransform
     // PaintTranslate, PaintVarTranslate
     // PaintScale*, PaintVarScale*
     // PaintRotate*, PaintVarRotate*
     // PaintSkew*, PaintVarSkew*
    apply the specified transform
      // compose the transform with the current transform state—see 5.7.11.1.5
    call renderPaint() passing the child paint table
    restore the previous transform state

  if format 32: // PaintComposite

    // render backdrop sub-graph
    call renderPaint() passing the backdrop child paint table and save the result

    // render source sub-graph
    call renderPaint() passing the source child paint table and save the result

    // compose source and backdrop
    compose the source and backdrop using the specified composite mode

    // compose final result
    compose the result of the above composition onto the surface using simple
    alpha blending

#### COLR table and OFF Font Variations

The COLR table can be used in variable fonts. For color glyphs defined using version 0 formats, the glyph outlines can be variable, but no other aspect of the color glyph is variable. For color glyphs defined using version 1 formats, items that can be variable include the glyph outlines plus other aspects of the color glyph definition:

- Alpha values

- Color stop offsets in gradient color lines

- Placement of gradients onto the design grid

- The arguments of transformations (matrix elements, angles, etc.)

Variation data is provided in an ItemVariationStore table (7.2.3) contained within the COLR table.

In a variable font, each value within the COLR version 1 formats that is variable needs to be associated with corresponding variation data (delta sets) in the item variation store. This is done using a DeltaSetIndexMap table (defined in 7.3.5.2). The delta-set index mapping table contains an array of entries that provide indices mapping into sets of delta data in the item variation store. Each variable item in the COLR table is given an index (base 0) into the mapping data. For example, if a variable item in the COLR table is given an index value of 5, the sixth entry in the mapping data is used to index into the item variation store.

The indices for variable items in the COLR table are indicated using a base/sequence scheme. Each table or record that contains variable items will use a contiguous sequence of entries in the mapping array, and will include a varIndexBase field that indicates the first entry in the mapping array to be used. The variable fields within that table or record use entries in the mapping array, starting with the varIndexBase entry, in the order the fields occur in the table or record.

For example, the VarAffine2x3 table (5.7.11.2.6.8) has six variable fields followed by the varIndexBase field. For the first variable field (xx), varIndexBase + 0 is used as the index into the mapping array; for the second variable field (yx), varIndexBase + 1 is used as the index into the mapping array; and so on.

If the index for a variable item is greater than or equal to the number of entries in the mapping array, the last mapping array entry shall be used.

The sequence of indices derived from a varIndexBase value do not wrap on overflow and shall not exceed 0xFFFFFFFF. A varIndexBase value of 0xFFFFFFFF is assigned a special meaning indicating that the variable fields in the given table or record do not have variation data.

Similarly, a delta-set index mapping entry with values 0xFFFF/0xFFFF can be used to indicate that an item has no variation data. (See 7.2.3.2.)

If the COLR table does not contain an ItemVariationStore subtable, the varIndexBase field of variable tables or records shall be ignored by applications, and should be set to zero.

If the COLR table contains an item variation store but does not contain a mapping table (varIndexMapOffset in the COLR header is NULL), then an implicit identity mapping is used: the sequence of values beginning with varIndexBase are treated directly as delta-set indices with 16-bit sub-fields for outer (high word) and inner (low word) index values. See 7.2.3.3 for more information regarding delta set indices.

For variable fonts that use COLR version 1 formats, special considerations apply to the effect of variation on the bounding box. See 5.7.11.1.8.2 for details. Also, special considerations apply for color lines and for radial gradients. See 5.7.11.1.2.2 and 5.7.11.1.2.4 for details.

Some COLR version 1 formats allow for variation deltas to be applied to F2DOT14 or Fixed values. While the F2DOT14 and Fixed types have fractional components, variation deltas are integer values. Applying deltas to F2DOT14 or Fixed values requires special handling in which these values are treated like integers. See 7.2.3.4 for details.

For general information on OFF font variations, see 7.1.

### CPAL—Palette Table

The palette table is a set of one or more palettes, each containing a predefined number of color records. It may also contain name table IDs describing the palettes and their entries.

Palettes are defined by a set of color records. Each color record specifies a color in the sRGB color space using 8-bit BGRA (blue, green, red, alpha) representation. The sRGB color space is specified in IEC 61966-2-1. Details on the specification for the sRGB color space, including the color primaries and “gamma” transfer function, are also provided in CSS Color Module Level 4, section 10.2 [34].

All palettes have the same number of color records, specified by numColorRecords. All color records for all palettes are arranged in a single array, and the color records for any given palette are a contiguous sequence of color records within that array. The first color record of each palette is provided in the colorRecordIndices array.

Multiple colorRecordIndices may refer to the same color record, in which case multiple palettes would use the same color records; hence the number of functionally-distinct palettes may be fewer than the numPalettes entry. Also, the sequence of color records for different palettes may overlap, with certain color records shared between multiple palettes. Thus, the total number of color records in the CPAL table may be less than the number of palette entries multiplied by the number of palettes.

The first palette, number 0, is the default palette. A minimum of one palette must be provided in the CPAL table if the table is present. Palettes shall have a minimum of one color record. An empty CPAL table, with no palettes and no color records is not permitted.

Colors within a palette are referenced by base-zero index. The number of colors in each palette is given by numPaletteEntries. The number of color records in the color records array (numColorRecords) shall be greater than or equal to max(colorRecordIndices) + numPaletteEntries.

Palette Table Header

The CPAL table begins with a header that starts with a version number. Currently, only versions 0 and 1 are defined.

CPAL version 0

The CPAL header version 0 is organized as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number (=0). |
| uint16 | numPaletteEntries | Number of palette entries in each palette. |
| uint16 | numPalettes | Number of palettes in the table. |
| uint16 | numColorRecords | Total number of color records, combined for all palettes. |
| Offset32 | offsetFirstColorRecord | Offset from the beginning of CPAL table to the first ColorRecord in the Color Records Array. |
| uint16 | colorRecordIndices[numPalettes] | Index of each palette’s first color record in the Color Records Array. |

CPAL version 1

The CPAL header version 1 adds three additional fields to the end of the table header and is organized as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | version | Table version number (=1). |
| uint16 | numPaletteEntries | Number of palette entries in each palette. |
| uint16 | numPalettes | Number of palettes in the table. |
| uint16 | numColorRecords | Total number of color records, combined for all palettes. |
| Offset32 | offsetFirstColorRecord | Offset from the beginning of CPAL table to the first ColorRecord in the Color Records Array. |
| uint16 | colorRecordIndices[numPalettes] | Index of each palette’s first color record in the Color Records Array. |
| Offset32 | offsetPaletteTypeArray | Offset from the beginning of CPAL table to the Palette Type Array. Set to 0 if no array is provided. |
| Offset32 | offsetPaletteLabelArray | Offset from the beginning of CPAL table to the Palette Label Array. Set to 0 if no array is provided. |
| Offset32 | offsetPaletteEntryLabelArray | Offset from the beginning of CPAL table to the Palette Entry Label Array. Set to 0 if no array is provided. |

Palette Entries and Color Records

Colors defined in the CPAL table are referenced by a palette index plus a palette-entry index. Indices are base zero. For a given palette index and palette-entry index, an entry within the color records array is derived: colorRecordIndex = colorRecordIndices[paletteIndex] + paletteEntryIndex.

The color records array is comprised of color records:

| Type | Name | Description |
| --- | --- | --- |
| ColorRecord | colorRecords[numColorRecords] | Color records for all palettes. |

Each color record has BGRA values. The color space for these values is sRGB.

| Type | Name | Description |
| --- | --- | --- |
| uint8 | blue | Blue value (B0). |
| uint8 | green | Green value (B1). |
| uint8 | red | Red value (B2). |
| uint8 | alpha | Alpha value (B3). |

The colors in the Color Record should not be pre-multiplied, and the alpha value should be explicitly set for each palette entry.

When placing and registering overlapping elements, there is the possibility of “seaming”, where the edge rendering of one element interferes with another element. This may be more or less visible based on the contrast of the colors used.

Palette Type Array

| Type | Name | Description |
| --- | --- | --- |
| uint32 | paletteTypes [numPalettes] | Array of 32-bit flag fields that describe properties of each palette. See below for details. |

The following flags are defined:

| Mask | Name | Description |
| --- | --- | --- |
| 0x0001 | USABLE_WITH_LIGHT_ BACKGROUND | Bit 0: palette is appropriate to use when displaying the font on a light background such as white. |
| 0x0002 | USABLE_WITH_DARK_ BACKGROUND | Bit 1: palette is appropriate to use when displaying the font on a dark background such as black. |
| 0xFFFC | Reserved | Reserved for future use — set to 0. |

Note that the usableWithLightBackground and usableWithDarkBackground flags are not mutually exclusive: they may both be set.

Palette Label Array

| Type | Name | Description |
| --- | --- | --- |
| uint16 | paletteLabels [numPalettes] | Array of name table IDs (typically in the font-specific name ID range) that specify user interface strings associated with each palette. Use 0xFFFF if no name ID is provided for a particular palette. |

Palette Entry Label Array

| Type | Name | Description |
| --- | --- | --- |
| uint16 | paletteEntryLabels [numPaletteEntries] | Array of name table IDs (typically in the font-specific name ID range) that specify user interface strings associated with each palette entry, e.g. “Outline”, “Fill”. This set of palette entry labels applies to all palettes in the font. Use 0xFFFF if no name ID is provided for a particular palette entry. |

Relationship to COLR and SVG Tables

Both the COLR and SVG tables can use CPAL to define their palettes.

COLR and CPAL

In fonts that have COLR table, the CPAL table is required, and contains all the font-specified colors used by multicolored glyphs.

As noted in the COLR table description, the palette entry index of 0xFFFF if specified in the COLR table represents the foreground color used in the system. This special value does not change across multiple palettes. The maximum palette entry index is 65535 – 1, as the 65536th position is used in the COLR table to indicate the foreground font color.

SVG and CPAL

In fonts that have an SVG table, the CPAL table can be used to contain the values of any color variables used by the SVG glyph descriptions in the SVG table. SVG glyph descriptions can also include color specifications directly, however. Thus, the CPAL table is optional for fonts with an SVG table.

Foreground color is expressed by the “currentColor” keyword in the SVG glyph descriptions.

When used with an SVG table, the default palette’s colors must be set to the same values as the default values for the color variables in the SVG glyph descriptions; this is for text engines that support the SVG table but not color palettes. The SVG glyph descriptions are able to express their own explicit or “hard-coded” colors as well. These are not related to color variables and thus do not vary by palette selection. See subclause 5.5.1 for more details.

Interpolation of Colors

The SVG table and version 1 of the COLR table both support color gradient fills. The gradients are defined using color stops to specify color values at specific positions along a color line, with color values for other positions on the color line derived by interpolation.

When interpolating color values, linear interpolation between color stop positions is used. For example, suppose adjacent color stops are specified for positions 0.5 and 0.9 on a color line, and a color value is being calculated for position 0.8. The color value of the first color stop will contribute 75% of the value ((0.8 - 0.5) / (0.9 - 0.5)), and the color value of the second color stop will contribute 25% of the value. Interpolated values at each position of the color line are computed in this way for each of the R, G and B color components.

When interpolating color values, specific aspects of the representation of colors as well as handling of alpha need to be considered.

Representations of sRGB color values are expressed as levels of red, green and blue color “primaries” with specific, absolute chromaticity values, which are defined in the sRGB specification. Color-primary levels can potentially be expressed using a linear-light scale that correlates directly to light energy. (On a linear-light scale, for example, a doubling of a color value would correspond to a doubling of display luminance.) For sRGB, however, standard practice is to represent levels using a scale defined by a non-linear transfer function, sometimes referred to as “gamma”. This transfer function is also defined in the sRGB specification. (See CSS Color Module Level 4, section 10.2 [34] for details.) In the CPAL table, sRGB color values are always specified in terms of the non-linear, sRGB transfer function.

NOTE 1	An advantage of representing colors using a non-linear scale is that it allows more effective use of limited bit depth when color-primary levels are represented as integers: smaller differences in light energy can be represented for lower levels than for higher levels. This is beneficial since the human visual system is more sensitive to differences at low luminance levels than to differences at high luminance levels.

When interpolating colors, different results will be obtained if the interpolation is computed using the non-linear scale for color levels than if using the linear-light scale. For interoperable results, whether the non-linear or linear-light scale is to be used needs to be specified.

For gradient color values in the SVG table, the required interpolation behavior is defined in the SVG 1.1 specification: the ‘color-interpolation’ property can be used in an SVG document to declare whether interpolation is done using the non-linear sRGB scale (the default), or using a linear-light scale by applying the inverse sRGB transfer function.

For gradient color values in the COLR table, interpolation shall be computed using linear-light values (i.e., after applying the inverse sRGB transfer function).

After an interpolated color value is computed, whether or not the non-linear sRGB transfer function needs to be re-applied is determined by the requirements of the implementation context.

For both the COLR and SVG tables, interpolation shall be done with alpha pre-multiplied into each linearized R, G and B component. For alpha specified in a CPAL ColorRecord, the value is converted to a floating value in the range [0, 1.0] by dividing by 255, then multiplied into each R, G and B component. In the COLR table, color references in formats used for version 1 include a separate alpha value; that alpha value (with variation, in a variable font) is multiplied into the R, G and B components as well. Interpolated values are then calculated by linear interpolation using these pre-multiplied, linear-light R, G and B values.

NOTE 2	Alpha components use a linear scale and can be directly interpolated apart from the R, G and B components without any linearization step.

Once interpolation of the pre-multiplied red, green and blue values and of the alpha value is complete, the red, green and blue results are then un-premultiplied by dividing each interpolated value by the corresponding interpolated alpha.

While color values are specified as 8-bit integers, the interpolation computations will require greater precision in each of the linearization, pre-multiply, and interpolation steps. Also, when rendered results are to be presented on an imaging device with known characteristics, visual banding artifacts in a gradient can be minimized by taking full advantage of the color bit depth supported by the device. For instance, if a display supports 10- or 12-bit quantization per color channel, then ideally the ramp of color values in a gradient would use that level of quantization. Other factors from the presentation context may, however, also affect the available capabilities. Therefore, no minimum level of precision is specified as a requirement

# Advanced Open Font layout tables

## Advanced Open Font layout extensions

### Overview of advanced typographic layout extensions

The Advanced Typographic tables (OFF Layout tables) extend the functionality of fonts with either TrueType or CFF outlines. OFF Layout fonts contain additional information that extends the capabilities of the fonts to support high-quality international typography:

- OFF Layout fonts allow a rich mapping between characters and glyphs, which supports ligatures, positional forms, alternates, and other substitutions.

- OFF Layout fonts include information to support features for two-dimensional positioning and glyph attachment.

- OFF Layout fonts contain explicit script and language information, so a text-processing application can adjust its behavior accordingly.

- OFF Layout fonts have an open format that allows font developers to define their own typographical features.

This overview introduces the power and flexibility of the OFF Layout font model. The OFF Layout tables are described in more detail in clause 5 "Advanced Open Font Layout Tables".

OFF Layout Common Table Formats are documented in subclause 6.2.

Registered OFF Layout Tags for scripts, languages, and baselines, are documented in subclause 6.4.

OFF layout at a glance

OFF Layout addresses complex typographical issues that especially affect people using text-processing applications in multi-lingual and non-Latin environments.

OFF Layout fonts may contain alternative forms of characters and mechanisms for accessing them. For example, in Arabic, the shape of a character often varies with the character's position in a word. As shown here, the ha character will take any of four shapes, depending on whether it stands alone or whether it falls at the beginning, middle, or end of a word. OFF Layout helps a text-processing application determine which variant to substitute when composing text.

Figure 6.1 – Isolated, initial, medial, and final forms of the Arabic character ha

Similarly, OFF Layout helps an application use the correct forms of characters when text is positioned vertically instead of horizontally, such as with Kanji. For example, Kanji uses alternative forms of parentheses when positioned vertically.

Figure 6.2 – Alternative forms of parentheses used when positioning Kanji vertically

The OFF Layout font format also supports the composition and decomposition of ligatures. For example, English, French, and other languages based on Latin can substitute a single ligature, such as "fi", for its component glyphs - in this case, "f" and "i". Conversely, the individual "f" and "i" glyphs could replace the ligature, possibly to give a text-processing application more flexibility when spacing glyphs to fill a line of justified text.

Figure 6.3 – Two Latin glyphs and their associated ligature

Figure 6.4 – Three Arabic glyphs and their associated ligature

Glyph substitution is just one way OFF Layout extends font capabilities. Using precise X and Y coordinates for positioning glyphs, OFF Layout fonts also can identify points for attaching one glyph to another to create cursive text and glyphs that need diacritical or other special marks.

OFF Layout fonts also may contain baseline information that specifies how to position glyphs horizontally or vertically. Because baselines may vary from one script (set of characters) to another, this information is especially useful for aligning text that mixes glyphs from scripts for different languages.

Figure 6.5 – A line of text, baselines adjusted, mixing Latin and Arabic scripts

### TrueType versus OFF layout

A TrueType font is a collection of several tables that contain different types of data: glyph outlines, metrics, bitmaps, mapping information, and much more. OFF Layout fonts contain all this basic information, plus additional tables containing information for advanced typography.

Text-processing applications - referred to as "clients" of OFF Layout - can retrieve and parse the information in OFF Layout tables. So, for example, a text-processing client can choose the correct character shapes and space them properly.

As much as possible, the tables of OFF Layout define only the information that is specific to the font layout. The tables do not try to encode information that remains constant within the conventions of a particular language or the typography of a particular script. Such information that would be replicated across all fonts in a given language belongs in the text-processing application for that language, not in the fonts.

### OFF layout terminology

The OFF Layout model is organized around glyphs, scripts, language systems, and features.

Characters versus glyphs

Users don't view or print characters: a user views or prints glyphs. A glyph is a representation of a character. The character "capital letter A" is represented by the glyph "A" in Times New Roman Bold and "A" in Arial Bold. A font is a collection of glyphs. To retrieve glyphs, the client uses information in the 'cmap' table of the font, which maps the client's character codes to glyph indices in the table.

Glyphs can also represent combinations of characters and alternative forms of characters: glyphs and characters do not strictly correspond one-to-one. For example, a user might type two characters, which might be better represented with a single ligature glyph. Conversely, the same character might take different forms at the beginning, middle, or end of a word, so a font would need several different glyphs to represent a single character. OFF Layout fonts contain a table that provides a client with information about possible glyph substitutions.

Figure 6.6 – Multiple glyphs for the ampersand character

Scripts

A script is composed of a group of related characters, which may be used by one or more languages. Latin, Arabic, and Thai are examples of scripts. A font may use a single script, or it may use many scripts. Within an OFF Layout font, scripts are identified by unique 4-byte tags.

Figure 6.7 – Glyphs in the Latin, Kanji, and Arabic scripts

Language systems

Scripts, in turn, may be divided into language systems. For example, the Latin script is used to write English, French, or German, but each language has its own special requirements for text processing. A font developer can choose to provide information that is tailored to the script, to the language system, or to both.

Language systems, unlike scripts, are not necessarily evident when a text-processing client examines the characters being used. To avoid ambiguity, the user or the operating system needs to identify the language system. Otherwise, the client will use the default language-system information provided with each script.

Figure 6.8 – Differences in the English, French, and German language system

Features

Features define the basic functionality of the font. A font that contains tables to handle diacritical marks will have a 'mark' feature. A font that supports substitution of vertical glyphs will have a 'vert' feature.

The OFF Layout feature model provides great flexibility to font developers because features do not have to be predefined. Instead, font developers can work with application developers to determine useful features for fonts, add such features to OFF Layout fonts, and enable client applications to support such features.

Figure 6.9 – The relationship of scripts, language systems, features, and lookups for 
substitution and positioning tables

OFF Layout tables

OFF Layout comprises five new tables: GSUB, GPOS, BASE, JSTF, and GDEF. These tables and their formats are discussed in detail in the clauses that follow this overview.

GSUB: Contains information about glyph substitutions to handle single glyph substitution, one-to-many substitution (ligature decomposition), aesthetic alternatives, multiple glyph substitution (ligatures), and contextual glyph substitution.

GPOS: Contains information about X and Y positioning of glyphs to handle single glyph adjustment, adjustment of paired glyphs, cursive attachment, mark attachment, and contextual glyph positioning.

BASE: Contains information about baseline offsets on a script-by-script basis.

JSTF: Contains justification information, including whitespace and Kashida adjustments.

GDEF: Contains information about all individual glyphs in the font: type (simple glyph, ligature, or combining mark), attachment points (if any), and ligature caret (if a ligature glyph).

Common Table Formats: Several common table formats are used by the OFF Layout tables.

### Text processing with OFF layout

A text-processing client follows a standard process to convert the string of characters entered by a user into positioned glyphs. To produce text with OFF Layout fonts:

- Using the cmap table in the font, the client converts the character codes into a string of glyph indices.

- Using information in the GSUB table, the client modifies the resulting string, substituting positional or vertical glyphs, ligatures, or other alternatives as appropriate.

- Using positioning information in the GPOS table and baseline offset information in the BASE table, the client then positions the glyphs.

- Using design coordinates the client determines device-independent line breaks. Design coordinates are high-resolution and device-independent.

- Using information in the JSTF table, the client justifies the lines, if the user has specified such alignment.

- The operating system rasterizes the line of glyphs and renders the glyphs in device coordinates that correspond to the resolution of the output device.

Throughout this process the text-processing client keeps track of the association between the character codes for the original text and the glyph indices of the final, rendered text. In addition, the client may save language and script information within the text stream to clearly associate character codes with typographical behavior.

Left-to-right and right-to-left text

When an OFF text layout engine applies the Unicode bidi algorithm and gets to the point where mirroring needs to be performed on runs with an even, i.e. left-to-right (LTR), resolved level, it does the following:

- Glyph-level mirroring:

Apply feature 'ltrm' to the entire LTR run to substitute mirrored forms.

- LTR glyph alternates:

Apply feature 'ltra' to the entire LTR run to finesse glyph selection.

For runs with an odd, i.e. right-to-left (RTL), resolved level, the engine does the following:

- Character-level mirroring:

For each character i in the RTL run:

If it is mapped to character j by the OMPL and cmap(j) is non-zero:

Use glyph cmap(j) at character i.

Here OMPL refers to the OFF Mirroring Pairs List (see Annex C), and cmap(j) refers to the glyph mapped from code point j in the Unicode cmap table.

For example, suppose U+0028, LEFT PARENTHESIS, occurred in the run at resolved level 1. The glyph at that code point in the run will be replaced by cmap(U+0029), since {U+0028, U+0029} is a pair in the OMPL.

- Glyph-level mirroring:

The engine applies the ‘rtlm’ feature to the entire RTL run. The feature, if present, substitutes mirrored forms for characters other than those covered by the first elements of OMPL pairs (otherwise, it could cancel the effects of character-level mirroring).

The data contents of the OMPL are identical to the Bidi Mirroring Glyph Property file of Unicode 5.1, and will never be revised. Thus, it will be up to the ‘rtlm’ feature to provide, if needed, mirrored forms for both (a) Unicode 5.1 code points with the “mirrored” property but no appropriate Unicode 5.1 character mirrors, as well as (b) all future “mirrored” property additions to Unicode, whether or not character mirrors exist for them.

With such a division of labor between the layout engine and the font, most fonts will not need to include an ‘rtlm’ feature, since the mirrored forms in their Unicode cmap subtable would be adequate.

- RTL glyph alternates:

The engine applies the ‘rtla’ feature to the entire RTL run. The feature, if present, substitutes variants appropriate for right-to-left text (other than mirrored forms).

In practice, the engine may apply features simultaneously; thus, it is up to the font vendor to ensure that the features’ lookups are ordered to achieve the desired effect of the algorithms described above. The engine may optimize its implementation in various ways, e.g. by taking advantage of the fact that character- and glyph-level mirroring won’t both apply on the same element in the run.

### OFF layout and Font variations

OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

Mechanisms used to support font variations are integrated into the tables used for OFF layout. Variation of glyph outlines and metrics across a font’s variation space can impact the design-grid distances that get used in OFF layout tables, such as anchor positions used in a GPOS attachment lookup. Enhancements to OFF layout formats allow the default values found in existing formats to be associated with variation data that describes how the given value is adjusted for different variation instances.

In some variable fonts, it may be desirable to have different glyph-substitution or glyph-positioning actions used for different regions within the font’s variation space. For example, for narrow or heavy instances in which counters become small, it may be desirable to make certain glyph substitutions to use alternate glyphs with certain strokes removed or outlines simplified to allow for larger counters. Such effects can be achieved using a feature variations table within either the GSUB or GPOS table. The feature variations table is described in subclause 6.2. See also the "Required variation alternates ('rvrn')" feature in the OFF layout tag registry.

Different variation instances of a variable font have the same glyph IDs. For that reason, it might seem possible for lookups to be applied across a glyph sequence in which glyphs are formatted using different variation instances of a variable font. Doing so, however, could lead to unpredictable behaviors since font developers may not have sufficient control over how lookup tables are generated, and it would not be feasible to test the vast number of possible cross-instance interactions. For these reasons, layout processing implementations must treat different variation instances of a variable font as distinct style runs for purposes of OFF Layout processing.

## OFF layout common table formats

### Overview

OFF Layout consists of five tables: the Glyph Substitution table (GSUB), the Glyph Positioning table (GPOS), the Baseline table (BASE), the Justification table (JSTF), and the Glyph Definition table (GDEF). These tables use some of the same data formats.

This clause explains the conventions used in all OFF Layout tables, and it describes the common table formats. Separate clauses provide complete details about the GSUB, GPOS, BASE, JSTF, and GDEF tables.

The OFF Layout tables provide typographic information for properly positioning and substituting glyphs, operations that are required for accurate typography in many language environments. OFF Layout data is organized by script, language system, typographic feature, and lookup.

Scripts are defined at the top level. A script is a collection of glyphs used to represent one or more languages in written form (see Figure 6.10). For instance, a single script-Latin is used to write English, French, German, and many other languages. In contrast, three scripts-Hiragana, Katakana, and Kanji-are used to write Japanese. With OFF Layout, multiple scripts may be supported by a single font.

Figure 6.10 – Glyphs in the Latin, Kanji, and Arabic scripts

A language system may modify the functions or appearance of glyphs in a script to represent a particular language. For example, the eszet ligature is used in the German language system, but not in French or English (see Figure 6.11). And the Arabic script contains different glyphs for writing the Farsi and Urdu languages. In OFF Layout, language systems are defined within scripts.

Figure 6.11 – Differences in the English, French, and German language systems

A language system defines features, which are typographic rules for using glyphs to represent a language. Sample features are a "vert" feature that substitutes vertical glyphs in Japanese, a "liga" feature for using ligatures in place of separate glyphs, and a "mark" feature that positions diacritical marks with respect to base glyphs in Arabic (see Figure 6.12). In the absence of language-specific rules, default language system features apply to the entire script. For instance, a default language system feature for the Arabic script substitutes initial, medial, and final glyph forms based on a glyph's position in a word.

Figure 6.12 – A ligature glyph feature substitutes the <etc> ligature for individual glyphs, and a mark feature positions diacritical marks above an Arabic ligature glyph

Features are implemented with lookup data that the text-processing client uses to substitute and position glyphs. Lookups describe the glyphs affected by an operation, the type of operation to be applied to these glyphs, and the resulting glyph output.

A font may also include FeatureVariations data within a GPOS or GSUB table that allows the default lookup data associated with a feature to be substituted by alternate lookup data when particular conditions apply. Currently, this mechanism is used only for variable fonts using OFF Font Variations.

### OFF layout and Font variations

OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

When different variation instances are selected, the design and metrics of individual glyphs changes. This can impact font-unit values given in GPOS, BASE, JSTF or GDEF tables, such as the X and Y coordinates of an attachment anchor position. The font-unit values given in these tables apply to the default instance of a variable font. If adjustments are needed for different variation instances, this is done using variation data with processes similar to those used for glyph outlines and other font data, as described in subclause 7.1. The variation data for GPOS, JSTF or GDEF values is contained in an ItemVariationStore table which, in turn, is contained within the GDEF table; variation data for BASE values is contained in an ItemVariationStore table within the BASE table itself. The format of the ItemVariationStore is described in detail in the subclause 7.2. For font-unit values within the GPOS, BASE, JSTF or GDEF tables that require variation, references to specific variation data within the ItemVariationStore are provided in VariationIndex tables, described below.

In some variable fonts, it may be desirable to have different glyph-substitution or glyph-positioning actions used for different regions within the font’s variation space. For example, for narrow or heavy instances in which counters become small, it may be desirable to make certain glyph substitutions to use alternate glyphs with certain strokes removed or outlines simplified to allow for larger counters. Such effects can be achieved using a FeatureVariations table within either the GSUB or GPOS table. The FeatureVariations table is described below.

### Table organization

Two OFF Layout tables, GSUB and GPOS, use the same data formats to describe the typographic functions of glyphs and the languages and scripts that they support: a ScriptList table, a FeatureList table, a LookupList table, and a FeatureVariations table. In GSUB, the tables define glyph substitution data. In GPOS, they define glyph positioning data. This subclause describes these common table formats.

The ScriptList identifies the scripts in a font, each of which is represented by a Script table that contains script and language-system data. Language system tables reference features, which are defined in the FeatureList. Each feature table references the lookup data defined in the LookupList that describes how, when, and where to implement the feature.

Figure 6.13 – The relationship of scripts, language systems, features, and lookups for substitution and positioning tables

NOTE	The data in the BASE and JSTF tables also is organized by script and language system. However, the data formats differ from those in GSUB and GPOS, and they do not include a FeatureList or LookupList. The BASE and JSTF data formats are described in the BASE and JSTF clauses.

The information used to substitute and position glyphs is defined in Lookup subtables. Each subtable supplies one type of information, depending upon whether the lookup is part of a GSUB or GPOS table. For instance, a GSUB lookup might specify the glyphs to be substituted and the context in which a substitution occurs, and a GPOS lookup might specify glyph position adjustments for kerning. OFF Layout has eight types of GSUB lookups (described in the GSUB subclause) and nine types of GPOS lookups (described in the GPOS subclause).

Each subtable (except for an Extension LookupType subtable) includes a Coverage table that lists the "covered" glyphs that will result in a glyph substitution or positioning operation. The Coverage table formats are described in this clause.

Some substitution or positioning operations may apply to groups, or classes, of glyphs. GSUB and GPOS Lookup subtables use the Class Definition table to assign glyphs to classes. This clause includes a description of the Class Definition table formats.

In non-variable fonts, GPOS lookup subtables may also contain Device tables to adjust scaled contour glyph coordinates for particular output sizes and resolutions. Device tables can also be used for similar adjustments to baseline metric or caret offset values in the BASE and GDEF tables. Similarly, in variable fonts, GPOS lookup subtables, BaseCoord tables and CaretValue tables may contain VariationIndex tables that reference variation data to adjust font-unit values as may be needed for different variation instances within a font’s design variation space. Device and VariationIndex tables are described in this subclause.

As mentioned above, a feature table references a set of lookups in the lookup list. The FeatureVariations table allows the default set of lookups used for a given feature to be substituted by a different set of lookups under particular conditions. This can be used in variable fonts to provide different substitution or positioning actions for different variation instances. For example, for narrow or heavy instances in which counters become small, it may be desirable to make certain glyph substitutions to use alternate glyphs with certain strokes removed or outlines simplified to allow for larger counters.

### Scripts and languages

Three tables and their associated records apply to scripts and languages: the Script List table (ScriptList) and its script record (ScriptRecord), the Script table and its language system record (LangSysRecord), and the Language System table (LangSys).

Script list table and Script record

OFF Layout fonts may contain one or more groups of glyphs used to render various scripts, which are enumerated in a ScriptList table. Both the GSUB and GPOS tables define Script List tables (ScriptList):

- The GSUB table uses the ScriptList table to access the glyph substitution features that apply to a script. For details, see the clause, The Glyph Substitution Table (GSUB).

- The GPOS table uses the ScriptList table to access the glyph positioning features that apply to a script. For details, see the clause, The Glyph Positioning Table (GPOS).

A ScriptList table consists of a count of the scripts represented by the glyphs in the font (ScriptCount) and an array of records (ScriptRecord), one for each script for which the font defines script-specific features (a script without script-specific features does not need a ScriptRecord). Each ScriptRecord consists of a ScriptTag that identifies a script, and an offset to a Script table. The ScriptRecord array is stored in alphabetic order of the script tags.

A Script table with the script tag 'DFLT' (default) may be used in a font to define features that are not script-specific. An application should use a 'DFLT' script table if there is not a script table associated with the specific script of the text being formatted, or if the text does not have a specific script (for example, it contains only symbols or punctuation).

NOTE	If symbols or punctuation have a Unicode script property “Common” but are used together with characters of a specific script, features that apply to those symbol or punctuation characters should not necessarily be organized under the 'DFLT' script, but can be organized under the specific script. Applications may process script-neutral characters together with immediately-preceding or following script-specific characters for better processing efficiency. In that case, an application would look for features that operate on the neutral characters by using the Script table for the specific script. The ‘DFLT’ script would still be used if the text contained only the neutral characters, however.

If there is a 'DFLT' script table, it must have a non-NULL DefaultLangSys value, which provides the offset to a default Language System table (described below). As languages are written using particular scripts, it is normally expected that language-specific typographic effects will be associated with the particular script, not with the generic 'DFLT' script. For this reason, the LangSysCount value of a 'DFLT script table should normally be 0 (no non-default language system tables). However, a font is permitted to have a 'DFLT' script table with non-default language system tables, and an application may use features associated with one of these if the 'DFLT' script table is applicable (no script table is present for the specific script, or there is no specific script in the text context), and if one of the particular language systems is specified. Applications should support use of a non-default language system table that is associated with 'DFLT' script, though some applications might not do so.

The ScriptRecord array stores the records alphabetically by a ScriptTag that identifies the script. Each ScriptRecord consists of a ScriptTag and an offset to a Script table.

Example 1 at the end of this clause shows a ScriptList table and ScriptRecords for a Japanese font that uses three scripts.

ScriptList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | scriptCount | Number of ScriptRecords |
| ScriptRecord | scriptRecords [scriptCount] | Array of ScriptRecords, listed alphabetically by script tag |

ScriptRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | scriptTag | 4-byte script tag identifier |
| Offset16 | scriptOffset | Offset to Script table, from beginning of ScriptList |

Script table and Language System record

A Script table identifies each language system that defines how to use the glyphs in a script for a particular language. It also references a default language system that defines how to use the script's glyphs in the absence of language-specific knowledge.

A Script table begins with an offset to the Default Language System table (defaultLangSys), which defines the set of features that regulate the default behavior of the script. Next, Language System Count (LangSysCount) defines the number of language systems (excluding the DefaultLangSys) that use the script. In addition, an array of Language System Records (LangSysRecord) defines each language system (excluding the default) with an identification tag (LangSysTag) and an offset to a Language System table (LangSys). The LangSysRecord array stores the records alphabetically by LangSysTag.

If no language-specific script behavior is defined, the LangSysCount is set to zero (0), and no LangSysRecords are allocated.

Script table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | defaultLangSys | Offset to default LangSys table, from beginning of Script table – may be NULL |
| uint16 | langSysCount | Number of langSysRecords for this script – excluding the DefaultLangSys |
| LangSysRecord | langSysRecords [langSysCount] | Array of LangSysRecords, listed alphabetically by LangSys tag |

LangSysRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | langSysTag | 4-byte LangSysTag identifier |
| Offset16 | langSysOffset | Offset to LangSys table, from beginning of Script table |

Language System table

The Language System table (LangSys) identifies language-system features used to render the glyphs in a script. (The LookupOrder offset is reserved for future use.)

Optionally, a LangSys table may define a Required Feature Index (ReqFeatureIndex) to specify one feature as required within the context of a particular language system. For example, in the Cyrillic script, the Serbian language system always renders certain glyphs differently than the Russian language system.

Only one feature index value can be tagged as the ReqFeatureIndex. This is not a functional limitation, however, because the feature and lookup definitions in OFF Layout are structured so that one feature table can reference many glyph substitution and positioning lookups. When no required features are defined, then the ReqFeatureIndex is set to 0xFFFF.

All other features are optional. For each optional feature, a zero-based index value references a record (FeatureRecord) in the FeatureRecord array, which is stored in a Feature List table (FeatureList). The feature indices themselves (excluding the ReqFeatureIndex) are stored in arbitrary order in the FeatureIndex array. The FeatureCount specifies the total number of features listed in the FeatureIndex array.

Features are specified in full in the FeatureList table, FeatureRecord, and Feature table, which are described later in this clause. Example 2 at the end of this clause shows a Script table, LangSysRecord, and LangSys table used for contextual positioning in the Arabic script.

LangSys table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | lookupOrder | = NULL (reserved for an offset to a reordering table) |
| uint16 | requiredFeatureIndex | Index of a feature required for this language system; if no required features = 0xFFFF |
| uint16 | featureIndexCount | Number of feature index values for this language system – excludes the required feature |
| uint16 | featureIndices[featureIndexCount] | Array of indices into the FeatureList, in arbitrary order |

### Features and lookups

Features define the functionality of an OFF Layout font and they are named to convey meaning to the text-processing client. Consider a feature named 'liga' to create ligatures. Because of its name, the client knows what the feature does and can decide whether to apply it. For more information, see subclause 6.4. Font developers can use these features, as well as create their own.

After choosing which features to use, the client assembles all lookups from the selected features. Multiple lookups may be needed to define the data required for different substitution and positioning actions, as well as to control the sequencing and effects of those actions.

To implement features, a client applies the lookups in the order the lookup definitions occur in the LookupList. As a result, within the GSUB or GPOS table, lookups from several different features may be interleaved during text processing. A lookup is finished when the client locates a target glyph or glyph context and performs a substitution (if specified) or a positioning (if specified).

NOTE	The substitution (GSUB) lookups always occur before the positioning (GPOS) lookups. The lookup sequencing mechanism in TrueType relies on the font to determine the proper order of text-processing operations.

Lookup data is defined in one or more subtables that contain information about specific glyphs and the operations to be performed on them. Each type of lookup has one or more corresponding subtable definitions. The choice of a subtable format depends upon two factors: the precise content of the information being applied to an operation, and the required storage efficiency. (For complete definitions of all lookup types and subtables, see the GSUB and GPOS clauses of this document.)

OFF Layout features define information that is specific to the layout of the glyphs in a font. They do not encode information that is constant within the conventions of a particular language or the typography of a particular script. Information that would be replicated across all fonts in a given language belongs in the text-processing application for that language, not in the fonts.

Feature list table

The headers of the GSUB and GPOS tables contain offsets to Feature List tables (FeatureList) that enumerate all the features in a font. Features in a particular FeatureList are not limited to any single script. A FeatureList contains the entire list of either the GSUB or GPOS features that are used to render the glyphs in all the scripts in the font.

The FeatureList table enumerates features in an array of records (FeatureRecord) and specifies the total number of features (FeatureCount). Every feature shall have a FeatureRecord, which consists of a FeatureTag that identifies the feature and an offset to a Feature table (described next). The FeatureRecord array is arranged alphabetically by FeatureTag names.

NOTE	The values stored in the FeatureIndex array of a LangSys table are used to locate records in the FeatureRecord array of a FeatureList table.

FeatureList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | featureCount | Number of FeatureRecords in this table |
| FeatureRecord | featureRecords[featureCount] | Array of FeatureRecords – zero-based (first feature has FeatureIndex = 0), listed alphabetically by feature tag |

FeatureRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | featureTag | 4-byte feature identification tag |
| Offset16 | featureOffset | Offset to Feature table, from beginning of FeatureList |

Feature table

A Feature table defines a feature with one or more lookups. The client uses the lookups to substitute or position glyphs.

Feature tables defined within the GSUB table contain references to glyph substitution lookups, and feature tables defined within the GPOS table contain references to glyph positioning lookups. If a text-processing operation requires both glyph substitution and positioning, then both the GSUB and GPOS tables must each define a Feature table, and the tables must use the same FeatureTags.

A Feature table consists of an offset to a Feature Parameters (FeatureParams) table (if one has been defined for this feature - see note in the following paragraph), a count of the lookups listed for the feature (LookupCount), and an arbitrarily ordered array of indices into a LookupList (LookupListIndex). The LookupList indices are references into an array of offsets to Lookup tables.

The format of the Feature Parameters table is specific to a particular feature, and must be specified in the feature's entry in the Feature Tags, see subclause 6.4.3 of the OFF Layout Tag Registry. The length of the Feature Parameters table must be implicitly or explicitly specified in the Feature Parameters table itself. The FeatureParams field in the Feature Table records the offset relative to the beginning of the Feature Table. If a Feature Parameters table is not needed, the FeatureParams field must be set to NULL.

To identify the features in a GSUB or GPOS table, a text-processing client reads the FeatureTag of each FeatureRecord referenced in a given LangSys table. Then the client selects the features it wants to implement and uses the LookupList to retrieve the Lookup indices of the chosen features. Next, the client arranges the indices in the LookupList order. Finally, the client applies the lookup data to substitute or position glyphs.

Example 3 at the end of this clause shows the FeatureList and Feature tables used to substitute ligatures in two languages.

Feature table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | featureParams | Offset to Feature Parameters table (if one has been defined for the feature), relative to the beginning of the Feature Table; = NULL if not required. |
| uint16 | lookupIndexCount | Number of LookupList indices for this feature |
| uint16 | lookupListIndices [lookupIndexCount] | Array of indices into the LookupList – zero-based (first lookup is lookupListIndex = 0) |

Lookup list table

The headers of the GSUB and GPOS tables contain offsets to Lookup List tables (LookupList) for glyph substitution (GSUB table) and glyph positioning (GPOS table). The LookupList table contains an array of offsets to Lookup tables (Lookup). The font developer defines the Lookup sequence in the Lookup array to control the order in which a text-processing client applies lookup data to glyph substitution and positioning operations. LookupCount specifies the total number of Lookup table offsets in the array.

Example 4 at the end of this clause shows three ligature lookups in the LookupList table.

LookupList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | lookupCount | Number of lookups in this table |
| Offset16 | lookups[lookupCount] | Array of offsets to Lookup tables-from beginning of LookupList – zero-based (first lookup is Lookup index = 0) |

Lookup table

A Lookup table (Lookup) defines the specific conditions, type, and results of a substitution or positioning action that is used to implement a feature. For example, a substitution operation requires a list of target glyph indices to be replaced, a list of replacement glyph indices, and a description of the type of substitution action.

Each Lookup table may contain only one type of information (LookupType), determined by whether the lookup is part of a GSUB or GPOS table. GSUB supports eight LookupTypes, and GPOS supports nine LookupTypes (for details about LookupTypes, see the GSUB and GPOS clauses of the document).

Each LookupType is defined with one or more subtables, and each subtable definition provides a different representation format. The format is determined by the content of the information required for an operation and by required storage efficiency. When glyph information is best presented in more than one format, a single lookup may contain more than one subtable, as long as all the subtables are the same LookupType. For example, within a given lookup, a glyph index array format may best represent one set of target glyphs, whereas a glyph index range format may be better for another set of target glyphs.

During text processing, a client applies a lookup to each glyph in the string before moving to the next lookup. A lookup is finished for a glyph after the client makes the substitution/positioning operation. To move to the "next" glyph, the client will typically skip all the glyphs that participated in the lookup operation: glyphs that were substituted/positioned as well as any other glyphs that formed a context for the operation. However, in the case of pair positioning operations (i.e., kerning), the "next" glyph in a sequence may be the second glyph of the positioned pair (see pair positioning lookup for details).

A Lookup table contains a LookupType, specified as an integer, that defines the type of information stored in the lookup. The LookupFlag specifies lookup qualifiers that assist a text-processing client in substituting or positioning glyphs. The subTableCount field specifies the total number of SubTables. The SubTable array specifies offsets, measured from the beginning of the Lookup table, to each SubTable enumerated in the SubTable array.

Lookup table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | lookupType | Different enumerations for GSUB and GPOS |
| uint16 | lookupFlag | Lookup qualifiers |
| uint16 | subTableCount | Number of subtables for this lookup |
| Offset16 | subTableOffsets [subTableCount] | Array of offsets to lookup subtables, from beginning of Lookup table |
| uint16 | markFilteringSet | Index (base 0) into GDEF mark glyph sets structure. This field is present only if bit useMarkFilteringSet of lookup flags is set. |

The LookupFlag uses two bytes of data:

- Each of the first four bits can be set in order to specify additional instructions for applying a lookup to a glyph string. The LookUpFlag bit enumeration table provides details about the use of these bits.

- The fifth bit indicates the presence of a MarkFilteringSet field in the Lookup table.

- The next three bits are reserved for future use.

- The high byte is set to specify the type of mark attachment.

LookupFlag bit enumeration

| Value | Name | Description |
| --- | --- | --- |
| 0x0001 | rightToLeft | This bit relates only to the correct processing of the cursive attachment lookup type (GPOS lookup type 3). When this bit is set, the last glyph in a given sequence to which the cursive attachment lookup is applied, will be positioned on the baseline. NOTE Setting of this bit is not intended to be used by operating systems or applications to determine text direction. |
| 0x0002 | ignoreBaseGlyphs | If set, skips over base glyphs |
| 0x0004 | ignoreLigatures | If set, skips over ligatures |
| 0x0008 | ignoreMarks | If set, skips over all combining marks |
| 0x0010 | useMarkFilteringSet | If set, indicates that the lookup table structure is followed by a MarkFilteringSet field. The layout engine skips over all mark glyphs not in the mark filtering set indicated. |
| 0x00E0 | reserved | For future use (Set to zero) |
| 0xFF00 | markAttachmentType | If not zero, skips over all marks of attachment type different from specified. |

IgnoreBaseGlyphs, IgnoreLigatures, or IgnoreMarks refer to base glyphs, ligatures and marks as defined in the Glyph Class Definition Table in the GDEF table. If any of these flags are set, a Glyph Class Definition Table must be present. If any of these bits is set, then lookups must ignore glyphs of the respective type; that is, the other glyphs must be processed just as though these glyphs were not present.

If MarkAttachmentType is non-zero, then mark attachment classes must be defined in the Mark Attachment Class Definition Table in the GDEF table. When processing glyph sequences, a lookup must ignore any mark glyphs that are not in the specified mark attachment class; only marks of the specified type are processed.

If any lookup has the UseMarkFilteringSet flag set, then the Lookup header must include the MarkFilteringSet field and a MarkGlyphSetsTable must be present in GDEF table. The lookup must ignore any mark glyphs that are not in the specified mark glyph set; only glyphs in the specified mark glyph set are processed.

If a mark filtering set is specified, this supersedes any mark attachment type indication in the lookup flag. If the IgnoreMarks bit is set, this supersedes any mark filtering set or mark attachment type indications.

For example, in Arabic text, a character string might have the pattern <base - mark - base>. That string could be converted into a ligature composed of two components, one for each base character, with the combining mark glyph over the first component. To produce this ligature, the font developer would set the IgnoreMarks bit of the ligature substitution lookup to tell the client to ignore the mark, substitute the ligature glyph first, and then position the mark glyph over the ligature in a subsequent GPOS lookup. Alternatively, a lookup which did not set the IgnoreMarks bit could be used to describe a three-component ligature glyph, composed of the first base glyph, the mark glyph, and the second base glyph.

For another example, a lookup which creates a ligature of a base glyph with a top mark could skip over all bottom marks by specifying the mark attachment type as a class that includes only top marks.

### Coverage table

Each subtable (except an Extension LookupType subtable) in a lookup references a Coverage table (Coverage), which specifies all the glyphs affected by a substitution or positioning operation described in the subtable. The GSUB, GPOS, and GDEF tables rely on this notion of coverage. If a glyph does not appear in a Coverage table, the client can skip that subtable and move immediately to the next subtable.

A Coverage table identifies glyphs by glyph indices (GlyphIDs) either of two ways:

- As a list of individual glyph indices in the glyph set.

- As ranges of consecutive indices. The range format gives a number of start-glyph and end-glyph index pairs to denote the consecutive glyphs covered by the table.

In a Coverage table, a format code (CoverageFormat) specifies the format as an integer: 1 = lists, and 2 = ranges.

A Coverage table defines a unique index value (Coverage Index) for each covered glyph. The Coverage Indexes are sequential, from 0 to the number of covered glyphs minus 1. This unique value specifies the position of the covered glyph in the Coverage table. The client uses the Coverage Index to look up values in the subtable for each glyph.

Coverage Format 1

Coverage Format 1 consists of a format code (CoverageFormat) and a count of covered glyphs (GlyphCount), followed by an array of glyph indices (GlyphArray). The glyph indices must be in numerical order for binary searching of the list. When a glyph is found in the Coverage table, its position in the GlyphArray determines the Coverage Index that is returned-the first glyph has a Coverage Index = 0, and the last glyph has a Coverage Index = GlyphCount -1.

Example 5 at the end of this clause shows a Coverage table that uses Format 1 to list the GlyphIDs of all lowercase descender glyphs in a font.

Coverage Format1 table: Individual glyph indices

| Type | Name | Description |
| --- | --- | --- |
| uint16 | coverageFormat | Format identifier – format = 1 |
| uint16 | glyphCount | Number of glyphs in the glyph array |
| uint16 | glyphArray[glyphCount] | Array of glyph IDs – in numerical order |

Coverage Format 2

Format 2 consists of a format code (coverageFormat) and a count of glyph index ranges (rangeCount), followed by an array of records (rangeRecords). Each RangeRecord consists of a start glyph index (startGlyphID), an end glyph index (endGlyphID), and the Coverage Index associated with the range's Start glyph. Ranges shall be in glyph ID order, and they must be distinct, with no overlapping.

The Coverage Indexes for the first range begin with zero (0), and increase sequentially to (endGlyphId - startGlyphId). For each successive range, the starting Coverage Index is one greater than the ending Coverage Index of the preceding range. Thus, startCoverageIndex for each non-initial range must equal the length of the preceding range (endGlyphID - startGlyphID + 1) added to the startGlyphIndex of the preceding range. This allows for a quick calculation of the Coverage Index for any glyph in any range using the formula: Coverage Index (glyphID) = startCoverageIndex + glyphID - startGlyphID.

Example 6 at the end of this clause shows a Coverage table that uses Format 2 to identify a range of numeral glyphs in a font.

CoverageFormat2 table: Range of glyphs

| Type | Name | Description |
| --- | --- | --- |
| uint16 | coverageFormat | Format identifier – format = 2 |
| uint16 | rangeCount | Number of RangeRecords |
| RangeRecord | rangeRecords [rangeCount] | Array of glyph ranges – ordered by startGlyphID |

RangeRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | startGlyphID | First glyph ID in the range |
| uint16 | endGlyphID | Last glyph ID in the range |
| uint16 | startCoverageIndex | Coverage Index of first glyph ID in range |

### Class definition table

In OFF Layout, index values identify glyphs. For efficiency and ease of representation, a font developer can group glyph indices to form glyph classes. Class assignments vary in meaning from one lookup subtable to another. For example, in the GSUB and GPOS tables, classes are used to describe glyph contexts. GDEF tables also use the idea of glyph classes.

Consider a substitution action that replaces only the lowercase ascender glyphs in a glyph string. To more easily describe the appropriate context for the substitution, the font developer might divide the font's lowercase glyphs into two classes, one that contains the ascenders and one that contains the glyphs without ascenders.

A font developer can assign any glyph to any class, each identified with an integer called a class value. A Class Definition table (ClassDef) groups glyph indices by class, beginning with Class 1, then Class 2, and so on. All glyphs not assigned to a class fall into Class 0. Within a given class definition table, each glyph in the font belongs to exactly one class.

The ClassDef table can have either of two formats: one that assigns a range of consecutive glyph indices to different classes, or one that puts groups of consecutive glyph indices into the same class.

Class Definition Table Format 1

The first class definition format (ClassDefFormat1) specifies a range of consecutive glyph indices and a list of corresponding glyph class values. This table is useful for assigning each glyph to a different class because the glyph indices in each class are not grouped together.

A ClassDef Format 1 table begins with a format identifier (ClassFormat). The range of glyph IDs covered by the table is identified by two values: the glyph ID of the first glyph (StartGlyphID), and the number of consecutive glyph IDs (including the first one) that will be assigned class values (GlyphCount). The ClassValueArray lists the class value assigned to each glyph ID, starting with the class value for StartGlyphID and following the same order as the glyph IDs. Any glyph not included in the range of covered glyph IDs automatically belongs to Class 0.

Example 7 at the end of this clause uses Format 1 to assign class values to the lowercase, x-height, ascender, and descender glyphs in a font.

ClassDefFormat1 table: Class array

| Type | Name | Description |
| --- | --- | --- |
| uint16 | classFormat | Format identifier-format = 1 |
| uint16 | startGlyphID | First glyph ID of the classValueArray |
| uint16 | glyphCount | Size of the classValueArray |
| uint16 | classValueArray[glyphCount] | Array of Class Values – one per glyph ID |

Class Definition Table Format 2

The second class definition format (ClassDefFormat2) defines multiple groups of glyph indices that belong to the same class. Each group consists of a discrete range of glyph indices in consecutive order (ranges cannot overlap).

The ClassDef Format 2 table contains a format identifier (ClassFormat), a count of ClassRangeRecords that define the groups and assign class values (ClassRangeCount), and an array of ClassRangeRecords ordered by the glyph ID of the first glyph in each record (ClassRangeRecord).

Each ClassRangeRecord consists of a Start glyph index, an End glyph index, and a Class value. All GlyphIDs in a range, from Start to End inclusive, constitute the class identified by the Class value. Any glyph not covered by a ClassRangeRecord is assumed to belong to Class 0.

Example 8 at the end of this clause uses Format 2 to assign class values to four types of glyphs in the Arabic script.

ClassDefFormat2 table: Class ranges

| Type | Name | Description |
| --- | --- | --- |
| uint16 | classFormat | Format identifier – format = 2 |
| uint16 | classRangeCount | Number of ClassRangeRecords |
| ClassRangeRecord | classRangeRecords [classRangeCount] | Array of ClassRangeRecords – ordered by startGlyphID |

ClassRangeRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | startGlyphID | First glyph ID in the range |
| uint16 | endGlyphID | Last glyph ID in the range |
| uint16 | class | Applied to all glyphs in the range |

### Device and VariationIndex tables

Device tables and VariationIndex tables are used to provide adjustments to font-unit values in GPOS, JSTF, GDEF or BASE tables, such as the X and Y coordinates of an attachment anchor position. Device tables are used only in non-variable fonts. VariationIndex tables are used only in variable fonts and are a variant format of the Device table. When values require adjustment data, the table containing that value will also include an offset to a Device table or VariationIndex table.

NOTE	Because the same fields are used to provide an offset to a Device table or an offset to a VariationIndex table, Device tables and VariationIndex tables cannot both be used for a given positioning value. Device tables should only be used in non-variable fonts; VariationIndex tables can only be used in variable fonts.

Glyphs in a font are defined in design units specified by the font developer. Font scaling increases or decreases a glyph's size and rounds it to the nearest whole pixel. However, precise glyph positioning often requires adjustment of these scaled and rounded values, particularly at small PPEM sizes. Hinting, applied to points in the glyph outline, is an effective solution to this problem, but it may require the font developer to redesign or re-hint glyphs.

Another solution, used by the GPOS, BASE, JSTF, and GDEF tables in non-variable fonts, is to use a Device table to specify correction values to adjust the scaled design units. A Device table applies the correction values to the range of sizes identified by StartSize and EndSize, which specify the smallest and largest pixel-per-em (ppem) sizes needing adjustment.

Because Device table adjustments often are very small (a pixel or two), the correction can be compressed into a 2-, 4-, or 8-bit representation per size. Two bits can represent a number in the range {-2, -1, 0, or 1}, four bits can represent a number in the range {-8 to 7}, and eight bits can represent a number in the range {-128 to 127}.

In variable fonts, X or Y font-unit values in GPOS, JSTF, or GDEF data may require adjustment for different variation instances within a font’s variation space. The variation data for this is contained in an ItemVariationStore table contained within the GDEF table. Similarly, values in a BASE table may require adjustment, and the variation data for this is contained in an ItemVariationStore table within the BASE table. The format of the ItemVariationStore is described in detail in subclause 7.2. It contains a number of delta values organized into sets that are referenced using a delta-set index. Data stored outside the ItemVariationStore provides delta-set indices for each of target items requiring variation. Within the GPOS, JSTF, GDEF and BASE tables, delta-set indices are stored in VariationIndex tables.

The Device and VariationIndex tables contain a DeltaFormat field that identifies the format of data contained. Format values 0x0001 to 0x0003 are used for Device tables, and indicate the format of delta adjustment values contained directly within the device table: signed 2-, 4,- or 8-bit values. A format value of 0x8000 is used for the VariationIndex table, and indicates that a delta-set index is used to reference delta data in an ItemVariationStore table.

DeltaFormat values

| Type | Name | Description |
| --- | --- | --- |
| 0x0001 | LOCAL_2_BIT_DELTAS | Signed 2-bit value, 8 values per uint16 |
| 0x0002 | LOCAL_4_BIT_DELTAS | Signed 4-bit value, 4 values per uint16 |
| 0x0003 | LOCAL_8_BIT_DELTAS | Signed 8-bit value, 2 values per uint16 |
| 0x8000 | VARIATION_INDEX | VariationIndex table, contains a delta-set index pairs. |
| 0x7FFC | Reserved | For future use – set to 0. |

The Device table includes an array of uint16 values (DeltaValue) that stores the adjustment delta values in a packed representation. The 2-, 4-, or 8-bit signed values are packed into uint16 values starting with the most significant bits first. For example, using a DeltaFormat of 2 (4-bit values), an array of values equal to {1, 2, 3, -1} would be represented by the DeltaValue 0x123F.

A single Device table provides delta information for one target value at a range of sizes. The DeltaValue array lists the number of pixels to adjust specified X or Y values at each ppem size in the targeted range. In the array, the first index position specifies the number of pixels to add or subtract from the coordinate at the smallest ppem size that needs correction, the second index position specifies the number of pixels to add or subtract from the coordinate at the next ppem size, and so on for each ppem size in the range.

Device table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | startSize | Smallest size to correct-in ppem |
| uint16 | endSize | Largest size to correct-in ppem |
| uint16 | deltaFormat | Format of deltaValue array data: 0x0001, 0x0002, or 0x0003 |
| uint16 | deltaValue[ ] | Array of compressed data |

Example 9 at the end of this clause uses a Device table to define the minimum extent value for a math script.

In a variable font, the ItemVariationStore table uses a two-level organization for variation data: a store can have multiple Item Variation Data subtables, and each subtable has multiple delta-set rows. A delta-set index is a two-part index: an outer index that selects a particular item variation data subtable, and an inner index that selects a particular delta-set row within that subtable. A VariationIndex table specifies both the outer and inner portions of the delta-set index.

VariationIndex table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | deltaSetOuterIndex | A delta-set outer index – used to select an item variation data subtable within the item variation store. |
| uint16 | deltaSetInnerIndex | A delta-set inner index – used to select a delta-set row within an item variation data subtable. |
| uint16 | deltaFormat | Format, = 0x8000 |

Note that the VariationIndex table is shorter than the Device table since it does not directly contain an array of delta data. Its format is similar to a Device table with an empty delta array. When applications get an offset to a Device or VariationIndex table, they should begin by reading the first three fields and then testing the DeltaFormat field to determine the interpretation of the first two fields and whether there is additional data to read.

### Feature variations

FeatureVariations Table

A feature variations table describes variations on the effects of features based on various conditions. That is, it allows the default set of lookups for a given feature to be substituted with alternates of lookups under particular conditions.

The feature list provides an array of feature tables and associated feature tags, and a LangSys table identifies a particular set of the feature-table/tag pairs that will be supported for a given script and language system. The feature tables specified in a LangSys table are used by default when current conditions do not match any of the conditions for variation defined in the feature variations table. Those defaults will also be used under all conditions in implementations that do not support the feature variations table.

The feature variations table has an array of condition records, each of which references a set of conditions (a condition set table), and a set of alternate feature tables to use when a runtime context matches the condition set.

The substitutions given are replacements of one feature table for another. The alternate feature tables are appended at the end of the feature variations table, and are not included in the feature list table. Hence, there are no feature records in the feature list table that correspond to the alternate feature tables. An alternate feature table maintains the same feature tag association as the default feature table. Also, whereas the default feature tables in the feature list table are referenced using 16-bit offsets, the alternate feature tables are referenced using 32-bit offsets within the feature variations table.

When processing text, a default set of feature tables, each with an associated feature tag, is obtained from a LangSys table for a given script and language system. Condition sets are evaluated in order, testing for a condition set that matches the current runtime context. When the first match is found, the corresponding feature table substitution table is used to revise the set of feature tables obtained by default via the LangSys table, as described below (see FeatureTableSubstitution table).

The format of the feature variations table is as follows:

FeatureVariations Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the FeatureVariations table – set to 1. |
| uint16 | minorVersion | Minor version of the FeatureVariations table – set to 0. |
| uint32 | featureVariationRecordCount | Number of feature variation records. |
| FeatureVariationRecord | featureVariationRecords [featureVariationRecordCount] | Array of feature variation records. |

A feature variation record has offsets to a condition set table and to a feature table substitution table.

If the ConditionSet offset is 0, there is no condition set table. This is treated as the universal condition: all contexts are matched.

If the FeatureTableSubstitution offset is 0, there is no feature table substitution table, and no substitutions are made.

Feature variation records shall be ordered in the order of precedence for the condition sets. During processing, the feature variation records will be read, and the corresponding condition sets tested, in the order in which they occur. If the condition set for a given record does not match the runtime context, then the next record is checked. The first feature variation record for which the condition set matches the runtime context will be considered as a candidate: if the version of the FeatureTableSubstitution table is supported, then this feature variation record will be used, and no additional feature variation records will be considered. If the version of the FeatureTableSubtitution table is not supported, then this feature variation record is rejected and processing will move to the next feature variation record.

FeatureVariationRecord

| Type | Name | Description |
| --- | --- | --- |
| Offset32 | conditionSet | Offset to a condition set table, from beginning of FeatureVariations table. |
| Offset32 | featureTableSubstitutionOffset | Offset to a feature table substitution table, from beginning of the FeatureVariations table. |

ConditionSet Table

A condition set table specifies a set of conditions under which a feature table substitution is to be applied. A condition set may specify conditions related to various factors; currently, one type of factor is supported: the variation instance of a variable font. Individual conditions are represented in subtables, which may use different formats according to the nature of the factor defining the condition.

For a given condition set, conditions are conjunctively related (boolean AND): all of the specified conditions must be met in order for the associated feature table substitution to be applied. A condition set does not need to specify conditional values for all possible factors. If no values are specified for some factor, then the condition set matches all runtime values for that factor.

If a given condition set contains no conditions, then it matches all contexts, and the associated feature table substitution is always applied, unless there was a FeatureVariation record earlier in the array with a condition set matching the current context.

ConditionSet Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | conditionCount | Number of conditions for this condition set. |
| uint32 | conditions[conditionCount] | Array of offsets to condition tables, from beginning of the ConditionSet table. |

Condition Table

The condition table describes a particular condition. Different formats for the condition table may be defined, with each format used for a particular kind of condition qualifier. Currently, one format is defined: ConditionTableFormat1, which is used to specify a value range for a variation axis value in a variable font.

New formats for other condition qualifiers may be added in the future, in which case the version of the ConditionSet table will be updated. If a layout engine supports a particular version of the condition set table and encounters a font with a later-version condition set table, it should fail to match the condition set whenever an unrecognized condition format is encountered. In this way, new condition formats can be defined and used in fonts that can work in a backward-compatible way in existing implementations. Therefore, introduction of new condition formats will typically result in minor version updates to the ConditionSet table.

Condition Table Format 1: Font Variation Axis Range

A font variation axis range condition refers to a range of values for a design variation axis in a variable font. The axes of variation are specified in the font variations ('fvar') table of a font. If a format 1 condition table is used, there must be an fvar table in the font, and the AxisIndex value (which is zero-based) must be less than the axisCount value in the fvar table. If the AxisIndex is invalid, the feature variation record containing this condition table is ignored.

A format 1 condition table specifies a matching range of variation instance values along a single axis. Absence of a format 1 condition for a given variation axis implies that that axis is not a factor in determining applicability of the condition set.

The fvar table defines a range of valid values for each variation axis. During processing for a particular variation instance, a normalization process is applied that maps user values in the range defined within the fvar table to a normalized scale with a range from -1 to 1. The values specified in a format 1 condition table are expressed in terms of the normalized scale, and so can be any value from -1 to 1.

A font variation axis range condition is met if the currently-selected variation instance has a value for the given axis that is greater than or equal to the FilterRangeMinValue, and that is less than or equal to the FilterRangeMaxValue.

ConditionTableFormat1

| Type | Name | Description |
| --- | --- | --- |
| uint16 | Format | Format, = 1 |
| uint16 | AxisIndex | Index (zero-based) for the variation axis within the 'fvar' table. |
| F2DOT14 | FilterRangeMinValue | Minimum value of the font variation instances that satisfy this condition. |
| F2DOT14 | FilterRangeMaxValue | Maximum value of the font variation instances that satisfy this condition. |

FeatureTableSubstitution Table

A feature table substitution table describes a set of feature table substitutions to be applied when the corresponding condition set matches the current runtime context. These substitutions are represented using an array of feature table substitution records. Each record gives a simple substitution of one feature table for another. When checking for a particular feature index, the first record having that index is matched, and searching ends if a record is encountered with a higher index value.

Note that the records shall be ordered in increasing order of the FeatureIndex values, and no two records may have the same FeatureIndex value.

FeatureTableSubstitution table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the feature table substitution table – set to 1 |
| uint16 | minorVersion | Minor version of the feature table substitution table – set to 0. |
| uint16 | substitutionCount | Number of feature table substitution records. |
| FeatureTableSubstitutionRecord | substitutions [substitutionCount] | Array of feature table substitution records. |

FeatureTableSubstitutionRecord:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | featureIndex | The feature table index to match. |
| Offset32 | alternateFeatureTable | Offset to an alternate feature table, from start of the FeatureTableSubstitution table. |

As described above, condition sets are evaluated and may be selected for processing of the associated feature table substitution table to replace a default feature table obtained from a LangSys table with an alternate feature table. Given a default array of feature tables for selected features obtained from the LangSys table, substitution of alternate feature tables can be done as follows:

- For each feature index, evaluate the FeatureTableSubstitutionRecords in order.

- If a matching record is encountered (FeatureIndex = the current feature index), then replace the feature table for that feature index using the alternate feature table at the offset given in the record. Stop processing for that feature index.

- If a record is encountered with a higher feature index value, stop searching for that feature index; no substitution is made.

### Common table examples

The rest of this clause describes and illustrates examples of all the common table formats. All the examples reflect unique parameters, but the samples provide a useful reference for building tables specific to other situations.

The examples have three columns showing hex data, source, and comments.

Example 1: ScriptList table and ScriptRecords

Example 1 illustrates a ScriptList table and ScriptRecord definitions for a Japanese font with multiple scripts: Han Ideographic, Kana, and Latin. Each script has script-specific behavior.

Example 1

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | ScriptList TheScriptList | ScriptList table definintion |
| 0003 | 3 | scriptCount |
|  | scriptRecords[0] | In alphabetical order by script tag. |
| 68616E69 | 'hani' | scriptTag, Han Ideographic script |
| 0014 | HanIScriptTable | offset to Script table |
|  | scriptRecords[1] | In alphabetical order by script tag. |
| 6B616E61 | 'kana' | scriptTag, Hiragana and Katakana scripts |
| 0018 | KanaScriptTable | offset to Script table |
|  | scriptRecords[2] | In alphabetical order by script tag. |
| 6C61746E | 'latn' | scriptTag, Latin script |
| 001C | LatinScriptTable | offset to Script table |

Example 2: Script Table, LangSysRecord, and LangSys table

Example 2 illustrates the Script table, LangSysRecord, and LangSys table definitions for the Arabic script and the Urdu language system. The default LangSys table defines three default Arabic script features used to replace certain glyphs in words with their proper initial, medial, and final glyph forms. These contextual substitutions are invariant and occur in all language systems that use the Arabic script.

Many alternative glyphs in the Arabic script have language-specific uses. For instance, the Arabic, Farsi, and Urdu language systems use different glyphs for numerals. To maintain character-set compatibility, the Unicode Standard includes separate character codes for the Arabic and Farsi numeral glyphs. However, the standard uses the same character codes for Farsi and Urdu numerals, even though three of the Urdu glyphs (4, 6, and 7) differ from the Farsi glyphs. To access and display the proper glyphs for the Urdu numerals, users of the text-processing client must enter the character codes for the Farsi numerals. Then the text-processing client uses a required OFF Layout glyph substitution feature, defined in the Urdu LangSys table, to access the correct Urdu glyphs for the 4, 6, and 7 numerals.

NOTE	The Urdu LangSys table repeats the default script features. This repetition is necessary because the Urdu language system also uses alternative glyphs in the initial, medial, and final glyph positions in words.

Example 2

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | Script ArabicScriptTable | Script table definition |
| 000A | DefLangSys | offset to DefaultLangSys table |
| 0001 | 1 | langSysCount |
|  | langSysRecords[0] | In alphabetical order by LangSys tag. |
| 55524420 | 'URD ' | langSysTag, Urdu language |
| 0016 | UrduLangSys | offset to LangSys table for Urdu |
|  | LangSys DefLangSys | default LangSys table definition |
| 0000 | NULL | lookupOrder, reserved, null |
| FFFF | 0xFFFF | requiredFeatureIndex, no required features |
| 0003 | 3 | featureIndexCount |
| 0000 | 0 | featureIndices[0], in arbitrary order 'init' feature (initial glyph) |
| 0001 | 1 | featureIndices [1], 'fina' feature (final glyph) |
| 0002 | 2 | featureIndices [2], for 'medi' feature (medial glyph) |
|  | LangSys UrduLangSys | LangSys table definition |
| 0000 | NULL | lookupOrder, reserved, null |
| 0003 | 3 | requiredFeatureIndex, numeral subsitution in Urdu |
| 0003 | 3 | featureIndexCount |
| 0000 | 0 | featureIndices [0], in arbitrary order 'init' feature (initial glyph) |
| 0001 | 1 | featureIndices [1], 'fina' feature (final glyph) |
| 0002 | 2 | featureIndices [2], 'medi' feature (medial glyph) |

Example 3: FeatureList table and Feature table

Example 3 shows the FeatureList and Feature table definitions for ligatures in the Latin script. The FeatureList has three features, all optional and named 'liga'. One feature, also a default, implements ligatures in Latin if no language-specific feature specifies other ligatures. Two other features implement ligatures in the Turkish and German languages, respectively.

Three lookups define glyph substitutions for rendering ligatures in this font. The first lookup produces the "ffi" and "fi" ligatures; the second produces the "ffl", "fl", and "ff" ligatures; and the third produces the eszet ligature.

The ligatures that begin with an "f" are separated into two sets because Turkish has a dotless "i" glyph and so does not use "ffi" and "fi" ligatures. However, Turkish does use the "ffl", "fl", and "ff" ligatures, and the TurkishLigatures feature table lists this one lookup.

Only the German language system uses the eszet ligature, so the GermanLigatures feature table includes a lookup for rendering that ligature.

Because the Latin script can use both sets of ligatures, the DefaultLigatures feature table defines two LookupList indices: one for the "ffi" and "fi" ligatures, and one for the "ffl", "fl", and "ff" ligatures. If the text-processing client selects this feature, then the font applies both lookups.

NOTE	The TurkishLigatures and DefaultLigatures feature tables both list a LookupListIndex of one (1) for the "ffl", "fl", and "ff" ligatures lookup. This is because language-specific lookups override all default language-system lookups, and a language-system feature table must explicitly list all lookups that apply to the language.

Example 3

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | FeatureList TheFeatureList | FeatureList table definition |
| 0003 | 3 | featureCount |
|  | featureRecords[0] |  |
| 6C696761 | 'liga' | featureTag |
| 0014 | TurkishLigatures | offset to Feature table, FflFfFlLiga |
|  | featureRecords[1] |  |
| 6C696761 | 'liga' | featureTag |
| 001A | DefaultLigatures | offset to Feature table, FfiFiLiga, FflFfFlLiga |
|  | featureRecords[2] |  |
| 6C696761 | 'liga' | featureTag |
| 0022 | GermanLigatures | Offset to Feature table, EszetLiga |
|  | Feature TurkishLigatures | Feature table definition |
| 0000 | NULL | featureParams, reserved, null |
| 0001 | 1 | lookupIndexCount |
| 0000 | 1 | lookupListIndices[1], ffl, fl, ff ligature substitution Lookup |
|  | Feature DefaultLigatures | Feature table definition |
| 0000 | NULL | featureParams - reserved, null |
| 0002 | 2 | lookupIndexCount |
| 0000 | 0 | lookupListIndices [0], in arbitrary order, ffi, fi ligatures |
| 0001 | 1 | lookupListIndices [1], ffl, fl, ff ligature substitution Lookup |
|  | Feature GermanLigatures | Feature table definition |
| 0000 | NULL | featureParams - reserved, null |
| 0001 | 3 | lookupIndexCount |
| 0000 | 0 | lookupListIndices [0], in arbitrary order, ffi, fi ligatures |
| 0001 | 1 | lookupListIndices [1], ffl, fl, ff ligature substitution Lookup |
| 0002 | 2 | lookupListIndices [2], eszet ligature substitution Lookup |

Example 4: LookupList table and Lookup table

A continuation of Example 3, Example 4 shows three ligature lookups in the LookupList table. The first generates the "ffi" and "fi" ligatures; the second produces the "ffl", "fl", and "ff" ligatures; and the third generates the eszet ligature. Each lookup table defines an offset to a subtable that contains data for the ligature substitution.

Example 4

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | LookupList TheLookupList | LookupList table definition |
| 0003 | 3 | lookupCount |
| 0008 | FfiFiLookup | offset to lookups[0] table, in design order |
| 0010 | FflFlFfLookup | offset to lookups[1] table |
| 0018 | EszetLookup | offset to lookups[2] table |
|  | Lookup FfiFiLookup | lookups[0] table definition |
| 0004 | 4 | lookupType, ligature subst |
| 000C | 0x000C | lookupFlag, IgnoreLigatures, IgnoreMarks |
| 0001 | 1 | subTableCount |
| 0018 | FfiFiSubtable | offset to FfiFi ligature substitution subtable |
|  | Lookup FflFlFfLookup | lookups[1] table definition |
| 0004 | 4 | lookupType: ligature subst |
| 000C | 0x000C | LookupFlag- IgnoreLigatures, IgnoreMarks |
| 0001 | 1 | subTableCount |
| 0028 | FflFlFfSubtable | offset to FflFlFf ligature substitution subtable |
|  | Lookup EszetLookup | lookups[2] table definition |
| 0004 | 4 | lookupType: ligature subst |
| 000C | 0x000C | lookupFlag: IgnoreLigatures, IgnoreMarks |
| 0001 | 1 | subTableCount |
| 0038 | EszetSubtable | offset to Eszet ligature substitution subtable |

Example 5: CoverageFormat1 table (glyph ID list)

Example 5 illustrates a Coverage table that lists the GlyphIDs of all lowercase descender glyphs in a font. The table uses the list format instead of the range format because the GlyphIDs for the descender glyphs are not consecutively ordered.

Example 5

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | CoverageFormat1 DescenderCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: glyph ID list |
| 0005 | 5 | glyphCount |
| 0038 | gGlyphID | glyphArray[0], in glyph ID order |
| 003B | jGlyphID | glyphArray[1] |
| 0041 | pGlyphID | glyphArray[2] |
| 0042 | qGlyphID | glyphArray[3] |
| 004A | yGlyphID | glyphArray[4] |

Example 6: CoverageFormat2 table (glyph ID ranges)

Example 6 shows a Coverage table that defines ten numeral glyphs (0 through 9). The table uses the range format instead of the list format because the glyph IDs are ordered consecutively in the font. The StartCoverageIndex of zero (0) indicates that the first glyph ID, for the zero glyph, returns a Coverage Index of 0. The second glyph ID, for the numeral one (1) glyph, returns a Coverage Index of 1, and so on.

Example 6

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | CoverageFormat2 NumeralCoverage | Coverage table definition |
| 0002 | 2 | coverageFormat: glyph ID ranges |
| 0001 | 1 | rangeCount |
|  | rangeRecord[0] |  |
| 004E | 0glyphID | startGlyphID |
| 0057 | 9glyphID | endGlyphID |
| 0000 | 0 | StartCoverageIndex, first CoverageIndex = 0 |

Example 7: ClassDefFormat1 table (Class array)

The ClassDef table in Example 7 assigns class values to the lowercase glyphs in a font. The x-height glyphs are in Class 0, the ascender glyphs are in Class 1, and the descender glyphs are in Class 2. The array begins with the index for the lowercase "a" glyph.

Example 7

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | ClassDefFormat1 LowercaseClassDef | ClassDef table definition |
| 0001 | 1 | classFormat: class array |
| 0032 | aGlyphID | startGlyph |
| 001A | 26 | glyphCount |
|  | classValueArray |  |
| 0000 | 0 | aGlyph, Xheight Class 0 |
| 0001 | 1 | bGlyph, Ascender Class 1 |
| 0000 | 0 | cGlyph, Xheight Class 0 |
| 0001 | 1 | dGlyph, Ascender Class 1 |
| 0000 | 0 | eGlyph, Xheight Class 0 |
| 0001 | 1 | fGlyph, Ascender Class 1 |
| 0002 | 2 | gGlyph, Descender Class 2 |
| 0001 | 1 | hGlyph, Ascender Class 1 |
| 0000 | 0 | iGlyph, Ascender Class 1 |
| 0002 | 2 | jGlyph, Descender Class 2 |
| 0001 | 1 | kGlyph, Ascender Class 1 |
| 0001 | 1 | lGlyph, Ascender Class 1 |
| 0000 | 0 | mGlyph, Xheight Class 0 |
| 0000 | 0 | nGlyph, Xheight Class 0 |
| 0000 | 0 | oGlyph, Xheight Class 0 |
| 0002 | 2 | pGlyph, Descender Class 2 |
| 0002 | 2 | qGlyph, Descender Class 2 |
| 0000 | 0 | rGlyph, Xheight Class 0 |
| 0000 | 0 | sGlyph, Xheight Class 0 |
| 0001 | 1 | tGlyph, Ascender Class 1 |
| 0000 | 0 | uGlyph, Xheight Class 0 |
| 0000 | 0 | vGlyph, Xheight Class 0 |
| 0000 | 0 | wGlyph, Xheight Class 0 |
| 0000 | 0 | xGlyph, Xheight Class 0 |
| 0002 | 2 | yGlyph, Descender Class 2 |
| 0000 | 0 | zGlyph, Xheight Class 0 |

Example 8: ClassDefFormat2 table (Class ranges)

In Example 8, the ClassDef table assigns class values to four types of glyphs in the Arabic script: medium-height base glyphs, high base glyphs, very high base glyphs, and default mark glyphs. The table lists only Class 1, Class 2, and Class 3; all glyphs not explicitly assigned a class fall into Class 0.

The table uses the range format because the glyph IDs in each class are ordered consecutively in the font. In the ClassRange array, ClassRange definitions are ordered by the Start glyph index in each range. The indices of the high base glyphs, defined in ClassRange[0], are first in the font and have a class value of 2. ClassRange[1] defines all the very high base glyphs and assigns a class value of 3. ClassRange[2] contains all default mark glyphs; the class value is 1. Class 0 consists of all the medium-height base glyphs, which are not explicitly assigned a class value.

Example 8

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | ClassDefFormat2 GlyphHeightClassDef | Class table definition |
| 0002 | 2 | classFormat: ranges |
| 0003 | 3 | classRangeCount |
|  | classRangeRecords[0] | ordered by startGlyphID |
| 0030 | tahGlyphID | startGlyphID – first glyph ID in the range |
| 0031 | dhahGlyphID | endGlyphID – last glyph ID in the range |
| 0002 | 2 | class: high base glyphs |
|  | classRangeRecords[1] |  |
| 0040 | cafGlyphID | startGlyphID |
| 0041 | gafGlyphID | endGlyphID |
| 0003 | 3 | class: very high base glyphs |
|  | classRangeRecords[2] |  |
| 00D2 | fathatanDefaultGlyphID | startGlyphID |
| 00D3 | dammatanDefaultGlyphID | endGlyphID |
| 0001 | 1 | class: default marks |

Example 9: Device table

Example 9 defines the minimum extent value for a math script, using a Device table to adjust the value according to the size of the output font. Here, the Device table defines single-pixel adjustments for font sizes from 11 ppem to 15 ppem. The DeltaFormat is 1, which signifies a packed array of signed 2-bit values, eight values per uint16.

Example 9

| Hex Data | Source | Comment |
| --- | --- | --- |
|  | DeviceTableFormat1 MinCoordDeviceTable | Device Table definition |
| 000B | 11 | startSize: 11 ppem |
| 000F | 15 | endSize: 15 ppem |
| 0001 | 1 | deltaFormat: signed 2 bit value (8 values per uint16) |
|  | 1 | increase 11ppem by 1 pixel |
|  | 1 | increase 12ppem by 1 pixel |
|  | 1 | increase 13ppem by 1 pixel |
|  | 1 | increase 14ppem by 1 pixel |
| 5540 | 1 | increase 15ppem by 1 pixel |

## Advanced typographic tables

There are also several optional tables that support vertical layout as well as other advanced typographic functions:

Advanced Typographic Tables

| Tag | Name |
| --- | --- |
| BASE | Baseline data |
| GDEF | Glyph definition data |
| GPOS | Glyph positioning data |
| GSUB | Glyph substitution data |
| JSTF | Justification data |
| MATH | Math layout data |

### BASE—Baseline table

#### BASE table overview

The Baseline table (BASE) provides information used to align glyphs of different scripts and sizes in a line of text, whether the glyphs are in the same font or in different fonts. To improve text layout, the Baseline table also provides minimum (min) and maximum (max) glyph extent values for each script, language system, or feature in a font.

Lines of text composed with glyphs of different scripts and point sizes need adjustment to correct interline spacing and alignment. For example, glyphs designed to be the same point size often differ in height and depth from one font to another (see Figure 6.14). This variation can produce interline spacing that looks too large or too small, and diacritical marks, math symbols, subscripts, and superscripts may be clipped.

Figure 6.14 – Incorrect alignment of glyphs from Latin and Kanji (Latin dominant)

In addition, different baselines can cause text lines to waver visually as glyphs from different scripts are placed next to one another. For example, ideographic scripts position all glyphs on a low baseline. With Latin scripts, however, the baseline is higher, and some glyphs descend below it. Finally, several Indic scripts use a high "hanging baseline" to align the tops of the glyphs.

To solve these composition problems, the BASE table recommends baseline positions and min/max extents for each script (see Figure 6.15). Script min/max extents can be modified for particular language systems or features.

Figure 6.15 – Proper alignment of glyphs from Latin and Kanji (Latin dominant)

Baseline values

The BASE table uses a model that assumes one script at one size is the "dominant run" during text processing – that is, all other baselines are defined in relation to this the dominant run.

For example, Latin glyphs and the ideographic Kanji glyphs have different baselines. If a Latin script of a particular size is specified as the dominant run, then all Latin glyphs of all sizes will be aligned on the roman baseline, and all Kanji glyphs will be aligned on the lower ideographic baseline defined for use with Latin text. As a result, all glyphs will look aligned within each line of text.

The BASE table supplies recommended baseline positions; a client can specify others. For instance, the client may want to assign baseline positions different from those in the font.

Figure 6.16 – Comparing Latin and Kanji baselines, with characters aligned according 
to the dominant run

Min/Max Extent values

The BASE table gives clients the option of using script, language system, or feature-specific extent values to improve composition (see Figure 6.16). For example, suppose a font contains glyphs in Latin and Arabic scripts, and the min/max extents defined for the Arabic script are larger than the Latin extents. The font also supports Urdu, a language system that includes specific variants of the Arabic glyphs, and some Urdu variants require larger min/max extents than the default Arabic extents. To accommodate the Urdu glyphs, the BASE table can define language-specific min/max extent values that will override the default Arabic extents-but only when rendering Urdu glyphs.

The BASE table also can define feature-specific min/max values that apply only when a particular feature is enabled. Suppose that the font described earlier also supports the Farsi language system, which has one feature that requires a minor alteration of the Arabic script extents to display properly. The BASE table can specify these extent values and apply them only when that feature is enabled in the Farsi language.

#### BASE table and OFF Font Variations

OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

When different variation instances are selected, the design of individual glyphs changes, and the metric characteristics of the font as a whole may also change. As a result, corresponding changes may also be required for metric values in the BASE table.

Metrics in the BASE table are expressed directly in BaseCoord tables using explicit X or Y font-unit values. In a variable font, these X and Y values apply to the default instance and may need to be adjusted for the current variation instance. This is done using variation data with processes similar to those used for glyph outlines and other font data, as described in the OFF Font Variations Overview chapter.

NOTE	Some BASE metrics can be expressed indirectly by reference to specific glyph outline points. In a variable font, use of glyph points to specify a metric value would require invoking the rasterizer to process the glyph-outline variation data in order to obtain the adjusted position of the point before the BASE metric value can be used. This may have a significant, negative impact on performance of text-layout processing. For this reason, it is recommended that, in a variable font, any BASE metric values that require adjustment for different variation instances should always be expressed directly as X and Y values.

Variation data for adjustment of BASE values is stored within an item variation store table within the BASE table. The item variation store and constituent formats are described in subclause7.2. The item variation store is also used in the 'GDEF' table, as well as in the 'MVAR' and other tables, but is different from the formats for variation data used in the 'cvar' or 'gvar' tables.

The variation data within an item variation store is comprised of a number of adjustment deltas that get applied to the default values of target items for variation instances within particular regions of the font’s variation space. The item variation store format use delta-set indices to reference variation delta data for particular target, font-data items to which they are applied. Data external to the item variation store identifies the delta-set index to be used for each given target item. Within the BASE table, these indices are specified within VariationIndex tables, with one VariationIndex table referenced for each item that requires variation adjustment.

Note that the VariationIndex table is a variant of a Device table, with a distinct format value. (For full details on the Device and VariationIndex table formats, see subclause 6.2.) This is done so that the default instance of a variable font can be compatible with applications that do not support Font Variations. As a result, variable fonts cannot use device tables. A VariationIndex table will be ignored in applications that do not support Font Variations, or if the font is not a variable font.

The item variation store format uses a two-level organization for variation data: a store can have multiple item variation data subtables, and each subtable has multiple delta-set rows. A delta-set index is a two-part index: an outer index that selects a particular item variation data subtable, and an inner index that selects a particular delta-set row within that subtable. A VariationIndex table specifies both the outer and inner portions of the delta-set index.

#### BASE table organization

The BASE table begins with offsets to Axis tables that describe layout data for the horizontal and vertical layout directions of text. A font can provide layout data for both text directions or for only one text direction:

- The Horizontal Axis table (HorizAxis) defines information used to lay out text horizontally. All baseline and min/max values refer to the Y direction.

- The Vertical Axis table (VertAxis) defines information used to lay out text vertically. All baseline and min/max values refer to the X direction.

NOTE	The same baseline tags can be used for both horizontal and vertical axes. For example, the 'romn' tag description used for the vertical axis would indicate the baseline of rotated Latin text.

The figure below shows how the BASE table is organized.

Figure 6.17 – High-level organization of BASE table

Text direction

The HorizAxis and VertAxis tables organize layout information by script in BaseScriptList tables. A BaseScriptList enumerates all scripts in the font that are written in a particular direction (horizontal or vertical).

For example, consider a Japanese font that contains Kanji, Kana, and Latin scripts. Because all three scripts are rendered horizontally, all three are defined in the BaseScriptList of the HorizAxis table. Kanji and Kana also are rendered vertically, so those two scripts are defined in the BaseScriptList of the VertAxis table, too.

Baseline data

Each Axis table also references a BaseTagList, which identifies all the baselines for all scripts written in the same direction (horizontal or vertical). The BaseTagList may also include baseline tags for scripts supported in other fonts.

Each script in a BaseScriptList is represented by a BaseScriptRecord. This record references a BaseScript table, which contains layout data for the script. In turn, the BaseScript table references a BaseValues table, which contains baseline information and several MinMax tables that define min/max extent values.

The BaseValues table specifies the coordinate values for all baselines in the BaseTagList. In addition, it identifies one of these baselines as the default baseline for the script. As glyphs in a script are scaled, they grow or shrink from the script's default baseline position. Each baseline can have unique coordinates. This contrasts with TrueType 1.0, which implies a single, fixed baseline for all scripts in a font. With the OFF Layout tables, each script can be aligned independently, although more than one script may use the same baseline values.

Baseline coordinates for scripts in the same font must be specified in relation to each other for correct alignment of the glyphs. Consider the font, discussed earlier, containing both Latin and Kanji glyphs. If the BaseTagList of the HorizAxis table specifies two baselines, the roman and the ideographic, then the layout data for both the Latin and Kanji scripts will specify coordinate positions for both baselines:

- The BaseValues table for the Latin script will give coordinates for both baselines and specify the roman baseline as the default.

- The BaseValues table for the Kanji script will give coordinates for both baselines and specify the ideographic baseline as the default.

Min/Max extents

The BaseScript table can define minimum and maximum extent values for each script, language system, or feature. (These values are distinct from the min/max extent values recorded for the font as a whole in the head, hhea, vhea, and OS/2 tables.) These extent values appear in three tables:

- The DefaultMinMax table defines the default min/max extents for the script.

- A MinMax table, referenced through a BaseLangSysRecord, specifies min/max extents to accommodate the glyphs in a specific language system.

- A FeatMinMaxRecord, referenced from the MinMax table, provides min/max extent values to support feature-specific glyph actions.

NOTE	Language-system or feature-specific extent values may be essential to define some fonts. However, the default min/max extent values specified for each script should usually be enough to support high-quality text layout.

The actual baseline and min/max extent values used by the BASE table reside in BaseCoord tables. Three formats are defined for BaseCoord table data. All formats define single X or Y coordinate values in design units, but two formats support fine adjustments to these values based on a contour point or a Device table.

The rest of this clause describes all the tables defined within the BASE table. Sample tables and lists that illustrate typical data for a font are supplied at the end of the clause.

#### BASE table structure

BASE header

The BASE table begins with a header that starts with a version number. Two versions are defined. Version1.0 contains offsets to horizontal and vertical Axis tables (HorizAxis and VertAxis). Version 1.1 also includes an offset to an Item Variation Store table.

Each Axis table stores all baseline information and min/max extents for one layout direction. The HorizAxis table contains Y values for horizontal text layout; the VertAxis table contains X values for vertical text layout.

A font may supply information for both layout directions. If a font has values for only one text direction, the Axis table offset value for the other direction will be set to NULL.

The optional Item Variation Store table is used in variable fonts to provide variation data for BASE metric values within the Axis tables.

Example 1 at the end of this clause shows a sample BASE Header.

BASE Header, Version 1.0

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the BASE table, =1 |
| uint16 | minorVersion | Minor version of the BASE table, =0 |
| Offset16 | horizAxisOffset | Offset to horizontal Axis table-from beginning of BASE table (may be NULL) |
| Offset16 | vertAxisOffset | Offset to vertical Axis table-from beginning of BASE table (may be NULL) |

BASE Header, Version 1.1

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the BASE table, = 1 |
| uint16 | minorVersion | Minor version of the BASE table, = 1 |
| Offset16 | horizAxisOffset | Offset to horizontal Axis table, from beginning of BASE table (may be NULL) |
| Offset16 | vertAxisOffset | Offset to vertical Axis table, from beginning of BASE table (may be NULL) |
| Offset32 | itemVarStoreOffset | Offset to Item Variation Store table, from beginning of BASE table (may be null) |

Axis tables: HorizAxis and VertAxis

An Axis table is used to render scripts either horizontally or vertically. It consists of offsets, measured from the beginning of the Axis table, to a BaseTagList and a BaseScriptList:

- The BaseScriptList enumerates all scripts rendered in the text layout direction.

- The BaseTagList enumerates all baselines used to render the scripts in the text layout direction. If no baseline data is available for a text direction, the offset to the corresponding BaseTagList may be set to NULL.

Example 1 at the end of this clause shows an example of an Axis table.

Axis Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | baseTagListOffset | Offset to BaseTagList table, from beginning of Axis table (may be NULL) |
| Offset16 | baseScriptListOffset | Offset to BaseScriptList table, from beginning of Axis table |

BaseTagList table

The BaseTagList table identifies the baselines for all scripts in the font that are rendered in the same text direction. Each baseline is identified with a 4-byte baseline tag. The Baseline Tags  of the OFF Tag Registry lists currently registered baseline tags. The BaseTagList can define any number of baselines, and it may include baseline tags for scripts supported in other fonts.

Each script in the BaseScriptList table must designate one of these BaseTagList baselines as its default, which the OFF Layout Services use to align all glyphs in the script. Even though the BaseScriptList and the BaseTagList are defined independently of one another, the BaseTagList typically includes a tag for each different default baseline needed to render the scripts in the layout direction. If some scripts use the same default baseline, the BaseTagList needs to list the common baseline tag only once.

The BaseTagList table consists of an array of baseline identification tags (baselineTag), listed alphabetically, and a count of the total number of baseline Tags in the array (baseTagCount).

Example 1 at the end of this clause shows a sample BaseTagList table.

BaseTagList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseTagCount | Number of baseline identification tags in this text direction – may be zero (0) |
| Tag | baselineTags[baseTagCount] | Array of 4-byte baseline identification tags – must be in alphabetical order |

BaseScriptList table

The BaseScriptList table identifies all scripts in the font that are rendered in the same layout direction. If a script is not listed here, then the text-processing client will render the script using the layout information specified for the entire font.

For each script listed in the BaseScriptList table, a BaseScriptRecord must be defined that identifies the script and references its layout data. BaseScriptRecords are stored in the baseScriptRecords array, ordered alphabetically by the baseScriptTag in each record. The baseScriptCount specifies the total number of BaseScriptRecords in the array.

Example 1 at the end of this clause shows a sample BaseScriptList table.

BaseScriptList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseScriptCount | Number of BaseScriptRecords defined |
| BaseScriptRecord | baseScriptRecords[baseScriptCount] | Array of BaseScriptRecords, in alphabetical order by baseScriptTag |

BaseScriptRecord

A BaseScriptRecord contains a script identification tag (baseScriptTag), which must be identical to the ScriptTag used to define the script in the ScriptList of a GSUB or GPOS table. Each record also must include an offset to a BaseScript table that defines the baseline and min/max extent data for the script.

Example 1 at the end of this clause shows a sample BaseScriptRecord.

BaseScriptRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | baseScriptTag | 4-byte script identification tag |
| Offset16 | baseScriptOffset | Offset to BaseScript table, from beginning of BaseScriptList |

BaseScript table

A BaseScript table organizes and specifies the baseline data and min/max extent data for one script. Within a BaseScript table, the BaseValues table contains baseline information, and one or more MinMax tables contain min/max extent data.

The BaseValues table identifies the default baseline for the script and lists coordinate positions for each baseline named in the corresponding BaseTagList. Each script can assign a different position to each baseline, so each script can be aligned independently in relation to any other script. (For more details, see the BaseValues table description later in this clause.)

The default MinMax table defines the default min/max extent values for the script. (For details, see the MinMax table description below.) If a language system or feature defined in the font has no effect on the script's default min/max extents, the OFF Layout Services will use the default script values.

Sometimes language-specific overrides for min/max extents are needed to properly render the glyphs in a specific language system. For example, a glyph substitution required in a language system may result in a glyph whose extents exceed the script's default min/max extents. Each language system that specifies min/max extent values must define a BaseLangSysRecord. The record should identify the language system (baseLangSysTag) and contain an offset to a MinMax table of language-specific extent coordinates.

Feature-specific overrides for min/max extents also may be needed to accommodate the effects of glyph actions used to implement a specific feature. For example, superscript or subscript features may require changes to the default script or language system extents. Feature-specific extent values not limited to a specific language system may be specified in the default MinMax table. However, extent values used for a specific language system require a BaseLangSysRecord and a MinMax table. In addition to specifying coordinate data, the MinMax table must contain offsets to FeatMinMaxRecords that define the feature-specific min/max data.

A BaseScript table has four components:

- An offset to a BaseValues table (baseValuesOffset). If no baseline data is defined for the script or the corresponding BaseTagList is set to NULL, the offset to the BaseValues table may be set to NULL.

- An offset to the default MinMax table. If no default min/max extent data is defined for the script, this offset may be set to NULL.

- An array of BaseLangSysRecords (baseLangSysRecords). The individual records stored in the BaseLangSysRecord array are listed alphabetically by baseLangSysTag.

- A count of the BaseLangSysRecords included (baseLangSysCount). If no language system or language-specific feature min/max values are defined, the baseLangSysCount may be set to zero (0).

Example 2 at the end of this clause shows a sample BaseScript table.

BaseScript Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | baseValuesOffset | Offset to BaseValues table-from beginning of BaseScript table-may be NULL |
| Offset16 | defaultMinMaxOffset | Offset to MinMax table, from beginning of BaseScript table (may be NULL) |
| uint16 | baseLangSysCount | Number of BaseLangSysRecords defined – may be zero (0) |
| BaseLangSysRecord | baseLangSysRecords [baseLangSysCount] | Array of BaseLangSysRecords, in alphabetical order by BaseLangSysTag |

BaseLangSysRecord

A BaseLangSysRecord defines min/max extents for a language system or a language-specific feature. Each record contains an identification tag for the language system (baseLangSysTag) and an offset to a MinMax table (MinMax) that defines extent coordinate values for the language system and references feature-specific extent data.

Example 2 at the end of this clause shows a BaseLangSysRecord.

BaseLangSysRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | baseLangSysTag | 4-byte language system identification tag |
| Offset16 | minMaxOffset | Offset to MinMax table, from beginning of BaseScript table |

BaseValues table

A BaseValues table lists the coordinate positions of all baselines named in the baselineTags array of the corresponding BaseTagList and identifies a default baseline for a script.

NOTE	When the offset to the corresponding BaseTagList is NULL, a BaseValues table is not needed. However, if the offset is not NULL, then each script must specify coordinate positions for all baselines named in the BaseTagList.

The default baseline, one per script, is the baseline used to lay out and align the glyphs in the script. The defaultBaselineIndex in the BaseValues table identifies the default baseline with a value that equals the array index position of the corresponding tag in the baselineTags array.

For example, the Han and Latin scripts use different baselines to align text. If a font supports both of these scripts, the BaselineTag array in the BaseTagList of the HorizAxis table will contain two tags, listed alphabetically: 'ideo' in baselineTags[0] for the Han ideographic baseline, and 'romn' in baselineTags[1] for the Latin baseline. The BaseValues table for the Latin script will specify the roman baseline as the default, so the defaultBaselineIndex in the BaseValues table for Latin will be "1" to indicate the roman baseline tag. In the BaseValues table for the Han script, the defaultBaselineIndex will be "0" to indicate the ideographic baseline tag.

Two or more scripts may share a default baseline. For instance, if the font described above also supports the Cyrillic script, the baselineTags array does not need a baseline tag for Cyrillic because Cyrillic and Latin share the same baseline. The defaultBaselineIndex defined in the BaseValues table for the Cyrillic script will specify "1" to indicate the roman baseline tag, listed in the second position in the baselineTags array.

In addition to identifying the defaultBaselineIndex, the BaseValues table contains an offset to an array of BaseCoord tables (baseCoords) that list the coordinate positions for all baselines, including the default baseline, named in the associated baselineTags array. One BaseCoord table is defined for each baseline. The baseCoordCount field defines the total number of BaseCoord tables, which must equal the number of baseline tags listed in baseTagCount in the BaseTagList.

Each baseline coordinate is defined as a single X or Y value in design units measured from the zero position on the relevant X or Y axis. For example, a BaseCoord table defined in the HorizAxis table will contain a Y value because horizontal baselines are positioned vertically. BaseCoord values may be negative. Each script may assign a different coordinate to each baseline.

Offsets to each BaseCoord table are stored in the baseCoords array within the BaseValues table. The order of the stored offsets corresponds to the order of the tags listed in the baselineTags array of the BaseTagList. In other words, the first entry in the baseCoords array will define the offset to the BaseCoord table for the first baseline named in the baselineTags array, the second position will define the offset to the BaseCoord table for the second baseline named in the baselineTags array, and so on.

Example 3 at the end of the clause has two parts, one that shows a BaseValues table and one that shows a chart with different baseline positions defined for several scripts.

BaseValues table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | defaultBaselineIndex | Index number of default baseline for this script – equals index position of baseline tag in BaselineArray of the BaseTagList |
| uint16 | baseCoordCount | Number of BaseCoord tables defined – should equal BaseTagCount in the BaseTagList |
| Offset16 | baseCoords[baseCoordCount] | Array of offsets to BaseCoord, from beginning of BaseValues table – order matches baselineTags array in the BaseTagList |

The MinMax table and FeatMinMaxRecord

The MinMax table specifies extents for scripts and language systems. It also contains an array of FeatMinMaxRecords used to define feature-specific extents.

Both the MinMax table and the FeatMinMaxRecord define offsets to two BaseCoord tables: one that defines the mimimum extent value (minCoord), and one that defines the maximum extent value (maxCoord). Each extent value is a single X or Y value, depending upon the text direction, and is specified in design units. Coordinate values may be negative.

Different tables define the min/max extent values for scripts, language systems, and features:

- Min/max extent values for a script are defined in the default MinMax table, referenced in a BaseScript table.

- Within the default MinMax table, FeatMinMaxRecords can specify extent values for features that apply to the entire script.

- Min/max extent values for a language system are defined in the MinMax table, referenced in a BaseLangSysRecord.

- FeatMinMaxRecords can be defined within the MinMax table to specify extent values for features applied within a language system.

In a FeatMinMaxRecord, the minCoord and maxCoord tables specify the minimum and maximum coordinate values for the feature, and a featureTableTag defines a 4-byte feature identification tag. The featureTableTag shall match the tag used to identify the feature in the FeatureList of the GSUB or GPOS table.

Each feature that exceeds the default min/max values requires a FeatMinMaxRecord. All FeatMinMaxRecords are listed alphabetically by featureTableTag in an array (featMinMaxRecords) within the MinMax table. The featMinMaxCount field defines the total number of FeatMinMaxRecords.

Text-processing clients should use the following procedure to access the script, language system, and feature-specific extent data:

- Determine script extents in relation to the text content.

- Select language-specific extent values with respect to the language system in use.

- Have the application or user choose feature-specific extent values.

- If no extent values are defined for a language system or for language-specific features, use the default min/max extent values for the script.

Example 4 at the end of this clause has two parts. One shows MinMax tables and a FeatMinMaxRecord for different script, language system, and feature extents. The second part shows how to define these tables when a language system needs feature-specific extent values for an obscure feature, but otherwise the language system and script extent values match.

MinMax table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | minCoord | Offset to BaseCoord table that defines the minimum extent value, from the beginning of MinMax table (may be NULL) |
| Offset16 | maxCoord | Offset to BaseCoord table that defines the maximum extent value, from the beginning of MinMax table (may be NULL) |
| uint16 | featMinMaxCount | Number of FeatMinMaxRecords – may be zero (0) |
| FeatMinMaxRecord | featMinMaxRecords [featMinMaxCount] | Array of FeatMinMaxRecords, in alphabetical order by featureTableTag |

FeatMinMaxRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | featureTableTag | 4-byte feature identification tag – must match FeatureTag in FeatureList |
| Offset16 | minCoord | Offset to BaseCoord table that defines the minimum extent value, from beginning of MinMax table (may be NULL) |
| Offset16 | maxCoord | Offset to BaseCoord table that defines the maximum extent value, from beginning of MinMax table (may be NULL) |

BaseCoord tables

Within the BASE table, a BaseCoord table defines baseline and min/max extent values. Each BaseCoord table defines one X or Y value:

- If defined within the HorizAxis table, then the BaseCoord table contains a Y value.

- If defined within the VertAxis table, then the BaseCoord table contains an X value.

All values are defined in design units, which typically are scaled and rounded to the nearest integer when scaling the glyphs. Values may be negative.

Three formats available for BaseCoord table data define single X or Y coordinate values in design units. Two of the formats also support fine adjustments to the X or Y values based on a contour point or a Device table. In a variable font, the third format uses a VariationIndex table (a variant of a Device table), as needed, to reference variation data for adjustment of the X or Y values for the current variation instance.

BaseCoord Format 1

The first BaseCoord format (BaseCoordFormat1) consists of a format identifier, followed by a single design unit coordinate that specifies the BaseCoord value. This format has the benefits of small size and simplicity, but the BaseCoord value cannot be hinted for fine adjustments at different sizes or device resolutions.

Example 5 at the end of the clause shows a sample of a BaseCoordFormat1 table.

BaseCoordFormat1 table: Design units only

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseCoordFormat | Format identifier – format = 1 |
| int16 | coordinate | X or Y value, in design units |

BaseCoord Format 2

The second BaseCoord format (BaseCoordFormat2) specifies the BaseCoord value in design units, but also supplies a glyph index and a contour point for reference. During font hinting, the contour point on the glyph outline may move. The point's final position after hinting provides the final value for rendering a given font size.

NOTE	Glyph positioning operations defined in the GPOS table do not affect the point's final position.

Example 6 shows a sample of a BaseCoordFormat2 table.

BaseCoordFormat2 table: Design units plus contour point

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseCoordFormat | Format identifier – format = 2 |
| int16 | coordinate | X or Y value, in design units |
| uint16 | referenceGlyph | Glyph ID of control glyph |
| uint16 | baseCoordPoint | Index of contour point on the reference glyph |

BaseCoord Format 3

The third BaseCoord format (BaseCoordFormat3) also specifies the BaseCoord value in design units, but, in a non-variable font, it uses a Device table rather than a contour point to adjust the value. This format offers the advantage of fine-tuning the BaseCoord value for any font size and device resolution. (For more information about Device tables, see the clause, Common Table Formats.)

In a variable font, BaseCoordFormat3 shall be used to reference variation data to adjust the X or Y value for different variation instances, if needed. In this case, BaseCoordFormat3 specifies an offset to a VariationIndex table, which is a variant of the Device table that is used for referencing variation data.

NOTE 1	While separate VariationIndex table references are required for each Coordinate value that requires variation, two or more values that require the same variation-data values can have offsets that point to the same VariationIndex table, and two or more VariationIndex tables can reference the same variation data entries.

NOTE 2	If no VariationIndex table is used for a particular X or Y value (the offset is zero, or a different BaseCoord format is used), then that value is used for all variation instances.

Example 7 at the end of this clause shows a sample of a BaseCoordFormat3 table.

BaseCoordFormat3 table: Design units plus Device or VariationIndex table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseCoordFormat | Format identifier – format = 3 |
| int16 | coordinate | X or Y value, in design units |
| Offset16 | deviceTable | Offset to Device table (non-variable font) / VariationIndex table (variable font) for X or Y value, from beginning of BaseCoord table (may be NULL). |

Item Variation Store Table

The format and processing of the ItemVariationStore table and its constituent formats is described in subclause 7.2. Specification of the interpolation algorithm used to derive values for particular variation instances is given in subclause 7.1.

The ItemVariationStore contains adjustment-delta values arranged in one or more sets of deltas that are referenced using delta-set indices. For values that requires variation adjustment, a delta-set index is used to reference the particular variation data needed for that target value. Within the BASE table, delta-set indices are provided in VariationIndex tables contained within a BaseCoordFormat3 table. For a description of the VariationIndex table, see the OFF Layout Common Table Formats chapter. For details on use of VariationIndex tables within BaseCoord tables, see discussion earlier in this chapter.

#### BASE table examples

The rest of this clause describes and illustrates examples of all the BASE tables. All the examples reflect unique parameters described below, but the samples provide a useful reference for building tables specific to other situations.

Most of the examples have three columns showing hex data, source, and comments.

Example 1: BASE header table, Axis table, BaseTagList table, BaseScriptList table, and BaseScriptRecord

Example 1 describes a sample font that contains four scripts: Cyrillic, Devanagari, Han, and Latin. All four scripts are rendered horizontally; only one script, Han, is rendered vertically. As a result, the BASE header gives offsets to two Axis tables: HorizAxis and VertAxis. Example 1 only shows data defined in the HorizAxis table.

In the HorizAxis table, the BaseScriptList enumerates all four scripts. The BaseTagList table names three horizontal baselines for rendering these scripts: hanging, ideographic, and roman. The hanging baseline is the default for Devanagari, the ideographic baseline is the default for Han, and the roman baseline is the default for both Latin and Cyrillic.

The VertAxis table (not shown) would be defined similarly: its BaseScriptList would enumerate one script, Han, and its BaseTagList would specify the vertically centered baseline for rendering the Han script.

Example 1

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BASEHeader TheBASEHeader | BASE table header definition |
| 00010000 | 0x00010000 | Version |
| 0008 | HorizontalAxisTable | Offset to HorizAxis table |
| 010C | VerticalAxisTable | Offset to VertAxis table |
|  | Axis HorizontalAxisTable | Axis table definition |
| 0004 | HorizBaseTagList | Offset to BaseTagList table |
| 0012 | HorizBaseScriptList | Offset to BaseScriptList table |
|  | BaseTagList HorizBaseTagList | BaseTagList table definition |
| 0003 | 3 | baseTagCount |
| 68616E67 | 'hang' | baselineTags[0], in alphabetical order |
| 6964656F | 'ideo' | baselineTags[1] |
| 726F6D6E | 'romn' | baselineTags[2] |
|  | BaseScriptList HorizBaseScriptList | BaseScriptList table definition |
| 0004 | 4 | baseScriptCount |
|  | baseScriptRecords[0] | Records in alphabetical order by baseScriptTag |
| 6379726C | 'cyrl' | baseScriptTag: Cyrillic script |
| 001A | HorizCyrillicBaseScriptTable | Offset to BaseScript table for Cyrillic script |
|  | baseScriptRecords[1] |  |
| 6465766E | 'devn' | baseScriptTag: Devanagari script |
| 0060 | HorizDevanagariBaseScriptTable | Offset to BaseScript table for Devanagari script |
|  | baseScriptRecords[2] |  |
| 68616E69 | 'hani' | baseScriptTag: Han script |
| 008A | HorizHanBaseScriptTable | Offset to BaseScript table for Han script |
|  | baseScriptRecords[3] |  |
| 6C61746E | 'latn' | baseScriptTag: Latin script |
| 00B4 | HorizLatinBaseScriptTable | Offset to BaseScript table for Latin script |

Example 2: BaseScript table and BaseLangSysRecord

Example 2 shows the BaseScript table and BaseLangSysRecord for the Cyrillic script, one of the four scripts included in the sample font described in Example 1. The BaseScript table specifies offsets to tables that contain the baseline and min/max extent data for Cyrillic. (The BaseScript tables for the other three scripts in the font would be defined similarly.) Again, the table specifies only the horizontal text-layout information.

The HorizCyrillicBaseValues table contains the baseline information for the script, and the HorizCyrillicDefaultMinMax table contains the default script extents. In addition, a BaseLangSysRecord defines min/max extent data for the Russian language system.

Example 2

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseScript HorizCyrillicBaseScriptTable | BaseScript table definition for Cyrillic script |
| 000C | HorizCyrillicBaseValuesTable | Offset to BaseValues table |
| 0022 | HorizCyrillicDefault MinMaxTable | Offset to default MinMax table – default script extents |
| 0001 | 1 | baseLangSysCount – BaseLangSysRecords for language-specific extents |
|  | baseLangSysRecords[0] | Records in alphabetical order by baseLangSysTag. |
| 52555320 | 'RUS ' | BaseLangSysTag, Russian language system |
| 0030 | HorizRussianMinMaxTable | Offset to MinMax table – language-specific extents |

Example 3: BaseValues table

Example 3 extends the BASE table definition for the Cyrillic script described in Examples 1 and 2. It contains two parts:

- Example 3A illustrates a fully defined BaseValues table for Cyrillic. The table includes the corresponding BaseCoord table definitions.

- Example 3B shows two different sets of baseline values that can be defined for each of the four scripts in the sample font.

The examples show only horizontal text-layout data, and the font uses 2,048 design units/em.

Example 3A: BaseValues table for Cyrillic

The BaseValues table of Example 3A identifies the default baseline for Cyrillic and specifies coordinate positions for each baseline listed in the BaseTagList shown in Example 1:

- The hanging baseline is the default for the Devanagari script, and it has the highest baseline position.

- The ideographic baseline is the default for the Han script, and it has the lowest baseline position.

- The roman baseline is the default for both the Latin and Cyrillic scripts, and its position lies between the hanging and ideographic baselines.

Example 3A

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseValues HorizCyrillicBaseValuesTable | BaseValues table definition for Cyrillic script |
| 0002 | 2 | defaultBaselineIndex: roman baseline baselineTag index |
| 0003 | 3 | baseCoordCount, equals baseTagCount |
| 000A | HorizHangingBaseCoordForCyrl | Offset to baseCoords[0] table: hanging baseline coordinate – order matches order of baselineTags array in BaseTagList |
| 000E | HorizIdeographicBaseCoordForCyrl | Offset to baseCoords[1] table: ideographic baseline coordinate |
| 0012 | HorizRomanBaseCoordForCyrl | Offset to baseCoords[2] table: roman baseline coordinate |
|  | BaseCoordFormat1 HorizHangingBaseCoordForCyrl | BaseCoord table definition |
| 0001 | 1 | baseCoordFormat: design units only |
| 05DC | 1500 | coordinate – Y value, in design units |
|  | BaseCoordFormat1 HorizIdeographicBaseCoordForCyrl | BaseCoord table definition |
| 0001 | 1 | baseCoordFormat: design units only |
| FEE0 | -288 | coordinate – Y value, in design units |
|  | BaseCoordFormat1 HorizRomanBaseCoordinateForCyrl | BaseCoord table definition |
| 0001 | 1 | baseCoordFormat: design units only |
| 0000 | 0 | coordinate - Y value, in design units |

Example 3B: Baseline values for four scripts

Example 3B shows two tables that contain baseline values for each of the four scripts in the sample font described in Example 1:

- The first table shows what might happen if the baseline values in all four scripts are designed consistently. Their respective BaseValues tables list identical baseline values with the roman baseline positioned at a Y value of zero (0), the ideographic baseline at 1500, and the hanging baseline at -288.

- The second table shows what might happen if the baseline values in the scripts are designed differently with the default baseline for each script at the zero (0) coordinate.

Either method of assigning baseline values can be used in the BASE table.

Example 3B: Identical baseline values

| Baseline type | Han | Latin | Cyrillic | Devanagari |
| --- | --- | --- | --- | --- |
| hanging | 1500 | 1500 | 1500 | 1500 |
| roman | 0 | 0 | 0 | 0 |
| ideographic | -288 | -288 | -288 | -288 |

Example 3B: Assigned baseline values with default baselines at 0

| Baseline type | Han | Latin | Cyrillic | Devanagari |
| --- | --- | --- | --- | --- |
| hanging | 1788 | 1500 | 1500 | 0 |
| roman | 288 | 0 | 0 | -1500 |
| ideographic | 0 | -288 | -288 | -1788 |

Example 4: MinMax table and FeatMinMaxRecord

Example 4 shows MinMax table and FeatMinMaxRecord definitions for the same Cyrillic script described in the previous example. It contains two parts:

- Example 4A defines tables with different script, language system, and feature extents.

- Example 4B shows these same table definitions written when the language system extents match the script extents, but an obscure feature of the language system requires feature-specific extents if that feature is implemented.

The examples show only horizontal text-layout data, and the font uses 2,048 design units/em.

Example 4A: Min/Max extents for Cyrillic script, Russian language, and Russian feature

Example 4A shows two MinMax tables and a FeatMinMaxRecord for the Cyrillic script, along with sample BaseCoord tables. Only the MinCoord extent data is included.

The DefaultMinMax table defines the default minimum and maximum extents for the Cyrillic script. Another MinMax table defines language-specific min/max extents for the Russian language system to accommodate the height and width of certain glyphs used in Russian. Also, a FeatMinMaxRecord defines min/max extents for a single feature in the Russian language system that substitutes a tall integral math symbol when required.

Example 4A

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MinMax HorizCyrillicDefault MinMaxTable | Default MinMax table definition, Cyrillic script |
| 0006 | HorizCyrillic MinCoordTable | minCoord – offset to BaseCoord table |
| 000A | HorizCyrillic MaxCoordTable | maxCoord – offset to BaseCoord table |
| 0000 | 0 | featMinMaxCount: no default feature extents featMinMaxRecords[]– no FeatMinMaxRecords |
|  | BaseCoordFormat1 HorizCyrillic MinCoordTable | BaseCoord table definition, default Cyrillic min extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| FF38 | -200 | coordinate – Y value, in design units |
|  | BaseCoordFormat1 HorizCyrillic MaxCoordTable | BaseCoord table definition, default Cyrillic max extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| 0674 | 1652 | coordinate – Y value, in design units |
|  | MinMax HorizRussianMinMaxTable | MinMax table definition, Russian language extents |
| 000E | HorizRussianLangSys MinCoordTable | minCoord – offset to BaseCoord table |
| 0012 | HorizRussianLangSys MaxCoordTable | maxCoord – offset to BaseCoord table |
| 0001 | 1 | featMinMaxCount |
|  | featMinMaxRecords[0] | Records in alphabetical order by featureTableTab. |
| 7469746C | 'titl' | featureTableTag: Titling Feature must be same as Tag in FeatureList |
| 0016 | HorizRussianFeature MinCoordTable | minCoord – offset to BaseCoord table |
| 001A | HorizRussianFeature MaxCoordTable | maxCoord – offset to BaseCoord table |
|  | BaseCoordFormat1 HorizRussianLangSys MinCoordTable | BaseCoord table definition: Russian language min extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| FF08 | -248 | coordinate – Y value, in design units. Increased min extent beyond default Cyrillic min extent |
|  | BaseCoordFormat1 HorizRussianLangSys MaxCoordTable | BaseCoord table definition: Russian language feature max extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| 06A4 | 1700 | coordinate – Y value, in design units. Increased max extent beyond default Cyrillic max extent |
|  | BaseCoordFormat1 HorizRussianFeature MinCoordTable | BaseCoord table definition: Russian language min extent coordinate |
| 0001 | 1 | baseCoordFormat: Design Units Only |
| FED8 | -296 | coordinate – Y value, in design units Increased min extent beyond default Cyrillic script and Russian language min extents |
|  | BaseCoordFormat1 HorizRussianFeature MaxCoordTable | BaseCoord table definition: Russian language feature Max extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| 06D8 | 1752 | coordinate – Y value, in design units Increased max extent beyond default Cyrillic script and Russian language max extents |

Example 4B: Min/Max extents for Cyrillic script and Russian feature

A particular language system does not need to define min/max extent coordinates if its extents match the default extents defined for the script. However, an obscure or infrequently used feature within the language system may require feature-specific extent values for proper rendering.

Example 4B shows the MinMax and FeatMinMaxRecord table definitions for this situation. The example also includes a BaseScript table, but not a BaseValues tables since it is not relevant in this example. The example shows horizontal text layout extents for the Cyrillic script and feature-specific extents for one feature in the Russian language system. Much of the data is repeated from Example 4A and modified here for comparison.

The BaseScript table includes a DefaultMinMax table for the Cyrillic script and a BaseLangSysRecord that defines a BaseLangSysTag and an offset to a MinMax table for the Russian language. The MinMax table includes a FeatMinMaxRecord and specifies a FeatMinMaxCount, but both the MinCoord and MaxCoord offsets in the MinMax table are set to NULL since no language-specific extent values are defined for Russian. The FeatMinMaxRecord defines the min/max coordinates for the Russian feature and specifies the correct FeatureTableTag.

Example 4B

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseScript HorizCyrillicBaseScriptTable | BaseScript table definition: Cyrillic script |
| 0000 | NULL | Offset to BaseValues table |
| 000C | HorizCyrillicDefault MinMaxTable | Offset to default MinMax table for default script extents |
| 0001 | 1 | baseLangSysCount |
|  | baseLangSysRecords[0] | For Russian feature-specific extents. |
| 52555320 | 'RUS ' | baseLangSysTag: Russian |
| 001A | HorizRussian MinMaxTable | Offset to MinMax table for language-specific extents |
|  | MinMax HorizCyrillicDefault MinMaxTable | Default MinMax table definition: Cyrillic script |
| 0006 | HorizCyrillic MinCoordTable | minCoord – offset to BaseCoord table |
| 000A | HorizCyrillic MaxCoordTable | maxCoord – offset to BaseCoord table |
| 0000 | 0 | featMinMaxCount, no default feature extents featMinMaxRecords[], no FeatMinMaxRecords |
|  | BaseCoordFormat1 HorizCyrillic MinCoordTable | BaseCoord table definition: default Cyrillic min extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| FF38 | -200 | coordinate – Y value, in design units |
|  | BaseCoordFormat1 HorizCyrillic MaxCoordTable | BaseCoord table definition: default Cyrillic max extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| 0674 | 1652 | coordinate – Y value, in design units |
|  | MinMax HorizRussian MinMaxTable | MinMax table definition for Russian feature – no extent differences for Russian language itself |
| 0000 | NULL | minCoord – offset to min BaseCoord table not defined, matches script default |
| 0000 | NULL | maxCoord – offset to max BaseCoord table not defined, matches script default |
| 0001 | 1 | featMinMaxCount |
|  | featMinMaxRecords[0] | Records in alphabetical order by featureTableTag |
| 7469746C | 'titl' | featureTableTag: Titling Feature must be same as Tag in FeatureList |
| 000E | HorizRussianFeature MinCoordTable | minCoord – offset to BaseCoord table |
| 0012 | HorizRussianFeature MaxCoordTable | maxCoord – offset to BaseCoord table |
|  | BaseCoordFormat1 HorizRussianFeature MinCoordTable | BaseCoord table definition: Russian 'titl' feature min extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| FED8 | -296 | coordinate – Y value, in design units Increased min extent beyond default Cyrillic Min extent |
|  | BaseCoordFormat1 HorizRussianFeature MaxCoordTable | BaseCoord table definition: Russian 'titl' feature max extent coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| 06D8 | 1752 | coordinate – Y value, in design units Increased max extent beyond default Cyrillic max extent |

Example 5: BaseCoordFormat1 table

Example 5 illustrates BaseCoordFormat1, which specifies single coordinate values in design units only. The font uses 2,048 design units/em. The example defines the default minimum extent coordinate for a math script.

Example 5

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseCoordFormat1 HorizMathMinCoordTable | Definition of BaseCoord table for Math min coordinate |
| 0001 | 1 | baseCoordFormat: design units only |
| FEE8 | -280 | coordinate – Y value, in design units |

Example 6: BaseCoordFormat2 table

Example 6 illustrates the BaseCoord Format 2. Like Example 5, it specifies the minimum extent coordinate for a math script. With this format, the coordinate value depends on the final position of a specific contour point on one glyph, the integral math symbol, after hinting. Again, the value is in design units (2,048 units/em).

Example 6

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseCoordFormat2 HorizMathMinCoordTable | BaseCoord table definition for Math min coordinate |
| 0002 | 2 | baseCoordFormat: design units plus contour point |
| FEE8 | -280 | coordinate – Y value, in design units |
| 0128 | IntegralSignGlyphID | referenceGlyph: math integral sign |
| 0043 | 67 | baseCoordPoint: glyph contour point index |

Example 7: BaseCoordFormat3 table

Example 7 illustrates the BaseCoord Format 3. Like Examples 5 and 6, it specifies the minimum extent coordinate for a math script in design units (2,048 units/em). This format, however, uses a Device table to modify the coordinate value for the point size and resolution of the output font. Here, the Device table defines pixel adjustments for font sizes from 11 ppem to 15 ppem. The adjustments add one pixel at each size.

Example 7

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | BaseCoordFormat3 HorizMathMinCoordTable | BaseCoord table definition for Math min coordinate |
| 0003 | 3 | baseCoordFormat: design units plus device table |
|  | -280 | coordinate – Y value, in design units |
| 000C | HorizMathMin CoordDeviceTable | Offset to Device table |
|  | DeviceTableFormat1 HorizMathMin CoordDeviceTable | Device table definition for MinCoord |
| 000B | 11 | startSize:11 ppem |
| 000F | 15 | endSize:15 ppem |
| 0001 | 1 | deltaFormat: signed 2 bit value (8 values per uint16) |
|  | 1 | Increase 11ppem by 1 pixel |
|  | 1 | Increase 12ppem by 1 pixel |
|  | 1 | Increase 13ppem by 1 pixel |
|  | 1 | Increase 14ppem by 1 pixel |
| 5540 | 1 | Increase 15ppem by 1 pixel |

### GDEF—The glyph definition table

The Glyph Definition (GDEF) table contains six types of information in six independent subtables:

- The GlyphClassDef table classifies the different types of glyphs in the font.

- The AttachmentList table identifies all attachment points on the glyphs, which streamlines data access and bitmap caching.

- The LigatureCaretList table contains positioning data for ligature carets, which the text-processing client uses on screen to select and highlight the individual components of a ligature glyph.

- The MarkAttachClassDef table classifies mark glyphs, to help group together marks that are positioned similarly.

- The MarkGlyphSetsTable allows the enumeration of an arbitrary number of glyph sets that can be used as an extension of the mark attachment class definition to allow lookups to filter mark glyphs by arbitrary sets of marks.

- The ItemVariationStore table is used in variable fonts to contain variation data used for adjustment of values in the GDEF, GPOS or JSTF tables.

The GSUB, GPOS or JSTF tables may reference certain GDEF table information used for processing of lookup tables. See, for example, the LookupFlag bit enumeration in "OFF layout common table formats".

In variable fonts, the GDEF, GPOS and JSTF tables may all reference variation data within the ItemVariationStore table contained within the GDEF table. See below for further discussion of variable fonts and the ItemVariationStore table.

#### Overview

A client may use any one or more of the six GDEF tables during text processing. This overview explains how each of the six tables are organized and used (See figure below). The rest of this clause describes the individual GDEF tables and the tables that they reference.

Figure 6.18 – High-level organization of GDEF table

#### Overview of GDEF subtables

Glyph Class Definition Table Overview

The Glyph Class Definition (GlyphClassDef) table identifies four types of glyphs in a font: base glyphs, ligature glyphs, combining mark glyphs, and component glyphs (see Figure 6.19). GSUB and GPOS lookups define and use these glyph classes to differentiate the types of glyphs in a string. For example, GPOS uses the glyph classes to distinguish between a simple base glyph and the mark glyph that follows it.

Figure 6.19 – A base glyph, ligature glyph, mark glyph, and component glyphs

In addition, a client uses class definitions to apply GSUB and GPOS LookupFlag data correctly. For example, a LookupFlag may specify ignoring ligatures and marks during a glyph operation. If the font does not include a GlyphClassDef table, the client must define and maintain this information when using the GSUB and GPOS tables.

Attachment Point List Table Overview

The Attachment Point List table (AttachmentList) identifies all the attachment points defined in the GPOS table and their associated glyphs so a client can quickly access coordinates for each glyph's attachment points. As a result, the client can cache coordinates for attachment points along with glyph bitmaps and avoid recalculating the attachment points each time it displays a glyph. Without this table, processing speed would be slower because the client would have to decode the GPOS lookups that define attachment points and compile the points in a list.

Ligature Caret List Table Overview

The Ligature Caret List table (LigatureCaretList), particularly useful in Arabic and other scripts with many ligatures, specifies coordinates for positioning carets on all ligatures in a font. The client uses this data to select and highlight ligature components in displayed text (see Figure 6.20).

Figure 6.20 – Proper ligature caret postioning

Each ligature can have more than one caret position, with each position defined as an X or Y value on the baseline according to the writing direction of the script or language system. The font developer can use any of three formats to represent a caret coordinate value. One format represents values in design units only, another fine-tunes a value based on a designated contour point, and the third uses a Device table (in non-variable fonts only) to adjust values at specific font sizes.

In a variable font, the caret positions may need to be adjusted for different variation instances. This is done using data in an ItemVariationStore table. See below for more regarding variable fonts and the ItemVariationStore table.

Without a Ligature Caret List table, the client would have to define caret positions without knowing the positions of the ligature components. The resulting highlighting or hit-testing might be ambiguous. For example, suppose a client places a caret at the midpoint position along the width of a hyphothetical "wi" ligature. Because the "w" is wider than the "i", that position would not clearly indicate which component is selected. Instead, for accurate selection, the caret should be moved to the right so that either the "w" or "i" could be clearly highlighted.

Mark Attachment Class Definition Table Overview

A Mark Class Definition Table is used to assign mark glyphs into different classes that can be used in lookup tables within the GSUB or GPOS table to control how mark glyphs within a glyph sequence are treated by lookups. For more information on the use of mark attachment classes, see the description of lookup flags in the “Lookup Table” section of subclause 6.2.

Mark Glyph Sets Table Overview

A Mark Glyph Sets table is used to define sets of mark glyphs that can be used in lookup tables within the GSUB or GPOS table to control how mark glyphs within a glyph sequence are treated by lookups. For more information on the use of mark glyph sets, see the description of lookup flags in the “Lookup Table” section of subclause 6.2.

Item Variation Store Overview

An Item Variation Store table is used in variable fonts. OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

When different variation instances are selected, the design of individual glyphs changes. The same contours and points are used, but the position in the design grid of each point can change, as can the overall glyph metrics. As a result, corresponding changes may also be required for caret X or Y positions.

Variation data for caret positions is stored in the Item Variation Store table. This same table, within the GDEF table, can also hold variation data used for X or Y values in the GPOS or JSTF tables. The Item Variation Store and constituent formats are described in subclause 7.2. The Item Variation Store is also used in the MVAR, HVAR and other tables, but is different from the formats for variation data used in the 'cvar' or 'gvar' tables.

The variation data within an Item Variation Store is comprised of a number of adjustment deltas that get applied to the default values of target items for variation instances within particular regions of the font’s variation space. The Item Variation Store format uses delta-set indices to reference variation delta data for particular target, font-data items to which they are applied. Data external to the Item Variation Store identifies the delta-set index to be used for each given target item. Within the GDEF, GPOS or JSTF tables, these indices are specified within VariationIndex tables, with one VariationIndex table referenced for each item that requires variation adjustment.

Note that the VariationIndex table is a variant of a Device table, with a distinct format value. (For full details on the Device and VariationIndex table formats, see the OFF layout common table formats.) This is done so that the default instance of a variable font can be compatible with applications that do not support Font Variations. As a result, variable fonts cannot use device tables. A VariationIndex table will be ignored in applications that do not support Font Variations, or if the font is not a variable font.

The Item Variation Store format uses a two-level organization for variation data: a store can have multiple Item Variation Data subtables, and each subtable has multiple delta-set rows. A delta-set index is a two-part index: an outer index that selects a particular item variation data subtable, and an inner index that selects a particular delta-set row within that subtable. A VariationIndex table specifies both the outer and inner portions of the delta-set index.

#### GDEF table structure

GDEF header

The GDEF table begins with a header that starts with a version number. Three versions are defined. Version 1.0 contains an offset to a Glyph Class Definition table (GlyphClassDef), an offset to an Attachment List table (AttachList), an offset to a Ligature Caret List table (LigCaretList), and an offset to a Mark Attachment Class Definition table (MarkAttachClassDef). Version 1.2 also includes an offset to a Mark Glyph Sets Definition table (MarkAttachClassDef). Version 1.3 also includes an offset to an Item Variation Store table.

Example 1 in subclause 6.3.2.3 shows a GDEF Header table.

GDEF header, Version 1.0

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GDEF table, =1 |
| uint16 | minorVersion | Minor version of the GDEF table, =0 |
| Offset16 | glyphClassDefOffset | Offset to class definition table for glyph type, from beginning of GDEF header (may be NULL) |
| Offset16 | attachListOffset | Offset to attachment point list table, from beginning of GDEF header (may be NULL) |
| Offset16 | ligCaretListOffset | Offset to ligature caret list table, from beginning of GDEF header (may be NULL) |
| Offset16 | markAttachClassDefOffset | Offset to class definition table for mark attachment type, from beginning of GDEF header (may be NULL) |

GDEF header, Version 1.2

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GDEF table, =1 |
| uint16 | minorVersion | Minor version of the GDEF table, =2 |
| Offset16 | glyphClassDefOffset | Offset to class definition table for glyph type, from beginning of GDEF header (may be NULL) |
| Offset16 | attachListOffset | Offset to attachment point list table, from beginning of GDEF header (may be NULL) |
| Offset16 | ligCaretListOffset | Offset to ligature caret list table, from beginning of GDEF header (may be NULL) |
| Offset16 | markAttachClassDefOffset | Offset to class definition table for mark attachment type, from beginning of GDEF header (may be NULL) |
| Offset16 | markGlyphSetsDefOffset | Offset to the table of mark glyph set definitions, from beginning of GDEF header (may be NULL) |

GDEF Header, Version 1.3

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GDEF table, = 1 |
| uint16 | minorVersion | Minor version of the GDEF table, = 3 |
| Offset16 | glyphClassDefOffset | Offset to class definition table for glyph type, from beginning of GDEF header (may be NULL) |
| Offset16 | attachListOffset | Offset to attachment point list table, from beginning of GDEF header (may be NULL) |
| Offset16 | ligCaretListOffset | Offset to ligature caret list table, from beginning of GDEF header (may be NULL) |
| Offset16 | markAttachClassDefOffset | Offset to class definition table for mark attachment type, from beginning of GDEF header (may be NULL) |
| Offset16 | markGlyphSetsDefOffset | Offset to the table of mark glyph set definitions, from beginning of GDEF header (may be NULL) |
| Offset32 | itemVarStoreOffset | Offset to the Item Variation Store table, from beginning of GDEF header (may be NULL) |

Glyph Class Definition table

The GSUB and GPOS tables use the Glyph Class Definition table (GlyphClassDef) to identify which glyph classes to adjust with lookups.

The table uses the same format as the Class Definition table (for details, see subclause 6.2). However, the GlyphClassDef table uses class values already defined in the GlyphClassDef Enumeration list:

GlyphClassDef Enumeration List

| Class | Description |
| --- | --- |
| 1 | Base glyph (single character, spacing glyph) |
| 2 | Ligature glyph (multiple character, spacing glyph) |
| 3 | Mark glyph (non-spacing combining glyph) |
| 4 | Component glyph (part of single character, spacing glyph) |

The font developer does not have to classify every glyph in the font, but any glyph not assigned a class value falls into Class zero (0). For instance, class values might be useful for the Arabic glyphs in a font, but not for the Latin glyphs. Then the GlyphClassDef table will list only Arabic glyphs, and-by default-the Latin glyphs will be assigned to Class 0. Component glyphs can be put together to generate ligatures. A ligature can be generated by creating a glyph in the font that references the component glyphs, or outputting the component glyphs in the desired sequence. Component glyphs are not used in defining any GSUB or GPOS formats.

Example 2 at the end of this clause defines a GlyphClassDef table with a sample glyph for each of the assigned classes.

Attachment Point List table

The Attachment Point List table (AttachList) may be used to cache attachment point coordinates along with glyph bitmaps.

The table consists of an offset to a Coverage table (Coverage) listing all glyphs that define attachment points in the GPOS table, a count of the glyphs with attachment points (GlyphCount), and an array of offsets to AttachPoint tables (AttachPoint). The array lists the AttachPoint tables, one for each glyph in the Coverage table, in the same order as the Coverage Index.

AttachList table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of AttachList table |
| uint16 | glyphCount | Number of glyphs with attachment points |
| Offset16 | attachPointOffsets[glyphCount] | Array of offsets to AttachPoint tables, from beginning of AttachList table-in Coverage Index order |

An AttachPoint table consists of a count of the attachment points on a single glyph (PointCount) and an array of contour indices of those points (PointIndex), listed in increasing numerical order.

Example 3 at the end of the clause demonstrates an AttachList table that defines attachment points for two glyphs.

AttachPoint table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | pointCount | Number of attachment points on this glyph |
| uint16 | pointIndices[pointCount] | Array of contour point indices – in increasing numerical order |

Ligature Caret List table

The Ligature Caret List table (LigCaretList) defines caret positions for all the ligatures in a font. The table consists of an offset to a Coverage table that lists all the ligature glyphs (Coverage), a count of the defined ligatures (LigGlyphCount), and an array of offsets to LigGlyph tables (LigGlyph). The array lists the LigGlyph tables, one for each ligature in the Coverage table, in the same order as the Coverage Index.

Example 4 at the end of this clause shows a LigCaretList table.

LigCaretList table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of LigCaretList table |
| uint16 | ligGlyphCount | Number of ligature glyphs |
| Offset16 | ligGlyphOffsets[ligGlyphCount] | Array of offsets to LigGlyph tables, from beginning of LigCaretList table – in Coverage Index order |

Ligature Glyph table

A Ligature Glyph table (LigGlyph) contains the caret coordinates for a single ligature glyph. The number of coordinate values, each defined in a separate CaretValue table, equals the number of components in the ligature minus one (1).

The LigGlyph table consists of a count of the number of CaretValue tables defined for the ligature (CaretCount) and an array of offsets to CaretValue tables (CaretValue).

Example 4 at the end of the clause shows a LigGlyph table.

LigGlyph table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | caretCount | Number of CaretValues for this ligature (components - 1) |
| Offset16 | caretValueOffsets [caretCount] | Array of offsets to CaretValue tables, from beginning of LigGlyph table – in increasing coordinate order |

Caret Values table

A Caret Values table (CaretValues), which defines caret positions for a ligature, can be any of three possible formats. One format uses design units to define the caret position. The other two formats use a contour point or (in non-variable fonts) a Device table to fine-tune a caret's position at specific font sizes and device resolutions. In a variable font, the third format uses a VariationIndex table (a variant of a Device table) to reference variation data for adjustment of the caret position for the current variation instance, as needed. Caret coordinates are either X or Y values, depending upon the text direction.

CaretValue Format 1

The first format (CaretValueFormat1) consists of a format identifier (CaretValueFormat), followed by a single coordinate for the caret position (Coordinate). The Coordinate is in design units.

This format has the benefits of small size and simplicity, but the Coordinate value cannot be hinted for fine adjustments at different device resolutions.

Example 4 at the end of this clause shows a CaretValueFormat1 table.

CaretValueFormat1 table: Design units only

| Type | Name | Description |
| --- | --- | --- |
| uint16 | caretValueFormat | Format identifier: format = 1 |
| int16 | coordinate | X or Y value, in design units |

CaretValue Format 2

The second format (CaretValueFormat2) specifies the caret coordinate in terms of a contour point index on a specific glyph. During font hinting, the contour point on the glyph outline may move. The point's final position after hinting provides the final value for rendering a given font size.

The table contains a format identifier (CaretValueFormat) and a contour point index (CaretValuePoint).

Example 5 at the end of this clause demonstrates a CaretValueFormat2 table.

CaretValueFormat2 table: Contour point

| Type | Name | Description |
| --- | --- | --- |
| uint16 | caretValueFormat | Format identifier: format = 2 |
| uint16 | caretValuePointIndex | Contour point index on glyph |

CaretValue Format 3

The third format (CaretValueFormat3) also specifies the value in design units, but, in non-variable fonts, it uses a Device table rather than a contour point to adjust the value. This format offers the advantage of fine-tuning the Coordinate value for any device resolution. (For more information about Device tables, see the clause, Common Table Formats.)

In variable fonts, CaretValueFormat3 must be used to reference variation data to adjust caret positions for different variation instances, if needed. In this case, CaretValueFormat3 specifies an offset to a VariationIndex table, which is a variant of the Device table used for variations.

NOTE 1	While separate VariationIndex table references are required for each value that requires variation, two or more values that require the same variation-data values can have offsets that point to the same VariationIndex table, and two or more VariationIndex tables can reference the same variation data entries.

NOTE 2	If no VariationIndex table is used for a particular caret position value, then that value is used for all variation instances.

The format consists of a format identifier (CaretValueFormat), an X or Y value (Coordinate), and an offset to a Device or VariationIndex table.

Example 6 at the end of this clause shows a CaretValueFormat3 table.

CaretValueFormat3 table: Design units plus Device or VariationIndex table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | caretValueFormat | Format identifier: format = 3 |
| int16 | coordinate | X or Y value, in design units |
| Offset16 | deviceOffset | Offset to Device table (non-variable font) / Variation Index table (variable font) for X or Y value, from beginning of CaretValue table |

Mark Attachment Class Definition table

A Mark Attachment Class Definition Table defines the class to which a mark glyph may belong. This table uses the same format as the Class Definition table (for details, see subclause 6.2).

Example 7 in this document shows a MarkAttachClassDef table.

Mark Glyph Sets table

Mark glyph sets are used in GSUB and GPOS lookups to filter which marks in a string are considered or ignored. Mark glyph sets are defined in a MarkGlyphSets table, which contains offsets to individual sets each represented by a standard Coverage table.

MarkGlyphSetsTable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | markGlyphSetTableFormat | Format identifier = 1 |
| uint16 | markGlyphSetCount | Number of mark glyph sets defined |
| Offset32 | coverageOffsets [markGlyphSetCount] | Array of offsets to mark glyph set Coverage tables |

Mark glyph sets are used for the same purpose as mark attachment classes, which is as filters for GSUB and GPOS lookups. Mark glyph sets differ from mark attachment classes, however, in that mark glyph sets may intersect as needed by the font developer. As for mark attachment classes, only one mark glyph set can be referenced in any given lookup.

Note that the array of offsets for the Coverage tables uses Offset32, not Offset16.

Item Variation Store Table

The format and processing of the Item Variation Store table and its constituent formats is described in the subclause "Font variations common table formats". Specification of the interpolation algorithm used to derive values for particular variation instances is given in subclause 7.1.

The Item Variation Store contains adjustment-delta values arranged in one or more sets of deltas that are referenced using delta-set indices. For values that require variation adjustment, a delta-set index is used to reference the particular variation data needed for that target value. Within the GDEF, GPOS or JSTF tables, delta-set indices are provided in VariationIndex tables associated with particular target items, such as caret positions in the GDEF table. For a description of the VariationIndex table, see subclause 6.2. For details on use of VariationIndex tables within the GDEF table, see discussion earlier in this chapter. For details on use of VariationIndex tables within the GPOS or JSTF tables, see the discussion of OFF Font Variations in the chapters for each of those tables.

#### GDEF table examples

The rest of this subclause describes examples of all the GDEF table formats. All the examples reflect unique parameters described below, but the samples provide a useful reference for building tables specific to other situations.

The examples have three columns showing hex data, source, and comments.

Example 1: GDEF header

Example 1 shows a GDEF Header definition with offsets to each of the main tables in GDEF.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | GDEFHeader TheGDEFHeader | GDEFHeader table definition |
| 00010000 | 0x00010000 | major/minor version |
| 000C | GlyphClassDefTable | Offset to GlyphClassDef table |
| 0026 | AttachListTable | Offset to AttachList table |
| 0040 | LigCaretListTable | Offset to LigCaretList table |
| 005A | MarkAttachClassDefTable | Offset to Mark Attachment Class Definition Table |

Example 2: GlyphClassDef table

The GlyphClassDef table in Example 2 specifies a glyph for the each of the glyph classes predefined in the GlyphClassDef Enumeration List.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ClassDefFormat2 GlyphClassDefTable | ClassDef table definition |
| 0002 | 2 | classFormat |
| 0004 | 4 | classRangeCount |
|  | classRangeRecords[0] |  |
| 0024 | iGlyphID | startGlyphID |
| 0024 | iGlyphID | endGlyphID |
| 0001 | 1 | class: base glyphs |
|  | classRangeRecords[1] |  |
| 009F | ffiLigGlyphID | startGlyphID |
| 009F | ffiLigGlyphID | endGlyphID |
| 0002 | 2 | class: ligature glyphs |
|  | classRangeRecords[2] |  |
| 0058 | umlautAccentGlyphID | startGlyphID |
| 0058 | umlautAccentGlyphID | endGlyphID |
| 0003 | 3 | class: mark glyphs |
|  | classRangeRecords[3] |  |
| 018F | CurvedTailComponentGlyphID | startGlyphID |
| 018F | CurvedTailComponentGlyphID | endGlyphID |
| 0004 | 4 | class: component glyphs |

Example 3: AttachList table

In Example 3, the AttachList table enumerates the attachment points defined for two glyphs. The GlyphCoverage table identifies the glyphs: "a" and "e". For each covered glyph, an AttachPoint table specifies the attachment point count and point indices: one point for the "a" glyph and two for the "e" glyph.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | AttachList AttachListTable | AttachList table definition |
| 0012 | GlyphCoverage | Offset to Coverage table |
| 0002 | 2 | glyphCount |
| 0008 | aAttachPoint | attachPointOffsets[0] |
| 000C | eAttachPoint | attachPointOffsets [1] |
|  | AttachPoint aAttachPoint | AttachPoint table definition |
| 0001 | 1 | pointCount |
| 0012 | 18 | pointIndices[0] |
|  | AttachPoint eAttachPoint | AttachPoint table definition |
| 0002 | 2 | pointCount |
| 000E | 14 | pointIndices [0] |
| 0017 | 23 | pointIndices [1] |
|  | CoverageFormat1 GlyphCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat |
| 0002 | 2 | glyphCount |
| 001C | aGlyphID | glyphArray[0] |
| 0020 | eGlyphID | glyphArray[1] |

Example 4: LigCaretList table, LigGlyph table and CaretValueFormat1 table

Example 4 defines a list of ligature carets. The LigCoverage table lists all the ligature glyphs that define caret positions. In this example, two ligatures are covered, "ffi" and "fi". For each covered glyph, a LigGlyph table specifies the number of carets for the ligature and their coordinate values. The "fi" ligature defines one caret, positioned between the "f" and "i" components; the "ffi" ligature defines two, one positioned between the two "f" components and the other positioned between the "f" and "i". The CaretValue tables shown here use Format1, where values are specified in design units only.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | LigCaretList LigCaretListTable | LigCaretList table definition |
| 0008 | LigCoverage | Offset to Coverage table |
| 0002 | 2 | ligGlyphCount |
| 0010 | fiLigGlyph | ligGlyphOffsets [0] |
| 0014 | ffiLigGlyph | ligGlyphOffsets [1] |
|  | CoverageFormat1 LigCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat |
| 0002 | 2 | glyphCount |
| 009F | ffiLigGlyphID | glyphArray[0] |
| 00A5 | fiLigGlyphID | glyphArray[1] |
|  | LigGlyph fiLigGlyph | LigGlyph table definition |
| 0001 | 1 | caretCount, equals the number of components - 1 |
| 000E | CaretFI | caretValueOffsets[0] |
|  | LigGlyph ffiLigGlyph | LigGlyph table definition |
| 0002 | 2 | caretCount, equals the number of components - 1 |
| 0006 | CaretFFI1 | caretValueOffsets [0] |
| 000E | CaretFFI2 | caretValueOffsets [1] |
|  | CaretValueFormat1 CaretFI | CaretValue table definition |
| 0001 | 1 | caretValueFormat: design units only |
| 025B | 603 | coordinate (X or Y value) |
|  | CaretValueFormat1 CaretFFI1 | CaretValue table definition |
| 0001 | 1 | caretValueFormat: design units only |
| 025B | 603 | coordinate (X or Y value) |
|  | CaretValueFormat1 CaretFFI2 | CaretValue table definition |
| 0001 | 1 | caretValueFormat: design units only |
| 04B6 | 1206 | coordinate (X or Y value) |

Example 5: CaretValueFormat2 table

Example 5 shows a CaretValueFormat2 table that specifies a ligature caret coordinate in terms of a contour point index on a specific glyph. The final position of the caret depends on the location of the contour point on the glyph after hinting.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | CaretValueFormat2 Caret1 | CaretValue table definition |
| 0002 | 2 | caretValueFormat: contour point |
| 000D | 13 | caretValuePointIndex |

Example 6: CaretValueFormat3 table

In Example 6, the CaretValueFormat3 table defines a caret position in design units, but includes a Device table to adjust the X or Y coordinate for the point size and resolution of the output font. Here, the Device table specifies pixel adjustments for font sizes from 12 ppem to 17 ppem.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | CaretValueFormat3 Caret3 | CaretValue table definition |
| 0003 | 3 | caretValueFormat: design units plus Device table |
| 04B6 | 1206 | coordinate (X or Y value, design units) |
| 0006 | CaretDevice | Offset to Device table |
|  | DeviceTableFormat2 CaretDevice | Device Table definition |
| 000C | 12 | startSize |
| 0011 | 17 | endSize |
| 0002 | 2 | deltaFormat |
|  | 1 | increase 12ppm by 1 pixel |
|  | 1 | increase 13ppm by 1 pixel |
|  | 1 | increase 14ppm by 1 pixel |
| 1111 | 1 | increase 15ppm by 1 pixel |
|  | 2 | increase 16ppm by 2 pixels |
| 2200 | 2 | increase 17ppm by 2 pixels |

Example 7: MarkAttachClassDef table

In Example 7, the MarkAttachClassDef table specifies an attachment class for the each of the glyph ranges predefined in the GlyphClassDef Enumeration List as marks.

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ClassDefFormat2 theMarkAttachClassDefTable | ClassDef table definition |
| 0002 | 2 | classFormat |
| 0004 | 4 | classRangeCount |
|  | classRangeRecords[0] |  |
| 0268 | graveAccentGlyphID | startGlyphID |
| 026A | circumflexAccentGlyphID | endGlyphID |
| 0001 | 1 | Class: top marks |
|  | classRangeRecords[1] |  |
| 0270 | diaeresisAccentGlyphID | startGlyphID |
| 0272 | acuteAccentGlyphID | endGlyphID |
| 0001 | 1 | Class: top marks |
|  | classRangeRecords[2] |  |
| 028C | diaeresisBelowGlyphID | startGlyphID |
| 028F | cedillaGlyphID | endGlyphID |
| 0002 | 2 | Class: bottom marks |
|  | classRangeRecords[3] |  |
| 0295 | circumflexBelowGlyphID | startGlyphID |
| 0295 | circumflexBelowGlyphID | endGlyphID |
| 0002 | 2 | Class: bottom marks |

### GPOS—The glyph positioning table

#### Overview

The Glyph Positioning table (GPOS) provides precise control over glyph placement for sophisticated text layout and rendering in each script and language system that a font supports.

Complex glyph positioning becomes an issue in writing systems, such as Vietnamese, that use diacritical and other marks to modify the sound or meaning of characters. These writing systems require controlled placement of all marks in relation to one another for legibility and linguistic accuracy.

Figure 6.21 – Vietnamese words with marks.

Other writing systems require sophisticated glyph positioning for correct typographic composition. For instance, Urdu glyphs are calligraphic and connect to one another along a descending, diagonal text line that proceeds from right to left. To properly render Urdu, a text-processing client must modify both the horizontal (X) and vertical (Y) positions of each glyph (see Figure 6.22).

Figure 6.22 – Urdu layout requires glyph positioning control, as well as contextual substitution

With the GPOS table, a font developer can define a complete set of positioning adjustment features in an OFF font. GPOS data, organized by script and language system, is easy for a text-processing client to use to position glyphs.

Positioning glyphs with TrueType 1.0

Glyph positioning in TrueType uses only two values, placement and advance, to specify a glyph's position for text layout. If glyphs are positioned with respect to a virtual "pen point" that moves along a line of text, placement describes the glyph's position with respect to the current pen point, and advance describes where to move the pen point to position the next glyph (see Figure 6.23). For horizontal text, placement corresponds to the left side bearing, and advance corresponds to the advance width.

Figure 6.23 – Glyph positioning with TrueType

TrueType specifies placement and advance only in the X direction for horizontal layout and only in the Y direction for vertical layout. For simple Latin text layout, these two values may be adequate to position glyphs correctly. But, for texts that require more sophisticated layout, the values must cover a richer range. Placement and advance may need adjustment vertically, as well as horizontally.

The only positioning adjustment defined in TrueType is pair kerning, which modifies the horizontal spacing between two glyphs. A typical kerning table lists pairs of glyphs and specifies how much space a text-processing client should add or remove between the glyphs to properly display each pair. It does not provide specific information about how to adjust the glyphs in each pair, and cannot adjust contexts of more than two glyphs.

Positioning glyphs with OFF

OFF fonts allow excellent control and flexibility for positioning a single glyph and for positioning multiple glyphs in relation to one another. By using both X and Y values that the GPOS table defines for placement and advance and by using glyph attachment points, a client can more precisely adjust the position of a glyph.

In addition, the GPOS table can reference a Device table to define subtle, device-dependent adjustments to any placement or advance value at any font size and device resolution. For example, a Device table can specify adjustments at 51 pixels per em (ppem) that do not occur at 50 ppem.

X and Y values specified in OFF fonts for placement operations are always within the typical Cartesian coordinate system (origin at the baseline of the left side), regardless of the writing direction. Additionally, all values specified are done so in font unit measurements. This is especially convenient for font designers, since glyphs are drawn in the same coordinate system. However, it's important to note that the meaning of "advance width" changes, depending on the writing direction.

For example, in left-to-right scripts, if the first glyph has an advance width of 100, then the second glyph begins at 100,0. In right-to-left scripts, if the first glyph has an advance width of 100, then the second glyph begins at -100,0. For a top-to-bottom feature, to increase the advance height of a glyph by 100, the YAdvance = 100. For any feature, regardless of writing direction, to lower the dieresis over an 'o' by 10 units, set the YPlacement = -10.

Other GPOS features can define attachment points to combine glyphs and position them with respect to one another. A glyph might have multiple attachment points. The point used will depend on the glyph to be attached. For instance, a base glyph could have attachment points for different diacritical marks.

Figure 6.24 – Base glyph with multiple attachment points.

To reduce the size of the font file, a base glyph may use the same attachment point for all mark glyphs assigned to a particular class. For example, a base glyph could have two attachment points, one above and one below the glyph. Then all marks that attach above glyphs would be attached at the high point, and all marks that attach below glyphs would be attached at the low point. Attachment points are useful in scripts such as Arabic that combine numerous glyphs with vowel marks.

Attachment points also are useful for connecting cursive-style glyphs. Glyphs in cursive fonts can be designed to attach or overlap when rendered. Alternatively, the font developer can use OFF to create a cursive attachment feature and define explicit exit and entry attachment points for each glyph (see Figure 6.25).

Figure 6.25 – Entry and exit points marked on contextual Urdu glyph variations

The GPOS table supports eight types of actions for positioning and attaching glyphs:

- A single adjustment positions one glyph, such as a superscript or subscript.

- A pair adjustment positions two glyphs with respect to one another. Kerning is an example of pair adjustment.

- A cursive attachment describes cursive scripts and other glyphs that are connected with attachment points when rendered.

- A mark-to-base attachment positions combining marks with respect to base glyphs, as when positioning vowels, diacritical marks, or tone marks in Arabic, Hebrew, and Vietnamese.

- A mark-to-ligature attachment positions combining marks with respect to ligature glyphs. Because ligatures may have multiple points for attaching marks, the font developer needs to associate each mark with one of the ligature glyph's components.

- A mark-to-mark attachment positions one mark relative to another, as when positioning tone marks with respect to vowel diacritical marks in Vietnamese.

- Contextual positioning describes how to position one or more glyphs in context, within an identifiable sequence of specific glyphs, glyph classes, or varied sets of glyphs. One or more positioning operations may be performed on "input" context sequences. Figure 6.26 illustrates a context for positioning adjustments.

- Chaining contextual positioning describes how to position one or more glyphs in a chained context, within an identifiable sequence of specific glyphs, glyph classes, or varied sets of glyphs. One or more positioning operations may be performed on "input" context sequences.

Figure 6.26 – Contextual positioning lowered the accent over a vowel glyph that followed an overhanging uppercase glyph

GPOS Table and OFF Font Variations

OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

When different variation instances are selected, the design of individual glyphs changes. The same contours and points are used, but the position in the design grid of each point can change, as can the default horizontal or vertical advance and side bearings. As a result, corresponding changes may also be required for positioning and advance adjustments in the GPOS table.

Positioning actions in the GPOS table are expressed directly using explicit X or Y font-unit values. In a variable font, these X and Y values apply to the default instance and may need to be adjusted for the current variation instance. This is done using variation data with processes similar to those used for glyph outlines and other font data, as described in the OFF Font Variations overview.

NOTE	For certain GPOS actions, positions can be expressed indirectly by reference to specific glyph outline points. In a variable font, use of glyph points to specify a positioning action would require invoking the rasterizer to process the glyph-outline variation data in order to obtain the adjusted position of the point before the glyph positioning operation can be completed. This may have a significant, negative impact on performance of text-layout processing. For this reason, it is recommended that, in a variable font, positions that require adjustment for different variation instances should always be expressed directly as X and Y values.

Variation data for adjustment of GPOS X or Y values is stored within an Item Variation Store table located within the GDEF table. The same Item Variation Store is also used for adjustment of values in the GDEF and JSTF tables. The Item Variation Store and constituent formats are described in subclause 7.2. These formats are also used in the BASE table, as well as in the MVAR and other tables, but is different from the formats for variation data used in the 'cvar' or gvar tables.

The variation data within an Item Variation Store is comprised of a number of adjustment deltas that get applied to the default values of target items for variation instances within particular regions of the font’s variation space. The Item Variation Store format uses delta-set indices to reference variation delta data for particular target, font-data items to which they are applied. Data external to the Item Variation Store identifies the delta-set index to be used for each given target item. Within the GPOS table, these indices are specified within VariationIndex tables, with one VariationIndex table referenced for each item that requires variation adjustment.

Note that the VariationIndex table is a variant of a Device table, with a distinct format value. (For full details on the Device and VariationIndex table formats, see subclause 6.2.) This is done so that the default instance of a variable font can be compatible with applications that do not support Font Variations. As a result, variable fonts cannot use device tables. A VariationIndex table will be ignored in applications that do not support Font Variations, or if the font is not a variable font.

The Item Variation Store format uses a two-level organization for variation data: a store can have multiple Item Variation Data subtables, and each subtable has multiple delta-set rows. A delta-set index is a two-part index: an outer index that selects a particular Item Variation Data subtable, and an inner index that selects a particular delta-set row within that subtable. A VariationIndex table specifies both the outer and inner portions of the delta-set index.

#### GPOS table organization

The GPOS table begins with a header that defines offsets to a ScriptList, a FeatureList, a LookupList, and an optional FeatureVariations table (see Figure 6.27):

- The ScriptList identifies all the scripts and language systems in the font that use glyph positioning.

- The FeatureList defines all the glyph positioning features required to render these scripts and language systems.

- The LookupList contains all the lookup data needed to implement each glyph positioning feature.

- The FeatureVariations table can be used to substitute an alternate set of lookup tables to use for any given feature under specified conditions. This is currently used only in variable fonts.

For a detailed discussion of ScriptLists, FeatureLists, LookupLists, and FeatureVariations tables see the OFF layout common table formats. The following discussion summarizes how the GPOS table works.

Figure 6.27 – High-level organization of GPOS table

The GPOS table is organized so text processing clients can easily locate the features and lookups that apply to a particular script or language system. To access GPOS information, clients should use the following procedure:

- Locate the current script in the GPOS ScriptList table.

- If the language system is known, search the script for the correct LangSys table; otherwise, use the script's default LangSys table.

- The LangSys table provides index numbers into the GPOS FeatureList table to access a required feature and a number of additional features.

- Inspect the featureTag of each feature, and select the feature tables to apply to an input glyph string.

- If a Feature Variation table is present, evaluate conditions in the Feature Variation table to determine if any of the initially-selected feature tables should be substituted by an alternate feature table.

- Each feature provides an array of index numbers into the GPOS LookupList table. Assemble all lookups from the set of chosen feature tables, and apply the lookups in the order given in the LookupList table.

For a detailed description of the Feature Variation table and how it is processed, see subclause 6.2.

Lookup data is defined in Lookup tables, which are defined in subclause 6.2. A LookupTable contains one or more subtables that define the specific conditions, type, and results of a positioning action used to implement a feature. Specific Lookup subtable types are used for glyph positioning actions, and are defined in this subclause. All subtables within a Lookup table shall be of the same lookup type, as listed in the following table for the GPOS LookupType Enumeration:

GPOS LookupType Enumeration

| Value | Type | Description |
| --- | --- | --- |
| 1 | Single adjustment | Adjust position of a single glyph |
| 2 | Pair adjustment | Adjust position of a pair of glyphs |
| 3 | Cursive attachment | Attach cursive glyphs |
| 4 | MarkToBase attachment | Attach a combining mark to a base glyph |
| 5 | MarkToLigature attachment | Attach a combining mark to a ligature |
| 6 | MarkToMark attachment | Attach a combining mark to another mark |
| 7 | Context positioning | Position one or more glyphs in context |
| 8 | Chained Context positioning | Position one or more glyphs in chained context |
| 9 | Extension positioning | Extension mechanism for other positionings |
| 10+ | Reserved | For future use (must be set to zero) |

Each LookupType has one or more subtable formats. The "best" format depends on the type of positioning operation and the resulting storage efficiency. When glyph information is best presented in more than one format, a single lookup may define more than one subtable, as long as all the subtables are of the same LookupType. For example, within a given lookup, a glyph index array format may best represent one set of target glyphs, whereas a glyph index range format may be better for another set.

Certain structures are used across multiple GPOS Lookup subtable types and formats. All Lookup subtables use the Coverage table, which is defined in subclause 6.2. Single and pair adjustments (LookupTypes 1 and 2) use a ValueRecord structure and associated ValueFormat enumeration, which are defined later in this subclause. Attachment subtables (LookupTypes 3, 4, 5 and 6) use Anchor and MarkArray tables, also defined later in this subclause.

A series of positioning operations on the same glyph or string requires multiple lookups, one for each separate action. Positioning adjustment values are accumulated in these cases. Each lookup has a different array index in the LookupList table and is applied in the LookupList order.

During text processing, a client applies a lookup to each glyph in the string before moving to the next lookup. A lookup is finished for a glyph after the client locates the target glyph or glyph context and performs a positioning action, if specified. To move to the "next" glyph, the client will typically skip all the glyphs that participated in the lookup operation: glyphs that were positioned as well as any other glyphs that formed a context for the operation.

There is just one exception: the "next" glyph in a sequence may be one of those that formed a context for the operation just performed. For example, in the case of pair positioning operations (i.e., kerning), if the ValueRecord for the second glyph is null, that glyph is treated as the "next" glyph in the sequence.

This rest of this clause describes the GPOS header and the subtables defined for each LookupType. Examples at the end of this clause illustrate the GPOS header and seven of the nine LookupTypes, as well as the ValueRecord and Anchor and MarkArray tables.

GPOS header

The GPOS table begins with a header that contains a version number (Version) initially set to 1.0 (0x00010000) and offsets to three tables: ScriptList, FeatureList, and LookupList. For descriptions of these tables, see subclause 6.2. Example 1 at the end of this clause shows a GPOS Header version 1.0 table definition.

GPOS Header, Version 1.0

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GPOS table, = 1 |
| uint16 | minorVersion | Minor version of the GPOS table, = 0 |
| Offset16 | scriptListOffset | Offset to ScriptList table, from beginning of GPOS table |
| Offset16 | featureListOffset | Offset to FeatureList table, from beginning of GPOS table |
| Offset16 | lookupListOffset | Offset to LookupList table, from beginning of GPOS table |

GPOS Header, Version 1.1

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GPOS table, = 1 |
| uint16 | minorVersion | Minor version of the GPOS table, = 1 |
| Offset16 | scriptListOffset | Offset to ScriptList table, from beginning of GPOS table |
| Offset16 | featureListOffset | Offset to FeatureList table, from beginning of GPOS table |
| Offset16 | lookupListOffset | Offset to LookupList table, from beginning of GPOS table |
| Offset32 | featureVariationsOffset | Offset to FeatureVariations table, from beginning of GPOS table (may be NULL) |

#### GPOS lookup type descriptions

##### Lookup Type 1: Single adjustment positioning subtable

A single adjustment positioning subtable (SinglePos) is used to adjust the placement or advance of a single glyph, such as a subscript or superscript. In addition, a SinglePos subtable is commonly used to implement lookup data for contextual positioning.

A SinglePos subtable will have one of two formats: one that applies the same adjustment to a series of glyphs (Format 1), and one that applies a different adjustment for each unique glyph (Format 2).

Single Adjustment Positioning: Format 1: Single Positioning Value

A SinglePosFormat1 subtable applies the same positioning value or values to each glyph listed in its Coverage table. For instance, when a font uses old-style numerals, this format could be applied to uniformly lower the position of all math operator glyphs.

The Format 1 subtable consists of a format identifier (posFormat), an offset to a Coverage table that defines the glyphs to be adjusted by the positioning values (coverageOffset), and the format identifier (valueFormat) that describes the amount and kinds of data in the ValueRecord.

The ValueRecord specifies one or more positioning values to be applied to all covered glyphs (valueRecord). For example, if all glyphs in the Coverage table require both horizontal and vertical adjustments, the ValueRecord will specify values for both xPlacement and yplacement.

Example 2 at the end of this clause shows a SinglePosFormat1 subtable used to adjust the placement of subscript glyphs.

SinglePosFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | coverageOffet | Offset to Coverage table, from beginning of SinglePos subtable. |
| uint16 | valueFormat | Defines the types of data in the ValueRecord. |
| ValueRecord | valueRecord | Defines positioning value(s) – applied to all glyphs in the Coverage table. |

Single Adjustment Positioning Format 2: Array of positioning values

A SinglePosFormat2 subtable provides an array of ValueRecords that contains one positioning value for each glyph in the Coverage table. This format is more flexible than Format 1, but it requires more space in the font file.

For example, assume that the Cyrillic script will be used in left-justified text. For all glyphs, Format 2 could define position adjustments for left side bearings to align the left edges of the paragraphs. To achieve this, the Coverage table would list every glyph in the script, and the SinglePosFormat2 subtable would define a ValueRecord for each covered glyph. Correspondingly, each ValueRecord would specify an xPlacement adjustment value for the left side bearing.

NOTE	A single ValueFormat applies to all ValueRecords defined in a SinglePos subtable.  In this example, if xPlacement is the only value that a ValueRecord needs to optically align the glyphs, then the X_PLACEMENT flag will be the only flag set in the valueFormat field of the subtable.

As in Format 1, the Format 2 subtable consists of a format identifier (posFormat), an offset to a Coverage table that defines the glyphs to be adjusted by the positioning values (coverageOffset), and the Value Format flags field (valueFormat) that describes the amount and kinds of data in the ValueRecords. In addition, the Format 2 subtable includes:

- A count of the ValueRecords (valueCount). One ValueRecord is defined for each glyph in the Coverage table.

- An array of ValueRecords that specify positioning values (valueRecords). Because the array follows the Coverage Index order, the first ValueRecord applies to the first glyph listed in the Coverage table, and so on.

Example 3 at the end of this clause shows how to adjust the spacing of three dash glyphs with a SinglePosFormat2 subtable.

SinglePosFormat2 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of SinglePos subtable. |
| uint16 | valueFormat | Defines the types of data in the ValueRecords. |
| uint16 | valueCount | Number of ValueRecords – must equal glyphCount in the Coverage table. |
| ValueRecord | valueRecords [valueCount] | Array of ValueRecords – positioning values applied to glyphs. |

##### Lookup Type 2: Pair adjustment positioning subtable

A pair adjustment positioning subtable (PairPos) is used to adjust the placement or advance of two glyphs in relation to one another – for instance, to specify kerning data for pairs of glyphs. Compared to a typical kerning table, however, a PairPos subtable offers more flexibility and precise control over glyph positioning. The PairPos subtable can adjust each glyph in a pair independently in both the X and Y directions, and it can explicitly describe the particular type of adjustment applied to each glyph. In addition, a PairPos subtable can use Device tables to subtly adjust glyph positions at each font size and device resolution.

PairPos subtables can be either of two formats: one that identifies glyphs individually by index (Format 1), and one that identifies glyphs by class (Format 2).

Pair Adjustment Positioning Format 1:Adjustments for glyph pairs

Format 1 uses glyph indices to access positioning data for one or more specific pairs of glyphs. All pairs are specified in the order determined by the layout direction of the text.

NOTE	For text written from right to left, the right-most glyph will be the first glyph in a pair; conversely, for text written from left to right, the left-most glyph will be first.

A PairPosFormat1 subtable contains a format identifier (posFormat) and two ValueFormat fields:

- valueFormat1 applies to the ValueRecords for the first glyph in each pair. The single ValueFormat field applies to ValueRecords for all first glyphs. If valueFormat1 is set to zero (0), the corresponding glyph has no ValueRecord and, therefore, should not be repositioned.

- valueFormat2 applies to the ValueRecords for the second glyph in each pair. The single ValueFormat field applies to ValueRecords for all second glyphs. If valueFormat2 is set to 0, then the second glyph of the pair is the "next" glyph for which a lookup should be performed.

A PairPos subtable also defines an offset to a Coverage table (coverageOffet) that lists the indices of the first glyphs in each pair. More than one pair can have the same first glyph, but the Coverage table will list that glyph only once.

The subtable also contains an array of offsets to PairSet tables (pairSetOffsets) and a count of the defined tables (pairSetCount). The PairSet array contains one offset for each glyph listed in the Coverage table and uses the same order as the Coverage Index.

PairPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of PairPos subtable-only the first glyph in each pair. |
| uint16 | valueFormat1 | Defines the types of data in ValueRecord1 – for the first glyph in the pair (may be zero). |
| uint16 | valueFormat2 | Defines the types of data in ValueRecord2 – for the second glyph in the pair (may be zero). |
| uint16 | pairSetCount | Number of PairSet tables. |
| Offset16 | pairSetOffsets [pairSetCount] | Array of offsets to PairSet tables. Offsets are from beginning of PairPos subtable, ordered by Coverage Index. |

A PairSet table enumerates all the glyph pairs that begin with a covered glyph. An array of PairValueRecords (PairValueRecord) contains one record for each pair and lists the records sorted by the glyph ID of the second glyph in each pair. The pairValueCount field specifies the number of PairValueRecords in the set.

PairSet table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | pairValueCount | Number of PairValueRecords |
| PairValueRecord | pairValueRecords [pairValueCount] | Array of PairValueRecords, ordered by glyph ID of the second glyph. |

A PairValueRecord specifies the second glyph in a pair (secondGlyph) and defines a ValueRecord for each glyph (valueRecord1 and valueRecord2). If valueFormat1 in the PairPos subtable is set to zero (0), valueRecord1 will be empty; similarly, if valueFormat2 is 0, valueRecord2 will be empty.

Example 4 at the end of this clause shows a PairPosFormat1 subtable that defines two cases of pair kerning.

PairValueRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | secondGlyph | Glyph ID of second glyph in the pair (first glyph is listed in the Coverage table). |
| ValueRecord | valueRecord1 | Positioning data for the first glyph in the pair. |
| ValueRecord | valueRecord2 | Positioning data for the second glyph in the pair. |

Pair Adjustment Positioning Format 2: Class pair adjustment

Format 2 defines a pair as a set of two glyph classes and modifies the positions of all the glyphs in a class. For example, this format is useful in Japanese scripts that apply specific kerning operations to all glyph pairs that contain punctuation glyphs. One class would be defined as all glyphs that may be coupled with punctuation marks, and the other classes would be groups of similar punctuation glyphs.

In a PairPosFormat2 subtable, glyph classes are defined using a Class Definition table, defined in subclause 6.2.

The PairPos Format2 subtable begins with a format identifier (posFormat) and an offset to a Coverage table (coverageOffset), measured from the beginning of the PairPos subtable. The Coverage table lists the indices of the first glyphs that may appear in each glyph pair. More than one pair may begin with the same glyph, but the Coverage table lists the glyph index only once.

A PairPosFormat2 subtable also includes two ValueFormat fields:

- valueFormat1 applies to the ValueRecords for the first glyph in each pair. The single ValueFormat field applies to ValueRecords for all first glyphs. If valueFormat1 is set to zero (0), then the ValueRecords for the first glyph will be empty and, therefore, the first glyph is not repositioned.

- valueFormat2 applies to the ValueRecords for the second glyph in each pair. The single ValueFormat field applies to ValueRecords for all second glyphs. If valueFormat2 is set to 0, then the ValueRecords for the second glyph of the pair will be empty, the second glyph is not repositioned, and it becomes the "next" glyph for which a lookup is performed.

PairPosFormat2 requires that each glyph in all pairs be assigned to a class, which is identified by an integer called a class value. Pairs are then represented in a two-dimensional array as sequences of two class values. Multiple pairs can be represented in one Format 2 subtable.

A PairPosFormat2 subtable contains offsets (classDef1Offset, classDef2Offset) to two class definition tables: one that assigns class values to all the first glyphs in all pairs (classDef1), and one that assigns class values to all the second glyphs in all pairs (classDef2). If both glyphs in a pair use the same class definition, the offset value can be the same for classDef1 as for classDef2, but they are not required to be the same. The subtable also specifies the number of glyph classes defined in classDef1 (class1Count) and in classDef2 (class2Count), including Class 0.

For each class identified in the ClassDef1 table, a Class1Record enumerates all pairs that contain a particular class as a first component. The Class1Record array stores all Class1Records according to class value.

NOTE	Class1Records are not tagged with a class value identifier. Instead, the index value of a Class1Record in the array defines the class value represented by the record. For example, the first Class1Record enumerates pairs that begin with a Class 0 glyph, the second Class1Record enumerates pairs that begin with a Class1 glyph, and so on.

PairPosFormat2 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of PairPos subtable |
| uint16 | valueFormat1 | ValueRecord definition – for the first glyph of the pair (may be zero) |
| uint16 | valueFormat2 | ValueRecord definition – for the second glyph of the pair (may be zero) |
| Offset16 | classDef1Offset | Offset to ClassDef table, from beginning of PairPos subtable – for the first glyph of the pair |
| Offset16 | classDef2Offset | Offset to ClassDef table, from beginning of PairPos subtable – for the second glyph of the pair |
| uint16 | class1Count | Number of classes in ClassDef1 table – includes Class0 |
| uint16 | class2Count | Number of classes in ClassDef2 table – includes Class0 |
| Class1Record | class1Record [class1Count] | Array of Class1 records, ordered by classes in classDef1 |

Each Class1Record contains an array of Class2Records (class2Record), which also are ordered by class value. One Class2Record must be declared for each class in the classDef2 table, including Class 0.

Class1Record

| Type | Name | Description |
| --- | --- | --- |
| Class2Record | class2Records[class2Count] | Array of Class2 records, ordered by classes in classDef 2 |

A Class2Record consists of two ValueRecords, one for the first glyph in a class pair (valueRecord1) and one for the second glyph (valueRecord2). If the PairPos subtable has a value of zero (0) for valueFormat1 or valueFormat2, then the corresponding record (valueRecord1 or valueRecord2) will be empty – that is, not present. For example, if valueFormat1 is zero, then the Class2Record will begin with and consist solely of valueRecord2. The text-processing client must be aware of the variable nature of the Class2Record and use the valueFormat1 and valueFormat2 fields to determine the size and content of the Class2Record.

Example 5 at the end of this clause demonstrates pair kerning with glyph classes in a PairPosFormat2 subtable.

Class2Record

| Type | Name | Description |
| --- | --- | --- |
| ValueRecord | valueRecord1 | Positioning for first glyph – empty if ValueFormat1 = 0 |
| ValueRecord | valueRecord2 | Positioning for second glyph – empty if ValueFormat2 = 0 |

##### Lookup Type 3: Cursive attachment positioning subtable

Some cursive fonts are designed so that adjacent glyphs join when rendered with their default positioning. However, if positioning adjustments are needed to join the glyphs, a cursive attachment positioning (CursivePos) subtable can describe how to connect the glyphs by aligning two anchor points: the designated exit point of a glyph, and the designated entry point of the following glyph.

Positioning adjustments from anchor alignment may be either horizontal or vertical. Note that the positioning effects in the text-layout direction (horizontal, for horizontal layout) work differently for than for the cross-stream direction (vertical, in horizontal layout):

- For adjustments in the line-layout direction, the layout engine adjusts the advance of the first glyph (in logical order). This effectively moves the second glyph relative to the first so that the anchors are aligned in that direction.

- For the cross-stream direction, placement of one glyph is adjusted to make the anchors align. Which glyph is adjusted is determined by the rightToLeft flag in the parent lookup table: if the rightToLeft flag is clear, the second glyph is adjusted to align anchors with the first glyph; if the rightToLeft flag is set, the first glyph is adjusted to align anchors with the second glyph.

Note that, if the rightToLeft lookup flag is set, then the last glyph in the connected sequence keeps its initial position in the cross-stream direction relative to the baseline, and the cross-stream positions of the preceding, connected glyphs are adjusted.

The subtable has one format: CursivePosFormat1.

Cursive attachment positioning Format1: Cursive attachment

The CursivePosFormat1 subtable begins with a format identifier (posFormat) and an offset to a Coverage table (coverageOffset), which lists all the glyphs that define cursive attachment data. In addition, the subtable contains one EntryExitRecord for each glyph listed in the Coverage table, a count of those records (entryExitCount), and an array of those records in the same order as the Coverage Index (entryExitRecords).

CursivePosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of CursivePos subtable. |
| uint16 | entryExitCount | Number of EntryExit records. |
| EntryExitRecord | entryExitRecord[entryExitCount] | Array of EntryExit records, in Coverage Index order. |

Each EntryExitRecord consists of two offsets: one to an Anchor table that identifies the entry point on the glyph (entryAnchorOffset), and an offset to an Anchor table that identifies the exit point on the glyph (exitAnchorOffset). (For a complete description of the Anchor table, see the end of this subclause.)

To position glyphs using the CursivePosFormat1 subtable, a text-processing client aligns the ExitAnchor point of a glyph with the EntryAnchor point of the following glyph. If no corresponding anchor point exists, either the EntryAnchor or ExitAnchor offset may be NULL.

At the end of this clause, Example 6 describes cursive glyph attachment in the Urdu language.

EntryExitRecord

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | entryAnchorOffset | Offset to EntryAnchor table, from beginning of CursivePos subtable (may be NULL). |
| Offset16 | exitAnchorOffset | Offset to ExitAnchor table, from beginning of CursivePos subtable (may be NULL). |

##### Lookup Type 4: Mark-to-Base attachment positioning subtable

The MarkToBase attachment (MarkBasePos) subtable is used to position combining mark glyphs with respect to base glyphs. For example, the Arabic, Hebrew, and Thai scripts combine vowels, diacritical marks, and tone marks with base glyphs.

In the MarkBasePos subtable, every mark glyph has an anchor point and is also assigned to a mark class. Each base glyph then defines an anchor point for each class of marks it uses. When a mark is combined with a given base, the mark placement is adjusted so that the mark anchor is aligned with the base anchor for the applicable mark class. Placement of the base glyph and advances of both glyphs are not affected.

For example, assume two mark classes: all marks positioned above base glyphs (Class 0), and all marks positioned below base glyphs (Class 1). In this case, each base glyph that uses these marks would define two anchor points, one for attaching the mark glyphs listed in Class 0, and one for attaching the mark glyphs listed in Class 1.

A mark class is identified by a specific integer. Within the MarkBasePos subtable, the anchor definition of each mark, and the assignment of each mark to a mark class, is provided using a MarkArray table in combination with a mark Coverage table. First, a mark Coverage table specifies all of the mark glyphs covered by the subtable. Then, for every mark in the Coverage table, the MarkArray table has a corresponding MarkRecord that defines the anchor and class assignment for the mark. The MarkArray table and MarkRecord are defined later in this subclause.

The MarkToBase Attachment subtable has one format: MarkBasePosFormat1.

Mark-to-Base attachment positioning Format1: Mark-to-Base attachment point

The MarkBasePosFormat1 subtable begins with a format identifier (posFormat) and offsets (markCoverageOffset, baseCoverageOffset) to two Coverage tables: one that lists all the mark glyphs referenced in the subtable (markCoverage), and one that lists all the base glyphs referenced in the subtable (baseCoverage).

The MarkBasePosFormat1 subtable also contains an offset (markArrayOffset) to a MarkArray table. For each mark glyph in the mark Coverage table, a MarkRecord in the MarkArray table specifies its class and an offset to the Anchor table that describes the mark’s attachment point. The classCount field specifies the total number of distinct mark classes defined in all the MarkRecords.

The MarkBasePosFormat1 subtable also contains an offset to a BaseArray table (baseArrayOffset), which defines for each base glyph an array of anchors, one for each mark class.

MarkBasePosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier-format = 1 |
| Offset16 | markCoverageOffset | Offset to MarkCoverage table, from beginning of MarkBasePos subtable |
| Offset16 | baseCoverageOffset | Offset to BaseCoverage table, from beginning of MarkBasePos subtable |
| uint16 | markClassCount | Number of classes defined for marks |
| Offset16 | markArrayOffset | Offset to MarkArray table, from beginning of MarkBasePos subtable |
| Offset16 | baseArrayOffset | Offset to BaseArray table, from beginning of MarkBasePos subtable |

The BaseArray table consists of an array (baseRecords) and count (baseCount) of BaseRecords. The array stores the BaseRecords in the same order as the baseCoverage Index. Each base glyph in the BaseCoverage table has a BaseRecord.

BaseArray table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | baseCount | Number of BaseRecords |
| BaseRecord | baseRecords[baseCount] | Array of BaseRecords, in order of baseCoverage Index |

A BaseRecord declares one Anchor table for each mark class (including Class 0) identified in the MarkRecords of the MarkArray. Each Anchor table specifies one attachment point used to attach all the marks in a particular class to the base glyph. A BaseRecord contains an array of offsets to Anchor tables (BaseAnchor). The zero-based array of offsets defines the entire set of attachment points each base glyph uses to attach marks. The offsets to Anchor tables are ordered by mark class.

NOTE	Anchor tables are not tagged with class value identifiers. Instead, the index value of an Anchor table in the array defines the class value represented by the Anchor table.

Example 7 at the end of this clause defines mark positioning above and below base glyphs with a MarkBasePosFormat1 subtable.

BaseRecord

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | baseAnchorOffset[markClassCount] | Array of offsets (one per mark class) to Anchor tables. Offsets are from beginning of BaseArray table, ordered by class. |

##### Lookup Type 5: Mark-to-Ligature attachment positioning subtable

The MarkToLigature attachment (MarkLigPos) subtable is used to position combining mark glyphs with respect to ligature base glyphs. With MarkToBase attachment, described previously, each base glyph has an attachment point defined for each class of marks. MarkToLigature attachment is similar, except that each ligature glyph is defined to have multiple components (in a virtual sense — not actual glyphs), and each component has a separate set of attachment points defined for the different mark classes.

As a result, a ligature glyph may have multiple base attachment points for one class of marks. For a given mark assigned to a particular class, the appropriate base attachment point is determined by which ligature component the mark is associated with. This is dependent on the original character string and subsequent character- or glyph-sequence processing, not the font data alone. While a text-layout client is performing any character-based preprocessing or any glyph-substitution operations using the GSUB table, the text-layout client must keep track of associations of marks to particular ligature-glyph components.

The MarkLigPos subtable can be used to define multiple mark-to-ligature attachments. In the subtable, every mark glyph has an anchor point and is associated with a class of marks. As with MarkToBase attachment, mark anchors and class assignments are defined using a MarkArray table in combination with a mark Coverage table. Every ligature glyph specifies a two-dimensional array of data: for each component in a ligature, an array of anchor points is defined, one for each class of marks.

For example, assume two mark classes: all marks positioned above base glyphs (Class 0), and all marks positioned below base glyphs (Class 1). In this case, each component of a base ligature glyph may define two anchor points, one for attaching the mark glyphs listed in Class 0, and one for attaching the mark glyphs listed in Class 1. Alternatively, if the language system does not allow marks on the second component, the first ligature component may define two anchor points, one for each class of marks, and the second ligature component may define no anchor points.

As with mark-to-base attachment, when a mark is combined with a given ligature base, the mark placement is adjusted so that the mark anchor is aligned with the applicable base anchor. Placement of the base glyph and advances of both glyphs are not affected.

To position a combining mark using a MarkToLigature attachment subtable, the text-processing client must work backward from the mark to the preceding ligature glyph. To correctly access the subtables, the client must keep track of the component associated with the mark. Aligning the attachment points combines the mark and ligature.

The MarkToLigature attachment subtable has one format: MarkLigPosFormat1.

Mark-to-Ligature attachment positioning Format1: Mark-to-Ligature Attachment

The MarkLigPosFormat1 subtable begins with a format identifier (posFormat) and two offsets (markCoverageOffset, baseCoverageOffset) to Coverage tables that list all the mark glyphs (markCoverage) and Ligature glyphs (ligatureCoverage) referenced in the subtable.

The MarkLigPosFormat1 subtable also contains an offset to a MarkArray table (markArrayOffset). For each mark glyph in the mark Coverage table, a MarkRecord in the MarkArray table specifies its class and an offset to the Anchor table that describes the mark’s attachment point. The markClassCount field specifies the total number of distinct mark classes defined in all the MarkRecords.

The MarkLigPosFormat1 subtable also contains an offset to a LigatureArray table (ligatureArrayOffset), which defines for each ligature glyph the two-dimensional array of anchor data: one anchor per ligature component per mark class.

MarkLigPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | markCoverageOffset | Offset to markCoverage table, from beginning of MarkLigPos subtable |
| Offset16 | ligatureCoverageOffset | Offset to ligatureCoverage table, from beginning of MarkLigPos subtable |
| uint16 | markClassCount | Number of defined mark classes |
| Offset16 | markArrayOffset | Offset to MarkArray table, from beginning of MarkLigPos subtable |
| Offset16 | ligatureArrayOffset | Offset to LigatureArray table, from beginning of MarkLigPos subtable |

The LigatureArray table contains a count (ligatureCount) and an array of offsets (ligatureAttachOffsets) to LigatureAttach tables. The ligatureAttachOffsets array lists the offsets to LigatureAttach tables, one for each ligature glyph listed in the ligatureCoverage table, in the same order as the ligatureCoverage index.

LigatureArray table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | ligatureCount | Number of LigatureAttach table offsets |
| Offset16 | ligatureAttachOffsets [ligatureCount] | Array of offsets to LigatureAttach tables. Offsets are from beginning of LigatureArray table, ordered by ligatureCoverage Index |

Each LigatureAttach table consists of an array (componentRecords) and count (componentCount) of the component glyphs in a ligature. The array stores the ComponentRecords in the same order as the components in the ligature. The order of the records also corresponds to the writing direction – that is, the logical direction – of the text. For text written left to right, the first component is on the left; for text written right to left, the first component is on the right.

LigatureAttach table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | componentCount | Number of ComponentRecords in this ligature. |
| ComponentRecord | componentRecords[componentCount] | Array of Component records, ordered in writing direction. |

A ComponentRecord, one for each component in the ligature, contains an array of offsets (ligatureAnchorOffsets) to the Anchor tables that define all the attachment points used to attach marks to the component. For each mark class (including Class 0) identified in the MarkArray records, an Anchor table specifies the point used to attach all the marks in a particular class to the ligature base glyph, relative to the component.

In a ComponentRecord, the zero-based ligatureAnchorOffsets array lists offsets to Anchor tables by mark class. If a component does not define an attachment point for a particular class of marks, then the offset to the corresponding Anchor table will be NULL.

Example 8 at the end of this clause shows a MarkLisPosFormat1 subtable used to attach mark accents to a ligature glyph in the Arabic script.

ComponentRecord

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | ligatureAnchorOffsets [markClassCount] | Array of offsets (one per class) to Anchor tables. Offsets are from beginning of LigatureAttach table, ordered by class (may be NULL) |

##### Lookup Type 6: Mark-to-Mark attachment positioning subtable

The MarkToMark attachment (MarkMarkPos) subtable is identical in form to the MarkToBase attachment subtable, although its function is different. MarkToMark attachment defines the position of one mark relative to another mark as when, for example, positioning tone marks with respect to vowel diacritical marks in Vietnamese.

The attaching mark is mark1, and the base mark being attached to is mark2. In the MarkMarkPos subtable, every mark1 glyph has an anchor attachment point and is assigned to a class of marks. As for mark glyphs in a MarkToBase attachment, the anchor point and class assignment of mark1 glyphs is defined using a MarkArray table in combination with a mark1 Coverage table. Each mark2 glyph defines an anchor point for each class of marks.

For example, assume two mark1 classes: all marks positioned to the left of mark2 glyphs (Class 0), and all marks positioned to the right of mark2 glyphs (Class 1). Each mark2 glyph that uses these marks defines two anchor points: one for attaching the mark1 glyphs listed in Class 0, and one for attaching the mark1 glyphs listed in Class 1.

The mark2 glyph that combines with a mark1 glyph is the glyph preceding the mark1 glyph in glyph string order (skipping glyphs according to LookupFlags). The subtable applies precisely when that mark2 glyph is covered by mark2Coverage. To combine the mark glyphs, the placement of the mark1 glyph is adjusted such that the relevant attachment points coincide. Advance widths are not affected. The input context for MarkToBase, MarkToLigature and MarkToMark positioning tables is the mark that is being positioned. If a sequence contains several marks, a lookup may act on it several times, to position them.

The MarkToMark attachment subtable has one format: MarkMarkPosFormat1.

Mark-to-Mark attachment positioning Format1: Mark-to-Mark attachment

The MarkMarkPosFormat1 subtable begins with a format identifier (posFormat) and two offsets (mark1CoverageOffset, mark2CoverageOffset) to Coverage tables: one that lists all the mark1 glyphs referenced in the subtable (mark1Coverage), and one that lists all the mark2 glyphs referenced in the subtable (mark2Coverage).

The subtable also has an offset to a MarkArray table for the mark1 glyphs (mark1ArrayOffset). For each mark glyph in the mark1Coverage table, a MarkRecord in the MarkArray table specifies its class and an offset to the Anchor table that describes the mark's attachment point. The markClassCount field specifies the total number of distinct mark classes defined in all the MarkRecords.

The MarkMarkPosFormat1 subtable also has an offset to a MarkArray table for mark2 glyph (mark2ArrayOffset), which defines for each mark2 glyph an array of anchors, one for each mark1 mark class.

MarkMarkPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | mark1CoverageOffset | Offset to Combining Mark Coverage table, from beginning of MarkMarkPos subtable |
| Offset16 | mark2CoverageOffset | Offset to Base Mark Coverage table, from beginning of MarkMarkPos subtable |
| uint16 | markClassCount | Number of Combining Mark classes defined |
| Offset16 | mark1ArrayOffset | Offset to MarkArray table for Mark1, from beginning of MarkMarkPos subtable |
| Offset16 | mark2ArrayOffset | Offset to Mark2Array table for Mark2, from beginning of MarkMarkPos subtable |

The Mark2Array table contains one Mark2Record for each mark2Coverage table. It stores the records in the same order as the mark2Coverage Index.

Mark2Array table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | mark2Count | Number of Mark2 records |
| Mark2Record | mark2Records [mark2Count] | Array of Mark2Records, in Coverage order. |

Each Mark2Record contains an array of offsets to Anchor tables (mark2AnchorOffsets). The zero-based array of offsets, measured from the beginning of the Mark2Array table, defines the entire set of mark2 attachment points used to attach mark1 glyphs to a specific mark2 glyph. The Anchor tables referenced in the mark2AnchorOffsets array are ordered by mark1 class value.

A Mark2Record declares one Anchor table for each mark class (including Class 0) identified in the MarkRecords of the MarkArray. Each Anchor table specifies one mark2 attachment point used to attach all the mark1 glyphs in a particular class to the mark2 glyph.

Example 9 at the end of the subclause shows a MarkMarkPosFormat1 subtable for attaching one mark to another in the Arabic script.

Mark2Record

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | mark2AnchorOffsets [markClassCount] | Array of offsets (one per class) to Anchor tables. Offsets are from beginning of Mark2Array table, in class order |

##### Lookup Type 7: Contextual positioning subtables

A Contextual Positioning (ContextPos) subtable defines the most powerful type of glyph positioning lookup. It describes glyph positioning in context so a text-processing client can adjust the position of one or more glyphs within a certain pattern of glyphs. Each subtable describes one or more "input" glyph sequences and one or more positioning operations to be performed on that sequence.

ContextPos subtables can have one of three formats, which closely mirror the formats used for contextual glyph substitution. One format applies to specific glyph sequences (Format 1), one defines the context in terms of glyph classes (Format 2), and the third format defines the context in terms of sets of glyphs (Format 3).

All three formats of ContextPos subtables specify positioning data in a PosLookupRecord. A description of that record follows.

Position Lookup Record

All contextual positioning subtables specify the positioning data in a PosLookupRecord. Each record contains a SequenceIndex, which indicates where the positioning operation will occur in the glyph sequence. In addition, a lookupListIndex field identifies the lookup to be applied at the glyph position specified by the sequenceIndex.

The order in which lookups are applied to the entire glyph sequence, called the "design order", can be significant, so PosLookupRecord data should be defined accordingly.

The contextual substitution subtables defined in Examples 10, 11, and 12 show PosLookupRecords.

PosLookupRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | sequenceIndex | Index (zero-based) to input glyph sequence |
| uint16 | lookupListIndex | Index (zero-based) into the LookupList for the Lookup table to apply to that position in the glyph sequence |

Context Positioning Subtable Format 1: Simple Glyph Contexts

Format 1 defines the context for a glyph positioning operation as a particular sequence of glyphs. For example, a context could be <To>, <xyzabc>, <!?*#@>, or any other glyph sequence.

Within the context, Format 1 identifies particular glyph-sequence positions (not glyph indices) as the targets for specific adjustments. When a text-processing client locates a context in a string of glyphs, it makes the adjustment by applying the lookup data defined for a targeted position at that location.

For example, suppose that accent mark glyphs above lowercase x-height vowel glyphs must be lowered when an overhanging capital letter glyph precedes the vowel. When the client locates this context in the text, the subtable identifies the position of the accent mark and a lookup index. A lookup specifies a positioning action that lowers the accent mark over the vowel so that it does not collide with the overhanging capital.

ContextPosFormat1 defines the context in two places. A Coverage table specifies the first glyph in the input sequence, and a PosRule table identifies the remaining glyphs. To describe the context used in the previous example, the Coverage table lists the glyph index of the first component of the sequence (the overhanging capital), and a PosRule table defines indices for the lowercase x-height vowel glyph and the accent mark.

A single ContextPosFormat1 subtable may define more than one context glyph sequence. If different context sequences begin with the same glyph, then the Coverage table should list the glyph only once because all first glyphs in the table must be unique. For example, if three contexts each start with an "s" and two start with a "t", then the Coverage table will list one "s" and one "t".

For each context, a PosRule table lists all the glyphs, in glyph-sequence order, that follow the first glyph. The table also contains an array of PosLookupRecords that specify the positioning lookup data for each glyph position (including the first glyph position) in the context.

All the PosRule tables defining contexts that begin with the same first glyph are grouped together and defined in a PosRuleSet table. For example, the PosRule tables that define the three contexts that begin with an "s" are grouped in one PosRuleSet table, and the PosRule tables that define the two contexts that begin with a "t" are grouped in a second PosRuleSet table. Each unique glyph listed in the Coverage table shall have a PosRuleSet table that defines all the PosRule tables for a covered glyph.

To locate a context glyph sequence, the text-processing client searches the Coverage table each time it begins processing a new glyph-sequence context. If the first glyph is covered, the client reads the corresponding PosRuleSet table and examines each PosRule table in the set to determine whether the rest of the context defined there matches the subsequent glyphs in the glyph sequence. If the context and glyph sequence match, the client finds the target glyph position, applies the lookup for that position, and completes the positioning action.

A ContextPosFormat1 subtable contains a format identifier (posFormat), an offset to a Coverage table (coverageOffset), a count of the number of PosRuleSets that are defined (posRuleSetCount), and an array of offsets to the PosRuleSet tables (posRuleSetOffsets). As mentioned, one PosRuleSet table shall be defined for each glyph listed in the Coverage table.

In the posRuleSetOffsets array, offsets for the PosRuleSet tables are ordered in the Coverage index order. The first PosRuleSet in the array applies to the first glyph ID listed in the Coverage table, the second PosRuleSet in the array applies to the second glyph ID listed in the Coverage table, and so on.

ContextPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of ContextPos subtable |
| uint16 | posRuleSetCount | Number of PosRuleSet tables |
| Offset16 | posRuleSetOffsets [posRuleSetCount] | Array of offsets to PosRuleSet tables. Offsets are from beginning of ContextPos subtable, ordered by Coverage Index |

There is one PosRuleSet table for each glyph in the Coverage table. Each PosRuleSet table corresponds to a given glyph in the Coverage table, and describes all of the contexts that begin with that glyph.

A PosRuleSet table consists of an array of offsets to PosRule tables (posRuleOffsets), ordered by preference, and a count of the PosRule tables defined in the set (posRuleCount).

PosRuleSet Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posRuleCount | Number of PosRule tables |
| Offset16 | posRuleOffsets [posRuleCount] | Array of offsets to PosRule tables. Offsets are from beginning of PosRuleSet, ordered by preference. |

A PosRule table consists of a count of the glyphs to be matched in the input context sequence (glyphCount), including the first glyph in the sequence, and an array of glyph indices that describe the context (inputSequence). The Coverage table specifies the index of the first glyph in the context, and the inputSequence array begins with the second glyph in the context sequence. As a result, the first element in the inputSequence array corresponds with glyph-sequence position index one (1), not zero (0). The inputSequence array lists the indices in the order the corresponding glyphs appear in the text, in writing direction (logical) order. For text written from right to left, the right-most glyph will be first; conversely, for text written from left to right, the left-most glyph will be first.

A PosRule table also contains a count of the positioning operations to be performed on the input glyph sequence (posCount) and an array of PosLookupRecords (posLookupRecords). Each record specifies a position in the input glyph sequence and a LookupList index to the positioning lookup to be applied there. The array should list records in design order, or the order the lookups should be applied to the entire glyph sequence.

Example 10 at the end of this subclause demonstrates glyph kerning in context with a ContextPosFormat1 subtable.

PosRule Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Number of glyphs in the Input glyph sequence |
| uint16 | posCount | Number of PosLookupRecords |
| uint16 | inputSequence [glyphCount - 1] | Array of input glyph IDs – starting with the second glyph |
| PosLookupRecord | posLookupRecords[posCount] | Array of positioning lookups, in design order |

Context positioning subtable Format 2: Class-based Glyph Contexts

Format 2, more flexible than Format 1, describes class-based context positioning. For this format, a specific integer, called a class value, must be assigned to each glyph in all context glyph sequences. Contexts are then defined as sequences of class values. This subtable may define more than one context.

In a ContextPosFormat2 subtable, glyph classes are defined using a Class Definition table, defined in subclause 6.2.

To clarify the notion of class-based context rules, suppose that certain sequences of three glyphs need special kerning. The glyph sequences consist of an uppercase glyph that overhangs on the right side, a punctuation mark glyph, and then a quote glyph. In this case, the set of uppercase glyphs would constitute one glyph class (Class1), the set of punctuation mark glyphs would constitute a second glyph class (Class 2), and the set of quote mark glyphs would constitute a third glyph class (Class 3). The input context might be specified with a context rule (PosClassRule) that describes "the set of glyph strings that form a sequence of three glyph classes, one glyph from Class 1, followed by one glyph from Class 2, followed by one glyph from Class 3".

Each ContextPosFormat2 subtable contains an offset to a class definition table (classDefOffset), which defines the class values of all glyphs in the input contexts that the subtable describes. Generally, a unique classDef will be declared in each instance of the ContextPosFormat2 subtable that is included in a font, even though several Format 2 subtables may share classDef tables. Classes are exclusive sets; a glyph cannot be in more than one class at a time. The output glyphs that replace the glyphs in the context sequence do not need class values because they are specified elsewhere by glyph ID.

The ContextPosFormat2 subtable also contains a format identifier (posFormat) and defines an offset to a Coverage table (coverageOffset). For this format, the Coverage table lists indices for the complete set of glyphs (not glyph classes) that may appear as the first glyph of any class-based context. In other words, the Coverage table contains the list of glyph indices for all the glyphs in all classes that may be first in any of the context class sequences. For example, if the contexts begin with a Class 1 or Class 2 glyph, then the Coverage table will list the indices of all Class 1 and Class 2 glyphs.

A ContextPosFormat2 subtable also defines an array of offsets to the PosClassSet tables (posClassSetOffsets), along with a count (including Class 0) of the PosClassSet tables (posClassSetCount). In the array, the PosClassSet tables are ordered by ascending class value (from 0 to posClassSetCount - 1).

The posClassSetOffsets array contains one offset for each glyph class, including Class 0. PosClassSets are not explicitly tagged with a class value; rather, the index value of the PosClassSet in the PosClassSet array defines the class that a PosClassSet represents. A PosClassSet enumerates all the PosClassRules that begin with a particular glyph class.

For example, the first PosClassSet listed in the array contains all the PosClassRules that define contexts beginning with Class 0 glyphs, the second PosClassSet contains all PosClassRules that define contexts beginning with Class 1 glyphs, and so on. If no PosClassRules begin with a particular class (that is, if a PosClassSet contains no PosClassRules), then the offset to that particular PosClassSet in the PosClassSet array will be set to NULL.

ContextPosFormat2 Subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of ContextPos subtable |
| Offset16 | classDefOffset | Offset to ClassDef table, from beginning of ContextPos subtable |
| uint16 | posClassSetCount | Number of PosClassSet tables |
| Offset16 | posClassSetOffsets [posClassSetCount] | Array of offsets to PosClassSet tables. Offsets are from beginning of ContextPos subtable, ordered by class (may be NULL) |

All the PosClassRules that define contexts beginning with the same class are grouped together and defined in a PosClassSet table. Consequently, the PosClassSet table identifies the class of a context's first component.

A PosClassSet enumerates all the PosClassRules that begin with a particular glyph class. For instance, PosClassSet0 represents all the PosClassRules that describe contexts starting with Class 0 glyphs, and PosClassSet1 represents all the PosClassRules that define contexts starting with Class 1 glyphs.

Each PosClassSet table consists of a count of the PosClassRules defined in the PosClassSet (posClassRuleCount) and an array of offsets to PosClassRule tables (posClassRuleOffsets). The PosClassRule tables are ordered by preference in the posClassRuleOffsets array of the PosClassSet.

PosClassSet Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posClassRuleCount | Number of PosClassRule tables |
| Offset16 | posClassRuleOffsets[posClassRuleCount] | Array of offsets to PosClassRule tables. Offsets are from beginning of PosClassSet, ordered by preference. |

For each context, a PosClassRule table contains a count of the glyph classes in a given context (glyphCount), including the first class in the context sequence. A class array lists the classes, beginning with the second class, that follow the first class in the context. The first class listed indicates the second position in the context sequence.

NOTE	Text order depends on the writing direction of the text. For text written from right to left, the right-most glyph will be first. Conversely, for text written from left to right, the left-most glyph will be first.

The values specified in the classes array are those defined in the classDef table. For example, consider a context consisting of the sequence: Class 2, Class 7, Class 5, Class 0. The classes array will read: classes[0] = 7, classes[1] = 5, and classes[2] = 0. The first class in the sequence, Class 2, is defined by the index into the posClassSetOffsets array. The total number and sequence of glyph classes listed in the classes array must match the total number and sequence of glyph classes contained in the input context.

A PosClassRule also contains a count of the positioning operations to be performed on the context (PosCount) and an array of PosLookupRecords (PosLookupRecord) that supply the positioning data. For each position in the context that requires a positioning operation, a PosLookupRecord specifies a LookupList index and a position in the input glyph class sequence where the lookup is applied. The PosLookupRecord array lists PosLookupRecords in design order, or the order in which lookups are applied to the entire glyph sequence.

Example 11 at the end of this clause demonstrates a ContextPosFormat2 subtable that uses glyph classes to modify accent positions in glyph strings.

PosClassRule Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Number of glyphs to be matched |
| uint16 | posCount | Number of PosLookupRecords |
| uint16 | classes [glyphCount - 1] | Array of classes to be matched to the input glyph sequence, beginning with the second glyph position |
| PosLookupRecord | posLookupRecords[posCount] | Array of PosLookupRecords-in design order |

Context positioning subtable Format 3: Coverage-based glyph contexts

Format 3, coverage-based context positioning, defines a context rule as a sequence of coverages. Each position in the sequence may specify a different Coverage table for the set of glyphs that matches the context pattern. With Format 3, the glyph sets defined in the different Coverage tables may intersect, unlike Format 2 which specifies fixed class assignments for the lookup (they cannot be changed at each position in the context sequence) and exclusive classes (a glyph cannot be in more than one class at a time).

For example, consider an input context that contains an uppercase glyph (position 0), followed by any narrow uppercase glyph (position 1), and then another uppercase glyph (position 2). This context requires three Coverage tables, one for each position:

- In position 0, the first position, the Coverage table lists the set of all uppercase glyphs.

- In position 1, the second position, the Coverage table lists the set of all narrow uppercase glyphs, which is a subset of the glyphs listed in the Coverage table for position 0.

- In position 2, the Coverage table lists the set of all uppercase glyphs again.

NOTE 1	Both position 0 and position 2 can use the same Coverage table.

Unlike Formats 1 and 2, this format defines only one context rule at a time. It consists of a format identifier (posFormat), a count of the number of glyphs in the sequence to be matched (glyphCount), and an array of Coverage offsets that describe the input context sequence (coverageOffsets).

NOTE 2	The Coverage tables referenced in the coverageOffsets array must be listed in text order according to the writing direction. For text written from right to left, the right-most glyph will be first. Conversely, for text written from left to right, the left-most glyph will be first.

The subtable also contains a count of the positioning operations to be performed on the input Coverage sequence (posCount) and an array of PosLookupRecords (posLookupRecords) in design order, or the order in which lookups are applied to the entire glyph sequence.

Example 12 at the end of this subclause changes the positions of math sign glyphs in math equations with a ContextPosFormat3 subtable.

ContextPosFormat3 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 3 |
| uint16 | glyphCount | Number of glyphs in the input sequence |
| uint16 | posCount | Number of PosLookupRecords |
| Offset16 | coverageOffsets [glyphCount] | Array of offsets to Coverage tables, from beginning of ContextPos subtable |
| PosLookupRecord | posLookupRecords [posCount] | Array of positioning lookups, in design order |

##### LookupType 8: Chaining contextual positioning subtable

A Chaining Contextual Positioning subtable (ChainContextPos) describes glyph positioning in context with an ability to look back and/or look ahead in the sequence of glyphs. The design of the Chaining Contextual Positioning subtable is parallel to that of the Contextual Positioning subtable, including the availability of three formats.

To specify the context, the coverage table lists the first glyph in the input sequence, and the ChainPosRule subtable defines the rest. Once a covered glyph is found at position i, the client reads the corresponding ChainPosRuleSet table and examines each table to determine if it matches the surrounding glyphs in the text. There is a match if the string <backtrack sequence>+<input sequence>+<lookahead sequence> matches with the glyphs at position i - BacktrackGlyphCount in the text.

If there is a match, then the client finds the target glyphs for positioning and performs the operations. Please note that (just like in the ContextPosFormat1 subtable) these lookups are required to operate within the range of text from the covered glyph to the end of the input sequence. No positioning operations can be defined for the backtracking sequence or the lookahead sequence.

To clarify the ordering of glyph arrays for input, backtrack and lookahead sequences, the following illustration is provided. Input sequence match begins at i where the input sequence match begins. The backtrack sequence is ordered beginning at i - 1 and increases in offset value as one moves away from i. The lookahead sequence begins after the input sequence and increases in logical order.

| Logical order - | a | b | c | d | e | f | g | h | i | j |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | i |  |  |  |  |  |
| Input sequence - |  |  |  |  | 0 | 1 |  |  |  |  |
| Backtrack sequence - | 3 | 2 | 1 | 0 |  |  |  |  |  |  |
| Lookahead sequence - |  |  |  |  |  |  | 0 | 1 | 2 | 3 |

Chaining context positioning Format 1: Simple glyph contexts

This Format is identical to Format 1 of Context Positioning lookup except that the PosRule table is replaced with a ChainPosRule table. (Correspondingly, the ChainPosRuleSet table differs from the PosRuleSet table only in that it lists offsets to ChainPosRule tables instead of PosRule tables; and the ChainContextPosFormat1 subtable lists offsets to ChainPosRuleSet tables instead of PosRuleSet subtables.)

ChainContextPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of ContextPos subtable |
| uint16 | chainPosRuleSetCount | Number of ChainPosRuleSet tables |
| Offset16 | chainPosRuleSetOffsets [chainPosRuleSetCount] | Array of offsets to ChainPosRuleSet tables. Offsets are from beginning of ChainContextPos subtable, ordered by Coverage Index |

There is one ChainPosRuleSet table for each glyph in the Coverage table. Each ChainPosRuleSet table corresponds to a given glyph in the Coverage table, and describes all of the contexts that begin with that glyph.

A ChainPosRuleSet table consists of an array of offsets to ChainPosRule tables (chainPosRuleOffsets), ordered by preference, and a count of the ChainPosRule tables defined in the set (chainPosRuleCount).

ChainPosRuleSet table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | chainPosRuleCount | Number of ChainPosRule tables |
| Offset16 | chainPosRuleOffsets [chainPosRuleCount] | Array of offsets to ChainPosRule tables. Offsets are from beginning of ChainPosRuleSet, ordered by preference |

ChainPosRule table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | backtrackGlyphCount | Total number of glyphs in the backtrack sequence |
| uint16 | backtrackSequence [backtrackGlyphCount] | Array of backtracking glyph IDs |
| uint16 | inputGlyphCount | Total number of glyphs in the input sequence - includes the first glyph |
| uint16 | inputSequence [inputGlyphCount - 1] | Array of input glyph IDs - starts with second glyph) |
| uint16 | lookaheadGlyphCount | Total number of glyphs in the look ahead sequence |
| uint16 | lookAheadSequence [lookAheadGlyphCount] | Array of lookahead glyph IDs |
| uint16 | posCount | Number of PosLookupRecords |
| PosLookupRecord | posLookupRecords [posCount] | Array of PosLookupRecords, in design order |

Chaining context positioning Format 2: Class-based glyph contexts

This lookup Format is parallel to the Context Positioning format 2, with PosClassSet subtable changed to ChainPosClassSet subtable, and PosClassRule subtable changed to ChainPosClassRule subtable.

In a ChainContextPosFormat2 subtable, glyph classes are defined using a Class Definition table, defined in subclause 6.2.

To chain contexts, three classes are used in the glyph ClassDef table: backtrackClassDef, inputClassDef, and lookaheadClassDef.

ChainContextPosFormat2 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of ChainContextPos subtable |
| Offset16 | backtrackClassDefOffset | Offset to ClassDef table containing backtrack sequence context, from beginning of ChainContextPos subtable |
| Offset16 | inputClassDefOffset | Offset to ClassDef table containing input sequence context, from beginning of ChainContextPos subtable |
| Offset16 | lookaheadClassDefOffset | Offset to ClassDef table containing lookahead sequence context, from beginning of ChainContextPos subtable |
| uint16 | chainPosClassSetCnt | Number of ChainPosClassSet tables |
| Offset16 | chainPosClassSetOffsets [chainPosClassSetCnt] | Array of offsets to ChainPosClassSet tables. Offsets are from beginning of ChainContextPos subtable, ordered by input class (may be NULL) |

All the ChainPosClassRules that define contexts beginning with the same class are grouped together and defined in a ChainPosClassSet table. Consequently, the ChainPosClassSet table identifies the class of a context's first component.

ChainPosClassSet table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | chainPosClassRuleCount | Number of ChainPosClassRule tables |
| Offset16 | chainPosClassRuleOffsets [chainPosClassRuleCount] | Array of offsets to ChainPosClassRule tables. Offsets are from beginning of ChainPosClassSet, ordered by preference |

ChainPosClassRule table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | backtrackGlyphCount | Total number of glyphs in the backtrack sequence |
| uint16 | backtrackSequence [backtrackGlyphCount] | Array of backtrack-sequence classes |
| uint16 | inputGlyphCount | Total number of classes in the input sequence - includes the first class |
| uint16 | inputSequence [inputGlyphCount - 1] | Array of input classes to be matched to the input glyph sequence, beginning with the second glyph position |
| uint16 | lookaheadGlyphCount | Total number of classes in the look ahead sequence |
| uint16 | lookAheadSequence [lookAheadGlyphCount] | Array of lookahead-sequence classes |
| uint16 | posCount | Number of PosLookupRecords |
| PosLookupRecord | posLookupRecords [posCount] | Array of PosLookupRecords, in design order |

Chaining context positioning Format 3: Coverage-based glyph contexts

Format 3 defines a chaining context rule as a sequence of Coverage tables. Each position in the sequence may define a different Coverage table for the set of glyphs that matches the context pattern. With Format 3, the glyph sets defined in the different Coverage tables may intersect, unlike Format 2 which specifies fixed class assignments (identical for each position in the backtrack, input, or lookahead sequence) and exclusive classes (a glyph cannot be in more than one class at a time).

NOTE	The order of the Coverage tables listed in the Coverage array must follow the writing direction. For text written from right to left, then the right-most glyph will be first. Conversely, for text written from left to right, the left-most glyph will be first.

The subtable also contains a count of the positioning operations to be performed on the input Coverage sequence (posCount) and an array of PosLookupRecords (posLookupRecords) in design order: that is, the order in which lookups should be applied to the entire glyph sequence.

ChainContextPosFormat3 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 3 |
| uint16 | backtrackGlyphCount | Number of glyphs in the backtracking sequence |
| Offset16 | backtrackCoverageOffsets [backtrackGlyphCount] | Array of offsets to coverage tables in backtracking sequence, in glyph sequence order |
| uint16 | inputGlyphCount | Number of glyphs in input sequence |
| Offset16 | inputCoverageOffsets [inputGlyphCount] | Array of offsets to coverage tables in input sequence, in glyph sequence order |
| uint16 | lookaheadGlyphCount | Number of glyphs in lookahead sequence |
| Offset16 | lookaheadCoverageOffsets [lookaheadGlyphCount] | Array of offsets to coverage tables in lookahead sequence, in glyph sequence order |
| uint16 | posCount | Number of PosLookupRecords |
| PosLookupRecord | posLookupRecords [posCount] | Array of PosLookupRecords, in design order |

##### LookupType 9: Extension positioning

This lookup provides a mechanism whereby any other lookup type's subtables are stored at a 32-bit offset location in the 'GPOS' table. This is needed if the total size of the subtables exceeds the 16-bit limits of the various other offsets in the 'GPOS' table. In this document, the subtable stored at the 32-bit offset location is termed the "extension" subtable.

This subtable type uses one format: ExtensionPosFormat1.

Extension Positioning Subtable Format1

ExtensionPosFormat1 subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | posFormat | Format identifier: format = 1. |
| uint16 | extensionLookupType | Lookup type of subtable referenced by ExtensionOffset (i.e. the extension subtable). |
| Offset32 | extensionOffset | Offset to the extension subtable, of lookup type ExtensionLookupType, relative to the start of the ExtensionPosFormat1 subtable. |

ExtensionLookupType must be set to any lookup type other than 9. All subtables in a LookupType 9 lookup must have the same ExtensionLookupType. All offsets in the extension subtables are set in the usual way, i.e. relative to the extension subtables themselves.

When an OFF layout engine encounters a LookupType 9 Lookup table, it shall:

- Proceed as though the Lookup table's LookupType field were set to the extensionLookupType of the subtables.

- Proceed as though each extension subtable referenced by extensionOffset replaced the LookupType 9 subtable that referenced it.

#### Shared tables: Value record, Anchor table and MarkArray table

Several lookup subtables described earlier in this clause refer to one or more of the same tables for positioning data: ValueRecord, Anchor table, and MarkArray table. These shared tables are described here.

Example 14 at the end of the clause uses a ValueFormat table and ValueRecord to specify positioning values in GPOS.

##### Value Record

GPOS subtables use ValueRecords to describe all the variables and values used to adjust the position of a glyph or set of glyphs. A ValueRecord may define any combination of X and Y values (in design units) to add to (positive values) or subtract from (negative values) the placement and advance values provided in the font. In non-variable fonts, a ValueRecord may also contain an offset to a Device table for each of the specified values. In a variable font, it may also contain an offset to a VariationIndex table for each of the specified values.

Note that all fields of a ValueRecord are optional: to save space, only the fields that are required need be included in a given instance. Because the GPOS table uses ValueRecords for many purposes, the sizes and contents of ValueRecords may vary from subtable to subtable. A ValueRecord is always accompanied by a ValueFormat flags field that specifies which of the ValueRecord fields is present. If a ValueRecord specifies more than one value, the values should be listed in the order shown in the ValueRecord definition. If the associated ValueFormat flags indicate that a field is not present, then the next present field follows immediately after the last preceding, present field. The text-processing client must be aware of the flexible and variable nature of ValueRecords in the GPOS table.

ValueRecord

| Type | Name | Description |
| --- | --- | --- |
| int16 | xPlacement | Horizontal adjustment for placement, in design units |
| int16 | yPlacement | Vertical adjustment for placement, in design units |
| int16 | xAdvance | Horizontal adjustment for advance, in design units - only used for horizontal layout |
| int16 | yAdvance | Vertical adjustment for advance, in design units - only used for vertical layout |
| Offset16 | xPlaDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for horizontal placement, from beginning of the immediate parent table (SinglePos or pairPosFormat2 lookup subtable, PairSet table within a PairPosFormat1 lookup subtable) – may be NULL. |
| Offset16 | yPlaDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for vertical placement, from beginning of the immediate parent table (SinglePos or pairPosFormat2 lookup subtable, PairSet table within a PairPosFormat1 lookup subtable) – may be NULL. |
| Offset16 | xAdvDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for horizontal advance, from beginning of the immediate parent table (SinglePos or pairPosFormat2 lookup subtable, PairSet table within a PairPosFormat1 lookup subtable) – may be NULL. |
| Offset16 | yAdvDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for vertical advance, from beginning of the immediate parent table (SinglePos or pairPosFormat2 lookup subtable, PairSet table within a PairPosFormat1 lookup subtable) – may be NULL. |

NOTE 1	Device tables are used only in non-variable fonts, while VariationIndex tables are used only in variable fonts.

In variable fonts, VariationIndex tables shall be used to reference variation data for any placement or advance value that requires adjustment for different variation instances.

NOTE 2	While a separate reference to a VariationIndex table is required for each value that requires variation, two or more values that require the same variation data can have offsets that point to the same VariationIndex table, and two or more VariationIndex tables can reference the same variation data entries.

NOTE 3	If no VariationIndex table is used for a particular placement or advance value, then that value is used for all variation instances.

A ValueFormat flags field defines the types of positioning adjustment data that ValueRecords specify. SinglePos subtables will have ValueRecords for a single glyph position in a glyph sequence; PairPos subtables will have separate ValueRecords for two glyph positions. In a given subtable, the same ValueFormat applies to every ValueRecord for a given glyph position.

The ValueFormat determines whether the ValueRecords:

- Apply to placement, advance, or both.

- Apply to the horizontal position (X coordinate), the vertical position (Y coordinate), or both.

- May refer to one or more Device tables (in non-variable fonts) or VariationIndex tables (in variable fonts) for any of the specified values.

Each defined bit in the ValueFormat flags corresponds to a field in the ValueRecord and increases the size of the ValueRecord by 2 bytes. A ValueFormat of 0x0000 corresponds to an empty ValueRecord, which indicates no positioning changes.

To identify the fields in each ValueRecord, the ValueFormat flags shown below are used. To specify multiple fields with a ValueFormat, the bit settings of the relevant fields are added with a logical OR operation.

For example, to adjust the left-side bearing of a glyph, the ValueFormat will be 0x0001, and the ValueRecord will define the xPlacement value. To adjust the advance width of a different glyph, the ValueFormat will be 0x0004, and the ValueRecord will describe the xAdvance value. To adjust both the xPlacement and xAdvance of a set of glyphs, the ValueFormat will be 0x0005, and the ValueRecord will specify both values in the order they are listed in the ValueRecord definition.

ValueFormat flags

| Mask | Name | Description |
| --- | --- | --- |
| 0x0001 | X_Placement | Includes horizontal adjustment for placement |
| 0x0002 | Y_Placement | Includes vertical adjustment for placement |
| 0x0004 | X_Advance | Includes horizontal adjustment for advance |
| 0x0008 | Y_Advance | Includes vertical adjustment for advance |
| 0x0010 | X_Placement_Device | Includes Device table (non-variable font) / VariationIndex table (variable font) for horizontal placement |
| 0x0020 | Y_Placement_Device | Includes Device table (non-variable font) / VariationIndex table (variable font) for vertical placement |
| 0x0040 | X_Advance_Device | Includes Device table (non-variable font) / VariationIndex table (variable font) for horizontal advance |
| 0x0080 | Y_Advance_Device | Includes Device table (non-variable font) / VariationIndex table (variable font) for vertical advance |
| 0xFF000 | Reserved | For future use (set to zero) |

##### Anchor tables

A GPOS table uses anchor points to position one glyph with respect to another. Each glyph defines an anchor point, and the text-processing client attaches the glyphs by aligning their corresponding anchor points.

To describe an anchor point, an Anchor table can use one of three formats. The first format uses X and Y coordinates, in design units, to specify a location for the anchor point in relation to the location of the outline for a given glyph. The other two formats refine the location of the anchor point using contour points (Format 2) or Device tables (Format 3). In a variable font, the third format uses a VariationIndex table (a variant of a Device table) to reference variation data for adjustment of the anchor position for the current variation instance, as needed.

Anchor table Format 1: Design Units

AnchorFormat1 consists of a format identifier (anchorFormat) and a pair of design-unit coordinates (xCoordinate and yCoordinate) that specify the location of the anchor point. This format has the benefits of small size and simplicity, but the anchor point cannot be hinted to adjust its position for different device resolutions.

Example 15 at the end of this clause uses AnchorFormat1.

AnchorFormat1 table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | anchorFormat | Format identifier, = 1 |
| int16 | xCoordinate | Horizontal value, in design units |
| int16 | yCoordinate | Vertical value, in design units |

Anchor table Format 2: Design Units Plus Contour Point

Like AnchorFormat1, AnchorFormat2 specifies a format identifier (anchorFormat) and a pair of design unit coordinates for the anchor point (xCoordinate and yCoordinate).

For fine-tuning the location of the anchor point, AnchorFormat2 also provides an index to a glyph contour point (anchorPoint) that is on the outline of a glyph. Hinting can be used to move the contour anchor point. In the rendered text, the anchor point will provide the final positioning data for a given ppem size.

Example 16 at the end of this clause uses AnchorFormat2.

AnchorFormat2 table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | anchorFormat | Format identifier, = 2 |
| int16 | xCoordinate | Horizontal value, in design units |
| int16 | yCoordinate | Vertical value, in design units |
| uint16 | anchorPoint | Index to glyph contour point |

Anchor table Format 3: Design Units Plus Device or VariationIndex Tables

Like AnchorFormat1, AnchorFormat3 specifies a format identifier (anchorFormat) and locates an anchor point (xcoordinate and ycoordinate). And, like AnchorFormat 2, it permits fine adjustments in variable fonts to the coordinate values. However, AnchorFormat3 uses Device tables, rather than a contour point, for this adjustment.

With a Device table, a client can adjust the position of the anchor point for any font size and device resolution. AnchorFormat3 can specify offsets to Device tables for the X coordinate (xDeviceTable) and the Y coordinate (yDeviceTable). If only one coordinate requires adjustment, the offset to the Device table for the other coordinate may be set to NULL.

In variable fonts, AnchorFormat3 shall be used to reference variation data to adjust anchor points for different variation instances, if needed. In this case, AnchorFormat3 specifies an offset to a VariationIndex table, which is a variant of the Device table used for variations. If no VariationIndex table is used for a particular anchor point X or Y coordinate, then that value is used for all variation instances. While separate VariationIndex table references are required for each value that requires variation, two or more values that require the same variation-data values can have offsets that point to the same VariationIndex table, and two or more VariationIndex tables can reference the same variation data entries.

Example 17 at the end of the clause shows an AnchorFormat3 table.

AnchorFormat3 table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | anchorFormat | Format identifier, = 3 |
| int16 | xCoordinate | Horizontal value, in design units |
| int16 | yCoordinate | Vertical value, in design units |
| Offset16 | xDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for X coordinate, from beginning of Anchor table (may be NULL) |
| Offset16 | yDeviceOffset | Offset to Device table (non-variable font) / VariationIndex table (variable font) for Y coordinate, from beginning of Anchor table (may be NULL) |

##### MarkArray table

The MarkArray table defines the class and the anchor point for a mark glyph. Three GPOS subtable types – MarkToBase attachment, MarkToLigature attachment, and MarkToMark attachment – use the MarkArray table to specify data for attaching marks.

The MarkArray table contains a count of the number of MarkRecords (markCount) and an array of those records (markRecord). Each mark record defines the class of the mark and an offset to the Anchor table that contains data for the mark.

A class value can be zero (0), but the MarkRecord must explicitly assign that class value. (This differs from the Class Definition table, in which all glyphs not assigned class values automatically belong to Class 0.) The GPOS subtables that refer to MarkArray tables use the class assignments for indexing zero-based arrays that contain data for each mark class.

In Example 18 at the end of the clause, a MarkArray table and two MarkRecords define two mark classes.

MarkArray table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | markCount | Number of MarkRecords |
| MarkRecord | markRecords [markCount] | Array of MarkRecords, ordered by corresponding glyphs in the associated mark Coverage order |

MarkRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | markClass | Class defined for the associated mark |
| Offset16 | markAnchorOffset | Offset to Anchor table, from beginning of MarkArray table |

#### GPOS subtable examples

The rest of this clause describes examples of all the GPOS subtable formats, including each of the three formats available for contextual positioning. All the examples reflect unique parameters described below, but the samples provide a useful reference for building subtables specific to other situations.

All the examples have three columns showing hex data, source, and comments.

Example 1: GPOS header table

Example 1 shows a typical GPOS Header table definition with offsets to a ScriptList, FeatureList, and LookupList.

Example 1

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | GPOSHeader TheGPOSHeader | GPOSHeader table definition |
| 00010000 | 0x00010000 | major / minor version |
| 000A | TheScriptList | Offset to ScriptList table |
| 001E | TheFeatureList | Offset to FeatureList table |
| 002C | TheLookupList | Offset to LookupList table |

Example 2: SinglePosFormat1 subtable

Example 2 uses the SinglePosFormat1 subtable to lower the Y placement of subscript glyphs in a font. The LowerSubscriptsSubTable defines one Coverage table, called LowerSubscriptsCoverage, which lists one range of glyph indices for the numeral/numeric subscript glyphs. The subtable's ValueFormat setting indicates that the ValueRecord specifies only the YPlacement value, lowering each subscript glyph by 80 design units.

Example 2

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | SinglePosFormat1 LowerSubscriptsSubTable | SinglePos subtable definition |
| 0001 | 1 | posFormat |
| 0008 | LowerSubscriptsCoverage | Offset to Coverage table |
| 0002 | 0x0002 | valueFormat: Y_PLACEMENT |
|  | ValueRecord |  |
| FFB0 | -80 | move Y position down |
|  | CoverageFormat2 LowerSubscriptsCoverage | Coverage table definition |
| 0002 | 2 | coverageFormat: ranges |
| 0001 | 1 | rangeCount |
|  | rangeRecords[0] |  |
| 01B3 | ZeroSubscriptGlyphID | Start, first glyph ID |
| 01BC | NineSubscriptGlyphID | End, last glyph ID |
| 0000 | 0 | StartCoverageIndex |

Example 3: SinglePosFormat2 subtable

This example uses a SinglePosFormat2 subtable to adjust the spacing of three dash glyphs by different amounts. The em dash spacing changes by 10 units, the en dash spacing changes by 25 units, and spacing of the standard dash changes by 50 units.

The DashSpacingSubTable contains one Coverage table with three dash glyph indices, plus an array of ValueRecords, one for each covered glyph. The ValueRecords use the same ValueFormat to modify the XPlacement and XAdvance values of each glyph. The ValueFormat bit setting of 0x0005 is produced by adding the XPlacement and XAdvance bit settings.

Example 3

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | SinglePosFormat2 DashSpacingSubTable | SinglePos subtable definition |
| 0002 | 2 | posFormat |
| 0014 | DashSpacingCoverage | Offset to Coverage table |
| 0005 | 0x0005 | valueFormat: X_PLACEMENT ¦¦ X_ADVANCE |
| 0003 | 3 | valueCount |
|  | valueRecords[0] | for dash glyph |
| 0032 | 50 | X_PLACEMENT |
| 0032 | 50 | X_ADVANCE |
|  | valueRecords[1] | for en dash glyph |
| 0019 | 25 | X_PLACEMENT |
| 0019 | 25 | X_ADVANCE |
|  | valueRecords[2] | for em dash glyph |
| 000A | 10 | X_PLACEMENT |
| 000A | 10 | X_ADVANCE |
|  | CoverageFormat1 DashSpacingCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0003 | 3 | glyphCount |
| 004F | DashGlyphID | glyphArray[0] |
| 0125 | EnDashGlyphID | glyphArray[1] |
| 0129 | EmDashGlyphID | glyphArray[2] |

Example 4: PairPosFormat1 subtable

Example 4 uses a PairPosFormat1 subtable to kern two glyph pairs - "Po" and "To" - by adjusting the XAdvance of the first glyph and the XPlacement of the second glyph. Two ValueFormats are defined, one for each glyph. The subtable contains a Coverage table that lists the index of the first glyph in each pair. It also contains an offset to a PairSet table for each covered glyph.

A PairSet table defines an array of PairValueRecords to specify all the glyph pairs that contain a covered glyph as their first component. In this example, the PPairSet table has one PairValueRecord that identifies the second glyph in the "Po" pair and two ValueRecords, one for the first glyph and one for the second. The TPairSet table also has one PairValueRecord that lists the second glyph in the "To" pair and two ValueRecords, one for each glyph.

Example 4

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | PairPosFormat1 PairKerningSubTable | PairPos subtable definition |
| 0001 | 1 | posFormat |
| 001E | PairKerningCoverage | Offset to Coverage table |
| 0004 | 0x0004 | valueFormat1: X_ADVANCE only |
| 0001 | 0x0001 | ValueFormat2: X_PLACEMENT only |
| 0002 | 2 | pairSetCount |
| 000E | PPairSetTable | pairSetOffsets[0] |
| 0016 | TPairSetTable | pairSetOffsets [1] |
|  | PairSetTable PPairSetTable | PairSet table definition |
| 0001 | 1 | pairValueCount |
|  | pairValueRecords[0] |  |
| 0059 | LowercaseOGlyphID | SecondGlyph |
|  | valueRecord1 | ValueRecord for first glyph |
| FFE2 | -30 | xAdvance |
|  | valueRecord2 | ValueRecord for second glyph |
| FFEC | -20 | xPlacement |
|  | PairSetTable PairSetTable | PairSet table definition |
| 0001 | 1 | pairValueCount |
|  | pairValueRecords[0] |  |
| 0059 | LowercaseOGlyphID | secondGlyph |
|  | valueRecord1 | ValueRecord for first glyph |
| FFD8 | -40 | xAdvance |
|  | valueRecord2 | ValueRecord for second glyph |
| FFE7 | -25 | xPlacement |
|  | CoverageFormat1 PairKerningCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 002D | UppercasePGlyphID | glyphArray[0] |
| 0031 | UppercaseTGlyphID | glyphArray[1] |

Example 5: PairPosFormat2 subtable

The PairPosFormat2 subtable in this example defines pairs composed of two glyph classes. Two ClassDef tables are defined, one for each glyph class. The first glyph in each pair is in a class of lowercase glyphs with diagonal shapes (v, w, y), defined Class1 in the LowercaseClassDef table. The second glyph in each pair is in a class of punctuation glyphs (comma and period), defined in Class1 in the PunctuationClassDef table. The Coverage table only lists the indices of the glyphs in the LowercaseClassDef table since they occupy the first position in the pairs.

The subtable defines two Class1Records for the classes defined in LowecaseClassDef, including Class0. Each record, in turn, defines a Class2Record for each class defined in PunctuationClassDef, including Class0. The Class2Records specify the positioning adjustments for the glyphs.

The pairs are kerned by reducing the XAdvance of the first glyph by 50 design units. Because no positioning change applies to the second glyph, its ValueFormat2 is set to 0, to indicate that Value2 is empty for each pair.

Since no pairs begin with Class0 or Class2 glyphs, all the ValueRecords referenced in Class1Record[0] contain values of 0 or are empty. However, Class1Record[1] does define an XAdvance value in its Class2Record[1] for kerning all pairs that contain a Class1 glyph followed by a Class2 glyph.

Example 5

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | PairPosFormat2 PunctKerningSubTable | PairPos subtable definition |
| 0002 | 2 | posFormat |
| 0018 | PunctKerningCoverage | Offset to Coverage table |
| 0004 | 0x0004 | valueFormat1: X_ADVANCE only |
| 0000 | 0 | valueFormat2: no ValueRecord for second glyph |
| 0022 | LowercaseClassDef | Offset to ClassDef1 table for first class in pair |
| 0032 | PunctuationClassDef | Offset to ClassDef2 table for second class in pair |
| 0002 | 2 | Class1Count |
| 0002 | 2 | Class2Count |
|  | class1Records[0] | First Class1Record, for contexts beginning with class 0 |
|  | class2Records[0] | First Class2Record for class1Records[0]; valueFormat2 is zero, so no valueRecord2 |
|  | valueRecord1 |  |
| 0000 | 0 | xAdvance: no change for first glyph |
|  | class2Records[1] | no valueRecord2 |
|  | valueRecord1 |  |
| 0000 | 0 | xAdvance: no change for first glyph |
|  | class1Records[1] | for contexts beginning with Class1 |
|  | class2Records[0] | no contexts with Class0 as second glyph; no valueRecord2 |
|  | valueRecord1 |  |
| 0000 | 0 | xAdvance: no change for first glyph |
|  | class2Records[1] | contexts with Class1 as second glyph; no valueRecord2 |
|  | valueRecord1 |  |
| FFCE | -50 | xAdvance: move punctuation glyph left |
|  | CoverageFormat1 PunctKerningCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0003 | 3 | glyphCount |
| 0046 | LowercaseVGlyphID | glyphArray[0] |
| 0047 | LowercaseWGlyphID | glyphArray[1] |
| 0049 | LowercaseYGlyphID | glyphArray[2] |
|  | ClassDefFormat2 LowercaseClassDef | ClassDef table definition |
| 0002 | 2 | classFormat: ranges |
| 0002 | 2 | classRangeCount |
|  | classRangeRecords[0] |  |
| 0046 | LowercaseVGlyphID | startGlyphID |
| 0047 | LowercaseWGlyphID | endGlyphID |
| 0001 | 1 | class |
|  | classRangeRecords[1] |  |
| 0049 | LowercaseYGlyphID | startGlyphID |
| 0049 | LowercaseYGlyphID | endGlyphID |
| 0001 | 1 | class |
|  | ClassDefFormat2 PunctuationClassDef | ClassDef table definition |
| 0002 | 2 | classFormat: ranges |
| 0001 | 1 | classRangeCount |
|  | classRangeRecords[0] |  |
| 006A | PeriodPunctGlyphID | startGlyphID |
| 006B | CommaPunctGlyphID | endGlyphID |
| 0001 | 1 | class |

Example 6: CursivePosFormat1 subtable

In Example 6, the Urdu language system uses a CursivePosFormat1 subtable to attach glyphs along a diagonal baseline that descends from right to left. Two glyphs are defined with attachment data and listed in the Coverage table-the Kaf and Ha glyphs. For each glyph, the subtable contains an EntryExitRecord that defines offsets to two Anchor tables, an entry attachment point, and an exit attachment point. Each Anchor table defines X and Y coordinate values. To render Urdu down and diagonally, the entry point's Y coordinate is above the baseline and the exit point's Y coordinate is located below the baseline.

Example 6

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | CursivePosFormat1 DiagonalWritingSubTable | CursivePos subtable definition |
| 0001 | 1 | posFormat |
| 000E | DiagonalWritingCoverage | offset to Coverage table |
| 0002 | 2 | entryExitCount |
|  | entryExitRecords[0] | EntryExitRecord for Kaf glyph |
| 0016 | KafEntryAnchor | offset to EntryAnchor table |
| 001C | KafExitAnchor | offset to ExitAnchor table |
|  | entryExitRecords[1] | EntryExitRecord for Ha glyph |
| 0022 | HaEntryAnchor | offset to EntryAnchor table |
| 0028 | HaExitAnchor | offset to ExitAnchor table |
|  | CoverageFormat1 DiagonalWritingCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 0203 | KafGlyphID | glyphArray[0] |
| 027E | HaGlyphID | glyphArray[1] |
|  | AnchorFormat1 KafEntryAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 05DC | 1500 | xCoordinate |
| 002C | 44 | yCoordinate |
|  | AnchorFormat1 KafExitAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0000 | 0 | xCoordinate |
| FFEC | -20 | yCoordinate |
|  | AnchorFormat1 HaEntryAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 05DC | 1500 | xCoordinate |
| 002C | 44 | yCoordinate |
|  | AnchorFormat1 HaExitAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0000 | 0 | xCoordinate |
| FFEC | -20 | yCoordinate |

Example 7: MarkBasePosFormat1 subtable

The MarkBasePosFormat1 subtable in Example 7 defines one Arabic base glyph, Tah, and two Arabic mark glyphs: a fathatan mark above the base glyph, and a kasra mark below the base glyph. The BaseGlyphsCoverage table lists the base glyph, and the MarkGlyphsCoverage table lists the mark glyphs.

Each mark is also listed in the MarkArray, along with its attachment point data and a mark Class value. The MarkArray defines two mark classes: Class0 consists of marks located above base glyphs, and Class1 consists of marks located below base glyphs.

The BaseArray defines attachment data for base glyphs. In this array, one BaseRecord is defined for the Tah glyph with offsets to two BaseAnchor tables, one for each class of marks. AboveBaseAnchor defines an attachment point for marks placed above the Tah base glyph, and BelowBaseAnchor defines an attachment point for marks placed below it.

Example 7

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MarkBasePosFormat1 MarkBaseAttachSubTable | MarkBasePos subtable definition |
| 0001 | 1 | posFormat |
| 000C | MarkGlyphsCoverage | offset to MarkCoverage table |
| 0014 | BaseGlyphsCoverage | offset to BaseCoverage table |
| 0002 | 2 | markClassCount |
| 001A | MarkGlyphsArray | offset to MarkArray table |
| 0030 | BaseGlyphsArray | offset to BaseArray table |
|  | CoverageFormat1 MarkGlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 0333 | fathatanMarkGlyphID | glyphArray[0] |
| 033F | kasraMarkGlyphID | glyphArray[1] |
|  | CoverageFormat1 BaseGlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 0190 | tahBaseGlyphID | glyphArray[0] |
|  | MarkArray MarkGlyphsArray | MarkArray table definition |
| 0002 | 2 | markCount |
|  | markRecords[0] | MarkRecords in Coverage index order |
| 0000 | 0 | markClass, for marks over base |
| 000A | fathatanMarkAnchor | markAnchorOffset |
|  | markRecords[1] |  |
| 0001 | 1 | markClass, for marks under |
| 0010 | kasraMarkAnchor | markAnchorOffset |
|  | AnchorFormat1 fathatanMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 015A | 346 | xCoordinate |
| FF9E | -98 | yCoordinate |
|  | AnchorFormat1 kasraMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0105 | 261 | xCoordinate |
| 0058 | 88 | yCoordinate |
|  | BaseArray BaseGlyphsArray | BaseArray table definition |
| 0001 | 1 | baseCount |
|  | baseRecords[0] |  |
| 0006 | AboveBaseAnchor | baseAnchorOffset [0] |
| 000C | BelowBaseAnchor | baseAnchorOffset [1] |
|  | AnchorFormat1 AboveBaseAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 033E | 830 | xCoordinate |
| 0640 | 1600 | yCoordinate |
|  | AnchorFormat1 BelowBaseAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 033E | 830 | xCoordinate |
| FFAD | -83 | yCoordinate |

Example 8: MarkLigPosFormat1 subtable

Example 8 uses the MarkLigPosFormat1 subtable to attach marks to a ligature glyph in the Arabic script. The hypothetical ligature is composed of three glyph components: a Lam (initial form), a meem (medial form), and a jeem (medial form). Accent marks are defined for the first two components: the sukun accent is positioned above lam, and the kasratan accent is placed below meem.

The LigGlyphsCoverage table lists the ligature glyph and the MarkGlyphsCoverage table lists the two accent marks. Each mark is also listed in the MarkArray, along with its attachment point data and a mark Class value. The MarkArray defines two mark classes: Class0 consists of marks located above base glyphs, and Class1 consists of marks located below base glyphs.

The LigGlyphsArray has an offset to one LigatureAttach table for the covered ligature glyph. This table, called LamWithMeemWithJeemLigAttach, defines a count and array of the component glyphs in the ligature. Each ComponentRecord defines offsets to two Anchor tables, one for each mark class.

In the example, the first glyph component, lam, specifies a high attachment point for positioning accents above, but does not specify a low attachment point for placing accents below. The second glyph component, meem, defines a low attachment point for placing accents below, but not above. The third component, jeem, has no attachment points since the example defines no accents for it.

Example 8

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MarkLigPosFormat1 MarkLigAttachSubTable | MarkLigPos subtable definition |
| 0001 | 1 | posFormat |
| 000C | MarkGlyphsCoverage | offset to MarkCoverage table |
| 0014 | LigGlyphsCoverage | offset to LigatureCoverage table |
| 0002 | 2 | markClassCount |
| 001A | MarkGlyphsArray | offset to MarkArray table |
| 0030 | LigGlyphsArray | offset to LigatureArray table |
|  | CoverageFormat1 MarkGlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 033C | sukunMarkGlyphID | glyphArray[0] |
| 033F | kasratanMarkGlyphID | glyphArray[1] |
|  | CoverageFormat1 LigGlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 0234 | LamWithMeemWithJeem LigatureGlyphID | glyphArray[0] |
|  | MarkArray MarkGlyphsArray | MarkArray table definition |
| 0002 | 2 | markCount |
|  | markRecords[0] | MarkRecords in Coverage index order |
| 0000 | 0 | markClass, for marks above components |
| 000A | sukunMarkAnchor | markAnchorOffset |
|  | markRecords[1] |  |
| 0001 | 1 | markClass, for marks below components |
| 0010 | kasratanMarkAnchor | markAnchorOffset |
|  | AnchorFormat1 sukunMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 015A | 346 | xCoordinate |
| FF9E | -98 | yCoordinate |
|  | AnchorFormat1 kasratanMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0105 | 261 | xCoordinate |
| 01E8 | 488 | yCoordinate |
|  | LigatureArray LigGlyphsArray | LigatureArray table definition |
| 0001 | 1 | ligatureCount |
| 0004 | LamWithMeemWithJeemLigAttach | ligatureAttachOffsets[0] |
|  | LigatureAttach LamWithMeemWithJeemLigAttach | LigatureAttach table definition |
| 0003 | 3 | componentCount |
|  | componentRecords[0] | Right-to-left text; ComponentRecords in writing-direction (logical) order: right-most glyph first |
| 000E | AboveLamAnchor | ligatureAnchorOffsets[0] – offsets ordered by mark class |
| 0000 | NULL | ligatureAnchorOffsets[1] – no attachment points for Class1 marks |
|  | componentRecords[1] |  |
| 0000 | NULL | ligatureAnchorOffsets[0] – no attachment points for Class0 marks |
| 0014 | BelowMeemAnchor | ligatureAnchorOffsets[1] – for Class1 marks (below) |
|  | componentRecords[2] |  |
| 0000 | NULL | ligatureAnchorOffsets[0] – no attachment points for Class0 marks |
| 0000 | NULL | ligatureAnchorOffsets[1] – no attachment points for Class1 marks |
|  | AnchorFormat1 AboveLamAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0271 | 625 | xCoordinate |
| 0708 | 1800 | yCoordinate |
|  | AnchorFormat1 BelowMeemAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 0178 | 376 | xCoordinate |
| FE90 | -368 | yCoordinate |

Example 9: MarkMarkPosFormat1 subtable

The MarkMarkPosFormat1 subtable in Example 9 defines two Arabic marks glyphs. The hanza mark, the base mark (Mark2), is identified in the Mark2GlyphsCoverage table. The damma mark, the attaching mark (Mark1), is defined in the Mark1GlyphsCoverage table.

Each Mark1 glyph is also listed in the Mark1Array, along with its attachment point data and a mark Class value. The Mark1GlyphsArray defines one mark class, Class0, that consists of marks located above Mark2 base glyphs. The Mark1GlyphsArray contains an offset to a dammaMarkAnchor table to specify the coordinate of the damma mark's attachment point.

The Mark2GlyphsArray table defines a count and an array of Mark2Records, one for each covered Mark2 base glyph. Each record contains an offset to a Mark2Anchor table for each Mark1 class. One Anchor table, AboveMark2Anchor, specifies a coordinate value for attaching the damma mark above the hanza base mark.

Example 9

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MarkMarkPosFormat1 MarkMarkAttachSubTable | MarkBasePos subtable definition |
| 0001 | 1 | posFormat |
| 000C | Mark1GlyphsCoverage | offset to mark1Coverage table |
| 0012 | Mark2GlyphsCoverage | offset to mark2Coverage table |
| 0001 | 1 | markClassCount |
| 0018 | Mark1GlyphsArray | offset to mark1Array table |
| 0024 | Mark2GlyphsArray | offset to mark2Array table |
|  | CoverageFormat1 Mark1GlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 0296 | dammaMarkGlyphID | glyphArray[0] |
|  | CoverageFormat1 Mark2GlyphsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 0289 | hanzaMarkGlyphID | glyphArray[1] |
|  | MarkArray Mark1GlyphsArray | MarkArray table definition |
| 0001 | 1 | markCount |
|  | markRecords[0] | MarkRecords in Coverage index order |
| 0000 | 0 | markClass – for marks above base mark |
| 0006 | dammaMarkAnchor | markAnchorOffset |
|  | AnchorFormat1 dammaMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 00BD | 189 | xCoordinate |
| FF99 | -103 | yCoordinate |
|  | Mark2Array Mark2GlyphsArray | Mark2Array table definition |
| 0001 | 1 | mark2Count |
|  | mark2Records[0] |  |
| 0004 | AboveMark2Anchor | mark2AnchorOffsets[0] |
|  | AnchorFormat1 AboveMark2Anchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 00DD | 221 | xCoordinate |
| 012D | 301 | yCoordinate |

Example 10: ContextPosFormat1 subtable and PosLookupRecord

Example 10 uses a ContextPosFormat1 subtable to adjust the spacing between three Arabic glyphs in a word. The context is the glyph sequence (from right to left): heh (initial form), thal (final form), and heh (isolated form). In the rendered word, the first two glyphs are connected, but the last glyph (the isolated form of heh), is separate. This subtable reduces the amount of space between the last glyph and the rest of the word.

The subtable contains a WordCoverage table that lists the first glyph in the word, heh (initial), and one PosRuleSet table, called WordPosRuleSet, that defines all contexts beginning with this covered glyph.

The WordPosRuleSet contains one PosRule that describes a word context of three glyphs and identifies the second and third glyphs (the first glyph is identified by the WordPosRuleSet). When a text-processing client locates this context in text, it applies a SinglePos lookup (not shown in the example) at position 2 to reduce the spacing between the glyphs.

Example 10

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextPosFormat1 MoveHehInSubtable | ContextPos subtable definition |
| 0001 | 1 | posFormat |
| 0008 | WordCoverage | offset to Coverage table |
| 0001 | 1 | posRuleSetCount |
| 000E | WordPosRuleSet | posRuleSetOffsets[0] |
|  | CoverageFormat1 WordCoverage | Coverage table offset |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 02A6 | hehInitialGlyphID | glyphArray[0] |
|  | PosRuleSet WordPosRuleSet | PosRuleSet table definition |
| 0001 | 1 | posRuleCount |
| 0004 | WordPosRule | posRuleOffsets[0] |
|  | PosRule WordPosRule | PosRule table definition |
| 0003 | 3 | glyphCount |
| 0001 | 1 | posCount |
| 02DD | thalFinalGlyphID | inputSequence [0] |
| 02C6 | hehIsolatedGlyphID | inputSequence [1] |
|  | posLookupRecords[0] |  |
| 0002 | 2 | sequenceIndex |
| 0001 | 1 | lookupListIndex |

Example 11: ContextPosFormat2 subtable

The ContextPosFormat2 subtable in Example 11 defines context strings for five glyph classes: Class1 consists of uppercase glyphs that overhang and create a wide open space on their right side; Class2 consists of uppercase glyphs that overhang and create a narrow space on their right side; Class3 contains lowercase x-height vowels; and Class4 contains accent glyphs placed over the lowercase vowels. The rest of the glyphs in the font fall into Class0.

The MoveAccentsSubtable defines two similar context strings. The first consists of a Class1 uppercase glyph followed by a Class3 lowercase vowel glyph with a Class4 accent glyph over the vowel. When this context is found in the text, the client lowers the accent glyph over the vowel so that it does not collide with the overhanging glyph shape. The second context consists of a Class2 uppercase glyph, followed by a Class3 lowercase vowel glyph with a Class4 accent glyph over the vowel. When this context is found in the text, the client increases the advance width of the uppercase glyph to expand the space between it and the accented vowel.

The MoveAccents subtable defines a MoveAccentsCoverage table that identifies the first glyphs in the two contexts and offsets to five PosClassSet tables, one for each class defined in the ClassDef table. Since no contexts begin with Class0, Class3, or Class4 glyphs, the offsets to the PosClassSet tables for these classes are NULL. PosClassSet[1] defines all contexts beginning with Class1 glyphs; it is called UCWideOverhangPosClass1Set. PosClassSet[2] defines all contexts beginning with Class2 glyphs, and it is called UCNarrowOverhangPosClass1Set.

Each PosClassSet defines one PosClassRule. The UCWideOverhangPosClass1Set uses the UCWideOverhangPosClassRule to specify the first context. The first class in this context string is identified by the PosClassSet that includes a PosClassRule, in this case Class1. The PosClassRule table lists the second and third classes in the context as Class3 and Class4. A SinglePos Lookup (not shown) lowers the accent glyph in position 3 in the context string.

The UCNarrowOverhangPosClass1Set defines the UCNarrowOverhangPosClassRule for the second context. This PosClassRule is identical to the UCWideOverhangPosClassRule, except that the first class in the context string is a Class2 lowercase glyph. A SinglePos Lookup (not shown) increases the advance width of the overhanging uppercase glyph in position 0 in the context string.

Example 11

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextPosFormat2 MoveAccentsSubtable | ContextPos subtable definition |
| 0002 | 2 | posFormat |
| 0012 | MoveAccentsCoverage | Offset to Coverage table |
| 0020 | MoveAccentsClassDef | Offset to ClassDef |
| 0005 | 5 | posClassSetCount |
| 0000 | NULL | posClassSetOffsets[0] – no contexts begin with Class0 glyphs |
| 0060 | UCWideOverhangPosClass1Set | posClassSetOffsets[1] – contexts beginning with Class1 glyphs |
| 0070 | UCNarrowOverhangPosClass2Set | posClassSetOffsets[2] – context beginning with Class2 glyphs |
| 0000 | NULL | posClassSetOffsets[3] – no contexts begin with Class3 glyphs |
| 0000 | NULL | posClassSetOffsets[4] – no contexts begin with Class4 glyphs |
|  | CoverageFormat1 MoveAccentsCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0005 | 5 | glyphCount |
| 0029 | UppercaseFGlyphID | glyphArray[0] |
| 0033 | UppercasePGlyphID | glyphArray[1] |
| 0037 | UppercaseTGlyphID | glyphArray[2] |
| 0039 | UppercaseVGlyphID | glyphArray[3] |
| 003A | UppercaseWGlyphID | glyphArray[4] |
|  | ClassDefFormat2 MoveAccentsClassDef | ClassDef table definition defines five classes = 0 (all else), 1 (T, V, W: UCUnderhang), 2 (F, P: UCOverhang), 3 (a, e, I, o, u: LCVowels), 4 (tilde, umlaut) |
| 0002 | 2 | classFormat: ranges |
| 000A | 10 | classRangeCount |
|  | classRangeRecords[0] |  |
| 0029 | UppercaseFGlyphID | startGlyphID |
| 0029 | UppercaseFGlyphID | endGlyphID |
| 0002 | 2 | class |
|  | classRangeRecords[1] |  |
| 0033 | UppercasePGlyphID | startGlyphID |
| 0033 | UppercasePGlyphID | endGlyphID |
| 0002 | 2 | class |
|  | classRangeRecords[2] |  |
| 0037 | UppercaseTGlyphID | startGlyphID |
| 0037 | UppercaseTGlyphID | endGlyphID |
| 0001 | 1 | class |
|  | classRangeRecords[3] |  |
| 0039 | UppercaseVGlyphID | startGlyphID |
| 003A | UppercaseWGlyphID | endGlyphID |
| 0001 | 1 | class |
|  | classRangeRecords[4] |  |
| 0042 | LowercaseAGlyphID | startGlyphID |
| 0042 | LowercaseAGlyphID | endGlyphID |
| 0003 | 3 | class |
|  | classRangeRecords[5] |  |
| 0046 | LowercaseEGlyphID | startGlyphID |
| 0046 | LowercaseEGlyphID | endGlyphID |
| 0003 | 3 | class |
|  | classRangeRecords[6] |  |
| 004A | LowercaseIGlyphID | startGlyphID |
| 004A | LowercaseIGlyphID | endGlyphID |
| 0003 | 3 | class |
|  | classRangeRecords[7] |  |
| 0051 | LowercaseOGlyphID | startGlyphID |
| 0051 | LowercaseOGlyphID | endGlyphID |
| 0003 | 3 | class |
|  | classRangeRecords[8] |  |
| 0056 | LowercaseUGlyphID | startGlyphID |
| 0056 | LowercaseUGlyphID | endGlyphID |
| 0003 | 3 | class |
|  | classRangeRecords[9] |  |
| 00F5 | TildeAccentGlyphID | startGlyphID |
| 00F6 | UmlautAccentGlyphID | endGlyphID |
| 0004 | 4 | class |
|  | PosClassSet UCWideOverhangPosClass1Set | PosClassSet table definition |
| 0001 | 1 | posClassRuleCount |
| 0004 | UCWideOverhangPosClassRule | posClassRuleOffsets[0] |
|  | PosClassRule UCWideOverhangPosClassRule | PosClassRule table definition |
| 0003 | 3 | glyphCount |
| 0001 | 1 | posCount |
| 0003 | 3 | classes[0] – lowercase vowel |
| 0004 | 4 | classes[1], – accent |
|  | posLookupRecords[0] |  |
| 0002 | 2 | sequenceIndex |
| 0001 | 1 | lookupListIndex – lower the accent |
|  | PosClassSet UCNarrowOverhangPosClass2Set | PosClassSet table definition |
| 0001 | 1 | posClassRuleCount |
| 0004 | UCNarrowOverhangPosClassRule | posClassRuleOffsets[0] |
|  | PosClassRule UCNarrowOverhangPosClassRule | PosClassRule table definition |
| 0003 | 3 | glyphCount |
| 0001 | 1 | posCount |
| 0003 | 3 | classes[0], – lowercase vowel |
| 0004 | 4 | classes[1], – accent |
|  | posLookupRecords[0] |  |
| 0000 | 0 | sequenceIndex |
| 0002 | 2 | lookupListIndex – increase overhang advance width |

Example 12: ContextPosFormat3 subtable

Example 12 uses a ContextPosFormat3 subtable to lower the position of math signs in math equations consisting of a lowercase descender or x-height glyph, a math sign glyph, and any lowercase glyph. Format3 is better to use for this context than the class-based Format2 because the sets of covered glyphs for positions 0 and 2 overlap.

The LowerMathSignsSubtable contains offsets to three Coverage tables (XhtDescLCCoverage, MathSignCoverage, and LCCoverage), one for each position in the context glyph string. When the client finds the context in the text stream, it applies the PosLookupRecord data at position 1 and repositions the math sign.

Example 12

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextPosFormat3 LowerMathSignsSubtable | ContextPos subtable definition |
| 0003 | 3 | posFormat |
| 0003 | 3 | glyphCount |
| 0001 | 1 | posCount |
| 0010 | XhtDescLCCoverage | coverageOffsets[0] |
| 003C | MathSignCoverage | coverageOffsets[1] |
| 0044 | LCCoverage | coverageOffsets[2] |
|  | posLookupRecords[0] |  |
| 0001 | 1 | sequenceIndex |
| 0001 | 1 | lookupListIndex |
|  | CoverageFormat1 XhtDescLCCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0014 | 20 | glyphCount |
| 0033 | LCaGlyphID | glyphArray[0] |
| 0035 | LCcGlyphID | glyphArray[1] |
| 0037 | LCeGlyphID | glyphArray[2] |
| 0039 | LCgGlyphID | glyphArray[3] |
| 003B | LCiGlyphID | glyphArray[4] |
| 003C | LCjGlyphID | glyphArray[5] |
| 003F | LCmGlyphID | glyphArray[6] |
| 0040 | LCnGlyphID | glyphArray[7] |
| 0041 | LCoGlyphID | glyphArray[8] |
| 0042 | LCpGlyphID | glyphArray[9] |
| 0043 | LCqGlyphID | glyphArray[10] |
| 0044 | LCrGlyphID | glyphArray[11] |
| 0045 | LCsGlyphID | glyphArray[12] |
| 0046 | LCtGlyphID | glyphArray[13] |
| 0047 | LCuGlyphID | glyphArray[14] |
| 0048 | LCvGlyphID | glyphArray[15] |
| 0049 | LCwGlyphID | glyphArray[16] |
| 004A | LCxGlyphID | glyphArray[17] |
| 004B | LCyGlyphID | glyphArray[18] |
| 004C | LCzGlyphID | glyphArray[19] |
|  | CoverageFormat1 MathSignCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 011E | EqualsSignGlyphID | glyphArray[0] |
| 012D | PlusSignGlyphID | glyphArray[1] |
|  | CoverageFormat2 LCCoverage | Coverage table definition |
| 0002 | 2 | coverageFormat: ranges |
| 0001 | 1 | rangeCount |
|  | rangeRecords[0] |  |
| 0033 | LCaGlyphID | startGlyphID |
| 004C | LCzGlyphID | endGlyphID |
| 0000 | 0 | startCoverageIndex |

Example 13: PosLookupRecord

The PosLookupRecord in Example 13 identifies a lookup to apply at the second glyph position in a context glyph string.

Example 13

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | PosLookupRecord PosLookupRecord[0] | PosLookupRecord definition |
| 0001 | 1 | sequenceIndex – for second glyph position |
| 0001 | 1 | lookupListIndex – apply this lookup to second glyph position |

Example 14: ValueFormat table and ValueRecord

Example 14 demonstrates how to specify positioning values in the GPOS table. Here, a SinglePosFormat1 subtable defines the ValueFormat and ValueRecord. The ValueFormat bit setting of 0x0099 says that the corresponding ValueRecord contains values for a glyph's XPlacement and YAdvance. Device tables specify pixel adjustments for these values at font sizes from 11 ppem to 15 ppem.

Example 14

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | SinglePosFormat1 OnesSubtable | SinglePos subtable definition |
| 0001 | 1 | posFormat |
| 000E | Cov | Offset to Coverage table |
| 0099 | 0x0099 | valueFormat: X_PLACEMENT + Y_ADVANCE + X_PLACEMENT_DEVICE, Y_ADVANCE_DEVICE |
| 0050 | 80 | xPlacement |
| 00D2 | 210 | yAdvance |
| 0018 | XPlaDeviceTable | xPlaDeviceOffset |
| 0020 | YAdvDeviceTable | yAdvDeviceOffset |
|  | CoverageFormat2 Cov | Coverage table definition |
| 0002 | 2 | coverageFormat: ranges |
| 0001 | 1 | rangeCount |
|  | rangeRecords[0] |  |
| 00C8 | 200 | startGlyphID |
| 00D1 | 209 | endGlyphID |
| 0000 | 0 | startCoverageIndex |
|  | DeviceTableFormat1 XPlaDeviceTable | Device Table definition |
| 000B | 11 | startSize |
| 000F | 15 | endSize – five delta values (sizes 11 to 15) |
| 0001 | 1 | deltaFormat: LOCAL_2_BIT_DELTAS |
|  | 1 | deltaValue[0]: increase 11ppem by 1 pixel |
|  | 1 | deltaValue[1]: increase 12ppem by 1 pixel |
|  | 1 | deltaValue[2]: increase 13ppem by 1 pixel |
|  | 1 | deltaValue[3]: increase 14ppem by 1 pixel |
| 5540 | 1 | deltaValue[4]: increase 15ppem by 1 pixel |
|  | DeviceTableFormat1 YAdvDeviceTable | Device Table definition |
| 000B | 11 | startSize |
| 000F | 15 | endSize |
| 0001 | 1 | deltaFormat: LOCAL_2_BIT_DELTAS |
|  | 1 | deltaValue[0]: increase 11ppem by 1 pixel |
|  | 1 | deltaValue[1]: increase 12ppem by 1 pixel |
|  | 1 | deltaValue[2]: increase 13ppem by 1 pixel |
|  | 1 | deltaValue[3]: increase 14ppem by 1 pixel |
| 5540 | 1 | deltaValue[4]: increase 15ppem by 1 pixel |

Example 15: AnchorFormat1 table

Example 15 illustrates an Anchor table for the damma mark glyph in the Arabic script. Format1 is used to specify X and Y coordinate values in design units.

Example 15

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | AnchorFormat1 dammaMarkAnchor | Anchor table definition |
| 0001 | 1 | anchorFormat: design units only |
| 00BD | 189 | xCoordinate |
| FF99 | -103 | yCoordinate |

Example 16: AnchorFormat2 table

Example 16 shows an AnchorFormat2 table for an attachment point placed above a base glyph. With this format, the coordinate value for the Anchor depends on the final position of a specific contour point on the base glyph after hinting. The coordinates are specified in design units.

Example 16

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | AnchorFormat2 AboveBaseAnchor | Anchor table definition |
| 0002 | 2 | anchorFormat: design units plus contour points |
| 0142 | 322 | xCoordinate |
| 0384 | 900 | yCoordinate |
| 000D | 13 | anchorPoint – glyph contour point index |

Example 17: AnchorFormat3 table

Example 17 shows an AnchorFormat3 table that specifies an attachment point above a base glyph. Device tables modify the X and Y coordinates of the Anchor for the point size and resolution of the output font. Here, the Device tables define pixel adjustments for font sizes from 12 ppem to 17 ppem.

Example 17

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | AnchorFormat3 AboveBaseAnchor | Anchor table definition |
| 0003 | 3 | anchorFormat: design units plus device table |
| 0117 | 279 | xCoordinate |
| 0515 | 1301 | yCoordinate |
| 000A | XDevice | xDeviceOffset (may be NULL) |
| 0014 | YDevice | yDeviceOffset (may be NULL) |
|  | DeviceTableFormat2 XDevice | Device Table definition |
| 000C | 12 | startSize |
| 0011 | 17 | endSize |
| 0002 | 2 | deltaFormat: LOCAL_4_BIT_DELTAS |
|  | 1 | deltaValue[0]: increase 12ppem by 1 pixel |
|  | 1 | deltaValue[1]: increase 13ppem by 1 pixel |
|  | 1 | deltaValue[2]: increase 14ppem by 1 pixel |
| 1111 | 1 | deltaValue[3]: increase 15ppem by 1 pixel |
|  | 2 | deltaValue[4]: increase 16ppem by 1 pixel |
| 2200 | 2 | deltaValue[5]: increase 17ppem by 1 pixel |
|  | DeviceTableFormat2 YDevice | Device Table definition |
| 000C | 12 | startSize |
| 0011 | 17 | endSize |
| 0002 | 2 | deltaFormat: LOCAL_4_BIT_DELTAS |
|  | 1 | deltaValue[0]: increase 12ppem by 1 pixel |
|  | 1 | deltaValue[1]: increase 13ppem by 1 pixel |
|  | 1 | deltaValue[2]: increase 14ppem by 1 pixel |
| 1111 | 1 | deltaValue[3]: increase 15ppem by 1 pixel |
|  | 2 | deltaValue[4]: increase 16ppem by 1 pixel |
| 2200 | 2 | deltaValue[5]: increase 17ppem by 1 pixel |

Example 18: MarkArray table and MarkRecord

Example 18 shows a MarkArray table with class and attachment point data for two accent marks, a grave and a cedilla. Two MarkRecords are defined, one for each covered mark glyph. The first MarkRecord assigns a mark class value of 0 to accents placed above base glyphs, such as the grave, and has an offset to a graveMarkAnchor table. The second MarkRecord assigns a mark class value of 1 for all accents positioned below base glyphs, such as the cedilla, and has an offset to a cedillaMarkAnchor table.

Example 18

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MarkArray MarkGlyphsArray | MarkArray table definition |
| 0002 | 2 | markCount |
|  | markRecords[0] | for first mark in MarkCoverage table: grave |
| 0000 | 0 | markClass – for marks placed above base glyphs |
| 000A | graveMarkAnchor | markAnchorOffset |
|  | markRecords[1] | for second mark in MarkCoverage table: cedilla |
| 0001 | 1 | markClass – for marks placed below base glyphs |
| 0010 | cedillaMarkAnchor | markAnchorOffset |

### GSUB—The glyph substitution table

#### GSUB table overview

The Glyph Substitution table (GSUB) contains information for substituting glyphs to render the scripts and language systems supported in a font. Many language systems require glyph substitutes. For example, in the Arabic script, the glyph shape that depicts a particular character varies according to its position in a word or text string (see Figure 6.28). In other language systems, glyph substitutes are aesthetic options for the user, such as the use of ligature glyphs in the English language (see Figure 6.29).

Figure 6.28 – Isolated, initial, medial, and final forms of the Arabic character HAH

Figure 6.29 – Two Latin glyphs and their associated ligature

OFF fonts use character encoding standards, such as the Unicode Standard, that assumes a distinction between characters and glyphs: text is encoded as sequences of characters, and the 'cmap' table provides a mapping from that character to a single default glyph. Multiple characters are not directly mapped to a single glyph, as needed for ligatures; and a single character is not mapped directly to multiple glyphs, as may be needed for some complex-script. The GSUB table provides a way to describe such substitutions, enabling applications to apply such substations during text layout and rendering to achieve desired results.

To access substitute glyphs, GSUB maps from the glyph index or indices defined in a 'cmap' subtable to the glyph index or indices of the substitute glyphs. For example, if a font has three alternative forms of an ampersand glyph, the 'cmap' table associates the ampersand’s character code with only one of these glyphs. In GSUB, the indices of the other ampersand glyphs are then referenced from this one default index.

The text-processing client uses the GSUB data to manage glyph substitution actions. GSUB identifies the glyphs that are input to and output from each glyph substitution action, specifies how and where the client uses glyph substitutes, and regulates the order of glyph substitution operations. Any number of substitutions can be defined for each script or language system represented in a font.

The GSUB table supports seven types of glyph substitutions that are widely used in international typography:

- A single substitution replaces a single glyph with another single glyph. This is used to render positional glyph variants in Arabic and vertical text in the Far East (see Figure 6.30).

Figure 6.30 – Alternative forms of parentheses used when positioning Kanji vertically

- A multiple substitution replaces a single glyph with more than one glyph. This is used to specify actions such as ligature decomposition (see Figure 6.31).

Figure 6.31 – Decomposing a Latin ligature glyph into its individual glyph components

- An alternate substitution identifies functionally equivalent but different looking forms of a glyph. These glyphs are often referred to as aesthetic alternatives. For example, a font might have five different glyphs for the ampersand symbol, but one would have a default glyph index in the cmap table. The client could use the default glyph or substitute any of the four alternatives (see Figure 6.32).

Figure 6.32 – Alternative ampersand glyphs in a font

- A ligature substitution replaces several glyph indices with a single glyph index, as when an Arabic ligature glyph replaces a string of separate glyphs (see Figure 6.33). When a string of glyphs can be replaced with a single ligature glyph, the first glyph is substituted with the ligature. The remaining glyphs in the string are deleted, this does not include those glyphs that are skipped as a result of lookup flags.

Figure 6.33 – Three Arabic glyphs and their associated ligature glyph

- Contextual substitution is a powerful extension of the above lookup types, describing glyph substitutions in context – that is, a substitution of one or more glyphs within a certain pattern of glyphs. Each substitution describes one or more input glyph sequences and one or more substitutions to be performed on that sequence. Contextual substitutions can be applied to specific glyph sequences, glyph classes, or sets of glyphs.

- Chaining contextual substitution extends the capabilities of contextual substitution. With this, one or more substitutions can be performed on one or more glyphs within a pattern of glyphs (input sequence), by chaining the input sequence to a 'backtrack' and/or 'lookahead' sequence. Each such substitution can be applied in three formats to handle glyphs, glyph classes or glyph sets in the input sequence. Each of these formats can describe one or more of the backtrack, input and lookahead sequences.

- Reverse Chaining contextual single substitution, allows one glyph to be substituted with another by chaining input glyph to a 'backtrack' and/or 'lookahead' sequence. The difference between this and other lookup types is that processing of input glyph sequence goes from end to start.

GSUB table and OFF font variations

OFF Font Variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF Font Variations, see subclause 7.1.

In a variable font, it may be desirable to have different glyph-substitution actions used for different regions within the font’s variation space. For example, for narrow or heavy instances in which counters become small, it may be desirable to make certain glyph substitutions to use alternate glyphs with certain strokes removed or outlines simplified to allow for larger counters. Such effects can be achieved using a FeatureVariations table within the GSUB table. The FeatureVariations table is described in subclause 6.2. See also the Required Variation Alternates ('rvrn') feature in the OFF Layout tag registry.

#### GSUB table organization and structure

Table organization

The GSUB table begins with a header that defines offsets to a ScriptList, a FeatureList, a LookupList, and an optional FeatureVariations table (see Figure 6.34):

- The ScriptList identifies all the scripts and language systems in the font that use glyph substitutes.

- The FeatureList defines all the glyph substitution features required to render these scripts and language systems.

- The LookupList contains all the lookup data needed to implement each glyph substitution feature.

- The FeatureVariations table can be used to substitute alternate sets of lookup tables to use for any given feature under specified conditions. This currently used only in variable fonts.

For a detailed discussion of ScriptLists, FeatureLists, LookupLists, and FeatureVariations tables see subclause 6.2.

Figure 6.34 – High-level organization of GSUB table

This organization helps text-processing clients to easily locate the features and lookups that apply to a particular script or language system. To access GSUB information, clients should use the following procedure:

- Locate the current script in the GSUB ScriptList table.

- If the language system is known, search the script for the correct LangSys table; otherwise, use the script's default language system (DefaultLangSys table).

- The LangSys table provides index numbers into the GSUB FeatureList table to access a required feature and a number of additional features.

- Inspect the featureTag of each Feature table, and select the feature tables to apply to an input glyph string.

- If a Feature Variations table is present, evaluate conditions in the Feature Variation table to determine if any of the initially-selected feature tables should be substituted by an alternate feature table.

- Each feature provides an array of index numbers into the GSUB LookupList table. Assemble all lookups from the set of chosen features, and apply the lookups in the order given in the LookupList table.

For a detailed description of the FeatureVariations table and how it is processed, see the "FeatureVariations Table" in subclause 6.2.

Lookup data is defined in Lookup tables, which are defined in subclause 6.2. A Lookup table contains one or more Lookup subtables that define the specific conditions, type, and results of a substitution action used to implement a feature. Specific Lookup subtable types are used for glyph substitution actions, and are defined in this chapter. All subtables within a Lookup table shall be of the same lookup type, as listed in the following table for the GSUB LookupType Enumeration:

GSUB LookupType Enumeration

| Value | Lookup Type | Description |
| --- | --- | --- |
| 1 | Single | Replace one glyph with one glyph |
| 2 | Multiple | Replace one glyph with more than one glyph |
| 3 | Alternate | Replace one glyph with one of many glyphs |
| 4 | Ligature | Replace multiple glyphs with one glyph |
| 5 | Context | Replace one or more glyphs in context |
| 6 | Chaining Context | Replace one or more glyphs in chained context |
| 7 | Extension Substitution | Extension mechanism for other substitutions (i.e. this excludes the Extension type substitution itself) |
| 8 | Reverse chaining context single | Applied in reverse order, replace single glyph in chaining context |
| 9+ | Reserved | For future use (must be set to zero) |

Each LookupType has one or more subtable formats. The "best" format depends on the type of substitution and the resulting storage efficiency. When glyph information is best presented in more than one format, a single lookup may define more than one subtable, as long as all the subtables are for the same LookupType. For example, within a given lookup, a glyph index array format may best represent one set of target glyphs, whereas a glyph index range format may be better for another set.

A series of substitution operations on the same glyph or string requires multiple lookups, one for each separate action. Each lookup has a different array index in the LookupList table and is applied in the LookupList order.

During text processing, a client applies a lookup to each glyph in the string before moving to the next lookup. A lookup is finished for a glyph after the client locates the target glyph or glyph context and performs a substitution, if specified. To move to the "next" glyph, the client will typically skip all the glyphs that participated in the lookup operation: glyphs that were substituted as well as any other glyphs that formed a context for the operation.

In the case of chained contextual lookups (LookupType 6), glyphs comprising backtrack and lookahead sequences may participate in more than one context.

The rest of this subclause describes the GSUB header and the subtables defined for each GSUB LookupType. Examples at the end of this subclause illustrate the GSUB header and six of the eight LookupTypes, including the three formats available for contextual substitutions (LookupType 5).

GSUB header

The GSUB table begins with a header that contains a version number for the table (Version) and offsets to  three tables: ScriptList, FeatureList, and LookupList. For descriptions of each of these tables, see clause 6.2, OFF Common Table Formats. Example 1 at the end of this subclause shows a GSUB Header table definition.

GSUB Header, Version 1.0

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GSUB table, = 1 |
| uint16 | minorVersion | Minor version of the GSUB table, = 0 |
| Offset16 | scriptList | Offset to ScriptList table, from beginning of GSUB table |
| Offset16 | featureList | Offset to FeatureList table, from beginning of GSUB table |
| Offset16 | lookupList | Offset to LookupList table, from beginning of GSUB table |

GSUB Header, Version 1.1

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the GSUB table, = 1 |
| uint16 | minorVersion | Minor version of the GSUB table, = 1 |
| Offset16 | scriptList | Offset to ScriptList table, from beginning of GSUB table |
| Offset16 | featureList | Offset to FeatureList table, from beginning of GSUB table |
| Offset16 | lookupList | Offset to LookupList table, from beginning of GSUB table |
| Offset32 | featureVariations | Offset to FeatureVariations table, from beginning of GSUB table (may be NULL) |

#### GSUB lookup type descriptions

##### LookupType 1: Single substitution subtable

Single substitution (SingleSubst) subtables tell a client to replace a single glyph with another glyph. The subtables can be either of two formats. Both formats require two distinct sets of glyph indices: one that defines input glyphs (specified in the Coverage table), and one that defines the output glyphs. Format 1 requires less space than Format 2, but it is less flexible.

Single substitution Format 1

Format 1 calculates the indices of the output glyphs, which are not explicitly defined in the subtable. To calculate an output glyph index, Format 1 adds a constant delta value to the input glyph index. For the substitutions to occur properly, the glyph indices in the input and output ranges shall be in the same order. This format does not use the Coverage Index that is returned from the Coverage table.

The SingleSubstFormat1 subtable begins with a format identifier (substFormat) of 1. An offset references a Coverage table that specifies the indices of the input glyphs. The deltaGlyphID is a constant value added to each input glyph index to calculate the index of the corresponding output glyph. Addition of deltaGlyphID is modulo 65536.

Example 2 at the end of this clause uses Format 1 to replace standard numerals with lining numerals.

SingleSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution subtable |
| int16 | deltaGlyphID | Add to original glyph ID to get substitute glyph ID |

Single substitution Format 2

Format 2 is more flexible than Format 1, but requires more space. It provides an array of output glyph indices (substituteGlyphIDs) explicitly matched to the input glyph indices specified in the Coverage table.

The SingleSubstFormat2 subtable specifies a format identifier (substFormat), an offset to a Coverage table that defines the input glyph indices, a count of output glyph indices in the substituteGlyphIDs array (glyphCount), as well as the list of the output glyph indices in the substitute array (substituteGlyphIDs).

The substituteGlyphIDs array shall contain the same number of glyph indices as the Coverage table. To locate the corresponding output glyph index in the substituteGlyphIDs array, this format uses the Coverage index returned from the Coverage table.

Example 3 at the end of this clause uses Format 2 to substitute vertically oriented glyphs for horizontally oriented glyphs.

SingleSubstFormat2 subtable: Specified output glyph indices

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of Substitution table |
| uint16 | glyphCount | Number of glyph IDs in the substituteGlyphIDs array |
| uint16 | substituteGlyphIDs [glyphCount] | Array of substitute glyph IDs – ordered by Coverage index |

##### LookupType 2: Multiple substitution subtable

A Multiple Substitution (MultipleSubst) subtable replaces a single glyph with more than one glyph, as when multiple glyphs replace a single ligature. The subtable has a single format: MultipleSubstFormat1.

Multiple Substitution Format1: Multiple output glyphs

The Multiple Substitution Format1 subtable specifies a format identifier (substFormat), an offset to a Coverage table that defines the input glyph indices, a count of offsets in the sequenceOffsets array (sequenceCount), and an array of offsets to Sequence tables that define the output glyph indices (sequenceOffsets). The Sequence table offsets are ordered by the Coverage index of the input glyphs.

For each input glyph listed in the Coverage table, a Sequence table defines the output glyphs. Each Sequence table contains a count of the glyphs in the output glyph sequence (glyphCount) and an array of output glyph indices (substituteGlyphIDs).

NOTE	The order of the output glyph indices depends on the writing direction of the text. For text written left to right, the left-most glyph will be first glyph in the sequence. Conversely, for text written right to left, the right-most glyph will be first.

The use of multiple substitution for deletion of an input glyph is prohibited. The glyphCount value should always be greater than 0.

Example 4 at the end of this clause shows how to replace a single ligature with three glyphs.

MultipleSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| uint16 | sequenceCount | Number of Sequence table offsets in the sequenceOffsets array |
| Offset16 | sequenceOffsets [sequenceCount] | Array of offsets to Sequence tables. Offsets are from beginning of substitution subtable, ordered by Coverage index |

Sequence table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Number of glyph IDs in the substituteGlyphIDs array. This shall always be greater than 0. |
| uint16 | substituteGlyphIDs [glyphCount] | String of glyph IDs to substitute |

##### LookupType 3: Alternate substitution subtable

An Alternate Substitution (AlternateSubst) subtable identifies any number of aesthetic alternatives from which a user can choose a glyph variant to replace the input glyph. For example, if a font contains four variants of the ampersand symbol, the cmap table will specify the index of one of the four glyphs as the default glyph index, and an AlternateSubst subtable will list the indices of the other three glyphs as alternatives. A text-processing client would then have the option of replacing the default glyph with any of the three alternatives.

The subtable has one format: AlternateSubstFormat1.

Alternate Substitution Format1: Alternative output glyphs

The Alternate Substitution Format1 subtable contains a format identifier (substFormat), an offset to a Coverage table containing the indices of glyphs with alternative forms (coverageOffset), a count of offsets to AlternateSet tables (alternateSetCount), and an array of offsets to AlternateSet tables (alternateSetOffets).

For each glyph, an AlternateSet subtable contains a count of the alternative glyphs (glyphCount) and an array of their glyph indices (alternateGlyphIDs). Because all the glyphs are functionally equivalent, they can be in any order in the array.

Example 5 at the end of this clause shows how to replace the default ampersand glyph with alternative glyphs.

AlternateSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| uint16 | alternateSetCount | Number of AlternateSet tables |
| Offset16 | alternateSetOffsets [alternateSetCount] | Array of offsets to AlternateSet tables. Offsets are from beginning of substitution table, ordered by Coverage index |

AlternateSet table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Number of glyph IDs in the alternateGlyphIDs array |
| uint16 | alternateGlyphIDs[glyphCount] | Array of alternate glyph IDs, in arbitrary order |

##### LookupType 4: Ligature substitution subtable

A Ligature Substitution (LigatureSubst) subtable identifies ligature substitutions where a single glyph replaces multiple glyphs. One LigatureSubst subtable can specify any number of ligature substitutions. The subtable has one format: LigatureSubstFormat1.

Ligature Substitution Format1: All ligature substitutions in a script

It contains a format identifier (substFormat), a Coverage table offset (coverageOffset), a count of the ligature sets defined in this table (ligatureSetCount), and an array of offsets to LigatureSet tables (ligatureSetOffsets). The Coverage table specifies only the index of the first glyph component of each ligature set.

LigatureSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of Substitution table |
| uint16 | ligatureSetCount | Number of LigatureSet tables |
| Offset16 | ligatureSetOffsets [ligatureSetCount] | Array of offsets to LigatureSet tables. Offsets are from beginning of substitution subtable, ordered by Coverage index |

A LigatureSet table, one for each covered glyph, specifies all the ligature strings that begin with the covered glyph. For example, if the Coverage table lists the glyph index for a lowercase "f", then a LigatureSet table will define the "ffl", "fl", "ffi", "fi", and "ff" ligatures. If the Coverage table also lists the glyph index for a lowercase "e", then a different LigatureSet table will define the "etc" ligature.

A LigatureSet table consists of a count of the ligatures that begin with the covered glyph (ligatureCount) and an array of offsets (ligatureSetOffsets) to Ligature tables, which define the glyphs in each ligature. The order in the Ligature offset array defines the preference for using the ligatures. For example, if the "ffl" ligature is preferable to the "ff" ligature, then the Ligature array would list the offset to the "ffl" Ligature table before the offset to the "ff" Ligature table.

LigatureSet table: All ligatures beginning with the same glyph

| Type | Name | Description |
| --- | --- | --- |
| uint16 | ligatureCount | Number of Ligature tables |
| Offset16 | ligatureSetOffsets [ligatureCount] | Array of offsets to Ligature tables. Offsets are from beginning of LigatureSet table, ordered by preference |

For each ligature in the set, a Ligature table specifies the glyph ID of the output ligature glyph (ligatureGlyph); a count of the total number of component glyphs in the ligature, including the first component (componentCount); and an array of glyph IDs for the components (componentGlyphIDs). The array starts with the second component glyph in the ligature (glyph sequnce index = 1, componentGlyphIDs array index = 0) because the first component glyph is specified in the Coverage table.

NOTE	The componentGlyphIDs array lists glyph IDs according to the writing direction – that is, the logical order – of the text. For text written right to left, the right-most glyph will be first. Conversely, for text written left to right, the left-most glyph will be first.

Example 6 at the end of this clause shows how to replace a string of glyphs with a single ligature.

Ligature table: Glyph components for one ligature

| Type | Name | Description |
| --- | --- | --- |
| uint16 | ligatureGlyph | Glyph ID of ligature to substitute |
| uint16 | componentCount | Number of components in the ligature |
| uint16 | componentGlyphIDs [componentCount - 1] | Array of component glyph IDs – start with the second component, ordered in writing direction |

##### Substitution Lookup Record

Substitution subtable types 1 to 4 allow for describing glyph substitution actions without providing a way to describe glyph-sequence contexts — that is, contexts in which glyphs must occur in order for substitutions to be applied. Subtable types 5, 6 and 8, on the other hand, allow for describing glyph substitutions that occur in particular contexts. Contextual substitution subtables of types 5 and 6 specify the substitution data in a Substitution Lookup Record (SubstLookupRecord), which identifies which glyphs within a context sequence are to be acted on, and which substitution action (given in a separate Lookup table) is to be applied.

Each record contains a glyphSequenceIndex, which indicates the position within a context glyph sequence where the substitution will occur. In addition, a lookupListIndex identifies the lookup to be applied at the glyph position specified by the glyphSequenceIndex.

The contextual substitution subtables defined in Examples 7, 8, and 9 at the end of this chapter show SubstLookupRecords.

SubstLookupRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphSequenceIndex | Index into current glyph sequence – first glyph = 0 |
| uint16 | lookupListIndex | Lookup to apply to that position – zero-based index. |

The glyphSequenceIndex in a SubstLookupRecord must take into consideration the order in which lookups are applied to the entire glyph sequence. Because multiple substitutions may occur in a given context, the glyphSequenceIndex refer to the glyph sequence after the text-processing client has applied any previous lookups. In other words, the glyphSequenceIndex identifies the location for the substitution at the time that the lookup is to be applied.

For example, consider an input glyph sequence of four glyphs. The first glyph does not have a substitute, but the middle two glyphs will be replaced with a ligature, and a single glyph will replace the fourth glyph. In this example, the ligature substitution and single substition are both to be expressed as contextual substitutions, with a context that begins at the first glyph:

- The first glyph is in position 0. No lookups will be applied at position 0, so no SubstLookupRecord is defined.

- The SubstLookupRecord defined for the ligature substitution specifies the glyphSequenceIndex as position 1, which is the position of the first glyph component in the ligature string. After the ligature replaces the glyphs in positions 1 and 2, however, the input glyph sequence consists of only three glyphs, not the original four.

- To replace the last glyph in the sequence, the lookup subtable specifies a context that consists of three glyphs, not four; and the SubstLookupRecord defines the glyphSequenceIndex as position 2, not position 3. This position and that context reflect the effect of the ligature substitution applied before this single substitution.

NOTE	This example assumes that the LookupList specifies the ligature substitution lookup before the single substitution lookup.

##### LookupType 5: Contextual substitution subtable

A Contextual Substitution (ContextSubst) subtable defines a powerful type of glyph substitution lookup: it describes glyph substitutions in context that replace one or more glyphs within a certain pattern of glyphs.

ContextSubst subtables can be any of three formats that define a context in terms of a specific sequence of glyphs, glyph classes, or glyph sets. Each format can describe one or more input glyph sequences and one or more substitutions for each sequence. All three formats specify substitution data in a SubstLookupRecord,described above.

Context substitution Format 1: Simple Glyph Contexts

Format 1 defines the context for a glyph substitution as a particular sequence of glyphs. For example, a context could be <xyz>, <holiday>, <!?*#@>, or any other glyph sequence.

Within a context sequence, Format 1 identifies particular glyph positions (not glyph indices) as the targets for specific substitutions. When a text-processing client locates a context in a string of glyphs, it finds the lookup data for a targeted position and makes a substitution by applying the lookup data at that location.

For example, if a client is to replace the glyph string <abc> with its reverse glyph string <cba>, the input context is defined as the glyph sequence, <abc>, and the lookups defined for the context are (1) "a" to "c" and (2) "c" to "a". When a client encounters the context <abc>, the lookups are performed in the order stored. First, "c" is substituted for "a" resulting in <cbc>. Second, "a" is substituted for the "c" that has not yet been touched, resulting in <cba>.

To specify a context, a Coverage table lists the first glyph in the sequence, and a SubRule table identifies the remaining glyphs. To describe the <abc> context used in the previous example, the Coverage table lists the glyph index of the first component of the sequence – the "a" glyph. A SubRule table defines indices for the "b" and "c" glyphs.

A single ContextSubstFormat1 subtable may define more than one context glyph sequence. If different context sequences begin with the same glyph, then the Coverage table should list the glyph only once because all glyphs in the table must be unique. For example, if three contexts each start with an "s" and two start with a "t", then the Coverage table will list one "s" and one "t".

For each context, a SubRule table lists all the glyphs that follow the first glyph. The table also contains an array of SubstLookupRecords that specify the substitution lookup data for each glyph position (including the first glyph position) in the context.

All of the SubRule tables defining contexts that begin with the same first glyph are grouped together and defined in a SubRuleSet table. For example, the SubRule tables that define the three contexts that begin with an "s" are grouped in one SubRuleSet table, and the SubRule tables that define the two contexts that begin with a "t" are grouped in a second SubRuleSet table. Each glyph listed in the Coverage table must have a SubRuleSet table defining all the SubRule tables that apply to a covered glyph.

To locate a context glyph sequence, the text-processing client searches the Coverage table each time it encounters a new text glyph. If the glyph is covered, the client reads the corresponding SubRuleSet table and examines each SubRule table in the set to determine whether the rest of the context matches the subsequent glyphs in the text. If the context and text string match, the client finds the target glyph positions, applies the lookups for those positions, and completes the substitutions.

A ContextSubstFormat1 subtable contains a format identifier (substFormat), an offset to a Coverage table (coverageOffset), a count of defined SubRuleSets (subRuleSetCount), and an array of offsets to the SubRuleSet tables (subRuleSetOffsets). As mentioned, one SubRuleSet table must be defined for each glyph listed in the Coverage table.

In the SubRuleSet array, the SubRuleSet table offsets are ordered in the Coverage index order. The first SubRuleSet in the array applies to the first glyph ID listed in the Coverage table, the second SubRuleSet in the array applies to the second glyph ID listed in the Coverage table, and so on.

Example 7 at the end of this subclause shows how to use the ContextSubstFormat1 subtable to replace a sequence of three glyphs with a sequence preferred for the French language system.

ContextSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| uint16 | subRuleSetCount | Number of SubRuleSet tables – must equal glyphCount in Coverage table |
| Offset16 | subRuleSetOffsets [subRuleSetCount] | Array of offsets to SubRuleSet tables. Offsets are from beginning of Substitution table, ordered by Coverage index |

A SubRuleSet table consists of an array of offsets to SubRule tables (subRuleOffsets), ordered by preference, and a count of the SubRule tables defined in the set (subRuleCount). The order in the SubRule array can be critical. Consider two contexts, <abc> and <abcd>. If <abc> is first in the SubRule array, all instances of <abc> in the text – including all instances of <abcd> – will be changed. If <abcd> comes first in the array, however, only <abcd> sequences will be changed, without affecting any instances of <abc>.

SubRuleSet table: All contexts beginning with the same glyph

| Type | Name | Description |
| --- | --- | --- |
| uint16 | subRuleCount | Number of SubRule tables |
| Offset16 | subRuleOffsets [subRuleCount] | Array of offsets to SubRule tables. Offsets are from beginning of SubRuleSet table, ordered by preference |

A SubRule table consists of a count of the glyphs to be matched in the input context sequence (glyphCount), including the first glyph in the sequence, and an array of glyph indices that describe the context (inputSequence). The Coverage table specifies the index of the first glyph in the context, and the Input array begins with the second glyph in the context sequence (glyph sequence index = 1, inputSequence array index = 0).

NOTE	The Input array lists the indices in the order the corresponding glyphs appear in the text. For text written from right to left, the right-most glyph will be first; conversely, for text written from left to right, the left-most glyph will be first.

A SubRule table also contains a count of the substitutions to be performed on the input glyph sequence (substitutionCount) and an array of SubstLookupRecords (substLookupRecords). Each record specifies a position in the input glyph sequence and a LookupList index to the substitution lookup that is applied at that position. The array should list records in design order, or the order the lookups should be applied to the entire glyph sequence.

SubRule table: One simple context definition

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Total number of glyphs in input glyph sequence – includes the first glyph. |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| uint16 | inputSequence [glyphCount - 1] | Array of input glyph IDs – start with second glyph |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of SubstLookupRecords, in design order |

Context substitution Format 2: Class-based Glyph Contexts

Format 2, a more flexible format than Format 1, describes class-based context substitution. For this format, a specific integer, called a class value, must be assigned to each glyph component in all context glyph sequences. Contexts are then defined as sequences of glyph class values. More than one context may be defined at a time.

For example, suppose that a swash capital glyph should replace each uppercase letter glyph that is preceded by a space glyph and followed by a lowercase letter glyph (a glyph sequence of space - uppercase - lowercase). The set of uppercase glyphs would constitute one glyph class (Class 1), the set of lowercase glyphs would constitute a second class (Class 2), and the space glyph would constitute a third class (Class 3). The input context might be specified with a context rule (called a SubClassRule) that describes "the set of glyph strings that form a sequence of three glyph classes, one glyph from Class 3, followed by one glyph from Class 1, followed by one glyph from Class 2".

Each ContextSubstFormat2 subtable contains an offset to a class definition table (classDefOffset), which defines the glyph class values of all input contexts. Generally, a unique ClassDef table will be declared in each instance of the ContextSubstFormat2 table that is included in a font, even though several Format 2 tables could share ClassDef tables. Class assignments are fixed (the same for each position in the context), and classes are exclusive (a glyph cannot be in more than one class at a time). The output glyphs that replace the glyphs in the context sequences do not need class values because they are specified elsewhere by glyph ID.

The ContextSubstFormat2 subtable also contains a format identifier (substFormat) and defines an offset to a Coverage table (coverageOffset). For this format, the Coverage table lists indices for the complete set of unique glyphs (not glyph classes) that may appear as the first glyph of any class-based context. In other words, the Coverage table contains the list of glyph indices for all the glyphs in all classes that may be first in any of the context class sequences. For example, if the contexts begin with a Class 1 or Class 2 glyph, then the Coverage table will list the indices of all Class 1 and Class 2 glyphs.

A ContextSubstFormat2 subtable also defines an array of offsets to the SubClassSet tables (subClassSetOffsets) and a count of the SubClassSet tables (subClassSetCount). The array contains one offset for each class (including Class 0) in the ClassDef table. In the array, the class value defines an offset's index position, and the SubClassSet offsets are ordered by ascending class value (from 0 to subClassSetCount - 1).

For example, the first SubClassSet listed in the array contains all contexts beginning with Class 0 glyphs, the second SubClassSet contains all contexts beginning with Class 1 glyphs, and so on. If no contexts begin with a particular class (that is, if a SubClassSet contains no SubClassRule tables), then the offset to that particular SubClassSet in the SubClassSet array will be set to NULL.

Example 8 at the end of this subclause uses Format 2 to substitute Arabic mark glyphs for base glyphs of different heights.

ContextSubstFormat2 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| Offset16 | classDefOffset | Offset to glyph ClassDef table, from beginning of substitution table |
| uint16 | subClassSetCount | Number of SubClassSet tables |
| Offset16 | subClassSetOffsets [subClassSetCount] | Array of offsets to SubClassSet tables. Offsets are from beginning of substitution subtable, ordered by class (may be NULL) |

Each context is defined in a SubClassRule table, and all SubClassRules that specify contexts beginning with the same class value are grouped in a SubClassSet table. Consequently, the SubClassSet containing a context identifies a context's first class component.

Each SubClassSet table consists of a count of the SubClassRule tables defined in the SubClassSet (subClassRuleCount) and an array of offsets to SubClassRule tables (subClassRuleOffsets). The SubClassRule tables are ordered by preference in the SubClassRule array of the SubClassSet.

SubClassSet subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | subClassRuleCount | Number of SubClassRule tables |
| Offset16 | subClassRuleOffsets [subClassRuleCount] | Array of offsets to SubClassRule tables. Offsets are from beginning of SubClassSet, ordered by preference |

For each context, a SubClassRule table contains a count of the glyph classes in the context sequence (glyphCount), including the first class. A Class array lists the classes, beginning with the class for the second context position (glyph sequence index = 1, inputSequence array index = 0).

NOTE	Text order depends on the writing direction of the text. For text written from right to left, the right-most class will be first. Conversely, for text written from left to right, the left-most class will be first.

The values specified in the Class array are the values defined in the ClassDef table. For example, a context consisting of the sequence "Class 2, Class 7, Class 5, Class 0" will produce a Class array of 7,5,0. The first class in the sequence, Class 2, is identified in the ContextSubstFormat2 table by the SubClassSet array index of the corresponding SubClassSet.

A SubClassRule also contains a count of the substitutions to be performed on the context (substitutionCount) and an array of SubstLookupRecords (substLookupRecords) that supply the substitution data. For each position in the context that requires a substitution, a SubstLookupRecord specifies a LookupList index and a position in the input glyph sequence where the lookup is applied. The substLookupRecords array lists SubstLookupRecords in design order – that is, the order in which lookups should be applied to the entire glyph sequence.

SubClassRule table: Context definition for one class

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Total number of classes specified for the context in the rule – includes the first class |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| uint16 | inputSequence [glyphCount - 1] | Array of classes to be matched to the input glyph sequence, beginning with the second glyph position |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of Substitution lookups, in design order |

Context substitution Format 3: Coverage-based Glyph Contexts

Format 3, coverage-based context substitution, defines a context rule as a sequence of coverage tables. Each position in the sequence may define a different Coverage table for the set of glyphs that matches the context pattern. With Format 3, the glyph sets defined in the different Coverage tables may intersect, unlike Format 2 which specifies fixed class assignments (identical for each position in the context sequence) and exclusive classes (a glyph cannot be in more than one class at a time).

For example, consider an input context that contains a lowercase glyph (position 0), followed by an uppercase glyph (position 1), either a lowercase or numeral glyph (position 2), and then either a lowercase or uppercase vowel (position 3). This context requires four Coverage tables, one for each position:

- In position 0, the Coverage table lists the set of lowercase glyphs.

- In position 1, the Coverage table lists the set of uppercase glyphs.

- In position 2, the Coverage table lists the set of lowercase and numeral glyphs, a superset of the glyphs defined in the Coverage table for position 0.

- In position 3, the Coverage table lists the set of lowercase and uppercase vowels, a subset of the glyphs defined in the Coverage tables for both positions 0 and 1.

Unlike Formats 1 and 2, this format defines only one context rule at a time. It consists of a format identifier (substFormat), a count of the glyphs in the sequence to be matched (glyphCount), and an array of Coverage offsets that describe the input context sequence (coverageOffsets).

NOTE	The order of the Coverage tables listed in the Coverage array must follow the writing direction. For text written from right to left, then the right-most glyph will be first. Conversely, for text written from left to right, the left-most glyph will be first.

The subtable also contains a count of the substitutions to be performed on the input Coverage sequence (substitutionCount) and an array of SubstLookupRecords (substLookupRecords) in design order – that is, the order in which lookups should be applied to the entire glyph sequence.

Example 9 at the end of this subclause uses ContextSubstFormat3 to substitute a swash glyph for two out of three glyphs in a sequence.

ContextSubstFormat3 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 3 |
| uint16 | glyphCount | Number of glyphs in the input glyph sequence |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| Offset16 | coverageOffsets[glyphCount] | Array of offsets to Coverage table. Offsets are -from beginning of substitution subtable, in glyph sequence order |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of SubstLookupRecords, in design order |

##### LookupType 6: Chaining contextual substitution subtable

A Chaining Contextual Substitution subtable (ChainContextSubst) describes glyph substitutions in context with an ability to look back and/or look ahead in the sequence of glyphs. The design of the Chaining Contextual Substitution subtable is parallel to that of the Contextual Substitution subtable, including the availability of three formats for handling sequences of glyphs, glyph classes, or glyph sets. Each format can describe one or more backtrack, input, and lookahead sequences and one or more substitutions for each sequence.

Chaining context substitution Format 1: Simple chaining context glyph substitution

Format 1 defines the context for a glyph substitution as a particular sequence of glyphs. For example, a context could be <xyz>, <holiday>, <!?*#@>, or any other glyph sequence.

Within a context sequence, Format 1 identifies particular glyph positions (not glyph indices) as the targets for specific substitutions. When a text-processing client locates a context in a string of glyphs, it finds the lookup data for a targeted position and makes a substitution by applying the lookup data at that location.

To specify the context, the coverage table lists the first glyph in the input sequence, and the ChainSubRule subtable defines the rest. Once a covered glyph is found at position i, the client reads the corresponding ChainSubRuleSet table and examines each table to determine if it matches the surrounding glyphs in the glyph string. In the simplest of cases, there is a match if the string <backtrack sequence>+<input sequence>+<lookahead sequence> matches with the glyphs at position i - backtrackGlyphCount in the text. Note that LookupFlags affect backtrack/lookahead sequences.

To clarify the ordering of glyph arrays for input, backtrack and lookahead sequences, the following illustration is provided. Input sequence match begins at i where the input sequence match begins. The backtrack sequence is ordered beginning at i - 1 and increases in offset value as one moves away from i. The lookahead sequence begins after the input sequence and increases in logical order.

| Logical order - | a | b | c | d | e | f | g | h | i | j |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | i |  |  |  |  |  |
| Input sequence - |  |  |  |  | 0 | 1 |  |  |  |  |
| Backtrack sequence - | 3 | 2 | 1 | 0 |  |  |  |  |  |  |
| Lookahead sequence - |  |  |  |  |  |  | 0 | 1 | 2 | 3 |

If there is a match, then the client finds the target glyph positions for substitutions and completes the substitutions. Please note that (just like in the ContextSubstFormat1 subtable) these lookups are required to operate within the range of text from the covered glyph to the end of the input sequence. No substitutions can be defined for the backtracking sequence or the lookahead sequence.

Once the substitutions are complete, the client should move to the glyph position immediately following the matched input sequence and resume the lookup process from there.

A single ChainContextSubstFormat1 subtable may define more than one context glyph sequence. If different context sequences begin with the same glyph, then the Coverage table should list the glyph only once because all glyphs in the table must be unique. For example, if three contexts each start with an "s" and two start with a "t", then the Coverage table will list one "s" and one "t".

All of the ChainSubRule tables defining contexts that begin with the same first glyph are grouped together and defined in a ChainSubRuleSet table. For example, the ChainSubRule tables that define the three contexts that begin with an "s" are grouped in one ChainSubRuleSet table, and the ChainSubRule tables that define the two contexts that begin with a "t" are grouped in a second ChainSubRuleSet table. Each glyph listed in the Coverage table must have a ChainSubRuleSet table defining all the ChainSubRule tables that apply to a covered glyph.

A ChainContextSubstFormat1 subtable contains a format identifier (substFormat), an offset to a Coverage table (coverageOffset), a count of defined ChainSubRuleSets (chainSubRuleSetCount), and an array of offsets to the ChainSubRuleSet tables (chainSubRuleSetOffsets). As mentioned, one ChainSubRuleSet table must be defined for each glyph listed in the Coverage table.

In the ChainSubRuleSet array, the ChainSubRuleSet table offsets are ordered in the Coverage index order. The first ChainSubRuleSet in the array applies to the first glyph ID listed in the Coverage table, the second ChainSubRuleSet in the array applies to the second glyph ID listed in the Coverage table, and so on.

ChainContextSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| uint16 | chainSubRuleSetCount | Number of ChainSubRuleSet tables – must equal glyphCount in Coverage table |
| Offset16 | chainSubRuleSetOffsets [chainSubRuleSetCount] | Array of offsets to ChainSubRuleSet tables. Offsets are from beginning of substitution table, ordered by Coverage index |

A ChainSubRuleSet table consists of an array of offsets to ChainSubRule tables (chainSubRuleOffsets), ordered by preference, and a count of the ChainSubRule tables defined in the set (chainSubRuleCount).

The order in the ChainSubRule array can be critical. Consider two contexts, <abc> and <abcd>. If <abc> is first in the ChainSubRule array, all instances of <abc> in the text – including all instances of <abcd> – will be changed. If <abcd> comes first in the array, however, only <abcd> sequences will be changed, without affecting any instances of <abc>.

ChainSubRuleSet table: All contexts beginning with the same glyph

| Type | Name | Description |
| --- | --- | --- |
| uint16 | chainSubRuleCount | Number of ChainSubRule tables |
| Offset16 | chainSubRuleOffsets [chainSubRuleCount] | Array of offsets to ChainSubRule tables. Offsets are from beginning of ChainSubRuleSet table, ordered by preference |

A ChainSubRule table consists of a count of the glyphs to be matched in the backtrack, input, and lookahead context sequences, including the first glyph in each sequence, and an array of glyph indices that describe each portion of the contexts. The Coverage table specifies the index of the first glyph in each context, and each array begins with the second glyph in the context sequence (glyph sequence index = 1, inputSequence array index = 0).

NOTE	All arrays list the indices in the order the corresponding glyphs appear in the text. For text written from right to left, the right-most glyph will be first; conversely, for text written from left to right, the left-most glyph will be first.

A ChainSubRule table also contains a count of the substitutions to be performed on the input glyph sequence (substitutionCount) and an array of SubstitutionLookupRecords (substLookupRecord). Each record specifies a position in the input glyph sequence and a LookupList index to the substitution lookup that is applied at that position. The array should list records in design order, or the order the lookups should be applied to the entire glyph sequence.

ChainSubRule subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | backtrackGlyphCount | Total number of glyphs in the backtrack sequence (number of glyphs to be matched before the first glyph of the input sequence) |
| uint16 | backtrackSequence [backtrackGlyphCount] | Array of backtracking glyph IDs – to be matched before the input sequence. |
| uint16 | inputGlyphCount | Total number of glyphs in the input sequence – includes the first glyph. |
| uint16 | inputSequence [inputGlyphCount - 1] | Array of input glyph IDs – start with second glyph. |
| uint16 | lookaheadGlyphCount | Total number of glyphs in the lookahead sequence (number of glyphs to be matched after the input sequence) |
| uint16 | lookAheadSequence [lookAheadGlyphCount] | Array of lookahead glyph IDs – to be matched after the input sequence. |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of SubstLookupRecords, in design order |

Chaining context substitution Format 2: Class-based glyph contexts

Format 2 describes class-based chaining context substitution. For this format, a specific integer, called a class value, must be assigned to each glyph component in all context glyph sequences. Contexts are then defined as sequences of glyph class values. More than one context may be defined at a time.

To chain contexts, three classes are used in the glyph ClassDef table: backtrack ClassDef, input ClassDef, and lookahead ClassDef.

The ChainContextSubstFormat2 subtable also contains a format identifier (substFormat) and defines an offset to a Coverage table (coverageOffset). For this format, the Coverage table lists indices for the complete set of unique glyphs (not glyph classes) that may appear as the first glyph of any class-based context. In other words, the Coverage table contains the list of glyph indices for all the glyphs in all classes that may be first in any of the context class sequences. For example, if the contexts begin with a Class 1 or Class 2 glyph, then the Coverage table will list the indices of all Class 1 and Class 2 glyphs.

A ChainContextSubstFormat2 subtable also defines an array of offsets to the ChainSubClassSet tables (chainSubClassSetOffsets) and a count of the ChainSubClassSet tables (chainSubClassSetCount). The array contains one offset for each class (including Class 0) in the ClassDef table. In the array, the class value defines an offset's index position, and the ChainSubClassSet offsets are ordered by ascending class value (from 0 to chainSubClassSetCount - 1).

If no contexts begin with a particular class (that is, if a ChainSubClassSet contains no ChainSubClassRule tables), then the offset to that particular ChainSubClassSet in the ChainSubClassSet array will be set to NULL.

ChainContextSubstFormat2 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 2 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| Offset16 | backtrackClassDef | Offset to glyph ClassDef table containing backtrack sequence data, from beginning of substitution table |
| Offset16 | inputClassDef | Offset to glyph ClassDef table containing input sequence data, from beginning of substitution table |
| Offset16 | lookaheadClassDef | Offset to glyph ClassDef table containing lookahead sequence data, from beginning of substitution table |
| uint16 | chainSubClassSetCount | Number of ChainSubClassSet tables |
| Offset16 | chainSubClassSetOffsets [chainSubClassSetCount] | Array of offsets to ChainSubClassSet tables. Offsets are from beginning of substitution table, ordered by input class (may be NULL) |

Each context is defined in a ChainSubClassRule table, and all ChainSubClassRules that specify contexts beginning with the same class value are grouped in a ChainSubClassSet table. Consequently, the ChainSubClassSet containing a context identifies a context's first class component.

Each ChainSubClassSet table consists of a count of the ChainSubClassRule tables defined in the ChainSubClassSet (chainSubClassRuleCount) and an array of offsets to ChainSubClassRule tables (chainSubClassRuleOffsets). The ChainSubClassRule tables are ordered by preference in the ChainSubClassRule array of the ChainSubClassSet.

ChainSubClassSet subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | chainSubClassRuleCount | Number of ChainSubClassRule tables |
| Offset16 | chainSubClassRuleOffsets [chainSubClassRuleCount] | Array of offsets to ChainSubClassRule tables. Offsets are from beginning of ChainSubClassSet, ordered by preference |

For each context (backtrack, input, lookahead), a ChainSubClassRule table contains a count of the glyph classes for each portion of the context sequence (backtrackGlyphCount, inputGlyphCount, lookaheadGlyphCount), including the first class. A Class array lists the classes, beginning with the class for the second context position (input glyph sequence index = 1, inputSequence array index = 0), that follow the first class in the context.

NOTE	Text order depends on the writing direction of the text. For text written from right to left, the right-most class will be first. Conversely, for text written from left to right, the left-most class will be first.

The values specified in the Class array are the values defined in the ClassDef table. The first class in the sequence, Class 2, is identified in the ChainContextSubstFormat2 table by the ChainSubClassSet array index of the corresponding ChainSubClassSet.

A ChainSubClassRule also contains a count of the substitutions to be performed on the context (substitutionCount) and an array of SubstLookupRecords (substLookupRecords) that supply the substitution data. For each position in the context that requires a substitution, a SubstLookupRecord specifies a LookupList index and a position in the input glyph sequence where the lookup is applied. The SubstLookupRecord array lists SubstLookupRecords in design order – that is, the order in which lookups should be applied to the entire glyph sequence.

ChainSubClassRule table: Chaining context definition for one class

| Type | Name | Description |
| --- | --- | --- |
| uint16 | backtrackGlyphCount | Total number of glyphs in the backtrack sequence (number of glyphs to be matched before the first glyph of the input sequence). |
| uint16 | backtrackSequence [BacktrackGlyphCount] | Array of backtracking classes – to be matched before the input sequence. |
| uint16 | inputGlyphCount | Total number of classes in the input sequence – includes the first class. |
| uint16 | inputSequence iInputGlyphCount - 1] | Array of classes to be matched with the input glyph sequence – beginning with second glyph position. |
| uint16 | lookaheadGlyphCount | Total number of glyphs in the lookahead sequence (number of glyphs to be matched after the input sequence). |
| uint16 | lookAheadSequence [lookAheadGlyphCount] | Array of lookahead classes – to be matched with glyph sequence after the input sequence. |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of SubstLookupRecords, in design order. |

Chaining context substitution Format 3: Coverage-based glyph contexts

Format 3 defines a chaining context rule as a sequence of Coverage tables. Each position in the sequence may define a different Coverage table for the set of glyphs that matches the context pattern. With Format 3, the glyph sets defined in the different Coverage tables may intersect, unlike Format 2 which specifies fixed class assignments (identical for each position in the backtrack, input, or lookahead sequence) and exclusive classes (a glyph cannot be in more than one class at a time).

NOTE	The order of the Coverage tables listed in the Coverage array must follow the writing direction. For text written from right to left, then the right-most glyph will be first. Conversely, for text written from left to right, the left-most glyph will be first.

The subtable also contains a count of the substitutions to be performed on the input Coverage sequence (substitutionCount) and an array of SubstLookupRecords (substLookupRecords) in design order – that is, the order in which lookups should be applied to the entire glyph sequence.

ChainContextSubstFormat3 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier: format = 3 |
| uint16 | backtrackGlyphCount | Number of glyphs in the backtracking sequence |
| Offset16 | backtrackCoverageOffsets [backtrackGlyphCount] | Array of offsets to coverage tables in backtracking sequence. Offsets are from the beginning of substitution subtable, in glyph sequence order. |
| uint16 | inputGlyphCount | Number of glyphs in input sequence |
| Offset16 | inputCoverageOffsets [inputGlyphCount] | Array of offsets to coverage tables in input sequence. Offsets are from the beginning of substitution subtable, in glyph sequence order |
| uint16 | lookaheadGlyphCount | Number of glyphs in lookahead sequence |
| Offset16 | lookaheadCoverageOffsets [lookaheadGlyphCount] | Array of offsets to coverage tables in lookahead sequence. Offsets are from the beginning of substitution subtable, in glyph sequence order. |
| uint16 | substitutionCount | Number of SubstLookupRecords |
| SubstLookupRecord | substLookupRecords [substitutionCount] | Array of SubstLookupRecords, in design order |

##### LookupType 7: Extension substitution

This lookup provides a mechanism whereby any other lookup type's subtables are stored at a 32-bit offset location in the 'GSUB' table. This is needed if the total size of the subtables exceeds the 16-bit limits of the various other offsets in the 'GSUB' table. In this document, the subtable stored at the 32-bit offset location is termed the "extension" subtable.

This subtable type uses one format: ExtensionSubstFormat1.

ExtensionSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier. Set to 1. |
| uint16 | extensionLookupType | Lookup type of subtable referenced by extensionOffset (that is, the extension subtable). |
| Offset32 | extensionOffset | Offset to the extension subtable, of lookup type extensionLookupType, relative to the start of the ExtensionSubstFormat1 subtable. |

The extensionLookupType field must be set to any lookup type other than 7. All subtables in a LookupType 7 lookup shall have the same extensionLookupType. All offsets within the extension subtables are set in the usual way, i.e. relative to the extension subtables themselves.

When an OFF layout engine encounters a LookupType 7 Lookup table, it shall:

- Proceed as though the Lookup table's lookupType field were set to the extensionLookupType of the subtables.

- Proceed as though each extension subtable referenced by extensionOffset replaced the LookupType 7 subtable that referenced it.

##### LookupType 8: Reverse chaining contextual single substitution subtable

Reverse Chaining Contextual Single Substitution subtable (ReverseChainSingleSubst) describes single-glyph substitutions in context with an ability to look back and/or look ahead in the sequence of glyphs. The major difference between this and other lookup types is that processing of input glyph sequence goes from end to start.

Compared to Chaining Contextual Substitution (lookup subtable type 6), this format is restricted to only coverage-based subtable format, input sequence can contain only a single glyph, and only single substitutions are allowed on this glyph. This constraint is integrated into the subtable format.

This lookup type is designed specifically for the Arabic script writing styles, like nastaliq, where the shape of the glyph is determined by the following glyph, beginning at the last glyph of the "joor", or set of connected glyphs.

Reverse chaining contextual single substitution Format 1: Coverage-based contexts

Format 1 defines a chaining context rule as a sequence of Coverage tables. Each position in the sequence may define a different Coverage table for the set of glyphs that matches the context pattern. With Format 1, the glyph sets defined in the different Coverage tables may intersect.

NOTE	Despite reverse order processing, the order of the Coverage tables listed in the Coverage array must be in logical order (follow the writing direction). The backtrack sequence is as illustrated in the LookupType 6: Chaining Contextual Substitution subtable. The input sequence is one glyph located at i in the logical string. The backtrack begins at i - 1 and increases in offset value as one moves toward the logical beginning of the string. The lookahead sequence begins at i + 1 and increases in offset value as one moves toward the logical end of the string. In processing a reverse chaining substitution, i began at the logical end of the string and moves to the beginning.

The subtable contains a Coverage table for the input glyph and Coverage table arrays for backtrack and lookahead sequences. It also contains an array of substitute glyphs indices (substituteGlyphIDs), which are substitutions for glyphs in the Coverage table, and a count of glyphs in the substituteGlyphIDs array. The substituteGlyphIDs array shall contain the same number of glyph indices as the Coverage table. To locate the corresponding output glyph index in the substituteGlyphIDs array, this format uses the Coverage index returned from the Coverage table.

Example 10 at the end of this subclause uses ReverseChainSingleSubstFormat1 to substitute Arabic glyphs with a correct stroke thickness on the left (exit) to match the stroke thickness on the right (entry) of the following glyph (in logical order).

ReverseChainSingleSubstFormat1 subtable:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | substFormat | Format identifier; format = 1 |
| Offset16 | coverageOffset | Offset to Coverage table, from beginning of substitution table |
| uint16 | backtrackGlyphCount | Number of glyphs in the backtracking sequence. |
| Offset16 | backtrackCoverageOffsets [backtrackGlyphCount] | Array of offsets to coverage tables in backtracking sequence, in glyph sequence order. |
| uint16 | lookaheadGlyphCount | Number of glyphs in lookahead sequence. |
| Offset16 | lookaheadCoverageOffsets [lookaheadGlyphCount] | Array of offsets to coverage tables in lookahead sequence, in glyph sequence order. |
| uint16 | glyphCount | Number of glyph IDs in the substituteGlyphIDs array. |
| uint16 | substituteGlyphIDs[glyphCount] | Array of substitute glyph IDs – ordered by Coverage index. |

#### GSUB subtable examples

The rest of this clause describes and illustrates examples of all the GSUB subtables, including each of the three formats available for contextual substitutions. All the examples reflect unique parameters described below, but the samples provide a useful reference for building subtables specific to other situations.

All the examples have three columns showing hex data, source, and comments.

Example 1: GSUB header table

Example 1 shows a typical GSUB Header table definition.

Example 1

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | GSUBHeader TheGSUBHeader | GSUBHeader table definition |
| 00010000 | 0x00010000 | major/minor version |
| 000A | TheScriptList | Offset to ScriptList table |
| 001E | TheFeatureList | Offset to FeatureList table |
| 002C | TheLookupList | Offset to LookupList table |

Example 2: SingleSubstFormat1 subtable

Example 2 illustrates the SingleSubstFormat1 subtable, which uses ranges to replace single input glyphs with their corresponding output glyphs. The indices of the output glyphs are calculated by adding a constant delta value to the indices of the input glyphs. In this example, the Coverage table has a format identifier of 1 to indicate the range format, which is used because the input glyph indices are in consecutive order in the font. The Coverage table specifies one range that contains a startGlyphID for the "0" (zero) glyph and an endGlyphID for the "9" glyph.

Example 2

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | SingleSubstFormat1 LiningNumeralSubtable | SingleSubst subtable definition |
| 0001 | 1 | substFormat: calculated output glyph indices |
| 0006 | LiningNumeralCoverage | Offset to Coverage table for input glyphs |
| 00C0 | 192 | deltaGlyphID = 192: add to each input glyph index to produce output glyph index |
|  | CoverageFormat2 LiningNumeralCoverage | Coverage table definition |
| 0002 | 2 | coverageFormat: ranges |
|  | 1 | rangeCount |
|  | rangeRecord[0] |  |
| 004E | 78 | Start glyphID for numeral zero glyph |
| 0058 | 87 | End glyphID for numeral nine glyph |
| 0000 | 0 | startCoverageIndex: first CoverageIndex = 0 |

Example 3: SingleSubstFormat2 subtable

Example 3 uses the SingleSubstFormat2 subtable for lists to substitute punctuation glyphs in Japanese text that is written vertically. Horizontally oriented parentheses and square brackets (the input glyphs) are replaced with vertically oriented parentheses and square brackets (the output glyphs).

The Coverage table, Format 1, identifies each input glyph index. The number of input glyph indices listed in the Coverage table matches the number of output glyph indices listed in the subtable. For correct substitution, the order of the glyph indices in the Coverage table (input glyphs) must match the order in the Substitute array (output glyphs).

Example 3

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | SingleSubstFormat2 VerticalPunctuationSubtable | SingleSubst subtable definition |
| 0002 | 2 | substFormat: lists |
| 000E | VerticalPunctuationCoverage | Offset to Coverage table |
| 0004 | 4 | glyphCount – equals glyphCount in Coverage table |
| 0131 | VerticalOpenBracketGlyph | substituteGlyphIDs[0], ordered by Coverage index |
| 0135 | VerticalClosedBracketGlyph | substituteGlyphIDs [1] |
| 013E | VerticalOpenParenthesisGlyph | substituteGlyphIDs [2] |
| 0143 | VerticalClosedParenthesisGlyph | substituteGlyphIDs [3] |
|  | CoverageFormat1 VerticalPunctuationCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0004 | 4 | glyphCount |
| 003C | HorizontalOpenBracketGlyph | glyphArray[0], ordered by glyph ID |
| 0040 | HorizontalClosedBracketGlyph | glyphArray[1] |
| 004B | HorizontalOpenParenthesisGlyph | glyphArray[2] |
| 004F | HorizontalClosedParenthesisGlyph | glyphArray[3] |

Example 4: MultipleSubstFormat1 subtable

Example 4 uses a MultipleSubstFormat1 subtable to replace a single "ffi" ligature with three individual glyphs that form the string <ffi>. The subtable defines a format identifier of 1, an offset to a Coverage table that specifies the glyph index of the "ffi" ligature (the input glyph), an offset to a Sequence table that specifies the sequence of glyph indices for the <ffi> string in its substitute array (the output glyph sequence), and a count of Sequence table offsets.

Example 4

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | MultipleSubstFormat1 FfiDecompSubtable | MultipleSubst subtable definition |
| 0001 | 1 | substFormat |
| 0008 | FfiDecompCoverage | Offset to Coverage table |
| 0001 | 1 | sequenceCount – equals glyphCount in Coverage table |
| 000E | FfiDecompSequence | sequenceOffsets[0] (offset to Sequence table 0) |
|  | CoverageFormat1 FfiDecompCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 00F1 | ffiGlyphID | ligature glyph |
|  | Sequence FfiDecompSequence | Sequence table definition |
| 0003 | 3 | glyphCount |
| 001A | fGlyphID | first glyph in sequence order |
| 001A | fGlyphID | second glyph |
| 001D | iGlyphID | third glyph |

Example 5: AlternateSubstFormat 1 subtable

Example 5 uses the AlternateSubstFormat1 subtable to replace the default ampersand glyph (input glyph) with one of two alternative ampersand glyphs (output glyph).

In this case, the Coverage table specifies the index of a single glyph, the default ampersand, because it is the only glyph covered by this lookup. The AlternateSet table for this covered glyph identifies the alternative glyphs: AltAmpersand1GlyphID and AltAmpersand2GlyphID.

In Example 5, the index position of the AlternateSet table offset in the AlternateSet array is zero (0), which correlates with the index position (also zero) of the default ampersand glyph in the Coverage table.

Example 5

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | AlternateSubstFormat1 AltAmpersandSubtable | AlternateSubstFormat1 subtable definition |
| 0001 | 1 | substFormat |
| 0008 | AltAmpersandCoverage | Offset to Coverage table |
| 0001 | 1 | alternateSetCount – equals glyphCount in Coverage table |
| 000E | AltAmpersandSet | alternateSetOffsets[0] (offset to AlternateSet table 0) |
|  | CoverageFormat1 AltAmpersandCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0001 | 1 | glyphCount |
| 003A | DefaultAmpersandGlyphID | glyphArray[0] |
|  | AlternateSet AltAmpersandSet | AlternateSet table definition |
| 0002 | 2 | glyphCount |
| 00C9 | AltAmpersand1GlyphID | alternateGlyphIDs[0] – glyphs in arbitrary order |
| 00CA | AltAmpersand2GlyphID | alternateGlyphIDs [1] |

Example 6: LigatureSubstFormat1 subtable

Example 6 shows a LigatureSubstFormat1 subtable that defines data to replace a string of glyphs with a single ligature glyph. Because a LigatureSubstFormat1 subtable can specify glyph substitutions for more than one ligature, this subtable defines three ligatures: "etc", "ffi", and "fi".

The sample subtable contains a format identifier (4) and an offset to a Coverage table. The Coverage table, which lists an index for each first glyph in the ligatures, lists indices for the "e" and "f" glyphs. The Coverage table range format is used here because the "e" and "f" glyph indices are numbered consecutively.

In the LigatureSubst subtable, ligatureSetCount specifies two LigatureSet tables, one for each covered glyph, and the ligatureSetOffsets array stores offsets to them. In this array, the "e" LigatureSet precedes the "f" LigatureSet, matching the order of the corresponding first-glyph components in the Coverage table.

Each LigatureSet table identifies all ligatures that begin with a covered glyph. The sample LigatureSet table defined for the "e" glyph contains only one ligature, "etc". A LigatureSet table defined for the "f" glyph contains two ligatures, "ffi" and "fi".

The sample FLigaturesSet table has offsets to two Ligature tables, one for "ffi" and one for "fi". The ligatureOffsets array lists the "ffi" Ligature table first to indicate that the "ffi" ligature is preferred to the "fi" ligature.

Example 6

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | LigatureSubstFormat1 LigaturesSubtable | LigatureSubstFormat1 subtable definition |
| 0001 | 1 | substFormat |
| 000A | LigaturesCoverage | Offset to Coverage table |
| 0002 | 2 | ligatureSetCount |
| 0014 | ELigaturesSet | ligatureSetOffsets[0] (offset to LigatureSet table 0) – LigatureSet tables in Coverage index order |
| 0020 | FLigaturesSet | ligatureSetOffsets[1] |
|  | CoverageFormat2 LigaturesCoverage | Coverage table definition |
| 0002 | 2 | coverageFormat: ranges |
| 0001 | 1 | rangeCount |
|  | rangeRecord[0] |  |
| 0019 | eGlyphID | Start, first glyph ID |
| 001A | fGlyphID | End, last glyph ID in range |
| 0000 | 0 | startCoverageIndex: coverage index of start glyphID = 0 |
|  | LigatureSet ELigaturesSet | LigatureSet table definition – all ligatures that start with e |
| 0001 | 1 | ligatureCount |
| 0004 | etcLigature | ligatureOffsets[0] (offset to Ligature table 0) |
|  | Ligature etcLigature | Ligature table definition |
| 015B | etcGlyphID | ligatureGlyph – output glyph ID |
| 0003 | 3 | componentCount |
| 0028 | tGlyphID | componentGlyphIDs[0] – second component in ligature |
| 0017 | cGlyphID | componentGlyphIDs[1] – third component in ligature |
|  | LigatureSet FLigaturesSet | LigatureSet table definition all ligatures start with f |
| 0002 | 2 | ligatureCount |
| 0006 | ffiLigature | ligatureOffsets[0] – listed first because ffi ligature is preferred to fi ligature |
| 000E | fiLigature | ligatureOffsets [1] |
|  | Ligature ffiLigature | Ligature table definition |
| 00F1 | ffiGlyphID | ligatureGlyph – output glyph ID |
| 0003 | 3 | componentCount |
| 001A | fGlyphID | componentGlyphIDs[0] – second component in ligature |
| 001D | iGlyphID | componentGlyphIDs[1] – third component in ligature |
|  | Ligature fiLigature | Ligature table definition |
| 00F0 | fiGlyphID | ligatureGlyph – output glyph ID |
| 0002 | 2 | componentCount |
| 001D | iGlyphID | componentGlyphIDs[0] – second component in ligature |

Example 7: ContextSubstFormat1 subtable and SubstLookupRecord

Example 7 uses a ContextSubstFormat1 subtable for glyph sequences to replace a string of three glyphs with another string. For the French language system, the subtable defines a contextual substitution that replaces the input sequence, space-dash-space, with the output sequence, thin space-dash-thin space.

The contextual substitution, called Dash Lookup in this example, contains one ContextSubstFormat1 subtable called the DashSubtable. The subtable specifies two contexts: a SpaceGlyph followed by a DashGlyph, and a DashGlyph followed by a SpaceGlyph. In each sequence, a single substitution replaces the SpaceGlyph with a ThinSpaceGlyph.

The Coverage table, labelled DashCoverage, lists two glyph IDs for the first glyphs in the SpaceGlyph and DashGlyph sequences. One SubRuleSet table is defined for each covered glyph.

SpaceAndDashSubRuleSet lists all the contexts that begin with a SpaceGlyph. It contains an offset to one SubRule table (SpaceAndDashSubRule), which specifies two glyphs in the context sequence, the second of which is a DashGlyph. The SubRule table contains an offset to a SubstLookupRecord that lists the position in the sequence where the glyph substitution should occur (position 0) and the index of the SpaceToThinSpaceLookup applied there to replace the SpaceGlyph with a ThinSpaceGlyph. DashAndSpaceSubRuleSet lists all the contexts that begin with a DashGlyph. An offset to a SubRule table (DashAndSpaceSubRule) specifies two glyphs in the context sequence, and the second one is a SpaceGlyph. The SubRule table contains an offset to a SubstLookupRecord, which lists the position in the sequence where the glyph substitution should occur, and an index to the same lookup used in the SpaceAndDashSubRule. The lookup replaces the SpaceGlyph with a ThinSpaceGlyph.

Example 7

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextSubstFormat1 DashSubtable | ContextSubstFormat1 subtable definition for Lookup[0], DashLookup |
| 0001 | 1 | substFormat |
| 000A | DashCoverage | Offset to Coverage table |
| 0002 | 2 | subRuleSetCount |
| 0012 | SpaceAndDashSubRuleSet | subRuleSetOffsets[0] (offset to SubRuleSet table 0) – SubRuleSets ordered by Coverage index |
| 0020 | DashAndSpaceSubRuleSet | subRuleSetOffsets[1] |
|  | CoverageFormat1 DashCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0002 | 2 | glyphCount |
| 0028 | SpaceGlyph | glyphArray[0] – glyphs in numeric order |
| 005D | DashGlyph | glyphArray[1], dash glyph ID |
|  | SubRuleSet SpaceAndDashSubRuleSet | SubRuleSet[0] table definition |
| 0001 | 1 | subRuleCount |
| 0004 | SpaceAndDashSubRule | subRuleSetOffsets[0] (offset to SubRule table 0) – SubRule tables ordered by preference |
|  | SubRule SpaceAndDashSubRule | SubRule[0] table definition |
| 0002 | 2 | glyphCount – number in input sequence |
| 0001 | 1 | substitutionCount |
| 005D | DashGlyph | inputSequence[0], starting with second glyph – SpaceGlyph, in Coverage table, is first glyph |
|  | substLookupRecords[0] |  |
| 0000 | 0 | sequenceIndex – substitution at first glyph position (0) |
| 0001 | 1 | lookupListIndex – index for SpaceToThinSpaceLookup in LookupList |
|  | SubRuleSet DashAndSpaceSubRuleSet | SubRuleSet[1] table definition |
| 0001 | 1 | subRuleCount |
| 0004 | DashAndSpaceSubRule | subRuleOffsets[0] (offset to SubRule table 0) – SubRule tables ordered by preference |
|  | SubRule DashAndSpaceSubRule | SubRule[0] table definition |
| 0002 | 2 | glyphCount – number in the input glyph sequence |
| 0001 | 1 | substitutionCount |
| 0028 | SpaceGlyph | inputSequence[0] – starting with second glyph |
|  | substLookupRecords[0] |  |
| 0001 | 1 | sequenceIndex – substitution at second glyph position (glyph sequence index = 1) |
| 0001 | 1 | lookupListIndex – index for SpaceToThinSpaceLookup |

Example 8: ContextSubstFormat2 subtable

Example 8 uses a ContextSubstFormat2 subtable with glyph classes to replace default mark glyphs with their alternative forms. Glyph alternatives are selected depending upon the height of the base glyph that they combine with-that is, the mark glyph used above a high base glyph differs from the mark glyph above a very high base glyph.

In the example, SetMarksHighSubtable contains a Class table that defines four glyph classes: medium-height glyphs (Class 0), all default mark glyphs (Class 1), high glyphs (Class 2), and very high glyphs (Class 3). The subtable also contains a Coverage table that lists each base glyph that functions as a first component in a context, ordered by glyph index.

Two SubClassSets are defined, one for substituting high marks and one for very high marks. No SubClassSets are specified for Class 0 and Class 1 glyphs because no contexts begin with glyphs from these classes. The SubClassSet array lists SubClassSets in numerical order, so SubClassSet 2 precedes SubClassSet 3.

Within each SubClassSet, a SubClassRule is defined. In SetMarksHighSubClassSet2, the SubClassRule table specifies two glyphs in the context, the first glyph in Class 2 (a high glyph) and the second in Class 1 (a mark glyph). The SubstLookupRecord specifies applying SubstituteHighMarkLookup at the second position in the sequence-that is, a high mark glyph will replace the default mark glyph.

In SetMarksVeryHighSubClassSet3, the SubClassRule specifies two glyphs in the context, the first in Class 3 (a very high glyph) and the second in Class 1 (a mark glyph). The SubstLookupRecord specifies applying SubstituteVeryHighMarkLookup at the second position in the sequence-that is, a very high mark glyph will replace the default mark glyph.

Example 8

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextSubstFormat2 SetMarksHighSubtable | ContextSubstFormat2 subtable definition |
| 0002 | 2 | substFormat |
| 0010 | SetMarksHighCoverage | Offset to Coverage table |
| 001C | SetMarksHighClassDef | Offset to Class Def table |
| 0004 | 4 | subClassSetCount |
| 0000 | NULL | subClassSetOffsets[0] – NULL: no contexts that begin with Class 0 glyphs are defined |
| 0000 | NULL | subClassSetOffsets[1] – NULL: no contexts that begin with Class 1 glyphs are defined |
| 0032 | SetMarksHighSubClassSet2 | subClassSetOffsets[2] – offset to SubClassSet table for contexts that begin with Class 2 glyphs (high base glyphs) |
| 0040 | SetMarksVeryHighSubClassSet3 | subClassSetOffsets[2] – offset to SubClassSet table for contexts that begin with Class 3 glyphs (very high base glyphs) |
|  | CoverageFormat1 SetMarksHighCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0004 | 4 | glyphCount |
| 0030 | tahGlyphID | glyphArray[0], high base glyph |
| 0031 | dhahGlyphID | glyphArray[1], high base glyph |
| 0040 | cafGlyphID | glyphArray[2], very high base glyph |
| 0041 | gafGlyphID | glyphArray[3], very high base glyph |
|  | ClassDefFormat2 SetMarksHighClassDef | Class table definition |
| 0002 | 2 | class Format: ranges |
| 0003 | 3 | classRangeCount |
|  | classRangeRecords[0] | ClassRangeRecords ordered by startGlyphID; record for Class 2, high base glyphs |
| 0030 | tahGlyphID | Start, first glyph ID in range |
| 0031 | dhahGlyphID | End, last glyph ID in range |
| 0002 | 2 | class: 2 |
|  | classRangeRecords[1] | ClassRangeRecord for Class 3, very high base glyphs |
| 0040 | cafGlyphID | Start, first glyph ID in the range |
| 0041 | gafGlyphID | End, last glyph ID in the range |
| 0003 | 3 | class: 3 |
|  | classRangeRecords[2] | ClassRangeRecord for Class 1, mark glyphs |
| 00D2 | fathatanDefaultGlyphID | Start, first glyph ID in range default fathatan mark |
| 00D3 | dammatanDefaultGlyphID | End, last glyph ID in the range default dammatan mark |
| 0001 | 1 | class: 1 |
|  | SubClassSet SetMarksHighSubClassSet2 | SubClassSet[2] table definition all contexts that begin with Class 2 glyphs |
| 0001 | 1 | subClassRuleCount |
| 0004 | SetMarksHighSubClassRule2 | subClassRuleOffsets[0] (offset to SubClassRule table 0) – SubClassRule tables ordered by preference |
|  | SubClassRule SetMarksHighSubClassRule2 | SubClassRule[0] table definition, Class 2 glyph (high base) glyph followed by a Class 1 glyph (mark) |
| 0002 | 2 | glyphCount |
| 0001 | 1 | substitutionCount |
| 0001 | 1 | input Sequence[0] – input sequence beginning with the second Class in the input context sequence; Class 1, mark glyphs |
|  | substLookupRecords[0] | substLookupRecords array in design order |
| 0001 | 1 | sequenceIndex – apply substitution to position 2, a mark |
| 0001 | 1 | lookupListIndex |
|  | SubClassSet SetMarksVeryHighSubClassSet3 | SubClassSet[3] table definition – all contexts that begin with Class 3 glyphs |
| 0001 | 1 | subClassRuleCount |
| 0004 | SetMarksVeryHighSubClassRule3 | subClassRuleOffsets[0] |
|  | SubClassRule SetMarksVeryHighSubClassRule3 | SubClassRule[0] table definition – Class 3 glyph (very high base glyph) followed by a Class 1 glyph (mark) |
| 0002 | 2 | glyphCount |
| 0001 | 1 | substitutionCount |
| 0001 | 1 | inputSequence[0] – input sequence beginning with the second Class in the input context sequence; Class 1 , mark glyphs |
|  | substLookupRecords[0] | substLookupRecords array in design order |
| 0001 | 1 | sequenceIndex – apply substitution to position 2, second glyph class (mark) |
| 0002 | 2 | lookupListIndex |

Example 9: ContextualSubstFormat3 subtable

Example 9 uses the ContextSubstFormat3 subtable with Coverage tables to describe a context sequence of three lowercase glyphs in the pattern: any ascender or descender glyph in position 0 (zero), any x-height glyph in position 1, and any descender glyph in position 2. The overlapping sets of covered glyphs for positions 0 and 2 make Format 3 better for this context than the class-based Format 2.

In positions 0 and 2, swash versions of the glyphs replace the default glyphs. The contextual-substitution lookup is SwashLookup (LookupList index = 0), and its subtable is SwashSubtable. The SwashSubtable defines three Coverage tables: AscenderDescenderCoverage, XheightCoverage, and DescenderCoverage-one for each glyph position in the context sequence, respectively.

The SwashSubtable also defines two SubstLookupRecords: one that applies to position 0, and one for position 2. (No substitutions are applied to position 1.) The record for position 0 uses a single substitution lookup called AscDescSwashLookup to replace the current ascender or descender glyph with a swash ascender or descender glyph. The record for position 2 uses a single substitution lookup called DescSwashLookup to replace the current descender glyph with a swash descender glyph.

Example 9

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ContextSubstFormat3 SwashSubtable | ContextSubstFormat3 subtable definition |
| 0003 | 3 | substFormat |
| 0003 | 3 | glyphCount – number in input glyph sequence |
| 0002 | 2 | substitutionCount |
| 0030 | AscenderDescenderCoverage | coverageOffsets[0] – offsets to Coverage tables, in context sequence order |
| 004C | XheightCoverage | coverageOffsets [1] |
| 006E | DescenderCoverage | coverageOffsets [2] |
|  | substLookupRecords[0] | SubstLookupRecords in glyph position order |
| 0000 | 0 | sequenceIndex |
| 0001 | 1 | lookupListIndex – single substitution to output ascender or descender swash |
|  | substLookupRecords[1] |  |
| 0002 | 2 | sequenceIndex |
| 0002 | 2 | lookupListIndex – single substitution to output descender swash |
|  | CoverageFormat1 AscenderDescenderCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 000C | 12 | glyphCount |
| 0033 | bGlyphID | glyphArray[0] – glyphs in glyph ID order |
| 0035 | dGlyphID | glyphArray[1] |
| 0037 | fGlyphID | glyphArray[2] |
| 0038 | gGlyphID | glyphArray[3] |
| 0039 | hGlyphID | glyphArray[4] |
| 003B | jGlyphID | glyphArray[5] |
| 003C | kGlyphID | glyphArray[6] |
| 003D | lGlyphID | glyphArray[7] |
| 0041 | pGlyphID | glyphArray[8] |
| 0042 | qGlyphID | glyphArray[9] |
| 0045 | tGlyphID | glyphArray[10] |
| 004A | yGlyphID | glyphArray[11] |
|  | CoverageFormat1 XheightCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 000F | 15 | glyphCount |
| 0032 | aGlyphID | glyphArray[0] |
| 0034 | cGlyphID | glyphArray[1] |
| 0036 | eGlyphID | glyphArray[2] |
| 003A | iGlyphID | glyphArray[3] |
| 003E | mGlyphID | glyphArray[4] |
| 003F | nGlyphID | glyphArray[5] |
| 0040 | oGlyphID | glyphArray[6] |
| 0043 | rGlyphID | glyphArray[7] |
| 0044 | sGlyphID | glyphArray[8] |
| 0045 | tGlyphID | glyphArray[9] |
| 0046 | uGlyphID | glyphArray[10] |
| 0047 | vGlyphID | glyphArray[11] |
| 0048 | wGlyphID | glyphArray[12] |
| 0049 | xGlyphID | glyphArray[13] |
| 004B | zGlyphID | glyphArray[14] |
|  | CoverageFormat1 DescenderCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 0005 | 5 | glyphCount |
| 0038 | gGlyphID | glyphArray[0] |
| 003B | jGlyphID | glyphArray[1] |
| 0041 | pGlyphID | glyphArray[2] |
| 0042 | qGlyphID | glyphArray[3] |
| 004A | yGlyphID | glyphArray[4] |

Example 10: ReverseChainSingleSubstFormat1 subtable and SubstLookupRecord

Example 10 uses a ReverseChainSingleSubstFormat1 subtable for glyph sequences to glyph with the correct form that has a thick connection to the left (thick exit). This allow the glyph to correctly connect to the letter form to the left of it.

The ThickExitCoverage table is the listing of glyphs to be matched for substitution.

The LookaheadCoverage table, labelled ThickEntryCoverage, lists four glyph IDs for the glyph following a substitution coverage glyph. This lookahead coverage attempts to match the context that will cause the substitution to take place.

The Substitute table maps the glyphs to replace those in the ThickConnectCoverage table.

Example 10

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | ReverseChainSingleSubstFormat1 ThickConnect | ReverseChainSingleSubstFormat1 subtable definition |
| 0001 | 1 | substFormat |
| 0068 | ThickExitCoverage | Offset to Coverage table |
| 0000 | 0 | backtrackGlyphCount |
| 0000 | null - not used | backtrackCoverageOffsets[0] |
| 0001 | 1 | lookaheadGlyphCount |
| 0026 | ThickEntryCoverage | lookaheadCoverageOffsets[0] |
| 000C | 12 | glyphCount |
| 00A7 | BEm2 | substituteGlyphIDs[0] – substitute glyphs ordered by Coverage index |
| 00B9 | BEi3 | substituteGlyphIDs [1] |
| 00C5 | JIMm3 | substituteGlyphIDs [2] |
| 00D4 | JIMi2 | substituteGlyphIDs [3] |
| 00EA | SINm2 | substituteGlyphIDs [4] |
| 00F2 | SINi2 | substituteGlyphIDs [5] |
| 00FD | SADm2 | substituteGlyphIDs [6] |
| 010D | SADi2 | substituteGlyphIDs [7] |
| 011B | TOEm3 | substituteGlyphIDs [8] |
| 012B | TOEi3 | substituteGlyphIDs [9] |
| 013B | AINm2 | substituteGlyphIDs [10] |
| 0141 | AINi2 | substituteGlyphIDs [11] |
|  | CoverageFormat1 ThickEntryCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 001F | 31 | glyphCount |
| 00A5 | ALEFf1 | glyphArray[0] – glyphs in glyph ID order |
| 00A9 | BEm4 | glyphArray[1] |
| 00AA | BEm5 | glyphArray[2] |
| 00E2 | DALf1 | glyphArray[3] |
| 0167 | KAFf1 | glyphArray[4] |
| 0168 | KAFfs1 | glyphArray[5] |
| 0169 | KAFm1 | glyphArray[6] |
| 016D | KAFm5 | glyphArray[7] |
| 016E | KAFm6 | glyphArray[8] |
| 0170 | KAFm8 | glyphArray[9] |
| 0183 | GAFf1 | glyphArray[10] |
| 0184 | GAFfs1 | glyphArray[11] |
| 0185 | GAFm1 | glyphArray[12] |
| 0189 | GAFm5 | glyphArray[13] |
| 018A | GAFm6 | glyphArray[14] |
| 018C | GAFm8 | glyphArray[15] |
| 019F | LAMf1 | glyphArray[16] |
| 01A0 | LAMm1 | glyphArray[17] |
| 01A1 | LAMm2 | glyphArray[18] |
| 01A2 | LAMm3 | glyphArray[19] |
| 01A3 | LAMm4 | glyphArray[20] |
| 01A4 | LAMm5 | glyphArray[21] |
| 01A5 | LAMm6 | glyphArray[22] |
| 01A6 | LAMm7 | glyphArray[23] |
| 01A7 | LAMm8 | glyphArray[24] |
| 01A8 | LAMm9 | glyphArray[25] |
| 01A9 | LAMm10 | glyphArray[26] |
| 01AA | LAMm11 | glyphArray[27] |
| 01AB | LAMm12 | glyphArray[28] |
| 01AC | LAMm13 | glyphArray[29] |
| 01EC | HAYf2 | glyphArray[30] |
|  | CoverageFormat1 ThickExitCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat: lists |
| 000C | 12 | glyphCount |
| 00A6 | BEm1 | glyphArray[0] |
| 00B7 | BEi1 | glyphArray[1] |
| 00C3 | JIMm1 | glyphArray[2] |
| 00D2 | JIMi1 | glyphArray[3] |
| 00E9 | SINm1 | glyphArray[4] |
| 00F1 | SINi1 | glyphArray[5] |
| 00FC | SADm1 | glyphArray[6] |
| 010C | SADi1 | glyphArray[7] |
| 0119 | TOEm1 | glyphArray[8] |
| 0129 | TOEi1 | glyphArray[9] |
| 013A | AINm1 | glyphArray[10] |
| 0140 | AINi1 | glyphArray[11] |

### JSTF—The justification table

#### JSTF table overview

The Justification table (JSTF) provides font developers with additional control over glyph substitution and positioning in justified text. Text-processing clients now have more options to expand or shrink word and glyph spacing so text fills the specified line length.

When justifying text, the text-processing client distributes the characters in each line to completely fill the specified line length. Whether removing space to fit more characters in the line or adding more space to spread the characters, justification can produce large gaps between words, cramped or extended glyph spacing, uneven line break patterns, and other jarring visual effects. For example:

Figure 6.35 – Poorly justified text

To offset these effects, text-processing clients have used justification algorithms that redistribute the space with a series of glyph spacing adjustments that progress from least to most obvious. Typically, the client will begin by expanding or compressing the space between words. If these changes aren't enough or look distracting, the client might hyphenate the word at the end of the line or adjust the space between glyphs in one or more lines.

To disguise spacing inconsistencies so they won't disrupt the flow of text for a reader, the font developer can use the JSTF table to enable or disable individual glyph substitution and positioning actions that apply to specific scripts, language systems, and glyphs in the font.

For instance, a ligature glyph can replace multiple glyphs, shortening the line of text with an unobtrusive, localized adjustment (see Figure 6.36). Font-specific positioning changes can be applied to particular glyphs in a text line that combines two or more fonts. Other options include repositioning the individual glyphs in the line, expanding the space between specific pairs of glyphs, and decreasing the spacing within particular glyph sequences.

Figure 6.36 – JSTF shortens the top line of this example by using the "ffi" ligature

The font designer or developer defines JSTF data as prioritized suggestions. Each suggestion lists the particular actions that the client can use to adjust the line of text. Justification actions may apply to both vertical and horizontal text.

#### Table organization and structure

The JSTF table organizes data by script and language system, as do the GSUB and GPOS tables. The JSTF table begins with a header that lists scripts in an array of JstfScriptRecords (see Figure 6.37). Each record contains a ScriptTag and an offset to a JstfScript table that contains script and language-specific data:

- A default justification language system table (JstfLangSys) defines script-specific data that applies to the entire script in the absence of any language-specific information.

- A justification language system table stores the justification data for each language system.

Figure 6.37 – High-level organization of JSTF table

A JstfLangSys table contains a list of justification suggestions. Each suggestion consists of a list of GSUB or GPOS LookupList indices to lookups that may be enabled or disabled to add or remove space in the line of text. In addition, each suggestion can include a set of dedicated justification lookups with maximum adjustment values to extend or shrink the amount of space.

The font developer prioritizes suggestions based on how they affect the appearance and function of the text line, and the client applies the suggestions in that order. Low-numbered (high-priority) suggestions correspond to "least bad" options.

Each script also may supply a list of extender glyphs, such as kashidas in Arabic. A client may use the extender glyphs in addition to the justification suggestions.

A client begins justifying a line of text only after implementing all selected GSUB and GPOS features for the string. Starting with the lowest-numbered suggestion, the client enables or disables the lookups specified in the JSTF table, reassembles the lookups in the LookupList order, and applies them to each glyph in the string one after another. If the line still is not the correct length, the client processes the next suggestion in ascending order of priority. This continues until the line length meets the justification requirements.

NOTE 1 	If any JSTF suggestion at any priority level modifies a GSUB or GPOS lookup that was previously applied to the glyph string, then the text processing client must apply the JSTF suggestion to an unmodified version of the glyph string.

NOTE 2	A FeatureVariations table may be used in either the GSUB or GPOS table to substitute the lookups triggered by a given feature with an alternate set of lookups based on certain conditions. (Currently, these conditions can pertain only to the use of variable fonts, which are discussed further below.) The actual lookups that were applied for a given feature may be different from the default set of lookups for that feature. When processing a justification suggestion, the list of lookups to check for previous application should be the actual lookups that applied, with any feature variations in effect, not the default lookups. Also, when adding data in the JSTF table to disable GSUB or GPOS lookups, the font developer should consider possible interactions with feature variation tables, such as a need to include such alternate lookups in the set of lookups to disable. Note that this applies only to features and lookups in the GSUB and GPOS table: for lookups contained directly within the JSTF table, there is no analogous feature-variation mechanism.

Later in this subclause, the following tables and records used by the JSTF table for scripts and language systems will be described:

- Script information, including the JstfScript table (plus its associated JstfLangSysRecords) and the ExtenderGlyph table.

- Language system information, including the JstfLangSys table, JstfPriority table (and its associated JstfDataRecord), the JstfModList table, and the JstfMax table.

JSTF table and OFF font variations

OFF font variations allow a single font to support many design variations along one or more axes of design variation. For example, a font with weight and width variations might support weights from thin to black, and widths from ultra-condensed to ultra-expanded. For general information on OFF font variations, see subclause 7.1.

When different variation instances are selected, the design and metrics of individual glyphs change, and the metric characteristics of the font as a whole may also change. As metrics are relevant for justification, the interaction between justification data and variations requires consideration.

As noted above, justification is assumed to be an iterative process in which the application tests different suggestions defined in the font in priority order to find a suggestion that results in a line length that meets application-determined justification requirements. When an instance of a variable font is selected, the line layout will use glyph outlines and metrics that are adjusted for that instance. Thus, the metrics for one variation instance may be different from another, and the text content of a given line after justification may be different if formatted with different variation instances, but the justification processing proceeds in the same manner.

As also noted above, justification suggestions are applied to the results after selected GSUB and GPOS features have been processed. If the GSUB or GPOS table includes a FeatureVariations table, there may be interactions between the effects of the FeatureVariations table and the justification suggestions. See above for additional discussion.

As noted, justification suggestions can make use of GPOS lookups contained within the GPOS table or directly within the JSTF table. GPOS lookup subtables contain X or Y font-unit values that specify modifications to individual glyph positions or metrics. In a variable font, these X and Y values apply to the default instance, and may require adjustment for different variation instances. This is done using variation data with processes similar to those used for glyph outlines and other font data, as described in subclause 7.1 (Font variations overview). Variation data for adjustment of X or Y values in GPOS lookups is stored within an item variation store table located within the GDEF table. This is true for lookups in either the GPOS or the JSTF table. The same item variation store is also used for adjustment of values in the GDEF table. See the GPOS chapter for additional details regarding variation of GPOS lookup values in variable fonts.

JSTF header

The JSTF table begins with a header that contains a version number for the table, a count of the number of scripts used in the font (jstfScriptCount), and an array of records (jstfScriptRecord). Each record contains a script tag (jstfScriptTag) and an offset to a JstfScript table (jstfScriptOffset).

Note that the jstfScriptTag values shall correspond with the script tags listed in the GSUB and GPOS tables.

Example 1 at the end of this clause shows a JSTF Header table and JstfScriptRecord.

JSTF header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the JSTF table, = 1 |
| uint16 | minorVersion | Minor version of the JSTF table, = 0 |
| uint16 | jstfScriptCount | Number of JstfScriptRecords in this table |
| JstfScriptRecord | jstfScriptRecords[jstfScriptCount] | Array of JstfScriptRecords, in alphabetical order by jstfScriptTag |

JstfScriptRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | jstfScriptTag | 4-byte JstfScript identification |
| Offset16 | jstfScriptOffset | Offset to JstfScript table, from beginning of JSTF Header |

Justification script table

A Justification Script (JstfScript) table describes the justification information for a single script. It consists of an offset to a table that defines extender glyphs (extenderGlyphOffset), an offset to a default justification table for the script (defJstfLangSysOffset), and a count of the language systems that define justification data (jstfLangSysCount).

If a script uses the same justification information for all language systems, the font developer defines only the default JstfLangSys table and sets the jstfLangSysCount value to zero (0). However, if any language system has unique justification suggestions, jstfLangSysCount will be a positive value, and the JstfScript table must include an array of records (jstfLangSysRecords), one for each language system. Each JstfLangSysRecord contains a language system tag (jstfLangSysTag) and an offset to a justification language system table (jstfLangSysOffset). In the jstfLangSysRecords array, records are ordered alphabetically by jstfLangSysTag.

NOTE	No JstfLangSysRecord is defined for the default script data; the data is stored in the DefJstfLangSys table instead.

Example 2 at the end of the clause shows a JstfScript table for the Arabic script and a JstfLangSysRecord for the Farsi language system.

JstfScript table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | extenderGlyphOffset | Offset to ExtenderGlyph table, from beginning of JstfScript table (may be NULL) |
| Offset16 | defJstfLangSysOffset | Offset to Default JstfLangSys table, from beginning of JstfScript table (may be NULL) |
| uint16 | jstfLangSysCount | Number of JstfLangSysRecords in this table, may be zero (0) |
| JstfLangSysRecord | jstfLangSysRecords [jstfLangSysCount] | Array of JstfLangSysRecords, in alphabetical order by jstfLangSysTag |

JstfLangSysRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | jstfLangSysTag | 4-byte JstfLangSys identifier |
| Offset16 | jstfLangSysOffset | Offset to JstfLangSys table, from beginning of JstfScript table |

Extender glyph table

The Extender Glyph table (ExtenderGlyph) lists indices of glyphs, such as Arabic kashidas, that a client may insert to extend the length of the line for justification. The table consists of a count of the extender glyphs for the script (glyphCount) and an array of extender glyph indices (extenderGlyphs), arranged in increasing numerical order.

Example 2 at the end of this clause shows an ExtenderGlyph table for Arabic kashida glyphs.

ExtenderGlyph table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphCount | Number of extender glyphs in this script |
| uint16 | extenderGlyphs[glyphCount] | Extender glyph IDs – in increasing numerical order |

Justification Language System table

The Justification Language System (JstfLangSys) table contains an array of justification suggestions, ordered by priority. A text-processing client doing justification should begin with the suggestion that has a zero (0) priority, and then-as necessary-apply suggestions of increasing priority until the text is justified.

The font developer defines the number and the meaning of the priority levels. Each priority level stands alone; its suggestions are not added to the previous levels. The JstfLangSys table consists of a count of the number of priority levels (jstfPriorityCount) and an array of offsets to Justification Priority tables (jstfPriorityOffsets), stored in priority order. Example 2 at the end of the clause shows how to define a JstfLangSys table.

JstfLangSys table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | jstfPriorityCount | Number of JstfPriority tables |
| Offset16 | jstfPriorityOffsets[jstfPriorityCount] | Array of offsets to JstfPriority tables, from beginning of JstfLangSys table, in priority order |

Justification Priority table

A Justification Priority (JstfPriority) table defines justification suggestions for a single priority level. Each priority level specifies whether to enable or disable GSUB and GPOS lookups or apply text justification lookups to shrink and extend lines of text.

JstfPriority has offsets to four tables with line shrinkage data: two are JstfGSUBModList tables for enabling and disabling glyph substitution lookups, and two are JstfGPOSModList tables for enabling and disabling glyph positioning lookups. offsets to JstfGSUBModList and JstfGPOSModList tables also are defined for line extension.

Example 3 at the end of this clause demonstrates two JstfPriority tables for two justification suggestions.

JstfPriority table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | shrinkageEnableGSUB | Offset to shrinkage-enable JstfGSUBModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | shrinkageDisableGSUB | Offset to shrinkage-disable JstfGSUBModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | shrinkageEnableGPOS | Offset to shrinkage-enable JstfGPOSModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | shrinkageDisableGPOS | Offset to shrinkage-disable JstfGPOSModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | shrinkageJstfMax | Offset to shrinkage JstfMax table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | extensionEnableGSUB | Offset to extension-enable JstfGSUBModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | extensionDisableGSUB | Offset to extension-disable JstfGSUBModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | extensionEnableGPOS | Offset to extension-enable JstfGPOSModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | extensionDisableGPOS | Offset to extension-disable JstfGPOSModList table, from beginning of JstfPriority table (may be NULL) |
| Offset16 | extensionJstfMax | Offset to extension JstfMax table, from beginning of JstfPriority table (may be NULL) |

Justification Modification List tables

The Justification Modification List tables (JstfGSUBModList and JstfGPOSModList) contain lists of indices into the lookup lists of either the GSUB or GPOS tables. The client can enable or disable the lookups to justify text. For example, to increase line length, the client might disable a GSUB ligature substitution.

Each JstfModList table consists of a count of Lookups (LookupCount) and an array of lookup indices (LookupIndex).

To justify a line of text, a text-processing client enables or disables the specified lookups in a JstfModList table, reassembles the lookups in the LookupList order, and applies them to each glyph in the string one after another.

NOTE	If any JSTF suggestion at any priority level modifies a GSUB or GPOS lookup previously applied to the glyph string, then the text-processing client must apply the JSTF suggestion to an unmodified version of the glyph string.

Example 3 at the end of this clause shows JstfGSUBModList and JstfGPOSModList tables with data for shrinking and extending text line lengths.

JstfGSUBModList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | lookupCount | Number of lookups for this modification |
| uint16 | gsubLookupIndices[lookupCount] | Array of Lookup indices into the GSUB Lookup List, in increasing numerical order |

JstfGPOSModList table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | lookupCount | Number of lookups for this modification |
| uint16 | gposLookupIndices[lookupCount] | Array of Lookup indices into the GPOS Lookup List, in increasing numerical order |

Justification Maximum table

A Justification Maximum table (JstfMax) consists of an array of offsets to justification lookups (Lookup) and a count of the defined lookups (Lookup). JstfMax lookups typically are located after the JstfMax table in the font definition.

JstfMax tables have the same format as lookup tables and subtables in the GPOS table, but the JstfMax lookups reside in the JSTF table and contain justification data only. The lookup data might specify a single adjustment value for positioning all glyphs in the script, or it might specify more elaborate adjustments, such as different values for different glyphs or special values for specific pairs of glyphs.

NOTE	All GPOS lookup types except contextual positioning lookups may be defined in a JstfMax table.

JstfMax lookup values are defined in GPOS ValueRecords and may be specified for any advance or placement position, whether horizontal or vertical. These values define the maximum shrinkage or extension allowed per glyph. To justify text, a text-processing client may choose to adjust a glyph's positioning by any amount from zero (0) to the specified maximum.

Example 4 at the end of this clause shows a JstfMax table. It defines a justification lookup to change the size of the word space glyph to extend line lengths.

JstfMax table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | lookupCount | Number of lookup Indices for this modification |
| Offset16 | lookupOffsets[lookupCount] | Array of offsets to GPOS-type lookup tables, from beginning of JstfMax table, in design order |

#### JSTF table examples

The rest of this clause describes examples of all the JSTF table formats. All the examples reflect unique parameters described below, but the samples provide a useful reference for building tables specific to other situations.

The examples have three columns showing hex data, source, and comments.

Example 1: JSTF header table and JstfScriptRecord

Example 1 demonstrates how a script is defined in the JSTF Header with a JstfScriptRecord that identifies the script and references its JstfScript table.

Example 1

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | JSTFHeader TheJSTFHeader | JSTFHeader table definition |
| 00010000 | 0x00010000 | major/minor version |
| 0001 | 1 | jstfScriptCount |
|  | jstfScriptRecords[0] |  |
| 74686169 | 'thai' | jstfScriptTag |
| 000C | ThaiScript | Offset to JstfScript table |

Example 2: JstfScript table, ExtenderGlyph table, JstfLangSysRecord, and JstfLangSys table

Example 2 shows a JstfScript table for the Arabic script and the tables it references. The default JstfLangSys table defines justification data to apply to the script in the absence of language-specific information. In the example, the table lists two justification suggestions in priority order.

JstfScript also supplies language-specific justification data for the Farsi language. The JstfLangSysRecord identifies the language and references its JstfLangSys table. The FarsiJstfLangSys lists one suggestion for justifying Farsi text.

The ExtenderGlyph table in JstfScript lists the indices of all the extender glyphs used in the script.

Example 2

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | JstfScript ArabicScript | JstfScript table definition |
| 000C | ArabicExtenders | extenderGlyphOffset |
| 0012 | ArabicDefJstfLangSys | Offset to default JstfLangSys table |
| 0001 | 1 | jstfLangSysCount |
|  | jstfLangSysRecords[0] |  |
| 50455220 | 'FAR ' | jstfLangSysTag |
| 0018 | FarsiJstfLangSys | jstfLangSysOffset |
|  | ExtenderGlyph ArabicExtenders | ExtenderGlyph table definition |
| 0002 | 2 | glyphCount |
| 01D3 | TatweelGlyphID | extenderGlyphs[0] |
| 01D4 | LongTatweelGlyphID | extenderGlyphs[1] |
|  | JstfLangSys ArabicDefJstfLangSys | JstfLangSys table definition |
| 0002 | 2 | jstfPriorityCount |
| 000A | ArabicScriptJstfPriority1 | jstfPriorityOffsets[0] |
| 001E | ArabicScriptJstfPriority2 | jstfPriorityOffsets[1] |
|  | JstfLangSys FarsiJstfLangSys | JstfLangSys table definition |
| 0001 | 1 | jstfPriorityCount |
| 002C | FarsiLangJstfPriority1 | jstfPriorityOffsets[0] |

Example 3: JstfPriority table, JstfGSUBModList table, and JstfGPOSModList table

Example 3 shows the JstfPriority and JstfModList table definitions for two justification suggestions defined in priority order. The first suggestion uses ligature substitution to shrink the lengths of text lines, and it extends line lengths by replacing ligatures with their individual glyph components. Other lookup actions are not recommended at this priority level and are set to NULL. The associated JstfModList tables enable and disable three substitution lookups.

The second suggestion enables glyph kerning to reduce line lengths and disables glyph kerning to extend line lengths. Each action uses three lookups. This suggestion also includes a JstfMax table to extend line lengths, called WordSpaceExpandMax, which is described in Example 4.

Example 3

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | JstfPriority USEnglishFirstJstfPriority | JstfPriority table definition |
| 0028 | EnableGSUBLookupsToShrink | shrinkageEnableGSUB (offset to shrinkage-enable JstfGSUBModList table) |
| 0000 | NULL | shrinkageDisableGSUB |
| 0000 | NULL | shrinkageEnableGPOS |
| 0000 | NULL | shrinkageDisableGPOS |
| 0000 | NULL | shrinkageJstfMax |
| 0000 | NULL | eExtensionEnableGSUB |
| 0038 | DisableGSUBLookupsToExtend | extensionDisableGSUB |
| 0000 | NULL | extensionEnableGPOS |
| 0000 | NULL | extensionDisableGPOS |
| 0000 | NULL | extensionJstfMax |
|  | JstfPriority USEnglishSecondJstfPriority | JstfPriority table definition |
| 0000 | NULL | shrinkageEnableGSUB |
| 0000 | NULL | shrinkageDisableGSUB |
| 0000 | NULL | shrinkageEnableGPOS |
| 001C | DisableGPOSLookupsToShrink | shrinkageDisableGPOS |
| 0000 | NULL | shrinkageJstfMax |
| 0000 | NULL | extensionEnableGSUB |
| 0000 | NULL | extensionDisableGSUB |
| 002C | EnableGPOSLookupsToExtend | extensionEnableGPOS |
| 0000 | NULL | extensionDisableGPOS |
| 0000 | NULL | extensionJstfMax |
|  | JstfGSUBModList EnableGSUBLookupsToShrink | JstfGSUBModList table definition, enable three ligature substitution lookups |
| 0003 | 3 | lookupCount |
| 002E | 46 | gsubLookupIndices[0] |
| 0035 | 53 | gsubLookupIndices [1] |
| 0063 | 99 | gsubLookupIndices [2] |
|  | JstfGPOSModList DisableGPOSLookupsToShrink | JstfGPOSModList table definition, disable three tight kerning lookups |
| 0003 | 3 | lookupCount |
| 006C | 108 | gposLookupIndices[0] |
| 006E | 110 | gposLookupIndices [1] |
| 0070 | 112 | gposLookupIndices [2] |
|  | JstfGSUBModList DisableGSUBLookupsToExtend | JstfGSUBModList table definition, disable three ligature substitution lookups |
| 0003 | 3 | lookupCount |
| 002E | 46 | gsubLookupIndices [0] |
| 0035 | 53 | gsubLookupIndices [1] |
| 0063 | 99 | gsubLookupIndices [2] |
|  | JstfGPOSModList EnableGPOSLookupsToExtend | JstfGPOSModList table definition enable three tight kerning lookups |
| 0003 | 3 | lookupCount |
| 006C | 108 | gposLookupIndices [0] |
| 006E | 110 | gposLookupIndices [1] |
| 0070 | 112 | gposLookupIndices [2] |

Example 4: JstfMax table

The JstfMax table in Example 4 defines a lookup to expand the advance width of the word space glyph and extend line lengths. The lookup definition is identical to the SinglePos lookup type in the GPOS table although it is enabled only when justifying text. The ValueRecord in the WordSpaceExpand lookup subtable specifies an XAdvance adjustment of 360 units, which is the maximum value the font developer recommends for acceptable text rendering. The text-processing client may implement the lookup using any value between zero and the maximum.

Example 4

| Hex Data | Source | Comments |
| --- | --- | --- |
|  | JstfMax WordSpaceExpandMax | JstfMax table definition |
| 0001 | 1 | lookupCount |
| 0004 | WordSpaceExpandLookup | lookupOffsets[0] (offset to JSTF Lookup table) |
|  | Lookup WordSpaceExpandLookup | Jstf Lookup table definition |
| 0001 | 1 | lookupType: SinglePos Lookup |
| 0000 | 0x0000 | lookupFlag |
| 0001 | 1 | subTableCount |
| 0008 | WordSpaceExpandSubtable | subtableOffsets[0], SinglePos subtable |
|  | SinglePosFormat1 WordSpaceExpandSubtable | SinglePos subtable definition |
| 0001 | 1 | posFormat |
| 0008 | WordSpaceCoverage | Offset to Coverage table |
| 0004 | 0x0004 | valueFormat: XAdvance only |
| 0168 | 360 | value – XAdvance value in Jstf: this is a max value, expand word space from zero to this amount |
|  | CoverageFormat1 WordSpaceCoverage | Coverage table definition |
| 0001 | 1 | coverageFormat |
| 0001 | 1 | glyphCount |
| 0022 | WordSpaceGlyphID | glyphArray[0] |

### MATH—The mathematical typesetting table

#### MATH table overview

Mathematical formulas are complex text objects in which multiple elements with various metric, style or positioning attributes are combined. In order for a math layout engine to support layout of mathematical formulas, several types of font-specific information particular to the layout of formulas are required. The MATH table provides this font-specific information necessary for math formula layout.

Note that this is not a complete specification of math layout. The MATH table provides font data required for math layout, but detailed algorithms for use of the data are not specified. Different math-layout engine implementations can use this data to produce different layout results in accordance with different purposes or goals.

Layout of math formulas is quite different from regular text layout that is done using tables such as GSUB and GPOS. Regular text layout mainly deals with a line of text, often formatted with a single font. In this situation, actions such as contextual substitution or kerning can be done with access to the complete context of the line of text, and the rules can be expressed in terms of known glyph sequences. Math layout is quite different from this.

The general structure of math formulas is hierarchical, with formulas composed of smaller sub-formula expressions, where each expression may be composed of even simpler expressions, and so on down to individual strings – operator symbols, variable names and numbers.

Figure 6.38 – Example of a complex hierarchical math formula and the diagram showing the pieces of metal type and spacing materials used in a traditional printing application

Likewise, the process of math formula layout is also recursive. Child components are formatted first and then arranged to form their parent’s layout with this process repeated on every level starting from simplest blocks up to the whole formula. Each sub-formula has its own component structure and rules for how to perform layout. For example, a fraction expression consists of numerator and denominator sub-expressions, which will be placed on top one another with a fraction rule separating them. An integral expression contains an integral sign, optional upper- and lower- limit expressions, and a following sub-expression.

The simplest blocks within a formula typically contain strings. In isolation, each could be laid out by regular text-layout processing using other tables such as GSUB and GPOS. Math layout goes beyond this. It deals mainly with layout processing involved in composing these simple blocks together into a complex, hierarchical formula. The data provided in the MATH table allows math-layout processing to be typographically aware, so that presentation with high typographic quality can be achieved.

A math layout engine works with boxes representing individual formula components as units of layout. During the layout process, individual boxes can be arranged relative to each other- they can be stretched depending on the sizes of other boxes they interact with; or they can use different glyph variants, based on the box size or position in the formula. The MATH table provides data that informs how these operations are done. Each box has an associated font that supplies information comprising typographic requirements for that box. These may be requirements for that box alone, based on the results of layout operations internal to the box; or they may be requirements on how other boxes interact with that box in layout.

By providing information about a font generally or about specific glyphs in the font, the MATH table can enable math-layout engines to produce layout that is appropriate for the font design, and that realizes the font’s full typographic potential by using stylistic and stretching glyph variants provided by the font.

#### MATH – Table organization and structure

##### Shared Formats: MathValueRecord, Coverage

MathValueRecord

MATH subtables use math value records to describe values that place or adjust elements of math formulas. These values are expressed in font design units.

A MathValueRecord may also specify an offset to a device table which provides corrections to a given value at particular display resolutions. The device table format is described in subclause 6.2. The device table can provide multiple correction values to be used for several different PPEM sizes. Different formats for representing the correction values are provided to allow for efficient representation, according to the size requirements of the correction values. When used in MathValueRecords, format 1 is recommended.

When designing a MATH table, device tables may be specified for many values used for positioning elements of a formula. This creates the potential that many device corrections may accumulate in a given layout. However, such accumulation would result in metrics for formula elements that are significantly different from scaled-adjusted dimensions of the same elements rendered on a high-resolution device. It would produce inconsistencies between screen and print renditions, and could also lead to clipping. For these reasons, accumulation of many corrections is undesirable.

While a font may specify device corrections, use of these corrections is under the control of a given layout engine implementation. To maintain consistency across devices with different resolutions, an engine may limit the number of device corrections that are accumulated, or may ignore them altogether. A layout engine can also supply its own corrections where none are indicated in the font. Since the accumulated size of corrections should be kept to a minimum it is recommended that device tables referenced by a MathValueRecord use format 1 for representation of correction values. This format allows corrections of at most -2to +1 pixels. The recommended values to use in MathValueRecords are -1, 0, or 1.

MathValueRecord

| Type | Name | Description |
| --- | --- | --- |
| int16 | value | The X or Y value in design units |
| Offset16 | deviceTableOffset | Offset to the device table – from the beginning of parent table. May be NULL. Suggested format for device table is 1. |

Coverage tables

MATH subtables make use of Coverage tables, defined in subclause 6.2, to specify sets of glyphs. All Coverage table formats may be used in the MATH table.

##### MATH Header

The MATH table begins with a header that consists of a version number for the table (majorVersion/minorVersion), which is currently set to 1.0, and specified offsets to the following tables that store information on positioning of math formula elements:

- MathConstants table stores font parameters to be used in typesetting of each supported mathematical function, such as a fraction or a radical.

- MathGlyphInfo table supplies positioning information that is defined on a per-glyph basis.

- MathVariants table contains information to be used in constructing glyph variants of different height or width, either by finding a pre-defined glyph with desired measurements in the font, or by assembling the required shape from pieces found in the glyph set.

Math Header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version of the MATH table, = 1 |
| uint16 | minorVersion | Minor version of the MATH table, = 0 |
| Offset16 | mathConstantsOffset | Offset to MathConstants table, from the beginning of MATH table. |
| Offset16 | mathGlyphInfoOffset | Offset to MathGlyphInfo table, from the beginning of MATH table. |
| Offset16 | mathVariantsOffset | Offset to MathVariants table, from the beginning of MATH table. |

##### MathConstants Table

The MathConstants table defines a number of constants required to properly position elements of mathematical formulas. These constants belong to several groups of semantically-related values, such as values for positioning of accents, positioning of superscripts and subscripts, and positioning of elements of fractions. The table also contains general-use constants that may affect all parts of the formula, such as axis height and math leading. Note that most of the constants deal with aspects of vertical positioning.

In most cases, values in the MathConstants table are assumed to be positive. For example, for descenders and shift-down values a positive constant signifies movement in a downwards direction. Most values in the MathConstants table are represented by a MathValueRecord, which allows the font designer to supply device corrections to those values when necessary.

For values that pertain to layout interactions between a base and dependent elements (e.g. superscripts or limits), the specific value used is taken from the font associated with the base, and the size of the value is relative to the size of the base.

The following naming convention are used for fields in the MathConstants table:

- Height – Specifies a distance from the main baseline.

- Kern – Represents a fixed amount of empty space to be introduced.

- Gap – Represents an amount of empty space that may need to be increased to meet certain criteria.

- Drop and Rise – Specifies the relationship between measurements of two elements to be positioned relative to each other (but not necessarily in a stack-like manner) that must meet certain criteria. For a Drop, one of the positioned elements has to be moved down to satisfy those criteria; for a Rise, the movement is upwards.

- Shift – Defines a vertical shift applied to an element sitting on a baseline.

- Dist – Defines a distance between baselines of two elements.

MathConstants Table

| Type | Name | Description |
| --- | --- | --- |
| int16 | scriptPercentScaleDown | Percentage of scaling down for level 1 superscripts and subscripts. Suggested value: 80%. |
| int16 | scriptScriptPercentScaleDown | Percentage of scaling down for level 2 (scriptScript) superscripts and subscripts. Suggested value: 60%. |
| uint16 | delimitedSubFormulaMinHeight | Minimum height required for a delimited expression (contained within parentheses, etc.) to be treated as a sub-formula. Suggested value: normal line height × 1.5. |
| uint16 | displayOperatorMinHeight | Minimum height of n-ary operators (such as integral and summation) for formulas in display mode (that is, appearing as standalone page elements, not embedded inline within text). |
| MathValueRecord | mathLeading | White space to be left between math formulas to ensure proper line spacing. For example, for applications that treat line gap as a part of line ascender, formulas with ink going above (os2.sTypoAscender + os2.sTypoLineGap – MathLeading) or with ink going below os2.sTypoDescender will result in increasing line height. |
| MathValueRecord | axisHeight | Axis height of the font. In math typesetting, the term axis refers to a horizontal reference line used for positioning elements in a formula. The math axis is similar to but distinct from the baseline for regular text layout. For example, in a simple equation, a minus symbol or fraction rule would be on the axis, but a string for a variable name would be set on a baseline that is offset from the axis. The axisHeight value determines the amount of that offset. |
| MathValueRecord | accentBaseHeight | Maximum (ink) height of accent base that does not require raising the accents. Suggested: x-height of the font (os2.sxHeight) plus any possible overshots. |
| MathValueRecord | flattenedAccentBaseHeight | Maximum (ink) height of accent base that does not require flattening the accents. Suggested: cap height of the font (os2.sCapHeight). |
| MathValueRecord | subscriptShiftDown | The standard shift down applied to subscript elements. Positive for moving in the downward direction. Suggested: os2.ySubscriptYOffset. |
| MathValueRecord | subscriptTopMax | Maximum allowed height of the (ink) top of subscripts that does not require moving subscripts further down. Suggested: 4/5 x- height. |
| MathValueRecord | subscriptBaselineDropMin | Minimum allowed drop of the baseline of subscripts relative to the (ink) bottom of the base. Checked for bases that are treated as a box or extended shape. Positive for subscript baseline dropped below the base bottom. |
| MathValueRecord | superscriptShiftUp | Standard shift up applied to superscript elements. Suggested: os2.ySuperscriptYOffset. |
| MathValueRecord | superscriptShiftUpCramped | Standard shift of superscripts relative to the base, in cramped style. |
| MathValueRecord | superscriptBottomMin | Minimum allowed height of the (ink) bottom of superscripts that does not require moving subscripts further up. Suggested: ¼ x-height. |
| MathValueRecord | superscriptBaselineDropMax | Maximum allowed drop of the baseline of superscripts relative to the (ink) top of the base. Checked for bases that are treated as a box or extended shape. Positive for superscript baseline below the base top. |
| MathValueRecord | subSuperscriptGapMin | Minimum gap between the superscript and subscript ink. Suggested: 4 × default rule thickness. |
| MathValueRecord | superscriptBottomMaxWith Subscript | The maximum level to which the (ink) bottom of superscript can be pushed to increase the gap between superscript and subscript, before subscript starts being moved down. Suggested: 4/5 x-height. |
| MathValueRecord | spaceAfterScript | Extra white space to be added after each subscript and superscript. Suggested: 0.5 pt for a 12 pt font. |
| MathValueRecord | upperLimitGapMin | Minimum gap between the (ink) bottom of the upper limit, and the (ink) top of the base operator. |
| MathValueRecord | upperLimitBaselineRiseMin | Minimum distance between baseline of upper limit and (ink) top of the base operator. |
| MathValueRecord | lowerLimitGapMin | Minimum gap between (ink) top of the lower limit, and (ink) bottom of the base operator. |
| MathValueRecord | lowerLimitBaselineDropMin | Minimum distance between baseline of the lower limit and (ink) bottom of the base operator. |
| MathValueRecord | stackTopShiftUp | Standard shift up applied to the top element of a stack. |
| MathValueRecord | stackTopDisplayStyleShiftUp | Standard shift up applied to the top element of a stack in display style. |
| MathValueRecord | stackBottomShiftDown | Standard shift down applied to the bottom element of a stack. Positive for moving in the downward direction. |
| MathValueRecord | stackBottomDisplayStyleShiftDown | Standard shift down applied to the bottom element of a stack in display style. Positive for moving in the downward direction. |
| MathValueRecord | stackGapMin | Minimum gap between (ink) bottom of the top element of a stack, and the (ink) top of the bottom element. Suggested: 3 × default rule thickness. |
| MathValueRecord | stackDisplayStyleGapMin | Minimum gap between (ink) bottom of the top element of a stack, and the (ink) top of the bottom element in display style. Suggested: 7 × default rule thickness. |
| MathValueRecord | stretchStackTopShiftUp | Standard shift up applied to the top element of the stretch stack. |
| MathValueRecord | stretchStackBottomShiftDown | Standard shift down applied to the bottom element of the stretch stack. Positive for moving in the downward direction. |
| MathValueRecord | stretchStackGapAboveMin | Minimum gap between the ink of the stretched element, and the (ink) bottom of the element above. Suggested: same value as upperLimitGapMin. |
| MathValueRecord | stretchStackGapBelowMin | Minimum gap between the ink of the stretched element, and the (ink) top of the element below. Suggested: same value as lowerLimitGapMin. |
| MathValueRecord | fractionNumeratorShiftUp | Standard shift up applied to the numerator. |
| MathValueRecord | fractionNumeratorDisplayStyle ShiftUp | Standard shift up applied to the numerator in display style. Suggested: same value as stackTopDisplayStyleShiftUp. |
| MathValueRecord | fractionDenominatorShiftDown | Standard shift down applied to the denominator. Positive for moving in the downward direction. |
| MathValueRecord | fractionDenominatorDisplay StyleShiftDown | Standard shift down applied to the denominator in display style. Positive for moving in the downward direction. Suggested: same value as stackBottomDisplayStyleShiftDown. |
| MathValueRecord | fractionNumeratorGapMin | Minimum tolerated gap between the (ink) bottom of the numerator and the ink of the fraction bar. Suggested: default rule thickness. |
| MathValueRecord | fractionNumDisplayStyle GapMin | Minimum tolerated gap between the (ink) bottom of the numerator and the ink of the fraction bar in display style. Suggested: 3 × default rule thickness. |
| MathValueRecord | fractionRuleThickness | Thickness of the fraction bar. Suggested: default rule thickness. |
| MathValueRecord | fractionDenominatorGapMin | Minimum tolerated gap between the (ink) top of the denominator and the ink of the fraction bar. Suggested: default rule thickness. |
| MathValueRecord | fractionDenomDisplayStyleGap Min | Minimum tolerated gap between the (ink) top of the denominator and the ink of the fraction bar in display style. Suggested: 3 × default rule thickness. |
| MathValueRecord | skewedFractionHorizontalGap | Horizontal distance between the top and bottom elements of a skewed fraction. |
| MathValueRecord | skewedFractionVerticalGap | Vertical distance between the ink of the top and bottom elements of a skewed fraction. |
| MathValueRecord | overbarVerticalGap | Distance between the overbar and the (ink) top of the base. Suggested: 3 × default rule thickness. |
| MathValueRecord | overbarRuleThickness | Thickness of overbar. Suggested: default rule thickness. |
| MathValueRecord | overbarExtraAscender | Extra white space reserved above the overbar. Suggested: default rule thickness. |
| MathValueRecord | underbarVerticalGap | Distance between underbar and (ink) bottom of the base. Suggested: 3 × default rule thickness. |
| MathValueRecord | underbarRuleThickness | Thickness of underbar. Suggested: default rule thickness. |
| MathValueRecord | underbarExtraDescender | Extra white space reserved below the underbar. Always positive. Suggested: default rule thickness. |
| MathValueRecord | radicalVerticalGap | Space between the (ink) top of the expression and the bar over it. Suggested: 1¼ default rule thickness. |
| MathValueRecord | radicalDisplayStyleVerticalGap | Space between the (ink) top of the expression and the bar over it. Suggested: default rule thickness + ¼ x-height. |
| MathValueRecord | radicalRuleThickness | Thickness of the radical rule. This is the thickness of the rule in designed or constructed radical signs. Suggested: default rule thickness. |
| MathValueRecord | radicalExtraAscender | Extra white space reserved above the radical. Suggested: same value as radicalRuleThickness. |
| MathValueRecord | radicalKernBeforeDegree | Extra horizontal kern before the degree of a radical, if such is present. |
| MathValueRecord | radicalKernAfterDegree | Negative kern after the degree of a radical, if such is present. Suggested: −10/18 of em. |
| int16 | radicalDegreeBottomRaise Percent | Height of the bottom of the radical degree, if such is present, in proportion to the ascender of the radical sign. Suggested: 60%. |

##### MathGlyphInfo Table

The MathGlyphInfo table contains positioning information that is defined on per-glyph basis. The table consists of the following parts:

- A MathItalicsCorrectionInfo table that contains information on italics correction values.

- A MathTopAccentAttachment table that contains horizontal positions for attaching mathematical accents.

- An Extended Shape coverage table. The glyphs covered by this table are to be considered extended shapes.

- A MathKernInfo table that provides per-glyph information for mathematical kerning.

MathGlyphInfo Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | mathItalicsCorrectionInfoOffset | Offset to MathItalicsCorrectionInfo table, from the beginning of the MathGlyphInfo table. |
| Offset16 | mathTopAccentAttachmentOffset | Offset to MathTopAccentAttachment table, from the beginning of the MathGlyphInfo table. |
| Offset16 | extendedShapeCoverageOffset | Offset to ExtendedShapes coverage table, from the beginning of the MathGlyphInfo table. When the glyph to the left or right of a box is an extended shape variant, the (ink) box should be used for vertical positioning purposes, not the default position defined by values in MathConstants table. May be NULL. |
| Offset16 | mathKernInfoOffset | Offset to MathKernInfo table, from the beginning of MathGlyphInfo table. |

##### MathItalicsCorrectonInfo Table

The MathItalicsCorrectionInfo table contains italics correction values for slanted glyphs used in math layout. The top portion of slanted glyphs may protrude beyond the glyph’s advance width. This can result in collision with other interacting elements, or an appearance in the placement of other interacting elements that is unpleasing unless some accommodation is made for the protrusion. The MathItalicsCorrectionInfo table provides correction values to accommodate for such protrusion.

The table consists of the following parts:

- Coverage of glyphs for which the italics correction values are provided. It is assumed to be zero for all other glyphs.

- Count of covered glyphs.

- Array of italic correction values for each covered glyph, in order of coverage. The italics correction value can be used as an adjustment for positioning of interacting elements to make allowance for protrusion to the right of the top part of the glyph. For example, taller letters tend to have larger italics correction, and a V will probably have larger italics correction than an L.

Italics correction can be used in the following situations:

  - When a run of slanted characters is followed by a straight character (such as an operator or a delimiter), the italics correction of the last glyph is added to its advance width.

  - When positioning limits on an N-ary operator (e.g., integral sign), the horizontal position of the upper limit is moved to the right by ½ of the italics correction, while the position of the lower limit is moved to the left by the same distance.

  - When positioning superscripts and subscripts, their default horizontal positions are also different by the amount of the italics correction of the preceding glyph.

MathItalicsCorrectionInfo Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | italicCorrectionCoverageOffset | Offset to Coverage table, from the beginning of MathItalicsCorrectionInfo table. |
| uint16 | italicsCorrectionCount | Number of italics correction values. Should coincide with the number of covered glyphs. |
| MathValueRecord | italicsCorrection [italicsCorrectionCount] | Array of MathValueRecords defining italics correction values for each covered glyph. |

##### MathTopAccentAttachment Table

The MathTopAccentAttachment table contains information on horizontal positioning of top math accents. The table consists of the following parts:

- Coverage of glyphs for which information on horizontal positioning of math accents is provided. To position accents over any other glyph, its geometrical center (with respect to advance width) can be used.

- Count of covered glyphs.

- Array of top accent attachment points for each covered glyph, in order of coverage. These attachment points are to be used for finding horizontal positions of accents over characters. It is done by aligning the attachment point of the base glyph with the attachment point of the accent. Note that this is very similar to mark-to-base attachment, but here alignment only happens in the horizontal direction, and the vertical positions of accents are determined by different means.

MathTopAccentAttachment Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | topAccentCoverageOffset | Offset to Coverage table, from the beginning of MathTopAccentAttachment table. |
| uint16 | topAccentAttachmentCount | Number of top accent attachment point values. Must be the same as the number of glyph IDs referenced in the Coverage table. |
| MathValueRecord | topAccentAttachment [topAccentAttachmentCount] | Array of MathValueRecords defining top accent attachment points for each covered glyph. |

##### ExtendedShapeCoverage Table

The glyphs covered by this table are to be considered extended shapes. These glyphs are variants extended in the vertical direction, e.g., to match height of another part of the formula. Because their dimensions may be very large in comparison with normal glyphs in the glyph set, the standard positioning algorithms will not produce the best results when applied to them. In the vertical direction, other formula elements will be positioned not relative to those glyphs, but instead to the ink box of the subexpression containing them.

For example, consider a fraction enclosed in parentheses with a superscript. Notice how the superscripts on ‘z’ and ‘Z’ are aligned vertically, although they have different heights. If the right parenthesis was not considered an extended shape, the superscript would be put in position aligned with any other superscript on the line, like this:

Because this is undesirable, the right parenthesis in this case should be considered an extended shape, resulting in superscript positioned relative to the whole subexpression:

##### MathKernInfo Table

The MathKernInfo table provides mathematical kerning values used for kerning of subscript and superscript glyphs relative to a base glyph. Its purpose is to improve spacing in situations such as or .

Mathematical kerning is height dependent; that is, different kerning amounts can be specified for different heights within a glyph's vertical extent. For any given glyph, different values can be specified for four corner positions — top-right, to-left, etc. — allowing for different kerning adjustments according to whether the glyph occurs as a subscript, a superscript, a base being kerned with a subscript, or a base being kerned with a superscript.

The kerning algorithm for subscripts and superscripts is as follows:

- Calculate the vertical positions of subscripts and superscripts using the MathConstants table.

- Set the default horizontal position for the subscript immediately after the base glyph.

- Set the default horizontal position for a superscript after the base, applying a shift for italics correction if indicated for the base glyph in the MathItalicsCorrectionInfo table.

- Calculate a superscript kerning value as follows:

- Evaluate two correction heights (illustrated in the figure below):

- At the bottom of the superscript-glyph bounding box. (The corresponding height for the base glyph is the distance from the base-glyph baseline to the bottom of the superscript bounding box.)

- At the top of the base-glyph bounding box. (The corresponding height for the superscript glyph is the distance from the superscript baseline to the top of the base-glyph bounding box.)

- For each correction height, add the top-right kerning value for the base glyph to the bottom-left kerning value for the superscript glyph.

- Take the minimum of these two sums: kern the base and superscript by that amount.

- Calculate a subscript kerning value as follows:

- Evaluate two correction heights:

- At the top of the subscript-glyph bounding box. (The corresponding height for the base glyph is the distance from the base-glyph baseline to the top of the subscript bounding box.)

- At the bottom of the base-glyph bounding box. (The corresponding height for the subscript glyph is the distance from the subscript baseline to the bottom of the base-glyph bounding box.)

- For each correction height, add the bottom-right kerning value for the base glyph to the top-left kerning value for the subscript glyph.

- Take the minimum of these two sums: kern the base and subscript by that amount.

NOTE 	If a base expression, subscript expression or superscript expression is a box, a math-layout engine may use kerning values of zero for each corner of the box, or may calculate height-dependent kerning amounts by some means

The following figure illustrates the correction heights for a base and superscript:

Correction Height for Superscript

Italic Correction

Correction Height for Base

Base Character Baseline

Superscript Baseline

Fig. 6.39 Example of horizontal and vertical kerning adjustments for superscript positioning

The MathKernInfo table consists of the following fields:

- A Coverage table of glyphs for which mathematical kerning information is provided. Mathematical kerning amounts are assumed to be zero for all other glyphs.

- Count of MathKernInfoRecords.

- Array of MathKernInfoRecords for each covered glyph, the order of glyphs in the Coverage table.

MathKernInfo Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | mathKernCoverageOffset | Offset to Coverage table, from the beginning of the MathKernInfo table. |
| uint16 | mathKernCount | Number of mathKernInfoRecords. Must be the same as the number of glyph IDs referenced in the Coverage table. |
| MathKernInfoRecord | mathKernInfoRecords [mathKernCount] | Array of MathKernInfoRecords, one for each covered glyph. |

MathKernInfoRecord

Each MathKernInfoRecord points to up to four kern tables for each of the corners around the glyph. If no kern table is provided for a corner, a kerning amount of zero is assumed.

MathKernInfoRecord

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | topRightMathKernOffset | Offset to MathKern table for top right corner, from the beginning of MathKernInfo table. May be NULL. |
| Offset16 | topLeftMathKernOffset | Offset to MathKern table for the top left corner, from the beginning of MathKernInfo table. May be NULL. |
| Offset16 | bottomRightMathKernOffset | Offset to MathKern table for bottom right corner, from the beginning of MathKernInfo table. May be NULL. |
| Offset16 | bottomLeftMathKernOffset | Offset to MathKern table for bottom left corner, from the beginning of MathKernInfo table. May be NULL. |

##### MathKern Table

The MathKern table provides kerning amounts for different heights in a glyph's vertical extent. An array of kerning values is provided, each of which applies to a height range. A corresponding array of heights indicate the transition points between consecutive ranges.

Correction heights for each glyph are relative to the glyph baseline, with positive height values above the baseline, and negative height values below the baseline. The correctionHeights array is sorted in increasing order, from lowest to highest.

The kerning value corresponding to a particular height is determined by finding two consecutive entries in the correctionHeight array such that the given height is greater than or equal to the first entry and less than the second entry. The index of the second entry is used to look up a kerning value in the kernValues array. If the given height is less than the first entry in the correctionHeights array, the first kerning value (index 0) is used. For a height that is greater than or equal to the last entry in the correctionHeights array, the last entry is used.

MathKern Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | heightCount | Number of heights at which the kern value changes. |
| MathValueRecord | correctionHeight[heightCount] | Array of correction heights, in design units, sorted from lowest to highest. |
| MathValueRecord | kernValue[heightCount+1] | Array of kern values for height ranges. Negative values are used to move glyphs closer to each other. |

##### MathVariants Table

The MathVariants table solves the following problem: given a particular default glyph shape and a certain width or height, find a variant shape glyph (or construct created by putting several glyph together) that has the required measurement. This functionality is needed for growing the parentheses to match the height of the expression within, growing the radical sign to match the height of the expression under the radical, stretching accents like tilde when they are put over several characters, for stretching arrows, horizontal curly braces, and so forth.

The MathVariants table consists of the following fields:

- Count and coverage of glyphs that can grow in the vertical direction.

- Count and coverage of glyphs that can grow in the horizontal direction.

- The minConnectorOverlap defines by how much two glyphs need to overlap with each other when used to construct a larger shape. Each glyph to be used as a building block in constructing extended shapes will have a straight part at either or both ends. This connector part is used to connect that glyph to other glyphs in the assembly. These connectors need to overlap to compensate for rounding errors and hinting corrections at a lower resolution. The minConnectorOverlap value tells how much overlap is necessary for this particular font.

- Two arrays of offsets to MathGlyphConstruction tables: one array for glyphs that grow in the vertical direction, and the other array for glyphs that grow in the horizontal direction. The arrays must be arranged in coverage order and have specified sizes.

MathVariants Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | minConnectorOverlap | Minimum overlap of connecting glyphs during glyph construction, in design units. |
| Offset16 | vertGlyphCoverageOffset | Offset to Coverage table, from the beginning of MathVariants table. |
| Offset16 | horizGlyphCoverageOffset | Offset to Coverage table, from the beginning of MathVariants table. |
| uint16 | vertGlyphCount | Number of glyphs for which information is provided for vertically growing variants. Must be the same as the number of glyph IDs referenced in the vertical Coverage table. |
| uint16 | horizGlyphCount | Number of glyphs for which information is provided for horizontally growing variants. Must be the same as the number of glyph IDs referenced in the horizontal Coverage table. |
| Offset16 | vertGlyphConstructionOffsets [vertGlyphCount] | Array of offsets to MathGlyphConstruction tables, from the beginning of the MathVariants table, for shapes growing in the vertical direction. |
| Offset16 | horizGlyphConstructionOffsets [horizGlyphCount] | Array of offsets to MathGlyphConstruction tables, from the beginning of the MathVariants table, for shapes growing in the horizontal direction. |

##### MathGlyphConstruction Table

The MathGlyphConstruction table provides information on finding or assembling extended variants for one particular glyph. It can be used for shapes that grow in either horizontal or vertical directions.

The first entry is the offset to the GlyphAssembly table that specifies how the shape for this glyph can be assembled from parts found in the glyph set of the font. If no such assembly exists, this offset will be set to NULL.

The MathGlyphConstruction table also contains the count and array of ready-made glyph variants for the specified glyph. Each variant consists of the glyph index and this glyph’s measurement in the direction of extension (vertical or horizontal).

Note that it is quite possible that both the GlyphAssembly table and some variants are defined for a particular glyph. For example, the font may provide several variants of curly braces with different sizes, and also a general mechanism for constructing larger versions of curly braces by stacking parts found in the glyph set. First, an attempt is made to find glyph among provided variants. If the required size is larger than any of the glyph variants provided, however, then the general mechanism can be employed to typeset the curly braces as a glyph assembly.

MathGlyphConstruction Table

| Type | Name | Description |
| --- | --- | --- |
| Offset16 | glyphAssemblyOffset | Offset to the GlyphAssembly table for this shape, from the beginning of the MathGlyphConstruction table. May be NULL. |
| uint16 | variantCount | Count of glyph growing variants for this glyph. |
| MathGlyphVariantRecord | mathGlyphVariantRecords [variantCount] | MathGlyphVariantRecords for alternative variants of the glyphs. |

MathGlyphVariantRecord

| Type | Name | Description |
| --- | --- | --- |
| uint16 | variantGlyph | Glyph ID for the variant. |
| uint16 | advanceMeasurement | Advance width/height, in design units, of the variant, in the direction of requested glyph extension. |

##### GlyphAssembly Table

The GlyphAssembly table specifies how the shape for a particular glyph can be constructed from parts found in the glyph set. The table defines the italics correction of the resulting assembly, and a number of parts that have to be put together to form the required shape. Some glyph parts can be designated as extenders, which can be repeated as needed to obtain a target size.

GlyphAssembly Table

| Type | Name | Description |
| --- | --- | --- |
| MathValueRecord | italicsCorrection | Italics correction of this GlyphAssembly. Should not depend on the assembly size. |
| uint16 | partCount | Number of parts in this assembly. |
| GlyphPartRecord | partRecords[partCount] | Array of GlyphPartRecords, from left to right (for assemblies that extend horizontally) or bottom to top (for assemblies that extend vertically). |

The result of the assembly process is an array of glyphs with an offset specified for each of those glyphs. When drawn consecutively at those offsets, the glyphs should combine correctly and produce the required shape.

The offsets in the direction of growth (advance offsets), as well as the number of extender parts, are determined based on the size requirement for the resulting assembly.

Note that the glyphs comprising the assembly should be designed so that they align properly in the direction that is orthogonal to the direction of growth.

Thus, a GlyphPartRecord consists of the following fields:

- Glyph ID for the part.

- Lengths of the connectors on each end of the glyph. The connectors are straight parts of the glyph that can be used to link it with the next or previous part. The connectors of neighboring parts can overlap by varying amounts, providing flexibility in how these glyphs can be put together. However, the overlap should not be less than the minConnectorOverlap value defined in the MathVariants tables, and it should not exceed the specified connector length for that end of the glyph. If the part does not have a connector on one of its ends, the corresponding connector length should be set to zero.

- The full advance of the part. It is also used to determine the measurement of the result by using the following formula:

Size of Assembly = Offset of the Last Part + Full Advance of the Last Part

- PartFlags is the last field. It identifies certain parts as extenders: those parts that can be repeated (that is, multiple instances of them can be used in place of one) or skipped altogether. Usually the extenders are vertical or horizontal bars of the appropriate thickness, aligned with the rest of the assembly.

To ensure that the width/height is distributed equally and the symmetry of the shape is preserved, following steps can be used by the math-layout engine.

- Assemble all parts with all extenders removed and with connections overlapping by the maximum amount. This gives the smallest possible result.

- Determine how much extra width/height can be obtained from all existing connections between neighboring parts by using minimal overlaps. If that is enough to achieve the size goal, extend each connection equally by changing overlaps of connectors to finish the job.

- If all connections have been extended to the minimum overlap and further growth is needed, add one of each extender, and repeat the process from the first step.

Note that for assemblies growing in the vertical direction, the distribution of height between ascent and descent is not defined. The math-layout engine is responsible for positioning the resulting assembly relative to the baseline.

GlyphPartRecord Table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | glyphID | Glyph ID for the part. |
| uint16 | startConnectorLength | Advance width / height, in design units, of the straight bar connector material at the start of the glyph in the direction of the extension (the left end for horizontal extension, the bottom end for vertical extension). |
| uint16 | endConnectorLength | Advance width / height, in design units, of the straight bar connector material at the end of the glyph in the direction of the extension (the right end for horizontal extension, the top end for vertical extension). |
| uint16 | fullAdvance | Full advance width/height for this part, in the direction of the extension, in design units. |
| uint16 | partFlags | Part qualifiers. PartFlags enumeration currently uses only one bit: 0x0001 fExtender If set, the part can be skipped or repeated. 0xFFFE Reserved. |

#### OFF layout tags used with the MATH Table

The following OFF layout tags can be used by math-layout engine to access a particular set of glyph variants. For detailed descriptions of the feature tags see subclause 6.4.3.2.

OFF layout tags for math processing

| Tag | Description |
| --- | --- |
| math | Script tag to be used for features in math layout. The only language system supported with this tag is the default language system. |
| ssty | Script Style This feature provides glyph variants adjusted to be more suitable for use in subscripts and superscripts. These script style forms should not be scaled or moved in the font; scaling and moving them is done by the math-layout engine. Instead, the ssty feature should provide glyph forms that result in shapes that look good as superscripts and subscripts when scaled and positioned by the math engine. When designing the script forms, the font developer may assume that the scriptPercentScaleDown and scriptScriptPercentScaleDown values in the MathConstants table will be scaling factors applied to the size of the alternate glyphs by the math engine. This feature can have a parameter indicating the script level: 1 for simple subscripts and superscripts, 2 for second level subscripts and superscripts (that is, scripts on scripts), and so on. (Currently, only the first two alternates are used). For glyphs that are not covered by this feature, the original glyph is used in subscripts and superscripts. Recommended format: Alternate Substitution table (Single Substitution if there are no second level forms). There should be no context. |
| flac | Flattened Accents over Capitals This feature provides flattened forms of accents to be used over high-rise bases such as capitals. This feature should only change the shape of the accent and should not move it in the vertical or horizontal direction. Moving of the accents is done by the math-layout engine. Accents are flattened by the math engine if their base is higher than theflattenedAccentBaseHeight value in the MathConstants table. Recommended format: Single Substitution table. There should be no context. |
| dtls | Dotless Forms This feature provides dotless forms for Math Alphanumeric characters, such as U+1D422 MATHEMATICAL BOLD SMALL I, U+1D423 MATHEMATICAL BOLD SMALL J, U+1D456 U+MATHEMATICAL ITALIC SMALL I, U+1D457 MATHEMATICAL ITALIC SMALL J, and so on. The dotless forms are to be used as base forms for placing mathematical accents over them. Recommended format: Single Substitution table. There should be no context. |

## Layout tag registry

OFF Layout tags are 4-byte character strings that identify the scripts, language systems, features and baselines in an OFF font. The registry establishes conventions for naming and using these tags. Registered tags have a specific meaning and convey precise information to developers and text-processing clients of OFF Layout. Font developers are encouraged to use registered tags to assure compatibility and ease of use across fonts, applications, and operating systems. Additional tags can be added to the tag registry when necessary.

### Scripts tags

Script tags generally correspond to a Unicode script. However, the associations between them may not always be one-to-one, and the OFF tags are not guaranteed to be the same as Unicode Script property-value aliases or ISO 15924 script IDs. Since the development of OFF (and the prior history of OpenType) script tags predates the ISO 15924 or Unicode Script property, the rules for script tags defined in this document may not always be the same as rules for ISO 15924 script IDs. The OFF script tags can also correlate with a particular OFF layout implementation, with the result that more than one script tag may be registered for a given Unicode script (e.g. ‘deva’ and ‘dev2’).

All tags are 4-byte character strings composed of a limited set of ASCII characters in the 0x20-0x7E range. A script tag can consist of four or fewer lowercase letters. If a script tag consists less than four lowercase letters, the letters are followed by the requisite number of spaces (0x20), each consisting of a single byte.

Use and processing of script tags in Script Records is described in the "Script list table and Script record" section of subclause 6.2.4 (Scripts and Languages).

| Script | Script Tag |
| --- | --- |
| Adlam | 'adlm' |
| Ahom | 'ahom' |
| Anatolian Hieroglyphs | 'hluw' |
| Arabic | 'arab' |
| Armenian | 'armn' |
| Avestan | 'avst' |
| Balinese | 'bali' |
| Bamum | 'bamu' |
| Bassa Vah | 'bass' |
| Batak | 'batk' |
| Bengali | 'beng' |
| Bengali v.2 | 'bng2' |
| Bhaiksuki | 'bhks' |
| Bopomofo | 'bopo' |
| Brahmi | 'brah' |
| Braille | 'brai' |
| Buginese | 'bugi' |
| Buhid | 'buhd' |
| Byzantine Music | 'byzm' |
| Canadian Syllabics | 'cans' |
| Carian | 'cari' |
| Caucasian Albanian | 'aghb' |
| Chakma | 'cakm' |
| Cham | 'cham' |
| Cherokee | 'cher' |
| Chorasmian | 'chrs' |
| CJK Ideographic | 'hani' |
| Coptic | 'copt' |
| Cypriot Syllabary | 'cprt' |
| Cypro-Minoan | ‘cpmn’ |
| Cyrillic | 'cyrl' |
| Default | 'DFLT' |
| Deseret | 'dsrt' |
| Devanagari | 'deva' |
| Devanagari v.2 | 'dev2' |
| Dives Akuru | 'diak' |
| Dogra | 'dogr' |
| Duployan | 'dupl' |
| Egyptian hieroglyphs | 'egyp' |
| Elbasan | 'elba' |
| Elymaic | 'elym' |
| Ethiopic | 'ethi' |
| Georgian | 'geor' |
| Glagolitic | 'glag' |
| Gothic | 'goth' |
| Grantha | 'gran' |
| Greek | 'grek' |
| Gujarati | 'gujr' |
| Gujarati v.2 | 'gjr2' |
| Gunjala Gondi | 'gong' |
| Gurmukhi | 'guru' |
| Gurmukhi v.2 | 'gur2' |
| Hangul | 'hang' |
| Hangul Jamo | 'jamo' |
| Hanifi Rohingya | 'rohg' |
| Hanunoo | 'hano' |
| Hatran | 'hatr' |
| Hebrew | 'hebr' |
| Hiragana | 'kana' |
| Imperial Aramaic | 'armi' |
| Inscriptional Pahlavi | 'phli' |
| Inscriptional Parthian | 'prti' |
| Javanese | 'java' |
| Kaithi | 'kthi' |
| Kannada | 'knda' |
| Kannada v.2 | 'knd2' |
| Katakana | 'kana' |
| Kayah Li | 'kali' |
| Kharosthi | 'khar' |
| Khitan Small Script | 'kits' |
| Khmer | 'khmr' |
| Khojki | 'khoj' |
| Khudawadi | 'sind' |
| Lao | 'lao ' |
| Latin | 'latn' |
| Lepcha | 'lepc' |
| Limbu | 'limb' |
| Linear A | 'lina' |
| Linear B | 'linb' |
| Lisu (Fraser) | 'lisu' |
| Lycian | 'lyci' |
| Lydian | 'lydi' |
| Mahajani | 'mahj' |
| Makasar | 'maka' |
| Malayalam | 'mlym' |
| Malayalam v.2 | 'mlm2' |
| Mandaic, Mandaean | 'mand' |
| Manichaean | 'mani' |
| Masaram Gondi | 'gonm' |
| Marchen | 'marc' |
| Mathematical Alphanumeric Symbols | 'math' |
| Medefaidrin (Oberi Okaime, Oberi Ɔkaimɛ) | 'medf' |
| Meitei Mayek (Meithei, Meetei) | 'mtei' |
| Mende Kikakui | 'mend' |
| Meroitic Cursive | 'merc' |
| Meroitic Hieroglyphs | 'mero' |
| Miao | 'plrd' |
| Modi | 'modi' |
| Mongolian | 'mong' |
| Mro | 'mroo' |
| Multani | 'mult' |
| Musical Symbols | 'musc' |
| Myanmar | 'mymr' |
| Myanmar v.2 | 'mym2' |
| Nabataean | 'nbat' |
| Nandinagari | 'nand' |
| Newa | 'newa' |
| New Tai Lue | 'talu' |
| Nyiakeng Puachue Hmong | 'hmnp' |
| N'Ko | 'nko ' |
| Nüshu | 'nshu' |
| Odia (formerly Oriya) | 'orya' |
| Odia (formerly Oriya) v.2 | 'ory2' |
| Ogham | 'ogam' |
| Ol Chiki | 'olck' |
| Old Italic | 'ital' |
| Old Hungarian | 'hung' |
| Old North Arabian | 'narb' |
| Old Permic | 'perm' |
| Old Persian Cuneiform | 'xpeo' |
| Old Sogdian | 'sogo' |
| Old South Arabian | 'sarb' |
| Old Turkic, Orkhon Runic | 'orkh' |
| Old Uyghur | ‘ougr’ |
| Osage | 'osge' |
| Osmanya | 'osma' |
| Pahawh Hmong | 'hmng' |
| Palmyrene | 'palm' |
| Pau Cin Hau | 'pauc' |
| Phags-pa | 'phag' |
| Phoenician | 'phnx' |
| Psalter Pahlavi | 'phlp' |
| Rejang | 'rjng' |
| Runic | 'runr' |
| Samaritan | 'samr' |
| Saurashtra | 'saur' |
| Sharada | 'shrd' |
| Shavian | 'shaw' |
| Siddham | 'sidd' |
| Sign Writing | 'sgnw' |
| Sinhala | 'sinh' |
| Sogdian | 'sogd' |
| Sora Sompeng | 'sora' |
| Soyombo | 'soyo' |
| Sumero-Akkadian Cuneiform | 'xsux' |
| Sundanese | 'sund' |
| Syloti Nagri | 'sylo' |
| Syriac | 'syrc' |
| Tagalog | 'tglg' |
| Tagbanwa | 'tagb' |
| Tai Le | 'tale' |
| Tai Tham (Lanna) | 'lana' |
| Tai Viet | 'tavt' |
| Takri | 'takr' |
| Tamil | 'taml' |
| Tamil v.2 | 'tml2' |
| Tangsa | ‘tnsa’ |
| Tangut | 'tang' |
| Telugu | 'telu' |
| Telugu v.2 | 'tel2' |
| Thaana | 'thaa' |
| Thai | 'thai' |
| Tibetan | 'tibt' |
| Tifinagh | 'tfng' |
| Tirhuta | 'tirh' |
| Toto | ‘toto’ |
| Ugaritic Cuneiform | 'ugar' |
| Vai | 'vai ' |
| Vithkuqi | ‘vith’ |
| Wancho | 'wcho' |
| Warang Citi | 'wara' |
| Yezidi | 'yezi' |
| Yi | 'yi ' |
| Zanabazar Square (Zanabazarin Dörböljin Useg, Xewtee Dörböljin Bicig, Horizontal Square Script) | 'zanb' |

### Language tags

Language system tags identify the language systems supported in an OFF Layout font data. What is meant by a "language system" in this context is a set of typographic conventions for how text in a given script should be presented. Such conventions may be associated with particular languages, with particular genres of usage, with different publications, and other such factors. For example, particular glyph variants for certain characters may be required for particular languages, or for phonetic transcription or mathematical notation.

In principle, a given set of conventions may be shared across multiple scenarios. For instance, two different languages (perhaps unrelated) may happen to follow the same conventions. Language system tags can be registered on a perceived-need basis, however; as a result, there is no guarantee that each tag represents a distinct and unique set of conventions. Tags can, however, be registered with the intent of representing conventions that apply to multiple languages. In such cases, the documented description for the tag should reflect that intent.

It should also be noted that there may be more than one set of typographic conventions that apply to a given language.

Therefore, in several respects, language system tags do not correspond in a one-to-one manner with languages. Even so, many registered tags are intended to represent typographic conventions for a particular language. For cases in which a correlation exists between a tag and one or more languages, the language identities are documented here by reference to ISO 639-2 and ISO 639-3.

If information is available to an application declaring the language of text content, then the application may make use of that to select a default language system tag to be applied when displaying that text. It is preferable, however, to give users control over the choice of language system tag to be used. (Depending on the application scenario, such control may be given to content authors, to content readers, or to both.)

NOTE	ISO 639-2 provides identifiers for individual languages as well as for certain collections of languages. ISO 639-3 provides identifiers for a far more comprehensive set of individual languages, though not for collections. Entities in ISO 639 that are referenced here may include any of the individual languages covered in ISO 639-2 or ISO 639-3, or to any of the collections covered in ISO 639-2.

All tags are 4-byte character strings composed of a limited set of ASCII characters; for details regarding the Tag data type see §4.3 “Data types”. By convention, registered language system tags use three or four capital letters (0x41 – 0x5A).

| Language System | Language System Tag | Corresponding ISO 639 ID (if applicable) |
| --- | --- | --- |
| Abaza | 'ABA ' | abq |
| Abkhazian | 'ABK ' | abk |
| Acholi | 'ACH ' | ach |
| Achi | 'ACR ' | acr |
| Adyghe | 'ADY ' | ady |
| Afrikaans | 'AFK ' | afr |
| Afar | 'AFR ' | aar |
| Agaw | 'AGW ' | ahg |
| Aiton | 'AIO ' | aio |
| Akan | 'AKA ' | aka, fat, twi |
| Batak Angkola | 'AKB ' | akb |
| Alsatian | 'ALS ' | gsw |
| Altai | 'ALT ' | atv, alt |
| Amharic | 'AMH ' | amh |
| Anglo-Saxon | 'ANG ' | ang |
| Phonetic transcription—Americanist conventions | APPH |  |
| Arabic | 'ARA ' | ara |
| Aragonese | 'ARG ' | arg |
| Aari | 'ARI ' | aiw |
| Rakhine | 'ARK ' | mhv, rmz, rki |
| Assamese | 'ASM ' | asm |
| Asturian | 'AST ' | ast |
| Athapaskan languages | 'ATH ' | aht, apa, apk, apj, apl, apm, apw, ath, bea, sek, bcr, caf, chp, clc, coq, crx, ctc, den, dgr, gce, gwi, haa, hoi, hup, ing, kkz, koy, ktw, kuu, mvb, nav, qwt, scs, srs, taa, tau, tcb, tce, tfn, tgx, tht, tol, ttm, tuu, txc, wlk, xup, xsl |
| Avatime | 'AVN ' | avn |
| Avar | 'AVR ' | ava |
| Awadhi | 'AWA ' | awa |
| Aymara | 'AYM ' | aym |
| Torki | 'AZB ' | azb |
| Azerbaijani | 'AZE ' | aze |
| Badaga | 'BAD ' | bfq |
| Banda | BAD0 | bad, bbp, bfl, bjo, bpd, bqk, gox, kuw, liy, lna, lnl, mnh, nue, nuu, tor, yaj, zmz |
| Baghelkhandi | 'BAG ' | bfy |
| Balkar | 'BAL ' | krc |
| Balinese | 'BAN ' | ban |
| Bavarian | 'BAR ' | bar |
| Baulé | 'BAU ' | bci |
| Batak Toba | 'BBC ' | bbc |
| Berber | 'BBR ' | auj, ber, cnu, gha, gho, grr, jbe, jbn, kab, mzb, oua, rif, sds, shi, shy, siz, sjs, swn, taq, tez, thv, thz, tia, tjo, tmh, ttq, tzm, zen, zgh |
| Bench | 'BCH ' | bcq |
| Bible Cree | 'BCR ' |  |
| Bandjalang | 'BDY ' | bdy |
| Belarusian | 'BEL ' | bel |
| Bemba | 'BEM ' | bem |
| Bengali | 'BEN ' | ben |
| Haryanvi | 'BGC ' | bgc |
| Bagri | 'BGQ ' | bgq |
| Bulgarian | 'BGR ' | bul |
| Bhili | 'BHI ' | bhi, bhb |
| Bhojpuri | 'BHO ' | bho |
| Bikol | 'BIK ' | bik, bhk, bcl, bto, cts, bln, fbl, lbl, rbl, ubl |
| Bilen | 'BIL ' | byn |
| Bislama | 'BIS ' | bis |
| Kanauji | 'BJJ ' | bjj |
| Blackfoot | 'BKF ' | bla |
| Baluchi | 'BLI ' | bal |
| Pa’o Karen | 'BLK ' | blk |
| Balante | 'BLN ' | bjt, ble |
| Balti | 'BLT ' | bft |
| Bambara (Bamanankan) | 'BMB ' | bam |
| Bamileke | 'BML ' | bai, bbj, bko, byv, fmp, jgo, nla, nnh, nnz, nwe, xmg, ybb |
| Bosnian | 'BOS ' | bos |
| Bishnupriya Manipuri | 'BPY ' | bpy |
| Breton | 'BRE ' | bre |
| Brahui | 'BRH ' | brh |
| Braj Bhasha | 'BRI ' | bra |
| Burmese | 'BRM ' | mya |
| Bodo | 'BRX ' | brx |
| Bashkir | 'BSH ' | bak |
| Burushaski | 'BSK ' | bsk |
| Batak Dairi (Pakpak) | 'BTD ' | btd |
| Beti | 'BTI ' | btb, beb, bum, bxp, eto, ewo, mct |
| Batak languages | 'BTK ' | akb, bbc, btd, btk, btm, bts, btx, btz |
| Batak Mandailing | 'BTM ' | btm |
| Batak Simalungun | 'BTS ' | bts |
| Batak Karo | 'BTX ' | btx |
| Batak Alas-Kluet | 'BTZ ' | btz |
| Bugis | 'BUG ' | bug |
| Medumba | 'BYV ' | byv |
| Kaqchikel | 'CAK ' | cak |
| Catalan | 'CAT ' | cat |
| Zamboanga Chavacano | 'CBK ' | cbk |
| Chinantec | CCHN | cco, chj, chq, chz, cle, cnl, cnt, cpa, csa, cso, cte, ctl, cuc, cvn |
| Cebuano | 'CEB ' | ceb |
| Chiga | 'CGG ' | cgg |
| Chamorro | 'CHA ' | cha |
| Chechen | 'CHE ' | che |
| Chaha Gurage | 'CHG ' | sgw |
| Chattisgarhi | 'CHH ' | hne |
| Chichewa (Chewa, Nyanja) | 'CHI ' | nya |
| Chukchi | 'CHK ' | ckt |
| Chuukese | CHK0 | chk |
| Choctaw | 'CHO ' | cho |
| Chipewyan | 'CHP ' | chp |
| Cherokee | 'CHR ' | chr |
| Chuvash | 'CHU ' | chv |
| Cheyenne | 'CHY ' | chy |
| Western Cham | 'CJA ' | cja |
| Eastern Cham | 'CJM ' | cjm |
| Comorian | 'CMR ' | swb, wlc, wni, zdj |
| Coptic | 'COP ' | cop |
| Cornish | 'COR ' | cor |
| Corsican | 'COS ' | cos |
| Creoles | 'CPP ' | abs, acf, afs, aig, aoa, bah, bew, bis, bjs, bpl, bpq, brc, bxo, bzj, bzk, cbk, ccl, ccm, chn, cks, cpe, cpf, cpi, cpp, cri, crp, crs, dcr, dep, djk, fab, fng, fpe, gac, gcf, gcl, gcr, gib, goq, gpe, gul, gyn, hat, hca, hmo, hwc, icr, idb, ihb, jam, jvd, kcn, kea, kmv, kri, kww, lir, lou, lrt, max, mbf, mcm, mfe, mfp, mkn, mod, msi, mud, mzs, nag, nef, ngm, njt, onx, oor, pap, pcm, pea, pey, pga, pih, pis, pln, pml, pmy, pov, pre, rcf, rop, scf, sci, skw, srm, srn, sta, svc, tas, tch, tcs, tgh, tmg, tpi, trf, tvy, uln, vic, vkp, wes, xmm |
| Cree | 'CRE ' | cre |
| Carrier | 'CRR ' | crx, caf |
| Crimean Tatar | 'CRT ' | crh |
| Kashubian | 'CSB ' | csb |
| Church Slavonic | 'CSL ' | chu |
| Czech | 'CSY ' | ces |
| Wayanad Chetti | 'CTT ' | ctt |
| Chittagonian | 'CTG ' | ctg |
| San Blas Kuna | 'CUK ' | cuk |
| Dagbani | 'DAG ' | dag |
| Danish | 'DAN ' | dan |
| Dargwa | 'DAR ' | dar |
| Dayi | 'DAX ' | dax |
| Woods Cree | 'DCR ' | cwd |
| German | 'DEU ' | deu |
| Dogri (individual language) | 'DGO ' | dgo |
| Dogri (macrolanguage) | 'DGR ' | doi |
| Dhangu | 'DHG ' | dhg |
| Divehi (Dhivehi, Maldivian) | 'DHV ' (deprecated) | div |
| Dimli | 'DIQ ' | diq |
| Divehi (Dhivehi, Maldivian) | 'DIV ' | div |
| Zarma | 'DJR ' | dje |
| Djambarrpuyngu | DJR0 | djr |
| Dangme | 'DNG ' | ada |
| Dan | 'DNJ ' | dnj |
| Dinka | 'DNK ' | din |
| Dari | 'DRI ' | prs |
| Dhuwal | 'DUJ ' | duj, dwu, dwy |
| Dungan | 'DUN ' | dng |
| Dzongkha | 'DZN ' | dzo |
| Ebira | 'EBI ' | igb |
| Eastern Cree | 'ECR ' | crj, crl |
| Edo | 'EDO ' | bin |
| Efik | 'EFI ' | efi |
| Greek | 'ELL ' | ell |
| Eastern Maninkakan | 'EMK ' | emk |
| English | 'ENG ' | eng |
| Erzya | 'ERZ ' | myv |
| Spanish | 'ESP ' | spa |
| Central Yupik | 'ESU ' | esu |
| Estonian | 'ETI ' | est |
| Basque | 'EUQ ' | eus |
| Evenki | 'EVK ' | evn |
| Even | 'EVN ' | eve |
| Ewe | 'EWE ' | ewe |
| French Antillean | 'FAN ' | acf |
| Fang | FAN0 | fan |
| Persian | 'FAR ' | fas |
| Fanti | 'FAT ' | fat |
| Finnish | 'FIN ' | fin |
| Fijian | 'FJI ' | fij |
| Dutch (Flemish) | 'FLE ' | vls |
| Fe’fe’ | 'FMP ' | fmp |
| Forest Enets | 'FNE ' | enf |
| Fon | 'FON ' | fon |
| Faroese | 'FOS ' | fao |
| French | 'FRA ' | fra |
| Cajun French | 'FRC ' | frc |
| Frisian | 'FRI ' | fry |
| Friulian | 'FRL ' | fur |
| Arpitan | 'FRP ' | frp |
| Futa | 'FTA ' | fuf |
| Fulah | 'FUL ' | ful |
| Nigerian Fulfulde | 'FUV ' | fuv |
| Ga | 'GAD ' | gaa |
| Scottish Gaelic (Gaelic) | 'GAE ' | gla |
| Gagauz | 'GAG ' | gag |
| Galician | 'GAL ' | glg |
| Garshuni | 'GAR ' |  |
| Garhwali | 'GAW ' | gbm |
| Geez | 'GEZ ' | gez |
| Githabul | 'GIH ' | gih |
| Gilyak | 'GIL ' | niv |
| Kiribati (Gilbertese) | GIL0 | gil |
| Kpelle (Guinea) | 'GKP ' | gkp |
| Gilaki | 'GLK ' | glk |
| Gumuz | 'GMZ ' | guk |
| Gumatj | 'GNN ' | gnn |
| Gogo | 'GOG ' | gog |
| Gondi | 'GON ' | gon |
| Greenlandic | 'GRN ' | kal |
| Garo | 'GRO ' | grt |
| Guarani | 'GUA ' | grn |
| Wayuu | 'GUC ' | guc |
| Gupapuyngu | 'GUF ' | guf |
| Gujarati | 'GUJ ' | guj |
| Gusii | 'GUZ ' | guz |
| Haitian (Haitian Creole) | 'HAI ' | hat |
| Haida | 'HAI0' | hai, hax, hdn |
| Halam (Falam Chin) | 'HAL ' | cfm |
| Harauti | 'HAR ' | hoj |
| Hausa | 'HAU ' | hau |
| Hawaiian | 'HAW ' | haw |
| Haya | 'HAY ' | hay |
| Hazaragi | 'HAZ ' | haz |
| Hammer-Banna | 'HBN ' | amf |
| Heiltsuk | 'HEI ' | hei |
| Herero | 'HER ' | her |
| Hiligaynon | 'HIL ' | hil |
| Hindi | 'HIN ' | hin |
| High Mari | 'HMA ' | mrj |
| A-Hmao | 'HMD ' | hmd |
| Hmong | 'HMN ' | hmn |
| Hiri Motu | 'HMO ' | hmo |
| Hmong Shuat | 'HMZ ' | hmz |
| Hindko | 'HND ' | hno, hnd |
| Ho | 'HO ' | hoc |
| Harari | 'HRI ' | har |
| Croatian | 'HRV ' | hrv |
| Hungarian | 'HUN ' | hun |
| Armenian | 'HYE ' | hye, hyw |
| Armenian East | HYE0 | hye |
| Iban | 'IBA ' | iba |
| Ibibio | 'IBB ' | ibb |
| Igbo | 'IBO ' | ibo |
| Ido | 'IDO ' | ido |
| Ijo languages | 'IJO ' | iby, ijc, ije, ijn, ijo, ijs, nkx, okd, okr, orr |
| Interlingue | 'ILE ' | ile |
| Ilokano | 'ILO ' | ilo |
| Interlingua | 'INA ' | ina |
| Indonesian | 'IND ' | ind |
| Ingush | 'ING ' | inh |
| Inuktitut | 'INU ' | iku |
| Nunavik Inuktitut | ‘INUK’ | ike, iku |
| Inupiat | 'IPK ' | ipk |
| Phonetic transcription—IPA conventions | IPPH |  |
| Irish | 'IRI ' | gle |
| Irish Traditional | 'IRT ' | gle |
| Irula | 'IRU ' | iru |
| Icelandic | 'ISL ' | isl |
| Inari Sami | 'ISM ' | smn |
| Italian | 'ITA ' | ita |
| Hebrew | 'IWR ' | heb |
| Jamaican Creole | 'JAM ' | jam |
| Japanese | 'JAN ' | jpn |
| Javanese | 'JAV ' | jav |
| Lojban | 'JBO ' | jbo |
| Krymchak | 'JCT ' | jct |
| Yiddish | 'JII ' | yid |
| Ladino | 'JUD ' | lad |
| Jula | 'JUL ' | dyu |
| Kabardian | 'KAB ' | kbd |
| Kabyle | KAB0 | kab |
| Kachchi | 'KAC ' | kfr |
| Kalenjin | 'KAL ' | kln |
| Kannada | 'KAN ' | kan |
| Karachay | 'KAR ' | krc |
| Georgian | 'KAT ' | kat |
| Kawi (Old Javanese) | ‘KAW ’ | kaw |
| Kazakh | 'KAZ ' | kaz |
| Makonde | 'KDE ' | kde |
| Kabuverdianu (Crioulo) | 'KEA ' | kea |
| Kebena | 'KEB ' | ktb |
| Kekchi | 'KEK ' | kek |
| Khutsuri Georgian | 'KGE ' | kat |
| Khakass | 'KHA ' | kjh |
| Khanty-Kazim | 'KHK ' | kca |
| Khmer | 'KHM ' | khm |
| Khanty-Shurishkar | 'KHS ' | kca |
| Khamti Shan | 'KHT ' | kht |
| Khanty-Vakhi | 'KHV ' | kca |
| Khowar | 'KHW ' | khw |
| Kikuyu (Gikuyu) | 'KIK ' | kik |
| Kirghiz (Kyrgyz) | 'KIR ' | kir |
| Kisii | 'KIS ' | kqs, kss |
| Kirmanjki | 'KIU ' | kiu |
| Southern Kiwai | 'KJD ' | kjd |
| Eastern Pwo Karen | 'KJP ' | kjp |
| Bumthangkha | 'KJZ ' | kjz |
| Kokni | 'KKN ' | kex |
| Kalmyk | 'KLM ' | xal |
| Kamba | 'KMB ' | kam |
| Kumaoni | 'KMN ' | kfy |
| Komo | 'KMO ' | kmw |
| Komso | 'KMS ' | kxc |
| Khorasani Turkic | 'KMZ ' | kmz |
| Kanuri | 'KNR ' | kau |
| Kodagu | 'KOD ' | kfa |
| Korean Old Hangul | 'KOH ' | kor, okm |
| Konkani | 'KOK ' | kok |
| Komi | 'KOM ' | kom |
| Kikongo | 'KON ' | ktu |
| Kongo | KON0 | kon |
| Komi-Permyak | 'KOP ' | koi |
| Korean | 'KOR ' | kor |
| Kosraean | 'KOS ' | kos |
| Komi-Zyrian | 'KOZ ' | kpv |
| Kpelle | 'KPL ' | kpe |
| Krio | 'KRI ' | kri |
| Karakalpak | 'KRK ' | kaa |
| Karelian | 'KRL ' | krl |
| Karaim | 'KRM ' | kdr |
| Karen | 'KRN ' | blk, bwe, eky, ghk, jkm, jkp, kar, kjp, kjt, ksw, kvl, kvq, kvt, kvu, kvy, kxf, kxk, kyu, pdu, pwo, pww, wea |
| Koorete | 'KRT ' | kqy |
| Kashmiri | 'KSH ' | kas |
| Ripuarian | KSH0 | ksh |
| Khasi | 'KSI ' | kha |
| Kildin Sami | 'KSM ' | sjd |
| S’gaw Karen | 'KSW ' | ksw |
| Kuanyama | 'KUA ' | kua |
| Kui | 'KUI ' | kxu |
| Kulvi | 'KUL ' | kfx |
| Kumyk | 'KUM ' | kum |
| Kurdish | 'KUR ' | kur |
| Kurukh | 'KUU ' | kru |
| Kuy | 'KUY ' | kdt |
| Kwakʼwala | 'KWK ' | kwk |
| Koryak | 'KYK ' | kpy |
| Western Kayah | 'KYU ' | kyu |
| Ladin | 'LAD ' | lld |
| Lahuli | 'LAH ' | bfu |
| Lak | 'LAK ' | lbe |
| Lambani | 'LAM ' | lmn |
| Lao | 'LAO ' | lao |
| Latin | 'LAT ' | lat |
| Laz | 'LAZ ' | lzz |
| L-Cree | 'LCR ' | crm |
| Ladakhi | 'LDK ' | lbj |
| Lelemi | 'LEF ' | lef |
| Lezgi | 'LEZ ' | lez |
| Ligurian | 'LIJ ' | lij |
| Limburgish | 'LIM ' | lim |
| Lingala | 'LIN ' | lin |
| Lisu | 'LIS ' | lis |
| Lampung | 'LJP ' | ljp |
| Laki | 'LKI ' | lki |
| Low Mari | 'LMA ' | mhr |
| Limbu | 'LMB ' | lif |
| Lombard | 'LMO ' | lmo |
| Lomwe | 'LMW ' | ngl |
| Loma | 'LOM ' | lom |
| Lipo | 'LPO ' | lpo |
| Luri | 'LRC ' | lrc, luz, bqi, zum |
| Lower Sorbian | 'LSB ' | dsb |
| Lule Sami | 'LSM ' | smj |
| Lithuanian | 'LTH ' | lit |
| Luxembourgish | 'LTZ ' | ltz |
| Luba-Lulua | 'LUA ' | lua |
| Luba-Katanga | 'LUB ' | lub |
| Ganda | 'LUG ' | lug |
| Luyia | 'LUH ' | luy |
| Luo | 'LUO ' | luo |
| Latvian | 'LVI ' | lav |
| Madura | 'MAD ' | mad |
| Magahi | 'MAG ' | mag |
| Marshallese | 'MAH ' | mah |
| Majang | 'MAJ ' | mpe |
| Makhuwa | 'MAK ' | vmw |
| Malayalam | 'MAL ' | mal |
| Mam | 'MAM ' | mam |
| Mansi | 'MAN ' | mns |
| Mapudungun | 'MAP ' | arn |
| Marathi | 'MAR ' | mar |
| Marwari | 'MAW ' | mwr, dhd, rwr, mve, wry, mtr, swv |
| Mbundu | 'MBN ' | kmb |
| Mbo | 'MBO ' | mbo |
| Manchu | 'MCH ' | mnc |
| Moose Cree | 'MCR ' | crm |
| Mende | 'MDE ' | men |
| Mandar | 'MDR ' | mdr |
| Me’en | 'MEN ' | mym |
| Meru | 'MER ' | mer |
| Pattani Malay | 'MFA ' | mfa |
| Morisyen | 'MFE ' | mfe |
| Minangkabau | 'MIN ' | min |
| Mizo | 'MIZ ' | lus |
| Macedonian | 'MKD ' | mkd |
| Makasar | 'MKR ' | mak |
| Kituba | 'MKW ' | mkw |
| Male | 'MLE ' | mdy |
| Malagasy | 'MLG ' | mlg |
| Malinke | 'MLN ' | mlq |
| Malayalam Reformed | 'MLR ' | mal |
| Malay | 'MLY ' | msa |
| Mandinka | 'MND ' | mnk |
| Mongolian | 'MNG ' | mon |
| Manipuri | 'MNI ' | mni |
| Maninka | 'MNK ' | man, mnk, myq, mku, msc, emk, mwk, mlq |
| Manx | 'MNX ' | glv |
| Mohawk | 'MOH ' | moh |
| Moksha | 'MOK ' | mdf |
| Moldavian | 'MOL ' | mol |
| Mon | 'MON ' | mnw |
| Moroccan | 'MOR ' |  |
| Mossi | 'MOS ' | mos |
| Maori | 'MRI ' | mri |
| Maithili | 'MTH ' | mai |
| Maltese | 'MTS ' | mlt |
| Mundari | 'MUN ' | unr |
| Muscogee | 'MUS ' | mus |
| Mirandese | 'MWL ' | mwl |
| Hmong Daw | 'MWW ' | mww |
| Mayan | 'MYN ' | acr, agu, caa, cac, cak, chf, ckz, cob, ctu, emy, hus, itz, ixl, jac, kek, kjb, knj, lac, mam, mhc, mop, myn, poc, poh, quc, qum, quv, toj, ttc, tzh, tzj, tzo, usp, yua |
| Mazanderani | 'MZN ' | mzn |
| Naga-Assamese | 'NAG ' | nag |
| Nahuatl | 'NAH ' | azd, azn, azz, nah, naz, nch, nci, ncj, ncl, ncx, ngu, nhc, nhe, nhg, nhi, nhk, nhm, nhn, nhp, nhq, nht, nhv, nhw, nhx, nhy, nhz, nlv, npl, nsu, nuz |
| Nanai | 'NAN ' | gld |
| Neapolitan | 'NAP ' | nap |
| Naskapi | 'NAS ' | nsk |
| Nauruan | 'NAU ' | nau |
| Navajo | 'NAV ' | nav |
| N-Cree | 'NCR ' | csw |
| Ndebele | 'NDB ' | nbl, nde |
| Ndau | 'NDC ' | ndc |
| Ndonga | 'NDG ' | ndo |
| Low Saxon | 'NDS ' | nds |
| Nepali | 'NEP ' | nep |
| Newari | 'NEW ' | new |
| Ngbaka | 'NGA ' | nga |
| Nagari | 'NGR ' |  |
| Norway House Cree | 'NHC ' | csw |
| Nisi | 'NIS ' | dap, njz, tgj |
| Niuean | 'NIU ' | niu |
| Nyankole | 'NKL ' | nyn |
| N’Ko | 'NKO ' | nqo |
| Dutch | 'NLD ' | nld |
| Nimadi | 'NOE ' | noe |
| Nogai | 'NOG ' | nog |
| Norwegian | 'NOR ' | nob |
| Novial | 'NOV ' | nov |
| Northern Sami | 'NSM ' | sme |
| Northern Sotho | 'NSO ' | nso |
| Northern Tai | 'NTA ' | nod |
| Esperanto | 'NTO ' | epo |
| Nyamwezi | 'NYM ' | nym |
| Norwegian Nynorsk (Nynorsk, Norwegian) | 'NYN ' | nno |
| Mbembe Tigon | 'NZA ' | nza |
| Occitan | 'OCI ' | oci |
| Oji-Cree | 'OCR ' | ojs |
| Ojibway | 'OJB ' | oji |
| Odia (formerly Oriya) | 'ORI ' | ori |
| Oromo | 'ORO ' | orm |
| Ossetian | 'OSS ' | oss |
| Palestinian Aramaic | 'PAA ' | sam |
| Pangasinan | 'PAG ' | pag |
| Pali | 'PAL ' | pli |
| Pampangan | 'PAM ' | pam |
| Punjabi | 'PAN ' | pan |
| Palpa | 'PAP ' | plp |
| Papiamentu | PAP0 | pap |
| Pashto | 'PAS ' | pus |
| Palauan | 'PAU ' | pau |
| Bouyei | 'PCC ' | pcc |
| Picard | 'PCD ' | pcd |
| Pennsylvania German | 'PDC ' | pdc |
| Polytonic Greek | 'PGR ' | ell |
| Phake | 'PHK ' | phk |
| Norfolk | 'PIH ' | pih |
| Filipino | 'PIL ' | fil |
| Palaung | 'PLG ' | pce, rbb, pll |
| Polish | 'PLK ' | pol |
| Piemontese | 'PMS ' | pms |
| Western Panjabi | 'PNB ' | pnb |
| Pocomchi | 'POH ' | poh |
| Pohnpeian | 'PON ' | pon |
| Provençal / Old Provençal | 'PRO ' | pro |
| Portuguese | 'PTG ' | por |
| Western Pwo Karen | 'PWO ' | pwo |
| Chin | 'QIN ' | bgr, biu, cek, cey, cfm, cbl, cka, ckn, clj, clt, cmr, cnb, cnh, cnk, cnw, csh, csj, csv, csy, ctd, cth, czt, dao, gnb, hlt, hmr, hra, lus, mrh, mwq, pck, pkh, pub, ral, rtc, sch, sez, shl, smt, tcp, tcz, vap, weu, zom, zyp |
| K’iche’ | 'QUC ' | quc |
| Quechua (Bolivia) | 'QUH ' | quh |
| Quechua | 'QUZ ' | quz |
| Quechua (Ecuador) | 'QVI ' | qvi |
| Quechua (Peru) | 'QWH ' | qwh |
| Rajasthani | 'RAJ ' | raj |
| Rarotongan | 'RAR ' | rar |
| Russian Buriat | 'RBU ' | bxr |
| R-Cree | 'RCR ' | atj |
| Rejang | 'REJ ' | rej |
| Rohingya | 'RHG ' | rhg |
| Riang | 'RIA ' | ria |
| Tarifit | 'RIF ' | rif |
| Ritarungo | 'RIT ' | rit |
| Arakwal | 'RKW ' | rkw |
| Romansh | 'RMS ' | roh |
| Vlax Romani | 'RMY ' | rmy |
| Romanian | 'ROM ' | ron |
| Romany | 'ROY ' | rom |
| Rusyn | 'RSY ' | rue |
| Rotuman | 'RTM ' | rtm |
| Kinyarwanda | 'RUA ' | kin |
| Rundi | 'RUN ' | run |
| Aromanian | 'RUP ' | rup |
| Russian | 'RUS ' | rus |
| Sadri | 'SAD ' | sck |
| Sanskrit | 'SAN ' | san |
| Sasak | 'SAS ' | sas |
| Santali | 'SAT ' | sat |
| Sayisi | 'SAY ' | chp |
| Sicilian | 'SCN ' | scn |
| Scots | 'SCO ' | sco |
| North Slavey | 'SCS ' | scs |
| Sekota | 'SEK ' | xan |
| Selkup | 'SEL ' | sel |
| Small Flowery Miao | 'SFM ' | sfm |
| Old Irish | 'SGA ' | sga |
| Sango | 'SGO ' | sag |
| Samogitian | 'SGS ' | sgs |
| Tachelhit | 'SHI ' | shi |
| Shan | 'SHN ' | shn |
| Sibe | 'SIB ' | sjo |
| Sidamo | 'SID ' | sid |
| Silte Gurage | 'SIG ' | xst, stv, wle |
| Skolt Sami | 'SKS ' | sms |
| Slovak | 'SKY ' | slk |
| Slavey | 'SLA ' | den, scs, xsl |
| Slovenian | 'SLV ' | slv |
| Somali | 'SML ' | som |
| Samoan | 'SMO ' | smo |
| Sena | 'SNA ' | seh |
| Shona | SNA0 | sna |
| Sindhi | 'SND ' | snd |
| Sinhala (Sinhalese) | 'SNH ' | sin |
| Soninke | 'SNK ' | snk |
| Sodo Gurage | 'SOG ' | gru |
| Songe | 'SOP ' | sop |
| Southern Sotho | 'SOT ' | sot |
| Albanian | 'SQI ' | sqi |
| Serbian | 'SRB ' | cnr, srp |
| Sardinian | 'SRD ' | srd |
| Saraiki | 'SRK ' | skr |
| Serer | 'SRR ' | srr |
| South Slavey | 'SSL ' | xsl |
| Southern Sami | 'SSM ' | sma |
| Saterland Frisian | 'STQ ' | stq |
| Sukuma | 'SUK ' | suk |
| Sundanese | 'SUN ' | sun |
| Suri | 'SUR ' | suq |
| Svan | 'SVA ' | sva |
| Swedish | 'SVE ' | swe |
| Swadaya Aramaic | 'SWA ' | aii |
| Swahili | 'SWK ' | swa |
| Swati | 'SWZ ' | ssw |
| Sutu | 'SXT ' | ngo, xnj, xnq |
| Upper Saxon | 'SXU ' | sxu |
| Sylheti | 'SYL ' | syl |
| Syriac | 'SYR ' | aii, amw, cld, syc, syr, tru |
| Syriac, Estrangela script-variant (equivalent to ISO 15924 'Syre') | 'SYRE' | syc, syr |
| Syriac, Western script-variant (equivalent to ISO 15924 'Syrj') | 'SYRJ' | syc, syr |
| Syriac, Eastern script-variant (equivalent to ISO 15924 'Syrn') | 'SYRN' | syc, syr |
| Silesian | 'SZL ' | szl |
| Tabasaran | 'TAB ' | tab |
| Tajiki | 'TAJ ' | tgk |
| Tamil | 'TAM ' | tam |
| Tatar | 'TAT ' | tat |
| TH-Cree | 'TCR ' | cwd |
| Dehong Dai | 'TDD ' | tdd |
| Telugu | 'TEL ' | tel |
| Tetum | 'TET ' | tet |
| Tagalog | 'TGL ' | tgl |
| Tongan | 'TGN ' | ton |
| Tigre | 'TGR ' | tig |
| Tigrinya | 'TGY ' | tir |
| Thai | 'THA ' | tha |
| Tahitian | 'THT ' | tah |
| Tibetan | 'TIB ' | bod |
| Tiv | 'TIV ' | tiv |
| Turkmen | 'TKM ' | tuk |
| Tlingit | 'TLI ' | tli |
| Tamashek | 'TMH ' | taq, thv, thz, tmh, ttq |
| Temne | 'TMN ' | tem |
| Tswana | 'TNA ' | tsn |
| Tundra Enets | 'TNE ' | enh |
| Tonga | 'TNG ' | toi |
| Todo | 'TOD ' | xal |
| Toma | TOD0 | tod |
| Tok Pisin | 'TPI ' | tpi |
| Turkish | 'TRK ' | tur |
| Tsonga | 'TSG ' | tso |
| Tshangla | 'TSJ ' | tsj |
| Turoyo Aramaic | 'TUA ' | tru |
| Tumbuka | 'TUL ' | tcy |
| Tulu | 'TUM ' | tum |
| Tuvin | 'TUV ' | tyv |
| Tuvalu | 'TVL ' | tvl |
| Twi | 'TWI ' | twi |
| Tày | 'TYZ ' | tyz |
| Tamazight | 'TZM ' | tzm |
| Tzotzil | 'TZO ' | tzo |
| Udmurt | 'UDM ' | udm |
| Ukrainian | 'UKR ' | ukr |
| Umbundu | 'UMB ' | umb |
| Urdu | 'URD ' | urd |
| Upper Sorbian | 'USB ' | hsb |
| Uyghur | 'UYG ' | uig |
| Uzbek | 'UZB ' | uzb |
| Venetian | 'VEC ' | vec |
| Venda | 'VEN ' | ven |
| Vietnamese | 'VIT ' | vie |
| Volapük | 'VOL ' | vol |
| Võro | 'VRO ' | vro |
| Wa | 'WA ' | wbm |
| Wagdi | 'WAG ' | wbr |
| Waray-Waray | 'WAR ' | war |
| Waci Gbe | 'WCI ' | wci |
| West-Cree | 'WCR ' | crk |
| Welsh | 'WEL ' | cym |
| Wolof | 'WLF ' | wol |
| Walloon | 'WLN ' | wln |
| Mewati | 'WTM ' | wtm |
| Lü | 'XBD ' | khb |
| Xhosa | 'XHS ' | xho |
| Minjangbal | 'XJB ' | xjb |
| Khengkha | 'XKF ' | xkf |
| Soga | 'XOG ' | xog |
| Kpelle (Liberia) | 'XPE ' | xpe |
| Bette Kuruma | 'XUB ' | xub |
| Jennu Kuruma | 'XUJ ' | xuj |
| Sakha | 'YAK ' | sah |
| Yao | 'YAO ' | yao |
| Yapese | 'YAP ' | yap |
| Yoruba | 'YBA ' | yor |
| Y-Cree | 'YCR ' | crj, crk, crl |
| Gepo | 'YGP ' | ygp |
| Yi Classic | 'YIC ' |  |
| Yi Modern | 'YIM ' | iii |
| Aluo | 'YNA ' | yna |
| Wuding-Luquan Yi | 'YWQ ' | ywq |
| Zealandic | 'ZEA ' | zea |
| Standard Moroccan Tamazight | 'ZGH ' | zgh |
| Zhuang | 'ZHA ' | zha |
| Chinese, Traditional, Hong Kong SAR | 'ZHH ' | zho |
| Chinese, Phonetic | 'ZHP ' | zho |
| Chinese, Simplified | 'ZHS ' | zho |
| Chinese, Traditional | 'ZHT ' | zho |
| Chinese, Traditional, Macao SAR | 'ZHTM' | zho |
| Zande | 'ZND ' | zne |
| Zulu | 'ZUL ' | zul |
| Zazaki | 'ZZA ' | zza |

### Feature tags

Features provide information about how to use the glyphs in a font to render a script or language. For example, an Arabic font might have a feature for substituting initial glyph forms, and a Kanji font might have a feature for positioning glyphs vertically. All OFF Layout features define data for glyph substitution, glyph positioning, or both.

Each OFF Layout feature has a feature tag that identifies its typographic function and effects. By examining a feature's tag, a text-processing client can determine what a feature does and decide whether to implement it. All tags are four-character strings composed of a limited set of ASCII characters; for details regarding the Tag data type see §4.3 “Data types”. By convention, registered feature tags use four lowercase letters. For instance, the ‘mark’ feature manages the placement of diacritical marks, and the ‘swsh’ feature renders swash glyphs.

The tag space of tags consisting of four uppercase letters (A-Z) with no punctuation, spaces, or numbers, is reserved as a vendor space. Font vendors may use such tags to identify private features. For example, the feature tag PKRN might designate a private feature that may be used to kern punctuation marks.

A feature definition may not provide all the information required to properly implement glyph substitution or positioning actions. In many cases, a text-processing client may need to supply additional data. For example, the function of the ‘init’ feature is to provide initial glyph forms. Nothing in the feature's lookup tables indicates when or where to apply this feature during text processing. To correctly use the ‘init’ feature in Arabic text, in which initial glyph forms appear at the beginning of connected letter groups determined by character-joining properties, text-processing clients must be able to identify the glyphs to which the feature should be applied, based on character context and joining properties. In all cases, the text-processing client is responsible for applying, combining, and arbitrating among features and rendering the result.

NOTE	There is no guarantee the compatibility or usability of private features, and it cannot be ensured that two font vendors will not choose the same tag for a private feature.

This Tag Registry describes all the OFF Layout features. Lookup information is provided for reference purposes only; the set of lookups used to implement a feature will vary across system platforms, applications, fonts, and font developers.

#### Feature tag list

Registered features

The features listed below are sorted in alphabetical order by tag name.

| Feature Tag | Friendly Name |
| --- | --- |
| 'aalt' | Access All Alternates |
| 'abvf' | Above-base Forms |
| 'abvm' | Above-base Mark Positioning |
| 'abvs' | Above-base Substitutions |
| 'afrc' | Alternative Fractions |
| 'akhn' | Akhands |
| 'blwf' | Below-base Forms |
| 'blwm' | Below-base Mark Positioning |
| 'blws' | Below-base Substitutions |
| 'calt' | Contextual Alternates |
| 'case' | Case-Sensitive Forms |
| 'ccmp' | Glyph Composition / Decomposition |
| 'cfar' | Conjunct Form After Ro |
| 'chws' | Contextual Half-Width Spacing |
| 'cjct' | Conjunct Forms |
| 'clig' | Contextual Ligatures |
| 'cpct' | Centered CJK Punctuation |
| 'cpsp' | Capital Spacing |
| 'cswh' | Contextual Swash |
| 'curs' | Cursive Positioning |
| 'cv01-cv99' | Character Variants |
| 'c2pc' | Petite Capitals From Capitals |
| 'c2sc' | Small Capitals From Capitals |
| 'dist' | Distances |
| 'dlig' | Discretionary Ligatures |
| 'dnom' | Denominators |
| 'dtls' | Dotless Forms |
| 'expt' | Expert Forms |
| 'falt' | Final Glyph on Line Alternates |
| 'fin2' | Terminal Forms #2 |
| 'fin3' | Terminal Forms #3 |
| 'fina' | Terminal Forms |
| 'flac' | Flattened accent forms |
| 'frac' | Fractions |
| 'fwid' | Full Widths |
| 'half' | Half Forms |
| 'haln' | Halant Forms |
| 'halt' | Alternate Half Widths |
| 'hist' | Historical Forms |
| 'hkna' | Horizontal Kana Alternates |
| 'hlig' | Historical Ligatures |
| 'hngl' | Hangul |
| 'hojo' | Hojo Kanji Forms (JIS X 0212-1990 Kanji Forms) |
| 'hwid' | Half Widths |
| 'init' | Initial Forms |
| 'isol' | Isolated Forms |
| 'ital' | Italics |
| 'jalt' | Justification Alternates |
| 'jp78' | JIS78 Forms |
| 'jp83' | JIS83 Forms |
| 'jp90' | JIS90 Forms |
| 'jp04' | JIS2004 Forms |
| 'kern' | Kerning |
| 'lfbd' | Left Bounds |
| 'liga' | Standard Ligatures |
| 'ljmo' | Leading Jamo Forms |
| 'lnum' | Lining Figures |
| 'locl' | Localized Forms |
| 'ltra' | Left-to-right glyph alternates |
| 'ltrm' | Left-to-right mirrored forms |
| 'mark' | Mark Positioning |
| 'med2' | Medial Forms #2 |
| 'medi' | Medial Forms |
| 'mgrk' | Mathematical Greek |
| 'mkmk' | Mark to Mark Positioning |
| 'mset' | Mark Positioning via Substitution |
| 'nalt' | Alternate Annotation Forms |
| 'nlck' | NLC Kanji Forms |
| 'nukt' | Nukta Forms |
| 'numr' | Numerators |
| 'onum' | Oldstyle Figures |
| 'opbd' | Optical Bounds |
| 'ordn' | Ordinals |
| 'ornm' | Ornaments |
| 'palt' | Proportional Alternate Widths |
| 'pcap' | Petite Capitals |
| 'pkna' | Proportional Kana |
| 'pnum' | Proportional Figures |
| 'pref' | Pre-Base Forms |
| 'pres' | Pre-base Substitutions |
| 'pstf' | Post-base Forms |
| 'psts' | Post-base Substitutions |
| 'pwid' | Proportional Widths |
| 'qwid' | Quarter Widths |
| 'rand' | Randomize |
| 'rclt' | Required Contextual Alternates |
| 'rkrf' | Rakar Forms |
| 'rlig' | Required Ligatures |
| 'rphf' | Reph Forms |
| 'rtbd' | Right Bounds |
| 'rtla' | Right-to-left alternates |
| 'rtlm' | Right-to-left mirrored forms |
| 'ruby' | Ruby Notation Forms |
| 'rvrn' | Required Variation Alternates |
| 'salt' | Stylistic Alternates |
| 'sinf' | Scientific Inferiors |
| 'size' | Optical size |
| 'smcp' | Small Capitals |
| 'smpl' | Simplified Forms |
| 'ss01' | Stylistic Set 1 |
| 'ss02' | Stylistic Set 2 |
| 'ss03' | Stylistic Set 3 |
| 'ss04' | Stylistic Set 4 |
| 'ss05' | Stylistic Set 5 |
| 'ss06' | Stylistic Set 6 |
| 'ss07' | Stylistic Set 7 |
| 'ss08' | Stylistic Set 8 |
| 'ss09' | Stylistic Set 9 |
| 'ss10' | Stylistic Set 10 |
| 'ss11' | Stylistic Set 11 |
| 'ss12' | Stylistic Set 12 |
| 'ss13' | Stylistic Set 13 |
| 'ss14' | Stylistic Set 14 |
| 'ss15' | Stylistic Set 15 |
| 'ss16' | Stylistic Set 16 |
| 'ss17' | Stylistic Set 17 |
| 'ss18' | Stylistic Set 18 |
| 'ss19' | Stylistic Set 19 |
| 'ss20' | Stylistic Set 20 |
| 'ssty' | Math script style alternates |
| 'stch' | Stretching Glyph Decomposition |
| 'subs' | Subscript |
| 'sups' | Superscript |
| 'swsh' | Swash |
| 'titl' | Titling |
| 'tjmo' | Trailing Jamo Forms |
| 'tnam' | Traditional Name Forms |
| 'tnum' | Tabular Figures |
| 'trad' | Traditional Forms |
| 'twid' | Third Widths |
| 'unic' | Unicase |
| 'valt' | Alternate Vertical Metrics |
| 'vatu' | Vattu Variants |
| 'vchw' | Vertical Contextual Half-Width Spacing |
| 'vert' | Vertical Writing |
| 'vhal' | Alternate Vertical Half Metrics |
| 'vjmo' | Vowel Jamo Forms |
| 'vkna' | Vertical Kana Alternates |
| 'vkrn' | Vertical Kerning |
| 'vpal' | Proportional Alternate Vertical Metrics |
| 'vrt2' | Vertical Alternates and Rotation |
| 'vrtr' | Vertical Alternates for Rotation |
| 'zero' | Slashed Zero |

#### Feature descriptions and implementations

Tag: 'aalt'

Friendly name: Access All Alternates

Function: This feature makes all variations of a selected character accessible. This serves several purposes: An application may not support the feature by which the desired glyph would normally be accessed; the user may need a glyph outside the context supported by the normal substitution, or the user may not know what feature produces the desired glyph. Since many-to-one substitutions are not covered, ligatures would not appear in this table unless they were variant forms of another ligature.

Example: A user inputs the P in Poetica, and is presented with a choice of the four standard capital forms, the eight swash capital forms, the initial capital form and the small capital form.

Recommended implementation: The aalt table groups glyphs into semantic units. These units include the glyph which represents the default form for the underlying Unicode value stored by the application. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. As in any one-from-many substitution, alternates present in more than one face should be ordered consistently across a family, so that those alternates can work correctly when switching between family members. This feature should be ordered first in the font, to take precedence over other features.

Application interface: The application determines the GID for the default form of a given character (Unicode value with no features applied). It then checks to see whether the GID is found in the aalt coverage table. If so, the application passes this value to the feature table and gets back the GIDs in the associated group.

UI suggestion: While most one-from-many substitution features can be applied globally with reasonable results, aalt is not designed to support this use. The application should indicate to the user which glyphs in the user's document have alternative forms (i.e. which are in the coverage table for aalt). When the user selects one of those glyphs and applies the aalt feature, an application could display the forms sequentially in context, or present a palette showing all the forms at once, or give the user a choice between these approaches. The application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly. When only one alternate exists, this feature could toggle directly between the alternate and default forms.

Script/language sensitivity: None.

Feature interaction: This feature may be used in combination with other features.

Tag: 'abvf'

Friendly name: Above-base Forms

Function: Substitutes the above-base form of a vowel.

Example: In complex scripts like Khmer, the vowel OE must be split into a pre-base form and an above-base form. The above-base form of OE would be substituted to form the correct piece of the letter that is displayed above the base consonant.

Recommended implementation: This feature substitutes the GID for OE with the above part of the glyph (GSUB lookup type 1).

Application interface: In a sequence where a split vowel with an above form is used, the application must insert the pre-base glyph into the correct location and then apply the above-base form feature. The application gets back the GID for the correct form for the piece that is placed above the base glyph. The application may also choose to position this glyph if required, after this feature is called.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Khmer script.

Feature interaction: This feature overrides the results of all other features.

Tag: 'abvm'

Friendly name: Above-base Mark Positioning

Function: Positions marks above base glyphs.

Example: In complex scripts like Devanagari (Indic), the Anuswar needs to be positioned above the base glyph. This base glyph can be a base consonant or conjunct. The base glyph and the presence/absence of other marks above the base glyph decides the location of the Anuswar, so that they do not overlap each other.

Recommended implementation: The abvm table provides positioning information (x,y) to enable mark positioning (GPOS lookup type 4, 5).

Application interface: The application must define the GIDs of the base glyphs above which marks need to be positioned, and the marks themselves. If these are located in the coverage table, the application passes the sequence to the abvm table and gets the positioning values (x,y) or positioning adjustments for the mark in return.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: Can be used to position default marks; or those that have been selected from a number of alternates based on contextual requirement using a feature like abvs.

Tag: 'abvs'

Friendly name: Above-base Substitutions

Function: Substitutes a ligature for a base glyph and mark that's above it.

Example: In complex scripts like Kannada (Indic), the vowel sign for the vowel I which a mark, is positioned above base consonants. This mark combines with the consonant Ga to form a ligature.

Recommended implementation: Lookups for this feature map each sequence of consonant and vowel sign to the corresponding ligature in the font (GSUB lookup type 4).

Application interface: The application must define the GIDs of the base glyphs and the mark that combines with it to form a ligature. The application passes the sequence to the abvs table. If these are located in the coverage table, it gets the GID for the ligature in return.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: None.

Tag: 'afrc'

Friendly name: Alternative Fractions

Function: Replaces figures separated by a slash with an alternative form.

Example: The user enters 3/4 in a recipe and get the three quarters nut fraction.

Recommended implementation: The afrc table maps sets of figures separated by slash (U+002F) or fraction (U+2044) characters to corresponding fraction glyphs in the font (GSUB lookup type 4).

Application interface: The application must define the full sequence of GIDs to be replaced. When the full sequence is found in the frac coverage table, the application passes the sequence to the afrc table and gets a new GID in return.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of all other features.

Tag: 'akhn'

Friendly name: Akhand

Function: Preferentially substitutes a sequence of characters with a ligature. This substitution is done irrespective of any characters that may precede or follow the sequence.

Example: In Devanagari script, the form Kssa is considered an Akhand character (meaning unbreakable), and the sequence Ka, Halant, Ssa should always produce the ligature Kssa, irrespective of characters that precede/follow the above given sequence.

Recommended implementation: This feature maps the sequences for generating Akhands defined in the given script, to the ligature they form (GSUB lookup type 4).

Application interface: The application passes the full sequence of GIDs. If these are located in the coverage table of the Akhand table, the application gets back the GID for the akhand ligature in return.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in most Indic scripts.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'blwf'

Friendly name: Below-base Forms

Function: Substitutes the below-base form of a consonant in conjuncts.

Example: In complex scripts like Oriya (Indic), the consonant Va has a below-base form that is used to generate conjuncts. Given a sequence Gha, Virama (Halant), Va; the below-base form of Va would be substituted to form the conjunct GhVa.

Recommended Implementation: This feature substitutes the GID sequence of virama (halant) followed by a consonant; by the GID of the below base form of the consonant (GSUB lookup type 4).

Application interface: In a conjunct formation sequence, if a consonant is identified as having a below base form, the application gets back the GID for this. The application may also choose to position this glyph if required, after this feature is called.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in a number of Indic scripts.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic and Indic-related scripts. For Indic scripts, the application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'blwm'

Friendly name: Below-base Mark Positioning

Function: Positions marks below base glyphs.

Example: In complex scripts like Gujarati (Indic), the vowel sign U needs to be positioned below base consonant/conjuncts that form the base glyph. This position can vary depending on the base glyph, as well as the presence/absence of other marks below the base glyph.

Recommended implementation: The blwm table provides positioning information (x,y) to enable mark positioning (GPOS lookup type 4, 5).

Application interface: The application must define the GIDs of the base glyphs below which marks need to be positioned, and the marks themselves. If these are located in the coverage table, the application passes the sequence to the blwm table and gets the positioning values (x,y) or positioning adjustments for the mark in return.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: Can be used to position default marks; or those that have been selected from a number of alternates based on contextual requirement using a feature like blws.

Tag: 'blws'

Friendly name: Below-base Substitutions

Function: Produces ligatures that comprise of base glyph and below-base forms.

Example: In the Malayalam script (Indic), the conjunct Kla, requires a ligature which is formed using the base glyph Ka and the below-base form of consonant La. This feature can also be used to substitute ligatures formed using base glyphs and below base matras in Indic scripts.

Recommended implementation: The blws table maps the identified conjunct forming sequences; or consonant vowel sign sequences; to their ligatures (GSUB lookup type 4).

Application interface: For GIDs found in the blws coverage table, the application passes the sequence of GIDs to the table, and gets back the GID for the ligature.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: This feature overrides the results of all other features.

Tag: 'calt'

Friendly name: Contextual Alternates

Function: In specified situations, replaces default glyphs with alternate forms which provide better joining behavior. Used in script typefaces which are designed to have some or all of their glyphs join.

Example: In Caflisch Script, o is replaced by o.alt2 when followed by an ascending letterform.

Recommended implementation: The calt table specifies the context in which each substitution occurs, and maps one or more default glyphs to replacement glyphs (GSUB lookup type 6).

Application interface: The application passes sequences of GIDs to the feature table, and gets back new GIDs. Full sequences must be passed.

UI suggestion: This feature should be active by default.

Script/language sensitivity: Not applicable to ideographic scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'case'

Friendly name: Case-Sensitive Forms

Function: Shifts various punctuation marks up to a position that works better with all-capital sequences or sets of lining figures; also changes oldstyle figures to lining figures. By default, glyphs in a text face are designed to work with lowercase characters. Some characters should be shifted vertically to fit the higher visual center of all-capital or lining text. Also, lining figures are the same height (or close to it) as capitals, and fit much better with all-capital text.

Example: The user selects a block of text and applies this feature. The dashes, bracketing characters, guillemet quotes and the like shift up to match the capitals, and oldstyle figures change to lining figures.

Recommended implementation: The font may implement this change by substituting different glyphs (GSUB lookup type 1) or by repositioning the original glyphs (GPOS lookup type 1).

Application interface: The application queries whether specific GIDs are found in the coverage table for the case feature. If so, it passes these IDs to the table and gets back either new GIDs or positional adjustments (XPlacement and YPlacement).

UI suggestion: It would be good to apply this feature (or turn it off) by default when the user changes case on a sequence of more than one character. Applications could also detect words consisting only of capitals, and apply this feature based on user preference settings.

Script/language sensitivity: Applies only to European scripts; particularly prominent in Spanish-language setting.

Feature interaction: This feature overrides the results of other features affecting the figures (e.g. onum and tnum).

Tag: 'ccmp'

Friendly name: Glyph Composition/Decomposition

Function: To minimize the number of glyph alternates, it is sometimes desirable to decompose the default glyph for a character into two or more glyphs. Additionally, it may be preferable to compose default glyphs for two or more characters into a single glyph for better glyph processing. This feature permits such composition/decomposition. The feature should be processed as the first feature processed, and should be processed only when it is called.

Example: In Syriac, the character 0x0732 is a combining mark that has a dot above AND a dot below the base character. To avoid multiple glyph variants to fit all base glyphs, the character is decomposed into two glyphs – a dot above and a dot below. These two glyphs can then be correctly placed using GPOS. In Arabic it might be preferred to combine the shadda with fatha (0x0651, 0x064E) into a ligature before processing shapes. This allows the font vendor to do special handling of the mark combination when doing further processing without requiring larger contextual rules.

Recommended implementation: The ccmp table maps the character sequence to its corresponding ligature (GSUB lookup type 4) or string of glyphs (GSUB lookup type 2). When using GSUB lookup type 4, sequences that are made up of larger number of glyphs must be placed before those that require fewer glyphs.

Application interface: For GIDs found in the ccmp coverage table, the application passes the sequence of GIDs to the table, and gets back the GID for the ligature, or GIDs for the multiple substitution.

UI suggestion: This feature should be on by default.

Script/language sensitivity: None.

Feature interaction: This feature needs to be implemented prior to any other feature.

Tag: 'cfar'

Friendly name: Conjunct Form After Ro

Function: Substitutes alternate below-base or post-base forms in Khmer script when occurring after conjoined Ro (“Coeng Ra”).

In Khmer script, the conjoined form of Ro re-orders to the left of the base consonant. It wraps under the base consonant, however, and so can interact typographically with below-base or post-base conjoined consonant and vowel forms. After the application has re-ordered the glyph for the conjoined Ro, it is no longer in the immediate context of glyphs for below-base or post-base forms. The application can detect this and apply this feature over the range for the below-base and post-base conjoining forms, triggering lookups to substitute alternate below-base or past-base forms as may be needed.

Example: In the Khmer script, Coeng Ro is denoted by a pre-base conjoining form, and Coeng Yo is denoted by a post-base conjoining form, but in both cases part of the form wraps under the base. The consonant cluster TRYo is denoted with an alternate form of Coeng Ya that descends lower so that it does not collide below the base with the Coeng Ro.

Recommended implementation: The cfar table maps below-base or post-base conjoining form into an alternate form (GSUB lookup type 1).

Application interface: For substitutions defined in the cfar table, the application passes the GID to the table and gets back the GID for an alternate form. The application is expected to apply this feature if a syllable contains a Coeng Ra followed by other conjoining consonants or vowels.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Khmer scripts.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Khmer script. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'chws'

Friendly name: Contextual Half-Width Spacing

Registered by: Adobe/W3C

Function: Contextually respaces glyphs designed to be set on full-em widths, fitting them onto individual half-width horizontal widths, to approximate more sophisticated text layout, such as what is described in Requirements for Japanese Text Layout (JLREQ [21]) or similar CJK text-layout specifications that expect half-width forms of characters whose default glyphs are full-width. This differs from 'halt' in that the respacing is contextual. This feature may be invoked to get better fit for punctuation or symbol glyphs without disrupting the monospaced alignment, such as for UIs or terminal apps.

Example: When FULLWIDTH RIGHT PARENTHESIS (U+FF09; "）") is followed by IDEOGRAPHIC COMMA (U+3001; "、"), the latter is respaced to remove half-em of width between them.

Recommended implementation: The font stores a set of adjustments for pairs of glyphs (GPOS lookup type 2 or 8). These may be stored as one or more tables matching left and right classes, &/or as individual pairs. Additional adjustments may be provided for larger sets of glyphs (e.g. triplets, quadruplets, etc.) to overwrite the results of pair kerns in particular combinations.

Application interface: The application passes a sequence of GIDs to the 'chws' table, and gets back adjusted positions (XPlacement, XAdvance, YPlacement, and YAdvance) for those GIDs. When using the type 2 lookup on a run of glyphs, it’s critical to remember to not consume the last glyph, but to keep it available as the first glyph in a subsequent run (this is a departure from normal lookup behavior).

UI suggestion: This feature would be off by default.

Script/language sensitivity: Used mostly in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g., 'fwid', 'halt', 'hwid', 'palt', 'pwid', 'qwid', 'twid'), which should be turned off when this feature is applied. It deactivates the 'kern' feature. See also 'vchw'.

Tag: 'cjct'

Friendly name: Conjunct Forms

Function: Produces conjunct forms of consonants in Indic scripts. This is similar to the Akhands feature, but is applied at a different sequential point in the process of shaping an Indic syllable.

Indic scripts are associated with conjoining-consonant behaviors, such as the use of 'half' forms. Some consonants may not have half forms and not exhibit conjoining behavior when combined with certain consonants, yet may conjoin as ligature forms with other consonants. Whether a given pair of consonants conjoins may impact other shaping behaviors for a syllable, such as where a re-ordering vowel mark or reph is placed. The Conjunct Forms feature can be used at a point in the shaping process immediately before final re-ordering such that the application can determine whether a re-ordering vowel or reph is placed in relation to the consonants.

More generally, the Akhands feature and Conjunct Forms feature can be used at two points in the shaping of an Indic syllable, together with other features such as Half Forms and Below Forms applied in between, providing the font developer with flexibility in how the shapes for Indic syllables are derived from the default glyphs for the character sequence.

Example: In Hindi (Devanagari script), the consonant cluster DGa is denoted with a conjunct ligature form.

Recommended implementation: The cjct table maps the sequence of a consonant (the nominal form) followed by a virama (halant) followed by a second consonant (the nominal form or a half form) to the corresponding conjunct form (GSUB lookup type 4).

Application interface: For substitution sequences defined in the cjct table, the application passes the sequence of GIDs to the table, and gets back the GID for the conjunct form.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Indic scripts that show similarity to Devanagari.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'clig'

Friendly name: Contextual Ligatures

Function: Replaces a sequence of glyphs with a single glyph which is preferred for typographic purposes. Unlike other ligature features, clig specifies the context in which the ligature is recommended. This capability is important in some script designs and for swash ligatures.

Example: The glyph for ft replaces the sequence f t in Bickham Script, except when preceded by an ascending letter.

Recommended implementation: The clig table maps sequences of glyphs to corresponding ligatures in a chained context (GSUB lookup type 8). Ligatures with more components must be stored ahead of those with fewer components in order to be found. The set of contextual ligatures will vary by design and script.

Application interface: For sets of GIDs found in the clig coverage table, the application passes the sequence of GIDs to the table and gets back a single new GID. Full sequences must be passed.

NOTE	This may include a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be active by default.

Script/language sensitivity: Applies to virtually all scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also dlig.

Tag: 'cpct'

Friendly name: Centered CJK Punctuation

Function: Centers specific punctuation marks for those fonts that do not include centered and non-centered forms.

Example: The user may invoke this feature in a Chinese font to get centered punctuation in case it is desired. Examples include U+3001 and U+3002, including their vertical variants, specifically U+FE11 and U+FE12, respectively.

Recommended implementation: The font specifies X- and Y-axis adjustments for a small number of full-width glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the cpct coverage table, the application passes the GIDs to the table and gets back positional adjustments (XPlacement, XAdvance, YPlacement and YAdvance).

UI suggestion: This feature would be off by default.

Script/language sensitivity: Used primarily in Chinese fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. tnum, fwid, hwid, halt, palt, twid), which should be turned off when it's applied.

Tag: 'cpsp'

Friendly name: Capital Spacing

Function: Globally adjusts inter-glyph spacing for all-capital text. Most typefaces contain capitals and lowercase characters, and the capitals are positioned to work with the lowercase. When capitals are used for words, they need more space between them for legibility and esthetics. This feature would not apply to monospaced designs. Of course the user may want to override this behavior in order to do more pronounced letterspacing for esthetic reasons.

Example: The user sets a title in all caps, and the Capital Spacing feature opens the spacing.

Recommended implementation: The cpsp table stores alternate advance widths for the capital letters covered, generally increasing them by a uniform percentage (GPOS lookup type 1).

Application interface: For GIDs found in the cpsp coverage table, the application passes a sequence of GIDs to the cpsp table and gets back a set of XPlacement and XAdvance adjustments. The application may rely on the user to apply this feature (e.g., by selecting text for a change to all-caps) or apply its own heuristics for recognizing words consisting of capitals.

UI suggestion: This feature should be on by default. Applications may want to allow the user to respecify the percentage to fit individual tastes and functions.

Script/language sensitivity: Should not be used in connecting scripts (e.g. most Arabic).

Feature interaction: May be used in addition to any other feature.

NOTE	This feature is additive with other GPOS features like kern.

Tag: 'cswh'

Friendly name: Contextual Swash

Function: This feature replaces default character glyphs with corresponding swash glyphs in a specified context. There may be more than one swash alternate for a given character.

Example: The user sets the word "HOLIDAY" in Poetica with this feature active, and is presented with a choice of three alternate forms appropriate for an initial H and one alternate appropriate for a medial L.

Recommended implementation: The cswh table maps GIDs for default forms to those for one or more corresponding swash forms in a chained context, which may require a selection from a set (GSUB lookup type 8). If several styles of swash are present across the font, the set of forms for each character should be ordered consistently.

Application interface: For GIDs found in the cswh coverage table, the application passes the GIDs to the swsh table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired.

UI suggestion: This feature should be inactive by default. When more than one GID is returned, an application could display the forms sequentially in context, or present a palette showing all the forms at once, or give the user a choice between these approaches. The application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: Does not apply to ideographic scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also swsh and init.

Tag: 'curs'

Friendly name: Cursive Positioning

Function: In cursive scripts like Arabic, this feature cursively positions adjacent glyphs.

Example: In Arabic, the Meem followed by a Reh are cursively positioned by overlapping the exit point of the Meem on the entry point of the Reh.

Recommended implementation: The curs table provides entry and exit points (x,y) for glyphs to be cursively positioned (GPOS lookup type 3).

Application interface: For GIDs located in the coverage table, the application gets back positioning point locations for the preceding and following glyphs.

UI suggestion: This feature could be made active or inactive by default, at the user's preference.

Script/language sensitivity: Can be used in any cursive script.

Feature interaction: None.

Tag: 'cv01' – 'cv99'

Friendly name: Character Variant 1 – Character Variant 99

Registered by: Microsoft

Function: A font may have stylistic-variant glyphs for one or more characters where the variations for one character are not systematically related to those for other characters. Or, a variation may exist for a character and its casing pair (or related pre-composed characters), but not be applicable to other unrelated characters. In some usage scenarios, it may be necessary to provide the application with control over glyph variations for different Unicode characters individually

The function of these features is similar to the function of the Stylistic Alternates feature (‘salt’) and the Stylistic Set features (see ‘ss01’ – ‘ss20’). Whereas the Stylistic Set features assume recurring stylistic variations that apply to a broad set of Unicode characters, these features are intended for scenarios in which particular characters have variations not applicable to a broad set of characters. The Stylistic Alternates feature provides access to glyph variants, but does not allow an application to control these on a character-by-character basis; the Character Variant features provide the greater granularity of control.

The function of these features is also related to that of the Localized Forms (‘locl’) feature, in that particular variations for a character may be preferred for particular languages. In practice, though, it may not be feasible to associate particular glyph variants with particular language systems for all the relevant languages; for example, the requirements of particular languages may not be known when a font is being developed.

The distinction between these features and the Stylistic set features is most easily understood in terms of variations applying to a single character versus variations applying across a range of characters. In practice, if a variation applies to a character in a bicameral script, then the casing-pair character may have the same variation. Also, Unicode includes pre-composed characters for certain base + mark combinations, hence a single abstract character may be incorporated into a number of Unicode characters. Therefore, a variation for a particular abstract character may be applicable to several related Unicode characters. The Character Variant features can be used for sets of related characters in these cases. The key distinction between such use and the intended use for Stylistic Set features is that a Character Variant feature should apply only to one character or a set of characters closely related in this way, while Stylistic Set features are intended for broader sets of characters.

Recommended implementation: A cvXX table maps the GID for the default form of a character to the GIDs for stylistic alternatives of that character. Each cvXX feature uses alternate (GSUB lookup type 3) substitutions. (If there is only one variant for a character, a single-substitution lookup, type 1, can also be used.)

The FeatureParams field of the Feature Table of these GSUB features may be set to 0, or to an offset to a Feature Parameters table. The Feature Parameters table for this feature is structured as follows:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format number is set to 0. |
| uint16 | featUiLabelNameId | The ‘name’ table name ID that specifies a string (or strings, for multiple languages) for a user-interface label for this feature. (May be NULL.) |
| uint16 | featUiTooltipTextNameId | The ‘name’ table name ID that specifies a string (or strings, for multiple languages) that an application can use for tooltip text for this feature. (May be NULL.) |
| uint16 | sampleTextNameId | The ‘name’ table name ID that specifies sample text that illustrates the effect of this feature. (May be NULL.) |
| uint16 | numNamedParameters | Number of named parameters. (May be zero.) |
| uint16 | firstParamUiLabelNameId | The first ‘name’ table name ID used to specify strings for user-interface labels for the feature parameters. (Must be zero if numParameters is zero.) |
| uint16 | charCount | The count of characters for which this feature provides glyph variants. (May be zero.) |
| uint24 | character[charCount] | The Unicode Scalar Value of the characters for which this feature provides glyph variants. |

The name ID provided by featUiLabelNameId is intended to provide a user-interface string for the feature; for example, “Capital-eng variants”. If set to NULL, no ‘name’ table string is used for the feature name.

The name ID provided by featUiTooltipTextNameId is intended to provide a user-interface string that provides a brief description of the feature that applications can use in popup “tooltip” help windows (e.g. “Select glyph variants for capital eng”).  If set to NULL, no ‘name’ table string is used for the feature “tooltip” help text.

The name ID provided by sampleTextNameId is intended to provide a string that can be used in a user-interface to illustrate the effect of the feature. If multiple characters are affected by the feature or if the feature affects a combining mark, it may not be evident to an application what string to use to present an illustrative sample; a ‘name’ table string can be provided for that purpose.

If numNamedParameters is non-zero, then firstParamUiLabelNameId and numNamedParameters specify a sequence of consecutive name IDs in the name table. These are used to provide user-interface strings for individual variants. The range of name IDs start at firstParamUiLabelNameId and end at firstParamUiLabelNameId + numNamedParameters – 1. Each of these name IDs corresponds to a feature parameter value used to select a particular GID from the array of GIDs returned by a type 3 substitution lookup; the relation between parameter values and name IDs is: name ID = parameter + firstParamUiLabelNameId - 1. The value of numNamedParameters should not exceed the number of alternate glyphs in lookups associated with the feature; note, however, that the number of GIDs in the returned array for a GSUB type 3 lookup should not be assumed to be equal to numNamedParameters: numNamedParameters should not be more than the number of GIDs in the array, but it may be less. If numNamedParameters is zero, then no ‘name’ table strings are associated with feature parameters.

The values of featUiLabelNameId, featUiTooltipTextNameId, sampleTextNameId and firstParamUiLabelNameId are expected to be in the font-specific name ID range (256–32767), though that is not a requirement in this Feature Parameters specification. The value of firstParamUiLabelNameId + numNamedParameters – 1 should not exceed 32767.

The user-interface label for the feature, for “tooltip” help text, or for feature parameters can be provided in multiple languages. English strings for each should be included as a fallback. A sample-text string likely would not need to be localized, though different sample-text strings for different UI languages can be used. If only one sample-text string is provided, applications may use it with any UI language.

The charCount field and character array are used to identify the Unicode characters for which this feature provides glyph variants. Applications can use this information in presenting user interface or for other purposes. Content of the character list is at the discretion of the font developer — the list may be exhaustive, representative, or empty — and does not affect the operation of the feature. If a font developer chooses not to include such information, charCount can be set to zero, in which case no character array can be included.

It is left to the discretion of application developers to determine whether or how to use the data provided in the feature parameters table or associated strings in the ‘name’ table.

NOTE	Since the strings provided using this feature parameter table will be used in application user interface, length is an important consideration. Strings should be as short as possible. It is recommended that the length of the feature or feature-parameter names be 25 characters or less, and that the length of “tooltip” help text be 250 characters or less.

Application interface: The application is responsible for counting and enumerating the number of features in the font with tag names of the format 'cv01' to 'cv99', and for presenting the user with an appropriate selection mechanism. The application is also responsible for interpreting any feature parameter tables (if the application developer wishes to use that data) and presenting referenced strings in user interface. For GIDs found in the cvXX coverage table, the application passes the GIDs to the cvXX table and gets back one or more new GIDs; the application selects one of the returned GIDs for display. The application may use an index parameter as an index into the array of returned GIDs.

UI suggestion: This feature should be off by default. An application can display glyph variants for a given character as a glyph palette in the user interface. If a Feature Parameters table is provided, the feature UI label or the feature and parameter UI labels (if provided) can be presented in the application user interface; or the sample-text string (if provided) can be presented in the application user interface.

Script/language sensitivity: None. For each respective[/distinct] 'cvXX' feature, the FeatureParams in the FeatureList must point to the same set of values.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. Note that after a cvXX feature has been applied, the user may wish to apply other typographic features, e.g. 'smcp'; font developers are responsible for ordering substitution lookups to obtain desired user experience. If it is to be used in conjunction with a complex script that requires obligatory substitution of ligatures or contextual forms, this feature should be applied before features for obligatory script behaviors.

Tag:'c2pc'

Friendly name: Petite Capitals From Capitals

Function: This feature turns capital characters into petite capitals. It is generally used for words which would otherwise be set in all caps, such as acronyms, but which are desired in petite-cap form to avoid disrupting the flow of text. See the pcap feature description for notes on the relationship of caps, smallcaps and petite caps.

Example: The user types UNICEF or NASA, applies c2pc and gets petite cap text.

Recommended implementation: The c2pc table maps capital glyphs to the corresponding petite cap forms (GSUB lookup type 1).

Application interface: For GIDs found in the c2pc coverage table, the application passes GIDs to the c2pc table, and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to scripts with both upper- and lowercase forms (e.g. Latin, Cyrillic, Greek).

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. Also see pcap.

Tag: 'c2sc'

Friendly name: Small Capitals From Capitals

Function: This feature turns capital characters into small capitals. It is generally used for words which would otherwise be set in all caps, such as acronyms, but which are desired in small-cap form to avoid disrupting the flow of text.

Example: The user types UNICEF or SCUBA, applies c2sc and gets small cap text.

Recommended implementation: The c2sc table maps capital glyphs to the corresponding small-cap forms (GSUB lookup type 1).

Application interface: For GIDs found in the c2sc coverage table, the application passes GIDs to the c2sc table, and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to bicameral scripts (i.e. those with case differences), such as Latin, Greek, Cyrillic, and Armenian.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. Also see smcp.

Tag: 'dist'

Friendly name: Distances

Function: Provides a means to control distance between glyphs.

Example: In the Devanagari (Indic) script, the distance between the vowel sign U and a consonant can be adjusted using this.

Recommended implementation: The dist table provides distances by which a glyph needs to move towards or away from another glyph (GPOS lookup type 2).

Application interface: For GIDs found in the dist coverage table, the application passes their GID to the table and gets back the distance that needs to be maintained between them.

UI suggestion: This feature could be made active or inactive by default, at the user's preference.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: None.

Tag: 'dlig'

Friendly name: Discretionary Ligatures

Function: Replaces a sequence of glyphs with a single glyph which is preferred for typographic purposes. This feature covers those ligatures which may be used for special effect, at the user's preference.

Example: The glyph for ct replaces the sequence of glyphs c t, or U+322E (Kanji ligature for "Friday") replaces the sequence U+91D1 U+66DC U+65E5.

Recommended implementation: The dlig table maps sequences of glyphs to corresponding ligatures (GSUB lookup type 4). Ligatures with more components must be stored ahead of those with fewer components in order to be found. The set of discretionary ligatures will vary by design and script.

Application interface: For sets of GIDs found in the dlig coverage table, the application passes the sequence of GIDs to the table and gets back a single new GID. Full sequences must be passed. This may include a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies to virtually all scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also clig.

Tag: 'dnom'

Friendly name: Denominators

Function: Replaces selected figures which follow a slash with denominator figures.

Example: In the string 11/17 selected by the user, the application turns the 17 into denominators when the user applies the fraction feature (frac).

Recommended implementation: The dnom table maps sets of figures and related characters to corresponding numerator glyphs in the font (GSUB lookup type 1).

Application interface: For GIDs found in the dnom coverage table, the application passes a GID to the table and gets back a new GID.

UI suggestion: This feature should normally be called by an application when the user applies the frac feature.

Script/language sensitivity: None.

Feature interaction: This feature supports frac. It may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'dtls'

Friendly name: Dotless forms

Function: This feature provides dotless forms for Math Alphanumeric characters, such as U+1D422 MATHEMATICAL BOLD SMALL I, U+1D423 MATHEMATICAL BOLD SMALL J, U+1D456 U+MATHEMATICAL ITALIC SMALL I, U+1D457 MATHEMATICAL ITALIC SMALL J, and so on.

The dotless forms are to be used as base forms for placing mathematical accents over them.

Example: In  formula dotted I is substituted with dotless form before attaching tilde accent on top of it.

Recommended implementation: Single substitution, for all dotted characters.

Application interface: Feature is invoked automatically by math layout handler depending on height of the base formula box.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of math formula. It should be applied in the appropriate contexts, as determined by math layout handler. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Applied to math formula layout.

Feature interaction: This feature is applied to individual glyphs during layout of math formula.

Tag: 'expt'

Friendly name: Expert Forms

Function: Like the JIS78 Forms described above, this feature replaces standard forms in Japanese fonts with corresponding forms preferred by typographers. Although most of the JIS78 substitutions are included, the expert substitution goes on to handle many more characters.

Example: The user would invoke this feature to replace kanji character U+5516 with U+555E.

Recommended implementation: The expt table maps many default (JIS90) GIDs to corresponding alternates (GSUB lookup type 1).

Application interface: For GIDs found in the expt coverage table, the application passes the GIDs to the table and gets back one new GID for each.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: Applications may choose to have this feature active or inactive by default, depending on their target markets.

Script/language sensitivity: Applies only to Japanese.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it's applied, except the palt, vpal, vert and vrt2 features, which may be used in addition.

Tag: 'falt'

Friendly name: Final Glyph on Line Alternates

Function: Replaces line final glyphs with alternate forms specifically designed for this purpose (they would have less or more advance width as need may be), to help justification of text.

Example: In the Arabic script, providing alternate forms for line final glyphs would result in better justification. e.g. replacing a long tailed Yeh-with-tail with one that has a slightly longer/shorter tail.

Recommended implementation: The falt table maps line final glyphs (in isolated or final forms) to their corresponding alternate forms (GSUB lookup type 3).

Application interface: For GIDs found in the falt coverage table, the application passes a GID to the table and gets back a new GID.

UI suggestion: This feature could be made active or inactive by default, at the user's preference.

Script/language sensitivity: Can be used in any cursive script.

Feature interaction: Would need to be applied last, only after all other features have been applied to the run.

Tag: 'fin2'

Friendly name: Terminal Form #2

Function: Replaces the Alaph glyph at the end of Syriac words with its appropriate form, when the preceding base character cannot be joined to, and that preceding base character is not a Dalath, Rish, or dotless Dalath-Rish.

Example: When an Alaph is preceded by a He, the Alaph would be replaced by an appropriate form. 
This feature is used only for the Syriac script alaph character.

Recommended implementation: The fin2 table maps default alphabetic forms to corresponding final forms (GSUB lookup type 5).

Application interface: The application is responsible for noting word boundaries. For GIDs in the middle of words and found in the fin2 coverage table, the application passes a GID to the feature and gets back a new GID.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Used only with the Syriac script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also init and fina.

Tag: 'fin3'

Friendly name: Terminal Form #3

Function: Replaces Alaph glyphs at the end of Syriac words when the preceding base character is a Dalath, Rish, or dotless Dalath-Rish.

Example: When an Alaph is preceded by a Dalath, the Alaph would be replaced by an appropriate form. 
This feature is used only for the Syriac script alaph character.

Recommended implementation: The fin3 table maps default alphabetic forms to corresponding final forms (GSUB lookup type 5).

Application interface: The application is responsible for noting word boundaries. For GIDs in the middle of words and found in the fin3 coverage table, the application passes a GID to the feature and gets back a new GID.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Used only with the Syriac script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also init and fina.

Tag: 'fina'

NOTE	This feature description was significantly revised in 2016.

Friendly name: Terminal Forms

Registered by: Microsoft/Adobe

Function: Replaces glyphs for characters that have applicable joining properties with an alternate form when occurring in a final context. This applies to characters that have one of the following Unicode Joining_Type property values:

- Right_Joining, if the characters are from a right-to-left script.

- Left_Joining, if the characters are from a left-to-right script.

- Dual_Joining.

Unicode Joining_Type property values are obtained from the Unicode Character Database (UCD) [22]. Specifically, Joining_Type property values are documented in the Unicode Character Database file for joining-script properties [20].

Example: In an Arabic-script font, the application would apply the 'fina' feature to the letter ARABIC LETTER WAW (U+0648 “و”) when it follows a left-joining character, thereby replacing the default “و” glyph with its right-joining, final form.

Recommended implementation: The 'fina' feature is used to map default forms to corresponding single-joining, final forms. This will usually be implemented using a single substitution (type 1) GSUB lookup, though contextual substitution GSUB lookups (types 5, 6 or 8) may also be appropriate.

Application interface: The application is responsible for parsing character strings and identifying which of the joining-related features — initial forms ('init'), medial forms ('medi'), terminal forms ('fina'), and isolated forms ('isol') — to apply to which GIDs, based on character Joining_Type properties. Additional factors, such as the presence of control characters, may also be considered. For GIDs to which the 'fina' feature is applied and that are found in the 'fina' coverage table, the application passes a GID to the lookup tables associate with the feature and gets back a new GID.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied by default in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Can be used in any script with joining behavior — that is, the scripts for which Joining_Type properties are given explicitly in Unicode Character Database file for joining-script properties [20].

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also ‘init’, ‘isol’, and ‘medi’.

Tag: 'flac'

Friendly name: Flattened accent forms

Function: This feature provides flattened forms of accents to be used over high-rise bases such as capitals. This feature should only change the shape of the accent and should not move it in the vertical or horizontal direction. Moving of the accents is done by the math handling client. Accents are flattened by the Math engine if their base is higher than MATH.MathConstants. FlattenedAccentBaseHeight.

Example: Depending on the font parameters, in   formula tilde may be used in default form and in  it may use flattened form

Recommended implementation: Single substitution, replacing ascent glyph with its flattened form. See MATH table specification for details.

Application interface: Feature is invoked automatically by math layout handler depending on height of the base formula box.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of math formula. It should be applied in the appropriate contexts, as determined by math layout handler. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Applied to math formula layout.

Feature interaction: This feature is applied to individual glyphs during layout of math formula.

Tag: 'frac'

Friendly name: Fractions

Function: Replaces figures separated by a slash with 'common' (diagonal) fractions.

Example: The user enters 3/4 in a recipe and gets the three quarters fraction.

Recommended implementation: The frac table maps sets of figures separated by slash or fraction characters to corresponding fraction glyphs in the font. These may be precomposed fractions (GSUB lookup type 4) or arbitrary fractions (GSUB lookup type 1).

Application interface: The application must define the full sequence of GIDs to be replaced, based on user input (i.e. user selection determines the string's delimitation). When the full sequence is found in the frac coverage table, the application passes the sequence to the frac table and gets a new GID in return. When the frac table does not contain an exact match, the application performs two steps. First, it uses the numr feature (see below) to replace figures (as used in the numr coverage table) preceding the slash with numerators, and to replace the typographic slash character (U+002F) with the fraction slash character (U+2044). Second, it uses the dnom feature (see below) to replace all remaining figures (as listed in the dnom coverage table) with denominators.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None.

Feature interaction: This feature may require the application to call the numr and dnom features. It may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'fwid'

Friendly name: Full Widths

Function: Replaces glyphs set on other widths with glyphs set on full (usually em) widths. In a CJKV font, this may include "lower ASCII" Latin characters and various symbols. In a European font, this feature replaces proportionally-spaced glyphs with monospaced glyphs, which are generally set on widths of 0.6 em.

Example: The user may invoke this feature in a Japanese font to get full monospaced Latin glyphs instead of the corresponding proportionally-spaced versions.

Recommended implementation: The font may contain alternate glyphs designed to be set on full widths (GSUB lookup type 1), or it may specify alternate (full-width) metrics for the proportional glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the fwid coverage table, the application passes the GIDs to the table and gets back either new GIDs or positional adjustments (XPlacement and XAdvance).

UI suggestion: This feature would normally be off by default.

Script/language sensitivity: Applies to any script which can use monospaced forms.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. tnum, halt, hwid, palt, pwid, qwid and twid), which should be turned off when it's applied. It deactivates the kern feature.

Tag: 'half'

Friendly name: Half Forms

Function: Produces the half forms of consonants in Indic scripts.

Example: In Hindi (Devanagari script), the conjunct KKa, obtained by doubling the Ka, is denoted with a half form of Ka followed by the full form.

Recommended implementation: The half table maps the sequence of a consonant followed by a virama (halant) to its half form (GSUB lookup type 4).

Application interface: For substitution sequences defined in the half table [consonant followed by the virama (halant)], the application passes the sequence of GIDs to the table, and gets back the GID for the half form.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Indic scripts that show similarity to Devanagari.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'haln'

Friendly name: Halant Forms

Function: Produces the halant forms of consonants in Indic scripts.

Example: In Sanskrit (Devanagari script), syllable final consonants are frequently required in their halant form.

Recommended implementation: The haln table maps the sequence of a consonant followed by a virama (halant) to its halant form (GSUB lookup type 4).

Application interface: For substitutions defined in the halant table, the application passes the sequence of GIDs to the feature (essentially the consonant and virama), and gets back the GID for the halant form.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: This feature overrides the results of all other features.

Tag: 'halt'

Friendly name: Alternate Half Widths

Function: Respaces glyphs designed to be set on full-em widths, fitting them onto half-em widths. This differs from hwid in that it does not substitute new glyphs.

Example: The user may invoke this feature in a CJKV font to get better fit for punctuation or symbol glyphs without disrupting the monospaced alignment.

Recommended implementation: The font specifies alternate metrics for the full-width glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the halt coverage table, the application passes the GIDs to the table and gets back positional adjustments (XPlacement, XAdvance, YPlacement and YAdvance).

UI suggestion: In general, this feature should be off by default. Different behavior should be used, however, in applications that conform to Requirements for Japanese Text Layout (JLREQ [21]) or similar CJK text-layout specifications that expect half-width forms of characters whose default glyphs are full-width. Such implementations should turn this feature on by default, or should selectively apply this feature to particular characters that require special treatment for CJK text-layout purposes, such as brackets, punctuation, and quotation marks.

Script/language sensitivity: Used only in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. tnum, fwid, hwid, palt, twid), which should be turned off when it's applied. It deactivates the kern feature. See also vhal.

Tag: 'hist'

Friendly name: Historical Forms

Function: Some letterforms were in common use in the past, but appear anachronistic today. The best-known example is the long form of s; others would include the old Fraktur k. Some fonts include the historical forms as alternates, so they can be used for a 'period' effect. This feature replaces the default (current) forms with the historical alternates. While some ligatures are also used for historical effect, this feature deals only with single characters.

Example: The user applies this feature in Adobe Jenson to get the archaic forms of M, Q and Z.

Recommended implementation: The hist table maps default forms to corresponding historical forms (GSUB lookup type 1).

Application interface: For GIDs found in the hist coverage table, the application passes the GIDs to the hist table and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'hkna'

Friendly name: Horizontal Kana Alternates

Function: Replaces standard kana with forms that have been specially designed for only horizontal writing. This is a typographic optimization for improved fit and more even color. Also see vkna.

Example: Standard full-width kana (hiragana and katakana) are replaced by forms that are designed for horizontal use.

Recommended implementation: The font includes a set of specially-designed glyphs, listed in the hkna coverage table. The hkna feature maps the standard full-width forms to the corresponding special horizontal forms (GSUB lookup type 1).

Application interface: For GIDs found in the hkna coverage table, the application passes GIDs to the feature, and gets back new GIDs.

UI suggestion: This feature would be off by default.

Script/language sensitivity: Applies only to fonts that support kana (hiragana and katakana).

Feature interaction: This feature may be used with the kern feature. Since it is for horizontal use, features applying to vertical behaviors (e.g. vkna, vert, vrt2 or vkrn) do not apply.

Tag: 'hlig'

Friendly name: Historical Ligatures

Function: Some ligatures were in common use in the past, but appear anachronistic today. Some fonts include the historical forms as alternates, so they can be used for a 'period' effect. This feature replaces the default (current) forms with the historical alternates.

Example: The user applies this feature using Palatino Linotype, and historic ligatures are formed for all long s forms, including: long s+t, long s+b, long s+h, long s+k, and several others.

Recommended implementation: The hlig table maps default ligatures and character combinations to corresponding historical ligatures (GSUB lookup type 1).

Application interface: For GIDs found in the hlig coverage table, the application passes the GIDs to the hlig table and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of all other features.

Tag: 'hngl' (DEPRECATED in 2016)

Friendly name: Hangul

Function: Replaces hanja (Chinese-style) Korean characters with the corresponding hangul (syllabic) characters. This effectively reverses the standard input methods, in which hangul are entered and replaced by hanja. Many of these substitutions are one-to-one (GSUB lookup type 1), but hanja substitution often requires the user to choose from several possible hangul characters (GSUB lookup type 3).

Example: The user may call this feature to get U+AC00 from U+4F3D.

Recommended implementation: This table associates each hanja character in the font with one or more hangul characters. The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. As in any one-from-many substitution, alternates should be ordered consistently across a family, so that those alternates can work correctly when switching between family members.

Application interface: For GIDs found in the hngl coverage table, the application passes the GIDs to the table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired.

NOTE	This is a change of semantic value. Besides the original character codes (when entered as hanja), the application should store the code for the new character.

UI suggestion: This feature should be inactive by default. The application may note the user's choice when selecting from multiple hangul, and offer it as a default the next time the source hanja character is encountered. In the absence of such prior information, the application may assume that the first hangul in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: Korean only.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it's applied, except the palt, vert and vrt2 may be used in addition.

Tag: 'hojo'

Friendly name: Hojo Kanji Forms (JIS X 0212-1990 Kanji Forms)

Registered by: Adobe

Function: The JIS X 0212-1990 (aka, "Hojo Kanji") and JIS X 0213:2004 character sets overlap significantly. In some cases their prototypical glyphs differ. When building fonts that support both JIS X 0212-1990 and JIS X 0213:2004 (such as those supporting the Adobe-Japan 1-6 character collection), it is recommended that JIS X 0213:2004 forms be preferred as the encoded form. The 'hojo' feature is used to access the JIS X 0212-1990 glyphs for the cases when the JIS X 0213:2004 form is encoded.

Example: The glyph is replaced by the glyph .

Recommended implementation: One-for-one substitution of JIS X 0213:2004 glyphs by the corresponding JIS X 0212-1990 glyph.

Application interface: For GIDs found in the hojo coverage table, the application passes the GIDs to the table and gets back one new GID for each.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Used only with Kanji script.

Feature interaction: This feature is exclusive with jp78, jp83, jp90, nlck and similar features. It can be combined with the palt, vpal, vert and vrt2 features.

Tag: 'hwid'

Friendly name: Half Widths

Function: Replaces glyphs on proportional widths, or fixed widths other than half an em, with glyphs on half-em (en) widths. Many CJKV fonts have glyphs which are set on multiple widths; this feature selects the half-em version. There are various contexts in which this is the preferred behavior, including compatibility with older desktop documents.

Example: The user may replace a proportional Latin glyph with the same character set on a half-em width.

Recommended implementation: The font may contain alternate glyphs designed to be set on half-em widths (GSUB lookup type 1), or it may specify alternate metrics for the original glyphs (GPOS lookup type 1) which adjust their spacing to fit in half-em widths.

Application interface: For GIDs found in the hwid coverage table, the application passes the GIDs to the table and gets back either new GIDs or positional adjustments (XPlacement and XAdvance).

UI suggestion: This feature would normally be off by default.

Script/language sensitivity: Generally used only in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. tnum, fwid, halt, qwid and twid), which should be turned off when it's applied. It deactivates the kern feature.

Tag: 'init'

NOTE	This feature description was significantly revised in 2016.

Friendly name: Initial Forms

Registered by: Microsoft/Adobe

Function: Replaces glyphs for characters that have applicable joining properties with an alternate form when occurring in an initial context. This applies to characters that have one of the following Unicode Joining_Type property values:

- Right_Joining, if the characters are from a left-to-right script.

- Left_Joining, if the characters are from a right-to-left script.

- Dual_Joining.

Unicode Joining_Type property values are obtained from the Unicode Character Database (UCD) [22]. Specifically, Joining_Type property values are documented in the Unicode Character Database file for joining-script properties [20].

Example: In an Arabic-script font, the application would apply the 'init' feature to the letter ARABIC LETTER SEEN (U+0633 “س”) when it precedes a right-joining character, thereby replacing the default “س” glyph with its left-joining, initial form.

Recommended implementation: The 'init' feature is used to map default forms to corresponding single-joining, initial forms. This will usually be implemented using a single substitution (type 1) GSUB lookup, though contextual substitution GSUB lookups (types 5, 6 or 8) may also be appropriate.

Application interface: The application is responsible for parsing character strings and identifying which of the joining-related features — initial forms ('init'), medial forms ('medi'), terminal forms ('fina'), and isolated forms ('isol') — to apply to which GIDs, based on character Joining_Type properties. Additional factors, such as the presence of control characters, may also be considered. For GIDs to which the 'init' feature is applied and that are found in the 'init' coverage table, the application passes a GID to the lookup tables associate with the feature and gets back a new GID.

UI suggestion: This feature should be active by default.

Script/language sensitivity: Can be used in any script with joining behavior — that is, the scripts for which Joining_Type properties are given explicitly in Unicode Character Database file for joining-script properties [20].

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also ‘fina’, ‘isol’, and ‘medi’.

Tag: 'isol'

NOTE	This feature description was significantly revised in 2016.

Friendly name: Isolated Forms

Registered by: Microsoft

Function: Replaces glyphs for characters that have applicable joining properties with an alternate form when occurring in a isolate (non-joining) context. This applies to characters that have one of the following Unicode Joining_Type property values:

- Right_Joining.

- Left_Joining.

- Dual_Joining.

- Non_Joining, if the characters are from a script with joining behavior.

Unicode Joining_Type property values are obtained from the Unicode Character Database (UCD) [22]. Specifically, Joining_Type property values are documented in the Unicode Character Database file for joining-script properties [20]. Scripts that have joining behavior are those scripts with character properties given explicitly in [20].

Note that, in many fonts that support the relevant scripts, this feature may not be implemented since the default forms of the relevant characters are the isolated forms. In some fonts, this feature may involve contextual substitution based on the specific, isolated context.

Example: In an Arabic-script font, the application would apply the 'isol' feature to the letter ARABIC LETTER HEH (U+0647 “ه”) when not adjacent to any joining character, thereby potentially replacing the default “ه” glyph with a special, isolated form (likely, a contextual and language-specific substitution, substituting one isolated form for another).

Recommended implementation: The 'isol' feature is used to map default forms to alternate non-joining, isolate forms. This will usually be implemented using a single substitution (type 1) GSUB lookup or, often, a contextual substitution GSUB lookup (types 5, 6 or 8).

Application interface: The application is responsible for parsing character strings and identifying which of the joining-related features — initial forms ('init'), medial forms ('medi'), terminal forms ('fina'), and isolated forms ('isol') — to apply to which GIDs, based on character Joining_Type properties. Additional factors, such as the presence of control characters, may also be considered. For GIDs to which the 'isol' feature is applied and that are found in the 'isol' coverage table, the application passes a GID to the lookup tables associated with the feature and gets back a new GID.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied by default in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Can be used in any script with joining behavior — that is, the scripts for which Joining_Type properties are given explicitly in Unicode Character Database file for joining-script properties [20].

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also ‘fina’, ‘init’, and ‘medi’.

Tag: 'ital'

Friendly name: Italics

Function: Some fonts (such as Adobe's Pro Japanese fonts) will have both Roman and Italic forms of some characters in a single font. This feature replaces the Roman glyphs with the corresponding Italic glyphs.

Example: The user would apply this feature to replace B with B.

Recommended implementation: The ital table maps the Roman forms in a font to the corresponding Italic forms (GSUB lookup type 1).

Application interface: For GIDs found in the ital coverage table, the application passes the GIDs to the table and gets back one new GID for each.

UI suggestion: When a user selects text and applies an Italic style, an application should check for this feature and use it if present.

Script/language sensitivity: Applies mostly to Latin; but it should be noted that many non-Latin fonts contain Latin as well.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. In CJKV fonts it should activate the kern feature (which would be on anyway in other scripts).

Tag: 'jalt'

Friendly name: Justification Alternates

Function: Improves justification of text by replacing glyphs with alternate forms specifically designed for this purpose (they would have less or more advance width as need may be).

Example: In the Arabic script, providing alternate forms for line final glyphs would result in better justification and reduce the use of tatweels (Kashidas). eg. replacing a Swash Kaf with an alternate form.

Recommended implementation: The jalt table maps the initial, medial, final or isolated forms to their corresponding alternate forms (GSUB lookup type 3).

Application interface: The application is responsible for noting line ends/boundaries. For GIDs found in the jalt coverage table, the application passes a GID to the feature and gets back a new GID.

UI suggestion: This feature could be made active or inactive by default, at the user's preference.

Script/language sensitivity: Can be used in any cursive script.

Feature interaction: If the font contains init, medi, fina, isol features, these need to be called prior to calling this feature.

Tag: 'jp78'

Friendly name: JIS78 Forms

Function: This feature replaces default (JIS90) Japanese glyphs with the corresponding forms from the JIS C 6226-1978 (JIS78) specification.

Example: The user would invoke this feature to replace kanji character U+5516 with U+555E.

Recommended implementation: When JIS90 glyphs correspond to JIS78 forms, the jp78 table maps each of those glyphs to their alternates. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions.

Application interface: For GIDs found in the jp78 coverage table, the application passes the GIDs to the table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired. The application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to Japanese.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it's applied, except the palt, vpal, vert and vrt2 features, which may be used in addition.

Tag: 'jp83'

Friendly name: JIS83 Forms

Function: This feature replaces default (JIS90) Japanese glyphs with the corresponding forms from the JIS X 0208-1983 (JIS83) specification.

Example: Because of the Han unification in Unicode, there are no JIS83 glyphs which have distinct Unicode values, so the substitution cannot be described specifically.

Recommended implementation: When JIS90 glyphs correspond to JIS83 forms, the jp83 table maps each of those glyphs to their alternates (GSUB lookup type 1).

Application interface: For GIDs found in the jp83 coverage table, the application passes the GIDs to the table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to Japanese.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it's applied, except the palt, vpal, vert and vrt2 features, which may be used in addition.

Tag: 'jp90'

Friendly name: JIS90 Forms

Function: This feature replaces Japanese glyphs from the JIS78 or JIS83 specifications with the corresponding forms from the JIS X 0208-1990 (JIS90) specification.

Example: The user would invoke this feature to replace kanji character U+555E with U+5516.

Recommended implementation: The jp90 table maps each JIS78 and JIS83 form in a font to JIS90 forms (GSUB lookup type 1). The application stores a record of any simplified forms which resulted from substitutions (the jp78 or jp83 features); for such forms, applying the jp90 feature undoes the previous substitution. When there is no record of a substitution, the application uses the jp90 table to get back to the default form.

Application interface: For GIDs found in the jp90 coverage table, the application passes the GIDs to the table and gets back one new GID for each.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to Japanese.

Tag: 'jp04'

Friendly name: JIS2004 Forms

Registered by: Adobe

Function: The National Language Council (NLC) of Japan has defined new glyph shapes for a number of JIS characters, which were incorporated into JIS X 0213:2004 as new prototypical forms. The 'jp04' feature is a subset of the 'nlck' feature, and is used to access these prototypical glyphs in a manner that maintains the integrity of JIS X 0213:2004.

Example: The glyph is replaced by the glyph .

Recommended implementation: One-for-one substitution of non-JIS X 0213:2004 glyphs by the corresponding JIS X 0213:2004 glyph.

Application interface: For GIDs found in the jp04 coverage table, the application passes the GIDs to the table and gets back one new GID for each.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Used only with Kanji script.

Feature interaction: This feature is exclusive with jp78, jp83, jp90, nlck and similar features. It can be combined with the palt, vpal, vert and vrt2 features.

Tag: 'kern'

Friendly name: Kerning

Function: Adjusts amount of space between glyphs, generally to provide optically consistent spacing between glyphs. Although a well-designed typeface has consistent inter-glyph spacing overall, some glyph combinations require adjustment for improved legibility. Besides standard adjustment in the horizontal direction, this feature can supply size-dependent kerning data via device tables, "cross-stream" kerning in the Y text direction, and adjustment of glyph placement independent of the advance adjustment.

NOTE	This feature may apply to runs of more than two glyphs, and would not be used in monospaced fonts. This feature does not apply to text set vertically.

Example: The o is shifted closer to the T in the combination "To".

Recommended implementation: The font stores a set of adjustments for pairs of glyphs (GPOS lookup type 2 or 8). These may be stored as one or more tables matching left and right classes, &/or as individual pairs. Additional adjustments may be provided for larger sets of glyphs (e.g. triplets, quadruplets, etc.) to overwrite the results of pair kerns in particular combinations.

Application interface: The application passes a sequence of GIDs to the kern table, and gets back adjusted positions (XPlacement, XAdvance, YPlacement and YAdvance) for those GIDs. When using the type 2 lookup on a run of glyphs, it's critical to remember to not consume the last glyph, but to keep it available as the first glyph in a subsequent run (this is a departure from normal lookup behavior).

UI suggestion: This feature should be active by default for horizontal text setting. Applications may wish to allow users to add further manually-specified adjustments to suit specific needs and tastes.

Script/language sensitivity: None.

Feature interaction: If 'kern' is activated, 'palt' must also be activated if it exists. (If 'palt' is activated, there is no requirement that 'kern' must also be activated.) May be used in addition to any other feature except those which result in fixed (uniform) advance widths (e.g. fwid, halt, hwid, qwid and twid).

Tag: 'lfbd'

Friendly name: Left Bounds

Function: Aligns glyphs by their apparent left extents at the left ends of horizontal lines of text, replacing the default behavior of aligning glyphs by their origins. This feature is called by the Optical Bounds ( opbd) feature above.

Example: Succeeding lines beginning with T, D and W would shift to the left by varying amounts when the text is left-justified and this feature is applied.

Recommended implementation: Values for affected glyphs describe the amount by which the placement and advance width should be altered (GPOS lookup type 1).

Application interface: For GIDs found in the lfbd coverage table, the application passes a GID to the table and gets back a new XPlacement and XAdvance value.

UI suggestion: This feature is called by an application when the user invokes the opbd feature.

Script/language sensitivity: None.

Feature interaction: Should not be applied to glyphs which use fixed-width features (e.g. fwid, halt, hwid, qwid and twid) or vertical features (e.g. vert, vrt2, vpal, valt and vhal). Is called by the opbd feature.

Tag: 'liga'

Friendly name: Standard Ligatures

Function: Replaces a sequence of glyphs with a single glyph which is preferred for typographic purposes. This feature covers the ligatures which the designer/manufacturer judges should be used in normal conditions.

Example: The glyph for ffl replaces the sequence of glyphs f f l.

Recommended implementation: The liga table maps sequences of glyphs to corresponding ligatures (GSUB lookup type 4). Ligatures with more components must be stored ahead of those with fewer components in order to be found. The set of standard ligatures will vary by design and script.

Application interface: For sets of GIDs found in the liga coverage table, the application passes the sequence of GIDs to the table and gets back a single new GID. Full sequences must be passed.

UI suggestion: This feature serves a critical function in some contexts, and should be active by default.

Script/language sensitivity: Applies to virtually all scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'ljmo'

Friendly name: Leading Jamo Forms

Function: Substitutes the leading jamo form of a cluster.

Example: In Hangul script, the jamo cluster is composed of three parts (leading consonant, vowel, and trailing consonant). When a sequence of leading class jamos are found, their combined leading jamo form is substituted.

Recommended implementation: The ljmo table maps the sequence required to convert a series of jamos into its leading jamo form (GSUB lookup type 4).

Application interface: For substitutions defined in the ljmo table, the application passes the sequence of GIDs to the feature, and gets back the GID for the leading jamo form.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required for Hangul script when Ancient Hangul writing system is supported.

Feature interaction: This feature overrides the results of all other features.

Tag: 'lnum'

Friendly name: Lining Figures

Function: This feature changes selected non-lining figures to lining figures.

Example: The user invokes this feature in order to get lining figures, which fit better with all-capital text. Various characters designed to be used with figures may also be covered by this feature. In cases where lining figures are the default form, this feature would undo previous substitutions.

Recommended implementation: The lnum table maps each oldstyle figure, and any associated characters to the corresponding lining form (GSUB lookup type 1). If the default figures are non-lining, they too are mapped to the corresponding lining form.

Application interface: For GIDs found in the lnum coverage table, the application passes a GID to the lnum table and gets back a new GID. Even if the current figures resulted from an earlier substitution, it may not be correct to simply revert to the original GIDs, because of interaction with the figure width features, so it's best to use this table.

UI suggestion: This feature should be inactive by default. Users can switch between the lining and oldstyle sets by turning this feature on or off.

NOTE	This feature is distinct from the figure width features (pnum and tnum). When the user invokes this feature, the application may wish to inquire whether a change in width is also desired.

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of the Oldstyle Figures feature ( onum).

Tag: 'locl'

Friendly name: Localized Forms

Function: Many scripts used to write multiple languages over wide geographical areas have developed localized variant forms of specific letters, which are used by individual literary communities. For example, a number of letters in the Bulgarian and Serbian alphabets have forms distinct from their Russian counterparts and from each other. In some cases the localized form differs only subtly from the script 'norm', in others the forms are radically distinct. This feature enables localized forms of glyphs to be substituted for default forms.

Example: The user applies this feature to text to enable localized Bulgarian forms of Cyrillic letters; alternatively, the feature might enable localized Russian forms in a Bulgarian manufactured font in which the Bulgarian forms are the default characters.

Recommended implementation: For a given Unicode value, the font contains glyphs for two or more locales. The locl table maps GIDs for default forms to GIDs for corresponding localized alternatives. These are one-to-one substitutions (GSUB lookup type 1).

Application interface: Localized forms are associated with specific languages and are activated by language tags. Which glyph is used as the localized form should be determined by the language the user has specified. The user can switch localized forms by selecting a new language, or may enable default forms by switching off the locl feature.

UI suggestion: This feature should be active by default.

Script/language sensitivity: Applies to all scripts and languages; but of course behavior differs by script and language.

Feature interaction: This feature can be used in combination with any other feature. It replaces and extends the earlier locale-specific tags zhcn, zhtw, jajp, kokr and vivn which had been defined for CJKV scripts.

Tag: 'ltra'

Friendly name: Left-to-right glyph alternates

Registered by: Adobe

Function: This feature applies glyphic variants (other than mirrored forms) appropriate for left-to-right text. (For mirrored forms, see 'ltrm'.)

Recommended implementation: These are required to be glyph substitutions, and it is recommended that they be one-to-one (GSUB lookup type 1).

Application interface: See section “Left-to-right and right-to-left text” in subclause 6.1.4 (Text processing with OFF Layout).

UI suggestion: None

Script/language sensitivity: Left-to-right runs of text.

Feature interaction: This feature is to be applied simultaneously with other pre-shaping features such as 'ccmp' and 'locl'.

Tag: 'ltrm'

Friendly name: Left-to-right mirrored forms

Registered by: Adobe

Function: This feature applies mirrored forms appropriate for left-to-right text. (For left-to-right glyph alternates, see 'ltra'.)

Example: The Old South Arabian script is a case of a strong right-to-left script that can have lines laid out left-to-right, in which case some glyphs would need to be mirrored with the ‘ltrm’ feature.

Recommended implementation: These are required to be glyph substitutions, and it is recommended that they be one-to-one (GSUB lookup type 1).

Application interface: See section “Left-to-right and right-to-left text” in subclause 6.1.4 (Text processing with OFF Layout).

UI suggestion: None

Script/language sensitivity: Left-to-right runs of text, also see Example above.

Feature interaction: This feature is to be applied simultaneously with other pre-shaping features such as 'ccmp' and 'locl'.

Tag: 'mark'

Friendly name: Mark Positioning

Function: Positions mark glyphs with respect to base glyphs.

Example: In the Arabic script, positioning the Hamza above the Yeh.

Recommended implementation: This feature may be implemented as a MarkToBase Attachment lookup (GPOS LookupType = 4) or a MarkToLigature Attachment lookup (GPOS LookupType = 5).

Application interface: For GIDs found in the mark coverage table, the application gets back the positioning or position adjustment values for the mark glyph.

UI suggestion: This feature should be active by default.

Script/language sensitivity: None.

Feature interaction: None.

Tag: 'med2'

Friendly name: Medial Forms #2

Function: Replaces Alaph glyphs in the middle of Syriac words when the preceding base character can be joined to.

Example: When an Alaph is preceded by a Heth, the Alaph would be replaced by an appropriate form. 
This feature is used only for the Syriac script alaph character.

Recommended implementation: The med2 table maps default alphabetic forms to corresponding medial forms (GSUB lookup type 5).

Application interface: The application is responsible for noting word boundaries. For GIDs in the middle of words and found in the med2 coverage table, the application passes a GID to the feature and gets back a new GID.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Used only with the Syriac script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also init and fina.

Tag: 'medi'

NOTE	This feature description was significantly revised in 2016.

Friendly name: Medial Forms

Registered by: Microsoft/Adobe

Function: Replaces glyphs for characters that have applicable joining properties with an alternate form when occurring in a medial context. This applies to characters that have the Unicode Joining_Type property value Dual_Joining.

Unicode Joining_Type property values are obtained from the Unicode Character Database (UCD) [22]. Specifically, Joining_Type property values are documented in the Unicode Character Database file for joining-script properties [20].

Example: In an Arabic-script font, the application would apply the 'medi' feature to the letter ARABIC LETTER QAF (U+0642 “ق”) when it follows a left-joining character and precedes a right-joining character, thereby replacing the default “ق” glyph with its dual-joining, medial form.

Recommended implementation: The 'medi' feature is used to map default forms to corresponding dual-joining, medial forms. This will usually be implemented using a single substitution (type 1) GSUB lookup, though contextual substitution GSUB lookups (types 5, 6 or 8) may also be appropriate.

Application interface: The application is responsible for parsing character strings and identifying which of the joining-related features — initial forms ('init'), medial forms ('medi'), terminal forms ('fina'), and isolated forms ('isol') — to apply to which GIDs, based on character Joining_Type properties. Additional factors, such as the presence of control characters, may also be considered. For GIDs to which the 'medi' feature is applied and that are found in the 'medi' coverage table, the application passes a GID to the lookup tables associate with the feature and gets back a new GID.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied by default in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Can be used in any script with joining behavior — that is, the scripts for which Joining_Type properties are given explicitly in Unicode Character Database file for joining-script properties [20].

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also ‘fina’, ‘init’, and ‘isol’.

Tag: 'mgrk'

Friendly name: Mathematical Greek

Function: Replaces standard typographic forms of Greek glyphs with corresponding forms commonly used in mathematical notation (which are a subset of the Greek alphabet).

Example: The user applies this feature to U+03A3 (Sigma), and gets U+2211 (summation).

Recommended implementation: The mgrk table maps Greek glyphs to the corresponding forms used for mathematics (GSUB lookup type 1).

Application interface: For GIDs found in the mgrk coverage table, the application passes a GID to the feature table and gets back a new GID.

NOTE	This is a change of semantic value. Besides the original character codes, the application should store the code for the new character.

UI suggestion: This feature should be off by default in most applications. Math-oriented applications may want to activate this feature by default.

Script/language sensitivity: Could apply to any font which includes coverage for the Greek script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'mkmk'

Friendly name: Mark to Mark Positioning

Function: Positions marks with respect to other marks. Required in various non-Latin scripts like Arabic.

Example: In Arabic, the ligaturised mark Ha with Hamza above it; can also be obtained by positioning these marks relative to one another.

Recommended implementation: This feature may be implemented as a MarkToMark Attachment lookup (GPOS lookup type 6).

Application interface: The application gets back positioning values or positional adjustments for marks.

UI suggestion: This feature should be active by default.

Script/language sensitivity: None.

Feature interaction: None.

Tag: 'mset'

Function: Positions Arabic combining marks in fonts for Windows 95 using glyph substitution

Example: In Arabic, the Hamza is positioned differently when placed above a Yeh Barree as compared to the Alef.

Tag: 'nalt'

Friendly name: Alternate Annotation Forms

Function: Replaces default glyphs with various notational forms (e.g. glyphs placed in open or solid circles, squares, parentheses, diamonds or rounded boxes). In some cases an annotation form may already be present, but the user may want a different one.

Example: The user invokes this feature to get U+3200 (the circled form of 'ga') from U+3131 (hangul 'ga').

Recommended implementation: The nalt table maps GIDs for various standard forms to one or more corresponding annotation forms. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. If more than one form is present, the set of forms for each character should be ordered consistently - both within the font and across the family.

Application interface: For GIDs found in the nalt coverage table, the application passes a GID and gets back a set of new GIDs, then stores the one selected by the user.

UI suggestion: This feature should be inactive by default. The application must provide a means for the user to select the desired form from the set returned by the table. It can note the position of the selected form in a set of alternates, and offer the glyph at that position as the default selection the next time this feature is invoked. In the absence of such prior information, the application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: Used mostly in CJKV fonts, but can apply to European scripts.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it's applied, except the vert and vrt2 features, which may be used in addition.

Tag: 'nlck'

Friendly name: NLC Kanji Forms

Function: The National Language Council (NLC) of Japan has defined new glyph shapes for a number of JIS characters. The 'nlck' feature is used to access those glyphs.

Example: The glyph is replaced by the glyph .

Recommended implementation: One-for-one substitution of non-NLC glyphs by the corresponding NLC glyph.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Used only with Kanji script.

Feature interaction: This feature is exclusive with the 'jp78', 'jp83', 'jp90' and similar features. It can be combined with the 'palt', 'vpal', 'vert' and 'vrt2' features.

Tag: 'nukt'

Friendly name: Nukta Forms

Function: Produces Nukta forms in Indic scripts.

Example: In Hindi (Devanagari script), a consonant when combined with a nukta gives its nukta form.

Recommended implementation: The nukt table maps the sequence of a consonant followed by a nukta to the consonant's nukta form (GSUB lookup type 4).

Application interface: The application passes the sequence of GIDs (consonant and nukta), to the table, and gets back the GID for the nukta form.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: This feature overrides the results of all other features.

Tag: 'numr'

Friendly name: Numerators

Function: Replaces selected figures which precede a slash with numerator figures, and replaces the typographic slash with the fraction slash.

Example: In the string 11/17 selected by the user, the application turns the 11 into numerators, and the slash into a fraction slash when the user applies the fraction feature (frac).

Recommended implementation: The numr table maps sets of figures and related characters to corresponding numerator glyphs in the font. It also maps the typographic slash (U+002F) to the fraction slash (U+2044). All mappings are one-to-one (GSUB lookup type 1).

Application interface: For GIDs found in the numr coverage table, the application passes a GID to the table and gets back a new GID.

UI suggestion: This feature should normally be called by an application when the user applies the frac feature.

Script/language sensitivity: None.

Feature interaction: This feature supports frac. It may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'onum'

Friendly name: Oldstyle Figures

Function: This feature changes selected figures from the default or lining style to oldstyle form.

Example: The user invokes this feature to get oldstyle figures, which fit better into the flow of normal upper- and lowercase text. Various characters designed to be used with figures may also have oldstyle versions.

Recommended implementation: The onum table maps each lining figure, and any associated characters, to the corresponding oldstyle form (GSUB lookup type 1). If the default figures are non-lining, they too are mapped to the corresponding oldstyle form.

Application interface: For GIDs found in the onum coverage table, the application passes a GID to the onum table and gets back a new GID.

UI suggestion: This feature should be inactive by default. Users can switch between the default and oldstyle figure sets by turning this feature on or off.

NOTE	This feature is separate from the figure-width features pnum and tnum. When the user changes figure style, the application may want to query whether a change in width is also desired.

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of the Lining Figures feature (lnum).

Tag: 'opbd'

Friendly name: Optical Bounds

Function: Aligns glyphs by their apparent left or right extents in horizontal setting, or apparent top or bottom extents in vertical setting, replacing the default behavior of aligning glyphs by their origins. Another name for this behavior would be visual justification. The optical edge of a given glyph is only indirectly related to its advance width or bounding box; this feature provides a means for getting true visual alignment.

Example: Succeeding lines beginning with T, D and W would shift to the left by varying amounts when the text is left-justified and this feature is applied. Succeeding lines ending with r, h and y would likewise shift to the right by differing degrees when the text is right-justified and this feature is applied.

Recommended implementation: Values for affected glyphs are defined with a separate record for left, right, top, and bottom. Each record describes the amount by which the placement and advance width should be altered (GPOS lookup type 1).

Application interface: For GIDs found in the opbd coverage table, the application calls one of two related tables, depending on the position of the glyph. For glyphs at the left end of a horizontal line, it calls the lfbd table, for glyphs at the right end of a horizontal line, it calls the rtbd table.

UI suggestion: This feature should be active by default. It effectively changes the line length, so justification algorithms should account for this adjustment.

Script/language sensitivity: None.

Feature interaction: Should not be applied to glyphs which use fixed-width features (e.g. fwid, halt, hwid, qwid and twid) or vertical features (e.g. vert, vrt2, vpal, valt and vhal). Uses lfbd and rtbd features.

Tag: 'ordn'

Friendly name: Ordinals

Function: Replaces default alphabetic glyphs with the corresponding ordinal forms for use after figures. One exception to the follows-a-figure rule is the numero character (U+2116), which is actually a ligature substitution, but is best accessed through this feature.

Example: The user applies this feature to turn 2.o into 2.o (abbreviation for secundo).

Recommended implementation: The ordn table maps various lowercase letters to corresponding ordinal forms in a chained context (GSUB lookup type 6), and the sequence No to the numero character (GSUB lookup type 4).

Application interface: For sets of GIDs found in the clig coverage table, the application passes the sequence of GIDs to the table and gets back new GIDs. Full sequences must be passed.

NOTE	This may be a change of semantic value. Besides the original character codes, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies mostly to Latin script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'ornm'

Friendly name: Ornaments

Function: This is a dual-function feature, which uses two input methods to give the user access to ornament glyphs (e.g. fleurons, dingbats and border elements) in the font. One method replaces the bullet character with a selection from the full set of available ornaments; the other replaces specific "lower ASCII" characters with ornaments assigned to them. The first approach supports the general or browsing user; the second supports the power user.

Example: The user inputs qwwwwwwwwwe to form the top of a flourished box in Adobe Caslon, or inputs the bullet character, then chooses the thistle dingbat.

Recommended implementation: The ornm table maps all ornaments in a font to the bullet character (GSUB lookup type 3) and each ornament in a font to a corresponding alphanumeric character (GUSB lookup type 1). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. As in any one-from-many substitution, alternates present in more than one face should be ordered consistently across a family, so that those alternates can work correctly when switching between family members.

Application interface: When this feature is invoked, the application must note whether the selected text is the bullet character (U+2022) or alphanumeric characters. In the first case, it passes the GID for bullet to the ornm table and gets back a set of GIDs, and gives the user a means to select from among them. In the second case, for GIDs found in the ornm coverage table, it passes GIDs to the ornm table and gets back new GIDs.

UI suggestion: This feature should be inactive by default. When more than one GID is returned (the bullet case), an application could display the forms sequentially in context, or present a palette showing all the forms at once, or give the user a choice between these approaches. Once the user has selected a specific ornament, that one should be the default selection the next time the bullet is typed. In the absence of such prior information, the application may assume that the first ornament in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: None.

Feature interaction: This feature is mutually exclusive with all other substitution (GSUB) features, which should be turned off when it's applied.

Tag: 'palt'

Friendly name: Proportional Alternate Widths

Function: Respaces glyphs designed to be set on full-em widths, fitting them onto individual (more or less proportional) horizontal widths. This differs from pwid in that it does not substitute new glyphs (GPOS, not GSUB feature). The user may prefer the monospaced form, or may simply want to ensure that the glyph is well-fit and not rotated in vertical setting (Latin forms designed for proportional spacing would be rotated).

Example: The user may invoke this feature in a Japanese font to get Latin, Kanji, Kana or Symbol glyphs with the full-width design but individual metrics.

Recommended implementation: The font specifies alternate metrics for the full-width glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the palt coverage table, the application passes the GIDs to the table and gets back positional adjustments (XPlacement, XAdvance, YPlacement and YAdvance).

UI suggestion: This feature would be off by default.

Script/language sensitivity: Used mostly in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. fwid, halt, hwid, qwid and twid), which should be turned off when it's applied. If palt is activated, there is no requirement that kern must also be activated.  If kern is activated, palt must also be activated if it exists.. See also vpal.

Tag: 'pcap'

Friendly name: Petite Capitals

Function: Some fonts contain an additional size of capital letters, shorter than the regular smallcaps and whimsically referred to as petite caps. Such forms are most likely to be found in designs with a small lowercase x-height, where they better harmonise with lowercase text than the taller smallcaps (for examples of petite caps, see the Emigre type families Mrs Eaves and Filosofia). This feature turns lowercase characters into petite capitals. Forms related to petite capitals, such as specially designed figures, may be included.

Example: The user enters text as lowercase or mixed case, and gets petite cap text or text with regular uppercase and petite caps.

NOTE	Some designers, might extend the petite cap lookups to include uppercase-to-smallcap substitutions, creating a shifting hierarchy of uppercase forms.

Recommended implementation: The pcap table maps lowercase glyphs to the corresponding petite cap forms (GSUB lookup type 1).

Application interface: For GIDs found in the pcap coverage table, the application passes GIDs to the pcap table, and gets back new GIDs. Petite cap substitutions should follow language rules for smallcap (smcp) substitutions.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to scripts with both upper- and lowercase forms (e.g. Latin, Cyrillic, Greek).

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'pkna'

Friendly name: Proportional Kana

Function: Replaces glyphs, kana and kana-related, set on uniform widths (half or full-width) with proportional glyphs.

Example: The user may invoke this feature in a Japanese font to get a proportional glyph instead of a corresponding half- or full-width kana glyph.

Recommended implementation: The font contains alternate kana and kana-related glyphs designed to be set on proportional widths (GSUB lookup type 1).

Application interface: For GIDs found in the pkna coverage table, the application passes the GIDs to the table and gets back new GIDs.

UI suggestion: This feature would normally be off by default.

Script/language sensitivity: Generally used only in Japanese fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. fwid, halt, hwid, palt, pwid, qwid, twid, and vhal), which should be turned off when it's applied. Applying this feature should activate the kern feature.

Tag: 'pnum'

Friendly name: Proportional Figures

Function: Replaces figure glyphs set on uniform (tabular) widths with corresponding glyphs set on glyph-specific (proportional) widths. Tabular widths will generally be the default, but this cannot be safely assumed. Of course this feature would not be present in monospaced designs.

Example: The user may apply this feature to get even spacing for lining figures used as dates in an all-cap headline.

Recommended implementation: In order to simplify associated kerning and get the best glyph design for a given width, this feature should use new glyphs for the figures, rather than only adjusting the fit of the tabular glyphs (although some may be simple copies); i.e. not a GPOS feature. The pnum table maps tabular versions of lining and/or oldstyle figures to corresponding proportional glyphs (GSUB lookup type 1).

Application interface: For GIDs found in the pnum coverage table, the application passes GIDs to the pnum table and gets back new GIDs.

UI suggestion: This feature should be off by default. The application may want to query the user about this feature when the user changes figure style (onum or lnum).

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of the Tabular Figures feature (tnum).

Tag: 'pref'

Friendly name: Pre-base Forms

Function: Substitutes the pre-base form of a consonant.

In some scripts of south or southeast Asia, such as Khmer, the conjoined form of certain consonants is always denoted as a pre-base form. In the case of some scripts of south India, variations in writing conventions exist such that a conjoined Ra consonant may be written as a pre-base form, or a below-base or post-base form. Fonts may be designed to support one or another convention. If a font is designed to support a writing convention in which conjoined Ra is a pre-base form, the Pre-Base Forms feature would be used.

Example: In the Khmer script, the consonant Ra has a pre-base subscript form subscript called Coeng Ra. When the sequence of Coeng followed by Ra, its pre-base form is substituted.

Recommended implementation: The pref table maps the sequence required to convert a consonant into its pre-base form (GSUB lookup type 4).

Application interface: For substitutions defined in the pref table, the application passes the sequence of GIDs to the table, and gets back the GID for the pre base form of the consonant. When shaping scripts of south India, the application may examine the results of processing this feature to determine if the conjoining consonant form needs to be re-ordered.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Khmer and Myanmar (Burmese) scripts that have pre-base forms for consonants. It is also required for southern Indic scripts that may display a pre-base form of Ra, such as Malayalam or Telugu.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of certain Indic and southeast Asian scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms for the given script. For Indic scripts, the following features should be applied in order: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'pres'

Friendly name: Pre-base Substitutions

Function: Produces the pre-base forms of conjuncts in Indic scripts. It can also be used to substitute the appropriate glyph variant for pre-base vowel signs.

Example: In the Gujarati (Indic) script, the doubling of consonant Ka requires the first Ka to be substituted by its pre-base form. This in turn ligates with the second Ka. Applying this feature would result in the ligaturised version of the doubled Ka.

Recommended implementation: The pres table maps a sequence of consonants separated by the virama (halant), to the ligated conjunct form (GSUB lookup type 4). In the case of pre-base matra substitution, the appropriate matra can be substituted using contextual substitution (GSUB lookup type 5).

Application interface: For substitutions defined in the pres table, the application passes the sequence of GIDs to the feature, and gets back the GID for the ligature (or matra as the case may be).

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required in Indic scripts.

Feature interaction: This feature overrides the results of all other features.

Tag: 'pstf'

Friendly name: Post-base Forms

Function: Substitutes the post-base form of a consonant.

Example: In the Gurmukhi (Indic) script, the consonant Ya has a post base form. When the Ya is used as the second consonant in conjunct formation, its post-base form is substituted.

Recommended implementation: The pstf table maps the sequence required to convert a consonant into its post-base form (GSUB lookup type 4).

Application interface: For substitutions defined in the pstf table, the application passes the sequence of GIDs to the feature, and gets back the GID for the post base form of the consonant.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in scripts of south and southeast Asia that have post-base forms for consonants eg: Gurmukhi, Malayalam, Khmer.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic and other related scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms for the given script. For Indic scripts, the following features should be applied in order: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'psts'

Friendly name: Post-base Substitutions

Function: Substitutes a sequence of a base glyph and post-base glyph, with its ligaturised form.

Example: In the Malayalam (Indic) script, the consonant Va has a post base form. When the Va is doubled to form a conjunct- VVa; the first Va [base] and the post base form that follows it, is substituted with a ligature.

Recommended implementation: The psts table maps identified conjunct formation sequences to corresponding ligatures (GSUB lookup type 4).

Application interface: For substitutions defined in the psts table, the application passes the sequence of GIDs to the feature, and gets back the GID for the ligature.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Can be used in any alphabetic script. Required in Indic scripts.

Feature interaction: This feature overrides the results of all other features.

Tag: 'pwid'

Friendly name: Proportional Widths

Function: Replaces glyphs set on uniform widths (typically full or half-em) with proportionally spaced glyphs. The proportional variants are often used for the Latin characters in CJKV fonts, but may also be used for Kana in Japanese fonts.

Example: The user may invoke this feature in a Japanese font to get a proportionally-spaced glyph instead of a corresponding half-width Roman glyph or a full-width Kana glyph.

Recommended implementation: The font contains alternate glyphs designed to be set on proportional widths (GSUB lookup type 1).

Application interface: For GIDs found in the pwid coverage table, the application passes the GIDs to the table and gets back new GIDs.

UI suggestion: Applications may want to have this feature active or inactive by default depending on their markets.

Script/language sensitivity: Although used mostly in CJKV fonts, this feature could be applied in European scripts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. fwid, halt, hwid, palt, qwid, twid, valt and vhal), which should be turned off when it's applied. Applying this feature should activate the kern feature.

Tag: 'qwid'

Friendly name: Quarter Widths

Function: Replaces glyphs on other widths with glyphs set on widths of one quarter of an em (half an en). The characters involved are normally figures and some forms of punctuation.

Example: The user may apply qwid to place a four-digit figure in a single slot in a column of vertical text.

Recommended implementation: The font may contain alternate glyphs designed to be set on quarter-em widths (GSUB lookup type 1), or it may specify alternate metrics for the original glyphs (GPOS lookup type 1) which adjust their spacing to fit in quarter-em widths.

Application interface: For GIDs found in the qwid coverage table, the application passes the GIDs to the table and gets back either new GIDs or positional adjustments (XPlacement and XAdvance).

UI suggestion: This feature would normally be off by default.

Script/language sensitivity: Generally used only in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. fwid, halt, hwid and twid), which should be turned off when it's applied. It deactivates the kern feature.

Tag: 'rand'

Friendly name: Randomize

Function: In order to emulate the irregularity and variety of handwritten text, this feature allows multiple alternate forms to be used.

Example: The user applies this feature in FF Kosmic to get three forms of f in one word.

Recommended implementation: The rand table maps GIDs for default glyphs to one or more GIDs for corresponding alternates (GSUB lookup type 3).

Application interface: For GIDs found in the rand coverage table, the application passes a GID to the rand table and gets back one or more new GIDs. The application selects one of these either by a pseudo-random algorithm, or by noting the sequence of IDs returned, storing that sequence, and stepping through that set as the corresponding character code is invoked.

UI suggestion: This feature should be enabled/disabled via a preference setting; "enabled" is the recommended default.

Script/language sensitivity: None.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'rclt'

Friendly name: Required Contextual Alternates

Function: In specified situations, replaces default glyphs with alternate forms which provide for better joining behavior or other glyph relationships. Especially important in 'script' typefaces which are designed to have some or all of their glyphs join, but applicable also to e.g. variants to improve spacing. This feature is similar to 'calt', but with the difference that it should not be possible to turn off 'rclt' substitutions: they are considered essential to correct layout of the font.

Example: In an Arabic calligraphic font the 'rclt' feature is used to contextually substitute variant forms of letters sin and yeh providing for a correct join between these two letters that differs from the default join of either to other letters.

Recommended implementation: The rclt table specifies the context in which each substitution occurs, and maps one or more default glyphs to replacement glyphs (GSUB lookup type 6).

Application interface: The application passes sequences of GIDs to the feature table, and gets back new GIDs. Note that full sequences must be passed.

UI suggestion: This feature should be active by default. It is recommended that this feature not be turned off, to avoid breaking obligatory shaping.

Script/language sensitivity: May apply to any script, but is especially important for many styles of Arabic.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. For complex scripts, lookups for this feature should be ordered and processed after basic script and language shaping features.

Tag: 'rkrf'

Friendly name: Rakar Forms

Function: Produces conjoined forms for consonants with rakar in Devanagari and Gujarati scripts.

In Devanagari and Gujarati scripts, consonant clusters involving Ra following another consonant are denoted by conjoining an alternate form of Ra to the preceding consonant. Depending on the particular syllable, the preceding consonant may be denoted in its full form or as a half form. Because of interactions involving other behaviors of these scripts, a font implementation may need to process substitution lookups for rakar forms and half forms in a particular sequence in order to derive the appropriate display for various sequences. In recommended usage, the Rakar Forms feature is processed before the Half Forms feature; a half form for a given consonant-Ra combination can be derived by subsequent application of the Half Forms feature. This sequential ordering allows for correct display results.

Example: In Hindi (Devanagari script), the conjunct KRa is denoted with a conjunct ligature form.

Recommended implementation: The rkrf table maps the sequence of a consonant (the nominal form only) followed by a virama (halant) followed by Ra (the nominal form) to the corresponding conjoined form (GSUB lookup type 4).

Application interface: For substitution sequences defined in the rkrf table, the application passes the sequence of GIDs to the table, and gets back the GID for the half form.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Devanagari and Gujarati scripts.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'rlig'

Friendly name: Required Ligatures

Function: Replaces a sequence of glyphs with a single glyph which is preferred for typographic purposes. This feature covers those ligatures, which the script determines as required to be used in normal conditions. This feature is important for some scripts to insure correct glyph formation.

Example: The Arabic character lam followed by alef will always form a ligated lamalef form. This ligated form is a requirement of the script's shaping. The same happens with the Syriac script.

Recommended implementation: The rlig table maps GIDs for default glyphs to one or more GIDs for corresponding alternates (GSUB lookup type 3).

Application interface: The rlig table maps sequences of glyphs to corresponding ligatures (GSUB lookup type 4). Ligatures with more components must be stored ahead of those with fewer components in order to be found. The set of standard ligatures will normally remain constant by script.

UI suggestion: This feature should be active by default. It is recommended that this feature not be turned off to avoid breaking obligatory script shaping.

Script/language sensitivity: Applies to Arabic and Syriac. May apply to some other scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. See also liga.

Tag: 'rphf'

Friendly name: Reph Form

Function: Substitutes the Reph form for a consonant and halant sequence.

Example: In the Devanagari (Indic) script, the consonant Ra possesses a reph form. When the Ra is a syllable initial consonant and is followed by the virama, it is repositioned after the post base vowel sign within the syllable, and also substituted with a mark that sits above the base glyph.

Recommended implementation: The rphf table maps the sequence of default form of Ra and virama to the Reph (GSUB lookup type 4).

Application interface: The application passes the GIDs for Ra and virama to the table and gets back the GID for the reph mark. The application may examine the results of processing other features to determine where in the sequence the reph mark should be re-ordered to.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Required in Indic scripts. eg: Devanagari, Kannada.

Feature interaction: This feature is used in conjunction with certain other features to derive required forms of Indic scripts. The application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'rtbd'

Friendly name: Right Bounds

Function: Aligns glyphs by their apparent right extents at the right ends of horizontal lines of text, replacing the default behavior of aligning glyphs by their origins. This feature is called by the Optical Bounds (opbd) feature above.

Example: Succeeding lines ending with r, h and y would shift to the right by differing degrees when the text is right-justified and this feature is applied.

Recommended implementation: Values for affected glyphs describe the amount by which the placement and advance width should be altered (GPOS lookup type 1).

Application interface: For GIDs found in the rtbd coverage table, the application passes a GID to the table and gets back a new XPlacement and XAdvance value.

UI suggestion: This feature is called by an application when the user invokes the opbd feature.

Script/language sensitivity: None.

Feature interaction: Should not be applied to glyphs which use fixed-width features (e.g. fwid, halt, hwid, qwid and twid) or vertical features (e.g. vert, vrt2, vpal, valt and vhal). Is called by opbd feature.

Tag: 'rtla'

Friendly name: Right-to-left alternates

Registered by: Adobe

Function: This feature applies glyphic variants (other than mirrored forms) appropriate for right-to-left text. (For mirrored forms, see 'rtlm'.)

Recommended implementation: These are required to be glyph substitutions, and it is recommended that they be one-to-one (GSUB lookup type 1).

Application interface: See section “Left-to-right and right-to-left text” in subclause 6.1.4 (Text processing with OFF Layout).

UI suggestion: None.

Script/language sensitivity: Right-to-left runs of text.

Feature interaction: This feature is to be applied simultaneously with other pre-shaping features such as 'ccmp' and 'locl'.

Tag: 'rtlm'

Friendly name: Right-to-left mirrored forms

Registered by: Adobe

Function: This feature applies mirrored forms appropriate for right-to-left text other than for those characters that would be covered by the character-level mirroring step performed by an OFF layout engine. (For right-to-left glyph alternates, see 'rtla'.)

Example: The ‘rtlm’ feature replaces the glyph for U+2232, CLOCKWISE CONTOUR INTEGRAL, with one in which the integral sign is mirrored but the circular arrow has retained its direction.

Recommended implementation: These are required to be glyph substitutions, and it is recommended that they be one-to-one (GSUB lookup type 1).

Application interface: See section “Left-to-right and right-to-left text” in subclause 6.1.4 (Text processing with OFF Layout).

UI suggestion: None.

Script/language sensitivity: Right-to-left runs of text.

Feature interaction: This feature is to be applied simultaneously with other pre-shaping features such as 'ccmp' and 'locl'.

Tag: 'ruby'

Friendly name: Ruby Notation Forms

Function: Japanese typesetting often uses smaller kana glyphs, generally in superscripted form, to clarify the meaning of kanji which may be unfamiliar to the reader. These are called ruby, from the old typesetting term for four-point-sized type. This feature identifies glyphs in the font which have been designed for this use, substituting them for the default designs.

Example: The user applies this feature to the kana character U+3042, to get the ruby form for annotation.

Recommended implementation: The font contains alternate glyphs for all kana characters which are enabled for ruby notation. The ruby table maps GIDs for default forms to GIDs for corresponding ruby alternates. These are one-to-one substitutions (GSUB lookup type 1).

Application interface: For GIDs found in the ruby coverage table, the application passes the GIDs for default forms to the table and gets back new GIDs for ruby forms. The application then scales and positions these forms according to its defaults, which may take user parameters.

UI suggestion: This feature should be inactive by default. Applications may offer the user an opportunity to specify the degree of scaling and baseline shift.

Script/language sensitivity: Applies only to Japanese.

Feature interaction: This feature overrides the results of any other feature for the affected characters.

Tag: 'rvrn'

Friendly name: Required Variation Alternates

Registered by: Microsoft

Function: This feature is used in fonts that support OFF Font Variations in order to select alternate glyphs for particular variation instances. (For background on OFF font variations, see "Font variations overview".)

When a variable font is used, all of the interpolated variants of a given glyph ID have exactly the same contours and points. It is possible to use glyph variation mechanisms to make significant outline changes, such as reducing strokes in heavy-weight or narrow-width variants, but this approach may be difficult to implement and may not produce desired results for all variation instances. Instead, better results for these scenarios might be achieved by substitution to a different glyph ID. The specific substitutions applied would be conditioned by the particular variation instance that is selected by the user. This conditional behavior is implemented using the required variation alternates feature in conjunction with a FeatureVariations table within the GSUB table.

Example: A variable font supports weight variations ranging from thin to black. The default glyph for the dollar sign has two vertical strokes running through the full extent of the glyph. In the bold variation instance, the default glyph is substituted to an alternate glyph that has only one vertical stroke. In the black variation instance, the default glyph is substituted to an alternate glyph have has only single vertical bars at the top and bottom extremities, with no vertical bars in the two counters in between.

Recommended implementation: The feature is used to activate single substitution (GSUB type 1) lookups, and is always used in conjunction with a FeatureVariations table. Typically, a FeatureTable referenced in a FeatureRecord with the 'rvrn' tag will have LookupCount set to 0; in this way, the default variation instance does not have any glyph substitution applied but, rather, uses default glyphs. Alternate glyphs for particular variation instances are obtained by adding a substitution of the feature table to an alternate feature table within a FeatureVariations table. Different alternate feature tables may be selected using condition sets that specify particular variation-axis value ranges.

One or more Condition tables is used to determine variation-axis value ranges for which an alternate feature table (and associated lookups) is selected. The axis values used to trigger a condition should normally be midway between values used for named instances. This will avoid any possibility of inconsistent behavior in different applications when using named instances that might arise due to small discrepancies in processing the numeric values.

The default language system for the 'DFLT' script can reference a feature record for this feature with a feature table that will be substituted for particular variation instances to use lookups that apply default, language-independent glyph substitutions; this feature record should be the first feature record for this feature. Some applications may choose to process this feature without processing other features or the script/language system hierarchy; for this purpose, they should choose the first feature record for this feature to obtain the most suitable substitutions for language-independent results.

Application interface: Application of the 'rvrn' feature is mandatory in implementations that support OFF Font Variations whenever a variable font is in use. The feature should be processed in any layout process that supports use of variations, even if other OFF Layout processing is not supported.

The feature is applied only during the process of deriving final glyph IDs (GSUB); it is not used for glyph positioning (GPOS). It should be processed early in GSUB processing, before application of the localized forms feature or features related to shaping of complex scripts or discretionary typographic effects.

Processing of the 'rvrn' feature also requires processing of the FeatureVariations table. ConditionSet tables are scanned for a ConditionSet matching the current variation instance, and then a corresponding FeatureTableSubstitution table is used to locate an alternate feature table. For complete details on processing a FeatureVariations table, see "OFF layout common table formats".

When an applicable feature table is located, only single substitution (GSUB type 1) lookups are processed; any other lookup types are ignored. For any glyph IDs in the coverage table, the application passes the glyph ID to the lookup, and gets back the new glyph ID.

UI suggestion: The 'rvrn' feature is mandatory: it should be active by default and not directly exposed to user control.

Script/language sensitivity: Used for all languages and scripts.

Feature interaction: The feature should be processed early after initial character-to-glyph mapping, before application of the localized forms ('locl') feature, any features related to shaping of complex scripts, or any discretionary features.

Tag: 'salt'

Friendly name: Stylistic Alternates

Function: Many fonts contain alternate glyph designs for a purely esthetic effect; these don't always fit into a clear category like swash or historical. As in the case of swash glyphs, there may be more than one alternate form. This feature replaces the default forms with the stylistic alternates.

Example: The user applies this feature to Industria to get the alternate form of g.

Recommended implementation: The salt table maps GIDs for default forms to one or more GIDs for corresponding stylistic alternatives. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. As in any one-from-many substitution, alternates present in more than one face should be ordered consistently across a family, so that those alternates can work correctly when switching between family members.

Application interface: For GIDs found in the salt coverage table, the application passes the GIDs to the salt table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired.

UI suggestion: This feature should be inactive by default. When more than one GID is returned, an application could display the forms sequentially in context, or present a palette showing all the forms at once, or give the user a choice between these approaches. The application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: None.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'sinf'

Friendly name: Scientific Inferiors

Function: Replaces lining or oldstyle figures with inferior figures (smaller glyphs which sit lower than the standard baseline, primarily for chemical or mathematical notation). May also replace lowercase characters with alphabetic inferiors.

Example: The application can use this feature to automatically access the inferior figures (more legible than scaled figures).

Recommended implementation: The sinf table maps figures to the corresponding inferior forms (GSUB lookup type 1).

Application interface: For GIDs found in the sinf coverage table, the application passes a GID to the feature and gets back a new GID.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Can apply to nearly any script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'size'

NOTE	Use of this feature has been superseded by the 'STAT' table. See subclause 9.9 in the "General recommendations" for more information.

Friendly name: Optical size

Function: This feature stores two kinds of information about the optical size of the font: design size (the point size for which the font is optimized) and size range (the range of point sizes which the font can serve well), as well as other information which helps applications use the size range. The design size is useful for determining proper tracking behavior. The size range is useful in families which have fonts covering several ranges. Additional values serve to identify the set of fonts which share related size ranges, and to identify their shared name.

NOTE	Sizes refer to nominal final output size, and are independent of viewing magnification or resolution.

Required implementation:

The Feature table of this GPOS feature contains no lookups; its Feature Parameters field records an offset from the beginning of the Feature table to an array of five 16-bit unsigned integer values. The size feature must be implemented in all fonts in any family which uses the feature. In this usage, a family is a set of fonts which share a TypographicFamily name (name ID 16), or Font Family name (name ID 1) if the TypographicFamily name is absent.

- The first value represents the design size in 720/inch units (decipoints). The design size entry must be non-zero. When there is a design size but no recommended size range, the rest of the array will consist of zeros.

- The second value has no independent meaning, but serves as an identifier that associates fonts in a subfamily. All fonts which share a Typographic or Font Family name and which differ only by size range shall have the same subfamily value, and no fonts which differ in weight or style shall have the same subfamily value. If this value is zero, the remaining fields in the array will be ignored.

- The third value enables applications to use a single name for the subfamily identified by the second value. If the preceding value is non-zero, this value must be set in the range 256 - 32767 (inclusive). It records the value of a field in the name table, which must contain English-language strings encoded in Windows Unicode and Macintosh Roman, and may contain additional strings localized to other scripts and languages. Each of these strings is the name an application should use, in combination with the family name, to represent the subfamily in a menu. Applications will choose the appropriate version based on their selection criteria.

- The fourth and fifth values represent the small end of the recommended usage range (exclusive) and the large end of the recommended usage range (inclusive), stored in 720/inch units (decipoints). Ranges must not overlap, and should generally be contiguous.

Example: The size information in Bell Centennial is [60 0 0 0 0]. This tells an application that the fontâs design size is six points, so larger sizes may need proportionate reduction in default inter-glyph spacing. The size information in Minion Pro Semibold Condensed Subhead is [180 3 257 139 240]. These values tell an application that:

- The font's design size is 18 points;

- This font is part of a subfamily of fonts that differ only by the size range which each covers, and which share the arbitrary identifier number 3;

- ID 257 in the name table is the suggested menu name for this subfamily. In this case, the string at name ID 257 is Semibold Condensed;

- This font is the recommended choice from sizes greater than 13.9-point up through 24-points.

Application interface: When the user specifies a size, the application checks for a size feature in the active font. If none is found, the application follows its default behavior. If one is found, the application follows the specified offset to retrieve the five values.

- Design size: Applications which offer size-based tracking have a pre-defined curve which they can apply. By default, this curve should be set to produce no adjustment at the font's design size (first value in the array, in decipoints).

- Size ranges: If the second value in the size array is non-zero, the font has a recommended size range. When any such font is selected by the user, the application builds a list of all fonts with this subfamily value and the same TypographicFamily name, and notes the size range in the current font. Applications may want to cache the subfamily list at this point. If the specified size falls in the current font's range, the application uses the current font. If not, the application checks the other ranges in the subfamily, and if the specified size falls in one of them, uses that font. If the specified size is not in any range present, the font with the range closest to the specified value is used. If the specified size falls exactly between two ranges, the range with the larger values is used. Since adding or removing fonts from a subfamily may cause reflow, applications should note which fonts are used for which text.

UI suggestion: This feature should be active by default. Applications may want to present the tracking curve to the user for adjustments via a GUI. At start-up, and when fonts are added or removed, applications may want to build a list of fonts with such ranges, and display the filtered subfamily names in their font selection UI, with each filtered name representing the full set of related sizes. Applications may also present a setting which allows the user to select non-default sizes (for example, in the case where final output is intended for on-screen viewing, a smaller optical size will produce better results). In such a case, the font-selection UI should present the unfiltered names. Applications should notify the user if fonts are removed or added from a subfamily with size ranges, and query about desired behavior.

Script/language sensitivity: None. The FeatureParams of all ‘size’ features in the GPOS FeatureList must point to the same set of values.

Feature interaction: None.

Tag: 'smcp'

Friendly name: Small Capitals

Function: This feature turns lowercase characters into small capitals. This corresponds to the common SC font layout. It is generally used for display lines set in Large & small caps, such as titles. Forms related to small capitals, such as oldstyle figures, may be included.

Example: The user enters text as mixed capitals and lowercase, and gets Large & small cap text.

Recommended implementation: The smcp table maps lowercase glyphs to the corresponding small-cap forms (GSUB lookup type 1).

Application interface: For GIDs found in the smcp coverage table, the application passes GIDs to the smcp table, and gets back new GIDs.

NOTE	Applications should treat ß (U+00DF) as a pair of s characters, and that the Turkish dotless i maps to the normal small cap I.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to bicameral scripts (i.e. those with case differences), such as Latin, Greek, Cyrillic, and Armenian.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. Also see c2sc.

Tag: 'smpl'

Friendly name: Simplified Forms

Function: Replaces 'traditional' Chinese or Japanese forms with the corresponding 'simplified' forms.

Example: The user gets U+53F0 when U+6AAF, U+81FA, or U+98B1 is entered.

Recommended implementation: The smpl table maps each traditional form in a font to a corresponding simplified form (GSUB lookup type 1).

NOTE	More than one traditional form may map to a single simplified form.

Application interface: For GIDs found in the smpl coverage table, the application passes the GIDs to the table and gets back one new GID for each.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature would be off by default, but could be made the default by a preference setting.

Script/language sensitivity: Applies only to Chinese and Japanese.

Feature interaction: This feature is mutually exclusive with all other features, which should be turned off when it’s applied, except the palt, vert and vrt2 features, which may be used in addition; trad and tnam are mutually exclusive, and override the results of smpl.

Tag: 'ss01' - 'ss20'

Friendly name: Stylistic Set 1 - Stylistic Set 20

Function: In addition to, or instead of, stylistic alternatives of individual glyphs (see 'salt' feature), some fonts may contain sets of stylistic variant glyphs corresponding to portions of the character set, e.g. multiple variants for lowercase letters in a Latin font. Glyphs in stylistic sets may be designed to harmonise visually, interact in particular ways, or otherwise work together. Examples of fonts including stylistic sets are Zapfino Linotype and Adobe's Poetica. Individual features numbered sequentially with the tag name convention 'ss01' 'ss02' 'ss03' . 'ss20' provide a mechanism for glyphs in these sets to be associated via GSUB lookup indexes to default forms and to each other, and for users to select from available stylistic sets.

Recommended implementation: An ssXX table maps GIDs for default forms to one GIDs for corresponding stylistic alternatives in each set. Each ssXX feature uses one-to-one (GSUB lookup type 1) substitutions. Font developers may choose to map only from default forms to variants for each stylistic set, or may choose to map between all stylistic sets in each feature, depending on intended user experience. For example, feature 'ss03' might contain lookups mapping variant glyphs from 'ss01' and 'sso2' to corresponding variants in 'ss03', in addition to mapping from default forms.

The FeatureParams field of the Feature Table of these GSUB features may be set to 0, or to an offset to a Feature Parameters table comprising two successive uint16 values, as follows:

- Version (set to 0): This corresponds to a "minor" version number. Additional data may be added to the end of this Feature Parameters table in the future.

- UI Name ID: The 'name' table name ID that specifies a string (or strings, for multiple languages) for a user-interface label for this feature. The value of uiLabelNameId is expected to be in the font-specific name ID range (256–32767), though that is not a requirement in this Feature Parameters specification. The user-interface label for the feature can be provided in multiple languages. An English string should be included as a fallback. The string should be kept to a minimal length to fit comfortably with different application interfaces.

Application interface: The application is responsible for counting and enumerating the number of features in the font with tag names of the format 'ss01' to 'ss20', and for presenting the user with an appropriate selection mechanism. For GIDs found in the ssXX coverage table, the application passes the GIDs to the ssXX table and gets back one or more new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None. For each respective[/distinct] 'ssXX' feature, the FeatureParams in the FeatureList must point to the same set of values.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override. After an ssXX feature has been applied, the user may wish to apply glyph-specific features, e.g. 'salt', to individual glyphs in the resulting layout; font developers are responsible for ordering substitution lookups to obtain desired user experience.

Tag: 'ssty'

Friendly name: Math script style alternates

Function: This feature provides glyph variants adjusted to be more suitable for use in subscripts and superscripts. The script style forms should not be scaled or moved in the font; scaling and moving them is done by the math handling client. Instead, the ssty feature should provide glyph forms that result in shapes that look good as superscripts and subscripts when scaled and positioned by the Math engine. When designing the script forms, the font developer may assume that MATH.MathConstants.ScriptPercentScaleDown and MATH.MathConstants.ScriptScriptPercentScaleDown will be the scaling factors used by the Math engine.

This feature can have a parameter indicating the script level: 1 for simple subscripts and superscripts, 2 for second level subscripts and superscripts (that is, scripts on scripts), and so on. (Currently, only the first two alternates are used). For glyphs that are not covered by this feature, the original glyph is used in subscripts and superscripts.

Example: Depending In  formula letter b will be substituted with script level 1 variant and letter c will be substituted with level 2 variant

Recommended implementation: Alternate substitution, with parameter 1 or 2 corresponding to sub- or super- script level alternate glyphs. If there are no second-level alternates defined in the font, single substitution may also be used. Glyphs that don’t have script alternates can be omitted from this table. See MATH table specification for details.

Application interface: Feature is invoked automatically by math layout handler depending on nesting level inside the math formula.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of math formula. It should be applied in the appropriate contexts, as determined by math layout handler. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Applied to math formula layout.

Feature interaction: This feature is applied to individual glyphs during layout of math formula.

Tag: 'stch'

Friendly name: Stretching Glyph Decomposition

Function: Unicode characters, such as the Syriac Abbreviation Mark (U+070F), that enclose other characters need to be able to stretch in order to dynamically adapt to the width of the enclosed text. This feature defines a decomposition set consisting of an odd number of glyphs which describe the stretching glyph. The odd numbered glyphs in the decomposition are fixed reference points which are distributed evenly from the start to the end of the enclosed text. The even numbered glyphs may be repeated as necessary to fill the space between the fixed glyphs. The first and last glyphs may either be simple glyphs with width at the baseline, or mark glyphs. All other decomposition glyphs should have width, but must be defined as mark glyphs.

Example: In Syriac, the character 0x070F is a control character that is rendered as a line above an abbreviation in Syriac script. The line should have a circle at each end and at the mid-point. The decomposition sequence for this character should consist of a circle at the start of a line, a connecting line, a circle on a line for the mid-point, a second connecting line, and a circle at the end of the line. The connecting lines will repeat in order to fill the space between the circle glyphs.

Recommended implementation: The stch table maps the character to a set containing an odd number of corresponding glyphs (GSUB lookup type 2). The rendering engine reorders the last glyph from the substituted set to the end of the set of characters being enclosed. The remaining glyphs from the substituted set are positioned at the start of the set of characters being enclosed. Odd-numbered glyphs in the decomposition set are positioned so that they are distributed evenly over the width of the text being enclosed. Even-numbered glyphs in the decomposition set are repeated by the rendering engine so the width of the space between fixed, odd-numbered glyphs is filled by the spacing, even-numbered glyphs.

Application interface: For GIDs found in the stch coverage table, the application passes the sequence of GIDs to the table, and gets back the GIDs for the multiple substitution.

UI suggestion: This feature should be on by default.

Script/language sensitivity:  None.

Feature interaction: None.

Tag: 'subs'

Friendly name: Subscript

Function: The 'subs' feature may replace a default glyph with a subscript glyph, or it may combine a glyph substitution with positioning adjustments for proper placement.

Recommended implementation: First, a single or contextual substitution lookup implements the subscript glyph (GSUB lookup type 1). Then, if the glyph needs repositioning, an application may apply a single adjustment, pair adjustment, or contextual adjustment positioning lookup to modify its position.

Application interface: For GIDs found in the subs coverage table, the application passes a GID to the feature and gets back a new GID. This is a change of semantic value. Besides the original character codes, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Can apply to nearly any script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'sups'

Friendly name: Superscript

Function: Replaces lining or oldstyle figures with superior figures (primarily for footnote indication), and replaces lowercase letters with superior letters (primarily for abbreviated French titles).

Example: The application can use this feature to automatically access the superior figures (more legible than scaled figures) for footnotes, or the user can apply it to Mssr to get the classic form.

Recommended implementation: The sups table maps figures and lowercase letters to the corresponding superior forms (GSUB lookup type 1).

Application interface: For GIDs found in the sups coverage table, the application passes a GID to the feature and gets back a new GID.

NOTE	This can include a change of semantic value. Besides the original character codes, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Can apply to nearly any script.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'swsh'

Friendly name: Swash

Function: This feature replaces default character glyphs with corresponding swash glyphs. It should be noted that there may be more than one swash alternate for a given character.

Example: The user inputs the ampersand character when setting text with Poetica with this feature active, and is presented with a choice of the 63 ampersand forms in that face.

Recommended implementation: The swsh table maps GIDs for default forms to those for one or more corresponding swash forms. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. If several styles of swash are present across the font, the set of forms for each character should be ordered consistently.

Application interface: For GIDs found in the swsh coverage table, the application passes the GIDs to the swsh table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired.

UI suggestion: This feature should be inactive by default. When more than one GID is returned, an application could display the forms sequentially in context, or present a palette showing all the forms at once, or give the user a choice between these approaches. The application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: Does not apply to ideographic scripts.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'titl'

Friendly name: Titling

Function: This feature replaces the default glyphs with corresponding forms designed specifically for titling. These may be all-capital and/or larger on the body, and adjusted for viewing at larger sizes.

Example: The user applies this feature in Adobe Garamond to get the titling caps.

Recommended implementation: The titl table maps default forms to corresponding titling forms (GSUB lookup type 1).

Application interface: For GIDs found in the titl coverage table, the application passes the GIDs to the titl table and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: None.

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'tjmo'

Friendly name: Trailing Jamo Forms

Function: Substitutes the trailing jamo form of a cluster.

Example: In Hangul script, the jamo cluster is composed of three parts (leading consonant, vowel, and trailing consonant). When a sequence of trailing class jamos are found, their combined trailing jamo form is substituted.

Recommended implementation: The tjmo table maps the sequence required to convert a series of jamos into its trailing jamo form (GSUB lookup type 4).

Application interface: For substitutions defined in the tjmo table, the application passes the sequence of GIDs to the feature, and gets back the GID for the trailing jamo form.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required for Hangul script when Ancient Hangul writing system is supported.

Feature interaction: This feature overrides the results of all other features.

Tag: 'tnam'

Friendly name: Traditional Name Forms

Function: Replaces 'simplified' Japanese kanji forms with the corresponding 'traditional' forms. This is equivalent to the Traditional Forms feature, but explicitly limited to the traditional forms considered proper for use in personal names (as many as 205 glyphs in some fonts).

Example: The user inputs U+4E9C and gets U+4E9E.

Recommended implementation: The tnam table maps simplified forms in a font to corresponding traditional forms which can be used in personal names (GSUB lookup type 1). The application stores a record of any simplified forms which resulted from substitutions (the smpl feature); for such forms, applying the tnam feature undoes the previous substitution.

Application interface: For GIDs found in the tnam coverage table, the application passes the GIDs to the table and gets back new GIDs.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to Japanese.

Feature interaction: May include some characters affected by the Proportional Alternate Widths feature (palt); trad and tnam are mutually exclusive, and override the results of smpl.

Tag: 'tnum'

Friendly name: Tabular Figures

Function: Replaces figure glyphs set on proportional widths with corresponding glyphs set on uniform (tabular) widths. Tabular widths will generally be the default, but this cannot be safely assumed. Of course this feature would not be present in monospaced designs.

Example: The user may apply this feature to get oldstyle figures to align vertically in a column.

Recommended implementation: In order to simplify associated kerning and get the best glyph design for a given width, this feature should use new glyphs for the figures, rather than only adjusting the fit of the proportional glyphs (although some may be simple copies); i.e. not a GPOS feature. The tnum table maps proportional versions of lining &/or oldstyle figures to corresponding tabular glyphs (GSUB lookup type 1).

Application interface: For GIDs found in the tnum coverage table, the application passes GIDs to the tnum table and gets back new GIDs.

UI suggestion: This feature should be off by default. The application may want to query the user about this feature when the user changes figure style (onum or lnum).

Script/language sensitivity: None.

Feature interaction: This feature overrides the results of the Proportional Figures feature ( pnum).

Tag: 'trad'

Friendly name: Traditional Forms

Function: Replaces 'simplified' Chinese hanzi or Japanese kanji forms with the corresponding 'traditional' forms.

Example: The user inputs U+53F0 and is offered a choice of U+6AAF, U+81FA, or U+98B1.

Recommended implementation: The trad table maps each simplified form in a font to one or more traditional forms. While many of these substitutions are one-to-one (GSUB lookup type 1), others require a selection from a set (GSUB lookup type 3). The manufacturer may choose to build two tables (one for each lookup type) or only one which uses lookup type 3 for all substitutions. As in any one-from-many substitution, alternates present in more than one face should be ordered consistently across a family, so that those alternates can work correctly when switching between family members.

Application interface: For GIDs found in the trad coverage table, the application passes the GIDs to the table and gets back one or more new GIDs. If more than one GID is returned, the application must provide a means for the user to select the one desired. The application stores a record of any simplified forms which resulted from substitutions (the smpl feature); for such forms, applying the trad feature undoes the previous substitution.

NOTE	This is a change of character code. Besides the original character code, the application should store the code for the new character.

UI suggestion: This feature should be inactive by default. If there's no record of a conversion from traditional to simplified, the user must be offered a set of possibilities from which to select. The application may note the user's choice, and offer it as a default the next time the source simplified character is encountered. In the absence of such prior information, the application may assume that the first glyph in a set is the preferred form, so the font developer should order them accordingly.

Script/language sensitivity: Applies only to Chinese and Japanese.

Feature interaction: May include some characters affected by the Proportional Alternate Widths feature (palt); trad and tnam are mutually exclusive, and override the results of smpl.

Tag: 'twid'

Friendly name: Third Widths

Function: Replaces glyphs on other widths with glyphs set on widths of one third of an em. The characters involved are normally figures and some forms of punctuation.

Example: The user may apply twid to place a three-digit figure in a single slot in a column of vertical text.

Recommended implementation: The font may contain alternate glyphs designed to be set on third-em widths (GSUB lookup type 1), or it may specify alternate metrics for the original glyphs (GPOS lookup type 1) which adjust their spacing to fit in third-em widths.

Application interface: For GIDs found in the twid coverage table, the application passes the GIDs to the table and gets back either new GIDs or positional adjustments (XPlacement and XAdvance).

UI suggestion: This feature would normally be off by default.

Script/language sensitivity: Generally used only in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-width features (e.g. fwid, halt, hwid and qwid), which should be turned off when it's applied. It deactivates the kern feature.

Tag: 'unic'

Friendly name: Unicase

Function: This feature maps upper- and lowercase letters to a mixed set of lowercase and small capital forms, resulting in a single case alphabet (for an example of unicase, see the Emigre type family Filosofia). The letters substituted may vary from font to font, as appropriate to the design. If aligning to the x-height, smallcap glyphs may be substituted, or specially designed unicase forms might be used. Substitutions might also include specially designed figures.

Example: The user enters text as uppercase, lowercase or mixed case, and gets unicase text.

Recommended implementation: The unic table maps some uppercase and lowercase glyphs to corresponding unicase forms (GSUB lookup type 1).

Application interface: For GIDs found in the unic coverage table, the application passes GIDs to the unic table, and gets back new GIDs.

UI suggestion: This feature should be off by default.

Script/language sensitivity: Applies only to scripts with both upper- and lowercase forms (e.g. Latin, Cyrillic, Greek).

Feature interaction: This feature may be used in combination with other substitution (GSUB) features, whose results it may override.

Tag: 'valt'

Friendly name: Alternate Vertical Metrics

Function: Repositions glyphs to visually center them within full-height metrics, for use in vertical setting. Typically applies to full-width Latin glyphs, which are aligned on a common horizontal baseline and not rotated when set vertically in CJKV fonts.

Example: Applying this feature would shift a Roman h down, or y up, from their default full-width positions.

Recommended implementation: The font specifies alternate metrics for the original glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the valt coverage table, the application passes the GIDs to the table and gets back positional adjustments (YPlacement).

UI suggestion: This feature should be active by default in vertical-setting contexts.

Script/language sensitivity: Applies only to scripts with vertical writing modes.

Feature interaction: This feature is mutually exclusive with all other glyph-height features (e.g. vhal and vpal), which should be turned off when it's applied. It deactivates the kern feature.

Tag: 'vatu'

Friendly name: Vattu Variants

Function: In an Indic consonant conjunct, substitutes a ligature glyph for a base consonant and a following vattu (below-base) form of a conjoining consonant, or for a half form of a consonant and a following vattu form.

Example: In the Devanagari (Indic) script, the consonant Ra takes a vattu form, when it is not the syllable initial consonant in a conjunct. This vattu form ligates with the base consonant as well as half forms of consonants.

Recommended implementation: The vatu table maps consonant and vattu form combinations to their respective ligatures (GSUB lookup type 4).

Lookups associated with the Vattu Variants feature apply to glyphs derived using the Below-base Forms feature and (for half-form plus vattu ligatures) the Half Forms features. The Below-base Forms feature should be used to derive the nominal vattu form of a consonant; the Vattu Variants feature should only be used to substitute the nominal vattu form and a base consonant or half form with a ligature glyph. If the Rakar Forms feature is used, the Vattu Variants feature is not required.

Application interface: For substitutions defined in the vatu table, the application passes the sequence of GIDs to the table, and gets back the GID for the vattu variant ligature.

UI suggestion: In recommended usage, this feature triggers substitutions that are required for correct display of the given script. It should be applied in the appropriate contexts, as determined by script-specific processing. Control of the feature should not generally be exposed to the user.

Script/language sensitivity: Used in Indic scripts. eg: Devanagari.

Feature interaction: This feature may be used in conjunction with certain other features to derive required forms of Indic scripts. For Indic script implementations that use the Vattu Variants feature, the application is expected to process this feature and certain other features in an appropriate order to obtain the correct set of basic forms: nukt, akhn, rphf, rkrf, pref, blwf, half, pstf, cjct. Other discretionary features for optional typographic effects may also be applied. Lookups for such discretionary features should be processed after lookups for this feature have been processed.

Tag: 'vchw'

Friendly name: Vertical Contextual Half-Width Spacing

Registered by: Adobe/W3C

Function: Contextually respaces glyphs designed to be set on full-em heights, fitting them onto individual half-width vertical heights, to approximate more sophisticated text layout, such as what is described in Requirements for Japanese Text Layout (JLREQ [21]) or similar CJK text-layout specifications that expect half-width forms of characters whose default glyphs are full-width. This differs from 'vhal' in that the respacing is contextual. This feature may be invoked to get better fit for punctuation or symbol glyphs without disrupting the monospaced alignment.

Example: When PRESENTATION FORM FOR VERTICAL RIGHT PARENTHESIS (U+FE36; "︶", vertical form of FULLWIDTH RIGHT PARENTHESIS U+FF09; "）") is followed by PRESENTATION FORM FOR VERTICAL IDEOGRAPHIC COMMA (U+FE11; "︑", vertical form of IDEOGRAPHIC COMMA U+3001; "、"), the latter is respaced to remove half-em of height between them.

Recommended implementation: The font stores a set of adjustments for pairs of glyphs (GPOS lookup type 2 or 8). These may be stored as one or more tables matching left and right classes, &/or as individual pairs. Additional adjustments may be provided for larger sets of glyphs (e.g. triplets, quadruplets, etc.) to overwrite the results of pair kerns in particular combinations.

Application interface: The application passes a sequence of GIDs to the 'vchw' table, and gets back adjusted positions (XPlacement, XAdvance, YPlacement, and YAdvance) for those GIDs. When using the type 2 lookup on a run of glyphs, it’s critical to remember to not consume the last glyph, but to keep it available as the first glyph in a subsequent run (this is a departure from normal lookup behavior).

UI suggestion: This feature would be off by default.

Script/language sensitivity: Used mostly in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-height features (e.g., 'valt', 'vhal', 'vpal'), which should be turned off when this feature is applied. It deactivates the 'vkrn' feature. See also 'chws'.

Tag: 'vert'

Friendly name: Vertical Alternates

Registered by: Adobe/Microsoft

Function: Transforms default glyphs into glyphs that are appropriate for upright presentation in vertical writing mode. While the glyphs for most characters in East Asian writing systems remain upright when set in vertical writing mode, some must be transformed – usually by rotation, shifting, or different component ordering – for vertical writing mode.

Example: In vertical writing mode, the opening parenthesis (U+FF08) is replaced by the rotated form (U+FE35).

In vertical writing mode, the glyph for HIRAGANA LETTER SMALL A (U+3041; "ぁ") is transformed into a glyph that is shifted up and to the right, which is properly positioned for upright presentation in vertical writing mode.

In vertical writing mode, the glyph for SQUARE MAIKURO (U+3343; "㍃"), whose component katakana characters are ordered from left to right then top to bottom (like horizontal writing mode), is transformed into a glyph whose component katakana characters are ordered from top to bottom then right to left (like vertical writing mode line progression).

Recommended implementation: The font includes versions of the glyphs covered by this feature that differ in some visual way from the default glyphs, such as by rotation, shifting, or different component ordering. The 'vert' feature maps the default glyphs to the corresponding, alternate glyphs for vertical writing mode using a type 1 (single substitution) GSUB lookup.

Application interface: For GIDs found in the 'vert' coverage table, the layout engine passes GIDs to the feature, then gets back new GIDs.

UI suggestion: This feature should be active by default in vertical writing mode.

Script/language sensitivity: Applies only to scripts with vertical writing capability.

Feature interaction: The 'vert' and 'vrtr' features are intended to be used in conjunction: 'vert' for glyphs to be presented upright in vertical writing, and 'vrtr' for glyphs intended to be presented sideways. Since they must never be activated simultaneously for a given glyph, there should be no interaction between the two features. These features are intended for layout engines that graphically rotate glyphs for sideways runs in vertical writing mode, such as those conforming to Unicode Standard Annex #50: Unicode Vertical Text Layout [23].

Note that layout engines that instead depend on the font to supply pre-rotated glyphs for all sideways glyphs should use the 'vrt2' feature in lieu of 'vert' and 'vrtr'. Because 'vrt2' supplies pre-rotated glyphs, the 'vert' feature should never be used with 'vrt2', but may be used in addition to any other feature.

Tag: 'vhal'

Friendly name: Alternate Vertical Half Metrics

Function: Respaces glyphs designed to be set on full-em heights, fitting them onto half-em heights.

Example: The user may invoke this feature in a CJKV font to get better fit for punctuation or symbol glyphs without disrupting the monospaced alignment.

Recommended implementation: The font specifies alternate metrics for the full-height glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the vhal coverage table, the application passes the GIDs to the table and gets back positional adjustments (XPlacement, XAdvance, YPlacement and YAdvance).

UI suggestion: In general, this feature should be off by default. Different behavior should be used, however, in applications that conform to Requirements for Japanese Text Layout (JLREQ [21]) or similar CJK text-layout specifications that expect half-width forms of characters whose default glyphs are full-width. Such implementations should turn this feature on by default, or should selectively apply this feature to particular characters that require special treatment for CJK text-layout purposes, such as brackets, punctuation, and quotation marks.

Script/language sensitivity: Used only in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-height features (e.g. valt and vpal), which should be turned off when it’s applied. It deactivates the kern feature. See also halt.

Tag: 'vjmo'

Friendly name: Vowel Jamo Forms

Function: Substitutes the vowel jamo form of a cluster.

Example: In Hangul script, the jamo cluster is composed of three parts (leading consonant, vowel, and trailing consonant). When a sequence of vowel class jamos are found, their combined vowel jamo form is substituted.

Recommended implementation: The vjmo table maps the sequence required to convert a series of jamos into its vowel jamo form (GSUB lookup type 4).

Application interface: For substitutions defined in the vjmo table, the application passes the sequence of GIDs to the feature, and gets back the GID for the vowel jamo form.

UI suggestion: This feature should be on by default.

Script/language sensitivity: Required for Hangul script when Ancient Hangul writing system is supported.

Feature interaction: This feature overrides the results of all other features.

Tag: 'vkna'

Friendly name: Vertical Kana Alternates

Function: Replaces standard kana with forms that have been specially designed for only vertical writing. This is a typographic optimization for improved fit and more even color. Also see hkna.

Example: Standard full-width kana (hiragana and katakana) are replaced by forms that are designed for vertical use.

Recommended implementation: The font includes a set of specially-designed glyphs, listed in the vkna coverage table. The vkna feature maps the standard full-width forms to the corresponding special vertical forms (GSUB lookup type 1).

Application interface: For GIDs found in the vkna coverage table, the application passes GIDs to the feature, and gets back new GIDs.

UI suggestion: This feature would be off by default.

Script/language sensitivity: Applies only to fonts that support kana (hiragana and katakana).

Feature interaction: Since this feature is only for vertical use, features applying to horizontal behaviors (e.g. kern) do not apply.

Tag: 'vkrn'

Friendly name: Vertical Kerning

Function: Adjusts amount of space between glyphs, generally to provide optically consistent spacing between glyphs. Although a well-designed typeface has consistent inter-glyph spacing overall, some glyph combinations require adjustment for improved legibility. Besides standard adjustment in the vertical direction, this feature can supply size-dependent kerning data via device tables, "cross-stream" kerning in the X text direction, and adjustment of glyph placement independent of the advance adjustment.

NOTE	This feature may apply to runs of more than two glyphs, and would not be used in monospaced fonts.   This feature applies only to text set vertically.

Example: When the katakana character U+30B9 or U+30D8 is followed by U+30C8 in a vertical setting, U+30C8 is shifted up to fit more evenly.

Recommended implementation: The font stores a set of adjustments for pairs of glyphs (GPOS lookup type 2 or 8). These may be stored as one or more tables matching left and right classes, &/or as individual pairs. Additional adjustments may be provided for larger sets of glyphs (e.g. triplets, quadruplets, etc.) to overwrite the results of pair kerns in particular combinations.

Application interface: The application passes a sequence of GIDs to the kern table, and gets back adjusted positions (XPlacement, XAdvance, YPlacement and YAdvance) for those GIDs. When using the type 2 lookup on a run of glyphs, it's critical to remember to not consume the last glyph, but to keep it available as the first glyph in a subsequent run (this is a departure from normal lookup behavior).

UI suggestion: This feature should be active by default for vertical text setting. Applications may wish to allow users to add further manually-specified adjustments to suit specific needs and tastes.

Script/language sensitivity: None

Feature interaction: If 'vkrn' is activated, 'vpal' must also be activated if it exists. (If 'vpal' is activated, there is no requirement that 'vkrn' must also be activated.) May be used in addition to any other feature except those which result in fixed (uniform) advance heights.

Tag: 'vpal'

Friendly name: Proportional Alternate Vertical Metrics

Function: Respaces glyphs designed to be set on full-em heights, fitting them onto individual (more or less proportional) vertical heights. This differs from valt in that it does not substitute new glyphs (GPOS, not GSUB feature). The user may prefer the monospaced form, or may simply want to ensure that the glyph is well-fit.

Example: The user may invoke this feature in a Japanese font to get Latin, Kanji, Kana or Symbol glyphs with the full-height design but individual metrics.

Recommended implementation: The font specifies alternate heights for the full-height glyphs (GPOS lookup type 1).

Application interface: For GIDs found in the vpal coverage table, the application passes the GIDs to the table and gets back positional adjustments (XPlacement, XAdvance, YPlacement and YAdvance).

UI suggestion: This feature would be off by default.

Script/language sensitivity: Used mostly in CJKV fonts.

Feature interaction: This feature is mutually exclusive with all other glyph-height features (e.g. valt and vhal), which should be turned off when it's applied. If vpal is activated, there is no requirement that vkrn must also be activated. If vkrn is activated, then vpal must also be activated if it exists.

Tag: 'vrt2'

Friendly name: Vertical Alternates and Rotation

Function: Replaces some fixed-width (half-, third- or quarter-width) or proportional-width glyphs (mostly Latin or katakana) with forms suitable for vertical writing (that is, rotated 90 degrees clockwise).

NOTE	These are a superset of the glyphs covered in the vert table.

Adobe Type Manager /NT 4.1 and the Windows 2000 OTF driver impose the following requirements for an OFF font with CFF outlines to be used for vertical writing: the vrt2 feature must be present in the GSUB table, it must comprise a single lookup of LookupType 1 and LookupFlag 0, and the lookup must have a single subtable. The predecessor feature, vert, is ignored.

A rotated glyph must be designed such that its top side bearing and vertical advance as recorded in the Vertical Metrics ('vmtx') table are identical to the left side bearing and horizontal advance, respectively, of the corresponding upright glyph as recorded in the Horizontal Metrics ('hmtx') table. (The horizontal advance of the rotated glyph may be set to any value, since the glyph is intended only for vertical writing use. The vendor may however set it to head.unitsPerEm, to prevent overlap during font proofing tests, for example.)

Thus, proportional-width glyphs with rotated forms in the vrt2 feature will appear identically spaced in both vertical and horizontal writing. In order for kerning to produce identical results as well, developers must ensure that the Vertical Kerning (vkrn) feature record kern values between the rotated glyphs that are the same as kern values between their corresponding upright glyphs in the Kerning (kern) feature.

Example: Proportional- or half-width Latin and half-width katakana characters are rotated 90 degrees clockwise for vertical writing.

Recommended implementation: The font includes rotated versions of the glyphs covered by this feature. The vrt2 table maps the standard (horizontal) forms to the corresponding vertical (rotated) forms (GSUB lookup type 1). This feature should be the last substitution in the font, and take input from other features.

Application interface: For GIDs found in the vrt2 coverage table, the application passes GIDs to the feature, and gets back new GIDs.

UI suggestion: This feature should be active by default when vertical writing mode is on, although the user must be able to override it.

Script/language sensitivity: Applies only to scripts with vertical writing capability.

Feature interaction: Overrides the vert (Vertical Writing) feature, which is a subset of this one. May be used in addition to any other feature.

Tag: 'vrtr'

Friendly name: Vertical Alternates for Rotation

Registered by: Adobe/Microsoft/W3C

Function: Transforms default glyphs into glyphs that are appropriate for sideways presentation in vertical writing mode. While the glyphs for most characters in East Asian writing systems remain upright when set in vertical writing mode, glyphs for other characters (such as those of other scripts or for particular Western-style punctuation) are expected to be presented sideways in vertical writing.

Example: As a first example, the glyphs for FULLWIDTH LESS-THAN SIGN (U+FF1C; "<") and FULLWIDTH GREATER-THAN SIGN (U+FF1E; ">") in a font with a non-square em-box are transformed into glyphs whose aspect ratio differs from the default glyphs, which are properly sized for sideways presentation in vertical writing mode. As a second example, the glyph for LEFT SQUARE BRACKET (U+005B, "[") in a brush-script font that exhibits slightly rising horizontal strokes may use an obtuse angle for its upper-left corner when in horizontal writing mode, but an alternate glyph with an acute angle for that corner is supplied for vertical writing mode.

Recommended implementation: The font includes versions of the glyphs covered by this feature that, when rotated 90° clockwise by the layout engine for sideways presentation in vertical writing, differ in some visual way from rotated versions of the default glyphs, such as by shifting or shape. The 'vrtr' feature maps the default glyphs to the corresponding to-be-rotated glyphs (GSUB lookup type 1).

Application interface: For GIDs found in the 'vrtr' coverage table, the application passes GIDs to the lookup tables associated with the feature, then gets back new GIDs.

UI suggestion: This feature should be active by default for sideways runs in vertical writing mode.

Script/language sensitivity: Applies to any script when set in vertical writing mode.

Feature interaction: The 'vrtr' and 'vert' features are intended to be used in conjunction: 'vrtr' for glyphs intended to be presented sideways in vertical writing, and 'vert' for glyphs to be presented upright. Since they must never be activated simultaneously for a given glyph, there should be no interaction between the two features. These features are intended for layout engines that graphically rotate glyphs for sideways runs in vertical writing mode, such as those conforming to Unicode Standard Annex #50: Unicode Vertical Text Layout [23].

Note that layout engines that instead depend on the font to supply pre-rotated glyphs for all sideways glyphs should use the 'vrt2' feature in lieu of 'vrtr' and vert. Because 'vrt2' supplies pre-rotated glyphs, the 'vrtr' feature should never be used with 'vrt2', but it may be used in addition to any other feature.

Tag: 'zero'

Friendly name: Slashed Zero

Function: Some fonts contain both a default form of zero, and an alternative form which uses a diagonal slash through the counter. Especially in condensed designs, it can be difficult to distinguish between 0 and O (zero and capital O) in any situation where capitals and lining figures may be arbitrarily mixed. This feature allows the user to change from the default 0 to a slashed form.

Example: When setting labels, the user applies this feature to get the slashed 0.

Recommended implementation: The zero table maps the GIDs for the lining forms of zero to corresponding slashed forms (GSUB lookup type 1).

Application interface: For GIDs in the zero coverage table, the application passes a GID to the zero table and gets back a new GID.

UI suggestion: Optimally, the application would store this as a preference setting, and the user could use the feature to toggle back and forth between the two forms. Most applications will want the default setting to disable this feature.

Script/language sensitivity: Does not apply to scripts which use forms other than 0 for zero.

Feature interaction: Applies only to lining figures, so is inactivated by oldstyle figure features (e.g. onum).

### Baseline tags

This clause defines the standard OFF Layout baseline tags. A registered baseline tag has a specific meaning when used in the horizontal writing direction (used in the 'BASE' table's HorizAxis table), vertical writing direction (used in the 'BASE' table's VertAxis table), or both, and conveys information to font users about a baseline's use. For example, the 'romn' baseline tag is commonly used to identify the baseline to layout Latin text in the horizontal, vertical, or both directions for Latin text layout.

This version of the Tag Registry identifies the baselines. All baseline tags are 4-byte character strings composed of a limited set of ASCII characters in the 0x20-0x7E range. Baseline tags consist of four lowercase letters.

| Baseline Tag | Baseline for HorizAxis | Baseline for VertAxis |
| --- | --- | --- |
| 'hang' | The hanging baseline. This is the horizontal line from which syllables seem to hang in Tibetan script. | The hanging baseline, (which now appears vertical) for Tibetan characters rotated 90 degrees clockwise, for vertical writing mode. |
| 'icfb' | Ideographic character face bottom edge baseline. (See Ideographic Character Face below for usage.) | Ideographic character face left edge baseline. (See clause Ideographic Character Face below for usage.) |
| 'icft' | Ideographic character face top edge baseline. (See Ideographic Character Face below for usage.) | Ideographic character face right edge baseline. (See clause Ideographic Character Face below for usage.) |
| 'ideo' | Ideographic em-box bottom edge baseline. (See clause Ideographic Em-Box below for usage.) | Ideographic em-box left edge baseline. If this tag is present in the VertAxis, the value must be set to 0. (See clause Ideographic Em-Box below for usage.) |
| 'idtp' | Ideographic em-box top edge baseline. (See Ideographic Em-Box below for usage.) | Ideographic em-box right edge baseline. If this tag is present in the VertAxis, the value is strongly recommended to be set to head.unitsPerEm. (See clause Ideographic Em-Box below for usage.) |
| 'math' | The baseline about which mathematical characters are centered. | The baseline about which mathematical characters, when rotated 90 degrees clockwise for vertical writing mode, are centered. |
| 'romn' | The baseline used by simple alphabetic scripts such as Latin, Cyrillic and Greek. | The alphabetic baseline for characters rotated 90 degrees clockwise for vertical writing mode. (This would not apply to alphabetic characters that remain upright in vertical writing mode, since these characters are not rotated.) |

Ideographic Em-box

[The notation <Axis>.<Baseline Tag> is used in the following description to mean the baseline tag as defined in the specified axis. For example, HorizAxis.ideo means the ideo baseline tag as defined in the HorizAxis of the BASE table. See above for a list of registered baseline tags.]

A font's ideographic em-box is the rectangle that defines a standard escapement around the full-width ideographic glyphs of the font, for both the horizontal and vertical writing directions. It is usually a square, but may be non-square as in the case of fonts used in Japanese newspaper layout that have a vertically condensed design.

The left, right, top and bottom edges of the ideographic em-box are to be determined as follows:

ideoEmboxLeft = 0

If HorizAxis.ideo defined:

ideoEmboxBottom = HorizAxis.ideo

If HorizAxis.idtp defined:

ideoEmboxTop = HorizAxis.idtp

Else:

ideoEmboxTop = HorizAxis.ideo + head.unitsPerEm

If VertAxis.idtp defined:

ideoEmboxRight = VertAxis.idtp

Else:

ideoEmboxRight = head.unitsPerEm

If VertAxis.ideo defined and non-zero:

Warning: Bad VertAxis.ideo value

Else If this is a CJK font:

ideoEmboxBottom = OS/2.sTypoDescender
ideoEmboxTop = OS/2.sTypoAscender
ideoEmboxRight = head.unitsPerEm

Else:

ideoEmbox cannot be determined for this font

Determining whether a font is CJK (Chinese, Japanese, or Korean) or not, as in the second-last "Else" clause above, can be done by checking the CJK-related bits of the OS/2.ulUnicodeRange fields.

NOTE	Font designers can specify a HorizAxis.ideo baseline in their non-CJK fonts; this can be used by applications when aligning the font with an ideographic font used on the same line of text, when the user has specified ideographic em-box alignment.

The ideographic em-box center baseline is defined as halfway between the ideographic em-box top and bottom baselines in the horizontal axis, and halfway between the ideographic em-box left and right baselines in the vertical axis. These center baselines are defined in whole character units. The division used in the calculation must round to the character unit nearest 0 if needed. Thus, for maximal precision of center baseline placement, vendors should ensure that opposite edges of the ideographic em-box box are an even number of character units apart.

Example:

The values of the ideographic baseline tags for the Kozuka Mincho font family (designed on a 1000-unit em) are:

HorizAxis.ideo = -120; HorizAxis.idtp = 880. 
Since this describes a square ideographic em-box, it is sufficient to record only the following:
HorizAxis.ideo = -120.
If HorizAxis.ideo is not present, then the following will be used for the ideographic em-box bottom and top, since this is a CJK font:
OS/2.sTypoDescender = -120; OS/2.sTypoAscender = 880.

Compatibility notes:

- Most applications expect the width of full-width ideographs in a CJK font to be exactly one em, thus it is strongly recommended that VertAxis.idtp, if present, be set to head.unitsPerEm. (The idtp baseline tag was introduced in OpenType 1.3.)

- While the OFF specification allows for CJK fonts' OS/2.sTypoDescender and OS/2.sTypoAscender fields to specify metrics different from the HorizAxis.ideo and HorizAxis.idtp in the 'BASE' table, CJK font developers should be aware that existing applications may not read the 'BASE' table at all but simply use the OS/2.sTypoDescender and OS/2.sTypoAscender fields to describe the bottom and top edges of the ideographic em-box. If developers want their fonts to work correctly with such applications, they should ensure that any ideographic em-box values in the 'BASE' table of their CJK fonts describe the same bottom and top edges as the OS/2.sTypoDescender and OS/2.sTypoAscender fields.

- Applications on platforms other than Windows that don't parse the 'OS/2' table won't have access to the OS/2.sTypoDescender and OS/2.sTypoAscender fields Thus, CJK fonts will typically have the same descender value recorded in hhea.Descender, OS/2.sTypoDescender, and HorizAxis.ideo (if present), and the same Ascender value recorded in hhea.Ascender, OS/2.sTypoAscender, and HorizAxis.idtp (if present).

See subclause 9.8 for more information about constructing CJK fonts.

Ideographic character face

[ The notation <Axis>.<Baseline Tag> is used in the following description to mean the baseline tag as defined in the specified axis. For example, HorizAxis.icfb means the icfb baseline tag as defined in the HorizAxis of the BASE table. See above for a list of registered baseline tags. ]

The Ideographic Character Face (ICF), also known as the Average Character Face (ACF), specifies the approximate bounding box of the full-width ideographic and kana glyphs in a CJK font. (This is different from the FontBBox, as described in the PostScript programming language, which is the bounding box of all glyphs in the font.) In Japanese, the term for ICF is heikin jizura.

It is typically expressed as a percentage that represents the ratio of the length of an ICF box edge to the length of an ideographic em-box edge, and is conceptualized as a square centered within the ideographic em-box. However, in OFF, the ICF box's left, bottom, right, and top edges are specified as the VertAxis.icfb, HorizAxis.icfb, VertAxis.icft, and HorizAxis.icft baselines, respectively, thus giving font designers the flexibility to specify a non-square and/or non-centered ICF box.

Font designers should set the value of the ICF box edges based on how tight or loose they want the font to appear when text is set with no tracking or kerning (beta gumi in Japanese). Therefore, the left-over boundary of the ideographic em-box around the ICF box is the default escapement of the font.

Applications can use the ICF box as an alignment tool, to ensure that glyphs touch the edges of the text frame and page objects are visually aligned to text edges. It is also useful for aligning glyphs of different sizes on the same line. In Japanese traditional paper-based workflow, the ICF box was often used for these purposes. It provides optically aligned results that are superior to using the ideographic em-box.

HorizAxis.icfb is the minimum piece of information required to define the ICF, in a CJK font. First, the ideographic em-box dimensions must be calculated as in the clause "Ideographic Em-Box" above. The ICF edges are then calculated in the following order:

If HorizAxis.icfb defined:

icfBottom = HorizAxis.icfb
margin = HorizAxis.icfb – ideoEmboxBottom

If HorizAxis.icft defined:

icfTop = HorizAxis.icft

Else:

icfTop = ideoEmboxTop – margin

If VertAxis.icfb defined:

icfLeft = VertAxis.icfb

Else:

icfLeft = margin

If VertAxis.icft defined:

icfRight = VertAxis.icft

Else:

icfRight = ideoEmBoxRight – icfLeft

Else:

ICF cannot be determined for this font

For the last case above, i.e. fonts that don't have ICF information in their 'BASE' table, an application may choose to apply a heuristic such as calculating the bounding box of some or all of the ideographic and kana glyphs, and then averaging its margin with the ideographic em-box.

The ICF center baseline is defined as halfway between the ICF top and bottom baselines in the horizontal axis, and halfway between the ICF left and right baselines in the vertical axis. These center baselines are defined in whole character units. The division used in the calculation must round to the character unit nearest 0 if needed. Thus, for maximal precision of center baseline placement, vendors should ensure that opposite edges of the ICF box are an even number of character units apart.

Example:

The values of the ICF baselines for the Extra Light and Heavy weights of the Kozuka Mincho font family (designed on a 1000-unit em, with ideographic em-box as given in the example in the previous clause) are:

Kozuka Mincho Extra Light:
VertAxis.icfb = 41; HorizAxis.icfb = -79;
VertAxis.icft = 959; HorizAxis.icft = 839. 
Since this describes a square ICF centered in a square ideographic em-box, it is sufficient to record only the following: 
HorizAxis.icfb = -79.

Kozuka Mincho Heavy:
VertAxis.icfb = 26; HorizAxis.icfb = -94;
VertAxis.icft = 974; HorizAxis.icft = 854. 
It is sufficient to record only:
HorizAxis.icfb = -94.

It is strongly recommended that each of the edges of the ICF box be equidistant from the corresponding edge of the ideographic em-box. Following this will result in more predictable results in applications that use these values. That is, for fonts based on a square ideographic em-box, the ICF box should be a centered square.

See subclause 9.8 for more information about constructing CJK fonts.

# OFF font variations

## Font variations overview

This chapter provides an overview of OFF Font Variations, including an introduction to essential concepts, a glossary of terminology, and a specification of key algorithms: coordinate normalization, and interpolation of instance values.

### General

OFF Font Variations allow a font designer to incorporate multiple font faces within a font family into a single font resource. Variable fonts – fonts that use OFF Font Variations mechanisms – provide great flexibility for content authors and designers while also allowing the font data to be represented in an efficient format.

A variable font allows for continuous variation along some given design axis, such as weight:

Figure 7.1: Continuous variation along a design axis

Conceptually, variable fonts define one or more axes over which design characteristics can vary. Weight is one possible axis of variation, but many different kinds of variation are possible. Variable fonts can combine two or more different axes of variation. For example, the following illustrates a combination of weight and width variation:

Figure 7.2: Continuous variation along multiple design axes

Typically, variable fonts will vary the design of glyph outlines. In general, however, potentially any aspect of the visual appearance may vary. For example, a font could vary line height metrics or appearance of gradients in color glyphs rather than (or in addition to) glyph outlines.

A variable font includes a table, the font variations ('fvar') table, that describes the axes of variation used by that font. This table determines how a variable font and its variation parameters will be presented to users and applications. Each axis is defined by a numeric range, using fractional values represented using the Fixed (16.16) data type. Conceptually, this provides a continuous gradient of variation, allowing for a large number of design-variation instances to be selected. Each instance would be designated by a coordinates array within the design-variation space – a specific value along each of the design axes. So, for instance, if a user or application requires a very-slightly narrower width or slightly more pronounced serifs, fine control over such axes of variation is available.

Every axis allows for a continuous range of instance-selection values, and typically there will be continuous variation in appearance for a given axis. In some cases, however, appearance can vary in discrete steps as the axis setting is changed. For example, an axis could trigger substitution to different alternate glyphs for discrete sub-ranges of axis values.

A font designer can pre-define certain instances to have particular names. For example, a font can have continuous variation on a weight axis, but the designer may identify particular variation instances as “Light” or “Semibold”. Named instances can be used for any instance in the supported design-variation space. For example, in a font with weight and width axes, named instances might include “Light”, “Extended”, or “Semibold Condensed”. Details regarding named instances are also included in the font variations table.

Weight and width are commonly-used axes of design variation, but a variable font may use a wide range of other, possible axes of variation. For more information regarding supported axes, see the font variations ('fvar') table description.

In addition to a font variations table, a variable font also includes a style attributes ('STAT') table that describes additional details about each axis of variation and about particular values (chosen by the designer) along each axis. These details include descriptor strings for those values, such as “Bold”, “Extended” or “Semi-sans”. For example, a weight/width variable font might support a “Bold Extended” variation, and the 'STAT' table would provide strings for “Bold” and “Extended” corresponding to the particular values along the weight and width axes, respectively. These strings can be used in creation of font-picker user interfaces. They can also be used for projecting members of a multi-axis font family into different models for font families that assume a limited number of axes of sub-family variation, such as a weight/width/slant model. (See 7.3.7 for more information.). Because the 'STAT' table identifies values on each axis, software never needs to parse subfamily strings and guess that string tokens such as “Halbfett” refer to a particular value on some axis.

NOTE	The style attributes table makes it possible for fonts with many design axes to be defined as a single, multi-axis family, yet still have instances across all of those axes supported in older applications that may only recognize a limited set of axes of variation, or a limited number of values on an axis. The host platform, which must support the style attributes table, can translate instances in a multi-axis family into fewer instances in multiple families that older applications will recognize.

As different variation instances of a font are selected, various items of data within a font can be adjusted accordingly. For example, a 'glyf' table can provide the default outline of a given glyph, but the outline can be adjusted in some manner to reflect different design variations. Several other data items besides glyph outlines may also need similar adjustments, including font-wide metrics, CVT values, or anchor positions within glyph-positioning lookup tables. A variable font includes required and optional tables that describe how such items within the font change from default values to different values as needed for different design-variation instances. For example, while a 'glyf' table can provide default outlines for glyphs, a glyph variations ('gvar') table would provide corresponding data that describes how each glyph outline changes for different variation instances.

A variable font has a default instance, with axis parameter values set to the defaults defined for each axis in the 'fvar' table. Several tables in the font provide default values for many different data items – such as positions of glyph outline points in the 'glyf' table, or a font-wide ascender distance in the OS/2 table. The default instance of a font uses the default values for such items without any adjustments, and the variation-specific tables are not needed. If the variation-specific tables – 'fvar', 'gvar', 'MVAR', etc. – were to be removed from the font or ignored, the remaining data would comprise a complete font for the default instance.

Font variation mechanisms for fonts using TrueType outlines were first introduced by Apple in “TrueType GX”. Some of the tables used for OFF Font Variations have been adapted from Apple's earlier specifications with some enhancements and revisions. (In particular, there are significant changes in the 'fvar' table specification in regard to both format and data values used, and the 'fmtx' table is not used.) Other extensions have also been created in order to integrate variation mechanisms into OFF. Implementers may wish to refer to Apple's specifications for historical insights, but should refer to the OFF specification as the reference for implementation of OFF Font Variations.

### Terminology

Several terms are useful in discussing font variations and will be used in this document.

OFF Font Variations: The name of the technology described in this chapter.

Font face: A logical collection of glyph data sharing specific design parameters, along with associated metric data, and names or other metadata.

Font resource: OFF data that includes (at least) the minimal set of tables needed to comprise a functional font face.

NOTE 1	Within OFF font files, each table directory and the tables it references comprise a font resource. A well-formed .OTF or .TTF file includes a single font resource; a well-formed .OTC or .TTC file includes one or more font resources. A font resource without variation-related tables provides data for a single font face. A single font resource that includes variation-related tables can provide data for multiple font faces.

Font family: A set of font resources that have a common family name – the same string values for name ID 16 (Typographic Family Name) or name ID 1.

NOTE 2	It is assumed that all fonts within a family will share certain design characteristics, but differ in others. The design characteristics that are different potentially might be supported using font variations.

Axis of variation: A designer-determined variable in a font face design that can be used to derive multiple, variant designs within a family.

Variable font: A font resource that supports multiple font faces in a family along designer-defined axes of variation using OFF Font Variations mechanisms – that is, by means of variation tables and other variation data in tables generally.

Glyph design grid: The visual, two-dimensional space in which a font’s glyph outlines are designed.

Design-variation space: An abstract, multi-dimensional space defined by the axes of variation used by a font designer when designing a font family. In the context of a variable font, the variation space refers to the n-dimensional space defined by the axes of variation specified in the font’s 'fvar' table.

NOTE 3	A variation space can have one or more axes. In a variable font, the variation space is bounded by minimum and maximum values specified in the 'fvar' table. The zero origin has no special significance within a design-variation space. Within a variable font, however, the zero origin (using normalized coordinate scales – defined below) is a marked position since it corresponds to the font face represented directly by the font resource's name, glyph and metric tables without reference to any variation tables or other variation data.

Variation data: Data used in a variable font to describe the way that values for data items in the font are adjusted from default values to alternate values needed for different instances within the variation space.

Variation tables: OFF tables specifically related to Font Variations, including the following:

- Axis variations (‘avar’) table

- CVT (control value table) variations (‘cvar’) table

- Font variations (‘fvar’) table

- Glyph variations (‘gvar’) table

- Horizontal metrics variations (‘HVAR’) table

- Metrics variations (‘MVAR’) table

- Vertical metrics variations (‘VVAR’) table

NOTE 4	The 'fvar' table describes a font’s variation space, and other variation tables provide variation data to describe how different data items are varied across the font’s variation space. Note that not all of these tables are required in a variable font. Also note that variation data for certain font data items may be contained in other tables not specifically related to Font Variations. In addition, certain tables not specifically related to Font Variations are required in variable fonts. See subclause 7.1.6 (Variation data tables and miscellaneous requirements) for more details.

Point: In order to avoid ambiguity, point will be used only to refer to (X, Y) positions within the glyph design grid. When discussing the design-variation space, position will be used to refer to positions within that space.

Variation instance: A font face corresponding to a particular position within the variation space of a variable font.

Named instance: A variation instance that is specifically defined and assigned a name within the ‘fvar’ table.

User coordinate scale: The numeric scale used to characterize a given axis of variation, and the scale used by applications when selecting instances of a variable font.

NOTE 5	Some axes of variation have a prescribed, limited range, expressed in terms of the user scale. When using a particular variable font, the user scale for a given axis is bounded by minimum and maximum specified within the 'fvar' table, and may be a sub-range of the valid range for that axis generally.

Normalized coordinate scale: When processing variation data in a variable font to derive values for particular instances, a normalization process is applied to map user-scale values on each axis to a normalized scale applicable within that font that ranges from -1 to 1.

NOTE 6	The 'fvar' table specifies user-scale minimum, default and maximum values for each axis. In the normalization process, these get mapped to -1, 0 and 1 respectively, with other values along each axis mapping to intervening points. Mapping of other values is modulated by the 'avar' table, if present. All of the variation data within the font makes reference to axis values or positions within the font’s variation space in terms of normalized-scale values.

Tuple / N-tuple: An ordered set of coordinate values used to designate a position within the variation space of a font.

NOTE 7	“Tuple” is used here with a meaning that is consistent with conventional usage in computer science and mathematics. In Apple TrueType specifications, “tuple” has been used with a different meaning to refer to sets of variation data associated with a particular region of the font’s design variation space. In this document, “tuple variation data” is used for that meaning, and “n-tuple” is used in many cases so as to avoid confusion with usage in Apple specifications.

Region: A sub-space (that is, some portion or subset) of the design variation space over which a variation adjustment is described.

NOTE 8	A region involves all of the axes of the font’s variation space; it is not a “sub-space” in the sense of involving only a subset of axes. In normalized coordinates, regions are always rectilinear: they have straight edges and right-angled corners. Variation data may be defined for up to 65,535 regions in a font’s variation space.

Master: A set of source font data that includes complete outline data for a particular font face, used in a font-development workflow.

NOTE 9	Some font-development workflows utilize several masters as source data for creating font resources for different faces within a family. Multiple source masters might also be used to create a variable font. Each source master would correspond to a single instance in the variation space, and possibly might correspond to variation data for a particular region in the variable font. Whereas each master includes complete outline data, however, the variable font includes only a single set of complete outline data (in the 'glyf' or 'CFF2' table), which is complemented with variation data for different regions to represent the full range of instances supported by the font.

Deltas / Adjustment deltas: Numeric values in variation data that specify adjustments to default values of data items for particular regions within the variation space or for sub-ranges within a particular axis.

Delta set: A set of adjustment deltas associated with a particular region of the variation space.

Scalars: Co-efficient values applied to deltas to derive adjustment values needed for a particular variation instance.

Interpolation: The process of deriving adjusted values for some font data items, such as the X and Y coordinates of glyph outline points, for a particular variation instance.

### Variation space, default instances and adjustment deltas

A variable font supports one or more axes of variation. Commonly-used axes of variation should be registered, though custom, designer-defined axes can also be used. Each axis has a distinct tag that is used to identify it in the 'fvar' table. See the 'fvar' table description for more details about axis tags.

The specification of axes used for a variable font is given in the 'fvar' table, along with minimum, default and maximum values for each axis. This defines a variation space for the font. It is entirely up to the designer what range of design variation is supported for each axis, and how the designs align with the scale for each axis.

For example, a variable font may support a full range of weights from thin to black:

Font A: Thin to black weight variation

But a designer might also choose to support only a limited weight range:

Font B: Regular to black weight variation

The variable font has a default instance, which corresponds to the position in the variation space with coordinates set to the default values for each axis specified in the 'fvar' table. The default instance uses default values for various data items that are provided directly in non-variations-specific font tables, such as the grid coordinates of outlines points for a glyph in the 'glyf' table.

Default glyph outline data in a ‘glyf’ table entry

All other instances have non-default coordinate values for one or more axes. These other instances are supported by variation data that provide adjustment deltas for various font data items that produce an adjustment from their default values.

Default glyph outline and adjusted point positions for a non-default instance

Detail inset showing contour point adjustments

Typically, deltas are provided for the extremes on each variation axis, though deltas can be provided for other positions in the variation space as well. (See below for more details.) For axis positions between the default and minimum or maximum extreme, other values are interpolated.

The font designer can determine which design is considered the default, and what deltas are provided. For example, a font with thin-to-black weight variation might be implemented with Regular (400) as the default, and Thin (100) and Black (900) as minimum/maximum values. In this case, variation data would include deltas for the Thin extreme and also deltas for the Black extreme.

But a different font with thin-to-black weight variation might be implemented with Thin as the default and minimum value and Black as the maximum. In this case, variation data might include deltas for only the Black extreme.

Note that a consideration in the choice of default is desired behavior in legacy applications or platforms that do not support Font Variations: in such software, only the default instance of a variable font will be supported.

A common process for developing a variable font involves the use of multiple, master source fonts. Each master provides complete glyph outline data for designs for a different position within a variation space. For example, a font designer might create fonts for thin and heavy extremes along a weight axis.

From these two source masters, font tools can derive a variable font that has complete glyph outlines for a default weight plus deltas for one or more non-default weights, including the minimum or maximum weights.

Note that each of the source, master fonts has complete outline data for a particular design variant. In contrast, the variable font has complete outlines for only one variation instance, with all other instances derived using the default outlines plus deltas. Each source master may correspond to a region with associated variation data in the variable font, though the relationship between source masters and the sets of variation data within the font will depend on the nature of the designs and on the tools used to produce the variable font.

Also note that a requirement for using multiple, master, font sources to derive a variable font is that corresponding glyph outlines must be point-compatible: they shall have the same number of contours and the same number of points in each contour.

### Coordinate scales and normalization

Positions within the variation space can be represented as an n-tuple – an ordered list of coordinate values. Examples will be seen below. The coordinate values of an n-tuple may use user-axis scales, or may use normalized scales. The precise relationship between these scales will be described.

User coordinates refers to an n-tuple of coordinate values expressed using user axis scales. User scales refer to the numeric scales used to describe a variation axis within the 'fvar' table. Each variation axis uses its own numeric scale, as appropriate to the nature of that axis of variation. The scales for registered axis tags are defined as part of the axis tag registration, though different fonts may support different sub-ranges of an axis scale. In this way, the 'fvar' table of a given font defines a particular coordinate system for the variation space of that font that may be unlike that of other fonts.

Whereas the definitions in the 'fvar' table are expressed in user coordinates, the variation data formats used within a variable font use a normalized coordinate system – normalized coordinates – in which the minimum, default and maximum values specified for each axis in the 'fvar' table are mapped to -1, 0 and 1, respectively.

For example, the following figure illustrates the user coordinate system of the variation space for a possible font with weight and width axes of variation. Various positions within the design space are indicated, including positions corresponding to the default, minimum and maximum values for each axis defined in the 'fvar' table.

(900, 200)

(900, 100)

(900, 50)

(100, 200)

(100, 100)

(100, 50)

(400, 50)

(400, 200)

(400, 100)

(184, 148)

The following figure illustrates the normalized coordinate system for the same positions in the design space:

(1, 1)

(1, 0)

(1, -1)

(-1, 1)

(-1, 0)

(-1, -1)

(0, -1)

(0, 1)

(0, 0)

(-0.72, 0.48)

The normalization transformation uses a default transformation followed by a secondary modification of the transformation defined in the 'avar' table, if present. An 'avar' table does not affect the mapping of minimum, default, and maximum values to -1, 0 and 1; it can only affect mapping of intervening values. This is described in more detail below.

The default normalization mapping divides the variation range for each axis into two segments: minimum value to default value, and default value to maximum value. The minimum, default and maximum values are mapped into -1, 0 and 1 respectively. Within each segment, all other values are interpolated linearly, as follows:

Let userValue be the user-scale coordinate value for a user-selected instance value for a given axis, let defaultNormalizedValue be the normalized instance value, let axisMin be the minimum value for the axis specified in the ‘fvar’ table, etc.

Force the user-scale coordinate value to be in range by clamping to the minimum and maximum values:

if userValue < axisMin
    userValue = axisMin;
if userValue > axisMax
    userValue = axisMax;

Interpolate values linearly within the different segments:

if (userValue < axisDefault)
{
    defaultNormalizedValue = -(axisDefault - userValue) / (axisDefault – axisMin);
}
else if (userValue > axisDefault)
{
    defaultNormalizedValue = (userValue – axisDefault) / (axisMax – axisDefault);
}
else
{
    defaultNormalizedValue = 0;
}

If an 'avar' table is present, then an additional normalization step is performed for each axis to compute the final normalized value. Within the 'avar' table, AxisValueMap records map default normalized values for an axis to modified normalized values. Pairs of consecutive AxisValueMap records define segments within the range for a given axis. Within a segment, intermediate values are interpreted linearly. Starting with the defaultNormalizedValue computed as above, the additional normalization step proceeds as follows:

Retrieve the SegmentMaps record for a given axis from the avar.axisSegmentMaps array using the index for the axis as defined in the 'fvar' table.

Scan the AxisValueMaps records in the SegmentMaps.axisValueMaps array to find the first record that has an AxisValueMaps.fromCoordinate value greater than or equal to defaultNormalizedValue. Designate this record as endSeg. (Note that endSeg cannot be the first map record, which is for -1.)

If endSeg.fromCoordinate equals defaultNormalizedValue, then set finalNormalizedValue to endSeg.toCoordinate. Return this value and end.

Else (endSeg.fromCoordinate is strictly greater than defaultNormalizedValue): designate the preceding AxisValueMaps record as startSeg.

finalNormalizedValue is computed as follows:

ratio = (defaultNormalizedValue - startSeg.fromCoordinate) /
        (endSeg.fromCoordinate - startSeg.fromCoordinate)

finalNormalizedValue = startSeg.toCoordinate + ratio *
                       (endSeg.toCoordinate - startSeg.toCoordinate)

See 7.3.1 for details on the structures mentioned above.

When processing variation instance coordinates and variation data, the amount of precision used and the handling of rounding can potentially have noticeable impacts on visual results. In order to ensure consistent behavior for a given font across implementations, implementations must observe the following requirements in relation to precision and rounding:

- The input to normalization must be in 16.16 format. If an application provides an input value represented as either a float or double data type, the method described below must be used for conversion to 16.16.

- The math calculations for normalization, specified above, are done in 16.16.

- After the default normalization calculation is performed, some results may be slightly outside the range [-1, +1]. Values must be clamped to this range:

if result < -1
    result = -1;
if result > 1
    result = 1;

- If an ‘avar’ table is present, math calculations are done in 16.16, and results are clamped to the range [-1, +1] as above.

- Convert the final, normalized 16.16 coordinate value to 2.14 by this method: add 0x00000002, and sign-extend shift to the right by 2.

- The 2.14 result must be stored and returned in certain operations, as described below.

- For subsequent calculations – calculation of interpolation scalars or accumulation of scaled delta values – the 2.14 representation may be converted to float, 16.16 or other implementation-specific representations. It is recommended that at least 16 fractional bits of precision be maintained, and that any rounding be done at the last point before a value is used.

When converting from float or double data types to 16.16, the following method shall be used:

- Multiply the fractional component by 65536, and round the result to the nearest integer (for fractional values of 0.5 and higher, take the next higher integer; for other fractional values, truncate). Store the result in the low-order word.

- Move the two’s-complement representation of the integer component into the high-order word.

NOTE	Apart from conversion of higher-precision representations to 16.16, this specification has no other requirements for instance coordinates, scaled deltas or derived instance values to be rounded. For example, for contour point coordinates after deltas have been applied, a rasterizer implementation can use a high-precision floating type or can round to a lower-precision representation depending on implementation-specific requirements. Different implementations can use different precision for calculation of instance values, resulting in minor visual differences. If data for a font instance is converted or exported to another representation — for example, in dynamic generation of a static font for a given instance — there can be minor differences between the derived static font versus the source variable font with that instance selected.

A normalized value in 2.14 representation must be obtained exactly as specified in steps 1 to 5 above. In fonts with TrueType instructions, this exact value must be returned by the GET VARIATION instruction. (See TrueType Instruction Set [[# Get Variation]].) If a font has OFF Layout tables in which FeatureVariation tables are used, this exact value must be used when comparing with axis range values specified in a condition table.

#### 'avar' normalization example

The following example illustrates how normalization using 'avar' mappings works.

Suppose that an axis in a font has minimum value of 100, a default of 400, and maximum of 900. And suppose that the instance selected has a user coordinate of 250. By the algorithm described above, the default normalized value will be calculated as follows:

defaultNormalizedValue = -(axisDefault - userValue) / (axisDefault - axisMin)
                       = -(400 - 250) / (400 - 100)
                       = -150 / 300
                       = -0.5

Suppose also that the font has an 'avar' table with the following mappings (AxisValueMap records) for this axis:

| Record index | fromCoordinate | toCoordinate |
| --- | --- | --- |
| 0 | -1.0 | -1.0 |
| 1 | -0.75 | -0.5 |
| 2 | 0 | 0 |
| 3 | 0.4 | 0.4 |
| 4 | 0.6 | 0.9 |
| 5 | 1.0 | 1.0 |

Given the default normalized value of -0.5, the relevant segment is defined by record 1 and record 2:

- The first record having a fromCoordinate greater than or equal to defaultNormalizedValue is record index 2. Thus, record 2 is endSeg.

- endSeg.fromCoordinate is strictly greater than defaultNormalizedValue. Thus, the preceding record, record 1, is startSeg.

Thus, the final normalized value is computed as follows:

ratio = (defaultNormalizedValue - startSeg.fromCoordinate) /
        (endSeg.fromCoordinate - startSeg.fromCoordinate)
      = (-0.5 - (-0.75)) / (0 - (-0.75))
      = 0.3333

finalNormalizedValue = startSeg.toCoordinate + ratio *
                       (endSeg.toCoordinate - startSeg.toCoordinate)
                     = -0.5 + 0.3333 * (0 - (-0.5))
                     = -0.3333

The following table shows how several normalized coordinate values would be modified by this 'avar' data:

| Default normalized value | Final normalized value |
| --- | --- |
| -1.0 | -1.0 |
| -0.75 | -0.5 |
| -0.5 | -0.3333 |
| -0.25 | -0.1667 |
| 0 | 0 |
| 0.25 | 0.25 |
| 0.5 | 0.65 |
| 0.75 | 0.9375 |
| 1.0 | 1.0 |

### Variation data

Variation data provide data that describes variation of particular font values over the variation space. For example, variation data in the 'gvar' table describes how glyph outlines in the 'glyf' table are transformed by specifying how individual points in a glyph outline get moved for different variation instances.

Variation for a given font value is expressed as combinations of deltas that apply to different regions of the variation space, and that are combined in a weighted manner to derive adjustments for instances at different positions in the variation space. Each delta in the variation data is associated with a specific region of the variation space over which it has an effect. The aggregate combination of deltas and their associated regions comprise the variation data. The variation data for different items in a font are stored in different locations. For example, variation data for entries in a 'glyf' table are stored in a 'gvar' table; variation data for certain entries in the 'OS/2' table is stored in an 'MVAR' table. In the case of outline data in a 'CFF2' table, variation data is stored within the 'CFF2' table itself. See the following section below for more details.

As mentioned, each delta value is associated with a particular region of the variation space over which it applies. The effective region for a delta is always rectilinear (in normalized coordinates). Therefore, this region can always be specified by a pair of n-tuples designating positions at diagonal-opposite corners of the region. Within the specified region, variation effects will vary from zero change to some peak change at a particular position within the region. Thus, in the general case, there are three positions that matter: the diagonal-opposite corners that define the extent of the region, and a position at which the peak change occurs.

NOTE 1	The figures shown below will use two axes of variation. The concepts and the statements made, however, apply to fonts with any number of axes of variation: regions are always rectilinear, and the diagonal-opposite corners plus a peak are the positions that describe a region.

This general case is not the most common in practice. In most cases, there is a need to describe a maximal variation at an outer position of the variation space that diminishes to zero change at the zero origin — the default instance. In this case, then, the zero origin is one of the corner positions for the applicable region, and the peak change occurs at the diagonal-opposite position. For this common case, then, the effective region

(1, 1)

and peak position can be described using a single n-tuple.

The more general, but less common, case involves arbitrary regions, illustrated earlier; these are referred to as intermediate regions. In these cases, the variation data requires three n-tuples: one for the peak-change position, and two for start and end positions at diagonal-opposite corners.

Delta values in the variation data specify a maximal adjustment for an instance at the peak position. The effects taper off for other instances, falling to zero adjustment for instances outside the region of applicability. When a given variation instance is selected, a scalar value is calculated and applied to a given delta to derive a net adjustment associated with that delta for that instance. These scalars will always be in the range 0 (zero adjustment) to 1 (maximal adjustment). Specific details on this scalar calculation are provided below.

An example will help to explain these concepts. Consider a single-axis font with weight variation. A particular glyph outline defined in the ‘glyf’ table might have a pair of points (among others) that are on-curve points on opposite sides of a stem. The entry in the ‘glyf’ table would specify glyph-design-grid coordinates for these points for the default instance of the font, perhaps corresponding to the regular weight:

(840, 700)

(720, 700)

Variation data would be defined for the maximum value on the weight axis, corresponding to 1.0 in the normalized weight scale. This data would provide X and Y deltas for the two contour points to shift their positions as needed for the heaviest-supported weight instance:

(980, 710)

(760, 710)

In this example, the first point would have X and Y deltas of +40 and +10, respectively; the second point would have deltas of +140 and +10. These provide a maximal adjustment of the outline points, applied when the user-selected instance is at the maximum weight. For weights between the default and the maximum, such as a normalized weight value of 0.5, the effect is scaled back.

(910, 705)

(740, 705)

In this case, a scalar co-efficient of 0.5 is applied to the delta values.

The scalar calculation can be thought of as a function that maps each normalized axis value from -1 to 1 onto a scalar range of 0 to 1. Each region that has associated variation data has its own scalar function, and the scalar function is defined precisely by the region description.

For example, in a single-axis font, if a delta is provided for the region from 0 to 1 with the peak effect at 1, the scalar function would be as follows:

This example considers a non-intermediate region. The same concepts can be generalized to intermediate regions. An intermediate region has start and end axis values between which there is some adjustment effect, and a peak axis value at which the full adjustment effect is applied. The scalar function has a triangular shape within the applicable range, with a value of 1.0 at the peak axis value, 0 at or below the start axis value, and 0 above the end axis value.

When generalizing to two or more axes, similar concepts apply, but contributions for each axis are combined into an overall effect. Scalars are calculated for each axis, and the per-axis scalars are multiplied together to produce an overall scalar for the given delta and the given instance. For example, the following graph illustrates an approximation of the scalar function for a region in a two-axis font with peak at (1, 1):

Since the scalar value calculated for each axis is between 0 and 1, the product when scalars for each axis are multiplied together is also between 0 and 1. The maximal adjustment effect for a given delta is obtained only when the instance axis values for all axes align with the peak coordinate values for the region associated with that delta.

The minimum and maximum values specified for an axis in the ‘fvar’ table determine limits on instances that can be selected by a user. If a user requests an instance with an axis value below the minimum, the minimum value is used; or if an axis value above the maximum is requested, the maximum is used. Thus, when processing variation data for a selected instance, the normalized axis values will always be between -1 and 1.

With that constraint assumed, let us consider the scalar value for a given delta when instance axis values are outside the region of applicability. If the selected instance is out of range on any axis, then the scalar value pertaining to that axis will be 0. As mentioned, per-axis scalars are multiplied together to produce an overall scalar. Thus, if the selected instance is out of range on any axis, then the overall scalar for that delta will be 0, and no adjustment from that delta will be applied.

When a delta is provided for a region defined by n-tuples that have a peak value of 0 for some axis, then that axis does not factor into scalar calculations. This means that the adjustment effect is the same for any value in that axis, if other axis values remain constant. In effect, the region of applicability spans the full range for the zeroed axis. For example, suppose a font has two axes, weight and width, and that deltas are provided for a region from (0, 0) to (1, 0). In this case, the deltas are applicable for any instance value on the second axis (width), so long as the instance value in the first axis (weight) is in range:

In this case, the scalar function for the second axis (width) is, effectively, a constant value of 1, with no effect on the net scalar calculation. The following graphs illustrate the scalar functions for each of the two axes, weight and width, in this example:

For a given font value, deltas may be provided for several different regions in the variation space. When a particular variation instance is selected, zero, one or many of those deltas may have an effect, according to whether the position of the instance falls within the region of applicability for each delta. Different scalars are calculated for each applicable delta, and the scaled values for applicable deltas are combined to derive a net adjustment.

When creating a single-axis font, deltas will be required for both the minimum and maximum extremes of that axis. (Both extremes, that is, unless one is also the default.) Additional intermediate-region deltas may also be provided. When creating a multi-axis font, deltas would typically be provided for the minimum and maximum extremes on each axis. The following figure illustrates this for a two-axis font:

Deltas for peak at (1, 0)

Deltas for peak at (-1, 0)

Deltas for peak at (0, -1)

Deltas for peak at (0, 1)

Default values

Two-axis font with deltas at minimum/maximum extremes on each axis

As noted above, when deltas are specified for a region with some axis value being zero, then the deltas apply to all values on that axis. Therefore, for the instance at position (1, 1), deltas for (1, 0) and (0, 1) will both apply. This means that the adjustments for the (1, 0) deltas and the adjustments for the (0, 1) deltas will both be applied to produce a combined effect. If the adjustments made for each axis are entirely independent of the adjustments for the other axis, then the two sets of deltas may be sufficient to provide the intended values for the (1, 1) instance.

Often, however, these two sets of deltas alone will not be sufficient to provide the desired results for all instances, and that additional deltas are required for the (1, 1) position in addition. Generalizing, in a multi-axis font, it will often be the case that at least some deltas are needed for the corner extremes as well as for the axis end points.

Deltas for peak at (1, 1)

Deltas for peak at (1, 0)

Deltas for peak at (1, -1)

Deltas for peak at (-1, 1)

Deltas for peak at (-1, 0)

Deltas for peak at (-1, -1)

Deltas for peak at (0, -1)

Deltas for peak at (0, 1)

Default values

Deltas at minimum/maximum extreme plus extrema-intersection corners

As noted in the Variation Space, Default Instances and Adjustment Deltas section above, the default instance can correspond to the minimum or maximum value on one or more axes. This can allow variations across a variation space to be implemented using fewer regions and associated delta data. The following figures illustrate some additional possibilities for a two-axis font.

Deltas for peak at (1, 0)

Deltas for peak at (0, -1)

Deltas for peak at (0, 1)

Default values

Deltas for peak at (1, 1)

Two-axis font with default at minimum for one axis

Deltas for peak at (1, 0)

Deltas for peak at (0, 1)

Deltas for peak at (1, 1)

Default values

Two-axis font with default at minimum for both axes

NOTE 2	Deltas for corner extremes are optional. Depending upon the needs of a particular font design, deltas can be added for none, some or all of the corner extremes. In the first example above, the design-space corners in the upper left and lower left of the diagram correspond to the minimum and maximum values for one of the axes, and so deltas in those corners are needed to provide variation on that axis. But deltas for the upper right and lower right corners are optional; in this example, supplemental deltas are added only for the upper right corner.

As noted above, an intermediate region provides an axis scalar function with a triangular, or “tooth”, shape. A pair of intermediate regions that barely overlap and that have a sharp incline at the overlap can be used to provide an inflection point along an axis in regard to some variation behavior.

Note that each intermediate region has its own associated delta values, and deltas can be used to give some sharp transition at the overlap point. For example, contour points could suddenly shift to make some element of a glyph's structure appear or disappear, as illustrated in the following figure.

Glyph structure simplified at heavier weights

NOTE 3	When using such techniques, placement of such a transition point along an axis and placement of named instances should be considered together so that sharp transitions do not occur close to a named instance. This will avoid any possibility of inconsistent behavior in different applications when using named instances that might arise due to small discrepancies in processing the numeric values.

NOTE 4	When using such techniques, it is important to bear in mind that some applications will support selection of arbitrary instances, including those with axis values in the overlapping range, and that, in the overlapping range, scaled deltas for both of the intermediate regions will apply with cumulative effect. Some design iteration may be needed, with small adjustments to delta values or the way that the regions overlap, in order to avoid unexpected or undesired results in the transitional range.

NOTE 5	The above figure illustrates the use of intermediate regions to implement a “stroke-reduction” effect. Another implementation technique that can be used to change the structure of a glyph for particular variation-axis value ranges is glyph substitution. The OFF Layout [[Required Variation Alternates]] feature in combination with a [[FeatureVariations]] table within the 'GSUB' table can be used to perform glyph substitutions when a variation instance is selected in some range along one or more axes. This may be an easier and more-easily maintained technique, and is generally recommended for achieving such effects.

The above has provided an overview of the basic concepts involved in variation data: regions of applicability, per-axis and overall scalars, and combined effects of multiple, applicable deltas. A detailed specification of the interpolation process is provided below.

### Variation data tables and miscellaneous requirements

The previous subclause identified X and Y coordinates of glyph outline points as data items that can be adjusted for different variation instances. Many other data items in a font can also be adjusted, including the following:

- Font-wide metric values in the ‘OS/2’, ‘hhea’, vhea’ or ‘post’ tables.

- Glyph metric values in the ‘hmtx’, ‘vmtx’ or VORG tables.

- PPEM ranges in the ‘gasp’ table.

- Anchor positions, and adjustments to glyph positions or advance in the ‘GPOS’ or ‘JSTF’ tables.

- X or Y coordinates for ligature caret positions in the ‘GDEF’ table.

- X or Y coordinates for baseline metrics in the ‘BASE’ table.

- CVT values.

- Gradient placement and colour stop offsets, colour alpha values, and transformations for colour glyphs in the ‘COLR’ table.

A variable font may contain variation data for any or all of these. The variation data for different items is provided in various tables within a font.

NOTE 1	While several data items in a font may require adjustment for different instances, there will be other items that do not change across instances. For example, the font family and unitsPerEm are not impacted by variation. It should be noted in particular, however, that certain values that can potentially be impacted by variations are not supported with variation data. In particular, the xMin, yMin, xMax, yMax, macStyle and lowestRecPPEM fields in the font header (‘head’) table are not supported by variation data and should only be used in relation to the default instance for the font. Also, variations for values in the kerning (‘kern’) table are not supported; variable fonts should handle kerning using the ‘GPOS’ table.

Two tables are required in all variable fonts:

- A font variations (‘fvar’) table is required to describe the variations supported by the font.

- A style attributes (‘STAT’) table is required and is used to establish relationships between different fonts belonging to a family and to provide some degree of compatibility with legacy applications by allowing platforms to project variation instances involving many axes into older font-family models that assume a limited set of axes.

A variable font shall contain some other variation-related data, according to the ways in which the design varies, but no other specific type of variation-related data is required in all variable fonts.

If a variable font has TrueType outlines in a ‘glyf’ table, the outline variation data will be provided in the glyph variations (‘gvar’) table. Variation data for CVT values can be provided in the optional CVT variations (‘cvar’) table.

If a variable font has PostScript-style outlines in a Compact Font Format 2.0 (‘CFF2’) table, the ‘CFF2’ table itself can also contain the associated variation data.

NOTE 2	A ‘CFF2’ table can be used in non-variable fonts as well as in variable fonts. Also note that variations for outlines using the Compact Font Format version 1.0 (‘CFF ’) table are not supported.

The metrics variations (‘MVAR’) table is used to provide variation data for various font-wide metrics or other numeric values in the ‘gasp’, ‘hhea’, ‘OS/2’, ‘post’ and ‘vhea’ tables. An ‘MVAR’ table should be added if adjustments to any of these values are required. Note that it is not required to provide variation data for all of the data items covered by the ‘MVAR’ table: variation data is optional for all items. If there is no variation data for a given item, the default value applies to all instances.

NOTE 3	Apple platforms allow for use of a font metrics (‘fmtx’) table to specify various font-wide metric values by reference to the X or Y coordinates of contour points for a specified glyph. OFF font variations does not use the font metrics table.

The ‘hmtx’ and ‘vmtx’ tables provide horizontal and vertical glyph metrics. Variation data for horizontal and vertical glyph metrics can be provided using the horizontal metrics variations (‘HVAR’) and vertical metrics variations (‘VVAR’) tables.

In a font with TrueType outlines, the rasterizer combines ‘hmtx’ and ‘vmtx’ values with glyph xMin, xMax, yMin and yMax values in the ‘glyf’ table to generate four “phantom” points that correspond to the glyph horizontal and vertical metric values. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) In a variable font, the variation data for a glyph in the ‘gvar’ table will include adjustment deltas for the glyph’s phantom points. As a result, interpolated glyph metrics for a given instance can be obtained by interpolating the phantom point positions for the instance. This may be costly for some text-layout operations, however. In order to provide the best performance on all platforms, it is recommended that all variable fonts with TrueType outlines include an ‘HVAR’ table. If the font supports vertical layout and includes ‘vhea’ and ‘vmtx’ tables, it is recommended that the font include a ‘VVAR’ table.

The CFF2 rasterizer does not generate phantom points, and CFF2 variation data will not include adjustment deltas for phantom points. For this reason, in a variable font with CFF2 outlines, an ‘HVAR’ table is required. If the font supports vertical layout, then a ‘VVAR’ table is required.

NOTE 4	The ‘hdmx’ and ‘VDMX’ tables are not used in variable fonts.

If a font has OFF Layout tables, variation data for values from the ‘GDEF’, ‘GPOS’ or ‘JSTF’ table will be included, as needed, within the ‘GDEF’ table. Variation data for the ‘BASE’ table will be included, as needed, within the ‘BASE’ table itself.

In some variable fonts, it may be desirable to have different glyph-substitution or glyph-positioning actions used for different regions within the font’s variation space. For example, for narrow-width or heavy-weight instances in which counters become small, it may be desirable to make certain glyph substitutions to use alternate glyphs with certain strokes removed or outlines simplified to allow for larger counters. Such effects can be achieved using a feature variations subtable within either the ‘GSUB’ or ‘GPOS’ table. See "OFF Layout Common Table Formats" for more information.

In a colour font using the COLR table, variation data can be included in the COLR table, as needed, for variable items in colour glyph descriptions.

In a variable font with TrueType outlines, the left side bearing for each glyph must equal xMin, and bit 1 in the flags field of the ‘head’ table must be set.

In all variable fonts, bit 5 in the flags field of the ‘head’ table must be cleared. (On certain platforms, bit 5 affects metrics in vertical layout. Bit 5 must be clear to ensure compatible behavior on all platforms.)

### Algorithm for interpolation of instance values

The process of interpolating adjusted values for different variation instances is used for all font data items that require variation — positions of outline glyph points, ascender or other font-wide metrics, etc. The interpolation process involves the following:

- Determining the deltas that are applicable for that instance.

- For each applicable delta, calculating per-axis scalars for that instance, then multiplying the per-axis scalars together to produce an overall scalar for that delta.

- Scaling each applicable delta by the calculated scalar for that delta.

- Combining all of the scaled deltas to produce an overall adjustment.

When processing the ‘gvar’ table, there is an additional step in calculations, which is to infer delta adjustments for points when deltas are not given explicitly. This applies only to the ‘gvar’ table, and is described in the ‘gvar’ table subclause.

As described earlier, an instance axis value that is outside the region of applicability for a given delta is equivalent to having a per-axis scalar value of zero. Also, having an axis that has no effect in relation to a given delta (the n-tuples have a peak value of zero for that axis) is equivalent to having a per-axis scalar value of one. Thus, determination of applicability and axis interactions can all be combined into a step of deriving an overall scalar.

The description of the interpolation process below will refer to start, peak, and end coordinate values. As described earlier, an intermediate region is described using three n-tuples, two for diagonal-opposite corners (start and end) that specify the extent of the region, and a peak. Non-intermediate regions have one of the corners at the peak and the other corner at the zero origin. In some variation data structures, a non-intermediate region is specified using a single n-tuple, that of the peak. In this case, the start and end coordinates are implicit: one is the same as the peak, and the other is the zero origin.

In order for the definition of a region within variation data to be valid, start, peak and end values must be well ordered. That is, for each axis, the start axis coordinate must be less than or equal to the peak coordinate, and the peak coordinate must be less than or equal to the end. Also, the start and end coordinates must both be non-negative or non-positive — they cannot cross zero.

In the discussion to this point, individual deltas have been described as having an associated region of applicability. Variation data can be organized in different ways. In some cases, as in the ‘gvar’ table, several deltas corresponding to many target items (all the outline points of a glyph) and a single region of the variation space are organized together. In some other cases, as in the ‘MVAR’ or ‘CFF2’ tables, deltas covering multiple regions are organized together by individual target items. In either case, each individual delta is associated with a particular region of the variation space. The following description of the interpolation process will refer to interpolating a value for an individual item, but when applied to particular contexts such as the ‘gvar’ table, it should be understood that the same calculations are applied to many different items in parallel.

As described above, the effect of a given delta is modulated by a scalar function ranging from 0 to 1, with a value of 1 for an instance at the peak position of the region associated with that delta. The overall scalar is the product of per-axis scalars, and each per-axis scalar is calculated as a proportion of the proximity of the instance coordinate value to the peak coordinate value relative to the distance of the peak from the edges of the region.

For example, consider an intermediate region (green in the figure below) in a two-axis variation space, with corners at (0.3, 0.15) and (1, 1), and a peak (blue in the figure below) at (0.7, 0.5):

(1, 1)

(0.7, 0.5)

(0.3, 0.15)

Then consider the instance (red in the figure below) at (0.5, 0.35). The per-axis scalars will be the ratio of the distance of the instance coordinate value from the nearest edge of the region divided by the distance of the peak from that edge:

(1, 1)

(0.7, 0.5)

(0.3, 0.15)

(0.5, 0.35)

The overall scalar for the instance in relation to this region will be the product of the two axis scalars: 
0.5 × 0.571429 = 0.285714.

The detailed algorithm for calculating the interpolated value for a given target item and for a given instance is as follows.

- Let instanceCoords be the normalized instance coordinate n-tuple for the instance, with axis elements instanceCoords[i].

- Let Regions be the set of regions for which associated deltas are provided for the given item, and let R be a region within that set.

- Let startCoords, peakCoords and endCoords be the start, peak and end n-tuples for some specified region, R. Let startCoords[i], etc. be coordinate values for a given axis.

- Let AS be a per-axis scalar, and let S be an overall scalar for a given region.

- Let delta be the delta value in the variation data associated with a given region, and let scaledDelta be the scaled delta for a given region and instance.

- Let netAdjustment be the accumulated adjustment for the given item.

- Let defaultValue be the default value of the item specified in the font, and let interpolatedValue be the interpolated value of the item for a given instance.

The following pseudo-code provides a specification of the interpolation algorithm:

netAdjustment = 0; /* initialize the accumulated adjustment to zero */

(for each R in Regions) /* For each region, calculate a scalar S */

{

S = 1; /* initialize the overall scalar for the region to one */

/* for each axis, calculate a per-axis scalar AS */

(for i = 0; i < axisCount; i++)

{

/* If a region definition is not valid in relation to some axis,

then ignore the axis. For a region to be valid in relation to a

given axis, it must have a peak that is between the start and

end values, and the start and end values cannot have different

signs if the peak is non-zero. (Start and end can have different

signs if the peak is zero, however: this can be used if an axis is

to be ignored in the scalar calculation.) */

if (startCoords[i] > peakCoords[i] || peakCoords[i] > endCoords[i])

AS = 1;

else if (startCoords[i] < 0 && endCoords[i] > 0 && peakCoords[i] != 0)

AS = 1;

/* Note: for remaining cases, start, peak and end will all be <= 0 or

will all be >= 0, or else peak will be == 0. */

/* If the peak is zero for some axis, then ignore the axis. */

else if (peakCoords[i] == 0)

AS = 1;

/* If the instance coordinate is out of range for some axis, then the

region and its associated deltas are not applicable. */

else if (instanceCoords[i] < startCoords[i] 
                 || instanceCoords[i] > endCoords[i])

AS = 0;

/* The region is applicable: calculate a per-axis scalar as a

proportion of the proximity of the instance to the peak within

the region. */

else

{

if (instanceCoords[i] == peakCoords[i])

AS = 1;

else if (instanceCoords[i] < peakCoords[i])

{

AS = (instanceCoords[i] – startCoords[i])

/ (peakCoords[i] – startCoords[i]);

}

else /* instanceCoords[i] > peakCoords[i] */

{

AS = (endCoords[i] – instanceCoords[i])

/ (endCoords[i] – peakCoords[i]);

}

}

/* The overall scalar is the product of all per-axis scalars.

Note: the axis scalar and the overall scalar will always be

>= 0 and <= 1. */

S = S * AS;

} /* per-axis loop */

/* get the scaled delta for this region */

scaledDelta = S * delta;

/* accumulate the adjustments from each region */

netAdjustment = netAdjustment + scaledDelta;

} /* per-region loop */

/* apply the accumulated adjustment to the default to derive the interpolated

value */

interpolatedValue = defaultValue + netAdjustment;

When scaled deltas are applied to a default value, it is possible that the combined result will be outside the range of the data type used in the font for the default value. For example, the default value could be represented in the font as a 16-bit value, but the instance value after applying deltas might require more bits to represent it. An implementation-determined representation may be used during calculation and for the final result (interpolatedValue). The numeric range used in calculation shall be at least that of the data type of the item to which deltas are applied; for example, at least [-32768, 32767] when applying scaled deltas to an FWORD value.

Applying scaled variation deltas to a scalar value requires calculations that involve fractional values.  In calculation of scalars (S, AS) and of interpolated values (scaledDelta, netAjustment, interpolatedValue), at least 16 fractional bits of precision should be maintained. If required for the internal representation, rounding should be done only when the final result is used, and may retain greater fractional bit-depth than that of the data type of the item to which deltas are applied. See also 7.1.4 for related discussion of precision and rounding.

Depending on the internal representation used, there is a possibility that the result of arithmetic operations when applying deltas could exceed the range supported by the internal representation. The order in which deltas are added is not prescribed, but can also be a factor in whether overflow occurs and, if it does, what the final result could be. If resources permit, applications should allow for larger ranges to avoid the possibility of overflow at any point during calculation, and to ensure that the order in which deltas are applied does not affect the final result. Regardless of the type used in the font representation for the default value, at least 32 significant bits should be used for calculations.

If overflow of the internal representation is inevitable, saturation arithmetic (clamping, rather than wrapping) may be used to mitigate error artifacts. In general, however, behavior on overflow is not defined. For this reason, font developers should take note of situations in which a combination of deltas could exceed the range of the data type of the font data to which the deltas are applied, and anticipate that resulting behavior could be inconsistent in different applications. In particular, font developers should not depend on the overflow behavior of particular applications.

### Interpolation example

The following example illustrates the interpolation process for a particular instance. This example is based on glyph 45 of the Skia font (glyph name “hyphen.oldstyle”), which is the glyph for the character U+002D HYPHEN-MINUS.

NOTE	The Skia font is included in Apple's OSX platform. At the time of publication of the ISO/IEC 14496-22 OFF (4th edition) specification, existing versions of the Skia font did not conform to the OFF specification as a whole, but the implementation of variation data in the 'gvar' table, which is what is illustrated here, did conform.

The glyph entry in the ‘glyf’ table has one contour with four points. Based on values for glyph 45 in the ‘hmtx’ table, “phantom” points are inferred in the rasterizer to represent left and right side-bearings. (For this example, horizontal layout is assumed, and so top and bottom phantom points are ignored.) These phantom points are at (0, 0) and (698, 0). Thus, there are six points requiring interpolation.

pt 3: (621, 647)

pt 0: (621, 512)

pt 1: (78, 512)

pt 2: (78, 647)

pt 4: (0, 0)

pt 5: (698, 0)

The Skia font has weight and width axes. The variation data for glyph 45 in the ‘gvar’ table has deltas associated with 8 regions within the weight-width variation space. Three of these will be considered, and will be referred to as R1, R2 and R3. Each of these is a non-intermediate region, and so is defined using a single n-tuple. The n-tuples for each are as follows:

| Region | (weight, width) |
| --- | --- |
| R1 | (1, 0) |
| R2 | (0, 1) |
| R3 | (1, 1) |

The following figures illustrate the range of applicability over the variation space for each of these regions:

R1 has a zero coordinate value for the width axis, which means that changes in width for the variation instance have no effect on the scalar calculations for this region.

R2 has a zero coordinate value for the weight axis, which means that changes in weight for the variation instance have no effect on the scalar calculations for this region.

R3 has non-zero coordinate values for both weight and width axes, which means that changes for the variation instance in either weight or width will affect the scalar calculations for this region.

Now consider the delta values specified in the font for each point in association with these three regions. X and Y deltas are specified for each point.

R1 has the following associated deltas:

|  | pt 0 | pt 1 | pt 2 | pt 3 | pt 4 | pt 5 |
| --- | --- | --- | --- | --- | --- | --- |
| X | 234 | -26 | -26 | 234 | 0 | 209 |
| Y | -135 | -135 | 175 | 175 | 0 | 0 |

Applying these deltas to the original point positions, the maximal effect of deltas associated with R1 would be to modify the outline as follows:

pt 3: (833, 822)

pt 0: (855, 377)

pt 1: (52, 377)

pt 2: (52, 822)

pt 4: (0, 0)

pt 5: (907, 0)

For a variation instance of (1, 0) (heaviest weight, default width), the scalars for other regions would be zero, and so this would be the resulting glyph outline for that instance. Reducing the weight value of the instance would attenuate the extent of the change, with the outline interpolated in between the original outline and this maximal modification of the outline.

Now consider R2: it has the following deltas associated with it:

|  | pt 0 | pt 1 | pt 2 | pt 3 | pt 4 | pt 5 |
| --- | --- | --- | --- | --- | --- | --- |
| X | 165 | 20 | 20 | 165 | 0 | 187 |
| Y | -2 | -2 | 2 | 2 | 0 | 0 |

Applying these deltas to the original point positions, the maximal effect of deltas associated with R2 would be to modify the outline as follows:

pt 3: (786, 649)

pt 0: (786, 510)

pt 1: (98, 510)

pt 2: (98, 649)

pt 4: (0, 0)

pt 5: (885, 0)

For a variation instance of (0, 1) (regular weight, widest width), the scalars for other regions would be zero, and so this would be the resulting, interpolated glyph outline for that instance.

Now consider R3: it has the following associated deltas:

|  | pt 0 | pt 1 | pt 2 | pt 3 | pt 4 | pt 5 |
| --- | --- | --- | --- | --- | --- | --- |
| X | 0 | 0 | 0 | 0 | 0 | 0 |
| Y | 0 | 0 | 0 | 0 | 0 | 0 |

Since all of the delta values are zero, the data associated with this region has no effect at all on the glyph outline. (In fact, this data is superfluous.)

Now, consider a variation instance of (1, 1) (heaviest weight, widest width). All three regions, R1, R2 and R3, are applicable for this instance. As noted, the variation data associated with R3 will have no effect on the glyph. But the data for regions R1 and R2 would also be applicable for this instance, and their maximal effects would be combined. That is, the X and Y deltas for each point from data associated with both R1 and R2 would be applied to the point X and Y coordinates. This would result in the glyph outline being modified as follows:

pt 3: (1020, 822)

pt 0: (1020, 375)

pt 1: (72, 375)

pt 2: (72, 822)

pt 4: (0, 0)

pt 5: (1094, 0)

For other variation instances with weight > 0 and < 1 and with width > 0 and < 1, the data for regions R1 and R2 would both be applied, but scalars for the two regions would vary, resulting in different proportional effects on the outline of the data for each region. For example, consider a variation instance with coordinates (0.2, 0.7) — a slight weight increase and a large width increase. The region scalars for R1 and R2 would be 0.2 and 0.7. Each of these would be applied to the deltas for each region, and the scaled delta values for a given point combined:

|  | pt 0 | pt 1 | pt 2 | pt 3 | pt 4 | pt 5 |
| --- | --- | --- | --- | --- | --- | --- |
| X | 0.2 × 234 + 0.7 × 165 = 162.3 | 0.2 × -26 + 0.7 × 20 = 8.8 | 0.2 × -26 + 0.7 × 20 = 8.8 | 0.2 × 234 + 0.7 × 165 = 162.3 | 0.2 × 0 + 0.7 × 0 = 0 | 0.2 × 209 + 0.7 × 187 = 172.7 |
| Y | 0.2 × -135 + 0.7 × -2 = -28.4 | 0.2 × -135 + 0.7 × -2 = -28.4 | 0.2 × 175 + 0.7 × 2 = 36.4 | 0.2 × 175 + 0.7 × 2 = 36.4 | 0.2 × 0 + 0.7 × 0 = 0 | 0.2 × 0 + 0.7 × = 0 |

This would result in the glyph outline being modified as follows:

pt 3: (783, 683)

pt 0: (783, 484)

pt 1: (87, 484)

pt 2: (87, 683)

pt 4: (0, 0)

pt 5: (871, 0)

### Dynamic generation of static instance fonts

In certain application workflows, it may be necessary to dynamically generate a static font resource for a particular instance – that is, conventional, non-variation font tables that use interpolated values for a particular instance. This may be needed in order to provide font data to legacy software or data formats that do not understand or support variable fonts, such as legacy printer drivers, or PDF or XPS files with embedded font data.

For example, it may be necessary to process ‘glyf’ and ‘gvar’ tables in a variable font to generate a new ‘glyf’ table that has interpolated outline data for a particular instance; or to process ‘hhea’ and ‘MVAR’ tables to generate a new ‘hhea’ table with data for a particular instance.

Different application scenarios may require more- or less-complete font data, entailing different sets of non-variations-specific font tables that need to be generated. No minimal requirements are specified here. The following points should be noted, however:

- Some scenarios may require use of a PostScript name (name ID 6) in instance font data, with distinct names for each instance that is used. An Adobe technical note provides a specification for Postscript name generation that can be used for instance fonts derived from variable fonts. See Adobe Technical Note #5902: “PostScript Name Generation for Variation Fonts” [27].

- For a variable font with CFF2 outlines, some workflows – for example, printing – may require an instance font to be generated with a ‘CFF ’ table. In such cases, if the variable font has more than one Font DICT in the FDArray, then a CID-keyed CFF font should be generated, with an ROS of “Adobe-Identity-0”. If the variable font has one Font DICT in the FDArray, then a name-keyed CFF font can be generated if glyph names are supplied in the ‘post’ table (some legacy workflows look to a glyph name for semantics); otherwise, a CID-keyed CFF can be generated as above. Converting CFF2 CharStrings to Type2 CharStrings would involve re-optimizing the CharString arguments and operators to avoid exceeding the maximum permitted stack depth. Most of the CFF fields removed from the CFF2 specification can be omitted, so that they will inherit the CFF default values. The source information for filling the rest of the fields is documented in "CFF2 changes from CFF 1.0" [28].

- A variable font that has TrueType outlines may utilize the GET VARIATION instruction to provide current variation axis coordinates to the glyph program. In scenarios that require dynamic generation of instance font data, it should be assumed that this instruction will not be supported. In the process for generating an interpolated ‘glyf’ table, special treatment of the GETVARIATION instruction will be needed to ensure that the program gets appropriate axis coordinate values for the given instance. For details, see TrueType Instruction Set [[# Get Variation]].

## Font variations common table formats

### Overview

OFF font variations allow continuous variation along one or more design axes, such as weight or width. An overview of font variations and a specification of the algorithm for interpolating variation instance values is provided in subclause 7.1, which should be read first. This subclause documents the formats for variation data that are used in various font tables, such as the ‘gvar’ or ‘MVAR’ tables.

A font has many different data items found in several different font tables that provide values that are specific to a particular font face. Examples include glyph-specific values, such as the positions of glyph outline points and glyph advance widths, and face-wide values, such as a sub-family name, a weight class, or ascender and descender values. In a variable font, most or all of these values may need to vary for different variation instances. When an application selects a variation instance within the font’s variation space, new values for such items appropriate to that instance need to be derived. This is done using delta adjustment values.

For example, the OS/2 table of a font may provide a default sxHeight value of 970. The ‘MVAR’ table might provide a delta value of +50 that is used for weight-axis values from the default to the heaviest-supported weight. For a particular instance, the interpolation process might scale that delta with a scalar co-efficient of 0.4, deriving an instance sxHeight value of 990.

These concepts and the interpolation algorithm for deriving instance values are described in detail in the Font variations overview.

The variation data for a font consists of a number of delta adjustment values. Each individual delta applies to a particular, target data item; for instance, the X coordinate of a point in a glyph outline , or the font’s sTypoAscender. Each delta is also associated with a specific region within the font’s design variation space over which it is applicable.. Thus, a given delta is logically keyed by the target data item and the applicable region.

A variable font includes many deltas. At the highest level, deltas are organized into collections for different target item sets:

- Deltas for positions of points of a ‘glyf’ table are stored in a ‘gvar’ table.

- Deltas for positions of points of a ‘CFF2’ table are stored within the ‘CFF2’ table.

- Deltas for CVT values in the ‘cvt ‘ table are stored in a ‘cvar’ table.

- Deltas for glyph metrics in an ‘hmtx’ table are stored in an ‘HVAR’ table; and deltas for glyph metrics in a ‘vmtx’ or ‘VORG’ table are stored in a ‘VVAR’ table.

- Deltas for anchor positions in ‘GPOS’ lookups and other items used in ‘GDEF’, ‘GPOS’ or ‘JSTF’ tables are stored within variation data contained in the ‘GDEF’ table.

- Deltas for font-wide metrics and other items from the ‘OS/2’, ‘hhea’, ‘gasp’, ‘post’ or ‘vhea’ tables are stored in an ‘MVAR’ table.

- Deltas for values in other tables are stored in the respective table: deltas for baseline metrics in the ‘BASE’ table and for various items in the ‘COLR’ table are stored in each table.

In a variable font, the largest group of deltas are for the positions of glyph outline points. For TrueType outlines in a ‘glyf’ table, the deltas are stored within the ‘gvar’ table, with a second level of organization grouping deltas by glyph ID. See the [[‘gvar’ table specification]] for details.

Below these higher levels of organization, most variation data is organized in one of two ways. (Variation data for CFF2 outlines is a partial exception – see below.)

- Organize sets of deltas for several target items into groupings by the variation-space region over which they apply. Since regions are defined using n-tuples (or “tuples”), such data sets will be referred to as tuple variation stores.

- Organize sets of deltas associated with different regions into groupings by the target item to which they apply. Such data sets will be referred to as item variation stores.

The two formats have different ways of representing n-tuples that define regions of applicability, and different ways of associating deltas with target font-data items. The tuple variation store format is optimized for compact representation of glyph outline variation data that is all processed for a given variation instance. The ItemVariationStore format, on the other hand, is designed to allow direct access to variation data for arbitrary target items, allowing more efficient processing in contexts that do not require interpolated values for all items to be computed. (Additional details are provided below.) The 'gvar' and 'cvar' tables use the tuple variation store format, while variation data in most other tables, including the 'MVAR', 'HVAR' and 'GDEF' tables, use ItemVariationStore formats.

Variation data for CFF2 outlines are handled slightly differently than other cases. The deltas for glyph outline descriptions are interleaved directly within the outline descriptions in the Compact Font Format 2 (‘CFF2’) table. But the sets of regions that are associated with the delta sets are defined in an item variation store, contained as a subtable within the CFF2 table.

### Tuple variation store

Tuple variation stores are used in the ‘gvar’ and ‘cvar’ tables, and organize sets of variation data into groupings, each of which is associated with a region of applicability within the variation space. Within the ‘gvar’ table, there is a separate variation store for each glyph. Within the ‘cvar’ table, there is one variation store providing variations for all CVT values.

There is a minor difference in the top-level structure of the store in these two contexts. Within the ‘cvar’ table, it is the entire ‘cvar’ table that comprises the specific variation store format, with a header that begins with major/minor version fields. The specific variation store format for glyph-specific data within the ‘gvar’ table is the GlyphVariationData table (one per glyph ID), which does not include any version fields. In other respects, the ‘cvar’ table and GlyphVariationData table formats are the same. There is also a minor difference in certain data that can occur in a GlyphVariationData table versus a ‘cvar’ table. Differences between the ‘gvar’ and ‘cvar’ tables will be summarized later in this subclause.

In terms of logical information content, the GlyphVariationData and ‘cvar’ tables consist of a set of logical, tuple variation data tables, each for a different region of the variation space. In physical layout, however, the logical tuple variation tables are divided into separate parts that get stored separately: a header portion, and a serialized-data portion.

In terms of overall structure, the GlyphVariationData table and the ‘cvar’ table each begin with a header, which is followed by serialized data. The header includes an array with the tuple variation headers. The serialized data include deltas and other data that will be explained below.

glyphVariationData table / ‘cvar’ table

header
(includes tuple variation headers)

Serialized data
(adjustment deltas and other data)

#### Tuple records

The tuple variation store formats make reference regions within the font’s variation space using tuple records. A tuple record identifies a position in terms of normalized coordinates, which use F2DOT14 values.

Tuple record

| Type | Name | Description |
| --- | --- | --- |
| F2DOT14 | coordinates[axisCount] | Coordinate array specifying a position within the font’s variation space. The number of elements must match the axisCount specified in the ‘fvar’ table. |

NOTE	The Tuple record and UserTuple record (used in the 'fvar' table) both describe a position in the variation space but are distinct: UserTuple uses Fixed values to represent user scale coordinates, while Tuple record uses F2DOT14 values to represent normalized coordinates.

#### Tuple variation store header

The two variants of a tuple variation store header, the GlyphVariationData table header and the ‘cvar’ header, are only slightly different. The formats of each are as follows:

GlyphVariationData Header:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | tupleVariationCount | A packed field. The high 4 bits are flags (see below), and the low 12 bits are the number of tuple variation tables for this glyph. The count can be any number between 1 and 4095. |
| Offset16 | dataOffset | Offset from the start of the GlyphVariationData table to the serialized data |
| TupleVariationHeader | tupleVariationHeaders [tupleVariationCount] | Array of tuple variation headers. |

‘cvar’ table header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the ’cvar’ table – set to 1. |
| uint16 | minorVersion | Minor version number of the ’cvar’ table – set to 0. |
| uint16 | tupleVariationCount | A packed field. The high 4 bits are flags (see below), and the low 12 bits are the number of tuple variation tables for this glyph. The count can be any number between 1 and 4095. |
| Offset16 | dataOffset | Offset from the start of the ‘cvar’ table to the serialized data. |
| TupleVariationHeader | tupleVariationHeaders [tupleVariationCount] | Array of tuple variation headers. |

The tupleVariationCount field contains a packed value that includes flags and the number of logical tuple variation tables — which is also the number of physical tuple variation headers. The format of the tupleVariationCount value is as follows:

| Mask | Name | Description |
| --- | --- | --- |
| 0x8000 | SHARED_POINT_NUMBERS | Flag indicating that some or all tuple variation tables reference a shared set of “point” numbers. These shared numbers are represented as packed point number data at the start of the serialized data. |
| 0x7000 | Reserved | Reserved for future use. |
| 0x0FFF | COUNT_MASK | Mask for the low bits to give the number of tuple variation tables. |

If the SHARED_POINT_NUMBERS flag is set, then the serialized data following the header begins with packed “point” number data. In the context of a GlyphVariationData table within the ‘gvar’ table, these identify outline point numbers for which deltas are explicitly provided. In the context of the ‘cvar’ table, these are interpreted as CVT indices rather than point indices. The format of packed point number data is described below.

#### TupleVariationHeader

The GlyphVariationData and ‘cvar’ header formats include an array of tuple variation headers. The TupleVariationHeader format is as follows:

TupleVariationHeader

| Type | Name | Description |
| --- | --- | --- |
| uint16 | variationDataSize | The size in bytes of the serialized data for this tuple variation table. |
| uint16 | tupleIndex | A packed field. The high 4 bits are flags (see below). The low 12 bits are an index into a shared tuple records array. |
| Tuple | peakTuple | Peak tuple record for this tuple variation table — optional, determined by flags in the tupleIndex value. NOTE this must always be included in the ‘cvar’ table. |
| Tuple | intermediateStartTuple | Intermediate start tuple record for this tuple variation table — optional, determined by flags in the tupleIndex value. |
| Tuple | intermediateEndTuple | Intermediate end tuple record for this tuple variation table — optional, determined by flags in the tupleIndex value. |

Note that the size of the TupleVariationHeader is variable, depending on whether peak or intermediate tuple records are included. (See below for more information.)

The variationDataSize value indicates the size of serialized data for the given tuple variation table that is contained in the serialized data. It does not include the size of the TupleVariationHeader.

Every tuple variation table has an associated peak tuple record. Most tuple variation tables use non-intermediate regions, and so require only the peak tuple record to define the region. In the ‘cvar’ table, there is only one variation store, and so any given region will only need to be referenced once. Within the ‘gvar’ table, however, there is a GlyphVariationData table for each glyph ID, and so any region may be referenced numerous times; in fact, most regions will be referenced within the GlyphVariationData tables for most glyphs. To provide a more efficient representation, the tuple variation store formats allow for an array of tuple records, stored outside the tuple variation store structures that can be shared across many tuple variation stores. This is used only within the ‘gvar’ table; it is not needed or supported in the ‘cvar’ table. The formats alternately allow for a peak tuple record that is non-shared, specific to the given tuple variation table, to be embedded directly within a TupleVariationHeader. This is optional within the ‘gvar’ table, but required in the ‘cvar’ table, which does not use shared peak tuple records.

See the glyph variations (‘gvar’) table description for details on the representation of shared tuple records within that table.

The tupleIndex field contains a packed value that includes flags and an index into a shared tuple records array (not used in the ‘cvar’ table). The format of the tupleIndex field is as follows:

tupleIndex format

| Mask | Name | Description |
| --- | --- | --- |
| 0x8000 | EMBEDDED_PEAK_TUPLE | Flag indicating that this tuple variation header includes an embedded peak tuple record, immediately after the tupleIndex field. If set, the low 12 bits of the tupleIndex value are ignored. NOTE this must always be set within the ‘cvar’ table. |
| 0x4000 | INTERMEDIATE_REGION | Flag indicating that this tuple variation table applies to an intermediate region within the variation space. If set, the header includes the two intermediate-region, start and end tuple records, immediately after the peak tuple record (if present). |
| 0x2000 | PRIVATE_POINT_NUMBERS | Flag indicating that the serialized data for this tuple variation table includes packed “point” number data. If set, this tuple variation table uses that number data; if clear, this tuple variation table uses shared number data found at the start of the serialized data for this glyph variation data or ‘cvar’ table. |
| 0x1000 | Reserved | Flag reserved for future use —set to 0. |
| 0x0FFF | TUPLE_INDEX_MASK | Mask for the low 12 bits to give the shared tuple records index. |

Note that the INTERMEDIATE_REGION flag is independent of the EMBEDDED_PEAK_TUPLE flag or the shared tuple records index. Every tuple variation table has a peak n-tuple indicated either by an embedded tuple record (always true in the ‘cvar’ table) or by an index into a shared tuple records array (only in the ‘gvar’ table). An intermediate-region tuple variation table additionally has start and end n-tuples that also get used in the interpolation process; these are always represented using embedded tuple records.

Also note that the PRIVATE_POINT_NUMBERS flag is independent of the SHARED_POINT_NUMBERS flag in the tupleVariationCount field of the GlyphVariationData or ‘cvar’ header. A GlyphVariationData or ‘cvar’ table may have shared point number data used by multiple tuple variation tables, but any given tuple variation table may have private point number data that it uses instead.

As noted, the size of tuple variation headers is variable. The next TupleVariationHeader can be calculated as follows:

const TupleVariationHeader*

NextHeader( const TupleVariationHeader* currentHeader, int axisCount )

{

int bump = 2 * sizeof( uint16 );

int tupleIndex = currentHeader->tupleIndex;

if ( tupleIndex & embeddedPeakTuple )

bump += axisCount * sizeof( F2DOT14 );

if ( tupleIndex & intermediateRegion )

bump += 2 * axisCount * sizeof( F2DOT14 );

return (const TupleVariationHeader*)((char*)currentHeader + bump);

}

#### Serialized data

After the GlyphVariationData or ‘cvar’ header (including the TupleVariationHeader array) is a block of serialized data. The offset to this block of data is provided in the header.

The serialized data block begins with shared “point” number data, followed by the variation data for the tuple variation tables. The shared point number data is optional: it is present if the corresponding flag is set in the tupleVariationCount field of the header. If present, the shared number data is represented as packed point numbers, described below.

Serialized data block

Shared “point” numbers
(optional per flag in the header)

Per-tuple-variation data

The remaining data contains runs of data specific to individual tuple variation tables, in order of the tuple variation headers. Each TupleVariationHeader indicates the data size for the corresponding run of data for that tuple variation table.

The per-tuple-variation-table data optionally begins with private “point” numbers, present if the privatePointNumbers flag is set in the tupleIndex field of the TupleVariationHeader. Private point numbers are represented as packed point numbers, described below.

After the private point number data (if present), the tuple variation data will include packed delta data. The format for packed deltas is given below. Within the ‘gvar’ table, there are packed deltas for X coordinates, followed by packed deltas for Y coordinates.

Private point numbers
(optional per flag in tupleVariationHeader)

X coordinate packed deltas

Y coordinate packed deltas

Per-tuple-variation data — ‘gvar’

Within the ‘cvar’ table, there is one set of packed deltas.

Private point numbers
(optional per flag in tupleVariationHeader)

CVT packed deltas

Per-tuple-variation data — ‘cvar’

The data size indicated in the TupleVariationHeader includes the size of the private point number data, if present, plus the size of the packed deltas.

#### Packed “point” numbers

Tuple variation data specify deltas to be applied to specific items: X and Y coordinates for glyph outline points within the ‘gvar’ table, and CVT values in the ‘cvar’ table. For a given glyph, deltas may be provided for any or all of a glyph’s points, including “phantom” points generated within the rasterizer that represent glyph side bearing points. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) Similarly, within the ‘cvar’ table, deltas may be provided for any or all CVTs. The set of glyph points or CVTs for which deltas are provided is specified by packed point numbers.

NOTE 1	If a glyph is a composite glyph, then “point” numbers are interpreted as indices for the components that make up the composite glyph. See the ‘gvar’ table chapter for complete details. Likewise, in the context of the ‘cvar’ table, “point” numbers are indices for CVT entries.

NOTE 2	Within the ‘gvar’ table, if deltas are not provided explicitly for some points, then inferred delta values may need to be calculated – see the ‘gvar’ table chapter for details. This does not apply to the ‘cvar’ table, however: if deltas are not provided for some CVT values, then no adjustments are made to those CVTs in connection with the particular tuple variation table.

Packed point numbers are stored as a count followed by one or more runs of point number data.

The count may be stored in one or two bytes. After reading the first byte, the need for a second byte can be determined. The count bytes are processed as follows:

- If the first byte is 0, then a second count byte is not used. This value has a special meaning: the tuple variation data provides deltas for all glyph points (including the “phantom” points), or for all CVTs.

- If the first byte is non-zero and the high bit is clear (value is 1 to 127), then a second count byte is not used. The point count is equal to the value of the first byte.

- If the high bit of the first byte is set, then a second byte is used. The count is read from interpreting the two bytes as a big-endian uint16 value, with the high-order bit masked out.

Thus, if the count fits in 7 bits, it is stored in a single byte, with the value 0 having a special interpretation. If the count does not fit in 7 bits, then the count is stored in the first two bytes with the high bit of the first byte set as a flag that is not part of the count – the count uses 15 bits.

For example, a count of 0x00 indicates that deltas are provided for all point numbers / all CVTs, with no additional point number data required; a count of 0x32 indicates that there are a total of 50 point numbers specified; a count of 0x81 0x22 indicates that there are a total of 290 (= 0x0122) point numbers specified.

Point number data runs follow after the count. Each data run begins with a control byte that specifies the number of point numbers defined in the run, and a flag bit indicating the format of the run data. The control byte’s high bit specifies whether the run is represented in 8-bit or 16-bit values. The low 7 bits specify the number of elements in the run minus 1. The format of the control byte is as follows:

| Mask | Name | Description |
| --- | --- | --- |
| 0x80 | POINTS_ARE_WORDS | Flag indicating the data type used for point numbers in this run. If set, the point numbers are stored as unsigned 16-bit values (uint16); if clear, the point numbers are stored as unsigned 8-bit values (uint8). |
| 0x7F | POINT_RUN_COUNT_MASK | Mask for the low 7 bits of the control byte to give the number of point number elements, minus 1. |

For example, a control byte of 0x02 indicates that the run has three elements represented as uint8 values; a control byte of 0xD4 indicates that the run has 0x54 + 1 = 85 elements represented as uint16 values.

In the first point run, the first point number is represented directly (that is, as a difference from zero). Each subsequent point number in that run is stored as the difference between it and the previous point number. In subsequent runs, all elements, including the first, represent a difference from the last point number.

Since the values in the packed data are all unsigned, point numbers will be given in increasing order. Since the packed representation can include zero values, it is possible for a given point number to be repeated in the derived point number list. In that case, there will be multiple delta values in the deltas data associated with that point number. All of these deltas must be applied cumulatively to the given point.

#### Packed deltas

Tuple variation data specify deltas to be applied to glyph point coordinates or to CVT values. As in the case of point number data, deltas are stored in a packed format.

Packed delta data does not include the total number of delta values within the data. Logically, there are deltas for every point number or CVT index specified in the point-number data. Thus, the count of logical deltas is equal to the count of point numbers specified for that tuple variation table. But since the deltas are represented in a packed format, the actual count of stored values is typically less than the logical count. The data is read until the expected logic count of deltas is obtained.

NOTE	In the ‘gvar’ table, there will be two logical deltas for each point number: one that applies to the X coordinate, and one that applies to the Y coordinate. Therefore, the total logical delta count is two times the point number count. The packed deltas are arranged with all of the deltas for X coordinates first, followed by the deltas for Y coordinates.

Packed deltas are stored as a series of runs. Each delta run consists of a control byte followed by the actual delta values of that run. The control byte is a packed value with flags in the high two bits and a count in the low six bits. The flags specify the data size of the delta values in the run. The format of the control byte is as follows:

| Mask | Name | Description |
| --- | --- | --- |
| 0x80 | DELTAS_ARE_ZERO | Flag indicating that this run contains no data (no explicit delta values are stored), and that all of the deltas for this run are zero. |
| 0x40 | DELTAS_ARE_WORDS | Flag indicating the data type for delta values in the run. If set, the run contains 16-bit signed deltas (int16); if clear, the run contains 8-bit signed deltas (int8). |
| 0x3F | DELTA_RUN_COUNT_MASK | Mask for the low 6 bits to provide the number of delta values in the run, minus one. |

For example, a control byte of 0x03 indicates that there are four 8-bit signed delta values following the control byte; a control byte of 0x40 indicates that there is one 16-bit signed delta value following the control byte; a control byte of 0x94 indicates that there is no additional data for this run, and that the run represents a sequence of 0x14 + 1 = 21 deltas equal to zero.

The following is an example of a block of packed delta data:

03 0A 97 00 C6 87 41 10 22 FB 34

This data has three runs: a run of four 8-bit values, a run interpreted as eight zeroes, and a run of two 16-bit values:

Run 1: 03 0A 97 00 C6

Run 2: 87

Run 3: 41 10 22 FB 34

This packed data would represent the following logical sequence of delta values:

10, -105, 0, -58, 0, 0, 0, 0, 0, 0, 0, 0, 4130, -1228

#### Processing tuple variation store data

When a variation instance has been selected, an application needs to process the variation store data to derive interpolated values for that instance – interpolated grid coordinates for outline points, or interpolated CVT values. In the case of the ‘gvar’ table, this will be done glyph-by-glyph as needed. The application can process the TupleVariationHeaders to filter the tuple variation tables that are applicable for the current instance, or to calculate a scalar for each tuple variation table directly. Scalars can then be applied to deltas in each tuple variation table, and the net adjustments applied to the target items.

NOTE 1	In the ‘cvar’ table, there is a logical delta for each CVT index given in the packed point number data. In the ‘gvar’ table, there are two logical deltas for each point number: one for the point’s X coordinate, and one for the X coordinate. The delta data is organized with all of the deltas for X coordinates first, followed by deltas for Y coordinates.

NOTE 2	In the ‘gvar’ table, there are two logical deltas for each point number: one for the point’s X coordinate, and one for the Y coordinate.

For details on determining applicability of a given tuple variation table, and on calculation of scalars and net adjustments to target items, see OFF Font Variations overview.

Because point number and delta data are stored in a packed representation, the data shall be processed from the start in order to determine the presence of any particular point number, or to retrieve the delta for a particular item. For this reason, the format is best suited to processing all of the data in a given tuple variation table at once rather than processing data for individual target items. In the case of glyph outlines, this is reasonable since there is no common application scenario for interpolating an adjusted position of a single outline point.

The “phantom” points, which provide side-bearing and advance width information, are a possible exception to that generalization, however. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) In particular, some text-layout operations require glyph metrics (advance widths or side bearings) without necessarily requiring glyph outline data. Yet the tuple variation store formats used in the ‘gvar’ table require that interpolated outlines be computed in order to obtain the interpolated glyph metrics. The ‘HVAR’ table and ‘VVAR’ table provide an alternative way to represent horizontal and vertical glyph metric variation data, and these use the ItemVariationStore format which is specifically designed to be suitable for processing data for particular target items.

#### Differences between ‘gvar’ and ‘cvar’ tables

The following is a summary of key differences between tuple variation stores in the ‘gvar’ and ‘cvar’ tables.

- The ‘gvar’ table is a parent table for tuple variation stores, and contains one tuple variation store (the glyph variation data table) for each glyph ID. In contrast, the entire ‘cvar’ table is comprised of a single, slightly-extended (with version fields) tuple variation store.

- Because the ‘gvar’ table contains multiple tuple variation stores, sharing of data between tuple variation stores is possible, and is used for shared tuple records. Because the ‘cvar’ table has a single tuple variation store, no possibility of shared data arises.

- The tupleIndex field of TupleVariationHeader structures within a tuple variation store includes a flag that indicates whether the structure instance includes an embedded peak tuple record. In the ‘gvar’ table, this is optional. In the ‘cvar’ table, a peak tuple record is mandatory.

- The serialized data includes packed “point” numbers. In the ‘gvar’ table, these refer to glyph contour point numbers or, in the case of a composite glyph, to component indices. In the context of the ‘cvar’ table, these are indices for CVT entries.

- In the ‘gvar’ table, point numbers cover the points or components defined in a ‘glyf’ entry plus four additional “phantom” points that represent the glyph’s horizontal and vertical advance and side bearings. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) The last four point numbers for any glyph, including composite glyphs, are for the phantom points.

- In the ‘gvar’ table, if deltas are not provided for some points and the point indices are not represented in the point number data, then interpolated deltas for those points will in some cases be inferred. This is not done in the ‘cvar’ table, however.

- In the ‘gvar’ table, the serialized data for a given region has two logical deltas for each point number: one for the X coordinate, and one for the Y coordinate. Hence the total number of deltas is twice the count of control points. In the ‘cvar’ table, however, there is only one delta for each point number.

### Item variation stores

Item variation stores are used for most variation data other than that used for TrueType glyph outlines, including the variation data in ‘MVAR’, ‘HVAR’, ‘VVAR’, ‘BASE’, ‘GDEF’ and ‘COLR’ tables.

NOTE	For CFF2 glyph outlines, delta values are interleaved directly within the glyph outline description in the ‘CFF2’ table. The sets of regions which are associated with the delta sets are defined in an item variation store, contained as a subtable within the CFF2 table. See the ‘CFF2’ table description for additional details.

The item variation store formats organize sets of variation data into groupings by the target items. This makes the formats well-suited to computing interpolated instance values for particular font data items. This is useful for certain text layout operations in which only certain data items are required, such as the advance widths of specific glyphs or anchor positions used in specific ‘GPOS’ lookup tables.

The different tables that use item variation stores have their own top-level formats. Each will include an offset to an ItemVariationStore table, containing the variation data. This chapter describes the shared formats: the ItemVariationStore and its component structures.

#### Associating target items to variation data

Variation data is comprised of delta adjustment values that apply to particular target items. Some mechanism is needed to associate delta values with target items. In the item variation store, a block of delta values has an implicit delta-set index, and separate data outside the item variation store is provided that indicates the delta-set index associated with a given target item. Depending on the parent table in which an item variation store is used, different means are used to provide these associations:

- In the MVAR table, an array of records identifies target data items in various other tables, along with the delta-set index for each respective item.

- In the HVAR and VVAR tables, the target data items are glyph metric arrays in the ‘hmtx’ and ‘vmtx’ tables. Mapping subtables in the HVAR and VVAR tables provide the mapping between the target data items and delta-set indices.

- In the COLR table, a mapping subtable is used, as in the HVAR and VVAR tables. Structures that support variable items provide a starting index into the mapping subtable, and use a slice of consecutive mapping entries.

- For the BASE, GDEF, GPOS, and JSTF tables, a target data item is associated with a delta-set index using a related VariationIndex table (6.2.8) within the same subtable that contains the target item.

In the COLR, HVAR and VVAR tables, DeltaSetIndexMap tables are used to provide the mapping to delta set indices. Two formats for the DeltaSetIndexMap are defined: format 0 uses a 16-bit count field, and so is limited to 65,535 entries; format 1 uses a 32-bit count field.

DeltaSetIndexMap format 0:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | DeltaSetIndexMap format, set to 0. |
| uint8 | entryFormat | A packed field that describes the compressed representation of delta-set indices. See details below. |
| uint16 | mapCount | The number of mapping entries. |
| uint8 | mapData [variable] | The delta-set index mapping data. See details below. |

DeltaSetIndexMap format 1:

| Type | Name | Description |
| --- | --- | --- |
| uint8 | format | DeltaSetIndexMap format, set to 1. |
| uint8 | entryFormat | A packed field that describes the compressed representation of delta-set indices. See details below. |
| uint32 | mapCount | The number of mapping entries. |
| uint8 | mapData [variable] | The delta-set index mapping data. See details below. |

NOTE	In previous versions of this specification, only one format of the DeltaSetIndexMap table was defined, and it began with a single, 16-bit entryFormat field. That single 16-bit field has been subdivided into separate 8-bit format and entryFormat fields. In the prior definition, the high-order byte of the 16-bit field was reserved and set to zero. Thus, format 0 is backward compatible with the definition used in previous versions.

The mapCount field indicates the number of delta-set index mapping entries. The logical entries in the mapData array use a base-zero index. In the context in which a DeltaSetIndexMap table is used, if an index into the mapping array is used that is greater than or equal to mapCount, then the last logical entry of the mapping array is used.

Each mapping entry represents a delta-set outer-level index and inner-level index combination. Logically, each of these indices is a 16-bit, unsigned value. These are represented in a packed format that uses one, two, three or four bytes. The entryFormat field is a packed bitfield that describes the compressed representation used in the mapData field of the given DeltaSetIndexMap table. The format of the entryFormat field is as follows:

entryFormat field masks:

| Mask | Name | Description |
| --- | --- | --- |
| 0x0F | INNER_INDEX_BIT_COUNT_MASK | Mask for the low 4 bits, which give the count of bits minus one that are used in each entry for the inner-level index. |
| 0x30 | MAP_ENTRY_SIZE_MASK | Mask for bits that indicate the size in bytes minus one of each entry. |
| 0xC0 | Reserved | Reserved for future use — set to 0. |

The INNER_INDEX_BIT_COUNT_MASK and MAP_ENTRY_SIZE_MASK values enable the inner index to range in size from 1 to 16 bits, and the outer index to range in size from 0 to 31 bits, with the combined size determined by the MAP_ENTRY_SIZE_MASK value. The size of each mapping entry is

entrySize = ((entryFormat & MAP_ENTRY_SIZE_MASK) >> 4 + 1).

The mapCount value gives the number of logical entries in the map data array; the total size of the map data array, in bytes, is:

entrySize * mapCount

For a given entry, the outer-level and inner-level indices can be obtained as follows:

outerIndex = entry >> ((entryFormat & INNER_INDEX_BIT_COUNT_MASK) + 1)

innerIndex = entry & 
((1 << ((entryFormat & INNER_INDEX_BIT_COUNT_MASK) + 1)) - 1)

For larger sets of variation data, such as may be needed for COLR, HVAR or VVAR tables, optimization of the indices data as well as the delta data may have a significant impact on overall size. Optimizing compilers may need to consider the impact on representation of indices in tandem as it optimizes the item variation store to achieve the best overall results.

#### Variation data

The ItemVariationStore table includes a variation region list, which defines the different regions of the font’s variation space for which variation data is defined. It also includes a set of item variation data subtables, each of which provides a portion of the total variation data. Each subtable is associated with some subset of the defined regions, and will include deltas used for one or more target items. Conceptually, the deltas form a two-dimensional array, with delta-set rows that include a delta for each of the regions referenced by that subtable. From this perspective, the delta-set columns correspond to regions.

The following figure illustrates the overall structure of the ItemVariationStore table.

Region indices array

Deltas

ItemVariationData subtables:

VariationRegionList

Region indices array

Deltas table

ItemVariationStore table

Note that multiple subtables are necessary only if the number of distinct delta-set data exceeds 65,536. Multiple subtables may also be used, however, to provide more compact data representation. There are different ways that the delta data can be made more compact.

First, deltas with a value of zero have no impact on their target items. If there are several delta-set rows that have a zero delta for the same region, then those rows could be moved into a subtable that does not reference that region. As a result, there will be fewer delta values in each row, making the size of data for those rows smaller.

Also, some delta values require 16-bit representations, but some require only 8 bits. For a given subtable, deltas in each row correspond in order to the regions that are referenced, but the ordering of regions has no effect. Hence, regions and corresponding deltas within each row can be re-ordered. Thus, regions that require 16-bit delta representations can be ordered together. The ItemVariationData format specifies a count of regions (columns) for which a 16-bit delta representation is used, with the remaining deltas in each row using 8 bits. By reordering columns, the size required for a given delta-set row can potentially be reduced. If a set of rows have similar requirements in regard to which columns have deltas requiring 16-bit versus 8-bit representations, then those rows can be moved into a subtable with a column order that allows a maximal number of deltas using 8-bit rather than 16-bit representations.

Similarly, ItemVariationData subtables can also combine delta values with 32-bit and 16-bit representations. An optimizing compiler can organize deltas into different subtables using 32-bit and 16-bit representations in some, but 16-bit and 8-bit representations in others, with columns re-ordered in each case to provide the most efficient representation of deltas.

Note that there is minimal overhead for each subtable: 10 bytes (6 bytes in the subtable header and 4 bytes for the offset in the parent table) plus 2 bytes for each region that is referenced.

A complete delta-set index involves an outer-level index into the ItemVariationData subtable array, plus an inner-level index to a delta-set row within that subtable. A special meaning is assigned to a delta-set index 0xFFFF/0xFFFF (that is, outer-level and inner-level portions are both 0xFFFF): this is used to indicate that there is no variation data for a given item. Functionally, this would be equivalent to referencing delta-set data consisting of only deltas of 0 for all regions.

#### Variation regions

As noted above, variation data is comprised of delta adjustment values that have effect over particular regions within the font’s variation space. In a tuple variation store (described earlier in this chapter), the deltas are organized into groupings by region of applicability, with each grouping associated with a particular region. In contrast, the ItemVariationStore format organizes deltas into groupings by the target items to which they apply, with each grouping having deltas for several regions. Accordingly, the item variation store uses different formats for describing the regions in which a set of deltas apply.

For a given item variation store, a set of regions is specified using a VariationRegionList.

VariationRegionList

| Type | Name | Description |
| --- | --- | --- |
| uint16 | axisCount | The number of variation axes for this font. This shall be the same number as axisCount in the ‘fvar’ table. |
| uint16 | regionCount | The number of variation region tables in the variation region list. Shall be less than 32,736. |
| VariationRegion | variationRegions [regionCount] | Array of variation regions. |

The high-order bit of the regionCount field is reserved for future use, and shall be cleared.

The regions can be in any order. Each region is defined using an array of RegionAxisCoordinates records, one for each axis defined in the ‘fvar’ table:

VariationRegion record

| Type | Name | Description |
| --- | --- | --- |
| RegionAxisCoordinates | regionAxes[axisCount] | Array of region axis coordinates records, in the order of axes given in the ‘fvar table. |

Each RegionAxisCoordinates record provides coordinate values for a region along a single axis:

RegionAxisCoordinates Record

| Type | Name | Description |
| --- | --- | --- |
| F2DOT14 | startCoord | The region start coordinate value for the current axis. |
| F2DOT14 | peakCoord | The region peak coordinate value for the current axis. |
| F2DOT14 | endCoord | The region end coordinate value for the current axis. |

The three values shall all be within the range -1.0 to +1.0. startCoord must be less than or equal to peakCoord, and peakCoord must be less than or equal to endCoord. The three values must be either all non-positive or all non-negative with one possible exception: if peakCoord is zero, then startCoord can be negative or 0 while endCoord can be positive or zero.

NOTE	The following guidelines are used for setting the three values in different scenarios:

- In the case of a non-intermediate region for which the given axis should factor into the scalar calculation for the region, either startCoord and peakCoord are set to a negative value (typically, -1.0) and endCoord is set to zero, or startCoord is set to zero and peakCoord and endCoord are set to a positive value (typically +1.0).

- In the case of an intermediate region for which the given axis should factor into the scalar calculation for the region, startCoord, peakCoord and endCoord are all set to non-positive values or are all set to non-negative values.

- If the given axis should not factor into the scalar calculation for a region, then this is achieved by setting peakCoord to zero. In this case, startCoord can be any non-positive value, and endCoord can be any non-negative value. It is recommended either that all three be set to zero, or that startCoord be set to -1.0 and endCoord be set to +1.0.

The full algorithm for interpolation of instance values is given in subclause 7.1 (Font variations overview). The logical algorithm involves computing per-axis scalar values for a given region and a given instance. The per-axis scalars for a region are then combined to yield an overall scalar for the region that is then applied to delta adjustment values. Given a selected variation instance, a per-axis scalar can be calculated for each RegionAxisCoordinates record. The overall scalar for a region can be calculated by combining the per-axis scalars for that region.

#### Item variation store header and item variation data tables

The item variation store table has a header with the following structure.

ItemVariationStore table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Set to 1. |
| Offset32 | variationRegionListOffset | Offset in bytes from the start of the item variation store to the variation region list |
| uint16 | itemVariationDataCount | The number of item variation data subtables. |
| Offset32 | itemVariationDataOffsets [itemVariationDataCount] | Offsets in bytes from the start of the item variation store to each item variation data subtable. |

The item variation store includes an offset to a variation region list and an array of offsets to item variation data subtables. A NULL offset in the array indicates that there is no item variation data subtable for that index into the array.

NOTE	Indices into the itemVariationDataOffsets array are stored in parent tables as delta-set “outer” indices with each such index having a corresponding “inner” index. If the outer index points to a NULL offset, then any inner index will be invalid and can be ignored: items associated with this index do not have any variation.

Each item variation data subtable includes deltas for some number of items, and some subset of regions. The regions are indicated by an array of indices into the variation region list.

ItemVariationData subtable

| Type | Name | Description |
| --- | --- | --- |
| uint16 | itemCount | The number of delta sets for distinct items. |
| uint16 | wordDeltaCount | A packed field: the high bit is a flag—see details below. |
| uint16 | regionIndexCount | The number of variation regions referenced. |
| uint16 | regionIndexes[regionIndexCount] | Array of indices into the variation region list for the regions referenced by this item variation data table. |
| DeltaSet | deltaSets[itemCount] | Delta-set rows. |

The wordDeltaCount field contains a packed value that includes a flag and a “word” delta count. The format of this value is as follows:

| Mask | Name | Description |
| --- | --- | --- |
| 0x8000 | LONG_WORDS | Flag indicating that “word” deltas are long (int32) |
| 0x7FFF | WORD_DELTA_COUNT_MASK | Count of “word” deltas |

The representation of delta values uses a mix of long types (“words”) and short types. If the LONG_WORDS flag is set, deltas are represented using a mix of int32 and int16 values. If the flag is not set, deltas are presented using a mix of int16 and int8 values. See the description of the DeltaSet record below for additional details.

The LONG_WORDS flag should only be used in top-level tables that include 32-bit values that can be variable—currently, only the COLR table.

The count value indicated by WORD_DELTA_COUNT_MASK is a count of the number of deltas that use the long (“word”) representation, and shall be less than or equal to regionIndexCount.

The deltaSets array represents a logical two-dimensional table of delta values with itemCount rows and regionIndexCount columns. Rows in the table provide sets of deltas for particular target items, and columns correspond to regions of the variation space. Each DeltaSet record in the array represents one row of the delta-value table — one delta set.

DeltaSet record:

| Type | Name | Description |
| --- | --- | --- |
| int16 and int8 or int32 and int16 | deltaData​[regionIndexCount] | Variation delta values. |

Logically, each DeltaSet record has regionIndexCount number of elements. The elements are represented using long and short types, as described above. These are either int16 and int8, or int32 and int16, according to whether the LONG_WORDS flag is set. The delta array has a sequence of deltas using the long type followed by a sequence of deltas using the short type. The count of deltas using the long type is derived using WORD_DELTA_COUNT_MASK. The remaining elements use the short type. The length of the data for each row, in bytes, is regionIndexCount + (wordDeltaCount & WORD_DELTA_COUNT_MASK) if the LONG_WORDS flag is not set, or 2 x that amount if the flag is set.

NOTE	Delta values are each represented directly. They are not packed as in the tuple variation store.

In general, variation deltas are, logically, signed 16-bit integers, and in most cases, they are applied to signed 16-bit values (FWORDs) or unsigned 16-bit values (UFWORDs). In the COLR table, however, scaled deltas can be applied to F2DOT14 or Fixed items, which are fixed-size floating types.

When applying scaled deltas to an F2DOT14 value, the F2DOT14 value is treated like a 16-bit integer. (In this sense, the delta and the F2DOT14 value can be viewed as integer values in units of 1/16384ths.) In some contexts in which the F2DOT14 is used, values may be constrained to a certain range, such as [0, 1]. If the context constrains the valid range for values, then any variations by applying deltas are clamped to that range.

Fixed is a 32-bit (16.16) type and, in the general case, requires 32-bit deltas. As described above, the DeltaSet record can accommodate deltas that are, logically, either 16-bit or 32-bit. When scaled deltas are applied to Fixed values, the Fixed value is treated like a 32-bit integer. (In this sense, the delta and the Fixed value can be viewed as integer values in units of 1/65536ths.)

#### Processing item variation store data

When a particular variation instance has been selected, an application needs to process the variation store data associated with particular target items to derive interpolated values for those items and that instance.

To compute the interpolated instance value for a given target item, the application first obtains the delta-set index for that item. It uses the outer-level index portion to select an item variation data subtable within the item variation store, and the inner-level index portion to select a delta-set row within that subtable. The delta set contains one delta for each region referenced by the subtable, in order of the region indices given in the regionIndices array. The application uses the regionIndices array for that subtable to identify applicable regions and to compute a scalar for each of these regions based on the selected instance. Each of the scalars is then applied to the corresponding delta within the delta set to derive a scaled adjustment. The scaled adjustments for the row are then combined to obtain the overall adjustment for the item.

Complete details on the interpolation algorithm logic are provided in subclause 7.1.

For a given variation instance, an application will often need to interpolate values for several items that may use deltas in different item variation data subtables. The subtables will reference region definitions in the shared variation region list. When the instance has been selected, applications can pre-compute and cache a scalar for that instance for each region in the region list. Then, when processing different target items, the cached scalar array can be used without needing to re-compute region scalars for each target item.

### Design-variation axis tag registry

This registry defines design-variation axis tags for use in OpenType fonts. By providing registered tags with well-defined semantics and associated numeric scales for variation, this provides some degree of interoperability between different fonts from different vendors, or between fonts and applications.

Design-variation axis tags are used within the 'fvar' table in variable fonts, and also within the 'STAT' table. Note that they are relevant for non-variable fonts as well as variable fonts: even though a font might not be a variable font, it is still a design variant within its font family. The 'STAT'' table allows applications to understand relationships of font design variants within a given family, whether they are implemented as non-variable fonts or as variation instances of a variable font.

Syntactic requirements for design-variation axis tags

Design-variation axis tags are four-byte arrays that can equivalently be interpreted as a string of four ASCII characters. (See 4.3 for a general description of Tag values.) Axis tags must begin with a letter (0x41 to 0x5A, 0x61 to 0x7A) and must use only letters, digits (0x30 to 0x39) or space (0x20). Space characters must only occur as trailing characters in tags that have fewer than four letters or digits.

Fonts may use tags defined in this registry, or may use foundry-defined tags. (Foundry-defined tags can also be referred to as “custom” or “private” tags.) Foundry-defined tags must begin with an uppercase letter (0x41 to 0x5A), and must use only uppercase letters or digits. Registered axis tags must not use that pattern, but can use any other valid pattern. This ensures that foundry-defined tags and registered tags are never conflicting.

Documentation of registered axis tags

For every axis tag defined in this registry, certain information is required or recommended:

- The tag specification must include a US English name for the axis that may be used as a display string in application user interfaces to refer to the axis, or as the basis of localized display strings.

- The tag specification must include a description of the intended meaning and design-variation behavior for the axis.

- The tag specification must include information regarding the numeric scale used for the axis. This must include a specification of the range of values that are valid for the axis. Depending on the nature of the axis, this may or may not be a bounded range. It must also provide information regarding the semantic interpretation for values, specifying either some objective measure or some convention by which values can be interpreted.

- Whenever appropriate, the tag specification should also indicate a numeric value that is recommended or required (by definition of the scale) for that axis in a “Regular” font.

NOTE 	The “Regular” font in a font family often has a sub-family name that omits axis qualifiers. For example, one foundry may create an optical-size font suited to 12 point size with “Text” included in the name as an indicator of the intended optical size, but another foundry may create a similar font without any indicator for optical size in the name. The choice for a recommended “Regular” axis value should be made with this in mind.

Additional information may also be provided, such as suggestions for programmatic selection of axis values that may be useful in applications.

The specification for the semantic interpretation of numeric values is required as a means to provide some degree of interoperability between different fonts, between fonts and software implementations, and between OpenType Font Variations and other specifications, such as font-weight values in CSS.

Note that interoperability is assumed to be attained in varying degrees, depending on the nature of an axis and the scale that it uses. For example, the scale for the Weight axis provides a limited degree of interoperability. Two different fonts with a Weight axis value of 700 (or “Bold”) may not result in the same amount of darkness or “color” when applied to the same text; but in both cases, a user can expect these to be darker than the “Regular” or “Semibold” fonts from each respective font family, and application developers can produce results that will be predictable for users if they associate that axis value with a particular state of a user-interface control or with a <strong> markup tag.

In contrast to this, the scale for the Optical size axis is designed to provide a much stronger degree of interoperability. For instance, two different fonts with an optical-size value of 20 are assumed to be best suited to text set at 20 points, because the scale is designed that way. If this axis had been defined with a different numeric scale, then an application might not be able to assume that two fonts with the same optical-size value are equally-well suited for a given context.

Not all axes will be equally amenable to a precise or objective measure. For example, there is no objective scale for an amount of italicness. But an Italic axis can be defined with a range from 0.0 to 1.0, representing whatever the font developer considers to be a non-italic design and a fully-italic design, and that is sufficient for applications to associate those numeric variation values with off/on states of an Italic toggle in a user interface to provide a meaningful and familiar experience. It also provides a useful basis of comparison between different fonts, which may be important, for instance, in font-fallback implementations: if the requested font face had an Italic axis setting of 1 but a font-fallback font must be used when displaying text, the application is able to select an appropriate Italic axis setting in the fallback font.

If an axis is intended to interact with programmatic mechanisms that automatically select axis values to provide some effect, then a more precise definition of the numeric scale and its interpretation may be needed. It must be clear to application and platform developers what independent variables should contribute as inputs for selection of axis values, and how the numeric values of the axis scale can be derived from those inputs.

For variable font implementations that support a given axis, the “Regular” value will often be a good choice for the default value of that axis in the 'fvar' variation axis record. The default values set in the 'fvar' table, however, are implementation-specific, and fonts are not required to use this “Regular” value as the axis default value.

How to register a design-variation axis tag

While font developers can always use foundry-defined axis tags however they might choose, we encourage font developers to use registered axis tags when implementing designs for which the registered axis is applicable. We welcome submissions for new design-variation axis tag registration.

Registration of an axis tag can be useful for two key purposes. One is to foster conventionality and familiarity for a kind of design variation. For example, by defining the 'opsz' tag for optical-size variation, to use for tailoring glyph outlines according to the font size, different vendors can incorporate this kind of design variation into fonts, and the variations in these various fonts can be presented to font users as the same kind of variation. The more fonts that are implemented using an 'opsz' axis, the more familiar designers and content authors will become with this kind of design variation. They will benefit from greater consistency in experience when fonts use the same concepts than if different fonts were using similar but different concepts.

Another key purpose for registration of axis tags is to facilitate interoperability between different fonts or between fonts and applications. For example, by specifying a numeric scale for the 'opsz' axis that corresponds to text size in points, this makes it possible for applications to implement mechanisms for automatic selection of optical-size variation that can work with any fonts that support variation in the 'opsz' axis.

The merits for adding a variation axis tag to the registry is primarily determined in relation to these two key purposes: What is the likelihood that an axis of design variation will be implemented in fonts from multiple vendors and found to be useful to designers; and what is the likelihood that applications will implement mechanisms that make use of an interoperable understanding of the axis.

To qualify for registration, a complete description of the axis must be provided, including each of the categories of information listed above. If the axis is intended to interact with mechanisms that select axis values programmatically, then the description must include a clear specification for the numeric scale. There must be reasonable indication of alignment with one or both of the two key purposes for registration described above, and a reasonable indication that the axis will be implemented in fonts from multiple vendors and supported in software platforms and applications. It is recommended that the party proposing the new registration seek input from and get consensus among multiple font and software vendors regarding the definition of the proposed axis and its merits.

#### Registered axis tags list

The following design-variation axes and tags have been registered; the following subclauses provide the axis tag descriptions. These are listed in alphabetic order of the tags.

| Axis Tag | Name |
| --- | --- |
| 'ital' | Italic |
| 'opsz' | Optical Size |
| 'slnt' | Slant |
| 'wdth' | Width |
| 'wght' | Weight |

#### Registered axis tags definitions

Tag: 'ital'

Name: Italic

Description: Used to vary between non-italic and italic.

Valid numeric range: Values must be in the range 0 to 1.

Scale interpretation: A value of 0 can be interpreted as “Roman” (non-italic); a value of 1 can be interpreted as (fully) italic.

Recommended or required “Regular” value: 0 is required.

Additional information:

The Italic axis has long been treated as a kind of design variation within a font family. The 'ital' axis tag is used within a 'STAT' table of italic fonts to provide a complete characterization of a font in relation to its family within the 'STAT' table. The Italic axis can be used as a variation axis within a variable font, though this is not expected to be common.

The Italic axis is distinct from the Slant axis ('slnt'). Fonts may use one or the other, depending on the nature of the design, but should rarely use both. While an italic design often incorporates some slant in the design, use of the Italic axis does not require use of the Slant axis. An italic font should not be characterized in the 'STAT' table as being italic and also having some slant, unless the font family includes multiple italic designs with different amounts of slant.

Tag: 'opsz'

Name: Optical Size

Description: Used to vary design to suit different text sizes.

Valid numeric range: Values shall be strictly greater than zero.

Scale interpretation: Values can be interpreted as text size, in typographic points, as defined in this specification: a physical unit equal to 1/72 of a standard physical inch.

Recommended or required “Regular” value: A value in the range 10 to 16 is recommended for typical text settings.

Suggested programmatic interactions: Applications may choose to select an optical-size variant automatically based on the displayed text size.

Additional information:

The Optical size axis can be used as a variation axis within a variable font. It can also be used within a 'STAT' table in non-variable fonts within a family that has optical-size variants to provide a complete characterization of a font in relation to its family within the 'STAT' table. In the 'STAT' table of a non-variable font, a format 2 axis value table is recommended to characterize the range of text sizes for which the optical-size variant is intended.

Typical font implementation of optical size design variants involves adapting glyph proportions, stem weights, and details to be appropriate to specific sizes of displayed text. These adaptations may be both functional and aesthetic, ensuring legibility at smaller sizes and refinement of fine details and overall width at larger sizes. The nature of the adaptations may depend on aspects of the individual typeface design, the characters or scripts involved, targeted devices on which the font may be displayed, and known or intended distance from which text will be viewed.

Type designers may develop size-specific design variations based on print or screen rendering, typically evaluating the design of these variations at a reading distance of 14 to 16 inches. This can be used as a basis for determining appropriate optical size variants for different distances. For example, if the design of a given optical size variant is appropriate for 12-point text viewed from a reading distance of 15 inches, it should also be appropriate for 24-point text viewed from a reading distance of 30 inches.

The scale for the Optical size axis is text size in points. For these purposes, the text size is as determined by the document or application for its intended use; the actual physical size on a display may be different due to platform or application scaling methods or intended viewing distance.

The target of size-specific design is optical; i.e., tailored to what the reader is seeing. For this reason, Optical size axis variant selection should be determined, so far as possible, by as much information as available regarding displayed size of text as seen by the reader. Factors to be taken into account should include the scaling of type on specific platforms and the translation of document and platform units to 1/72 of a physical inch, as well as typical reading distances for applications and devices. This may mean that a nominally specified text size in a document could result in a different Optical size axis variant selection on different platforms and devices, determined by the actual size of text seen by the reader.

NOTE	User-perceived size is very dependent on viewing distance, not only physical size on the display. For example, when viewing text on a large-screen TV, the physical size on the display will be many times larger than the size perceived by the user. In this case, selecting an Optical size variant based only on the physical size on the display would likely result in reduced legibility for the user.

When translating between document units and the point (1/72 inch) scale for this axis, care should be taken not to assume equivalences between units that may apply on some platforms but not others.

In applications that automatically select an Optical size variant, this should normally be done based on the text size with a default or “100%” zoom level, not on a combination of text size and zoom level. Types of zoom that do not trigger re-layout of text should not change Optical size variant selection, while content enlarging or diminishing operations that change re-layout of text should make a new Optical size variant selection based on the new displayed sized.

If the size of displayed text is smaller or greater than the minimum and maximum extent of the axis range, Optical size axis variant selection should be clamped to the appropriate minimum or maximum axis value, not reset to the default instance.

It is recommended that software provide means to override automatic Optical size variant selection, as may be appropriate for particular platforms, intended use, known viewing distance, or accessibility.

Tag: 'slnt'

Name: Slant

Description: Used to vary between upright and slanted text.

Valid numeric range: Values must be greater than -90 and less than +90.

Scale interpretation: Values can be interpreted as the angle, in counter-clockwise degrees, of oblique slant from whatever the designer considers to be upright for that font design.

Recommended or required “Regular” value: 0 is required.

Additional information:

The Slant axis can be used as a variation axis within a variable font. It can also be used within a 'STAT' table in non-variable, oblique fonts to provide a complete characterization of a font in relation to its family within the 'STAT' table.

The Slant axis is distinct from the Italic axis ('ital'). Fonts may use one or the other, depending on the nature of the design, but should rarely use both. While an italic design often incorporates some slant in the design, use of the Italic axis does not require use of the Slant axis. An italic font should not be characterized in the 'STAT' table as being italic and also having some slant, unless the font family includes multiple italic designs with different amounts of slant.

Note that the scale for the Slant axis is interpreted as the angle of slant in counter-clockwise degrees from upright. This means that a typical, right-leaning oblique design will have a negative slant value. This matches the scale used for the italicAngle field in the 'post' table.

In a variable font that implements 'slnt' variations, the value in the italicAngle field of the 'post' table must match the default 'slnt' value specified in the 'fvar' table. For non-default instances of a variable font, the 'slnt' axis value can be used as the post.italicAngle value for the instance.

Tag: 'wdth'

Name: Width

Description: Used to vary width of text from narrower to wider.

Valid numeric range: Values must be strictly greater than zero.

Scale interpretation: Values can be interpreted as a percentage of whatever the font designer considers “normal width” for that font design.

Recommended or required “Regular” value: 100 is required.

Suggested programmatic interactions: Applications may choose to select a width variant in a variable font automatically in order to fit a span of text into a target width.

Additional information:

The Width axis has long been used in conjunction with face names such as “Condensed” or “Extended”. Change in glyph width is typically the primary aspect of the design that varies, though other secondary details such as stroke thickness may also be encompassed in this variation.

The Width axis can be used as a variation axis within a variable font. It can also be used within a 'STAT' table in non-variable fonts within a family that has width variants to provide a complete characterization of a font in relation to its family within the 'STAT' table.

The Width axis uses a scale that correlates with but is different from the scale used for the usWidthClass field of the 'OS/2' table. The description of usWeightClass in the 'OS/2' table documentation provides a table of mappings from usWidthClass values to “% of normal” values. Because usWidthClass is limited to nine integer values, it has much less granularity than the Width axis.

When mapping from 'wdth' values to usWidthClass, interpolate fractional values between the mapped values and then round, and clamp to the range 1 to 9.

In a variable font that implements 'wdth' variations, the value in the usWidthClass field of the 'OS/2' table must correspond to the default 'wdth' value specified in the 'fvar' table. For non-default instances of a variable font, the 'wdth' axis value can be used to derive the OS/2.usWidthClass value for that instance.

Percentage of normal width is a comparative scale that will depend on the specific items being compared. The width of a line of text very much depends on the content of the text. No specific reference string is specified here as the basis for comparisons; a font designer can choose what they consider to be representative strings assigning a 'wdth' value to a design variant. Ideally, the 'wdth' value should provide a good estimate for most strings in the target languages of how the width of the string formatted with that 'wdth' variant compares to the width of the same string when formatted with the “normal” variant.

When using a variable font, applications may choose to make small, automated 'wdth' adjustments in order to fit a span of text to some target size. This might be done, for instance, to fit headings within a column, or to improve paragraph layout. The relative change in 'wdth' value (ratio of the original 'wdth' to the adjusted 'wdth') may be used as a first approximation of the adjustment needed. Since the relative change in width may depend on the actual text content, however, this may not provide the exact adjustment needed to obtain the desired width adjustment. The application will likely need to refine the adjustment over multiple attempts.

Tag: 'wght'

Name: Weight

Description: Used to vary stroke thicknesses or other design details to give variation from lighter to blacker.

Valid numeric range: Values must be in the range 1 to 1000.

Scale interpretation: Values can be interpreted in direct comparison to values for usWeightClass in the 'OS/2' table, or the CSS font-weight property.

Recommended or required “Regular” value: 400 is required.

Additional information:

The Weight axis has long been used in conjunction with face names such as “Regular”, “Light” or “Bold”. Change in stroke thickness is typically the primary aspect of the design that varies, though other secondary details such as glyph width or thick-thin contrast may also be encompassed in this variation.

The Weight axis can be used as a variation axis within a variable font. It can also be used within a 'STAT' table in non-variable fonts within a family that has weight variants to provide a complete characterization of a font in relation to its family within the 'STAT' table.

In a variable font that implements 'wght' variations, the value in the usWeightClass field of the 'OS/2' table must match the default 'wght' value specified in the 'fvar' table. For non-default instances of a variable font, the 'wght' axis value can be used as the OS/2.usWeightClass value for the instance.

## Font variations tables

For an overview of OFF font variations and the specification of the interpolation algorithm used for variations, see OFF "Font variations overview". For details regarding which tables are required or optional in variable fonts, see "Variation data tables and miscellaneous requirements".

For information on common table formats used for variations, see OFF "Font variations common table formats".

Note that some variation-related formats may be used in tables other than the variations-specific tables listed above. In particular, the 'GDEF' or 'BASE' tables in a variable font can include variation data using common table formats. A 'CFF2' table in a variable font can also include variation data, though using formats that are specific to the 'CFF2' table.

### avar—Axis variations table

The axis variations table ('avar') is an optional table used in variable fonts. It can be used to modify aspects of how a design varies for different instances along a particular design-variation axis. Specifically, it allows modification of the coordinate normalization that is used when processing variation data for a particular variation instance.

The 'avar' table is used in combination with a font variations ('fvar') table and other required or optional tables used in variable fonts.

#### Overview

The ‘fvar’ table defines a font’s axes and the numeric range for each axis supported by the font. The numeric range for each axis is defined in terms of a “user” scale appropriate to each axis. When processing a font’s variation data to derive glyph outlines or other values for an instance, the instance coordinate for each axis must be mapped from the user scale to a normalized scale. A default normalization is defined that maps the minimum, default and maximum user values in the ‘fvar’ table to -1, 0, and 1, respectively. (See 7.1.4 for a detailed specification.)if (userValue < axisDefaultValue)
{
  defaultNormalizedValue = -(axisDefault - userValue) / (axisDefault – axisMin);
}
else if (userValue > axisDefaultValue)
{
  defaultNormalizedValue = (userValue – axisDefault) / (axisMax – axisDefault);
}
else
{
  defaultNormalizedValue = 0;
}

If an 'avar' table is present, then the output of the default normalization can be further modified by allowing mappings to be defined for additional positions along each scale, creating multiple segments, with other values within each segment interpolated. The following figure illustrates an example of such a modification —the blue line shows the default mapping, with two segments, and the orange line shows the modified mapping, with additional segments.

Figure 7.3: Example of 'avar'-modified normalization – horizontal axis is user scale, vertical axis is normalized scale

The conceptual effect of these additional scale mappings is to make the variation along an axis less linear. Values change linearly within each segment, but additional segments make the way that values change across the entire axis range less linear overall. The effect might also be described as compressing some portions of the scale while making other portions less compressed.

The visual effects of additional axis-value mappings in the 'avar' table are seen in how glyph outlines or other visual elements change as the user-scale values for an axis change. The same amount of change in the user-scale value may correspond to a subtle visual change in one portion of the axis range, but more dramatic changes in another portion of the axis range.

Note that it may be possible to achieve the same or similar effects by adding variation data for additional regions of the variation space. That approach requires more work and more font data, however, and tedious design iteration may be needed to obtain the desired results. The 'avar' table may provide a simple and light-weight way to achieve a particular effect.

Note also that the variation data created by the font designer will also have a significant effect on whether user-scale values and visual elements change uniformly. When an 'avar' table is used, the 'avar' table and the variation data are both factors in the variation behavior that the user will see.

#### Table formats

The 'avar' table is comprised of a small header plus segment maps for each axis.

Axis variation table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the axis variations table – set to 1. |
| uint16 | minorVersion | Minor version number of the axis variations table – set to 0. |
| uint16 | <reserved> | Permanently reserved, set to 0. |
| uint16 | axisCount | The number of variation axes for this font. This must be the same number as axisCount in the 'fvar' table. |
| SegmentMaps | axisSegmentMaps[axisCount] | The segment maps array – one segment map for each axis, in the order of axes specified in the ‘fvar’ table. |

There must be one segment map for each axis defined in the 'fvar' table, and the segment maps for the different axes shall be given in the order of axes specified in the 'fvar' table. The segment map for each axis is comprised of a list of axis-value mapping records.

SegmentMaps record

| Type | Name | Description |
| --- | --- | --- |
| uint16 | positionMapCount | The number of correspondence pairs for this axis. |
| AxisValueMap | axisValueMaps[positionMapCount] | The array of axis value map records for this axis. |

Each axis value map record provides a single axis-value mapping correspondence.

AxisValueMap record

| Type | Name | Description |
| --- | --- | --- |
| F2DOT14 | fromCoordinate | A normalized coordinate value obtained using default normalization. |
| F2DOT14 | toCoordinate | The modified, normalized coordinate value. |

Axis value maps can be provided for any axis, but are required only if the normalization mapping for an axis is being modified. If the segment map for a given axis has any value maps, then it must include at least three value maps: -1 to -1, 0 to 0, and 1 to 1. These value mappings are essential to the design of the variation mechanisms and are required even if no additional maps are specified for a given axis. If any of these is missing, then no modification to axis coordinate values will be made for that axis.

All of the axis value map records for a given axis shall have different fromCoordinate values, and axis value map records shall be arranged in increasing order of the fromCoordinate value. If the fromCoordinate value of a record is less than or equal to the fromCoordinate value of a previous record in the array, then the given record may be ignored.

Also, for any given record except the first, the toCoordinate value must be greater than or equal to the toCoordinate value of the preceding record. This requirement ensures that there are no retrograde behaviors as the user-scale value range is traversed. If a toCoordinate value of a record is less than that of the previous record, then the given record may be ignored.

#### Processing

The complete axis coordinate normalization process, with or without an 'avar' table is described in 7.1.4. The process begins with a default normalization computation. Then if an 'avar' table is present, the normalized value is modified using data in this table.

Each pair of consecutive AxisValueMap records for a given axis defines a segment within the range for that axis, and a mapping from default normalized values for start and end points of the given segment to the final normalized values. Within a segment, intermediate values are interpolated linearly. For complete details of the normalization process, as well as an example using 'avar' data, see 7.1.4.

### cvar—CVT variations table

The control value table (CVT) variations table is used in variable fonts to provide variation data for CVT values.

Fonts that use the TrueType outline format for glyphs and that have hinting instructions will typically also have a CVT (the ‘cvt ’ table). The CVT table provides an indexed list of control values that can be referenced by instructions. Example values are the height of a serif, x-height, or the width of upper case stems. When glyph outline points are adjusted by instructions to improve rasterization at a particular PPEM size, the control values may be used by the instructions to provide design-distance values for those adjustments – typically, values that need to be kept constant across all glyphs in the font for a given PPEM size.

Within a variable font, the numeric value of particular control values may need to be adjusted for different variation instances, to match the changes to outlines for different instances. The CVT variations table provides variation data for that purpose. By using interpolation to derive adjusted CVT values for a particular instance, instructions can obtain instance-appropriate values, and the same instructions can be used for all variations.

The CVT variations table must be used in combination with TrueType outlines and a 'cvt ' table, and also in combination with a font variations ('fvar') table and other required or optional tables used in variable fonts.

#### Table format

The 'cvar' table uses a variant of the tuple variation store format, described in 7.2.2. The 'gvar' table uses a very-slightly different variant of this format for each glyph.

In terms of overall structure, the ‘cvar’ table begins with a header, which is followed by serialized variation data.

Figure 7.4: High-level organization of the 'cvar' table

The variation data includes logical groupings of data that apply to different regions of the variation space – tuple-variation data tables. These logical groupings are stored in two parts: a header, and serialized data. The 'cvar' header includes an array of tuple-variation data headers, each of which is associated with a particular portion of the serialized data.

The serialized data includes adjustment delta values and also packed “point number” data that identify the CVT entries to which the deltas apply. The serialized data for a given region can include point number data that applies to that specific tuple-variation data, but there can also be a shared set of point number data, stored at the start of the serialized data, that can be used in relation to multiple tuple-variation data tables.

The format of the ‘cvar’ header is as follows.

‘cvar’ table header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the CVT variations table — set to 1. |
| uint16 | minorVersion | Minor version number of the CVT variations table — set to 0. |
| uint16 | tupleVariationCount | A packed field. The high 4 bits are flags, and the low 12 bits are the number of tuple-variation data tables for this glyph. The count can be any number between 1 and 4095. |
| Offset16 | dataOffset | Offset from the start of the 'cvar' table to the serialized data. |
| TupleVariationHeader | tupleVariationHeaders [tupleVariationCount] | Array of tuple variation headers. |

Complete details regarding the tupleVariationCount field, the flags used in the tupleVariationCount, the TupleVariationHeader format and the format of the serialized data are provided in the OFF "Font variations common table formats". Details regarding how the data are processed to derive interpolated CVT values for particular instances are provided in that subclause and in the OFF "Font variations overview".

As noted above, the format of the 'cvar' table is closely related to formats used in the 'gvar' table. The following are key differences to note:

- The TupleVariationHeader structure includes a particular field, tupleIndex. This is a packed field that includes flag bits, one of which indicates whether the structure includes an embedded peak tuple record. In the 'gvar' table, this is optional, and the flag does not always need to be set. In the 'cvar' table, however, the embedded peak tuple record is mandatory, and this flag shall always be set.

- The serialized data includes packed “point” numbers. In the context of the 'gvar' table, these are indices for the outline points of a particular glyph. In the 'cvar' table, these are indices for CVT values within the 'cvt ' table.

- In the 'gvar' table, there are two logical deltas for each enumerated outline point: one for the X coordinate, and one for the Y coordinate. Hence, the total number of logical deltas is twice the count of point numbers. In the 'cvar' table, there is exactly one logical delta for each point number.

Note that the CVT values are all FWORDs, and that the total number of CVT values is determined by the length of the 'cvt ' table. Hence, CVT index values range from 0 to floor(cvtLength / sizeof(FWORD)) – 1.

### fvar—Font variations table

Font Variations allow a font designer to incorporate multiple faces within a font family into a single font resource. Variable fonts can provide great flexibility for content authors and designers while also allowing the font data to be represented in an efficient format.

A variable font allows for continuous variation along some given design axis, such as weight:

Figure 7.5: Continuous variation along a design axis

Conceptually, variable fonts define one or more axes over which design characteristics can vary. Weight is one possible axis of variation, but many different kinds of variation are possible. Variable fonts are not limited to a single axis of variation, but can combine two or more different axes of variation. For example, the following illustrates a combination of weight and width variation:

Figure 7.6: Continuous variation along multiple design axes

The set of axes supported by a font define a variation space for that font, supporting a potentially-vast number of design-variation instances at positions across that space.

The font variations table ('fvar') provides the global definition of variations supported within the font. It specifies the axes of variation that are used and the ranges of variation for each axis. It also allows the font designer to specify certain coordinate positions within the font's variation space as named instances. Named instances have designer-provided names, effectively equivalent to sub-family names, that applications can use as a short list of “pre-chosen” design variants they can offer to users.

All variable fonts shall include a font variations table, as well as other required or optional tables used in variable fonts.

Note that some of the information in the font variations table also needs to be reflected in the style attributes ('STAT') table, which is also required in all variable fonts. In particular, each axis and each named instance specified in the font variations table shall have matching axis records and axis value tables in the style attributes table.

#### Table formats

The font variations table consists of a table header, followed by an array of variation axis records, followed by an array of named-instance records:

Figure 7.7: High-level organization of the font variations table

'fvar' Header

The format of the font variations table header is as follows.

Font variations header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the font variations table – set to 1. |
| uint16 | minorVersion | Minor version number of the font variations table – set to 0. |
| Offset16 | axesArrayOffset | Offset in bytes from the beginning of the table to the start of the VariationAxisRecord array. |
| uint16 | (reserved) | This field is permanently reserved. Set to 2. |
| uint16 | axisCount | The number of variation axes in the font (the number of records in the axes array). |
| uint16 | axisSize | The size in bytes of each VariationAxisRecord – set to 20 (0x0014) for this version. |
| uint16 | instanceCount | The number of named instances defined in the font (the number of records in the instances array). |
| uint16 | instanceSize | The size in bytes of each InstanceRecord – set to either axisCount * sizeof(Fixed) + 4, or axisCount * sizeof(Fixed) + 6. |

The header is followed by axes and instances arrays. The location of the axes array is specified in the axisArrayOffset field; the instances array directly follows the axes array.

| Type | Name | Description |
| --- | --- | --- |
| VariationAxisRecord | axes[axisCount] | The variation axis array. |
| InstanceRecord | instances[instanceCount] | The named instance array. |

NOTE	The axisSize and instanceSize fields indicate the size of the VariationAxisRecord and InstanceRecord structures. In this version of the 'fvar' table, the InstanceRecord structure has an optional field, and so two different size formulations are possible. Future minor-version updates of the 'fvar' table may define compatible extensions to either formats. Implementations must use the axisSize and instanceSize fields to determine the start of each record.

The set of axes that make up the font's variation space are defined by an array of variation axis records. The number of records, and the number of axes, is determined by the axisCount field. A functional variable font must have an axisCount value that is greater than zero. If axisCount is zero, then the font is not functional as a variable font, and must be treated as a non-variable font; any variation-specific tables or data is ignored.

VariationAxisRecord

The format of the variation axis record is as follows:

VariationAxisRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | axisTag | Tag identifying the design variation for the axis. |
| Fixed | minValue | The minimum coordinate value for the axis. |
| Fixed | defaultValue | The default coordinate value for the axis. |
| Fixed | maxValue | The maximum coordinate value for the axis. |
| uint16 | flags | Axis qualifiers – see details below. |
| uint16 | axisNameID | The name ID for entries in the 'name' table that provide a display name for this axis. |

Each axis has a tag that identifies the design variation for the axis. For example, the tag 'wght' designates a weight variation. Tags are registered for commonly-used design axes, but foundry-defined tags may also be used. Registered tags define valid ranges of coordinate values for the axis across all fonts. The variation axis record defines minimum and maximum values supported by the font, which may be more limited that the valid ranges defined for a registered tag.

NOTE	Axis values given in the variation axis record use user scale coordinates that are specific to each axis tag. The user scale for each registered tag is described with the definition of each tag. In most other font tables that contain variation-related data, axis coordinate values are expressed using normalized coordinate scales. For more information regarding user scales and normalized scales, and a specification of the normalization process, see the "Coordinate Scales and Normalization" section in the OFF "Font Variations Overview".

To be valid, axis tags must conform to general requirements for the Tag type (see 4.3) as well as certain additional requirements. Also, the valid patterns for registered tags and for foundry-defined tags are mutually exclusive. This is required to ensure there is never a future conflict between foundry-defined tags in existing fonts and newly-registered tags.

For more details on axis tags, their validity requirements, and definitions for registered design-variation axes, see 7.2.4 "Design-variations axis tags registry".

The default value interacts with the glyph and glyph variations tables in a particular way: the variation instance that has the default coordinate value for each axis will use glyph outlines as defined in the glyph table, without any variations from the glyph variations table applied. This instance is referred to as the default instance.

Flags can be assigned to indicate certain uses or behaviors for a given axis, independent of the specific axis tag and its definition. If no flags are set, then no assumptions are to be made beyond the definition for a registered axis. The following flags are defined.

| Mask | Name | Description |
| --- | --- | --- |
| 0x0001 | HIDDEN_AXIS | The axis should not be exposed directly in user interfaces. |
| 0xFFFE | Reserved | Reserved for future use – set to 0. |

The HIDDEN_AXIS flag is provided to indicate a recommendation by the font developer that the axis not be exposed directly to end users in application user interfaces. Reasons for setting this flag might include that the axis is intended only for programmatic interaction, or is intended for font-internal use by the font developer. If this flag is set, the axis should not be exposed to users in application user interfaces except in specialized scenarios, such as a font inspection utility. The flag should not affect handling of named instances, which should always be exposed in text-formatting user interfaces. If this flag is not set, then applications may expose the given axis in a default user interface or, based on the nature of the axis, may choose to expose it in an advanced-user interface.

The axisNameID field provides a name ID that can be used to obtain strings from the 'name' table that can be used to refer to the axis in application user interfaces. The name ID must be greater than 255 and less than 32768. For registered axis tags, a conventional US English axis name is provided; it is recommended that that name, or localized derivative names, be used in application user interfaces to provide greater consistency in user experience between different fonts.

InstanceRecord

The instance record format includes a UserTuple record that specifies a position within the font’s variation space. The UserTuple record has an array of user scale coordinates:

UserTuple record:

| Type | Name | Description |
| --- | --- | --- |
| Fixed | coordinates[axisCount] | Coordinate array specifying a position within the font’s variation space. |

NOTE	The UserTuple record and Tuple record (7.2.2.1) both describe a position in the variation space but are distinct: UserTuple uses Fixed values to represent user scale coordinates, while the Tuple record uses F2DOT14 values to represent normalized coordinates.

The format of the instance record is as follows:

InstanceRecord:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | subfamilyNameID | The name ID for entries in the 'name' table that provide subfamily names for this instance. |
| uint16 | flags | Reserved for future use – set to 0. |
| UserTuple | coordinates | The coordinate array for this instance. |
| uint16 | postScriptNameID | Optional. The name ID for entries in the 'name' table that provide PostScript names for this instance. |

The postScriptNameID field is optional. All of the instance records in a given font must be the same size, with all either including or omitting the postScriptNameID field.

The subfamilyNameID field provides a name ID that can be used to obtain strings from the 'name' table that can be treated as equivalent to name ID 17 (typographic subfamily) strings for the given instance. Values of 2 or 17 can be used; otherwise, values must be greater than 255 and less than 32768. The values 2 or 17 should only be used if the named instance corresponds to the font's default instance.

The postScriptNameID field provides a name ID that can be used to obtain strings from the 'name' table that can be treated as equivalent to name ID 6 (PostScript name) strings for the given instance. Values of 6 and 0xFFFF can be used; otherwise, values must be greater than 255 and less than 32768. If the value is 0xFFFF, then the value is ignored, and no PostScript name equivalent is provided for the instance. The value 6 should only be used if the named instance corresponds to the font's default instance.

All of the instance records in a font should have distinct coordinates and distinct subfamilyNameID and postScriptName ID values. If two or more records share the same coordinates, the same nameID values or the same postScriptNameID values, then all but the first can be ignored.

The default instance of a font is that instance for which the coordinate value of each axis is the defaultValue specified in the corresponding variation axis record. An instance record is not required for the default instance, though an instance record can be provided. When enumerating named instances, the default instance should be enumerated even if there is no corresponding instance record. If an instance record is included for the default instance (that is, an instance record has coordinates set to default values), then the nameID value should be set to 2 or 17 or to a name ID with the same value as name ID 2 or 17. Also, if a postScriptNameID is included in instance records, the value should be set to 6 or to a name ID with the same value as name ID 6.

NOTE	Since an instance record for the default instance is not required, a variable font that has no instance records defined in the 'fvar' table (instanceCount is zero) still has one named instance.

#### Variation Instance Selection

When formatting text using a variable font, applications must select a particular variation instance; that is, specific, in-range values must be specified for each of the axes defined in the font variation table. An instance may be selected by reference to a named instanced defined in an instance record, or by using a set of arbitrary axis values for the various axes. If a value is not specified for any particular axis, the default value for that axis defined in the font is used. If an application specifies a value for an axis that is less than the minValue defined in the font, then minValue must be used. Similarly, if an application specifies a value greater than the maxValue defined in the font, then maxValue must be used.

#### Example

This example is for a hypothetical font with family name “SelawikV” that has two axes of variation, for weight and width. This table summarizes the description of the axes for the font:

| Axis tag | Mininum value | Default value | Maximum value | Axis name ID |
| --- | --- | --- | --- | --- |
| ‘wght’ | 300 | 400 | 700 | 256 |
| ‘wdth’ | 62.5 | 100 | 150 | 257 |

This font also has the following named instances:

| Instance subfamily name | Subfamily name ID | PostScript name | PostScript name ID | ‘wght’ value | ‘wdth’ value |
| --- | --- | --- | --- | --- | --- |
| Regular | 258 | SelawikV-Regular | 262 | 400 | 100 |
| Bold | 259 | SelawikV-Bold | 263 | 700 | 100 |
| Condensed | 260 | SelawikV-Condensed | 264 | 400 | 75 |
| Condensed Bold | 261 | SelawikV-CondensedBold | 265 | 700 | 75 |

The 'fvar' table is constructed as follows:

| Hex data | Field | Comment |
| --- | --- | --- |
|  | Header |  |
| 0001 | majorVersion |  |
| 0000 | minorVersion |  |
| 0010 | axesArrayOffset | 16 bytes – combined size of fields before the axes array |
| 0002 | countSizePairs | 2 count/size pairs: axis, instance |
| 0002 | axisCount | 2 axes ('wght', 'wdth') |
| 0014 | axisSize | Size of each variation axis record is 20 bytes. |
| 0004 | instanceCount | 4 named instances. |
| 000E | instanceSize | Size of instance records is 14 bytes. |
|  | First variation axis record |  |
| 77676874 | axisTag | Axis tag 'wght'. |
| 012C0000 | minValue | Minimum 'wght' value is 300 (Fixed format). |
| 01900000 | defaultValue | Default 'wght' value is 400. |
| 02BC0000 | maxValue | Maximum 'wght' value is 700. |
| 0000 | flags |  |
| 0100 | axisNameID | Display name “Weight” uses name ID 256 |
|  | Second variation axis record |  |
| 77647468 | axisTag | Axis tag 'wdth' |
| 003E8000 | minValue | Minimum 'wdth' value is 62.5 (Fixed format). |
| 00640000 | defaultValue | Default 'wdth' value is 100. |
| 00960000 | maxValue | Maximum 'wdth' value is 150. |
| 0000 | flags |  |
| 0101 | axisNameID | Display name “Width” uses name ID 257. |
|  | First instance record |  |
| 0102 | subfamilyNameID | Instance subfamily name "Regular" uses name ID 258. |
| 0000 | flags |  |
| 01900000 | coordinates[0] | ‘wght’ coordinate is 400. |
| 00640000 | coordinates[1] | ‘wdth’ coordinate is 100. |
| 0106 | postScriptNameID | Instance PostScript name "SelawikV-Regular" uses name ID 262. |
|  | Second instance record |  |
| 0103 | subfamilyNameID | Instance subfamily name "Bold" uses name ID 259. |
| 0000 | flags |  |
| 02BC0000 | coordinates[0] | 'wght' coordinate is 700. |
| 00640000 | coordinates[1] | 'wdth' coordinate is 100. |
| 0107 | postScriptNameID | Instance PostScript name "SelawikV-Bold" uses name ID 263. |
|  | Third instance record |  |
| 0104 | subfamilyNameID | Instance subfamily name "Condensed" uses name ID 260. |
| 0000 | flags |  |
| 01900000 | coordinates[0] | 'wght' coordinate is 400. |
| 004B0000 | coordinates[1] | 'wdth' coordinate is 75. |
| 0108 | postScriptNameID | Instance PostScript name "SelawikV-Condensed" uses name ID 264. |
|  | Fourth instance record |  |
| 0105 | subfamilyNameID | Instance subfamily name "Condensed Bold" uses name ID 261. |
| 0000 | flags |  |
| 02BC0000 | coordinates[0] | 'wght' coordinate is 700. |
| 004B0000 | coordinates[1] | 'wdth' coordinate is 75. |
| 0109 | postScriptNameID | Instance PostScript name "SelawikV-CondensedBold" uses name ID 265. |

The total size of the table is 112 bytes.

### gvar—Glyph variations table

Font Variations allow a font designer to incorporate multiple faces within a font family into a single font resource. In a variable font, the font variations ('fvar') table defines a set of design variations supported by the font, and then various tables provide data that specify how different font values, such as X-height or X and Y coordinates for glyph outline points, are adjusted for different variation instances. The glyph variations ('gvar') table provides all of the variation data that describe how TrueType glyph outlines in a 'glyf' table change across the font’s variation space.

The glyph variations table can only be used in combination with TrueType outlines — a glyph ('glyf') table – and also in combination with a font variations ('fvar') table and other required or optional tables used in variable fonts.

The 'gvar' table contains glyph variation data sub-tables with variation data for each glyph in the 'glyf' table. The glyph variation data is a specific variant of the tuple variation store format. Another variant of the tuple variation store is also used in the 'cvar' table (see 7.2.2). Differences between these and details specific to the ‘gvar’ table are described in 7.2.2.8.

Variation data is comprised of many adjustment-delta values. These deltas apply to particular target items, such as the X or Y coordinate of some glyph outline point, and are applicable for instances within a particular region of the font’s variation space. The tuple variation store format organizes deltas into groupings by region of applicability, with a different group of data for each region. As glyph outlines often comprise the largest volume of data in a font, the tuple variation store format uses run-length encoding and other optimization mechanisms to provide efficient representation of the variation data.

Each region-specific grouping of data includes data covering all of the outline points for the given glyph. This means that the tuple variation store formats are suited to unpacking and processing delta values for all outline points at once, rather than for random outline points. In most application scenarios, glyph outline processing involves the entire glyph outline at once, so this bias in the format is generally not a particular limitation.

A notable exception, however, is the use of horizontal or vertical glyph metrics in text-layout operations that occur prior to rendering. The TrueType rasterizer dynamically generates “phantom” points for each glyph that represent horizontal and vertical advance widths and side bearings, and the variation data within the 'gvar' table includes data for these phantom points. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) Thus, a text-layout implementation could utilize the 'gvar' table to obtain interpolated glyph metrics for a given variation instance. Doing so, however, would require invocation of the rasterizer and processing of data for all outline points of each glyph rather than just the glyph-metric phantom points. As an alternative, the horizontal metrics variations ('HVAR') and vertical metrics variations ('VVAR') tables can provide variation data for glyph metrics that can be processed without invoking the rasterizer, and that use different formats that are better suited to processing data for particular items — advances or side bearings for specific glyphs. For this reason, it is recommended that variable fonts include an 'HVAR' table, and also a 'VVAR' table if the font has 'vhea' and 'vmtx' tables to support vertical layout.

#### Glyph variations table format

The glyph variations table is comprised of a header followed by GlyphVariationData subtables for each glyph that describe the ways that each glyph is transformed across the font’s variation space.

Each glyph variation data table includes sets of data that reference various regions within the font’s variation space. Each region is defined using one or three tuple records, with a “peak” tuple record required. In many cases, a region referenced by one glyph will also be referenced by many other glyphs. As an optimization, the 'gvar' table allows for a shared set of tuple records that can be referenced by the tuple variation store data for any glyph.

The high-level structure of the 'gvar' table is as follows:

Figure 7.8: High-level organization of 'gvar' table

The header includes offsets to the start of the shared tuples data, and to the start of the glyph variation data tables.

Each glyph variation data table provides variation data for a particular glyph. These are variable in size. For this reason, the header also includes an array of offsets for each glyph variation data table from the start of the glyph variation data table array. There is one offset corresponding to each glyph ID, plus one extra offset. (Note that the same scheme is also used in the index to location (‘loca’) table.) The difference between two consecutive offsets in the array indicates the size of a given table, with an extra offset in the array to indicate the size of the last table. Some sizes derived in this way may be zero, in which case there is no glyph variation data for that particular glyph, and the same outline is used for that glyph ID across the entire variation space.

##### 'gvar' header

The glyph variations table header format is as follows:

‘gvar’ header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the glyph variations table – set to 1. |
| uint16 | minorVersion | Minor version number of the glyph variations table – set to 0. |
| uint16 | axisCount | The number of variation axes for this font. This must be the same number as axisCount in the 'fvar' table. |
| uint16 | sharedTupleCount | The number of shared tuple records. Shared tuple records can be referenced within glyph variation data tables for multiple glyphs, as opposed to other tuple records stored directly within a glyph variation data table. |
| Offset32 | sharedTuplesOffset | Offset from the start of this table to the shared tuple records. |
| uint16 | glyphCount | The number of glyphs in this font. This must match the number of glyphs stored elsewhere in the font. |
| uint16 | flags | Bit-field that gives the format of the offset array that follows. If bit 1 is clear, the offsets are uint16; if bit 1 is set, the offsets are uint32. |
| Offset32 | glyphVariationDataArrayOffset | Offset from the start of this table to the array of GlyphVariationData tables. |
| Offset16 or Offset32 | glyphVariationDataOffsets [glyphCount + 1] | Offsets from the start of the GlyphVariationData array to each GlyphVariationData table. |

If the short format (Offset16) is used for offsets, the value stored is the offset divided by 2. Hence, the actual offset for the location of the GlyphVariationData table within the font will be the value stored in the offsets array multiplied by 2.

##### Shared tuples array

The shared tuples array provides a set of variation-space positions that can be referenced by variation data for any glyph. The shared tuples array follows the glyphVariationData offsets array at the end of the 'gvar' header. This data is simply an array of tuple records, each representing a position in the font’s variation space.

Shared tuples array:

| Type | Name | Description |
| --- | --- | --- |
| Tuple | sharedTuples[sharedTupleCount] | Array of tuple records shared across all glyph variation data tables. |

Tuple records that are in the shared array or that are contained directly within a given glyph variation data table use 2.14 values to represent normalized coordinate values. See 7.2.2.1 for details.

##### The GlyphVariationData table array

The GlyphVariationData table array follows the 'gvar' header and shared tuples array. Each GlyphVariationData table describes the variation data for a single glyph in the font.

The glyph variation data table is a specific form of the tuple variation store format, described in 7.2.2. It is comprised of a header, followed by serialized data.

Figure 7.9: High-level organization of GlyphVariationData table

The variation data includes logical groupings of data that apply to different regions of the variation space – tuple variation data tables. These logical groupings are stored in two parts: a header, and serialized data. The glyph variation data header includes an array of tuple variation-data headers, each of which is associated with a particular portion of the serialized data.

The serialized data includes adjustment delta values and also packed “point” numbers that identify the points in the glyph outline to which the deltas apply. In the case of a composite glyph, these numbers are component indices rather than point number indices. The serialized data for a given region can include point number data that applies to that specific tuple-variation data, but there can also be a shared set of point number data, stored at the start of the serialized data. Shared point number data can be used in relation to multiple tuple-variation data tables for the given glyph.

The glyphVariationData header is structured as follows:

GlyphVariationData header:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | tupleVariationCount | A packed field. The high 4 bits are flags, and the low 12 bits are the number of tuple variation tables for this glyph. The number of tuple variation tables can be any number between 1 and 4095. |
| Offset16 | dataOffset | Offset from the start of the GlyphVariationData table to the serialized data |
| TupleVariationHeader | tupleVariationHeaders [tupleCount] | Array of tuple variation headers. |

Complete details regarding the tupleVariationCount field and the flags used in that field, the TupleVariationHeader format, and the format of the serialized data are provided in 7.2.2.3 to 7.2.2.6.

#### Processing the 'gvar' table

When processing glyph data in a variable font for a particular variation instance, default glyph outline data will be obtained from 'glyf' table, which is combined with the corresponding glyph variation data subtable within the 'gvar' table.

The tuple variation headers within the selected glyph variation data table will each specify a particular region of applicability within the font’s variation space. These will be compared with the coordinates for the selected variation instance to determine which of the tuple-variation data tables are applicable, and to calculate a scalar value for each. These comparisons and scalar calculations are done using normalized-scale coordinate values.

For each of the tuple-variation data tables that are applicable, the point number and delta data will be unpacked and processed. The data for applicable regions can be processed in any order. Derived delta values will correspond to particular point numbers derived from the packed point number data. For a given point number, the computed scalar is applied to the X coordinate and Y coordinate deltas as a coefficient, and then resulting delta adjustments applied to the X and Y coordinates of the point.

There are two aspects of processing that are specific to the 'gvar' table. First, in the case of composite glyphs, point numbers refer either to components or to phantom points. Adjustments of phantom points are handled as for regular points, but adjustments to components are handled differently. Additional information regarding processing of variation data for composite glyphs is provided below.

Secondly, within a given tuple-variation data table, deltas may be provided for all of the glyph’s points (and phantom points), or only for some points. If deltas are provided for only some point numbers in a glyph outline, then delta values for un-referenced points may need to be inferred using interpolation. This is additional processing specific to the 'gvar' table, and is described below.

#### Point Numbers and processing for composite glyphs

The general discussion above and in the description of interpolation in the Overview chapter assumes simple glyphs. Certain special considerations apply to composite glyphs.

The variation data for composite glyphs also use packed point number data representing a series of numbers, but the numbers in this case, apart from the last four “phantom” point numbers, refer to the components that make up the glyph rather than to outline points. The glyph components of a composite glyph are assigned these pseudo-point numbers according to the order the components are given in the glyph entry, starting with 0. The four phantom point numbers representing side bearings are still added at the end, as for simple glyphs.

For example, consider a composite glyph for “é” made up of two components: the base glyph “e”, and a glyph for the accent, positioned at a specified offset. Pseudo- and phantom point numbers would be as follows:

| Point number | Element |
| --- | --- |
| 0 | Base glyph |
| 1 | Accent glyph |
| 2 | Left side bearing point |
| 3 | Right side bearing point |
| 4 | Top side bearing point |
| 5 | Bottom side bearing point |

Packed point number data for this glyph could include numbers referencing any or all of these elements.

The adjustment deltas for component glyphs adjust the placement of the component. If a glyph component is not referenced in the packed point numbers, then its position is not adjusted. Each component glyph within a composite will have its own glyph entry that, itself, has its own variation data. The processing of component glyphs must begin with the most deeply-nested, non-composite glyphs.

The position of a component can be represented in one of two ways: directly using X and Y offset values, or indirectly using point numbers in the parent and component glyphs that get aligned. Which method is used is determined by a bit in the flags field of a composite glyph description: if ARGS_ARE_XY_VALUES (bit 1) is set, then X and Y offsets are used; if that bit is clear, then point numbers are used. If the position of a component is represented using X and Y offsets – the ARGS_ARE_XY_VALUES flag is set – then adjustment deltas can be applied to those offsets. However, if the position of a component is represented using point numbers – the ARGS_ARE_XY_VALUES flag is not set – then adjustment deltas have no effect on that component and should not be specified.

In addition to a component’s placement, the composite glyph description can also specify a scale value or a 2 × 2 matrix transformation to be applied to the component. Adjustment deltas do not have any effect on scaling or 2 × 2 transformations applied to a component. The deltas only affect the positioning of the component.

If any component entry in a composite glyph has the USE_MY_METRICS flag set, then the hinted glyph metrics for the composite as a whole are taken from that component, rather than using the values given for the composite glyph itself. That is, the positions of phantom points for the composite glyph are set to the hinted positions of the component’s phantom points. If more than one component glyph has this flag set, then the metrics for the composite glyph are taken from the last component that has this flag set. In a variable font, if a component entry has this flag set, then the phantom point positions for the composite glyph are set to the interpolated and hinted positions of the component’s phantom point, and delta values for the composite glyph’s phantom points are not used.

NOTE	When a composite glyph has a component with the USE_MY_METRICS flag set, the ‘hmtx’ and 'HVAR' data for the composite glyph are used in the same manner in which the ‘hmtx’ data would be used for a non-variable font. The ‘hmtx’ and 'HVAR' data should be set to appropriate values for the composite glyph, though the hinted phantom point positions may not exactly match the linearly-scaled metrics obtained from the ‘hmtx’ and 'HVAR' data.

The process for transforming a composite glyph is as follows:

Let componentCount be the number of components in the composite glyph. Let components[] be the component descriptions of a composite glyph (base 0 index), and let C be a component entry in a composite glyph description.

Using the standard variation interpolation algorithm, process the variation data for the glyph to obtain net X and Y delta adjustments for each of the components and phantom points.

Apply the net X and Y delta adjustments to the phantom points (p = componentCount to componentCount + 3; note that the phantom point positions may be later overridden by a component):

For AW and LSB points, apply the net X delta adjustments, and ignore Y deltas.

For AH and TSB points, apply the net Y delta adjustments, and ignore X deltas.

For p = 0 to componentCount - 1:

C = components[p]

For the glyph at C.glyphIndex, apply the variation interpolation process and run the hinting program.

If ARGS_ARE_XY_VALUES is set in C.flags:

Apply net X and Y delta adjustments for index p to the X and Y positioning offsets for C:

X position offset = C.argument1 + netAdjustmentX
Y position offset = C.argument2 + netAdjustmentY

Else (ARGS_ARE_XY_VALUES is not set):

Ignore any delta adjustments provided for this component.

Apply scaling to C as applicable — if positioning offsets are scaled, it is the delta-adjusted offsets that are scaled.

If USE_MY_METRICS is set in C.flags:

Set the positions of the composite’s phantom points equal to the hinted positions of the phantom points for C.

Loop (next p)

For example, consider glyph 128 of the Skia font, which is the glyph for “Ä”. The glyph entry has two component entries, both with ARGS_ARE_XY_VALUES set. The advance width in the ‘hmtx’ table is 1358, and the left side bearing is 16; this puts left side bearing and right side bearing phantom points (point indices 2 and 3) at (0, 0) and (1358, 0). Relevant details from the composite glyph description are as follows:

| Component | glyphIndex | argument1 (X offset) | argument2 (Y offset) |
| --- | --- | --- | --- |
| 0 | 65 | 0 | 0 |
| 1 | 315 | 286 | 0 |

Glyph 128 has tuple variation data for various regions within the variation space; three will be considered for this example, with normalized coordinate positions as follows:

| Region | (weight, width) |
| --- | --- |
| R1 | (1, 0) |
| R2 | (0, 1) |
| R3 | (1, 1) |

The data for R1 the following deltas:

|  | pt 0 | pt 1 | pt 2 | pt 3 |
| --- | --- | --- | --- | --- |
| X | 0 | 69 | 58 | 145 |
| Y | 0 | 0 | 0 | 0 |

The data for R2 the following deltas:

|  | pt 0 | pt 1 | pt 2 | pt 3 |
| --- | --- | --- | --- | --- |
| X | 0 | 53 | 38 | 351 |
| Y | 0 | 0 | 0 | 0 |

The data for R3 the following deltas:

|  | pt 0 | pt 1 | pt 2 | pt 3 |
| --- | --- | --- | --- | --- |
| X | 0 | 21 | -6 | 25 |
| Y | 0 | 0 | 0 | 0 |

As all of the Y delta values are zero, there is no adjustment to Y coordinate values.

Now consider a variation instance with coordinates (0.2, 0.7). Variation data for all three of the regions above will have an effect, with scalar values as follow:

| Region | Scalar |
| --- | --- |
| R1 | 0.2 |
| R2 | 0.7 |
| R3 | 0.14 |

The net adjustment to X coordinate values for component offsets and phantom points will be as follows:

| Item | Derived X coordinate adjustment |
| --- | --- |
| Component 1 X offset | 0.2 × 69 + 0.7 × 53 + 0.14 × 21 = 53.84 |
| Left side bearing point X coordinate | 0.2 × 58 + 0.7 × 38 + 0.14 × -6 = 37.36 |
| Right side bearing point X coordinate | 0.2 × 145 + 0.7 × 351 + 0.14 × 25 = 278.2 |

Combining these adjustments with the default values in the 'glyf' entry yields the following:

| Component 0 (X, Y) offset | (0, 0) |
| --- | --- |
| Component 1 (X, Y) offset | (286 + 53.84, 0) = (339.84, 0) |
| Left side bearing phantom point | (0 + 37.36, 0) = (37.36, 0) |
| Right side bearing phantom point | (1358 + 278.2, 0) = (1636.2, 0) |

#### Inferred deltas for un-referenced point numbers

The tuple variation data for a given glyph and region (a given tuple variation data set) may include deltas for all outline points, or for only some. The packed point number data identify the points for which deltas are provided. If some points are omitted from the list of point numbers, then the data does not explicitly include delta values for them, and deltas may need to be inferred. This is done for a given glyph on a region-by-region basis based on the point numbers specified with each set of tuple variation data.

NOTE	Inferring of deltas for un-referenced points applies only to simple glyphs, not to composite glyphs.

A single set of point-number data is used for both X- and Y-direction deltas. If a point has explicit deltas, then it has explicit deltas for both X and Y directions. If the point requires inferred deltas, then both X and Y deltas are inferred. The values of inferred X and Y deltas are calculated separately.

The scalar calculated for a given region and variation instance is applied to the inferred deltas to obtain scaled delta adjustments that are applied to the point coordinates, just as for explicit deltas.

The process of calculating inferred variation deltas is somewhat comparable to the TrueType Interpolate Untouched Points (IUP) instruction. Both calculate an adjustment for an unaffected point based on the adjustments to adjacent, affected points. In the case of the IUP instruction, however, the calculated adjustment is based on the position of adjacent points after other instructions have applied. In contrast, calculation of inferred variation deltas is based on the default positions of points and the unscaled delta values for a given region. It is not impacted by the order in which the tuple-variation data for different regions is processed, and the inferred deltas can be pre-computed before any processing for a specific instance is done. Also, the calculations used for deriving the value of inferred deltas are slightly different from the calculations used for the IUP instruction.

Calculation of inferred deltas is done for a given glyph and a given region on a contour-by-contour basis.

For a given contour, if the point number list does not include any of the points in that contour, then none of the points in the contour are affected and no inferred deltas need to be computed.

If the point number list includes some but not all of the points in a given contour, then inferred deltas must be derived for the points that were not included in the point number list, as follows.

First, for any un-referenced point, identify the nearest points before and after, in point number order, that are referenced. Note that the same referenced points will be used for calculating both X and Y inferred deltas. If there is no lower point number from that contour that was referenced, then the highest, referenced point number from that contour is used. Similarly, if no higher point number from that contour was referenced, then the lowest, referenced point number is used.

Once the adjacent, referenced points are identified, then inferred-delta calculation is done separately for X and Y directions.

Next, the (X or Y) grid coordinate values of the adjacent, referenced points are compared. If these coordinates are the same, then the delta values for the adjacent points are compared: if the delta values are the same, then this value is used as the inferred delta for the target, un-referenced point. If the delta values are different, then the inferred delta for the target point is zero.

NOTE 	If exactly one point from the contour is referenced in the point number list, then every point in that contour uses the same X and Y delta values as that point. This follows as a specific case of the above: for all other points that are not referenced, the one referenced point is at once both the preceding adjacent point and the following adjacent point. Hence, the adjacent points have the same coordinate value and the same delta, and therefore the un-referenced points get an inferred delta of the same value.

If the coordinates of the adjacent, reference points are different, then the coordinate for the same (X or Y) direction of the target point is compared to those coordinates. If the coordinate of the target point is between the coordinates of the adjacent points, then a delta is interpolated, as described below. But if the coordinate of the target point is not between the coordinates of the adjacent points, then the inferred delta is the delta for whichever of the adjacent points is closer in the given direction.

The following pseudo-code summarizes the above details.

if precedingPoint.coord = followingPoint.coord

{

if precedingPoint.delta = followingPoint.delta

targetPoint.delta = precedingPoint.delta

else

targetPoint.delta = 0

}

else  /* precedingPoint.coord <> followingPoint.coord */

{

if targetPoint.coord <= min(precedingPoint.coord, followingPoint.coord)

{

if precedingPoint.coord < followingPoint.coord

targetPoint.delta = precedingPoint.delta

else  /* followingPoint.coord < precedingPoint.coord */

targetPoint.delta = followingPoint.delta

}

else if targetPoint.coord >= max(precedingPoint.coord, followingPoint.coord)

{

if precedingPoint.coord > followingPoint.coord, then

targetPoint.delta = precedingPoint.delta

else  /* followingPoint.coord > precedingPoint.coord */

targetPoint.delta = followingPoint.delta

}

else  /* target point coordinate is between adjacent point coordinates */

{

/* target point delta is derived from the adjacent point deltas

using linear interpolation */

}

}

When the coordinates of the two adjacent points are different and the coordinate of the target point is between those coordinates, then a delta for the target point is computed by linear interpolation of the deltas for the two adjacent points. The following describes this calculation for either X or Y direction.

NOTE 	The logical flow of the algorithm to this point implies that the coordinates of the two adjacent points are different. This avoids a division by zero in the following calculations that would otherwise occur.

Select one of the two adjacent points as the reference point, and let the other be the comparison point. It doesn’t matter which is which. Let refCoord be the coordinate value for the current direction of the former, and let compCoord be the coordinate value for the latter. Let targetCoord be the coordinate value for the target, un-referenced point. Calculate a proportion, proportion, as follows:

Now let deltaRef and deltaComp be the unscaled adjustment-delta values in the variation data for the reference and comparison points. The inferred delta for the target point, deltaTarget, is calculated as follows:

The following example illustrates the process for obtaining inferred deltas. Suppose P1, P2 and P3 are points in the same contour, with coordinate positions as shown below, and that P1 and P3 are referenced in the point number data while P2 is not.

Note that P1 and P3 have different X and Y coordinates. Also note that, in the X direction, P2 is between the two adjacent points, while in the Y direction it is not. For the Y direction, the inferred delta will be the delta of the closer point, P3.

For the X direction, let P1 be the reference point; P3 is the comparison point. A proportion is calculated:

Now suppose that P1 and P3 have X and Y deltas in the variation data as follows:

| Point | X delta | Y delta |
| --- | --- | --- |
| P1 | +28 | -62 |
| P3 | -42 | -57 |

The inferred X delta for P2, deltaX_P2, is calculated as follows:

The inferred Y delta for P2 is the value of the Y delta for P3. Thus, the deltas for all three points is obtained:

| Point | X delta | Y delta |
| --- | --- | --- |
| P1 | +28 | -62 |
| P2 | +10.5 | -57 |
| P3 | -42 | -57 |

These delta values can now be used in the interpolation algorithm, with a scalar applied to each based on the region coordinates and the coordinates for the current variation instance.

### HVAR—Horizontal metrics variations table

The 'HVAR' table is used in variable fonts to provide variations for horizontal glyph metrics values. This can be used to provide variation data for advance widths in the ‘hmtx’ table. In fonts with TrueType outlines, it can also be used to provide variation data for left and right side bearings obtained from the ‘hmtx’ table and glyph bounding boxes.

For a general overview of OFF Font Variations and terminology related to variations, see subclause 7.1.

In a font with TrueType outlines, the rasterizer will generate “phantom” points that represent left, right, top and bottom side bearings. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) In a TrueType variable font, the glyph variations (‘gvar’) table will include variation data for the phantom points of each glyph, allowing glyph metrics to be interpolated for different variation instances as part of deriving the interpolated glyph outlines. For this reason, the 'HVAR' table is not required in variable fonts that have TrueType outlines. For text-layout operations that require glyph metrics but not actual glyph outlines, however, there can be significant performance benefits by being able to obtain adjusted glyph metrics for an instance without needing to interpolate glyph outlines. For this reason, it is recommended that an 'HVAR' table be included in variable fonts that have TrueType outlines.

The CFF2 rasterizer does not generate phantom points as in the TrueType rasterizer. For this reason, an 'HVAR' table is required to handle any variation in horizontal glyph metrics in a variable font with CFF2 outlines.

Take note that, in a variable font with TrueType outlines, the left side bearing for each glyph must equal xMin, and bit 1 in the flags field of the 'head' table must be set.

The 'HVAR' table contains an item variation store table to represent variation data. The item variation store and constituent formats are described in subclause 7.2. The item variation store is also used in the 'VVAR', 'GDEF' and certain other tables, but is different from the formats for variation data used in the 'cvar' or 'gvar' tables.

The item variation store format uses delta-set indices to reference variation delta data for particular target, font-data items to which they are applied, such as the advance width of a particular glyph. Data external to the item variation store identifies the delta-set index to be used for each given target item. Within the 'HVAR' table, glyph IDs can be used as implicit indices for advance width variations, or an optional delta-set index mapping table can be used that explicitly provides delta-set indices for advance widths to be associated with each glyph ID.

An advance width mapping subtable adds additional data within the HVAR table, but it also makes it possible to use a more compact representation of the data in the item variation store. For example, if multiple glyphs have the same advance widths, the mapping subtable allows all of them to reference a single delta set within the store. Additional optimizations within the item variation store are possible. See the Common Table Formats chapter for more discussion about size optimization. In general, inclusion of an advance width mapping subtable is recommended.

Optional mapping subtables can also be used to provide delta-set indices for glyph side bearings. In variable fonts with TrueType outlines, variation data for side bearings is recommended. If variation data for side bearings is provided, it should include data for both left and right side bearings, and mapping subtables for left and right side bearings shall also be included.

#### Related and co-requisite tables

The 'HVAR' table is used only in variable fonts. It shall be used in combination with a horizontal metrics ('hmtx') table, and also in combination with a font variations ('fvar') table, and other required or optional tables used in variable fonts. See subclause 7.1.6 in the OFF Font Variations overview for general information.

For variable fonts that have TrueType outlines, the 'HVAR' table is optional but recommended. For variable fonts that have CFF2 outlines, the 'HVAR' table is required if there is any variation in glyph advance widths across the variation space.

NOTE	The 'hdmx' table is not used in variable fonts.

#### Table formats

The horizontal metrics variations table has the following format:

Horizontal metrics variations table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the horizontal metrics variations table – set to 1. |
| uint16 | minorVersion | Minor version number of the horizontal metrics variations table – set to 0. |
| Offset32 | itemVariationStoreOffset | Offset in bytes from the start of this table to the item variation store table. |
| Offset32 | advanceWidthMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for advance widths (may be NULL). |
| Offset32 | lsbMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for left side bearings (may be NULL). |
| Offset32 | rsbMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for right side bearings (may be NULL). |

The item variation store table is documented in subclause 7.2.

Mapping subtables are represented using a DeltaSetIndexMap table; see 7.2.3.1 for the definition of this table. In the HVAR table, only format 0 of the DeltaSetIndexMap is used. Mapping subtables are optional. If a given mapping subtable is not provided, the offset is set to NULL.

Variation data for advance widths is required. A delta-set index mapping subtable for advance widths can be provided, but is optional. If a mapping subtable is not provided, glyph indices are used as implicit delta-set indices. To access the delta set for the advance of given glyph, the delta-set outer-level index is zero, and the glyph ID is used as the inner-level index.

Variation data for side bearings are optional. If included, mapping tables are required to provide the delta-set index for each glyph.

#### Processing

When performing text layout using a particular variation instance of a variable font, the application will need to obtain adjusted glyph metrics for that instance. The application obtains default values from the 'hmtx' and 'glyf' tables, and uses the 'HVAR' table to obtain interpolated adjustment values that are applied to the defaults.

Delta-set indices are obtained based on the glyph ID. If there is no delta-set index mapping subtable for advance widths (advanceWidthMappingOffset is NULL), then glyph IDs implicitly provide the indices: for a given glyph ID, the delta-set outer-level index is zero, and the glyph ID is the delta-set inner-level index. If delta-set index mapping table is provided, glyph IDs are used to lookup a mapping entry, which provides the outer- and inner-level delta-set indices in a packed format. See 7.2.3.1 for details.

The delta-set indices are used to reference a delta set for the target advance width or side bearing within the item variation store. The two-level organization of data within the item variation store is described in subclause 7.2. Each delta set includes different deltas that apply to variation instances falling within different regions of the variation space. The process by which the deltas are processed to derive an interpolated value for a given target item is described in subclause 7.1.

### MVAR—Metrics variations table

The metrics variations table is used in variable fonts to provide variations for font-wide metric values found in the OS/2 table and other font tables. For a general overview of OFF Font Variations and terminology related to variations, see subclause 7.1.

The metrics variations table contains an item variation store structure to represent variation data. The item variation store and constituent formats are described in subclause 7.2. The item variation store is also used in the 'HVAR' and 'GDEF' tables, but is different from the formats for variation data used in the 'cvar' or 'gvar' tables.

The item variation store format uses delta-set indices to reference variation delta data for particular target font-data items to which they are applied. Data external to the item variation store identifies the delta-set index to be used for each given target item. Within the 'MVAR' table, an array of value tag records identifies a set of target items, and provides the delta-set index used for each. The target items are identified by four-byte tags, with a given tag representing some font-wide value found in another table. For example, the tag 'hasc' represents the OS/2.sTypoAscender value. More details on tags are provided below.

The item variation store format uses a two-level organization for variation data: a store can have multiple item variation data subtables, and each subtable has multiple delta-set rows. A delta-set index is a two-part index: an outer index that selects a particular item variation data subtable, and an inner index that selects a particular delta-set row within that subtable. A value record specifies both the outer and inner portions of the delta-set index.

NOTE	Apple platforms allow for use of a font metrics ('fmtx') table to specify various font-wide metric values by reference to the X or Y coordinates of contour points for a specified “magic” glyph. OFF Font Variations does not use the font metrics table.

The metrics variations table shall be used in combination with a font variations ('fvar') table and other required or optional tables used in variable fonts. See "Variation data tables and miscellaneous requirements" for additional details.

#### Table formats

The metrics variations table has the following format:

Metrics variations table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the metrics variations table – set to 1. |
| uint16 | minorVersion | Minor version number of the metrics variations table – set to 0. |
| uint16 | (reserved) | Not used; set to 0. |
| uint16 | valueRecordSize | The size in bytes of each value record — must be greater than zero. |
| uint16 | valueRecordCount | The number of value records — may be zero. |
| Offset16 | itemVariationStoreOffset | Offset in bytes from the start of this table to the item variation store table. If valueRecordCount is zero, set to zero; if valueRecordCount is greater than zero, must be greater than zero. |
| ValueRecord | valueRecords[valueRecordCount] | Array of value records that identify target items and the associated delta-set index for each. The valueTag records must be in binary order of their valueTag field. |

The valueRecordSize field indicates the size of each value record. Future, minor version updates of the 'MVAR' table may define compatible extensions to the value record format with additional fields. Implementations shall use the valueRecordSize field to determine the start of each record.

The valueRecords array is an array of value records that identify the target, font-wide measures for which variation adjustment data is provided (target items), and outer and inner delta-set indices for each item into the item variation store data.

ValueRecord:

| Type | Name | Description |
| --- | --- | --- |
| Tag | valueTag | Four-byte tag identifying a font-wide measure. |
| uint16 | deltaSetOuterIndex | A delta-set outer index — used to select an item variation data subtable within the item variation store. |
| uint16 | deltaSetInnerIndex | A delta-set inner index — used to select a delta-set row within an item variation data subtable. |

The value records must be given in binary order of the valueTag values. Each tag identifies a font-wide measure found in some other font table. For example, if a value record has a value tag of 'hasc', this corresponds to the OS/2.sTypoAscender field. Details on the tags used within the 'MVAR' table are provided below.

#### Processing

When reading a value within a variable font, such as the OS/2.sCapHeight value (the target item), the value tags array in the metrics variations table is scanned to find the tag that corresponds to that target item. Records in the array are stored in binary order of values in the valueTag fields. If the tag does not occur in the tag array, then the item is constant across the font’s variation space. If the tag does occur, however, then the delta-set index is used to reference a set of deltas within the item variation store. The two-level organization of data within the item variation store is described in subclause 7.2. Each delta set includes different deltas that apply to variation instances falling within different regions of the variation space. The process by which the deltas are processed to derive an interpolated value for a given target item is described in the OFF Font Variations overview.

#### Value tags

Four-byte tags are used to represent particular metric or other values. For example, the tag 'hasc' (horizontal ascent) is used to represent the OS/2.sTypoAscender value. Tags are defined for various values found in the OS/2 and Windows metrics ('OS/2') table, the horizontal header ('hhea') table, the grid-fitting and scan-conversion ('gasp') table, the PostScript ('post') table, and the vertical metrics header ('vhea') table.

NOTE	The OS/2.usWeightClass, OS/2.usWidthClass and post.italicAngle values are not supported by variation data in the 'MVAR' table. This is because values for these three fields correspond directly to input axis values for the 'wght', 'wdth' and 'slnt' variation axes. See the discussion of these axes in the "Design-variation axis tags registry" for details on the relationship between these fields and the corresponding design axes.

Tags in the metrics variations table are case sensitive. Tags defined in this table use only lowercase letters or digits.

Tags that are used in a font’s metrics variations table should be those that are documented in this table specification. A font may also use privately-defined tags, which have semantics known only by private agreement. Private-use tags must begin with an uppercase letter and use only uppercase letters or digits. If a private-use tag is used in a given font, any application that does not recognize that tag should ignore it.

The following tags are defined:

Value tags, ordered by logical groupings

| Value | Mnemonic | Value represented |
| --- | --- | --- |
| 'hasc' | horizontal ascender | OS/2.sTypoAscender |
| 'hdsc' | horizontal descender | OS/2.sTypoDescender |
| 'hlgp' | horizontal line gap | OS/2.sTypoLineGap |
| 'hcla' | horizontal clipping ascent | OS/2.usWinAscent |
| 'hcld' | horizontal clipping descent | OS/2.usWinDescent |
| 'vasc' | vertical ascender | vhea.ascent |
| 'vdsc' | vertical descender | vhea.descent |
| 'vlgp' | vertical line gap | vhea.lineGap |
| 'hcrs' | horizontal caret rise | hhea.caretSlopeRise |
| 'hcrn' | horizontal caret run | hhea.caretSlopeRun |
| 'hcof' | horizontal caret offset | hhea.caretOffset |
| 'vcrs' | vertical caret rise | vhea.caretSlopeRise |
| 'vcrn' | vertical caret run | vhea.caretSlopeRun |
| 'vcof' | vertical caret offset | vhea.caretOffset |
| 'xhgt' | x height | OS/2.sxHeight |
| 'cpht' | cap height | OS/2.sCapHeight |
| 'sbxs' | subscript em x size | OS/2.ySubscriptXSize |
| 'sbys' | subscript em y size | OS/2.ySubscriptYSize |
| 'sbxo' | subscript em x offset | OS/2.ySubscriptXOffset |
| 'sbyo' | subscript em y offset | OS/2.ySubscriptYOffset |
| 'spxs' | superscript em x size | OS/2.ySuperscriptXSize |
| 'spys' | superscript em y size | OS/2.ySuperscriptYSize |
| 'spxo' | superscript em x offset | OS/2.ySuperscriptXOffset |
| 'spyo' | superscript em y offset | OS/2.ySuperscriptYOffset |
| 'strs' | strikeout size | OS/2.yStrikeoutSize |
| 'stro' | strikeout offset | OS/2.yStrikeoutPosition |
| 'unds' | underline size | post.underlineThickness |
| 'undo' | underline offset | post.underlinePosition |
| 'gsp0' | gaspRange[0] | gasp.gaspRange[0].rangeMaxPPEM |
| 'gsp1' | gaspRange[1] | gasp.gaspRange[1].rangeMaxPPEM |
| 'gsp2' | gaspRange[2] | gasp.gaspRange[2].rangeMaxPPEM |
| 'gsp3' | gaspRange[3] | gasp.gaspRange[3].rangeMaxPPEM |
| 'gsp4' | gaspRange[4] | gasp.gaspRange[4].rangeMaxPPEM |
| 'gsp5' | gaspRange[5] | gasp.gaspRange[5].rangeMaxPPEM |
| 'gsp6' | gaspRange[6] | gasp.gaspRange[6].rangeMaxPPEM |
| 'gsp7' | gaspRange[7] | gasp.gaspRange[7].rangeMaxPPEM |
| 'gsp8' | gaspRange[8] | gasp.gaspRange[8].rangeMaxPPEM |
| 'gsp9' | gaspRange[9] | gasp.gaspRange[9].rangeMaxPPEM |

Value tags, in alphabetical order of tags:

| Value | Mnemonic | Value represented |
| --- | --- | --- |
| 'cpht | cap height | OS/2.sCapHeight |
| 'gsp0' | gaspRange[0] | gasp.gaspRange[0].rangeMaxPPEM |
| 'gsp1' | gaspRange[1] | gasp.gaspRange[1].rangeMaxPPEM |
| 'gsp2' | gaspRange[2] | gasp.gaspRange[2].rangeMaxPPEM |
| 'gsp3' | gaspRange[3] | gasp.gaspRange[3].rangeMaxPPEM |
| 'gsp4' | gaspRange[4] | gasp.gaspRange[4].rangeMaxPPEM |
| 'gsp5' | gaspRange[5] | gasp.gaspRange[5].rangeMaxPPEM |
| 'gsp6' | gaspRange[6] | gasp.gaspRange[6].rangeMaxPPEM |
| 'gsp7' | gaspRange[7] | gasp.gaspRange[7].rangeMaxPPEM |
| 'gsp8' | gaspRange[8] | gasp.gaspRange[8].rangeMaxPPEM |
| 'gsp9' | gaspRange[9] | gasp.gaspRange[9].rangeMaxPPEM |
| 'hasc' | horizontal ascender | OS/2.sTypoAscender |
| 'hcla' | horizontal clipping ascent | OS/2.usWinAscent |
| 'hcld' | horizontal clipping descent | OS/2.usWinDescent |
| 'hcof' | horizontal caret offset | hhea.caretOffset |
| 'hcrn' | horizontal caret run | hhea.caretSlopeRun |
| 'hcrs' | horizontal caret rise | hhea.caretSlopeRise |
| 'hdsc' | horizontal descender | OS/2.sTypoDescender |
| 'hlgp' | horizontal line gap | OS/2.sTypoLineGap |
| 'sbxo' | subscript em x offset | OS/2.ySubscriptXOffset |
| 'sbxs' | subscript em x size | OS/2.ySubscriptXSize |
| 'sbyo' | subscript em y offset | OS/2.ySubscriptYOffset |
| 'sbys' | subscript em y size | OS/2.ySubscriptYSize |
| 'spxo' | superscript em x offset | OS/2.ySuperscriptXOffset |
| 'spxs' | superscript em x size | OS/2.ySuperscriptXSize |
| 'spyo' | superscript em y offset | OS/2.ySuperscriptYOffset |
| 'spys' | superscript em y size | OS/2.ySuperscriptYSize |
| 'stro' | strikeout offset | OS/2.yStrikeoutPosition |
| 'strs' | strikeout size | OS/2.yStrikeoutSize |
| 'undo' | underline offset | post.underlinePosition |
| 'unds' | underline size | post.underlineThickness |
| 'vasc' | vertical ascender | vhea.ascent |
| 'vcof' | vertical caret offset | vhea.caretOffset |
| 'vcrn' | vertical caret run | vhea.caretSlopeRun |
| 'vcrs' | vertical caret rise | vhea.caretSlopeRise |
| 'vdsc' | vertical descender | vhea.descent |
| 'vlgp' | vertical line gap | vhea.lineGap |
| 'xhgt' | x height | OS/2.sxHeight |

Note that the tags 'gsp0' to 'gsp9' are used to provide variation data for the rangeMaxPPEM member of records in a grid-fitting and scan-conversion procedure ('gasp') table. The last 'gasp' table entry always uses a rangeMaxPPEM value of 0xFFFF. The maximum number of value records for 'gasp' entries must never be more than one less the number of entries in the 'gasp' table.

### STAT—Style attributes table

The style attributes table describes design attributes that distinguish font-style variants within a font family. It also provides associations between those attributes and name elements that may be used to present font options within application user interfaces. This information is especially important for variable fonts, but also relevant for non-variable fonts.

A font family is a set of font faces that share key aspects of design that are common to all of the fonts in the family, and differentiate that family from other font families. The fonts within a family also differ from one another in particular ways: in stroke thicknesses, in contrast, etc.; or combinations of such differences. In this way, the fonts within a family are style variants of the family design.

A given family will have a particular set of attribute types by which the member fonts differ: the axes of variation. These may be implemented as dynamic variations in a variable font; or they may be implemented in the form of discrete, static instance fonts. For example, weight is a variable axis in the Skia variable font, but is also reflected as an axis of design variation in the Arial family with static instances that include “Arial Regular” and “Arial Bold”.

The 'STAT' table provides richer information about the axes of variation and individual styles within a family, as well as relationships between the styles. This information may be used for different purposes, including:

- It provides a way to accommodate rich typographic families with many styles in legacy applications that use a more limited understanding of font families. This is especially relevant for variable fonts.

- By providing more information about styles within a family, it can support alternate designs for font-selection user interfaces that may be useful with rich typographic families.

- By providing more information about relationships between styles within a family, it can support application logic for selection of styles in particular application conditions.

Note that, in regard to the first purpose of bridging between different notions of what is included in a font family, this has been handled in the past using separate family/subfamily name pairs in the 'name' table: name IDs 1 and 2 versus name IDs 16 and 17 versus name IDs 21 and 22. The 'STAT' provides a way to bridge between different family models without proliferation of additional family/subfamily pairs in the 'name' table.

The information in the 'STAT' table can characterize a font in relation to the entire family to which it belongs, not just the font in isolation. For example, the face name “Arial Bold” reflects that the Arial family includes weight as an axis of design variation, but not that the family also includes an italic axis of design variation. The 'STAT' table provides a way to indicate in the Arial Bold font that the family includes an italic axis variation and that this font has a particular value—non-italic—in regard to that axis.

A style attributes table is required in all variable fonts. For a general overview of Font Variations, see subclause 7.1.

The style attributes table is also recommended for all new, non-variable fonts, especially if fonts have style attributes in axes other than weight, width, or slope.

The information provided in the 'STAT' table includes string labels for specific style attributes. For example, “Bold” and “Condensed” as individual style attribute labels within a “Condensed Bold” font. These may be used in user interfaces but are not intended to supersede subfamily names provided in the 'name' table or in the 'fvar' table of variable fonts.

#### Alternate Font Family Models

From a design perspective, a typographically-rich font family can potentially include a large number of style variants involving many axes of design variation. Some variable fonts, for example, have a dozen or more axes. Modern applications should be designed to accommodate families with such variety.

Many applications, however, are constrained by legacy assumptions regarding the faces or axes of design variation that can be included within a font family. Some applications use a font family model that allows only weight, width or slope (italic/oblique/slant) as design axes (a “WWS” model). Some applications use an even more limited model that allows for only regular, bold, italic, or bold italic fonts within a family (the “R/B/I/BI” model).

In applications that use a limited font family model, any style distinctions within a font’s typographic family that do not fit within the application’s family model must be treated as a separate family. For example, if an application uses the R/B/I/BI model, the Calibri Light font must be treated as the “Regular” style in a “Calibri Light” family. Thus, the typographic family and subfamily names need to be projected into the legacy font family model with alternate family and subfamily names.

In non-variable fonts, alternate family and subfamily names are accommodated using different name ID pairs in the 'name' table:

- Name IDs 16 (Typographic Family) and 17 (Typographic Subfamily) support an unrestricted family model.

- Name IDs 1 (Font Family) and 2 (Font Subfamily) support the R/B/I/BI family model.

- Name IDs 21 (WWS Family) and 22 (WWS Subfamily) support the WWS family model.

So, for example, the Calibri Light font has “Calibri” for name ID 16 and “Light” for name ID 17; but it also includes “Calibri Light” as name ID 1 (family name), and “Regular” as name ID 2 (subfamily name).

In variable fonts, however, this is not possible. The font includes strings for name IDs 16 and 17, with the latter used for the default instance; and the 'fvar' table provides alternate subfamily strings for named instances that are the equivalents to name ID 17 for each named instance. For legacy font family models, however, the ‘fvar’ table does not provide named instance strings for name IDs 1 and 2, or for name IDs 21 and 22. Even if a variable font only has a single axis, such as weight, instance names such as “Light” or “Extrabold” must be handled in older applications by some means other than alternate name IDs.

The 'STAT' accommodates this through axis value tables. An axis value table can provide a string label for a specific axis value on any axis that is relevant for the font or font family. (For special cases, an axis value table can also provide a label for a combination of values from a set of relevant axes.) These labels, together with the typographic family name (name ID 16), can be used to compose alternate names dynamically to fit the requirements of different font family models.

For example, consider the Sitka font family, and the Sitka Display Bold font. The family has optical size and weight (and also italic) axes of design variation. The optical size axis is not supported in either the WWS or the R/B/I/BI family models. The Sitka Display Bold font can include axis value tables that provide the label “Display” for that value on the optical size axis, the label “Bold” for that value on the weight axis. An application can use these to dynamically compose an alternate family name “Sitka Display” and alternate subfamily name “Bold” to fit the requirements of these alternate font family models.

In a similar way, a variable font with any number of variation axes can provide a complete set of labels for the axis values that pertain to the font’s named instances. Using these, a compatibility layer can project names for any of the named instances into a legacy font family model, allowing all of the variable font’s named instances to work in older applications.

The 'STAT' table also provides additional information regarding the axes of design variation. This includes an axis ordering value that can be used to influence the relative ordering of axis value labels when multiple labels need to be combined in a dynamically generated name. See 7.3.7.3 for more information.

NOTE	When generating family and subfamily names for compatibility with the R/B/I/BI family model, then any subfamily name elements that are not “Regular”, “Bold”, “Italic” or “Oblique” will be moved into the family name. For example, given a family name “Selawik” and a subfamily name “Condensed Bold”, this will be transformed into the R/B/I/BI model as family name “Selawik Condensed” and subfamily name “Bold”. Similarly, when names are generated for compatibility with the WWS family model, any subfamily name elements that do not represent a weight value, a width value, or italic or oblique will be moved into family names. For this reason, it is recommended that width, weight and italic/oblique labels always be placed last in a subfamily name, to minimize differences in how names will appear in different applications that use different family models.

#### Style attributes header

The style attributes table, version 1.2, is organized as follows:

Style attributes header

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the style attributes table – set to 1. |
| uint16 | minorVersion | Minor version number of the style attributes table – set to 2. |
| uint16 | designAxisSize | The size in bytes of each axis record. |
| uint16 | designAxisCount | The number of design axis records. In a font with an ‘fvar’ table, this value shall be greater than or equal to the axisCount value in the ‘fvar’ table. In all fonts, shall be greater than zero if axisValueCount is greater than zero. |
| Offset32 | designAxesOffset | Offset in bytes from the beginning of the 'STAT' table to the start of the design axes value offsets array. If axisValueCount is zero, set to zero; if axisValueCount is greater than zero, shall be greater than zero. |
| uint16 | axisValueCount | The number of axis value tables. |
| Offset32 | offsetToAxisValueOffsets | Offset in bytes from the beginning of the 'STAT' table to the start of the design axes value offsets array. |
| uint16 | elidedFallbackNameID | Name ID used as fallback when projection of names into a particular font model produces a subfamily name containing only elidable elements. |

In version 1.0 of the 'STAT' table, the elidedFallbackNameId field was not included. Use of version 1.0 is deprecated. Version 1.1 adds the elidedFallbackNameId field. Version 1.2 adds support for the format 4 axis value table; otherwise, version 1.2 and version 1.1 are the same.

The elidedFallbackNameId field provides a name that can be used when composing a name if all of the axis-value names are elidable. For example, "Normal" weight and "Roman" slant may both be marked as elidable axis-value names, and so a composed name for normal weight and Roman slant may result in an empty string. The elidedFallbackNameId is used to provide an alternative name ID to use in this case, such as "Regular". In many fonts, this may reference name ID 17 or name ID 2.

The header is followed by the design axes array, and the axis value offsets array. The offsets to these arrays are provided by offset fields in the header. These arrays may be contiguous in a font; parsers should, nevertheless, use the offset fields to locate each array.

| AxisRecord | designAxes [designAxisCount] | The design-axes array. |
| --- | --- | --- |
| Offset16 | axisValueOffsets [axisValueCount] | Array of offsets to axis value tables, in bytes from the start of the axis value offsets array. |

The designAxisSize field indicates the size of each axis record. Future minor-version updates of the 'STAT' table may define compatible extensions to the axis record format with additional fields. Implementations must use the designAxisSize field to determine the start of each record.

#### Axis records

The axis record provides information about a single design axis.

AxisRecord

| Type | Name | Description |
| --- | --- | --- |
| Tag | axisTag | A tag identifying the axis of design variation. |
| uint16 | axisNameID | The name ID for entries in the 'name' table that provide a display string for this axis. |
| uint16 | axisOrdering | A value that applications can use to determine primary sorting of face names, or for ordering of labels when composing family or face names. |

Each axis record has a tag designating the axis. Tag values must follow the rules for tags described in the "Design-variation axis tags registry"(7.2.4). No two axis records should have the same axisTag value.

The axisNameID field provides a name ID that can be used to obtain strings from the 'name' table that can be used to refer to the axis in application user interfaces. A defined name ID can be used if strings for that name ID have the appropriate string for an axis name. Otherwise, name IDs must be greater than 255 and less than 32768.

In a variable font, there must be an axis record for every axis defined in the 'fvar' table, and it shall use the same name ID used in the 'fvar' table. The order of axis records in the 'STAT' table is arbitrary and does not need to match the order of records in the 'fvar' table.

If a variable font is part of a family that has additional axes of design variation but that are not implemented as dynamic-variation axes in the 'fvar' table, records for these other design variation axes should also be included.

A non-variable font should include axis records for any axes that are relevant for the font or the family of which it is a member, especially if axes other than weight, width and slope are used.

For every axis declared using an axis record, the axis should be a variation axis defined in an 'fvar' table, or else it should be reflected in the font’s sub-family name (name ID 17 or name ID 2) except in the case that the font’s value on that axis is a “normal” value that is suppressed from the sub-family name.

The axisOrdering field can also be used by applications in the design of user interfaces. For example, rather than listing the styles within a family in an arbitrary order, the application could use the axis ordering information to sort the styles or present them in groups in a more organized manner. Using this field, a font developer can indicate a recommendation for how the styles within the family will be presented; for example, whether they are sorted or grouped first by weight, then by width, or vice versa.

To ensure consistency in how face names are presented to users, the axis ordering given in axis records should be consistent across different fonts within a family, and with the ordering of axis-value labels used in the typographic subfamily (name ID 17). In a variable font, the axis ordering should also be consistent with the ordering of axis-value labels used in the strings referenced by named instances defined in the 'fvar' table.

Different fonts belonging to the same family should have matching axis records. If a set of fonts for a family are released, and then at some later time the family is extended with additional fonts using new axes of variation, the previously-shipped fonts would not necessarily need to be updated with the additional axis records; because axis records give characteristics for a font family, not just a single font within the family, axis records in the newer fonts could provide axis details for the extended family.

#### Axis value tables

Axis value tables provide details regarding a specific style-attribute value on some specific axis of design variation, or a combination of design-variation axis values. They also provide the relationship from those values to labels used as elements in subfamily names. This information can be useful for presenting fonts in application user interfaces. It is also used by platforms to provide compatibility between rich typographic families with a wide range of styles and older applications that use legacy font family models. (See 7.3.7.1 for more information.)

In a variable font, it is strongly recommended that axis value tables be included for every element of typographic subfamily names for all of the named instances defined in the 'fvar' table.

NOTE 1	Some applications could rely on these axis value tables to compose subfamily names for named instances. Unexpected user interface behavior could result if a complete set of axis value tables is not included in the font.

Additional axis value tables may be included for name elements that do not appear in any named instances; these may be used by applications in user interfaces or for other purposes, but are not required for compatibility with legacy font family models. Some variable fonts may include axes that are not reflected in subfamily names for any named instances — that is, variants along these axes are selectable only by means of numeric axis values. In such cases, there is no requirement to assign names or to create axis value tables for values on these axes.

In many cases, a name element will be associated with a particular value on a single axis. For example, “Bold” representing a specific weight-axis value; or “Condensed” representing a specific width value. Such name elements are assumed to be combinable with name elements associated with values on other axes, as in an instance name “Bold Condensed”. Name elements of this type are referred to as analytic names.

In other cases, a name element may be associated with a particular combination of values on multiple axes, and not be amenable to analysis into simpler, independent elements. For example, a variable font for lettering might use several custom axes to provide different stroke or swash-terminal modifications, and named instances may be defined for certain combinations of values for these axes with non-decomposable names for those combinations; for example, “Florid” for a particular combination of stroke- and termination-axes values. Name elements of this type are referred to as non-analytic. Note that a font with non-analytic names might also use an axis such as weight that uses analytic names, leading to some named instances that combine non-analytic and analytic elements, such as “Florid Bold” and “Florid Semibold”.

NOTE 2	If an application transforms family and subfamily names to be compatible with an R/B/I/BI family model, then any subfamily name elements that are not “Regular”, “Bold”, “Italic” or “Oblique” will be moved into a family name. For example, given a family name “Selawik” and a subfamily name “Condensed Bold”, this will be transformed into the R/B/I/BI model as family name “Selawik Condensed” and subfamily name “Bold”. (“Selawik” and “Selawik Condensed” will be treated as separate families in the R/B/I/BI model.) Similarly, when names are transformed to the WWS family model, any subfamily name elements that do not represent a weight value, a width value, or italic or oblique will be moved into family names. For this reason, it is recommended that width, weight and italic/oblique descriptors always be placed last in a subfamily name, to minimize differences in how names will appear in different applications that use different family models.

Note that a variable font could have some distinguishing, subfamily attributes that are static, not implemented as a variation, but that are relevant in relation to the complete typographic family. For example, a weight-variation font may have a paired, italic weight-variation font. Axis value tables (as well as axis records) should also be provided for any such distinctions that are relevant within a family.

Axis value tables are particularly important for variable fonts, but can also be used in non-variable fonts. When used in non-variable fonts, axis value tables for any given value should be implemented consistently across fonts in the family. In particular, two different fonts within a family may share certain style attributes in common, and axis value tables for these values should be implemented consistently. For example, Bold Condensed and Bold Semi-Condensed fonts both have the same weight attribute, Bold, and should have matching axis value tables for Bold. If they are not consistent, some applications could exhibit unexpected behavior in font-related operations, such as how the fonts are presented in user interfaces, or how the fonts are chosen in fallback or other font-selection operations.

Four different axis value table formats are defined. Formats 1, 2 and 3 are appropriate for single-axis values associated with analytic name elements. Format 4 is used for multi-axis value combinations associated with non-analytic name elements. Additional formats may be supported in the future, with a minor-version update of the 'STAT' table.

NOTE 3	Use of format 1, format 2, or format 3 axis value tables is especially recommended for values on 'wght' and 'wdth' axes, and also for “Italic” (value 1.0 on the 'ital' axis) or “Oblique” (a non-zero value on the 'slnt' axis) variants.

NOTE 4	Use of format 4 axis value tables is especially recommended for non-analytic subfamily names and the corresponding axis-value combinations in static-font families, or for non-analytic subfamily names in variable fonts that also involve 'wght' or 'wdth' axes, or “Italic” or “Oblique” variants.

For any format of axis value table, the first field is read to determine the format. If the format is not recognized, then the axis value table may be ignored.

Each format includes a valueNameID field, which references a display string to be associated with the numeric axis value or combination of axis values. Defined name IDs, such as name IDs 2, 17 or 22, can be used if those have the appropriate string for an axis value. Otherwise, name IDs shall be greater than 255 and less than 32768.

The different formats all include fields with numeric axis values. These values use the same scale as is used for the given axis in the 'fvar' table.

In some cases, an attribute using a new design axis could be introduced into a family after other fonts have been released, with the new attribute not anticipated in any way in the initial fonts. For example, a font designer might initially create Regular, Bold and Italic variants of a design, and then later add width variants such as Condensed. In that case, the initial fonts might not have identified that they represent the “Normal” attribute on the width scale. The newer fonts should include axis value tables that describe the earlier fonts in relation to the new design axis. A flag is defined to designate such axis value tables; this is described in detail below.

No two tables should provide information for the same combination of axis values:

- There should not be a format 1 table with axisIndex and value that match the axisIndex and value of a separate format 3 table.

- There should not be a format 1 table with the same axisIndex as a separate format 2 table if the format 1 value is greater than the rangeMinValue and less than the rangeMaxValue of the format 2 table. The format 1 value may be the same as the minimum or maximum of the format 2 range, but the format 2 nominalValue should be different.

- Since format 2 and format 3 each provide additional and distinct information associated with an axis value, a font may contain a format 2 table and a format 3 table with the same axisIndex where the format 2 nominal value is the same as the format 3 value. The flags and valueNameID of the two tables shall be the same.

- A format 4 table may specify an axis value that is also specified in another table, but only if there is some other difference in the combination of axis values specified by those two tables.

Since use of format 4 tables is recommended only for multi-axis value combinations associated with a non-analytic name element, there should not be cases of a format 4 table with a single axis value that matches the value or nominal value of another axis value table.

Axis value table, format 1

Axis value table format 1 has the following structure.AxisValueFormat1:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format identifier – set to 1 |
| uint16 | axisIndex | Zero-base index into the axis record array identifying the axis of design variation to which the axis value table applies. . Shall be less than designAxisCount. |
| uint16 | flags | Flags – see below for details. |
| uint16 | valueNameID | The name ID for entries in the 'name' table that provide a display string for this attribute value. |
| Fixed | value | A numeric value for this attribute value. |

A format 1 table is used simply to associate a specific axis value with a name.

Axis value table, format 2

Axis value table format 2 has the following structure.AxisValueFormat2:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format identifier – set to 2 |
| uint16 | axisIndex | Zero-base index into the axis record array identifying the axis of design variation to which the axis value table applies. Shall be less than designAxisCount. |
| uint16 | flags | Flags – see below for details. |
| uint16 | valueNameID | The name ID for entries in the 'name' table that provide a display string for this attribute value. |
| Fixed | nominalValue | A nominal numeric value for this attribute value. |
| Fixed | rangeMinValue | The minimum value for a range associated with the specified name ID. |
| Fixed | rangeMaxValue | The maximum value for a range associated with the specified name ID. |

A format 2 table can be used if a given name is associated with a particular axis value, but is also associated with a range of values. For example, in a family that supports optical size variations, “Subhead” may be used in relation to a range of sizes. The rangeMinValue and rangeMaxValue fields are used to define that range. In a variable font, a named instance has specific coordinates for each axis. The nominalValue field allows some specific, nominal value to be associated with a name, to align with the named instances defined in the font variations table, while the rangeMinValue and rangeMaxValue fields allow the same name also to be associated with a range of axis values.

Some design axes may be open ended, having an effective minimum value of negative infinity, or an effective maximum value of positive infinity. To represent an effective minimum of negative infinity, set rangeMinValue to 0x80000000. To represent an effective maximum of positive infinity, set rangeMaxValue to 0x7FFFFFFF.

The range specification of a format 2 table is inclusive: both the minimum and maximum values are included within the range. Two tables for a given axis may have ranges that touch (the rangeMaxValue of one range is the rangeMinValue of the other), but ranges should not overlap more than that. In the case of two ranges that touch:

- At most one of the ranges should have the nominalValue set to the axis value at which the ranges touch.

- When the requested axis value is the value at which the ranges touch, the higher range shall be used unless the nominalValue for the lower range is set to the value at which the ranges touch, and the nominalValue for the higher range is greater than that value.

Similar behavior is used if the value of a format 1 or format 3 table touches the range of a format 2 table:

- If the value of a format 1 or format 3 table is equal to the rangeMaxValue of a format 2 table, the format 1 or format 3 table is used.

- If the value of a format 1 or format 3 table is equal to the rangeMinValue of a format 2 table, the format 1 or format 3 table is used except if the nominalValue of the format 2 table is also equal to the rangeMinValue.

If two format 2 tables have ranges for the same axis with non-zero overlap, then the following guidance is recommended for applications:

- If two tables have identical ranges, the application should consistently choose one and ignore the other, by its own criteria.

- Else, if the range of one table is entirely contained within the range of other, then the table with the smaller range should be ignored.

- Else, for axis values within the overlapping range, use the table with the higher range (both rangeMinValue and rangeMaxValue are higher).

Axis value table, format 3

Axis value table format 3 has the following structure.AxisValueFormat3:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format identifier – set to 3 |
| uint16 | axisIndex | Zero-base index into the axis record array identifying the axis of design variation to which the axis value table applies. Shall be less than designAxisCount. |
| uint16 | flags | Flags – see below for details. |
| uint16 | valueNameID | The name ID for entries in the 'name' table that provide a display string for this attribute value. |
| Fixed | value | A numeric value for this attribute value. |
| Fixed | linkedValue | The numeric value for a style-linked mapping from this value. |

A format 3 table can be used to indicate another value on the same axis that is to be treated as a style-linked counterpart to the current value. This is primarily intended for “bold” style linking on a weight axis. These mappings may be used in applications to determine which style within a family should be selected when a user selects a “Bold” formatting option. A mapping is defined from a “non-bold” value to its “bold” counterpart. It is not necessary to provide a “bold” mapping for every weight value; mappings should be provided for lighter weights, but heavier weights (typically, semibold or above) would already be considered “bold” and would not require a “bold” mapping.

NOTE	Applications are not required to use these style-linked mappings when implementing text formatting user interfaces. This data can be provided in a font for the benefit of applications that choose to do so. If a given application does not apply such style mappings for the given axis, then the linkedValue field is ignored.

Axis value table, format 4

Axis value table format 4 has the following structure.

AxisValueFormat4:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | format | Format identifier – set to 4 |
| uint16 | axisCount | The total number of axes contributing to this axis-values combination. |
| uint16 | flags | Flags – see below for details. |
| uint16 | valueNameID | The name ID for entries in the 'name' table that provide a display string for this combination of axis values. |
| AxisValue | axisValues[axisCount] | Array of AxisValue records that provide the combination of axis values, one for each contributing axis. |

The axisCount value should be greater than 1.

The axisValues array uses AxisValue records, which have the following format.

AxisValue record:

| Type | Name | Description |
| --- | --- | --- |
| uint16 | axisIndex | Zero-base index into the axis record array identifying the axis to which this value applies. Must be less than designAxisCount. |
| Fixed | value | A numeric value for this attribute value. |

Each AxisValue record shall have a different axisIndex value. The records can be in any order.

When searching for an axis value table to match a particular combination of values, if two format 4 tables are found to be a partial match for that combination of values, the table that matches a greater number of values (the most specific match) should be used. If two matching format 4 tables are equally specific—the same number of values for a different set of axes—then the first matching table should be used.

Similarly, if a format 1, format 2 or format 3 table has a (nominal) value used in a format 4 table that also has values for other axes, the format 4 table, being the more specific match, should be used.

Because a format 4 table combines values on multiple axes, there can be ambiguity about axis ordering. This could arise when dynamically composing names using the labels provided by axis value tables, or in other situations in which the axisOrdering values of axis records are used. For a format 4 table, the axisOrdering value assumed should be the lowest axisOrdering value for the axes referenced by the format 4 table.

Flags

The following axis value table flags are defined:

| Mask | Name | Description |
| --- | --- | --- |
| 0x0001 | OLDER_SIBLING_FONT_ATTRIBUTE | If set, this axis value table provides axis value information that is applicable to other fonts within the same font family. This is used if the other fonts were released earlier and did not include information about values for some axis. If newer versions of the other fonts include the information themselves and are present, then this table is ignored. |
| 0x0002 | ELIDABLE_AXIS_VALUE_NAME | If set, it indicates that the axis value represents the “normal” value for the axis and may be omitted when composing name strings. |
| 0xFFFC | Reserved | Reserved for future use — set to zero. |

When the OLDER_SIBLING_FONT_ATTRIBUTE flag is used, implementations may use the information provided to determine behavior associated with a different font in the same family. If a previously-released family is extended with fonts for style variations from a new axis of design variation, then all of them should include a OLDER_SIBLING_FONT_ATTRIBUTE table for the “normal” value of earlier fonts. The values in the different fonts should match; if they do not, application behavior may be unpredictable.

NOTE 1	When the OLDER_SIBLING_FONT_ATTRIBUTE flag is set, that axis value table is intended to provide default information about other fonts in the same family, but not about the font in which that axis value table is contained. The font should contain different axis value tables that do not use this flag to make declarations about itself.

The ELIDABLE_AXIS_VALUE_NAME flag can be used to designate a “normal” value for an axis that should not normally appear in a face name. For example, the designer may prefer that face names not include “Normal” width or “Regular” weight. If this flag is set, applications are permitted to omit these labels from face names, though they may also include them in certain scenarios.

NOTE 2	Fonts should provide axis value tables for “normal” axis values even if they should not normally be reflected in face names.

NOTE 3	If a font or a variable-font instance is selected for which all axis values have the ELIDABLE_AXIS_VALUE_NAME flag set, then applications may keep the name for the weight axis, if present, to use as a constructed subfamily name, with names for all other axis values omitted.

When the OLDER_SIBLING_FONT_ATTRIBUTE flag is set, this will typically be providing information regarding the “normal” value on some newly-introduced axis. In this case, the ELIDABLE_AXIS_VALUE_NAME flag may also be set, as desired. When applied to the earlier fonts, those likely would not have included any labels for the new axis, and so the effects of the ELIDABLE_AXIS_VALUE_NAME flag are implicitly assumed.

If multiple axis value tables have the same axis index, then one of the following should be true:

- The font is a variable font, and the axis is defined in the font variations table as a variation axis.

- The OLDER_SIBLING_FONT_ATTRIBUTE flag is set in one of the tables.

Two different fonts within a family may share certain style attributes in common. For example, Bold Condensed and Bold Semi Condensed fonts both have the same weight attribute, Bold. Axis value tables for particular values should be implemented consistently across a family. If they are not consistent, applications may exhibit unpredictable behaviors.

#### Examples

The following examples illustrate data provided by a style attributes table for various font scenarios. Except where indicated otherwise, axis value data shown could be implemented using AxisValueTable format 1 or, if a linked value is included, format 3.

Example 1: Family with different weight variants using non-variable fonts

Suppose a font family has Regular, Bold and Heavy weight variants. These fonts would have matching axis records:

| Axis tag | Axis name | Axis ordering |
| --- | --- | --- |
| ‘wght’ | Weight | 0 |

The three fonts would have axis value data as follows:

| Font | Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- | --- |
| Font 1 | ‘wght’ | 400 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 700 |
| Font 2 | ‘wght’ | 700 | Bold |  |  |
| Font 3 | ‘wght’ | 900 | Heavy |  |  |

Example 2: Family using non-variable fonts for different weight values plus italic

Suppose the font family from example 1 also has italic variants. The fonts would have matching axis records reflecting weight and italic axes:

| Axis tag | Axis name | Axis ordering |
| --- | --- | --- |
| ‘wght’ | Weight | 0 |
| ‘ital’ | Italic | 1 |

Each of the three non-italic fonts would include an additional axis value record to reflect the non-italic attribute. The six fonts would have data as follows:

| Font | Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- | --- |
| Font 1 | ‘wght’ | 400 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 700 |
| Font 1 | ‘ital’ | 0 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 1 |
| Font 2 | ‘wght’ | 700 | Bold |  |  |
| Font 2 | ‘ital’ | 0 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 1 |
| Font 3 | ‘wght’ | 900 | Heavy |  |  |
| Font 3 | ‘ital’ | 0 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 1 |
| Font 4 | ‘wght’ | 400 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 700 |
| Font 4 | ‘ital’ | 1 | Italic |  |  |
| Font 5 | ‘wght’ | 700 | Bold |  |  |
| Font 5 | ‘ital’ | 1 | Italic |  |  |
| Font 6 | ‘wght’ | 900 | Heavy |  |  |
| Font 6 | ‘ital’ | 1 | Italic |  |  |

Example 3: Family using non-variable fonts with weight and italic variants, later extended to add width variants

Suppose the font family from example 2 is later extended with different width variants. The new fonts in the family would include matching axis records reflecting three axes:

| Axis tag | Axis name | Axis ordering |
| --- | --- | --- |
| ‘wdth’ | Width | 0 |
| ‘wght’ | Weight | 1 |
| ‘ital’ | Italic | 2 |

Newer versions of the initially-released fonts would also include the additional axis record. When newer fonts co-exist with the original version of the earlier fonts, the ordering from the more-complete axis records in the newer fonts is used.

To allow for situations in which one of the newer fonts co-exists with the older fonts, which did not reference the width axis, the newer fonts should each include an axis record to describe the “Normal” width, which is inferred onto the earlier fonts.

| Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- |
| ‘wdth’ | 100 | Normal | OLDER_SIBLING_FONT_ATTRIBUTE |  |

Example 4: A weight/width variable font

Consider a family comprised of a single variation font with weight and width variations. This font would have axis records for the two variation axes:

| Axis tag | Axis name | Axis ordering |
| --- | --- | --- |
| ‘wght’ | Weight | 0 |
| ‘wdth’ | Width | 1 |

Suppose the variable font has 6 named instances that correspond to three different weights for each of two widths. The style attributes table should include axis value tables for at least those three weights and those two widths, but could also include tables for other weight or width values. The font can include the following axis value tables:

| Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- |
| ‘wght’ | 300 | Light |  | linkedValue = 600 |
| ‘wght’ | 400 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 700 |
| ‘wght’ | 600 | Semibold |  |  |
| ‘wght’ | 700 | Bold |  |  |
| ‘wght’ | 900 | Black |  |  |
| ‘wdth’ | 62.5 | Extra-Condensed |  |  |
| ‘wdth’ | 75 | Condensed |  |  |
| ‘wdth’ | 100 | Normal | ELIDABLE_AXIS_VALUE_NAME |  |
| ‘wdth’ | 125 | Expanded |  |  |
| ‘wdth’ | 150 | Extra-Expanded |  |  |

Example 5: A family comprised of a non-italic variable font plus an italic variable font

Consider a family comprised of a non-italic, weight/width variation font plus a corresponding italic, weight/width variation font. Each font would have axis records for three axes:

| Axis tag | Axis name | Axis ordering |
| --- | --- | --- |
| ‘wght’ | Weight | 0 |
| ‘wdth’ | Width | 1 |
| ‘ital’ | Italic | 2 |

In addition to axis value tables for various weight or width values, the first font would also include an axis value table to reflect the non-italic attribute:

| Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- |
| ‘ital’ | 0 | Normal | ELIDABLE_AXIS_VALUE_NAME |  |

The second font would include an axis value table to reflect the italic attribute, in addition to the axis value tables for various weight and width values:

| Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- |
| ‘ital’ | 1 | Italic |  |  |

The pattern in this example can be applied to other cases involving a family with style variations implemented using a combination of font variation mechanisms plus static, non-variation designs. The axis records in each font would span both variation and non-variation axes. Axis value tables in each font would include multiple values for axes defined in the ‘fvar’ table, plus single tables for the relevant attribute values of other axes.

Example 6: A variable font with non-analytic subfamily names associated with multiple axis values

Consider a variable font that uses several custom axes, 'TRM1', 'TRM2', 'STK1', 'STK2', and also the registered 'wght' axis. Suppose that this font has named instances “Florid” and “Jagged” that involve particular combinations of values for the custom axes; and additional named instances that correspond to those two named instances but with other 'wght' values: “Florid Bold”, “Florid Semibold”, etc. The font would have axis value tables for 'wght' values, with data such as the following:

| Axis tag | Value | Name string | Flag | Other data |
| --- | --- | --- | --- | --- |
| ‘wght’ | 400 | Regular | ELIDABLE_AXIS_VALUE_NAME | linkedValue = 700 |
| ‘wght’ | 700 | Bold |  |  |
| ‘wght’ | 900 | Heavy |  |  |

The font would also have format 4 axis value tables corresponding to “Florid” and “Jagged”, with data such as the following:

| Name string | AxisValue records |
| --- | --- |
| Florid | 'TRM1' = 250 'TRM2' = 1000 'STK1' = 550 'STK2' = 0 |
| Jagged | 'TRM1' = 900 'TRM2' = 450 'STK1' = 0 'STK2' = 310 |

### VVAR—Vertical metrics variations table

The 'VVAR' table is used in variable fonts to provide variations for vertical glyph metric values. This can be used to provide variation data for advance heights in the 'vmtx' table. In fonts with TrueType outlines, it can also be used to provide variation data for top and bottom side bearings obtained from the 'vmtx' table and glyph bounding box. In addition, it can also be used in fonts that have CFF2 outlines to provide vertical-origin variation data.

For a general overview of OFF Font Variations and terminology related to variations, see subclause 7.1.

In a font with TrueType outlines, the rasterizer will generate “phantom” points that represent left, right, top and bottom side bearings. (See "Instructing TrueType Glyphs" [24] for more background on phantom points.) In a TrueType variable font, the glyph variations ('gvar') table will include variation data for the phantom points of each glyph, allowing glyph metrics to be interpolated for different variation instances as part of deriving the interpolated glyph outlines. For this reason, the 'VVAR' table is not required in variable fonts that have TrueType outlines. For text-layout operations that require glyph metrics but not actual glyph outlines, however, there can be significant performance benefits by being able to obtain adjusted glyph metrics for an instance without needing to interpolate glyph outlines. For this reason, it is recommended that a 'VVAR' table be included in variable fonts that have TrueType outlines and that support vertical layout.

The CFF2 rasterizer does not generate phantom points as in the TrueType rasterizer. For this reason, an 'VVAR' table is required to handle any variation in vertical glyph metrics in a variable font with CFF2 outlines.

The format and processing of the 'VVAR' table is analogous to the horizontal metrics variations ('HVAR') table.

#### Related and co-requisite tables

The 'VVAR' table is used only in variable fonts that support vertical layout. It must be used in combination with a vertical metrics ('vmtx') table, and also in combination with a font variations ('fvar') table, and other required or optional tables used in variable fonts. See Variation Data Tables and Miscellaneous Requirements in the Font Variations Overview chapter for general information.

For variable fonts that have TrueType outlines and that support vertical layout, the 'VVAR' table is optional but recommended. For variable fonts that have CFF2 outlines and that support vertical layout, the 'VVAR' table is required if there is any variation in glyph advance heights across the variation space.

NOTE	The 'VDMX' table is not used in variable fonts.

#### Table formats

The vertical metrics variations table has the following format:

Vertical metrics variations table

| Type | Name | Description |
| --- | --- | --- |
| uint16 | majorVersion | Major version number of the vertical metrics variations table — set to 1. |
| uint16 | minorVersion | Minor version number of the vertical metrics variations table — set to 0. |
| Offset32 | itemVariationStoreOffset | Offset in bytes from the start of this table to the item variation store table. |
| Offset32 | advanceHeightMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for advance heights (may be NULL). |
| Offset32 | tsbMappingOffet | Offset in bytes from the start of this table to the delta-set index mapping for top side bearings (may be NULL). |
| Offset32 | bsbMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for bottom side bearings (may be NULL). |
| Offset32 | vOrgMappingOffset | Offset in bytes from the start of this table to the delta-set index mapping for Y coordinates of vertical origins (may be NULL). |

The item variation store table is documented in subclause 7.2.

Mapping subtables are represented using a DeltaSetIndexMap table; see 7.2.3.1 for the definition of this table. In the VVAR table, only format 0 of the DeltaSetIndexMap is used. Mapping subtables are optional. If a given mapping subtable is not provided, the offset is set to NULL.

Variation data for advance heights is required. A delta-set index mapping subtable for advance heights can be provided, but is optional. If a mapping table is not provided, glyph indices are used as implicit delta-set indices, as in the HVAR table.

Variation data for side bearings are optional. If included, mapping subtables are required to provide the delta-set index for each glyph.

Mappings and variation data for vertical origins are not used in fonts with TrueType outlines, but can be included in variable fonts with CFF 2 outlines if there is variability in the Y coordinates of glyph vertical origins, the default values of which are recorded in the VORG table. A mapping subtable is required for vertical-origin variation data.

# Recommendations for OFF fonts

This clause outlines recommendations for creating OFF fonts.

## Byte ordering

All OFF fonts use Motorola-style byte ordering (Big Endian).

## Mixing outline formats

It is not recommended to mix outline formats within a single font. Choose the format that meets your feature requirements.

## Filenames

OFF fonts may have the extension .OTF, .TTF, .OTC or .TTC, depending on the kind of outlines in the font and the desired backward compatibility.

- A file containing a single font resource with TrueType outlines should have either .OTF or .TTF as the extension. The choice between .OTF and .TTF may depend on the desire for backward compatibility on older systems or with previous versions of the font.

- A file containing a single font resource with only CFF outline data (no TrueType outlines) should have an .OTF extension.

- A font collection file (one that contains multiple font resources) should have either .OTC or .TTC as the extension, regardless of whether or not layout tables are present in any of the font resources, and regardless of the kind of outline data used. The .TTC extension may be used for font collection files containing font resources that use CFF outline data if needed for backward compatibility with older software that was not aware of the .OTC extension.

- A variable font that uses OFF font variations mechanisms and associated tables should use the extensions .OTF, .TTF, .OTC or .TTC following the above guidance. If there is a need to provide some indication within a filename that the file contains a variable font, a recommended convention is to append “VF” (with a preceding delimiter character) at the end of the file name (before the extension) — e.g., “Selawik-VF.ttf”.

In all cases, software must determine the kind of outlines present in a font not from the filename extension but from the contents of the file.

## Table alignment and length

All tables should be aligned to begin at offsets which are multiples of four bytes. While this is not required by the TrueType rasterizer, it does prevent ambiguous checksum calculations and greatly speeds table access on some processors.

All tables should be recorded in the table directory with their actual length. To ensure that checksums are calculated correctly, it is suggested that tables begin on 32-bit boundaries. Any extra space after a table (and before the next 32-bit boundary) should be padded with zeros.

## Glyph 0: the .notdef glyph

Glyph 0 must be assigned to a .notdef glyph. The .notdef glyph is very important for providing the user feedback that a glyph is not found in the font. This glyph should not be left without an outline as the user will only see what looks like a space if a glyph is missing and not be aware of the active font's limitation.

It is recommended that the shape of Glyph ID 0 be either an empty rectangle, a rectangle with a question mark inside of it, or a rectangle with an 'X'. Creative shapes, like swirls or other symbols, may not be recognized by users as indicating that a glyph is missing from the font and is not being displayed at that location.

## 'BASE' table

The 'BASE' table allows for different scripts in the font to specify different values for the same baseline tag. This situation could arise, for example, when a developer makes a multi-script font by combining glyphs from multiple fonts that use different baseline systems.

However, glyphs from different scripts in this font may not appear correctly aligned relative to each other when used with applications that either don't support the 'BASE' table or that support it but assume that a particular baseline will not vary across scripts. Furthermore, it is not always possible to determine the script of every glyph in the font, some "weakly-scripted" characters such as punctuation may be used in several scripts, and some glyphs such as ornaments may not have a script at all.

Thus, it is strongly recommended that developers construct their fonts so that all scripts in the 'BASE' table record the same value for a particular baseline if they want their fonts to work as expected in the above situations.

If baselines vary by script, then it is strongly recommended that the vendor add a DFLT script entry to the BASE table, which can be used if the script requested by the client is not matched or if the client does not or can not determine the script.

## 'cmap' table

When building a font for Windows, a 'cmap' subtable for platform ID 3 should be included. When building a Unicode font, encoding ID 1 should be used for this subtable. (This subtable must use format 4.) When building a symbol font for Windows, encoding ID 0 should be used for this subtable.

When building a font to support Unicode supplementary characters (U+10000 to U+10FFFF)), include a 'cmap' subtable for platform ID 3, encoding ID 10. (This subtable must use format 12.) To provide compatibility with older software, a subtable for platform 3, encoding ID 1 should also be included. Depending on application support and the content of text being displayed, either the 3/1/4 or 3/10/12 subtable may be used. Therefore, glyph mappings for characters in the range U+0000 to U+FFFF must be identical between the 3/1/4 or 3/10/12 subtables. Also note that the characters mapped in the 3/10/12 subtable must be a superset of the characters mapped in the 3/1/4 subtable.

Remember that encoding records must be stored in sorted order by platform ID, then by encoding ID.

## 'cvt' table

Should be defined only if required by font instructions.

## 'fpgm' table

Should be defined only if required by TrueType font instructions.

## 'glyf' table

The 'glyf' table contains TrueType outline data, and can be optimized by applying Microtype Express compression defined in ISO/IEC 14496-18.

NOTE	It is recommended that developers perform this optimization prior to finalizing and adding a digital signature to the font. This is necessary for the creator's signature to remain valid in embedded OFF fonts.

## 'hdmx' table

This table improves the performance of OFF fonts with TrueType outlines. This table is not necessary at all unless instructions are used to control the "phantom points", and should be omitted if bits 2 and 4 of the flags field in the 'head' table are zero. (See the 'head' table description.) It is recommended that this table be included for fonts with one or more non-linearly scaled glyphs (i.e., bit 2 or 4 of the 'head' table flags field are set).

Device records should be defined for all sizes from 8 through 14 point, and even point sizes from 16 through 24 point. However, the table requires pixel-per-em sizes, which depend on the horizontal resolution of the output device. The records in 'hdmx' should cover both 96 dpi devices (CGA, EGA, VGA) and 300 dpi devices (laser and ink jet printers).

Thus, 'hdmx' should contain entries for the following pixel sizes (PPEM): 11, 12, 13, 15, 16, 17, 19, 21, 24, 27, 29, 32, 33, 37, 42, 46, 50, 54, 58, 67, 75, 83, 92, 100. These values have been rounded to the nearest pixel. For instance, 12 points at 300 dpi would measure 37.5 pixels, but this is rounded down to 37 for this list.

This will add approximately 9,600 bytes to the font file. However, there will be a significant improvement in speed when a client requests advance widths covered by these device records.

If the font includes an 'LTSH' table, the hdmx values are not needed above the linearity threshold.

## 'head' table

Although historical usage of the fontRevision value is varied, the recommended use of the field is to set it as a Fixed 16.16 value, and to report it rounded and zero-padded to three fractional decimal places. Examples: Decimal 1.5 is set as 0x00018000 and is reported as "1.500"; decimal 1.001 is set as 0x00010041 and is reported as "1.001". All data required. If the font has been compressed with Microtype Express compression defined in ISO/IEC 14496-18 this must be indicated in the flags field of the 'head' table.

## 'hhea' table

All data required. It is suggested that monospaced fonts set numberOfHMetrics to three (see hmtx).

## 'hmtx' table

All data required. It is suggested that monospaced fonts have three entries in the numberOfHMetrics field.

## 'kern' table

Should contain a single kerning pair subtable (format 0). Windows will not support format 2 (two-dimensional array of kern values by class); nor multiple tables (only the first format 0 table found will be used) nor coverage bits 0 through 4 (i.e. assumes horizontal data, kerning values, no cross stream, and override).

The OFF specification allows CFF OT fonts to express their kerning in a kern table. Many OFF text layout engines support this. Windows GDI’s CFF OT driver, however, ignores the kern table in a CFF OT font when it prepares kerning pairs to report via its pair kerning API.

When a kern table and GPOS table are both present in a font, and an OFF layout engine is requested to apply kerning to a run of text of a particular script and language system: (a) If the number of kern feature lookups in the resolved language system in the GPOS table is zero, then the kern table should be applied, followed by any remaining GPOS features requested. (b) If the number of kern feature lookups in the resolved language system in the GPOS table is non-zero, then all GPOS lookups, including the kern lookups, should be applied in the usual way and the kern table data ignored.

If a kern table present but no GPOS table is present in the font, then an OFF layout engine should apply the kern table to the text, regardless of the resolved language system of the text.

If compatibility with legacy environments is not a concern, font vendors are encouraged to record kerning in the GPOS table's kern feature and not in the kern table.

OFF Font Variations mechanisms do not include any way to represent variation of data within a kern table. Therefore, kerning in a variable should be implemented using the GPOS table.

## 'loca' table

All data required for fonts with TrueType outlines. We recommend that local offsets should be word-aligned, in both the short and long formats of this table.

The actual ordering of the glyphs in the font can be optimized based on expected utilization, with the most frequently used glyphs appearing at the beginning of the font file. Additionally, glyphs that are often used together should be grouped together in the file. The will help to minimize the amount of swapping required when the font is loaded into memory.

## 'LTSH' table

This table improves the performance of OFF fonts with TrueType outlines. The table should be used if bit 2 or 4 of flags in 'head' is set.

## 'maxp' table

All data required for a font with TrueType outlines. Fonts with CFF or CFF2 data must only fill the numGlyphs field.

## 'name' table

Platform and encoding ID's in the name table should be consistent with those in the 'cmap' table. If they are not, the font will not load in Windows. When building a Unicode font for Windows, the platform ID should be 3 and the encoding ID should be 1. When building a symbol font for Windows, the platform ID should be 3 and the encoding ID should be 0, and the referenced string data must be encoded in UTF-16.

When building a font containing Roman characters that will be used on the Macintosh, an additional name record is required, specifying platform ID of 1 and encoding ID of 0.

Each set of name records should appear for US English (language ID = 0x0409 for Microsoft records, language ID = 0 for Macintosh records); additional language strings for the Microsoft set of records (platform ID 3) may be added at the discretion of the font vendor.

Remember that, despite references to "first" and "second", the name record must be stored in sorted order (by platform ID, encoding ID, language ID, name ID). The 'name' table platform/encoding IDs must match the 'cmap' table platform/encoding IDs, which is how Windows knows which name set to use.

Name strings

We recommend using name ID's 8 – 12, to identify manufacturer, designer, description, URL of the vendor, and URL of the designer. URL's must contain the protocol of the site: for example, http:// or mailto: or ftp://. The OFF font properties extension can enumerate this information to the users.

The Subfamily string in the 'name' table should be used for variants of weight (ultra light to extra black) and style (oblique/italic or not). So, for example, the full font name of "Helvetica Narrow Italic" should be defined as Family name "Helvetica Narrow" and Subfamily "Italic". This is so that Windows can group the standard four weights of a font in a reasonable fashion for non-typographically aware applications which only support combinations of "bold" and "italic".

The Full font name string usually contains a concatenation of strings 1 and 2. If the font is 'Regular' as indicated in string 2, then sometimes only the family name contained in string 1 is used for the full font name. In many contexts, the full font name is what will be exposed to users.

In variable fonts, the Typographic Family and Typographic Subfamily names (name IDs 16 and 17) are required. Applications that support OFF font variations will typically present to users the Typographic Family name along with the Typographic Subfamily name or alternative subfamily names for named instances, as specified in the font variations ('fvar') table. In some situations, a font vendor may want to make available a variable font as well as some set of non-variable fonts corresponding to the named instances of the variable font. In such situations, the vendor may want to have distinct family names for the family implemented as a variable font and the family implemented using several non-variable fonts. In that case, a suggested convention is to append “VF” at the end of the variable-font family name. Note, however, that this will result in distinct families, and content formatted with the one may not display as intended in some contexts if only the other is available. For example, if a document is formatted using the regular and bold instances of a variable font with family name "Selawik VF" and then the document is viewed in a context in which only the non-variable fonts Selawik Regular and Selawik Bold are available, the viewing application will generally not be able to associate the non-variable fonts available to it with the formatting declarations in the content.

OFF fonts that include a name with name ID of 6 should include these two names with name ID 6, and characteristics as follows:

- Platform: 1 [Macintosh]; Platform-specific encoding: 0 [Roman]; Language: 0 [English].

- Platform: 3 [Windows]; Platform-specific encoding: 1 [Unicode]; Language: 0x409 [English (American)].

Names with name ID 6 other than the above two, if present, may be ignored.

When translated to ASCII, these two name strings must be identical and restricted to the printable ASCII subset, codes 33 through 126, except for the 10 characters: '[', ']', '(', ')', '{', '}', '<', '>', '/', '%'. Some implementations have a 63-character length limit; however, a 127-character length limit is recommended.

The term "PostScript Name" here means a string identical to the two identical name ID 6 strings described above.

Depending on the particular font format that PostScript language font uses, the invocation method for the PostScript font differs, and the semantics of the resulting PostScript language font differ. The method used to invoke this font depends on the presence of Name ID 20.

If a Name ID 20 is present in this font, then the default assumption should be that the PostScript Name defined by name ID 6 should be used with the "composefont" invocation. This PostScript Name is then the name of a PostScript language CIDFont resource which corresponds to the glyphs of the OFF font. This name is valid to pass, with an appropriate PostScript language CMap reference, and an instance name, to the PostScript language "composefont" operator.

If no Name ID 20 is present in this font, then the default assumption should be that the PostScript Name defined by name ID 6 should be used with the "findfont" invocation, for locating the font in the context of a PostScript interpreter. This PostScript Name is then the name of a PostScript language Font resource which corresponds to the glyphs of the OFF font. This name is valid to pass to the PostScript language "findfont" operator. This does not necessarily imply that the resulting font dictionary accepts an /Encoding array, such as when the font referenced is a Type 0 PostScript font.

This requirement applies only to data fork OFF fonts. Macintosh resource-fork TrueType and other Macintosh sfnt-wrapped fonts supply the PostScript font name to be used with the "findfont" invocation, in order to invoke the font in a PostScript interpreter, in the FOND resource style-mapping table.

Developers may choose to ignore the default usage when appropriate. For example, PostScript printers whose version is earlier than 2015 cannot process CID font resources and a CJK OFF/CFF-CID font can be downloaded only as a set of Type 1 PostScript fonts. Legacy CJK TrueType fonts, which do not have a Name ID 20, may still be most effectively downloaded as a CID font resource. Definition of the full set of situations in which the defaults should not be followed is outside the scope of this document.

The value held in the name ID 20 string is interpreted as a PostScript font name that is meant to be used with the "findfont" invocation, in order to invoke the font in a PostScript interpreter.

If the name ID 20 is present in a font, there must be one name ID 20 record for every Macintosh platform cmap subtable in that font. A particular name ID 20 record is associated with the encoding specified by the matching cmap subtable. A name ID 20 record is matched to a cmap subtable when they have the same platform and platform-specific encoding IDs, and corresponding language/version IDs. Name ID 20 records are meant to be used only with Macintosh cmap subtables. The version field for a cmap subtable is one more than the language ID value for the corresponding name ID 20 record, with the exception of the cmap subtable version field 0. This version field, meaning "not language-specific", corresponds to the language ID value 0xFFFF, or decimal 65535, for the corresponding name ID 20 record.

When translated to ASCII, this name string must be restricted to the printable ASCII subset, codes 33 through 126, except for the 10 characters: '[', ']', '(', ')', '{', '}', '<', '>', '/', '%'.

This requirement applies only to data fork OFF fonts. Macintosh resource-fork TrueType and other Macintosh sfnt-wrapped fonts supply the PostScript font name to be used with the "findfont" invocation, in order to invoke the font in a PostScript interpreter, in the FOND resource style-mapping table.

A particular Name ID 20 string always corresponds to a particular Macintosh cmap subtable. However, some host OFF/TTF fonts also contain a post table, format 4, which provides a mapping from glyph ID to encoding value, and also corresponds to a particular Macintosh cmap subtable. Unfortunately, the post table format 4 contains no provision for identifying which Macintosh cmap subtable it matches, nor for providing more than one mapping. Host fonts which contain a post table format 4, should also contain only a single Macintosh cmap subtable, and a single Name ID 20 string. In the case where there is more than one Macintosh cmap subtable and more than one Name ID 20 string, there is no definition of which one matches the post table format 4.

## 'OS/2' table

All data required.

PANOSE values

PANOSE values improve the user's experience for font selection in some applications or font management utilities.

If the font is a symbol font, the first byte of the PANOSE value must be set to “Latin Pictorial” (value = 5).

In a variable font that uses OFF Font Variation mechanisms, there is no way to represent different PANOSE values for different instances supported by the font. The PANOSE values can be set based on the default instance

sTypoAscender, sTypoDescender and sTypoLineGap

The sTypoAscender, sTypoDescender and sTypoLineGap fields are used to specify the recommended default line spacing for single-spaced horizontal text. The baseline-to-baseline distance is calculated as follows:

OS/2.sTypoAscender - OS/2.sTypoDescender + OS/2.sTypoLineGap

sTypoAscender should be used to determine an optimal default offset from the top of a text frame to the first baseline. Similarly, sTypoDescender should be used to determine an offset from the last baseline to the bottom of the text frame.

It is often appropriate to set the sTypoAscender and sTypoDescender values such that the distance (sTypoAscender - sTypoDescender) is equal to one em. This is not a requirement, however, and may not be suitable in some situations. For example, if a font is designed for a script that (in horizontal layout) requires greater vertical extent relative to Latin script but also needs to support Latin script, and needs to have the visual size of Latin glyphs be similar to other fonts when set at the same text size, then the (sTypoAscender - sTypoDescender) distance for that font would likely need to be greater than one em.

The sTypoLineGap value will often be set such that the default baseline-to-baseline distance is approximately 120% of the em. For example, in the Minion Pro font family, fonts are designed on a 1000 units-per-em grid, the (sTypoAscender - sTypoDescender) distance is one em, and the sTypoLineGap value is set to 200.

In CJK (Chinese, Japanese, and Korean) fonts, it is permissible for the sTypoDescender and sTypoAscender fields to specify metrics different from the HorizAxis.ideo and HorizAxis.idtp baselines in the 'BASE' table. However, some applications may not read the 'BASE' table at all but simply use the sTypoDescender and sTypoAscender fields to describe the bottom and top edges of the ideographic em-box. If developers want their fonts to work correctly with such applications, they should ensure that any ideographic em-box values in the 'BASE' table describe the same bottom and top edges as the sTypoDescender and sTypoAscender fields. See §9.8 "OFF CJK Font Guidelines" and "Ideographic Em-Box" respectively for more details.

usLowerOpticalPointSize and usUpperOpticalPointSize

Use of the usLowerOpticalPointSize and usUpperOpticalPointSize fields has been superseded by the ‘STAT’ table. See §9.9 "Families with optical size variants" for more information.

## 'post' table

All information required, although the VM Usage fields may be set to zero. OFF fonts containing CFF outlines use only format 3.0 of the 'post' table. Glyph names are described in the Adobe document "Unicode and Glyph Names" [3], which specifies glyph naming conventions for all Unicode characters as well as those that don't have standard Unicode values such as certain ligatures or glyphic variants.

NOTE 1	Names for all glyphs must be supplied as it cannot be assumed that all Windows platforms will support the default names supplied on the Macintosh.

NOTE 2	The PostScript glyph name must be no longer than 31 characters, include only uppercase or lowercase English letters, European digits, the period or the underscore, i.e. from the set [A-Za-z0-9_.] and start with a letter, except the special glyph names “,notdef” and “.null” which start with a period.

## 'prep' table

Should be defined only if required by the TrueType font instructions.

## 'VDMX' table

This table improves the performance of OFF fonts with TrueType outlines. It should be present if hints cause the font to scale non-linearly. If not present, the font is assumed to scale linearly. Clipping may occur if values in this table are absent and font exceeds linear height.

## TrueType Collections

The process of building TTC files involves paying close attention to the issue of glyph renumbering in a font and the side effects that can result, in the 'cmap' table and elsewhere. The fonts to be merged must also have compatible TrueType instructions – i.e. their pre-programs, function definitions, and control values must not conflict.

# General recommendations

## Optimized table ordering

OFF fonts with TrueType outlines are more efficient in the Windows operating system when the tables are ordered as follows (from first to last):

head, hhea, maxp, OS/2, hmtx, LTSH, VDMX, hdmx, cmap, fpgm, prep, cvt, loca, glyf, kern, name, post, gasp, PCLT, DSIG

The initial loading of an OFF font containing CFF data will be more efficiently handled if the following sfnt table ordering is used within the body of the sfnt (listed from first to last):

head, hhea, maxp, OS/2, name, cmap, post, CFF, (other tables, as convenient)

## Non-standard (Symbol) fonts

Non-standard fonts such as Symbol or Wingdings™ have special requirements for Windows platforms. These requirements affect the 'cmap,' 'name,' and 'OS/2' tables; the requirements and recommendations for all other tables remain the same.

For non-standard fonts on Windows platforms, however, the 'cmap' and 'name' tables must use platform ID 3 () and encoding ID 0 (Unicode, non-standard character set). Remember that 'name' table encodings should agree with the 'cmap' table. Additionally, the first byte of the PANOSE value in the 'OS/2' table must be set to 'Latin Pictorial’ (value = 5).

The 'cmap' subtable (platform 3, encoding 0) must use format 4. The character codes should start at 0xF000, which is in the Private Use Area of Unicode. It is suggested to derive the format 4 encodings by simply adding 0xF000 to the format 0 (Macintosh) encodings.

Under Windows, only the first 224 characters of non-standard fonts will be accessible: a space and up to 223 printing characters. It does not matter where in user space these start, but 0xF020 is suggested. The usFirstCharIndex and usLastCharIndex values in the 'OS/2' table would be set based on the actual minimum and maximum character indices used.

## Baseline to baseline distances

The 'OS/2' table fields sTypoAscender, sTypoDescender, and sTypoLineGap free applications from Macintosh- or Windows-specific metrics which are constrained by backward compatibility requirements. The following discussion only pertains to the platform-specific metrics.

The suggested Baseline to Baseline Distance (BTBD) is computed differently for Windows and the Macintosh, and it is based on different OFF metrics. However, if the recommendations below are followed, the BTBD will be the same for both Windows and the Mac.

Windows

The Windows metrics in the table below are returned as part of the logical font data structure.

| Windows Metric | OFF Metric |
| --- | --- |
| Ascent | usWinAscent |
| descent | usWinDescent |
| internal leading | usWinAscent + usWinDescent - unitsPerEm |
| external leading | MAX(0, LineGap - ((usWinAscent + usWinDescent) - (Ascender - Descender))) |

The suggested BTBD = ascent + descent + external leading

It should be clear that the "external leading" can never be less than zero. Pixels above the ascent or below the descent will be clipped from the character; this is true for all output devices.

The usWinAscent and usWinDescent are values from the 'OS/2' table. The unitsPerEm value is from the 'head' table. The LineGap, Ascender and Descender values are from the 'hhea' table.

Macintosh

Ascender and Descender are metrics defined and are not to be confused with the Windows ascent or descent, nor should they be confused with the true typographic ascender and descender that are found in AFM files.

| Macintosh Metric | OFF Metric |
| --- | --- |
| ascender | Ascender |
| descender | Descender |
| Leading | LineGap |

The suggested BTBD = ascent + descent + leading

If pixels extend above the ascent or below the descent, the character will be squashed in the vertical direction so that all pixels fit within these limitations; this is true for screen display only.

Making Them Match

If you perform some simple algebra, you will see that the suggested BTBD across both Macintosh and Windows will be identical if and only if:

LineGap >= (yMax - yMin) - (Ascender - Descender)

## Style bits

For backwards compatibility with previous versions of Windows, the macStyle bits in the 'head' table will be used to determine whether or not a font is regular, bold or italic (in the absence of an 'OS/2' table). This is completely independent of the usWeightClass and PANOSE information in the 'OS/2' table, the ItalicAngle in the 'post' table, and all other related metrics. If the 'OS/2' table is present, then the fsSelection bits are used to determine this information.

## Drop-out control

Drop-out control is needed if there is a difference in bitmaps with dropout control on and off. Two cases where drop-out control is needed are when the font is rotated or when the size of the font is at or below 8 ppem. Do not use SCANCTRL unless needed. SCANCTRL or the drop-out control rasterizer should be avoided for Roman fonts above 8 points per em (ppem) when the font is not under rotation. SCANCTRL should not be used for "stretched" fonts (e.g. fonts displayed at non-square aspect ratios, like that found on an EGA).

## Embedded bitmaps

Three tables are used to embed bitmaps in OFF fonts. They are the 'EBLC' table for embedded bitmap locators, the 'EBDT' table for embedded bitmap data, and the 'EBSC' table for embedded bitmap scaling information. OFF embedded bitmaps are also called 'sbits'.

The behavior of sbits within an OFF font is essentially transparent to the client. A client need not be aware whether the bitmap returned by the rasterizer comes from an sbit or from a scan-converted outline.

The metrics in 'sbit' tables overrule the outline metrics at all sizes where sbits are defined. Fonts with 'hdmx' tables should correct those tables with 'sbit' values.

'Sbit only' fonts, that is fonts with embedded bitmaps but without outline data, are permitted. Care must be taken to ensure that all required OFF tables except 'glyf' and 'loca' are present in such a font. Obviously, such fonts will only be able to return glyphs and sizes for which sbits are defined. These metrics are returned as part of the logical font data structure in the Macintosh platform.

## OFF CJK font guidelines

This clause provides a checklist of links to various CJK-related clauses of the OFF specification. Some items are requirements; others, recommendations:

- The ideographic em-box of an OFF font will be determined as described in "Ideographic Em-Box" in the Baseline Tags of the OFF Layout Tag Registry. Also see the description for OS/2.sTypoAscender and OS/2.sTypoDescender, and the 'BASE' table recommendation in clause 6.above.

- CJK font vendors can choose to provide the ideographic character face (ICF) metrics, which applications can use for accurate text alignment. This is described in "Ideographic Character Face" in the Baseline Tags clause of the OFF Layout Tag Registry.

- All OFF fonts that are used for vertical writing must include a Vertical Header ('vhea') table and a Vertical Metrics ('vmtx') table. It is strongly recommended that CFF fonts that are used for vertical writing include a Vertical Origin ('VORG') table.

- If an OFF font with CFF outlines is to be used for vertical writing, Adobe Type Manager/NT 4.1 and the Windows 2000 OTF driver require that a Vertical Rotation ('vrt2') feature be present in the Glyph Substitution ('GSUB') table. See the Feature Tags (subclause 6.4.3) and informative reference [11] in the bibliography for a description of and further requirements for this feature.

- See the Feature Tags (subclause 6.4.3) and Reference [11] for descriptions of currently registered OFF layout features, such as Alternate Half Widths ('halt') and Traditional Forms ('trad') that can be specified in the font.

## Stroke reduction in variable fonts

When designing a font family to support a number of variations, there may be cases in which it is desirable to make significant, structural changes to particular glyphs for certain variations. A common example is stroke reduction for heavier weights or narrower widths, simplifying the structure of a glyph so that counters do not become filled and disappear at smaller text sizes. Within a variable font, two techniques might be used to implement a stroke-reduction effect:

- Use a pair of slightly-overlapping intermediate regions within the variation data for a glyph in order to introduce deltas for particular contour points that result in the desired structural change and that apply for only problem ranges on one or more axes.

- Use an OFF Layout Required Variation Alternates feature in combination with a FeatureVariations table wihthin the 'GSUB' table to perform a glyph substitution when a variation instance is selected in some range along one or more axes.

While both techniques are possible, it should be noted that the first technique, using overlapping intermediate-regions, can be tricky to implement and may result in unexpected or undesired results if an instance is selected using arbitrary axis values in the range over which the transition occurs. The second technique is recommended as it will generally be easier to implement and maintain, and provides the font designer better control over behavior near the point of transition.

## Families with optical size variants

In families that have fonts for different optical sizes or in variable fonts that support the optical size ('opsz') design axis, a 'STAT' table with format 2 axis value tables should be used to indicate text size ranges for which the different optical-size variants or variable-font named instances are recommended. This supersedes the use of the usLowerOpticalPointSize and usUpperOpticalPointSize fields in the 'OS/2' table, and the OFF Layout 'size' feature.

When creating an axis value table to correspond to the font or named instance that is intended for the largest text sizes, the upper text size limit should be, effectively, infinity. To represent this in a format 2 axis value table, set the rangeMaxValue to 0x7FFFFFFF.

- (informative)

IBM font family classifications

  - General

The section defines the family class and subclass values that may be used in the sFamilyClass field of a font’s OS/2 table (see 5.1.8.16). A font designer or supplier can assign these values in a font to provide a classification of the font family design. The registered values documented here were originally defined by IBM.

These values classify a font design as to its appearance, but do not identify the specific font family, typeface variation, designer, supplier, size, or metric table differences. It should be noted that some font designs can be classified equally well into more than class or subclass. Such designs should be matched to a classification for which substitution of another font design from the same class or subclass would generally result in a similar appearance of the presented document.

  - Class ID=0 No Classification

This class ID is used to indicate that the associated font has no design classification or that the design classification is not of significance to the creator or user of the font resource.

  - Class ID=1 Oldstyle Serifs

This style is generally based upon the Latin printing style of the 15th to 17th century, with a mild diagonal contrast in stroke emphasis (lighter in upper left to lower right, heavier in upper right to lower left) and bracketed serifs. This Class reflects the ISO Serif Class, Oldstyle and Legibility Subclasses as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design subclassification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: IBM Rounded Legibility

This style is generally characterized by a large x-height, with short ascenders and descenders. Specifically, it is distinguished by a medium resolution, hand tuned, bitmap rendition of the more general rounded legibility subclass. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Garalde

This style is generally characterized by a medium x-height, with tall ascenders. An example of this font style is the ITC Garamond family. This IBM Subclass reflects the ISO Serif Class, Oldstyle Subclass, and Garalde Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Venetian

This style is generally characterized by a medium x-height, with a relatively monotone appearance and sweeping tails based on the designs of the early Venetian printers. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: Modified Venetian

This style is generally characterized by a large x-height, with a relatively monotone appearance and sweeping tails based on the designs of the early Venetian printers. An example of this font style is the Allied Linotype Palatino family. This IBM Subclass reflects the ISO Serif Class, Transitional Subclass, and Modified Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5: Dutch Modern

This style is generally characterized by a large x-height, with wedge shaped serifs and a circular appearance to the bowls similar to the Dutch Traditional Subclass below, but with lighter stokes. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 6: Dutch Traditional

This style is generally characterized by a large x-height, with wedge shaped serifs and a circular appearance of the bowls. An example of this font style is the IBM Press Roman family. This IBM Subclass reflects the ISO Serif Class and Legibility Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 7: Contemporary

This style is generally characterized by a small x-height, with light stokes and serifs. An example of this font style is the University family. This IBM Subclass reflects the ISO Serif Class and Contemporary Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 8: Calligraphic

This style is generally characterized by the fine hand writing style of calligraphy, while retaining the characteristic Oldstyle appearance. This IBM Subclass is not reflected in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 9 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=2 Transitional Serifs

This style is generally based upon the Latin printing style of the 18th to 19th century, with a pronounced vertical contrast in stroke emphasis (vertical strokes being heavier than the horizontal strokes) and bracketed serifs. This IBM Class reflects the ISO Serif Class, Transitional Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Direct Line

This style is generally characterized by a medium x-height, with fine serifs, noticeable contrast, and capitol letters of approximately the same width. An example of this font style is the Monotype Baskerville family. This IBM Subclass reflects the ISO Serif Class, Transitional Subclass, and Direct Line Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Script

This style is generally characterized by a hand written script appearance while retaining the Transitional Direct Line style. An example of this font style is the IBM Nasseem (Arabic) family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard, though the ISO Serif Class, Transitional Subclass, and Direct Line Specific Group would be a close approximation.

### Subclass ID = 3 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=3 Modern Serifs

This style is generally based upon the Latin printing style of the 20th century, with an extreme contrast between the thick and thin portion of the strokes. This IBM Class reflects the ISO Serif Class, Modern Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Italian

This style is generally characterized by a medium x-height, with thin hairline serifs. An example of this font style is the Monotype Bodoni family. This IBM Subclass reflects the ISO Serif Class, Modern Subclass, and Italian Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Script

This style is generally characterized by a hand written script appearance while retaining the Modern Italian style. An example of this font style is the IBM Narkissim (Hebrew) family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard, though the ISO Serif Class, Modern Subclass, and Italian Specific Group would be a close approximation.

### Subclass ID = 3 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=4 Clarendon Serifs

This style is a variation of the Oldstyle Serifs and the Transitional Serifs, with a mild vertical stroke contrast and bracketed serifs. This IBM Class reflects the ISO Serif Class, Square Serif Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Clarendon

This style is generally characterized by a large x-height, with serifs and strokes of equal weight. An example of this font style is the Allied Linotype Clarendon family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Clarendon Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Modern

This style is generally characterized by a large x-height, with serifs of a lighter weight than the strokes and the strokes of a lighter weight than the Traditional. An example of this font style is the Monotype Century Schoolbook family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Clarendon Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Traditional

This style is generally characterized by a large x-height, with serifs of a lighter weight than the strokes. An example of this font style is the Monotype Century family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Clarendon Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: Newspaper

This style is generally characterized by a large x-height, with a simpler style of design and serifs of a lighter weight than the strokes. An example of this font style is the Allied Linotype Excelsior Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Clarendon Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5: Stub Serif

This style is generally characterized by a large x-height, with short stub serifs and relatively bold stems. An example of this font style is the Cheltenham Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Short Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 6: Monotone

This style is generally characterized by a large x-height, with monotone stems. An example of this font style is the ITC Korinna Family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 7: Typewriter

This style is generally characterized by a large x-height, with moderate stroke thickness characteristic of a typewriter. An example of this font style is the Prestige Elite Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Typewriter Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 8 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=5 Slab Serifs

This style is characterized by serifs with a square transition between the strokes and the serifs (no brackets). This IBM Class reflects the ISO Serif Class, Square Serif Subclass (except the Clarendon Specific Group) as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Monotone

This style is generally characterized by a large x-height, with serifs and strokes of equal weight. An example of this font style is the ITC Lubalin Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Humanist

This style is generally characterized by a medium x-height, with serifs of lighter weight that the strokes. An example of this font style is the Candida Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Geometric

This style is generally characterized by a large x-height, with serifs and strokes of equal weight and a geometric (circles and lines) design. An example of this font style is the Monotype Rockwell Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: Swiss

This style is generally characterized by a large x-height, with serifs and strokes of equal weight and an emphasis on the white space of the characters. An example of this font style is the Allied Linotype Serifa Family. This IBM Subclass reflects the ISO Serif Class, Square Serif Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5: Typewriter

This style is generally characterized by a large x-height, with serifs and strokes of equal but moderate thickness, and a geometric design. An example of this font style is the IBM Courier Family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard, though the ISO Serif Class, Square Serif Subclass, and Monotone Specific Group would be a close approximation.

### Subclass ID = 6 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment, and shall not be used without formal assignment by IBM.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=6 (reserved for future use)

This class ID is reserved for future assignment.

  - Class ID=7 Freeform Serifs

This style includes serifs, but which expresses a design freedom that does not generally fit within the other serif design classifications. This IBM Class reflects the remaining ISO Serif Class subclasses as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Modern

This style is generally characterized by a medium x-height, with light contrast in the strokes and a round full design. An example of this font style is the ITC Souvenir Family. This IBM Subclass is not reflected in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=8 Sans Serifs

This style includes most basic letter forms (excluding Scripts and Ornamentals) that do not have serifs on the strokes. This IBM Class reflects the ISO Sans Serif Class as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: IBM Neo-grotesque Gothic

This style is generally characterized by a large x-height, with uniform stroke width and a simple one story design distinguished by a medium resolution, hand tuned, bitmap rendition of the more general Neo-grotesque Gothic Subclass. An example of this font style is the IBM Sonoran Sans Serif family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Humanist

This style is generally characterized by a medium x-height, with light contrast in the strokes and a classic Roman letterform. An example of this font style is the Allied Linotype Optima family. This IBM Subclass reflects the ISO Sans Serif Class, Humanist Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Low-x Round Geometric

This style is generally characterized by a low x-height, with monotone stroke weight and a round geometric design. An example of this font style is the Fundicion Tipograficia Neufville Futura family. This IBM Subclass reflects the ISO Sans Serif Class, Geometric Subclass, Round Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: High-x Round Geometric

This style is generally characterized by a high x-height, with uniform stroke weight and a round geometric design. An example of this font style is the ITC Avant Garde Gothic family. This IBM Subclass reflects the ISO Sans Serif Class, Geometric Subclass, Round Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5: Neo-grotesque Gothic

This style is generally characterized by a high x-height, with uniform stroke width and a simple one story design. An example of this font style is the Allied Linotype Helvetica family. This IBM Subclass reflects the ISO Sans Serif Class, Gothic Subclass, Neo-grotesque Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 6: Modified Neo-grotesque Gothic

This style is similar to the Neo-grotesque Gothic style, with design variations to the G and Q. An example of this font style is the Allied Linotype Univers family. This IBM Subclass is not strictly specified in the ISO/IEC 9541-1 Amendment 1 standard, though the ISO Sans Serif Class, Gothic Subclass, Neo-grotesque Specific Group would be a close approximation.

### Subclass ID = 7 – 8: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 9: Typewriter Gothic

This style is similar to the Neo-grotesque Gothic style, with moderate stroke thickness characteristic of a typewriter. An example of this font style is the IBM Letter Gothic family. This IBM Subclass reflects the ISO Sans Serif Class, Gothic Subclass, Typewriter Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 10: Matrix

This style is generally a simple design characteristic of a dot matrix printer. An example of this font style is the IBM Matrix Gothic family. This IBM Subclass is not reflected in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 11 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=9 Ornamentals

This style includes highly decorated or stylized character shapes that are typically used in headlines. This IBM Class reflects the ISO Ornamental Class and the ISO Blackletter Class as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Engraver

This style is characterized by fine lines or lines engraved on the stems. An example of this font style is the Copperplate family. This IBM Subclass reflects the ISO Ornamental Class and Inline Subclass, or the Serif Class and Engraving Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Black Letter

This style is generally based upon the printing style of the German monasteries and printers of the 12th to 15th centuries. An example of this font style is the Old English family. This IBM Subclass reflects the ISO Blackletters Class as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Decorative

This style is characterized by ornamental designs (typically from nature, such as leaves, flowers, animals, etc.) incorporated into the stems and strokes of the characters. An example of this font style is the Saphire family. This IBM Subclass reflects the ISO Ornamental Class and Decorative Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: Three Dimensional

This style is characterized by a three dimensional (raised) appearance of the characters created by shading or geometric effects. An example of this font style is the Thorne Shaded family. This IBM Subclass reflects the ISO Ornamental Class and Three Dimensional Subclass as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=10 Scripts

This style includes those typefaces that are designed to simulate handwriting. This IBM Class reflects the ISO Script Class and Uncial Class as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1: Uncial

This style is characterized by unjoined (nonconnecting) characters that are generally based on the hand writing style of Europe in the 6th to 9th centuries. An example of this font style is the Libra family. This IBM Subclass reflects the ISO Uncial Class as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 2: Brush Joined

This style is characterized by joined (connecting) characters that have the appearance of being painted with a brush, with moderate contrast between thick and thin strokes. An example of this font style is the Mistral family. This IBM Subclass reflects the ISO Script Class, Joined Subclass, and Informal Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 3: Formal Joined

This style is characterized by joined (connecting) characters that have a printed (or drawn with a stiff brush) appearance with extreme contrast between the thick and thin strokes. An example of this font style is the Coronet family. This IBM Subclass reflects the ISO Script Class, Joined Subclass, and Formal Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4: Monotone Joined

This style is characterized by joined (connecting) characters that have a uniform appearance with little or no contrast in the strokes. An example of this font style is the Kaufmann family. This IBM Subclass reflects the ISO Script Class, Joined Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 5: Calligraphic

This style is characterized by beautifully hand drawn, unjoined (non-connecting) characters that have an appearance of being drawn with a broad edge pen. An example of this font style is the Thompson Quillscript family. This IBM Subclass reflects the ISO Script Class, Unjoined Subclass, and Calligraphic Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 6: Brush Unjoined

This style is characterized by unjoined (non-connecting) characters that have the appearance of being painted with a brush, with moderate contrast between thick and thin strokes. An example of this font style is the Saltino family. This IBM Subclass reflects the ISO Script Class, Unjoined Subclass, and Brush Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 7: Formal Unjoined

This style is characterized by unjoined (non-connecting) characters that have a printed (or drawn with a stiff brush) appearance with extreme contrast between the thick and thin strokes. An example of this font style is the Virtuosa family. This IBM Subclass reflects the ISO Script Class, Unjoined Subclass, and Formal Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 8: Monotone Unjoined

This style is characterized by unjoined (non-connecting) characters that have a uniform appearance with little or no contrast in the strokes. An example of this font style is the Gilles Gothic family. This IBM Subclass reflects the ISO Script Class, Unjoined Subclass, and Monotone Specific Group as documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 9 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment, and shall not be used without formal assignment by IBM.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=11 (reserved for future use)

This class ID is reserved for future assignment.

  - Class ID=12 Symbolic

This style is generally design independent, making it suitable for Pi and special characters (icons, dingbats, technical symbols, etc.) that may be used equally well with any font. This IBM Class reflects various ISO Specific Groups, as noted below and documented in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 0: No Classification

This subclass ID is used to indicate that the associated font has no design sub-classification or that the design sub-classification is not of significance to the creator or user of the font resource.

### Subclass ID = 1 – 2: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 3: Mixed Serif

This style is characterized by either both or a combination of serif and sans serif designs on those characters of the font for which design is important (e.g., superscript and subscript characters, numbers, copyright or trademark symbols, etc.). An example of this font style is found in the IBM Symbol family. This IBM Subclass is not reflected in the ISO/IEC 9541-1 Amendment 1 standard.

### Subclass ID = 4 – 5: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 6: Oldstyle Serif

This style is characterized by a Oldstyle Serif IBM Class design on those characters of the font for which design is important (e.g., superscript and subscript characters, numbers, copyright or trademark symbols, etc.). An example of this font style is found in the IBM Sonoran Pi Serif family. This IBM Subclass is not directly reflected in the ISO/IEC 9541-1 Amendment 1 standard, though it is indirectly by the ISO Serif Class and Legibility Subclass (implies that all characters of the font exhibit the design appearance, while only a subset of the characters actually exhibit the design).

### Subclass ID = 7: Neo-grotesque Sans Serif

This style is characterized by a Neo-grotesque Sans Serif IBM Font Class and Subclass design on those characters of the font for which design is important (e.g., superscript and subscript characters, numbers, copyright or trademark symbols, etc.). An example of this font style is found in the IBM Sonoran Pi Sans Serif family. This IBM Subclass is not directly reflected in the ISO/IEC 9541-1 Amendment 1 standard, though it is indirectly by the ISO Sans Serif Class and Gothic Subclass (implies that all characters of the font exhibit the design appearance, while only a subset of the characters actually exhibit the design).

### Subclass ID = 8 – 14: (reserved for future use)

These subclass IDs are reserved for future assignment.

### Subclass ID = 15: Miscellaneous

This subclass ID is used for miscellaneous designs of the associated design class that are not covered by another Subclass.

  - Class ID=13 Reserved

  - Class ID=14 Reserved

- (informative)
Earlier versions of OS/2 – OS/2 and Windows metrics

  - OS/2 - OS/2 and Windows metrics (version 0)

NOTE This is maintained for purposes of being able to validate version 0 OS/2 tables. Please note that the description of the OS/2 table version 0 in the original Apple TrueType specification [7] differs from this document – the fields ‘sTypoAscender’, ‘sTypoDescender’, ‘sTypoLineGap’, ‘usWinAscent’ and ‘usWinDescent’ are missing in the Apple TrueType specification. The OS/2 table consists of a set of metrics that are required in OFF fonts. The layout of version 0 of this table is as follows: Type; Name of Entry; Comments. uint16; version; 0x0000. int16; xAvgCharWidth. uint16; usWeightClass. uint16; usWidthClass. uint16; fsType. int16; ySubscriptXSize. int16; ySubscriptYSize. int16; ySubscriptXOffset. int16; ySubscriptYOffset. int16; ySuperscriptXSize. int16; ySuperscriptYSize. int16; ySuperscriptXOffset. int16; ySuperscriptYOffset. int16; yStrikeoutSize. int16; yStrikeoutPosition. int16; sFamilyClass. uint8; panose[10]. uint32; ulCharRange[4]; Bits 0-31. Tag; achVendID. uint16; fsSelection. uint16; usFirstCharIndex. uint16; usLastCharIndex. int16; sTypoAscender. int16; sTypoDescender. int16; sTypoLineGap. uint16; usWinAscent. uint16; usWinDescent. version Format:; uint16. Units:; n/a. Title:; OS/2 table version number.. Description:; The version number for this OS/2 table.. Comments:; The version number allows for identification of the precise contents and layout for the OS/2 table. The version number for this layout is zero (0).. xAvgCharWidth Format:; int16. Units:; Pels / em units. Title:; Average weighted escapement.. Description:; The Average Character Width parameter specifies the arithmetic average of the escapement (width) of all of the 26 lowercase letters a through z of the Latin alphabet and the space character. If any of the 26 lowercase letters are not present, this parameter should equal the weighted average of all glyphs in the font. For non-UGL (platform 3, encoding 0) fonts, use the unweighted average.. Comments:; This parameter is a descriptive attribute of the font that specifies the spacing of characters for comparing one font to another for selection or substitution. For proportionally spaced fonts, this value is useful in estimating the length for lines of text. The weighting factors provided with this example are only valid for Latin lowercase letters. If other character sets, or capital letters are used, the corresponding frequency of use values should be used. One needs to be careful when comparing fonts that use different frequency of use values for font mapping. The average character width for the following set of upper and lowercase letters only, is calculated according to this formula: Sum the individual character widths multiplied by the following weighting factors and then divide by 1000. For example:. Letter; Weight Factor; Letter; Weight Factor. a; 64; o; 56. b; 14; p; 17. c; 27; q; 4. d; 35; r; 49. e; 100; s; 56. f; 20; t; 71. g; 14; u; 31. h; 42; v; 10. i; 63; w; 18. j; 3; x; 3. k; 6; y; 18. l; 35; z; 2. m; 20; space; 166. n; 56. usWeightClass Format:; uint16. Title:; Weight class.. Description:; Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font.. Comments:. Value; Description; C Definition (from windows.h). 100; Thin; FW_THIN. 200; Extra-light (Ultra-light); FW_EXTRALIGHT. 300; Light; FW_LIGHT. 400; Normal (Regular); FW_NORMAL. 500; Medium; FW_MEDIUM. 600; Semi-bold (Demi-bold); FW_SEMIBOLD. 700; Bold; FW_BOLD. 800; Extra-bold (Ultra-bold); FW_EXTRABOLD. 900; Black (Heavy); FW_BLACK. usWidthClass Format:; uint16. Title:; Width class.. Description:; Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font.. Comments:; Although every character in a font may have a different numeric aspect ratio, each character in a font of normal width has a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the characters in the new font is some percentage greater or less than those same characters in the normal font -- it is this difference that this parameter specifies.. Value; Description; C Definition; % of normal. 1; Ultra-condensed; FWIDTH_ULTRA_CONDENSED; 50. 2; Extra-condensed; FWIDTH_EXTRA_CONDENSED; 62.5. 3; Condensed; FWIDTH_CONDENSED; 75. 4; Semi-condensed; FWIDTH_SEMI_CONDENSED; 87.5. 5; Medium (normal); FWIDTH_NORMAL; 100. 6; Semi-expanded; FWIDTH_SEMI_EXPANDED; 112.5. 7; Expanded; FWIDTH_EXPANDED; 125. 8; Extra-expanded; FWIDTH_EXTRA_EXPANDED; 150. 9; Ultra-expanded; FWIDTH_ULTRA_EXPANDED; 200. fsType Format:; uint16. Title:; Type flags.. Description:; Indicates font embedding licensing rights for the font. Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font. Applications that implement support for font embedding, either through use of the Font Embedding DLL or through other means, must not embed fonts which are not licensed to permit embedding. Further, applications loading embedded fonts for temporary use (see Preview & Print and Editable embedding below) must delete the fonts when the document containing the embedded font is closed.. Bit; Bit Mask; Description. 0x0000; Installable Embedding: No fsType bit is set. Thus fsType is zero. Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser.. 0; 0x0001; Reserved, must be zero.. 1; 0x0002; Restricted License embedding: Fonts that have only this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: For Restricted License embedding to take effect, it must be the only level of embedding selected.. 2; 0x0004; Preview & Print embedding: When this bit is set, the font may be embedded, and temporarily loaded on the remote system. Documents containing Preview & Print fonts must be opened "read-only;" no edits can be applied to the document.. 3; 0x0008; Editable embedding: When this bit is set, the font may be embedded but must only be installed temporarily on other systems. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened for reading, editing is permitted, and changes may be saved.. 4-15; Reserved, must be zero.. Comments:; If multiple embedding bits are set, the least restrictive license granted takes precedence. For example, if bits 1 and 3 are set, bit 3 takes precedence over bit 1and the font may be embedded with Editable rights. For compatibility purposes, most vendors granting Editable embedding rights are also setting the Preview & Print bit (0x000C). This will permit an application that only supports Preview & Print embedding to detect that font embedding is allowed.. Restricted License embedding (0x0002): Fonts that have this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: note that for Restricted License embedding to take effect, it must be the only level of embedding selected (as noted in the previous paragraph).. Preview & Print embedding (0x0004): Fonts with this bit set indicate that they may be embedded within documents but must only be installed temporarily on the remote system. Any document which includes a Preview & Print embedded font must be opened “read-only;” the application must not allow the user to edit the document; it can only be viewed and/or printed.. Editable embedding (0x0008): Fonts with this bit set indicate that they may be embedded in documents, but must only be installed temporarily on the remote system. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened “read-write;” editing is permitted, and changes may be saved.. Installable embedding (0x0000): Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser.. ySubscriptXSize Format:; int16. Units:; Font design units. Title:; Subscript horizontal font size.. Description:; The recommended horizontal size in font design units for subscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript character would be 1/10th the size of the normal character.. ySubscriptYSize Format:; int16. Units:; Font design units. Title:; Subscript vertical font size.. Description:; The recommended vertical size in font design units for subscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The horizontal font size specifies a font designer's recommendation for horizontal font size of subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the characters in a font or by substituting characters from another font, this parameter specifies the recommended horizontal EmInc for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript character would be 1/10th the size of the normal character.. ySubscriptXOffset Format:; int16. Units:; Font design units. Title:; Subscript x offset.. Description:; The recommended horizontal offset in font design untis for subscripts for this font.. Comments:; The Subscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin of the font to the character origin of the subscript's character -- for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters, this parameter specifies the recommended horizontal position from the character escapement point of the last character before the first subscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for subscript characters is usually adjusted to compensate for the angle of incline.. ySubscriptYOffset Format:; int16. Units:; Font design units. Title:; Subscript y offset.. Description:; The recommended vertical offset in font design units from the baseline for subscripts for this font.. Comments:; The Subscript Y offset parameter specifies a font designer's recommended vertical offset from the character baseline to the character baseline for subscript characters associated with this font. Values are expressed as a positive offset below the character baseline. If a font does not include all of the required subscript for an application, this parameter specifies the recommended vertical distance below the character baseline for those subscript characters.. ySuperscriptXSize Format:; int16. Units:; Font design units. Title:; Superscript horizontal font size.. Description:; The recommended horizontal size in font design units for superscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript character would be 1/10th the size of the normal character.. ySuperscriptYSize Format:; int16. Units:; Font design units. Title:; Superscript vertical font size.. Description:; The recommended vertical size in font design units for superscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The vertical font size specifies a font designer's recommended vertical font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended EmHeight for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript character would be 1/10th the size of the normal character.. ySuperscriptXOffset Format:; int16. Units:; Font design units. Title:; Superscript x offset.. Description:; The recommended horizontal offset in font design units for superscripts for this font.. Comments:; The Superscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin to the superscript character's origin for the superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended horizontal position from the escapement point of the character before the first superscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for superscript characters is usually adjusted to compensate for the angle of incline.. ySuperscriptYOffset Format:; int16. Units:; Font design units. Title:; Superscript y offset.. Description:; The recommended vertical offset in font design units from the baseline for superscripts for this font.. Comments:; The Superscript Y offset parameter specifies a font designer's recommended vertical offset -- from the character baseline to the superscript character's baseline associated with this font. Values for this parameter are expressed as a positive offset above the character baseline. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended vertical distance above the character baseline for those superscript characters.. yStrikeoutSize Format:; int16. Units:; Font design units. Title:; Strikeout size.. Description:; Width of the strikeout stroke in font design units.. Comments:; This field should normally be the width of the em dash for the current font. If the size is one, the strikeout line will be the line represented by the strikeout position field. If the value is two, the strikeout line will be the line represented by the strikeout position and the line immediately above the strikeout position. For a Roman font with a 2048 em square, 102 is suggested.. yStrikeoutPosition Format:; int16. Units:; Font design units. Title:; Strikeout position.. Description:; The position of the top of the strikeout stroke relative to the baseline in font design units.. Comments:; Positive values represent distances above the baseline, while negative values represent distances below the baseline. A value of zero falls directly on the baseline, while a value of one falls one pel above the baseline. The value of strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. For a Roman font with a 2048 em square, 460 is suggested.. sFamilyClass Format:; int16. Title:; Font-family class and subclass.. Description:; This parameter is a classification of font-family design.. Comments:; The font class and font subclass are registered values assigned by IBM to each font family. This parameter is intended for use in selecting an alternate font when the requested font is not available. The font class is the most general and the font subclass is the most specific. The high byte of this field contains the family class, while the low byte contains the family subclass.. Panose Format:; 10 byte array. Title:; PANOSE classification number. International:; Additional specifications are required for PANOSE to classify non-Latin character sets.. Description:; This 10 byte series of numbers is used to describe the visual characteristics of a given typeface. If provided, these characteristics are then used to associate the font with other fonts of similar appearance having different names; the default values should be set to 'zero'. The variables for each digit are listed below.. Comments:; The specification for assigning PANOSE values [14] can be found in bibliography and is maintained by Monotype Imaging Inc.. PANOSE. Type; Name; Description. uint8; bFamilyType;; Family Type. uint8; bSerifStyle;; Serif Style. uint8; bWeight;; Weight. uint8; bProportion;; Proportion. uint8; bContrast;; Contrast. uint8; bStrokeVariation;; Stroke Variation. uint8; bArmStyle;; Arm Style. uint8; bLetterform;; Letterform. uint8; bMidline;; Midline. uint8; bXHeight;; X Height. Family Type. Value; Description. 0; Any. 1; No Fit. 2; Text and Display. 3; Script. 4; Decorative. 5; Pictorial. Serif Style. Value; Description. 0; Any. 1; No Fit. 2; Cove. 3; Obtuse Cove. 4; Square Cove. 5; Obtuse Square Cove. 6; Square. 7; Thin. 8; Bone. 9; Exaggerated. 10; Triangle. 11; Normal Sans. 12; Obtuse Sans. 13; Perp Sans. 14; Flared. 15; Rounded. Weight. Value; Description. 0; Any. 1; No Fit. 2; Very Light. 3; Light. 4; Thin. 5; Book. 6; Medium. 7; Demi. 8; Bold. 9; Heavy. 10; Black. 11; Nord. Proportion. Value; Description. 0; Any. 1; No Fit. 2; Old Style. 3; Modern. 4; Even Width. 5; Expanded. 6; Condensed. 7; Very Expanded. 8; Very Condensed. 9; Monospaced. Contrast. Value; Description. 0; Any. 1; No Fit. 2; None. 3; Very Low. 4; Low. 5; Medium Low. 6; Medium. 7; Medium High. 8; High. 9; Very High. Stroke Variation. Value; Description. 0; Any. 1; No Fit. 2; Gradual/Diagonal. 3; Gradual/Transitional. 4; Gradual/Vertical. 5; Gradual/Horizontal. 6; Rapid/Vertical. 7; Rapid/Horizontal. 8; Instant/Vertical. Arm Style. Value; Description. 0; Any. 1; No Fit. 2; Straight Arms/Horizontal. 3; Straight Arms/Wedge. 4; Straight Arms/Vertical. 5; Straight Arms/Single Serif. 6; Straight Arms/Double Serif. 7; Non-Straight Arms/Horizontal. 8; Non-Straight Arms/Wedge. 9; Non-Straight Arms/Vertical. 10; Non-Straight Arms/Single Serif. 11; Non-Straight Arms/Double Serif. Letterform. Value; Description. 0; Any. 1; No Fit. 2; Normal/Contact. 3; Normal/Weighted. 4; Normal/Boxed. 5; Normal/Flattened. 6; Normal/Rounded. 7; Normal/Off Center. 8; Normal/Square. 9; Oblique/Contact. 10; Oblique/Weighted. 11; Oblique/Boxed. 12; Oblique/Flattened. 13; Oblique/Rounded. 14; Oblique/Off Center. 15; Oblique/Square. Midline. Value; Description. 0; Any. 1; No Fit. 2; Standard/Trimmed. 3; Standard/Pointed. 4; Standard/Serifed. 5; High/Trimmed. 6; High/Pointed. 7; High/Serifed. 8; Constant/Trimmed. 9; Constant/Pointed. 10; Constant/Serifed. 11; Low/Trimmed. 12; Low/Pointed. 13; Low/Serifed. X-height. Value; Description. 0; Any. 1; No Fit. 2; Constant/Small. 3; Constant/Standard. 4; Constant/Large. 5; Ducking/Small. 6; Ducking/Standard. 7; Ducking/Large. ulCharRange Format:; 16-byte unsigned long array (4 elements). Title:; Character Range. Description:; This field is split conceptually into two bit fields of 96 and 32 bits each. The low 96 bits are used to specify the Unicode blocks encompassed by the font file. The high 32 bits are used to specify the character or script sets that are covered by the font file. The actual bit assignments are not yet completed; presently, all bits must be set to zero (0).. achVendID Format:; Tag. Title:; Font Vendor Identification. Description:; The four character identifier for the vendor of the given type face.. Comments:; This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. It is reasonable to assume that there will be 6 vendors of ITC Zapf Dingbats for use on desktop platforms in the near future (if not already). It is also likely that the vendors will have other inherent benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor's type to be used over another, possibly inferior, font file. The Vendor ID value is not required. The Vendor ID list can be accessed via the informative reference 6 in the bibliolgraphy.. fsSelection Format:; 2-byte bit field.. Title:; Font selection flags.. Description:; Contains information concerning the nature of the font patterns, as follows:. Bit #; macStyle bit; C definition; Description. 0; bit 1; ITALIC; Font contains Italic characters, otherwise they are upright.. 1; UNDERSCORE; Characters are underscored.. 2; NEGATIVE; Characters have their foreground and background reversed.. 3; OUTLINED; Outline (hollow) characters, otherwise they are solid.. 4; STRIKEOUT; Characters are overstruck.. 5; bit 0; BOLD; Characters are emboldened.. 6; REGULAR; Characters are in the standard weight/style for the font.. Comments:; All undefined bits must be zero. This field contains information on the original design of the font. Bits 0 & 5 can be used to determine if the font was designed with these features or whether some type of machine simulation was performed on the font to achieve this appearance. Bits 1-4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. If bit 6 is set, then bits 0 and 5 must be clear, else the behavior is undefined. As noted above, the settings of bits 0 and 1 must be reflected in the macStyle bits in the 'head' table. While bit 6 on implies that bits 0 and 1 of macStyle are clear (along with bits 0 and 5 of fsSelection), the reverse is not true. Bits 0 and 1 of macStyle (and 0 and 5 of fsSelection) may be clear and that does not give any indication of whether or not bit 6 of fsSelection is clear (e.g., Arial Light would have all bits cleared; it is not the regular version of Arial).. usFirstCharIndex Format:; uint16uint16. Description:; The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020.. usLastCharIndex Format:; 2-byte uint16. Description:; The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports.. sTypoAscender Format:; int16. Description:; The typographic ascender for this font. One good source for sTypoAscender in Latin based fonts is the Ascender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoAscender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatibility requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoAscender is that which describes the top of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout.. sTypoDescender Format:; int16. Description:; The typographic descender for this font. Remember that this is not the same as the Descender value in the 'hhea' table. One good source for sTypoDescender in Latin based fonts is the Descender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoDescender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoDescender is that which describes the bottom of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout.. sTypoLineGap Format:; int16. Description:; The typographic line gap for this font. Remember that this is not the same as the LineGap value in the 'hhea' table. The suggested usage for sTypoLineGap is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. Typical values average 7-10% of units per em. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements (see chapter, "Recommendations for Windows Fonts). These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table.. usWinAscent Format:; uint16. Description:; The ascender metric for Windows. For platform 3 encoding 0 fonts, it is the same as yMax. Windows will clip the bitmap of any portion of a glyph that appears above this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMax. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped.. usWinDescent Format:; uint16. Description:; The descender metric for Windows. For platform 3 encoding 0 fonts, it is the same as -yMin. Windows will clip the bitmap of any portion of a glyph that appears below this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMin. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped..

  - OS/2 - OS/2 and Windows metrics (version 1)

NOTE This is maintained for purposes of being able to validate version 1 OS/2 tables. The OS/2 table consists of a set of metrics that are required in OFF fonts. The layout of version 1 of this table is as follows: Type; Name of Entry; Comments. uint16; version; 0x0001. int16; xAvgCharWidth. uint16; usWeightClass. uint16; usWidthClass. uint16; fsType. int16; ySubscriptXSize. int16; ySubscriptYSize. int16; ySubscriptXOffset. int16; ySubscriptYOffset. int16; ySuperscriptXSize. int16; ySuperscriptYSize. int16; ySuperscriptXOffset. int16; ySuperscriptYOffset. int16; yStrikeoutSize. int16; yStrikeoutPosition. int16; sFamilyClass. uint8; panose[10]. uint32; ulUnicodeRange1; Bits 0-31. uint32; ulUnicodeRange2; Bits 32-63. uint32; ulUnicodeRange3; Bits 64-95. uint32; ulUnicodeRange4; Bits 96-127. Tag; achVendID. uint16; fsSelection. uint16; usFirstCharIndex. uint16; usLastCharIndex. int16; sTypoAscender. int16; sTypoDescender. int16; sTypoLineGap. uint16; usWinAscent. uint16; usWinDescent. uint32; ulCodePageRange1; Bits 0-31. uint32; ulCodePageRange2; Bits 32-63. version Format:; uint16. Units:; n/a. Title:; OS/2 table version number.. Description:; The version number for this OS/2 table.. Comments:; The version number allows for identification of the precise contents and layout for the OS/2 table. The version number for this layout is one (1). The version number for the previous layout (in rev.1.5 of this spec and earlier) was zero (0). Version 0 of the OS/2 table was 78 bytes; Version 1 is 86 bytes, having added the ulCodePageRange1 and ulCodePageRange2 fields.. xAvgCharWidth Format:; int16. Units:; Pels / em units. Title:; Average weighted escapement.. Description:; The Average Character Width parameter specifies the arithmetic average of the escapement (width) of all of the 26 lowercase letters a through z of the Latin alphabet and the space character. If any of the 26 lowercase letters are not present, this parameter should equal the weighted average of all glyphs in the font. For non-UGL (platform 3, encoding 0) fonts, use the unweighted average.. Comments:; This parameter is a descriptive attribute of the font that specifies the spacing of characters for comparing one font to another for selection or substitution. For proportionally spaced fonts, this value is useful in estimating the length for lines of text. The weighting factors provided with this example are only valid for Latin lowercase letters. If other character sets, or capital letters are used, the corresponding frequency of use values should be used. One needs to be careful when comparing fonts that use different frequency of use values for font mapping. The average character width for the following set of upper and lowercase letters only, is calculated according to this formula: Sum the individual character widths multiplied by the following weighting factors and then divide by 1000. For example:. Letter; Weight Factor; Letter; Weight Factor. a; 64; O; 56. b; 14; P; 17. c; 27; Q; 4. d; 35; R; 49. e; 100; S; 56. f; 20; T; 71. g; 14; U; 31. h; 42; V; 10. i; 63; W; 18. j; 3; X; 3. k; 6; Y; 18. l; 35; Z; 2. m; 20; space; 166. n; 56. usWeightClass Format:; uint16. Title:; Weight class.. Description:; Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font.. Comments:. Value; Description; C Definition (from windows.h). 100; Thin; FW_THIN. 200; Extra-light (Ultra-light); FW_EXTRALIGHT. 300; Light; FW_LIGHT. 400; Normal (Regular); FW_NORMAL. 500; Medium; FW_MEDIUM. 600; Semi-bold (Demi-bold); FW_SEMIBOLD. 700; Bold; FW_BOLD. 800; Extra-bold (Ultra-bold); FW_EXTRABOLD. 900; Black (Heavy); FW_BLACK. usWidthClass Format:; uint16. Title:; Width class.. Description:; Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font.. Comments:; Although every character in a font may have a different numeric aspect ratio, each character in a font of normal width has a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the characters in the new font is some percentage greater or less than those same characters in the normal font -- it is this difference that this parameter specifies.. Value; Description; C Definition; % of normal. 1; Ultra-condensed; FWIDTH_ULTRA_CONDENSED; 50. 2; Extra-condensed; FWIDTH_EXTRA_CONDENSED; 62.5. 3; Condensed; FWIDTH_CONDENSED; 75. 4; Semi-condensed; FWIDTH_SEMI_CONDENSED; 87.5. 5; Medium (normal); FWIDTH_NORMAL; 100. 6; Semi-expanded; FWIDTH_SEMI_EXPANDED; 112.5. 7; Expanded; FWIDTH_EXPANDED; 125. 8; Extra-expanded; FWIDTH_EXTRA_EXPANDED; 150. 9; Ultra-expanded; FWIDTH_ULTRA_EXPANDED; 200. fsType Format:; uint16. Title:; Type flags.. Description:; Indicates font embedding licensing rights for the font. Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font. The Font Embedding DLL Specification and DLL release notes describe the APIs used to implement support for OFF font embedding and loading. Applications that implement support for font embedding, either through use of the Font Embedding DLL or through other means, must not embed fonts which are not licensed to permit embedding. Further, applications loading embedded fonts for temporary use (see Preview & Print and Editable embedding below) must delete the fonts when the document containing the embedded font is closed.. Bit; Bit Mask; Description. 0x0000; Installable Embedding: No fsType bit is set. Thus fsType is zero. Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser.. 0; 0x0001; Reserved, must be zero.. 1; 0x0002; Restricted License embedding: Fonts that have only this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: For Restricted License embedding to take effect, it must be the only level of embedding selected.. 2; 0x0004; Preview & Print embedding: When this bit is set, the font may be embedded, and temporarily loaded on the remote system. Documents containing Preview & Print fonts must be opened "read-only;" no edits can be applied to the document.. 3; 0x0008; Editable embedding: When this bit is set, the font may be embedded but must only be installed temporarily on other systems. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened for reading, editing is permitted, and changes may be saved.. 4-15; Reserved, must be zero.. Comments:; If multiple embedding bits are set, the least restrictive license granted takes precedence. For example, if bits 1 and 3 are set, bit 3 takes precedence over bit 1and the font may be embedded with Editable rights. For compatibility purposes, most vendors granting Editable embedding rights are also setting the Preview & Print bit (0x000C). This will permit an application that only supports Preview & Print embedding to detect that font embedding is allowed.. Restricted License embedding (0x0002): Fonts that have this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: NOTE For Restricted License embedding to take effect, it must be the only level of embedding selected (as noted in the previous paragraph).. Preview & Print embedding (0x0004): Fonts with this bit set indicate that they may be embedded within documents but must only be installed temporarily on the remote system. Any document which includes a Preview & Print embedded font must be opened “read-only;” the application must not allow the user to edit the document; it can only be viewed and/or printed.. Editable embedding (0x0008): Fonts with this bit set indicate that they may be embedded in documents, but must only be installed temporarily on the remote system. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened “read-write;” editing is permitted, and changes may be saved.. Installable embedding (0x0000): Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser.. ySubscriptXSize Format:; int16. Units:; Font design units. Title:; Subscript horizontal font size.. Description:; The recommended horizontal size in font design units for subscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript character would be 1/10th the size of the normal character.. ySubscriptYSize Format:; int16. Units:; Font design units. Title:; Subscript vertical font size.. Description:; The recommended vertical size in font design units for subscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The horizontal font size specifies a font designer's recommendation for horizontal font size of subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the characters in a font or by substituting characters from another font, this parameter specifies the recommended horizontal EmInc for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript character would be 1/10th the size of the normal character.. ySubscriptXOffset Format:; int16. Units:; Font design units. Title:; Subscript x offset.. Description:; The recommended horizontal offset in font design untis for subscripts for this font.. Comments:; The Subscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin of the font to the character origin of the subscript's character -- for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters, this parameter specifies the recommended horizontal position from the character escapement point of the last character before the first subscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for subscript characters is usually adjusted to compensate for the angle of incline.. ySubscriptYOffset Format:; int16. Units:; Font design units. Title:; Subscript y offset.. Description:; The recommended vertical offset in font design units from the baseline for subscripts for this font.. Comments:; The Subscript Y offset parameter specifies a font designer's recommended vertical offset from the character baseline to the character baseline for subscript characters associated with this font. Values are expressed as a positive offset below the character baseline. If a font does not include all of the required subscript for an application, this parameter specifies the recommended vertical distance below the character baseline for those subscript characters.. ySuperscriptXSize Format:; int16. Units:; Font design units. Title:; Superscript horizontal font size.. Description:; The recommended horizontal size in font design units for superscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript character would be 1/10th the size of the normal character.. ySuperscriptYSize Format:; int16. Units:; Font design units. Title:; Superscript vertical font size.. Description:; The recommended vertical size in font design units for superscripts for this font.. Comments:; If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The vertical font size specifies a font designer's recommended vertical font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended EmHeight for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript character would be 1/10th the size of the normal character.. ySuperscriptXOffset Format:; int16. Units:; Font design units. Title:; Superscript x offset.. Description:; The recommended horizontal offset in font design units for superscripts for this font.. Comments:; The Superscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin to the superscript character's origin for the superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended horizontal position from the escapement point of the character before the first superscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for superscript characters is usually adjusted to compensate for the angle of incline.. ySuperscriptYOffset Format:; int16. Units:; Font design units. Title:; Superscript y offset.. Description:; The recommended vertical offset in font design units from the baseline for superscripts for this font.. Comments:; The Superscript Y offset parameter specifies a font designer's recommended vertical offset -- from the character baseline to the superscript character's baseline associated with this font. Values for this parameter are expressed as a positive offset above the character baseline. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended vertical distance above the character baseline for those superscript characters.. yStrikeoutSize Format:; int16. Units:; Font design units. Title:; Strikeout size.. Description:; Width of the strikeout stroke in font design units.. Comments:; This field should normally be the width of the em dash for the current font. If the size is one, the strikeout line will be the line represented by the strikeout position field. If the value is two, the strikeout line will be the line represented by the strikeout position and the line immediately above the strikeout position. For a Roman font with a 2048 em square, 102 is suggested.. yStrikeoutPosition Format:; int16. Units:; Font design units. Title:; Strikeout position.. Description:; The position of the top of the strikeout stroke relative to the baseline in font design units.. Comments:; Positive values represent distances above the baseline, while negative values represent distances below the baseline. A value of zero falls directly on the baseline, while a value of one falls one pel above the baseline. The value of strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. For a Roman font with a 2048 em square, 460 is suggested.. sFamilyClass Format:; int16. Title:; Font-family class and subclass.. Description:; This parameter is a classification of font-family design.. Comments:; The font class and font subclass are registered values assigned by IBM to each font family. This parameter is intended for use in selecting an alternate font when the requested font is not available. The font class is the most general and the font subclass is the most specific. The high byte of this field contains the family class, while the low byte contains the family subclass.. Panose Format:; 10 byte array. Title:; PANOSE classification number. International:; Additional specifications are required for PANOSE to classify non-Latin character sets.. Description:; This 10 byte series of numbers is used to describe the visual characteristics of a given typeface. If provided, these characteristics are then used to associate the font with other fonts of similar appearance having different names; the default values should be set to 'zero'. The variables for each digit are listed below.. Comments:; The specification for assigning PANOSE values [14] can be found in bibliography and is maintained by Monotype Imaging Inc.. PANOSE. Type; Name; Description. uint8; bFamilyType;; Family Type. uint8; bSerifStyle;; Serif Style. uint8; bWeight;; Weight. uint8; bProportion;; Proportion. uint8; bContrast;; Contrast. uint8; bStrokeVariation;; Stroke Variation. uint8; bArmStyle;; Arm Style. uint8; bLetterform;; Letterform. uint8; bMidline;; Midline. uint8; bXHeight;; X Height. Family Type. Value; Description. 0; Any. 1; No Fit. 2; Text and Display. 3; Script. 4; Decorative. 5; Pictorial. Serif Style. Value; Description. 0; Any. 1; No Fit. 2; Cove. 3; Obtuse Cove. 4; Square Cove. 5; Obtuse Square Cove. 6; Square. 7; Thin. 8; Bone. 9; Exaggerated. 10; Triangle. 11; Normal Sans. 12; Obtuse Sans. 13; Perp Sans. 14; Flared. 15; Rounded. Weight. Value; Description. 0; Any. 1; No Fit. 2; Very Light. 3; Light. 4; Thin. 5; Book. 6; Medium. 7; Demi. 8; Bold. 9; Heavy. 10; Black. 11; Nord. Proportion. Value; Description. 0; Any. 1; No Fit. 2; Old Style. 3; Modern. 4; Even Width. 5; Expanded. 6; Condensed. 7; Very Expanded. 8; Very Condensed. 9; Monospaced. Contrast. Value; Description. 0; Any. 1; No Fit. 2; None. 3; Very Low. 4; Low. 5; Medium Low. 6; Medium. 7; Medium High. 8; High. 9; Very High. Stroke Variation. Value; Description. 0; Any. 1; No Fit. 2; Gradual/Diagonal. 3; Gradual/Transitional. 4; Gradual/Vertical. 5; Gradual/Horizontal. 6; Rapid/Vertical. 7; Rapid/Horizontal. 8; Instant/Vertical. Arm Style. Value; Description. 0; Any. 1; No Fit. 2; Straight Arms/Horizontal. 3; Straight Arms/Wedge. 4; Straight Arms/Vertical. 5; Straight Arms/Single Serif. 6; Straight Arms/Double Serif. 7; Non-Straight Arms/Horizontal. 8; Non-Straight Arms/Wedge. 9; Non-Straight Arms/Vertical. 10; Non-Straight Arms/Single Serif. 11; Non-Straight Arms/Double Serif. Letterform. Value; Description. 0; Any. 1; No Fit. 2; Normal/Contact. 3; Normal/Weighted. 4; Normal/Boxed. 5; Normal/Flattened. 6; Normal/Rounded. 7; Normal/Off Center. 8; Normal/Square. 9; Oblique/Contact. 10; Oblique/Weighted. 11; Oblique/Boxed. 12; Oblique/Flattened. 13; Oblique/Rounded. 14; Oblique/Off Center. 15; Oblique/Square. Midline. Value; Description. 0; Any. 1; No Fit. 2; Standard/Trimmed. 3; Standard/Pointed. 4; Standard/Serifed. 5; High/Trimmed. 6; High/Pointed. 7; High/Serifed. 8; Constant/Trimmed. 9; Constant/Pointed. 10; Constant/Serifed. 11; Low/Trimmed. 12; Low/Pointed. 13; Low/Serifed. X-height. Value; Description. 0; Any. 1; No Fit. 2; Constant/Small. 3; Constant/Standard. 4; Constant/Large. 5; Ducking/Small. 6; Ducking/Standard. 7; Ducking/Large. ulUnicodeRange1 (Bits 0-31) ulUnicodeRange2 (Bits 32-63) ulUnicodeRange3 (Bits 64-95) ulUnicodeRange4 (Bits 96-127) Format:; 32-bit unsigned long(4 copies) totaling 128 bits.. Title:; Unicode Character Range. Description:; This field is used to specify the Unicode blocks or ranges encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Microsoft platform). If the bit is set (1) then the Unicode range is considered functional. If the bit is clear (0) then the range is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by ranges if at all possible. All reserved fields must be zero. Each long is in Big-Endian form. See the Basic Multilingual Plane of ISO/IEC 10646 or the Unicode Standard for the list of Unicode ranges and characters.. Bit; Description. 0; Basic Latin. 1; Latin-1 Supplement. 2; Latin Extended-A. 3; Latin Extended-B. 4; IPA Extensions. 5; Spacing Modifier Letters. 6; Combining Diacritical Marks. 7; Basic Greek. 8; Greek Symbols and Coptic. 9; Cyrillic. 10; Armenian. 11; Basic Hebrew. 12; Hebrew Extended (A and B blocks combined). 13; Basic Arabic. 14; Arabic Extended. 15; Devanagari. 16; Bengali. 17; Gurmukhi. 18; Gujarati. 19; Oriya. 20; Tamil. 21; Telugu. 22; Kannada. 23; Malayalam. 24; Thai. 25; Lao. 26; Basic Georgian. 27; Georgian Extended. 28; Hangul Jamo. 29; Latin Extended Additional. 30; Greek Extended. 31; General Punctuation. 32; Superscripts And Subscripts. 33; Currency Symbols. 34; Combining Diacritical Marks For Symbols. 35; Letterlike Symbols. 36; Number Forms. 37; Arrows. 38; Mathematical Operators. 39; Miscellaneous Technical. 40; Control Pictures. 41; Optical Character Recognition. 42; Enclosed Alphanumerics. 43; Box Drawing. 44; Block Elements. 45; Geometric Shapes. 46; Miscellaneous Symbols. 47; Dingbats. 48; CJK Symbols And Punctuation. 49; Hiragana. 50; Katakana. 51; Bopomofo. 52; Hangul Compatibility Jamo. 53; CJK Miscellaneous. 54; Enclosed CJK Letters And Months. 55; CJK Compatibility. 56; Hangul. 57; Reserved for Unicode SubRanges. 58; Reserved for Unicode SubRanges. 59; CJK Unified Ideographs. 60; Private Use Area. 61; CJK Compatibility Ideographs. 62; Alphabetic Presentation Forms. 63; Arabic Presentation Forms-A. 64; Combining Half Marks. 65; CJK Compatibility Forms. 66; Small Form Variants. 67; Arabic Presentation Forms-B. 68; Halfwidth And Fullwidth Forms. 69; Specials. 70-127; Reserved for Unicode SubRanges. achVendID Format:; Tag. Title:; Font Vendor Identification. Description:; The four character identifier for the vendor of the given type face.. Comments:; This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. It is reasonable to assume that there will be 6 vendors of ITC Zapf Dingbats for use on desktop platforms in the near future (if not already). It is also likely that the vendors will have other inherent benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor's type to be used over another, possibly inferior, font file. The Vendor ID value is not required. The Vendor ID list can be accessed via the informative reference 6 in the bibliolgraphy.. fsSelection Format:; 2-byte bit field.. Title:; Font selection flags.. Description:; Contains information concerning the nature of the font patterns, as follows:. Bit #; macStyle bit; C definition; Description. 0; bit 1; ITALIC; Font contains Italic characters, otherwise they are upright.. 1; UNDERSCORE; Characters are underscored.. 2; NEGATIVE; Characters have their foreground and background reversed.. 3; OUTLINED; Outline (hollow) characters, otherwise they are solid.. 4; STRIKEOUT; Characters are overstruck.. 5; bit 0; BOLD; Characters are emboldened.. 6; REGULAR; Characters are in the standard weight/style for the font.. Comments:; All undefined bits must be zero. This field contains information on the original design of the font. Bits 0 & 5 can be used to determine if the font was designed with these features or whether some type of machine simulation was performed on the font to achieve this appearance. Bits 1-4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. If bit 6 is set, then bits 0 and 5 must be clear, else the behavior is undefined. As noted above, the settings of bits 0 and 1 must be reflected in the macStyle bits in the 'head' table. While bit 6 on implies that bits 0 and 1 of macStyle are clear (along with bits 0 and 5 of fsSelection), the reverse is not true. Bits 0 and 1 of macStyle (and 0 and 5 of fsSelection) may be clear and that does not give any indication of whether or not bit 6 of fsSelection is clear (e.g., Arial Light would have all bits cleared; it is not the regular version of Arial).. usFirstCharIndex Format:; 2-byte USHORT. Description:; The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020.. usLastCharIndex Format:; uint16. Description:; The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports.. sTypoAscender Format:; int16. Description:; The typographic ascender for this font. Remember that this is not the same as the Ascender value in the 'hhea' table. One good source for sTypoAscender in Latin based fonts is the Ascender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoAscender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatibility requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoAscender is that which describes the top of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout.. sTypoDescender Format:; int16. Description:; The typographic descender for this font. Remember that this is not the same as the Descender value in the 'hhea' table One good source for sTypoDescender in Latin based fonts is the Descender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoDescender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoDescender is that which describes the bottom of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout.. sTypoLineGap Format:; int16. Description:; The typographic line gap for this font. Remember that this is not the same as the LineGap value in the 'hhea' table. The suggested usage for sTypoLineGap is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. Typical values average 7-10% of units per em. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements (see chapter, "Recommendations for Windows Fonts). These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table.. usWinAscent Format:; uint16. Description:; The ascender metric for Windows. For platform 3 encoding 0 fonts, it is the same as yMax. Windows will clip the bitmap of any portion of a glyph that appears above this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMax. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped.. usWinDescent Format:; uint16. Description:; The descender metric for Windows. For platform 3 encoding 0 fonts, it is the same as -yMin. Windows will clip the bitmap of any portion of a glyph that appears below this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMin. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped.. ulCodePageRange1 Bits 0-31 ulCodePageRange2 Bits 32-63 Format:; 32-bit unsigned long(2 copies) totaling 64 bits.. Title:; Code Page Character Range. Description:; This field is used to specify the code pages encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Microsoft platform). If the font file is encoding ID 0, then the Symbol Character Set bit should be set. If the bit is set (1) then the code page is considered functional. If the bit is clear (0) then the code page is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by code pages if at all possible. Symbol character sets have a special meaning. If the symbol bit (31) is set, and the font file contains a 'cmap' subtable for platform of 3 and encoding ID of 1, then all of the characters in the Unicode range 0xF000 - 0xF0FF (inclusive) will be used to enumerate the symbol character set. If the bit is not set, any characters present in that range will not be enumerated as a symbol character set. All reserved fields must be zero. Each long is in Big-Endian form.. Bit; Code Page; Description. 0; 1252; Latin 1. 1; 1250; Latin 2: Eastern Europe. 2; 1251; Cyrillic. 3; 1253; Greek. 4; 1254; Turkish. 5; 1255; Hebrew. 6; 1256; Arabic. 7; 1257; Windows Baltic. 8-15; Reserved for Alternate ANSI. 16; 874; Thai. 17; 932; JIS/Japan. 18; 936; Chinese: Simplified chars--PRC and Singapore. 19; 949; Korean Wansung. 20; 950; Chinese: Traditional chars--Taiwan and Hong Kong. 21; 1361; Korean Johab. 22-28; Reserved for Alternate ANSI & OEM. 29; Macintosh Character Set (US Roman). 30; OEM Character Set. 31; Symbol Character Set. 32-47; Reserved for OEM. 48; 869; IBM Greek. 49; 866; MS-DOS Russian. 50; 865; MS-DOS Nordic. 51; 864; Arabic. 52; 863; MS-DOS Canadian French. 53; 862; Hebrew. 54; 861; MS-DOS Icelandic. 55; 860; MS-DOS Portuguese. 56; 857; IBM Turkish. 57; 855; IBM Cyrillic; primarily Russian. 58; 852; Latin 2. 59; 775; MS-DOS Baltic. 60; 737; Greek; former 437 G. 61; 708; Arabic; ASMO 708. 62; 850; WE/Latin 1. 63; 437; US.

  - OS/2 - OS/2 and Windows metrics (version 2)

NOTE	This is maintained for purposes of being able to validate version 2 OS/2 tables.

The OS/2 table consists of a set of metrics that are required in OFF fonts. There are two versions of this table, the second version having five additional fields: sxHeight, sCapHeight, usDefaultChar, usBreakChar, usMaxContext. The layout of version 2 of this table is as follows:

| Type | Name of Entry | Comments |
| --- | --- | --- |
| uint16 | Version | 0x0002 |
| int16 | xAvgCharWidth |  |
| uint16 | usWeightClass |  |
| uint16 | usWidthClass |  |
| uint16 | fsType |  |
| int16 | ySubscriptXSize |  |
| int16 | ySubscriptYSize |  |
| int16 | ySubscriptXOffset |  |
| int16 | ySubscriptYOffset |  |
| int16 | ySuperscriptXSize |  |
| int16 | ySuperscriptYSize |  |
| int16 | ySuperscriptXOffset |  |
| int16 | ySuperscriptYOffset |  |
| int16 | yStrikeoutSize |  |
| int16 | yStrikeoutPosition |  |
| int16 | sFamilyClass |  |
| uint8 | Panose[10] |  |
| uint32 | ulUnicodeRange1 | Bits 0-31 |
| uint32 | ulUnicodeRange2 | Bits 32-63 |
| uint32 | ulUnicodeRange3 | Bits 64-95 |
| uint32 | ulUnicodeRange4 | Bits 96-127 |
| Tag | achVendID |  |
| uint16 | fsSelection |  |
| uint16 | usFirstCharIndex |  |
| uint16 | usLastCharIndex |  |
| int16 | sTypoAscender |  |
| int16 | sTypoDescender |  |
| int16 | sTypoLineGap |  |
| uint16 | usWinAscent |  |
| uint16 | usWinDescent |  |
| uint32 | ulCodePageRange1 | Bits 0-31 |
| uint32 | ulCodePageRange2 | Bits 32-63 |
| int16 | sxHeight |  |
| int16 | sCapHeight |  |
| uint16 | usDefaultChar |  |
| uint16 | usBreakChar |  |
| uint16 | usMaxContext |  |

#### version

| Format: | uint16 |
| --- | --- |
| Units: | n/a |
| Title: | OS/2 table version number. |
| Description: | The version number for this OS/2 table. |
| Comments: | The version number allows for identification of the precise contents and layout for the OS/2 table. The version number for this layout is two (2). Versions one (1) and zero (0) have been used previously, in rev.1.66 and 1.5, respectively, of the TrueType specification. |

#### xAvgCharWidth

| Format: |  |  | int16 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Units: |  |  | Pels / em units |  |  |  |
| Title: |  |  | Average weighted escapement. |  |  |  |
| Description: |  |  | The Average Character Width parameter specifies the arithmetic average of the escapement (width) of all of the 26 lowercase letters a through z of the Latin alphabet and the space character. If any of the 26 lowercase letters are not present, this parameter should equal the weighted average of all glyphs in the font. For non-UGL (platform 3, encoding 0) fonts, use the unweighted average. |  |  |  |
| Comments: |  |  | This parameter is a descriptive attribute of the font that specifies the spacing of characters for comparing one font to another for selection or substitution. For proportionally spaced fonts, this value is useful in estimating the length for lines of text. The weighting factors provided with this example are only valid for Latin lowercase letters. If other character sets, or capital letters are used, the corresponding frequency of use values should be used. One needs to be careful when comparing fonts that use different frequency of use values for font mapping. The average character width for the following set of upper and lowercase letters only, is calculated according to this formula: Sum the individual character widths multiplied by the following weighting factors and then divide by 1000. For example: |  |  |  |
| Letter | Weight Factor |  | Letter | Weight Factor |
| a | 64 |  | O | 56 |
| b | 14 |  | P | 17 |
| c | 27 |  | Q | 4 |
| d | 35 |  | R | 49 |
| e | 100 |  | S | 56 |
| f | 20 |  | T | 71 |
| g | 14 |  | U | 31 |
| h | 42 |  | V | 10 |
| i | 63 |  | W | 18 |
| j | 3 |  | X | 3 |
| k | 6 |  | Y | 18 |
| l | 35 |  | Z | 2 |
| m | 20 |  | space | 166 |
| n | 56 |  |  |  |

#### usWeightClass

| Format: | uint16 |
| --- | --- |
| Title: | Weight class. |
| Description: | Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font. |
| Comments: |  |

| Value | Description | C Definition (from windows.h) |
| --- | --- | --- |
| 100 | Thin | FW_THIN |
| 200 | Extra-light (Ultra-light) | FW_EXTRALIGHT |
| 300 | Light | FW_LIGHT |
| 400 | Normal (Regular) | FW_NORMAL |
| 500 | Medium | FW_MEDIUM |
| 600 | Semi-bold (Demi-bold) | FW_SEMIBOLD |
| 700 | Bold | FW_BOLD |
| 800 | Extra-bold (Ultra-bold) | FW_EXTRABOLD |
| 900 | Black (Heavy) | FW_BLACK |

#### usWidthClass

| Format: | uint16 |
| --- | --- |
| Title: | Width class. |
| Description: | Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font. |
| Comments: | Although every character in a font may have a different numeric aspect ratio, each character in a font of normal width has a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the characters in the new font is some percentage greater or less than those same characters in the normal font -- it is this difference that this parameter specifies. |

| Value | Description | C Definition | % of normal |
| --- | --- | --- | --- |
| 1 | Ultra-condensed | FWIDTH_ULTRA_CONDENSED | 50 |
| 2 | Extra-condensed | FWIDTH_EXTRA_CONDENSED | 62.5 |
| 3 | Condensed | FWIDTH_CONDENSED | 75 |
| 4 | Semi-condensed | FWIDTH_SEMI_CONDENSED | 87.5 |
| 5 | Medium (normal) | FWIDTH_NORMAL | 100 |
| 6 | Semi-expanded | FWIDTH_SEMI_EXPANDED | 112.5 |
| 7 | Expanded | FWIDTH_EXPANDED | 125 |
| 8 | Extra-expanded | FWIDTH_EXTRA_EXPANDED | 150 |
| 9 | Ultra-expanded | FWIDTH_ULTRA_EXPANDED | 200 |

#### fsType

| Format: | uint16 |
| --- | --- |
| Title: | Type flags. |
| Description: | Indicates font embedding licensing rights for the font. Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font. The OFF Font Embedding DLL Applications that implement support for font embedding, either through use of the Font Embedding DLL or through other means, must not embed fonts which are not licensed to permit embedding. Further, applications loading embedded fonts for temporary use (see Preview & Print and Editable embedding below) must delete the fonts when the document containing the embedded font is closed. |

| Bit | Bit Mask | Description |
| --- | --- | --- |
|  | 0x0000 | Installable Embedding: No fsType bit is set. Thus fsType is zero. Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser. |
| 0 | 0x0001 | Reserved, must be zero. |
| 1 | 0x0002 | Restricted License embedding: Fonts that have only this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: For Restricted License embedding to take effect, it must be the only level of embedding selected. |
| 2 | 0x0004 | Preview & Print embedding: When this bit is set, the font may be embedded, and temporarily loaded on the remote system. Documents containing Preview & Print fonts must be opened "read-only;" no edits can be applied to the document. |
| 3 | 0x0008 | Editable embedding: When this bit is set, the font may be embedded but must only be installed temporarily on other systems. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened for reading, editing is permitted, and changes may be saved. |
| 4-7 |  | Reserved, must be zero. |
| 8 | 0x0100 | No subsetting: When this bit is set, the font may not be subsetted prior to embedding. Other embedding restrictions specified in bits 0-3 and 9 also apply. |
| 9 | 0x0200 | Bitmap embedding only: When this bit is set, only bitmaps contained in the font may be embedded. No outline data may be embedded. If there are no bitmaps available in the font, then the font is considered unembeddable and the embedding services will fail. Other embedding restrictions specified in bits 0-3 and 8 also apply. |
| 10-15 |  | Reserved, must be zero. |

Comments: 	If multiple embedding bits are set, the least restrictive license granted takes precedence. For example, if bits 1 and 3 are set, bit 3 takes precedence over bit 1and the font may be embedded with Editable rights. For compatibility purposes, most vendors granting Editable embedding rights are also setting the Preview & Print bit (0x000C). This will permit an application that only supports Preview & Print embedding to detect that font embedding is allowed.

#### ySubscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript vertical font size. |
| Description: | The recommended vertical size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The horizontal font size specifies a font designer's recommendation for horizontal font size of subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the characters in a font or by substituting characters from another font, this parameter specifies the recommended horizontal EmInc for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript x offset. |
| Description: | The recommended horizontal offset in font design untis for subscripts for this font. |
| Comments: | The Subscript X Offset parameter specifies a font designer's recommended horizontal offset -- from the character origin of the font to the character origin of the subscript's character -- for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters, this parameter specifies the recommended horizontal position from the character escapement point of the last character before the first subscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for subscript characters is usually adjusted to compensate for the angle of incline. |

#### ySubscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for subscripts for this font. |
| Comments: | The Subscript Y Offset parameter specifies a font designer's recommended vertical offset from the character baseline to the character baseline for subscript characters associated with this font. Values are expressed as a positive offset below the character baseline. If a font does not include all of the required subscript for an application, this parameter specifies the recommended vertical distance below the character baseline for those subscript characters. |

#### ySuperscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript vertical font size. |
| Description: | The recommended vertical size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The vertical font size specifies a font designer's recommended vertical font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended EmHeight for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript x offset. |
| Description: | The recommended horizontal offset in font design units for superscripts for this font. |
| Comments: | The Superscript X Offset parameter specifies a font designer's recommended horizontal offset -- from the character origin to the superscript character's origin for the superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended horizontal position from the escapement point of the character before the first superscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for superscript characters is usually adjusted to compensate for the angle of incline. |

#### ySuperscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for superscripts for this font. |
| Comments: | The Superscript Y Offset parameter specifies a font designer's recommended vertical offset -- from the character baseline to the superscript character's baseline associated with this font. Values for this parameter are expressed as a positive offset above the character baseline. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended vertical distance above the character baseline for those superscript characters. |

#### yStrikeoutSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout size. |
| Description: | Width of the strikeout stroke in font design units. |
| Comments: | This field should normally be the width of the em dash for the current font. If the size is one, the strikeout line will be the line represented by the strikeout position field. If the value is two, the strikeout line will be the line represented by the strikeout position and the line immediately above the strikeout position. For a Roman font with a 2048 em square, 102 is suggested. |

#### yStrikeoutPosition

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout position. |
| Description: | The position of the top of the strikeout stroke relative to the baseline in font design units. |
| Comments: | Positive values represent distances above the baseline, while negative values represent distances below the baseline. A value of zero falls directly on the baseline, while a value of one falls one pel above the baseline. The value of strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. For a Roman font with a 2048 em square, 460 is suggested. |

#### sFamilyClass

| Format: | int16 |
| --- | --- |
| Title: | Font-family class and subclass. |
| Description: | This parameter is a classification of font-family design. |
| Comments: | The font class and font subclass are registered values per Annex A. the to each font family. This parameter is intended for use in selecting an alternate font when the requested font is not available. The font class is the most general and the font subclass is the most specific. The high byte of this field contains the family class, while the low byte contains the family subclass. |

#### Panose

| Format: | 10 byte array |
| --- | --- |
| Title: | PANOSE classification number |
| International: | Additional specifications are required for PANOSE to classify non-Latin character sets. |
| Description: | This 10-byte series of numbers is used to describe the visual characteristics of a given typeface. If provided, these characteristics are then used to associate the font with other fonts of similar appearance having different names; the default values should be set to 'zero'. The variables for each digit are listed below. |
| Comments: | The specification for assigning PANOSE values [14] can be found in bibliography and is maintained by Monotype Imaging Inc. |

| Type | Name |
| --- | --- |
| uint8 | bFamilyType; |
| uint8 | bSerifStyle; |
| uint8 | bWeight; |
| uint8 | bProportion; |
| uint8 | bContrast; |
| uint8 | bStrokeVariation; |
| uint8 | bArmStyle; |
| uint8 | bLetterform; |
| uint8 | bMidline; |
| uint8 | bXHeight; |

#### ulUnicodeRange

ulUnicodeRange1 (Bits 0-31)
ulUnicodeRange2 (Bits 32-63)
ulUnicodeRange3 (Bits 64-95)
ulUnicodeRange4 (Bits 96-127)

| Format: | 32-bit unsigned long(4 copies) totaling 128 bits. |
| --- | --- |
| Title: | Unicode Character Range |
| Description: | This field is used to specify the Unicode blocks or ranges encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform). If the bit is set (1) then the Unicode range is considered functional. If the bit is clear (0) then the range is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by ranges if at all possible. All reserved fields must be zero. Each long is in Big-Endian form. See the Basic Multilingual Plane of ISO/IEC 10646 or the Unicode Standard for the list of Unicode ranges and characters. |

| Bit | Description |
| --- | --- |
| 0 | Basic Latin |
| 1 | Latin-1 Supplement |
| 2 | Latin Extended-A |
| 3 | Latin Extended-B |
| 4 | IPA Extensions |
| 5 | Spacing Modifier Letters |
| 6 | Combining Diacritical Marks |
| 7 | Greek and Coptic |
| 8 | Reserved for Unicode SubRanges |
| 9 | Cyrillic |
|  | Cyrillic Supplementary |
| 10 | Armenian |
| 11 | Hebrew |
| 12 | Reserved for Unicode SubRanges |
| 13 | Arabic |
| 14 | Reserved for Unicode SubRanges |
| 15 | Devanagari |
| 16 | Bengali |
| 17 | Gurmukhi |
| 18 | Gujarati |
| 19 | Oriya |
| 20 | Tamil |
| 21 | Telugu |
| 22 | Kannada |
| 23 | Malayalam |
| 24 | Thai |
| 25 | Lao |
| 26 | Georgian |
| 27 | Reserved for Unicode SubRanges |
| 28 | Hangul Jamo |
| 29 | Latin Extended Additional |
| 30 | Greek Extended |
| 31 | General Punctuation |
| 32 | Superscripts And Subscripts |
| 33 | Currency Symbols |
| 34 | Combining Diacritical Marks For Symbols |
| 35 | Letterlike Symbols |
| 36 | Number Forms |
| 37 | Arrows |
|  | Supplemental Arrows-A |
|  | Supplemental Arrows-B |
| 38 | Mathematical Operators |
|  | Supplemental Mathematical Operators |
|  | Miscellaneous Mathematical Symbols-A |
|  | Miscellaneous Mathematical Symbols-B |
| 39 | Miscellaneous Technical |
| 40 | Control Pictures |
| 41 | Optical Character Recognition |
| 42 | Enclosed Alphanumerics |
| 43 | Box Drawing |
| 44 | Block Elements |
| 45 | Geometric Shapes |
| 46 | Miscellaneous Symbols |
| 47 | Dingbats |
| 48 | CJK Symbols And Punctuation |
| 49 | Hiragana |
| 50 | Katakana |
|  | Katakana Phonetic Extensions |
| 51 | Bopomofo |
|  | Bopomofo Extended |
| 52 | Hangul Compatibility Jamo |
| 53 | Reserved for Unicode SubRanges |
| 54 | Enclosed CJK Letters And Months |
| 55 | CJK Compatibility |
| 56 | Hangul Syllables |
| 57 | Non-Plane 0 * |
| 58 | Reserved for Unicode SubRanges |
| 59 | CJK Unified Ideographs |
|  | CJK Radicals Supplement |
|  | Kangxi Radicals |
|  | Ideographic Description |
|  | CJK Unified Ideograph Extension A |
| 60 | Private Use Area |
| 61 | CJK Compatibility Ideographs |
| 62 | Alphabetic Presentation Forms |
| 63 | Arabic Presentation Forms-A |
| 64 | Combining Half Marks |
| 65 | CJK Compatibility Forms |
| 66 | Small Form Variants |
| 67 | Arabic Presentation Forms-B |
| 68 | Halfwidth And Fullwidth Forms |
| 69 | Specials |
| 70 | Tibetan |
| 71 | Syriac |
| 72 | Thaana |
| 73 | Sinhala |
| 74 | Myanmar |
| 75 | Ethiopic |
| 76 | Cherokee |
| 77 | Unified Canadian Aboriginal Syllabics |
| 78 | Ogham |
| 79 | Runic |
| 80 | Khmer |
| 81 | Mongolian |
| 82 | Braille Patterns |
| 83 | Yi |
|  | Yi Radicals |
| 84-127 | Reserved for Unicode SubRanges |

NOTE	* Setting bit 57 implies that there is at least one codepoint beyond the Basic Multilingual Plane that is supported by this font.

#### achVendID

| Format: | Tag |
| --- | --- |
| Title: | Font Vendor Identification |
| Description: | The four character identifier for the vendor of the given type face. |
| Comments: | This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. It is reasonable to assume that there will be 6 vendors of ITC Zapf Dingbats for use on desktop platforms in the near future (if not already). It is also likely that the vendors will have other inherent benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor's type to be used over another, possibly inferior, font file. The Vendor ID value is not required. The Vendor ID list can be accessed via the Reference [6] in the bibliolgraphy. |

#### fsSelection

| Format: | 2-byte bit field. |
| --- | --- |
| Title: | Font selection flags. |
| Description: | Contains information concerning the nature of the font patterns, as follows: |

| Bit # | macStyle bit | C definition | Description |
| --- | --- | --- | --- |
| 0 | bit 1 | ITALIC | Font contains Italic characters, otherwise they are upright. |
| 1 |  | UNDERSCORE | Characters are underscored. |
| 2 |  | NEGATIVE | Characters have their foreground and background reversed. |
| 3 |  | OUTLINED | Outline (hollow) characters, otherwise they are solid. |
| 4 |  | STRIKEOUT | Characters are overstruck. |
| 5 | bit 0 | BOLD | Characters are emboldened. |
| 6 |  | REGULAR | Characters are in the standard weight/style for the font. |

| Comments: | All undefined bits must be zero. This field contains information on the original design of the font. Bits 0 & 5 can be used to determine if the font was designed with these features or whether some type of machine simulation was performed on the font to achieve this appearance. Bits 1-4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. If bit 6 is set, then bits 0 and 5 must be clear, else the behavior is undefined. As noted above, the settings of bits 0 and 1 must be reflected in the macStyle bits in the 'head' table. While bit 6 on implies that bits 0 and 1 of macStyle are clear (along with bits 0 and 5 of fsSelection), the reverse is not true. Bits 0 and 1 of macStyle (and 0 and 5 of fsSelection) may be clear and that does not give any indication of whether or not bit 6 of fsSelection is clear (e.g., Arial Light would have all bits cleared; it is not the regular version of Arial). |
| --- | --- |

#### usFirstCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020. |

#### usLastCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports. |

#### sTypoAscender

| Format: | int16 |
| --- | --- |
| Description: | The typographic ascender for this font. Remember that this is not the same as the Ascender value in the 'hhea' table, . One good source for sTypoAscender in Latin based fonts is the Ascender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoAscender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatibility requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoAscender is that which describes the top of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoDescender

| Format: | int16 |
| --- | --- |
| Description: | The typographic descender for this font. Remember that this is not the same as the Descender value in the 'hhea' table. One good source for sTypoDescender in Latin based fonts is the Descender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoDescender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoDescender is that which describes the bottom of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoLineGap

| Format: | int16 |
| --- | --- |
| Description: | The typographic line gap for this font. Remember that this is not the same as the LineGap value in the 'hhea' table. The suggested usage for sTypoLineGap is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. Typical values average 7-10% of units per em. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements (see clause 7, "Recommendations for OFF Fonts"). These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. These metrics will be exposed through Windows APIs. Macintosh applications will need to access the 'sfnt' resource and parse it to extract this data from the "OS/2" table. |

#### usWinAscent

| Format: | uint16 |
| --- | --- |
| Description: | The ascender metric for Windows. For platform 3 encoding 0 fonts, it is the same as yMax. Windows will clip the bitmap of any portion of a glyph that appears above this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMax. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### usWinDescent

| Format: | uint16 |
| --- | --- |
| Description: | The descender metric for Windows. For platform 3 encoding 0 fonts, it is the same as -yMin. Windows will clip the bitmap of any portion of a glyph that appears below this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMin. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### ulCodePageRange

ulCodePageRange1 Bits 0-31
ulCodePageRange2 Bits 32-63

| Format: | 32-bit unsigned long (2 copies) totaling 64 bits. |
| --- | --- |
| Title: | Code Page Character Range |
| Description: | This field is used to specify the code pages encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform). If the font file is encoding ID 0, then the Symbol Character Set bit should be set. If the bit is set (1) then the code page is considered functional. If the bit is clear (0) then the code page is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by code pages if at all possible. Symbol character sets have a special meaning. If the symbol bit (31) is set, and the font file contains a 'cmap' subtable for platform of 3 and encoding ID of 1, then all of the characters in the Unicode range 0xF000 - 0xF0FF (inclusive) will be used to enumerate the symbol character set. If the bit is not set, any characters present in that range will not be enumerated as a symbol character set. All reserved fields must be zero. Each long is in Big-Endian form. |

| Bit | Code Page | Description |
| --- | --- | --- |
| 0 | 1252 | Latin 1 |
| 1 | 1250 | Latin 2: Eastern Europe |
| 2 | 1251 | Cyrillic |
| 3 | 1253 | Greek |
| 4 | 1254 | Turkish |
| 5 | 1255 | Hebrew |
| 6 | 1256 | Arabic |
| 7 | 1257 | Windows Baltic |
| 8 | 1258 | Vietnamese |
| 9-15 |  | Reserved for Alternate ANSI |
| 16 | 874 | Thai |
| 17 | 932 | JIS/Japan |
| 18 | 936 | Chinese: Simplified chars--PRC and Singapore |
| 19 | 949 | Korean Wansung |
| 20 | 950 | Chinese: Traditional chars--Taiwan and Hong Kong |
| 21 | 1361 | Korean Johab |
| 22-28 |  | Reserved for Alternate ANSI & OEM |
| 29 |  | Macintosh Character Set (US Roman) |
| 30 |  | OEM Character Set |
| 31 |  | Symbol Character Set |
| 32-47 |  | Reserved for OEM |
| 48 | 869 | IBM Greek |
| 49 | 866 | MS-DOS Russian |
| 50 | 865 | MS-DOS Nordic |
| 51 | 864 | Arabic |
| 52 | 863 | MS-DOS Canadian French |
| 53 | 862 | Hebrew |
| 54 | 861 | MS-DOS Icelandic |
| 55 | 860 | MS-DOS Portuguese |
| 56 | 857 | IBM Turkish |
| 57 | 855 | IBM Cyrillic; primarily Russian |
| 58 | 852 | Latin 2 |
| 59 | 775 | MS-DOS Baltic |
| 60 | 737 | Greek; former 437 G |
| 61 | 708 | Arabic; ASMO 708 |
| 62 | 850 | WE/Latin 1 |
| 63 | 437 | US |

#### sxHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of non-ascending lowercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0078 (LATIN SMALL LETTER X). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in font substitution: the xHeight value of one font can be scaled to approximate the apparent size of another. |

#### sCapHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of uppercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0048 (LATIN CAPITAL LETTER H). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in systems that specify type size by capital height measured in millimeters. It can also be used as an alignment metric; the top of a drop capital, for instance, can be aligned to the sCapHeight metric of the first line of text. |

#### usDefaultChar

| Format: | uint16 |
| --- | --- |
| Description: | Whenever a request is made for a character that is not in the font, Windows provides this default character. If the value of this field is zero, glyph ID 0 is to be used for the default character otherwise this is the Unicode encoding of the glyph that Windows uses as the default character. |

#### usBreakChar

| Format: | uint16 |
| --- | --- |
| Description: | This is the Unicode encoding of the glyph that Windows uses as the break character. The break character is used to separate words and justify text. Most fonts specify 'space' as the break character. |

#### usMaxContext

| Format: | uint16 |
| --- | --- |
| Description: | The maximum length of a target glyph context for any feature in this font. For example, a font which has only a pair kerning feature should set this field to 2. If the font also has a ligature feature in which the glyph sequence 'f f i' is substituted by the ligature 'ffi', then this field should be set to 3. This field could be useful to sophisticated line-breaking engines in determining how far they should look ahead to test whether something could change that effect the line breaking. For chaining contextual lookups, the length of the string (covered glyph) + (input sequence) + (lookahead sequence) should be considered. |

  - OS/2 - OS/2 and Windows metrics (version 3)

The OS/2 table consists of a set of metrics that are required in OFF fonts.

NOTE	This is maintained for purposes of being able to validate version 3 OS/2 tables.

| Type | Name of Entry | Comments |
| --- | --- | --- |
| uint16 | Version | 0x0003 |
| int16 | xAvgCharWidth |  |
| uint16 | usWeightClass |  |
| uint16 | usWidthClass |  |
| uint16 | fsType |  |
| int16 | ySubscriptXSize |  |
| int16 | ySubscriptYSize |  |
| int16 | ySubscriptXOffset |  |
| int16 | ySubscriptYOffset |  |
| int16 | ySuperscriptXSize |  |
| int16 | ySuperscriptYSize |  |
| int16 | ySuperscriptXOffset |  |
| int16 | ySuperscriptYOffset |  |
| int16 | yStrikeoutSize |  |
| int16 | yStrikeoutPosition |  |
| int16 | sFamilyClass |  |
| uint8 | Panose[10] |  |
| uint32 | ulUnicodeRange1 | Bits 0-31 |
| uint32 | ulUnicodeRange2 | Bits 32-63 version 0x0001 and later |
| uint32 | ulUnicodeRange3 | Bits 64-95 version 0x0001 and later |
| uint32 | ulUnicodeRange4 | Bits 96-127 version 0x0001 and later |
| Tag | achVendID |  |
| uint16 | fsSelection |  |
| uint16 | usFirstCharIndex |  |
| uint16 | usLastCharIndex |  |
| int16 | sTypoAscender |  |
| int16 | sTypoDescender |  |
| int16 | sTypoLineGap |  |
| uint16 | usWinAscent |  |
| uint16 | usWinDescent |  |
| uint32 | ulCodePageRange1 | Bits 0-31 version 0x0001 and later |
| uint32 | ulCodePageRange2 | Bits 32-63 version 0x0001 and later |
| int16 | sxHeight | version 0x0002 and later |
| int16 | sCapHeight | version 0x0002 and later |
| uint16 | usDefaultChar | version 0x0002 and later |
| uint16 | usBreakChar | version 0x0002 and later |
| uint16 | usMaxContext | version 0x0002 and later |

#### version

| Format: | uint16 |
| --- | --- |
| Units: | n/a |
| Title: | OS/2 table version number. |
| Description: | The version number for this OS/2 table. |
| Comments: | The version number allows for identification of the precise contents and layout for the OS/2 table. The version number for this layout is three (3). See Annex A. |

#### xAvgCharWidth

| Format: | int16 |
| --- | --- |
| Units: | Pels / em units |
| Title: | Average weighted escapement. |
| Description: | The Average Character Width parameter specifies the arithmetic average of the escapement (width) of all non-zero width glyphs in the font. |
| Comments: | The value for xAvgCharWidth is calculated by obtaining the arithmetic average of the width of all non-zero width glyphs in the font. Furthermore, it is strongly recommended that implementers do not rely on this value for computing layout for lines of text. Especially, for cases where complex scripts are used. The calculation algorithm differs from one being used in previous versions of OS/2 table. For details see Annex A. |

#### usWeightClass

| Format: | uint16 |
| --- | --- |
| Title: | Weight class. |
| Description: | Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font. |
| Comments: |  |

| Value | Description | C Definition (from windows.h) |
| --- | --- | --- |
| 100 | Thin | FW_THIN |
| 200 | Extra-light (Ultra-light) | FW_EXTRALIGHT |
| 300 | Light | FW_LIGHT |
| 400 | Normal (Regular) | FW_NORMAL |
| 500 | Medium | FW_MEDIUM |
| 600 | Semi-bold (Demi-bold) | FW_SEMIBOLD |
| 700 | Bold | FW_BOLD |
| 800 | Extra-bold (Ultra-bold) | FW_EXTRABOLD |
| 900 | Black (Heavy) | FW_BLACK |

#### usWidthClass

| Format: | uint16 |
| --- | --- |
| Title: | Width class. |
| Description: | Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font. |
| Comments: | Although every character in a font may have a different numeric aspect ratio, each character in a font of normal width has a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the characters in the new font is some percentage greater or less than those same characters in the normal font -- it is this difference that this parameter specifies. |

| Value | Description | C Definition | % of normal |
| --- | --- | --- | --- |
| 1 | Ultra-condensed | FWIDTH_ULTRA_CONDENSED | 50 |
| 2 | Extra-condensed | FWIDTH_EXTRA_CONDENSED | 62.5 |
| 3 | Condensed | FWIDTH_CONDENSED | 75 |
| 4 | Semi-condensed | FWIDTH_SEMI_CONDENSED | 87.5 |
| 5 | Medium (normal) | FWIDTH_NORMAL | 100 |
| 6 | Semi-expanded | FWIDTH_SEMI_EXPANDED | 112.5 |
| 7 | Expanded | FWIDTH_EXPANDED | 125 |
| 8 | Extra-expanded | FWIDTH_EXTRA_EXPANDED | 150 |
| 9 | Ultra-expanded | FWIDTH_ULTRA_EXPANDED | 200 |

#### fsType

| Format: | uint16 |
| --- | --- |
| Title: | Type flags. |
| Description: | Indicates font embedding licensing rights for the font. Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font. The OFF Font Embedding DLL Applications that implement support for font embedding, either through use of the Font Embedding DLL or through other means, must not embed fonts which are not licensed to permit embedding. Further, applications loading embedded fonts for temporary use (see Preview & Print and Editable embedding below) must delete the fonts when the document containing the embedded font is closed. This version of the OS/2 table makes bits 0 - 3 a set of exclusive bits. In other words, at most one bit in this range may be set at a time. The purpose is to remove misunderstandings caused by previous behavior of using the least restrictive of the bits that are set. |

| Bit | Bit Mask | Description |
| --- | --- | --- |
|  | 0x0000 | Installable Embedding: No fsType bit is set. Thus fsType is zero. Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser. |
| 0 | 0x0001 | Reserved, must be zero. |
| 1 | 0x0002 | Restricted License embedding: Fonts that have only this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: For Restricted License embedding to take effect, it must be the only level of embedding selected. |
| 2 | 0x0004 | Preview & Print embedding: When this bit is set, the font may be embedded, and temporarily loaded on the remote system. Documents containing Preview & Print fonts must be opened "read-only;" no edits can be applied to the document. |
| 3 | 0x0008 | Editable embedding: When this bit is set, the font may be embedded but must only be installed temporarily on other systems. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened for reading, editing is permitted, and changes may be saved. |
| 4-7 |  | Reserved, must be zero. |
| 8 | 0x0100 | No subsetting: When this bit is set, the font may not be subsetted prior to embedding. Other embedding restrictions specified in bits 0-3 and 9 also apply. |
| 9 | 0x0200 | Bitmap embedding only: When this bit is set, only bitmaps contained in the font may be embedded. No outline data may be embedded. If there are no bitmaps available in the font, then the font is considered unembeddable and the embedding services will fail. Other embedding restrictions specified in bits 0-3 and 8 also apply. |
| 10-15 |  | Reserved, must be zero. |

#### ySubscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript vertical font size. |
| Description: | The recommended vertical size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The horizontal font size specifies a font designer's recommendation for horizontal font size of subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the characters in a font or by substituting characters from another font, this parameter specifies the recommended horizontal EmInc for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript x offset. |
| Description: | The recommended horizontal offset in font design untis for subscripts for this font. |
| Comments: | The Subscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin of the font to the character origin of the subscript's character -- for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters, this parameter specifies the recommended horizontal position from the character escapement point of the last character before the first subscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for subscript characters is usually adjusted to compensate for the angle of incline. |

#### ySubscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for subscripts for this font. |
| Comments: | The Subscript Y offset parameter specifies a font designer's recommended vertical offset from the character baseline to the character baseline for subscript characters associated with this font. Values are expressed as a positive offset below the character baseline. If a font does not include all of the required subscript for an application, this parameter specifies the recommended vertical distance below the character baseline for those subscript characters. |

#### ySuperscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript vertical font size. |
| Description: | The recommended vertical size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The vertical font size specifies a font designer's recommended vertical font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended EmHeight for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript x offset. |
| Description: | The recommended horizontal offset in font design units for superscripts for this font. |
| Comments: | The Superscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin to the superscript character's origin for the superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended horizontal position from the escapement point of the character before the first superscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for superscript characters is usually adjusted to compensate for the angle of incline. |

#### ySuperscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for superscripts for this font. |
| Comments: | The Superscript Y offset parameter specifies a font designer's recommended vertical offset -- from the character baseline to the superscript character's baseline associated with this font. Values for this parameter are expressed as a positive offset above the character baseline. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended vertical distance above the character baseline for those superscript characters. |

#### yStrikeoutSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout size. |
| Description: | Width of the strikeout stroke in font design units. |
| Comments: | This field should normally be the width of the em dash for the current font. If the size is one, the strikeout line will be the line represented by the strikeout position field. If the value is two, the strikeout line will be the line represented by the strikeout position and the line immediately above the strikeout position. For a Roman font with a 2048 em square, 102 is suggested. |

#### yStrikeoutPosition

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout position. |
| Description: | The position of the top of the strikeout stroke relative to the baseline in font design units. |
| Comments: | Positive values represent distances above the baseline, while negative values represent distances below the baseline. A value of zero falls directly on the baseline, while a value of one falls one pel above the baseline. The value of strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. For a Roman font with a 2048 em square, 460 is suggested. |

#### sFamilyClass

| Format: | int16 |
| --- | --- |
| Title: | Font-family class and subclass. |
| Description: | This parameter is a classification of font-family design. |
| Comments: | The font class and font subclass are registered values per Annex A. the to each font family. This parameter is intended for use in selecting an alternate font when the requested font is not available. The font class is the most general and the font subclass is the most specific. The high byte of this field contains the family class, while the low byte contains the family subclass. |

#### Panose

| Format: | 10 byte array |
| --- | --- |
| Title: | PANOSE classification number |
| International: | Additional specifications are required for PANOSE to classify non-Latin character sets. |
| Description: | This 10 byte series of numbers is used to describe the visual characteristics of a given typeface. If provided, these characteristics are then used to associate the font with other fonts of similar appearance having different names; the default values should be set to 'zero'. The variables for each digit are listed below. |
| Comments: | The specification for assigning PANOSE values [14] can be found in bibliography and is maintained by Monotype Imaging Inc. |

| Type | Name |
| --- | --- |
| uint8 | bFamilyType; |
| uint8 | bSerifStyle; |
| uint8 | bWeight; |
| uint8 | bProportion; |
| uint8 | bContrast; |
| uint8 | bStrokeVariation; |
| uint8 | bArmStyle; |
| uint8 | bLetterform; |
| uint8 | bMidline; |
| uint8 | bXHeight; |

#### ulUnicodeRange

ulUnicodeRange1 (Bits 0-31)
ulUnicodeRange2 (Bits 32-63)
ulUnicodeRange3 (Bits 64-95)
ulUnicodeRange4 (Bits 96-127)

| Format: | 32-bit unsigned long(4 copies) totaling 128 bits. |
| --- | --- |
| Title: | Unicode Character Range |
| Description: | This field is used to specify the Unicode blocks or ranges encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform). If the bit is set (1) then the Unicode range is considered functional. If the bit is clear (0) then the range is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by ranges if at all possible. All reserved fields must be zero. Each long is in Big-Endian form. See the Basic Multilingual Plane of ISO/IEC 10646 or the Unicode Standard for the list of Unicode ranges and characters. |

| Bit | Description |
| --- | --- |
| 0 | Basic Latin |
| 1 | Latin-1 Supplement |
| 2 | Latin Extended-A |
| 3 | Latin Extended-B |
| 4 | IPA Extensions |
| 5 | Spacing Modifier Letters |
| 6 | Combining Diacritical Marks |
| 7 | Greek and Coptic |
| 8 | Reserved for Unicode SubRanges |
| 9 | Cyrillic |
|  | Cyrillic Supplementary |
| 10 | Armenian |
| 11 | Hebrew |
| 12 | Reserved for Unicode SubRanges |
| 13 | Arabic |
| 14 | Reserved for Unicode SubRanges |
| 15 | Devanagari |
| 16 | Bengali |
| 17 | Gurmukhi |
| 18 | Gujarati |
| 19 | Oriya |
| 20 | Tamil |
| 21 | Telugu |
| 22 | Kannada |
| 23 | Malayalam |
| 24 | Thai |
| 25 | Lao |
| 26 | Georgian |
| 27 | Reserved for Unicode SubRanges |
| 28 | Hangul Jamo |
| 29 | Latin Extended Additional |
| 30 | Greek Extended |
| 31 | General Punctuation |
| 32 | Superscripts And Subscripts |
| 33 | Currency Symbols |
| 34 | Combining Diacritical Marks For Symbols |
| 35 | Letterlike Symbols |
| 36 | Number Forms |
| 37 | Arrows |
|  | Supplemental Arrows-A |
|  | Supplemental Arrows-B |
| 38 | Mathematical Operators |
|  | Supplemental Mathematical Operators |
|  | Miscellaneous Mathematical Symbols-A |
|  | Miscellaneous Mathematical Symbols-B |
| 39 | Miscellaneous Technical |
| 40 | Control Pictures |
| 41 | Optical Character Recognition |
| 42 | Enclosed Alphanumerics |
| 43 | Box Drawing |
| 44 | Block Elements |
| 45 | Geometric Shapes |
| 46 | Miscellaneous Symbols |
| 47 | Dingbats |
| 48 | CJK Symbols And Punctuation |
| 49 | Hiragana |
| 50 | Katakana |
|  | Katakana Phonetic Extensions |
| 51 | Bopomofo |
|  | Bopomofo Extended |
| 52 | Hangul Compatibility Jamo |
| 53 | Reserved for Unicode SubRanges |
| 54 | Enclosed CJK Letters And Months |
| 55 | CJK Compatibility |
| 56 | Hangul Syllables |
| 57 | Non-Plane 0 * |
| 58 | Reserved for Unicode SubRanges |
| 59 | CJK Unified Ideographs |
|  | CJK Radicals Supplement |
|  | Kangxi Radicals |
|  | Ideographic Description Characters |
|  | CJK Unified Ideograph Extension A |
|  | CJK Unified Ideographs Extension B |
|  | Kanbun |
| 60 | Private Use Area |
| 61 | CJK Compatibility Ideographs |
|  | CJK Compatibility Ideographs Supplement |
| 62 | Alphabetic Presentation Forms |
| 63 | Arabic Presentation Forms-A |
| 64 | Combining Half Marks |
| 65 | CJK Compatibility Forms |
| 66 | Small Form Variants |
| 67 | Arabic Presentation Forms-B |
| 68 | Halfwidth And Fullwidth Forms |
| 69 | Specials |
| 70 | Tibetan |
| 71 | Syriac |
| 72 | Thaana |
| 73 | Sinhala |
| 74 | Myanmar |
| 75 | Ethiopic |
| 76 | Cherokee |
| 77 | Unified Canadian Aboriginal Syllabics |
| 78 | Ogham |
| 79 | Runic |
| 80 | Khmer |
| 81 | Mongolian |
| 82 | Braille Patterns |
| 83 | Yi Syllables |
|  | Yi Radicals |
| 84 | Tagalog |
|  | Hanunoo |
|  | Buhid |
|  | Tagbanwa |
| 85 | Old Italic |
| 86 | Gothic |
| 87 | Deseret |
| 88 | Byzantine Musical Symbols |
|  | Musical Symbols |
| 89 | Mathematical Alphanumeric Symbols |
| 90 | Private Use (plane 15) |
|  | Private Use (plane 16) |
| 91 | Variation Selectors |
| 92 | Tags |
| 93-127 | Reserved for Unicode SubRanges |

NOTE	* Setting bit 57 implies that there is at least one codepoint beyond the Basic Multilingual Plane that is supported by this font.

#### achVendID

| Format: | Tag |
| --- | --- |
| Title: | Font Vendor Identification |
| Description: | The four character identifier for the vendor of the given type face. |
| Comments: | This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. It is reasonable to assume that there will be 6 vendors of ITC Zapf Dingbats for use on desktop platforms in the near future (if not already). It is also likely that the vendors will have other inherent benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor's type to be used over another, possibly inferior, font file. The Vendor ID value is not required. The Vendor ID list can be accessed via the informative reference 6 in the bibliolgraphy. |

#### fsSelection

| Format: | 2-byte bit field. |
| --- | --- |
| Title: | Font selection flags. |
| Description: | Contains information concerning the nature of the font patterns, as follows: |

| Bit # | macStyle bit | C definition | Description |
| --- | --- | --- | --- |
| 0 | bit 1 | ITALIC | Font contains Italic characters, otherwise they are upright. |
| 1 |  | UNDERSCORE | Characters are underscored. |
| 2 |  | NEGATIVE | Characters have their foreground and background reversed. |
| 3 |  | OUTLINED | Outline (hollow) characters, otherwise they are solid. |
| 4 |  | STRIKEOUT | Characters are overstruck. |
| 5 | bit 0 | BOLD | Characters are emboldened. |
| 6 |  | REGULAR | Characters are in the standard weight/style for the font. |

| Comments: | All undefined bits must be zero. This field contains information on the original design of the font. Bits 0 & 5 can be used to determine if the font was designed with these features or whether some type of machine simulation was performed on the font to achieve this appearance. Bits 1-4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. If bit 6 is set, then bits 0 and 5 must be clear, else the behavior is undefined. As noted above, the settings of bits 0 and 1 must be reflected in the macStyle bits in the 'head' table. While bit 6 on implies that bits 0 and 1 of macStyle are clear (along with bits 0 and 5 of fsSelection), the reverse is not true. Bits 0 and 1 of macStyle (and 0 and 5 of fsSelection) may be clear and that does not give any indication of whether or not bit 6 of fsSelection is clear (e.g., Arial Light would have all bits cleared; it is not the regular version of Arial). |
| --- | --- |

#### usFirstCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF if the minimum index value is a supplementary character. |

#### usLastCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF. |

#### sTypoAscender

| Format: | int16 |
| --- | --- |
| Description: | The typographic ascender for this font. Remember that this is not the same as the Ascender value in the 'hhea' table, . One good source for sTypoAscender in Latin based fonts is the Ascender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoAscender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatibility requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoAscender is that which describes the top of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoDescender

| Format: | int16 |
| --- | --- |
| Description: | The typographic descender for this font. One good source for sTypoDescender in Latin based fonts is the Descender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoDescender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoDescender is that which describes the bottom of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoLineGap

| Format: | int16 |
| --- | --- |
| Description: | The typographic line gap for this font. Remember that this is not the same as the LineGap value in the 'hhea' table. The suggested usage for sTypoLineGap is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. Typical values average 7-10% of units per em. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements (see clause 7, "Recommendations for OFF Fonts"). These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. |

#### usWinAscent

| Format: | uint16 |
| --- | --- |
| Description: | The ascender metric for Windows. For platform 3 encoding 0 fonts, it is the same as yMax. Windows will clip the bitmap of any portion of a glyph that appears above this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMax. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### usWinDescent

| Format: | uint16 |
| --- | --- |
| Description: | The descender metric for Windows. For platform 3 encoding 0 fonts, it is the same as -yMin. Windows will clip the bitmap of any portion of a glyph that appears below this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMin. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### ulCodePageRange

ulCodePageRange1 Bits 0-31
ulCodePageRange2 Bits 32-63

| Format: | 32-bit unsigned long (2 copies) totaling 64 bits. |
| --- | --- |
| Title: | Code Page Character Range |
| Description: | This field is used to specify the code pages encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform). If the font file is encoding ID 0, then the Symbol Character Set bit should be set. If the bit is set (1) then the code page is considered functional. If the bit is clear (0) then the code page is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by code pages if at all possible. Symbol character sets have a special meaning. If the symbol bit (31) is set, and the font file contains a 'cmap' subtable for platform of 3 and encoding ID of 1, then all of the characters in the Unicode range 0xF000 - 0xF0FF (inclusive) will be used to enumerate the symbol character set. If the bit is not set, any characters present in that range will not be enumerated as a symbol character set. All reserved fields must be zero. Each long is in Big-Endian form. |

| Bit | Code Page | Description |
| --- | --- | --- |
| 0 | 1252 | Latin 1 |
| 1 | 1250 | Latin 2: Eastern Europe |
| 2 | 1251 | Cyrillic |
| 3 | 1253 | Greek |
| 4 | 1254 | Turkish |
| 5 | 1255 | Hebrew |
| 6 | 1256 | Arabic |
| 7 | 1257 | Windows Baltic |
| 8 | 1258 | Vietnamese |
| 9-15 |  | Reserved for Alternate ANSI |
| 16 | 874 | Thai |
| 17 | 932 | JIS/Japan |
| 18 | 936 | Chinese: Simplified chars--PRC and Singapore |
| 19 | 949 | Korean Wansung |
| 20 | 950 | Chinese: Traditional chars--Taiwan and Hong Kong |
| 21 | 1361 | Korean Johab |
| 22-28 |  | Reserved for Alternate ANSI & OEM |
| 29 |  | Macintosh Character Set (US Roman) |
| 30 |  | OEM Character Set |
| 31 |  | Symbol Character Set |
| 32-47 |  | Reserved for OEM |
| 48 | 869 | IBM Greek |
| 49 | 866 | MS-DOS Russian |
| 50 | 865 | MS-DOS Nordic |
| 51 | 864 | Arabic |
| 52 | 863 | MS-DOS Canadian French |
| 53 | 862 | Hebrew |
| 54 | 861 | MS-DOS Icelandic |
| 55 | 860 | MS-DOS Portuguese |
| 56 | 857 | IBM Turkish |
| 57 | 855 | IBM Cyrillic; primarily Russian |
| 58 | 852 | Latin 2 |
| 59 | 775 | MS-DOS Baltic |
| 60 | 737 | Greek; former 437 G |
| 61 | 708 | Arabic; ASMO 708 |
| 62 | 850 | WE/Latin 1 |
| 63 | 437 | US |

#### sxHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of non-ascending lowercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0078 (LATIN SMALL LETTER X). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in font substitution: the xHeight value of one font can be scaled to approximate the apparent size of another. |

#### sCapHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of uppercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0048 (LATIN CAPITAL LETTER H). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in systems that specify type size by capital height measured in millimeters. It can also be used as an alignment metric; the top of a drop capital, for instance, can be aligned to the sCapHeight metric of the first line of text. |

#### usDefaultChar

| Format: | uint16 |
| --- | --- |
| Description: | Whenever a request is made for a character that is not in the font, Windows provides this default character. If the value of this field is zero, glyph ID 0 is to be used for the default character otherwise this is the Unicode encoding of the glyph that Windows uses as the default character. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). |

#### usBreakChar

| Format: | uint16 |
| --- | --- |
| Description: | This is the Unicode encoding of the glyph that Windows uses as the break character. The break character is used to separate words and justify text. Most fonts specify 'space' as the break character. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). |

#### usMaxContext

| Format: | uint16 |
| --- | --- |
| Description: | The maximum length of a target glyph context for any feature in this font. For example, a font which has only a pair kerning feature should set this field to 2. If the font also has a ligature feature in which the glyph sequence 'f f i' is substituted by the ligature 'ffi', then this field should be set to 3. This field could be useful to sophisticated line-breaking engines in determining how far they should look ahead to test whether something could change that effect the line breaking. For chaining contextual lookups, the length of the string (covered glyph) + (input sequence) + (lookahead sequence) should be considered. |

  - OS/2 - OS/2 and Windows metrics (version 4)

The OS/2 table consists of a set of metrics that are required in OFF fonts.

NOTE	This is maintained for purposes of being able to validate version 4 OS/2 tables.

| Type | Name of Entry | Comments |
| --- | --- | --- |
| uint16 | Version | 0x0004 |
| int16 | xAvgCharWidth |  |
| uint16 | usWeightClass |  |
| uint16 | usWidthClass |  |
| uint16 | fsType |  |
| int16 | ySubscriptXSize |  |
| int16 | ySubscriptYSize |  |
| int16 | ySubscriptXOffset |  |
| int16 | ySubscriptYOffset |  |
| int16 | ySuperscriptXSize |  |
| int16 | ySuperscriptYSize |  |
| int16 | ySuperscriptXOffset |  |
| int16 | ySuperscriptYOffset |  |
| int16 | yStrikeoutSize |  |
| int16 | yStrikeoutPosition |  |
| int16 | sFamilyClass |  |
| uint8 | Panose[10] |  |
| uint32 | ulUnicodeRange1 | Bits 0-31 |
| uint32 | ulUnicodeRange2 | Bits 32-63 version 0x0001 and later |
| uint32 | ulUnicodeRange3 | Bits 64-95 version 0x0001 and later |
| uint32 | ulUnicodeRange4 | Bits 96-127 version 0x0001 and later |
| Tag | achVendID |  |
| uint16 | fsSelection |  |
| uint16 | usFirstCharIndex |  |
| uint16 | usLastCharIndex |  |
| int16 | sTypoAscender |  |
| int16 | sTypoDescender |  |
| int16 | sTypoLineGap |  |
| uint16 | usWinAscent |  |
| uint16 | usWinDescent |  |
| uint32 | ulCodePageRange1 | Bits 0-31 version 0x0001 and later |
| uint32 | ulCodePageRange2 | Bits 32-63 version 0x0001 and later |
| int16 | sxHeight | version 0x0002 and later |
| int16 | sCapHeight | version 0x0002 and later |
| uint16 | usDefaultChar | version 0x0002 and later |
| uint16 | usBreakChar | version 0x0002 and later |
| uint16 | usMaxContext | version 0x0002 and later |

#### version

| Format: | uint16 |
| --- | --- |
| Units: | n/a |
| Title: | OS/2 table version number. |
| Description: | The version number for this OS/2 table. |
| Comments: | The version number allows for identification of the precise contents and layout for the OS/2 table. The version number for this layout is four (4). See Annex B. |

#### xAvgCharWidth

| Format: | int16 |
| --- | --- |
| Units: | Pels / em units |
| Title: | Average weighted escapement. |
| Description: | The Average Character Width parameter specifies the arithmetic average of the escapement (width) of all non-zero width glyphs in the font. |
| Comments: | The value for xAvgCharWidth is calculated by obtaining the arithmetic average of the width of all non-zero width glyphs in the font. Furthermore, it is strongly recommended that implementers do not rely on this value for computing layout for lines of text. Especially, for cases where complex scripts are used. The calculation algorithm differs from one being used in previous versions of OS/2 table. For details see Annex A. |

#### usWeightClass

| Format: | uint16 |
| --- | --- |
| Title: | Weight class. |
| Description: | Indicates the visual weight (degree of blackness or thickness of strokes) of the characters in the font. |
| Comments: |  |

| Value | Description | C Definition (from windows.h) |
| --- | --- | --- |
| 100 | Thin | FW_THIN |
| 200 | Extra-light (Ultra-light) | FW_EXTRALIGHT |
| 300 | Light | FW_LIGHT |
| 400 | Normal (Regular) | FW_NORMAL |
| 500 | Medium | FW_MEDIUM |
| 600 | Semi-bold (Demi-bold) | FW_SEMIBOLD |
| 700 | Bold | FW_BOLD |
| 800 | Extra-bold (Ultra-bold) | FW_EXTRABOLD |
| 900 | Black (Heavy) | FW_BLACK |

#### usWidthClass

| Format: | uint16 |
| --- | --- |
| Title: | Width class. |
| Description: | Indicates a relative change from the normal aspect ratio (width to height ratio) as specified by a font designer for the glyphs in a font. |
| Comments: | Although every character in a font may have a different numeric aspect ratio, each character in a font of normal width has a relative aspect ratio of one. When a new type style is created of a different width class (either by a font designer or by some automated means) the relative aspect ratio of the characters in the new font is some percentage greater or less than those same characters in the normal font -- it is this difference that this parameter specifies. |

| Value | Description | C Definition | % of normal |
| --- | --- | --- | --- |
| 1 | Ultra-condensed | FWIDTH_ULTRA_CONDENSED | 50 |
| 2 | Extra-condensed | FWIDTH_EXTRA_CONDENSED | 62.5 |
| 3 | Condensed | FWIDTH_CONDENSED | 75 |
| 4 | Semi-condensed | FWIDTH_SEMI_CONDENSED | 87.5 |
| 5 | Medium (normal) | FWIDTH_NORMAL | 100 |
| 6 | Semi-expanded | FWIDTH_SEMI_EXPANDED | 112.5 |
| 7 | Expanded | FWIDTH_EXPANDED | 125 |
| 8 | Extra-expanded | FWIDTH_EXTRA_EXPANDED | 150 |
| 9 | Ultra-expanded | FWIDTH_ULTRA_EXPANDED | 200 |

#### fsType

| Format: | uint16 |
| --- | --- |
| Title: | Type flags. |
| Description: | Indicates font embedding licensing rights for the font. Embeddable fonts may be stored in a document. When a document with embedded fonts is opened on a system that does not have the font installed (the remote system), the embedded font may be loaded for temporary (and in some cases, permanent) use on that system by an embedding-aware application. Embedding licensing rights are granted by the vendor of the font. The OFF Font Embedding DLL Applications that implement support for font embedding, either through use of the Font Embedding DLL or through other means, must not embed fonts which are not licensed to permit embedding. Further, applications loading embedded fonts for temporary use (see Preview & Print and Editable embedding below) must delete the fonts when the document containing the embedded font is closed. This version of the OS/2 table makes bits 0 - 3 a set of exclusive bits. In other words, at most one bit in this range may be set at a time. The purpose is to remove misunderstandings caused by previous behavior of using the least restrictive of the bits that are set. |

| Bit | Bit Mask | Description |
| --- | --- | --- |
|  | 0x0000 | Installable Embedding: No fsType bit is set. Thus fsType is zero. Fonts with this setting indicate that they may be embedded and permanently installed on the remote system by an application. The user of the remote system acquires the identical rights, obligations and licenses for that font as the original purchaser of the font, and is subject to the same end-user license agreement, copyright, design patent, and/or trademark as was the original purchaser. |
| 0 | 0x0001 | Reserved, must be zero. |
| 1 | 0x0002 | Restricted License embedding: Fonts that have only this bit set must not be modified, embedded or exchanged in any manner without first obtaining permission of the legal owner. Caution: For Restricted License embedding to take effect, it must be the only level of embedding selected. |
| 2 | 0x0004 | Preview & Print embedding: When this bit is set, the font may be embedded, and temporarily loaded on the remote system. Documents containing Preview & Print fonts must be opened "read-only;" no edits can be applied to the document. |
| 3 | 0x0008 | Editable embedding: When this bit is set, the font may be embedded but must only be installed temporarily on other systems. In contrast to Preview & Print fonts, documents containing Editable fonts may be opened for reading, editing is permitted, and changes may be saved. |
| 4-7 |  | Reserved, must be zero. |
| 8 | 0x0100 | No subsetting: When this bit is set, the font may not be subsetted prior to embedding. Other embedding restrictions specified in bits 0-3 and 9 also apply. |
| 9 | 0x0200 | Bitmap embedding only: When this bit is set, only bitmaps contained in the font may be embedded. No outline data may be embedded. If there are no bitmaps available in the font, then the font is considered unembeddable and the embedding services will fail. Other embedding restrictions specified in bits 0-3 and 8 also apply. |
| 10-15 |  | Reserved, must be zero. |

#### ySubscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptXSize is set to 205, then the horizontal size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript vertical font size. |
| Description: | The recommended vertical size in font design units for subscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g. numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The horizontal font size specifies a font designer's recommendation for horizontal font size of subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters by scaling the characters in a font or by substituting characters from another font, this parameter specifies the recommended horizontal EmInc for those subscript characters. For example, if the em square for a font is 2048 and ySubScriptYSize is set to 205, then the vertical size for a simulated subscript character would be 1/10th the size of the normal character. |

#### ySubscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript x offset. |
| Description: | The recommended horizontal offset in font design untis for subscripts for this font. |
| Comments: | The Subscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin of the font to the character origin of the subscript's character -- for subscript characters associated with this font. If a font does not include all of the required subscript characters for an application, and the application can substitute characters, this parameter specifies the recommended horizontal position from the character escapement point of the last character before the first subscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for subscript characters is usually adjusted to compensate for the angle of incline. |

#### ySubscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Subscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for subscripts for this font. |
| Comments: | The Subscript Y offset parameter specifies a font designer's recommended vertical offset from the character baseline to the character baseline for subscript characters associated with this font. Values are expressed as a positive offset below the character baseline. If a font does not include all of the required subscript for an application, this parameter specifies the recommended vertical distance below the character baseline for those subscript characters. |

#### ySuperscriptXSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript horizontal font size. |
| Description: | The recommended horizontal size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the em square size of the font being used for a subscript. The horizontal font size specifies a font designer's recommended horizontal font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended em square for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptXSize is set to 205, then the horizontal size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptYSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript vertical font size. |
| Description: | The recommended vertical size in font design units for superscripts for this font. |
| Comments: | If a font has two recommended sizes for subscripts, e.g., numerics and other, the numeric sizes should be stressed. This size field maps to the emHeight of the font being used for a subscript. The vertical font size specifies a font designer's recommended vertical font size for superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, and the application can substitute characters by scaling the character of a font or by substituting characters from another font, this parameter specifies the recommended EmHeight for those superscript characters. For example, if the em square for a font is 2048 and ySuperScriptYSize is set to 205, then the vertical size for a simulated superscript character would be 1/10th the size of the normal character. |

#### ySuperscriptXOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript x offset. |
| Description: | The recommended horizontal offset in font design units for superscripts for this font. |
| Comments: | The Superscript X offset parameter specifies a font designer's recommended horizontal offset -- from the character origin to the superscript character's origin for the superscript characters associated with this font. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended horizontal position from the escapement point of the character before the first superscript character. For upright characters, this value is usually zero; however, if the characters of a font have an incline (italic characters) the reference point for superscript characters is usually adjusted to compensate for the angle of incline. |

#### ySuperscriptYOffset

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Superscript y offset. |
| Description: | The recommended vertical offset in font design units from the baseline for superscripts for this font. |
| Comments: | The Superscript Y offset parameter specifies a font designer's recommended vertical offset -- from the character baseline to the superscript character's baseline associated with this font. Values for this parameter are expressed as a positive offset above the character baseline. If a font does not include all of the required superscript characters for an application, this parameter specifies the recommended vertical distance above the character baseline for those superscript characters. |

#### yStrikeoutSize

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout size. |
| Description: | Width of the strikeout stroke in font design units. |
| Comments: | This field should normally be the width of the em dash for the current font. If the size is one, the strikeout line will be the line represented by the strikeout position field. If the value is two, the strikeout line will be the line represented by the strikeout position and the line immediately above the strikeout position. For a Roman font with a 2048 em square, 102 is suggested. |

#### yStrikeoutPosition

| Format: | int16 |
| --- | --- |
| Units: | Font design units |
| Title: | Strikeout position. |
| Description: | The position of the top of the strikeout stroke relative to the baseline in font design units. |
| Comments: | Positive values represent distances above the baseline, while negative values represent distances below the baseline. A value of zero falls directly on the baseline, while a value of one falls one pel above the baseline. The value of strikeout position should not interfere with the recognition of standard characters, and therefore should not line up with crossbars in the font. For a Roman font with a 2048 em square, 460 is suggested. |

#### sFamilyClass

| Format: | int16 |
| --- | --- |
| Title: | Font-family class and subclass. |
| Description: | This parameter is a classification of font-family design. |
| Comments: | The font class and font subclass are registered values per Annex A. the to each font family. This parameter is intended for use in selecting an alternate font when the requested font is not available. The font class is the most general and the font subclass is the most specific. The high byte of this field contains the family class, while the low byte contains the family subclass. |

#### Panose

| Format: | 10 byte array |
| --- | --- |
| Title: | PANOSE classification number |
| International: | Additional specifications are required for PANOSE to classify non-Latin character sets. |
| Description: | This 10 byte series of numbers is used to describe the visual characteristics of a given typeface. If provided, these characteristics are then used to associate the font with other fonts of similar appearance having different names; the default values should be set to 'zero'. |
| Comments: | The specification for assigning PANOSE values [14] can be found in bibliography. |

#### ulUnicodeRange

ulUnicodeRange1 (Bits 0-31)
ulUnicodeRange2 (Bits 32-63)
ulUnicodeRange3 (Bits 64-95)
ulUnicodeRange4 (Bits 96-127)

| Format: | 32-bit unsigned long(4 copies) totaling 128 bits. |
| --- | --- |
| Title: | Unicode Character Range |
| Description: | This field is used to specify the Unicode blocks or ranges encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Microsoft platform, Unicode) and platform 3, encoding ID 10 (Microsoft platform, UCS-4). If the bit is set (1) then the Unicode range is considered functional. If the bit is clear (0) then the range is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by ranges, if at all possible. All reserved fields must be zero. Each long is in Big-Endian form. See ISO/IEC 10646 or the most recent version of the Unicode Standard for the list of Unicode ranges and characters. |

| Bit | Unicode Range | Block range |
| --- | --- | --- |
| 0 | Basic Latin | 0000-007F |
| 1 | Latin-1 Supplement | 0080-00FF |
| 2 | Latin Extended-A | 0100-017F |
| 3 | Latin Extended-B | 0180-024F |
| 4 | IPA Extensions | 0250-02AF |
|  | Phonetic Extensions | 1D00-1D7F |
|  | Phonetic Extensions Supplement | 1D80-1DBF |
| 5 | Spacing Modifier Letters | 02B0-02FF |
|  | Modifier Tone Letters | A700-A71F |
| 6 | Combining Diacritical Marks | 0300-036F |
|  | Combining Diacritical Marks Supplement | 1DC0-1DFF |
| 7 | Greek and Coptic | 0370-03FF |
| 8 | Coptic | 2C80-2CFF |
| 9 | Cyrillic | 0400-04FF |
|  | Cyrillic Supplement | 0500-052F |
|  | Cyrillic Extended-A | 2DE0-2DFF |
|  | Cyrillic Extended-B | A640-A69F |
| 10 | Armenian | 0530-058F |
| 11 | Hebrew | 0590-05FF |
| 12 | Vai | A500-A63F |
| 13 | Arabic | 0600-06FF |
|  | Arabic Supplement | 0750-077F |
| 14 | NKo | 07C0-07FF |
| 15 | Devanagari | 0900-097F |
| 16 | Bengali | 0980-09FF |
| 17 | Gurmukhi | 0A00-0A7F |
| 18 | Gujarati | 0A80-0AFF |
| 19 | Oriya | 0B00-0B7F |
| 20 | Tamil | 0B80-0BFF |
| 21 | Telugu | 0C00-0C7F |
| 22 | Kannada | 0C80-0CFF |
| 23 | Malayalam | 0D00-0D7F |
| 24 | Thai | 0E00-0E7F |
| 25 | Lao | 0E80-0EFF |
| 26 | Georgian | 10A0-10FF |
|  | Georgian Supplement | 2D00-2D2F |
| 27 | Balinese | 1B00-1B7F |
| 28 | Hangul Jamo | 1100-11FF |
| 29 | Latin Extended Additional | 1E00-1EFF |
|  | Latin Extended-C | 2C60-2C7F |
|  | Latin Extended-D | A720-A7FF |
| 30 | Greek Extended | 1F00-1FFF |
| 31 | General Punctuation | 2000-206F |
|  | Supplemental Punctuation | 2E00-2E7F |
| 32 | Superscripts And Subscripts | 2070-209F |
| 33 | Currency Symbols | 20A0-20CF |
| 34 | Combining Diacritical Marks For Symbols | 20D0-20FF |
| 35 | Letterlike Symbols | 2100-214F |
| 36 | Number Forms | 2150-218F |
| 37 | Arrows | 2190-21FF |
|  | Supplemental Arrows-A | 27F0-27FF |
|  | Supplemental Arrows-B | 2900-297F |
|  | Miscellaneous Symbols and Arrows | 2B00-2BFF |
| 38 | Mathematical Operators | 2200-22FF |
|  | Supplemental Mathematical Operators | 2A00-2AFF |
|  | Miscellaneous Mathematical Symbols-A | 27C0-27EF |
|  | Miscellaneous Mathematical Symbols-B | 2980-29FF |
| 39 | Miscellaneous Technical | 2300-23FF |
| 40 | Control Pictures | 2400-243F |
| 41 | Optical Character Recognition | 2440-245F |
| 42 | Enclosed Alphanumerics | 2460-24FF |
| 43 | Box Drawing | 2500-257F |
| 44 | Block Elements | 2580-259F |
| 45 | Geometric Shapes | 25A0-25FF |
| 46 | Miscellaneous Symbols | 2600-26FF |
| 47 | Dingbats | 2700-27BF |
| 48 | CJK Symbols And Punctuation | 3000-303F |
| 49 | Hiragana | 3040-309F |
| 50 | Katakana | 30A0-30FF |
|  | Katakana Phonetic Extensions | 31F0-31FF |
| 51 | Bopomofo | 3100-312F |
|  | Bopomofo Extended | 31A0-31BF |
| 52 | Hangul Compatibility Jamo | 3130-318F |
| 53 | Phags-pa | A840-A87F |
| 54 | Enclosed CJK Letters And Months | 3200-32FF |
| 55 | CJK Compatibility | 3300-33FF |
| 56 | Hangul Syllables | AC00-D7AF |
| 57 | Non-Plane 0 * | D800-DFFF |
| 58 | Phoenician | 10900-1091F |
| 59 | CJK Unified Ideographs | 4E00-9FFF |
|  | CJK Radicals Supplement | 2E80-2EFF |
|  | Kangxi Radicals | 2F00-2FDF |
|  | Ideographic Description Characters | 2FF0-2FFF |
|  | CJK Unified Ideographs Extension A | 3400-4DBF |
|  | CJK Unified Ideographs Extension B | 20000-2A6DF |
|  | Kanbun | 3190-319F |
| 60 | Private Use Area (plane 0) | E000-F8FF |
| 61 | CJK Strokes | 31C0-31EF |
|  | CJK Compatibility Ideographs | F900-FAFF |
|  | CJK Compatibility Ideographs Supplement | 2F800-2FA1F |
| 62 | Alphabetic Presentation Forms | FB00-FB4F |
| 63 | Arabic Presentation Forms-A | FB50-FDFF |
| 64 | Combining Half Marks | FE20-FE2F |
| 65 | Vertical Forms | FE10-FE1F |
|  | CJK Compatibility Forms | FE30-FE4F |
| 66 | Small Form Variants | FE50-FE6F |
| 67 | Arabic Presentation Forms-B | FE70-FEFF |
| 68 | Halfwidth And Fullwidth Forms | FF00-FFEF |
| 69 | Specials | FFF0-FFFF |
| 70 | Tibetan | 0F00-0FFF |
| 71 | Syriac | 0700-074F |
| 72 | Thaana | 0780-07BF |
| 73 | Sinhala | 0D80-0DFF |
| 74 | Myanmar | 1000-109F |
| 75 | Ethiopic | 1200-137F |
|  | Ethiopic Supplement | 1380-139F |
|  | Ethiopic Extended | 2D80-2DDF |
| 76 | Cherokee | 13A0-13FF |
| 77 | Unified Canadian Aboriginal Syllabics | 1400-167F |
| 78 | Ogham | 1680-169F |
| 79 | Runic | 16A0-16FF |
| 80 | Khmer | 1780-17FF |
|  | Khmer Symbols | 19E0-19FF |
| 81 | Mongolian | 1800-18AF |
| 82 | Braille Patterns | 2800-28FF |
| 83 | Yi Syllables | A000-A48F |
|  | Yi Radicals | A490-A4CF |
| 84 | Tagalog | 1700-171F |
|  | Hanunoo | 1720-173F |
|  | Buhid | 1740-175F |
|  | Tagbanwa | 1760-177F |
| 85 | Old Italic | 10300-1032F |
| 86 | Gothic | 10330-1034F |
| 87 | Deseret | 10400-1044F |
| 88 | Byzantine Musical Symbols | 1D000-1D0FF |
|  | Musical Symbols | 1D100-1D1FF |
|  | Ancient Greek Musical Notation | 1D200-1D24F |
| 89 | Mathematical Alphanumeric Symbols | 1D400-1D7FF |
| 90 | Private Use (plane 15) | F0000-FFFFD |
|  | Private Use (plane 16) | 100000-10FFFD |
| 91 | Variation Selectors | FE00-FE0F |
|  | Variation Selectors Supplement | E0100-E01EF |
| 92 | Tags | E0000-E007F |
| 93 | Limbu | 1900-194F |
| 94 | Tai Le | 1950-197F |
| 95 | New Tai Lue | 1980-19DF |
| 96 | Buginese | 1A00-1A1F |
| 97 | Glagolitic | 2C00-2C5F |
| 98 | Tifinagh | 2D30-2D7F |
| 99 | Yijing Hexagram Symbols | 4DC0-4DFF |
| 100 | Syloti Nagri | A800-A82F |
| 101 | Linear B Syllabary | 10000-1007F |
|  | Linear B Ideograms | 10080-100FF |
|  | Aegean Numbers | 10100-1013F |
| 102 | Ancient Greek Numbers | 10140-1018F |
| 103 | Ugaritic | 10380-1039F |
| 104 | Old Persian | 103A0-103DF |
| 105 | Shavian | 10450-1047F |
| 106 | Osmanya | 10480-104AF |
| 107 | Cypriot Syllabary | 10800-1083F |
| 108 | Kharoshthi | 10A00-10A5F |
| 109 | Tai Xuan Jing Symbols | 1D300-1D35F |
| 110 | Cuneiform | 12000-123FF |
|  | Cuneiform Numbers and Punctuation | 12400-1247F |
| 111 | Counting Rod Numerals | 1D360-1D37F |
| 112 | Sundanese | 1B80-1BBF |
| 113 | Lepcha | 1C00-1C4F |
| 114 | Ol Chiki | 1C50-1C7F |
| 115 | Saurashtra | A880-A8DF |
| 116 | Kayah Li | A900-A92F |
| 117 | Rejang | A930-A95F |
| 118 | Cham | AA00-AA5F |
| 119 | Ancient Symbols | 10190-101CF |
| 120 | Phaistos Disc | 101D0-101FF |
| 121 | Carian | 102A0-102DF |
|  | Lycian | 10280-1029F |
|  | Lydian | 10920-1093F |
| 122 | Domino Tiles | 1F030-1F09F |
|  | Mahjong Tiles | 1F000-1F02F |
| 123-127 | Reserved |  |

NOTE	* Setting bit 57 implies that there is at least one codepoint beyond the Basic Multilingual Plane that is supported by this font.

#### achVendID

| Format: | Tag |
| --- | --- |
| Title: | Font Vendor Identification |
| Description: | The four character identifier for the vendor of the given type face. |
| Comments: | This is not the royalty owner of the original artwork. This is the company responsible for the marketing and distribution of the typeface that is being classified. It is reasonable to assume that there will be 6 vendors of ITC Zapf Dingbats for use on desktop platforms in the near future (if not already). It is also likely that the vendors will have other inherent benefits in their fonts (more kern pairs, unregularized data, hand hinted, etc.). This identifier will allow for the correct vendor's type to be used over another, possibly inferior, font file. The Vendor ID value is not required. The Vendor ID list can be accessed via the informative reference 6 in the bibliolgraphy. |

#### fsSelection

| Format: | 2-byte bit field. |
| --- | --- |
| Title: | Font selection flags. |
| Description: | Contains information concerning the nature of the font patterns, as follows: |

| Bit # | macStyle bit | C definition | Description |
| --- | --- | --- | --- |
| 0 | bit 1 | ITALIC | Font contains Italic or oblique characters, otherwise they are upright. |
| 1 |  | UNDERSCORE | Characters are underscored. |
| 2 |  | NEGATIVE | Characters have their foreground and background reversed. |
| 3 |  | OUTLINED | Outline (hollow) characters, otherwise they are solid. |
| 4 |  | STRIKEOUT | Characters are overstruck. |
| 5 | bit 0 | BOLD | Characters are emboldened. |
| 6 |  | REGULAR | Characters are in the standard weight/style for the font. |
| 7 |  | USE_TYPO_METRICS | If set, it is strongly recommended to use OS/2.sTypoAscender - OS/2.sTypoDescender+ OS/2.sTypoLineGap as a value for default line spacing for this font. (OS/2 version 4 and later) |
| 8 |  | WWS | The font family this face belongs to is composed of faces that only differ in weight, width and slope (please see more detailed description below.) (OS/2 version 4 and later) |
| 9 |  | OBLIQUE | Font contains oblique characters. (OS/2 version 4 and later) |

| Comments: | All undefined bits must be zero. This field contains information on the original design of the font. Bits 0 & 5 can be used to determine if the font was designed with these features or whether some type of machine simulation was performed on the font to achieve this appearance. Bits 1-4 are rarely used bits that indicate the font is primarily a decorative or special purpose font. If bit 6 is set, then bits 0 and 5 must be clear, else the behavior is undefined. As noted above, the settings of bits 0 and 5 must be reflected in the macStyle bits in the 'head' table. While bit 6 on implies that bits 0 and 1 of macStyle are clear (along with bits 0 and 5 of fsSelection), the reverse is not true. Bits 0 and 1 of macStyle (and 0 and 5 of fsSelection) may be clear and that does not give any indication of whether or not bit 6 of fsSelection is clear (e.g., Arial Light would have all bits cleared; it is not the regular version of Arial). Bit 7 was specified in OS/2 table v. 4. If fonts created with an earlier version of the OS/2 table are updated to the current version of the OS/2 table, then, in order to minimize potential reflow of existing documents which use the fonts, the bit would be set only for fonts for which using the OS/2.usWin* metrics for line height would yield significantly inferior results than using the OS/2.sTypo* values. New fonts, however, are not constrained by backward compatibility situations, and so are free to set this bit always. If bit 8 is set in OS/2 table v. 4, then the font’s typographic family contains faces that differ only in one or more of the attributes weight, width and slope. For example, a family with only weight and slope attributes will set this bit. If unset in OS/2 table v. 4, then this font’s typographic family contains faces that differ in attributes other than weight, width or slope. For example, a family with faces that differ only by weight, slope, and optical size will not set this bit. This bit must be unset in OS/2 table versions less than 4. In these cases, it is not possible to determine any information about the typographic family’s attributes by examining this bit. In this context, "typographic family" is the Microsoft Unicode string for name ID 16, if present, else the Microsoft Unicode string for name ID 1; "weight" is OS/2.usWeightClass; "width" is OS/2.usWidthClass; "slope" is OS/2.fsSelection bit 0 (ITALIC) and bit 9 (OBLIQUE). If bit 9 is set in OS/2 table v. 4, then this font is to be considered an "oblique" style by processes which make a distinction between oblique and italic styles, e.g. Cascading Style Sheets font matching. For example, a font created by algorithmically slanting an upright face will set this bit. If unset in OS/2 table v. 4, then this font is not to be considered an "oblique" style. For example, a font that has a classic italic design will not set this bit. This bit must be unset in OS/2 table versions less than 4. In these cases, it is not possible to determine any information about this font's attributes by examining this bit. This bit, unlike the ITALIC bit, is not related to style-linking for Windows GDI or Mac OS applications in a traditional four-member family of regular, italic, bold and bold italic". It may be set or unset independently of the ITALIC bit. In most cases, if OBLIQUE is set, then ITALIC will also be set, though this is not required. |
| --- | --- |

#### usFirstCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The minimum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and platform- specific encoding ID 0 or 1. For most fonts supporting Win-ANSI or other character sets, this value would be 0x0020. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF if the minimum index value is a supplementary character. |

#### usLastCharIndex

| Format: | uint16 |
| --- | --- |
| Description: | The maximum Unicode index (character code) in this font, according to the cmap subtable for platform ID 3 and encoding ID 0 or 1. This value depends on which character sets the font supports. This field cannot represent supplementary character values (codepoints greater than 0xFFFF). Fonts that support supplementary characters should set the value in this field to 0xFFFF. |

#### sTypoAscender

| Format: | int16 |
| --- | --- |
| Description: | The typographic ascender for this font. Remember that this is not the same as the Ascender value in the 'hhea' table, . One good source for sTypoAscender in Latin based fonts is the Ascender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoAscender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatibility requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoAscender is that which describes the top of the of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoAscender must be set to 880. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoDescender

| Format: | int16 |
| --- | --- |
| Description: | The typographic descender for this font. One good source for sTypoDescender in Latin based fonts is the Descender value from an AFM file. For CJK fonts see below. The suggested usage for sTypoDescender is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements. These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. For CJK (Chinese, Japanese, and Korean) fonts that are intended to be used for vertical writing (in addition to horizontal writing), the required value for sTypoDescender is that which describes the bottom of the ideographic em-box. For example, if the ideographic em-box of the font extends from coordinates 0,-120 to 1000,880 (that is, a 1000x1000 box set 120 design units below the Latin baseline), then the value of sTypoDescender must be set to -120. Failing to adhere to these requirements will result in incorrect vertical layout. Also see the Recommendations clause 7 for more on this field. |

#### sTypoLineGap

| Format: | int16 |
| --- | --- |
| Description: | The typographic line gap for this font. Remember that this is not the same as the LineGap value in the 'hhea' table. The suggested usage for sTypoLineGap is that it be used in conjunction with unitsPerEm to compute typographically-correct default line spacing. Typical values average 7-10% of units per em. The goal is to free applications from Macintosh or Windows-specific metrics which are constrained by backward compatability requirements (see clause 7, "Recommendations for OFF Fonts"). These new metrics, when combined with the character design widths, will allow applications to lay out documents in a typographically correct and portable fashion. |

#### usWinAscent

| Format: | uint16 |
| --- | --- |
| Description: | The ascender metric for Windows. For platform 3 encoding 0 fonts, it is the same as yMax. Windows will clip the bitmap of any portion of a glyph that appears above this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMax. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### usWinDescent

| Format: | uint16 |
| --- | --- |
| Description: | The descender metric for Windows. For platform 3 encoding 0 fonts, it is the same as -yMin. Windows will clip the bitmap of any portion of a glyph that appears below this value. Some applications use this value to determine default line spacing. This is strongly discouraged. The typographic ascender, descender and line gap fields in conjunction with unitsPerEm should be used for this purpose. Developers should set this field keeping the above factors in mind. If any clipping is unacceptable, then the value should be set to yMin. However, if a developer desires to provide appropriate default line spacing using this field, for those applications that continue to use this field for doing so (against OFF recommendations), then the value should be set appropriately. In such a case, it may result in some glyph bitmaps being clipped. |

#### ulCodePageRange

ulCodePageRange1 Bits 0-31
ulCodePageRange2 Bits 32-63

| Format: | 32-bit unsigned long (2 copies) totaling 64 bits. |
| --- | --- |
| Title: | Code Page Character Range |
| Description: | This field is used to specify the code pages encompassed by the font file in the 'cmap' subtable for platform 3, encoding ID 1 (Windows platform). If the font file is encoding ID 0, then the Symbol Character Set bit should be set. If the bit is set (1) then the code page is considered functional. If the bit is clear (0) then the code page is not considered functional. Each of the bits is treated as an independent flag and the bits can be set in any combination. The determination of "functional" is left up to the font designer, although character set selection should attempt to be functional by code pages if at all possible. Symbol character sets have a special meaning. If the symbol bit (31) is set, and the font file contains a 'cmap' subtable for platform of 3 and encoding ID of 1, then all of the characters in the Unicode range 0xF000 - 0xF0FF (inclusive) will be used to enumerate the symbol character set. If the bit is not set, any characters present in that range will not be enumerated as a symbol character set. All reserved fields must be zero. Each long is in Big-Endian form. |

| Bit | Code Page | Description |
| --- | --- | --- |
| 0 | 1252 | Latin 1 |
| 1 | 1250 | Latin 2: Eastern Europe |
| 2 | 1251 | Cyrillic |
| 3 | 1253 | Greek |
| 4 | 1254 | Turkish |
| 5 | 1255 | Hebrew |
| 6 | 1256 | Arabic |
| 7 | 1257 | Windows Baltic |
| 8 | 1258 | Vietnamese |
| 9-15 |  | Reserved for Alternate ANSI |
| 16 | 874 | Thai |
| 17 | 932 | JIS/Japan |
| 18 | 936 | Chinese: Simplified chars--PRC and Singapore |
| 19 | 949 | Korean Wansung |
| 20 | 950 | Chinese: Traditional chars--Taiwan and Hong Kong |
| 21 | 1361 | Korean Johab |
| 22-28 |  | Reserved for Alternate ANSI & OEM |
| 29 |  | Macintosh Character Set (US Roman) |
| 30 |  | OEM Character Set |
| 31 |  | Symbol Character Set |
| 32-46 |  | Reserved for OEM |
| 47 |  | Reserved |
| 48 | 869 | IBM Greek |
| 49 | 866 | MS-DOS Russian |
| 50 | 865 | MS-DOS Nordic |
| 51 | 864 | Arabic |
| 52 | 863 | MS-DOS Canadian French |
| 53 | 862 | Hebrew |
| 54 | 861 | MS-DOS Icelandic |
| 55 | 860 | MS-DOS Portuguese |
| 56 | 857 | IBM Turkish |
| 57 | 855 | IBM Cyrillic; primarily Russian |
| 58 | 852 | Latin 2 |
| 59 | 775 | MS-DOS Baltic |
| 60 | 737 | Greek; former 437 G |
| 61 | 708 | Arabic; ASMO 708 |
| 62 | 850 | WE/Latin 1 |
| 63 | 437 | US |

#### sxHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of non-ascending lowercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0078 (LATIN SMALL LETTER X). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in font substitution: the xHeight value of one font can be scaled to approximate the apparent size of another. |

#### sCapHeight

| Format: | int16 |
| --- | --- |
| Description: | This metric specifies the distance between the baseline and the approximate height of uppercase letters measured in font design units. This value would normally be specified by a type designer but in situations where that is not possible, for example when a legacy font is being converted, the value may be set equal to the top of the unscaled and unhinted glyph bounding box of the glyph encoded at U+0048 (LATIN CAPITAL LETTER H). If no glyph is encoded in this position the field should be set to 0. This metric, if specified, can be used in systems that specify type size by capital height measured in millimeters. It can also be used as an alignment metric; the top of a drop capital, for instance, can be aligned to the sCapHeight metric of the first line of text. |

#### usDefaultChar

| Format: | uint16 |
| --- | --- |
| Description: | Whenever a request is made for a character that is not in the font, Windows provides this default character. If the value of this field is zero, glyph ID 0 is to be used for the default character otherwise this is the Unicode encoding of the glyph that Windows uses as the default character. This field cannot represent supplementary character values (codepoints greater than 0xFFFF), and so applications are strongly discouraged from using this field. |

#### usBreakChar

| Format: | uint16 |
| --- | --- |
| Description: | This is the Unicode encoding of the glyph that Windows uses as the break character. The break character is used to separate words and justify text. Most fonts specify 'space' as the break character. This field cannot represent supplementary character values (codepoints greater than 0xFFFF), and so applications are strongly discouraged from using this field. |

#### usMaxContext

| Format: | uint16 |
| --- | --- |
| Description: | The maximum length of a target glyph context for any feature in this font. For example, a font which has only a pair kerning feature should set this field to 2. If the font also has a ligature feature in which the glyph sequence 'f f i' is substituted by the ligature 'ffi', then this field should be set to 3. This field could be useful to sophisticated line-breaking engines in determining how far they should look ahead to test whether something could change that effect the line breaking. For chaining contextual lookups, the length of the string (covered glyph) + (input sequence) + (lookahead sequence) should be considered. |

- (informative)

OFF Mirroring Pairs List

This file is a copy of the Bidi_Mirroring_Glyph Property of Unicode 5.1 (http://www.unicode.org/Public/5.1.0/ucd/BidiMirroring.txt), with header comments changed and the commented list at the end removed. Consult the URL above for specifications for the format of the data.

The data in this Annex will not be revised.

See the section "Left-to-right and right-to-left text" in subclause 6.1.4 for a description of how this file is to be used by a text layout engine.

0028; 0029 # LEFT PARENTHESIS

0029; 0028 # RIGHT PARENTHESIS

003C; 003E # LESS-THAN SIGN

003E; 003C # GREATER-THAN SIGN

005B; 005D # LEFT SQUARE BRACKET

005D; 005B # RIGHT SQUARE BRACKET

007B; 007D # LEFT CURLY BRACKET

007D; 007B # RIGHT CURLY BRACKET

00AB; 00BB # LEFT-POINTING DOUBLE ANGLE QUOTATION MARK

00BB; 00AB # RIGHT-POINTING DOUBLE ANGLE QUOTATION MARK

0F3A; 0F3B # TIBETAN MARK GUG RTAGS GYON

0F3B; 0F3A # TIBETAN MARK GUG RTAGS GYAS

0F3C; 0F3D # TIBETAN MARK ANG KHANG GYON

0F3D; 0F3C # TIBETAN MARK ANG KHANG GYAS

169B; 169C # OGHAM FEATHER MARK

169C; 169B # OGHAM REVERSED FEATHER MARK

2039; 203A # SINGLE LEFT-POINTING ANGLE QUOTATION MARK

203A; 2039 # SINGLE RIGHT-POINTING ANGLE QUOTATION MARK

2045; 2046 # LEFT SQUARE BRACKET WITH QUILL

2046; 2045 # RIGHT SQUARE BRACKET WITH QUILL

207D; 207E # SUPERSCRIPT LEFT PARENTHESIS

207E; 207D # SUPERSCRIPT RIGHT PARENTHESIS

208D; 208E # SUBSCRIPT LEFT PARENTHESIS

208E; 208D # SUBSCRIPT RIGHT PARENTHESIS

2208; 220B # ELEMENT OF

2209; 220C # NOT AN ELEMENT OF

220A; 220D # SMALL ELEMENT OF

220B; 2208 # CONTAINS AS MEMBER

220C; 2209 # DOES NOT CONTAIN AS MEMBER

220D; 220A # SMALL CONTAINS AS MEMBER

2215; 29F5 # DIVISION SLASH

223C; 223D # TILDE OPERATOR

223D; 223C # REVERSED TILDE

2243; 22CD # ASYMPTOTICALLY EQUAL TO

2252; 2253 # APPROXIMATELY EQUAL TO OR THE IMAGE OF

2253; 2252 # IMAGE OF OR APPROXIMATELY EQUAL TO

2254; 2255 # COLON EQUALS

2255; 2254 # EQUALS COLON

2264; 2265 # LESS-THAN OR EQUAL TO

2265; 2264 # GREATER-THAN OR EQUAL TO

2266; 2267 # LESS-THAN OVER EQUAL TO

2267; 2266 # GREATER-THAN OVER EQUAL TO

2268; 2269 # [BEST FIT] LESS-THAN BUT NOT EQUAL TO

2269; 2268 # [BEST FIT] GREATER-THAN BUT NOT EQUAL TO

226A; 226B # MUCH LESS-THAN

226B; 226A # MUCH GREATER-THAN

226E; 226F # [BEST FIT] NOT LESS-THAN

226F; 226E # [BEST FIT] NOT GREATER-THAN

2270; 2271 # [BEST FIT] NEITHER LESS-THAN NOR EQUAL TO

2271; 2270 # [BEST FIT] NEITHER GREATER-THAN NOR EQUAL TO

2272; 2273 # [BEST FIT] LESS-THAN OR EQUIVALENT TO

2273; 2272 # [BEST FIT] GREATER-THAN OR EQUIVALENT TO

2274; 2275 # [BEST FIT] NEITHER LESS-THAN NOR EQUIVALENT TO

2275; 2274 # [BEST FIT] NEITHER GREATER-THAN NOR EQUIVALENT TO

2276; 2277 # LESS-THAN OR GREATER-THAN

2277; 2276 # GREATER-THAN OR LESS-THAN

2278; 2279 # [BEST FIT] NEITHER LESS-THAN NOR GREATER-THAN

2279; 2278 # [BEST FIT] NEITHER GREATER-THAN NOR LESS-THAN

227A; 227B # PRECEDES

227B; 227A # SUCCEEDS

227C; 227D # PRECEDES OR EQUAL TO

227D; 227C # SUCCEEDS OR EQUAL TO

227E; 227F # [BEST FIT] PRECEDES OR EQUIVALENT TO

227F; 227E # [BEST FIT] SUCCEEDS OR EQUIVALENT TO

2280; 2281 # [BEST FIT] DOES NOT PRECEDE

2281; 2280 # [BEST FIT] DOES NOT SUCCEED

2282; 2283 # SUBSET OF

2283; 2282 # SUPERSET OF

2284; 2285 # [BEST FIT] NOT A SUBSET OF

2285; 2284 # [BEST FIT] NOT A SUPERSET OF

2286; 2287 # SUBSET OF OR EQUAL TO

2287; 2286 # SUPERSET OF OR EQUAL TO

2288; 2289 # [BEST FIT] NEITHER A SUBSET OF NOR EQUAL TO

2289; 2288 # [BEST FIT] NEITHER A SUPERSET OF NOR EQUAL TO

228A; 228B # [BEST FIT] SUBSET OF WITH NOT EQUAL TO

228B; 228A # [BEST FIT] SUPERSET OF WITH NOT EQUAL TO

228F; 2290 # SQUARE IMAGE OF

2290; 228F # SQUARE ORIGINAL OF

2291; 2292 # SQUARE IMAGE OF OR EQUAL TO

2292; 2291 # SQUARE ORIGINAL OF OR EQUAL TO

2298; 29B8 # CIRCLED DIVISION SLASH

22A2; 22A3 # RIGHT TACK

22A3; 22A2 # LEFT TACK

22A6; 2ADE # ASSERTION

22A8; 2AE4 # TRUE

22A9; 2AE3 # FORCES

22AB; 2AE5 # DOUBLE VERTICAL BAR DOUBLE RIGHT TURNSTILE

22B0; 22B1 # PRECEDES UNDER RELATION

22B1; 22B0 # SUCCEEDS UNDER RELATION

22B2; 22B3 # NORMAL SUBGROUP OF

22B3; 22B2 # CONTAINS AS NORMAL SUBGROUP

22B4; 22B5 # NORMAL SUBGROUP OF OR EQUAL TO

22B5; 22B4 # CONTAINS AS NORMAL SUBGROUP OR EQUAL TO

22B6; 22B7 # ORIGINAL OF

22B7; 22B6 # IMAGE OF

22C9; 22CA # LEFT NORMAL FACTOR SEMIDIRECT PRODUCT

22CA; 22C9 # RIGHT NORMAL FACTOR SEMIDIRECT PRODUCT

22CB; 22CC # LEFT SEMIDIRECT PRODUCT

22CC; 22CB # RIGHT SEMIDIRECT PRODUCT

22CD; 2243 # REVERSED TILDE EQUALS

22D0; 22D1 # DOUBLE SUBSET

22D1; 22D0 # DOUBLE SUPERSET

22D6; 22D7 # LESS-THAN WITH DOT

22D7; 22D6 # GREATER-THAN WITH DOT

22D8; 22D9 # VERY MUCH LESS-THAN

22D9; 22D8 # VERY MUCH GREATER-THAN

22DA; 22DB # LESS-THAN EQUAL TO OR GREATER-THAN

22DB; 22DA # GREATER-THAN EQUAL TO OR LESS-THAN

22DC; 22DD # EQUAL TO OR LESS-THAN

22DD; 22DC # EQUAL TO OR GREATER-THAN

22DE; 22DF # EQUAL TO OR PRECEDES

22DF; 22DE # EQUAL TO OR SUCCEEDS

22E0; 22E1 # [BEST FIT] DOES NOT PRECEDE OR EQUAL

22E1; 22E0 # [BEST FIT] DOES NOT SUCCEED OR EQUAL

22E2; 22E3 # [BEST FIT] NOT SQUARE IMAGE OF OR EQUAL TO

22E3; 22E2 # [BEST FIT] NOT SQUARE ORIGINAL OF OR EQUAL TO

22E4; 22E5 # [BEST FIT] SQUARE IMAGE OF OR NOT EQUAL TO

22E5; 22E4 # [BEST FIT] SQUARE ORIGINAL OF OR NOT EQUAL TO

22E6; 22E7 # [BEST FIT] LESS-THAN BUT NOT EQUIVALENT TO

22E7; 22E6 # [BEST FIT] GREATER-THAN BUT NOT EQUIVALENT TO

22E8; 22E9 # [BEST FIT] PRECEDES BUT NOT EQUIVALENT TO

22E9; 22E8 # [BEST FIT] SUCCEEDS BUT NOT EQUIVALENT TO

22EA; 22EB # [BEST FIT] NOT NORMAL SUBGROUP OF

22EB; 22EA # [BEST FIT] DOES NOT CONTAIN AS NORMAL SUBGROUP

22EC; 22ED # [BEST FIT] NOT NORMAL SUBGROUP OF OR EQUAL TO

22ED; 22EC # [BEST FIT] DOES NOT CONTAIN AS NORMAL SUBGROUP OR EQUAL

22F0; 22F1 # UP RIGHT DIAGONAL ELLIPSIS

22F1; 22F0 # DOWN RIGHT DIAGONAL ELLIPSIS

22F2; 22FA # ELEMENT OF WITH LONG HORIZONTAL STROKE

22F3; 22FB # ELEMENT OF WITH VERTICAL BAR AT END OF HORIZONTAL STROKE

22F4; 22FC # SMALL ELEMENT OF WITH VERTICAL BAR AT END OF HORIZONTAL STROKE

22F6; 22FD # ELEMENT OF WITH OVERBAR

22F7; 22FE # SMALL ELEMENT OF WITH OVERBAR

22FA; 22F2 # CONTAINS WITH LONG HORIZONTAL STROKE

22FB; 22F3 # CONTAINS WITH VERTICAL BAR AT END OF HORIZONTAL STROKE

22FC; 22F4 # SMALL CONTAINS WITH VERTICAL BAR AT END OF HORIZONTAL STROKE

22FD; 22F6 # CONTAINS WITH OVERBAR

22FE; 22F7 # SMALL CONTAINS WITH OVERBAR

2308; 2309 # LEFT CEILING

2309; 2308 # RIGHT CEILING

230A; 230B # LEFT FLOOR

230B; 230A # RIGHT FLOOR

2329; 232A # LEFT-POINTING ANGLE BRACKET

232A; 2329 # RIGHT-POINTING ANGLE BRACKET

2768; 2769 # MEDIUM LEFT PARENTHESIS ORNAMENT

2769; 2768 # MEDIUM RIGHT PARENTHESIS ORNAMENT

276A; 276B # MEDIUM FLATTENED LEFT PARENTHESIS ORNAMENT

276B; 276A # MEDIUM FLATTENED RIGHT PARENTHESIS ORNAMENT

276C; 276D # MEDIUM LEFT-POINTING ANGLE BRACKET ORNAMENT

276D; 276C # MEDIUM RIGHT-POINTING ANGLE BRACKET ORNAMENT

276E; 276F # HEAVY LEFT-POINTING ANGLE QUOTATION MARK ORNAMENT

276F; 276E # HEAVY RIGHT-POINTING ANGLE QUOTATION MARK ORNAMENT

2770; 2771 # HEAVY LEFT-POINTING ANGLE BRACKET ORNAMENT

2771; 2770 # HEAVY RIGHT-POINTING ANGLE BRACKET ORNAMENT

2772; 2773 # LIGHT LEFT TORTOISE SHELL BRACKET

2773; 2772 # LIGHT RIGHT TORTOISE SHELL BRACKET

2774; 2775 # MEDIUM LEFT CURLY BRACKET ORNAMENT

2775; 2774 # MEDIUM RIGHT CURLY BRACKET ORNAMENT

27C3; 27C4 # OPEN SUBSET

27C4; 27C3 # OPEN SUPERSET

27C5; 27C6 # LEFT S-SHAPED BAG DELIMITER

27C6; 27C5 # RIGHT S-SHAPED BAG DELIMITER

27C8; 27C9 # REVERSE SOLIDUS PRECEDING SUBSET

27C9; 27C8 # SUPERSET PRECEDING SOLIDUS

27D5; 27D6 # LEFT OUTER JOIN

27D6; 27D5 # RIGHT OUTER JOIN

27DD; 27DE # LONG RIGHT TACK

27DE; 27DD # LONG LEFT TACK

27E2; 27E3 # WHITE CONCAVE-SIDED DIAMOND WITH LEFTWARDS TICK

27E3; 27E2 # WHITE CONCAVE-SIDED DIAMOND WITH RIGHTWARDS TICK

27E4; 27E5 # WHITE SQUARE WITH LEFTWARDS TICK

27E5; 27E4 # WHITE SQUARE WITH RIGHTWARDS TICK

27E6; 27E7 # MATHEMATICAL LEFT WHITE SQUARE BRACKET

27E7; 27E6 # MATHEMATICAL RIGHT WHITE SQUARE BRACKET

27E8; 27E9 # MATHEMATICAL LEFT ANGLE BRACKET

27E9; 27E8 # MATHEMATICAL RIGHT ANGLE BRACKET

27EA; 27EB # MATHEMATICAL LEFT DOUBLE ANGLE BRACKET

27EB; 27EA # MATHEMATICAL RIGHT DOUBLE ANGLE BRACKET

27EC; 27ED # MATHEMATICAL LEFT WHITE TORTOISE SHELL BRACKET

27ED; 27EC # MATHEMATICAL RIGHT WHITE TORTOISE SHELL BRACKET

27EE; 27EF # MATHEMATICAL LEFT FLATTENED PARENTHESIS

27EF; 27EE # MATHEMATICAL RIGHT FLATTENED PARENTHESIS

2983; 2984 # LEFT WHITE CURLY BRACKET

2984; 2983 # RIGHT WHITE CURLY BRACKET

2985; 2986 # LEFT WHITE PARENTHESIS

2986; 2985 # RIGHT WHITE PARENTHESIS

2987; 2988 # Z NOTATION LEFT IMAGE BRACKET

2988; 2987 # Z NOTATION RIGHT IMAGE BRACKET

2989; 298A # Z NOTATION LEFT BINDING BRACKET

298A; 2989 # Z NOTATION RIGHT BINDING BRACKET

298B; 298C # LEFT SQUARE BRACKET WITH UNDERBAR

298C; 298B # RIGHT SQUARE BRACKET WITH UNDERBAR

298D; 2990 # LEFT SQUARE BRACKET WITH TICK IN TOP CORNER

298E; 298F # RIGHT SQUARE BRACKET WITH TICK IN BOTTOM CORNER

298F; 298E # LEFT SQUARE BRACKET WITH TICK IN BOTTOM CORNER

2990; 298D # RIGHT SQUARE BRACKET WITH TICK IN TOP CORNER

2991; 2992 # LEFT ANGLE BRACKET WITH DOT

2992; 2991 # RIGHT ANGLE BRACKET WITH DOT

2993; 2994 # LEFT ARC LESS-THAN BRACKET

2994; 2993 # RIGHT ARC GREATER-THAN BRACKET

2995; 2996 # DOUBLE LEFT ARC GREATER-THAN BRACKET

2996; 2995 # DOUBLE RIGHT ARC LESS-THAN BRACKET

2997; 2998 # LEFT BLACK TORTOISE SHELL BRACKET

2998; 2997 # RIGHT BLACK TORTOISE SHELL BRACKET

29B8; 2298 # CIRCLED REVERSE SOLIDUS

29C0; 29C1 # CIRCLED LESS-THAN

29C1; 29C0 # CIRCLED GREATER-THAN

29C4; 29C5 # SQUARED RISING DIAGONAL SLASH

29C5; 29C4 # SQUARED FALLING DIAGONAL SLASH

29CF; 29D0 # LEFT TRIANGLE BESIDE VERTICAL BAR

29D0; 29CF # VERTICAL BAR BESIDE RIGHT TRIANGLE

29D1; 29D2 # BOWTIE WITH LEFT HALF BLACK

29D2; 29D1 # BOWTIE WITH RIGHT HALF BLACK

29D4; 29D5 # TIMES WITH LEFT HALF BLACK

29D5; 29D4 # TIMES WITH RIGHT HALF BLACK

29D8; 29D9 # LEFT WIGGLY FENCE

29D9; 29D8 # RIGHT WIGGLY FENCE

29DA; 29DB # LEFT DOUBLE WIGGLY FENCE

29DB; 29DA # RIGHT DOUBLE WIGGLY FENCE

29F5; 2215 # REVERSE SOLIDUS OPERATOR

29F8; 29F9 # BIG SOLIDUS

29F9; 29F8 # BIG REVERSE SOLIDUS

29FC; 29FD # LEFT-POINTING CURVED ANGLE BRACKET

29FD; 29FC # RIGHT-POINTING CURVED ANGLE BRACKET

2A2B; 2A2C # MINUS SIGN WITH FALLING DOTS

2A2C; 2A2B # MINUS SIGN WITH RISING DOTS

2A2D; 2A2E # PLUS SIGN IN LEFT HALF CIRCLE

2A2E; 2A2D # PLUS SIGN IN RIGHT HALF CIRCLE

2A34; 2A35 # MULTIPLICATION SIGN IN LEFT HALF CIRCLE

2A35; 2A34 # MULTIPLICATION SIGN IN RIGHT HALF CIRCLE

2A3C; 2A3D # INTERIOR PRODUCT

2A3D; 2A3C # RIGHTHAND INTERIOR PRODUCT

2A64; 2A65 # Z NOTATION DOMAIN ANTIRESTRICTION

2A65; 2A64 # Z NOTATION RANGE ANTIRESTRICTION

2A79; 2A7A # LESS-THAN WITH CIRCLE INSIDE

2A7A; 2A79 # GREATER-THAN WITH CIRCLE INSIDE

2A7D; 2A7E # LESS-THAN OR SLANTED EQUAL TO

2A7E; 2A7D # GREATER-THAN OR SLANTED EQUAL TO

2A7F; 2A80 # LESS-THAN OR SLANTED EQUAL TO WITH DOT INSIDE

2A80; 2A7F # GREATER-THAN OR SLANTED EQUAL TO WITH DOT INSIDE

2A81; 2A82 # LESS-THAN OR SLANTED EQUAL TO WITH DOT ABOVE

2A82; 2A81 # GREATER-THAN OR SLANTED EQUAL TO WITH DOT ABOVE

2A83; 2A84 # LESS-THAN OR SLANTED EQUAL TO WITH DOT ABOVE RIGHT

2A84; 2A83 # GREATER-THAN OR SLANTED EQUAL TO WITH DOT ABOVE LEFT

2A8B; 2A8C # LESS-THAN ABOVE DOUBLE-LINE EQUAL ABOVE GREATER-THAN

2A8C; 2A8B # GREATER-THAN ABOVE DOUBLE-LINE EQUAL ABOVE LESS-THAN

2A91; 2A92 # LESS-THAN ABOVE GREATER-THAN ABOVE DOUBLE-LINE EQUAL

2A92; 2A91 # GREATER-THAN ABOVE LESS-THAN ABOVE DOUBLE-LINE EQUAL

2A93; 2A94 # LESS-THAN ABOVE SLANTED EQUAL ABOVE GREATER-THAN ABOVE SLANTED EQUAL

2A94; 2A93 # GREATER-THAN ABOVE SLANTED EQUAL ABOVE LESS-THAN ABOVE SLANTED EQUAL

2A95; 2A96 # SLANTED EQUAL TO OR LESS-THAN

2A96; 2A95 # SLANTED EQUAL TO OR GREATER-THAN

2A97; 2A98 # SLANTED EQUAL TO OR LESS-THAN WITH DOT INSIDE

2A98; 2A97 # SLANTED EQUAL TO OR GREATER-THAN WITH DOT INSIDE

2A99; 2A9A # DOUBLE-LINE EQUAL TO OR LESS-THAN

2A9A; 2A99 # DOUBLE-LINE EQUAL TO OR GREATER-THAN

2A9B; 2A9C # DOUBLE-LINE SLANTED EQUAL TO OR LESS-THAN

2A9C; 2A9B # DOUBLE-LINE SLANTED EQUAL TO OR GREATER-THAN

2AA1; 2AA2 # DOUBLE NESTED LESS-THAN

2AA2; 2AA1 # DOUBLE NESTED GREATER-THAN

2AA6; 2AA7 # LESS-THAN CLOSED BY CURVE

2AA7; 2AA6 # GREATER-THAN CLOSED BY CURVE

2AA8; 2AA9 # LESS-THAN CLOSED BY CURVE ABOVE SLANTED EQUAL

2AA9; 2AA8 # GREATER-THAN CLOSED BY CURVE ABOVE SLANTED EQUAL

2AAA; 2AAB # SMALLER THAN

2AAB; 2AAA # LARGER THAN

2AAC; 2AAD # SMALLER THAN OR EQUAL TO

2AAD; 2AAC # LARGER THAN OR EQUAL TO

2AAF; 2AB0 # PRECEDES ABOVE SINGLE-LINE EQUALS SIGN

2AB0; 2AAF # SUCCEEDS ABOVE SINGLE-LINE EQUALS SIGN

2AB3; 2AB4 # PRECEDES ABOVE EQUALS SIGN

2AB4; 2AB3 # SUCCEEDS ABOVE EQUALS SIGN

2ABB; 2ABC # DOUBLE PRECEDES

2ABC; 2ABB # DOUBLE SUCCEEDS

2ABD; 2ABE # SUBSET WITH DOT

2ABE; 2ABD # SUPERSET WITH DOT

2ABF; 2AC0 # SUBSET WITH PLUS SIGN BELOW

2AC0; 2ABF # SUPERSET WITH PLUS SIGN BELOW

2AC1; 2AC2 # SUBSET WITH MULTIPLICATION SIGN BELOW

2AC2; 2AC1 # SUPERSET WITH MULTIPLICATION SIGN BELOW

2AC3; 2AC4 # SUBSET OF OR EQUAL TO WITH DOT ABOVE

2AC4; 2AC3 # SUPERSET OF OR EQUAL TO WITH DOT ABOVE

2AC5; 2AC6 # SUBSET OF ABOVE EQUALS SIGN

2AC6; 2AC5 # SUPERSET OF ABOVE EQUALS SIGN

2ACD; 2ACE # SQUARE LEFT OPEN BOX OPERATOR

2ACE; 2ACD # SQUARE RIGHT OPEN BOX OPERATOR

2ACF; 2AD0 # CLOSED SUBSET

2AD0; 2ACF # CLOSED SUPERSET

2AD1; 2AD2 # CLOSED SUBSET OR EQUAL TO

2AD2; 2AD1 # CLOSED SUPERSET OR EQUAL TO

2AD3; 2AD4 # SUBSET ABOVE SUPERSET

2AD4; 2AD3 # SUPERSET ABOVE SUBSET

2AD5; 2AD6 # SUBSET ABOVE SUBSET

2AD6; 2AD5 # SUPERSET ABOVE SUPERSET

2ADE; 22A6 # SHORT LEFT TACK

2AE3; 22A9 # DOUBLE VERTICAL BAR LEFT TURNSTILE

2AE4; 22A8 # VERTICAL BAR DOUBLE LEFT TURNSTILE

2AE5; 22AB # DOUBLE VERTICAL BAR DOUBLE LEFT TURNSTILE

2AEC; 2AED # DOUBLE STROKE NOT SIGN

2AED; 2AEC # REVERSED DOUBLE STROKE NOT SIGN

2AF7; 2AF8 # TRIPLE NESTED LESS-THAN

2AF8; 2AF7 # TRIPLE NESTED GREATER-THAN

2AF9; 2AFA # DOUBLE-LINE SLANTED LESS-THAN OR EQUAL TO

2AFA; 2AF9 # DOUBLE-LINE SLANTED GREATER-THAN OR EQUAL TO

2E02; 2E03 # LEFT SUBSTITUTION BRACKET

2E03; 2E02 # RIGHT SUBSTITUTION BRACKET

2E04; 2E05 # LEFT DOTTED SUBSTITUTION BRACKET

2E05; 2E04 # RIGHT DOTTED SUBSTITUTION BRACKET

2E09; 2E0A # LEFT TRANSPOSITION BRACKET

2E0A; 2E09 # RIGHT TRANSPOSITION BRACKET

2E0C; 2E0D # LEFT RAISED OMISSION BRACKET

2E0D; 2E0C # RIGHT RAISED OMISSION BRACKET

2E1C; 2E1D # LEFT LOW PARAPHRASE BRACKET

2E1D; 2E1C # RIGHT LOW PARAPHRASE BRACKET

2E20; 2E21 # LEFT VERTICAL BAR WITH QUILL

2E21; 2E20 # RIGHT VERTICAL BAR WITH QUILL

2E22; 2E23 # TOP LEFT HALF BRACKET

2E23; 2E22 # TOP RIGHT HALF BRACKET

2E24; 2E25 # BOTTOM LEFT HALF BRACKET

2E25; 2E24 # BOTTOM RIGHT HALF BRACKET

2E26; 2E27 # LEFT SIDEWAYS U BRACKET

2E27; 2E26 # RIGHT SIDEWAYS U BRACKET

2E28; 2E29 # LEFT DOUBLE PARENTHESIS

2E29; 2E28 # RIGHT DOUBLE PARENTHESIS

3008; 3009 # LEFT ANGLE BRACKET

3009; 3008 # RIGHT ANGLE BRACKET

300A; 300B # LEFT DOUBLE ANGLE BRACKET

300B; 300A # RIGHT DOUBLE ANGLE BRACKET

300C; 300D # [BEST FIT] LEFT CORNER BRACKET

300D; 300C # [BEST FIT] RIGHT CORNER BRACKET

300E; 300F # [BEST FIT] LEFT WHITE CORNER BRACKET

300F; 300E # [BEST FIT] RIGHT WHITE CORNER BRACKET

3010; 3011 # LEFT BLACK LENTICULAR BRACKET

3011; 3010 # RIGHT BLACK LENTICULAR BRACKET

3014; 3015 # LEFT TORTOISE SHELL BRACKET

3015; 3014 # RIGHT TORTOISE SHELL BRACKET

3016; 3017 # LEFT WHITE LENTICULAR BRACKET

3017; 3016 # RIGHT WHITE LENTICULAR BRACKET

3018; 3019 # LEFT WHITE TORTOISE SHELL BRACKET

3019; 3018 # RIGHT WHITE TORTOISE SHELL BRACKET

301A; 301B # LEFT WHITE SQUARE BRACKET

301B; 301A # RIGHT WHITE SQUARE BRACKET

FE59; FE5A # SMALL LEFT PARENTHESIS

FE5A; FE59 # SMALL RIGHT PARENTHESIS

FE5B; FE5C # SMALL LEFT CURLY BRACKET

FE5C; FE5B # SMALL RIGHT CURLY BRACKET

FE5D; FE5E # SMALL LEFT TORTOISE SHELL BRACKET

FE5E; FE5D # SMALL RIGHT TORTOISE SHELL BRACKET

FE64; FE65 # SMALL LESS-THAN SIGN

FE65; FE64 # SMALL GREATER-THAN SIGN

FF08; FF09 # FULLWIDTH LEFT PARENTHESIS

FF09; FF08 # FULLWIDTH RIGHT PARENTHESIS

FF1C; FF1E # FULLWIDTH LESS-THAN SIGN

FF1E; FF1C # FULLWIDTH GREATER-THAN SIGN

FF3B; FF3D # FULLWIDTH LEFT SQUARE BRACKET

FF3D; FF3B # FULLWIDTH RIGHT SQUARE BRACKET

FF5B; FF5D # FULLWIDTH LEFT CURLY BRACKET

FF5D; FF5B # FULLWIDTH RIGHT CURLY BRACKET

FF5F; FF60 # FULLWIDTH LEFT WHITE PARENTHESIS

FF60; FF5F # FULLWIDTH RIGHT WHITE PARENTHESIS

FF62; FF63 # [BEST FIT] HALFWIDTH LEFT CORNER BRACKET

FF63; FF62 # [BEST FIT] HALFWIDTH RIGHT CORNER BRACKET

- (informative)

Example CFF2 Font

This annex illustrates the CFF2 format with an example CFF2 table.

Binary dump (226 bytes):

0000: 02 00 05 00 07 CF 0C 24 C3 11 9B 18 00 00 00 00
0010: 00 26 00 01 00 00 00 0C 00 01 00 00 00 1C 00 01
0020: 00 02 C0 00 E0 00 00 00 C0 00 C0 00 E0 00 00 00
0030: 00 00 00 02 00 00 00 01 00 00 00 02 01 01 03 05
0040: 20 0A 20 0A 00 00 00 01 01 01 05 F7 06 DA 12 77
0050: 9F F8 6C 9D AE 9A F4 9A 95 9F B3 9F 8B 8B 8B 8B
0060: 85 9A 8B 8B 97 73 8B 8B 8C 80 8B 8B 8B 8D 8B 8B
0070: 8C 8A 8B 8B 97 17 06 FB 8E 95 86 9D 8B 8B 8D 17
0080: 07 77 9F F8 6D 9D AD 9A F3 9A 95 9F B3 9F 08 FB
0090: 8D 95 09 1E A0 37 5F 0C 09 8B 0C 0B C2 6E 9E 8C
00A0: 17 0A DB 57 F7 02 8C 17 0B B3 9A 77 9F 82 8A 8D
00B0: 17 0C 0C DB 95 57 F7 02 85 8B 8D 17 0C 0D F7 06
00C0: 13 00 00 00 01 01 01 1B BD BD EF 8C 10 8B 15 F8
00D0: 88 27 FB 5C 8C 10 06 F8 88 07 FC 88 EF F7 5C 8C
00E0: 10 06

Analysis:

| Hex data | Source | Comments |
| --- | --- | --- |
| 02 | majorVersion | = 2 |
| 00 | minorVersion | = 0 |
| 05 | headerSize | = 5 |
| 00 07 | topDictSize | = 7 |
| CF 0C 24 | [ offset ] FontDICTINDEXOffset | = 68 FontDICTINDEXOffset The bytes 0C 24 represent the FontDICTINDEXOffset operator. The operand byte is hex CF = decimal 207, which is decoded using the rule for byte values from 32 to 246 (see Table 3, Operand Encoding): b0 - 139.This gives the offset of the FontDICTINDEX: decimal 68 = hex 44. |
| C3 11 | [ offset ] CharStringINDEXOffset | = 56 CharStringINDEXOffset The byte 11 represents the CharStringINDEXOffset operator. The operand byte is hex C3, which is the encoded representation of the value 56.This gives the offset of the CharStringINDEX: decimal 56 = hex 38. |
| 9B 18 | [ offset ] VariationStoreOffset | = 16 vstore The byte 18 represents the VariationStoreOffset operator. The operand byte is hex 9B, which is the encoded representation of the value 16.This gives the offset of the VariationStore data: decimal 16 = hex 10. |
|  | GlobalSubrINDEX | CFF2 offsets: 000C to 000F |
| 00 00 00 00 | count | = empty INDEX; no additional fields represented. |
|  | VariationStore data | CFF2 offsets: 0010 to 0037 |
| 00 26 | length | = 38 — length in bytes of the Item Variation Store structure that follows. |
|  | Item Variation Store | CFF2 offsets: 0012 to 0037 |
| 00 01 | format | = 1 |
| 00 00 00 0C | variationRegionListOffset | = 12 — offset in bytes from the start of the Item Variation Store. |
| 00 01 | itemVariationDataCount | = 1 — number of ItemVariationData subtables. |
| 00 00 00 1C | itemVariationDataOffsets[0] | = 28 — offset in bytes from start of the ItemVariationStore to ItemVariationData subtable 0. |
|  | VariationRegionList | CFF2 offsets: 001E to 002D |
| 00 01 | axisCount | = 1 |
| 00 02 | regionCount | = 2 |
|  | variationRegions[0] | CFF2 offsets: 0022 to 0027 |
|  | regionAxes[0] | CFF2 offsets: 0022 to 0027 |
| C0 00 | startCoord | = -1.0 (F2DOT14 value) |
| E0 00 | peakCoord | = -0.5 (F2DOT14 value) |
| 00 00 | endCoord | = 0.0 (F2DOT14 value) |
|  | variationRegions[1] | CFF2 offsets: 0028 to 002D |
|  | regionAxes[0] | CFF2 offsets: 0028 to 002D |
| C0 00 | startCoord | = -1.0 (F2DOT14 value) |
| C0 00 | peakCoord | = -1.0 (F2DOT14 value) |
| E0 00 | endCoord | = -0.5 (F2DOT14 value) |
|  | ItemVariationData subtable 0 | CFF2 offsets: 002E to 0037 |
| 00 00 | itemCount | = 0 |
| 00 00 | shortDeltaCount | = 0 |
| 00 02 | regionIndexCount | = 2 |
| 00 00 00 01 | regionIndexes[] | = {0, 1} |
|  | CharStringINDEX | CFF2 offsets: 0038 to 0043 |
| 00 00 00 02 | count | = 2 |
| 01 | offSize | = 1 |
| 01 03 05 | offsets[] | = {1, 3, 5} (number of elements is count + 1) |
|  | CharString 0 | CFF2 offsets: 0040 to 0041 |
| 20 0A | [ subr# ] callsubr | = -107 callsubr The byte 0A represents the callsubr operator. The operand byte is 20, which is the encoded representation of the value -107. |
|  | CharString 1 | CFF2 offsets: 0042 to 0043 |
| 20 0A | [ subr# ] callsubr | = -107 callsubr The byte 0A represents the callsubr operator. The operand byte is 20, which is the encoded representation of the value -107. |
|  | FontDICTINDEX | CFF2 offsets: 0044 to 004E |
| 00 00 00 01 | count | = 1 |
| 01 | offSize | = 1 |
| 01 05 | offsets[] | = {1, 5} (number of elements is count + 1) |
|  | FontDICT 0 | CFF2 offsets: 004B to 004E |
| F7 06 DA 12 | [ size offset ] PrivateDICTOffset | = 114 79 PrivateDICTOffset The byte 12 represents the PrivateDICTOffset operator. The operand bytes F7 06 are the encoded representation of the value 114. The operand byte DA is the encoded representation of the value 79.This gives the size and offset of a PrivateDICT: size is 114 bytes, offset (from start of CFF2 table) is 79 bytes (hex 4F). |
| 77 9F F8 6C 9D AE 9A F4 9A 95 9F B3 9F 8B 8B 8B 8B 85 9A 8B 8B 97 73 8B 8B 8C 80 8B 8B 8B 8D 8B 8B 8C 8A 8B 8B 97 17 06 | [ num* ] blend BlueValues | = -20 20 472 18 35 15 105 15 10 20 40 20 0 0 0 0 -6 15 0 0 12 -24 0 0 1 -11 0 0 0 2 0 0 1 -1 0 0 12 blend BlueValues The byte 06 represents the BlueValues operator. The byte 17 represents the blend operator. The operand bytes 77.. 97 are the encoded representation of the values -20… 12. |
| FB 8E 95 86 9D 8B 8B 8D 17 07 | [ num* ] blend OtherBlues | = -250 10 -5 18 0 0 2 blend OtherBlues The byte 07 represents the OtherBlues operator. The byte 17 represents the blend operator. The operand bytes FB.. 8D are the encoded representation of the values -250… 2. |
| 77 9F F8 6D 9D AD 9A F3 9A 95 9F B3 9F 08 | [ num* ] FamilyBlues | = -20 20 473 18 34 15 104 15 10 20 40 20 FamilyBlues The byte 08 represents the operator FamilyBlues. The operand bytes 77… 9F are the encoded representation of the values -20… 20. |
| FB 8D 95 09 | [ num* ] FamilyOtherBlues | = -249 10 FamilyOtherBlues The byte 09 represents the FamilyOtherBlues operator. The operand bytes FB 8D are the encoded representation of the value -249. The operand byte 95 is the encoded representation of the value 10. |
| 1E A0 37 5F 0C 09 | [ num ] BlueScale | = 0.0375 BlueScale The bytes 0C 09 represent the BlueScale operator. The operand bytes 1E A0 37 5F are the encoded representation of the value 0.0375. |
| 8B 0C 0B | [ num ] BlueFuzz | = 0 BlueFuzz The bytes 0C 0B represents the BlueFuzz operator. The operand byte 8B is the encoded representation of the value 0. |
| C2 6E 9E 8C 17 0A | [ num* ] blend StdHW | = 55 -29 19 1 blend StdHW The byte 0A represents the StdHW operator. The byte 17 represents the blend operator. The operand bytes C2 6E 9E 8C are the encoded representation of the values 55 -29 19 1. |
| DB 57 F7 02 8C 17 0B | [ num* ] blend StdVW | = 80 -52 110 1 blend StdVW The byte 0B represents the StdVW operator. The byte 17 represents the blend operator. The operand bytes DB 57 F7 02 8C are the encoded representation of the values 80 -52 110 1. |
| B3 9A 77 9F 82 8A 8D 17 0C 0C | [ num* ] blend StemSnapH | = 40 15 -20 20 -9 -1 2 blend StemSnapH The bytes 0C 0C represent the StemSnapH operator. The byte 17 represents the blend operator. The operand bytes B3 9A 77 9F 82 8A 8D are the encoded representation of the values 40 15 -20 20 -9 -1 2. |
| DB 95 57 F7 02 85 8B 8D 17 0C 0D | [ num* ] blend StemSnapV | = 80 10 -52 110 -6 0 2 blend StemSnapV The bytes 0C 0D represent the StemSnapV operator. The byte 17 represents the blend operator. The operand bytes DB 95 57 F7 02 85 8B 8D are the encoded representation of the values 80 10 -52 110 -6 0 2. |
| F7 06 13 | [ offset ] LocalSubrINDEXOffset | = 114 Subrs The byte 13 represents the LocalSubrINDEXOffset operator. The operand bytes F7 06 are the encoded representation of the value 114.This gives the offset (from the start of the PrivateDICT) to the INDEX containing the local subroutines: 114 bytes (hex 72). |
|  | LocalSubrINDEX | CFF2 offsets: 00C1 to 00E1 |
| 00 00 00 01 | count | = 1 |
| 01 | offSize | = 1 |
| 01 1B | offsets[] | = {1, 27} (number of elements is count + 1) |
|  | subr 0 | CFF2 offsets: 00C8 to 00E1 |
| BD BD EF 8C 10 8B 15 | [ num* ] blend num rmoveto | = 50 50 100 1 blend 0 rmoveto The byte 15 represents the rmoveto CharString operator. The byte 10 represents the blend CharString operator. The operand bytes BD BD EF 8C are the encoded representation of the values 50 50 100 1. The operand byte 8B is the encoded representation of the value 0. |
| F8 88 07 | [ num ] vlineto | = 500 vlineto The byte 07 represents the vlineto CharString operator. The operand bytes F8 88 are the encoded representation of the value 500. |
| FC 88 EF F7 5C 8C 10 06 | [ num* ] blend hlineto | = -500 100 200 1 blend hlineto The byte 06 represents the hlineto CharString operator. The byte 10 represents the blend CharString operator. The operand bytes FC 88 EF F7 5C 8 C are the encoded representation of the values -500 100 200 1. |

- (informative)

Registration of Media Type: application/font-sfnt

The content of this Annex (presented below in its entirety) represents a text of new media type registration for a 'font-sfnt" subtype of the "application" tree. Since the new top-level media type "font" tree is now available as introduced by the RFC 8081 (https://tools.ietf.org/html/rfc8081), we recommend using the "font/sfnt" subtype as a generic font media type.

The use of "application/font-sfnt" media type is now DEPRECATED and the text of its registration below provided only for historic references.

This appendix registers a new MIME media type, in conformance with http://www.ietf.org/rfc/rfc4288.txt.

Type name:

application

Subtype name:

font-sfnt

Required parameters:

None

Optional parameters:

1) 	Name: 	Outlines
Value: 	TTF, CFF

2) 	Name: 	Layout
Value: 	OTF, AAT, SIL

Encoding considerations:

binary

Security considerations:

Fonts are collections of different tables containing data structures that represent different types of information, including glyph outlines in various data formats, hinting instructions, metrics and layout information for multiple languages and writing systems, rules for glyph substitution and positioning, etc. Depending on the data format used to represent the glyph data the font may contain either TrueType or PostScript outlines and their respective hint instructions. There are many existing, already standardized font table tags and formats that allow an unspecified number of entries containing predefined data fields for storage of variable length binary data.

Many existing (TrueType, OpenType and OFF, SIL Graphite, WOFF and many other) font formats are based on the table-based SFNT (scalable font) format which is extremely flexible, highly extensible and offers an opportunity to introduce additional table structures when needed, in a way that would not affect existing font rendering engines and text layout implementations. However, this very extensibility may present specific security concerns – the flexibility and ease of adding new data structures makes it easy for any arbitrary data to be hidden inside a font file. There is a significant risk that the flexibility of font data structures may be exploited to hide malicious binary content disguised as a font data component.

Fonts may contain 'hints' – programmatic instructions that are executed by the font engine for the alignment of graphical elements of glyph outlines with the target display pixel grid. Depending on the font technology utilized in the creation of a font these hints may represent active code interpreted and executed by the font rasterizer. Even though hints operate within the confines of the glyph outline conversion system and have no access outside the font rendering engine, hint instructions can be, however, quite complex, and a maliciously designed complex font could cause undue resource consumption (e.g. memory or CPU cycles) on a machine interpreting it. Indeed, fonts are sufficiently complex, and most (if not all) interpreters cannot be completely protected from malicious fonts without undue performance penalties.

Widespread use of fonts as necessary component of visual content presentation warrants that a careful attention should be given to security considerations whenever a font is either embedded into an electronic document or transmitted alongside media content.

Interoperability considerations:

As it was noted in the first paragraph of the "Security considerations" section, the same font format wrapper can be used to encode fonts with different types of glyph data represented as either TrueType or PostScript (CFF) outlines. Existing font rendering engines may not be able to process some of the particular outline formats, and downloading a font resource that contains unsupported glyph data format would result in inability of application to render and display text. Therefore, it would be extremely useful to clearly identify the format of the glyph outline data within a font using an optional parameter, and allow applications to make decisions about downloading a particular font resource sooner. Similar, another optional parameter is suggested to identify the type of text shaping and layout mechanism that is supported by a font. Please note that as new outline formats and text shaping mechanisms may be defined in the future, the set of allowed values for two optional parameters defined by this application may be extended.

Published specification:

As of the date of this submission, the main published specification is ISO/IEC 14496-22:2009 "Open Font Format". This media type registration is extracted from the ISO/IEC 14496-22/AMD2 of Open Font Format (OFF) specification being developed by ISO/IEC SC29/WG11.

Applications that use this media type:

Any and all applications that are able to create, edit or display textual media content.

Additional information:

Magic number(s):

The TrueType fonts and OFF / OpenType fonts containing TrueType outlines should use 0x00010000 as the 'sfnt' version number.

The OFF / OpenType fonts containing CFF data should use the tag 'OTTO' as 'sfnt' version number.

File extension(s):

Font file extensions used for OFF / OpenType fonts: .ttf, .otf

Typically, .ttf extension is only used for fonts containing TrueType outlines, while .otf extension can be used for any OpenType/OFF font, either with TrueType or CFF outlilnes.

Macintosh file type code(s):

(no code specified)

Intended usage:

COMMON

Restrictions on usage:

None

Author:

The ISO/IEC 14496-22 "Open Font Format" specification is a product of the ISO/IEC JTC1 SC29/WG11.

Change controller:

The ISO/IEC has change control over the above-mentioned standard.

# Bibliography

[MS-LCID]: Windows Language Code Identifier (LCID) Reference,  https://learn.microsoft.com/en-us/openspecs/windows_protocols/ms-lcid/70feba9f-294e-491e-b6eb-56532684c37f

Apple’s TrueType Reference Manual, chapter 6: The ‘post’ table - https://developer.apple.com/fonts/TrueType-Reference-Manual/RM06/Chap6post.html

Adobe Glyph List Specification - 
https://github.com/adobe-type-tools/agl-specification

Type 2 Charstring format, http://partners.adobe.com/public/developer/en/font/5177.Type2.pdf>

The Compact Font Format specification, <http://partners.adobe.com/public/developer/en/font/5176.CFF.pdf>

Registered font vendors, http://www.microsoft.com/typography/links/vendorlist.aspx

Apple's TrueType Reference Manual -  https://developer.apple.com/fonts/TrueType-Reference-Manual/

TrueType Font Files, Technical Specification  V1.66 http://www.microsoft.com/typography/SpecificationsOverview.mspx

OpenType Layout Font Specification http://www.microsoft.com/typography/otspec/TTOCHAP1.htm

Script-specific Development: http://www.microsoft.com/typography/SpecificationsOverview.mspx

Feature Tags: http://www.microsoft.com/typography/developers/OpenType/featuretags.aspx

PKCS#7 signatures: ftp://ftp.rsa.com/pub/pkcs/ascii/pkcs-7.asc

Counter-signatures: ftp://ftp.rsa.com/pub/pkcs/ascii/pkcs-9.asc

PANOSE Classification Metrics Guide:  http://www.panose.com/

Portable Network Graphics (PNG) specification: http://www.w3.org/TR/PNG/

Conformance requirements for SVG viewers: http://www.w3.org/TR/SVG11/conform.html#ConformingSVGViewers

SVG Integration: http://www.w3.org/TR/svg-integration/

CSS Custom Properties for Cascading Variables Module Level 1 specification: http://www.w3.org/TR/css-variables-1/

Scalable Vector Graphics (SVG) 2 specification: http://www.w3.org/TR/SVG2

Unicode Character Database file for joining-script properties http://www.unicode.org/Public/UCD/latest/ucd/ArabicShaping.txt

Requirements for Japanese Text Layout (JLREQ): https://www.w3.org/TR/jlreq/

Unicode Character Database (Unicode Annex #44) http://www.unicode.org/reports/tr44/tr44-18.html

Unicode Standard Annex #50: Unicode Vertical Text Layout. http://www.unicode.org/reports/tr50/

The WOFF2 specification: http://www.w3.org/TR/WOFF2/

IETF BCP 47 specification, “Tags for Identifying Languages”. 
https://www.rfc-editor.org/info/bcp47

IANA Language Subtag Registry.
http://www.iana.org/assignments/language-subtag-registry/language-subtag-registry

Adobe Technical Note #5902: “PostScript Name Generation for Variation Fonts”. http://partners.adobe.com/public/developer/en/font/5902.AdobePSNameGeneration.pdf

CFF2 changes from CFF 1.0 https://www.microsoft.com/typography/otspec/cff2.htm#appendixD

Example CFF2 Font: 
https://www.microsoft.com/typography/otspec/cff2.htm#appendixA

Adobe Technical Note #5015: "Type 1 Font Format Supplement".
http://partners.adobe.com/public/developer/en/font/5015.Type1_Supp.pdf

RFC 1952: "GZIP file format specification version 4.3".
https://datatracker.ietf.org/doc/html/rfc1952

HTML Living Standard, 4.12.5, The canvas element. https://html.spec.whatwg.org/multipage/canvas.html#the-canvas-element

Compositing and Blending Level 1. W3C Candidate Recommendation, 13 January 2015. 
https://www.w3.org/TR/compositing-1/

CSS Color Module Level 4. W3C Working Draft Recommendation, 12 November 2020.
https://www.w3.org/TR/css-color-4/

TrueType Fundamentals.
http://www.microsoft.com/typography/otspec/TTCH01.htm

Instructing TrueType Glyphs.
https://learn.microsoft.com/typography/opentype/spec/tt_instructing_glyphs

OpenType® is the trademark of a product supplied by Microsoft. This information is given for the convenience of users of this document and does not constitute an endorsement by ISO or IEC of this product.

TrueType® is the trademark of a product supplied by Apple Inc. This information is given for the convenience of users of this document and does not constitute an endorsement by ISO or IEC of this product.

Also available as a W3C Recommendation (Reference [15]).

MicroType® is the trademark of a product supplied by Monotype Imaging Inc. This information is given for the convenience of users of this document and does not constitute an endorsement by ISO or IEC of this product.

ClearType® is the trademark of a product supplied by Microsoft. This information is given for the convenience of users of this document and does not constitute an endorsement by ISO or IEC of this product.