ISO/IEC JTC 1/SC 29/WG 03 N00849

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Technologies under considerations on Carriage of geometry-based point cloud compression data

Status:	Approved

Date of document:	2023-10-17

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	26 (with cover page)

Email of Convenor:	young.L @ samsung . com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 3

MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 3 N00849

Antalya  – April 2023

Source:		WG 03, MPEG Systems

Status:		Approved

Title: 	Technologies under Consideration for Carriage of Geometry-based Point Cloud Compression

Serial Number: 	22591

# Introduction

This document includes technologies under consideration on carriage of G-PCC data. This includes the following:

- Partial access based on LoD (m54394)

- Grouping tracks for the same point cloud (m54294)

- Signaling of G-PCC Tile inventory information (m54953)

- On the signalling of decoder configuration record for multiple tracks (m57438)

- G-PCC file format miscellaneous [correction to number of spatial regions signalling] (m57474)

- Signaling Alternatives for G-PCC Content (m57825)

- Signaling Attribute Instances of the Same Attribute Type (m57897)

- On point reliability (m59523)

- On point reliability for applications fusing data source (m62013)

- Derived items for PCC (m62341)

# Partial Access of G-PCC

## Partial Access Based on LoD (m54394)

In the latest G-PCC specification, the spatial scalability functionality is supported by using Lifting Transform with lifting scalability enabled. To achieve harmonized spatial scalability, the LoD generation process of attribute data is extended to align with the geometry Octree structure. Thus, when the partial octree decoded point cloud is given to the attribute decoder, it can be constructed the Lifting LoD from the given level in the decoder side correctly.

According to the structure of geometry data unit, one unit can be naturally divided into multiple levels based on the depth of octree. And one attribute data unit using Lifting Transform can be divided into multiple refinement levels corresponding to the depth of octree of geometry data. However, due to the lack of signaling of LoD in system level, a point cloud can only be delivered or accessed with complete levels. For some scenarios that do not require high resolution of the point cloud, this will bring unnecessary burden to the network and decoder, especially when the source point cloud is dense even in the local area.

Therefore, for future improvements in availability, performance and efficiency of G-PCC data delivering services across various networks and to customers using a broad range of decoding and viewing devices, it is essential to identify levels of details of G-PCC data in system level.

### Multiple Track Encapsulation of Scalable G-PCC bitstream

#### General

Fig 1 Structures of scalable G-PCC bitstream with multiple track encapsulation

As shown in Fig 1, a scalable G-PCC bitstream could be represented by multiple tracks in a file. Each track represents a level of details of the scalable stream. In that case, the decoding process of higher levels of G-PCC data requires data with lower LoD values. Thus different tracks may logically share data using extractors.

When a content provider wants to provide a G-PCC bitstream with attribute data encoded with lifting scalability, tracks can be created according to the multiple levels of detail. And a track with higher level of G-PCC data shall contain extractors which indicate instructions to extract data of lower levels from another track. Tracks with lower level of detail shall be linked to the track with higher level using a track reference of type 'scal'. When a G-PCC bitstream is represented by multiple levels in multiple tracks, the player must reconstruct G-PCC bitstream before passing them to the decoder.

There is a set of one or more tracks that, when taken together, contain the complete set of encoded point cloud data. Let the lowest operating point be the one of all the operating points represented by level of details that has the least value of LoD. The complete encoded information can be retained when the tracks included in the complete track group are retained.

#### Scalable G-PCC stream track for multi-track encapsulation

##### Sample entry

###### Definition

Sample Entry Type:	'gpcs'
Container: 		SampleDescriptionBox ('stsd')
Mandatory:		Yes
Quantity:		One or more sample entries may be present

Under the ‘gpcs’ sample entry, all parameter sets shall be in the setupUnit array. The G-PCC bitstream contained in the track shall support spatial scalability functionality, and the track represents a level of detail of the scalable bitstream. Extractors may be present to reference G-PCC data units.

###### Syntax

aligned(8) class GPCCSampleEntry()

extends VolumetricVisualSampleEntry (‘gpcs’) {

GPCCConfigurationBox config; 	//mandatory

GPCCComponentTypeBox type;	// optional

unsigned int(16) lod;

}

###### Semantics

compressorname in the base class VolumetricVisualSampleEntry indicates the name of the compressor used with the value "\013GPCC Coding" being recommended; the first byte is a count of the remaining bytes, here represented by \013, which (being octal 13) is 11 (decimal), the number of bytes in the rest of the string.

config includes G-PCC decoder configuration record information.

type indicates the type of G-PCC component carried in the respective track.

lod indicates the maximum value of level of detail of geometry data and attribute data in the track.

##### Extractor

###### Definition

Extractors in G-PCC bitstream is defined to enable compact formation of tracks that extract, by reference, geometry data unit or attribute data unit from other tracks. When an extractor is processed by a file reader that requires it, the extractor is logically replaced by the bytes it references. An extractor may contain one or more constructors for extracting data from another track that is linked to the track in which the extractor resides by means of a track reference of type 'scal'. Extractor extends the syntax structure type of TLV as follows.

TLV type:

Table 1 Mapping of tlv_type and associated data unit

| tlv_type | Description |
| --- | --- |
| 0 | Sequence parameter set |
| 1 | Geometry parameter set |
| 2 | Geometry data unit or partial geometry data unit |
| 3 | Attribute parameter set |
| 4 | Attribute data unit or partial attribute data unit |
| 5 | Tile inventory |
| 6 | Frame boundary marker |
| 7 | Extractor |

###### Syntax

class aligned(8) Extractor () {
	unsigned int(16) constructor_count;

for(i = 0; i < constructor_count; i++ ){

unsigned int(8) track_ref_index;

signed   int(8) sample_offset;

unsigned int(32) data_offset;

unsigned int(32) data_length;

}

}

###### Semantics

constructor_type specifies the number of constructor that follows.

track_ref_index specifies the index of the track reference of type 'scal' to use to find the track from which to extract data. The sample in that track from which data is extracted is temporally aligned or nearest preceding in the media decoding timeline, i.e. using the time-to-sample table only, adjusted by an offset specified by sample_offset with the sample containing the Extractor. The first track reference has the index value 1; the value 0 is reserved.

sample_offset gives the relative index of the sample in the linked track that shall be used as the source of information. Sample 0 (zero) is the sample with the same, or the closest preceding, decoding time compared to the decoding time of the sample containing the extractor; sample 1 (one) is the next sample, sample -1 (minus 1) is the previous sample, and so on.

data_offset: The offset of the first byte within the reference sample to copy. If the extraction starts with the first byte of data in that sample, the offset takes the value 0.

data_length: The number of bytes to copy. When data_offset + data_length is greater than the size of the sample, the bytes from the byte pointed to by data_offset until the end of the sample, inclusive, are copied.

##### Complete track group

###### Definition

TrackGroupTypeBox with track_group_type equal to 'cptg' indicates that this track is part of the same scalable G-PCC bitstream. The complete encoded information can be retained when the tracks included in the ‘complete subset’ are retained.

###### Syntax

aligned(8) class ScalableGroupBox

extends TrackGroupTypeBox('cptg') {

unsigned int(16) lod_num;

for(i = 0; i < lod_num; i++ ){

unsigned int(8) entry_id;

unsigned int(16) lod;

}

}

###### Semantics

lod_num indicates the maximum value of level of detail of a complete set of point cloud.

entry_id indicates the track_id of the entry point of a level, that is, a track with geometry data.

lod indicates the value of level of detail of the track with track_id equal to entry_id.

### Single Track Encapsulation of Scalable G-PCC bitstream

#### General

At MPEG #130, m53461[4] provides a single track encapsulation of scalable G-PCC bitstream to enable partial access of geometry data with a desired level of detail. That is, a scalable G-PCC bitstream is represented in one track in a file using sub-sample structure.

Similarly, the LoD information of attribute data can also be signaled using sub-sample . That is, each level of each component of G-PCC data is signaled in a sub-sample structure. In that case, the decoding process of higher levels of G-PCC data requires data with lower LoD values which could be obtained in one G-PCC sample.

Fig 3 Structures of sub-sample representing single level of detail

As shown in Fig 3, the geometry data unit and attribute data unit in a sample is further divided into data unit header and G-PCC data of multiple levels by using sub-sample structure.

Fig 4 Structures of sub-sample representing single level of detail

As shown in Fig 4, G-PCC geometry data and attribute data is carried in a single track and each sample in this track contains one or more G-PCC components with the complete levels of details. The division of each level is signaled in sub-sample structure.

#### Scalable G-PCC stream track for single-track encapsulation

##### Sample entry

###### Definition

Sample Entry Type:	'gpe1'
Container: 		SampleDescriptionBox
Mandatory:		Yes
Quantity:		One or more sample entries may be present

Under the 'gpe1' sample entry, information of levels of details of G-PCC geometry data and attribute data shall be signaled in sub-sample when the lifting_scalability_enabled_flag is equal to 1.

###### Syntax

aligned(8) class GPCCSampleEntry()

extends VolumetricVisualSampleEntry ('gpe1') {
	GPCCConfigurationBox config; //mandatory

unsigned int(1) lifting_scalability_enabled_flag;

if (lifting_scalability_enabled_flag)

unsigned int(15) lod;

else

bit(15) reserved = 0;
}

###### Semantics

GPCCConfigurationBox specifies the G-PCC decoder configuration information for geometry-based point cloud content. The setupUnit array shall include G-PCC TLV encapsulation structures that are constant for the stream referred to by the sample entry in which the decoder configuration record is present.

lifting_scalability_enabled_flag equal to 1 when attribute data in the sub-sample or the attribute data associated with geometry data in the sub-sample is encoded by the LoD with Lifting Transform with lifting scalability enabled. Otherwise lifting_scalability_enabled_flag equal to 0.

lod indicates the maximum value of level of detail of geometry data and attribute data in the track.

##### Sub-sample

###### Definition

For scalable G-PCC stream in single-track encapsulation, a sub-sample either contains only one G-PCC TLV encapsulation structure as defined in ISO/IEC 23090-9 [GPCC], or contains a part of geometry/attribute data unit(i.e., header of a geometry/attribute data unit or a part of unit data corresponding to a level of detail)

###### Syntax

The codec_specific_parameters field of the SubsampleInformationBox is defined as below:

unsigned int(8) PayloadType;

if (PayloadType == 2) { // geometry payload

bit(8) reserved = 0;

unsigned int(1) lifting_scalability_enabled_flag;

if (lifting_scalability_enabled_flag) {

unsigned int(1) is_geometry_data_unit_header;

if(!is_geometry_data_unit_header)

unsigned int(14) lod;

else

bit(14) reserved = 0;

}

}

else if (PayloadType == 4) { // attribute payload

unsigned int(8) AttrIdx;

unsigned int(1) lifting_scalability_enabled_flag;

if (lifting_scalability_enabled_flag) {

unsigned int(1) is_attribute_data_unit_header;

if(!is_attribute_data_unit_header)

unsigned int(14) lod;

else

bit(14) reserved = 0;

}

}

else

bit(24) reserved = 0;

###### Semantics

PayloadType indicates the tlv_type of the TLV encapsulation structure or a part of that contained in the sub-sample.

NOTE: 	When PayloadType equals to 2 (geometry data unit), the boundary of one TLV structure can be identified as the boundary of a set of continuous sub-samples with the same PayloadType.

lifting_scalability_enabled_flag equal to 1 when attribute data in the sub-sample or the attribute data associated with geometry data in the sub-sample is encoded by the LoD with Lifting Transform with lifting scalability enabled. Otherwise lifting_scalability_enabled_flag equal to 0.

is_geometry_data_unit_header equal to 1 when the sub-sample only contains geometry data unit header. Is_geometry_data_unit_header equal to 0 when the sub-sample contains part of geometry data unit data which belongs to the same LoD level.

is_attribute _data_unit_header equal to 1 when the sub-sample only contains attribute data unit header. Is_attribute_data_unit_header equal to 0 when the sub-sample contains part of attractive data unit data which belongs to the same LoD level.

lod indicates the maximum value of level of detail of the sub-sample, when the TLV encapsulation structure containing geometry payload or attribute payload is decoded from its first sub-sample to this sub-sample.

AttrIdx indicates the ash_attr_sps_attr_idx of the TLV encapsulation structure containing attribute payload in the sub-sample.

# Grouping Tracks for the Same Point Cloud

## Point Cloud Group (m54294)

[NOTE : rename to show the purpose of this track grouping]

### Definition

TrackGroupTypeBox with track_group_type equal to 'gpcg' indicates that this track belongs to a group of tracks that contribute to the  point cloud data.

Tracks contribute to the same point cloud data have the same value of track_group_id for track_group_type 'gpcg', and the track_group_id of tracks from one point cloud data differs from the track_group_id of tracks from any other point cloud data.

### Syntax

aligned(8) class GPCCPointCloudGroupBox 
	extends TrackGroupTypeBox('gpcg') {

}

### Semantics

Tracks that have the same value of track_group_id within TrackGroupTypeBox with track_group_type equal to 'gpcg' belong to the same point cloud data. The track_group_id within TrackGroupTypeBox with track_group_type equal to 'gpcg' is therefore used as the identifier of the point cloud data

# MPEG-DASH Signaling of G-PCC Tiles Information

## G-PCC Tile Inventory Information (m54953)

3D spatial region information enables partial access of the G-PCC bitstream but does not provide finer granularity partial access in advanced streaming applications. A typical G-PCC streaming client may stream and play the content of 3D region(s) present in the user’s viewport in higher quality to efficiently utilize the available network bandwidth rather than streaming the entire point cloud content. When the associated 3D regions are big and the user changes their position and orientation, the player may need to stream multiple large 3D regions with high quality and therefore consume significant network bandwidth.

A fine granular partial access of the G-PCC content based on the tiles bounding box information can reduce the required bandwidth significantly. For example, when a user is moving their position and/or orientation, the 3D tiles near to the center of the user viewport and the tiles closer to the user’s movement direction can be streamed in high quality. The tiles away from the center of user viewport and their movement direction can be streamed with lower quality rather than streaming all the tiles present in the 3D region(s) within the viewport in high quality.

To achieve the finer granularity of partial access in adaptive streaming use cases, 3D tiles bounding box information should be available to the player by signalling them the MPD file. In the case of static 3D spatial regions, we propose a GPCCTileInventory descriptor that signals such information.

### GPCCTileInventory Descriptor

When multiple tiles in a geometry-based point cloud are present and the tiles’ bounding box information is static for the entire stream, the tiles bounding box information shall be signaled in the MPD using a GPCCTileInventory descriptor. This descriptor is a SupplementalProperty element with a @schemeIdUri attribute equal to "urn:mpeg:mpegI:gpcc:2020:gptl". A GPCCTileInventory descriptor shall be present at the adaptation set level of the Main Adaptation Set of the G-PCC content.

The elements and the attributes of the GPCCTileInventory descriptor are shown in Table 2.

Table 2 Elements and attributes for the GPCCTileInventory descriptor

| Elements and Attributes for GPCCTileInventory descriptor | Use | Data type | Description |
| --- | --- | --- | --- |
| gptl | 0..1 | gpcc:tileInventory | Container element whose attributes and elements specify the information of G-PCC tiles. |
| gptl.tile | 1..N | gpcc:tileInfo | An element whose attributes define a tile and provide the information of the G-PCC tiles. |
| gptl.tile@tile_id | M | xs:string | Indicates the unique id of the G-PCC tile. The value of @tile_id shall be identical to that of the tile_id field for one of the tiles in tile_inventory() defined in Error: Reference source not found, or identical to the inferred value of the tile id based on the index of the tile in the tile inventory. |
| gptl.tile@x_offset | M | xs:int | The x offset of the tile bounding box. Same as tile_bounding_box_offset_x in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@y_offset | M | xs:int | The y offset of the tile bounding box. Same as tile_bounding_box_offset_x in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@z_offset | M | xs:int | The z offset of the tile bounding box. Same as tile_bounding_box_offset_y in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@width | M | xs:int | Width of the tile bounding box. Same as tile_bouding_box_size_width in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@height | M | xs:int | Height of the tile bounding box. Same as tile_bouding_box_size_height in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@depth | M | xs:int | Depth of the tile bounding box. Same as tile_bouding_box_size_depth in tile_inventory() defined in ISO/IEC 23090-9 |
| gptl.tile@asId | CM | xs:int | An identifier corresponding to the values of the @id attribute for Tile Component Adaptation Set associated with the tile. This attribute shall be absent in the case of single-track encapsulation of the G-PCC content. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>..<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |

The data types for the various elements and attributes of the GPCCTileInventory descriptor shall be as defined in the following XML schema.

<?xml version="1.0" encoding="UTF-8"?> 
<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" 
	targetNamespace="urn:mpeg:mpegI:gpcc:2020" 
	xmlns:gpcc="urn:mpeg:mpegI:gpcc:2020" 
	elementFormDefault="qualified"> 

	<xs:element name="gptl" type="gpcc:tileInventory" /> 

	<xs:complexType name=" tileInventory"> 
		<xs:element name="tile" type="gpcc:tileinfo" minOccurs="1"/> 
	</xs:complexType> 

	<xs:complexType name="tileInfo"> 
		<xs:attribute name="tile_id" type="xs:string" use="required" /> 
		<xs:attribute name="x_offset" type="xs:int" use="required" /> 
		<xs:attribute name="y_offset" type="xs:int" use="required" /> 
		<xs:attribute name="z_offset" type="xs:int" use="required" /> 
		<xs:attribute name="width" type="xs:int" use="required" /> 
		<xs:attribute name="height" type="xs:int" use="required" /> 
		<xs:attribute name="depth" type="xs:int" use="required" />

<xs:attribute name="asId" type=" xs:int"/> 
	</xs:complexType> 

</xs:schema>

### Dynamic Tile Inventory Information

When multiple tiles in a geometry-based point cloud are present and the tiles’ bounding box information is dynamically changing, the tiles bounding box information along with the parameter sets data are carried in Representations of the Main Adaptation Set.

# On signalling of decoder configuration record for multiple tracks (m57438)

Some syntax elements in decoder configuration record will have the same values for all tracks containing components of one G-PCC bitstream. Syntax elements such as simple_profile_compatibility_flag, dense_profile_compatibility_flag, predictive_profile_compatibility_flag, main_profile_compatibility_flag and level_idc will have the same values in geometry track and its referred attribute tracks. So, signaling of ptl_present_flag is added to specify whether or not syntax element related to profile and operating point information is present for the track.

### Syntax

aligned(8) class GPCCDecoderConfigurationRecord {
	unsigned int(8)	configurationVersion = 1;

unsigned int(1)	ptl_info_present_flag;

if(ptl_info_present_flag ){
		unsigned int(1)	simple_profile_compatibility_flag; 
		unsigned int(1)	dense_profile_compatibility_flag;
		unsigned int(1)	predictive_profile_compatibility_flag;
		unsigned int(1)	main_profile_compatibility_flag;
		unsigned int(18)	reserved_profile_compatibility_18bits;

unsigned int(8)	level_idc;

}

unsigned int(8)	numOfSetupUnits;
	for (i=0; i<numOfSetupUnits; i++) {

tlv_encapsulation	setupUnit; 	//as defined in ISO/IEC 23090-9
	}
	// additional fields
}

### Semantics

ptl_info_present_flag specifies the presence of syntax element related to profile and operating points in the sample entry. ptl_info_present_flag shall be equal to 0 when the track is a track with a sample entry type of ‘gpc1’ or ‘gpcg’ and the value of gpcc_data is equal to 4. ptl_info_present_flag shall be equal to 1 when at least one of the following is true:

- The track is a track with a sample entry type that is not ‘gpc1’ or ‘gpcg’.

- The track is a track with a sample entry type of ‘gpc1’ or ‘gpcg’ and the value of gpcc_data is not equal to 4.

When the track is a track with a sample entry type of ‘gpc1’ or ‘gpcg’ and the value of gpcc_data is equal to 4, the value of the following syntax elements are inferred to be equal to the corresponding syntax elements in the geometry track that refer to this track:

- simple_profile_compatibility_flag

- dense_profile_compatibility_flag;

- predictive_profile_compatibility_flag;

- main_profile_compatibility_flag;

- level_idc

# G-PCC file format miscellaneous (m57474)

## Introduction

Modify the signaling of syntax element that specifies the number of spatial regions such that the number that is signalled is one less than the number of region(s) that is present. To avoid signaling zero regions when the is GPCCSpatialRegionInfoBox or GPCCSpatialRegionInfoProperty present. In other words, change the syntax element num_regions to num_regions_minus1. The number of regions is derived as num_regions_minus1 + 1.

The modifications to GPCCSpatialRegionInfoBox and GPCCSpatialRegionInfoProperty are highlighted in the below section.

## Proposal

### Syntax

aligned(8) class GPCCSpatialRegionInfoBox extends FullBox('gpsr',0,0){

unsigned int(15)	num_regions_minus1;
	for (int i=0; i <= num_regions_minus1; i++) {

GPCCSpatialRegionStruct(1);
		unsigned int(16) num_tiles;
		for(int j=0; j < num_tiles; j++)
			unsigned int(16) tile_id;
	}

}

### Semantics

num_regions_minus1 plus 1 specifies indicate the number of 3D spatial regions

GPCCSpatialRegionStruct provides the 3D spatial region information of the part or all point cloud data indicated by the anchor point and the size of the spatial region along to X, Y, Z axes relative to the anchor point.

num_tiles indicate the number of G-PCC tiles associated with the 3D spatial region

tile_id identifies the G-PCC tile associated with the spatial region indicated by the GPCCSpatialRegionStruct.

### Syntax

aligned(8) class GPCCSpatialRegionInfoProperty 
			extends ItemFullProperty('gpsr', 0, 0)
{
	unsigned int(16)	num_regions_minus1;
	for(int i=0; i<= num_regions_minus1; i++){
			GPCCSpatialRegionStruct(1); 
			unsigned int(8) num_tiles;
			for(int j=0; j < num_tiles; j++){
				unsigned int(16) tile_id;
			}
	}
}

### Semantics

num_regions_minus1 plus 1 specifies indicate the number of 3D spatial regions.

# Signaling Alternatives for G-PCC Content (m57825)

## Introduction

Alternatives for the same GPCC content and alternatives for attributes referenced by the same geometry track are both supported by 23090-18. However, explicitly signaling of differences between alternative representations is missing. In such case, the client has to parse all the parameters in GPCCDecoder-ConfigurationRecord to decide which alternative track should be selected. Since the different parameters between alternatives are usually multiple, it’s hard for client to judge which alternative is suitable.

## Proposal

### GPCC alternatives quality box

#### Definition

Box Type:	'gpaq'
Container:	GPCCSampleEntry ('gpe1', 'gpeg', 'gpc1', 'gpcg', 'gpeb', 				'gpcb') or GPCCTileSampleEntry
Mandatory:	No
Quantity:	Zero or One

GPCCAlternativesQualityBox is used to indicate the quality difference between tracks within the same alternative group. When this box is presented, the quality level is ranked by the content provider.

#### Syntax

aligned(8) class GPCCAlternativesQualityBox extends FullBox('gpaq',0,0) {
	unsigned int(8) quality_ranking;

}

#### Semantics

quality_ranking indicates quality ranking information of the corresponding track. The lower value of this field suggests a higher quality.

# Signaling Attribute Instances of the Same Attribute Type (m57897)

## Introduction

In the current 23090-18 specification, GPCC attributes have been defined and multiple attributes of the same geometry component have been supported. But the support for multiple instances of the same attribute type is not clear.

The first kind of use case can support more customized applications like it’s mentioned in 23090-9. Different color attributes can be selected based on viewport or user interactions.

Another kind of use case can support combine frame coding which is a supported functionality in 23090-9. “The proposed combine frame coding addresses both issues by introducing the encoding of frame index in the combined point cloud. Moreover, it improves coding efficiency largely so that it could be also beneficial for storage usage of frame-based point cloud content.”

When combine frame coding is used, attribute type of frame index should be used to help re-order the GPCC frames. If certain points exist in both frame1 and frame2, then they shall have frame index 1 and 2 as their attribute. Since frame index is a one-dimensional signal, two attribute instances of frame index shall be included in the GPCC bitstream.

The related changes to clause 7.2.3 in G-PCC carriage specification are highlighted in the following section.

## Proposal

### GPCC component information box

      - Definition

This box indicates the type of G-PCC components, e.g., geometry, attribute. When this box is present in a sample entry of tracks carrying G-PCC component data, it indicates the type of G-PCC components carried by the respective track. This box also provides the attribute name, index and an optional attribute instance type and attribute instance priority of G-PCC attribute component carried by the respective G-PCC attribute track.

When the G-PCC bitstream is stored in a single track, it shall not be present in the sample entry.

      - Syntax

aligned(8) class GPCCComponentInfoBox

extends FullBox('ginf', version = 0, 0) {

unsigned int(8)	gpcc_type;

if(gpcc_type == 4) {

unsigned int(8)	attr_index;

unsigned int(1) 	attr_type_present_flag;

unsigned int(1) 	multi_attr_instance_flag;

bit(3) 				reserved = 0;

if (attr_type_present_flag || multi_attr_instance_flag)

{

unsigned int(3)	attr_type;

} else {

bit(3) 		reserved = 0;

}

if(multi_attr_instance_flag)

{

unsigned int(4)	attr_instance_priority

unsigned int(4)	attr_instance_type

}

utf8string			attr_name;

}

}

      - Semantics

attr_index identifies the order of the attribute signalled in the SPS.

attr_type_present_flag indicates the presence of attribute type information in the GPCCComponentInfoBox. Value 1 indicates the attribute type information is signalled in this box. Value 0 indicates the attribute type information is not signalled.

multi_attr_instance_flag indicates whether there are multiple instances of respective attribute type. Value 1 indicates there are multiple instances of respective attribute type. Value 0 indicates there is only one instance of respective attribute type.

attr_type identifies the type of attribute component as specified in Error: Reference source not found8 of ISO/IEC 23090-9 Error: Reference source not found.

attr_instance_priority indicates the priority of the respective attribute instance. The lower value of this field suggests a higher priority. The attribute instance with lower priority can be dropped during transport or decoding procedure as long as the one with higher priority is guaranteed.

attr_instance_type indicates the use case type when the respective attribute instance is required as specified in Table x

Table x Attribute Instance Type

| attr_instance_type value | Description |
| --- | --- |
| 0 | Associated to viewports |
| 1 | Selected by user interaction |
| 2 | Related to codec process |
| 3..15 | Reserved. |

NOTE: More detailed description about attr_instance_type is helpful to understand the use-case.

attr_name specifies a human-readable name for the type of G-PCC attribute components.

# Point reliability (m59523 and m62013)

## Definition of confidence

It is proposed to leverage the concept of confidence defined in the existing frameworks such as Intel® RealSense™ and Apple ARKit.

In Intel® RealSense™ [1], the sensor is able to deliver a confidence map of the corresponding depth map measurement.

Figure 1 - Confidence map in Intel® RealSense™

In Apple ARKit, the same concept exists and called the confidence map [2]s.

confidenceMap

The framework’s confidence in the accuracy of the depth-map data.

The natural light of the physical environment affects the depthMap property such that ARKit is less confident about the accuracy of the LiDAR Scanner’s depth measurements for surfaces that are highly reflective, or that have high light absorption. This property measures the accuracy of the scene depth-data by containing an ARConfidenceLevel raw-value for every component in depthMap.

ARConfidenceLevel

Degrees to which the framework is confident about depth-data accuracy.

ARConfidenceLevelLow

Depth-value accuracy in which the framework is less confident.

ARConfidenceLevelMedium

Depth-value accuracy in which the framework is moderately confident.

ARConfidenceLevelHigh

Depth-value accuracy in which the framework is fairly confident.

## Example use cases

### Home appliances

The recent years have seen the advent of smart home appliances and especially robot vacuum cleaner. A typically way for robot vacuum cleaner to navigate is to scan the environment via a spinning head on top of the device, hence creating a point cloud representative of the room.

Figure 2 - Point cloud scanning by robot vacuum cleaner\

Based on the collected data, those appliances are for instance able to detect wall and create maps.

In case a detected point is a false positive, that is there is no real point in front of the device, or that the absence of a probed point a false negative, that is there is actually an obstacle on the scanned direction, this may lead to the possible issues:

- Non-detected wall

- Fall of the devices

- Wrong location determination

- Hard collision with pieces of furniture, walls, etc.

### Smart vehicles

Another domain where point cloud data is heavily used is for self-driving or assisted driving as shown below.

Figure 3 - Example of point cloud frame captured by a vehicle

For this result, typical settings combined multiple sensors around the vehicle in order to generate a view all around the vehicle as shown below.

Figure 4 - Multi-sensor setting example on vehicles

However, in those settings, a sensor failure may lead to an entire section of the combined point cloud to exhibit no points while there may be an obstacle. On the other end, a different sensor failure may detect points while there is no obstacle at those distances. As a result, analysis of point cloud data with false positive or negative points may lead to:

- unnecessary maneuver such as emergency break

- failure to avoid collision if obstacles are not detected.

## Time persistence of the information

The proposed signalling is part of the GPCCSpatialRegionInfoBox, which is either part of the sample entry or stored as sample of timed metadata track defined by DynamicGPCCSpatialRegionSample, clause 9.1.3 Signalling of Dynamic Spatial Region Information in ISOBMFF.

## Proposal

## 9.1.1.4 G-PCC Spatial Region information structure

GPCCSpatialRegionStruct provides the 3D spatial region information including the anchor point, and the size of the 3D spatial region in the cartesian coordinate along to X, Y, Z axes relative to the anchor point.

## 9.1.1.4.1 Syntax

aligned(8) class GPCCSpatialRegionStruct(dimension_included) {
	unsigned int(32) size;

unsigned int(16) region_id;

unsigned int(1) bounding_box_present_flag;

unsigned int(1) dimensions_included_flag;

unsigned int(1) tm_present_flag;

unsigned int(1) confidence_score_present_flag;

unsigned int(4) reserved;

if(bounding_box_present_flag)

GPCCBoundingBox bounding_box(dimensions_included_flag);

if(tm_present_flag) {

TileMappingInfo tile_map();

}
	if(confidence_score_present_flag) {

unsigned int(8) confidence_score;

}

}

## 9.1.1.4.1 Semantics

size is an integer that specifies the number of bytes in this element, including all its fields and contained elements.

region_id is an identifier for the spatial region.

bounding_box_present_flag indicates the presence of bounding box information. For sync samples in dynamic spatial region metadata track this flag shall be set to 1. For non-sync samples in dynamic spatial region metadata track, this flag shall be set to 1 when the position and/or dimensions of this 3D region are updated with reference to the previous sync sample.

dimensions_included_flag indicates the presence of the bounding box with the scale field. For sync samples in dynamic spatial region metadata sample, this flag shall be set to 1. For non-sync samples in dynamic spatial region metadata sample, this flag shall only be set to 1 when the dimensions of this 3D region are updated with reference to the previous sync sample. This flag may be set to 1 only if bounding_box_present_flag is set to 1.

tm_present_flag indicates the presence of tile mapping information. For sync samples in dynamic spatial region metadata track this flag shall be set to 1 when tile mapping information is available. For non-sync samples in dynamic spatial region metadata track, this flag shall only be set to 1 when the associated 3D tiles of this 3D region are updated with reference to the previous sync sample.

confidence_score_present_flag indicates the presence of the confidence score for the spatial region.

confidence_score indicates the degree of confidence in unit of 1/255 in the range of 0 to 1 for the data in the spatial region. A signaled value of 0 expresses a score of 0 which means the confidence in the data in the spatial region is the lowest. A signaled value of 255 expresses a score of 1 which means the confidence in the data in the spatial region is the highest. An application should take into account the confidence score when processing the data and deriving actions, results, interpretations, etc… When not present, the confidence score is unknown.

# Derived items for PCC (m62341)

## Introduction

This contribution proposes derived items for storing point cloud data as:

- a 3D grid item.

- a 3D fusion item.

Several devices store 2D images using HEIF grids for enhancing their capacities, in particular the encoding speed. As the number of devices able to generate 3D point clouds is increasing, there is a strong interest for providing efficient means of storing 3D point clouds using an ISOBMFF-based format. This includes the ability to store a point cloud as a 3D grid or as a more freeform 3D fusion.

## Proposal

### Grid of Point Clouds

#### Definition

An item with item_type value of 'grd3' defines a derived point cloud item whose reconstructed point cloud is formed from one or more input point clouds in a given 3D grid order.

The input point clouds are inserted in an order depending on the value of morton_order, in the order of SingleItemTypeReferenceBox of type 'dimg' for this derived point cloud item within the ItemReferenceBox. In the SingleItemTypeReferenceBox of type ‘dimg’, the value of from_item_ID identifies the derived point cloud item of type 'grd3', the value of reference_count shall be equal to width*depth*height, and the values of to_item_ID identify the input point clouds.

NOTE:	When removing an item that is marked as an input point cloud of a point cloud grid item, the content of the point cloud grid item might need to be rewritten.

If the value of morton_order is 0, then the input point clouds are inserted in a line-scan order, with increasing X first, followed by increasing Y, and followed by increasing Z. If the value of morton_order is 1, then the input point clouds are inserted in a Morton order.

If the value of same_origin is 0, each input point cloud shall be translated to insert it into the grid. The translation vector is defined as follows:

Where (, , ) is the size of a grid’s cell, and (, , ) are the 0-based indices of the position of the input point cloud in the grid. In addition, after translation, all the points of an input point cloud shall be inside the boundaries of the grid’s cell where the input point cloud is located.

If the value of same_origin is 1, then all the input point clouds shall share the same coordinate system and they shall be inserted without any translation into the grid. In addition, all the points of an input point cloud shall be inside the boundaries of the grid’s cell where the input point cloud is located. When the value of same_origin is 1, no transformation is required for inserting an input point cloud item in the grid.

If the value of has_cell_size is 0, then all the bounding boxes of the input point cloud shall have the same size, which is the cell’s size.

If the value of has_cell_size is 1, then the cell’s size is specified by the cell_size field.

#### Syntax

aligned(8) class GPCCGrid {

unsigned int(8) version = 0;

unsigned int(8) flags;

unsigned int(8) width_minus_one;

unsigned int(8) depth_minus_one;

unsigned int(8) height_minus_one;

unsigned int(1) same_origin;

unsigned int(1) morton_order;

unsigned int(1) has_cell_size;

unsigned int(5) reserved;

if (has_cell_size == 1) {

unsigned int(8) precision;

Vector3         cell_size(precision);

}

}

#### Semantics

version shall be equal to 0.

flags shall be equal to 0.

width_minus_one, height_minus_one and depth_minus_one specifies the number of cells in the grid respectively along the X, Y and Z axis.

same_origin equal to 1 specifies that the input point clouds shall not be translated to insert them into the grid. same_origin equal to 0 specifies that each input point cloud shall be translated to insert it into the grid.

morton_order equal to 0 indicates that the input point clouds are ordered in a line-scan order. morton_order equal to 1 indicates that the input point clouds are ordered in a Morton order.

has_cell_size equal to 0 indicates that the size of a grid’s cell is not specified. has_cell_size equal to 1 indicates that the size of a grid’s cell is specified.

precision specifies the precision used for encoding the size of a cell.

cell_size is the size of a grid’s cell.

### Point Cloud Fusion

#### Definition

An item with item_type value of 'fus3' defines a derived point cloud item whose reconstructed point cloud is formed by combining one or more input point clouds.

The input point clouds are inserted in the order of SingleItemTypeReferenceBox of type 'dimg' for this derived point cloud item within the ItemReferenceBox. In the SingleItemTypeReferenceBox of type ‘dimg’, the value of from_item_ID identifies the derived point cloud item of type 'fus3' and the values of to_item_ID identify the input point clouds.

NOTE:	When removing an item that is marked as an input point cloud of a point cloud fusion item, the content of the point cloud fusion item might need to be rewritten.

If the value of same_origin is 0, each input point cloud shall be translated before combining it. This translation is defined by the anchor field corresponding to the input point cloud, i.e. the ith anchor applies to the ith occurrence in the order of SingleItemTypeReferenceBox of type 'dimg' for this derived point cloud item within the ItemReferenceBox.

If the value of same_origin is 1, all the input point clouds use the same coordinate system, and no translation shall be applied before combining them.

If the value of overlap is 0, then the intersection of the bounding boxes of any pair of input point cloud shall have an empty volume.

If overlap is equal to 1, the intersection of the bounding boxes of any pair of input point cloud may or may not have an empty volume.

#### Syntax

aligned(8) class GPCCFusion {

unsigned int(8) version = 0;

unsigned int(8) flags;

unsigned int(1) overlap;

unsigned int(1) same_origin;

unsigned int(6) reserved;

if (same_origin == 0) {

unsigned int(8) precision;

for (int i=0; i < reference_count; i++) {

Vector3       anchor(precision);

}

}

#### Semantics

version shall be equal to 0.

flags shall be equal to 0.

overlap equal to 1 indicates that the intersection of the bounding boxes of any pair of input point cloud may or may not have an empty volume. overlap equal to 0 indicates that the intersection of the bounding boxes of any pair of input point cloud shall have an empty volume.

same_origin equal to 1 specifies that the input point clouds shall not be translated before combining it. same_origin equal to 0 specifies that each input point cloud shall be translated before combining it.

reference_count is obtained from the SingleItemTypeReferenceBox of type ‘dimg’ where this item is identified by the from_item_ID field.

anchor specifies the translation to apply to an input point cloud before combining it.

# References

- Intel® RealSenseTM LiDAR Camera L515, Intel® RealSense™ LiDAR Camera L515 user guide, Revision 1.0, July 2020, source: https://support.intelrealsense.com/hc/en-us/article_attachments/360081846174/L515_User_Guide_v1.0.pdf

- confidenceMap | Apple Developer Documentation, source: https://developer.apple.com/documentation/arkit/ardepthdata/3566295-confidencemap?language=objc