# ISO/IEC JTC 1/SC 29/WG 03 N00861

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Input Contribution

Title:	Potential improvements of ISO/IEC 23090-14 CDAM 2: Support for Haptics, Augmented Reality, Avatars, Interactivity, MPEG-I Audio, and Lighting

Date of document:	2023-06-2626

Source:	WG03

No. of pages:	50 (with cover page)

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

In clause 2 on normative references, add the following references

Khronos, EXT_lights_image_based, https://github.com/KhronosGroup/glTF/tree/main/extensions/2.0/Vendor/EXT_lights_image_based

Update Figure 3 with the attached figure.

Add clause 8 with the following content

- Advanced Features

## 8.1 	AR Anchoring

### 8.1.1	MPEG_scene_anchor and MPEG_node_anchor  extensions

AR anchoring is supported at the scene level and at the root node level through the definition of two extensions MPEG_scene_anchor and MPEG_node_anchor.

When present, the MPEG_scene_anchor extension shall be included as extension to the scene object.

When present, the MPEG_node_anchor extension shall be included as extension to one or several root nodes. Multiple AR anchoring is then supported by adding the MPEG_node_anchor extension to several root nodes.

### 8.1.2	Semantics

The MPEG_scene_anchor and MPEG_node_anchor extensions require the Trackable and Anchor objects provided in Table 8.1-1. The definition of these objects shall be provided within an extension at the glTF file level.

Table 8.1-1 – Common objects of the MPEG_scene_anchor and MPEG_node_anchor extensions provided at the glTF file level

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| trackables | array(Trackable) | N/A | M | provides a list of trackables used by the anchor objects. |
| anchors | array(Anchor) | N/A | M | Provides a list of anchors for a scene or for root nodes to enable AR anchoring |

The syntax and semantics of the Anchor object is provided in Table 8.1-2.

Table 8.1-2 – Definition of the Anchor object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| trackable | integer | M |  | Index of the trackable in the trackables array that will be used for this anchor. |
| requiresAnchoring | boolean | M |  | If TRUE, the application shall skip the virtual assets attached to this anchor until the pose of this anchor in the real world is known. if FALSE, the application shall process the virtual assets attached to this anchor |
| minimumRequiredSpace (x, y, z) | vec3 | O | (0,0,0) | Space required to anchor the AR asset (x, y, z in meters). This space corresponds to an axis-aligned bounding box expressed in the trackable local space. This value shall be compared to the bounding box of the real available space determined by the application. |
| aligned | enumeration | O | NOT_USED | the aligned flag may take one of the following values: NOT_USED=0, ALIGNED_NOTSCALED=1, ALIGNED_SCALED=2. If ALIGNED_SCALED is set, the bounding box of the virtual assets attached to that anchor is aligned and scaled to match the bounding box of the real available space determined by the application. |
| actions | array(Action) | O |  | Indices of the actions in the actions array of the interactivity extension to be executed once the pose of this anchor is determined. An example is a setTransform action to place the virtual assets attached to that anchor. |
| light | integer | N/A | O | Reference to an item in the lights array of the MPEG_lights_texture_based extension. |

The definition of the Trackable object is provided in the Table 8.1-3.

A trackable is a model of an element of the real world of which features are available and/or may be extracted. Each trackable provides a local reference space in which an anchor pose can be expressed.

A MPEG anchor corresponds to a real-world pose, identified using one trackable. Each MPEG anchor provides a local reference space in which a pose can be expressed. This local reference space is derived from the trackable local reference space using a pre-defined 3D transform.

The semantics of a trackable are provided in table 8.1-3.

Table 8.1-3 – Definition of the Trackable object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M | TRACKABLE_PLANE | The type of the trackable as defined in table 8.1-4 |
| if (type == TRACKABLE_FLOOR) { |  |  |  | This trackable corresponds to the OpenXR stage reference space. It is a runtime defined flat, rectangular space that is empty and can be walked around on. Editor’s note: the definition of the TRACKABLE_FLOOR needs to be done in the standard text and will be using a reference to OpenXR. This does not require an OpenXR conforming runtime, but if another runtime is used, the implementation needs to map the signaling to the runtime application. |
| } |  |  |  |  |
| if (type == TRACKABLE_VIEWER) { |  |  |  | This trackable corresponds to the OpenXR View reference space. It tracks the view origin used to generate view transforms for the primary viewer. Editor’s note: the definition of the TRACKABLE_VIEWER needs to be done in the standard text and will be using a reference to OpenXR. This does not require an OpenXR conforming runtime, but if another runtime is used, the implementation needs to map the signaling to the runtime application. |
| } |  |  |  |  |
| if (type == TRACKABLE_CONTROLLER) { |  |  |  | Specifies a trackable that is mapped to a controller. This corresponds to one of the OpenXR action spaces. Editor’s note: the definition of the TRACKABLE_CONTROLLER needs to be done in the standard text and will be using a reference to OpenXR. This does not require an OpenXR conforming runtime, but if another runtime is used, the implementation needs to map the signaling to the runtime application. |
| path | string | M |  | a path that describes the action space as specified by OpenXR. An example is “/user/hand/left/input” |
| } |  |  |  |  |
| if (type== TRACKABLE_GEOMETRIC) { |  |  |  |  |
| geometricConstraint | enumeration | M | HORIZONTAL_PLANE | the geometricConstraint flag may take one of the following values: HORIZONTAL_PLANE=0, VERTICAL_PLANE=1 |
| } |  |  |  |  |
| if (type== TRACKABLE_MARKER_2D OR MARKER_3D) { |  |  |  |  |
| markerNode | number | M |  | Index to the node in the nodes array in which the marker geometry and texture are described. |
| } |  |  |  |  |
| if (type== TRACKABLE_MARKER_GEO) { |  |  |  |  |
| geoCoordinate | number | M | N/A | index to geo coordinate item in array in MPEG_GEO_COORDINATES extension. Editor’s note: to check the availability of the MPEG_GEO_COORDINATES extension |
| } |  |  |  |  |
| if (type== TRACKABLE_APPLICATION) { |  |  |  |  |
| trackable_id | string | M |  | An application-defined trackable id, that is known to the application. |
| } |  |  |  |  |

Table 8.1-4 – Definition of the Trackable type

| Trackable type | Description |
| --- | --- |
| TRACKABLE_FLOOR = 0 | trackable mapped to the viewer’s floor |
| TRACKABLE_VIEWER = 1 | trackable mapped to the viewer’s origin in the forward direction for the viewer |
| TRACKABLE_CONTROLLER = 2 | trackable mapped to a controller |
| TRACKABLE_PLANE = 3 | trackable mapped to a plane |
| TRACKABLE_MARKER_2D = 4 | trackable mapped to a 2D marker |
| TRACKABLE_MARKER_3D = 5 | trackable mapped to a 3D marker |
| TRACKABLE_MARKER_GEO = 6 | trackable mapped to geospatial coordinates |
| TRACKABLE_APPLICATION = 7 | Trackable managed by the application |

Table 8.1-5– MPEG_anchor_ object instantiation at the scene level

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| anchor | integer | 0 | M | Reference to an item in the anchors array of the MPEG_anchor extension. |

Table 8.1-6 – MPEG_anchor_based object instantiation at the node level

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| anchor | integer | 0 | M | Reference to an item in the anchors array of the MPEG_anchor extension. |

### 8.1.3	Processing model

Each trackable provides a local reference space in which an anchor pose can be expressed. This local reference space is right-handed and depends on the type of trackable as follows:

- TRACKABLE_FLOOR

The width and length of a plane span the xz-plane of a floor anchor instance's local coordinate system. The origin of the local coordinate system is located at the center of the detected flat rectangular surface. The y-axis of the plane anchor is the plane’s normal vector and the X and Z axes are aligned with the rectangle edges.

- TRACKABLE_VIEWER

The VIEW space tracks the view origin used to generate view transforms for the primary viewer (or centroid of view origins if stereo), with +Y up, +X to the right, and -Z forward. This space points in the forward direction for the viewer without incorporating the user’s eye orientation, and is not gravity-aligned. It’s a head locked space.

- TRACKABLE_CONTROLLER

The grip, aim and palm controller types are defined.

A grip pose allows applications to reliably render a virtual object held in the user’s hand, whether it is tracked directly or by a motion controller. The grip pose is defined as follows:

- The grip position:

  - For tracked hands: The user’s palm centroid when closing the fist, at the surface of the palm.

  - For handheld motion controllers: A fixed position within the controller that generally lines up with the palm centroid when held by a hand in a neutral position. This position should be adjusted left or right to center the position within the controller’s grip.

- The grip orientation

  - +X axis: When you completely open your hand to form a flat 5-finger pose, the ray that is normal to the user’s palm (away from the palm in the left hand, into the palm in the right hand).

  - -Z axis: When you close your hand partially (as if holding the controller), the ray that goes through the center of the tube formed by your non-thumb fingers, in the direction of little finger to thumb.

  - +Y axis: orthogonal to +Z and +X using the right-hand rule.

An aim pose allows applications to point in the world using the input source, according to the platform’s conventions for aiming with that kind of source. The aim pose is defined as follows:

- For tracked hands: The ray that follows platform conventions for how the user aims at objects in the world with their entire hand, with +Y up, +X to the right, and -Z forward. The ray chosen will be runtime-dependent, for example, a ray emerging from the palm parallel to the forearm.

- For handheld motion controllers: The ray that follows platform conventions for how the user targets objects in the world with the motion controller, with +Y up, +X to the right, and -Z forward. This is usually for applications that are rendering a model matching the physical controller, as an application rendering a virtual object in the user’s hand likely prefers to point based on the geometry of that virtual object. The ray chosen will be runtime-dependent, although this will often emerge from the frontmost tip of a motion controller.

A palm pose allows applications to reliably anchor visual content relative to the user’s physical hand, whether the user’s hand is tracked directly or its position and orientation is inferred by a physical controller. The palm pose is defined as follows:

- The palm position: The user’s physical palm centroid, at the surface of the palm.

- The palm orientation:

  - +X axis: When a user is holding the controller and straightens their index finger, the ray that is normal to the user’s palm (away from the palm in the left hand, into the palm in the right hand)

  - -Z axis: When a user is holding the controller and straightens their index finger, the ray that is parallel to their finger’s pointing direction.

  - +Y axis: orthogonal to +Z and +X using the right-hand rule.

- TRACKABLE_PLANE

The width and length of a plane span the xz-plane of the anchor instance's local coordinate system. The origin of the local coordinate system is located at the center of the detected flat rectangular surface. The y-axis of the plane anchor is the plane’s normal vector, and the X and Z axes are aligned with the rectangle edges.

- TRACKABLE_MARKER_2D

The width and length of the marker 2D span the xz-plane of the anchor instance's local coordinate system. The origin of the local coordinate system is located at the center of the detected marker 2D surface. The y-axis of the anchor is the plane’s normal vector, and the X and Z axes are aligned with the rectangle edges.

- TRACKABLE_MARKER_3D

For 3D models, the origin is the center of the mesh. The X, Y, and Z axes correspond to the axes of the world space.

- TRACKABLE_MARKER_GEO

The y-axis matches the direction of gravity as detected by the device's motion sensing hardware, y points downward.

The x- and z-axes match the longitude and latitude directions. -Z points to true north -X points west.

- TRACKABLE_APPLICATIO

The application-defined trackable object must have a right-handed coordinate space.

Either scene_anchor extension or node_anchor extension(s) may exclusively be present in the MPEG-I Scene Description. Upon activation of the XR mode, the Presentation Engine identifies the anchor points associated with the scene or with the root nodes and the related trackables.

If the requiresAnchoring Boolean parameter of an anchor is set to TRUE, the application shall skip the virtual assets attached to this anchor until the pose of this anchor in the real world is known. Otherwise, the application shall process the virtual assets attached to this anchor.

Upon the detection of a trackable within the user’s real environment, the application creates an XR space and possibly determines the bounding box of the real available space.

If the optional minimumRequiredSpace parameter of the anchor referencing the trackable is present, the application shall skip the virtual assets attached to that anchor if the real available space is less than the minimumRequiredSpace.

Otherwise, the application shall start the tracking, shall launch the actions provided in the actions parameter of the anchor and shall apply the necessary spatial transformations to the virtual assets with respect to the local space of the anchor prior to rendering.

The application could update at runtime a tracking status of each trackable defined in the trackables array at the scene level, for instance as follows:

- A tracking status set to TRUE when the trackable is detected and when its pose is well identified.

- A tracking status set to FALSE when the trackable position is lost (e.g., no more detected).

To provide the flexibility to define an anchor space different than the trackable’s space,

- The TRS of the node having the MPEG_node_anchor extension defines the relative transformation between the trackable’s space and the anchor space in which the child nodes containing the virtual assets are placed.

- A setTransform action may be defined to affect the scene root nodes in the case of a MPEG_scene_anchor extension. This transformation defines the TRS between the trackable’s space and the anchor space in which all the scene root nodes are placed.

Actions are defined in MPEG_scene_interactivity extension.

If the array of action is not empty, the actions are executed once the pose of anchor is determined. If the tracking status is set to FALSE after being TRUE, actions are not canceled.

## 8.2 	Interactivity

### 8.2.1	General

The proposed compromise approach supports XR interactivity at the scene level and at the node level through the definition of two extensions MPEG_scene_interactivity and MPEG_node_interactivity.

The MPEG_scene_interactivity extension shall be included as extension to the scene object.

The MPEG_node_interactivity extension shall be included as extension to node object.

This extension is used to complement the interactivity defined at the scene level.

### Semantics

#### 8.2.2.1	Semantics at scene level

The semantic of the MPEG_scene_interactivity glTF extension is based on the definition of trigger, action and behavior objects as shown in Table 8.2-1.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M | [] | Contains the definition of all the triggers used in that scene |
| actions | array | M | [] | Contains the definition of all the actions used in that scene |
| behaviors | array | M | [] | Contains the definition of all the behaviors used in that scene. A behavior is composed of a pair of (triggers, actions), control parameters of triggers and actions, a priority weight and an optional interrupt action |

Table 8.2-1: Semantic of the MPEG_scene_interactivity extension

The semantic of a trigger is provided in Table 8.2-2.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M | "TRIGGER_COLLISION" | One element of Table 8.2-3 that defines the type of the trigger. |
| if (type == TRIGGER_COLLISION){ |  |  |  |  |
| Nodes | array | M |  | Indices of the nodes in the nodes array to be considered for collision determination. Any detection of collision shall activate the trigger |
| primitives | array(Primitive) | O | [] | List of primitives used to activate the proximity or collision trigger. Semantics are presented in table 8.2-5. |
| } |  |  |  |  |
| if (type == TRIGGER_PROXIMITY){ |  |  |  |  |
| referenceNode | number | M | Default Camera | Index in the nodes array, of the node to consider for the proximity evaluation. |
| distanceLowerLimit | number | M | 0 | Threshold min in meters for the node proximity calculation |
| distanceUpperLimit | number | O |  | Threshold max in meters for the node proximity calculation |
| nodes | array | M | [] | Indices of the nodes in the nodes array to be considered. All the nodes shall have a distance from the user camera above the distanceLowerLimit and below the distanceUpperLimit to activate the trigger |
| primitives | array(Primitive) | O | [] | List of primitives used to activate the proximity or collision trigger. Semantics are presented in table 8.2-5. |
| } |  |  |  |  |
| if (type== TRIGGER_USER_INPUT){ |  |  |  |  |
| userInputDescription | string | M |  | Describe the user body part and gesture related to the input. E.g. “/user/hand/left/grip” |
| Nodes | array | O |  | Indices of the nodes in the nodes array to be considered for this user input |
| } |  |  |  |  |
| if (type== TRIGGER_VISIBILITY){ |  |  |  |  |
| cameraNode | number | M |  | Index to the node containing a camera in the nodes array for which the visibilities are determined |
| nodes | array | M |  | Indices of the nodes in the nodes array to be considered. All the nodes shall be visible by the camera to activate the trigger |
| } |  |  |  |  |

Table 8.2-3: Semantic of a trigger

| Trigger type | Description |
| --- | --- |
| TRIGGER_COLLISION=0 | Collision Trigger |
| TRIGGER_PROXIMITY | Proximity Trigger |
| TRIGGER_USER_INPUT | User_Input Trigger |
| TRIGGER_VISIBILITY | Visibility Trigger |

Table 8.2-4: type of trigger

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | O | BV_SPHEROID | Describes the type of primitive used to activate the proximity trigger. The available options are. BV_CUBOID, BV_PLANE_REGION, BV_CYLINDER_REGION, BV_CAPSULE_REGION, and the default BV_SPHEROID. Semantics are presented in table 8.2-.6 |
| boundary | number | O | 0.0 | Defines the region of intersection within the primitive. if zero then all area of the primitive activates the trigger. |

Table 8.2-5: Semantic description of “MPEG_scene_interactive.trigger.primitive” properties.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| if (type == BV_CUBOID) { |  |  |  |  |
| width | number | M | 1.0 | Width of the box. |
| height | number | M | 1.0 | Height of the box. |
| depth | number | M | 1.0 | Depth of the box. |
| centroid | vec3 | M | (0.0, 0.0, 0.0) | Centroid 3D coordinate (x,y,z) of the cube |
| } |  |  |  |  |
| if (type == BV_PLANE_REGION) { |  |  |  |  |
| width | number | M | 1.0 | Width of the plane. |
| height | number | M | 1.0 | Height of the plane. |
| centroid | vec2 | M | (0.0, 0.0) | Centroid 2D coordinate (x,y) or (x,z) or (y,z) of the plane. |
| } |  |  |  |  |
| if (type == BV_CYLINDER_REGION) { |  |  |  |  |
| radius | number | M | 1.0 | Radius of the cylinder. |
| length | number | M | 1.0 | Length of the cylinder. |
| centroid | vec3 | M | (0.0, 0.0, 0.0) | Centroid 3D coordinate (x,y,z) of the cylinder |
| } |  |  |  |  |
| if (type == BV_CAPSULE_REGION) { |  |  |  |  |
| radius | number | M | 1.0 | Radius of the capsule. |
| base | vec3 | M | (0.0, 0.0, 0.0) | Centroid 3D coordinate (x,y,z) of the base semi-sphere of the capsule. |
| top | vec3 | M | (0.0, 0.0, 0.0) | Centroid 3D coordinate (x,y,z) of the top semi-sphere of the capsule. |
| } |  |  |  |  |
| if (type == BV_SPHEROID) { |  |  |  |  |
| radius | number | M | 1.0 | Radius of the sphere. |
| centroid | vec3 | M | (0.0, 0.0, 0.0) | Centre 3D coordinate (x,y,z) of the sphere. |
| } |  |  |  |  |

Table 8.2-6: Semantical description of each primitive region.

The semantic of an action is provided in Table 8.2-7.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M | ACTION_ACTIVATE | One element of Table 8.2-5 that defines the type of the action. |
| delay | number | O |  | Duration of delay in second before executing the action |
| if (type== ACTION_ACTIVATE){ |  |  |  |  |
| activationStatus | enum | M | ENABLED | ENABLED=0: the node shall be processed by the application DISABLED =1: the node shall skipped by the application |
| Nodes | array | M | [] | Indices of the nodes in the nodes array to set the activation status |
| } |  |  |  |  |
| if (type== ACTION_TRANSFORM){ |  |  |  |  |
| transform |  | M |  | 4x4 transformation matrix to apply to the nodes |
| nodes | array | M |  | Indices of the nodes in the nodes array to be transformed |
| } |  |  |  |  |
| if (type== ACTION_BLOCK){ |  |  |  |  |
| nodes | array | M |  | Indices of the nodes in the nodes array to lock their related transforms. |
| } |  |  |  |  |
| if (type == ACTION_ANIMATION){ |  |  |  |  |
| animation | number | M |  | index of the animation in the animations array to be considered |
| animationControl | string | M | "ANIMATION_PLAY" | One element of Table 8.2-6 that defines the control of the animation. |
| } |  |  |  |  |
| if (type == ACTION_MEDIA){ |  |  |  |  |
| media | number | M |  | Index of the media in the MPEG media array to be considered |
| mediaControl | enumeration | M | MEDIA_PLAY | One element of Table 8.2-7 that defines the control of the media. |
| } |  |  |  |  |
| if (type == ACTION_MANIPULATE){ |  |  |  |  |
| manipulate_action_type | string | M | ACTION_MANIPULATE_FREE | One element of Table 8.2-8 that defines the action manipulate type. |
| axis | array | O | Up | (x,y,z) coordinates of the axis used for rotation and sliding. These coordinates are relative to the local space created by the USER_INPUT trigger activation. E.g. a “/user/hand/left/pose” user input trigger creates a local space attached to the user left hand |
| nodes | array | M |  | Indices of the nodes in the nodes array to be manipulated. |
| } |  |  |  |  |
| if (type == ACTION_SET_MATERIAL){ |  |  |  |  |
| material | number | M |  | Index of the material in the materials array to apply to the nodes |
| nodes | array | M |  | Indices of the nodes in the nodes array to set their material |
| } |  |  |  |  |
| if (type == ACTION_SET_HAPTIC){ |  |  |  |  |
| hapticActionNodes | array(HapticActionNode) | M | N/A | List of haptic action nodes. |
| } |  |  |  |  |
| if (type == ACTION_SET_AVATAR) { |  |  |  |  |
| avatarAction | string | M |  | The avatarAction is provided by a URN that uniquely identifies the avatar actions list. For example, the URN of the reference MPEG avatar actions list is given in Annex H. |
| nodes | array |  |  | Indices of the nodes in the nodes array to launch their avatar actions. |
| } |  |  |  |  |

Table 8.2-4: Semantic of action

| Action type | Description |
| --- | --- |
| ACTION_ACTIVATE=0 | Set activation status of a node |
| ACTION_TRANSFORM | Set transform to a node |
| ACTION_BLOCK | Block the transform of a node |
| ACTION_ANIMATION | Select and control an animation |
| ACTION_MEDIA | Select and control a media |
| ACTION_MANIPULATE | Select a manipulate action |
| "ACTION_SET_MATERIAL | Set new material to nodes |
| ACTION_SET_HAPTIC | Get haptic feedbacks on a set of nodes |
| ACTION_SET_AVATAR | Get avatar related actions |

Table 8.2-5: Type of action

| Animation Control | Description |
| --- | --- |
| ANIMATION_PLAY=0 | Play the animation |
| ANIMATION_PAUSE | Pause the animation |
| ANIMATION_RESUME | Resume the animation |
| ANIMATION_STOP | Stop the animation |

Table 8.2-6: Control of animation

| Media Control | Description |
| --- | --- |
| MEDIA_PLAY=0 | Play the media |
| MEDIA_PAUSE | Pause the media |
| MEDIA_RESUME | Resume the media |
| MEDIA_STOP | Stop the media |

Table 8.2-7: control of media

| Action Manipulate Type | description |
| --- | --- |
| ACTION_MANIPULATE_FREE=0 | the nodes follow the user pointing device and its rotation |
| ACTION_MANIPULATE_SLIDE | the nodes move linearly along the provided axis by following the user pointing device |
| ACTION_MANIPULATE_TRANSLATE | the nodes translate by following the user pointing device |
| ACTION_MANIPULATE_ROTATE | the nodes rotate around the provided axis by following the user pointing device |
| ACTION_MANIPULATE_SCALE | performs a central scaling of the nodes by following the user pointing device |

Table 8.2-8: Action Manipulate Type

The semantic of a behavior is provided in Table 8.2-9.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M |  | Indices of the triggers in the triggers array considered for this behavior |
| actions | array | M |  | Indices of the actions in the actions array considered for this behavior |
| triggersCombinationControl | string | M | “” | Set of logical operations to apply to the triggers A ‘#’ indicates the trigger index, ‘&’ indicates a logical AND operation, ‘¦’ a logical OR operation and ‘~’ a NOT operation. Parenthesis are used to group some operations. Such a syntax may give the following string: “#1&~#2¦(#3&#4)”. The default empty string is understood as a logical OR between all the triggers. A regex expression (https://json-schema.org/understanding-json-schema/reference/regular_expressions.html) is specified in the JSON schema to validate this string. |
| triggersActivationControl | string | M | "TRIGGER_ACTIVATE_FIRST_ENTER" | Indicates when the combination of the triggers shall be activated for launching the actions. One element of that Table 8.2-10 defines the activation control of triggers. |
| actionsControl | enum | M | SEQUENTIAL | Defines the way to execute the defined actions. SEQUENTIAL=0: each defined action is executed sequentially in the order of the actions array, PARALLEL=1: the defined actions are executed concurrently |
| interruptAction | number | O |  | Index of the action in the actions array to be executed if the behavior is still on-going and is no more defined in a newly received scene update |
| priority | number | M | 0 | Weight associated to the behavior. Used to select a behavior when several behaviors are active at same time for one node |

Table 8.2-9:semantic of behavior

| Trigger Activation Control | Description |
| --- | --- |
| TRIGGER_ACTIVATE_FIRST_ENTER=0 | activate when the conditions are first met |
| TRIGGER_ACTIVATE_EACH_ENTER | activate each time the conditions are first met |
| TRIGGER_ACTIVATE_ON | activate as long as the conditions are met |
| TRIGGER_ACTIVATE_FIRST | activate when the conditions are first no longer met |
| TRIGGER_ACTIVATE_EACH_EXIT | activate each time the conditions are no longer met |
| TRIGGER_ACTIVATE_OFF | activate as long as the conditions are not met |

Table 8.2-10:Trigger Activation Control

| Property | Type | Required | Default | Description |
| --- | --- | --- | --- | --- |
| node | integer | M | N/A | Id of the node in the glTF nodes array |
| actionLocation | integer | O | 0xFFFFFFFF | Body part mask specifying where on the body the signal can be rendered. |
| washout | boolean | O | False | Specifies whether the action should trigger a washout (reset to the origin) of the associated devices. |
| useCollider | boolean | O | False | Used with a Collision trigger. If True, the rendering engine shall use collision information to estimate the desired location of the haptic feedback on the body. If false, the signal shall be rendered based on the information specified in the Haptic file. |
| materialHapticModality | array(enum) | O | N/A | List of haptic material modalities that shall be rendered. Possible values are detailed in Table 8.2-11. |
| hapticActionMedias | array(HapticActionMedia) | M | N/A | List of Haptic Action Media |

Table 8.2-11: Semantic of HapticActionNode object

| Property | Type | Required | Default | Description |
| --- | --- | --- | --- | --- |
| mediaIndex | integer | M | N/A | Index in the Media_reference array of the MPEG_haptic extension. |
| perceptionIndices | array(integer) | M | N/A | Indices of the perceptions of the media that shall be rendered. If the list if empty all perceptions shall be rendered |
| hapticModality | array(enum) | O | N/A | List of haptic modalities that can be rendered. Possible values are described in Table 8.2-10. |
| hapticControl | string | O | “HAPTIC_PLAY” | One element of Table 8.2-10 that defines the control of the haptic rendering. |
| loop | boolean | O | False | Specifies if the haptic rendering of the data should be continuously looping |

Table 8.2-12: Semantic of the HapticActionMedia object

Pressure

Acceleration

Velocity

Position

Temperature

Vibrotactile

Water

Wind

Force

Electrotactile

Vibrotactile Texture

Stiffness

Friction

Other

Table 8.2-13: List of haptic modalities

Stiffness

Friction

Vibrotactile Texture

Temperature

Vibration

Custom

Table 8.2-14: List of haptic Material Modalities

|  | Name | Body_part_mask (binary) | Hexadecimal | Decimal |
| --- | --- | --- | --- | --- |
| 0 | Unspecified | 00000000000000000000000000000000 | 0x00000000 | 0 |
| 1 | Head Face | 00000000000000000000000000000001 | 0x00000001 | 1 |
| 2 | Head Back/Neck/Ears | 00000000000000000000000000000010 | 0x00000002 | 2 |
| 3 | Mouth Bag | 00000000000000000000000000000100 | 0x00000004 | 4 |
| 4 | Lower Jaw | 00000000000000000000000000001000 | 0x00000008 | 8 |
| 5 | Upper Jaw | 00000000000000000000000000010000 | 0x00000010 | 16 |
| 6 | Eye Left | 00000000000000000000000000100000 | 0x00000020 | 32 |
| 7 | Eye Right | 00000000000000000000000001000000 | 0x00000040 | 64 |
| 8 | Chest Front | 00000000000000000000000010000000 | 0x00000080 | 128 |
| 9 | Chest Back | 00000000000000000000000100000000 | 0x00000100 | 256 |
| 10 | Upper Arm Left | 00000000000000000000001000000000 | 0x00000200 | 512 |
| 11 | Lower Arm Left | 00000000000000000000010000000000 | 0x00000400 | 1 024 |
| 12 | Hand Left | 00000000000000000000100000000000 | 0x00000800 | 2 048 |
| 13 | Upper Arm Right | 00000000000000000001000000000000 | 0x00001000 | 4 096 |
| 14 | Lower Arm Right | 00000000000000000010000000000000 | 0x00002000 | 8 192 |
| 15 | Hand Right | 00000000000000000100000000000000 | 0x00004000 | 16 384 |
| 16 | Abdomen Front | 00000000000000001000000000000000 | 0x00008000 | 32 768 |
| 17 | Abdomen Back | 00000000000000010000000000000000 | 0x00010000 | 65 536 |
| 18 | Upper Leg Left | 00000000000000100000000000000000 | 0x00020000 | 131 072 |
| 19 | Lower Leg Left | 00000000000001000000000000000000 | 0x00040000 | 262 144 |
| 20 | Foot Left | 00000000000010000000000000000000 | 0x00080000 | 524 288 |
| 21 | Upper Leg Right | 00000000000100000000000000000000 | 0x00100000 | 1 048 576 |
| 22 | Lower Leg Right | 00000000001000000000000000000000 | 0x00200000 | 2 097 152 |
| 23 | Foot Right | 00000000010000000000000000000000 | 0x00400000 | 4 194 304 |
| 24-32 | Reserved | 00000000010000000000000000000000 - 10000000000000000000000000000000 | 0x00800000 -0x80000000 | 8 388 608 - 2 147 483 648 |

Table 8.2-15: Body part masks

| Name | Body_part_mask (binary) | Hexadecimal | Decimal |
| --- | --- | --- | --- |
| Right arm | 00000000000000000111000000000000 | 0x00007000 | 28 672 |
| Left arm | 00000000000000000000111000000000 | 0x00000E00 | 3 584 |
| Right leg | 00000000011100000000000000000000 | 0x00700000 | 7 340 032 |
| Left leg | 00000000000011100000000000000000 | 0x000E0000 | 917 504 |
| Upper body | 00000000000000011111111111111111 | 0x0001FFFF | 131 071 |
| Lower body | 00000000011111100000000000000000 | 0x007E0000 | 8 257 536 |
| Full body | 11111111111111111111111111111111 | 0xFFFFFFFF | 4 294 967 295 |

# Table 8.2-16: Examples of body part combinations

| Haptic Control | Description |
| --- | --- |
| “HAPTIC_PLAY” | Start the rendering of the haptic data |
| “HAPTIC_PAUSE” | Pause the rendering of the haptic data |
| “HAPTIC_RESUME” | Resume the rendering of the haptic data |
| “HAPTIC_STOP” | Stop the rendering of the haptic data |

# Table 8.2-17: List of Haptic controls

#### 8.2.2.2	Semantics at node level

In complement to the interactivity objects defined in the glTF scene-level extension, some additional data could be provided at the level of the affected glTF nodes to specialize the trigger activation.

The semantic of the MPEG_node_interactivity glTF extension is shown in 8.2-11.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M | [] | Array of node triggers (as defined in Table 8.2-12). Only distinct types are allowed. The minimum size of this array is 0, and the maximum size is size of trigger types as defined in this specification. |

Table 8.2-11: Semantic of the MPEG_node_interactivity extension

| Name |  | Type |  | Usage |  | Default |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type |  | enumeration |  | M |  | TRIGGER_COLLISION |  | One element of Table 8.2-3 that defines the type of the trigger |
| if (type == TRIGGER_COLLISION){ |  |  |  |  |  |  |  |  |
| Collider |  | integer |  | M |  | N/A |  | the index of the mesh element that provides the collider geometry for the current node. The collider mesh may reference a material. |
| static |  | boolean |  | M |  | True |  | If True, the collider is defined as a static collider. |
| usePhysics |  | boolean |  | M |  | false |  | Indicates if the object shall be considered by the physics simulation. |
| if (usePhysics) { |  |  |  |  |  |  |  |  |
| useGravity |  | boolean |  | M |  | true |  | Indicates if the gravity affects the object |
| mass |  | number |  | M |  | 1 |  | Mass of the object in kilogram. |
| bounciness |  | number |  | M |  | 0.0 |  | Provides the amount that an object bounces in response to a collision. The typical range is [0.0 1.0], where 0.0 represents completely not bouncy and 1.0 represents the maximum bounciest. |
| roughness |  | number |  | M |  | 0.5 |  | Provides the roughness of an object which affects how much the object moves in response to a collision. The typical range is [0.0 1.0]. |
| } |  |  |  |  |  |  |  |  |
| primitives |  | array(Primitive) |  | O |  | [] |  | List of primitives used to activate the proximity or collision trigger. Semantics are presented in table 8.2-5. |
| } |  |  |  |  |  |  |  |  |
| if (type == TRIGGER_PROXIMITY){ |  |  |  |  |  |  |  |  |
| allow_occlusion |  | boolean |  | M |  | True |  | Indicates if occlusion by other nodes should be considered |
| upper_distance_weight |  | number |  | O |  | 1 |  | The weight applied to the distanceUpperLimit parameter defined at scene level |
| lower_distance_weight |  | number |  | O |  | 1 |  | The weight applied to the distanceLowerLimit parameter defined at scene level |
| primitives |  | array(Primitive) |  | O |  | [] |  | List of primitives used to activate the proximity or collision trigger. Semantics are presented in table 8.2-5. |
| } |  |  |  |  |  |  |  |  |
| if (type ==TRIGGER_USER_INPUT){ |  |  |  |  |  |  |  |  |
| userInputParameters |  | array |  | M |  | [] |  | Provides additional information related to the user inputs (eg “max speed = 0.5”) |
| } |  |  |  |  |  |  |  |  |
| if (type== TRIGGER_VISIBILITY){ |  |  |  |  |  |  |  |  |
| visibilityFull |  | boolean |  | M |  | True |  | Indicates if the visibility shall be full or partial to activate the trigger |
| nodes |  | array |  | O |  | [] |  | Set of nodes that shall not be considered for the visibility computation |
| mesh |  | number |  | O |  |  |  | Index of the mesh in the scene meshes array that will be used to compute visibility |
| } |  |  |  |  |  |  |  |  |

Table 8.2-12: Semantic of the MPEG_node_interactivity.trigger extension

### 8.2.3	Processing model

When a scene description file becomes available, the Presentation Engine

- parses the related glTF file

- creates each behavior composed of triggers and actions described at the interactivity scene level extension

- specializes the trigger for each affected node with the additional data provided at the interactivity node level extension if present

At runtime, the presentation Engine iterates on each behavior and performs the following tasks:

- checks the activation status of each trigger of this behavior by following the procedure detailed in Figure 8.2-1,

- checks the logical combination of these trigger status,

- if this status satisfies the triggersActivationControl value, launches the corresponding actions.

Figure 8.2-1: processing model for the activation of a single trigger

When several behaviors are in concurrence to affect the same node(s) at the same time, the behavior having the highest priority is considered.

When a new scene description update is received, the application follows the procedure detailed in Figure 8.2-2. An on-going behavior corresponds to a behavior:

- having its triggers status verifying the triggersActivationControl value for that frame,

- or having previously launched a play action related to a media/animation,

- or having previously launched an action with a delay not yet expired.

Figure 8.2-2: Processing model when a new scene description update is received.

## Avatar

### 8.3.1	General

Signalling an avatar at a node level is achieved by the definition of the MPEG_node_avatar extension.

Depending on the input components and the offline model, the proper Avatar reconstruction and animation needs to be instantiated to generate the final dynamic/animated 3D mesh of the Avatar.

When considering describing a user’s avatar in scene description, the following requirements are derived:

- The avatar may be reconstructed/animated from a wide range of Avatar representations,

- The reconstructed/animated avatar representation abides by the supported primitives in scene description,

- It is possible to associate interactivity triggers with different parts/segments of the user avatar (e.g. hand or finger) using the node interactivity triggers.

To address these requirements, the avatar reconstruction/animation is assumed to be independent of the avatar rendering. This design aligns with the principles and architecture of this specification. Irrespective of the chosen avatar representation format, the Presentation Engine is then only required to render the reconstructed avatar in one of the supported 3D formats, such as a dynamic mesh. The following figure depicts how this approach.

Figure 8.3-1 Avatar media pipeline

The MPEG_node_avatar extension shall be included as extension to node object. All referenced mesh elements by this node will be reconstructed from an Avatar representation.

The mesh of humanoid avatars is assumed to be in a T-pose.

### 8.3.2	Semantics

The application identifies the avatar format and reconstruction method based on the type of the MPEG_node_avatar extension. The default type is defined in Annex H of this specification.

Table 1: Description of `MPEG_node_avatar` extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | string | M | N/A | The type of the Avatar representation is provided as a URN that uniquely identified the Avatar representation scheme. The Avatar representation scheme defines the format of all components that are used to reconstruct and animate the Avatar. The reference MPEG avatar URN is defined in section 8.3.3. |
| mappings | array(Mapping) | M | N/A | The mapping between child nodes and semantics. |

The Mapping object is defined as follows:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| path | AvatarPath | M | N/A | Provides the Avatar path for this node as described in clause 8.3.4. |
| node | integer | M | N/A | The index of the child node of the mesh part that corresponds to the label in the path. |

### 8.3.3	MPEG Reference Avatar

The MPEG reference Avatar shall be identified by “urn:mpeg:sd:2023:avatar” as the Avatar type. The reference Avatar framework builds on the reference Avatar as defined in Annex H.

### 8.3.4	Avatar Path Definition

The Avatar path allows the addressing of every segment in an Avatar, which allows for associating them with interactivity triggers and skeleton animations.

Table 8.3.4-1 Avatar Paths

"full_body": { "upper_body":{ "head":{ "face":{ "mouth", "lower_jaw", "upper_jaw", "eye_right", "eye_left" }, "neck", "ear_right", "ear_left", "back" }, "thorax":{ "chest_front", "chest_back", "shoulder_front_right", "shoulder_front_left", "shoulder_back_right", "shoulder_back_left" }, "arm_right":{ "upper_arm_right", "lower_arm_right", "hand_right":{ "thumb_proximal_right":{ "thumb_intermediate_right":{ "thumb_distal_right" } }, "index_proximal_right":{ "index_intermediate_right":{ "index_distal_right":{ "index_top_right" } } }, "middle_proximal_right":{ "middle_intermediate_right":{ "middle_distal_right":{ "middle_top_right" } } }, "ring_proximal_right":{ "ring_intermediate_right":{ "ring_distal_right":{ "ring_top_right" } } }, "litttle_proximal_right":{ "litttle_intermediate_right":{ "litttle_distal_right":{ "litttle_top_right" } } } } }, "arm_left":{ "upper_arm_left", "lower_arm_left", "hand_left":{ "thumb_proximal_left":{ "thumb_intermediate_left":{ "thumb_distal_left" } }, "index_proximal_left":{ "index_intermediate_left":{ "index_distal_left":{ "index_top_left" } } }, "middle_proximal_left":{ "middle_intermediate_left":{ "middle_distal_left":{ "middle_top_left" } } }, "ring_proximal_left":{ "ring_intermediate_left":{ "ring_distal_left":{ "ring_top_left" } } }, "litttle_proximal_left":{ "litttle_intermediate_left":{ "litttle_distal_left":{ "litttle_top_left" } } } } } } "lower_body":{ "pelvis":{ "pelvis_front":{ "pelvis_front_right", "pelvis_front_left" }, "pelvis_back":{ "pelvis_back_right", "pelvis_back_left" } }, "leg_right":{ "upper_leg_right", "lower_leg_right", "foot_right":{ "toes_right" } }, "leg_left":{ "upper_leg_left", "lower_leg_left", "foot_left":{ "toes_left" } } } }

### 8.3.5	Processing Model

Each avatar in a scene is a node using the MPEG_node_avatar extension. When present in a node, it means that all referenced mesh elements by this node will be reconstructed from an Avatar representation.

## 8.4 	Lighting

### 8.4.1	General

The MPEG_lights_texture_based extension provides the ability to define image-based lights in a glTF scene. This extension expands the functionalities of the EXT_lights_image_based extension by enabling the lighting information to vary over time. The lighting information in the MPEG_lights_texture_based  extension comprises two timed-dependent sequences that are:

- a video sequence wherein each frame represents the specular radiance information

- a timed metadata sequence where each sample of the sequence represents irradiance information

At each time instance of the rendering of the glTF scene, the corresponding specular radiance decoded frame and the irradiance sample provide the necessary information to apply image-based lighting rendering techniques.

When present, the MPEG_lights_texture_based extension shall be included as a top-level extension.

### 8.4.2	Semantics

The definition of all objects within MPEG_lights_texture_based extension is provided in Table 1.

Table 3 – Definitions of top-level objects of MPEG_lights_texture_based extension

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| lights | array | N/A | M | An array of items that describe the texture-based light sources, referenced in this scene description document. |

Table 4 – Definitions of item in the lights array of MPEG_lights_texture_based extension

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| name | string | N/A | O | Name of the light |
| nature | string | physical | O | Indicates whether the lighting information corresponds to a physical light, a virtual light, both or unknown. Editor’s note: To be re-evaluated in light of the possible work on real environment integration in SD whether this attribute remains here or goes to a new node to be defined. |
| position | array | (0,0,0) | O | Position of both light and reflection probes associated with the lighting information In the absence of this position, the lighting information is assumed to be global (i.e. for the entire scene) for which the center is the scene origin. When an anchor refers to a light source, the position of the anchor takes precedence over this position attribute. Editor’s note: The case of one or more light sources being at the same position needs to be clarified whether it is allowed or not and possibly with new scene validation rules. |
| projection | string | equirectangular | O | Provides the projection type (which may be equirectangular or cubemap) of the specular images. In the latter case, the 6 faces of the radiance map should be packed according to the order described by the OMAF specification in section 5.2.3. Editor’s note: To be studied if the projection could reference a general specification and not an application standard that has no relation with ISO/IEC 23090-14. |
| rotation_accessor | integer | N/A | O | Provides a reference to the accessor giving a sequence of quaternions as described in rotation of in EXT_lights_image_based. When an anchor refers to a light source, the rotation of the anchor takes precedence over this rotation attribute. Editor’s note: Study if position and rotation attribute should be consistent, i.e. either both static or both dynamic. Editor’s note: Study if improvements can be done in case the information is static, i.e. not using an accessor just for one value. |
| intensity_accessor | integer | N/A | O | Provides a reference to the accessor giving a sequence of intensity values as described in intensity of in EXT_lights_image_based. Editor’s note: The absence of the parameter needs to be specified. Editor’s note: Study if possible improvement can be done in case the information is static, i.e. not using an accessor just for one value. |
| irradiance_accessor | integer | N/A | M | Provides a reference to the accessor giving a sequence of irradiance coefficient samples. Irradiance coefficients are defined as in irradianceCoefficients of EXT_lights_image_based. See Khronos EXT_lights_image_based for the definition of the irradiance coefficient sample. Editor’s note: study if sample format needs to be specified and provide it if desired. Editor’s note: she absence of the parameter needs to be specified. Editor’s note: study if improvements can be done in case the information is static, i.e. not using an accessor just for one value. |
| specular_images | array | N/A | M | Provides a list of references to textures providing specular images (environment map). The length of the array shall be equal to “1” or “6”. When the length of the array is equal to “1”, the specular images are provided in a packed format as defined by the projection attribute. When the length of the array is equal to “6”, the projection attribute shall be of type cubemap and the order of faces are TBD. Editor’s note: Study if Khronos definition could be used, i.e. cube faces are defined in the following order and adhere to the standard orientations as shown in the Khronos EXT_lights_image_based extension. Editor’s note: The absence of the parameter needs to be specified. |

Table 5 – Definitions of object at the scene level of MPEG_lights_texture_based

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| light | array(integer) | N/A | M | Reference to an item in the lights array of the MPEG_lights_texture_based extension. |

The JSON schema for the MPEG_lights_texture_based extension is provided in A.1.

### 8.4.3	Processing model

Add the following to Annex A

A.10 JSON schema for MPEG_scene_anchor and MPEG_node_anchor

MPEG_scene_anchor schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_scene_anchor.schema.json.

MPEG_node_anchor schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_node_anchor.schema.json.

A.11 JSON schema for MPEG_haptics

MPEG_haptics.schema.json schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_haptics.schema.json.

A.12 JSON schema for MPEG_scene_interactivity and MPEG_node_interactivity

MPEG_scene_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_scene_interactivity.schema.json

MPEG_node_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_node_interactivity.schema.json

A.13 	JSON schema for MPEG_avatar

MPEG_avatar schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_avatar.schema.json

A.14		JSON schema for MPEG_lights_texture_based

MPEG_lights_texture_based schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_lights_texture_based.schema.json

Add the following to Annex G

## G.3 	Support for MPEG-I Haptics

### G.3.1	General

We propose to implement the concepts of haptics in SD through two glTF extensions, MPEG_haptic and MPEG_material_haptic. The proposed extensions are detailed in the following sections.

Figure G.3-1: Proposed location of the extensions for haptics among all MPEG extensions to glTF

The following examples detail several use cases of the Phase 1 haptic signals:

| Use Case | Phase 1 material definition | Rendering behavior |
| --- | --- | --- |
| Touch the surface of a wall and feel a texture | A Texture perception is defined describing a surface friction pattern. | Upon collision between a body part and the textured wall, the Texture track of the media is read based on the displacement of the body part on the surface. |
| Press a button to get stiffness feedback | A Stiffness/force feedback perception is defined describing a force curve profile. | When the button is pressed, the force profile curve is read based on the pressure depth of the button. |
| Opening a door and feeling friction of the door with the floor | A Texture perception is defined describing a surface friction pattern. | When the door is moving, the Texture track of the media is read based on the displacement of the door (rotation angle converted to distance traveled). |
| Walking in a street and at some time rain is falling on the user body | Localized vibrations are defined to simulate raindrop | Upon time event “rain starts” vibrations patterns are synthesized and rendered on the appropriate device. |

### G.3.2	 Semantics for MPEG_haptic

The MPEG_haptic extension represents haptics data defined in Phase 1. It is an independent haptic media like an audio content or an image. This extension is attached at the node level to indicate that the node refers to haptic media. There are a number of reasons to have a dedicated extension for haptics instead of using MPEG_media.media directly:

- It encapsulates all references to haptic media in one location

- It explicitly identifies haptic enabled nodes without having to go through other extensions (e.g. MPEG_interactivity)

- It provides room for future extensions (typically for Haptic Phase 2A and Phase 2B)

- Using MPEG_media.media directly with the interactivity extension would be problematic for the support of spatial haptics since that extension is associated with timed media.

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| Media_reference | Array<Integer> | N/A | A reference to one or more media sources in MPEG_media.media array containing haptic media files. |

The JSON schema associated with this medium is already defined in Phase 1 (http://mpegx.int-evry.fr/software/haptics/mpeg_haptics_json_specifications).

### G.3.3	Semantics MPEG_material_haptic

The MPEG_material_haptic extension is a type of material defined to provide a way to enable the association of haptic media with the interactivity extension. The list of haptic properties defined here is based on the work from [10]. The main concept is that the haptic texture associated with a 3D object does not contain RGB values but haptic values. These values are exploited directly by the haptic renderer. The proposed extension also uses the concept of taxels as introduced in [11]. Using this principle, each pixel of the texture can be mapped to a distinct spatial (or temporal) signal as illustrated in Figure 2.

Figure G.3-2: Grid of taxels, mapping pixels to  different haptic textures

We here propose to combine these types of haptic texture representations by using an array of textures for each haptic property. With this method, a haptic texture can be provided both as a traditional 2D texture and as a taxel map in the same file, giving the possibility to the rendering engine to choose the most appropriate.

An additional information is added to each element of the haptic texture arrays for the rendering engine to adequately interpret a texture. Each array element then contains a haptic texture and a texture type expressed as an Enumeration. Possible values of the enumeration are:

- High_Resolution: The haptic texture is a high resolution 2D texture directly storing haptic values

- Low_Resolution: The haptic texture is a low resolution 2D texture directly storing haptic values

- Reference: The haptic texture is a 2D taxel map containing references to haptic signals. Each pixel of the texture corresponds to an index in the Media_reference array of the MPEG_haptic extension.

- Other: Unknown proprietary texture format.

To interpret the data contained in 2D textures, the bit depth and range of these textures are specified according to the following tables.

The following table gives the bit depth and range values for each haptic property for low resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| Stiffness | 8-bit | 0-10000 N.s −1 /m.s −1 | 40 N.s −1 /m.s −1 |
| Friction | 8-bit | ±5 | 0.04 |
| Vibrotactile Texture | 8-bit | ±10 | 0.08 |
| Temperature | 8-bit | [-50:+75]°C | 0.5°C |
| Vibration | 8-bit | [0-1] | 0,004 |
| Custom | 8-bit | 0-255 | 1 |

The following table gives the bit depth and range values for each haptic property for high resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| Stiffness | 16-bit | 0-10000 N.s −1 /m.s −1 | 0.15 N.s −1 /m.s −1 |
| Friction | 16-bit | ±100 | 0.003 |
| Vibrotactile Texture | 16-bit | ±100 | 0.0015 |
| Temperature | 16-bit | [-100:+150]°C | 0.004°C |
| Vibration | 8-bit (amplitude) 8-bit (frequency) | [0-1] [0-300] Hz | 0.004 1.17Hz |
| Custom | 16-bit | 0-65535 | 1 |

For the high resolution texture, values of each texture map pixel are divided in two: the first byte contains the magnitude value and the second byte contains the frequency.

The following table describes the list of haptic properties of the extension:

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| Stiffness | array<Enum, textureInfo> | NULL | It determines the perceived stiffness of a surface. Which means the force perceived by the user opposed to the normal penetration of a material by a body part. It is described with a texture storing the stiffness coefficients. The suggested rendering model is : F = kx where k is the value of stiffness for the displacement x along the asset stiffness function. This model is valid for an isotropic material. |
| Friction | array<Enum, textureInfo> | NULL | It indicates the perceived friction, which is a force opposing the movement of a body part sliding on a surface. It is described with a texture storing the coefficient of friction. The suggested rendering model is: F_f = mu * Fn where mu is the coefficient of friction, and Fn is the normal applied force by the body part on the surface. |
| Vibrotactile Texture | array<Enum, textureInfo> | NULL | It indicates the perceived texture by a body part while sliding on a surface. |
| Temperature | array<Enum, textureInfo> | NULL | It indicates the perceived temperature of an object. It is described with a texture storing the temperature distribution. |
| Vibration | array<Enum, textureInfo> | NULL | It indicates a vibration signal. It is described with a texture storing the amplitude and / or frequency of the signal. |
| Custom | array<Enum, textureInfo> | NULL | Texture containing custom haptic data. |

### G.3.4	Processing Model

The Presentation Engine shall support the MPEG_haptic and MPEG_material_haptic extensions.

In Haptics Phase 2A, the user avatar representation consists only of the camera, a collider (i.e., invisible user representation) and associated view frustum (see Figure 3). In this model, the collisions that occur during interaction are between the view frustum geometry and the objects in the scene (see Figure 3 object #1). During interaction, the presentation engine will generate a list of collided objects at each rendering frame. The collided objects that also have an associated MPEG_material_haptic will then be inspected to determine their associated interaction model and this information, along with the haptic media, will drive the creation of a haptic effect to be rendered by available haptic peripherals.

Another type of collision may happen when an animated object moves toward the user and touches the collider (potentially outside of the user’s field of view, see Figure 3 object #2). This would trigger haptic feedback on the touched body part.

Of course, any haptic content defined in Phase 1 could be triggered like any media (see Figure 3 object #3 with a potential defined body part)

Note that mixing, transcoding and other haptic media manipulation is non-normative to the interactivity model. If a reference to one or multiple MPEG_haptic is made within an interaction, the definition of the action allows to link a haptic behavior to this model and to drive the haptic rendering from the action itself.

Examples of how haptics would be driven by spatial displacement actions and additional parameters to tune the rendering are provided bellow

| Action | Default haptic behavior | Override haptic parameters |
| --- | --- | --- |
| None | N/A | N/A |
| Free movement | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, fix rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, pivot rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Sliding | Read Texture based on slider displacement distance as input. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Rotation around pivot | Read Texture based on distance traveled during the rotation. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Button | Read force feedback curve based on the current depth of the button. | Rescale the haptics rendering independent variable by an arbitrary length. |

Spatial displacement does not need to drive time-based haptic feedback (such as vibrations).

Add the following clause to Annex G.

## G.4 	Support for MPEG-I Haptics

### G.4.1	General

We propose to implement the concepts of haptics in SD through two glTF extensions, MPEG_haptic and MPEG_material_haptic. The proposed extensions are detailed in the following sections.

Figure G.4-1: Proposed location of the extensions for haptics among all MPEG extensions to glTF

The following examples detail several use cases of the Phase 1 haptic signals:

| Use Case | Phase 1 material definition | Rendering behavior |
| --- | --- | --- |
| Touch the surface of a wall and feel a texture | A Texture perception is defined describing a surface friction pattern. | Upon collision between a body part and the textured wall, the Texture track of the media is read based on the displacement of the body part on the surface. |
| Press a button to get stiffness feedback | A Stiffness/force feedback perception is defined describing a force curve profile. | When the button is pressed, the force profile curve is read based on the pressure depth of the button. |
| Opening a door and feeling friction of the door with the floor | A Texture perception is defined describing a surface friction pattern. | When the door is moving, the Texture track of the media is read based on the displacement of the door (rotation angle converted to distance traveled). |
| Walking in a street and at some time rain is falling on the user body | Localized vibrations are defined to simulate raindrop | Upon time event “rain starts” vibrations patterns are synthesized and rendered on the appropriate device. |

### G.4.2	 Semantics for MPEG_haptic

The MPEG_haptic extension represents haptics data defined in Phase 1. It is an independent haptic media like an audio content or an image. This extension is attached at the node level to indicate that the node refers to haptic media. There are a number of reasons to have a dedicated extension for haptics instead of using MPEG_media.media directly:

- It encapsulates all references to haptic media in one location

- It explicitly identifies haptic enabled nodes without having to go through other extensions (e.g. MPEG_interactivity)

- It provides room for future extensions (typically for Haptic Phase 2A and Phase 2B)

- Using MPEG_media.media directly with the interactivity extension would be problematic for the support of spatial haptics since that extension is associated with timed media.

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| Media_reference | Array<Integer> | N/A | A reference to one or more media sources in MPEG_media.media array containing haptic media files. |

The JSON schema associated with this medium is already defined in Phase 1 (http://mpegx.int-evry.fr/software/haptics/mpeg_haptics_json_specifications).

### G.4.3	Semantics MPEG_material_haptic

The MPEG_material_haptic extension is a type of material defined to provide a way to enable the association of haptic media with the interactivity extension. The list of haptic properties defined here is based on the work from [10]. The main concept is that the haptic texture associated with a 3D object does not contain RGB values but haptic values. These values are exploited directly by the haptic renderer. The proposed extension also uses the concept of taxels as introduced in [11]. Using this principle, each pixel of the texture can be mapped to a distinct spatial (or temporal) signal as illustrated in Figure 2.

Figure G.3-2: Grid of taxels, mapping pixels to  different haptic textures

We here propose to combine these types of haptic texture representations by using an array of textures for each haptic property. With this method, a haptic texture can be provided both as a traditional 2D texture and as a taxel map in the same file, giving the possibility to the rendering engine to choose the most appropriate.

An additional information is added to each element of the haptic texture arrays for the rendering engine to adequately interpret a texture. Each array element then contains a haptic texture and a texture type expressed as an Enumeration. Possible values of the enumeration are:

- High_Resolution: The haptic texture is a high resolution 2D texture directly storing haptic values

- Low_Resolution: The haptic texture is a low resolution 2D texture directly storing haptic values

- Reference: The haptic texture is a 2D taxel map containing references to haptic signals. Each pixel of the texture corresponds to an index in the Media_reference array of the MPEG_haptic extension.

- Other: Unknown proprietary texture format.

To interpret the data contained in 2D textures, the bit depth and range of these textures are specified according to the following tables.

The following table gives the bit depth and range values for each haptic property for low resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| Stiffness | 8-bit | 0-10000 N.s −1 /m.s −1 | 40 N.s −1 /m.s −1 |
| Friction | 8-bit | ±5 | 0.04 |
| Vibrotactile Texture | 8-bit | ±10 | 0.08 |
| Temperature | 8-bit | [-50:+75]°C | 0.5°C |
| Vibration | 8-bit | [0-1] | 0,004 |
| Custom | 8-bit | 0-255 | 1 |

The following table gives the bit depth and range values for each haptic property for high resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| Stiffness | 16-bit | 0-10000 N.s −1 /m.s −1 | 0.15 N.s −1 /m.s −1 |
| Friction | 16-bit | ±100 | 0.003 |
| Vibrotactile Texture | 16-bit | ±100 | 0.0015 |
| Temperature | 16-bit | [-100:+150]°C | 0.004°C |
| Vibration | 8-bit (amplitude) 8-bit (frequency) | [0-1] [0-300] Hz | 0.004 1.17Hz |
| Custom | 16-bit | 0-65535 | 1 |

For the high resolution texture, values of each texture map pixel are divided in two: the first byte contains the magnitude value and the second byte contains the frequency.

The following table describes the list of haptic properties of the extension:

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| Stiffness | array<Enum, textureInfo> | NULL | It determines the perceived stiffness of a surface. Which means the force perceived by the user opposed to the normal penetration of a material by a body part. It is described with a texture storing the stiffness coefficients. The suggested rendering model is : F = kx where k is the value of stiffness for the displacement x along the asset stiffness function. This model is valid for an isotropic material. |
| Friction | array<Enum, textureInfo> | NULL | It indicates the perceived friction, which is a force opposing the movement of a body part sliding on a surface. It is described with a texture storing the coefficient of friction. The suggested rendering model is: F_f = mu * Fn where mu is the coefficient of friction, and Fn is the normal applied force by the body part on the surface. |
| Vibrotactile Texture | array<Enum, textureInfo> | NULL | It indicates the perceived texture by a body part while sliding on a surface. |
| Temperature | array<Enum, textureInfo> | NULL | It indicates the perceived temperature of an object. It is described with a texture storing the temperature distribution. |
| Vibration | array<Enum, textureInfo> | NULL | It indicates a vibration signal. It is described with a texture storing the amplitude and / or frequency of the signal. |
| Custom | array<Enum, textureInfo> | NULL | Texture containing custom haptic data. |

### G.4.4	Processing Model

The Presentation Engine shall support the MPEG_haptic and MPEG_material_haptic extensions.

In Haptics Phase 2A, the user avatar representation consists only of the camera, a collider (i.e., invisible user representation) and associated view frustum (see Figure 3). In this model, the collisions that occur during interaction are between the view frustum geometry and the objects in the scene (see Figure 3 object #1). During interaction, the presentation engine will generate a list of collided objects at each rendering frame. The collided objects that also have an associated MPEG_material_haptic will then be inspected to determine their associated interaction model and this information, along with the haptic media, will drive the creation of a haptic effect to be rendered by available haptic peripherals.

Another type of collision may happen when an animated object moves toward the user and touches the collider (potentially outside of the user’s field of view, see Figure 3 object #2). This would trigger haptic feedback on the touched body part.

Of course, any haptic content defined in Phase 1 could be triggered like any media (see Figure G.4-1 object #3 with a potential defined body part)

Note that mixing, transcoding and other haptic media manipulation is non-normative to the interactivity model. If a reference to one or multiple MPEG_haptic is made within an interaction, the definition of the action allows to link a haptic behavior to this model and to drive the haptic rendering from the action itself.

Examples of how haptics would be driven by spatial displacement actions and additional parameters to tune the rendering are provided bellow.

| Action | Default haptic behavior | Override haptic parameters |
| --- | --- | --- |
| None | N/A | N/A |
| Free movement | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, fix rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, pivot rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Sliding | Read Texture based on slider displacement distance as input. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Rotation around pivot | Read Texture based on distance traveled during the rotation. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Button | Read force feedback curve based on the current depth of the button. | Rescale the haptics rendering independent variable by an arbitrary length. |

Spatial displacement does not need to drive time-based haptic feedback (such as vibrations).

Add the following Annex H

Annex H
Reference Avatar

## H.1	Introduction

This Annex provides a description of a reference topology for the representation of humanoid avatars. The number of vertices and triangles is defined, with the associated semantic. Different levels of detail (LoD) from the same topology are also defined. The associated glTF files are downloadable from X.

The MPEG reference Avatar shall be identified by “urn:mpeg:sd:2023:avatar” as the Avatar type.

## H.2	Body Model

### H.2.1	 General

The reference body avatar (Figure H.1) comes with its own mesh topology modeled as a female (right side) or as a male body (left side). The body base mesh is composed of three levels of details. As described in Figure H.1, the density of vertex is not the same for the body and the face, because the later requires more accuracy for realism.

The different parts are described into more details in the following sections.

Female

Male

Figure H.1: Full body mesh topology (Right side: female version, Left side: male version). The neutral genre face is zoomed for better visualization (right picture).

### H.2.1	Level of Details

The reference body avatar is proposed in three levels of details (mesh resolution): Low, Medium and High (see Figure H.2). The goal of these different resolutions is to ensure that the mesh topology is supported on the runtime platform (mobile, PC, HMD etc.).

Each level is described and referenced as:

- High Resolution (referenced as Morgan_HD)

  - 53,695 vertices and 56,496 quad faces (106,952 triangular faces).

- Medium Resolution (referenced as Morgan_MD)

  - 13,225 vertices and 13,196 quad faces (26,356 triangular faces)

  - Corresponds to 25% of the HD mesh

- Low Resolution (referenced as Morgan_SD)

  - 5,366 vertices and 5,300 quad faces (10,571 triangular faces).

  - Corresponds to 90% of the HD mesh

The mesh decimation ensures that vertices positions are the same between from a lower level to a higher level. Meaning that all vertices of the SD mesh are all on the MD mesh and all vertices of the MD mesh are on the HD mesh. So original vertices are kept.

Figure H.2: Levels of details of Morgan - From left to right: High (50k), Medium (12k) and Small (5k).

The associated files are named morgan_SD.gltf, morgan_MD.gltf and morgan_HD.gltf.

### H.2.3	Body Semantics

The body is divided into 18 semantic areas allowing to directly refer to them for specific use cases and applications.

Figure H.3, Figure H.4, and Figure H.5 describes those 18 areas for respectively the front body, the back of the body and the head. The corresponding semantics and vertex coordinates are given by Table H.1. The numbering and order are the same as in the reference glTF file provided. It allows easy access to parts of the body by any applications, even without knowing the model.

The provided semantic list relates to the High Resolution level, aka Morgan_HD.

Figure H.3: front view of body semantic areas (left: areas, right: wireframe areas).

Figure H.4: back view of body semantic areas (left: areas, right: wireframe areas).

Figure H.5: front view of the head. Top line: segmentation areas, bottom line: wireframe areas. (Left) complete head view with the face (white), left eye (red) and right eye (olive green) - (Middle) internal view without the face with the mouth bag (blue) - (Right) view without the face and the mouth bag - upper jaw (light green) and lower jaw (purple).

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Full | None |  | 53695 | [0:53694] | 53496 | 106952 | [0:53495] |
| Upper Body | None |  | 45518 | [0:45517] | 45290 | 90540 | [0:45289] |
| - Head | None |  | 36584 | [0:36583] | 36362 | 72684 | [0:36361] |
| n Face |  | (1, 1, 1) | 6873 | [0:6872] | 6785 | 13570 | [0:6784] |
| n Back/Neck/Ears |  | (0,0,0) | 4535 | [6873:11407] | 4566 | 9132 | [6785:11350] |
| n Mouth Bag |  | (0,0.5,0.5) | 791 | [11408:12198] | 823 | 1646 | [11351:12173] |
| n Lower Jaw |  | (0.5, 0, 0.5) | 10440 | [12199:22638] | 10380 | 20760 | [12174:22553] |
| n Upper Jaw |  | (0.5, 1, 0.5) | 11107 | [22639:33745] | 11040 | 22080 | [22554:33593] |
| n Eye Left |  | (0.5,0,0) | 1419 | [33746:35164] | 1384 | 2748 | [33594:34977] |
| n Eye Right |  | (0.5,0.5,0) | 1419 | [35165:36583] | 1384 | 2748 | [34978:36361] |
| - Chest | None |  | 1828 | [36584:38411] | 1792 | 3584 | [36362:38153] |
| n Chest Front |  | (0.5, 1, 0) | 1140 | [36584:37723] | 1108 | 2216 | [36362:37469] |
| n Chest Back |  | (0.5, 0, 1) | 688 | [37724:38411] | 684 | 1368 | [37470:38153] |
| - Arm Left | None |  | 3553 | [38412:41964] | 3568 | 7136 | [38154:41721] |
| n Upper Arm Left |  | (1, 0.5, 1) | 320 | [38412:38731] | 320 | 640 | [38154:38473] |
| n Lower Arm Left |  | (1, 0, 0) | 512 | [38732:39243] | 512 | 1024 | [38474:38985] |
| n Hand Left |  | (0,1,0) | 2721 | [39244:41964] | 2736 | 5472 | [38986:41721] |
| - Arm Right | None |  | 3553 | [41965:45517] | 3568 | 7136 | [41722:45289] |
| n Upper Arm Right |  | (0, 1, 1) | 320 | [41965:42284] | 320 | 640 | [41722:42041] |
| n Lower Arm Right |  | (1, 0, 1) | 512 | [42285:42796] | 512 | 1024 | [42042:42553] |
| n Hand Right |  | (0,1,0.5) | 2721 | [42797:45517] | 2736 | 5472 | [42554:45289] |
| Lower Body | None |  | 8177 | [45518:53694] | 8206 | 16412 | [45290:53495] |
| - Abdomen | None |  | 1855 | [45518:47372] | 1856 | 3712 | [45290:47145] |
| n Abdomen Front |  | (0, 0, 1) | 929 | [45518:46446] | 896 | 1792 | [45290:46185] |
| n Abdomen Back |  | (0.5, 0.5, 0.5) | 926 | [46447:47372] | 960 | 1920 | [46186:47145] |
| - Leg Left | None |  | 3161 | [47373:50533] | 3175 | 6350 | [47146:50320] |
| n Upper Leg Left |  | (0.5, 0.5, 1) | 681 | [47373:48053] | 680 | 1360 | [47146:47825] |
| n Lower Leg Left |  | (1, 0.5, 0) | 815 | [48054:48868] | 814 | 1628 | [47826:48639] |
| n Foot Left |  | (1, 1, 0) | 1665 | [48869:50533] | 1681 | 3362 | [48640:50320] |
| - Leg Right | None |  | 3161 | [50534:53694] | 3175 | 6350 | [50321:53495] |
| n Upper Leg Right |  | (1, 0.5, 0.5) | 681 | [50534:51214] | 680 | 1360 | [50321:51000] |
| n Lower Leg Right |  | (0, 0.5, 0) | 815 | [51215:52029] | 814 | 1628 | [51001:51814] |
| n Foot Right |  | (1, 1, 0.5) | 1665 | [52030:53694] | 1681 | 3362 | [51815:53495] |

Table H.1: Semantics table – The visual and geometric details of each semantic area are provided. For each of them, the number of vertices, quad and triangular faces are provided with the ID ranges covered by the areas.

### H.2.4	Base UV

The reference avatar comes with its own UV coordinates. It is based on UDIM (U-Dimension, an enhancement to the UV mapping and texturing workflow that makes UV map generation easier and assigning textures simpler) which consists of a tile system where each tile is a different texture with its own UV space.

The following look-up table provides the link between the tile on the bottom UDIM map and the corresponding body parts:

| Upper Jaw | Lower Jaw | Right Eyeball |  | Left Iris |
| --- | --- | --- | --- | --- |
| Face | Body | Left Eyeball | - Right Lens - Left Lens | Right Iris |

The UDIM map for the full morgan’s body is illustrated in Figure H.6, and provided with the associated file morgan_HD.gltf. The medium resolution morgan_MD.gltf and low resolution morgan_SD.gltf of the mesh also comes with their similar but decimated UDIM map.

Figure H.6: UDIM map of the full body - It goes from UV space (0-1/0-1) on the bottom left corner to UV space (4-5/1-2) to the upper right corner.

### H.2.5	Base Skeleton

Skeleton’s joints are identified by names and hierarchy with the following structure and naming convention to enable an easy identification of the avatar skeleton (see complete nomenclature in Table H.2).

#### H.2.5.1 Complete Skeleton

Morgan’s skeleton is composed of 63 joints, and Figure H.7 precisely depicts the place of the skeleton’s joints in the hierarchy.

| Body Hierarchy (25 joints) | Left hand hierarchy (19 joints) | Right hand hierarchy (19 joints) |
| --- | --- | --- |
| Hips --- Spine --- --- Chest --- --- --- UpperChest --- --- --- --- Shoulder_Left --- --- --- --- --- UpperArm_Left --- --- --- --- --- --- LowerArm_Left --- --- --- --- --- --- --- Hand_Left --- --- --- --- --- --- --- --- See below --- --- --- --- Shoulder_Right --- --- --- --- --- UpperArm_Right --- --- --- --- --- --- LowerArm_Right --- --- --- --- --- --- --- Hand_Right --- --- --- --- --- --- --- --- See below --- --- --- --- Neck --- --- --- --- --- Head --- --- --- --- --- --- Eye_Left --- --- --- --- --- --- Eye_Right --- --- --- --- --- --- Jaw --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- --- --- --- Toes_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right --- --- --- --- Toes_Right | (Hand_Left) --- ProximalThumb_Left --- --- IntermediateThumb_Left --- --- --- DistalThumb_Left --- ProximalIndex_Left --- --- IntermediateIndex_Left --- --- --- DistalIndex_Left --- --- --- --- TopIndex_Left --- ProximalMiddle_Left --- --- IntermediateMiddle_Left --- --- --- DistalMiddle_Left --- --- --- --- TopMiddle_Left --- ProximalRing_Left --- --- IntermediateRing_Left --- --- --- DistalRing_Left --- --- --- --- TopRing_Left --- ProximalLittle_Left --- --- IntermediateLittle_Left --- --- --- DistalLittle_Left --- --- --- --- TopLittle_Left | (Hand_Right) --- ProximalThumb_Right --- --- IntermediateThumb_Right --- --- --- DistalThumb_Right --- ProximalIndex_Right --- --- IntermediateIndex_Right --- --- --- DistalIndex_Right --- --- --- --- TopIndex_Right --- ProximalMiddle_Right --- --- IntermediateMiddle_Right --- --- --- DistalMiddle_Right --- --- --- --- TopMiddle_Right --- ProximalRing_Right --- --- IntermediateRing_Right --- --- --- DistalRing_Right --- --- --- --- TopRing_Right --- ProximalLittle_Right --- --- IntermediateLittle_Right --- --- --- DistalLittle_Right --- --- --- --- TopLittle_Right |

#### H.2.5.2 	Example of Skeletal Hierarchies

.

| Simplified |  | Simplistic |
| --- | --- | --- |
| Body Hierarchy (24 joints) |  | Body Hierarchy (15 joints) |
| Hips --- Spine --- --- UpperChest --- --- --- Shoulder_Left --- --- --- --- LowerArm_Left --- --- --- --- --- Hand_Left --- --- --- --- --- --- ProximalThumb_Left --- --- --- Shoulder_Right --- --- --- --- LowerArm_Right --- --- --- --- --- Hand_Right --- --- --- --- --- --- ProximalThumb_Right --- --- --- --- Neck --- --- --- --- --- Head --- --- --- --- --- --- Eye_Left --- --- --- --- --- --- Eye_Right --- --- --- --- --- --- Jaw --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- --- --- --- Toes_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right --- --- --- --- Toes_Right |  | Hips --- UpperChest --- --- Shoulder_Left --- --- --- LowerArm_Left --- --- --- --- Hand_Left --- --- Shoulder_Right --- --- --- LowerArm_Right --- --- --- --- Hand_Right --- --- --- Head --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right |

Table H.6: Skeleton joints nomenclature,

Figure H.7: Position of the joints for the MPEG reference geometrical model.

## H.3	Face Model

### H.3.1	 General

The reference facial avatar consists of a base topology and a gender-neutral morphology. It corresponds to the “head area” in the semantics Table H.1 and its geometry is displayed in Figure H.8. The facial morphology comes from a combined dataset of high-resolution face scans (males and females) mapped to the same base topology using 3D reconstruction facial technique [Danieau et al. 2019], and thus does not correspond to an existing person nor cannot allow the reconstruction of one person's image.

Figure H.8: MPEG Morgan faces (right to left: front, back and right profile).

### H.3.2	Base Mesh

The corresponding topology is depicted in Figure H.8. The base face mesh is composed of 36,584 vertices / 36,362 quad faces (and 72,784 triangular faces). As illustrated on Figure H.9, four internal parts are added:

- a mouth bag (Figure H.9a)

- an upper (Figure H.9b) and lower jaw (H.9c)

- two eyeballs (Figure H.HH.9d)

Topological details of these specific parts can be found in the table H.1.

Figure H.9: MPEG Morgan’s face details 
((a) mouth bag, 
(b) upper jaw, 
(c) lower jaw, 
(d) eyeballs).

### H.3.3	Facial Blend Shapes

The facial expression shapes naming is close but not equal to the FACS naming convention defined and used in anatomy to classify human facial motions (See in Paul Ekman’s book [Ekman et al. 1978]).

Shapes are separated into Left (“_L”) and Right (“_R”) components. And sometimes these last components have been split into (“_1”) and (“_2”) sub---components to increase the precision.

The shapes of Morgan are the given in table H.3.

| AU | FACS Name | Morgan shape |
| --- | --- | --- |
| 1 | Inner Brow Raiser | AU1_Inner_Brow_Raiser |
| 2 | Outer Brow Raiser | AU2_Outer_Brow_Raiser_L1 + L2 + R1 + R2 |
| 4 | Brow Lowerer | AU4_Brow_Lowerer_L + R |
| 5 | Upper Lid Raiser | AU5_Upper_Lid_Raiser_L + R |
| 6 | Cheek Raiser | AU6_Cheek_Raiser_L + R |
| 7 | Lig Tightener | AU7_Lid_Tightener_L + R |
| 9 | Nose Wrinkler | AU9_Nose_Wrinkler_L + R |
| 10 | Upper Lip Raiser | AU10_Upper_Lip_Raiser_L + R |
| 11 | Nasolabial Deepener | AU11_Nasolabial_Deepener_L + R |
| 12 | Lip Corner Puller | AU12_Lip_Corner_Puller_L + R |
| 14 | Dimpler | AU14_Dimpler_L + R |
| 15 | Lip Corner Depressor | AU15_Lip_Corner_Depressor_L + R |
| 16 | Lower Lip Depressor | AU16_Lower_Lip_Depressor_L |
| 17 | Chin Raiser | AU17_Chin_Raiser |
| 18 | Lip Pucker | AU18_Lip_Pucker |
| 20 | Lip Stretcher | AU20_Lip_Stretcher_L + R |
| 22 | Lip Funneler | AU22_Funneler |
| 23 | Lip Tightener | AU23_Lip_Tightener |
| 24 | Lip Pressor | AU24_Lip_Pressor |
| 26 | Jaw Drop | AU26_Jaw_Drop |
| 27 | Mouth Stretch | AU27_Mouth_Stretcher |
| 28 | Lip Suck | AU28_Lip_Suck_Low + Up |
| 29 | Jaw Thrust | AU29_Jaw_Thrust |
| 30 | Jaw Sideways | AU30_Jaw_Sideways_L + R |
| 31 | Jaw Clencher | AU31_Jaw_Clencher |
| 34 | Cheek Puff | AU34_Cheek_Puff_L + R |
| 38 | Nostril Dilator | AU38_Nostril_Dilator |
| 43 | Eyes Closed | AU43_Eyes_Closed_L + R |
| 61 | Eyes Turn Left | AU61_Eyes_Turn_Left |
| 62 | Eyes Turn Right | AU62_Eyes_Turn_Right |
| 63 | Eyes Up | AU63_Eyes_Up |
| 64 | Eyes Down | AU64_Eyes_Down |

Table H.3: MPEG Morgan’s facial blend shapes,

All blendshapes are accessible as individual glTF files names with the corresponding AU name and also in a more complete glTF file name morgan_rigged.gltf

### H.3.4	Facial Landmarks

To help processing and animation, a set of 68 semantic facial landmarks is also provided (see Figure H.10). It can be useful to drive mesh retargeting algorithm (retopoligization) or to link facial performance tracking solution to Morgan’s face. The number 68 was picked up based on a recent literature overview where most of the approaches use the same number of landmarks ([Kowalski et al. 2017], [Li et al. 2020]).

The landmarks are accessible in the file morgan_landmarks.txt.

Right Jaw Line				Left Jaw Line

Mouth					Nose

Left Eye 				Right Eye

Figure H.10: MPEG Morgan’s landmarks (global face and close views).

### H.3.5	Teeth Model

The facial rig also includes an upper and a lower jaw as illustrated in Figure H.11. The lower jaw (right) is composed of 10, 440 vertices and 10, 380 quad faces (20, 760 triangular faces). The upper jaw (left) is composed of 11, 107 vertices and 11, 040 quad faces (22, 080 triangular faces). They both come with their own UV coordinates and associated textures.

Here are the corresponding lines of the semantic Table H.1 to access the upper and lower jaws from the original glTF file.

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lower Jaw |  | (0.5, 0, 0.5) | 10440 | [12199:22638] | 10380 | 20760 | [12174:22553] |
| Upper Jaw |  | (0.5, 1, 0.5) | 11107 | [22639:33745] | 11040 | 22080 | [22554:33593] |

Upper Jaw			Lower Jaw

Upper UV Map		           Upper Albedo Map          	 Lower UV 		Lower Albedo Map

Figure H.11: MPEG Morgan’s teeth model.

### H.3.6	Eye Model

The reference avatar comes with a complete eye model depicted in Figure H.12. The eye model follows an anatomical eye model and is composed of three elements: the sclera, the iris, and the lens.

The polygon count is the following:

- lens (201 vertices and 200 quad face (380 triangular faces)

- iris (928 vertices and 896 quad face (1792 triangular faces)

- sclera (290 vertices and 288 quad face (576 triangular faces)

Here are the corresponding lines of the semantic Table H.1 to access the left and right eye from the original glTF file. All ranges of ID (vertex or face) are ordered in that following order: lens  iris  sclera.

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Eye Left |  | (0.5,0,0) | 1419 | [33746:35164] | 1384 | 2748 | [33594:34977] |
| Eye Right |  | (0.5,0.5,0) | 1419 | [35165:36583] | 1384 | 2748 | [34978:36361] |

Sclera				Iris			 	Lens

Figure H.12: MPEG Morgan’s eyes model.

The sclera corresponds to the eye globe and is displayed as the white area (with blood veins) of the eye. The iris is located inside the sclera and corresponds to the eye diaphragm and is displayed as the unique eye colored pattern of the eye. The lens is located inside the sclera and behind the iris. It corresponds to the lens that projects the light on the cornea (internal back border of the eye globe where the image is created). Here is how these three elements are organized.

The eye model comes with two blend shapes for each eye:

- Pupil_Dilatation – it controls the aperture of the pupil of the eye

- Sclera_Flatness – it controls the degree of flatness of the cornea

Figure H.13: Pupil Dilation from low (left) to high (right) and Cornea Flatness from low (left) to high (right).

## H.4	Interactive Format Description for Interactive Scenes

## H.4.1	Model Format

This section illustrates how to signal the type of avatar being used under the node extension “MPEG_node_avatar” in glTF.

## H.4.1.1	Processing Model and Semantics

The application will have the knowledge of what avatar to reconstruct or render given the “model” extension to the properties of “MPEG_node_avatar”. The default representation is the “MPEG_avatar_reference”, and this shall be fully supported under this extension.

Table H.4.1.1: Description of the proposed extension to signal the type of avatar in “MPEG_node_avatar”.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| model | array | O | N/A | Avatar model (see Table H.4.1.2). If present refers to the avatar format semantics. |

Table H.4.1-2 – Avatar model description.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | M | “” | User-defined name of the 3D object format. |
| semantical_type | string | M | “” | A string to indicate which type of avatar the format refers to, an example of different semantical types is defined in Table H.4.1.3. A combination of several types is possible e.g., “Humanoid/Aquatic” refers to a human-like avatar with swimming proprieties. |
| resolution | array | M | [“”] | An array of strings that semantically describes each alternative in the “MPEG_media.media” extension reference by the source element in terms of format resolution e.g., low-resolution, medium-resolution, or high-resolution. |
| source | integer | M | N/A | Index to a “MPEG_media.media” extension that defines external media. The “alternative” propriety will be index aligned with the resolution propriety to facilitate the application choice in terms of capability. |

Table H.4.1-3: The table presents the higher-level semantical representation depending on styles of avatars and format representation.  We can combine several items from this list or from other more detailed representations to facilitate the client application to parse the format of an avatar.

# Table H.4.1-3 Avatar Styles

Semantical type

Humanoid

Non-Humanoid

Aquatic

Aerial

Terrestrial

Subterranean

Arboreal

Other

The source attribute references a “MPEG_media.media” extension in ISO/IEC 23090-14 Scene description specification, with the intent to have the object “MPEG_media.media.alternative” defined to match the resolution. The “alternative” propriety contains the “uri” and “mimeType” that are used to reference any media input format.

## H.4.1.2	glTF Schema Examples

The following glTF example will instantiate a “MPEG_node_avatar” in clients that support this extension, and otherwise, fall back to a standard “MPEG_node” without the semantical avatar node extension.

"MPEG_media":[ { "alternatives":[ { "uri": "MPEG_avatar_reference_low.gltf" "mimeType": "mesh/gltf" }, { "uri": "MPEG_avatar_reference_medium.gltf" "mimeType": "mesh/gltf" }, { "uri": "MPEG_avatar_reference_high.gltf" "mimeType": "mesh/gltf" } ] } ] "nodes": [ { "extensions": { "MPEG_node_avatar": { "isAvatar": True, "model" : [{ "name": "MORGAN", "semantical_type": ["humanoid","terrestrial"], "resolution": ["low-resolution","medium-resolution","high-resolution"] "source": 0 }] } } } ]

## H.4.2	Metadata Format

This section presents an attribute to identify an avatar socially to facilitate interactions with other avatars or objects in the scene. This attribute contains the identity information of an avatar, such as name, gender, and age. The identity information is not necessarily the accurate information of a real user, but instead an identity attributed to the user’s avatar. The age attribute has additional significance in this proposal because it can be linked with parental permission and content access depending on the application side.

This attribute enables the creation of several avatar representations, which facilitates the description of an avatar in the scene and identifies the type of interactivity with the scene such an avatar should have.

In the scenario of sensibility to the metadata information, such as identity, mechanisms for data protection shall be considered in the future, and in the current description, we assume pseudo and non-real values not to infringe on user personal data protection systems.

Table H.4.2-1: The “metadata” propriety to the “MPEG_node_avatar”.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| metadata | object | O | “” | Information about the avatar allows social identification in virtual environments. Table H.4.2-2 provides the semantics of this object. |

Table H.4.2-2: Semantics of the proposed object extension "metadata".

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | M | “” | Name of the avatar |
| age | integer | M |  | Age of the avatar |
| gender | string | O | “” | Gender of the avatar |
| disability | array | O | [“”] | Avatar’s physical disability, to indicate any physical impairment, for example, brain injury, spinal cord injuries, amputations, musculoskeletal, hearing loss or visual impairment. |
| capability | array | O | [“”] | A list of avatar abilities, such as the possibility to fly, walk, grasp, jump, and others. |
| personality | array | O | [“”] | A list of characteristics or qualities for an avatar individual character, for example, shy, courageous. As another example, the Myers-Briggs type can also be used. |
| emotion | array | O | [“neutral”] | An emotional state that reflects an avatar state, for example, admiration, anger, fear, sadness, joy, and others. Different lists of emotions categories are possible, such as Robert Plutchik’s categories. |
| social | array | O | [“”] | A type of social behavior that the avatar is able to perform, such as conversation or interaction with other 3D environment elements and avatars. |
| parental | array | O | [“”] | Permission content to be accessed by the avatar. This can be any standard method to infer parental permissions such as the ones used in video games and the streaming industry. |

## H.4.2.1	glTF Schema Examples

The extension “MPEG_node_avatar.metadata” specifies by default the avatar attributes that define a base representation of a user avatar preference. This extension to the proprieties of “MPEG_node_avatar” allow the possibility to identify and generate user-specific avatars.

The following glTF will instantiate a “MPEG_node_avatar” in clients that support this extension, and otherwise, fall back to a standard “MPEG_node_avatar” without the metadata semantical extension.

"nodes": [ { "extensions": { "MPEG_node_avatar": { "isAvatar": True, "metadata" : [{ "name": "Red", "aga": 18, "gender": "N/A", "disability": ["colorblind", "hearing_loss"], "capability": ["walk", "run", "grasp"], "personality": ["happy"], "emotion": ["admiration", "joy"], "social": ["interaction", "conversation"], "parental": ["18", "bad_language", "violence", "fear"], }] } } } ]

## H.4.3	Interactive Actions Format

We present an extension that allows glTF models to use and interact with the reference avatar and any other object. We extend the glTF scene element “MPEG_scene_interactivity” action properties to define “ACTION_SET_AVATAR” which contains more avatar-related actions.

The semantics of the new proposed actions are provided below and the MPEG reference Avatar actions list is identified by “urn:mpeg:sd:2023:avatar:actionslist”.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | string | M | “” | One element of Table H.4.3-1 defines the extended type of social action. |
| if (type == “Action_ AVATAR_Social”){ |  |  |  |  |
| authorised | array | M |  | One or more elements of Table H.4.3-2 define the type of social actions. |
| nodes | array | M |  | Indices of the nodes in the array to apply “Social parameters” (Table H.4.3-3) listed on the “authorized” field. |
| } |  |  |  |  |
| if (type == “Action_ AVATAR_Restricted”){ |  |  |  |  |
| permission_id | string | M |  | Unique string identifier that restricts interaction between nodes without an equal permission_id. |
| nodes | array | M |  | Indices of the nodes in the array to apply “Restricted parameters”, i.e., the nodes whose permission IDs are to be checked/applied. |
| } |  |  |  |  |
| if (type == “Action_ AVATAR_Parental”){ |  |  |  |  |
| age | number | M |  | One element of Table H.4.3-4 that define the minimum age recommendation for users given the content of the list of nodes. |
| descriptors | Array | M |  | One or more elements of Table H.4.3-5 that add additional explicit semantics of the content present in the list of nodes. |
| nodes | array | M |  | Indices of the nodes to apply “Parental parameters”. |
| } |  |  |  |  |
| If(type == “Action_ AVATAR_Speech”){ |  |  |  |  |
| microphone | Boolean | O | 0 | Indicates if the user uses a microphone type of input device for audio. “0” is False and “1” is True. The default value is 0. |
| media | String | O | “” | URI (Uniform Resource Identifier) to media track to play a pre-recorder audio file. |
| nodes | array | M |  | Indices of the nodes in the array to allow “Speech” media. |
| } |  |  |  |  |
| If(type == “Action_ AVATAR_Capabilities”){ |  |  |  |  |
| capabilities | array | M | [] | One or more elements of Table H.4.3-6 that define the capabilities of an avatar/object. |
| nodes | array | M | [] | Indices of the nodes in the array to apply “capabilities parameters”. |
| } |  |  |  |  |
| If(type == “Action_ AVATAR_Disabilities”){ |  |  |  |  |
| disabilities | array | M | [] | One or more elements of Table H.4.3-7 that define the disabilities of an avatar/object. |
| nodes | array | M |  | Indices of the nodes to apply “Disability parameters”. |
| } |  |  |  |  |

Table H.4.3.1: Semantical description of new action properties.

| Action type | Description |
| --- | --- |
| “Action_ AVATARSocial” | Set action of a node. |
| “Action_ AVATAR_Restricted” | Set permissions of a node. |
| “Action_ AVATAR_Parental” | Set parental and content usage permissions of a node. |
| “Action_ AVATAR_Speech” | Set speech active of a node. |
| “Action_ AVATAR_Capabilities” | Set capabilities of a node. |
| “Action_ AVATAR_Disabilities” | Set disabilities of a node. |

Table H.4.3.2: Type of avatar action.

| Social action list | Description |
| --- | --- |
| “avatar_social_conversation” | Allow social speaking with users and enable the ability for “Action_AVATAR _Speech” if not already enabled. |
| “avatar_social_interaction” | Allow interaction between users. |
| “avatar_social_other” | Any other type of social interaction. |

Table H.4.3-3: Type of avatar social actions.

| “age” list | Description |
| --- | --- |
| 3 | Content suitable for all ages. |
| 7 | Content with scenes or sounds possibly frightening to younger children. |
| 12 | Content with violence of graphic non-realistic characters. |
| 16 | Content with violence that mimics the reality. |
| 18 | Content design for adults only. |

Table H.4.3-4: Type of avatar age levels.

| parental descriptors list | Description |
| --- | --- |
| “violence” | Contains depiction of violence. |
| “bad_language” | Contains bad language. |
| “fear” | Contains pictures or sounds that may be frightening or scary. |
| “gambling” | Contains elements that encourage or teach gambling. |
| “sex” | Contains sexual posturing. |
| “drugs” | Contains the illustration of the use of illegal drugs, alcohol or tobacco. |
| “discrimination” | The game contains depictions of ethnic, religious, nationalistic or other stereotypes likely to encourage hatred |
| “in-game_purchases” | Offers the option of purchase digital services. |

Table H.4.3-5: Type of avatar parental descriptors.

| Capabilities list | Description |
| --- | --- |
| “walk” | The ability to walk. |
| “run” | The ability to run. |
| “jump” | The ability to jump. |
| “fly” | The ability to fly. |
| “swim” | The ability to swim. |
| “climb” | The ability to go over, get on, climb to, to descend objects, such as climb to a chair, climb up the stairs, climb down the stairs, climb the wall etc. |
| “grasp” | The ability to hold objects with hands’ like representations. |
| “manipulate” | The ability to interact and change spatial position of 3D objects by using collision or proximity type of detectors. |
| “ride” | The ability to ride a vehicle or animal, such as motorcycles or horses etc. |
| “drive” | The ability to use a vehicle, such as cars or trucks etc. |
| “pilot” | The ability of piloting a vehicle, such as ships or airplanes. |

Table H.4.3.6: Capabilities semantics.

| Disabilities list | Description |
| --- | --- |
| “Cerebral palsy” | A group of disorders that impact a person's ability to move and maintain balance |
| “Spinal cord injuries” | Spina cord injury indicates the damages to any part of the spinal cord or nerves at the end of the spinal canal. Result in permanent loss of strength, sensation, and function (mobility and feeling) |
| “Amputation” | Indicates removal of part of all of a body part that is enclosed by skin |
| “Musculoskeletal injuries” | Refer to the damage of muscular or skeletal systems, which is usually due to strenuous activities. |

Table H.4.3.7: Disabilities semantics.