# ISO/IEC JTC 1/SC 29/WG 03 N01106

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Input Contribution

Title:	Revised text of ISO/IEC 23090-14 DAM 2: Support for Haptics, Augmented Reality, Avatars, Interactivity, MPEG-I Audio, and Lighting

Date of document:	2024-04-16

Source:	WG03

No. of pages:	92 (with cover page)

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

In clause 2 on normative references, add the following references

Khronos, EXT_lights_image_based, https://github.com/KhronosGroup/glTF/tree/main/extensions/2.0/Vendor/EXT_lights_image_based

Khronos, The OpenXR Specification
https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html

IETF RFC 7946, The GeoJSON Format
RFC 7946: The GeoJSON Format (rfc-editor.org)

========================== Next Change ===========================

In section 3.1 add the following definitions:

3.1.15
Augmented Reality (AR)

technique of composing a view from virtual objects and the user’s real-world environment

3.1.16
trackable

real-world object that can be tracked by the XR runtime

3.1.17
AR anchor

real-world pose in the trackable space

3.1.18

avatar

virtual representation of a user

3.1.19

T-pose

Default body pose of a 3D model’s skeleton

========================== Next Change ===========================

In section 3.2 add the following abbreviations:

TRS	translation, rotation and scale

========================== Next Change ===========================

Update Figure 3 with the attached figure.

In section 5.1.1, replace Figure 3 by the following figure:

Figure 3 — An overview of the glTF document structure with MPEG extensions defined in this document

========================== Next Change ===========================

Replace table 4 by the following table:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | O | N/A | The user-defined name of the media. |
| startTime | number | O | N/A | The startTime gives the time at which the rendering of the timed media will begin. The value is provided in seconds. In the case of timed textures, the static image should be rendered as a texture until the startTime is reached. A startTime of 0 means the presentation time of the current scene. |
| startTimeOffset | number | O | 0 | The startTimeOffset indicates the time offset into the source, starting from which the timed media shall be generated. The value is provided in seconds, where 0 corresponds to the start of the source. |
| endTimeOffset | number | O | N/A | The endTimeOffset indicates the end time offset into the source, up to which the timed media shall be generated. The value is provided in seconds. If not present, the endTimeOffset corresponds to the end of the source media. |
| autoplay | boolean | O | False | When present and set to True, it specifies that the media will start playing as soon as it is ready. When present and set to True, startTime shall not be present or shall be ignored by the client. Rendering of all media for which the autoplay flag is set to True should happen simultaneously. When set to False, it indicates that the media may be controlled by the functions in Scene Description such as interactivity. |
| autoplayGroup | integer | O | N/A | All media that have the same autoplayGroup identifier shall start playing synchronously as soon as all autoplayGroup media are ready. autoplayGroup is only allowed if autoplay is set to True. |
| loop | boolean | O | False | Specifies that the media will start over again, every time it is finished. The timestamp in the buffer shall be continuously increasing when the media source loops, i.e. the playback duration prior to looping shall be added to the media time after looping. |
| controls | boolean | O | False | Specifies that media controls should be displayed (such as a play/pause button etc). |
| alternatives | array | M |  | An array of items that indicate alternatives of the same media (e.g. different video codecs used) NOTE The client can select items (i.e. U and track) included in alternatives depending on the client’s capability. |

========================== Next Change ===========================

Replace section 5.4.1.1 by the following text:

#### 5.4.1.1	General

The MPEG audio extension adds support for spatial audio.

When present, the MPEG_audio_spatial extension shall be included as extension of a camera object, or a scene object defined in ISO/IEC 12113.

When MPEG_audio_spatial extension is included as an extension for a node object, either MPEG_audio_spatial.source or MPEG_audio_spatial.listener properties of the extension shall be present. When MPEG_audio_spatial extension is included as an extension at a glTF file level extension, MPEG_audio_spatial.reverb property shall be present.

The MPEG_audio_spatial extension supports three different object types:

source: an audio source that provides input audio data into the scene. Mono objects and HOA sources (as defined in ISO/IEC 23008-3:2022, Annex F.1) are supported in this version of the document.

  - Type: 'Object' or 'HOA'

  - HOA audio sources shall ignore the parent node's position and be rendered only in 3DoF.

- Moreover, an audio source may be a cluster audio source, which denotes a pre-mixed representation of a selection of audio sources. To indicate a cluster audio source, the isCluster flag should be set equal to True.

reverb: A reverb effect can be attached to the output of an audio source. Several reverb units can exist and sound sources can feed into one or more of these reverb units. An audio renderer that does not support reverb shall ignore it if the bypass attribute is set to true. If the bypass attribute is set to false, the audio renderer shall return an error message

listener: An audio listener represents the output of audio in the scene. A listener should be attached to a node in the scene. It is recommended to have the audio listener in a node that is a child node of the camera. By being a child node of the camera, additional transformations can be applied to the audio listener relative to the transformation applied to the parent camera.

Figure 5 depicts the processing chain for audio in a scene.

Figure 5 — An example of the processing chain for audio in a scene

The specification of any audio effect processing is outside the scope of this document.

The characteristics of a listener depend on the actual output devices available to the audio renderer.

========================== Next Change ===========================

Replace caption of Table 13 as follows:

Table 13 — Definition of objects of MPEG_audio_spatial extension

========================== Next Change ===========================

In table 14, change

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| id | integer | M |  | unique identifier of the audio source in the scene. |
| type | string | M |  | Indicates the type of the audio source. type value equal to “Object” indicates mono object type value equal to “HOA” indicates HOA object |
| pregain | number | O | 0.0 | provides a level-adjustment in dB for the signal associated with the source. |
| playbackSpeed | number | O | 1.0 | defines the playback speed of the audio signal. A value of 1.0 corresponds to playback at normal speed. The value shall be between 0.5 and 2.0. |
| attenuation | enumeration | O | “linearDistance” | indicates the function used to calculate the attenuation of the audio signal based on the distance to the source. attenuation value equal to "“noAttenuation" indicates no attenuation function should be used. attenuation value equal to "inverseDistance" indicates inverse distance function should be used. attenuation value equal to "linearDistance" indicates linear distance function should be used. attenuation value equal to "exponentialDistance" indicates exponential distance function should be used. attenuation value equal to "custom" indicates custom function should be used. The definition of custom function is outside of the scope of this document. The attenuation functions and their parameters are specified in Annex D. |
| attenuationParameters | array | O | N/A | array of parameters that are input to the attenuation function. The semantics of these parameters depend on the attenuation function itself. |
| referenceDistance | number | O | 1.0 | provides the distance in meters for which the distance gain is implicitly included in the source signal after application of pregain. When type equals ‘HOA’ the element shall not be present. |
| accessors | array | M |  | provides an array of accessor references, by specifying the accessors indices in accessors array, that describe the buffers where the decoded audio will be made available. |
| reverbFeed | array | O | N/A | provides one or more pointers to reverb units, optionally extended by a floating point scaling factor. A reverb unit represents a reverberation audio processor that is configured by the metadata from a single reverb object. Typically, a reverb object represents reverberation properties of a single room. |
| reverbFeedGain | array | O | N/A | provides an array of gain [dB] values to be applied to the source’s signal(s) when feeding it to the corresponding reverbFeed. The array shall have the same number of elements as the reverbFeed array field. |

to

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| id | integer | M |  | Unique identifier of the audio source in the scene. |
| type | string | M |  | Indicates the type of the audio source. The value “Object” indicates mono object The value “HOA” indicates HOA object |
| targetSampleRate | number | M |  | Provides the target audio sampling rate that is expected to be supplied by the media pipeline of the corresponding audio source. If the sampling rate of one of the selected media source alternatives differs from this value, then the media pipeline shall perform resampling to match the target sample rate. |
| pregain | number | O | 0.0 | Provides a level-adjustment in dB for the signal associated with the source. |
| playbackSpeed | number | O | 1.0 | Defines the playback speed of the audio signal. A value of 1.0 corresponds to playback at normal speed. The value shall be between 0.5 and 2.0. |
| attenuation | enumeration | O | “linearDistance” | Indicates the function used to calculate the attenuation of the audio signal based on the distance to the source. The value "noAttenuation" indicates that no attenuation function should be used. The value "inverseDistance" indicates that the inverse distance function should be used. The value "linearDistance" indicates that the linear distance function should be used. The value "exponentialDistance" indicates that the exponential distance function should be used. The value "custom" indicates that a custom function should be used. The definition of custom functions is outside of the scope of this document. The attenuation functions and their parameters are specified in Annex D. |
| attenuationParameters | array | O | N/A | Array of parameters that are input to the attenuation function. The semantics of these parameters depend on the attenuation function itself. |
| referenceDistance | number | O | 1.0 | Provides the distance in meters for which the distance gain is implicitly included in the source signal after application of pregain. When type equals ‘HOA’, the element shall not be present. |
| accessors | array | M |  | Provides an array of accessor references, by specifying the accessors indices in accessors array, that describe the buffers where the decoded audio will be made available. |
| reverbFeed | array | O | N/A | Provides one or more pointers to reverb units, optionally extended by a floating point scaling factor. A reverb unit represents a reverberation audio processor that is configured by the metadata from a single reverb object. Typically, a reverb object represents reverberation properties of a single room. |
| reverbFeedGain | array | O | N/A | Provides an array of gain [dB] values to be applied to the source’s signal(s) when feeding it to the corresponding reverbFeed. The array shall have the same number of elements as the reverbFeed array field. |
| isCluster | boolean | False | O | Specifies if the audio source is a pre-mixed representation of a selection of audio sources. |
| clusterProperties | object | N/A | O | A sourceClusterProperties object that contains cluster properties. This object must be defined, when the isCluster attribute is set to True. |

========================== Next Change ===========================

In section 5.4.1.2, change the first sentence and add Table 18 with the following content

The definition of all objects within MPEG_audio_spatial extension is provided in Tables 13 to 18.

Table 18 – Definition of sourceClusterProperties object of MPEG_audio_spatial extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| sourceId | array | M |  | Array of integers that contains the unique identifiers of the audio sources contained in this cluster. |
| radius | number | O | 0.0 | Distance in meters used to encompass the audio sources contained in this cluster. |

========================== Next Change ===========================

In section 6.2, change

enum AttributeType {"ATTRIB_NORMAL","ATTRIB_POSITION","ATTRIB_COLOR", "ATTRIB_TEXCOORD","ATTRIB_INDEX","ATTRIB_TANGENT", "ATTRIB_WEIGHTS", "ATTRIB_JOINTS"}

to

enum AttributeType {"ATTRIB_NORMAL","ATTRIB_POSITION","ATTRIB_COLOR", "ATTRIB_TEXCOORD","ATTRIB_INDEX","ATTRIB_TANGENT", "ATTRIB_WEIGHTS", "ATTRIB_JOINTS", "ATTRIB_PCM", "ATTRIB_METADATA"}

========================== Next Change ===========================

In section 6.2, change

interface BufferInfo {
	attribute integer bufferId;
	attribute BufferHandle handle;
	attribute unsigned long componentType;
	attribute SampleType type;
	attribute integer offset;
	attribute integer stride;
	attribute AttributeType attributeType;
};

to

interface BufferInfo {

typedef enum ColorFormat {RED, GREEN, BLUE, RG, RGB, RGBA, BGR, BGRA, DEPTH_COMPONENT};
     typedef struct OutputMedia {

unsigned long width;
       unsigned long height;
       ColorFormat format;
       unsigned long sampleRate;

}
	attribute integer bufferId;
	attribute BufferHandle handle;
	attribute unsigned long componentType;
	attribute SampleType type;
	attribute integer offset;
	attribute integer stride;
	attribute AttributeType attributeType;
     attribute OutputMedia outputMedia;
};

========================== Next Change ===========================

In section 6.2, change the last paragraph from

The MAF may use the ViewInfo to optimize the streaming of the requested media, e.g. by adjusting the level of detail (number of polygons/points, texture resolution etc.) based on the distance to and orientation of the viewer. The BufferInfo contains information about each Buffer and describes the format of the samples and frames that are stored in that buffer. One or more tracks from the MediaInfo may feed into the same buffer. The link between the track that provides the actual media and the buffer that will store the output of the media pipeline is established through the bufferId attribute.

to

The MAF may use the ViewInfo to optimize the streaming of the requested media, e.g. by adjusting the level of detail (number of polygons/points, texture resolution etc.) .), or by choosing a cluster audio source (i.e., a pre-mixed representation of a selection of audio sources) instead of the audio sources contained in this cluster individually, based on the distance to and/or the orientation of the viewer. The BufferInfo contains information about each Buffer and describes the format of the samples and frames that are stored in that buffer. One or more tracks from the MediaInfo may feed into the same buffer. The link between the track that provides the actual media and the buffer that will store the output of the media pipeline is established through the bufferId attribute.

========================== Next Change ===========================

In table 21, change

| BufferInfo | The BufferInfo provides information about the expected format of the output buffer of the media pipeline. The buffer may be allocated by the MAF or by the Presentation Engine. The handler is used to read or pass a reference to an allocated buffer. If allocated by the MAF, then only read access shall be allowed. If the pipeline is initialized without a valid buffer handler, then the MAF shall allocate the buffer. the buffer format information in componentType, type, offset, and stride shall correspond to componentType, type, offset, stride in the corresponding accessor and bufferView. The attributeType shall correspond to the corresponding primitive attribute. The BufferHandle holds a handle for a buffer, which can be used to access the buffer. |
| --- | --- |

to

| BufferInfo | The BufferInfo provides information about the expected format of the output buffer of the media pipeline. The buffer may be allocated by the MAF or by the Presentation Engine. The handler is used to read or pass a reference to an allocated buffer. If allocated by the MAF, then only read access shall be allowed. If the pipeline is initialized without a valid buffer handler, then the MAF shall allocate the buffer. the buffer format information in componentType, type, offset, and stride shall correspond to componentType, type, offset, stride in the corresponding accessor and bufferView. The attributeType shall correspond to the corresponding primitive attribute. ATTRIB_PCM indicates PCM audio data. ATTRIB_METADATA indicates the presence of binary metadata, of which the format is dependent on the referencing component/extension. The BufferHandle holds a handle for a buffer, which can be used to access the buffer. |
| --- | --- |

========================== Next Change ===========================

In section 6.3, change

interface Frame {
	attribute integer index;
	attribute double timestamp;
	attribute integer length;
	attribute octet[length] data;
};

to

interface Frame {

typedef struct ExtraParameters {
                  unsigned long width;
                  unsigned long height; 
            }
	attribute unsigned long index;
	attribute unsigned long long timestamp;
	attribute unsigned long length;
	attribute octet[length] data;
           attribute ExtraParameters extraFrameInfo;
};

========================== Next Change ===========================

In section 6.3, replace

interface CircularBuffer {
	readonly attribute Frame frames[];
	readonly attribute count;
	readonly attribute integer read_idx;
	readonly attributre integer write_idx;
	attribute integer headerLength;
	attribute EventHandler   onframewrite;
	attribute EventHandler   onframeread;
	void allocate(int count);
	void writeFrame(Frame frame);
	Frame readFrame(optional double timestamp);
	void free();
};

with

interface CircularBuffer {
	readonly attribute Frame frames[];
	readonly attribute count;
	readonly attribute integer read_idx;
	readonly attributre integer write_idx;
	attribute integer headerLength;
	attribute EventHandler   onframewrite;
	attribute EventHandler   onframeread;
	void allocate(int count);
	void writeFrame(Frame frame);
	Frame readFrame(optional double timestamp);
     void releaseFrame(int index);
	void free();
};

========================== Next Change ===========================

In section 6.3, replace table  22 with

| Method | Description |
| --- | --- |
| allocate() | Allocates a buffer for the data exchange between the MAF and the Presentation Engine. |
| writeFrame() | writes a frame to the buffer. The write pointer moves to the next frame. |
| readFrame() | reads a frame from the buffer. If no timestamp is provided, the buffer pointer moves to the next frame. Otherwise, the read pointer remains unchanged. If the buffer is empty, an error is returned. Prior frames may still be accessed using their timestamp as long as they are not overwritten. |
| releaseFrame() | Informs the buffer management that the data in the buffer frame at the specified index may be overwritten by the write process. |
| free() | Destroys the buffers and frees any resources associated with it. |

========================== Next Change ===========================

In table 23, change

| headerLength | This provides the length in bytes of the buffer frame header that is available at the start of every frame in the buffer as specified in Table 27. A headerLength of 0 indicates that there is no buffer header. When the buffer API is used, the buffer frame header shall not be present as part of the buffer frame data. For implementations that do not use the Buffer API, the buffer frame header information may be provided as part of the buffer frame data. |
| --- | --- |

to

| headerLength | This provides the length in bytes of the buffer frame header that is available at the start of every frame in the buffer as specified in Table 24. A headerLength of 0 indicates that there is no buffer header. When the buffer API is used, the buffer frame header shall not be present as part of the buffer frame data. For implementations that do not use the Buffer API, the buffer frame header information may be provided as part of the buffer frame data. |
| --- | --- |

========================== Next Change ===========================

Add clause 8 with the following content

- Advanced Features

## 8.1 	AR Anchoring

### 8.1.1	MPEG_anchor  extension

AR anchoring allows for the anchoring of the scene or some root nodes into the user’s physical environment to provide an Augmented Reality experience. AR anchoring is supported at the scene level and at the node level, as well as a  extension through the definition of the MPEG_anchor extension.

Multiple AR anchoring is then supported by adding the MPEG_anchor extension to several root-levelnodes.

### 8.1.2	Semantics

The MPEG_anchor extension consists of Trackable and Anchor objects as provided in Table 26. The definition of Trackable and Anchor objects shall be provided within an MPEG_anchor extension at the glTF file level.

Table 1 – MPEG_anchor extension provided at the glTF file level

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| trackables | array(Trackable) | M |  | Provides a list of trackables used by the anchor objects. |
| anchors | array(Anchor) | M |  | Provides a list of anchors for a scene or for root nodes to enable AR anchoring |

The syntax and semantics of the Anchor object is provided in Table 2.

Table 2 – Definition of the Anchor object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| trackable | integer | M |  | Index of the trackable in the trackables array that will be used for this anchor. |
| requiresAnchoring | boolean | M |  | Indicates if AR anchoring is required for the rendering of the associated nodes. If TRUE, the application shall skip the virtual assets attached to this anchor until the pose of this anchor in the real world is known. if FALSE, the application shall process the virtual assets attached to this anchor |
| minimumRequiredSpace | vec3 | O | (0,0,0) | Space required to anchor the AR asset (x, y, z in meters). This space corresponds to an axis-aligned bounding box, placed at the origin of the trackable space and extending along the positive x+,y+, and z+ axes, expressed in the trackable local space. This value shall be compared to the bounding box of the real-world available space associated with the trackable as estimated by the XR runtime. |
| aligned | enumeration | O | NOT_USED | The aligned flag may take one of the following values: NOT_USED=0, ALIGNED_NOTSCALED=1, ALIGNED_SCALED=2. If ALIGNED_SCALED is set, the bounding box of the virtual assets attached to that anchor is aligned and scaled to match the bounding box of the real world available space associated with the trackable as estimated by the XR runtime. |
| actions | array(number) | O | N/A | Indices of the actions in the actions array of the interactivity extension to be executed once the pose of this anchor is determined. An example is a setTransform action to place the virtual assets attached to that anchor. |
| light | integer | O | N/A | Reference to an item in the lights array of the MPEG_lights_texture_based extension. |

The definition of the Trackable object is provided in the Table 3.

Table 3: Definition of the Trackable object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | The type of the trackable as defined in Table 4. |
| if (type == TRACKABLE_CONTROLLER) { |  |  |  |  |
| path | string | M |  | A path that describes the action space as specified by the OpenXR specification in clause 6.2. An example is “/user/hand/left/input”. |
| } |  |  |  |  |
| if (type== TRACKABLE_GEOMETRIC) { |  |  |  |  |
| geometricConstraint | enumeration | M |  | The geometricConstraint flag may take one of the following values: HORIZONTAL_PLANE=0, VERTICAL_PLANE=1 |
| } |  |  |  |  |
| if (type== TRACKABLE_MARKER_2D OR MARKER_3D) { |  |  |  |  |
| markerNode | number | M |  | Index to the node in the nodes array in which the marker geometry and texture are described. |
| } |  |  |  |  |
| if (type== TRACKABLE_MARKER_GEO) { |  |  |  |  |
| coordinates | array | M |  | Array of three float numbers giving the longitude, the latitude, and the elevation of the geolocation of the center, that shall be as specified in [GeoJSON] for the point geometry type as follows: - the longitude in units of decimal degrees. The value is in range of −180.0 to 180.0, inclusive. Positive values represent eastern longitude and negative values represent western longitude. - the latitude in units of decimal degrees. The value is in range of −90.0 to 90.0, inclusive. Positive value represents northern latitude and negative value represents southern latitude. - the elevation is in units of meters above the WGS 84 reference ellipsoid. |
| } |  |  |  |  |
| if (type== TRACKABLE_APPLICATION) { |  |  |  |  |
| trackableId | string | M |  | An application-defined trackable id, that is known to the application. |
| } |  |  |  |  |

Table 4: Definition of the Trackable type

| Trackable type | Description |
| --- | --- |
| TRACKABLE_FLOOR = 0 | See 8.1.2.1.1 |
| TRACKABLE_VIEWER = 1 | See 8.1.2.1.2 |
| TRACKABLE_CONTROLLER = 2 | See 8.1.2.1.3 |
| TRACKABLE_PLANE = 3 | See 8.1.2.1.4 |
| TRACKABLE_MARKER_2D = 4 | See 8.1.2.1.5 |
| TRACKABLE_MARKER_3D = 5 | See 8.1.2.1.6 |
| TRACKABLE_MARKER_GEO = 6 | See 8.1.2.1.7 |
| TRACKABLE_APPLICATION = 7 | See 8.1.2.1.8 |

Table 5: MPEG_anchor object instantiation at the node or scene level

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| anchor | integer | M |  | Reference to an item in the anchors array of the MPEG_anchor extension. |

#### 8.1.2.1	Trackable Definitions

##### 8.1.2.1.1 TRACKABLE_FLOOR

A trackable of type TRACKABLE_FLOOR is an anchor with a plane that spans the xz-plane in the anchor’s local coordinate system. The origin of the local coordinate system is located at the center of the detected flat rectangular surface considered to be on the floor of the scene. The y-axis of the anchor’s local coordinate system is the plane’s normal vector and the X and Z axes are aligned with the rectangle edges.

NOTE	This corresponds to the XR_REFERENCE_SPACE_TYPE_STAGE defined in OpenXR in 7.1. Reference Spaces.

##### 8.1.2.1.2 TRACKABLE_VIEWER

A trackable of type TRACKABLE_VIEWER is a trackable that corresponds to the viewer’s pose.

It tracks the primary viewer with the center at the viewer’s position, with +Y up, +X to the right, and -Z forward. This trackable is viewer head locked.

NOTE	This corresponds to the XR_REFERENCE_SPACE_TYPE_VIEW defined in OpenXR in 7.1. Reference Spaces.

##### 8.1.2.1.3 TRACKABLE_CONTROLLER

A trackable of type TRACKABLE_CONTROLLER is a trackable that corresponds to one of the active controllers. Three controller types are defined: grip, aim, and palm.

A grip pose allows applications to reliably render a virtual object held in the user’s hand, whether it is tracked directly or by a motion controller. The grip pose is defined as follows:

- The grip position:

  - For tracked hands: The user’s palm centroid when closing the fist, at the surface of the palm.

  - For handheld motion controllers: A fixed position within the controller that generally lines up with the palm centroid when held by a hand in a neutral position. This position should be adjusted left or right to center the position within the controller’s grip.

- The grip orientation

  - +X axis: When you completely open your hand to form a flat 5-finger pose, the ray that is normal to the user’s palm (away from the palm in the left hand, into the palm in the right hand).

  - -Z axis: When you close your hand partially (as if holding the controller), the ray that goes through the center of the tube formed by your non-thumb fingers, in the direction of little finger to thumb.

  - +Y axis: orthogonal to +Z and +X using the right-hand rule.

An aim pose allows applications to point in the world using the input source, according to the platform’s conventions for aiming with that kind of source. The aim pose is defined as follows:

- For tracked hands: The ray that follows platform conventions for how the user aims at objects in the world with their entire hand, with +Y up, +X to the right, and -Z forward. The ray chosen will be runtime-dependent, for example, a ray emerging from the palm parallel to the forearm.

- For handheld motion controllers: The ray that follows platform conventions for how the user targets objects in the world with the motion controller, with +Y up, +X to the right, and -Z forward. This is usually for applications that are rendering a model matching the physical controller, as an application rendering a virtual object in the user’s hand likely prefers to point based on the geometry of that virtual object. The ray chosen will be runtime-dependent, although this will often emerge from the frontmost tip of a motion controller.

A palm pose allows applications to reliably anchor visual content relative to the user’s physical hand, whether the user’s hand is tracked directly or its position and orientation is inferred by a physical controller. The palm pose is defined as follows:

- The palm position: The user’s physical palm centroid, at the surface of the palm.

- The palm orientation:

  - +X axis: When a user is holding the controller and straightens their index finger, the ray that is normal to the user’s palm (away from the palm in the left hand, into the palm in the right hand)

  - -Z axis: When a user is holding the controller and straightens their index finger, the ray that is parallel to their finger’s pointing direction.

  - +Y axis: orthogonal to +Z and +X using the right-hand rule.

NOTE	This corresponds to the Standard pose identifiers defined in OpenXR in 6.3.2. Input subpaths.

##### 8.1.2.1.4 TRACKABLE_PLANE

The width and length of a plane span the xz-plane of the anchor instance's local coordinate system. The origin of the local coordinate system is located at the center of the detected flat rectangular surface. The y-axis of the plane anchor is the plane’s normal vector, and the X and Z axes are aligned with the rectangle edges.

##### 8.1.2.1.5 TRACKABLE_MARKER_2D

The width and length of the marker 2D span the xz-plane of the anchor instance's local coordinate system. The origin of the local coordinate system is located at the center of the detected marker 2D surface. The y-axis of the anchor is the plane’s normal vector, and the X and Z axes are aligned with the rectangle edges.

##### 8.1.2.1.6 TRACKABLE_MARKER_3D

For 3D models, the origin is the center of the mesh. The X, Y, and Z axes correspond to the axes of the world space.

##### 8.1.2.1.7 TRACKABLE_MARKER_GEO

The y-axis matches the direction of gravity as detected by the device's motion sensing hardware, y points downward.

The x- and z-axes match the longitude and latitude directions. -Z points to true north -X points west.

##### 8.1.2.1.8 TRACKABLE_APPLICATION

The application-defined trackable object must have a right-handed coordinate space.

### 8.1.3	Processing model

Each trackable provides a local reference space in which an anchor pose can be expressed. This local reference space is right-handed and depends on the type of trackable as described below.

The  MPEG_anchor extension shall not be present at scene and node level at the same time in the MPEG-I Scene Description. Upon activation of the XR mode, the Presentation Engine identifies the anchor points associated with the scene or with the root nodes and the related trackables.

If the requiresAnchoring Boolean parameter of an anchor is set to TRUE, the application shall skip the virtual assets attached to this anchor until the pose of this anchor in the real world is known. Otherwise, the application shall process the virtual assets attached to this anchor.

Upon the detection of a trackable within the user’s physical environment, the application creates an XR space and possibly determines the bounding box of the real world available space in which the virtual assets can be added.

If the optional minimumRequiredSpace parameter of the anchor referencing the trackable is present, the application shall skip the virtual assets attached to that anchor if the estimated real world available space is smaller in volume than the minimumRequiredSpace.

Otherwise, the application shall start the tracking, shall launch the actions provided in the actions parameter of the anchor and shall apply the necessary spatial transformations to the virtual assets with respect to the local space of the anchor prior to rendering.

The application could update at runtime a tracking status of each trackable defined in the trackables array at the scene level, for instance as follows:

- A tracking status set to TRUE when the trackable is detected and when its pose is well identified.

- A tracking status set to FALSE when the trackable position is lost (e.g., no more detected).

To provide the flexibility to define an anchor space different than the trackable’s space,

- The TRS of the node having the MPEG_anchor extension defines the relative transformation between the trackable’s space and the anchor space in which the child nodes containing the virtual assets are placed.

- A setTransform action may be defined to affect the scene root nodes in the case of a MPEG_anchor extension. This transformation defines the TRS between the trackable’s space and the anchor space in which all the scene root nodes are placed.

Actions are defined in MPEG_scene_interactivity extension.

If the array of action is not empty, the actions are executed once the pose of anchor is determined. If the tracking status is set to FALSE after being TRUE, actions are not canceled.

## 8.2 	Interactivity

### 8.2.1	General

Interactivity is supported at the scene level and at the node level through the definition of two extensions MPEG_scene_interactivity and MPEG_node_interactivity.

When present, the MPEG_scene_interactivity extension shall be included as extension to the scene object.

When present, the MPEG_node_interactivity extension shall be included as extension to node object.

The MPEG_node_interactivity extension is used to complement the interactivity defined at the scene level. One particular case is the definition of the parameters for the physics engine. That is, when an MPEG_node_interactivity extension contains a trigger of type TRIGGER_COLLISION without being referenced by a trigger of type TRIGGER_COLLISION at the MPEG_scene_interactivity extension, this node shall not be considered for collision detection and instead only be used by the physics engine.

### Semantics

#### 8.2.2.1	Semantics at scene level

The semantic of the MPEG_scene_interactivity extension is based on the definition of trigger, action and behavior objects as shown in Table 6.

Table 6: Semantic of the MPEG_scene_interactivity extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M |  | Contains the definition of all the triggers used in that scene. |
| actions | array | M |  | Contains the definition of all the actions used in that scene. |
| behaviors | array | M |  | Contains the definition of all the behaviors used in that scene. A behavior is composed of a pair of (triggers, actions), control parameters of triggers and actions, a priority weight and an optional interrupt action. |

The semantic of a trigger is provided in Table 7.

Table 7: Semantic of a trigger

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | One element of Table 8 that defines the type of the trigger. |
| if (type == TRIGGER_COLLISION){ |  |  |  |  |
| nodes | array | M |  | Indices of the nodes in the nodes array to be considered for collision determination. Any detection of collision shall activate the trigger |
| primitives | array(Primitive) | O | N/A | List of primitives used to activate the proximity or collision trigger. Semantics are presented in Table 9. |
| } |  |  |  |  |
| if (type == TRIGGER_PROXIMITY){ |  |  |  |  |
| referenceNode | number | O | Active camera | Index in the nodes array, of the node to consider for the proximity evaluation. In the absence of the referenceNode attribute, the active camera managed by the application shall be used. |
| distanceLowerLimit | number | O | 0 | Threshold minimum in meters for the proximity calculation, based on the distance between the nodes and the referenceNode. |
| distanceUpperLimit | number | M |  | Threshold maximum in meters for the proximity calculation, based on the distance between the nodes and the referenceNode. |
| nodes | array | M |  | Indices of the nodes in the nodes array to be considered. All the nodes shall have a distance from the origin of their local space to the referenceNode above the distanceLowerLimit and below the distanceUpperLimit to activate the trigger |
| primitives | array(Primitive) | O | N/A | List of primitives used to activate the proximity or collision trigger. Semantics are presented in Table 9. |
| } |  |  |  |  |
| if (type== TRIGGER_USER_INPUT){ |  |  |  |  |
| userInputDescription | string | M |  | Describes the user body part and gesture related to the input. The format shall follow the OpenXR input path description as defined in [OpenXR] section 6. An example is: “/user/hand/left/grip”. |
| nodes | array | O |  | Indices of the nodes in the nodes array to be considered for this user input. |
| } |  |  |  |  |
| if (type== TRIGGER_VISIBILITY){ |  |  |  |  |
| cameraNode | number | M |  | Index to the node containing a camera in the nodes array for which the visibilities are determined. The visibility trigger shall be evaluated only if the related camera is active. |
| nodes | array | M |  | Indices of the nodes in the nodes array to be considered. All the nodes shall be visible by the camera to activate the trigger. |
| } |  |  |  |  |

Table 8: type of trigger

| Trigger type | Description |
| --- | --- |
| TRIGGER_COLLISION=0 | Collision Trigger |
| TRIGGER_PROXIMITY | Proximity Trigger |
| TRIGGER_USER_INPUT | User_Input Trigger |
| TRIGGER_VISIBILITY | Visibility Trigger |

The semantics of a trigger primitive are defined in Table 9.

Table 9: Semantics of MPEG_scene_interactive.trigger.primitive properties

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | O | BV_SPHEROID | Describes the type of primitive used to activate the proximity trigger. The available options are: BV_CUBOID = 0, BV_PLANE_REGION = 1, BV_CYLINDER_REGION = 2, BV_CAPSULE_REGION = 3, BV_SPHEROID = 4 The default is BV_SPHEROID. Semantics are presented in Table 10. |
| boundary | number | O | 0.0 | Defines the region of intersection within the primitive. if zero, then all area of the primitive activates the trigger. Otherwise, the region of intersection decreases following the normal direction of all sides of the primitive from its centroid. For the capsule primitive, it should be applied over the radius, top, and base attributes. |
| transformationMatrix | array | O | [1.0,0.0,0.0,0.0, 0.0,1.0,0.0,0.0, 0.0,0.0,1.0,0.0, 0.0,0.0,0.0,1.0] | Floating-point 4x4 matrix that defines the initial orientation, translation, and scale of a primitive. Formatted in column-major order. The primitive shall follow x+ for width, y+ for height, z+ for length . The matrix transformation allows to transform any primitive after initialization. |

Table 10: Semantical description of each primitive region

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| if (type == BV_CUBOID) { |  |  |  |  |
| width | number | M |  | Width of the box. |
| height | number | M |  | Height of the box. |
| length | number | M |  | Length of the box. |
| centroid | vec3 | M |  | Centroid 3D coordinate (x,y,z) of the cube. |
| } |  |  |  |  |
| if (type == BV_PLANE_REGION) { |  |  |  |  |
| width | number | M |  | Width of the plane. |
| height | number | M |  | Height of the plane. |
| centroid | vec2 | M |  | Centroid 2D coordinate (x,y) or (x,z) or (y,z) of the plane. |
| } |  |  |  |  |
| if (type == BV_CYLINDER_REGION) { |  |  |  |  |
| radius | number | M |  | Radius of the cylinder. |
| length | number | M |  | Length of the cylinder. |
| centroid | vec3 | M |  | Centroid 3D coordinate (x,y,z) of the cylinder |
| } |  |  |  |  |
| if (type == BV_CAPSULE_REGION) { |  |  |  |  |
| radius | number | M |  | Radius of the capsule. |
| baseCentroid | vec3 | M |  | Centroid 3D coordinate (x,y,z) of the base semi-sphere of the capsule. |
| topCentroid | vec3 | M |  | Centroid 3D coordinate (x,y,z) of the top semi-sphere of the capsule. |
| } |  |  |  |  |
| if (type == BV_SPHEROID) { |  |  |  |  |
| radius | number | M |  | Radius of the sphere. |
| centroid | vec3 | M |  | Centre 3D coordinate (x,y,z) of the sphere. |
| } |  |  |  |  |

The semantic of an action is provided in Table 11.

Table 11: Semantic of action

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | One element of Table 12 that defines the type of the action. |
| delay | number | O | 0.0 | Duration of delay in second before executing the action. |
| if (type== ACTION_ACTIVATE){ |  |  |  |  |
| activationStatus | enum | M |  | ENABLED=0: the node shall be processed by the application DISABLED =1: the node shall be skipped by the application. |
| nodes | array | M |  | Indices of the nodes in the nodes array to set the activation status. |
| } |  |  |  |  |
| if (type== ACTION_TRANSFORM){ |  |  |  |  |
| transform |  | M |  | A 4x4 transformation matrix to apply to the nodes. |
| nodes | array | M |  | Indices of the nodes in the nodes array to be transformed. |
| } |  |  |  |  |
| if (type== ACTION_BLOCK){ |  |  |  |  |
| nodes | array | M |  | Indices of the nodes in the nodes array to lock their related transforms. |
| } |  |  |  |  |
| if (type == ACTION_ANIMATION){ |  |  |  |  |
| animation | number | M |  | Index of the animation in the animations array to be considered. |
| animationControl | enumeration | M |  | One element of Table 13 that defines the control of the animation. |
| } |  |  |  |  |
| if (type == ACTION_MEDIA){ |  |  |  |  |
| media | number | M |  | Index of the media in the MPEG media array to be considered. |
| mediaControl | enumeration | M |  | One element of Table 14 that defines the control of the media. |
| } |  |  |  |  |
| if (type == ACTION_MANIPULATE){ |  |  |  |  |
| manipulateActionType | enumeration | M |  | One element of Table 15 that defines the action manipulate type. |
| axis | array | O | Up | (x,y,z) coordinates of the axis used for rotation and sliding. These coordinates are relative to the local space created by the user input described in the userInputDescription. For example a “/user/hand/left/pose” user input trigger creates a local space attached to the user left hand. |
| userInputDescription | string | M |  | Describe the user input related to the manipulation action. The format shall follow the OpenXR input path description as defined in [OpenXR] section 6. An example is: “/user/hand/left/aim/pose”. |
| nodes | array | M |  | Indices of the nodes in the nodes array to be manipulated. |
| } |  |  |  |  |
| if (type == ACTION_SET_MATERIAL){ |  |  |  |  |
| material | number | M |  | Index of the material in the materials array to apply to the set of nodes. |
| nodes | array | M |  | Indices of the nodes in the nodes array to set their material. |
| } |  |  |  |  |
| if (type == ACTION_HAPTIC){ |  |  |  |  |
| hapticActionNodes | array(HapticActionNode) | M |  | List of haptic action nodes. |
| } |  |  |  |  |
| if (type == ACTION_SET_AVATAR) { |  |  |  |  |
| avatarAction | string | M |  | The avatarAction is a URN that uniquely identifies the avatar action. For the MPEG reference Avatar, a set of avatar actions and their respective URNs is defined in Table H.4.3.2 of Annex H. For example, the URN “urn:mpeg:sd:2023:avatar:actionslist/speech” referenced in Table H.4.3.2 of Annex H sets the optional attributes “microphone” and “media”. Considering only the boolean attribute “microphone”, the nodes in the node array will activate/deactivate their “microphone” ability accordingly when this action is launched. |
| nodes | array | M |  | Indices of the nodes in the nodes array to launch their avatar actions. |
| } |  |  |  |  |

Table 12: Type of action

| Action type | Description |
| --- | --- |
| ACTION_ACTIVATE=0 | Set activation status of a node |
| ACTION_TRANSFORM | Set transform to a node |
| ACTION_BLOCK | Block the transform of a node |
| ACTION_ANIMATION | Select and control an animation |
| ACTION_MEDIA | Select and control a media |
| ACTION_MANIPULATE | Select a manipulate action |
| ACTION_SET_MATERIAL | Set new material to nodes |
| ACTION_HAPTIC | Get haptic feedbacks on a set of nodes |
| ACTION_SET_AVATAR | Get avatar related actions |

Table 13: Control of animation

| Animation Control | Description |
| --- | --- |
| ANIMATION_PLAY=0 | Play the animation from time 0 or from any other time provided by a control. |
| ANIMATION_PAUSE | Pause the animation |
| ANIMATION_RESUME | Resume the animation from the last pause position. |
| ANIMATION_STOP | Stop the animation |

Table 14: control of media

| Media Control | Description |
| --- | --- |
| MEDIA_PLAY=0 | Play the media from time 0 or from any other time provided by a control. |
| MEDIA_PAUSE | Pause the media |
| MEDIA_RESUME | Resume the media from the last pause position. |
| MEDIA_STOP | Stop the media |

Table 15: Action Manipulate Type

| Action Manipulate Type | description |
| --- | --- |
| ACTION_MANIPULATE_FREE=0 | The nodes follow the user pointing device and its rotation. |
| ACTION_MANIPULATE_SLIDE | The nodes move linearly along the provided axis by following the user pointing device. |
| ACTION_MANIPULATE_TRANSLATE | The nodes translate by following the user pointing device. |
| ACTION_MANIPULATE_ROTATE | The nodes rotate around the provided axis by following the user pointing device. |
| ACTION_MANIPULATE_SCALE | Performs a central scaling of the nodes by following the user pointing device. |

The semantics of a behavior is provided in Table 16.

Table 16: semantic of behavior

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M |  | Indices of the triggers in the triggers array considered for this behavior |
| actions | array | M |  | Indices of the actions in the actions array considered for this behavior. |
| triggersCombinationControl | string | M |  | Set of logical operations to apply to the triggers A ‘#’ indicates the trigger index, ‘&’ indicates a logical AND operation, ‘¦’ a logical OR operation and ‘~’ a NOT operation. Parenthesis are used to group some operations. Such a syntax may give the following string: “#1&~#2¦(#3&#4)”. An empty string is understood as a logical OR between all the triggers. A regex expression (https://json-schema.org/understanding-json-schema/reference/regular_expressions.html) is specified in the JSON schema to validate this string. |
| triggersActivationControl | enumeration | M |  | Indicates when the combination of the triggers shall be activated for launching the actions. One element ofTable 17 that defines when the combination of the triggers shall be activated for launching the actions. |
| actionsControl | enumeration | M |  | Defines the way to execute the defined actions. SEQUENTIAL=0: each defined action is executed sequentially in the order of the actions array. PARALLEL=1: the defined actions are executed concurrently. |
| interruptAction | number | O | N/A | Index of the action in the actions array to be executed if the behavior is still on-going and is no more defined in a newly received scene update. |
| priority | number | O | 0 | Integer value defining the priority associated to the behavior When several behaviors are in concurrence to affect the same node(s) at the same time, the behavior having the highest priority value shall be processed. The lower priority behavior(s) shall not be processed. In the case of behaviors having the same priority, the application should apply its own criteria. |

Table 17:Trigger Activation Control

| Trigger Activation Control | Description |
| --- | --- |
| TRIGGER_ACTIVATE_FIRST_ENTER=0 | Activated when the conditions are first met |
| TRIGGER_ACTIVATE_EACH_ENTER | Activated each time the conditions are first met |
| TRIGGER_ACTIVATE_ON | Activated as long as the conditions are met |
| TRIGGER_ACTIVATE_FIRST_EXIT | Activated when the conditions are first no longer met |
| TRIGGER_ACTIVATE_EACH_EXIT | Activated each time the conditions are no longer met |
| TRIGGER_ACTIVATE_OFF | Activated as long as the conditions are not met |

Table 18: Semantic of HapticActionNode object

| Property | Type | Required | Default | Description |
| --- | --- | --- | --- | --- |
| node | integer | M |  | Identifier of the node in the glTF nodes array. |
| hapticObject | integer | O | N/A | Index to a haptic object in the hapticObjects array of the MPEG_haptic extension. |
| actionLocation | integer | O | 0xFFFFFFFF | Body part mask specifying where on the body the signal can be rendered. Possible values are detailed in Table 22. |
| washout | boolean | O | False | Specifies whether the action should trigger a washout (reset to the origin) of the associated devices. |
| useCollider | boolean | O | False | Used with a Collision trigger. If True, the rendering engine shall use collision information to estimate the desired location of the haptic feedback on the body. For haptic materials, the presentation engine retrieves the associated haptic texture media and generates haptic feedback based on texture information and collision position. If false, the signal shall be rendered based on the information specified in the Haptic file. |
| materialHapticModality | array(enum) | O | N/A | List of haptic material modalities that shall be rendered. Possible values are detailed in Table 21. |
| hapticActionMedias | array(HapticActionMedia) | M |  | List of Haptic Action Media. |

Table 19: Semantic of the HapticActionMedia object

| Property | Type | Required | Default | Description |
| --- | --- | --- | --- | --- |
| mediaIndex | integer | M |  | Index in the accessors array of the associated haptic data. |
| perceptionIndices | array(integer) | M |  | Indices of the perceptions of the media that shall be rendered. If the list if empty all perceptions shall be rendered |
| hapticModality | array(enumeration) | O | N/A | List of haptic modalities that can be rendered. Possible values are described in Table 20. |
| hapticControl | enumeration | O | HAPTIC_PLAY | One element of Table 24 that defines the control of the haptic rendering. |
| loop | boolean | O | False | Specifies if the haptic rendering of the data should be continuously looping. |

Table 20: List of haptic modalities

Pressure = 0

Acceleration

Velocity

Position

Temperature

Vibrotactile

Water

Wind

Force

Electrotactile

Vibrotactile Texture

Stiffness

Friction

Other

Table 21: List of haptic Material Modalities

Stiffness = 0

Friction

Vibrotactile Texture

Temperature

Vibration

Custom

Table 22: Body part masks

|  | Name | Body_part_mask (binary) | Hexadecimal | Decimal |
| --- | --- | --- | --- | --- |
| 0 | Unspecified | 00000000000000000000000000000000 | 0x00000000 | 0 |
| 1 | Head Face | 00000000000000000000000000000001 | 0x00000001 | 1 |
| 2 | Head Back/Neck/Ears | 00000000000000000000000000000010 | 0x00000002 | 2 |
| 3 | Mouth Bag | 00000000000000000000000000000100 | 0x00000004 | 4 |
| 4 | Lower Jaw | 00000000000000000000000000001000 | 0x00000008 | 8 |
| 5 | Upper Jaw | 00000000000000000000000000010000 | 0x00000010 | 16 |
| 6 | Eye Left | 00000000000000000000000000100000 | 0x00000020 | 32 |
| 7 | Eye Right | 00000000000000000000000001000000 | 0x00000040 | 64 |
| 8 | Chest Front | 00000000000000000000000010000000 | 0x00000080 | 128 |
| 9 | Chest Back | 00000000000000000000000100000000 | 0x00000100 | 256 |
| 10 | Upper Arm Left | 00000000000000000000001000000000 | 0x00000200 | 512 |
| 11 | Lower Arm Left | 00000000000000000000010000000000 | 0x00000400 | 1 024 |
| 12 | Hand Left | 00000000000000000000100000000000 | 0x00000800 | 2 048 |
| 13 | Upper Arm Right | 00000000000000000001000000000000 | 0x00001000 | 4 096 |
| 14 | Lower Arm Right | 00000000000000000010000000000000 | 0x00002000 | 8 192 |
| 15 | Hand Right | 00000000000000000100000000000000 | 0x00004000 | 16 384 |
| 16 | Abdomen Front | 00000000000000001000000000000000 | 0x00008000 | 32 768 |
| 17 | Abdomen Back | 00000000000000010000000000000000 | 0x00010000 | 65 536 |
| 18 | Upper Leg Left | 00000000000000100000000000000000 | 0x00020000 | 131 072 |
| 19 | Lower Leg Left | 00000000000001000000000000000000 | 0x00040000 | 262 144 |
| 20 | Foot Left | 00000000000010000000000000000000 | 0x00080000 | 524 288 |
| 21 | Upper Leg Right | 00000000000100000000000000000000 | 0x00100000 | 1 048 576 |
| 22 | Lower Leg Right | 00000000001000000000000000000000 | 0x00200000 | 2 097 152 |
| 23 | Foot Right | 00000000010000000000000000000000 | 0x00400000 | 4 194 304 |
| 24-32 | Reserved | 00000000010000000000000000000000 - 10000000000000000000000000000000 | 0x00800000 -0x80000000 | 8 388 608 - 2 147 483 648 |

Table 23: Examples of body part combinations

| Name | Body_part_mask (binary) | Hexadecimal | Decimal |
| --- | --- | --- | --- |
| Right arm | 00000000000000000111000000000000 | 0x00007000 | 28 672 |
| Left arm | 00000000000000000000111000000000 | 0x00000E00 | 3 584 |
| Right leg | 00000000011100000000000000000000 | 0x00700000 | 7 340 032 |
| Left leg | 00000000000011100000000000000000 | 0x000E0000 | 917 504 |
| Upper body | 00000000000000011111111111111111 | 0x0001FFFF | 131 071 |
| Lower body | 00000000011111100000000000000000 | 0x007E0000 | 8 257 536 |
| Full body | 11111111111111111111111111111111 | 0xFFFFFFFF | 4 294 967 295 |

Table 24: List of Haptic controls

| Haptic Control | Description |
| --- | --- |
| HAPTIC_PLAY = 0 | Start the rendering of the haptic data from time 0 or from any other time provided by a control |
| HAPTIC_PAUSE | Pause the rendering of the haptic data |
| HAPTIC_RESUME | Resume the rendering of the haptic data from the last pause position. |
| HAPTIC_STOP | Stop the rendering of the haptic data |

#### 8.2.2.2	Semantics at node level

In complement to the interactivity objects defined in the glTF scene-level extension, some additional data could be provided at the level of the affected glTF nodes to specialize the trigger activation.

The semantic of the MPEG_node_interactivity extension is shown in Table 25.

Table 25: Semantic of the MPEG_node_interactivity extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| triggers | array | M |  | Array of node triggers (as defined in Table 26). Only distinct types are allowed. The minimum size of this array is 1, and the maximum size is size of trigger types as defined in this specification. |

Table 26: Semantic of the MPEG_node_interactivity.trigger extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | One element of Table 8 that defines the type of the trigger. |
| if (type == TRIGGER_COLLISION){ |  |  |  |  |
| collider | integer | M |  | the index of the mesh element that provides the collider geometry for the current node. The collider mesh may reference a material. |
| isStatic | boolean | M |  | If True, the collider is defined as a static collider. |
| usePhysics | boolean | M |  | Indicates if the object shall be considered by the physics simulation. |
| if (usePhysics) { |  |  |  |  |
| useGravity | boolean | M |  | Indicates if the gravity affects the object |
| mass | number | M |  | Mass of the object in kilogram. |
| restitution | number | M |  | Provides the ratio of the final to initial relative velocity between two objects after they collide. |
| staticFriction | number | M |  | Unitless friction coefficient as defined in the Coulomb friction model. Friction is the quantity which prevents surfaces from sliding off each other. StaticFriction is used when the object is lying still. It will prevent the object from starting to move. |
| dynamicFriction | number | M |  | Unitless friction coefficient as defined in the Coulomb friction model. When a large enough force is applied to the object, the dynamicFriction is used, and will attempt to slow down the object while in contact with another. |
| } |  |  |  |  |
| primitives | array(Primitive) | O | N/A | List of primitives used to activate the proximity or collision trigger. Semantics are presented in Table 9. |
| } |  |  |  |  |
| if (type == TRIGGER_PROXIMITY){ |  |  |  |  |
| allowOcclusion | boolean | M |  | Indicates if occlusion by other nodes should be considered |
| upperDistanceWeight | number | O | 1 | The weight applied to the distanceUpperLimit parameter defined at scene level |
| lowerDistanceWeight | number | O | 1 | The weight applied to the distanceLowerLimit parameter defined at scene level |
| primitives | array(Primitive) | O | N/A | List of primitives used to activate the proximity or collision trigger. Semantics are presented in Table 9. |
| } |  |  |  |  |
| if (type ==TRIGGER_USER_INPUT){ |  |  |  |  |
| userInputParameters | array | M |  | Provides additional information related to the user inputs (eg “max speed = 0.5”) |
| } |  |  |  |  |
| if (type== TRIGGER_VISIBILITY){ |  |  |  |  |
| allowsPartialOcclusion | boolean | M |  | The visibility computation shall take into account both the occultation by other node(s) and the camera frustrum. If the allowsPartialOcclusion Boolean is TRUE, then a partial visibility of this node activates the trigger. If the allowsPartialOcclusion Boolean is FALSE, then this node shall be fully in the camera frustrum and not be occluded by any other node(s) except the nodes listed in the nodes array to activate the trigger. |
| nodes | array | O | N/A | Set of nodes that shall not be considered for the visibility computation, when the allowsPartialOcclusion is FALSE. |
| mesh | number | O | N/A | Index of the mesh in the scene meshes array that will be used to compute visibility. |
| } |  |  |  |  |

### 8.2.3	Processing model

When a scene description file becomes available, the Presentation Engine

- parses the related glTF file

- creates each behavior composed of triggers and actions described at the interactivity scene level extension

- specializes the trigger for each affected node with the additional data provided at the interactivity node level extension if present

At runtime, the presentation Engine iterates on each behavior and performs the following tasks:

- checks the activation status of each trigger of this behavior by following the procedure detailed in Figure 1,

- checks the logical combination of these trigger status,

- if this status satisfies the triggersActivationControl value, launches the corresponding actions.

Figure 1: processing model for the activation of a single trigger

When several behaviors are in concurrence to affect the same node(s) at the same time, the behavior having the highest priority value shall be processed. The lower priority behavior(s) shall not be processed. In the case of behaviors having the same priority, the application should apply its own criteria.

When a new scene description update is received, the application follows the procedure detailed in Figure 2. An on-going behavior corresponds to a behavior:

- having its triggers status verifying the triggersActivationControl value for that frame,

- or having previously launched a play action related to a media/animation,

- or having previously launched an action with a delay not yet expired.

To check if the behavior is still defined, the application should check if the operations contained in the json patch document of the scene update lead to:

- removal of this behavior in the behaviors array.

- modification of any parameter of this behavior (e.g. by adding a new trigger, by changing the trigger activation control,…).

- removal of any of the referenced nodes by the action.

The application shall process the interrupt action only if the timing requirement of the scene update is met.

Figure 2: Processing model when a new scene description update is received.

If the usePhysics Boolean is TRUE on any of the collision trigger extensions defined at the node level, the application should handle a physics simulation. When a collision occurs between two nodes, the application should calculate the combination of the restitution, static friction and dynamic friction values based on the values provided by the collision trigger extension of the two nodes.

## Avatar

### 8.3.1	General

Signaling an avatar at a node level is achieved by the definition of the MPEG_node_avatar extension.

Depending on the input components and the offline model, the proper avatar reconstruction and animation needs to be instantiated to generate the final dynamic/animated 3D mesh of the avatar.

When considering describing a user’s avatar in scene description, the following requirements are derived:

- The avatar may be reconstructed/animated from a wide range of Avatar representations,

- The reconstructed/animated avatar representation abides by the supported primitives in scene description,

- It is possible to associate interactivity triggers with different parts/segments of the user avatar (e.g. hand or finger) using the node interactivity triggers.

To address these requirements, the avatar reconstruction/animation is assumed to be independent of the avatar rendering. This design aligns with the principles and architecture of this specification. Irrespective of the chosen avatar representation format, the Presentation Engine is then only required to render the reconstructed avatar in one of the supported 3D formats, such as a dynamic mesh. The following figure depicts how this approach.

Figure 3: Avatar media pipeline

The MPEG_node_avatar extension shall be included as extension to node object. All referenced mesh elements by this node will be reconstructed from an avatar representation.

### 8.3.2	Semantics

The application identifies the avatar format and reconstruction method based on the type of the MPEG_node_avatar extension. The default type is defined in Annex H of this specification.

Table 27: Description of MPEG_node_avatar extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | string | M |  | The type of the avatar representation is provided as a URN that uniquely identified the avatar representation scheme. The avatar representation scheme defines the format of all components that are used to reconstruct and animate the avatar. The reference MPEG avatar URN is defined in section 8.3.3. |
| mappings | array(Mapping) | M |  | The mapping between child nodes and their associated avatar path. Note that the corresponding path for a parent node shall be a prefix of the path of its child nodes. |

The Mapping object is defined as follows:

Table 28: Description of Mapping object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| path | AvatarPath | M |  | Provides the Avatar path for this node as described in clause 8.3.4. |
| node | integer | M |  | The index of the child node of the mesh part that corresponds to the label in the path. |

### 8.3.3	MPEG reference avatar

The MPEG reference avatar is identified by “urn:mpeg:sd:2023:avatar” as avatar type. The reference avatar framework builds on the reference avatar as defined in Annex H.

### 8.3.4	Avatar path definition

The avatar path allows the addressing of every segment in an avatar, which allows for associating them with interactivity triggers and skeleton animations.

Table 29: Avatar paths

"full_body": { "upper_body":{ "head":{ "face":{ "mouth", "lower_jaw", "upper_jaw", "eye_right", "eye_left" }, "neck", "ear_right", "ear_left", "back" }, "thorax":{ "chest_front", "chest_back", "shoulder_front_right", "shoulder_front_left", "shoulder_back_right", "shoulder_back_left" }, "arm_right":{ "upper_arm_right", "lower_arm_right", "hand_right":{ "thumb_proximal_right":{ "thumb_intermediate_right":{ "thumb_distal_right" } }, "index_proximal_right":{ "index_intermediate_right":{ "index_distal_right":{ "index_top_right" } } }, "middle_proximal_right":{ "middle_intermediate_right":{ "middle_distal_right":{ "middle_top_right" } } }, "ring_proximal_right":{ "ring_intermediate_right":{ "ring_distal_right":{ "ring_top_right" } } }, "litttle_proximal_right":{ "litttle_intermediate_right":{ "litttle_distal_right":{ "litttle_top_right" } } } } }, "arm_left":{ "upper_arm_left", "lower_arm_left", "hand_left":{ "thumb_proximal_left":{ "thumb_intermediate_left":{ "thumb_distal_left" } }, "index_proximal_left":{ "index_intermediate_left":{ "index_distal_left":{ "index_top_left" } } }, "middle_proximal_left":{ "middle_intermediate_left":{ "middle_distal_left":{ "middle_top_left" } } }, "ring_proximal_left":{ "ring_intermediate_left":{ "ring_distal_left":{ "ring_top_left" } } }, "litttle_proximal_left":{ "litttle_intermediate_left":{ "litttle_distal_left":{ "litttle_top_left" } } } } } } "lower_body":{ "pelvis":{ "pelvis_front":{ "pelvis_front_right", "pelvis_front_left" }, "pelvis_back":{ "pelvis_back_right", "pelvis_back_left" } }, "leg_right":{ "upper_leg_right", "lower_leg_right", "foot_right":{ "toes_right" } }, "leg_left":{ "upper_leg_left", "lower_leg_left", "foot_left":{ "toes_left" } } } }

### 8.3.5	Processing model

Each avatar in a scene is represented by a node that contains the MPEG_node_avatar extension. When present in a node, it means that all referenced mesh elements by this node will be reconstructed from an avatar representation.

## 8.4 	Lighting

### 8.4.1	General

This document defines types of light sources with their corresponding extensions:

- The image-based lights via the MPEG_lights_texture_based extension

- The punctual lights via the MPEG_light_punctual extension

The MPEG_lights_texture_based extension defines image-based lights in a glTF scene. This extension expands the functionalities of the EXT_lights_image_based extension by enabling the lighting information to vary over time. The lighting information in the MPEG_lights_texture_based extension comprises two timed dependent sequences that are:

- a video sequence wherein each frame represents the specular radiance information

- a timed metadata sequence where each sample of the sequence represents irradiance information

At each time instance of the glTF scene rendering, the corresponding specular radiance decoded frame and the irradiance sample provide the necessary information to apply image-based lighting rendering techniques.

Regarding the punctual lights extension, identified by MPEG_light_punctual, it provides the ability to differentiate between phyiscal and virtual light sources present as KHR_lights_punctual elements as well as providing accessor to retrieve timed dependent information it.

### 8.4.2	Semantics

#### 8.4.2.1	Semantics of the MPEG_lights_texture_based extension

When present, the MPEG_lights_texture_based extension shall be included as a glTF file level extension and as a scene level extension.

The definition of all objects within MPEG_lights_texture_based extension is provided in Table 30.

Table 30: Definitions of glTF file level objects of MPEG_lights_texture_based extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| lights | array | M |  | An array of items that describe the texture-based light sources, referenced in this scene description document. |

Table 31: Definitions of item in the lights array of MPEG_lights_texture_based extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | O | N/A | Name of the light |
| nature | enumeration | O | PHYSICAL | Indicates whether the lighting information corresponds to - PHYSICAL = 0: a physical light, - VIRTUAL = 1: a virtual light, - PHYSICAL_VIRTUAL = 2:both physical and virtual, UNKNOWN = 3: unknown. In case of a physical light source, an AR anchor that refers to this light source should be present. |
| position | array | O | (0,0,0) | Position of both light and reflection probes associated with the lighting information. In the absence of this position, the lighting information is assumed to be global (i.e. for the entire scene) for which the center is the scene origin. When an anchor refers to a light source, the position of the anchor takes precedence over this position attribute. |
| projection | numeration | O | EQUIRECTANGULAR | Provides the projection typeof the specular images. The type may be either - EQUIRECTANGULAR = 0, - CUBEMAP = 1 In the latter case, the 6 faces of the radiance map should be packed according to the order described by Table 10 of the ISO/IEC 23090-7 specification. |
| rotationAccessor | integer | O | N/A | Provides a reference to the accessor giving a sequence of quaternions as described in rotation of in EXT_lights_image_based. When an anchor refers to a light source, the rotation of the anchor takes precedence over this rotation attribute. In the absence this rotation_accessor, there is no rotation to apply to the irradiance map and specular images. |
| intensityAccessor | integer | O | N/A | Provides a reference to the accessor giving a sequence of intensity values as described in intensity of in EXT_lights_image_based. In the absence this intensity_accessor, the intensity of the light is 1.0. |
| irradianceAccessor | integer | M | N/A | Provides a reference to the accessor giving a sequence of irradiance coefficient samples. Irradiance coefficients are defined as in irradianceCoefficients of EXT_lights_image_based. An irradiance coefficients sample is an array of 27 values of floating-point type corresponding to 9 vectors of 3 components, one per RGB channels. The coefficients are stored in the RGB order and from low to high order channel-major fashion i.e. [r0, g0, b0,r1, g1, b1, ... ]. See Khronos EXT_lights_image_based for the definition of the irradiance coefficient sample. |
| specularImages | array | M | N/A | Provides a list of references to textures providing specular images (environment map). The length of the array shall be equal to “1” or “6”. When the length of the array is equal to “1”, the specular images are provided in a packed format as defined by the projection attribute. When the length of the array is equal to “6”, the projection attribute shall be of type cubemap and the order of faces are TBD. |

Table 32: Definitions of objects at the scene level of MPEG_lights_texture_based

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| light | array(integer) | M |  | Reference to an item in the lights array of the MPEG_lights_texture_based extension. |

The JSON schema for the MPEG_lights_texture_based extension is provided in A.14.

#### 8.4.2.2	Semantics of the MPEG_light_punctual extension

When present, the MPEG_light_punctual extension shall be included as a glTF file level extension.

The definition of all objects within MPEG_light_punctual extension is provided in Table 33.

Table 33 – Definition of glTF file objects of MPEG_light_punctual extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| light | integer | M |  | Index of the item in the array of the lights parameter of the KHR_lights_punctual extension |
| nature | numeration | M |  | Indicates whether the lighting information corresponds to - PHYSICAL = 0: a physical light, - VIRTUAL = 1: a virtual light, - PHYSICAL_VIRTUAL = 2:both physical and virtual, - UNKNOWN = 3: unknown. In case of a physical light source, an AR anchor that refers to this light source should be present |
| color_accessor | integer | O | N/A | Provides a reference to the accessor giving a sequence of color value as described in color of in KHR_lights_punctual. In the absence of the accessor, the value in the referenced light is considered as the constant value for the rendering duration. |
| intensity_accessor | integer | O | N/A | Provides a reference to the accessor giving a sequence of intensity value as described in intensity of KHR_lights_punctual. In the absence of the accessor, the value in the referenced light is considered as the constant value for the rendering duration. |
| range_accessor | integer | O | N/A | Provides a reference to the accessor giving a sequence of range value as described in range of in KHR_lights_punctual. In the absence of the accessor, the value in the referenced light is considered as the constant value for the rendering duration. |

The JSON schema for the MPEG_light_punctual extension is provided in A.15.

### 8.4.3	Processing model

When a scene description document contains light source defined in the MPEG_lights_texture_based extension or the MPEG_light_punctual extension, the renderer should take into consideration this lighting information when rendering the view of the scene. The operations to be applied depends on the context of the application, e.g. if it is a VR or an AR application. In addition, the renderer should also consider whether the lights are virtual or physical based on the value of the nature field.

For instance, when the scene contains both physical and virtual lighting information, such as in AR applications, then the renderer should relight the virtual elements of the scene (e.g. virtual objects) present in the scene with both physical and virtual lighting information. Regarding the real elements of the scene, the renderer should only relight them using the virtual lighting information since, by definition, the real elements of the scene are already illuminated by the physical light sources in the scene.

Regardless of the application type, when the lighting information is parsed, the Presentation Engine places the image-based light in the scene based on the rotation, intensity and irradiance values, as well as setting the specular images in the lighting model

The Presentation Engine places the punctual lights using the same processing model as the KHR_lights_punctual elements and retrieves timed dependent information through the color, intensity and range accessors.

Depending on the view pose, the render should illuminate each object of the scene by the lights surrounding the objects, possibly by interpolating the lighting contribution based on the relative distance.

========================== Next Change ===========================

Add the following to Annex A

A.10 JSON schema for MPEG_anchor

MPEG_anchor schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_anchor.schema.json.

A.11 JSON schema for MPEG_haptics

MPEG_haptics.schema.json schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_haptics.schema.json.

A.12 JSON schema for MPEG_scene_interactivity and MPEG_node_interactivity

MPEG_scene_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_scene_interactivity.schema.json

MPEG_node_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_node_interactivity.schema.json

A.13 	JSON schema for MPEG_avatar

MPEG_avatar schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_avatar.schema.json

A.14		JSON schema for MPEG_lights_texture_based

MPEG_lights_texture_based schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_lights_texture_based.schema.jsonA.15		JSON schema for MPEG_light_punctual

MPEG_light_punctual schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_light_punctual.schema.json

========================== Next Change ===========================

Add the following clause to Annex G.

## G.3 	Support for MPEG-I Haptics

### G.3.1	General

The MPEG_haptic and MPEG_haptic material extensions provide the ability to define the integration of haptics in a glTF scene . The extensions are depicted in Figure G.3-1 and are defined as follows:

MPEG_haptic:

- At the glTF file level: The extension contains an array defining every haptic object

MPEG_haptic_material:

- At the glTF file level: The extension contains an array defining all texture-based haptic data

At the mesh level: The extension contains a single reference to the array of the same extension at the glTF file level

Figure G.3-1: extensions for haptics among all MPEG extensions to glTF

The following table provides use cases for haptic signals:

| Use Case | Haptic Data | Rendering behavior |
| --- | --- | --- |
| Touch the surface of a wall and feel a texture | A Texture perception is defined describing a surface friction pattern. | Upon collision between a body part and the textured wall, the Texture track of the media is read based on the displacement of the body part on the surface. |
| Press a button to get stiffness feedback | A Stiffness/force feedback perception is defined describing a force curve profile. | When the button is pressed, the force profile curve is read based on the pressure depth of the button. |
| Opening a door and feeling friction of the door with the floor | A Texture perception is defined describing a surface friction pattern. | When the door is moving, the Texture track of the media is read based on the displacement of the door (rotation angle converted to distance traveled). |
| Walking in a street and at some time rain is falling on the user body | Localized vibrations are defined to simulate raindrop | Upon time event “rain starts” vibrations patterns are synthesized and rendered on the appropriate device. |

### G.3.2	 Semantics for MPEG_haptic

The MPEG_haptic extension allows the support of haptic data as defined in ISO/IEC 23090-31 [10]. It is an independent haptic media like an audio content or an image. This extension is attached at the the glTF file level and stores the haptic data contained in the scene.

As detailed in G.3-1, the MPEG_haptic extension contains an array of haptic objects. The data for each element of the array is detailed in Table G.3-2, it contains a list of accessors to medias in the MPEG_media extension.

Table G.3-1: Semantic description of the MPEG_haptic extension at the glTF file level

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| hapticObjects | Array<MPEG_Haptics.hapticObject> | N/A | Provides a list of haptic elements at the glTF file level to enable haptic support |

Table G.3-2: Semantic description of the MPEG_haptics.hapticObject items of the hapticObjects array from the MPEG_haptic extension

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| accessors | array<integer> | N/A | Array of accessors to one or more media sources in MPEG_media.media array containing haptic media files. |

The data referenced by the accessors is stored in dedicated Haptic buffers. The Haptic buffer formats corresponds to the HMPG binary format detailed in ISO/IEC 23090-31.

### G.3.3	Semantics MPEG_haptic_material

The MPEG_haptic_material extension defines the association of haptic media with the interactivity extension. The haptic texture associated with a 3D object does not contain RGB values but haptic values. These values are exploited directly by the haptic renderer. The extension also uses the concept of taxels. Each pixel of the texture can be mapped to a distinct spatial (or temporal) signal as illustrated in Figure G.3-2.

Figure G.3-2: Grid of taxels, mapping pixels to different haptic textures

An array of textures for each haptic property is used. An haptic texture can be provided both as a traditional 2D texture and as a taxel map in the same file, giving the possibility to the rendering engine to choose the most appropriate.

Additional information is added to each element of the haptic texture arrays for the rendering engine to adequately interpret a texture. Each array element then contains a haptic texture and a texture type expressed as an Enumeration. Possible values of the enumeration are:

- HIGH_RESOLUTION: The haptic texture is a high resolution 2D texture directly storing haptic values

- LOW_RESOLUTION: The haptic texture is a low resolution 2D texture directly storing haptic values

- REFERENCE: The haptic texture is a 2D taxel map containing references to haptic signals. Each pixel of the texture corresponds to an index in the accessors array of the MPEG_haptic extension.

- OTHER: Unknown proprietary texture format.

To interpret the data contained in 2D textures, the bit depth and range of these textures are specified according to the following tables.

The following table gives the bit depth and range values for each haptic property for low resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| stiffness | 8-bit | 0-10000 N.s −1 /m.s −1 | 40 N.s −1 /m.s −1 |
| friction | 8-bit | ±5 | 0.04 |
| vibrotactileTexture | 8-bit | ±10 | 0.08 |
| temperature | 8-bit | [-50:+75]°C | 0.5°C |
| vibration | 8-bit | [0-1] | 0,004 |
| custom | 8-bit | 0-255 | 1 |

The following table gives the bit depth and range values for each haptic property for high resolution haptic textures:

| Haptic map | Format | Range | Resolution |
| --- | --- | --- | --- |
| stiffness | 16-bit | 0-10000 N.s −1 /m.s −1 | 0.15 N.s −1 /m.s −1 |
| friction | 16-bit | ±100 | 0.003 |
| vibrotactileTexture | 16-bit | ±100 | 0.0015 |
| temperature | 16-bit | [-100:+150]°C | 0.004°C |
| vibration | 8-bit (amplitude) 8-bit (frequency) | [0-1] [0-300] Hz | 0.004 1.17Hz |
| custom | 16-bit | 0-65535 | 1 |

For the high-resolution texture, values of each texture map pixel are divided in two: the first byte contains the magnitude value and the second byte contains the frequency.

As detailed in Table G.3-5, at the glTF file level, the MPEG_haptic_material extension contains an array of haptic materials. The data for each element of the array is detailed in Table G.3-6, it contains multiple lists of textures (each combined with an enumeration) associated to different haptic properties.

Table G.3-5: Semantic description of the MPEG_haptic_material extension at the glTF file level

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| materials | Array<MPEG_haptic_material.material> | N/A | Provides a list of haptic materials at the glTF file level to enable haptic support. |

The following table describes the list of haptic properties of the extension:

Table G.3-6: Semantic description of the MPEG_haptics_material.material items of the materials array from the glTF file level MPEG_haptic_material extension

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| haptic | integer | N/A | Index to an element of the hapticObjects array of the MPEG_haptic extension. This is used for “Reference” textures to access the haptic information. |
| stiffness | array<enumeration, textureInfo> | NULL | It determines the perceived stiffness of a surface. Which means the force perceived by the user opposed to the normal penetration of a material by a body part. It is described with a texture storing the stiffness coefficients. The suggested rendering model is : F = kx where k is the value of stiffness for the displacement x along the asset stiffness function. This model is valid for an isotropic material. |
| friction | array<enumeration, textureInfo> | NULL | It indicates the perceived friction, which is a force opposing the movement of a body part sliding on a surface. It is described with a texture storing the coefficient of friction. The suggested rendering model is: F_f = mu * Fn where mu is the coefficient of friction, and Fn is the normal applied force by the body part on the surface. |
| vibrotactileTexture | array<enumeration, textureInfo> | NULL | It indicates the perceived texture by a body part while sliding on a surface. |
| temperature | array<enumeration, textureInfo> | NULL | It indicates the perceived temperature of an object. It is described with a texture storing the temperature distribution. |
| vibration | array<enumeration, textureInfo> | NULL | It indicates a vibration signal. It is described with a texture storing the amplitude and / or frequency of the signal. |
| custom | array<enumeration, textureInfo> | NULL | Texture containing custom haptic data. |

As detailed in Table G.3-7 at the mesh level, the MPEG_haptic_material extension contains a single reference to an element of the materials array detailed in Table G.3-5.

Table G.3-7: description of the MPEG_Haptic_material extension at the mesh level

| Name | Type | Default | Description |
| --- | --- | --- | --- |
| hapticMaterialIndex | Integer | N/A | Reference to an item in the materials array of the MPEG_haptic_material extension defined at glTF file level |

### G.3.4	Processing Model

When a scene description file becomes available, the Presentation Engine

- parses the related glTF file.

- detects if the MPEG_haptic and MPEG_haptic_material extension is used.

- identifies nodes associated with haptics by analyzing the haptic actions defined in the interactivity extension.

- identifies haptic media in the MPEG_media extension where the autoplay property is set to true or the startTime property is defined.

At runtime:

- the presentation engine renders the identified haptic media based on the autoplay or startTime property.

- when behaviors launch a haptic action following the trigger activation, the presentation engine retrieves the haptic data specified in the associated haptic object through the MAF API.

  - the presentation engine then renders the data based on the properties specified in the associated haptic action.

Figure G.4.4-1 provides a simplified example of a glTF scene combning the interactivity and haptics extensions. Haptic information is stored at the glTF file level with the MPEG_haptic and MPEG_haptic_material extensions.

Figure G.4.4-1: Example of glTF scene using MPEG_haptic and MPEG_haptic_material extensions

The relation between a node in the scene and haptic data from the MPEG_haptic extension is established in the MPEG_interactivity extension through haptic actions. Interactive haptic feedback is produced by defining behaviors with triggers (e.g collisions, proximity, etc.) and haptic actions. For each node in a haptic action, the associated Haptic data is defined either through a reference to an element of the MPEG_Haptic extension (action A1 and node1 in Figure G4.4-1) or through a MPEG_haptic_material attached to a mesh of the node (action A2 and node2 in Figure G4.4-1). When a haptic action is triggered, the associated haptic data shall be rendered according to the properties specified in the action.

Examples of how haptics would be driven by spatial displacement actions and additional parameters to tune the rendering are provided bellow.

| Action | Default haptic behavior | Override haptic parameters |
| --- | --- | --- |
| None | N/A | N/A |
| Free movement | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, fix rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Free position, pivot rotation | Read Texture based on distance traveled in space | Rescale the haptics rendering independent variable by an arbitrary length. |
| Sliding | Read Texture based on slider displacement distance as input. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Rotation around pivot | Read Texture based on distance traveled during the rotation. | Rescale the haptics rendering independent variable by an arbitrary length. |
| Button | Read force feedback curve based on the current depth of the button. | Rescale the haptics rendering independent variable by an arbitrary length. |

Spatial displacement does not need to drive time-based haptic feedback (such as vibrations).

========================== Next Change ===========================

Add the following Annex H

Annex H
Reference avatar
(Informative)

## H.1	Introduction

This Annex provides a description of a reference topology for the representation of humanoid avatars. The number of vertices and triangles is defined, with the associated semantics. Different levels of detail (LoD) from the same topology are also defined. The associated glTF files are downloadable from MPEG Content Repository (https://mpegfs.int-evry.fr/mpegcontent/ws-mpegcontent/MPEG-I/Part14-SceneDescriptions/MPEGRefAvatarModel-Morgan)

The reference avatar detail in this informative document is identified by the URN “urn:mpeg:sd:2023:avatar” as an avatar type.

## H.2	Body Model

### H.2.1	 General

The reference body avatar (Figure H.1) comes with its own mesh topology modeled as a female (right side) or a male body (left side). The body base mesh is composed of three levels of details. As described in Figure H.1, the density of the vertex is not the same for the body and the face, because the latter requires more accuracy for realism.

The different parts are described in more detail in the following sections.

Female

Male

Figure H.1: Full body mesh topology (Right side: female version, Left side: male version). The neutral genre face is zoomed for better visualization (right picture).

### H.2.1	Level of Details

The reference body avatar is structured in three levels of details (mesh resolution): Low, Medium and High (see Figure H.2). The goal of these different resolutions is to ensure that the mesh topology is supported on the runtime platform (mobile, PC, HMD etc.).

Each level is described and referenced as:

- High Resolution (referenced as Morgan_HD)

  - 53,695 vertices and 56,496 quad faces (106,952 triangular faces).

- Medium Resolution (referenced as Morgan_MD)

  - 13,225 vertices and 13,196 quad faces (26,356 triangular faces)

  - Corresponds to 25% of the HD mesh

- Low Resolution (referenced as Morgan_SD)

  - 5,366 vertices and 5,300 quad faces (10,571 triangular faces).

  - Corresponds to 90% of the HD mesh

The mesh decimation ensures that vertices positions are consistent between levels. All SD mesh vertices are on the MD mesh and all MD mesh vertices are on the HD mesh.

Figure H.2: Levels of details of Morgan - From left to right: High (50k), Medium (12k) and Small (5k).

The associated files are named morgan_SD.gltf, morgan_MD.gltf and morgan_HD.gltf.

### H.2.3	Body Semantics

The body is divided into 18 semantic areas allowing to directly refer to them for specific use cases and applications.

Figure H.3, Figure H.4, and Figure H.5 describes those 18 areas for respectively the front body, the back of the body and the head. The corresponding semantics and vertex coordinates are given by Table H.1. The numbering and order are the same as in the reference glTF file provided. It allows easy access to parts of the body by any applications, even without knowing the model.

The provided semantic list relates to the High Resolution level, aka Morgan_HD.

Figure H.3: front view of body semantic areas (left: areas, right: wireframe areas).

Figure H.4: back view of body semantic areas (left: areas, right: wireframe areas).

Figure H.5: front view of the head. Top line: segmentation areas, bottom line: wireframe areas. (Left) complete head view with the face (white), left eye (red) and right eye (olive green) - (Middle) internal view without the face with the mouth bag (blue) - (Right) view without the face and the mouth bag - upper jaw (light green) and lower jaw (purple).

Table H.1: Semantics table – The visual and geometric details of each semantic area are provided. For each of them, the number of vertices, quad and triangular faces are provided with the ID ranges covered by the areas.

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Full | None |  | 53698 | [0:53697] | 46244 | 97296 | [0:46243] |  |
| - Upper Body | / | / | 46340 | [0:46339] | 45504 | 93584 | [0:45503] |  |
| - - Head | / | / | 36584 | [0:36583] | 36184 | 72328 | [0:36183] |  |
| - - - Face |  | (1, 1, 1) | 6873 | [0:6872] | 6785 | 13570 | [0:6784] |  |
| - - - Back/Neck/Ears |  | (0, 0, 0) | 4535 | [6873:11407] | 4454 | 8908 | [6785:11238] |  |
| - - - Mouth Bag |  | (0, 0.5, 0.5) | 791 | [11408:12198] | 757 | 1514 | [11239:11995] |  |
| - - - Lower Jaw |  | (0.5, 0, 0.5) | 10440 | [12199:22638] | 10380 | 20760 | [11996:22375] |  |
| - - - Upper Jaw |  | (0.5, 1, 0.5) | 11107 | [22639:33745] | 11040 | 22080 | [22376:33415] |  |
| - - - Eye Left |  | (0.5, 0, 0) | 1419 | [33746:35164] | 1384 | 2748 | [33416:34799] |  |
| - - - Eye Right |  | (0.5, 0.5, 0) | 1419 | [35165:36583] | 1384 | 2748 | [34800:36183] |  |
| - - Thorax | / | / | 1828 | [36584:38411] | 1636 | 3272 | [36184:37819] |  |
| - - - Chest |  | (0.5, 1, 0) | 936 | [36584:37519] | 892 | 1784 | [36184:37075] |  |
| - - - Upper Back |  | (0.5, 0, 1) | 486 | [37520:38005] | 426 | 852 | [37076:37501] |  |
| - - - Shoulders | / | / | 406 | [38006:38411] | 318 | 636 | [37502:37819] |  |
| - - - - Shoulder Front | / | / | 182 | [38006:38187] | 138 | 276 | [37502:37639] |  |
| - - - - - Shoulder Front Left |  | (0.5,1,1) | 91 | [38006:38096] | 69 | 138 | [37502:37570] |  |
| - - - - - Shoulder Front Right |  | (0.5,0.5,1) | 91 | [38097:38187] | 69 | 138 | [37571:37639] |  |
| - - - - Shoulder Back | / | / | 224 | [38188:38411] | 180 | 360 | [37640:37819] |  |
| - - - - - Shoulder Back Left |  | (0.5,0.5,1) | 112 | [38188:38299] | 90 | 180 | [37640:37729] |  |
| - - - - - Shoulder Back Right |  | (0.5,1,1) | 112 | [38300:38411] | 90 | 180 | [37730:37819] |  |
| - - Arm Left | / | / | 3553 | [38412:41964] | 3472 | 7136 | [37820:37681] |  |
| - - - Upper Arm Left |  | (1, 0.5, 1) | 320 | [38412:38731] | 288 | 576 | [37502:37789] |  |
| - - - Lower Arm Left |  | (1, 0, 0) | 512 | [38732:39243] | 480 | 960 | [37790:38269] |  |
| - - - Hand Left |  | (0, 1, 0) | 2721 | [39244:41964] | 2704 | 5408 | [38270:40973] |  |
| - - Arm Right | / | / | 3553 | [41965:45517] | 3472 | 7136 | [40974:44445] |  |
| - - - Upper Arm Right |  | (1, 0, 1) | 320 | [41965:42284] | 288 | 576 | [40974:41261] |  |
| - - - Lower Arm Right |  | (0, 1, 1) | 512 | [42285:42796] | 480 | 960 | [41262:41741] |  |
| - - - Hand Right |  | (0, 1, 0.5) | 2721 | [42797:45517] | 2704 | 5408 | [41742:44445] |  |
| - - Abdomen | / | / | 822 | [45518:46339] | 740 | 3712 | [45504:46243] |  |
| - - - Abdomen Front |  | (0, 0, 1) | 420 | [45518:45937] | 382 | 764 | [45504:45885] |  |
| - - - Lower Back |  | (0.5, 0.5, 0.5) | 402 | [45938:46339] | 358 | 716 | [45886:46243] |  |
| - Lower Body | / | / | 7358 | [46340:53697] | 7054 | 14108 | [45504:52557] |  |
| - - - Pelvis | / | / | 1036 | [46340:47375] | 900 | 1800 | [45504:46403] |  |
| - - - - Pelvis Front | / | / | 512 | [46340:46851] | 443 | 886 | [45504:45946] |  |
| - - - - - Pelvis Front Left |  | (1,0,0.5) | 266 | [46340:46605] | 231 | 462 | [45504:45734] |  |
| - - - - - Pelvis Front Right |  | (0,0,0.5) | 246 | [46606:46851] | 212 | 424 | [45735:45946] |  |
| - - - - Pelvis Back | / | / | 524 | [46852:47375] | 457 | 914 | [45947:46403] |  |
| - - - - - Pelvis Back Left |  | (0,0,0.5) | 252 | [46852:47103] | 219 | 438 | [45947:46165] |  |
| - - - - - Pelvis Back Right |  | (1,0,0.5) | 272 | [47104:47375] | 238 | 476 | [46166:46403] |  |
| - - Leg Left | / | / | 3161 | [47162:50322] | 3077 | 6154 | [46244:49320] |  |
| - - - Upper Leg Left |  | (0.5, 0.5, 1) | 681 | [47162:47842] | 650 | 1300 | [46244:46893] |  |
| - - - Lower Leg Left |  | (1, 0.5, 0) | 815 | [47843:48657] | 780 | 1560 | [46894:47673] |  |
| - - - Foot Left |  | (1, 1, 0) | 1665 | [48658:50322] | 1647 | 3294 | [47674:49320] |  |
| - - Leg Right | / | / | 3161 | [50323:53483] | 3077 | 6154 | [49321:52397] |  |
| - - - Upper Leg Right |  | (1, 0.5, 0.5) | 681 | [50323:51003] | 650 | 1300 | [49321:49970] |  |
| - - - Lower Leg Right |  | (0, 0.5, 0) | 815 | [51004:51818] | 780 | 1560 | [49971:50750] |  |
| - - - Foot Right |  | (1, 1, 0.5) | 1665 | [51819:53697] | 1647 | 3294 | [50751:46243] |  |

### H.2.4	Base UV

The reference avatar comes with its own UV coordinates. It is based on UDIM (U-Dimension, an enhancement to the UV mapping and texturing workflow that makes UV map generation easier and assigning textures simpler) which consists of a tile system where each tile is a different texture with its own UV space.

The following look-up table provides the link between the tile on the bottom UDIM map and the corresponding body parts:

| Upper Jaw | Lower Jaw | Right Eyeball |  | Left Iris |
| --- | --- | --- | --- | --- |
| Face | Body | Left Eyeball | - Right Lens - Left Lens | Right Iris |

The UDIM map for the full morgan’s body is illustrated in Figure H.6, and provided with the associated file morgan_HD.gltf. The medium resolution morgan_MD.gltf and low resolution morgan_SD.gltf of the mesh also comes with their similar but decimated UDIM map.

Figure H.6: UDIM map of the full body - It goes from UV space (0-1/0-1) on the bottom left corner to UV space (4-5/1-2) to the upper right corner.

### H.2.5	Base Skeleton

Skeleton’s joints are identified by names and hierarchy with the following structure and naming convention to enable the identification of the avatar skeleton (see complete nomenclature in Table H.2).

#### H.2.5.1 Complete Skeleton

Morgan’s skeleton is composed of 63 joints, and Figure H.7 precisely depicts the place of the skeleton’s joints in the hierarchy.

| Body Hierarchy (25 joints) | Left hand hierarchy (19 joints) | Right hand hierarchy (19 joints) |
| --- | --- | --- |
| Hips --- Spine --- --- Chest --- --- --- UpperChest --- --- --- --- Shoulder_Left --- --- --- --- --- UpperArm_Left --- --- --- --- --- --- LowerArm_Left --- --- --- --- --- --- --- Hand_Left --- --- --- --- --- --- --- --- See below --- --- --- --- Shoulder_Right --- --- --- --- --- UpperArm_Right --- --- --- --- --- --- LowerArm_Right --- --- --- --- --- --- --- Hand_Right --- --- --- --- --- --- --- --- See below --- --- --- --- Neck --- --- --- --- --- Head --- --- --- --- --- --- Eye_Left --- --- --- --- --- --- Eye_Right --- --- --- --- --- --- Jaw --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- --- --- --- Toes_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right --- --- --- --- Toes_Right | (Hand_Left) --- ProximalThumb_Left --- --- IntermediateThumb_Left --- --- --- DistalThumb_Left --- ProximalIndex_Left --- --- IntermediateIndex_Left --- --- --- DistalIndex_Left --- --- --- --- TopIndex_Left --- ProximalMiddle_Left --- --- IntermediateMiddle_Left --- --- --- DistalMiddle_Left --- --- --- --- TopMiddle_Left --- ProximalRing_Left --- --- IntermediateRing_Left --- --- --- DistalRing_Left --- --- --- --- TopRing_Left --- ProximalLittle_Left --- --- IntermediateLittle_Left --- --- --- DistalLittle_Left --- --- --- --- TopLittle_Left | (Hand_Right) --- ProximalThumb_Right --- --- IntermediateThumb_Right --- --- --- DistalThumb_Right --- ProximalIndex_Right --- --- IntermediateIndex_Right --- --- --- DistalIndex_Right --- --- --- --- TopIndex_Right --- ProximalMiddle_Right --- --- IntermediateMiddle_Right --- --- --- DistalMiddle_Right --- --- --- --- TopMiddle_Right --- ProximalRing_Right --- --- IntermediateRing_Right --- --- --- DistalRing_Right --- --- --- --- TopRing_Right --- ProximalLittle_Right --- --- IntermediateLittle_Right --- --- --- DistalLittle_Right --- --- --- --- TopLittle_Right |

#### H.2.5.2 	Example of Skeletal Hierarchies

Table H.0-1: Skeleton joints nomenclature

| Simplified |  | Simplistic |
| --- | --- | --- |
| Body Hierarchy (24 joints) |  | Body Hierarchy (15 joints) |
| Hips --- Spine --- --- UpperChest --- --- --- Shoulder_Left --- --- --- --- LowerArm_Left --- --- --- --- --- Hand_Left --- --- --- --- --- --- ProximalThumb_Left --- --- --- Shoulder_Right --- --- --- --- LowerArm_Right --- --- --- --- --- Hand_Right --- --- --- --- --- --- ProximalThumb_Right --- --- --- --- Neck --- --- --- --- --- Head --- --- --- --- --- --- Eye_Left --- --- --- --- --- --- Eye_Right --- --- --- --- --- --- Jaw --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- --- --- --- Toes_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right --- --- --- --- Toes_Right |  | Hips --- UpperChest --- --- Shoulder_Left --- --- --- LowerArm_Left --- --- --- --- Hand_Left --- --- Shoulder_Right --- --- --- LowerArm_Right --- --- --- --- Hand_Right --- --- --- Head --- UpperLeg_Left --- --- LowerLeg_Left --- --- --- Foot_Left --- UpperLeg_Right --- --- LowerLeg_Right --- --- --- Foot_Right |

Figure H.7: Position of the joints for the MPEG reference geometrical model.

## H.3	Face Model

### H.3.1	 General

The reference facial avatar consists of a base topology and a gender-neutral morphology. It corresponds to the “head area” in the semantics Table H.1 and its geometry is displayed in Figure H.8. The facial morphology comes from a combined dataset of high-resolution face scans (males and females) mapped to the same base topology using 3D reconstruction facial technique, and thus does not correspond to an existing person nor cannot allow the reconstruction of one person's image.

Figure H.8: MPEG Morgan faces (right to left: front, back and right profile).

### H.3.2	Base Mesh

The corresponding topology is depicted in Figure H.8. The base face mesh is composed of 36,584 vertices / 36,362 quad faces (and 72,784 triangular faces). As illustrated on Figure H.9, four internal parts are added:

- a mouth bag (Figure H.9a)

- an upper (Figure H.9b) and lower jaw (H.9c)

- two eyeballs (Figure H.HH.9d)

Topological details of these specific parts can be found in the table H.1.

Figure H.9: MPEG Morgan’s face details 
((a) mouth bag, 
(b) upper jaw, 
(c) lower jaw, 
(d) eyeballs).

### H.3.3	Facial Blend Shapes

The facial expression shapes naming is close but not equal to the FACS naming convention defined and used in anatomy to classify human facial motions.

Shapes are separated into Left (“_L”) and Right (“_R”) components. And sometimes these last components have been split into (“_1”) and (“_2”) sub---components to increase the precision.

The shapes of Morgan are given in table H.3.

Table H.3: MPEG Morgan’s facial blend shapes

| AU | FACS Name | Morgan shape |
| --- | --- | --- |
| 1 | Inner Brow Raiser | AU1_Inner_Brow_Raiser |
| 2 | Outer Brow Raiser | AU2_Outer_Brow_Raiser_L1 + L2 + R1 + R2 |
| 4 | Brow Lowerer | AU4_Brow_Lowerer_L + R |
| 5 | Upper Lid Raiser | AU5_Upper_Lid_Raiser_L + R |
| 6 | Cheek Raiser | AU6_Cheek_Raiser_L + R |
| 7 | Lig Tightener | AU7_Lid_Tightener_L + R |
| 9 | Nose Wrinkler | AU9_Nose_Wrinkler_L + R |
| 10 | Upper Lip Raiser | AU10_Upper_Lip_Raiser_L + R |
| 11 | Nasolabial Deepener | AU11_Nasolabial_Deepener_L + R |
| 12 | Lip Corner Puller | AU12_Lip_Corner_Puller_L + R |
| 14 | Dimpler | AU14_Dimpler_L + R |
| 15 | Lip Corner Depressor | AU15_Lip_Corner_Depressor_L + R |
| 16 | Lower Lip Depressor | AU16_Lower_Lip_Depressor_L |
| 17 | Chin Raiser | AU17_Chin_Raiser |
| 18 | Lip Pucker | AU18_Lip_Pucker |
| 20 | Lip Stretcher | AU20_Lip_Stretcher_L + R |
| 22 | Lip Funneler | AU22_Funneler |
| 23 | Lip Tightener | AU23_Lip_Tightener |
| 24 | Lip Pressor | AU24_Lip_Pressor |
| 26 | Jaw Drop | AU26_Jaw_Drop |
| 27 | Mouth Stretch | AU27_Mouth_Stretcher |
| 28 | Lip Suck | AU28_Lip_Suck_Low + Up |
| 29 | Jaw Thrust | AU29_Jaw_Thrust |
| 30 | Jaw Sideways | AU30_Jaw_Sideways_L + R |
| 31 | Jaw Clencher | AU31_Jaw_Clencher |
| 34 | Cheek Puff | AU34_Cheek_Puff_L + R |
| 38 | Nostril Dilator | AU38_Nostril_Dilator |
| 43 | Eyes Closed | AU43_Eyes_Closed_L + R |
| 61 | Eyes Turn Left | AU61_Eyes_Turn_Left |
| 62 | Eyes Turn Right | AU62_Eyes_Turn_Right |
| 63 | Eyes Up | AU63_Eyes_Up |
| 64 | Eyes Down | AU64_Eyes_Down |

All blendshapes are accessible as individual glTF files names with the corresponding AU name and also in a more complete glTF file name morgan_rigged.gltf.

### H.3.4	Facial Landmarks

A set of 68 semantic facial landmarks is provided (see Figure H.10) for animation and processing. It can be use to drive mesh retargeting algorithm (retopoligization) or to link facial performance tracking solution to Morgan’s face.

The landmarks are accessible in the file morgan_landmarks.txt.

Right Jaw Line				Left Jaw Line

Mouth					Nose

Left Eye 				Right Eye

Figure H.10: MPEG Morgan’s landmarks (global face and close views).

### H.3.5	Teeth Model

The facial rig also includes an upper and a lower jaw as illustrated in Figure H.11. The lower jaw (right) is composed of 10, 440 vertices and 10, 380 quad faces (20, 760 triangular faces). The upper jaw (left) is composed of 11, 107 vertices and 11, 040 quad faces (22, 080 triangular faces). They both come with their own UV coordinates and associated textures.

Here are the corresponding lines of the semantic Table H.1 to access the upper and lower jaws from the original glTF file.

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lower Jaw |  | (0.5, 0, 0.5) | 10440 | [12199:22638] | 10380 | 20760 | [12174:22553] |
| Upper Jaw |  | (0.5, 1, 0.5) | 11107 | [22639:33745] | 11040 | 22080 | [22554:33593] |

Upper Jaw			Lower Jaw

Upper UV Map		           Upper Albedo Map          	 Lower UV 		Lower Albedo Map

Figure H.11: MPEG Morgan’s teeth model.

### H.3.6	Eye Model

The reference avatar comes with a complete eye model depicted in Figure H.12. The eye model follows an anatomical eye model and is composed of three elements: the sclera, the iris, and the lens.

The polygon count is the following:

- lens (201 vertices and 200 quad face (380 triangular faces)

- iris (928 vertices and 896 quad face (1792 triangular faces)

- sclera (290 vertices and 288 quad face (576 triangular faces)

The following extract of the semantic Table H.1 shows left and right eye from the referenced glTF file. The order of ID ranges (vertex or face) is: lens  iris  sclera.

| Semantic Name | Color | Color (r,g,b) | # of vertices | Vertex ID | # of quad faces | # of tri faces | Face ID (quad) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Eye Left |  | (0.5,0,0) | 1419 | [33746:35164] | 1384 | 2748 | [33594:34977] |
| Eye Right |  | (0.5,0.5,0) | 1419 | [35165:36583] | 1384 | 2748 | [34978:36361] |

Sclera				Iris			 	Lens

Figure H.12: MPEG Morgan’s eyes model.

The sclera corresponds to the eye globe and is displayed as the white area (with blood veins) of the eye. The iris is located inside the sclera and corresponds to the eye diaphragm and is displayed as the unique eye colored pattern of the eye. The lens is located inside the sclera and behind the iris. It corresponds to the lens that projects the light on the cornea (internal back border of the eye globe where the image is created).

The eye model comes with two blend shapes for each eye:

- Pupil_Dilatation – it controls the aperture of the pupil of the eye

- Sclera_Flatness – it controls the degree of flatness of the cornea

Figure H.13: Pupil Dilation from low (left) to high (right) and Cornea Flatness from low (left) to high (right).

## H.4	Interactive Format Description for Interactive Scenes

### H.4.1	Model Format

#### This section illustrates how to signal a type of avatar that can be used under the node extension “MPEG_node_avatar” as additional properties in glTF format. To this end it defines the “extras” of “MPEG_node_avatar” that contains the additional properties describing the Morgan model.H.4.1.1	Processing Model and Semantics

The application can have the knowledge of what avatar to reconstruct or render given the “model” attribute.

Table H.4.1.1: Morgan model description

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| morgan_model | Object | O | N/A | Avatar model (see Table H.4.1.2). If present refers to the avatar format semantics. |

Table H.4.1-2 – Avatar model description.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | M | “” | User-defined name of the 3D object format. |
| semantical_type | string | M | “” | A string to indicate which type of avatar the model refers to, see Table H.4.1.3. A combination of several types is possible e.g., “Humanoid/Aquatic” refers to a human-like avatar with swimming proprieties. |
| resolution | array | M | [“”] | An array of strings that semantically describes each alternative in the “MPEG_media.media” extension reference by the source element in terms of format resolution e.g., low-resolution, medium-resolution, or high-resolution. |
| source | integer | M |  | Index to a “MPEG_media.media” extension that defines external media. The “alternative” propriety will be index aligned with the resolution propriety to facilitate the application choice in terms of capability. |

Table H.4.1-3: The table presents the higher-level semantical representation depending on styles of avatars and format representation.  Several items from this list or from other more detailed representations can be combined to facilitate the client application to parse the format of an avatar.

Table H.4.1-3 Semantical type of an Avatar

Semantical type

Humanoid

Non-Humanoid

Aquatic

Aerial

Terrestrial

Subterranean

Arboreal

Other

The source attribute references a “MPEG_media.media” extension in ISO/IEC 23090-14 Scene description specification, with the intent to have the object “MPEG_media.media.alternative” defined to match the resolution. The “alternative” propriety contains the “uri” and “mimeType” that are used to reference any media input format.

#### H.4.1.2	glTF Schema Examples

The following glTF example will instantiate a “MPEG_node_avatar” in clients that support this extension, and otherwise, fall back to a standard “MPEG_node” without the semantical avatar node extension. It includes descriptive elements of the Morgan presented in this annex, whose attributes are instantiated within the “extras” of “MPEG_node_avatar”.

"MPEG_media":[ { "alternatives":[ { "uri": "morgan_reference_low.gltf" "mimeType": "mesh/gltf" }, { "uri": "morgan_reference_medium.gltf" "mimeType": "mesh/gltf" }, { "uri": "morgan_reference_high.gltf" "mimeType": "mesh/gltf" } ] } ] "nodes": [ { "extensions": { "MPEG_node_avatar": { "isAvatar": true, "type": "urn:mpeg:sd:2023:avatar", "mappings": [ { "path": "full_body/upper_body/arm_left", "node": 1 }, { "path": " full_body/upper_body/arm_right", "node": 1 } ], "extras" : { "morgan_model" : { "name": "MORGAN", "semantical_type": "humanoid/terrestrial", "resolution": ["low-resolution","medium-resolution","high-resolution"] "source": 0 } } } } } ]

### H.4.2	Metadata Format

This section presents an attribute to identify an avatar socially to facilitate interactions with other avatars or objects in the scene. This attribute contains the identity information of an avatar, such as name, gender, and age. The identity information is not necessarily the accurate information of a real user, but instead an identity attributed to the user’s avatar. The age attribute has additional significance in this proposal because it can be linked with parental permission and content access depending on the application side.

This attribute enables the creation of several avatar representations, which facilitates the description of an avatar in the scene and identifies the type of interactivity with the scene such an avatar have.

These metadata contain sensitive information. A service provider using them should deploy adequate data protection mechanisms to guarantee their security and privacy.

Table H.4.2-1: Semantics of the Morgan metadata property

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| morgan_metadata | object | O | “” | Information about the avatar allows social identification in virtual environments. Table H.4.2-2 provides the semantics of this object. |

Table H.4.2-2: Semantics of the proposed object extension "metadata".

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| name | string | M |  | Name of the avatar |
| age | integer | M |  | Age of the avatar |
| gender | string | O | “” | Gender of the avatar |
| disability | array | O | [“”] | Avatar’s physical disability, to indicate any physical impairment, for example, brain injury, spinal cord injuries, amputations, musculoskeletal, hearing loss or visual impairment. |
| capability | array | O | [“”] | A list of avatar abilities, such as the possibility to fly, walk, grasp, jump, and others. |
| personality | array | O | [“”] | A list of characteristics or qualities for an avatar individual character, for example, shy, courageous. As another example, the Myers-Briggs type can also be used. |
| emotion | array | O | [“neutral”] | An emotional state that reflects an avatar state, for example, admiration, anger, fear, sadness, joy, and others. Different lists of emotions categories are possible, such as Robert Plutchik’s categories. |
| social | array | O | [“”] | A type of social behavior that the avatar is able to perform, such as conversation or interaction with other 3D environment elements and avatars. |
| parental | array | O | [“”] | Permission content to be accessed by the avatar. This can be any standard method to infer parental permissions such as the ones used in video games and the streaming industry. |

#### H.4.2.1	glTF Schema Examples

The extension “morgan_metadata” specifies by default the avatar attributes that define a base representation of a user avatar preference. This extension allows the possibility to identify and generate user-specific avatars.

The following glTF example will instantiate a “MPEG_node_avatar” in clients that support this extension, and otherwise, fall back to a standard “MPEG_node” without the metadata semantical extension.

"nodes": [ { "extensions": { "MPEG_node_avatar": { "isAvatar": True, "type": "urn:mpeg:sd:2023:avatar", "extras": { "morgan_metadata" : { "name": "Red", "aga": 18, "gender": "N/A", "disability": ["colorblind", "hearing_loss"], "capability": ["walk", "run", "grasp"], "personality": ["happy"], "emotion": ["admiration", "joy"], "social": ["interaction", "conversation"], "parental": ["18", "bad_language", "violence", "fear"], } } } } } ]

## H.4.3	Interactive Actions Format

We enhance the actions property of the “MPEG_scene_interactivity” extension by introducing new actions falling in a new “ACTION_SET_AVATAR” action type and grouped in action lists. Each list is uniquely identified by a URN and referenced by the “MPEG_scene_interactivity.avatarAction” property that is required when “MPEG_scene_interactivity.type” is set to “ACTION_SET_AVATAR”.

The semantics of the avatar-related actions are provided below and identified by the following URNs:

- “urn:mpeg:sd:2023:avatar:actionslist:social”

- “urn:mpeg:sd:2023:avatar:actionslist:restricted”

- “urn:mpeg:sd:2023:avatar:actionslist:parental”

- “urn:mpeg:sd:2023:avatar:actionslist:speech”

- “urn:mpeg:sd:2023:avatar:actionslist:capabilities”

- “urn:mpeg:sd:2023:avatar:actionslist:disabilities”

Table H.4.3.1: Semantical description of new action properties.

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| if (avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:social”){ |  |  |  |  |
| authorised | array | M |  | One or more elements of Table H.4.3-2 define the type of social actions. |
| } |  |  |  |  |
| if (avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:restricted” ){ |  |  |  |  |
| permission_id | string | M |  | Unique string identifier that restricts interaction between nodes without an equal permission_id. |
| } |  |  |  |  |
| if (avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:parental”){ |  |  |  |  |
| age | number | M |  | One element of Table H.4.3-4 that define the minimum age recommendation for users given the content of the list of nodes. |
| descriptors | Array | M |  | One or more elements of Table H.4.3-5 that add additional explicit semantics of the content present in the list of nodes. |
| } |  |  |  |  |
| If(avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:speech”){ |  |  |  |  |
| microphone | Boolean | O | 0 | Indicates if the user uses a microphone type of input device for audio. “0” is False and “1” is True. The default value is 0. |
| media | String | O | “” | URI (Uniform Resource Identifier) to media track to play a pre-recorder audio file. |
| } |  |  |  |  |
| If(avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:capabilities” ){ |  |  |  |  |
| capabilities | array | M | [] | One or more elements of Table H.4.3-6 that define the capabilities of an avatar/object. |
| } |  |  |  |  |
| If(avatarAction == “urn:mpeg:sd:2023:avatar:actionslist:disabilities”){ |  |  |  |  |
| disabilities | array | M | [] | One or more elements of Table H.4.3-7 that define the disabilities of an avatar/object. |
| } |  |  |  |  |

Table H.4.3.2: Type of avatar action.

| Action type | Description |
| --- | --- |
| “urn:mpeg:sd:2023:avatar:actionslist:social” | Set action of a node. |
| “urn:mpeg:sd:2023:avatar:actionslist:restricted” | Set permissions of a node. |
| “urn:mpeg:sd:2023:avatar:actionslist:parental” | Set parental and content usage permissions of a node. |
| “urn:mpeg:sd:2023:avatar:actionslist:speech” | Set speech active of a node. |
| “urn:mpeg:sd:2023:avatar:actionslist:capabilities” | Set capabilities of a node. |
| “urn:mpeg:sd:2023:avatar:actionslist:disabilities” | Set disabilities of a node. |

Table H.4.3-3: Type of avatar social actions.

| Social action list | Description |
| --- | --- |
| “avatar_social_conversation” | Allow social speaking with users and enable the ability for conversation if not already enabled. |
| “avatar_social_interaction” | Allow interaction between users. |
| “avatar_social_other” | Any other type of social interaction. |

Table H.4.3-4: Type of avatar age levels.

| “age” list | Description |
| --- | --- |
| 3 | Content suitable for all ages. |
| 7 | Content with scenes or sounds possibly frightening to younger children. |
| 12 | Content with violence of graphic non-realistic characters. |
| 16 | Content with violence that mimics the reality. |
| 18 | Content design for adults only. |

Table H.4.3-5: Type of avatar parental descriptors.

| parental descriptors list | Description |
| --- | --- |
| “violence” | Contains depiction of violence. |
| “bad_language” | Contains bad language. |
| “fear” | Contains pictures or sounds that may be frightening or scary. |
| “gambling” | Contains elements that encourage or teach gambling. |
| “sex” | Contains sexual posturing. |
| “drugs” | Contains the illustration of the use of illegal drugs, alcohol or tobacco. |
| “discrimination” | The game contains depictions of ethnic, religious, nationalistic or other stereotypes likely to encourage hatred |
| “in-game_purchases” | Offers the option of purchase digital services. |

Table H.4.3.6: Capabilities semantics.

| Capabilities list | Description |
| --- | --- |
| “walk” | The ability to walk. |
| “run” | The ability to run. |
| “jump” | The ability to jump. |
| “fly” | The ability to fly. |
| “swim” | The ability to swim. |
| “climb” | The ability to go over, get on, climb to, to descend objects, such as climb to a chair, climb up the stairs, climb down the stairs, climb the wall etc. |
| “grasp” | The ability to hold objects with hands’ like representations. |
| “manipulate” | The ability to interact and change spatial position of 3D objects by using collision or proximity type of detectors. |
| “ride” | The ability to ride a vehicle or animal, such as motorcycles or horses etc. |
| “drive” | The ability to use a vehicle, such as cars or trucks etc. |
| “pilot” | The ability of piloting a vehicle, such as ships or airplanes. |

Table H.4.3.7: Disabilities semantics.

| Disabilities list | Description |
| --- | --- |
| “Cerebral palsy” | A group of disorders that impact a person's ability to move and maintain balance |
| “Spinal cord injuries” | Spina cord injury indicates the damages to any part of the spinal cord or nerves at the end of the spinal canal. Result in permanent loss of strength, sensation, and function (mobility and feeling) |
| “Amputation” | Indicates removal of part of all of a body part that is enclosed by skin |
| “Musculoskeletal injuries” | Refer to the damage of muscular or skeletal systems, which is usually due to strenuous activities. |

## H.4.4	Download

The Morgan reference Avatar is available for download under https://content.mpeg.expert/data/MPEG-I/Part14-SceneDescriptions/Morgan.