MPEG-I immersive audio Encoder Input Format

# Introduction

This annex specifies the MPEG-I 6DoF audio encoder input format (EIF) for the 
MPEG-I 6DoF Audio encoder. The architecture of the evaluation platform can be found in [1].

The encoder input data contains information describing an MPEG-I 6DoF Audio scene. This covers all contents of the virtual auditory scene, i.e. all of its sound sources, and resource data, such as audio waveforms, source radiation patterns, information on the acoustic environment, etc.  The input data also allows to describe changes in the scene. These changes, referred to as updates, can either happen at distinct times, allowing scenes to be animated (e.g. moving objects). Alternatively, they can be triggered manually or by a condition (e.g. listener enters proximity) or be dynamically updated from an external entity.

The annex is organized as follows: First, the meta model of the MPEG-I 6DoF audio encoder input format is defined. Then several specifications of important entities according to the audio requirements in [2] are presented. Annex A defines conventions (e.g. for radiation patterns, geometry, etc.). The specifications in this annex are used to describe the input to 6DoF metadata encoders.

## Files and directories

The audio encoder input data for a single scene consists of a package of files. These files are to be stored in a single directory that has the name of the scene (e.g. FountainMusicVR). A scene’s input data consists of the following files:

- the scene description XML file, and additional files for

- WAV files containing original (un-coded) audio signals,

- SOFA files describing source directivities

- HDF5 files describing granular databases

In a different directory the WAV files containing MPEG-H LC encoded and decoded audio signals are provided. More details of the folder structure can be found in [1]. MPEG-I encoders can access the whole scene described above.

The EIF file declares the scene’s entities and contains references to data that is in the other input files. These input files (actually file pathnames) are referenced in the declaration syntax of the entities. All pathnames are specified relative to the directory containing the EIF file. The EIF file also describes dynamic behavior over time (e.g. animations), if such exists.

# Scene meta model

The general structure of a MPEG-I 6DoF audio scene description is defined by a meta model (see Fig. 1). Note, that this structure is used for the evaluation only and does not represent a possible MPEG-I bitstream. It describes the general anatomy of a scene, without focus on specific entities:

- An audio scene consists of entities (e.g. sources, geometry, resources, etc.).

- Entities are identified by an ID (string).
The format of this ID is arbitrary, but IDs must be unique.

- Entities exist over the entire time of the scene.
They are not created/deleted dynamically, i.e. during runtime.

- Entities can be associated with each other using their IDs
(e.g. a source with a directivity).

- Entities have properties (e.g. position, volume, material, etc.).

- Entities have a specified entity type, determined by their declaration.

- The type of an entity determines its properties and possible modifications
of these (similar to a datatype or class in programming).

When the scene is written down in XML, entities are represented by XML nodes. Additional nodes are used to specify the data. These nodes do not represent entities (with an ID).

Figure 1: Scene meta model

## Behavior over time

An MPEG-I audio scene can vary over time with the following behavior, which is illustrated in Fig. 2

- Entities in a scene cannot be created/deleted dynamically, they exist by declaration,

- Modifiable properties of some entities can change over time.

Changes in the scene are expressed by updates, which are defined as follows:

- An update occurs at a specific point in time and
contains one or more modifications.

- An update, can be initiated by a condition, manually triggered or dynamically updated by an external entity.

- A modification changes one or more properties of a single entity.
Not every property of an entity can be changed (e.g. IDs cannot be changed).

- An update can bundle several modifications for different entities.

- Updates shall be handled atomically.

To clarify, ‘atomically’ means that encoders only observe the final outcome of an entire update, but not its individual modifications. This is essential to allow for changing multiple entities synchronously, e.g. moving several sources coherently.

Figure 2: Scene behavior over time

## Scene declaration

The EIF file consists of a single <AudioScene> node, in which all entities are declared (e.g. sound sources, acoustic environment, geometric elements, audio signals, etc.) and updates are defined. How the elements may be arranged is defined by a syntax tree, shown in Fig. 3. It defines the valid hierarchy (parent-child relations) and multiplicities. Elements visualized by rectangles are EIF entities (with an ID) represented by XML nodes. Rounded rectangles mark additional XML nodes used for defining the elements (e.g. data points). The allowed multiplicities are specified in UML format. 0..* indicates that the parent node may have any number of these child nodes. 1..* indicates that is must at least one child.

The following brief example defines a simple, time-invariant test scene with one MPEG-I Audio Object, that represents a trumpet with directivity. Note, that this example is neither complete nor does it specify any syntax of an ObjectSource or other entities.

<AudioScene>

<AudioStream id=”signal:trumpet”
                file=”armstrong.wav”

aepInputChannels=”0, 1” />

<SourceDirectivity id=”dir:trumpet”

file=”trumpet.sofa” />

<ObjectSource id=”src:trumpet”

position=”2 1.7 -1.25”

orientation=”30 -12 0”

signal=”signal:trumpet”

directivity=”dir:trumpet”

gainDb=”-2”

active=”true” />

</AudioScene>

Figure 3: Structure of an EIF file

## Scene updates

The following types of updates can be defined in an EIF file

- A timed update (L1) is executed by the renderer once at	a fixed, predefined time.

- A conditional update (L2) is initiated by the renderer when a certain condition is met.

- A triggered update (L2) is manually triggered from an external entity (using an OSC message).

- A dynamic update (L3) is an update from an external entity that includes the values of the attributes to be updated.

The attributes and values that an update modifies are defined in the EIF file and known to the encoder. The only exception are dynamic updates, which allow to insert parameter variables for attribute values to be modified. These values are provided to the renderer by the update message. Parameter variables are indicated by ‘$’ followed by a number.

| <Update> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares one or more changes to the audio scene. The update is performed, when - the specified time is reached, or - the condition changed its state to the logical value expressed by fireOn, - the update is triggered by its ID or index by an external entity The fireOn parameter determines whether the update fires when the condition changes from false-to-true (fireOn=”true”) or from true-to-false (fireOn=”false”). This is helpful for if-else type conditional updates. An <Update> node has one or more <Modify> child nodes. |  |  |  |  |
| Child node | Count | Description |  |  |
| <Modify> | >=1 | Modifications (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | O* | none | Identifier |
| index | Integer | O* | none | Index identifying the update (globally unique) |
| time | Value | O* | none | Time when the update is initiated (seconds). Note: Must be less than or equal to the duration attribute of the AudioScene, and the update doesn’t affect the scene before this time stamp. |
| condition | Condition ID | O* | none | Condition |
| fireOn | Boolean | O* | true | Update fires when this state is reached |
| delay | Float >= 0 | O* | 0 | Postpone the update (seconds) |

* in accordance with Table 1

<Update> nodes can only be used in specific ways to make it possible to clearly identify the different update types. Table 1 gives an overview about the types of updates and the viable attributes and variables.

| Update type | <Update> Attributes id, index | <Update> Attribute time | <Update> Attribute condition | <Update> Attribute fireOn, delay | <Modify> Attribute duration | <Modify> Parameter variables $0, $1, $2, … |
| --- | --- | --- | --- | --- | --- | --- |
| Timed update | optional | required | not allowed | not allowed | optional | not allowed |
| Conditional update | optional | not allowed | required | optional | optional | not allowed |
| Triggered update | required | not allowed | not allowed | not allowed | optional | not allowed |
| Dynamic update | required | not allowed | not allowed | not allowed | not allowed | present |

Table 1: Allowed attributes and variables for different types of updates and modifications

| <Modify> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a modification of modifiable parameters of a single entity. The target entity is selected by the id attribute. Following attributes must be attributes of the corresponding entity. The attribute values are assigned the entities property values. For scene updates where attribute values are not available until rendering, typically user interactions, the attribute values are replaced by parameter indicators to specify in which order the attribute values are expected on the renderer interface. Parameter indicators are $0, $1, $2, etc. The ‘duration’ attribute controls how the value change shall be handled. A value of zero indicates that an immediate switch to the new value is intended. Positive nonzero ‘duration’ values represent the timespan for a continuous (e.g. interpolated) changeover. How this is handled and realized is up to the renderer. The ‘duration’ attribute is not allowed for Dynamic updates (See Table 1). Example: <Modify id=”src1” position=”1 2 3” orientation=”-20 5 0” /> sets the attributes position and orientation for the entity with ID src1 |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Target entity to be modified |
| duration | Float >= 0 | see Table 1 | 0 | Period for adapting from the current values to the new values (seconds) |
| * | * | * | * | Attribute of the target entity |

Note, that not every attribute can be changed. Only those entities that have an entity type specification that allows for modification can be modified (labelled ‘M’).

As updates are atomic, the following rules must be met to prevent ambiguities

- No two timed updates may have the same execution time.

- No two conditional updates may refer to the same condition with same firing order.

- No two triggered updates may have the same id or index.

- Any attribute of any entity may only be modified once within one update

Different updates types can be mixed to control an entity. However, scene creators are advised not to mix different update type for the same attribute of an entity, which may result in an undesired behavior. If an entities' attribute is statically animated (i.e. modified by timed updates), it may not be updated with conditional or triggered updates. Note that this distinction is on attribute-level not entity-level.

Figure 4: Example of a moving car modeled by several AudioElements

The following timed updates synchronously move three ObjectSources of a vehicle in motion (see Fig. 4) along a trajectory.

<Update time=”0.2”>

<Modify id=”engine” position=”2.2 1.7 -1.25” />

<Modify id=”tire1” position=”2.2 1.7 0.75” />

<Modify id=”tire2” position=”2.2 1.7 -0.95” />

</Update>

<Update time=”0.4”>

<Modify id=”engine” position=”2.4 1.7 -1.20” />

<Modify id=”tire1” position=”2.4 1.7 0.70” />

<Modify id=”tire2” position=”2.4 1.7 -0.95” />

</Update>

The scene loops at the rate of the scene duration as specified in the AudioScene’s attribute. Timed updates are triggered for every loop of the scene.

The following conditional updates turn on the sources of a car when the listener gets close.

<Box id=”geo:region1” position=”5 0 -5” size=”10 2 10” />

<ListenerProximityCondition id=”cond:listenerNearCar” region=”geo:region1” />

<!—- Turn on the engine sound 100ms after the listener entered

the region. Smoothly activate the source within 50ms. -->

<Update condition=”cond:listenerNearCar” delay=”0.1”>

<Modify id=”engine” 
          duration=”0.05” active=”true” />

</Update>

<!—- Turn on the other sources 100ms later from the engine -->

<Update condition=”cond:listenerNearCar” delay=”0.2”>

<Modify id=”radio” duration=”0.2” active=”true” />

<Modify id=”exhaust” duration=”0.1” active=”true”/>

</Update>

The following triggered updates are used to turn on/off a radio source.

<Update id=”upd:radio_on” index=”0”>

<Modify id=”src:radio” active=”true” />

</Update>

<Update id=”upd:radio_off” index=”1”>

<Modify id=”src:radio” active=”false” />

</Update>

This last example shows how the radio can be positioned dynamically (i.e. at presentation time) using a dynamic update. The position/orientation values are provided when the update is triggered by the external source (e.g. VR/AR engine).

<Update id=”upd:radio_place” index=”2”>

<Modify id=”src:radio” position=”$0 $1 $2” orientation=”$3 $4 $5” />

</Update>

# Entity types

In the following, entities and their properties are defined. These definitions were derived from the
MPEG-I Audio requirements [2].  Each table lists the properties of an entity, their semantic, possible values, etc. The types of the properties are defined in the appendix. The flags show if a property is required ‘R’ (i.e. it must be defined) or ‘O’ (i.e. it can be defined, otherwise it has default value). ‘M’ indicates that a property can be modified using updates (see 2.1). Properties not labelled with an ‘M’ are static and can only be assigned in the entity’s declaration (see 2.2).

## Audio scene

| <AudioScene> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an audio scene. Each scene description XML file must have exactly one AudioScene node. This node marks the root node of the static description of the audio scene. It contains all entities of the scene as children. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| duration | Float | O | 0 | The duration of the scene, in seconds. If the value is zero, then the scene will not loop |
| type | String | O | ‘VR’ | Scene type. Set to ‘VR’ for VR scenes and ‘AR’ for AR scenes. |
| enableLocallyCapturedAudio | Boolean | O | True | Flag to allow locally captured audio to be used in the scene. |
| overrideListenerVoiceDirectivity | ID | O |  | ID of the directivity to be used for the rendering of the listener voice instead of the default provided by the renderer. |

In the evaluation procedure, the scene’s ID is used to identify different scenes for the test. This ID must be unique for all scenes in the evaluation. The type attribute informs encoders whether the scene is intended for AR or VR applications.

## Audio elements

Geometric elements, meshes and primitives, may be used to specify the extent of an audio source, indicating the geometry that the sound is intended to radiate from. Any mesh used must be manifold, and primitives must have all dimensions > 0.

If a geometric element has materials assigned, such that it is also used for acoustic effects, the materials are not taken into consideration for the rendering of the audio source that references it.

When a geometric element is specified for the spatial extent, the position, orientation and cspace of the source is taken to be that of the geometric element. The front and up directions (see

Annex A.3) are taken from the geometric element.

For an ObjectSource, if the user is inside of the extent the sound field is assumed to be homogeneous, and does not change with user position.

For an HOA source the expected rendering behavior is as defined in Table 3.

## ObjectSource

| <ObjectSource> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an ObjectSource which emits sound into the virtual scene. The ObjectSource has a position/orientation in space. The radiation pattern can be controlled by a directivity. If no directivity attribute is present, the source radiates omnidirectional. Optionally it can have a spatial extent, which is specified through a geometric object. If no extent is specified, the source is a point source. The signal component of the ObjectSource must contain at least one waveform. When the signal has multiple waveforms, the spatial layout of these waveforms must be specified in an <InputLayout> subnode (details see 3.2.1.1). |  |  |  |  |
| Child node | Count | Description |  |  |
| <InputLayout> | 0..1 | Signal positioning (required when signal has multiple waveforms) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O | relative | Spatial frame of reference |
| active | Boolean | O, M | true | If true, then render this source |
| gainDb | Gain | O, M | 0 | Gain (dB) |
| refDistance | Float > 0 | O | 1 | Reference distance (m) (see comment below) |
| signal | AudioStream ID or GranularSound ID | R, M |  | Audio stream or granular database |
| extent | Geometry ID | O, M | none | Spatial extent |
| directivity | Directivity ID | O, M | none | Sound radiation pattern |
| directiveness | Value | O, M | 1 | Directiveness (see 3.6.1) |
| aparams | Authoring parameters | O | none | Authoring parameters (see 4.14) |
| mode | Playback mode | O | continuous | Playback mode {“continuous”, “event”} |
| play | Boolean | O, M | False | Playback enabled? |
| vdlMethod | String | O | “spline” | Defines the Variable Delay Line method to be used (“spline”, “linear”, “spline_erLinear”) |
| reverbGainDb | Gain | O, M | 0 | Additional gain (dB) to be used for reverb rendering in all acoustic environments. It shall range between -119.0 to +12.0. |
| muteReverb | Boolean | O, M | False | A flag switching the reverb on or off for the given source |
| grainDescrCoor | Position | O, M | (0 0 0) | The target position within the granular database. All coordinates within [0,1] |
| grainGroupProb | Float, Float | O, M | none | The index and probability of a grain group |

Example:

<!-- This transform marks the pivot point of the car.

It can be used for animations. -->

<Transform id=”tf:car”

position=”23.5 0.0 102.3”

orientation=”-12.4 0 0” />

<!—- The car’s source is place 0.4m above the ground and
         0.8m towards the front (negative z-direction) -->

<ObjectSource id=”src:engine”

position=”0 0.4 -0.8”

signal=”signal:engine”

extent=”geo:car_extent”

vdlMethod=”linear”

reverbGainDb=”-20”

muteReverb=”true”/>

<!-- This box is relative to the origin of the car.

It raises 1.4 meters high from the ground. -->

<Box id=”geo:car_extent” position=”0 0.7 0” size=”1.6 1.4 4.5” />

</Transform>

### Extended ObjectSources with multi-channel signals

An ObjectSource with spatial extent can have a multi-channel AudioStream to give the renderer the possibility to render the ObjectSource with greater realism than what is possible with a mono AudioStream. This can for example be useful when rendering diffused audio sources such as fountains, waterfalls, rivers, breaking waves, etc.

An ObjectSource with an extent is always perceived by the listener in an elevation-azimuth sector from the listener. This sector is determined by the relative position of the ObjectSource with respect to the listener and the extent of the ObjectSource, all in an acoustical perceptual sense. This is exemplified in Figure 5a for an object source with a cylindrical extent where the ObjectSource is in the front-right hemisphere of the listener. The intersection of a plane that is orthogonal to the observation vector to the center of the elevation-azimuth sector and the elevation-azimuth sector specifies a rectangle. This rectangle represents the acoustically perceived horizontal and vertical extent of the ObjectSource by the listener from the position of the listener. As the listener moves around the ObjectSource, moves closer to it or further away from it, this rectangle will be translated, rotated and resized in the world space coordinate system. Figure 5b illustrates this when the cylindrical ObjectSource is positioned in the front-left hemisphere of the listener. But in the x-y coordinate system that has the origin at the center of these perceived extent rectangles, these rectangles are always positioned with the center in the (0, 0) point of the source.

| - observed in the front-right hemisphere of a listener | - observed in the front-left hemisphere of a listener |
| --- | --- |

Figure 5: Object Source with cylindrical extent and “user” alignment

An InputLayout child node of an ObjectSource is composed of an alignment flag and a string, containing positioning mnemonics separated by whitespaces:

| <InputLayout> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of an <ObjectSource> node, it describes the positioning and handling of several associated waveforms. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| alignment | String | R |  | Indicates anchoring mode: - “user” indicates that the labels are anchored by the user viewing direction - “object” indicates that the labels are anchored by the position/orientation of the ObjectSource element |
| positioning | String | R |  | Indicates intended spatial assignment of waveforms |

The alignment attribute defines the way how the waveforms (channels) of the associated audio stream are located/anchored with respect to the source. The positioning attribute is a string, containing mnemonic labels separated by whitespaces where for each waveform a mnemonic label has to be supplied. Nine relative position mnemonics referring to the previously described x-y coordinate system are supported in the as described in Table 2.

| TL top-left | T top | TR top-right |
| --- | --- | --- |
| L left | C center | R right |
| BL bottom-left | B bottom | BR bottom-right |

Table 2: Relative signal channel positions

Alternatively to the anchoring of the labels relative to the listener viewing direction (as described in the initial example in Figure 5 with attribute alignment=”user”), the signal channel relative position labels can be used to indicate the usage of the waveforms for rendering an ObjectSource with size such that it is anchored to certain object in the scene (attribute alignment=”object”). An example is the sound of a Piano with one signal channel predominantly containing lower notes, the other containing predominantly higher notes. In this case, the position labels apply to a rectangle on a plane through the object position (center) that is perpendicular to the orientation of the ObjectSource when looking at its front (the ObjectSource’s ‘orientation’ attribute must be present). During rendering, the positions indicated by the labels are then projected to the user observation plane (plane that is orthogonal to the observation vector), as shown in Figure 6. Please note that this may also imply putting the sources (potentially with extent) ‘behind’ each other (when looking at the Piano from the side, see Figure 6c) or even swapping them (when looking at the Piano from the back).

- Piano with orientation (front), extent geometry and label plane.

(b)  Observed from the front of the Piano			(c)  Observed from the left of the Piano

Figure 6: ObjectSource (piano) with box-shaped extent and “object” alignment

Examples:

<InputLayout alignment=”user” positioning=”L R” />

indicates that 2 waveforms are used to render a horizontal width of a source.

<InputLayout alignment=”user” positioning=”B T” />

indicates that 2 waveforms are used to render vertical width of a source.

<InputLayout alignment=”user” positioning=”BL TL TR BR” />

indicates that 4 waveforms are used to render both horizontal and vertical width of a source.

<ObjectSource id=”src:piano”

position=”1.2 1.0 -0.3”

orientation=”38 0 0”

extent=”geo:piano_extent”

signal=”signal:piano”>

<InputLayout alignment=”object” positioning=”L R” />

</ObjectSource>

indicates that 2 waveforms are used to render left and right of a Piano object.

## HOASource

| <HOASource> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a source which emits sound into the virtual scene, that is described by higher-order Ambisonics (HOA). The HOASource has a position/orientation in space. The HOASource can describe an incoming soundfield or an outgoing soundfield, as denoted by the representation attribute. An internal representation (representation=0) describes a sound field resulting from sources located outside the spatial sampling area (see Fig. 7a). An external representation (representation=1) describes a sound field resulting from sources located inside the spatial sampling area (see Fig. 7b). The spatial sampling area is defined by the extent attribute. It is expected that a HOASource is also rendered outside its region of validity. For an internal HOASource the region of validity is inside the source extent, while for an external HOASource the region of validity is outside of the source extent. If such behavior is not desired, this is signaled t by setting the extentTransform attribute to False. The transitionDistance attribute determines the area in which the listener transitions between external and internal representation. The HOASource can be rendered either in 3DOF or 6DOF, which is specified with the is6DOF Boolean attribute. The specification of this attribute in combination with the attributes extent, extentTransform and cspace makes it possible to specify a range of HOASource rendering behavior that is summarized in Table 3 and exemplified in the examples provided below. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference Cannot be set to ‘user’, when is6DOF==True |
| active | Boolean | O, M | true | If true, then render this source |
| gainDb | Gain | O, M | 0 | Gain (dB) |
| signal | AudioStream ID or GranularSound ID | R, M |  | Audio stream or granular database |
| aparams | Authoring parameters | O | none | Authoring parameters (see 4.14) |
| mode | Playback mode | O | continuous | Playback mode {“continuous”, “event”} |
| play | Boolean | O, M | False | Playback enabled? |
| extent | Geometry ID | O, M | none | Region of validity, spatial extent |
| extentTransform | Boolean | O | True | When an extent is defined, turns on/off the external rendering of an interior source or the internal rendering of an exterior source. |
| transitionDistance | Value | O | 0 | When an extent is defined, determines the transition area between external and internal representation (m) (see Fig. 8). |
| representation | Integer | R |  | Interior or exterior HOA expansion 0 = interior, 1 = exterior |
| is6DoF | Boolean | O, M | False | If True, the source is rendered in 6DoF within its region of validity (or everywhere if no extent is defined). Also enables the following attributes when True: group, refDistance |
| group | ID | O, M | none | Parent HOAGroup |
| refDistance | Float >= 0 | O | 0 - interior representation 1 - exterior representation | Reference distance (m) (see Reference Distance section below) For an internal HOASource the distance is from the origin/center of the extent, while for an external HOASource the distance is from the extent boundary into the region of validity in the direction of the normal vector. |
| vdlMethod | String | O | “spline” | Defines the Variable Delay Line method to be used (“spline”, “linear”, “spline_erLinear”) |
| reverbGainDb | Gain | O, M | 0 | Additional gain (dB) to be used for reverb rendering in all acoustic environments. It shall range between -119.0 to +12.0. |
| muteReverb | Boolean | O, M | False | A flag switching the reverb on or off for the given source |
| grainDescrCoor | Position | O, M | (0 0 0) | The target position within the granular database. All coordinates within [0,1] |
| grainGroupProb | Float, Float | O, M | none | The index and probability of a grain group |

| <HOAGroup> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a group of HOA and optionally associated object sources which represent multiple HOA and optionally associated object sampling points of a sound scene. HOASources in a HOAGroup are meant to be processed as a unit. |  |  |  |  |
| Child node | Count | Description |  |  |
| <InformedSource> | Informed source element | O | none | A single InformedSource in the group. The group can have multiple InformedSource elements. |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| region | Geometry ID | O, M | none | Spatial region of validity |
| coSources | List of ObjectSource IDs | O | none | ObjectSources sampled simultaneously with the HOASources |
| exteriorProjectionRadius | Value | O | 4.0 | Projection radius used for exterior rendering |
| adaptiveExteriorProjectionRadius | Boolean | O | True | If true, set projection radius based on positions of exterior <InformedSource> elements, if present. |

| <InformedSource> |  |  |  |  |
| --- | --- | --- | --- | --- |
| An informed source description for MPHOA rendering. The informed sources represent the known audio sources within the scene. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| listenerThreshold | Value | O | 3.0 | Distance from the listener to the informed source in meters where the informed source is active. 0.0 distance disables the threshold. If not defined in EIF, 3.0 will be used. |
| hoaSourceThreshold | Value | O | 3.0 | Distance from the HOASources to the informed source in meters where the informed source is active. 0.0 distance disables the threshold. If not defined in EIF, 3.0 will be used. |
| priorityValue | Integer | O | 0 | Determines the processing priority between informed sources that are close to each other. |

Figure 7: HOA sound field representations

Figure 8: HOA transition distance

| is6DoF | extent | extentTransform | cspace | HOA Rendering |
| --- | --- | --- | --- | --- |
| False | None | N/A | Relative | 3DoF everywhere in the scene |
| False | None | N/A | User | 0DoF everywhere in the scene |
| False | Defined | True | Relative | 3DoF within the region of validity Renderer dependent outside of the region of validity |
| False | Defined | True | User | 0DoF within the region of validity Renderer dependent outside of the region of validity |
| False | Defined | False | Relative | 3DoF within the region of validity No rendering outside of the region of validity |
| False | Defined | False | User | 0DoF within the region of validity No rendering outside of the region of validity |
| True | None | N/A | Relative | 6DoF everywhere in the scene |
| True | Defined | True | Relative | 6DoF within the region of validity Renderer dependent outside of the region of validity |
| True | Defined | False | Relative | 6DoF within the region of validity No rendering outside of the region of validity |
| True | None/ Defined | True/ False | User | Not valid |

Table 3 - Summary of HOA rendering behavior for combinations of is6DoF and extent/cspace attributes.

Examples:

Example of legacy HOA sources rendered in 3DoF

In this example, the HOA signal is premixed ambience content. With is6DoF set to “False”, it is assumed that the origins of the sounds in the HOA signal are sufficiently far away (or intangible like the wind) such that only 3DoF rendering is required. Note that no extent is specified and cspace is “relative” so that the HOA rendering is fixed to the scene.

<AudioStream id="signal:ambienceHOA"

aepInputChannels="1:16"

file="ambienceHOA.wav"/>

<HOASource id="src:ambience"

signal="signal:ambienceHOA"

position="0, 0, 0"

orientation="30, 0, 0"

cspace="relative"

mode="continuous"

play="true"

representation="0"

is6DoF="False”

vdlMethod=”linear”

reverbGainDb=”-20”

muteReverb=”true”/>

Example of 3DoF HOA with extent

If the HOA signal is music content to be used as background music following the listener, then cspace would be set to “user”. In this case the HOA source’s orientation is always aligned with the listener.

<AudioStream id="signal:musicHOA"

aepInputChannels="1:16"

file="musicHOA.wav"/>

<HOASource id="src:backgroundmusic"

signal="signal:musicHOA"

orientation="0, 0, 0"

cspace="user"

mode="continuous"

play="true"

representation="0"

is6DoF="False"

vdlMethod=”linear”/>

In this example the listener can explore the inside and outside of an aviary. The HOASource includes an HOA signal with bird sounds and an extent with the same geometry as that of the aviary. The HOASource will be rendered as a 3DoF HOA signal when the listener is inside the aviary. It will be up to the renderer to transition from 3DoF to an unspecified rendering as the listener travels the 2 meters of transitionDistance beyond the extent. When the listener is outside the aviary (outside the extent), it is expected that the listener hears the bird sounds as originating from inside the aviary.

<AudioStream id="signal:aviaryHOA"

aepInputChannels="1:16"

file="aviaryHOA.wav"/>

<HOASource id="src:aviary"

signal="signal:aviaryHOA"

orientation="30, 0, 0"

position="0, 0, 0"

cspace="relative"

mode="continuous"

play="true"

extent="geom:aviary_extent"

extentTransform="True"

transitionDistance="2"

representation="0"

is6DoF="False"

vdlMethod=”linear”/>

## ChannelSource

| <ChannelSource> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a source which renders multi-channel signals, belonging to a loudspeaker setup. The ChannelSource has a position/orientation in space. The signal component of the ChannelSource must contain the same number of waveforms as indicated by the inputLayout attribute. |  |  |  |  |
| Child node | Count | Description |  |  |
| <Loudspeaker> | >=1 | Virtual loudspeaker (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference |
| inputLayout | CICP layout | R |  | Speaker layout of the original audio signal |
| active | Boolean | O, M | true | If true, then render this source |
| gainDb | Gain | O, M | 0 | Gain (dB) |
| refDistance | Float > 0 | O | 1 | Reference distance (m) (see comment below) |
| signal | AudioStream ID or GranularSound ID | R, M |  | Audio stream or granular database |
| aparams | Authoring parameters | O | none | Authoring parameters (see 4.14) |
| mode | Playback mode | O | continuous | Playback mode {“continuous”, “event”} |
| play | Boolean | O, M | False | Playback enabled? |
| vdlMethod | String | O | “spline” | Defines the Variable Delay Line method to be used (“spline”, “linear”, “spline_erLinear”) |
| reverbGainDb | Gain | O, M | 0 | Additional gain (dB) to be used for reverb rendering in all acoustic environments. It shall range between -119.0 to +12.0. |
| muteReverb | Boolean | O, M | False | A flag switching the reverb on or off for the given source |
| grainDescrCoor | Position | O, M | (0 0 0) | The target position within the granular database. All coordinates within [0,1] |
| grainGroupProb | Float, Float | O, M | none | The index and probability of a grain group |

| <Loudspeaker> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of an <ChannelSource> node, it declares single virtual loudspeaker. The position and orientation are specified relative to the parent ChannelSource’s position/orientation. Loudspeakers are not modifiable at runtime. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| position | Position | R |  | Position |
| orientation | Rotation | O | towards channel-source position | Orientation; reference point is the position of the <ChannelSource>. The algorithm to be used for rotation calculation is specified in the LSDF, see the alignment of <Loudspeaker> elements inside <LoudspeakerSetup> elements. |
| channel | Integer | R |  | Channel index in the associated AudioStream |
| directivity | Directivity ID | O | none | Associated loudspeaker radiation pattern |
| directiveness | Value | O | 1 | Directiveness (see 3.6.1) |

Example:
<!-- Example of a five speaker layout according to CICP layout 5 -->

<ChannelSource id=”src1” position=”1.2 1.65 -2.10” signal=”sig:spk” inputLayout=”5” vdlMethod=”linear” reverbGainDb=”-20”/>

<Loudspeaker position=" 0   0 -1.8" orientation=” 180 0 0” channel=”0”/>

<Loudspeaker position="-1   0 -1.4" orientation=”-145 0 0” channel="1"/>

<Loudspeaker position=" 1   0 -1.4" orientation=” 145 0 0” channel="2"/>

<Loudspeaker position="-1.4 0  1.4" orientation=” -45 0 0” channel="3"/>

<Loudspeaker position=" 1.4 0  1.4" orientation=”  45 0 0” channel="4"/>

</ChannelSource>

Remarks

- When the signal of an AudioElement is modified, the new AudioStream must have the same numbers of channels as the original AudioStream.

## Reference distance and playback control

Reference distance

Certain audio elements are rendered in dependence on the listener’s distance to the audio source (“distance attenuation”). The reference distance field allows to specify a distance at which – independently from the distance attenuation law used – the computed attenuation of the audio element is 0 dB (see Fig. 9). Thus, different renderer implementations will deliver the same level balance between audio elements for points within a reference distance and thus sound comparable (assuming free-field conditions and no room reflections). Note that the reference distance is independent of the gain. For a ChannelSource, the same reference distance is used for all virtual loudspeakers.

Figure 9: Reference distances of AudioElements to an imaginary reference location (red)

### Playback control

Audio Elements feature two attributes that allow controlling the playback of sound. The mode attribute controls the way the playback behaves: It can either be event-driven or continuous. Both modes have a different behavior:

- In mode=”continuous” the sound is continuously playing from the beginning of the scene.
In this mode, the play attribute has no effect.

- In mode=”event” the sound playback is triggered by setting the play attribute to true. When the playback is finished, the play attribute is automatically reset to false. The playback always starts from the beginning of the signal.

The following example illustrates how a sound is played triggered by an event. The playback control is formulated as an update:

<AudioStream id=”sig:doorHandle1” file=”doorHandle1.wav” />

<AudioStream id=”sig:doorHandle2” file=”doorHandle2.wav” />

<ObjectSource id=”src:doorHandle” signal=”sig:doorHandle1” mode=”event” />

<Update id=”upd:doorOpen”>

<!-— Start playback of sound 1 -->

<Modify id=”src:doorHandle” play=”true” />

</Update>

<Update id=”upd:doorOpen2”>

<!-— Start playback of sound 2 -->

<Modify id=”src:doorHandle” signal=”sig:doorHandle2” play=”true” />

</Update>

The following example illustrates how an ObjectSource is enabled/disabled when the listener enters/leaves its proximity:

<!—- Define a proximity controlled playback -->

<ListenerProximityCondition id=”cond:listenerInside” region=”geo:region1” />
<Box id=”geo:region1” position=”1 0 4” size=”10 5 50” />

<Update condition=”cond:listenerInside”>

<Modify id=”src:musician” active=”true” />

</Update>

<Update condition=”cond:listenerInside” fireOn=”false”>

<Modify id=”src:musician” active=”false” />

</Update>

## Spatial transforms

Multiple spatial entities (see Fig. 3) can be grouped and be translated/rotated together. Such spatial parenting (as found in classical scene graphs) is realized using a <Transform> node. In order to limit the complexity, nesting of transforms is not allowed.

Note that the geometry specification using voxels <Voxels> as specified in Section 3.7 is not applicable to <Transform> node.

A transform defines a frame of reference, called transform coordinate system (see Fig. 10). It is derived from the global coordinate system by translation and rotation, specified for the transform. Elements placed within the transform, having cspace=”relative” selected, expresses their position and orientation within this transform’s system.

Figure 10: Global coordinates and transform coordinates

| <Transform> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a spatial transform node that translates and/or rotates child entities. Note that position or orientation or both must be specified. Selecting cspace=’listener’ attaches the transform to the listener. This allows positioning entities shifted/rotated relative to the listener’s head. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | O, M | (0 0 0) | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O | relative | Spatial frame of reference |

<Transform id=”tf:bus” position=”12.3 0 -50.2” orientation=”90 0 0”>

<ObjectSource id=”src:tire1” position=”...” ... />

<ObjectSource id=”src:tire2” position=”...” ... />

<ObjectSource id=”src:engine” position=”...”

extent=”ext.engine” ... />

<Box id=”ext:engine” ... />

<Box id=”geo:bus_occluder” material=”mat:bus_occluder” ... />

</Transform>

| <Anchor> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Implements a spatial transform used for placing objects in an Augmented Reality (AR) setting. The position and orientation are fixed and can only be defined once, before the scene is started. The audioSourceRescale factor is applied to the position of child audio sources (object, Channel, HOA Source, HOA Group). The order of application is translation, rotation and then rescaling. No audio source may have extent when used with rescaling. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| lsdf_ref | ID | R |  | Identifier of the corresponding LSDF anchor |
| audioSourceRescale | Rescale Factor | O, M | (1 1 1) | Rescale factor to be applied to the position and dimensions of all audio sources (Object, Channel, HOA Source, HOA Group) nodes of the transform, in x-, y-, z- dimensions. |
| autoRescale | Bool | O | False | Enable the renderer to determine the rescale factor by comparing the Acoustic Environment region to the one defined in the LSDF. |
| tolerance | float | O, M | (0.0 0.0 0.0 0.0 0.0 0.0) | Control attribute for autoRescale which defines the percentage of scene region that remains outside of LSDF region, (max x-dimension, min x-dimension, max y-dimension, min y-dimension, max z- dimension, min z-dimension). |

<Anchor id=”tf:picture_on_the_wall” lsdf_ref=”picture_anchor”>

<ObjectSource id=”src:voice” position=”…” ... />

<Box id=”ext:picture” ... />

</Anchor>

The content creator can use the audioSourceRescale attribute to change the scale of the audio sources in the scene for an optimal experience. For example, to shrink a concert experience to fit in a smaller listener space. If the scene is flexible to fit any listener space size, then they can choose to set the autoRescale attribute to True. And the tolerance attribute can be used to limit the amount of rescaling that is automatically performed.

<Anchor id="tf:tv" lsdf_ref="tv" autoRescale="true" tolerance="…">

<Box id="geo:venue" size="…" position="…"/>

<HOAGroup	id="…" region="geo:venue" />

<HOASource id="…"…/>

<HOASource id="…"…/>

</Anchor>

## Conditions

The following condition can be used to trigger updates inside the renderer:

| <ListenerProximityCondition> |  |  |  |  |
| --- | --- | --- | --- | --- |
| This condition expresses if the listener (user) resides within a certain spatial region (volume). The region is specified by a geometric object (see Sec. 3.7). Note: When a mesh is used to specify this spatial volume, it must be ensured by the author, that the mesh has a closed hull (no openings). |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| region | Geometry ID | R |  | Spatial region |

## Audio resources

| <AudioStream> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an audio stream provided by a local audio file. AudioElements can use it as signal source. The file can have any number of channels but must be at a sampling rate of 48000 Hz. For the evaluation platform, input channels for the renderers have to be specified (indices starting with 0). |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| isLocallyCaptured | Bool | O | False | Marks the stream as locally captured. The content for this stream is expected to be provided to the renderer in real time and is not present at encoding time. The number of channels for locally captured streams is assumed to be one. |
| file | String | R |  | Path of the audio file (relative to scene.xml file) |
| aepInputChannels | List of integers or integer range | R |  | List of the channel indices, that the renderers are supplied with the signal in the Audio Evaluation Platform (AEP) |

### Original signals

The original audio files have to be in Microsoft WAV format, sampled at a rate of 48000 Hz. Each Audio Element shall obtain its audio signal from a single audio file (i.e. interleaved if multichannel). For HOA, the channel order of the original audio stream must be ACN and the normalization method must be SN3D.

### Encoded/decoded signals

The MPEG-H 3D Audio is used to encode and decode all original audio signals. The resulting (uncompressed) WAV files are sampled at a rate of 48000 Hz. Each Audio Element shall obtain its audio signal from a single audio file (i.e. interleaved if multichannel). The HOA audio data in the HOA output interface is provided in the so-called equivalent spatial domain representation. The conversion from the HOA domain into the equivalent spatial domain representation and vice versa is described in Annex C.5.1 of ISO/IEC 23008-3.

### Channel routing in the evaluation

Note, that the aepInputChannels attribute is used for the evaluation only, but not an MPEG-I bitstream. The evaluation platform has to supply all audio streams in the scene as input channels to the audio renderers. In order to match the routing in the AEP, each AudioStream must specify its target input channels as provided to the renderers by the AEP. The Max patch reads this information, loads the audio files, provides them as audio streams and maintains the correct routing for the renderers. aepInputChannels can be a list of channel indices separated by commas. Alternatively, it can be specified as an integer range (format ‘startIndex:endIndex’).

Example:

<!-- Stereo signal routed to renderer input channels 0 and 1 -->

<AudioStream id=”signal:piano” file=”ellington.wav”

aepInputChannels =”0, 1” />

<!-- Mono signal routed to renderer input channel 2 -->

<AudioStream id=”signal:trumpet” file=”armstrong.wav”

aepInputChannels =”2” />

<!—- A 5.1 signal routed to renderer input channels 3..8 -->

<AudioStream id=”signal:drums” file=”drums51.wav”

aepInputChannels =”3:8” />

<!-- Mono signal routed to renderer input channel 9 -->

<AudioStream id=”signal:voice” file=”ella.wav”

aepInputChannels =”9” />

<!—- HOA order 2 signal routed to renderer input channels 10..18 -->

<AudioStream id=”signal:ambience” file=”ambienceHOA2.wav”

aepInputChannels =”10:18” />

### Granular Sound

To specify that a sound source is based on generated granular sound rather than on an audio file, a GranularSound can be referenced instead of an AudioStream.

| <GranularSound> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a granular sound database provided by a local file. AudioElements can use it as signal source. The file can have any number of channels but must use the sampling rate of 48000 Hz. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| file | String | R |  | Path of the granular database file (relative to scene.xml file) |
| numChannels | Integer | R |  | Number of channels used to render granular sound |

The granular sound is generated from a granular database that is stored in an HDF5 file. The granular database contains audio grains specified at a certain ‘descriptor coordinate’. It also contains metadata pertaining to granular rendering such as length of grain history, search radius to look for grains in the descriptor space, probability values pertaining to each grain to name a few. The format of the granular database files is described in Annex A.5.

## Source radiation patterns

Sound radiation patterns (directivities) of ObjectSources are described by a finite number of sampled directions and a range of different frequencies. A directivity is an additional direction-dependent filter applied to the source. Such data is loaded from SOFA files [3] or other equivalent formats. Each data point describes the transmission characteristic in this direction. This can be magnitude responses in octave or 1/3 octave frequency resolution. In the SOFA format, this is specified in the generalTF scheme [3]. Whenever possible, the set of directions shall cover the full sphere. Arbitrary spherical grids are supported. When a directivity is assigned to an ObjectSource the following alignment is assumed: The frontal direction of the directivity (0°, 0°) points into -z direction of the source’s local coordinate axes, the left direction (90°, 0°) to -x, and the up direction (0°, 90°) to +y. A local SOFA file representing a radiation patterns shall use a spherical coordinate system following the EIF coordinate convention as described in Annex A.2.

| <SourceDirectivity> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a source radiation pattern provided by a local file. ObjectSources can be associated such a directivity. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| file | String | R |  | Path of the directivity file (relative to scene.xml file) |

### Directiveness

The directiveness parameter d≥0 controls the intensity of the directivity over all frequency bands. The directivity factors m are altered by the function  m→md. According to this equation, the value d=1 keeps the original directivity. Values 0<d≤1 weaken the directiveness, until d=0 results at omnidirectional directivity. Values d>1 emphasize the directiveness.

### Normalization

It is assumed that when a directivity is assigned to an ObjectSource, that all normalization required is inherent in the directivity file, and that the ObjectSource audio stream has been produced or filtered such that the application of the directivity filters does not produce unwanted audio artifacts or frequency-domain filtering.

## Geometric elements

Geometric information is fundamental for a 6DoF scene in order to realize spatially-dependent room acoustic effects, particularly diffraction and occlusion. Introducing abstract geometric elements enables an elegant solution for aspects beyond merely room models. A scene can contain geometric elements to

- specify the spatial extent of ObjectSources (e.g. the source width)

- describe Acoustic Elements (e.g. occlusion, diffraction and reflection, etc.)

- spatially decompose a scene into sub scenes (e.g. audibility ranges of AudioElements)

Three geometry concepts are supported:

- Primitives, such as a box, sphere or cylinder (see Fig. 11), which are mathematically modeled (without a spatial discretization) and thus well suited for bounding volumes.

- Meshes can be used to approximate arbitrary shapes (with a spatial discretization). A mesh consists of a list of vertices (3D coordinates) and a number of triangular faces (i.e. the indices of three vertices). Meshes can be for specifying geometrical models of the environment (e.g. a room).

- Voxels can be used to approximate arbitrary shapes (with a spatial discretization). A voxel represents a volume referenced by indexes of a regular grid in three-dimensional space. Each voxel is associated with acoustically relevant properties and audio rendering parameters.

Geometric elements can be associated with an acoustic material. Otherwise, they are considered acoustically transparent.

### Primitives

| <Box> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a box with three spatial dimensions (see Fig. 11a). The origin of a box is always in its center. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference |
| size | Dimensions | R, M |  | Size (x-, y-, z-direction) |
| material | Material ID | O | none | Associated acoustic material |

| <Sphere> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a sphere (see Fig. 11b). Selecting different spatial dimensions (x, y, z) warps the sphere into an ellipsoid. The origin of a sphere is always in its center. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference |
| size | Dimensions | R, M |  | Size (x-, y-, z-direction) |
| material | Material ID | O | none | Associated acoustic material |

| <Cylinder> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a cylinder (see Fig. 11c). The rotation axis is y. The size in y-direction specifies the height, whereas the sizes in x- and z-direction define the cross section. Equal sizes for the x- and z-direction result in circular cross section. Selecting different spatial dimensions for x and z warps the cross section to an ellipse. The origin of a cylinder is always in its center. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | R, M |  | Position |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference |
| size | Dimensions | R, M |  | Size (x-, y-, z-direction) |
| material | Material ID | O | none | Associated acoustic material |

When one of the three size dimensions is zero, the element becomes 2D (plane, disc, etc.). The surface of a primitive faces towards the outside. When an acoustic material is assigned to a primitive, it is applied on the outer hull of the primitive. Note: The inside of a shoebox room cannot be defined by a single primitive.

| - Box | - Sphere | (c) Cylinder |
| --- | --- | --- |

Figure 11: Geometric primitives

### Meshes

| <Mesh> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a triangle mesh. A mesh consists of a list of vertices (3D coordinates) and a number of triangular faces (i.e. the indices of three vertices). Meshes can be used to describe arbitrary geometry. Any <Mesh> node has to have one or more <Vertex>, <Face> child nodes, defining points and triangles. |  |  |  |  |
| Child node | Count | Description |  |  |
| <Vertex> | >=1 | Vertex (see below) |  |  |
| <Face> | >=1 | Face (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| position | Position | O, M | (0, 0, 0) | Position (origin of the mesh) |
| orientation | Rotation | O, M | (0° 0° 0°) | Orientation |
| cspace | Coordinate space | O, M | relative | Spatial frame of reference (of the entire mesh, but not its vertices) |

| <Vertex> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of a <Mesh> node, it declares a vertex (point) for spanning a triangle. Vertices have an index making them referenceable (e.g. in a Face). Vertices are always expressed in the mesh’s local coordinate system, i.e. relative to the mesh’s position. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| index | Integer number | R |  | Index of the vertex (unique integer) |
| position | Position | R |  | Position (relative to the mesh’s origin) |

| <Face> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of a <Mesh> node, it declares a triangle spanning three vertices. Vertices are selected by their indices. The order of vertices matters, as it affects the faces normal, i.e. determining which side is front/back. Faces themselves have an index in order to make them referenceable. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| index | Integer number | R |  | Index of the face (unique integer) |
| vertices | List of three indices | R |  | Vertex indices |
| material | Material ID | O | none | Associated acoustic material |

Faces of a mesh are (acoustically) one-sided only. The order of the vertices in a triangle is important, as it determines the normal direction (front). Given a triangle (v1,v2,v3) its front is considered the side, where its normal vector points to. Matching OpenGL conventions, the normal n of a triangle (v1,v2,v3) is calculated by the cross product of two edge vectors n=v2-v1×(v3-v1).

Examples:

<!-- Define a box -->

<Box id=”room_bounds”

position=”2.2 0.0 -3.5”

rotation=”0.0 45.0 0.0”

size=”12.5 3.2 8.4”
     material=”marble” />

<!—- Define a sphere -->

<Sphere id=”src1_region”

position=”-2.0 0.6 1.0”

size=”1.0 1.0 1.0”

material=”marble” />

<!—- Define a mesh -->

<Mesh id=”room_surface”>

<Vertex index=”0” position=”1 2 3” />

<Vertex index=”1” position=”0.2 -1 0” />

<Vertex index=”2” position=”2 2 1” />

<Vertex index=”3” position=”1 0.5 -0.1” />

...

<Face index=”0” vertices=”0 1 2” material=”upholstery” />

<Face index=”1” vertices=”0 2 3” />

...

</Mesh>

### Voxels

| <VoxelScene> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares voxel scene dimensions and voxel scene spatial resolution of the volumetric-based representation of the scene geometry. The overall scene geometry may be comprised of multiple voxel scenes. Note that audio sources may be located outside of the voxel scene. |  |  |  |  |
| Child node | Count | Description |  |  |
| <VoxelBlock> | >=0 | Voxel block element specification (see below) |  |  |
| <Instructions> | >=0 | Instructions for voxel-based processing |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| vox_resolution | Voxel resolution | R |  | Voxel scene resolution (x-, y-, z-direction) |
| position_s | Position | R, M |  | Corner point indicating the start position (x-, y-, z-coordinates) of the voxel scene |
| position_e | Position | R, M |  | Corner point indicating the end position (x-, y-, z-coordinates) of the voxel scene |
| diffraction_map_height | Voxel index | O, M |  | Voxel index specifying the scene height at which the diffraction map is obtained, (i.e., the scene slice at diffraction_map_height) |

| <VoxelBlock> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a voxel block element defining a set of voxels sharing the same acoustic material attribute. A voxel scene is defined by a list of voxel block elements. Sequentially defined block elements override the overlapping voxel data. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| material | Material ID | O, M | ”” (empty string) | Associated acoustic material |
| grid_coordinate_s | Voxel Grid Position | R, M |  | Start grid position (x-, y-, z-indices) of the first voxel defining the block element sharing the same acoustic material attribute (range [1.. vox_resolution] for each dimension) |
| grid_coordinate_e | Voxel Grid Position | R, M |  | End grid position (x-, y-, z-indices) of the second voxel defining the block element sharing the same acoustic material attribute (range [1.. vox_resolution] for each dimension) |

| <Instructions> |  |  |
| --- | --- | --- |
| Declares rendering instructions for voxel-based rendering |  |  |
| Child node | Count | Description |
| <VoxelDiffraction> | >=0 | Specify instructions for voxel-based diffraction modeling |

| <VoxelDiffraction> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specify instructions for voxel-based diffraction modeling |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| material | ID | R |  | The material to be enabled or disabled for voxel-based diffraction modeling |
| enabled | bool | R |  | Material is enabled (“true”, default) or disabled (“false”) for voxel-based diffraction modeling |

Example:

<VoxelScene id="vox:scene:0" diffraction_map_height="5" ...>

<Instructions>

<VoxelDiffraction material="material_1 " enabled="true" />

<VoxelDiffraction material="material_2 " enabled="false" />

</Instructions>

<VoxelBlock />

...

</VoxelScene>

Figure 1 — Example of 
voxel scene (left): green voxels — scene geometry; red voxel — first voxel and blue voxel — second voxel defining the scene dimensions;voxel block element defining the scene geometry (right): green voxels — block element defining the scene geometry; red voxel — first voxel forming the block element; blue voxel — second voxel forming the block element

Sequentially defined block elements extend the voxel scene. All overlapping voxel data is re-defined by subsequent defined block elements. This enables a voxel scene representation by small number of voxel block elements.

Example:

Figure 2 — Example of voxel scene definition by means of the sequence of overlapping voxel block elements

<AudioScene id=”Demo” duration=”10”>

<!—Define a voxel scene

<VoxelScene id="half_opened_door" position_s="-1.5 -1.5 -0.5" position_e="1.5 1.5 0.5" vox_resolution="16 16 16">

<!-- The voxel scene is composed of at least one VoxelBlock -->

<VoxelBlock grid_coordinate_s="1 1 1" grid_coordinate_e="16 16 16" material="" />

<VoxelBlock grid_coordinate_s="1 8 1" grid_coordinate_e="16  9 16" material="wall" />

<VoxelBlock grid_coordinate_s="5 8 1" grid_coordinate_e="12  9 13" material="door" />

</VoxelScene>

<!-- Defines an update to open the door in two steps over 1.5sec -->

<Update index="1">

<Modify id="half_opened_door" grid_coordinate_s="5 8 1" grid_coordinate_e="8  9 13" duration="0.5" material="" />

<Modify id="half_opened_door" grid_coordinate_s="5 8 1" grid_coordinate_e="5 10 13" duration="1.0" material="" />

</Update>

</AudioScene>

Objects represented using voxels are addressed by their corresponding material-object IDs. To represent the effect of geometry transformation/animation within the voxel scene, material-object IDs of the corresponding voxels are modified in a “frame-by-frame” manner. It is done by the scene update mechanism (similar to 3D sprite animation), where each update describes all changed voxel elements (or even the entire scene). The corresponding scene changes are defined by external scene authoring tools or voxel scene presentation engine.

Note that the voxel-based geometry specification for the following attributes/nodes is not supported by the RM encoder:

extent	(in <ObjectSource>, <HOASource> and <HOAGroup>)

region	(in <AcousticEnvironment> and <ListenerProximityCondition> and <URR>)

If a scene is authored from “voxel-only” geometry representations, the MPEG-I Audio EIF description should contain the corresponding extent and region specification(s) in mesh format. These mesh parts can be obtained by the application of voxel-to-mesh conversion algorithms (e.g., marching cube algorithm) prior to encoding.

## Acoustic materials

Acoustic materials are characterized by four parameters r, s, t, c illustrated in Fig. 12:

- Specular reflected energy r is reflected back in a distinct outgoing direction

- Diffuse reflected energy s is diffusely scattered back from the material

- Transmitted energy t passes through the material without changing the sound’s direction

- Coupled energy c excites vibrations in the structure and is reemitted by the entire structure

Each value is a number in the range 0..1. The sum of these four values, if specified, must not exceed one:

r + s + t + c ≤ 1. The remaining energy 1-(r+s+t+c) is dissipated energy that is absorbed by the material (turned into heat). Materials are assumed to be isotropic (i.e. rotation-invariant). As acoustic materials behave differently depending on the frequency (wavelength), these coefficients are typically measured and tabulated in octave or 1/3-octave resolution.

Figure 12: Acoustic material characterization

Such data can be found in the data sheets of material manufacturers, textbooks and in publicly available databases. The material’s behavior outside its definition ranges (above/below) shall be extrapolated (last known values are repeated). When only a single frequency is specified, the material properties are frequency-independent.

Authors are encouraged to design scenes in a way that geometric objects with acoustic materials do not overlap in space. For sound transmissions (transmission and coupling, see Fig. 12), one has to distinguish between primitives and meshes. A primitive has a common transmission characteristic by definition (only one material can be assigned to the primitive). This transmission characteristic considers the case that sound from outside of the primitive is transmitted through the primitive. The transmission behavior to a listener inside the primitive is undefined.

Meshes having multiple faces with potentially different acoustic materials are regarded as follows: When a sound path intersects the mesh or parts of it (e.g. multiple faces), the transmission is only effective, when the path intersects a face on its frontal side (normal side). Given that a path intersects a face from its backside, the sound is unaffected. When a sound path intersects the same concave shape multiple times, each front-side intersection accumulates the sound transmission.

| <AcousticMaterial> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an acoustic material. A material characterizes the acoustic behavior of surfaces in a 3D model. A material’s properties are expressed frequency-dependent by absorption and optionally scattering coefficients. Any <AcousticMaterial> node has to have one or more <Frequency> child nodes. |  |  |  |  |
| Child node | Count | Description |  |  |
| <Frequency> | >=1 | Data specification (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |

| <Frequency> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of an <AcousticMaterial> node, it declares the material's acoustic behavior at a specific frequency. A requirement for the coefficients is that r + s + t + c ≤ 1. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| f | Float | R |  | Frequency in Hertz |
| r | Float | O | 0 | Specular reflection coefficient (range 0..1) |
| s | Float | O | 0 | Diffuse scattering coefficient (range 0..1) |
| t | Float | O | 0 | Transmission coefficient (range 0..1) |
| c | Float | O | 0 | Coupling coefficient (range 0..1) |

Examples:

<!—- Frequency-independent material -->

<AcousticMaterial id=”marble”>

<Frequency f="0.0" r=”0.85” s=”0.05” c=”0.05” />

</AcousticMaterial>

<!—- Frequency-dependent material -->

<AcousticMaterial id=”upholstery”>

<Frequency f="125.0"  r=”0.5” />

<Frequency f="250.0"  r=”0.42” />

<Frequency f="500.0"  r=”0.30” />

<Frequency f="1000.0" r=”0.21” />

<Frequency f="2000.0" r=”0.15” />

<Frequency f="4000.0" r=”0.06” />

</AcousticMaterial>

## Acoustic Environments

An <AcousticEnvironment> describes the acoustic conditions within the entire scene or a certain spatial zone by means of room acoustic parameters. The conditions are specified at one or more points in the environment. Some parameters are frequency-independent (e.g. pre-delay), while others are specified for different frequencies (e.g. reverberation time, diffuse-to-direct ratio). The <AcousticEnvironment> provides hints on the envisioned environment’s acoustic behavior. Authors of scenes are encouraged to report how they determined the acoustic parameters of their scene. Hereafter, the different parameters are explained.

The 60 dB reverberation time, short RT60, is defined as the time it takes for the sound pressure level in a room to reduce by 60 dB, measured after a generated steady-state test signal is abruptly ended. It is defined for a specific <Frequency> as an attribute rt60 and specified in seconds. RT60 measurement is performed by calculating twice the T30 decay time. The T30 is derived from a “non-curved” 30 dB attenuation region of the energy decay curve of the RIR. This region is selected by the scene provider.

The pre-delay time indicates the delay between the emission at the source and the onset of the diffuse late reverberation part of a signal (i.e. the sound after the early reflections) and is specified in seconds.

The equation estimating an appropriate predelay value is based on the time of flight (ToF) from a sound wave between the two most distant boundaries of the room:

tpredelay=4*dlongestc

with c=343 m/s, the speed of sound, and dlongest is the largest axis of the bounding volume (in m).

The reasoning is that it takes at least two back-and-forth bounces between the most distant walls before a diffuse sound field has been achieved in the room. In order to capture only the diffuse response in the RIR, predelay must be high enough to avoid the influence of early reflections. predelay is frequency-independent and thus an attribute of <AcousticParameters>.

The Diffuse-to-Source-Ratio (DSR) specifies the level of the diffuse reverberation relative to the level of the total emitted sound. This can be determined while making an RT60 measurement.  It is defined for a specific <Frequency> as an attribute dsr and can be computed as follows:

DSR=total diffuse reverb energytotal emitted energy

For example, a value of 0 indicates direct sound only, while large values will describe an almost completely reverberant (wet) acoustic environment. Note that the dsr values do not influence the amplitude of the direct sound in the process of rendering. While DSR is a general description of a room’s acoustic properties, rendering reverberation using DSR requires taking into account the source’s directivity pattern to find the total emitted energy from the PCM signal’s reference level. DSR values are independent of directivity and may be determined with a source of any directivity, e.g. an omni-directional source. The total diffuse reverb energy denotes the reverberation energy at any point in the region for which the acoustic environment is defined, and is therefore directly linked to the PCM signal’s reference level. A more detailed informative description of the DSR measure can be found in [Error! Reference source not found.].

The complexityLevel attribute of <AcousticParameters> controls the computational complexity of the reverb in an acoustic environment. Smaller values of complexityLevel will also reduce the quality (echo density) of reverb while larger values will increase the quality (echo density). complexityLevel indicates a suggested reverb configuration which would result in sufficient quality, but renderers can deviate from the indicated setting in either direction depending on the available computational resources.

| <AcousticEnvironment> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an acoustic environment. The environment is characterized by acoustic parameters at a number of points in space. Additionally, a bounding volume can be specified. |  |  |  |  |
| Child node | Count | Description |  |  |
| <AcousticParameters> | >=1 | Acoustic parameters (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| region | Geometry ID | O | none | Region in which properties are specified |

Note that the geometry specifying the region of an acoustic environment must be static to the encoder and may not be changing at presentation time.

| <AcousticParameters> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of an <AcousticEnvironment> node, it declares the environment’s acoustic behavior at a specific point in space. |  |  |  |  |
| Child node | Count | Description |  |  |
| <Frequency> | >=1 | Frequency data specification (see below) |  |  |
| Attribute | Type | Flags | Default | Description |
| position | Position | O |  | Position (Required if multiple <AcousticParameters> are specified within one <AcousticEnvironment> |
| predelay | Float | O | none | Pre-delay time in seconds |
| complexityLevel | Integer | O | 3 | ComplexityLevel of reverb in the <AcousticEnvironment> on a five-point scale 1, 2, …, 5. The value of 3 causes the default reverb settings to be used providing a good quality/complexity compromise for most situations. Values 1 and 2 correspond to computationally lighter configurations having also less echoes and density in reverb. Correspondingly, values 4 and 5 create an increasingly dense but also more complex reverb suitable for cases where reverb quality is very important. |

| <Frequency> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Inside of a <AcousticParameters> node inside an <AcousticEnvironment> node, it declares the environment’s acoustic behavior at a specific frequency. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| f | Float | R |  | Frequency in Hertz |
| rt60 | Float | R |  | Reverberation time (RT60) in seconds |
| dsr | Float | O | none | Diffuse-to-source ratio Note: dsr can only be specified, if the predelay is defined |

Example:

<!—- Region where these conditions shall be simulated (optional) -->

<Box id=”geo:cube1” position=”1 2 -4” size=”10 3.4 12” />

<AcousticEnvironment id=“room1“ region=“geo:cube1”>

<!-— Measurements performed at position 1 -->

<AcousticParameters position=”4 2 6.5” predelay=”0.075”>

<Frequency f="63.0"  rt60="0.82" dsr=”0.3” />

<Frequency f="125.0" rt60="1.23" dsr=”0.25” />

<Frequency f="250.0" rt60="1.49" dsr=”0.22” />

<Frequency f="500.0" rt60="1.05" dsr=”0.2” />

</AcousticParameters>

<!-— Measurements performed at position 2 -->

<AcousticParameters position=”-1 0.3 1.2” predelay=”0.062”>

<Frequency f=”500.0”  rt60=”0.96” dsr=”0.24” />

<Frequency f=”1000.0” rt60=”0.89” dsr=”0.20” />

<Frequency f=”2000.0” rt60=”0.82” dsr=”0.14” />

</AcousticParameters>

</AcousticEnvironment>

## Portals

A <Portal> describes an acoustic connection between environments. A simple example is shown in Figure 3, in a top view of two environments. Portals are used for determining how and where sound from one environment transfers to another. For example, how reverberation in a source environment is audible in a target environment as coming from the source environment through the portal. Or for determining where and how much of a source in one environment contributes to reverberation being generated in another. There are many other possible uses for portals, and this element provides the option to define them and their relevant properties.

It should be noted that not all space in a scene is necessarily represented by an <AcousticEnvironment>, while a portal may connect an acoustic environment with an area not represented by an acoustic environment. Or a portal can even be defined between two areas that are both not represented by an acoustic environment. In this case the corresponding acousticEnvironment fields shall be omitted.

The portalExtent attribute identifies a geometry that represents the portal’s extent. The thickness of the portal may often be relevant when there is also (possibly) an occluding geometry in place of the portal (e.g. a door). In such cases the portal’s positioning and thickness shall at least coincide with the occluding geometry to prevent the portal being occluded from either side.

Portals may also be defined in places where there is still an occluding element such as a windowpane or even the wall itself, in case that there is a possibility that the occluding element is removed, or the portal is used for a coupled source. The occluding element may be removed, for example, due to user interactions or dynamic updates.

Figure 3 – Example of how a portal extent geometry is used to represent a doorway in a wall.

| <Portal> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a portal. Each portal connects two environments (not necessarily represented by acoustic environment nodes). It describes properties of a segment of geometry through which acoustic energy is transferred between the two environments. The Location and size of a portal may be specified by either reference to a ‘portalExtent’ or by giving ‘portalPosition’ and ‘portalSize’. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| portalExtent | ID | O | None | Geometry ID representing the extent of the portal |
| portalNormal | Vector3D | R |  | Normal vector for the portal, pointing into acousticEnvironment1. When flipped, it points into acousticEnvironment2. |
| portalPosition | Position | O | None | Specifies the centre point of the portal. |
| portalSize | Dimensions | O | None | Specifies the dimensions of the portal relative to the portal normal [width, height, depth]. |
| acousticEnvironment1 | ID | O | None | AcousticEnvironment ID to first associated AE |
| acousticEnvironment2 | ID | O | none | AcousticEnvironment ID to second associated AE |
| material | ID | O, M | none | Material ID associated with the portal |

Example:

<Portal id="prt:nurseRoom2Corridor"
         portalExtent="box:nurseRoom2CorridorPortalExtent"
         portalNormal=”0 0 -1”
         acousticEnvironment1="NurseRoom"
         acousticEnvironment2="Corridor"
         material="AcMat:PaintedWood"/>

## Acoustic Tuning Parameters

Acoustic tuning parameters control scene wide acoustic effects, and are optional but with default values applied otherwise.

The attribute DefaultAE enables a default acoustic environment. With this enabled, a default acoustic environment will be applied to all regions of the scene where no other AcousticEnvironment is specified. This enriches especially the perception of outdoor scenes.

The GlobalAEControl element allows control over the distance gain attenuation and minimum distance of the distance gain attenuation in the acoustic environments specified in all AcousticEnvironment elements.

The attributes VoxDirectOcclusionControl and VoxDiffractionControl specify the direct occlusion and diffracted source attenuation effect modelling strength parameters.

The attribute EarlyTuningGainDb controls the level of attenuation of the Early Reflections.

The LCEarlyOutdoorTuning element allows control over the tuning parameters of the Low Complexity Early Reflections.

| <AcousticTuningParameters> |  |  |
| --- | --- | --- |
| An <AcousticTuningParameters> node declares global acoustic behaviors. |  |  |
| Child node | Count | Description |
| DefaultAE | 0..1 | Default Acoustic Environment data (see below) |
| GlobalAEControl | 0..1 | Additional global AcousticEnvironment control parameters (see below) |
| VoxDirectOcclusionControl | 0..1 | Voxel direct occlusion control data (see below) |
| VoxDiffractionControl | 0..1 | Voxel diffracted source control data (see below) |
| VoxEarlyReflectionControl | 0..1 | Voxel early reflection control data (see below) |
| EarlyTuningGainDb | 0..1 | Early Reflection gain data (see below) |
| LCEarlyOutdoorTuning | 0..1 | Low Complexity Early Reflection outdoor tuning data (see below) |

| <DefaultAE> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares a default acoustic environment to be used in all areas of the scene not contained within an <AcousticEnvironment> You can only specify preset OR environment, but must specify one of them. The list of valid presets is contained in the MPEG-I immersive audio specification document. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| enabled | Bool | O | False | If true, render the default acoustic environment |
| preset | String | O | none | Select which preset to use for LUT |
| environment | ID | O | none | Reference to an AcousticEnvironment to be used for the default AE |
| gainDb | Float | O | 0.0 | Gain of environment in decibels |
| density | Float | O | 1.5 | Controls the diffuseness of the reverb, >= 1.5 |
| delayMinMs | Float | O | 38.19 | Controls the diffuseness of the reverb, given in milliseconds, >= 15 |
| distanceDropDb | Float | O | 4.0 | Decibel constant for controlling the amount of distance gain attenuation in late reverberation. Distance doubling reduces reverberation level by reverbDistanceGainDropDb decibels. |
| minDist | Float | O | 1.0 | Minimum distance threshold applied to late reverberation distance gain attenuation. |

| <GlobalAEControl > |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters that are applied to all acoustic environments defined as an <AcousticEnvironment> element and which are no default AE. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| distanceDropDb | Float | O | 1.5 | Decibel constant for controlling the amount of distance gain attenuation in late reverberation. Distance doubling reduces reverberation level by reverbDistanceGainDropDb decibels. |
| minDist | Float | O | 1.0 | Minimum distance threshold applied to late reverberation distance gain attenuation. |
| fadeInDistance | Float | O | 0.1 | The threshold distance in meters at which cross-fade processing shall be enabled for immersive reverberation when not at a portal opening. |

| <VoxDirectOcclusionControl > |  |  |
| --- | --- | --- |
| Contains a list of <Setting> elements that control the direct occlusion strength of voxel materials |  |  |
| Child node | Count | Description |
| <Setting> | >=1 | Direct Occlusion Control Setting (see below) |

| <Setting> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Material specific occlusion strength control values |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| material | ID | R |  | Acoustic material identifier |
| value | Float | O | 0 | Strength of the occlusion effect (0..100). 0 indicates infinite occlusion “shadow” |

| <VoxDiffractionControl> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Diffracted source attenuation effect modelling strength parameter |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Float | O | 1 | Strength of the diffraction effect (range 0..1) |

| <EarlyTuningGainDb> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Early Reflection tuning gain |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Float | R | -8 | Early Reflection tuning gain (dB) to be used for early reflection rendering (range -21.5..+10) |

| <LCEarlyOutdoorTuning> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares custom tuning parameters for the outdoor mode of Low Complexity Early Reflections, to override the values derived by the geometry analysis in the encoder. If the node is not specified, the values from the geometry analysis will be used as default. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| preDelay1 | Float | R |  | Predelay of first pair of outdoor reflections, in ms |
| preDelay2 | Float | R |  | Predelay of second pair of outdoor reflections, in ms |
| ampFac1Db | Float | R |  | Tuning factor of amplitude factor relative to 1/r falloff for first pair of outdoor reflections, in dB |
| ampFac2Db | Float | R |  | Tuning factor of amplitude factor relative to 1/r falloff for second pair of outdoor reflections, in dB |

| <VoxEarlyReflectionControl> |  |  |
| --- | --- | --- |
| Control parameters for voxel-based Early Reflection Modeling |  |  |
| Child node | Count | Description |
| <NumOrigins> | 0..1 | Number of origin points (see below) |
| <NumClusters> | 0..1 | Number of clusters (see below) |
| <CombineReflectionsPerCluster> | 0..1 | Combine reflections per clusters (see below) |
| <EarlyTuningIndoorGainDb> | 0..1 | Early reflection tuning gain for indoor environment type (see below) |
| <EarlyTuningOutdoorGainDb> | 0..1 | Early reflection tuning gain for outdoor environment type (see below) |

| <NumOrigins> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of origin points for emitting rays |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Integer | R | n/a | Defines the number of origin points for the volumetric ray marching algorithm (range 2..32) Note: higher number provides more precise early reflection modelling but requires more rendering workload. |

| <NumClusters> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of clusters for ray incoming directions |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Integer | R | n/a | Defines the number of clusters for the modeled early reflection incoming directions (range 2..32) Note: higher number provides more precise early reflection modelling but requires more rendering workload. |

| <EarlyTuningIndoorGainDb> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Early Reflection gain data for indoor environment type |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Gain | R |  | Additional gain (dB) to be used for early reflection rendering for indoor environment type. |

| <EarlyTuningOutdoorGainDb> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Early Reflection gain data for outdoor environment type |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| value | Gain | R |  | Additional gain (dB) to be used for early reflection rendering for outdoor environment type. |

Example:

<AcousticTuningParameters>

<EarlyTuningGainDb value="-8.0" />

<VoxDirectOcclusionControl>

<Setting material="mat:Wall" value="0.5" />

<Setting material="mat:Wood" value="42.0" />

</VoxDirectOcclusionControl>

<VoxDiffractionControl value="0.75" />

<VoxEarlyReflectionControl>

<NumOrigins value="5" />

<NumClusters value="8" />

<CombineReflectionsPerCluster />

<EarlyTuningIndoorGainDb value="3.0" />

<EarlyTuningOutdoorGainDb value="-6.0" />

</VoxEarlyReflectionControl>

<DefaultAE enabled="true" preset="outside" gainDb="-3"

density="2" delayMinMs="50" distanceDropDb="5" minDist="2"/>

<GlobalAEControl distanceDropDb=”3” minDist=”5” />

</AcousticTuningParameters>

## Content Creator Headphone Data

To facility an accurate reproduction of the scene, in line with the content creators intent, the content creator may specify the frequency response of the headphones and/or playback system that was used during the authoring process.

| <ContentCreatorHeadphone> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares the Content Creator Headphone equalization data. This can de defined as either a list of frequency-gain pairs, a list of parametric filters, or using one of the predefined presets. |  |  |  |  |
| Child node | Count | Description |  |  |
| <FrequencyGain> | >=0 |  |  |  |
| Attribute | Type | Flags | Default | Description |
| spectype | ID | R |  | Specification type, can be “freqgain”, “parametricfilters”, or “preset” |
| Preset | Int | O | None | Integer identifier to preset EQ used, required if spectype is “preset” |

| <FrequencyGain > |  |  |  |  |
| --- | --- | --- | --- | --- |
| Within a ContentCreatorHeadphone defines either a frequency-gain pair, or a parametric filter definition. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| f | Float | R |  | Frequency in Herts |
| gainDb | Gain | R |  | Gain in dB |
| Q | Float | O | None | Q of the filter, required if profileType is “parametricfilters”. |
| Type | String | O | None | Defines the type of filter specified by the parametric filter, either “highshelf”, “lowshelf” or “peak”. Required if spectype is “parametricfilters”. |

## Airflow Simulation

An AirflowSource declares the acoustic properties to be used for the simulation of listener perceived airflow noise, meaning the sound that a user hears due to the effect of air passing over the ear canal opening.

| <AirflowSource> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an airflow source that should be used for the simulation of air flow noise at the listeners ears. If no airflow profile is specified the default profile of the renderer is used. If no active regions are specified, the airflowSource is assumed to be active across all locations in the scene. |  |  |  |  |
| Child node | Count | Description |  |  |
| <ActiveEnvironment> | >=0 |  |  |  |
| <AirflowProfile> | >=0 |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| speed | Float | R, M |  | Air flow speed |
| orientation | Rotation | R, M |  | Air flow orientation |
| gainDb | Gain | O, M | 0dB | Overall gain applied to the source. |
| active | Boolean | O, M | True | Whether source is active in scene |
| airflowGenerator | Generator ID | O | None | ID of AirflowGenerator |

| <ActiveEnvironment> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares whether an AirflowSource should be active in a specified AcousticEnvironment |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| AcEnvID | AcousticEnvironmentID | R |  |  |
| active | Boolean | O, M | True | Whether source is active in AcEnvID |

| <AirflowProfile> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an airflow profile to be used for an airflow source. Only 1 of speed, azimuth, or elevation must be specified. |  |  |  |  |
| Child node | Count | Description |  |  |
| <AirflowFrequency> | >=1 |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | Identifier |
| profileType | int | R, M |  | 0 = Frequency-Gain pairs, 1 = Peak filters. |
| speed | Float | O |  | Speed for which the profile is defined, in meters per second. |
| azimuth | Float | O |  | Azimuth w.r.t. the listener for which the profile is defined. 0-360 degrees, see Annex A.2 |
| elevation | Float | O |  | Elevation w.r.t. the listener for which the profile is defined. 0-360 degrees see Annex A.2 |

| <AirflowFrequency> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Within an AirflowProfile defines the per-frequency gain and optional filter Q. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| f | Float | R |  | Frequency in Herts |
| gainDb | Gain, or 2 * Gain | R |  | Gain in dB, if modifying Azimuth must be a tuple of gains. |
| Q | Float, or 2 * Float | O | 1 | Q of the filter, required if profileType == 1. if modifying Azimuth must be a tuple of floats. |

Airflow generators define the position in 3D space from which an airflow source emanates. Four types are defined, “Global”, “Point”, “Cone”, and “Plane”.

| <AirflowGenerator> |  |  |  |  |
| --- | --- | --- | --- | --- |
| Declares an airflowGenerator to be used by an airflow source |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| id | ID | R |  | unique identifier |
| type | string | O | “Point” | Valid strings: “Point”, “Cone”, “Plane” |
| position | Position | R |  |  |
| height | Float | O |  | Required for “Cone” or “Plane” types. |
| width | Float | O |  | Required for “Cone” or “Plane” types, determines base diameter of the cone. |
| startWidth | Float | O | 0 | Determines start diameter of cone instead of a point. |
| distanceAttenuationFactor | Float | O | 1 | -5 to +5, Scaling factor for inverse-square distance attenuation, where gain=1r(2*distanceAttenuationFactor) |
| edgeRolloff | int | O | 50 | 0-100, for cones, determines the distance from the center line of the cone, in %, to start fading of the speed parameter. For planes, it determines the distance from the edge of the plane, in % of the width or height, over which to fade out the speed parameter. |

# User Reachable Region

In order to allow optimized encoding/rendering (e.g. early reflection and diffraction parameter analysis on the encoder), it is useful to provide information of the permitted listening positions in the scene.

The user reachable region (URR) is specified as one or several EIF primitives (box, sphere, cylinder).

User positions outside the specified regions is not permissible.

The URR is indicated by specific box naming, which is “userregion1 …n”.

For the voxel scene the permitted listening positions in the scene corresponds to the region defined by two points position_s and position_e. These two points define the start and end corner coordinates of the voxel scene. User positioning outside this region is not permissible.

# Property types

## Boolean

| Value | Description |
| --- | --- |
| true | True / yes / on / enabled |
| false | False / no / off / disabled |

## Float

Floating point values are expressed in decimal notation (e.g. “-124.23543”).

## Identifier (ID)

Entities are identified by an ID, which as a unique string. In XML, an entities’ ID is specified by the id attribute. IDs can be used to reference other entities.

Example:

id=”piano”

## File path

File system paths are relative to the location of the scene.xml file.
In order to ensure platform independence, slashes (/) are used exclusively as filesystem separators. On Windows platforms, these are manually translated to backslashes (\).

Example:

file=”audio/piano.wav”

## Gain

Gains are expressed in decibels (dB) as float numbers. Decibels refer to the effective value of the signal amplitude, i.e. factor=10gain_db/20. 0 dB indicates no gain change relative to the WAV file signal level.

Example:

gainDb=”-6.0” denotes a gain of factor 10-6/20≈0.5

## Coordinate spaces

The specification supports different frames of reference in which positions and orientations are expressed.

By default, an element’s position and orientation are expressed in its parent entities’ coordinate space.

The global cartesian coordinate space is defined by the root entity, that is the audio scene itself. Any other entity with a position and an orientation defines its own local coordinate space. The user inside a VR/AR environment also defines his or her own frame of reference, the user coordinate space. The cspace attribute defines for an entity in which its position and orientation is specified.

| Value | Description |
| --- | --- |
| relative | Expresses the position/orientation relative to the local coordinates of the parent spatial entity (i.e. an entity that has a position/orientation). Two cases can occur: - When the entity is a direct child node of the <AudioScene>, its position/orientation refer to the global coordinate system. - When the entity is a child node of a <Transform>, its position/orientation are relative to the transforms frame of reference. |
| user | Expresses the position/orientation already relative to the user. |

## Position

Spatial positions are expressed by three spatial coordinates referring to a cartesian coordinate system. In XML, a position is specified by a position attribute, followed by a string with three numbers separated by whitespaces. The unit of coordinates is meters.

Example:

position=”1.2 -0.4 2.3” denotes the point at x=1.2, y=-0.4, z=2.3

## Voxel grid positions

Spatial voxel positions are expressed by three spatial indices referring to a regular grid associated with the scene coordinate system. In XML, voxel positions are specified by grid_coordinate_s and grid_coordinate_e attributes, followed by a string with three positive integer numbers (range [1, vox_resolution] per grid dimension) separated by whitespaces. The unit of coordinates is grid index.

Example:

grid_coordinate_s=”1 2 3” denotes the point at grid position ix=1, iy=2, iz=3.

## Dimensions

Spatial dimensions are expressed by three sizes (x, y, z) referring to the entity’s local coordinate system. In XML, the dimensions are specified by a size attribute, followed by a string with three numbers separated by whitespaces. Each value must be a non-negative number. The unit of sizes is meters.

Example:

size=”1 0 2.1” denotes spatial dimensions of x=1, y=0 and z=2.1 meters.

## Vector 3D

3D vectors are expressed by three components (x y z) in the coordinate system of the entity they belong to. In XML, the dimensions are specified by vector attribute, followed by a string with three numbers separated by whitespaces. The unit of vector components is meters.

## Voxel resolution

Spatial voxel resolution is expressed by three values (Nx, Ny, Nz) referring to the number of voxels per each scene dimension associated with the scene coordinate system. In XML, the voxel resolution specified by a vox_resolution attribute, followed by a string with three integer numbers separated by whitespaces. Each value must be a positive number. The unit of sizes is number of voxels.

Example:

vox_resolution=”128 64 32” denotes voxel scene resolution of 128  ix  1, 64  iy  1 and 32  iz  1.

## Rotation

The spatial orientation of an entity is expressed by three axis rotations in yaw-pitch-roll convention (see Fig. 15). This common convention is used by the majority of 3D modeling programs (here sometimes called heading-pitch-bank). A definition of these rotations is provided in annex A.2. In XML, a rotation is described by a rotation attribute, followed by a string with three numbers separated by whitespaces. The unit of angles is degrees.

Example:

rotation=”45 -20 5” corresponds to 45° yaw, -20° pitch and 5° roll.

## CICP layout

Loudspeaker layout indicated by a CICP ChannelConfiguration as specified in ISO/IEC 23091-3:2018 CICP-Audio. The layout is described by an CICP loudspeaker layout index.

## Authoring parameters

The auralization of Audio Elements can be controlled using authoring parameters, representing the artistic intent. The following are available:

| Value | Description |
| --- | --- |
| noDoppler | Omit the simulation of Doppler shifts (e.g. when these are already contained in the signal) |
| noDistance | Omit distance attenuation (e.g. when it is already contained in the signal) |
| noRescale | Omit audio source rescaling (e.q. when the element’s position should not change w.r.t. an anchor object) |
| noCulling | Disable gain and loudness culling. |
| noDirectionalFocus | Omit audio source processing with the Directional Focus effect for Accessibility (e.g. when source signal is rendered without adjustment due to the Directional Focus). |

The parameters are specified in XML by an aparams attribute, whose value is a string with the list of the desired parameters. An empty string identifies no restrictions (default parameters).

Example:
  aparams=”noDoppler,noDistance”

Notes:

Direct signal output, without rendering and spatialization, is not considered
in the evaluation at the moment.

# Conclusions

This annex specifies the encoder input format for the MPEG-I Audio 6DoF metadata encoder.

A general scene meta model is introduced, defining static declarations and dynamic updates. Many essential entity and property types are defined, documented and illustrated by the help of examples.

# References

- N0057 MPEG-I Immersive Audio Draft Documentation for the Audio Evaluation Platform

- N0028 MPEG-I Immersive Audio Architecture and Requirements

- Spatially oriented format for acoustics (SOFA), https://www.sofaconventions.org

- Koppens, J. and Kechichian, P. and Jelfs S., A Generic Reverberation Characterization Metric for Accurate Simulation in Virtual and Augmented Reality Environments, 2023 Immersive and 3D Audio: from Architecture to Automotive (I3DA), Bologna, Italy, 2023, pp. 1-7, doi: 10.1109/I3DA57090.2023.10289204

# Annex A

A.1  Cartesian coordinate system

MPEG-I uses the OpenGL cartesian coordinate system (see Fig. 13),

which is right-handed and +y points up.

Figure 13: OpenGL coordinate system
(right-handed, +y points up)

A.2 Spherical coordinate system

Spherical coordinates (r, φ, θ) are defined by a distance r, azimuth angle φ and elevation angle θ in the following system:

Figure 14: Spherical coordinate system

Positive azimuth angles turn counterclockwise (CCW) with respect to the up axis (y+).
Points in the upper hemisphere (+y) have positive elevation angles. Radii are always positive.

Examples:

- (1, 0°, 0°) spherical matches (0, 0, -1) cartesian

- (1, 90°, 0°) spherical matches (-1, 0, 0) cartesian

- (1, 0°, 90°) spherical matches (0, 1, 0) cartesian

A.3 Yaw-pitch-roll rotations

Yaw-pitch-roll rotations are specific convention of Euler angles (i.e. rotations are three axis). Fig. 15 shows the three different axis rotations. Yaw-pitch-roll rotations are performed intrinsic, i.e. each rotation is relative to the previously rotated axis. This is achieved by executing the (extrinsic) rotations around the global coordinate axis in the following order

- roll = rotation around the global +z axis

- pitch = rotation around the global +x axis

- yaw = rotation around the global +y axis

| - yaw rotation | - pitch rotation | (c) roll rotation |
| --- | --- | --- |

Figure 15: Yaw, pitch and roll rotations

Written in rotation matrices Rx, Ry, Rz  (see [8]) for the three global coordinate axes x, y, z the total 
yaw-pitch-roll rotation matrix yields: Ryprφ,θ,ρ=Ry(φ) Rx(θ) Rz(ρ).

Note, that the order of matrix products read right to left.

An object with yaw-pitch-roll angles (0°, 0°, 0°) has the following orientation in the global cartesian coordinate axes:

- Its front direction points into -z

- Its up direction points to +y

- Its right direction points to +x

A.4 Determining of regions by geometric objects

Using geometric objects to specify spatial regions demands a clear semantic whether a point lies inside or outside. For primitives (cube, cylinder, sphere) this is trivial. For most meshes this can be tested as follows (informal mathematical definition):

A point x∈R3 lies inside a given mesh M, when a ray x+su (s∈R) shot from
x in an arbitrary direction u∈R3 intersects an odd number of M’s faces.

Different test cases are illustrated in Fig. 16. The point in (a) lies in the mesh as one intersection occurs. In (b) it lies outside, as two intersections occur. Note, that when no intersections occur as in (c), the mesh is incomplete and unsuited for defining regions. The content creator must ensure that meshes used for this purpose are complete, i.e. do not have undefined openings.

| (a) Point inside mesh | (b) Point outside mesh | (c) Incomplete mesh |
| --- | --- | --- |

Figure 16: Ray-mesh intersection tests

A.5 Granular synthesis database files

Granular database files are stored in HDF5 files using a special convention. While writing a granular database, one could first set the different dimensions with values, then the variables and finally the metadata.

The convention for granular HDF5 files follows the format of the SOFA conventions, as described in [3]. The table uses the following legend:

Name: the name of the metadata

A colon: the metadata is an attribute

“GLOBAL:”: the metadata is a global attribute

“X:Y”: the metadata is an attribute Y of the variable X

No colon: the metadata is a variable

Data.X: the metadata is structured within the data

Default: default value for the metadata

An asterisk (*): special handling required, mentioned in the column Comment

Flags:

r: read-only, must be the default value

m: mandatory, must be saved in the file

Dimensions: dimensions of the metadata

lower case: the variable size in that dimension determines the dimension size in the file

upper case: variable must be of that dimension (or one of these dimensions)

Dimensions:

| C | descriptor space coordinate dimension |
| --- | --- |
| M | number of grains in the database |
| Ch | number of channels |
| I | singleton scalar value (=1), |
| NumMixRule | number of locations in the descriptor coordinate space where type of mixing rule is defined |
| P | number of probability groups |

Variables:

| Name | Default | Dimensions | Flag | Type | Comment |
| --- | --- | --- | --- | --- | --- |
| GLOBAL:Conventions | GRANULARSOUND |  | rm | attribute |  |
| GLOBAL:GRANULARSOUNDConventions | GranularSoundBase |  | rm | attribute |  |
| GLOBAL:Version | 1.0 |  | rm | attribute |  |
| GLOBAL:randomSelection | True |  |  | attribute | Random selection active in granular synthesis |
| DescriptorCoordinates |  | M,C | m | double | Descriptor coordinates of the grains |
| Grains |  | M,Ch | m | double | Audio grains |
| GLOBAL:trend |  |  |  | attribute | Is trend of descriptor specified |
| DescriptorCoordinatesTrend |  | M,C |  | double | Trend of the descriptor coordinate for the grains |
| descTrendMethod | 0 | I |  | double | Index of method to evaluate descriptor trend |
| GLOBAL:position_in_file |  |  |  | attribute | Is original recording position in file available |
| PositionWithinAudioFile |  | M,I |  | double | Original recording position within file |
| GLOBAL: grain_overlap_amount | True |  |  | attribute | Is overlap percentage (for overlap-add) available |
| GrainOverlapAmount | 0.1 | I |  | double | Overlap percentage or amount for overlap-add |
| coordinateScaling | 1 | I,C |  | double | Scaling parameters for descriptor coordinates |
| smoothingTargetCoordinates | 1 | I,C |  | double | Smoothing parameter for target descriptor coordinate values |
| searchRadius | 0.1 | I |  | double | Search radius for KDTree search for grains |
| grainHistoryLength | 5 | I |  | double | Length of history of used grains |
| grainMixingRule |  | NumMixRule,I |  | double | Mixing rule coefficient (0-1) |
| grainMixingRuleCoordinates |  | NumMixRule,C |  | double | list of descriptor coordinates at which a grainMixRule is specified |
| GLOBAL:useInterpolation |  |  |  | attribute | Is interpolation used in the granular database |
| GLOBAL:useInterpolationResampling |  |  |  | attribute | Is resampling used for interpolation |
| pitchDimension |  | I,C |  | double | Descriptor coordinate dimension corresponding to pitch (Boolean values) |
| interpMethod |  | I |  | double | Index of method used for interpolation |
| GLOBAL:fastGrainSwitchAvailable |  |  |  | attribute | Is fast grain switch threshold available |
| fastGrainSwitchThreshold |  | I |  | double | Distance threshold for fast grain switching |
| probabilityExponentPosition | 1 | I |  | double | exponent used to calculate the probability value associated with the position of the grain in descriptor space |
| probabilityExponentTrend | 1 | I |  | double | exponent used to calculate the probability value associated with the descriptor coordinate trend of the grains |
| probabilityExponentHistory | 1 | I |  | double | exponent used to calculate the probability value associated with the history of the grains |
| probabilityExponentOriginalPos | 1 | I |  | double | exponent used to calculate the probability value associated with the original recording position of the grains |
| probabilityWeightPosition | 1 | I |  | double | weight used to calculate the probability value associated with the descriptor position evaluation |
| probabilityWeightTrend | 1 | I |  | double | weight used to calculate the probability value associated with the descriptor trend evaluation. |
| probabilityWeightHistory | 1 | I |  | double | weight used to calculate the probability value associated with the history of grains |
| probabilityWeightOriginalPos | 1 | I |  | double | weight used to calculate the probability value associated with the original recording position of the grains. |
| GLOBAL:probabilityGroups |  |  |  | attribute | Are probability groups specified in the database |
| probabilityGroup |  | P,I |  | double | Initial probability values for each group |
| grainProbabilityGroup |  | M,I |  | double | Index of probability group to which the grain belongs |
| GLOBAL:perGrainProbability |  |  |  | attribute | Is per grain probability value available in the database |
| perGrainProbabilityVal |  | M,I |  | double | Probability value per grain |