ISO 23090-4:202#(X)

ISO/IEC  JTC 1/SC 29/WG 6

Date: 2024-07-19

Information technology — Coded representation of immersive media — Part 4: MPEG-I immersive audio

DIS stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO 2023

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	vii

Introduction	viii

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

3.1	Acronyms	1

3.2	Terms	2

3.2.1	auralize	2

3.2.2	audio element	2

3.2.3	audio scene	2

3.2.4	control workflow	2

3.2.5	cover	2

3.2.6	conditional scene update	3

3.2.7	Doppler effect	3

3.2.8	dynamic scene update	3

3.2.9	effective spatial extent	3

3.2.10	exterior representation	3

3.2.11	interior representation	3

3.2.12	listener centric format	3

3.2.13	limiter	3

3.2.14	location	3

3.2.15	metadata	3

3.2.16	MPEG-H 3DA decoder	3

3.2.17	orientation	3

3.2.18	portal	3

3.2.19	position	4

3.2.20	primary  ray	4

3.2.21	primary RI	4

3.2.22	renderer	4

3.2.23	renderer pipeline	4

3.2.24	rendering workflow	4

3.2.25	render item	4

3.2.26	RI type	4

3.2.27	scene	4

3.2.28	scene controller	4

3.2.29	scene object	4

3.2.30	scene state	4

3.2.31	secondary ray	4

3.2.32	secondary RI	4

3.2.33	source-centric format	5

3.2.34	spatializer	5

3.2.35	spatially-heterogeneous audio element	5

3.2.36	stage	5

3.2.37	teleport	5

3.2.38	timed scene update	5

3.2.39	voxel sub-scene update	5

3.2.40	triggered scene update	5

3.2.41	user	5

3.2.42	voxel	5

3.2.43	voxel sub-scene	5

3.3	Mnemonics	5

4	Overview	6

4.1	Technical overview	6

4.2	MPEG-I Immersive Audio profiles and levels	9

4.2.1	General	9

4.2.2	Profiles	9

5	MPEG-I Immersive Audio transport	10

5.1	Overview	10

5.2	Definitions	10

5.3	MHAS syntax	11

5.3.1	Audio stream	11

5.3.2	Audio stream packet	11

5.4	Semantics	19

6	MPEG-I Immersive Audio renderer	28

6.1	Definitions	28

6.2	Syntax	28

6.2.1	General	28

6.2.2	Generic Codebook	29

6.2.3	Directivity payloads syntax	31

6.2.4	Diffraction payload syntax	35

6.2.5	Voxel payload syntax	40

6.2.6	Early reflection payload syntax	45

6.2.7	Portal payload syntax	48

6.2.8	Reverberation payload syntax	50

6.2.9	Audio plus payload syntax	52

6.2.10	Dispersion payload syntax	52

6.2.11	Scene plus payload syntax	53

6.2.12	Airflow payload syntax	69

6.2.13	Granular payload syntax	71

6.2.14	RasterMap payload syntax	74

6.2.15	Support Elements	75

6.3	Data structure	82

6.3.1	General	82

6.3.2	Renderer payloads data structure	82

6.4	Renderer framework	131

6.4.1	Control workflow	131

6.4.2	Rendering workflow	143

6.5	Geometry Data Decompression	159

6.5.1	General	159

6.5.2	Geometry	160

6.5.3	Materials	162

6.6	Renderer stages	164

6.6.1	Effect Activator	164

6.6.2	Room assignment	165

6.6.3	Granular synthesis	167

6.6.4	Reverberation	180

6.6.5	Portals	234

6.6.6	Early reflections	248

6.6.7	Airflow Simulation 	260

6.6.8	Discover SESS	267

6.6.9	Occlusion	269

6.6.10	Diffraction	274

6.6.11	Voxel-based occlusion and diffraction	289

6.6.12	Multi-Path voxel-based diffraction with RasterMaps	313

6.6.13	Voxel-based early reflections	319

6.6.14	Metadata culling	325

6.6.15	Heterogeneous extent	331

6.6.16	Directivity	358

6.6.17	Distance	363

6.6.18	Directional Focus	372

6.6.19	Consolidation of Render Items	373

6.6.20	Equalizer (EQ)	379

6.6.21	Low-Complexity Early Reflections (LC-ERs)	381

6.6.22	Fade	390

6.6.23	Single point higher order ambisonics (SP-HOA)	393

6.6.24	Homogeneous extent	398

6.6.25	Panner	402

6.6.26	Multi-point higher order ambisonics (MP-HOA)	409

6.6.27	Low-complexity MP-HOA	447

6.7	Spatializer	452

6.7.1	Binaural spatializer	453

6.7.2	Adaptive loudspeaker rendering	469

6.8	Limiter	499

6.8.1	General	499

6.8.2	Data elements and variables	499

6.8.3	Description	500

6.9	Interface for audio utilization information	502

6.9.1	General	502

6.9.2	Syntax and semantics of an interface for renderer audio utilization	502

Annex A (normative)	503

A.1	Panner default output positions	503

A.2	RIR analysis: loudspeaker source directivity factor	503

A.3	Default acoustic environment presets	503

A.4	VR filter design initialization vector	510

A.5	Octave band neural network parameters	510

A.6	Third-octave band neural network parameters	514

A.7	Third-octave GEQ design filter bandwidths	542

A.8	Constants for feedback matrix calculation	542

A.9	Dispersion filter coefficient template	543

A.10	freqVec(b) - STFT Band center frequencies	548

A.11	Closest center frequency bin for each one-third octave band frequency	549

A.12	EQbin	549

A.13	Fast convolution	550

A.13.1	Uniformly partitioned overlap-save convolution	550

A.13.2	Fast stereo convolution	551

A.14	Support element lookup tables	552

A.15	Airflow default frequency profiles	562

A.16	Reverberation Extent Mesh Definitions	573

A.17	Headphone Equalization Preset Responses	575

A.18	Listener Voice Default Directivity Pattern	576

A.19	Portal LoS Data Decoder Tables	577

A.20	Reverberator output directions	578

Annex B (informative)	581

B.1	Encoder overview	581

B.2	Encoder modules	582

B.2.1	Scene Configuration parameters	582

B.2.2	Audio plus metadata creation	582

B.2.3	Reverberation parametrization	582

B.2.4	Default acoustic environment (Default AE)	582

B.2.5	Low complexity early reflection parametrization	583

B.2.6	Portal creation in implicit portal mode	585

B.2.6.1	Creation of the geometry of the portal	585

B.2.6.2	Identification of the connection state between two portals	585

B.2.6.3	Creation of the portal struct containing all its metadata to be encoded	586

B.2.7	Line-of-sight data creation in explicit portal mode	586

B.2.8	Source/geometry staticity analysis	587

B.2.9	Diffraction edges and paths analysis	587

B.2.10	Early reflection surfaces and sequences analysis	587

B.2.11	Module data collection	587

B.2.12	Module data serialization	587

B.3	Listener space description format (LSDF)	588

B.4	Encoder input format (EIF)	588

B.5	Accessibility User Interface	588

B.6	Guidance on Own Voice Usage – Influence of System Delay	589

Bibliography	592

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Technical Committee [or Project Committee] ISO/TC [or ISO/PC] ###, [name of committee], Subcommittee SC ##, [name of subcommittee].

This second/third/… edition cancels and replaces the first/second/… edition (ISO #####:####), which has been technically revised.

The main changes are as follows:

—	xxx xxxxxxx xxx xxxx

A list of all parts in the ISO ##### series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

The document contains a best-effort standards description of the winning technology submitted to the CfP and is known to be incomplete at this point due to either (a) limitations in writing time or (b) the style of CfP technology evaluation (use of Audio Evaluation Platform as opposed to a real-world application environment). Passages that are known to be incomplete are marked with the searchable label [TO BE AMENDED] and are planned to be described fully by the time of the delivery of the final RM0 Working Draft text also containing the merged technology from the low bitrate category winning system.

Introduction

MPEG-I Immersive Audio reproduction with 6 degrees of freedom (6DoF) movement of the listener in an audio scene enables the experience of virtual acoustics in a Virtual Reality (VR) or Augmented Reality (AR) simulation. Audio effects and phenomena known from real-world acoustics like, for example, localization, distance attenuation, reflections, reverberation, occlusion, diffraction and the Doppler effect are modelled by a renderer that is controlled through metadata transmitted in a bitstream with additional input of interactive listener position data.

Along with other parts of MPEG-I (i.e., Part 12, “Immersive Video”, Part 5, “Visual Volumetric Video-based Coding (V3C) and Video-based Point Cloud Compression” and Part 2, “Systems Support”), the suite of standards supports a complete audio-visual VR or AR presentation in which the user can navigate and interact with the simulated environment using 6DoF, that being spatial navigation (x, y, z) and user head orientation (yaw, pitch, roll).

While VR presentations impart the feeling that the user is actually present in the virtual world, AR enables the enrichment of the real world by virtual elements that are perceived seamlessly as being part of the real world. The user can interact with the virtual scene or virtual elements and, in response, cause sounds that are perceived as realistic and matching the users’ experience in the real world.

This document provides means for rendering a real-time interactive audio presentation while permitting the user to have 6DoF movement. It defines metadata to support this rendering and a bitstream syntax that enables efficient storage and streaming of the MPEG-I Immersive Audio content.

The International Organization for Standardization (ISO) draws attention to the fact that it is claimed that compliance with this document may involve the use of a patent.

ISO takes no position concerning the evidence, validity and scope of this patent right.

The holder of this patent right has assured ISO that he/she is willing to negotiate licences under reasonable and non-discriminatory terms and conditions with applicants throughout the world. In this respect, the statement of the holder of this patent right is registered with ISO. Information may be obtained from the patent database available at www.iso.org/patents.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights other than those in the patent database. ISO shall not be held responsible for identifying any or all such patent rights.

Title (MPEG-I — Main element — Part #: Audio)

# Scope

This document specifies technology that supports the real-time interactive rendering of an immersive virtual or augmented reality audio presentation while permitting the user to have 6DoF movement in the audio scene. It defines metadata to support this rendering and a bitstream syntax that enables efficient storage and streaming of Immersive Audio content.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 23008-3, High efficiency coding and media delivery in heterogeneous environments — Part 3: 3D audio

ISO 9613-1, Attenuation of sound during propagation outdoors — Part 1: Calculation of the absorption of sound by the atmosphere

ISO 266 Acoustics — Preferred frequencies

# Terms and definitions

## Acronyms

| 6DoF | 6-Degrees-of-Freedom |
| --- | --- |
| AE | Acoustic Environment |
| AR | Augmented Reality |
| CoRI | Consolidation of Render Items |
| CV | Configuration Variable |
| DOA | Direction-Of-Arrival |
| DSR | Diffuse-to-Source energy Ratio |
| EIF | Encoder Input Format |
| EP | Extent Processor |
| EQ | Equalizer |
| ER | Early Reflections |
| ES | Extended Source |
| ESD | Equivalent Spatial Domain |
| FDN | Feedback Delay Network |
| HRIR | Head-Related Impulse Responses |
| HRTF | Head Related Transfer Function |
| IACC | Interaural Cross Correlation |
| IALD | Interaural Level Differences |
| IAPD | Interaural Phase Differences |
| ID | Identifier of an entity in the scene |
| IIR | Infinite Impulse Response |
| LC-ER | Low Complexity Early Reflections |
| LCS | Listener Coordinate System |
| LoS | Line-of-Sight |
| LSDF | Listener Space Description Format |
| MP-HOA | Multi Point Higher Order Ambisonics |
| RDR | Reverberant-to-Direct Ratio |
| RT60 | Reverberation Time for reverberation energy to drop 60 dB |
| RI | Render Item |
| RIR | Room Impulse Response |
| SESS | Spatially Extended Sound Sources |
| SN3D | Data exchange format for Ambisonics (Schmidt semi-normalisation) |
| SO | Scene Object |
| SOFA | Spatially Oriented Format for Acoustics |
| SOS | Second-Order-Section |
| SP-HOA | Single Point Higher Order Ambisonics |
| SRIR | Spatial Room Impulse Response |
| VBAP | Vector-Base-Amplitude-Panning |
| VDL | Variable Delay Line |
| VR | Virtual Reality |

## Terms

### auralize

Note: PCM waveform synthesis of immersive audio effects.

### audio element

Note: Audio signal data corresponding to the three signal types (audio objects, channels, HOA).

### audio scene

Note: All audio elements, acoustic elements and acoustic environment which are needed to render the sound in the scene.

### control workflow

Note: All components of the Renderer that are not part of the Rendering Workflow (i.e. the Scene and all related components)

### cover

Note: A frequency dependant quasi-uniform grid on the surface of a unit sphere whereupon each point is a scalar value in dB.

### conditional scene update

Note: A scene update initiated by the renderer when a certain condition is true.

### Doppler effect

Note: Pitch change of sound perceived when distance between sound source and listener changes.

### dynamic scene update

Note: An update triggered by external entity that includes the values of the attributes to be updated.

### effective spatial extent

Note: The part of an extent that is acoustically relevant given a specific listening position relative to the extent.

### exterior representation

Note: The exterior representation of an audio element is what is rendered when the listener is outside the extent of the audio element and is based on a source-centric format.

### interior representation

Note: The interior representation of an audio element is what is rendered when the listener is inside the extent of the audio element and is based on a listener-centric format.

### listener centric format

Note: An audio format for representing an audio element that is rendered to reproduce a sound field around the listener, such as HOA or channel-based formats like, e.g., 5.1 or 7.4.1.

### limiter

Note: A processing block that prevents clipping of the multi-channel output signal of the spatializer.

### location

Note: 3DoF location of an object in space in Cartesian coordinates.

### metadata

Note: All input and state parameters that are used to calculate the acoustic events of a virtual environment.

### MPEG-H 3DA decoder

Note: MPEG-H 3D Audio Low Complexity (LC) Profile decoder that receives as input an MPEG-H 3D Audio LC Profile MHAS stream and provides as output decoded PCM audio together with all metadata available in the MHAS packets. Decoded PCM audio contains channels, objects and reconstructed HOA as described in ISO/IEC 23008-3:2018 clause 17.10.

### orientation

Note: 3DoF rotation of an object in Tait-Bryan angles (yaw, pitch, roll).

### portal

Note: Portals model spatial transfer regions where there is a possible acoustic link that can transfer acoustic energy from one AE into another AE for reverberation and/or coupled transmission.

### position

Note: Consists of 6DoF location + orientation.

### primary  ray

Note: Rays used by the system to explore the scene geometry, the results of casting Primary Rays stored and addressed individually by unique ray IDs.

### primary RI

Note: A RI that is directly derived from an audio element in the scene.

### renderer

Note: The entire software specified in this standard.

### renderer pipeline

Note: The collection of stages, which sequentially perform audio and metadata processing.

### rendering workflow

Note: The control and multithreading of the Renderer Pipeline and the Spatializer.

### render item

Note: Any audio element in the Renderer pipeline. Abbreviated with RI.

### RI type

Note: RI have an associated type, denoting their relevance for the pipeline stages.

### scene

Note: The Scene is the aggregate of all entities that represent the acoustic events of a virtual environment modeled in the renderer through metadata.

### scene controller

Note: The Renderer processing block that holds and updates the Scene State.

### scene object

Note: Any entity in the scene (geometry, audio element, listener).

### scene state

Note: The Scene State reflects the current state of all 6DoF metadata of the scene.

### secondary ray

Note: Auxiliary rays used to refine the results based on casting the Primary Rays, the results of casting Secondary Rays are aggregated and stored under the respective primary ray IDs.

### secondary RI

Note: RI representing additional aspects of another RI, called its primary RI (e.g. mirror sources for modelling early reflections, sources at diffracting edges for modelling diffraction, extended sources corresponding to ray bundles having the same occlusion material lists).

### source-centric format

Note: An audio format for representing an audio element that is rendered such that all direct sound from the audio element appears to radiate from a bounded region in space that does not include the listener.

### spatializer

Note: A processing block situated directly after the renderer pipeline to produce a multi-channel output signal for a specific playback method (e.g. headphones or loudspeakers).

### spatially-heterogeneous audio element

Note: An audio element which has an extent and a source signal with more than one channel.

### stage

Note: A processing block in the Renderer Pipeline that addresses a dedicated rendering aspect.

### teleport

Note: Instant change of listener position triggered by user interaction within VR environment.

### timed scene update

Note: A scene update executed by the renderer once at a fixed, predefined time.

### voxel sub-scene update

Note: A scene update executed by the renderer and applied to a particular available voxel sub-scene including the currently rendered one.

### triggered scene update

Note: A scene update triggered manually from an external entity and executed by the renderer immediately after receiving the trigger.

### user

Note: The listener whose position is input to the render.

### voxel

Note: The geometry element defining a volume on a regular three-dimensional grid.

### voxel sub-scene

Note: A scene containing scene geometry representation based on voxels and including one or more independent voxel sub-scenes.

## Mnemonics

The following mnemonics are defined to describe the different data types used in the coded bitstream payload.

bslbf	Bit string, left bit first, where “left” is the order in which bit strings are written in ISO/IEC 14496 (all parts). Bit strings are written as a string of 1s and 0s within single quote marks, for example '1000 0001'. Blanks within a bit string are for ease of reading and have no significance.

uimsbf	Unsigned integer, most significant bit first.

vlclbf	Variable length code, left bit first, where “left” refers to the order in which the variable length codes are written.

tcimsbf 	Two’s complement integer, most significant (sign) bit first.

New mnemonics have been added. These mnemonics are temporary and only used during the development period of the MPEG-I bitstream. The intent is to remove these in the future. The following mnemonics have been added.

cstring	A C style string; a sequence of ascii characters, in bytes, terminated with a null byte (0x00).

float	An IEEE 754 floating single point precision number.

# Overview

## Technical overview

The Renderer operates with a global sampling frequency of 48 kHz. Input PCM audio data with other sampling frequencies must be resampled to 48 kHz before processing. A block diagram of the MPEG-I architecture overview is shown in Figure 2. The overview illustrates how the Renderer is connected to external units like MPEG-H 3DA coded Audio Element bitstreams, the metadata MPEG-I bitstream and other interfaces. The MPEG-H 3DA coded Audio Elements are decoded by the MPEG-H 3DA Decoder. All Audio Elements (channels, objects and HOA) that are to be input into the renderer have a counterpart in the MPEG-I Immersive audio standard, namely so-called source types (see Figure 1): Objects are represented as object sources, equipped with many VR/AR specific properties. Channels are channel sources that are played back in the virtual world through a virtual loudspeaker setup. Finally, HOA sources can be rendered into the virtual world in two different ways: rendering one or more HOA sources individually with three degrees of freedom (user orientation) or rendering one or more HOA sources as a group with six degrees of freedom. For all three paradigms, encoded waveforms can preferably be carried over from MPEG-H 3D audio to MPEG-I Immersive audio directly without the need for any re-encoding and associated loss in quality.

Figure 1 — Correspondences in MPEG-H Audio and MPEG-I Immersive Audio.

The decoded audio is subsequently rendered together with the MPEG-I bitstream, which is described in clause 5. The MPEG-I bitstream carries the Audio Scene description and other metadata used by the Renderer. In addition, the Renderer has interfaces are available to access consumption environment information, scene updates during playback, user interactions and user position and orientation information. Following the MPEG-I architecture overview, a more detailed description of the Renderer and the Renderer pipeline is presented.

Figure 2 —MPEG-I architecture overview

The Renderer allows real-time auralization of complex 6DoF audio scenes where the user may directly interact with entities in the scene. To achieve this, the multithreaded software architecture is divided into several workflows and components.  A block diagram with all Renderer components is shown in Figure 3. The Renderer supports the rendering of VR as well as AR scenes. In case of VR and AR scenes, the rendering metadata and the Audio Scene information is obtained from the bitstream. In case of AR scenes, the listener space information is obtained as LSDF file (see B.3) during playback. The components in the diagram are briefly described in the following. A complete description of the rendering framework and processing is given in clause 6.4.

Figure 3 — MPEG-I immersive audio renderer components overview

The control workflow is the entry point of the Renderer and responsible for the interfaces with external systems and components.

Its main functionality is embedded in the scene controller component, which coordinates the state of all entities in the 6DoF scene and implements the interactive interfaces of the Renderer. The Scene supports external updates of modifiable properties of scene objects, as well as reading and parsing the LSDF (Listener Space Description Format, see B.3) files to complete the information in the bitstream. The Scene also keeps track of time- or location-dependent properties of scene objects (e.g. interpolated locations or listener proximity conditions).

The scene state always reflects the current state of all scene objects, including audio elements, transforms/anchors and geometry. Other components of the Renderer can subscribe to changes in the Scene State. Before rendering starts, all objects in the entire scene are created and their metadata is updated to the state that reflects the desired scene configuration at start of playback.

The stream manager provides a unified interface for renderer components to access audio streams associated with an audio element in the Scene State. Audio streams are input to the Render as PCM float samples. The source of an audio stream may for example be decoded MPEG-H audio streams or locally captured audio.

The clock provides an interface for renderer components to get the current scene time in seconds. The Clock input may for example be a synchronization signal from other subsystems or the internal wall clock of the renderer. The Clock input to the Scene is not related to audio synchronization.

The rendering workflow is producing PCM float audio output signals. It is separated from the Control Workflow and only the Scene State (for communicating any changes in the 6DoF scene) and the Stream Manager (for providing input audio streams) are accessible from the Rendering Workflow for communication between both workflows.

The renderer pipeline auralizes the input audio streams provided by the Stream Manager based on the current Scene State. The rendering is organized in a sequential pipeline, such that individual renderer stages implement independent perceptual effects and make use of the processing of preceding and subsequent stages.

The spatializer is situated after the Renderer Pipeline and auralizes the output of the Renderer stages to a single output audio stream suitable for the desired playback method (e.g. binaural or adaptive loudspeaker rendering).

The limiter provides clipping protection for the auralized multi-channel output signal.

Figure 4 illustrates the Renderer pipeline where each box represents a separate Renderer stage. The Renderer stages are instantiated during Renderer initialization. Renderer stages are computed in the sequence presented in the figure.

Figure 4 — Renderer pipeline

## MPEG-I Immersive Audio profiles and levels

### General

[TO BE AMENDED]

### Profiles

[TO BE AMENDED]

# MPEG-I Immersive Audio transport

## Overview

MPEG-I Audio introduces three additional MHASPacketType values and associated MHASPacketPayload for the existing MPEG-H 3DA MHAS stream to transport the data necessary for 6DoF rendering of MPEG-H audio content (channels, objects, HOA signals). The MHASPacketLabel of these packets is used to connect MPEG-H 3DA Audio content to its associated 6DoF scene data. MHAS Packets of the MHASPacketType PACTYP_MPEGI_CFG, PACTYP_MPEGI_UPD and PACTYP_MPEGI_PLD embed MPEG-I 6DoF scene data, mpegiSceneConfig, mpegiSceneUpdate and mpegiScenePayload in the MHASPacketPayload().

The mpegiSceneConfig packet is a lightweight packet for the MPEG-I bitstream. It provides all relevant information for the renderer to configure itself for initialization. It provides a mapping between identifiers for all entities within a scene such that the renderer can translate integer identifiers transmitted from the Update and Payload packets into human readable string identifiers. In scenarios where side-channels are present, the config packet shall detail the side-channel locations and what Payload packets are available via said side-channels.

The mpegiSceneUpdate packet communicates L1, L2 (i.e. changes to the entities in a scene which are known when the stream starts), and L3 updates (i.e. changes to the entities in a scene that are unknown when the stream starts).

The mpegiScenePayload packet is the main container for all “bulk” metadata in the MPEG-I Audio bitstream. It can contain, for example, Directivities, Geometries and other metadata for individual audio effects like Reverberation, Early Reflections or Diffraction. The distribution of payloads into Scene Payload packets can be organized by the encoder, taking into the account when and where the metadata is needed by the renderer, which metadata is essential for rendering, as well as maximum size of the packet, etc. In a server-client scenario, payloads can be loaded via a separate channel. For pure broadcast scenarios, Payload sizes can be restricted to save bandwidth.

In a server-client application, the MHASPacketType PACTYP_MPEGI_CFG shall be interleaved periodically with the MHAS audio packets in the broadcast stream, but the large PACTYP_MPEGI_PLD packets should be sent on request only.

To sync to the broadcast stream an MHAS sync packet PACTYP_SYNC shall be inserted before each mpegiSceneConfig() packet. The MPEG-I scene payload can be packaged into one or more mpegiScenePayload() packets. Fine granular interleaving between MPEG-I metadata and audio content can be achieved by distributing MPEG-I metadata into Interleaving MPEG-I metadata over multiple payload packets.

## Definitions

The bitstream syntax is based on ISO/IEC ISO/IEC 23008-3 (MPEG-H Part 3), Clause 5. Modifications and amendments to the existing bitstream syntax are listed below.

In environments that require byte alignment, MPEG-I Immersive audio configuration elements or payload elements that are not an integer number of bytes in length are padded at the end to achieve an integer byte count. This is indicated by the function ByteAlign()	.

The payloads in mpegiScenePayload can contain more bits than what the corresponding syntax defines. These can be bits beyond byte alignment, representing future extensions of payloads. Consequently, the implementation shall ignore any additional bits until payloadLength bytes have been parsed for any payload, as indicated by discardBits, before continuing with the next payload.

The syntax description contains so-called support elements. These are functions that are called to read and decode the raw bitstream into internal renderer parameters. Support elements are defined in 6.2.15.

noOfBits denotes the No. of bits used for transmission of a given parameter as noted in the respective bitstream syntax table containing said parameter.

## MHAS syntax

### Audio stream

- Table 1 — Syntax of mpegiAudioStream ()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| mpegiAudioStream() |  |  |
| { |  |  |
| bitstreamIdentifier; | 48 | cstring |
| bitstreamVersion; | 16 | uimsbf |
| while (bitsAvailable() != 0) { |  |  |
| mpegiAudioStreamPacket(); |  |  |
| } |  |  |
| } |  |  |

### Audio stream packet

- Table 2 — Syntax of mpegiAudioStreamPacket()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| mpegiAudioStreamPacket() |  |  |
| { |  |  |
| MHASPacketType; | 11 | uimsbf |
| MHASPacketLabel; | 2 | uimsbf |
| MHASPacketLength; | 59 | uimsbf |
| switch (MHASPacketType) { |  |  |
| case PACTYP_MPEGI_CFG: { |  |  |
| mpegiSceneConfig(); |  |  |
| break; |  |  |
| } |  |  |
| case PACTYP_MPEGI_UPD: { |  |  |
| mpegiSceneUpdate(); |  |  |
| break; |  |  |
| } |  |  |
| case PACTYP_MPEGI_PLD: { |  |  |
| mpegiScenePayload(); |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| ByteAlign(); |  |  |
| } |  |  |

#### Scene configuration

- Table 3 — Syntax of mpegiSceneConfig ()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| mpegiSceneConfig() |  |  |
| { |  |  |
| entityCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < entityCount; i++) { |  |  |
| integerId = GetID(); |  |  |
| stringId; | 8..* | cstring |
| } |  |  |
| delayBufferSize; | 3 | uimsbf |
| gainCullingThreshold; | 3 | uimsbf |
| enableCullingReflectionRI | 1 | bslbf |
| overrideSpeedOfSound; | 1 | bslbf |
| if (overrideSpeedOfSound) { |  |  |
| speedOfSound; | 13 | uimsbf |
| } |  |  |
| overrideTemperature; | 1 | bslbf |
| if (overrideTemperature) { |  |  |
| temperature; | 5 | uimsbf |
| } |  |  |
| overrideHumidity; | 1 | bslbf |
| if (overrideHumidity) { |  |  |
| humidity; | 4 | uimsbf |
| } |  |  |
| isSmallScene; | 1 | bslbf |
| FreqGridData() |  |  |
| hasEarlyTuningGain; | 1 | bslbf |
| if (hasEarlyTuningGain) { |  |  |
| earlyTuningGain; | 6 | uimsbf |
| } |  |  |
| useLowComplexityER; | 1 | bslbf |
| hasHeadphoneEQ; | 1 | bslbf |
| if (hasHeadphoneEQ) { |  |  |
| readHeadphoneEQ(); |  |  |
| } |  |  |
| hasLoudnessCullingThreshold; | 1 | bslbf |
| if (hasLoudnessCullingThreshold) { |  |  |
| loudnessCullingThreshold; | 6 | Uimsbf |
| } |  |  |
| hasConsolidationThreshold; | 1 | bslbf |
| if (hasConsolidationThreshold) { |  |  |
| consolidationThreshold; | 5 | uimsbf |
| } |  |  |
| enableLocallyCapturedAudio | 1 | bslbf |
| if (enableLocallyCapturedAudio) { |  |  |
| overrideListenerVoiceDirectivity; | 1 | bslbf |
| if (overrideListenerVoiceDirectivity) { |  |  |
| listenerVoiceDirectivityId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

- Table 4 Syntax of FreqGridData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| FreqGridData () |  |  |
| { |  |  |
| fgdNrGrids = GetCountOrIndex(); |  |  |
| for ( g = 0:fgdNrGrids – 1) |  |  |
| { |  |  |
| fgdMethod; | 2 | uimsbf |
| if (fgdMethod == 'Individual frequencies') |  |  |
| { |  |  |
| fgdNrBands[g] = GetCountOrIndex(); |  |  |
| for ( b = 0:fgdNrBands[g] – 1) |  |  |
| { |  |  |
| fgdCenterFreq[g][b] = GetFrequency(); |  |  |
| } |  |  |
| } else if (fgdMethod == 'Start-Hop-Amount') |  |  |
| { |  |  |
| fgdNrBands[g] = GetCountOrIndex(); |  |  |
| fgdCenterFreq[g][0] = GetFrequency(); |  |  |
| frequencyHop = LUT(frequencyHopCode); | var | vlclbf |
| for ( b = 1:fgdNrBands[g] – 1) |  |  |
| { |  |  |
| fgdCenterFreq[g][b] = |  |  |
| fgdCenterFreq[g][b - 1] * frequencyHop; |  |  |
| } |  |  |
| } else if (fgdMethod == 'Default banding') |  |  |
| { |  |  |
| fgdDefaultGrid; | 4 | uimsbf |
| if (fgdIsSubGrid) | 1 | bslbf |
| { |  |  |
| fgdDefaultGridOffset; | 3 | uimsbf |
| fgdDefaultGridNrBands; | 6 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

- Table 5 Syntax of readHeadphoneEQ()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| readHeadphoneEQ () |  |  |
| { |  |  |
| specType; | 3 | uimsbf |
| if (specType == “freqgain”) { |  |  |
| freqGridIdx = getCountOrIndex(); |  |  |
| nrBands = fgdNrBands[freqGridIdx]; |  |  |
| for (int i = 0; i < nrBands; i++) { |  |  |
| gains[i] = GetGain(); |  |  |
| } |  |  |
| } else if (specType == “parametricfilters”) { |  |  |
| freqGridIdx = getCountOrIndex(); |  |  |
| nrBands = fgdNrBands[freqGridIdx]; |  |  |
| for (int i = 0; i < nrBands; i++) { |  |  |
| filterType[i]; | 2 | uimsbf |
| gains[i] = GetGain(); |  |  |
| q[i]; | 7 | uimsbf |
| } |  |  |
| } else if (specType == “preset”) { |  |  |
| preset; | 7 | uimsbf |
| } |  |  |
| } |  |  |

#### Scene update

- Table 6 — Syntax of mpegiSceneUpdate()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| mpegiSceneUpdate() |  |  |
| { |  |  |
| updatesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < updatesCount; i++) { |  |  |
| modificationsCount = GetCountOrIndex(); |  |  |
| for (int j = 0; j < modificationsCount; j++) { |  |  |
| targetId = GetID(); |  |  |
| hasDuration; | 1 | bslbf |
| if (hasDuration) { |  |  |
| Duration = GetDuration(isLongRangeMode=True); |  |  |
| } |  |  |
| changesCount; | 5 | uimsbf |
| for (int k = 0; k < changesCount; k++) { |  |  |
| targetAttribute; | 6 | uimsbf |
| switch (targetAttribute) { |  |  |
| case 0: { |  |  |
| for (int l = 0; l < 3; l++) { |  |  |
| isPositionParameterVariable; | 1 | bslbf |
| if (isPositionParameterVariable) { |  |  |
| positionParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newPositionValue = GetDistance(isSmallScene); isNegative; if (isNegative) { newPositionValue = -newPositionValue } | 1 | bslbf |
| } |  |  |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 1: { |  |  |
| for (int l = 0; l < 3; l++) { |  |  |
| isOrientationParameterVariable; | 1 | bslbf |
| if (isOrientationParameterVariable) { |  |  |
| orientationParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newOrientationValue; | 32 | float |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 2: { |  |  |
| newCoordSpaceValue; | 1 | bslbf |
| break; |  |  |
| } |  |  |
| case 3: { |  |  |
| newActiveValue; | 1 | bslbf |
| break |  |  |
| } |  |  |
| case 4: { |  |  |
| isGainDbParameterVariable; | 1 | bslbf |
| if (isGainDbParameterVariable) { |  |  |
| gainDbParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newGainValue = GetGain(isHiPrecGain=True) |  |  |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 5: { |  |  |
| newSignalId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 6: { |  |  |
| newExtentId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 7: { |  |  |
| newDirectivityId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 8: { |  |  |
| newDirectivenessValue; | 8 | uimsbf |
| break; |  |  |
| } |  |  |
| case 9: { |  |  |
| newPlayValue; | 1 | bslbf |
| break; |  |  |
| } |  |  |
| case 10:{ |  |  |
| newGroupId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 11: { |  |  |
| newRegionId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 12: { |  |  |
| newSizeXValue = GetDistance(isSmallScene); |  |  |
| newSizeYValue = GetDistance(isSmallScene); |  |  |
| newSizeZValue = GetDistance(isSmallScene); |  |  |
| break; |  |  |
| } |  |  |
| case 13: ( |  |  |
| isSpeedParameterVariable; | 1 | bslbf |
| if (isSpeedParameterVariable) { |  |  |
| speedParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newSpeedValue; | 10 | uimsbf |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 14: { |  |  |
| newMaterialId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 15: { |  |  |
| newReverbGainDbValue = |  |  |
| GetGain(isHiPrecGain=True); |  |  |
| break; |  |  |
| } |  |  |
| case 16: { |  |  |
| newNoReverb; | 1 | bslbf |
| break; |  |  |
| } |  |  |
| case 17: ( |  |  |
| for (int l = 0; l < 3; l++) { |  |  |
| isGranParameterVariable; | 1 | bslbf |
| if (isGranParameterVariable) { |  |  |
| granPosParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newGranPosValue; | 16 | uimsbf |
| } |  |  |
| } |  |  |
| break; |  |  |
| case 18: { |  |  |
| isGranGrpNoParameterVariable; | 1 | bslbf |
| if (isGranGrpNoParameterVariable) { |  |  |
| granGrpNoParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newGranGrpNoValue; } | 8 | uimsbf |
| isGranGrpProbParameterVariable; | 1 | bslbf |
| if (isGranGrpProbParameterVariable) { |  |  |
| granGrpNoParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newGranGrpProbValue; | 16 | uimsbf |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 19: { |  |  |
| isLoudnessDbParameterVariable; | 1 | bslbf |
| if (isLoudnessDbParameterVariable) { |  |  |
| loudnessDbVariableIndex | 4 | uimsbf |
| } else { |  |  |
| newLoudnessDbValue; | 32 | float |
| } |  |  |
| } |  |  |
| case 20: { |  |  |
| for (int l = 0; l < 3; l++) { |  |  |
| isAudioSourceRescaleVariable; | 1 | bslbf |
| if (isAudioSourceRescaleVariable) { |  |  |
| audioSourceRescaleVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newAudioSourceRescaleValue = | var | vlclbf |
| LUT(decimalCode); |  |  |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 21: { |  |  |
| for (int l = 0; l < 6; l++) { |  |  |
| isToleranceParameterVariable; | 1 | bslbf |
| if (isToleranceParameterVariable) { |  |  |
| toleranceParameterVariableIndex; | 4 | uimsbf |
| } |  |  |
| else { |  |  |
| newToleranceParameterValue = | var | vlclbf |
| LUT(decimalCode); |  |  |
| } |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| updateType; | 2 | uimsbf |
| switch (updateType) { |  |  |
| case 0: { |  |  |
| timedUpdateId = GetID(); |  |  |
| timedUpdatedHasIndex; | 1 | bslbf |
| if (timedUpdateHasIndex) { |  |  |
| timedUpdateIndex = GetCountOrIndex(); |  |  |
| } |  |  |
| Time = GetDuration(isLongRangeMode=True); |  |  |
| break; |  |  |
| } |  |  |
| case 1: { |  |  |
| conditionalUpdateId = GetID(); |  |  |
| conditionalHasIndex; | 1 | bslbf |
| if (conditionalHasIndex) { |  |  |
| conditionalUpdateIndex = GetCountOrIndex(); |  |  |
| } |  |  |
| fireOn; | 1 | bslbf |
| conditionalHasDelay; | 1 | bslbf |
| if (conditionalHasDelay) { |  |  |
| conditionalDelay = |  |  |
| GetDuration(isLongRangeMode=True); |  |  |
| } |  |  |
| conditionUpdateId = GetID(); |  |  |
| break; |  |  |
| } |  |  |
| case 2: { |  |  |
| triggeredUpdateId = GetID(); |  |  |
| triggeredUpdateIndex = GetCountOrIndex(); |  |  |
| break; |  |  |
| } |  |  |
| case 3: { |  |  |
| dynamicUpdateId = GetID(); |  |  |
| dynamicUpdateIndex = GetCountOrIndex(); |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Scene payload

Table 7 — Syntax of mpegiScenePayload()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| mpegiScenePayload() |  |  |
| { |  |  |
| payloadId; | 16 | uimsbf |
| payloadCount; | 16 | uimsbf |
| for (int i = 0; i < payloadCount; i++) { |  |  |
| payloadType; | 11 | uimsbf |
| payloadLabel; | 2 | uimsbf |
| payloadLength; | 59 | uimsbf |
| switch (payloadType) { |  |  |
| case PLD_DIRECTIVITY: { |  |  |
| bitsRead = payloadDirectivity(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_DIFFRACTION: { |  |  |
| bitsRead = payloadDiffraction(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_EARLY_REFLECTIONS: { |  |  |
| bitsRead = payloadEarlyReflections(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_PORTAL: { |  |  |
| bitsRead = payloadPortal(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_REVERB: { |  |  |
| bitsRead = payloadReverb(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_AUDIO_PLUS: { |  |  |
| bitsRead = payloadAudioPlus(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_DISPERSION: { |  |  |
| bitsRead = payloadDispersion(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_VOX_DATA: { |  |  |
| bitsRead = payloadVoxData(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_AIRFLOW: { |  |  |
| bitsRead = payloadAirflow(); |  |  |
| break; |  |  |
| } case PLD_RASTERMAP: { bitsRead = payloadRasterMap(); break; } |  |  |
| case PLD_SCENE: { |  |  |
| bitsRead = payloadScene(); |  |  |
| break; |  |  |
| } |  |  |
| case PLD_GRANULAR: { |  |  |
| bitsRead = payloadGranular(); |  |  |
| break; |  |  |
| } |  |  |
| default: { |  |  |
| bitsRead = 0; |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| // Byte-alignment and potential extensions |  |  |
| nDiscardBits = (payloadLength << 3) – bitsRead; |  |  |
| discardBits; | nDiscardBits |  |
| } |  |  |
| } |  |  |

## Semantics

bitstreamIdentifier	This integer represents “MPEGI” in the form of a C string. It is used for development purposes to verify MPEG-I bitstreams. This is a prevention mechanism for reading other files by accident.

bitstreamVersion	This integer represents the version number for this bitstream. The integer changes with alongside the syntax to ensure that the renderer can correctly decode this bitstream. It is primarily used for development purposes whilst the syntax is in flux.

MHASPacketLabel	This element provides an indication of which packets belong together. For example, with using different labels, different MPEG-H 3D audio configuration structures may be assigned to particular sequences of MPEG-H 3D audio access units.

MHASPacketLength	This element indicates the length of the packet in bytes.

Table 8 — Value of MHASPacketType

| MHASPacketType | Value |
| --- | --- |
| PACTYP_MPEGI_CFG | 23 |
| PACTYP_MPEGI_UPD | 24 |
| PACTYP_ MPEGI_PLD | 25 |

mpegiSceneConfig ()	MPEG-I data structure for configuration

mpegiSceneUpdate ()	MPEG-I data structure for update

mpegiScenePayload ()	MPEG-I data structure for parameter payload

payloadId	This integer is the unique identifier of the payload packet. This is to distinguish it from other payload packets.

payloadCount	This integer indicates how many payloads are currently present in this packet.

payloadType	This integer denotes the type of the current payload.

Payload elements listed in Table 9 are defined in 6.2.3.

Table 9 — Value of payloadType

| payloadType | Value |
| --- | --- |
| PLD_DIRECTIVITY | 6 |
| PLD_DIFFRACTION | 10 |
| PLD_EARLY_REFLECTIONS | 11 |
| PLD_PORTAL | 12 |
| PLD_REVERB | 14 |
| PLD_AUDIO_PLUS | 15 |
| PLD_DISPERSION | 17 |
| PLD_VOX_DATA | 18 |
| PLD_AIRFLOW | 19 |
| PLD_RASTERMAP | 20 |
| PLD_GRANULAR | 22 |
| PLD_SCENE | 31 |

payloadLabel	This element is used to group multiple payloads together.

payloadLength	This element is the length of the payload in bytes.

bitsRead	Indicates the number of bits read by a payload element.

nDiscardBits	Indicates the number of bits to be read and discarded after a payload element.

discardBits	Indicates the bits to be read and discarded after a payload element.

entityCount	This value represents the number of entities that exist with identifiers.

integerId	This value represents the newly derived integer from the string identifier. All integerId values shall be unique.

stringId	This string is the original string found from the Encoder Input Format for this entity. The intention is to map strings to integers so that the rest of the bitstream can use integers as ids to the bitstream size. All stringId values shall be unique.

delayBufferSize	This element sets the size of the propagation delay buffers. The size must be large enough to handle the largest propagation delay that can occur in the scene.

Table 10 — Value of delayBufferSize

| Value | Buffer size (in samples) |
| --- | --- |
| 0 | 10000 |
| 1 | 20000 |
| 2 | 30000 |
| 3 | 50000 |
| 4 | 100000 |
| 5 | 200000 |
| 6 | 500000 |
| 7 | 1000000 |

gainCullingThreshold	This element sets a threshold at which a render item with a large attenuation (e.g., due to large distance attenuation) is deactivated. The deactivation threshold factor g can be calculated from the value v with g = 10v-10. With v ranging between 0 and 7, this leads to a deactivation threshold between -100 dB and -30 dB in increments of 10 dB.

enableCullingReflectionRI	A boolean flag that enables or disables the Reflection RI culling. This flag is recommended to be set false if the renderer is running in the Voxel mode.

overrideSpeedOfSound	This flag indicates whether the default speed of sound (340 m/s), used for the calculation of propagation delay, is overridden for this scene.

speedOfSound	This value sets the speed of sound.

overrideTemperature	This flag indicates whether the default temperature (20°C), used for the calculation of medium attenuation, is overridden for this scene.

temperature	This value sets the temperature. The temperature T in °C is calculated from the value v with T=5 v-50.

overrideHumidity	This flag indicates whether the default humidity (40%), used for the calculation of medium attenuation, is overridden for this scene.

humidity	This value sets the humidity. The humidity H in % is calculated from the value v with H=20v3.

isSmallScene	This flag indicates whether the scene is considered a small scene, where no individual dimension is greater than 100m.

fgdMethod	Indicates the method with which the frequency grid is coded.

Table 11 – Value of fgdMethod.

| bits | Meaning |
| --- | --- |
| 0b00 | Individual frequencies |
| 0b01 | Start-Hop-Amount |
| 0b10 | Default banding |
| 0b11 | Reserved |

fgdCenterFreq	Indicates the center frequency in Hz, for each band in each frequency grid.

frequencyHopCode	Code indicating the hop-factor for the frequency banding.

Table 12 – Value of frequencyHopCode.

| bits | frequencyHopCode |
| --- | --- |
| 0b0010 | 2(1/12) |
| 0b0011 | 2(1/6) |
| 0b0000 | 2(1/4) |
| 0b01 | 2(1/3) |
| 0b0001 | 2(1/2) |
| 0b11 | 21 |
| 0b10 | 22 |

fgdDefaultGrid	Field indicating which default grid to use as frequency banding.

fgdIsSubGrid	Flag indicating whether further data is present indicating a subset of the default grids.

fgdDefaultGridOffset	Indicates the (0-based) index of the first relevant frequency of the default grid that is used.

fgdDefaultGridNrBands	Indicates the number of bands used from the default grid.

fgdNrBands[g] = fgdDefaultGridNrBands + 1.

Table 13 – Value of fgdDefaultGrid.

| bits | fgdCenterFreq[g] in Hz | fgdNrBands[g] | Description |
| --- | --- | --- | --- |
| 0b0000 | {31.5, 63, 125, 250, 500, 1000, 2000, 4000, 8000, 16000} | 10 | Octave - ISO |
| 0b0001 | {25, 50, 100, 200, 400, 800, 1600, 3150, 6300, 12500} | 10 | Octave alternative |
| 0b0010 | {20, 25, 31.5, 40, 50, 63, 80, 100, 125, 160, 200, 250, 315, 400, 500, 630, 800, 1000, 1250, 1600, 2000, 2500, 3150, 4000, 5000, 6300, 8000, 10000, 12500, 16000, 20000} | 31 | 1/3 Octave – ISO |
| 0b0011 | {25, 100, 400, 1600, 6300} | 5 | 2 Octave – ISO |
| 0b0100 | {125, 250, 500, 1000, 2000, 4000} | 6 | Octave subset |
| 0b0101 | {25, 250, 2500} | 3 |  |
| 0b0110 | {27, 56, 89, 126, 168, 214, 265, 323, 387, 459, 539, 628, 727, 839, 963, 1101,1256, 1429, 1621, 1836, 2077, 2345, 2644, 2978, 3351, 3767, 4232, 4750, 5329, 5975, 6697, 7502, 8401, 9405, 10525, 11775, 13171, 14729, 16468, 18410, 20577} | 41 | 1 ERB scale |
| 0b0111 | {27, 89, 168, 265, 387, 539, 727, 963, 1256, 1621, 2077, 2644, 3351, 4232, 5329, 6697, 8401, 10525, 13171, 16468, 20577} | 21 | 2 ERB scale |
| 0b1000 | {50, 150, 250, 350, 450, 570, 700, 840, 1000, 1170, 1370, 1600, 1850, 2150, 2150, 2500, 2900, 3400, 4000, 4800, 5800, 7000, 8500, 10500, 13500} | 25 | Bark scale |
| 0b1001 | Reserved |  |  |
| 0b1010 | Reserved |  |  |
| 0b1011 | Reserved |  |  |
| 0b1100 | Reserved |  |  |
| 0b1101 | Reserved |  |  |
| 0b1110 | Reserved |  |  |
| 0b1111 | Reserved |  |  |

hasEarlyTuningGain	Flag indicating the presence of earlyTuningGain.

earlyTuningGain	Early reflection tuning gain in 0.5dB steps starting at -21.5dB,
atuning=(earlyTuningGain - 43) * 0.5dB.

useLowComplexityER	This flag indicates that the low complexity early reflection stage is used to render early reflections.

hasHeadphoneEQ	Flag indicating the presence of content creator target headphone equalization data;

specType	Integer indicating which method is used to specify the headphone equalization data.

Table 14 — Value of specType

| bits | specType |
| --- | --- |
| 0b000 | freqgain |
| 0b001 | parametricfilters |
| 0b010 | preset |
| 0b011 | Reserved |
| … | … |
| 0b111 | Reserved |

gains	Headphone gain in decibel at specified frequency.

filterType	Integer value indicating the type of parametric filter that is specified.

Table 15 – Value of filterType.

| bits | Meaning |
| --- | --- |
| 0b00 | Low Shelf |
| 0b01 | Peak |
| 0b10 | High Shelf |
| 0b11 | Reserved |

q			Quantized float value representing the Q of the parametric filter, with a value between 0 and 10. To dequantize it to a floating point value, use the following equation:

| floatVal=10*q2noOfBits | (1) |
| --- | --- |

preset	Integer value representing the pre-defined equalization spectral data.

Table 16 — Value of preset

| bits | preset |
| --- | --- |
| 0b0000000 | flat |
| 0b0000001 | classical |
| 0b0000010 | club |
| 0b0000011 | dance |
| 0b0000100 | midcut |
| 0b0000101 | large_hall |
| 0b0000110 | party |
| 0b0000111 | pop |
| 0b0001000 | reggae |
| 0b0001001 | rock |
| 0b0001010 | soft |
| 0b0001011 | ska |
| 0b0001100 | full_bass |
| 0b0001101 | soft_rock |
| 0b0001110 | full_treble |
| 0b0001111 | full_bass_treble |
| 0b0010000 | live |
| 0b0010001 | techno |
| 0b0010010 | RESERVED |
| ……………. | ………… |
| 0b1111111 | RESERVED |

hasLoudnessCullingThreshold	Flag indicating the presence of the threshold for loudness based culling.

loudnessCullingThreshold	Loudness culling threshold [-73, -10dB]; threshold = (-73 + loudnessCullingThreshold)dB, mapping the 6 bits values of [0, 63] to the [-73, -10] range.

hasConsolidationThreshold	Flag indicating the presence of consolidationThreshold

consolidationThreshold	Value for the perceptual threshold below which render items may be consolidated.

enableLocallyCapturedAudio	This flag indicates if the renderer should allow the inclusion of audio input local to the user into the rendered scene.

overrideListenerVoiceDirectivity	This flag indicates if the default directivity used for the Listener Voice should be overridden by one provided in the bitstream.

listenerVoiceDirectivityId	This ID corresponds to the directivity that should be used for the Listener Voice instead of the renderer default.

updatesCount	This value is the number of updates in this payload.

modificationsCount	This value is the number of modifications in this update.

targetId	This value is the unique identifier of the target entity which is being modified.

hasDuration	This flag indicates if the modification occurs over a period of time.

duration	This value is the total duration of the modification in seconds.

changesCount	This integer represents how many value changes there are in this modification.

targetAttribute	This integer indicates which attribute is being modified.

Table 17 — Value of targetAttribute

| bits | targetAttribute |
| --- | --- |
| 0b000000 | POSITION |
| 0b000001 | ORIENTATION |
| 0b000010 | COORDSPACE |
| 0b000011 | ACTIVE |
| 0b000100 | GAIN |
| 0b000101 | SIGNAL |
| 0b000110 | EXTENT |
| 0b000111 | DIRECTIVITY |
| 0b001000 | DIRECTIVENESS |
| 0b001001 | PLAY |
| 0b001010 | GROUP |
| 0b001011 | REGION |
| 0b001100 | SIZE |
| 0b001101 | SPEED |
| 0b001110 | MATERIAL |
| 0b001111 | REVERB GAIN |
| 0b010000 | NO REVERB |
| 0b010001 | GRANULARPOS |
| 0b010010 | GRANULARGROUPPROB |
| 0b010011 | LOUDNESS |
| 0b010100 | RESCALE |
| 0b010101 | TOLERANCE |
| 0b010110 | RESERVED |
| ……………. | ………… |
| 0b111111 | RESERVED |

isPositionParameterVariable	This flag indicates if the value is coming from the evaluation platform.

positionParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newPositionValue	This value is the new position value in meters for the target entity.

isNegative	This flag indicates if the newPositionValue is negative.

isOrientationParameterVariable	This flag indicates if the value is coming from the evaluation platform.

orientationParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newOrientationValue	This float is the new orientation value in degrees for the target entity.

newCoordSpaceValue	This flag is the new coordinated space value for the target entity.

Table 18 — Value of newCoordSpaceValue

| bits | newCoordSpaceValue |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

newActiveValue	This flag de/activates the rendering of target entity.

isGainDbParameterVariable	This flag indicates if the value is coming from the evaluation platform.

gainDbParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newGainValue	This value is the new gain value for the target entity.

newSignalId	This value is the new unique audio stream identifier for the target entity.

newExtentId	This value is the new unique geometry identifier for the extent attribute of the target entity.

newDirectivityId	This value is the new unique directivity identifier for the source directivity of the target entity.

newDirectivenessValue	This value is new directiveness value for the target entity. It ranges between 0.0 to 20.0. To dequantize it to a floating point value, use the following equation:

| floatVal=20*newDirectivenessValue2noOfBits | (2) |
| --- | --- |

newPlayValue	This flag indicates the new play value for the target entity.

newGroupId	This value represents the new unique HOA Group for the target HOA source.

newRegionId	This value represents the new unique geometry identifier for the region attribute of the target entity.

newSizeXValue	This value represents the new size (m) attribute in the X axis for the target primitive entity.

newSizeYValue	This value represents the new size (m) attribute in the Y axis for the target primitive entity.

newSizeZValue	This value represents the new size (m) attribute in the Z axis for the target primitive entity.

isSpeedParameterVariable	This flag indicates if the value is coming from the evaluation platform.

speedParameterVariable	This integer is the update value channel index which is supplied from the evaluation platform.

newSpeedValue	This value is the new speed value for the target entity.

newMaterialId	Indicates a unique ID of a material for the target entity.

newReverbGainDbValue 	This value is the new reverb gain dB value for the target entity.

newNoReverb	This flag indicates the new reverb state for the target entity.

isGranPosParameterVariable	This flag indicates if the value is coming from the evaluation platform.

granPosParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newGranPosValue	This value is the new granular position value for the target entity.

isGranGrpNoParameterVariable	This flag indicates if the value is coming from the evaluation platform.

granGrpNoParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newGranGrpNoValue	This value is the new granular group number for the target entity.

isGranGrpProbParameterVariable	This flag indicates if the value is coming from the evaluation platform.

granGrpProbParameterVariableIndex	This integer is the update value channel index which is supplied from the evaluation platform.

newGranGrpProbValue	This value is the new granular group probability value for the target entity. It ranges between 0.0 to 1000.0. To dequantize it to a floating point value, use the following equation:

floatVal=1000newGranGrpProbValue(2noOfBits-1)

isLoudnessDbParameterVariable	This flag indicates if the value is provided via renderer interface.

loudnessDbVariableIndex	This integer is the update value index which is provided via renderer interface.

newLoudnessDbValue	This value is the new loudness value [dB] for the target entity.

isAudioSourceRescaleVariable	This flag indicates if the value is provided via renderer interface.

audioSourceRescaleVariableIndex	This integer is the update value index which is provided via renderer interface.

newAudioSourceRescaleValue	This value is the new rescale factor value.

isToleranceParameterVariable	This flag indicates if the value is provided via renderer interface.

toleranceParameterVariableIndex	This integer is the update value index which is provided via renderer interface.

newToleranceParameterValue	This value is the new tolerance parameter target value.

updateType	This integer indicates an update to be of the following types: timed, conditional, dynamic, or triggered.

Table 19 — Value of updateType

| bits | updateType |
| --- | --- |
| 0b00 | TIMED |
| 0b01 | CONDITIONAL |
| 0b10 | TRIGGERED |
| 0b11 | DYNAMIC |

timedUpdateId	This value indicates the unique identifier for this timed update.

timedUpdateHasIndex	This flag indicates if the timed update has an index value.

timedUpdateIndex	This value is the index value for this timed update.

time	This value is the point in time in which the update begins in seconds.

conditionalUpdateId	This value is the unique identifier for this conditional update.

conditionHasIndex	This flag indicates if this conditional update has a unique index.

conditionalUpdateIndex	This value is the index value for this conditional update.

fireOn	This flag determines when this update is triggered. It is triggered when the state of this value is reached.

conditionalHasDelay	This flag indicates if the conditional update is delayed after the trigger.

conditionalDelay	This value is the delay in seconds between the update trigger and the actualization of the update itself.

conditionUpdateId	This value is the unique listener proximity condition identifier that this update is triggered upon.

triggeredUpdateId	This value is the unique identifier for this triggered update.

triggeredUpdateIndex	This value is the index value for this triggered update.

dynamicUpdateId	This value is the unique identifier for this dynamic update.

dynamicUpdateIndex	This value is the index value for this dynamic update.

# MPEG-I Immersive Audio renderer

## Definitions

## Syntax

### General

In environments that require byte alignment, MPEG-I Immersive Audio configuration elements or payload elements that are not an integer number of bytes in length are padded at the end to achieve an integer byte count. This is indicated by the function ByteAlign()	.

To round a floating point number to the nearest integer the round() function is used - halfway cases round up.

The syntax description contains so-called support elements. These are functions that are called to read and decode the raw bitstream into internal renderer parameters. Support elements are defined in 6.2.14.

### Generic Codebook

Some payloads like payloadEarlyReflections() utilize individual codebooks which are defined within the bitstream using the following syntax:

Table 20 — Syntax of genericCodebook()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| genericCodebook() |  |  |
| { |  |  |
| this.flagFixedLength; | 1 | uimsbf |
| this.flagOffset; | 1 | uimsbf |
| if (this.flagOffset) { |  |  |
| wordSizeOffset; | 6 | uimsbf |
| this.offset; | wordSizeOffset | uimsbf |
| } |  |  |
| else { |  |  |
| this.offset = 0; |  |  |
| } |  |  |
| this.wordSize; | 6 | uimsbf |
| if (this.flagFixedLength) { |  |  |
| numCodes = 1 << this.wordSize; |  |  |
| for (unsigned int n = 0; n < numCodes; n++) { |  |  |
| // initialize bool array of given length |  |  |
| this.codeList[n] = Bitarray(n, this.wordSize); |  |  |
| this.symbolList[n] = n + this.offset; |  |  |
| } |  |  |
| } |  |  |
| else { |  |  |
| Bitarray code = []; |  |  |
| this.codeList = traverseTreeDecode( code ); |  |  |
| for (int n = 0; n < this.codeList.size(); n++) { |  |  |
| rawList[n]; | this.wordSize | uimsbf |
| this.symbolList[n] = rawList[n] + this.offset; |  |  |
| } |  |  |
| } |  |  |
| return this; |  |  |
| } |  |  |

The code word list “codeList” is transmitted using the following recursive tree traversal algorithm:

Table 21 — Syntax of traverseTreeDecode()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| traverseTreeDecode(Bitarray code) |  |  |
| { |  |  |
| Bitarray codeList[]; |  |  |
| isLeaf; | 1 | uimsbf |
| if (isLeaf) { |  |  |
| codeList.append(code); |  |  |
| } |  |  |
| else { |  |  |
| Bitarray codeLeft = code; |  |  |
| Bitarray codeRight = code; |  |  |
| codeLeft.append(0); |  |  |
| codeLeft.append(1); |  |  |
| codeList.append( traverseTreeDecode( codeLeft ) ); |  |  |
| codeList.append( traverseTreeDecode( codeRight ) ); |  |  |
| } |  |  |
| return codeList; |  |  |
| } |  |  |

The keyword “Bitarray” is used as an alias for a bit sequence of a certain length. Furthermore, the keyword “append()” denotes a method which extends the length of the array by one or more elements, that are added at the end. The following listing shows an example implementation in C++:

#include <vector>

using namespace std;

struct Bitarray

{

vector<bool> bit_seq;

// default constructor; denoted by [] in syntax table

Bitarray() {}

// copy constructor

Bitarray(const Bitarray& rhs)

{

bit_seq = rhs.bit_seq;

}

// construct Bitarray from uimsbf value

Bitarray(unsigned int val, unsigned int len)

{

for (int n = len - 1; n >= 0; n--) {

bit_seq.push_back(val & (1 << n));

}

}

// append single bit

void append(bool bit)

{

bit_seq.push_back(bit);

}

// append bit sequence

void append(const Bitarray& rhs)

{

for (bool bit : rhs.bit_seq) {

bit_seq.push_back(bit);

}

}

// equality check

bool operator==(const Bitarray& rhs)

{

return bit_seq == rhs.bit_seq;

}

};

An instance “exampleCodebook” of such a codebook is created as follows:

exampleCodebook = genericCodebook();

In addition to the data fields of the returned data structure, generic codebooks have a method “get_symbol()” which reads in a valid code word from the bitstream, i.e. the nth element of codeList[], and returns the corresponding symbol, i.e. symbolList[n]. The usage of this method is indicated as follows:

exampleVariable = exampleCodebook.get_symbol();

### Directivity payloads syntax

Table 22 — Syntax of payloadDirectivity()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadDirectivity() |  |  |
| { |  |  |
| directivitiesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < directivitiesCount; i++) { |  |  |
| directivityId = GetID(); |  |  |
| directivityCodedLength; | 32 | uimsbf |
| coverSet(); | directivityCodedLength | bslbf |
| } |  |  |
| } |  |  |

Table 23 — Syntax of coverSet()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| coverSet() |  |  |
| { |  |  |
| direcCoverCount; | 6 | uimsbf |
| direcFreqQuantType; | 2 | uimsbf |
| for (int i = 0; i < direcCoverCount; i++) { |  |  |
| covers[i] = directivityCover(); |  |  |
| } |  |  |
| frdFinish(); |  | vlclbf |
| } |  |  |

Table 24 — Syntax of directivityCover()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| directivityCover() |  |  |
| { |  |  |
| direcUseRawBaseline; | 1 | uimsbf |
| freq = readQuantFrequency(); |  | vlclbf |
| dbStepIdx = frdReadUniform(12); |  | vlclbf |
| dbStep = (dbStepIdx + 1) * 0.25; |  |  |
| intPer90 = frdReadUniform(45); |  | vlclbf |
| intPer90 += 1; |  |  |
| minPosVal = round(-128.0 / dbStep); |  |  |
| maxPosVal = round(127.0 / dbStep); |  |  |
| posValCount = maxPosVal – minPosVal + 1; |  |  |
| elCnt, aziCntPerEl, coverWidth = generateSphereGrid(intPer90); |  |  |
| if (direcUseRawBaseline) { |  |  |
| cover = rawCover(); |  |  |
| } |  |  |
| else { |  |  |
| cover = optimizedCover(); |  |  |
| } |  |  |
| for (int ei = 0; ei < elCnt; ei++) { |  |  |
| for (int ai = 0; ai < aziCntPerEl[ei]; ai++) { |  |  |
| cover[ei][ai] *= dbStep; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 25 — Syntax of readQuantFrequency()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| readQuantFrequency() |  |  |
| { |  |  |
| freq1oIdxMin = -5; |  |  |
| freq1oIdxMax = 4; |  |  |
| freq3oIdxMin = -17; |  |  |
| freq3oIdxMax = 13; |  |  |
| freq6oIdxMin = -34; |  |  |
| freq6oIdxMax = 26; |  |  |
| if (direcFreqQuantType == 0) { |  |  |
| alphaSize = freq1oIdxMax – freq1oIdxMin + 1; |  |  |
| freqIdx = frdReadUniform(alphaSize); |  | vlclbf |
| freqIdx += freq1oIdxMin; |  |  |
| freq = 1000 * pow(2, freqIdx); |  |  |
| } |  |  |
| else if (direcFreqQuantType == 1) { |  |  |
| alphaSize = freq3oIdxMax – freq3oIdxMin + 1; |  |  |
| freqIdx = frdReadUniform(alphaSize); |  | vlclbf |
| freqIdx += freq3oIdxMin; |  |  |
| freq = 1000 * pow(2, freqIdx / 3); |  |  |
| } |  |  |
| else if (direcFreqQuantType == 2) { |  |  |
| alphaSize = freq6oIdxMax – freq6oIdxMin + 1; |  |  |
| freqIdx = frdReadUniform(alphaSize); |  | vlclbf |
| freqIdx += freq6oIdxMin; |  |  |
| freq = 1000 * pow(2, freqIdx / 6); |  |  |
| } |  |  |
| else { |  |  |
| freqIdx = frdReadUniform(24000); |  | vlclbf |
| freq = freqIdx + 1; |  |  |
| } |  |  |
| } |  |  |

Table 26 — Syntax of rawCover()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| rawCover() |  |  |
| { |  |  |
| minVal = frdReadUniform(posValCount); |  | vlclbf |
| minVal += minPosVal; |  |  |
| maxVal = frdReadUniform(posValCount); |  | vlclbf |
| maxVal += minPosVal; |  |  |
| valAlphabetSize = maxVal – minVal + 1; |  |  |
| for (int ei = 0; ei < elCnt; ei++) { |  |  |
| for (int ai = 0; ai < aziCntPerEl[ei]; ai++) { |  |  |
| cover[ei][ai] = frdReadUniform(valAlphabetSize); |  | vlclbf |
| cover[ei][ai] += minVal; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 27 — Syntax of optimizedCover()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| optimizedCover() |  |  |
| { |  |  |
| predictionOrder = frdReadUniform(5); |  | vlclbf |
| predictionOrder += 1; |  |  |
| if (predictionOrder != 5) { |  |  |
| coverResiduals[0][0] = frdReadUniform(posValCount); |  | vlclbf |
| } |  |  |
| minResidual = frdReadCompactUint(10); |  | vlclbf |
| maxResidual = frdReadCompactUint(10); |  | vlclbf |
| alph = maxResidual – minResidual + 1; |  |  |
| for (int ei = 0; ei < elCnt; ei++) { |  |  |
| for (int ai = 0; ai < aziCntPerEl[ei]; ai++) { |  |  |
| coverResiduals[ei][ai] = frdRead(alph); |  | vlclbf |
| coverResiduals[ei][ai] += minResidual; |  |  |
| } |  |  |
| } |  |  |
| cover = unpredict(coverResiduals, predictionOrder, covers[-1]); |  |  |
| } |  |  |

### Diffraction payload syntax

Table 28 — Syntax of diffractionPayload()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffractionPayload() |  |  |
| { |  |  |
| diffrVoxelGrid(); |  |  |
| diffrStaticEdgeList(); |  |  |
| diffrStaticPathDict(); |  |  |
| diffrListenerVoxelDict(); |  |  |
| diffrSourceVoxelDict(); |  |  |
| diffrValidPathDict(); |  |  |
| diffrDynamicEdges(); |  |  |
| diffrDynamicPaths(); |  |  |
| } |  |  |

Table 29 — Syntax of diffrVoxelGrid()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrVoxelGrid() |  |  |
| { |  |  |
| [diffrVoxelOriginX; |  |  |
| diffrVoxelOriginY; |  |  |
| diffrVoxelOriginZ;] = GetPosition(isSmallScene) |  |  |
| diffrVoxelPitchX = GetDistance(isSmallScene); |  |  |
| diffrVoxelPitchY = GetDistance(isSmallScene); |  |  |
| diffrVoxelPitchZ = GetDistance(isSmallScene); |  |  |
| diffrVoxelShapeX = GetID(); |  |  |
| diffrVoxelShapeY = GetID(); |  |  |
| diffrVoxelShapeZ = GetID(); |  |  |
| } |  |  |

Table 30 — Syntax of diffrStaticEdgeList()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrStaticEdgeList() |  |  |
| { diffrHasStaticEdgeData; if (diffrHasStaticEdgeData) { codebookEdgeID = genericCodebook(); codebookVtxID = genericCodebook(); codebookTriID = genericCodebook(); | 1 | uimsbf |
| numberOfStaticEdges = GetID(); |  |  |
| for (int i = 0; i < numberOfStaticEdges; i++) { |  |  |
| staticEdge[i] = diffrEdge(codebookEdgeID, codebookVtxID, codebookTriID); |  |  |
| } } |  |  |
| } |  |  |

Table 31 — Syntax of diffrEdges()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrEdge(codebookEdgeID, codebookVtxID, codebookTriID) |  |  |
| { |  |  |
| edgeId = codebookEdgeID.get_symbol(); |  | vlclbf |
| edgeVertexId1 = codebookVtxID.get_symbol(); |  | vlclbf |
| edgeVertexId2 = codebookVtxID.get_symbol(); |  | vlclbf |
| edgeAdjacentTriangleID1 = codebookTriID.get_symbol(); |  | vlclbf |
| edgeAdjacentTriangleID2 = codebookTriID.get_symbol(); |  | vlclbf |
| edgeIsRounded; | 1 | bslbf |
| edgeIsRelevant; | 1 | bslbf |
| } |  |  |

Table 32 — Syntax of diffrStaticPathDict()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrStaticPathDict() |  |  |
| { |  |  |
| diffrHasStaticPathData; | 1 | uimsbf |
| if (diffrHasStaticPathData) { |  |  |
| staticPathDict = diffrPathDict(); |  |  |
| } |  |  |
| } |  |  |

Table 33 — Syntax of diffrPathDict()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrPathDict() |  |  |
| { |  |  |
| codebookEdgeIDSeqLen = genericCodebook(); |  |  |
| codebookEdgeIDSeq = genericCodebook(); |  |  |
| codebookAngleSeq = genericCodebook(); |  |  |
| numBitsForAngle; | 6 | uimsbf |
| numberOfRelevantEdges = GetID(); |  |  |
| for (int i = 0; i < numberOfRelevantEdges; i++) { |  |  |
| numberOfPaths = GetID(); |  |  |
| for (int j = 0; j < numberOfPaths; j++) { |  |  |
| numberOfEdgesInPath = codebookEdgeIDSeqLen.get_symbol(); |  | vlclbf |
| for (int k = 0; i < numberOfEdgesInPath; k++) { |  |  |
| edgeId[i][j][k] = codebookEdgeIDSeq.get_symbol(); |  | vlclbf |
| faceIndicator[i][j][k]; | 1 | uimsbf |
| angle[i][j][k] = codebookAngleSeq.get_symbol(); |  | vlclbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 34 — Syntax of diffrListenerVoxelDict()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrListenerVoxelDict() |  |  |
| { |  |  |
| diffrHasListenerVoxelData; if (diffrHasListenerVoxelData) { x = -1; y = -1; z = -1; codebookVcX = genericCodebook(); codebookVcY = genericCodebook(); codebookVcZ = genericCodebook(); codebookNumEdges = genericCodebook(); codebookEdgeId = genericCodebook(); codebookIndicesRemoved = genericCodebook(); numberOfListenerVoxels = GetID(); | 1 | uimsbf |
| for (int i = 0; i < numberOfListenerVoxels; i++) { z += 1; hasVoxelCoordZ; if (hasVoxelCoordZ) { z = codebookVcZ.get_symbol(); y += 1; hasVoxelCoordY; if (hasVoxelCoordY) { y = codebookVcY.get_symbol(); x += 1; hasVoxelCoordX; if (hasVoxelCoordX) { x = codebookVcX.get_symbol(); } } } | 1 1 1 | uimsbf vlclbf uimsbf vlclbf uimsbf vlclbf |
| listenerVoxelGridIndexX[i] = x; |  |  |
| listenerVoxelGridIndexY[i] = y; |  |  |
| listenerVoxelGridIndexZ[i] = z; |  |  |
| diffrListenerVoxelMode[i]; bool remove_loop = diffrListenerVoxelMode[i] != 0; int k = 0; while (remove_loop) { diffrListenerVoxelIndexDiff[i][k] = codebookIndicesRemoved.get_symbol(); remove_loop = diffrListenerVoxelIndexDiff[i][k] != 0; k += 1; } numberOfEdgesAdded=codebookNumEdges.get_symbol(); for (int j = 0; j < numberOfEdgesAdded; j++) { diffrListenerVoxelEdge[i][j] = codebookEdgeId.get_symbol(); } | 2 | uimsbf vlclbf vlclbf vlclbf |
| } |  |  |
| } |  |  |
| } |  |  |

Table 35 — Syntax of diffrSourceVoxelDict()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrSourceVoxelDict() |  |  |
| { |  |  |
| diffrHasSourceVoxelData; if (diffrHasSourceVoxelData) { numberOfStaticSources = GetID(); | 1 | uimsbf |
| for (int i = 0; i < numberOfStaticSources; i++) { |  |  |
| staticSourceId = GetID(); |  |  |
| numberOfVoxelsPerStaticSource = GetID(); |  |  |
| for (int j = 0; j < numberOfVoxelsPerStaticSource; j++) { |  |  |
| sourceVoxelGridIndexX[i][j] = GetID(); |  |  |
| sourceVoxelGridIndexY[i][j] = GetID(); |  |  |
| sourceVoxelGridIndexZ[i][j] = GetID(); |  |  |
| numberOfEdgesPerSourceVoxel = GetID(); |  |  |
| for (k = 0; k < numberOfEdgesPerSourceVoxel; k++) { |  |  |
| sourceVisibleEdgeId[i][j][k] = GetID(); |  |  |
| } |  |  |
| } } |  |  |
| } |  |  |
| } |  |  |

Table 36 — Syntax of diffrValidPathDict()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrValidPathDict() |  |  |
| { |  |  |
| diffrHasValidPathData; if (diffrHasValidPathData) { numberOfValidStaticSources = GetID(); | 1 | uimsbf |
| for (int i = 0; i < numberOfValidStaticSources; i++) { |  |  |
| validStaticSourceId = GetID(); x = -1; y = -1; z = -1; codebookVcX = genericCodebook(); codebookVcY = genericCodebook(); codebookVcZ = genericCodebook(); codebookNumPaths = genericCodebook(); codebookEdgeId = genericCodebook(); codebookPathId = genericCodebook(); codebookIndicesRemoved = genericCodebook(); |  |  |
| numberOfMaximumListenerVoxels = GetID(); |  |  |
| for (int j = 0; j < numberOfMaximumListenerVoxels; j++) { z += 1; hasVoxelCoordZ; if (hasVoxelCoordZ) { z = codebookVcZ.get_symbol(); y += 1; hasVoxelCoordY; if (hasVoxelCoordY) { y = codebookVcY.get_symbol(); x += 1; hasVoxelCoordX; if (hasVoxelCoordX) { x = codebookVcX.get_symbol(); } } } | 1 1 1 | uimsbf vlclbf uimsbf vlclbf uimsbf vlclbf |
| validListenerVoxelGridIndexX[i][j] = x; |  |  |
| validListenerVoxelGridIndexY[i][j] = y; |  |  |
| validListenerVoxelGridIndexZ[i][j] = z; |  |  |
| diffrValidPathMode[i][j]; bool remove_loop = diffrValidPathMode[i][j] != 0; int k = 0; while (remove_loop) { diffrValidPathIndexDiff[i][j][k] = codebookIndicesRemoved.get_symbol(); remove_loop = diffrValidPathIndexDiff[i][j][k] != 0; k += 1; } numberOfPathsAdded[i][j] = codebookNumPaths.get_symbol(); for (int k = 0; k < numberOfPathsAdded[i][j]; k++) { diffrValidPathEdge[i][j][k] = codebookEdgeId.get_symbol(); diffrValidPathPath[i][j][k] = codebookPathId.get_symbol(); } | 2 | uimsbf vlclbf vlclbf vlclbf vlclbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 37 — Syntax of diffrDynamicEdges()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrDynamicEdges() |  |  |
| { |  |  |
| diffrHasDynamicEdgeData; if (diffrHasDynamicEdgeData) { dynamicGeometryCount = GetID(); | 1 | uimsbf |
| for (int i = 0; i < dynamicGeometryCount; i++) { |  |  |
| geometryId[i] = GetID(); codebookEdgeID = genericCodebook(); codebookVtxID = genericCodebook(); codebookTriID = genericCodebook(); dynamicEdgesCount = GetID(); |  |  |
| for (int j = 0; j < dynamicEdgesCount; j++) { |  |  |
| dynamicEdge[i][j] = diffrEdge(codebookEdgeID, codebookVtxID, codebookTriID); |  |  |
| } } |  |  |
| } } |  |  |

Table 38 — Syntax of diffrDynamicPaths()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| diffrDynamicPaths() |  |  |
| { |  |  |
| diffrHasDynamicPathData; if (diffrHasDynamicPathData) { dynamicGeometryCount = GetID(); | 1 | uimsbf |
| for (int i = 0; i < dynamicGeometryCount; i++) { |  |  |
| relevantGeometryId = GetID(); |  |  |
| dynamicPathDict[g] = diffrPathDict(); |  |  |
| } } |  |  |
| } |  |  |

### Voxel payload syntax

Table 39 — Syntax of payloadVoxData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadVoxData() |  |  |
| { |  |  |
| numberOfVoxSubScenes = escapedValue(8, 16, 32) + 1; |  |  |
| for (int i = 0; i < numberOfVoxSubScenes; i ++) { |  |  |
| voxSceneHeader(); |  |  |
| voxSceneData(); |  |  |
| } |  |  |
| } |  |  |

Table 40 — Syntax of voxSceneHeader()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneHeader() |  |  |
| { |  |  |
| voxScenePosS = GetPosition(isSmallScene); |  |  |
| voxScenePosE = GetPosition(isSmallScene); |  |  |
| for (int i = 0; i < 3; i++) { |  |  |
| voxSceneDimensions[i]; | 16 | uimsbf |
| } |  |  |
| } |  |  |

Table 41 — Syntax of voxSceneData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneData() |  |  |
| { |  |  |
| voxSceneElements(); |  |  |
| voxSceneMaterials(); |  |  |
| voxSceneSettings(); |  |  |
| voxSceneUpdates(); |  |  |
| } |  |  |

Table 42 — Syntax of voxSceneElements()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneElements() |  |  |
| { numberOfVoxSceneElements = escapedValue(8, 16, 32); for (int i = 0; i < numberOfVoxSceneElements; i++) { voxSceneElementMatID[i] = escapedValue(4, 8, 32); voxSceneElementPosPackedS[i]; voxSceneElementPosPackedE[i]; } | NbitsVox a NbitsVox a | uimsbf uimsbf |
| } |  |  |

a	NbitsVox = ceil(log2(prod(voxSceneDimensions)-1)

Table 43 — Syntax of voxSceneMaterials()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneMaterials() |  |  |
| { |  |  |
| numberOfVoxSceneMaterials = escapedValue(8, 16, 32); |  |  |
| for (int i = 0; i < numberOfVoxSceneMaterials; i++) { |  |  |
| voxSceneMaterialID = escapedValue(8, 16, 32); |  |  |
| } |  |  |
| } |  |  |

Table 44 — Syntax of voxSceneSettings()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneSettings() |  |  |
| { |  |  |
| voxSceneInstructionsForOcclusion(); |  |  |
| voxSceneInstructionsForDiffraction(); |  |  |
| voxSceneInstructionsForReflections(); |  |  |
| } |  |  |

Table 45 — Syntax of voxSceneInstructionsForOcclusion()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneInstructionsForOcclusion() |  |  |
| { |  |  |
| voxSceneOcclusionInstructionsPresentFlag; | 1 | bslbf |
| if (voxSceneOcclusionInstructionsPresentFlag) { |  |  |
| for (int i = 0; i < numberOfVoxSceneMaterials; i++) { |  |  |
| voxSceneOcclusionRecoveryFlag[i]; | 1 | bslbf |
| if (voxSceneOcclusionRecoveryFlag[i]) { |  |  |
| voxSceneOcclusionRecoveryValue[i]; | 32 | float |
| } |  |  |
| } |  |  |
| } |  |  |
| voxSceneOcclusionSourceMaterialDependency(); } |  |  |

Table 46 — Syntax of voxSceneOcclusionSourceMaterialDependency()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneOcclusionSourceMaterialDependency() |  |  |
| { |  |  |
| voxSceneOcclusionSourceMaterialDependencyFlag; | 1 | bslbf |
| if (voxSceneOcclusionSourceMaterialDependencyFlag) { |  |  |
| numberOfSourceMaterialEntries = escapedValue(8, 16, 32); |  |  |
| for (int i = 0; i < numberOfSourceMaterialEntries; i++) { |  |  |
| voxSceneOcclusionSkipForObjectID[i] = escapedValue(8, 16, 32); |  |  |
| voxSceneOcclusionSkipForMaterialID[i] = escapedValue(8, 16, 32); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 47 — Syntax of voxSceneInstructionsForDiffraction()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneInstructionsForDiffraction() |  |  |
| { |  |  |
| voxSceneDiffractionInstructionsPresentFlag; | 1 | bslbf |
| if (voxSceneDiffractionInstructionsPresentFlag) { |  |  |
| for (int i = 0; i < numberOfVoxSceneMaterials; i++) { |  |  |
| voxSceneDiffractionEnabledFlag[i]; | 1 | bslbf |
| } |  |  |
| } |  |  |
| voxSceneDiffractionEffectControlFlag; | 1 | bslbf |
| if (voxSceneDiffrationEffectControlFlag) { |  |  |
| voxSceneDiffractionEffectControlValue; | 32 | float |
| } |  |  |
| voxSceneDiffractionMapPresentFlag; | 1 | bslbf |
| if (voxSceneDiffractionMapPresentFlag) { |  |  |
| voxSceneDiffractionMap(); |  |  |
| } else { |  |  |
| if (voxSceneDiffractionMapHeightPresentFlag) { | 1 | bslbf |
| voxSceneDiffractionMapHeight; | 16 | uimsbf |
| } |  |  |
| } |  |  |
| voxSceneDiffractionPreComputedPathData(); |  |  |
| } |  |  |

Table 48 — Syntax of voxSceneDiffractionMap()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneDiffractionMap() |  |  |
| { |  |  |
| numberOfVoxDiffractionMapElements = escapedValue(8, 16, 32); |  |  |
| for (int i = 0; i < numberOfVoxDiffractionMapElements; i++) { |  |  |
| voxDiffractionMapValue[i]; | 1 | bslbf |
| voxDiffractionMapPosPackedS[i]; | NbitsMap a | uimsbf |
| voxDiffractionMapPosPackedE[i]; | NbitsMap a | uimsbf |
| } |  |  |
| } |  |  |

a	NbitsMap = ceil(log2(voxSceneDimensions[0]*voxSceneDimensions[1]-1)

Table 49 — Syntax of voxSceneUpdates()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneUpdates() |  |  |
| { |  |  |
| numberOfVoxSceneUpdates = escapedValue(8, 16, 32); |  |  |
| for (int i = 0; i < numberOfVoxSceneUpdates; i++) { |  |  |
| voxSceneUpdateID[i] = escapedValue(8, 16, 32); |  |  |
| voxSceneUpdateReferenceFlag[i]; | 1 | bslbf |
| if (voxSceneUpdateReferenceFlag[i]) { |  |  |
| voxSceneUpdateReferencePos[i] = GetPosition(isSmallScene) |  |  |
| } |  |  |
| numberOfVoxSceneUpdateSequences[i] = escapedValue(8, 16, 32); |  |  |
| for (int j = 0; j < numberOfVoxSceneUpdateSequences[i]; j++) { |  |  |
| voxSceneUpdateTime[i][j]; | 16 | uimsbf |
| voxSceneElements(); |  |  |
| voxSceneSettings(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 50 — Syntax of voxSceneDiffractionPreComputedPathData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxSceneDiffractionPreComputedPathData() |  |  |
| { |  |  |
| if (pcpdPresent) { | 1 | bslbf |
| pcpdNumStartPositions = escapedValue(8,16,32) |  |  |
| if (pcpdFull) { | 1 | bslbf |
| pcpdDecodeFull(pcpdNumStartPositions) |  |  |
| } else { |  |  |
| pcpdDecodeSelective(pcpdNumStartPositions) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 51 — Syntax of pcpdDecodeFull()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| pcpdDecodeFull(pcpdNumStartPositions) |  |  |
| { |  |  |
| for (int i = 0; i < pcpdNumStartPositions; ++i) { |  |  |
| if (!pcpdDataPresentForPos) { | 1 | bslbf |
| continue; |  |  |
| } |  |  |
| pcpdStartVoxelPacked; | NbitsMap a | uimsbf |
| for (int x = 1; x <= voxSceneDimensions[0]; ++x) { |  |  |
| for (int y = 1; y <= voxSceneDimensions[1]; ++y) { |  |  |
| if (pcpdPathExistFlag) { | 1 | bslbf |
| if (!pcpdUsePreviousPath) { | 1 | bslbf |
| pcpdSourceDirectionPacked; | NbitsMap a | uimsbf |
| pcpdPathLength; | 32 | float |
| } else { |  |  |
| pcpdPathDelta; | 2 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

a	NbitsMap = ceil(log2(voxSceneDimensions[0]*voxSceneDimensions[1]-1)

Table 52 — Syntax of pcpdDecodeSelective()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| pcpdDecodeSelective(pcpdNumStartPositions) |  |  |
| { |  |  |
| for (int i = 0; i < pcpdNumStartPositions; ++i) { |  |  |
| pcpdNumEndPositions = escapedValue(8,16,32) |  |  |
| pcpdStartVoxelPacked; | NbitsMap a | uimsbf |
| for (int j = 0; j < pcpdNumEndPositions; ++j) { |  |  |
| pcpdEndVoxelPacked; | NbitsMap a | uimsbf |
| if (pcpdPathExistFlag) { | 1 | bslbf |
| if (!pcpdUsePreviousSourceDirection) { | 1 | bslbf |
| pcpdSourceDirectionPacked; | NbitsMap a | uimsbf |
| } |  |  |
| pcpdPathLength; | 32 | float |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

a	NbitsMap = ceil(log2(voxSceneDimensions[0]*voxSceneDimensions[1]-1)

Syntax of escapedValue() shall be as defined in ISO/IEC 23003-3

Table 53 — Syntax of voxReflectionConfigurationParameters()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| voxReflectionConfigurationParameters() |  |  |
| { |  |  |
| voxReflectionNumOriginsPresentFlag; | 1 | bslbf |
| if (voxReflectionNumOriginsPresentFlag) { |  |  |
| voxReflectionNumOrigins; | 5 | uimsbf |
| } |  |  |
| voxReflectionNumClustersPresentFlag; | 1 | bslbf |
| if (voxReflectionNumClustersPresentFlag) { |  |  |
| voxReflectionNumClusters; | 5 | uimsbf |
| } |  |  |
| voxReflectionTuningIndoorOutdoorTypeFlag; | 1 | bslbf |
| if (voxReflectionTuningIndoorOutdoorTypeFlag) { |  |  |
| voxReflectionEarlyTuningIndoorGainDb = GetGain(isHiPrecGain=True); voxReflectionEarlyTuningOutdoorGainDb = GetGain(isHiPrecGain=True); |  |  |
| } |  |  |
| voxReflectionSectorDownmixFlag; | 1 | bslbf |
| } |  |  |

### Early reflection payload syntax

Table 54 — Syntax of payloadEarlyReflections()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadEarlyReflections() |  |  |
| { |  |  |
| earlyTriangleCullingDistanceOrder1; | 8 | uimsbf |
| earlyTriangleCullingDistanceOrder2; | 8 | uimsbf |
| earlyTriangleSourceDistanceOrder1; | 8 | uimsbf |
| earlyTriangleSourceDistanceOrder2; | 8 | uimsbf |
| [earlyVoxelGridOriginX; |  |  |
| earlyVoxelGridOriginY; |  |  |
| earlyVoxelGridOriginZ;] = GetPosition(isSmallScene) |  |  |
| earlyVoxelGridPitchX = GetDistance(isSmallScene); |  |  |
| earlyVoxelGridPitchY = GetDistance(isSmallScene); |  |  |
| earlyVoxelGridPitchZ = GetDistance(isSmallScene); |  |  |
| earlyVoxelGridShapeX; | 32 | uimsbf |
| earlyVoxelGridShapeY; | 32 | uimsbf |
| earlyVoxelGridShapeZ; | 32 | uimsbf |
| earlyHasSurfaceData; | 1 | uimsbf |
| if (earlyHasSurfaceData) { |  |  |
| earlySurfaceDataLength; | 32 | uimsbf |
| earlySurfaceData(); |  |  |
| } |  |  |
| earlyHasVoxelData; | 1 | uimsbf |
| if (earlyHasVoxelData) { |  |  |
| earlyVoxelDataLength; | 32 | uimsbf |
| earlyVoxelData(); |  |  |
| } |  |  |
| } |  |  |

Table 55 — Syntax of earlySurfaceData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| earlySurfaceData() |  |  |
| { |  |  |
| codebookLengthFaceIdx = genericCodebook(); |  |  |
| codebookFaceIdx = genericCodebook(); |  |  |
| codebookAzi = genericCodebook(); |  |  |
| codebookEle = genericCodebook(); |  |  |
| codebookDist = genericCodebook(); |  |  |
| earlySurfaceDistOffset; | 22 | tcimsbf |
| numberOfSurfaces; | 16 | uimsbf |
| for (int s = 0; s < numberOfSurfaces; s++) { |  |  |
| earlySurfaceLengthFaceIdx[s] = codebookLengthFaceIdx.get_symbol(); |  | vlclbf |
| for (int f = 0; f < earlySurfaceLengthFaceIdx[s]; f++) { |  |  |
| earlySurfaceFaceIdx[s][f] = codebookFaceIdx.get_symbol(); |  | vlclbf |
| } |  |  |
| earlySurfaceAzi[s] = codebookAzi.get_symbol(); |  | vlclbf |
| earlySurfaceEle[s] = codebookEle.get_symbol(); |  | vlclbf |
| earlySurfaceDist[s] = codebookDist.get_symbol(); |  | vlclbf |
| } |  |  |
| } |  |  |

Table 56 — Syntax of earlyVoxelData()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| earlyVoxelData() |  |  |
| { |  |  |
| codebookL = genericCodebook(); |  |  |
| codebookS = genericCodebook(); |  |  |
| codebookIndicesRemoved = genericCodebook(); |  |  |
| codebookNumPaths = genericCodebook(); |  |  |
| codebookOrder = genericCodebook(); |  |  |
| codebookSurf = genericCodebook(); |  |  |
| numberOfVoxelPairs; | 32 | uimsbf |
| for (int v = 0; v < numberOfVoxelPairs; v++) { |  |  |
| earlyVoxelL[v] = codebookL.get_symbol(); |  | vlclbf |
| earlyVoxelS[v] = codebookS.get_symbol(); |  | vlclbf |
| earlyVoxelMode[v]; | 2 | uimsbf |
| bool remove_loop = earlyVoxelMode[v] != 0; |  |  |
| int k = 0; |  |  |
| while (remove_loop) { |  |  |
| earlyVoxelIndicesRemovedDiff[v][k] = codebookIndicesRemoved.get_symbol(); |  | vlclbf |
| remove_loop = earlyVoxelIndicesRemovedDiff[v][k] != 0; |  |  |
| k += 1; |  |  |
| } |  |  |
| earlyVoxelNumPaths[v] = codebookNumPaths.get_symbol(); |  | vlclbf |
| for (int p = 0; p < earlyVoxelNumPaths[v]; p++) { |  |  |
| earlyVoxelOrder[v][p] = codebookOrder.get_symbol(); |  | vlclbf |
| for (int o = 0; o < earlyVoxelOrder[v][p]; o++) { |  |  |
| earlyVoxelSurf[v][p][o] = codebookSurf.get_symbol(); |  | vlclbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 57 — Syntax of payloadLcEarlyReflections()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadLcEarlyReflections() { |  |  |
| lcEarlyNumberOfEnvironments; | 16 | uimsbf |
| for (int i = 0; s < lcEarlyNumberOfEnvironments; i++) { |  |  |
| lcEarlyEnvType; | 4 | uimsbf |
| if (lcEarlyEnvType == LCERoutdoor) { |  |  |
| lcEarlyEnvironmentPriority; | 8 | uimsbf |
| lcEarlyAmpFac1Db = GetGain(true); |  |  |
| lcEarlyAmpFac2Db = GetGain(true); |  |  |
| lcEarlyPreDelay1; | 16 | uimsbf |
| lcEarlyPreDelay2; | 16 | uimsbf |
| lcEarlyBoundingBox(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 58 — Syntax of lcEarlyBoundingBox()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| lcEarlyBoundingBox() { |  |  |
| [lcEarlyBoundingBoxPositionX; |  |  |
| lcEarlyBoundingBoxPositionY; |  |  |
| lcEarlyBoundingBoxPositionZ;] = GetPosition(isSmallScene) |  |  |
| [lcEarlyBoundingBoxOrientationYaw; |  |  |
| lcEarlyBoundingBoxOrientationPitch; |  |  |
| lcEarlyBoundingBoxOrientationRoll] = GetOrientation(); |  |  |
| [[lcEarlyBoundingBoxSizeX; |  |  |
| lcEarlyBoundingBoxSizeY; |  |  |
| lcEarlyBoundingBoxSizeZ;] = GetPosition(isSmallScene); |  |  |
| } |  |  |

### Portal payload syntax

Table 59 — Syntax of payloadPortal()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadPortal() |  |  |
| { |  |  |
| numPortals = GetCountOrIndex(); |  |  |
| if (numPortals > 0) { |  |  |
| isExplicitPortalMode; | 1 | bslbf |
| } |  |  |
| for (int i = 0; i < numPortals; i++) { |  |  |
| portalId = GetID(); |  |  |
| if (hasExtent) { | 1 | bslbf |
| portalExtentId = GetID(); |  |  |
| } |  |  |
| if (isExplicitPortalMode) { | 1 | bslbf |
| if (isConnectedToAE) { | 1 | bslbf |
| portalEnv1 = GetID(); |  |  |
| if (isConnectedToSecondAE) { | 1 | bslbf |
| portalEnv2 = GetID(); |  |  |
| } |  |  |
| } |  |  |
| if (hasMaterial) { | 1 | bslbf |
| portalMaterialId = GetID(); |  |  |
| } |  |  |
| portalNormal = GetPosition(true); |  |  |
| if (explicitPortalProperties) { | 1 | bslbf |
| portalCenterPos = GetPosition(isSmallScene); |  |  |
| equivalentPortalWidth = GetDistance(isSmallScene); |  |  |
| equivalentPortalHeight = GetDistance(isSmallScene); |  |  |
| equivalentPortalDepth = GetDistance(isSmallScene); |  |  |
| } |  |  |
| if (hasPortal2PortalFactors) { | 1 | bslbf |
| for (int j = 0; j < i; j++) { |  |  |
| if ((portalEnv1[i]==portalEnv1[j] |  |  |
| && portalEnv2[i] != portalEnv2[j]) |  |  |
| ¦¦ (portalEnv2[i] == portalEnv2[j] |  |  |
| && portalEnv1[i] != portalEnv1[j]) |  |  |
| ¦¦ (portalEnv1[i] == portalEnv2[j] |  |  |
| && portalEnv2[i] != portalEnv1[j]) |  |  |
| ¦¦ (portalEnv2[i] == portalEnv1[j] |  |  |
| && portalEnv1[i] != portalEnv2[j])) { |  |  |
| if (hasAcousticLink) { | 1 | bslbf |
| portal2PortalFactor[i][j] = pow(10, portal2PortalFactorCode / 10); | var | vlclbf |
| portal2PortalFactor[j][i] = pow(10, portal2PortalFactorCode / 10); | var | vlclbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| portalParentEnvId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| if (isExplicitPortalMode) { |  |  |
| if (hasAeLosVoxelData) { | 1 | bslbf |
| numAEs = GetCountOrIndex(); |  |  |
| for (int i = 0; i < numAEs; i++) { |  |  |
| aeBsId = GetCountOrIndex(); |  |  |
| losVoxelDataStruct(); |  |  |
| } |  |  |
| } |  |  |
| if (hasOutsideLosVoxelData) { | 1 | bslbf |
| numOutsideAreas = GetCountOrIndex(); |  |  |
| for (int i = 0; i < numOutsideAreas; i++) { |  |  |
| losVoxelDataStruct(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 60 — Syntax of losVoxelDataStruct()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| losVoxelDataStruct() |  |  |
| { |  |  |
| nVoxelsX = GetCountOrIndex(); |  |  |
| nVoxelsY = GetCountOrIndex(); |  |  |
| nVoxelsZ = GetCountOrIndex(); |  |  |
| cornerP1 = GetPosition(isSmallScene); |  |  |
| cornerP2 = GetPosition(isSmallScene); |  |  |
| voxelSideLenCm = GetCountOrIndex(); |  |  |
| portalCount = GetCountOrIndex(); |  |  |
| for (j = 0; j < portalCount; j++) { |  |  |
| portalBsId = GetID(); |  |  |
| nLosBitMasks = GetCountOrIndex(); |  |  |
| for (k = 0; k < nLosBitMasks; k++) { |  |  |
| nMaskBytes = GetCountOrIndex(); |  |  |
| for (l = 0; l < nMaskBytes; l++) { |  |  |
| maskByte[l]; | 8 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Reverberation payload syntax

Table 61 — Syntax of payloadReverb()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadReverb() |  |  |
| { |  |  |
| if (revpresent) | 1 | bslbf |
| { |  |  |
| revNrElements = GetCountOrIndex(); |  |  |
| for ( e = 0; e < revNrElements; e++) { |  |  |
| revAcEnvID = GetID(); |  |  |
| revFreqGridIdx[e] = GetCountOrIndex(); |  |  |
| revPredelay[e] = GetDuration(false); |  |  |
| if (revDelayLineCountPresent) | 1 | bslbf |
| { |  |  |
| revDelayLineCount = LUT(delayLineCountCode) | var | vlclbf |
| } |  |  |
| for ( b = 0:fgdNrBands[revFreqGridIdx[e]] – 1) |  |  |
| { |  |  |
| revRT60[e][b] = GetDuration(false); |  |  |
| } |  |  |
| for ( b = 0:fgdNrBands[revFreqGridIdx[e]] – 1) { |  |  |
| revDSR[e][b] = LUT(dsrCode); | var | vlclbf |
| } |  |  |
| } |  |  |
| if (revRenderControlPresent) { | 1 | bslbf |
| revDistanceGainDropDb = GetGain(true); |  |  |
| revMinDistance = GetDistance(true); |  |  |
| } |  |  |
| if (revMultiRoomConfigPresent) { | 1 | bslbf |
| if (revFadeEnabled) | 1 | bslbf |
| { |  |  |
| revFadeDistance = GetDistance(true); |  |  |
| } |  |  |
| revHeadtrackingEnabled; | 1 | bslbf |
| } |  |  |
| if (revARConfigPresent) { | 1 | bslbf |
| revLsdfTheoreticalRDRtoactualRDRdB = GetGain(true); |  |  |
| revLsdfRDRtoActualRDRdB = GetGain(false); |  |  |
| revOverrideLSDF_RDR; | 1 | bslbf |
| } |  |  |
| } |  |  |
| if (revDefaultAEPresent) { | 1 | bslbf |
| if (revHasCustomDefaultAE) { | 1 | bslbf |
| revDefaultAEFreqGridIdx = GetCountOrIndex(); |  |  |
| revDefaultAEPredelay = GetDuration(false); |  |  |
| revDefaultAEDelayRatio; | 32 | float |
| revDefaultAEDelayMinMs; | 32 | float |
| for ( b = 0:fgdNrBands[defaultAEFreqGridIdx] – 1) |  |  |
| { |  |  |
| revDefaultAERT60[b] = GetDuration(false); |  |  |
| } |  |  |
| for ( b = 0:fgdNrBands[defaultAEFreqGridIdx] – 1) |  |  |
| { |  |  |
| revDefaultAEDSR[b] = LUT(dsrCode); |  |  |
| } |  |  |
| revDefaultAEDistanceGainDropDb = GetGain(true); |  |  |
| revDefaultAEMinimumDistance = GetDistance(true); |  |  |
| } else { |  |  |
| revDefaultAEPreset; | 6 | uimsbf |
| if (revHasDefaultAEGain) | 1 | bslbf |
| revDefaultAEGain = GetGain(true); |  |  |
| if (revHasDefaultAEDelayRatio) | 1 | bslbf |
| revDefaultAEDelayRatio; | 32 | float |
| if (revHasDefaultAEDelayMinMs) | 1 | bslbf |
| revDefaultAEDelayMinMs; | 32 | float |
| if (revHasDefaultAEDistanceGainDrop) | 1 | bslbf |
| revDefaultAEDistanceGainDropDb = GetGain(true); |  |  |
| if (revHasDefaultAEMinDist) | 1 | bslbf |
| revDefaultAEMinimumDistance = GetDistance(true); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Audio plus payload syntax

Table 62 — Syntax of payloadAudioPlus()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadAudioPlus() |  |  |
| { |  |  |
| numberOfSources = GetCountOrIndex() |  |  |
| for (int i = 0; i < numberOfSources; i++) { |  |  |
| audioSourceId = GetID(); |  |  |
| distanceGainModel; | 1 | bslbf |
| hasDiffuseness; | 1 | bslbf |
| if (hasDiffuseness) { |  |  |
| diffuseness; | 7 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |

### Dispersion payload syntax

Table 63 — Syntax of payloadDispersion ()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadDispersion() |  |  |
| { |  |  |
| enableDispersionFilterER; | 1 | bslbf |
| enableDispersionFilterDiffr; | 1 | bslbf |
| enableFilter = enableDispersionFilterER ¦¦ enableDispersionFilterDiffr; |  |  |
| if (enableFilter) { |  |  |
| codedDispersionFilterLength; | 4 | uimsbf |
| dispersionFilterLength = (codedDispersionFilterLength + 1) * 2; |  |  |
| codedDispersionFilterGain; | 5 | uimsbf |
| dispersionFilterGaindB = codedDispersionFilterGain – 21; |  |  |
| enableSpatialDispersion | 1 | bslbf |
| } |  |  |
| } |  |  |

### Scene plus payload syntax

Table 64 — Syntax of payloadScene()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadScene() |  |  |
| { |  |  |
| sceneDuration; | 8 | uimsbf |
| sceneType; | 1 | bslbf |
| transforms(); |  |  |
| anchors(); |  |  |
| audioStreams(); |  |  |
| granularSounds(); |  |  |
| materials(); |  |  |
| directivities(); |  |  |
| primitives(); |  |  |
| meshes(); |  |  |
| environments(); |  |  |
| objectSources(); |  |  |
| hoaGroups(); |  |  |
| hoaSources(); |  |  |
| channelSources(); |  |  |
| airflowGeneratorObjects(); |  |  |
| } |  |  |

Table 65 — Syntax of transforms()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| transforms() |  |  |
| { |  |  |
| transformsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < transformsCount; i++) { |  |  |
| transformId = GetID(); |  |  |
| [transformPositionX; |  |  |
| transformPositionY; |  |  |
| transformPositionZ;] = GetPosition(isSmallScene); |  |  |
| [transformOrientationYaw; |  |  |
| transformOrientationPitch; |  |  |
| transformOrientationRoll;] = Getorientation() |  |  |
| transformCoordSpace; | 1 | bslbf |
| } |  |  |
| } |  |  |

Table 66 — Syntax of anchors()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| anchors() |  |  |
| { |  |  |
| anchorsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < anchorsCount; i++) { |  |  |
| anchorId = GetID(); |  |  |
| anchorLsdfRef; | 8..* | cstring |
| [audioSourceRescaleX; |  |  |
| audioSourceRescaleY; |  |  |
| audioSourceRescaleZ] = GetAudioSourceRescale(); |  |  |
| autoRescale; | 1 | bool |
| [toleranceXMax; |  |  |
| toleranceXMin; |  |  |
| toleranceYMax; |  |  |
| toleranceYMin; |  |  |
| toleranceZMax; |  |  |
| toleranceZMin] = GetTolerance(); |  |  |
| } |  |  |
| } |  |  |

Table 67 — Syntax of audioStreams()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| audioStreams() |  |  |
| { |  |  |
| audioStreamsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < audioStreamsCount; i++) { |  |  |
| audioStreamId; = GetID() |  |  |
| if (enableLocallyCapturedAudio) { |  |  |
| isLocallyCaptured; | 1 | bslbf |
| } else { |  |  |
| isLocallyCaptured = false; |  |  |
| } |  |  |
| if (isLocallyCaptured == false) { |  |  |
| audioStreamFilePath; | 8..* | cstring |
| inputChannelsCount = GetCountOrIndex(); |  |  |
| for (int j = 0; j < inputChannelsCount; j++) { |  |  |
| inputChannelIndex = GetCountOrIndex(); |  |  |
| } |  |  |
| isSoundEffect; | 1 | bslbf |
| if (isSoundEffect) { |  |  |
| streamDuration = GetDuration(isLongRangeMode=True); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 68 — Syntax of granularSounds()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| granularSounds () |  |  |
| { |  |  |
| granularSoundsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < granularSoundsCount; i++) { |  |  |
| GranularSoundId = GetID() |  |  |
| GranularDatabaseFilePath; | 8..* | cstring |
| granularSoundChannels = GetCountOrIndex(); |  |  |
| } |  |  |
| } |  |  |

Table 69 — Syntax of materials()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| materials() |  |  |
| { |  |  |
| materialsCount = GetCountorIndex(); |  |  |
| for (int i = 0; i < materialsCount; i++) { |  |  |
| materialId = GetID(); |  |  |
| materialCodedLength; | 32 | uimsbf |
| materialCoeffs(); | materialCodedLength | bslbf |
| } |  |  |
| } |  |  |

Table 70 — Syntax of materialCoeffs()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| materialCoeffs() |  |  |
| { |  |  |
| frdReadUniform(18); // skip |  | vlclbf |
| materialCount = frdReadCompactUint(10); |  | vlclbf |
| assert(materialCount == 1); |  |  |
| stepIdx = frdReadCompactUint(10); |  | vlclbf |
| cxStep = round(10000.0/ stepIdx); |  |  |
| materialFreqGridIdx = frdReadUniform(128) |  | vlclbf |
| materialFreqCount = fgdNrBands[materialFreqGridIdx] |  |  |
| spcRflIs0; | 1 | uimsbf |
| diffScatIs0; | 1 | uimsbf |
| trnsIs0; | 1 | uimsbf |
| cplIs0; | 1 | uimsbf |
| for (int i = 0; i < materialFreqCount; i++) { |  |  |
| freq = fgdCenterFreq[materialFreqGridIdx][i] |  |  |
| specularReflection = materialCoeff(spcRflIs0, cxStep); |  |  |
| diffusedScattering = materialCoeff(diffScarIs0, cxStep); |  |  |
| transmission = materialCoeff(trnsIs0, cxStep); |  |  |
| coupling = materialCoeff(cplIs0, cxStep); |  |  |
| } |  |  |
| reserved; | 32 | uimsbf |
| } |  |  |

Table 71 — Syntax of materialCoeff()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| materialCoeff(is0, cxStep) |  |  |
| { |  |  |
| materialCoeff = 0; |  |  |
| if (!is0) { |  |  |
| materialCoeff = frdReadUniform(cxStep + 1) / cxStep; |  | vlclbf |
| } |  |  |
| return materialCoeff; |  |  |
| } |  |  |

Table 72 — Syntax of sourceDirectivities()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| sourceDirectivities() |  |  |
| { |  |  |
| sourceDirectivitiesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < sourceDirectivitiesCount; i++) { |  |  |
| sourceDirectivityId = GetID(); |  |  |
| } |  |  |
| } |  |  |

Table 73 — Syntax of primitives()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| primitives() |  |  |
| { |  |  |
| primitivesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < primitivesCount; i++) { |  |  |
| primitiveType; | 2 | uimsbf |
| primitiveId = GetID(); |  |  |
| [primitivePositionX; |  |  |
| primitivePositionY; |  |  |
| primitivePositionZ] = GetPosition(isSmallScene); |  |  |
| [primitiveOrientationYaw; |  |  |
| primitiveOrientationPitch; |  |  |
| primitiveOrientationRoll] = GetOrientation(); |  |  |
| primitiveCoordSpace; | 1 | bslbf |
| primitiveSizeX = GetDistance(isSmallScene); |  |  |
| primitiveSizeY = GetDistance(isSmallScene); |  |  |
| primitiveSizeZ = GetDistance(isSmallScene); |  |  |
| primitiveHasMaterial; | 1 | bslbf |
| if (primitiveHasMaterial) { |  |  |
| primitiveMaterialId = GetID(); |  |  |
| } |  |  |
| primitiveHasSpatialTransform; | 1 | bslbf |
| if (primitiveHasSpatialTransform) { |  |  |
| primitiveHasAnchor; | 1 | bslbf |
| if (primitiveHasAnchor) { |  |  |
| primitiveParentAnchorId = GetID(); |  |  |
| } |  |  |
| else { |  |  |
| primitiveParentTransformId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| isPrimitiveStatic; | 1 | bslbf |
| isEarlyReflectionPrimitive; | 1 | bslbf |
| } |  |  |
| } |  |  |

Table 74 — Syntax of meshes()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| meshes() |  |  |
| { |  |  |
| meshesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < meshesCount; i++) { |  |  |
| meshId = GetID(); |  |  |
| meshCodedLength; | 32 | uimsbf |
| meshFaces(); | meshCodedLength | bslbf |
| [meshPositionX; |  |  |
| meshPositionY; |  |  |
| meshPositionZ;] = GetPosition(isSmallScene) |  |  |
| [meshOrientationYaw; |  |  |
| meshOrientationPitch; |  |  |
| meshOrientationRoll;] = GetOrientation() |  |  |
| meshCoordSpace; | 1 | bslbf |
| meshHasSpatialTransform; | 1 | bslbf |
| if (meshHasSpatialTransform) { |  |  |
| meshHasAnchor; | 1 | bslbf |
| if (meshHasAnchor) { |  |  |
| meshParentAnchorId = GetID(); |  |  |
| } |  |  |
| else { |  |  |
| meshParentTransformId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| isMeshStatic; | 1 | bslbf |
| isEarlyReflectionMesh; | 1 | bslbf |
| } |  |  |
| } |  |  |

Table 75 — Syntax of meshFaces()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| meshFaces() |  |  |
| { |  |  |
| vertexCount = frdReadUniform(18); |  | vlclbf |
| faceCount = frdReadUniform(18); |  | vlclbf |
| frdReadUniform(10); // skip |  |  |
| vertexQuantStep = frdReadCompactUint(10) / 10000.0; |  | vlclbf |
| bbBits; | 5 | uismbf |
| bbBits += 1; |  |  |
| boudingBox(); |  |  |
| preprocessGCD(); |  |  |
| vertexList(); |  |  |
| faceList(); |  |  |
| reserved; | 32 | uimsbf |
| } |  |  |

Table 76 — Syntax of boundingBox()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| boundingBox() |  |  |
| { |  |  |
| bb_x_min = boundingBoxDimension(); |  |  |
| bb_x_max = boundingBoxDimension(); |  |  |
| bb_y_min = boundingBoxDimension(); |  |  |
| bb_y_max = boundingBoxDimension(); |  |  |
| bb_z_min = boundingBoxDimension(); |  |  |
| bb_z_max = boundingBoxDimension(); |  |  |
| } |  |  |

Table 77 — Syntax of boundingBox()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| boundingBoxDimension() |  |  |
| { |  |  |
| bb_dim; | bbBits - 1 | uimsbf |
| bb_dim_sign; | 1 | uimsbf |
| if (bb_dim_sign) { |  |  |
| bb_dim = -bb_dim; |  |  |
| } |  |  |
| return bb_dim; |  |  |
| } |  |  |

Table 78 — Syntax of preprocessGCD()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| preprocessGCD() |  |  |
| { |  |  |
| gcdX = 1; |  |  |
| gcdY = 1; |  |  |
| gcdZ = 1; |  |  |
| gcdPresent; | 1 | uismbf |
| if (gcdPresent) { |  |  |
| gcdXPresent; | 1 | uismbf |
| if (gcdXPresent) { |  |  |
| gcdX = gcdX; | bbBits | uismbf |
| } |  |  |
| gcdYPresent; | 1 | uismbf |
| if (gcdYPresent) { |  |  |
| gcdY = gcdY; | bbBits | uismbf |
| } |  |  |
| gcdZPresent; | 1 | uismbf |
| if (gcdZPresent) { |  |  |
| gcdZ = gcdZ; | bbBits | uismbf |
| } |  |  |
| } |  |  |
| } |  |  |

Table 79 — Syntax of vertexList()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| vertexList() |  |  |
| { |  |  |
| for (int i = 0; i < vertexCount; i++) { |  |  |
| vertexX = vertex(); |  | vlclbf |
| vertexY = vertex(); |  | vlclbf |
| vertexZ = vertex(); |  | vlclbf |
| } |  |  |
| } |  |  |

Table 80 — Syntax of faceList()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| faceList() |  |  |
| { |  |  |
| alph_mat = frdRead(10) + 1; |  | vlclbf |
| alph_cmi = 0; |  |  |
| for (int i = 0; i < alph_mat; i++) { |  |  |
| if (i == alph_mat – 1 && alph_cmi == 0) { |  |  |
| is_present[i] = 1; |  |  |
| } |  |  |
| else { |  |  |
| is_present[i]; | 1 | uismbf |
| } |  |  |
| if (is_present[i]) { |  |  |
| cmi_to_mi.insert(i); |  |  |
| alph_cmi++; |  |  |
| } |  |  |
| } |  |  |
| for (int i = 0; i < faceCount; i++) { |  |  |
| v1[i] = frdReadUniform(vertexCount); |  | vlclbf |
| v2[i] = frdReadUniform(vertexCount); |  | vlclbf |
| v3[i] = frdReadUniform(vertexCount); |  | vlclbf |
| if (alph_cmi == 1) { |  |  |
| cmi = 0; |  |  |
| } |  |  |
| else { |  |  |
| same_cmi = frdRead(2); |  | vlclbf |
| if (same_cmi) { |  |  |
| cmi = last_cmi; |  |  |
| } |  |  |
| else { |  |  |
| if (cmi_alph == 2) { |  |  |
| cmi = 1 – last_cmi; |  |  |
| } |  |  |
| else { |  |  |
| cmi = frdRead(alph_cmi); |  | vlclbf |
| } |  |  |
| last_cmi = cmi; |  |  |
| } |  |  |
| } |  |  |
| mi[i] = cmi_to_mi[cmi] – 1; |  |  |
| } |  |  |
| } |  |  |

Table 81 — Syntax of environments()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| environments () |  |  |
| { |  |  |
| environmentsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < environmentsCount; i++) { |  |  |
| acousticParametersCount = 0; | 1 | bslbf |
| for (int j = 0; j < acousticParametersCount; j++) { |  |  |
| [paramPosX; |  |  |
| paramPosY; |  |  |
| paramPosZ] = GetPosition(isSmallScene); |  |  |
| } |  |  |
| environmentId = GetID(); |  |  |
| acousticRegionId = GetID(); |  |  |
| } |  |  |
| } |  |  |

Table 82 — Syntax of objectSources()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| objectSources() |  |  |
| { |  |  |
| objectSourcesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < objectSourcesCount; i++) { |  |  |
| hasInputLayout; | 1 | bslbf |
| if (hasInputLayout) { |  |  |
| inputLayoutAlignment; | 1 | bslbf |
| inputLayoutTL; | 1 | bslbf |
| inputLayoutT; | 1 | bslbf |
| inputLayoutTR; | 1 | bslbf |
| inputLayoutL; | 1 | bslbf |
| inputLayoutC; | 1 | bslbf |
| inputLayoutR; | 1 | bslbf |
| inputLayoutBL; | 1 | bslbf |
| inputLayoutB; | 1 | bslbf |
| inputLayoutBR; | 1 | bslbf |
| } |  |  |
| objectSourceId = GetID(); |  |  |
| [objectSourcePositionX; |  |  |
| objectSourcePositionY; |  |  |
| objectSourcePositionZ] = GetPosition(isSmallScene) |  |  |
| [objectSourceOrientationYaw; |  |  |
| objectSourceOrientationPitch; |  |  |
| objectSourceOrientationRoll] = GetOrientation(); |  |  |
| objectSourceCoordSpace; | 1 | bslbf |
| objectSourceActive; | 1 | bslbf |
| objectSourceGainDb = GetGain(isHiPrecGain=True); |  |  |
| objectSourceRefDistance = GetDistance(isSmallScene); |  |  |
| objectSourceSignalId = GetID(); |  |  |
| objectSourceHasReverbGain | 1 | bslbf |
| if (objectSourceHasReverbGain) { |  |  |
| objectSourceReverbGainDb = GetGain(isHiPrecGain=True); |  |  |
| } |  |  |
| objectSourceHasExtent; | 1 | bslbf |
| if (objectSourceHasExtent) { |  |  |
| objectSourceExtentId = GetID(); |  |  |
| } |  |  |
| objectSourceHasDirectivity; | 1 | bslbf |
| if (objectSourceHasDirectivity) { |  |  |
| objectSourceDirectivityId = GetID(); |  |  |
| } |  |  |
| objectSourceDirectiveness; | 8 | uimsbf |
| objectSourceNoReverb; | 1 | bslbf |
| objectSourceNoDoppler; | 1 | bslbf |
| objectSourceNoDistance; | 1 | bslbf |
| objectSourceNoCulling; | 1 | bslbf |
| objectSourceNoDirectionalFocus; | 1 | bslbf |
| objectSourceMode; | 1 | bslbf |
| objectSourcePlay; objectSourceVDLMethod; | 1 3 | Bslbf uimsbf |
| objectSourceHasSpatialTransform; | 1 | bslbf |
| if (objectSourcehasSpatialTransform) { |  |  |
| objectSourceHasAnchor; | 1 | bslbf |
| if (objectSourceHasAnchor) { |  |  |
| objectSourceParentAchorId = GetID(); |  |  |
| } |  |  |
| else { |  |  |
| objectSourceParentTransformId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| objectSourceIsStatic; | 1 | bslbf |
| objectSourceHasGranularPos; | 1 | bslbf |
| if (objectSourceHasGranularPos) { |  |  |
| granularPosX; | 16 | uimsbf |
| granularPosY; | 16 | uimsbf |
| granularPosZ | 16 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |

Table 83 — Syntax of hoaGroups()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| hoaGroups() |  |  |
| { |  |  |
| hoaGroupsCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < hoaGroupsCount; i++) { |  |  |
| hoaGroupId = GetID(); |  |  |
| hoaGroupHasRegion; | 1 | bslbf |
| if (hoaGroupHasRegion) { |  |  |
| hoaGroupRegionId = GetID(); |  |  |
| } |  |  |
| coSourceCount = GetCountOrIndex(); |  |  |
| for (int j = 0; j < coSourceCount; j++) { |  |  |
| coSourceId = GetID(); |  |  |
| } |  |  |
| FreqBandConfig(); |  |  |
| exteriorRenderingProjectionRadius = GetDistance(isSmallScene); |  |  |
| hoaGroupHasInformedSources; | 1 | bslbf |
| if (hoaGroupHasInformedSources == True) { |  |  |
| informedSourceCount = GetCountOrIndex(); |  |  |
| maxSimulInformedSources = GetCountOrIndex(); |  |  |
| for (j = 0; j < informedSourceCount; j++) { |  |  |
| InformedSourceInfoStruct() |  |  |
| } |  |  |
| adaptiveExteriorRenderingProjectionRadius; | 1 | bslbf |
| } |  |  |
| hoaGroupHasLowProfileConfig; | 1 | bslbf |
| if (hoaGroupHasLowProfileConfig == True) { |  |  |
| LowProfileConfig() |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 84 — Syntax of FreqBandConfig()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| FreqBandConfig() |  |  |
| { |  |  |
| hoaGroupHasFreqBandConfig; | 1 | bslbf |
| if (hoaGroupHasFreqBandConfig == True) { |  |  |
| highestSingleBinBandsIndex; | 8 | uimsbf |
| lowestHighBandIndex; | 8 | uimsbf |
| intermediateBandsERBWidth; | 8 | uimsbf |
| } |  |  |
| } |  |  |

Table 85 — Syntax of InformedSourceInfoStruct()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| InformedSourceInfoStruct() |  |  |
| { |  |  |
| informedSourceId = GetID(); |  |  |
| informedSourcePositionX; | 31 | uimsbf |
| informedSourcePositionY; | 31 | uimsbf |
| informedSourcePositionZ; | 31 | uimsbf |
| listenerThresholdPresent; | 1 | bslbf |
| hoaSourceThresholdPresent; | 1 | bslbf |
| if (listenerThresholdPresent == True) { |  |  |
| lpdInformedSourceEnableThreshold; | 16 | uimsbf |
| } |  |  |
| if (hoaSourceThresholdPresent == True) { |  |  |
| hpdInformedSourceEnableThreshold; | 16 | uimsbf |
| } |  |  |
| priorityValue; | 6 | uimsbf |
| } |  |  |
| } |  |  |

Table 86— Syntax of LowProfileConfig()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| LowProfileConfig() |  |  |
| { |  |  |
| distanceFactor; | 8 | uimsbf |
| panningGridOrder; | 3 | uimsbf |
| } |  |  |

Table 87 — Syntax of hoaSources()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| hoaSources() |  |  |
| { |  |  |
| hoaSourcesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < hoaSourcesCount; i++) { |  |  |
| hoaSourceId = GetID(); |  |  |
| [hoaSourcePositionX; |  |  |
| hoaSourcePositionY; |  |  |
| hoaSourcePositionZ] = GetPosition(isSmallScene); |  |  |
| [hoaSourceOrientationYaw; |  |  |
| hoaSourceOrientationPitch; |  |  |
| hoaSourceOrientationRoll] = GetOrientation(); |  |  |
| hoaSourceCoordSpace; | 1 | bslbf |
| hoaSourceActive; | 1 | bslbf |
| hoaSourceGainDb = GetGain(isHiPrecGain=True); |  |  |
| hoaSourceHasReverbGain; | 1 | bslbf |
| if (hoaSourceHasReverbGain) { |  |  |
| hoaSourceReverbGainDb = GetGain(isHiPrecGain=true); |  |  |
| } |  |  |
| hoaSourceRefDistance = GetDistance(isSmallScene); |  |  |
| hoaSourceSignalId = GetID(); |  |  |
| hoaSourceHasExtent; | 1 | bslbf |
| if (hoaSourceHasExtent) { |  |  |
| hoaSourceExtentId = GetID(); |  |  |
| } |  |  |
| hoaSourceExtentTransform; | 1 | bslbf |
| hoaSourceRepresentation; | 1 | bslbf |
| hoaSourceHasGroup; | 1 | bslbf |
| if (hoaSourceHasGroup) { |  |  |
| hoaSourceGroupId = GetID(); |  |  |
| } |  |  |
| hoaSourceNoReverb; | 1 | bslbf |
| hoaSourceNoDoppler; | 1 | bslbf |
| hoaSourceNoDistance; | 1 | bslbf |
| hoaSourceNoCulling; | 1 | bslbf |
| hoaSourceNoDirectionalFocus; | 1 | bslbf |
| hoaSourceMode; | 1 | bslbf |
| hoaSourcePlay; hoaSourceVDLMethod; | 1 3 | Bslbf uimsbf |
| hoaSourceHasSpatialTransform; | 1 | bslbf |
| if (hoaSourceHasSpatialTransform) { |  |  |
| hoaSourceHasAnchor; | 1 | bslbf |
| if (hoaSourceHasAnchor) { |  |  |
| hoaSourceParentAnchorId = GetID(); |  |  |
| } |  |  |
| else { |  |  |
| hoaSourceParentTransformId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| hoaSourceIsStatic; | 1 | bslbf |
| objectSourceHasGranularPos; | 1 | bslbf |
| if (objectSourceHasGranularPos) { |  |  |
| granularPosX; | 16 | uimsbf |
| granularPosY; | 16 | uimsbf |
| granularPosZ | 16 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |

Table 88 — Syntax of channelSources()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| channelSources() |  |  |
| { |  |  |
| channelSourcesCount = GetCountOrIndex(); |  |  |
| for (int i = 0; i < channelSourcesCount; i++) { |  |  |
| loudspeakerCount = GetCountOrIndex(); |  |  |
| for (int j = 0; j < loudspeakerCount; j++) { |  |  |
| loudspeakerId = GetID(); |  |  |
| [loudspeakerPositionX; |  |  |
| loudspeakerPositionY; |  |  |
| loudspeakerPositionZ] = GetPosition(isSmallScene); |  |  |
| [loudspeakerOrientationYaw; |  |  |
| loudspeakerOrientationPitch; |  |  |
| loudspeakerOrientationRoll] = GetOrientation(); |  |  |
| loudspeakerChannel = GetCountOrIndex(); |  |  |
| loudspeakerHasDirectivity; | 1 | bslbf |
| if (loudspeakerHasDirectivity) { |  |  |
| loudspeakerDirectivityId = GetID(); |  |  |
| } |  |  |
| loudspeakerDirectiveness; | 8 | uimsbf |
| } |  |  |
| channelSourceId = GetID(); |  |  |
| [channelSourcePositionX; |  |  |
| channelSourcePositionY; |  |  |
| channelSourcePositionZ] = GetPosition(isSmallScene); |  |  |
| [channelSourceOrientationYaw; |  |  |
| channelSourceOrientationPitch; |  |  |
| channelSourceOrientationRoll] = Getorientation(); |  |  |
| channelSourceCoordSpace; | 1 | bslbf |
| channelSourceInputLayout; | 7 | uimsbf |
| channelSourceActive; | 1 | bslbf |
| channelSourceGainDb = GetGain(isHiPrecGain=True); |  |  |
| channelSourceHasReverbGain; | 1 | bslbf |
| if (channelSourceHasReverbGain) { |  |  |
| channelSourceReverbGainDb = GetGain(isHiPrecGain=True); |  |  |
| } |  |  |
| channelSourceRefDistance = GetDistance(isSmallScene); |  |  |
| channelSourceSignalId = GetID(); |  |  |
| channelSourceNoReverb; | 1 | bslbf |
| channelSourceNoDoppler; | 1 | bslbf |
| channelSourceNoDistance; | 1 | bslbf |
| channelSourceNoCulling; | 1 | bslbf |
| channelSourceNoDirectionalFocus; | 1 | bslbf |
| channelSourceMode; | 1 | bslbf |
| channelSourcePlay; channelSourceVDLMethod; | 1 3 | Bslbf uimsbf |
| channelSourceHasSpatialTransform; | 1 | bslbf |
| if (channelSourceHasSpatialTransform) { |  |  |
| channelSourceHasAnchor; | 1 | bslbf |
| if (channelSourceHasAnchor) { |  |  |
| channelSourceParentAnchorId = GetID(); |  |  |
| } |  |  |
| else { |  |  |
| channelSourceParentTransformId = GetID(); |  |  |
| } |  |  |
| } |  |  |
| channelSourceIsStatic; | 1 | bslbf |
| objectSourceHasGranularPos; | 1 | bslbf |
| if (objectSourceHasGranularPos) { |  |  |
| granularPosX; | 16 | uimsbf |
| granularPosY; | 16 | uimsbf |
| granularPosZ | 16 | uimsbf |
| } |  |  |
| } |  |  |

Table 89 - Syntax of airflowGeneratorObjects()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| airflowGeneratorObjects () |  |  |
| { |  |  |
| numberOfGeneratorObjects = GetCountOrIndex() |  |  |
| for (n = 0:numberOfGeneratorObjects-1) |  |  |
| { |  |  |
| id = GetID(); |  |  |
| generatorType; | 3 | uimsbf |
| speed; | 10 | uimsbf |
| orientation = GetOrientation(); |  |  |
| gainDb = GetGain(); |  |  |
| active; | 1 | bslbf |
| if (generatorType == ‘Point Source’) |  |  |
| { |  |  |
| position = GetPosition(isSmallScene); |  |  |
| hasLimits; | 1 | bslbf |
| if (hasLimits) |  |  |
| { |  |  |
| aziRange; | 8 | uimsbf |
| elevRange; | 8 | uimsbf |
| } |  |  |
| } else if (generatorType == ‘Cone’) |  |  |
| { |  |  |
| position = GetPosition(isSmallScene); |  |  |
| baseRadius = GetDistance(isSmallScene); |  |  |
| length = GetDistance(isSmallScene); |  |  |
| hasTopRadius; | 1 | bslbf |
| if (hasTopRadius) |  |  |
| { |  |  |
| topRadius = GetDistance(isSmallScene); |  |  |
| } |  |  |
| edgeRolloff; | 10 | uimsbf |
| } else if (generatorType == ‘Plane’ |  |  |
| position = GetPosition(isSmallScene); |  |  |
| width = GetDistance(isSmallScene); |  |  |
| height = GetDistance(isSmallScene); |  |  |
| edgeRolloff; | 10 | uimsbf |
| } |  |  |
| hasDistanceAttenuationFactor; | 1 | bslbf |
| if (hasDistanceAttenuationFactor) |  |  |
| { |  |  |
| distanceAttenuationFactor; | 8 | uimsbf |
| } |  |  |
| } |  |  |
| } |  |  |

### Airflow payload syntax

Table 90 - Syntax of payloadAirflow()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadAirflow () |  |  |
| { |  |  |
| numberOfAirFlowSources = GetCountOrIndex(): |  |  |
| for (i=0:numberOfAirFlowSources - 1) |  |  |
| { |  |  |
| hasCustomProfile; | 1 | bslbf |
| if (hasCustomProfile) |  |  |
| { |  |  |
| profileType | 2 | uimsbf |
| if (profileType == ‘freq-gain pairs’) |  |  |
| { |  |  |
| // per velocity frequency-gain pair lists |  |  |
| numSpeedProfiles = GetCountOrIndex(); |  |  |
| numAziProfiles = GetCountOrIndex(); |  |  |
| numElevProfiles = GetCountOrIndex(); |  |  |
| for (k = 0:numSpeedProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refSpeed[k]; | 10 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| speedProfile[k][b] = getGain(true); |  |  |
| } |  |  |
| } |  |  |
| for (k = 0:numAziProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refAzimuth[k]; | 9 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| azimuthProfile[k][b][0] = getGain(true); |  |  |
| azimuthProfile[k][b][1] = getGain(true); |  |  |
| } |  |  |
| } |  |  |
| for (k = 0:numElevProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refElevation[k]; | 9 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| elevationProfile[k][b] = getGain(true); |  |  |
| } |  |  |
| } |  |  |
| } else if (profileType == ‘filterParameters’) |  |  |
| { |  |  |
| // per velocity list of centre freq , gain, Q. |  |  |
| numSpeedProfiles = GetCountOrIndex(); |  |  |
| numAziProfiles = GetCountOrIndex(); |  |  |
| numElevProfiles = GetCountOrIndex(); |  |  |
| for (k = 0:numSpeedProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refSpeed[k]; | 10 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| speedFilterGain[k][b] = getGain(true); |  |  |
| speedFilterQ[k][b]; | 7 | uimsfb |
| } |  |  |
| } |  |  |
| for (k = 0:numAziProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refAzimuth[k]; | 9 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| azimuthFilterGain[k][b][0] = getGain(true); |  |  |
| azimuthFilterGain[k][b][1] = getGain(true); |  |  |
| azimuthFilterQ[k][b][0]; | 7 | uimsfb |
| azimuthFilterQ[k][b][1]; | 7 | uimsfb |
| } |  |  |
| } |  |  |
| for (k = 0:numElevProfiles-1) |  |  |
| { |  |  |
| freqGridIdx[i] = GetCountOrIndex(); |  |  |
| refElevation[k]; | 8 | uimsbf |
| for (b = 0:fgdNrBands[freqGridIdx[i]] – 1) |  |  |
| { |  |  |
| elevationFilterGain[k][b] = getGain(true); |  |  |
| elevationFilterQ[k][b]; | 7 | uimsfb |
| } |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| profile; //specify which LUT to use. | 2 | uimsbf |
| } |  |  |
| generatorID[i] = GetID(); |  |  |
| activeInDefaultRegion; | 1 | bslbf |
| hasValidRegions; | 1 | bslbf |
| if (hasValidRegions) |  |  |
| { |  |  |
| numRegions = GetCountOrIndex(); |  |  |
| for (e =0:numRegions-1) |  |  |
| { |  |  |
| activeRegionID[i][e] = GetID(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Granular payload syntax

- Table 91 - Syntax of payloadGranular()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadGranular() |  |  |
| { |  |  |
| numberOfGranularDatabases=GetCountOrIndex(); |  |  |
| for (int i=0; i<numberOfGranularDatabases; i++) { |  |  |
| granularID = GetID(); |  |  |
| GranularAudioEncodingMode=GetCountOrIndex(); |  |  |
| DescriptorCoordinateDimensions=GetCountOrIndex(); |  |  |
| numberOfChannels=GetCountOrIndex(); |  |  |
| numberOfGrains; | 20 | uimsbf |
| for (int i=0; i<DescriptorCoordinateDimensions; i++) { |  |  |
| coordScaling[i]; | 10 | uimsbf |
| smoothingTargetCoordinates[i] | 10 | uimsbf |
| } |  |  |
| searchRadius; | 10 | uimsbf |
| grainHistoryLength=GetCountOrIndex(); |  |  |
| numMixRule=GetCountOrIndex(); |  |  |
| for (int i=0; i<numMixRule; i++) { |  |  |
| grainMixRule[i] | 8 | uimsbf |
| for (int j=0; j< DescriptorCoordinateDimensions; j++) { |  |  |
| grainMixingRuleCoordinates[i][j] | 16 | uimsbf |
| } |  |  |
| } |  |  |
| fastGrainSwitchThresholdAvailable; | 1 | bslbf |
| if (fastGrainSwitchThresholdAvailable) { |  |  |
| fastGrainSwitchThreshold; | 10 | uimsbf |
| } |  |  |
| probabilityExponentPosition; | 8 | uimsbf |
| probabilityWeightPosition; | 16 | uimsbf |
| probabilityExponentHistory; | 8 | uimsbf |
| probabilityWeightHistory; | 16 | uimsbf |
| descrTrendAvailable; | 1 | bslbf |
| if (descrTrendAvailable) { |  |  |
| probabilityExponentTrend; | 8 | uimsbf |
| probabilityWeightTrend; | 16 | uimsbf |
| descrTrendMethod=GetCountOrIndex(); |  |  |
| } |  |  |
| originalRecordingPositionAvailable; | 1 | bslbf |
| if (originalRecordingPositionAvailable) { |  |  |
| probabilityExponentOriginalRecordingPosition; | 8 | uimsbf |
| probabilityWeightOriginalRecordingPosition; | 16 | uimsbf |
| } |  |  |
| probabilityGroupsAvailable; | 1 | bslbf |
| perGrainProbabilityAvailable; | 1 | bslbf |
| if (probabilityGroupsAvailable) { |  |  |
| numProbGroups=GetCountOrIndex(); |  |  |
| For (int i=0; i<numProbGroups;i++) { |  |  |
| probabilityGroupVal[i]; | 16 | uimsbf |
| } |  |  |
| } |  |  |
| randomSelection; | 1 | bslbf |
| useInterpolation; | 1 | bslbf |
| if (useInterpolation) { |  |  |
| grainInterpolationDistThreshold; | 10 | uimsbf |
| useInterpolationResampling; | 1 | bslbf |
| InterpolationMethod=GetCountOrIndex(); |  |  |
| for (int i=0; i< DescriptorCoordinateDimensions;i++) { |  |  |
| pitchDimension[i]; | 1 | bslbf |
| } |  |  |
| } |  |  |
| grainOverlapAmountAvailable; | 1 | uimsbf |
| if (grainOverlapAmountAvailable) { |  |  |
| grainOverlapAmount; | 10 | uimsbf |
| } |  |  |
| for (int j=0; j<numberOfGrains; j++) { |  |  |
| ReadGrain(); |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

- Table 92 - Syntax of ReadGrain()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ReadGrain() |  |  |
| { |  |  |
| for (int i = 0; i < DescriptorCoordinateDimensions; i++) { |  |  |
| DescrCoord [i]; | 16 | uimsbf |
| } |  |  |
| GrainLen; | 18 | uimsbf |
| if (descrTrendAvailable) { |  |  |
| for (int i = 0; i < DescriptorCoordinateDimensions; i++) { |  |  |
| DescrCoordTrend[i]; | 16 | uimsbf |
| } |  |  |
| } |  |  |
| if (originalRecordingPositionAvailable) { |  |  |
| origRecordingPos; | 16 | uimsbf |
| } |  |  |
| if (probabilityGroupsAvailable) { |  |  |
| grainProbabilityGroup=GetCountOrIndex(); |  |  |
| } |  |  |
| if (perGrainProbabilityAvailable) { |  |  |
| perGrainProbabilityValue; | 16 | uimsbf |
| } |  |  |
| switch (GranularAudioEncodingMode) { |  |  |
| case 0: { //No compression |  |  |
| for (int j=0; j<numberOfChannels; j++) { |  |  |
| for (int i=0; i<GrainLen; i++) { |  |  |
| GrainSamples[i][j]; | 16 | uimsbf |
| } |  |  |
| } |  |  |
| break; |  |  |
| } |  |  |
| case 1: { //PCM data from separately provided PCM signal |  |  |
| grainSampleIndex; | 32 | uimsbf |
| break; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### RasterMap payload syntax

- Table 93 - Syntax of payloadRasterMap()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| payloadRasterMap() |  |  |
| { |  |  |
| numberOfVoxSubScenes = GetCountOrIndex(); for (int i = 0; i < numberOfVoxSubScenes; i ++) { rasterMapSubScene(); } |  |  |
| } |  |  |

- Table 94 - Syntax of rasterMapSubScene()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| rasterMapSubScene() |  |  |
| { |  |  |
| hasVoxSceneDiffractionRasterMap; if (hasVoxSceneDiffractionRasterMap) { numRasterMapSources = GetCountOrIndex(); for (int s = 0; s < numRasterMapSources; s++) { rasterMapSourceIdLen[s] = GetCountOrIndex(); for (int i = 0; i < numRasterMapSources; i++) { rasterMapSourceId[s][i] = GetID(); } numRasterMapLayers[s]; for (int l = 0; l < numRasterMapLayers[s]; l++) { rasterMap[s][l] = rasterMapLayer(l); } } } | 1 3 | bslbf Uimsbf |
| } |  |  |

- Table 95 - Syntax of rasterMapLayer()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| rasterMapLayer(l) |  |  |
| { |  |  |
| vcX = 0; vcY = -1; codebookVcX = genericCodebook(); codebookVcY = genericCodebook(); codebookWpX = genericCodebook(); codebookWpY = genericCodebook(); numRasterMapVoxels = GetID(); for (int v = 0; v < numRasterMapVoxels; v++) { vcY += 1; hasVoxelCoordY; if (hasVoxelCoordY) { vcY = codebookVcY.get_symbol(); vcX += 1; hasVoxelCoordX; if (hasVoxelCoordX) { vcX = codebookVcX.get_symbol(); } } | 1 1 | bslbf vlclbf bslbf vlclbf |
| this.voxelCoordX[v] = vcX; this.voxelCoordY[v] = vcY; this.refMode[v]; if (this.refMode[v] == 0) { mapChange; this.waypointX[v] = codebookWpX.get_symbol(); this.waypointY[v] = codebookWpY.get_symbol(); this.waypointL[v] = l - mapChange; } } return this; | 2 1 | uimsbf uimsbf vlclbf vlclbf |
| } |  |  |

### Support Elements

Support elements are functions that are called to read and decode the raw bitstream into internal renderer parameters.

- Table 96 - Syntax of GetID()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| id = GetID() |  |  |
| { |  |  |
| id = idVal; | 7 | uimsbf |
| while (largerValue) | 1 | bslbf |
| { |  |  |
| idVal; | 7 | uimsbf |
| id = id * 128 + idVal; |  |  |
| } |  |  |
| return id; |  |  |
| } |  |  |

idVal	ID value or partial ID value.

largerValue	Flag that indicates whether the ID value is larger.

- Table 97 - Syntax of GetCountOrIndex()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| number = GetCountOrIndex() |  |  |
| { |  |  |
| number = LUT(countOrIndexLoCode); |  | vlclbf |
| if (isLargerNumber) | 1 | bslbf |
| { |  |  |
| numberHi = LUT(countOrIndexHiCode); |  | vlclbf |
| number = number + numberHi * 64; |  |  |
| } |  |  |
| return number; |  |  |
| } |  |  |

countOrIndexLoCode	Code indicating the lower bits of a count or index value.

LUT()	Executes query on look-up table corresponding to the field whose name is provided as argument. The look-up tables used by the support elements are listed in A.14.

isLargerNumber	Flag indicating whether more bits are sent to indicate a larger number.

countOrIndexHiCode	Code indicating the higher bits of a count or index value, see table in Annex A.14.

- Table 98 - Syntax of GetDuration()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| duration = GetDuration( isLongRangeMode) |  |  |
| { |  |  |
| deciSeconds = LUT(deciSecondsCode); | var | vlclbf |
| duration = deciSeconds; |  |  |
| if (addMilliseconds) | 1 | bslbf |
| { |  |  |
| miliSeconds = LUT(milliSecondsCode); | var | vlclbf |
| duration = duration + miliSeconds; |  |  |
| if (addMicroseconds) | 1 | bslbf |
| { |  |  |
| microseconds = LUT(microsecondsCode); | var | vlclbf |
| duration = duration + microseconds; |  |  |
| } |  |  |
| } |  |  |
| if (addSeconds) | 1 | bslbf |
| { |  |  |
| if (isLongRangeMode) |  |  |
| { |  |  |
| duration = duration + rawSeconds; | 12 | uimsbf |
| if (addHours) | 1 | bslbf |
| { |  |  |
| duration = duration + (rawHours + 1) * 3600; | 5 | uimsbf |
| } |  |  |
| } |  |  |
| else |  |  |
| { |  |  |
| seconds = LUT(secondsCode); | var | vlclbf |
| duration = duration + seconds; |  |  |
| } |  |  |
| } |  |  |
| return duration; |  |  |
| } |  |  |

deciSecondsCode	Code for indicating decimal seconds duration offset.

addMilliseconds	Flag indicating whether milliseconds duration offset is transmitted next.

milliSecondsCode	Code for indicating milliseconds duration offset.

addMicroseconds	Flag indicating whether microseconds duration offset is transmitted next.

microsecondsCode	Code for indicating number of microseconds duration offset.

addSeconds	Flag indicating whether seconds duration offset is transmitted next.

secondsCode	Code for indicating seconds duration offset.

rawSeconds	Value indicating seconds duration offset in case of an initialization data point. Valid values in range 0..3600.

rawHours	Value indicating seconds duration offset in case of an initialization data point.

- Table 99 - Syntax of Getfrequency()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| frequency = GetFrequency() |  |  |
| { |  |  |
| frequency = LUT(frequencyCode); | var | vlclbf |
| if (moreAccuracy) | 1 | bslbf |
| { |  |  |
| frequency = frequency * 2^((frequencyRefine + 1) / 51); | 4 | uimsbf |
| } |  |  |
| return frequency; |  |  |
| } |  |  |

frequencyCode	Code that indicates a center frequency in Hz of a one-third octave band.

moreAccuracy	Flag that indicates whether data for a more accurate frequency is transmitted.

frequencyRefine	Field that indicates a value for refining the frequency value.

- Table 100 - Syntax of GetPosition()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| position = GetPosition(isSmallScene) |  |  |
| { |  |  |
| for c = 0:2 |  |  |
| { |  |  |
| coord = GetDistance(isSmallScene); |  |  |
| isNegative; | 1 | bslbf |
| if (isNegative) |  |  |
| { |  |  |
| position[c] = -1 * coord; |  |  |
| } else |  |  |
| { |  |  |
| position[c] = coord; |  |  |
| } |  |  |
| } |  |  |
| return position; |  |  |
| } |  |  |

position	Contains a position in [x, y, z] coordinates.

isNegative	Flag that indicates whether the coordinate magnitude in coord is in the negative direction (1) or positive direction (0) of the corresponding dimension.

- Table 101 - Syntax of GetDistance()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| distance = GetDistance(isSmallScene) |  |  |
| { |  |  |
| meters = LUT(metersCode); | var | vlclbf |
| distance = meters; |  |  |
| if (isSmallScene == false) |  |  |
| { |  |  |
| if (addHectometers) | 1 | bslbf |
| { |  |  |
| hectometers = LUT(hectometersCode); | var | vlclbf |
| distance = distance + 100 * hectometers; |  |  |
| while (addKilometers) | 1 | bslbf |
| { |  |  |
| kilometers = LUT(kilometersCode); | var | vlclbf |
| distance = distance + kilometers * 1000; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if (addCentimeters) | 1 | bslbf |
| { |  |  |
| centimeters = LUT(centimetersCode); | var | vlclbf |
| distance = distance + centimeters / 100; |  |  |
| if (addMillimeters) | 1 | bslbf |
| { |  |  |
| millimeters = LUT(millimetersCode); | var | vlclbf |
| distance = distance + millimeters / 1000; |  |  |
| } |  |  |
| } |  |  |
| return distance; |  |  |
| } |  |  |

metersCode		Code for indicating meters coordinate offset.

addHectometers	Flag indicating whether hectometers coordinate offset is transmitted next.

hectometersCode	Code for indicating hectometers coordinate offset.

addKilometers		Flag indicating whether kilometers coordinate offset is transmitted next.

kilometersCode	Code for indicating kilometers coordinate offset. Multiple occurrences can be provided for distances beyond 10 km.

addCentimeters	Flag indicating whether centimeters coordinate offset is transmitted next.

centimetersCode	Code for indicating centimeters coordinate offset.

addMillimeters		Flag indicating whether millimeters coordinate offset is transmitted next.

millimetersCode	Code for indicating millimeters coordinate offset.

- Table 102 - Syntax of GetOrientation()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| orientation = GetOrientation() |  |  |
| { |  |  |
| orientation = LUT(orientationCode); | var | vlclbf |
| if (orientationCode == 'Default angles') { |  |  |
| orientation[0] = LUT(defaultYawCode); | var | vlclbf |
| orientation[1] = LUT(defaultPitchCode); | var | vlclbf |
| orientation[2] = LUT(defaultRollCode); | var | vlclbf |
| } else if (orientationCode == 'Explicit angles') { |  |  |
| for (o = 0:2) { |  |  |
| coarseAngle = LUT(coarseAngleCode); | var | vlclbf |
| angle = coarseAngle; |  |  |
| if (addFineAngle) | 1 | bslbf |
| { |  |  |
| fineAngle = LUT(fineAngleCode); | var | vlclbf |
| angle = angle + fineAngle; |  |  |
| } |  |  |
| orientation[o] = angle; |  |  |
| } |  |  |
| } else if (orientationCode == ‘High resolution angles’) { |  |  |
| for (o = 0:2) { |  |  |
| orientation[o] = angleVal; | 32 | float |
| } |  |  |
| } |  |  |
| return orientation; |  |  |
| } |  |  |

orientationCode	Code for orientation, indicating either a default orientation or one of two escape values for which further data defines the orientation.

defaultYawCode	Code for a default yaw angle.

defaultPitchCode	Code for a default pitch angle.

defaultRollCode	Code for a default roll angle.

coarseAngleCode	Code for a coarse angle indication in steps of 5 degrees.

addFineAngle	Flag indicating whether finer granularity angle data is transmitted.

fineAngleCode	Code for a fine angle indication in steps of 0.1 degrees.

- Table 103 - Syntax of GetGain()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| gain = GetGain(isHiPrecGain) |  |  |
| { |  |  |
| gain = LUT(coarseGainCode); | var | vlclbf |
| if (addFineGain) | 1 | bslbf |
| { |  |  |
| fineGain = LUT(fineGainCode); | var | vlclbf |
| gain = gain + fineGain; |  |  |
| } |  |  |
| if (isHiPrecGain) |  |  |
| { |  |  |
| if (addDecimalGain) | 1 | bslbf |
| { |  |  |
| decimalGain = LUT(deciGainCode); | var | vlclbf |
| gain = gain + decimalGain; |  |  |
| } |  |  |
| } |  |  |
| return gain; |  |  |
| } |  |  |

isHiPrecGain	Input argument indicating whether the gain is a high precision gain with 0.1 dB accuracy.

coarseGainCode	Code for a coarse gain value (in dB).

addFineGain	Flag indicating whether further data is transmitted to provide a finer resolution gain value.

fineGainCode	Code for a finer gain resolution (1 dB resolution).

addDecimalGain	Flag indicating whether further data is transmitted to provide a decimal precision gain code.

decimalCode	Code for a decimal value.

- Table 104 - Syntax of GetAudioSourceRescale()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| audioSourceRescale = GetAudioSourceRescale() |  |  |
| { |  |  |
| for d = 0:2 |  |  |
| { |  |  |
| audioSourceRescale[d] = LUT(decimalCode); | var | vlclbf |
| } |  |  |
| return audioSourceRescale; |  |  |
| } |  |  |

audioSourceRescale	Rescale factors used to modify the position of audio elements.

- Table 105 - Syntax of GetTolerance()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| tolerance = GetTolerance() |  |  |
| { |  |  |
| for d = 0:5 |  |  |
| { |  |  |
| tolerance[d] = LUT(decimalCode); | var | vlclbf |
| } |  |  |
| return tolerance; |  |  |
| } |  |  |

decimalCode	Percentage of scene region that remains outside of LSDF region,(max x-dimension, min x-dimension, max y-dimension, min y-dimension, max z- dimension, min z-dimension).

## Data structure

### General

In environments that require byte alignment, MPEG-I Immersive Audio configuration elements or payload elements that are not an integer number of bytes in length are padded at the end to achieve an integer byte count. This is indicated by the function ByteAlign()	.

noOfBits denotes the No. of bits used for transmission of a given parameter as noted in the respective bitstream syntax table containing said parameter.

### Renderer payloads data structure

#### Geometry

##### Mesh Geometry Data

Scene geometry can be described by meshes. Mesh data is arithmetically coded for efficient transmission in the bitstream. Only the vertices, faces, and associated material IDs per face are coded.

Acoustic materials are used for geometries to determine their acoustic properties. These materials are transmitted in the bitstream using efficient arithmetic coding. An acoustic material lists, per frequency: specular reflection coefficient, diffuse scattering coefficient, transmission coefficient, and a coupling coefficient.

The detailed decoding procedure is described in 6.5.

##### Static geometry for Early Reflection and Diffraction Stage

The static mesh data for the Early Reflection and the Diffraction stage is derived via geometry data conversion.

The earlySurfaceFaceIdx[s][f] bitstream elements of the earlySurfaceData() payload component specify the triangle index of the static early reflection geometry which is derived as follows:

The decoder iterates over all meshes and geometric primitives in bitstream order and if the isEarlyReflectionMesh flag or the isEarlyReflectionPrimitive flag is set, then a mesh representation of the geometric object is added to a list of geometries. The static early reflection mesh is then generated by concatenating the mesh data (i.e. vertices and triangles) as follows (with meshHandle being a pointer to the instance of the static early reflection mesh):

for (const auto& geo : geometries) {

const auto& mesh_new = geo->getMesh();

const auto& vertices = meshHandle->getVertices();

size_t v0 = vertices.size();

// add vertices

const auto& vertices_new = mesh_new->getVertices();

for (const auto& v : vertices_new) {

meshHandle->addVertex(v);

}

// add triangles

const auto& triangles_new = mesh_new->getTriangles();

for (const auto& t : triangles_new) {

meshHandle->addTriangle(v0 + t.v1, v0 + t.v2, v0 + t.v3, t.material);

}

}

The static diffraction mesh is derived in a very similar way with the difference being that cylinder primitives are added to the geometries list, if the isStatic flag is set and if the cylinder has material data. Consequently, the static early reflection mesh and the static diffraction mesh only differ by the mesh data for cylinders which are handled separately by the Early Reflection stage (see Section 6.6.6.3.6).

The mesh representation is retrieved in the source code above via geo->getMesh().

For a sphere, an icosahedron with the following vertices and faces is used as basis:

a = 0.2929428;

b = 0.47399141;

sphere_vertices = {

{-a,  b,  0},

{ a,  b,  0},

{-a, -b,  0},

{ a, -b,  0},

{ 0, -a,  b},

{ 0,  a,  b},

{ 0, -a, -b},

{ 0,  a, -b},

{ b,  0, -a},

{ b,  0,  a},

{-b,  0, -a},

{-b,  0,  a}

};

sphere_faces = {

{0, 11, 5},

{0, 5, 1},

{0, 1, 7},

{0, 7, 10},

{0, 10, 11},

{1, 5, 9},

{5, 11, 4},

{11, 10, 2},

{10, 7, 6},

{7, 1, 8},

{3, 9, 4},

{3, 4, 2},

{3, 2, 6},

{3, 6, 8},

{3, 8, 9},

{4, 9, 5},

{2, 4, 11},

{6, 2, 10},

{8, 6, 7},

{9, 8, 1}

};

For a cylinder, the following mesh is used as basis:

a = 0.5086662;

b = 0.4405178;

c = 0.2543331;

cylinder_vertices = {

{ a,  0.5,  0},

{ b,  0.5,  c},

{ c,  0.5,  b},

{ 0,  0.5,  a},

{-c,  0.5,  b},

{-b,  0.5,  c},

{-a,  0.5,  0},

{-b,  0.5, -c},

{-c,  0.5, -b},

{ 0,  0.5, -a},

{ c,  0.5, -b},

{ b,  0.5, -c},

{ a, -0.5,  0},

{ b, -0.5,  c},

{ c, -0.5,  b},

{ 0, -0.5,  a},

{-c, -0.5,  b},

{-b, -0.5,  c},

{-a, -0.5,  0},

{-b, -0.5, -c},

{-c, -0.5, -b},

{ 0, -0.5, -a},

{ c, -0.5, -b},

{ b, -0.5, -c},

{ 0,  0.5,  0},

{ 0, -0.5,  0}

};

cylinder_faces = {

{11, 0, 12},

{11, 12, 23},

{0, 1, 13},

{0, 13, 12},

{1, 2, 14},

{1, 14, 13},

{2, 3, 15},

{2, 15, 14},

{3, 4, 16},

{3, 16, 15},

{4, 5, 17},

{4, 17, 16},

{5, 6, 18},

{5, 18, 17},

{6, 7, 19},

{6, 19, 18},

{7, 8, 20},

{7, 20, 19},

{8, 9, 21},

{8, 21, 20},

{9, 10, 22},

{9, 22, 21},

{10, 11, 23},

{10, 23, 22},

{24, 1, 0},

{24, 2, 1},

{24, 3, 2},

{24, 4, 3},

{24, 5, 4},

{24, 6, 5},

{24, 7, 6},

{24, 8, 7},

{24, 9, 8},

{24, 10, 9},

{24, 11, 10},

{24, 0, 11},

{25, 12, 13},

{25, 13, 14},

{25, 14, 15},

{25, 15, 16},

{25, 16, 17},

{25, 17, 18},

{25, 18, 19},

{25, 19, 20},

{25, 20, 21},

{25, 21, 22},

{25, 22, 23},

{25, 23, 12}

}

For a box, the following mesh is used as basis:

a = 0.5;

b = 0.5;

c = 0.5;

box_vertices = {

{-a,  b,  c},

{ a,  b,  c},

{-a, -b,  c},

{ a, -b,  c},

{-a,  b, -c},

{ a,  b, -c},

{-a, -b, -c},

{ a, -b, -c}

};

box_faces = {

{1, 0, 2},

{1, 2, 3},

{4, 5, 7},

{4, 7, 6},

{5, 4, 0},

{5, 0, 1},

{3, 2, 6},

{3, 6, 7},

{0, 4, 6},

{0, 6, 2},

{5, 1, 3},

{5, 3, 7}

}

These base vertices are then transformed according to the position, orientation, and size values of the primitives() payload component.

##### Voxel Geometry Data

Voxel-based scene representation is loss-less coded for its efficient storage and transmission in the bitstream. Only the integer grid indices of two bounding voxels are considered per one voxel block element associated with one material ID. All voxels (of the voxel block i) with grid indices between these bounding voxels (corresponding to the variables start and end) share the same material ID (defined by the variable voxSceneElementMatID), where

start = voxSceneElementPosS[i]

end = voxSceneElementPosE[i]

All sequentially specified overlapping voxel block elements override the corresponding voxel material IDs inside of the overlapping region. The sequential specification of voxel block elements allows to describe level of details progressively - from the most significant to significant less important scene geometry elements.

The uncompressed voxel scene data is represented by the 3D matrix variable VoxDataMatrix is defined by the payload element voxSceneElements() using the following pseudo code:

for (int i = 0; i < numberOfVoxSceneElements; i++) {

PackedPosS[i] = voxSceneElementPosPackedS[i]

PackedPosE[i] = voxSceneElementPosPackedE[i]

for (int j = 0; j < 3; j++) {

(voxSceneElementPosS[i][j], PackedPosS[i]) = FunctionUnpackPos(PackedPosS[i],

voxSceneDimensions[j])

(voxSceneElementPosE[i][j], PackedPosE[i]) = FunctionUnpackPos(PackedPosE[i],
 			voxSceneDimensions[j])

start[j] = voxSceneElementPosS[i][j]

end[j] = voxSceneElementPosE[i][j]

}

for (int x = start[0]; x <= end[0]; x++) {

for (int y = start[1]; y <= end[1]; y++) {

for (int z = start[2]; z <= end[2]; z++) {

VoxDataMatrix[x][y][z] = voxSceneElementMatID[i]

}

}

}

}

where

(UnpackedPos, PackedData) = FunctionUnpackPos(PackedData, Dimension)

{

UnpackedPos = mod(PackedData, Dimension) + 1

PackedData = floor(PackedData / Dimension)

}

The same approach is used for uncompressing of the 2D marix variable VoxDataDiffractionMap used for the diffraction modeling processing stage. If the 2D marix variable VoxDataDiffractionMap is specified by the encoder (i.e., voxSceneDiffractionMapPresentFlag == 1), it is defined by the bitstream payload element voxSceneDiffractionMap(); otherwise (i.e., voxSceneDiffractionMapPresentFlag == 0), it is defined by the bitstream payload element voxSceneElements() using the following pseudo code:

if (voxSceneDiffractionMapPresentFlag == 1) {

for (int i = 0; i < numberOfVoxDiffractionMapElements; i++) {

PackedPosS[i] = voxDiffractionMapPosPackedS[i]

PackedPosE[i] = voxDiffractionMapPosPackedE[i]

for (int j = 0; j < 2; j++) {

(voxDiffractionMapPosS[i][j], PackedPosS[i]) = FunctionUnpackPos(PackedPosS[i],
 				voxSceneDimensions[j])

(voxDiffractionMapPosE[i][j], PackedPosE[i]) = FunctionUnpackPos(PackedPosE[i],
 				voxSceneDimensions[j])

start[j] = voxDiffractionMapPosS[i][j]

end[j] = voxDiffractionMapPosE[i][j]

}

for (int x = start[0]; x <= end[0]; x++) {

for (int y = start[1]; y <= end[1]; y++) {

VoxDataDiffractionMap[x][y] = voxDiffractionMapValue[i]

}

}

}

} else {

if (voxSceneDiffractionMapHeightPresentFlag) {

H = voxSceneDiffractionMapHeight

} else {

H = round(0.5*voxSceneDimensions[2])

}

for (int x = 0; x < voxSceneDimensions[0]; x++) {

for (int y = 0; y < voxSceneDimensions[1]; y++) {

VoxDataDiffractionMap[x][y] = VoxDataMatrix[x][y][H]

}

}

}

Mapping of voxel positions to global scene coordinates is defined by two reference points voxScenePosS and voxScenePosE specifying two voxel extreme scene corner positions and the number of voxels voxSceneDimensions per each scene dimension. Each of two reference points has three floating point values that represent a point in Cartesian space in meters.

Table 106 — Mapping of two bounding voxel positions to the global scene coordinates

| Coordinates of scene corners | Corresponding voxels and grid indeces |
| --- | --- |
| voxScenePosS | VoxDataMatrix[0][0][0] |
| voxScenePosE | VoxDataMatrix[voxSceneDimensions[0]-1] [voxSceneDimensions[1]-1][voxSceneDimensions[3]-1] |

Once VoxDataMatrix and VoxDataDiffractionMap are created, the uncompressed voxel-based scene representation data is used in the renderer pipeline. The method for conversion between uncompressed and compressed data representation formats is specified in 6.3.2.1.4.

The same approach is used for uncompressing the voxel scene diffraction pre-computed path data represented by the set of variables voxDiffractionPathStartVoxel, voxDiffractionPathEndVoxel and voxDiffractionSourceDirection, it is defined by the bitstream payload element voxSceneDiffractionPreComputedPathData() using the following pseudo code:

// Encoding mode for voxel scene diffraction pre-computed path data

if (pcpdFull) {

// Full encoding mode for pre-computed diffraction data

for (int i = 0; i < numStartPositions; i++) {

if (pcpdPresentForStartPosition[i]) {

startPosition = pcpdStartVoxelPacked[i].unpack()

// Previous path information

prev = {}

// Traverse the diffraction map

for(int x = 1; x < voxSceneDimensions[0]; ++x) {

for(int y = 1; y < voxSceneDimensions[1]; ++y) {

endPosition = [ x, y ]

if (!pcpdPathExistFlag[i][x][y]) {

continue;

}

if (!pcpdUsePreviousPath[i][x][y]) {

// Complete data-point

sourceDirection[startPosition, endPosition] = pcpdSourceDirectionPacked[i][x][y].unpack()

pathLength[startPosition, endPosition] = pcpdPathLength[i][x][y]

// Update memory

prev.sourceDirection = sourceDirection[startPosition, endPosition]

prev.pathLength = pathLength[startPosition, endPosition]

} else {

sourceDirection[startPosition, endPosition] = prev.sourceDirection

// Delta-coding of path-length

delta_table = {1, sqrt(2)-1, 1-sqrt(2), -1}

pathLength[startPosition, endPosition] = prev.pathLength + delta_table[delta[i][x][y]]

prev.PathLength = pathLength[startPosition, endPosition]

}

}

}

}

} else {

// Selective encoding mode for pre-computed diffraction data

for (i = 0; i < pcpdNumStartPositions; ++i) {

// Previous path information

prev = {}

startPosition = pcpdStartVoxelPacked[i].unpack()

numEndPositions = pcpdNumEndPositions[i]

for(j = 0; j < numEndPositions; ++j) {

endPosition = pcpdEndVoxelPacked[i][j].unpack()

if (pcpdPathExistFlag[i][j]) {

if (!pcpdUsePreviousSourceDirection[i][j]) {

sourceDirection[startPosition][endPosition] =

pcpdSourceDirectionPacked[i][j].unpack()

prev.sourceDirection = sourceDirection[startPosition][endPosition]

} else {

sourceDirection[startPosition][endPosition] = prev.sourceDirection

}

pathLength[startPosition][endPosition] = pcpdPathLength[i][j]

}

}

}

}

// Helper function to unpack packed voxel coordinates

unpack(packedCoordinate, voxSceneDimensions) {

unpackedCoordinate = {}

for (int j = 0; j < voxSceneDimensions.size(); j++) {

(unpackedCoordinate[j], packedCoordinate) =

FunctionUnpackPos(packedCoordinate, voxSceneDimensions[j])

}

reuturn unpackedCoordinate;

}

##### Conversion of matrix to voxel block representation format

The voxel-based scene representation is coded lossless for its efficient storage and transmission. The voxel matrix vox to compressed format BS_vox conversion is defined by the following pseudo-code:

// ---------------------------------------------------------------------------------------

// Voxel matrix to voxel blocks conversion

//

// Input

//  voxel_matrix

// Output

//  voxel_blocks

//   id: matrix entry in the voxel block element

//   p0: voxel indices of the first (start) voxel defining the voxel block element

//   p1: voxel indices of the second (end) voxel defining the voxel block element

// ---------------------------------------------------------------------------------------

convert_voxel_matrix_to_blocks

{

vox_matrix_tmp = voxel_matrix;

for (unsigned int x = 0; x < dim[0]; ++x) {

for (unsigned int y = 0; y < dim[1]; ++y) {

for (unsigned int z = 0; z < dim[2]; ++z) {

mat_id id = vox_matrix_tmp[x][y][z]

if (id != 0) {

if (materials.find(id) != materials.end()) {

materials.emplace(id);

}

}

}

}

}

for (mat_id id : materials) {

std::array<unsigned int, N> last_ps = {0, 0, 0};

do {

bool vox_matrix_id_is_empty = true;

unsigned int x, y, z;

voxel_block block;

for (z = last_ps[2]; z < dim[2]; ++z) {

for (y = last_ps[1]; y < dim[1]; ++y) {

for (x = last_ps[0]; x < dim[0]; ++x) {

if (vox_matrix_tmp[x][y][z] == id) {

vox_matrix_id_is_empty = false;

break;

}

}

last_ps[0] = 0;

if (vox_matrix_id_is_empty == false)

break;

}

last_ps[1] = 0;

if (vox_matrix_id_is_empty == false)

break;

}

last_ps[0] = 0;

if (vox_matrix_id_is_empty)

break;

last_ps = ps;

// Block initialization

std::array<unsigned int, N> ps{x, y, z};

std::array<unsigned int, N> pe = ps;

std::array<bool, N> checked_dim = {false, false, false};

// Search voxel matrix for one voxel block

for (unsigned int i = 0; i < N; i++)

process_one_dimension(vox_matrix_tmp, id, ps, pe, checked_dim, dim);

// Add voxel block to voxel block list

block.id = id;

block.p0 = ps;

block.p1 = pe;

voxel_bocks.push_back(block);

} while (true);

}

}

process_one_dimension

{

for (unsigned int j = 0; j < N; j++) {

i_e[j] = 0;

D[j].clear();

B[j].clear();

for (unsigned int k = ps[j]; k <= pe[j]; k++) {

B[j].push_back(k);

}

}

for (unsigned int k = 0; k < N; k++) {

if (checked_dim[k] == false) {

for (unsigned int i = ps[k]; i < dim[k]; i++) {

i_e[k] = i + 1;

unsigned int P = min(i_e[k], dim[k] - 1);

for (unsigned int j = 0; j < N; j++)

D[j] = B[j];

D[k] = std::vector<unsigned int>(1, P);

bool is_empty = false;

for (unsigned int x : D[0]) {

for (unsigned int y : D[1]) {

for (unsigned int z : D[2]) {

if (vox_matrix[x][y][z] != id) {

is_empty = true;

break;

}

}

if (is_empty == true)

break;

}

if (is_empty == true)

break;

}

if (is_empty == true)

break;

}

}

}

// Find dimension with maximum value

int max_val = std::numeric_limits<int>::lowest()

int max_dim = 0;

for (unsigned int j = 0; j < N; j++) {

int diff = (int)i_e[j] - (int)ps[j];

if (diff > max_val) {

max_val = diff;

max_dim = j;

}

}

pe[max_dim] = i_e[max_dim] - 1;

B[max_dim].clear();

for (unsigned int k = ps[max_dim]; k <= pe[max_dim]; k++)

B[max_dim].push_back(k);

// Clear block in voxel matrix

for (unsigned int x : B[0])

for (unsigned int y : B[1])

for (unsigned int z : B[2])

vox_matrix_id[x][y][z] = 0;

checked_dim[max_dim] = true;

}

#### Directivity payload data structure

directivitiesCount	This value represents the number of source directivities that are present in the payload.

directivityId	This value is the identifier for this source directivity

directivityCodedLength	This integer represents the size of the coded source directivity data in Bytes.

direcCoverCount	This integer represents the number of covers that are available.

direcFreqQuantType	This integer determines the quantization type of the frequency for every cover.

Table 107 — Value of direcFreqQuantValue

| Quantization Type | Value |
| --- | --- |
| Octave Quantization | 0 |
| Third Octave Quantization | 1 |
| Sixth Octave Quantization | 2 |
| Integer Quantization | 3 |

direcUseRawBaseline	This flag determines which mode of quantization that the cover is using. If true, it uses the baseline mode, if false, it uses the optimized mode.

#### Diffraction payload data structure

Data belonging to diffrVoxelGrid():

diffVoxelOriginX	This element indicates the X-coordinate of the voxel at the origin of the scene. Trimesh python library can generate its coordinate from a given scene.

diffVoxelOriginY	This element indicates the Y-coordinate of the voxel at the scene origin.

diffVoxelOriginZ	This element indicates the Z-coordinate of the voxel at the scene origin.

diffVoxelPitchX	This element indicates the side length of a voxel along the X axis.

diffVoxelPitchY	This element indicates the side length of a voxel along the Y axis.

diffVoxelPitchZ	This element indicates the side length of a voxel along the Z axis.

diffVoxelShapeX	This element indicates the number of voxels along the Y axis within the voxelized scene.

diffVoxelShapeY	This element indicates the number of voxels along the Y axis within the voxelized scene.

diffVoxelShapeZ	This element indicates the number of voxels along the Z axis within the voxelized scene.

Data belonging to diffrStaticEdgeList():

diffrHasStaticEdgeData	This element indicates the presence of static edge data.

codebookEdgeID	This element is the generic codebook for the edgeId element.

codebookVtxID	This element is the generic codebook for the edgeVertexId1  and edgeVertexId2 element.

codebookTriID	This element is the generic codebook for the elements  edgeAdjacentTriangleID1 and edgeAdjacentTriangleID2.

numberOfStaticEdges	This element indicates the number of potential edges on the combined static geometry which can be created from a single or multiple static geometries.

staticEdge[i]	This element contains the data structure of a static diffraction edge.

Data belonging to diffrEdges():

edgeId	This element indicates the edge Id starting from 0 as a non-negative integer.

edgeVertexId1	Two vertex points are allocated to define unique edge position and this element, which refers to the static diffraction mesh, indicates the first edge vertex Id of a given edge.

edgeVertexId2	This element, which refers to the static diffraction mesh, indicates the second edge vertex Id of a given edge.

edgeVertexPos1X	This internal element represents the X coordinate of the first edge vertex of a given edge. It is a copy of the X coordinate of vertices[edgeVertexId1] of the static diffraction mesh.

edgeVertexPos1Y	This internal element represents the Y coordinate of the first edge vertex of a given edge. It is a copy of the Y coordinate of vertices[edgeVertexId1] of the static diffraction mesh.

edgeVertexPos1Z	This internal element represents the Z coordinate of the first edge vertex of a given edge. It is a copy of the Z coordinate of vertices[edgeVertexId1] of the static diffraction mesh.

edgeVertexPos2X	This internal element represents the X coordinate of the second edge vertex of a given edge. It is a copy of the X coordinate of vertices[edgeVertexId2] of the static diffraction mesh.

edgeVertexPos2Y	This internal element represents the Y coordinate of the second edge vertex of a given edge. It is a copy of the Y coordinate of vertices[edgeVertexId2] of the static diffraction mesh.

edgeVertexPos2Z	This internal element represents the Z coordinate of the second edge vertex of a given edge. It is a copy of the Z coordinate of vertices[edgeVertexId2] of the static diffraction mesh.

edgeVectorX	This internal element represents the X coordinate of the edge vector edgeVector:
edgeVectorX = edgeVertexPos2X – edgeVertexPos1X

edgeVectorY	This internal element represents the Y coordinate of the edge vector edgeVector:
edgeVectorY = edgeVertexPos2Y – edgeVertexPos1Y

edgeVectorZ	This internal element represents the Z coordinate of the edge vector edgeVector:
edgeVectorZ = edgeVertexPos2Z – edgeVertexPos1Z

edgeNormalizedDirectionX	This internal element represents the X coordinate of the normalized edge vector:
edgeNormalizedDirectionX = edgeVectorX / norm(edgeVector)

edgeNormalizedDirectionY	This internal element represents the Y coordinate of the normalized edge vector:
edgeNormalizedDirectionY = edgeVectorY / norm(edgeVector)

edgeNormalizedDirectionZ	This internal element represents the Z coordinate of the normalized edge vector:
edgeNormalizedDirectionZ = edgeVectorZ / norm(edgeVector)

edgeLength	This internal element represents the edge length in meter: 
edgeLength = norm(edgeVector)

edgeAdjacentTriangleId1	This element, which refers to the static diffraction mesh, represents the triangle Id of the first adjacent triangle.

edgeAdjacentTriangleId2	This element, which refers to the static diffraction mesh, represents the triangle Id of the second adjacent triangle.

edgeAdjacentTriangle1Vertex0	This internal element represents the first vertex Id of the first adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId1][0].

edgeAdjacantTriangle1Vertex1	This internal element represents the second vertex Id of the first adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId1][1].

edgeAdjacantTriangle1Vertex2	This internal element represents the third vertex Id of the first adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId1][2].

edgeAdjacantTriangle2Vertex0	This internal element represents the first vertex Id of the second adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId2][0].

edgeAdjacantTriangle2Vertex1	This internal element represents the second vertex Id of the second adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId2][1].

edgeAdjacantTriangle2Vertex2	This internal element represents the third vertex Id of the second adjacent triangle. It is a copy of the static diffraction mesh triangle triangles[edgeAdjacentTriangleId2][2].

edgeAdjacantTriangle1CenterX	This internal element represents the X coordinate of the first adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle1CenterY	This internal element represents the Y coordinate of the first adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle1CenterZ	This internal element represents the Z coordinate of the first adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2CenterX	This internal element represents the X coordinate of the second adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2CenterY	This internal element represents the Y coordinate of the second adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2CenterZ	This internal element represents the Z coordinate of the second adjacent triangle’s centroid (computed by the mean of the static diffraction mesh vertices defined above).

edgeAdjacantTriangle1NormalX	This internal element represents the X coordinate of the first adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAdjacantTriangle1NormalY	This internal element represents the Y coordinate of the first adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAdjacantTriangle1NormalZ	This internal element represents the Z coordinate of the first adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2NormalX	This internal element represents the X coordinate of the second adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2NormalY	This internal element represents the Y coordinate of the second adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAdjacantTriangle2NormalZ	This internal element represents the Z coordinate of the second adjacent triangle’s surface normal (computed from the static diffraction mesh vertices defined above).

edgeAngle	This element represents the angle of the edge. It is computed as follows:
center0 = {edgeAdjacantTriangle1CenterX,
           edgeAdjacantTriangle1CenterY,
           edgeAdjacantTriangle1CenterZ}
center1 = {edgeAdjacantTriangle2CenterX,
           edgeAdjacantTriangle2CenterY,
           edgeAdjacantTriangle2CenterZ}
normal0 = {edgeAdjacantTriangle1NormalX,
           edgeAdjacantTriangle1NormalY,
           edgeAdjacantTriangle1NormalZ}
normal1 = {edgeAdjacantTriangle2NormalX,
           edgeAdjacantTriangle2NormalY,
           edgeAdjacantTriangle2NormalZ}
angleAbs = 180 - acos(dot(normal0, normal1)) * 180 / pi
d0 = dot(normal0, center0)
center1_plane0_dist = dot(normal0, center1) - d0
angleSgn = sgn(center1_plane0_dist) * abs(angleAbs)
edgeAngle = mod(angleSgn + 360, 360)

edgeCenterPointX	This internal element represents the X coordinate of the center position of the edge.

edgeCenterPointY 	This internal element represents the Y coordinate of the center position of the edge.

edgeCenterPointZ 	This internal element represents the Z coordinate of the center position of the edge.

edgeCenterPointVisibilityTestX	This element represents the X coordinate of the position cp_vt used for the visibility check. The position cp_vt is computed as follows:
cp = {edgeCenterPointX,
      edgeCenterPointY,
      edgeCenterPointZ}
normal_sum = normal0 + normal1
normal_mean = normal_sum / norm(normal_sum)
cp_vt = cp + 0.001 * normal_mean

edgeCenterPointVisibilityTestY 	This element represents the Y coordinate of the position cp_vt used for the visibility check.

edgeCenterPointVisibilityTestZ 	This element represents the Z coordinate of the position cp_vt used for the visibility check.

edgeIsRounded	This element indicates if a given edge is a part of the rounded surface or not.

edgeIsRelevant	This element indicates if a given edge is potentially relevant for diffraction at a reachable listener location. For a static scene with static geometries and sources, it indicates if a given edge is reachable from a source with a given encoding parameters. For a dynamic edge, all edges are marked as relevant.

Data belonging to diffrStaticPathDict():

diffrHasStaticPathData	This element indicates the presence of static path data.

staticPathDict	This element contains the data structure of a static path dictionary.

Data belonging to diffrPathDict():

codebookEdgeIDSeqLen	This element is the generic codebook for the element numberOfEdgesInPath.

codebookEdgeIDSeq	This element is the generic codebook for the element edgeId[i][j][k].

codebookAngleSeq	This element is the generic codebook for the element angle[i][j][k].

numBitsForAngle	This element represents the number of bits used for quantizing the element angle[i][j][k].

numberOfRelevantEdges	This element represents the number of relevant edges.

numberOfPaths	This element represents the number of potential paths for a given mesh and the process of decoding and allocating relevant path information into a certain type of data structure will be iteratively called as many as the given value.

numberOfEdgesInPath	This element represents the number of edges included in a potential path and it can be considered as path order. This element will be used to generate a Path instance with the given number of relevant edges, faces/triangles, and angles.

edgeId[i][j][k]	This element indicates an edge Id to generate a Path instance.

faceIndicator[i][j][k]	This element indicates which adjacent face/triangle is associated to define a diffraction direction along a given edge. 0 indicates the first adjacent triangle and 1 indicates the second adjacent triangle to the associated edge.

angle[i][j][k]	This quantized element represents an associated angle within a given path. The de-quantized angle sequence angleSeq[i][j][k] is derived from this element as follows:
quant2angle = 360 / (2 ^ numBitsForAngle)
angleSeq[i][j][k] = quant2angle * angle[i][j][k]

Data belonging to diffrListenerVoxelDict():

diffrHasListenerVoxelData	This element indicates the presence of listener voxel data.

codebookVcX	This element is the generic codebook for the element listenerVoxelGridIndexX.

codebookVcY	This element is the generic codebook for the element listenerVoxelGridIndexY.

codebookVcZ	This element is the generic codebook for the element listenerVoxelGridIndexZ.

codebookNumEdges	This element is the generic codebook for the element numberOfEdgesAdded.

codebookEdgeId	This element is the generic codebook for the element diffrListenerVoxelEdge[i][k].

codebookIndicesRemoved	This element is the generic codebook for the element diffrListenerVoxelIndexDiff[i][k].

numberOfListenerVoxels	This element represents the number of user-/listener-reachable voxels in a scene.

hasVoxelCoordZ	This element indicates the presence of z-coordinate data.

hasVoxelCoordY	This element indicates the presence of y-coordinate data.

hasVoxelCoordX	This element indicates the presence of x-coordinate data.

listenerVoxelGridIndexX[i]	This element represents the listener-reachable voxel X index.

listenerVoxelGridIndexY[i]	This element represents the listener-reachable voxel Y index.

listenerVoxelGridIndexZ[i]	This element represents the listener-reachable voxel Z index.

diffrListenerVoxelMode[i]	This element indicates the differential voxel encoding mode:
0: no reference voxel data
1: use neighbour in negative x-direction as reference
2: use neighbour in negative y-direction as reference
3: use neighbour in negative z-direction as reference

diffrListenerVoxelIndexDiff[i][k]	This element represents the differential encoding of the removal list indices.

numberOfEdgesAdded	This element represents the number of edges that are added to the list of listener-reachable edges for the current voxel.

diffrListenerVoxelEdge[i][j]	This element represents an edge Id that is added to the list of listener-reachable edges for the current voxel.

diffrListenerVoxelDatabase[l][j]	This element is the decoded listener voxel dictionary that contains for a voxel l the list of visible edge IDs.

Data belonging to diffrSourceVoxelDict():

diffrHasSourceVoxelData	This element indicates the presence of source voxel data.

numberOfStaticSources	This element represents the number of static sources in a scene.

staticSourceId	This element represents a static source’s Id.

numberOfVoxelsPerStaticSource	This element represents the number of occupied voxels by a static source.

sourceVoxelGridIndexX	This element represents the source voxel X index.

sourceVoxelGridIndexY	This element represents the source voxel Y index.

sourceVoxelGridIndexZ	This element represents the source voxel Z index.

numberOfEdgesPerSourceVoxel	This element represents the number of potential edges at a given source voxel.

sourceVisibleEdgeId	This element represents potential edge Id at a given source voxel.

Data belonging to diffrValidPathDict():

diffrHasValidPathData	This element indicates the presence of valid path data.

numberOfValidStaticSources	This element represents the number of static sources.

validStaticSourceId	This element represents a static source Id.

codebookVcX	This element is the generic codebook for the element validListenerVoxelGridIndexX.

codebookVcY	This element is the generic codebook for the element validListenerVoxelGridIndexY.

codebookVcZ	This element is the generic codebook for the element validListenerVoxelGridIndexZ.

codebookNumPaths	This element is the generic codebook for the element numberOfPathsAdded[i][j].

codebookEdgeId	This element is the generic codebook for the element diffrValidPathEdge[i][j][k].

codebookPathId	This element is the generic codebook for the element diffrValidPathPath[i][j][k].

codebookIndicesRemoved	This element is the generic codebook for the element diffrValidPathIndexDiff[i][j][k].

numberOfMaximumListenerVoxels	This element represents the number of listener-reachable voxels.

validListenerVoxelGridIndexX[i][j]	This element represents the listener-reachable voxel’s X index to link the valid path data from a given source location.

validListenerVoxelGridIndexY[i][j]	This element represents the listener-reachable voxel’s Y index for valid path data

validListenerVoxelGridIndexZ[i][j]	This element represents the listener-reachable voxel’s Z index for valid path data.

diffrValidPathMode[i][j]	This elements indicates the differential voxel encoding mode:
0: no reference voxel data
1: use neighbour in negative x-direction as reference
2: use neighbour in negative y-direction as reference
3: use neighbour in negative z-direction as reference

diffrValidPathIndexDiff[i][j][k]	This element represents the differential encoding of the removal list indices.

numberOfPathsAdded[i][j]	This element represents the number of paths that are added to the list of valid paths for the current voxel.

diffrValidPathEdge[i][j][k]	This element represents the Id of the initial edge within a given valid path and this edge is visible to a given static source.

diffrValidPathPath[i][j][k]	This element represents the valid path index to make it possible to fetch the precomputed path from the list of paths starting from a given initial edge.

Data belonging to diffrDynamicEdges():

diffrHasDynamicEdgeData	This element indicates the presence of dynamic edge data.

dynamicGeometryCount	This element represents the number of dynamic geometries – the number of dynamic lists.

geometryId[i]	This element represents a (dynamic) geometry Id which is used as a key when creating a dictionary of dynamic lists per dynamic geometry.

codebookEdgeID	This element is the generic codebook for the edgeId element.

codebookVtxID	This element is the generic codebook for the edgeVertexId1  and edgeVertexId2 element.

codebookTriID	This element is the generic codebook for the elements  edgeAdjacentTriangleID1 and edgeAdjacentTriangleID2.

dynamicEdgesCount	This element represents the number of edges on a given geometry.

dynamicEdge[i][j]	This element contains the data structure of a dynamic diffraction edge.

Data belonging to diffrDynamicPaths():

diffrHasDynamicPathData	This element indicates the presence of dynamic path data.

dynamicGeometryCount	This element represents the number of dynamic geometries

relevantGeometryId	This element represents a (dynamic) geometry Id.

dynamicPathDict[g]	This element contains the data structure of a dynamic path dictionary.

#### Voxel payload data structure

numberOfVoxSubScenes	This element represents the number of voxel sub-scenes.

voxScenePosS	This element represents the global coordinates of the first corner point defining the voxel scene dimensions.

voxScenePosE	This element represents the global coordinates of the second corner point defining the voxel scene dimensions.

voxSceneDimensions	This element represents the number of voxels for each voxel scene dimension.

numberOfVoxSceneElements	This element represents the number of block elements defining the voxel scene geometry.

voxSceneElementMatID	This element represents the material ID for the voxels in the voxel block element.

voxSceneElementPosPackedS	This element represents the packed form of the variable voxSceneElementPosS indicating the voxel indices of the first (start) voxel defining the voxel block element.

voxSceneElementPosPackedE	This element represents the packed form of the variable voxSceneElementPosE indicating the voxel indices of the second (end) voxel defining the voxel block element.

numberOfVoxSceneMaterials	This element represents the number of materials defined for the voxel (sub-) scene.

voxSceneMaterialID	This element represents the material ID for mapping the material properties to voxSceneElementMatID.

voxSceneOcclusionInstructionsPresentFlag

This element indicates whether the occlusion modelling instructions are present or not.

voxSceneOcclusionRecoveryFlag	This element indicates whether the occlusion recovery control value is present or not.

voxSceneOcclusionRecoveryValue	This element represents the control value for the occlusion recovery effect.

voxSceneOcclusionSourceMaterialDependencyFlag
This element indicates whether the occlusion modelling instructions for the source-material pendant deactivation are present or not.

numberOfSourceMaterialEntries	This element represents the number of source-material pairs for source-material pendant deactivation of the occlusion modelling.

voxSceneOcclusionSkipForObjectID	This element represents the object source ID (corresponding to objectSourceId) of the source-material pair for which occlusion modelling is skipped.

voxSceneOcclusionSkipForMaterialID	This element represents the material ID (corresponding to materialId) of the source-material pair for which occlusion modelling is skipped.

voxSceneDiffractionInstructionsPresentFlag

This element indicates whether the diffraction modelling is material selective or not.

voxSceneDiffractionEnabledFlag	This element indicates whether the diffraction modelling is enabled for the corresponding material or not.

voxSceneDiffractionMapPresentFlag	This field indicates whether the diffraction map for diffraction modelling is present or not.

voxSceneDiffractionMapHeightPresentFlag

This field indicates whether the height for automatic generation of the diffraction map is present or not.

voxSceneDiffractionMapHeight	This field represents the height on which the scene is considered for an automatic generation of the diffraction map for diffraction modelling.

voxSceneDiffractionEffectControlFlag	
This element indicates whether the diffraction control value is present or not.

voxSceneDiffractionEffectControlValue	
This element represents the control value for the diffraction modelling effect.

numberOfVoxDiffractionMapElements	This element represents the number of block elements defining the voxel scene diffraction map.

voxDiffractionMapValue	This element represents the voxel type ID for the diffraction map in the voxel block element.

voxDiffractionMapPosPackedS	This element represents the packed form of the variable voxDiffractionMapPosS indicating the voxel indices of the first (start) voxel defining the diffraction map block element.

voxDiffractionMapPosPackedE	This element represents the packed form of the variable voxDiffractionMapPosE indicating the voxel indices of the second (end) voxel defining the diffraction map block element.

numberOfVoxSceneUpdates	This element represents the number of voxel scene geometry updates.

voxSceneUpdateID	This element represents the voxel scene geometry update ID.

voxSceneUpdateReferenceFlag	This element indicates whether the voxel scene geometry update has the reference position or not.

voxSceneUpdateReferencePos	This element represents the reference point for scene coordinate mapping to voxel indices. This reference point corresponds to the variable VoxScenePosS update.

numberOfVoxSceneUpdateSequences	This element represents the number of the update sequences.

voxSceneUpdateTime	This element represents the update duration in milliseconds.

numberOfVoxDiffractionPathData	This element represents the number of pre-computed diffraction path data sets.

voxDiffractionPathStartVoxelPacked	This element represents the packed form of the variable voxDiffractionPathStartVoxel indicating the voxel indices of the path start voxel of the pre-computed diffraction path.

voxDiffractionPathEndVoxelPacked	This element represents the packed form of the variable voxDiffractionPathEndVoxel indicating the voxel indices of the path end voxel of the pre-computed diffraction path.

voxDiffractionPathDataExistFlag	This element indicates whether the diffraction path exists or not.

voxDiffractionSourceDirectionPacked
This element represents the packed form of the variable voxDiffractionSourceDirection indicating the voxel indices of the voxel for determining diffracted source azimuth value.

voxDiffractionPathLength	This element represents the diffraction path length on the diffraction map 2D matrix.

pcpdPresent	This element indicates presence of pre-computed path data for voxel diffraction.

pcpdFull	Indicates pre-computed path data encoding mode. 0: selective, 1: full

pcpdPresentForStartPosition	Indicates whether pre-computed path data is available for a specific start position.

pcpdStartVoxelPacked	This element represents the packed form of the variable voxDiffractionPathStartVoxel indicating the voxel indices of the path start voxel of the pre-computed diffraction path.

pcpdEndVoxelPacked	This element represents the packed form of the variable voxDiffractionPathEndVoxel indicating the voxel indices of the path end voxel of the pre-computed diffraction path.

pcpdPathExistFlag	This element indicates whether the diffraction path exists or not.

pcpdUsePreviousPath	Indicates whether the previously transmitted path information shall be reused.

pcpdUsePreviousDirection	Indicates whether the previously transmitted direction information shall be reused.

pcpdSourceDirectionPacked	This element represents the packed form of the variable voxDiffractionSourceDirection indicating the voxel indices of the voxel for determining diffracted source azimuth value.

pcpdPathLength	This element represents the diffraction path length on the diffraction map 2D matrix.

pcpdPathDelta	Encoded path-length delta. Shall be encoded according to Table 108

Table 108 – Encoding of pcpdPathDelta

| bits | Value |
| --- | --- |
| 0b00 | 1 |
| 0b01 | sqrt(2)-1 |
| 0b10 | 1-sqrt(2) |
| 0b11 | -1 |

voxReflectionNumClustersPresentFlag

This flag indicates whether the number of clusters is present or not.

voxReflectionNumClusters	This element represents the number of clusters for the modelled early reflection incoming directions 
NS = voxReflectionNumClusters + 2.

voxReflectionTuningIndoorOutdoorTypeFlag

Flag indicating the presence of additional gains (dB) to be used for early reflection rendering for indoor/outdoor environment types.

voxReflectionEarlyTuningIndoorGainDb

This value is the additional tuning gain (dB) to be used for early reflection rendering for the indoor environment type. 
It shall range between -119 to +12 (dB). Default value is 0 (dB).

voxReflectionEarlyTuningOutdoorGainDb

This value is the additional tuning gain (dB) to be used for early reflection rendering for the outdoor environment type. 
It shall range between -119 to +12 (dB). Default value is 0 (dB).

voxReflectionNumOriginsPresentFlag

This flag indicates whether the number of origin points is present or not.

voxReflectionNumOrigins	This element represents the number of origin points for the volumetric ray marching algorithm 
NP = voxReflectionNumOrigins + 2.

voxReflectionSectorDownmixFlag

This element indicates whether the reflection modeling produces one rendering item per cluster or not.

#### Early reflection payload data structure

earlyTriangleCullingDistanceOrder1	Triangle culling distance for 1st order reflections.

earlyTriangleCullingDistanceOrder2	Triangle culling distance for 2nd order reflections.

earlySourceCullingDistanceOrder1	Source culling distance for 1st order reflections.

earlySourceCullingDistanceOrder2	Source culling distance for 2nd order reflections.

earlyVoxelGridOriginX	x-component of the Cartesian coordinate of the voxel grid origin [0,0,0].

earlyVoxelGridOriginY	y-component of the Cartesian coordinate of the voxel grid origin [0,0,0].

earlyVoxelGridOriginZ	z-component of the Cartesian coordinate of the voxel grid origin [0,0,0].

earlyVoxelGridPitchX	Voxel grid spacing along the x-axis (voxel width).

earlyVoxelGridPitchY	Voxel grid spacing along the y-axis (voxel length).

earlyVoxelGridPitchZ	Voxel grid spacing along the z-axis (voxel height).

earlyVoxelGridShapeX	Number of voxels along the x-axis.

earlyVoxelGridShapeY	Number of voxels along the y-axis.

earlyVoxelGridShapeZ	Number of voxels along the z-axis.

earlyHasSurfaceData	Flag indicating the presence of earlySurfaceData.

earlySurfaceDataLength	Length of the earlySurfaceData block in bytes.

earlyHasVoxelData	Flag indicating the presence of earlyVoxelData.

earlyVoxelDataLength	Length of the earlySurfaceData block in bytes.

earlySurfaceDistOffset	Offset in mm for earlySurfaceDist.

numberOfSurfaces	Number of surfaces.

earlySurfaceLengthFaceIdx	Array length of earlySurfaceFaceIdx.

earlySurfaceFaceIdx	List of triangle IDs.

earlySurfaceAzi	Array with azimuth angles specifying the surface normals in spherical coordinates (Hesse normal form).

earlySurfaceEle	Array with elevation angles specifying the surface normals in spherical coordinates (Hesse normal form).

earlySurfaceDist	Array with distance values (Hesse normal form).

numberOfVoxelPairs	Number of source & listener voxel pairs with available voxel data.

earlyVoxelL	Array with listener voxel indices.

earlyVoxelS	Array with source voxel indices.

earlyVoxelMode	Array specifying the encoding mode of the voxel data.

earlyVoxelIndicesRemovedDiff	Differentially encoded removal list specifying the indices of the reference reflection sequence list that shall be removed.

earlyVoxelNumPaths	Number of reflection paths.

earlyVoxelOrder	2D Array specifying the reflection order.

earlyVoxelSurf	Reflection sequences given as 3D array of surface indices.

lcEarlyNumberOfEnvironments	Number of low-complexity early reflection environments

lcEarlyEnvType	Type of low-complexity early reflection environments

Table 109 – Encoding of lcEarlyEnvType

| int | Value |
| --- | --- |
| 0 | Not set |
| 1 | LCERindoor |
| 2 | LCERoutdoor |
| 3..15 | reserved |

lcEarlyEnvironmentPriority	Priority of low-complexity environment

lcEarlyAmpFac1Db	First low-complexity outdoor amplification in dB

lcEarlyAmpFac2Db	Second low-complexity outdoor amplification  in dB

lcEarlyPreDelay1	First low-complexity outdoor predelay

lcEarlyPreDelay2	Second low-complexity outdoor predelay

lcEarlyBoundingBoxPositionX	X-position of low-complexity environment bounding box

lcEarlyBoundingBoxPositionY	Y-position of low-complexity environment bounding box

lcEarlyBoundingBoxPositionZ	Z-position of low-complexity environment bounding box

lcEarlyBoundingBoxOrientationYaw	Yaw orientation of low-complexity environment bounding box

lcEarlyBoundingBoxOrientationPitch	Pitch orientation of low-complexity environment bounding box

lcEarlyBoundingBoxOrientationRoll	Roll orientation of low-complexity environment bounding box

lcEarlyBoundingBoxSizeX	Size X of low-complexity environment bounding box

lcEarlyBoundingBoxSizeY	Size Y of low-complexity environment bounding box

lcEarlyBoundingBoxSizeZ	Size Z of low-complexity environment bounding box

#### Portal payload data structure

numPortals	This value indicates the total number of portals in the scene.

isExplicitPortalMode	Indicates whether the scene uses implicit or explicit portals. When true, explicit portal data is provided.

portalId	This value indicates a unique identification for each portal in the scene.

hasExtent	Flag indicating whether a portal extent geometry is indicated.

portalExtentId	This value indicates a unique identification of the geometry representing the spatial extent of this portal.

isConnectedToAE	Flag indicating whether either side is connected to an acoustic environment or neither.

portalEnv1	Indicates the first acoustic environment the portal is connected to.

isConnectedToSecondAE	Flag indicating whether on one side of the portal is a region that is not defined as an acoustic environment. Typically, the ‘outdoors’ region.

portalEnv2 	Indicates the second acoustic environment the portal is connected to.

hasMaterial	Flag indicating whether the portal has a material associated with it.

portalMaterialId	Material ID for the portal.

portalNormal	Indicates the normal vector for the portal, pointing into portalEnv1, and perpendicular to the centre of the portal.

explicitPortalProperties	Flag indicating whether explicit portal properties are provided. If not, these shall be derived from the portal extent geometry.

portalCenterPos	Location of the centre of the portal (x, y, z).

equivalentPortalWidth	Explicit indication of a box-equivalent portal width. Size in x-axis direction if portalNormal were [0, 0, -1].

equivalentPortalHeight	Explicit indication of a box-equivalent portal height. Size in y-axis direction if portalNormal were [0, 0, -1].

equivalentPortalDepth	Explicit indication of a box-equivalent portal depth. Size in z-axis direction if portalNormal were [0, 0, -1].

hasPortal2PortalFactors	Flag indicating whether portal to portal factors are included in the portal payload.

hasAcousticLink	Flag indicating whether two portals are acoustically linked. When set to true, a portal2PortalFactor is available for those portals.

portal2PortalFactorCode	Huffman code indicating the portal2PortalFactor according to Table A.24 —.

portalParentEnvId	ID of the acoustic environment to which the implicit portal applies.

hasAeLosVoxelData 	Flag indicating whether acoustic environment Line-of-Sight (LoS) voxel data is present. The AE LoS voxel data shall be such that the voxel grid covers at least a portion of an AE.

numAEs	Indicates the number of AEs for which LoS voxel data is provided.

aeBsId	Bitstream identifier of the AE.

hasOutsideLosVoxelData	Flag indicating whether outside LoS voxel data is present. Outside LoS voxel data shall be created for portals which only have one AE defined to cover an area at the side of the portal where no AE connection is defined, in most cases covering an area in the “outside” region of the scene.

numOutsideAreas	Number of outside LoS voxel grids.

nVoxelsX	Number of voxels along the x-axis.

nVoxelsY	Number of voxels along the y-axis.

nVoxelsZ	Number of voxels along z-axis.

cornerP1	Coordinate of top-left AE bounding geometry diagonal corner-point. The bounding geometry shall be a rectangular axes-aligned region. If x, y, and z represent the corner coordinate vectors of the geometry then cornerP1 = [min(x), max(y), min(z)].

cornerP2	Coordinates of bottom-right AE bounding geometry diagonal corner-point. If x, y, and z represent the corner coordinate vectors of the geometry then cornerP1 = [max(x), min(y), max(z)]

voxelSideLenCm	Length of LoS voxel side in cm.

portalCount	Number of portals for which LoS data is provided.

portalBsId	Bitstream identifier of the portal for which LoS data is provided.

nLosBitMasks	Number of bitmasks the portal LoS info consists of.

nMaskBytes	Number of bytes in the run length encoded LoS bitmask.

maskByte	Run length encoded code value of LoS bitmask.

#### Reverberation payload data structure

revPresent	Flag indicating whether reverberation data is present.

revNrElements	Number of reverberation parameter set.

revAcEnvID	Identifier of the acoustic environment to which the reverberation parameter set relates.

revFreqGridIdx	Index in the list of frequency grids defined in FreqGridData().

revPredelay	Offset in seconds from where DSR is calculated in the RIR, a delay of 0 coincides with emission at the source.

revDelayLineCountPresent	Indicates whether reverb delay line count is present for the acoustic environment indicated by revAcEnvID.

revDelayLineCount	Decoded value of the number of delay lines for reverb controlling the computational complexity and quality (echo density and diffuseness) of reverberation rendering.

delayLineCountCode	Huffman code indicating the number of delay lines for reverb.

revRT60	RT60 time in seconds calculated from 0 to -30 dB in a linear part of the EDR, after early decay.

revDSR	Diffuse to Source energy Ratio. Diffuse reverberation energy is calculated from revPredelay into the RIR. Source energy is the total emitted source energy that gives rise to that diffuse energy.

dsrCode	Huffman code indicating the DSR value.

revRenderControlPresent	Indicates whether distance attenuation control data is present, overriding the default settings.

revDistanceGainDropDb	Decibel constant for controlling the amount of distance gain attenuation in late reverberation. Distance doubling reduces reverberation level by reverbDistanceGainDropDb decibels. When revRenderControlPresent == false, the default value is 1.5 dB.

revMinDistance	Minimum distance threshold applied to late reverberation distance gain attenuation. When revRenderControlPresent == false, the default value is 1 m.

revMultiRoomConfigPresent	Indicates whether multi-room reverberation rendering control data is present, overriding the default values.

revFadeEnabled	This field indicates whether late reverberation shall use reverberation fade-in outside the acoustic environment enclosure.

revFadeDistance	Distance in meters outside an acoustic environment enclosure at which fade in of that acoustic environment reverberation is started. When revMultiRoomConfigPresent == false, the default value is 1 m.

revHeadtrackingEnabled	This field indicates whether late reverberation shall use head tracking of late reverberation output channels for immersive reverberation rendering.

revARConfigPresent	Indicates whether reverberation control parameters are present for AR environments.

revLsdfTheoreticalRDRtoactualRDRdB	Offset in decibels to be added to the log base 10 of an RDR value obtained from a statistical diffuse approximation based on the RT60 value when RDR values have not been provided in an LSDF file or when overideLSDF_RDR is set.

revLsdfRDRtoActualRDRdB	Offset in decibels to be added to the log base 10 of the RDR value obtained from an LSDF file.

revOverrideLSDF_RDR	This field indicates whether late reverberation shall derive RDR values using a statistical diffuse approximation based on the RT60 value instead of the RDR values in an LSDF file.

revDefaultAEPresent 	Flag indicating whether Default AE data is present.

revHasCustomDefaultAE 	Flag signalling a transmitted customized Default AE.

revDefaultAEFreqGridIdx 	Index in the list of frequency grids defined in ReverbFreqGridData(), indicating the grid applicable for the Default AE parameters.

revDefaultAEPredelay 	Offset in seconds from where revDefaultAEDSR is calculated in the RIR, a delay of 0 coincides with emission at the source.

revDefaultAEDelayRatio 	Delay ratio used in calculation of delay line lengths for Default AE (VR) in 6.6.4.3.2.3.

revDefaultAEDelayMinMs	Minimum delay line length in milliseconds used in calculation of delay line lengths for Default AE (VR) in 6.6.4.3.2.3.

revDefaultAERT60 	Default AE RT60 time in seconds calculated from 0 to -30 dB in a linear part of the EDR, after early decay.

revDefaultAEDSR 	Default AE Diffuse to Source energy Ratio.

revDefaultAEDistanceGainDropDb	Decibel constant for controlling the amount of distance gain attenuation in late reverberation for the Default AE. The default value is 6 dB.

revDefaultAEMinimumDistance	Minimum distance threshold applied to Default AE late reverberation distance gain attenuation. The default value is 5 m.

revDefaultAEPreset	Index into a table containing Default AE presets. See A.3 for preset tables.

Table 110 – Value of revDefaultAEPreset.

| bits | ID | Meaning |
| --- | --- | --- |
| 0b000000 | 0 | Default AE preset 0 |
| 0b000001 | 1 | Default AE preset 1 |
| 0b000010 | 2 | Default AE preset 2 |
| 0b000011 | 3 | Default AE preset 3 |
| 0b000100 | 4 | Default AE preset 4 |
| 0b000101 | 5 | Default AE preset 5 |
| 0b000110 | 6 | Default AE preset 6 |
| 0b000111 - 0b111111 | 7-63 | Reserved |

revHasDefaultAEGain 	Flag signalling whether an overall reverberation gain of the Default AE is transmitted.

revDefaultAEGain	Overall gain for the late reverberation of the Default AE.

revHasDefaultAEDelayRatio 	Flag signalling a transmitted delay ratio overriding the respective preset value for the Default AE.

revHasDefaultAEDistanceGainDrop 	Flag signalling a transmitted distance gain drop overriding the respective preset value for the Default AE.

revHasDefaultAEMinDist 	Flag signalling a transmitted minimum distance overriding the respective preset value for the Default AE.

#### Audio plus payload data structure

The Audio Plus Payload contains Audio Plus metadata for audio elements with extent. For more information on the type of audio elements that have Audio Plus metadata see B.2.2.

numberOfSources	Number of audio elements with Audio Plus metadata

For each audio element with Audio Plus metadata, the following metadata is specified:

audioSourceId	Source ID of audio element.

distanceGainModel	Choice of distance gain model (bool). 0 means volumetric model, 1 means point-source model.

hasDiffuseness	Flag signalling if the audio element has a diffuseness parameter (bool).

diffuseness	Diffuseness of the audio element (float) in the range 0.0 to 1.0, where 1.0 means fully diffuse.

#### Dispersion Payload Data Structure

enableDispersionFilterER	A boolean flag that enables or disables the dispersion filter processing for early reflections sounds.

enableDispersionFilterDiffr	A boolean flag that enables or disables the dispersion filter processing for diffracted sounds.

codedDispersionFilterLength	The coded value of dispersionFilterLengthMSec. dispersionFilterLengthMSec = (codedDispersionFilterLength +1)*2;

codedDispersionFilterGain	The coded value of dispersionFilterGaindB. dispersionFilterGaindB =  codedDispersionFilterGain – 21;

enableSpatialDispersion	A parameter that enables spatial dispersion processing.

#### Scene plus payload data structure

sceneDuration	This value is duration of the scene in seconds. If the duration is 0.0, then the scene shall loop. This value represents a range between 0.0 and 180.0. To dequantize it to a floating point value, use the following equation:

| floatVal=180*sceneDuration2noOfBits - 1 | (3) |
| --- | --- |

sceneType	This flag determines the type of the scene, either a VR or an AR scene.

Table 111 — Value of sceneType

| bits | sceneType |
| --- | --- |
| 0b0 | VR |
| 0b1 | AR |

transformsCount	This value is the number of transforms in this payload.

transformId	This value represents the unique identifier for this transform

transformPositionX	This value represents the Cartesian spatial position of the transform along the x dimension in meters

transformPositionY	This value represents the Cartesian spatial position of the transform along the y dimension in meters

transformPositionZ	This value represents the Cartesian spatial position of the transform along the z dimension in meters

transformOrientationYaw	This value represents the Euler orientation of the transform in the Yaw axis in degrees

transformOrientationPitch	This value represents the Euler orientation of the transform in the Pitch axis in degrees

transformOrientationRoll	This value represents the Euler orientation of the transform in the Roll axis in degrees

transformCoordSpace	This flag represents in which coordinate system the transform refers to.

Table 112 — Value of transformCoordSpace

| bits | transformCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

anchorsCount	This value is the number of anchors in this payload

anchorId	This value represents the unique identifier for this anchor.

anchorLsdfRef	This string is the reference for the LSDF anchor which is known to the renderer via the LSDF. The LSDF anchor indicates the position of the anchor in the listener space.

audioStreamsCount	This value is the number of audio streams in this payload

audioStreamId	This value represents the unique identifier for this audio stream.

isLocallyCaptured	This flag indicates that the audio content of the stream is provided to the renderer via local capturing in real time and thus is not available as an audio file at encoding time. The number of input channels is assumed to be one if the flag is set.

audioStreamFilePath	This string is the file path to the respective WAV file for this audio stream. The path is relative to the bitstream itself.

inputChannelsCount	This value represents the number of Input Channels for this audio stream.

inputChannelIndex	This value is the input channel index.

granularSoundsCount	This value is the number of granular sounds in this payload

granularSoundId	This value represents the unique identifier for this granular sound.

granularDatabaseFilePath	This string is the file path to the respective granular database file for this granular sound. The path is relative to the bitstream itself.

granularSoundChannels   	This value represents the number of channels of this granular sound.

materialsCount	This value represents how many acoustic materials are in this payload.

materialId	This value represents the unique identifier for this acoustic material

materialCodedLength	This integer represents the length of the coded material binary in bytes

sourceDirectivitiesCount	This value represents how many source directivities are in this payload.

sourceDirectivityId	This value represents the unique identifier for this source directivity.

primitivesCount	This value represents the number of primitive geometries are in this payload

primitiveType	This integer represents the type of primitive geometry.

Table 113 — Value of primitiveType

| bits | primitiveType |
| --- | --- |
| 0b00 | BOX |
| 0b01 | SPHERE |
| 0b10 | CYLINDER |
| 0b11 | RESERVED |

primitiveId	This value represents the unique identifier for this primitive.

primitivePositionX	This value represents the Cartesian spatial position of the primitive along the x dimension in meters

primitivePositionY	This value represents the Cartesian spatial position of the primitive along the y dimension in meters

primitivePositionZ	This value represents the Cartesian spatial position of the primitive along the z dimension in meters

primitiveOrientationYaw	This value represents the Euler orientation of the primitive in the Yaw axis in degrees

primitiveOrientationPitch	This value represents the Euler orientation of the primitive in the Pitch axis in degrees

primitiveOrientationRoll	This value represents the Euler orientation of the primitive in the Roll axis in degrees

primitiveCoordSpace	This flag represents in which coordinate system the primitive refers to.

Table 114 — Value of primtiiveCoordSpace

| bits | primitiveCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

primitiveSizeX	This value represents the size of the primitive along the X axis in meters. The interpretation of this variable is dependent on the primitiveType.

primitiveSizeY	This value represents the size of the primitive along the Y axis in meters. The interpretation of this variable is dependent on the primitiveType.

primitiveSizeZ	This value represents the size of the primitive along the Z axis in meters. The interpretation of this variable is dependent on the primitiveType.

primitiveHasMaterial	This flag indicates if this primitive has an acoustic material attributed to it.

primitiveMaterialId	This value is the identifier of an acoustic material.

primitiveHasSpatialTransform	This flag indicates if this primitive has a parent spatial transform.

primitiveHasAnchor	This flag indicates if this primitive’s spatial transform is an anchor. If false, then the parent is a regular transform.

primitiveParentAnchorId	This value represents the unique identifier for the parent anchor.

primitiveParentTransformId	This value represents the unique identifier for the parent transform.

isPrimitiveStatic	This flag indicates if the primitive is static or dynamic. If static, then the primitive is stationary throughout the entire duration of the scene, whereas the position of the primitive is updatable if it is dynamic.

isEarlyReflectionPrimitive	This flag indicates if the primitive is added by the geometry data converter to the static mesh for the Early Reflection stage.

meshesCount	This value is the number of meshes in this payload.

meshId	This value is a unique identifier for this mesh.

meshCodedLength	This integer is the length of the coded mesh data in bytes, this data is decoded by meshFaces(), (Table 75).

meshPositionX	This value represents the Cartesian spatial position of the mesh along the x dimension in meters.

meshPositionY	This value represents the Cartesian spatial position of the mesh along the y dimension in meters.

meshPositionZ	This value represents the Cartesian spatial position of the mesh along the z dimension in meters.

meshOrientationYaw	This value represents the Euler orientation of the mesh in the Yaw axis in degrees.

meshOrientationPitch	This value represents the Euler orientation of the mesh in the Pitch axis in degrees.

meshOrientationRoll	This value represents the Euler orientation of the mesh in the Roll axis in degrees.

meshCoordSpace	This flag represents in which coordinate system the mesh refers to.

Table 115 — Value of meshCoordSpace

| bits | meshCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

meshHasSpatialTransform	This flag indicates if this mesh has a parent spatial transform.

meshHasAnchor	This flag indicates if this mesh’s spatial transform is an anchor. If false, then the parent is a regular transform.

meshParentAnchorId	This value represents the unique identifier for the parent anchor.

meshParentTransformId	This value represents the unique identifier for the parent transform.

isMeshStatic	This flag indicates if the mesh is static or dynamic. If static, then the mesh is stationary throughout the entire duration of the scene, whereas the position of the mesh is updatable if it is dynamic.

isEarlyReflectionMesh	This flag indicates if the mesh is added by the geometry data converter to the static mesh for the Early Reflection stage.

environmentsCount	This value represents the number of acoustic environments in this payload.

acousticParametersCount	This value represents the number of acoustic parameters in this environment.

paramPosX	This value represents the Cartesian spatial position of the acoustic parameter along the x dimension in meters.

paramPosY	This value represents the Cartesian spatial position of the acoustic parameter along the y dimension in meters.

paramPosZ	This value represents the Cartesian spatial position of the acoustic parameter along the z dimension in meters.

environmentId	This value is the unique identifier for this acoustic environment.

acousticRegionId	This value is the unique identifier for the geometry that represents the region in this environment.

objectSourcesCount	This value represents the number of object sources in this payload.

hasInputLayout	This flag indicates if the object source has an input layout. An input layout can map multi-channel audio to create an extended sound source.

inputLayoutAlignment	This flag indicates the anchoring mode: either “user” or “object”.

Table 116 — Value of inputLayoutAlignment

| bits | inputLayoutAlignment |
| --- | --- |
| 0b0 | USER |
| 0b1 | OBJECT |

inputLayoutTL	This flag indicates if the source is projected to the top left of the user spatial observation plane.

inputLayoutT	This flag indicates if the source is projected to the top center of the user spatial observation plane.

inputLayoutTR	This flag indicates if the source is projected to the top right of the user spatial observation plane.

inputLayoutL	This flag indicates if the source is projected to the middle left of the user spatial observation plane.

inputLayoutC	This flag indicates if the source is projected to the direct center of the user spatial observation plane.

inputLayoutR	This flag indicates if the source is projected to the middle right of the user spatial observation plane.

inputLayoutBL	This flag indicates if the source is projected to the bottom left of the user spatial observation plane.

inputLayoutB	This flag indicates if the source is projected to the bottom center of the user spatial observation plane.

inputLayoutBR	This flag indicates if the source is projected to the bottom right of the user spatial observation plane.

objectSourceId	This value represents the unique identifier for the object source.

objectSourcePositionX	This value represents the Cartesian spatial position of the object source along the x dimension in meters.

objectSourcePositionY	This value represents the Cartesian spatial position of the object source along the y dimension in meters.

objectSourcePositionZ	This value represents the Cartesian spatial position of the object source along the z dimension in meters.

objectSourceOrientationYaw	This value represents the Euler orientation of the object source in the Yaw axis in degrees.

objectSourceOrientationPitch	This value represents the Euler orientation of the object source in the Pitch axis in degrees.

objectSourceOrientationRoll	This value represents the Euler orientation of the object source in the Roll axis in degrees.

objectSourceCoordSpace	This flag represents in which coordinate system the object source refers to.

Table 117 — Syntax of objectSourceCoordSpace

| bits | objectSourceCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

objectSourceActive	This flag represents if the object source will be rendered.

objectSourceGainDb	This value is the Gain (dB) that adjusts the level of this object source. It ranges between -119.0 to 36.0.

objectSourceHasReverbGain	This flag indicates that this object source has reverbGainDb parameter set.

objectSourceReverbGainDb	This value is the reverb gain (dB) of this object source. It shall range between -119.0 to +12.0. Default value is 0 (dB)

objectSourceRefDistance	This value is the reference distance (m) of the object source. The reference distance is the point in which the source becomes 0dB, which is useful for the distance attenuation law.

objectSourceSignalId	This value represents the unique audio stream identifier that this object source requires for its audio.

objectSourceHasExtent	This flag indicates if the object source is spatially extended.

objectSourceExtentId	This value is the unique geometry identifier that represents the extent of this object source.

objectSourceHasDirectivity	This flag indicates if the object source has directivity data attributed to it.

objectSourceDirectivityId	This value is the unique directivity identifier for that this object source uses for its directivity data.

objectSourceDirectiveness	This value is directiveness coefficient for directivity, indicating its strength. The range is between 0.0 and 20.0. To dequantize it to a floating point value, use the following equation:

| floatVal=20*objectSourceDirectiveness2noOfBits | (4) |
| --- | --- |

objectSourceNoReverb	This flag indicates that no reverberation shall be applied to this object source.

objectSourceNoDoppler	This flag indicates that no Doppler shifts shall be applied to this object source.

objectSourceNoDistance	This flag indicates that no distance attenuation shall be applied to this object source.

objectSourceNoDirectionalFocus	This flag indicates that directional focus processing shall not be applied to this object source.

objectSourceNoCulling	This flag indicates that this object source shall not be removed by culling algorithms.

objectSourceMode	This flag indicates the playback mode for this object source.

Table 118 — Value of objectSouceMode

| bits | objectSourceMode |
| --- | --- |
| 0b0 | CONTINUOUS |
| 0b1 | EVENT |

objectSourcePlay	This flag indicates if playback is enabled for this object source.

objectSourceVDLMethod	This indicates the VDL method combination to be used for the particular object source with the bits vdlMethod. In case that no argument is being provided, “spline” VDL method will be used as a default setting.

objectSourceHasSpatialTransform	This flag indicates if this object source has a parent spatial transform.

objectSourceHasAnchor	This flag indicates if this object source’s spatial transform is an anchor. If false, then the parent is a regular transform.

objectSourceParentAnchorId	This value represents the unique identifier for the parent anchor.

objectSourceParentTransformId	This value represents the unique identifier for the parent transform.

isObjectSourceStatic	This flag indicates if the object source is static or dynamic. If static, then the object source is stationary throughout the entire duration of the scene, whereas the position of the object source is updatable if it is dynamic.

hoaGroupsCount	This value represents how many HOA groups are in this payload.

hoaGroupId	This value represents the unique identifier for this HOA group.

hoaGroupHasRegion	This flag indicates if this HOA group has a region attributed to it.

hoaGroupRegionId	This value is the unique geometry identifier that represents the region of validity for this HOA group.

hoaGroupHasFreqBandConfig	A value equal to 0 indicates that the 6DoF HOA rendering utilizes all the frequency bins specified in the Appendix A.10. A value equal to 1 indicates that the 6DoF HOA rendering utilizes the frequency bins/bands as specified in Section 6.6.26.3.1.4.

highestSingleBinBandsIndex	Indicates the highest bin index until which all the frequency bins specified in Appendix A.10 are used as single bins for the subsequent rendering for the particular HOA group.

lowestHighBandIndex	Indicates the lowest bin index above all the frequency bins are merged and treated as a single bin for the subsequent rendering for the particular HOA group. Additionally, this value also indicates the maximum frequency limit index for spatial metadata calculation, comprising DOA and DTR.

intermediateBandsERBWidth 	Indicates the ERB width for merging the frequency bins in between the highestSingleBinBandsIndex and the lowestHighBandIndex.

exteriorRenderingProjectioRadius 	Indicates the exterior projection radius for exterior 6DoF HOA rendering.

adaptiveExteriorRenderingProjectionRadius 	Indicates whether the exterior projection radius for 6DoF HOA rendering is modified based on informed source positions.

hoaGroupHasInformedSources 	Indicates if the HOA group has informed sources.

maxSimulInformedSources 	Indicates the maximum number of active informed sources for a particular HOA group.

informedSourceCount 	Indicates the number of informed sources for a particular HOA group.

informedSourceId 	Indicates a unique identifier for the informed source.

informedSourcepositionX	Indicates the position of the informed source in cartesian coordinate X axis (in OpenGL notation) with reference to the origin of the audio scene coordinates system. The original value in meters is quantized with toInt(posx, -2000, 2000, 31).

informedSourcepositionY	Indicates the position of the informed source in cartesian coordinate Y axis (in OpenGL notation) with reference to the origin of the audio scene coordinates system. The original value in meters is quantized with toInt(posy, -2000, 2000, 31).

informedSourcepositionZ	Indicates the position of the informed source in cartesian coordinate Z axis (in OpenGL notation) with reference to the origin of the audio scene coordinates system. The original value in meters is quantized with toInt(posz, -2000, 2000, 31).

listenerThresholdPresent 	A value equal to 1 indicates the presence of audio source or informed source distance threshold with respect to the listener position in the audio scene. A value equal to 0 indicates the absence of any such listener proximity threshold.

hoaSourceThresholdPresent 	A value equal to 1 indicates the presence of audio source or informed source distance threshold with respect to the HOA source position in the audio scene. A value equal to 0 indicates the absence of any such HOA source proximity threshold.

lpdInformedSourceEnableThreshold 	Specifies the proximity threshold distance between the listener and any informed source. If the listener is at a distance less than this threshold from any informed source, the informed mode processing is enabled for the corresponding one or more informed source(s). The original value in meters is quantized with toInt(threshold, 0, 100, 16).

hpdInformedSourceEnableThreshold 	Specifies the proximity threshold distance between the HOA source and any informed source. If the HOA source is at a distance greater than this threshold from any informed source, the HOA source shall not be utilized for performing informed mode processing for the corresponding one or more informed source(s). The original value in meters is quantized with toInt(threshold, 0, 100, 16).

priorityValue 	Specifies the importance of this source for informed mode rendering. This value is used to disable less important sources from informed mode rendering when the maximum limit of active informed mode sources is reached.

hoaGroupHasLowProfileConfig	A value equal to 0 indicates that the low complexity MP-HOA renderer will utilize the default values for the parameters distanceFactor (1) and panningGridOrder (0). A value equal to 1 indicates that the low complexity MP-HOA renderer will utilize will set the parameters distanceFactor and panningGridOrder according to the payload. The usage of these two parameters is described in Section 6.6.27.2.

distanceFactor	Indicates a quantized decimal value in the range [0.25, 4] to be multiplied with the VLO radius, as described in Section 6.6.27. To dequantize it to a floating-point value, use the following equation:

| floatVal=0.25+3.75*distanceFactor4 | (5) |
| --- | --- |

panningGridOrder	Indicates the VBAP node grid order to be used by the MP-HOA renderer, as described in Section 6.6.27. If set to 0 (default), the same order as the input ESD signals will be used.

Table 119— Value of panningGridOrder

| bits | panningGridOrder |
| --- | --- |
| 0b000 | SAME ORDER AS INPUT HOA SIGNAL |
| 0b001 | 1ST ORDER GRID (4 NODES) |
| 0b010 | 2ND ORDER GRID (9 NODES) |
| 0b011 | 3RD ORDER GRID (16 NODES) |
| 0b100 | 4TH ORDER GRID (25 NODES) |
| 0b101-0b111 | RESERVED |

coSourceCount	This value is the number of object sources that are also sampled alongside the HOA sources.

coSourceId	This value is the unique object source identifier that is sampled within this HOA group.

hoaSourcesCount	This value is the number of HOA sources in this payload.

hoaSourceId	This value represents the unique identifier for the HOA source.

hoaSourcePositionX	This value represents the Cartesian spatial position of the HOA source along the x dimension in meters.

hoaSourcePositionY	This value represents the Cartesian spatial position of the HOA source along the y dimension in meters.

hoaSourcePositionZ	This value represents the Cartesian spatial position of the HOA source along the z dimension in meters.

hoaSourceOrientationYaw	This value represents the Euler orientation of the HOA source in the Yaw axis in degrees.

hoaSourceOrientationPitch	This value represents the Euler orientation of the HOA source in the Pitch axis in degrees.

hoaSourceOrientationRoll	This value represents the Euler orientation of the HOA source in the Roll axis in degrees.

hoaSourceCoordSpace	This flag represents in which coordinate system the HOA source refers to.

Table 120 — Value of hoaSourceCoordSpace

| bits | hoaSourceCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

hoaSourceActive	This flag represents if the HOA source will be rendered.

hoaSourceGainDb	This value is the Gain (dB) that adjusts the level of this HOA source. The range is between -119.0 to 36.0. hoaSourceRefDistance	This value is the reference distance (m) of the HOA source. The reference distance is the point in which the source becomes 0dB, which is useful for the distance attenuation law.

hoaSourceHasReverbGain	This flag indicates that this HOA source has reverbGainDb parameter set.

hoaSourceReverbGainDb	This value is the reverb gain (dB) of this HOA source. It ranges between -119.0 to +12.0. Default value is 0 (dB)

hoaSourceSignalId	This value represents the unique audio stream identifier that this object source requires for its audio.

hoaSourceHasExtent	This flag indicates if the HOA source is spatially extended.

hoaSourceExtentId	This value is the unique geometry identifier that represents the extent of this HOA source.

hoaSourceExtentTransform	This flag turns on/off the external rendering of an interior source or the internal rendering of an exterior source.

hoaSourceRepresentation	This flag indicates the representation of this HOA source, either interior or exterior.

Table 121 — Value of hoaSourceRepresentation

| bits | hoaSourceRepresentation |
| --- | --- |
| 0b0 | INTERIOR |
| 0b1 | EXTERIOR |

hoaSourceHasGroup	This flag indicates if this HOA source belongs to a HOA group.

hoaSourceGroupId	This value is the unique HOA group identifier in which this HOA source belongs to.

hoaSourceNoReverb	This flag indicates that no reverberation shall be applied to this HOA source.

hoaSourceNoDoppler	This flag indicates that no Doppler shifts shall be applied to this HOA source.

hoaSourceNoDistance	This flag indicates that no distance attenuation shall be applied to this HOA source.

hoaSourceNoDirectionalFocus	This flag indicates that directional focus processing shall not be applied to this HOA source.

hoaSourceNoCulling	This flag indicates that this HOA source shall not be removed by culling algorithms.

hoaSourceMode	This flag indicates the playback mode for this HOA source.

Table 122 — Value of hoaSourceMode

| bits | hoaSourceMode |
| --- | --- |
| 0b0 | CONTINUOUS |
| 0b1 | EVENT |

hoaSourcePlay	This flag indicates if playback is enabled for this HOA source.

hoaSourceVDLMethod	This indicates the VDL method combination to be used for the particular HOA source with the bits vdlMethod. In case that no argument is being provided, “spline” VDL method will be used as a default setting.

hoaSourceHasSpatialTransform	This flag indicates if this HOA source has a parent spatial transform.

hoaSourceHasAnchor	This flag indicates if this HOA source’s spatial transform is an anchor. If false, then the parent is a regular transform.

hoaSourceParentAnchorId	This value represents the unique identifier for the parent anchor.

hoaSourceParentTransformId	This value represents the unique identifier for the parent transform.

isHoaSourceStatic	This flag indicates if the HOA source is static or dynamic. If static, then the HOA source is stationary throughout the entire duration of the scene, whereas the position of the object source is updatable if it is dynamic.

channelSourcesCount	This value represents how many channel sources are in this payload.

loudspeakerCount	This value represents how many loudspeakers are in this channel source.

loudspeakerId	This value represents the unique identifier for this loudspeaker.

loudspeakerPositionX	This value represents the Cartesian spatial position of the loudspeaker along the x dimension in meters.

loudspeakerPositionY	This value represents the Cartesian spatial position of the loudspeaker along the y dimension in meters.

loudspeakerPositionZ	This value represents the Cartesian spatial position of the loudspeaker along the z dimension in meters.

loudspeakerOrientationYaw	This value represents the Euler orientation of the loudspeaker in the Yaw axis in degrees.

loudspeakerOrientationPitch	This value represents the Euler orientation of the loudspeaker in the Pitch axis in degrees.

loudspeakerOrientationRoll	This value represents the Euler orientation of the loudspeaker in the Roll axis in degrees.

loudspeakerChannel	This value represents the channel index in the associated audio stream.

loudspeakerHasDirectivity	This flag indicates if the loudspeaker has directivity data attributed to it.

loudspeakerDirectivityId	This value is the unique directivity identifier for that this loudspeaker uses for its directivity data.

loudspeakerDirectiveness	This value is the directiveness coefficient for directivity, indicating its strength. The range is between 0.0 and 20.0. To dequantize it to a floating point value, use the following equation:

| floatVal=20*loudspeakerDirectiveness2noOfBits | (6) |
| --- | --- |

channelSourceId	This value represents the unique identifier for the channel source.

channelSourcePositionX	This value represents the Cartesian spatial position of the channel source along the x dimension in meters.

channelSourcePositionY	This value represents the Cartesian spatial position of the channel source along the y dimension in meters.

channelSourcePositionZ	This value represents the Cartesian spatial position of the channel source along the z dimension in meters.

channelSourceOrientationYaw	This value represents the Euler orientation of the channel source in the Yaw axis in degrees.

channelSourceOrientationPitch	This value represents the Euler orientation of the channel source in the Pitch axis in degrees.

channelSourceOrientationRoll	This value represents the Euler orientation of the channel source in the Roll axis in degrees.

channelSourceCoordSpace	This flag represents in which coordinate system the channel source refers to.

Table 123 — Value of channelSourceCoordSpace

| bits | channelSourceCoordSpace |
| --- | --- |
| 0b0 | RELATIVE |
| 0b1 | USER |

channelSourceInputLayout	This integer represents the original CICP loudspeaker layout.

channelSourceActive	This flag represents if the channel source will be rendered

channelSourceGainDb	This integer represents a floating point value for the Gain (dB) that adjusts the level of this channel source. The range is between -119.0 to 36.0

channelSourceRefDistance	This value is the reference distance (m) of the channel source. The reference distance is the point in which the source becomes 0dB, which is useful for the distance attenuation law.

channelSourceSignalId	This value represents the unique audio stream identifier that this channel source requires for its audio.

channelSourceHasReverbGain	This flag indicates that this channel source has reverbGainDb parameter set.

channelSourceReverbGainDb   	This value is the reverb gain (dB) of this channel source. It ranges between -119.0 to +12.0. Default value is 0 (dB)

channelSourceNoReverb	This flag indicates that no reverberation shall be applied to this channel source.

channelSourceNoDoppler	This flag indicates that no Doppler shifts shall be applied to this channel source.

channelSourceNoDistance	This flag indicates that no distance attenuation shall be applied to this channel source.

channelSourceNoCulling	This flag indicates that this channel source shall not be removed by culling algorithms.

channelSourceNoDirectionalFocus	This flag indicates that directional focus processing shall not be applied to this channel source.

channelSourceMode	This flag indicates the playback mode for this channel source.

Table 124 — Value of channelSourceMode

| bits | channelSourceMode |
| --- | --- |
| 0b0 | CONTINUOUS |
| 0b1 | EVENT |

channelSourcePlay	This flag indicates if playback is enabled for this channel source.

channelSourceVDLMethod	This indicates the VDL method combination to be used for the particular channel source with the bits vdlMethod. In case that no argument is being provided, “spline” VDL method will be used as a default setting.

channelSourceHasSpatialTransform	This flag indicates if this channel source has a parent spatial transform.

channelSourceHasAnchor	This flag indicates if this channel source’s spatial transform is an anchor. If false, then the parent is a regular transform.

channelSourceParentAnchorId	This value represents the unique identifier for the parent anchor.

channelSourceParentTransformId	This value represents the unique identifier for the parent transform.

isChannelSourceStatic	This flag indicates if the channel source is static or dynamic. If static, then the channel source is stationary throughout the entire duration of the scene, whereas the position of the channel source is updatable if it is dynamic.

numberOfGeneratorObjects	Number of Airflow Generators in the scene.

id	ID of the generator.

generatorType	Index indicating which generator type is being specified.

Table 125 - Value of generatorType

| bits | ID | Meaning |
| --- | --- | --- |
| 0b000 | 0 | Global |
| 0b001 | 1 | Point |
| 0b010 | 2 | Cone |
| 0b011 | 3 | Planar |
| 0b010 – 0b111 | 4-7 | Reserved |

speed	Speed of the airflow in meters per second. This value represents a range between 0.0 and 100.0. To dequantize it to a floating point value, use the following equation:

| floatVal=100*speed2noOfBits - 1 | (7) |
| --- | --- |

orientation	Euler orientation of the airflow generator.

gainDb	Gain value of the generator source in dB.

active	Flag indicating if the generator is currently active.

position	Position of the generator object.

hasLimits	Flag indicating if the point-source generator has Azimuth and Elevation range limits.

aziRange	Azimuth range over which the generator emits airflow relative to its orientation. This value represents a range between 0.0 and 180.0 and is applied plus-minus zero. To dequantize it to a floating point value, use the following equation:

| floatVal=360*aziRange2noOfBits - 1 | (8) |
| --- | --- |

elevRange	Elevation range over which the generator emits airflow relative to its orientation. This value represents a range between 0.0 and 180.0 and is applied plus-minus zero. To dequantize it to a floating point value, use the following equation:

| floatVal=180*elevRange2noOfBits - 1 | (9) |
| --- | --- |

baseRadius	Radius of the base of the cone source, the base being the end of the cone at length distance from the position.

length	The length of the cone that describes the generator source emission pattern.

hasTopRadius	Flag indicating if the cone has a top radius.

topRadius	Radius of the top of the cone. Top radius must be less than or equal to base radius, and if equal describes a cylinder.

edgeRolloff	The edge fade distance, in percentage, relative to the cross-section of the generator.

width	Width of the planar source.

height	Height of the planar source.

hasdistanceAttenuationFactor	Flag indicating if the generator object has a distance attenuation factor.

distanceAttenuationFactor	Distance attenuation factor with which the power in the inverse square law is scaled. This value represents a range between -5.0 and 5.0. To dequantize it to a floating point value, use the following equation:

| floatVal=10*distanceAtteniationFactor2noOfBits - 1-5 | (7) |
| --- | --- |

audioSourceRescaleX	This value is the rescale factor applied to the Cartesian position of audio element along the x dimension in meters.

audioSourceRescaleY	This value is the rescale factor applied to the Cartesian position of audio element along the y dimension in meters.

audioSourceRescaleZ    	This value is the rescale factor applied to Cartesian position of audio element along the z dimension in meters.

autoRescale	This value enables the renderer to determine the rescale factor by comparing the audio element Acoustic Environment (source space) region defined in the LSDF (playback region/dimensions).

toleranceXMax	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s x dimension maximum.

toleranceXMin	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s x dimension minimum.

toleranceYMax	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s y dimension maximum.

toleranceYMin	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s y dimension minimum.

toleranceZMax	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s z dimension maximum.

toleranceZMin	This value represents the percentage of source region that remains outside of the listener region with respect to the source region’s z dimension minimum.

#### Airflow payload data structure

numberOfAirflowSources	Number of airflow sources in the scene

hasCustomProfile	Flag indicating of the source uses a custom set of frequency profiles.

profileType	Index as to which type of profile is specified.

Table 126 - Value of profileType

| bits | ID | Meaning |
| --- | --- | --- |
| 0b00 | 0 | Frequency-Gain pairs |
| 0b01 | 1 | Filters specified as centre frequency and Q |
| 0b10 – 0b11 | 2-3 | Reserved |

numSpeedProfiles	Number of speed-dependent profiles that are specified.

numAziProfiles	Number of azimuth-dependent profiles that are specified.

numElevProfiles	Number of elevation-dependent profiles that are specified.

refSpeed	Reference speed for which the profile is valid in meters per second. This value represents a range between 0.0 and 100.0. To dequantize it to a floating point value, use the following equation:

| floatVal=100*refSpeed2noOfBits - 1 | (10) |
| --- | --- |

speedProfile	Frequency-dependent filter profile for a given refSpeed.

refAzimuth	Reference azimuth for which the profile is valid in degrees. This value represents a range between 0.0 and 360.0. To dequantize it to a floating point value, use the following equation:

| floatVal=360*refAzimuth2noOfBits - 1 | (11) |
| --- | --- |

azimuthProfile	Frequency-dependent filter profile for a given refAzimuth.

refElevation	Reference elevation for which the profile is valid in degrees. This value represents a range between 0.0 and 360.0. To dequantize it to a floating point value, use the following equation:

| floatVal=360*refElevation2noOfBits - 1 | (12) |
| --- | --- |

elevationProfile	Frequency-dependent filter profile for a given refelevation.

speedFilterGain	Gain of the filter at the centre frequency given.

speedFilterQ	Q of the filter, where Q is defined as the filter centre frequency divided by the bandwidth of the filter at -3dB below speedFilterGain.

azimuthFilterGain	Gain of the filter at the centre frequency given, for the left and right ears.

azimuthFilterQ	Q of the filter, where Q is defined as the filter centre frequency divided by the bandwidth of the filter at -3dB below azimuthFilterGain. For the left and right ears.

elevationFilterGain	Gain of the filter at the centre frequency given.

elevationFilterQ	Q of the filter, where Q is defined as the filter centre frequency divided by the bandwidth of the filter at -3dB below elevationFilterGain.

profile	Integer index into table of default frequency profiles.

generatorID	ID of the generator object associated with the airflow source.

activeInDefaultRegion	Flag indicating if the source should be active when the listener is inside the default acoustic region.

hasValidRegions	Flag indicating if the source contains a list of acoustic regions where it should be active.

numRegions	Number of valid Regions for the source.

activeRegionID	ID of the region where the source should be active.

#### Granular payload data structure

numberOfGranularDatabases	This value represents the number of granular databases in the payload.

granularID	This value is the identifier of the granular database. This is referenced by GranularSoundId in the in granularSounds structure and provides the mapping from a granular sound to a specific granular database.

GranularAudioEncodingMode	This value indicates how the audio samples of grains are provided:

0: Uncompressed PCM

1: Using refences to a separate PCM signal.

Values 2-9 are reserved for ISO use. Values 10-15 are reserved for use outside of ISO scope.

DescriptorCoordinateDimensions	This value represents the dimension of the descriptor coordinate space.

numberOfChannels	This value represents the number of channels of the granular database.

numberOfGrains	This value represents the number of grains in the granular database.

GrainLen	This value represents the length of a certain audio grain.

coordScaling	This vector represents the scaling parameter for all descriptor coordinate dimensions.  Scaling of each dimension of the descriptor space. Increasing the scaling of a dimension makes the grain search in that dimension more precise. It is in a range between 0.0 and 100.0 To dequantize it to a floating point, use the following equation:

| floatVal=100 coordScaling(2noOfBits-1) | (13) |
| --- | --- |

smoothingTargetCoordinates	This vector represents smoothing parameters used to smooth target coordinate values obtained from the user input.  It is in a range between 0.0 and 1.0. To dequantize it to a floating point, use the following equation:

| floatVal= smoothingTargetCoordinates(2noOfBits-1) | (14) |
| --- | --- |

searchRadius	This value represents the radius of search while searching for grains via kDTree search.  This radius is used in the scaled descriptor space.  It is in a range between 0.0 and 1.0. To dequantize it to a floating point, use the following equation:

| floatVal= searchRadius(2noOfBits-1) | (15) |
| --- | --- |

grainHistoryLength	This value represents length of the history of used grains.

numMixRule	This value represents the number of mixing rule coefficients available in the database.

grainMixRule	This value represents the mixing rule coefficient that is in a range between 0.0 and 1.0.  To dequantize it to a floating point, use the following equation:

| floatVal= grainMixRule(2noOfBits-1) | (16) |
| --- | --- |

grainMixingRuleCoordinates	This vector of values is the list of descriptor coordinates at which a grainMixRule is specified. Each value is in a range between 0.0 and 1.0. To dequantize it to a floating point,  use the following equation:

| floatVal[i]= grainMixingRuleCoordinates[i](2noOfBits-1) | (17) |
| --- | --- |

fastGrainSwitchThreshold	This value represents the fast grain switching threshold distance in the descriptor space. It is in a range between 0.0 and 2.0.  To dequantize it to a floating point, use the following equation:

| floatVal= fastGrainSwitchThreshold(2noOfBits-1) | (18) |
| --- | --- |

probabilityExponentPosition	This value represents the exponent used to calculate the probability value associated with the position of the grain in the descriptor space for random selection.  It is in a range between 0.0 and 10.0. To dequantize it to a floating point, use the following equation:

| floatVal= 10probabilityExponentPosition(2noOfBits-1) | (19) |
| --- | --- |

probabilityWeightPosition	This value represents the weight used to calculate the probability value associated with the descriptor position evaluation in the random selection.  It is in a range between 0.0 and 100.0. To dequantize it to a floating point, use the following equation:

| floatVal= 100probabilityWeightPosition(2noOfBits-1) | (20) |
| --- | --- |

probabilityExponentHistory	This value represents the exponent used to calculate the probability value associated with the history of the grains rendered for random selection.  It is in a range between 0.0 and 10.0. To dequantize it to a floating point, use the following equation:

| floatVal=10 probabilityExponentHistory(2noOfBits-1) | (21) |
| --- | --- |

probabilityWeightHistory	This value represents the weight used to calculate the probability value associated with the history of the grains rendered for random selection.  It is in a range between 0.0 and 100.0. To dequantize it to a floating point, use the following equation:

| floatVal= 100probabilityWeightHistory(2noOfBits-1) | (22) |
| --- | --- |

descrTrendAvailable	This value indicates whether descriptor coordinate trend is available as a parameter in the granular database.

probabilityExponentTrend	This value represents the exponent used to calculate the probability value associated with the trend of the position of the grain in the descriptor space for random selection of grains.  It is in a range between 0.0 and 10.0. To dequantize it to a floating point, use the following equation:

| floatVal=10 probabilityExponentTrend2noOfBits | (23) |
| --- | --- |

probabilityWeightTrend	This value represents the weight used to calculate the probability value associated with the descriptor position evaluation in the random selection.  It is in a range between 0.0 and 100.0. To dequantize it to a floating point, use the following equation:

| floatVal= 100probabilityWeightTrend(2noOfBits-1) | (24) |
| --- | --- |

descrTrendMethod	This value indicates the index of the method chosen to evaluate the descriptor trend coordinate values for random selection.

originalRecordingPositionAvailable	This value indicates whether original recording position is available as a parameter in the granular database.

probabilityExponentOriginalRecordingPosition	This value represents the exponent used to calculate the probability value associated with the index or position of the grain in the original recording  for random selection.  It is in a range between 0.0 and 10.0. To dequantize it to a floating point, use the following equation:

| floatVal=10 probabilityExponentOriginalRecordingPosition2noOfBits | (25) |
| --- | --- |

probabilityWeightOriginalRecordingPosition	This value represents the weight used to calculate the probability value associated with the index or position of the grain in the original recording  for random selection.  It is in a range between 0.0 and 100.0. To dequantize it to a floating point, use the following equation:

| floatVal= 100probabilityWeightOriginalRecordingPosition(2noOfBits-1) | (26) |
| --- | --- |

probabilityGroupsAvailable	This value indicates whether probability groups are available in the granular database.

perGrainProbabilityAvailable	This value indicates whether individual or per grain probabilities are available in the granular database.

numProbGroups	This value represents the number of probability groups in the granular database.

probabilityGroupVal	This value specifies the probability weight of a probability group. The value is in a range between 0.0 and 1000.0.  To dequantize it to a floating point, use the following equation:

| floatVal= 1000probabilityGroupVal(2noOfBits-1) | (27) |
| --- | --- |

randomSelection	This value indicates whether random selection is to be used for granular synthesis. If 0, then the grain with highest probability weight is chosen.

useInterpolation	This value indicates whether interpolation is to be used for granular synthesis.

grainInterpolationDistThreshold	This value represents the distance threshold for interpolation.  It is in a range between 0.0 and 1.0.  To dequantize it to a floating point, use the following equation:

| floatVal= grainInterpolationDistThreshold(2noOfBits-1) | (28) |
| --- | --- |

useInterpolationResampling	This value indicates whether resampling operation is to be performed for granular interpolation.

pitchDimension	This vector of values indicate the descriptor coordinate dimension that corresponds to pitch. This is used to calculate length of the interpolated grain.

InterpolationMethod	This value represents the index of interpolation method to be chosen.

grainOverlapAmountAvailable	This value indicates whether the parameter of overlap amount or percentage used for overlap-add is available.

grainOverlapAmount	This value represents the parameter of overlap amount or percentage used for overlap-add operation in granular synthesis. It is in a range between 0.0 and 1.0.  To dequantize it to a floating point, use the following equation:

| floatVal= grainOverlapAmount(2noOfBits-1) | (29) |
| --- | --- |

DescrCoord	This vector of values represents the grain descriptor coordinates for a certain grain. Each value is in a range between 0.0 and 1.0.  To dequantize it to a floating point, use the following equation:

| floatVal[i]= DescrCoord[i](2noOfBits-1) | (30) |
| --- | --- |

GrainLen	This value represents the length of the grain.

DescrCoordTrend	This vector of values represents the trend of the grain descriptor coordinates for a certain grain.  Each value is in a range between 0.0 and 1.0.  To dequantize it to a floating point, use the following equation:

| floatVal[i]= DescrCoordTrend[i](2noOfBits-1) | (31) |
| --- | --- |

origRecordingPos	This value represents the position of the grain in the original recording. To dequantize it to a floating point, use the following equation:

| floatVal= origRecordingPos(2noOfBits-1) | (32) |
| --- | --- |

grainProbabilityGroup	This value represents the probability group to which the grain belongs.

perGrainProbabilityValue	This value represents the per-grain probability value.  Each value is in a range between 0.0 and 1000.0.  To dequantize it to a floating point, use the following equation:

| floatVal= 1000perGrainProbabilityValue(2noOfBits-1) | (33) |
| --- | --- |

GrainSamples	This vector of values represents the samples of the grain. When  GranularAudioEncodingMode =0, each value is in a range between -1.0 and 1.0. To dequantize it to a floating point, use the following equation:

| floatVal= 2GrainSamples[i](2noOfBits-1)-1 | (34) |
| --- | --- |

grainSampleIndex	This value specifies at which sample index the grain starts within a PCM audio signal that is made available to the renderer using some separate channel when the encoding option is set to 1.

#### RasterMap payload data structure

numberOfVoxSubScenes	This element represents the number of voxel sub-scenes.

hasVoxSceneDiffractionRasterMap	Flag indicating the presence of rasterMapSubScene data.

numRasterMapSources	This element represents the number of sources for which RasterMap data is available (informational).

rasterMapSourceIdLen[s]	This element represents the number of elements of rasterMapSourceId[s].

rasterMapSourceId[s][i]	This element represents the i-th source ID which belongs to s-th RasterMap source.

numRasterMapLayers[s]	This element represents the number of available RasterMap layers.

rasterMap[s][l]	This element represents the l-th layer of the RasterMap for source s.

codebookVcX	Generic Codebook for the x-component of voxel coordinates.

codebookVcY	Generic Codebook for the y-component of voxel coordinates.

codebookWpX	Generic Codebook for the x-component of waypoint voxel coordinates.

codebookWpY	Generic Codebook for the y-component of waypoint voxel coordinates.

numRasterMapVoxels	This element represents the total number of RasterMap data entries.

hasVoxelCoordY	This flag indicates if the y-component of a voxel coordinate is transmitted (true) or predicted (vcY is increased if false).

vcY		This element represents the y-component of the current voxel coordinate (zero indexed value).

hasVoxelCoordX	This flag indicates if the x-component of a voxel coordinate is transmitted (true) or predicted (vcX is increased if false).

vcX		This element represents the x-component of the current voxel coordinate (zero indexed value).

this.voxelCoordX[v]	This data structure element of rasterMap[s][l] represents the x-component of the v-th voxel coordinate having associated voxel data, i.e. a 2D waypoint (zero indexed value).

this.voxelCoordY[v]	This data structure element of rasterMap[s][l] represents the y-component of the v-th voxel coordinate having associated voxel data, i.e. a 2D waypoint (zero indexed value).

this.refMode[v]	This element represents the transmission mode for the v-th RasterMap waypoint [wpX, wpY, wpL] as follows where rasterMapMatrix[vcX, vcY, vcL] = [wpX, wpY, wpL] stores the previously transmitted RasterMap data:

If 0: The waypoint coordinate [wpX, wpY, wpL] is transmitted explicitly.

If 1: [wpX, wpY, wpL] = rasterMapMatrix[vcX-1, vcY, l]

If 2: [wpX, wpY, wpL] = rasterMapMatrix[vcX, vcY-1, l]

If 3: [wpX, wpY, wpL] = rasterMapMatrix[vcX, vcY, l-1]

mapChange	This flag indicates a reduction of the layer to the next lower layer.

this.waypointX[v]	This element represents the x-component of the v-th waypoint associated to a voxel coordinate. If not transmitted explicitly, the x-component of the waypoint [wpX, wpY, wpL] is transmitted according to the transmission mode specified above (zero indexed value).

this.waypointY[v]	This element represents the y-component of the v-th waypoint associated to a voxel coordinate. If not transmitted explicitly, the y-component of the waypoint [wpX, wpY, wpL] is transmitted according to the transmission mode specified above (zero indexed value).

this.waypointL[v]	This element represents the l-component of the v-th waypoint associated to a voxel coordinate. If not transmitted explicitly, the l-component of the waypoint [wpX, wpY, wpL] is transmitted according to the transmission mode specified above (zero indexed value).

### Generic Codebook

this.flagFixedLength	This flag indicates if fixed length encoding (true) or variable length encoding (false) is used.

this.flagOffset	This flag indicates if an offset shall be added to the transmitted values.

wordSizeOffset	Word size for offset value.

this.offset	Offset value.

this.wordSize	Word size for codebook entries.

numCodes	Number of codebook entries.

this.codeList[n]	This code word corresponds to the symbol this.symbolList[n]. This code word and symbol pair yields the n-th entry of the transmitted code book.

this.symbolList[n]	This symbol corresponds to the code word this.codeList[n]. This code word and symbol pair yields the n-th entry of the transmitted code book.

rawList[n]	This intermediate value contains a symbol without an offset.

isLeaf	This flag indicates a leaf within the transmitted code tree.

## Renderer framework

### Control workflow

#### General

The Control Workflow consists of the components that are required to handle a 6DoF scene. It provides interfaces for receiving a bitstream and an LSDF, as well as an API for local updates of metadata, clock synchronization and local audio stream inputs. For an overview of the Renderer components, see Figure 3.

#### Scene controller

The Scene Controller is the central component for maintaining a 6DoF scene representation including all audio elements and geometries. It holds the Scene State (6.4.1.2.1) and handles all internal and external modifications to it through updates (6.4.1.2.3), which can be received via the bitstream or a local update interface. If the scene is an AR scene indicated by the bitstream element sceneType 6.2.11, the Scene Controller additionally reads the LSDF, describing the acoustic properties and anchor positions of the listener space, which are integrated into the Scene State.

##### Scene state

The Scene State always reflects the current state of all entities in the scene, incorporating metadata from multiple sources, including the bitstream which can contain timed and conditional updates, an LSDF and local updates. The entities are represented as Scene Objects (SOs) (6.4.1.2.2). Only the Scene Controller can modify the Scene State, whereas all other components in the Renderer have read-only access to the Scene State and all SOs.

Components may also subscribe to changes in the Scene State and of individual SOs, so that a callback is called when attributes are modified. For this, the component can implement the SceneStateObserver and SceneObjectObserver interfaces. The callback of a SceneStateObserver is called when an SO is added or removed from the Scene State.

class SceneStateObserver {

public:

virtual ~SceneStateObserver() {};

virtual void sceneStateAttached(const SceneState* sceneState) = 0;

virtual void sceneStateDetached() = 0;

virtual void sceneObjectAdded(SceneObject* object) = 0;

virtual void sceneObjectRemoved(SceneObject* object) = 0;

};

The SceneObjectObserver callback notifies about any modifications of individual SOs.

class SceneObjectObserver {

public:

virtual ~SceneObjectObserver() {};

enum class Property {

Position,

Activity,

Directivity,

Gain,

DistanceModel,

AudioStream,

Extent,

ReferenceDistance,

Staticity,

VDLMethod,

ReverbGain,

NoReverb,

Loudness,

Play,

Stop

};

virtual void objectChanged(SceneObject* obj, Property modification) = 0;

};

##### Scene objects

###### General

Any audio element, geometry, transform and the listener in the Scene is represented as a Scene Object (SO). Every SO has at least the attributes specified in Table 127.

Table 127 — Scene Objects

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |

###### Identifiers

The id attribute is used to reference the SO internally. The Scene keeps a dictionary of human-readable bitstream identifiers to associate them with the corresponding SO, so that the bitstream identifier can be used for local updates.

###### Positions

The position of an SO consists of its location in cartesian coordinates, its orientation in Tait-Bryan angles, as well as a coordinate space reference cSpace, which specifies whether the position is given in global coordinates or relative to a parent coordinate system.

enum class CSpace {

WCS, // World Coordinate System

RCS, // Parent-relative Coordinate System

LCS, // Listener-relative Coordinate System

};

struct Position {

Vector3 location;

Euler orientation;

CSpace cSpace;

};

The representation of location and orientation follows the OpenGL coordinate format as shown in Figure 5 and Figure 6. Any SO can be the parent of another SO, so that relative coordinates are relative to the position of the parent SO. The parent relationship is specified by the parent attribute which can be empty or contain a reference to another SO.

Figure 5 — OpenGL coordinate system (right-handed, +y points up)

| - yaw rotation | - pitch rotation | (c) roll rotation |
| --- | --- | --- |

Figure 6 — Yaw, pitch and roll rotations

Written in rotation matrices Rx, Ry, Rz for the three global coordinate axes x, y, z, the total 
yaw-pitch-roll rotation matrix yields: Ryprφ,θ,ρ=Ry(φ) Rx(θ) Rz(ρ).

An object with yaw-pitch-roll angles (0°, 0°, 0°) has the following orientation in the global cartesian coordinate axes:

- Its front direction points into -z

- Its up direction points to +y

- Its right direction points to +x

###### Listener

The Listener SO cannot have a parent and isStatic is always false. The bitstream identifier “listener” is reserved for this SO and may not be used by other entities in the scene. This way, the listener properties like its position can be updated using the local update API (6.4.1.2.3) with the identifier “listener”.

Associated to the listener SO is an audio element SO of type ListenerVoice that represents the listener’s own voice from a locally captured signal (i.e. microphone input) within the scene unless enableLocallyCapturedAudio is set to false.

Guidance on the usage of Locally Captured Audio, especially the user’s own voice is given in B.6.

###### Audio elements

Audio elements are represented as SOs with the additional attributes given in Table 128 and Table 130.

Table 128 — Object Source

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier oft he SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| gain | float | Frequency-independent gain in dB |  |
| itemReverbGain | float | Frequency-independent gain as factor | 1 |
| noReverb | bool | Turns the reverb on or off per source | false |
| directivity | Directivity* | Optional reference to a Directivity representation | None |
| directiveness | float | Parameter to control the frequency-independent intensity of the Directivity | 1 |
| extent | Geometry* | Optional reference to a Geometry that represents the extent of the Object Source | None |
| refDistance | float | Reference distance for the distance attenuation model | 1 |
| audioStream | Audio stream reference | ID of an Audio Stream that can be requested from the Stream Manager (cf. 6.4.1.3) | - |
| vdlMethod | int | Parameter to indicate the Variable Delay Line method used for the primary render item and its reflections | 0 |
| loudness | float | The current loudness of the source (linear), if any | - |

Channel source

A Channel Source is represented as a collection of Object Sources located at the respective loudspeaker positions. The loudspeaker layout of the Channel Source is specified by a layout ID according to ISO/IEC 23091-3:2018 CICP-Audio and a radius that specifies the distance between center point and each loudspeaker.

The Object Sources have their parent attribute set to the SO that represents the Channel Source and their coordinates are set relative to the parent.

Listener Voice

The Listener Voice is a specialized Object Source that represents the listener's voice within the scene. It is located at a fixed position relative to the listener SO’s location. The listener’s voice is obtained from a locally captured stream, e.g. a microphone signal, and therefore requires additional parameters to adapt the levels to the listener’s physical input and room properties. The Listener Voice SO attributes extend the Object Source SO by the parameters given in given in Table 129. If overrideListenerVoiceDirectivity is false the ListenerVoiceDefaultDirectity is obtained from the payload of type coverSet() specified as listenerVoiceDefaultDirectivityData in Annex A.18. The attributes erGainDb, itemReverbGain, directGainDb shall be obtained via a real-time adjustable user interface.

Table 129 — Listener Voice Source

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier oft he SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | 0, -0.05, 0 relative to the listener Position |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| gain | float | Frequency-independent gain in dB |  |
| itemReverbGain | float | Frequency-independent gain as factor | See Annex B.6 |
| noReverb | bool | Turns the reverb on or off per source | false |
| directivity | Directivity* | Optional reference to a Directivity representation | ListenerVoiceDefaultDirectivity |
| directiveness | float | Parameter to control the frequency-independent intensity of the Directivity | 1 |
| extent | Geometry* | Optional reference to a Geometry that represents the extent of the Object Source | None |
| refDistance | float | Reference distance for the distance attenuation model | 1 |
| audioStream | Audio stream reference | ID of an Audio Stream that can be requested from the Stream Manager (cf. 6.4.1.3) | - |
| vdlMethod | int | Parameter to indicate the Variable Delay Line method used for the primary render item and its reflections | 0 |
| erGainDb | float | Early Reflection gain in dB | See Annex B.6 |
| directGainDb | float | Direct sound gain in dB | See Annex B.6 |

Table 130 — HOA Source

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | Uint | Unique identifier oft he SO | - |
| Position | Position | The position (location and orientation) of the SO in global coordinates | - |
| Velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| Parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | Bool | Whether or not the SO is allowed to be moved in the Scene | false |
| Gain | float | Frequency-independent gain in dB |  |
| itemReverbGain | float | Frequency-independent gain as factor | 1 |
| noReverb | bool | Turns the reverb on or off per source | false |
| representation | Representation | Whether the HOA Source represents an Interiour or Exterior sound filed | - |
| Extent | Geometry* | Optional reference to a Geometry that represents the extent of the Object Source | None |
| doesExtentTransform | bool | Whether or not the HOA Source shall be handled with extent transform | false |
| transitionDistance | float | Transition distance range for the optional extent transform (only if doesExtentTransform == true) | 0 |
| refDistance | float | Reference distance for the distance attenuation model | 1 |
| Group | int | Optional group ID to add this HOA Source to a HOA Group (-1 for no group) | -1 |
| Is6DoF | bool | Whether or not this HOA Source shall be considered as a 6DoF source. If false, the HOA Source is always listener-relative with location = 0,0,0 | false |
| vdlMethod | int | Parameter to indicate the Variable Delay Line method used for the primary render item and its reflections | 0 |
| loudness | float | The current loudness of the source (linear), if any | - |

###### Geometry

Geometry entities are represented as SOs with the attributes given in Table 131 - Table 134. A geometry must further implement a common interface to allow interoperability between any kind of geometry.

Class Geometry {

virtual bool isInside(const Vector3& point) const = 0;

virtual Vector3 closestPointTo(const Vector3& point) const = 0;

virtual Type getType() const = 0;

virtual float getArea() const = 0;

virtual float getVolume() const = 0;

virtual const Mesh* getMesh() const = 0;

virtual float greatestDistance() const = 0;

};

isInside() – returns true when the given point in global coordinates is inside the geometry.

closestPointTo() – returns the closest point on the surface of the geometry in global coordinates, both if the query point is inside or outside of the geometry.

getType() – returns an identifier for the type of geometry (Box, Sphere, Cylinder or Mesh)

getArea() – returns the total surface area of the geometry in square meters.

getVolume() – returns the total volume of the geometry in cubic meters.

getMesh() – returns a reference to a Mesh instance that represents the same geometry. If the geometry is a Box, Sphere or Cylinder, the Mesh representation is a tesselation of the original geometry.

greatestDistance() – returns the longest distance between two points on the surface of the geometry in meters.

Table 131 — Box

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| volume | Vector3 | Extent of the box in x/y/z direction of its local coordinate system. The origin of the local coordinate system is the center of the box. | - |
| material | Material* | Optional reference to a Material representation that describes the acoustic properties of the geometry. Geometries without a Material are acoustically transparent. | None |

Table 132 — Sphere

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| Id | uint | Unique identifier of the SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| radius | float | Radius of the sphere. The origin of the local coordinate system is the centre of the sphere. | - |
| material | Material* | Optional reference to a Material representation that describes the acoustic properties of the geometry. Geometries without a Material are acoustically transparent. | None |

Table 133 — Cylinder

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| height | float | Height of the cylinder in y-direction of the local coordinate system. The origin of the local coordinate system is the middle of the cylinder. | - |
| xRadius | float | Radius of the cylinder in x-direction of the local coordinate system. | - |
| zRadius | float | Radius of the cylinder in z-direction of the local coordinate system. | - |
| isCircular | Bool | True if xRadius == zRadius | - |
| material | Material* | Optional reference to a Material representation that describes the acoustic properties of the geometry. Geometries without a Material are acoustically transparent. | None |

Table 134 — Mesh

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| velocity | Vector3 | The optional velocity of the SO | 0, 0, 0 |
| parent | SceneObject* | Reference to an optional parent SO | None |
| isStatic | bool | Whether or not the SO is allowed to be moved in the Scene | false |
| height | float | Height of the cylinder in y-direction of the local coordinate system. The origin of the local coordinate system is the middle of the cylinder. | - |
| vertices | List<Vector3> | Variable-length list of vertices (locations in local coordinate system) | - |
| triangles | List<Triangle> | List of triangles, defined by triplets of vertices. Different triangles may have different materials. | - |
| surfaces | List<Surface> | List of surfaces, | - |

Triangle

A triangle face of a mesh is characterized by three vertices v1, v2, and v3 that reference the vertices list of the parent mesh. Each triangle can have an individual material that defines its acoustic properties (see below).

struct Triangle {

unsigned int v1, v2, v3;

Mesh* parent;

std::shared_ptr<Material> material;

unsigned int surfaceId;

};

Surface

A surface is a collection of connected triangles that lie in the same plane, forming a single reflecting surface. Surfaces must be calculated from the transmitted triangles in the renderer.

struct Plane {

Vector3 normalVector;

float originDist;

};

struct Surface {

unsigned int surfaceId;

std::vector<unsigned int> triangles;

Plane plane;

};

Material

The material specifies the acoustic properties of a box, sphere, cylinder or individual triangles of a mesh.

Struct Material {

std::vector<float> reflection;

std::vector<float> scattering;

std::vector<float> transmission;

std::vector<float> coupling;

};

The frequency-dependent material properties are given in EQ bands as specified in .

- Specular reflected energy (reflection) is reflected back in a distinct outgoing direction

- Diffuse reflected energy (scattering) is diffusely scattered back from the material

- Transmitted energy (transmission) passes through the material without changing the sound’s direction

- Coupled energy (coupling) excites vibrations in the structure and is reemitted by the entire structure

Each value is a number in the range 0..1. The sum of these four values, must not exceed one. The remaining energy is dissipated energy that is absorbed by the material. Materials are assumed to be isotropic (i.e. rotation-invariant).

###### Airflow Generator

Airflow generators are SOs that represent an object in the virtual space that emits air. The four classes of Global, Point, Cone, and Planar source (Table 135 - Table 138) inherit from a base AirflowGenerator class.

class AirflowGeneratorBase{

virtual Vector3 getAirspeed(const Vector3& position) const =0;

virtual Type getType() const =0;

float getDistanceAttenuationFactor() const  = 1.0f;

}

Table 135 - Global

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| name | string | Unique string identifier of the SO |  |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |

Table 136 - Point

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| name | string | Unique string identifier of the SO |  |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| hasLimits | Boolean | Flag to indicate of either aziRange or elevRange is different from default. | False |
| aziRange | float | Azimuthal range of the point generator. | 360 |
| elevRange | float | Elevation range of the point generator. | 360 |

Table 137 - Cone

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| name | string | Unique string identifier of the SO |  |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| baseRadius | float | Radius of the base of the cone. | 1.0 |
| length | float | Length of the cone. | 1.0 |
| edgeRolloff | float | Percentage of the cross-section over which to fade out the speed parameter. | 50.0 |
| hasTopRadius | Boolean | Flag to indicate if topRadius is set. | false |
| topRadius | float | Radius of the top of the cone. | 0.0f |

Table 138 - Planar

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| id | uint | Unique identifier of the SO | - |
| name | string | Unique string identifier of the SO |  |
| position | Position | The position (location and orientation) of the SO in global coordinates | - |
| width | float | Width of the planar source. | 1.0 |
| height | float | Height of the planar source. | 1.0 |
| edgeRolloff | float | Percentage of the cross-section over which to fade out the speed parameter. | 50.0 |

###### Transform

A Transform is a SO that can be used as parent for audio elements and geometry. Transforms can be used in the bitstream to group other SOs into a separate relative coordinate system, and to represent ARAnchors from the LSDF.

##### Update

An Update is a collection of modifications to metadata of Scene Objects. The Scene Controller allows Updates in the Scene State through the following means:

- SceneUpdate Packets in the bitstream contain information about changes to individual entities in the scene (cf. 5.3.2.2). They allow immediate, pre-defined, location- or time-based, and interpolated updates.

  - Immediate updates: The Scene updates the Scene State as soon as the SceneUpdate packet is received from the Bitstream interface.

  - Pre-defined updates: A specified packet of modifications is transmitted and triggered locally by a given identifier.

  - Location- or time-based single updates: The Scene evaluates previously received location- and time-based criteria and updates the Scene State if the criteria require a metadata change.

  - Interpolated updates: the Scene evaluates previously received and started metadata trajectories and updates the Scene State accordingly.

- A Local Update API for other systems and components at the decoder (cf. 6.4.1.2.3.1)

Evaluation of location- and time-based criteria, as well as interpolation is done in a separate thread created by the Scene Controller. The thread must operate at a rate of at least 100 executions per second.

Temporal interpolations can be triggered by pre-defined, location-based or local updates when the update contains a duration and are always linear interpolations. Temporal interpolations are stopped when the scene is looped. Start value for a temporary interpolation is always the metadata property value at the time the update is triggered, not considering delay.

The behavior is undefined when a SO metadata property is changed with multiple types of updates simultaneously, e.g., when an Object Source is moving on a timed trajectory, its location cannot be changed by a local update.

###### Local update API

External components and subsystems of the decoder can construct local updates to change metadata of Scene Objects. Allowed changes through updates are indicated in the bitstream as L2 or L3 updates.

struct Modification {

std::string entityId;

std::string attributeName;

Variant targetValue;

bool teleport = false;

};

struct Update {

double timestamp = -1;

std::vector<Modification> modifications;

};

The timestamp is given in Scene time. If the current Scene time when the Update is received is larger than the Update timestamp, the Update is executed immediately. Furthermore, each Update contains a list of Modifications, that each consist of

- An entityId. The reserved identifier “listener” is used to update e.g. the Listener position through the same API as other entities,

- An attributeName,

- A targetValue, whose data type depends on the attribute, and

- A teleport flag to indicate non-interpolated modifications to the location of an entity, affecting e.g. the propagation delay.

#### Stream manager

The Stream Manager provides access to Audio Streams by an identifier that can be referenced in the bitstream or in external updates. The Audio Stream source is variable and can either be a local PCM stream or a decoded MPEG-H audio stream from the bitstream.

Audio Stream access is frame-based. Components in the Renderer can create a StreamAccessBuffer, that is associated with a certain Audio Stream and a block of samples is written into its memory buffer for each processed frame of the Audio Stream. Stream access may support seeking in the Audio Stream. The Stream Manager crossfades between Audio Streams if the accessed stream changes.

class StreamBuffer {

RealFrame& getFrame();

const RealFrame& getFrame() const;

};

class StreamAccessBuffer : public StreamBuffer

{

void setStream(const std::string& id, double t = 0.0);

void play();

void stop();

void seekTo(double t);

void setLoop(bool shouldLoop);

std::size_t getReadPosition() const;

inline const std::string& getStreamId() const;

};

Components may also create empty StreamBuffer instances, which have the same properties as a StreamAccessBuffer, but the contents must be managed by the owning component.

#### Clock

The Clock component allows to synchronize the Scene time to an external timekeeper. In the stand-alone case, an InternalClock implementation of the Clock interface uses the CPU wallclock (e.g. std::chrono::steady_clock) to determine the time in seconds that has passed since the Scene has started. The current time of InternalClock is synchronized with the audio thread by counting the number of samples that has been played out since the Scene started.

class Clock {

public:

virtual ~Clock() {};

virtual double getCurrentTime() = 0;

virtual void start() = 0;

virtual void stop() = 0;

virtual bool isRunning() = 0;

virtual void sync(double t) = 0;

};

#### Multithreading

The Scene Controller creates a separate thread to handle time- and location-based updates, as well as interpolations. The update routine in this thread shall be executed at a rate of at least 100 Hz.

Observing the Scene State and SOs is read-only. Observer callbacks are called from the thread where the Update was triggered. The observing component must ensure thread-safety of the callback.

The Clock::sync() routine must be thread-safe with regard to concurrent calls to getCurrentTime().

StreamBuffers and StreamAccessBuffers may only be accessed in the audio thread. The Stream Manager ensures StreamAccessBuffers to contain the correct samples at the beginning of each audio thread callback.

### Rendering workflow

#### General

The Rendering Workflow consist of the components that auralize the scene for a listener using information from the Control Workflow. The required processing of metadata and audio signals is implemented in a number of Renderer Stages (6.4.2.2), which are executed sequentially in the Renderer Pipeline (6.4.2.2) to contribute acoustic effects to the auralization. The metadata interface between Renderer stages consists of a Render List (6.4.2.3) which contains a variable number of RIs (6.4.2.4), each representing an acoustically relevant element in the scene, for example a point source or a specular reflection. The Renderer Pipeline is followed by the Spatializer (6.4.2.5) that produces a final output signal from the remaining and active RIs for a given playback method. Last, a limiter prevents clipping of the output signal.

#### Multithreaded renderer pipeline

The Renderer Pipeline organizes the sequential execution of Renderer stages. Each stage has an interface (6.4.2.2.1) that ensures interoperability between stages.

The Renderer Pipeline translates all audio elements in the Scene into a list of RIs, which is processed by the stages sequentially.

##### Renderer stages interface

The Renderer stage interface consists of 3 routines that are executed in different threads.

Figure 7 — Renderer stage interface

initializeStage() – The initialization routine is called when the Scene is loaded. It cannot be called when rendering is running. The interface provides references to the Stream Manager, as well as the Scene State, so that the stage can access the entities in the Scene that are not audio elements (i.e., Geometry). The stage uses this routine to prepare for rendering by allocating possibly required resources before rendering starts. The initialization routine is called from the main thread in which the Renderer Pipeline is initialized.

update() – The update routine is called in a dedicated update thread which is executed in regular intervals (80-160 Hz) to update the metadata of all processed signals. The metadata associated with each signal is given as RI (6.4.2.4) in a Render List (6.4.2.3). The update routine of one stage prepares the Render List for the succeeding stage, so that each stage builds up on the metadata provided by previous stages. The stage prepares a directive for the DSP processing (DSP directive in the following) that is sent to the audio processing thread without blocking it. The exchange of DSP directives is synchronized across all stages and the next DSP directive is used, when all stages have finished their update routine.  The DSP directive contains all information that is required by the audio processing routine, including references to the input and output signal buffers, as well as the processing configuration, like filter coefficients or algorithm parameters. The update routine guarantees that the signals associated with the RIs in the output Render List are correctly processed in the respective DSP directive. A stage that only modifies metadata uses empty DSP directives.

processBlock() – The audio processing routine is called by the audio callback thread of the host system when new samples are requested by the sound playback hardware (e.g. a PC sound card). It uses the previously prepared DSP directive from the update thread to process the audio streams given in the DSP directive. Between successive calls to update(), the DSP directive is constant and as the update routine ensures consistency between the signals associated with the output RIs and the DSP processing, the audio processing routine of all stages are called in the pipeline order.

#### Render list

The Render List is the primary interface for a Renderer stage to receive metadata about the audio elements in the scene. It consists of a unique identifier id, a variable-length list of RIs (6.4.2.4), and a listener state (Table 139). The id is used for the synchronization of active DSP diagrams between stages. All stages can modify, add and remove RIs from the Render List. The listener state is read-only and prepared by the Renderer Pipeline controller. The attributes of the listener state is given in Table 139.

Table 139 — Listener state

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| position | Position | The position (location and orientation) of the listener in global coordinates (see Position type description) | - |
| trajectory | Trajectory | Optional constant-velocity trajectory to interpolate the location of the listener between successive calls to the update() routine | None |
| cs | Homogeneous transform matrix | 4x4 transformation matrix representing the transform from global coordinates into the listener coordinate system | - |
| teleport | bool | Whether or not a change in the listener position shall be handled as teleport | false |
| hasMoved | bool | Whether or not the listener position has changed since the last call to the update() routine | false |
| acEnvId | int | Identifier of the acoustic environment the listener is located in (special value -1 if the listener is outside of all acoustic environments in the scene) | -1 |

#### Render item

A RI is the representation of an acoustically active element in the scene. The RI can be primary, i.e., derived directly from an audio element in the scene, or secondary, i.e., derived from another RI (for example a reflection or diffraction path). The attributes of an RI are given in Table 140.

Table 140 — RI metadata fields

| Field name | Data type | Description | Default value |
| --- | --- | --- | --- |
| idx | const int | Unique identifier of the RI | - |
| status | ItemStatus | The status of the RI (see ItemStatus type description) | - |
| type | ItemType | The type of the RI (see ItemType type description) | - |
| subtype | ItemSubtype | The subtype of the RI (see ItemSubtype description) | - |
| changed | ItemProperty | Flags to mark changed properties of the RI (see ItemProperty type description) | - |
| aparams | AParam | Flags to mark special rendering instructions for the RI (see AParam type description) | - |
| acEnvId | int | Identifier of the acoustic environment this RI is located in (special value -1 if the RI is outside of all acoustic environments in the scene) | -1 |
| trajectory | Trajectory | Optional constant-velocity trajectory to interpolate the location of the RI between successive calls to the update() routine | None |
| teleport | bool | Whether or not this RI shall be handled as teleport | false |
| position | Position | The position (location and orientation) of the RI in global coordinates (see Position type description) | - |
| apparentDistanceDelta | float | Compensation for the distance to the listener for synchronizing multiple RIs with different locations in terms of their propagation delay and distance attenuation | 0 |
| refDistance | float | Reference distance for the distance attenuation model | 1 |
| signal | StreamBuffer | Reference to a StreamBuffer instance (see Stream Manager section) | - |
| eq | List<float> | Frequency-dependent gain for the signal associated with this RI in globally defined bands (see Equalizer section) | N x 1 |
| gain | float | Global frequency-independent gain for the signal associated with this RI | 1 |
| itemReverbGain | float | Frequency-independent gain as factor | 1 |
| noReverb | bool | Turns the reverb on or off per source | false |
| directivity | Directivity | Optional reference to a Directivity representation for the RI (see Directivity type description) | None |
| directiveness | float | Parameter to control the frequency-independent intensity of the Directivity | 1 |
| extent | Geometry | Optional reference to a Geometry that describes the extent of the RI | None |
| extentLayout | ExtentLayout | Reference to the channel layout of a heterogeneous extended source | None |
| rayHits | List<RayHit> | Data structure to aid the processing of extended sources (see respective stages) | Empty |
| reflectionInfo | ReflectionInfo | Optional reference to a special struct that contains information about the reflection path this RI represents (see 6.6.6) | None |
| occlusionInfo | OcclusionInfo | Optional reference to a special struct that contains information about the occlusion of this RI (see 6.6.9) | None |
| channelPositions | List<Position> |  | None |
| hoaInfo | HoaInfo | Optional reference to a special struct that contains information about the HOA source this RI represents (see 6.6.23) | None |
| vdlMethod | int | Parameter to indicate the Variable Delay Line method used for the primary render item and its reflections | 0 |
| isSoundEffect | bool | Valueindicating if the RI is representative of a sound effect source. | false |

ItemStatus

enum data type

enum class ItemStatus
{
  New = 0,
  Active,
  FadeOutInactive,
  Inactive,
  FadeOutOrphaned,
  Orphaned
};

- New: This RI appears for the first time. Stages may have to allocate new resources and change their DSP processing to accommodate the new RI. It will become Active in the next update() call.

- Active: This RI shall be handled as active by all stages. If the status has been different in the previous update() call (e.g. because the RI changed from Inactive to Active), the changed flag is set accordingly.

- FadeOutInactive: This RI is being faded out and becomes Inactive afterwards. Stages shall continue to process it until the Inactive state is reached.

- Inactive: This RI shall be handled as inactive by all stages. It shall have no influence on the metadata or signal output of the stage.

- FadeOutOrphaned: This RI is being faded out and becomes orphaned afterwards. Stages shall continue to process it until the Orphaned state is reached.

- Orphaned: This RI reached its end of life and will be removed from the Render List in the next update() call. It shall have no influence on the metadata or signal output of the stage. Stages may permanently free any resources allocated for the processing of this RI.

ItemType

enum data type

enum class ItemType
{
  Primary = 0,
  HOA,
  Representative,
  Reflection,
  Diffraction,
  DirectionalReverb,
  Nondiegetic,
  Downmix,
  Pannable,
  Reverb,
  Airflow,
  ListenerVoice,
};

- Primary: A RI that is directly derived from a Scene Object

- HOA: An HOA RenderItem directly derived from a HOASource

- Representative: A representative RI derived from another RI (e.g. a cluster or an extended source representative)

- Reflection: A secondary RI derived as a specular reflection of another RI

- Diffraction: A secondary RI derived as a geometrically diffracted path of another RI

- DirectionalReverb: A representative RI for directional reverberation from a portal

- Nondiegetic: A RI that shall be spatialized, but is not part of the scene

- Downmix: RI that shall be mixed directly to the output without additional processing

- Pannable: RI that shall be panned into a virtual loudspeaker layout that is going to be spatialized

- Reverb: RI that is generated as a reverberation output for an acoustic environment

- Airflow: RI that is generated by the airflow simulation that shall be directly downmixed to the output without further processing.

- ListenerVoice: A RI that represents the listener’s voice as part of the scene

ItemSubtype

enum data type

enum class ItemSubtype
{
  None = 0,
  CoupledSource,
};

- None: No subtype was provided

- CoupledSource: A RI of an extent source that is a coupling source.

ItemProperty

enum data type

enum class ItemProperty
{
  None = 0,
  Status = 1,
  Position = 2,
  Aparams = 4,
  Directivity = 8,
  Extent = 16,
  Signal = 32,
  Gain = 64,
  Eq = 128,
  RefDist = 256,
  Orientation = 512,
  Location = 1024,
  ParentRI = 2048,
  Reverb = 4096,
  ReflectionInfo = 8192,
  RayHits = 16384,

VDLMethod = 32768,

ReverbGain = 65536,

NoReverb = 131072
};

Multiple item properties can be represented in a single variable using bitwise operators.

Example: adding two changed properties of a RI:

item.changed = ItemProperty::Status | ItemProperty::Position;

Example: checking if a property changed for an RI:

if (item.changed & ItemProperty::Position) { ... }

AParam

enum data type

enum class AParam
{
  None = 0,
  NoDoppler = 1,
  NoDistanceGain = 2,
  NoMediumAttenuation = 4,
  ForceNoDoppler = 8,

NoDirectionalFocus = 16,
};

#### Spatializer

The Spatializer receives the output of the Renderer pipeline and produces the signal that is, after passing the limiter (6.8), played back to the listener via the configured playback method. The binaural spatializer is described in (6.7.1).

class Spatializer {

public:

virtual ~Spatializer() {};

virtual void update(std::shared_ptr<const RenderList> newRenderList) = 0;

virtual void requestBlock(EAR::RealFrame& output) = 0;

virtual std::size_t getNbrOfChannels() const = 0;

virtual void cleanupDSPData(const std::vector<std::size_t>& toClear) = 0;

virtual void initializeSpatializer(float fs, std::size_t B) = 0;

};

update(), requestBlock(), intializeSpatializer() and cleanupDSPData() are called analogous to update(), processBlock(), intializeStage() and cleanupDSPData()of the stages. getNbrOfChannels() returns the number of output channels of the spatializer, so that the Renderer controller can allocate the required output buffers accordingly.

The adaptive loudspeaker rendering spatialization method is defined in (6.7.2).

#### Configuration

The Renderer Framework has local configuration parameters. The parameters and their values are given in Table 141.

Table 141 — Renderer framework local configuration parameters

| Name |  |  |  |  |  |  |  | Type |  |  |  | Value |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binaural_HRIRFilterLength_LoQ |  |  |  |  |  |  |  | int |  |  |  | 256 |  |  | The HRIR windowing length for binauralization of secondary RIs |  |  |
| PanningHRIRConvolution |  |  |  |  |  |  |  | int |  |  |  | 0 |  |  | 0: Regular HRIR convolution 1: HRIR convolution by panning Values larger than or equal to 2 are reserved. |  |  |
| DiffractionCullingGain |  |  |  |  |  |  |  | float |  |  |  | 0.1 |  |  | The gain of the diffraction source that are culled |  |  |
| DiffractionDistanceCullingThreshold |  |  |  |  |  |  |  | float |  |  |  | 64 |  |  | The distance threshold for which diffraction items are culled in meters |  |  |
| DiffractionFastSpeed |  |  |  |  |  |  |  | float |  |  |  | 5 |  |  | For a source with slower speed than DiffractionFastSpeed, edges farther away from a test point than DiffractionDistanceCullingThreshold will be no longer relevant for diffraction. For a source with faster speed, the distance between a source and the listener will be used as CullingThreshold |  |  |
| DiffractionFilterTuneParameter |  |  |  |  |  |  |  | float |  |  |  | 0.5 |  |  | A factor for diffraction filter slope control whose range varies in between 0 to 1 and 1 applies no attenuation effect by bending mode of propagation |  |  |
| DiffractionGain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for diffraction items |  |  |
| DiffractionItemNumber |  |  |  |  |  |  |  | int |  |  |  | 10 |  |  | The maximum number of diffraction-type render items per primary source |  |  |
| DiffractionLinearPanning |  |  |  |  |  |  |  | bool |  |  |  | false |  |  | Obsolete |  |  |
| DiffractionPathSelection |  |  |  |  |  |  |  | bool |  |  |  | false |  |  | Obsolete |  |  |
| DiffractionRegularizationAngle |  |  |  |  |  |  |  | float |  |  |  | 5 |  |  | Regularization coefficient in degree for weighting diffraction items according to their accumulated diffraction angle. |  |  |
| DiffractionSESSGain |  |  |  |  |  |  |  | float |  |  |  | 1.5 |  |  | The additional gain of diffraction-type render items in extended source case |  |  |
| DiffractionSourceExtentDistanceThreshold |  |  |  |  |  |  |  | float |  |  |  | 32 |  |  | For an extended source that is farther away from the listener than the given distance threshold, diffraction is not applied. |  |  |
| DiffractionSourceExtentGain |  |  |  |  |  |  |  | float |  |  |  | 0.25 |  |  | The initial gain of diffraction-type render items in extended source case |  |  |
| DiffractionSourceExtentGainNSample |  |  |  |  |  |  |  | int |  |  |  | 10 |  |  | The number of gains to generate smooth gain change in extended source case |  |  |
| DiffractionUpdateRate |  |  |  |  |  |  |  | int |  |  |  | 10 |  |  | The update rate of diffractions in Hz |  |  |
| Diffraction_GeometricErrorThreshold |  |  |  |  |  |  |  | float |  |  |  | 0.001 |  |  | The acceptable geometrical margin in meter |  |  |
| DirectSoundEnabled |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable or disable direct sound |  |  |
| DirectSoundGain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for direct sound |  |  |
| Dispersion_Diffraction |  |  |  |  |  |  |  | bool |  |  |  | false |  |  | Flag to enable dispersion filter for diffraction sources |  |  |
| Dispersion_Early_Reflections |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable dispersion filter for early reflections |  |  |
| Dispersion_Filter_Length |  |  |  |  |  |  |  | float |  |  |  | 0.015 |  |  | The length of the dispersion filter in seconds |  |  |
| Dispersion_Gain_dB |  |  |  |  |  |  |  | float |  |  |  | 0 |  |  | The gain level for the dispersion filter |  |  |
| Distance_EnableDoppler |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable propagation delay processing |  |  |
| Distance_MinDistance |  |  |  |  |  |  |  | float |  |  |  | 0.2 |  |  | The minimum distance for the distance attenuation in meters |  |  |
| Distance_SmoothingFactor |  |  |  |  |  |  |  | float |  |  |  | 0.5 |  |  | A factor for smoothing of object trajectories to counteract sudden movements (avoiding excessive Doppler effects) |  |  |
| Distance_TeleportThreshold |  |  |  |  |  |  |  | float |  |  |  | 25 |  |  | Threshold in m/s after which a movement of the listener is treated as a teleportation (avoiding excessive Doppler effects) |  |  |
| EarlyFloorAngleDeg |  |  |  |  |  |  |  | float |  |  |  | 15 |  |  | The threshold in which to determine if early reflections come from the floor |  |  |
| EarlyFloorGainDB |  |  |  |  |  |  |  | float |  |  |  | -6 |  |  | The gain level for reflections that have hit the floor |  |  |
| EarlyLimitOrder |  |  |  |  |  |  |  | int |  |  |  | 2 |  |  | The maximum possible order of early reflections |  |  |
| EarlyTuningGain |  |  |  |  |  |  |  | float |  |  |  | 0.4 |  |  | Overrides the early reflection tuning gain gtuning |  |  |
| EarlyMaxOrderNoVoxel |  |  |  |  |  |  |  | int |  |  |  | 1 |  |  |  |  |  |
| EarlyReflectionsEnabled |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable or disable early reflections |  |  |
| EarlySpeedLimit |  |  |  |  |  |  |  | float |  |  |  | 5 |  |  |  |  |  |
| EarlyUpdateRate |  |  |  |  |  |  |  | int |  |  |  | 4 |  |  | The update rate of early reflections in Hz |  |  |
| ExtentFilterSmoothing |  |  |  |  |  |  |  | float |  |  |  | 0 |  |  |  |  |  |
| ExtentHitCountMax |  |  |  |  |  |  |  | float |  |  |  | 0.6 |  |  |  |  |  |
| ExtentHitCountMin |  |  |  |  |  |  |  | float |  |  |  | 0.3 |  |  |  |  |  |
| Fade_Length |  |  |  |  |  |  |  | float |  |  |  | 0.05 |  |  | Fade in/out time for activated/deactivated audio elements in seconds |  |  |
| GeometricErrorThreshold |  |  |  |  |  |  |  | float |  |  |  | 0.001 |  |  |  |  |  |
| HOAEnabled |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable or disable HOA stage |  |  |
| HOAGain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain for HOA sources |  |  |
| Mphoa:gain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for MPHOA sources |  |  |
| OcclusionFastSpeed |  |  |  |  |  |  |  | float |  |  |  | 5 |  |  |  |  |  |
| Occlusion_BinaryMode |  |  |  |  |  |  |  | bool |  |  |  | false |  |  |  |  |  |
| Occlusion_RepeatDist |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  |  |  |  |
| OctaveBands |  |  |  |  |  |  |  | int |  |  |  | 9 |  |  |  |  |  |
| Panner:oldGainWeight |  |  |  |  |  |  |  | float |  |  |  | 0 |  |  | Apply gain smoothing in panner between frames if value > 0 by using oldGainWeight for old panning gains and (1-oldGainWeight) for new panning gains. |  |  |
| Panner:spreadAuxiliarySourceCount |  |  |  |  |  |  |  | int |  |  |  | 0 |  |  | Number of auxiliary sources for multiple direction amplitude panning (MDAP). |  |  |
| Panner:spreadGainFraction |  |  |  |  |  |  |  | float |  |  |  | 0 |  |  | Fraction of gain for auxiliary sources in MDAP. |  |  |
| Panner:spreadRadiusDeg |  |  |  |  |  |  |  | float |  |  |  | 0 |  |  | Radius for auxiliary sources in MDAP. |  |  |
| PortalSoundEnabled |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable or disable the portal stage |  |  |
| DirectionalReverb_Gain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for reverb extent render items |  |  |
| Portal_Enable_Coupling |  |  |  |  | bool |  |  |  |  |  | true |  |  | Flag to enable / disable rendering of coupling sources |  |  |  |
| RayHitSubdivCount |  |  |  |  |  |  |  | int |  |  |  | 20 |  |  |  |  |  |
| Reverb:Crossfade |  |  |  |  |  |  |  | bool |  |  |  | true |  |  | Flag to enable reverberation fade in outside the AE enclosure |  |  |
| Reverb:Crossfade_Fade_In_Distance |  |  |  |  |  |  |  | float |  |  |  | 1.00002 |  |  | Fade in distance in meters for reverberation fade in outside an AE |  |  |
| Reverb:DistanceGainDropDb |  |  |  |  |  |  |  | float |  |  |  | 1.50002 |  |  | Control for late reverberation distance gain attenuation. Distance doubling reduces late reverberation level by the given value in dB. |  |  |
| Reverb:Gain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for the late reverberation |  |  |
| Reverb:HeadTrackingEnabled |  |  |  |  |  |  |  | bool |  |  |  | false |  |  | Flag to enable or disable head tracking for reverberation |  |  |
| Reverb:LSDF_RDRToActualRDR |  |  |  |  |  |  |  | float |  |  |  | -8.0 |  |  | Offset in decibels to be added to the log base 10 of the RDR value obtained from an LSDF file |  |  |
| Reverb:MinimumDistance |  |  |  |  |  |  |  | float |  |  |  | 1.00024 |  |  | Minimum distance threshold applied to late reverberation distance gain attenuation. |  |  |
| Reverb:numDelayLines |  |  |  |  |  |  |  | int |  |  |  | 15 |  |  | Number of delay lines for reverb. If provided, this shall configure all FDN reverberators with this value and override any possible value carried in the bitstream revDelayLineCount. |  |  |
| Reverb:OverrideLSDF_RDR |  |  |  |  |  |  |  | bool |  |  |  | false |  |  | This flag indicates whether late reverberation shall derive RDR values using a statistical diffuse approximation based on the RT60 value instead of the RDR values for AR. |  |  |
| Reverb:PredelayFactor |  |  |  |  |  |  |  | float |  |  |  | 4.0 |  |  | A divisor applied to the predelay to adjust the reverberation onset for VR AEs |  |  |
| Reverb:PredelayFactorAR |  |  |  |  |  |  |  | float |  |  |  | 1.0 |  |  | A divisor applied to the predelay (diffuseOnset) to adjust the reverberation onset for AR AEs |  |  |
| Reverb:PredelayOffset |  |  |  |  |  |  |  | float |  |  |  | 0.0 |  |  | A temporal offset, in seconds, added to the reverberator predelay |  |  |
| Reverb:SpeedOfSound |  |  |  |  |  |  |  | float |  |  |  | 343 |  |  | The value for the speed of sound for reverberation only in meters per second. |  |  |
| Reverb:TheoreticalRdrToActualRdr |  |  |  |  |  |  |  | float |  |  |  | -7.99954 |  |  | Offset in decibels to be added to the log base 10 of an RDR value obtained from a statistical diffuse approximation based on the RT60 value. |  |  |
| Reverb:Default_Acoustic_Environment_Preset_Override |  |  |  |  |  |  |  | int |  |  |  | 0 |  |  | Index of a preset containing predefined reverberation parameters for Default AE. |  |  |
| Reverb:Default_Acoustic_Environment_Gain |  |  |  |  |  |  |  | float |  |  |  | 1.0 |  |  | The overall gain level for the reverberation in Default AE. |  |  |
| Reverb:Default_Acoustic_Environment_Predelay_Override |  |  |  |  |  |  |  | float |  |  |  | 0.154 |  |  | Offset in seconds from where revDefaultAEDSR is calculated in the RIR in the case of Default AE. |  |  |
| Reverb:PredelayFactorDefaultAE |  |  |  |  |  |  |  | float |  |  |  | 1.0 |  |  | A divisor applied to the predelay to adjust the reverberation onset for Default AE. |  |  |
| Reverb:DistanceGainDropDbDefaultAE |  |  |  |  |  |  |  | float |  |  |  | 6.0 |  |  | Control for late reverberation distance gain attenuation in Default AE. Distance doubling reduces late reverberation level by the given value in dB. |  |  |
| Reverb:MinimumDistanceDefaultAE |  |  |  |  |  |  |  | float |  |  |  | 5.0 |  |  | Minimum distance threshold in meters applied to late reverberation distance gain attenuation in case of Default AE. |  |  |
| Reverb:Default_Acoustic_Environment_Delay_Ratio |  |  |  |  |  |  |  | float |  |  |  | 8.78 |  |  | Delay ratio used in calculation of delay line lengths for Default AE. |  |  |
| Reverb:Default_Acoustic_Environment_Delay_Min_Ms_Override |  |  |  |  |  |  |  | float |  |  |  | 38.19 |  |  | Minimum delay line length in milliseconds used in calculation of the reverberation delay line lengths for Default AE. |  |  |
| SmoothingDecayRate |  |  |  |  |  |  |  | float |  |  |  | -2 |  |  |  |  |  |
| Sphoa:gain |  |  |  |  |  |  |  | float |  |  |  | 1 |  |  | The overall gain level for SP-HOA sources |  |  |
| virtualHeadSize |  |  |  |  |  |  |  | float |  |  |  | 0.1 |  |  | The radius of the listeners head in meters used for transitioning into spatially extended sound sources |  |  |
| HeadphoneEQ:Mode |  |  | string |  |  |  |  |  |  |  |  |  |  |  | The specType used to specify the local headphone equalization data, as given in Table 14 |  |  |
| HeadphoneEQ:CenterFreq |  |  | Vector of floats |  |  |  |  |  |  |  |  |  |  |  | Vector of centre frequencies, in Hertz. |  |  |
| HeadphoneEQ:Gain |  |  | Vector of floats |  |  |  |  |  |  |  |  |  |  |  | Vector of gains, in decibel. |  |  |
| HeadphoneEQ:filterType |  |  | Vector of strings |  |  |  |  |  |  |  |  |  |  |  | Vector of filterType, as given in Table 15 |  |  |
| HeadphoneEQ:Q |  |  | Vactor of floats |  |  |  |  |  |  |  |  |  |  |  | Vector of parametric filter Q values. |  |  |
| HeadphoneEQ:Preset |  |  | int |  |  |  |  |  |  |  |  |  |  |  | Preset to be used, as given in Table 16 |  |  |
| TargetHeadphoneEQ:Mode |  | string |  |  |  |  |  |  |  | The specType used to specify the target headphone equalization data, as given in Table 14 |  |  |  |  |  |  |  |
| TargetHeadphoneEQ:CenterFreq |  | Vector of floats |  |  |  |  |  |  |  | Vector of centre frequencies, in Hertz. |  |  |  |  |  |  |  |
| TargetHeadphoneEQ:Gain |  | Vector of floats |  |  |  |  |  |  |  | Vector of gains, in decibel. |  |  |  |  |  |  |  |
| TargetHeadphoneEQ:filterType |  | Vector of strings |  |  |  |  |  |  |  | Vector of filterType, as given in Table 15 |  |  |  |  |  |  |  |
| TargetHeadphoneEQ:Q |  | Vactor of floats |  |  |  |  |  |  |  | Vector of parametric filter Q values. |  |  |  |  |  |  |  |
| TargetHeadphoneEQ:Preset |  | int |  |  |  |  |  |  |  | Preset to be used, as given in Table 16 |  |  |  |  |  |  |  |
| AccessibilityEqLeft:Mode / AccessibilityEqRight:Mode |  |  |  | string |  |  |  |  |  |  |  | The specType used to specify the accessibility spectral compensation data for left/right ear. Can be freqgain or parametricfilters, as defined in Table 14 |  |  |  |  |
| AccessibilityEqLeft:CenterFreq / AccessibilityEqRight:CenterFreq |  |  |  | Vector of floats |  |  |  |  |  |  |  | Vector of center frequencies, in Hertz to define the accessibility spectral compensation for left/right ear |  |  |  |  |
| AccessibilityEqLeft:Gain / AccessibilityEqRight:Gain |  |  |  | Vector of floats |  |  |  |  |  |  |  | Vector of gains, in dB to define the accessibility spectral compensation for left/right ear |  |  |  |  |
| AccessibilityEqLeft:filterType / AccessibilityEqRight:filterType |  |  |  | Vector of strings |  |  |  |  |  |  |  | Vector of filterType, as given in Table 15 to define the accessibility spectral compensation for left/right ear |  |  |  |  |
| AccessibilityEqLeft:Q / AccessibilityEqRight:Q |  |  |  | Vector of floats |  |  |  |  |  |  |  | Vector of parametric filter Q values to define the accessibility spectral compensation for left/right ear |  |  |  |  |
| EventActivator:delay |  |  |  | float |  |  |  | 1.0 |  |  |  | The duration in seconds to wait before deactivating sound effect sources. |  |  |  |  |
| VoxelEarlyReflectionMaxNumReflections |  |  |  | int |  |  |  | 16 |  |  |  | Number of the clusters (NS) for defining the maximum number of audio sources used for the reflection modelling. |  |  |  |  |
| VoxelEarlyReflectionNumPoints |  |  |  | int |  |  |  | 3 |  |  |  | Number of the origin points (NP) for the volumetric ray marching algorithm. |  |  |  |  |
| VoxelEarlyReflectionDirections |  |  |  | int |  |  |  | 26 |  |  |  | Number of the ray directions (NR) for the volumetric ray marching algorithm. |  |  |  |  |
| VoxelEarlyReflectionGainIndoor |  |  |  | float |  |  |  | 2.0 |  |  |  | Early reflection tuning gain (gERIndoor_tuning) for indoor environment. This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |
| VoxelEarlyReflectionGainOutdoor |  |  |  | float |  |  |  | 1.0 |  |  |  | Early reflection tuning gain (gEROutdoor_tuning) for outdoor environment. This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |
| VoxelEarlyReflectionSectorDownmixEnabled |  |  |  | bool |  |  |  | false |  |  |  | The early reflection sector downmix mode flag (dERSectorDownmixFlag). |  |  |  |  |
| VoxelEarlyReflectionMinDelayMicroSeconds |  |  |  | int |  |  |  | 100 |  |  |  | Minimum early reflection propagation delay in microseconds. This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |
| VoxelEarlyReflectionMaxDelayMicroSeconds |  |  |  | int |  |  |  | 150000 |  |  |  | Maximum early reflection propagation delay in microseconds. This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |
| VoxelDiffractionFilterTuneParameter |  |  |  | float |  |  |  | 1.0 |  |  |  | Diffracted source attenuation effect control value (tdiffraction). This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |
| VoxelStorePreComputedPathDataByRequest |  |  |  | bool |  |  |  | false |  |  |  | Flag triggering the output of the currently cached JPS path data and diffraction map. This parameter is modifiable during the rendering process (i.e., runtime configurable). |  |  |  |  |

## Geometry Data Decompression

### General

The parametrized geometry data underpins the foundation of the acoustic description of a scene. Multiple renderer stages (for example Early Reflections (6.6.6), Occlusion(6.6.9), and Diffraction(6.6.10)), rely on the geometry data for high quality auralization. This processing is done through a combination of intersection tests, ray tracing and filtering based upon the geometry and the acoustic materials on the geometry. Once the transmission and the decoding of the geometry data is done, the geometry is then inputted into the framework where RIs can refer and query it. Further information about geometry parameter definitions can be found in the Encoder Input Format (EIF, B.4).

### Metadata extraction

The compressed metadata is extracted from the bitstream and decompressed using the Flexible Range Decoder functions defined in the pseudocode below.

First, the structure FlexRangeDecoder that stores the internal state of the decompression algorithm is created and the Flexible Range Decoder is initialized.

typedef int (*frdReadByteFunction)();

typedef struct

{

unsigned int low;

unsigned int range;

int bit_count;

int max_allowable_bit_count;

int eof_bytes_read_count;

frdReadByteFunction read_byte;

} FlexRangeDecoder;

unsigned int readByte(FlexRangeDecoder* frd) {

int byte_read = frd->read_byte();

frd->bit_count += 8;

if (byte_read == -1) /* EOF */

{

frd->eof_bytes_read_count += 1;

/* 3 additional bytes are read by the finalization of the range decoder */

if (frd->eof_bytes_read_count <= 3) {

return 0; /* return a dummy 0 byte used for finalization */

}

}

return (unsigned int)byte_read;

}

void frdInit(FlexRangeDecoder* frd, int max_available_bits, frdReadByteFunction read_byte) {

frd->low = 0;

frd->range = 0xFFFFFFFF;

frd->bit_count = 0;

if (max_available_bits != -1) {

frd->max_allowable_bit_count = max_available_bits + 30;

} else {

frd->max_allowable_bit_count = 0x7FFFFFFF;

}

frd->eof_bytes_read_count = 0;

frd->read_byte = read_byte;

for (int i = 0; i < 4; i++) {

frd->low = (frd->low << 8) + readByte(frd);

}

}

Subsequent, the metadata is read from the bitstream and decompressed calling the following function listed as pseudocode below. The function has a pointer to the structure FlexRangeDecoder and the alphabet size as two inputs. However, in the further syntax description, the first pointer argument is always omitted for better readability.

unsigned int frdReadUniform(FlexRangeDecoder* frd, unsigned int alphabet_size) {

frd->range /= alphabet_size;

unsigned int value = frd->low / frd->range;

frd->low -= value * frd->range;

if (frd->range < 0x01000000) {

frd->low = (frd->low << 8) + readByte(frd);

frd->range <<= 8;

if (frd->range < 0x01000000) {

frd->low = (frd->low << 8) + readByte(frd);

frd->range <<= 8;

}

}

return value;

}

### Geometry

#### Data elements and variables

vertexCount	The number of vertices in this mesh

faceCount	The number of faces in this mesh

vertexQuantStep	The quantization step size for vertices in meters. The range of this value is between 0.1 and 0.0001

reserved	Skip and ignore these bits

bbBits	The number of bits used to store bounding box dimensions

bb_x_min	The minimum value for the bounding box dimension on the x axis

bb_x_max	The maximum value for the bounding box dimension on the x axis

bb_y_min	The minimum value for the bounding box dimension on the y axis

bb_y_max	The maximum value for the bounding box dimension on the y axis

bb_z_min	The minimum value for the bounding box dimension on the z axis

bb_z_max	The maximum value for the bounding box dimension on the z axis

bb_dim	The value of the current bounding box dimension

bb_dim_sign	The flag which indicates the sign for the bounding box dimension value. If true, then minus sign is used, and if false, then plus sign is used

gcdX	The greatest common divisor amongst all vertices in the x axis

gcdY	The greatest common divisor amongst all vertices in the y axis

gcdZ	The greatest common divisor amongst all vertices in the z axis

gcdPresent	The flag to indicate, that if true, there is a greatest common divisor that is greater than one in any of the dimensions

gcdXPresent	The flag to indicate, that if true, there is a greatest common divisor that is greater than one in the x dimension

gcdYPresent	The flag to indicate, that if true, there is a greatest common divisor that is greater than one in the y dimension

gcdZPresent	The flag to indicate, that if true, there is a greatest common divisor that is greater than one in the z dimension

vertexX	The list of vertices along the x dimension

vertexY	The list of vertices along the y dimension

vertexZ	The list of vertices along the z dimension

alph_mat	The number of symbols in the alphabet for materials

alph_cmi	The number of symbols in the alphabet for compacted material indices

is_present[]	The boolean array to indicate if a material index is present in the bitstream

cmi_to_mi	The map from compacted material index to actual material index

v1[]	The list of vertex indices for the first vertex of all faces

v2[]	The list of vertex indices for the second vertex of all faces

v3[]	The list of vertex indices for the third vertex of all faces

cmi	The compacted material index

same_cmi	The flag to indicate if the current compacted material index is the same as the previous compacted material index

mi[]	The list of material indices for all faces where the element values of -1 indicate no acoustic material for that face

#### Decoding Process

The geometry decoding is composed of six subsequent processing units.

The first unit decodes the header information, such as the number of vertices (vertexCount) and the encoder selectable quantization step (vertexQuantStep).

The second unit decodes the bounding box for the geometry. The bounding box is the spatial constraint of all vertices. In other words, no vertex can lie outside of this bounding box. Since the vertices are quantized to a uniform spaced grid, the vertices will fall on a uniform space grid bounded on this box. The variables bb_x_min, bb_x_max, bb_y_min, bb_y_max, bb_z_min and bb_x_max are used to represent this bounding box.

The third unit decodes the step size between all vertices per dimension (gcdX, gcdY, gcdZ) – this is the greatest common divisor amongst all vertex values. This is done with the intention to reduce the number of points in a dimension along the bounding box to achieve less bitstream complexity.

The fourth unit decodes the vertices for each dimension. The vertex is either represented as a raw value or an index. Indices point to a sorted, dynamically-sized array which contains the vertices; the size of this array never exceeds the difference between the bounding box maximum and minimum divided by the greatest common divisor (i.e. gcdX). Before vertex decoding begins, the maximum and minimum bouding box points are pre-inserted into this array. For each vertex (as determined by vertexCount), the following processing is taking place: if any vertex has not been decoded yet, a flag indicates whether or not the vertex is coded with its index in the array or its raw value; if an index is transmitted, that index to access and read the value from the array is used; if the raw value is transmitted, the value is send to the range decoder where the alphabet size is determined as the difference between the maximum and minimum bounding box dimension divided by the greatest common divisor. Finally the value is stored into the array.  This is done across all three dimensions. The pseudocode below exemplary demonstrates the implementation of the aforementioned unit for the x-dimension:

vertex() {

alph = ((bbq_max - bbq_min) / gcd) + 1;

us; // variable sized array containing unique sorted values only

us.insert(0);

us.insert(bbq_max – bbq_min);

for (int i = 0; i < vertexCount; i++) {

coded_with_index = true;

if (us.size() < alph) {

coded_with_index = frdRead(2);

}

if (coded_with_index) {

val = us[frdReadUniform(us.size())];

} else {

val = frdReadUniform(alph);

us.insert(val);

}

}

}

The fifth unit reconstructs the vertices in accordance with the data retrieved from the prior units. For each vertex, the vertex is multiplied by the associated greatest common divisor, summed with the minimum bounding box value and multiplied by the quantization step. This results in the final vertex data.

xv[i] = (xv[i] * gcdX + bbq_x_min) * vertexQuantStep;

The sixth unit decodes the face list. A face is a triplet of vertex indices and, optionally, an acoustic material associated with the described surface. The vertex indices access the array mentioned in the fifth unit. If a value of 0 is transmitted for the material, this represents no associated material for that face; whereas a positive non-zero value indicates the index to material list detailed in 6.5.3. The vertices are decoded via range decoder using a uniform distribution where the alphabet size is vertexCount. The bitstream informs the decoder if the material for the current face is the same as the previous face, saving bitstream complexity.

### Materials

The renderer uses information from acoustic materials to auralize scenes with high quality. For example, the early reflection stage and occlusion stage additionally consider acoustic materials to enrich auralization (see 6.6.6 and  6.6.9 respectively).

Therefore, Geometry elements can optionally have acoustic materials attributed to them; whether it’s a primitive, where the material is associated with the entire surface area of the primitive, or a mesh, whereupon each face can be attributed with a material. The integer used by geometry to reference acoustic materials acts as an index to the list in which all materials are decoded into: an acoustic material is described via that list where each element holds four coefficients and an associated frequency. The coefficients are:

- specular reflection, which represents the energy being reflected in a distinct outgoing direction from the direct sound;

- diffused scattering, which represents energy being diffusely scattering back from the material;

- transmission, which represents the energy that is passed through the material without changing the direction;

- coupling, which represents the energy that excites vibrations in the structure and is reemitted by the entire structure.

The sum of these four coefficients, per frequency, must be less than or equal to 1, and be greater than or equal to 0. The difference between 1 and the sum of the four coefficients, per frequency, represents the energy that is dissipated into heat.

#### Data elements and variables

materialCount	The number of materials in this set, which is always one in the current implementation

stepIdx	The index of the quantization step for the coefficients

cxStep	The computed step size for the material coefficients

materialFreqGridIdx	Index in the list of frequency grids defined in FreqGridData().

materialFreqCount	The number of frequencies in this acoustic material

spcRflIs0	The flag to indicate that, if true, the specular reflection coefficient is 0

diffScatIs0	The flag to indicate that, if true, the diffuse scattering coefficient is 0

trnsIs0	The flag to indicate that, if true, the transmission coefficient is 0

cplIs0	The flag to indicate that, if true, the coupling coefficient is 0

freq	The frequency associated with the following coefficients, retrieved from the frequency grid in FreqGridData().

specularReflection	The specular reflection coefficient for this frequency, with a range of values between 0.0 and 1.0. Indicates the energy reflected back in a distinct outgoing direction

diffusedScattering	The diffused scattering coefficient for this frequency, with a range of values between 0.0 and 1.0. Indiates the energy that is diffusely scattered back from the material

transmission	The transmission coefficient for this frequency, with a range of values between 0.0 and 1.0. Indiciates the energy which passes through the material without changing the direction of the sound

coupling	The coupling coefficient for this frequency, with a range of values between 0.0 and 1.0. Indiciates the energy which excites vibrations in the structure and is reemitted by the entire structure

#### Decoding Process

First, the number of materials (materialCount) and the encoder-selectable coefficient step size (cxStep) is decoded.

After that, for each material (as determined by materialCount) and for each frequency the set of coefficients (specularReflection, diffusedScattering, transmission and coupling) is decoded. All coefficients are within a range between 0 and 1.

Next, the frequency is decoded which is an integer between 1 and 24000.

Each coefficient has a flag to indicate if its value is equal to zero (spcRflIs0, diffScatIs0, trnsIs0, cplIs0). If the flag value is false, the coefficient is decoded using a range coder where the alphabet size is cxStep + 1.

## Renderer stages

### Effect Activator

#### General

The Effect Activator stage manages the activation and deactivation of render items that are associated with the playback of sound effects.

Scene objects may be activated and deactivated within the scene state during runtime. When a scene object is activated the Effect Activator stage checks if the object is associated with a sound effect stream, and if so ensures that after the sound effect has been active for the duration of the audio stream it is currently rendering that it is deactivated, and all Render Items are likewise deactivated.

#### Data Elements and Variables

audioFileDurations	a map of audioStreamId and streamDuration, as received in the AudioPlus payload.

deactivationDelay	a float value representing the time in seconds after the audio stream is finished before render items should be deactivated.

frameCounter	an integer value that represents the number of audio frames that have been processed since the start of scene playback.

activeEffects	a map of currently active audio effects, and the time, in frames, at which they should be deactivated.

#### Stage Description

##### Initialization

At initialization the EffectActivator stage creates the map of audioFileDurations from the audio stream information contained in the scene payload, 6.3.2.10. The map contains all the duration of all audio streams that are indicated as being used for sound effects, referenced by their integer id.

For all object sources that exist in the scene state, the Effect Activator stage subscribes to the SceneObjectObserver functionality, as described in 6.4.1.2.1, for those scene objects sources.

##### Object Changes

The stage shall be notified of all changes to the object sources for which a subscription was added during initialization. When the stage is notified that the Play property of an object source is set, it shall check whether the audio stream currently associated with the scene object source exists in the map of audioFileDurations. If the audio stream exists in the map, the id of the object source is added to the map of active effects, with the deactivation time given as the sum of the frameCounter, deactivationDelay, and duration given for the audio stream in the audioFileDurations. The active flag of the scene object is set to true.

The deactivationDelay shall be set such that sufficient time is allowed for the time taken for the sound to reach the listener from the source position is accounted for. By default this is configured to be 1 second, but may be changed via the local configuration interface, 6.4.2.6, or may be set based on the dimensions of the scene, such as the time of flight equivalent to two times the longest dimension of the scene bounding volume.

##### Update Thread

When called, the update thread shall check if the deactivation time of any audio effect in the list of activeEffects has been exceeded by the frameCounter, and if so, shall deactivate the object within the scene state and all associated render items that have been created.

##### Audio Thread

The Effect Activator stage does not do any audio processing. During audio thread processing callback only the frameCounter is incremented.

### Room assignment

#### General

The Acoustic Environment (AE) Assignment updates metadata of each RI and the listener at each update step to reflect the current scene configuration with respect to AEs defined in the scene. This can be used by the Reverb-, Portal- and other stages to determine which signals are currently relevant for the late reverberation processors and which reverberation extent items are audible. A special case of the Room Assignment is the Default AE, which is a generic AE that can be applied whenever the listener is situated in a scene part without a bounded AE.

This stage checks whether the listener and the RIs are inside any of the acoustic environments (AEs) in the scene and documents the result for later stages.

#### Data elements and variables

acousticEnvs	a vector of acousticEnv objects. An acousticEnv object contains all the relevant information of an AE:

- Unique identifier as an integer

- Bitstream ID from the encoder as an integer

- Boundary

- Human-readable name as string

The relation of acousticEnv and AE is a one-to-one mapping. acousticEnv is the renderer’s representation of an AE with the same data.

acEnvId 	Unique ID of an AE in the scene. This is different from the bitstream ID that came from the encoder and is generated incrementally by the renderer upon the creation of each acousticEnv.

#### Stage description

##### Update thread

For each update, the stage loops through the listener and all the RIs. If the listener or a RI is in none of the acousticEnvs, the id of the Default AE is assigned onto the acEnvId data field of the RI. Otherwise, the id of the corresponding acousticEnv is assigned. Figure 8 is an example of the classification of whether a sound source or a listener is inside or outside of an AE.

Figure 8 — Illustration of an acoustic environment (AE) with a listener inside (1) and one outside (2) the region

##### Audio thread

There is no Audio Thread processing in this stage.

#### Default acoustic environment

##### General

This stage is a special case of room assignment that can be applied whenever the listener is situated in a scene part without a bounded AE. It provides a definition for a generic room environment used by the renderer to control reverberation parameters.

The purpose of this stage is to provide perceptually plausible acoustical properties in the scene regions, which do not have an AE specified in the bitstream or the LSDF. The lack of the early reflections and/or late reverberation signals results in unnaturally “dry” listening impression. To overcome this, the Default AE provides generic reverberation that aims at creating a more natural and not overly “dry” scene acoustics.

The Default AE solution consists of two main aspects:

- Reverberation parameters

- Signaling mechanism and secondary controls

##### Reverberation parameters

The reverberation parameters are provided using one of the predefined sets in A.3 or by providing custom Default AE reverberation parameters as part of the payloadReverb() bitstream

##### Signaling mechanism and secondary controls

When detecting which acoustic environment the listener is currently present in, the renderer generates a Null pointer when the listener is located in a scene part, which is not contained in a defined acoustic environment. In this case the renderer refers the Default AE parameters to be used by the reverberation stage.

Additional control parameters are being provided for further adjustments of the Default AE rendering. Those can be set either as part of the the payloadReverb() bitstream payload, or as presets in the renderer as defined in A.3.

revDefaultAEDistanceGainDropDb : defines amplitude gain drop of the reverberation level based on distance doubling between source and listener in dB

Respective preset and default value (float) in renderer:

presetDefaultAEDistanceGainDropDb{ 6.0f };

revDefaultAEMinimumDistance : defines minimum distance between source and listener in meters, below which the amplitude of the reverberation remains constant

Respective preset and default value (float) in renderer:

presetDefaultAEMinimumDistance{ 5.0f };

revDefaultAEGain : defines a independent global reverberation gain factor for Default AE

Respective preset and default value (float) in renderer:

presetDefaultAEGain{ 1.0f };

### Granular synthesis

#### General

Granular synthesis is a method to render procedural audio, which means sound that can evolve in a controlled way using real-time input. The synthesis is based on original recordings that are divided into small pieces, a.k.a. grains. The synthesis works by chaining together grains at rendering time, where the choice of which grains to use can be controlled by live user input, virtual scene state changes, pre-defined trajectories etc. Granular synthesis is typically used for generating continuously evolving sounds that cannot easily be generated using triggered audio files or continuous loops where you have limited control over how the sound evolves.

Examples of sound sources can be generated with granular synthesis:

- Engine sounds where the RPM of the engine changes over time.

- The sound of flowing water where the flow varies.

- Nature sounds, such as the sound of a forest where the frequency of bird chirps varies.

- Friction sounds, where objects slide against each other and where the speed and force vary.

A special version of granular synthesis is when grains are stored into a database where each grain has a specified position in a so-called descriptor space. The descriptor space is a multi-dimensional space where each dimension represents a certain descriptor of the sound. Figure 9 shows an example of a granular database with a 2-dimensional descriptor space.

A descriptor can be a measurable feature of the sound, like pitch, amplitude, energy in a certain frequency band etc. But it can also represent some abstract feature, related to how the sound is generated. For example, for the squeaking sound of an old door, one descriptor can represent the opening angle of the door. Another example is the sound of a water stream into a glass, where one descriptor can describe the level of water in the glass and another descriptor can describe the flow of water, which are aspects that both have an influence on the generated sound. In the latter case, the granular database would have a 2-dimensional descriptor space where the position in both dimensions has an influence on the generated sound.

Figure 9 – An example of a 2-dimensional granular database where grains, depicted as blue circles, are spread out in the descriptor space. The target position is marked with a cross.

At rendering time, a target descriptor coordinate, or target position, in the descriptor space is continuously updated in real-time to control the sound so it evolves in a way that matches the user input or changes in the virtual scene.

The granular databases can be included in the bitstream, or they can be retrieved from a separate channel by using the GranularSoundId and/or GranularDatabaseFilePath, which are specified in the GranularSounds item in payloadScene. If a granular database is missing, the rendering stage will skip rendering of the source that is referring to this database and silence will be produced for this source until the missing database is retrieved.

#### Data elements and variables

##### Grain

The grain structure holds information that belongs to one grain.

Dim						This value represents the dimension of the descriptor 							coordinate space that the grain resides in.

DescrCoord	This vector of values represents the grain descriptor coordinates for the grain.

grainLen	This value represents the length of the grain.

DescrCoordTrend	A 3-dimensional vector of values that represents the trend of the grain descriptor coordinates for the grain.

origRecordingPos	The position of the grain within the original audio file that the grain was extracted from.

grainProbabilityGroup	The group that the grain belongs to.

perGrainProbabilityValue	Individual probability value associated with the grain.

mixingRule	The optimal mixing rule for this grain.

##### GranularDatabase

The GranularDatabase structure holds information that belongs to one granular database.

dimensions					The dimension of the descriptor space of the									database.

numberOfGrains					The number of grains in the database.

channels					The number audio channels used in the database.

grains					A vector of grains.

coordScaling			A vector of scaling factors for scaling the descriptor space 				coordinate system.

smoothingTargetCoordinates	This vector represents smoothing parameters used to smooth target coordinate values obtained from the user input.

searchRadius	This value represents the radius of search while searching for grains via kDTree search.  This radius is used in the scaled descriptor space.

grainHistoryLength	This value represents length of the history of used grains.

numMixRule	This value represents the number of mixing rule coefficients available in the database.

grainMixRule	This is a vector of values representing the mixing rule coefficients in different positions.

grainMixingRuleCoordinates	This is a vector positions corresponding to the mixing rules in the vector grainMixRule.

probabilityWeightPosition	This value represents the weight used to calculate the probability value associated with the descriptor position evaluation in the random selection.

probabilityExponentPosition	This value represents the exponent used to calculate the probability value associated with the descriptor position evaluation in the random selection.

probabilityWeightHistory	This value represents the weight used to calculate the probability value associated with the history of the grains rendered for random selection.

probabilityExponentHistory	This value represents the exponent used to calculate the probability value associated with the history of the grains rendered for random selection.

descrTrendAvailable	This value indicates whether descriptor coordinate trend is available as a parameter in the granular database.

probabilityWeightTrend	This value represents the weight used to calculate the probability value associated with the trend of the position of the grain in the descriptor space for random selection of grains.

probabilityExponentTrend	This value represents the exponent used to calculate the probability value associated with the trend of the position of the grain in the descriptor space for random selection of grains.

probabilityWeightOriginalRecordingPosition	This value represents the weight used to calculate the probability value associated with the index or position of the grain in the original recording for random selection.

probabilityExponentOriginalRecordingPosition This value represents the exponent used to calculate the probability value associated with the index or position of the grain in the original recording for random selection.

randomSelection	This value indicates whether random selection is to be used for granular synthesis. If 0, then the grain with highest probability weight is always chosen.

useInterpolation	This value indicates whether interpolation is to be used for granular synthesis.

grainInterpolationDistThreshold	This value represents the distance threshold for interpolation.

useInterpolationResampling	This value indicates whether resampling operation is to be performed for granular interpolation.

grainOverlapAmount	This value represents the parameter of overlap amount or percentage used for overlap-add operation in granular synthesis.

descrTrendMethod	This value indicates the index of the method chosen to evaluate the descriptor trend coordinate values for random selection.

fastGrainSwitchThreshold	This value represents fast grain switching threshold distance.

InterpolationMethod	This value represents the index of interpolation method to be chosen.

pitchDimension	This vector of values indicates the descriptor coordinate dimension that corresponds to pitch. This is used to calculate the length of the interpolated grain.

newGranPosValue	This value sets new target descriptor space coordinates.

probabilityGroupVal	This value sets the initial probability weight of a group in the bitstream.

granularProbGroupNo	This value specifies an index of a grain probability group for a dynamic update.

granularGroupProb	This value sets the probability weight of a group with a dynamic update.

#### Stage description

Figure 10 – A block diagram of the main functional blocks of the granular rendering stage.

In contrast to most other rendering stages, the granular synthesis rendering stage produces audio output buffers without using any input audio buffers. When initializing the renderer, the granular databases are loaded for each sound source, and these are used for producing the audio output. Since the renderer has random access to the whole granular database and grains may be selected from any part of the descriptor space, fast transitions can be generated with very low latency.

Smoothing of updated target position

The received updates of the target position, as specified by newGranPosValue, are smoothed using a forgetting factor controlled by the parameter smoothingTargetCoordinates which is a vector of smoothing coefficients for each dimension of the descriptor space. The smoothed target position, Tn,Smooth, in the n:th dimension is calculated from the updated coordinate, Tn, as

| Tn,Smooth=smoothingTargetCoordinatesn∙Tn,Smooth+ (1-smoothingTargetCoordinatesn)∙Tn | (35) |
| --- | --- |

Overlap-add of consecutive grains

Each grain in the database has its own length. A part of the length of the grain is used for overlap-add windowing to transition from one grain to the next. The length of the overlap region of a grain, LOverlap, is calculated as

| LOverlap=LGrain∙grainOverlapAmount-1 | (36) |
| --- | --- |

The first and last LOverlap samples of each grain are the overlap regions of the grain. When the rendering of the current grain reaches the overlap region at the end, the scheduling of the next grain is initiated and an overlap-add is done where the current grain is cross-faded into the next. Here an optimized overlap window is used, which is described in section 6.6.3.3.7. The length of the overlap region is set to the shortest of the two grains overlap regions. If grainOverlapAmount is not specified in the bitstream, it is assumed to be 0.

##### Grain scheduling using a weighted random selection

As described above, the grains in the granular databases are stored with metadata specifying a position within the descriptor space, which is a multi-dimensional coordinate system with up to three dimensions. A target position in the descriptor space controls which grains should be considered to be selected. To achieve a natural generated sound, the selection, or scheduling, of grains is based on a weighted random selection where different aspects of the grains are included.

The first step in the weighted random selection is to select a set of candidate grains that will be further evaluated. The candidate grains are selected from the granular database by extracting the grains that reside within a radius distance from the target position in the descriptor space.

For each grain within the set of candidate grains, a weight is calculated, and then a weighted random selection is performed to select a grain from the set of candidate grains.

The weight for each grain is taking several aspects into account, where each aspect contributes to the final weight:

- The distance from the position of the grain to the target position.

- The history of previously used grains, where the previously used grains will be assigned a lower weight.

- The trajectory trend associated with the grain compared to the current trajectory trend of the target position.

- The distance between the grain and the currently used grain in the original recording.

- The probability weight of an individual grain given by the perGrainProbabilityValue.

- The probability weight of the group the grain belongs to if it belongs to a group.

The final weight, piFIN, for grain i is calculated as

| piFIN=pi0pi1pi2pi3pi4pi5 | (37) |
| --- | --- |

where pi0 to pi5 are the weights corresponding to the different aspects. The calculation of the weights for the different aspects is described in more detail in the respective subsections below.

The degree of influence of each aspect can be controlled by the creator of the database by setting the respective probabilityWeight and probabilityExponent parameters that control how much each aspect should affect the final weight of each grain in the weighted random selection. The probabilityWeight parameter sets the difference between the highest and lowest values of probability weight for that aspect. E.g., setting the probabilityWeightPosition to 2.0 means that the grain(s) closest to the target position will get two times higher probability weight compared to the grain that is furthest away.

The probabilityExponent parameter adds an exponent to the probability weight of that aspect, which will affect both the difference in maximum and minimum value of the probability weight of that aspect but also the shape of the probability function. E.g., if the probabilityWeightPosition is set to 2.0, setting the probabilityExponentPosition to 2.0 would increase the difference between the maximum and minimum probability weight to 4 but also make the probability function more exponential so that more weight is given to grains that are the closest to the target position as is shown in Figure 11. The parameters probabilityWeightHistory, probabilityExponentHistory, probabilityWeightTrend, probabilityExponentTrend, probabilityWeightOriginalRecordingPosition and probabilityExponentOriginalRecordingPosition specify the weight and exponent of the respective aspects.

The individual grain probability and group probability do not have weight or exponent parameters since they are given explicitly as probabilities and are not derived from other parameters.

Figure 11 – An example of how the probability weight for the descriptor space position aspect can be set using the probabilityWeightPosition and probabilityExponentPosition parameters.

In some special cases, a random selection of grains is not desirable. For example, a granular database may contain a set of single cycle waveforms where the expected output is a very controlled static sound without any randomization. In this case the flag randomSelection can be set to false, in which case the grain with the highest weight will always be selected without any random selection. If randomSelection is set to true, a weighted random selection will be carried out.

##### Distance in the descriptor space

The target position is used to control from which part of the descriptor space grains should be selected. This means that grains that have a position that is close to the target position is given a higher weight in the weighted random selection.  The distance between the position of a grain, given by DescrCoord, and the target descriptor coordinate comes in at two stages in the weighted random selection, first when selecting the set of candidate grains, where only the grains that are closer than a specified radius, searchRadius, are included as described in section 6.6.3.3. Then the distance between the grain and the target position is also used as one of the aspects when calculating the weights for the candidate grains and can there further give higher weight to grains that are close to the target position. Here the Euclidian distance from the position of a grain to the target position, di, is measured for all candidate grains. The weight, p0i, for candidate grain i, is then calculated as

| p0i =mindMAXdi, probabilityWeightPositionpEP , di>0 probabilityWeightPositionpEP ,di=0 | (38) |
| --- | --- |

where dMAX is the distance of the candidate grain that is furthest away from the target position and pEP is the parameter probabilityExponentPosition.

##### Grain history

It is often not desired that the same grain is used repeatedly since that may lead to an unnatural sound. To be able to avoid repetition of grains, a history of used grains is kept and updated by the granular renderer. By checking if a grain is present in the grain history, and at what position in the grain history it is present, a probability weight can be calculated.

The length of the grain history is set by the parameter grainHistoryLength in the granular database. If a grain is present in the grain history, the probability weight is calculated based on its position, h, in the grain history. If h=0, it means that the grain was the last one used. If h=4, it means that four other grains has been used since the grain was used that last time.

| p1i = R+h-11-R(grainHistoryLength -1)pEH , h>0 0 ,h=0 1 , grain is not in the history | (39) |
| --- | --- |

Where R = 1/ probabilityWeightHistory

and pEH is the parameter probabilityExponentHistory.

##### Descriptor trajectory trend

When performing grain extraction, the descriptor coordinates are used as the main selection criterion. But the trend of the original descriptor trajectory of the grain also gives important information. For example, if an engine sound is modelled with a granular database with one descriptor that denotes the RPM of the engine, the trend of the descriptor corresponds to the acceleration or deceleration of the engine at the time instant in the recording that the grain was extracted from. A grain that was extracted from a portion of the recording where the engine was accelerating will have a pitch that is slightly lower at the start than at the end and will therefor fit best when the desired output is the sound of an accelerating engine. The descriptor trend of a grain is specified with the parameter DescrCoordTrend.

In the more general case, where a multi-dimensional descriptor space is used, the descriptor trend is a vector that corresponds to the direction of the trajectory that describes how the descriptors were changing at the time of the original recording. The value of the trajectory trend represents the change of descriptor coordinates per second for the duration of the grain and can be calculated as:

| trend =coordEND-coordSTARTduration | (40) |
| --- | --- |

Where the coordSTART and coordEND are the descriptor coordinates of the original recording at the time instants representing the start and end of the grain. The duration is the length of the grain in seconds.

Similarly, the trend of the target descriptor trajectory at rendering time describes the change in target position per second:

| targetTrend =coordNEW-coordLASTtime_delta | (41) |
| --- | --- |

where coordNEW is the latest target descriptor coordinate, coordLAST is the last target descriptor coordinates and time_delta is the time elapsed since the last update.

The descriptor trend describes both the direction and the rate of change. Referring again to the example of the engine, if the granular database includes grains that correspond to the same RPM but with different acceleration, the grains that correspond to a similar acceleration as that of the target descriptor trajectory should be preferred.

The probability weight of the trajectory trend aspect can be calculated using three different modes:

- The default mode where the probability weight is calculated from the norm of the difference between the trajectory trend of the grain and the current trajectory trend of the target position.

- A binary mode where the sign of the dot-product of the trajectory trend of the grain and the trajectory trend of the target position is used to set the probability weight of a grain. If the sign of the dot-produce is positive, the two vectors are pointing in directions less than 90 degrees apart and the grain will receive the highest probability weight. If the sign is negative, the two vectors are pointing in directions that are more than 90 degrees apart and the grain will receive the lower probability weight.

- A combined mode where both the sign of the dot-product of the trajectory trend vectors and norm of the difference in trajectory trend are evaluated. For the case that the sign of the dot-product is negative, the probability for the evaluated grain will be set low. If the sign is positive, the probability will depend on the size of the norm of the difference in trajectory trend, in the same way as for the default mode.

The selection of method is controlled by the parameter descrTrendMethod which may have the value 1, 2 or 3 for the respective methods.

For mode 1, the norm of the difference in the trajectory trend of a grain and the current trajectory trend, ni, is calculated:

| ni =DescrCoordTrend-smoothedTargetTrend | (42) |
| --- | --- |

Where smoothedTargetTrend is calculated as

| smoothedTargetTrend =0.9∙smoothedTargetTrend+0.1∙(T-TPREV) | (43) |
| --- | --- |

where T is the current target position and TPREV is the previous target position.

A weight, pi, for candidate grain i, is then calculated as

| p2i =minnMAXni,probabilityWeightTrend pET , ni>0 probabilityWeightTrendpET ,ni=0 | (44) |
| --- | --- |

where nMAX is the trend norm of the candidate grain that has the largest trend norm and pET is the parameter probabilityExponentTrend.

For the binary mode 2, the probability weight, p2i, is set to a low number close to zero, 0.00001, if the dot-product is negative.  If the dot product is positive p2i is set to 1.

In mode 3, the probability weight, p2i, is set to a low number close to zero, 0.00001, if the dot-product is negative, just as with mode 2. Otherwise, it is calculated as for mode 1.

If descrTrendAvailable is set to false, no descriptor trend information is available for the granular database and p2i is set to 1.

##### Difference position in the original recording

For some sound sources, the way that the sound evolves may not be completely described by the changes in descriptor coordinates. Sometimes sound evolves in a way that depends on what happened earlier. For example, the screeching sound of an old door may have a slightly different screeching sound every time it is opened, even if the opening of it is done at the same speed etc. In these cases, the sound of grains with similar descriptor values and descriptor trend may sound very different and combining them may result in unnatural discontinuities that were not there in the original recording. A way to avoid these discontinuities is to assign a higher probability to grains that come from the same part of the original recording as the grain that was rendered previously. Grains that came from the same part of the original recording are expected to be closely related and resemble each other in character and are therefore good candidates when selecting the next grain.

In order to measure how close one grain from a particular recording is to another grain from the same recording, a time difference can be calculated which corresponds to the difference in time instant in the recording that the two grains were extracted from. If the grains were close, this time difference is small. To calculate the time difference between two grains, metadata that tells from which original recording each grain was extracted and at what time instant can be used. This metadata, therefore, provides a measure of closeness between the two grains. This metadata can be specified in a compact way as two values: a recording identifier assigned to the recording of origin and a position value within that file that identifies the time instant in that recording that the grain was extracted from.

The position of a grain in the original recording can be specified with the origRecordingPos parameter in the bitstream. This is a floating-point value where the integer number corresponds to an index of a certain original audio file or segment of an audio file, and the decimal part describes the position within that audio file or segment of an audio file. If the integer parts of the origRecordingPos of two grains are different, it means that they come from different recordings and therefor the probability weight is set to its minimum value. If the integer part is the same, it means that the two grains come from the same recording and the distance between them is measured as the difference between the respective decimal parts.

For example, a value of 1.003 means that the grain originally came from the recording with index 1 and that the position of the grain within that recording was close to the start.

Another grain has the value of 1.981, which means that the grain originally came from the same recording with index 1 and that the position of the grain within that recording was close to the end.

The difference in position, di, for these grains is calculated by setting the integer number to zero and calculate the norm of the difference of the decimal parts only:

| di =0.981-0.003 | (45) |
| --- | --- |

A weight, p3i, for candidate grain i, is then calculated as

| p3i =min1di, probabilityWeightOriginalRecordingPosition pER , di>0 probabilityWeightOriginalRecordingPosition pER ,di=0 | (46) |
| --- | --- |

Where pER is the parameter probabilityExponentOriginalRecordingPosition.

##### Grain groups

If a grain is assigned to a grain group as specified by the parameter grainProbabilityGroup, the probability weight of that group, given by granularGroupProb, is used in the calculation of the final weight for the grain. If the grain does not belong to any group, this aspect will not have any influence of the final weight for the grain, which is equivalent to that the grain belongs to a group with the probability weight set to 1.0.

Grain groups are defined in the bitstream as a list of probabilityGroupVal where the index of the group is implicitly given by the order in the list.

The probability of groups can then be updated in real-time using dynamic updates where granularProbGroupNo is specified along with the new probability weight, granularGroupProb.

All grains are by default assigned to the group with id 0, which has the default probability weight 1.0.

###### Scaling of the descriptor space coordinate system

For some granular databases, the different dimensions of the descriptor space have very different meaning and may need different weight in the weighted random selection. For example, one dimension may represent the pitch of a sound while another dimension represents a difference in the tonal character of the sound. In this case, the accuracy requirement in the pitch dimension may be higher than in the other dimension.  By scaling the coordinate system differently in the different dimensions, the accuracy can be increased in a certain dimension.

This has an influence on the selection of the set of candidate grains. All grains within a radius around the target position are chosen as candidate grains, but if the coordinate system has been scaled, this radius may be different in the different dimensions so that the selection of candidate grains is done using e.g., an elliptical area around the target position instead of a circular area, or an ellipsoid if the descriptor space has three dimensions. The coordinate scaling is specified by the parameter coordScaling. The scaled coordinates are calculated as

| Xscaled=coordScalingx∙X Yscaled=coordScalingx∙Y Zscaled=coordScalingx∙Z | (47) |
| --- | --- |

The scaled coordinates are used throughout in the granular rendering in place of the original coordinates.

##### Grain interpolation

In some cases, the granular database may have too few grains, or grains that do not evenly fill the descriptor space. In these cases, the selection of grains becomes more restricted. For some target positions in the descriptor space, there might not be any grains that are close, or the close grains are few so that they need to be repeated frequently.

In the case that the grain scheduler does not find a grain that is close enough to the target position, or that all grains close to the target position have already been used recently, a new grain can be created; this new grain is referred to as an “interpolated grain”. The method for creating an interpolated grain includes selecting two grains that are then interpolated to form a new temporary grain. It is important to select two grains that are not on the same side of the target position in the descriptor space. This is done using the following steps:

- Given an updated target position, select a set of candidate grains and use the weighted random selection described above to select a first grain. If the selected grain is within the threshold grainInterpolationDistThreshold from the target position, use this grain without doing any interpolation, otherwise continue with the next step.

- For each grain remaining in the set of candidate grains, assign a final weight, where the final weight includes an extra aspect that depends on if the grain is in a complementary position in the descriptor space relative to the first grain with respect to the target position. Grains that are in positions that are not complementary are given a low weight and therefore have a low probability to be chosen.

- Select a second grain from the set of candidate grains using the recalculated weights including the aspect of a complementary grain position.

- Select a length for the new, interpolated, grain. The selection of the length of the interpolated grain depends on whether the useInterpolationResampling is set to true or false. This is explained in a separate section below.

- If useInterpolationResampling is active, the selected grains are resampled to match the length of the new interpolated grain. If useInterpolationResampling is not active, only the first section, matching the length of the new interpolated grain, of each grain is used.

- Generate the new interpolated grain using a weighted mix of the selected grains where the weights depend on the distances of the respective grains to the target position.

Complementary position criterion

In step 2, to decide if a grain is in a complementary position to the first grain with respect to the target position, a criterion is evaluated where the position is seen as complementary only if it satisfies the following pseudo code:

bool positionIsComplementary = true;

if ((coordsA[D] <= targetCoord[D] - Thr) && (coordsB[D] <= targetCoord[D])) {

positionIsComplementary = false;

}

if ((coordsA[D] >= targetCoord[D] + Thr) && (coordsB[D] >= targetCoord[D]))

{

positionIsComplementary = false;

}

Where coordsA and coordsB are the coordinates of the two grains and Thr is set to the grainInterpolationDistThreshold. The evaluation is repeated for each dimension, D, of the database and the position is complementary only if the coordinates satisfy this condition for all dimensions.

Choice of the length of the interpolated grain

If the flag useInterpolationResampling is set, the length of the new interpolated grain is set as a weighted mean of the lengths of the two grains selected as basis for the interpolated grain where the weights are inversely proportional to the respective grains distance to the target position:

| L=w1L1w2L2 | (48) |
| --- | --- |

Where L is the length of the new interpolated grain, L1 and L2 are the lengths of the selected grains and w1 and w2 are weights that are calculated as

| w1=d2d1+d2 w2=d1d1+d2 | (49) |
| --- | --- |

Were d1 and d2 are the distances from respective grains to the target position. In some databases, only one, or some, of the dimensions should have an influence of the pitch, and therefore the length of the interpolated grain. Therefor only the dimensions that are selected with the parameter pitchDimension will be included in the calculations of the distances d1 and d2.  pitchDimension is a vector of booleans, where the dimensions that should be included are set to true.

The final interpolated grain, GINT, is calculated as a weighted sum:

| GINT=w1G'1w2G'2 | (50) |
| --- | --- |

where G’1 and G’2 are resampled versions of the original selected grains if resampling is enabled. If resampling is not enabled G’1 and G’2 are subsections of the original selected grains where only the first L samples are used.

The resampling is either done using simple, low complexity, linear interpolation or a higher quality resampling using a windowed sinc interpolation where the sinc function is windowed by a Hamming window:

| L(x)=HammingxSincx -a<x<a0 otherwise | (51) |
| --- | --- |

The variable a is called the kernel size and is set to 3, which gives a good trade-off between complexity and quality. The choice of resampling method is set with the parameter InterpolationMethod where 0 means that the low complexity interpolation is used and 1 means that the high-quality interpolation is used.

##### Optimized overlap-add window

When doing the overlap between two grains, an optimized overlap-add window needs to be selected. For grains that are highly correlated the grains will add up constructively, and therefore a linear overlap-add window, WL, is optimal:

| WL(x)=xX-1 | (52) |
| --- | --- |

where x = 0, …, X-1

On the other hand, for grains that are mostly uncorrelated a power preserving overlap-add window, WP ,  is optimal since the grains will not add up constructively.

| WP(x)=xX-1 | (53) |
| --- | --- |

To generate an optimized mixing window, WO, a mixing window coefficient, m, is used, which gives a continuous control over the mixing window, from linear to power preserving. WO is calculated as a weighted sum of the linear and power preserving windows, where the mixing window coefficient controls the weights:

| WO=mWP+(1-m)WL | (54) |
| --- | --- |

This means that when the mixing window coefficient is 1.0, a power preserving mixing window will be used and if the mixing window coefficient is 0.0, a linear mixing window will be used. Values in-between 0.0 and 1.0 will produce a window that is somewhere in-between a linear and power preserving window, which means that the mixing window can be optimized for grains that are only partly correlated.

A granular database may have clusters of grains that are highly correlated with each other, and other clusters of grains that are mostly uncorrelated with each other. The mixing window coefficient is derived directly from the grainMixRule parameter in the grain database. The grainMixRule can be specified for several positions in the descriptor space and the optimized mixing window for a particular grain is given by the grainMixRule that is specified in a point that is closest to the grain’s position.

Before starting the overlap-add between two grains, the respective mixing window coefficients are evaluated and the greatest of the two is used for deriving the optimal mixing window to be used for the overlap.

##### Fast grain switching

Normally, the scheduling of the next grain is triggered when reaching the overlap region of the current grain. In some cases, however, the grains are long and waiting to switch grains until the currently playing grain has been completed will result in a slow update where the target position may have changed greatly during the time a grain is completely rendered. This may lead to a generated sound output with large stepwise changes in character rather than a fast and smooth change. Using shorter grains would improve the dynamic response, but this is not always desirable since longer grains are sometimes needed to produce a natural sounding output. To avoid this problem, a fast grain switch can be triggered before the current grain has been completed.

A fast grain switch should be triggered when the target position has moved so much that the currently used grain is no longer close to the target, but as long as the currently used grain is still close to the target position a fast grain switch is not needed.

Before the rendering of a new audio buffer is started, a check is made if a fast grain switch should be triggered. This is done by calculating the distance from the target position to the position of the current grain and if this distance has increased more than the threshold fastGrainSwitchThreshold since the current grain was selected, a fast grain switch is triggered.

In the case that grain interpolation is used, the distance from the target position to a line between the two currently used grains is calculated, and if this distance has increased more than the threshold fastGrainSwitchThreshold since the two currently used grains were selected, a fast grain switch is triggered.

##### Addressing audio samples in an externally provided PCM signal

If the GranularAudioEncodingMode option is set to 1, the audio samples of each grain are not included in the bitstream. Instead, the audio samples of the grains are provided in a PCM signal that is made available to the renderer using some separate channel. The GranularSoundId and/or GranularDatabaseFilePath can be used to identify the PCM signal that correspond to the granular database.

For each grain in the bitstream, the parameter grainSampleIndex is specified. This index specifies at which sample index, within the externally provided PCM signal, the audio samples of a grain starts.

Using the grainSampleIndex together with the parameters GrainLen and numberOfChannels, the audio samples for each grain can be extracted from the PCM signal. The grainSampleIndex refers to an index in a 1-dimensional array where the channels are interleaved. This means that the index of the sample at time instant T within a grain, for channel k, is calculated as:

| SampleIndexT,k=grainSampleIndex +numberOfChannels∙T+k | (55) |
| --- | --- |

Where k is the index of the channel and is within the range [0, numberOfChannels-1] and T is within the range [0, GrainLen-1].

### Reverberation

#### General

The Reverb stage is used for rendering the diffuse late reverberation for each Acoustic Environment (AE). The Reverb stage is depicted in Figure 12.

The Reverb stage takes as input RIs which are of type Primary or HOA and which are not Inactive, Orphaned, or have the NoReverb authoring parameter set. The input audio samples are indicated by Input audio in Figure 12. The Reverb stage executes several Feedback Delay Network (FDN) reverberators in parallel, one for each currently active AE. These are referred to as Reverberators. The parameters for each FDN are optimized according to the reverberation characteristics of the corresponding AE. Each reverberator takes as direct input those audio elements which are located inside the AE which this reverberator is associated to, and audio elements from other AEs that contribute energy via portals. The Room Assignment stage produces the information of which audio elements are currently inside each AE and this is used for selecting the input signals for each reverberator.

Figure 12 — Overview of the Reverb Stage

The Reverb stage produces its output as a number of output RIs with their coordinates in a listener centric coordinate system. The output RIs contain the diffuse late reverberation signals as a mix of the currently audible AEs. This is depicted as Immersive reverberated signals in Figure 12. There are as many output RIs as there are delay lines in the largest FDN reverberator since the output signals are obtained from taps in the FDN delay lines. If the largest FDN reverberator has less than 15 delay lines, there shall be 15 output RIs. The reverberator output is rendered immersively enveloping the listener by positioning and rendering these delay line outputs nearly uniformly around the listener. The immersive reverberated signals are rendered to virtual loudspeaker output with the Panner stage. The virtual loudspeaker output of the Panner stage is auralized with the Spatializer.

Head tracked and non-head tracked rendering of the delay line outputs are supported. In head-tracked rendering mode, the reverberator output spatial positions are world orientation locked and in non-head-tracked rendering mode the spatial positions are locked to the listener head orientation. Smooth acoustic transitions when entering and exiting an AE are implemented with cross fading of the immersive reverberator output signals.

Reverberators associated with AEs having portals shall also provide their output to the Portal stage which renders the directional diffuse late reverberation from a connected AE through a portal opening. Coupling of acoustic environments and their reverberators is implemented by feeding the previous output buffer of a reverberator related to one environment of a portal as an input to the reverberator related to the other environment of the portal, scaled by a factor that depends on the dimensions of the portal opening.

The Reverb stage reads the payloadReverb (6.2.8) from the bitstream and initializes the Feedback Delay Network (FDN) reverberators for each AE using reverberator configuration parameters calculated from the reverberation parameters in the bitstream. The FDN configuration is the same both for Virtual Reality (VR) and Augmented Reality (AR) scenes. For rendering of AR scenes having sceneType (6.2.11) equal to “AR”, the FDN parameters are calculated based on the listener space description file (LSDF) description of the listening space AE, whose acoustic characteristics are defined as either reverberation parameters or a room impulse response (RIR), and control parameters from the bitstream.

The Reverb stage supports simultaneous rendering of reverberation from AEs in the bitstream and LSDF AE. Bitstream AEs and LSDF AE can also be connected via a portal. When the bitstream contains a portal with only one connected VR AE, a check shall be done to determine whether the center of the portal opening is within a threshold distance of 1m from the ARAnchor of the LSDF AE. If such a portal exists, a portal connection shall be established from the VR AE to the LSDF AE, enabling directional and coupled reverberation rendering via the portal.

The reverberation time of the FDN reverberator is controlled with the delay line attenuation filters and delays. The level and spectrum of reverberation is controlled by the reverberation energy ratio control filter and the source directivity gain filters. For AR scenes the configuration calculation routine selects between octave and third octave resolution based on the input data.

The FDN is configured with either 3, 7, 15, 31, or 63 delay lines. All delay line configurations are implemented in a computationally efficient way and with an optimized calculation of the feedback matrix operation using precalculated coefficient sequences. The reverberation quality, specifically echo density and diffuseness increases as the number of delay lines increases. Lower number of delay lines causes less echo density and diffuseness and provides low complexity operation.

#### Data elements and variables

DSRb	Diffuse to Source energy Ratio for frequency b, in the filter design frequency grid, for the predelay value in the bitstream.

DSRrender,b	Diffuse to Source energy Ratio for frequency b, in the filter design frequency grid, for the adjusted predelayRender value to be used in rendering.

delFltOrder	Filter order for the delay line attenuation filters.

dirFltOrder	Filter order for the directivity filters.

fcFltDes	Vector with frequencies for the VR filter design.

Hdel,d,b 	Target amplitude response for delay line attenuation filter d in frequency b.

Hdir,k,b 	Target amplitude response for directivity filter k in frequency b.

Hratio,b	Target amplitude response for the reverberation energy ratio control filter.

I		Number of elevations for a certain directivity pattern.

Ji		Number of azimuths for a certain directivity pattern, in its ith elevation.

md		Delay for delay line d in samples.

ratioFltOrder	Filter order for the ratio control filter.

reverbCount	Number of active reverberators.

slopeLimit	Steepest slope allowed in the VR EQ filter, in dB/octave.

slopeLimitPivotFreq	The starting frequency from which to perform slope limiting.

T60,b		RT60 value in seconds for frequency b, in the filter design frequency grid.

wi,j		Surface weighing factor for directivity gain.

maxNumberOfOutputChannels	Number of outputs from the FDN reverberators.

numDelayLines(r)	Number of delay lines in the reverberator r. The reverberators of different acoustic environments can have different number of delay lines.

B		Number of samples in a processing block.

headtrackedOutputRenderItems(j,n,azi,ele)

Immersive reverberation output signal for output channel j, sample index n, to be rendered at azimuth azi, elevation ele by the Panner stage.

predelay	Input predelay from the bitstream or LSDF file in seconds.

GEQdel,d,r	Delay line filter d for reverberator r.

GEQdir,k	Directivity gain control filter for directivity pattern k

GEQratio,r	Reverberation energy ratio control filter for reverberator r

predelayFactor	The ratio between bitstream/LSDF predelay and predelayRender to be used for rendering

predelayRender	The lag in the system’s IR at which reverberation rendering starts.

predelayAdjustedInSamples	The delay with which the reverberator’s predelay shall be configured in order to achieve the desired predelayRender.

distanceGainDropDb	Amount of source-listener distance dependent attenuation in decibels per distance doubling

minimumDistance	Minimum distance in meters within which source-listener distance no longer increases reverberation level

fadeInDistance	Cross-fade distance in meters for fading immersive reverberation when entering/exiting an AE

crossfade.enabled	Toggle cross-fade between AEs on/off

headTrackingEnabled	Toggle head tracking on/off for immersive reverberation output

speedOfSound	Speed of sound in ms for calculating predelay

lsdfRDRToActualRDRDb	Offset to be applied to RDR values in LSDF

overrideLsdfRdr	Force override of LSDF RDR values

thereticalRDRToActualRDR	Offset applied to RDR values calculated from an acoustic approximation based on RT60 values

G0		SOS filter global gain

a1,a2,b1,b2	SOS filter feedback and feedforward coefficients, respectively

levelDb	Level offset to be added to the SOS filter gain

stageF	Stage gain for SOS filter

zpredelayAdjusted	Delay for the predelay line

GEQmd	Attenuation filter for the delay line having delay md

A		FDN feedback matrix

maxDelayLineDelay	Delay of the longest FDN delay line in samples

minDelayLineDelay	Delay of the shortest FDN delay line in samples

delayLine[d]	dth delay line of the FDN

m_alpha	FDN feedback matrix calculation constant

boundingBoxWidth	Width of the AE region defining the virtual or physical room in m

boundingBoxHeight	Height of the AE region defining the virtual or physical room in m

boundingBoxDepth	Depth of the AE region defining the virtual or physical room in m

sourceContribution	Energy portion of the source that contributes to a specific other AE than where the source is located.

portalCenter	Center position of a portal, derived in the portal stage.

portalFactor	The portion of source energy reaching a portal, indicating the maximum amount of energy that may transfer into the acoustic environment at the other side of the portal, barring the effect of any portal material.

samplingRate	Sampling rate in Hz

controlGainsDb	Control gains in dB for GEQ design

optimizedGainsDb	Optimized gains in dB for GEQ design

Wi		Weighting matrix of the ith layer of the neural network used for GEQ design

xMin, xMax	Minimum and maximum input control gains for GEQ design NN, respectively

tMin, tMax	Minimum and maximum of training data targets for GEQ NN, respectively

thetai	Bias vector of the ith layer of the GEQ design NN

controlGainsDb	Control gains for GEQ design

fc1		Octave band center frequencies

fc3		Third octave band center frequencies

fs  or fs	The input signals’ sample-rate

sourceLocation	Location in Cartesian coordinates of the sound source to be rendered

distanceGain	Source-listener distance dependent reverberation gain

directivityBusAdded	Status variable indicating the count of sources added to a directivity bus

numBands	Number of bands in a SOS cascade filter

levelAndFactor	Combined overall gain for a SOS cascade filter

m_inputBuffer	FDN reverberator input buffer

srev,rj,n	Reverberated output signals for reverberator r, output channel j, sample index n. These are the input to the Portal stage.

srev,rn	Reverberated output signals for reverberator r, summed across all its output channels, sample index n. These are stored to be used as input for reverberators in connected acoustic environments. Acoustic environments connected via a portal are referred as connected acoustic environments.

reverbGain	Overall reverberation gain

∆a,i		Azimuthal width of a directivity cover at elevation i, in radians.

∆e,i		Elevational width of a directivity cover at elevation i, in radians.

θi		Elevation angle for directivity covers at elevation index i in radians.

delayMinMs	Minimum delay line length in milliseconds used in calculation of the reverberation delay line lengths.

delayRatio					Delay ratio used in calculation of delay line lengths.

itemReverbGain	Per-item reverberation gain

m_noLineOfSightCoeff	Multiplier applied to the energy which a source contributes via a portal connection when there is no line-of-sight from the source to the portal.

m_unrestrictedSor	If true, enables bidirectional connected reverberation rendering where reverberation energy is constantly passed to both directions via a portal. Supported for scenes where all reverberators have at least 15 delay lines. If false, one-directional connected reverberation is implemented where reverberated energy is fed only into the listener AE reverberator from connected AE reverberators. Listener-only connected reverberation is the default and the only option for scenes with reverberators configured with less than 15 delay lines.

#### Stage description

##### Stage initialization: common

###### General

The initialization of the Reverb stage is carried out before processing of the audio samples delivered by the decoder takes place. The initialization consists of several processing steps.

During audio processing, configuration data for the reverberation input gains is calculated dynamically, depending on in which room the listener is. The audio processing uses these static and dynamic configuration parameters to configure its components.

###### Run-time configuration of reverberation parameters

It is possible to override the following parameters relevant for reverberation parameterization from the config variables starting with the prefix “Reverb:” in the Renderer Framework local configuration parameters 6.4.2.6. If local configuration parameter value is not set, bitstream values are used when available, or default values when bitstream parameters do not exist.

Parameters related to AcousticEnvironmentData for bitstream or LSDF acoustic environments (the first available value shall be used from left to right:

predelayFactor = Reverb:PredelayFactor or 4.0 (VR) or Reverb:PredelayFactorAR or 1.0 (AR)

predelayOffset = Reverb:Predelay_Offset or 0.0

distanceGainDropDb = Reverb:DistanceGainDropDb or reverbDistanceGainDropDb or 1.5

minimumDistance = Reverb:MinimumDistance or reverbMinimumDistance or 1.0

reverbGain = Reverb:Gain or 1.0

When the Accessibility parameter reverbAttenuationDb is provided via the Accessbility User Interface (See Annex B.5), reverbGain is attenuated:  reverbGain *= pow(10,-0.05* reverbAttenuationDb).

Parameters related to defaultAcousticEnvironmentData and Default AE:

predelay = Reverb:Default_Acoustic_Environment_Predelay_Override or revDefaultAEPredelay or presetDefaultAEPredelay

predelayFactor = Reverb:PredelayFactorDefaultAE or 1.0

distanceGainDropDb = Reverb:DistanceGainDropDbDefaultAE or revDefaultAEDistanceGainDropDb or presetDefaultAEDistanceGainDropDb

minimumDistance = Reverb:MinimumDistanceDefaultAE or revDefaultAEMinimumDistance or presetDefaultAEMinimumDistance

reverbGain = Reverb:Default_Acoustic_Environment_Gain or revDefaultAEGain or presetDefaultAEGain

delayRatio  = Reverb:Default_Acoustic_Environment_Delay_Ratio or revDefaultAEDelayRatio or

presetDefaultAEDelayRatio

delayMinMs = Reverb:Default_Acoustic_Environment_Delay_Min_Ms_Override or revDefaultAEDelayMinMs or presetDefaultAEDelayMinMs

Parameters related to cross fading of immersive reverberation:

fadeInDistance = Reverb:Crossfade_Fade_In_Distance or reverbFadeInDistance or 1.0

crossfade.enabled = Reverb:Crossfade or reverbFadeInEnabled or true

Parameters related to enabling/disabling head tracking of immersive reverberation:

headTrackingEnabled =Reverb:HeadTrackingEnabled or reverbHeadtrackingEnabled or false

Parameters related to LSDF acoustic environment reverberator configuration:

speedOfSound = Reverb:SpeedOfSound or 343 m/s

lsdrRDRToActualRDR = Reverb:LSDF_RDRToActualRDR or lsdfRDRtoactualRDRdB or -8.0

overrideLSDF_RDR = Reverb:OverrideLSDF_RDR or overideLSDF_RDR or false

theoreticalRDRToActualRDR = Reverb:TheoreticalRDRToActualRDR or lsdfTheorecticalRDRtoactualRDRdB or -8.0

Parameter related to computational complexity and quality of reverb: for VR reverb configuration

numDelayLines = Reverb:numDelayLines or revDelayLineCount or 15

and for AR reverb configuration

numDelayLines = Reverb:numDelayLines or numDelayLinesFromComplexityLevel(complexityLevel) or 15

Parameter related to line-of-sight data from sources to portals:

m_noLineOfSightCoeff = Reverb:NoLineOfSightGain or 0.1

Parameter controlling whether bi-directional connected room reverberation is enabled or whether connected reverberation feeds only into the listener environment from environments connected to it:

m_unrestrictedSor = Reverb:SecondOrderReverbUnrestricted or false

###### Initializing reverberation for different types of AcousticEnvironments

The Reverb Stage initializes as many FDN reverberators as there are active AEs within the audio scene. Active AEs can be one of three types: AEs listed in the bitstream corresponding to virtual environments for VR, an AE from the LSDF for AR reproduction corresponding to a physical listening space, and a Default AE (also indicated in the bitstream) for background reverberation. Each AE is characterized by its id, and acoustic parameters RT60, DSR or RDR, and predelay (VR) or diffuseOnset (AR). Bounded AEs also have a region and thus dimensions. Optionally, a bitstream AE can have a reverberator delay line count signalled as the bitstream parameter revDelayLineCount and an LSDF AE a complexityLevel value which is then converted to the number of delay lines by the renderer. A reverberator is initialized for each active AE based on its dimensions and acoustic parameters to reproduce the reverberation of this AE. Thus, the number of reverberators reverbCount at maximum equals the number of bitstream AEs plus a possible LSDF AE.

The configuration and processing for different AEs has the following main characteristics:

—	Reverberation parameters for VR AEs obtained from the bitstream, whereas reverberation parameters for AR AEs are obtained from the LSDF.

—	Reverberation parameters for the Default AE are obtained either from the bitstream signalling the use of a preset defined in A.3 or by sending a full set of parameters.

—	Bitstream AEs with an associated region and LSDF AEs are bounded whereas the Default AE does not have boundaries. A bounded AE has a geometric region which defines the boundaries of the region within which the acoustic characteristics of the AE are valid. For the Default AE the geometric region is null.

—	The initialization and configuration of the reverberators is different for AEs in the bitstream and the LSDF. From the perspective of the Reverb stage the virtual acoustic characteristics for AEs in the bitstream and the acoustic characteristics of the AE from the LSDF are merged into a common scene state. Subsequently, the rendering of the reverberation for the bitstream and LSDF AEs is the same.

—	Since the Default AE is unbounded, the output gain processing differs for the Default AE and the bounded AEs to handle smooth transitions between reverberation from bounded AEs and the Default AE.

The Reverb stage initializes an FDN reverberator for each bounded AE based on the received reverberator payload information from the bitstream or the LSDF acoustic parameter information. To limit computational cost when there are many bounded AEs, the number of active reverberators shall be limited by the value of MAX_NUM_BOUNDED_AES_BEFORE_DROPPING_DEFAULT. This will limit the number of active  reverberators reverbCount to be at maximum equal to the number of bounded bitstream AEs plus a possible LSDF AE by disabling the Default AE if one exists and there are at least MAX_NUM_BOUNDED_AES_BEFORE_DROPPING_DEFAULT bounded AEs.

The method for conditionally disabling the defaultAE reverberator shall be as follows:

—	Determine whether an unbounded AE exists. An unbounded AE exists if there is a reverberator for which the AE enclosure pointer is null.

—	If an unbounded AE exists, determine if the number of reverberators reverbCount equals or exceeds  MAX_NUM_BOUNDED_AES_BEFORE_DROPPING_DEFAULT + 1. If both conditions are true, reduce reverbCount by one and do not initialize the reverberator for the defaultAE.

###### Initializing directivity gain control filters

The Reverb stage initializes as many source directivity gain control filters as there are directivity patterns in the scene. These filters are used in the Reverb stage to adjust the frequency-dependent reverberation level and spectrum for sources with directivity depending on the average source directivity pattern to all directions. The coefficients for the directivity gain control filters shall be obtained from the directivity patterns in payloadDirectivity of the bitstream. Whereas the original source directivity pattern contains frequency-dependent directivity gain data for different spatial directions, the directivity gain filter represents the average frequency-dependent gain over all directions. The filter’s target frequency response shall be obtained by averaging the frequency-dependent directivity pattern data over all directions according to:

| Hdir,k,b=14π∙i=1Ij=1Ji10cb,i,j10∙wi,j | (56) |
| --- | --- |

where b is the frequency index in the directivity pattern’s frequency grid, I= elCnt, Ji= aziCntPerEl[i], cb= cover (as defined in 6.6.16) for frequency index b, and

| wi,j=maxcosθi∙∆a,i∙∆e,i,0 | (57) |
| --- | --- |

with θi=π∙(i-1)I-1-0.5∙π,     ∆a,i=2πJi     and

| ∆e,i=π2∙I-1for i==1 or i==IπI-1otherwise | (58) |
| --- | --- |

Next, the raw target response Hdir,k,b shall be resampled to the filter design frequency grid, using the function defined in 6.6.4.3.2.2:

H_prime_dir[k] = resampleToGrid(fcDir[k], H_hat_dir[k], fcFltDes);

with Hdir,k,b'= H_prime_dir[k][b], fcDir[k] the frequency grid corresponding to directivity pattern k, Hdir,k,b= H_hat_dir[k][b] and fcFltDes the filter design grid defined in 6.6.4.3.2.2.

Finally, the raw target spectrum shall be smoothed using the function defined in 6.6.4.3.2.2:

H_dir[k] = smoothResponse(H_prime_dir[k], fcFltDes, 1);

with Hdir,k,b= H_dir[k][b].

The filter coefficients for a Second-Order-Section (SOS) cascade filter  shall be derived to model target frequency response Hdir,k,b using the algorithm described in clause 6.6.4.3.2.7 for VR scenes. The filter order is given by dirFltOrder.

The maximum error input argument to the algorithm shall be:

| εb=Hdir,k,b∙10maxDirErr20-1 | (59) |
| --- | --- |

where maxDirErr = 3 dB.

For AR scenes, a SOS cascade graphic EQ filter shall be designed to model the target frequency response Hdir,k,b using the algorithm described in clause 6.6.4.3.3.4.

The source directivity gain control filter for the directivity pattern k in the scene is denoted as GEQdir,k.

The Reverb stage shall initialize a directivity filter input bus for each directivity filter. The input buses are vectors equal to the block length B. Each bus is associated with the directivity filter parameters GEQdir,k and the directivity id (directivityId).

###### Initializing stage outputs

The  Reverb stage creates two kinds of output RIs: ItemType::Pannable and ItemType::Reverb.

ItemType::Pannable items shall contain the immersive reverberation signal for the current listener position. The number of ItemType::Pannable reverberation items shall equal maxNumberOfOutputChannels. They shall be set to have the listener coordinate space and the ForceNoDoppler and NoDistanceGain authoring parameters. The signals from all audible acoustic environments during crossfading shall be summed into ItemType::Pannable output RIs.

maxNumberOfOutputChannels = max(maxNumDelayLines, REVERB_MIN_NUMBER_OF_LOUDSPEAKERS)

where maxNumDelayLines is the largest number of delay lines in a reverb in the scene and REVERB_MIN_NUMBER_OF_LOUDSPEAKERS = 15.

Each reverberator’s output shall be given a total of numDelayLines(r) spatial positions provided in delayLineAzimuth and delayLineElevation. The procedure used for calculating the spatial positions of the delay lines depends on whether the reverberator represents a VR or AR environment, and is detailed in 6.6.4.3.2.3 and 6.6.4.3.3.2, respectively.

The RIs of type ItemType::Pannable, which aggregate signals from reverberators in the scene, shall have their spatial positions assigned by the method corresponding to the type of acoustic environments in the scene. That is, if the scene contains exclusively VR acoustic environments, the directions shall be assigned following the VR delay line directional encoding procedure. If the scene contains an AR acoustic environment (even if VR acoustic environments are also preset), the spatial positions shall be determined according to the AR delay line directional encoding procedure.

The reverberators r having their number of outputs numDelayLines(r) less than maxNumberOfOutputChannels shall have their output channel positions mapped to the closest output position of the reverberator having maxNumberOfOutputChannels output positions. The output mapping shall be done by calculating the Euclidean distances between the numDelayLines(r) output positions and maxNumberOfOutputChannels output positions on the unit sphere and obtaining for each numDelayLines(r) output position the index corresponding to the closest output position of the larger output position set. The variable outputMapping[numDelayLines] shall then contain the list of output channel indices for a reverb with numDelayLines delay lines. The creation of this mapping ensures that all reverberators can have their output positions as a subset of the output positions of the largest reverberator, which makes it possible to mix the immersive reverberator outputs during cross fading.

The output positions shall be headtracked based on listener orientation so that the output directions remain static when the listener's head turns if headTrackingEnabled = 1. If headTrackingEnabled = 0 (default) no head tracking shall be applied and output directions remain static with regard the listener head orientation.

The number of ItemType::Reverb items shall equal r=0reverbCount-1numDelayLines(r) and contain reverberation output delay line audio for acoustic environments regardless of whether the listener is located in the environment or not. These outputs shall be used by the Portal stage to render directional reverberation from portal openings.

##### Stage initialization: VR reverberator configuration

###### General

For virtual scene-based AcousticEnvironments the reverberator parameters are obtained from the bitstream (payloadReverb). An FDN reverberator, as depicted in Figure 13 shall be initialized for each AE identified by acousticEnvironmentId in payloadReverb.

Figure 13 — FDN reverberator

The steps in the FDN reverberator configuration are depicted in Figure 14. The configuration shall be done in this order since there are dependencies between the different control parameters.

Figure 14 — Steps in FDN reverberator configuration (VR)

###### Acoustic parameter pre-processing

For each acoustic environment, the corresponding acoustic parameters and other related bitstream data shall be processed before further configuration steps are applied to it.

For bounded environments, the dimensions of the acoustic environment shall be derived from the bounding box of the acoustic region.

boundingBoxWidth  = boundingBox.xMax – boundingBox.xMin;

boundingBoxHeight = boundingBox.yMax – boundingBox.yMin;

boundingBoxDepth  = boundingBox.zMax – boundingBox.zMin;

The frequency dependent RT60 and DSR parameters as conveyed in the bitstream shall be mapped to a fixed filter design frequency grid (fcFltDes).

RT60 = resampleToGrid(fcBs, revRT60, fcFltDes);

DSR  = resampleToGrid(fcBs, revDSR, fcFltDes);

where source frequency grid fcBs is the vector with the frequencies associated with the RT60 and DSR values from the bitstream. The target frequency grid fcFltDes shall be defined by the following procedure:

fcFltDes[0] = ERB2Hz(0.75);

fcFltDes[1] = ERB2Hz(2);

fcFltDes[2] = ERB2Hz(2);

b = 2;

for (e = 3.5; e <= 34; e += 0.25) {

b++;

fcFltDes[b] = ERB2Hz(e);

}

hfHop = (fs/2 – fcFltDes[b]) / floor((fs/2 – fcFltDes[b]) / 500);

while (fcFltDes[b] + hfHop < fs / 2) {

b++;

fcFltDes[b] = fcFltDes[b - 1] + hfHop;

}

where ERB2Hzerb=e0.11*erb-10.00437 converts ERB frequencies into Hz.

The mapping of the data with resampleToGrid shall be done according to the following procedure:

valOut = resampleToGrid(fcIn, val, fcTgt)

{

if (fcIn[0] != 0) {

fcIn = cat(0, fcIn);

valIn = cat(val[0], val);

}

if (fcIn[endIdx(fcIn)] != fs/2) {

fcIn = cat(fcIn, fs/2);

valIn = cat(val, val[endIdx(val)]);

}

valOut = interpolate(fcIn, valIn, fcTgt);

}

The function cat(vec1, vec2) concatenates two vectors, endIdx(vec) returns the last index of the vector and interpolate(fInVec, valVec, fTgtVec) performs a linear interpolation interpolating values from valVec corresponding with frequencies fInVec to a new set of values corresponding to the target frequencies in fTgtVec. I.e., for every target frequency fTgtVec[i] the two values in valVec corresponding to the first frequency in fInVec smaller and the first frequency larger than fTgtVec[i] will be interpolated linearly according to the relative distances of fTgtVec[i] to the selected frequencies from fInVec.

Next, the RT60 and DSR data shall be smoothed according to the following procedure:

if (mean(RT60) < 0.3) {

RT60 = smoothResponse(RT60, fcFltDes, 1);

}

else {

RT60 = smoothResponse(RT60, fcFltDes, 2);

}

RT60 = smoothResponse(RT60, fcFltDes, 3);

RT60[0] = mean(RT60[0], RT60[1], RT60[2]);

RT60[1] = RT60[0];

RT60[2] = RT60[0];

RT60[3] = mean(RT60[2], RT60[4]);

DSR[0] = mean(DSR[0], DSR[1], DSR[2]);

DSR[1] = DSR[0];

DSR[2] = DSR[0];

DSR[3] = mean(DSR[2], DSR[4]);

RT60_sm = smoothResponse(RT60, fcFltDes, 1);

DSR_sm = smoothResponse(DSR, fcFltDes, 1);

for (b = 0; b < 8; b++) {

RT60[b] = RT60_sm[b];

DSR[b] = DSR_sm[b];

}

The function mean(val1, val2, ...) shall return the mean of all provided values and smoothResponse() shall perform a smoothing according to:

valOut = smoothResponse(valIn, fIn, Noct)

{

nrFreq = length(fIn);

positiveVal = true;

for (b = 0; b != nrFreq; b++) {

valOut[b] = val[b];

if (valIn[b] < 0) {

positiveVal = false;

}

}

if (Noct > 0) {

for (b = 0; b != nrFreq; b++) {

if (fIn[b] > 0) {

g = gauss_f(fIn, fIn[b], Noct);

valOut[b] = 0;

for (b2 = 0; b2 != nrFreq; b2++) {

valOut[b] += g[b2] * valIn[b2];

}

}

}

if (positiveVal) {

for (b = 0; b != nrFreq; b++) {

if (valOut[b] < 0) {

valOut[b] = 0;

}

}

}

}

}

where

vecOut = gauss_f(fVec, Fc, Noct)

{

nrFreq = length(Fc);

sigma = (Fc / Noct) / PI;

sum = 0;

den = 2 * (pow(sigma, 2));

for (b = 0; b != nrFreq; b++) {

num = -pow((fVec[b] - Fc), 2);

sum += (vecOut[b] = exp(num / den));

}

for (b = 0; b != nrFreq; b++) {

vecOut[b] /= sum;

if (vecOut[b] < 1e-15) {

vecOut[b] = 0;

}

}

}

with PI= π, powx,  y=xy and expx=ex.

###### Delay lines (VR)

The reverberator delay line lengths delays[d] shall be initialized with the following procedure deriving delays from room dimensions.

minRoomDim = min(boundingBoxWidth, boundingBoxHeight, boundingBoxDepth);

maxRoomDim = max(boundingBoxWidth, boundingBoxHeight, boundingBoxDepth);

minDelay = max(minRoomDim * 2 / speedOfSound, 1e-3) * fs;

delayRatio = max(maxRoomDim / minRoomDim, 1.5);

nrLoops = numDelayLines;

base = pow(0.14623044588361, 1 / -(nrLoops - 1));

for (n = 0; n != nrLoops; n++) {

r[n] = pow(base, -n);

}

alpha = (r[0] - delayRatio * r[nrLoops - 1]) / (delayRatio - 1);

for (n = 0; n != nrLoops; n++) {

r[n] += alpha;

}

delays[nrLoops - 1] = minDelay;

for (n = nrLoops - 2; n != -1; n--) {

delays[n] = delays[nrLoops - 1] * r[n] / r[nrLoops - 1];

}

Next, each delay shall be mapped to the nearest prime number:

for (n = 0; n != nrLoops; n--) {

delayLine[n] = nearestPrime(delays[n]);

}

where p = nearestPrime(x) shall be a function that returns the prime number p with the smallest distance abs(x – p).

In the case of the Default AE, minDelay and delayRatio are not calculated from room dimensions but are provided directly by the preset data or from the bitstream data revDefaultAEDelayMinMs and revDefaultAEDelayRatio, as no room geometry is provided for a Default AE.

minDelay = decoded value of revDefaultAEDelayMinMs or presetDefaultAEDelayMinMs

delayRatio =  decoded value of revDefaultAEDelayRatio or presetDefaultAEDelayRatio

The delay lines are each assigned a spatial encoding direction. The following procedure shall be used for calculating spatial positions nearly uniformly distributed on the unit sphere:

outputDirections = vector[numDelayLines];

phi = M_PI * (3.0 - sqrt(5.0));

for (i = 0; i < numDelayLines; i++) {

y = 1.0 - (i / (numDelayLines - 1.0)) * 2.0;

radius = sqrt(1.0 - y * y);

theta = phi * i;

x = cos(theta) * radius;

z = sin(theta) * radius;

outputDirections[i] = cartesianToPolarCoordinates([-y, z, -x]);

}

where the procedure cartesianToPolarCoordinates converts a cartesian vector having the coordinates x, y, z into polar coordinates azimuth and elevation, and M_PI = π.

The reverberation delay lines are sorted into ascending order according to their length (delay[d]) and having indices d =[0, …, numDelayLines-1], shall be assigned the positions delayLineAzimuth[d] and delayLineElevation[d] with the following procedure:

i = 0;

for (d = 0; d < numDelayLines; d+=2) {

delayLineAzimuth[d], delayLineElevation[d] = round(outputDirections[i])

if ((d + 1) < numDelayLines) {

delayLineAzimuth[d+1], delayLineElevation[d+1] =

round(outputDirections[numDelayLines – 1 - i])

}

i += 1;

}

With the above procedure the shortest delay lines having more similar lengths shall be positioned far away from each other to ensure immersive reverberation rendering.

###### Predelay (VR)

The revPredelay value carried in the bitstream denotes the start of diffuse reverberation, from which DSR is calculated. This predelay value may be adjusted to a different value by the renderer application to be used for rendering using predelayFactor. The Reverb stage is used together with the Early Reflection stage to render complete reverberation for an audio scene. Whereas Reverb stage renders the late reverberation portion based on acoustic parameters of an environment, the Early Reflection stage renders discrete early reflections based on the acoustic material and geometry description parameters of the audio scene. Decoder-side changes to predelay shall be coordinated with the early reflection order to ensure that the early reflection portion and late reverberation portion blend together well.

Firstly, the predelay is divided by predelayFactor to get a predelay for starting diffuse reverberation rendering, as long as the result is equal or larger than the smallest delay line delay.

| predelayRender=maxpredelaypredelayFactor, minDelayLineDelayfs | (60) |
| --- | --- |

Next, the shortest delay line length shall be subtracted from predelayRender.

| predelayAdjustedInSamples=predelayRender*fs-minDelayLineDelay | (61) |
| --- | --- |

The predelay delayline module prior to the ratio filter and FDN structure shall be configured to apply a delay of predelayAdjustedInSamples to its input samples so that the FDN output starts after predelayRender seconds.

In case of the reverberator for the Default AE, the predelayFactor is always 1.

###### Delay line attenuation filters (VR)

The delay line attenuation filters are initialized using the acoustic parameters provided in payloadReverb.  Target spectra are calculated for all delay line attenuation filters after which the filter coefficients for the delay line attenuation filters are derived.

The filter target amplitude response for delay line d and frequency b shall be calculated as:

| Hdel,d,b=10-3∙mdfs∙T60,b | (62) |
| --- | --- |

Filter coefficients for a SOS cascade filter shall be derived to model the target frequency response Hdel,d,b using the algorithm described in clause 6.6.4.3.2.7. The filter order is given by delFltOrder. The frequency-dependent maximum error input argument to the algorithm shall be:

| εb=1+maxDelErr- Hdel,d,b∙0.07∙flen,b | (63) |
| --- | --- |

where maxDelErr = 0.01 and

| flen,b=1.5fcFltDesb≤10 Hz1.5-0.590∙b-1010<fcFltDesb≤100 Hz1100<fcFltDesb≤10,000 Hz1+1.5fs2-10,000∙b-10,00010,000<fcFltDesb≤fs2 Hz | (64) |
| --- | --- |

The global gain G0 of each SOS cascade filter shall be forced to be positive:

| G0=G0 | (65) |
| --- | --- |

The parameters of the SOS delay line attenuation filter d for reverberator r are initialized and denoted as GEQdel,d,r.

###### Reverberation ratio control filter (VR)

For each reverberator the reverberation ratio control filter performs adjustment of the reverberation level and spectrum based on the  DSR data from the  corresponding acoustic environment definition in the bitstream. Additionally, a calibration component is included to compensate for the configuration of the FDN.

A frequency dependent FDN energy estimate (Eb) shall be calculated to compensate for the effect of FDN configuration on reverberation level as follows:

| Eb=E0∙10-3∙2∙minDelayLineDelayT60,b∙fs2∙αb∙T60,bt60Cal | (66) |
| --- | --- |

where

| E0=offsetCal∙1000∙minDelayLineDelayfsdelayCal∙RdelayratioCal∙fs48000∙15numDelayLines2 Rdelay=maxDelayLineDelayminDelayLineDelay | (67) |
| --- | --- |

The three calibration coefficients shall be dependent on the number of delay lines in the corresponding reverberator.

| numDelayLines | t60Cal | offsetCal | delayCal | ratioCal |
| --- | --- | --- | --- | --- |
| 3 | 0.02277 | 26.2736 | -1.01887 | -0.79637 |
| 7 | 0.09588 | 90.2901 | -1.07545 | -0.87111 |
| 15 | 0.11870 | 225.3668 | -1.09620 | -0.89751 |
| 31 | 0.12366 | 523.4558 | -1.11467 | -0.90326 |
| 63 | 0.11445 | 1,237.1433 | -1.13477 | -0.93452 |

If the predelay value from the corresponding acoustic environment is modified from the value received in the bitstream, and the reverberator is not used for Default AE, the frequency dependent DSR shall be modified to compensate for the changed predelay. For each frequency index b, DSRb shall be modified as follows:

| DSRrender,b=DSRb∙e-2∙αb∙∆predelay | (68) |
| --- | --- |

where ∆predelay=predelayRender-predelay  and αb=ln10-6020T60,b.

In case the reverberator is used for Default AE ∆predelay shall be assumed 0.

The raw ratio control filter target spectrum shall be calculated as:

| Hratio,b=DSRrender,bEb | (69) |
| --- | --- |

Then, the raw filter target shall be smoothed if the mean RT60 is below 0.3 s, where the mean shall be calculated over all frequencies b.

if (mean(RT60) < 0.3) {

H_ratioRaw = smoothResponse(H_ratioRaw, fcFltDes, 1);

}

where H_ratioRaw[b] = Hratio,b.

The target spectrum shall subsequently be processed by a slope limiter.

H_ratio = responseSlopeLimit(H_ratioRaw, fcFltDes, slopeLimit,	slopeLimitPivotFreq);

where slopeLimit = 20 dB/octave and slopeLimitPivotFreq = 1000 Hz.

The slope limiter shall perform the following processing on its input spectrum and return an updated spectrum.

Y = responseSlopeLimit(X, fc, T, pivotFreq)

{

logFRatio[0] = 0;

for (b = 1; b < length(X); b++) {

logFRatio[b] = log2(fc[b] / max(fc[b – 1], 1));

}

bPivot = minidx(abs(fc – pivotFreq));

Y[bPivot] = X[bPivot];

for (b = bPivot + 1; b < length(X); b++) {

threshold = pow(10, T / 20 * logFRatio[b]);

desiredChange = X[b] / Y[b – 1] ;

if (desiredChange >= 1) {

change = min(abs(desiredChange), threshold);

}

else {

change = max(abs(desiredChange), 1 / threshold);

}

Y[b] = Y[b – 1] * change ;

}

for (b = bPivot – 1; b >= 0; b--) {

threshold = pow(10, T / 20 * logFRatio[b]);

desiredChange = X[b] / Y[b + 1] ;

if (desiredChange >= 1) {

change = min(abs(desiredChange), threshold);

}

else {

change = max(abs(desiredChange), 1 / threshold);

}

Y[b] = Y[b + 1] * change ;

}

}

where max(val1, val2) returns the largest of the two values, and max(vec) returns the largest element in vec, similarly min returns the smalles value, minidx(vec) returns the index of the smallest value in vec and abs(vec) returns the absolute value of each element in vec.

Finally, the common component will be removed from the ratio filter target.

commonComponentRatioFlt = mean(H_ratio);

for (b = 0; b < length(H_ratio); b++) {

H_ratio[b] /= commonComponentClrFlt;

}

The filter coefficients for the ratio control filter shall be derived with Hratio = H_ratio as the filter’s target spectrum, using the algorithm described in clause 6.6.4.3.2.7. The filter order is given by ratioFltOrder. The frequency-dependent maximum error input argument to the algorithm shall be:

| εb=Hratio,b∙1-10maxRatioErrb/20 | (70) |
| --- | --- |

where

| maxRatioErrb=-3-6*3-b4b≤3-3b>3 | (71) |
| --- | --- |

The reverberator’s ratio control filter for reverberator r shall be initialized with a cascade of SOS IIR filters using the values obtained from the filter design (6.6.4.3.2.7) on the target spectrum (Hratio,b) and is denoted with GEQratio,r for reverberator r.

The global gain G0 of the SOS cascade filter shall be forced to be positive:

| G0=G0 | (72) |
| --- | --- |

###### Filter design

The filters used in the reverberator processing depend on input data and are designed based on target spectra. The filter design algorithm used for designing the VR filters shall take the following inputs:

- Target amplitude spectrum Hb,

- Error margin per frequency εb,

- The frequencies corresponding to the values in Hb and εb.

- Filter order (shall be a multiple of 2).

The algorithm shall produce as output a filter definition for a cascade of Second-Order-Sections, with the following elements:

- G0 denoting a global broadband gain.

- a1,s denoting the first feedback coeffficient for each SOS.

- a2,s denoting the second feedback coefficient for each SOS.

- b1,s denoting the first feed-forward coefficient for each SOS.

- b2,s denoting the second feed-forward coefficient for each SOS.

These parameters shall form a filter that can be expressed as:

| GEQ(z)=G0∙s=0S1+b1,s∙z-1+b2,s∙z-21+a1,s∙z-1+a2,s∙z-2 | (73) |
| --- | --- |

where S indicates the number of SOSs.

The algorithm that shall be used is based on a minimax algorithm around a quasi-Newton optimization described below in pseudo code.

filterOpt = DesignIIR(H[], maxErr[], fc[], order)

{

errOpt = 2e14;

errOptFreqBand = 0;

filterOpt = 0;

maxSearchAttempts = 64;

maxIterations = 64;

nrSearchAttempts = 1;

minimaxSuccess = false;

stableFilter = true;

for (i = 0; i != length(fc); i++) {

fcNorm[i] = 2 * fc[i] / fs;

}

while ((((errOpt >= maxErr[errOptFreqBand]) && !minimaxSuccess)

|| !stableFilter) && nrSearchAttempts <= maxSearchAttempts) {

if ((nrSearchAttempts > 1) && (nrSearchAttempts < 20)) {

for (i = 0; i != length(fc); i++) {

maxErr[i] *= 1.05;

}

}

if (nrSearchAttempts > 1)

{

for (b = 0; b != length(H); b++) {

x[b] = -0.5 + rand();

}

}

else {

x = XSTART;

}

[filter, filterErr, filterErrIdx, minimaxSuccess] =

minimax(H, maxErr, fcNorm, order, x, maxIterations, fs);

if (filterErr < errOpt) {

errOpt = filterErr;

errOptIdx = filterErrIdx;

filterOpt = filter;

}

stableFilter = isStableFilter(filter);

nrSearchAttempts++;

}

}

where rand() produces a random floating point number based on a uniform distribution in the interval 0 to 1, and XSTART is defined in Annex A.4.

The minimax algorithm shall operate as follows:

[filter, err, errIdx, success] = minimax(H, maxErr, fc, order, x, maxIterations, fs)

{

err = 0;

errIdx = -1;

success = false;

minErrDiff = 1e-4;

prevIterErr = 1 << 16;

p = 2;

m = 2;

maxP = 64;

nrSOS = order / 2;

nrFreq = length(fc);

for (i = 0; i != nrFreq; i++) {

w[i] = fc[i] * PI;

}

for (i = 0; i != nrFreq; i++) {

if (fc[i] > 30000 / fs || fc[i] < 100 / fs) {

undershootIgnore[i] = 1;

}

else {

undershootIgnore[i] = 0;

}

}

for (iterIdx = 0; iterIdx != maxIterations; iterIdx++) {

x = quasiNewton(tgtMagn, w, order, x, p, maxIterations);

magnResp = getMagnitudeResponse(x, w, order);

for (i = 0; i != nrFreq; i++) {

e[i] = magnResp[i] - tgtMagn[i];

if (undershootIgnore[i]) {

e[i] = max(e[i], 0.0);

}

}

for (i = 0; i != nrFreq; i++) {

errNorm[i] = pow(e[i] / maxErr[i], 2);

}

errNormWorstBand = -1;

for (i = 0; i != nrFreq; i++) {

if (errNormWorstBand < errNorm[i]) {

errNormWorstBand = errNorm[i];

errIdx = i;

}

}

err = abs(e[errIdx]);

num = 0;

den = 0;

for (i = 0; i != nrFreq; i++) {

num += e[i] * e[i];

den += maxErr[i] * maxErr[i];

}

errNormTotal = num / den;

if ((err < maxErr[errIdx])

|| (errNormWorstBand < 1.2 && errNormTotal < 0.02)

|| (abs(err - prevIterErr) < minErrorDiff)

|| (iterIdx >= maxIterations))

{

if ((err < maxErr[errIdx])

|| (errNormWorstBand < 1.2 && errNormTotal < 0.02)) {

success = true;

}

h0 = x[(2 * order)];

for (sosIdx = 0; sosIdx != nrSOS; sosIdx++) {

idx0 = sosIdx * 4;

bSos[0] = 1;

bSos[1] = x[idx0];

bSos[2] = x[idx0 + 1];

bComplex = false;

rootage = x[idx0 + 2] * x[idx0 + 2] - 4 * x[idx0 + 3];

if (rootage < 0) {

rootage = -rootage;

bComplex = true;

}

K = sqrt(rootage);

if (bComplex) {

p1r = -x[idx0 + 2] / 2;

p1i = K / 2;

} else {

p1r = (-x[idx0 + 2] + K) / 2;

p1i = 0;

}

if (bComplex) {

p2r = -x[idx0 + 2] / 2;

p2i = -K / 2;

} else {

p2r = (-x[idx0 + 2] - K) / 2;

p2i = 0;

}

if ((p1r * p1r) + (p1i * p1i) > 1) {

tmpr = p1r / (p1r * p1r + p1i * p1i);

tmpi = p1i / (p1r * p1r + p1i * p1i);

p1r = tmpr;

p1i = tmpi;

if (!bComplex) {

h0 *= p1r;

}

}

if ((p2r * p2r) + (p2i * p2i) > 1) {

tmpr = p2r / (p2r * p2r + p2i * p2i);

tmpi = p2i / (p2r * p2r + p2i * p2i);

p2r = tmpr;

p2i = tmpi;

if (bComplex) {

h0 *= (p2r * p2r + p2i * p2i);

}

else {

h0 *= p2r;

}

}

aSos[0] = 1;

aSos[1] = -(p1r + p2r);

aSos[2] = (p1r * p2r) - (p1i * p2i);

x[order * 2] = h0;

x[idx0 + 2] = aSos[1];

x[idx0 + 3] = aSos[2];

filter.b[0][sosIdx] = bSos[1];

filter.b[1][sosIdx] = bSos[2];

filter.a[0][sosIdx] = aSos[1];

filter.a[1][sosIdx] = aSos[2];

}

filter.G0 = h0;

break;

}

else {

p *= m;

if (p > maxP) {

p = maxP;

}

}

prevIterErr = err;

}

}

The quasiNewton function shall operate as follows:

x = quasiNewton(H, w, order, x, p, maxIterations)

{

nrCoeffs = (order * 2) + 1;

e1 = 1e-5;

e2 = 1e-6;

m = 0;

Mh = 600;

r = 0.1;

s = 0.7;

t = 0.1;

c = 0.75;

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

if (i == j) {

Sk[i][i] = 1.0;

}

else {

Sk[i][j] = 0.0;

}

}

}

fk = getObjFunc(H, w, order, x, p);

gk = getObjFuncGrad(H, w, order, x, p);

m += 2;

fk_prev = fk;

dfk = fk;

for (k = 0; k != maxIterations; k++) {

for (i = 0; i != nrCoeffs; i++) {

acc = 0;

for (j = 0; j != nrCoeffs; j++) {

acc += Sk[i][j] * gk[j];

}

dk[i] = -acc;

}

aL = 0;

aU = pow(2.0, 20);

for (i = 0; i != nrCoeffs; i++) {

xIn[i] = x[i] + aL * dk[i];

}

gradOut = getObjFuncGrad(H, w, order, xIn, p);

dfL = 0.0;

for (i = 0; i != nrCoeffs; i++) {

dfL += (gradOut[i] * dk[i]);

}

ak = 1;

if (abs(dfL) > e2) {

ak = -2 * (dfk / dfL);

}

if ((ak <= 0) || (ak > 1)) {

ak = 1;

}

fL = fk;

while (m < Mh) {

for (i = 0; i != nrCoeffs; i++) {

deltak[i] = ak * dk[i];

}

for (i = 0; i != nrCoeffs; i++) {

xi[i] = x[i] + deltak[i];

}

fk = getObjFunc(H, w, order, xi, p);

m++;

if ((fk > (fL + r * (ak - aL) * dfL)) && (abs(fL - fk) > e2)

&& (m < Mh)) {

if (ak < aU) {

aU = ak;

}

ah0 = aL + (0.5 * (((ak - aL) * (ak - aL)) * dfL) /

((fL – fk) + ((ak – aL) * dfL)));

ah0L = aL + (t * (aU – aL));

if (ah0 < ah0L) {

ah0 = ah0L;

}

ah0U = aU – (t * (aU – aL));

if (ah0 > ah0U) {

ah0 = ah0U;

}

ak = ah0;

continue;

}

for (i = 0; i != nrCoeffs; i++) {

xIn[i] = x[i] + ak * dk[i];

}

gradOut = getObjFuncGrad(H, w, order, xIn, p);

dfk = 0;

for (i = 0; i != nrCoeffs; i++) {

dfk += gradOut[i] * dk[i];

}

m++;

if ((dfk < (s * dfL)) && (abs(fL – fk) > e2) && (m < Mh)) {

da0 = ((ak – aL) * dfk) / (dfL – dfk);

if (da0 <= 0.0) {

ah0 = 2.0 * ak;

}

else {

ah0 = ak + da0;

}

ah0U = ak + (c * (aU – ak));

if (ah0 > ah0U) {

ah0 = ah0U;

}

aL = ak;

fL = fk;

dfL = dfk;

ak = ah0;

continue;

}

break;

}

for (i = 0; i != nrCoeffs; i++) {

x[i] += deltak[i];

}

dfk = fk_prev – fk;

fnorm = norm(deltak);

if (((fnorm < e1) && (abs(dfk) < e1)) || (m >= Mh)) {

break;

} else {

m = m;

}

fk_prev = fk;

for (i = 0; i != nrCoeffs; i++) {

gk_prev[i] = gk[i];

}

gk = getObjFuncGrad(H, w, order, x, p);

for (i = 0; i != nrCoeffs; i++) {

gammak[i] = gk[i] – gk_prev[i];

}

D = 0;

for (i = 0; i != nrCoeffs; i++) {

D += (deltak[i] * gammak[i]);

}

if (D <= 0) {

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

if (i == j) {

Sk[i][j] = 1;

}

else {

Sk[i][j] = 0;

}

}

}

} else {

nfactor = 0;

for (i = 0; i != nrCoeffs; i++) {

nfactor += (deltak[i] * gammak[i]);

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM1[i][j] = deltak[i] * gammak[j];

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

acc = 0;

for (l = 0; l != nrCoeffs; l++) {

acc += MM1[i][l] * Sk[l][j];

}

MM4[i][j] = acc;

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM1[i][j] = gammak[i] * deltak[j];

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

acc = 0;

for (l = 0; l != nrCoeffs; l++) {

acc += Sk[i][l] * MM1[l][j];

}

MM2[i][j] = acc;

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM1[i][j] = MM4[i][j] + MM2[i][j];

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM2[i][j] = deltak[i] * deltak[j];

}

}

for (i = 0; i != nrCoeffs; i++) {

acc = 0;

for (j = 0; j != nrCoeffs; j++) {

acc += Sk[i][j] * gammak[j];

}

VV1[i] = acc;

}

int1 = 0;

for (i = 0; i != nrCoeffs; i++) {

int1 += gammak[i] * VV1[i];

}

int1 /= nfactor;

int1 += 1.0;

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM4[i][j] = MM2[i][j] * int1;

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

MM4[i][j] = MM4[i][j] – MM1[i][j];

}

}

ratio = 1 / nfactor;

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

C[i][j] = (MM4[i][j] * ratio);

}

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrCoeffs; j++) {

Sk[i][j] = Sk[i][j] + C[i][j];

}

}

}

}

}

where norm(v) returns the norm of v, v=ivi2,  break exits the innermost while or for loop enclosing it and continue exits only the active iteration of the innermost while or for loop enclosing it.

The function isStableFilter shall test the resulting filter for stability as follows:

isStable = isStableFilter(filter)

{

isStable = true;

maxPoleAmpl = 0.995;

for (sosIdx = 0; sosIdx != filter.order / 2; sosIdx++) {

a0 = filter.a[0][sosIdx];

a1 = filter.a[1][sosIdx];

discr = a0 * a0 - 4 * a1;

if (discr < 0) {

if (sqrt(a1) > maxPoleAmpl) {

isStable = false;

}

} else {

if ((abs(0.5 * (-a0 - sqrt(discr))) > maxPoleAmpl)

|| (abs(0.5 * (-a0 + sqrt(discr))) > maxPoleAmpl)) {

isStable = false;

}

}

}

}

The function getMagnitudeResponse shall return the magnitude response corresponding to the provided filter coefficients, as follows:

Hx = getMagnitudeResponse(x, w, order)

{

nrSos = order / 2;

nrCoeffs = (order * 2) + 1;

nrFreq = length(w);

for (i = 0; i != nrFreq; i++) {

Hx[i] = x[nrCoeffs - 1];

}

for (freqIdx = 0; freqIdx != nrFreq; freqIdx++) {

cosw = cos(w[freqIdx]);

cos2w = cos(w[freqIdx] * 2);

for (sosIdx = 0; sosIdx != nrSos; sosIdx++) {

idx0 = 4 * sosIdx;

b0 = x[idx0];

b1 = x[idx0 + 1];

a0 = x[idx0 + 2];

a1 = x[idx0 + 3];

Num = (1.0 + ((b0 * b0) + ((b1 * b1)

+ ((2.0 * (b0 * ((1.0 + b1) * cosw))) + (2.0 * (b1 * cos2w))))));

Den = (1.0 + ((a0 * a0) + ((a1 * a1)

+ ((2.0 * (a0 * ((1.0 + a1) * cosw))) + (2.0 * (a1 * cos2w))))));

if ((Num < 0) || (Den < 0)) {

if (Num < 0) {

Num = 0;

}

if (Den < 0) {

Den = 0;

}

}

Den = max(Den, 1e-10);

Hx[freqIdx] *= sqrt(Num / Den);

}

}

}

The function getObjFunc shall return the result of the objective function (i.e. the function to be minimized) at x as follows:

fk = getObjFunc(H, w, order, x, p)

{

nrFreq = length(w);

Mag = getMagnitudeResponse (x, w, order);

for (i = 0; i != nrFreq; i++) {

Mag[i] -= H[i];

}

Eh = 0;

sumE = 0;

for (i = 0; i != nrFreq; i++) {

delta = abs(Mag[i]);

if (delta > Eh) {

Eh = delta;

}

sumE += delta;

}

oc = 0.0;

for (i = 0; i != nrFreq; i++) {

delta = abs(Mag[i]);

r = 1.0;

for (j = 0; j < p; j++) {

r = r * (delta / (Eh + 1.0e-8));

}

oc += r;

}

o = pow(oc, 1.0 / p);

o *= Eh;

return o;

}

The function getObjFuncGrad shall return the objective function gradient at x as follows:

g = getObjFuncGrad(H, w, order, x, p)

{

nrFreq = length(w);

nrCoeffs = (order * 2) + 1;

Mag = getMagnitudeResponse(x, w, order);

for (i = 0; i != nrFreq; i++) {

e[i] = Mag[i] - H[i];

}

ge = getErrorGradient(Mag, e, w, order, x);

Eh = 0;

sumE = 0;

for (i = 0; i != nrFreq; i++) {

delta = abs(e[i]);

if (delta > Eh) {

Eh = delta;

}

sumE += delta;

}

oc = 0.0;

for (i = 0; i != nrFreq; i++) {

delta = abs(e[i]) / Eh;

r = 1.0;

for (j = 0; j < p; j++) {

r = r * delta;

}

oc += r;

}

o = pow(oc, (1.0 / p) - 1.0);

for (i = 0; i != nrFreq; i++) {

v1[i] = pow((abs(e[i]) / Eh), (p - 1.0));

}

for (i = 0; i != nrCoeffs; i++) {

for (j = 0; j != nrFreq; j++) {

prod[i][j] = v1[j] * ge[i][j];

}

}

for (i = 0; i != nrCoeffs; i++) {

sum = 0;

for (j = 0; j != nrFreq; j++) {

sum += prod[i][j];

}

v3[i] = sum;

}

for (i = 0; i != nrCoeffs; i++) {

g[i] = o * v3[i];

}

}

The function getErrorGradient shall return the error gradient at x as follows:

ge = getErrorGradient(H, e, w, order, x)

{

nrFreq = length(w);

nrSos = order / 2;

nrCoeffs = (order * 2) + 1;

eps = 1.0e-10;

for (i = 0; i != nrFreq; i++) {

if (e[i] >= 0) {

s[i] = 1;

}

else {

s[i] = -1;

}

}

for (i = 0; i != nrFreq; i++) {

ge[nrCoeffs - 1][i] = (1 / x[nrCoeffs - 1]) * (s[i] * H[i]);

}

for (fIdx = 0; fIdx != nrFreq; fIdx++) {

cosw = cos(w[fIdx]);

cos2w = cos(w[fIdx] * 2);

for (sosIdx = 0; sosIdx != nrSos; sosIdx++) {

idx0 = 4 * sosIdx;

b0 = x[idx0];

b1 = x[idx0 + 1];

a0 = x[idx0 + 2];

a1 = x[idx0 + 3];

Num = 1.0 + ((b0 * b0) + ((b1 * b1)

+ ((2.0 * (b0 * ((1.0 + b1) * cosw))) + (2.0 * (b1 * cos2w)))));

Den = 1.0 + ((a0 * a0) + ((a1 * a1)

+ ((2.0 * (a0 * ((1.0 + a1) * cosw))) + (2.0 * (a1 * cos2w)))));

Num = max(Num, eps);

Den = max(Den, eps);

ge[idx0][fIdx] = ((s[fIdx] * (b0 + ((1 + b1) * cosw))) / Num) * H[fIdx];

ge[idx0 + 1][fIdx] = ((s[fIdx] * (b1 + ((b0 * cosw) + cos2w))) / Num)

* H[fIdx];

ge[idx0 + 2][fIdx] = ((-s[fIdx] * (a0 + ((1 + a1) * cosw))) / Den)

* H[fIdx];

ge[idx0 + 3][fIdx] = ((-s[fIdx] * (a1 + ((a0 * cosw) + cos2w))) / Den)

* H[fIdx];

}

}

}

##### Stage initialization: AR reverberator configuration

###### General

Whereas for VR scenes the acoustic parameters for AcousticEnvironments are provided in payloadReverb, for AR scenes the acoustic parameters are obtained at the renderer during playback, based on the LSDF. The acoustic parameters shall either be provided in the LSDF or be analyzed from a room impulse response (RIR) specified in the LSDF. Non-iterative filter design method using neural network acceleration for the calculation of the ratio control filter and delay line attenuation filter parameters shall be used when the target response is known from the LSDF acoustic parameters, or when the target response is analyzed from a RIR specified in the LSDF.

Figure 15 depicts an overview of the steps in FDN reverberator configuration for AR.

Figure 15 — Steps in FDN reverberator configuration (AR)

###### Delay lines (AR)

If complexityLevel has been specified in the LSDF, it shall be converted to numDelayLines as follows:

int numDelayLinesFromComplexityLevel(complexityLevel) {

switch(complexityLevel) {

case 1: return 3;

case 2: return 7;

case 3: return 15;

case 4: return 31;

case 5: return 63;

default: return 15;

}

}

Lenghts of numDelayLines delay lines shall then be calculated.

The delay line lengths shall be determined based on room dimensions. The Cartesian dimensions boundingBoxWidth, boundingBoxHeight, and boundingBoxDepth of a bounding box enclosing the AE are used.

First, all room modes permutations (except for mode [0,0,0]) shall be generated up to a maximum mode index maxMode by:

maxMode = 2;

vector<array<3>> modePermutations;

for (i=0; i <= maxMode; ++i) {

for (int j=0; j <= maxMode; ++j) {

for (k = 0; k <= maxMode; ++k) {

if (i > 0 || j > 0 || k > 0) // don't use mode [0,0,0]

modePermutations.append({i,j,k});

}

}

}

Delays for all mode permutations shall then calculated, in samples, then sorted in ascending order as:

numModeDelays = (pow(maxMode+1, 3)) - 1; // -1 to skip mode [0,0,0]

vector modeDelays(numModeDelays); // unit: samples

for (i = 0; ires < numModeDelays; ++ires) {

xMode = modePermutations[i][0] / roomW;

yMode = modePermutations[i][1] / roomH;

zMode = modePermutations[i][2] / roomD;

resonanceFreq = 343.f/2.f * sqrt(xMode*xMode + yMode*yMode + zMode*zMode);

modeDelays[i] = fs / resonanceFreq;

}

sort(modeDelays.begin(), modeDelays.end());

Log-spaced delays shall then be generated between the frequencies corresponding to the minimum and maximum mode frequencies as follows:

vector logDelays(numDelayLines); // unit: samples

minResFreq = fs / modeDelays.back();

maxResFreq = fs / modeDelays.front();

// clamp min and max frequency between 5 and 300 Hz

minResFreq = max(5.0, minResFreq);

maxResFreq = min(300.0, maxResFreq);

start = fs / maxResFreq;

end = fs / minResFreq;

base = end / start;

for (i = 0; i < numDelayLines; ++i) {

logDelays[i] = start * pow(base, (float)(i) / (numDelayLines - 1));

}

The log-spaced delays shall then be adjusted to match nearby delays corresponding to the generated mode frequencies as follows:

vector nearestLogDelIdx(numModeDelays);

// For each mode (resonant) delay, find the index of the nearest log-spaced delay

for (ires = 0; ires < numModeDelays; ++ires) {

resDel = modeDelays[ires];

nearestIdx = 0;

minDist = LARGE_VALUE;

for (ilog = 0; ilog < numDelayLines; ++ilog) {

dist = abs(logDelays[ilog] - resDel);

if (dist < minDist) {

minDist = dist;

nearestIdx = ilog;

}

}

nearestLogDelIdx[ires] = nearestIdx;

}

// Assign the log-spaced delay the value of the nearest mode delay, if available

for (ilog = 0; ilog < numDelayLines; ++ilog) {

logDelay = logDelays[ilog]; // log-spaced delay to be (possibly) replaced

minDist = LARGE_VALUE;

for (ires = 0; ires < numModeDelays; ++ires) {

if (nearestLogDelIdx[ires] == ilog) {

modeDelay = modeDelays[ires];

dist = abs(logDelay - modeDelay);

if (dist < minDist) {

minDist = dist;

logDelays[ilog] = modeDelay;

}

}

}

}

The delays shall be sorted into ascending order. The delays shall be converted to mutually prime integers so that they do not have common factors. The length md for the dth delay line shall be obtained as

md = nearestPrime(delays[i]);

The attenuation filter coefficients in the delay lines are adjusted so that a desired amount in decibels of attenuation happens at each signal recirculation through the delay line so that the desired RT60(f) time is obtained. This is done in a frequency specific manner to ensure the appropriate rate of decay of signal energy at specified frequencies f.

The input to the attenuation filter configuration are the desired RT60 times per specified frequencies f denoted as RT60(f) and the delay line lengths md. For a frequency f, the desired attenuation per signal sample shall be calculated as attenuationPerSample(f) = -60 / (samplingRate * RT60(f)). The attenuation in decibels for a delay line of length md shall then be attenuationDb(f) = md * attenuationPerSample(f). Linear attenuation shall then be obtained as attenuation(f) = pow(10, attenuationDb(f) / 20). The attenuation values shall first be mapped to octave or third octave band center frequencies as described in 6.6.4.3.3.3 and then a cascade SOS filter shall be designed as described in 6.6.4.3.3.4.

Directions for reproducing output signals of the reverberator around the listener position shall be assigned to each delay line. The directions shall correspond to a spherical t-design with a number of points equal to the number of delay lines. The directions for the supported numbers of delay lines are tabulated in A.20. The direction assigned to each delay line shall be determined as follows.

Cartesian unit vectors representing spherical directions shall each be scaled by a Cartesian vector of the room dimensions divided by two. I.e. the spherical direction vectors shall be scaled to form an ellipsoid approximately fitted to the bounding box of the acoustic environment. The indices of the list of these “ellipsoidal” vectors shall then be sorted in the order of ascending vector lengths. Reproduction directions shall then be assigned to each delay line by iterating through the delay lines, in order of ascending delay line length, assigning a (spherical) direction by the indices of the sorted (ellipsoidal) vectors. Thereby, directions shall be assigned, in order of shortest-to-longest ellipsoidal directional vector length, to delay lines in order of ascending delay line length. As a result, the early response of the FDN will more closely reflect the temporal distribution of middle-stage early reflections of the Acoustic Environment.

###### Calculating input frequency response at band center frequencies

The frequency dependent response values frequencyResponse(f) shall be mapped to a set of frequency bands m, which are either octave or third octave bands, depending on the resolution at which the input data frequencyResponse(f) has been provided. Each value in frequencyResponse(f) shall first be mapped to third octave band to obtain data frequencyResponse (fc3), where fc3 are the center frequencies of a third octave filter bank. The third octave center frequencies fc3 shall be as defined in the ISO 266 (Acoustics — Preferred frequencies).

Mapping input data frequencyResponse (f) to the third octave center frequencies fc3 shall be done by finding for all fc3 the input frequency f having the smallest absolute difference to fc3. This shall result in third octave mapped frequencyResponse data frequencyResponse (fc3):

| frequencyResponsefc3=frequencyResponseargminffc3-f | (74) |
| --- | --- |

A set of inputIndices(fc3) shall be formed indicating the index i of the closest frequency f∈f1,f2,…,fi,…,fF for each fc3:

| inputIndicesfc3=argminifc3-fi | (75) |
| --- | --- |

A determination shall then be performed whether using the third octave resolution is necessary given the resolution of the input data. Third octave resolution shall be determined to be necessary if at least six adjacent frequency points fi map to different bands fc3 (somewhere in the spectrum). This shall be indicated by a trail of six adjacent ones in diff(inputIndicesfc3) where diff denotes the difference between adjacent values. If such a trail of adjacent ones is found then the third octave resolution frequencyResponse data frequencyResponse(fc3) shall be used.

Otherwise octave band data shall be formed as

| frequencyResponsefc1=frequencyResponseargminffc1-f | (76) |
| --- | --- |

where fc1 shall be the octave center frequencies defined in ISO 266.

The determination above shall result in frequency mapped frequencyResponse values frequencyResponse(m) where m are the indices of the selected frequency bands (octave or third octave bands).

###### Calculating SOS cascade graphic equalizer coefficients

The parameters for a SOS cascade graphic equalizer are:

—	G0 denoting a global broadband gain.

—	a1 denoting the first feedback coefficient for each SOS.

—	a2 denoting the second feedback coefficient for each SOS.

—	b1 denoting the first feed-forward coefficient for each SOS.

—	b2 denoting the second feed-forward coefficient for each SOS.

—	levelDb denoting a second global broadband gain.

—	stageF denoting a broadband gain for each SOS.

The SOS cascade graphic equalizer (GEQ) design shall receive as an input a target magnitude response frequencyResponse(m) at octave or third octave bands. The frequencyResponse(m) shall first be preprocessed such that values equal to zero are substituted with the smallest value larger than zero in frequencyResponse(m) to prevent numerical problems in the filter coefficient calculation. This creates the zero-fixed input target magnitude response zeroFixedInput(m).

Target response in decibels shall then be calculated as controlGainNoOffsetDb(m) = 20*log10(zeroFixedInput(m)).

The mean attenuation in decibels shall then be subtracted from the target response controlGainNoOffsetDb(m). The mean shall be stored as levelDb = -mean(zeroFixedInput(m)) and combined to the overall GEQ gain G0 during filter calculation. The mean removal is used to limit the range of control gains that need to be approximated with the response of the GEQ filter. The target response or control gains for the GEQ design shall be the mean removed response controlGainsDb(m) = controlGainNoOffsetDb(m) + levelDb.

The optimization of the GEQ parameters is done in a computationally efficient manner utilizing acceleration data in the form of neural network coefficients. The neural network has been trained to optimize the GEQ filter gains in decibels based on the input control gains in decibels. For octave band filters, the description follows Välimäki [1] and for third octave band filters Rämö [2].

For octave band filters, the optimal gains in decibels gains shall be obtained as:

g = 2.0 * hadamardDivision(controlGainsDb - xMin, xMax - xMin) - 1;

o = tanh(W1 * g + theta1);

go = W2 * o + theta2;

optimizedGains = hadamard((tMax - tMin), 0.5 * (go + 1.0)) + tMin;

where hadamardDivision performs an element wise division and tanh implements the hyperbolic tangent function which is the hidden layer activation function. The neural network parameters are the parameters of a one-layer fully-connected feedforward neural network including the maximum and minimum of the input control gains xMin and yMin, respectively, the weighting matrices W1 and W2, bias vectors theta1 and theta2, and the maximum and minimum of the training data targets tMax and tMin, respectively. The neural network parameters provided in A.5 shall be used.

Based on the optimized gains optimizedGains, the parameters of the SOS band filters shall be obtained. The frequency response of a cascade graphic EQ with M bands is

| HejωTs=G0m=1MHmejωTs | (77) |
| --- | --- |

where G0 is an overall gain factor. HejωTs are the frequency responses of equalizing filters, ω is the radial frequency and Ts=1/samplingRate is the sample interval.

A free design parameter c=0.3 shall be set. The number of bands shall equal M=10. The center frequency of the filters in radians shall be ωc,m=2πfcm/samplingRate where fcm is the filter center frequency at band m in Hz. The bandwidth of each band filter shall be obtained as Bm=1.5ωc,m. The following corrections shall be made to the bandwidths of three highest band filters: B8=1.395ωc,8, B9=1.170ωc,9 and B10=0.760ωc,10.

The transfer function of the GEQ filter at band m is

| Hmz=b0,m1+b1,mz-1+b2,mz-21+a1,mz-1+a2,mz-2 | (78) |
| --- | --- |

where the scaling coefficient shall be defined as

| b0,m=1+Gmβm1+βm | (79) |
| --- | --- |

where Gm is the target filter gains, Gm=db2mag(gains(m)) and βm shall be defined as

| βm=GB,m2-1Gm2-GB,m2 tanBm2 | (80) |
| --- | --- |

when Gm≠1 or as

| βm= tanBm2 | (81) |
| --- | --- |

when Gm=1.

The gain at bandwidth GB,m shall be set as GB,m= db2mag (c*Gm).

The numerator coefficients shall be

| b1,m=-2cosωc,m1+Gmβm | (82) |  |
| --- | --- | --- |
| b2,m=1-Gmβm1+Gmβm | (83) |

where cosωc,m is the center frequency for band m.

The denominator coefficients shall be

| a1,m=-2cosωc,m1+βm | (84) |
| --- | --- |

| a2,m=1-βm1+βm | (85) |
| --- | --- |

where ωc,m=2πfc,m/samplingRate is the normalized center frequency in radians.

The gain factor G0 for the GEQ filters shall be the product of the scaling coefficients of the band filters

| G0=m=1Mb0,m | (86) |
| --- | --- |

As a result of the optimization, the parameters G0, a1, a2, b1, b2 for the SOS cascade filter shall be obtained. These parameterize the SOS cascade along with the levelDb.

The coefficients for the third octave band filters shall be obtained in the same manner, with the following differences to octave band filters:

There shall be one more layer in the network used to optimize the gains for the third octave cascade SOS. The optimized gains in decibels shall be obtained with the following procedure:

g = 2.0 * hadamardDivision(controlGainsDb - xMin, xMax - xMin) - 1;

o1 = tanh(W1 * g + theta1);

o2 = tanh(W2 * o1 + theta2);

go = W3 * o2 + theta3;

optimizedGains = hadamard((tMax - tMin), 0.5 * (go + 1.0)) + tMin;

The neural network parameters are the parameters of a two-layer fully-connected feedforward neural network including the maximum and minimum of the input control gains xMin and yMin, respectively, the weighting matrices W1, W2, and W3, bias vectors theta1, theta2, and theta3, and the maximum and minimum of the training data targets tMax and tMin, respectively. The neural network parameters provided in A.6 shall be used.

The free design parameter shall be set to c=0.4. The number of bands shall equal M=31. The bandwidths shall be set as

| B=2πfB,m | (87) |
| --- | --- |

with the values of fB,m tabulated in the Annex A.7.

###### Reverberation ratio control filter (AR)

Figure 16 — Determining reverberation ratio parameters to be used for RDR filter design

A reverberation ratio control filter (RDR control filter) shall be initialized using the input RDR data of the LSDF AE, the RDR calculated from the RIR specified in the LSDF, or the dimensions of the AE. The RDR refers to the reverberant-to-direct energy ratio and is defined for an omnidirectional sound source at one meter distance. It is noted that the ratio of rendered reverberant sound energy to rendered direct sound energy for the listener varies within the audio scene because of distance gain attenuation applied on the direct part and distance-gain attenuation applied on the reverberant part. The RDR parameter is used to adjust the level and spectrum of reverberation at one-meter distance for an omnidirectional source. The other parameter impacting the static reverberation level is source directivity which is handled by the source directivity gain control filters. Generally, the more directive the source is the less reverberant energy it contributes.

It is possible to use bitstream parameters to control how the reverberation ratio is handled when designing the reverberation ratio control filter for AR. The bitstream parameters provide the possibility to override the input ratio parameters from the LSDF with parameters obtained from a theoretical approximation or to adjust the reverberation level to an overall perceptually plausible level. Note that bitstream parameters (and corresponding runtime configuration parameters) allow the user to manually adjust the overall reverberation ratio with theoreticalRDRToActualRDR and lsdfRDRToActualRDR, respective to the chosen RDR specification. The RDR values specified in the LSDF (either as <AcousticParameters> or derived from a provided <RoomImpulseResponse>) can be overridden with the bitstream/runtime configuration parameter overrideLSDF_RDR. The acquisition of band-wise RDR values used as gain parameters to the reverberation ratio control filters is detailed in 6.6.4.3.3.7.6. Figure 16 depicts an overview of handling RDR data.

Figure 17 — Overview of the RDR control filter design

Using the obtained reverberation ratio values a ratio control filter shall be designed as depicted in Figure 17. The filter is designed such that, when the filter is applied to the input data of the FDN reverberator, the output reverberation will have the desired energy ratio defined by the RDR(f). The input to the design procedure are the RDR values RDR(f) at a set of input frequencies f∈f1,f2,…,fF.

The values RDR(f) shall be first mapped to a set of frequency bands m, which are either octave or third octave bands, depending on the resolution at which the input data RDR(f) has been provided. This is done as described in the clause 6.6.4.3.3.3. The mapping to frequency bands results in frequency mapped RDR values RDR(m) where m are the indices of the selected frequency bands (octave or third octave bands).

The energy ratio control filter GEQratio,r matches the reverberator r output spectrum energy to the target spectrum energy provided by the input ratio data. To design the filter an estimate of the RDR of the reverberator output and the target RDR are needed. The RDR of the reverberator output shall be obtained by rendering a unit impulse through the reverberator which has been parameterized without the ratio control filter GEQratio,r and measuring the energy of the reverberator output and energy of the unit impulse and calculating the ratio of these energies.

A unit impulse input shall be formed where the first sample value is 1 and the length of the zero tail is long enough. The length of the zero tail is adjusted to equal max(RT60(f)) plus the predelay in samples. The monophonic output of the reverberator is of interest in the ratio control filter design so a sum over the delay lines j is calculated to obtain the reverberator output s_rev(t) as a function of time t.

An FFT of length NFFT shall be  calculated over s_rev(t) and its absolute value obtained as

| FFA(k) = abs(FFT(s_rev(t)) | (88) |
| --- | --- |

Here, k are the FFT bin indices. NFFT is selected as the next power of two of the length of s_rev(t). The positive half spectral energy density shall be obtained as

| S(k) = 1/NFFT * FFA(k)2 | (89) |
| --- | --- |

where the energy from the negative frequency indices k into the corresponding positive frequency indices kk has been added.

The positive half spectral energy density of a unit impulse shall be obtained analytically as

Su(k) = 2/NFFT

Band energies are calculated of both the positive half spectral energy density of the reverberator S(k) and the positive half spectral energy density of the unit impulse Su(k). Band energies shall be calculated as

| Sm=kk=mlowmhighS(k) | (90) |
| --- | --- |

where mlow and mhigh are the lowest and highest bin index in k belonging to band m, respectively. The band bin indices are obtained by comparing the frequencies of the bins to the lower and upper frequencies of each band.

The lower and upper frequencies shall be obtained with the help of a ratio ρ: for third octave bands ρ=216 and for octave bands ρ=212. Band low frequencies are then obtained as flow=fc/ρ and band high frequencies as fhigh=fc×ρ where fc are the band center frequencies. The first band low frequency is set equal to zero and the last band high frequency is set equal to the Nyquist frequency.

The reproduced RDRrevm of the reverberator output at the frequency band m shall be obtained as

| RDRrevm=S(m)/Su(m) | (91) |
| --- | --- |

The target linear magnitude response for GEQratio,r shall be obtained as

| ratioFilterTargetResponse(m) = sqrt(RDR(m)) / sqrt(RDRrevm) | (92) |
| --- | --- |

This shall be the target magnitude input to the SOS cascade equalizer calculation routine described in the clause 6.6.4.3.3.4.

###### Predelay (AR)

A delay, here referred to as “predelay”, shall be applied to the input signal in order to align the onset of the diffuse response of the reverberator with the diffuse onset specification, which is either declared explicitly as the diffuseOnset attribute of the <AcouticParameters> in the LSDF file, or determined from analysis of a specified room impulse response (6.6.4.3.3.7.3.1 and 6.6.4.3.3.7.3.2). If the diffuse onset is not available through the LSDF file, it shall be approximated as

| tdiffuse=dlongest/vsos | (93) |
| --- | --- |

where vsos is the speed of sound, and the longest room dimension dlongest is the longest dimension of the bounding box enclosing the Acoustic Environment. Note that this differs from the predelay calculation used in the EIF in that a scaling factor of 4.0 has been omitted.

The diffuseOnset shall then be scaled by dividing by the predelayFactor, then offset by the addition of predelayOffset, both of which are user-specified runtime configuration parameters.

The predelay, in samples, mpre shall equal

| mpre=max0, (tdiffuse-tdiffuse, FDN)⋅fs | (94) |
| --- | --- |

where tdiffuse, FDN is the diffuse onset time of the reverberator’s impulse response, which shall be spatially encoded into the SH domain and measured via the method described in 6.6.4.3.3.7.3.1. The predelay delay line shall be a ring buffer of length B + mpre.

m_buffer = array(B + predelayAdjustedInSamples);

m_readIndex = 0;

m_readSamples = 0;

m_writeIndex = predelayAdjustedInSamples;

m_writtenSamples = predelayAdjustedInSamples;

###### Room Impulse Response Analysis

The acoustic environment to which the reverb corresponds, shall be defined in the LSDF by either explicit acoustic parameters or a room impulse response (RIR). The RIR can be provided as a monophonic file, representing an omnidirectional signal s(t). Alternatively, a spatial room impulse response (SRIR) can be provided, comprising spherical harmonic (SH) domain, or ambisonic, signals st, where s∈ RN+12×1 for a SRIR of SH order N, formatted according to the ambisonic channel numbering, ortho-normalized convention (ACN-N3D). In the case that the SRIR is provided in ambisonic semi-normalized (ACN-SN3D) format, the signal set shall first be converted to the N3D normalization scheme before further processing by

| s=diagb⋅sSN3D | (95) |
| --- | --- |

where b∈RN+12×1 comprises conversion coefficients calculated according to

| bin2+jm=2in+1, where in∈{0,…,N}, jm∈{0,…,2N}. | (96) |
| --- | --- |

The RIR analysis workflow generates the band-wise reverberation time (T60), and reverberant-to-direct energy ratio (RDR) which parameterize the reverberator. The following subsections describe the analysis procedures used to determine these acoustic parameters.

###### Frequency band filtering

Room acoustic parameters shall be derived from a room impulse response in a band-wise manner, using K=8 octave bands with nominal center frequencies defined in Hertz as:

fc={125, 250, 500, 1000, 2000, 4000, 8000, 16000}.

The filter bank is initialized with crossover frequencies

| fxk=fck*2, for k∈1,…,K | (97) |
| --- | --- |

where the first and last filters are lowpass and high-pass filters, respectively, with bandpass filters in between. The band filters shall be constructed as linear phase FIR filters. The filter design can be done using the windowing method of [18], with a Hamming window and a filter length of Mfilt=400 samples. The filtering can be performed by frequency-domain convolution on each of the (S)RIR signal channels.

The resultant filter delay of Mfilt/2 samples shall be removed from the head of the band-filtered signals in order that they remain time-aligned with the input signal, and the signals shall further be truncated at the tail such that their lengths are equal to that of the input RIR.

###### Direct sound arrival detection

The sample index of the direct sound arrival in the RIR (or omnidirectional component of the SRIR) for both the broadband and band-wise omnidirectional signals, denoted idirect and idirect(k) respectively, shall be chosen to be the first sample index for which the condition

| si≥γthresh⋅max isi, | (98) |
| --- | --- |

is met, where γthresh∈0,1 denotes a threshold level (set to 10-6dB/20) below the maximum absolute value max isi within the signal s.

###### Diffuse onset calculation

A critical feature of reverberation is the diffuse onset—the point in time in RIR when the response becomes spatially and temporally diffuse and uncorrelated, i.e. in which individual early reflection echoes can no longer be distinguished as separate from one another. The diffuse onset is also commonly referred to as ‘mixing time’. The diffuse onset metric shall be determined by two different methods depending on whether the input is an SH domain SRIR or a monophonic RIR, as detailed in the following subsections.

###### Diffuse onset calculation — SRIR

In the case of SRIR processing, the diffuse onset shall be calculated using the intensity-based metric of diffuseness, calculated in the spatial time-frequency domain using the Short-time Fourier Transform (STFT). The time-domain signals from the first four HOA components (omnidirectional and three dipole components) shall first be transformed into the pressure-velocity signal vector, with velocity components ordered according to principal Cartesian axes by

| spv= 10000001/301/300001/30 s. | (99) |
| --- | --- |

The time-domain signal spvt shall then be windowed into a size KFFT samples and converted to the time-frequency domain by the forward transform of the (real) STFT, resulting in a frequency-domain analysis frame matrix S∈C(4×KFFT), comprising bin-wise components for pressure pk=S1,k and velocity vk=S2,k,S3,k,S2,k⊤, where k∈{1,…,KFFT}.

From the pressure and velocity signals, the active intensity ia, sound field energy E, and diffuseness ψ, shall be constructed by the following (frequency bin index k is omitted for brevity):

| ia=-R{p⋅vH}, | (100) |
| --- | --- |
| E=12p2+vHv, |  | (101) |
| ψ=1-iaE. |  | (102) |

In practice, KFFT can be set to 128 and the analysis frame matrix can be culled to the hold only the frequency bins up to a cutoff frequency which diffuseness should be measured (MAX_DIFF_FREQ_HZ, which shall be set to 3000 Hz).

The broadband average of the above quantities shall be calculated by the covariance matrix C=SSH, where the superscript H denotes the Hermitian conjugate and the quantities of active intensity and energy density are

| ia=C2,1,C3,1,C4,1⊤, | (103) |
| --- | --- |
| E=12i=14Ci,i. | (104) |

Analysis shall begin at a position which centers the analysis window on idirect and proceed frame-by-frame, with the analysis window advancing Khop=KFFT/2 samples at a time. Smoothing shall be carried out over J=2 analysis frames according to

| iaj=1-κiaj+κiaj-1, and | (105) |
| --- | --- |
| Ej=1-κEj+κEj-1 | (106) |

where j is the current analysis frame and κ=exp-1/J is the smoothing constant, yielding a smoothed diffuseness measure

| ψj=1-iajEj. | (107) |
| --- | --- |

The diffuse onset shall be determined to occur at the analysis frame index jψ, which is the earliest frame for which

| ψjψ≥ψthresh⋅ψmax | (108) |
| --- | --- |

where ψthresh∈0,1 (set to 2/2) is a fraction of the maximum diffuseness ψmax over all analysis frames. The sample index of the diffuse onset idiffuse shall be set to the sample that is the floor of the center position of the diffuse onset analysis frame at index jψ.

###### Diffuse onset calculation — RIR

In the case that a monophonic RIR is specified in the LSDF, the diffuse onset shall be calculated based on the echo density metric

| ψt=1erfc1/2τ=t-δt+δwτ⋅1sτ > σ | (109) |
| --- | --- |

where s is the RIR, τ is the index into the frame window of size Kwin=2δ+1 (set to 1024 samples), and wτ is a Hann window function. The complementary error function erfc⋅ is used to normalize the echo density by the fraction of samples that are expected to lay outside one standard deviation of a Gaussian distribution, such that ψt∈0,1. The function 1{⋅} shall return a boolean value of 1 or 0 indicating whether or not its argument is true—in this case whether the absolute value of the RIR sample exceeds the standard deviation

| σ=τ=t-δt+δwτs2τ 12. | (110) |
| --- | --- |

Analysis shall begin at a position which centers the analysis window on idirect and proceed through the RIR with a hop size of Khop=64 samples until the diffuse onset criteria ψt≥ψthresh, where ψthresh∈0,1 (set to 2/2), is met. The sample index of the diffuse onset idiffuse shall then be set to the index within the RIR corresponding to the sub-index τ=0, the nominal center index, of the analysis frame which satisfies the diffuse onset criterion.

###### Energy decay curve (EDC) calculation

The energy decay profile of a RIR provides a useful measure for determining the reverberation time of a room.  The energy decay curve EDC shall be calculated by backward integration of the squared input signal. This calculation shall be performed on the broadband RIR signals (or omnidirectional component of the SRIR sOMNI), denoted EDC(t) and EDCt,f, respectively:

| EDCt=1Eτ=tLsOMNIτ2, with E=τ=1LsOMNIτ2 | (111) |
| --- | --- |

| EDCt,f=1E(f)τ=tLsOMNIτ,f2, with Ef=τ=1LsOMNIτ,f2 | (112) |
| --- | --- |

###### Reverberation time estimation

The reverberation time T60k is estimated for the band-wise RIR signals (or band-wise omnidirectional component of the SRIR), representing an omnidirectional rate of energy decay, also called a “common slope” decay, because the metric is considered isotropic. The estimated values shall be used in the FDN attenuation filter configuration, as detailed in 6.6.4.3.3.2.

T60 for a signal shall be estimated by first finding a linear line-of-best-fit to a relevant span of the EDC on a logarithmic scale (decibels). A least-squares fit shall be performed on the EDC using sum of squares terms

| ssxx=i=1L'xi-x2, ssyy=i=1L'yi-y2, ssxy=i=1L'xi-xyi-y | (113) |
| --- | --- |

where x is a constructed vector of values representing a line with a slope of one dB per sample with a mean of zero, and y is EDCt,f, in logarithmic scale, and the overbar denotes the mean value over the measured span of length L'. The regression coefficient shall then be

| b=ssxyssxx, | (114) |
| --- | --- |

representing the linear decay per sample, in decibels, of the line-of-best-fit. The quality of the fit is captured by the correlation coefficient

| r2=ssxy2ssxxssyy. | (115) |
| --- | --- |

The length of the measured span L' shall be determined through an iterative process, in which b and r2 shall be calculated for spans comprising different decay ranges from 10 to 60 dB, in steps of 2 dB, and from starting indices which range from idirect+4ms to idiffuse (see 6.6.4.3.3.7.3 and 6.6.4.3.3.7.2). The chosen fit shall be the one with the highest correlation coefficient r2. An exception to this rule shall be made when there are multiple fits for which r2>0.95, in which case the fit with the largest measurement length L' among the qualified fits shall be chosen.

The rate of decay for the signal shall then be converted to reverberation time by

| T60=-60b⋅fs | (116) |
| --- | --- |

where fs is the sampling rate.

A rule-based error check shall be applied to the estimated T60 values, to guard against degenerate band signals which may, for example, be contaminated by a high noise floor. In particular, the lowest band (125 Hz) is susceptible to noise floor contamination, so it shall be checked that its estimated T60, stored in the variable t60Buf[ibnd], with ibnd denoting the band index, does not exceed twice that of the next-highest band (250 Hz). If the estimate is found to be out-of-range, it shall be set to a linear back projection of the values of the second and third bands, clipped at a lower bound of 0 dB (no cut allowed), as follows:

if (t60Buf[0] > t60Buf[1] * 2) {

t60Buf[0] = t60Buf[1] + max(0, (t60Buf[1] - t60Buf[2]));

validSlopeBoolBuf[0] = 0;

}

From band three upwards, the estimated T60 shall be checked that it does not exceed three times the value of the previous band. If the estimate is found to be out-of-range, it shall be set to a linear forward projection of the values of the previous two bands, clipped at an upper bound of 0 dB (no boost allowed), as follows:

if (ibnd >= 2 && t60Buf[ibnd] > t60Buf[ibnd - 1] * 3) {

t60Buf[ibnd] = t60Buf[ibnd-1] - max(0, (t60Buf[ibnd-2] - t60Buf[ibnd - 1]));

validSlopeBoolBuf[ibnd] = 0;

}

where ibnd is the band index. The band-wise flag validSlopeBoolBuf shall indicate whether the estimated T60 was valid for each band, and shall be used in downstream processing, namely the RDR and directional gain calculations, to signal whether to proceed with the metric calculation from the respective band signal, or otherwise handle the degenerate case.

###### Reverberant-to-direct energy ratio calculation

The reverberant-to-direct ratio (RDR) shall be used to scale the reverberation level in each frequency band via the gains passed to the GEQratio filter of the reverberator (6.6.4.3.3.5). The RDR shall be defined for an omnidirectional RIR (or the omnidirectional component of a SRIR) as

| RDR=10⋅log10 t2tends2tdtrmeas2t0t1s2tdtdB, | (117) |
| --- | --- |

where time bounds t0 and t1 shall denote endpoints of a window containing the direct sound, and time bounds t2 and tend shall denote endpoints of a window containing the late, diffuse portion of the energy decay, from the diffuse onset to the moment that the noise floor is reached, respectively. The distance rmeas shall be the distance between the sound source and the receiver.

If the source and receiver position, from which rmeas is calculated, are not specified in the LSDF, or if the bitstream configuration parameter overrideLSDFRDR is true, the RDR shall not be calculated from the (S)RIR, but a model-based linear approximation using the T60f, diffuse onset, and room volume shall instead be used. The approximation of RDR values shall then be

| RDRf=10⋅log10T60f/V – 60⋅Tdiffuse/T60f+25 dB, | (118) |
| --- | --- |

where Tdiffuse is the time of the diffuse onset, in seconds (see 6.6.4.3.3.7.3). The room volume V shall be calculated as the volume of the bounding box enclosing the Acoustic Environment. A user-specified offset to all RDR values shall be applied as

| RDRf=RDRf+RDRoffset, | (119) |
| --- | --- |

where RDRoffset is the bitstream parameter (or corresponding runtime configuration parameter) theoreticalRDRToActualRDR or lsdfRDRToActualRDR, depending on the source of the RDR values. Linear RDR values shall be provided to the RDR filter design routine described in 6.6.4.3.3.5:

| RDRlinf=10RDRf10. | (120) |
| --- | --- |

The direct onset window endpoints t0 and t1 shall be determined by centering a window on the direction onset index idirect, with a window size that is max{0.0025, 2/fck}⋅fs; that is, the duration of two periods of the center frequency of the kth filter band, clipped at a minimum value of 2.5 milliseconds, in samples. The late energy window shall be bound between the diffuse onset idiffuse and the time point from the diffuse onset at which the energy has decayed 30 dB, calculated by idiffuse+T60⋅fs/2.

The directivity of the sound sourced used in the (S)RIR measurement can be signaled in the LSDF file by the directivity attribute of <RoomImpulseResponse>, where a Boolean value of 0 or 1 shall indicate an omnidirectional or nominal loudspeaker directivity, respectively. In the case of an omnidirectional sound source, no modification of the RDR shall be done. In the case of loudspeaker directivity, a frequency-dependent scaling of the RDR shall be applied as

| RDRlinf=RDRlinf*glsk. | (121) |
| --- | --- |

The loudspeaker directivity factor gls is listed and described in more detail in Annex A.2.

##### Reverb stage processing

###### General

Reverberation processing for each block of samples of length B comprises the steps of determining input items, performing input gain processing for the input items, applying reverberator processing, crossfading output gains, and generating output signals.

###### Reverberation input processing

Figure 18 — Input processing for Reverberators.

The Reverb stage shall take as input RIs which are of type Primary or HOA and which are not Inactive, Orphaned, or have the NoReverb attribute set to true. The reverberation input items are added as dspItems at each update() step of the Reverb stage.

The Room Assignment stage produces the information of which audio elements are currently inside which AE. Each eligible input audio element (i.e. input render item) can contribute to all reverberators or a subset of them, depending on its location. A render item inside an AE that is different from the AE associated with the reverberator shall be considered an external source for that reverberator. Contributions of external sources shall only be included in explicit portal mode (isExplicitPortalMode = true). Their contribution shall be a portion of the full source energy, depending on how much of their energy reaches the reverberator’s AE through one or more portals.

Conversely, internal sources shall be sources that are located inside the AE associated with the reverberator. Such sources’ contribution to that reverberator shall be reduced with the portions of source energy that are lost through one or more portals, unless the portals are operating in implicit portal mode,

The reverberation input items shall be gain controlled and summed into mono reverberator input bus. Details of reverberation input gain processing are depicted in Figure 18 and described in 6.6.4.3.4.3 and 6.6.4.3.4.5.

###### Reverberation input gain processing

The input gain calculation shall consist of six main components:

- Source gain

- Source contribution indicating which portion of the source’s energy contributes to the AE’s reverberation

- Distance attenuation-based listener to source distance inside the AE

- Conversion factor for converting PCM samples to RMS source energy

- A common component from the reverberator

- Arbitrary source reverb gain selected by the scene author

The static source gain is indicated with itemGain.

The dynamic source reverb gain is indicated with itemReverbGain.

In implicit portal mode, a source contributes only to reverberators associated with the AEs in which the source is located. Therefore, the source contribution (sourceContribution) is 1 if the source is in the AE associated with the reverberator, and 0 otherwise.

In explicit portal mode, the source contribution represents the effect of source energy leaking to other acoustic environments through portals. If the source AE is different from the reverberator AE, then the source contribution shall indicate the portion of source energy that reaches the reverberator AE through one or more portals connecting the two AEs.

If the source AE and reverberator AE are the same, then the source contribution shall indicate the portion of source energy that is not lost to other AEs through portals.

When there are no portals directly or indirectly connecting the source AE with the reverberator AE, sourceContribution = 0.

Otherwise, for each portal connected to the source AE with the reverberator AE, a portal contribution (portalFactor) shall be calculated, and all portal contributions shall be combined to form the source contribution for item i:

| sourceContributioni,k=p ∈Pathsi,klosFactori, p1*portalFactorip1 *materialAttenp*p2pAttenp | (122) |
| --- | --- |

where

| materialAttenp=j=1JpmaterialGain[pj] | (123) |
| --- | --- |
| p2pAttenp=j=1Jp-1p2pFactorpj[pj+1] | (124) |

losFactor(i, p1) shall be a further gain factor controlling the amount of energy propagation from the render item i through portal p1 and adjusting the level of reverberation the source signal creates in the connected acoustic environment. If there is a line of sight (LoS) from the position of the source i to portal p1 then losFactor (i, p1) = 1.0. Otherwise losFactor(i, p1) = m_noLineOfSightCoeff.

Determination whether there is a LoS from the position of the source i to portal p shall be obtained based on LoS voxel grids losVoxelGridp,area for portal p covering a subset of scene positions as possible source positions and which have been decoded from payloadPortal 6.2.7. area defines a rectangular axes-aligned subregion of the scene covered by LoS voxels. If there is no LoS voxel grid associated with p then it shall be determined that LoS exists. The search shall be first done from the LoS voxel grid covering a part of the AE where the source locates if such a grid exists or from the outside LoS voxel grid for p if one exists. If there exists a LoS voxel grid losVoxelGridp,area for the portal p then the LoS voxel index voxIdx(i) can be obtained using the following procedure, where pos = [x, y, z] contains the Cartesian coordinates of the source i position:

getLosVoxelIndex(pos)

{

idx = [-1, -1, -1];

for (i = 0; i < 3; i++) {

M_TO_MM = 1000.f;

CM_TO_MM = 10.f;

mmvoxelSize = voxelSize * CM_TO_MM;

mmpos = round(pos[i] * M_TO_MM);

mmp1 = round(cornerP1[i] * M_TO_MM);

nearest = abs((mmpos - mmp1) / mmvoxelSize);

idx[i] = min(nearest, (nVoxels[i] - 1));

}

voxIdx = idx[1] * nVox[2] * nVox[0] + idx[2] * nVox[0] + idx[0];

return voxIdx;

}

cornerP1, cornerP2, and voxelSize are the decoded values of cornerP1, cornerP2, and voxelSize of losVoxelDataStruct corresponding to losVoxelGridp, respectively. nVoxels = [nVoxelsX, nVoxelsY, nVoxelsZ], where nVoxelsX, nVoxelsY, and nVoxelsZ are the decoded values of nVoxelsX, nVoxelsY, and nVoxelsZ of losVoxelGridp,area respectively.

If voxIdx(i) ≥ 0 and losVoxelGridp,area[voxIdx(i)] = 1 then there is a LoS. Otherwise there is no LoS.

Pathsi,k shall be the set of all non-cyclical paths from source i to reverberator k where i≠k. A non-cyclical path p is comprised of Jp portals p1,p2,…pJp, such that:

- p1 shall have portalEnv1 or portalEnv2 equal to the AE of source i

- pJp shall have portalEnv1 or portalEnv2 equal to the AE of reverberator k

- Every adjacent pair of portals pj and pj+1 shall share exactly one portal environment

- The AE of source i and the AE of reverberator k shall occur exactly once in the portal environments of all portals in a path.

- Other AEs shall occur exactly 0 or 2 times in all portal environments related to the portals in a path.

Here Jp denotes the number of portals in a path. Note that in case a path contains only a single portal, then the empty product j=10p2pFactorj[j+1] shall be 1.

The materialGain variable shall represent the material property of the portal pj. For calculating how much the source contributes to the other AE, materialGain shall be the sum of the transmission coefficient at 1 kHz and the coupling coefficient at 1 kHz.

The p2pFactor[pj][pj+1] shall represent the portal2PortalFactor between portal pj

and portal pj+1 if it exists in the portal payload, and 0 otherwise.

Calculation of portalFactor shall be performed based on the source position (x, y, z) in Cartesian coordinates. The renderer can first consult a cache of previously calculated portalFactor values for the source. If the source position has changed, it indicates dynamic source movement. If the source position has not changed, the previously calculated portalFactor can be retrieved from the cache for the static source position. If the source has moved or no cache for it exists, portalFactor shall be calculated.

The calculation of the portalFactor shall be based on the source to portal vector and the portal’s normal vector. First, the source to portal vector shall be rotated with rotation matrix Rn that aligns the portal normal to (0, 0, -1).

| v=Rns-p | (125) |
| --- | --- |

where s shall be the coordinate of the source location and p shall be the coordinate of the portal’s centre (portalCenter).

| a=n×00-1 | (126) |
| --- | --- |
| cosA=n∙00-1 | (127) |
| k=0cosA<-0.999911+cosAotherwise | (128) |
| Rn=ax2k+cosAayaxk-azazaxk+ayaxayk+azay2k+cosAazayk-axaxazk-ayayazk+axaz2k+cosA | (129) |

Then, the portalFactor shall be calculated according to the following equations:

| da=vx2+vz2 | (130) |
| --- | --- |
| de=vy2+vz2 | (131) |
| cosφa=sign-vz∙max⁡-vz, εmaxvx2+vz2,ε | (132) |
| cosφe=sign-vz∙max-vz,εmaxvy2+vz2,ε | (133) |
| θa=atandaWsignda2-14W2,∙maxda2-14W2,ε2∙maxcosφa,ε mod π | (134) |
| θe=atandeHsignde2-14H2,∙maxde2-14H,ε2∙maxcosφe,ε mod π | (135) |
| portalFactor= θa∙sin0.5θe2π | (136) |

Variable W indicates the portal width (portalWidth) and H the portal height (portalHeight).

To calculate the energy a source i contributes to the AE k in which it is located the energy lost to other AEs shall be subtracted as:

| sourceContributioni,k,k=1-l≠kp ∈ Pk,llosFactori, p*portalFactori,p*materialGainp-p ∈ Pk,xlosFactori, p*portalFactori,p*materialGainp | (137) |
| --- | --- |

Here, materialGain shall be the transmission coefficient at 1 kHz. Pk,x represents the set of portals that connect the source AE to a non-AE (isConnectedToSecondAE == false).

The Reverb stage shall further apply slight distance gain attenuation to match the expected decay of the late reverberation level particularly in large spaces.

This shall be done as follows for sources within an AE when they contribute to the AE reverberator:

The location (x, y, z) in Cartesian coordinates of the item to be rendered is stored in sourceLocation.

The Euclidean distance dist between the sourceLocation and listenerLocation (also in Cartesian coordinates):

dist = abs(sourceLocation - listenerLocation);

The method then takes the maximum of dist and minimumDistance. This is done to prevent excessive level increase of the late reverberation when close to a sound source. minimumDistance is obtained from the reverbMinimumDistance field in the payloadReverb.

dist = max(minimumDistance, dist);

The distanceGain value to be applied to the reverberation input signal shall be calculated by the method calculateDistanceGain, based on dist and the refDistance value of the rendered item. The refDistance is a reference distance in meters for the rendering item obtained from the bitstream. The reference distance is the distance at which the calculated distance attenuation for the input signal is 0dB.

distanceGain = calculateDistanceGain(dist, refDistance);

The method performed in calculateDistanceGain is as follows:

dbGain = distanceGainDbFactor * log10(refDistance / distance);

distanceGain = pow(10.0, dbGain / 20.0);

Here, distanceGainDbFactor is calculated as

distanceGainDbFactor = distanceGainDropDb / log10(2.0);

distanceGainDropDb shall be obtained from the decoded value of the parameter reverbDistanceGainDropDb in the payloadReverb unless a run-time override value is provided. Suitable values are between 1dB and 2dB to implement a level decrease between of 1dB to 2dB per distance doubling.

The distanceGain value shall be  calculated at each update() step of the Reverb stage. It shall be multiplied with the itemGain and with the itemReverbGain to obtain the final gain value to be applied to the audio signal input samples.

If item is not in any bounded AE, it shall be in the Default AE. The distance gain attenuation processing shall be the same as above but the refDistance shall be set to 1 and the decay and minimum distance shall be set differently to bounded acoustic environments. distanceGainDbFactor shall be set according to the decoded value of reverbPayload revDefaultAEDistanceGainDropDb or presetDefaultAEDistanceGainDropDb unless a run-time override is available. minimumDistance shall be set according to the decoded value of revDefaultAEMinimumDistance or presetDefaultAEDistanceGainDropDb unless a run-time override is available.

When a source contributes to the reverberator of an AE via a portal, the distance gain attenuation shall have the following differences to the above: the distance shall be calculated between the portal and the source location. If there are multiple portals between the AE where the source is located and the reverberator AE, the distance shall be calculated from the source to the portal closest to the listener. Since this mechanism can cause a switch in the closest portal and thus distance gain value, smoothing shall be applied to mitigate the effects. Distance gain shall be smoothed over update cycles in such a way that any change longer than 0.25 meters is not applied instantly, but the change is clamped to 0.25 m and any remaining change is applied in the upcoming update rounds (with a maximum of 0.25 m) until the new distance value is reached.

Then, the gain g that is to be applied to the reverberation input signal for this rendering item during DSP processing is calculated as follows:

g = itemGain * sqrt(sourceContribution) * distanceGain * PCM2SourcEnergyFactor * commonComponentRatioFlt * itemReverbGain;

In this calculation, PCM2SourcEnergyFactor is the scale factor that indicates how the source’s PCM samples scale to RMS source energy emitted into the room if the source were omnidirectional, and for VR environments shall be calculated as:

PCM2SourcEnergyFactor = sqrt(4 * PI / earSurf) * refDistance

where earSurf = 0.001 m2.

The factor commonComponentRatioFlt shall be the common component of the reverberation ratio control filter of the corresponding reverberator.

For AR environments:

PCM2SourcEnergyFactor = 1 and commonComponentRatioFlt = 1.

itemReverbGain is an optional parameter that can be updated via immediate, pre-defined, or interpolated scene updates. The dB value contained in the bitstream (reverbGainDb) is being converted to a linear value upon reading on the renderer side.

itemReverbGain = pow(10.0, reverbGainDb / 20.0);

If it is not set in the bitstream, then default itemReverbGain = 1.0.

The gain g then contains the gain to be applied to the reverberation input signal for this rendering item during DSP processing, and combines any static gain in itemGain, the calculated distanceGain and the dynamic itemReverbGain.

###### Reverberation gain processing for connected acoustic environments

Acoustic environments connected via a portal are referred to as connected acoustic environments. In the case of connected acoustic environments the amount of reverberation leaking through a portal opening to a connected acoustic environment is represented with a connected reverberation coefficient gsor,pj. The coefficient value shall depend on the dimensions of the portal opening and the material property materialGain[pj] of the portal pj:

| gsor,pj=WH16π* materialGain[pj] | (138) |
| --- | --- |

where W indicates the portal width (portalWidth) and H the portal height (portalHeight) in meter.

Clamping shall be applied to limit gsor,pj between 0 and 1.

materialGain[pj] shall be updated during the update() cycle to enable determining active portal connections which can leak energy, i.e., whose transmission or coupling coefficients are non-zero.

The gain gsor,pj shall be applied to the stored past output srev,rn of a reverberator r which provides energy through portal pj to a connected acoustic environment.

###### Directivity gain filtering and summing to reverberator input buses

Each reverberator shall be associated with at least one input bus which shall be zeroed in the beginning of processBlock() processing for each reverberator. The audio signal of the dspItems for each reverberator shall be summed as mono mix audio signal into the corresponding reverberator predelay input bus before reverberation processing.

To facilitate spatial rendering of reverberation in multiple acoustic environments, a reverberator which corresponds to an acoustic environment having a portal defined in the bitstream shall have additional input buses for feeding sound sources and reverberation that propagate through the portal opening. The additional input buses are used to account for the additional propagation delay of sources and reverberation that propagate through the portal. There shall be as many input buses to reverberators as there are different predelay values that need to be applied: one for the acoustic environment predelay and one for each different propagation path delay via a portal opening.

The selection of the input bus to which feed the audio signal is done based on the predelay as follows: for sources within an acoustic environment AE1 the predelay input buffer corresponding to predelayAdjustedInSamplesAE1 for this acoustic environment (AE1) is selected. For sources outside AE1 and within AE2 and when feeding into the reverberation of AE1, the predelay buffer shall be selected based on connectedPredelayAE1, obtained as

connectedPredelayAE1 = predelayAdjustedInSamplesAE1 + max(minDelayAE2,

1/8*(predelayAdjustedInSamplesAE2 + minDelayAE2));

When connected reverberation is enabled into AE1 based on an active portal connection between AE1 and AE2, the past output of reverberator of the acoustic environment AE2 is fed into a predelay buffer of AE1 having the length

secondOrderPredelayAE2,AE1 = 0.5*(predelayAdjustedInSamplesAE1 + predelayAdjustedInSamplesAE2).

When m_unrestrictedSor = false then an active portal connection from AE2 to AE1 shall exist when listener is within AE1, AE2 is connected to AE1 via a portal and material properties of the portal enable acoustic transmission. Otherwise, an active portal connection shall exist between all pairs of AEs which have a portal connection defined assuming the portal material enables acoustic transmission.

Sources with a directivity pattern and thus reverberation directivity gain filter shall be first summed to the corresponding reverberator directivity filter input bus. The reverberator directivity filter input buses shall be zeroed in the beginning of processBlock() processing for each reverberator. Each reverberator shall have its own set of directivity filter input buses.

Sources having a directivity pattern k shall be first summed into the reverberator directivity input bus associated with the directivity pattern k. Several sources with the same reverberation directivity gains shall be summed to the same bus and filtered only once with the corresponding reverberation directivity gain filter GEQdir,k. A status variable directivityBusAdded shall be reset to count how many input signals are added to each directivity filter input bus.

After adding all dspItems into either the reverberator input bus or one of the directivity filter input buses, the directivity gain filters shall be processed. The directivity gain filter processing for each directivity filter shall inspect the value of directivityBusAdded for this directivity filter to determine if any input signals have been added to this input bus and if at least one input signal has been added run processing for the directivity gain filter. The filtering with the directivity gain control filter shall be done in the same manner as for the ratio control filter described in clause 6.6.4.3.4.7. The filtered output from each directivity filter shall be added to a reverberator predelay input bus.

If a source does not have a directivity pattern it shall be directly summed into a reverberator predelay input bus.

###### Reverberator processing – predelay

The audio processing for each audio block of length B in each reverberator predelay input bus in each reverberator shall contain the following steps:

Writing B input samples into the predelay delay line with the following method.

write(source) {

remainingSamples = length(source);

firstWriteSize = min(remainingSamples, length(m_buffer) - m_writeIndex);

copy_n(source, firstWriteSize, buffer + m_writeIndex);

remainingSamples -= firstWriteSize;

copy_n(source + firstWriteSize, remainingSamples, buffer);

m_writtenSamples += length(source);

m_writeIndex = m_writtenSamples % length(m_buffer);

}

copy_n(sourcePointer, count, targetPointer) shall copy count samples starting from the memory address sourcePointer to a memory location starting from the address targetPointer.

Reading B predelayed input samples into destination from the predelay delay line with the following method:

read(destination) {

remainingSamples = length(destination);

firstReadSize = min(remainingSamples, length(m_buffer) - m_readIndex);

copy_n(m_buffer + m_readIndex, firstReadSize, destination);

remainingSamples -= firstReadSize;

copy_n(m_buffer, remainingSamples, destination + firstReadSize);

m_readSamples += length(destination);

m_readIndex = m_readSamples % length(m_buffer);

}

Samples shall then be summed across predelay lines to form one buffer of length B as input for the following steps.

###### Reverberator processing – reverberation energy ratio control filter

Filtering the predelayed input samples from destination with the ratio control filter GEQratio,AE shall be performed to obtain ratio-controlled input samples to the FDN reverberator.

The directivity gain control filters, ratio control filters GEQratio,r and the delay line attenuation filters shall use the below procedure filterSOSCascade for calculating an output filteredSample  for each input sample sample:

filterSOSCascade(filterParameters, sample) {

a1, a2, b1, b2, wPast0, wPast1, levelAndFFactor, m_F = filterParameters;

filteredSample = sample;

for (i = 0; i < numSOSs; i++) {

filteredSample = filterSOS(

filteredSample, a1[i], a2[i], b1[i], b2[i], wPast0[i], wPast1[i]);

filteredSample *= m_F[i];

}

filteredSample = levelAndFFactor * filteredSample;

}

numSOSs is the number of SOS filters in the cascade.

m_F are the stage gains for the SOS filters. If stage gains are not present or are all equal to one the multiplication with the stage gain is omitted.

levelAndFactor shall be calculated as levelAndFactor = G0 * db2mag(-levelDb).

The coefficients a1[i], a2[i], b1[i], b2[i] are the feedback and feedforward coefficients of the ith filter in the SOS cascade. The past output values wPast0 and wPast1 shall be initialized to vectors of zeroes of length numSOSs and stored between sequential calls of the filterSOSCascade and filterSOS methods.

The procedure filterSOS shall implement a SOS filter in direct form 2 as follows:

float filterSOS(float sample, float a1, float a2, float b1, float b2,

float& wPast0, float& wPast1)

{

w = sample - a1 * wPast0 - a2 * wPast1;

sample = w + b1 * wPast0 + b2 * wPast1;

wPast1 = wPast0;

wPast0 = w;

return sample;

}

###### Reverberator processing - FDN processing

The FDN reverberator shall process the ratio-controlled input samples at m_inputBuffer to produce numDelayLines output signals srev,r(j,n) where j is the output channel index.

For feedback matrix calculation, the constants m_alpha, m_hasPositiveFeedbackGain and m_hasNegativeFeedbackGain shall be initialized with the values listed in Table A.8, depending on numDelayLines.

The FDN reverberator processing shall be as follows:

outputBuffer = array(0.0f, numDelayLines);

numPositiveItems = m_hasPositiveFeedbackGain.size();

numNegativeItems = m_hasNegativeFeedbackGain.size();

for (n = 0; n < sampleCount; n++) {

d = 0.0f;

for (i = 0; i < numDelayLines; i++) {

d += v[i];

}

alphaTimesD = alpha * d;

updated_v = array(0.0f, numDelayLines);

for (delayLineIndex = 0; delayLineIndex < numDelayLines; delayLineIndex++) {

s = &m_v[numDelayLines - delayLineIndex];

b = 0;

c = 0;

for (dlId2 = 0; dlId2 < numPositiveItems; dlId2++) {

b += s[m_hasPositiveFeedbackGain[dlId2]];

}

for (dlId3 = 0; dlId3 < numNegativeItems; dlId3++) {

c += s[m_hasNegativeFeedbackGain[dlId3]];

}

a = 0.25F * (b - c) + alphaTimesD;

delayedSample =

getDelayedSample(m_delayLines[delayLineIndex], m_inputBuffer[n] + a);

filteredSample = filterSOSCascade(m_filters[delayLineIndex], delayedSample);

updated_v[delayLineIndex] = filteredSample;

m_outputBuffer[delayLineIndex][n] += filteredSample;

}

copy_n(&updated_v[0], numDelayLines, &m_v[0]);

copy_n(&updated_v[0], numDelayLines, &m_v[numDelayLines]);

outputScaler = 1.0f / numDelayLines;

for (outputChannelIndex = 0; outputChannelIndex < numDelayLines; outputChannelIndex++) {

m_outputBuffer[j][n] *= outputScaler;

}

}

copy_n(sourcePointer, count, targetPointer) shall copy count samples starting from the memory address sourcePointer to a memory location starting from the address targetPointer.

array(value, count) shall initialize an array of length count with values value.

The operator &variable shall return the memory address of variable.

The operator *pointer shall return the value of the variable located at the address specified by pointer.

The operator variable++ shall increment the value of variable by one.

The output signal is thus at m_outputBuffer[j][n]. All samples of the output signal shall further be multiplied with the overall reverberation gain reverbGain to obtain reverberated output signals srev,rj,n.

For background information on the FDN processing see Rocchesso [3].

###### Reverberator output signal creation

Reverb stage outputs are created from the delay line outputs srev,rj,n. The immersive reverberation signal shall be stored at headtrackedOutputRenderItems(j,n,azi,ele), where j is the output channel (delay line) index, n the sample index, and azi and ele the desired user-centric coordinates for rendering the immersive reverberation output signals.

Depending on the value of headTrackingEnabled, head tracking shall be applied to the output signals.

—	if headTrackingEnabled=1, the delay line positions are to stay "static within the scene" when the listener head rotates and thus the delay line positions (azi and ele in headtrackedOutputRenderItems(j,n,azi,ele)) shall be rotated in inverse of the listener head rotation.

—	if headTrackingEnabled=0, the delay line positions are to stay “static around the listener head” and no rotation about the listener head shall be applied.

To implement smooth transitions between the AEs of an audio scene, transition region processing shall be applied to enable smooth audio transition between acoustic environments or when moving in/out of an AE. The size of the transition region shall be defined by the parameter fadeInDistance in meters. The fadeInDistance is the threshold distance in meters at which cross-fade processing shall be enabled for immersive reverberation. The threshold distance shall define a transition region having a width equal to fadeInDistance outside the region or enclosure of each AE. The cross-fade shall start at fadeInDistance from the region of an AE and end at the region.

To implement the smooth transition for the immersive reverberation output, an output gain coefficient gOutr shall be calculated for each active reverberator r associated with an AE enclosure at every update phase depending on the listener position. The coefficient shall be equal to 1.0 if the listener is inside the AE having the reverberator rlist: gOutr=rlist=1.0. For other reverberators gOutr≠rlist the gain shall be calculated linearly based on the distance of the listener from the enclosure. This shall be done as follows for gOutr≠rlist:

—	Obtain the AE enclosure associated with the reverberator r.

—	Calculate the Euclidean distance dlist,encr between the listener position and the closest point to the listener position on the AE enclosure

—	Calculate the gain gOutr=1-dlist,encr/fadeInDistance, where fadeInDistance is the fade-in distance for immersive reverberation.

In the explicit portal mode, the smooth transition shall be disabled when the listener distance from the closest portal is less than revExtentOffset defined in 6.6.5.2.

The Default AE shall be only audible if the listener is not in any bounded acoustic environment. The Default AE gain shall be faded in when the loudest bounded AE is faded out, and vice versa. The gain for the Default AE output shall be obtained as

| gOutr=rdefaultAE= min1.0,minr≠rdefaultAE1-gOutr≠rdefaultAE*defaultAEgain | (139) |
| --- | --- |

defaultAEgain is set from defaultAEGain unless overridden runtime.

When the gains gOutr are applied to immersive reverberation signals enveloping the listener a fade in/out effect when the listener is approaching/moving away from an acoustic environment is produced. This ensures smooth transitions to and from acoustic environments.

The immersive reverberation signals for each block of B samples shall be obtained as:

headtrackedOutputRenderItems(j,n,azi,ele)=array(0.0f, maxNumberOfOutputChannels*B)

for(r = 0; r < reverbCount; r++) {

if gOutr>0.0f {

outIndices = outputMapping[numDelayLines[r]];

for(j = 0; j < numDelayLines[r]; j++) {

outputChId = outIndices[j];

for(n = 0; n < B; n++) {

headtrackedOutputRenderItems(outputChId,n,azi,ele) += gOutr*srev,rj,n;

}

}

}

}

When the gain gOutr=0 for all reverberators, and portal representation is in explicit portal mode, the gain attribute of the headtrackedOutputRenderItems shall be set to 0. Otherwise it shall be set to 1.

The immersive reverberated signals headtrackedOutputRenderItems(j,n,azi,ele) shall be passed to Panner stage for immersive reverberation rendering.

The reverberated signals srev,rj,n shall be passed to the Portal stage for directional reverberation rendering via acoustic portals when needed.

Output signals of reverberators shall be stored to be used for reverberation of connected acoustic environments. The output signal is stored as

| srev,rn=j=1NDsrev,rj,n. | (140) |
| --- | --- |

To reduce excessive emphasis on low frequencies due to summing the reverberator outputs, a second-order IIR low-shelving filter having the coefficients b = [0.978, -1.9276, 0.9549] and a = [1.0, -1.9263, 0.9290] shall be applied to attenuate the low end of the spectrum. The filtered signal shall be denoted as srev,rn.

### Portals

#### General

This stage manages the activation and deactivation of two source types associated with portals. Firstly, reverberation extent sources so that reverberation is audible outside an acoustic environment, through acoustic openings (portals), as well as managing signal mixing for playback on these reverberation extent sources. A reverberation extent source is rendered as a Spatially Extended Sound Source (SESS), specifically Homogeneous Extended Sound Source, but of the 'DirectionalReverb' type, causing slightly different handling in subsequent stages.

Secondly, the stage manages coupling sources that render sources on opposite sides of portals with material, simulating the sound inducing vibrations in, for example, doors or windows causing them to become extended sources themselves.

The stage shall support two types of portals. Implicit portals are uniquely associated with an Acoustic Environment (AE), while explicit portals represent the actual portals between two environments. A scene shall only use one type. Coupling sources shall only be supported for explicit portal scenes.

Figure 19 and Figure 20 demonstrate the principle of an implicit portal. There are three AEs, illustrated as white-filled rectangles, in the example scene from Figure 19. In Figure 20, when the listener, illustrated as red sphere, is inside one of the AEs, the other two AEs are identified and rendered as portals, illustrated as blue rectangles. If there are acoustic paths from the portals to the listener position, the listener may hear the reverberation from the corresponding acoustic environment.

Figure 19 – Example of acoustic environments in a scene.

Figure 20 – Identification of active implicit portals based on the listener’s location.

Explicit portals define specific openings or other acoustic connections between two environments, as illustrated in Figure 21. If there are acoustic paths through the portals to the listener, the listener may hear the reverberation from the corresponding environment. This mode supports further multi-room reverberation features and rendering of coupled transmission.

Figure 21 – Example of explicit portals, identifying acoustic connections between two environments.

#### Data elements and variables

acEnvId 	Unique ID of each AE in the scene

bbPE	Bounding box object for the portal extent geometry rotated such that the portalNormal is aligned with the negative z axis, indicating minimum and maximum coordinate values in all dimensions.

portalWidth	Width of the portal.

portalHeight	Height of the portal

portalDepth	Depth of the portal

portalCenter	Centre coordinate of the portal extent.

revExtentOffset	Offset of the reverberation extent from the portal extent.

aeGeometry	Geometry defining the spatial region of an acoustic environment. Identified in the environments() syntax as acousticRegionId.

backFaceOffset	Offset in metres of additional faces for a reverberation extent mesh that form the back-side, ensuring a manifold mesh.

vertextToAdd	Vertex to be added to a reverberation extent mesh.

directionalStrengthCoeff

Strength coefficient in the range [0, 1], indicating how strong a reverberation extent source should be rendered, corresponding to 0- 100% of its amplitude.

reverbStrengthCoeff	Strength coefficient in the range [0, 1], indicating how strong a corresponding immersive reverberation should be rendered, corresponding to 0-100% of its amplitude.

ptListener	Listener position.

revExtentRIByAE 	Map storing key-value pairs where the key is the acEnvId of an AE, and the value is a vector of reverberation extent render items, whose audio signal shall be downmixed from the respective AE’s reverberator output.

reverbExtentRI	Reverberation extent render item, an extent source defined in the portal stage and representing reverberation from a specific acoustic environment for a specific portal.

allAcEnvIdsInScene 	A vector with the unique IDs of all AEs in the scene.

currentSignal 	An output signal frame (15 channels) from the current reverberator instance.

reverbSignalOutput 	A vector of output signal frames from all reverberator instances in the scene.

numDelayLines(r)	The number of outputs from reverberator instance r.

reverbExtentSrcSignalBuffer

The signal buffer of a reverberation extent RI.

#### Stage description

##### Portal payload decoding

###### Portal line-of-sight data decoding

The portal payload contains LoS data indicating the LoS from a grid of possible sound source positions towards each of the portals. The LoS grid supports both static and dynamic sources as long as they stay within the LoS grid area. Sources outside the LoS grid shall be considered to have a LoS.

The binary values at the voxel grids shall be divided into 2D planes and each plane shall be treated as a LoS bitmask and compressed. The compression shall be lossless run-length encoding and shall be decoded in the renderer. The decoding results are stored as binary vectors losVoxelGridp,area where a positive value indicates that there is a LoS at the area corresponding of the voxel to portal p. The area shall define a rectancular axes-aligned subregion of the scene covered by the LoS voxels. The area shall be be defined by its corner coordinates at decoded values of cornerP1 and cornerP2.

The number of bitmasks to be decoded shall be indicated by the decoded value of nLosBitMasks of losVoxelDataStruct Table 60. Each encoded bitmask shall contain a number of bytes indicated by the decoded value of nMaskBytes and the actual encoded bitmask data shall be a sequence of maskBytes stored into array maskBytes. Each encoded bitmask in maskBytes shall be decoded as follows:

Initial steps:

- 16-bit unsigned integers bitmask_width and bitmask_heigth shall be decoded from the maskBytes.

- A 2-dimensional vector variable reconBitmask with the size of bitmask_width and bitmask_heigth shall be constructed and all elements in reconBitmask set to zero.

- A Variable prevRow shall be initialized as a zero vector with the dimension of bitmask_width.

- Variables rowIdx, prevStart, prevEnd, and vIdx shall be set to zero.

Next, the LoS data entropy decoder shall be initialized by the following steps.

- Initialize vector variable CDV_URLC and CDV_SPRS as described in the Annex A.19.

- Initialize variable CDV_URLC_OFFSET and CDV_SPRS_OFFSET as the following:

CDV_URLC_OFFSET = 0

CDV_SPRC_OFFSET = -128

- Initialize LoS data decoder giving the rest of maskBytes data as the input variable bitstream following the process defined in 6.6.5.3.1.2.

After the LoS data entropy decoder is initialized, to decode the LoS data, the following steps shall be repeated until an exit condition is met. After the decoding, reconBitmask shall contain the reconstructed LoS data.

- decode a number n from LoS data entropy decoder following the entropy decoding process defined in 6.6.5.3.1.3 giving CDV_URLC and CDV_URLC_OFFSET as the input variables cdf and offset, respectively,

- set the next n rows in reconBitmask to be identical to prevRow, and increase rowIdx by n,

- if the rowIdx is equal or greater than bitmask_heigth, exit the loop,

- perform a decoding process (described below) to reconstruct a row from the payload, and copy reconstructed row to the current row in reconBitmask,

- copy the reconstructed row to variable prevRow,

- increase rowIdx by one, and

- if rowIdx is equal or greater than bitmask_heigth, exit the loop.

The decoding process reconstructs the LoS data for a layer of LoS voxels in the variable reconBitmask. The above process shall be repeated for all LoS bitmasks for portal p and the the reconstructed reconBitMasks shall be vectorized and appended one after another to create losVoxelGridp for portal p.

The following steps shall be performed to reconstruct a row from the payload:

- initialize variable reconRow as a zero vector with the dimension of bitmask_width,

- decodes a number from LoS data entropy decoder following the entropy decoding process defined in 6.6.5.3.1.3 giving CDV_SRPC and CDV_SRPC_OFFSET as input, and assign the sum of the decoded number and prevStart to variable startIndex,

- assign startIndex to prevStart,

- decode a number from LoS data entropy decoder following the entropy decoding process defined in 6.6.5.3.1.3 giving CDV_URLC and CDV_URLC_OFFSET as input, and assign the sum of the decoded number and variable startIndex to variable endIndex,

- copy the segment between the beginning to the element before startIndex from variable prevRow to variable reconRow,

- copy the segment between endIndex to the end of the row from variable prevRow to variable reconRow,

- initialize variable prevValue as the value of the element prevRow[startIndex],

- initialize variable pixelIndex as the value of variable startIndex,

- while variable pixelIndex is less than variable endIndex:

  - decode a number n from LoS data entropy decoder following the entropy decoding process defined in 6.6.5.3.1.3 giving CDV_URLC and CDV_URLC_OFFSET as input,

  - assign the next n elements in reconRow starting from index pixelIndex to the value of 1 - prevValue,

  - set variable prevValue to 1-prevValue, and increase variable pixelIndex by n.

- return variable reconRow

###### LoS data entropy decoder initialization

Given input variable bitstream as a vector of 32-bit unsigned integer numbers, LoS data entropy decoder shall be initialized as the following where variable rans_state is a 64-bit unsigned integer.

rans_state  =  (bitstream[0]) << 0;

rans_state |=  (bitstream[1]) << 32;

bitstream_index = 2;

###### RANS entropy decoding process

Given vector variable cdf, and variable offset a symbol shall be decoded from the initialized LoS data entropy decoder by the following procedure.

decode_symbol(cdf, offset)

max_value = length(cdf) - 2;

value = entropy_decode_symbol(cdf);

real_value = 0;

if (value == max_value) {

real_value = decode_oor_bypass(max_value, offset);

}

} else {

real_value += value + offset;

}

return real_value;

}

The decode_oor_bypass  process shall follow the following procedure.

decode_oor_bypass(max_value, offset) {

bypass_precision = 3;

max_bypass_val = (1 << bypass_precision) – 1;

val = decode_bits(bypass_precision);

n_bypass = val;

while (val == max_bypass_val) {

val = decode_bits(bypass_precision);

n_bypass += val;

}

raw_val = 0;

for (j = 0; j < n_bypass; ++j) {

val = decode_bits(bypass_precision);

raw_val |= val << (j * bypass_precision);

}

value = raw_val >> 1;

if (raw_val & 1) {

value = -value - 1;

} else {

value += max_value;

}

return value + offset;

}

The decode_bits process shall follow the following procedure.

decode_bits(n_bits) {

x = rans_state;

val = rans_state & ((1 << n_bits) – 1);

RANS64_L = 1 << 31

x = x >> n_bits;

if (x < RANS64_L) {

x = (x << 32) | bitstream[bitstream_index];

bitstream_index += 1;

}

rans_state = x;

return val;

}

The entropy_decode_symbol process shall follow the following procedure.

entropy_decode_symbol(cdf) {

cum_freq = rans_state & ((1 << 16) - 1);

symbol = find_index_of_first_element_greater_than_a_value(cdf, cum_freq) - 1

start = cdf[symbol]

freq = cdf[symbol+1]

mask = (1 << 16) - 1;

RANS64_L = 1 << 31

uint64_t x = rans_state;

x = freq * (x >> 16) + (x & mask) - start;

if (x < RANS64_L) {

x = (x << 32) | bitstream[bitstream_index];

bitstream_index += 1;

}

rans_state = x;

return symbol

}

Process find_index_of_first_element_greater_than_a_value shall return the index of the first element in an input vector with the value greater than a given input value.

##### Stage initialization

During the initialization, the data of all portals and their associated acoustic environments shall be parsed from the bitstream. For implicit portal mode the reverberation extent geometry for each portal shall be provided in the bitstream (i.e., portalExtentId), while for explicit portal mode, the extent geometries shall be derived from the portal metadata. One for each acoustic environment associated with the portal.

Determining the reverberation extent geometries from the portal metadata shall be done according to the following process.

First, a reverberation extent offset shall be calculated. The initial offset shall be dependent on the smallest width or height dimension of the portal.

The dimensions of the portal may be provided explicitly in the portal payload (explicitPortalProperties = true). In that case the portal dimensions shall be:

portalWidth = equivalentPortalWidth;

portalHeight = equivalentPortalHeight;

portalDepth = equivalentPortalDepth;

Otherwise, the dimensions shall be derived from the portal extent geometry indicated by portalExtentId.

The extent geometry shall be rotated without roll such that the portalNormal is aligned with the negative z axis. The dimensions of the bounding box determine the portal dimensions.

portalWidth  = bbPE.xMax – bbPE.xMin;

portalHeight = bbPE.yMax – bbPE.yMin;

portalDepth  = bbPE.zMax – bbPE.zMin;

Similarly, the portal centre position shall be taken directly from the portal payload when included and otherwise derived from the bounding box.

if (explicitPortalProperties) {

portalCenter = portalCenterPos;

} else {

portalCenter.x = (bbPE.xMax + bbPE.xMin) / 2;

portalCenter.y = (bbPE.yMax + bbPE.yMin) / 2;

portalCenter.z = (bbPE.zMax + bbPE.zMin) / 2;

}

In case explicit portal properties are provided in the portal payload, but no portal extent geometry, a Box geometry (eqBoxExtent) shall be created instead. A new ID shall be generated, the position set to the portal position (portalCenter), and the [x, y, z] dimensions to [portalWidth, portalHeight, portalDepth]. The orientation shall be yaw and pitch required to rotate the negative z-axis to align with the portal normal, and zero roll.

With that, the initial reverberation extent offset shall be calculated with:

initialRevExtentOffset = max(0.8 * min(portalWidth, portalHeight), 0.3);

Subsequently, the reverberation extent offset shall be reduced when necessary such that it is not more than half the available space in the acoustic environment.

For each corner of the portal’s (box-equivalent) extent, the acoustic environment shall be probed whether it provides enough space, and adjusted if it doesn’t, according to the following pseudo-code.

revExtentOffset = initialRevExtentOffset;

checkCornersLocal[0] = {-portalWidth / 2,  portalHeight / 2, -portalDepth / 2 – 2 * initialRevExtentOffset};

checkCornersLocal[1] = {-portalWidth / 2, -portalHeight / 2, -portalDepth / 2 – 2 * initialRevExtentOffset};

checkCornersLocal[2] = { portalWidth / 2,  portalHeight / 2, -portalDepth / 2 – 2 * initialRevExtentOffset};

checkCornersLocal[3] = { portalWidth / 2, -portalHeight / 2, -portalDepth / 2 – 2 * initialRevExtentOffset};

checkCornersGlobal[0]  = globalCSTransform * checkCornersLocal[0];

checkCornersGlobal[1]  = globalCSTransform * checkCornersLocal[1];

checkCornersGlobal[2]  = globalCSTransform * checkCornersLocal[2];

checkCornersGlobal[3]  = globalCSTransform * checkCornersLocal[3];

for (c = 0; c < 4; c++) {

if (!aeGeometry.isInside(checkCornersGlobal[c])) {

globalClosest = aeGeometry.closestPointTo(checkCornersGlobal[c]);

localClosest = localCSTransform * globalClosest;

revExtentOffset = min(initialRevExtentOffset, -((localClosest[2] + hd) / 2));

}

}

Where globalCSTransform is the transform matrix for the portal that transforms local coordinates into global coordinates, and localCSTransform is the transform matrix for the portal that transforms global coordinates into local coordinates for the portal extent geometry.

The Geometry object aeGeometry is the geometry of the acoustic environment in which the reverberation extent is created.

Two reverberation extent meshes shall be constructed at the portalCenter using the vertices and face descriptions from A.16. The vertices shall be defined relative to the portalCenter using the portalWidth, portalHeight, and portalDepth of the portal, as well as the revExtentOffset as given in A.16. By default, three segments shall be used, unless a lower quality to complexity trade-off is required. The orientation of the first mesh shall be such that the resultant mesh is perpendicular to the normal axis of the portal, without roll rotation, and this mesh shall be associated with the acEnvId of the acoustic environment within which it is contained. The second mesh shall be orientated such that it is perpendicular to the inverse of the portal normal axis, without roll rotation, and it shall be associated to the acEnvId of the acoustic environment within which it is constrained.

Figure 22 – Cross-section of a 3 segment reverberation extent with a portal (dashed box) and walls (gray), no backfaceOffset.

The reverberation extent meshes shall be defined as either the default mesh as defined in A.16, which are not a closed mesh, or as having a back face defined by backfaceOffset = 0.05. If backfaceOffset is specified additional vertices are added to the mesh. For every vertex in the mesh the vector from the mesh origin to the vertex location is calculated, and the additional vertex location is calculated as the location at backfaceOffset further along the vector. Faces shall be added to create a manifold mesh using the newly added vertices, with the normal of every face pointing out of the mesh.

The reverberation extent meshes shall be checked to ensure that they are not occluded from the nearest point of the portal by any acoustic environment enclosure. This shall be accomplished by casting a ray from the nearest vertex of the box-equivalent of the portal to the reverberation extent vertex, and checking for intersections. The new position of the vertex shall be moved towards the ray origin if it is beyond or too close to a geometry.

If the mesh is defined with a backfaceOffset, the vertices that describe the inner faces of the mesh (insideFace = true) shall be moved additionally so as to maintain the backfaceOffset distance with respect to the outer vertices whenever possible.

The check and adjustment of vertexToAdd taken from a table in A.16 shall be performed as described by the following pseudo code.

portalVertexLocal[0] = {-portalWidth / 2,  portalHeight / 2, -portalDepth / 2};

portalVertexLocal[1] = {-portalWidth / 2, -portalHeight / 2, -portalDepth / 2};

portalVertexLocal[2] = { portalWidth / 2,  portalHeight / 2, -portalDepth / 2};

portalVertexLocal[3] = { portalWidth / 2, -portalHeight / 2, -portalDepth / 2};

minDistance = infinity;

for (i = 0; i < 4; i++) {

distance = abs(portalVertexLocal[i] - vertexToAdd)

if (distance < minDistance) {

distance = minDistance;

closestPortalVertexLocal = portalVertexLocal[i];

}

}

closestPortalVertexGlobal = globalCSTransform * closestPortalVertexLocal;

globalVertex = globalCSTransform * vertexToAdd;

vertexDirection = globalVertex - closestPortalVertexGlobal;

vertexDistance = abs(vertexDirection);

distanceToGeometry = raytrace(closestPortalVertexGlobal, vertexDirection,

AE_Enclosures);

if (isInsideFace == true) {

targetDistance = minDistInside + backfaceOffset;

} else {

targetDistance = minDistInside;

}

if (distanceToGeometry >= vertexDistance + targetDistance) {

constrainedDistance = vertexDistance;

} else if (distanceToGeometry >= targetDistance) {

constrainedDistance = distanceToGeometry - targetDistance;

} else if (distanceToGeometry >= backfaceOffset) {

if (insideFace == true) {

constrainedDistance = 0;

} else {

constrainedDistance = backfaceOffset;

}

} else {

if (insideFace == true) {

constrainedDistance = 0;

} else {

constrainedDistance = distanceToGeometry;

}

}

globalVertexToAdd = closestPortalVertexGlobal + (vertexDirection / vertexDistance) *

constrainedDistance;

vertexToAdd = localCSTransform * globalVertexToAdd;

Constant minDistInside = 0.15 m, abs(x) returns the Euclidian norm of vector x, and raytrace(x, y, geometry) returns the distance along the ray starting at point x in the direction y until the first intersection with geometry, or infinity if such an intersection does not exist. AE_Enclosures is a list of all geometries used for constraining the reverberation extent vertices. For portal reverberation extents that are inside an acoustic environment, AE_Enclosures shall contain only that acoustic environment enclosure. For portal reverberation extents that are not inside an acoustic environment, AE_Enclosures shall contain the acoustic environment enclosures of all acoustic environments.

The newly calculated vertex position shall be checked against the pre-existing vertices in the mesh, and if a vertex already exists within 0.001m of the vertex to be added, the pre-existing vertex shall be used instead. If reusing a vertex in such a way would result in a face with two or more identical vertices, that face shall be omitted from the mesh.

As described in 6.6.5.3.3, the portal stage adds reverberation extent render items of type 'DirectionalReverb' to the render list for further processing by the discover SESS stage and homogeneous extent stages. In implicit portal mode, the render item shall be configured with the portal extent geometry carried in the bitstream as the render item’s extent, representing a shrunken version of the acoustic environment geometry.

The position shall be set the same as the position of the portal extent geometry, the reference distance be set to 1m, the status initialized to Inactive, aparams set to ForceNoDoppler, the acEnvId to the ID of the corresponding acoustic environment and the parentPortalId set to the corresponding portal’s ID.

For explicit portal mode, there shall be up to two reverberation extent render items of type ‘DirectionalReverb’, one for each acoustic environment associated with the portal. The extents of these render items shall be the corresponding reverberation extent geometries as derived according to above description.

In these render items the position shall be the centre position of the portal, the reference distance set to the corresponding revExtentOffset, the status initialized to Inactive, aparams set to ForceNoDoppler, the acEnvId to the ID of the corresponding acoustic environment and the parentPortalId set to the corresponding portal’s ID.

For the rendering of coupling sources, when enabled (Portal_Enable_Coupling = true), extent source render items shall be created using the portal geometry, indicated by portalExtentId, as the extent. In case no portal geometry is provided (explicitPortalProperties = true), the equivalent box extent (eqBoxExtent) is used.

The coupling source render items shall be of type 'Primary', subtype 'CoupledSource', position set to the portal extent position, aparams set to NoDoppler and EQ set to the square root of the coupling coefficients of the current portal material. The coefficients shall be interpolated to the centre frequencies used by the EQ stage (6.6.20) as described in sub-clause 6.6.20.1.

##### Configuration update for external reverberation

For each update, the portal stage shall activate and/or deactivate render items of type 'DirectionalReverb' based on the position of the listener. In implicit portal mode, all reverberation extent render items associated with an acoustic environment (given by portalParentEnvId) that is not the acoustic environment with the listener, shall be activated with full strength (portalStrengthCoeff = 1). Otherwise, it shall be deactivated.

For explicit portal mode, there shall be up to two reverberation extent render items per portal, one for each acoustic environment associated with the portal.

If the listener is inside the portal extent, all associated reverberation extent render items shall be activated with full strength (portalStrengthCoeff = 1), while the immersive reverberation render items shall be fully muted (reverbStrengthCoeff = 0).

If the listener is in the acoustic environment of a reverberation extent render item, the reverberation extent render item shall be deactivated, unless the listener is inside the cross-fade region. No modifications shall be made to the immersive reverberation render items.

A listener shall be denoted as being in the cross-fade region when the listener’s closest distance to the corresponding portal extent is less than or equal to the reverberation extent offset for that acoustic environment. If the listener is in the cross-fade region of a reverberation extent render item, cross-fade coefficients (portalStrengthCoeff, reverbStrengthCoeff) shall be calculated.

The cross-fade gain calculation shall be based on the listener position (ptListener), the point on the portal extent closest to the listener (ptOnPortal) and the point on the reverberation extent closest to the listener (ptOnRevExt). The calculation shall be conformant with the following pseudo code.

n = ptOnPortal – ptOnRevExt;

n = n / abs(n);

alpha = dot(ptOnRevExt – ptListener, n);

pa = ptOnRevExt – alpha * normal;

crossFadeCoeff = 0.0;

d1 = ptOnRevExt – pa;

d2 = ptOnPortal – pa;

if (dot(d1, d2) < 0) {

l1 = abs(ptOnRevExt – ptListener);

l2 = abs(ptOnPortal – ptListener);

crossFadeCoeff = l2 / (l1 + l2);

} else {

if (abs(d1) < abs(d2)) {

crossFadeCoeff = 1;

} else {

crossFadeCoeff = 0;

}

}

reverbStrengthCoeff = sin(crossFadeCoeff * PI / 2);

portalStrengthCoeff = sin((1 – crossFadeCoeff) * PI / 2);

where abs(x) returns the Euclidian norm of vector x, dot(x, y) returns the dot product of x and y and PI = .

If the listener is outside the acoustic environment of a reverberation extent render item and the listener is at the other side of the corresponding portal, spatially, and the listener acoustic environment is in the other acoustic environment associated with the corresponding portal, or there is a second portal connecting the listener acoustic environment to the other acoustic environment associated with the corresponding portal, the reverberation extent render item shall be activated with full strength (portalStrengthCoeff = 1) when the largest transmission coefficient of the portal material (or combined portal material in the case there is a second portal) is larger than minTransmissionForRevExtent, or when the portal has no material. No modifications shall be made to the immersive reverberation render items. minTransmissionForRevExtent is the threshold for enabling reverberation extent sources behind a portal with material, its value is: 10-6010.

The largest transmission coefficient of combined portal material shall be determined by multiplying the transmission coefficients of both portals at each frequency listed by either of the portal materials and taking the largest combined transmission coefficient. If the frequency grids are not the same, the transmission coefficient shall be interpolated for the material in which a frequency was not represented.

The determination of the listener spatially being at the other side of a portal shall be done as follows:

If the acoustic environment of the reverberation extent render item is the same as portalEnv1 of the corresponding portal:

portalOffsetPosition = portalCenter + portalNormal * portalDepth/2;

if ((listenerPosition – portalOffsetPosition).dot(portalNormal) >= 0) {

return false;

} else {

return true

}

otherwise

portalOffsetPosition = portalCenter – portalNormal * portalDepth/2;

if ((listenerPosition – portalOffsetPosition).dot(portalNormal) >= 0) {

return true;

} else {

return false

}

In all other cases, the reverberation extent render item shall be deactivated. No modifications shall be made to the immersive reverberation render items.

When the listener is in more than one cross-fade region for reverberation extent render items associated with the same acoustic environment, the strength coefficients for all involved reverberation extent render items Pcf and the related immersive reverberation render item shall be adjusted according to the following equations.

| directionalStrengthCoeffp=directionalStrengthCoeffp*maxDirectionalPortiontotalDirectionalPortion | (141) |
| --- | --- |
| reverbStrengthCoeff=1-maxDirectionalPortion2 | (142) |

where

| maxDirectionalPortion=maxp ∈ Pcf(directionalStrengthCoeffp)⁡ | (143) |
| --- | --- |
| totalDirectionallPortion=p ∈ PcfdirectionalStrengthCoeffp | (144) |

The strength coefficient shall be applied as an additional gain for the corresponding render item

RI[p].gain *= portalStrengthCoeff[p];

RI[immRev].gain *= reverbStrengthCoeff;

A reverberation extent render item is a representation of an acoustic environment, so the audio signal for the reverberation extent render items is copied from the reverberator output of the corresponding AE.

##### Configuration update for coupled transmission

When portals have a material with sufficiently strong coupling coefficients, they can be rendered as extent sources, representing coupled transmission between environments. For this, coupling sources shall be introduced at the portal geometries.

When rendering of coupling sources is enabled (Portal_Enable_Coupling = true) the following shall be done to generate the corresponding render items.

For each portal, an active render item for a coupling source is added when the portal has a material with sufficiently strong coupling coefficients and the listener is either in one of the acoustic environments  related to the portal of interest (portalEnv1 or portalEnv2), or the listener is in an acoustic environment that is indirectly connected, via one other portal, to one of the acoustic environments related to the portal of interest.

A material's coupling coefficients are considered sufficiently strong when the maximum coupling coefficient is larger than couplingAbsThreshold and the ratio of the maximum coupling coefficient to the maximum transmission coefficient is larger than couplingRelThreshold.

couplingAbsThreshold = 10-6010

couplingRelThreshold = 10-610

In other cases, the coupling source render items shall be deactivated.

The EQ of the coupling source render item shall be set to the square root of the coupling coefficients of the current portal material. The coefficients shall be interpolated to the centre frequencies used by the EQ stage (6.6.20) as described in sub-clause 6.6.20.1.

From the acoustic environments related to the portal of interest coupling input AEs shall be selected for each coupling source. In cases where the listener environment is directly connected to the portal, the other environment shall be the only coupling input AE. In case where the listener environment is indirectly connected, as described above in this sub-clause, either the portal environment through which the listener environment was not connected, or both portal environments shall be the coupling input AEs. Both portal environments shall be selected when both provide an indirect connection to the listener environment.

For active coupling source render items, or deactivated render items that are still fading out, a coupling downmix shall be created from sources and reverberation in the selected coupling input AEs.

Any render item (i) of type 'Primary', of which the state is not Inactive and located in one of the input AEs shall be added to the downmix for the coupling source render item associated with portal p with downmix coefficient:

| cplDmxi,p=itemGaini*portalFactori,p | (145) |
| --- | --- |

where itemGaini shall be the render item gain and portalFactori,p the portal contribution of render item i to portal p according to equation (136).

If there are no render items that would contribute to the downmix, and the coupling input AEs have no associated, active reverberators, the corresponding coupling source render item may be deactivated.

##### Audio processing

There are originally numDelayLines(r) output channels from each reverberator instance r, and only two signals are needed for rendering a portal as Homogeneous Extent. These two signals bypass the decorrelator step in Homogeneous Extent stage and go through the mixing explained in 6.6.24.3 and illustrated in Figure 80. To generate two such downmix signals yoddn and yevenn from the reverberator outputs, the signals shall be downmixed for each reverberator that is used for a reverberation extent source. In such case, each reverberator output channel shall be downmixed into one of the two channels of the reverberation extent RI. The odd and even channels shall be summed according to Equation (146) and (147).

| yodd(n) = c = 0C/2 - 1 x2c+1(n) | (146) |
| --- | --- |
| yeven(n) = c = 0C/2 - 1 x2c(n) | (147) |

where C=numDelayLines – 1. The last channel xC(n) shall be mixed to both channels to preserve energy: yoddn=yoddn+0.5*xC(n) and yevenn=yevenn+0.5*xC(n) .

As mentioned in the Update Thread Processing description above, the signal output of each reverberator instance shall be mapped to a corresponding reverberation extent RI.

for acEnvId in allAcEnvIdsInScene{

currentSignal = reverbSignalOutput[acEnvId];

for reverbExtentRI in revExtentRIByAE[acEnvId]{

reverbExtentSrcSignalBuffer[reverbExtentRI.id].copyFrom(currentSignal);

}

}

When rendering of coupling sources is enabled (Portal_Enable_Coupling = true) the following shall be done to generate the signal for each of the coupling source render items that are not Inactive.

The signals of all render items of type 'Primary', of which the state is not Inactive and located in one of the input AEs shall be downmixed to a single signal. Each render item signal shall be multiplied with the corresponding downmix coefficient cplDmxi,p.

For each reverberator related to one of the coupling input AEs of a coupling source, all signals shall be downmixed to mono (according to the downmix described above in this sub-clause, except that all signals mix to a single output signal) and added to the downmix from the corresponding Primary render items. The resulting signal shall be used as the signal of the coupling source render item.

### Early reflections

#### General

The Early Reflection (ER) stage utilizes the transmitted geometry data to compute specular reflections from reflecting surfaces. An image source model is used for checking the visibility of potential propagation paths from the sound sources to the listener. Since this process involves sending a large number of rays from the listener to the potential image source locations and checking whether the correct surfaces are hit, pre-computed voxel data is used to speed up this procedure. The voxel data contains a comparably shorter list of potentially valid propagation paths for a given cubic source and listener area, relative to the large number of rays. It is generated by and transmitted from the encoder for selected source and listener voxels. This allows to render second order reflections in real time even in complex geometric environments. If no voxel data is available, only first order reflections are generated. The Early Reflection stage also supports sound transmission through partly occluding (transmissive) materials and uses reflection and transmission filters to model the sound propagation.

#### Terms and definitions

##### Voxel grid

The renderer uses voxel data to speed up the computational complex visibility check of reflected sound propagation paths. The scene is rasterized into a regular grid with a grid spacing that can be defined individually for each dimension. Each voxel is identified by a unique voxel ID and a sparse database is used to store pre-computed data for a given source/listener voxel pair. The relevant variables and data structures are specified in clause 6.3.2.5:

- earlyVoxelGridOriginX

- earlyVoxelGridOriginY

- earlyVoxelGridOriginZ

- earlyVoxelGridPitchX

- earlyVoxelGridPitchY

- earlyVoxelGridPitchZ

- earlyVoxelGridShapeX

- earlyVoxelGridShapeY

- earlyVoxelGridShapeZ

These variables are the basis for voxel coordinates V = [vx, vy, vz]T with 3 integer numbers as components. For any point P = [px, py, pz]T located in the scene, the corresponding voxel coordinate is computed by the following rounding operations to the nearest integer number:

| vx=round( (px-earlyVoxelGridOriginX) / earlyVoxelGridPitchX) | (148) |
| --- | --- |
| vy=round( (py-earlyVoxelGridOriginY) / earlyVoxelGridPitchY) | (149) |
| vz=round( (pz-earlyVoxelGridOriginZ) / earlyVoxelGridPitchZ) | (150) |

A voxel coordinate can be converted into a voxel index n:

| n=vx+earlyVoxelGridShapeX*(vy+earlyVoxelGridShapeY*vz) | (151) |
| --- | --- |

This representation is for example used in the sparse voxel database earlyVoxelDatabase[l][s][p] for the listener voxel ID l and the source voxel ID s.

##### Culling distances

The encoder can use source and/or triangle distance culling to speed up the pre-computation of voxel data. The culling distances are encoded in the bitstream to allow the renderer to smoothly fade-out reflections that reach the used culling thresholds. The relevant variables and data structures are specified in clause 6.3.2.5:

- earlyTriangleCullingDistanceOrder1

- earlyTriangleCullingDistanceOrder2

- earlySourceCullingDistanceOrder1

- earlySourceCullingDistanceOrder2

##### Surface data

Surface data is geometrical data which defines the reflection planes on which sound is reflected. It is specified in Hesse normal form. The relevant variables and data structures are specified in Section 6.3.2.5:

- earlySurfaceIdx[s];

- earlySurfaceFaceIdx[s][f];

- earlySurface_N0[s]

- earlySurface_d[s]

The surface index earlySurfaceIdx[s] identifies the surface and is referenced by the sparse voxel database earlyVoxelDatabase[l][s][p]. The triangle ID list earlySurfaceFaceIdx[s][f] defines the triangles of the static mesh which belong to this surface. One of these triangles must be hit for a successful visibility test of a specular planar reflection (see Section 6.6.6.3.4). The reflection plane of each surface is given in Hesse normal form using the surface normal N0 and the surface distance d which are converted as follows:

int max_steps_azi = 1 << 12;

int max_steps_ele = 1 << 11;

int num_steps_azi = 144 * (max_steps_azi / 144);

int num_steps_ele = 72 * (max_steps_ele / 72);

int shift_ele = num_steps_ele / 2;

float quant2azi = double(2.0 * M_PI) / double(num_steps_azi);

float quant2ele = double(M_PI) / double(num_steps_ele);

float quant2dist = 0.001f;

for (int s = 0; s < numberOfSurfaces; s++) {

earlySurfaceIdx[s] = s;

float azi = earlySurfaceAzi[s] * quant2azi;

float ele = (earlySurfaceEle[s] - shift_ele) * quant2ele;

earlySurface_N0[s][0] = -1.0 * sin(azi) * cos(ele);

earlySurface_N0[s][1] = sin(ele);

earlySurface_N0[s][2] = -1.0 * cos(azi) * cos(ele);

earlySurface_d[s] = (earlySurfaceDist[s] + earlySurfaceDistOffset) *
                       quant2dist;

}

where M_PI = π.

##### Voxel data

Early Reflection Voxel Data is a sparse voxel database containing lists of reflection sequences of potentially visible image sources for given pairs of source and listener voxels. The entries of the database can either be undefined for the case that the given pair of source and listener voxel is not specified in the bitstream, they can be an empty list, or they can contain a list of surface IDs. The relevant variables and data structures are specified in Section 6.3.2.5:

- numberOfVoxelPairs

- earlyVoxelL[v]

- earlyVoxelS[v]

- earlyVoxelMode[v]

- earlyVoxelIndicesRemovedDiff[v][k]

- earlyVoxelNumPaths[v]

- earlyVoxelOrder[v][p]

- earlyVoxelSurf[v][p][o]

The sparse voxel database earlyVoxelDatabase[l][s][p] is derived from these variables by the following algorithm:

int delta_x = voxelCoordinateToVoxelIndex( {1, 0, 0} );

int delta_y = voxelCoordinateToVoxelIndex( {0, 1, 0} );

int delta_z = voxelCoordinateToVoxelIndex( {0, 0, 1} );

int delta_list[4] = { 0, -delta_x, -delta_y, -delta_z };

for (int v = 0; v < numberOfVoxelPairs; v++) {

PathList path_list;

int l = earlyVoxelL[v];

int s = earlyVoxelS[v];

int mode = earlyVoxelMode[v];

if (mode != 0) {

int l_ref = l + delta_list[mode];

path_list = earlyVoxelDatabase[l_ref][s];

// generate list with removed items in reverse order

int numberOfIndicesRemoved =
			length(earlyVoxelIndicesRemovedDiff[v]) - 1;

int listIndicesRemoved[numberOfIndicesRemoved];

int val = -1;

for (int k = 0; k < numberOfIndicesRemoved; k++) {

val += earlyVoxelIndicesRemovedDiff[v][k];

listIndicesRemoved[numberOfIndicesRemoved - 1 - k] = val;

}

// remove reflection sequences

for (int k = 0; k < numberOfIndicesRemoved; k++) {

path_list.erase(listIndicesRemoved[k]);

}

}

// add reflection sequences

for (int p = 0; p < earlyVoxelNumPaths[v]; p++) {

path_list.append(earlyVoxelSurf[v][p]);

}

// add sorted path list to sparse voxel database

path_list = shortlex_sort(path_list);

int num_paths = length(path_list);

for (int p = 0; p < num_paths; p++) {

earlyVoxelDatabase[l][s][p] = path_list[p];

}

}

In this algorithm, the function voxelCoordinateToVoxelIndex()denotes the voxel coordinate to voxel index conversion specified in Equation (151). The keyword PathList denotes a list of integer arrays which can be modified by the method append(), that adds an element at the end of the list, and the method erase(), that removes a list element at a given position. Furthermore, the function shortlex_sort() denotes a sorting function which sorts the given list of reflection sequences in shortlex order.

#### Stage description

##### General

The processing of specular early reflections consists of two components running in two different threads. The first component running in the update thread is responsible for generating and updating RIs for early reflections. The details are described in the following sections.

The second component which runs within the DSP thread is responsible for generating a binaural audio signal based on these RIs. The details of this component are described in the clause 6.7.1.

The Early Reflection stage shall be used if the bitstream flag useLowComplexityER in the mpegSceneConfig() is set to false. If useLowComplexityER is set to true, the Early Reflection stage shall be skipped in the rendering pipeline, as the early reflections are handled by the Low Complexity Early Reflection stage.

##### Render item data

The Early Reflection stage generates and updates RIs which contain the following rendering parameters:

- Image source location and orientation

- RI EQ

- RI gain

How the image source location and orientation of reflections on the planar surfaces of triangle meshes are computed is described in clause 6.6.6.3.3. The corresponding visibility check is described in clause 6.6.6.3.4. The computation of image source locations and orientations of reflections on mesh edges and corners of “rounded” surfaces are described in clause 6.6.6.3.5. Clause 6.6.6.3.6 describes the computation of image source locations and orientations of reflections on cylinders.

The synthesis of the RI EQ is described in clause 6.6.6.3.7.

The computation of the RI Gain is described in clause 6.6.6.3.8.

Within these paragraphs the variables described in clause 6.3.2.4 are used which are derived from the bitstream elements described in clause 6.3.2.5.

##### Planar reflections

The first type of reflections which are handled by the Early Reflection stage are specular reflections on planar surfaces. The image source model (Allen [5]) is used to determine image sources, i.e. mirrored versions of a sound source located at position P. The locations of the image sources are computed by the following formula using the Hesse normal coefficients N0 and d of a given reflection plane:

| mirror(P, N0, d)=P+2 (d-P*N0)*N0 | (152) |
| --- | --- |

Higher order image source locations are computed by sequentially applying mirror(P, N0, d) in the corresponding order.

Since the Directivity stage (see Clause 6.6.16) requires the orientation of the RIs, the orientation of the image sources is needed in addition to the location of the image sources. And because the orientation of odd-order image sources cannot be represented by rotations given as Euler angles, a homogeneous transformation matrix is used to specify the orientation of the image sources. The homogeneous transformation matrix is constructed using the following base vectors:

X0 = [1, 0, 0]T

Y0 = [0, 1, 0]T

Z0 = [0, 0, 1]T

O0 = [0, 0, 0]T

Sequentially applying the mirror() function using the Hesse normal coefficients of the corresponding reflection planes yields the mirrored base vectors:

Xn+1 = mirror(Xn, N0,n, dn)

Yn+1 = mirror(Yn, N0,n, dn)

Zn+1 = mirror(Zn, N0,n, dn)

On+1 = mirror(On, N0,n, dn)

The homogeneous transformation matrix which combines the complete process is given as follows where N denotes the reflection order:

M = [XN - ON, YN - ON, ZN - ON, ON]

##### Visibility check

The Early Reflection stage renders only image sources which are “visible”, i.e. image sources with a propagation path that is not blocked by non-transmissive materials and where the sound is reflected on the “positive” side of the geometry (in direction of the surface normal). A visibility check is performed to check if this condition is fulfilled as illustrated in Figure 23.

Figure 23 — Visibility Check

The sparse voxel database earlyVoxelDatabase[l][s][p] is used to speed up the visibility check by limiting the checks to those reflection sequences which are stored for the listener located in the voxel with voxel ID l and for the source located in the voxel with voxel ID s.

If the sparse voxel database earlyVoxelDatabase[l][s][p] has no entry for the voxel pair <l,s>, then only the visibility of all first order reflections are checked. In this case only first order reflections are generated.

##### Rounded edges and corners

If the geometry data approximates curved surfaces by triangles or rectangles, the classic geometrical acoustics methods are no longer valid and artefacts become audible. The resulting “disco ball effect” is illustrated in Figure 24. For a moving listener or a moving sound source the visibility of the image source will alternate between visible and invisible, resulting in a permanently switching localization, timbre, and loudness.

Figure 24 — Illustration of the “disco ball effect” (the reflecting surfaces are sketched in black, gray areas mark the regions where the n-th image source “S|n” is visible, “S” marks the source location, and “L” marks the listener location)

The renderer uses a mitigation technique to reduce this effect. The applied method involves two components:

- A geometry pre-processor which detects curved surfaces (“round edges or round corners”)

- An extended image source model for the identified curved surfaces (“round edges or round corners”)

The geometry pre-processor is executed once during initialization as it does not depend on the listener or source locations. The extended image source model is executed at run-time and determines edge and corner reflections depending on the listener and source locations.

###### Curved surface detection (geometry pre-processor)

The geometry pre-processor implements the round edge and round corner detector using the triangle mesh. This is done by computing the angle between two adjacent triangles, here called face angle (see Figure 25) and if this angle is within a range of 0.1° … 31°, the adjacent edge and both triangles are considered to represent a curved surface section and are marked as such. If all edges which are connected to a corner are marked as being round, also the corner is marked as being round.

Figure 25 — Face angle spanned by two adjacent faces in a triangle mesh

###### Extended image source model for round edges and round corners

The extended image source model needs to extrapolate the image source location in the “dark zone” of the reflectors, i.e. the areas between the “bright zones” in which the image source is visible (see Figure 26). To do so, a frustum is created for each round edge and it is checked, if the listener is located within this frustum (indicated by the white area in Figure 26).

Figure 26 — Edge frustum (blue area)

The frustum is created as illustrated in Figure 26: For the two adjacent planes of the edge, namely the left and the right plane, the image sources SL and SR are computed by mirroring the source on the left and the right plane. From these points together with the beginning and the end point of the edge four planes k∈1,4 in Hesse normal form are obtained where the normal vectors Nk are pointing inside of the frustum:

| NkX-dk=0 | (153) |
| --- | --- |

If the distance

| lk=NkL-dk | (154) |
| --- | --- |

is greater than or equal zero for all 4 planes, then the listener is located within the frustum that defines the coverage area of the model for the given round edge. In this case, the reflection point on the round edge is determined as follows:

If the listener is located within the frustum,  the orthogonal projection PS of the source location S onto the edge and the orthogonal projection PL  of the listener location L onto the edge is computed. This yields the reflection point R as follows:

| dS=¦PS-S¦ | (155) |
| --- | --- |
| dL=¦PL-L¦ | (156) |
| R=dSdS+dLPL+dLdS+dLPS | (157) |  |

Figure 27 illustrates the geometric construction principle.

Figure 27 — Determination of the edge reflection point

From the reflection point R the Hesse-Normal coefficients N0 and d are computed as follows:

| N=S-RS-R+L-RL-R | (158) |
| --- | --- |
| N0=NN | (159) |
| d=N0 R | (160) |

The image source location and orientation is then computed from the Hesse normal coefficients N0 and d by the same method that is used for the planar reflections described in clause 6.6.6.3.3.

Reflections on round corners are handled in the same way. Here, the k adjacent planes yield k image sources which together with the corner location result in a frustum that is bounded by k planes. Again, if the distances of the listener to these planes are all greater than or equal zero, the listener is located within the coverage area of the round corner. Figure 28 illustrates the frustum from a round corner with 4 adjacent round edges.

Figure 28 — Corner frustum (blue area)

With the round corner being the reflection point R, the reflection plane with Hesse normal coefficient N0 and d is computed as described in the previous paragraph. The image source location and orientation is then computed in the same way.

##### Cylinder reflections

The previous sections considered reflections on triangle meshes. Reflections on cylinders are handled by a specific algorithm that involves the following steps:

- Determination of the reflection point within the normal 2D projection plane.

- Determination of the reflection point in Cartesian coordinates.

- Visibility (occlusion) check.

For the first step a coordinate transformation is used which maps the source location S and the listener location L into the normal 2D projection plane of the cylinder with the cylinder axis as the origin. The 2 orthogonal unit base vectors u0 and v0 of this 2D coordinate system can be chosen arbitrarily and the index “uv” will be used in the following to refer to this coordinate system. Figure 29 illustrates this projection for the source location Suv, the listener location Luv, and the reflection point Yuv.

Figure 29 — Cylinder reflection in the uv-plane

The reflection point Yuv = r * [cos(α), sin(α)]T with r being the cylinder radius is found by a Newton root search for the error function e(α):

| eα=Y0α Suv- YαSuv- Yα-Y0α Luv- YαLuv- Yα | (161) |
| --- | --- |
| Y0α=cosαsinα | (162) |
| Yα=r Y0α=uv | (163) |

After the reflection point Yuv was found within the normal projection plane, the reflection point Y in 3D Cartesian coordinates is determined as follows where PS and PL are the source and listener locations projected onto the axis of the cylinder:

| lS=Suv-Yuv | (164) |
| --- | --- |
| lL=Luv-Yuv | (165) |
| wS=lLlS+lL | (166) |
| wL=1-wS | (167) |
| PY=wS PS+wL PL | (168) |
| Y=PY+u u0+v v0 | (169) |

The found cylinder reflection is only visible, if the point Y is located on the cylinder surface and if the propagation path from the source to the reflection point and from the reflection point to the listener is not blocked. For this, the visibility check described in clause 6.6.6.3.4 is utilized.

##### RI EQ

The Early Reflection stage creates a RI for each image source and generates equalizer (EQ) parameter settings for each of them. These are linear scaling factors for the frequency bands which are handled by the EQ stage (see clause 6.6.20). The following properties are taken into account:

- Wall absorption EQ

- Transmission EQ (transmissive materials only)

- Shape EQ (cylinders only)

- Directivity EQ (taken into account by the Binaural Spatializer stage, see clause 6.7.1)

The wall absorption EQ accumulates the frequency dependent absorption coefficients of the materials on which the sound is reflected.

If the propagation path of the reflected sound is occluded by transmissive materials, then a transmission EQ accumulates the transmission coefficients of the transmissive materials.

For cylinder reflections a specific shape EQ is used which depends on the radius r of the cylinder. For a frequency band with center frequency f, the shape EQ is computed as follows:

| Hshapef=f2f2+43 Hz∙mr2 | (170) |
| --- | --- |

The directivity of the image source is accounted for in the Binaural Spatializer stage (see clause 6.7.1) in the same way as primary sources are handled.

##### RI gain

The Binaural stage (see clause 6.7.1) implements a distance-dependant attenuation for all RIs including the image sources of the Early Reflection stage. The total gain for the image sources is the product of the linear distance gain of the Binaural Spatializer stage and the following gain values:

- Tuning Gain gtuning = 0.4

- Floor Damping gfloor = 0.5 (15° threshold)

- Culling Gain gculling (linear fade-out starting 5 m before the source culling distance)

- Cylinder Gain gcylinder (apparent distance / optical magnification)

- Accessibility Gain gAccessibility

The tuning gain gtuning = 0.4 attenuates the sound level of image sources to reduce sound coloration from strong individual reflections. This results in a reflection which has ~8 dB less sound pressure level than a reflection in the real world. If hasEarlyTuningGain is set to true, the default value of 0.4 is overridden as follows (see Clause 6.2.6):

| atuning = earlyTuningGain - 43* 0.5 dB | (171) |  |
| --- | --- | --- |
| gtuning = 10^(atuning / 20dB) | (172) |

The floor damping gfloor is used to further reduce the sound level of floor reflections by 6 dB, if the sound is reflected on a surface which has a normal vector within 15° off the y-axis.

The culling gain gculling implements a linear fade-out of image sources which are close to the source distance culling value earlySourceCullingDistanceOrder1 for first-order reflections or earlySourceCullingDistanceOrder2 for second-order reflections with clip(x, range) specifying a limitation to the given range:

| gculling=clipearlySourceCullingDistanceOrder1-S-L5,[0,1] | (173) |
| --- | --- |
| gculling=clipearlySourceCullingDistanceOrder2-S-L5,[0,1] | (174) |

A linear fade-out of image sources which are reflected on triangles close to the triangle culling distances
earlyTriangleCullingDistanceOrder1 and earlyTriangleCullingDistanceOrder2 is not implemented in the reference model.

The cylinder gain gcylinder is only applied for cylinder reflections. The distance attenuation of cylinder reflections differs from the distance attenuation of reflections on planar surfaces due to the magnification effect of the curved surface. The cylinder gain gcylinder takes this into account by using the apparent distance distapparent of the image source:

| lS=Suv-Yuv | (175) |
| --- | --- |
| lL=Luv-Yuv | (176) |
| scaleplanar=lS+lLlL | (177) |
| μ=0.01 | (178) |
| α=angleSuv-Yuv,Luv-Yuv2 | (179) |
| ∆Y=r tanμ cosα/2, with μ=0.01 | (180) |
| ∆S=lS tan2μ+scaleplanar ∆Y, with μ=0.01 | (181) |
| distapparent=lL scaleplanar ∆S∆Y | (182) |
| gcylinder=S-Y+L-Ydistapparent | (183) |
| The Accessibility Gain gAccessibility shall be applied to all early reflections when the user parameter erAttenuationDb is provided via the Accessibility User Interface (see Annex B.5). It is computed as: |  |
| gAccessibility = 10(-0.05*erAttenuationDb) | (184) |
| For all early reflections derived from a primary item of type ListenerVoice, the associated listener voice early reflection gain gListenerVoice shall additionally be applied. It is computed as: |  |
| gListenerVoice = 10(0.05*erGainDb) | (185) |

##### Fast moving objects

Image sources are only generated for sound sources which move not faster than 5 m/s. A hysteresis technique in combination with a fade-in/fade-out is used to yield a smooth transition between those 2 speed operation modes (slow vs. fast moving sources).

Within the update thread the Early Reflection stage estimates the current speed v of each sound source by the location difference between 2 consecutive update calls:

| v=Sn-Sn-1T | (186) |
| --- | --- |

Where n denotes the frame index and T denotes the update period. The state is switched from slow to fast mode, if the speed of the sound source exceeds 5 m/s for 10 consecutive update frames. Vice versa, the state is switched from fast to slow mode, if the speed is less than or equal 5 m/s for 10 consecutive update frames.

A fade-out/fade-in with a linear ramp with increments/decrements of 0.1 per frame is performed on each mode switch. This multiplicative speeding fade-out/fade-in gain is applied in addition to the gains described in the previous paragraph.

##### Heavy/light updates

In order to reduce the workload within the update thread, the Early Reflection stage distinguishes between light and heavy updates. The Early Reflection stage uses the frame counter starting at 0 to distinguish between these 2 update modes. If the modulo of the frame counter plus the RI index of the slow active sources is 0, a heavy update is performed while only a light update is performed otherwise.

In heavy update mode, all visibility checks described in the previous paragraphs are performed and the RIs are created accordingly.

In light update mode, the visibility checks are skipped and the RI list from the last heavy update is used with updated image source positions.

### Airflow Simulation

#### General

This stage is used to simulate the sound that is perceived by a listener due to air passing over their ears. The sound that the listener hears is dependent on the speed of the airflow and the direction relative to the listeners orientation.

The perceived sound shall be simulated by filtering pink noise by a set of speed, azimuth, and elevation dependent gain profiles. The primary influence on the overall level of the perceived sound is due to the speed gain. Azimuth and elevation gains are always considered to be a reduction in the level of the perceived sound, and the three effects are multiplicative.

The frequency filtering is carried out in the stage using a 1/3rd Octave spaced set of biquad filters.

#### Data Elements and variables

airflowSource	A struct containing the parameters of an airflow source, as carried in the bitstream, see 6.2.12.

sourceID 	Unique ID of the source within the scene.

generatorID	Unique ID of the generator object within the scene state that is associated with the source.

defaultProfile	Map of key:value pairs where the key is an index and the value is a set of speedProfiles, aximuthProfiles, and elevationProfiles as given in A.15.

customProfileParameters	A vector of structs containing parameters to construct custom speedProfiles, azimuthProfiles and elevationProfiles. Specified as either frequency-gain pairs, or filters with centre frequencies, gains, and Q factors.

speedProfiles	Map of key-value pairs where the key is an airflow speed in metres per second, and the value is a 31-band frequency dependent gain profile.

azimuthProfiles	Map of key-value pairs where the key is an airflow azimuth in degrees, and the value is a pair of 31-band frequency dependent gain profiles for the left and right ear respectively.

elevationProfiles	Map of key-value pairs where the key is an airflow elevation in degrees, and the value is a 31-band frequency dependent gain profile.

itemStore	a pointer to an object which provides the functionalities for creating new RIs, updating existing RIs, and removing inactive RIs. This needs to be initialized before calling the update thread.

Peak-Q	A filter specified by the gain at a given centre frequency, and the Quality factor of the filter, which defines the bandwidth at -3 dB from the centre peak.

activeInDefaultRegion	Flag that indicates if a source is active within the default acoustic environment.

validRegions	List of acoustic environments for which a source should be considered active.

Euler rotation	Vector of rotations as [yaw, pitch, roll].

Polar rotation	Rotation represented as azimuth, elevation and distance.

elevationRange	Limited range of elevations for which a point generator is considered active.

azimuthRange	Limited range of azimuths for which a point generator is considered active.

#### Stage Description

##### Initialization

During the initialization of the airflow stage the data for all sources is read from the bitstream. For each source an airflowSource is constructed and stored. An airflowSource consists of a sourceID, a generatorID, and either a defaultProfile or a set of customProfileParameters.

For each airflowSource an Airflow renderItem shall be created and added to the itemStore. Two audio buffers are created per source, which are filled with pink noise. Additionally, 2 filterbanks consisting of 1/3rd octave spaced biquad bandpass filters shall be allocated per airflow source.

###### Construction of Profiles

Lookup tables are defined for defaultProfiles, specified in A.15, these specify the speed, azimuth, or elevation frequency-dependent gains for a number of predefined measurement points which shall be used when a custom profile is not specified. When custom profiles are specified in the bitstream these shall be converted from the bitstream specification into the required speedProfiles, azimuthProfiles, and elevationProfiles.

Custom profiles may be carried in the bitstream as either frequency-gain pairs, or as Peak-Q filter parameters. In the case of frequency-gain pairs the profiles are created by interpolating the gains from the specified frequencies into 31 1/3rd Octave frequencies. For Peak-Q filters the filter parameters shall be converted to a polynomial representation using the equations in (187)

| a=x1y3- y2+ x2y1- y3x1- x2x1- x3x2- x3 b=y2- y1x2- x1-a(x1+ x2) c= y1-ax12-bx1 where: cf=Centre frequncy of filter bw=0.5 cfq x1= cf- bw x2= cf x3= cf+ bw y1= gain-3, y2= gain y1= gain-3 | (187) |
| --- | --- |

For each filter the gain at the 1/3rd Octave frequencies, f, is calculating as

| gain=af2+bf+c | (188) |
| --- | --- |

The profile is calculated as the per-frequency maximum gain per filter.

speedProfiles are given as a single 31-element vector that is applied to both left and right outputs, and specified with a reference airflow in metres per second.

azimuthProifiles are given as a pair of 31-element vectors, where the first vector is applied to the left output and the second vector is applied to the right output. They are specified with a reference azimuth in degrees counter clockwise around the positive y axis.

elevationProfiles are given as a single 31-element vector that is applied to both the left and right outputs, and specified with a reference angle in degrees counter clockwise around the positive z axis.

###### Initialization of filters

Each airflow source requires two 1/3rd octave spaced filterbanks. These shall be constructed from a set of biquad filters, where the transfer function H(z) takes the form of (189):

| Hz= b0+ b1z-1+ b2z-2a0+ a1z-1+ a2z-2 | (189) |
| --- | --- |

The parameters a0, a1, a2, b0, b1 and b2 shall be calculated using the following pseudocode, and the centre frequencies listed in A.11.

filterParams(sampleRate_hz, centreFrequency_Hz) {

Fc = centreFrequency_hz / sampleRate_Hz;

Q = 4.3290;

K = tan(pi * Fc);

norm = 1 / ( 1 + K / Q + K * K);

a0 = K / Q * norm;

a1 = 0;

a2 = -a0;

b1 = 2 ( K * K -1) * norm;

b2 = (1 -K / Q + K * K) * norm;

}

##### Update thread processing

For each airflowSource it is checked if they are considered active in the Acoustic Environment (AE) that the listener is currently in. A source is considered active if the current listener AE is in the list of validRegions for that source, or in the case that the listener is in the default AE, that the sources activeInDefaultRegion flag is true.

For each active airflowSource the velocity vector of the airflow shall be calculated from the generator specified in the generatorID. The velocity vector comprises the speed of the airflow at the listener position, as well as the relative azimuth and elevation of the airflow with respect to the listeners pose as described in 6.6.7.3.2.1.

From the calculated velocity vector the speedProfile, azimuthProfile, and elevationProfile can be retrieved from the lookup tables. If the profile does not exist within the provided profiles, linear interpolation of the nearest two profiles is used to construct the desired profile. The per-frequency gain is taken to be the multiplication of the speedProfile, azimuthProfile, and elevationProfile for that frequency. Calculated gains for each source is stored in the dspData to be used in the audio thread processing.

###### Calculating velocity-vector per generator type

Four generator object types exist within the scenestate, as defined in 6.4.1.2.2.7 each have differing methods by which to calculate the listener relative velocity vector.

Global generator

Global generators shall be assumed to be uniform within their valid AE regions, intended to model sources such as atmospheric wind. As such they have no location within the scene, only an orientation. The orientation of the global generator is converted from the Euler rotations into Polar rotations. The polar rotation shall be rotated by 180° in the elevation axis, to convert from rotation relative to the generator object to rotation relative to the listener.

Point Generator

For point generators the distance from listener to the generator location shall be calculated and the speed of the airflow source determined by (190)

| perceivedSpeed=speed*1distance2*distanceAttenuationFactor | (190) |
| --- | --- |

distanceAttenuationFactor allows for scaling the rate of decay of the inverse square law.

Point generators contain an optional elevationRange and azimuthRange parameter, when these are set the associated source shall only considered active when the user is in the azimuth and elevation range specified, with respect to the generator.

Cone Generator

Figure 30 shows a section view of a cone airflow generator, positioned at point A, with length l and base radius br. For a listener at B the vector B-A shall be used to calculate the relative azimuth and elevation of A at B, and the length of B-A determines the distance used to calculate the perceived speed, with (190)

Figure 30 - Cone airflow generator

Figure 31 shows a cone generator where in addition to the base radius an edgeFade parameter, ef is considered. At position B’ the perpendicular distance pd from the centre line of the cone c is calculated. The distance of B’ along the section of pd between d and e is calculated as a fraction, and this shall be used to scale the perceived speed using (191)

Figure 31 - Cone airflow generator with edgeFade parameter

| perceivedSpeed=speed*1distance2*distanceAttenuationFactor*(1-distFadeRegion) | (191) |
| --- | --- |

Figure 32 illustrates a cone specified with a top radius tr. In this case the perceived source location, pA, is offset from A within the top radius by the percentage distance that B is across the perpendicular radius of the cone at B. The vector B-pA shall be used for calculating the perceived azimuth, elevation, and speed in place of B-A in the basic cone above.

Figure 32 - Cone airflow generator with top radius

Planar Generator

Planar generators are 2-dimensional surfaces that emit air along their normal axis. Figure 33 illustrates a planar generator at position A with height h and width w. For a listener at position B the perceived source position A’ shall be translated from the position A by the offset of B from the normal axis of the generator, w. The vector B-A’ shall be used for the calculation of perceived azimuth, elevation and distance, with perceived speed calculated using (190)

Figure 33 - Planar airflow generator

Figure 34 illustrates a planar airflow parameter with edge fade parameters. As the edge fade is a percentage of the dimensions, ef and ef’ are defined as the same value, but correspond to different dimensions according to w and h respectively. The perceived speed shall be scaled using (191), where the distFadeRegion shall be calculated as the percentage that A’ is between the boundary of the planar generator object and the boundary of the fade region.

Figure 34 - Planar airflow generator with edgeFade parameter.

##### Audio thread processing

For each active airflow source the left and right channels shall be processed independently, by:

  - Generation of pink noise.

  - Filtering of pink noise into 1/3rd octave bands with the biquad filterbanks.

  - Application of bandwise gains.

  - Summation of the 1/3rd octave bands into the output buffers.

Pink noise may be generated by any known method such that the power spectral density of the generated noise is inversely proportional to frequency.

The following pseudocode describes how to process the audio.

std::array<std::vector<BiquadFilter>> filters

// contains the Biqaud filters created at init.

audiobuffer outputBuffer; // buffer for 2 channel output audio

audiobuffer pinkNoiseBuffer; // buffer containing 2 channels of input pink noise.

std::array<std::vector> previousFilterGains; // Vector of filter gains used in the last process block

std::array<std::vector> currentFilterGains; // Vector of filter gains to be used in the current frame

int frameLength = 256; // Size of frame used by the DSP thread

for (int ear = 0; ear < 2; ++ear) {

// Clear the output buffer

outputBuffer.clear();

for (int filterIndex = 0; filterIndex < filters[0].size; ++filterIndex) {

for (int offset = 0; offset < frameLength[0].size(); ++offset) {

interpolatedGain = (1.0 – (offset / frameLength) * previousFilterGains[ear][filterIndex]) + ((offset / frameLength) * currentfilterGains[ear][filterIndex])

outputBuffer[ear][offset] += filter[ear][filterIndex].process(inputBuffer[ear][offset]) * interpolatedGain;

}

}

}

previousFilterGains = currentFilterGains;

Where the biquad process function uses the coefficients calculated in 6.6.7.3.1.2, and internal state variables z1 and z2:

Biquad::process(float in) {

out = in * a0 + z1;

z1 = in * a1 + x2 – b1 * out;

z2 = in * a2 – b2 * out;

return out;

}

If a global gain is applied to the airflow source, this shall be included by multiplication of the interpolated gain used.

### Discover SESS

#### General

This stage is a helper stage to render Spatially Extended Sound Sources (SESS). In a virtual 3D scene that contains only the meshes of all SESS (including portals), a predefined number of rays is cast into all directions. This is done in each update cycle, given that any relevant scene object or the listener position has changed. For each extended source/reverberation extent RI, the ray hits are stored. This information is then used in later stages like Occlusion and Homogeneous Extent.

#### Data elements and variables

alpha 	Variable containing the angle of a secondary ray’s endpoint in the polar coordinate system in the plane orthogonal to a primary ray

axis0, axis1	Unit vectors defining the local coordinate system in the plane orthogonal to a primary ray

R 		Radius of the disc around each primary ray, wherein the secondary rays are cast

ray		List of current secondary rays

rays	4096 element list of 3-component vectors corresponding to the directions of primary rays

rayId	Index of the current primary ray

#### Stage description

        - Update thread processing

The Update thread shall only process the scene if either any relevant scene object position has changed, or the position of the listener has changed significantly since the last time the update processing occurred, otherwise the previously calculated rays shall be considered as still valid and will be reused by the later stages. The threshold for listener position change shall be 0.001m translation in any dimension, or 1 degree rotation in any axis. The threshold for scene object position change shall be 0.001m in any dimension, or any amount of rotation.

4096 primary rays are cast in all directions, measured relative to the listener’s orientation. The list of ray directions is stored in a list in the source code. All ray hits that are caused by an intersection of a ray with a source extent geometry (including portal geometries) are stored. However, there is a distinction between a ray hitting the outside or the inside of an extent geometry. If one ray hits the same extent geometry multiple times, only the closest hit is considered.

For each primary ray, 6 additional rays are cast in a circular pattern. These secondary rays start at the same point as the primary ray, and pass through 6 points equidistributed on a circle of radius 23R on a plane, perpendicular to the primary ray’s direction at a distance 1 m from the listener (see Figure 35), where  R=tan⁡(0.0543).

if (rayId == 0)

axis0 = rays[rayId] - rays[rayId + 1];

else

axis0 = rays[rayId - 1] - rays[rayId];

axis0.normalize();

axis1 = rays[rayId].cross(axis0);	 // cross product between the current ray and axis0

for (int i=0; i<6; i++) {

alpha = 2*pi*i/6.0;

ray[i] = rays[rayId] + (2.0*R/3.0)*axis0*sin(alpha) + (2.0*R/3.0)*axis1*cos(alpha);

}

Figure 35 – Secondary ray distribution

The primary ray and all of the additional rays are given an equal weight of 1/7. For each ray that hits a source extent geometry, its weight is added to the total weight associated with its primary ray’s ID.

All rays with a non-zero weight are stored in the RI for later stages to consume.

In a second loop over all RIs, additional refined rays are cast for extent geometries that have been hit by fewer rays than RayHitSubdivCount = 20. For each of the primary rays that hit the geometry, 18 secondary rays are cast in circular patterns:  6 rays through points equidistributed on a circle of radius 25R that is located on a plane perpendicular to the primary ray’s direction at a distance of 1 m from the listener; 12 rays through points equidistributed  on a circle of radius  45R that is located on a plane perpendicular to the primary ray’s direction at a distance 1 m from the listener (see Figure 36).

for (int i=0; i<6; i++) {

alpha = 2*pi*i/6.0;

ray[i] = rays[rayId] + (2.0*R/5.0)*axis0*sin(alpha) + (2.0*R/5.0)*axis1*cos(alpha);

}

for (int i=0; i<12; i++) {

alpha = 2*pi*i/12.0;

ray[i] = rays[rayId] + (4.0*R/5.0)*axis0*sin(alpha) + (4.0*R/5.0)*axis1*cos(alpha);

}

Figure 36 – Refined secondary ray distribution

The primary ray and all of the secondary rays are given an equal weight of 1/19. For each ray that hits a source extent geometry, its weight is added to the total weight associated with its primary ray’s ID. In the record associated to the primary ray’s ID, for each of the 19 rays a bit is set to 1 if the corresponding ray hits the geometry and to 0 otherwise.

        - Audio thread processing

This stage performs no processing on the audio thread.

### Occlusion

#### General

The Occlusion stage provides occlusion information with respect to a direct path - a line of sight - from a source to the listener. When this straight line is occluded by acoustically opaque/partially-transparent objects, the geometry/mesh information encountered along the line of sight is updated in a dedicated data structure. Then, the status flags of the RI, e.g. relevant for the control of the fade-in/out process and its EQ are updated correspondingly.

For extended sources, a number of ray bundles are created, one for each combination of occluding materials, encountered by a ray-cast from a listener to the geometry, representing the extent. Each bundle is provided with an EQ curve, based on the transmission properties of the occluding materials in the corresponding list. This data is used in the Homogeneous Extent stage to generate the final binaural signal, combining the occluded and the un-occluded parts of the extent. For example, in Figure 37 there are three bundles for the SESS on the right: bundles 1 and 3 correspond to two different occluders so each has its own EQ curve corresponding to the transmission properties of the respective occluders. Bundle 2 is unoccluded, and its contribution has no additional processing.

Figure 37 — SESS ray bundles

#### Data elements and variables

angle	variable holding the polar angle of and additional ray endpoint

distance	distance between the listener and the current RI

EQNBands	CV storing the number of parametric EQ bands in itemEQ array; default value 31

frameCounter	frame counter

itemEQ	array of EQ gains stored in the current RI

itemPosition	current RI position

itemRayHits	list of records, storing the information about the rays cast in Discover SESS stage (6.6.8) for each SESS

listenerPosition	current listener position

materialTransmission	two dimensional array of transmission coefficients for the current material list

bbPERot	Rotated bounding box object created from the portal extent geometry rotated such that the portalNormal is aligned with the negative z axis, indicating minimum and maximum coordinate values in each dimension, and resulting box rotated back with the inverse rotated.

plane	The side of a box-equivalent geometry closest to the acoustic environment in which the listener is located. The box-equivalent geometry shall be an axis-aligned bounding-box defined by the smallest and largest coordinates of the portal extent geometry in each axis, after this has been rotated such that the portal normal points along the negative z-axis.

ignoreRay	Boolean indicating whether a ray shall be discarded, and not be used for further processing of the reverberation extent source.

nMaterials	total count of materials in the current material list

nofHits	count of unoccluded ray hits

objectName	name of the object, corresponding to the current RI

OcclusionFastSpeed	CV storing the threshold speed value for separate fast-moving object processing

r		vector from the listener to the center of the current RI geometry

r_orth1, r_orth2	unit vectors orthogonal to r, used to calculated the endpoints of the additional rays

speed	current object’s speed estimate

previousPosition	RI position used in the previous speed calculation

previousTime	Scene Time used in the previous speed calculation

#### Stage description

##### Update thread processing

###### Point source processing

Cast the ray corresponding to r against the scene geometry. For each of the intersected geometries, collect the material properties at the point of intersection. Adjust the RI EQ curve by the transmission coefficients, corresponding to each of the materials in the list. If the ray is unoccluded, reset the RI EQ curve to constant 1.0 for each frequency band.

for (int i = 0; i<EQNBands; i++)

itemEQ[i] = 1.0f;

if (nMaterials != 0)  {

for (int m = 0; m<nMaterials; m++) {

for (int i = 0; i<EQNBands; i++)

itemEQ[i] *= materialTransmission[m][i];

}

}

###### Extended source processing

If itemRayHits list is empty, apply the same processing that is used for point sources.

For each ray hit in itemRayHits, cast secondary rays in two concentric circles around the main ray (1 ray in the centre constituting the main ray, 6 rays on the inner circle, and 12 on the outer); the exact parameters of the ray distribution are given in the Discover SESS stage (6.6.8) description. The indices of the secondary rays, hitting the same SESS as the primary ray, are stored by the Discover SESS stage in a binary mask in primary ray’s ray hit data structure. Skip the secondary rays not present in the binary mask since they do not hit the extent of the current RI.

With the portal representation in explicit portal mode (isExplicitPortalMode = true), for sources with extent of type 'DirectionalReverb', each ray shall be tested whether it goes through the portal identified by its parentPortalId field. If it does not go through the portal the ray shall be ignored and not stored in the binary mask.

This shall be tested by defining a portal plane and testing whether the ray intersects with the portal plane. This shall be tested according to the procedure outlined in the following pseudo-code.

u = plane.lowerRightCorner – plane.lowerLeftCorner;

v = plane.upperLeftCorner – plane.lowerLeftCorner;

uDotLL = dot(u, lowerLeftCorner);

uDotLR = dot(u, lowerRightCorner);

vDotLL = dot(v, lowerLeftCorner);

vDotUL = dot(v, upperLeftCorner);

d = dot(plane.center, -1 * plane.normal);

denom = dot(rayVector, planeNormal);

if (abs(denom) < 1E-7f) {

ignoreRay = true;

} else {

t = -(d + dot(rayOrigin, planeNormal) / denom;

intersect = rayOrigin + t * rayVector;

insideX = dot(u, intersect) >= uDotLL && dot(u, intersect) <= uDotLR;

insideY = dot(v, intersect) >= vDotLL && dot(v, intersect) <= vDotUL;

ignoreRay = (t <= 0.0f || !insideX || !insideY);

}

where dot(a, b) shall be the dot product between vectors a and b, abs(x) the absolute value of a scalar, rayVector the absolute direction of the ray (listener orientation offset with listener-relative ray direction)  and rayOrigin the listener position.

The portal plane shall be the side of a box-equivalent geometry closest to the acoustic environment in which the listener is located, as illustrated in Figure 38. The box-equivalent geometry shall be an axis-aligned bounding-box defined by the smallest and largest coordinates of the portal extent geometry in each axis, after this has been rotated such that the portal normal points along the negative z-axis.

The plane corner coordinates shall correspond to the following coordinates of the bounding box (bbPERot), but after being rotated back with the inverse rotation. Similarly, the plane centre coordinates shall be derived from the bounding box.

For portalEnv1:

plane.lowerLeftCorner  = [bbPERot.xMin, bbPERot.yMin, bbPERot.zMin] – bbPERot.center;

plane.lowerRightCorner = [bbPERot.xMax, bbPERot.yMin, bbPERot.zMin] – bbPERot.center;

plane.upperLeftCorner  = [bbPERot.xMin, bbPERot.yMax, bbPERot.zMin] – bbPERot.center;

plane.upperRightCorner = [bbPERot.xMax, bbPERot.yMax, bbPERot.zMin] – bbPERot.center;

plane.center = [(bbPERot.xMax + bbPERot.xMin)/2, (bbPERot.yMax + bbPERot.yMin)/2,

bbPERot.zMin] – bbPERot.center;

For portalEnv2:

plane.lowerLeftCorner  = [bbPERot.xMin, bbPERot.yMin, bbPERot.zMax] – bbPERot.center;

plane.lowerRightCorner = [bbPERot.xMax, bbPERot.yMin, bbPERot.zMax] – bbPERot.center;

plane.upperLeftCorner  = [bbPERot.xMin, bbPERot.yMax, bbPERot.zMax] – bbPERot.center;

plane.upperRightCorner = [bbPERot.xMax, bbPERot.yMax, bbPERot.zMax] – bbPERot.center;

plane.center = [(bbPERot.xMax + bbPERot.xMin)/2, (bbPERot.yMax + bbPERot.yMin)/2,

bbPERot.zMax] – bbPERot.center;

For both planes, the plane.normal shall be equal to the corresponding portalNormal.

Figure 38 – Illustration of portal planes (dashed lines) in a cross-section of a portal extent (grey) and walls (black).

Collect the remaining secondary rays into bundles, based on the material lists of the geometries they hit. For each bundle, generate a secondary RI with the EQ curve corresponding to the transmission coefficients of the materials on the corresponding list.

if (nMaterials != 0)  {

for (int m = 0; m<nMaterials; m++) {

for (int i = 0; i<EQNBands; i++)

itemEQ[i] *= materialTransmission[m][i];

}

}

In each created secondary RI create a ray hit record and  store the ratio of the number of rays in the bundle to the total number of secondary rays (19) in the weight field of the ray hit record.

###### Fast-moving object processing

This step provides smoother transitions between the occluded and unoccluded states for fast-moving audio sources relative to the listener.

Every fifth frame the speed estimate for each of the active objects in the scene is calculated, storing the item position and scene time to be used in the next speed calculation.

if (frameCounter % 5 == 0) {

//length() returns the magnitude of a vector

distance = length(itemPosition – previousPosition)

timeDifference = currentTime – previousTime;

speed = distance / timeDifference;

previousPosition = itemPosition;

previousTime = currentTime;

}

If the current speed estimate is above the threshold given by CV OcclusionFastSpeed = 2 [m/s], cast 20 rays from the listener towards the source with endpoints equidistributed on a circle, whose radius is proportional to the speed.

if (speed > OcclusionFastSpeed) {

r = itemPosition – listenerPosition;

r_orth1 = r;

// Make r_orth1 not parallel to r

if (r[2] != 0) {

r_orth1[2] = -1 * (r[0] + r[1]) / r[2];

} else if (r[1] != 0) {

r_orth1[1] = -1 * (r[0] + r[2]) / r[1];

} else {

r_orth1[0] = -1 * (r[1] + r[2]) / r[0];

}

//a.cross(b) returns the cross product of a vector a with vector b

//Make r_orth1 orthogonal to r

r_orth1 = r.cross(r_orth1).normalize();

//r_orth2 is orthogonal to both r and r_orth1

r_orth2 = r.cross(r_orth1).normalize();

radius = speed/50.f;

for (int j = 0; j<20; j++) {

angleStep = 2 / 20

angle = j * angleStep;

tracingTarget[j] = r_orth1 * radius * cos(angle*pi) +

r_orth2 * radius * sin(angle*pi);

tracingTarget[j] = sample[j] + itemPosition;

}

Where cos(angle) and sin(angle) represent respectively the sine and cosine of an angle in radians.

Cast each of the rays described by tracingTarget[j] against the scene geometry and store the number of unoccluded rays (the rays which do not intersect any geometry) in nofHits. Adjust the EQ of the RI by the ratio of unoccluded rays to the total ray count (20).

for (int i = 0; i<EQNBands; i++)

itemEQ[i] *= nofHits/20;

The fast moving RIs (the ones, whose speed exceeds the above threshold) are excluded from the processing described in 6.6.9.3.1.1 and 6.6.9.3.1.2. For this reason if the nofHits is smaller than the rays traced the RI is marked as occluded using its occlusionInfo field.

###### Final processing steps

Remove all the cached occlusion-specific data corresponding to the removed RIs and increment the frameCounter.

##### Audio thread processing

This stage performs no processing on the audio thread.

### Diffraction

#### General

The Diffraction stage provides the information required for generating diffracted sounds from hidden sources to a listener around occluding objects. This stage uses pre-processed geometrical data from the bitstream including edges, paths, and voxel data. These data are used to efficiently identify relevant diffraction paths from a given source to the listener location during rendering, and these diffraction paths are used to create relevant additional RIs for diffraction. For static sources, it is possible to use pre-calculated paths stored in corresponding voxel data to achieve fast computation at runtime. For dynamic sources, precomputed edges visible at the locations of a source and the listener need to be found using ray tracing techniques and these edges are used to fetch and evaluate relevant paths before creating RIs. This stage also uses the line-of-sight occlusion information that is provided by the Occlusion Stage to decide whether the diffraction path finding process is activated based on the absence of a visible line-of-sight.

#### Data elements and variables

sources	a map of Source-type objects where each Source object is instantiated from the primary RI in renderList and the corresponding key is the corresponding item’s unique ID. Source objects contain the following list of variables: their global locations in the previous and current time frames, speed, current orientation, unique source ID, a flag of relocation status from the previous time frame,  source status to check if it is active or inactive, source type, visible edge list and path index list, a flag to indicate the occlusion status in the previous time frame (isPreviouslyOccluded) and a flag to indicate whether the occlusion status has been changed since the previous time frame (isOcclusionStateChanged). Each source object information is updated at every frame.

listener	a unique pointer to the Listener object which contains its location, orientation, relevant visible edge list, a flag to indicate whether the listener location has been changed since the last time frame and this Listener-type object is updated at every update cycle.

meshes	a vector containing the meshes in a scene, which is used to instantiate the Embree tracer for visibility checks with all non-transparent static and dynamic meshes.

dynMeshes	a vector containing non-stationary/dynamic meshes in a scene, which is used to instantiate the Embree tracer for visibility checks with all non-transparent dynamic meshes.

tracer	a unique pointer to the instantiated Embree tracer with meshes which needs to be set properly before calling the update thread.

dynTracer	a unique pointer to the instantiated Embree tracer with dynMeshes which needs to be set properly before calling the update thread.

diffrPayload	a shared pointer to the diffraction payload object which contains pre-processed bitstream data including static edges under the name of staticEdgeList, dynamic edges as dynamicEdgeDict, paths around static meshes as staticPathDict, paths around dynamic meshes as dynamicPathDict, and source-visible edges as sourceEdgeDict, listener-visible edges as listenerEdgeDict, and valid paths from a static source to a given listener location as validPathDict. It needs to be set properly before calling the update thread, but after the Diffraction payload object is created from the bitstream.

rtDiffractionPathTrackers	a map of RTDiffractionPathTracker-type objects with RI ID as a key and RTDiffractionPathTracker-type object as a corresponding value which provides interfaces and functionalities to update the valid path list from a given source location to the listener location and generate diffraction items with location, orientation, and filter information for diffraction-type RI creation.

edgeListAccess	mutex object to offer exclusive ownership regarding edge list access/update when geometries are relocated.

itemStore	a pointer to an object which provides the functionalities for creating new RIs, updating existing RIs, and removing inactive RIs. This needs to be initialized before calling the update thread.

secondaryItems	a map of RIs created by the Diffraction stage (update thread) which contains validated RIs at every time frame. This map has a key value as RI index and its corresponding value is a map of RIs with a unique hash.

isDynamicGeometryIncluded	a Boolean flag which indicates whether a given scene has dynamic geometries. This flag is set as false if dynMeshes is empty.

isDynamicGeometryRelocated	a Boolean flag which indicates whether a dynamic geometry is relocated from a previous time frame. If this flag is true, then dynamic edges are updated. By default, this flag is set as true to check possible paths around existing dynamic meshes initially.

geometryObservers	a vector of Observer-type objects to each of which a dynamic geometry pointer is allocated. An Observer-type object supports the interfaces and functionalities needed to monitor the attached geometry’s status change. This includes location and orientation such that meshes of the geometry are promptly updated when they have changed. This vector needs to be updated before calling update thread.

diffrItemInitialEQs	a vector of float values to store initial EQs for diffraction RI which is set with 1.0 entities assuming there is no attenuation for diffraction-type RIs initially.

cullingDist	culling distance threshold that is used to skip unnecessary visibility tests between source/listener to an edge and is initialized as 64 m by default.

B		block size in samples to update source speed

Fs		sampling frequency in samples to update source speed.

updateCounter	counter that increases by 1 in every update cycle and is initialized as 0.

updateRate 	update rate for slow update mode which is set as 10 by default.

gainHistoryMap	a map that contains source names as a key and a vector of historical source gains as its value.

intersectionTestSamples a map of vertex points for intersection tests. This is used for visibility tests to identify the volume/portion of visible source extent from the listener position.

#### Voxel data

The voxel database diffrListenerVoxelDatabase[l][j] for the listener voxel dictionary is derived from the variables specified in Section 6.3.2.3 by the following algorithm:

int delta_x = voxelCoordinateToVoxelIndex({ 1, 0, 0 });

int delta_y = voxelCoordinateToVoxelIndex({ 0, 1, 0 });

int delta_z = voxelCoordinateToVoxelIndex({ 0, 0, 1 });

int delta_list[4] = { 0, -delta_x, -delta_y, -delta_z };

for (int i = 0; i < numberOfListenerVoxels; i++) {

EdgeList edge_list;

VoxelCoordinate voxelCoord = { listenerVoxelGridIndexX[i],

listenerVoxelGridIndexY[i],

listenerVoxelGridIndexZ[i] };

int l = voxelCoordinateToVoxelIndex(voxelCoord);

int mode = diffrListenerVoxelMode[i];

if (mode != 0) {

int l_ref = l + delta_list[mode];

edge_list = diffrListenerVoxelDatabase[l_ref];

// generate list with removed items in reverse order

int numberOfIndicesRemoved =

length(diffrListenerVoxelIndexDiff[i]) - 1;

int listIndicesRemoved[numberOfIndicesRemoved];

int val = -1;

for (int k = 0; k < numberOfIndicesRemoved; k++) {

val += diffrListenerVoxelIndexDiff[i][k];

listIndicesRemoved[numberOfIndicesRemoved - 1 - k] = val;

}

// remove edge sequences

for (int k = 0; k < numberOfIndicesRemoved; k++) {

edge_list.erase(listIndicesRemoved[k]);

}

}

// add edge sequences

for (int j = 0; j < numberOfEdgesAdded[j]; j++) {

edge_list.append(diffrListenerVoxelEdge[i][j]);

}

// add sorted edge list to listener voxel dictionary

edge_list = sort(edge_list);

int num_edges = length(edge_list);

for (int j = 0; j < num_edjes; j++) {

diffrListenerVoxelDatabase[l][j] = edge_list[j];

}

}

The voxel database diffrValidPathVoxelDatabase[i][l][k] for the valid path dictionary is derived from the variables specified in Section 6.3.2.3 by the following algorithm:

int delta_x = voxelCoordinateToVoxelIndex({ 1, 0, 0 });

int delta_y = voxelCoordinateToVoxelIndex({ 0, 1, 0 });

int delta_z = voxelCoordinateToVoxelIndex({ 0, 0, 1 });

int delta_list[4] = { 0, -delta_x, -delta_y, -delta_z };

for (int i = 0; i < numberOfValidStaticSources; i++) {

for (int j = 0; j < numberOfMaximumListenerVoxels[i]; j++) {

ValidPathList validPathList;

VoxelCoordinate voxelCoord = { validListenerVoxelGridIndexX[i][j],

validListenerVoxelGridIndexY[i][j],

validListenerVoxelGridIndexZ[i][j] };

int l = voxelCoordinateToVoxelIndex(voxelCoord);

int mode = diffrValidPathMode[i][j];

if (mode != 0) {

int l_ref = l + delta_list[mode];

validPathList = diffrValidPathVoxelDatabase[i][l_ref];

// generate list with removed items in reverse order

int numberOfIndicesRemoved =

length(diffrValidPathIndexDiff[i][j]) - 1;

int listIndicesRemoved[numberOfIndicesRemoved];

int val = -1;

for (int k = 0; k < numberOfIndicesRemoved; k++) {

val += diffrValidPathIndexDiff[i][j][k];

listIndicesRemoved[numberOfIndicesRemoved - 1 - k] = val;

}

// remove path sequences

for (int k = 0; k < numberOfIndicesRemoved; k++) {

validPathList.erase(listIndicesRemoved[k]);

}

}

// add valid path sequences

for (int k = 0; k < numberOfPathsAdded[i][j]; k++) {

ValidPathPair validPath = {diffrValidPathEdge[i][j][k],

diffrValidPathPath[i][j][k]};

validPathList.append(validPath);

}

// add sorted path list to valid path dictionary

validPathList = sort(validPathList);

int num_edges = length(validPathList);

for (int k = 0; k < num_edjes; k++) {

diffrValidPathVoxelDatabase[i][l][k] = validPathList[k];

}

}

}

#### Stage description

The Diffraction stage is handled by two different threads for RI update and audio processing. Compared to other stages such as Homogeneous stage, Diffraction stage is only called within the update thread to generate secondary diffraction-type RIs. After the diffraction-relevant variables described in 6.6.10.2 are properly initialized, the Diffraction stage’s RI update function is called at a given update cycle.

##### Update thread processing

The general processing in the Diffraction stage is visualized as a flow diagram in Figure 39.

Figure 39 — Update process for generating diffraction-type RIs from active primary RIs based on source, listener, and precomputed path information

The update function for creating diffraction-type RIs is called with the renderList as an input which contains RIs and listener-relevant information such as location and orientation. At every update call, the listener is updated from the listener structure in renderList, and it checks every RI in it.

###### Exceptional cases

The elements of renderList contain primary RIs and secondary RIs. Secondary RIs are derived from primary RIs along the pipeline. These can be, for instance, reflection-type, diffraction-type or DirectionalReverb-type. The Diffraction stage supports the rendering of diffracted sounds from primary RIs which are either point sources or extended sources.

The update process will be skipped for the following cases: Firstly, if a given RI’s type is not primary, the path-finding and creation of new diffraction-type RIs will be skipped in the loop over RIs and the next RI will be checked. Secondly, if a given RI within the loop is defined in listener coordinate system (LCS), this RI will be skipped assuming that a LCS-based RI is not affected by geometries. Thirdly, the update process can be skipped according to the RI’s current status. A given RI’s status which indicates whether the RI is active or inactive can be changed at runtime. If a given RI’s status is inactive, previously created diffraction-type RIs are removed in order not to render diffracted sounds from inactive primary RIs.

###### Initialization and update of basic data elements

If a given RI is primary and active, Source-type object is instantiated from a given RI. The Source-type  object is stored in sources, i.e. a vector of Source objects. If the object already exists in the sources vector, it is updated. As mentioned in 6.6.10.2, Source-type objects contain positional information, occlusion flags, and path-relevant variables. Its positional information is updated directly from a given RI and the other source variables are updated later in the update thread.

A RTDiffractionPathTracker object is instantiated with the initialized along with the Source object, using listener, eifPayLoad and diffrPayload information to keep track of paths with the updated source and listener information.

###### Occlusion status check

The path finding and diffraction-type RI creation processes are only called if the line-of-sight from the listener to a given primary RI (i.e., a source) is not visible. In this regard, the occlusionInfo structure needs to be checked. The occlusionInfo structure contains updated occlusion-related information such as occluded surfaces’ materials and corresponding material EQs along the line-of-sight.

For instance, if occlusionInfo is empty, meaning there are no surfaces blocking the line-of-sight, the update stage needs to remove the diffraction-type RIs stored in itemStore. This is done to avoid rendering invalid diffraction-type RIs in the current time frame. The source’s occlusion flag isPreviouslyOccluded shall be set as false for the next update cycle.

If occlusionInfo is not empty and isPreviouslyOccluded is false,  isOccludedStateChanged shall be updated to true. Otherwise, isOccludedStateChanged shall be set to false. These flags are implemented to call the following processes in 6.6.10.4.1.5 – 6.6.10.4.1.7.

###### Diffraction item EQ initialization with occlusion information

If a given primary RI is occluded and its type is a point source, diffrItemInitialEQs is updated using the given RI’s EQ. The diffrItemInitialEQs is updated considering the occluded surface material’s EQ in the previous Occlusion stage. This information is used to render the sound through acoustically transparent or non-transparent surfaces. The diffrItemInitialEQs element is updated as follows:

// Given: diffrItemInitialEQs, RI.EQs

// RI.EQs contains RI’s EQ coefficients updated in the Occlusion Stage

for (int i = 0; i < diffrItemInitialEQs.size(); i++) {

diffrItemInitialEQs[i] = 1.0 – RI.EQs[i];

}

In case a RI corresponds to an extended source, it is required to additionally adjust diffraction-type RI’s gain according to the extended source’s partial visibility. The additional gain is calculated by checking which part of the source extent is visible from the listener. The ratio of the invisible surface of a given extended source to its original surface is used to adjust diffraction-type RI’s gain.

In addition to the gain adjustment, two operations need to be performed to avoid unrealistic sound level changes behind occluding geometries of extended sources. These operations are carried out assuming that the extended source consists of spatially equally distributed point sources. The first operation consists in using ray tracing to count hit rays which are cast out from the listener position to test sample points stored in intersectionTestSamples per a given source extent. For instance, if there are N number of ray-hits out of M samples stored in intersectionTestSamples[extentID] where extentID is the ID of a given RI’s source extent, the diffrItemInitialEQs‘s entities are multiplied by 1-N/M. The second operation consists in calculating the centre of the occluded part of a source extent using the occluded test samples to update the source‘s location. For this,  intersectionTestSamples needs to be initialized as a pair of 1) source extent ID as a key, and 2) sample positions as a value. The sample positions are updated with intersections between a given source extent and rays which are cast out from the source’s center position to a uniformly distributed directions using azimuth and elevation angles.

###### Path finding and validation process

###### General

After diffrItemInitialEQs and source location have been updated, relevant diffraction path information is updated depending on the locations of RIs and the listener. This process validates the precomputed path list transmitted to the renderer using the bitstream.

The path finding and validation process is computationally expensive, so it is only called at a slower rate than other processes in the stage. The path finding and validation process is also triggered immediately if the occlusion state of the source changed.

The process makes a distinction between static scenes and dynamic scenes due to complexity reasons. Static scenes do not contain moving geometries, so the path finding and validation process is only based on the precomputed diffraction paths in the bitstream, the position of the source and the position of listener. In contrast, dynamic scenes contain moving geometries, and additional diffraction paths need to be calculated at rendering time that depend on the current position of the moving geometry, requiring a higher computational load.

###### Static scenes

For a static scene with static sources and geometries, potential path information can be precomputed and transmitted to the renderer. The stored path data is available as a path index list in the voxel data alongside with information about RIs and listener.

Static scene path fetching with static sources

The potential diffraction paths are fetched from the precomputed list present in the bitstream’s diffraction payload by considering the positions of the source and the listener. After fetching, each potential path needs to be validated in order to be rendered into a diffraction audio signal.

To fetch the proper voxel data corresponding to a given primary RI (e.g., the source) and listener, the listener location needs to be converted into voxel indices which is a list of integers indicating X, Y, and Z indices in the pre-processed voxel grid. This voxel index is the key to fetch the stored path index list in the voxel data corresponding to a given RI’s ID at a given listener location. The voxel index is generated from a given position as below.

// Given: diffrVoxelOriginalX, diffrVoxelOriginalY, diffrVoxelOriginalZ, diffrVoxelPitchX,

// 		   diffrVoxelPitchY, diffrVoxelPitchZ

VoxelIndex getVoxelIndexFromPosition(Position position) {

int x = int((position.Xcoord – diffrVoxelOrignalX)/diffrVoxelPitchX + 0.5);

int y = int((position.Ycoord – diffrVoxelOrignalY)/diffrVoxelPitchY + 0.5);

int z = int((position.Zcoord – diffrVoxelOrignalZ)/diffrVoxelPitchZ + 0.5);

return VoxelIndex(x, y, z);

}

And the potential path list for a static RI at a given listener location is fetched as below.

// Given: sourceID, listenerVoxelIndex, diffrPayload

potentialPathIndexList = diffrPayload.validPathDict[sourceID][listenerVoxelIndex];

Additionally, the neighbouring voxels around the listener position are also checked for potential diffraction paths. If a diffraction path is present in one or more of the neighbouring voxels, and its end point is visible by the listener (i.e., not occluded), this path is also fetched as a potential path.

Then, Source’s potential path index list is updated from the fetched voxel data follows:

// Given: source, potentialPathIndexList

source.pathIndexList.clear();

for (int i = 0; i < potentialPathIndexList.size(); i++) {

source.pathIndexList.push_back(potentialPathIndexList[i]);

}

Static scene path fetching with moving sources

In the case of moving sources, the path fetching procedure is somewhat different and more computationally expensive, if performed at a fast rate, due to changing source positions. In order to proceed with path fetching, the occlusion status of the paths between source and listener needs to be checked first.  This procedure is called each time the source changes position or the source is occluded.

For testing the occlusion status, visibility tests are needed. Visibility tests are used to determine whether the sound source is visible or obstructed from a given receiver location, and vice versa (i.e., they are carried out each time from the source point of view, and from the listener point of view). They involve analyzing the direct line-of-sight between the source and receiver, as well as considering any obstacles or obstructions that may exist in the acoustic environment. The visibility tests return a list of visible geometry edges (addressed by unique identifiers), the visibleEdgeIdList, from the point of view of the source (source.visibleEdgeIdList) and the listener (listener.visibleEdgeIdList).

In order to reduce computational complexity, a first coarser visibility test is carried out using voxel data, which is computationally less expensive. If no visible edges were found, a second, finer, more expensive search for visible edges is performed.

After the visibility tests are performed, the list of potential paths is retrieved from the diffraction payload using the visible edges from source.visibleEdgeIdList and listener.visibleEdgeIdList information.

For fast sources (i.e., sources moving faster than a pre-determined speed), the visibility test (i.e., the visible edge search procedure) is only performed for edges closer to the source than a pre-defined distance threshold, the so-called diffraction distance culling threshold.

Potential path index list structure

Each path index in this list includes two integers for identifying a unique path: the first integer indicates an initial geometry edge ID. The initial geometry edge refers to the first geometry edge encountered by a diffraction path stemming from the source (see path validation).  The  second integer indicates the index within the stored path list which contains all potential paths starting from the mentioned initial edge ID.

The potential path list will be subject to path validation. The path validation will determine which of the diffraction rays will be actually rendered to an audible event. As the validation process uses geometrical calculations (ray tracing) to determine if a diffraction path in the list actually reaches the listener, information about the edges (edgeIDs) and faces (faceIndicators) of the involved geometries, as well as diffraction ray angles around them, is needed.

All the paths are in a form of Path-type object which contain precomputed path information together with a sequence of included edgeIDs, faceIndicators and angles. This process is briefly shown below.

Static scene path validation

Potential paths are validated by using two angle calculations: one is based on the source angle. This is defined as the angle at source location with respect to the first face, where the first face is determined by the first entity of included faceIndicators. The other angle is the listener angle. This is defined as the angle at listener location with respect to the last face which is determined by the last faceIndicator. The faceIndicator can be either 0 or 1, for example, to indicate which adjacentTriangle is included per path. The pseudo code for getting face or triangle information is given below.

// Given: path, diffrPayload

// path includes its edgeIDs, faceIndicators, angles, and its path order.

// Get the first triangle information within a given path

firstEdge = diffrPayload.staticEdges[path.edgeIDs[0]];

firstTriangle = firstEdge.adjacentTriangles[path.faceIndicators[0]];

// Get the last triangle information within a given path

lastEdge = diffrPayload.staticEdges[path.edgeIDs[path.order - 1]];

lastTriangle = firstEdge.adjacentTriangles[path.faceIndicators[path.order - 1]];

Figure 40 illustrates the validation process as a simple example. Figure 40 – (A) shows the scene configuration with a static source, a listener, and a rectangular geometry in 2D for simplicity. Figure 40 – (B) shows the possible two diffraction paths around the geometry: Path - i and j. Figure 40 – (C) and (D) show each path with illustrated source and listener angles. Path - i and j, can be both uniquely defined by three sequences of included edgeIDs, face IDs (or faceIndicators), and angles. For instance, Path - i can be described with the following sequences:

- Path – i

  - EdgeIDSequence: Edge A – Edge B

  - FaceIDSequence: Surface 4 – Surface 1

  - AngleSequence [in deg.]: 90 – 180

Figure 40 — Example to show the angles at source and listener per a given path

Note: In cases in which the geometry’s triangle information is not included in the bitstream, an alternative data structure FaceIDSequence can be replaced with faceIndicator sequence. The values of faceIndicator can be either 0 or 1 to indicate which adjacentTriangle of a corresponding edge is included.

In this example, Path - i represents one of the potential paths for the given configuration. The sourceAngle is the angle of the vector from the first edge (Edge A) to the source, relative to Surface 4 (Figure 40 – (C)). The listenerAngle is the angle of the vector from the last edge (Edge B) to the listener, relative to Surface 1.

The validation process for Path - i is carried out as follows: If the source angle is smaller than the first element of the AngleSequence in Path - i, and the listener angle is larger than the last element of the AngleSequence in Path - i, then Path - i is activated for the creation of diffraction-type RI signals.

Once the path is validated, an associated diffraction type RI will be created (audible event), For this purpose, information about the involved diffraction angles is needed. The process for updating DiffractionAngleSequence is provided below.

// Given: path, sourceAngle, listenerAngle

// path is a Path-type object which contains (included) edgeIDSequence,

// faceIndicatorSequence, angleSequence, nEdges and DiffractionAngleSequence to update

// diffraction-type RI’s EQ.

// sourceAngle and listenerAngle are calculated per each given path

angle1 = path.angleSequence[0];

angle2 = path.angleSequence[path.nEdges – 1];

path.DiffractionAngleSequence.clear();

if (angle1 > sourceAngle and angle2 < listenerAngle) {

if (path.nEdges == 1) { // First-order Diffraction (FOD) case

// A single angle for FOD is computed from listenerAngle and sourceAngle directly.

path.DiffractionAngleSequence.push_back(listenerAngle – sourceAngle – 180);

} else {

path.DiffractionAngleSequence.push_back(angle1 - sourceAngle);

for (int i = 0; I < path.nEdges – 2; i++) {

path.DiffractionAngleSequence.push_back(path.angleSequence[i]);

}

path.DiffractionAngleSequence.push_back(listenerAngle – angle2);

}

}

DiffractionAngleSequence consists of floating values each of which represents a diffracted angle per each edge considering a given configuration of source, listener, and geometries. For instance, as illustrated in Figure 40 – (C), Path – j starts from the source to the listener along the two included edges (Edge A and B) and for this path, two diffraction angles are required and are included in DiffractionAngleSequence.

Static scene diffraction-type RI creation

After the paths are validated, they will be activated. An active state of a diffraction path means that an audible diffraction-type RI will be created based on said path. In principle, all validated paths are activated with a few exceptions discussed in Section 6.6.10.4.1.6.

The above mentioned angle sequence is used for two specific updates: firstly, it is used for updating secondary diffraction-type RI’s EQ due to angle-dependent attenuation, In this case, the diffraction-type RI’s EQ coefficients will be updated N times according to the number of included edges in a given path. Secondly, the DiffractionAngleSequence is used for updating diffraction-type RI’s positional data including location and orientation. To generate diffraction-type RI’s location, the primary RI’s location is rotated with respect to corresponding edges by the amount of corresponding DiffractionAngleSequence. The diffraction-type RI’s directivity is obtained by rotating primary RI’s orientation in the same way. If there are multiple valid paths considering the given configuration of source, listener, and geometries, e.g. as shown in Figure 40– (B), multiple diffraction-type RIs per primary RI are generated.

The process for path fetching, path validation and diffraction-type RI creation for static scenes is summarized as below.

// Given: validPaths, source, listener, diffrPayload, diffrItemInitialEQ

// validPaths is the list of valid Path-type objects per primary RI and Path-type object

// contains (included) edgeIDSequence, faceIndicatorSequence, angleSequence, nEdges, and

// diffractionAngleSequence.

validPaths.clear();

// Path finding and validation process

for (int i = 0; i < source.pathIndexList.size(); i++) {

//Fetch the Path-type object from diffrPayload

initialEdgeID = source.pathIndexList[i].first;

pathLocalIndex = source.pathIndexList[i].second;

path = diffrPayload.staticPathDict[initialEdgeID][pathLocalIndex];

nEdges = path.nEdges;

edgeIDSequence = path.edgeIDSequence;

// edge is an Edge-type object which contains edge-relevant data

edge = diffrPayload.staticEdges[edgeIDSequence[0]];

// triangle is a Triangle-type object which contains triangle-relevant data such as

// 3 vertex IDs, centroid, and surface normal.

triangle = edge.faces[path.faceIndicatorSequence[0]];

angle1 = path.angleSequence[0];

sourceAngle = calculateAngle(source, edge, triangle);

if (sourceAngle > angle1)

Go to the next iteration.

edge = diffrPayload.staticEdges[edgeIDSequence[nEdges - 1]];

triangle = edge.triangles[path.faceIndicatorSequence[nEdges - 1]];

angle2 = path.angleSequence[nEdges - 1];

listenerAngle = calculateAngle(listener, edge, triangle);

if (listenerAngle < angle2)
        Go to the next iteration.

path.updateDiffractionAngleSequence(sourceAngle, listenerAngle); // as mentioned above

validPaths.push_back(path);

}

// Diffraction-type RI creation from validPaths

for (int i = 0; i < validPaths.size(); i++) {

position = getRotatedPosition(validPaths[i]);

orientation = getRotatedOrientation(validPaths[i]);

filterEQ = getDiffractionFilter(validPaths[i], diffrItemInitialEQ);

Create diffraction-type RI with position, orientation, and filterEQ.

}

###### Dynamic scenes

If a scene has dynamic geometries, the static paths fetched from the payload (i.e., only considering static geometries, diffrPayload.staticPathDict) need to be checked for occluded intermediate propagation paths by dynamic geometries. Intermediate propagation paths can be paths between included static edges or between a source and the first edge or between the listener and the last edge.

If these are not occluded, the previously mentioned diffraction angle calculation in terms of sourceAngle and listenerAngle shall be called. However, if there are occluded path segments, the path finding/fetching process around occluding dynamic geometries shall be called to determine the additional attenuation for dynamic occlusion at runtime performing a series of visibility tests.

For this process, the following steps need to be done:

- An initial visibility test is conducted to identify the dynamic geometries in the scene that are causing occlusion. Each dynamic geometry in the scene includes a dynamic path dictionary within the diffraction payload (diffrPayload.dynamicPathDict). The dynamic path dictionary of an occluding dynamic geometry is then used for the validation of dynamic paths.

- The occlusion status of various pairs of points needs to be checked in order to identify occluded paths. These pairs of points can include a source point and the first edge in each dynamic path, two possible adjacent dynamic edges, or the last edge in a given path and the listener point.

- Afterwards, it is necessary to perform an additional visibility test in order to identify the visible dynamic edges considering said pairs of points. This test considers the diffrPayload.dynamicPathDict associated with the occluding dynamic geometry.

- The relevant dynamic path list is then updated to determine which dynamic paths remain valid from these two occluded points.

- Once the validated dynamic paths are established, an additional attenuation filter is determined using the diffraction angle sequences. A visual representation of this scenario is provided in Figure 41.

Figure 41 — Example to show the case with occluded/not-occluded static paths with static and dynamic geometries

There are two possible static paths StaticPath –i and StaticPath – j. StaticPath –i is not occluded by dynamic geometries but, StaticPath –j is occluded by a dynamic geometry in between Edge C and the listener. As described above, by using the visible edge lists – one from Edge C and the other from the listener location – and precomputed dynamic path data (diffrPayload.dynamicPathDict), the relevant dynamic paths (DynPath –k, DynPath – l) can be found through the steps described in 6.6.10.4.1.5.2. The additional attenuation filter is updated from these two dynamic paths accordingly. This process is described as below.

// Given: validPaths, source, listener, diffrPayload, diffrItemInitialEQ, dynMeshes,

// Update source’s path index list as done for static scene.

source.pathIndexList = getPathIndexList(source, listener, diffrPayload);

validPaths.clear();

// Update validPaths with occluded static paths by dynamic geometries

For (int i = 0; i < source.pathIndexList.size(); i++) {

path = diffrPayload.staticPathDict
           [source.pathIndexList[i].first][ source.pathIndexList[i].second];

// Check if the given path

// is a valid static path without considering dynamic geometries

// it returns true if a given path is valid one

isValid = checkPath(path, source, listener);

if (isValid = true) {

if (isDynamicGeometryIncluded == true) {

// path.additionalDynEQs is composed of

// multiple lists of floating values to contain

// additional EQs for dynamic occlusion.

path.additionalDynEQs.clear();

for (int j = -1; j < path.nEdges; j++) {

if (j == -1) {

eLocation1 = source.location;

edge = diffrPayload.staticEdges[path.edgeIDSequence[0]];

eLocation2 = e.centerLocation;

} else if (j == path.nEdges - 1) {

edge = diffrPayload.staticEdges[path.edgeIDSequence[path.nEdges - 1]];

eLocation1 = e.centerLocation;

eLocation2 = listener.location;

} else {

edge1 = diffrPayload.staticEdges[path.edgeIDSequence[j + 0];

edge2 = diffrPayload.staticEdges[path.edgeIDSequence[j + 1];

eLocation1 = edge1.centerLocation;

eLocation2 = edge2.location;

}

occludingDynGeoID = getOccludingDynGeoID(eLocation1, eLocation2, dynTracer);

if (occludingDynGeoID is empty) {

Go to the next iteration.

}

// Find relevant dynamic paths

dynEdges = diffrPayload.dynamicEdges[occludingDynGeoID];

edgesFromE1 = getVisibleDynEdges(eLocation1, dynEdges, tracer);

edgesFromE2 = getVisibleDynEdges(eLocation2, dynEdges, tracer);

potentialDynPaths = getPotentialDynPaths (eLocation1, eLocation2, edgesFromE1, edgeFromE2);

// Update path’s additional dynamic EQs

for (auto& pDynPath : potentialDynPaths) {

pDynPath.updateDiffractionAngleSequence(eLocation1, eLocation2);

additionalDynEQ = getEQfromAngles(pDynPath.diffractionAngleSequence);

path.additionalDynEQs.push_back(additionalDynEQ);

{

}

}

}

else {

Go to the next path index in the loop.

}

validPaths.push_back(path);

}

Diffraction paths around dynamic geometries

In addition, it is necessary to check for potential diffraction paths around dynamic geometries that do not interact with static geometries. This process differs from static scenes because voxel data cannot be generated at encoding time due to the large number of potential configurations.

To fetch the potential dynamic diffraction paths, a first visibility test (to determine visible edges) is needed to determine the list of dynamic edges that are visible to both the source (primary RI) and the listener. This dynamic edge data is stored in diffrPayload.dynamicEdgeDict.

From these two lists of visible dynamic edges (one from the source and the other from the listener), the potential dynamic paths that start from a dynamic edge visible to the source and end at a dynamic edge visible to the listener are fetched from diffrPayload.dynamicPathDict.

Similar to finding additional dynamic paths, each dynamic path is then subjected to a validation process using sourceAngle and listenerAngle. During the validation process, the diffractionAngleSequence of the path is updated as described in Section 6.6.10.4.1.5.2.

If a dynamic path is determined to be valid, it is used to create a secondary diffraction-type RI. Additionally, whenever the positional data (location and orientation) of a dynamic geometry is changed, the vertex positions of the dynamic edges must be updated accordingly, using a combination of observer features as described in Section 6.6.10.4.1.5.2.

The process for path finding, path validation and diffraction-type RI creation in case of dynamic scenes is done as follows:

// Given: validPaths, source, listener, diffrPayload, diffrItemInitialEQ, dynMeshes,

// Update source’s path index list as done for static scene.

source.pathIndexList = getPathIndexList(source, listener, diffrPayload);

validPaths.clear();

// Update validPaths with unoccluded dynamic paths

for (auto& dynMesh: dynMeshes) {

dynEdges = diffrPayload.dynamicEdges[dynMesh.ID];

edgesFromS = getVisibleDynEdges(source.location, dynEdges, tracer);

edgesFromL = getVisibleDynEdges(listenr.location, dynEdges, tracer);

potentialDynPaths = getPotentialDynPaths(source.location, listener.location, edgesFromS, edgeFromL);

for (auto& dynPath: potentialDynPaths) {

dynPath.updateDiffractionAngleSequence(source.location, listener.location);

validPaths.push_back(dynPath);

}

}

createDiffrTypeRIs(validPaths);

###### Valid diffraction path activation and gain normalization

Activation refers to the process of flagging a given diffraction path to be rendered into an audible diffraction-type RI. In principle, all valid diffraction paths are activated, except for one case.

The exceptional case occurs for the longer diffraction paths in case there are numerous paths flagged as active. It is assumed that very long and/or contrived diffraction paths (i.e., diffracting on many geometries) will be inaudible. To limit the number of active diffraction paths, only the P shortest paths will remain active, where P is a predefined maximum amount of diffraction-type RI allowed.

Additionally, the gain of a rendered diffraction item can be set to zero (and therefore rendered inaudible) at the ray summation stage at the listener position. As explained before, each diffraction path has an associated gain and EQ coefficient values that are dependent on the associated diffraction angle sequences. The signal contribution of all diffraction-type RIs is summed at listener position. In order to avoid loudness peaks, the diffraction RI gains are normalized. In this context, rays with gain values below a defined culling gain threshold are removed from the normalization procedure and set to zero.

###### Slow update mode

The slow update mode is introduced in order to tackle some efficiency issues in the renderer. Namely, the Diffraction stage will be called multiple times if the direct line-of-sight between a source and the listener is started to be occluded or unoccluded by geometries in an alternating fashion. Each call, the path fetching, validation and diffraction-type RI creation routines are executed, which may lead to a very high computational load.

The slow update mode addresses this issue by calling the processes of the Diffraction stage at a slower rate, under the basic assumption is that a listener is not highly sensitive to changes in the diffracted sound when a source is not visible from the listener. Conversely, the listener is more  sensitive to the diffracted sound when the direct line-of-sight between a source and the listener is started to be occluded or unoccluded by geometries. This assumption allows for increased rendering efficiency without perceived sound degradation.

In other words, the renderer already needs to run all processes in the Diffraction stage including path finding, path validation, diffraction-type RI creation promptly when a source begins to be occluded in order to achieve smooth transitions. However, the renderer calls the processes at a slower rate while a source is being occluded.

To accomplish the slow mode, three integer numbers are included: itemCounter, updateCounter and updateRate. itemCounter is set as 0 at the beginning of the Diffraction stage update call and it increases by 1 at every iteration of renderItems. updateCounter initialized as 0 increases by 1 at every update cycle, where updateRate is the rate in the slow update mode. For instance, if updateRate is 10, the diffraction-type RIs are updated per every 10 cycles unless the source occlusion flag, i.e. isOcclusionStateChanged is true. The general process for slow update mode is described below.

// Given: renderList.listener, renderList.renderItems

// Initialize updateRate as 10 and updateCounter as 0 before update processing.

itemCounter = 0;

listener.update(renderList.listener);

for (auto& RI : renderList.renderItems) {

//Skip the exceptional cases as described in clause 6.6.10.4.1.1

if (RI.type != ItemType::Primary || RI.cSpace == CSpace::LCS) {

// LCS: Listener Coordinate System

continue;

}

source.update(RI); // as described in clause 6.6.10.4.1.2

if (RI.occlusionInfo is empty) {

source.isPreviouslyOccluded = false;

Go to the next iteration.

} else {

if (source.isPreviouslyOccluded == false)

source.isOcclusionStatusChanged = true;

else

source.isOcclusionStatusChanged = false;

source.isPreviouslyOccluded = true;

}

initializeEQs(diffrItemInitialEQs, 1.0);

if ((source.isOcclusionStatusChanged == true

|| (updateCounter + itemCounter)%updateRate == 0)) {

validPaths.clear();

updateValidPaths(validPaths);

// run the processes to update validPaths in clause 6.6.10.4.1.5

}

createDiffrTypeRIs(validPaths);

itemCounter++;

}

updateCounter++;

### Voxel-based occlusion and diffraction

#### General

#### The voxel-based occlusion stage provides occlusion information with respect to a direct path (i.e., a line-of-sight) from the audio source to the listener. The voxel-based diffraction stage provides the information required for generating additional diffracted sounds around occluding objects. The diffraction stage uses information that shall be provided by the occlusion stage.

The voxel-based occlusion and diffraction stages shall be activated if VOXDATA_PAYLOAD is present in the bitstream.

#### Data elements and variables

VoxDataMatrix	Voxel scene data 3D matrix

P	Diffraction map 2D matrix

B	Voxel scene bounding box

Lc	Listener position coordinates

Lvox	Listener voxel (enclosing Lc) grid indices,
where Lvox=(Lvoxx, Lvoxy,Lvoxz)

Lvox'	Listener adjacent (neighboring) voxel Lvox'∈N(L) grid indices

N(L)	Set of voxels adjacent to the listener voxel Lvox

Oc	Audio source position coordinates

Ovox	Audio source voxel (enclosing Oc) grid indices,
where Ovox=(Ovoxx, Ovoxy,Ovoxz)

Sc	Audio source reference point on the voxel scene bounding box B

Svox	Audio source reference voxel (enclosing Sc) grid indices,
where Svox=(Svoxx, Svoxy,Svoxz)

Xc	General position coordinate for Xc = {Lc, Oc, Sc}

Xvox	General voxel (enclosing Xc) grid indices,
where Xvox=(Xvoxx, Xvoxy,Xvoxz)

Vsize	Voxel size scalar

Vstart	Global coordinates of the first corner point defining the voxel scene,
where Vstart = voxScenePosS

Vend	Global coordinates of the second corner point defining the voxel scene,
where Vend = voxScenePosE

Vdim	Number of voxels for each voxel scene dimension,
where Vdim = voxSceneDimensions

Dprimary	Primary diffraction audio source data for Lvox

Dprimary'	Primary diffraction audio source data for Lvox'

Dsecondary	Secondary diffraction audio source data for Lvox

Dc	Primary diffraction source position coordinates

Dc'	Secondary diffraction source position coordinates

azD 	Diffraction source azimuth angle in respect to the listener position Lc

azD', azD*	Diffraction source azimuth angle in respect to the listener position Lc'

azK	Azimuth angle between Lvox and Kvox

elD 	Virtual diffraction source elevation angle in respect to the listener position Lc

rdiffracted	Estimation for the total diffraction path length between listener Lc and audio source Oc positions

rdirect	Direct distance between the listener Lc and audio source Oc positions

rpath	Diffraction path length between the listener voxel Lvox and audio source reference voxel Svox

routside	Distance between audio source reference point Sc and audio source Oc positions the outside the voxel scene bounding box B

rlistener_to_occluder	Listener-to-occluder distance

Iset	Ordered set of voxels intersected by the line-of-sight segment (Sc, Lc)

Ic	First closest to the listener occluding voxel position coordinates on the line-of-sight voxel set Ic∈ Iset

Pset	Ordered set of voxels approximating the shortest path from Svox to Lvox

Cset	Set of voxels representing corner voxels on the diffraction map

Cvox	Voxel on the diffraction path Cvox∈ Pset for obtaining the diffracted source azimuth azD value

Cvox' 	Voxel on the diffraction path Cvox∈ Pset for obtaining the diffracted source azimuth azD' value

tocclusion	Direct occlusion effect recovery control value, where
tocclusion= voxSceneOcclusionRecoveryValue[i]
if voxSceneOcclusionRecoveryFlag[i] == 1, 
tocclusion= 1 if voxSceneOcclusionRecoveryFlag[i] == 0

tdiffraction	Diffracted source attenuation effect control value, where tdiffraction= voxSceneDiffractionEffectControlValue
if voxSceneDiffrationEffectControlFlag == 1, 
tdiffraction= 0 if voxSceneDiffrationEffectControlFlag == 0

TA	Voxel-material transmission value controlling occlusion effect before the first closest to the listener occluding voxel Ic

TB	Voxel-material transmission value controlling occlusion effect after the first closest to the listener occluding voxel Ic

f	Interpolation factor for the direct occlusion gain

αdiffraction_angle	Estimation for the diffraction angle

αdiffraction_idx	Index for the diffraction gain lookup table

gdirect	Direct occlusion attenuation gain

gdiffracted	Diffracted source attenuation gain

gdiffracted_path	Diffracted source attenuation path effect gain contribution

gdiffracted_max_energy	Maximal value of the diffracted source attenuation gain

gprimary	Gain for primary diffraction audio source

gsecondary	Gain for secondary diffraction audio source

G3x3	Gaussian gain weight matrix

Kvox	Single-voxel occluding element

N(K)	Set of voxels adjacent to the single-voxel element Kvox

h 	Threshold value for the diffraction azimuth angle difference between the corresponding “primary” and “secondary” diffraction azimuth directions

tmin 	Distance from the ray origin and ray box intersection point for the ray box intersection algorithm

#### Stage description

##### Direct occlusion

###### Interfaces for real-time scene updates

The direct occlusion rendering stage shall support a runtime update interface for the following metadata changes during the rendering process of the current voxel sub-scene:

VoxDataMatrix	Voxel scene data 3D matrix (via the renderer API. The sequence of voxel blocks voxSceneElements() containing {voxSceneElementMatID, voxSceneElementPosPackedS, voxSceneElementPosPackedE} shall modify the voxel-based scene data VoxDataMatrix)

The direct occlusion rendering stage supports the runtime update interface for the modifiable configuration parameters listed in the Table 141 used for this rendering stage.

###### Voxel sub-scene selection

If two or more voxel sub-scene are available (defined by the variable numberOfVoxSubScenes), the voxel-based scene data 3D matrix VoxDataMatrix shall be determined using the current listener position coordinates Lc:

| Lc∈B | (192) |
| --- | --- |

where the voxel scene bounding box B shall be defined as:

| B = Vstart, Vend | (193) |
| --- | --- |

If the listener is outside of the current bounding box B,

| Lc∉B | (194) |
| --- | --- |

and another voxel sub-scene enclosing the current listener position Lc is available, then a voxel sub-scene switch shall be performed.

Figure 42 – Example of two overlapping voxel sub-scenes and listener positions trajectories (leaving the current voxel sub-scene bounding box B)

###### Listener position handling

The following processing steps shall be executed for each listener position coordinates Lc update and voxel-based scene VoxDataMatrix update.

- Get listener voxel Lvox indices for the listener position coordinates Lc using Cartesian coordinate Xc to voxel grid indices Xvox conversion equation (195), for X={L,S,O}:

| Xvox = ceil Xc – VstartVsize+12 | (195) |
| --- | --- |

where:

| Vsize =Vendi*-Vstarti*Vdimi*, i*: Vdim[i*] =maxi(Vdim[i]) | (196) |
| --- | --- |

- Check the listener voxel Lvox type:

  - if voxel type is associated with a sound occluding material (i.e., not sound transmitting media): the sound transmission gains (associated with Lvox voxel material transmission coefficients) shall be combined with all corresponding audio source gains

###### Audio source position handling

The following processing steps shall be executed for each specified audio source position Oc update.

- Determine whether the audio source position Oc is inside of the voxel scene bounding box B or not:

  - if Oc ∉ B:		get the source reference voxel Svox indices, using equation (195), for the line-of-sight intersection position coordinates Sc, determined by the ray box intersection algorithm (which determines tmin), calculating an intersection point of the line (Lc, Oc) with the voxel scene bounding box B; set the distance between the audio source Oc and scene bounding box B:

| routside = ¦¦Oc – Sc¦¦ | (197) |
| --- | --- |
| Sc=tmin (Oc- Lc) | (198) |

  - if Oc ∈ B:		get the audio source voxel Ovox indices, using equation (195), for the audio source position coordinates Oc coordinates; set zero distance between the audio source Oc and scene bounding box B:

| routside = 0 | (199) |
| --- | --- |

set the following variables:

| Sc=Oc | (200) |
| --- | --- |
| Svox=Ovox | (201) |

check the audio object voxel Ovox type and if voxel type is associated with a sound occluding material (i.e., not sound transmitting media) then combine the sound transmission gains (associated with Ovox voxel material transmission coefficients) with the corresponding audio source gains.

The following pseudocode represents the ray box intersection algorithm for tmin:

// ----------------------------------------------------------------------------------------

// Ray box intersection algorithm

//

// Input

//  origin, direction: 3D vectors describing the ray 
//  vmin, vmax: ordered 3D corners of the bounding box

// Output

//  tmin = (txmin, tymin, tzmin): distance from the ray origin

//

rayBoxIntersection(origin[3], direction[3], vmin[3], vmax[3])

{

if (direction[0] >= 0) {

txmin = (vmin[0] - origin[0]) / direction[0];

txmax = (vmax[0] - origin[0]) / direction[0];

} else {

txmin = (vmax[0] - origin[0]) / direction[0];

txmax = (vmin[0] - origin[0]) / direction[0];

}

if (direction[1] >= 0) {

tymin = (vmin[1] - origin[1]) / direction[1];

tymax = (vmax[1] - origin[1]) / direction[1];

} else {

tymin = (vmax[1] - origin[1]) / direction[1];

tymax = (vmin[1] - origin[1]) / direction[1];

}

if ((txmin > tymax) || (tymin > txmax)) { txmin = -1.0; } // No ray/box intersection

if (tymin > txmin) { txmin = tymin; }

if (tymax < txmax) { txmax = tymax; }

if (direction[2] >= 0) {

tzmin = (vmin[2] - origin[2]) / direction[2];

tzmax = (vmax[2] - origin[2]) / direction[2];

} else {

tzmin = (vmax[2] - origin[2]) / direction[2];

tzmax = (vmin[2] - origin[2]) / direction[2];

}

if ((txmin > tzmax) || (tzmin > txmax)) { txmin = -1; } // No ray/box intersection

if (tzmin > txmin) { txmin = tzmin; }

if (tzmax < txmax) { txmax = tzmax; }

}

###### Voxel grid line-of-sight traversing

- Get the ordered set Iset of all voxels intersected by the listener-to-source line-of-sight segment (Lc, Sc) by applying the segment traversal algorithm on the 3D voxel grid VoxDataMatrix

- Check the line-of-sight voxel Iset types:

  - if all voxel types are not occluding (e.g., sound transmitting media): no further attenuation shall be applied and the sub-sequent occlusion and diffraction stages shall be skipped

  - otherwise: the direct occlusion attenuation gain shall be applied as follows

B

Vstart

Vend

B

Vstart

Vend

Figure 43 – Example of the voxel scene data 3D matrix, audio source and listener positions

The following pseudocode represents the segment traversal algorithm on the 3D voxel grid:

// ----------------------------------------------------------------------------------------

// Voxel traversal algorithm

//

// Input

//  gridSize: 3D vector describing the grid size 
//  voxelSize: Size of voxel

//  minBound, maxBound: Ordered 3D corners of the bounding box

//  listenerPos: 3D vector describing the listener position

//  objectPos: 3D vector describing the audio source position

//  listenerPosIdx: 3D voxel index of the listener

//  objectPosIdx: 3D voxel index of the audio source

// Output

//  N: Number of voxels intersected by the listener-to-source

//  voxelIdx: N dimensional vector of voxels intersected by the listener-to-source

//

voxelTraversal(gridSize[3], voxelSize, minBound[3], maxBound[3], listenerPos[3], objectPos[3], listenerPosIdx[3], objectPosIdx[3], voxelIdx[][3])

{

step[3] = {0, 0, 0};

tVoxel[3] = {0, 0, 0};

idx[0] = listenerPosIdx[0];

idx[1] = listenerPosIdx[1];

idx[2] = listenerPosIdx[2];

vec[0] = objectPos[0] - listenerPos[0];

vec[1] = objectPos[1] - listenerPos[1];

vec[2] = objectPos[2] - listenerPos[2];

norm = sqrt(vec[0] * vec[0] + vec[1] * vec[1] + vec[2] * vec[2]);

dir[0] = vec[0] / norm;

dir[1] = vec[1] / norm;

dir[2] = vec[2] / norm;

for (i = 0; i < 3; i++) {

if (idx[i] == gridSize[i]) { idx[i] = idx[i] - 1; }

if (dir[i] >= 0) {

tVoxel[i] = (idx[i] + 1) / gridSize[i];

step[i] = 1;

} else {

tVoxel[i] = (idx[i]) / gridSize[i];

step[i] = -1;

}

}

voxelMax[0] = minBound[0] + tVoxel[0] * (maxBound[0] - minBound[0]);

voxelMax[1] = minBound[1] + tVoxel[1] * (maxBound[1] - minBound[1]);

voxelMax[2] = minBound[2] + tVoxel[2] * (maxBound[2] - minBound[2]);

tMax[0] = (voxelMax[0] - listenerPos[0]) / dir[0];

tMax[1] = (voxelMax[1] - listenerPos[1]) / dir[1];

tMax[2] = (voxelMax[2] - listenerPos[2]) / dir[2];

tDelta[0] = size_vox / abs(dir[0]);

tDelta[1] = size_vox / abs(dir[1]);

tDelta[2] = size_vox / abs(dir[2]);

N = 0;

while (

(idx[0] * step[0] <= objectPosIdx[0] * step[0]) &&

(idx[1] * step[1] <= objectPosIdx[1] * step[1]) &&

(idx[2] * step[2] <= objectPosIdx[2] * step[2]) &&

(idx[0] <  gridSize[0]) && (idx[0] >= 0) &&

(idx[1] <  gridSize[1]) && (idx[1] >= 0) &&

(idx[2] <  gridSize[2]) && (idx[2] >= 0)) {

voxelIdx[N][0] = idx[0];

voxelIdx[N][1] = idx[1];

voxelIdx[N][2] = idx[2];

N++;

if (tMax[0] < tMax[1]) {

if (tMax[0] < tMax[2]) {

idx[0] = idx[0] + step[0];

tMax[0] = tMax[0] + tDelta[0];

} else {

idx[2] = idx[2] + step[2];

tMax[2] = tMax[2] + tDelta[2];

}

} else {

if (tMax[1] < tMax[2]) {

idx[1] = idx[1] + step[1];

tMax[1] = tMax[1] + tDelta[1];

} else {

idx[2] = idx[2] + step_[2];

tMax[2] = tMax[2] + tDelta[2];

}

}

}

}

###### Local and global occluders

Along the direct path from listener position coordinates Lc to audio source point Sc, represented by the ordered set of voxels Iset, only two particular voxel types are considered by the direct occlusion modeling. These voxels shall be associated with the occluding material having the transmission coefficients smaller than one to produce an acoustic occlusion effect (hereafter referred to as “occluding voxels”). This occlusion effect shall be modeled either as

- local 		for 	voxSceneDiffractionEnabledFlag == 0

(i.e., producing direct occlusion effect only)

or

- global 		for 	voxSceneDiffractionEnabeldFlag == 1

(i.e., producing both direct occlusion and acoustic diffraction effects)

occlusion mode. The local occlusion mode setting may be used for small scale acoustic obstacles associated with high sound transmission coefficients. The global occlusion mode setting is useful for large scale obstruction associated with significant sound absorption. These mode settings are specified at the encoder side (via scene definition data) per occluding voxel material.

Only the first voxels from Iset closest to the listener position Lc indicating local occlusion mode (if any) and global occlusion mode (if any) shall be used to obtain the direct occlusion gain gdirect. The direct occlusion gain gdirect shall be calculated by linear interpolation between the corresponding voxel-material transmission values TA (i.e., occlusion effect before the first closest to the listener occluding voxel) and TB (i.e., occlusion effect after the first closest to the listener occluding voxel):

| gdirect=f TA+1-f TB | (202) |
| --- | --- |

The direct occlusion gain gdirect for the audio source Oc shall be combined with the gains of other rendering stages.

The local and global voxel appearance order in respect to set of voxels Iset determines the values TA and TB as given in Table 142. The corresponding voxel material transmission coefficients tlocal and tglobal shall be obtained from the bitstream elements containing the transmissive material coefficients.

- Table 142 – Definition of interpolation coefficients

| Occluding voxel type |  | Coefficients for interpolation |  |
| --- | --- | --- | --- |
| 1st voxel | 2nd voxel | TA | TB |
| local | global | tglobal | tglobal tlocal |
| local | any other local | 1 | tlocal |
| global | any | 1 | tglobal |

###### Occlusion effect recovery

Occlusion recovery is a process of reducing the direct occlusion gain gdirect with increased distance between the listener and occluder. It is used to imitate the contribution of acoustic diffraction to the sound energy far from the local occluder (for which the explicit acoustic diffraction modelling is not applied). This reduces “strength and length” of the “occlusion shadow” behind the small-scale occluder.

The direct occlusion effect recovery factor f shall be obtained using the listener-to-occluder distance rlistener_to_occluder and occlusion effect recovery value tocclusion as:

| f=21+2-tocclusion rlistener_to_occluder-1 | (203) |
| --- | --- |

| rlistener_to_occluder=¦¦Lc–Ic¦¦ | (204) |
| --- | --- |

Figure 44 – Interpolation factor values for different occlusion effect recovery values

The value tocclusion shall be specified per each occluding voxel material to control the strength of the occlusion effect spatial relaxation. It determines the how fast the occlusion effect loses its relevance with the distance from an occluding geometry.

Figure 45 – Example of the direct occlusion gain estimation method

##### Diffraction

###### Interfaces for real-time scene updates

The acoustic diffraction rendering stage shall support the runtime update interface for the following metadata changes during the rendering process of the current voxel sub-scene:

VoxDataDiffractionMap	Voxel scene diffraction map data 2D matrix (via the renderer API. The sequence of voxel blocks voxSceneDiffractionMap() containing {voxDiffractionMapValue, voxDiffractionMapPosPackedS, voxDiffractionMapPosPackedE} shall modify the voxel-based scene data VoxDataDiffractionMap)

The diffraction rendering stage supports the runtime  update interfaces for the modifiable configuration parameters listed in the Table 141 used for this rendering stage.

###### Diffraction map

The diffraction modeling shall be performed using the diffraction map matrix P (VoxDataDiffractionMap) obtained as described in 6.3.2.1.3.

###### Corner detection

The following processing steps shall be executed for each update of the diffraction map P.

- Get set Cset of the diffraction map P corner voxels (Cset: VoxDataCornerMap[x][y]) using the following pseudo code (for detecting and removing non-corner voxels):

for (int x = 0; x < voxSceneDimensions[0]; x++) {

for (int y = 0; y < voxSceneDimensions[1]; y++) {

VoxDataCornerMap[x][y] = VoxDataDiffractionMap[x][y]

}

}

for (int x = 1; x < voxSceneDimensions[0]-1; x++) {

for (int y = 1; y < voxSceneDimensions[1]-1; y++) {

if (VoxDataDiffractionMap[x][y] == 1) {

if ((VoxDataDiffractionMap[x-1][y] == 1) &&

(VoxDataDiffractionMap[x+1][y] == 1)) ||

((VoxDataDiffractionMap[x][y-1] == 1) &&

(VoxDataDiffractionMap[x][y+1] == 1)) {

VoxDataCornerMap[x][y] = 0

}

}

}

}

###### Diffracted path finding

  - If the information for the diffraction modelling (i.e., Cvox, rpath) for the current voxel scene state (i.e., Lvox, Svox and P) is available in either the memory or the bitstream, it shall be applied. Otherwise, it shall be calculated.

The scene state shall be defined via the input parameters Lvox, Svox and P for the function DiffractionInfoCalculation(). It includes the path-finding algorithm, voxel Cvox selection and path length rpath estimation steps for obtaining the diffraction path information, see Figure 46:

| Cvox, rpath=DiffractionInfoCalculation(Lvox, Svox, P) | (205) |
| --- | --- |

The diffraction path information, if it is available, shall be directly obtained from the memory cache or bitstream element voxSceneDiffractionPreComputedPathData() for the corresponding scene state and applied without the function DiffractionInfoCalculation() call.

Figure 46 – Example of the scene state with pre-computed diffraction path information

  - Get ordered set of voxels Pset representing an approximation of the shortest path between the listener Lvox and audio source reference Svox voxels by running VoxDataDiffractionMap grid path-finding algorithm.

The following pseudocode represents the path-finding algorithm on the 2D voxel grid:

// ----------------------------------------------------------------------------------------

// Variables used for path-finding algorithm

struct jps_node

{

x; y; // coordinates of the node

g; h; // Euclidean and Manhattan distances of the node to active node

f; // sum of g and h distance

opened; closed; // node opened and closed flags

struct jps_node* parent; // parent of node

};

vector<vector<jps_node>> jps_grid_nodes: // 2D matrix of size voxSceneDimensions

vector<jps_node*> opened_list, closed_list: // opened and closed node lists

vector<pair<int, int>> neighbors: // neighbor list

// ----------------------------------------------------------------------------------------

// Path-finding algorithm (jump point search)

//

// Input

//  startX, startY: listener 2D voxel coordinates

//  endX, endY: audio source 2D voxel coordinates

//  VoxDataDiffractionMap: 2D matrix of size voxSceneDimensions

// Output

//  pathFound: indicating if there is a path from the listener to the audio source

//  path: N dimensional vector containing the path from listener to the audio source

//

findPath(startX, startY, endX, endY, VoxDataDiffractionMap[][], vector<int>& path)

{

counter = 0;

pathFound = false;

path.clear();

opened_list.clear();

closed_list.clear();

startNode = getNodeAt(startX, startY);

endNode = getNodeAt(endX, endY);

// Check if start or end node is blocked

if (isWalkableAt(startNode->x, startNode->y, VoxDataDiffractionMap) &&
      isWalkableAt(endNode->x, endNode->y, VoxDataDiffractionMap)) {

startNode->opened = true;

opened_list.push_back(startNode);

// List is empty when no successors have been found

while (opened_list.size()) {

activeNode = m_opened_list.back();

activeNode->closed = true;

opened_list.pop_back();

closed_list.push_back(activeNode);

if (activeNode->x == endNode->x && activeNode->y == endNode->y) {

backtracePath(activeNode, path);

pathFound = true;

break;

}

IdentifySuccessors(activeNode, endX, endY, VoxDataDiffractionMap);

counter++;

}

}

}

// ----------------------------------------------------------------------------------------

// Generate the path from the active node to the start node by adding all node coordinates

// of the jump points to the path and add intermediate points between the jump points with // a line drawing algorithm

backtracePath(jps_node* activeNode, vector<int>& path)

{

vector<pair<int, int>> path_points;

path_points.push_back(make_pair(activeNode->x, activeNode->y));

while ((activeNode = activeNode->parent) != NULL) {

path_points.push_back(make_pair(activeNode->x, activeNode->y));

}

vector<pair<int, int>> smooth_path;

it = path_points.begin();

end = path_points.end();

vector<pair<int, int>> lastJump;

smooth_path.push_back(*it);

while (it != end) {

next = it + 1;

while (next != end) {

vector<pair<int, int>> line;

interpolateLine(it->first, it->second, next->first, next->second, line);

if (line.size() != 0) {

lastJump.clear();

copy(line.begin() + 1, line.end(), back_inserter(lastJump));

next++;

} else {

next--;

break;

}

}

for (point : lastJump) { smooth_path.push_back(move(point)); }

it = next;

}

for (point : smooth_path) {

path.push_back(point.first);

path.push_back(point.second);

}

}

// ----------------------------------------------------------------------------------------

// Run a jump point search in direction of each available neighbour, adding any open nodes
// found to the open list; returns all the nodes have found jumpable from the activeNode

identifySuccessors(jps_node* activeNode, endX, endY, vector<jps_node*> opened_list)

{

findNeighbors(activeNode);

for (neighbor : neighbors) {

jx = neighbor.first;

jy = neighbor.second;

jumpPoint = jump(jx, jy, activeNode->x, activeNode->y, endX, endY);

if (jumpPoint) {

jumpNode = getNodeAt(jx, jy);

if (jumpNode->closed) { continue; }

d = euclidean(abs(jx - activeNode->x), abs(jy - activeNode->y));

ng = activeNode->g + d;

if (!jumpNode->opened || ng < jumpNode->g) {

jumpNode->g = ng;

if (jumpNode->h == 0) {

jumpNode->h = manhattan(abs(jx - endX), abs(jy - endY));

}

jumpNode->f = jumpNode->g + jumpNode->h;

jumpNode->parent = activeNode;

if (!jumpNode->opened) {

jumpNode->opened = true;

opened_list.push_back(jumpNode);

}

// keep open list sorted by f value in descending order

for (i = m_opened_list.size() - 1; i > 0; i--) {

if (opened_list[i]->f > opened_list[i - 1]->f) {

swap(opened_list[i], opened_list[i - 1]);

}

}

}

}

}

}

// ----------------------------------------------------------------------------------------

// Search recursively in the direction [px, py] from [x, y], untill a jump point is found

jump(x, y, px, py, endX, endY)

{

dx = x - px;

dy = y - py;

if (!isWalkableAt(x, y)) {

return false;

} else {

if (x == endX && y == endY) { return true; }

}

// check for forced neighbors along the diagonal

if (dx != 0 && dy != 0) {

if ((isWalkableAt((x - dx), (y + dy)) && !isWalkableAt((x - dx), y))

|| (isWalkableAt((x + dx), (y - dy)) && !isWalkableAt(x, (y - dy)))) {

return true;

}

// when moving diagonally, check for vertical/horizontal jump points

xpdx = x + dx;

ypdy = y + dy;

if (jump(xpdx, y, x, y, endX, endY) || jump(x, ypdy, x, y, endX, endY)) {

return true;

}

} else { // horizontally/vertically

if (dx != 0) {

if ((isWalkableAt((x + dx), (y + 1)) && !isWalkableAt(x, (y + 1)))

|| (isWalkableAt((x + dx), (y - 1)) && !isWalkableAt(x, (y - 1)))) {

return true;

}

} else {

if ((isWalkableAt((x + 1), (y + dy)) && !isWalkableAt((x + 1), y))

|| (isWalkableAt((x - 1), (y + dy)) && !isWalkableAt((x - 1), y))) {

return true;

}

}

}

px = x;

py = y;

x = x + dx;

y = y + dy;

return jump(x, y, px, py, endX, endY);

}

// ----------------------------------------------------------------------------------------

// Find the neighbors for the active Node. If the node has a parent, prune the neighbors

// based on the jump point search algorithm, otherwise return all available neighbors.

findNeighbors(jps_node* activeNode)

{

jps_node* parent = activeNode->parent;

x = activeNode->x;

y = activeNode->y;

neighbors.clear();

if (parent) {

px = parent->x;

py = parent->y;

dx = (x - px) / max(abs(x - px), 1);

dy = (y - py) / max(abs(y - py), 1);

if (dx != 0 && dy != 0) { // Diagonal

if (isWalkableAt(x, (y + dy))) {

neighbors.push_back(make_pair(x, y + dy));

}

if (isWalkableAt((x + dx), y)) {

neighbors.push_back(make_pair(x + dx, y));

}

if (isWalkableAt(x, (y + dy)) || isWalkableAt((x + dx), y)) {

neighbors.push_back(make_pair(x + dx, y + dy));

}

if (!isWalkableAt((x - dx), y) && isWalkableAt(x, (y + dy))) {

neighbors.push_back(make_pair(x - dx, y + dy));

}

if (!isWalkableAt(x, (y - dy)) && isWalkableAt((x + dx), y)) {

neighbors.push_back(make_pair(x + dx, y - dy));

}

} else { // Horizontal / vertical

if (dx == 0) {

if (isWalkableAt(x, (y + dy))) {

if (isWalkableAt(x, (y + dy))) {

neighbors.push_back(make_pair(x, y + dy));

}

if (!isWalkableAt((x + 1), y)) {

neighbors.push_back(make_pair(x + 1, y + dy));

}

if (!isWalkableAt((x - 1), y)) {

neighbors.push_back(make_pair(x - 1, y + dy));

}

}

} else {

if (isWalkableAt((x + dx), y)) {

if (isWalkableAt((x + dx), y)) {

neighbors.push_back(make_pair(x + dx, y));

}

if (!isWalkableAt(x, (y + 1))) {

neighbors.push_back(make_pair(x + dx, y + 1));

}

if (!isWalkableAt(x, (y - 1))) {

neighbors.push_back(make_pair(x + dx, y - 1));

}

}

}

}

} else {

d0 = false;

d1 = false;

d2 = false;

d3 = false;

if (isWalkableAt(x, y - 1)) {

neighbors.push_back(make_pair(x, y - 1));

d0 = d1 = true;

}

if (isWalkableAt(x + 1, y)) {

neighbors.push_back(make_pair(x + 1, y));

d1 = d2 = true;

}

if (isWalkableAt(x, y + 1)) {

neighbors.push_back(make_pair(x, y + 1));

d2 = d3 = true;

}

if (isWalkableAt(x - 1, y)) {

neighbors.push_back(make_pair(x - 1, y));

d3 = d0 = true;

}

if (d0 && isWalkableAt(x - 1, y - 1)) {

neighbors.push_back(make_pair(x - 1, y - 1));

}

if (d1 && isWalkableAt(x + 1, y - 1)) {

neighbors.push_back(make_pair(x + 1, y - 1));

}

if (d2 && isWalkableAt(x + 1, y + 1)) {

neighbors.push_back(make_pair(x + 1, y + 1));

}

if (d3 && isWalkableAt(x - 1, y + 1)) {

neighbors.push_back(make_pair(x - 1, y + 1));

}

}

}

// ----------------------------------------------------------------------------------------

// Interpolate line from [x0, y0] to [x1, y1] using Bresenham's algorithm

interpolateLine(x0, y0, x1, y1, vector<pair<int, int>>& line)

{

line.clear();

dx = abs(x1 - x0);

dy = abs(y1 - y0);

sx = (x0 < x1) ? 1 : -1;

sy = (y0 < y1) ? 1 : -1;

err = dx - dy;

while (true) {

if (isWalkableAt(x0, y0)) {

line.push_back(make_pair(x0, y0));

} else {

line.clear();

break;

}

if (x0 == x1 && y0 == y1) { break; }

e2 = 2 * err;

if (e2 > -dy) {

err = err - dy;

x0 = x0 + sx;

}

if (e2 < dx) {

err = err + dx;

y0 = y0 + sy;

}

}

}

// ----------------------------------------------------------------------------------------

// Get node at position at [x, y]

getNodeAt(x, y)

{

return jps_grid_nodes[x][y];

}

// ----------------------------------------------------------------------------------------

// Check if position at [x, y] is inside the VoxDataDiffractionMap and not blocked

isWalkableAt(x, y)

{

return (isInsideAt(x, y) && VoxDataDiffractionMap[x][y] == false);

}

// ----------------------------------------------------------------------------------------

// Check if position at [x, y] is inside the VoxDataDiffractionMap

isInsideAt(x, y)

{

return (x >= 0 && x < voxSceneDimensions[0]) && (y >= 0 && y < voxSceneDimensions[1]);

}

// ----------------------------------------------------------------------------------------

// Calculate Euclidean distance

euclidean(dx, dy)

{

distance = sqrt(dx * dx + dy * dy);

}

// ----------------------------------------------------------------------------------------

// Calculate Manhattan distance

manhattan(dx, dy)

{

distance = (dx + dy);

}

If a path is not found (i.e., Pset is empty) then all corresponding diffracted source(s) shall be muted. If a path is found (i.e., Pset is not empty) then the modelling of the diffracted source(s) shall be performed as follows:

  - Select the voxel Cvox ∈ Pset, which is close to a ‘visible’ (from the listener position coordinates Lc) corner voxel belonging to Cset and causing the path Pset direction change; if there are more than one such corner, the one furthest away from the listener along the path Pset shall be selected

  - Get the diffraction path length rpath (=r_path) within the voxel scene bounding box B calculated from the path Pset (=P_set),voxel size Vsize (=V_size), and direct distance between the listener Lc and audio source Oc positions rdirect (=r_direct) using the following pseudo code:

length_on_grid = 0

for (int i = 0; i < P_set.size – 1; i++) {

if ((P_set.x[i] != P_set.x[i+1]) && (P_set.y[i] != P_set.y[i+1])) {

step = sqrt(2)

} else {

step = 1

}

length_on_grid = length_on_grid + step

}

r_path = max(length_on_grid * voxel_size, r direct)

  - When the diffraction path information Cvox, rpath is obtained for the current scene state Lvox, Svox, VoxDataDiffractionMap, this information can be converted to the existing bitstream format to make it accessible to a renderer. The format of the metadata corresponds to the bitstream syntax elements described in 6.2.5. The method for conversion between uncompressed and compressed VoxDataDiffractionMap data representation formats is specified in 6.3.2.1.4.

  - The Diffraction Look-Up Table (DLUT) dataset shall be extended for all voxels of the diffraction map P  on the line segment [Lvox,Cvox] connecting the user position Lc to the corresponding corner voxel Cvox for a single run of the pathfinding algorithm, see Figure 47.

Figure 47 — Examples of DLUT voxels (red) with the same corner Cvox for the scene state Lvox, P, Svox.

###### Diffracted source position estimation

  - Get the diffracted source azimuth azD from listener voxel Lvox and selected voxel Cvox:

| azD=arctanCvoxy - LvoxyCvoxx - Lvoxx | (206) |
| --- | --- |

  - Get the diffracted source elevation elD from the listener voxel Lvox and source reference voxel Svox:

| elD=arctanSvoxz - LvoxzSvoxx - Lvoxx2+Svoxy - Lvoxy2 | (207) |
| --- | --- |

  - Get the diffracted source radius rdiffracted from the listener voxel Lvox, source reference voxel Svox and corresponding distances rpath, routside:

| rdiffracted=rpath2+Svoxz - Lvoxz2+routside | (208) |
| --- | --- |

  - Get the direct audio source distance r from the listener Lc and audio source Oc positions as:

| rdirect = ¦¦Oc – Lc¦¦ | (209) |
| --- | --- |

  - Set the virtual diffraction source position coordinates Dc based on the listener position Lcto correspond to the diffracted source azimuth azD, elevation elD and direct distance rdirect:

| Dc = sph2cart(azD, elD, rdirect) + Lc | (210) |
| --- | --- |

Figure 48 – Example of the diffracted source position estimation method

  - Get the diffracted source attenuation gain gdiffracted modelling the total diffracted path effect as:

| gdiffracted_path=tdiffraction⋅DiffractionFilterCoeffsαdiffraction_idx | (211) |
| --- | --- |

where

| αdiffraction_idx=180-ceilαdiffraction_angle | (212) |  |
| --- | --- | --- |
| αdiffraction_angle=2⋅rad2degarcsinrdirect/rdiffracted | (213) |

  - Apply the diffracted energy limitation requiring that the sum of the direct and diffracted energy behind an occluding voxel (causing the diffraction effect) does not exceed the energy before it:

| gdirect + gdiffracted≤A | (214) |
| --- | --- |

Get the maximal value gdiffracted_max_energy of the diffracted source attenuation gain gdiffracted (≤gdiffracted_max_energy) using equation (195) for gdirect as:

| gdiffracted_max_energy=1-fA-B | (215) |
| --- | --- |

  - Set the diffracted source attenuation gain gdiffracted as:

| gdiffracted=min(gdiffracted_path,gdiffracted_max_energy) | (216) |
| --- | --- |

###### Primary and secondary diffraction audio sources

The “primary” and “secondary” diffraction audio source data shall be defined by two sets containing the corresponding virtual diffraction source position and diffracted source attenuation gain. The “primary” diffraction audio source data Dprimary shall be defined as:

| Dprimary=Dc, gprimary | (217) |
| --- | --- |

The “secondary” diffraction audio source data Dsecondary shall be obtained using the “primary” data Dprimary:

| Dsecondary=Dc', gsecondary | (218) |
| --- | --- |

The “secondary” virtual diffraction source position Dc'  shall be obtained from the (pre-)computed (or cached) DLUT “primary” data. In contrast to the “primary” diffraction audio source data Dprimary, the “secondary” diffraction audio source data Dsecondary is not part of the DLUT data but calculated during the audio rendering process.

The “secondary” diffraction audio source data Dsecondary is calculated using one of the following two variants of the function SecondaryDiffractionCalculation(), depending on the listener position Lc and voxel occluding elements causing the acoustic diffraction effect. These methods differ from each other by the following functional dependencies:

(A) “General” method:

| Dsecondary=SecondaryDiffractionCalculationDprimary, Dprimary', h | (219) |
| --- | --- |

where Dprimary' data is the “primary” diffraction audio source data for the listener adjacent (neighboring) voxel Lvox'; the variable h represents a threshold value for the diffraction azimuth angle difference between the corresponding “primary” and “secondary” diffraction azimuth directions.

(B) “Single-voxel” method:

| Dsecondary=SecondaryDiffractionCalculationDprimary, Kvox | (220) |
| --- | --- |

where Kvox is the single-voxel occluding element (causing the acoustic diffraction effect) adjacent to the listener voxel Lvox.

Processing steps at the scene initialization and diffraction map update stages

The set  N(K) of voxels, for which the “single-voxel” method (B) shall be applied, shall be obtained at the scene initialization and diffraction map P update stages to prepare for the method selection.

The set N(K) of voxels adjacent to the single-voxel occluding element Kvox shall be determined by evaluating the results of application of the following matrix comparison for all diffraction map P values:

| Nx,y(K)=111101111 , if Px+1, y-1Px+1, y+0Px+1, y+1Px+0, y-1Px,yPx+0, y+1Px-1, y-1Px-1, y+0Px-1, y+1==000010000 | (221) |
| --- | --- |

The set N(K) contains all voxels adjacent to all single-voxel occluding elements on the diffraction map P.

Figure 49 — Set N(K) of the voxel neighborhood of the single-voxel element Kvox.

Processing steps at the listener or audio source voxel updates

For the listener voxel Lvox the following check shall be performed to select the “secondary” diffraction audio source data estimation method.

(A) “general” method shall be applied if the listener voxel Lvox is not adjacent to the single-voxel occluding element K or the corresponding corner voxel Cvox is far from the listener voxel Lvox:

| Lvox∉N(K) or Lvox-Cvox≥1.5 | (222) |
| --- | --- |

(B) “single-voxel” method shall be applied if the listener voxel Lvox is adjacent to the single-voxel occluding element Kvox and the corresponding corner voxel Cvox is close to the listener voxel Lvox:

| Lvox∈N(K) and Lvox-Cvox<1.5 | (223) |
| --- | --- |

where the distance Lvox-Cvox shall be expressed in the number of the voxel size Vsize unites.

“General” method

If the listener voxel Lvox does not belong to the neighborhood voxel set N(K) of any single-voxel occluding element Kvox or the listener voxel Lvox and the corresponding corner voxel Cvox are not adjacent to each other, the following processing steps of the “general” method (A) for calculation of function SecondaryDiffractionCalculation() shall be performed:

- Select one voxel Lvox'∈N(L) from the voxel set N(L) of the listener voxel Lvox neighborhood on the diffraction map P.

Figure 50 — Set N(L) of the voxel neighborhood of the listener voxel Lvox.

The following selection order for the neighbouring voxel Lvox' shall be applied:

| Lvox'=Li'Lvox, Li'Lvox∈L7'L3'L5'L2'LvoxL1'L8'L4'L6', i=1…8 | (224) |
| --- | --- |
| Li'Lvox∈Lvoxx+1, Lvoxy-1Lvoxx+1, LvoxyLvoxx+1, Lvoxy+1Lvoxx, Lvoxy-1LvoxLvoxx, Lvoxy+1Lvoxx-1, Lvoxy-1Lvoxx-1, LvoxyLvoxx-1, Lvoxy+1 | (225) |

This results in the following sequence of the neighboring voxel Lvox' candidates:

| Lvox'∈Lvoxx, Lvoxy+1, Lvoxx, Lvoxy-1, Lvoxx+1, Lvoxy,Lvoxx-1, Lvoxy, Lvoxx+1, Lvoxy+1, Lvoxx-1, Lvoxy+1, Lvoxx+1, Lvoxy-1,Lvoxx-1, Lvoxy-1 | (226) |
| --- | --- |

If the “secondary” diffraction audio source data Dsecondary is not determined, but all voxels Li' of the neighborhood voxel set N(L) are already processed – define the “primary” diffraction audio source only (without the “secondary” one):

| gprimary=gdiffracted, gsecondary= 0 | (227) |
| --- | --- |

- Check whether the neighboring voxel Lvox' represents a valid candidate for further processing by checking simultaneous fulfilment of the following three conditions:

  - Lvox' is inside of the diffraction map P dimensions

  - Lvox' is of the sound transmission media type (i.e., if of the air-type and not an occluder-voxel)

  - Lc is not in the direct (i.e., un-occluded) view-line from the listener position Lc to audio source position Oc

If at least one of these conditions is not fulfilled – this voxel shall be neglected, the next voxel shall be selected from the neighborhood matrix N(L) and this check again shall be repeated.

- Check whether the DLUT data set contains the “primary” diffraction data Dprimary' for the selected neighboring voxel Lvox'. If the data exists - retrieve the corresponding corner voxel Cvox' value.

  - Get the “secondary” diffracted source azimuth angle azD' from the neighboring voxel Lvox' and the corresponding corner voxel Cvox' as:

| azD'=arctan Cvoxy' - Lvoxy'Cvoxx' - Lvoxx' | (228) |
| --- | --- |

- Set the “secondary” virtual diffraction source position Dc', if the value of the diffraction azimuth angle difference is bigger than the corresponding threshold value h:

| Dc'=sph2cart(azD', elD, rdirect)+Lc, if azD-azD'≥h | (229) |
| --- | --- |

If the condition is not fulfilled (i.e., azD-azD'<h) – this voxel shall be neglected, the next voxel shall be selected from the neighborhood matrix N(L) and the processing shall be repeated.

Figure 51 — Examples of the case (A) where voxels Lvox and Cvox are not adjacent to each other 
(green sectors depict the azimuth difference between primary Dprimary and secondary Dsecondary diffraction directions defined by azimuth angles azD and azD').

The threshold value h determines how much the “secondary” diffraction direction azD' must differ from the “primary” one azD to be considered for the audio rendering. The threshold value h shall be set to 30 degrees:

| h=π6 | (230) |
| --- | --- |

- Get the “primary” gprimary and “secondary” gsecondary gains for the diffraction audio sources using the following relationship (for distributing the relative energy between them):

| gsecondary=1-gprimary | (231) |
| --- | --- |

The following Gaussian gain weight matrix G3x3 applied to the “primary” and “secondary” elements (to ensure total diffraction gain preservation):

| G3x3×L3x3'=gdiffracted | (232) |
| --- | --- |

where operation × denotes element-by-element multiplication and

| G3x3=116121242121 | (233) |
| --- | --- |
| L3x3'=IsSelected(L7')IsSelected(L3')IsSelected(L5')IsSelected(L2')1IsSelected(L1')IsSelected(L8')IsSelected(L4')IsSelected(L6') | (234) |

where IsSelected(Li')=1 - if the index i corresponds to the selected neighboring voxel Lvox' and IsSelected(Li')=0 - otherwiese.

Using these two conditions the “primary” gprimary and “secondary” gsecondary gains shall be obtained (depending on wither the neighbouring voxel Lvox' corresponds to the upright or diagonal position in respect to the listener voxel Lvox):

| gprimary=23gdiffracted, gsecondary=13gdiffracted, if Lvox-Lvox'<1.5 | (235) |
| --- | --- |
| gprimary=45gdiffracted, gsecondary=15gdiffracted, if Lvox-Lvox'≥1.5 | (236) |

where the distance Lvox-Lvox'  shall be expressed in the number of the voxel size Vsize unites.

The diffracted source attenuation gains gprimary and gsecondary for the “primary” Dc and “secondary” Dc' virtual diffraction sources shall be combined with the corresponding gains of other audio rendering stages.

“Single-voxel” method

If the listener voxel Lvox belongs to the neighborhood voxel set N(K) of the single-voxel occluding element Kvox and the listener voxel Lvox is adjacent to the corresponding corner voxel Cvox, the following processing steps of the “single-voxel” method (B) for calculation of function SecondaryDiffractionCalculation() shall be performed:

  - Get the “secondary” diffracted source azimuth azD*:

| azD*=azD+signazD–azKπ2 | (237) |
| --- | --- |

where

| azK=arctan Kvoxy - Lvoxy'Kvoxx - Lvoxx' | (238) |
| --- | --- |

The difference between the “primary” and “secondary” diffraction directions shall be always equal to π/2, since the vector azD* shall be determined by the 90 degree rotation of the corresponding “primary” diffraction audio source azimuth victor azD (azimuth angle between Lvox and Cvox) around the vector azK (azimuth angle between Lvox and Kvox) pointing to the single-voxel occluding element Kvox from the listener voxel Lvox.

  - Set the “secondary” virtual diffraction source position Dc':

| Dc'=sph2cart(azD*,elD, rdirect)+Lc | (239) |
| --- | --- |

Figure 52 — Examples of the case (B) where all voxels Kvox and Cvox are adjacent to each other.

The “secondary” virtual diffraction source shall be always obtained for the “single-voxel” method (B).

- Get the energy gains for the “primary” gprimary and “secondary” gsecondary diffraction audio sources:

| gprimary=23gdiffracted, gsecondary= 13gdiffracted | (240) |
| --- | --- |

The diffracted source attenuation gains gprimary and gsecondary for the “primary” Dc  and “secondary” Dc' virtual diffraction sources shall be combined with the corresponding gains of other audio rendering stages.

### Multi-Path voxel-based diffraction with RasterMaps

#### General

This section describes an alternative to the diffraction modelling approach specified in 6.6.11.3.2.

This method is only applicable to static voxel-based scenes.

Static voxel-based scenes are defined as the ones where voxel geometry representation and audio source positions remain unchanged until the end of the rendering process. Dynamic voxel-based scenes are defined as the ones where voxel geometry or audio source positions can be changed during the rendering process.

The Multi-Path Diffraction with RasterMaps method uses diffraction metadata obtained from RasterMap data stored in the bitstream element payloadRasterMap(). It is activated by the presence of the bitstream element payloadRasterMap() and stays active until the end of the rendering process.

The following scenarios are not supported by the  multi-path voxel-based diffraction rendering:

- voxSceneUpdates() is present or

- voxSceneInstructionsForDiffraction() is present or

- scene geometry metadata VoxDataMatrix is changed during the rendering process or

- any audio source position is changed during the rendering process.

The following rendering stages can be used in addition for modelling further acoustic effects for static voxel-based scenes:

- direct occlusion modelling (6.6.11.3.1)

- early reflection modelling (6.6.13)

#### Data elements and variables

VoxDataMatrix	Voxel scene data 3D matrix

B		Voxel scene bounding box

VoxDataDiffractionMap	Diffraction map 2D matrix

Lc		Listener position coordinates

Lf		Listener position in continuous voxel grid domain having floating point components, where Lvoxx=round(Lfx), Lvoxy=round(Lfy), and Lvoxz=round(Lfz) for Lf=(Lfx, Lfy,Lfz)

Lvox		Listener voxel (enclosing Lc) grid indices,
where Lvox=(Lvoxx, Lvoxy,Lvoxz)

Lvox'		Listener adjacent (neighbouring) voxel Lvox'∈N(L) grid indices

Oc		Audio source position coordinates

Ovox	Audio source voxel (enclosing Oc) grid indices,
where Ovox=(Ovoxx, Ovoxy,Ovoxz)

Sc		Audio source reference point on the voxel scene bounding box B

Svox		Audio source reference voxel (enclosing Sc) grid indices,
where Svox=(Svoxx, Svoxy,Svoxz)

Vsize	Voxel size scalar

Vstart	Global coordinates of the first corner point defining the voxel scene,
where Vstart = voxScenePosS

Vend	Global coordinates of the second corner point defining the voxel scene,
where Vend = voxScenePosE

Vdim	Number of voxels for each voxel scene dimension,
where Vdim = voxSceneDimensions

Dprimary	Primary diffraction audio source data for Lvox

Dprimary'	Primary diffraction audio source data for Lvox'

Dsecondary	Secondary diffraction audio source data for Lvox

Dc		Primary diffraction source position coordinates

Dc'		Secondary diffraction source position coordinates

rdiffracted	Estimation for the total diffraction path length between listener Lc and audio source Oc positions

rpath	Diffraction path length between the listener voxel Lvox and audio source reference voxel Svox

rpath_continous	Continuous diffraction path length between the listener voxel Lc and static audio source reference voxel Svox

Wvox	Waypoint for multi-layer diffraction path between the listener voxel Lvox and audio source reference voxel Svox

dl		Euclidian point-line distance for layer l (in voxel units) from the continuous listener position Lf  to the initial segment of the split path section

#### Stage description

##### Diffraction map

The diffraction modeling is performed using the static diffraction map matrix P (VoxDataDiffractionMap) obtained as described in 6.3.2.1.3.

##### Retrieval of diffraction path information from RasterMap data

The multi-layer RasterMap data is stored in the structure rasterMap[s][l]. This data is used to obtain the diffraction path information {Wvox, rpath, l} for the layer l ∈ {0, 1}. The first layer (l=0) represents the shortest diffraction path; whereas the second layer (l=1) represents an additional diffraction path.

The diffraction path information {Wvox, rpath, l}  is retrieved by iterating over all elements of rasterMap[s][l] as described by the following pseudo code which sets the diffraction path information path_info:

typedef array<int, 3> RasterMapCoord;

typedef array<int, 2> VoxelCoord;

WaypointMatrix waypointMatrix;

// initialize key-value store for RasterMap coordinates

for (int l = 0; l < numRasterMapLayers[s]; l++) {

for (int v = 0; v < numRasterMapVoxels; v++) {

int vcX = rasterMap[s][l].voxelCoordX[v];

int vcY = rasterMap[s][l].voxelCoordY[v];

int vcL = l;

int wpX = rasterMap[s][l].waypointX[v];

int wpY = rasterMap[s][l].waypointY[v];

int wpL = rasterMap[s][l].waypointL[v];

waypointMatrix.at(vcX, vcY, vcL) = { wpX, wpY, wpL };

}

}

// extract diffractions paths

for (int l = 0; l < numRasterMapLayers[s]; l++) {

for (int v = 0; v < numRasterMapVoxels; v++) {

int vcX = rasterMap[s][l].voxelCoordX[v];

int vcY = rasterMap[s][l].voxelCoordY[v];

int vcL = l;

RasterMapCoord vc = { vcX, vcY, vcL };

vector<RasterMapCoord> path = waypointMatrix.getPath(vc);

RasterMapCoord vcSrc = path.back();

RasterMapCoord vcCorner = path.front();

// convert zero-indexed values to one-indexed voxel coordinates

VoxelCoord L_vox = { vcX + 1, vcY + 1 };

VoxelCoord S_vox = { vcSrc[0] + 1, vcSrc[1] + 1 };

VoxelCoord C_vox = { vcCorner[0] + 1, vcCorner[1] + 1 };

// compute path length

float r_path = waypointMatrix.getPathLength(vc, path);

// RasterMap layer index

int layer = vc[2];

int corner_layer = vcCorner[2];

// add path to cache

path_info.at(L_vox, S_vox, layer) = { C_vox, r_path, corner_layer };

}

}

where s denotes the RasterMap  source index; path_info denotes the diffraction path information of a given voxel sub-scene and where the functions waypointMatrix.getPath(vc) and waypointMatrix.getPathLength(vc, path) are defined as:

vector<RasterMapCoord> WaypointMatrix::getPath(RasterMapCoord pos)

{

vector<RasterMapCoord> path;

RasterMapCoord wp = at(pos);

while (pos != wp) {

pos = wp;

wp = at(wp);

path.push_back(pos);

}

return path;

}

float WaypointMatrix::getPathLength(RasterMapCoord pos, vector<RasterMapCoord>& path)

{

float length = 0.f;

for (const RasterMapCoord& wp : path) {

float delta_x = float(wp[0] - pos[0]);

float delta_y = float(wp[1] - pos[1]);

length += sqrtf(delta_x * delta_x + delta_y * delta_y);

pos = wp;

}

return length;

}

##### Estimation of the diffraction source positions

The following processing steps are executed for each listener position coordinates Lc update:

  - Get the diffracted source azimuth azD from listener voxel Lvox and selected voxel Wvox:

| azD=arctanWvoxy - LvoxyWvoxx - Lvoxx | (241) |
| --- | --- |

  - Get the diffracted source elevation elD from the listener voxel Lvox and source reference voxel Svox:

| elD=arctanSvoxz - LvoxzSvoxx - Lvoxx2+Svoxy - Lvoxy2 | (242) |
| --- | --- |

  - Get the direct audio source distance r from the listener Lc and audio source Oc positions as:

| rdirect = ¦¦Oc – Lc¦¦ | (243) |
| --- | --- |

  - Set the virtual diffraction source position coordinates Dc based on the listener position Lcto correspond to the diffracted source azimuth azD, elevation elD and direct distance rdirect:

| Dc = sph2cart(azD, elD, rdirect) + Lc | (244) |
| --- | --- |

##### Estimation of the diffraction source gains

The following processing steps are executed for each listener position coordinates Lc update:

- Get listener voxel Lvox indices for the listener position coordinates Lc using Cartesian coordinate Xc to voxel grid indices Xvox conversion equation (195), for X={L,S,O}:

| Xvox = ceil Xc – VstartVsize+12 | (245) |
| --- | --- |

where:

| Vsize =Vendi*-Vstarti*Vdimi*, i*: Vdim[i*] =maxi(Vdim[i]) | (246) |
| --- | --- |

- Get continuous listener position Lf as:

| Lf = Lc – VstartVsize+1 | (247) |
| --- | --- |

The continuous listener position Lf is used to compute the continuous path length rpath_continous:

| rpath_continous=maxlength_on_grid + rgrid_offsetVsize, r_direct | (248) |
| --- | --- |

where

| rgrid_offset =Wvoxx - Lfx2+Wvoxy - Lfy2-Wvoxx - Lvoxx2+Wvoxy - Lvoxy2 | (249) |
| --- | --- |

  - Get the diffracted source radius rdiffracted from the listener position coordinates Lc, source reference voxel Svox, the continuous path length rpath_continous and corresponding distance routside:

| rdiffracted=rpath_continous2+Scz - Lcz2+routside | (250) |
| --- | --- |

  - Get the diffracted source attenuation gain gdiffracted modelling the total diffracted path effect as:

| gdiffracted_path=tdiffraction⋅DiffractionFilterCoeffsαdiffraction_idx | (251) |
| --- | --- |

where

| αdiffraction_idx=180-ceilαdiffraction_angle | (252) |  |
| --- | --- | --- |
| αdiffraction_angle=2 rad2degarcsinrdirect/rdiffracted | (253) |

##### Crossfading of multi-path diffraction sources

If two diffraction paths are rendered for a sound object, then individual layer gains glayer, l are computed. For computing these layer gains, the diffraction paths are determined for both layers by back tracing the points. If all paths are complete, i.e. can be back-traced to Svox, then the split path sections of these paths are determined by reverting the paths and removing those segments which are common for all paths.

The following algorithm is applied to check, if a reversed path denoted as seq, i.e. first element is the listener voxel coordinate, with tail_idx being the split point of the diffraction paths is convex:

typedef array<int, 2> VoxelCoord;

bool path_is_convex(VoxelCoord seq[], int tail_idx)

{

if (tail_idx < 3) {

return true;

}

int dx1 = seq[1][0] - seq[2][0];

int dy1 = seq[1][1] - seq[2][1];

int dx2 = seq[0][0] - seq[1][0];

int dy2 = seq[0][1] - seq[1][1];

int ref_val = dy1 * dx2 - dx1 * dy2;

int ref_sign = (ref_val > 0) - (ref_val < 0);

for (int n = 3; n <= tail_idx; n++) {

dx1 = seq[n - 1][0] - seq[n][0];

dy1 = seq[n - 1][1] - seq[n][1];

dx2 = seq[n - 2][0] - seq[n - 1][0];

dy2 = seq[n - 2][1] - seq[n - 1][1];

int test_val = dy1 * dx2 - dx1 * dy2;

int test_sign = (test_val > 0) - (test_val < 0);

if ((test_sign != 0) && (test_sign != ref_sign)) {

return false;

}

}

return true;

}

If all split path sections are convex, the Euclidean point-line distance dl (in voxel units) from the continuous listener position Lf  to the initial segments of the split path sections are determined as illustrated in Figure 53. The distances dl yield the unnormalized layer gains  g'layer, l:

| d'l=max⁡(dl, 1) | (254) |
| --- | --- |
| g'layer, l=kd'kd'l | (255) |

where

| g'layer,0=d'1, g'layer,1=d'0 | (256) |
| --- | --- |

If not all split path sections are convex, the following unnormalized layer gains  g'layer, l are used:

| g'layer, l=limit(1-(rpath,l- rpath,0)/2, [0,1]) | (257) |
| --- | --- |

where limit(y, [ymin, ymax]) denotes a limitation to the range [ymin, ymax].

| g'layer, 0=1, g'layer, 1≤1 | (258) |
| --- | --- |

Figure 53 - Distances d0 and d1 to the initial segments of the split path sections

The final layer gains glayer, l are computed by applying the following normalization that corresponds approximately to a 4.5dB level summing law:

| γ=4/3 | (259) |
| --- | --- |
| Pl=g'layer, lrdirectrpath,lγ | (260) |

where:

| P0=g'layer, 0rdirectrpath,0γ, P1=g'layer, 1rdirectrpath,1γ | (261) |
| --- | --- |

The final layer gains glayer, l are computed as

| glayer, l=g'layer, lP0kPk1γ | (262) |
| --- | --- |

where:

| glayer, 0=g'layer, 0P0P0+P11γ, glayer, 1=g'layer, 1P0P0+P11γ | (263) |
| --- | --- |

### Voxel-based early reflections

#### General

#### The voxel-based early reflection stage provides acoustically relevant reflection information with respect to the audio source(s) position(s), listener position and reflective geometry description. This stage utilizes the scene voxel geometry data to compute first order reflections from reflecting voxel elements. The segment traversing algorithm [16] on the 3D voxel grid (used for voxel-based occlusion modeling) shall be used for checking the visibility of potential propagation paths from the sound sources to the listener. The Voxel-based Early Reflection stage uses reflection pathlength and material reflection coefficients to model the corresponding reflected sound attenuation. This stage doesn’t need any pre-computed data transmitted from the encoder. This allows to render early reflections in real time in dynamically updatable and complex geometric environments.

The voxel-based early reflections stage shall be activated if VOXDATA_PAYLOAD is present, EARLY_REFLECTION_PAYLOAD is not present and flag useLowComplexityER in the mpegSceneConfig() is set to false in the bitstream.

#### Data elements and variables

VOX	Voxel scene data 3D matrix

Lc	Listener position coordinates

Oc	Audio source position coordinates

NS	Maximum number of the audio sources for the reflection modelling

SER, SERi	Uniform angular sectors for the reflection modelling

Ssize	Sector angular size

NR	Number of the ray directions for the volumetric ray marching algorithm

RER, RERi	Set of the ray directions for the volumetric ray marching algorithm

NP	Number of the audio sources representing reflections

PER, PERi	Set of the ray directions for the volumetric ray marching algorithm

Chit	Set of the ray-voxel collision voxels

Chit*	Set of the “air”-type voxel prior the first ray-voxel collision “material”-type voxel Chit on the line (PER,Chit)

CER'	Sub-set of the ray-voxel collision voxels CER'⊆Chit corresponding to the considered reflection trajectory path lengths

CER	Sub-set of the ray-voxel collision voxels CER⊆CER' corresponding to the un-occluded reflection trajectories

rEROC 	Reflection trajectory length component for the segment Oc,Chit

rER	Reflection trajectory length component for the segment Chit,Lc

azER, elER, rER	Spherical coordinates of all virtual image sources contributing to the reflection modelling

gER	Reflection attenuation gains for all image sources contributing to the reflection modelling

gERDistanceAttenuation	Frequency dependent distance attenuation for the segment Oc,CER

gERReflectionAttenuation	Frequency dependent reflection coefficients

DER 	Rendered audio sources for the reflection modelling

azERk, elERk, rERk	Spherical coordinates of the rendered audio sources DER

gERk	Reflection attenuation gains for the rendered audio sources DER

fER	Early reflection gain adjustment function for indoor/outdoor environment types

fIndoor	The indoor environment type characterization function

gERIndoor_tuning	Indoor environment type early reflection tuning gain obtained from voxReflectionEarlyTuningIndoorGainDb

gEROutdoor_tuning	Outdoor environment type early reflection tuning gain obtained from voxReflectionEarlyTuningOutdoorGainDb

Nhits	Number of early reflection ray hits

Nrays	Number of all considered early reflection rays

dERSectorDownmixFlag	Early reflection sector downmix mode flag

#### Stage description

##### Initialization of acoustic reflection number and ray direction pattern

The following processing steps shall be executed at the (sub-)scene initialization stage to prepare for the dynamic early reflection modelling.

- Set the number of the clusters NS of clusters for the modelled early reflection incoming directions. The variable NS∈2, 32 corresponds to the audio renderer configuration parameter VoxelEarlyReflectionMaxNumReflections. It is obtained from the bitstream element voxReflectionNumClusters. The value of NS=16 shall be used as the default parameter (for voxReflectionNumClustersPresentFlag == 0). It corresponds to the maximum number of virtual audio sources representing acoustically relevant early reflections.

- Set the uniform angular sector clusters SER as:

| SER=SERi=12Ssizei-1, 12Ssizei, i=1,…,NS | (264) |
| --- | --- |

Figure 54 — Uniform angular sectors (clusters) for early reflection incoming directions.

where the angular sector size Ssize defined as:

| Ssize=2πNS | (265) |
| --- | --- |

- Set the number of the ray directions NR of the ray directions for the volumetric ray marching algorithm. The variable NR=26  corresponds to the audio renderer configuration parameter VoxelEarlyReflectionDirections.

- Set the considered ray directions RER for the volumetric ray marching algorithm as:

| RER=RERi∈RER1=x-1,y-1,z-1, …,RERNR=x+1,y+1,z+1 |  | (266) |
| --- | --- | --- |

Figure 55 — All possible 26 ray directions Rseti=1…NR on the 3D voxel grid (left) 
and axis aligned projection view (right).

##### Processing steps at the listener or audio source updates

The following processing steps shall be executed for each listener position Lc and/or specified audio source position Oc updates.

- If the information for the early reflections modelling (i.e., DER and gER) for the current voxel scene state (i.e., Lvox, Oc and VOX) is available, it shall be applied. Otherwise, it shall be calculated.

- Set the number of the origin points NP for the volumetric ray marching algorithm. The variable NP∈2, 32 corresponds to the audio renderer configuration parameter VoxelEarlyReflectionNumPoints. It is obtained from the bitstream element voxReflectionNumOrigins. The value of NP =3 shall be used as the default parameter (for voxReflectionNumOriginsPresentFlag == 0).

- Get the origin points PER on the line segment Oc,Lc connecting the audio source Oc and listener Lc positions to run the volumetric ray marching algorithm:

| PER=PERj=Oc-j-1Oc-Lc(NP-1)∈B, j=1,…,NP | (267) |
| --- | --- |

The set of points PER divides the segment Oc,Lc into NP-1 equal parts. The set PER includes only points located inside of the voxel scene bounding box B defined by equation エラー! 参照元が見つかりません。.

Figure 56 — Example of reflecting voxel-based geometry (left) and origin points for the volumetric ray marching algorithm (right).

- Get the ray-voxel collision set of voxels Chit for each considered ray and origin point RERPER as:

| Chit=FirstRayVoxelCollisionRERi(PERj)∩VOX | (268) |
| --- | --- |

where i=1,…,NR and j=1,…,NP.

The voxel-based audio scene geometry representation VOX contains all acoustically relevant reflecting geometry elements associated with the corresponding material reflection coefficients.

Figure 57 — Example of the considered ray directions for the ray marching algorithm (left) and ray marching algorithm outcome of one origin point (right).

- Get the trajectory "Oc-Chit-Lc" path length components rEROC and rER as:

| rEROC=CER-Oc | (269) |
| --- | --- |
| rER=CER-Lc | (270) |

- Get the voxel sub-set CER'⊆Chit corresponding to the considered reflection trajectory path lengths:

| Chit∈CER' if rER+rEROC∈lmin,lmax | (271) |
| --- | --- |

Discard all trajectory candidate "Oc-Chit-Lc" from consideration, which results in too short or too long trajectory paths:

| Chit∉CER' if rER+rEROC∉lmin,lmax | (272) |
| --- | --- |

where the variables lmin and lmax are the audio renderer configuration parameters VoxelEarlyReflectionMinDelayMicroSeconds, VoxelEarlyReflectionMaxDelayMicroSeconds.

- Get the “reflecting” voxel sub-set CER⊆CER' corresponding to the un-occluded reflection trajectories "Oc-Chit-Lc" by checking the “direct visibility” condition for the following two line-segments:

| CER'∈CER if Oc-Chit*∪Chit*-Lc∩VOX=∅ | (273) |
| --- | --- |

where Chit* is the “air”-type voxel prior the first ray-voxel collision “material”-type voxel Chit on the line (PER,Chit).

The trajectory candidate "Oc-Chit-Lc" shall be considered for early reflection modelling if both segments go only through the sound propagation “air”-type voxel(s). If at least one of two following line-segments are blocked (or separated) by any “material”-type occluding voxel(s), then the trajectory "Oc-Chit-Lc"  and position Chit shall be discarded from the early reflection modelling:

| CER'∉CER if Oc-Chit*∪Chit*-Lc∩VOX≠∅ | (274) |
| --- | --- |

Figure 58 — Example of the positive (left) and negative “direct visibility” condition check (right).

- Get the spherical coordinates of all virtual image sources contributing to the reflection modelling:

| azER=arctan CERy- LcyCERx- Lcx | (275) |
| --- | --- |
| elER=arctanCERz - LczCERx - Lcx2+CERy - Lcy2 | (276) |

- Get the corresponding reflection attenuation gains for all image sources as:

| gER=gERDistanceAttenuationgERReflectionAttenuation | (277) |
| --- | --- |

where the frequency dependent values gERDistanceAttenuation∈0,1 accounts for the distance attenuation between the audio source Oc and reflecting voxel element CER positions and the values gERReflectionAttenuation∈0,1 accounts for reflection energy absorption obtained from the material reflection coefficients:

| gERDistanceAttenuation =DistanceAttenuationrEROC | (278) |
| --- | --- |
| gERReflectionAttenuation=ReflectionAttenuationVOX(CER) | (279) |

where the value VOX(CER) corresponds to the material reflection property of the voxel CER.

- Get the coordinates of the audio sources DER and the reflection attenuation gains gERk as:

| DER=DERk=sph2cart(azERk, elERk, rERk)+Lc, k=1,…,NS | (280) |
| --- | --- |

where the spherical coordinates shall be obtained by image sources azimuth clustering and averaging:

| azERk=1nki=1nkazER | (281) |
| --- | --- |
| elERk=1nki=1nkelER | (282) |
| rERk =1nki=1nkrER | (283) |
| gERk =gEREIF_tuning gERvox_mesh_adjfERNS1nki=1nkgER | (284) |

where azER∈SERk and the values nk correspond to the number of all image sources contributing to the considered early reflection sector SERk.

The gain gEREIF_tuning is the reflection level tuning parameter (obtained from earlyTuningGain). The gain gERvox_mesh_adjis the reflection level alignment parameter (to align reflection levels of voxel and mesh reflection modelling modes).

##### Early reflection gain adjustment to environment type

The following process of adjustment to environment type shall be applied to obtain the early reflection gain adjustment function fER.

- Set the tuning gain gERIndoor_tuning for indoor environments. The variable gERIndoor_tuning corresponds to the audio renderer configuration parameter VoxelEarlyReflectionGainIndoor. It is obtained from the bitstream element voxReflectionEarlyTuningIndoorGainDb.

- Set the tuning gain gEROutdoor_tuning for outdoor environments. The variable gEROutdoor_tuning corresponds to the audio renderer configuration parameter VoxelEarlyReflectionGainOutdoor. It is obtained from the bitstream element voxReflectionEarlyTuningOutdoorGainDb.

If the maximum of the indoor and outdoor environment gains max(gERIndoor_tuning ,gEROutdoor_tuning) is greater than the voxel early reflection rendering stage activation threshold (-100dB), the early reflection gain adjustment function fER is obtained as:

| fER=gERIndoor_tuning fIndoor+gEROutdoor_tuning(1-fIndoor) | (285) |
| --- | --- |

where the indoor environment characterization function fIndoor is obtained by the ratio between the number of early reflection ray hits Nhits and the number of all considered rays Nrays:

| fIndoor=NhitsNrays | (286) |
| --- | --- |
| Nrays=NRNP | (287) |
| Nhits=numberChit | (288) |

otherwise

| fER=0 | (289) |
| --- | --- |

##### Audio rendering of obtained early reflections

- Set the early reflection sector downmix mode flag dERSectorDownmixFlag. The variable dERSectorDownmixFlag corresponds to the audio renderer configuration parameter VoxelEarlyReflectionSectorDownmixEnabled. It is obtained from the bitstream element voxReflectionSectorDownmixFlag.

All calculated early reflections DERo for all audio sources shall be rendered as individual render items:

| DERk=DERo, gERk=gERo, if dERSectorDownmixFlag==false | (290) |
| --- | --- |

Otherwise, as downmixed render items:

| DERk=mean(DERo), gERk =mean(gERo), if dERSectorDownmixFlag==true | (291) |
| --- | --- |

where DERo∈SERk are all early reflections DERo for all audio sources belonging to the cluster SERk.

The audio rendering of all obtained audio sources DERk shall be performed using the attenuation gains gERk applied to the corresponding weighted early reflection downmix signal(s).

### Metadata culling

#### General

In the Metadata Culling stage, RIs that become inaudible due to very low gain or EQ (e.g. because of a strong distance attenuation or occlusion) are deactivated to save computations that would otherwise occur in subsequent stages. The audio signal itself is not analysed in this stage.

In addition, Reflection RIs that become perceived as a part of their parent Primary RIs due to the precedence effect can be also deactivated and culled. This does not apply to RIs of type ListenerVoice since the precedence effect can be prevented by tuning the level of its direct sound down or turning it completely off.

Furthermore, the calculation and application of the directivity filter gains shall be skipped in the Directivity stage (as described in 6.6.16) if the corresponding RIs are fulfilling culling conditions in the Metadata Culling stage.

#### Data elements and variables

gainCullingThreshold 	minimum per-band gain value to keep RI activated, configured from bitstream parameter to a value between -100 dB and -30 dB.

enableCullingReflectionRI	A boolean flag that enables or disables the Reflection RI culling.

timeDiff	time difference between Primary RI and Reflection RI at the listener’s position in ms.

gainThresh	gain threshold for Reflection RI culling in dB.

Ggr		global gain of Reflection RI in linear scale

Geqr	averaged EQ gain of Reflection RI in linear scale.

Gd		dispersion filter gain in linear scale.

Ggp		global gain of Primary RI in linear scale

Geqp	averaged EQ gain of Primary RI in linear scale

gainDiff	gain difference between Primary RI and Reflection RI at the listener’s position in dB

#### Stage description

##### Run-time configuration of culling parameter

It is possible to override the culling parameters gainCullingThreshold and loudnessCullingThreshold from a local renderer configuration parameter. If the local configuration parameter value is not set, the bitstream value is used. Further, sources shall be excluded from processing if the corresponding attribute is set (i.e., objectSourceNoCulling, channelSourceNoCulling, hoaSourceNoCulling).

##### Gain-based culling of RIs

Only RI types Primary, Representative, Reflection, Portal, HOA, Diffraction are considered.

Only RIs without extent are considered.

For each of those RIs, the EQ band with the maximum amplification value is found. This value is multiplied by the overall gain value of the RI. If the resulting value is below gainCullingThreshold, the RI is deactivated. When the value exceeds the threshold again, the RI is re-activated.

##### Loudness-based culling of RIs

In addition to the gain-based culling mechanism described in 6.6.14.3.2 a loudness-based culling mechanism is available. With respect to processed RIs, the same restrictions as in 6.6.14.3.2 apply.

###### Loudness interface

The renderer shall provide an interface to accept generic loudness data (broadband) per audio stream from an external source (e.g., MPEG-H metadata). The exact nature of the loudness data (e.g., measurement method, system) is out of scope of this specification.

service loudness_interface {

i32 update(1: i32 audio_stream_id, 2: double loudness_db),

}

Where:

- audio_stream_id: Unique identifier of the audio stream, i.e., audioStreamId.

- loudness_db: Loudness value in dB (e.g., derived from MPEG-D/H metadata). The loudness attribute of the corresponding source(s) shall be updated accordingly. The loudness value shall remain valid until another update is received.

If no loudness values are available, the render-item shall be excluded from loudness-based processing.

##### Processing

The culling mechanisms described in 6.6.14.3.2 and 6.6.14.3.3 shall be combined in a two-step process as follows:

// look up table for item gain calculation
std::map<int, float> itemGain;

// first loop, calculate item gains and max item gain (incl. loudness factor)

{

float maxItemGain = 1.0e-10f;

for (auto& item : renderList.items) {

// only HOA, portal, primary and directly derived render items are processed

if (item->type != Primary || Representative || Reflection || Portal

|| Diffraction)

continue;

// do not process extended items

if (item->extent)

continue;

// do not process orphaned items

if (item->status == Orphaned)

continue;

// do not process items with “no culling” attribute

if (item->aparams & NoCulling)

continue;

// calculate minimal attenuation of gain and EQ combined

float maxBandGain = *std::max_element(item->eq.begin(), item->eq.end());

itemGain[item->idx] = maxBandGain * item->gain;

// skip maxItemGain calculation if no loudness information is available

if (!item->loudness.has_value())

continue;

If (item->status != ItemStatus::Inactive && 
       itemGain[item->idx] * item->loudness.value() > maxItemGain)

maxItemGain = itemGain[item->idx] * item->loudness.value()

}

// second loop, culling

for (auto& item : renderList.items) {

// only HOA, portal, primary and directly derived render items are processed

if (item->type != Primary || Representative || Reflection || Portal

|| Diffraction)

continue;

// do not process extended items

if (item->extent)

continue;

// do not process orphaned items

if (item->status == Orphaned)

continue;

// do not process items with "no culling" attribute

if (item->aparams & NoCulling)

continue;

// get item status
  auto& deactivate = item->getStateOrCreate<ItemState>().deactivate;

bool freshActivation = false;

// only perform deactivation check if item status has changed
  if (item->changed & (Gain | Loudness | Eq | Status)) {

// if attenuation is below absolute threshold (default -60dB), or

// attenuation is below relative threshold, deactivate the source

if (item->status != Inactive && (itemGain[item->idx] < DeactivationThreshold

|| (item->loudness.has_value()

&& itemGain[item->idx] * item->loudness.value() / maxItemGain <

DeactivationThresholdRel))) {

deactivate = true;

} else {

if (deactivate) {

freshActivation = true;

}

deactivate = false;

}

}

if (deactivate) {

// deactivate item

itemStore->deactivate(item.get());

} else if (freshActivation) {

// explicitly reactivate item

item->setStatus(ItemStatus::Active, true);

}

}

##### Culling of Early Reflections

For the mesh-based ER RIs, which are created through Sec. 6.6.6, Reflection RI culling process is applied as described below.

Following to the above deactivation process, Reflection RI culling process is performed, if enableCullingReflectionRI is true. However, this process shall be skipped if Voxel ER (Sec. 6.6.13) is used. The Reflection RI culling process is performed as follows. For each of those RIs, if their RI type is Reflection, and it has a parent Primary RI other than type ListenerVoice, and the parent Primary RI is without extent, time difference and sound volume ratio (gain difference in dB) between the RI and its parent Primary (Direct) RI at the listener’s position are calculated. For each of the calculated pairs of time difference and sound volume ratio, the calculated time difference is converted to a corresponding gain threshold, and the calculated sound volume ratio is compared with the gain threshold. If the calculated sound volume ratio is below the gain threshold, such Reflection RI is deactivated. This process is performed in each update cycle.

Time difference at the listener’s position in ms is calculated by the following equation. The distDiff is a difference between distances from Direct RI’s position and Reflection RI’s position to the listener’s position in meter, which are calculated using those positions (coordinates of the points).

| timeDiff=1000.0*distDiff/speedOfSound | (292) |
| --- | --- |

Once the timeDiff is calculated, it is converted to a gain threshold as follows.

| gainThresh = -0.10*timeDifff-17.5 , if 1.0 ms<timeDiff<10.0 ms-0.55*timeDifff-13.0 , if 10.0 ms≤timeDiff<20.0 ms-0.60*timeDiff-12.0 if 20.0 ms≤timeDiff<30.0 ms-0.80*timeDiff-6.0 ,if 30.0 ms≤timeDiff≤40.0 ms | (293) |
| --- | --- |

For the cases where timeDiff is equal to or less than 1 ms and larger than 40 ms, gainThresh is not calculated and this culling is not performed. This is because the precedence effect does not occur or negligible when the time difference is outside of the range between 1 ms and 40 ms typically.

Sound volume ratio (gain difference in dB) between Reflection RI and Direct RI at the listener’s position, gainDiff, is calculated using each global gain and averaged EQ gain of the RIs. The global and EQ gains are read from the “gain” and “eq” fields of RI metadata (c.f. Table 140 — RI metadata fields) of those RIs. Therefore, without any calculation, those gains are already available at this stage. Calculation of the averaged EQ gain is performed in A-weighted domain. This is done by multiplying each EQ value by a corresponding A-weighting coefficient. The A-weighting coefficients can be found in the international standard IEC 61672-1:2013 and various national standards relating to the sound pressure level measurement. They are shown inTable 143. It should be noted that gAccessibility is applied to the global gain as described in Section 6.6.6.3.8 when erAttenuationDb is provided via the Accessibility User Interface described in Section B.5.

Table 143 — A-weighting coefficients (linear scale, 20Hz – 20kHz, 1/3 Octave band)

| Frequency (Hz) | A-weighting |
| --- | --- |
| 20 | 0.00299 |
| 25 | 0.00582 |
| 31.5 | 0.01072 |
| 40 | 0.01862 |
| 50 | 0.03090 |
| 63 | 0.04898 |
| 80 | 0.07499 |
| 100 | 0.11092 |
| 125 | 0.15668 |
| 160 | 0.21380 |
| 200 | 0.28510 |
| 250 | 0.37154 |
| 315 | 0.46774 |
| 400 | 0.57544 |
| 500 | 0.69183 |
| 630 | 0.80353 |
| 800 | 0.91201 |
| 1000 | 1.00000 |
| 1250 | 1.07152 |
| 1600 | 1.12202 |
| 2000 | 1.14815 |
| 2500 | 1.16145 |
| 3150 | 1.14815 |
| 4000 | 1.12202 |
| 5000 | 1.05925 |
| 6300 | 0.98855 |
| 8000 | 0.88105 |
| 10000 | 0.74989 |
| 12500 | 0.60954 |
| 16000 | 0.46774 |
| 20000 | 0.34277 |

A dispersion filter gain, Gd, is further applied to Reflection RI. If the dispersion filter is applied to Reflection RI in the binaural spatializer, a configured dispersion filter gain, which is described in Section 6.7.1.3.4 Dispersion filter generation and processing, is used as Gd as follows.

| Gd =100.05*dispersionFilterGaindB | (294) |
| --- | --- |

Otherwise, Gd is set to 1.0 in default. The gain difference at the listener’s position is calculated as follows.

| gainDiff =20log10Ggr*Geqr*GdGgp*Geqp | (295) |
| --- | --- |

### Heterogeneous extent

#### General

The Heterogeneous Extent rendering stage takes care of the rendering of so-called spatially-heterogeneous audio elements, which are audio elements that have an extent and a source signal with more than one audio channel. This includes both object sources with more than one source channel and HOA sources with an extent that are specified to be rendered for listening positions both inside and outside the extent. The rendering uses the provided extent to render a plausible representation of the audio element from any listening position, including both width and height information.

When the listener position is inside the extent of an audio element, an interior representation is rendered which is in a listener centric format that is rendered as a sound field that surrounds the listener. When the listener is outside of the extent, an exterior representation is rendered using a source-centric format that is rendered so that it appears to radiate from the extent of the audio element. The extent of the audio element serves as the boundary of a spatial region wherein the interior representation is valid. For listening positions within the extent, the rendering is essentially homogeneous, meaning that the same sound field is rendered as the listener moves around inside the extent. E.g., for the example of an HOA source with an extent, the HOA signal is rendered as a 3DoF HOA signal when the listener is inside the extent but when the listener is outside the extent, an exterior representation derived from the HOA signal is rendered so that it has a localizable position and extent.

Figure 59 - A functional overview of the heterogeneous rendering stage

In the case of HOA sources, the exterior representation is derived from the provided interior representation, while for object sources the interior representation is derived from the provided source signals that describe the exterior representation. A smooth transition is provided between the interior and exterior representations within a transition region around the extent, so that the listener can move in and out of the extent without glitches or unnatural behaviour.

The Heterogeneous Extent rendering stage supports object sources with up to nine channels where the channels represent the spatial information as specified by the input layout parameters. Both user- and object-aligned input layouts are supported. For object sources that lack the spatial information in a dimension, decorrelation is used to synthesize the missing spatial information.

The update function of the heterogeneous rendering stage receives a list of rendering items. For each rendering item that represents an audio element with an extent and has a source signal with more than one channel, a heterogeneousStageItem is created and added to the map heterogeneousStageItems. The heterogeneousStageItem objects keep track of the state of each heterogeneous audio element in the scene. For each heterogeneousStageItem a set of output rendering items are created, which are used to represent the audio element. The original rendering item is deactivated.

Each heterogeneousStageItem has an instance of the class HeterogeneousSource as a member and this is where most of the rendering is done. The function HeterogeneousSource::Update updates the state of each heterogeneous audio element, defines the simplified extent, updates the interior and exterior representations etc.

State data that is updated in the update function and is needed by the audio thread processBlock function is stored in a DSPItem structure, which can be shared from the update thread to the audio processing thread without causing thread problems. In the audio thread call to processBlock the output signal for each rendering item is calculated.

#### Data elements and variables

observationFrontVec		Observation coordinate system front vector

observationUpVec		Observation coordinate system up vector

Ox, Oy, Oz		x, y, z components of observationFrontVec

normalAzimuth		Horizontal angle to the closest normal

closestNormal		The closest outline normal

maxAzimVertex_local		Point on the extent with maximum azimuth angle

minAzimVertex_local		Point on the extent with minimum azimuth angle

maxElevVertex_local		Point on the extent with maximum elevation angle

minElevVertex_local		Point on the extent with minimum elevation angle

horizontalDirectionVec_local	Vector between maxAzimVertex_local and minAzimVertex_local

displacement_left	Distance from the anchor point to the left edge of the simplified extent

displacement_right	Distance from the anchor point to the right edge of the simplified extent

maxAzim		Azimuth angle to maxAzimVertex_local

minAzim		Azimuth angle to minAzimVertex_local

horizontalAngle	Horizontal angle of horizontalDirectionVec_local

min_distance		Distance from the listening position to the anchor point

verticalDirectionVec_local	Vector between maxElevVertex_local and minElevVertex_local

displacement_top	Distance from the anchor point to the top edge of the simplified extent

displacement_bottom	Distance from the anchor point to the bottom edge of the simplified extent

verticalAngle		Horizontal angle of verticalDirectionVec_local

maxElev	Elevation angle to maxElevVertex_local

minElev	Elevation angle to minElevVertex_local

anchorPoint_relative	Anchor point position relative to the listener

leftEdgePos_relative	Left edge position relative to the listener

rightEdgePos_relative	Top edge position relative to the listener

topEdgePos_relative	Top edge position relative to the listener

bottomEdgePos_relative	Bottom edge position relative to the listener

∆D		Change of relative position of the audio element and the listener

D		Distance to the extent when calculating effective spatial extent

L		Geometrical size of the extent when calculating the effective spatial extent

trans_fac[]	Array of transition factors

trans_fac1	Transistor factor controlled by azim_ang

trans_fac2	Transistor factor controlled by elev_ang

azim_ang	Horizontal opening angle divided by 2

elev_ang	Vertical opening angle divided by 2

ang_st	Angle where the transition starts

ang_end	Angle where the transition ends

ptLineAtt	Gain scaler for transitions between virtual loudspeaker setups

ptLineAmp	Gain scaler for transitions between virtual loudspeaker setups

lin2DimAtt	Gain scaler for transitions between virtual loudspeaker setups

lin2DimAmp	Gain scaler for transitions between virtual loudspeaker setups

gainCompPt	Gain compensation for a one virtual loudspeaker setup

gainCompLine	Gain compensation for a three virtual loudspeaker setup

gainComp2D	Gain compensation for a five virtual loudspeaker setup

g0-g4		Gain adjustments for virtual loudspeakers with index 0-4

midSpeakerPos	Position of the middle virtual loudspeakers

leftToListenerDist	Distance from the listening position to the left edge point

rightToListenerDist	Distance from the listening position to the right edge point

distanceToExtent	Distance from the listening position to the anchor point. If the listening position is inside the extent, it is negative.

transitionDistance	Distance from the extent where the exterior/interior transition starts

interiorExteriorWeight	Ratio of the interior and exterior in the transition

extraMargin	Extra margin distance from the extent where only the interior representation is used

rearGain	Gain applied to only the rear hemisphere

distToOutline	Distance from the listening position to the outline of the extent

topHeightDiff	Altitude difference of the listening position and top of the extent

topGain	Gain applied to only the top hemisphere

topTransitionDistance	Distance from the top of the extent where the top attenuation starts

bottomGain	Gain applied to only the bottom hemisphere

bottomHeightDiff	Altitude difference of the listening position and the bottom of the extent

bottomTransitionDistance	Distance from the bottom of the extent where the bottom attenuation starts

sout 		Output signal from a virtual microphone

iFRONT, iLEFT, iBACK, iRIGHT	Signals of the interior representation in quadraphonics format

θ		Horizontal angle of a virtual microphone

φ		Vertical angle of a virtual microphone

layerGainTop	Gain of the top layer of a layered quadraphonics form

layerGainBottom	Gain of the bottom layer of a layered quadraphonics format

layerGainMiddle	Gain of the middle layer of a layered quadraphonics format

micElevOverlap	Controlled cross-feed between the layers in a layered quadraphonics format

sTOP, sMID, sBOTTOM 	Output signal from the top, mid and bottom layers in a layered quadraphonics format

i0-i3 	Signals of the interior representation in FOA format

p	Virtual microphone pattern

C		Distance gain correction

DS		Distance to the source when calculating the distance gain correction

gtarget	Target distance gain

grendered 	Distance gain of rendered source before distance gain correction

gi 	Overall gain of the rendered signal of virtual loudspeaker i at the listener position

gi,processing 	Gain including any gain applied to the virtual loudspeaker in the heterogeneous source processing before calculating the distance gain correction

gi,distance 	Distance gain for virtual loudspeaker i corresponding to the current distance Di between the listener and virtual loudspeaker i

Di		Distance between the listener and virtual loudspeaker i

distanceGainModel	Choice of distance gain model. 0 means volumetric model, 1 means point-source model

hasDiffuseness	Flag signalling if the audio element has a diffuseness parameter

diffuseness	Diffuseness of the audio element

azimRange	The horizontal opening angle of an extent as seen from the listening position

elevRange	The vertical opening angle of an extent as seen from the listening position

numOfAzimDivisions	The number of columns in the grid of rays used for detecting cropping occlusion

numOfElevDivisions	The number of rows in the grid of rays used for detecting cropping occlusion

gridShiftCnt	A counter used for calculating the shift of the positions in the ray grid

gridShift 	The shift of the positions in the ray grid

(azimPos, elevPos)	A 2-dimensional position using the local coordinate system of the simplified extent.

gSA,f		The gain for frequency f, for a certain sub area

gn,f		The gain for frequency f, for a certain point of a sub area

horizRays		The number of columns in the grid of rays used for detecting occlusion within a sub area.

vertRays		The number of rows in the grid of rays used for detecting occlusion within a sub area.

(subAreaAzimPos, subAreaElevPos)	A 2-dimensional position using the local coordinate system of the simplified extent.

eqScaler	A gain scaler that is applied to all occlusion EQs to compensate for the cropping occlusion.

activeExtentRatio	The ratio of the area of the modified, “cropped” extent and the original simplified extent.

f1-f6	The occlusion filters for sub areas 1-6.

fTL, fTR, fM, fBL, and fBR	The occlusion filters for each virtual loudspeaker used to render the heterogeneous source.

FS		A matrix of the occlusion filters fTL, fTR, fM, fBL, and fBR.

FO		A matrix of the occlusion filters f1-f6.

MPOINT, MLINE, MPLANE		Mapping matrices for the 1, 2, and 5 speaker setups.

aPOINT, aLINE, aPLANE		Control parameters that control the weight of mapping matrices for the 1, 2, and 5 speaker setups.

modTransDist	The transition distance modified due to occlusion.

occlLeftAngleScaler	A scaling factor that is used to modify the direction of the virtual microphone capturing the left side of the interior representation due to occlusion.

occlRightAngleScaler	A scaling factor that is used to modify the direction of the virtual microphone capturing the right side of the interior representation due to occlusion.

occlTopAngleScaler	A scaling factor that is used to modify the direction of the virtual microphone capturing the top part of the interior representation due to occlusion.

occlBottomAngleScaler	A scaling factor that is used to modify the direction of the virtual microphone capturing the bottom part of the interior representation due to occlusion.

#### Stage description

##### Finding the description points and defining the simplified extent

The rendering of the exterior representation is based on a simplified extent in the form of a plane that represents the extent as observed from the current listening position. The plane is formed by finding the anchor point of the audio element on the extent, which is defined as the point on the extent that is closest to the listening position. Given the anchor point, the observation vector is defined as the vector from the listening position to the anchor point.

A local coordinate system is defined with the observation vector defining the direction of the front-vector, observationFrontVec. The up-vector, observationUpVec, is then calculated as

| observationUpVec=[-Oxsin(asinOy),cosasinOy, -Ozsin(asinOy)] | (296) |
| --- | --- |

where Ox, Oy and Oz denote the x, y, z components of observationFrontVec.

A special case happens when the listening position is directly above or below the extent, in which case the observation vector would point exactly upwards or downwards, i.e., both Ox and Oz would be zero. To avoid this, an alternative definition is used for the front vector if the y-component of the observation vector exceeds 0.9 or is below -0.9. In those cases, the front vector is instead based on the closest outline normal of the extent, which is described in 6.6.15.3.4, and calculated as

| observationFrontVec=-sinnormalAzimuth100,OyabsOy, -cosnormalAzimuth100 | (297) |
| --- | --- |

where normalAzimuth is calculated as

| normalAzimuth=atanclosestNormalx, closestNormalz. | (298) |
| --- | --- |

The up-vector, observationUpVec, is then calculated as described above.

Using the local coordinate system, all vertices of the extent are evaluated and the points representing the max and min azimuth angles and max and min elevation angles are identified. Together with the anchor point, these are called the description points that the simplified extent is based on.

Figure 60 – The simplified extent is defined as a plane that represents the audio element from a certain listening position. The red dot shows the anchor point and the blue dots show the description points representing the points on the extent with the min/max azimuth and min/max elevation relative to a local coordinate system based on the observation vector, from the listening position to the anchor point.

In the search for the description points representing the edges of the extent, only points with an azimuth angle in the range [-90°, 90°] and elevation [-89°, 89°] are evaluated. As a result, the four description points maxAzimVertex_local, minAzimVertex_local, maxElevVertex_local and minElevVertex_local are identified.

An additional search for points on the extent with the maximum and minimum elevation is performed along all edges that passes through the maxElevVertex_local and minElevVertex_local.

The simplified extent is defined as a plane that is going through the anchor point at a horizontal angle that corresponds to the horizontal angle between the min and max azimuth description points. The vertical angle of the plane is adjusted so that it corresponds to the vertical angle between the min and max elevation description points. The width and height of the simplified extent is set so that it is covering the same angular width and height as defined by the description points. This is achieved by using the anchor point as the point of reference and then calculating the horizontal and vertical vectors that describe how the plane extends in the two dimensions.

Figure 61 – The simplified extent is calculated from the description points by calculating the horizontal and vertical angle and how much it extends from the anchor point. In the figure the red dot shows the anchor point and the blue dots show the minimum and maximum azimuth points. The angles a and b correspond to the variables minAzim and maxAzim and the distances L1 and L2 correspond to displacement_left and displacement_right respectively.

The vector describing the horizontal angle of the plane is calculated as the vector between the vertices representing the minimum and maximum azimuth angles:

| horizontalDirectionVec_local= maxAzimVertex_local-minAzimVertex_local. | (299) |
| --- | --- |

The vector is then projected onto the horizontal plane:

| horizontalDirectionVec_localy= 0.0. | (300) |
| --- | --- |

Then the width of the plane is determined by calculating how much the plane extends in the horizontal dimension to match the minimum and maximum azimuth angles. The variable displacement_left represents the distance along horizontalDirectionVec_local that the left edge of the extent is displaced in relation to the anchor point.

| displacement_left= -min_distancesin(-minAzim)sin(max90°+ horizontalAngle+minAzim, 5°) | (301) |
| --- | --- |
| horizontal_angle= atanhorizontalDirectionVec_localx, horizontalDirectionVec_localz, | (302) |

where min_distance is the distance from the listening position to the anchor_point and minAzim is the azimuth angle of minAzimVertex_local.

Similarly, the length of the right side of the simplified extent is calculated as

| displacement_right= min_distancesin(maxAzim)sin(max90°- horizontalAngle-maxAzim, 5°), | (303) |
| --- | --- |

where maxAzim is the azimuth angle of maxAzimVertex_local.

The vector describing the vertical angle of the plane is calculated as the vector between the vertices representing the maximum and minimum elevation angles:

| verticalDirectionVec_local = maxElevVertex_local-minElevVertex_local | (304) |
| --- | --- |
| verticalDirectionVec_localz= 0.0. | (305) |

The vector is then projected on the vertical plane of the local coordinate system:

The height of the plane is determined by how much the plane extends in the vertical dimension to match the minimum and maximum elevation angles. The variable displacement_top represents the distance along verticalDirectionVec_local that the top edge is displaced in relation to the anchor point.

| displacement_top=-min_distancesin(-maxElev)sin(max90°- verticalAngle+maxElev, 0.1°) , | (306) |
| --- | --- |

where maxElev is the elevation angle of maxElevVertex_local and verticalAngle is calculated as

| vertical_angle= atanverticalDirectionVec_localy, verticalDirectionVec_localx. | (307) |
| --- | --- |

Similarly, the displacement of the bottom edge of the simplified extent is calculated as

| displacement_bottom=min_distancesin(minElev)sin(max90°- verticalAngle+minElev, 0.1°), | (308) |
| --- | --- |

where minElev is the elevation angle of minElevVertex_local.

Before the actual edge points of the simplified extent are computed, the effective extent size is calculated as described in 6.6.15.3.2. In this calculation the geometric width is the sum of displacement_left and displacement_right and the distance to the anchor point is used as the distance to the extent. The effective width is then calculated and a scaler, width_scaler, is calculated as the effective width divided by the geometrical width. This scaler is then applied to both displacement_left and displacement_right. The same principle is used to calculate the effective height and thereby calculating the resulting displacement_top and displacement_bottom.

Finally, the edge points of the simplified extent are calculated as

| leftEdgePos_relative= anchorPoint_relative+displacement_left*horizontalDirectionVec_local | (309) |
| --- | --- |

| rightEdgePos_relative= anchorPoint_relative+displacement_right*horizontalDirectionVec_local | (310) |
| --- | --- |

| topEdgePos_relative = anchorPoint_relative+displacement_top*verticalDirectionVec_local | (311) |
| --- | --- |

| bottomEdgePos_relative= anchorPoint_relative+displacement_right*verticalDirectionVec_local | (312) |
| --- | --- |

Before the edge points of the simplified extent are stored, they are temporally smoothed to avoid sudden jumps when they are not expected. This smoothing is taking into account the change of relative position of the audio element and the listening position from the previous frame to the current. Before updating the edge point positions, the change of relative position of the audio element and the listener since the last frame, ∆D, is first calculated. The edge point positions will then only be changed with a maximum distance of ∆D. In addition to this there is also a slow smoothing applied based on a forgetting factor, that will allow the edge points to move even if ∆D is zero.

##### Calculating the effective spatial extent

For large extents, or when the listener is close to a moderately sized extent, the part of the extent that is close to the listener will usually overpower the energy coming from parts of the extent that are further away. Therefore, only a part of the geometrical extent may be acoustically relevant at a given listening position. Basing the rendering on the geometrical size may in such situations result in a subjective extent that is unnaturally wide and therefore the acoustically relevant part of the extent is determined, which is called the effective spatial extent.

The effective spatial extent, expressed in meters, is calculated according to the following formula:

| effective spatial extent=6D; D≤L6L; D>L6 | (313) |
| --- | --- |

where L is the geometrical size of the (simplified) extent, and D is the distance to the (simplified) extent, both expressed in meters. Eq. x was derived from a physics model of the sound field radiated by extended sound sources, combined with the psychoacoustical notion that parts of a source that do not contribute noticeably to the loudness of the source will also not contribute significantly to its perceived spatial size.

As can be seen from (313) , the effective spatial extent is only different from the geometrical spatial extent size at relatively close distances to the source, i.e., closer than 1/6 of the geometrical size, where the effective spatial extent becomes directly proportional to the distance. (Another way of looking at this is that the effective spatial extent becomes constant when expressed in angular terms). Note however, that for very large sources the listener may essentially always be in this “close” region.

In the Heterogeneous Extent stage, the effective spatial extent concept is used to modify the simplified extent according to the description in 6.6.15.3.1. This modified simplified extent is then used for positioning the virtual loudspeakers that are used to render the source to the listener, as described in more detail in 6.6.15.3.3.

##### Updating the virtual speaker configuration for the exterior representation

The exterior representation is rendered using an adaptive setup of virtual loudspeakers. The number of virtual loudspeakers used in the setup depends on the angular width and height of the extent, as seen from the listener position. Three different setups of virtual loudspeakers are used with either one, three or five virtual loudspeakers and smooth transitions between these setups are provided.

Figure 62 – The three virtual loudspeaker setups used by the heterogeneous rendering stage. The same five virtual loudspeakers are used for the different setups by controlling their positions and gain.

When an extent is large, or the listener is close to the extent, the setup with five virtual loudspeakers is used in order to provide good spatial quality rendering including both the width and height information. For extents where the height is relatively small compared to the width, the setup with three virtual loudspeakers is used since it is perceptually not relevant to render the spatial information in the height dimension. For small extents, or extents at a large distance, only one virtual loudspeaker is used.

The adaptation of the virtual loudspeaker setup is based on the horizontal and vertical opening angle of the extent as seen from the listening position. By adapting both the positions and gains of the virtual loudspeakers, it is possible to go seamlessly from one setup to another.

Figure 63 - Angle a denotes the horizontal opening angle and is associated with the perceived width of the audio element. Angle b denotes the vertical opening angle and is associated with the perceived height of the audio element.

Two transition factors control the adaptation of the rendering setup. These factors are stored in the array trans_fac[]. In this description they will be denoted transfac1 and transfac2 for convenience.

| transfac1=sinazim_ang- sin(ang_st) sinang_end-sin(ang_st)transfac2=sinelev_ang- sin(ang_st) sinang_end-sin(ang_st) | (314) |
| --- | --- |

where azim_ang is the horizontal angle divided by 2, elev_ang is the vertical opening angle divided by 2, ang_st is the angle where the transition starts and ang_end is the angle where the transition ends. In the implementation ang_st is set to π/64 and ang_end is set to π/24.

In order to keep the overall power of the virtual loudspeaker setup constant, a number of gain scaler variables are calculated.

| ptLineAtt=gainCompPt-gainCompPt-gainCompLine∙transfac1ptLineAmp=gainCompLine ∙transfac1lin2DimAtt=gainCompLine-gainCompLine-gainComp2D∙transfac2lin2DimAmp=gainComp2D ∙transfac2 | (315) |
| --- | --- |

where gainCompPt, gainCompLine and gainComp2D are gain scalers for the three different rendering setups that compensate for the different number of speakers used.

| gainCompPt=1gainCompLine=13gainComp2D=15 | (316) |
| --- | --- |

The gains of the signals for the five virtual loudspeakers used for the exterior representation are then set as

| g0=ptLineAmp ∙lin2DimAttg1=ptLineAmp ∙lin2DimAttg2=ptLineAtt ∙lin2DimAttg3=lin2DimAmpg4=lin2DimAmp, | (317) |
| --- | --- |

where g0 to g4 are gain adjustments of the signals going to the virtual loudspeakers shown in Figure 62.

The positions of virtual loudspeakers 0 and 1 are adapted between being situated in the lower left and right corners, for the 5-speaker setup, and being situated left and right edge points, for the 3-speaker setup. This is done with a linear interpolation based on trans_fac1.

The position of the middle speaker is adaptively controlled by the angular width of the extent as seen from the listening position. When the angular width of the extent is small, the middle speaker is placed  in the centre between the left and right edge points of the simplified extent but as the angular width increases, the position of the middle speaker will go towards the closest point of the simplified extent. This will avoid the problem with a psychoacoustical hole in the middle when the listener is close to the extent. This is done with the following equation:

| midSpeakerPos= leftEdgePoint∙rightToListenerDist+rightEdgePoint∙leftToListenerDistleftToListenerDist+rightToListenerDist | (318) |
| --- | --- |

where leftEdgePoint is the point between the top left and bottom left corners of the simplified extent and rightEdgePoint is the point between the top right and bottom right corners. leftToListenerDist and rightToListenerDist are the distances to the left and right edge points from the listening position.

The overall gain of the rendered exterior representation follows the distance gain behaviour of the target distance gain model. Using the bitstream variable distanceGainModel, the target distance gain model is chosen to be either that of a point-source or that of a volumetric source where the distance gain is calculated taking the size of the extent into account. corresponding to an extended source with the specified extent. To ensure that the target distance gain behaviour is achieved, a gain correction is performed, as described in 6.6.15.3.10.

##### Updating the virtual speaker configuration for the interior representation

The interior representation is rendered with a spherical setup of 12 virtual loudspeakers positioned around the listening position, arranged as a lower, a middle and a top circle.

Figure 64 – The 12 virtual loudspeakers used for the interior representation are arranged as three circles at different height.

The positions of the virtual speakers relative to the front vector of the rendering setup is listed in Table 144.

- Table 144 – Spherical coordinates of the virtual loudspeakers of the rendering setup for the interior representation relative to the front vector.

| Virtual speaker index | Spherical coordinates (azimuth, elevation) |
| --- | --- |
| 0 | -90, 0 |
| 1 | 90, 0 |
| 2 | 0, 0 |
| 3 | -180, 0 |
| 4 | -45, 45 |
| 5 | -45, -45 |
| 6 | 45, 45 |
| 7 | 45, -45 |
| 8 | -135, 45 |
| 9 | -135, -45 |
| 10 | 135, 45 |
| 11 | 135, -45 |

The horizontal orientation of the setup is defined by the direction of the front speaker. The orientation is not following the head movements of the listener but is instead aligned with the horizontal outline of the extent of the audio element.

The horizontal outline of the extent is calculated as the outline of the projection of the extent onto the horizontal plane. The outline is stored as a list of normals along with their positions. When aligning the rendering setup of the interior representation, the closest normal is found and the horizontal orientation of the rendering setup is adjusted so that its front vector is pointing in the negative normal direction, i.e., inwards from the outline of the extent.

Figure 65 – The figure shows the horizontal outline of an extent as the dotted line and its normal. The rendering setup for the interior representation is visualized as the dotted circle and its front vector with a red arrow. The closest normal of the outline is shown as a blue arrow and the front vector is aligned to point in the negative direction of the closest normal. In the left figure the listener position is outside of the extent and in the right figure the listening position is inside the extent.

Only the horizontal orientation of the rendering setup is changed, thus it does not matter if the listening position is above or below the extent. When identifying the closest normal of the outline, the listening position is projected onto the horizontal plane.

##### Transition between exterior and interior representations

Internally in the heterogeneous rendering stage the state of the rendering setups for the exterior and interior representations is continuously updated, including positions of virtual loudspeakers. The output of the rendering stage is however a combination of the rendering setups where some virtual loudspeakers are reused for both of the representations. Most of the time only a subset of the virtual loudspeakers is active and virtual loudspeakers that are not active are deactivated by setting their gain to zero. These are then excluded from the rendering and will not add unnecessary complexity.

The heterogeneous rendering stage uses a maximum of 14 virtual loudspeakers to render the combined output of the interior and exterior representations. Three virtual loudspeakers, 0, 1 and 2 in Table 144, are used in the rendering of both the interior and exterior representations, which is why 14 virtual loudspeakers are enough to render the combination of the 5 virtual loudspeakers of the exterior representation and the 12 virtual loudspeakers of the interior representation.

Within a transition region outside of the extent of the audio element, there is a gradual transition between the exterior and interior representations. The rendering setup used for the interior representation is aligned with the horizontal outline of the extent and therefor the virtual loudspeakers 0, 1 and 2 can be reused by both representations since they represent the left, middle and right of the audio element as seen from the listening position in both representations. The reusage of virtual loudspeakers is done both for reduction of complexity involved in rendering the virtual loudspeakers but also to avoid problems with comb-filtering effects due to virtual loudspeakers that are fed with correlated signals when they move close to each other.

During the transition, there is a linear interpolation of the positions of the reused virtual loudspeakers so that they are seamlessly going from the positions corresponding to the interior and exterior representations respectively. The signals going to the reused virtual loudspeakers are also cross-faded between the signals that correspond to the interior and exterior representations respectively.

Figure 66 – The transition between the exterior and interior representations. Active virtual loudspeakers are shown as red. In part 1, the listener is outside the transition region and only the exterior representation is rendered. In part 2, the listener is inside the transition region and interpolation is used between positions of the virtual loudspeakers of the interior and exterior representations are used for the reused virtual loudspeakers. In part 3, the listener is at the surface of the extent and here only the interior representation is used. The virtual loudspeaker representing the back of the rendering setup is suppressed. In part 4, the listener has moved some distance into the extent and here only the interior representation is used.

The transition is controlled by comparing the distance from the listening position to the extent, distanceToExtent, to the transitionDistance parameter. If the listener is inside the extent, distanceToExtent is negative. The parameter, interiorExteriorWeight, is then calculated as

| interiorExteriorWeight =0distanceToExtent >transitionDistance1distanceToExtent<0transitionDistance-(distanceToExtent-extraMargin)transitionDistance 0≤distanceToExtent ≤transitionDistance | (319) |
| --- | --- |

where extraMargin is a distance from the extent where only the interior representation is used.

The interiorExteriorWeight is used to interpolate the positions of the reused virtual loudspeakers:

| outputSpeakerPos0= interiorExteriorWeight∙interiorSpeakerPos0 +(1- interiorExteriorWeight)∙exteriorSpeakerPos0outputSpeakerPos1=interiorExteriorWeight∙interiorSpeakerPos1 +(1- interiorExteriorWeight)∙exteriorSpeakerPos1outputSpeakerPos2=interiorExteriorWeight∙interiorSpeakerPos2 +(1- interiorExteriorWeight)∙exteriorSpeakerPos2 | (320) |
| --- | --- |

The same linear interpolation, based on the parameter interiorExteriorWeight, is used for the cross-fade of the signals going to the virtual loudspeaker.

##### Modification of the interior representation for listening positions close to the extent

When the listening position is outside the extent but within the transition region and the transition between the exterior and interior representation is active, the interior representation is modified so that the rear hemisphere of the interior representation rendering setup is suppressed. This is done since the listener does not expect to hear sound coming from other directions than from the extent. Since the rendering setup of the interior representation is aligned with the surface of the extent, the rear hemisphere is always pointing away from the extent. This makes it possible to suppress only the virtual loudspeakers representing this hemisphere.

Likewise, when the listening position is above or below the extent, the top or bottom hemisphere of the rendering setup is suppressed.

Referencing the virtual loudspeaker indexes in Figure 62 the rear, top and bottom hemispheres are represented by the virtual loudspeakers as listed in Table 145. Notice that some virtual loudspeakers belong to two different hemispheres.

- Table 145 – Table of the different hemispheres of the rendering setup of the interior representation and which speakers belong to them.

| Hemisphere | Virtual loudspeakers |
| --- | --- |
| Rear | 3, 8, 9, 10, 11 |
| Top | 4, 6, 8, 10 |
| Bottom | 5, 7, 9, 11 |

A gain factor is calculated for each hemisphere, and these are applied when calculating the signals for the corresponding group of virtual loudspeakers.

The gain factor, rearGain, is controlled by the distance, distToOutline, from the listener position, projected onto the horizontal plane, to the horizontal outline of the extent and a check if the projected listening position is inside the outline or not. Whenever the projected listening position is outside of the horizontal outline, rearGain is set to 0.25 and the rear hemisphere is suppressed. Just inside of the horizontal outline there is a linear interpolation towards 1, that will fade in the rear hemisphere as the listener moves further inside the horizontal outline.

| rearGain =0.25outside of the outline 1distanceToOutline >transitionDistancemax⁡(0.5, 1-transitionDistance-distToOutlinetransitionDistance)20≤distanceToOutline ≤transitionDistance | (321) |
| --- | --- |

For the top hemisphere, the topGain, is calculated in a similar way but is instead controlled by the difference in altitude, topHeightDiff, of the listener position and the highest point of the extent. If the listening position is below the top point of the extent, topGain is set to 1. If the listening position is within a vertical transition distance, topTransitionDistance, above the top point of the extent there is a linear interpolation towards 0.25 so that the top hemisphere is gradually supressed the higher above the extent the listener is situated. In addition, topGain is set to 1 for all listening positions inside the extent.

| topGain =0.25topHeightDiff < -topTransitionDistance 1 inside the extent or when topHeightDiff>0max⁡(0.5, 1+topHeightDifftopTransitionDistance)2topTransitionDistance ≤topHeightDiff≤0 | (322) |
| --- | --- |

where

| topTransitionDistance = transitionDistance/6 | (323) |
| --- | --- |

The gain for the bottom hemisphere, bottomGain, is controlled by the difference in altitude, bottomHeightDiff, of the listening position and the lowest point of the extent. If the listening position is above the lowest point of the extent, bottomGain, is set to 1. If the listening position is within a vertical transition distance, bottomTransitionDistance, below the bottom point of the extent there is a linear interpolation towards 0.25 so that the bottom hemisphere is gradually supressed the lower below the extent the listener is situated. In addition, bottomGain is set to 1 for all listening positions inside the extent.

| bottomGain =0.25 bottomHeightDiff > bottomTransitionDistance 1 inside the extent or when bottomHeightDiff<0max⁡(0.5, 1-bottomHeightDiffbottomTransitionDistance)20≤bottomHeightDiff ≤bottomTransitionDistance | (324) |
| --- | --- |

where

| bottomTransitionDistance = transitionDistance/6 | (325) |
| --- | --- |

##### Handover to single-point HOA rendering stage for rendering the interior representation

For audio elements with a source signal in HOA format there is a handover implemented to the Single-Point HOA rendering stage so that the interior representation is rendered by that rendering stage as the listening position moves inside the extent of the audio element. The handover is implemented with a smooth cross-fade that starts when the listener moves inside the extent and completes when the distance from the surface of the extent is greater than transitionDistance.

##### Deriving the interior representation signals from the source signal

The rendering of heterogeneous audio elements is always based on the interior representation. For HOA sources with an extent, the interior representation is directly based on the first order spherical harmonics of the HOA source signal. For object sources with an extent and a multi-channel source signal, the interior representation is derived from the source signal.

The format of the interior representation depends on the format of the source signal and the extent layout labels associated with the respective channels of the source signal. As far as possible the original source signals are used as they are and only the missing signals in the interior representation need to be generated using a combination of mixing and decorrelation of the source signals.

- Table 146 – The format of the interior representation given the source format.

| Source signal format | Interior representation format |
| --- | --- |
| HOA | FOA |
| Stereo, left-right | Quadraphonic |
| Multi-channel Only horizontal extent layout labels | Quadraphonic |
| Multi-channel Horizontal and vertical extent layout labels | 3-layer Quadraphonic |

HOA source signal

For audio elements with a source signal in HOA format, the first order spherical harmonics are used as the interior representation in FOA format.

Stereo source signal

For audio elements with a source signal in left-right stereo format, the interior representation is based on a quadraphonic format, where the left and right channels from the source signal are reused as they are. The front signal is generated as a mix of the left and right. The back signal is then generated as a decorrelated version of the front signal.

Multi-channel source signal

For audio elements with a source signal in a multi-channel format other than stereo, the format used for the interior representation depends on the extent layout labels associated with the respective channels of the source signal. If the extent layout only includes labels describing the horizontal dimension, i.e., left, centre, right, the interior representation will be based on a quadraphonic audio format with four audio signals representing the left, right, front and back of the audio element as described for the case of a stereo source signal. If the extent layout includes labels describing the vertical dimension, i.e., top-left, top, top-right, bottom-left, bottom or bottom-right, a 3-layered quadraphonic audio format is used where the top layer corresponds to the labels top-left, top and top-right. The lower layer corresponds to the labels bottom-left, bottom and bottom-right.

If a middle signal of some layer is not present in the source signal, the front signal of that layer is generated as a mix of the left and right channels of that layer. The back signal of each layer is generated as a decorrelated version of the front signal of the same layer.

Figure 67 – The format of the interior representation depends on the format of the source signal. To the left is the single layer quadraphonic format used when the source signal is a stereo signal or a multi-channel format with only left, centre right channels. To the right is the three-layer quadraphonic format used for multi-channel source signals where channels are also represents the top or bottom of the audio element.

##### Deriving the exterior representation signals from the interior representation

The signals for the exterior representation are derived from the interior representation using a set of virtual microphones that capture a certain direction of the sound field that is described by the interior representation.

Figure 68 – In order to derive signals for the virtual loudspeakers of the exterior representation, one virtual microphone per virtual loudspeaker is used to capture a certain direction of the sound field described by the interior representation. In the figure, three speakers, depicted as blue dots, are used to represent the simplified extent, depicted as a green line. In this case the middle virtual loudspeaker receives a signal mostly consisting of the front signal and left and right virtual loudspeakers receives signals mostly consisting of the left respectively right signals of the interior representation.

The virtual microphones are specified with a horizontal and elevation angle relative to the front vector of the interior representation, i.e., the direction which the front signal is directed.

For the case of an interior representation based on single layer quadraphonic format, the microphone works a directional mixing of the signals of the interior representation.

| sOUT=max0, cosθ∙iFRONT+max0,cosθ-π2∙iLEFT+max0,cosθ-π∙iBACK+max⁡(0, cos(θ-3π2))∙iRIGHT | (326) |
| --- | --- |

Where sOUT is the output signal of the virtual microphone, θ is the horizontal angle of the virtual microphone and iFRONT, iLEFT, iRIGHT and iBACK are the signals of the interior representation.

For the case of an interior representation based on a three-layer quadraphonic format, there is a directional mixing also between the layers based on the elevation angle. Then each layer has a gain according to the following:

| layerGainTop =1 φ> π2-micElevOverlap 0φ<-π2-micElevOverlap max⁡(0,sinφ+micElevOverlapotherwise | (327) |
| --- | --- |

| layerGainBottom =0 φ> π2-micElevOverlap 1φ<-π2-micElevOverlap max⁡(0,sin-φ+micElevOverlapotherwise | (328) |
| --- | --- |

| layerGainMiddle =0 φ> π2-micElevOverlap 0φ<-π2-micElevOverlap layerGainTop2+ layerGainBottom2otherwise | (329) |
| --- | --- |

where φ is the elevation angle and micElevOverlap is a controlled cross-feed between the layers and is set to π8.

The combined output of the three layers is then just the weighted sum of these where the signal from each layer is calculated the same way as for the single layer case.

| sOUT=layerGainTop∙sTOP+layerGainMiddle∙sMID+layerGainBottom∙sBOTTOM | (330) |
| --- | --- |

For the case of an interior representation based on a FOA format the signal from a virtual microphone is calculated as

| sOUT=p∙i0+1-pcosφsinθ∙i1+∙i2+cos(φ)cos(θ)∙i3 | (331) |
| --- | --- |

Where i0, i1, i2 and i3 are the four channels of the FOA signal of the interior representation in ACN channel order and p which is a floating point value between 0 and 1 that describes the microphone pattern.

Figure 69 – Examples of different input layout alignments. To the left, the input layout is aligned with the front vector of the audio element. To the right, the input layout is aligned with the direction to the listening position.

The orientation of the interior representation can be controlled in two ways, either it is aligned with the front vector of the audio element, or it is aligned with the direction to the listening position. In cases where the audio element has a clear directionality, what is left and right, e.g., a piano, the interior representation can be aligned with the front vector of the audio element. In other cases where the audio element is not expected to have clear directionality, such as the sound of a beach line, it is beneficial to let the interior representation be aligned with the direction to the listener since the best audio quality can be retained as the listener moves around the audio element because the generated back signals are not used other than when the listener is inside, about or below the audio element.

##### Distance gain correction

As described in the preceding subsections, heterogeneous extended sources are rendered to the listener using a (variable) number of discrete virtual loudspeakers. This affects the distance attenuation behaviour of the rendered extended source, i.e., the resulting sound level of the extended source at the listener position as a function of distance. Generally, the resulting distance gain curve will be different from the distance gain curve that is expected for an extended source of the corresponding size.

This issue is addressed by applying an overall scalar distance gain correction to the rendered heterogeneous source. The distance gain correction module essentially calculates the “target” distance gain gtarget for an extended source of the specified size at the current listener distance, as well as the distance gain grendered of the rendered heterogeneous source at that same distance, and then calculates the distance gain correction C that is required to match the distance gain of the rendered source to the target distance gain, as follows:

| CDS=gtargetDSgrenderedDS | (332) |
| --- | --- |

with DS the current distance to the source.

The target distance gain for the extended source is calculated from the geometrical distance gain model described in 6.6.17.4. Using the bitstream variable distanceGainModel, the target distance gain model is chosen to be either that of a point-source or that of a volumetric source where the distance gain is calculated taking the size of the extent into account.

The distance gain of the rendered heterogeneous source is calculated from the instantaneous gains and positions of the virtual loudspeakers (which, as described in the preceding subsections, are dynamic functions of the listener distance), according to the following formula:

| grenderedDS=i=04gi2 | (333) |
| --- | --- |

where:

| gi=gi,processingDS*gi,distanceDi | (334) |
| --- | --- |

is the overall gain of the rendered signal of virtual loudspeaker i at the listener position, which combines the gain gi,processing that includes any gain applied to the virtual loudspeaker in the heterogeneous source processing described in the preceding subsections of 6.6.15.3, and the distance gain gi,distance for virtual loudspeaker i corresponding to the current distance Di between the listener and virtual loudspeaker i.

In (333), the virtual loudspeakers i=0…4 correspond to the virtual loudspeakers with the same labels inFigure 62 in 6.6.15.3.3.

The distance gain correction C follows from inserting (333) and (334) into (332) and is applied equally to all virtual loudspeakers i=0…4.

##### Occlusion of Heterogeneous Audio Elements

The occlusion of heterogeneous audio elements is rendered by modifying the simplified extent and by applying filters on the signals going to the virtual loudspeakers that are used for rendering the audio element.  In order to do the dection in an efficient way, it is done in a two-step process as described in the following sections.

The occlusion detection makes use of the same ray tracing functionality that is used in the occlusion rendering stage described in 6.6.9.  If voxel-based scene geometry is available in the bitstream, the voxel grid line-of-sight traversing described in 6.6.11.3.1.5 is used.

###### Detection of cropping occlusion

Figure 70 – An example of a heterogeneous audio element and an occluding object. A set of rays are cast towards the simplified extent. The part of the simplified extent which is occluded from the listening position is discarded to form the modified, or “cropped”, extent.

The first stage of the occlusion detection detects so called cropping occlusion, wich is occlusion that completely occludes one or several edges of the simplified extent. If one ore more edges of the simplified extent is completely occluded, a modified extent will be calculated where the completely occluded parts are removed.

The detection of cropping occlusion is performed by sending out rays from the listening position towards a sparse grid of points on the simplified extent and checking which rays are not completely occluded. The edges of the modified, or cropped, extent are found by identifying the points on the simplified extent representing the highest and lowest horizontal and vertical angles, as seen from the listening position, that are not completely occluded. These points are stored as starting points for binary searches where the exact edges are found.

The number of rays sent towards the simplified extent is adaptive and depends on the horizontal and vertical opening angles of the extent, as seen from the listening position. The number of columns of the grid is calculated as

| numOfAzimDivisions=min20, max5,azimRange/5.0 , | (335) |
| --- | --- |

where azimRange denotes the horizontal opening angle.

Similarly, the number of rows in the grid is calculated as

| numOfElevDivisions=min10, max3,elevRange/5.0 , | (336) |
| --- | --- |

where elevRange denotes the vertical opening angle.

In order to avoid checking the same azimuth and elevation positions several times, the grid is shifted in position by adding an accumulated offset to both the horizontal and vertical positions between consecutive updates of the rendering stage. The following pseudo code illustrates the calculation of the positions in the grid:

gridShiftCnt++;

if (gridShiftCnt > 10) {

gridShiftCnt = 0;

}

float gridShift = gridShiftCnt / 10;

for (int v = 0; v < numOfElevDivisions; v++) {

float elevPos = 0.01

+ 0.98 * (gridShift / numOfElevDivisions + v / numOfElevDivisions);

for (int h = 0; h < numOfAzimDivisions; h++) {

float azimPos = 0.01

+ 0.98 * (gridShift / numOfAzimDivisions + h / numOfAzimDivisions);

// Check occlusion for point (azimPos, elevPos)

}

}

In this code, the positions are given in a local coordinate system (azimPos, elevPos) where (0.0, 0.0) is the bottom-left corner of the simplified extent and (1.0, 1.0) is the top-right corner.

In addition to the even grid, the center, top-left, top-right. bottom-left and bottom right points of the previous cropped extent are checked. Finally, binary searches are performed to find the exact left, right, top, and bottom edges of the cropped extent.

When the cropped extent has been calculated, the positions of the center and the four corners are stored so that they can be checked at the next call of the update function of the rendering stage.

###### Detection of soft and partial occlusion

Figure 71 – Soft and partial occlusion within the cropped extent is detected by dividing the cropped extent into six sub-areas and casting a grid of rays to each. An occlusion EQ is calculated for each sub-area, which corresponds to the soft and partial occlusion within the sub area.

In the second stage of occlusion detection the modified, or cropped, extent of the audio element is checked for soft and partial occlusion. This is done by first defining six sub areas of the modified extent: top-left, top-mid, top-right, bottom-left, bottom-mid and bottom-right. For each sub area, an even grid of points is checked for occlusion with raytracing.

For each checked point on the sub areas, an occlusion filter is calculated which describes the occlusion effect for that specific point, from the listening position. The occlusion filters are specified in form of a filter that the EQ renderin stage can process.  If there is no occlusion, the EQ filter will be specified as having no attenuation for any frequency. If the point is completely occluded, the filter will be specified to attenuate all frequencies completely. If there was soft occlusion, the filter will be specified according to the transmission parameters of the occluding material. If the ray passes through more than one material, a compound occlusion filter is calculated by accumulating the occlusion filter of each material.

An overall occlusion filter is then calculated for each sub area by accumulating the occlusion filters for each checked point of that sub area. Since the heterogeneous sources are assumed to be diffuse, the filters are not accumulated linearly but instead according to the formula

| gSA,f=1Ngn,f2N | (337) |
| --- | --- |

where gSA denotes the accumulated gain for a certain frequency, f,  and for one sub area, gn is the gain of frequency f for one sample point in the sub area and N is the number of sample points.

If any occlusion was detected in the cropping occlusion check, the number of rays per sub area is set according to:

| horizRays=min10, max1, azimRange | (338) |
| --- | --- |

| vertRays=min5, max1, elevRange | (339) |
| --- | --- |

If no occlusion was detected in the cropping occlusion check, only one ray per sub area is used.

When calculating the grid positions to check within each sub area, a skewed regular grid is used, where an increasing offset is added to the horizontal positions in each row of rays, This avoids that the same horizontal position is checked for each row. This spreading of horizontal positions provides a smoothing effect when an occluder has a straight vertical edge, as is the case in many cases, like e.g. a doorway or the end of a wall etc. The following pseudo code illustrates the calculation of the positions in the grid for each sub area:

// Send a grid of rays towards the subarea and accumulate any occlusion EQs

for (int elevLoop = 0; elevLoop < vertRays; elevLoop++) {

// Add an offset to the azimuth to prevent check of the same azimuth many times

azimOffset = elevLoop / vertRays / horizRays;

for (int azimLoop = 0; azimLoop < horizRays; azimLoop++) {

float subAreaAzimPos = azimLoop / horizRays + azimOffset;

float subAreaVertPos = elevLoop / vertRays;

// Check occlusion for point (subAreaAzimPos, subAreaVertPos) within the sub area

}

}

In this code, the positions are given in a local coordinate system [subAreaAzimPos, subAreaElevPos] where (0.0, 0.0) is the bottom-left corner of the sub area and (1.0, 1.0) is the top-right corner.

All occlusion filters are scaled with a gain factor that accounts for how much of the simplified extent was cropped:

| eq_scaler=sqrtactiveExtentRatio, activeExtentRatio>0 0, activeExtentRatio= 0 | (340) |
| --- | --- |

The ratio, activeExtentRatio, is calculated as the area of the cropped extent divided by the area of the simplified extent.

###### Mapping of sub area occlusion filters to filters for each virtual speaker

Since the virtual loudspeakers used for rendering the heterogeneous audio element do not correspond directly to the sub areas, one-to-one, the occlusion filters of the sub areas need to be mapped to occlusion filters that are applied to each virtual loudspeaker. This is done using a set of three mapping matrices that correspond to the three virtual loudspeaker setups described in 6.6.15.3.3.

For the five-speaker setup the mapping is done using the following matrix:

| fTLfTRfMfBLfBR=1.00.00.00.00.00.00.00.01.00.00.00.00.150.20.150.150.20.150.00.00.01.00.00.00.00.00.00.00.01.0f1f2f3f4f5f6 | (341) |
| --- | --- |

where f1-f6 are the occlusion filters for sub areas 1-6 and fTL, fTR, fM, fBL, and fBR are the occlusion filters for the corresponding virtual loudspeakers.

For the three-speaker setup the following mapping matrix is used:

| fTLfTRfMfBLfBR=0.50.00.00.50.00.00.00.00.50.00.00.50.150.20.150.150.20.150.00.00.00.00.00.00.00.00.00.00.00.0f1f2f3f4f5f6 | (342) |
| --- | --- |

For the one-speaker setup the following mapping matrix is used:

| fTLfTRfMfBLfBR=0.00.00.00.00.00.00.00.00.00.00.00.50.16660.16660.16660.16660.16660.16660.00.00.00.00.00.00.00.00.00.00.00.0f1f2f3f4f5f6 | (343) |
| --- | --- |

Since the speaker setup is adaptive, the mapping needs to follow this adaptation. This is done as

| FS=aPOINTMPOINT+aLINEMLINE+ aPLANEMPLANEF0 | (344) |
| --- | --- |

where Fs is a matrix of the mapped filters, F0 is a matrix of the sub area occlusion filters f1-f6, MPOINT is the mapping matrix of the one-speaker setup, MLINE is the mapping matrix of the two-speaker setup, MPLANE is the mapping matrix of the five-speaker setup. aPOINT, aLINE, and aPLANE are control parameters that control the weight of the three mapping matrices. These control parameters depend on the transition parameters transfac1 and transfac2, which were described in 6.6.15.3.3.

| aPOINT=1- transfac1 aLINE=transfac1(1-transfac2) aPLANE=transfac1∙transfac2 | (345) |
| --- | --- |

###### Adaptation of the transition distance due to occlusion

The transition from the exterior representation of a heterogeneous audio element to the interior representation happens within a transition distance from the extent of the audio element as described in 6.6.15.3.5. However, in the case that there is an occluder between the listener and the extent, this transition needs to be modified. This is done by calculating a modified transition distance as

| modTransDist=activeExtRatio∙transitionDistance | (346) |
| --- | --- |

where transitionDistance is the normal transition distance when there is no occlusion and activeExtRatio is the amount of extent which is active, i.e. not occluded. This has the effect that, for example, if the whole of the extent is occluded from the listening position, activeExtRatio is zero and the transition distance will also become zero.

###### Adaptation of the virtual microphones due to occlusion

When a large part of a heterogeneous audio element is occluded it is expected that the signals sent to the virtual loudspeakers reflects the part of the extent that is audible, e.g., if the left half of the extent of a piano is occluded, it is expected that the higher notes are heard more than the lower since the lower strings are to the left of the piano, assuming a position in front of the keybed. This effect can be achieved by controlling the angles of the virtual microphones that are used to derive signals from the interior representation, as described in 6.6.15.3.9.

By default, the virtual microphones are configured to capture the full width and height of the audio element. If a substantial part of the extent is occluded the angles of the microphones are adapted so that they capture less of the part that is occluded, e.g., the left part or the lower part. This is done with a scaling factor for left, right, top, and bottom edges respectively:

| occlLeftAngleScaler=max0.5, 2.0*1.0 - minUnoccludedAzim- 1.0 occlRightAngleScaler=max0.5, 2.0 * maxUnoccludedAzim- 1.0 occlTopAngleScaler=max0.5, 2.0 * 1.0 - minUnoccludedElev- 1.0 occlLeftAngleScaler=max0.5, 2.0 * 1.0 - minUnoccludedElev- 1.0 | (347) |
| --- | --- |

where minUnoccludedAzim, maxUnoccludedAzim, minUnoccludedElev and minUnoccludedElev denotes the positions of the left, right, top, and bottom edges of the cropped extent expressed as a number in the range [0, 1.0] where an azimuth position of 0.0 corresponds to the left edge of the original simplified extent, and an azimuth position of 1.0 corresponds to the right edge. An elevation position of 0.0 similarly corresponds to the bottom edge of the original simplified extent and 1.0 corresponds to the top edge. Each scaling factor is limited to never go below 0.5 to avoid an unnaturally strong effect on the signals derived.

### Directivity

#### General

Directivity is used to auralize the Directivity property of Audio Elements. To do this, the Directivity tool is comprised of two components: the coding of the Directivity data, and the rendering of the Directivity data. The Directivity is represented as a number of Covers (see 6.6.16.2.1), where each Cover is arithmetically coded. The rendering of the Directivity is done by checking to see which RIs use Directivity, taking the filter gain coefficients from the Directivity, and applying an EQ to the metadata of the RI.

#### Data elements and variables

covers	This array holds all decoded directivity Covers

dbStepIdx	This is the index of the decibel quantization range.

dbStep	This number is the decibel step that the values have been quantized to.

intPer90	This integer is the interval of azimuth points per 90 degrees around the equator of the Cover.

elCnt	This integer is the number of elevation points on the Cover.

aziCntPerEl	Each element in this array represents the number of azimuth points per elevation point.

coverWidth	This number is the maximum azimuth points around the equator.

minPosVal	This number is the minimum possible decibel value to be coded.

maxPosVal	This number is the maximum possible decibel value to be coded.

minVal	This number is the lowest decibel value that is actually present in the coded data.

maxVal	This number is the lowest decibel value that is actually present in the coded data.

valAlphabetSize	This is the number of symbols in the alphabet for decoding.

predictionOrder	This number represents the prediction order for this Cover. This influences how the Cover is reconstructed using the previous residual data, if present.

cover	This 2d matrix represents the Cover for a given frequency band. The first index is the elevation, and the second index is the azimuth. The value is the dequantized decibel value for that azimuth and elevation. Note, the length of the azimuth points is variant.

max_cover	This is the maximum dequantized decibel value of the whole cover.

max_band_gain	This is the maximum gain value of the central EQ metadata.

directiveness	Intensity of the directivity filter.

broadband_gain	Accumulated broadband gain of the current RI.

distance	Distance between the listener and the Audio Element.

refDistance	Reference distance of the current RI for the distance attenuation model.

coverResiduals	This 2d matrix represents the residual compression data for the Cover. It mirrors the same data structure as cover, however the value is the residual data instead of the decibel value itself.

freq	This is the final dequantized frequency value in Hertz.

freqIdx	This is the index of the frequency that needs to be dequantized to retrieve the original value.

freq1oIdxMin	This is the minimum possible index in the octave quantization mode.

freq1oIdxMax	This is the maximum possible index in the octave quantization mode.

freq3oIdxMin	This is the minimum possible index in the third octave quantization mode.

freq3oIdxMax	This is the maximum possible index in the third octave quantization mode.

freq6oIdxMin	This is the minimum possible index in the sixth octave quantization mode.

freq6oIdxMax	This is the maximum possible index in the sixth octave quantization mode.

##### Definitions

Sphere Grid	A quasi-uniform grid of points upon the surface a unit sphere.

vei,ai	Where v is the current Cover, ei is the elevation index, and ai is the azimuth index.

vei,ai	Where v is the current Cover’s fixed linear predictor, ei is the elevation index, and ai is the azimuth index.

vei,ai	Where v is the current Cover that has been circularly interpolated, and where ei is the elevation index, and where ai is the azimuth index.

nei		Where n is the number of azimuth points in the Sphere Grid per elevation, and where ei is the elevation index.

##### Decoding process

Once the directivity payload is received by the renderer, before the Directivity stage initialization, the decoding process begins. Each Cover has an associated frequency; direcFreqQuantType indicates how the frequency is decoded, i.e. determining the width of the frequency band, which is done in readQuantFreq(). The variable dbStep determines the quantized step sizes for the gain coefficients; its value lies within a range between 0.5 and 3.0 with increments of 0.5. intPer90 is the number of azimuth points around a quadrant of the equator and is the key variable used for the Sphere Grid generation (see 6.6.16.2.2.1). direcUseRawBasline determines which of two decoding modes is chosen for the gain coefficients. The available decoding modes are either the “Baseline Mode” or the “Optimized Mode” (see 6.6.16.2.2.2 and 6.6.16.2.2.3, respectively). The baseline mode simply codes each decibel index arithmetically using a uniform probability distribution. Whereas, the optimized mode uses residual compression in conjunction with an adaptive probability estimator alongside five different prediction orders. Finally, after the completion of decoding, the directivities are passed to the Scene State where other Scene Objects can refer to them.

###### Sphere grid generation

The Sphere Grid determines the spatial resolution of a Cover that can be different across Covers. The Sphere Grid of the Cover has a number of different points. Across the equator, there are at least 4 points, possibly more depending on the intPer90 value. At the north and south poles, there is exactly one point. At different elevations, the number of points is equal or less than the number of points across the equator, and is decreasing as the elevation approaches the poles. Upon each elevation layer, the first azimuth point is always 0°, creating a line of evenly spaced points from the south pole, to the equator, and, finally, to the north pole. This property is not guaranteed for the rest of the azimuth points across different elevations. The following is a description in pseudocode format:

generateSphereGrid(intPer90)

{

piOver180 = acos(-1) / 180;

degStep = 90 / intPer90;

elCnt = 2 * intPer90 + 1;

azCnt[elCnt] = { 0 };

coverWidth = 4 * intPer90; // maximum number of azimuth points (at equator)

for (ei = 0; ei < elCnt; ei++)

{

elAng = (ei - intPer90) * degStep;

elLen = cos(elAng * piOver180);

azCnt[ei] = max(round(elLen * 4 * intPer90), 1);

}

return elCnt, aziCntPerEl, coverWidth

}

###### Baseline mode

The baseline mode uses a range decoder with a uniform probability distribution to decode quantized decibel values. The maximum and minimum possible values (i.e., maxPosVal, minPosVal) that can be stored are -128.0 and 127, respectively. The alphabet size can be found using dbStep and the actual maximum and minimum possible value (maxVal, minVal). After decoding the decibel, a simple rescaling is done to find the actual dB value. This can be seen in Table 26.

###### Optimized mode

The optimized mode decoding uses a sequential prediction scheme, which traverses the Cover in a special order. This scheme is determined by predictionOrder, where its value can be an integer between 1 and 5 inclusive. predictionOrder dictates which linear prediction order (1 or 2) to use. When predictionOrder == 1 || predictionOrder == 3, the linear prediction order is 1 and when predictionOrder == 2 || predictionOrder == 4, the linear prediction order is 2. The traversal is composed of four different sequences:

The first sequence goes vertically, from the value at the South Pole to the North Pole, all with azimuth 0. The first value of the sequence (coverResiduals[0][0]), at the South Pole is not predicted. This value serves as the basis in which the rest of the values are predicted from. This prediction uses either a linear prediction of order 1 or 2. Using a prediction order of 1 uses the previous elevation value, where a prediction order of 2 uses the two previous elevation values as a basis for prediction.

The second sequence goes horizontally, at the equator, from the value next to the one at azimuth 0 degrees (which was already predicted during the first sequence), until the value previous to it at azimuth close to 360 degrees. The values are predicted from previous values also using linear prediction of order 1 or 2. Similarly to sequence one, using a prediction order of 1 uses the previous azimuth value, where using a prediction of 2 uses the previous two azimuth values as a basis prediction.

The third sequence goes horizontally, in order for each elevation, starting from the one next to the equator towards the North Pole until the one previous to the North Pole. Each horizontal subsequence starts from the value next to the one at azimuth 0 degrees (which was already predicted during the first sequence), until the value previous to it at azimuth close to 360 degrees. When (predictionOrder == 1 || predictionOrder == 2 || predictionOrder == 3 || predictionOrder == 4)  the values are predicted from previous values using either linear prediction of order 1 or 2, as explained above. Furthermore, when (predictionOrder == 3 || predictionOrder == 4), in addition to the previous values on the current Cover, the values are also used from the previously predicted elevation. Since the number of points upon the Sphere Grid nei-1 at the previously predicted elevation ei-1 is different from the number of points nei at the currently predicted elevation ei, the number of azimuth points do not match across the elevations in the Sphere Grid. Therefore, the points vei-1,ai at the previously predicted elevation ei-1 are circularly interpolated to produce nei new points, where aiis azimuth index and v is a 2d vector representing the Cover. For example, if the number of points at the current elevation is 24, and the number of points at the previous elevation is 27, they are circularly interpolated to produce 24 new points. Interpolation is linear to preserve monotonicity. For a given point value to be predicted vei,ai, the previous point value horizontally vei,ai-1 and the corresponding previous point value vei-1,ai-1 and current point value vei-1,ai on the circularly interpolated new points (which are derived from the previous elevation level) are used as regressors to create a predictor with 3 linear prediction coefficients. A fixed linear predictor is used, i.e. vei,ai=vei,ai-1+vei-1,ai-vei-1,ai-1, which predicts perfect 2D linear slopes in dB domain.

The fourth sequence also goes horizontally, in order for each elevation, exactly like the third sequence, however starting from the one next to the equator towards the South Pole until the one previous to the South Pole.

The following pseudocode describes the aforementioned algorithm:

unpredict(predOrder, coverRes, prevCover)

{

if (predOrder == 5) {

for (ei = 0; ei < elCnt; ei++) {

for (ai = 0; ai < aziCntPerEl[ei]; ai++) {

i = ei * coverWidth + ai;

cover[ei][ai] = coverRes[ei][ai] + prevCover[ei][ai];

}

}

return;

}

// copy the original value at the South pole,

// coverRes[0], which is not predicted

cover[0] = coverRes[0];

// predict vertically, from the one after the

// South pole to the North pole, at azimuth 0

for (int ei = 1; ei < elCnt; ++ei) {

if ((predOrder == 1) || (ei == 1) || (predOrder == 3)) {

pred_v = cover[ei - 1][0];

}

else if ((predOrder == 2) || (predOrder == 4)) {

pred_v = 2 * cover[ei - 1][0] - cover[ei - 2][0];

}

cover[ei][0] = coverRes[ei][0] + pred_v;

// always use true order 1 or true order 2 horizontal prediction at the equator

if (((predOrder == 3) || (predOrder == 4)) && (ei != intPer90)) {

continue;

}

// predict horizontally, from azimuth 0 to the maximum azimuth

for (int ai = 1; ai < aziCntPerEl[ei]; ++ai) {

if ((predOrder == 1) || (ai == 1) || (predOrder == 3)) {

pred_h = cover[ei][ai - 1];

}

else if ((predOrder == 2) || (predOrder == 4)) {

pred_h = 2 * cover[ei][ai - 1] - cover[ei][ai - 2];

}

cover[ei][ai] = coverRes[ei][ai] + pred_h;

}

}

if ((predOrder == 3) || (predOrder == 4)) {

cResample[coverWidth] = { 0 };

// predict horizontally for each elevation,

// from the one following the equator to the South pole

for (int ei = intPer90 - 1; ei >= 1; --ei) {

input = cover;

start = (ei + 1) * coverWidth;

count = aziCntPerEl[ei + 1];

newCount = aziCntPerEl[ei];

output = cResample;

circularResample(input, start, count, newCount, output);

for (int ai = 1; ai < aziCntPerEl[ei]; ++ai) {

pred_h = cover[ei][ai - 1] + (cResample[ai] - cResample[ai - 1]);

cover[ei][ai] = coverRes[ei][ai] + pred_h;

}

}

// predict horizontally for each elevation,

// from the one following the equator to the North pole

for (int ei = intPer90 + 1; ei < elCnt - 1; ++ei) {

input = cover;

start = (ei - 1) * coverWidth;

count = aziCntPerEl[ei - 1];

newCount = aziCntPerEl[ei];

output = cResample;

circularResample(input, start, count, newCount, output);

for (int ai = 1; ai < aziCntPerEl [ei]; ++ai) {

pred_h = cover[ei][ai - 1] + (cResample[ai] - cResample[ai - 1]);

cover[ei][ai] = coverRes[ei][ai] + pred_h;

}

}

}

}

#### Stage description

##### General

The stage iterates over all RIs in the update thread, checks whether Directivity can be applied, and, if so, the stage takes the relative position between the Listener and the RI, and queries the Directivity for filter coefficients. Finally, the stage applies these filter gain coefficients to the central EQ metadata field of the RI, to be finally auralized in EQ stage.

##### Update thread processing

Directivity is applied to all RIs with a value of true in the data elements of objectSourceHasDirectivity  and loudspeakerHasDirectivity (and by secondary RIs derived from such RIs in the Early Reflections and Diffraction stages) by using the central EQ metadata field that accumulates all EQ effects before they are applied to the audio signals by the EQ stage (6.6.20). The listener’s relative position in polar coordinates to the RI is needed to query the Directivity. This can be done, e.g. using Cartesian to Polar coordinate conversion, homogenous matrix transforms, or quaternions. In the case of secondary RIs, their relative position for their parents must be used to correctly auralize the Directivity. For consistent frequency resolution, the directivity data is linearly interpolated to match the EQ bands of the metadata field, which can differ from the bitstream representation, depending on the bitstream compression configuration. For each frequency band, directiveness (available from objectSourceDirectiveness or loudspeakerDirectiveness) is applied according to the formula Ceq=exp(d logm), where d is the directiveness value and m is the interpolated magnitude derived from the Covers adjacent to the requested frequency band, and Ceq is the coefficient used for the EQ.

The filter gain coefficients calculation shall be skipped to keep the overall complexity low, if the corresponding RI is fulfilling culling conditions in the Metadata Culling stage. Namely, the application of the Directivity stage shall be omitted, if the following product of RI gains is smaller than the culling threshold.

The following pseudocode describes the condition for skipping the Directivity stage:

max_magnitude = pow(10, (max_cover / 20) * directiveness);

distance = calculateDistance(listener_position, RI_position);

distanceGain = calculateDistanceGain(distance, refDistance);

if (max_band_gain * max_magnitude * broadband_gain * distanceGain < gainCullingThreshold) {

// skip directivity filter gain calculation

} else {

// perform directivity filter gain calculation

}

The distanceGain value to be applied to the RI shall be calculated by the method calculateDistanceGain(), based on the distance  and refDistance value of the current RI. The method calculateDistance() provides the Euclidean distance value between the listener position  listener_position and RI position RI_position.

##### Audio thread processing

The directivity stage has no additional processing in the audio thread. The application of the filter coefficients is done in the EQ stage.

### Distance

#### General

The distance stage renders three independent perceptual effects that are related to the travelling of sound through the air: propagation delay, distance gain and medium absorption. The stage calculates the current distance between each RI and the listener and interpolates the distance between calls to the update routine based on a constant velocity model.

Propagation delay is applied to the signals associated with RIs to produce a physically accurate delay and Doppler effect using a variable delay line with subsample interpolation. A smoothing is applied to the distance used for propagation delay rendering when updating the model velocity in order to mitigate jitter in the head-tracked listener location and RI location updates. The conversion from distance to propagation delay is calculated with a speed of sound given by the local configuration parameter SpeedOfSound (6.4.2.7). The propagation delay is also responsible for the delay of early reflections relative to the corresponding direct sound RI. See 6.6.17.3 for details.

Distance attenuation models the frequency-independent attenuation of audio elements due to geometrical spreading of the source energy. For the distance attenuation of geometrically extended sources, a model is used that takes the size of the sound source into account. See 6.6.17.4 for details.

Medium absorption models the frequency-dependent attenuation of audio elements related to the absorption characteristics of air, as defined in ISO 9613-1. See 6.6.17.5 for details.

#### Data elements and variables

currentTime	Current timer in audio processing thread

listenerLocation	Current listener location as 3D coordinate vector

itemLocation	RI location of most recent timestamp as 3D coordinate vector

itemTimestamp	Timestamp associated with most recent RI location

isCV	Boolean Flag indicating constant velocity RI

itemTeleport	Boolean Flag indicating that an item has been teleported

speedOfSound	Speed of sound (constant)

fs		System audio sampling rate

B		Length of audio processing block

teleportThreshold	Configuration variable for teleportation detection threshold (default: 25 m/s)

smoothingFactor	Configuration variable for velocity smoothing weighting (default: 0.95)

minDelay	Minimum delay that can be handled by variable delay line (fixed value originating from  delay line topology)

maxDelay	Configured maximum delay that can be handled by variable delay line (value is given by delayBufferSize according to Table 10)

listenerMovement 	Calculated listener movement speed

listenerTimeDiff	Calculated difference between listener timestamps

detectedListenerTeleport	Boolean flag indicating that listener teleport has been detected

dist	Calculated momentary distance between listener and RI

deltaT	Calculated time difference between current and previous timestamp

distPrevScene	Estimated distance between listener and RI for previous scene update

v		Calculated relative velocity between listener and RI

smoothVelocity	Smoothed relative velocity between listener and RI

smoothDistance	Smoothed relative distance between listener and RI

previousTime	State variable for previous audio thread timestamp

previousDistance	State variable for previous distance

previousListenerTrajTime	 State variable for previous listener location update timestamp

previousListenerLocation State variable for previous listener location as 3D coordinate vector

previousItemLocation	State variable for previous item location as 3D coordinate vector

previousItemTimestamp		State variable for previous item location update

previousTimeDiff	State variable for previous time differences between location updates

dly		Calculated delay in samples resulting from smoothed velocity model

#### Distance-based delay calculation with smooth velocity model

##### Overview

The distance between the listener location and a RI is calculated as the Euclidean distance in between, when a location update is provided by the update thread. The distance corresponds to the momentary propagation delay, which is reproduced using an interpolated variable delay line. Continuous variations of the propagation delay inherently produce a physically accurate Doppler effect. The Doppler pitch shift is a function of the relative velocity between a sound source and the observer, i.e., the derivative of the distance.

The human hearing is rather sensitive to sudden changes in pitch, and consequently a smooth velocity curve is desirable. Even minor jitter in RI and listener locations, e.g. due to head-tracking, numerical precision, clock jitter and asynchronous updates of the update thread and audio threads can cause artefacts. To ensure consistent movement, the distance is updated continuously with a smoothed velocity model in the audio thread. As long as no new RI location or listener location is provided by the update thread, the distance is continuously extrapolated assuming a constant velocity model.  When a location update is received, the relative velocity is re-estimated based on the previous location and the difference between the corresponding timestamps and the model velocity is smoothly updated by a weighted combination of the new and previous model velocity to prevent jumps in pitch shift.

##### Processing in the audio thread

###### Listener teleport detection

When a deliberate discontinuity in the listener location or RI location occurs, e.g. due to a teleportation, the smoothing is disabled and the velocity is reset to prevent implausible Doppler effects with extreme movement speeds.  Since in a user-interactive framework not all relevant listener location discontinuities are semantically marked as teleportation, the distance stage implements a listener teleport detection by checking for implausibly high velocity.

If an update of the listener location is received, and the renderer is not in the start-up phase, then the teleportation detection is performed. The Euclidean distance of the listener location and the time difference between the current and previous timestamp are calculated and the velocity of the listener movement is calculated. The velocity calculation shall be encapsulated in a sanity check to avoid division by zero (e.g. for updates with duplicate timestamps). If the listener movement velocity exceeds a configurable teleportation threshold, this shall be detected as a teleportation and marked by setting the Flag detectedListenerTeleport = true.  The teleportation detection is performed as follows.

listenerMovement = abs(listenerLocation-previousListenerLocation);

listenerTimeDiff = ct - previousListenerTrajTime;

if (listenerTimeDiff > 0) {

listenerMovement /= listenerTimeDiff;

if (listenerMovement > teleportThreshold) {

detectedListenerTeleport = true;

}

}

The threshold value shall be set to teleportThreshold = 25 [m/s] for room scale applications.

###### Velocity calculation

The model velocity v is defined as the difference in the relative distance between the listener and an RI within one block of the audio thread.

The Scene State distinguishes between RI with and without “constant velocity”, i.e. RI that are a priori defined with time-continuous locations by a constant velocity and a timestamped reference location (e.g. by the scene description) and RI with “static” locations that are defined by time discrete location updates with attached timestamps at runtime (e.g. provided by interactive scene updates), for which the underlying movement’s velocity is estimated by the distance stage at runtime.

In both cases, the current distance between listener and RI location is calculated as:

dist = abs(itemLocation – listenerLocation);

In case of RI with constant velocity that are marked isCV == true, the location for arbitrary points in time is uniquely defined by the equation of motion. Thus, the velocity relative to the listener is calculated as the difference between the distance in the current and previous audio frame.

v = dist – previousDistance;

In case of RI with static location updates that are marked isCV == false, location updates with attached timestamps are provided at a different (typically lower) rate than the audio frame rate and without explicit specification of the underlying movement. Therefore, the velocity relative to the listener is estimated by the Distance stage based on the distances between RI and listener location for the previous and new timestamp.

The elapsed time between location updates is calculated as the difference between the RI location timestamps. The time differences are calculated based on the original location update‘s timestamps rather than the audio thread timer, as external location updates are not synchronized to the audio processing block rate.

deltaT = itemTimestamp – previousItemTimestamp

The previous relative distance distPrevScene is calculated from the previously received update of RI or listener location. In case of listener teleportation or RI teleportation, the teleportation shall be represented to be instantaneous, i.e. not as an implausibly fast movement. In order to provide continuous Doppler effects in case of teleportation, the previous distance is estimated relative to the new location.

if (!itemTeleport && !detectedListenerTeleport) {

distPrevScene = abs(previousItemLocation - previousListenerLocation);

} else if (!itemTeleport && detectedListenerTeleport) {

distPrevScene = abs(previousItemLocation - listenerLocation);

} else if (itemTeleport && !detectedListenerTeleport) {

distPrevScene = abs(itemLocation - previousListenerLoc);

} else {

distPrevScene = abs(itemLocation-listenerLocation);

}

The relative velocity is calculated as the difference in relative distance divided by the elapsed time difference, and normalized to represent the velocity per audio processing block of block length B.

v = (dist - distPrevScene) / deltaT * B/fs;

###### Velocity smoothing

To prevent discontinuities in Doppler pitch shift, the velocity is smoothed by updating the model velocity smoothVelocity with a weighted contribution of the momentary velocity v (corresponding to a 1-tap IIR low-pass filter).

smoothVelocity = smoothVelocity*smoothingFactor + v*(1 – smoothingFactor);

The weighting factor for smoothing shall be set to smoothingFactor=0.95 (corresponding to a reduction to below 5% after 60 audio blocks, i.e. 0.32 s for 256 samples block length).

The smoothing is only applied if no teleportation of RI or listener occurred in the previous and current frame. If a teleportation occurred (at least one of previousItemTeleport, itemTeleport, previousListenerTeleport, detectedListenerTeleport == true), the smoothed velocity shall be reset to the new velocity estimate by setting smoothVelocity = v.

For increased robustness of constant velocity RI (isCV == true) against temporal inconsistencies (e.g. due to dropped audio frames), the audio thread timer difference calculated between currentTime and previousTime is checked to be consistent within the audio block size. If the time difference exceeds the audio block size beyond the tolerance of 10 samples, then smoothVelocity shall not be updated.

###### Smoothed distance model

Based on the smoothed velocity model, a smoothed distance model smoothDistance is continuously updated in each audio processing block, as long as no new location update is received. In order to mitigate drift, the model distance is combined with a weighted contribution of the current distance.

smoothDistance = smoothDistance + smoothVelocity;

smoothDistance = smoothDistance * 0.995 + dist * 0.005;

In case a listener teleport was detected as detectedListenerTeleport == true, the distance model is reset to the current distance by setting smoothDistances = dist.

For increased robustness against inconsistencies in the update rate for static location RI that otherwise lead to implausible velocities, the time difference is checked for consistency. If an increase in time difference exceeds a factor of 10, the RI teleportation flag shall be set and the smoothed distance be reset to the current distance.

if ((!isCV) && (deltaT > previousTimeDiff* 10.0f) {

smoothDistances = dist;

itemTeleport = true;

}

###### Delay calculation and variable delay line

The delay dly to be applied by the variable delay line is calculated in samples by dividing the model distance by speed of sound multiplied by sampling rate.

dly = dist / speedOfSound * fs

To comply with the delay line buffer size, the delay is limited between the minimum and maximum delay. The minimum delay minDelay is governed by the algorithmic latency of the delay processing described below. For a sampling rate of 48 kHz, the group delay is twice the value given in samples in LPAA_IIR_48000_18000_2x_GROUPDELAY below. The maximum delay maxDelay is configured by delayBufferSize through lookup in Table 10. If the maximum delay is exceeded, the teleportation flag for the RI is set to prevent interpolation beyond the allowed delay.

if (dly < minDelay {

dly = minDelay;

} else if (dly > maxDelay) {

dly = maxDelay;

itemTeleport = true;

}

The calculated delay for each RI is passed to a variable delay line (VDL) that delays the associated signal(s) by dly samples, including sub-sample interpolation for fractional values of dly. When the delay changes, the VDL interpolates between the previous and the newly specified delay over the duration of one block.

The signal is stored in a ring buffer of length Distance_MaxDelay. One or multiple read cursors can read from the ring buffer and calculate fractional delays using one of the two following methods dependent on the choice signaled in vdlMethod.

Table 147 — Bit assignments to vdlMethod

| bits | vdlMethod |
| --- | --- |
| 0b000 | Direct sound - vdl method is spline Early reflections - vdl method is spline |
| 0b001 | Direct sound - vdl method is linear Early reflections - vdl method is linear |
| 0b010 | Direct sound - vdl method is spline Early reflections - vdl method is linear |
| 0b011 | RESERVED |
| 0b100 | RESERVED |
| 0b101 | RESERVED |
| 0b110 | RESERVED |
| 0b111 | RESERVED |

- Spline method

A third order spline interpolation implemented in Newton Structure, Lamb [4]. With buffer as the ring buffer, rc as the integer read cursor position, and fdly as the fractional part (represented in a number range of 0 to 4096) of dly, the ith output sample is calculated with

k0 = buffer[rc];

k1 = buffer[rc-1];

k2 = buffer[rc-2];

k3 = buffer[rc-3];

b0 = k0;

b1 = k0 - k1;

b2 = k0 - 2 * k1 + k2;

b3 = k0 - 3 * k1 + 3 * k2 - k3;

fd = -fdly / 4096;

output[i] = k0 + 0.5 * (b2 + b3) + (fd - 1.f) * (

b1 + 1/6 * b3 + fd * (

0.5 * b2 + (fd + 1.f) * 1/6 * b3

)

);

- Linear method

With buffer as the ring buffer, rc as the integer read cursor position, and fdly as the fractional part (represented in a number range of 0 to 4096) of dly, the ith output sample is calculated using linear interpolation with

k0 = buffer[rc];

k1 = buffer[rc-1];

fd = -fdly / 4096;

output[i] = k1 + ((k0 - k1) * fd / 4096);

If the listener or RI have been teleported (i.e., the location changed and  teleport == true for at least one of them), the change in the read cursor position shall not be interpolated. In this case, the VDL uses a one-block crossfade between the signal samples at the old and new read cursor position.

The Distance stage creates a new StreamBuffer for each delayed signal, which replaces the signal buffer associated with each RI, to allow processing multiple RIs with the same input signal but different listener-relative locations.

###### Update of state memory

The state variables are updated to save the previous state after each block at the end of the distance stage.

previousTimes = currentTime;

previousDistance = dist;

previousTimeDiff = deltaT;

previousItemTeleport = itemTeleport;

previousListenerTeleport = detectedListenerTeleport;

previousListenerLocation = listenerLocation;

previousListenerTrajTime = currentTime

previousItemLocation = itemLocation;

previousItemTimestamp = itemTimestamp;

The start-up phase shall be handled as a teleportation for the RI and the listener where no previous state is available.

##### Processing in the update thread.

None of the processing described in 6.6.17.3 takes place in the update thread.

#### Distance attenuation due to geometrical spreading

##### Overview

For the distance attenuation due to geometrical spreading, a parametric model is used that takes the size of the sound source into account. The model is based on the physics of real-life sound sources, which may have a different distance attenuation behaviour depending on their size and shape. While a very small point-like source may have a distance attenuation curve that essentially follows the theoretical 1/r point source curve, the distance attenuation behaviour may be very different for non-point like real-life sources.

##### Processing in the audio thread

None of the processing takes place in the audio thread.

##### Processing in the update thread.

###### General model properties

For audio objects without an extent, i.e., point source audio objects, the distance attenuation curve produced by the model is the classical 1/r point source distance attenuation curve.

For making MPEG-I more accessible, the tuning parameter a is introduced for the calculation of the distance-based gain attenuation, changing the distance r to ra. The intended effect is that the depth of the sound scene (comprised of audio objects) can be increased or decreased to the user’s needs. For instance, when a>1 the depth of the scene expands and objects that are further away will become less audible and vanish. In opposite, when a<1, the depth of the scene shrinks and objects that are further away will become more audible.  A value of a=1 preserves the default behaviour. Figure 72 depicts the effect of the exponent a for the three different values 0.7, 1.0, and 1.1.

Note that audio elements, that are authored with the NoDistanceGain authoring parameter are excluded from this processing. The distance exponent a is provided via the parameter distanceExponent via the Accessibility User Interface (See Annex B.5).

When the parameter a is provided via the Accessibility User parameter  distanceExponent Interface (See Annex B.5), the distance r of an RI shall be modified to  r= ra .

Figure 72 — Example for three different distance exponents a

.

For audio objects with an extent, the distance attenuation curve produced by the model depends on the size of the extent, matching the distance attenuation behaviour of a real-life sound source of the corresponding size. Generally, at large distances the distance attenuation behaviour of the extended sound source is essentially the same as that of a point source, with the distance attenuation curve having the corresponding slope of -6 dB per distance doubling, while at smaller distances the slope of the distance attenuation curve of the extended sound source is smaller, i.e., the level increases less steeply when closely approaching the source as it is the case for a point source. In this context, “large” and “small” distances are to be understood relative to the largest size of the extent.

The size of the extent that is used to evaluate the geometrical distance attenuation model is the “apparent” size of the extent as observed from the current observation position. It is expressed by two values, representing the apparent size of the extent in two orthogonal dimensions (e.g. apparent width and height). The extent size that is used as input to the geometrical distance attenuation model is derived dynamically from the audio object’s extent geometry as specified in the audio object’s metadata, and the current observation position relative to the audio object (which also depends on the audio object’s position and orientation).

###### Core geometrical distance attenuation model

The parametric geometrical distance attenuation model approximates the real distance attenuation curve of the extended source by using a different distance gain function in different distance regions, each having a constant slope of -6 dB, -4.5 dB, -3 dB or -0.75 dB per distance doubling, respectively, with the boundaries between the regions being determined by the sizes L1 and L2 of the two dimensions of the apparent extent, as expressed by this formula:

| g=L1×maxL2,L16-0.25×L26-0.375×D-0.125 ; D<L26L1×max⁡(L2,L16)-0.25×D-0.5 ; L26≤D<max⁡(L2,L16) L1-0.25×D-0.75; max⁡(L2,L16)≤D<L1D-1 ; D≥L1 | (348) |
| --- | --- |

Here, g is the distance attenuation expressed as a linear gain, L1 represents the size of the apparent extent in the largest of the two dimensions, L2 represents the size of the apparent extent in the smallest of the two dimensions, and D is the distance to the extent. When plotted on a double-logarithmic scale (logarithmic distance versus dB), the resulting distance attenuation curve is a piecewise-linear function of distance.

As can be seen from (348), for a source without an extent (L1, L2=0) the model reduces to the simple 1/r distance attenuation model of a point source. For a source that only has an extent in one dimension (L2=0, line source) the model reduces to a 3-segment curve with 1/r (-6 dB per distance doubling) point source behaviour at distances larger than the size L1 of the extent, 1/r  (-3 dB per distance doubling) infinite line source behavior at distances smaller than L1/6, and an intermediate slope of -4.5 dB per distance doubling in between. If the line source is infinitely long (L1=∞, L2=0), the model reduces to the 1/r  distance attenuation of an infinite line source at all distances.

###### Additional conditioning of geometrical distance attenuation

The distance attenuation value that is obtained from the core geometrical distance attenuation model described in 6.6.17.4.3.2 is scaled such that it is 0 dB at the reference distance specified for the audio object. This is done by evaluating the core geometrical distance attenuation model at the reference distance, and then using the obtained value to scale the value obtained for the current observation distance.

To prevent excessive distance gain when coming very close to a source, the distance attenuation curve from the core geometrical distance attenuation model is only applied from a certain threshold distance from the extent onwards. For distances within this threshold distance, the distance gain is kept constant to the value at the threshold distance (default: 0.2 m).

To prevent excessive gains for listener voice reflections when e.g. approaching a wall, the threshold distance for RI of type Reflection derived from items of type ListenerVoice shall be set to 1.0 m.

To ensure a more realistic distance attenuation of implicit Portals, which are radiating late reverberation sound instead of dry direct sound, an additional Portal Factor, Fportal, is calculated based on (349), where Vportal is the volume of the portal geometry. This factor shall not be applied in explicit portal mode (isExplicitPortalMode = true).

| Fportal=min15, 0.05*Vportal | (349) |
| --- | --- |

#### Medium attenuation

##### Processing in the audio thread

None of the processing takes place in the audio thread.

##### Processing in the update thread.

Medium attenuation describes the frequency-dependent attenuation of sound sources due to the geometrical spreading of energy. The distance stage models this effect by modifying the eq field of relevant RIs.

The EQ values for each band are calculated according to ISO 9613-1 (“Attenuation of sound during propagation outdoors — Part 1: Calculation of the absorption of sound by the atmosphere”), using the band center frequency (Table 78), the local configuration parameters (6.4.2.7) for Temperature and Humidity, and an atmospheric pressure of 101.325 kPa. This results in a low-pass effect for audio elements that are far away from the listener.

### Directional Focus

#### General

Intended as a functionality for improving accessibility, the directional focus is meant to attenuate distracting sounds from directions outside a spatial region of interest (see Figure 73). The focus is radial symmetric with one “main lobe” region. Its damping behavior is configurable with three parameters provided by the Accessibility User Interface (see Annex B.5). The default direction steers towards the frontal viewing direction of the user but can also be re-oriented to other directions, e.g., to enable control through other services or modalities (e.g., eye tracker, handheld controller, etc.).

- Audio elements, that are signalled to be part of the Listener-relative Coordinate System (LCS, see clause 6.4.1.2.2.3) and are therefore associated with the listener shall not be processed with the Directional Focus.

- RIs of reflection-type are excluded from this processing.

- Audio elements, that are signalled with the NoDistanceGain or NoDirectionalFocus authoring parameter are excluded from this processing. The direct sound of an audio element of type ListenerVoice is excluded from this processing

Figure 73 – Visualization of Directional Focus Parameter

directionalFocusAperture		Opening angle of the directional focus’ main lobe in degree.

startStopbandAngle	Angle in degree where the stopband starts: directionalFocusAperture+directionalFocusTransitionWidth

stopbandGainLinear 	Linear gain computed as

pow(10.0, -0.05 * directionalFocusStopbandAttenuationDb)

coefA	Variable to compute the attenuation in the transition band:

-0.05directionalFocusStopbandAttenuationDb startStopbandAngle - directionalFocusAperture

DirYawRad    	Yaw angle of the primary direction of the directional focus with respect to the frontal head orientation in degree.

DirPitchRad   	Pitch angle of the primary direction of the directional focus with respect to the frontal head orientation in degree.

#### Processing in the audio thread

None of the processing takes place in the audio thread.

#### Processing in the update thread

The directional focus attenuates the frequency-independent gain of a RI. For this, the RI location in global coordinates is transformed to listener-relative polar coordinates with θRI and ϑRI being the azimuth and elevation angles of the RI direction in radians in the listener-relative polar coordinates.

The angular distance angDist between the direction of the RI and the viewing direction is computed as:

angDist = cos-1(sinDirPitchRad* sinϑRI   + cosDirYawRad   * cosϑRI*cos(DirYawRad - θRI)* 180π

based on angDist, the gain of the RI is scaled as follows:

if (angDist > directionalFocusAperture) {

if (angDist >= startStopbandAngle) {

gain *= stopbandGainLinear;

} else {

gain *= pow(10.0, coefA * (angDist - directionalFocusAperture));

}

}

### Consolidation of Render Items

#### General

The Consolidation stage combines RI with similar localization properties, to reduce the total number or RI in the renderer pipeline and therefore computational complexity. RI for which the differences in perceptual localization properties are below a given threshold are identified via a psychoacoustic model. Based thereon, groups of RI within these thresholds are selected via a computationally efficient clustering algorithm. The RI within each group are consolidated into a common representative RI. For temporal stability, the assignment of RI is optimized to avoid unnecessarily frequent re-assignment and crossfades are applied if the assignment of RI changes.

#### Data Elements and Variables

p		user-relative RI location as 3D coordinate vector

xp, yp, zp	x,y,z components of RI location vector

rp		radius distance of RI location to user

q 		RI location in perceptual coordinates as 3D coordinate vector

uq, vq, wq	components of perceptual coordinate vector

cLR		perceptual weighting factor for left-right differences, shall be 20

cFB		perceptual weighting factor for front-back differences, shall be 20

cUD		perceptual weighting factor for up-down differences, shall be 20

cr		perceptual weighting factor for radius differences, shall be 0 when

consolidation is positioned after distance stage.

cEQ		perceptual weighting factor for equalizer differences, shall be 1

cER		perceptual weighting factor for early reflection, shall be 6

rPCS		radius distance component in perceptual coordinates

rref		reference radius for perceptual distance mapping, shall be 0.2m

EQlin	RI equalizer magnitude gains in linear domain as vector

EQlog	RI equalizer magnitude gains in logarithmic domain as vector

b		equalizer band index

isER		flag indicating that an RI is of type Reflection

i, j	I	ndex variables referring to RI indices

E(i)		Energy of RI of index i

itemGain(i)	item gain of RI of index i

dq(i,j)	perceptual localization difference between RI of index i and j

dEQ(i,j)	perceptual equalization difference between RI of index i and j

dERi,j	perceptual reflection type difference between RI of index i and j

dtotali,j	total perceptual difference between RI of index i and j

thrPCS	threshold for perceptual differences from bitstream, default value 5,

can be adjusted via bitstream in range 0 to 31

isBelowThr(i,j)	flag indicating that RI of index i and j are within perceptual thresholds

Nin		number of input RI into the consolidation algorithm

k		number of consolidated RI clusters from clustering in current frame

kprev		number of consolidated RI clusters from clustering in previous frame

m	I	ndex of consolidated RI cluster

centroids(m)	centroid location of consolidated RI in perceptual domain

assigned(m)	number of RI assigned to centroid of index m

memberships(i)	membership of RI, i.e. RI of index i is assigned to centroid(memberships(i))

Eperm	Energy permutation matrix of size k x kprev

rout		radial distance of output centroid location

pout		location of consolidated RI centroid as 3D coordinate vector

EQcons	equalizer gains of consolidated RI in linear magnitude domain as vector

#### Stage description

##### Overview

The Consolidation stage collects all RI types that may be consolidated and converts the RI metadata into a perceptual representation. This representation enables the calculation of pairwise differences between RI in order to determine pairs for which the differences are below a given perceptual threshold. Based on this threshold detection, a greedy clustering algorithm is applied to select groups of RI to be combined into clusters. For improved temporal stability, the result of the clustering algorithm is optimized to minimize re-assignments due to permutation changes. The RI within each group are consolidated into a common representative RI and the corresponding audio signals are combined via a downmixing. To prevent discontinuities if the assignment of RI changes, crossfades are applied when necessary. The workflow is illustrated in the block diagram in Figure 74.

Figure 74 – Block diagram of Consolidation of Render Items

##### Update Thread Processing

The consolidation stage considers render items that represent point sound sources, therefore, RI of type Nondiegetic, Pannable, Reverb, HOA, ListenerVoice, Airflow, Downmix, and RI with extent are omitted.

If consolidationThreshold is set to 0, no RI will be combined. Therefore, in this case, the Consolidation stage shall omit processing of all RI to avoid unnecessary distance calculations.

###### Mapping of RI properties to Perceptual Coordinate System

To enable efficient calculation of perceptual differences, the location of each RI is transformed and scaled into a listener-relative Perceptual Coordinate System (PCS) by the following processing.

The RI location is transformed into user relative coordinate space to obtain the user-relative location

p = (xp, yp, zp). Spatial hearing is based on different localization cues for horizontal and vertical direction, as well as for distance. Therefore, the location is separated in a radial distance component rp = |p| and a normalized position p’ = p/r on the unit sphere, similar to a polar coordinate representation. To reflect human hearing’s different sensitivity for different directions, the location is scaled into a perceptual coordinate representation q by weighting constants for left-right cLR, front-back cFB and up-down cUD directions as

| q= uqvqwq=1rxp⋅cLRyp⋅cUDzp⋅cFB. | (350) |
| --- | --- |

The distance component rPCS is modeled in analogy to the perception of distance-based gain differences by a logarithmic transformation of the radius in relation to the reference distance rref as

| rPCS= 20 log10rrref . | (351) |
| --- | --- |

This representation gives a generalized perceptual model for free-field localization of point sources. However, further processing is applied in the renderer to model further effects of sound propagation which must be considered for the perceptual model as well.

Individual equalizer (EQ) curves are applied per RI e.g., to represent directivity and propagation dampening. The shape of the EQ given by the magnitude filter curve EQlin for each of the Nb octave band b is represented by the log-magnitude curve. Any full-band offset of the EQ curve can be compensated by a linear gain and is therefore removed.

| EQ'log(b)= 20 log10EQlin(b) | (352) |
| --- | --- |
| EQlogb= EQ'logb-1NbbEQlog'(b). | (353) |

Furthermore, for RI of type Reflection, a dispersion filter is applied by the spatialization, thus introducing perceptual difference between different RI types. This is considered by a flag isER set as:

| isER =1, if item type is Reflection0, otherwise | (354) |
| --- | --- |

###### Perceptual Differences and Thresholds

The CoRI is based on a pair-wise estimation of the perceptual difference between the properties of two RI. If the perceptual difference is below an appropriate perceptual threshold, the corresponding RI can be combined. Since the consolidation of two or more RI into one RI results into one combined, position, EQ curve, and item type, all differences between these individual factors contribute to the overall perceptual differences. Therefore, a threshold for a weighted, overall sum of the perceptual differences is considered, rather than individual thresholds.

The PCS representation enables to estimate perceptual differences between the localization from simple distance calculations in the PCS coordinates. The perceptual difference dPCS (i,j) between the directional properties two RI of index i and j is calculated as

| dq(i,j)=qi-qj. | (355) |
| --- | --- |

The difference in distance perception is calculated as

| dr(i,j)=rPCS, i-rPCS,j | (356) |
| --- | --- |

and the average difference in EQ curves as

| dEQ(i,j)=1NbbEQlog,ib- EQlog,j(b). | (357) |
| --- | --- |

Combining items with different isER property enables/disables the application of the dispersion filter, therefore the perceptual difference between ER and non-ER items is considered as

| dER(i,j) =1, if isERi ≠isERj0, otherwise . | (358) |
| --- | --- |

The individual perceptual differences are combined into a total difference dtotal as we weighted sum:

| dtotali,j=dq(i,j)+cr⋅dr(i,j)+cEQ⋅dEQ(i,j)+cER⋅dER(i,j) | (359) |
| --- | --- |

Note: Since the location difference dq already includes its individual weighting factors cLR, cFB, and cUD, it is not weighted twice.

To determine if two RI may be combined, the perceptual distance is compared against the perceptual threshold obtained from the bitstream as thrPCS = consolidationThreshold to yield a flag isBelowThr(I,j) as

| isBelowThr(i,j) =1, if dtotali,j<thrPCS0, otherwise . | (360) |
| --- | --- |

###### Clustering Algorithm

The goal of CoRI is to determine clusters of RI with small perceptual differences that can be combined in order to reduce the number of RI in the pipeline and therefore complexity, while maintaining low complexity for the clustering algorithm itself. Therefore, a greedy clustering algorithm is applied. The approach is, starting at the RI with the lowest index, to determine for each RI, which of the other RI are within the perceptual thresholds and not already assigned to another cluster. All corresponding RI are combined and marked as already processed. This is repeated for the remaining render items, until no further differences below the perceptual thresholds remain.

The clustering algorithm is defined by the following pseudo-code. Here for the algorithm input,
Nin indicates the total number of input RI, q[i] the input RI location in PCS coordinates, E[i] the energy of the RI. For the algorithm output, centroid indicates the combined centroid position, assigned(i) indicates the number of assigned RI for each centroid, memberships(i) to which centroid each RI is assigned to. The algorithm is initialized with each RI being one individual cluster.

for (i = 0; i < Nin; i++) {

memberships[i] = i;

assigned[i] = 1;

centroids[i] = q[i];

E[i] = itemGain[i]^2;

}

for (i = 0; i < Nin; i++) {

for (j = i + 1; j < Nin; j ++) {

if ( (assigned[i] > 0) && (assigned[j] > 0) && (isBelowThr(i,j) == 1)) {

memberships[j] = i;

assigned[i] = assigned[i] + assigned[j];

assigned[j] = 0;

centroids[i] = (centroids[i]*E[i] + centroids[j]*E[j])/(E[i]+E[j])

centroids[i] = q[i]*E[i] + q[j]*E[j];

E[i] = E[i] + E[j];

E[j] = 0;

}

}

}

The clustering is applied in-place, leaving empty centroid indices with assigned[i]=0, and a total number of k centroids with assigned[i]>0. The resulting k centroid positions and memberships of index m are collected and re-assigned as defined by the following pseudo-code:

m = 0;

for (i = 0; i < Nin; i++) {

if (assigned[i] > 0) {

centroidsOut[m] = centroids[i];

membershipCollected[m] = membership[i];

m++;

}

}

for (i = 0; i < Nin; i++) {

for (m = 0; m < k; m++) {

if (membership[i] == membershipCollected[m]) {

membershipOut[i] = m;

}

}

}

###### Cluster Order Permutation Optimization

The index order of the obtained centroids depends on the member RI with the lowest index. To avoid permutations of centroid assignments due to small re-assignment of individual RI, the permutation order of the assigned output clusters is optimized to improve temporal stability. The goal of the optimization is to keep as much input signal energy assigned to the downmix signal with the index where it was assigned in the previous frame. This is achieved by calculating an energy permutation matrix to determine how much energy is re-assigned between clusters in consecutive frames and iteratively selecting the maximum entries to maximize the amount of energy that is kept assigned to the same cluster as before. The permutation optimization shall be performed as defined by the following pseudo-code.

The energy permutation matrix Eperm of size k x kprev is initialized to zero and then filled by accumulating the RI energies, based on their current membership membershipOut and their previous membership membershipPrevious for the k current and kprev previous number of clusters.

for (i = 0; i < Nin; i++) {

Eperm[membershipOut[i]][membershipPrevious[i]] += E[i];

}

The permutation is optimized by a greedy approach, iteratively picking the matrix entry with the highest energy, from the rows/columns of permutations that have not yet been assigned.

for (m = 0; m < k; m++) {

[rowMax, colMax] = argMax(Eperm)

permutation[colMax] = rowMax;

permutedClusters[rowMax] = 1;

Eperm(rowMax, : ) = 0;

Eperm(:, colMax ) = 0;

}

Finally, the permutation is applied to obtain the permutated memberships and centroid positions, (replacing the variables centroids and memberships for improved readability).

for (m = 0; m < k; m++) {

centroids[m] = centroidsOut[permutation[m]];

for (int i = 0; i < numObjIn; i++) {

if (membershipOut[i] == permutation[m]) {

memberships[i] = m;

}

}

}

###### Inversion of Perceptual Mapping

After the consolidation is performed in the perceptual domain, the output RI properties are transformed back into geometric coordinates.

The radius is converted back from logarithmic to linear coordinate representation as

| rout= rref⋅10rPCS20 | (361) |
| --- | --- |

and the user-relative, geometric coordinates pOut are calculated by inverting the perceptual weighting and the normalization to the unit sphere as

| pout=rout⋅uq/cLRvq/cUDwq/cFB. | (362) |
| --- | --- |

For the combined RI, the Equalizer Curve is calculated as the RMS average of the contributing RI as weighted by the contributing RI energies in relation to the cluster’s total energy Em

| EQcons,mb=1EmiEi⋅EQlin, ib2 ∀ i with membershipsi=m. | (363) |
| --- | --- |

###### Assignment and Deactivation of Render Items

When two or more input RI are assigned to one centroid by the clustering algorithm, they shall be combined into a consolidated RI with the location and EQ properties of the obtained centroid and the original RI are deactivated. Though the consolidation is time-variant, it is expected to remain identical or at least similar, as long as there are only small changes in the user and sound source positions. Therefore, a consolidated RI that has been created shall be kept persistent and active as long as it has input RI assigned to it and shall only be deactivated when no input RI are assigned to it anymore. Vice versa, input RI that are no longer assigned to a centroid shall be re-activated.

For input RI of type Reflection, the binaural spatializer applies a dispersion filter. The consolidation allows, though with a given penalty, the combination of reflection and non-reflection RI. In this case, the perceptual impact of applying the dispersion filter to a primary item is assumed to be more detrimental to sound quality than the impact of not applying the dispersion filter to a reflection RI that is combined with other non-reflection RI. Therefore, the default item type of the consolidated RI shall be of type Representative. If, and only if, all members of a consolidated RI are or type Reflection, then also the consolidated RI shall be of type Reflection, and thus enable the dispersion filter.

##### Audio Thread Processing

In the Audio Processing Thread, the individual input signals of the RI that have been consolidated are summed up into the respective target consolidated RI. In case the assignment changes, a short crossfade of one audio block length is applied in order to prevent signal discontinuities.

When the membership of an RI changes, the existing signal is cross-faded into the newly assigned consolidated RI signal over the course of one audio block. For render items that have previously not been consolidated, the fade out window is applied in the original RI’s signal. Conversely, for RI that are no longer part of a consolidated RI and are reactivated, a fadeout is applied in the previously assigned consolidated RI and a fade in is applied to the re-activated original RI’s signal. For crossfading, the fade in and fade out window shapes are taken from the Fade stage, as defined in Section 6.6.22 Fade. Since the activation and deactivation of RI by the consolidation corresponds to a re-assignment of signals and is crossfaded within the CoRI stage, the Fade stage shall not apply any additional fade in or fade out in case RI are activated or deactivated by the CoRI stage.

### Equalizer (EQ)

#### General

The Equalizer (EQ) stage applies frequency-dependent gains to all relevant audio signals after the frequency-dependent attenuation has been accumulated for all contributing acoustic effects (e.g. occlusion, diffraction, reflection, directivity, medium attenuation) by preceding stages. For accumulation, all frequency-dependent gain values are interpolated from their given original resolution to the default resolution of N=31 third-octave bands given in  using linear interpolation. Outside of the interpolation range, a hold of the first and the last supporting point value is done, respectively.

The gain for N bands with fixed center frequency is stored in the eq metadata element of each RI (6.4.2.4). The EQ stage implements a minimum phase FIR filter design based on N=31 third-octave bands and center frequencies given in . The EQ stage further applies the frequency-independent gain of each RI to the associated signals. For RI of type ListenerVoice, the EQ stage further applies the frequency-independent directGainDb as gain = gain*10^(-0.05*directGainDb).

Table 148 – Center frequencies of EQ bands

| Band | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Freq. [Hz] | 19.69 | 24.80 | 31.25 | 39.37 | 49.61 | 62.50 | 78.75 | 99.21 |
| Band | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Center Freq. [Hz] | 125.00 | 157.49 | 198.43 | 250.00 | 314.98 | 396.85 | 500.00 | 629.96 |
| Band | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Center Freq. [Hz] | 793.70 | 1000.00 | 1259.92 | 1587.40 | 2000.00 | 2519.84 | 3174.80 | 4000.00 |
| Band | 25 | 26 | 27 | 28 | 29 | 30 | 31 |  |
| Center Freq. [Hz] | 5039.68 | 6349.60 | 8000.00 | 10079.37 | 12699.21 | 16000.00 | 20158.74 |  |

#### Signal processing

The EQ filters designed in the EQ stage have a length of L=1024 for primary RIs, and a length L = 256 for secondary RIs of ItemType::Reflection. For obtaining these filters, the gains contained in the eq metadata element are first limited to a minimum gain of 0.0005 and are subsequently interpolated from the N bands given in eq to a real-valued, one-sided magnitude spectrum Mω of size of  K=L2+1 using linear interpolation. Outside of the interpolation range, a hold of the first or last supporting point value is done, respectively.

The obtained magnitude spectrum Mω with bin frequency ωi with i∈1:K is smoothed by an envelope follower:

for (j=0; j<K; j++) {

env[j] = 0.25+0.25*(j+1)/K;

}

for (j=1; j<K; j++) {

M[j] = max(M[j],env[j-1]*M[j-1]);

}

for (j=K-2; j>=0; j--) {

M[j] = max(M[j],env[j+1]*M[j+1]);

}

Next, the smoothed magnitude spectrum is convolved with the 3-tap transfer function of a Hamming window. Note, that for the computation the spectrum shall be padded on both ends with one mirrored value. Both padded values are discarded after computation.

| MWin=M*WHamming;WHamming=[0.2355 0.55295 0.2355] | (364) |
| --- | --- |

To minimize group delay in the downstream signal processing, a minimum-phase conversion using the cepstrum-windowing method (Oppenheim [6] pp.788) is performed: the one-sided smoothed magnitude spectrum is squared and extended to a double-sided spectrum MWin of length L, and its cepstral coefficients ck for k∈1:L are obtained as

| ck=1Li=1LlnMWin2ωiejωik/L | (365) |
| --- | --- |

The cepstral coefficients are processed by a cepstral window that multiplies c0with 0.5 and ck for k∈L-12+1:L with 0. The final minimum-phase filter Hωi is given as

| Hωi=expk=1(L-1)/2cke-jωik/L+c02 | (366) |
| --- | --- |

The impulse response of the FIR filter of length 1024 or 256, respectively, are obtained by the IFFT of Hω.

#### Stage description

For each RI, the EQ stage designs an equalizing FIR filter according to (6.6.20.2) and prepares it for a filter exchange according to  (A.13) in the update() routine. A separate FIR filter instance is created for each channel of the signal(s) associated with the RI.

In the processBlock() routine, each single-channel signal is filtered according to (A.13) and the gain of the corresponding RI is multiplied with the signal. If gain has changed from the previous block, the multiplication value is interpolated linearly to avoid discontinuities by large changes in the amplitude of the signal.

### Low-Complexity Early Reflections (LC-ERs)

#### General

The impulse response in an indoor acoustical environment comprises of direct sound, early reflections (ERs) and late reverb. In the high-quality early reflections (ER stage) implementation, the direct sound and every single early reflection dynamically change their individual direction and distance to the listener when the listener and/or the sound source is moving in the environment. This implementation accurately models physics in real world and thereby provides highest perceptual quality.

However, in most cases humans mainly localize a source based on the direct sound (law of the first wave front, precedence effect). In terms of perception, the ER essentially just fill the gap between direct sound and late reverb and provide some additional cues about the room properties in addition to the main information provided by the late reverb.

For LC-ER, only one common reflection pattern is applied for all primary sound sources within a scene. Moreover, the LC-ER stage needs only one common integer delay line for all ERs, because the distance delays between listener and ERs are fixed. Consequently, the LC-ER stage is situated at the end of the renderer pipeline, see Figure 4, enabling most efficient computation

The LC-ER stage shall be activated if the bitstream flag useLowComplexityER in the mpegSceneConfig() is set to true. In this case, the ER stage shall be skipped in the renderer pipeline. Otherwise, if useLowComplexityER is set to false, the ER stage is engaged and the LC-ER stage shall be skipped in the renderer pipeline.

#### Data elements and variables

NumER	Number of LC-ERs

distFactor	Distance factor between indoor LC-ERs

initDelay	Initial delay to the first indoor LC-ER [ms]

ampFac	Amplitude factor

β1(n)	Azimuth angles of first indoor spiral pattern [rad]

β2(n)	Azimuth angles of second indoor spiral pattern [rad]

r1(n)	Radius of first indoor spiral pattern [m]

r2(n)	Radius of second indoor spiral pattern [m]

slDistance	Source listener distance [m]

absorption	Estimated absorption of the indoor environment

ampCorrection1	Amplitude correction of first indoor spiral pattern

ampCorrection2	Amplitude correction of second indoor spiral pattern

ampAtt 	Amplitude attenuation factor

amp1	Amplitude of first indoor spiral pattern

amp2	Amplitude of second indoor spiral pattern

roomDim	Estimation of the room dimension

βoffs	Angular offset dependent on listener position

LCER_r(n)	Radii of LC-ERs

LCER_ampn	Amplitudes of LC-ERs

lcEarlyBoundingBox()	Bounding box for current set of LC-ER parameters

lcEarlyPreDelay1	preDelay to outdoor reflection number 1 and 3

lcEarlyPreDelay2	preDelay to outdoor reflection number 2 and 4

lcEarlyAmpFac1Db	log amplitude correction beyond 1/r for reflection 1 and 3

lcEarlyAmpFac2Db	log amplitude correction beyond 1/r for reflection 2 and 4

ampFac1	linear amplitude correction beyond 1/r for reflection 1 and 3

ampFac2	linear amplitude correction beyond 1/r for reflection 2 and 4

rstart1	Radius start value for outdoor reflection number 1 and 3 [m]

rstart2	Radius start value for outdoor reflection number 2 and 4 [m]

EarlyTuningGain 	Gain for ERs

lcEarlyRelativeTuningGain	Relative tuning gain for LC-ERs

lcEarlyMasterGain 	Absolute gain for LC-ERs

envCrossfadeFactor 	Environment crossfade factor

distLimit	Distance limit [m]

distanceToPortal	Distance to portal [m]

LCER_totalGain	LC-ER total gain

signalsIn(i)	Input signal per sample

dmxWeight	Downmix weight for sources not processed by Distance stage

dmx(i)	Weighted downmix for samples for all sound sources

slDistGain	Source listener distance gain

reflectionOut(n,i)	Individual LC-ER reflection

n		Reflection index

m		Input signal index

i		Sample index

#### Stage description

##### Implementation of LC-ERs

There are different implementations for in- and outdoor LC-ERs. Figure 75 gives an overview of the implementation of LC-ER algorithm in en- and decoder/renderer. The structogram is structured in three rows: encoder, bitstream and decoder. It has two main columns: the left is for indoor scenes, using the predefined indoor ER spiral pattern, the second is for outdoor scenes, using the predefined outdoor ER pattern. Indoor scenes are defined by an acoustic environment. All other scenes are handled with the outdoor LC-ER pattern. The four parameters for the outdoor pattern can either be defined in the EIF file or they can be calculated out of a geometry analysis in the encoder.

Figure 75 – Implementation of LC-ER algorithm in en- and decoder/renderer

Because the LC-ER patterns are not dependent on any mesh information, they can also be used for voxel-based scenes.

The following LC-ER calculations are executed in every frame.

##### Implementation of Indoor LC-ER Pattern

Figure 76 shows the indoor LC-ER pattern plotted a) over time, b) across location (top view). In particular, the level values over time of the ERs are indicated as blue dots in (a) that are calculated according to the algorithm explained below. In (b), the crosses mark the apparent locations of the ERs from a top view. The green and blue lines indicate the two spirals calculated as explained below. It is a pattern with reflections only in the horizontal plane at ear height. It is just “moving” with the lateral position of the listener. It is not dependent on the orientation of the listener.

Figure 76 – Indoor LC-ER pattern plotted a) over time, b) across location (top view)

The predelay time to the start of the late reverb determines the number of ERs, numER in eq (367), the factor determining their spacing, distFac in eq (368) and their initial delay, initDelay in eq (369). The room’s RT60(1kHz) determines the internal amplification factor, ampFac in eq (370). This predelay and RT60 time are obtained from the Reverb stage (6.6.4.2): predelayRender, T60,b.

| NumER/2=floor(3∙lnpredelay2) | (367) |
| --- | --- |
| distFactor=2, predelay≤150 ms3, predelay>150 ms | (368) |
| initDelay =15, predelay≤150 ms30, predelay>150 ms | (369) |
| ampFac =0.5, RT601kHz≤0.35 s 1, 0.35s<RT601kHz≤4 s1.5, RT601kHz>4 s | (370) |

Azimuth angles β calculation of ER positions for the two spirals are defined in eq (371) and (372).

| β1(n)=n∙π3-π18, n = [1:NumER/2] | (371) |
| --- | --- |
| β2(n)=π8+n∙π3-π18, n = [1:NumER/2] | (372) |
| Radius calculation of ER positions for the two spirals are defined in eq (373) to (375), with base=1.85, c msspeed of sound, n = [1:NumER/2]. |  |
| r1base=distFactor∙base2∙βα1(1)/π | (373) |
| r1(n)=initDelay∙c1000+distFactor∙base2∙βα1(n)/π-r1base | (374) |
| r2(n)=initDelay∙c1000+distfactor∙base2∙β2(n)/π | (375) |

The amplitudes of the LC-ERs are adapted by the absorption factor and the source-listener distance slDistance is calculated as the Euclidean distance between the item location and the current listener location.

| slDistance = abs(itemLocation – listenerLocation) | (376) |
| --- | --- |

When multiple sound sources are within a scene, slDistance for the closest source is used for generating the overall pattern. The absorption influence is taken from the RT60 values and room volume V [m3], room surface A [m2], eq (377) to (379).

| absorption=0.163∙V/(A∙0.5∙(RT601kHz+RT604kHz) |  |  | (377) |  |
| --- | --- | --- | --- | --- |
| ampCorrection1n=ampFac∙1-absorptionr1n⋅slDistance | (378) |  |  |
| ampCorrection2n=ampFac∙1-absorptionr2n⋅slDistance | (379) |  |  |
| The amplitude of the pattern is limited to maximum 0.7 by applying an attenuation factor based on the amplitude of the first reflection, (380) to (382). |  |  |  |
| ampAtt =0.7ampCorrection11, ampCorrection11>0.7 1, otherwise | (380) |  |  |
| amp1n= ampCorrection1n*ampAtt |  | (381) |  |
| amp2n= ampCorrection2n*ampAtt |  | (382) |  |

To include some variation into the pattern when the listener is moving, the pattern is rotated around the listener dependent on the listener position. The pattern is rotated by an angle offset βoffs which is calculated from the listener coordinates in the horizontal plane (listenerX, listenerY) in relation to the room dimensions estimated from the initDelay [ms] as follows (383) to (386).

The two spiral arms of the reflection pattern are combined and mapped into output arrays as follows.

| roomDim=initDelay∙c1000 | (383) |  |
| --- | --- | --- |
| βoffs=listenerXroomDim+listenerYroomDim∙π4 | (384) |
| β'1 (n)=β1(n)+βoffs | (385) |
| β'2n=β2n+βoffs | (386) |

The two spiral arms of the reflection pattern are combined and mapped into output arrays as follows for n = [1:NumER/2].

| LCER_r(n)=r1(n) | (387) |
| --- | --- |
| LCER_r(n+NumER/2)=r2(n) | (388) |
| LCER_ampn=amp1n | (389) |
| LCER_ampn+NumER/2=amp2n | (390) |

The reflection positions are converted from listener-relative polar coordinates into Cartesian world coordinates as output positions for the generated render items.

##### Implementation of Outdoor LC-ER Pattern

Specifically for outdoor scenes, a pattern with four roughly cross-positioned ERs is designed, see Figure 84. Such a sparse pattern does not produce an indoor perception, but it is also more than just the direct sound only. The pattern is highly adjustable in time and in level. With this the scene author has a high degree of freedom to adjust the perception of the pattern to individual scenes. The pattern is independent from the position of the listener and is not taking into account any geometry of the scene. With this the high complexity reduction compared to ERs of the regular ER stage is achieved.

Figure 77 – Outdoor LC-ER pattern plotted a) over time, red circle normalized direct sound, blue circles reflections; b) across location (top view), green circle listener position, red circle source position

The pattern consists out of four ERs, defined by the bit stream parameters:

lcEarlyPreDelay1		preDelay to reflection number 1 and 3, default 60 ms

lcEarlyPreDelay2		preDelay to reflection number 2 and 4, default 100 ms

lcEarlyAmpFac1Db 	log amplitude correction beyond 1/r for reflection 1 and 3, default 0 dB

lcEarlyAmpFac2Db 	log amplitude correction beyond 1/r for reflection 2 and 4, default 0 dB

These parameters can be obtained e.g. by an encoder geometry analysis as described in Annex B.2.5 or be set manually by the scene content creator via EIF parameters. This allows the full freedom to adapt to every individual scene.

The bitstream parameters are converted to linear gains by (391) and (392)

| ampFac1=10^(0.05⋅lcEarlyAmpFac1Db) | (391) |
| --- | --- |
| ampFac2=10^(0.05⋅lcEarlyAmpFac2Db) | (392) |

The four angles stay constant: β(n) = [0.44,1.84,3.2,4.7], n=[1:4]. They are not dependent on listener orientation. There is also no pattern rotation for outdoor LC-ERs.

The distances of the reflections to the listener are calculated by (393) until (398).

| rstart1=0.001∙lcEarlyPreDelay1∙c | (393) |  |  |
| --- | --- | --- | --- |
| rstart2=0.001∙lcEarlyPreDelay2∙c | (394) |  |  |
| LCER_r(1)=0.7∙rstart1 | (395) |  |  |
| LCER_r(2)=0.9∙ rstart2 | (396) |  |  |
| LCER_r(3)=1.1∙ rstart1 | (397) |  |  |
| LCER_r(4)=1.3∙ rstart2 | (398) |  |  |
| The amplitudes are calculated by (399) until (402). |  |  |  |
| LCER_amp(1)=ampFac1/(rOutdoor(1) ) | (399) |  |  |
| LCER_amp(2)=ampFac2/(rOutdoor(2) ) |  | (400) |  |
| LCER_amp(3)=ampFac1/(rOutdoor(3) ) |  | (401) |
| LCER_amp(4)=ampFac2/(rOutdoor(4) ) |  | (402) |

##### Selection of active LC-ER Environment

If the listener is within a part of the scene that has a defined Acoustic Environment, the indoor reflection pattern is applied, based on the parameters provided by the Reverb stage.

If the listener is within an outdoor section of the scene, the outdoor reflection pattern is applied. To determine the parameters for the outdoor pattern generation, it is checked for the environments in the payloadLcEarlyReflections() if the listener position is contained in the respective environment’s lcEarlyBoundingBox() geometry. If the listener is inside one bounding box, the parameters for the corresponding environment are selected. If the listener is within multiple, overlapping bounding boxes, the environment with the highest lcEarlyEnvironmentPriority is selected. If no outdoor LC-ER environment has been transmitted for the current listener location, the default pattern parameters are used.

##### Application of Tuning Gain and Environment Crossfade Factor to LC-ER Gain Calculation

There are several gain factors involved in the final LC-ER level.

The scene dependent tuning gain EarlyTuningGain that is set by the scene author for ER is also applied for LC-ER. Additionally, a factor to align the level of the LC-ER relative to ER is applied as lcEarlyRelativeTuningGain = 0.7. This combined, constant level adjustment is calculated per scene as

| lcEarlyMasterGain = EarlyTuningGain ∙ lcEarlyRelativeTuningGain | (403) |
| --- | --- |

To mitigate discontinuities at the transition between acoustic environments, a crossfade is applied when a listener is close to a portal. Based on the distance to the closest point of the portal, a position-dependent factor envCrossfadeFactor is calculated with distLimit = 0.2 m as follows.

| envCrossfadeFactor =distLimit-distanceToPortaldistLimit , for distanceToPortal≤distLimit 1 , otherwise | (404) |
| --- | --- |

Total gain of LC-ER items is calculated from the individual reflection pattern factors, the tuning gain and the environment crossfade as

| LCER_totalGain(n) = LCER_amp(n)∙ envCrossfadeFactor ∙ lcEarlyMasterGain | (405) |
| --- | --- |

##### Signal Processing of LC-ERs

###### Overview

The LC-ER stage calculates a downmix of all processed sound sources that feeds into the individual reflections of the reflection patterns defined in sections 6.6.21.3.2 and 6.6.21.3.3. The LC-ER stage is located in the renderer pipeline behind the Distance stage and EQ stage, therefore the input signals have already been processed in terms of delay and level alignment dependent on the source-listener distance.

The delay times for the individual reflections within a pattern are fixed (i.e. independent of the source-listener distance) and therefore obtained via one integer delay line with multiple taps.

###### Summation of Several Direct Sources into one LC-ER Pattern

All sound sources are summed before they are fed into the LC-ER pattern. Due to the positioning of the LC-ER stage, the direct sound signals entering the LC-ER stage are already adjusted in delay and level according to their individual source listener distance by the preceding stages (distance, EQ).  Therefore, the contribution of the individual sound sources to the summation is dependent on the individual listener to direct source distance and this can vary over time, see Figure 78. The LC-ER pattern is applied as a relative level and delay adjustment to the sum signal. The closest listener to direct source distance determines the shape of the pattern.

Figure 78 – Summation of several audio sources into one ER pattern

For each frame, a weighted Downmix dmx for samples of index i of all sound sources of index m that feed into the reflection pattern is calculated.

| dmxi=msignalsInm,i⋅dmxWeightm | (406) |
| --- | --- |

For extended sound sources, which are not processed by the Distance stage, a weighting is applied to align the source-listener distance based gain, which is calculated based on the slDistance by the renderer’s calculateDistanceGain()function.

| dmxWeightm= =slDistGain⋅0.1, &for extended sound source items1, &otherwise | (407) |
| --- | --- |
| For primary items of type ListenerVoice, the associated listener voice early reflection gain gListenerVoice shall additionally be applied. It is computed as: |  |
| gListenerVoice = 10(0.05*erGainDb) | (408) |

###### Output Signals for LC-ER render items

All render items are fed with signals from the downmix, by using multiple taps in an integer delay line and individual amplitude factors for each reflection item.

The integer delay of the signals for the reflection render items of index n are calculated from the reflection radius.

| intDelayn=floorLCER_rnc∙Fs | (409) |
| --- | --- |

The individual reflection signals are obtained by reading from the downmix delay line at the position with the corresponding read offset and weighting with the individual reflections amplitude factor.

| reflectionOutn,i=dmxi-intDelayn⋅LCER_totalGain(n,i) | (410) |
| --- | --- |

### Fade

#### General

The Fade stage is responsible for applying fade-in and fade-out ramps to audio signals before RIs are activated or deactivated, respectively. The deactivation is synchronized across stages, so that all stages continue to process the audio signals of RIs (e.g. propagation delay) before their fade-out is complete. The fade stage is also active during teleporting to mute sound output.

#### Data elements and variables

activeListId	list id of the currently active list

B		audio block size

controlMessages	an array of control messages to be passed via the message queue from the update thread to the audio thread processing logic

FADE_NONE, FADED_IN,

FADED_OUT, FADING_IN,

FADING_OUT	constant integer values used to indicate a particular state of the RI fade

fadeActiveDirection	1 indicates a fade-in, -1 indicates a fade-out, 0 indicates an idle state

fadeCurrentState	an atomic integer variable holding the current state of the RI fade, it is set on the audio thread, and read on the update thread

fadeId	a unique reference to the memory, storing the fade state of the current RI

fadeSamplesLeft	number of sample left until the fade envelope is complete

ictrl	index into the controlMessages array

itemActiveEnvelope	an array with the sample values of the currently active fade shape

itemChannels	number of channels in the itemSignal array

itemStatus	an integer variable holding the value representing the current state of a RI

itemSignal	an array of audio buffers containing the current audio frame’s PCM audio samples for each of the RI source channels

itemStatusChanged	a flag indicating whether the itemStatus value for the current RI has been changed by a previous stage in this frame

ITEM_ACTIVE,

ITEM_FADEOUT_INACTIVE,

ITEM_FADEOUT_ORPHANED,

ITEM_INACTIVE,

ITEM_NEW	constant integer values used to indicate a particular status of a RI

L		length of the fade segment to be processed in the current audio frame

messageQueue	Single Producer Single Consumer message queue used to pass messages from the update thread to the audio thread

needsReset	a boolean variable indicating whether the fade needs to be reset

new	FadeDirection	1, -1, or 0, depending on whether the RI needs to fade in, fade out, or remain unchanged after this update frame

renderListId	list ID of the current render list, populated by the update thread

#### Stage description

##### Update thread processing

Based on the current status of each RI, determine the required fade direction.

if (itemStatus == ITEM_INACTIVE) {

newFadeDirection= 0;

}

else if ((itemStatus == ITEM_FADEOUT_INACTIVE || itemStatus == ITEM_FADEOUT_ORPHANED)

&& itemStatusChanged) {

newFadeDirection = -1;// fade out

} else if (itemStatus == ITEM_NEW || (itemStatus == ITEM_ACTIVE && itemStatusChanged)) {

newFadeDirection = 1;// fade in

}

Based on the current state of the RI fade and the itemStatus, determine whether a reset of the fade state is necessary.

needsReset = false;

if (fadeCurrentState == FADED_IN || fadeCurrentState == FADED_OUT

|| itemStatus == ITEM_INACTIVE)

needsReset = true;

Prepare the fade control messages.

controlMessages[ictrl++] = {fadeId, needsReset, newFadeDirection};

Once all active RIs have been processed, add the control messages to the messageQueue, along with the current renderListId.

messageQueue.push({renderListId, controlMessages});

##### Audio thread processing

###### Message queue processing

Figure 79 — Fade stage message queue

The memory layout of the message queue is illustrated in Figure 79. Each message consists of a Render list ID and a pointer to an array of control messages. Each control message contains a fadeId used to identify a specific fade instance, a Boolean flag needsReset, and an integer fadeDirection.

While the Render list ID of the front message in the queue is less than the current activeListId, pop the front message from the queue and update the states of the fades with the given IDs according to the control messages.

Now the message at the front of the queue has its Render list ID equal to the activeListId. Apply the control messages to the fades referenced by the fade IDs as before, but do not remove the message from the queue. It will be removed in a future audio frame, once activeListId has been incremented.

###### Fade application

For each RI with an active fade, multiply the samples in the RI audio buffer by the fade envelope, stored in the itemActiveEnvelope array.

int L = B;

if (fadeSamplesLeft < B)

L = fadeSamplesLeft;

for (int ch=0; ch<itemChannels; ch++) {

for(int i=0; i<L; i++) {

itemSignal[ch][i] *= itemActiveEnvelope[fadeLength – fadeSamplesLeft + i];

}

}

When fading out, if the remaining fade duration is less than B, set the leftover samples to 0.f.

if (L < B && fadeActiveDirection == -1) {

for (int ch=0; ch<itemChannels; ch++) {

for(int i=L; i<B; i++) {

itemSignal[ch][i] = 0.f;

}

}

}

fadeSamplesLeft -= B;

###### Fade state update

Check the current fade parameters for every RI with an active fade, and set the fadeCurrentState atomic variable value accordingly.

fadeCurrentState = FADE_NONE;

if (fadeActiveDirection != 0) {

if (fadeSampleLeft <= 0) {

fadeSampleLeft;

fadeCurrentState = fadeActiveDirection < 0 ? FADED_OUT : FADED_IN;

}

else {

fadeCurrentState = fadeActiveDirection < 0 ? FADING_OUT : FADING_IN;

}

}

### Single point higher order ambisonics (SP-HOA)

#### General

The Single Point Higher Order Ambisonics (SP-HOA) stage renderers a single HOA source binaurally based on the listener position and orientation with respect to the source position. The binaural rendering is performed with a magnitude least-squares (MagLS) decoder. Unlike in the MP-HOA stage, the SP-HOA stage does not use parameterized spatial metadata for its processing.

#### Data elements and variables

j – sub-frame index, k – audio frame index, b – frequency bin index

MESDtoHOA	ESD to HOA transformation matrix

MHOA2bin(b)	Spherical harmonics to binaural decoding matrix of the bth frequency bin

sESD(k)	ESD audio signal of the kth audio frame

sHOA(k)	HOA audio signal of the kth audio frame

Rsh(k)	Real spherical harmonic rotation matrix of the kth audio frame

sHOA,rot(k)	Rotated HOA audio signal of the kth audio frame

S(k)	Time-frequency domain signal of the kth audio frame

Sbink	Time-frequency domain binaural signal of the kth audio frame

sbink	Time domain binaural signal of the kth audio frame

Lframe	Signal frame length in samples

Lhop	Hop length of one STFT window in samples

NsubFrames	Number of sub-frames in the time-frequency domain signal, Lframe / Lhop

Nch,in	Number of channels in the input signal

NFBins	Number of frequency bins in the time-frequency domain signal

Gsphoa	Linear gain multiplier

#### Stage description

##### General

Figure 80 describes the SP-HOA audio thread processing. The binaural decoding is performed in the time-frequency domain.

Each block in Figure 80 is described in 6.6.23.3.2. The initializations for SP-HOA stage are described in 6.6.23.3.3. Update thread processing is described in 6.6.23.3.4.

Figure 80 — SP-HOA audio thread processing

##### Audio thread processing

###### HOA format conversion (ESD2HOA)

The input source signal sESD(k), which is in the Equivalent Spatial Domain (ESD) representation, are converted to Higher-Order Ambisonics (HOA) signals sHOA(k) with:

| sHOA(k)=MESDtoHOAsESD(k) | (411) |
| --- | --- |

where MESDtoHOA is a transformation matrix, see 6.6.23.3.3.3. sHOA(k) signals are N3D normalized.

###### Rotation

Obtain a real spherical harmonic rotation matrix Rsh(k) based on the signal rotation values, and apply it to the input signal sHOA(k):

| sHOA, rotk=RshksHOAk | (412) |
| --- | --- |

The matrix Rsh(k) can be obtained by using the pseudo-code function getSHRotationMatrix(R, L), shown below. R is a rotation matrix according to the listener head orientation and L is the order of the HOA source. For more information, refer to [11].

getSHRotationMatrix(R, L) {

M = (L+1) * (L+1);

R_sh = zero_matrix(M, M);

R_1 = zero_matrix(3, 3);

R_lm1 = zero_matrix(M, M);

R_sh[0][0] = 1.0;

R_1[1][1] = R[1][1];

R_1[1][2] = R[1][2];

R_1[1][3] = R[1][0];

R_1[2][1] = R[2][1];

R_1[2][2] = R[2][2];

R_1[2][3] = R[2][0];

R_1[3][1] = R[0][1];

R_1[3][2] = R[0][2];

R_1[3][3] = R[0][0];

for (i = 0; i < 3; i++) {

for (j = 0; i < 3; j++) {

R_sh[i+1][j+1] = R_1[i][j];

R_lm1[i][j] = R_1[i][j];

}

}

band_idx = 4;

for (l = 2;l <= L;l++) {

R_l = zero_matrix(2*l+1);

for (m=-l;m <= l; m++) {

for(n=-l; n<=l; n++) {

d = m == 0 ? 1 : 0;

denom = abs(n) == l ? (2*l)*(2*l-1) : (l*l-n*n);

u = sqrt((l*l-m*m) / denom);

v = sqrt((1+d)*(l+abs(m)-1)*(l+abs(m)) / denom) * (1-2*d)*0.5;

w = sqrt((l-abs(m)-1)*(l-abs(m)) / denom) * (1-d)*(-0.5);

if (u!=0) {

u = u * getU(l,m,n,R_1,R_lm1);

}

if (v!=0) {

v = v * getV(l,m,n,R_1,R_lm1);

}

if (w!=0) {

w = w* getW(l,m,n,R_1,R_lm1);

}

R_l[m+l][n+l] = u+v+w;

}

}

for(i=0; i<2*l+1; i++) {

for(j=0; j<2*l+1; j++) {

R_sh[(bandIdx + i)*M + (bandIdx + j)] = R_l[i][j];

}

}

for(i=0; i<2*l+1; i++) {

for (j=0; j < 2*l+1; j++) {

R_lm1[i][j] = R_l[i][j];

}

}

bandIdx += 2*l+1;

}

return R_sh;

}

getU(l,m,n,R_1,R_lm1) {

return getP(0, l, m, n, R_1, R_lm1);

}

getV((l,m,n,R_1,R_lm1) {

if (m == 0) {

p0 = getP(1, l, 1, n, R_1, R_lm1);

p1 = getP(-1, l, -1, n, R_1, R_lm1);

ret = p0 + p1;

}

else {

if (m>0) {

d = m == 1 ? 1 : 0;

p0 = getP(1, l, m - 1, n, R_1, R_lm1);

p1 = getP(-1, l, -m + 1, n, R_1, R_lm1);

ret = p0*sqrt(1.0 + d) - p1*(1.0 - d);

}

else {

d = m == -1 ? 1 : 0;

p0 = getP(1, l, m + 1, n, R_1, R_lm1);

p1 = getP(-1, l, -m - 1, n, R_1, R_lm1);

ret = p0*(1.0 - d) + p1*sqrt(1.0 + d);

}

}

return ret;

}

getW((l,m,n,R_1,R_lm1) {

ret = 0.0;

if (m != 0) {

if (m>0) {

p0 = getP(1, l, m + 1, n, R_1, R_lm1);

p1 = getP(-1, l, -m - 1, n, R_1, R_lm1);

ret = p0 + p1;

}

else {

p0 = getP(1, l, m - 1, n, R_1, R_lm1);

p1 = getP(-1, l, -m + 1, n, R_1, R_lm1);

ret = p0 - p1;

}

}

return ret;

}

getP(i, l, a, b, R_1, R_lm) {

ri1 = R_1[i + 1][1 + 1];

rim1 = R_1[i + 1][-1 + 1];

ri0 = R_1[i + 1][0 + 1];

if (b == -l)

ret = ri1 * R_lm1[a + l - 1][0] + rim1 * R_lm1[a + l - 1][2 * l - 2];

else {

if (b == l)

ret = ri1*R_lm1[a + l - 1][2 * l - 2] - rim1 * R_lm1[a + l - 1][0];

else

ret = ri0 * R_lm1[a + l - 1][b + l - 1];

}

return ret;

}

###### STFT

The time domain signal sHOA,rot(k) shall be converted into a time-frequency domain signal S(k) by applying a short-term Fourier transform (STFT), see 6.6.23.3.3.4.

###### Binaural decoding

For each input signal S(j,k,b), apply a decoding matrix MHOA2bin(b) to obtain the output binaural signal:

| Sbinj,k,b=MHOA2bin(b)Sj,k,b | (413) |
| --- | --- |

###### Inverse STFT

The binaural time-frequency domain signal Sbink shall be converted into a time domain signal sbink using an inverse STFT..

###### Linear gain multiplication

If a linear gain multiplier Gsphoa is set for the HOA source, apply it to the binaural output signal:

| sbink=Gsphoasbink | (414) |
| --- | --- |

##### Initialization

###### General

The initialization steps are executed once before the processing chain.

###### Initialize ESD to HOA conversion

Pre-calculate the MESDtoHOA transformation matrix used in 6.6.23.3.2.1. as specified in [7] and [8].

###### Initialize binaural decoder

Obtain a MHOA2binb decoding matrix for each frequency bin, which decodes an input time-frequency domain spherical harmonic signal with Nch,in channels into a binaural time-frequency domain output signal. MHOA2bin shall be calculated as described in 6.6.26.3.1.3.

Instead of computing the decoding matrices during every execution, the matrices may be pre-computed and only read during execution to reduce processing load.

###### Initialize time-frequency domain conversion

STFT shall be initialized with an appropriate number of channels Nch,in, with a hop size Lhop of 128 samples and band center frequencies according to A.10.

The output of the STFT for a single frame of audio data shall contain NsubFrames=Lframe/Lhop amount of sub-frames, each containing Nch,in by NFBins amount of data.

###### Initialize linear gain multiplication

If hoaSourceGainDb is set in the scene plus payload (6.3.2.10), a linear gain multiplier Gsphoa shall be calculated using the following equation

| Gsphoa=10Gsphoa,dB/20 | (415) |
| --- | --- |

where Gsphoa,dB is obtained from hoaSourceGainDb.

##### Update thread processing

In the update thread, a new audio frame is read into buffers, and the signal rotation values are updated based on the positions and orientations of the listener and the HOA source. Gsphoa shall be updated using equation if the Gsphoa,dB for the HOA source is modified.

### Homogeneous extent

#### General

This stage synthesizes Spatially Extended Sound Sources (SESS) for headphone reproduction for object sources that have the associated flag objectSourceHasExtent set to 1. The respective parameters for the object source are identified by objectSourceExtentId.

The synthesis is based on a description of a SESS by an (ideally) infinite number of decorrelated point sources distributed over the entire source extent spatial range. By continuously projecting the SESS geometry in the direction towards the current listener position, the range covered by said geometry can be identified every frame and updated in real-time. In other words, the geometry is projected onto a sphere representing the user’s virtual listening space every frame. And the spatial sections occupied by the projected geometry on the sphere are the ones included in the auralization of the SESS.

Given a desired source extent range, an SESS is synthesized using two decorrelated input signals. These input signals are processed in such a way, that perceptually important auditory cues are synthesized. This includes the following interaural cues: Interaural Cross Correlation (IACC), Interaural Phase Differences (IAPD) and Interaural Level Differences (IALD). Besides that, monaural spectral cues are reproduced. This is illustrated in Figure 81.

Figure 81 — Block diagram of SESS synthesis

#### Data elements and variables

itemStore	a local pointer to the RenderItemStore object

B		block size

Fs		sampling rate

extentProcessors	map from item id to its extentProcessor instance

extentDownmixItem 	RI to store the final output of all extent’s binaural signal.

#### Stage description

To save real-time computational cost, individual HRTF points are assigned into pre-defined grid tables that separate the listener’s virtual listening sphere into uniformly distributed regions. During the initialization, a N-point DFT is performed to get N/2+1 frequency components for each HRIR, where N is the length of it. Then, three intermediate values for each grid are obtained by integrating the data of all HRTF points within, which are the gains of the left and right channels, non-normalized IACC. In addition, the number of HRTF data points included in each grid is also stored. These are used to calculate the final cues in real-time.

The gains of both channels for each grid are calculated with equation 1 and 2, where  Al,n and Ar,n is the magnitude of the left and right HRTF respectively, N is the number of HRTF points that are within this grid:

| G l ω=n=1NAl, n2 | (416) |
| --- | --- |
| G r ω=n=1NAr, n2 | (417) |

The non-normalized IACC for each grid is calculated with equation 3, where , l and , r is the phase of left and right HRTF respectively:

| IACC ω = n=1NAl, n2∙ Ar,n2 ∙ ej ( ϕl,n - ϕr,n) | (418) |
| --- | --- |

During the real-time processing, each unique extended sound source is generated and managed by an Extent Processor. For every frame, each active processor receives a buffer of audio samples and the metadata indicating how to synthesize the extended sound source. Two separate processing chains exist: metadata handling in the update thread and audio processing in the audio thread. These are described respectively in the following sections, and their results are combined at the end of the second chain to produce binaural audio output.

Calculations performed in the Update Thread:

For each unique extended sound source, one or more metadata carriers, in the form of RIs, are generated by the Occlusion stage.

This stage loops through all the incoming RIs and assigns relevant extent metadata to the corresponding processor. If one of the spatial sections from the pre-defined table is covered it  shall be included for auralizing an Extent in this frame. Then, the incoming metadata contains a gain factor and a list of gains corresponding to some pre-defined frequency bins for it. By selecting, weighting and eventually accumulating the stored intermediate data with the gain and EQs, the generation of arbitrary shape of extended sound source with any form and degree of occlusion (size/material) is achieved.

The final filter is obtained by the following steps: After integrating all grid points indicated in the RI, the gain of the left and right channel and IACC are normalized with the total weighted number of HRTF data points:

| G l ω= n=1NGgrid, l,nNweighted | (419) |
| --- | --- |
| G r ω= n=1NGgrid, r,nNweighted | (420) |
| IACC ω= n=1NIACCgrid,n (ω)Nweighted | (421) |

The frequency dependent H α  and H β  are calculated using the normalized IACC:

| H β ω= 12( 1 - 1 -IACC (ω)2 ) | (422) |
| --- | --- |
| H α ω= 1 - Hβ2 (ω) | (423) |  |

The final stereo filters are obtained using H α  and H β , gains of left and right channels (Gl and Gr) and the phase extracted from the HRTF point corresponds to the center of the extent. (phasel and phaser):

| F1, l ω=Hαω ∙ Glω ∙ ej∙phaselω | (424) |
| --- | --- |
| F1, r (ω) = Hβ (ω) ∙ Gr (ω) ∙ ej ∙ phaser (ω) | (425) |
| F2, l (ω) = Hβ (ω) ∙ Gl (ω) ∙ ej ∙ phasel (ω) | (426) |
| F2, r (ω) = Hα (ω) ∙ Gr (ω) ∙ ej ∙ phaser (ω) | (427) |

Calculations performed in the Audio Thread:

The input mono signal is first fed into the decorrelator to obtain two decorrelated versions.

Figure 82 — Decorrelator

The MPEG-I decorrelator performs the following steps to create two completely decorrelated signals from one. Figure 82 is the block diagram of the decorrelator. The decorrelator has an internal processing cycle of a fixed number of 256 samples regardless of the global block size B, so a circular buffer is used to manage the reading and writing of samples in to the decorrelator. The incoming B samples of the renderer are written into the internal buffer. The write cursor starts 128 samples ahead of the read cursor, which acts as a delay compensation unit that is parallel to the whole processing chain. There is a 128-sample of overlap between each decorrelator processing frame. As a consequence, when more than 128 new samples come in, N samples (128 old + 128 new) are stored in the input buffer and the decorrelation processing starts.

A 256-point DFT is performed on the windowed frame to obtain K=129 frequency bins. A sine window is applied as shown in equation 10, where N = 256.

| xwin(n) = x(n) * sin( (n + 0.5) * π / N ) | (428) |
| --- | --- |

The complex DFT coefficients are passed through a delay, as illustrated in Equation (429) for the n-th frame, where Dd is the pre-delay. The pre-delay is set to 4 frames. Next, the delayed signal is passed on to a series of first order all-pass filters, which is illustrated in direct form  in Equation (430), where a1=b0=0.7  and  b1=1 represent the coefficients, and Dapi represents the amount of delay of each all-pass filter, which is 1 , 2, 3, 5 frames for i = 1, 2, 3, 4.  In the following steps, X(n) is called the direct component (DC), while Y(n), which went through the delay and all-pass filter, is called the processed component (PC).

| Y(n)=X(n-Dd) | (429) |
| --- | --- |
| Y(n)=b0X(n)+b1X(n-Dapi)-a1Y(n-Dapi) | (430) |

A transient in the current frame is detected by calculating whether the energy of the current frame, summed over certain frequency bins, is stronger than the previous frame by a threshold T=2.8. Two counters control the transient processing: a hold counter and a inhibition counter. Both are initially set to their inactive state 0. When a transient is detected and the inhibition counter is inactive, a hold counter is started for the next 8 frames to control a muting of the processed signal in the output mix. Also, the inhibition counter starts counting to prevent a hold counter start in the next 56 frames. In addition, if another transient is detected during this inhibition time, this will re-start the inhibition counter, and the inhibition time will be increase from 56 to 64 frames. When active counters reach their maximum count, they are reset to their inactive state 0.

The energy in current frame is calculated using the DCs. The energy of the current frame is smoothed by a factor of  d=0.4 with the previous frame. Equation (431) and (432) illustrates how energy of the current frame, E(n), is calculated with the energy of the previous frame, E(n-1), and the DCs, Xk(n), and how the decision of transient detection is made respectively.

| E(n) = d * (k=4KXk(n)2 )+ (1 - d) * E(n-1) | (431) |
| --- | --- |
| transient detected ,if En> T * En – 1& no transient detected , otherwise | (432) |

Each bin of the PCs is amplified or attenuated if it is weaker or stronger by a factor of  = 1.5 comparing to the DC. Equation (433) and (434) illustrates how to calculate the energy of the current PC, Ep,  k(n), and the current DC, Ed,  k(n) with α = 0.4. Equation (435) demonstrates the boosting or suppression process depending on the energy difference. Finally, the PC is multiplied with a fixed normalized factor f = 1.1.

| Ep, k(n) = a * Yk(n)2+(1 - a) * Ep,k(n-1) | (433) |
| --- | --- |
| Ed, k(n) =a * Xk(n)2+(1 - a) * Ed,k(n-1) | (434) |
| Y(n) = f * Y(n) * b * Ed, k(n)Ep, k(n) , if Ep, k(n) > b * Ed, k(n) f * Yn* Ed, knb * Ep, kn , if Ed, kn> b * Ep, kn f * Y(n) , otherwise. | (435) |

A N-point IDFT is performed to transform the processed frequency bins to time domain and frames are combined in a windowed overlap-add procedure applying a sine window.

The decorrelated output with normalization is generated as illustrated in Equation (436). If the hold counter is inactive, the two decorrelated output frames are the sum and the difference of the windowed and original input and the processed signal respectively. If a transient was detected and the hold counter is activated, the two output frames will be identical, and contain the windowed and original input weighted by a scaling factor.

| xout(n) = xorign± xproc n , if hold counter inactive 22 *xorig(n) ,if hold counter active | (436) |
| --- | --- |

The writing of 128 samples from the decorrelator output and the reading of B samples back to the renderer input for the two decorrelated signals are managed using circular buffers.

Finally, each of the two decorrelated signals are convolved with the corresponding stereo filters calculated in the update thread, which results in four channels of output. Then, a cross mixing will be performed to produce the final binaural output. Equation (437) and (438) defines the mixing process, where S1 and S2 stands for the two decorrelated signals, and F1, and F2,   are the two stereo filters calculated in the metadata processing section.  Figure 83 is a signal flow diagram for the process.

| Sl (ω) = F1, l ω ∙ S1 (ω) + F2, l (ω) ∙ S2 (ω) | (437) |
| --- | --- |
| Sr (ω) = F1, r ω ∙ S1 (ω) + F2, r (ω) ∙ S2 (ω) | (438) |

Figure 83 — Block diagram of the signal mixing process

### Panner

#### General

The Panner stage is used for panning audio sources to a virtual loudspeaker (LS) setup. The Panner stage implements Vector-Base-Amplitude-Panning (VBAP) with additional controls such as configurable spatial spread.

#### Data elements and variables

speakerNodeDirections	Loudspeaker directions (azimuth, elevation)

speakerNodeCount	Number of LS

m_internalSpeakerNodeCount	Number of internal LS nodes including possible virtual top or bottom node

B		Number of samples in a processing block.

outputRenderItems	Panned output RIs containing the LS signals

m_tripletSelect	Triplet selections for panning directions

SpeakerNode	A data structure defining a LS node including its unit vector, azimuth and elevation in degrees, and SpeakerNodeGroup

SpeakerNodeGroup	Unique grouping of the LS node depending on its elevation. One of bottomHalf, horizontal, or topHalf.

oldGainWeight	Apply gain smoothing between frames if oldGainWeight > 0. (default 0.0)

spreadAuxiliarySourceCount	Number of auxiliary sources for multiple direction amplitude panning (MDAP, [2]) (default 0)

spreadRadiusDeg	Radius for auxiliary sources in MDAP (default 0.0)

spreadGainFraction	Fraction of gain for auxiliary sources (default 0.0)

l1, l2, l3	Unit vectors (the vector base) of a LS triplet

g		Vector of amplitude panning gains

p		Unit vector pointing towards the amplitude panning direction

#### Stage description

The Panner stage implements Vector-Base Amplitude Panning (VBAP, [1]) of its input rendering items into the configured 3D LS configuration with additional controls such as configurable spatial spread.

VBAP refers to the method where three unit-vectors l1, l2, l3 (the vector base) are assumed from the point of origin to the positions of the three LSs forming a triplet (or triangle) wherein the panning direction is located.

The panning gains for these three LSs are determined as weights of the corresponding three unit-vectors such that the weighted sum of these vectors points towards the desired amplitude panning direction. This is solved as follows. A column unit vector p is formulated pointing towards the desired amplitude panning direction, and a vector g containing the amplitude panning gains can be solved by a matrix multiplication

| gT=pTl1Tl2Tl3T-1 | (439) |
| --- | --- |

where -1 denotes the matrix inverse. After formulating gains g, their overall level is normalized such that for the final gains the energy sum gTg = 1.

Audio source positioning in VBAP is performed in three phases

1)	Triangulate the LS setup.

2)	Based on the direction from the listener to the audio source, select a triplet and compute the gains for the LSs of the triplet.

3)	Apply the gains to the audio signal to be positioned.

The phase 1) is performed at Panner stage initialization and the result of the initialization are a list of LS nodes and a lookup table of precalculated LS triplet inverse matrices which are then utilized in real-time processing.

##### Stage initialization: general

The initialization of the Panner stage is carried out before processing of the audio samples delivered by the decoder takes place.

##### Run-time configuration of panner parameters

It is possible to override the following parameters relevant for Panner stage parameterization from the config variables starting with the prefix “Panner:”. If config variables are not given, then default values are used.

speakerNodeDirections = Panner:directions or defaultDirection in 6.6.25.4.

oldGainWeight = Panner:oldGainWeight or 0.0.

spreadAuxiliarySourceCount = Panner:spreadAuxiliarySourceCount or 0.

spreadRadiusDeg = Panner:spreadRadiusDeg or 0.0.

spreadGainFraction = Panner:spreadGainFraction or 0.0.

##### Panner stage initialization

The Panner stage is initialized with the parameters described in 6.6.25.3.2.

The Panner stage initializes its output RIs as type Nondiegetic and having their positions according to the virtual LS positions in speakerNodeDirections.

##### Stage update

The Panner stage determines its input items as RIs having ItemType::Pannable and which are not Inactive or Orphaned. The number of inputs is stored as inputCount.

Panner instance created at the first call to update when inputCount > 0.

##### Panner creation

###### LS configuration basic checks

Basic checks are done to check that valid LS directions have been provided:

—	speakerNodeCount <= 128 and speakerNodeCount > 3.

—	speakerNodeDirections does not contain duplicates.

###### Adding virtual top or bottom LS node

A check is then done if the speaker node setup needs a virtual top or bottom node. This updates m_internalSpeakerNodeCount.

If a speaker node is found

—	above NO_VIRTUAL_SPEAKER_NODE_ELE_LIMIT_DEG (45.0 degrees), no virtual top or bottom node is used

—	above DISTRIBUTE_VIRTUAL_SPEAKER_NODE_ELE_LIMIT_DEG (20.0 degrees), energy-spreading virtual top or bottom node is used. The energy of an energy-spreading virtual node is distributed into its neighbors.

—	not above DISTRIBUTE_VIRTUAL_SPEAKER_NODE_ELE_LIMIT_DEG, energy-omitting

virtual top or bottom node is used. The energy from an energy-omitting virtual node is omitted, i.e., it is not distributed into its neighbors.

The same applies for both elevations and inclinations, i.e., the two half-spheres.

Internal speaker node configuration is then initialized, which is the original LS configuration appended with the virtual top and/or bottom LSs. A possible virtual top LS is created at azimuth 0.0, elevation 90.0. A possible virtual bottom LS is created at azimuth 0, elevation -90.0.

Triangulation of the internal LS node configuration is then performed to obtain the LS triplets for all possible azimuth, elevation combinations. This is done in three main phases: 1) creating internal LS nodes, 2) determining connections between internal LS nodes; and 3) determining virtual surface triplets.

###### Creating internal LS nodes

Triangulation of the internal loudspeaker node configuration is then performed to obtain the loudspeaker triplets for all possible azimuth, elevation combinations. The goal of triangulation is to completely cover the virtual surface defined by the LS nodes with non-overlapping virtual surface triplets of three LS nodes. This is done in three main phases: 1) creating internal loudspeaker nodes, 2) determining connections between internal loudspeaker nodes; and 3) determining virtual surface triplets. Care is taken in order to have no triplets with edges which cross the horizontal plane as panning of sound sources on the horizontal plane is preferably done with LS triplets that have an edge on the horizontal plane. The horizontal plane refers to the virtual plane spanned by the LS nodes at zero elevation. The reason for this is that the binaural cues for elevation perception relate mostly to the sound spectrum (which is not well reproduced with amplitude panning), while the binaural cues for azimuth perception relate mostly to the inter-aural level and time differences (which are better reproduced with amplitude panning). Also, when there are at least two loudspeakers at some elevation level above/below the horizontal level (0 elevation) spanning a virtual plane, the method tries to avoid crossing such a virtual plane with a triplet edge (although this can happen).

Internal speaker nodes contain the input LS nodes and the possible top and/or bottom virtual nodes. The nodes are ordered in the same order as the input speakerNodeDirections. The last two nodes are the possible bottom and top virtual node, respectively, if present. The nodes have indices nodej, j=1,…, internalSpeakerNodeCount . The internal speaker nodes are classified to three groups:

—	horizontal group containing speaker nodes with elevation between -5 degrees and +5 degrees. The elevation for the horizontal group speaker nodes is set to zero degrees. That is, the speaker nodes in the horizontal group are moved onto the virtual plane at zero elevation.

—	top half group containing speaker nodes with elevation larger than +5 degrees

—	bottom half group containing speaker nodes with elevation smaller than -5 degrees

Unit vectors are formed for each node as

| lT=cos(aziRad)cos(eleRad)sin(aziRad)cos(eleRad)sin(eleRad) | (440) |
| --- | --- |

where aziRad and eleRad are the speaker node azimuth and elevation in radians.

###### Determining connections between internal speaker nodes

All valid connections between internal speaker nodes are then determined. This is done in the method determineConnections. Connections between LS nodes are candidates for triplet edges.

First, some prominent elevated planes that are favoured in making node-to-node connections are determined. A prominent elevated plane is determined if

—	There are at least three speaker nodes outside the horizontal group which have the same elevations (within a tolerance of 0.001 degrees) and for which the maximum horizontal gap is less than 140 degrees.

The elevations of determined prominent elevated planes are stored into nonCrossingPlaneElesDeg.

The node-to-node connections are determined in three stages: top, horizontal and bottom. The aim for this is to avoid connections crossing the horizontal plane. This provides better quality for audio objects placed on the horizontal plane.

Node-to-node connections for the top half are first determined. This contains the following steps:

—	get the next possible half-sphere connection where at least one node is in the top half group and the other is in the same group or in the horizontal group.

—	determine a connection class for the half-sphere connection by checking the possible half-sphere connection with respect to all LS nodes:

- in case there is a node behind or nearly behind the connection line, discard the connection. These are not valid connections as they do not lie on the convex surface enclosing the LS nodes.

- in case there is a node that is closer to the connection line than 1/5 of the connection length AND at the same horizontal plane. These connections need to be weighted with a penalty. The connection class is set as elevatedPlaneThinTriangle. This is done in order to favor connections which are horizontal.

- Otherwise the connection class is set as regular.

—	Connections of type regular and elevatedPlaneThinTriangle are processed further.

- The favorability of a connection is measured by the angle between unit vectors of the nodes in the connection. The larger the angle, the less favorable the connection is. The angle is stored as the variable newArcWeighted for the connection.

- For connections of class elevatedPlaneThinTriangle, newArcWeighted *= 2 (they are penalized)

- For connections crossing a non-crossing plane newArcWeighted *= 2 (they are penalized)

- The validity of a connection is determined by comparing against all previously defined connections. A comparison to a previously defined connection oldConnection is done if the connections do not share the same nodes. The comparison involves determining whether the connections cross. The crossing is determined by studying the intersection of two planes defined by the two connections (two node pairs). The determination of whether there is a crossing is done by the method detectCrossing.

  - If there is a crossing and newArcWeighted <= (arcWeighted of oldConnection) then oldConnection is discarded.

  - Else if there is a crossing and newArcWeighted > (arcWeighted of oldConnection) then the new connection is not valid and is discarded.

- If the connection passes the validity checks it is stored into the connection data.

—	The process repeats for the next possible half-sphere connection until no possible half-sphere connections remain.

Node-to-node connections for the horizontal plane are then determined.  This is done by iterating through all speaker nodes and forming, for each node nodeA, a connection to the closest node. The closest node is searched among all the other nodes and the closeness is measured as the azimuth angle difference from the other node to nodeA.

Node-to-node connections for the bottom half are then determined. This is done in the same way for as for the top half but searching connections within the bottom half instead of the top.

###### Determining virtual surface triplets

Virtual surface triplets are then determined from the node-to-node connection data. This is done in the method determineVirtualSurfaceTriplets.

The method forms possible triplets by selecting combinations of three LS nodes nodeA, nodeB, and nodeC having indices nodeAIndex, nodeBIndex, and nodeCIndex, respectively, such that

—	nodeCIndex > nodeBIndex > nodeAIndex and

—	there is a connection from nodeA to nodeB and nodeC and

—	there is a connection between nodeB and nodeC.

For each possible triplet a check is made to ensure that no speaker node is located within the triplet. This is done by calculating, for each possible triplet of nodeA, nodeB, nodeC, the VBAP gains for all speaker nodes not in the triplet and checking that no speaker node obtains positive gains (indicating that the speaker node is within the triplet).

The triplet is valid if no speaker node is found inside the triplet. In this case the speaker node indices and the inverse matrix of the triplet are stored.

###### Determining how to distribute virtual speaker node energies

A determination of how the virtual LS node energies are distributed to real LS node energies is then performed.

For energy distributing virtual node, the energy of the virtual node is divided equally to all real speaker nodes connected to the virtual node. The division gain is, for the neighbour n of total of N neighbours

| vn=(1/N)0.8 | (441) |
| --- | --- |

The exponent 0.8 was obtained by listening and selecting a value which provides good audio quality over a range of test signals.

###### Forming the triplet selection table

A triplet selection table is then formed. This is done by searching for all possible azimuth and elevation combinations, with resolutions of 2 degrees and 5 degrees, respectively, the LS triplet into which this azimuth and elevation direction falls. The triplet selections are stored into m_tripletSelect.

##### Stage processing

The LS gains for each input item at the panning direction azi, ele are determined as follows. The triplet corresponding to the direction azi, ele is obtained by table lookup from m_triplectSelect. The panning vector is obtained as

| pT=cos(aziRad)cos(eleRad)sin(aziRad)cos(eleRad)sin(eleRad) | (442) |
| --- | --- |

where aziRad and eleRad are azi and ele in radians. The panning gains g are solved as described in the clause 6.6.25.3, using the precalculated inverse matrix obtained from the triplet data. If a virtual LS node is part of the selected triplet, then the virtual LS node energy is distributed to its real LS node neighbors with the division gains calculated in clause 6.6.25.3.5.6 multiplied with the virtual LS node gain.

If spreadAuxiliarySourceCount > 0, multiple direction amplitude panning is used to add auxiliary panned sources around the actual source to create an extended or spread sound perception. In this case, the direction azi and ele and spreadGain for the spreadIndexth auxiliary source are calculated as

angle = 2 * pi * spreadIndex / spreadAuxiliarySourceCount;

azi += spreadRadiusDeg * cos(angle);

ele += spreadRadiusDeg * sin(angle);

spreadGain = spreadGainFraction / spreadAuxiliarySourceCount;

If oldGainWeight > 0, gain smoothing is applied to obtain smoothed gains gsmoothed=oldGainWeight * gprevious + (1-oldGainWeight) * g, where gprevious are the gains from the previous frame. After smoothing the gsmoothed are normalized.

The panning gains are stored into m_targetChannelGains.

In case the input item gain is zero, spreadGain shall be set to 0 as well as all m_targetChannelGains.

Linear interpolation within the block B is applied to ensure that the gain value changes smoothly between buffers. To accomplish this, previous panning gains are stored at previousChannelGains and a linear fade-in is done within each block from previousChannelGains to m_targetChannelGains and previousChannelGains are updated. The gain value to be applied to each audio sample is stored in the variable gain.

If spreadGain > 0 it is multiplied with gain.

#### Panner default directions

defaultDirection = {{90, 0},

{-90, 0},

{114, 20},

{-60, -6},

{85, -44},

{-130, -21},

{49, 41},

{-67, 52},

{154, -9},

{-48, -55},

{19, -31},

{-162, 21},

{151, 73},

{-9, 20},

{180, -63 }};

#### Directional Focus of the Panning stage

When the parameter hasDirectionalFocus provided by the Accessibility User Interface (Annex B.5) is set to true, a directional focus effect is applied, and the panned LS gains are weighted using the following algorithm. (See also visualization in Figure 73):

directionalFocusAperture		Opening angle of the directional focus’ main lobe in degree.

startStopbandAngle	Angle in degree where the stopband starts: directionalFocusAperture +directionalFocusTransitionWidth

stopbandGainLinear 		Linear gain computed as

pow(10.0, -0.05 * directionalFocusStopbandAttenuationDb)

coefA	Variable to compute the attenuation in the transition band:

-0.05directionalFocusStopbandAttenuationDb startStopbandAngle - directionalFocusAperture

The Angular distance angDist in degrees between the direction of each panned LS and the frontal viewing direction is computed as:

angDist = cos-1(cosθ*cosϑ)* 180π

with θ and ϑ being the azimuth and elevation angles in radians per LS as defined in speakerNodeDirections.

Based on based on angDist the gain g of the particular LS is scaled as:

if (angDist > directionalFocusAperture) {

if (angDist >= startStopbandAngle) {

gain *= stopbandGainLinear;

} else {

gain *= pow(10.0, coefA * (angDist - directionalFocusAperture));

}

}

### Multi-point higher order ambisonics (MP-HOA)

#### General

The Multi-Point Higher Order Ambisonics (MP-HOA) stage renders multiple HOA sources simultaneously and creates a 6 Degrees-of-Freedom (6DoF) listening experience for the listener. Figure 84 shows an example situation where a listener is experiencing an audio scene comprised of five HOA sources. The aim is to create a binaural output signal at the listener position in the Audio Scene and orientation based on the HOA source positions, orientations and audio signals and provided HRIRs.

Figure 84 — A top-down view of a listener with a position and orientation in a scene comprising five HOA sources with different positions and orientations.

Figure 85 shows a block diagram describing the input and output of the MPHOA stage. The MP-HOA stage performs MP-HOA processing for each HOA group defined in hoaGroups() of payloadScene(). The MP-HOA stage receives as input a set of input audio signals sESD(i,j) (one for each HOA source i) and their associated positions and orientations, the listener position pL(j) in the Audio Scene and a set of provided HRIRs. As output, the MP-HOA stage provides a binaural output signal soutj.

Figure 85 — The MP-HOA stage provides binaural output sout(j) for each HOA Group. The binaural output is provided based on audio and metadata for HOA Sources belonging to the HOA group.

#### Data elements and variables

pL(j)	listener position (x, y, z) for frame j

oL(j)	listener orientation (yaw, pitch, roll) for frame j

pi	position of HOA source i, hoaSourcePositionX, hoaSourcePositionY and hoaSourcePositionZ from hoaSources() in payloadScene()

oi	orientation of HOA source i, hoaSourceOrientationYaw, hoaSourceOrientationPitch and hoaSourceOrientationRoll from hoaSources() in payloadScene()

MHOA2bin 	Ambisonics to binaural decoding matrix. (Nch×2)

sESD(i,j)	audio signal of HOA source i for audio frame j in ESD format, (Nch×Lf)

sHOA(i,j)	audio signal of HOA source i for audio frame j in HOA format, (Nch×Lf)

sESD(i,j,k)	audio signal of HOA source i for subframe k of audio frame j in ESD format, (Nch×Lsf)

sHOA(i,j,k)	audio signal of HOA source i for subframe k for audio frame j in HOA format, (Nch×Lsf)

S(i,j,k)	time-frequency domain signal of HOA source i for subframe k of the jth audio frame, (Nch×Nb)

S(i,j,k, b)	time-frequency domain signal of HOA source i for subframe k of the jth audio frame for frequency bin b, (Nch×1)

MESDtoHOA(b)	ESD to HOA conversion matrix (Nch×Nch) for frequency bin b

Lf	audio frame length, 256 samples

Lsf	sub-frame length, 128 samples

Ns	number of HOA sources in HOA group, coSourceCount element in hoaGroups() in payloadScene()

Nch	number of audio channels in the audio signal for an HOA source, inputChannelsCount from audioStreams() in payloadScene()

Nsf	number of subframes per audio frame, 2

Nb	number of frequency bins for time-frequency domain signals of HOA sources, 133

H(b,d)			HRTF at frequency bin b for direction index d, (2×1)

CFOAi,j,k,b	covariance matrix (1st order) for HOA source i for subframe k of audio frame j for frequency band b, (4×4)

bn 	Barycentric coordinates for triangle n and current listener position. (3×1)

Ti	ith triangle of the triangulation of the HOA source space

NT	number of triangles in the triangulation of the HOA source space	TLj triangle, which the listener is in, for frame j.

TA(j) 	active triangle for frame j

Tc(j)	chosen triangle for frame j

TRR	Triangle track record.

NTRR 	Length of triangle track record

wcj,k	chosen spatial metadata interpolation weights for subframe k of frame j, (Ns×1)

wA(j)	spatial metadata interpolation weights for active triangle for frame j, (Ns×1)

w(j,k)	interpolated spatial metadata interpolation weights for active triangle for subframe k of frame j, (Ns×1)

ii,j,k,b 	sound field intensity vector for HOA source i for subframe k of audio frame j for frequency band b

e(i,j,k,b)	energy for HOA source i for subframe k of audio frame j for frequency band b

ei,j,b 	Average (over sub-frames) energy for HOA source i and audio frame j for frequency band b.

θi,j,b	azimuth parameter for HOA source i for frame j and frequency bin b

φi,j,b	elevation parameter for HOA source i for frame j and frequency bin b

ri,j,b	direct-to-total energy ratio parameter for HOA source i for frame j and frequency bin b

θi,j,k,b	azimuth parameter for HOA source i for sub-frame k of frame j and frequency bin b

φi,j,k,b	elevation parameter for HOA source i for sub-frame k of frame j and frequency bin b

vi,j,k,b	spatial metadata in vector form for HOA source i for sub-frame k of frame j and frequency bin b

vi,j,k,b	rotated spatial metadata in vector form for HOA source i for sub-frame k of frame j and frequency bin b

Rhead(j)	rotation matrix corresponding to the listener orientation for frame j

Rsource(i,j)	rotation matrix corresponding to the HOA source i orientation for frame j

vj,k,b	interpolated spatial metadata vector for sub-frame k of frame j and frequency bin b

θj,k,b	interpolated azimuth parameter for sub-frame k of frame j for frequency bin b

φj,k,b	interpolated elevation parameter for sub-frame k of frame j for frequency bin b

rj,k,b	interpolated direct-to-total energy ratio parameter for sub-frame k of frame j for frequency bin b

ej,k,b	interpolated energy parameter for sub-frame k of frame j for frequency bin b

Geqj,k,b	signal interpolation EQ gain value for sub-frame k of frame j for frequency bin b

GeqMax	Max value for EQ gain for signal interpolation, constant value 4.0

mcj	index of chosen HOA source for signal interpolation

dT	distance threshold multiplier for HOA source switching in signal interpolation, constant value 1.2

Sb,cj,k	channel c of interpolated audio signal for sub-frame k of frame j for frequency bin b

Bj,b	rotated interpolated audio signal for frame j and frequency bin b (prototype signal for mixing)

Cx(j,b)	covariance matrix of protype signal for frame j and frequency bin b

Cylin(j,b)	matrix with diagonal elements from Cx(j,b)

∂	recursion smoothing coefficient, constant value 0.9

Cydirect(j,b)	direct portion of target covariance matrix for frame j and frequency bin b

Cydiffuse(j,b)	diffuse portion of target covariance matrix for frame j and frequency bin b

Cy(j,b)	target covariance matrix for frame j and frequency bin b

Mj,b	mixing matrix for frame j and frequency bin b

Mrj,b 	residual mixing matrix for frame j and frequency bin b

M(j,k,b)	mixing matrix parameter for sub-frame k of frame j for frequency bin b

Mr(j,k,b) 	residual mixing matrix parameter for sub-frame k of frame j for frequency bin b

Oj,k,b	output time-frequency domain signal for sub-frame k of frame j for frequency bin b

soutj	binaural output signal from frame j

#### Stage description

Figure 86, below shows a block diagram with the required processing steps taken in the MP-HOA stage to obtain the binaural output signal.

Figure 86 — An overview of the main processing steps in the MP-HOA stage. The blocks shown here are explained in separate sections below (Pre-processing in 6.6.26.3.1, Position pre-processing in 6.6.26.3.1.5, spatial analysis in 6.6.26.3.3, Signal and Spatial metadata interpolation in 6.6.26.3.4 and Mixing and Output in 6.6.26.3.5).

##### Pre-processing

###### Triangulation of HOA source space

The HOA source space is triangulated using Delaunay triangulation. HOA source positions are provided as input to the Delaunay triangulation. The output of the triangulation is a set of triangles Ti. Each Ti is a sorted (lowest to highest) list of indices of the HOA sources defining the triangle. See Figure 87 for an example. Any triangle edges that are too long (4m) are removed and their corresponding triangles are omitted (see clause 6.6.26.4.1.2).

For scenes having hoaGroups() in Scene plus payload (6.3.2.10) with only two HOA sources (referred to as scenes with two HOA sources), the Delaunay triangulation is skipped. Instead, a single triangle, T1={1,2,2} is created from the two sources (m1 and m2) present in the scene.

For scenes having hoaGroups() in Scene plus payload (6.3.2.10) with only a single HOA source (referred to as scenes with one HOA source), the Delaunay triangulation is skipped. Instead, a single triangle, T_1={1,1,1} is created from the single source (m1) present in the scene.

Figure 87 — An example triangulation of HOA source space.

###### Calculate HRTFs from HRIRs

The provided HRIRs shall be passed through a Short-time Fourier transform to obtain HRTFs. The STFT shall be configured with a frame hop of 128 samples and band centre frequencies according to A.10.  For each HRIR direction dH, a corresponding HRTF, H(dH), of with Nb=133 frequency bins shall be calculated.

A grid of directions shall then be determined. The grid shall be formed of azimuth and elevation pairs at a constant resolution for each (2 degrees for azimuth and 5 degrees for elevation). This results in a grid of 6480 directions. For each HRIR (and HRTF) direction dH, the closest grid direction is determined dG. This information is stored in a list which provides a mapping from a grid direction dg to an HRIR direction dH.

Thus, an HRTF corresponding to a direction defined by azimuth θ and elevation φ  may be expressed as

| HGdθr, φr | (443) |
| --- | --- |

where θr and φr are the rounded azimuth θ  and elevation φ values to the nearest grid points and  Gd is a mapping between the rounded azimuth and elevation values and the HRTF direction grid indices.

###### Obtain ambisonics to binaural conversion matrix

An Ambisonics to binaural conversion matrix MHOA2bin(b) is determined for each frequency bin using the magnitude least-squares (MagLS) approach. See [9] for more information.

For frequency bins b=[0,11], whose centre frequencies fall below 1500Hz, MHOA2bin(b) is obtained by linearly solving

| YNYNT=MHOA2binbYNHbH | (444) |
| --- | --- |

In the above equation,

| Hb=Hbl(d1)…Hbl(dG)Hbr(d1)…Hbr(dG), | (445) |
| --- | --- |

where Hbl(dg) and Hbr(dg) are the (complex) values of the HRTF for direction dg in the grid of directions (6.6.26.3.1.2) and frequency bin b for the left and right ear respectively.

YN is obtained by calculating, for every direction dg in the grid of directions a set of spherical harmonics  Ynkθi,σi for each spherical harmonics order n and degree k=-n,n using the following equation:

Ynkθ,σ=2n+1n-k!n+k!Pnk(cosσ)*2sinkθ,  if k<0    1,                              if k==02coskθ,  if k>0     ,

where Pnk is the Legendre function of order n and degree k. The Legendre functions are present without the Condon-Shortley phase.

YN is constructed so that the spherical harmonics for a direction di are found in the ith column of the matrix. The spherical harmonics are ordered in the columns first by order and second by degree in ascending order.

YN=Y00(d1)…Y1dG⋮⋱⋮YNN(d1)…YNdG,

for each spherical harmonics order n and degree k =[-n,n].

For frequency bins b=[12,132],  with centre frequencies equal to or above 1500Hz, MHOA2binb is obtained iteratively by calculating the angle ϕb-1

ϕb-1=∠MHOA2binb-1YN,

and then linearly solving:

YNYNTMHOA2binb=Hbexpiϕb-1.

###### Determine frequency bands

If hoaGroupHasFreqBandConfig is set to true, a set of frequency bands shall be created. A frequency band q combines together a set of frequency bins b and is defined by an upper frequency limit bin index erbUpperIndexesq and a lower frequency bin index erbLowerIndexesq. Frequency band q contains the frequency bins b∈erbLowerIndexesq, erbUpperIndexesq. The frequency bands shall be determined as follows.

First, set the quantized upper frequency limit for the first frequency band, BERB,h0 as follows:

| BERB,h0=freqVec[0] | (446) |
| --- | --- |

Then, iteratively determine the quantized upper frequency limits for frequency bands q>0 by first calculating the upper frequency limit and then quantizing it to the closest frequency in the band center frequency table (freqVec).

| BERB,h(q)=24.7*wERB+BERB,hq-1*(1.0+0.108*multB*wERB) | (447) |
| --- | --- |
| closestBinIndex=argmini∈0,Nb-1freqVeci-BERB,hq | (448) |
| BERB,hq=freqVec[closestBinIndex] | (449) |
| erbUpperIndexesq=closestBinIndex | (450) |

where wERB is the width for the ERB bands. wERB is signaled by intermediateBandsERBWidth multB shall be defined as:

| multB=213+1.02 | (451) |
| --- | --- |

The above process is continued until BERB,hq>freqVec[lowestHighBandIndex]. During the iteration, if erbUpperIndexesq is equal to the one calculated on the previous iteration erbUpperIndexesq-1, erbUpperIndexesq is set to closestBinIndex+1 and BERB,hq=freqVec[closestBinIndex+1].

A single frequency band shall be used for frequencies above freqVec[lowestHighBandIndex]. For this band, erbUpperIndexes is appended with the value Nb-1 and the quantized upper frequency limit for this band is set to freqVec[Nb-1].

The lower frequency bands (for which erbUpperIndexesq<highestSingleBinBandsIndex) shall be replaced with single bin bands, where erbUpperIndexesq=q.

After adding the single high frequency band and the single bin bands, the BERB,hq values shall be updated:

| BERB,hq= erbUpperIndexesq | (452) |
| --- | --- |

The lower frequency bin indices erbLowerIndexes(q) shall be obtained as follows:

| erbLowerIndexesq= 0, q=0erbUpperIndexesq-1+1, q∈q>0 | (453) |
| --- | --- |

If hoaGroupHasFreqBandConfig is set to false, no frequency bands shall be created.

###### Automatic Rescaling

During the initialization of the MP-HOA stage, the HOA Source positions are rescaled by the bitstream parameter audioSourceRescale. When the HOA Group has a parent anchor and autoRescale is set to True in the parent anchor, then the HOA Sources positions are rescaled automatically. Bounding boxes, Rv and Rl, are obtained respectively for the HOA Group region defined in the virtual scene, and for the physical listener space where the playback is to occur. Each box has Nboxdim sides corresponding to height, width, and depth. The max and min values for each region dimension are also obtained. The tolerance values set the percentage of Rv that remains outside of Rl and are used to limit the amount of automatic rescaling performed. The process for automatically determining the rescaling factors is as follows:

determine_rescale(Rv, Rl, tolerance) {

for (i = 0; i < Nboxdim; i++) {

rescale[i] = (Rl_max[i] - Rl_min[i]) /

((Rv_max[i] * (1 - tolerance[i*2]))

- Rv_min[i] * (1 - tolerance[i*2+1]);

if (rescale[i] > 1.0) {rescale[i] = 1.0;}

}

The rescale factors are applied to the source positions of the HOA Translator, with an offset so that the rescaling is performed with respect to the anchor position.

##### Determine interpolation weights

This section corresponds to the Position pre-processing block of Figure 86. It takes as input the listener position as well as the HOA source positions. In addition, the triangulation data from the pre-processing stage is received as input. From these, a set of interpolation weights wcj,k that are used to control signal and spatial metadata interpolation are determined.

###### Determine active triangle

For each frame of audio data j, an active triangle TA(j) and a set of interpolation weights wA(j) for the triangle shall be determined. First, the triangle TL in which the listener is in is found. Then, a matrix Txy, n is created for each triangle Tn:

| Txy,n=xn,1xn,2xn,3yn,1yn,2yn,3111 | (454) |
| --- | --- |

where xn,m and yn,m are the x- and y-coordinates of the position of the mth HOA source defining the triangle Tn. Then, Barycentric coordinates bn for each triangle are obtained linearly solving the following formula:

| Txy,nbn=pL,xy | (455) |
| --- | --- |

where pL,xy=[px py 1] is the listener position on the x-y plane.

A triangle Tn, for which all Barycentric coordinates (elements of bn) are positive is the triangle in which the listener is located and is set as the active triangle TA(j). The Barycentric coordinates are used as the interpolation weights for the HOA sources belonging to the triangle TA(j). Interpolation weights for all other HOA sources are set to 0. The interpolation weights are stored in a vector wA(j).

###### Interpolation of interpolation weights

The interpolation weights shall then be interpolated over the current and previous frames as shown below to obtain interpolation weights for each subframe k of frame j.

| w(j,k)=wAj*kNsf+wAj-1*(1-kNsf),k∈1..Nsf | (456) |
| --- | --- |

###### Determine chosen weights and triangle

The active triangle TA(j) and the weights w(j,k), wAj and wpj=wAj-1 shall be stored in a triangle track record TRR. The TRR is a list of active triangle and weight information from the current and previous audio frames. The length of the TRR is the number of audio frames required to provide a single frame of STFT output.

| TRR=TA(j),wj,k,wAj, wpj TAj-1,wj-1,k,wAj-1, wpj-1⋮ TAj-(NTRR-1),wj-(NTRR-1),k,wAj-(NTRR-1), wpj-(NTRR-1) | (457) |
| --- | --- |

The TRR is used to handle the case when a listener moves from one triangle into another. Special care is needed in this situation as the STFT (see 6.6.26.3.3.4) is not performed for every HOA source for every audio frame j. It is only performed for HOA sources which are required to compute the interpolated spatial metadata for audio frame j. When STFT output requires more audio data than what is available in a single audio frame, an HOA source cannot be used for spatial metadata processing until enough audio data has been accumulated.

Whenever a listener enters a triangle which has an HOA source for which the STFT buffers are not up-to-date, the last calculated interpolation weights from the previous triangle are used. When the listener has remained in the new triangle long enough for STFT to provide meaningful output, new interpolation weights are calculated based on the new triangle. For the first frame after the listener has remained long enough in the new triangle, the interpolation weights are calculated by interpolating between the last interpolation weights of the previous triangle and the first interpolation weights of the new triangle. The process is shown below and in Figure 88.

determine_chosen_weights(TRR, Tc(j-1)) {

# shift TRR values for frame

for (i=NTRR; i > 1; i--)  {

TRRi = TRRi-1;

}

TRR1=TA(j),wj,k,wAj, wAj-1;

isTriangleSwitched = False;

for (i=1; i <= NTRR; i++)  {

# go through track record

Ttemp=TRRi;

if (Tc(j-1)== Ttemp) {

# previous chosen triangle T_c(j-1) found in track record

Tcj, wcj,k,wcj, wcpj=TRRi;

return;

}

}

# previous chosen triangle T_c(j-1) was NOT found in track record, listener

# has been in new triangle for a long enough time

isTriangleSwitched = True;

Tcj, wcj,k,wcj, wcpj=TRRNTRR;

for (k=1; k <= Nsf; k++)  {

wc(j,k)=wcj*kNsf+wcpj*(1-kNsf)

}

TRR1=Tcj, wcj,k,wcj, wcpj

}

Figure 88 —Interpolation weight determination when the listener moves from a triangle to another.  a) The listener has remained in triangle {m1, m2, m4} at least 6 frames, STFT has been calculated for HOA sources m1, m2 and m3 and provides meaningful output. For m3, STFT has not been calculated and the buffers are not up-to-date. Interpolation weights calculated for the latest frame (j)are used. b) The listener moves to triangle {m2, m3, m4}, the signal for HOA source m3 is now fed into STFT, but for it the buffer is not yet full. In this case interpolation weights from the last frame of the previous triangle (j-2) are used. c) The STFT buffer for m3 fills and interpolation weights are calculated for both triangles. The interpolation weights used for this frame are interpolated between the interpolation weights of frames (j-5) and (j-4). d) The listener has remained in triangle {m2, m3, m4} for a while, STFT has been calculated for HOA sources m2, m3 and m4 and provides meaningful output. Interpolation weights are calculated based on listener position in frame j.

###### Determine HOA source for signal interpolation

The HOA source mc(j) which is chosen for signal interpolation for frame j shall be determined based on the chosen weights wcj, previous chosen HOA source mcj-1 and the chosen triangle Tc as follows:

determine_closest_HOA_source(wcj,  mcj-1, Tc) {

candidate= argmaxi=1..Nswcij

if (mcandidatej∈ Tc and mcj-1∈ Tc) {

if distance_between(mcandidatej,  pLj) > dT*distance_between(mcj-1,  pLj) {

mc(j) =  mcandidate(j)

} else {

mc(j) =  mc(j-1)

} else {

mc(j) =  mcandidate(j)

}

}

distance_between(ma, mb) function calculates the Euclidean distance between the HOA Sources ma and mb. dT is a distance threshold multiplier (1.2).

###### Crossfade

If the chosen HOA source is changed, crossfade shall be started. For this, fade-in and fade-out weights (wfij,k and wfoj,k) and fade-out bands Bfo(j) shall be calculated. The fade-in weights, fade-out weights and fade-out bands are used in metadata and signal interpolation during cross-fade (see sections 6.6.26.3.4.1 and 6.6.26.3.4.2).

check_cross_fade_start() {

if (crossFadeInProgress) {

get_fade_out_bands()

} else {

if (mc(j) !=  mc(j-1)) {

if (isTriangleSwitched) {

for (k = 1; k <= Nsf; k++) {

wfoj,k=wcj-1

}

} else {

wfoj,k=wc(j,k)

}

wfij,k=wc(j,k)

crossFadeInProgress = True;

framesUntillCrossFadeStart = 6;

get_fade_out_bands()

}

}

}

get_fade_out_bands() is used to obtain a list of fade-out bands Bfo(j). The size of the list is decreased as the cross-fade progresses. isTriangleSwitched is determined in determine_chosen_weights(), described above.

get_fade_out_bands() {

cfBandLow = {0, 2, 3, 4, 5, 6, 8, 10, 12, 14, 17, 20, 23, \

27, 32, 37, 43, 50, 57, 66, 76, 88, 101, 116}

cfBandHigh = {1, 2, 3, 4, 5, 7, 9, 11, 13, 16, 19, 22, 26,\

31, 36, 42, 49, 56, 65, 75, 87, 100, 115, 132}

cfBandSwaporder = {7, 1, 2, 22, 21, 12, 18, 0, 5, 9, 8, 14, 13, \

23, 15, 20, 19, 4, 16, 17, 10, 11, 3, 6}

for (k = 1; k <= Nsf; k++) {

if (framesUntilCrossFadeStart > 0) {

framesUntilCrossFadeStart--;

}

if (h->framesUntilCrossFadeStart == 0) {

++h->crossFadeProgressIndx;

}

for (int cfIdx = crossFadeProgressIndx; cfIdx < cfLen; ++cfIdx) {

bandIdx = crossfadeBandSwaporder[cfIdx]

for (band = h->cfBandLow[bandIdx]; band <= h->cfBandHigh[bandIdx]; ++band) {

Bfo(j,k)← band

}

}

}

}

Crossfade is stopped when crossFadeProgressIndx == cfLen, by setting crossFadeInProgress = False.

##### Spatial analysis

###### General

This section corresponds to the Spatial analysis block in Figure 86. As input, the block retrieves the set of audio signals corresponding to the HOA sources that are found in the TRR. As output, for each HOA source, a set of spatial metadata parameters, namely, azimuth, elevation, direct-to-total energy ratio and energy are calculated.

###### HOA format conversion

The input source signals, which are in the Equivalent Spatial Domain (ESD) representation shall be converted to Higher-Order Ambisonics (HOA) signals. This is achieved by multiplying the ESD signals with a conversion matrix MESDtoHOA. The conversion matrix is obtained as specified in [7] and [8]. .

| sHOA(i,j)=MESDtoHOAsESD(i,j) | (458) |
| --- | --- |

where sESD(i,j) is  a Nch×Lf matrix of containing the ESD input signals for HOA source i and frame index j, MESDtoHOA is the Nch×Nch conversion matrix and sHOA(i,j) is the Nch×Lf matrix of output HOA signals for HOA source i and frame index j.

###### Sub-framing

The frames of HOA signals shall be divided into Nsf sub-frames of equal length.

| sHOA(i,j,1)…sHOA(i,j,Nsf)=sHOA(i,j) | (459) |
| --- | --- |

where sHOA(i,j,k) is the HOA signal corresponding to the kth subframe.

###### Time-frequency domain conversion

A Short-time Fourier Transform is used to convert HOA format signals into time-frequency domain. For each subframe k of audio frame j and for each HOA Source i that is a part of a triangle found in the triangle track record (TRR, see clause 6.6.26.3.2.3) sHOA(i,j,k), a time-frequency domain signal matrix S(i,j,k) shall determined with the STFT forward transformation. STFT shall be configured with a hop size of 128 samples and band center frequencies according to A.10.  S(i,j,k) is a Nch×Nb  matrix containing the time-frequency domain signals of length Nb  for each HOA channel. For the HOA source that has been chosen for signal interpolation, STFT shall be run for all channels and S(i,j,k)  is a Nch×Nb  matrix containing the time-frequency domain signals of length Nb for each HOA channel. For all other HOA sources found in the triangle track record, STFT shall be run for only the first four channels (FOA) and S(i,j,k) is a 4×Nb  matrix containing the time-frequency domain signals of length Nb for the first four channels. During cross-fade (6.6.26.3.2.5), the HOA source that is the target of the crossfade, STFT shall also be run for all channels.

###### Signal analysis vector

A signal analysis vector si,j,k,b shall be calculated from the time-frequency domain signals S(i,j,k) as follows for each frequency bin b:

| si,j,k,b=Sb,1i,j,k*1.0Sb,2i,j,k*0.5774Sb,3i,j,k*0.5774Sb,4i,j,k*0.5774 | (460) |
| --- | --- |

where Sb,ci,j,k is the value in matrix Si,j,k corresponding to channel c and frequency bin b.

###### Intensity vector

An inverse sound field intensity vector that points to the opposing direction of the propagating sound shall be calculated from the signal analysis vector for frequency bins b≤bmax as follows.

| ii,j,k,b=Res1i,j,k,b*s4i,j,k,bs1i,j,k,b*s2i,j,k,bs1i,j,k,b*s3i,j,k,b, b≤bmax, | (461) |
| --- | --- |

where sci,j,k,b denotes the complex conjugate of sci,j,k,b. bmax is signaled by  highestSingleBinBandsIndex.

###### Signal energy

Signal energy shall be calculated from the analysis vector for each frequency bin b  as follows:

| ei,j,k,b=12c=14sci,j,k,b*sci,j,k,b | (462) |
| --- | --- |

###### Average intensity vector and energy

An average (over sub-frames) intensity vector and energy shall be calculated as follows.

| ei,j,b=1Nsfk=1Nsfei,j,k,b | (463) |
| --- | --- |
| ii,j,b=1Nsfk=1Nsfii,j,k,b, b≤bmax | (464) |

###### Azimuth and elevation

The direction of the average intensity vector shall be expressed as azimuth and elevation as follows:

| θi,j,b=atan2(i2i,j,b,i1i,j,b) | (465) |
| --- | --- |
| φi,j,b=atan2i3i,j,b,i1i,j,bi2i,j,b | (466) |

where ini,j,b is the nth element of the average intensity vector  ii,j,b, θi,j,b is the azimuth and φi,j,b is the elevation.

###### Direct-to-total energy ratio

The direct-to-total energy ratio ri,j,b shall be calculated from the average intensity vector and signal energy.

| ri,j,b=ii,j,bei,j,b | (467) |
| --- | --- |

###### Copy energy to subframes

The obtained energy ei,j,b for the jth frame is used as the as the energy for all sub-frames k for the jth frame.

| ei,j,k,b=ei,j,b, k∈1..Nsf | (468) |
| --- | --- |

##### Interpolation

To enable a 6DoF listening experience, the spatial metadata and time-frequency domain audio signal at the listener position for sub-frame k of frame j, shall be estimated.

###### Metadata interpolation

This section describes the processing done in the Spatial metadata interpolation block of Figure 86. As input this block receives the spatial metadata calculated in the Spatial analysis block and the chosen interpolation weights wcj,k (calculated based on the listener position HOA source positions). As output, the Spatial metadata interpolation block provides interpolated spatial metadata parameters at the listener position.

The estimation of the spatial metadata at the listener position shall be done by a weighted interpolation (using the chosen interpolation weights wcj,k, see clause 6.6.26.3.2.3) of the calculated spatial metadata.

For interpolation, the spatial metadata for the HOA sources is converted into vector form.

| vi,j,b=-sinθi,j,bcosφi,j,bsinφi,j,b-cosφi,j,bcosφi,j,bri,j,b, b≤bmax | (469) |
| --- | --- |

The spatial metadata data vectors shall be rotated according to the orientation of the HOA sources and the listener’s head.

| vi,j,b=RheadjRsourcei,jvi,j,b, b≤bmax | (470) |
| --- | --- |

where Rhead(j) and Rsource(i,j) are 3x3 rotation matrices for the listener head and source orientations, respectively for frame j.

The rotation matrices are obtained using OpenGL conventions as shown below.

| R=Rpitch*Rroll*Ryaw | (471) |
| --- | --- |

where

| Ryaw=cos⁡(yaw)0sin⁡(yaw)010-sin⁡(yaw)0cos⁡(yaw) | (472) |
| --- | --- |
| Rroll=1000cos⁡(roll)-sin⁡(roll)0sin⁡(roll)cos⁡(roll) | (473) |
| Rpitch=cos⁡(pitch)-sin⁡(pitch)0sin⁡(pitch)cos⁡(pitch)0001 | (474) |

For Rheadj the yaw, pitch and roll values are flipped as rotating the head means that source directions move in the opposite direction. For Rsourcei,j, the yaw, pitch and roll values are the hoaSourceOrientationYaw, hoaSourceOrientationPitch and hoaSourceOrientationRoll values from hoaSources() in payloadScene().

A weighted average spatial metadata vector, vj,k,b, shall be calculated for frequency bins b≤bmax using the interpolation weights wcj,k (see clause 6.6.26.3.2.3)  and the rotated spatial metadata vector.

| vj,k,b=i=1Nswcij,kvi,j,b, b≤bmax | (475) |
| --- | --- |

If a cross-fade is in progress (see clause 6.6.26.3.56.6.26.3.2.5), the weighted average spatial metadata vector shall be obtained using the following equation.

| vj,k,b=i=1Nswfoij,kvi,j,k,b, b∈Bfo and b≤bmaxi=1Nswfiij,kvi,j,k,b, b∉Bfo and b≤bmax | (476) |
| --- | --- |

A low frequency limit index blow for spatial metadata averaging shall be calculated as follows. First, the center frequency for frequency bin bmax  freqVec(bmax) (see A.10) is determined.  freqVec(bmax) is then rounded to the closest one-third-octave band center frequency. The third octave center frequencies shall be as defined in the ISO 266 (Acoustics — Preferred frequencies). The index of the closest one-third-octave frequency oneThirdOctaveIndexes(iclosest)  shall be determined from the table in A.11.

blow is then obtained as follows:

if (iclosest>0) {

blow=oneThirdOctaveIndexes(iclosest-1)

if (blow==bmax+1) {

blow=bmax

}

} else if (bmax>0) {

blow=bmax

} else {

blow=0

}

Interpolated spatial metadata shall then be calculated based on the weighted spatial metadata vector as shown below.

Azimuth shall be calculated for frequency bins b≤bmax as follows:

| θj,k,b=atan2-v1(j,k,b),-v3(j,k,b), b≤bmax | (477) |
| --- | --- |

and, for frequency bin bmax+1 azimuth shall be calculated as follows:

| θj,k,bmax+1=b=blowbmaxθj,k,bbmax-blow+1 | (478) |
| --- | --- |

Elevation shall be calculated for frequency bins b≤bmax as follows:

| φj,k,b=atan2-v2(j,k,b),-v1(j,k,b)2+-v3(j,k,b)2 | (479) |
| --- | --- |

and, for frequency bin bmax+1 elevation shall be calculated as follows:

| φj,k,b=atan2-v2(j,k,b),-v1(j,k,b)2+-v3(j,k,b)2 | (480) |
| --- | --- |

Direct-to-total energy ratio shall be calculated for frequency bins b≤bmax as follows:

| rj,k,b=v1(j,k,b)2+v2(j,k,b)2+v3(j,k,b)2 | (481) |
| --- | --- |

and, for frequency bin bmax+1 direct-to-total energy ratio shall be calculated as follows:

| rj,k,bmax+1=b=blowbmaxrj,k,bbmax-blow+1 | (482) |
| --- | --- |

Energy shall be calculated for all frequency bins b as follows:

| ej,k,b=i=1Nswcij,kei,j,k,b | (483) |
| --- | --- |

If a crossfade is in progress, the energy shall be obtained as follows:

| ej,k,b=i=1Nswfoij,kei,j,k,b,b∈Bfo i=1Nswfiij,kei,j,k,b, b∉Bfo | (484) |
| --- | --- |

###### Signal interpolation

This section corresponds to the Signal interpolation block of Figure 86 It receives the time-frequency domain signal corresponding to the HOA source chosen for interpolation (in the Position pre-processing block) and the energy parameters calculated in the Spatial analysis block and provides as output the interpolated signal at the listener position.

Signal interpolation shall be done by taking the chosen HOA source, mcj, to the listener position and applying equalization to it. The EQ gain Geq(j,k,b)  is calculated based on the interpolated signal energy ej,k,b and the energy of the HOA source mcj (see clause 6.6.26.3.2.4) emcj,j,k,b as follows:

| Geqj,k,b=minej,k,bemcj,j,k,b+ε,GeqMax | (485) |
| --- | --- |

where GeqMax is the maximum EQ gain value (4.0).

The interpolated signal S(j,k) is obtained by applying the gain over all frequency bins b and signal channels c as follows:

| Sb,cj,k=Geqj,k,bSb,cmcj,j,k | (486) |
| --- | --- |

where mc(j) is index of the HOA source chosen for interpolation (see clause 6.6.26.3.2.4).

During a crossfade (see clause 6.6.26.3.2.5), the above two equations are replaced with the following.

| Geqj,k,b=minej,k,bemcj-1,j,k,b+ε,GeqMax,b∈Bfominej,k,bemcj,j,k,b+ε,GeqMax,b∉Bfo | (487) |
| --- | --- |
| Sb,cj,k=Geqj,k,bSb,cmcj-1,j,k,b∈BfoGeqj,k,bSb,cmcj,j,k,b∉Bfo | (488) |

##### Binaural output

###### General

To obtain binaural output from the interpolated spatial metadata and signal, a prototype signal is generated from the interpolated signal, on which optimal mixing is performed to obtain an output signal in agreement with the interpolated spatial metadata. This section corresponds to the Mixing block of Figure 86.

###### Prototype signal generation

The interpolated signal Sj,k,b shall be rotated according to the listeners head orientation and the orientation of the HOA source from which the interpolated signal was obtained. First, 3×3 rotation matrices for the listener’s head orientation Rhead(j)  and the orientation of the HOA source from which  the interpolated signal was calculated from Rsource(j) are obtained.  These are then multiplied to obtain a rotation matrix  R(j). See clause 6.6.26.3.4.1 for details on calculating  Rheadj and  Rsource(j).

| Rj= Rhead(j)Rsource(j) | (489) |
| --- | --- |

From R(j), a spherical harmonics rotation matrix  Rshj (Nch×Nch) shall be calculated according to 6.6.23.3.2.2. The prototype signal Bj,b for frame j and all frequency bins b is then obtained as follows:

| Bj,b=MHOA2bin(b)*Rsh(j)*Sb,1j,1…Sb,1j,Nsf⋮⋱⋮Sb,Nchj,1…Sb,Nchj,Nsf | (490) |
| --- | --- |

where MHOA2bin(b) is the Ambisonics to binaural matrix for frequency bin b, calculated in the pre-processing phase.

###### Optimal mixing

###### Signal and target covariance matrices

Mixing matrices that are used to obtain the final binaural output shall be calculated using optimal mixing, see Vilkamo [10] for more information. First, covariance matrices Cx and Cy shall be estimated (frequency bin-wise) from binaural time-frequency domain signal Bj,b and interpolated spatial metadata, respectively. A recursive averaging scheme shall be applied when calculating both Cx and Cy.

To obtain Cx, first the covariance matrix for Bj,b shall be calculated as shown below for frequency bins , b≤bmax.

| Cxnewj,b=Bj,bBj,bH, b≤ bmax | (491) |
| --- | --- |

For frequency bins b>bmax, only the diagonal values shall be calculated:

| Cx,1,1newj,b=B1,1j,bB1,1 Hj,b+B1,2j,bB1,2 Hj,b, b>bmax | (492) |
| --- | --- |
| Cx,2,2newj,b=B2,2j,bB2,2 Hj,b+B2,1j,bB2,1 Hj,b, b>bmax | (493) |

Recursive averaging shall then be applied to obtain Cx for the current frame j as follows.

| Cx(j,b)=(1-∂)Cxnew(j,b)+∂Cx(j-1,b) | (494) |
| --- | --- |

where ∂=0.9 is the recursion smoothing coefficient.

Cy shall be obtained from the interpolated spatial metadata and HRTF data . Direct portion of Cy shall be obtained as follows:

| Cydirectj,b= k=1Nsfej,k,brj,k,bHb,dHHb,d, b≤bmax | (495) |
| --- | --- |

where Hb,d is the value of the HRTF for frequency bin b, d is the direction index corresponding to the interpolated azimuth θj,k,b and elevation φj,k,b, (see 6.6.26.3.1.2). For frequency bins b>bmax, only the diagonal values shall be calculated:

| Cy,n,ndirectj,b= k=1Nsfej,k,brj,k,bmax+1HHn,nbmax+1,d , | (496) |
| --- | --- |

where HHn,nbmax+1,d is precalculated and reused for each frequency bin b≤bmax,  and shall be defined as follows:

| HHbmax+1,d=Hbmax+1,dHHbmax+1,d | (497) |
| --- | --- |

The diffuse portion of Cy shall be calculated as follows.

| Cydiffuse(j,b)=k=1Nsf1-rj,k,bej,k,bCdifb, b≤bmax, | (498) |
| --- | --- |

and

| Cy,n,ndiffusej,b=k=1Nsf1-rj,k,bmax+1ej,k,bCdif,n,nb, b>bmax,n∈1,2 | (499) |
| --- | --- |

where:

| Cdifb=d=1NdHb,dHHb,dNd | (500) |
| --- | --- |

where the sum shall be calculated over all direction indices (see 6.6.26.3.1.2). Cynewj,b shall then be obtained by summing Cydirectj,b and Cydiffusej,b:

| Cynewj,b=Cydirectj,b+Cydiffuse(j,b) | (501) |
| --- | --- |

Recursive averaging shall then be applied to obtain Cy for the current frame j as follows.

| Cy(j,b)=(1-∂)Cynew(j,b)+∂Cy(j-1,b) | (502) |
| --- | --- |

where ∂=0.9 is the recursion smoothing coefficient.

###### Covariance matrices for frequency bands

If hoaGroupHasFreqBandConfig is set to true, the frequency bin specific signal covariance matrices Cx(j,b) and Cy(j,b) shall be averaged to obtain frequency band specific signal covariance matrices Cx(j,q) and  Cy(j,q) as follows:

| Cy(j,q)=1erbUpperIndexesq-erbLowerIndexesq+1b=erbLowerIndexes(q)erbUpperIndexes(q)Cy(j,b) | (503) |
| --- | --- |
| Cx(j,q)=1erbUpperIndexesq-erbLowerIndexesq+1b=erbLowerIndexes(q)erbUpperIndexes(q)Cx(j,b) | (504) |

where erbUpperIndexes(q) and erbUpperIndexes(q) shall define the upper and lower frequency band limit indices (see 6.6.26.3.1.4).

If hoaGroupHasFreqBandConfig is set to false, the frequency band specific signal covariance matrices shall not be calculated.

Note that in the following description it is assumed that frequency bands have been created. However, in the case of no frequency bands, the frequency band indices q shall  be substituted with frequency bin indices b. Calculations in 6.6.26.3.5.3.9 shall be skipped in this case.

###### Configure mixing matrix calculation

Mixing matrices M(j,q) and Mr(j,q) shall be calculated based on the covariance matrices Cy(j,q) (target covariance at listener position) and Cx(j,q) (covariance of interpolated signal) such that the covariance matrix of the output x, obtained by mixing x=My+MrD, is aligned with Cy.

For each frequency band q, mixing matrix calculation shall be configured by the function determine_computed_mixing_matrices(q) as follows:

determine_computed_mixing_matrices(q)

{

band = bottomBandIndex(q)

if (h->freqVec[band] < h->lowerBandFreqLimit) {

useOptimalMixingForM = true;

useOptimalMixingForMr = true;

useEnergyFLAG = false;

} else if (band < h->maxFreqLimitIndex) {

useOptimalMixingForM = true;

useOptimalMixingForMr = false;

useEnergyFLAG = true;

} else {

useOptimalMixingForM = false;

useOptimalMixingForMr = false;

}

};

###### Calculate mixing matrix,useOptimalMixingForM == true, useEnergyFLAG == false

For frequency bands q, for which useOptimalMixingForM is true, the covariance matrices shall be normalized as follows:

| Cxj,q=Cxj,qEmax,ch | (505) |
| --- | --- |

and

| Cyj,q=Cyj,qEmax,ch | (506) |
| --- | --- |

where

| Emax,ch=maxEx,1, Ex,2,Ey,1,Ey,2, | (507) |
| --- | --- |

where

| Ex,1=realCx1,1(j,q)Ex,2=realCx2,2(j,q)Ey,1=realCy1,1(j,q)Ey,2=realCy2,2(j,q) | (508) |
| --- | --- |

Then, the normalized target covariance matrix Cyj,q is decomposed with Cholesky decomposition as:

| Cyj,q=KyKyH | (509) |
| --- | --- |

where

| Ky=Ey,10Cy2,1Ey,1max0,Ey,2-Cy1,2Cy2,1Ey,1, if Ey,1>Ey,2 | (510) |
| --- | --- |
| Ky=max0,Ey,1-Cy1,2Cy2,1Ey,2Cy1,2Ey,20Ey,2, if Ey,1≤Ey,2 | (511) |
| Ey,1=realCy1,1(j,q),Ey,2=realCx2,2(j,q) | (512) |

The normalized input covariance matrix Cx(j,q)  shall be decomposed with eigendecomposition as:

| Cx(j,q)=UxSxUxH | (513) |
| --- | --- |

where Sx consists of the eigenvalues of Cx(j,q) on the diagonal. Ux shall be constructed from the eigenvectors of Cx(j,q) as:

If Ex,2-realSx1, 1>Ex,1-realSx1, 1 :

| Ux1, 1= Sx1, 1-Ex,22Cx1,2Cx2,1+Sx1, 1-Ex,22 | (514) |
| --- | --- |

| Ux2,1= Cx2,12Cx1,2Cx2,1+Sx1, 1-Ex,22 | (515) |
| --- | --- |

else:

| Ux2, 1= Sx1, 1-Ex,12Cx1,2Cx2,1+Sx1, 1-Ex,12 | (516) |
| --- | --- |
| Ux1, 1= Cx1,22Cx1,2Cx2,1+Sx1, 1-Ex,12 | (517) |

If Ex,2-realSx2, 2>Ex,1-realSx2, 2 :

| Ux1, 2= Sx2, 2-Ex,22Cx1,2Cx2,1+Sx2, 2-Ex,22 | (518) |
| --- | --- |

| Ux2, 2= Cx2,12Cx1,2Cx2,1+Sx2, 2-Ex,22 | (519) |
| --- | --- |

else:

| Ux2, 2= Sx2, 2-Ex,12Cx1,2Cx2,1+Sx2, 2-Ex,12 | (520) |
| --- | --- |
| Ux1, 2= Cx1,22Cx1,2Cx2,1+Sx2, 2-Ex,12 | (521) |
| Ex,1=realCx1,1(j,q), Ex,2=realCx2,2(j,q) | (522) |

If the input matrix Cx is close to a zero matrix or to an identity matrix with a gain, the Ux matrix shall be set as an identity matrix. The decomposition shall be further continued with:

| Cx(j,q)=KxKxH | (523) |
| --- | --- |
| Kx=UxSx | (524) |

A matrix A is calculated based on the decomposition:

| A=KyHGyQKx | (525) |
| --- | --- |

where Gy is a normalization matrix formulated from the channel energies:

| Gy=Ey,1maxEx,1, 0.001*Ex,200Ey,2maxEx,2, 0.001*Ex,1 | (526) |
| --- | --- |

and Q is an identity matrix. he nearest orthonormal matrix P to A is searched, where P=A1AHA. The term AHA shall be decomposed with eigendecomposition:

| AHA =UASAUAH | (527) |
| --- | --- |

The decomposition is performed similarly as for Cx above. The eigenvalues for 1AHA shall be obtained with:

| SA =1SA | (528) |
| --- | --- |

which results in the decomposition of:

| 1AHA =UA1SAUAH | (529) |
| --- | --- |

The orthonormal matrix P shall be obtained with:

| P =A*UA1SAUAH | (530) |
| --- | --- |

The optimal mixing matrix M shall be obtained with:

| M =KyPKx,reg-1 | (531) |
| --- | --- |

Kx,reg represents a regularized matrix of Kx and is obtained by regularizing the Sx term in 
Kx=UxSx. The diagonal values for the regularized term shall be obtained with:

| Sxreg1, 1=maxrealSx1, 1, ω*realSx2, 2 | (532) |
| --- | --- |
| Sxreg2, 2=maxrealSx2, 2, ω*realSx1, 1 | (533) |

where ω=0.2 is a regularization factor. Because Ux is orthogonal, the inversion of Kx can be written as:

| Kx-1=(UxSx)-1=Sx-1UxH=UxHSx | (534) |
| --- | --- |

The mixing matrix shall then be obtained with:

| M(j,q) =KyPUxHSxreg | (535) |  |
| --- | --- | --- |
| A residual matrix Cr shall be computed as follows: Cr(j,q) =Cy(j,q)-M(j,q)Cx(j,q)MH(j,q) | (536) |

###### Calculate mixing matrix, useOptimalMixingForM == true, useEnergyFLAG == true

The same procedure as described in 6.6.26.3.5.3.4 (equations (505) to (535)). The residual matrix calculation shall be replaced with the following compensation:

| M(j,q)=diagCyj,qMj,qCxj,qMHj,q*M(j,q) | (537) |
| --- | --- |

###### Calculate mixing matrix, useOptimalMixingForM == false

When useOptimalMixingForM is set to false, the mixing matrix Mj,b is a diagonal matrix with the diagonal elements that shall be calculated as:

| Ml,lj,q= Cyl,l(j,q)Cylinl,lj,q+ε | (538) |
| --- | --- |

where

| Cylin(j,q)=Cx1,1j,q00Cx2,2j,q | (539) |
| --- | --- |

###### Calculate residual mixing matrix, useOptimalMixingForMr==true

The residual mixing matrix Mr(j,q) shall be calculated using the equations (505) to (535), but with Cy(j,q) replaced with Crj,q calculated above in (536). And Cx(j,q) replaced with Cylin(j,q) (see equation (539).

###### Calculate residual mixing matrix, useOptimalMixingForMr==false

When useOptimalMixingForMr is false, the residual mixing matrix Mr(j,q) is a zero matrix.

###### Copy to frequency bands

Frequency bin specific mixing matrices shall be obtained from the requency band specific mixing matrices as follows:

| Mj,b= Mj,q, b∈[erbLowerIndexesq,erbUpperIndexesq] | (540) |
| --- | --- |
| Mrj,b= Mrj,q, b∈[erbLowerIndexesq,erbUpperIndexesq] | (541) |

###### Equalize mixing matrices

Equalization shall be applied on the mixing matrices as follows:

| Mj,b= EQbinbMj,b | (542) |
| --- | --- |
| Mrj,b= EQbinbMr(j,b) | (543) |

###### Linear interpolation of mixing matrices

Linear interpolation between the obtained mixing matrices Mj,b and Mrj,b and the mixing matrices from the previous audio frame Mj-1,b and Mrj-1,b  shall be used to obtain mixing matrices for the sub-frames as follows:

| M(j,k,b)=Mj,b*kNsf+Mj-1,b*(1-kNsf) | (544) |
| --- | --- |
| Mr(j,k,b)=Mrj,b*kNsf+Mrj-1,b*(1-kNsf) | (545) |

###### Binaural output

The output time-frequency domain signal for the sub-frame k of audio frame j and frequency bin b Oj,k,bshall then be obtained using the following equation:

| Oj,k,b=Mj,k,b*Bj-1,k,b+Mrj,k,b*Dj,b | (546) |
| --- | --- |

where Dj,b is a decorrelated time-frequency domain signal obtained from a buffer of previous binaural signals B obtained as follows:

for (b = 1; b <=Nb; b++) {

for (c = 1; c <=2; c++) {

delay=decorrelationDelays(b*2 + c)

Dc(j,b) = Bc(j-delay,b)

}

}

decorrelationDelays() is calculated during the initialization as follows. First, for each frequency band b, determine a delay range, which is defined by  Dmin and Dmax.

| Dmax(b)=50000freqVecb+2.23*10-9 | (547) |
| --- | --- |

| Dmin(b)=10000freqVecb+2.23*10-9 | (548) |
| --- | --- |

For each frequency bin b, the range of values between Dmin(b) and Dmax(b) is divided into as many intervals of equal length as there are output channels (c). Each of these intervals is then randomly assigned to each channel. The delay value for frequency bin b and channel c is then randomly chosen from this interval and inserted into decorrelationDelays(). freqVec[b] holds the channel center frequencies.

As a final step,  the binaural time-frequency domain signal Oj,k,b shall be converted into a time domain signal soutj with the afSTFT backward transformation.

#### Exterior rendering

The triangulation process in 6.6.26.3.1.1 creates a capturing region (see Figure 87). When the listener is inside the region, he is enclosed by a triangle defined by set of HOA sources. When the listener moves outside of the region, the listener is no longer enclosed by a triangle. Thus, an alternate processing scheme is required for obtaining interpolation weights. When the listener is outside of the region (exterior rendering mode), the listener position shall be projected to the edge of the capturing region, and the spatial metadata values of the sources shall be modified accordingly.

pL,proj	projected listener position (x, y, z)

pHOA,i	position (x, y, z) of the ith HOA source

eij	exterior edge formed by the HOA sources i and j

Exvalid	valid exterior edges, that enclose the capturing region after long edges are omitted

Exorig	original exterior edges (includes the omitted edges)

Exomit	omitted exterior edges

nL,proj 	normal at the projected listener position

nHOA,i	normal of the ith HOA source

nij	normal of the exterior edge formed by the HOA sources i and j

Rproj	radius of projection circle, constant value 4.0m

##### Initialization

###### Original exterior edges

Figure 89 represents a capturing region after the triangulation process. The edges connecting the outermost HOA sources form the exterior edges. In Figure 89, the exterior edges are (e12, e23,e34,e41). These are denoted as Exorig.

Figure 89 — Triangulation of a capturing region

###### Omitted exterior edges

The triangulation process can produce triangles that have long edges. This leads to ineffective spatial interpolation between the connected HOA sources. For improved audio quality, these long edges shall be omitted. This case is shown in Figure 90, where the edge e41 and the related triangle T4 is omitted.

Figure 90 — Triangulation of a capturing region, omitting the edge e41

The omitted edges are used in the exterior processing, denoted as Exomit The new valid exterior edges are denoted as Exvalid In Figure 90, Exomit=e41 and Exvalid=(e12, e23,e34,e45,e51).  Exorig remains the same from the original processing. All the edges are sorted in a traversing order that encloses the capturing region, when the edges are processed sequentially.

###### Exterior edge normals

For the edges in Exorig and Exomit, the edge normals are the normals pointing outwards from the capturing region. In Figure 91, n12 and n14 represent these normals.

In Exvalid, after omitting long exterior edges, there can be some edges, that form a non-convex hull for the exterior shape. For these edges, the normal of the omitted edge shall be used. In Figure 91, the edge e41 is omitted. The normals  n14 and  n45  are replaced with  n14.

Figure 91 — Normals of the exterior edges

###### Exterior HOA source normals

For the HOA sources on the edge of the capturing region, the normals shall be calculated as:

| nHOA=ne1+ne2ne1+ne2 | (549) |
| --- | --- |

where ne1 and ne2 are the normals of the connecting edges, and nHOA is the resulting normal for the edge HOA source. The edge normals are searched within Exorig. For HOA sources that are inside a non-convex shape, the normals shall be replaced with the normals of the omitted edges. This is shown in Figure 92.

Figure 92 — Normals of the exterior HOA sources

##### Projection of the listener position

###### Find closest HOA source

The closest HOA source to the listener shall be determined by the squared horizontal distance:

| DHOA,i2=(pHOA,i,x-pL,x)2+ (pHOA,i,y-pL,y)2 | (550) |
| --- | --- |

where DHOA,i2 is the squared horizontal distance from the listener position pL to an HOA source position pHOA,i, and x and y mark the horizontal components of both positions.

###### Find closest exterior edge

The distance from the listener position to an exterior edge is determined by a line parallel to the edge normal. The calculations are computed in the horizontal plane, left un-noted in the following equations for simplicity. An edge vector between two HOA sources is determined by:

| v12=pHOA,2-pHOA,1 | (551) |
| --- | --- |

and normalized by:

| v12=v12v12 | (552) |
| --- | --- |

The listener position in the first HOA source space shall be given by:

| pL,HOA1=pL-pHOA,1 | (553) |
| --- | --- |

The dot product between the normalized edge vector and the edge normal n12 is:

| dotv,n=v12⋅n12 | (554) |
| --- | --- |

The distance from the listener position to the edge shall be given by:

| D12=pL,HOA1⋅n12 ,if dotv,n==0pL,HOA1⋅n12-pL,HOA1⋅v12dotv,n dotv,n-1dotv,n ,else | (555) |
| --- | --- |

The edge weight shall be given by:

| w12=1-pL,HOA1⋅v12v12 ,if dotv,n==01-pL,HOA1⋅v12-pL,HOA1⋅n12dotv,n dotv,n-1dotv,n/v12 ,else | (556) |
| --- | --- |

If the listener is positioned outside all the valid and omitted triangles, the closest edge shall be searched within Exorig. This is the case for the listener position pL,1 in Figure 93.

If the listener is inside an omitted triangle, only the Exvalid edges shall be processed. This is the case for the listener position pL,2 in Figure 93. The distance to the edge e45 is calculated parallel to the normal n14.

When searching edges within Exorig, it is possible, that the closest edge to the listener is an omitted edge. This is the case for the listener position pL,3 in Figure 93, where the edge e14 is omitted. In this case, the distance to the closest edge shall be the distance to the omitted edge, D14 in Figure 93, but the listener is projected to an edge that can be found in Exvalid. In Figure 93, it is the edge e15 for pL,3. The process of projecting the listener to an exterior is explained in Figure 93.

Figure 93 — Distances and edge weights to exterior edges and HOA sources

###### Listener projection to the closest exterior HOA source

If an exterior HOA source is closer to the listener than any of the exterior edges, the listener shall be projected on top of the HOA source as:

| pL,proj=pHOA,i | (557) |
| --- | --- |

###### Listener projection to the closest exterior edge

If an exterior edge is closer to the listener than any of the exterior HOA sources, the listener shall be projected to the edge as:

| pL,proj=w12pHOA,1+(1-w12)pHOA,2 | (558) |
| --- | --- |

where pHOA,1 and pHOA,2 are the positions of the HOA sources forming the edge and w12 is the edge weight calculated with Equation (556).

The closest edge to the listener can be an omitted edge, as explained at the end of 6.6.26.4.2.2. In this case, the listener shall be projected to an exterior edge, that can be found in in Exvalid. In Figure 93, the closest edge for the listener position pL,3 is e14, which is not found in Exvalid. The listener is therefore projected to the edge e15.

##### Interpolation weights for the projected listener position

In the exterior rendering, the interpolation weights w(j) (as in 6.6.26.3.1.5) are calculated from the projected listener position. If the listener is projected on top of an HOA source, the weight for the projected HOA source shall be set to 1, and the rest of the weights shall be set to 0.

If the listener is projected to an exterior edge, the interpolation weights are obtained directly from the exterior weights:

| w1,j= w12 | (559) |
| --- | --- |
| w2,j= 1-w12 | (560) |

where w12 is the exterior weight between two HOA sources, calculated with Equation (556). Weights w1,j and w2,j are the weights for the HOA sources connecting the projected edge.

##### Calculate the normal for the projected listener position

If the listener is projected on top of an HOA source, the normal of the HOA source shall be used as the projected listener normal:

| nL,proj=nHOA | (561) |
| --- | --- |

If the listener is projected to an exterior edge, the normals of the connected HOA sources shall be interpolated at the listener position to obtain the projected listener normal:

| nL,proj=w12nHOA,1+(1-w12)nHOA,2 | (562) |
| --- | --- |

##### Adjust the spatial metadata outside the capturing region

###### Modify the DOA and direct-to-total energy ratio values

A unit DOA vector from the projected listener position to the edge of the projection circle is:

| uDOA=-sinθcosφsinφ-cosθcosφ | (563) |
| --- | --- |

and the weighted vector:

| uDOA=uDOARproj | (564) |
| --- | --- |

where θ and φ represent the interpolated azimuth and elevation angles, respectively, and Rproj is the radius of a projection circle. The value for Rproj is set by the bitstream parameter exteriorRenderingProjectionRadius.  A modified DOA vector from the original listener position to the edge of the projection circle shall be obtained with:

| uDOA,mod=pL,proj+uDOA-pL | (565) |
| --- | --- |

and the unit-length version with:

| uDOA,mod=uDOA,moduDOA,mod | (566) |
| --- | --- |

The DOA modification is shown in Figure 94. The direct-to-total energy ratio value shall be modified as:

| rmod=min⁡(r, uDOA,modRproj) | (567) |
| --- | --- |

where r is the interpolated direct-to-total energy ratio value.

Figure 94 — Modified DOA vector

###### Apply directional weighting

Directional weighting is applied to more heavily modify parameters from sound sources outside the capturing region, while applying less modifications to sources inside the capturing region. The directional weighting shall be calculated as:

| wdir=122(1+nL,proj⋅uDOA)2 | (568) |
| --- | --- |

The weighted modified DOA shall be calculated as:

| uDOA,mod,weighted=wdiruDOA,mod+(1-wdir)uDOA | (569) |
| --- | --- |

and the direct-to-total energy ratio shall be calculated as:

| rmod,weighted=min⁡(1, wdirrmod+(1-wdir)r) | (570) |
| --- | --- |

rmod,weighted will always be less than or equal to the original interpolated direct-to-total value r, meaning that sound sources will be rendered equally or more diffuse during exterior rendering. As the listener gets closer to a projection (uDOA) on the circle, diffuseness increases. rmod,weighted goes to zero, i.e. fully diffuse, when uDOA,modis zero.

###### Apply head rotation

In exterior rendering, the rotation in 6.6.26.3.4.1 (470) is skipped and the head rotation is applied after the exterior processing as:

| uDOA,mod,rotated=RheaduDOA,mod,weighted | (571) |
| --- | --- |

The modified spatial metadata values shall be calculated as in 6.6.26.3.3 (see (465), (466), (467) and (462)) and saved to the interpolated metadata buffers for the next steps of the processing.

##### Diffuse weighting for the covariance matrix Cydiffuse

The diffuse exterior weight shall be calculated as:

| wdiff=min⁡(1, pL,xy-pL,proj,xyRproj) | (572) |
| --- | --- |

where pL,xy and pL,proj,xy are the listener position and the projected listener position in the horizontal plane. In exterior rendering, the diffuse covariance matrix in 6.6.26.3.5.3 (equation (498)) shall be replaced with:

| Cydiffuse=(1-rmod,weighted)eCweighted | (573) |
| --- | --- |

| Cweighted=wdiffDCMdiff+(1-wdiff)DCMambi | (574) |
| --- | --- |

| DCMdiff=CdifNsfn=12Cdif(n,n) | (575) |
| --- | --- |

| DCMambi=B100B2/(B1+ B2) | (576) |
| --- | --- |

where Bn is the norm of the nth binaural channel.

#### Rendering of informed sources

When informed sources (hoaGroupHasInformedSources == True) are present informed source processing shall be performed. Informed source processing modifies the target covariance matrix Cx(j,b) computation described in 6.6.26.3.5.3.1.

##### Pre-processing for informed sources

###### Geometric parameters

Figure 95 — An example MPHOA scene with three HOA sources pHOA(1…3), one interior informed source psrc1 and one exterior informed source psrc2.

Figure 95 shows an example scenario with three HOA sources and two informed sources, one inside and one outside the triangulated capturing area. For each HOA source position pHOA(i), a vector to each informed source position psrcm is calculated as:

| vi, m=psrcm-pHOA(i) | (577) |
| --- | --- |

The vectors are rotated according to the orientation of each HOA source:

| vroti, m=RHOA(i)vi, m | (578) |
| --- | --- |

where RHOA(i) is the rotation matrix for HOA source i in OpenGL convention. The time delay in samples for each HOA source and informed source pair shall be obtained with:

| dsamplesi, m=vi, mc*fs | (579) |
| --- | --- |

where c=343 m/s is the speed of sound and fs the sampling rate. The delay between two HOA sources i1 and i2 for a single informed source shall be obtained with:

| dsamplesi1,i2, m=dsamplesi2, m-dsamples(i1, m) | (580) |
| --- | --- |

The delay in short-time Fourier transform (STFT) hop sizes shall be obtained with:

| dhopi1,i2, m=rounddsamplesi1,i2, mLhop | (581) |
| --- | --- |

where Lhop is the set hop length.

For each informed source m, the closest HOA source iclosestm shall be determined by the length of v. The attenuation gain between iclosestm and some other HOA source i for informed source m shall be obtained with:

| gatti, m=viclosest(m), mvi, m | (582) |
| --- | --- |

###### Geometric processing for exterior informed sources

If an informed source is located outside the triangulated capturing area, additional parameters shall be calculated. In Figure 95, the source psrc2 is located outside the capturing area. The projected position of the source is at psrc,proj2, and the distance to the projected position from the original position is D1. The distance to the closest HOA source, v2, 2, shall be compared to D1 and the shorter distance is saved as the exterior distance Dext2 for psrc2.

The distance calculations shall be executed similarly as in 6.6.26.4.2.1 and 6.6.26.4.2.2, when looking for the closest HOA sources and exterior edges for the listener position. Only the valid exterior edges Exvalid and normals nExvalid shall be considered for the exterior informed source calculations.

The exterior distances shall be calculated for all the exterior informed sources in the scene. The average distance to all the exterior informed sources is saved for later processing.

| Dext,avg=m=1NextDext(m)Next | (583) |
| --- | --- |

###### Pre-processing for beamforming

For each informed source m, beamforming weights w(iclosest(m),m) that focus the beam pattern from the closest HOA source iclosestm towards the informed source m are determined. Real spherical harmonics shall be used as the basis for the beamforming weights, determined as:

| Ynkθ,σ=2n+1n-k!n+k!Pnk(cosσ)*2sinkθ, if k<0 1, if k==02coskθ, if k>0 | (584) |
| --- | --- |

where (θ,σ) is the direction of interest in azimuth and inclination angles, n is the order 0, nHOA, k is the degree -n, n and Pnk is the Legendre function of order n and degree k. The Legendre functions are present without the Condon-Shortley phase. nHOA is the order of the input HOA signal, see Rafely [12] for more information.

For each HOA source iclosestm and informed source m pairs, the direction (θ,σ) is obtained from vroticlosest(m), m. Denoting Y(iclosest(m),m) as the set of the real spherical harmonic functions for the pair (iclosest(m),m) for all the orders n∈0, niclosest(m) and degrees k∈-n, n, the beamforming weights shall be obtained from the normalized real spherical harmonics:

| wiclosest(m), m=Yiclosest(m),m(niclosest(m)+1)2 | (585) |
| --- | --- |

For each HOA source i and informed source m pairs, the outer product of Yi,m and Yi,mT shall be calculated up to order 1:

| yyi, m=Yi,mYi,mT, for n≤1 | (586) |
| --- | --- |

##### Determine active informed sources

For each frame j, active informed sources shall be determined by their distance to the listener position. The mean listener position shall be calculated from the current and previous frame positions:

| pl, meanj=plj+plj-12 | (587) |
| --- | --- |

The distance between the mean listener position and each informed source is:

| Dl2srcj,m=psrcm-pl,mean(j) | (588) |
| --- | --- |

If an informed source m is close enough to the listener position, i.e., Dl2srcj,m<Dmax, and the distance between the informed source m and its closest HOA source is small enough, i.e. viclosest(m), m<DHOA2src,max, the source shall be marked as a candidate for active sources. Dmax is signaled as lpdInformedSourceEnableThreshold and DHOA2src,max is signaled as hpdInformedSourceEnableThreshold. If the number of candidate active sources exceeds the maximum number of active informed sources Nsrc,max, (signaled with maxSimulInformedSources) candidate active informed sources are chosen in priority order (informed source priority is signaled with priorityValue) such that number of active informed sources does not exceed Nsrc,max. The closest HOA source for each candidate shall be stored in list of source arrays SA(j):

| SAj=iclosestm1, iclosestm2,…iclosestmN | (589) |
| --- | --- |

where iclosestmn represents the closest HOA source for the nth informed source and N is the number of candidates for the current frame j. The list SA(j) is stored in a source track record STR, which stores SA's from the current and previous audio frames:

| STR=SAjSAj-1⋮SA(j-(NTRR-1)) | (590) |
| --- | --- |

Each HOA source i found in STR is transformed via STFT (see 6.6.26.3.3.4) during the current processing loop. NTRR limits the number of previous frames saved in the track record. The limit NTRR is the minimum number of frames required to obtain output from the STFT processing, similarly as in the case of the triangle track record TRR in 6.6.26.3.2.3.

In order to be sure that the STFT has been processed long enough for an HOA source iclosestm in SA(j), the HOA source shall be searched from the last entries of STR and TRR. If the HOA source is found from the end of either of the track records, the associated informed source m shall be marked as an active source. The further informed mode processing shall be performed only to the active sources mactive. If no source is active, the original MPHOA processing shall be performed instead of the informed mode processing.

##### Obtain beamformed source signals

The beamformed source signal for an informed source m shall be obtained with:

| Ssrcj,k,b,m= wiclosestm, mSj,k,b,iclosestm | (591) |
| --- | --- |

where wiclosest(m), m shall be the beamforming weights for the closest HOA source, obtained with Equation (585) and Sj,k,b,iclosest(m) is the input signal of the closest HOA source in time-frequency domain for frame j, sub-frame k and frequency band b.

##### Post-filter source signals

The post-filtered source signal shall be obtained with:

| Ssrcj,k,b,m=Gpostj,k,b,mSsrcj,k,b,m | (592) |
| --- | --- |

where Gpostj,k,b is a post-filter gain. A cross-pattern coherence (CroPaC) based post-filter shall be used, calculated as:

| Gpostj,k,b,m=realSfig8j,k,b,iclosest(m)Sj,k,b,iclosest(m),0HEFOAj,k,b,iclosest(m) | (593) |
| --- | --- |

where EFOA(j,k,b,iclosest) is the energy from the first four channels of S(j,k,b,iclosest(m)). Sfig8j,k,b,iclosest(m) is a figure-of-eight beamformer applied to the source signal:

| Sfig8j,k,b,iclosest(m)=vroticlosest(m), m,1 Sj,k,b,iclosestm,4+vroticlosest(m), m,2 Sj,k,b,iclosestm,2+vroticlosest(m), m,3 Sj,k,b,iclosestm,3 | (594) |
| --- | --- |

The values for the post-filter are limited as:

| 0≤Gpostj,k,b,m≤1 | (595) |
| --- | --- |

The obtained source signals shall be stored in a source signal buffer for time-alignment purposes (see 6.6.26.5.5). The buffer holds the source signals for the previous Nssb subframes.

Nssb=dhop,max mod Nsf+dhop,max+2Nsf-1

##### Time-align source signals

Each post-filtered informed source signal Ssrcj,k,b,m shall be time-aligned at each HOA source position as:

| Ssrcj,k,b,i,m=Ssrct(j,i,m),k,b,m | (596) |
| --- | --- |

where t(j,i,m) marks the time-aligned audio frame:

| t(j,i,m)=j-min⁡(dhopi,iclosest(m), m,dhop,max) | (597) |
| --- | --- |

dhop,max is the defined maximum number of delayed hops for the scene. dhopi,iclosest(m), m is the delay between some HOA source i and the closest HOA source iclosestm for the informed source m. The delay shall be calculated in the pre-processing phase in Equation (581).

The informed source signals shall be time-aligned similarly at the listener position:

| Sl.srcj,k,b,m=Ssrct(j,il,closest,m),k,b,m | (598) |
| --- | --- |

where il,closest indicates the closest HOA source to the listener.

##### Estimate HOA-to-source SCMs

The signal covariance matrices (SCM) for the signals from the HOA arrays towards the informed sources shall be obtained with:

| Csrcj,b,i=m ∈ mactivegatt(i,m)2*Esrcj,b,i,m*yyi,m | (599) |
| --- | --- |

where Esrcj,b,i,m is the energy of the time-aligned signal Ssrcj,b,i,m for the whole frame j, gatt(i,m) shall be obtained from Equation (582)  and yyi,m from Equation (586).

##### Estimate listener-to-source SCM

The rotated vector from the listener to an informed source m shall be:

| vl,rotj,m=Rhead(j)psrcm-pl,meanj | (600) |
| --- | --- |

where Rhead(j) is the head rotation matrix, which is formed from the flipped yaw, pitch and roll angles of the head orientation. The energy for the time-aligned signal Sl,srcj,b,m for the whole frame j shall be marked as El,srcj,b,m. The attenuation gain shall be calculated as:

| gl,attj,iclosestm,m=minviclosestm, mvl,rotj,m,viclosestm, mDl,max | (601) |
| --- | --- |

where Dl,max determines the distance from the source, where the attenuation gain stops growing, when the listener moves closer to the source.

The SCM from the listener towards an informed source shall be calculated with:

| Clj,b=m ∈ mactivegl,attj,iclosestm,m2*El,srcj,b,m*HRTFvl,rotj,m,b*HRTFvl,rotj,m,bH | (602) |
| --- | --- |

where HRTFvl,rotj,m,b is the HRTF from the listener towards the informed source m.

##### Spatial analysis

Spatial metadata parameters for the source signals shall be extracted from the HOA-to-source SCMs. The intensity vector in a cartesian (x, y, z) order for a single sub-frame is:

| isrcj,k,b,i=Csrcj,b,i,1, 4Csrcj,b,i,1, 2Csrcj,b,i,1, 3(3*Nsf) | (603) |
| --- | --- |

where Nsf is the number of sub-frames in one processing frame. The energy for a single sub-frame shall be obtained with:

| Esrcj,k,b,i=n=14Csrcj,b,i,n,n4*Nsf | (604) |
| --- | --- |

The intensity vector iHOAj,k,b,i and energy EHOAj,k,b,i for the original HOA signal shall be obtained from the spatial analysis block (see 6.6.26.3.3). The average values over sub-frames shall be used, see 6.6.26.3.3.8.

##### Compute residual parameters

The residual spatial parameters shall be obtained from the subtraction between the original HOA parameters and the HOA-to-source model parameters:

| iresj,k,b,i=iHOAj,k,b,i-isrcj,k,b,i | (605) |
| --- | --- |
| Eresj,k,b,i=EHOAj,k,b,i-Esrcj,k,b,i | (606) |

The azimuth direction-of-arrival (DOA) is:

| θresj,k,b,i=atan2(iresj,k,b,i,2, iresj,k,b,i,1) | (607) |
| --- | --- |

and elevation:

| φresj,k,b,i=atan2iresj,k,b,i,3,iresj,k,b,i,12+iresj,k,b,i,22 | (608) |
| --- | --- |

The direct-to-total energy ratio (DTR) is:

| rresj,k,b,i=iresj,k,b,i,12+iresj,k,b,i,22+iresj,k,b,i,32Eresj,k,b,i | (609) |
| --- | --- |

The residual metadata shall be interpolated similarly as explained in 6.6.26.3.4.1, resulting in Eres,intrpj,k,b, θres,intrpj,k,b, φres,intrpj,k,b and rres,intrpj,k,b.

In the exterior rendering, the metadata adjustment process explained in 6.6.26.4.5. is slightly modified. Namely, if the bitstream parameter adaptiveExteriorRenderingProjectionRadius is true, the radius of the exterior projection circle Rproj shall be modified based on the distance to the exterior informed sources. The distance to the closest active exterior informed source shall be used as the projection radius. If there are no active exterior informed sources, the average distance to all the exterior informed sources (Davg,ext) shall be used. If there are no exterior informed sources in the scene or if adaptiveExteriorRenderingProjectionRadius is false, the projection radius set by the bitstream parameter exteriorRenderingProjectionRadius shall be used.

##### Build target SCM

The target SCM shall be built from separate SCMs for the source and residual signals. The two SCMs contain contribution from the informed sources (source SCM) and non-informed sources (residual SCM).  Clj,b shall be used as the source SCM. The residual SCM Cresj,b shall be built from the interpolated residual metadata as:

| Cresj,b=Cres,dirj,b+Cres,diffj,b | (610) |
| --- | --- |
| where the direct signal part shall be calculated as: |  |
| Cres,dirj,b=k=1NsfEres,intrpj,k,b*rres,intrpj,k,b*HRTFθres,intrpj,k,b,φres,intrpj,k,b*HRTFθres,intrpj,k,b,φres,intrpj,k,bH | (611) |

and the diffuse part as shall be calculated as:

| Cres,diffj,b=k=1NsfEres,intrpj,k,b*(1.0-rres,intrpj,k,b)*Cdifb | (612) |
| --- | --- |

where Cdifb is the diffuse field covariance matrix (see 6.6.26.3.5.3). The new target SCM for the current processing frame shall be obtained by combining the source and residual SCMs:

| Cy, newj,b=Clj,b+Cresj,b | (613) |
| --- | --- |

Recursive averaging shall be applied to obtain the target SCM Cyj,b, similarly as in 6.6.26.3.5.3.

#### Rendering for scenes with two HOA sources

##### General

In the case of a scene with only two HOA sources, the rendering shall follow the description in section 6.6.26.4, but with the exceptions described in the following subsections. With such scenes, the listener is never inside the triangulated HOA source area, thus exterior rendering (see 6.6.26.4) is always performed when the listener is inside the HOA group extent.

##### Exterior edges and HOA sources

The exterior edge definitions in Sections 6.6.26.4.1.1 and 6.6.26.4.1.2 shall be replaced with the following.

The exterior edges Exvalid and the original exterior edges Exorig shall be the edges connecting (in both directions) the two HOA sources m1 and m2.

| Exvalid=e12, e21 | (614) |
| --- | --- |
| Exorig=e12, e21 | (615) |

##### Exterior HOA source normals

Calculation of the exterior HOA source normals of Section 6.6.26.4.1.4 shall be replaced with the following.

The exterior HOA source normals for the HOA sources m1 and m2 shall be calculated as follows:

| nHOA,m1=pHOA,1-pHOA,2pHOA,1-pHOA,2 | (616) |
| --- | --- |
| nHOA,m2=-nHOA,m1 | (617) |

##### Calculate the normal for the projected listener position

The listener normal calculation of Section 6.6.26.4.4for the case when the listener is projected to an exterior edge shall be replaced with the following.

The listener normal nL,proj shall be calculated as follows when the listener has been projected on an exterior edge eab:

| nL,proj=wsnHOA,a+wena,b, wab>0.5,wsnHOA,b+wena,b, wab≤0.5 | (618) |
| --- | --- |

where

| ws=wab-0.50.5, wab>0.5 (1-wab)-0.50.5, wab≤0.5 | (619) |
| --- | --- |

and

| we=1-ws | (620) |
| --- | --- |

#### Rendering for scenes with a single HOA source

##### General

In the case of a scene with only a single HOA source, the rendering shall follow the description in section 6.6.26.4, but with the exceptions described in the following subsections.

##### Exterior edge determination

No exterior edges, as defined in Sections 6.6.26.4.1.1 and 6.6.26.4.1.2 shall be calculated as there are no edges in the HOA source space.

##### Exterior HOA source and edge normal calculation

Exterior HOA source and edge normal calculation of Sections 6.6.26.4.1.3 and 6.6.26.4.1.4 shall be skipped.

##### Listener normal calculation

The listener normal calculation of Section 6.6.26.4 shall be replaced with the following. For scenes with a single HOA source, the listener normal shall be a (normalized) vector pointing from the single HOA source position to the listener position:

| nL,proj=pL-pHOA,1pL-pHOA,1 | (621) |
| --- | --- |

### Low-complexity MP-HOA

The low-complexity profile of the MP-HOA renderer stage provides an alternative way to render multiple HOA sources for a 6DoF listening experience using fewer computations than the default profile described in Section 6.6.26. As in other rendering stages, HOA sources comprise multichannel signals and metadata which includes the HOA source position, among other parameters. In practice, these multichannel signals typically originate from a recording of an acoustic scene with one or more microphone setups, wherein each of the one or more microphone setups comprises one or more microphones and the HOA source position corresponds to the center position of the respective microphone setup. A block diagram of this renderer stage is shown in Figure 96. The rendering steps are listed below.

Figure 96— Block diagram of low-complexity MP-HOA renderer stage.

#### Data elements and variables

NVLO(i)	number of virtual loudspeaker objects (VLOs) of HOA source i

Nch(i)	number of input channels of HOA source i

sESD(i, j,n)	equivalent spatial domain (ESD) signal of VLO n of HOA source i for frame j

pi	position of HOA source i, hoaSourcePositionX, hoaSourcePositionY and hoaSourcePositionZ from hoaSources() in payloadScene()

θi,  n	azimuth of VLO n of HOA source i

φi,  n	elevation of VLO n of HOA source i

θnESD[Nch]	azimuth of node n of the ESD representation for Nch channels

φnESD[Nch(i)]	elevation of node n of the ESD representation for Nch channels

ri	VLO radius of HOA source i

diavg	statistical average of the distances between HOA source i and its neighbors

diri	directivity factor of HOA source i

revi	reverberation factor of HOA source i

distanceFactor	bitstream-defined scene-wide scaling factor for the VLO radius

pi,n	position of VLO n of HOA source i

oi	orientation of HOA source i, hoaSourceOrientationYaw, hoaSourceOrientationPitch and hoaSourceOrientationRoll from hoaSources() in payloadScene()

Rsource(i)	3×3 rotation matrix calculated from the HOA source orientation according to Section 6.6.26.3.4.1.

LUT(θ,φ) 	VBAP look-up table

NVBAP	number of VBAP nodes

Nchmax	highest number of channels across all HOA sources in the scene

Hqleft, Hqright	minimum phase spectra of the left and right head-related impulse responses (HRIRs) for the direction corresponding with VBAP node q

fc	cut-off frequency for the frequency-dependent time alignment HRTF preprocessing

ITDq	interaural time difference (ITD) in samples of the HRIRs for the direction corresponding with VBAP node q

ITDmax	maximum ITD in samples across all HRIRs corresponding to VBAP node directions

αω	frequency-dependent weighting applied in the HRTF pre-processing

ks	smoothing factor used in the HRTF pre-processing

wi(j)	interpolation weights of HOA source i for frame j

pL(j)	listener position (x, y, z) for frame j

oL(j)	listener orientation (yaw, pitch, roll) for frame j

pi, nrelj	position of VLO n of HOA source i relative to the listener position for frame j, also represented as [xi, nrelj, yi, nrelj, zi, nrelj]T

RheadT(j)	transpose of a 3x3 rotation matrix for the listener head orientation for frame j, calculated according to Section 6.6.26.3.4.1

θi,nrel(j)	azimuth of VLO n of HOA source i relative to the listener position for frame j

φi,nrel(j)	elevation of VLO n of HOA source i relative to the listener position for frame j

wi, nVBAP(j)	VBAP weights of VLO n of HOA source i for frame j

ei,n,q(j,t)	cross-fading envelope function for VBAP node q and VLO n of HOA source i for frame j

B	block size in samples

sqj	VBAP signal buffer of node q for frame j

soutj	binaural output signal for frame j

hqleft,hqright	left and right head-related impulse responses (HRIRs) for the direction corresponding with VBAP node q

EQ	timbre-correction filter

#### Pre-processing

##### Triangulation of HOA source space

Triangulation of the HOA source space shall be performed as described in Section 6.6.26.3.1.1.

##### Initialization of virtual loudspeaker objects (VLOs)

A set of virtual loudspeaker objects (VLOs), which are abstract sound output objects within a virtual free field, shall be assigned to each HOA source. The virtual free field is defined as a virtual sound field that consists of direct sound without reverberant sound. The number of VLOs (NVLO) in each set shall be equal to the number of channels (Nch) of the Equivalent Spatial Domain (ESD) signal (sESD) of the corresponding HOA source. Therefore, for the ith HOA source:

| NVLO(i)=Nch(i) | (622) |
| --- | --- |

Each set of VLOs shall be arranged in a sphere with center at the corresponding HOA source’s position (pi). The azimuth and elevation of each VLO relative to the sphere center shall be given by the corresponding ESD representation which is already used by the MPEG-H encoder and decoder (see ISO/IEC 23008-3 (MPEG-H Part 3), Tables F.2 to F.5). Therefore, for the nth VLO of the ith HOA source, the azimuth (θi,  n) and elevation (φi,  n) shall be given by:

| θi, n=θnESD[Nch(i)] | (623) |
| --- | --- |
| φi, n=φnESD[Nch(i)] | (624) |

The radius of the sphere (ri) for a given set of VLOs shall be calculated as a statistical average (e.g. median) of the distances between the corresponding HOA source’s position and each of its neighbor HOA sources’ positions (diavg), multiplied by a directivity factor (diri), a reverberation factor (revi), and a bitstream-defined scene-wide scaling factor (distanceFactor).

| ri=diavg∙diri∙revi∙distanceFactor | (625) |
| --- | --- |

The directivity factor is a HOA-source-specific scaling factor which shall be adjusted depending on the directivity order of the corresponding microphone setup, i.e. mapped to the spherical harmonics order of the HOA source. A map between HOA source order and directivity factor is defined in Table 149.

Table 149— Value of directivity factor diri depending on HOA source order

| HOA source order | diri |
| --- | --- |
| 1 | 1.5 |
| 2 | 1.25 |
| 3 or higher | 1.0 |

The reverberation factor is a HOA-source-specific scaling factor which shall be adjusted depending on the reverberation level of the rendered room, which is characterized by the broadband diffuse-to-source energy ratio (DSRi) associated to the acoustic environment containing the HOA source.

If payloadReverb (6.2.8) is available in the bitstream and the flag revPresent is set to “true”, DSRi shall be derived from the frequency-dependent diffuse-to-source energy ratio (revDSR) of the corresponding acoustic environment by calculating a weighted average across frequencies, as follows:

totalWidthERB = 0; for (b = 0; b < numFreqBins; b++) { if (b == 0) { widthERB[b] = Hz2ERBRateScale(fcBs[b+1]) – Hz2ERBRateScale(fcBs[b]); } else if (b == numFreqBins – 1) { widthERB[b] = Hz2ERBRateScale(fcBs[b]) – Hz2ERBRateScale(fcBs[b-1]); } else { widthERB[b] = 0.5 * (Hz2ERBRateScale(fcBs[b+1]) - Hz2ERBRateScale(fcBs[b-1])); } totalWidthERB += widthERB[b]; } DSRi = 0; for (b = 0; b < numFreqBins; b++) { DSRi += revDSR[b] * widthERB[b] / totalWidthERB; }

where source frequency grid fcBs is the vector (fgdCenterFreq[g]) of size numFreqBins = fgdNrBands[g] containing the frequencies of the grid associated with the DSR values from the bitstream. Then, revi shall be calculated as:

| revi=DSRiDSRavg | (626) |
| --- | --- |

where DSRavg is the average DSRi across all HOA sources.

Function Hz2ERBRateScale(f)shall be:

erb = Hz2ERBRateScale(f) {

erb = 21.4 * log10(1 + 0.00437*f)

}

If payloadReverb is not available in the bitstream or the flag revPresent is set to “false”, revi shall be set to 1 for all HOA sources, which corresponds to an acoustic environment where the critical distance is constant across space.

The absolute position of each VLO (pi,n) shall be then calculated by rotating its relative coordinates by the HOA source orientation (oi) and adding them to the HOA source position, as follows:

| pi,n=Rsource(i)-sinθi, ncosφi, nsinφi, n-cosθi, ncosφi, nri+pi | (627) |
| --- | --- |

where Rsource(i) is a 3×3 rotation matrix calculated from the HOA source orientation according to Section 6.6.26.3.4.1.

##### Vector-based amplitude panning initialization

A vector-based amplitude panning (VBAP) weight lookup table (LUT(θ,φ)) shall be then constructed, containing VBAP weights for each direction within a spherical grid with 1-degree resolution in azimuth and elevation. A set of VBAP nodes shall be then defined, working as an additional layer of virtual loudspeakers that enables efficient binaural rendering. The number of VBAP nodes (NVBAP), defined as the number of virtual loudspeakers in said layer, shall depend on the highest number of channels the HOA sources (Nchmax) and the bitstream-defined panningGridOrder parameter, as follows:

| NVBAP=minNchmax,panningGridOrder+12 | (628) |
| --- | --- |

The spatial distribution of VBAP nodes shall follow the ESD representation specification for the corresponding number of nodes, as used by the MPEG-H encoder and decoder (see ISO/IEC 23008-3 (MPEG-H Part 3), Tables F.2 to F.5). The calculation of the VBAP weights to be stored in the table shall be done in the same way as described in Section 6.6.25, except that an exponent of 1 shall be applied at the energy distribution step.

##### Calculation of HRTFs from HRIRs

The calculation of HRTFs from HRIRs shall be performed as described in Section 6.6.26.3.1.2, with the grid of HRIR directions being defined by the VBAP nodes’ directions. In order to reduce timbral coloration of the binaural rendering, the HRIRs shall be pre-processed via frequency-dependent time-alignment, as follows. First, the interaural time differences for each qth direction (ITDq, where q is the VBAP node index, ranging from 1 to the number of VBAP nodes, NVBAP) shall be estimated by applying a second-order low-pass IIR filter (frequency cut-off: 750 Hz, Q-factor: 0.7071) to the corresponding left and right HRIRs (hqleft, hqright), then calculating the cross-correlation between the two, and finding the sample where the resulting value is maximum. Next, the minimum-phase spectra of the HRIRs (Hqleft, Hqright) shall be obtained, following Equations (365) and (366) from Section 6.6.20.2. Finally, the ITDs shall be re-inserted on a frequency-dependent basis, ensuring that they are preserved for frequencies below a given cut-off (fc=2kHz is recommended) and removed above it. This shall be done by applying a phase-shift in the frequency domain, as described in the following Equations (note that the “left” and “right” superscripts have been omitted for brevity):

| hqt=IDFT( Hqω∙e-iω(0.5ITDmax+α(ω)ITDq)) | (629) |
| --- | --- |
| αω=minmax0.5+ks∙logω2πfclog2,0,1 | (630) |

where IDFT is the inverse discrete Fourier transform, ω is the angular frequency, ITDmax is the maximum ITD value in samples across the whole grid of directions, α is a frequency-dependent weight in the range [0, 1] that controls the amount of phase shift, and ks is a scalar smoothing factor (e.g., ks=1 applies a 1-octave smoothing).

#### Stage update

The active triangle shall be determined and interpolation weights shall be calculated for the current frame (j) for each HOA source (wi(j)) using barycentric coordinates, as described in Section 6.6.26.3.2.1 and Section 6.6.26.3.2.2.

Then, each VLO shall be assigned a weight based on its corresponding HOA source’s interpolation weight, as follows:

| wi, nj=wij0.5 | (631) |
| --- | --- |

Then, the position of each VLO relative to the current listener position (pL(j)) and current listener orientation (oL(j)) shall be calculated as follows:

| pi, nrelj=[xi, nrelj, yi, nrelj, zi, nrelj]T=RheadT(j)(pi,n-pL(j)) | (632) |
| --- | --- |

where RheadT(j) is the transpose of a 3x3 rotation matrix for the listener head orientation for frame j, calculated according to Section 6.6.26.3.4.1.

Then, the relative azimuth and elevation of each VLO shall be calculated from the cartesian coordinates:

| θi,nrel(j)=atan2(-xi, nrelj, - zi, nrelj) | (633) |
| --- | --- |
| φi,nrel(j)=asin( yi, nrelj/pi, nrelj2) | (634) |

Finally, the VBAP weights for each VLO (wi, nVBAP(j)) shall be obtained from the VBAP weight look-up table, based on its relative azimuth and elevation, and they shall be multiplied by the corresponding VLO weights wi, nj.

| wi, nVBAP(j)=LUT(θi,nrelj,φi,nrel(j))∙wi, nj | (635) |
| --- | --- |

#### Stage processing

For an efficient rendering, the input stream of ESD signals shall be encoded into a reduced set of VBAP signals based on the listener position and the VLO positions.

Thus, each VLO shall be assigned an ESD signal (sESD(i, j,n)). This signal shall be mapped to the corresponding VBAP nodes (up to three), as follows.

To ensure a smooth transition between frames, an envelope function shall be calculated for each VLO, based on its VBAP weights for the current and previous frames. Therefore, for the qth VBAP node (where q is the VBAP node index, ranging from 1 to the number of VBAP nodes, NVBAP), the envelope function (ei,n,q(j,t)) shall be calculated as:

| ei,n,qj,t=wi, n,qVBAPj-1+twi, n,qVBAPj-wi, n,qVBAPj-1B, t∈[0, B-1] | (636) |
| --- | --- |

where B is the block size.

Then, for each VBAP node, a VBAP signal buffer shall be constructed by adding the corresponding weighted ESD signals as:

| sqj=i, nei,n,qj,t∘sESD(i, j,q) | (637) |
| --- | --- |

where ∘ indicates element-wise multiplication.

Each of these VBAP signal buffers shall be filtered with the corresponding pair of HRIR filters and the result shall be added to the output binaural signal (soutj):

| soutj=qsqj*hqleft,qsqj*hqright | (638) |
| --- | --- |

Finally, equalization may be applied to both channels of the output binaural signal by means of an IIR or FIR filter:

| soutj=soutj*EQ | (639) |
| --- | --- |

The output binaural signal shall be assigned to a Downmix RI, which is the output of the rendering stage.

#### Exterior rendering

When the listener is outside the region enclosed by triangle defined by a set of HOA sources, the rendering shall follow the description in Section 6.6.27.3 and Section 6.6.27.4 with the exception that the interpolation weights are calculated according to section 6.6.26.4.

#### Rendering of informed sources

Informed sources are not supported by the low-complexity MP-HOA rendering profile.

#### Rendering for scenes with two HOA sources

In the case of a scene with only two HOA sources, the rendering shall follow the description in Section 6.6.27.5 with the exceptions described in Section 6.6.26.6.

## Spatializer

### Binaural spatializer

#### General

The direction-dependent characteristics of sound reaching the ears of a listener can be emulated for point sources using head-related impulse response (HRIR) filters. The Binaural Spatializer creates a binaural downmix of all remaining spatialized RIs (all remaining primary and secondary RIs of type Primary, Reflection, Diffraction, Representative, Nondiegetic,ListenerVoice). For this, the single-channel signal associated with each of these RIs is convolved with two-channel HRIRs according to the listener-relative location of the RI and the resulting binaural stereo signals are summed to form the final auralized output signal.

When the value of “PanningHRIRConvolution” of Renderer DSP configuration is 1, HRIR convolution by panning is activated. For this, the single-channel signal associated with each of these RIs is first distributed for panning to pre-defined representative positions as virtual loudspeakers based on the directions from the listener to the RIs and the directions from the listener to the representative positions. To perform the panning, appropriate time shifts and gains for each of the left and right ears are applied to the RIs, where the shift and gain parameters are pre-computed and stored in a separate table during the initialization of the Binaural Spatializer. The table is called “SG table”. Panned signals are then summed at the representative positions and convoluted with HRIRs according to the listener-relative locations of the representative positions. The resulting binaural signals are then summed to form the final auralized output signals. The concept of the panning operation is shown in Figure 97, which is an example case where the signals of audio objects (RIs) located between two representative positions are distributed to each of the two representative positions.

Figure 97— Concept of HRIR convolution by panning

The spatialization can be bypassed using the RI type Downmix or Airflow. RIs of these type shall have an associated two-channel signal, which is directly added to the final binaural output signal. Preceding stages that create Downmix RIs (e.g. the Homogeneous Extent or HOA stages) shall provide corresponding signals that are auralized and/or can directly be played to the listener via headphones.

#### Data elements and variables

#### Stage description

##### HRIR interface

External HRIRs are provided in the SOFA format [13].  In addition, the Binaural Spatializer expects the SOFA file to follow these conventions:

- The SOFA file shall be saved according to the SimpleFreeFieldHRIR convention version 1.0.

- The sampling rate of the filters in the SOFA file shall match the sampling rate of the current Renderer DSP configuration.

- The SOFA should contain a sufficient resolution of measurement locations (min. 2 degrees in the azimuth) to support perceptually smooth spatial rendering.

- The SOFA file may contain HRIRs measured at different distances to the listener.

- The following metadata in the SOFA file is ignored and the default value is assumed: ListenerUp, ListenerView, EmitterPosition, ReceiverPosition, ListenerPosition, SourceUp, SourceView, Data.Delay

- The HRIR filters in the SOFA file are used as-is. Preprocessing like normalization, diffuse-field or headphone equalization, etc. should be done outside of the Renderer. Furthermore, the supplier of HRIR filters should avoid excessive group delay in the filters, as this adds to the overall DSP processing delay.

The available measurement locations in the SOFA file are stored in a 3-dimensional k-d tree data structure, Bentley [14] using listener-relative Cartesian coordinates. This way, a look-up of the nearest available HRIR for a given listener-relative location can be performed efficiently and independent of the measurement distances provided in the SOFA file.

The length of the HRIR filter may be windowed to a length of 128 samples for binauralizing secondary RIs to reduce computational complexity.

##### Initialization of panning parameters for HRIR convolution by panning

###### General

In the initializeSpatializer(), initialization of HRIR convolution by panning is conducted, where panning parameters are computed and stored in a SG table. Creation method of a SG table is shown in the chapters  6.7.1.3.2.2- 6.7.1.3.2.5

###### Selection of representative HRIRs

Representative HRIRs corresponding to the virtual loudspeakers mentioned in 6.7.1.1 are selected from the provided HRIR data set. Suppose that azimuth angles of HRIRs are measured counterclockwise on condition that the HRIR right in front of a listener is 0 degree.

The selection process is as shown below:

First, choose six HRIRs from the provided HRIR dataset with azimuth angles closest to k * 60 + 30 degrees (k = 0, 1, 2, 3, 4, 5) on the horizontal plane. If there are no HRIRs on the horizontal plane, opt for HRIRs with elevation angles close to 0 degree. If multiple candidates appear for a single k value, the one having smaller azimuth angle is prioritized within the range of 0 to 180 degrees, and the one having larger azimuth angle is prioritized within the range of 180 to 360 degrees.  Next, choose an HRIR with an elevation angle closest to 90 degrees within the range of 0 to 90 degrees on the median plane. If there are no HRIRs on the median plane, choose one with an azimuth angle close to 0 degree. This is regarded as a zenith.  Finally choose an HRIR with an elevation angle closest to -90 degrees within the range of 0 to -90 degrees on the median plane. If there are no HRIRs on the median plane, choose one with an azimuth angle close to 0 degree. This is regarded as a nadir.

In this way, 6 representative HRIRs from horizontal plane, and zenith and nadir HRIRs are picked, resulting in total of 8 representative HRIRs.

###### Low Pass Filtering of HRIRs

Before computing the shift values and gain values to be stored in a SG table for panning audio signals to representative directions, all the HRIR data in the provided HRIR data sets are low pass filtered by an FIR filter with the coefficients given in Table 150.  Frequency response of the magnitude of the FIR filter is shown in Figure 98.

Table 150— Coefficients of Low Pass Filter

| LPFCoeff[0] | 0.0315095432760708 |
| --- | --- |
| LPFCoeff[1] | -0.0472082307959231 |
| LPFCoeff[2] | -0.0504311478995756 |
| LPFCoeff[3] | 0.00529714829513192 |
| LPFCoeff[4] | 0.130896073599700 |
| LPFCoeff[5] | 0.266767170948375 |
| LPFCoeff[6] | 0.325763238942777 |
| LPFCoeff[7] | 0.266767170948375 |
| LPFCoeff[8] | 0.130896073599700 |
| LPFCoeff[9] | 0.00529714829513192 |
| LPFCoeff[10] | -0.0504311478995756 |
| LPFCoeff[11] | -0.0472082307959231 |
| LPFCoeff[12] | 0.0315095432760708 |

Figure 98— Frequency response of the Low Pass Filter

In the following description, a time domain waveform of low pass filtered version of an HRIR is defined to be a vector, whose components are sample values of the waveform with the sampling rate of 48kHz. Notations below are used.

xr1  : a vector of an HRIR of the first representative position

xr2  : a vector of an HRIR of the second representative position

xr3  : a vector of an HRIR of the third representative position

x   : a vector of an HRIR from the provided HRIR dataset closest to the original audio object.

Here xr1 , xr2 , and xr3 are selected from the 8 representative HRIRs by the steps below:

Select two HRIRs on the horizontal plane having the most and the 2nd most closest azimuth angle to x . Then select the zenith or the nadir HRIR depending on the elevation of x .  If the elevation angle of x is positive or 0, the zenith is chosen, and if the elevation angle of x is negative, the nadir is chosen.

The approximation of  x using xr1 , xr2 , and xr3 is done by the two steps shown in 6.7.1.3.2.4 and  6.7.1.3.2.5.  Note that the time shift values and gain values to approximate x using xr1 , xr2 , and xr3 are used to distribute a signal of the original audio object to the representative positions to conduct the panning.

###### Computation of time shift parameters

Make time shifts to xr1 , xr2 , and xr3 to maximize the cross correlations between xr1 and x,  xr2 and x, xr3 and x, respectively. The resultant vectors are referred to as x1 ,  x2 , and x3.  The time shift values corresponding to  xr1 , xr2 , and xr3  for each of all the provided HRIR data set covering the entire sphere are computed and stored in the SG table. The values are used as “appropriate shift values” explained in  6.7.1.1.

###### Computation of gain parameters

Define a vector xs as,

xs= Ax1+Bx2+Cx3

which approximates the target vector x , where A, B and C are scalar weight values. The magnitude of the error vector e = x- xs is minimized by solving,

∂e2∂A=0       ∂e2∂B=0       ∂e2∂C=0.

The result is,

ABC= x12x1∙x2x1∙x3x1∙x2x22x2∙x3x1∙x3x2∙x3x32-1x1∙xx2∙xx3∙x ,

where x1∙x2 denotes the inner product of x1 and x2 , and X-1 denotes the inverse matrix of matrix X.   The values A, B and C  are computed for each of all the provided HRIR data set covering the entire sphere, and they are stored in the SG table to be used as “appropriate gain values” explained in 6.7.1.1.

##### HRIR convolution

###### General

The configuration parameter “PanningHRIRconvolution” is first checked by the renderer.  When the value of “PanningHRIRconvolution” is 0, regular HRIR convolution explained in chapter 6.7.1.3.3.2 is conducted.  When the value of “PanningHRIRconvolution” is 1, HRIR convolution by panning explained in chapter 6.7.1.3.3.3 is conducted.

###### Regular HRIR convolution

In the processBlock() routine of the Binaural Spatializer, the signals associated with all remaining and active RIs are filtered with the respective HRIRs using fast stereo convolution (A.13). The binauralized stereo output of each filter is summed in a stereo output buffer. The signals associated with Downmix RIs are summed to the same buffer without further processing.

If the listener-relative location of an auralized RI changes (e.g. item.changed & ItemProperty::Location == true, cf. (6.4.2.4) in an update()call, the correct HRIR is selected using a nearest-neighbour search as outlined in clause 6.7.1.3.1.  For this, the RI location in global coordinates is transformed to listener-relative coordinates. If the nearest HRIR changed from the previous update() call, the new filter is prepared and partitioned in update() according to (A.13) and the filter exchange is performed in the next processBlock() call.

###### HRIR convolution by panning

When the HRIR convolution by panning is activated, according to the relative directions of RIs from a listener, RIs are time shifted and gain adjusted, and summed at pre-defined representative directions. The values of time shift and gain adjustment are taken from the SG table created in section 6.7.1.3.2. Summed results for each of the representative directions are then convoluted with HRIRs of those representative directions respectively. The process of HRIR convolution by panning is shown below.

In the update() of the binaural spatializer process, the item type of each RI is checked.  RIs with item types of Primary, Reflection, Diffraction, Representative, and Nondiegetic are subject to HRIR convolution by panning.  Memory areas for the arrays to store the panning parameters（time shift and gain values）and other data for panning processing for the RIs are allocated.  When a listener-relative location of a RI is different from that of the previous update() step, and the selected SOFA index nearest to the location is different from that of the previous update() step, panning parameters for each of the RIs to be used at corresponding representative positions are set according to the locations of the RIs.  A “change flag” is then set to show that panning parameters are changed.  The flowchart of the update() is shown in Figure 99.

In the processBlock() of the Binaural Spatializer process, time shifts and gain multiplications are applied to the source signals to make distributed signals at the representative positions.  These distributed signals are added up at each of the representative positions. When panning parameters of a RI in a current frame are not identical to those of a previous frame, the “change flag” of the RI was set in update().  If a “change flag” is set, time domain cross fading is conducted.  If panning parameters of a RI are identical to those of the previous frame, i.e., if the “change flag” is not set, cross fading is not conducted with regard to such a RI.  A reset of the “change flag” is done every frame. Copying signal data of RIs of the previous frame to a delay buffer is done every frame. In this way, signals at representative positions are created and convolved with the HRIRs of the representative positions. The flowchart of the processBlock() is shown in Figure 100.

Figure 99— Flowchart of update() for HRIR convolution by panning

Figure 100 — Flowchart of processBlock() for the HRIR convolution by panning

##### Dispersion filter generation and processing

Acoustic reflections and sound propagation in the air do not behave fully linear. By applying a purposefully designed filter, the effect of acoustic dispersion can efficiently improve the perception of early reflection simulations and enhance plausibility and realism with very moderate cost in computational complexity. The signal flow diagram is seen in Figure 101.

Figure 101 — Dispersion signal filter flow after the binauralized output of early reflections (ER) and diffracted sound (DF); together with direct sound (DS) and late reverberation (LR) to complete the headphone output signal

The dispersion filter is generated from two vectors of 1440 samples of partially decorrelated white noise T1 and T2, (see A.9 Dispersion filter coefficient template), which corresponds to 30 ms at 48 kHz sampling rate. In the function configureDispersionFilter()the following parameters can be set:

enableDispersionFilter 	Boolean flag that allows to enable or disables (pass-thru) the dispersion filter processing

enableDispersionFilterER 	Boolean flag that enables or disables (pass-thru) the dispersion filter processing for early reflections sounds

enableDispersionFilterDiffr 	Boolean flag that enables or disables (pass-thru) the dispersion filter processing for diffracted sounds

dispersionFilterLengthMSec	Parameter to configure the duration of the dispersion filter in msec. The maximum value is 30.

dispersionFilterGaindB 	Parameter to configure the dispersion filter gain in dB

enableSpatialDispersion	Parameter to enable spatial dispersion

When there is no payloadDispersion() present in the bitstream, the following default values apply:

enableDispersionFilter = true

enableDispersionFilterEnableER = true

enableDispersionFilterDiffr = false

dispersionFilterLengthMsec = 15

dispersionFilterGaindB = 0

enableSpatialDispersion = true

Based on the configuration parameter, the Dispersion Filter is generated as follows:

- Depending on the state of the enableSpatialDispersion parameter, taking the first filterlength Samples from T1 and T2 with

filterlength = int(dispersionFilterLengthMsec * fs / 1000.0);

for (int i = 0; i < filterlength; i++) {

S_1(i) = T_1(i);

S_2(i) = T_1(i);

if (enableSpatialDispersion)

S_2(i) = T_2(i);

}

- Individual L2 normalization of S1 and S2:

w_1 = 0;

w_2 = 0;

for (int i = 0; i < filterlength; i++) {

w_1 += S_1(i)*S_1(i);

w_2 += S_2(i)*S_2(i);

}

w_1 = 1 / sqrt(w_1);

w_2 = 1 / sqrt(w_2);

- Temporal weighting of S1 and S2

for (int i = 0; i < filterlength; i++) {

g = 1.0 - sqrt(i/filterlength);

S_1(i) *= w_1 * g;

S_2(i) *= w_2 * g;

}

- Final gain adjustment

w_1 = 0.0;

w_2 = 0.0;

for (int i = 0; i < filterlength; i++) {

w_1 += S_1(i)*S_1(i);

w_2 += S_2(i)*S_2(i);

}

w = pow(10.0,dispersionFilterGainDb*0.05) / sqrt(max(w_1, w_2));

for (int i = 0; i < filterlength; i++) {

S_1(i) *= w;

S_2(i) *= w;

}

As shown in Figure 101, the final FIR filters S1 and S2 are applied to the left and right channel, respectively.

##### Headphones equalization

###### General

This binaural spatializer supports personalized headphones frequency response equalization to account for differences between the local listener's headphones and a target, which can be the content creator headphones, when provided. It compensates the differences between the scene author’s intent and the listener’s reproduction equipment.

Headphones equalization is carried out within the Binaural spatializer. Listener headphones data is provided through the local configuration interface as specified in section 6.4.2.6. It shall be provided as either frequency-dependent gains, as a cascade of second order (biquad) parametric filters, or as a frequency dependent preset. If a preset is specified, the corresponding frequency-dependent gain data is given in A.17.

A default target spectrum is defined, which can be overwritten with the content creator’s headphones data via the bitstream in the configuration payload, given in Table 5, or via the local configuration interface.

###### Initialization

At initialization, if listener specific equalization data is provided a FIR filter shall be constructed to equalize the listener headphones frequency response to that of the content creator. During playback, the binauralized output signals are filtered using this constructed filter. To minimize any processing delays the FIR filter shall be designed as a minimum-phase filter. The filter shall be designed using the frequency sampling design method.

###### Conversion of parametric filters

If the listener headphone equalization data or the target equalization data is specified using a cascade of parametric filters they shall first be converted to a vector of frequency-specific gains, prior to construction of the filters. This shall be done by first calculating the coefficients of the transform function given in (640)

| Hz= b0+ b1z-1+ b2z-21+ a1z-1+ a2z-2 | (640) |
| --- | --- |

The following pseudo code describes how the coefficients shall be calculated.

void calcBiquadCoeffs(string filterType, double fc, double Q, double peakGain, double sampleRate)

{

Fc_norm = fc / sampleRate;

K = tan(pi * Fc);

if (filterType == “peak”) {

if (peakGain >= 0) {

norm = 1 / (1 + 1 / Q * K + K * K);

b0 = (1 + V / Q * K + K * K) * norm;

b1 = 2 * (K * K - 1) * norm;

b2 = (1 - V / Q * K + K * K) * norm;

a1 = a1;

a2 = (1 - 1 / Q * K + K * K) * norm;

} else {

norm = 1 / (1 + V / Q * K + K * K);

b0 = (1 + 1 / Q * K + K * K) * norm;

b1 = 2 * (K * K - 1) * norm;

b2 = (1 - 1 / Q * K + K * K) * norm;

a1 = a1;

a2 = (1 - V / Q * K + K * K) * norm;

}

} else if (filterType == “lowshelf”) {

if (peakGain >= 0) {

norm = 1 / (1 + sqrt(2) * K + K * K);

b0 = (1 + sqrt(2 * V) * K + V * K * K) * norm;

b1 = 2 * (V * K * K - 1) * norm;

b2 = (1 - sqrt(2 * V) * K + V * K * K) * norm;

a1 = 2 * (K * K - 1) * norm;

a2 = (1 - sqrt(2) * K + K * K) * norm;

} else {

norm = 1 / (1 + sqrt(2 * V) * K + V * K * K);

b0 = (1 + sqrt(2) * K + K * K) * norm;

b1 = 2 * (K * K - 1) * norm;

b2 = (1 - sqrt(2) * K + K * K) * norm;

a1 = 2 * (V * K * K - 1) * norm;

a2 = (1 - sqrt(2 * V) * K + V * K * K) * norm;

}

} else if (filterType = “highshelf”) {

if (peakGain >= 0) {

norm = 1 / (1 + sqrt(2) * K + K * K);

b0 = (V + sqrt(2 * V) * K + K * K) * norm;

b1 = 2 * (K * K - V) * norm;

b2 = (V - sqrt(2 * V) * K + K * K) * norm;

a1 = 2 * (K * K - 1) * norm;

a2 = (1 - sqrt(2) * K + K * K) * norm;

} else {

norm = 1 / (V + sqrt(2 * V) * K + K * K);

b0 = (1 + sqrt(2) * K + K * K) * norm;

b1 = 2 * (K * K - 1) * norm;

b2 = (1 - sqrt(2) * K + K * K) * norm;

a1 = 2 * (K * K - V) * norm;

a2 = (V - sqrt(2 * V) * K + K * K) * norm;

}

}

}

Subsequently, the magnitude of the filter for a specific frequency shall be calculated from the filter coefficients using the following pseudocode:

filterMagnitude(double f, double b0, double b1, double b2, double a1, double a2) {

double w = 2.0 * pi * f;

double numerator = b0 * b0 + b1 * b1 + b2 * b2 + 2.0 * (b0 * b1 + b1 * b2) *

cos(w) + 2.0 * b0 * b2 * cos(2.0 * w);

double denominator = 1.0 + a1 * a1 + a2 * a2 + 2.0 * (a1 + a1 * a2) * cos(w) +

2.0 * a2 * cos(2.0 * w);

double magnitude = sqrt(numerator / denominator);

}

The gains for the subsequent FIR filter design shall be determined according to the following pseudo code:

for (n = 0; n < Ntaps; n++) {

gains[n] = filterMagnitude(f[n], b0, b1, b2, a1, a2);

}

where b0, b1, b2, a1, a2 shall be as calculated in calcBiquadCoeffs().

###### Design of an EQ filter given a target response

The filter shall be specified by the length of the filter as a power of two (Ntaps = 2n with n an integer number), the sample rate and the target frequency response given by a frequency vector and a gain vector that shall have equal lengths. Using the frequency sampling method, first a linear phase FIR filter shall be designed, which shall then be converted to a minimum phase filter using the Hilbert transform. The frequencies can be chosen freely, but the values must be increasing.

To design the linear phase filter, the target curve gains shall first be linearly interpolated to a frequency axis having a length Nf  equal to the first power of 2 greater than the specified filter length. Then for each interpolated frequency the linear phase shall be determined, and the inverse FFT shall be taken to obtain the impulse response. Finally, a Hamming window is applied to prevent aliasing.

To obtain the minimum-phase filter, the impulse response shall first be zero-padded to at least twice the length of the filter, and the FFT taken of it. Then the Hilbert transform is taken of the natural log of the absolute spectrum. The result is transformed back to the time domain.

The following pseudo method describes how the filter shall be designed from the interpolated frequency and gain vectors of the user and content creator:

/*! Designs a minimum phase filter given a frequency vector and a gain using the frequency sampling method.

frequency vector f runs from [DC ... nyquist] in (Hz)

gain vector contains gains per frequency  (dimensionless)

returns b, vector of length Ntaps with the filtercoefficients.

*/

Buffer& b FIR2MinphaseFilter(Buffer& userFrequencies, Buffer& userGains, Buffer& creatorFrequencies, Buffer& creatorGains)

{

// Linearly interpolate the desired responses on a onesided spectrum

// with a uniform mesh wn of length Ntaps

piecelin(userFrequencies, userGains, wn, userk);

piecelin(creatorFrequencies, creatorGains, wn, creatork);

userk *= (1.0 / (mean(userk) + epsilon);

creatork *= (1.0 / (mean(creatork) + epsilon);

// get transfer function and limit dynamic range between -20 and +20 dB

for (int n = 0; n < Nf; n++) {

k[n]=creatork[n]/userk[n];

k[n]=max(k[n], 0.1);

k[n]=min(k[n], 10.0);

}

// Adjust the phases of the coefficients so that the first `Ntaps` of the

// inverse FFT are the desired filter coefficients.

for (int_t n = 0; n < Nf; n++) {

H[n] = k[n] * exp(wn[n] * complex(0.0f, -((Ntaps - 1) * pi)) / fs);

}

// inverse fft of onesided spectrum

h = real(ifft(H));

// crop and apply window on impulse response

for (int n = 0; n < Ntaps; n++) {

b[n] = h[n] * (0.54 - 0.46 * cos(2 * pi * n / Ntaps));

}

// get minimumphase of impulse response

b = minimumPhase(b);

}

where mean() returns the arithmetic mean of the input buffer.

The following pseudo code describes how the piecewise linear interpolation shall be implemented:

Buffer& v = piecelin(Buffer& x, Buffer& y, Buffer& u)

// elementwise piecewise linear interpolation

// evaluates values in y at points x at query points u

int K = length(y) - 1;

int N = length(u);

// get derivative dy/dx

for (int k = 0; k < K; k++) {

dydx[k]=(y[k + 1] - y[k]) / (x[k + 1] - x[k]);

}

// get interpolated values

int k = 0;

for (int n = 0; n < N; n++) {

// get subinterval index

while ((k < K) && (u[n] >= x[k + 1])) {

k++;

}

// linear interpolation

v[n] = y[k] + (u[n] – x[k]) * dydx[k];

}

}

The following pseudo method describes how a minimum phase filter shall be calculated from the linear phase filter of length Ntaps:

Buffer& r = minimumPhase(Buffer& b)

{

// zeropad by setting vector b(Ntaps .. 2*Ntaps) to zero

for (int n = Ntaps; n < 2 * Ntaps; n++) {

b[n] = 0;

}

// go to frequencydomain

Hin = fft(b);

// ln amplitude (eps= small number preventing division by zero)

for (int n = 0; n < 2 * Ntaps; n++) {

Hin[n] = ln(abs(Hin[n]) + eps);

}

// hilbert

Hout = hilbert(Hin);

// exp(conj())

for (int n = 0; n < 2 * Ntaps; n++) {

Hout[n] = exp(conj(Hout[n]));

}

// go back to timedomain

out = real(ifft(Hout));

// remove zeropadding

for (int n = 0; n < Ntaps; n++) {

r[n] = out[n];

}

}

where conj(c) returns the complex conjugate of c and exp(c)= ec, and ln is the natural logarithm. The Hilbert transform shall be calculated according to the following pseudo code:

Buffer& out = hilbert(Buffer& b)

{

// go to frequencydomain

H = fft(b);

Nyquist = Ntaps/2

for (int n = 1; n < Nyquist; n++) {

H[n] = H * 2.0;

}

for (int n = Nyquist + 1; n < Ntaps; n++) {

H[n] = 0.0;

}

// go back to timedomain

out = real(ifft(H));

}

###### Audio Processing

For applying the filters the uniformly partitioned overlap-save convolution method as specified in section A.13.1  shall be used. The filters shall be applied as the final step in the binauralized rendering, directly prior to output.

##### Spectral Compensation for Accessibility

###### General

Hearing impairments are usually governed by frequency-dependent spectral hearing loss. To mitigate for that during rendering, it is possible to configure an equalizer for each ear individually.

###### Initialization

At initialization, an individual FIR filter shall be constructed for each ear to mitigate the spectral hearing loss of a listener. The frequency dependent compensation weights are configured via the AccessibilitEqLeft and AccessibilityEqRight configuration parameter as listed in Table 141.

During playback, the binauralized output signals are filtered using this constructed filter. To minimize any processing delays the FIR filter shall be designed as a minimum-phase filter. If both headphone compensation as well as spectral compensation for accessibility is applied, the filter design shall first combine both filter curves and then design a joint minimum-phase filter. If the resulting filters of the left and right ear are not phase-aligned (due to different spectral weights), the phase of the two filters shall be phase-corrected as defined in clause 0.

###### Design of the spectral compensation FIR filter

First, the linear-phase spectral compensation filter curves for both ears shall be designed from the parameters AccessibilityEqLeft/Right (see Table 141) which provide either parametric filters (which are parsed as a described in clause 6.7.1.3.5.3) or directly via the filtergain definition, resulting in the parameters accessEqGains and  accessEqFrequencies that describe the linear gain and frequency values. The filter design combines the headphone equalization and spectral compensation using the following extended version of FIR2MinphaseFilter as introduced in clause 6.7.1.3.5.4:

/*! Designs a minimum phase filter given a frequency vector and a gain using the frequency sampling method.

frequency vector f runs from [DC ... nyquist] in (Hz)

gain vector contains linear gains per frequency (dimensionless)

returns b, vector of length Ntaps with the filtercoefficients.

*/

Buffer& b FIR2MinphaseFilter(Buffer& userFrequencies, Buffer& userGains, Buffer& creatorFrequencies, Buffer& creatorGains, Buffer& accessEqFrequencies, Buffer& accessEqGains)

{

// Linearly interpolate the desired responses on a onesided spectrum

// with a uniform mesh wn of length Ntaps

piecelin(userFrequencies, userGains, wn, userk);

piecelin(creatorFrequencies, creatorGains, wn, creatork);

piecelin(accessEqFrequencies, accessEqGains, wn, accessEqk);

userk *= (1.0 / (mean(userk) + epsilon);

creatork *= (1.0 / (mean(creatork) + epsilon);

// get transfer function and limit dynamic range between -20 and +20 dB

for (int n = 0; n < Nf; n++) {

k[n] = accessEqk[n] * creatork[n] / userk[n];

k[n] = max(k[n], 0.1);

k[n] = min(k[n], 10.0);

}

// Adjust the phases of the coefficients so that the first `Ntaps` of the

// inverse FFT are the desired filter coefficients.

for (int_t n = 0; n < Nf; n++) {

H[n] = k[n] * exp(wn[n] * complex(0.0f, -((Ntaps - 1) * pi)) / fs);

}

// inverse fft of onesided spectrum

h = real(ifft(H));

// crop and apply window on impulse response

for (int n = 0; n < Ntaps; n++) {

b[n] = h[n] * (0.54-0.46*cos(2*pi*n/Ntaps));

}

// get minimumphase of impulse response

b = minimumPhase(b);

}

where mean() returns the arithmetic mean of the input buffer.

The following rules shall apply:

- When no parameter AccessibilityEqLeft (see Table 141) is provided, no spectral compensation filtering shall be applied.

- When only AccessibilityEqLeft (the filter for the left ear, see Table 141) is provided but not AccessibilityEqRight (filter for the right ear), the right ear filter shall be designed with the definition of the left ear filter AccessibilityEqLeft.

- When both parameters AccessibilityEqLeft and AccessibilityEqRight are provided, the spectral compensation filter for the left ear shall be designed based on AccessibilityEqLeft and the spectral compensation filter for the right ear shall be designed based on AccessibilityEqRight.

- When no creator headphone EQ target response is provided, the EQ target response shall be set to 1.0 for all frequency bins.

When no user headphone response data is provided, the user EQ response shall be set to 1.0 for all frequency bins.

Cross-filter Phase Alignment

To reduce phase differences between the left ear and the right ear filters, which may lead to binaural localization artifacts, the phase differences between these filters removed as described in the following pseudo code:

- Zero-pad the left and right ear FIR filter to at least twice the length of the filter (Ntaps), and apply FFT to obtain H_left and H_right, with Nds being the length of the vector H_left and H_right.

- Compute phase of the cross-power spectrum and align phase of H_left and H_right with i being the imaginary number:

for (n=1; n<Nds; n++) {

phi = angle(H_left[n] * conj(H_right[n]));

if (phi > 0)

H_left[n] *= exp(-phi*i);

else

H_right[n] *= exp(phi*i);

}

- IFFT of H_left and H_right to obtain FIRs h_left and h_right

- Check for possible wrap-around samples in the time domain and add time shift if needed:

kl = 1;

t = max(abs(h_left)) * 0.0001;

while (abs(h_left[Nds-kl]) > t)

kl++;

kr = 1;

t = max(abs(h_right)) * 0.0001;

while (abs(h_right[Nds-kr]) > t)

kr++;

shiftSamp = max(kr, kl)-1;

if (shiftSamp) {

phi = -shiftSamp * 2 * pi / Nds;

for (n=1; n<Nds, n++) {

H_left[n] *= exp(n*phi*i);

H_right[n] *= exp(n*phi*i);

}

h_left = ifft(H_left);

h_right = ifft(H_right);

}

- Truncate h_left and h_right by keeping the first Ntaps real-value time-domain samples.

###### Audio Processing

For applying the filters, the uniformly partitioned overlap-save convolution method as specified in section A.13.1 shall be used. The filters shall be applied as the final step in the binauralized rendering, directly prior the end-of-chain limiter.

### Adaptive loudspeaker rendering

#### General

##### Preface

As an alternative to rendering and binauralizing the output to headphones, the playback over loudspeakers is specified. In this operation mode, the binaural spatializer (HRTF based renderer) is replaced with a dedicated loudspeaker-based renderer.

For a high quality listening experience, loudspeaker setups assume the listener to be situated in a dedicated fixed location, the so-called sweep spot. Typically, within a 6DOF playback situation, the listener is moving. Therefore, the 3D spatial rendering has to be instantly and continuously adapted to the changing listener position. This is achieved in two hierarchically nested technology levels:

- Gains and delays are applied to the loudspeaker signals such that at the loudspeaker signals reach the listener position at a similar gain and delay. Optionally a high shelving compensation filter is applied to each loudspeaker signal related to the current listener position and the loudspeakers’ orientation with respect to the listener.  This way, as a listener moves to positions off-axis for a loudspeaker or further away from it, high frequency loss due to the loudspeaker’s radiation high-frequency pattern is compensated.

- Due to the 6DoF movement, the angles between loudspeakers, objects and the listener change as a function of listener position. Therefore, the 3D amplitude panning algorithm is updated in real-time with the relative positions and angles of the varying listener position and the fixed loudspeaker configuration as set in the LSDF. All coordinates (listener position, source positions) are transformed into the listening room coordinate system.

##### Physical compensation level (Level 1)

Level 1: real-time updated compensation of loudspeaker (frequency-dependent) gain & delay enables ‘enhanced rendering of content’. By exploiting the tracked user position information, the listener can move within a large “sweet area” (rather than a sweet spot) and experience a stable sound stage in this large area when listening to legacy content (e.g. stereo, 5.1, 7.1+4H). For immersive formats (i.e., not for stereo), the sound seems to detach from the loudspeakers rather than collapse into the nearest speakers when walking away from the sweet spot, i.e. a quality somewhat close to what is known from wavefield synthesis, but for a single-user experience. For stereo reproduction, the technology offers left-right sound stage stability for a wide range of user positions (i.e. the range between the left and right loudspeakers at arbitrary distance).

Figure 102 — Overview of the Level 1 system with its main components and parameters.

The gain compensation in Level 1 is based on an amplitude decay law. In free field, the amplitude is proportional to 1/r, where r is the distance from the listener to a loudspeaker (1/r corresponds to 6dB decay per distance doubling). In a room, due to the presence of acoustic reflections and reverberation, sound is decaying more slowly as the distance to a loudspeaker increases. Therefore nearfield decay, farfield decay,  and critical distance parameters are used to specify decay rate as a function of distance to a loudspeaker. Additionally there is a nearfield-farfield transition parameter beta. The larger beta is, the faster is the transition between nearfield and farfield decay.

The delay compensation in Level 1 computes the propagation delay from each loudspeaker to the listener position and then applies a delay to each loudspeaker to compensate for the propagation delay differences between loudspeakers. Delays are normalized (offset added or subtracted) such that the smallest delay applied to a loudspeaker signal is zero.

##### Object rendering level (Level 2)

Level 2:  user-tracked object panning enables rendering of point sources (objects, channels) within the 6DoF play space and requires Level 1 as a prerequisite. Thus, it addresses the use case of ‘6DoF VR/AR rendering’.

A 3D amplitude panning algorithm is used which works in loudspeaker layers, e.g. horizontal and height layers. Each layer applies a 2D panning algorithm for the projection of the object onto the layer. The final 3D object is rendered by applying amplitude panning between the two virtual objects from the 2D panning in the two layers.

When an object is located above the highest layer, then 2D panning is applied in that layer. The final 3D object is rendered by applying amplitude panning between the virtual object from the 2D panning and an (non-existent) object in an upper vertical direction. The signal of the vertical object is equalized to mimic timbre of top sound and distributed to the loudspeakers of the highest layer.

When an object is located below the lowest layer, then 2D panning is applied in that layer. The final 3D object is rendered by applying amplitude panning between the virtual object from the 2D panning and an (non-existent) object in a lower vertical direction. The signal of the vertical object is equalized to mimic timbre of bottom sound and distributed to the loudspeakers of the lowest layer.

The vertical panning as described, is equally applicable to loudspeaker setups with one layer such as 5.1 and with multiple layers such as 7.4.6.

Levels  2 loudspeaker reproduction applied to object rendering faithfully renders MPEG-I scenes similar to headphone reproduction. Note that RIs of type Nondiegetic are point sources and are rendered accordingly, enabling also reverb to be rendered over loudspeaker inside an AE.

In Level 2 rendering, the DiscoverSESS stage and the Homogeneous Extent stage are not supported and are therefore not part of the renderer pipeline. In consequence, Homogeneous Extent is rendered as a point source. Heterogeneous extent is fully supported by Level 2 since it is based on a set of point sources. The SP-HOA and MP-HOA stages are not supported and are therefore not part of the renderer pipeline. The output of HOA sources shall be silent. Also, the Airflow Simulation stage is not supported.

#### Physical compensation level (Level 1)

##### Data elements and variables

###### Variables

SFREQ_MIN		minimum sample rate [Hz] = 44100

SFREQ_MAX		maximum sample rate [Hz] = 48000

VSOUND		speed of sound in air [m/s] = 340.0

MAX_DELAY		maximum delay [samples] = 960

OVERHEAD_GAIN		overhead [lin] = 0.25

framesize		number of samples per frame, default: 256

sfreq_Hz		sampling frequency of input audio, default: 48000

nchan		number of channels (loudspeakers)

max_delay		maximum delay [samples], default: MAX_DELAY

bypass_on		0: normal operation, 1: bypass, default: 0

ref_proc		0: normal operation, 1: processing like for sweet spot, default: 0

cal_system		0: normal operation, 1: calibrated system, default: 0

gain_on		0: gain off, 1: on, default: 1

delay_on		0: delay off, 1: on, default: 1

decay_1_dB			nearfield sound decay per distance doubling [dB], corresponds to LSDF parameter LoudspeakerSetup::LevelCompensation::slope1

decay_2_dB			farfield sound decay per distance doubling [dB], corresponds to LSDF parameter LoudspeakerSetup::LevelCompensation::slope2

beta		nearfield-farfield transition, >1 faster transition, corresponds to LSDF parameter LoudspeakerSetup::LevelCompensation::beta

crit_dist_m			critical distance [m], corresponds to LSDF parameter LoudspeakerSetup::LevelCompensation::criticalDistance

max_m_s		maximum rate of delay change [v in m/s], default: 1

max_m_s_s		maximum rate of delay 2nd order change [a in m/s], default: 1

gain_ms		gain smoothing time constant [ms], default: 40

sweet_spot		sweet spot position [m,m,m]

spk_pos		loudspeaker coordinates [m,m,m]

listener_pos		listener coordinates [m,m,m]

eq_on       		loudspeaker directivity compenstation eq 0: off, 1: on

eq_freq_Hz          			high shelving frequency [Hz], corresponds to LSDF parameter LoudspeakerSetup::DirectivityCompensation::eqFreq

eq_gain_dB          			high shelving gain [dB], corresponds to LSDF parameter LoudspeakerSetup::DirectivityCompensation::eqGainDb

eq_angle_start_deg      	off-axis-angle [degree] below which 0 dB eq, corresponds to LSDF parameter LoudspeakerSetup::DirectivityCompensation::eqAngleStart

eq_angle_range_deg      	off-axis-angle [degree] above which eq_gain_dB eq, corresponds to LSDF parameter LoudspeakerSetup::DirectivityCompensation::eqAngleRange

eq_warp             		eq_angle_start_deg-eq_angle_range_deg transition warping (1 = neutral)

All coordinates are relative to the listening room as defined in the LSDF file. These parameters are stored in the following data structures:

###### Public data structures

typedef struct rendering_gd_cfg {

int         framesize;

float       sfreq_Hz;

int         nchan;

float       max_delay;

} rendering_gd_cfg_t;

typedef struct rendering_gd_rt_cfg {

int         bypass_on;

int         ref_proc;

int         cal_system;

int         gain_on;

int         delay_on;

float       decay_1_dB;

float       decay_2_dB;

float       crit_dist_m;

float       beta;

float       max_m_s;

float       max_m_s_s;

float       gain_ms;

float       sweet_spot[3];

float       spk_pos[NCHANMAX][3];

float       listener_pos[3];

int         eq_on;

float       eq_freq_Hz;

float       eq_gain_dB;

float       eq_angle_start_deg;

float       eq_angle_range_deg;

float       eq_warp;

} rendering_gd_rt_cfg_t;

Internal parameters that are calculated from the above listed parameters and states are stored in the following structure:

###### Internal data structure

typedef struct {

/* static parameters */

float               sfreq_Hz;

int                 nchan;

int                 framesize;

/* real-time parameters */

int                 bypass_on;

int                 gain_on;

float               delta_gi;

float               delta_gd;

float               gain_alpha;

float               delay_delta;

float               delay_delta2;

int                 eq_on;

il_biquad_cf_t      iircoefs[NCHANMAX];

/* state */

float               delay0[NCHANMAX];

float               delay[NCHANMAX];

float               gain0[NCHANMAX];

float               gain[NCHANMAX];

il_biquad_st_t      iirmem[NCHANMAX];

} rendering_gd_data_t;

##### Stage description

###### Initialize

The loudspeaker setup and its associated compensation parameters is loaded from the given LSDF file described in annex B.3.

A structure of type rendering_gd_cfg_t is initialized with default values and the nchan field is set to the number of loudspeakers in the loudspeaker setup.

A structure of type rendering_gd_rt_cfg_t is initialized with default values. The loudspeaker positions from the LSDF file are stored in the field spk_pos. If the ReferencePoint element was given in the LSDF file, its coordinates are stored in the field sweet_spot. The field cal_system is set to the value of the attribute calibrated if present.

The aforementioned structures are passed to the rendering_gd_init function.

int rendering_gd_init(

rendering_gd_data_h          *handle,

rendering_gd_cfg_t           *cfg,

rendering_gd_rt_cfg_t        *rt_cfg

)

{

int err = 0;

rendering_gd_data_t *data = NULL;

if ((cfg->sfreq_Hz < SFREQ_MIN) || (cfg->sfreq_Hz > SFREQ_MAX)) {

return -7;

}

data = (rendering_gd_data_t *)calloc(1, sizeof(rendering_gd_data_t));

if (data == NULL) {

return -1;

}

*handle = data;

// Obviously these parameters need to be stored permanently as they are needed for processing

/* init */

data->sfreq_Hz =    cfg->sfreq_Hz;

data->nchan =       cfg->nchan;

data->framesize =   cfg->framesize;

// Return max delay parameter, needed for init of MPEG delay lines

cfg->max_delay =    MAX_DELAY;

// Compute internal parameters

/* set rt parameters */

err = rendering_gd_updatecfg(data, cfg, rt_cfg);

if (err)

return err;

// Set state to neutral

/* reset state */

rendering_gd_audioreset(data);

return 0;

}

###### Release

void rendering_gd_done(

rendering_gd_data_h          handle

)

{

rendering_gd_data_t   *data = (rendering_gd_data_t *)handle;

free(data);

}

###### Reset

Flush all internal buffers:

void rendering_gd_audioreset(

rendering_gd_data_h         handle

)

{

int i;

rendering_gd_data_t   *data = (rendering_gd_data_t *)handle;

for (i = 0; i < data->nchan; i++) {

data->gain[i] =     data->gain0[i];

data->delay[I] =    data->delay0[i];

}

ZEROMEM(data->iirmem, sizeof(data->iirmem));

}

###### Update real-time parameters

In the update thread, the virtual listener position is transformed into the listening room coordinate system. This is only relevant for VR scenes, in AR scenes the two coordinate systems coincide.

All further processing happens in the audio thread.

The structure of type rendering_gd_rt_cfg_t  is updated by setting the listener_pos field to the listener position (in the listening room coordinate system).

The structure is then passed to the rendering_gd_updatecfg function:

int rendering_gd_updatecfg(

rendering_gd_data_h          handle,

rendering_gd_rt_cfg_t        *rt_cfg

)

{

int     i, err = 0;

float   lpos_x, lpos_y, lpos_z, r_ref;

rendering_gd_data_t   *data = (rendering_gd_data_t *)handle;

data->bypass_on =   rt_cfg->bypass_on;

data->gain_on =     rt_cfg->gain_on;

/* reference readius is first loudspeaker's distance */

{

float x, y, z;

x = rt_cfg->sweet_spot[0] - rt_cfg->spk_pos[0][0];

y = rt_cfg->sweet_spot[1] - rt_cfg->spk_pos[0][1];

z = rt_cfg->sweet_spot[2] - rt_cfg->spk_pos[0][2];

r_ref = sqrt(x*x+y*y+z*z);

}

/* listener position */

lpos_x = rt_cfg->listener_pos[0];

lpos_y = rt_cfg->listener_pos[1];

lpos_z = rt_cfg->listener_pos[2];

// For the reference processing option, the listener position is set to the sweet spot.

if (rt_cfg->ref_proc) {

/* set listener position to sweet spot, to below evoke sweet spot processing */

lpos_x = rt_cfg->sweet_spot[0];

lpos_y = rt_cfg->sweet_spot[1];

lpos_z = rt_cfg->sweet_spot[2];

}

/* update gain delay calibration */

for (i = 0; i < data->nchan; i++) {

float r, x, y, z;

For each loudspeaker the compensation gain and delay is computed. The reference distance r_ref (computed above) is the distance at which gain and delay compensation are zero (dB, samples). Based on the loudspeaker's distance to listener r and reference distance r_ref, gain and delay compensation are computed.

/* vector: listener position - loudspeaker */

x = lpos_x - rt_cfg->spk_pos[i][0];

y = lpos_y - rt_cfg->spk_pos[i][1];

z = lpos_z - rt_cfg->spk_pos[i][2];

r = sqrt(x*x+y*y+z*z);

In freefield sound decays by 6dB per distance doubling. Often, in a room, decay is approximated by using less decay, e.g. 4dB per distance doubling. Alternatively, one can consider critical distance (hall radius). When one is near a loudspeaker, decay is decay_1_dB per distance doubling. Beyond the critical distance crit_dist_m sound is only decaying slowly, as specifyable by decay_2_dB. The transition between the fast and slow decay is determined by the beta parameter. A beta > 1 causes a fast transition between decay_1_dB and decay_2_dB and a beta < 1 provides a slow transition.

Gain compensation compensates gain changes due to the described sound decay.

/* gain relative to 0dB at r_ref */

{

float pow_nf_cd, pow_ff_cd, pow_nf, pow_ff, pow_ref, scale;

/* power at critical distance with decay 1 */

pow_nf_cd = pow(r_ref / rt_cfg->crit_dist_m, 2.0f * rt_cfg->decay_1_dB/6.0);

/* power at critical distance with decay 2 */

pow_ff_cd = pow(r_ref / rt_cfg->crit_dist_m, 2.0f * rt_cfg->decay_2_dB/6.0);

/* nearfield power at distance r */

pow_nf = pow(r_ref / (r+1e-20f), 2.0 * rt_cfg->decay_1_dB/6.0);

/* farfield power at distance r */

pow_ff = pow(r_ref / (r+1e-20f), 2.0 * rt_cfg->decay_2_dB/6.0);

/* scale pow_ff to be same as pow_nf at critical distance */

scale = pow_nf_cd / pow_ff_cd;

pow_ff *= scale;

/* power at r_ref */

pow_ref = powf(1.0f + powf(scale, rt_cfg->beta), (1.0f/rt_cfg->beta));

pow_ref = il_fminf(pow_ref, 1.0f/(OVERHEAD_GAIN*OVERHEAD_GAIN));

/* compenstation gain */

data->gain0[i] = powf(powf(pow_ref,rt_cfg->beta) / (powf(pow_ff,rt_cfg->beta) +

powf(pow_nf,rt_cfg->beta)), 0.5f/rt_cfg->beta) * OVERHEAD_GAIN;

}

Depending on distance of loudspeaker to listener position, sound transmission time is varying. These variations are compensated by applying delays. An offset MAX_DELAY/2 is added to the compensation delays, such that they are always positive.

/* negative of delay from spk to listener */

if (rt_cfg->delay_on) {

data->delay0[i] = MAX_DELAY/2 - (r-r_ref) / VSOUND * data->sfreq_Hz;

data->delay0[i] = fminf(data->delay0[i], MAX_DELAY-1);

data->delay0[i] = fmaxf(data->delay0[i], 0.0);

} else {

data->delay0[i] = MAX_DELAY/2;

}

}

An overhead is used, determined by OVERHEAD_GAIN. That is, this system can amplify signals, when a listener is far away from a loudspeaker up to a factor of 1/OVERHEAD_GAIN. Ifthe gains surpass this value, then all gains across the channels are scaled with the same factor such that the largest gain is 1.0 (0 dB). This corresponds to inter-channel linked limiter action.

/* limit gains to 0dB (linked "limiter action") */

{

float mx = 0.0f;

for (i = 0; i < data->nchan; i++) {

mx = il_fmaxf(data->gain0[i], mx);

}

if (mx > 1.0f) {

for (i = 0; i < data->nchan; i++) {

data->gain0[i] /= mx;

}

}

}

Number of samples the delay is allowed to change from frame to frame is computed as a function of maximum allowed movement velocity max_m_s

/* samples per frame for specified speed */

data->delay_delta = (float)data->framesize*rt_cfg->max_m_s/VSOUND;

data->delay_delta = MAX(data->delay_delta, 1.0/FD_IDX_MAX);

Number of samples the delay change is allowed to change from frame to frame is computed as a function of maximum allowed movement acceleration max_m_s_s

/* samples per frame delay change for specified acceleration */

data->delay_delta2 = (float)data->framesize*rt_cfg->max_m_s_s/VSOUND;

data->delay_delta2 = MAX(data->delay_delta2, 1.0/FD_IDX_MAX);

Gains are smoothed with singe-pole averaging. The averaging constant is computed as a function of the smoothing time constant gain_ms

/* gain smoothing constant */

data->gain_alpha  =  1.f - expf(- 1.f / (1e-3f * rt_cfg->gain_ms * data->sfreq_Hz));

Calibrated system option cal_system is used when we are operating on a system which applies already its own optimal gains and delays (and etc.) for the sweet spot. In this case, we are additionally computing the gain and delay compensation of the sweet spot is computed (above we computed these were computed for the listener position). What in this case is applied, is the difference between the two computations is applied.

/* make sweet spot action zero for calibrated systems */

if (rt_cfg->cal_system) {

float gain_ss, delay_ss; /* sweet spot gain/delay */

/* set listener position to sweet spot, and compute below sweet spot gains/delays */

lpos_x = rt_cfg->sweet_spot[0];

lpos_y = rt_cfg->sweet_spot[1];

lpos_z = rt_cfg->sweet_spot[2];

for (i = 0; i < data->nchan; i++) {

float r, x, y, z;

/* vector: listener position – loudspeaker */

x = lpos_x – rt_cfg->spk_pos[i][0];

y = lpos_y – rt_cfg->spk_pos[i][1] ;

z = lpos_z – rt_cfg->spk_pos[i][2] ;

r = sqrt(x*x+y*y+z*z) ;

/* negative of delay from spk to listener */

delay_ss = - (r-r_ref) / VSOUND * data->sfreq_Hz;

/* gain relative to 0dB at r_ref */

{

float pow_nf_cd, pow_ff_cd, pow_nf, pow_ff, pow_ref, scale;

/* power at critical distance with decay 1 */

pow_nf_cd = pow(r_ref / rt_cfg->crit_dist_m, 2.0f * rt_cfg->decay_1_dB/6.0);

/* power at critical distance with decay 2 */

pow_ff_cd = pow(r_ref / rt_cfg->crit_dist_m, 2.0f * rt_cfg->decay_2_dB/6.0);

/* nearfield power at distance r */

pow_nf = pow(r_ref / (r+1e-20f), 2.0 * rt_cfg->decay_1_dB/6.0);

/* farfield power at distance r */

pow_ff = pow(r_ref / (r+1e-20f), 2.0 * rt_cfg->decay_2_dB/6.0);

/* scale pow_ff to be same as pow_nf at critical distance */

scale = pow_nf_cd / pow_ff_cd;

pow_ff *= scale;

/* power at r_ref */

pow_ref = powf(1.0f + powf(scale, rt_cfg->beta), (1.0f/rt_cfg->beta));

pow_ref = il_fminf(pow_ref, 1.0f/(OVERHEAD_GAIN*OVERHEAD_GAIN));

/* compenstation gain */

gain_ss = powf(powf(pow_ref,rt_cfg->beta) / (powf(pow_ff,rt_cfg->beta) +

powf(pow_nf,rt_cfg->beta)), 0.5f/rt_cfg->beta);

}

/* remove action from sweet spot (because calibrated system */

if (rt_cfg->delay_on) {

data->delay0[i] -= delay_ss;

data->delay0[i] = fminf(data->delay0[i], MAX_DELAY-1);

data->delay0[i] = fmaxf(data->delay0[i], 0.0);

}

data->gain0[i] /= gain_ss;

}

/* limit gains to 0dB (interchannel-linked limiter action) */

{

float mx = 0.0f;

for (i = 0; i < data->nchan; i++) {

mx = il_fmaxf(data->gain0[i], mx);

}

if (mx > 1.0f) {

for (i = 0; i < data->nchan; i++) {

data->gain0[i] /= mx;

}

}

}

}

For each loudspeaker the angle is computed, indicating how much off axis the listener position is for that loudspeaker. Based on this angle, a high shelving filter is computed to compensate high frequency loss for off-axis listening.

/* update eq */

data->eq_on =       rt_cfg->eq_on;

for (i = 0; i < data->nchan; i++) {

float ux, uy, uz, ur, vx, vy, vz, vr, angle, gain, r;

if (rt_cfg->spk_dir[i][0] > 1000.0f) {

/* reference pos for angles is sweet spot */

ux = rt_cfg->sweet_spot[0] - rt_cfg->spk_pos[i][0];

uy = rt_cfg->sweet_spot[1] - rt_cfg->spk_pos[i][1];

uz = rt_cfg->sweet_spot[2] - rt_cfg->spk_pos[i][2];

} else {

/* reference pos for angles */

ux = rt_cfg->spk_dir[i][0] - rt_cfg->spk_pos[i][0];

uy = rt_cfg->spk_dir[i][1] - rt_cfg->spk_pos[i][1];

uz = rt_cfg->spk_dir[i][2] - rt_cfg->spk_pos[i][2];

}

ur = sqrt(ux*ux+uy*uy+uz*uz);

vx = lpos_x - rt_cfg->spk_pos[i][0];

vy = lpos_y - rt_cfg->spk_pos[i][1];

vz = lpos_z - rt_cfg->spk_pos[i][2];

vr = sqrt(vx*vx+vy*vy+vz*vz);

angle = (ux*vx+uy*vy+uz*vz)/(ur*vr+1e-20f);

// condition, otherwise with ref proc, can be 1.0000...0001, and acos on win undefined

angle = fminf(angle, 1.0f);

angle = acos(angle) * 180.0f / M_PI;

r = (angle - rt_cfg->eq_angle_start_deg)/(il_fmaxf(rt_cfg->eq_angle_range_deg –

rt_cfg->eq_angle_start_deg, 1e-20f));

r = il_fmaxf(il_fminf(r, 1.0f), 0.0f);

r = powf(r, rt_cfg->eq_warp);

gain = r * rt_cfg->eq_gain_dB;

il_biquad_hfshelving2(&data->iircoefs[i], rt_cfg->eq_freq_Hz, gain,

data->sfreq_Hz);

}

return err;

}

where M_PI = π.

###### Audio processing

After rendering_gd_updatecfg has been called, the function rendering_gd_process is called, specifying the input and output buffers:

void rendering_gd_process(

rendering_gd_data_h     handle,

float                   **x,            /* in:  input audio */

float                   **y             /* out: output audio */

)

{

float                   *x[NCHANMAX];

float                   *y[NCHANMAX];

int                     n, ch;

rendering_gd_data_t     *data = (rendering_gd_data_t *)handle;

/* bypass */

if (data->bypass_on) {

for (ch = 0; ch < data->nchan; ch++) {

for (n = 0; n < data->framesize; n++) {

y[ch][n] = x[ch][n] * OVERHEAD_GAIN;

}

}

} else

/* apply gain per channel */

for (ch = 0; ch < data->nchan; ch++) {

float   *in, *out, fdelay, fdelay_prev, fdstate[FD_ORDER];

int     i, ptr, ptr_prev;

in =    x[ch];

out =   y[ch];

Apply gains with single-pole averaging:

/* apply gain with smoothing */

{

float alpha, g, gt;

alpha = data->gain_alpha;

if (data->gain_on) {

gt = data->gain0[ch];

} else {

gt = OVERHEAD_GAIN;

}

g = data->gain[ch];

for (i = 0; i < subframe; i++) {

out[i] *= g;

g = alpha*gt + (1.0f-alpha)*g;

}

data->gain[ch] = g;

}

/* apply eq */

if (data->eq_on) {

il_biquad_filter(data->iircoefs[ch], &data->iirmem[ch], out, out, subframe);

}

}

Compute delays for external delay lines. The delay change per frame, and 2nd order delay change per frame is limited, to reduce artefacts and pitch-shifting:

/* compute smoothed delay for external delay lines */

for (ch = 0; ch < data->nchan; ch++) {

float fdelay;

/* limit delay change */

if (data->delay_sm[ch] > data->delay0[ch]) {

fdelay = fmaxf(data->delay0[ch], data->delay_sm[ch]-data->delay_delta);

} else {

fdelay = fminf(data->delay0[ch], data->delay_sm[ch]+data->delay_delta);

}

/* limit delay change 2nd order */

if (fabsf(fdelay – data->delay_sm[ch]) > data->delay_delta2) {

if (fdelay > data->delay_sm[ch]) {

fdelay = data->delay_sm[ch] + data->delay_delta2;

} else {

fdelay = data->delay_sm[ch] – data->delay_delta2;

}

}

/* save current target values */

data->delay_sm[ch] = fdelay;

}

return data->delay_sm;

}

The returned delay value for each output channel is used as target value for an associated variable delay line, which applies the appropriate delay to the corresponding output signal. These output delay lines use the same implementation as the VDLs described in clause 6.6.17.3.2.5.

#### Object rendering level (Level 2)

##### Data elements and variables

###### Variables

SFREQ_MIN	minimum sample rate [Hz] = 44100

SFREQ_MAX	maximum sample rate [Hz] = 48000

FRAMESIZE_MAX	maximum frame size sample rate [samples] = 512

NOBJMAX	maximum number of audio objects = 256

NCHANMAX	maximum number of output channels = 16

NLAYERS	number of loudspeaker layers = 2

BIT_L1	rendering bit layer 1 = (1<<0)

BIT_L2	rendering bit layer 2 = (1<<1)

BIT_BOTTOM	rendering bit bottom = (1<<2)

BIT_TOP	rendering bit top = (1<<3)

BIT_LFE	rendering bit LFE = 0 (LFE not used)

###### Public data structures

typedef struct rendering_pan_cfg {

int         framesize;              /* in:  number of samples per frame */

float       sfreq_Hz;               /* in:  sampling frequency of input audio */

int         nchan;                  /* in:  number of channels (loudspeakers) */

int         nobj;                   /* in:  number of objects to amplitude pan */

} rendering_pan_cfg_t;

###### Real-time updatable parameters:

typedef struct rendering_pan_rt_cfg {

int         bypass;                 /* 0: normal operation, 1: bypass */

int         ref_proc;               /* 0: normal operation, 1: processing like for sweet spot */

float       gain_ms;                /* gain smoothing time constant [ms] */

float       sweet_spot[3];          /* sweet spot position [m,m,m] */

float       listener_pos[3];        /* listener coordinates [m,m,m] */

float       spk_pos[NCHANMAX][3];   /* loudspeaker coordinates [m,m,m] */

/* spk_layer[NCHANMAX]:

Each speaker can have these bits/flags

bit0 (1):   loudspeakers for layer 1 panning

bit1 (2):   loudspeakers for layer 2 panning

bit2 (4):   loudspeakers for bottom direction panning

(usually these have also bit0 and/or bit1 set)

bit3 (8):   loudspeakers for top direction panning

(usually also bit0 and/or bit1 are set)

bit4 (16):  exclude loudspeaker for low-band processing

*/

int         spk_layer[NCHANMAX];

float       obj_pos[NOBJMAX][3];    /* object coordinates [m,m,m] */

int         obj_type[NOBJMAX];      /* 0: inactive, 1: active,

(future values: hom. Ext.) */

float       l_obj_min;              /* minimum distance listener-object [m]

object will fly over/under listener at this

distance, instead of going through listener */

int         ele_eq;                	/* 0: off, 1: default top/bottom eqs,

2, 3: alternative eqs */

float       ele_thr_deg;            /* 0..180 => elevation threshold

(0=off, >=90=elevation off) */

} rendering_pan_rt_cfg_t;

###### Internal data structures

typedef struct {

/* static parameters */

float               sfreq_Hz;

int                 nchan;

int                 nobj;

int                 framesize;

/* real-time parameters */

int                 bypass;

int                 layer_spks[NLAYERS][NOBJMAX][2];

float               layer_gains[NLAYERS][NOBJMAX][2];

float               pan_gains0[NOBJMAX][NCHANMAX];

float               gain_alpha;

int                 bottom_top[NOBJMAX];

float               pan_gains0_bottom_top[NOBJMAX][NCHANMAX];

il_biquad_cf_t      iircoefs_top[3] ;

il_biquad_cf_t      iircoefs_bottom[3] ;

/* state */

float               pan_gains[NOBJMAX][NCHANMAX];

int                 obj_type[NOBJMAX];

float               pan_gains_bottom_top[NOBJMAX][NCHANMAX];

float               tmp[FRAMESIZE_MAX];

il_biquad_st_t      iirmem_top[NOBJMAX][5];

il_biquad_st_t      iirmem_bottom[NOBJMAX][4];

} rendering_pan_data_t;

##### Stage description

###### Internal functions

void tangent_law(float alpha, float alpha0, float* a1, float* a2)

{

float sinsum, sindif, den;

// alpha0 => zero, gave very large gain, so added these safety range conditions

alpha0 =    il_fmaxf(alpha0, 2.0);

alpha =     il_fminf(alpha, alpha0);

alpha =     il_fmaxf(alpha, -alpha0);

alpha =     alpha * M_PI / 180.0;

alpha0 =    alpha0 * M_PI / 180.0;

sinsum =    sin(alpha0 + alpha);

sindif =    sin(alpha0 – alpha);

den =       sqrt(sinsum * sinsum + sindif * sindif);

*a1 =       sinsum / den;

*a2 =       sindif / den;

}

void object_flush(

rendering_pan_data_h     handle,

int                      index

)

{

int ch;

rendering_pan_data_t     *data = (rendering_pan_data_t *)handle;

for (ch = 0; ch < data->nchan; ch++) {

data->pan_gains0[index][ch] = 0.0f;

data->pan_gains[index][ch] = 0.0f;

data->pan_gains0_bottom_top[index][ch] = 0.0f;

data->pan_gains_bottom_top[index][ch] = 0.0f;

}

for (ch = 0; ch < 5; ch++) {

ZEROMEM(&data->iirmem_top[index][ch], sizeof(il_biquad_st_t));

}

for (ch = 0; ch < 4; ch++) {

ZEROMEM(&data->iirmem_bottom[index][ch], sizeof(il_biquad_st_t));

}}

where M_PI = π.

###### Configure

Get default parameters:

int rendering_pan_defaultcfg(

rendering_pan_rt_cfg_t        *rt_cfg,

int                           cicp_layout   /* in:  MPEG spk layout (0 = not defined) */

)

{

int i;

if (rt_cfg != NULL) {

For experimentation (default off):

rt_cfg->bypass =            0;

rt_cfg->ref_proc =          0;

These are default tuning parameter values:

rt_cfg->gain_ms =           40.0f;

rt_cfg->l_obj_min =         0.4 ;

rt_cfg->ele_eq   =          1;

rt_cfg->ele_thr_deg   =     0.0;

Set for each supported loudspeaker setup the internal layer bits of this renderer:

/* set layer assignments based on CICP layout */

if (cicp_layout != 0) {

/* reset layers */

for (i = 0; i < NCHANMAX; i++) {

rt_cfg->spk_layer[i] = 0;

}

/* set layers according to loudspeaker setup */

switch (cicp_layout) {

case 1:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 2:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 3:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 4:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 5:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 6:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 7:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_LFE;

rt_cfg->spk_layer[3] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[6] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[7] = BIT_L1;

rt_cfg->spk_layer[8] = BIT_L1;

break;

case 8:

// seems to be not used in MPEG-I

break;

case 9:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[3] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 10:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[3] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 11:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 12:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[6] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[7] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

break;

case 13:

// bottom layer panning not supported, so the bottom layer is used for bottom rendering only

// 24-channels, NCHANMAX too small, for now it is not defined

break;

case 14:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM | BIT_TOP;

rt_cfg->spk_layer[6] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[7] = BIT_L2 | BIT_TOP;

break;

case 15:

// bottom layer panning not supported, so the bottom layer is used for bottom rendering only

rt_cfg->spk_layer[0] = BIT_L1;

rt_cfg->spk_layer[1] = BIT_L1;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[3] = BIT_BOTTOM;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_BOTTOM;

rt_cfg->spk_layer[7] = BIT_L1;

rt_cfg->spk_layer[8] = BIT_L1;

rt_cfg->spk_layer[9] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[10] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[11] = BIT_L2 | BIT_TOP;

break;

case 16:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[7] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[8] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[9] = BIT_L2 | BIT_TOP;

break;

case 17:

// voice of god is not supported at this point, so it is just ignored for now

// solution: set BIT_TOP for voice of god and clear for all channels

// (but then eq has to be switched off, and TODO: testing)

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[7] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[8] = BIT_L2;

rt_cfg->spk_layer[9] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[10] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[11] = 0;

break;

case 18:

// voice of god is not supported at this point, so it is just ignored for now

// solution: set BIT_TOP for voice of god and clear for all channels

// (but then eq has to be switched off, and TODO: testing)

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[7] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[8] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[9] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[10] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[11] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[12] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[13] = 0;

break;

case 19:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[7] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[8] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[9] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[10] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[11] = BIT_L2 | BIT_TOP;

break;

case 20:

rt_cfg->spk_layer[0] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[1] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[2] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[3] = BIT_LFE;

rt_cfg->spk_layer[4] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[5] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[6] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[7] = BIT_L1 | BIT_BOTTOM;

rt_cfg->spk_layer[8] = BIT_L2;

rt_cfg->spk_layer[9] = BIT_L2;

rt_cfg->spk_layer[10] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[11] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[12] = BIT_L2 | BIT_TOP;

rt_cfg->spk_layer[13] = BIT_L2 | BIT_TOP;

break;

}

} else {

return -1;

}

}

return 0;

}

###### Initialize

int rendering_pan_init(

rendering_pan_data_h          *handle,

rendering_pan_cfg_t           *cfg,

rendering_pan_rt_cfg_t        *rt_cfg

)

{

int i, err = 0;

rendering_pan_data_t *data = NULL;

if ((cfg->sfreq_Hz < SFREQ_MIN) || (cfg->sfreq_Hz > SFREQ_MAX)) {

return -7;

}

if (cfg->framesize > FRAMESIZE_MAX) {

return -1;

}

data = (rendering_pan_data_t *)calloc(1, sizeof(rendering_pan_data_t));

if (data == NULL) {

return -1;

}

*handle = data;

/* zero out data */

ZEROMEM(data, sizeof(rendering_pan_data_t));

/* init */

data->sfreq_Hz =    cfg->sfreq_Hz;

data->nchan =       cfg->nchan;

data->framesize =   cfg->framesize;

data->nobj =        cfg->nobj;

for (i = 0; i < NOBJMAX; i++) {

data->obj_type[i] = 1;

}

/* set rt parameters */

err = rendering_pan_updatecfg(data, cfg, rt_cfg);

if (err) {

return err;

}

/* reset state */

rendering_pan_audioreset(data);

return 0;

}

###### Release

void rendering_pan_done(

rendering_pan_data_h          handle

)

{

rendering_pan_data_t   *data = (rendering_pan_data_t *)handle;

free(data);

}

###### Reset

Flush all internal buffers:

void rendering_pan_audioreset(

rendering_pan_data_h         handle

)

{

int                     o, c;

rendering_pan_data_t    *data = (rendering_pan_data_t *)handle;

for (o = 0; o < data->nobj; o++) {

for (c = 0; c < data->nchan; c++) {

data->pan_gains[o][c] = data->pan_gains0[o][c];

data->pan_gains_bottom_top[o][c] = data->pan_gains0_bottom_top[o][c];

}

}

ZEROMEM(data->iirmem_top, sizeof(data->iirmem_top));

ZEROMEM(data->iirmem_bottom, sizeof(data->iirmem_bottom));

}

###### Update real-time parameters

This function shall be called after rendering_pan_init() has been called.

Int rendering_pan_updatecfg(

rendering_pan_data_h          handle,

rendering_pan_rt_cfg_t        *rt_cfg

)

{

int     i, k, l, err = 0;

float   lpos[3], r_ref;

int     nspks_layer[NLAYERS+2];

rendering_pan_data_t    *data = (rendering_pan_data_t *)handle;

data->bypass =          rt_cfg->bypass;

For ref_proc set listener position to sweet spot (for test output without tracking):

for (i = 0; i < 3; i++) {

if (rt_cfg->ref_proc) {

lpos[i] = rt_cfg->sweet_spot[i];

} else {

lpos[i] = rt_cfg->listener_pos[i];

}

}

/* gain smoothing constant */

data->gain_alpha  =  1.f – expf(- 1.f / (1e-3f * rt_cfg->gain_ms * data->sfreq_Hz));

/* switch objects on or off */

for (i = 0; i < data->nobj; i++) {

if (!rt_cfg->obj_type[i] && data->obj_type[i]) {

/* flush objects which are switched off */

object_flush(handle, i);

}

data->obj_type[i] = rt_cfg->obj_type[i];

}

A reference radius is computed. An objects signal is rendered with 0dB when the object distance is equal to reference radius.

/* reference readius for object gain is first loudspeaker’s distance */

{

float x, y, z;

x = rt_cfg->sweet_spot[0] – rt_cfg->spk_pos[0][0];

y = rt_cfg->sweet_spot[1] – rt_cfg->spk_pos[0][1];

z = rt_cfg->sweet_spot[2] – rt_cfg->spk_pos[0][2];

r_ref = sqrt(x*x+y*y+z*z) ;

}

/* compute number of loudspeakers in each layer */

for (l = 0; l < NLAYERS+2; l++) {

nspks_layer[l] = 0;

for (k = 0; k < data->nchan; k++) {

if (rt_cfg->spk_layer[k] & (1<<l)) {

nspks_layer[l]++;

}

}

}

/* amplitude panning of objects */

ZEROMEM(data->pan_gains0, sizeof(data->pan_gains0));

ZEROMEM(data->pan_gains0_bottom_top, sizeof(data->pan_gains0_bottom_top));

for (i = 0; i < data->nobj; i++) {

MTVec3D l_obj, l_spk[NCHANMAX];

float   angle_spk_spk, angle_obj_spk;

float   angles[NCHANMAX], angle1, angle2, a1, a2;

int     idx1, idx2;

if (!data->obj_type[i]) {

/* skip inactive objects */

continue;

}

data->bottom_top[i] = 0;  // 0: bottom is used, 1: top is used

/* listener – object vector */

l_obj.x = rt_cfg->obj_pos[i][0] – lpos[0];

l_obj.y = rt_cfg->obj_pos[i][1] – lpos[1];

l_obj.z = rt_cfg->obj_pos[i][2] – lpos[2];

/* prevent issues when listener pos = object pos */

if (mtVectorLength3D(l_obj) < 1e-8) {

l_obj.x = 1e-8;

}

To prevent an object moving horizontally through the listener (ear plane), such objects are slightly shifted up or down when getting close to the listener.

/* as object gets closer to listener move it above / below through listener */

if (mtVectorLength3D(l_obj) < rt_cfg->l_obj_min) {

float y;

y = sqrtf(rt_cfg->l_obj_min*rt_cfg->l_obj_min

– l_obj.x*l_obj.x – l_obj.z*l_obj.z);

if (l_obj.y >= 0) {

l_obj.y = y;

} else {

l_obj.y = -y;

}

}

There is either only one layer (horizontal layer) or two layers (horizontal and height layer). Objects are amplitude panned separately in each layer by selecting the two angular closest loudspeakers and applying a tangent panning law between them. When an object is above the highest layer (horizontal layer if there is one layer, height layer if there are two layers), then a virtual top loudspeaker is considered. Similarly, for objects below the lowest layer (horizontal layer), a virtual bottom loudspeaker is considered. Additionally considering these virtual speakers, the system can have up to four layers.

/* compute angles between listener-object, and listener-loudspeaker directions */

for (k = 0; k < data->nchan; k++) {

/* listener - loudspeaker vector */

l_spk[k].x = rt_cfg->spk_pos[k][0] - lpos[0];

l_spk[k].y = rt_cfg->spk_pos[k][1] - lpos[1];

l_spk[k].z = rt_cfg->spk_pos[k][2] - lpos[2];

/* prevent issues when listener pos = spk pos */

if (mtVectorLength3D(l_spk[k]) < 1e-10) {

l_spk[k].x = 1e-8;

}

/* angle between listener-object, and listener-loudspeaker directions */

{

float l_spk_y, l_obj_y;

/* projection on loudspeaker layer plane, set y coordinates to zero */

l_spk_y = l_spk[k].y;

l_obj_y = l_obj.y;

l_spk[k].y = 0.0f;

l_obj.y = 0.0f;

angles[k] = mtAngleVectorVector(l_spk[k], l_obj);

l_spk[k].y = l_spk_y;

l_obj.y = l_obj_y;

}

}

/* panning for each layer */

for (l = 0; l < NLAYERS; l++) {

/* find the two loudspeakers best matching object position */

angle1 = 1e10;

angle2 = 1e10;

idx1 = -1;

idx2 = -1;

Find loudspeaker with smallest angle to the object:

for (k = 0; k < data->nchan; k++) {

if (!(rt_cfg->spk_layer[k] & (1<<l))) {

/* skip channels of other layers */

continue;

}

if (angles[k] < angle1) {

angle1 = angles[k];

idx1 = k;

}

}

Find loudspeaker with second smallest angle to the object (on other side of object):

for (k = 0; k < data->nchan; k++) {

if (!(rt_cfg->spk_layer[k] & (1<<l))) {

/* skip channels of other layers */

continue;

}

if (k == idx1) {

/* skip already selected loudspeaker */

continue;

}

if (angles[k] < angle2) {

{

float l_spk_y[2];

/* projection on loudspeaker layer plane, set y coordinates to zero */

l_spk_y[0] = l_spk[k].y;

l_spk_y[1] = l_spk[idx1].y;

l_spk[k].y = 0.0f;

l_spk[idx1].y = 0.0f;

angle_spk_spk = mtAngleVectorVector(l_spk[idx1], l_spk[k]);

l_spk[k].y = l_spk_y[0];

l_spk[idx1].y = l_spk_y[1];

}

//angle_obj_spk = mtAngleVectorVector(l_obj, l_spk[k]);

angle_obj_spk = angles[k];

//if ((angle_spk_spk >= angle1) && (angle_spk_spk >= angles[k])) {

if (angle_spk_spk+0.1f >= angle_obj_spk) { // +0.1 numerical

angle2 = angles[k];

idx2 = k;

}

}

}

/* compute amplitude panning gains for object */

if (idx2 >= 0) {

float alpha0, alpha;

alpha0 = 0.5*(angle1+angle2);

alpha = alpha0 - angle1;

tangent_law(alpha, alpha0, &a1, &a2);

/* save layer's panning info */

data->layer_spks[l][i][0] = idx1;

data->layer_spks[l][i][1] = idx2;

data->layer_gains[l][i][0] = a1;

data->layer_gains[l][i][1] = a2;

} else {

/* indicate that layer has not valid panning */

data->layer_spks[l][i][0] = -1;

}

}

/* pan between layers */

{

If there is only one horizontal layer, an object results in a phantom object in the horizontal layer. Panning with only one layer (only first layer valid), e.g. 5.1:

if ((data->layer_spks[0][i][0] >= 0) && (data->layer_spks[1][i][0] < 0)) {

/* first layer valid */

float l_obj_l_ang_vertical, l_obj_l_ang;

/* angle between object and panned object */

{

MTVec3D l_obj_l;

/* compute for each layer: listener - pan-direction vector */

idx1 = data->layer_spks[0][i][0];

idx2 = data->layer_spks[0][i][1];

a1 = data->layer_gains[0][i][0];

a2 = data->layer_gains[0][i][1];

//

l_obj_l.x = a1*rt_cfg->spk_pos[idx1][0]

+ a2*rt_cfg->spk_pos[idx2][0] - lpos[0];

l_obj_l.y = a1*rt_cfg->spk_pos[idx1][1]

+ a2*rt_cfg->spk_pos[idx2][1] - lpos[1];

l_obj_l.z = a1*rt_cfg->spk_pos[idx1][2]

+ a2*rt_cfg->spk_pos[idx2][2] - lpos[2];

/* compute angle between panned object and object for layer */

l_obj_l_ang = mtAngleVectorVector(l_obj_l, l_obj);

/* threshold for elevation */

l_obj_l_ang = il_fmaxf(l_obj_l_ang-rt_cfg->ele_thr_deg, 0.0f);

}

The elevation is modelled by considering a virtual top or bottom source. The object is amplitude panned between its horizontal layer phantom source and virtual top or bottom source.

/* angle between object and bottom/top direction */

{

// VIVE: y=-1 = bottom direction!

MTVec3D vertical;

float   l_obj_l_ang_bottom, l_obj_l_ang_top;

// bottom

vertical.x = 0.0f;

vertical.y = -1.0f;

vertical.z = 0.0f;

l_obj_l_ang_bottom = mtAngleVectorVector(vertical, l_obj);

// top

vertical.x = 0.0f;

vertical.y = 1.0f;

vertical.z = 0.0f;

l_obj_l_ang_top = mtAngleVectorVector(vertical, l_obj);

// min angle

l_obj_l_ang_vertical = il_fminf(l_obj_l_ang_bottom, l_obj_l_ang_top);

data->bottom_top[i] = 0; // bottom direction

if (l_obj_l_ang_top < l_obj_l_ang_bottom) {

data->bottom_top[i] = 1; // top direction

}

}

/* compute panning gains */

{

float alpha0, alpha, a_layer, a_vertical;

alpha0 = 0.5*(l_obj_l_ang+l_obj_l_ang_vertical);

alpha = alpha0 - l_obj_l_ang;

tangent_law(alpha, alpha0, &a_layer, &a_vertical);

data->pan_gains0[i][idx1] = a1 * a_layer;

data->pan_gains0[i][idx2] = a2 * a_layer;

/* contribution for vertical direction */

if (a_vertical > 0.01f) {

/* distribute power equally to all first layer spks */

a_vertical /= sqrtf(nspks_layer[2]);

for (k = 0; k < data->nchan; k++) {

if (data->bottom_top[i] && (rt_cfg->spk_layer[k] & 4)) {

data->pan_gains0_bottom_top[i][k] += a_vertical;

} else if (!data->bottom_top[i] && (rt_cfg->spk_layer[k] & 8)){

data->pan_gains0_bottom_top[i][k] += a_vertical;

}

}

}

}

In the above processing, each object has been transformed to a phantom object in each layer. Based on the height of the object, amplitude panning is applied to the two phantom objects to get one phantom source from both phantom sources matching the object position. Panning with two layers:

} else if ((data->layer_spks[0][i][0] >= 0)

&& (data->layer_spks[1][i][0] >= 0)) {

MTVec3D l_obj_l[NLAYERS];

float   l_obj_l_ang[NLAYERS], angle_l_l, alpha, alpha0;

float a_bottom, a_top;

a_bottom = 0.0f;

a_top = 0.0f;

/* both layers valid */

for (l = 0; l < NLAYERS; l++) {

/* compute for each layer: listener - pan-direction vector */

idx1 = data->layer_spks[l][i][0];

idx2 = data->layer_spks[l][i][1];

a1 = data->layer_gains[l][i][0];

a2 = data->layer_gains[l][i][1];

//

l_obj_l[l].x = a1*rt_cfg->spk_pos[idx1][0]

+ a2*rt_cfg->spk_pos[idx2][0] - lpos[0];

l_obj_l[l].y = a1*rt_cfg->spk_pos[idx1][1]

+ a2*rt_cfg->spk_pos[idx2][1] - lpos[1];

l_obj_l[l].z = a1*rt_cfg->spk_pos[idx1][2]

+ a2*rt_cfg->spk_pos[idx2][2] - lpos[2];

/* compute angle between panned object and object for layer */

l_obj_l_ang[l] = mtAngleVectorVector(l_obj_l[l], l_obj);

}

angle_l_l = mtAngleVectorVector(l_obj_l[0], l_obj_l[1]);

/* if direction between layers: tangent law between layers */

if ((angle_l_l > l_obj_l_ang[0]) && (angle_l_l > l_obj_l_ang[1])) {

alpha0 = 0.5*(l_obj_l_ang[0]+l_obj_l_ang[1]);

alpha = alpha0 - l_obj_l_ang[0];

tangent_law(alpha, alpha0, &a1, &a2);

If the object position is below the lower layer, then the object is amplitude panned between its lower layer phantom source and its virtual bottom source.

/* panning between layer 0 and bottom direction */

} else if (l_obj_l_ang[0] < l_obj_l_ang[1]) {

/* threshold for elevation */

l_obj_l_ang[0] = il_fmaxf(l_obj_l_ang[0]-rt_cfg->ele_thr_deg, 0.0f);

l_obj_l_ang[1] = il_fmaxf(l_obj_l_ang[1]-rt_cfg->ele_thr_deg, 0.0f);

// VIVE: y=-1 = bottom direction!

MTVec3D bottom;

float   l_obj_l_ang_bottom;

bottom.x = 0.0f;

bottom.y = -1.0f;

bottom.z = 0.0f;

l_obj_l_ang_bottom = mtAngleVectorVector(bottom, l_obj);

alpha0 = 0.5*(l_obj_l_ang[0]+l_obj_l_ang_bottom);

alpha = alpha0 - l_obj_l_ang[0];

tangent_law(alpha, alpha0, &a1, &a_bottom);

a2 = 0.0f;

If the object position is above the height layer, then the object is amplitude panned between its height layer phantom source and its virtual top source.

/* panning between layer 1 and top direction */

} else {

// VIVE: y=1 = top direction!

MTVec3D top;

float   l_obj_l_ang_top;

data->bottom_top[i] = 1;

top.x = 0.0f;

top.y = 1.0f;

top.z = 0.0f;

l_obj_l_ang_top = mtAngleVectorVector(top, l_obj);

alpha0 = 0.5*(l_obj_l_ang[1]+l_obj_l_ang_top);

alpha = alpha0 - l_obj_l_ang[1];

tangent_law(alpha, alpha0, &a2, &a_top);

a1 = 0.0f;

}

According to loudspeakers’ layer bits, virtual top and bottom source signals are rendered through a set of loudspeakers.

{

int spk_1_L1, spk_2_L1, spk_1_L2, spk_2_L2;

spk_1_L1 = data->layer_spks[0][i][0];

spk_2_L1 = data->layer_spks[0][i][1];

spk_1_L2 = data->layer_spks[1][i][0];

spk_2_L2 = data->layer_spks[1][i][1];

/* pan between layers */

data->pan_gains0[i][spk_1_L1] = a1*data->layer_gains[0][i][0];

data->pan_gains0[i][spk_2_L1] = a1*data->layer_gains[0][i][1];

data->pan_gains0[i][spk_1_L2] = a2*data->layer_gains[1][i][0];

data->pan_gains0[i][spk_2_L2] = a2*data->layer_gains[1][i][1];

/* modify normalization when signal of two layers goes to same loudspeaker */

if ((a1 > 0.05f) && (a2 > 0.05f)) {

/* only if both layers are used */

a1 = 1.0f / (a1 + a2);

a2 = 1.0f - a1;

if (spk_1_L1 == spk_1_L2) {

data->pan_gains0[i][spk_1_L1] = a1*data->layer_gains[0][i][0]

+ a2*data->layer_gains[1][i][0];;

}

if (spk_2_L1 == spk_2_L2) {

data->pan_gains0[i][spk_2_L1] = a1*data->layer_gains[0][i][1]

+ a2*data->layer_gains[1][i][1];;

}

if (spk_1_L1 == spk_2_L2) {

data->pan_gains0[i][spk_1_L1] = a1*data->layer_gains[0][i][0]

+ a2*data->layer_gains[1][i][1];;

}

if (spk_2_L1 == spk_1_L2) {

data->pan_gains0[i][spk_2_L1] = a1*data->layer_gains[0][i][1]

+ a2*data->layer_gains[1][i][0];;

}

}

/* contribution for bottom direction */

if (a_bottom > 0.01f) {

a_bottom /= sqrtf(nspks_layer[2]);

/* distribute power equally to all first layer spks */

for (k = 0; k < data->nchan; k++) {

if (rt_cfg->spk_layer[k] & 4) {

data->pan_gains0_bottom_top[i][k] += a_bottom;

}

}

}

/* contribution for top direction */

if (a_top > 0.01f) {

a_top /= sqrtf(nspks_layer[3]);

/* distribute power equally to all second layer spks */

for (k = 0; k < data->nchan; k++) {

if (rt_cfg->spk_layer[k] & 8) {

data->pan_gains0_bottom_top[i][k] += a_top;

}

}

}

}

} else {

/* no layer valid */

printf("Panning not valid!\n");

}

}

}

Compute EQs for top/bottom virtual source signals

// compute bottom / top sound equalizers

for (i = 0; i < 5; i++) {

il_biquad_unity(&data->iircoefs_top[i]);

}

for (i = 0; i < 4; i++) {

il_biquad_unity(&data->iircoefs_bottom[i]);

}

if (rt_cfg->ele_eq == 1) {

float sc;

il_biquad_hfshelving2(&data->iircoefs_top[0], 600.0f, -2.8f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[1], 8400.0f, 6.0f, 3.0f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[2], 1250.0f, 2.5f, 2.5f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[3], 720.0f, -4.0f, 2.5f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[4], 13000.0f, -7.0f, 2.5f, data->sfreq_Hz);

sc = 4.0f/3.0f; // as bottom eq is defined for ele = -45

il_biquad_lfshelving2(&data->iircoefs_bottom[0], 800.0f, -2.0f*sc, data->sfreq_Hz);

il_biquad_hfshelving2(&data->iircoefs_bottom[1], 3000.0f, -3.0f*sc,

data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_bottom[2], 9000.0f, 4.0f*sc, 3.0f,

data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_bottom[3], 6400.0f, -6.0f*sc, 4.0f,

data->sfreq_Hz);

} else if (rt_cfg->ele_eq == 2) {

il_biquad_peaking(&data->iircoefs_top[0], 800.0f, -10.0f, 2.0f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_bottom[0], 6000.0f, -10.0f, 1.5f,

data->sfreq_Hz);

} else if (rt_cfg->ele_eq == 3) {

il_biquad_peaking(&data->iircoefs_top[0], 800.0f, -6.0f, 2.0f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[1], 1300.0f, 4.0f, 2.5f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_top[2], 7500.0f, 5.0f, 2.0f, data->sfreq_Hz);

il_biquad_hfshelving2(&data->iircoefs_bottom[0], 600.0f, -3.0f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_bottom[1], 900.0f, 4.0f, 1.0f, data->sfreq_Hz);

il_biquad_peaking(&data->iircoefs_bottom[2], 8000.0f, 3.0f, 2.0f, data->sfreq_Hz);

}

return err;

}

###### Audio processing

Process one frame of audio, return one frame of audio:

void rendering_pan_process(

rendering_pan_data_h     handle,

float                    **x,             /* in:  input audio */

float                    **y              /* out: output audio */

)

{

int                     framesize;

rendering_pan_data_t     *data = (rendering_pan_data_t *)handle;

framesize = data->framesize;

/* bypass: simply passing objects to output channels */

if (data->bypass) {

int c, i, nobj;

nobj = data->nobj;

nobj = MIN(nobj, data->nchan);

for (c = 0; c < nobj; c++) {

if (!data->obj_type[c]) {

/* silence inactive objects */

for (i = 0; i < data->framesize; i++) {

y[c][i] = 0.0f;

}

}

for (i = 0; i < data->framesize; i++) {

y[c][i] = x[c][i];

}

}

for ( ; c < data->nchan; c++) {

for (i = 0; i < data->framesize; i++) {

y[c][i] = 0.0f;

}

}

return;

}

/* zero output */

{

int c, i;

for (c = 0; c < data->nchan; c++) {

for (i = 0; i < data->framesize; i++) {

y[c][i] = 0.0f;

}

}

}

/* render objects */

{

int c, i, o;

for (o = 0; o < data->nobj; o++) {

if (!data->obj_type[o]) {

/* skip inactive objects */

continue;

}

Render objects to horizontal and height layers, sample-wise interpolate rendering gains.

for (c = 0; c < data->nchan; c++) {

for (i = 0; i < data->framesize; i++) {

y[c][i] += data->pan_gains[o][c] * x[o][i];

data->pan_gains[o][c] = data->gain_alpha*data->pan_gains0[o][c]

+ (1.0f-data->gain_alpha)*data->pan_gains[o][c];

}

}

}

Apply EQs to top/bottom virtual source signals, render these signals to existing loudspeakers.

for (o = 0; o < data->nobj; o++) {

if (!data->obj_type[o]) {

/* skip inactive objects */

continue;

}

if (data->bottom_top[o]) {

for (i = 0; i < 5; i++) {

il_biquad_filter(

data->iircoefs_top[i],

&data->iirmem_top[o][i],

i == 0 ? x[o] : data->tmp,

data->tmp, data->framesize);

}

} else {

for (i = 0; i < 4; i++) {

il_biquad_filter(

data->iircoefs_bottom[i],

&data->iirmem_bottom[o][i],

i == 0 ? x[o] : data->tmp,

data->tmp, data->framesize);

}

}

for (c = 0; c < data->nchan; c++) {

for (i = 0; i < data->framesize; i++) {

y[c][i] += data->pan_gains_bottom_top[o][c] * data->tmp[i];

data->pan_gains_bottom_top[o][c] =

data->gain_alpha*data->pan_gains0_bottom_top[o][c]

+ (1.0f-data->gain_alpha)*data->pan_gains_bottom_top[o][c];

}

}

}

}

}

#### Filter related functions

For adaptive loudspeaker rendering, several filter design and application functions are called. These functions are listed in the following pseudo code.

##### Design a unity filter of 2nd order

void il_biquad_unity(

il_biquad_cf_t          *coefs      /* out: filter coefficients */

)

{

coefs->b[0] =   1.f;

coefs->b[1] =   0.f;

coefs->b[2] =   0.f;

coefs->a[0] =   0.f;

coefs->a[1] =   0.f;

}

##### Design a high frequency shelving filter of 2nd order

void il_biquad_hfshelving2(

il_biquad_cf_t          *coefs,     /* out: filter coefficients */

float                   fc,         /* in:  cut-off frequency [Hz] */

float                   G,          /* in:  gain [dB] */

float const             sfreq       /* in:  sampling rate [Hz] */

)

{

double          wd, nrm, phi, sp, cp, V0;

/* restrict range of cut-off frequency to Nyquist */

fc =            fminf(fc, sfreq * 0.5f);

/* shift cut filters cutoff frequency to have symmetric curves as boost filters */

V0 =            pow(10.0, 0.05 * G);

V0 =            pow(V0, 1.0/2.0);

if (G < 0.f)

wd =        tan(M_PI * fc / sfreq) * V0;

else

wd =        tan(M_PI * fc / sfreq);

/* compute next normalized Butterworth polynomial: s^2 + 2*sin(phi)*s + 1 */

phi =           M_PI * (2.0 * 0 + 1.0) / (2.0  * 2);

sp =            sin(phi);

cp =            cos(phi);

/* normalization factor */

nrm =           1.0 / ((wd + sp) * (wd + sp) + cp*cp);

/* coefficient derivation */

coefs->a[0] =   (float) (nrm * 2.0 * ((wd + sp) * (wd - sp) - cp*cp));

coefs->a[1] =   (float) (nrm * ((wd - sp) * (wd - sp) + cp*cp));

sp *=           V0;

cp *=           V0;

coefs->b[0] =   (float) (nrm * ((wd + sp) * (wd + sp) + cp*cp));

coefs->b[1] =   (float) (nrm * 2.0 * ((wd + sp) * (wd - sp) - cp*cp));

coefs->b[2] =   (float) (nrm * ((wd - sp) * (wd - sp) + cp*cp));

}

where M_PI = π.

##### Design a low frequency shelving filter of 2nd order

void il_biquad_lfshelving2(

il_biquad_cf_t          *coefs,     /* out: filter coefficients */

float                   fc,         /* in:  cut-off frequency [Hz] */

float                   G,          /* in:  gain [dB] */

float const             sfreq       /* in:  sampling rate [Hz] */

)

{

double          wd, nrm, phi, sp, cp, V0;

/* restrict range of cut-off frequency to Nyquist */

fc =            fminf(fc, sfreq * 0.5f);

/* shift cut filters cutoff frequency to have symmetric curves as boost filters */

V0 =            powf(10.0, 0.05 * G);

V0 =            powf(V0, 1.0/2.0);

if (G < 0.f)

wd =        tanf(M_PI * fc / (sfreq)) / V0 ;

else

wd =        tanf(M_PI * fc / sfreq);

/* compute next normalized Butterworth polynomial: s^2 + 2*sin(phi)*s + 1 */

phi =           M_PI * (2.0 * 0 + 1.0) / (2.0  * 2);

sp =            sin(phi);

cp =            cos(phi);

/* normalization factor */

nrm =           1.0 / ((sp + 1.0/wd) * (sp + 1.0/wd) + cp*cp);

/* coefficient derivation */

coefs->a[0] =   (float) (nrm * 2.0 * ((sp + 1.0/wd) * (sp - 1.0/wd) + cp*cp));

coefs->a[1] =   (float) (nrm * ((sp - 1.0/wd) * (sp - 1.0/wd) + cp*cp));

sp *=           V0;

cp *=           V0;

coefs->b[0] =   (float) (nrm * ((sp + 1.0/wd) * (sp + 1.0/wd) + cp*cp));

coefs->b[1] =   (float) (nrm * 2.0 * ((sp + 1.0/wd) * (sp - 1.0/wd) + cp*cp));

coefs->b[2] =   (float) (nrm * ((sp - 1.0/wd) * (sp - 1.0/wd) + cp*cp));

}

where M_PI = π.

##### Design peaking filter of 2nd order

void il_biquad_peaking(

il_biquad_cf_t          *coefs,     /* out: filter coefficients */

float                   fc,         /* cut-off frequency [Hz] */

float                   G,          /* gain [dB] */

float                   Q,          /* Q factor */

float const             sfreq       /* sampling rate [Hz] */

)

{

double   wd, gd, qd, nrm;

/* restrict range of cut-off frequency to Nyquist */

fc =                fminf(fc, sfreq * 0.5f);

wd =                tanf(M_PI * fc / sfreq);

gd =                powf(10.0, 0.05 * fabsf(G));

qd =                1.0 / Q;

if (G > 0.f) {

/* normalization factor */

nrm =           1.0 / (1.0 + wd*qd + wd*wd);

/* coefficient derivation */

coefs->b[0] =   (float) (nrm * (1.0 + gd*qd*wd + wd*wd));

coefs->b[1] =   (float) (nrm * (2.0 * (wd*wd - 1.0)));

coefs->b[2] =   (float) (nrm * (1.0 - gd*qd*wd + wd*wd));

coefs->a[0] =   (float) (nrm * (2.0 * (wd*wd - 1.0)));

coefs->a[1] =   (float) (nrm * (1.0 - qd*wd + wd*wd));

}

else if (G < 0.f) {

/* normalization factor */

nrm =           1.0 / (1.0 + gd*qd*wd + wd*wd);

/* coefficient derivation */

coefs->b[0] =   (float) (nrm * (1.0 + qd*wd + wd*wd));

coefs->b[1] =   (float) (nrm * (2.0 * (wd*wd - 1.0)));

coefs->b[2] =   (float) (nrm * (1.0 - qd*wd + wd*wd));

coefs->a[0] =   (float) (nrm * (2.0 * (wd*wd - 1.0)));

coefs->a[1] =   (float) (nrm * (1.0 - gd*qd*wd + wd*wd));

}

else {

coefs->b[0] =   1.f;

coefs->b[1] =   0.f;

coefs->b[2] =   0.f;

coefs->a[0] =   0.f;

coefs->a[1] =   0.f;

}

}

where M_PI = π.

##### Apply a biquad IIR filter

void il_biquad_filter(

il_biquad_cf_t const    coefs,      /* in:      filter coefficients */

il_biquad_st_t          *state,     /* in/out:  filter memory */

float const             *in,        /* in:      input signal buffer */

float                   *out,       /* out:     output signal buffer */

int const               nsize       /* in:      input signal buffer size */

)

{

int         k;

float       xn;

float       yn, xnd, xndd, ynd, yndd;

/* load state */

xnd =       state->xnd;     /* x(n-1) */

xndd =      state->xndd;    /* x(n-2) */

ynd =       state->ynd;     /* y(n-1) */

yndd =      state->yndd;    /* y(n-2) */

for (k = 0; k < nsize; k++) {

/* input */

xn =    *in++;

/* 2nd order IIR filter */

yn =    xn * coefs.b[0];

yn +=   xnd * coefs.b[1];

yn +=   xndd * coefs.b[2];

yn -=   ynd * coefs.a[0];

yn -=   yndd * coefs.a[1];

/* output */

*out++ =    yn;

/* prepare next iteration */

xndd =  xnd;

xnd =   xn;

yndd =  ynd;

ynd =   yn;

}

/* save state */

state->xnd =    xnd;    /* x(n)   */

state->xndd =   xndd;   /* x(n-1) */

state->ynd =    ynd;    /* y(n)   */

state->yndd =   yndd;   /* y(n-1) */

}

## Limiter

### General

The limiter provides clipping protection for the auralized signal that is output from the spatializer. It has an attack time of 0.0025 seconds, a threshold of 0.89 (-0.01 dBFS) and implements a heuristic to determine the release time. To avoid any processing delay, no look-ahead is used.

The releaseHeuristic is a value ranging from 0 to 1. It increases on each frame that contains a sample with a value above the threshold and decreases on each frame with all sample values below the threshold.

Values of 0 and 1 map to the shortest (0.005 seconds) and longest (1 second) release time, respectively.

The goal of this heuristic is to avoid the "pumping" effect when only sharp transients exceed the threshold, but also keep the gain curve smoother if the threshold is exceeded in many frames in a short span of time.

### Data elements and variables

output	Input/Output buffer

gain	Gain to be applied to the output. It varies per sample and reaches 99% of frame_gain value within attack time from frame start

releaseHeuristic	Used to calculate the release time

limiter_mem[2]	State variable for previous gain and releaseHeuristic values. Shall be initialized to [1.f, 0.f]

output_frame	Number of samples per channel in the buffer

nchan_out	Number of channels in the buffer

max_val	Absolute maximum sample value in the buffer

output_Fs	System audio sampling rate, always set to 48000

attack_seconds	Attack in seconds, set to 0.0025

threshold	Threshold, which needs to be exceeded, to activate the limiter, set to 0.89

frame_gain	Target gain to be applied to current frame

attack_constant	Used to calculate the gain

release_constant	Used to calculate the gain

EPSILON	Set to 0.000000000000001f and used for float comparison

### Description

For the current frame the limiter will be active, if either the threshold is exceeded in this frame, or the threshold has been exceeded in frames processed earlier and the system has not returned yet to its default state. The limiter works as follows:

- The maximum absolute sample value for the current frame max_val is calculated:

max_val = 0.f;

for ( i = 0; i < output_frame; i++ ) {

for ( c = 0; c < nchan_out; c++ ) {

tmp = abs( output[c][i] );

if ( tmp > max_val ) {

max_val = tmp;

}

}

}

- Depending on max_val the values for releaseHeuristic and frame_gain are calculated. Limiting will not be applied and the function returns early if the threshold is not exceeded and the system is in its default state:

releaseHeuristic = limiter_mem[1];

if ( max_val > threshold ) {

frame_gain = threshold / max_val;

releaseHeuristic = min( 1.f, releaseHeuristic + ( 4.f * output_frame / output_Fs ) );

} else {

releaseHeuristic = max( 0.f, releaseHeuristic - ( output_frame / output_Fs ) );

if ( limiter_mem[0] >= 1.f - EPSILON ) {

limiter_mem[0] = 1.f;

limiter_mem[1] = releaseHeuristic;

return;

}

frame_gain = 1.f;

}

- attack_constant and release_constant are calculated, for both variables the denominator of the exponent determines after how many samples the gain curve will reach 99% of its target value:

gain = limiter_mem[0];

attack_constant = pow( 0.01f, 1.0f / ( attack_seconds * output_Fs ) );

release_constant = pow( 0.01f, 1.0f / ( 0.005f * powf( 200.f, releaseHeuristic ) * output_Fs ) );

- Limiting is applied with smooth sample-individual gains gain derived from the frame_gain. These gains are common to all channels followed by a hard clipping to +/-1 on individual channels.:

for ( i = 0; i < output_frame; i++ ) {

if ( frame_gain < gain ) {

gain = attack_constant * ( gain - frame_gain ) + frame_gain;

} else {

gain = release_constant * ( gain - frame_gain ) + frame_gain;

}

for ( c = 0; c < nchan_out; c++ ) {

output[c][i] = gain * output[c][i];

if ( output[c][i] > 1.0f ) {

output[c][i] = 1.0f;

} else if ( output[c][i] < -1.0f ) {

output[c][i] = -1.0f;

}

}

}

- The current state is saved:

limiter_mem[0] = gain;

limiter_mem[1] = releaseHeuristic;

## Interface for audio utilization information

### General

The audio source elements that the renderer utilizes varies over time. The HOA sources that are required for rendering 6DoF HOA content (see 6.6.26 and 6.6.27) depends on the listener position. In case of 6DoF HOA rendering according to subclause 6.6.26, the HOA sources that are utilized for rendering are required either at first or at full order. In case of 6DoF HOA rendering according to subclause 6.6.27 all HOA sources that are utilized for rendering required at full order. In addition, interactive audio elements are not used by the renderer when inactive. This subclause describes the renderer output interface for describing the audio elements currently utilized by the renderer.

### Syntax and semantics of an interface for renderer audio utilization

The output interface shall provide the list of audio elements currently utilized for 6DoF HOA rendering, their used order and interactive audio elements that are currently inactive. The interface may be implemented with the syntax described below.

- Table 151 - Syntax of RendererAudioUtilizationInterface()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| RendererAudioUtilizationInterface |  |  |
| { |  |  |
| for (int i=0;i<activeMPHOASources.size();i++) { |  |  |
| audioStreamId; | 8..* | cstring |
| processing_level; | 8 | uimsbf |
| } |  |  |
| for (int j=0;j<notActiveSources.size();j++) { |  |  |
| audioStreamId; | 8..* | cstring |
| } |  |  |

activeMPHOASources.size()		shall indicate the number of active HOA sources utilized for 6DoF HOA rendering in the audio scene.

audioStreamId		shall indicate the identifier of the audio stream used in the audio scene.

processing_level		shall indicate the order of the HOA audio element audio data being processed in the renderer. The maximum value is equal to the maximum HOA order.

notActiveStreams.size()		shall indicate the number of audio streams that are not being utilized for rendering the audio scene. This excludes audio streams related to 6DoF HOA rendering.

(normative)

Panner default output positions

OutputDirection = {{90, 0},

{-90, 0},

{114, 20},

{-60, -6},

{85, -44},

{-130, -21},

{49, 41},

{-67, 52},

{154, -9},

{-48, -55},

{19, -31},

{-162, 21},

{151, 73},

{-9, 20},

{180, -63 }};

RIR analysis: loudspeaker source directivity factor

In the determination of room acoustic parameters for reverb rendering, the reverberant-to-direct energy ratio is, when applicable, obtained by analysis a room impulse response.  A loudspeaker directivity factor is optionally applied to this ratio, in the case that the measurement was made using a non-omnidirectional loudspeaker. The directivity factor used shall be:

srcLSDirectivity[8] = { 1.1216f, 1.8731f, 3.6854f, 5.4908f, 5.1385f, 6.8701f, 5.1735f, 6.3951f };

The band-wise directivity factor has been calculated by

DFf=4πprmsΩpeak,f2ΩprmsΩ,f2​,

that is, the energy in the direction of the peak directivity (on-axis, in the case of a loudspeaker), divided by the mean of the energy integrated over all directions. In practice, this was done using a discrete sampling of the tenth-order spherical harmonic representation of the measured directivity of a Genelec 8351A loudspeaker, provided by the Source Directivity Toolkit [20].

Default acoustic environment presets

Table 152 - Default AE preset 0

| ID | 0 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.154 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 8.78 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.34 | 0.0000011143567903 |
| 63 | 0.37 | 0.0000013400360494 |
| 80 | 0.4 | 0.0000014650820423 |
| 100 | 0.43 | 0.0000017083103813 |
| 125 | 0.48 | 0.0000017244643093 |
| 160 | 0.49 | 0.0000017406182374 |
| 200 | 0.5 | 0.0000017567721650 |
| 250 | 0.51 | 0.0000017729260930 |
| 315 | 0.52 | 0.0000017890800211 |
| 400 | 0.54 | 0.0000018052339487 |
| 500 | 0.57 | 0.0000018213878767 |
| 630 | 0.56 | 0.0000018375418048 |
| 800 | 0.56 | 0.0000017422179767 |
| 1000 | 0.55 | 0.0000016637636401 |
| 1250 | 0.52 | 0.0000014537780316 |
| 1600 | 0.48 | 0.0000012278621924 |
| 2000 | 0.44 | 0.0000009515894828 |
| 2500 | 0.42 | 0.0000007297347767 |
| 3150 | 0.40 | 0.0000005045510506 |
| 4000 | 0.37 | 0.0000002764094252 |
| 5000 | 0.35 | 0.0000001299161398 |

Table 153 - Default AE preset 1

| ID | 1 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.300 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 10.0 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.51 | 0.0000011143567903 |
| 63 | 0.555 | 0.0000013400360494 |
| 80 | 0.6 | 0.0000014650820423 |
| 100 | 0.645 | 0.0000017083103813 |
| 125 | 0.72 | 0.0000017244643093 |
| 160 | 0.735 | 0.0000017406182374 |
| 200 | 0.75 | 0.0000017567721650 |
| 250 | 0.765 | 0.0000017729260930 |
| 315 | 0.78 | 0.0000017890800211 |
| 400 | 0.81 | 0.0000018052339487 |
| 500 | 0.855 | 0.0000018213878767 |
| 630 | 0.84 | 0.0000018375418048 |
| 800 | 0.84 | 0.0000017422179767 |
| 1000 | 0.825 | 0.0000016637636401 |
| 1250 | 0.78 | 0.0000014537780316 |
| 1600 | 0.72 | 0.0000012278621924 |
| 2000 | 0.66 | 0.0000009515894828 |
| 2500 | 0.63 | 0.0000007297347767 |
| 3150 | 0.6 | 0.0000005045510506 |
| 4000 | 0.555 | 0.0000002764094252 |
| 5000 | 0.525 | 0.0000001299161398 |

Table 154 - Default AE preset 2

| ID | 2 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.300 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 10.0 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.51 | 0.0000011143567903 |
| 63 | 0.555 | 0.0000013400360494 |
| 80 | 0.6 | 0.0000014650820423 |
| 100 | 0.645 | 0.0000017083103813 |
| 125 | 0.72 | 0.0000017244643093 |
| 160 | 0.735 | 0.0000017406182374 |
| 200 | 0.75 | 0.0000017567721650 |
| 250 | 0.765 | 0.0000017729260930 |
| 315 | 0.78 | 0.0000017890800211 |
| 400 | 0.81 | 0.0000018052339487 |
| 500 | 0.855 | 0.0000015373515425 |
| 630 | 0.84 | 0.0000012872756162 |
| 800 | 0.84 | 0.0000009921263488 |
| 1000 | 0.825 | 0.0000007500468282 |
| 1250 | 0.78 | 0.0000005009678675 |
| 1600 | 0.72 | 0.0000003849552577 |
| 2000 | 0.66 | 0.0000003225158752 |
| 2500 | 0.63 | 0.0000002567920419 |
| 3150 | 0.6 | 0.0000001778239684 |
| 4000 | 0.555 | 0.0000000941312271 |
| 5000 | 0.525 | 0.0000000410830907 |

Table 155 - Default AE preset 3

| ID | 3 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.300 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 10.0 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.51 | 0.0000011143567903 |
| 63 | 0.555 | 0.0000013400360494 |
| 80 | 0.6 | 0.0000014650820423 |
| 100 | 0.645 | 0.0000017083103813 |
| 125 | 0.72 | 0.0000017244643093 |
| 160 | 0.735 | 0.0000017406182374 |
| 200 | 0.75 | 0.0000017567721650 |
| 250 | 0.765 | 0.0000017729260930 |
| 315 | 0.78 | 0.0000017890800211 |
| 400 | 0.81 | 0.0000018052339487 |
| 500 | 0.855 | 0.0000023973612960 |
| 630 | 0.84 | 0.0000030719466589 |
| 800 | 0.84 | 0.0000036005124733 |
| 1000 | 0.825 | 0.0000041607332893 |
| 1250 | 0.78 | 0.0000043239432477 |
| 1600 | 0.72 | 0.0000040140488246 |
| 2000 | 0.66 | 0.0000031653604276 |
| 2500 | 0.63 | 0.0000024404777313 |
| 3150 | 0.6 | 0.0000016766047356 |
| 4000 | 0.555 | 0.0000009017242929 |
| 5000 | 0.525 | 0.0000004108309067 |

Table 156 - Default AE preset 4

| ID | 4 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.077 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 8.78 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.17 | 0.0000011143567903 |
| 63 | 0.185 | 0.0000013400360494 |
| 80 | 0.2 | 0.0000014650820423 |
| 100 | 0.215 | 0.0000017083103813 |
| 125 | 0.24 | 0.0000017244643093 |
| 160 | 0.245 | 0.0000017406182374 |
| 200 | 0.25 | 0.0000017567721650 |
| 250 | 0.255 | 0.0000017729260930 |
| 315 | 0.26 | 0.0000017890800211 |
| 400 | 0.27 | 0.0000018052339487 |
| 500 | 0.285 | 0.0000018213878767 |
| 630 | 0.28 | 0.0000018375418048 |
| 800 | 0.28 | 0.0000017422179767 |
| 1000 | 0.275 | 0.0000016637636401 |
| 1250 | 0.26 | 0.0000014537780316 |
| 1600 | 0.24 | 0.0000012278621924 |
| 2000 | 0.22 | 0.0000009515894828 |
| 2500 | 0.21 | 0.0000007297347767 |
| 3150 | 0.2 | 0.0000005045510506 |
| 4000 | 0.185 | 0.0000002764094252 |
| 5000 | 0.175 | 0.0000001299161398 |

Table 157 - Default AE preset 5

| ID | 5 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.077 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 8.78 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.17 | 0.0000011143567903 |
| 63 | 0.185 | 0.0000013400360494 |
| 80 | 0.2 | 0.0000014650820423 |
| 100 | 0.215 | 0.0000017083103813 |
| 125 | 0.24 | 0.0000017244643093 |
| 160 | 0.245 | 0.0000017406182374 |
| 200 | 0.25 | 0.0000017567721650 |
| 250 | 0.255 | 0.0000017729260930 |
| 315 | 0.26 | 0.0000017890800211 |
| 400 | 0.27 | 0.0000018052339487 |
| 500 | 0.285 | 0.0000015373515425 |
| 630 | 0.28 | 0.0000012872756162 |
| 800 | 0.28 | 0.0000009921263488 |
| 1000 | 0.275 | 0.0000007500468282 |
| 1250 | 0.26 | 0.0000005009678675 |
| 1600 | 0.24 | 0.0000003849552577 |
| 2000 | 0.22 | 0.0000003225158752 |
| 2500 | 0.21 | 0.0000002567920419 |
| 3150 | 0.2 | 0.0000001778239684 |
| 4000 | 0.185 | 0.0000000941312271 |
| 5000 | 0.175 | 0.0000000410830907 |

Table 158 - Default AE preset 6

| ID | 6 |  |
| --- | --- | --- |
| presetDefaultAEPredelay | 0.077 |  |
| presetDefaultAENumDelayLines | 15 |  |
| presetDefaultAEDelayRatio | 8.78 |  |
| presetDefaultAEDelayMinMs | 38.19 |  |
| presetDefaultAEGain | 1.0 |  |
| presetDefaultAEDistanceGainDropDb | 6.0 |  |
| presetDefaultAEMinimumDistance | 5.0 |  |
| Frequency | RT60 | DSR |
| 50 | 0.17 | 0.0000011143567903 |
| 63 | 0.185 | 0.0000013400360494 |
| 80 | 0.2 | 0.0000014650820423 |
| 100 | 0.215 | 0.0000017083103813 |
| 125 | 0.24 | 0.0000017244643093 |
| 160 | 0.245 | 0.0000017406182374 |
| 200 | 0.25 | 0.0000017567721650 |
| 250 | 0.255 | 0.0000017729260930 |
| 315 | 0.26 | 0.0000017890800211 |
| 400 | 0.27 | 0.0000018052339487 |
| 500 | 0.285 | 0.0000023973612960 |
| 630 | 0.28 | 0.0000030719466589 |
| 800 | 0.28 | 0.0000036005124733 |
| 1000 | 0.275 | 0.0000041607332893 |
| 1250 | 0.26 | 0.0000043239432477 |
| 1600 | 0.24 | 0.0000040140488246 |
| 2000 | 0.22 | 0.0000031653604276 |
| 2500 | 0.21 | 0.0000024404777313 |
| 3150 | 0.2 | 0.0000016766047356 |
| 4000 | 0.185 | 0.0000009017242929 |
| 5000 | 0.175 | 0.0000004108309067 |

VR filter design initialization vector

XSTART = {	-0.203920067266352, 0.128787908879483, 0.079837810189545, 0.099929196624988,

-0.234180882464493, -0.215314119358636, -0.246411794226213, -0.172436052311266,

-0.355835699346580, -0.334387138798732, 0.463930529067942, 0.460226715285694,

-0.311585344404065, -0.475693438370513, -0.295444453620049, 0.199843614126557,

0.279514585555530, 0.265907856480316, 0.018417987872943, -0.203199498423778,

-0.312278771338748, -0.419258731235125, 0.238440296198970, 0.265907856480316,

0.018417987872943, -0.278006828910261, 0.370732306177376, -0.293280844660574,

0.418610907937922, -0.011588811205171, 0.111743862902646, 0.265907856480316,

0.018417987872943, -0.203199498423778, -0.312278771338748, -0.419258731235125,

0.238440296198970, -0.058690777104047, -0.341690132287349, 0.379937031201279,

-0.225913538007775, -0.085764980918949, -0.203920067266352, 0.128787908879483,

0.079837810189545, 0.099929196624988, -0.234180882464493, -0.215314119358636,

-0.246411794226213, -0.172436052311266, -0.355835699346580, -0.334387138798732,

0.463930529067942, 0.460226715285694, -0.311585344404065, -0.475693438370513,

-0.295444453620049, 0.199843614126557, 0.279514585555530, -0.477066907560919,

0.077662858129756}

Octave band neural network parameters

theta1 = {8.359878713865152e-04,8.547322220158112e-05,-1.658299472673640e-03,-8.897020573858766e-04,1.374055647247520e-03,9.828561674148563e-04,1.223515725462025e-03,-1.352706791125740e-04,2.724056744377736e-04,1.288206991958871e-03,9.499631219219911e-04,-2.469974848592716e-04,-1.141580875010866e-03,-1.295112494331233e-03,-5.978261550422934e-04,-3.034776890279737e-05,-8.632207571268030e-04,1.373229561220072e-04,6.262151869926911e-04,-6.840537319682704e-04};

theta2 = {3.552895209336471e-02,-3.683101094544935e-02,1.544195828729876e-02,1.407231881609986e-02,-1.168004730233060e-03,-1.845297313636336e-02,-1.515047750276116e-02,-2.149486868001570e-02,6.112918761945306e-03,2.420529174998849e-02};

xMin = {-12,-12,-12,-12,-12,-12,-12,-12,-12,-12};

xMax = {12,12,12,12,12,12,12,12,12,12};

tMin = {-1.743401668682568e+01,-2.131708842485039e+01,-2.170441288042948e+01,-2.154042529662784e+01,-2.143074691592492e+01,-2.175557419031379e+01,-2.039338478527605e+01,-1.946007373459756e+01,-1.991523334344047e+01,-1.494945109544445e+01};

tMax = {1.645155259154675e+01,2.288543627872786e+01,2.112323652545153e+01,2.093612436245674e+01,2.143413645157312e+01,2.257964735584436e+01,2.112400872713990e+01,2.034775555368313e+01,1.955805350970688e+01,1.463274290482193e+01};

W1[0] = {-1.249845103906119e-01,5.266457386598127e-01,-1.265557143907060e-01,7.551067725628659e-03,-3.017663214061779e-03,3.963342872930754e-04,2.350432300903882e-03,-1.120885209993510e-03,9.247806821622239e-04,-1.447298285062381e-03};

W1[1] = {-5.544670077519056e-04,2.386676094659368e-04,7.374074945967561e-03,-8.806089429944368e-02,3.741833390649099e-01,-8.938358102566210e-02,6.681568818197932e-03,-2.510890468796177e-03,-6.534703146453762e-04,2.626359751386651e-03};

W1[2] = {5.820373666894878e-01,-1.608550923629943e-01,6.843832526723191e-03,-1.147557131961238e-04,-2.029196670649964e-03,-6.170282969242773e-04,-3.427656648159464e-04,7.664552001633552e-04,-1.225923211224387e-03,-2.340092819962502e-04};

W1[3] = {6.560423380746320e-03,-8.843793100057799e-02,3.687100918092583e-01,-8.851080892205522e-02,9.700292730847760e-03,-3.415750124449459e-03,-9.975800795366122e-04,8.245389408965012e-04,1.477817808765500e-03,3.866488408599131e-04};

W1[4] = {2.293222066592418e-03,-2.708853860889269e-03,-4.324940634069773e-03,-1.142052071251931e-03,5.690213657432463e-03,-9.549478554529714e-03,1.369666644646367e-01,-5.335488374328827e-01,1.241760503804226e-01,5.764867698586685e-03};

W1[5] = {2.112360427730359e-04,-5.081608424528544e-03,-2.770039475388752e-03,2.815059118873478e-03,-2.972762742112710e-03,4.862962280744462e-04,7.359230380200678e-03,-1.151007460191757e-01,5.304724389763745e-01,-9.611719995014199e-02};

W1[6] = {3.359030950933401e-04,1.795270925309229e-04,-4.630404786345881e-04,-2.231517009672016e-03,6.034059076361724e-05,-3.750670884220089e-04,7.573908711449153e-03,-2.270169283055933e-03,-4.480777658373232e-03,-1.052137767662975e-01};

W1[7] = {2.135195640000022e-03,-4.352595982285545e-03,8.777941272376243e-02,-3.682332090459548e-01,8.726269260907039e-02,-2.419028418929964e-03,1.484817060572245e-03,-1.316126408435669e-03,1.089476547538634e-03,-1.625586144777415e-03};

W1[8] = {-4.655428637618245e-03,2.301095736956214e-03,2.697953667631031e-03,-7.489187804290042e-03,9.281173974793475e-02,-3.734112224540869e-01,9.364968575239353e-02,-9.700398498820314e-03,2.711850819791076e-03,5.642200810607369e-04};

W1[9] = {2.274523371536748e-03,-2.540339671592814e-03,-4.013987379835674e-03,-1.119091211970584e-03,5.046791835568902e-03,-7.173193757816045e-03,8.353800928845698e-02,-3.443838571658540e-01,7.446937422591765e-02,5.086483671991818e-03};

W1[10] = {2.842897102199699e-04,-4.701889892732403e-03,-2.554084620367180e-03,2.533365854631808e-03,-2.537052600995105e-03,3.429725026792958e-04,5.351075646560870e-03,-6.038078410518619e-02,3.181793876435340e-01,-4.758646349688016e-02};

W1[11] = {4.816803037359867e-03,-2.317635526578950e-03,-2.828497012243485e-03,8.740315836120593e-03,-1.320999680303542e-01,5.146332879978940e-01,-1.340291964424077e-01,1.130071596372745e-02,-2.620224526789094e-03,-7.186667907996960e-04};

W1[12] = {-1.776615379448316e-04,9.280153737536673e-04,9.233628900040926e-04,1.748818662140622e-03,-1.302533374053542e-05,-8.788171563248662e-04,-4.865567076036258e-03,3.450087387685173e-03,-3.439936090368157e-02,3.213260968713526e-01};

W1[13] = {1.650672570469666e-01,-3.193342430828329e-02,2.398542683578112e-03,-1.056700526743423e-03,-1.688542475643315e-03,-4.642678514278148e-04,-1.044936608882553e-03,4.616220830725696e-04,-1.094462270390874e-03,2.842146381543858e-04};

W1[14] = {7.491421600958060e-04,1.374934358910792e-03,2.692669170432229e-04,-2.719933276471133e-03,1.028161677012390e-02,-9.146118155485616e-02,3.823733463102059e-01,-8.654510766669690e-02,1.846851677706178e-03,1.568221048608851e-03};

W1[15] = {-5.287532964048631e-04,3.176370773997585e-04,8.806971895951808e-03,-1.270908245847784e-01,5.144507101357128e-01,-1.284011983125171e-01,7.910161590454733e-03,-2.480879638715174e-03,-7.395168726949991e-04,2.654398701880602e-03};

W1[16] = {7.926515279394603e-03,-1.277755610042752e-01,5.101376102793813e-01,-1.276976541024332e-01,1.113533818189427e-02,-3.377627283699616e-03,-1.073939905372327e-03,7.641125391644651e-04,1.607138622669868e-03,4.269766151509610e-04};

W1[17] = {-2.105659856222192e-03,5.767043661084203e-03,-1.298122219259415e-01,5.196069503289698e-01,-1.293946470960059e-01,3.751578037219106e-03,-1.529654016436069e-03,1.468910246911912e-03,-1.216170718616350e-03,1.604304656603586e-03};

W1[18] = {-7.192943159633246e-04,-1.392238904331236e-03,-3.060988278888274e-04,2.710021057642362e-03,-1.180104278128020e-02,1.293292606421065e-01,-5.186659128385702e-01,1.237835553214401e-01,-2.718528832983392e-03,-1.872207082265746e-03};

W1[19] = {8.292710864461444e-02,-3.709472738793035e-01,8.359623308008396e-02,-5.999503529834692e-03,2.942383423982314e-03,-3.435960574891508e-04,-2.207387601895270e-03,1.001058654549236e-03,-9.108468781068886e-04,1.328705658582639e-03};

W2[0] = {-5.362654975811766e-02,2.610775998229408e-03,-3.479077737471670e-01,-1.661135882441614e+00,2.713084866151550e-03,-8.050026049203143e-03,4.912538454850060e-02,-2.501448126131754e-01,-5.325713266010247e-02,-5.413908591370461e-03,3.710196544179083e-02,-2.786643987916668e-02,1.291192419843890e-02,5.760907986781017e+00,3.450124140329197e-02,2.901909202852385e-02,1.201224336922085e+00,-1.654040699450098e-01,1.180257790613339e-02,2.074188949139317e-01};

W2[1] = {-2.299910666241552e+00,2.241407896564608e-01,-1.612962073878989e-01,9.917453965403393e-02,-2.645343263476570e-03,5.059318927986218e-04,-9.292925039466506e-02,1.159873518382173e+00,6.965488000928281e-02,3.578808175485068e-04,-1.130289625174389e-02,5.110609921296610e-02,-2.127117369872963e-02,2.284871198024709e-01,-2.195523448724401e-02,-1.526760325057060e-01,-1.832418222221722e-01,8.243686413912834e-01,-5.650966004744622e-03,-4.906067294463353e+00};

W2[2] = {-2.193197430837718e-01,-1.328757646185728e+00,2.717121593585667e-01,5.632814204010336e+00,5.692102516170808e-03,9.381392418837359e-03,7.064777367275021e-02,-1.235552641578222e-01,-2.197092387822926e-01,1.590893342710247e-03,-2.865050000937937e-02,-1.488215489651381e-01,2.325055466603200e-02,-9.505819653202172e-01,-3.249844118684987e-02,9.526240505079028e-01,-2.819950745010270e+00,-1.730021936722720e-01,-3.344685950665918e-02,-2.147736614306238e-01};

W2[3] = {8.240792944325345e-01,1.577360767565291e-01,-5.393697413487785e-02,1.883639724555884e-01,-3.453412704425833e-02,-1.377844491567404e-02,-5.203891007499317e-02,-5.419161507390196e+00,1.203934866973946e+00,5.250906555082605e-02,2.500785261207901e-02,8.997071055035865e-01,-3.378219066361068e-02,2.275336320460611e-01,2.054144942567362e-01,-2.034756873216009e-01,-2.191798573484430e-01,-2.605485215656132e+00,1.430379858340748e-01,1.150949995301596e+00};

W2[4] = {-1.142380032568625e-01,5.616960061248720e+00,1.684543844756680e-02,-1.304362361658231e+00,1.031741991076227e-01,2.814507612175948e-02,5.156347938990066e-02,-9.674380034593323e-02,-1.517954269854813e-01,-1.805550304934425e-01,-3.276358432273232e-02,-2.011176244375439e-01,7.461889596709664e-05,-3.358905775648201e-02,-1.293902724098420e+00,-2.847799671683095e+00,9.367378985858679e-01,-1.633690988156630e-01,-9.557695155945171e-01,-1.536757144358032e-01};

W2[5] = {2.007418537812703e-02,2.072982516956859e-01,-4.700074331171047e-03,2.353212219992850e-01,-6.111302795932311e-01,-9.446945338284869e-02,1.656777325757606e-01,1.147555230491997e+00,-5.433297066306639e+00,9.619464721159764e-01,1.785536965834977e-01,-2.743369135443336e+00,5.639080131194836e-02,-6.070299979612768e-02,1.704989623172130e-01,-2.162351140811685e-01,-1.553973303904409e-01,8.183374144452195e-01,2.046902784756062e-01,6.202071772201182e-03};

W2[6] = {2.398920932390386e-03,-1.371771141598966e+00,1.856380627816486e-03,-7.310851057347635e-02,1.170690916300339e-01,5.246175381767495e-01,-6.166191181114852e-01,-1.984501842055680e-01,-1.357715760879329e-01,-3.407614516132427e-02,-8.807286747207850e-01,-1.900184519263931e-01,-1.969012773787292e-01,-2.408888266142615e-02,5.904526010284056e+00,9.810325612060302e-01,5.581177883990580e-02,-1.346649681395594e-01,3.088134054765971e+00,1.855485759596761e-02};

W2[7] = {2.513720681548717e-02,2.803739503250021e-01,-2.985060902592453e-03,-6.504641347102737e-02,2.057994625301883e+00,-1.741465186012960e-01,3.259423741754866e+00,4.516060220355166e-02,1.335998028487334e+00,-5.174738053393857e+00,3.016228178205144e-01,9.732488389831140e-01,1.132089307137149e+00,4.411002630315893e-02,-4.607927580199540e-02,-1.804349186132682e-01,1.864359981290027e-02,3.997508159779085e-02,8.558762630222690e-02,6.681380703501888e-02};

W2[8] = {-3.885378044986900e-02,-1.779770150904100e-02,-1.062362236612843e-03,6.260473998013071e-02,-1.299709727173198e-01,-1.500705760756160e+00,-6.900352211157339e-01,6.763970540966570e-03,-1.809529731329133e-01,3.851344880877086e-01,4.524687261307224e+00,-1.345165787228233e-01,-4.548577818170572e-01,1.733619361416238e-02,-1.228718360589634e+00,2.842739418104910e-02,-2.324280737010848e-02,-5.360357948595969e-03,-9.072437945140798e-01,-1.043669718123008e-01};

W2[9] = {5.276735230234735e-03,-3.657663579523172e-02,-3.955062357293170e-03,-7.066288236455442e-02,-5.188602668681692e-01,5.020777067414161e-01,-1.156262730123498e+01,1.498093672584307e-01,-2.008277460901300e-02,8.486154716430934e-01,-1.502103673069740e+00,1.330127172110584e-02,-1.210328746874824e+00,2.631913866536619e-02,6.777701792298022e-01,1.095176658127521e-02,2.855401754696564e-02,5.923450919222457e-02,3.421226069073460e-01,1.547672672505750e-02};

Third-octave band neural network parameters

xMin = {-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12,-12};

xMax = {12,12,12,12,12,12,12,12,12,12,12,12,12,12,12,12,12,12,12,12, 12,12,12,12,12,12,12,12,12,12,12};

tmin = { -2.038870608440465e+01, -2.678849815964725e+01, -2.757636053895062e+01, -2.765344282385239e+01, -2.716098081395197e+01, -2.581423496482778e+01, -2.723128674501405e+01, -2.763868838366484e+01, -2.556399567416048e+01, -2.697758941200400e+01, -2.724126535689017e+01, -2.744919796206791e+01, -2.726143977153325e+01, -2.776948602363049e+01, -2.607297788243555e+01, -2.707771393608530e+01, -2.641589721784473e+01, -2.727456177463340e+01, -2.593256090802651e+01, -2.636116803999377e+01, -2.724430835038541e+01, -2.683941356896038e+01, -2.713223493489686e+01, -2.757544478196990e+01, -2.557585962140894e+01, -2.619087878878048e+01, -2.566657946258719e+01, -2.667127036790179e+01, -2.423212002811150e+01, -2.313463264195814e+01, -1.440222109525858e+01 };

tmax = { 2.054556464737180e+01, 2.678849815952580e+01, 2.756207951039231e+01, 2.765344282376445e+01, 2.667128786973017e+01, 2.613574896932572e+01, 2.619253163804886e+01, 2.733638838582886e+01, 2.723129923844913e+01, 2.723521611285197e+01, 2.793883888941780e+01, 2.727641967392399e+01, 2.819185839678409e+01, 2.728774270391036e+01, 2.814981788805706e+01, 2.672289476327011e+01, 2.670769604382733e+01, 2.736021010100513e+01, 2.667094916920739e+01, 2.551439790715242e+01, 2.779977488507454e+01, 2.679395939402407e+01, 2.837862926608989e+01, 2.757544478197003e+01, 2.680643087380155e+01, 2.578710853870419e+01, 2.708257445180220e+01, 2.698100098770572e+01, 2.423212002811155e+01, 2.313463264195811e+01, 1.440222109525863e+01 };

W1 = { -1.205929942253572e-04, 7.970516355561727e-04, -2.783620092285350e-04, -1.994527534928567e-04, 2.347698439978771e-05, -3.398856773451466e-04, -1.081767877208057e-04, 9.573020933218775e-04, -2.188587324877659e-03, -1.219964682139673e-03, -2.909198620235967e-04, -9.693189427033699e-04, -1.725442934692025e-03, -1.732075723256216e-05, 2.441163606168298e-04, -1.724756448079270e-03, 4.904511943464306e-04, -7.521352565716084e-03, -1.004919694986394e-04, -1.985743001801228e-01, 5.972437269462961e-01, -1.991485417181242e-01, -4.167750123796576e-04, -8.090745796669799e-03, 7.741219308141721e-06, -2.811596778922621e-03, -1.134067915631453e-03, -1.717065905688127e-03, -3.945223811773435e-04, -4.592714525010132e-04, -1.594107676972680e-03, -1.568762859020788e-02, -7.655546781403515e-03, 1.616388768057758e-02, -9.198475375968596e-03, -5.228364384549117e-03, -2.324032792415502e-03, -7.302722261827847e-03, 2.822776635897114e-03, -5.516028267952844e-03, 1.454015343518510e-04, -1.467826249702701e-04, 8.929023381685540e-03, 1.591839086748880e-03, 3.635150415858306e-02, 5.573951393232023e-03, -2.090080963725512e-02, -1.924643726829652e-02, 6.645581883165793e-03, -3.918446446085644e-03, -2.941409180436321e-04, 1.205752538815127e-02, 5.532879751028764e-03, -7.869690805633306e-04, 2.549022366419519e-03, 5.891209968530791e-03, -4.600315159228235e-03, -2.313952100943919e-03, 7.588508399435905e-03, -1.296513252005192e-02, 1.525834633005370e-02, 1.631875137589078e-02, 8.973513744754169e-05, 3.278330202616116e-04, -9.779534047534134e-04, 1.221493998369634e-03, 7.294823867904890e-04, 5.640552177977216e-04, -6.037715592959158e-04, -7.155128612022828e-04, 6.396254473081331e-04, -1.900531728475317e-03, -8.751820044734212e-06, -1.600146129197874e-03, 5.963489921030400e-04, 3.106623796655930e-04, 1.412767385406773e-03, 3.625794700361803e-04, -6.307499604576117e-05, 8.158513556233214e-04, -9.435563540711524e-04, 7.394435167396343e-04, 5.788594173394652e-04, -2.176863080413200e-04, -1.178041946076603e-03, -1.905785973876941e-03, -6.146067379594580e-03, 1.257665239499912e-03, -1.846252236947820e-01, 5.881885558146267e-01, -2.005075579787388e-01, -2.289540939625320e-02, -7.728595525877748e-03, -6.408380044241126e-04, -5.574730257363119e-03, 1.368377887094464e-03, -1.362403525964883e-01, 4.203539405854665e-01, -1.376309041358541e-01, 7.927548330341569e-04, -4.384202804301316e-03, -1.709813709225018e-03, -1.973163989891247e-03, 1.172049423214363e-03, 1.919410312879963e-04, -3.745329655324365e-04, -3.989228446201443e-05, 1.354866822474912e-04, -8.363396087415978e-04, -1.891696196733436e-03, -2.104121664050531e-04, 8.733304722959609e-04, -9.028703227201802e-05, 4.102562071387589e-04, -1.762789638329752e-04, -7.298502758302870e-04, -5.291660379599583e-04, -4.912363715362130e-04, -6.461691936818310e-04, 7.060148833370427e-04, 1.346474889986398e-03, -7.116617672737618e-04, -4.883764275934055e-04, 7.071752440135739e-04, -7.692977569465908e-05, 2.712316220477503e-04, 3.233309633151900e-04, 1.438443823953110e-03, -1.059601453626433e-03, -7.339550363198612e-04, 8.930780837661272e-04, 6.162943205150872e-04, -1.294994723712162e-03, 7.298431968745137e-04, 1.508820167608550e-03, 1.523059703883000e-03, 7.474875881941269e-03, -1.049185268426369e-03, 2.120460132600882e-01, -6.396169958630962e-01, 2.132467414016447e-01, -1.413124914119697e-03, 7.561950387435764e-03, 1.157566489884644e-03, 7.017315923571403e-04, 7.849573995010162e-04, 7.359750125488732e-04, -3.367905757893708e-04, -1.817031677261977e-03, 1.335156442339239e-04, -9.367250957359649e-04, -1.669625729392768e-03, -4.227448411531152e-04, -3.052203703145366e-04, -1.043956292679833e-03, -2.918161066131680e-04, -2.118859729648476e-01, 6.323135709864747e-01, -2.108700089670788e-01, -3.630892524502000e-04, -7.282592503280884e-03, -1.721865402071791e-03, -1.639763803702128e-03, -9.194312745683670e-04, -2.665346625136815e-04, -3.149901068493578e-04, 8.374097779893163e-04, 6.587899549997006e-04, -1.861952947965133e-04, 2.426693953820317e-04, -1.966633747559766e-04, -3.416325747181439e-04, 5.714271035174355e-04, -1.004305427857754e-04, 1.809107182580013e-05, -1.287946971628885e-04, -3.100820235338318e-04, 2.179436256393108e-04, -5.810240261192121e-05, 6.804238337094855e-04, -4.565770897181996e-05, 9.713572910414674e-05, 2.505531269043026e-04, -4.022399107440849e-05, 3.184195021893900e-05, 1.194847959339187e-03, 1.716980514409185e-03, -1.524242800258861e-04, 1.428299115959252e-03, -9.490764413232124e-04, -3.407948750815434e-04, 9.844976058086348e-04, 1.178002673838288e-03, -4.947645932504606e-04, 1.019181799462759e-03, -8.498769924054603e-04, 6.531775856557540e-03, 2.845960259900397e-03, 2.046060824757393e-01, -6.184592513185315e-01, 2.052517278182078e-01, 9.420341381553312e-05, 7.011968791569160e-03, 1.402020870890799e-03, 2.855528395351592e-03, -9.449491592629735e-04, 7.357988294115545e-04, -1.602128607119915e-03, 1.434094755277669e-03, -5.666523335112608e-04, -8.654607197323709e-04, -1.578348325430839e-04, 1.283629815485372e-03, 2.852161054149409e-04, 5.518579196977798e-04, 2.857509705271505e-04, -4.913278741942027e-04, 1.033252993973679e-02, -1.582748746884082e-03, -5.878707398400568e-03, -1.161007038620093e-02, -1.820766494582588e-02, -4.135679462521438e-04, 7.029378872904266e-03, 1.189259322811567e-02, 8.865455764503897e-03, -4.850912246174177e-04, -1.337107634302752e-03, -8.355004763870051e-03, -7.339526006706313e-03, 6.938249100293261e-03, 1.569507600259194e-02, 1.352674839550099e-02, 1.473979781709554e-03, -1.416696312758943e-02, -9.369700623377690e-03, 4.548164194924955e-03, 1.379929137764035e-02, -7.181161776305900e-03, -1.861766231951567e-02, -8.216453879620384e-03, 8.735729395804859e-03, 7.046468172654663e-03, -4.110332386712828e-04, -1.340184324878251e-05, 1.810670942432183e-02, 7.941863674542100e-03, 4.631923825815392e-03, -1.173256204098135e-04, -6.286624104418205e-04, 8.976778859458485e-04, 1.415686477957975e-03, 6.833280511208166e-04, 2.023817832782682e-03, -1.784161480312963e-04, 3.733878271612483e-04, 9.540676561281649e-04, 1.912397842398795e-03, 7.764446104954814e-05, 6.499986198365895e-03, -1.544414239147099e-03, 1.413354192071221e-01, -4.302061392434766e-01, 1.421884770313671e-01, 3.225049807935607e-04, 6.621603964540591e-03, 3.406690757100338e-03, 4.272460928819285e-04, 5.293128200136866e-04, -3.429258132673108e-04, 6.794866675584011e-04, -1.256328461259094e-03, 1.181525832529086e-03, 2.033114111835232e-03, 1.280202245736207e-03, -2.126436268768683e-05, 9.146567163296734e-04, 2.052060981702923e-03, -1.143663074249387e-04, -1.704706822545422e-03, -3.369739894918286e-03, 3.871708588033454e-03, 9.379879567434992e-04, 7.813709751397734e-03, 8.038460558584020e-03, 8.753804081416041e-03, 6.241023814381453e-03, -5.696232075750001e-03, 3.676044399248144e-03, -7.207726902930094e-03, -2.134096773315121e-02, -1.378701216651811e-02, 2.879548232547366e-02, 1.981817795439702e-02, -5.820467940472487e-03, -3.876290152932329e-03, -2.352023874204130e-03, -9.273214391848464e-03, 1.105049845988865e-03, -1.828227455147122e-02, -4.205474296420244e-03, 3.653880338179398e-04, 5.511698564712350e-03, -8.608138064106844e-03, 3.132228428212637e-04, -1.178910610569719e-03, -2.691986597609076e-02, 5.795197484407657e-03, -1.909092698946819e-03, -6.362073366641480e-03, 1.461720440385010e-03, 3.783759020863821e-03, 6.709569754713596e-03, 1.086910223783803e-02, 6.882843426743077e-03, 5.986595031184421e-03, -5.525653292311737e-03, -1.518724635769004e-03, -8.069936367030125e-03, -6.435531808213756e-04, -1.720976691153141e-03, 1.062204927511355e-04, 1.536450288900096e-03, 1.054690273710682e-02, -3.516825106425279e-03, -1.665125795441684e-02, -1.136336253370159e-02, 1.458251624592730e-02, 1.295066857427773e-02, -7.405553402440230e-03, 2.975836605979203e-02, -3.416847035430732e-03, -2.883107082557634e-02, -2.730066442760833e-02, -2.615512503467679e-03, -1.585167710165159e-03, 8.110321210998937e-03, -2.463685094898996e-03, 2.470880630505847e-03, -4.182935380186519e-03, -7.495726602830129e-03, -8.921137644657143e-03, 8.188446224585551e-04, -2.139814588293731e-01, 6.368886492086376e-01, -2.083062123646365e-01, -6.955474777956520e-04, -7.744161939015611e-03, 3.944579271963477e-04, -2.105032499894958e-03, 1.045885802681324e-03, -2.706548034595130e-04, -1.347690202044176e-03, -9.362605243153012e-04, 4.141362754659712e-04, 1.322056987892183e-03, -5.770620065164828e-04, 1.131361252738295e-03, 8.215618036104647e-04, -5.212376233638505e-04, 3.616744653908494e-04, -1.950890152234336e-04, 1.334995023557478e-04, 6.982448365312121e-04, 1.178161985947194e-03, 3.182713834558349e-04, -3.373466052446516e-04, -6.174064474555755e-05, -1.233636560374716e-03, 5.779564387926015e-04, -5.975641114332172e-04, 1.153494885540592e-03, 1.245140852201173e-02, -1.825410310522547e-03, -8.547134122212242e-03, 6.756132063974957e-03, 1.567887323307659e-03, -1.226228745670554e-04, -1.169643763081018e-04, -1.159713652988344e-03, -5.523972885774124e-03, 2.176489074029391e-03, 2.662417399601825e-03, -5.796396837864635e-04, 3.472234898899790e-03, -1.244395292859288e-02, 1.304871477305598e-03, 1.830367265498466e-02, 4.838817936583372e-04, -1.028698828958755e-02, -1.138858085992475e-02, -4.862903048205548e-03, 3.320674709685174e-02, 1.516410485819169e-02, -6.114234036133497e-03, -8.922945913166745e-04, -4.553942790465548e-03, 1.038387500373919e-02, -3.725417525459404e-04, -1.584846496882981e-02, -1.073376091536490e-02, 7.885357662911411e-03, 4.261361970394560e-03, 1.120167471572489e-04, 8.501599938348930e-04, 1.640704875456846e-04, 1.460775037120362e-03, -4.729712087576206e-04, 4.520215013752574e-03, -1.458228787629140e-03, 1.064849405314778e-01, -3.180274432829032e-01, 1.010320360177337e-01, -6.681813736745748e-04, 5.039020667648949e-03, 8.782977332132542e-04, -1.582465377790786e-04, -2.480045424014591e-04, 5.073129035172784e-04, 2.825276960782949e-04, -5.016606524606415e-04, 1.141120908866179e-03, 6.120963092410867e-04, 3.719941789940389e-04, 4.560045518840887e-04, 6.348876299204420e-04, -7.547255362942523e-04, -3.982895078585661e-04, 8.447871909578030e-05, 8.557940308630791e-04, 4.843790122380318e-04, -4.211190770511939e-04, 1.492665277429930e-03, 3.274578376961667e-04, -1.135484000471029e-03, 1.173473985586520e-04, -3.438762376003063e-04, -5.162369517532857e-04, 2.700314611683501e-05, -1.084479339632070e-04, 6.959784378975992e-05, 2.222899462102687e-04, 7.911063231125573e-04, -1.760128810891601e-03, -1.381002109387806e-04, -3.951358655137245e-04, -1.496712043017055e-03, -1.469716167035907e-03, -2.528645886432067e-03, -1.716539659878928e-03, -2.951920410249026e-04, -1.866660146836168e-03, -5.953002380367454e-03, -1.581736652282856e-04, -1.830108514598418e-01, 5.491118781464548e-01, -1.815182145679226e-01, 2.750053418321319e-04, -6.740429107138318e-03, -1.246372055965038e-03, -2.885645395519562e-03, 7.453518434631589e-05, 1.525478852023182e-04, -6.409249765452154e-04, 5.895118917339620e-04, -1.633808076345638e-03, -1.062763169757799e-04, -1.349734551790701e-03, -1.065137659329400e-03, -6.091562804595506e-04, -1.153633879981871e-03, -1.593345846875010e-03, -1.790641698399679e-03, -5.037585297439279e-03, -2.604727783214061e-03, -1.769054288769868e-01, 5.380370478580637e-01, -1.777883478509697e-01, -6.853094749143336e-04, -6.674299388287991e-03, -1.541235013400293e-04, -1.133724864499650e-03, -8.250039848778137e-04, -9.818185578277482e-04, 7.074348611755422e-04, -1.921890771021504e-04, 3.019886308672291e-04, 1.168702900721673e-05, -7.194625611058955e-04, 6.674516307787905e-04, 9.948731780385048e-04, -1.764102962345309e-03, 1.192846442429838e-03, -8.381154967837844e-04, 2.374084566831046e-03, 9.658111968827930e-04, -4.244822549428388e-04, 2.932694183550845e-04, 5.822886209558083e-05, -1.350659226120655e-03, -2.011805210914866e-04, -4.648923050335075e-04, -4.045328879118072e-04, -5.619402627822368e-04, -1.482953940735060e-03, 9.708544346728656e-04, 1.332836561508970e-03, -2.436104741614877e-04, 6.560874300091992e-04, 1.815410644711236e-03, -1.007697731541112e-03, -1.412027555140891e-03, 6.500441899046277e-04, 2.323945148458794e-04, 1.010216157306426e-03, 2.375153676742893e-04, -2.109232734667971e-04, 9.911153365454731e-03, 3.033688190834930e-04, 2.197123716417126e-01, -6.539746565883310e-01, 2.172243234002909e-01, 5.827941110599238e-03, 9.584903141886857e-03, 4.260806377586678e-03, 3.645229041606204e-03, 2.697802255003974e-03, -6.134065507646872e-01, 2.324895114011212e-01, 6.196761033929029e-03, 1.072880963629633e-02, 1.963411728041746e-03, 2.877421488232373e-03, 1.591424449903842e-03, 5.324946778989056e-04, 2.382819892904654e-03, 3.067840828444496e-05, 2.356149734974071e-03, 2.895894359126145e-03, 1.201763734040750e-03, -2.653250813036878e-03, -5.168419545790539e-04, 3.250860797151295e-03, -2.540529557694329e-05, -2.160552849346083e-03, -1.254881632648256e-03, 8.259505293931080e-04, -1.714925985192704e-03, -1.669692873714589e-03, 8.590377923319817e-05, -1.900603781270611e-03, 2.852196292676296e-03, 1.547628722432932e-03, 4.025420775254016e-04, -1.779699829505146e-03, -6.692794330933704e-04, -1.290665299513319e-03, 6.551655635169016e-04, 4.365499101798426e-03, -7.485392129069330e-03, 2.394364574666206e-02, -4.241600065612753e-02, -1.363554904282819e-02, 1.383920616446533e-02, 1.600890197353426e-02, 1.010697151820537e-03, 8.338929721304796e-04, 9.087915333861924e-04, -2.602879212386267e-03, -2.270843780461978e-03, 2.664824057091839e-03, 4.916162618467487e-03, 2.426056185370045e-03, 5.598692897998938e-03, -5.773459122175598e-03, -1.225605542132731e-03, -7.528469705064276e-03, 1.710254670178232e-03, 3.649315242563260e-03, 8.369732259089440e-03, 8.071806778529084e-03, -3.434856626767854e-03, -1.191075315043317e-02, -3.815844049364431e-03, 3.339268187374930e-03, 8.723342411982259e-03, -1.113277874715978e-02, -1.407211796944454e-02, -8.376866942629898e-03, 1.188934466762065e-01, -3.498332865329343e-01, 1.074793654408413e-01, 7.221872802390850e-03, 6.931840048072394e-03, 1.847541342846878e-03, 1.715798563273908e-03, -2.115112935522167e-03, 1.084513041813532e-03, -7.526572732207494e-04, 7.169501358316373e-05, 3.949769338523673e-03, -3.804549225754696e-04, 4.509586187112288e-04, 2.599796778093223e-03, -2.039711797673191e-04, -1.901046358015361e-03, 8.176325453906196e-04, 1.134927789625255e-03, -4.138935223436895e-04, 1.359854437645705e-03, -2.155073832791991e-03, -1.203071875046927e-03, -1.583606874655692e-03, -1.531571567453363e-03, 2.819675067760680e-03, -1.333372230469917e-03, 1.673515125439289e-03, 1.386956409219856e-03, 9.865144398755370e-04, 1.677688361122375e-03, 9.281732445020417e-07, -5.040714059979085e-04, 9.591370731338912e-04, 6.626694183987105e-04, 1.051561223719572e-04, 1.848257233511424e-03, 1.484083306791414e-03, 7.007104667083325e-03, 2.963719375722466e-04, 2.044855125997198e-01, -6.154975692299843e-01, 2.039887930779257e-01, 9.954526556672232e-04, 7.868108445752904e-03, -4.718512090607728e-04, 2.045926265220052e-03, 3.948762975286340e-04, -9.748925381388857e-04, -8.154987844291599e-04, 1.118614821388988e-03, -4.227817567045477e-04, 2.628079519348745e-03, -2.204783596063333e-03, 5.915766457255920e-04, 8.198006523790321e-04, -8.361962237553018e-05, -1.612376677235944e-03, 5.969407889992163e-04, 1.489791848131178e-03, -1.196733266301573e-04, -7.917527232609939e-04, 5.087507886318168e-05, 6.343220218950141e-04, -1.017892761208779e-03, -1.430289833662259e-03, -7.203705525814017e-04, -2.226004440425081e-03, 1.264534543682914e-04, -5.060550391179737e-04, -9.116706176544337e-04, -2.233620805886148e-03, -3.475132472235041e-04, -7.468834089400109e-03, 1.768244306380288e-03, -1.776635413434409e-01, 5.377617810012674e-01, -1.786335633092467e-01, -7.076134115380843e-04, -8.053407235625193e-03, -3.791169183394339e-03, -6.484133175363109e-04, -6.645748341567949e-04, 3.996403092414109e-04, -7.335431038101118e-04, 1.243402818554333e-03, -1.555898520241265e-03, -2.118133293227001e-03, -1.281643637670869e-03, 5.548890917379850e-05, -9.542042126200506e-04, -2.154474408865240e-03, -3.571712009126807e-05, 5.792846686437734e-04, -5.755378364577044e-04, 4.359096235331584e-04, -9.283584195415460e-05, -1.619748547150106e-03, 3.935870900902210e-04, 3.336432153637783e-05, -1.156130437812339e-05, 1.018519820938987e-03, -6.265503005916335e-04, 9.310500526255475e-04, -9.161782848086175e-04, 1.447045006158836e-04, -1.381077698654417e-03, 1.303015770717537e-05, -8.025544326681669e-04, -3.134407444841807e-04, -2.926969210298598e-04, 3.896132789495131e-04, -1.101314879271190e-03, -1.634652147073988e-03, -1.966217356108644e-03, -6.157345896865335e-03, -1.769913803633742e-05, -2.151854763592420e-01, 6.437484128258902e-01, -2.122843778469697e-01, -6.366780723067859e-03, -7.805272684703725e-03, -6.217908632189128e-03, -2.013732590878610e-03, -6.071601027764976e-03, -1.083482449024144e-03, 1.681844126080031e-03, 1.069619497692549e-02, 8.337974166687733e-03, -4.608911522293147e-03, 5.686829078230444e-03, 5.171997803220627e-03, -1.746395484134265e-03, -4.164572335727478e-03, -3.524030077842538e-04, 2.253943830535681e-03, 8.569116614266676e-03, -2.399781010095073e-03, 2.678106536895602e-04, 8.686992827682589e-04, -9.940903532305622e-03, -8.161577599497215e-03, -8.024939376369116e-03, 5.585101907747135e-03, 1.367093286969676e-02, 3.850309013981596e-03, -7.337583680502152e-03, 9.185798206672166e-03, 1.792671795557507e-02, -2.008605327396823e-03, -1.946215746733264e-02, -2.165453913638881e-02, 2.060459374643092e-02, -7.486994823535535e-03, -1.587094640833492e-02, -3.796644694061150e-02, 3.721266783658290e-02, -2.260735732675345e-02, 2.308578510525099e-02, -3.637392696186324e-02, 2.225247773305056e-02, -2.289287232562252e-02, 2.795642710194904e-02, -3.260563731721144e-02, 2.081575489983701e-02, -2.620875169221695e-02, 1.482766639620922e-02, -3.308913020174291e-02, 2.240876520097666e-02, -7.059269086391911e-03, 1.785584437991512e-02, -2.352150760522851e-02, 4.533474032736241e-02, -4.359097209096376e-02, 9.327179255010814e-03, -7.118006927764601e-03, 1.589322726267077e-02, -3.010732319861029e-02, 3.466730158542254e-02, -7.161669850664038e-03, 1.921633052068467e-02, -2.707879791806480e-02, 2.851050087331429e-02, -4.211425707114892e-02, 1.589564950577023e-02, 6.044793701566263e-04, 2.139116506548611e-03, 7.357070213513403e-03, 2.184411682630680e-02, -2.879185379134690e-03, -6.073372727192889e-03, 4.873325972915918e-03, -8.066081085044642e-03, 1.986064134336356e-03, -4.292848106600832e-03, 6.890509900332171e-03, 7.841603302306975e-03, -3.195021513193090e-03, -8.424163194771166e-03, -1.040197099163762e-02, -7.482978327805334e-03, -8.038434867165628e-03, 8.734385332940141e-03, 8.532927524766656e-03, 4.102779123760027e-03, -1.638583566812106e-02, -6.501650951098589e-03, -3.129471078783996e-03, -2.978059084860310e-03, 7.091120859670387e-03, 5.484365885284400e-03, 1.061678620753086e-02, -5.695552881685806e-03, -1.809935058976017e-02, 7.963838659595344e-03, 2.795081669607906e-02, 9.765437431680034e-03, 6.952661251293384e-04, -1.087633675069654e-03, -1.009699359643899e-03, -7.916268197374314e-04, -1.135043299150830e-03, -9.294192302520189e-04, -6.075879664675757e-04, 9.818700881030207e-04, 5.086850780715384e-05, 4.530051805593311e-04, 4.044922058568476e-04, 1.923320606003645e-03, 2.102441102756917e-04, 5.002314436693907e-03, 3.622006532378497e-04, 1.404281903270895e-01, -4.187952702227514e-01, 1.407265854257829e-01, -1.354956537308076e-03, 6.792042009337060e-03, -1.375730106727571e-04, -4.822719130176434e-05, 1.435439819248665e-03, 2.577307305984824e-04, 1.302449774340247e-03, -6.313537811373138e-04, -3.913511252361609e-04, 7.208980422754737e-04, -2.701710995498810e-04, 7.268836125768900e-04, 1.301949077324198e-04, -7.083415082528082e-04, 1.188262466641807e-03, 8.939648311886469e-04, 9.549361565321224e-04, 1.303540988180478e-03, 1.083335134162066e-03, 6.118546136072054e-04, -1.036488979112619e-03, -5.397949830775202e-05, -3.143201132170562e-04, -3.886255240255815e-04, -2.113381440682210e-03, -3.050080125414157e-04, -5.944742604972101e-03, -4.892903109537683e-04, -1.759432082932917e-01, 5.231553853176133e-01, -1.761868709852475e-01, 1.306901680925260e-03, -7.824758760096015e-03, -1.174712132600379e-04, -7.156668713422468e-06, -1.590500451271343e-03, -3.565452515669903e-04, -1.436076721410098e-03, 6.781972692767043e-04, 4.104578635433407e-04, -7.570695003580832e-04, 4.195313801590563e-04, -7.726329560240846e-04, -1.192928122241199e-04, -1.403495728122371e-03, -1.732679749510839e-04, -1.285111072948964e-03, -1.166587611653877e-03, -5.004179580709896e-04, -9.275988310722942e-04, -1.411309996980913e-03, -1.754124147019996e-03, -4.128314236776932e-03, -2.435659012764424e-03, -1.401098642161368e-01, 4.295683724653398e-01, -1.412790035645870e-01, -6.896222776439969e-04, -5.617104406060747e-03, 1.187955054158322e-04, -9.879379920408860e-04, -8.421660291935232e-04, -9.445023918981451e-04, 6.286822706723828e-04, -1.841157108190864e-04, 3.494976488774942e-04, -1.619044886522313e-04, -9.216344223207190e-04, 8.061909040436588e-04, 7.221624965596242e-04, -1.560973321764216e-03, 1.171420272351810e-03, -8.971549363556262e-04, 2.138061642377276e-03, 7.474442363392808e-04, -3.079137155726424e-03, -1.511341451479783e-03, 2.814333249625924e-03, 4.125009556751670e-03, 7.924769053207430e-03, -5.990772197522900e-04, -2.733021379925827e-03, -3.943791007948055e-04, -9.446677284394731e-04, 1.365183480451273e-03, 3.921176522800486e-03, 6.932810882716988e-03, -9.393342261075629e-04, 1.065905746962712e-03, 5.838981269942414e-03, -1.728970193943720e-03, 4.746069413974601e-03, 1.512026641589067e-02, -2.216205799448352e-02, -5.459598022130808e-02, 9.678843608702426e-03, 9.152634033213848e-03, -9.490775640329396e-04, 3.175486638018676e-03, -5.986218148376724e-03, -1.799483041269170e-04, 4.399006554673319e-03, 9.716009260238872e-03, 2.184740665706316e-03, -7.372227780482017e-03, -3.458870167006224e-03, -9.228993388255553e-04, -6.831203048106781e-03, 1.176887906055659e-03, -1.767583634125006e-01, 5.410295005447526e-01, -1.785971297895632e-01, 5.681859300407773e-04, -5.410885058350492e-03, -2.061931392000840e-03, -2.418612929540152e-03, 1.305618717958373e-03, 3.494141269330845e-05, -4.027786489901206e-04, -8.721189091814120e-05, 1.595744671458500e-04, -8.827583769714936e-04, -1.898463436899041e-03, -1.833088626690065e-04, 1.065403526027117e-03, -4.326071891886805e-05, 3.992095469618645e-04, -2.492498162491430e-04, -6.933371198699787e-04, -6.455651398375309e-04, -3.311011484321754e-04, -7.061866439229174e-04, 7.787712609079759e-04, 1.498260578075796e-03, -7.562332780513164e-04, -6.804691244370244e-04, 8.660628894524660e-04, -6.885712722668916e-04, -2.086135968390814e-03, -8.968914743669866e-04, 1.653295447117978e-03, 1.564075542872204e-03, -1.088323838355684e-03, -1.044365968111380e-03, 1.919461311833650e-04, 3.229765446443227e-04, 3.680501646819275e-04, -7.602207135223990e-04, -1.425014467781795e-03, 1.596549111095358e-03, -8.689025844979967e-04, -6.605593466482092e-04, 1.371691881229364e-03, 1.710732311587376e-03, 1.222826422841706e-03, 1.638923998693222e-03, 7.401755514230693e-04, 7.332232082445346e-03, -1.331290804138272e-03, 1.811499594567714e-01, -5.442369609430205e-01, 1.868267744616512e-01, 1.875916914580607e-03, 7.022795113539729e-03, 3.442811007446725e-04, 1.825387758627590e-03, 2.850026992719217e-03, 4.962664353970110e-04, -1.200346877956198e-03, 1.397298891900383e-03, 1.420758426055164e-03, 4.324980586460644e-03, -2.678394109209867e-04, 1.389989095993099e-01, -4.170047550547304e-01, 1.378659180573931e-01, 4.559570381748336e-04, 8.023202255144116e-03, -2.427410757788993e-05, 1.083949662504807e-04, 7.456387712706363e-04, 2.344369016932376e-04, 1.052917110571123e-03, 1.713429648594446e-03, 5.695655497021662e-04, 5.592249900824436e-04, -2.504437193819306e-04, -3.689245017153440e-04, 2.449733193690471e-04, 1.145672167482652e-03, 1.109703825583593e-03, -4.363737724276168e-04, 1.227723623160473e-03, -1.840020944232052e-03, 6.438860335761748e-04, 5.756151606856301e-04, 5.524765780736909e-04, -7.033939669362523e-04, 1.402637609666936e-04, -1.719176083181980e-02, -2.568343137839893e-03, 1.118314567323294e-02, 1.347721100472413e-03, -5.551380562662640e-03, -1.285594848031126e-02, 5.983357728632640e-03, 1.446708236709328e-02, 1.188905862915775e-02, -5.662183837628118e-03, -3.335864830899598e-03, -1.078273152045225e-03, 6.818165400191457e-03, -2.551771212597535e-03, -1.358013742209587e-03, 4.017045394019154e-03, 2.294953008769476e-03, -4.127497445548446e-04, 1.794330369481553e-04, 2.767447881980232e-03, 2.380593815092429e-03, -2.933452749717918e-03, 3.570043009599026e-03, -4.520924919473118e-03, -1.062180837143737e-02, 2.406455298954025e-02, 9.356112976297229e-03, -1.830629860907328e-02, -4.530635606856777e-04, -5.186284910012574e-03, -3.370594925195824e-03, 1.746684111374368e-01, -5.231217740798706e-01, 1.686116294086445e-01, 3.855148893420927e-03, 8.838683913619192e-03, 1.539989938568774e-03, 2.612597520425317e-03, -2.041557806903788e-03, 1.051639906639481e-03, -1.149068802772475e-03, -1.014468570251733e-04, 3.951183963955236e-03, -5.996381502580230e-04, 7.093880456741041e-04, 1.464276451525546e-03, -7.936462601776171e-04, -1.275492625259429e-03, 1.089985476691305e-03, 4.696502544684127e-04, -7.856927599540438e-04, 1.580393068777298e-03, -1.486304332710932e-03, -4.428834706572799e-04, -1.375502450084649e-03, -9.339942994817813e-04, 2.767328935826679e-03, -5.669133034051287e-04, 1.451796262419697e-03, 7.768051304484167e-04, 4.084579131702272e-04, 2.300512476210597e-03, 2.614794909245629e-04, 1.487134729409802e-03, 6.134021339969304e-04, 1.645953030931185e-03, -1.568255419947316e-04, 6.518541234151821e-03, -1.228830986737474e-04, 1.756809504790379e-01, -5.270139125317463e-01, 1.716290571141346e-01, -3.790040473963496e-04, 7.826694026361193e-03, 1.385969329399965e-03, 6.811048937961236e-06, -2.500004168371821e-04, 9.011865583944429e-04, 2.527642258680288e-04, -5.522811255088657e-04, 1.260551084977396e-03, 6.199046030158400e-04, 5.035972014031369e-04, 8.505239961814563e-04, 1.454715301650904e-03, -3.944700841924928e-04, -6.131049825251398e-05, 1.546165051687046e-04, 7.273291765896007e-04, 4.784033294016805e-04, 2.165925827577601e-04, 2.114267394247897e-03, -5.200960898702592e-05, 1.023836383592338e-03, -1.404034792957242e-03, 2.047800226879615e-03, -7.084112797893867e-04, 2.141034954374899e-03, 1.090118532552465e-03, 1.490443963686105e-03, 1.781145128467426e-03, -1.318573918163557e-03, 5.508939270046328e-04, -9.823718177747814e-04, 1.373881969318214e-03, 5.974313843582504e-04, 1.462330511593505e-03, 9.156724288426780e-04, 1.371367070156392e-03, 1.352696863752179e-03, -1.590156644825697e-05, -1.645034981786954e-03, -3.050749785103941e-04, -2.077983168856135e-03, -9.915664535518525e-04, -4.547100793545302e-04, -1.470531995365185e-03, -5.304386046087394e-04, -8.542529368303733e-04, 3.576832984109740e-03, -2.021667245474807e-01, 6.808748208127469e-01, -2.347024000854592e-01, -2.664758742658840e-02, 3.501890095166919e-03, 4.600067943666819e-03, -1.981277642621829e-03, -1.765797895639645e-03, 7.196401589450878e-03, 4.639143192944295e-03, -4.046248953894631e-03, -1.836833005375284e-02, -8.364428686579160e-03, 1.773046961273132e-02, 6.587114414982330e-03, -7.060947464966952e-03, -3.857063732810809e-02, 1.607555406131605e-02, 7.025782691693436e-04, -5.263762122426133e-03, -5.372545863388688e-03, -3.803537117868764e-03, -4.135749989674031e-03, 9.187508134752640e-03, 7.294676325327533e-03, -2.556047157559366e-04, -4.888838055492718e-03, 6.304425118680965e-04, 1.331910279404652e-02, 1.209445558203271e-02, 7.435981452053719e-03, 5.472637275507938e-03, -5.633637745748935e-03, -1.158968114908997e-02, -6.342499407956492e-03, 7.949147957501006e-04, -2.877733007426371e-04, -1.546307676748521e-03, 3.388713023209627e-04, 3.440741035015504e-05, 1.766288631922705e-04, -1.243002329894142e-03, -5.656591732266842e-04, -1.368626522081734e-03, 4.652369597892085e-04, 1.349697889370894e-04, -4.478252118358829e-05, 1.065264477698046e-04, -2.813700617523143e-04, -1.125393941118381e-03, -6.621489428739516e-03, -4.765012384847509e-05, -1.806048472563686e-01, 5.464996373169048e-01, -1.815519919856529e-01, 1.260940328930233e-03, -6.489999975951458e-03, -1.120709612401121e-03, -1.483833358967789e-03, -1.927758752225761e-03, -1.038745502352507e-03, -1.581723147350961e-03, -1.217385055825027e-03, -9.111742339799087e-04, -3.070340725633569e-04, -8.637410156212121e-04, -1.695995015270861e-03, 9.519104231699094e-04, -2.059615471251115e-03, 5.915891532932099e-04, -2.308458414810584e-03, -4.541217683992376e-04, -8.250768503279592e-03, 7.665943066929135e-05, -1.318702249905294e-01, 3.975961606420041e-01, -1.315438265269402e-01, -1.882253323583364e-03, -4.237868989408365e-03, -7.243469023240673e-04, -1.238922929215980e-03, -2.082461647522800e-03, -1.737151378375132e-03, -1.569594633717582e-03, -2.017003058992970e-03, 5.809363168311352e-04, -7.269408260372972e-05, 1.448872180255932e-03, -3.560587569266064e-03, 9.741545214621228e-04, 7.784238833611882e-04, -1.101314261165775e-03, -2.267714886699592e-03, 6.351231156482909e-04, -6.919454095912004e-04, 3.947449961828500e-04, -1.251500294667397e-03, -1.098707970656294e-03, -1.926457480493004e-04, -3.604272416172404e-04, 1.762722398398400e-03, -1.216885671548507e-03, -3.872024899614915e-05, -2.056412209980060e-03, -1.062375556420157e-03, -2.529451996761697e-04, -1.290198429348312e-03, 1.212425248102057e-03, 1.273471570512773e-03, 9.057492954734909e-04, 5.335875288217495e-04, -1.355009837361159e-03, -1.816560865991746e-03, 6.188190380768973e-04, 2.289119225101083e-03, 9.472609999107975e-04, -2.874440374975664e-04, 1.690840550704439e-03, 9.018450857137013e-04, -2.625142274010171e-04, -1.826515934061400e-03, 1.783757728175598e-03, -8.579428154271315e-04, -5.635210772818918e-03, 1.008646987858983e-02, -5.054602263689133e-02, 2.161125462180015e-01, -6.030510455473033e-02, -1.364444369707351e-03, 1.217285987752393e-04, -3.412810226492481e-04, 1.233184220268053e-03, -1.433479585673795e-03, 5.752580238534349e-04, -1.475483494977076e-03, -1.105730786379470e-03, -6.021354995627080e-04, -1.028884509304951e-03, 1.103165347302560e-03, 8.250420582537758e-04, 4.013288631211771e-04, 9.062667807292856e-05, -9.863850034514782e-04, -1.355692478732918e-03, 6.764156647092100e-04, 2.914182854880240e-03, 1.149935981891636e-03, 2.822287435554826e-04, 1.772550790694986e-03, 8.725950766677492e-04, 1.390970578469886e-04, -1.450441514404292e-03, 1.530274723637363e-03, -1.391130276101980e-03, -5.389365157430093e-03, 1.100941697841567e-02, -1.083585501557175e-01, 4.407417762799068e-01, -1.421064015687141e-01, 2.254629929215129e-05, 1.789788622380093e-03, 1.953900031647981e-03, 1.115683720563744e-04, 8.558983754461907e-03, -1.281838950324667e-03, 2.021150658090843e-01, -6.094328856294682e-01, 2.058713819752585e-01, -1.394738519761223e-03, 7.108651017671976e-03, 9.407484937316793e-04, 7.948349792417171e-04, 1.234751611779436e-05, -9.952906838432394e-04, 2.648181121854614e-04, 2.183392103620166e-04, -2.861455286599525e-05, 1.754402574125813e-03, -5.713371994681742e-04, 5.272484154583218e-04, -5.571147068215124e-04, 5.907557327764147e-04, -3.368444652445004e-05, 8.392390038469453e-04, 1.206928090838946e-03, -1.028937284411941e-03, -1.626787674121808e-03, 1.055295463466435e-03, 9.980616507825274e-05, -9.926766577495725e-04, -5.513372333306573e-03, -5.352812748450060e-03, -4.086894682372874e-03, 4.397737465299263e-03, 3.777809379186700e-03, 5.111672523704960e-03, -6.188697375865720e-04, 8.045009016520537e-03, -6.112091652117319e-03, 5.627379017752158e-03, 5.547406872291705e-02, 1.264122755536494e-02, -3.144434718763928e-02, 1.084490090572601e-02, -1.479113188301828e-03, 6.209666469100936e-04, -2.439938159427724e-03, 1.355216182498267e-03, 1.953349206692481e-03, -2.502152241771108e-03, -2.335559706823766e-03, -1.789798609061545e-03, 1.031295269831812e-03, -7.077239444736623e-04, -3.476842843817959e-03, -1.552994629607446e-03, -7.726545751601055e-05, -4.583178108818509e-03, 2.577173731205311e-03, 3.011960218298967e-03, -1.125925355987355e-03, -1.712037681430131e-03, 8.142912122656577e-04, -2.194682584761586e-03, 5.888677951361208e-04, -2.686102250284426e-03, -6.788399270176554e-04, -9.237937550720226e-03, 1.333665986314802e-04, -1.736593278166904e-01, 5.206232243168230e-01, -1.731486220055584e-01, -2.323725222756786e-03, -5.381743671590348e-03, -1.004352144992038e-03, -1.296405789414975e-03, -2.158545244048495e-03, -1.715609445999635e-03, -1.605466938203246e-03, -2.063749616899838e-03, 7.755121768928049e-04, -1.237283233907608e-04, 1.808665708325843e-03, -3.816638191009911e-03, 1.316736412138619e-03, 1.042783509523145e-03, -1.264635045084205e-03, -2.534164255644998e-03, 6.241377719708763e-04, -8.512717008426635e-04, 7.606906209291740e-04, -1.465193026494636e-03, 4.879500890616137e-03, -4.224793039205334e-03, -3.611919174510936e-03, -1.334424691544010e-02, -3.136891212701928e-03, 1.056854049949140e-02, 5.826517708444036e-03, -1.660720852680216e-04, -1.479588566878069e-03, -3.423156239960547e-03, 6.368405691106158e-03, 3.994614895516176e-03, 2.319188432798166e-04, 2.586700645532067e-03, 2.186588201945348e-03, -3.130631634073127e-03, 4.732878292809845e-03, 6.142496742579045e-03, 4.084470096289640e-03, -1.145907373503461e-02, -2.481312266154100e-03, -2.701089485354898e-02, -2.333380773491286e-02, 6.541457173202776e-03, 3.750033942810733e-02, -3.205014074150375e-03, 5.709034728991425e-03, -6.835070765568869e-03, -9.132334657169845e-03, -9.461639399368267e-03, -5.786885169063341e-03, 5.993187196107030e-04, -3.683223873139356e-04, -1.354927898279781e-03, 2.107837247979673e-04, -4.089413328906187e-05, 1.722615597672235e-04, -1.185218889835379e-03, -5.576857734265171e-04, -1.352895051877487e-03, 2.772072840118551e-04, 9.398188108593321e-05, -4.690328184758741e-05, 1.149188581037011e-04, -1.258575874123462e-04, -8.822932433727529e-04, -5.514209671553235e-03, 9.684470623487816e-05, -1.415425114443026e-01, 4.307235358566962e-01, -1.421281299403207e-01, 1.105298616893650e-03, -5.481673131580037e-03, -8.348711353556309e-04, -1.102595695133670e-03, -1.729890332329889e-03, -1.058515075345192e-03, -1.443699810033344e-03, -9.800110108100649e-04, -8.675040344196328e-04, -3.555020826428119e-04, -7.049804704547439e-04, 3.673936179763409e-04, -1.328134010444642e-03, -9.402804772348889e-04, -9.603002190979034e-04, -2.530470591200091e-03, -1.291756230702950e-04, 2.182012085451866e-04, -1.779403863609586e-03, -1.560624475487743e-03, -8.768684366624079e-03, -1.656360190933302e-03, -2.036721272584562e-01, 6.152198598836940e-01, -2.048611687308630e-01, -6.862236274566125e-04, -7.718511916134370e-03, -1.599348049616292e-03, -4.678765322659665e-04, -2.525030224936334e-03, -1.596708574656799e-03, -2.308411642514436e-04, -2.660899838481072e-04, 4.070136843167565e-04, 1.057665676547986e-03, -2.629761126434020e-04, -1.077033447926740e-03, 4.299526892069157e-04, 8.601798397112133e-04, 3.179075487970433e-04, 6.909444884017273e-04, -4.922242106939059e-04, -1.165270055126633e-03, -2.388576717872874e-03, -8.231246111123082e-03, 1.306942607656234e-03, -2.141010964717318e-01, 6.381940136912031e-01, -2.109995710419976e-01, -9.392449964080510e-04, -6.008346459019455e-03, -5.928274716716312e-04, -3.472178030329484e-03, -1.454521978108384e-03, 1.956440401853752e-04, -4.450296973143517e-04, -1.126925095565915e-03, -4.944433488525800e-04, 8.251973371485222e-05, -1.652716117252193e-03, -1.221759670917717e-03, 4.227139014612628e-04, 4.922399126399147e-04, -3.565265248610560e-04, -1.585902326744742e-04, 3.314341530648600e-04, -1.597169571332901e-03, -4.901617565812097e-05, -4.634257233832445e-04, 5.546793030950438e-04, 9.829402743210191e-04, 1.121850024093032e-03, 3.120876164936328e-04, -1.268080498528614e-02, -1.402969392254740e-02, 4.496212230293508e-03, -2.052287331799368e-02, -1.758183547148474e-03, 6.519633824934820e-03, 2.520729435438214e-02, 2.629645371677793e-04, 4.851704327775875e-03, 3.319762067407256e-03, -9.553437505979810e-03, -1.125935452886197e-02, -7.224681214435288e-03, -4.498638534076846e-03, -1.423557216629447e-02, -1.304886378687449e-02, 1.352163331115407e-02, 2.222769880108000e-02, 8.924500540074308e-03, -1.364406672175216e-02, 2.254453141894702e-02, 7.036787421695278e-03, -2.269159878914701e-03, 1.526041832722914e-03, 4.342229220938723e-03, -2.760925782649913e-03, -1.007650710327462e-02, -7.733926753164398e-03, 2.521389281897470e-03, -5.589858700964029e-04, 5.016990847922776e-04, 9.861805680412897e-04, -3.683526596843390e-04, 4.109076832974721e-04, -8.996993121067304e-04, -2.112348582217745e-03, 9.449490351098306e-05, -1.722620588716860e-04, 1.646513009240090e-03, -3.957355610677801e-04, 7.292223711300649e-04, -5.039396046853919e-04, 6.849591909449395e-04, 3.680311120008110e-04, 1.018242405632550e-03, -2.935891869924979e-04, 4.320661635757382e-04, -9.632400210220926e-05, 3.160780154513339e-03, 9.180788805743690e-04, 7.402590349743410e-03, -2.632515856382949e-03, 2.172646347360573e-01, -6.487213217763049e-01, 2.195520166935789e-01, 1.477355395944114e-03, 7.690370605478834e-03, 1.922060423924889e-03, 1.595502540327915e-03, 1.538785269029348e-03, -1.233285433924140e-03, 3.955256443281698e-05, -2.401871903047045e-02, 2.826205060857400e-03, 2.600415043070924e-02, 4.132683507010793e-03, 4.555684886814297e-03, -5.822948470113995e-03, -9.297032983259183e-03, -1.068129181278992e-03, 1.118608190029170e-03, -2.420201910019449e-04, -1.188712958609771e-03, -1.184494583881377e-02, -3.417030591693295e-03, -1.812341814622314e-04, 1.330534152511078e-02, 2.063240299502336e-02, 8.489496129673552e-03, -8.351443925890255e-03, -4.845413227332526e-03, 1.787323115396758e-03, 8.960857689573075e-03, -3.035389100283566e-03, -9.424335883325826e-03, 1.310412904681841e-04, 8.070868495274920e-03, -1.211453449431983e-02, 1.387883787371603e-03, 1.445504794689854e-02, -1.136333941813900e-02, -4.486643322480085e-03, 7.094944450711400e-03, 6.436586369834565e-04, -8.149027363144960e-03, 7.802426892739535e-04, 1.480191170903506e-02, -9.200840367385938e-03, -1.209448891352373e-02, 2.016126806608519e-02, 2.693893186627488e-02, 1.346364625222893e-02, 3.705118322530101e-03, 1.205452617565562e-03, -4.660222565335249e-03, -1.248328177422611e-02, -1.235157203788835e-02, -4.628541433870121e-03, -3.983296643670356e-03, -1.544738877461877e-02, -2.217511331795039e-02, -9.804605411251286e-03, 7.346913297247177e-03, -5.910566423645155e-03, 8.856222514215528e-03, -6.105422791967074e-03, 9.233617546595598e-04, 4.750145476115085e-03, -6.594032100974542e-03, 4.872054976269670e-03, 1.233839470139767e-02, -7.393728706755214e-03, 5.408192763443512e-03, 2.195045944001895e-03, 6.685961044868246e-04, 1.974132138963340e-03, 8.877104120436668e-04, -1.581258118367478e-03, -1.357098041485336e-03, 1.057198757815101e-03, 1.017633790856976e-03, -8.024747466405050e-05, -3.550343972973433e-04, -1.957491379331453e-04, 7.062338299810342e-04, 1.313381614447761e-03, -1.624612665882043e-03, 6.859179898105216e-04, 5.756006888274716e-04, -1.322005147324091e-03, -1.719690916343882e-03, -1.139183707571425e-03, -1.252220505704109e-03, -6.216545109639790e-04, -5.988596981314613e-03, 1.045117150467221e-03, -1.426716346171186e-01, 4.315978952167684e-01, -1.479920100809296e-01, -1.598269951561570e-03, -5.804674889830357e-03, -1.927542795016463e-04, -1.451971089787042e-03, -2.459193449221887e-03, -3.567047996705369e-04, 7.468058194682633e-04, 3.888053543324072e-04, 1.860739693279205e-04, -7.957306258402835e-04, -1.254232388814269e-03, 8.919316343287026e-05, 3.052757638988426e-04, -1.428414398380359e-03, -6.818692537167444e-04, -5.708326907593578e-04, -2.041912109541378e-03, 8.691933203326395e-04, 9.846733264243896e-04, -4.250062453116903e-04, 1.099340204032380e-03, 3.743965298116548e-04, 4.502378568020521e-05, 3.871467962334017e-04, 3.095316325599263e-04, 9.286667897297914e-04, -1.651886071236622e-03, 8.818743781355516e-04, 8.055106845236637e-04, 4.988025748431909e-03, -3.092473553255993e-03, 1.794719646488382e-01, -5.584491505054408e-01, 1.816616570517432e-01, 1.099783946094462e-02, 8.061146571320807e-03, 2.913647627625237e-03, -7.886527558434697e-04, -4.534805972978226e-04, -3.527083979429980e-04, 5.002442627460520e-04, 1.086413209615326e-03, -1.893209793020981e-05, -3.418578699285568e-04, 1.327676424946630e-03, 2.467422587708313e-04, 4.419480833149234e-04, 1.952714546293650e-03, -5.775147327310227e-04, -8.573366308427602e-04, -1.329425549072758e-04, -8.691856783886370e-04, -4.090580023375702e-04, 1.748670278862628e-04, -1.616266224101950e-04, 3.710102681881684e-05, -6.257181576140549e-04, 1.198548987035879e-03, -1.410582841463128e-03, -9.350243709692261e-04, -4.529144942778386e-03, 3.058792499621949e-03, -1.374597492339774e-01, 4.316339221611710e-01, -1.398608720842346e-01, -7.761272179317090e-03, -7.137744708556511e-03, -2.036464672976123e-03, -4.935276706914683e-04, -1.350455601542480e-03, -6.983204040848798e-04, -1.465978467210649e-03, -1.763162068319659e-04, -7.672257698354387e-05, 9.722381625025679e-04, 5.263313686536639e-04, 2.964932459095750e-04, -7.506790061191492e-04, 6.996463147942467e-04, -3.653690808915991e-04, -1.176530985894726e-03, -3.048078045263524e-04, -5.735790100734609e-04, -1.837865155341222e-03, -8.805970795155304e-03, 3.370573381948783e-04, -1.992381360985047e-01, 6.047954776361059e-01, -1.998589860276435e-01, 1.634612848492509e-04, -7.543269958944459e-03, -8.818407885278522e-04, -1.489415502766179e-03, -1.804129944553742e-03, 8.133801978944695e-04, -3.240437898200968e-04, 7.611554335227982e-04, 4.809688247316425e-05, -6.891599165853822e-04, 8.939506749987102e-04, -6.964029408495693e-05, 3.870169661740323e-04, 5.010852249721475e-04, -6.926261085749031e-06, 6.910065060056263e-05, -1.790455699554218e-04, -3.096373573100425e-04, -8.357553041264898e-04, 1.564845456471929e-03, -4.058589802639716e-05, 4.174514099040551e-04, 1.301706871700774e-03, 1.233857509134545e-03, 2.345762449229320e-03, 1.578593956184883e-03, 4.099839931414283e-05, 1.584956748087022e-03, 5.003434587513388e-03, 2.640918254458588e-04, 1.451780609373930e-01, -4.380577376535978e-01, 1.437254010831438e-01, -5.211834720667608e-04, 5.776285634683478e-03, 1.050677332622231e-03, 2.432615649985105e-03, 3.488490041176290e-05, -1.497509322993147e-04, 7.361136795897689e-04, -5.347456197247490e-04, -1.496472103620480e-04, 3.197101658485348e-04, 2.999112644469160e-04, 5.901356360906891e-04, 1.230385169449296e-04, -2.711851168884314e-04, 5.765667399003559e-04, 1.932171033780344e-03, -1.953598007554572e-04, -7.387447565994989e-04, -1.544143800010226e-03, -9.461614460751599e-04, -2.320131622176591e-03, -3.337330534843353e-03, -7.572193810619315e-03, 1.089661008255670e-03, -2.040576434093440e-01, 6.154843371676451e-01, -2.044928322226415e-01, 1.025848223096159e-03, -7.566071872824989e-03, -2.838206554657897e-03, -5.328574083134959e-04, -8.003549760391135e-04, -6.231404861829204e-05, -9.533904782532941e-04, 1.180596592670442e-04, 1.414886953299556e-03, -7.064872511815355e-04, -8.939132654808289e-04, -3.744931204773314e-04, 7.172627466554974e-04, 1.122797869805587e-02, 4.140026039918793e-03, -8.102234303939015e-04, -1.252664731799421e-02, -2.885580594450007e-02, -2.141364922623579e-02, -5.102890147868618e-03, 3.368602896535502e-02, 4.042693179955996e-03, 6.917034453514794e-03, 6.790856044345947e-03, -1.228268690374328e-02, 2.945302086184113e-04, -4.879838470818080e-03, -8.027167114327986e-03, -1.847189629400188e-03, 6.218932961940137e-03, 4.948549150789673e-03, -1.144730352114192e-02, -3.146491076903046e-03, -2.246166622935093e-04, -5.405232272964941e-03, 1.034225799200215e-03, 1.681879594151478e-04, -5.526773711419145e-03, -4.497019880192400e-03, -1.325738779943850e-02, -1.669038491616112e-04, -5.315991261961147e-03, -9.576870262696244e-03, 1.235017696179164e-03, -1.717807270034307e-03, -1.536999484778344e-03, -5.440163877185497e-03, -1.350664220235076e-04, -1.779102465210054e-01, 5.314738073467462e-01, -1.766621557740184e-01, -2.213357696403343e-04, -9.381279950794616e-03, 2.840184489545086e-04, -2.895807285398884e-04, -7.781024551962234e-04, -3.133011160390420e-04, -1.294806143928086e-03, -1.941779023085079e-03, -7.562563447724067e-04, -7.716661677606751e-04, 5.845456798746775e-04, 4.113572236714431e-04, -3.940508397544090e-04, -1.161306440053792e-03, -1.187320283012415e-03, 3.753645702362135e-04, -1.281563923670720e-03, 1.861317269065768e-03, -8.930081289203005e-04, -6.934582333786224e-04, -6.009595687326465e-04, 8.357695167731388e-04, -8.913535216922757e-05, 1.211524179998974e-02, 3.274231287505682e-03, -4.884016703377380e-03, 2.177331861947330e-02, -1.235249923031378e-02, -1.754817319953913e-02, 6.231885723927041e-05, 2.553585467878667e-02, 4.544092229290767e-03, 2.050594783306916e-05, 7.704254981465748e-04, -1.229609709145789e-02, -5.144977072260315e-03, 1.503509591656874e-02, -1.281021423089624e-03, -8.181793331584655e-03, 4.969769765583795e-03, 1.663014481530538e-02, 4.241498443929821e-03, -5.874437741723608e-03, 3.106125805079276e-03, 1.173276232346881e-03, 1.052348236836720e-02, 7.130886238501433e-03, 4.180839064390697e-04, -4.516137026333421e-03, 9.959513910503314e-04, 1.401671584755195e-02, -8.602609045314406e-03, -2.675435750917819e-03, 5.877591261473839e-03 };

theta1 = { -4.040709182625838e-05, -4.334363276254913e-03, 2.502234375834936e-04, -2.594232441081447e-04, 2.831766559941445e-04, 1.233129279742708e-04, 1.055991783898955e-05, 4.685964623026757e-03, 8.640352891365322e-05, 2.357225899828425e-04, 9.879714356307683e-04, -7.355608809040953e-04, 4.197587303330965e-04, 6.229818642920501e-05, 5.791874636295702e-05, -3.247984236835811e-04, -9.086580063187564e-04, 4.552776283204515e-04, -4.720335834437302e-04, -1.214108919732219e-03, -1.189839347177780e-03, -1.399143959824108e-04, -7.528389765394667e-04, 5.793808206295142e-04, 2.355899915609774e-03, -1.333562079620204e-03, 1.197035257429802e-04, -1.321922124452930e-04, -2.391425987318331e-04, -3.224550697297640e-03, -2.730099673879324e-04, -8.859492306105532e-04, 2.461859273686723e-04, 3.028800325778886e-03, -5.256750259698728e-04, 3.826426345219857e-04, -1.432020405453263e-03, -8.799769454858972e-04, -6.659885663602676e-04, -2.257226826883991e-05, -3.452396989901762e-04, -4.027076204209237e-04, -6.680398826431685e-04, 1.380320645185957e-03, 1.067347300296815e-04, 4.709454475683036e-03, -5.647188671326862e-04, 1.521367566961891e-04, 5.770105052131478e-04, 4.983935341502479e-04, -3.089642103470293e-04, 4.757109844192357e-03, 7.086806448907163e-04, 8.644140677842517e-04, -2.268712741545547e-04, 2.407669676096329e-04, 5.267597431597891e-04, -9.067771940188576e-05, 2.237816360600355e-04, 4.336338441499682e-03, -1.878426715409044e-04, -7.124238533178776e-04 };

W2 = { -1.996642255962705e-03, 6.533161067900498e-01, 8.980854542024246e-04, -2.411385571216547e-01, 3.455579399917825e-03, -1.042213982478371e-01, -4.395988420053862e-03, -6.463360645891283e-01, -1.529999859792100e-02, -1.767587567915116e-01, -1.546700748255420e-01, -4.393635751175210e-02, -1.092322795175040e+00, 4.330115639190463e-02, -3.162778022299243e-03, 8.060548614613115e-03, 7.961518559528532e-03, -4.824200036886555e-02, -8.351346115496863e-01, -4.261464556787697e-02, -1.391923864323977e-03, -1.013124532197378e-02, -9.319927773491428e-03, 3.652064857371423e-01, -4.936729355228737e-02, -2.868473214381957e-01, 8.141400467017142e-02, 5.024047858469333e-02, -1.846927726620535e-02, 3.195780795409612e-01, 8.735555124145772e-02, 1.642179788046037e-03, 1.700416104994843e-02, 1.680883565254008e+00, -3.910937595396267e-02, -1.650641287510473e-02, 1.352965718676733e-03, 3.765449634313841e-02, -3.288086416539012e-03, 6.953854870336273e-02, 4.603320869264475e-02, -1.985691042871129e-03, -5.014544223712808e-03, 3.853406537402722e-01, -3.007580204049762e-02, -2.293369810010372e-01, -1.284937818291023e-03, 1.866473443772825e-03, 1.376299459502022e-03, 7.279238718922308e-01, -5.700677799552138e-03, 1.683870245355023e+00, 2.936348287071912e-01, -4.982186220017944e-04, -1.662905915107177e-02, -3.289309786612213e-02, -6.250517684701981e-03, -1.550958652847698e-02, 6.044012497414674e-03, -3.072571350823907e-01, 7.531659299001708e-03, -1.355395484310158e+00, -2.843527503867042e-02, 3.071372550502658e-01, 3.809530872111109e-03, -1.941630306880170e-01, -2.148885688885688e-02, -1.900018792763234e-02, 3.836152178227230e-03, -1.951187666886245e-01, -9.690615247996037e-02, -6.237588983641836e-01, 9.899857288951188e-01, 1.370466431024314e-03, -3.190002728755517e-01, 2.948513828174787e-01, -4.020450713001836e-02, -2.441433450148164e-01, -1.386170306680573e-04, -1.309648288448391e-02, 4.817806799061747e-01, -2.485094664951634e-02, 1.228975984458620e-03, -3.670691718345032e-02, -1.883799911011940e-02, 4.495101745427016e-01, -8.640621332998118e-03, 4.193492291304189e-01, -2.105267992278304e-02, 3.017037246456832e-02, 3.066071712166961e-01, -1.092077471727780e-01, 1.450134990061463e-01, -5.877899540453908e-02, -4.027012891213232e-01, 7.905238865427782e-01, 1.968907080935915e-02, -1.610575316762538e-01, 4.035711978802758e-03, -7.624763541864654e-02, 2.498824658627574e-03, -3.621980238043119e-01, 1.249843971670726e-01, -2.259079457990544e-02, -4.651208552591025e-03, -2.481251198284979e-01, 2.744723116934224e-01, 1.406934161955796e-01, -4.258646826687814e-02, -4.215946193122513e-02, 2.706261957952878e-03, -8.699087532483742e-01, -2.206714975779687e-02, -1.879631884055571e-01, 5.323931243479040e-03, -2.808796057337696e-02, 2.653877425892467e-02, 5.968672049731625e-03, -2.575143775844747e-02, -7.058821403424501e-02, 5.256791916118175e-03, -4.059701004364117e-01, -2.527662016097882e-01, -2.010101776194002e-01, 9.220343531491891e-03, -5.294683839625445e-01, -2.097450045950805e-02, -8.233752808249684e-02, 2.837652081020033e-03, 2.511686552311551e-03, 1.467705391474721e-04, -2.784780511480171e-01, -6.341769627983902e-02, 2.882190265480269e-01, -5.930986558222982e-02, -1.380741775149856e-02, -8.500987785144694e-01, -7.548939488202810e-02, 6.099442194674727e-02, 1.001784810363440e-02, -2.844222466310305e-02, -1.705272263976603e-02, 3.878784505703998e-01, 1.070794091428463e-01, 2.607311637946464e-04, -4.723839787606907e-02, 4.968974513046207e-02, 1.220414939030817e+00, 2.449352756463374e-02, -2.153361861727965e-01, 1.458277703362606e-02, 2.730763795621494e-02, 1.168101914843425e-02, 3.512105606274535e-01, 3.014850161511897e-02, 1.153703940617483e-01, 1.049079149674455e-02, -1.285935152457092e+00, -6.050509654355781e-02, -3.511090337253417e-03, -3.264432637797914e-02, -9.385321137008951e-02, -5.266814181954431e-02, 1.167655673451394e-01, 3.008878944867748e-01, -1.041377650541992e-01, -3.578479085629369e-02, 2.804760391933098e-01, -6.584905606582693e-02, -1.407177283131132e+00, 1.178827371729074e-01, 9.048456613462874e-03, -4.412342091790106e-03, 1.229563116761598e-01, 4.214707520652853e-02, 4.457641246328252e-01, -3.298533709846045e-01, 1.319700206203482e-01, 3.373501988291914e-02, 1.375755001370427e-01, 4.909257202127667e-03, 1.636437853433680e-01, 7.877874169425510e-03, 1.335386764532285e-02, 4.236736088466311e-02, -6.785544121657104e-02, -1.472606164587244e-02, 3.328706701987700e-01, -3.695464758164551e-02, -4.690030901300670e-02, 6.350209595864641e-03, -8.518042303884477e-03, 1.802736814039234e-02, -5.715671855314735e-01, 1.634077938167171e-02, 1.926066100732822e-01, 8.094489325527067e-02, -5.797953822228364e-03, 3.474736078873229e-01, -5.074267754567587e-02, 7.419208363861460e-03, 5.936597977232824e-02, -6.082539748312106e-02, -8.583908782063223e-03, 3.371672992949080e-01, -1.032874933774984e-02, -5.306093518544361e-03, -2.089851895294529e-02, 2.069133529807930e-02, -3.937135657598592e-01, 2.119861907719643e-02, -5.021798323701574e-01, -6.553144243673364e-02, -3.842994667850919e-02, -8.593742183762249e-02, 3.589795108822367e-01, 2.015136837676991e-02, 9.845441143856998e-02, 2.284915340740470e-02, -4.371519088853289e-01, -1.013409988293172e-02, 1.558300973054823e-02, 4.044467754744135e-02, -5.232266541433687e-02, 1.778519763890245e-02, 7.519263934012158e-03, 1.087444404994384e-01, 1.898313333697206e-02, -5.116248333573270e-03, -1.029766868706956e-01, -5.826978484851995e-03, -2.588679704671805e-01, 3.867753128402895e-03, -9.652676383185111e-03, 3.563208957688829e-03, -1.441313674868038e-01, 7.287434756829286e-03, 4.616617534715984e-01, 5.493039298555731e-01, 1.616601248620563e-01, 4.155157958883369e-01, 6.966164796874826e-01, 7.914346685549694e-03, 6.276758176859513e-02, 1.480923539118087e-02, -6.651013712185053e-02, 1.647012926377102e-02, -2.354891804584505e-02, -3.826859530213865e-03, 7.513874408496892e-01, 9.923245306250404e-04, -1.253812280772240e-01, 2.567695544181782e-03, -5.089728186515660e-03, -3.486534359784669e-03, 2.039947617120247e-01, -1.816145939691417e-01, 2.942460523263855e-02, -5.911258407752471e-01, 2.607787126100282e-02, 3.769459223253542e-01, -5.761605718681826e-03, 1.796621021445795e-01, 1.625888376864817e-02, 1.930014075952781e-02, 4.305049822137896e-03, 3.114861388719970e-01, -1.287899577983195e-01, 8.500816691080082e-03, -1.306270784066528e-01, 7.902614693751839e-03, 1.837578022748467e-02, 3.105952708250654e-02, 5.884403587137149e-01, 1.167562822082591e-01, 9.763016854580234e-02, -6.207029311129039e-03, -4.661055129444143e-01, 1.309986018543766e-01, 1.507546040754588e-01, -2.322411073059695e-01, -7.858051271051484e-01, 7.156468258521467e-02, -1.793401820885165e-02, 2.090042031543334e-03, -3.813176530028138e-01, -6.403321545817501e-03, 1.126653638105361e-01, -2.950486594317950e-01, 4.264595562006027e-02, -3.149681175829996e-03, 3.341862384545721e-01, -6.090535595870035e-02, 1.109731496711259e-01, 1.962552976168091e-03, 2.201117096278010e-02, 1.034771585810586e-02, -1.502050503556907e-01, 1.309791288811544e-02, 5.697145510795101e-02, 4.390132806982461e-01, 1.440811772027957e-01, -2.690908052024032e-02, -7.121209564499630e-03, -2.694702703713535e-02, 3.042266240541057e-01, 2.090965982562850e-02, 2.625744922624466e-01, -1.340909948918358e-01, -1.823257148035851e-01, -8.883257461830012e-03, -3.145141837746639e-01, 1.420563118675712e-03, 9.665092086117133e-02, 2.296672443259664e-02, 1.379141167541146e-02, 1.606230006765555e-02, 7.796373894238534e-01, -3.451024541631265e-01, 4.383201113033716e-01, -5.044837523834727e-02, -4.081665597114145e-03, -7.082151683597587e-02, -9.949876955660242e-02, 2.307571994208829e-02, 4.956941452613893e-02, 1.341103685219880e-02, 1.097395912114027e-02, -1.666569333901549e-01, 5.385196020386415e-02, -7.549135065661352e-03, -2.779595055147147e-01, -1.188385819457356e-02, 1.982223601850807e-01, -1.310670860606017e-02, -8.344107058618148e-01, 2.564668189800342e-01, 2.105618829079274e-01, -7.232756897843164e-03, 2.024789985965708e-01, -9.110314324478343e-02, 2.183980306337576e-03, 1.593634514311471e-01, 6.983794627016697e-02, -2.387492207055383e-02, 4.421082247759816e-02, -7.982743907902141e-03, 7.334636335984107e-01, -8.771261890167563e-02, 9.332846818352269e-02, 2.954998933446204e-01, -5.676735020221375e-02, -1.974597757628670e-02, -7.064455949463068e-03, -5.638448268639682e-02, 5.198785520994770e-03, 1.784653109036030e-01, 3.459727222017527e-02, -1.489332019456957e-03, -5.569442271980882e-01, -1.219129303265338e-02, 9.213029490670130e-02, -2.806302424396248e-01, 1.603808540027918e-02, -2.507832209275034e-02, -6.355534496204869e-02, -2.095064148145070e-02, 1.703969242916337e-02, 4.267295723103979e-02, 1.367806431571900e-01, 1.183661210627961e-01, -2.400581959956314e-01, -2.789504132404031e-02, -7.627726541249915e-01, -3.144144270875241e-02, -1.007937595294425e-01, -4.249237178601768e-03, -1.924368475340966e-03, 6.748878017654045e-04, -2.626508499850060e-01, -1.077773728889151e-02, 1.036557247242517e+00, 1.145994408895596e+00, -1.291062532021863e-03, 1.613872003327271e-01, 1.240577078586181e-01, 4.001063942487465e-01, 1.591948417774149e-02, -2.109413043656860e-01, -6.915809118782094e-05, 8.946761916790750e-01, -4.788212074153057e-03, 1.470484652761663e-03, -2.538353739186826e-03, 6.255137318325493e-02, -2.918313250108406e+00, -1.111782248775766e-01, -9.969539333621472e-01, -1.088311930907010e-01, -5.763525308349889e-02, -2.580408640018035e-02, 8.230240365789698e-01, 4.762315078264313e-02, 5.211820720305743e-01, 6.129402421128916e-03, 1.872622348683838e+00, 3.646098532877166e-03, -3.267276204049049e-02, -2.183044216307192e-02, -4.925646725478799e-01, -2.755367803000049e-02, -8.066625428971232e-02, 4.349490330058524e-02, -2.318683362722800e-02, -2.395874626921793e-03, 7.468682667367869e-01, 3.561710599355716e-02, -4.178668923421791e+00, 3.853542417848244e-02, 1.273132546012280e-03, -2.825879340856859e-03, -2.563069466220119e-01, 1.567573758423259e-01, -1.428007602944183e+00, -3.223163227555128e-01, 1.132062253313130e+00, 3.550299503536803e-01, 3.014289282557592e-01, -1.514904268080160e-03, 7.916454710718691e-01, 6.608701285744577e-05, 9.500639931045751e-02, 2.870351770990774e-03, -1.044026422984587e+00, -4.116707134542380e-04, -1.736442395431110e-01, -3.151847913075076e-03, 6.938800359573730e-02, -2.485255339457559e-03, 6.841596458990291e-03, -1.158480523489162e-02, 3.835320520007616e-01, 6.746914177981082e-02, 4.586734652976817e-01, -8.432901712923983e-02, 9.421934063649584e-03, -1.092444780848308e-01, -1.878690757980903e+00, 3.739552912983182e-02, -1.759510444498919e-01, -1.271706010156516e-03, -4.014186147685335e-04, -5.658488394934916e-02, 1.005869422459320e-02, -1.645444757220955e-01, 4.874650463573055e-02, -1.469836941907480e-03, -1.225927496912096e-01, 1.228979914390910e-01, -9.870543819521095e-02, 8.018461804636423e-03, 5.331613375068985e-03, 3.900488769495641e-01, -9.003137836021417e-02, 9.176662039660758e-03, -1.821822980980366e-02, 1.078743140785314e+00, 1.625144299937336e-01, -7.745463481462126e-03, 7.782606442950385e-01, -3.908518608443471e-04, -1.174786764927631e-01, -3.700633761486702e-02, 3.668097339232106e-01, -3.830831284841120e-03, 1.529943872874339e-03, 6.925803099845676e-02, -8.990858250334834e-01, -2.581238412718178e-01, -1.960244929550358e-01, 6.903659630537819e-02, 3.228371992852501e-02, 3.134614939982949e-02, 4.090696788922207e-01, 1.995319841205152e-03, 3.348100525052074e-02, 4.996953973647235e-01, -4.239592417561089e-02, 2.708364486765645e-02, 4.777969509596975e-02, -4.188922179812023e-03, 6.878817458642449e-02, 2.067360218430782e-03, 1.820446566232288e-01, 8.039386174855689e-01, 3.873756013728404e-01, 1.896745823762283e-03, 1.308914536546778e+00, -2.569568371388083e-03, 3.284873934120217e-02, -2.711334604996257e-02, -2.831944475788951e-04, 7.257553441099157e-02, -1.225307136386466e+00, 8.327394133169740e-01, -2.363843815747847e+00, 6.437873298572744e-01, 1.015704595407114e-03, -7.390927562173302e-02, -3.230881886538142e-01, -6.387772301298968e-02, 5.942266014434266e-01, -1.535891635716124e-03, -6.111259095101314e-04, 6.638310001795730e-01, -3.868348859723578e-03, -1.889837946375017e-01, 7.235405935975109e-01, -8.767361314943894e-04, 1.720645620727486e+00, 1.303219512698144e-01, -3.418212242350215e-01, -2.407828890198473e-01, -8.034348011083199e-02, -1.276022752725003e+00, 2.671738035748362e-01, -1.050075384495671e-02, 2.963241972787949e-02, -1.578240178579993e-01, 1.330891028779171e+00, 2.516319700511796e-03, 1.970321067821151e-02, 1.939029863374622e-03, -3.677063524922876e+00, 6.194542180867035e-02, 1.008778674697478e+00, -8.999262527901773e-02, 2.390964330238547e-02, -1.318304982270957e-02, -2.248769487665055e+00, -4.317578305535715e-01, 1.380472869134473e+00, -8.884281916282281e-02, -2.739598095426373e-01, -3.870242023124395e-04, -2.381433764212052e+00, -2.227990412190465e-03, -1.634460895335149e+00, -7.628240043489777e-01, 5.828349741482506e-02, 4.531086457423260e-02, 8.022545389206381e-02, 5.005179072446414e-03, -1.144525745589560e-01, 1.012579849039429e-02, -1.578998160748821e+00, -7.889894423799819e-02, -5.896832691538300e-01, -2.382424264530444e-02, 6.783892326302859e-01, 1.202697882514733e-02, 1.227934407543181e-02, -4.652830770640403e-02, -1.946429892273157e-02, 5.115379472753677e-02, 5.900976873498846e-02, 4.989373801816715e-03, -2.814932835067784e-01, 6.552069871730297e-01, -1.314586983155555e-02, -6.470005533876130e-01, -1.235806806376839e-01, 5.012371935446717e-02, 1.646581759933627e-01, -3.563252219771805e-03, -4.591316999850805e-04, 2.396026383050391e-01, 1.680983633045684e-02, -1.054482776274123e-02, -6.320006813179275e-03, 3.477598873515795e-03, 9.804457071849857e-01, 1.933532559236254e-02, 5.424140980437751e-03, 1.311288316994668e-01, 1.410220333506322e-01, -3.036621624122928e-01, -4.670139031102669e-01, 8.958950392882086e-03, 1.428674275991042e-02, 1.046285153414688e-01, 2.208006368295259e-01, 9.728451162529610e-03, 7.832293390462718e-02, -6.345277695738673e-03, 1.913234935147642e-01, -1.603285303764043e-01, 1.319169343364257e-01, 2.120118973838958e-01, -6.471056753794545e-02, 1.985917872371484e-02, -2.509641068305221e-01, -1.047383028139277e-01, -4.530975318552051e-01, 1.700913324214625e-01, -3.404634566028963e-02, 2.983693872842786e-02, -4.337903979972892e-01, -1.019947367844411e-02, -6.356716915312435e-01, 1.749547349529951e-01, 4.395287043787972e-02, 3.239696326556719e-03, 3.844599912682328e-02, -4.858081008294554e-02, 8.502585927723103e-02, 2.551079576909405e-03, 4.778354125175562e-01, 8.951095206904829e-02, 2.427497789245540e-01, 7.207163923120754e-03, -2.920126196809849e-01, -3.528177268426095e-04, -3.084526976095336e-01, 5.160126079542848e-03, -5.064982071579686e-02, 1.826334862894428e-03, 8.233806385800158e-01, -5.044120529686077e-02, 1.332350949405749e-01, -1.185977554272914e-02, 2.272640285008441e-02, -4.821116356642526e-01, 1.612144299349560e-01, -1.260708574680308e-01, -1.177727398117522e-02, -1.776065219919190e-02, 7.299295962935234e-03, 1.089635192596545e+00, -1.902128095842056e-01, -2.077788651905406e-02, -4.345438096859476e-02, 9.227795900570629e-03, 2.492101746898429e-01, -5.472244966834737e-03, 4.306424814571431e-01, 6.590209466531742e-02, 4.991316642049792e-02, 8.422964775977666e-02, 8.228651475692315e-02, 2.659136379577150e-01, -9.305296038373498e-02, -2.262550862475395e-01, 3.632836447177104e-01, 1.398869330842467e-01, 1.085605648603106e-02, -8.897564606958184e-03, -1.067075367800130e-01, -6.272294215689843e-02, -1.398303563884568e-01, 1.832592329669251e-01, -4.943494458317940e-02, 6.364320761603044e-02, -2.735598370530470e-01, 6.896788437291894e-02, -5.714530562831046e-01, 1.426951850536945e-01, 1.590050087800490e-02, 3.029169236572758e-02, 1.254249065952280e-01, 9.243020026218811e-03, 4.076688173659767e-01, -4.292148802418438e-02, -1.151952349833908e-01, 1.691839742196301e-02, 3.532936900091711e-02, 2.148301242659528e-02, -1.790073885327051e-01, 2.671424100845659e-03, 7.971278952539019e-01, -1.788247475356448e-01, 7.300098340244093e-01, 1.401739874436763e-02, -3.285389143067109e+00, -1.692858503706489e-03, -8.919592594602420e-02, 2.567551993234069e-01, -1.510815659279383e-03, -1.906147361256677e-01, 1.703511791826050e+00, 1.028205337543294e+00, 1.529703343331187e-01, -2.309686941686054e+00, -1.226456194012625e-03, 2.272126938158442e+00, 1.366325761207744e-01, -4.868854395442874e-02, -1.165274140461829e-01, -2.280694704491991e-04, -1.777894109783241e-04, 1.204256916593594e-01, 3.540854290261953e-02, -1.033151162122708e-02, 4.559188896178855e-01, 6.345686791152756e-04, -3.910356146102774e-01, -2.090291297195132e-01, -5.927955587395723e-01, 1.351139398495447e+00, 6.275392648220269e-01, 3.095323968534902e-01, 3.368070290699779e-01, 4.196392915062144e-02, -7.069635974369175e-02, 3.976857654650433e-02, 9.401562963392526e-02, -1.173572553791791e-02, -3.755648947879132e-02, -1.108423208008230e-03, -1.067413463786564e+00, -2.846237334826784e-01, 8.587881713516460e-02, 2.145544085760792e-01, -2.811471522621217e-02, -1.753797470340722e-03, 3.863145003840818e-01, -3.149695932345497e-02, 2.409982855663214e-01, 6.244632519945963e-01, 3.385599353439755e-02, -4.537823673698958e-05, -6.304995245518920e-01, -5.610128181423089e-03, 4.023935199767983e+00, -6.855467771545819e-01, -1.276762055816481e-01, -3.294780640098009e-02, -6.140704382495291e-02, 4.131412583136297e-02, -1.129510372130115e-01, 1.920054055191489e-01, -3.923264874540926e-01, 2.293213932954184e-02, -7.483177830418096e-01, 3.836038175436493e-03, 3.258072995290570e-01, -7.026680969375983e-04, 1.012539946148227e+00, -1.121446248997078e-03, -3.084291142930960e-02, 1.691388266452525e-03, 1.291019753922421e+00, -3.779616633025819e-02, -1.054398325481606e+00, -6.474643775748191e-01, -1.706476599103612e-01, -3.301580493609160e-01, 1.273667008309306e+00, -1.025367230789766e-03, 6.311656892984521e-02, 1.518990720736610e-03, 7.325703259811232e-03, -2.002559732949172e+00, 2.990617151741060e-01, 1.292142594514287e-02, -2.002983428689003e-02, 9.566075074994410e-04, 4.025902140097987e-01, 1.737428453076957e-01, -1.087927152893332e+00, -1.781848926732821e-02, -5.463620027113446e-03, -1.358196598096841e-01, -1.061762865179218e-01, -4.083045270500107e-01, -5.384925497680271e-02, -1.198731919350945e+00, 1.855970516687459e+00, -4.098015844963367e-02, -2.623754256415055e-01, -6.290121720148789e-04, -5.228521828340850e-01, 1.252067718525984e-02, -2.469036489190293e-01, 4.588707589996822e-02, -3.173716626451961e-03, 1.957584990288974e-01, -7.547886492128035e-01, 6.232037573483962e-02, -8.621318171566893e-01, -3.396242686707793e-02, -1.636077019493328e-03, 2.430203156574545e-01, -8.508646154763684e-01, -8.870489884747903e-04, 7.122597884182670e-01, 5.119971652425823e-01, -1.040124220674493e-01, 3.775460101958865e-02, 8.209663278921372e-02, 1.120977821212406e-04, -5.676509470142230e-03, -7.761378112687726e-04, 4.501728660621581e+00, -5.203452026968279e-01, 2.319058849813940e+00, -4.908825505241441e-04, 3.660310834930538e+00, -7.139268669769948e-04, -3.908164071680760e-02, -1.714352360573842e-01, -6.092215084892185e-04, 2.714420553445312e-01, 1.276186026487824e+00, 1.272628835450599e+00, 3.175859338282347e+00, 6.375142311406151e-01, 1.171875978697669e-04, -9.333628617840070e-01, -2.482996194028981e-01, -4.885555165240623e-02, 6.928254842704179e-01, 1.964610736507380e-03, -1.920315245183245e-03, 7.131708963064938e-02, -8.599138299841372e-02, -3.324128290044671e-02, 5.973797813305338e-01, 6.694636237327584e-04, 4.317309811774903e-01, -9.757542379260735e-02, -1.449334418531391e+00, -9.007414961136787e-01, -4.510439896416359e-01, -7.666599975392524e-01, 5.440711665295094e-01, 2.101891271777029e-02, -6.174639733679098e-03, -1.508066544107257e-01, -3.079989976356450e-01, 2.372027670241202e-02, 5.263048806649245e-02, -9.035712985656245e-05, 1.684304597953136e+00, -1.444048639585016e-01, -1.545222110594253e-01, 1.133971760330352e-01, -1.430953532785730e-02, -7.098832996653300e-03, 1.733626474220235e-02, 1.228450704348450e-01, -2.802876764962813e-01, 3.036871897830450e-01, -7.187043259716948e-02, 3.833673699765373e-03, -1.001605444713842e+00, -1.573696243576189e-03, -1.036894107951434e+00, -1.108447347377732e+00, -1.171670007561581e-02, -4.925143066607256e-02, -9.325034775047737e-02, 8.397511277522567e-03, -9.204289465085344e-02, 3.951936117718373e-02, -4.969645923761144e-02, -7.144134558336404e-02, 1.764590979042000e+00, 1.998137737358841e-03, -1.049994630063051e-01, 4.659549657960530e-03, -8.815502447297322e-03, 8.189828299155129e-03, -1.309454231319691e-03, 2.666157257033264e-02, 1.754955848916213e-01, 1.179075897912152e-01, 1.409962200379813e+00, 7.072264604282361e-02, -2.771559648625388e-03, -2.816529527476047e-01, -9.021113989249655e-01, -2.367438790610348e-02, -5.497837237719038e-01, 2.065147723582142e-03, -2.667936134346480e-03, -5.651786755033142e-01, -6.883489240421922e-02, -2.762694810704237e-01, 2.081574111581409e-02, 1.732174641709930e-03, 3.621741250852354e-02, -1.948740574160632e-01, -7.633439089577168e-01, 1.512889839758489e-01, 7.479048725528609e-02, 1.190905655788967e+00, 2.268667457622723e-01, -1.496756746828198e-03, 4.554682856469430e-03, 3.228083379985610e-01, 6.152866062718388e-01, 1.853011210658407e-02, 4.988479211112418e-01, -1.188891894601789e-03, -8.534368457076045e-01, -4.724187028566678e-02, 9.285714012937287e-01, 1.844586495328959e-01, -2.892649046588635e-02, 3.158599227704659e-02, -4.670104667828641e+00, -3.277975819555786e-01, 1.681920868857914e-01, 9.348686721784123e-02, -1.895762307436520e-01, -6.599115132905780e-03, 7.871919068761559e-01, -1.638325669876519e-03, 9.714781054265988e-02, 3.375315532022198e-01, 1.120316555910403e-02, -3.633715683419585e-02, -5.689677586612603e-02, 2.140573993667485e-03, -4.810202484467942e-02, 1.306255264284838e-03, -9.439317955242069e-01, 1.165084080118983e-01, 4.522138621159040e-01, 2.950388789575181e-05, 1.888444632946642e-01, 2.676573431170022e-01, -6.444337657210478e-02, 9.814600924873999e-04, 3.428662588438098e-03, -2.493012462493269e-03, 3.550716827434429e-01, 3.884259458104645e-02, 2.668151760628920e+00, -7.103943583239483e-01, -4.080473117768299e-04, 2.357067731583463e+00, 4.929246149791952e-02, -2.435523366421359e-02, -7.663917501448972e-02, 1.660252010493132e-02, 1.020139077892688e-03, -1.151828996965191e+00, 6.263896688972992e-02, -4.094164197677587e-04, 2.206412023053022e-02, -1.348074767163337e-01, 1.457315318324143e+00, 6.330759283960960e-02, 2.343876332719083e+00, 1.184530496753111e-01, 6.717952204660334e-02, 1.383959563298049e-01, -1.111467151428767e+00, 3.835085369809967e-02, -6.503911258982813e-02, -7.491846288531008e-02, 2.008584934917660e+00, -2.282317166891865e-02, -1.308921929441154e-02, 3.271115496750770e-02, -7.708527868624589e-01, -1.904784029951980e-02, -3.626938771984629e-02, 3.054116229477652e-01, -3.313270221718939e-01, 1.428677692491102e-03, 2.051083608537995e-01, 2.285627030678038e-02, -1.492550008410455e-01, 3.826960440368332e-02, -2.346216949819737e-04, 2.256556151034390e-03, 5.130050299405043e-01, 6.028599879808429e-03, -1.846969992104479e+00, -1.392116145646126e+00, -1.825676168793108e-01, 4.650429495819302e-01, 9.769369276091741e-01, -6.367925859831141e-04, -3.347111169166436e-02, 1.398501987124755e-03, 1.122538018754719e+00, -4.126025642210100e-02, -1.315654075189609e+00, 3.218703478843536e-02, -6.609807003556659e-02, 2.344184408544649e-03, 1.324314622170226e-02, -3.216134287725262e-02, 5.918455466807550e-03, -1.864359393657539e-03, -2.939495815253970e-01, -2.810964819345756e-01, -2.756754743392026e-02, 4.838116687609020e-01, 1.381994832632525e-04, -5.041885168442106e-01, -1.959700150696499e-01, -1.571898928607745e-02, 1.003173188799446e-01, 1.301615383766885e-02, -5.161707582113721e-03, -3.804738726069982e-01, 5.899940572008550e-02, 6.374657207178471e-03, -1.576898874184594e-01, -1.097498632132760e-02, -6.928049818987870e-01, -2.239755704711568e-04, 3.382498460442686e-01, 3.787202986542851e-01, 2.825694017477116e-01, -1.011181076648469e-01, -8.567153962187983e-01, 1.365881985128226e-02, -1.347059799255533e-02, -5.882708573864250e-02, 3.411585068101088e-01, -2.558816520615538e-02, 6.831518587650377e-02, 9.748319288100209e-03, 4.754081436092609e-01, -2.488480065862113e-01, 3.547051006667731e-01, -1.207647711104446e-01, 4.292844241650776e-02, -1.802999018381527e-02, 1.706839894159074e-01, -2.254588161179595e-01, 4.705330354107143e-01, 2.884052406211572e-01, 3.249104071647249e-02, 8.556530183871182e-03, 1.019920790588513e-01, -2.686771451304976e-02, -4.706891673923835e-02, 8.683403154274118e-02, 2.972215460039941e-02, -1.749227561110064e-02, -6.251637647049212e-02, -2.435440101517880e-02, -1.421509317414143e-01, -2.514920645098305e-02, 6.875553214849423e-03, 1.445140174896824e-03, 1.835170472880900e-01, -1.709877451923485e-03, 1.727433603063619e+00, -1.112382847234287e-02, 1.724008025149787e-01, 6.928276462174159e-04, -7.108533885355163e-04, 1.358920804207849e-03, 5.830242244821497e-01, 3.342883400071889e-02, -2.736787136387717e-02, -4.948626500863306e-01, -1.418370386831755e-03, 1.197656084085271e+00, -1.355206492091905e-01, 2.001217344621883e-02, -2.587729532511412e-02, -5.470909706492081e-03, -6.787379925460925e-04, -1.464651619812634e+00, -4.457205156387511e-02, 2.027170085277506e-03, 1.608676028266389e-02, 4.891647600716068e-03, -2.132427780039209e+00, 3.295585603412840e-02, 1.472271389569656e+00, 9.553357229691342e-02, 5.169092379949462e-02, 3.862573729959089e-02, -5.853002579585121e-01, -9.017736750175946e-02, -5.334584676489940e-02, -1.735695055162198e-01, -9.938638158217237e-01, 1.365098411352990e-02, 3.091055757650778e-02, 2.111986073144793e-02, -1.055805906761907e+00, 3.945117674078922e-02, -2.263531868874109e-02, -1.343250919593967e+00, 2.927264851213643e-01, -2.132807279918495e-04, -1.307102158028199e-02, 1.245106263751247e-02, -1.970940748633627e-01, -6.828918672884451e-02, -1.048927307798165e-03, -1.323630231869252e-03, 3.172902275866677e-01, -3.263376575158663e-03, 2.795270071651645e-01, 5.092280741322223e-01, -8.447177742662179e-02, -3.563259914873498e-02, -5.428556339596680e-02, 2.889495079668125e-03, 4.138808775777304e-02, 9.618300658475458e-04, -6.528523966687019e-01, -9.406087680473194e-02, -3.896989902751298e-01, 4.735186334786800e-03, 1.509494984288876e-01, 1.217675706374059e-03, -3.360160108194750e-01, -1.524995627467801e-03, 3.134921874494000e-02, -1.890163440762148e-04, -5.590678612746492e-01, 6.991498871524697e-02, 1.351900309323050e-01, -2.389967910781034e-01, -1.315624974946813e-01, -1.412282253056296e-01, -2.026117722183683e-02, -1.614567843303701e-02, 1.282343158777564e-02, -2.255810277179731e-03, -6.685675043752885e-02, -8.163949251184746e-01, 1.533756187749772e+00, 7.281152553769906e-04, 4.817048280605364e-02, 1.294185224657004e-03, 1.384768754942655e-01, 9.649253284679561e-02, -4.842091148968450e-01, -8.429934378919655e-02, -6.396160287862360e-02, -3.424341332918016e-02, 2.777204859310373e-01, 1.611675966882539e-01, -3.188309228137246e-02, 2.395007537143887e-02, 4.519209637346825e-01, -7.123733029943219e-01, 1.515569216943325e-02, 4.200007110151459e-03, -1.145235004853948e-01, 1.175003597611853e-02, 2.530812592409149e-03, -8.727306685762097e-03, 7.646946655876739e-03, 2.151935439426481e-02, 1.632497469908744e-01, -4.564748476944948e-03, -5.751178594787710e-02, -7.827905127683249e-03, -5.694070265900728e-03, -3.276710276638821e-02, 5.044690392707759e-01, 4.650450999892042e-03, 3.625896713274421e-01, 2.621352395191486e-01, -6.648564368352898e-02, 3.075238264812014e-02, 6.537344911439874e-02, 1.224623431530434e-02, -1.914206344992567e-02, -7.197000452581996e-03, -2.401293488474877e-01, -1.151868945950714e-02, -2.218349964265547e-01, 2.428201828820825e-03, -6.348577487798791e-01, 3.217681471814701e-04, -1.167697639858664e+00, -6.215537141008573e-04, -6.061623039932163e-02, 1.418851945503638e-03, -1.896437829138069e+00, 5.042962440333198e-02, 6.544984632769708e-01, 7.594293098391872e-01, -2.603836441562964e-01, 6.466162242804422e-01, -6.583727410446746e-02, -6.042121604688973e-03, -1.961405306876557e-02, -1.020354894533537e-03, -1.023591936564298e-02, -4.820969597473259e+00, 1.162050648944715e+00, -4.425673776757970e-03, 2.479633411794665e-02, 2.648563022862693e-04, 5.679310221829355e-01, -1.212049365865609e-01, -1.691265764442624e+00, 5.295943144149687e-02, 3.240323108428277e-02, 5.045238278241700e-02, 6.408079294135790e-01, 4.835378909934260e-01, -2.220215698038213e-02, -7.691343191991903e-01, 6.940177977750106e-01, -6.956928138953513e-01, -1.811157026465456e-03, -5.784545080409830e-05, 2.508749520608821e-01, 5.122472715200334e-02, 1.297811278373003e-01, 9.910812168418000e-02, -1.146987028975378e-02, -4.094741869006974e-02, 3.704426944862668e-02, -4.910405024045930e-02, -7.801709696214125e-01, -9.955612556484157e-02, 1.559147803608305e-03, 1.764893656944739e-01, -1.122020485012500e+00, 1.140751977282355e-04, 1.146597632838067e+00, 4.034781256189245e-01, -3.902329411206093e-02, -5.091638328428328e-02, -1.006996277942689e-01, 8.452749101421673e-04, -1.501003801922188e-02, 1.135098593566528e-03, -7.640533664963250e-02, -3.543286197426353e-01, 1.647050727826202e+00, 1.888963537862793e-02, 6.025881811941110e-01, -2.398948990400335e-02, 1.559056122957405e-01, -2.814249510965683e-02, 1.702558553097617e-02, 1.915879969461765e-02, -1.124530159440697e-01, -9.063182408844861e-02, 3.713334038187128e-01, 2.373771827155624e-02, -1.942048048425898e-03, 1.273292204052110e-01, 1.226344070532080e-01, 5.475963600386271e-02, -1.218988695473424e-01, -7.608693743136039e-03, 7.679303286091385e-03, -4.999339728036462e-01, 1.575553669837195e-02, -1.209264523952374e-02, -5.158770658746738e-02, 1.536020417877580e-02, -7.360996111269096e-01, 1.509801268436062e-02, -4.090541719270864e-01, 2.390078940361184e-01, 1.985755375361264e-01, 1.719156043948790e-01, -6.717485402728668e-01, -1.278205220065831e-01, 7.232191843161943e-02, 1.249326217570527e-02, -9.008016560922169e-01, -2.111330704330142e-02, -5.344778643871540e-02, 5.434430873576345e-04, -8.685855294117062e-01, -1.287667965597599e-01, -3.233370263182226e-01, -2.503652496700242e-01, 4.450042487205071e-02, 1.283002790397694e-02, -8.630751793836342e-02, 2.232753145789838e-01, 1.453337528796738e-01, 1.633106828230159e-01, -2.576156268028545e-02, -2.655186488041622e-02, 3.350907925836369e-01, -5.949935369427287e-04, -1.735464546874634e-01, 9.483665614942435e-02, 9.133913099479084e-02, 1.176811775910907e-02, 2.499309149004325e-02, -1.931446787216635e-02, 6.244476226059156e-02, 1.686139977341222e-03, -2.949811252506541e-01, -3.841455076153256e-02, -1.651247058137641e-02, 5.639440446326911e-02, -6.319367442422189e-01, -3.601863226687813e-03, -4.483851593978613e-02, -7.253028737940185e-03, -2.256904813311748e-02, -2.861038500192512e-02, -5.637410897684204e-01, 3.613815577533685e-02, -6.629498449989503e-01, -2.747220877835249e-01, -2.421193800770035e-02, -1.033651866385387e+00, 9.043899443987587e-02, 1.437175105378944e-01, -1.253651528825044e-01, 1.836009581617018e-02, 2.472501047013087e-03, 9.808793037622619e-02, 1.056420904394971e-01, 1.890752171603541e-02, 7.116687215862005e-02, 1.883010181384511e-02, 8.526126813838791e-01, 1.365171131810883e-02, 9.051544901139755e-02, -1.266689703124064e-02, -2.228688751754779e-02, 1.289034772577563e-01, -1.326880943370098e-02, 1.938773049330201e-02, 2.297765128725060e-01, -8.558986606391684e-03, -5.404362680598626e-02, -1.627297016621592e-02, -1.913454441870537e-02, -7.191692798260272e-03, -5.299354581156072e-02, -2.447245861799805e-02, -2.239750960980460e-02, 1.516575756096770e-01, -2.595551598692042e-02, -1.420592250954459e-03, 7.222772858173808e-02, -6.122444166931136e-05, 3.586951504191592e-01, 1.138295455220039e-03, -3.154020891527590e-03, 2.009306051109089e-02, -4.356190799277567e-01, 6.260771283657804e-03, 2.315859915785522e-01, -4.980741164033253e-01, 2.944497299286816e-01, 1.129227532936713e-02, 7.071211911996704e-02, 1.308570591627288e-02, 1.374770375828449e-01, -2.716319796768239e-02, 6.571233819318439e-01, 3.586417930437547e-02, 4.068782659173137e-01, 3.316826511099608e-02, -9.508216992601218e-01, -2.642659835365838e-03, -2.828680564995227e-02, -1.683150101653520e-01, -3.244546797551621e-04, -3.844557841655272e-02, -1.315693387739482e+00, -7.588170931778239e-01, -6.555017948905232e-01, 1.520661963245580e+00, 9.472480708623124e-04, -1.604213835426590e+00, 2.354114861492343e-02, -9.553845939827710e-02, -7.988485981054302e-02, -3.530931221581281e-03, 1.251176569672397e-03, -9.130455656161586e-01, 1.101480998913444e-02, -7.694351278982332e-04, -3.733765695818416e-01, 2.330872228852843e-03, -1.549172821222827e+00, -1.859711249163215e-01, 1.042553223438833e+00, 1.381820395559682e+00, 6.418441038279230e-01, 1.573320882642233e-01, 9.597034895282387e-01, 1.627373341924020e-02, 2.856191316266287e-02, -1.821363313069482e-02, 6.175195512772754e-01, -7.440488379994714e-04, -4.502595457111243e-03, -1.172024748135961e-03, -6.203608587849779e-01, 6.053271518651828e-01, 5.746966469539778e-02, 1.357544290634883e-01, -1.540880487753322e-02, 2.672978120635073e-03, 2.907778793026973e-01, -2.493588827320280e-02, 1.160946061281866e+00, -1.341308305483362e+00, 9.445966115487770e-03, -2.063357725439160e-03, 3.065265640137274e+00, -9.004233679588669e-03, -7.142952418512103e-01, -2.380313487328205e+00, 7.095402719018784e-02, -4.096615861143414e-02, -8.148767877356537e-02, 2.012381494496679e-01, -2.651931437600367e-01, -2.797858400995573e-01, 1.451771731737868e-01, -1.305572970086154e-02, 2.966942086242038e+00, -8.785936732880403e-02, 4.416681394947530e-02, 4.171792917295421e-04, 1.053815640221387e-01, -3.609591404014569e-02, -2.280070287996965e-04, -5.856170738578892e-03, -6.922483933898399e-02, -1.409362846107629e-01, -4.407189065859392e-01, 6.164222727121309e-01, -2.005934515210212e-03, -1.065837718898862e+00, -5.487189333912946e-02, 6.262423016296432e-01, -4.864287534893082e-02, -1.079636626679087e-02, -2.420669347913374e-04, 2.667688389403902e-01, 4.730158649552720e-02, -2.793433395735212e-04, -6.705657285652454e-02, 8.890684164640207e-03, 3.671238884309174e-01, -1.372287941399460e-01, -8.752889988776349e-01, -6.457924166599679e-01, -3.062465679478088e-01, 9.223714305928127e-02, -5.400789687517849e+00, -5.546904191521090e-02, 9.379080479175468e-02, 6.655894853109613e-02, 4.065094646964874e-01, -1.386098569036849e-02, 1.882215380938359e-02, 1.720927888205653e-03, 4.458444072643041e-01, 5.417748552458647e-01, 1.252795528050357e-01, -3.786471279042870e-02, 1.058641003484469e-02, 2.479504608475542e-03, 5.241553526871553e-01, -6.182973598333729e-02, -5.960056527733546e-01, -1.210020353763083e+00, 2.577512759850423e-03, 5.079804082759900e-05, -1.337360931281041e+00, -3.264609650506736e-02, 2.899660081158574e-01, 6.851356455223370e-01, 2.689097571748027e-01, -2.360255023659019e-02, -4.029630309539078e-02, -3.023542812238483e-01, 6.680794452315074e-01, 1.960869552124459e-01, -5.248034015201275e-01, 3.222297016815501e-02, -6.620964204424604e-01, 2.211939186286205e-02, 5.538382163569456e-01, 2.726512678374410e-02, -5.066483066315287e-02, 1.810306739385907e-02, -2.607430685603864e-02, 2.914552011148277e-02, -1.592952669598084e-01, 2.245756547774179e-03, 7.823665678314569e-01, -4.396433853391822e-01, -2.376905839076441e-02, 4.972070544938101e-01, -1.048984070085148e-01, 6.433042235426621e-02, 4.561489801913091e-03, 2.655595011333026e-02, -1.881896453215779e-02, -5.834936481352696e-01, 5.641464140178554e-02, -2.115328195307499e-02, -5.561038605319819e-02, -1.289429472138855e-03, -8.555817359637882e-02, -8.819812011382361e-04, 9.805547677857446e-01, 9.598019669665117e-02, 4.816903974088077e-02, 5.652570505621904e-02, -1.256359358260046e-01, 3.201411586314930e-02, 3.661196933859927e-02, -9.366838843072260e-02, -1.765422076050166e-01, -2.628688653426391e-02, 4.271544050569335e-02, 1.926176521788903e-02, 5.887014281846800e-02, -1.470646251229915e-02, -8.331116154310444e-02, -2.015264608815855e-01, 4.619588562411811e-02, 1.903161679690271e-03, -6.254190936128341e-01, 4.817049472543603e-02, 2.701003754838813e-01, 5.542297296071046e-02, -4.821953377308620e-03, 2.030363598778862e-02, -3.320930018632617e-01, -3.255429237556558e-03, 1.025243493370598e+00, -3.161521088401112e-03, -4.334738572696369e-03, -5.473449291690800e-02, -6.043886638124345e-02, 1.369456584640595e-02, 6.303892257695451e-02, 1.075022803643271e-02, -2.442557794247559e-01, -3.239672846596801e-02, 4.693816011532658e-01, 2.499481527731050e-02, -1.076442708546655e-01, 2.312639729736217e-02, -4.890753714639512e-02, -4.448943762081060e-03, 2.823116602796851e-02, 3.081626591101055e-02, 5.306617861185736e-02, -1.185298896438589e-02, 5.947326805645821e-01, -4.951410236898411e-01, 3.519944487706240e-02, -4.194770793752087e-01, 8.977766594037097e-02, 6.266359373966467e-02, 2.673050337317160e-02, 1.256846575213129e-02, 4.299739362437606e-03, 5.547941540707612e-01, -1.050382032920758e-01, -2.233997866466443e-02, -4.304960711344160e-02, 6.920084120899140e-03, 8.705660120551578e-01, -1.482610302709281e-02, -4.584407224870131e-01, 1.193921542474090e-01, 9.017345307151572e-02, -1.066965037668198e-02, 1.514927289451519e-01, 3.277034528805253e-02, 9.594084798586719e-02, 3.112982305917485e-02, 1.496744960762402e-01, 2.239423173424554e-02, -4.603273284856364e-02, 1.465356811718266e-03, 3.090739689058835e-01, -1.359203686763431e-01, -1.897100107438419e-01, 2.143770760840013e-01, -3.955339144783997e-02, 2.232078559011863e-02, -2.655205710628462e-01, 1.468337977463791e-01, 5.312238676822402e-01, 2.306930008775912e-01, -1.972296669629678e-02, -3.453477993841944e-02, 2.947169076007924e-01, 4.289839259987930e-03, -7.058298043241547e-01, 1.411822374344396e-01, 1.139485483391002e-01, -1.378316323367882e-02, 2.996997417056905e-02, 1.134003167502668e-03, 5.621539976593518e-02, 8.150633410695815e-03, -6.246237107423623e-01, -6.780943825615428e-02, 1.669196159239207e-01, -1.984359993839907e-01, 2.337607139110437e-01, -1.132445928519323e-02, 7.182541020341436e-02, 7.086376777441181e-04, -3.500962432154025e-04, 2.182663080841672e-03, 2.317630661160643e+00, 1.407174544747583e-01, -8.778833265860880e-01, 3.440850472800153e+00, 1.242491012492166e-03, 6.749256573756351e-01, 1.392602315828635e-01, -5.285064994921106e-01, -2.727560540918236e-03, 1.594425294651738e-01, -1.589848711663766e-03, -1.489440474925534e+00, -3.424428627354266e-02, -1.672958450353865e-03, 7.431628146556325e-02, -2.569700732425893e-02, -5.368900279272697e-01, -2.162613950846662e-01, -4.389317530498946e-01, -5.546708704013155e-02, -3.915542520164404e-02, 6.606513486346471e-03, -4.677680263718957e-01, -3.355614869117321e-02, 5.006067433578983e-01, -7.339630924170051e-02, -8.361851630884791e-02, 1.131467850651690e-02, -3.281846878837295e-02, -5.892518517796780e-03, -2.218330273130075e-01, 9.503742176611892e-02, 9.738178890785285e-02, 3.193046421341148e-02, -1.036363153503917e-02, 2.109286513702890e-04, -2.894598773662941e-01, -4.621092250036424e-02, 3.164153937178884e+00, -2.677288850585673e-01, -8.035938264250435e-04, 2.897797224783516e-03, 5.608612250481934e-01, -2.247751653948549e-01, 1.597952457708169e+00, 7.326128134695411e-01, 1.125650676049371e+00, -7.907908521704003e-02, -2.339057696213743e-01, -2.605046135927228e-02, -1.139847339553409e+00, -1.081303435511911e-02, 6.536046686130255e-01, -3.763809028487276e-02, -1.019381994516780e+00, 1.965072320738966e-03, 4.397517889282858e-01, -1.425566361720662e-03, -9.810370500570991e-01, 3.988179207899656e-03, -9.021555984278765e-03, 5.786833658671642e-03, -1.799420292764344e+00, -5.697350238899416e-03, -4.475904037849700e-01, 1.148688101518609e-01, -3.003483425908329e-02, 3.851971631763223e-01, -1.269318743141445e+00, 6.611240116830838e-02, 1.632368078767500e-01, 7.788898942090728e-04, 2.967287647374705e-03, -7.409330063415938e-01, 2.512425068439296e-01, 3.295840046020698e-02, -7.724100325149304e-03, -1.110410374430591e-03, -2.383817981909049e-01, 1.728966592578308e-01, 4.640596799302906e-01, -8.643274194318948e-02, -4.634973859228139e-02, -3.891913358313737e-01, 3.528977404002624e-01, 4.517243545093139e-01, 7.446400097585263e-03, -1.302305563652431e+00, -2.456839722215691e+00, -6.687374704221487e-02, 1.994932394922664e-01, 1.063297852587919e-04, 2.038449526983730e+00, -3.264783000059452e-02, 1.243597235869823e+00, -9.044501853502104e-02, 1.118513797036171e-02, -2.788059110391075e-01, -1.627238799047551e-01, -8.872485109356308e-01, -7.742990105970169e-01, 6.198568773557114e-02, -8.115964607845597e-03, -1.742844276697178e-01, -9.588402518420375e-01, -8.950069218392530e-04, 1.448642753337438e-01, -2.484635721272136e+00, 9.351237462473842e-03, 6.013470841589264e-02, 1.198847338313983e-01, -1.783083703982064e-03, 1.146885326682845e-01, -9.866130050162577e-04, -6.097172087311726e-01, -5.699741775193078e-01, -3.492072643428161e+00, 3.249659680748579e-01, -1.346240598221266e+00, -2.766926483105543e-03, -5.342659626377108e-02, 8.406889244713433e-03, 1.658548636545083e-03, 1.178199266350360e-03, -1.908599287726471e+00, 4.964370894317352e-02, 1.754072196885621e+00, -2.704729684755720e+00, -2.002336298296977e-04, -3.604038187706820e+00, 4.979554284337153e-02, -6.176639266705726e-01, -6.205363001664661e-02, 3.171267217358471e-02, 1.138170698690141e-03, -5.792375867713843e-01, 1.463698216687435e-01, 1.265793721048456e-03, 1.835926732984933e-02, -9.565049059169061e-03, -1.753574091856326e+00, -1.322152056588493e-01, 8.369525127946056e-01, 3.021521864323127e-01, 1.112878980025618e-01, 1.080407527117506e-01, -1.285537871248937e+00, 2.820806995123489e-02, -2.973995423832561e-01, -1.476287557940891e-01, 7.299617344288688e-01, -4.815221985414948e-02, -1.539726433804626e-02, -4.275874434181399e-05, -7.788853809680261e-01, -5.791210766422537e-01, 1.478099356614941e-01, 4.997776067929623e-02, -1.012527756827649e-02, -4.490114287190866e-03, 1.040939612152048e-01, -7.244082936035004e-02, 1.117946337869876e+00, 5.805732768882331e-01, -2.132218970165402e-03, 7.593105734355111e-04, -1.850827371001962e+00, 1.743590386269324e-01, -4.644228267480733e-01, -3.296047574815744e-01, -6.415697421643616e-01, -7.746862532868455e-02, -1.753192330348178e-01, 8.261686349952853e-02, -1.324764874402670e+00, -3.215315983137512e-02, -2.792270647795160e-01, -7.198798446084620e-02, 2.226550239974559e-02, 4.563357372275617e-03, 9.918621911384716e-01, 2.588304111771410e-02, 3.750239048624660e-02, -4.650890083034172e-03, -5.898868908584831e-03, 7.807214542793878e-03, -6.425752233028363e-01, -5.762016300456738e-02, 9.206794765379764e-02, -6.627500409376437e-01, -1.933907431608155e-02, 1.012373636482190e+00, -4.142179612937244e-02, -1.698196386965861e-01, -6.114361540439865e-02, -2.732317238751191e-02, -1.246328563871484e-03, -2.051718984446423e-01, 1.457069335138158e-02, 1.618213846318052e-02, -3.262177109841442e-02, 1.261382836040549e-02, 1.462686314651073e-01, 3.160987628583222e-02, 1.607889050650822e-01, 4.090985788343716e-02, 8.530830567416491e-02, 3.483112211747279e-02, 2.739733850328607e-01, -6.124175992409207e-02, -1.648518945024910e-01, 4.874293228160172e-02, -2.326114829783094e-01, -1.439220627340572e-02, -8.621480818815487e-03, 1.816855166761703e-02, -9.635538615306550e-03, -9.474951345023645e-02, -8.543650151098903e-02, -1.886919090102595e-01, 3.511564356772236e-02, -2.267723249314271e-02, 2.959939748492309e-02, 4.646376481338788e-02, -5.380351252781280e-01, 2.479093356765475e-01, -1.399396696715356e-02, -5.745738821669352e-03, -3.354631954816531e-01, 2.660130570812339e-02, -3.744634448468742e-01, 5.596113197549948e-02, -2.469589041189257e-01, 4.210874089275727e-02, 1.106874125844043e-01, 2.773340122674793e-02, -2.009678906230734e-01, 3.121540071994616e-02, -3.995885909258017e-02, 5.251376361281947e-02, -5.343990836256589e-01, -1.333186312857893e-02, -3.648344057486968e-02, -6.027430463087145e-02, -7.103261050035254e-03, -8.058624070545823e-03, -1.814951001430193e-03, 1.898249454136581e-02, -2.434428224189612e-01, -8.954127774626197e-02, -2.838419943308310e-01, 3.740088010407500e-01, -5.030243566082312e-03, -4.219909481847584e-01, 6.644757587431866e-03, -4.044985995103618e-02, 4.826978891077818e-02, -6.155504412774271e-03, -2.564812787480035e-03, 5.824829112743324e-01, 3.851708116292699e-03, -1.845355822842587e-03, -7.522706133868770e-02, 5.326629722828456e-03, 3.530284798426510e-01, -5.359241276231578e-03, -7.943271665929713e-01, 1.455580806599541e-04, 3.870233758424756e-02, -7.107857234983504e-02, 2.542671526023310e-01, -1.212869487158952e-03, -2.864057030343430e-02, 6.951506706810977e-02, -2.434045755731212e-02, -2.171872188374117e-03, -2.792445539357734e-03, -2.044853516525911e-02, 5.896868378969097e-01, -5.522417234752479e-02, -3.904044828416774e-02, -3.548390348584042e-01, 1.195232423579224e-01, -3.844014306092288e-03, -8.826157198787966e-02, 2.324583085920229e-02, -1.169292544302797e-01, 1.004265738767567e-01, -3.527077968285026e-03, 5.094164626784192e-03, -4.441631453238029e-01, -1.648216301664709e-04, 1.500869084133802e-01, 1.787081364081720e-01, -2.648473990200502e-02, -4.675206019999706e-02, -1.345512560820570e-01, -1.206277585191718e-02, -4.112733660073677e-02, 1.758280299221906e-02, -4.122162385667398e-02, 4.831209520132407e-02, 3.125436504622622e-01 };

theta2 = { -4.681097172966038e-03, 3.873987688213609e-03, 5.651541825710441e-03, 7.516398578663715e-03, 4.188686272022755e-03, -5.420886742251522e-03, 1.645038182948392e-02, -3.565207316682262e-03, 1.567085340042967e-02, 4.105204344381021e-03, -7.261793735823108e-03, -3.948089706158554e-02, -2.847673430554983e-02, 1.992409583846194e-02, 3.102836223786604e-03, -6.025267760918994e-03, -2.077143098119519e-03, 9.561991396836288e-03, 5.857521956586932e-03, 4.747273310667786e-04, 4.394644126429145e-03, -1.031901790135007e-02, 2.418210198883934e-03, -1.622248357969865e-02, 3.023886303455862e-03, 9.014716461247517e-04, -4.175646437591368e-02, 2.571983590294981e-02, 1.243529498193873e-03, 1.039355790329568e-02, 5.930352971212425e-04 };

W3 = { -5.516356679934454e+00, 5.200320397767753e-01, -4.689445730414280e-02, -1.044941738428322e-01, 4.605007472667124e-01, 4.242079892835774e-01, -1.043995317143338e-01, 4.923605946685692e-02, 9.956620279799171e-02, -9.218523436770369e-02, -3.622849366156606e-01, 1.001666656816373e-01, -7.061539721544691e-02, -2.203509339921301e-03, -1.604000346554732e-02, 1.039063948878520e-01, 2.565099697086555e-01, 9.039024882484016e-02, 7.898617071411853e-01, 3.463663374584407e-01, -2.580383035123807e-01, -1.237400009747178e-02, -3.605054073457185e-02, -1.738900640240282e-02, -5.576085373233922e-02, -2.696206479970501e-01, -5.708993646704854e-03, -6.329594436148225e-02, 4.548721923096406e-02, -1.573805380616415e-01, 2.555413012813357e-01, 4.242392433739120e-01, -9.688446737318904e-02, -1.542213953054987e-01, 1.213341203990467e-01, -4.115160290531990e-02, -5.816253343654336e-01, -1.505122934775358e-02, 1.689510343557294e-02, -2.926895325528937e-02, -1.182513014464961e-01, 2.402177431534765e-01, -3.341715680339853e-02, 4.875948475023915e-02, 4.869597028519362e-03, 8.707171904062252e-03, -7.528187509532392e-04, -1.573222263833356e-01, -9.577940555559347e-02, -3.035930716384725e+00, -2.378989193549228e-01, -5.644543112213428e-02, 7.886438827882834e-02, 2.227075393051942e-02, 1.075360530495559e-02, 2.179263147595798e-01, 6.798071825204112e-02, -8.667710531126027e-03, 2.051522937336258e-02, 1.321940968162999e-02, 1.251399573118274e-01, 1.214809796160896e-01, 7.506299844933129e-01, 2.870017535104073e+00, 1.989517694264986e+00, 2.438213704221110e-01, 4.595946858221656e-01, -1.522069451010890e+00, 6.388331383360794e-02, 6.567921767042657e-03, 9.115532533424166e-02, 4.958296732310795e-01, 1.413524256732700e+00, 1.145583749381743e-01, -5.222975687276020e-01, 3.559509355313455e-02, -1.760997971256417e-01, 2.420719603559991e-01, 1.462448603063721e+00, -2.073408732416017e-01, 1.619676449964513e+00, -1.254607609325150e+00, -2.471134860065385e+00, 9.186276862456420e-01, -6.993149397460384e-02, 3.692198792666383e-02, 3.949314523909000e+00, -2.302787406234970e+00, -9.917961464101063e-02, -3.541998370517052e-01, -8.476138623376740e-02, -3.972964280407547e-01, 1.839708919189076e+00, -3.302561215810920e-02, 1.305477688651161e-02, -1.249125356176600e-01, 3.043585221527568e-02, 9.264142199170693e-02, -3.768772492197685e-02, -1.195735089302163e-02, -6.861570817915180e-03, -3.531150218878629e-03, 9.590474037371672e-03, 1.624599228071592e-01, 1.263215237472702e-03, -1.620659191573521e-02, 4.952204138331887e-03, 5.953820638595650e-03, -1.591727867928532e-02, -7.100946427890949e-02, -2.293731312852582e-02, 3.734224267249165e-02, 2.229816033695629e+00, 2.080799265695785e-02, 5.131139557723449e-02, -1.677311293693593e-02, 3.029450504031766e-03, -8.254544307194067e-02, -2.135485086407742e-02, 5.498045576101313e-03, 2.629239457660338e-02, 5.198720007394421e-03, -2.915518979548853e-02, -4.718704470079813e-02, -1.160071690842331e-01, 2.117334361582972e-01, 2.741111713774456e+00, -2.090479437794621e-01, -2.311874052413095e+00, 5.243864781881996e-01, -2.728193234038395e-02, 6.548211512870172e-01, 8.629531748375928e-02, -1.337839665346318e+00, -5.670606932578590e+00, 6.305239968916448e-02, 1.446445772173240e+00, 1.143456697433007e-01, -1.454156110132546e-01, 4.623059452966836e-02, 1.267823874558024e+00, -1.860631294739257e-01, -5.791175370571676e-01, -1.330947768845634e+00, 1.315085001812539e-02, -1.623348740800221e+00, 5.253765925005474e-02, -9.925377436518622e-02, 6.297663905003700e-01, 1.304135285812514e-01, -1.295278656129007e-04, -6.417556674417448e-01, -4.122913375913364e-03, 5.756194153576407e-01, -2.553037141143334e-01, -2.318418713028590e-02, -6.283222716631762e-02, -6.311805093351343e-02, -1.286228691494305e-02, 3.734449887382590e-02, 2.344245760649218e-02, -1.558672560006930e-02, 9.245059599165962e-03, -5.930033903310488e-03, 1.325691135526001e-01, 1.790583812367648e-01, 1.274106677898500e-02, -2.490584330090092e+00, -9.252453737130309e-03, -1.089424653101065e-02, 3.334822050263238e-03, 4.407181944306882e-02, 3.091374465789710e-03, 8.857417543878679e-03, 3.993624966512295e-03, -1.115035742558628e-02, 1.903420214458434e-01, 1.216947710176003e-02, 6.429168064042872e-03, 4.289931975610280e-02, -1.226816256039781e-01, -9.428625380744175e-04, -2.314520070470511e-02, 4.410094104371418e-03, 7.600298475146568e-02, 8.843384030254585e-02, -2.578377462561542e-01, 2.437536158540534e+00, 1.124352322901799e+00, -5.356235446562015e-02, 1.952688462378598e+00, -2.246227930310182e+00, -6.057533335664932e-02, 1.916895114705428e-01, 3.013687536797977e-01, -3.208566147691510e+00, 1.456657033317878e+00, 8.008005590031131e-02, 1.412653943007718e+00, 1.006603188855042e-01, -4.054578602114862e-01, -1.035719699107671e-01, 1.360036006516349e+00, -1.276659452142268e-01, 5.494056276194743e-03, 6.288084826721667e-01, 1.522887273169112e-01, -3.065468223249055e+00, -1.195413629673929e-02, 1.622108570999387e-02, -1.018524339015672e+00, 4.148618456809472e+00, -5.270209085790526e-03, 1.294119155603845e+00, -4.292656946342231e-02, -1.530490275852237e+00, -5.852291268328881e-01, -4.135849006593857e-03, 3.160553739625479e-02, 7.079793706164864e-02, -2.916166138823899e-02, -8.078241392486460e-02, 3.382704894770906e-02, -2.172423337840891e-03, -2.918735459627662e-02, -2.194079808393774e-03, -2.758060837163634e-02, -1.039128440256875e-01, 2.027625743033131e-02, -1.475467153823252e-02, 8.150659628776048e-03, -3.057659233010895e-02, 1.241915917105222e-02, 9.725158257002997e-02, -2.714482650181954e-03, -3.343179081001322e-02, -3.312878655410900e-02, 4.797626556893722e-04, 8.594011750038527e-02, -1.458251552589537e-02, -3.176660528336470e-03, 5.066368960867497e-02, -7.516338277834712e-02, -1.452670536276358e-02, -2.300176619770479e+00, -1.200589097356030e-02, 2.669444640160268e-02, 6.075926483690253e-03, -1.392154631049473e-02, 8.107673577763976e-02, 9.289297086093117e-04, -5.051330123862036e-02, -1.241397467081461e-01, 4.881366542129235e-02, -1.060636957153584e-02, 2.701409912013892e+00, 1.608898208440613e-02, -4.644680952670978e-02, 7.891652563583366e-02, -5.969422957610424e-03, -1.041669801111873e-02, -3.503569930925119e-03, -5.754725415856607e-03, -1.290722066061060e-02, -5.720282535750969e-03, 3.329077759592237e-02, 6.263899709942693e-03, 1.330661316200181e-02, 1.913985734864596e-02, 2.407686728362626e-02, 2.488825646413029e-02, -4.232150238479834e-03, 2.453715325637921e-02, 1.856605356375551e-01, 1.803643978479117e-02, 1.175375928018536e-01, -2.020960359716559e-02, 8.419124316907088e-02, -2.900356457776385e-02, 7.298379890135451e-02, -2.935692011186979e+00, 1.052385461210733e+00, 5.035705262047637e-01, 2.858180520178148e+00, 8.469086468547412e-01, -1.343757798170472e-01, -1.953049475093819e+00, 7.358689203116963e-01, -6.379549300479470e-01, -4.845707979716479e-01, 6.888589658829268e-02, -2.984310335244645e-01, 5.700532292126375e-01, 1.332359070250976e+00, -2.266475324027292e-03, 3.726074099366356e+00, -1.398123214755792e-01, 4.124984619691407e-02, 8.593125473430363e-02, -3.895059792650118e+00, -3.251834219180278e-01, 1.155489216510221e-01, 8.790315824826324e-02, -1.986543600399100e+00, -2.665639242880599e+00, -8.091447587680851e-02, -3.596480925195987e-02, -1.611140982557381e-01, 4.895276340717770e-01, -6.382821304687512e-01, -1.352079042678775e-02, 1.174948704103876e-01, 1.852636734133261e-01, -8.623256534181872e-02, -1.080833206306507e-01, -2.301978656223134e-02, -3.881616825785660e-02, 7.367032734771096e-02, 4.657198894222246e-02, -1.825012722124633e-01, 1.051168825824673e-01, 1.852596557055195e-02, 1.578961453204185e-02, 5.064769240827521e-02, -2.313786689178589e+00, -2.148764324311910e-03, 9.768782673324000e-02, 5.302294891296976e-02, 1.524401459275865e-02, 1.656950405445232e-02, -4.583659010618678e-02, 3.852184323016616e-02, -3.379899546029979e-03, 3.461089111631725e-02, 2.942911459436014e-03, 2.131602319999905e-01, 1.638199739629190e-02, -5.065273850799545e-02, -2.841075470585572e-02, 1.243265998591520e-01, -6.686822436791937e-02, -6.501071301562145e-02, 4.560848057418829e+00, -3.416767822665662e-02, -6.815335959730277e-01, 1.822644469105090e+00, -5.700159641453760e-01, -7.796288837348053e-02, 7.712377478502428e-01, -1.292800942169229e+00, -1.907340647380634e+00, -1.557318352592042e+00, 3.820901642636881e-01, -1.495985006145540e-01, 1.704029220138384e-01, 1.256808967135362e+00, -5.694989647642487e-02, -2.429396109148118e-01, 5.773024118942355e-02, -4.875654086040529e-02, 5.219768332637000e-02, 4.586076027835709e-01, 3.036897090092384e+00, 1.347682353750311e-01, -1.263236698114622e-02, -3.029152869232190e+00, -1.030684298866978e+00, 8.481263809350206e-03, -4.898766749030051e-01, -9.562792297433299e-02, 2.654879614185820e+00, -6.557524359918530e-01, 1.699033530273710e-02, -1.911095201884631e-01, 8.088653902230980e-02, -3.550286982329450e-02, 8.501056930987997e-03, 2.709601859557087e-02, -7.790245822773940e-03, 4.578011345578417e-03, 2.282364917627234e+00, -1.653556957060099e-03, 3.706384009874544e-03, 1.189176418801913e-02, 1.233875609939916e-02, -2.131986217722230e-02, -2.596425403065610e-02, 1.875973178639575e-02, 1.043758920210027e-01, -5.080046060940947e-02, -1.162494465328437e-02, 2.558077978855505e-03, 1.286148325946690e-02, -9.073801713106228e-02, 1.062969879252957e-03, 1.318800992204787e-02, 1.915366645085019e-03, -7.011194130253641e-02, 2.923778005175829e-03, -2.673184662500481e-02, 1.779873988996846e-03, -1.361749874150179e-02, -5.220929581524586e-02, -2.467104370788145e-02, 8.141698212399724e-02, -1.123128191196475e-02, -1.117752574520463e-02, -5.739903356265260e-02, -1.209372255250428e-01, 8.940650807607238e-03, 7.204573799313998e-03, -4.709361190169187e-02, -5.464851103038131e-02, -2.687033472176634e-04, 1.933803849181314e-02, -1.519193364980759e-02, 2.276463445653415e+00, -8.983921955583334e-03, 1.799290704990957e-03, 8.358804678493960e-02, 4.517812846398125e-03, -1.098765368222323e-02, -9.126415179022698e-03, -1.913602549959163e-02, 9.295203183285357e-02, -2.253043828418583e-02, -2.402221352451121e-03, -6.874963267386269e-02, 7.151674190757799e-03, -2.370220824084105e-02, 6.830319110705859e-03, -8.897521161976776e-03, 3.168872454075142e-02, -9.134254815233556e-02, 2.523775847840022e-01, 2.189516982796627e-01, 1.265000357614477e+00, 3.813333300331104e-01, 9.716463449918549e-01, 5.755169462197010e+00, -4.235434906613042e-02, 2.199955300652899e-01, 1.664955176731091e-01, -2.420820892981566e+00, 2.187452797766546e+00, -1.184385118955247e+00, 1.789316645231163e-01, -1.193517645257741e+00, -5.035612489487818e-01, 1.322399272312175e-01, 1.032651891014004e+00, 5.657141134561808e-02, 1.903483873999324e-01, -5.408732675830006e-02, 9.139522136020214e-01, -1.333443630157832e+00, 6.433370276929542e-01, 2.272055902287008e-01, 2.123159204590766e+00, 8.888237680403474e-01, -9.427868549442577e-02, -1.601311299842358e-01, -2.188930399330685e-01, 1.144409185355139e+00, 9.208757258740249e-01, -1.902306237046251e-02, 5.255235677493810e-02, -5.075266827040253e-02, -1.401695689779318e-02, 9.175607768978646e-02, -1.433373646257809e-01, 2.976239761610089e-02, -1.052295174424868e-02, 1.998496695696379e-02, 1.632301374011723e-01, -8.671680784388905e-02, 2.331005699731199e+00, -4.794852887914825e-03, 3.509655726310077e-02, 2.060936170534835e-02, -4.508676396663166e-03, 8.574008716571951e-02, -1.048735759522515e-02, -1.242315945086551e-02, 1.579318102922135e-02, -1.324322830629080e-02, -3.832167255736095e-03, 7.260343545333472e-03, -2.622649684599309e-03, -1.355542876750486e-01, 3.459699285191803e-02, -3.371540067239240e-03, -4.032221922199494e-03, 7.069355252156191e-03, 6.241422302351751e-02, -3.028390149209018e-02, -2.632596074138317e-01, -1.215906905357522e+00, 5.086067043503035e-01, -3.585341486767067e-01, -2.526178768954079e+00, -1.691310640962673e+00, -3.010934066824844e-01, 1.767047695165471e-01, 5.030241041755369e-01, -5.665933151067741e+00, 1.273863511682364e+00, -1.328240109457488e+00, -3.051300697406687e-02, 7.391281748080251e-01, -9.791699779523104e-02, 3.113570823427352e-02, -2.167023980386502e-01, -1.822283232237907e-02, -3.306634227686660e-02, 6.433131663310536e-02, -1.301539649675994e+00, 1.643569427124421e+00, -1.204439811961882e+00, 6.212246816922150e-01, -3.529275106468152e-01, -1.653189136063725e+00, -4.879300603737470e-02, -1.808371380859533e-01, -3.865425113479069e-01, -1.000921669200890e+00, -1.768676013453425e+00, 1.143679942205015e-02, 4.433664400103388e-02, 8.044127623934828e-02, 3.307346804957803e-03, 3.318668887368877e-02, 2.491786928523673e-01, -1.063968478909860e-02, 2.016237014072001e-02, 4.490212458835077e-02, -3.191361768549431e-02, 3.660512214629014e-02, 2.931974774829562e-03, 4.256017095263890e-03, 2.838041158010598e-02, -1.672997385698240e-02, -8.817080626092743e-03, 8.393395050303100e-03, 7.220800811739046e-02, 1.364038805129171e-03, 6.097803777940338e-03, -1.219973549280824e-01, -3.440287925586861e-02, 2.324944342761407e+00, 4.673189718109556e-02, -1.106940025370746e-02, 1.089230984298667e-01, 9.833463118113218e-03, -2.036430749673764e-02, -2.926419394163693e-02, 1.891342829617269e-01, -4.146857568936717e-02, 1.170356770549589e-01, 7.143374704621557e-01, 3.657101205835491e-01, -6.782957898199240e-01, -1.660437083414187e+00, -5.110129452529284e-01, 5.957459155318162e-03, -1.653371731379313e-03, 1.229815760008475e-01, 6.792535418862856e-01, 9.672397996430141e-01, 5.909863178139479e-01, -1.340146202985115e-01, 1.456839097023499e-01, -1.638921142985322e-01, 7.193907510570677e-02, 5.715875063626046e+00, 5.613292704602904e-02, -1.340791939657742e-01, -3.793697931177987e-02, 3.245475140768379e+00, 1.773169342934218e+00, -1.226575825079082e+00, -1.191476320664937e+00, -6.631646308112444e-01, 3.750581174517191e-01, -4.742573415760812e-01, -1.180070774703379e-01, -1.356200083119097e-01, 6.952564453620576e-02, -6.424583709189158e-01, -2.738633314678398e-02, 2.029908657222900e-02, 2.860071790748218e-02, -3.947389516502009e-02, -4.828394739858812e-02, -4.555183201386388e-02, -4.420780710255919e-03, 1.923197850911862e-02, 3.260661301066586e-02, -2.001274353646263e-01, -9.697461398779398e-03, -4.219789043101772e-03, -7.802840096119818e-03, 2.911618562203258e-02, -1.148259256742562e-02, 8.401998374687830e-03, 3.443228161991630e-02, -1.451146158722591e-02, -1.920034642388975e-02, 6.404321636546353e-03, -1.231474664284349e-01, -9.871627528791178e-03, -2.771607831066794e-03, 2.361621088507305e+00, 4.241537832251353e-02, -1.353175011660360e-02, -2.874488728919146e-02, -1.256357838519620e-02, 4.706705533557832e-02, 2.331796577481240e-02, -3.133566152938492e-02, 2.704612032901450e-03, 7.363623650844588e-02, 3.511413921866435e-02, 9.531166861921882e-04, 2.235280966376720e-02, 8.459375602763317e-02, 8.428127006570994e-03, 5.482833448660761e-03, 1.432540753379479e-02, -2.511949074372376e-02, -8.268296533661836e-02, 1.117209975109269e-02, -1.015309670775610e-02, 1.139609046490511e-02, -6.818279850380627e-04, 2.190874747580251e-03, 8.800412356223501e-02, 9.970165429733682e-02, 2.715422400493425e-04, -8.650826117767865e-03, -1.152673113842104e-01, 1.761190636633580e-01, 2.811848400372248e-02, -3.199423942179686e-02, -3.346003415713619e-02, 7.477769081719798e-02, -1.635048813825511e-02, -6.578665855271716e-03, -2.173978435535755e+00, 5.916471617180392e-03, -9.757853342324145e-02, -5.139067639854734e-01, -3.825017568805527e-01, 1.740033955152706e+00, 1.212024797058593e-01, -1.785990376335731e+00, -7.976715704758078e-01, -5.738043757874323e-01, 1.224587242586470e-01, 7.883375122352243e-02, -9.928798124011956e-01, 2.372016193659034e+00, 6.822804215519512e-02, 1.062883055565731e-01, 3.421095313720900e-02, -5.418065354170329e-02, -2.035681388314440e-01, -1.714330699306885e-01, 1.664842221325949e-01, 7.475909780958263e-03, 1.160267608037874e-01, -1.628379403047166e+00, -2.515186932714680e+00, 5.234850632063034e-01, 5.145328531100059e-02, -1.511556358591843e+00, 4.092745109391054e-01, 1.382444927768512e+00, -1.393125596933844e-01, 1.000996876720065e+00, 5.451195694444346e+00, 6.311378227969062e-01, 1.507434589842196e-02, 5.507946973430612e-02, 1.308499230766867e-02, -6.793887556270158e-02, -9.813759680170783e-02, 9.881196393261689e-02, -1.795855725646571e-02, -9.175703191513946e-04, 1.646460943037720e-02, -3.165159195547048e-02, 7.769797609925945e-02, -6.674569504417214e-03, -6.707983646442408e-04, 1.595819722881499e-02, -5.279551068413883e-03, 2.384397635281444e-03, 7.272432441507211e-02, 1.015442213148731e-01, 1.515843856291720e-02, -4.117579714709847e-03, -9.789513603963880e-02, 3.631461962356623e-02, 1.462749446542736e-02, 3.668190183580839e-02, -2.971209634341327e-02, 1.543836524000380e-02, -2.351531661442466e+00, -2.631712506656511e-03, -1.781991491247008e-02, 9.589770688204571e-03, -8.632286429533292e-03, -2.414075040465659e-01, -1.951547664821709e+00, 9.797508095013429e-01, 3.355797390821604e-01, 2.073583028464535e+00, -7.269066142458114e-01, 1.482687603849931e+00, -2.233015559454566e-02, 1.038435232287648e-01, -7.322395995641146e-01, -3.865317044508458e-01, -6.660592886896535e-02, 3.259241628113207e-02, -9.690571384289266e-04, 4.353757900298755e-02, -5.089952609808982e-01, 2.937694564844013e-01, 1.625406452285607e-01, -1.008761739058139e-01, 2.565755927362965e-02, -2.547471459474634e-01, 4.478652879179613e+00, 1.699061729315105e-01, 4.458931048826866e-01, 2.265999112081026e+00, 3.863148282755935e+00, 1.402085209376141e+00, -9.695123557233130e-02, -5.911466331451913e-01, 1.828092030437996e+00, -1.449462594897241e+00, -2.567941932268602e-03, -2.321452462957578e-02, 1.040197011704400e-01, 4.899561413303790e-02, -2.101004277043047e-01, 1.634638376963961e-01, -2.601004225965503e+00, -2.250586286406173e-02, 4.069440255976944e-02, -8.905798726139191e-02, 2.258009899560715e-01, 1.727039031444059e-03, -8.122655964504738e-03, 6.043594606765087e-02, -1.802730667517859e-02, -7.187703651014749e-02, 8.356539281995441e-02, 2.017288693462863e-01, 2.708770075446747e-02, -1.105930867360040e-02, -1.229690670451051e-01, 1.671338495580313e-01, 2.656305120075714e-02, 3.666568007666407e-02, -1.801101105290764e-01, 1.081615675675685e-01, -2.960122183734873e-02, 1.188183992451220e-02, -1.392858818682945e-02, 2.436507372233264e-01, -1.471353306296003e-01, 3.553941749776230e-01, 1.798473582807496e+00, -4.118053533095349e+00, -3.579845547154202e+00, -2.096200279200767e+00, 1.530431122931736e+00, 1.288185126337671e+00, 3.886681409712089e-02, 9.961960807855540e-02, -2.544604397094334e-02, -9.421811848415695e-01, 7.517982307238205e-02, -5.753024273534744e-02, 9.218102221470839e-02, -5.321892524420766e-02, 1.203630122989498e-01, 1.456222692386163e+00, 6.245421913185243e-01, 1.774805214351885e-01, -4.154737747118108e-02, -3.883154373644734e+00, -6.130449259166966e-01, 1.115957415189399e-01, 4.678778459561121e-02, 1.181787631481672e+00, 1.601770727717407e+00, -6.540655428055705e-01, -3.676658930642088e-02, -1.731173281268848e-01, 1.361952206147629e+00, -1.470802058918329e-01, -2.084124009723895e-01, 7.809740894099438e-01, -9.815725684572834e-01, 5.764488724303813e+00, -1.779364392038660e+00, 9.518389025030248e-01, 1.076905166550247e-01, 1.385865755445472e-02, 8.789564343258072e-02, -9.847749550753687e-01, 4.828823901556712e-01, -7.741277671296710e-03, 2.607882587836585e-02, 4.762757025791896e-02, -8.821480625130501e-02, 9.265785969106756e-01, 1.451786979851499e+00, 1.521916017559892e+00, -2.318069656418176e-02, -7.478781282362441e-02, -3.295568674874593e-01, 1.676919432526015e+00, -7.902795037352392e-02, 1.114247111178208e-01, -1.580544046231608e+00, 3.586417486608359e-01, -2.826058759870467e-01, 1.448732043846696e-02, -1.704955478038178e-01, -1.045873629064557e+00, 2.649064335961540e-01, -2.891333693064341e-02, 1.532862062130396e-01, 2.276457337494744e-01, -7.221204592768189e-02, -1.369791018820597e-01, 2.313507652625108e-01, -3.073190373080292e-02, 4.186331163664930e-03, 2.713205828777434e-03, 1.127308261720312e-01, 6.236458100781282e-02, 1.588908474595202e-02, 6.959928984248169e-03, 1.494333818615647e-02, 7.219749241878258e-03, -2.563232057107444e+00, 1.256456830016648e-02, 3.656474053911821e-01, 3.635878891370457e-02, 1.898743612513639e-02, -3.065966540864692e-01, 1.621440705118586e-01, 4.426548304272343e-03, 1.154815614513680e-02, -2.274918067671579e-01, 2.931348567464007e-01, -1.729026236380158e-02, -4.109905912937780e-02, -2.189026978032371e-02, 4.042093078141847e-02, -3.999845405869226e-01, 2.523682832604260e-01, 1.626722811597221e+00, 6.306555033387683e+00, -1.448284816828466e+00, -5.343325070438874e+00, 4.380416502606752e+00, -6.726811266709904e-01, -2.104828549794457e-01, -4.202695086445622e-02, 2.602615703649290e+00, 2.200459585983410e+00, 2.193782278269442e-02, -8.562280668123841e-02, 5.915818171864080e-02, -4.765589609387593e-02, 1.460771366698983e+00, -1.872084353837619e+00, 9.072700611867537e+00, 1.589888788470997e-01, 9.244640944294037e-02, -4.899345303412207e+00, 4.077056541733421e+00, 5.877659636046067e-02, 1.179699893443666e-01, -5.276579708905818e+00, 4.881802983815964e+00, -3.252038494604634e-02, -8.889380274934559e-02, -1.164160372759622e-01, -2.744842003910990e+00, 9.033511671529895e-01, -1.684592634389632e-01, 3.276374190372613e-01, -7.700282760527852e-01, 4.878876039200851e-01, 1.063143691858050e+00, -1.214588497551615e-01, 4.198099829581412e-02, 4.521244160632727e-02, -6.202709530013615e-02, 4.721302822159657e-01, -4.494805144322342e-01, -2.546483932673370e-03, 5.108880321941218e-02, 1.023863971964588e-01, -7.122211814025865e-02, -9.680638782264088e-01, 4.674669859616559e-01, -4.252766329515346e+00, -5.399564399053130e-02, -7.342260537143681e-02, 1.389666100829384e+00, -1.982976956259845e+00, -3.193114949632313e-02, 1.800482717209063e-02, 1.690183771859884e+00, -1.516708179037768e+00, -5.079020641475498e-02, -6.821864706862688e-02, -4.497636724277843e-03, 1.592133441302178e+00, -7.064168658213657e+00, -7.842522657409151e-02, -3.192000309737120e-01, -4.325917638786400e-01, -1.060544678354059e-01, 8.480743340130789e-01, -6.768690062399328e-01, 5.767348345776516e-02, 5.405474396383981e-02, 4.162801460434278e-02, -3.549462915860515e-01, -3.089065208398677e-01, 3.594399223650170e-03, 2.787355548958687e-03, -4.317632504573049e-03, 1.637952693720236e-02, 6.605419823272347e-02, 3.162583896784267e-01, 4.271586545711717e+00, -7.994893204620672e-02, -8.578761264357080e-03, 6.920214475693924e-01, -5.826730350989723e-01, -1.584728338528852e-02, -7.713554171449697e-03, 7.172390718776381e-01, -7.197426804679755e-01, 2.213659150190142e-02, -2.795918998711060e-02, 4.137773266545860e-02, 1.611833100416015e-01, 7.087146109478260e-02 };

theta3 = { -2.587314312816550e-03, 1.263149387263797e-02, -1.938670944587279e-02, 1.506819534571759e-03, 8.930614848947263e-03, -5.142967110828502e-03, 1.069874890012597e-03, 4.461117554509239e-03, -2.937530338670139e-02, -1.269462686017623e-02, -1.397089613138689e-02, 2.267940535981259e-02, -1.688516390458924e-02, 1.009995691450546e-02, -5.095993818065537e-02, 5.908500773063745e-03, 1.398005376235503e-02, 4.679388627918362e-04, -9.826563556498394e-03, 1.664790422269319e-02, -9.947476794843426e-03, 1.905987048805634e-02, -2.085015465173271e-02, 3.557098751065305e-02, -2.511479243592302e-02, 1.143294254705308e-02, -3.160886091854637e-02, -2.481353796677534e-03, 6.390274250754066e-02, -2.666895646994289e-02, -1.264933715220511e-02 };

Third-octave GEQ design filter bandwidths

f_B_m = { 9.178, 11.56, 14.57, 18.36, 23.13, 29.14, 36.71, 46.25, 58.28, 73.43, 92.51, 116.6, 146.9, 185.0, 233.1, 293.7, 370.0, 466.2, 587.4, 740.1, 932.4, 1175, 1480, 1865, 2350, 2846, 3502, 4253, 5038, 5689, 5570 };

Constants for feedback matrix calculation

numDelayLines = 3:

m_hasPositiveFeedbackGain = {1};

m_hasNegativeFeedbackGain = {0,2};

m_alpha = -0.1666666667;

numDelayLines = 7:

m_hasPositiveFeedbackGain = {1, 2, 4};

m_hasNegativeFeedbackGain = {0, 3, 5, 6};

m_alpha = -0.0923495156;

numDelayLines = 15:

m_hasPositiveFeedbackGain = {1, 2, 3, 5, 6, 9, 11};

m_hasNegativeFeedbackGain = {0, 4, 7, 8, 10, 12, 13, 14};

m_alpha = -0.05;

numDelayLines = 31:

m_hasPositiveFeedbackGain = {1, 2, 3, 4, 6, 7, 9, 12, 13, 19, 20, 21, 24, 28, 30};

m_hasNegativeFeedbackGain = {0, 5, 8, 10, 11, 14, 15, 16, 17, 18, 22, 23, 25, 26, 27, 29};

m_alpha = -0.0265555905;

numDelayLines = 63:

m_hasPositiveFeedbackGain = {1, 2, 3, 4, 5, 7, 8, 9, 10, 13, 14, 15, 17, 19, 20, 25, 27, 28, 29, 33, 34, 36, 37, 39, 42, 46, 49, 50, 53, 55, 57};

m_hasNegativeFeedbackGain = {0, 6, 11, 12, 16, 18, 21, 22, 23, 24, 26, 30, 31, 32, 35, 38, 40, 41, 43, 44, 45, 47, 48, 51, 52, 54, 56, 58, 59, 60, 61, 62};

m_alpha = -0.0138888889;

Dispersion filter coefficient template

T_1 = { 1.8851, 1.3414, -0.94643, -0.0022999, 0.98879, 1.4412, 0.90585, -0.20469, 1.8207, 0.59234, -0.49456, 0.75898, -0.71271, 0.15531, 1.4836, 0.55975, 0.92137, 0.25426, -0.42576, 0.49106, -0.12198, 0.17224, 0.88671, 0.21324, 0.14227, -0.93968, -0.066212, -0.0932, -0.66966, 1.9493, 0.91102, -0.36128, 0.66588, -0.68229, 1.4441, 1.9007, 0.28748, 0.78619, -2.142, -1.5365, 0.29466, -0.6012, -1.1083, -1.2338, -0.47357, 0.21661, 0.25799, 0.50719, -0.30746, 0.22622, 0.60989, 0.46916, -1.1549, -0.31592, -0.11128, -1.3765, 0.77415, -0.86679, 0.30029, 1.409, -0.32353, -0.44088, -0.92876, 0.25209, 2.9322, -0.44763, -0.21962, 2.0233, -1.2859, -1.0554, 1.3965, 0.88539, -0.99837, -0.80886, -0.82709, -0.50952, -1.2025, 0.82418, 2.0891, 0.49768, 0.35544, 0.55602, 2.4414, 1.4896, -1.9328, -1.3886, 0.62531, 2.3308, 0.21226, -1.5505, 1.4585, -0.42583, -2.1545, -0.23733, -0.0047392, 0.99103, 0.15236, 0.41017, -0.71493, 0.67415, 1.477, -2.075, -0.91026, -0.80227, -0.64036, 0.50027, 0.4737, -0.037732, -1.4948, -1.1626, -0.97111, -0.23079, 2.158, -0.93907, 0.62561, 2.8619, -1.2238, 0.68249, -0.22967, -2.3132, 0.54088, 0.22264, -0.14336, -1.8643, 1.6966, 0.81241, -2.6671, -0.26208, -0.2364, 0.54183, 1.8319, 0.92692, -0.16398, -0.042301, 1.0194, 0.59384, -0.17035, 0.42828, -0.46514, -1.5001, 1.1012, -0.67172, -1.6165, 0.16067, -0.43493, 0.36332, 0.17062, 0.31841, 0.010483, 1.7787, 0.46583, -0.74559, 0.1499, -0.062328, 0.036548, -0.71394, -1.2466, -0.28818, 0.49365, 0.096309, -1.5605, -0.4777, 1.3364, -1.2375, -0.53021, 0.86708, -0.70674, -1.1301, -1.0483, -1.0366, 1.2537, 1.5829, 0.42856, 0.41962, 0.33163, 0.87747, 0.79698, 0.58817, 0.42938, -0.54925, -1.7494, 0.57535, 2.1975, -0.30985, 0.19281, 0.53724, 0.97838, -1.1005, -0.45414, 0.42717, -0.5864, -1.2376, -0.89444, 1.3916, 1.0278, -1.2451, -0.31968, 1.004, -0.10647, 0.10862, 1.2345, 1.581, -0.24568, -1.2426, -0.16336, 0.18966, 0.93218, 1.4373, -1.0878, -0.57004, 1.887, -0.52739, -0.74903, 1.1521, 1.0992, 1.1928, -0.80639, 0.025742, 1.0172, 0.58679, 0.094761, -0.2504, -0.40399, -1.5525, -0.1376, -1.1622, -0.50813, 0.063796, -1.7861, 1.0683, -0.086027, 0.61956, 1.3082, -1.755, -0.95426, 1.1732, 1.513, 0.24195, -0.68367, -0.1487, -0.87765, -2.0024, -1.1004, 1.1408, 2.1504, 1.5583, -0.50169, -0.51793, -0.036382, -0.57687, -1.822, -1.6994, 0.28333, 0.048964, -0.12792, -2.4738, -1.5254, -0.22522, -0.46277, 1.2754, -1.6316, -1.4512, -0.29362, -1.5579, 1.7222, 1.0675, 0.39094, 0.80198, -0.019308, 0.32201, -2.0857, -2.1224, -0.1415, -0.25619, 0.15049, 0.68237, -0.22002, 0.50058, -0.2647, 0.95616, 2.1124, -0.47257, 0.53084, 3.0057, 0.52129, -0.71536, 1.6155, -0.13791, -0.46119, -0.93999, -2.8265, -1.4292, -1.1573, -0.26923, 0.39585, 0.39784, 1.511, 1.3157, 1.3648, 0.34196, 0.14936, -0.64408, -0.40062, 0.59027, 0.91135, -0.45745, 0.91117, 0.67185, -1.5215, 0.17215, 1.3194, -1.1268, -0.89613, 0.43674, 1.2104, 2.2666, -1.2025, -0.68112, 0.93081, 1.3322, -0.37393, -0.34653, 0.074589, -0.23802, 0.6611, -1.2193, -0.068246, 0.19256, -0.56603, -1.5555, -1.4625, 0.49081, 0.22211, -0.58461, -1.3616, 0.22747, 1.273, 0.13213, 1.2471, 0.42381, 0.44023, 0.30176, 0.1964, 0.63784, -0.46852, -0.78287, -0.51291, -1.1121, -1.533, 2.2722, 0.30738, -0.051575, 1.1377, -1.3299, 1.199, 0.27247, -0.16592, -0.32467, -2.1976, 0.79707, 0.45171, -0.11396, 1.5281, 0.74011, -1.3266, -1.2091, 1.9386, 0.77081, 1.2352, 1.1044, 0.79787, 0.81993, -0.63923, 0.0014934, 1.0363, 0.26782, -2.1945, -0.82959, 1.1231, -0.91429, 0.56731, -0.3239, -0.90184, 1.216, 0.63775, -1.0125, -0.56066, 0.30611, -0.26593, 0.66461, 0.96967, 1.0736, 1.6628, 0.81802, -0.22736, 0.20303, 0.89797, -0.4333, 0.69114, -0.2852, -1.4395, -0.32792, -1.0594, 0.036372, 0.31747, 0.16395, -0.62495, -0.2948, 0.52609, -0.72596, -1.4849, 1.3737, 2.2942, -0.83944, -0.33261, -0.67935, 0.2632, 0.64378, 0.070675, -0.83187, -1.3621, 0.55969, -0.46518, -1.2697, -0.92838, -1.1305, -0.35009, -1.5699, -1.2397, -0.024887, -0.746, 0.78995, 0.11261, -0.7189, 0.27577, 1.6429, -0.04819, 0.48135, 0.753, -0.89835, -0.93653, -0.67369, 0.29632, -0.41486, -0.78646, -0.16357, 1.6531, -1.6816, -0.023137, 1.2885, -0.22256, 1.3262, -0.31439, -0.83153, -0.8038, -0.16468, 0.73468, 0.80747, -0.93496, -1.2389, 1.3676, 2.4885, 0.0075579, -3.3601, 0.42406, 0.95879, 0.18042, 0.048067, -0.73255, 2.1972, 0.046881, -0.25773, 2.6769, 1.0329, 1.0814, 1.1006, -1.2349, -0.29112, 0.92271, 1.9965, 0.51026, -1.5237, -0.034136, 0.7268, 1.1458, 0.03347, 0.11994, 1.1223, 0.22669, -0.15446, 0.47807, 0.33524, 0.036938, -0.12337, -1.1922, -0.082579, -0.54891, 0.01796, 0.73568, -0.6512, -0.39291, 1.2906, 0.47413, -0.5478, -1.2419, 0.45106, 0.87875, 0.44776, -0.32967, 0.077977, -0.18238, -0.14245, -0.31018, -2.4049, -1.9576, -1.5272, 1.4696, -0.066605, 0.35396, 3.6372, 0.30864, -0.60084, -0.74512, -1.0826, 0.45355, 1.0313, -0.27307, -0.41372, 0.66811, 1.3798, 1.6432, -0.61296, -1.8251, -1.3808, 0.15039, 0.44825, 0.1049, -0.19996, 1.5007, -0.074169, -0.47377, 2.324, 1.244, -0.9842, -1.3003, -1.2345, 0.021463, -0.19201, -0.057094, 0.47427, -0.18103, 0.36028, 0.89696, 0.0002241, -0.34101, -2.0678, -0.35073, 2.0354, -0.41844, 0.17491, -0.87469, -0.98279, 1.3595, -0.17279, -0.50881, 0.91399, -1.2112, 0.43936, 1.013, 0.78833, -0.19166, -0.56061, -0.35643, 1.1484, 0.34582, 0.13736, 2.0375, -0.50189, -0.62823, 1.2324, 0.028023, -1.6654, 1.0996, 0.89769, -0.57706, 0.23164, 0.502, 1.5745, -1.1387, -2.0202, 0.081976, -0.61196, -1.0216, -0.03905, -0.03142, 0.16238, -0.54101, 2.2796, 1.8255, 0.35508, 1.0244, -0.090408, -1.7015, -1.1112, -0.23899, 0.45999, 0.32527, -1.743, 0.93785, 0.383, 0.0011851, 1.0963, -1.0476, 0.21869, 1.8064, 0.7521, 0.031923, -1.0341, 0.87797, -1.1746, -0.73876, 0.64769, 0.55481, 0.2874, -0.38528, -0.27698, -0.92587, 2.7035, 0.10127, 0.10259, 0.84137, -1.4944, 1.1726, 1.0888, 0.73451, 1.1671, 0.76776, 0.82625, -0.6032, -1.26, -0.97156, -0.40604, -1.2246, -1.6539, -1.7923, -0.36546, -1.1928, -0.68116, 2.009, -0.98855, -1.2862, 0.70962, -1.3275, 0.54642, 2.3815, -0.03765, -0.92236, -1.9742, -0.086957, 1.5332, -1.1357, 0.43163, 0.27794, -0.653, -0.025559, -0.33756, -0.11669, -1.0553, -0.097422, -0.46209, -1.7241, 0.37894, 1.4668, -3.0278, -0.59254, 0.17241, 0.14883, 0.39733, -0.88392, 0.97449, -0.71204, -0.044824, -0.9691, -0.78373, 0.097861, 1.1341, 1.17, -1.4466, 0.23358, -0.10334, 0.75049, 0.91693, -0.83555, 0.73117, -0.21269, 0.79311, 1.2242, 0.88908, 0.75052, 0.26755, 0.56912, -1.251, -0.16399, 0.34424, -1.2486, -0.66139, 0.8219, 0.68039, 1.4426, -1.5162, -1.3877, 0.21503, -0.042725, 2.7976, 0.71211, 0.013552, -0.3893, -2.366, 0.26273, 1.6795, 0.83604, -0.47941, -0.5707, 1.0274, 0.88531, -0.48918, 0.57961, 0.82575, 1.3595, 0.16566, 1.1078, -0.33823, -1.404, 0.69607, -1.8622, 0.074717, 1.4778, -1.8682, -2.0655, 0.7506, 1.3301, -0.6443, 0.23906, 0.52738, -0.85608, 1.362, -0.93056, 0.58876, 2.7884, -0.51813, -0.46731, 0.10754, -0.98527, -0.33371, -0.088166, -0.83007, 0.040104, 0.78619, -0.085668, -0.44826, 0.24394, -0.67234, 0.82832, 0.17496, 0.13062, 2.8271, 1.4975, -0.40908, 0.1359, 0.9072, -1.7599, -0.5453, 2.1171, -0.014887, -2.5589, -0.57775, 1.6834, -0.080809, 0.093551, 0.61804, -1.0751, -1.2765, -1.3712, -0.51046, -1.0615, -1.1314, 0.4715, -0.21498, 0.97776, -1.1221, -0.48521, -1.4482, -0.21924, 1.2974, -0.28956, 1.7851, 0.071719, 0.6046, -0.29271, -0.17605, 0.9617, -1.5625, -0.052854, -0.95211, -1.1408, -0.11444, 1.1291, -0.10331, -1.2746, 0.3409, 1.1205, -0.007955, 0.72281, -0.5432, -1.0362, 0.70673, -1.4891, -1.4421, -0.79956, 0.34391, 1.7518, -1.2119, -1.0638, 3.0957, 1.0524, -1.5077, 0.40659, -0.13836, 0.70665, 0.1415, -1.6408, -0.21954, -1.9064, -1.1034, -0.22287, -2.893, 0.64603, 0.54533, -0.37797, 2.0813, 0.0096322, 1.4301, 1.544, 0.26251, -0.60614, -0.81926, -0.055482, -0.19908, -0.40061, -1.3979, -0.22105, -0.49838, -0.18867, 0.2221, 1.195, 0.92599, -0.47433, -1.0105, -1.0042, 0.5298, -1.0114, 0.12258, -0.55754, -2.2667, 1.9457, 1.1418, 0.47313, -0.16508, -1.1121, 1.1526, -1.5508, 0.085199, 1.257, -0.032102, 0.25237, 1.5262, 0.74626, -0.67467, 0.5987, -2.4428, -0.96545, 1.0493, 0.84523, -0.71383, 0.64297, -0.91477, -0.90476, -0.076182, -0.52942, 0.71457, -0.44154, 1.8578, 0.46729, 1.1348, 1.0283, -1.7163, 0.3485, -0.56561, -1.9252, -1.6894, 0.33806, 1.1391, 0.019754, 0.075867, -0.40031, -1.2373, -0.55594, 1.8686, -0.18068, -0.32389, -1.1951, -1.3943, 1.0211, 0.86427, 0.41696, -0.064811, -0.52159, -1.4744, 0.10421, 1.4487, 1.3691, 0.30712, 0.51162, 1.143, -0.69583, 0.99675, -0.7941, -1.7184, 0.49574, -0.5451, -0.40853, 0.054156, 0.29137, 0.1873, 0.87522, -0.26537, -0.21206, 1.0851, 0.77455, -0.52056, 0.21701, 0.55345, -0.26027, -0.21041, 0.72292, 0.74644, -0.92501, 0.95202, 0.33238, -1.3193, -0.60032, -0.58882, -0.36836, -0.45182, -0.11879, 1.2434, -1.0261, -1.6561, -0.58256, -1.8611, 1.5295, 0.8271, -1.5577, 0.97718, 1.1691, 0.032562, 0.021012, -0.73012, -0.69482, -0.95646, 0.25551, 0.57942, -0.020484, -1.4317, -1.0873, 0.93029, 0.68092, 1.3767, -0.4535, 0.031838, 0.10265, -0.062462, 0.1742, -0.76579, -1.0554, -1.7138, 0.92156, 0.5084, 0.61305, 0.38835, -1.4137, -0.16386, -0.60906, 0.99488, -0.15377, -0.43294, 0.63945, -0.93808, 0.046884, 0.28169, -0.35728, -0.0070352, -1.3972, 0.081111, -0.67337, -1.5621, 0.039233, 0.24375, -0.39277, 0.44538, -0.21229, 1.0072, 0.7884, -0.49646, 1.4641, 0.37058, -0.029677, 0.22157, 0.87804, 0.44536, 0.19623, 1.4059, 0.13539, 1.9609, 0.90209, -0.01327, -1.9248, -1.9553, 1.6211, 0.99334, -0.2009, -1.9405, 1.3016, 0.2025, -0.23645, 0.90779, -0.87049, -1.2309, -0.48495, 1.0632, -0.77715, -0.43676, 1.1997, 2.1881, 0.66908, 1.3644, 1.4551, -1.3083, 0.45117, 0.098604, 0.19604, 0.48614, -2.0011, 1.4728, 1.9863, 0.32458, 1.5602, -0.73228, -0.90268, -0.18635, -1.4851, -0.47345, 1.4504, -0.22424, -1.1756, 0.96195, 0.5339, 0.78421, 1.1679, -0.90154, 0.59978, 1.1049, -0.4953, -0.73007, 0.34732, -0.039512, -1.1357, 0.67044, -0.88231, 0.69941, 0.9471, -1.9209, -0.57087, -1.6958, -0.138, -0.56867, -0.85448, 0.32733, -1.104, -0.088407, -0.22002, -0.46841, 0.15759, -0.37281, -1.5106, 1.1484, -0.5359, 0.054622, 1.5979, -0.35963, 3.6665, 0.18604, -1.4568, -1.5638, -1.1508, 0.22391, -0.047055, 0.71316, -0.74877, 0.56384, 2.2421, 0.85364, 0.56248, -0.87945, -1.1682, 0.9451, 0.6915, -0.077629, -1.1818, -0.17716, -1.6785, -1.3966, -0.00312, 2.2395, 1.3149, -0.18159, 0.57985, -0.94257, 0.66172, 1.7027, -0.31718, -0.94815, 0.64889, 0.36113, -1.784, -1.1272, 2.5596, -0.18054, -1.2766, 1.9195, 0.63457, 1.5751, 2.1586, -1.1666, -2.3267, -1.5599, -0.16989, 0.23129, 0.10809, -0.099515, -0.1756, 0.39558, 2.3103, 1.1172, 1.4088, -0.61327, -0.13879, 0.1258, -0.89995, 1.9482, 0.62827, 1.2225, 0.37326, -0.38694, 0.77359, 0.37657, 0.47068, -1.0603, -1.0868, -1.1322, -0.40877, 0.24499, -2.3171, -0.12558, -0.25279, -1.3003, 1.2074, 1.0945, 1.8136, 2.5855, 1.6878, 0.24752, 0.69339, 2.0613, -0.85689, -1.4839, 0.49037, -1.4878, 1.6408, 0.98252, -3.1073, 1.4137, 0.59122, -1.7983, 0.95415, -0.59267, 0.028297, 0.075472, 0.69037, 1.7256, 1.2552, 1.4722, 0.53618, 0.65475, 1.8263, 1.3225, -0.50106, 0.34231, 0.98076, -1.2248, 0.086563, 0.70364, -1.1404, 1.4855, 0.63401, -1.0052, -0.56062, 1.4559, -0.18931, -1.3686, -1.2008, -0.33159, -0.28738, -0.5392, -1.3028, -3.2032, -1.2642, 0.28874, 1.2, -0.85951, -0.3494, -0.73112, -0.15607, 1.7478, 1.0546, 0.18679, -1.7647, 1.2978, 0.47215, 0.51319, -2.0868, -2.4207, 0.079512, 0.16965, -1.0297, -0.84355, -0.73938, 0.33954, 1.4896, 1.1705, 0.76244, 0.58082, 1.8582, -1.3445, -0.68385, 0.32823, 1.4576, 1.4928, -1.4179, 0.39287, 1.9265, -1.7712, -1.8243, 1.1277, 1.1727, 1.1575, 0.12053, 2.7316, 1.96, -0.5638, 0.8561, -0.028812, -1.0375, -1.3379, 0.86438, 0.10214, -1.4781, 0.21744, -0.04637, -2.485, -2.0821, 0.38918, -0.69387, -2.4561, -0.7664, 0.94953, -0.19945, -1.2875, 0.21535, 2.0149, 1.7681, 1.0109, 0.053485, 0.88251, -0.14319, 0.16218, 0.038135, 0.70807, -0.13572, 0.049937, -1.0621, -0.9522, 1.071, -2.0988, -0.46026, -0.162, -0.2635, -1.1812, -0.083951, -0.12916, -1.0836, -1.5345, -1.5286, 0.75613, -1.0262, 0.17711, 2.4183, 1.9178, 0.45965, 1.3011, -1.1437, -0.080648, 1.662, 1.3997, -0.97531, -0.33742, 2.3715, -1.5555, 0.061547, 0.29282, -0.14715, -0.21413, 1.491, 0.67859, -1.0014, -0.63877, 0.29502, -0.39921, -1.5649, -1.4655, -1.0633, 0.060291, 1.2752, 1.5975, 0.66559, 0.79621, -0.27771, 3.9322, -0.17422, -2.8988, -0.64256, -0.035866, 0.45182, 1.7466, -0.4418, -1.6681, 0.32632, 0.088848, -0.13264, -2.6827, -0.039628, 0.14426, -2.4108, -0.31075, 0.22664, 1.3976, 0.38042, -0.6491, 0.56804, -0.6117, -0.163, -1.4403, 0.6444, 0.32245, -0.50725, 1.6954, -1.4665, -0.74155, -1.5187, 0.48159, 0.19066, -1.9828, 0.37601, -2.1533, -0.24562, -0.18498, 0.37735, 3.3855, -0.17819, 1.9374, -1.4737, -0.46226, 0.39557, 0.041312, 1.666, -0.15342, 0.082775, 0.2045, 0.29464, -0.36529, 0.67643, 1.8706, 1.2919, -0.26429, -0.23554, -0.59964, -0.26342, -0.87437, -0.30131, 0.060096, 0.994, 0.26277, 0.56049, 2.3882, -0.78876, 0.24919, 0.77415, 1.1389, -0.63739, -0.16345, 1.0443, -1.3892, -0.97766, 0.12252, -0.84555, -1.1002, -0.6454, -0.35156, 0.012707, 0.21588, -0.66209, 1.0655, 1.6524, -0.98077, -0.23999, 0.4741, 0.87485, 0.72076, 0.88723, -0.43524 };

T_2 = { 0.33704, 1.8552, 1.5559, 1.1498, 0.29595, -0.8019, 0.52389, -0.66144, 0.54709, 2.5769, -0.87282, -0.24696, 0.22304, -0.064749, 1.5146, 0.88741, 0.56086, 0.16268, 1.1603, -0.60712, -0.34242, 1.5964, 0.72092, 0.39141, -0.84366, -0.84313, 0.74418, 0.053387, -0.10267, -0.82628, 0.68397, 2.3725, -0.55335, -0.41403, 1.9029, 0.63753, -0.4045, 0.78201, 0.56852, -1.12, -1.7211, -0.47454, -1.3491, -1.0273, -0.279, 0.53526, 0.52262, -0.52169, -0.29594, -0.30505, 0.4919, -0.21775, 0.15409, 0.85339, -0.50445, -1.1324, 0.092041, 0.58563, 0.39248, -1.4605, -0.20777, 1.5919, -0.86334, 0.84079, 1.5304, -0.94995, -0.63923, 2.609, -0.41717, -1.5121, 1.5954, -0.84186, 0.25923, 0.32429, -2.4553, -0.14961, 1.009, -0.66521, 1.4325, 1.4663, 0.80996, -1.0873, 0.82252, 0.54403, -1.2881, 1.0047, -0.41165, 1.266, 0.23886, -0.49206, 1.0506, -0.83075, -0.97906, -0.34056, -0.034318, 0.41944, 0.11133, 0.90768, 0.62157, 0.39752, -0.13391, -0.57207, -1.8574, -0.23518, -0.51491, -0.46862, 1.7918, -1.8901, -1.4628, -1.026, -0.65574, 1.0677, 0.77964, 0.47473, 1.3111, 1.0101, 0.016137, -0.30027, -1.5499, -1.531, 0.21709, 0.02692, 0.33537, -0.11021, -0.51926, 0.91416, -1.1898, -1.0827, -0.69013, 1.6389, 1.4012, 0.56894, 0.45598, 1.4704, 1.2893, -1.2394, -0.24188, -1.2206, 0.11813, 0.38838, -0.50556, 0.26342, 0.03092, -0.35421, -0.58325, 0.24278, 1.3958, -0.29391, -1.1698, 0.98864, 0.95754, -0.28964, -1.1917, 0.58401, 1.6481, -0.20266, -2.0118, -0.80588, 0.13727, 0.68899, 0.21898, 0.086773, -1.2007, -0.77953, 0.78799, -1.3044, -0.95748, -0.94668, -0.46495, -0.68115, -0.50627, 1.4451, 1.307, 0.83167, 0.49846, 0.34255, 1.053, -0.3802, -1.471, 1.6847, 0.7785, -0.87307, 1.1198, -0.49134, -0.45272, 1.9656, -0.78851, -0.7673, 1.2983, 0.36764, -1.3388, -1.3338, -0.14936, -0.53219, 1.2894, 0.71968, 0.77959, -0.47448, -1.7105, 0.32192, 1.599, 1.0707, 0.019071, 0.62937, 0.0046704, -1.0743, -0.26611, 1.4551, -0.20943, 1.1382, 1.0842, -1.2566, 1.9956, 0.51984, 0.019087, 0.78675, 0.3818, 0.00049256, 0.65056, 0.50333, -1.5187, 1.7538, -1.161, -2.7199, 1.2423, -2.0084, -0.58767, 0.94025, -0.21432, 0.33092, 0.029031, -0.8362, -0.52163, -1.1983, 0.70371, 1.9852, -0.78991, 1.5322, 0.41272, -1.975, -0.42063, -0.94821, -1.1931, 1.2805, 0.48645, 0.2869, 1.1124, -1.2244, -1.1608, 0.10938, 0.33888, -1.639, -2.2469, 1.2495, 0.86219, -2.0408, -1.6087, -0.45979, -0.45242, -0.99223, -1.1851, -2.1266, -0.74243, 1.9099, 1.0189, 1.1119, -0.15854, -0.6576, 0.93811, -0.70401, -0.79916, -0.69329, -1.8685, -0.084733, -0.12283, -0.051298, 0.38029, -0.67276, 1.8527, 2.2858, -0.49767, 0.32174, 2.1044, 3.1248, -0.071749, -1.0253, 2.0693, -1.0513, -1.2872, -0.75784, -1.5602, 0.45591, -0.48604, -2.3631, -0.87964, 0.6573, 1.5335, 2.7387, 0.28443, 0.21852, 0.10627, -0.12449, 0.20125, -0.028731, 0.65568, 0.1833, 1.4757, 0.24608, -0.73643, -0.98884, -0.24246, -0.72545, -0.095471, 1.2128, -0.47347, 2.1234, 1.6192, -1.2881, -0.083671, -0.2872, -1.5767, 1.184, -0.14623, -0.53203, 1.5546, -0.75153, 0.075176, -0.29992, -0.88358, -0.11278, 0.29812, -0.68469, -3.0627, -0.49166, 0.19398, 0.068504, 1.2363, 0.26959, 0.025349, 0.33712, 0.53274, 0.45818, 0.65563, 0.66046, 0.26902, -0.84743, -1.589, -0.086598, -0.011354, -0.88862, 0.77249, -0.87089, 1.7522, 1.4763, -1.7303, 0.56167, -0.33832, 0.46719, -0.59106, -0.36334, 1.3962, 0.28315, -1.1496, 0.63594, 1.517, -2.0234, 0.44827, 1.6261, 1.2468, 1.817, -0.95394, 0.71779, 0.77603, 0.77744, 1.194, -0.69959, -0.93329, -0.92455, 0.30441, -0.049116, -0.13609, 0.55469, -0.6371, -1.5834, 1.4743, -0.24954, -0.40993, 0.38456, -0.092218, 0.95002, -0.21552, -0.089123, 2.0048, 0.81859, -0.47682, 0.72501, 1.0292, 0.58896, -0.78309, -0.31489, -1.3425, 0.54391, 1.2229, -1.3552, -0.61795, -0.061518, -0.49812, -1.732, 1.1944, 1.1786, -0.77453, 0.44529, 0.89877, -0.48052, 0.23, -0.010488, -0.45732, -0.50447, 1.0143, 1.4025, -1.03, -1.2556, -0.36858, 0.91854, -2.3942, -2.6824, -0.31797, -1.8175, 0.51033, 0.22296, -0.75284, -0.67553, 0.48786, -0.32356, 0.085113, 1.5534, -0.34831, -0.37006, 1.1978, -0.0027619, -1.4551, -0.81313, -0.95102, 0.61799, -0.088164, -0.9666, 0.0059797, -0.76823, 1.2901, -0.31528, -0.39783, 1.3573, -0.27944, -0.48675, 0.60418, 0.31995, 0.37287, -0.14901, -1.1554, -0.34517, 1.3302, 0.57795, 0.092785, -0.55833, -0.34171, -0.22151, 0.66335, 0.49831, -0.80389, 1.4837, 0.59337, 0.23949, 2.0932, 0.19687, -0.83747, 1.9598, 0.49214, 0.84913, 0.2026, -0.19694, 0.17104, -0.88549, 1.2027, 2.1284, 0.78125, -0.17787, -1.1086, 0.95265, 1.6233, -1.1052, 0.011355, -0.88489, 0.58229, -0.18906, -0.64274, 0.80716, -0.46034, -0.17627, -0.42372, 1.5244, 0.089254, -0.64681, 0.0078003, -0.55643, 0.70197, -0.29594, -0.22835, 0.66337, 0.63272, 0.72066, -0.21104, -2.2308, 0.29007, -1.0276, -1.6288, -0.18517, 0.34936, -0.28661, 0.12428, 2.7762, 0.062884, -0.25702, -1.0724, -1.4431, 1.5073, -0.44252, -0.97865, 0.40451, 1.4464, 0.24852, -0.013475, 1.105, -0.67392, 0.2782, -1.3171, -1.2994, 1.3763, -0.017518, 0.69087, 0.36684, 0.50159, 1.299, 0.5254, -0.62122, -1.3943, -1.7058, 0.52703, 0.74739, 0.0106, -0.044392, 0.0050638, 1.1411, -0.54949, -0.4152, 0.77568, -0.68106, 0.33867, 0.92296, -0.46335, -0.30179, -1.6231, -1.6172, 1.0904, -0.90463, 0.38892, 1.7374, -0.87816, 0.85408, 0.49735, 1.1521, -1.377, -1.2081, 0.29667, -0.40238, 1.3198, 1.2231, 1.1266, 1.1561, -0.15888, 0.35422, 0.79558, -0.42835, -1.2696, -0.29916, 0.64598, 0.075794, 0.34819, 0.64148, 0.17849, 1.093, -1.1214, -2.1301, -1.401, -0.5865, 0.487, 1.0779, 0.51937, 0.70704, 1.1146, 1.4153, 0.74479, -0.60093, 0.12953, -0.249, 0.18788, -0.61264, -0.42081, 0.55264, -1.1526, -0.22557, -0.3344, 0.63683, 1.0571, 0.074912, 0.82951, 0.19649, 0.37998, -0.32912, -0.50012, -0.087908, -1.8437, 0.18846, 1.6438, 0.70178, -0.68383, 0.32705, -0.59778, -0.069792, 0.49, 1.0508, 1.0392, -1.3976, 0.73296, 0.8593, 1.0681, 0.62332, 0.2509, 1.4719, 0.38228, -0.22802, -1.5836, -1.967, -0.93725, -0.10295, -1.0985, -1.6327, -0.070733, -0.6365, 0.59223, -0.26226, -2.1457, -0.96183, 0.58327, 0.2515, 0.13263, 0.72086, 0.38575, 0.73918, -0.75213, -0.55062, -0.99559, 0.89334, 0.92862, -0.39449, -2.1194, -1.1847, 1.6586, -0.5555, -1.8634, 0.13825, -0.41866, -1.7278, 1.4808, 0.65387, -2.5345, -1.7106, 2.0064, 1.1408, -2.0511, 0.59862, 0.21942, -1.9931, 0.76636, -0.69259, -1.1741, 0.5761, 0.96149, -0.17797, -0.95562, 0.61296, 1.0413, -0.085247, -0.74634, 0.11808, 0.77916, 1.183, 0.89734, 1.1209, 0.65507, 0.17496, 0.82561, -0.035875, 0.91258, 0.9509, -2.0915, -1.2121, -0.30108, -0.38053, 0.55359, -0.49203, -0.039222, 0.41694, 0.60789, 1.4381, 1.2282, -0.81418, -1.3535, -1.0761, 1.7111, -0.55229, 0.59931, 3.0443, -0.32315, -0.78287, -0.23602, 1.7729, 0.93005, -0.3961, 1.1237, 1.0485, -0.80548, -0.027681, -1.1923, -0.40887, -0.79284, -0.55238, 0.70318, 1.2776, -0.019774, -0.90922, 0.65507, -0.9209, 0.066807, 0.51621, -2.2343, 1.3425, 0.8099, -0.95493, 1.0883, 0.14033, 1.2051, -0.094286, -0.38591, -0.74, -2.2652, 0.52709, 0.40585, 0.16463, -0.54107, -1.0449, 1.2798, 0.71493, 0.84614, -0.17936, 0.46717, 1.6372, 1.0552, 1.1256, 0.47366, -0.86937, -1.0255, 1.2103, 0.17531, -1.464, -1.2289, -0.56568, 0.37409, -0.041711, 1.5229, 0.47519, -0.16121, -1.6774, -1.9766, 0.86582, -1.0764, -1.0976, 0.55111, -1.5532, 0.43097, 0.78801, -2.3539, -1.667, 1.0349, 0.20383, 0.13893, 0.70712, -0.16359, 0.9384, -0.7775, 0.50179, 1.4519, -1.4373, -0.30179, -0.74156, -0.54495, -0.37917, 0.69651, -0.62169, 1.2675, 0.79003, -1.271, -0.6222, 0.021504, 0.44991, -0.94106, -1.9431, 0.11631, 1.047, -1.1258, -0.18238, 0.58775, -0.23439, 0.33059, 1.8423, 0.34121, -0.81267, 0.05624, 0.15172, -1.5359, -0.55776, 0.1886, -0.27966, -1.2302, -1.86, -0.25435, -1.1575, -0.32568, 1.3999, -0.52897, 0.81488, -0.0076271, 0.70344, -0.028607, -0.8689, 2.6373, -0.90456, -1.5622, 0.78061, -0.15256, -0.42505, 0.96154, -0.12014, -0.32566, 0.41901, -0.33607, 1.507, -0.77138, -1.5959, 1.0757, -0.97569, -0.91751, 0.15427, -1.4128, 0.64319, 0.77505, 0.35153, -0.068519, -0.13968, -0.38263, -1.197, 0.98241, 0.9884, -0.52542, 0.043785, 0.58224, 0.30032, 1.0727, 0.45644, -1.9095, -0.13341, 0.23684, -0.43355, 0.59424, -0.16034, 0.60438, 0.54342, -0.79794, -0.84868, 0.4583, -0.43446, -0.2027, 0.29193, 1.3834, 1.0838, 0.60274, -0.20582, -1.1924, 0.29398, -2.1465, -0.86608, -0.010905, -0.10809, 0.5573, 0.72338, 0.11933, -0.25956, -1.2342, -0.48209, -1.1702, 0.8056, 1.3086, -2.0599, 0.17172, -0.811, 0.75286, 2.1094, -1.4289, 0.30489, -0.54365, -0.11663, 1.8204, 1.5472, 1.3812, -0.23902, -0.75983, -0.56053, 0.19655, -0.11889, -0.024891, -1.0343, -1.3407, -0.34606, 0.078127, 1.787, 0.052711, -0.99729, 1.212, -0.71616, 0.7305, 0.92835, 0.53485, 0.036136, -0.12721, 0.31347, -0.19024, 1.6087, -0.059717, 0.28888, 0.18797, -1.4319, -0.99523, -1.5278, -1.8524, 2.4909, 0.96329, -1.9591, -0.45869, -0.35718, -0.39169, -0.95629, -0.83167, -0.88986, 0.55599, 0.9224, 1.0862, 0.79843, 0.60553, -0.528, -0.54671, -0.38303, -0.9454, -0.078765, -0.95739, 1.075, 0.61077, -0.98378, -0.6026, -0.24061, 1.1547, 1.3901, -0.72881, -0.16609, -1.8277, -0.35833, 1.1332, -1.7111, 0.82531, 1.059, 0.13715, 0.75059, 1.0543, -2.2335, -1.5594, -0.045526, 0.2567, 0.39568, -0.50714, 0.041196, 0.17957, -0.011558, -2.6584, -1.0323, 1.9567, -0.18367, -0.88119, -1.7847, -0.38159, 0.3368, 0.4363, 0.1516, -1.3689, 0.50722, 0.93187, 1.5275, 0.17389, 0.2949, 0.40645, 0.36684, 0.88846, 1.4203, 0.76952, 0.10855, 0.26894, 2.3895, 0.25376, -1.6953, -0.12518, -0.93675, -0.2145, -0.16332, 0.41978, 0.33395, -1.8554, 1.1659, 1.8946, -1.5105, -0.061268, 0.10337, -0.8616, -0.43875, -1.3936, 1.3585, 2.1239, 1.1395, 0.82367, -0.21878, 1.0791, 0.38356, 0.34319, 0.70683, -0.62383, -0.056037, 0.42804, 0.14634, 0.69401, 1.227, 2.3897, -1.361, -0.14217, -0.52936, -1.656, -1.8544, 1.3218, 1.7732, -0.52906, 1.7429, -0.92599, 0.08697, 1.3273, 0.33381, 0.73708, -0.038159, -0.45331, 0.59648, -0.7201, 0.46711, 0.93125, -2.4249, 0.39834, 1.7964, -0.31333, -1.7536, -0.96653, -1.7653, -1.2899, 0.78943, -0.1746, -0.15463, -1.1354, -1.5331, -0.26345, -0.66208, 0.20485, -0.15815, -0.7595, 0.39711, -0.65171, 1.5399, 1.4928, -0.20195, 0.68991, 0.28685, 0.098254, -1.7425, -1.4003, 0.35139, 0.26994, 1.1136, 0.44249, -0.65781, 0.63859, -0.96194, 1.6689, 0.30808, 0.11181, 1.3207, -2.2637, 1.0982, 0.37232, -0.86763, -0.28191, -1.0983, -0.41028, 1.1885, 1.1597, -0.85041, 1.0035, 0.050844, 0.38651, 0.79874, -0.12508, 0.32351, -0.86048, 0.8073, -0.36111, -0.65969, 0.33931, -0.45306, 2.0425, 1.2442, -1.3958, 1.0821, 0.49311, 0.036128, -0.88861, -2.01, 1.3543, 0.28695, -2.2478, -0.13565, -0.071995, 1.7661, 1.6821, 0.33885, 0.57322, -0.0061708, 0.80808, 0.23508, -0.34033, 1.0234, 1.7307, -0.12226, 0.021101, 0.36503, 0.54197, 0.57415, -0.4611, -0.56372, -1.1684, 0.027439, 0.15537, -0.9306, -2.0143, -0.38769, -0.27463, -1.5039, 1.2383, 1.0166, 1.6031, 1.0427, 2.5844, 0.71155, -0.54751, 1.6813, 0.65732, -0.59946, -0.85848, 0.1142, 0.88508, 1.0068, -1.7088, -0.67397, 0.36077, -0.033055, -0.47782, -0.33132, 1.6336, 1.4813, -0.85975, 1.8087, 1.3929, -0.13376, 1.2894, 0.69987, 1.1426, 2.1059, -0.36684, -1.0949, 1.1906, 0.089333, 0.17633, 0.13397, 0.72654, 2.2442, -1.654, 0.10799, -0.23958, -0.70555, -0.94461, -0.085478, 1.4369, -1.3882, -2.3178, -1.8045, -1.2788, -0.70669, 0.16231, -0.68577, 1.1149, -0.66789, -1.3089, -0.91223, -0.065167, 0.42725, 1.064, 0.38384, 0.069534, 0.37599, 0.37596, -0.60273, -1.552, -1.4075, 0.0076051, 0.2854, -1.1357, -0.84781, -0.44064, 1.0737, -0.22861, 0.73523, 0.77984, 0.5365, 0.45187, -1.4949, 1.2603, 1.8745, 0.19604, -0.067873, 1.1009, 1.2712, -0.74435, -0.2481, -0.49231, -0.21557, 0.79266, 1.0207, 1.5139, 1.6367, 0.84882, -0.54459, 1.3408, -0.28398, -1.4471, -1.9498, 1.0679, 1.4374, -1.0639, 0.82198, -1.242, -1.5331, -1.0809, -2.7287, -0.94559, -0.1898, -0.43659, -0.0082367, -1.1914, -0.42594, 1.1755, 0.91157, 1.3659, 0.66957, 0.55832, 0.63291, 1.0257, 0.72418, -2.02, 0.003046, 1.9944, -0.90001, 0.043342, -1.2348, -0.27117, -0.94613, -0.25462, 0.077065, -2.0894, -0.221, -1.9531, -1.3847, 0.80325, -0.21443, -1.9434, -1.309, 0.5086, 1.8988, 1.8301, 1.2468, -0.32472, 1.4931, 1.2705, -0.085228, -0.31084, -0.31576, 1.6693, -0.47475, 1.0311, 0.63006, -0.44175, 0.65529, -0.92325, 0.28645, 0.45158, 1.1932, 0.73085, -1.9236, 0.81218, -0.58016, -2.0183, -0.58721, -0.30354, 0.18068, -0.1505, 0.12707, 1.5491, 1.4981, 0.62685, 2.0135, -1.1446, -0.39161, -0.259, -0.7951, 1.3221, -0.65827, 0.15617, -0.30129, -2.3034, 0.854, -0.43808, -1.6036, -0.68123, -2.0963, 1.892, -0.78227, -0.93879, -0.43246, 0.52054, 0.4836, -1.4507, 0.77917, -0.83623, -1.4031, 0.84757, 0.57253, -0.027712, 1.3238, -1.5286, -0.64992, 0.8009, -0.43247, -0.45119, -1.0307, -0.96312, -2.1582, 0.37888, 0.72913, 0.14049, 3.1725, 1.1284, 0.23249, -0.94105, -0.72276, 0.60969, -0.81174, 0.73446, 2.0861, -0.3353, 0.39472, 0.87566, -0.09792, -1.1364, 1.9492, 2.0596, -0.57767, -0.19193, -0.9996, -0.69024, 0.37739, -0.45541, -0.37005, 1.3886, 0.33196, -0.0068382, 0.99834, 1.8246, 0.37572, 0.76989, 0.3298, -1.0552, -0.17576, -0.37864, 0.21822, 0.0091368, -1.3222, -0.39459, -0.013082, -0.22519, -1.452, -1.4705, 0.21574, 0.48951, 1.2735, 0.99043, -0.68052, -0.27068, -0.10329, 1.0468, 0.40699, 0.74148, -1.1751 };

freqVec(b) - STFT Band center frequencies

| bin index | center frequency | bin index | center frequency | bin index | center frequency |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.000000000 | 51 | 8812.499997984 | 102 | 18374.999996125 |
| 1 | 140.644316361 | 52 | 9000.000008860 | 103 | 18562.499990092 |
| 2 | 234.355478108 | 53 | 9187.500004401 | 104 | 18749.999991865 |
| 3 | 328.144332285 | 54 | 9375.000001529 | 105 | 18937.499986965 |
| 4 | 421.855497937 | 55 | 9562.500006565 | 106 | 19124.999985762 |
| 5 | 515.644326841 | 56 | 9750.000006335 | 107 | 19312.499985261 |
| 6 | 609.355515147 | 57 | 9937.499999557 | 108 | 19499.999989766 |
| 7 | 703.144330614 | 58 | 10125.000002928 | 109 | 19687.499988292 |
| 8 | 796.855543885 | 59 | 10312.500002384 | 110 | 19874.999989851 |
| 9 | 937.500032020 | 60 | 10500.000004406 | 111 | 20062.499978542 |
| 10 | 1125.000017338 | 61 | 10687.500002820 | 112 | 20249.999981602 |
| 11 | 1312.500035449 | 62 | 10875.000001403 | 113 | 20437.500005879 |
| 12 | 1500.000075751 | 63 | 11062.500002219 | 114 | 20625.000004853 |
| 13 | 1687.500031782 | 64 | 11250.000001097 | 115 | 20812.500015815 |
| 14 | 1875.000024239 | 65 | 11437.500001292 | 116 | 20999.999958305 |
| 15 | 2062.499975101 | 66 | 11625.000000815 | 117 | 21187.499980259 |
| 16 | 2250.000053703 | 67 | 11812.500000140 | 118 | 21374.999997733 |
| 17 | 2437.500044271 | 68 | 12000.000000000 | 119 | 21562.499955794 |
| 18 | 2625.000002315 | 69 | 12187.499999584 | 120 | 21749.999946298 |
| 19 | 2812.500019782 | 70 | 12374.999999473 | 121 | 21937.500025004 |
| 20 | 3000.000041692 | 71 | 12562.499999294 | 122 | 22124.999975461 |
| 21 | 3187.499983930 | 72 | 12749.999998799 | 123 | 22312.499968567 |
| 22 | 3374.999995137 | 73 | 12937.499997514 | 124 | 22499.999924162 |
| 23 | 3562.499994173 | 74 | 13124.999998543 | 125 | 22687.499964503 |
| 24 | 3750.000018557 | 75 | 13312.499997602 | 126 | 22874.999982475 |
| 25 | 3937.500021643 | 76 | 13499.999995904 | 127 | 23062.499968048 |
| 26 | 4125.000009859 | 77 | 13687.499996961 | 128 | 23249.999976609 |
| 27 | 4312.500011528 | 78 | 13874.999996550 | 129 | 23437.499982579 |
| 28 | 4500.000010423 | 79 | 14062.500000495 | 130 | 23624.999922020 |
| 29 | 4687.500014446 | 80 | 14249.999993960 | 131 | 23812.499893152 |
| 30 | 4875.000013588 | 81 | 14437.499993440 | 132 | 24000.000000000 |
| 31 | 5062.500013570 | 82 | 14624.999997861 |  |  |
| 32 | 5250.000007575 | 83 | 14812.499995461 |  |  |
| 33 | 5437.500010288 | 84 | 14999.999991137 |  |  |
| 34 | 5625.000004178 | 85 | 15187.500001756 |  |  |
| 35 | 5812.500003421 | 86 | 15374.999999428 |  |  |
| 36 | 6000.000005158 | 87 | 15562.500000999 |  |  |
| 37 | 6187.500003404 | 88 | 15749.999993809 |  |  |
| 38 | 6375.000003488 | 89 | 15937.499992382 |  |  |
| 39 | 6562.500007191 | 90 | 16124.999995683 |  |  |
| 40 | 6750.000005972 | 91 | 16312.499995240 |  |  |
| 41 | 6937.500008499 | 92 | 16499.999994365 |  |  |
| 42 | 7125.000006936 | 93 | 16687.499991354 |  |  |
| 43 | 7312.500008549 | 94 | 16874.999992234 |  |  |
| 44 | 7500.000005032 | 95 | 17062.499991361 |  |  |
| 45 | 7687.500004875 | 96 | 17249.999994298 |  |  |
| 46 | 7875.000004878 | 97 | 17437.499992410 |  |  |
| 47 | 8062.500007586 | 98 | 17624.999995960 |  |  |
| 48 | 8250.000006218 | 99 | 17812.499995945 |  |  |
| 49 | 8437.499999805 | 100 | 17999.999994836 |  |  |
| 50 | 8625.000000113 | 101 | 18187.499996913 |  |  |

Closest center frequency bin for each one-third octave band frequency

| bin index | center frequency |
| --- | --- |
| 0 | 16.0 |
| 0 | 20.0 |
| 0 | 25.0 |
| 0 | 31.5 |
| 0 | 40.0 |
| 0 | 50.0 |
| 0 | 63.0 |
| 1 | 80.0 |
| 1 | 100.0 |
| 1 | 125.0 |
| 1 | 160.0 |
| 2 | 200.0 |
| 2 | 250.0 |
| 3 | 315.0 |
| 4 | 400.0 |
| 5 | 500.0 |
| 6 | 630.0 |
| 8 | 800.0 |
| 9 | 1000.0 |
| 11 | 1250.0 |
| 13 | 1600.0 |
| 15 | 2000.0 |
| 17 | 2500.0 |
| 21 | 3150.0 |
| 25 | 4000.0 |
| 31 | 5000.0 |
| 38 | 6300.0 |
| 47 | 8000.0 |
| 57 | 10000.0 |
| 71 | 12500.0 |
| 89 | 16000.0 |
| 111 | 20000.0 |

EQbin

Binaural EQ EQbin (see Clause 6.6.26.3.5.3)

| bin index | EQbin(b) | bin index | EQbin(b) | bin index | EQbin(b) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.9260 | 51 | 1.6291 | 102 | 1.5237 |
| 1 | 0.9125 | 52 | 1.5511 | 103 | 1.3058 |
| 2 | 0.9081 | 53 | 1.4354 | 104 | 1.2100 |
| 3 | 0.9125 | 54 | 1.2503 | 105 | 1.1955 |
| 4 | 0.9353 | 55 | 1.0908 | 106 | 1.2367 |
| 5 | 0.9539 | 56 | 1.0447 | 107 | 1.1854 |
| 6 | 0.9851 | 57 | 1.0882 | 108 | 1.2017 |
| 7 | 0.9864 | 58 | 1.0956 | 109 | 1.4122 |
| 8 | 1.0052 | 59 | 1.1305 | 110 | 1.6408 |
| 9 | 0.9861 | 60 | 1.2584 | 111 | 1.7691 |
| 10 | 0.8966 | 61 | 1.2681 | 112 | 1.5915 |
| 11 | 0.8599 | 62 | 1.3551 | 113 | 1.5407 |
| 12 | 0.8723 | 63 | 1.5122 | 114 | 1.5362 |
| 13 | 0.9705 | 64 | 1.5078 | 115 | 1.5532 |
| 14 | 1.1584 | 65 | 1.4253 | 116 | 1.5277 |
| 15 | 1.3672 | 66 | 1.5348 | 117 | 1.4606 |
| 16 | 1.4724 | 67 | 1.5593 | 118 | 1.5091 |
| 17 | 1.4113 | 68 | 1.5143 | 119 | 1.5231 |
| 18 | 1.3476 | 69 | 1.5152 | 120 | 1.6351 |
| 19 | 1.2923 | 70 | 1.6453 | 121 | 1.8617 |
| 20 | 1.2496 | 71 | 1.6941 | 122 | 1.9810 |
| 21 | 1.2379 | 72 | 1.6581 | 123 | 1.9883 |
| 22 | 1.2737 | 73 | 1.7024 | 124 | 1.8054 |
| 23 | 1.3830 | 74 | 1.4535 | 125 | 1.6002 |
| 24 | 1.5525 | 75 | 1.3868 | 126 | 1.5267 |
| 25 | 1.7644 | 76 | 1.3063 | 127 | 1.4320 |
| 26 | 1.8759 | 77 | 1.2892 | 128 | 1.3283 |
| 27 | 2.0243 | 78 | 1.4462 | 129 | 1.1892 |
| 28 | 2.1751 | 79 | 1.3555 | 130 | 1.0683 |
| 29 | 2.1952 | 80 | 1.3570 | 131 | 1.0110 |
| 30 | 2.0104 | 81 | 1.3459 | 132 | 0.9367 |
| 31 | 1.8608 | 82 | 1.3748 |  |  |
| 32 | 1.7755 | 83 | 1.3809 |  |  |
| 33 | 1.6988 | 84 | 1.4103 |  |  |
| 34 | 1.5921 | 85 | 1.5158 |  |  |
| 35 | 1.5094 | 86 | 1.5419 |  |  |
| 36 | 1.4751 | 87 | 1.4814 |  |  |
| 37 | 1.3455 | 88 | 1.4838 |  |  |
| 38 | 1.2756 | 89 | 1.4973 |  |  |
| 39 | 1.2593 | 90 | 1.5224 |  |  |
| 40 | 1.1974 | 91 | 1.5630 |  |  |
| 41 | 1.2166 | 92 | 1.5617 |  |  |
| 42 | 1.2831 | 93 | 1.5790 |  |  |
| 43 | 1.2675 | 94 | 1.6489 |  |  |
| 44 | 1.2008 | 95 | 1.7983 |  |  |
| 45 | 1.1575 | 96 | 1.9983 |  |  |
| 46 | 1.1915 | 97 | 2.2378 |  |  |
| 47 | 1.0990 | 98 | 2.2779 |  |  |
| 48 | 1.1590 | 99 | 2.3001 |  |  |
| 49 | 1.3169 | 100 | 1.8893 |  |  |
| 50 | 1.5064 | 101 | 1.8594 |  |  |

Fast convolution

Uniformly partitioned overlap-save convolution

This section describes the process of convolving a digital signal x[k]∈ R with a finite impulse response (FIR) filter hk∈R of length N to yield an output signal yk=xk*hk=n=1Nxk-nhn. Fast convolution is possible with the fast Fourier transform (FFT), where xk⊛hk=IFFT{FFT{x[k]}∙ FFT{h[k]}} with ⊛ as the operator for cyclic convolution. This reduces the algorithmic complexity from O(N2) for time domain convolution to O(N log N) for the fast convolution. This advantage can be leverage in practice, as efficient implementations of the FFT are available for almost any platform.

In a real-time system, the signal xk is processed in frames, i.e., B samples of xk are used to produce B samples of yk in a single processing step. For fast frame-based convolution in frequency domain, a length N FIR filter is split into P partitions of length L=B. Each partition is zero-padded to length K = 2B and transformed to frequency domain via FFT.

For convolution with this uniformly partitioned filter, a single-channel input frame of length B is appended to a fixed-length time domain buffer of length K, so that the buffer contains the last two input frames. This buffer is transformed to frequency domain via FFT and written into a frequency delay line (FDL). The FDL contains the last P input buffers in frequency domain, and the spectrum of each filter partition is multiplied with the respective input buffer spectrum from the FDL (see Figure 103). The IFFT of the sum of the multiplied filter partitions contains K samples, of which the last B are valid samples of yn, as the filtering corresponds to the overlap-save method.

Figure 103 — Uniformly Partitioned Overlap-Save Convolution

(adapted from Wefers [15] figure 5.9)

When the filter changes (time-varying convolution), one input frame is processed with the previous and the new partitioned filter. The resulting filtered outputs signal are crossfaded with an equal-power crossfade in time domain.

Fast stereo convolution

The higher efficiency of complex multiplications can be leveraged in the case where a single-channel input signal is convolved with two different FIR filters (e.g. HRIRs for left and right output channel). The selected left and right filters hLk∈R and hRk∈R are combined to yield a complex filter hCk=hLk+jhRk, where j=-1 is the imaginary unit. This way, the single-channel input signal can be filtered with both filters using a single uniformly partitioned overlap-save convolution, where the real-to-complex (R2C) FFT is replaced by a complex-to-complex (C2C) FFT. The convolution in frequency domain now requires K instead of K+1/2 complex multiplications and the inverse C2C FFT yields an output signal yCk∈C. The left and right channels for the binaural downmix are then given by yLk=Re{yCk} and yRk=Im{yCk}, respectively.

Support element lookup tables

Table of countOrIndexLoCode

| countOrIndexLoCode | Value | countOrIndexLoCode | Value | countOrIndexLoCode | Value | countOrIndexLoCode | Value | countOrIndexLoCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0111 | 0 | 001010 | 13 | 111101 | 26 | 1101000 | 39 | 1011011 | 52 |
| 100 | 1 | 001001 | 14 | 111100 | 27 | 1100111 | 40 | 1011010 | 53 |
| 01100 | 2 | 001000 | 15 | 111011 | 28 | 1100110 | 41 | 1011001 | 54 |
| 01101 | 3 | 000111 | 16 | 111010 | 29 | 1100101 | 42 | 1011000 | 55 |
| 01010 | 4 | 000110 | 17 | 111001 | 30 | 1100100 | 43 | 1010111 | 56 |
| 01011 | 5 | 000101 | 18 | 111000 | 31 | 1100011 | 44 | 1010110 | 57 |
| 01000 | 6 | 000100 | 19 | 1101111 | 32 | 1100010 | 45 | 1010101 | 58 |
| 01001 | 7 | 000011 | 20 | 1101110 | 33 | 1100001 | 46 | 1010100 | 59 |
| 001111 | 8 | 000010 | 21 | 1101101 | 34 | 1100000 | 47 | 1010011 | 60 |
| 001110 | 9 | 000001 | 22 | 1101100 | 35 | 1011111 | 48 | 1010010 | 61 |
| 001101 | 10 | 000000 | 23 | 1101011 | 36 | 1011110 | 49 | 1010001 | 62 |
| 001100 | 11 | 111111 | 24 | 1101010 | 37 | 1011101 | 50 | 1010000 | 63 |
| 001011 | 12 | 111110 | 25 | 1101001 | 38 | 1011100 | 51 |  |  |

Table of countOrIndexHiCode

| countOrIndexHiCode | Value |
| --- | --- |
| 001 | 1 |
| 000 | 2 |
| 110 | 3 |
| 101 | 4 |
| 100 | 5 |
| 0111 | 6 |
| 0101 | 7 |
| 1111 | 8 |
| 1110 | 9 |
| 01101 | 10 |
| 01001 | 11 |
| 01000 | 12 |
| 011001 | 13 |
| 0110001 | 14 |
| 0110000 | 15 |

Table of deciSecondsCode

| deciSecondsCode | Value |
| --- | --- |
| 110 | 0 |
| 100 | 0.1 |
| 101 | 0.2 |
| 0110 | 0.3 |
| 0111 | 0.4 |
| 111 | 0.5 |
| 0100 | 0.6 |
| 0101 | 0.7 |
| 0010 | 0.8 |
| 0011 | 0.9 |
| 000 | 1 |

Table of millisecondsCode

| milliSecondsCode | Value | milliSecondsCode | Value | milliSecondsCode | Value | milliSecondsCode | Value | milliSecondsCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1111010 | 0 | 10010 | 0.02 | 10000 | 0.04 | 10110 | 0.06 | 10100 | 0.08 |
| 1111011 | 0.001 | 0101001 | 0.021 | 0111111 | 0.041 | 0001101 | 0.061 | 0010011 | 0.081 |
| 1111000 | 0.002 | 0101110 | 0.022 | 0111100 | 0.042 | 0000010 | 0.062 | 0010000 | 0.082 |
| 1111001 | 0.003 | 0101111 | 0.023 | 0111101 | 0.043 | 0000011 | 0.063 | 0010001 | 0.083 |
| 1111110 | 0.004 | 0101100 | 0.024 | 0110010 | 0.044 | 0000000 | 0.064 | 0010110 | 0.084 |
| 1111111 | 0.005 | 0101101 | 0.025 | 0110011 | 0.045 | 0000001 | 0.065 | 0010111 | 0.085 |
| 1111100 | 0.006 | 0100010 | 0.026 | 0110000 | 0.046 | 0000110 | 0.066 | 0010100 | 0.086 |
| 1111101 | 0.007 | 0100011 | 0.027 | 0110001 | 0.047 | 0000111 | 0.067 | 0010101 | 0.087 |
| 1110010 | 0.008 | 0100000 | 0.028 | 0110110 | 0.048 | 0000100 | 0.068 | 1101010 | 0.088 |
| 1110011 | 0.009 | 0100001 | 0.029 | 0110111 | 0.049 | 0000101 | 0.069 | 1101011 | 0.089 |
| 11001 | 0.01 | 10011 | 0.03 | 10001 | 0.05 | 10111 | 0.07 | 10101 | 0.09 |
| 1110000 | 0.011 | 0100110 | 0.031 | 0110100 | 0.051 | 0011010 | 0.071 | 1101000 | 0.091 |
| 1110001 | 0.012 | 0100111 | 0.032 | 0110101 | 0.052 | 0011011 | 0.072 | 1101001 | 0.092 |
| 1110110 | 0.013 | 0100100 | 0.033 | 0001010 | 0.053 | 0011000 | 0.073 | 1101110 | 0.093 |
| 1110111 | 0.014 | 0100101 | 0.034 | 0001011 | 0.054 | 0011001 | 0.074 | 1101111 | 0.094 |
| 1110100 | 0.015 | 0111010 | 0.035 | 0001000 | 0.055 | 0011110 | 0.075 | 1101100 | 0.095 |
| 1110101 | 0.016 | 0111011 | 0.036 | 0001001 | 0.056 | 0011111 | 0.076 | 1101101 | 0.096 |
| 0101010 | 0.017 | 0111000 | 0.037 | 0001110 | 0.057 | 0011100 | 0.077 | 1100010 | 0.097 |
| 0101011 | 0.018 | 0111001 | 0.038 | 0001111 | 0.058 | 0011101 | 0.078 | 1100011 | 0.098 |
| 0101000 | 0.019 | 0111110 | 0.039 | 0001100 | 0.059 | 0010010 | 0.079 | 110000 | 0.099 |

Table of microsecondsCode

| microseconds Code | Value | microseconds Code | Value | microseconds Code | Value | microsecond sCode | Value | microseconds Code | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110111100 | 0.00001 | 110110110 | 0.00021 | 110011100 | 0.00041 | 110101010 | 0.00061 | 110100000 | 0.00081 |
| 10010 | 0.00002 | 001000 | 0.00022 | 000110 | 0.00042 | 011100 | 0.00062 | 111010 | 0.00082 |
| 110111101 | 0.00003 | 110110111 | 0.00023 | 110011101 | 0.00043 | 110101011 | 0.00063 | 110100001 | 0.00083 |
| 10011 | 0.00004 | 001001 | 0.00024 | 000111 | 0.00044 | 011101 | 0.00064 | 111011 | 0.00084 |
| 1101111110 | 0.00005 | 110110100 | 0.00025 | 110010010 | 0.00045 | 110101000 | 0.00065 | 110100110 | 0.00085 |
| 10000 | 0.00006 | 001110 | 0.00026 | 000100 | 0.00046 | 010010 | 0.00066 | 111000 | 0.00086 |
| 1101111111 | 0.00007 | 110110101 | 0.00027 | 110010011 | 0.00047 | 110101001 | 0.00067 | 110100111 | 0.00087 |
| 10001 | 0.00008 | 001111 | 0.00028 | 000101 | 0.00048 | 010011 | 0.00068 | 111001 | 0.00088 |
| 1101111100 | 0.00009 | 110011010 | 0.00029 | 110010000 | 0.00049 | 110101110 | 0.00069 | 110100100 | 0.00089 |
| 10110 | 0.00010 | 001100 | 0.00030 | 011010 | 0.00050 | 010000 | 0.00070 | 111110 | 0.00090 |
| 1101111101 | 0.00011 | 110011011 | 0.00031 | 110010001 | 0.00051 | 110101111 | 0.00071 | 110100101 | 0.00091 |
| 10111 | 0.00012 | 001101 | 0.00032 | 011011 | 0.00052 | 010001 | 0.00072 | 111111 | 0.00092 |
| 110110010 | 0.00013 | 110011000 | 0.00033 | 110010110 | 0.00053 | 110101100 | 0.00073 | 110111010 | 0.00093 |
| 10100 | 0.00014 | 000010 | 0.00034 | 011000 | 0.00054 | 010110 | 0.00074 | 111100 | 0.00094 |
| 110110011 | 0.00015 | 110011001 | 0.00035 | 110010111 | 0.00055 | 110101101 | 0.00075 | 110111011 | 0.00095 |
| 10101 | 0.00016 | 000011 | 0.00036 | 011001 | 0.00056 | 010111 | 0.00076 | 111101 | 0.00096 |
| 110110000 | 0.00017 | 110011110 | 0.00037 | 110010100 | 0.00057 | 110100010 | 0.00077 | 110111000 | 0.00097 |
| 001010 | 0.00018 | 000000 | 0.00038 | 011110 | 0.00058 | 010100 | 0.00078 | 11000 | 0.00098 |
| 110110001 | 0.00019 | 110011111 | 0.00039 | 110010101 | 0.00059 | 110100011 | 0.00079 | 110111001 | 0.00099 |
| 001011 | 0.00020 | 000001 | 0.00040 | 011111 | 0.00060 | 010101 | 0.00080 |  |  |

Table of secondsCode

| secondsCode | Value | secondsCode | Value | secondsCode | Value | secondsCode | Value | secondsCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0011 | 1 | 1011 | 7 | 01011 | 13 | 10101 | 19 | 101001 | 25 |
| 0001 | 2 | 1001 | 8 | 01001 | 14 | 011111 | 20 | 0101001 | 26 |
| 0000 | 3 | 1000 | 9 | 01000 | 15 | 011110 | 21 | 0101000 | 27 |
| 1111 | 4 | 01110 | 10 | 00101 | 16 | 010101 | 22 | 1010001 | 28 |
| 1101 | 5 | 01101 | 11 | 11101 | 17 | 001001 | 23 | 10100001 | 29 |
| 1100 | 6 | 01100 | 12 | 11100 | 18 | 001000 | 24 | 10100000 | 30 |

Table of frequencyCode

| frequencyCode | Value | frequencyCode | Value | frequencyCode | Value | frequencyCode | Value | frequencyCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100011 | 16 | 011010 | 80 | 011111 | 400 | 1101 | 2000 | 010100 | 10000 |
| 001110 | 20 | 011011 | 100 | 1111 | 500 | 010000 | 2500 | 010101 | 12500 |
| 001111 | 25 | 0001 | 125 | 011100 | 630 | 010001 | 3150 | 0010 | 16000 |
| 1001 | 31.5 | 011000 | 160 | 011101 | 800 | 1010 | 4000 | 10000 | 20000 |
| 001100 | 40 | 011001 | 200 | 1100 | 1000 | 010110 | 5000 | 10001010 | 25000 |
| 001101 | 50 | 1110 | 250 | 010010 | 1250 | 010111 | 6300 | 10001011 | 31500 |
| 0000 | 63 | 011110 | 315 | 010011 | 1600 | 1011 | 8000 | 1000100 | 40000 |

Table of metersCode

| metersCode | Value | metersCode | Value | metersCode | Value | metersCode | Value | metersCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 111101 | 0 | 000001 | 20 | 010101 | 40 | 10011110 | 60 | 11100010 | 80 |
| 110010 | 1 | 000110 | 21 | 101010 | 41 | 10011111 | 61 | 11100011 | 81 |
| 110011 | 2 | 000111 | 22 | 101011 | 42 | 10011100 | 62 | 11100000 | 82 |
| 110000 | 3 | 000100 | 23 | 101000 | 43 | 10011101 | 63 | 11100001 | 83 |
| 110001 | 4 | 000101 | 24 | 101001 | 44 | 10010010 | 64 | 11100110 | 84 |
| 110110 | 5 | 011010 | 25 | 101110 | 45 | 10010011 | 65 | 11100111 | 85 |
| 110111 | 6 | 011011 | 26 | 101111 | 46 | 10010000 | 66 | 11100100 | 86 |
| 110100 | 7 | 011000 | 27 | 101100 | 47 | 10010001 | 67 | 11100101 | 87 |
| 110101 | 8 | 011001 | 28 | 101101 | 48 | 10010110 | 68 | 11111010 | 88 |
| 001010 | 9 | 011110 | 29 | 10000 | 49 | 10010111 | 69 | 11111011 | 89 |
| 001011 | 10 | 011111 | 30 | 1000100 | 50 | 10010100 | 70 | 11111000 | 90 |
| 001000 | 11 | 011100 | 31 | 1000101 | 51 | 10010101 | 71 | 11111001 | 91 |
| 001001 | 12 | 011101 | 32 | 10001110 | 52 | 11101010 | 72 | 11111110 | 92 |
| 001110 | 13 | 010010 | 33 | 10001111 | 53 | 11101011 | 73 | 11111111 | 93 |
| 001111 | 14 | 010011 | 34 | 10001100 | 54 | 11101000 | 74 | 11111100 | 94 |
| 001100 | 15 | 010000 | 35 | 10001101 | 55 | 11101001 | 75 | 11111101 | 95 |
| 001101 | 16 | 010001 | 36 | 10011010 | 56 | 11101110 | 76 | 11110010 | 96 |
| 000010 | 17 | 010110 | 37 | 10011011 | 57 | 11101111 | 77 | 11110011 | 97 |
| 000011 | 18 | 010111 | 38 | 10011000 | 58 | 11101100 | 78 | 11110000 | 98 |
| 000000 | 19 | 010100 | 39 | 10011001 | 59 | 11101101 | 79 | 11110001 | 99 |

Table of hectometersCode

| hectometersCode | Value |
| --- | --- |
| 000 | 0 |
| 001 | 1 |
| 110 | 2 |
| 111 | 3 |
| 100 | 4 |
| 101 | 5 |
| 0110 | 6 |
| 0111 | 7 |
| 0100 | 8 |
| 0101 | 9 |

Table of kilometersCode

| kilometersCode | Value |
| --- | --- |
| 10 | 1 |
| 011 | 2 |
| 001 | 3 |
| 000 | 4 |
| 111 | 5 |
| 0101 | 6 |
| 0100 | 7 |
| 1101 | 8 |
| 11001 | 9 |
| 11000 | 10 |

Table of centimetersCode

| centimetersCode | Value | centimetersCode | Value | centimetersCode | Value | centimetersCode | Value | centimetersCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110010 | 0 | 0100110 | 20 | 0001010 | 40 | 0011110 | 60 | 100010 | 80 |
| 110011 | 1 | 0100111 | 21 | 0001011 | 41 | 0011111 | 61 | 100011 | 81 |
| 110000 | 2 | 0100100 | 22 | 0001000 | 42 | 0011100 | 62 | 100000 | 82 |
| 110001 | 3 | 0100101 | 23 | 0001001 | 43 | 0011101 | 63 | 100001 | 83 |
| 110110 | 4 | 0111010 | 24 | 0001110 | 44 | 0010010 | 64 | 100110 | 84 |
| 110111 | 5 | 0111011 | 25 | 0001111 | 45 | 0010011 | 65 | 100111 | 85 |
| 110100 | 6 | 0111000 | 26 | 0001100 | 46 | 0010000 | 66 | 100100 | 86 |
| 110101 | 7 | 0111001 | 27 | 0001101 | 47 | 0010001 | 67 | 100101 | 87 |
| 0101010 | 8 | 0111110 | 28 | 0000010 | 48 | 0010110 | 68 | 1111010 | 88 |
| 0101011 | 9 | 0111111 | 29 | 0000011 | 49 | 0010111 | 69 | 1111011 | 89 |
| 0101000 | 10 | 0111100 | 30 | 0000000 | 50 | 0010100 | 70 | 1111000 | 90 |
| 0101001 | 11 | 0111101 | 31 | 0000001 | 51 | 0010101 | 71 | 1111001 | 91 |
| 0101110 | 12 | 0110010 | 32 | 0000110 | 52 | 101010 | 72 | 1111110 | 92 |
| 0101111 | 13 | 0110011 | 33 | 0000111 | 53 | 101011 | 73 | 1111111 | 93 |
| 0101100 | 14 | 0110000 | 34 | 0000100 | 54 | 101000 | 74 | 1111100 | 94 |
| 0101101 | 15 | 0110001 | 35 | 0000101 | 55 | 101001 | 75 | 1111101 | 95 |
| 0100010 | 16 | 0110110 | 36 | 0011010 | 56 | 101110 | 76 | 111010 | 96 |
| 0100011 | 17 | 0110111 | 37 | 0011011 | 57 | 101111 | 77 | 111011 | 97 |
| 0100000 | 18 | 0110100 | 38 | 0011000 | 58 | 101100 | 78 | 111000 | 98 |
| 0100001 | 19 | 0110101 | 39 | 0011001 | 59 | 101101 | 79 | 111001 | 99 |

Table of millimetersCode

| millimetersCode | Value |
| --- | --- |
| 000 | 0 |
| 001 | 1 |
| 0110 | 2 |
| 0111 | 3 |
| 0100 | 4 |
| 0101 | 5 |
| 110 | 6 |
| 111 | 7 |
| 100 | 8 |
| 101 | 9 |

Table of orientationCode

| orientationCode | Value | Description |
| --- | --- | --- |
| 01 | [ 0, 0, 0] | Default orientation: (yaw, pitch, roll) |
| 0010 | [ 90, 0, 0] | Default orientation: (yaw, pitch, roll) |
| 0011 | [ 180, 0, 0] | Default orientation: (yaw, pitch, roll) |
| 0000 | [-90, 0, 0] | Default orientation: (yaw, pitch, roll) |
| 0001 | [ 0, 90, 0] | Default orientation: (yaw, pitch, roll) |
| 1010 | [ 0, -90, 0] | Default orientation: (yaw, pitch, roll) |
| 1011 | [ 0, 0, 180] | Default orientation: (yaw, pitch, roll) |
| 110 | N/A | Default angles transmitted |
| 111 | N/A | Explicit angles transmitted |
| 100 | N/A | High resolution angles transmitted |

Table of defaultYawCode

| defaultYawCode | Value |
| --- | --- |
| 010 | -135 |
| 011 | -90 |
| 000 | -45 |
| 001 | 0 |
| 110 | 45 |
| 111 | 90 |
| 100 | 135 |
| 101 | 180 |

Table of defaultPitchCode

| defaultPitchCode | Value |
| --- | --- |
| 11 | -90 |
| 100 | -45 |
| 00 | 0 |
| 101 | 45 |
| 01 | 90 |

Table of defaultRollCode

| defaultRollCode | Value |
| --- | --- |
| 010 | -135 |
| 011 | -90 |
| 000 | -45 |
| 001 | 0 |
| 110 | 45 |
| 111 | 90 |
| 100 | 135 |
| 101 | 180 |

Table of coarseAngleCode

| coarseAngleCode | Value | coarseAngleCode | Value | coarseAngleCode | Value | coarseAngleCode | Value | coarseAngleCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 010010 | -180 | 0111101 | -105 | 101100 | -30 | 111111 | 45 | 001110 | 120 |
| 010011 | -175 | 0110010 | -100 | 101101 | -25 | 111100 | 50 | 001111 | 125 |
| 010000 | -170 | 0110011 | -95 | 100010 | -20 | 111101 | 55 | 001100 | 130 |
| 010001 | -165 | 0110000 | -90 | 100011 | -15 | 110010 | 60 | 001101 | 135 |
| 010110 | -160 | 0110001 | -85 | 100000 | -10 | 110011 | 65 | 000010 | 140 |
| 010111 | -155 | 0110110 | -80 | 100001 | -5 | 110000 | 70 | 000011 | 145 |
| 010100 | -150 | 0110111 | -75 | 100110 | 0 | 110001 | 75 | 000000 | 150 |
| 010101 | -145 | 0110100 | -70 | 100111 | 5 | 110110 | 80 | 000001 | 155 |
| 0111010 | -140 | 0110101 | -65 | 100100 | 10 | 110111 | 85 | 000110 | 160 |
| 0111011 | -135 | 101010 | -60 | 100101 | 15 | 110100 | 90 | 000111 | 165 |
| 0111000 | -130 | 101011 | -55 | 111010 | 20 | 110101 | 95 | 000100 | 170 |
| 0111001 | -125 | 101000 | -50 | 111011 | 25 | 001010 | 100 | 000101 | 175 |
| 0111110 | -120 | 101001 | -45 | 111000 | 30 | 001011 | 105 |  |  |
| 0111111 | -115 | 101110 | -40 | 111001 | 35 | 001000 | 110 |  |  |
| 0111100 | -110 | 101111 | -35 | 111110 | 40 | 001001 | 115 |  |  |

Table of fineAngleCode

| fineAngleCode | Value | fineAngleCode | Value | fineAngleCode | Value | fineAngleCode | Value | fineAngleCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11100 | 0.1 | 10110 | 1.1 | 001100 | 2.1 | 011010 | 3.1 | 010000 | 4.1 |
| 11101 | 0.2 | 10111 | 1.2 | 001101 | 2.2 | 011011 | 3.2 | 010001 | 4.2 |
| 111110 | 0.3 | 10100 | 1.3 | 000010 | 2.3 | 011000 | 3.3 | 010110 | 4.3 |
| 111111 | 0.4 | 10101 | 1.4 | 000011 | 2.4 | 011001 | 3.4 | 010111 | 4.4 |
| 111100 | 0.5 | 001010 | 1.5 | 000000 | 2.5 | 011110 | 3.5 | 010100 | 4.5 |
| 111101 | 0.6 | 001011 | 1.6 | 000001 | 2.6 | 011111 | 3.6 | 010101 | 4.6 |
| 10010 | 0.7 | 001000 | 1.7 | 000110 | 2.7 | 011100 | 3.7 | 11010 | 4.7 |
| 10011 | 0.8 | 001001 | 1.8 | 000111 | 2.8 | 011101 | 3.8 | 11011 | 4.8 |
| 10000 | 0.9 | 001110 | 1.9 | 000100 | 2.9 | 010010 | 3.9 | 11000 | 4.9 |
| 10001 | 1 | 001111 | 2 | 000101 | 3 | 010011 | 4 | 11001 | 5 |

Table of coarseGainCode

| coarseGainCode | Value |
| --- | --- |
| 011011011 | -90 |
| 0110100 | -60 |
| 0110101 | -30 |
| 011110 | -24 |
| 011111 | -18 |
| 01110 | -15 |
| 1100 | -12 |
| 111 | -9 |
| 100 | -6 |
| 101 | -3 |
| 00 | 0 |
| 010 | 3 |
| 1101 | 6 |
| 01100 | 9 |
| 01101110 | 12 |
| 01101111 | 15 |
| 01101100 | 24 |
| 011011010 | 36 |

Table of fineGainCode

| fineGainCode | Value | fineGainCode | Value | fineGainCode | Value | fineGainCode | Value | fineGainCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000110 | -29 | 010000 | -23 | 001010 | -17 | 001100 | -11 | 01110 | -5 |
| 000111 | -28 | 010001 | -22 | 001011 | -16 | 001101 | -10 | 01111 | -4 |
| 000100 | -27 | 010110 | -21 | 001000 | -15 | 011010 | -9 | 0000 | -3 |
| 000101 | -26 | 010111 | -20 | 001001 | -14 | 011011 | -8 | 10 | -2 |
| 010010 | -25 | 010100 | -19 | 001110 | -13 | 011000 | -7 | 11 | -1 |
| 010011 | -24 | 010101 | -18 | 001111 | -12 | 011001 | -6 |  |  |

Table of decimalCode

| decimalCode | Value |
| --- | --- |
| 100 | 0.1 |
| 101 | 0.2 |
| 0110 | 0.3 |
| 0111 | 0.4 |
| 111 | 0.5 |
| 0100 | 0.6 |
| 0101 | 0.7 |
| 0010 | 0.8 |
| 0011 | 0.9 |

Table of dsrCode

| dsrCode | Value | dsrCode | Value | dsrCode | Value | dsrCode | Value | dsrCode | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10001100 | -150 | 011100101 | -121 | 011000110 | -92 | 110001 | -63 | 010010 | -34 |
| 10001101 | -149 | 011111010 | -120 | 011000111 | -91 | 110110 | -62 | 010011 | -33 |
| 100011110 | -148 | 011111011 | -119 | 011000100 | -90 | 110111 | -61 | 010000 | -32 |
| 100011111 | -147 | 011111000 | -118 | 011000101 | -89 | 110100 | -60 | 010001 | -31 |
| 100011100 | -146 | 011111001 | -117 | 011011010 | -88 | 110101 | -59 | 010110 | -30 |
| 100011101 | -145 | 011111110 | -116 | 011011011 | -87 | 001010 | -58 | 011010010 | -29 |
| 10000010 | -144 | 011111111 | -115 | 011011000 | -86 | 001011 | -57 | 011010011 | -28 |
| 10000011 | -143 | 011111100 | -114 | 011011001 | -85 | 001000 | -56 | 011010000 | -27 |
| 10000000 | -142 | 011111101 | -113 | 011011110 | -84 | 001001 | -55 | 011010001 | -26 |
| 10000001 | -141 | 011110010 | -112 | 011011111 | -83 | 001110 | -54 | 011010110 | -25 |
| 10000110 | -140 | 011110011 | -111 | 011011100 | -82 | 001111 | -53 | 011010111 | -24 |
| 10000111 | -139 | 011110000 | -110 | 011011101 | -81 | 001100 | -52 | 011010100 | -23 |
| 10000100 | -138 | 011110001 | -109 | 010100 | -80 | 001101 | -51 | 011010101 | -22 |
| 10000101 | -137 | 011110110 | -108 | 010101 | -79 | 000010 | -50 | 010111010 | -21 |
| 011101010 | -136 | 011110111 | -107 | 100110 | -78 | 000011 | -49 | 010111011 | -20 |
| 011101011 | -135 | 011110100 | -106 | 100111 | -77 | 000000 | -48 | 010111000 | -19 |
| 011101000 | -134 | 011110101 | -105 | 100100 | -76 | 000001 | -47 | 010111001 | -18 |
| 011101001 | -133 | 011001010 | -104 | 100101 | -75 | 000110 | -46 | 010111110 | -17 |
| 011101110 | -132 | 011001011 | -103 | 111010 | -74 | 000111 | -45 | 010111111 | -16 |
| 011101111 | -131 | 011001000 | -102 | 111011 | -73 | 000100 | -44 | 010111100 | -15 |
| 011101100 | -130 | 011001001 | -101 | 111000 | -72 | 000101 | -43 | 010111101 | -14 |
| 011101101 | -129 | 011001110 | -100 | 111001 | -71 | 101010 | -42 | 10001010 | -13 |
| 011100010 | -128 | 011001111 | -99 | 111110 | -70 | 101011 | -41 | 10001011 | -12 |
| 011100011 | -127 | 011001100 | -98 | 111111 | -69 | 101000 | -40 | 10001000 | -11 |
| 011100000 | -126 | 011001101 | -97 | 111100 | -68 | 101001 | -39 | 10001001 | -10 |
| 011100001 | -125 | 011000010 | -96 | 111101 | -67 | 101110 | -38 |  |  |
| 011100110 | -124 | 011000011 | -95 | 110010 | -66 | 101111 | -37 |  |  |
| 011100111 | -123 | 011000000 | -94 | 110011 | -65 | 101100 | -36 |  |  |
| 011100100 | -122 | 011000001 | -93 | 110000 | -64 | 101101 | -35 |  |  |

Table of delayLineCountCode

| delayLineCountCode | Value |
| --- | --- |
| 100 | 3 |
| 01 | 7 |
| 00 | 15 |
| 11 | 31 |
| 101 | 63 |

Table of portal2PortalCode

| portal2PortalCode | Value | portal2PortalCode | Value | portal2PortalCode | Value |
| --- | --- | --- | --- | --- | --- |
| 1000 | -6 | 10 | -16 | 11000 | -39 |
| 1001 | -7 | 11 | -17 | 11001 | -42 |
| 1010 | -8 | 0 | -18 | 11110 | -48 |
| 1011 | -9 | 1 | -20 | 11111 | -54 |
| 1000 | -10 | 110 | -22 | 11100 | -60 |
| 1001 | -11 | 111 | -24 | 11101 | -66 |
| 1110 | -12 | 100 | -27 | 10110 | -72 |
| 1111 | -13 | 101 | -30 | 10111 | -78 |
| 1100 | -14 | 11010 | -33 | 1010 | -90 |
| 1101 | -15 | 11011 | -36 |  |  |

Airflow default frequency profiles

Three defaults are provided, using a combination of different speed, azimuth, and elevation profiles.:

Table of default airflow profile mappings.

| Default Number | Speed Profile | Azimuth Profile | Elevation Profile |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |
| 2 | 0 | 0 | 0 |

Two default velocity-dependent speed profiles are defined.

const SpeedProfiles defaultSpeedProfiles0 = {

{ 0.f,

{ 0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 3.6f,

{ 0.000000000000000f, 0.058210321777087f, 0.074131024130092f, 0.098855309465694f,

0.092257142715476f, 0.087096358995608f, 0.066069344800760f, 0.042169650342858f,

0.026607250597988f, 0.041686938347034f, 0.023713737056617f, 0.025703957827689f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 5.2f,

{ 0.000000000000000f, 0.105925372517729f, 0.139636836105594f, 0.175792361395869f,

0.158489319246111f, 0.160324539069004f, 0.151356124843621f, 0.105925372517729f,

0.069984199600227f, 0.056885293084384f, 0.033113112148259f, 0.031260793671240f,

0.023988329190195f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 6.4f,

{ 0.000000000000000f, 0.153108746168203f, 0.144543977074593f, 0.162181009735893f,

0.164058977319954f, 0.201836636368156f, 0.254097270554930f, 0.248313310529557f,

0.177827941003892f, 0.108392691402120f, 0.059566214352901f, 0.042657951880159f,

0.029174270140012f, 0.022646443075931f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 7.6f,

{ 0.000000000000000f, 0.389045144994280f, 0.254097270554930f, 0.272270130807791f,

0.184077200146896f, 0.237137370566166f, 0.316227766016838f, 0.389045144994280f,

0.416869383470335f, 0.319889510969140f, 0.158489319246111f, 0.082224264994707f,

0.057543993733716f, 0.048417236758410f, 0.029853826189180f, 0.024547089156850f,

0.020183663636816f, 0.021877616239496f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 8.8f,

{ 0.000000000000000f, 0.653130552647472f, 0.602559586074357f, 0.398107170553497f,

0.350751873952568f, 0.402717034325459f, 0.512861383991364f, 0.716143410212902f,

0.645654229034655f, 0.653130552647472f, 0.412097519097330f, 0.242661009508241f,

0.118850222743702f, 0.074989420933246f, 0.047315125896148f, 0.038904514499428f,

0.029512092266664f, 0.027861211686298f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } },

{ 9.5f,

{ 0.000000000000000f, 1.000000000000000f, 0.912010839355909f, 0.645654229034655f,

0.495450190804790f, 0.462381021399260f, 0.518800038928961f, 0.653130552647472f,

0.749894209332456f, 0.794328234724281f, 0.660693448007595f, 0.441570447353312f,

0.254097270554930f, 0.156675107010815f, 0.098855309465694f, 0.081283051616410f,

0.057543993733716f, 0.043151907682776f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f, 0.000000000000000f,

0.000000000000000f, 0.000000000000000f, 0.000000000000000f } }

};

const SpeedProfiles defaultSpeedProfiles1 = {

{ 0.0,

{ 0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f } },

{ 2.5,

{ 0.000005092005791f, 0.000006900632336f, 0.000009432110693f, 0.000012327893926f,

0.000013768571649f, 0.000011422703105f, 0.000005092005791f, 0.000003265042877f,

0.000003127079258f, 0.000002546178748f, 0.000001636397807f, 0.000000690063234f,

0.000000134010519f, 0.000000006514617f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f } },

{ 5.0,

{ 0.001496142877377f, 0.001932130724140f, 0.002599744534007f, 0.003606337868175f,

0.004852356241708f, 0.006188018709359f, 0.006667604793840f, 0.005114226348140f,

0.002146303046534f, 0.000565625465467f, 0.001007093370471f, 0.001231117371727f,

0.000673559958237f, 0.000364566234870f, 0.000397605366962f, 0.000435153296683f,

0.000471188518773f, 0.000489349224973f, 0.000471323990286f, 0.000381726075753f,

0.000239271349497f, 0.000095059565827f, 0.000016297468792f, 0.000000621138043f,

0.000000003297720f, 0.000000000100000f, 0.000000000100000f, 0.000000000100000f,

0.000000000100000f, 0.000000000100000f, 0.000000000100000f } },

{ 7.5,

{ 0.051951034630881f, 0.042518504056564f, 0.023092524650082f, 0.011082702747709f,

0.014740224014510f, 0.020215870905670f, 0.027825971169683f, 0.035377220188290f,

0.038851991245055f, 0.030132004936994f, 0.012995742071336f, 0.001989110184054f,

0.000493645557350f, 0.004020371131487f, 0.014186241985814f, 0.001280391940051f,

0.000656095417394f, 0.000723913741598f, 0.000805988598663f, 0.000909941448295f,

0.001002775807613f, 0.001063925155869f, 0.001036166061737f, 0.000839126876812f,

0.000506914119304f, 0.000174087953424f, 0.000021253678937f, 0.000000643304836f,

0.000000001713468f, 0.000000000100000f, 0.000000000100000f } },

{ 10.0,

{ 0.267636569489333f, 0.284803586843580f, 0.256398484904269f, 0.162759165528627f,

0.060192925930007f, 0.065087648200385f, 0.090062802021128f, 0.121841561034339f,

0.157443080882683f, 0.179698914853259f, 0.151197793916446f, 0.074964898022991f,

0.013439205027281f, 0.000996370611024f, 0.001049346779756f, 0.002996652360232f,

0.111897800202304f, 0.003820805076652f, 0.001477250652030f, 0.001684887503003f,

0.001915759152607f, 0.002176677229576f, 0.002433305794620f, 0.002565020905680f,

0.002384533395304f, 0.001743668950299f, 0.000798161745722f, 0.000185498716540f,

0.000013172525079f, 0.000000070160215f, 0.000000000100000f } },

{ 12.5,

{ 0.501502066903696f, 0.533669923120632f, 0.480443948208625f, 0.304980960097758f,

0.112790553340592f, 0.121962369210889f, 0.168761247578815f, 0.228308839894651f,

0.295019588066788f, 0.336722957521143f, 0.283317060538691f, 0.140470532018433f,

0.025182616529512f, 0.003498448253642f, 0.003684457739404f, 0.003932544862838f,

0.004266438154341f, 0.004670441692359f, 0.099862866925050f, 0.067786280289128f,

0.006726597701381f, 0.007642731096543f, 0.008543803192886f, 0.009006280202113f,

0.008372553947550f, 0.006122356005497f, 0.002802498924147f, 0.000651321560222f,

0.000046251261176f, 0.000000246345966f, 0.000000000100000f } },

{ 15.0,

{ 0.939723310564639f, 1.000000000000000f, 0.900264240861151f, 0.571478636671866f,

0.211348903983665f, 0.228535212360695f, 0.316227766016838f, 0.427809081987638f,

0.552812844204640f, 0.630957344480193f, 0.530884444230988f, 0.263216130294607f,

0.047187625606211f, 0.012283722590767f, 0.012936837559659f, 0.013807919016826f,

0.014980282382062f, 0.016398816265346f, 0.018212236496936f, 0.031622776601684f,

0.501187233627272f, 0.026835094253010f, 0.029998931149624f, 0.031622776601684f,

0.029397642214907f, 0.021496765800423f, 0.009840111057611f, 0.002286914878569f,

0.000162397045938f, 0.000000864967919f, 0.000000000244343f } },

};

Two default sets of azimuth-dependent profiles and one set of elevation-dependent profiles are provided.

const AzimuthProfiles defaultAzimuthProfiles0 = {

{ 0.0f,

{ { 0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f },

{ 0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f } } },

{ 30.0f,

{ { 0.618728327553234f, 0.618728327553234f, 0.787045789695099f, 0.587489352529776f,

0.934329368257315f, 0.655390285833477f, 0.540132114764635f, 0.431022558004996f,

0.309741929921658f, 0.285430251378658f, 0.375837404288445f, 0.519995996533516f,

0.493173803954936f, 0.398565772313401f, 0.461317574560380f, 0.474241985260245f,

0.401790810848940f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f },

{ 0.416869383470336f, 0.416869383470336f, 0.555904257270404f, 0.418311688651495f,

0.574777818845522f, 0.500034534976979f, 0.312968047156367f, 0.201372424986239f,

0.146049474944163f, 0.138038426460288f, 0.171593171012100f, 0.267300640866331f,

0.272270130807791f, 0.225164541674462f, 0.248885731828239f, 0.242661009508242f,

0.221309470960564f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f } } },

{ 60.0f,

{ { 0.701455298419971f, 0.701455298419971f, 1.000000000000000f, 0.836565595755810f,

0.970509967245491f, 0.729457510254568f, 0.382384253658113f, 0.242381796455463f,

0.178443198854037f, 0.194984459975804f, 0.216023003477475f, 0.281190083039894f,

0.247742205763329f, 0.222843514927031f, 0.239056193534463f, 0.251767692775886f,

0.223100220748196f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f },

{ 0.368553041852479f, 0.368553041852479f, 0.698232404077171f, 0.542625251329014f,

0.678422120498261f, 0.468813382145265f, 0.276375791917037f, 0.204173794466953f,

0.142724983016489f, 0.135207256319428f, 0.149623565609444f, 0.183442516711773f,

0.148593564228701f, 0.127350308101666f, 0.195659076793494f, 0.242661009508242f,

0.279254384123734f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f } } },

{ 90.0f,

{ { 0.185139889420527f, 0.185139889420527f, 0.212569034521711f, 0.175388050184176f,

0.194984459975805f, 0.112460497396693f, 0.059156163417547f, 0.043802611117036f,

0.038904514499428f, 0.058748935252978f, 0.085703784523037f, 0.143879857825585f,

0.159220872705117f, 0.157398286446622f, 0.186208713666287f, 0.202534960682497f,

0.225164541674462f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f },

{ 0.432513831035009f, 0.432513831035009f, 0.603253707926613f, 0.470435405725976f,

0.498884487460013f, 0.306902198839116f, 0.123452531517069f, 0.067608297539198f,

0.038994198667654f, 0.045341929777901f, 0.040318094644347f, 0.082794216371233f,

0.098174794301998f, 0.115611224219210f, 0.177623326828844f, 0.220292646305346f,

0.253512863049791f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f } } },

{ 120.0f,

{ { 0.166532882778502f, 0.166532882778502f, 0.312248240622327f, 0.207491351745491f,

0.260015956316528f, 0.144045601024638f, 0.080445174855213f, 0.060673632958851f,

0.037583740428844f, 0.048809010124758f, 0.064194822150804f, 0.114947624341984f,

0.128973355789875f, 0.117489755493953f, 0.180509473713157f, 0.223872113856834f,

0.257632115700258f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f },

{ 0.393097244907467f, 0.393097244907467f, 0.564936974812303f, 0.443098214489298f,

0.532108259266795f, 0.296483138952434f, 0.201836636368156f, 0.123168598573636f,

0.121478662047122f, 0.127790920956629f, 0.231206479017560f, 0.289734358770133f,

0.365594791613125f, 0.326964046834306f, 0.354405130280993f, 0.266072505979881f,

0.266685866452148f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f } } },

{ 150.0f,

{ { 0.199296650616472f, 0.199296650616472f, 0.200216562158904f, 0.145881426027535f,

0.146049474944163f, 0.073029781287336f, 0.040318094644347f, 0.028575905433749f,

0.021037784397665f, 0.038949330770365f, 0.039856577231340f, 0.086996143306527f,

0.102683343166809f, 0.120920518294321f, 0.185780445509170f, 0.230409297605585f,

0.265155109300921f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f },

{ 0.227247963527084f, 0.227247963527084f, 0.329989407910835f, 0.210377843976647f,

0.203704207770572f, 0.140604752412991f, 0.087297136838811f, 0.066450767290778f,

0.045551218616574f, 0.053888984537329f, 0.070307231988383f, 0.104231742939330f,

0.110280823384955f, 0.129867356382916f, 0.199526231496888f, 0.247457146133381f,

0.284773780017375f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f } } },

{ 180.0f,

{ { 0.347136275887698f, 0.347136275887698f, 0.352776786618874f, 0.252638771013190f,

0.212569034521711f, 0.112590046889494f, 0.072443596007499f, 0.049317380395494f,

0.040597566325157f, 0.054954087385762f, 0.073620709749474f, 0.122320711904993f,

0.127643880881134f, 0.118576874816716f, 0.182179707309187f, 0.225943577022098f,

0.260015956316527f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f },

{ 0.347136275887698f, 0.347136275887698f, 0.352776786618874f, 0.252638771013190f,

0.212569034521711f, 0.112590046889494f, 0.072443596007499f, 0.049317380395494f,

0.040597566325157f, 0.054954087385762f, 0.073620709749474f, 0.122320711904993f,

0.127643880881134f, 0.118576874816716f, 0.182179707309187f, 0.225943577022098f,

0.260015956316527f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f, 0.250610925303211f,

0.250610925303211f, 0.250610925303211f, 0.250610925303211f } } },

{ 210.0f,

{ { 0.227247963527084f, 0.227247963527084f, 0.329989407910835f, 0.210377843976647f,

0.203704207770572f, 0.140604752412991f, 0.087297136838811f, 0.066450767290778f,

0.045551218616574f, 0.053888984537329f, 0.070307231988383f, 0.104231742939330f,

0.110280823384955f, 0.129867356382916f, 0.199526231496888f, 0.247457146133381f,

0.284773780017375f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f, 0.274473234347854f,

0.274473234347854f, 0.274473234347854f, 0.274473234347854f },

{ 0.199296650616472f, 0.199296650616472f, 0.200216562158904f, 0.145881426027535f,

0.146049474944163f, 0.073029781287336f, 0.040318094644347f, 0.028575905433749f,

0.021037784397665f, 0.038949330770365f, 0.039856577231340f, 0.086996143306527f,

0.102683343166809f, 0.120920518294321f, 0.185780445509170f, 0.230409297605585f,

0.265155109300921f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f, 0.255564190106414f,

0.255564190106414f, 0.255564190106414f, 0.255564190106414f } } },

{ 240.0f,

{ { 0.393097244907467f, 0.393097244907467f, 0.564936974812303f, 0.443098214489298f,

0.532108259266795f, 0.296483138952434f, 0.201836636368156f, 0.123168598573636f,

0.121478662047122f, 0.127790920956629f, 0.231206479017560f, 0.289734358770133f,

0.365594791613125f, 0.326964046834306f, 0.354405130280993f, 0.266072505979881f,

0.266685866452148f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f, 0.257039578276886f,

0.257039578276886f, 0.257039578276886f, 0.257039578276886f },

{ 0.166532882778502f, 0.166532882778502f, 0.312248240622327f, 0.207491351745491f,

0.260015956316528f, 0.144045601024638f, 0.080445174855213f, 0.060673632958851f,

0.037583740428844f, 0.048809010124758f, 0.064194822150804f, 0.114947624341984f,

0.128973355789875f, 0.117489755493953f, 0.180509473713157f, 0.223872113856834f,

0.257632115700258f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f, 0.248313310529557f,

0.248313310529557f, 0.248313310529557f, 0.248313310529557f } } },

{ 270.0f,

{ { 0.432513831035009f, 0.432513831035009f, 0.603253707926613f, 0.470435405725976f,

0.498884487460013f, 0.306902198839116f, 0.123452531517069f, 0.067608297539198f,

0.038994198667654f, 0.045341929777901f, 0.040318094644347f, 0.082794216371233f,

0.098174794301998f, 0.115611224219210f, 0.177623326828844f, 0.220292646305346f,

0.253512863049791f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f, 0.244343055269397f,

0.244343055269397f, 0.244343055269397f, 0.244343055269397f },

{ 0.185139889420527f, 0.185139889420527f, 0.212569034521711f, 0.175388050184176f,

0.194984459975805f, 0.112460497396693f, 0.059156163417547f, 0.043802611117036f,

0.038904514499428f, 0.058748935252978f, 0.085703784523037f, 0.143879857825585f,

0.159220872705117f, 0.157398286446622f, 0.186208713666287f, 0.202534960682497f,

0.225164541674462f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f, 0.245188445380952f,

0.245188445380952f, 0.245188445380952f, 0.245188445380952f } } },

{ 300.0f,

{ { 0.368553041852479f, 0.368553041852479f, 0.698232404077171f, 0.542625251329014f,

0.678422120498261f, 0.468813382145265f, 0.276375791917037f, 0.204173794466953f,

0.142724983016489f, 0.135207256319428f, 0.149623565609444f, 0.183442516711773f,

0.148593564228701f, 0.127350308101666f, 0.195659076793494f, 0.242661009508242f,

0.279254384123734f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f, 0.269153480392692f,

0.269153480392692f, 0.269153480392692f, 0.269153480392692f },

{ 0.701455298419971f, 0.701455298419971f, 1.000000000000000f, 0.836565595755810f,

0.970509967245491f, 0.729457510254568f, 0.382384253658113f, 0.242381796455463f,

0.178443198854037f, 0.194984459975804f, 0.216023003477475f, 0.281190083039894f,

0.247742205763329f, 0.222843514927031f, 0.239056193534463f, 0.251767692775886f,

0.223100220748196f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f, 0.233345806228100f,

0.233345806228100f, 0.233345806228100f, 0.233345806228100f } } },

{ 330.0f,

{ { 0.416869383470336f, 0.416869383470336f, 0.555904257270404f, 0.418311688651495f,

0.574777818845522f, 0.500034534976979f, 0.312968047156367f, 0.201372424986239f,

0.146049474944163f, 0.138038426460288f, 0.171593171012100f, 0.267300640866331f,

0.272270130807791f, 0.225164541674462f, 0.248885731828239f, 0.242661009508242f,

0.221309470960564f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f, 0.226464430759306f,

0.226464430759306f, 0.226464430759306f, 0.226464430759306f },

{ 0.618728327553234f, 0.618728327553234f, 0.787045789695099f, 0.587489352529776f,

0.934329368257315f, 0.655390285833477f, 0.540132114764635f, 0.431022558004996f,

0.309741929921658f, 0.285430251378658f, 0.375837404288445f, 0.519995996533516f,

0.493173803954936f, 0.398565772313401f, 0.461317574560380f, 0.474241985260245f,

0.401790810848940f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f, 0.409260659730011f,

0.409260659730011f, 0.409260659730011f, 0.409260659730011f } } },

{ 360.0f,

{ { 0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f },

{ 0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f, 0.229879370926401f,

0.229879370926401f, 0.229879370926401f, 0.229879370926401f } } }

};

const AzimuthProfiles defaultAzimuthProfiles1 = {

{ 0.0f,

{ { 0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f },

{ 0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f } } },

{ 30.0f,

{ { 0.697629761974222f, 0.697629761974222f, 0.835603018231248f, 0.671042458632901f,

0.950331226974181f, 0.728408491522093f, 0.630049976930408f, 0.531955128236972f,

0.415192978147146f, 0.390503562294295f, 0.480009684908112f, 0.612350391724774f,

0.588504787243298f, 0.501620180363360f, 0.559757601495111f, 0.571478636671867f,

0.504661297563529f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f },

{ 0.518800038928961f, 0.518800038928961f, 0.643798558586410f, 0.520145684953132f,

0.660123205654783f, 0.594634359165687f, 0.418432105762394f, 0.300607630262823f,

0.236251731414293f, 0.226464430759306f, 0.266609119134929f, 0.371749163536557f,

0.376920709547802f, 0.326869952558687f, 0.352370871042487f, 0.345740296418239f,

0.322663618591148f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f } } },

{ 60.0f,

{ { 0.766478540162591f, 0.766478540162591f, 1.000000000000000f, 0.874731970892050f,

0.977799925864087f, 0.789314352732207f, 0.486267226589422f, 0.345441889099858f,

0.274552245465151f, 0.293426950395597f, 0.316865531946885f, 0.386144630346289f,

0.351155924516291f, 0.324339617349349f, 0.341881027081702f, 0.355426659637028f,

0.324619795632621f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f },

{ 0.473015094678584f, 0.473015094678584f, 0.763835783577690f, 0.632229854806186f,

0.747523757863696f, 0.566565336409729f, 0.381175518739995f, 0.303738609194611f,

0.232206835516786f, 0.222971830894677f, 0.240574726093395f, 0.280301227735976f,

0.239331575640539f, 0.213181738713594f, 0.294188031312244f, 0.345740296418239f,

0.384149259178759f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f } } },

{ 90.0f,

{ { 0.282244183025396f, 0.282244183025396f, 0.313058139566081f, 0.271019163189084f,

0.293426950395597f, 0.194200346295108f, 0.119949930314938f, 0.095746961354662f,

0.087599171763312f, 0.119330098744878f, 0.158398111714294f, 0.233614610221893f,

0.252057717963911f, 0.249890646139567f, 0.283465363348967f, 0.301908263350306f,

0.326869952558687f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f },

{ 0.533334895487621f, 0.533334895487621f, 0.684502439096536f, 0.568034874449755f,

0.593608348191371f, 0.412334809789612f, 0.208269176583772f, 0.132586711776056f,

0.087750580873113f, 0.098259601842788f, 0.089975651500707f, 0.154347642332453f,

0.175388050184176f, 0.198266801282380f, 0.273605611323968f, 0.321551100290820f,

0.357272838151929f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f } } },

{ 120.0f,

{ { 0.260690376924823f, 0.260690376924823f, 0.417710122705241f, 0.307432658063637f,

0.364124582768118f, 0.233816416405714f, 0.151051485272583f, 0.122250318705779f,

0.085359133929137f, 0.103842467675821f, 0.127533711865269f, 0.197412659832141f,

0.215216220349500f, 0.200678109001819f, 0.276933184666555f, 0.325461783498046f,

0.361617966771595f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f },

{ 0.496449411258169f, 0.496449411258169f, 0.651628394060843f, 0.543093993977907f,

0.623017147429065f, 0.401790810848940f, 0.301127209760909f, 0.207909818841500f,

0.205766656125932f, 0.213734682309585f, 0.333426412763235f, 0.394911699348976f,

0.470164679266681f, 0.432389361460998f, 0.459330199671557f, 0.370467402210716f,

0.371107729492926f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f } } },

{ 150.0f,

{ { 0.298280594494943f, 0.298280594494943f, 0.299312600355623f, 0.236047823318058f,

0.236251731414293f, 0.140483396911322f, 0.089975651500707f, 0.069502431758880f,

0.055239533208935f, 0.087674843633931f, 0.089202083840017f, 0.160186163484006f,

0.181394869714599f, 0.205057189184673f, 0.282976258457614f, 0.332563819955743f,

0.369508982288584f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f },

{ 0.329135706827428f, 0.329135706827428f, 0.435386541329689f, 0.310634723076088f,

0.303214523861240f, 0.229614864811236f, 0.160601648946313f, 0.130880516437444f,

0.098599565242491f, 0.111847170029336f, 0.136536887991945f, 0.183442516711773f,

0.191370503715938f, 0.216334109374689f, 0.298538261891796f, 0.350852842987021f,

0.389829768501105f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f } } },

{ 180.0f,

{ { 0.452246276656300f, 0.452246276656300f, 0.457746465951409f, 0.356348553306142f,

0.313058139566081f, 0.194368104770253f, 0.139636836105594f, 0.104652594586409f,

0.090443008436831f, 0.113501081567231f, 0.141335090067637f, 0.206835461656757f,

0.213550208604744f, 0.202069143199226f, 0.278852793796739f, 0.327717776805172f,

0.364124582768118f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f },

{ 0.452246276656300f, 0.452246276656300f, 0.457746465951409f, 0.356348553306142f,

0.313058139566081f, 0.194368104770253f, 0.139636836105594f, 0.104652594586409f,

0.090443008436831f, 0.113501081567231f, 0.141335090067637f, 0.206835461656757f,

0.213550208604744f, 0.202069143199226f, 0.278852793796739f, 0.327717776805172f,

0.364124582768118f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f, 0.354201176996633f,

0.354201176996633f, 0.354201176996633f, 0.354201176996633f } } },

{ 210.0f,

{ { 0.329135706827428f, 0.329135706827428f, 0.435386541329689f, 0.310634723076088f,

0.303214523861240f, 0.229614864811236f, 0.160601648946313f, 0.130880516437444f,

0.098599565242491f, 0.111847170029336f, 0.136536887991945f, 0.183442516711773f,

0.191370503715938f, 0.216334109374689f, 0.298538261891796f, 0.350852842987021f,

0.389829768501105f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f, 0.379205825055067f,

0.379205825055067f, 0.379205825055067f, 0.379205825055067f },

{ 0.298280594494943f, 0.298280594494943f, 0.299312600355623f, 0.236047823318058f,

0.236251731414293f, 0.140483396911322f, 0.089975651500707f, 0.069502431758880f,

0.055239533208935f, 0.087674843633931f, 0.089202083840017f, 0.160186163484006f,

0.181394869714599f, 0.205057189184673f, 0.282976258457614f, 0.332563819955743f,

0.369508982288584f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f, 0.359438836681872f,

0.359438836681872f, 0.359438836681872f, 0.359438836681872f } } },

{ 240.0f,

{ { 0.496449411258169f, 0.496449411258169f, 0.651628394060843f, 0.543093993977907f,

0.623017147429065f, 0.401790810848940f, 0.301127209760909f, 0.207909818841500f,

0.205766656125932f, 0.213734682309585f, 0.333426412763235f, 0.394911699348976f,

0.470164679266681f, 0.432389361460998f, 0.459330199671557f, 0.370467402210716f,

0.371107729492926f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f, 0.360994013586417f,

0.360994013586417f, 0.360994013586417f, 0.360994013586417f },

{ 0.260690376924823f, 0.260690376924823f, 0.417710122705241f, 0.307432658063637f,

0.364124582768118f, 0.233816416405714f, 0.151051485272583f, 0.122250318705779f,

0.085359133929137f, 0.103842467675821f, 0.127533711865269f, 0.197412659832141f,

0.215216220349500f, 0.200678109001819f, 0.276933184666555f, 0.325461783498046f,

0.361617966771595f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f, 0.351762873244939f,

0.351762873244939f, 0.351762873244939f, 0.351762873244939f } } },

{ 270.0f,

{ { 0.533334895487621f, 0.533334895487621f, 0.684502439096536f, 0.568034874449755f,

0.593608348191371f, 0.412334809789612f, 0.208269176583772f, 0.132586711776056f,

0.087750580873113f, 0.098259601842788f, 0.089975651500707f, 0.154347642332453f,

0.175388050184176f, 0.198266801282380f, 0.273605611323968f, 0.321551100290820f,

0.357272838151929f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f, 0.347536161443206f,

0.347536161443206f, 0.347536161443206f, 0.347536161443206f },

{ 0.282244183025396f, 0.282244183025396f, 0.313058139566081f, 0.271019163189084f,

0.293426950395597f, 0.194200346295108f, 0.119949930314938f, 0.095746961354662f,

0.087599171763312f, 0.119330098744878f, 0.158398111714294f, 0.233614610221893f,

0.252057717963911f, 0.249890646139567f, 0.283465363348967f, 0.301908263350306f,

0.326869952558687f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f, 0.348437589004521f,

0.348437589004521f, 0.348437589004521f, 0.348437589004521f } } },

{ 300.0f,

{ { 0.473015094678584f, 0.473015094678584f, 0.763835783577690f, 0.632229854806186f,

0.747523757863696f, 0.566565336409729f, 0.381175518739995f, 0.303738609194611f,

0.232206835516786f, 0.222971830894677f, 0.240574726093395f, 0.280301227735976f,

0.239331575640539f, 0.213181738713594f, 0.294188031312244f, 0.345740296418239f,

0.384149259178759f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f, 0.373680125382116f,

0.373680125382116f, 0.373680125382116f, 0.373680125382116f },

{ 0.766478540162591f, 0.766478540162591f, 1.000000000000000f, 0.874731970892050f,

0.977799925864087f, 0.789314352732207f, 0.486267226589422f, 0.345441889099858f,

0.274552245465151f, 0.293426950395597f, 0.316865531946885f, 0.386144630346289f,

0.351155924516291f, 0.324339617349349f, 0.341881027081702f, 0.355426659637028f,

0.324619795632621f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f, 0.335737614242955f,

0.335737614242955f, 0.335737614242955f, 0.335737614242955f } } },

{ 330.0f,

{ { 0.518800038928961f, 0.518800038928961f, 0.643798558586410f, 0.520145684953132f,

0.660123205654783f, 0.594634359165687f, 0.418432105762394f, 0.300607630262823f,

0.236251731414293f, 0.226464430759306f, 0.266609119134929f, 0.371749163536557f,

0.376920709547802f, 0.326869952558687f, 0.352370871042487f, 0.345740296418239f,

0.322663618591148f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f, 0.328284214316547f,

0.328284214316547f, 0.328284214316547f, 0.328284214316547f },

{ 0.697629761974222f, 0.697629761974222f, 0.835603018231248f, 0.671042458632901f,

0.950331226974181f, 0.728408491522093f, 0.630049976930408f, 0.531955128236972f,

0.415192978147146f, 0.390503562294295f, 0.480009684908112f, 0.612350391724774f,

0.588504787243298f, 0.501620180363360f, 0.559757601495111f, 0.571478636671867f,

0.504661297563529f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f, 0.511681835540308f,

0.511681835540308f, 0.511681835540308f, 0.511681835540308f } } },

{ 360.0f,

{ { 0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f },

{ 0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f, 0.331989998205157f,

0.331989998205157f, 0.331989998205157f, 0.331989998205157f } } },

const ElevationProfiles defaultElevationProfiles0 = {

{ -30.0f,

{ 0.917156297265849f, 0.917156297265849f, 0.671341280053693f, 0.550102162631601f,

0.644827205614767f, 0.586061715903898f, 0.528376327914776f, 0.565513988819906f,

0.436961679761170f, 0.377522945080379f, 0.362612985324198f, 0.399892585916292f,

0.601787762910887f, 0.761102857216751f, 0.655304804833470f, 0.534494637435879f,

0.497099529755547f, 0.456502701871178f, 0.456502701871178f, 0.456502701871178f,

0.456502701871178f, 0.456502701871178f, 0.456502701871178f, 0.456502701871178f,

0.456502701871178f, 0.456502701871178f, 0.456502701871178f, 0.456502701871178f,

0.456502701871178f, 0.456502701871178f, 0.456502701871178f } },

{ -15.0f,

{ 0.930986433276774f, 0.930986433276774f, 0.839350497745053f, 0.794224632377244f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

0.825930211407267f, 0.694134055646429f, 0.584041022071381f, 0.558397354794353f,

0.779728399132162f, 0.948294763330940f, 0.933132578652771f, 0.815536404779250f,

0.745492626509754f, 0.711939931250462f, 0.711939931250462f, 0.711939931250462f,

0.711939931250462f, 0.711939931250462f, 0.711939931250462f, 0.711939931250462f,

0.711939931250462f, 0.711939931250462f, 0.711939931250462f, 0.711939931250462f,

0.711939931250462f, 0.711939931250462f, 0.711939931250462f } },

{ 0.0f,

{ 1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 1.000000000000000f,

1.000000000000000f, 1.000000000000000f, 1.000000000000000f } },

{ 15.0f,

{ 0.923513721003843f, 0.923513721003843f, 0.761979613868237f, 0.681464681311002f,

0.738657909027877f, 0.722675534451525f, 0.733573023748635f, 0.740360691366310f,

0.708668864395584f, 0.634517131621849f, 0.555192195272969f, 0.477466990647510f,

0.596270539537662f, 0.538819559249321f, 0.398972858490850f, 0.328808739537999f,

0.299532086887260f, 0.301955783526424f, 0.301955783526424f, 0.301955783526424f,

0.301955783526424f, 0.301955783526424f, 0.301955783526424f, 0.301955783526424f,

0.301955783526424f, 0.301955783526424f, 0.301955783526424f, 0.301955783526424f,

0.301955783526424f, 0.301955783526424f, 0.301955783526424f } },

{ 30.0f,

{ 1.000000000000000f, 1.000000000000000f, 1.000000000000000f, 0.731885852612517f,

0.808990370719907f, 0.879920330795670f, 0.873863001845037f, 0.674440050607649f,

0.594899155352234f, 0.464989720749418f, 0.409207280833171f, 0.403592746456686f,

0.461257406003648f, 0.462320714139681f, 0.412993629184471f, 0.360946929993716f,

0.397597232046371f, 0.384984595501267f, 0.384984595501267f, 0.384984595501267f,

0.384984595501267f, 0.384984595501267f, 0.384984595501267f, 0.384984595501267f,

0.384984595501267f, 0.384984595501267f, 0.384984595501267f, 0.384984595501267f,

0.384984595501267f, 0.384984595501267f, 0.384984595501267f } },

};

Reverberation Extent Mesh Definitions

Four meshes are defined for the reverberation extents, with 0, 1, 2, or 3 segmentations. 0 segments describes a flat 2D surface, while the segmented versions describe a surface that is curved. The meshes are defined by a list of vertex coordinates, and a list of faces, defined by the vertices that construct them.

For all versions:

hw = portal width / 2

hh = portal height / 2

hd = portal depth / 2

offset = reverb extent offset

ho = offset / 2

irt2 = 1 / sqrt(2)

cos30o = offset * cos (30 * pi / 180)

O segments

VertexCoordinates = {{ -hw - 1.f, hh + 1.f, -hd - offset }, { hw + 1.f, hh + 1.f, -hd - offset },

{ -hw - 1.f, -hh - 1.f, -hd - offset }, { hw + 1.f, -hh - 1.f, -hd - offset }}

FaceVertices = {{ 0, 2, 1 }, { 1, 2, 3 }}

1 Segment

VertexCoordinates = { { -hw, hh, -hd - offset }, { hw, hh, -hd - offset }, { -hw, -hh, -hd - offset },

{ hw, -hh, -hd - offset }, { -hw, hh + offset, -hd }, { hw, hh + offset, -hd },

{ -hw - offset, hh, -hd }, { hw + offset, hh, -hd }, { -hw - offset, -hh, -hd },

{ hw + offset, -hh, -hd }, { -hw, -hh - offset, -hd }, { hw, -hh - offset, -hd } };

FaceVertices = { { 0, 2, 1 }, { 1, 2, 3 }, { 0, 5, 4 }, { 1, 5, 0 },

{ 0, 6, 8 }, { 0, 8, 2 }, { 3, 2, 10 }, { 3, 10, 11 }, { 7, 1, 3 }, { 7, 3, 9 },

{ 4, 6, 0 }, { 5, 1, 7 }, { 2, 8, 10 }, { 9, 3, 11 } };

2 Segments

VertexCoordinates = { { -hw, hh, -hd - offset }, { hw, hh, -hd - offset },

{ -hw, -hh, -hd - offset }, { hw, -hh, -hd - offset }, { -hw, hh + offset, -hd },

{ hw, hh + offset, -hd }, { -hw - offset, hh, -hd }, { hw + offset, hh, -hd },

{ -hw - offset, -hh, -hd }, { hw + offset, -hh, -hd }, { -hw, -hh - offset, -hd },

{ hw, -hh - offset, -hd }, { -hw, hh + (offset * irt2), -hd - (offset * irt2) },

{ hw, hh + (offset * irt2), -hd - (offset * irt2) },

{ -hw, -hh - (offset * irt2), -hd - (offset * irt2) },

{ hw, -hh - (offset * irt2), -hd - (offset * irt2) },

{ -hw - (offset * irt2), hh, -hd - (offset * irt2) },

{ -hw - (offset * irt2), -hh, -hd - (offset * irt2) },

{ hw + (offset * irt2), hh, -hd - (offset * irt2) },

{ hw + (offset * irt2), -hh, -hd - (offset * irt2) },

{ -hw - (offset * irt2), hh + (offset * irt2), -hd },

{ -hw - ho, hh + (irt2 * offset), -hd - ho },

{ hw + (offset * irt2), hh + (offset * irt2), -hd },

{ hw + ho, hh + (irt2 * offset), -hd - ho },

{ -hw - (offset * irt2), -hh - (offset * irt2), -hd },

{ -hw - ho, -hh - (irt2 * offset), -hd - ho },

{ hw + (offset * irt2), -hh - (offset * irt2), -hd },

{ hw + ho, -hh - (irt2 * offset), -hd - ho } };

FaceVertices = { { 0, 2, 1 }, { 1, 2, 3 }, { 5, 4, 12 }, { 5, 12, 13 },

{ 13, 12, 0 }, { 13, 0, 1 }, { 3, 2, 14 }, { 3, 14, 15 }, { 15, 14, 10 }, { 15, 10, 11 },

{ 16, 6, 8 }, { 16, 8, 17 }, { 0, 16, 17 }, { 0, 17, 2 }, { 18, 1, 3 }, { 18, 3, 19 },

{ 7, 18, 19 }, { 7, 19, 9 }, { 21, 16, 0 }, { 12, 21, 0 }, { 22, 18, 7 }, { 18, 22, 23 },

{ 4, 21, 12 }, { 4, 20, 21 }, { 23, 13, 1 }, { 23, 1, 18 }, { 20, 6, 16 }, { 22, 5, 23 },

{ 5, 13, 23 }, { 16, 21, 20 }, { 2, 17, 25 }, { 2, 25, 14 }, { 24, 25, 17 }, { 24, 17, 8 },

{ 14, 25, 10 }, { 25, 24, 10 }, { 3, 15, 27 }, { 3, 27, 19 }, { 27, 15, 11 },

{ 27, 11, 26 }, { 19, 27, 26 }, { 9, 19, 26 } };

3 Segments

VertexCoordinates = { { -hw, hh, -hd - offset }, { hw, hh, -hd - offset },

{ -hw, -hh, -hd - offset }, { hw, -hh, -hd - offset }, { -hw, hh + offset, -hd },

{ hw, hh + offset, -hd }, { -hw - offset, hh, -hd }, { hw + offset, hh, -hd },

{ -hw - offset, -hh, -hd }, { hw + offset, -hh, -hd }, { -hw, -hh - offset, -hd },

{ hw, -hh - offset, -hd }, { -hw, hh + cos30o, -hd - ho }, { hw, hh + cos30o, -hd - ho },

{ -hw, hh + ho, -hd - cos30o }, { hw, hh + ho, -hd - cos30o },

{ -hw, -hh - ho, -hd - cos30o }, { hw, -hh - ho, -hd - cos30o },

{ -hw, -hh - cos30o, -hd - ho }, { hw, -hh - cos30o, -hd - ho },

{ -hw - cos30o, hh, -hd - ho }, { -hw - ho, hh, -hd - cos30o },

{ -hw - cos30o, -hh, -hd - ho }, { -hw - ho, -hh, -hd - cos30o },

{ hw + ho, hh, -hd - cos30o }, { hw + cos30o, hh, -hd - ho },

{ hw + ho, -hh, -hd - cos30o }, { hw + cos30o, -hh, -hd - ho },

{ -hw - cos30o, hh + ho, -hd }, { -hw - ho, hh + cos30o, -hd },

{ -hw - (offset * 0.75), hh + ho, -hd - (offset * 0.4330) },

{ -hw - (offset * 0.4330), hh + cos30o, -hd - (offset * 0.25) },

{ -hw - (offset * 0.25), hh + cos30o, -hd - (offset * 0.4330) },

{ -hw - (offset * 0.4330), hh + ho, -hd - (offset * 0.75) }, { hw + cos30o, hh + ho, -hd },

{ hw + ho, hh + cos30o, -hd }, { hw + (offset * 0.75), hh + ho, -hd - (offset * 0.4330) },

{ hw + (offset * 0.4330), hh + cos30o, -hd - (offset * 0.25) },

{ hw + (offset * 0.25), hh + cos30o, -hd - (offset * 0.4330) },

{ hw + (offset * 0.4330), hh + ho, -hd - (offset * 0.75) }, { -hw - cos30o, -hh - ho, -hd },

{ -hw - ho, -hh - cos30o, -hd },

{ -hw - (offset * 0.75), -hh - ho, -hd - (offset * 0.4330) },

{ -hw - (offset * 0.4330), -hh - cos30o, -hd - (offset * 0.25) },

{ -hw - (offset * 0.25), -hh - cos30o, -hd - (offset * 0.4330) },

{ -hw - (offset * 0.4330), -hh - ho, -hd - (offset * 0.75) },

{ hw + cos30o, -hh - ho, -hd }, { hw + ho, -hh - cos30o, -hd },

{ hw + (offset * 0.75), -hh - ho, -hd - (offset * 0.4330) },

{ hw + (offset * 0.4330), -hh - cos30o, -hd - (offset * 0.25) },

{ hw + (offset * 0.25), -hh - cos30o, -hd - (offset * 0.4330) },

{ hw + (offset * 0.4330), -hh - ho, -hd - (offset * 0.75) } };

FaceVertices = { { 0, 2, 1 }, { 1, 2, 3 }, { 12, 5, 4 }, { 12, 13, 5 },

{ 14, 13, 12 }, { 14, 15, 13 }, { 0, 15, 14 }, { 0, 1, 15 }, { 3, 2, 16 }, { 3, 16, 17 },

{ 17, 16, 18 }, { 17, 18, 19 }, { 19, 18, 10 }, { 19, 10, 11 }, { 20, 6, 8 }, { 20, 8, 22 },

{ 21, 20, 22 }, { 21, 22, 23 }, { 0, 21, 23 }, { 0, 23, 2 }, { 24, 1, 3 }, { 24, 3, 26 },

{ 25, 24, 26 }, { 25, 26, 27 }, { 7, 25, 27 }, { 7, 27, 9 }, { 4, 32, 12 }, { 4, 31, 32 },

{ 4, 29, 31 }, { 33, 14, 32 }, { 14, 12, 32 }, { 31, 29, 30 }, { 30, 29, 28 },

{ 21, 0, 33 }, { 14, 33, 0 }, { 32, 31, 33 }, { 33, 31, 30 }, { 5, 13, 38 }, { 5, 38, 37 },

{ 5, 37, 35 }, { 38, 13, 15 }, { 38, 15, 39 }, { 37, 38, 39 }, { 37, 39, 36 },

{ 35, 37, 36 }, { 35, 36, 34 }, { 39, 15, 1 }, { 39, 1, 24 }, { 36, 39, 24 },

{ 36, 24, 25 }, { 34, 36, 25 }, { 34, 25, 7 }, { 10, 43, 41 }, { 10, 44, 43 },

{ 10, 18, 44 }, { 41, 42, 40 }, { 43, 42, 41 }, { 43, 45, 42 }, { 44, 45, 43 },

{ 44, 16, 45 }, { 18, 16, 44 }, { 40, 22, 8 }, { 42, 22, 40 }, { 42, 23, 22 },

{ 45, 23, 42 }, { 45, 2, 23 }, { 16, 2, 45 }, { 11, 50, 19 }, { 11, 49, 50 },

{ 11, 47, 49 }, { 17, 51, 3 }, { 3, 51, 26 }, { 49, 47, 48 }, { 48, 47, 46 },

{ 48, 46, 27 }, { 46, 9, 27 }, { 51, 48, 26 }, { 26, 48, 27 }, { 51, 50, 49 },

{ 49, 48, 51 }, { 19, 50, 17 }, { 17, 50, 51 }, { 30, 28, 20 }, { 20, 28, 6 },

{ 21, 30, 20 }, { 33, 30, 21 } };

Headphone Equalization Preset Responses

Default preset responses are provided such that the content creator or the user may specify their use in place of measured data. For each frequency response 10 gain values are specified, corresponding to the following centre frequencies given in hertz:

fc = [70, 180, 320, 600, 1000, 3000, 6000, 12000, 14000, 16000];

The presets indexes are defined in Table 16, and the gain values are given in the following  map:

std::map<int, std::vector<float>> gains = { { 0, { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0 } },

{ 1, { 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, -6.5, -6.5, -6.5, -8.5 } },

{ 2, { 0.0, 0.0, 2.5, 4.5, 4.5, 4.5, 2.5, 0.0, 0.0, 0.0 } },

{ 3, { 7.5, 5.5, 1.5, -0.5, -0.5, -5.5, -6.5, -6.5, -0.5, -0.5 } },

{ 4, { 3.5, 8.5, 4.0, -3.5, -2.5, 1.0, 3.5, 7.5, 10.0, 11.5 } },

{ 5, { 8.0, 8.0, 4.5, 4.5, 0.0, -4.5, -4.5, -4.5, 0.0, 0.0 } },

{ 6, { 5.5, 5.5, 0.0, 0.0, 0.0, 0.0, 0.0, 0.0, 5.5, 5.5 } },

{ 7, { -2.0, 3.5, 5.5, 6.0, 4.0, -1.5, -2.5, -2.5, -2.0, -2.0 } },

{ 8, { 0.0, 0.0, -1.0, -5.5, 0.0, 5.0, 5.0, 0.0, 0.0, 0.0 } },

{ 9, { 6.0, 3.5, -5.0, -7.0, -3.5, 3.0, 7.0, 8.5, 8.5, 8.5 } },

{ 10, { 3.5, 1.0, -1.5, -2.5, -1.5, 3.0, 6.5, 7.5, 8.5, 9.5 } },

{ 11, { -2.5, -4.5, -4.0, -1.0, 3.0, 4.5, 7.0, 7.5, 8.5, 7.5 } },

{ 12, { 7.5, 7.5, 7.5, 4.5, 1.0, -4.0, -7.5, -9.0, -9.5, -9.5 } },

{ 13, { 3.0, 3.0, 1.5, -1.0, -4.0, -5.0, -3.5, -1.0, 2.0, 7.0 } },

{ 14, { -8.5, -8.5, -8.5, -4.0, 2.0, 8.5, 12.5, 12.5, 12.5, 13.5 } },

{ 15, { 5.5, 4.5, 0.0, -6.5, -4.5, 1.0, 6.5, 8.5, 9.5, 9.5 } },

{ 16, { -4.5, 0.0, 3.0, 4.0, 4.5, 4.5, 3.0, 2.0, 2.0, 1.5 } },

{ 17, { 6.0, 4.5, 0.0, -5.0, -4.5, 0.0, 6.0, 7.5, 7.5, 7.0 } } };

Listener Voice Default Directivity Pattern

listenerVoiceDefaultDirectivityData = { 0x7d, 0x81, 0x12, 0xe6, 0xa6,

0x5e, 0x30, 0x98, 0x3b, 0x7d, 0x57, 0x4a, 0x39, 0x87, 0x88, 0x44, 0xc0, 0x5f, 0x5d, 0x54, 0x09,

0xb9, 0xb0, 0xc7, 0xd8, 0xed, 0xbc, 0x31, 0xcd, 0x2b, 0x58, 0xb3, 0xe7, 0x07, 0x66, 0xb0, 0x69,

0x36, 0x9b, 0x7b, 0xe7, 0x56, 0x27, 0x72, 0xf0, 0xf4, 0x5d, 0xa9, 0xfa, 0x2d, 0xce, 0x0d, 0x4d,

0x16, 0xac, 0x07, 0xcd, 0x33, 0xe0, 0x3d, 0x99, 0xcb, 0x7d, 0x4a, 0x3c, 0x46, 0x15, 0x8e, 0x82,

0xc2, 0x53, 0x37, 0x8b, 0x3a, 0xba, 0x09, 0x1c, 0x7f, 0x09, 0x4a, 0xc8, 0x2f, 0xc8, 0xce, 0x2e,

0x1c, 0x47, 0x96, 0xd2, 0xb6, 0x53, 0xc9, 0x68, 0x35, 0xf2, 0xea, 0x9d, 0xbc, 0xfa, 0x73, 0x67,

0x39, 0x20, 0x03, 0x0c, 0x2c, 0xd0, 0x70, 0x3f, 0x18, 0x07, 0xef, 0x73, 0xe4, 0xdb, 0x3f, 0xc6,

0x96, 0x67, 0x85, 0xfa, 0xb1, 0x07, 0xce, 0xb4, 0xc4, 0x0b, 0x92, 0x3a, 0xe5, 0x3a, 0x0d, 0x04,

0x6c, 0x9a, 0xf3, 0xa3, 0x6a, 0x6d, 0x78, 0xf8, 0x48, 0x8b, 0x01, 0xca, 0x1d, 0xe2, 0x46, 0x80,

0x1d, 0xaf, 0xa3, 0x54, 0x42, 0x54, 0x01, 0x35, 0x99, 0x66, 0x84, 0xee, 0xb5, 0x3e, 0x07, 0x95,

0x62, 0x9b, 0xa3, 0x56, 0xd5, 0x63, 0x4a, 0x97, 0x32, 0x2a, 0x94, 0xa8, 0xd8, 0xed, 0x3c, 0xb8,

0xd9, 0x3f, 0x47, 0x26, 0x3b, 0x5d, 0xa3, 0x04, 0x65, 0x75, 0x06, 0x53, 0xba, 0x84, 0x03, 0x85,

0x7e, 0x91, 0x01, 0xc0, 0x61, 0x6e, 0x6d, 0xe6, 0xf7, 0xa3, 0x6a, 0x5c, 0x99, 0x53, 0x3b, 0xfb,

0x25, 0xd4, 0000, 0x1d, 0xa9, 0x73, 0x58, 0xb8, 0xa4, 0x42, 0xd8, 0x7c, 0x1a, 0xa2, 0x5d, 0x27,

0x46, 0xf3, 0x43, 0xdb, 0x23, 0x0e, 0xc0, 0x60, 0x9d, 0x34, 0x84, 0x36, 0x49, 0xe0, 0x84, 0xc2,

0x27, 0x1e, 0x14, 0x0d, 0xf4, 0x5b, 0x5a, 0x3f, 0x1f, 0x93, 0x94, 0x39, 0x84, 0xa4, 0x62, 0x9d,

0x02, 0x20, 0xa0, 0x53, 0xde, 0x9e, 0x06, 0xc4, 0x85, 0xa8, 0xcb, 0x87, 0x94, 0xda, 0xaa, 0x37,

0xcb, 0xd6, 0x92, 0xc3, 0x56, 0xbc, 0x74, 0x36, 0x20, 0x0a, 0xa4, 0xc7, 0x2a, 0xfc, 0xd6, 0x5a,

0xb3, 0x64, 0x13, 0x52, 0xc5, 0x30, 0x8d, 0x12, 0xe3, 0x73, 0x73, 0xba, 0xbe, 0xc8, 0xbf, 0x26,

0x34, 0x4a, 0xf4, 0x3d, 0xbc, 0x66, 0x8a, 0xac, 0xe0, 0xe4, 0xa3, 0x64, 0x0a, 0xf4, 0x3e, 0x88,

0xc6, 0x37, 0x1d, 0x0d, 0x7c, 0x07, 0x6e, 0xca, 0x77, 0xf0, 0x55, 0x56, 0x82, 0x57, 0x20, 0x9e,

0xd0, 0x1f, 0x34, 0xd3, 0x2d, 0xc0, 0xba, 0xde, 0x2d, 0x3a, 0xb7, 0x54, 0xd9, 0xc5, 0xfa, 0xaf,

0xd6, 0x3f, 0x84, 0xe1, 0x49, 0x51, 0x51, 0x04, 0xbb, 0xb8, 0xd4, 0xf6, 0x6d, 0x9d, 0x40, 0x47,

0xae, 0xc8, 0x08, 0xf5, 0x4a, 0x9b, 0xdf, 0xed, 0xdf, 0xc7, 0x01, 0x47, 0x56, 0x92, 0xf2, 0x16,

0xe5, 0x68, 0x28, 0x40, 0x86, 0x03, 0xb6, 0x8d, 0xe1, 0x0e, 0x16, 0x1b, 0xdc, 0x3a, 0xa0, 0x3f,

0xe2, 0xe5, 0x90, 0xfc, 0000, 0x9b, 0x65, 0x44, 0x50, 0x5b, 0x55, 0x08, 0x90, 0xea, 0x6e, 0x62,

0xce, 0xa4, 0x51, 0x09, 0x68, 0xd2, 0xf3, 0xd2, 0x03, 0x7c, 0xfb, 0xc5, 0x58, 0x75, 0xa0, 0xe2,

0x0e, 0x92, 0x9e, 0x68, 0x4d, 0x1b, 0x88, 0x42, 0x7b, 0x84, 0x30, 0x03, 0xc4, 0x4f, 0x8a, 0xa8,

0x88, 0xdc, 0x44, 0x80, 0xf4, 0x40, 0xf9, 0x25, 0x80, 0x36, 0x41, 0x0a, 0x09, 0x69, 0xae, 0x7f,

0x27, 0xb3, 0x81, 0x87, 0x14, 0x23, 0xc8, 0x98, 0xa6, 0xfd, 0xa2, 0x3b, 0x8e, 0x26, 0x1b, 0x5b,

0x6a, 0xfa, 0x90, 0x7c, 0xd9, 0x2d, 0xa5, 0x4b, 0xc7, 0x46, 0xc1, 0x87, 0xe4, 0xf7, 0x1d, 0x6d,

0x0c, 0x4b, 0x6f, 0x1f, 0xdf, 0xd7, 0xf5, 0x07, 0x81, 0xb0, 0x98, 0xd1, 0x9f, 0x7b, 0x4e, 0x35,

0xa6, 0xea, 0xf3, 0x39, 0x4b, 0x6a, 0x98, 0x24, 0x6d, 0x2a, 0x38, 0xb1, 0xd2, 0xfe, 0xc9, 0x5e,

0x44, 0x70, 0x15, 0x63, 0xf2, 0x33, 0x73, 0x48, 0x1e, 0x71, 0xaf, 0x24, 0x3b, 0x7c, 0x28, 0x3d,

0xb2, 0xae, 0x4f, 0x9a, 0x10, 0x7d, 0x2e, 0xab, 0x2b, 0xd2, 0x84, 0x6e, 0x34, 0xc6, 0xec, 0x81,

0xb9, 0000, 0xa1, 0xa4, 0x19, 0x3c, 0xe5, 0x4c, 0xa2, 0xae, 0x67, 0xb0, 0x26, 0xb5, 0xd4, 0xac,

0x4d, 0x32, 0x97, 0xd5, 0xe6, 0x80, 0x35, 0xad, 0x70, 0x1a, 0x69, 0xea, 0xb8, 0x71, 0x3e, 0x32,

0x81, 0xea, 0x4a, 0xf4, 0x48, 0xad, 0x19, 0xca, 0x84, 0xcd, 0x75, 0x86, 0xba, 0x0d, 0xa6, 0x44,

0x44, 0x85, 0x75, 0x6e, 0x22, 0x9f, 0x3a, 0xac, 0xc9, 0xcd, 0x6a, 0xd5, 0x57, 0x58, 0xaa, 0x6a,

0x91, 0x54, 0xab, 0x88, 0x18, 0x29, 0x02, 0x80, 0x59, 0x12, 0x84, 0x9a, 0x16, 0x6e, 0xb0, 0x29,

0xba, 0x58, 0x67, 0x91, 0xe1, 0x3e, 0xb2, 0xaf, 0xc4, 0x1d, 0x9f, 0x57, 0x33, 0x7d, 0x91, 0xe2,

0xc8, 0x6a, 0x09, 0x27, 0xcb, 0xa4, 0x88, 0x28, 0xa7, 0x8f, 0xad, 0xf7, 0x3b, 0x0e, 0x01, 0x7a,

0x6e, 0x8b, 0xb1, 0xcb, 0x6b, 0x0c, 0x9b, 0x1c, 0x1b, 0x34, 0x3b, 0x9a, 0xfa, 0x8c, 0xe9, 0xb0,

0xe0, 0x06, 0x7b, 0xf9, 0x25, 0x86, 0x35, 0x76, 0x9f, 0x70, 0x1b, 0xbe, 0xc1, 0xc8, 0x5d, 0xbe,

0x40 };

Portal LoS Data Decoder Tables

CDV_URLC =

{ 0,3782,7564,11346,15128,17436,19744,22052,24360,26054,27748,29442,31136,32241,33346,34451,35556,36710,37864,39018,40172,40736,41300,41864,42428,42722,43016,43310,43604,43898,44192,44486,44780,45246,45712,46178,46644,47012,47380,47748,48116,48312,48508,48704,48900,48998,49096,49194,49292,49365,49438,49511,49584,49780,49976,50172,50368,50417,50466,50515,50564,50662,50760,50858,50956,51103,51250,51397,51544,51642,51740,51838,51936,51960,51984,52008,52032,52130,52228,52326,52424,52448,52472,52496,52520,52667,52814,52961,53108,53181,53254,53327,53400,53473,53546,53619,53692,53790,53888,53986,54084,54206,54328,54450,54572,54670,54768,54866,54964,55013,55062,55111,55160,55258,55356,55454,55552,55601,55650,55699,55748,55772,55796,55820,55844,55942,56040,56138,56236,56260,56284,56308,56332,56356,56380,56404,56428,56452,56476,56500,56524,56597,56670,56743,56816,56840,56864,56888,56912,56961,57010,57059,57108,57132,57156,57180,57204,57253,57302,57351,57400,57473,57546,57619,57692,57716,57740,57764,57788,57861,57934,58007,58080,58104,58128,58152,58176,58225,58274,58323,58372,58421,58470,58519,58568,58617,58666,58715,58764,58862,58960,59058,59156,59278,59400,59522,59644,59668,59692,59716,59740,59764,59788,59812,59836,59860,59884,59908,59932,59956,59980,60004,60028,60052,60076,60100,60124,60148,60172,60196,60220,60269,60318,60367,60416,60440,60464,60488,60512,60536,60560,60584,60608,60632,60656,60680,60704,60728,60752,60776,60800,60824,60848,60872,60896,60920,60944,60968,60992,61041,61090,61139,61188,61319,61450,61581,63496,65536}

CDV_SPRC=

{ 0,15,30,45,60,75,90,105,120,135,150,165,180,195,210,225,240,255,270,340,410,480,550,643,736,829,899,969,1039,1109,1202,1295,1388,1458,1528,1598,1668,1761,1854,1947,2017,2087,2157,2227,2297,2367,2437,2507,2600,2693,2786,2856,2926,2996,3066,3159,3252,3345,3415,3485,3555,3625,3718,3811,3904,4044,4184,4324,4464,4557,4650,4743,4813,4883,4953,5023,5233,5443,5653,5863,5956,6049,6142,6212,6282,6352,6422,6515,6608,6701,6771,6841,6911,6981,7168,7355,7542,7682,7822,7962,8102,8242,8382,8522,8662,8755,8848,8941,9292,9643,9994,10345,10720,11095,11470,12102,12734,13366,13998,14841,15684,16527,17370,18213,19056,19899,20367,20835,21303,21654,22005,22356,22707,22917,23127,23337,23547,23640,23733,23826,23896,23966,24036,24106,24199,24292,24385,24525,24665,24805,24945,25038,25131,25224,25294,25364,25434,25504,25644,25784,25924,26064,26157,26250,26343,26413,26483,26553,26623,26716,26809,26902,26972,27042,27112,27182,27275,27368,27461,27531,27601,27671,27741,27834,27927,28020,28090,28160,28230,28300,28370,28440,28510,28580,28673,28766,28859,28929,28999,29069,29139,29232,29325,29418,29488,29558,29628,29698,29791,29884,29977,30047,30117,30187,30257,30327,30397,30467,30537,30630,30723,30816,30886,30956,31026,31096,31189,31282,31375,31445,31515,31585,31655,31748,31841,31934,32004,32074,32144,32214,32307,32400,32493,48947,65536}

Reverberator output directions

Reverberator output directions are provided here for the spherical designs with a number of azimuth-elevation pairs suitable for each supported output channel count, namely 15, 31, and 63 channels. Note that these designs are t-designs rotated such that a pair of directions lies on the horizontal plane, oriented symmetrically around the forward axis.

dirs_t_fwpair_15_deg[15][2] = {

{ 82.658851354417308f, 45.776701836081472f },

{ -103.211797148889076f, -20.996660207713223f },

{ 62.447049548527559f, 0.000000000000002f },

{ 172.658851354417294f, -0.000000000000009f },

{ -5.115301174902946f, -55.075721661607460f },

{ -139.498775509469226f, 12.794209241377812f },

{ 14.604676412414083f, -12.794209241377812f },

{ -119.778797922152194f, 55.075721661607460f },

{ 97.341148645582692f, -30.446384317065231f },

{ -62.447049548527559f, 0.000000000000002f },

{ 137.764752257362176f, 30.446384317065224f },

{ -21.682301948166053f, 20.996660207713226f },

{ 152.447049548527559f, -45.776701836081472f },

{ -11.552377423280149f, 69.731081550693503f },

{ -113.341721673774956f, -69.731081550693489f } };

dirs_t_fwpair_31_deg[31][2] = {

{ -21.227914684628225f, -0.000000000000004f },

{ 144.166146415942535f, -39.058501810257034f },

{ -0.000000042133876f, 32.319159519795448f },

{ 160.696218451645649f, -8.646319455760947f },

{ 46.092132941458154f, 35.592827198843139f },

{ -46.092133060988431f, 35.592827131173195f },

{ -165.285328486345094f, -6.013873952385691f },

{ 35.341081142546805f, -41.005150567886659f },

{ 136.549914552511922f, 30.894355659464448f },

{ 21.227914684628200f, 0.000000000000007f },

{ -64.555276959969802f, 7.303982613202332f },

{ -139.462967778490594f, 32.677072071279753f },

{ -174.435316727748955f, -61.771453724156927f },

{ -0.000000217873171f, 70.344154027604731f },

{ -90.004136402389676f, -60.304065739604162f },

{ 156.161941041166216f, 65.716199543234879f },

{ -121.572244417656435f, -2.398536477252398f },

{ 53.378132586776388f, -7.684054402112428f },

{ -144.464385588860097f, -33.806320503878077f },

{ -5.000092606567491f, -72.215563373533598f },

{ -177.969313143670092f, 29.976473465812681f },

{ -10.970804291206807f, -31.495601569421947f },

{ 86.057785094463583f, 12.162232881350828f },

{ -94.105804021871819f, -21.464815261831014f },

{ 87.526615506354688f, 49.121590079422631f },

{ -54.075652088035646f, -29.406603527914680f },

{ 123.665584936594925f, 0.429138722951919f },

{ -100.673331810731455f, 66.536017458738542f },

{ 92.484356152494456f, -24.124555266313592f },

{ -96.656013946955284f, 28.545378419433657f },

{ 92.030804464154855f, -56.947682877906125f } };

dirs_t_fwpair_63_deg[63][2] = {

{ 113.697644297577199f, 75.586367185940546f },

{ 166.310646192394728f, -77.684915450136458f },

{ -17.389103694898271f, 16.199603089248804f },

{ 158.822754037832453f, -12.645966321239733f },

{ 60.358089557801456f, -61.633632065821807f },

{ -55.400505826335845f, 72.236746983300861f },

{ -138.621699406575971f, 18.136412679207336f },

{ 58.826283607279628f, -1.414381990856097f },

{ -140.436766467579957f, 39.290274137497249f },

{ -49.081811554803778f, -37.081634002506441f },

{ -162.016399094256514f, 11.539415068276861f },

{ 14.462909900643668f, 0.000000000000016f },

{ -96.689514963231886f, 58.590970970599642f },

{ 121.385234198446426f, -35.154804586166392f },

{ -39.884596536300933f, -18.186308271001522f },

{ 29.176384353080813f, 18.573181877103718f },

{ -143.929354548781674f, 60.389893911892045f },

{ 90.113195006351290f, 26.173048875247730f },

{ -86.026684054801592f, -4.064365329271832f },

{ 107.359577791370299f, 40.970430011138482f },

{ -158.378140809581964f, -9.362257369704528f },

{ 59.201385791935110f, 15.036228755803268f },

{ -154.251908659831230f, -30.221887395193693f },

{ 7.331796515755554f, 67.240009845692171f },

{ -68.418666082070501f, 6.579590634863780f },

{ -90.418339489950952f, -24.173441813919823f },

{ 92.850643546644875f, 5.790286144586245f },

{ 29.499830538080069f, -15.138225248667363f },

{ 52.052426573120464f, 41.363214877831737f },

{ 109.539653840480028f, -6.242935338992117f },

{ -48.769156080158716f, 0.413993055858015f },

{ 177.107021781366853f, 41.812771277039012f },

{ -63.314596947338423f, -70.552282445636862f },

{ -58.937151467299643f, 40.255558615281394f },

{ 170.546592751151849f, -0.995005064330493f },

{ -75.184580491607932f, -40.262910557670033f },

{ -121.137777758556467f, -41.683881870833638f },

{ -3.116505077969357f, -24.715273106713525f },

{ 171.423156108720008f, 26.037622270870678f },

{ 175.861985785744793f, -56.730887113546807f },

{ 178.356183795915598f, -35.585540530141550f },

{ 46.779122330655440f, -29.137637580452569f },

{ 151.274088900546161f, 64.484910903387217f },

{ 88.207759148636541f, -34.876563400358158f },

{ -89.801268305889096f, 30.053225210144532f },

{ 77.656926399373148f, -21.278351389684364f },

{ -17.821507128036220f, -66.949086169393908f },

{ 132.771099505294984f, 35.665865658227233f },

{ -42.401567716459439f, 30.718049375001772f },

{ 129.638946492271032f, -1.716547557921829f },

{ 12.475304819954122f, 33.661183083303406f },

{ 29.965758471370243f, -48.848969323800397f },

{ -116.840219983601131f, -1.288753780714446f },

{ -14.462909900637902f, 0.000000000000003f },

{ 139.940681600195944f, 16.136446227182592f },

{ -122.541856340902655f, -58.114702705768522f },

{ 65.287791671586234f, 55.343181967946883f },

{ -10.454909013743984f, 45.637942442249106f },

{ 106.651521415697275f, -62.578150231561231f },

{ -126.085866385379063f, -14.978780360888869f },

{ 141.473502636798969f, -34.649424345748770f },

{ -107.014661653642463f, 24.017281223767185f },

{ -9.370605842054440f, -40.314579424013793f } };

(informative)

Encoder overview

The encoder parses the Immersive Audio Encoder Input Format (EIF) scene description file into readable data structures and generates different categories of side information as well as the scene description. Finally, it codes and serializes the data to create an MHAS bitstream file.

In this bitstream, the encoder represents different categories of side information as separate payloads elements bundled in an MHAS payload packet. These payloads are used to enrich renderer stages with extra data for higher quality rendering. The side information is often represented as a pair – the identifier (ID) of the entity as authored in the scene description coupled with the side information itself. For example, the encoder encodes reverberation parameters and couples them with the AcousticEnvironment ID found in the scene description. An AcousticEnvironment describes the spatial zones for which reverberation parameters are defined.

A block diagram of the scene description encoder is provided in the Figure 104. The first major step in the encoding process is to parse the scene description file and provide readable data structures as Scene Entities to the different modules of the encoder. A short description of some of the encoding modules and the associated parameters for their respective bitstream payload elements is provided in the following.

Figure 104 — Econer overview

Encoder modules

Scene Configuration parameters

The maximum propagation delay that can be rendered on RIs should be chosen larger than the delay introduced by the maximum possible distance between listener location and a RI location in the scene. This maximum delay is configured by delayBufferSize (cf. Table 10). delayBufferSize should be chosen as small as possible for a given scene to avoid unnecessarily large memory consumption for delay buffers on the renderer host system.

The configuration parameter gainCullingThreshold sets a threshold at which a RI with a large attenuation (e.g., due to large distance attenuation) is deactivated. With parameter values ranging from 0 to 7, deactivation thresholds between -100 dB and -30 dB can be signaled in increments of 10 dB.

The boolean flag enableCullingReflectionRI enables (true) or disables (false) the Reflection RI culling. If Voxel-based early reflection is used, this flag is set to false.

Audio plus metadata creation

Audio Plus metadata contains additional audio source characterization parameters like distanceGainModel, hasDiffuseness, and diffuseness, for audio sources with extent, based on analysis of the audio signals themselves. This is for example used in the Heterogeneous rendering stage (6.6.15) to improve the rendering of Heterogeneous sources, which are sources that have an extent and more than one audio channel. This includes both object sources with more than one source channel and HOA sources with an extent and both an interior and exterior representation.

The parameter distanceGainModel makes it possible to control the distance gain behaviour so that it either follows the distance gain fall-off of a point-source or that it follows a distance gain model based on the size of the extent of the audio element. The parameter can be used as a design parameter set manually by the content creator or can be automatically set by the encoder by an analysis of the audio signals provided for the audio element.

The parameter diffuseness describes the amount of correlation between the different channels of the audio signals of the Audio Element. This can be used as a design parameter to control the diffuseness of an audio element or can be automatically set by the encoder by measuring the mean correlation between the channels over time.

Reverberation parametrization

Reverberation in each Acoustic Environment (AE) is described based on its parameters, such as RT60, predelay and diffuse-to-source energy ratio (DSR) and are associated to the AE with an identifier.

Default acoustic environment (Default AE)

The bitstream may contain metadata for a Default AE. There are either several pre-defined presets (see A.3) to choose from and modify, or the possibility to define an entire customized Default AE.

| Preset name | ID | Description |
| --- | --- | --- |
| Default | 0 | Medium sized space with moderate reflective surfaces |
| LargeNeutral | 1 | Large sized space with moderate absorbent reflective surfaces |
| LargeDark | 2 | Large sized space with highly absorbent reflective surfaces |
| LargeBright | 3 | Large sized space with less absorbent reflective surfaces |
| SmallNeutral | 4 | Small sized space with moderate absorbent reflective surfaces |
| SmallDark | 5 | Small sized space with highly absorbent reflective surfaces |
| SmallBright | 6 | Small sized space with less absorbent reflective surfaces |

This metadata is used by the renderer to apply generic acoustic properties when the listener is not inside an EIF specified AE. The Default AE uses the same parameters as utilized for the EIF specified AE to control the Reverberation modules.

Low complexity early reflection parametrization

As an alternative to the full-blown geometry-based calculation of early reflections (ERs) in the renderer, the bitstream can also contain parameters for so-called low complexity early reflections that can be created in the renderer without complex geometry-based calculations.

Specifically for outdoor scenes, a new pattern with four roughly cross-positioned ERs is designed. The different distances are defined here by a predelay time and a compression factor, which are derived from geometry analysis of the scene.

Usage of ER patterns for outdoor environments known is highly individual and dependent on the physical setup of the scene. The geometrical analysis described hereafter captures perceptually important characteristics of the outdoor scene, which are relevant to the perception of ERs:

From a central listening point, concentric rings are positioned. The area of the rings, defined by radius and height, represents the maximum possible reflection energy at this distance, see Figure 105. There is a spacing d between the rings (e.g. 3m). Rays with an angular spacing α (e.g. 6°) are sent out from the analysis point. The first surfaces that hit are counted to the existing reflection surface at this distance and summed up over the ring.

Figure 105: Geometrical outdoor scene analysis. a) Top view of rings around an analysis point. b) Side view around an analysis point with rings of increasing height.

Figure 106: Mesh of analysis points in top a) and side b) view. The dot-dashed line indicates the user reachable area of a scene.

There are a number of analysis points (e.g. 9) positioned in the inner part of a user reachable area, see Figure 106. It is a 3D mesh, because some of the points are inside the geometrical mesh of the scene and have to be deselected. The data over all mesh points are averaged and the distribution is analyzed. It represents the reflective outdoor energy over space and distance, see 107.

Figure 107: Distribution of reflection surface area over distance, averaged over several analysis points

The amplitudes a1 and a2 - together with their distances - are the input values to calculate the outdoor ER pattern values, see (641)..(646).

| preDelay1=25*p1 ms | (641) |
| --- | --- |
| preDelay2=3*p2 ms | (642) |
| minValue1=a1 a1≥0.0010.001 a1<0.001 | (643) |
| minValue2=a2 a2≥0.0010.001 a2<0.001 | (644) |
| ampFac1=minimum(100*minValue1;3) | (645) |
| ampFac2=minimum(100*minValue2;3) | (646) |

Theses values can be the starting values for a tuning of the parameters by ear.

Portal creation in implicit portal mode

This section describes how the encoder generates portals based on the acoustic environments (AEs) in a scene. An important concept to keep in mind here is that a portal is a representation of an AE in the implicit portal mode, whereas in the explicit portal mode the portal represents the oppening between AEs. When the listener is not in a particular AE, but it is still acoustically relevant, it will be represented as a portal.

There are three steps covering the main processes of generating portals:

Creation of the geometry of the portal

One portal geometry with unique portalExtentId is generated from each AE in the scene. Its geometry can be obtained by shrinking the geometry of the corresponding portalParentEnvironment slightly, this is done to avoid overlap between the geometry of the portal and potential occluding boundaries (e.g. walls), as illustrated in Figure 108.

Figure 108 — A bird eye 2D view of how a portal is shrunken from an AE

Identification of the connection state between two portals

There are three possible states of connection between two AEs: not connected, connected with an opening, connected with an occluder (or in other words: closed), which are demonstrated in Figure 109 to Figure 111 respectively. This step utilizes raytracing and voxelization techniques to identify potential empty spaces or geometries between each pair of AEs or between one AE and the ‘outside’ environment. Furthermore, it provides the information of isConnectedWithOpening, and if this variable is true, also the location of the opening, i.e. openingPosX, openingPosY and openingPosZ.

Figure 109 — A bird eye 2D view of two AEs that are not connected

Figure 110 — A bird eye 2D view of two connected AEs with an opening

Figure 111 — A bird eye 2D view of two AEs connected with an occluder

Creation of the portal struct containing all its metadata to be encoded

All the metadata obtained through the above two steps are organized into a structure for bitstream serialization. This step takes care of a) creating one portal struct with unique portalId for each portal geometry, b) assigning them under relevant acousticEnvironmentId (portals are relevant for a specific acoustic environment if they are not created from the given AE), and c) calculating portalFactor for each opened connection based on the area of the opening, volume of the source AE and the absorption coefficient of the source AE estimated from RT60.

Portal bitstream parameters are sent for each acoustic environment to describe the rendering of late reverberation between and outside of acoustic environments.

Line-of-sight data creation in explicit portal mode

In the explicit portal mode, a voxel grid of line-of-sight (LoS) data is used to accelerate calculations of LoS from source to portals. The LoS affects the level of reverberation a source audio signal causes through a portal in a connected reverberant environment such that reverberation level is lower when there is no LoS. This section describes the encoder side processing to create the LoS voxel grids.

Grids of LoS voxels is created the encoder to represent the visibility of each sound source at positions of the grid towards portals to be used for controlling energy propagation of sources through the portals to reverberation in the renderer. The voxel grids support both static and dynamic sources as long as the source is within the LoS-area-grid.

The encoder obtains AudioScene acoustic environment (AE) geometries. A bounding geometry is created around each AE to be the voxel grid area where LoS is calculated. Each AE bounding geometry is limited in height to a certain value (2m) and the x and z extents are expanded by a certain amount to add some tolerance to the voxel grid (1m, 1m).

For portals which are connected only a single AE, an outside LoS voxel grid is created. This is to cover audio source positions outside the AE. The outside LoS grid is set to cover an extended bounding geometry of the AE such that the AE bounding geometry is expanded by 10m over the x and z axes so that it covers the AE and some area around it.

The bounding geometry is divided into a voxel grid and the center of each voxel is used as the LoS checking point. The resolution of the LoS grid depends on the number of voxels with more voxels leading to a biffer resolution.

Los-check is performed from each of the voxel centers. To improve computational performance, only meshes that have a certain percentage of vertices inside the bounding geometry are taken into account when performing the LoS check. The LoS-check is performed as follows:

Using a ray tracing routine, n rays are shot from each voxel center towards the vertices of the open face of each portal. Based on ray hits, a los_ratio = rays passed / rays occluded is calculated. This approach is used for all the distributed points and all portals producing a set of approximated LoS values for each point and portal combination. The amount of rays is dependent on number of vertices in the open portal faces. The open portal faces are the ones which a ray cast along the the portal normal passes. Line of sight can be converted into a boolean value by setting a threshold of the percentage of rays needed to hit portal vertices within a tolerance conclude that a LoS exists.

LoS values are stored as a binary array for each portal in the AE. Knowing the extents of the voxel grid and the voxel size, a voxel representation can be formed. Voxel index can be derived from position of the sound source, which can used to check LoS value for that voxel.

Before writing the LoS-data into the payloadPortal of the bitstream, the truth array of the portal gets converted into multidimensional array based on populated geometry extents, producing several 2D planes of binary values. The 2D planes are treated as bitmasks, each of which is compressed by using lossless run-length-encoding with entropy encoding and the coded data is written into bitstream. Decoding is done by the renderer producing the original binary-array for each portal.

Source/geometry staticity analysis

The staticity data describes whether or not an audio source or geometry is static - meaning that it remains unchanged - throughout the duration of the scene. The Source/Geometry Staticity Analysis evaluates all of the updates in the scene description and tags each audio source (hoaSource, channelSource, objectSource) and geometry (mesh, box, sphere, cylinder) with staticity data.

Diffraction edges and paths analysis

The bitstream may contain data which facilitate accelerated rendering for the Renderer Diffraction stage (6.6.10). Specifically, this can include diffraction edges on all the geometry and paths between sources and the listener, based on an analysis of the scene description.

Early reflection surfaces and sequences analysis

Similarly, the bitstream can contain acceleration data for Early Reflection stage (6.6.6) in the Renderer. Specifically, this can include lists of the relevant reflection surfaces on all of the geometry and the possible sequences of reflection between them, based on an analysis of the scene description.

Module data collection

The output of each encoding module (portal creation, diffraction data, early reflection sequences, etc.) is stored in the Module Data Collection processing block.

Module data serialization

The Module Data Serialization block serializes the collected module data to be written in the bitstream that is finally transmitted in the MHAS payload packet (5.3.2.3).

Listener space description format (LSDF)

The Listener Space Description Format (LSDF) provides listening space environment information directly to the Renderer. The specification of the LSDF is contained in Annex C of ISO/IEC 23090-34, Immersive audio reference software.

Encoder input format (EIF)

The encoder input format (EIF) contains information describing an MPEG-I 6DoF Audio scene. The specification of the EIF is contained in Annex B of ISO/IEC 23090-34, Immersive audio reference software.

Accessibility User Interface

This clause specifies a user interface for providing accessibility-related user data to MPEG-I.

| <AcousticSceneAdjustments> |  |  |  |  |
| --- | --- | --- | --- | --- |
| This field describes the user input parameter to adjust relevant scene-related parameter for rendering. |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| erAttenuationDb | Float | O | 0 | Amount of early reflection level reduction in decibel. |
| reverbAttenuationDb | Float | O | 0 | Amount of reverb level reduction in decibel. |
| distanceExponent | Float | O | 1.0 | Input parameter to adjust a as further described in clause 6.6.17.4.3.1 |
| <DirectionalFocus> |  |  |  |  |
| This field describes the user input parameter for the directional focus as further described and applied in clause 6.6.18 and 6.6.25.5 |  |  |  |  |
| Attribute | Type | Flags | Default | Description |
| hasDirectionalFocus | Bool | O | False | Flag to indicate if the directional focus is enabled. |
| directionalFocusAperture | Float | O | 30.0 | Opening angle of the directional focus’ main lobe in degree. |
| directionalFocusTransitionWidth | Float | O | 30.0 | Width of the transition region between main lobe and stopband in degree. |
| directionalFocusStopbandAttenuationDb | Float | O | 20.0 | Directional gain reduction in the stopband in decibel. |
| hasNondefaultFocusDirection | Bool | O | False | Flag to indicate if the directional focus has a non-default direction. |
| directionalFocusDirectionYaw | Float | O | 0 | Yaw angle of the primary direction of the directional focus with respect to the frontal head orientation in degree. |
| directionalFocusDirectionPitch | Float | O | 0 | Pitch angle of the primary direction of the directional focus with respect to the frontal head orientation in degree. |

Guidance on Own Voice Usage – Influence of System Delay

The integration of locally captured audio allows the user voice and other real time signals to be rendered in the scene the same way as any other audio source. With this feature the immersion of the user into the scene is increased. It is a pre-requirement for SocialVR.

The parametrization of direct sound, early reflection, and reverb level especially for the own voice is highly dependent on the level of the locally captured signal. As seen from Figure 112, the level of the person speaking, the sensitivity of the used microphone and the input gain of the microphone amplifier has a significant influence on the input signal. For a consistent experience a headset with a fixed distance between mouth and microphone is highly recommended. Furthermore, it was observed that the parametrization of direct sound, early reflection and reverb level was subjective to the individual listener and could be scene dependent. Only by introducing a user interface for these parameters a plausible and satisfying fusion of the own voice into a scene for all listeners is possible.

Figure 112 — Overview diagram of complete renderer chain for locally captured audio with listener’s voice and all involved data files.

Humans hear their own voice over bone conduction and over the air-conducted path. For the AR/VR simulation of the own voice, this is captured by a microphone close to the mouth, recorded by a sound device converted into the digital domain, processed by the MPEG-I renderer, converted back into the analogue domain and played back over the reproduction device, normally the headphone. This processing chain has some significant delay. The natural over the air path is reduced by the headset depending on its type. It can be completely open, then no extra electrical direct sound path is necessary and desirable over the headset. It can be a closed headphone, damping e.g. 20 dB of the air path. In this case the direct part should be reproduced over the headset. The mouth to ear distance is about 10 cm, resulting in a natural acoustic latency of ~0,3 ms. The current MPEG-I implementation, with a block length 256 samples and fs = 48 kHz, has a minimum latency of 22 ms (2x AD/DA conversion, 2x processing block length, plus processing delay). The bone conduction of the own voice stays the same as in the natural case without extra delay. The late contribution of the electrical reproduced direct sound part is perceived individually different: some subjects like a natural timbre balance of the direct sound part over headset (resulting in a high direct sound gain), some subjects like a lower direct sound gain, not to be annoyed by the latency. Therefore, the possibility to adjust direct sound gain is very important.

There is a long history of investigations on the effects of so-called sidetone in telephony. The influence of level and delay of the sidetone on intelligibility was already quantified in [21]. Intelligibility was only degraded statistically significant for delays ≥ 100 ms. The audibility of delayed signal in the presence of direct signal, as a function of delay and relative level is presented in Figure 4. A curve about the preferred (delayed) direct sound level is not known, but it will lie above the here presented curve and will also be delay dependent.

Figure 113 — Audibility of delayed signal in the presence of direct signal, as a function of delay and relative level, [17].

The total round-trip delay in a practical hard- and software implementation can vary widely. This is beside the personal level preference a further reason to keep the gain values variable. In general, it is recommended to keep the round-trip delay below 100 ms.

The following values are recommended for listenerVoice scene object attributes as default for reproduction with closed headphones:

- directGainDb:		-17 dB (0.14 linear)

- erGainDb:				-13 dB (0.22 linear)

- itemReverbGain:		-6 dB (0.50 linear)

For open headphones, it is recommended to set the default value for directGaindB lower. Since the reproduction is hardware dependent, default values should be adapted to the respective hardware setup in case it is known. The optimal set of gain values may also be scene dependent. It can be helpful to give guidance to the user, when the used scenes or kind of scenes (indoor or outdoor) are known.

It can be useful to provide an overall gain parameter via a UI to a user, that only changes the overall gain offset but keeps the relative differences of the individual attributes.

It is additionally assumed that locally captured audio will be pre-processed by some kind of acoustic front end that delivers the signals at a certain maximum loudness level. Such a front end is not part of the MPEG-I specification. The processing delay of such a pre-processor has to be considered as well.

Bibliography

[1]	Välimäki, V. and j. Rämö, Neurally Controlled Graphic Equalizer. IEEE/ACM Transcations on Audio, Speech, and Language Processing, 2019. 27(12).

[2]	Rämö, J., J. Liski, and V. Välimäki, Third-Octave and Bark Graphic-Equalizer Design with Symmetric Band Filters. Appl. Sci., 2020. 10(4) DOI: 10.3390/app10041222.

[3]	Rocchesso, D., Maximally Diffusive Yet Efficient Feedback Delay Networks for Artificial Reverberation. IEEE Signal Processing Letters, 1997. 4(9): p. 225-255.

[4]	Lamb, D., L.F.O. Chamon, and V.H. Nascimento, Efficient Filtering Structure for Spline Interpolation and Decimation. Electronics Letters, 2016. 52: p. 39..41.

[5]	Allen, J.B. and D.A. Berkley, Image method for eficiently simulating small-room acoustics. J. Acoust. Soc. Am., 1979. 65(4): p. 943-950 DOI: 10.1121/1.382599.

[6]	Oppenheim, A.V. and R.W. Schafer, Discrete-time signal processing. 2nd ed. 1999: Printice-Hall.

[7]	ETSI TS 126 260, 5G; Objective test methodologies for the evaluation of immersive audio systems (3GPP TS 26.260 version 15.0.0 Release 15), 2018. https://www.etsi.org/deliver/etsi_TS/126200_126299/126260/15.00.00_60/ts_126260v150000p.pdf.

[8]	ISO/IEC 20180319, Information technology – High efficiency coding and media delivery in heterogeneous environments – Part 3: MPEG-H_3DA_2nd_ed, 2018, International Standards Organization / International Electrotechnical Commission, Geneva, Switzerland.

[9]	Zotter, F. and M. Frank, Ambisonics: A Practical 3D Audio Theory for Recording, Studio Production, Sound Reinforcement, and Virtual Reality. Springer Topics in Signal Processing, ed. J. Benesty and W. Kellermann. Vol. 19. 2019: Springer Open.

[10]	Vilkamo, J., T. Bäckström, and A. Kuntz, Optimized Covariance Domain Framework for Time-frequency Processing of Spatial Audio. J. Audio Eng. Soc., 2013. 61(6): p. 403..411.

[11]	Ivanic, J. and K. Ruedenberg, Rotation Matrices for Real Spherical Harmonics. Direct Determination by Recursion Page: Additions and Corrections. Journal of Physical Chemistry A, 1998. 102(45): p. 9099..9100.

[12]	Rafely, B., Fundamentals of spherical array processing. Vol. 8. 2015, Berlin: Springer.

[13]	AES69, AES Standard for File Exchange - Spatial Acoustic Data File Format (Sofa), 2020, Audio Engineering Society, New York, USA. https://secure.aes.org/store/GetFile.cfm?ProdID=99&cptID=10.

[14]	Bentley, J.L., Multidimensional Binary Search Trees Used for Associative Searching. Commun. ACM, 1975. 18(9): p. 509..517.

[15]	Wefers, F., Partitioned Convolution Algorithms for Real-time Auralization. Institute of Technical Acoustics, RWTH Aachen University, Doctoral Dissertation. 2015: Logos Verlag Berlin

[16]	Atkinson, C.J., Some Effects on Intelligibility as the Sidetone Level and the Amount of Sidetone Delay Are Changed. Proceedings of the Iowa Academy of Science, 1954. 61. Available from: https://scholarworks.uni.edu/cgi/viewcontent.cgi?article=3074&context=pias.

[17]	ITU-R BT.2044, Tolerable Round-Trip Time Delay for Sound-Programme and Television Broadcast Programme Inserts – Context and Rationale, 2004. Available from: https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-BT.2044-2004-PDF-E.pdf.

[18]	Digital Signal Processing Committee of the IEEE Acoustics, Speech, and Signal Processing Society, eds. Programs for Digital Signal Processing. New York: IEEE Press, 1979, Algorithm 5.2.

[19]	Abel, J. S. and Huang, P., “A Simple, Robust Measure of Reverberation Echo Density,” in Proceedings of the 121st Convention of the Audio Engineering Society, Oct. 2006.

[20]	Ahrens, J., Database of Spherical Harmonic Representations of Sound Source Directivities [Data set]. 2020. DOI: 10.5281/zenodo.3707708.

The following bits are skipped because they are not used. This will be revised in the future.

The following bits are skipped because they are not used. This will be revised in the future.