ISO/IEC JTC 1/SC 29/WG 7 N942

ISO/IEC JTC 1/SC 29/WG 7
MPEG 3D Graphics and Haptics Coding 
Convenorship: AFNOR (France)

Document type:	Output Document

Title:	G-PCC 2nd Edition codec description

Status:	Approved

Date of document:	2024-10-15

Source:	ISO/IEC JTC 1/SC 29/WG 7

Expected action:		   None

Action due date:		   None

No. of pages:	190 (with cover page)

Email of Convenor:	marius.preda @ it-sudparis.eu

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg7

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 7 MPEG 3D Graphics AND HAPTICS Coding

ISO/IEC JTC 1/SC 29/WG 7 N942

July 2024, Sapporo

| Title | G-PCC 2nd Edition codec description |
| --- | --- |
| Source | WG 7, MPEG 3D Graphics and Haptics Coding |
| Status | Approved |
| Serial Number | 24176 |

# Abstract

This codec description document summarizes technologies for the second edition of G-PCC. They advance G-PCC in the following areas: predictive and octree geometry coding, inter prediction for geometry coding, attribute coding, inter prediction for attribute coding, and TriSoup.

# Contents

1	Abstract	1

2	Contents	1

3	Predictive Geometry Coding	7

3.1	Adaptive Azimuthal Angle Quantization [1][2]	7

3.2	Improved Coding of Azimuthal Angle Residual [3][7]	10

3.3	Improved Coding of Number of Azimuthal Angle Steps [4][7]	11

3.3.1	Method For Scaling Azimuthal Angle Step	12

3.3.2	Enabling Scaling of Azimuthal Angle Step	12

3.3.3	Computing Scaled Azimuthal Angle Step	12

3.4	Improved Radius Residual Sign Coding and Points Ordering [5][7]	13

3.4.1	Improved Radius Residual Sign Coding	15

3.4.2	Improved Points Ordering	16

3.5	Improved Predictor List [6][7]	16

3.5.1	List of Predictors	17

3.5.2	Derivation of the predictors	18

3.5.3	Management of the prediction buffer	18

3.6	Improved Coding of Magnitude of Radius Residual [8][9]	19

3.6.1	Radius Residual Coding in G-PCC Ed.1	19

3.6.2	Problem Description	20

3.6.3	Proposed Coding Method of the Magnitude of Radius Residual	20

3.6.4	Prior Information in Prediction Process	22

3.6.5	Context Determination Method	23

3.6.6	Complexity Reduction	24

3.7	Disabling of Cartesian Residual Coding [10][11]	24

3.8	Prediction List Index Coding [13][12]	25

3.9	Removing Parsing Dependency in Predictive Geometry Coding [14]	25

3.10	Radius Residual Coding by the Threshold of the Number of Azimuthal Angle Steps [15][16]	27

4	Octree Geometry Coding	28

4.1	Residual Coding of Angular Mode in IDCM [17][20]	28

3.1.1 Motivation	28

3.1.2 IDCM Angular Mode in G-PCC’s Ed.1	28

3.1.3 Improved Method	28

4.2	Improved Azimuthal Mode in IDCM [18][20]	30

4.2.1	Azimuthal Mode in G-PPC Ed.1 IDCM	30

4.2.2	Problem Description	31

4.2.3	Proposed Angle Interpolation	33

4.3	Context Enhancement of Azimuthal IDCM [19][20]	35

4.3.1	Context Selection in G-PCC’s Ed.1 Azimuthal Coding Mode	35

4.3.2	Problem Description	35

4.3.3	Improved Method	37

4.4	Planar Flag Signalling [21][22]	38

4.4.1	Description of the Method	38

4.4.2	Signalling One Flag Instead of Three	38

4.4.3	Signalling One Flag Instead of Two	39

4.5	Disabling Planar Mode for IDCM Nodes [23][24]	39

4.6	Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [25][26][27][28]	40

4.7	Removal of Z-Coding Path in New Octree Coding [31]	42

4.8	Modified Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [32][33]	43

4.9	Common Dynamic OBUF Class for Octree and TriSoup [34]	43

4.10	Eligibility of Planar Mode [29][30]	44

4.10.1	Planar Mode Eligibility in G-PCC Ed.1	44

4.10.2	Proposed Eligibility Based on Density	44

4.11	Low-Memory Footprint Dynamic OBUF [35]	45

4.12	Initialization of Dynamic OBUF [36]	45

4.13	Probability Bounds in Dynamic OBUF [37][38]	45

4.14	Improved Planar Mode [39][40]	47

4.15	Improvements of Octree-Coding for Spin-LIDAR Sequences [41][42]	50

4.15.1	Introduction	50

4.15.2	Z-Coordinates Compensation Process	50

5	Inter Prediction for Octree Geometry Coding	51

5.1	Inter Prediction for Octree Geometry Coding Based on Global Motion Using Two-Threshold Classification of Road/Object Points [44][45][50][52]	51

5.2	Inter Planar Mode Coding [51]	54

5.2.1	Modified Planar Mode Coding with Inter Prediction	55

5.2.2	Context Selection for Planar Flag and Plane Position Coding Using Inter Prediction	55

5.2.3	Planar Copy Mode	56

5.2.4	Use Inter Prediction in Occupancy When the Node Has a Planar Direction	57

5.3	IDCM Eligibility for Inter [60]	57

5.4	Inter Modification for Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [60][61]	57

5.5	Unified Partitioning Method for Motion Compensation [62]	59

5.6	Bi-Directional Inter Prediction [64][65]	59

5.7	Inter IDCM [71][72]	61

5.7.1	Inter Eligibility	61

5.7.2	Inter-prediction IDCM Mode Coding	62

5.7.3	Inter-prediction IDCM numPoints Coding	62

1.1.3	Inter-prediction IDCM resLaser Coding	62

5.8	Improved Inter Predictive Information for Occupancy Tree [77] (GeS-TM)	63

5.9	On Inter Context Design of Octree Coding [78] (GeS-TM)	64

6	Inter Prediction for Predictive Geometry Coding	65

6.1	Inter Prediction for Predictive Geometry Coding [47][48][49]	65

6.2	Additional Predictor Candidate [55]	67

6.3	Improved Inter Prediction Flag Coding [56]	68

6.4	Integration with Improved Predictive Geometry Coding and Global Motion [59]	68

6.4.1	Integration with Improved Predictive Geometry Coding	68

6.4.2	Integration with Global Motion	68

6.5	Integer Implementation of Cartesian to Spherical Coordinate Conversion for Global Motion Compensation [63]	68

6.6	Zero Motion and Global Motion Inter Predictors [66][67][68]	69

6.7	Additional Inter Predictors [69]	70

6.7.1	Modified Estimation of qphi	70

6.7.2	Selection of Additional Inter Predictors from Either GMC Reference Frame or from a Second Non-MC Reference Frame	71

6.7.3	Geometry Data Unit Header Syntax Modifications	72

6.8	Inter Prediction Buffers in Predictive Geometry Coding [74]	73

6.9	Downsampling For Inter Prediction [79]	74

6.10	On Inter Prediction Buffers in Predictive Geometry Coding [81]	74

6.10.1	Spherical coordinate conversion of reference frame	75

6.10.2	Min position adjustment for predicting transform and lifting transform	75

6.10.3	Spherical coordinate conversion of reference frame on-the-fly	75

7	Inter Prediction for Attribute Coding	75

7.1	Unidirectional Inter Prediction for Attribute Coding [82][83][84][85]	75

7.2	Bidirectional Inter Prediction for Attribute Coding [64][65]	76

7.3	Inter Prediction for DC and AC RAHT Coefficients [86][87]	77

7.4	Improvements to Inter Prediction for RAHT Attribute Coding [91][92]	78

7.5	Improved Coefficient Inter Prediction for RAHT Attribute Coding [93][94]	78

7.6	Attribute Inter Prediction with Slice Partitioning [95]	80

7.7	Attribute Inter Prediction for predLifting Transform Coding [96]	81

7.7.1	Introduction	81

7.7.2	Inter-Attribute Nearest Neighbor Search	82

7.8	Bidirectional Inter Prediction Attribute Coding for Predictive Geometry [97]	83

7.9	Removing Same Reference Frame Octree Decomposition for RAHT Inter Prediction; Average Prediction [98][99]	83

7.9.1	Direct Inter Prediction	83

7.9.2	Average Prediction	83

7.10	Temporal Filtering for RAHT [100][101]	84

7.11	Intra or Inter Prediction Decision Per RAHT Layer [102]	85

7.12	RAHT Inter-Eligibility [104][105]	85

7.13	Resampling for RAHT Inter-Prediction [107]	86

7.14	Global Motion-compensated Inter-prediction for Attribute Coding [109]	86

7.15	Bi-directional Inter-prediction for RAHT Attribute Coding [110]	88

7.16	Improved Inter-attribute Coding Method for RAHT [111]	90

7.17	Reference Node Resampling for Lossless RAHT [112]	92

7.18	Global Motion Compensated Inter Prediction for Attribute Coding [114] (clipping aspect)	93

7.19	Motion Compensated Attributes Projection Without Using Dual Motion for Octree [116] (GeS-TM)	93

7.20	Method to Enhance Inter RAHT Average Prediction [117] (GeS-TM)	94

8	Inter Prediction: Other Techniques	95

8.1	Dependent Entropy Frame Coding [53][54]	95

8.2	Signalling of Global Motion Data [58]	96

8.3	Inter Entropy Continuation HLS [70]	96

8.3.1	Background	96

8.3.2	Proposed Change	97

8.4	Dependent Entropy Frame Coding for Attribute [88][89][90]	98

8.5	Temporal ID for Temporal Scalability [73]	99

8.6	Frame Unused for Reference Indication [75]	101

8.7	TLV Type Constraint When Multiple Slices Present [76]	101

8.8	On Inter Prediction Syntax [80]	101

8.9	Attribute Syntax Improvements [113]	102

8.10	Inter-frame Context Dependency for RAHT Pred Mode [115]	102

9	Attribute Coding	102

9.1	Adaptive Quantization for LoD-Based Attribute Predicting Transform Coding [118][119]	102

9.2	Neighbor Search Method for Attribute LoD Prediction [120][121][128]	104

9.3	Low-Latency Attribute Coding Inside Slice [122][123]	107

9.4	Extended Prediction for RAHT [124][125]	108

9.5	Disabling Transform Domain Prediction of RAHT [126][127]	109

9.6	RAHT Reconstruction Buffer Rounding Operation Removal [129][130]	111

9.7	3D Quantization Matrix Signalling for RAHT Coefficient Coding [131][132]	112

9.8	Lossless RAHT Extension [134][135]	114

9.9	Cross-Attribute Prediction [136][137]	115

9.10	Extend Context for Attribute Coefficient Entropy Coding [139]	116

9.11	Inter-Chroma Channel Attribute Prediction for RAHT [140]	116

9.12	Restriction of Scale Values Used in Spherical Coordinate Conversion [75]	117

9.13	Sample Domain Prediction for RAHT [142][143] (TMC13, GeS-TM)	117

9.14	Improved Intra-attribute Coding Method for RAHT [144]	118

9.15	Inter-chroma Channel Prediction for RAHT [145] (GeS-TM)	119

9.16	Context Reduction for RAHT Entropy Coding [146] (GeS-TM)	120

9.17	RAHT Prediction Mode Signalling [147] (GeS-TM)	120

9.18	Canonical Geometry Order-based Periodic Subsampling Method for LoD Generation [149]	121

9.19	RAHT Code Optimizations [150] (GeS-TM)	122

9.20	Cross-component Residual Prediction [151] (GeS-TM)	123

10	TriSoup	123

10.1	Alternative Method for Determining Projection Plane [152][153]	123

10.2	Refinement of Trisoup Projection Plane Determination [154]	124

10.3	Improved TriSoup [155][156][157][158][159][160][161][162]	125

10.4	Adaptive Halo Improvement for Voxelization [163]	125

10.5	Non-Cubic TriSoup [166][167]	126

10.6	Voxelization of TriSoup Triangles [168]	127

10.7	Inter TriSoup Based on Motion Compensated Point Cloud [169]	127

10.8	On TriSoup Voxelization [170]	128

10.9	Centroid Vertex Positioning [171]	128

10.10	Raster Scan Order for Octree Coding (GeS-TM) [172][173]	128

10.11	TriSoup Node Grid Alignment (GeS-TM) [174][175]	130

10.12	Motion-Compensated Inter-Frame Attribute Coding Scheme for Dynamic Solid Point Clouds (GeS-TM) [177][178]	130

10.12.1	Overview	130

10.12.2	Motion Compensation	131

10.12.3	Prediction Mode Decision	131

10.12.4	Prediction Mode Signalling	132

10.12.5	Prediction Mode Inference	132

10.13	Harmonized Inter Prediction for RAHT (GeS-TM) [179]	133

10.14	Face Vertex for TriSoup [180][181]	133

10.14.1	Problem Description	133

10.14.2	Proposed Method	135

10.14.3	Generation of Face Vertex	135

10.14.4	Transmission Condition of Face Values	136

10.14.5	Reconstruction	137

10.14.6	Syntax	138

10.15	Inter Skip Mode (GeS-TM) [182]	139

10.16	RAHT Mode Selection (GeS-TM) [183]	139

10.17	Face Vertex for GeS-TM [185]	140

10.18	Initialization of Dynamic OBUF LUT for Octree Coding (TriSoup in TMC13) [43]	140

10.19	Fix for Visual Gap at Slice Boundary [186]	141

10.20	Reconstructing Non-Closed Surfaces in TriSoup Nodes [187] (TMC13 and GeS-TM)	144

10.21	Improvement of TriSoup Surface Reconstruction [188] (TMC13 and GeS-TM)	145

10.22	Triangle Voxelization Process Simplification [189] (TMC13 and GeS-TM)	146

10.23	Versatile Geometry Quantization of TriSoup [190] (GeS-TM)	146

10.24	TriSoup Node Sizes Non-powers of Two [191] (GeS-TM)	146

10.25	On the OBUF Scheme for TriSoup Vertex Presence Flag [192] (GeS-TM)	146

10.26	Speedup of Triangle Soup Ray Tracing [193] (TMC13)	149

10.27	Local Quality Units for GeS-TM [194] (GeS-TM)	150

10.28	Memory Reduction of Trisoup Vertex Position Coding Using OBUF [195][196] (GeS-TM)	150

11	Entropy Coding	151

11.1	Simplification of Bypass Mode [197]	151

12	Fine-Granularity Slicing Using Layer-Group Structure	153

12.1	Layer Group-Based Slice Segmentation [198]-[204]	153

12.2	Geometry Slicing	156

12.2.1	Coding Process	156

12.2.2	Syntax and Semantics	160

12.2.3	Context Buffer Management	165

12.2.4	Managing Number of Points Per Slice [208]	168

12.2.5	Adaptive Planar Eligibility Based on Density [209]	170

12.2.6	Occupancy Tree Depth Signalling [216]	171

12.2.7	Direct Mode Eligibility Condition on Layer-group Boundary [217]	172

12.2.8	Improved Context Buffer Management [218]	172

12.2.9	Syntax Refinement for Fine Granularity Slice [219]	172

12.3	Attribute Slicing	172

12.3.1	Coding Process [210][211][212]	172

12.3.2	Syntax and Semantics	174

12.3.3	Context Reference Signalling [213]	176

12.3.4	Subgroup Quantization Weight [214]	177

13	References	179

13.1	Predictive Geometry Coding	179

13.2	Octree Geometry Coding	180

13.3	Inter Prediction for Geometry Coding	181

13.4	Inter Prediction for Attribute Coding	183

13.5	Attribute Coding	185

13.6	TriSoup Coding	186

13.7	Entropy Coding	188

13.8	Fine Granularity Slicing	188

# Predictive Geometry Coding

## Adaptive Azimuthal Angle Quantization [1][2]

When using spherical coordinates in predictive geometry coding of LiDAR acquired point clouds in G-PCC Ed. 1, azimuthal angles are quantized regardless of the distance between the points and the LiDAR acquisition head. The sampling result of this quantization is roughly as illustrated in Figure 1. It shows that the sampling density is high close to the origin, where a spinning sensors head is located, and becomes low in regions far from the spinning sensors head. Depending on the value Δϕ, one gets either too much precision for points (r1, ϕ1) close to the spinning sensors head or not enough precision for points (r2, ϕ2) far away. In the first case, for close points there is too much coded information for the residual error of azimuthal angle prediction. On the other hand, in the second case, there is not enough information coded for the residual error of azimuthal angle prediction of faraway points to have accurate precision on inverse transformed (x, y) values, thus leading to higher magnitude residual error in cartesian coordinates (xres, yres) to be coded. In both cases, the compression of the azimuthal angle ϕ is not optimal. In summary, the uniform quantization of ϕ does not lead to optimal representation of the point positions when considering the overall compression scheme of points in cartesian space.

Figure 1: Sampling of azimuthal angles and radius using uniform quantization, as in G-PCC Ed. 1.

The proposed method [1][2] adaptively quantizes the azimuthal angle according to the radius, resulting in improved compression performance. To compress more efficiently, it is proposed to use an adaptive quantization step of the azimuthal angle ϕ. Using the value of the reconstructed radius r2D, the proposed non-uniform adaptive angular quantization step is changed to:

Δϕ(r2D) = Δϕarc / r2D.

By using this non-uniform quantization step, the length of the arc resulting of the Δϕ(.) quantization step is uniform for any radius r1, r2 as this length is equal to r1.Δϕ(r1) = Δϕarc = r2.Δϕ(r2).

This non-uniform quantization step in ϕ domain is thus providing a uniform quantization of circular arcs, with quantization step Δϕarc, for any radius as is illustrated in Figure 2 and Figure 3. Figure 3 also shows the more uniform angular sectors implied by uniform quantization of the circular arcs, leading to more uniform maximum error introduced by the quantization of ϕ.

Figure 2: Proposed non-uniform quantization of the azimuthal angles, leading to uniform quantization of arcs.

Figure 3: Uniform quantization of circular arcs using Δϕarc quantization step.

Implementation details are described in [1] with some additional modifications in [2]. The integer division in inverse quantization of the azimuthal residual is approximated by using the Newton-Raphson division approximation algorithm. In addition, the internal precision for representing azimuthal angles is increased (e.g., 24 bit for lossless), which necessitated a modification of the implementation of integer sine and cosine functions to keep 32 bit arithmetic [2], but the modification does not affect the normative definition of these functions. It was also necessary to adapt the scaling of spherical coordinates for attribute coding due to the increased precision.

The improved quantization of azimuthal angle has been made backward compatible with G-PCC Ed. 1 by adding a flag in the geometry parameter set extension to enable/disable the feature.

## Improved Coding of Azimuthal Angle Residual [3][7]

The following method to improve the coding of the azimuthal angle residual is implemented on top of the adaptive azimuthal angle quantization that is described in the previous section 3.1. When using spherical coordinates in predictive geometry coding of LiDAR acquired point clouds in G-PCC Ed.1, the prediction of the azimuthal angle of a point can be refined by adding a number ‘k’ (coded in bitstream) of azimuthal steps ‘φstep’ to the azimuthal angle prediction ‘φ-n’ provided by the ‘n’-th predictor:

φpred = k * φstep + φ-n.

The azimuthal step ‘φstep’ may basically correspond to the rotation performed by the LiDAR sensor head between two successive attempts for the acquisitions of points with a laser at a given elevation angle. It corresponds to the azimuthal angle provided by:

φstep = geom_angular_azimuth_speed_minus1 + 1,

where ‘geom_angular_azimuth_speed_minus1’ is obtained from the geometry parameter set (GPS).

In G-PCC Ed.1, there is no constraint on the value of ‘k’. Thus, the residual ‘φres’ of the prediction of the azimuthal angle ‘φ’ by predictor ‘φpred’:

φres = φ - φpred,

is unbounded.

The presented method imposes that the value of ‘k’ shall be set equal to:

k = round((φ - φ-n) / φstep),

(as it is already the case in G-PCC Test Model TMC13v12) in order to bound the residual ‘φres’ such that it fits in the interval [- φstep/2; + φstep/2].

More precisely, in the context of the adaptive quantization of azimuthal angle described in section 3.1, the quantized azimuthal angle residual ‘Qφres’ will satisfy:

-Qφ(φstep/2, r) = Qφ(-φstep/2, r) ≤ Qφres = Qφ(φres, r) ≤ Qφ(φstep/2, r),

where ‘Qφ(x, r)’ is the adaptive quantization of ‘x’ based on the coded radius ‘r’.

Then, by using the value of the bound ‘B = Qφ(φstep/2, r)’, the presented method improves the entropy coding of the quantized residual ‘Qφres’.

First, bound ‘B = Qφ(φstep/2, r)’ is computed for each point as follows:

const int rec_radius_scaling = rPred + residual[0] << 3; // ~r*2*pi

auto speed_r = int64_t(_geomAngularAzimuthSpeed)*rec_radius_scaling;

int phiBound = divExp2RoundHalfInf(speed_r, _geom_angular_azimuth_scale_log2+1);

Then, coding is performed as illustrated in Figure 4. If bound ‘B’ equals zero, the quantized residual ‘Qφres’ is zero, hence, no coding is needed. Otherwise, a flag is coded to indicate if ‘Qφres’ is equal to zero. If it is nonzero, a sign bin is coded. Then if bound ‘B’ equals one, ‘Qφres’ is either minus one or one, hence, no more coding is needed. Otherwise, a flag is coded to indicate if the absolute value of ‘Qφres’ is equal to one. If it is not, but bound ‘B’ equals two, ‘Qφres’ is either minus two or two, and coding stops. Otherwise, the remainder (i.e., ‘|Qφres|-2’) is coded using an expGolomb code. The number of entropy coding contexts is equal to 24.

Figure 4: Entropy coding of quantized residual azimuthal angle using bound B.

## Improved Coding of Number of Azimuthal Angle Steps [4][7]

As described in the previous section 3.2, when using spherical coordinates in predictive geometry coding of LiDAR acquired point clouds, in G-PCC Ed.1 the prediction of the azimuthal angle of a point can be refined by adding a number ‘k’ of azimuthal angle steps ‘φstep’ to the azimuthal angle prediction ‘φ-n’ provided by the ‘n’-th predictor. When a radius is sufficiently small (i.e., when a point is sufficiently close to the LiDAR sensor), it is possible that a rotation of one ‘φstep’ angle does not change cartesian coordinates of the output point.

As illustrated in Figure 5, it may occur when more than one azimuthal directions of a laser from the LiDAR head (i.e., the (x, y) origin) are crossing the square to which belongs the point (the black disc) with 2D (x, y) coordinates. The squares correspond to a regular sampling of x and y axis in the cartesian space. The size of the squares depends on the precision of the input point cloud in case of lossless compression, or it roughly depends on quantization step in case of lossy compression.

Figure 5: Example of a point for which azimuthal angle can be equivalently predicted by more than one number of azimuthal angle steps.

To improve LiDAR point cloud compression, the presented method dynamically scales the azimuthal angle step ‘φstep’ when coding points that are close to the sensor (regarding the “default” ‘φstep’ precision).

### Method For Scaling Azimuthal Angle Step

In G-PCC Ed.1 a cartesian coordinates prediction (xpred, ypred) is obtained by:

(xpred, ypred) = (round(r2D-rec*cos(φrec)), round(r2D-rec*sin(φrec)),

where ‘φrec’ is the reconstructed azimuthal angle, and ‘r2D-rec’ is a reconstructed radius.

If implemented on top of the methods presented in the previous sections, ‘φrec= φpred + IQφ(Qφ(φres, r), r)’, with ‘Qφ’ the adaptive quantization of azimuthal angle described in section 3.1, ‘IQφ’ the inverse quantization, and ‘φres’ the azimuthal angle residual of the prediction.

In G-PCC Ed.1 and above methods, ‘r2D-rec = r << geom_angular_radius_inv_scale_log2’; in comparison to the coded point cloud cartesian precision, the radius ‘r’ which is internally used in, and coded by, the codec has a precision reduced by a number of bits equal to ‘geom_angular_radius_inv_scale_log2’ obtained from the geometry parameter set (this is equivalent to a quantization of the radius).

### Enabling Scaling of Azimuthal Angle Step

If both the improved quantization of azimuthal angle as presented in section 3.1 and the improved azimuthal angle residual coding as presented in section 3.2 are implemented, the bound ‘B = Qφ(φstep/2, r)’ (see section 3.2), which is computed for each point for entropy (de)coding, is used to enable the scaling of the azimuthal angle step. If the integer bound ‘B’ is equal to 0 or equivalently ‘B < 1’, then:

φpred = k * S(φstep,r) + φ-n,

where ‘S(φstep,r)’ is a scaled azimuthal angle step.

### Computing Scaled Azimuthal Angle Step

In order to code an optimal number ‘k’ of scaled azimuthal angle steps ‘S(φstep,r)’, an optimal ‘S(φstep,r)’ would become:

S(φstep,r) = 2geom_angular_azimuth_scale_log2/(r<<3).

One issue with this equation is that it requires an integer division in the decoder. Therefore, an approximation of the division in S(φstep,r) is implemented. To compute the approximation, the highest power is used of the ‘2n’ factor of ‘φstep’ such that ‘2n * φstep < 2geom_angular_azimuth_scale_log2/(r<<3)’.

The scaled azimuthal angle step ‘2n * φstep’ can simply be obtained by iteratively scaling ‘φstep’ and ‘φstep * (r<<3)’ by ‘2n’, starting from ‘n=0’, and using successive bitwise shift of 1 bit operations on both ‘2n * φstep * (r<<3)’ and ‘2n * φstep’ while ‘2n * φstep * (r<<3)’ is lower than 2π angle (i.e. ‘2geom_angular_azimuth_scale_log2’):

auto rec_radius_scaling = pred[0] + residual[0] << 3; // ~r*2*pi

auto azimuthSpeed = _geomAngularAzimuthSpeed;

if (rec_radius_scaling && rec_radius_scaling < Th0) {

const int32_t pi = 1 << _geom_angular_azimuth_scale_log2 - 1;

int32_t speed_r = azimuthSpeed*rec_radius_scaling;

while (speed_r < pi) {

speed_r <<= 1;

azimuthSpeed <<= 1;

}

}

Then, in encoder, the number of azimuthal steps ‘qphi’, and in both encoder and decoder, the azimuthal angle predictor updated by the number of azimuthal angle steps ‘pred[1]’, are computed using ‘azimuthSpeed’ instead of ‘_geomAngularAzimuthSpeed = gps.geom_angular_azimuth_speed_minus1 + 1’:

-qphi = residual[1] >= 0 ? (residual[1] + (_geomAngularAzimuthSpeed >> 1))

-    / _geomAngularAzimuthSpeed

-                    : -(-residual[1] + (_geomAngularAzimuthSpeed >> 1))

-    / _geomAngularAzimuthSpeed;

-pred[1] += qphi * _geomAngularAzimuthSpeed;

+qphi = residual[1] >= 0 ? (residual[1] + (azimuthSpeed >> 1))

+    / azimuthSpeed

+                    : -(-residual[1] + (azimuthSpeed >> 1))

+    / azimuthSpeed;

+pred[1] += qphi * azimuthSpeed;

residual[1] = point[1] - pred[1];

## Improved Radius Residual Sign Coding and Points Ordering [5][7]

Figure 6 illustrates the natural tendency of points that are acquired by a spinning sensor. The distance between the sensor and the acquired points on the objects is locally monotonous as a function of the sample indexed by the azimuthal angle ϕ.

Figure 6: Natural monotonicity of the radius associated with points depending on the sample indexed by the azimuthal angle.

Therefore, if a current radius is predicted by the radius of the precedingly acquired point by the same sensor, the radius residual rres exhibits a locally constant sign, either a series of negative radius residuals or a series of positive radius residuals. Of course, this is not always true but statistically significant enough to use this property to improve the compression of the sign sres of rres.

Furthermore, it was observed from QNX sequences that for a range of azimuthal angles of successive points encoded with predictive geometry, and for a same laser index (i.e., a same elevation angle), as is shown in Figure 7, it is possible to distinguish that a nearly piecewise constant curve is obtained. If we look closely it is not totally piecewise constant but we can attribute this to noise due to coordinate conversions coming from the conversion between raw acquisition data and final cartesian coordinates present in the input files and from the cartesian to spherical coordinate conversion in G-PCC. It shows that several points would have the same azimuthal angle, which is not coherent with the LiDAR acquisition mechanism where the head would be constantly turning, so the azimuthal angle may have been quantized (by the system?) to 360 angles steps while the sensor had made acquisitions with smaller than 1 degree angular steps.

Figure 7: Azimuthal angle values locally constant for successive points encoded for one laser, from a qnx file.

Second, by looking at radius values of the same successive points, as is shown in Figure 8, it can be observed that natural monotonicity of the radius, that should be observed for successively acquired points, is not verified locally when radius tends to decrease on average (zoomed window on the left). Discontinuities like saw tooths are observed: the radius increases locally. But when radius tends to increase on average (zoomed window on the right), the monotonicity of the radius is well observed. This is because the predictive geometry encoder is re-ordering the points so that the radius is locally increasing for a similar azimuthal angle.

Figure 8: Radius values for successive points encoded for one laser, from a qnx file.

This re-ordering in the encoder is not suited for correctly simulating a real ordering of the LiDAR acquired points, as monotonicity, rather than saw tooths, should be observed for both increasing and decreasing radiuses.

Therefore, the presented method encodes the sign of the radius by using entropy coding contexts which are chosen based on the monotonicity assumption. And second, the method re-orders points in the encoder when azimuthal angles are similar to better reflect the monotonicity that would normally be obtained.

### Improved Radius Residual Sign Coding

In the predictive geometry encoder, the sign of a radius residual is encoded with a single entropy coding context. Since the radius residual sign should be more or less piecewise constant when the radius is predicted from the preceding point radius (i.e., parent node in the predictive tree), the sign probability would be highly correlated with the sign value of the radius of preceding encoded point, when the parent node is used as a predictor. Moreover, this probability should increase when the successively coded points have similar azimuthal angle (i.e., the number of azimuthal steps encoded in the bitstream and added to the predictor is zero).

Therefore, the presented method uses a table of 2x2x2x2 (i.e., 16) contexts as follows:

ctxsign = ctxTab[Iprevious][Ipenulm][Ilast][sres,prec],

where ‘ctxTab’ it the table of contexts, ‘Iprevious’ is a Boolean value indicating if the selected predictor is the parent node, ‘Ipenulm’ is a Boolean value indicating if the coded number of azimuthal steps for preceding point is equal to zero, ‘Ilast’ is a Boolean value indicating if the coded number of azimuthal steps for the current point is equal to zero, and ‘sres,prec’ is a Boolean value indicating the sign of the last coded radius residual.

### Improved Points Ordering

To simulate LiDAR sensor points ordering, points are ordered first by increasing ϕc azimuthal angles, ϕc corresponding to azimuthal angle ϕ quantized according to azimuthal step ϕstep:

ϕc = ϕ / ϕstep,

with ϕstep = geom_angular_azimuth_speed_minus1 + 1, and secondly by increasing radius.

Then, for each ϕc, in increasing order, the minimum radius rmin-ϕc and the maximum radius rmax-ϕc are respectively the radiuses of the first and of the last ordered points having that ϕc value. The absolute difference to the radius of the last point with a lower ϕc value (e.g., rlast- ϕc-1) is computed for rmin-ϕc and rmax-ϕc. If the absolute difference is lower for rmax-ϕc, the order of the point with ϕc azimuthal angle is reversed, so that radiuses are ordered in decreasing order. This allows to increase the monotonicity of the radius, by minimizing the radius jumps between points with successive ϕc azimuthal angle values, as is illustrated in Figure 9. On the left the points are ordered with radius r in increasing order (as in TM) and on the right is the result of reversing the order of the points if it is better.

Figure 9: Radius ordering based on jump (in red) minimization.

## Improved Predictor List [6][7]

In G-PCC’s predictive tree, for each node in the prediction tree, one predictor index ‘n’ is encoded in the bitstream. This index points to a selected predictor PRn among a list of possible predictors. When angular mode is used (for LiDAR point cloud) the possible predictors may be one of:

- “None”: PR0 = (rmin, φ0, θ0), where rmin is the minimum radius value (provided in the geometry parameter set), and φ0 and θ0 are equal to 0 if the node has no parent, or are equal to φ and θ values of the point coded in the parent node;

- “Delta”: PR1 = p0 = (r0, φ0, θ0), where r0, φ0 and θ0 are respectively the radius r, the azimuthal angle φ and the laser index θ values of the parent point p0 coded in the parent node;

- “Linear2”: PR2 = 2*p0-p1, where p0 and p1 are the parent and grandparent points/nodes;

- “Linear3”: PR3 = p0+p1-p2, where p0, p1 and p2 are the parent, the grandparent and great grandparent points/nodes;

A prediction residual (rres, φres, θres) is obtained in the encoder by

(rres, φres, θres) = (r, φ, θ) – PRn – (0, k * φstep, 0),					(1)

where PRn is one of the predictors PR0, PR1, PR2 or PR3, and k is a number of azimuthal angle steps (φstep) to be added to the prediction. The prediction index ‘n’ and the number ‘k’ of azimuthal angle steps are encoded in the bitstream for each node, while the value of (φstep) is encoded in the geometry parameter set by geom_angular_azimuth_speed_minus1. The residual (rres, φres, θres) is also encoded in the bitstream.

With the improved quantization of azimuthal angle proposed in section 3.1, φres is additionally quantized according to the radius r before being encoded in the bitstream.

In both encoder and decoder side, the coordinates (rdec, φdec, θdec) of points are retrieved by doing the reverse process: a reconstructed point (rdec, φdec, θdec) is obtained by

(rdec, φdec, θdec) = PRn + (0, k * φstep, 0) + (rres, φres,rec, θres),

where φres,rec is obtained from the quantized φres after being inversely quantized.

PR2 and PR3 predictors (aka Linear2 and Linear3 predictors) may be efficient for non-angular coordinate prediction with quite regular sampling in space (for instance for 3D fused map contents). But, in the presence of noise, the candidate predictors PR2 and PR3 are generally not efficient. Therefore, for LiDAR point clouds, they are rarely selected because point cloud data captured by spinning sensor head are usually noisy due to the very nature of the acquisition process. PR1 (aka Delta predictor) is then very often selected as it is less sensitive to noise.

However, sensors generally capture objects at various distances from the sensor and huge jumps in radius between two successive points may be observed, leading to high-magnitude residual radial values to be encoded in the bitstream. Without reordering the input points or building more complex trees, which would introduce delays, neither candidate predictor PR1 nor PR2 or PR3 are suited for the prediction of these jumps. Candidate predictor PR0 may sometimes be useful if the object is close to the sensor (i.e., close to the minimum radius), but for objects far from the sensor, the candidate predictor PR0 is also inefficient. Moreover, a disadvantage of the candidate predictor PR0 is the introduction of a latency of one frame in the processing of the point cloud data in order to get the suited minimum radius. For real time processing, a suboptimal value, like 0 for instance, would be used.

Therefore, for LiDAR content, it seems interesting to use predictors that would be more suited to the nature of the data, without needing reordering points. The presented method dynamically builds and updates a list of predictors as described in more details in the next section.

### List of Predictors

A dynamic list of predictors is derived to perform better prediction after a laser beam has moved from a first object, with a first distance, to another object, with a different distance, has passed over it and is passing back to the first object. It may occur, for instance, when one object is in front of another one (like a car in from of a wall, for instance), or when an object has holes (walls with open doors or windows, or entrance wall for instance) as could be illustrated by Figure 10.

Figure 10: Example of a laser probing two different objects.

Instead of using the list of G-PCC predictors, a list of N predictors is built from a prediction buffer of N pairs of one radius and one azimuthal angle (rn, φn). The predictors derivation is detailed in subsection 3.5.2 and the buffer management is explained in subsection 3.5.3.

The coding of the predictor index is simply performed using a unary coding with one context per predictor index.

### Derivation of the predictors

The derivation of a predictor is performed as follows:

- If the point being predicted is the first point of the tree (i.e., there is no parent node), the predictor PR0 is set equal to (rmin, 0, 0), the other predictors PRn>0 are set equal to (0, 0, 0).

- If the point has a parent point,

  - the predictor PR0 is set equal to (r0, φ0, θ0), where θ0 is the laser index θ value of the parent point p0 coded in the parent node, and where (r0, φ0) is the first pair in the buffer (as will be understood from the buffer management, it is also equal to respectively the radius r, and the azimuthal angle φ of the parent point p0 coded in the parent node);

  - the predictors PRn>0 are set equal to (rn, φn + k* φstep, θ0), where θ0 is the laser index θ value of the parent point p0 coded in the parent node, and where (rn, φn) is the n-th pair in the buffer, and k equals 0 if | φ0 - φn | < φstep, else k equals the integer division (φ0 - φn) / φstep.

Since it is better to avoid integer division in decoder, (φ0 - φn) / φstep is approximated using the divApprox function of G-PCC: k = divApprox(φ0 - φn, φstep, 0).

### Management of the prediction buffer

The buffer used for the predictors’ derivation is managed as follows. Each pair of the buffer is first initialized to (0, 0). After the (de)coding of a point, the buffer is updated as follows:

- If the absolute value of (de)coded radius residual rres is higher than a threshold Th, it is considered that the laser has probed a new object. Then a new element (r0, φ0) is inserted in front of the buffer, with r0 and φ0 the reconstructed radius and the reconstructed azimuthal angle of the (de)coded point. The last element of the buffer is discarded. This is performed by letting the buffer element (rn, φn) be equal to (rn-1, φn-1) for n=3 to 1. Then, setting the first buffer element values from the decoded point.

- If the absolute value of (de)coded rres is not higher than the threshold Th, it is considered that the laser has probed an object present in the buffer. Then, the element of the buffer with index predIdx, corresponding to the index of the predictor that has been used for the prediction, is moved to the front of the list and is updated with (r0, φ0) the reconstructed radius and the reconstructed azimuthal angle of the (de)coded point. This is performed by letting the buffer elements (rn, φn) be equal to (rn-1, φn-1) for n=predIdx to 1, then, setting the first buffer element values from the decoded point.

- Th is equal to ps.predgeom_radius_threshold_for_pred_list and has been fixed in the encoder to 2048 >> ps.geom_angular_radius_inv_scale_log2.

## Improved Coding of Magnitude of Radius Residual [8][9]

This method improves the magnitude encoding of radius residual in G-PCC’s predictive tree geometry coding for LiDAR-acquired point cloud compression.

### Radius Residual Coding in G-PCC Ed.1

In the framework of predictive tree-based point cloud coding in G-PCC Ed.1, to encode each point in a predictive tree coding, the point P is predicted by a predictor point Ppred, and a prediction residual (r2D_res, φres, θres) between the point P and the predictor point Ppred is encoded into bitstream after prediction. The detailed encoding process of prediction residual in G-PCC follows the steps shown in Figure 11.

Figure 11: Encoding process of prediction residual in G-PCC predictive tree geometry coding.

As shown in Figure 11, for each point, firstly the radius residual r2D_res is encoded into bitstream, then azimuthal angle residual φres, and finally elevation angle residual θres is encoded into bitstream. During the encoding of the radius residual r2D_res, the magnitude of the radius residual is firstly encoded and then the sign of the radius residual r2D_res is encoded into the bitstream.

The encoding process of the magnitude of radius residual follows these steps:

- encodes the flag f0 which indicates if the value of r2D_res is equal to 0. If the value of flag f0 is true, then the coding of r2D_res is finished as r2D_res =0 has been coded; otherwise, if the value of f0 is false, then continue to encode the absolute value |r2D_res| of radius residual.

- computes the bit number Numbits needed to represent |r2D_res| by using the equation

Numbits=log2(|r2D_res|-1)                   (1)

- encodes each bit bN representative of Numbits iteratively by an adaptive entropy encoder.

- bypass encodes the lowest (Numbits -1) bits bm of value |r2D_res| into bitstream from lowest bit to highest bit. The highest bit of value |r2D_res| is always 1, so it is not needed to be encoded into bitstream.

### Problem Description

In current G-PCC framework, the compression of radius residual magnitude is not efficient due to the weakness of the entropy coding of it, because the entropy coding of radius residual magnitude does not make use of any prior information from prediction process, and it encodes the bits of absolute value |r2D_res| of radius residual by bypass coding, which causes not optimal compression efficiency.

### Proposed Coding Method of the Magnitude of Radius Residual

The proposed method is shown in Figure 12, it uses context-adaptive entropy coder to encode bits of magnitude of radius residual, and the determination method of context will be described in Section 3.6.5.

As shown in Figure 12, after obtaining the radius residual, firstly, it determines a value of a flag f0, signalling whether the radius residual r2D_res is equal to 0, and encodes it into the bitstream by a binary entropy encoder based on the context ctx. If the radius residual r2D_res is equal to 0, the encoding of radius residual r2D_res is finished as r2D_res =0 is encoded; otherwise, the value of a flag f1, signalling whether the absolute value |r2D_res| is equal to 1, is determined and encoded into the bitstream by a binary entropy encoder based on the context ctx.

Then, if the absolute value |r2D_res| is equal to 1, the encoding of the magnitude of radius residual r2D_res is finished; otherwise, the value of a flag f2, signalling whether the absolute value |r2D_res| is equal to 2 or not, is determined and encoded into the bitstream by a binary entropy encoder based on the context ctx. If the absolute value |r2D_res| is equal to 2, the encoding of the magnitude of radius residual r2D_res is finished; otherwise, the absolute value (|r2D_res|-3) is encoded using exp-Golomb coding based on the selected context ctx, and the determination method of context.

Figure 12: Overview of proposed encoding method of magnitude of radius residual.

Figure 13: Overview of proposed decoding method of magnitude of radius residual.

The overview of proposed decoding method of magnitude of radius residual is shown in Figure 13.

### Prior Information in Prediction Process

In the prediction process proposed in Section 3.5, a dynamic predictor list L is used to obtain better prediction for cases when a laser beam has moved from a first object to another object, and then passes back to the first object, or cases when an object has holes (walls with open doors or windows for instance). A predictor point (Ppred) is selected from the predictor list L to minimize the bit cost of the residual between the predictor point (Ppred) and the current point P, and the predictor point (Ppred) is determined based on the element pointed by the predictor index (i) within the predictor list (L).

Statistically, cases when the obtained radius residual r2D_res is greater than the residual threshold Th-r2D happen rarely in point cloud dataset, which means most points in point cloud are in the same object as previous coded point, and the best predictor for each of them is more probably to be the last already coded point in the same object, which is always put in the front of the dynamic predictor list to get the updated list used for predicting next point. Consequently, the index i of selected predictor for these points is more probably 0. Cases where the obtained radius residual r2D_res is greater than (or equal to) residual threshold Th-r2D only happen when points are at the edge of the objects of different depths, and the last already coded point is naturally not the closest point of current coded point since it is from an object of different distance, so the encoder may search from other elements (not the first element) in the list to get best predictor and then gets i>0.

If the predictor index (i) is equal to 0, then it is more probable that the point P is in the same object as the predictor point (Ppred), which means the point P is not in a new object, so the magnitude of radius residual is more probably smaller; and if the predictor index (i) is not equal to 0, then it is more probable that the current point P is not in the same object as the predictor point (Ppred), which means it is in a new object, so the magnitude of the radius residual is more probable to be larger than when the index (i) is equal to 0. Thus, the statistics of the magnitude of the radius residual when the predictor index (i) is equal to 0 is different from when the predictor index (i) is not equal to 0.

Also, in the prediction process, the prediction of the φ coordinate can benefit from the regular distribution of points along the φ axis by coding an integer number qphi (illustrated in Figure 14), which represents the number of elementary azimuthal step ∆φ used to obtain the refined predictor point (Ppred') from the predictor point (Ppred). The relationship between them can be described by

Ppred'= Ppred+(0,qphi*∆φ,0).

As shown in Figure 14 (a), if qphi is equal to 0, then the point P is acquired with the same or similar azimuthal angle as the predictor point Ppred, and the two points are spatially close, so the radius residual between them is naturally to be very small, and almost close to 0; If qphi is not equal to 0, as shown in Figure 14 (b), then there is a high probability that the predictor point Ppred is not close to current point P, so the radius residual magnitude is much larger. Thus, the statistics of radius residual magnitude when qphi is equal to 0 is different from that when qphi is not equal to 0.

      - Case when qphi is equal to 0.

      - Case when qphi is not equal to 0.

Figure 14: Relationship among integer number qphi, the predictor point (Ppred) and the refined predictor point (Ppred').

In the proposed method, in order to profit from the prior information described above in prediction process, the magnitude of the radius residual is entropy coded by using context-adaptive entropy coder. The context used to code the magnitude of radius residual is selected based on the predictive information comes from two aspects:

1. predictor index i in the predictor list L,

2. integer number qphi of elementary azimuthal step between the predictor φpred and the refined predictor φpred'.

### Context Determination Method

To encode each bit of radius residual magnitude, the proposed method determines context index ctxIdx by using predictor index i and the integer number qphi of elementary azimuthal step according to equation below,

ctxIdx=0,  if i≠0 and qphi=0 1,  if i≠0 and qphi≠0 2,  if i=0 and qphi=0  3,  if i=0 and qphi≠0

and then select a context ctx in context table ctxTable_T to entropy encode the bits of magnitude of radius residual according to ctxIdx.

ctx=ctxTable_T[ctxIdx]

### Complexity Reduction

To encode radius residual of each point, the function to estimate bits of exp-Golomb coding is called several times to select a predictor, this will cost a lot of running time. To reduce the complexity, a simpler coarse method is proposed for the function to estimate bits of exp-Golomb coding.

Since the radius residual is large, so the order k of exp_golomb coding used is set as 2, assume the suffix bits number as n, and the encoded value radius resR (is (|r2D_res|-3)) satisfies

22+23+24+…+2n-1≤resR≤ 22+23+24+…+2n-1

then n is equal to log2resR+4, prefix bits number is n-2, and the total bits needed for exp-Golomb coding is 2* log2resR+4-1, so the function can be replaced by following implementation in code:

## Disabling of Cartesian Residual Coding [10][11]

In the framework of predictive tree-based point cloud coding in G-PCC Ed.1, after prediction using predictive tree structure, two residuals are encoded into the bitstream, which are spherical residual1 (rres,φres,θres) and cartesian residual2 (xres,yres,zres), respectively. As shown in Figure 15, the input data is represented in cartesian coordinates (x,y,z) and then converted into spherical domain. After generating the predicted points in spherical domain, residual1 (rres,φres,θres) is obtained by calculating the difference between the spherical position of the original points and the predicted points. The residual1 (rres,φres,θres) is coded into the bitstream by lossless coding. Subsequently, the predicted points with added residual1 are converted back into the cartesian domain, and the cartesian residual2 (xres,yres,zres) is obtained by calculating the difference between the cartesian position of the original points and predicted points after converting. In lossy coding conditions, the residual2 (xres,yres,zres) is quantized and the quantized residual2 Q(xres,yres,zres) is encoded into the bitstream.

Figure 15: Encoding process in G-PCC Ed.1 predictive tree geometry coding.

It was observed that the values of the quantized residual2 Q(xres,yres,zres) in cartesian domain are almost always 0 under lossy coding conditions, while residual2 is still coded in the bitstream. The proposed method uses a flag that indicates whether residual2 is coded or not. At encoder side, if this flag value is false then the encoding of quantized residual2 is disabled, otherwise, the coding is enabled, and lossless coding can be achieved as illustrated in Figure 16.

Figure 16: Framework of predictive geometry encoding with proposed method.

## Prediction List Index Coding [13][12]

Firstly, it is proposed to use truncated unary coding instead of the unary coding for the encoding of the predIndex. Secondly, it is proposed to set the maximum number of intra predictors used by the encoder in the geometry parameter set. The latter modification enables the decoder to allocate the right memory size for the dynamic list of intra predictors and the truncated unary code can be applied to the last predictor index.

## Removing Parsing Dependency in Predictive Geometry Coding [14]

It is proposed to remove the dependence of parsing on the decoding/reconstruction of points in predictive geometry coding. It is asserted that the parsing of syntax elements associated with the azimuth residual is dependent on the reconstructed value of radii of preceding points in the current frame. This dependence is reported to occur through the variable boundPhi. The presence of this dependence is asserted to be undesirable because it forces implementations to necessarily introduce decoding blocks within parsing modules, or tightly couple the parsing and decoding modules, thus resulting in implementation complexity. Predictive geometry coding in G-PCC Ed.1 maintained parsing independence and it is argued that this property should be maintained. It is suggested, therefore, to remove the elements in the parsing process that have dependence on reconstruction; particularly, the dependence of boundPhi on the parsing of azimuth residual syntax elements as highlighted below:

int r = rPred + residual[0] << 3;

auto speedTimesR = int64_t(_geomAngularAzimuthSpeed) * r;

int phiBound = divExp2RoundHalfInf(speedTimesR, _azimuthTwoPiLog2 + 1);

int32_t

PredGeomDecoder::decodeResPhi(int predIdx, int boundPhi, const bool interFlag

, int refNodeIdx

)

{

if (boundPhi == 0)

return 0;

int interCtxIdx = interFlag ? 1 : 0;

int ctxL = interFlag ? (refNodeIdx > 1 ? 1 : 0) : (predIdx ? 1 : 0);

//int ctxL = predIdx ? 1 : 0;

if (!_aed->decode(_ctxResPhiGTZero[interCtxIdx][ctxL]))

return 0;

int absVal = 1;

if (boundPhi > 1)

absVal += _aed->decode(_ctxResPhiGTOne[interCtxIdx][ctxL]);

int interEGkCtxIdx = interFlag ? (refNodeIdx > 1 ? 2 : 1) : 0;

if (absVal == 2 && boundPhi > 2)

absVal += _aed->decodeExpGolomb(

1, _ctxResPhiExpGolombPre[interEGkCtxIdx][boundPhi - 3 > 6],

_ctxResPhiExpGolombSuf[interEGkCtxIdx][boundPhi - 3 > 6]);

bool sign = _aed->decode(_ctxResPhiSign[ctxL][interCtxIdx ? 4 : _resPhiOldSign]);

_resPhiOldSign = interFlag ? (refNodeIdx > 1 ? 3 : 2) : (sign ? 1 : 0);

return sign ? -absVal : absVal;

}

The parsing of the azimuth residual depends on the variable boundPhi both for conditioning the signalling of syntax elements, but also selection of contexts. The following changes are proposed (red strikethrough to indicate removal):

int32_t

PredGeomDecoder::decodeResPhi(int predIdx, int boundPhi, const bool interFlag

, int refNodeIdx

)

{

if (boundPhi == 0)

return 0;

int interCtxIdx = interFlag ? 1 : 0;

int ctxL = interFlag ? (refNodeIdx > 1 ? 1 : 0) : (predIdx ? 1 : 0);

//int ctxL = predIdx ? 1 : 0;

if (!_aed->decode(_ctxResPhiGTZero[interCtxIdx][ctxL]))

return 0;

int absVal = 1;

if (boundPhi > 1)

absVal += _aed->decode(_ctxResPhiGTOne[interCtxIdx][ctxL]);

int interEGkCtxIdx = interFlag ? (refNodeIdx > 1 ? 2 : 1) : 0;

if (absVal == 2 && boundPhi > 2)

absVal += _aed->decodeExpGolomb(

1, _ctxResPhiExpGolombPre[interEGkCtxIdx][boundPhi - 3 > 6],

_ctxResPhiExpGolombSuf[interEGkCtxIdx][boundPhi - 3 > 6]);

bool sign = _aed->decode(_ctxResPhiSign[ctxL][interCtxIdx ? 4 : _resPhiOldSign]);

_resPhiOldSign = interFlag ? (refNodeIdx > 1 ? 3 : 2) : (sign ? 1 : 0);

return sign ? -absVal : absVal;

}

As an added benefit, this also simplifies the parsing of the azimuth residual and reduces a few contexts.

## Radius Residual Coding by the Threshold of the Number of Azimuthal Angle Steps [15][16]

In the implementation of angular mode (for Spinning LiDAR point clouds) in predictive geometry, to encode (decode) each bit of radius residual magnitude, context index ctxIdx is determined by the following condition:

ctxIdx=0,1,2,3,ififififi=0 and qphi=0i≠0 and qphi=0i=0 and qphi≠0i≠0 and qphi≠0

Where i denotes the predictor index and a context ctx is selected in context table ctx_Table_T according to ctxIdx.

ctx=ctxTable_T[ctxIdx]

It is proposed to apply a threshold to qphi to improve radius residual coding for intra predictive geometry. The proposed method changes the conditions for the context selection of radius residuals as follows.

ctxIdx=0,1,2,3,ififififi=0 and absqphi≤thrQphii≠0 and absqphi≤thrQphii=0 and absqphi>thrQphii≠0 and absqphi>thrQphi

Where thrQphi is the threshold for determining the magnitude of qphi and is given by GPS. abs(qphi) represents the magnitude of qphi. Note that if thrQphi is 0, the behavior is identical to the current implementation. The number of contexts used in the proposed method is identical to that of the anchor method. The thrQphi is obtained using the existing parameter positionAzimathSpeed in the following equation.

thrQphi=1,600/positionAzimathSpeed

The positionAzimuthSpeed is the parameter of the azimuthal angle step for each sequence and each rate point given by configurations. The following table shows thrQphi of the result of the calculation for each sequence and each rate point.

| Sequences | thrQphi |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | lossless | r06 | r05 | r04 | r03 | r02 | r01 |
| Ford | 0 | 0 | 0 | 3 | 6 | 12 | 12 |
| Qnx | 0 | 0 | 0 | 0 | 0 | 1 | 1 |

# Octree Geometry Coding

## Residual Coding of Angular Mode in IDCM [17][20]

### 3.1.1 Motivation

In G-PCC Ed.1, if angular mode coding is enabled for the Inferred Direct Coding Mode (IDCM) of the octree-based geometry coding, a recursive splitting of a vertical interval, and at each iteration, a bin signalling the split is coded based on contexts determined from the closest laser position. Conversely, in predictive tree geometry coding, the vertical residual is coded, and it doesn’t use a recursive splitting method for the vertical interval. The proposed method harmonizes the angular mode in IDCM.

### 3.1.2 IDCM Angular Mode in G-PCC’s Ed.1

After finding the closest laser index laserIdx for the current point and coding the laser index residual between laser index of current point and current node, the vertical interval is iteratively encoded by splitting. The iterative splitting method is shown in Figure 17.

Figure 17: Iterative vertical interval splitting of angular mode in IDCM.

### 3.1.3 Improved Method

Three changes have been proposed:

- Better determination method of closest laser by using the vertical correction zList.

- Better compression method for closest laser residual.

- Vertical residual encoding: the proposed method encodes the vertical residual instead of the splitting of the interval; this method is similar to the one used in predictive tree geometry coding.

The detailed proposed coding process of angular mode in IDCM follows the following steps:

- Firstly, selecting the closest laser index of current point laserIdx according to the z coordinate of current point. To better determine laserIdx, the proposed method uses the vertical correction zList.

- Secondly, encoding the closest laser residual resLaser between laser index of current point laserIdx and the laser index of current node nodeLaserIdx by entropy coding. To better encode resLaser, the proposed method introduces a context determined based on the sign of resLaser of preceding encoded point to encode the sign of resLaser of current point, which is shown in Figure 18. And it uses a buffer _ResLaserBuffer[nodeLaserIdx] to store the resLaser of a preceding point, and updates the buffer for each point.

Figure 18: Entropy coding resLaser by introducing the sign of resLaser of preceding encoded point.

- Finally, encoding the vertical residual by using the known x coordinate of the current point xLidar, y coordinate of current point yLidar, theta angle of laser index thetaLaser and vertical laser offset zLaser to get the vertical predictor of the current point, and it doesn’t depend on iteratively determining each bit of z. So r (in x-y plane) can be got by using xLidar and yLidar,

r=xLidar2+yLidar2

and the vertical predictor zRec can be determined by:

zRec=thetaLaserlaserIdx*r-zLaser[laserIdx].

Then, the vertical prediction residual zRes is obtained by:

zRes=pos2-angularOrigin2-zRec,

where pos2 is the z coordinate of the current point. The residual zRes is encoded by entropy coding. The detailed encoding process of the proposed vertical residual method is shown in Figure 19. In the decoding process, the predictor zRec and the vertical residual zRes are used to obtain the reconstructed z.

Figure 19: Proposed vertical residual encoding of angular mode in IDCM [19].

## Improved Azimuthal Mode in IDCM [18][20]

### Azimuthal Mode in G-PPC Ed.1 IDCM

In the framework of occupancy tree-based point cloud coding, the azimuthal coding mode has been introduced to improve both the planar mode and the Inferred Direct Coding Mode (IDCM). It uses azimuthal angles of already coded nodes to provide prediction information for coding current point in IDCM, and this very significantly improves the compression of binary occupancy coding through the prediction of x or y-coordinate bits in IDCM nodes.

IDCM is used to code directly the position of points within a current node belonging to the occupancy tree. When the azimuthal angular coding mode is activated, azimuthal angles are obtained relative to a coordinate origin in the horizontal xy plane. Firstly, an IDCM node azimuthal coding direction (either along the x axis or the y axis), for which point coordinates will be encoded by azimuthal angle contexts, is selected based on the position of the current node relative to the x and y axes. The azimuthal coding direction is along the x axis if xnode≤ynode, otherwise, it is along the y axis. In case the selected azimuthal coding direction is along the x (respectively y) axis, the following coordinate coding are processed in order:

- the y (respectively x) coordinate bits are bypassed coded,

- the x (respectively y) coordinate bits are entropy coded by using azimuthal angle contexts,

- and z coordinate are entropy coded.

The detailed azimuthal IDCM coding process in case the azimuthal coding is along the x axis is depicted in Figure 20.

Figure 20: Azimuthal IDCM encoding process in G-PCC Ed.1.

When encoding a point in IDCM node, firstly determine the azimuthal coding direction (here x), an azimuthal prediction angle φpred for current node, and an initial interval along azimuthal coding direction. The two angles φleft and φright are associated with the left half interval and the right half interval, and they are determined from the position xleft and xright representative of the leftmost bound of the interval and the middle of the interval, respectively. The positions of xleft and xright are depicted below in Figure 21.

An azimuthal angular context is selected based on prediction angle φpred and the two angles φleft and φright to encode bit bd. The encoded bit bd at each depth d indicates that the coordinate of the point is located either in the left half interval (bd=0) or the right half interval (bd=1), and then entropy code bit bd into bitstream by using the selected context. Next, update the interval as the half interval to which the points belong (as indicated by the coded bit bd) and code the next bit (bd+1) of the coordinate iteratively, until a minimum interval size (typically size is 1) is reached.

Figure 21: Implementation of specific φleft and φright in current G-PCC Ed.1.

### Problem Description

One problem in current Ed.1 is that the angles φleft and φright used in azimuthal IDCM coding are slightly sub-optimal and this leads to non-optimal compression efficiency. Figure 22 shows an exemplary node of size 8 along the x axis, which is the selected direction for azimuthal coding; angles φleft and φright are the leftmost bound of the interval and the middle of the interval, and a prediction angle φpred has been obtained from already coded points. The prediction angle φpred provides some predictive information on the point position, and if φpred points to the left half interval, it is statistically more probable that the coded point belongs to the left half interval (i.e., bd=0) than to the right half interval (i.e., bd=1). The angle difference |φpred-φleft| and |φpred-φright| can be used to select a context. For example,

- if |φpred-φleft|<|φpred-φright|, bd=0 is more probable, select a first context,

- otherwise bd=1 is more probable, select a second context.

When using the selected context in a context adaptive binary entropy coder (for example CABAC) to code the bit bd, the coding probabilities p will evolve naturally such that p(bd=1) <0.5 for the first context and p(bd=1) >0.5 for the second context. In Figure 22, the prediction angle φpred is pointing to the left half interval, which indicates that bd=0 is more likely than bd=1. However, it will select the second context because |φpred-φleft|≥|φpred-φright| with the configuration of angles φleft and φright in current Ed.1. This bad context selection impacts the cost of coding the bits bd, because it is likely that bd=0 is coded with the second context which has a probability p(bd=0)=1-p(bd=1)<0.5 and the coding cost

-log2(1-p(bd=1) )

is high. Therefore, the angles φleft and φright used for context selection in current Ed.1 are slightly sub-optimal and lead to non-optimal compression efficiency.

Figure 22: Problem in current Azimuthal IDCM coding.

If the angles φleft and φright are changed to optimal angles, which are shown in Figure 23, the value of bit bd can be well anticipated by context selection. In Figure 23, angle φleft is close to the middle of left half interval and φright is close to the middle of right half interval. Then |φpred-φleft|<|φpred-φright| and the first context is selected. The probability of bd=0 is more higher than bd=1, so it codes bd=0 with the second context which has a probability p(bd=0)>0.5, and the coding cost is low.

Figure 23: Illustration of somewhat optimal φleft and φright for determining contexts.

Another problem in G-PCC Ed.1 is that the implementation complexity is high, because angle φright,d is calculated by calling iatan2() function at each depth d.

### Proposed Angle Interpolation

The problem to solve is to obtain optimal angles φleft and φright to improve the compression efficiency while maintaining a reasonable complexity. Basically, maintaining a reasonable complexity mostly means minimizing the number of calls to the complex arctangent function.

It is proposed to use angle interpolation to obtain angles φleft and φright by using offsets and known angles φnode and φmiddle , and the proposed method can avoid the systematic call of iatan2() for φright at each depth d, and provide flexibility in the choice of φleft and φright.

The angle interpolation equation can be described as:

φleft,d=φnode,d+Offleft*(φmiddle,d-φnode,d)/s'd

φright,d=φmiddle,d+Offright*(φmiddle,d-φnode,d)/s'd

where s'd is half of the interval length sd at depth d, the relationship between these angles is shown in Figure 24. The angle φmiddle,d is associated with the middle of the interval, and it can be obtained by calling iatan2() function at each depth d, and φnode,d is associated with the left boundary of the interval at depth d. Angle φnode,d inherits from previous depth, and angles φnode,d and  φmiddle,d are kept for angle φnode,d+1 at next depth d+1. Figure 24 also illustrates the interval updating process. The inheriting equation can be described as:

- If bd=0, the updated interval at depth d+1 is the left half interval at depth d, then

φnode,d+1=φnode,d

- If bd=1, the updated interval at depth d+1 is the right half interval at depth d, then

φnode,d+1=φmiddle,d

Figure 24: The interpolation angles and detail interval updating illustration.

#### Determining Offsets

The basic idea to obtain good context selection is to obtain angles φleft and φright point close to the center of their respective half intervals, so it needs offsets  Offleft and Offright to interpolate angles φleft and φright, respectively. In our proposed method, offsets  Offleft and Offright are a compound of two elementary offsets offsetmid and offset2, the corresponding equation can be described as

Offleft=offsetmid-offset2

Offright=offsetmid+offset2.

The first offset offsetmid is used to determine angle φleft to be the middle of the left half interval and angle φright to be the middle of the right half interval. The middle of the half interval is defined as the average positions of all potential points belonging to the half interval, as depicted in Figure 25. It is not the straightforward (φmiddle,d-φnode,d)/2 for the left interval, because of the discrete position of the points. Considering a half interval of length s'd, potential points are located at position 0, 1, …, s'd-1 from the lower bound of the half interval. The mean position is:

offsetmid  = (0 + 1 + … + s'd-2 + s'd-1) / s'd.

Figure 25: Middle of discrete intervals.

However, it has been observed that optimal compression performance is obtained when angles φleft and φright do not point exactly to the centers of their half interval. To achieve better compression efficiency, it has been observed that introducing a second constant offset2 as a refinement of offsetmid leads to a more efficient selection of the contexts and small extra compression gains are obtained. The basic effect of the second offset is to push slightly the angle φleft toward the lower bound of the interval and the angle φright toward the upper bound of the interval. The value for offset2 has relationship with interval size, and the implementation of offset2 is:

#### Adjusting Prediction Angle φpred

In current G-PCC Ed.1, after each division of interval, φpred is adjusted closest to φnode,d. It is proposed to adjust it close to φmiddle,d, which is more accurate prediction of angle φcurrent, which indicates current point position. Prediction angle φpred can be calculated as:

φpredn=φal+n*∆φ

Where n is a selected rounding value to minimize |φcurrent-φpredn|.

#### Reducing Complexity

In the presented method, the angle φmiddle,d used for interpolation can be obtained by calling iatan2() function at each depth d. To reduce complexity, we propose to determine φmiddle,d by interpolation with angles φnode,d and φtop,d, the interpolation equation can be described as:

φmiddle,d=φnode,d+φtop,d≫1

The interpolation method for φmiddle,d can only be used when φnode,d and φtop,d are not ‘too far apart’, so we introduce a threshold th, and th=1≪13. If the interval length sd at depth d is smaller than th, then determine φmiddle,d by interpolation, otherwise, determine it by calling iatan2()  function.

## Context Enhancement of Azimuthal IDCM [19][20]

### Context Selection in G-PCC’s Ed.1 Azimuthal Coding Mode

The azimuthal angle contexts have 8 values and depend on:

- if (φleft-φpred) and (φright-φpred) have the same sign or not (2 possible values)

- assume m=φleft-φpred, M=φright-φpred, if m>Mor not (2 possible values)

- if M>2m or m>2M (2 possible values)

The detailed context selection process in the G-PCC Ed.1 standard is graphically depicted in Figure 26. The blue horizontal line represents the x-interval at depth d, and angles φleft and φright are respectively associated with the middle of the left half x-interval and the right half x-interval. Since there are 8 contexts in total, the context value is represented by 3 bits b2b1b0, and each bit bi indicates if one of the three conditions described above is fulfilled or not.

Figure 26: Context selection in G-PCC Ed.1.

As shown in Figure 26, each context value represents a possible position range that φpred belongs to in current interval. If φpred points to the 1st position range, then context = 010 and the current point has a higher possibility to be in left half x-interval at current depth d. In other words, this means that p(bd =1) is very low. If φpred points to the rightmost 8th position range, then context =011 and the current point has a higher possibility to be in right half x-interval at current depth d. This means that p(bd =1) is very high. A context adaptive binary entropy coder (for example CABAC) is used to code the bit bd.  The coding probabilities, associated with each of the 8 contexts, will evolve naturally such that p(bd =1)<0.5 for ‘010’ context and p(bd =1)>0.5 for the ‘011’ context. As a general rule, assuming p is the probability of the bit bd to be 1, the cost for coding 1 is -log2(p), the cost for coding 0 is -log2(1-p). If φpred can provide a good prediction for which interval the current point belongs to (bd =1 or bd =0), then the cost for entropy encoding the bit bd will be lowered.

### Problem Description

As described above, in the current G-PCC Ed.1 software, the prediction angle φpred is directly used to select azimuthal angle contexts for entropy encoding bit bd together with angles φleft and φright. However, current method does not consider the “quality” of the predictor. Quality is understood as the capability of the angle φpred to anticipate correctly which half interval the current point belongs to.

The angle φpred is obtained from the azimuthal angle φal of a (closest) already coded node and a multiple n of the elementary azimuthal shift ∆φ. The integer n is chosen such as to obtain φpred=φal+n∆φ as the closest possible angle (among all possible choices for n) from the azimuthal angle φcenter associated with the center of the current node. The prediction quality of the predictor φpred can be influenced by several factors such as the interval length and the distance r between Lidar and the node. Some predictors φpred may be good or bad depending on the node location and the node size.

Figure 27: Bad prediction cases caused by too large/small distance r.

Figure 27 depicts two bad prediction cases caused by either too large or too small distance r, where the two nodes have the same node (interval) size, but they have different distance r from LiDAR. As for the closest node in blue in Figure 27, the distance r between LiDAR and left node is very small relative to the node size. As a consequence, the apparent angle φ1 associated with the node is large and it gets φ1≫∆φ. In this case, a point in the node may have been potentially probed by several (even many) sensor positions and the arbitrary choice of n, such that φpred=φal+n∆φ is the closest to φcenter, is unlikely to be accurate at representing which position of the sensor has actually probed the current point. Also, a small change ∆xpoint=±1 in the x point position of the current point would induce a large change ∆φpoint in the azimuthal angle φ associated with the point because of ∆φpoint =∆xpoint/r with a small radius r. This means that the azimuthal angle φ  of the current point is very sensitive to noise. This causes an unstable prediction result of the predictor φpred.  As for the furthest node in orange in Figure 27, the distance r between LiDAR and right node is so large that φ2≪∆φ where φ2 is the apparent angle associated with the node. Consequently, the bits bd provide a fine precision (smaller than ∆φ) that cannot be well anticipated by φpred that has been determined up to +/-  ∆φ/2.

Figure 28: Bad prediction cases caused by too big/small node size.

The good or bad prediction capability of the predictor φpred does not only depend on the distance of the node from the sensor. It also depends on the node size as exemplified on Figure 28. This figure depicts bad prediction cases caused by too big (blue node) or too small (orange node) apparent angle (φ1 or φ2) of the interval relative to ∆φ. The two nodes shown in Figure 28 have different node size. For the big node in blue, the node angle φ1 is so large that φ1≫∆φ, and it will cause the same problem as for small distance r . For the small node in orange, the node angle φ2 is so small that φ2≪∆φ, and it will cause the same problem as for large distance r.

### Improved Method

The presented method determines contexts that exhibit more accurate statistics for the probability of bits bd to 0 or 1. In particular, proposed enhanced contexts will be indicative of the quality of the predictor φpred. By doing so, context adaptive entropy coding is improved, and better compression efficiency is obtained.

#### Assess the prediction quality of predictor φpred

The proposed method is to use the relative values between ∆φ and apparent angle Sd of the (x- or y-) interval to determine better contexts. The apparent angle Sd is an estimation of the interval angle seen from the sensor at depth d, and the apparent angle Sd is defined to be:

Sd=|φtop,d-φnode,d|.

And the relative values between ∆φ and apparent angle Sd are assessed by ratio ∆φ/Sd, which depends on the interval length and the distance of the node from the sensor and the value of ∆φ.

The relationship between ∆φ/Sd and prediction accuracy can be described as below. If the current node is very far from LiDAR or the interval size is very small, then the apparent angle Sd is so small that the ratio ∆φ/Sd becomes much larger than 1 and the prediction of the current point has bad quality. On the other hand, if the current node is very near the LiDAR or the interval size is very large, then the apparent angle Sd is very large that the ratio ∆φ/Sd becomes much smaller than 1, and the prediction of the current point has bad quality. In between, when the ratio ∆φ/Sd is close to 1, the predictor φpred has maximum quality.

#### Determine contexts depending on predictor quality

In the described method, the relative magnitudes between ∆φ and Sd is divided into three levels by using two thresholds, each level is indicated by a value ctxSetIdx that is obtained by:

ctxSetIdx=0,   if ∆φ<23*Sd1,   if (∆φ≥23*Sd) &&(∆φ<Sd)2,   if ∆φ≥Sd,

Figure 29 depicts three levels of the relative magnitudes between ∆φ and Sd. In Figure 29, the blue line depicts the apparent angle Sd, and the purple line depicts the elementary azimuthal shift ∆φ. For case (a), ∆φ is much smaller relative to the apparent angle Sd, the ratio ∆φ/Sd<23 , so ctxSetIdx=0, and that will limit the predictor φpred  into a narrow range within current node. For case (b), ∆φ is a little smaller relative to apparent angle Sd, the ratio 23<∆φ/Sd<1 , so ctxSetIdx=1, and the prediction accuracy is the best among the three cases. For case (c), the apparent angle Sd is so small that ∆φ is larger than it, the ratio ∆φ/Sd>1 , so ctxSetIdx=2, then the predictor φpred  obtained by ∆φ provides no information for the position of the current point in current node.

Figure 29: Three levels of the relative magnitudes between ∆φ and Sd.

After the subset index ctxSetIdx is determined, angles φleft, φright and φpred are used to determine a context ctxIdx, among a subset of contexts indicated by ctxSetIdx. The proposed method uses a context table ctxTable_T to obtain the context ctx , and the table can be described as

ctx=ctxTable_T[ctxSetIdx][ ctxIdx]

## Planar Flag Signalling [21][22]

### Description of the Method

Planar coding mode was proposed to represent the occupancy information when occupied child nodes belong to the same plane. This coding mode is applied to all three directions when eligible. Specifically, three is_planar_flag[axisIdx] are introduced to indicate whether the occupied child nodes are located on the same plane perpendicular to the axisIdx-th axis. If it is equal to 1, an extra bit plane_position is then signalled to indicate the plane position.

To efficiently signal the planar flags, following modifications are made.

### Signalling One Flag Instead of Three

When current node is eligible for planar mode in all three directions, a flag xyz_planar_flag is signalled. When xyz_planar_flag = 1, it indicates that the positions of the current node's children form a single plane in all three directions.

At the encoder side:

- If current node is eligible for planar mode in all three directions, xyz_planar_flag is arithmetically encoded.

- If xyz_planar_flag equals to 1, there is no need to code three is_planar_flag[axisIdx]. In this case, coding three bits is replaced by coding one bit.

- If xyz_planar_flag equals to 0, is_planar_flag[0] and is_planar_flag[1] are coded as normal. If they are both equal to 1, there is no need to code is_planar_flag[2], since it must be equal to 0; If not, is_planar_flag[2] is coded as normal.

Correspondingly, at the decoder side:

- If current node is eligible for planar mode in all three directions, xyz_planar_flag is arithmetically decoded.

- If xyz_planar_flag equals to 1, three is_planar_flag[axisIdx] are directly set to be 1.

- If xyz_planar_flag equals to 0, is_planar_flag[0] and is_planar_flag[1] should be decoded from the bitstream as normal. If they are both equal to 1, is_planar_flag[2] is directly set to be 0; If not, is_planar_flag[2] should also be decoded from the bitstream as normal.

### Signalling One Flag Instead of Two

Similarly, when current node is eligible for planar mode in two but not all three directions, three new flags, xy_planar_flag, xz_planar_flag and yz_planar_flag are introduced.

For example, xy_planar_flag equals to 1 indicates that the positions of the current node's children form a single plane in x and y direction.

At the encoder side:

- If current node is eligible for x and y but not z planar, xy_planar_flag is arithmetically encoded.

- If xy_planar_flag equals to 1, there is no need to code is_planar_flag[0] and is_planar_flag[1] . In this case, coding two bits is replaced by coding only one bit.

- If xy_planar_flag equals to 0, is_planar_flag[0] are coded as normal. If it equal to 1, there is no need to code is_planar_flag[1] since it must be equal to 0; If not, is_planar_flag[1] is coded as normal.

Correspondingly, at the decoder side:

- If current node is eligible for x and y but not z planar, xy_planar_flag is arithmetically decoded.

- If xy_planar_flag equals to 1, is_planar_flag[0] and is_planar_flag[1] are directly set to be 1.

If xy_planar_flag equals to 0, is_planar_flag[0] should be decoded from the bitstream as normal. If is_planar_flag[0] equal to 1, is_planar_flag[1] must be equal to 0, so it directly set to be 0; If not, is_planar_flag[1] also should be decoded from the bitstream as normal.

## Disabling Planar Mode for IDCM Nodes [23][24]

In G-PCC Ed.1 the planar information is used to improve the signalling of the point positions in IDCM nodes and the occupancy of the non-IDCM nodes. The planar information is signalled first. Then, for an IDCM eligible node, the IDCM information is derived and signalled to the decoder. For an IDCM node, the position of the points in the node is encoded utilizing the planar information and the angular information. For a non-IDCM node, the occupancy of the node is encoded and the node is split into child nodes which are  added to the node buffer. The encoding process proceeds to the next node in the node buffer until the buffer is empty.

It is noted that when an IDCM node is coded as planar in an axis, one bit is saved in encoding the position of points in that direction. However, it will cost 2 bits to encode the planar mode and planar position in each direction. When the angular information is available, the signalling of a position bit is cheaper than the signalling cost of the planar information. Utilizing this observation, this method proposes to disable the planar mode for IDCM coded nodes when the angular mode is enabled. Doing so, the planar information is not signalled to the decoder for IDCM nodes and the planar information is not used in encoding the point positions when the angular mode is enabled.

| occupancy_tree_node( Lvl, NodeIdx ) { | Descriptor |
| --- | --- |
| if( occ_subtree_qp_offset_present ) { |  |
| occ_subtree_qp_offset_abs[ Ns ][ Nt ][ Nv ] | ae(v) |
| if( occ_subtree_qp_offset_abs[ Ns ][ Nt ][ Nv ] ) |  |
| occ_subtree_qp_offset_sign[ Ns ][ Nt ][ Nv ] | ae(v) |
| } |  |
| if( occtree_direct_coding_mode && DirectModePresent && geo_disable_planar_idcm_angular) |  |
| occ_direct_node | ae(v) |
| if( occtree_planar_enabled ) |  |
| for( k = 0; k < 3; k++ ) |  |
| if( PlanarEligible[ k ] ) { |  |
| occ_single_plane[ k ] | ae(v) |
| if( occ_single_plane[ k ] ) |  |
| occ_plane_pos[ k ] | ae(v) |
| } |  |
| if( occtree_direct_coding_mode && DirectModePresent && !geo_disable_planar_idcm_angular) |  |
| occ_direct_node | ae(v) |
| if( occ_direct_node ) |  |
| occupancy_tree_direct_node( ) |  |
| else { |  |
| …. |  |
| } |  |
| } |  |

## Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [25][26][27][28]

The improvement is based on a flexible way to use the neighbourhood at same depth and at child depth relative to a current node in order to code the occupancy word associated with the current node. Flexibility is provided by a reduction of the many possible neighbourhood configurations and by an OBUF-like entropy coding of the occupancy bits taking reduced neighbourhood configurations as input. An overview of the proposed method is provided in Figure 30 and Figure 31.

Figure 30: Overview of neighbour-based octree occupancy coding with dynamic OBUF (modifications in red).

The following are high-level steps for the new non-zero path:

- Obtain occupancy of neighbours

  - 26 adjacent parents

  - child neighbours

  - already coded child nodes of current node

- Combine occupancy data into words as contextual information to code bj

  - a primary word W1,j

  - a secondary word W2,j

- Code bj

  - using words W1,j and W2,j as input to dynamic OBUF

  - update tables of dynamic OBUF

By construction, it has only an effect when the neighbourhood of the current node is not empty. This is most of the time the case for dense point clouds like AR/VR point clouds in cat1-A. Concerning less dense or even sparse point clouds (on the remaining of cat1-A, cat1-B and cat3), the proposed improvement is less effective as it applies only down to some depth of the octree from which most of node neighbourhoods become empty.

Figure 31: Overview of dynamic OBUF scheme.

Further description needed

## Removal of Z-Coding Path in New Octree Coding [31]

It was observed that the Z-coding path is marginally used due to IDCM that generally prunes the octree beforehand before reaching a Z-coding path configuration. It is proposed to remove the Z-coding path and keep only the NZ-coding path, and to move the noSingleFlag information to the NZ-coding path as illustrated in Figure 32.

Figure 32: Z-coding path is removed in new octree coding scheme.

## Modified Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [32][33]

Neighbourhood analysis in octree NZ path is modified by determining the occupancy of twenty parent neighbouring nodes and using partial occupancy as illustrated in Figure 33 and Figure 34.

| (a) | (b) |
| --- | --- |

Figure 33: Modified NZ-coding path.

Figure 34: Parent occupancy bits.

Further description needed

## Common Dynamic OBUF Class for Octree and TriSoup [34]

The two dynamic OBUFs, one for octree and one for TriSoup coding are unified in this proposal with common parameters and an auto adaptive threshold in replacement of manually tuned thresholds for the number of visits in the tree.

Further description needed

## Eligibility of Planar Mode [29][30]

### Planar Mode Eligibility in G-PCC Ed.1

Eligibility of planar mode for a current node in one direction is determined as in the following description. The eligibility criterion is based on tracking the probability of past coded nodes being planar:

- eligible if and only if

p = prob(planar) >= threshold and local node density > 3

- typical threshold value is 0.6.

Updating the probability prob(planar) as follows:

- using a channel model

- with length L=255

- where δ(coded node) is 1 if the coded node is planar and 0 otherwise.

Update is performed when

- a node occupancy is (de)coded

- or/and a node planar information is (de)coded.

Tracking the local node density

- rolling average of number of occupied siblings.

When the occupancy bits are available to be coded with three planar modes x, y, z, the process is as in following description.

The simple juxtaposition of eligibility criteria does not work.

- Sub-optimal because a first activated planar mode masks 4 occupancy bits in the current node, but a second activated planar mode will then only mask 2 bits of the remaining 4, and a third one would mask only 1 bit of the remaining 2

A combined eligibility criterion that preserves symmetry:

- does not preclude that one direction is more important than the other two

- definition of “first”, “second” and “third” planar mode is based on the probabilities px, py and pz

- three thresholds th1 < th2 < th3 (typical values 0.6, 0.77, 0.88)

Eligible if local node density >3.

### Proposed Eligibility Based on Density

The method to determine a common eligibility of all the nodes in each octree layer is based on the real density. This density is calculated as follows:

realDensityi+1=numPoints-numPointsCodedByIdcminumSubnodesi

where numPoints indicates the number of points in a point cloud, numPointsCodedByIdcmi is the number of points that are coded using IDCM from root node layer to ith octree layer, numSubnodesi is the number of occupied sub-nodes generated by the nodes in ith octree layer, and realDensityi+1 is the real density of points for i+1th octree layer.

Before the coding process of i+1th octree layer, the eligibility of all the nodes to xyz-planar mode in i+1th octree layer is determined as follows:

planarEligibleKOctreeDepthi+1=1, realDensityi+1<th0,                       otherwise

where th is a predefined threshold, whose typical value is set to 1.3.

Especially, there is no more the process of determining the eligibility of planar mode when angular mode is enabled.

This method does not require the process of updating probabilities and calculating eligibility of each node to xyz-planar.

## Low-Memory Footprint Dynamic OBUF [35]

The aim is to reduce the memory footprint of dynamic OBUF for both octree and TriSoup geometry coding.

Description needed

## Initialization of Dynamic OBUF [36]

OBUF relies on a set of coders with pre-determined probabilities for each. Initialization should be done considering the desired probability range of each binary coder. However, in the current design, all the 32 adaptive arithmetic coders are initialized with the same probabilities of 0.5 for octree (and TriSoup) geometry coding, which may decelerate the probability update iteration. It is proposed to initialize the 32 arithmetic coders with initial probabilities:

static const int coder_ini_pro[32] = {

65461, 65160, 64551, 63637, 62426, 60929, 59163, 57141, 54884, 52413, 49753,

46929, 43969, 40899, 37750, 34553, 31338, 28135, 24977, 21893, 18914, 16067,

13382, 10883, 8596,  6542,  4740,  3210,  1967,  1023,  388,   75 };

## Probability Bounds in Dynamic OBUF [37][38]

In G-PCC, the design of OBUF relies on mapping geometry coding context to binary coders by a LUT-based update process. The coder whose probability is closest to the context probability is selected. While the current OBUF design expects that the probability value of symbol “0” maintained by the coder decreases monotonically with the coder index, the actual probability of the binary coder, however, does not behave so. As such, the most suitable true coder may not be selected as expected.

To solve this issue, it is proposed that the probability associated with each binary coder is limited to a specific range. The upper and lower bound of the probability range for each coder is able to be updated adaptively in OBUF. The initial boundaries for the 32 coders are set as below:

Table 1: Initial boundaries.

obufBoundOrigin [33]

65535, 65388, 64933, 64169, 63105, 61747, 60112, 58214, 56069, 53699, 51128, 48379, 45480, 42458, 39340, 36160, 32946, 29730, 26541,23413, 20374, 17454, 14681, 12083, 9684, 7509, 5575, 3905, 2515, 1419, 627,150, 0

The probability upper/lower bounds for coder i are calculated as below:

Algorithm 1 Procedure of setting upper/lower bounds of coder i’s probability

- procedure AdaptiveAdjust(Pij, Lij, Uij) - if Pij> Uij-1: - Pij ←Uij-1 // Limit the probability - Uij += ∆Uij // Adaptive adjust the Upper bound - if i > 0 && Uij> Ui-1j: // Extreme case - Uij = Ui-1j - Li+1j ← Uij // Ensure the continuity of boundaries - else if Pij < Lij-1: - Pij ← Lij-1 // Limit the probability - Lij -= ∆Lij // Adaptive adjust the Lower bound - if i < k-1 && Lij < Li+1j： // Extreme case - Lij = Li+1j - Ui-1j ← Lij // Ensure the continuity of boundaries - return Pij, Lij, Uij - end procedure

Figure 35: Probability bounds procedure.

## Improved Planar Mode [39][40]

As shown in Figure 36, the planar information for seven coded neighbors (i.e., red co-planar nodes, blue co-edge nodes, and black co-vertex node) is available for the current green node. The proposed method first introduces a variable neighAvailable to represent whether these neighbors are available. If there is at least one available neighbor, neighAvailable is true, otherwise, it is false.

- When neighAvailable is true:

The proposed method uses planar information of available coded neighbors to code the planePosition flag of the current node by designing new contexts.

Figure 36: Current node neighbors with available planar information.

First, three co-planar neighbors’ planar information, including isPlanar[N] and planePosition[N], is obtained during the encoding/decoding process, where N is an index (0,1, or 2) to represent the co-left planar neighbor, co-front planar neighbor, and co-below planar neighbor, respectively. The first variable coPlanarCtx is initialized as zero and updated by using the formula:

for(neighIdx=0;neighIdx<3;++neighIdx) {

coPlanarCtx|=(planePosition[neighIdx]<<3)|isPlanar[neighIdx];

coPlanarCtx<<=2;

}

Similarly, three-co-edge neighbors’ planar information and the co-vertex neighbor’s planar information are used to calculate the second variable coEdgeVerCtx which is initialized as zero and updated by using the formula:

for(neighIdx=0;neighIdx<4;++neighIdx) {

coEdgeVerCtx |=(planePosition[neighIdx]<<4)|isPlanar[neighIdx];

coEdgeVerCtx <<=2;

}

The above contexts for coding the planePosition flag are categorized as primary and secondary information as follows.

- Primary information

  - The distance from the closest coded node at the same coordinate and depth is discretized into two values: “near” or “far”.

  - The plane position (if any) of the closest coded node at the same coordinate and depth: low or high.

  - The coPlanarCtx of the current node predicted by the three co-planar neighbors’ planar information.

- Secondary information

- The planePosition of the current node is predicted by the occupancy information of its neighbors. (three values: predicted as low, high, or unknown)

- The coEdgeVerCtx of the current node predicted by the three co-edge and co-vertex neighbors’ planar information.

- When neighAvailable is false:

This report uses the occupancy information of 12 neighbors as shown in Figure 37, to code the current node’s planePosition flag by designing new contexts.

Figure 37: Current node neighbors with occupancy information.

First, three co-planar neighbors’ occupancy information is used to derive the variable coPlanarOcc. Let the occupanied[N] represents whether the neighbor is occupied or not, where N is an index (0,1, or 2) to represent the co-right planar neighbor, co-below planar neighbor, and co-upper planar neighbor. The variable coPlanarOcc is initialized as zero and updated by using the formula:

for(neighIdx=0;neighIdx<3;++neighIdx) {

coPlanarOcc |= occupanied [neighIdx];

coPlanarOcc<<=1;

}

Similarly, the nine-co-edge neighbor’s position relative to the current node is shown in Table 2.

Table 2: Adjacent co-edge neighbors relative to the current node.

| neighIdx | x | y | z |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 2 | 1 | 0 | -1 |
| 3 | 1 | -1 | 0 |
| 4 | 0 | 1 | 1 |
| 5 | 0 | 1 | -1 |
| 6 | 0 | -1 | 1 |
| 7 | -1 | 1 | 0 |
| 8 | -1 | 0 | 1 |

The variable CoEdgeOcc is initialized as zero and updated by using the formula:

for(neighIdx=0;neighIdx<9;++neighIdx) {

CoEdgeOcc |= occupanied [neighIdx];

CoEdgeOcc <<=1;

}

The above contexts for coding the planePosition flag are categorized as primary information and secondary information as follows.

- Primary information:

  - The distance from the closest coded node at the same coordinate and depth is discretized into two values: “near” or “far”.

  - The plane position (if any) of the closest coded node at the same coordinate and depth: low or high.

  - The planePosition of the current node is predicted by the occupancy information of its neighbors. (three values: predicted as low, high, or unknown).

  - The coPlanarOcc of the current node predicted by the three co-planar neighbors’ occupancy information.

- Secondary information:

- The CoEdgeOcc of the current node predicted by the nine co-edge neighbors’ occupancy information.

The Dynamic OBUF uses the above information to process and the final number of contexts for coding planePosition flag is 16 for each direction. As a result, there are a total of 48 contexts for coding planePosition while the number of original contexts is 117.

## Improvements of Octree-Coding for Spin-LIDAR Sequences [41][42]

### Introduction

For spinning-LIDAR content, the number of lasers is fixed, e.g., Ford and QNX sequences have 64 and 16 lasers, respectively. Each laser has a fixed elevation angle. Each point in spinning-LIDAR point cloud should be on the trajectory of its hitting laser. In other words, each point should have the same elevation angle as its hitting laser. However, the geometry quantization will change the point elevation angle when the lossy geometry compression is performed. At the same time, the elevation angle of each laser has been signalled in GPS. Hence, it is proposed to compensate z-coordinates using the elevation angles information to reduce the geometry distortion in the case of lossy geometry compression. The compensation is performed by aligning its elevation angle with the elevation angle of its hitting laser.

The z-compensation is performed after attribute coding as shown in Figure 38.

Decoding geometry

Compensating z

Decoding attribute

Bitstream

Reconstructed point cloud

Encoding geometry

Input point cloud

Bitstream

Encoding attribute

Figure 38: Compensating z after attribute coding.

One flag named geom_z_compensation_enabled_flag is used to indicate whether z compensation is enabled.

### Z-Coordinates Compensation Process

Firstly, for each decoded point, its hitting laser is derived by finding the point’s nearest elevation angle from all lasers’ elevation angles. Secondly, as shown in Figure 39, a compensated z-coordinate zc of this point x,y,z is computed according to the elevation angle of its hitting laser as follows:

zc=round (tanθLx2+y2-GQS×zL)

where tanθL is the tangent value of its hitting laser’s elevation angle, GQS is the geometry quantization step, zL is the quantized laser position of its hitting laser, and round () is the function that finds the nearest integral value. Finally, the compensated z-coordinate zc will directly replace the decoded z.

laser

Z

X

z

zc

θL

Y

Figure 39: Compensating z-coordinate according to the elevation angle of its hitting laser.

Not all decoded points will be compensated. Only the points which satisfy the following both conditions will be compensated:

GQSx2+y2+z2<tan∆θ2and abs(z-zc)<GQS

where ∆θ is the minimum difference between adjacent lasers’ elevation angles, abs() is absolute value function.

laser

∆θ2

laser

Found right hitting laser

point

Found wrong hitting laser

Figure 40: The first condition.

The z-compensation is implemented using a division-free fixed-point form.

# Inter Prediction for Octree Geometry Coding

## Inter Prediction for Octree Geometry Coding Based on Global Motion Using Two-Threshold Classification of Road/Object Points [44][45][50][52]

In point cloud data captured by LiDAR sensors attached to moving vehicles, the road and objects commonly have different motions. Since the distance between the road and LiDAR sensor is relatively constant and the road undergoes minor changes from one vehicle position to the next, the movement of the points representing the road with respect to the LiDAR sensor position is small. In contrast, objects like buildings, road signs, vegetation, or other vehicles have larger movements, which is different from the road movement as illustrated in Figure 41.

In the initial interEM-v1 software [46] that was studied in EE13.2, the points are selected for estimation of the global motion without a classification into road and objects. Since the road and object points have different motions, a division of a cloud into road and objects will improve the accuracy of the global motion estimation and compensation, resulting in improved compression efficiency [44][45].

Figure 41: Two consecutive frames of Ford_02_q_1mm (Frame 200 (green) and Frame 201 (red)) illustrating that road point movements are typically smaller than object point movements.

Objects are typically situated above the road. However, in some point clouds, e.g., the point clouds that are acquired when the vehicle is moving across a bridge or up a road ramp, the objects may also exist below the road. Based on this observation, the proposed method utilizes a classification of points into road and objects based on the height of the points (e.g., z-coordinate). The height of the points is compared with two thresholds, bottom_thr and top_thr, with bottom_thr smaller than top_thr. A point with height that is in between both thresholds is classified as a road point, otherwise, as a point belonging to objects.

One example of a method to estimate these thresholds, which is based on computing the standard deviation of the histogram of the height of the points, is described in [44][45]. It is a non-normative method that may be implemented in the encoder, or alternatively, the thresholds are obtained externally and provided as parameter inputs to the encoder. Figure 42 illustrates the estimation of the thresholds using the histogram method and Figure 43 illustrates the classification of a LiDAR point cloud into road and object points.

Figure 42: Example of the derivation of the thresholds based on the histogram, Ford_01_q1mm, frame 200.

Figure 43: Result of classification, Ford_01_q1mm, frame 200.

After the points in the cloud are classified into road and object points, only the object points are considered in the global motion estimation process in the encoder. The global motion parameters consist of a translation vector and rotation matrix. In the TMC13 software, the non-normative LMS parameter estimator that was initially part of InterEM-v1 is integrated with speedups [50]. In addition, the non-normative internal ICP estimator is supported [52]. Alternatively, the global motion parameters can be externally estimated (e.g., ICP method, or based on Global Positioning System data) and provided as inputs to the encoder (TMC13).

Global motion is enabled with a flag in the geometry parameter set (GPS). If enabled, global motion parameters and the two thresholds are signalled.

In the inter prediction process, global motion compensation is applied to a previously reconstructed point cloud frame, for example, the previous frame. Similarly, as in the estimation process, the global motion compensation process classifies the points into road and object points based on the two thresholds that are signalled in the bitstream (GPS) together with the global motion parameters. Subsequently, the global motion is applied to the object points only, while the road points are predicted as static (zero motion).

The reference frame is used in occupancy coding of the current frame’s nodes. The reference node is the collocated node in the reference frame, which is derived by compensating the prediction frame using global motion (if enabled). When the reference node is obtained, it is split into 8 child nodes of equal size. Points in each child node of the reference node are counted to form the inter prediction occupancy (preOcc) variable. If there is at least one point in a child node, the corresponding bit (out of 8 bits) in preOcc is set equal to 1, otherwise, it is set equal to 0.

The quality of the inter prediction is then evaluated by a parameter called ‘occupancyIsPredictable’. occupancyIsPredictable of a node is derived by the number of siblings missed predictions. The number of siblings missed predictions (numSiblingsMispredicted) is calculated by comparing the occupancy of the parent node with the occupancy of the parent’s reference node as illustrated in Figure 44. If the predOccupancy of the current node is 0 or the number of siblings missed predictions is larger than 5, occupancyIsPredictable is set equal to 0, otherwise, it is set equal to 1.

Figure 44: The occupancies of parent node and reference node of parent node are compared to obtain numSiblingsMispredicted. If the occupancy bit of a child node in the parent node and the occupancy bit of the corresponding node in the reference node are different, this child is considered as missed prediction.

occupancyIsPredictable is then used to update preOcc, planar copy mode eligibility (see section 5.2) and IDCM eligibility. If occupancyIsPredictable is equal to 0, preOcc, and planar copy mode (see next section) eligibility are set equal to 0.

The section 5.4 provides further details about inter modifications to octree occupancy coding in inter slices.

## Inter Planar Mode Coding [51]

The following method includes several techniques to utilize inter prediction in the coding of planar mode, plane position, and occupancy when the planar mode is enabled. The method is integrated in TMC13 for inter slice.

### Modified Planar Mode Coding with Inter Prediction

The planar information of the collocated reference node (named as reference node) in the motion compensated cloud is utilized. The occupancy of the reference node is derived first. Then this occupancy is used to generate the planar mode (planar_ref) and plane position (plane_ref) of the reference node. Finally, this planar information is updated using the planar eligibility of the current node. Specifically, if a direction in the current node is not eligible, the planar mode in the corresponding direction in the reference node is set equal to 0.

Let planarMode_ref[axisIdx] be the planar mode in axisIdx axis direction in the reference node.

Let plane_ref[axisIdx] be the plane position in axisIdx axis direction in the reference block. If planarMode_ref[axisIdx] is equal to 0, plane_ref[axisIdx] is set equal to -1.

Let planarEligible[axisIdx] be the planar eligibility in axisIdx axis direction in the current node.

Let planarMode[axisIdx] be the planar mode in axisIdx axis direction in the current node.

Let plane[axisIdx] be the plane position in axisIdx axis direction in the current node.

### Context Selection for Planar Flag and Plane Position Coding Using Inter Prediction

Context selection for planar flag:

In the TMC13 version 12 and the G-PCC Ed.1 specification text, the index of the context for encoding the is_planar_flag is set equal to the axis index axisIdx. In the presented method, this index is extended using the plane position of the reference block and it is set equal to: (3*axisIdx + (plane_ref [ axisIdx ] + 1)).

Context selection for plane position coding (angular mode disabled):

In G-PCC Ed.1 the context for coding the plane position of the axisIdx axis is given as:

if (!IsPlanarNode[axisIdx][axisPos]])

ctxIdx = adjPlaneCtxInc

else {

prevPlane = PlanarPlane[axisIdx][axisPos]

distCtxInc = (dist > 1)

ctxIdx = 12 × axisIdx + 4 × adjPlaneCtxInc + 2 × distCtxInc + prevPlane + 3

}.

Where adjPlaneCtxInc, distCtxInc, prevPlane are used in the context derivation based on neighbor information and the planar information in the buffer. In the presented method, the plane position in the inter reference block is utilized as follows:

if (!IsPlanarNode[axisIdx][axisPos]])

ctxIdx = adjPlaneCtxInc + 3 * (plane_ref [ axisIdx] + 1)

else {

prevPlane = PlanarPlane[axisIdx][axisPos]

distCtxInc = (dist > 1)

ctxIdx = (12 × axisIdx + 4 × adjPlaneCtxInc + 2 × distCtxInc + prevPlane + 3) + 36*(plane_ref [ axisIdx] + 1)

}

Context selection for plane position coding (angular enabled):

For angular mode, the plane position in the inter reference block is utilized as follows to derive the context index:

contextAngular += 4 x (RefPlane[axisIdx] + 1)

And for azimuthal mode, the plane position in the inter reference block is utilized as follows to derive the context index:

if (Abs(sPos) <= Abs(tPos))

contextAzimuthalS = contextAnglePhi + 8 x (RefPlane [axisIdx] + 1)

else

contextAzimuthalT = contextAnglePhi + 8 x (RefPlane [axisIdx] + 1)

### Planar Copy Mode

In inter slice, a node may be coded as a planar copy mode (PCM) node if the current node and the reference node have the same planar mode in all the axis directions that are planar eligible. A flag named pcm_flag is signalled in the bitstream to indicate whether it is PCM or not. If a node is PCM, the planar flag and plane position are not signalled in all axis directions, and they are copied from the reference node in the decoder.

The PCM mode decision of a node may be presented as follows:

The current node and the reference node are matching along the k-th axis if planarEligible[k] of the current node equals to false, or at this axis the current node and the reference node share the same planar mode and plane position:

match_dir[k] = !planarEligible[k] || (planarMode[k] == planar_ref[k] && plane[k] == plane_Ref[k])

pcm_flag =  match_dir[0] && match_dir[1] && match_dir[2]

PCM mode restriction:

PCM is restricted to nodes having at least one planar eligible direction.

PCM is restricted to the node when occupancyIsPredictable is equal to 1.

Signalling of the pcm_flag:

The PCM flag is context coded and signalled in the slice if planar mode is enabled. The context index is derived using the planar mode of the reference node, the planar information in the buffer and the planar eligibility of the current node as follows:

ctxIdx_PCM = 8 * (3 * (planarEligible[0] + planarEligible[1] + planarEligible[2] - 1)    + ref2buff_match[0] + ref2buff_match[1] + ref2buff_match[2])

+ planar_ref[2] + 2 * planar_ref[1] + 4 * planar_ref[0].

ref2buff_match represents the planar comparison between the reference node and the planar information in the buffer. This value is derived for the k-th axis as follows:

- If all below conditions are satisfied, ref2buff_match[k] is set equal to true:

- planarEligible[k] is true.

- The buffer is not empty.

- IsPlanarNode (planar node in the buffer) is equal to the planarMode_ref [k].

- PlanarPlane[k] (plane position in buffer) is equal to plane_Ref[k].

- Otherwise, ref2buff_match[k] is set equal to false.

Avoid redundancy in signalling of the planar position for non-PCM node:

For a non-PCM node, signalling of the plane position of the last eligible planar axis may be ignored. For example, if all the following conditions are satisfied, the signalling of the plane position plane[k] of the last eligible planar axis (let this direction be k-th direction) may be skipped:

- Current node and reference node are planar matching in all previous axis directions.

- The planar mode of the k-th axis in the reference node planar_Ref[k] is true.

When all above conditions are satisfied, the plane position is implicitly derived using the plane position in the reference frame in the corresponding last axis as follows:

plane[k] =  plane_Ref[k] == 1 ? 0 : 1

### Use Inter Prediction in Occupancy When the Node Has a Planar Direction

Signalling of the single_occupancy_flag based on the inter prediction and planar masks:

In this technique, the single_occupancy_flag is signalled conditionally when the inter prediction is non-zero (inter_pred_occ is occupancy of collocated reference node) or the node has non-zero mask in at least one direction as follows:

The variable OccupancyIdxMaybePresent specifies when single_occupancy_flag is present in the octree node syntax. let numPlanarAxes = IsPlanar[0] + IsPlanar[1] + IsPlanar[2] let numNonPlanarAxes = IsNotPlanar[0] + IsNotPlanar[1] + IsNotPlanar[2] OccupancyIdxMaybePresent = numPlanarAxes < 3 ¦¦ (!NeighbourPattern && (!inter_pred_Occ ¦¦ (mask_planar[0] ¦ mask_planar[1] ¦ mask_planar[2]) ) && !numNonPlanarAxes)

## IDCM Eligibility for Inter [60]

For an intra frame (angular mode enabled and disabled) or for an inter frame with angular mode enabled, the IDCM eligibility is not modified. For an inter frame and if angular mode is disabled, the IDCM eligibility is as follows:

if (!IDCM_intensity)

return false;

if (occupancyIsPredictable)

return false;

return (nodeSizeLog2 >= 2) && (nodeNeighPattern == 0)

&& (child.numSiblingsPlus1 == 1) && (node.numSiblingsPlus1 <= 2);

Note: (child.numSiblingsPlus1 == 1) means child node has no sibling. (node.numSiblingsPlus1 <= 2) means the parent node has at most 2 siblings.

## Inter Modification for Neighbour-Based Octree Occupancy Coding with Dynamic OBUF [60][61]

The presented inter modification extends the neighbour-based octree occupancy coding with dynamic OBUF described in section 4.6.

The occupancy of the collocated node in the motion compensated frame is denoted as preOcc which includes 8 bits. The i-th bit in preOcc represents the occupancy bit of i-th child node in the reference node. Let pi be the collocated reference child node of i-th child node in the current node. In this method, the occupancy bit of the i-th child node in reference node is used to select the context to encode the occupancy bit of i-th child node in the current node.

As an example, the following code segment is modified (green highlights) when the neighbour configuration is nonzero (function decodeOccupancyFullNeihbourgsNZ()):

{

…

// encode

int bit = (occupancy >> i) & 1;  // i is the position index of the child node to be encoded

int interCtx = (predOcc >> i) & 1; // the inter context based using the occupancy of the ith child in the reference node.

if (Sparse[i]) {

ctx2 |= (Word7Adj[i] & 31) << 6;

int ctx1 = ((Word7Adj[i] >> 5) << i) | partialOccupancy;

_arithmeticEncoder->encode(bit, _ctxMapOccupancy[_MapOccupancySparse[interCtx][i].getEvolve(bit, ctx2, ctx1)]);

} else {

ctx2 |= (Word7Adj[i] & 7) << 6;

int ctx1 = ((Word7Adj[i] >> 3) << i) | partialOccupancy;

_arithmeticEncoder->encode(bit, _ctxMapOccupancy[_MapOccupancy[interCtx][i].getEvolve(bit, ctx2, ctx1)]);

}

…

}

In another example, the following code segment is modified (green highlights) when the neighbour configuration is zero (function decodeOccupancyNeighZsimple()):

{

…

int bit = 1;

if (!(bitIsOneX || bitIsOneY || bitIsOneZ)) {

int bitPred = (predOcc >> i) & 1;

int interCtx = bitPred;

bit = _arithmeticDecoder->decode(_ctxZ[i][numOccupiedAcc][interCtx]);

coded0[mask0X] += !bit;

coded0[mask0Y] += !bit;

coded0[mask0Z] += !bit;

}

…

}

The map occupancy is defined as follows:

CtxMapOccupancy _MapOccupancy[2][8];// 2: reference child node is occupied or not, 8: bit position

CtxMapOccupancy _MapOccupancySparse[2][8];

Further description needed

## Unified Partitioning Method for Motion Compensation [62]

Three-dimensional cuboid partitioning is proposed and can be enabled with a flag that is signalled in the geometry data unit. The cuboid block size is signalled per dimension. As a special case, for horizontal partitioning {0, 0, block height} is used with 0 value indicating that the current slice bounding box in the corresponding dimension is used. Conversely, for cubic partitioning {octree node size=s, s, s} can be applied. Per block a flag is signalled to indicate whether global motion is applied to the block or not. If cuboid partitioning is disabled, then two-threshold road/object segmentation is enabled.

## Bi-Directional Inter Prediction [64][65]

In [65], it is proposed to use a hierarchical GOF structure to perform inter prediction for geometry coding and attribute coding. In the hierarchical GOF structure, the first frame in each GOF is an I-frame or a P-frame. The other frames in the GOF are B-frames, which use two reference frames from the forward and backward directions.

As shown in Figure 45, frame “0” ~ “7” are the frames in one hierarchical GOF and frame “8” is the first frame of the next GOF.

Figure 45: Example of hierarchical reference frame relationships within one hierarchical GOF.

For frames “0” ~ “8” in hierarchical GOF structure, the reference frames are shown in Table 3.

Table 3: Reference frame for each frame in one hierarchical GOF.

| Frame time stamp | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference frame time stamp | None | 0, 2 | 0, 4 | 2, 4 | 0, 8 | 4, 6 | 4, 8 | 6, 8 | 0 |

In [64], it is proposed to use one IBBP GOF structure to perform inter prediction for geometry coding and attribute coding. In the IBBP GOF structure, the first frame in each GOF is an I-frame or a P-frame. The other frames in the GOF are B-frames, which use two reference frames from the forward and backward directions.

As shown in Figure 46, frame “0” ~ “7” are the frames in one IBBP GOF and frame “8” is the first frame of the next GOF.

Figure 46: Example of reference frame relationships within one IBBP GOF.

For frames “0” ~ “8” in IBBP GOF structure, the reference frames are shown in Table 4.

Table 4: Reference frame for each frame in one IBBP GOF.

| Frame time stamp | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference frame time stamp | None | 0, 8 | 1, 8 | 2, 8 | 3, 8 | 4, 8 | 5, 8 | 6, 8 | 0 |

For geometry coding, the prediction direction of the child nodes of the current node are derived based on the relationship between the occupancy codes of the current node and those of the reference nodes as illustrated in Figure 47.

Figure 47: Example of deriving the prediction direction of child nodes.

Figure 47 shows an example of how to derive the prediction direction. For a child node of the current node with the occupancy flag equal to 1, the corresponding occupancy flags of the previous reference frame and the following reference frame are denoted as bit_pre and bit_follow, respectively. Subsequently, the prediction direction of the current node is derived following these rules:

- If bit_pre = 1 and bit_follow = 0, the prediction direction of the child node is set to using the previous reference (forward) node to perform inter prediction.

- If bit_pre = 0 and bit_follow = 1, the prediction direction of the child node is set to using the following reference (backward) node to perform inter prediction.

- If bit_pre = bit_follow = 1, the numbers of the mismatched bits between the occupancy code of current node and the occupancy codes of the reference nodes are calculated. If the numbers of the mismatched bits are different, the prediction direction of the child node is set to the prediction direction with less mismatched number. Otherwise, the prediction direction of the child node is set to the prediction direction of the parent node.

A FrameMerge mode is implemented where the two reference frames can be merged into one new reference frame to perform inter prediction for geometry coding. In the FrameMerge mode, the new reference frame contains all points of the two reference frames after the motion compensation. The new reference frame is used as the only one reference frame of the inter prediction for octree geometry coding. The derivation of prediction direction is disabled in FrameMerge mode as there is only one reference frame.

It is proposed to use global motion information to decide the GOF structure for each GOF. The global motion information between the first frame of the current GOF and the first frame of the next GOF is derived. The rotation degrees (Rx, Ry, Rz) and translation vector (Sx, Sy, Sz) are calculated based on the motion information. If the motion information meets two conditions, the Hierarchical GOF structure or IBBP GOF structure is applied to the current GOF:

(1) All of the rotation degrees are less than thr1:

thr1=0.05

(2) The translation vector is less than thr2:

thr2=0.001*slice_size

Otherwise, the IPPP GOF structure is applied to the current GOF.

## Inter IDCM [71][72]

The current IDCM coding only uses geometry information within the current frame. However, points from previously decoded frames may provide additional prediction information. Based on the IPPP structure, this contribution proposes a method to improve the IDCM coding by further considering the points from the previously coded frame. Based on the inter-octree-based geometry coding information, the inter-geometry information was utilized to code the DCM mode (DCMFlag), point number (numPoints), and the laser index (pointLaserIdx) of the points for the current node.

### Inter Eligibility

In addition to intra-based IDCM eligibility, when inter-prediction is enabled for the current frame, this contribution proposes to add an inter-based IDCM eligibility. More specifically, a prediction node for the current node is selected from the reference frame. As shown in Figure 48, the green node is in the current frame, and the yellow node is the prediction node for the green node in the reference frame. The following criterion is used to determine if this prediction node is an IDCM node as follows:

- kTwoPoints: The node at most has th points, e.g. th as 2, and these th points do not include the duplicate points.

- kAllPointsSame: The node has N points where N is not less than 1 and all of these N points are duplicate points.

Let a binary interEligible represent inter-eligibility is true or not. interEligible is true if any of the above two conditions are satisfied and interEligible is false if the above two conditions are both not satisfied. Let a binary IDCM_eligible represent IDCM eligibility for the current node. The IDCM_eligible is calculated as follows:

IDCM_eligible=intraEligible ||  interEligible

Figure 48: Overview of Inter IDCM.

### Inter-prediction IDCM Mode Coding

Depending on the value of IDCM_eligible it is determined whether the current node is eligible as IDCM or not. If the current node is not eligible as IDCM, octree coding is applied. If the current node is eligible to code as IDCM, a binary flag (DCMFlag) is coded to signal if IDCM is applied.

When the current node (blue node) is eligible to be coded as IDCM, a prediction node (red node) in the reference frame is determined. Then, the IDCM mode of the prediction node (InterDCM_Mode) is obtained. InterDCM_Mode is used to determine a context for coding current node’s DCM mode (DCMFlag) as follows:

- The DCM mode of the prediction node (InterDCM_Mode). (three values: not-DCM, kTwoPoints, and kAllPointsSame)

### Inter-prediction IDCM numPoints Coding

If an IDCM is applied, the number of points in this IDCM node is binarized by using a truncated unary code. A flag (NumPointsGt1) is coded with a context to indicate if the number of points in the current node is larger than one. This contribution proposes to add another context based on the number of points (numPredPoints) in the prediction node to code NumPointsGt1. The added context is determined as follows.

context=numPredPoints>1

### Inter-prediction IDCM resLaser Coding

For the LiDAR-acquired point cloud, IDCM coding will use the Lidar-acquired parameters to improve the IDCM coding efficiency (likely angular mode and azimuthal coding). For each IDCM node, a laser index of the DCM node (nodeLaserIdx) is calculated first based on the node’s position. And then, an index for each point (pointLaserIdx) in the IDCM node is calculated, and the difference (resLaser) between pointLaserIdx and nodeLaserIdx is coded in the bitstream:

resLaser=pointLaserIdx-nodeLaserIdx

Figure 49: Overview of the Laser index coding.

When inter-prediction is enabled, a prediction point in the prediction node for the current node is determined. A laser index (predLaserIdx) of this prediction point is then obtained. The laser index of the current DCM node is updated by using the following formula:

resLaser=pointLaserIdx-predLaserIdx

Figure 50: Overview of the inter Laser index coding.

## Improved Inter Predictive Information for Occupancy Tree [77] (GeS-TM)

Occupancy tree is a geometry coding scheme in GeS-TM that may be used as a single geometry coding scheme, targeting lossless use cases, or may be used as an underlying tree for TriSoup, targeting lossy use cases. Inter prediction has been plugged to the occupancy tree by leveraging a local motion compensated reference frame.

A contextual information is constructed based on inter predictive information and is input to OBUF. Occupancy bits representing the tree are coded using an entropy coder selected by OBUF. Contribution m65916 has shown how to improve the construction of the contextual information based on inter predictive information to obtain better compression of the inter scheme.

Description requested

## On Inter Context Design of Octree Coding [78] (GeS-TM)

In the current design of GeS-TM, the integration of local motion significantly improves the inter-frame encoding efficiency when compressing dynamic dense point clouds. Specifically, temporal information is inserted into the dynamic OBUF to better guide the entropy coding of octree and Trisoup syntax. Figure 51 illustrates the octree occupancy coding method in GesTM.

Figure 51: The octree occupancy coding method in GeS-TMv5.0.

In the proposed method evaluated in this test, the inter and intra contexts are combined in a different way as shown in Figure 52.

Figure 52: Flowchart of the proposed method.

1. 1-bit of inter-frame information is incorporated, which indicates whether the current node is occupied or not. Consequently, the number of inter states is reduced from 221 to 220 for each sub-node.

2.Send the four groups of state to six coder groups according to the inter-frame information (i.e., Pred, PredL and PredLL).

# Inter Prediction for Predictive Geometry Coding

## Inter Prediction for Predictive Geometry Coding [47][48][49]

Predictive geometry coding uses a prediction tree structure to predict the positions of the points. When angular coding is enabled, the x, y, z coordinates are transformed to radius, azimuth and laserID and residuals are signalled in these three coordinates as well as in the x, y, z dimensions. The intra prediction used for radius, azimuth and laserID may be one of four modes and the predictors are the nodes that are classified as parent, grand-parent and great-grandparent in the prediction tree with respect to the current node. The predictive geometry coding, as currently designed in G-PCC Ed.1, is an intra coding tool as it only uses points in the same frame for prediction. Additionally, using points from previously decoded frames may provide a better prediction and thus better compression performance.

For inter prediction it was initially proposed in [47][48] to predict the radius of a point from a reference frame. For each point in the prediction tree, it is determined whether the point is inter predicted or intra predicted (indicated by a flag). When intra predicted, the intra prediction modes of predictive geometry coding are used. When inter-prediction is used, the azimuth and laserID are still predicted with intra prediction, while the radius is predicted from the point in the reference frame that has the same laserID as the current point and an azimuth that is closest to the current azimuth. A further improvement of this method in [49] also enables inter prediction of the azimuth and laserID in addition to radius prediction. When inter-coding is applied, the radius, azimuth and laserID of the current point are predicted based on a point that is near the azimuth position of a previously decoded point in the reference frame. In addition, separate sets of contexts are used for inter and intra prediction.

The method in [49] is illustrated in Figure 53. The extension of inter prediction to azimuth, radius, and laserID consists of the following steps:

- For a given point, choose the previous decoded point (prevDecP0).

- Choose position in reference frame (refFrameP0) that has same scaled azimuth and laserID as prevDecP0.

- In reference point cloud frame, find the first point (interPredPt) that has azimuth greater than that of refFrameP0.

Figure 54 illustrates the decoding flow associated with the “inter_flag” that is signalled for every point. The method is available in InterEM-v3.0.

Figure 53: Example of inter-prediction of current point (curPoint) from a point (interPredPt) in the reference frame.

Figure 54: Decoder flowchart.

## Additional Predictor Candidate [55]

In the inter prediction method for predictive geometry described in section 6.1, the radius, azimuth and laserID of the current point are predicted based on a point that is near the collocated azimuth position in the reference frame when inter coding is applied using the following steps:

- for a given point, choose the previous decode point,

- choose a position in the reference frame that has the same scaled azimuth and laserID as a),

- choose a position in the reference frame from the first point that has azimuth greater than the position in b), to be used as the inter predictor point.

Figure 55: Additional inter predictor point obtained from the first point that has azimuth greater than the inter predictor point.

The present method adds an additional inter predictor point that is obtained by finding the first point that has azimuth greater than the inter predictor point in c) as shown in Figure 55. Additional signalling is used to indicate which of the predictors is selected if inter coding has been applied.

## Improved Inter Prediction Flag Coding [56]

An improved context selection algorithm is applied for coding the inter prediction flag. The inter prediction flag values of the five previously coded points are used to select the context of the inter prediction flag in predictive geometry coding.

## Integration with Improved Predictive Geometry Coding and Global Motion [59]

### Integration with Improved Predictive Geometry Coding

The following adaptations were implemented to integrate inter predictive geometry coding with the improved predictive geometry coding tools described in section 0.

- Context switching based on interFlag to integrate with improved radius and phi residual signalling.

- Context switching based on whether previous node is inter predicted to integrate with contexts switching based on qphi and predIdx of previous node.

- Use radius residual difference (“new object”) only for intra predictor, not for inter predictor.

### Integration with Global Motion

When global motion compensation is applied, the azimuth position of the points are modified depending on the motion parameters. Therefore, resampling is needed to align the azimuth points before and after compensation as illustrated in Figure 56.

Figure 56: Azimuth resampling of motion compensated reference.

## Integer Implementation of Cartesian to Spherical Coordinate Conversion for Global Motion Compensation [63]

The motion compensation module for inter prediction in predictive geometry coding includes a step to convert from the cartesian coordinates to the spherical coordinates that uses floating point operations. G-PCC Ed.1 uses a fixed-point/integer implementation convertXyzToRpl to convert from cartesian to spherical coordinates in the attribute coding process. However, the convertXyzToRpl method is not directly applicable for geometry as it uses a different scaling factor and offset. It is proposed to apply an alternative fixed-point implementation similar to convertXyzToRpl by adjusting the scaling factor and offset such that the derived azimuth is in the same scale factor as the coded azimuth.

Given a position (x,y,z) in cartesian coordinate system, the corresponding radius and azimuthal angle are calculated (floating point implementation) as follows:

int64_t r0 = int64_t(std::round(hypot(xyz[0], xyz[1])));

auto phi0 = std::round((atan2(xyz[1], xyz[0]) / (2.0 * M_PI)) * scalePhi);

where, scalePhi is modified for different rate points in the lossy configuration; a maximum value of 24 bits is used for azimuth angle when coding the geometry losslessly. The fixed-point implementation of derivation of radius is the same as in convertXyzToRpl method, whereas fixed-point implementation of azimuth derivation was updated as follows:

Radius:

| Floating implementation | int64_t r0 = int64_t(std::round(hypot(xyz[0], xyz[1]))); |
| --- | --- |
| Fixed point implementation (same as in convertXyzToRpl) | int64_t xLaser = xyz[0] << 8; int64_t yLaser = xyz[1] << 8; int64_t r0 = isqrt(xLaser * xLaser + yLaser * yLaser) >> 8; |

Azimuth:

| Floating implementation | auto phi0 = std::round((atan2(xyz[1], xyz[0]) / (2.0 * M_PI)) * scalePhi); |
| --- | --- |
| Fixed point implementation (in convertXyzToRpl) | (*dst)[1] = (iatan2(yLaser, xLaser) + 3294199) >> 8; |
| Fixed point implementation (Proposed) | int64_t tmp = iatan2(yLaser, xLaser); int64_t invPi = std::round(((1 << 24) + 0.) / M_PI); // in the final code, the value of invPi is hard-coded int sh = 44 - (azimLog2 - 1); int off = 1 << (sh - 1); auto phi0 = (((tmp + 3294199) * invPi + off) >> sh) - (1 << (azimLog2 - 1)); // invPi = 5340354 |

The value of azimLog2 is the number of bits used to code the azimuth (in the CTC, it is 24 bits for the lossless case, and it varies for the lossy case).

## Zero Motion and Global Motion Inter Predictors [66][67][68]

It is proposed to have four candidate inter predictor points, two from a non-motion compensated reference frame (zero motion) and two from a global motion compensated (GMC) reference, as shown in Figure 57, as an extension of the inter prediction for predictive geometry angular coding method. It is asserted that this set of inter predictor points are better able to exploit the global motion information for prediction of points belonging to static objects of little local motion in a non-static scene. The radius of the inter predictor is obtained from the inter predictor points while the azimuth is obtained from the parent node of the current point to be predicted.

Figure 57: Additional inter predictors from global motion compensated reference frame.

The Reference Node Signalling Flag is replaced with Reference Node Index ‘refNodeIdx’ to cater to the signalling of the additional inter predictors. The value of ‘refNodeIdx’ is also used to influence context used for coding of Residual, ResPhi, ResR and Phi Multiplier.

In [68] a flag is proposed that specifies whether resampling/interpolation is applied to the global motion compensated reference. Secondly, when global motion is disabled, it is proposed to only allow two inter prediction candidates (excluding the global compensated reference frame candidates) and in this case only one bit predictor index is signalled.

## Additional Inter Predictors [69]

### Modified Estimation of qphi

Originally when inter predictors from GMC Reference Frame are used, their azimuth is obtained from the parent node of the current point to be predicted. It is proposed to first find the difference between the azimuth obtained from the inter predictor with that from the parent node of the current point to be predicted. Given the angular azimuth speed, qphi can be better estimated resulting in better azimuth prediction of GMC inter predictors. The changes made to the source code are as follows:

if (

#if M61586_IMPROVE_B

!movingState &&

#endif

refNodeIdx > 1)

#if M61586_IMPROVE_A

{

auto deltaPhi = pred[1] - parentPos[1];

#endif

pred[1] = parentPos[1];

#if M61586_IMPROVE_A

if (

deltaPhi >= (_geomAngularAzimuthSpeed >> 1)

|| deltaPhi <= -(_geomAngularAzimuthSpeed >> 1)) {

int qphi0 = divApprox(

int64_t(deltaPhi) + (_geomAngularAzimuthSpeed >> 1),

_geomAngularAzimuthSpeed, 0);

pred[1] += qphi0 * _geomAngularAzimuthSpeed;

}

}

#endif

### Selection of Additional Inter Predictors from Either GMC Reference Frame or from a Second Non-MC Reference Frame

It is proposed to select additional inter predictors from either GMC reference frame or from a second non-MC reference frame depending on whether the scene is detected as in motion or static at the encoder.

A moving state signalling mechanism is introduced to perform the selection. When the scene is detected as in motion, additional inter predictors generated from GMC reference frame are selected to compliment those from reference frame. Otherwise, when the scene is detected as static, additional inter predictors generated from a second non-MC reference frame (previous previous frame) are selected to compliment those from reference frame. This is illustrated in Figure 58.

The memory buffer for storing GMC reference frame buffer is also used to store the second non-MC reference frame in our implementation. The contents of the reference frame buffer are cleared at each Intra frame to support Random Access Frame functionality.

Figure 58: Inter predictors based on available Global Motion information.

### Geometry Data Unit Header Syntax Modifications

The slice_inter_frame_ref_gmc specifies the inter prediction GMC reference frame to be applied to the geometry data unit associated with the Geometry Data Unit (GDU) header with behaviour illustrated in Table 5 below.

When GMC is enabled and slice_inter_frame_ref_gmc is true for a frame i, the most recently decoded reference frame i – 1, one non-motion compensated, and one GMC is used in the inter prediction process. When slice_inter_frame_ref_gmc is false, two of the most recently decoded non-motion compensated frames i – 1 and i – 2 are used instead.

With GMC disabled, slice_inter_frame_ref_gmc is inferred to be false. Only the most recently decoded non-motion compensated frame i – 1 is used.

Table 5: Types of Decoded Reference Frames used based on GMC availability and slice_inter_frame_ref_gmc.

The following highlighted GDU Header syntax modifications are required.

## Inter Prediction Buffers in Predictive Geometry Coding [74]

G-PCC Ed. 2 supports inter prediction of geometry and attributes. LiDAR sequences are typically coded using predictive geometry coding with angular mode enabled and spherical coordinate conversion also enabled. The prediction for both attributes and geometry occur in the spherical coordinate domain; however, currently two different buffers are used for this purpose – one for geometry, and one for attributes. It must be noted that the reference frame used for attributes is a scaled and offset version compared to the reference frame used for geometry.

Figure 59: Inter prediction buffers for geometry and attributes.

It is proposed to use the same reference frame towards inter prediction for both the geometry and attribute inter prediction. For the reference frame used in geometry, a parameter inter_azim_scale_log2 was specified to enable to the quantization of reference frame. It was shown that modifying the value of this factor allows to tradeoff between the reference frame buffer size and compression efficiency. By using the same reference frame for geometry and attributes, it is possible to extend this flexibility to inter attribute coding also.

Figure 60: Shared inter prediction buffer for geometry and attributes.

The azimuth in the spherical table representation for inter geometry coding is coded with each index represented by one quantized azimuth and each entry containing only one point. If there are more than one point with the same quantized azimuth value, only the first point is chosen, and the rest of the points are dropped off. This is detrimental for QNX sequences because there are 8-10 points that are associated with the same azimuth value in the QNX sequences. Therefore, to retain the performance of QNX, it is proposed to retain the additional points in the buffer entries so that inter attribute prediction performance is not affected. The maximum number of points per buffer entry is signalled in the bitstream and may be used, along with the azimuth scale factor, to restrict the maximum buffer size in profiles. Once the reference frame has been used for geometry, the scale and offset (spherical coordinate conversion) are applied to prepare the reference frame for attributes.

## Downsampling For Inter Prediction [79]

In the current G-PCC, both geometry and attributes support inter prediction. The inter prediction process requires one or multiple reference frames.

The straightforward method for downsampling a reference frame was introduced. The proposed method involves selecting one point from a group of points that have the same attribute values and are located within the specified range. The selected point is then stored in the reference frame, and the rest, which are not selected, are eliminated from the reference frame. This process helps in reducing the data size and inter prediction complexity while maintaining the essential information.

The downsampling range is defined as follows:

- if the range is -1, the downsampling is disabled.

- if the range is 0, the downsampling occurs for points with the same attribute and at the same position.

- if the range is greater than 0, the downsampling occurs for points with the same attribute and laser index within the range.

## On Inter Prediction Buffers in Predictive Geometry Coding [81]

The case where angular mode and spherical coordinate conversion is considered, which is the common configuration for coding point cloud sequences captured using spinning LIDAR systems. It is asserted that the spherical coordinate conversion for the reference under RAHT occurs in two stages: first stage applies the spherical coordinate conversion, and the second stage applies a minimum position difference compared to the current frame. Secondly, the minimum position difference is only adjusted for RAHT inter prediction and not so for predicting or lifting transforms. This is asserted to be a oversight and hence it is proposed to align these so that the spherical coordinate conversion processes can be shared. Moreover it is proposed to modify the software to apply the spherical coordinate conversion on-the-fly as and when the positions are accessed for attribute inter prediction. This enables the use of single buffer, or avoid multiple conversions, in the case of multiple slices.

### Spherical coordinate conversion of reference frame

When spherical coordinate conversion is enabled and inter prediction is enabled, the reference frame is converted to the spherical coordinate space before using it for inter prediction. Currently, for RAHT prediction, this is applied in two steps: first step converts the reference frame positions based on the minimum position of the reference frame, and then a subsequent step may be applied if the minimum positions of the reference frames and the current frames are not aligned, potentially applying the process in two stages.

Proposal: It is proposed to apply spherical coordinate conversion of the reference frame in one step directly using a minimum of the minimum positions of the current and reference frame.

### Min position adjustment for predicting transform and lifting transform

Currently min position adjustment is only applied to the RAHT transform, whereas this is not applied in the case of predicting transform or lifting transform. This looks like an oversight as difference in the minimum position value of current and reference frame can occur both in RAHT transform, as well as predicting/lifting transform.

Proposal: It is proposed to use the adjusted min position for applying spherical coordinate conversion of the reference frame in predicting and lifting transform as well.

### Spherical coordinate conversion of reference frame on-the-fly

Currently the spherical coordinate conversion is applied to the reference frame as a whole. First the reference frame is used for inter prediction of geometry, and subsequently the reference frame is used for attributes after applying spherical coordinate conversion. In case of multiple slices, either both these frames have to be stored, or spherical coordinate conversion (and inverse) may have to be performed multiple times. This is not desirable.

Proposal (software-only): Implement the spherical coordinate conversion on-the-fly, so that the same buffer can be used for attribute and geometry inter prediction.

# Inter Prediction for Attribute Coding

## Unidirectional Inter Prediction for Attribute Coding [82][83][84][85]

The initial InterEM-v1 (also v2) included a tool for attribute inter prediction. As shown in Figure 61, the encoder and decoder perform nearest neighbor search in the current frame and the reference frame. The search center point has the same index in Morton code order as the current point. As a search result, at most three reference points, {point0, point1, point2}, are selected from the reference frame. The predicted attribute value will be calculated based on the attribute values of the reference points.

Figure 61: Nearest neighbor search in attribute inter prediction.

The attribute inter prediction tool is adopted into InterEM-v5.0 including improvements:

- The point with the largest Morton code is used that is less than the Morton code of current point as the search center:

Figure 62: Improved nearest neighbor search in attribute inter prediction.

- A two-thresholds method is used to constrain the attribute inter prediction if there is significant rotation or translation. If the maximum rotation angle in three directions is less than Th1 and the maximum translation amplitude in three directions is less than Th2, the attribute inter prediction will be performed.

As an encoder option in InterEM-v5, a hierarchical QP structure to perform inter prediction for attribute coding can be enabled. There is a QPshift value for each frame. The QPshift value for the reference frame should be lower than that of the current frame. For each frame, the real attribute QP value is set to:

QPoriginal+QPshift

For a IPPP GOF structure, the QPshift value for P-frame is equal to QPshift_step.

Proposals [82][83] enable attribute inter predicting for octree geometry, and proposals [84][85] for predictive geometry coding.

## Bidirectional Inter Prediction for Attribute Coding [64][65]

See also section 5.6 on bidirectional inter prediction for geometry coding.

For attribute coding, three reference points, {point0, point1, point2}, are selected from the two reference frames. The predicted attribute value will be calculated based on the attribute values of the reference points. The selection of reference points and calculation of predicted attribute values is the same as for the unidirectional inter prediction. In more detail, the two reference frames are merged into one new reference frame to perform inter prediction for attribute coding. The new reference frame contains all points in the two reference frames which are reconstructed from the geometry coding. Then unidirectional inter prediction for attribute coding is applied based on the new reference frame. The inter search range to find reference points is half of the original inter prediction search range assigned for unidirectional inter prediction.

As for the unidirectional case, a hierarchical QP structure is proposed. There is a QPshift value for each frame. The QPshift value for reference frame should be lower than that of the current frame. For each frame, the real attribute QP value is set to:

QPoriginal+QPshift

The QPshift value is controlled by one parameter QPshift_step value and the reference relationship. For a hierarchical GOF structure, the QPshift values for frames in a GOF are shown in Table 6.

Table 6: QPshift value for each frame in one hierarchical GOF.

| Frame time stamp | QPshift |
| --- | --- |
| 0 | 0 |
| 1 | QPshift_step *4 |
| 2 | QPshift_step *3 |
| 3 | QPshift_step *4 |
| 4 | QPshift_step*2 |
| 5 | QPshift_step *4 |
| 6 | QPshift_step *3 |
| 7 | QPshift_step *4 |
| 8 | QPshift_step |

For one IBBP GOF structure, the QPshift values for frames in a GOF are shown in Table 7.

Table 7: QPshift value for each frame in one IBBP GOF.

| Frame time stamp | QPshift |
| --- | --- |
| 0 | 0 |
| 1 | QPshift_step *2 |
| 2 | QPshift_step *2 |
| 3 | QPshift_step *2 |
| 4 | QPshift_step *2 |
| 5 | QPshift_step *2 |
| 6 | QPshift_step *2 |
| 7 | QPshift_step *2 |
| 8 | QPshift_step |

## Inter Prediction for DC and AC RAHT Coefficients [86][87]

It is proposed to apply inter prediction to DC and AC coefficients in RAHT. For DC coefficients coding, the residual between the DC coefficient for the root node of the current frame and the DC coefficient for the root node of the reference frame is calculated as:

DCresidual= DCcurrent- DCreference

The DCresidual is signalled to the decoder in place of DCcurrent.

For AC coefficients coding, the same bottom to up scan is performed on the reference frame and the current frame to generate the current octree and the reference octree. Then the same up to bottom scan is performed on the first 5 layers of the current octree and the reference octree to generate the average attribute of each node and corresponding reference node.

The prediction residual is calculated and signalled as:

ACresidual= ACcurrent- ACpredicted

ACpredicted=TransformAttrpredicted

Attrpredicted = Attrpredicted_inter  ? Attrpredicted_inter : Attrpredicted_intra

If the Attrpredicted_inter is equal to zero, the Attrpredicted_intra is applied as the original prediction.

## Improvements to Inter Prediction for RAHT Attribute Coding [91][92]

For RAHT inter prediction, it is proposed to signal the number of layers where the attribute inter prediction is enabled for the inter prediction of AC coefficients. The number of layers is indicated by one parameter “attrInterEnabledLayersForRAHT” in APS.  For different point cloud sequences, different numbers may be applied based on the characters of the sequence. In addition, it is proposed to disable the global motion constraint method for inter prediction for RAHT attribute coding. Whether the inter prediction for RAHT attribute coding is enabled is only determined by whether the current frame is an I-frame.

## Improved Coefficient Inter Prediction for RAHT Attribute Coding [93][94]

Figure 63 shows the block diagram of RAHT inter prediction (Note: Conventional Intra prediction is omitted from this figure), and Figure 64 shows the concept of the inter prediction. After RAHT transform, transformed coefficient for each node are memorized for the inter prediction of the next frame. In the encoding of the target node, residual value of the coefficient is calculated by using the coefficient of the reference frame and current frame, and residual value is coded. Decoder has the same process.

Figure 63: Block diagram of the RAHT inter prediction (encoder).

Figure 64: Overview of the RAHT inter prediction.

Inter prediction is applied only if the current point cloud and reference point cloud has the same position node. If predictive coefficient value does not exist in the reference buffer, inter prediction is not applied. If the corresponding AC coefficient does not exist in the reference buffer, inter prediction is not applied. Because proposal method was implemented on top of TMC13v21 in this EE, process has changed to ‘If the corresponding AC coefficient is not exit in the reference buffer, inter-prediction is applied.’ To be checked

Whether to use intra or inter is decided by the octree depth. If the depth of the node is less than the MAXdepth that is set in the encoder, then apply inter, otherwise, apply intra. The size of the buffer is determined by the value of MAXdepth. MAXdepth is signalled in the APS.

Figure 65: Condition to apply Inter-RAHT.

Table 8 shows the proposed syntax. attr_inter_prediction_enabled intend to use the inter prediction is used or not. If the inter prediction is used, the number of the depth that apply the inter prediction is signalled.

Table 8: Proposed syntax (APS).

| attribute_parameter_set( ) { | Descriptor |
| --- | --- |
| aps_attr_parameter_set_id | u(4) |
| aps_seq_parameter_set_id | u(4) |
| attr_coding_type | ue(v) |
| attr_primary_qp_minus4 | ue(v) |
| ・・・・ |  |
| aps_extension_present | u(1) |
| if( aps_extension_present ){ |  |
| attr_inter_prediction_enabled | u(1) |
| if( attr_inter_prediction_enabled ) { |  |
| if( attr_coding_type == 0 ) { //RAHT |  |
| raht_inter_prediction_depth_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |

Additional description needed.

## Attribute Inter Prediction with Slice Partitioning [95]

Improvements are proposed to enable attribute inter prediction when there are multiple slices. In G-PCC if there is more than one slice for each frame, the reference point cloud would not be correct. It is proposed to use the previous reconstructed frame as the reference point cloud of the attribute inter prediction for the current slice.

The reference point cloud is refined before attribute inter prediction as follows. Firstly, the geometry bounding box of the current slice, B, is calculated. B can be represented by (xmin, xmax, ymin, ymax, zmin, zmax) when the geometry information of the reference point cloud is in Euclidean coordinate system. Secondly, the points outside B are removed from the reference point cloud to generate the refined reference point cloud. The refined reference point cloud is used in attribute inter prediction. When attribute coding is finished, the reconstructed slice is merged with other coded slices in the same frame to get the whole reconstructed frame. The flow chart of the slice level inter prediction is shown in Figure 66.

Figure 66: Flow chart of slice level inter prediction.

In addition, it is proposed to use the geometry information and attribute information of the refined reference point cloud to derive the reference AC and DC coefficients. The derived reference AC and DC coefficients are used to predict the current AC and DC coefficients in the similar way as the current AC inter prediction method.

## Attribute Inter Prediction for predLifting Transform Coding [96]

### Introduction

In the current G-PCC predlift inter-attribute framework, an inter prediction is used to find the nearest neighbor in the reference frame.

- Firstly, as shown in Figure 67, the Morton code of the current point is obtained in the current frame. A reference point in the reference frame is identified whose Morton code is the first point greater than the Morton code of the current point. The reference point serves as the search center in the reference frame.

- Secondly, get the nearest neighbors in the neighbor search range [j-searchRange, j+searchRange] of the reference frame, where j is the Morton code in the reference frame and searchRange is the predetermined search range.

- Finally, at most N (N=3) reference points are selected from the reference frame, and the predicted attribute value will be calculated by averaging the N attribute values of the reference points.

Figure 67: Overview nearest neighbor search in attribute inter prediction.

### Inter-Attribute Nearest Neighbor Search

An inter-neighbor search method is proposed based on the geometric structure and spatial distribution of the reference frame point cloud. As shown in Figure 68, the proposed method tries to find the predictors of the current point P which shares a face, an edge, or a vertex with block B.

Figure 68: Overview of the proposed method.

For the current point P, the proposed inter-nearest neighbors search consists of the following steps.

- Firstly, use the existing method to find the search center in the reference frame.

- Secondly, a block B is obtained based on the geometry position of the determined search center in the reference frame.

- Finally, the co-planar, co-edge, and co-vertex neighbors of Block B are determined based on the atlas in the reference frame, and these neighbors are then considered candidates.

Note: considering the constraints on the atlas memory size, the proposed method sets the size of the atlas to {3×3×3}.

Additional description requested.

## Bidirectional Inter Prediction Attribute Coding for Predictive Geometry [97]

In the above, the bi-prediction is enabled for octree-predlift-inter configuration. There are two bi-prediction modes. When bi-prediction mode is set to 1, the IBBP GOF structure is applied for each GOF which is determined to be applied bi-prediction. When bi-prediction mode is set to 2, the hierarchical GOF structure is applied for each GOF which is determined to be applied bi-prediction.

Here, it is proposed to enable bi-prediction for attribute coding (predlift) when predictive geometry is enabled. For each predictive tree of B-frame, there are two reference predictive tree. The predictor search process and RDO based selection process like current unidirectional prediction are applied to each reference predictive tree. There is one additional flag “refDirFlag” to indicate which reference predictive tree that the predictor is from. The additional flag is also used in bits estimation process.

Additional description requested.

## Removing Same Reference Frame Octree Decomposition for RAHT Inter Prediction; Average Prediction [98][99]

In G-PCC, the RAHT inter prediction is performed by applying the same octree decomposition to the reference frame. The octree decomposition is determined by the geometry information of the current frame. To perform the same octree decomposition, each point in the reference frame is matched with one point in the current frame based on the Morton code distance. This kind of forced matching will affect the decomposition results of the reference frame, and there will be "pseudo-occupied node" with zero points in it. This will limit the efficiency of inter prediction.

### Direct Inter Prediction

The octree decomposition applied to the reference frame is determined by the geometry information of the reference frame. In each layer where the RAHT inter prediction is enabled, the octree location of the current node is derived. The nodes in the same layer in the reference frame are searched and the octree locations of these nodes are derived. If one of the searched nodes had the same octree location as the current node, the similar RAHT inter prediction as current design is enabled. Otherwise, only the intra prediction is enabled.

When the inter prediction is enabled for the current node, the average attribute of the reference node and the weight of the reference node is used to calculate the prediction transform coefficients.

### Average Prediction

It is proposed to perform RAHT average prediction for some specific layers.

For the first N layers of octree decomposition, the inter prediction is enabled. The inter prediction value is used to replace the intra prediction value if the inter prediction value is nonzero. For the next M layers of octree decomposition, the average prediction is enabled. the average prediction value is used to replace the intra prediction value when the inter prediction and the intra prediction are both nonzero. Otherwise, the inter prediction is used to replace the intra prediction when the inter prediction is nonzero. For other cases, the intra prediction value is used as the prediction value.

The RAHT average prediction value is calculated as the weighted average of the inter prediction value and intra prediction value:

Attraverage_predict = Attrinter_predict ×Winter + Attrintra_predict ×WintraWeightSum

The layer is closer to the root node layer, the weight of inter prediction value is higher. For example, when the node is in the Kth layer, the weights of inter prediction value and intra prediction value are calculated as:

Winter = WeightSum+N - K, Wintra = K – N

In implementation, bit shifting of the sumBits bits is used for the weighted average calculation:

Attraverage_predict = (Attrinter_predict ×Winter + Attrintra_predict ×Wintra) >> sumBits

WeightSum = 2sumBits

sumBits is derived as the minimum value to satisfy the following condition:

2sumBits>= M

N – 1 and M are signalled in APS, as “rahtInterPredictionDepthMinus1” and “rahtAveragePredictionDepth”.

## Temporal Filtering for RAHT [100][101]

In current G-PCC, the AC coefficients for the current RAHT node are simply copied from the co-located node in the reference frame. Such a prediction assumes the current node and reference node are perfectly correlated. The current proposal addresses this by scaling reference transform coefficients to obtain filtered prediction. Further, reference transform coefficients in different RAHT layers are scaled by different scaling factors.

It is proposed to obtain ACpredicted as follows:

ACpredicted=ACpredicted_inter  ? α×ACpredicted_inter : ACpredicted_intra

where α is the scaling factor. Since the degree of correlation between original RAHT node and reference RAHT node varies for different RAHT layers, α is chosen different for different layers. α could be a floating-point number. To eliminate floating point operations, we consider the integer version as follows:

αint=round(α×128)

Thus, the filtered AC prediction is given by,

ACfiltered-prediciton= (αint×ACpredicted_inter)≫7

The weights are signalled to the decoder.

Some restrictions for the method are as follows:

- The method is disabled for the first three inter-predicted layers since they don’t benefit much from the method.

- For QP greater than 40, there is not enough bandwidth (for sparse content) to convey the filter taps as side information. Thus, instead conveying these filter taps to the decoder, we rather use a set of fixed filter taps.

## Intra or Inter Prediction Decision Per RAHT Layer [102]

In the current RAHT transform coding, the inter prediction is applied only if the current node can find the prediction node in the reference frame for the first N layers. If the prediction node does not exist in the reference buffer, inter prediction is not applied, and the intra prediction will be used instead.

This contribution proposes not to restrict the number of layers that can use the inter prediction. In addition, one coding mode for every RAHT code layer is introduced to indicate the use of intra or inter prediction. On the encoder side, RDO is used to adaptively select the best coding mode.

Figure 69: Overview of the proposed method.

## RAHT Inter-Eligibility [104][105]

In current GPCC, during RAHT AC inter-prediction, the encoder and decoder have access to the DC value of the current node and the reference node. However, this information is not utilized during AC inter-prediction.

It is proposed to use the DC value of the current node (DCcur) and the reference node (DCref) to determine the eligibility for the AC inter-prediction. Specifically, it is proposed to disable AC inter-prediction and instead use intra-prediction if the following condition is satisfied:

DCref<Th1×DCcur   or    DCref>Th2×DCcur

where Th1 and Th2 are some thresholds. In the proposal, the thresholds are set to 0.5 and 2, respectively.

## Resampling for RAHT Inter-Prediction [107]

In current G-PCC, current frame and reference frame have different transform trees depending on their respective weight distributions. Thus, the reference node and current node are transformed with transform kernels that depends on their respective weight distributions. This ignores the occupancy and weight distribution information of the current node while generating the inter-prediction signal and further causes a mismatch in prediction domains. To address this, we propose to resample the reference node such that the transform domains of the current node and inter-prediction signal are aligned.

It is proposed to derive the inter-prediction in the following two steps:

Step1: Derive the prediction in mean attribute space: Depending on the weight of the child node (c) of the current RAHT node and the weight of its corresponding child node (r) in the reference RAHT node, three cases are considered:

Case 1:  If c is zero, then its inter-prediction is set to zero.

Case 2: If c is non-zero, but r is non-zero, the prediction is set to be the mean attribute of the reference RAHT node.

Case 3: If both c and r are non-zero, the prediction is set to be the mean attribute of the corresponding child node in the reference RAHT node.

Step2: Inverse Normalization: The prediction signal in the mean attribute space derived in step 1 is inverse normalized with respect to the square root of the weights of the current node to obtain the final inter-prediction signal.

The above steps follow the same flow as intra prediction, where prediction is first derived in mean attribute space followed by an inverse normalization.

## Global Motion-compensated Inter-prediction for Attribute Coding [109]

In the current G-PCC, the inter-prediction structure is used for geometry coding and attribute coding. As shown in Figure 70, when the current frame is eligible for inter-prediction, the reference frame is obtained from the reference buffer, and the global matrix is used to generate a motion-compensated inter-frame to code the geometry of the current frame. However, when the current frame is eligible for inter-attribute coding, the reference frame is obtained from the reference buffer or the predictor list, and directly used as the reference frame to code the attribute of the current frame.

Figure 70: Overview of the inter-prediction for the geometry and attribute coding.

This contribution proposes that like the geometry coding of the current frame, when the current frame attribute coding is based on the cartesian coordinates system, the global motion compensated frame may also be used in the attribute coding. As shown in Figure 71, the global motion matrix is used to generate a motion-compensated reference frame to code the attributes of the current frame.

Figure 71: Overview of proposed motion-compensated inter-prediction.

## Bi-directional Inter-prediction for RAHT Attribute Coding [110]

When the current frame has two reference frames, the proposed method will use the information of these two reference frames for attribute coding.

In the current G-PCC, the inter-prediction structure shown in Figure 72 is used for attribute coding to code the RAHT DC and AC coefficients, and the coding follows a top-down scan order along the octree. If the current point cloud and the reference point cloud have the same position node and there exist the corresponding AC coefficients in the reference frame, the inter-prediction coding will be used to code the current node, otherwise, intra-prediction is used to code the current node. More specifically, if slice_raht_layer_code_mode[depth] is equal to 1, the inter-prediction method is used to code the current layer, otherwise, the intra-prediction method is used to code the current layer.

Figure 72: Overview of the RAHT inter-prediction.

The following is proposed:

| attribute_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| adu_attr_parameter_set_id | u(4) | 7.4.4.2 |
| adu_temporal_id | u(3) | 7.4.4.2 |
| adu_sps_attr_idx | ue(v) | 7.4.4.2 |
| adu_slice_id | ue(v) | 7.4.4.2 |
| …………… |  |  |
| if( attr_inter_prediction_enabled){ |  |  |
| slice_attr_inter_prediction | u(1) | 7.4.4.2 |
| if( slice_biprediction) |  |  |
| slice_attr_inter_prediction2 | u(1) | 7.4.4.2 |
| if( slice_attr_inter_prediction && raht_send_inter_filters && ¬raht_inter_layer_code_enabled){ |  |  |
| num_inter_filters | ue(v) | 7.4.4.2 |
| for( filteridx = 0; filteridx < num_inter_filters; filteridx++) |  |  |
| raht_inter_filter_qidx [filteridx] | se(v) | 7.4.4.2 |
| } |  |  |
| ………… |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

slice_biprediction equal to 1 specifies that bi-prediction may be used to derive the positions in the GDU. slice_biprediction equal to 0 specifies that bi-prediction is not used to derive the positions in the GDU. When slice_biprediction is not present, it shall be inferred to be 0.

slice_attr_inter_prediction2 equal to 1 specifies that the inter prediction from the second reference frame may be used to derive the attributes in the ADU. slice_attr_inter_prediction equal to 0 specifies that the inter prediction from the second reference frame is not used to derive the attributes in the ADU. When slice_attr_inter_prediction2 is not present, it shall be inferred to be 0.

For an inter frame, if there are two reference frames, this inter frame is defined as a B-frame and the nodes within a B-frame may be coded with bi-directional prediction. More specifically, if the current layer is coded as an inter-layer, then the current node may be coded with bi-directional inter-prediction.

  - When the reference point in the first reference frame has the same position as the current point, set the reference AC coefficient as ref1, otherwise, set the ref1 as zero;

  - When the reference point in the second reference frame has the same position as the current point, set the reference AC coefficient as ref2, otherwise, set the ref2 as zero;

  - The prediction of AC coefficient for the current point is computed as follows:

predVal= ref1,            ref2=0&&ref2,              ref1=0&&ref1+ref2+1≫1 ,      ref1≠0 and&&&&&&&&&&& ref2≠0

## Improved Inter-attribute Coding Method for RAHT [111]

It is proposed to add the coding mode non-prediction for every RAHT coding layer. As a result, the RAHT coding layer may be coded as either inter-prediction, intra-prediction, or non-prediction. On the encoder side, RDO is used to select the best coding mode adaptively.

In the current G-PCC, the inter-prediction flowchart shown in Figure 73 for attribute coding is used to code the RAHT DC and AC coefficients. On the encoder side, separately compute the intra-prediction cost and inter-prediction cost. If the intra cost is less than the inter cost, then intra-prediction will be used to code the current RAHT coding layer, otherwise, the inter-prediction will be used instead.

On the decoder side, more specifically, if slice_raht_layer_code_mode[depth] is equal to 1, then the inter-prediction method is used to code the current layer, otherwise, the intra-prediction method is used to code the current layer.

Figure 73: Flowchart of inter-raht coding.

It is proposed to remove the syntax element layer_code_depth which is similar to the meaning of the syntax element raht_inter_layer_depth_minus1. It is also proposed to introduce an additional coding mode, non-prediction, for every RAHT coding layer as illustrated in Figure 74. As a result, the current RAHT coding layer may be coded as either intra-prediction, inter-prediction, or no-prediction.

Figure 74: Overview of the RAHT inter/intra/non-prediction.

## Reference Node Resampling for Lossless RAHT [112]

For lossless inter-RAHT, the current frame and reference frame have different transform trees depending on their respective weight distributions. Thus, the reference node and current node are transformed with transform kernels that depend on their respective weight/occupancy distributions. This causes a mismatch in prediction domains. To address this, we propose to resample the reference node such that the transform domains of the current node and inter-prediction signal are aligned. A similar method was proposed for lossy RAHT.

It is proposed to derive the inter-prediction by applying a resampling step. Depending on the weight of the child node (c) of the current RAHT node and the weight of its corresponding child node (r) in the reference RAHT node, three cases are considered:

Case 1:  If c is zero, then its inter-prediction is set to zero.

Case 2: If c is non-zero, but r is non-zero, the prediction is set to be the mean attribute of the reference RAHT node.

Case 3: If both c and r are non-zero, the prediction is set to be the mean attribute of the corresponding child node in the reference RAHT node.

## Global Motion Compensated Inter Prediction for Attribute Coding [114] (clipping aspect)

In the current G-PCC, when the current frame is eligible for inter prediction, and the attribute coding is based on the cartesian coordinates system (i.e., attr_coord_conv_enabled equal to zero), then the motion-compensated reference frame will be used to generate attribute predictions for the current frame. It is proposed that the motion-compensated inter-prediction is also applied for spinning-lidar category attribute coding, when the attribute coding is based on the cartesian coordinates system (i.e., attr_coord_conv_enabled equal to zero).

Figure 75: Overview of proposed motion-compensated inter-prediction.

It will follow the process described below to get the reference frame to predict the current frame attribute.

- Same as the geometry coding, the global matrix is used to generate a motion-compensated reference frame.

- After getting the reference frame, constraints the reference frame point position larger than 0.

- Using the reference frame to code the attribute coding.

## Motion Compensated Attributes Projection Without Using Dual Motion for Octree [116] (GeS-TM)

With motion compensated attributes projection (MCAP) the same geometry is used for both the current frame/attributes and the inter frame/attributes used for its prediction.

This property can be used to simplify inter RAHT and it should improve its coding performances. Thus, always using motion compensated attributes projection for both octree and TriSoup would be beneficial for the codec.

Currently, dual-motion is required to enable MCAP, but coding its second motion field is too expensive for being used with octree without negatively impacting the BD-Rate.

This contribution then provides some re-architecting of the codec to be able to reuse the motion field from the geometry within the motion compensated attributes projection; and so, it allows using motion compensated attributes projection with octree, without using dual motion.

Additional description requested

## Method to Enhance Inter RAHT Average Prediction [117] (GeS-TM)

When the average prediction is enabled for inter RAHT, the winter and wintra are calculated based on voteInter and voteIntra. The voteInter and voteIntra currently are derived based on the prediction modes (inter-/ intra-/ null prediction) of the parent node, uncle nodes, and cousin nodes. However, it may not utilize the benefits of all the neighborhood information. It is proposed to add a grandparent prediction mode.

This contribution proposes to add the number of prediction mode of the grandparent node in the calculation of voteInter and voteIntra. The proposed modifications are described in detail as follows.

- If one uncle node is applied with inter-/ intra-/ null prediction, voteInter += 2/ voteIntra += 2/ voteInter += 1, voteIntra += 1.

- If one cousin node is applied with inter-/ intra-/ null prediction, voteInter += 6/ voteIntra += 6/ voteInter += 3, voteIntra += 3.

- If the parent node is applied with inter-/ intra-/ null prediction, voteInter += 12/ voteIntra += 12/ voteInter += 6, voteIntra += 6.

- If the grandparent node is applied with inter-/ intra-/ null prediction, voteInter += 28/ voteIntra += 28/ voteInter += 14, voteIntra += 14.

The updated voteInter and voteIntra will be used to calculate the weights for deriving the final prediction.

Figure 76: Proposed method of neighborhood information about calculating the voteInter and voteIntra.

# Inter Prediction: Other Techniques

## Dependent Entropy Frame Coding [53][54]

The entropy continuing slice method is used in low-latency scenarios in intra frame coding of G-PCC Ed.1. In inter frame coding, each frame is linked to global/local motion and inter occupancy prediction. This method introduces the dependent P frame by continuing entropy as illustrated in Figure 77.

Figure 77: Flowchart of using dependent entropy frames.

## Signalling of Global Motion Data [58]

The two segment thresholds and global motion matrix, denoted by global motion parameters, are coded in the slice header. To align with the statistics of the global motion parameters, global motion parameters are coded by using the signed 0-th order Exp-Golomb coding method. At the encoder side, a signed motion parameter is first converted to an unsigned integer, then coded using an unsigned 0-th order Exp-Golomb code. At the decoder side, the decoded unsigned integer value will be converted back. The conversion between a signed value x and an unsigned value y is done as follows:

y=-2x,  x≤02x-1,     x>0

## Inter Entropy Continuation HLS [70]

### Background

#### Slice Entropy Continuation and Constraint of Slice Reordering

The entropy continuing slice method is used in low-latency scenarios in intra frame coding of G-PCC Ed.1 and current G-PCC. Slice entropy continuation corresponding syntax “entropy_continuation_enabled” is used to control whether to restore the entropy state saved in the previous slice. For each slice, “gbh.entropy_continuation_flag” indicates the current slice reuses contexts from the previous slice. The “entropy_continuation_enabled” flag is closely related with “slice_reordering_constraint”, which specifies the bitstream whether sensitive to the reordering or removal of slices.

- For encoder, the value of sps.profile.slice_reordering_constraint_flag is determined by sps.entropy_continuation_enabled_flag:

- For decoder, it is a requirement of bitstream conformance that “entropy_continuation_enabled” shall be equal to 0 when “slice_reordering_constraint” is equal to 0:

This mechanism makes sure slice entropy continue function can be used only when reorder of the slice is restricted.

#### Inter Entropy Continuation

In current G-PCC, dependent entropy frame coding is included. The corresponding flag “gps.gof_geom_entropy_continuation_enabled_flag” exists in geometry parameter set. In decoder, current frame/slice shall use geometry entropy context state dependent on reference frame/slice when gps.gof_geom_entropy_continuation_enabled_flag is 1 and gbh.interPredictionEnabledFlag is 1 no matter the value of gbh.entropy_continuation_flag.

Obviously, the constraint of slice reordering should also be considered when inter entropy continuation is performed.

### Proposed Change

- The flag of inter entropy continuation “gof_geom_entropy_continuation_enabled_flag” in GPS should be placed in SPS called “inter_entropy_continuation_enabled_flag”.

| encoder |  |
| --- | --- |
| if (!gbh.entropy_continuation_flag) { if ( !_gps->gof_geom_entropy_continuation_enabled_flag ¦¦ !gbh.interPredictionEnabledFlag) { _ctxtMemOctreeGeom->reset(); _ctxtMemPredGeom->reset(); } for (auto& ctxtMem : _ctxtMemAttrs) ctxtMem.reset(); } | if (!gbh.entropy_continuation_flag) { if ( !_sps->inter_entropy_continuation_enabled_flag ¦¦ !gbh.interPredictionEnabledFlag) { _ctxtMemOctreeGeom->reset(); _ctxtMemPredGeom->reset(); } for (auto& ctxtMem : _ctxtMemAttrs) ctxtMem.reset(); } |
| decoder |  |
| if (_gbh.entropy_continuation_flag) { assert(!_firstSliceInFrame); assert(_gbh.prev_slice_id == _prevSliceId); } else { // forget (reset) all saved context state at boundary if ( !_gps->gof_geom_entropy_continuation_enabled_flag ¦¦ !_gbh.interPredictionEnabledFlag) { _ctxtMemOctreeGeom->reset(); _ctxtMemPredGeom->reset(); } for (auto& ctxtMem : _ctxtMemAttrs) ctxtMem.reset(); } | if (_gbh.entropy_continuation_flag) { assert(!_firstSliceInFrame); assert(_gbh.prev_slice_id == _prevSliceId); } else { // forget (reset) all saved context state at boundary if ( !_sps->inter_entropy_continuation_enabled_flag ¦¦ !_gbh.interPredictionEnabledFlag) { _ctxtMemOctreeGeom->reset(); _ctxtMemPredGeom->reset(); } for (auto& ctxtMem : _ctxtMemAttrs) ctxtMem.reset(); } |

- The slice reordering constraint should be considered in inter entropy continuation.

- In encoder, the “sps.profile.slice_reordering_constraint_flag” should be determined by both slice and inter entropy continuation enable flags “sps.entropy_continuation_enabled_flag” and “sps->inter_entropy_continuation_enabled_flag”

- In decoder, it is an additional requirement of the bitstream conformance that both inter prediction and inter entropy continuation enable flags should be 0 when “sps.profile.slice_reordering_constraint_flag” is 0.

| encoder |  |
| --- | --- |
| params->sps.profile.slice_reordering_constraint_flag = params->sps.entropy_continuation_enabled_flag; | params->sps.profile.slice_reordering_constraint_flag = (params->sps.entropy_continuation_enabled_flag ¦¦ (params->sps.inter_frame_prediction_enabled_flag && params->sps.inter_entropy_continuation_enabled_flag)); |
| decoder |  |
| if (sps.entropy_continuation_enabled_flag) assert(sps.profile.slice_reordering_constraint_flag); | if ( sps.entropy_continuation_enabled_flag ¦¦ (sps.inter_frame_prediction_enabled_flag && sps.inter_entropy_continuation_enabled_flag)) assert(sps.profile.slice_reordering_constraint_flag); |

## Dependent Entropy Frame Coding for Attribute [88][89][90]

Dependent P frame coding by continuing entropy for attribute coding is proposed. Continuing entropy context for attribute will be determined according to random access period, gps.gof_geom_entropy_continuation, and inter prediction flag for attribute as illustrated in Figure 78.

Figure 78: Flowchart of using proposed dependent entropy frames 
(proposed change is highlighted with blue box).

Additional fixes:

- Dependent frame entropy coding is controlled for attributes separately based on attribute inter prediction flag. It is an unlikely case that geometry uses inter prediction and attribute does not. It is proposed that when dependent frame entropy coding is applied, both geometry and attribute entropy states are copied from previous frame.

- Entropy continuation signals prev_slice_id but dependent frame entropy coding does not signal. This ID is signalled in order to detect whether a dependent slice can be decoded when a loss occurs. It is proposed to signal a prev_slice_id and prev_frame_id when entropy coding states are copied from a previous frame (i.e., under dependent frame entropy coding).

- When entropy continuation is disabled, dependent frame entropy coding takes over and does the job of entropy continuation. It is proposed to signal a slice_dependent_frame_entropy_coding flag; set to 1 only for the first slice of a frame.

## Temporal ID for Temporal Scalability [73]

In G-PCC Edition 1, temporal ID was not explicitly supported in the specification because only intra prediction was used; it was suggested that systems specification can support this functionality. However, with the introduction of inter prediction in Ed. 2, there is a need to have some indication in the G-PCC bitstream so that temporal scalability can be provided without much complexity. In video, a temporal identifier is signalled in slices to indicate which slices belong to which temporal layer. When a bitstream is to be transmitted/decoded at a particular frame rate, slices that have a temporal ID up to a particular value are extracted/transmitted/decoded.

In G-PCC Edition 1, identifying the need for future support for temporal scalability when inter prediction is enabled, three bits were reserved in the ADU and the GDU, as highlighted below.

| geometry_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| gdu_geometry_parameter_set_id | u(4) | 7.4.3.2 |
| gdu_reserved_zero_3bits | u(3) | 7.4.3.2 |
| slice_id | ue(v) | 7.4.3.2 |
| slice_tag | u(v) | 7.4.3.2 |
| frame_ctr_lsb | u(v) | 7.4.3.2 |

| attribute_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| adu_attr_parameter_set_id | u(4) | 7.4.4.2 |
| adu_reserved_zero_3bits | u(3) | 7.4.4.2 |
| adu_sps_attr_idx | ue(v) | 7.4.4.2 |
| adu_slice_id | ue(v) | 7.4.4.2 |

When temporal ID is signalled, there are couple of flexibilities that must be constrained to make temporal scalability work as expected. It does not make sense to have slices within a frame to have different temporal ID. Also decoding of some frames may be dependent on other frames due to inter prediction. To support temporal scalability, such dependency should be restricted to only occur such that temporal operating points of the bitstream do not result in missing references.

It is proposed to rename the reserved bits (as in GPCC Ed. 1 bitstream) in the GDU and ADU headers as temporal ID. The changes are as follows:

| geometry_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| gdu_geometry_parameter_set_id | u(4) | 7.4.3.2 |
| gdu_temporal_id | u(3) | 7.4.3.2 |
| slice_id | ue(v) | 7.4.3.2 |
| slice_tag | u(v) | 7.4.3.2 |
| frame_ctr_lsb | u(v) | 7.4.3.2 |

| attribute_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| adu_attr_parameter_set_id | u(4) | 7.4.4.2 |
| adu_temporal_id | u(3) | 7.4.4.2 |
| adu_sps_attr_idx | ue(v) | 7.4.4.2 |
| adu_slice_id | ue(v) | 7.4.4.2 |

In addition, it is proposed to add the following two constraints in the specification to ensure that frame at different temporal operating points of the bitstream do not refer to a frame that has been removed due to temporal scalability.

- It is a requirement of bitstream conformance that the temporal ID of all GDUs and ADUs headers of a point cloud frame are equal. The temporal ID of a point cloud frame is equal to the temporal ID of any of the GDUs/ADUs of the frame.

- It is a requirement of bitstream conformance that the decoding of current frame shall not depend on any point cloud frame F in the bitstream where the temporal ID of frame F is greater than the temporal ID of the current frame.

## Frame Unused for Reference Indication [75]

It is proposed to add two new data unit payload types which specify that the geometry and attribute data units that are not used for reference. The existing data units would not have any such restriction, whereby they may or may not be used for reference. There is no change to the syntax structure of the data units. No change to CTC is necessary at this stage, as this would be a feature enabled by the encoders according to the respective application.

Table 9: Mapping of tlv_type and associated data unit to syntax tables.

| tlv_type | Syntax table | Description |
| --- | --- | --- |
| 0 | 7.3.2.1 | Sequence parameter set data unit |
| 1 | 7.3.2.5 | Geometry parameter set data unit |
| 2 | 7.3.3.1 | Geometry data unit |
| 3 | 7.3.2.6 | Attribute parameter set data unit |
| 4 | 7.3.4.1 | Attribute data unit |
| 5 | 7.3.2.4 | Tile inventory data unit |
| 6 | 7.3.2.8 | Frame boundary marker data unit |
| 7 | 7.3.5 | Defaulted attribute data unit |
| 8 | 7.3.2.7 | Frame-specific attribute properties data unit |
| 9 | 7.3.2.9 | User data data unit |
| 10 | 7.3.3.1 | Geometry data unit unused for reference |
| 11 | 7.3.4.1 | Attribute data unit unused for reference |

## TLV Type Constraint When Multiple Slices Present [76]

There may be multiple slices for one point cloud frame, and the geometry and attribute data from multiple slices may be encapsulated at different TLV data units. As a result, one slice (geometry or/and attribute) may be set as a reference while another slice in the same point cloud frame could be set as non-reference. To ensure the multiple slices bitstream is decodable, this contribution proposes that when one point cloud frame has multiple slices, all the geometry TLV data units for all slices within a frame must have the same TLV type, all the attribute TLV data units for all slices within a frame must have the same TLV type.

## On Inter Prediction Syntax [80]

This document proposes four modifications to the signalling of syntax elements associated with inter prediction (and biprediction) in parameter sets and data unit headers. It is asserted that the changes are needed to avoid ambiguous interpretation of the syntax elements, fix a bug and improve readability of the specification. The four changes are as follows:

- Aspect 1: Add constraint that the GPS and APS flags enabling inter prediction are disabled when the SPS inter prediction flag is disabled.

- Aspect 2: Add constraint that the APS attribute inter prediction flag is disabled when the GPS inter prediction flag is disabled.

- Aspect 3: Condition the signalling of slice_attr_inter_prediction2 (indicating slice-level attribute bi-prediction) on slice_attr_inter_prediction (indicating slice-level attribute inter prediction)

- Aspect 4: Re-order global motion parameter syntax elements in the GDU to improve readability of the specification.

## Attribute Syntax Improvements [113]

AC coefficients QP offset and temporal filtering are designed to improve the inter prediction of RAHT. The syntax elements related to these tools are transmitted in the bitstream, when RAHT attribute coding is applied. However, two tools can only be applied to RAHT lossy coding. When RAHT lossless coding is applied, there are some unnecessary bits to transmit these elements in the bitstream.

## Inter-frame Context Dependency for RAHT Pred Mode [115]

RAHT layers are coded using three modes, intra prediction, inter prediction or no prediction - indicated by a mode. The RAHT prediction mode is signalled as a context coded element – two contexts are used. For intra-coded attribute data units (ADU), the context is reset at the beginning of each ADU. However, for inter-coded ADUs, the contexts are not reset. This introduces parsing dependency between ADUs which is not desirable.

Moreover, current codec supports the continuation of entropy continuation within slices of a frame (entropy continuation) and for the first slice of an inter-coded frame (inter entropy continuation). The current implementation of prediction mode signalling does not use this mechanism thereby violating the entropy continuation mechanism.

It is proposed to use the context mechanism used for other attribute-related contexts, so that parsing dependence between ADUs is only introduced under entropy continuation/inter entropy continuation.

- Reset mode context variables unless slice_entropy_continuation is equal to 1 or slice_inter_entropy_continuation is equal to 1.

Also, the prediction modes for RAHT are signalled once per RAHT layer, and there are at most couple of dozen RAHT layers in a slice. Context coding for these modes seems like an overkill; bypass coding should be sufficient without impact on coding efficiency.

# Attribute Coding

## Adaptive Quantization for LoD-Based Attribute Predicting Transform Coding [118][119]

In the Predicting Transform that is part of attribute coding in G-PCC Ed.1, the quantization step value obtained from the Attribute Parameter Set will be directly used to quantize the residual values. This quantization step value remains the same for all points.

In the LoD-based prediction strategy, points in lower LoDs are more influential since they are used more often for prediction. Therefore, in [118] it was proposed to introduce an adaptive quantization method in the Predicting Transform. Specifically, the points are traversed in reverse LoD-based order to update the quantization weights for the nearest neighbors of the current point, then multiply the prediction residuals by the quantization weight of each point and then perform quantization. The decoder can be implemented inversely.

The level of detail (LoD) generation process (see Figure 79) from G-PCC Ed.1 re-organizes the points into a set of refinement levels according to a set of L1 (Manhattan) distances. In addition, this re-ordering process is deterministic and operated based on reconstructed geometry. It is applied at both the encoder and the decoder side.

Figure 79: Level of detail generation process.

The LoD-based prediction strategy makes points in lower LoDs more influential since they are used more frequently for prediction. Therefore, the proposed method is an adaptive quantization method, which assigns different quantization steps to points according to their importance as described in the following.

Let w(P) be the quantization weight associated with a point P. Then w(P) is computed by applying the following traversal procedure:

- Set the initial w(P) of all points to 256.

- Traverse the points according to the inverse of the LoD-based order.

- For a point Q, update the quantization weights for its 3-nearest neighbors P∈∇(Q) as follows:

w(P)←w(P)+(αP,Q×wQ)≫8,

where ∇(Q) means the set of 3-nearest neighbors of point Q, and the value of α(P,Q) is given as follows:

αP,Q=32,  if P is the 1st nearest neighbor of Q16,        if P is the 2nd nearest neighbor of Q8,  if P is the 3rd nearest neighbor of Q

After getting the quantization weights of all points, compare the value of w(P) and quantization step Qstep, then let wP=min⁡(w(P),  Qstep). Note that the value of αP,Q can be stored in the Attribute Parameter Set (APS). The proposed syntax is shown in Table 10.

Table 10: Proposed New Attribute Parameter Set Syntax.

| attribute_parameter_set( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( attr_coding_type = = 1 ) { |  |
| max_num_direct_predictors | ue(v) |
| if( max_num_direct_predictors) { |  |
| adaptive_prediction_threshold | u(8) |
| direct_avg_predictor_disabled_flag | u(1) |
| } |  |
| for( idx = 0; idx < max_num_direct_predictors; idx++){ |  |
| impactFactorOfNearestNeighborsInAdaptiveQuant[idx] | ue(v) |
| } |  |
| intra_lod_prediction_skip_layers | ue(v) |
| …… |  |
| } |  |
| …… |  |
| } |  |

impactFactorOfNearestNeighborsInAdaptiveQuant[idx] specifies the impact factor of nearest neighbor points at different distance level (i.e., 1st nearest, 2nd nearest and etc).

On the encoder side, the prediction residuals associated with a point P are multiplied by the factor wP. An inverse scaling process by the same factor is applied after inverse quantization on the decoder side. Note that the quantization weights wP are completely determined by the reconstructed geometry and they do not need to be encoded in the bitstream.

## Neighbor Search Method for Attribute LoD Prediction [120][121][128]

In G-PCC Ed.1 design, the attribute prediction is performed between the point to be coded and its N nearest neighbors. Considering the complex distribution of 3D points, choosing the nearest points as predictors (i.e., using distance as the only criterion) may not always be optimal.

Figure 80: Neighboring points distribution.

The proposed method considers the point distribution when selecting potential predictors. More specifically, the relative location of potential predictors of current point P are considered together with their distance to P. An example is given in Figure 80. In this case, although P2 is nearer to current point P than P3, P3 may be a better predictor of P.

The following steps detail the method:

Step 1: build two neighbor lists.

List1: the nearest 3 neighbors obtained using existing method in G-PCC.

List2: a list with N (e.g., 3) points that are dropped out when updating List1.

The final predictor list is generated by updating list1 using points in list2, as the eligibility check described in Step 2 and Step 3. Figure 81 shows the generation of list1 and list2.

Figure 81: The generation of List1 and List2.

Figure 82: Definition of opposite direction. The eight octants around P are indexed from 0-7.

Step 2: check of the point distribution (strict opposite check)

Check the distribution of points in list1. If P1 or P2 are already at the strict opposite direction (Definition of opposite is illustrated in Figure 82) to P0, it is believed that the distribution of neighbors is already well spread, and thus there is no need to check points in list2 anymore. Otherwise, check points Pn in list2, if dist(Pn, P) <= T1 and Pn is at the strict opposite direction to P0, use Pn to replace P2; if not, then, if dist(Pn, P)<= T2 and Pn is at the strict opposite direction to P1, use Pn to replace P2. In the current implementation, T1 = w*dist(P2, P), T2 = w*dist(P1, P), and w <<5 = 54.

Step 3: Perform check of the point distribution (loose opposite check)

If P2 has not been replaced after step 2, and P2 or P1 is at the same direction (P0, P1 and P2 are in the same region) to P0, the distribution check is softened to some extent. Specifically, the distribution check is based on the pre-defined loose opposite directions as shown in Figure 82. Similar to Step 2, points Pn in list1 are checked. If dist(Pn, P) <= T1 and Pn is at the loose opposite direction to P0, use Pn to replace P2; if not, then, if dist(Pn, P) <= T2 and Pn is at the loose opposite direction to P1, use Pn to replace P2.

In [128] it was observed that the following checks are missing in the implementation of the algorithm:

- Missing strict opposite check of P1 and P2 – the implementation checks whether P0 and P2 are strictly opposite, or whether P0 and P1 are strictly opposite. If either of this is true, then no replacement occurs. However, if P1 and P2 are strictly opposite, there are cases where P2 will be replaced by another node that is farther than P2.

- Missing loose opposite check of P1 and P2 – similar to the strict check, the loosely opposite direction check also does not always consider the case where P1 and P2 are loosely opposite to each other. This also results in sub-optimal swapping.

- Missing checks of equality for P1 and P2 – during loosely opposite direction check, when P0 and P1 are equal or when P0 and P2 are equal, steps are taken to replace P2. However, no steps seem to be taken when P1 and P2 are equal. This also may lead to sub-optimal distribution of points.

An implementation with the proposed solutions is provided below:

- Add a strict opposite check for P1 and P2 (yellow highlight).

- Use single threshold to check whether directions of P3..Pn are strictly opposite to direction of P0 or P1 (green highlight).

- When strict opposite directions are not present, do not replace if P1 and P2 are loosely opposite to each other. (implicitly achieved)

- When P1 and P2 are equal, replace P2 (if not already so) with a loosely opposite direction to P0/P1 (cyan highlight).

int replaceIdx = -1;

if (

dir[1] == 7 - dir[0] || dir[2] == 7 - dir[0] || dir[2] == 7 - dir[1])

replaceFlag = false;

for (int h = 3; replaceFlag && h < numend1; ++h) {

if ((dir[h] == 7 - dir[0]) || (dir[h] == 7 - dir[1])) {

replaceFlag = false;

replaceIdx = h;

}

}

bool equal01 = dir[0] == dir[1];

bool equal02 = dir[0] == dir[2];

bool equal12 = dir[1] == dir[2];

const auto& looseDirs0 = looseDirTable[dir[0]];

if (replaceFlag) {

if ((equal02 || equal12) && equal01) {

for (int h = 3; replaceFlag && h < numend1; h++) {

if (

dir[h] == looseDirs0[0] || dir[h] == looseDirs0[1]

|| dir[h] == looseDirs0[2]) {

replaceFlag = false;

replaceIdx = h;

}

}

} else if ((equal02 || equal12) && !equal01) {

if (!(dir[1] == looseDirs0[0] || dir[1] == looseDirs0[1]

|| dir[1] == looseDirs0[2]))

for (int h = 3; replaceFlag && h < numend1; h++)

if (dir[h] != dir[0] && dir[h] != dir[1]) {

replaceFlag = false;

replaceIdx = h;

}

} else if (equal01) {

if (!(dir[2] == looseDirs0[0] || dir[2] == looseDirs0[1]

|| dir[2] == looseDirs0[2]))

for (int h = 3; replaceFlag && h < numend1; h++) {

if (

dir[h] == looseDirs0[0] || dir[h] == looseDirs0[1]

|| dir[h] == looseDirs0[2]) {

replaceFlag = false;

replaceIdx = h;

}

}

}

}

if (replaceIdx >= 0) {

localIndexes[2] = localIndexes[replaceIdx];

localRef[2] = localRef[replaceIdx];

}

## Low-Latency Attribute Coding Inside Slice [122][123]

In G-PCC Ed.1, low-latency applications can be supported by slicing according to the input point order. For example, given a list of input points, the encoder could generate a new slice every n points. Here, it is proposed to support low-latency applications inside a slice for geometry coding and attribute coding. For geometry coding, the tree structure of predictive tree geometry coding can be generated every n points (maxPtsPerTree). For attribute coding, the number of levels of detail (LOD) is set to 1 and the sorting process based on Morton code is skipped before LOD generation process. For low-latency attribute coding inside a slice, this is not flexible enough. Points are either sorted entirely or not sorted at all. There is no flexible trade-off between coding efficiency and latency, and the geometry latency cannot be matched.

To enhance the flexibility of low-latency attribute coding, it is proposed to control the sorting process based on Morton code by a syntax element named “max_points_per_sort_log2_plus1” in APS in case the number of LOD is equal to 1.  Specifically, if max_points_per_sort_log2_plus1 is equal to 0, the sorting process will be performed. If max_points_per_sort_log2_plus1 is greater than 0, a segmented sorting process will be performed. The point number of every sorted segment is equal to maxPtsPerSort, where maxPtsPerSort = 1 << (max_points_per_sort_log2_plus1 - 1). As shown in Figure 83, compared with the low-latency solution by slicing, the attributes can be predicted crossing boundary using the proposed method.

Slice A

Slice B

Can’t be predicted crossing boundary

Segment B

Can be predicted crossing boundary

SearchRange

Segment A

Slice

Figure 83: Predicting crossing boundary.

## Extended Prediction for RAHT [124][125]

In G-PCC Ed.1, upsampled transform domain prediction is used to improve the efficiency of RAHT coding. In upsampled transform domain prediction, the RAHT transform is performed from top to bottom. In each depth, RAHT is applied to each unit node containing 2×2×2 sub-nodes in Morton order. When performing transform on a unit node that contains 2×2×2 sub-nodes at depth d, 3 coplane parent neighbour nodes and 3 coline parent neighbour nodes and a parent node at depth d-1 are used to predict the attribute in each sub-node. Prediction weights are assigned to these parent-level nodes, where ωparent:ωcoplane:ωcoline=4:2:1. And the prediction is performed in the mean attribute space, i.e., ai=Ai/ωi.

sub-node

parent-level neighbour node

parent node

Figure 84: Parent-level nodes for each sub-node of transform unit node.

However, some neighbour nodes are transformed before the current transformation unit node, so attributes of the sub-node of these already-coded neighbour nodes are available, which has not been utilized in the current design.

z

x

y

Figure 85: Neighbour nodes that may be transformed earlier than current transformation unit node: earlier node (green), possibly earlier node depending on Morton code (purple).

Here, a method is proposed of sub-node-based prediction in transform domain for RAHT, which uses the attributes of a sub-node of already-coded neighbour nodes to improve transform domain prediction for RAHT. For a coplane parent neighbour node, if it has a coplane sub-node that shares a face with the predicted sub-node, attributes of the coplane sub-node will be used as the prediction instead of attributes of its corresponding coplane parent neighbour node.

coplane parent neighbour node

coplane sub-node

predicted sub-node

Figure 86: Coplane parent neighbour node with coplane sub-node with predicted sub-node.

Similarly, for a coline parent neighbour node, if it has a coline sub-node that shares an edge with the predicted sub-node, attributes of the coline sub-node will be used as the prediction instead of attributes of its corresponding coline parent neighbour node.

coline parent neighbour node

coline sub-node

predicted sub-node

Figure 87: Coline parent neighbour node with coline sub-node with predicted sub-node.

The prediction weights will be signalled in APS and will be coded by unsigned Exp-Golomb. The default prediction weights are assigned as follows:

ωparent:ωcoplane:ωcoline:ωcoplaneSub:ωcolineSub=9:3:1:5:2

## Disabling Transform Domain Prediction of RAHT [126][127]

In G-PCC Ed.1, transform domain prediction was introduced to improve the coding efficiency of RAHT. To reduce the runtime of predictions, an early termination decision flow of the transform domain prediction was introduced, as illustrated in Figure 88. Specifically, for the attribute prediction of the 8 sub-nodes in the current node, the prediction termination is controlled by examining following two parameters:

•	NumValidP: total number of valid parent neighbour nodes of the 8 sub-nodes

•	NumValidGP: total number of valid grandparent neighbour nodes of the 8 sub-nodes

At first, NumValidGP is checked to see whether it is larger than a threshold TH1 (= 2). If it is true, then the parent neighbor nodes are searched and the value of NumValidP is further obtained. In this flow, if either NumValidGP or NumValidP is less than TH1 or TH2, respectively, the prediction is disabled and the 8 sub-nodes are encoded without prediction.

Figure 88: G-PCC Ed.1 decision flow to disable the transform domain prediction.

It is observed that if there is only one occupied child in the current node, the transform of the original and the predicted attribute value only results in one DC coefficient, no AC coefficient, and thus no AC coefficient residuals are coded, as shown in Figure 89. In that case, the transform and attribute prediction are actually redundant since the DC coefficient of the current node used in the inverse transform is directly inherited from its parent. To avoid the time-consuming neighbour search and attribute prediction in this case, following modifications are proposed.

Figure 89: Upsampled RAHT for one occupied sub-node case.

To reduce the unnecessary complexity caused by the above issue, an additional parameter:

•	NumValidC: total number of occupied child nodes in the current node

is specified. A modified termination decision flow is shown in Figure 90.

Figure 90: Proposed decision flow to disable transform domain prediction.

In the proposed decision flow, if NumValidC is equal to 1, the attribute prediction is disabled without the examination of other modules. Note that in this case, the value of NumValidP is default as zero in the specification since the parent neighbour search is skipped. This will cause that the predictions for its child level are forced to be terminated. To avoid the situation, NumValidP should be set to a value larger than TH1 (e.g., Val=19) in the proposed method when NumValidC =1.

## RAHT Reconstruction Buffer Rounding Operation Removal [129][130]

RAHT coefficients are computed using a FixedPoint object that stores data using int64_t values, with 15 bits for the fractional part. However, the reconstruction buffers store data as an integer (see code 3a). The reconstructed coefficients undergo a rounding operation prior to storage in this buffer, as shown in source code below, removing the fractional part and resulting in some loss of information.

Code 1: rounding operation for the reconstruction buffer

However, as both encoder and decoder have access to this same data, it is proposed to store this data in full precision. The buffer stores either int64_t or FixedPoint values, eliminating the need for rounding. It is proposed to use int64_t values, because there is no difference of using FixedPoint values as the latter also stores data as int64_t. In this fashion, the value of the reconstructed coefficient is copied after weight normalization:

Code 2: no rounding operation for the reconstruction buffer

Without the loss of information from the rounding, a better inter layer prediction is achieved. Memory usage for the reconstruction buffer is doubled from 4∙4∙numPoints∙numAttrs bytes to 8∙4∙numPoints∙numAttrs bytes, where numPoints is the number of points for a given slice.

a: Original

b: Modified

Code 3: reconstruction buffers

## 3D Quantization Matrix Signalling for RAHT Coefficient Coding [131][132]

In G-PCC Ed.1 layer-based QP offsets are signalled for RAHT coefficient coding. Here, signalling of 3D quantization matrices is proposed so that a QP offset can be applied to each AC coefficient of the transformed 2×2×2 residual. Note that the DC coefficient is inherited and thus there is no residual DC component that is signalled to the decoder. The following syntax and semantics are proposed:

| attribute_slice_header( ) { | Descriptor |
| --- | --- |
| ash_attr_parameter_set_id | ue(v) |
| ash_attr_sps_attr_idx | ue(v) |
| ash_attr_geom_slice_id | ue(v) |
| ash_attr_layer_QP_present_flag | ue(v) |
| If( ash_attr_layer_QP_present_flag ) { |  |
| num_layer = ( attribute_coding_type = = 1 ) ? raht_depth : num_detail_levels_minus1 + 1 |  |
| for( idx = 0; idx < num_layer; idx++ ) { |  |
| ash_attr_delta layer_QP_luma[idx] | se(v) |
| ash_attr_delta_layer_QP_chroma[idx] | se(v) |
| } |  |
| } |  |
| If(attribute_coding_type==1){ |  |
| ash_attr_ACcomp_QP_luma_present_flag | u(1) |
| ash_attr_ACcomp_QP_chroma_present_flag | u(1) |
| If( ash_attr_ACcomp_QP_luma_present_flag ){ |  |
| num_layer = raht_depth |  |
| for( idx = 0; idx < num_layer; idx++ ) { |  |
| ash_attr_ACcomp_ layer_QP_luma_present_flag[idx] | u(1) |
| if(ash_attr_ACcomp_layer_QP_luma_present_flag[idx]){ |  |
| for (compidx=0; compidx<7;compidx++){ |  |
| ash_attr_delta_ACcomp _layer_QP_luma[idx][compidx] | se(v) |
| } |  |
| } |  |
| If( ash_attr_ACcomp_QP_chroma_present_flag ){ |  |
| num_layer = raht_depth |  |
| for( idx = 0; idx < num_layer; idx++ ) { |  |
| ash_attr_ACcomp_ layer_QP_chroma_present_flag[idx] | u(1) |
| if(ash_attr_ACcomp_layer_QP_chroma_present_flag[idx]){ |  |
| for (compidx=0; compidx<7;compidx++){ |  |
| ash_attr_delta_ACcomp _layer_QP_chroma[idx][compidx] | se(v) |
| } |  |
| } |  |
| } |  |

ash_attr_ACcomp_QP_luma_present_flag: Indicates whether the delta QP for AC components are present for the luma channel

ash_attr_ACcomp_QP_chroma_present_flag: Indicates whether the delta QP for AC components are present for the chroma channel

ash_attr_ACcomp_layer_QP_luma_present_flag[idx]: Indicates whether the delta QP for AC components are present for the luma channel for a particular layer indexed by idx in octree

ash_attr_ACcomp_layer_QP_chroma_present_flag[idx]: Indicates whether the delta QP for AC components are present for the chroma channel for a particular layer indexed by idx in octree

ash_attr_delta_ACcomp_layer_QP_luma[idx][compidx]: Indicates the delta QP for AC component indexed by compidx for the luma channel for a particular layer indexed by idx in octree

ash_attr_delta_ACcomp_layer_QP_chroma[idx][compidx]: Indicates the delta QP for AC component indexed by compidx for the chroma channel for a particular layer indexed by idx in octree

The QP for each layer is first derived as per the current specification and is shown in Figure 91.

Figure 91: Operation of Delta Layer QP for RAHT.

Upon deriving QP for a layer ‘l’, the QP for each AC coefficient indexed by ‘i’, is derived as

QPl,i=QPl+∆QP(l,i);  i=0,1..6

where,  QPl,i is the final QP used for a layer l and an AC component index i. ∆QP(l,i) is the delta value that is obtained from the syntax elements presented before.

## Lossless RAHT Extension [134][135]

This proposal introduces integer Haar transform on top of the fixed-point RAHT design. The following is proposed:

- Introduce integer Haar transform and inverse Haar transform with flag to control it

- If integer Haar transform flag is equal to 1 then

  - skip normalization process with weight value

  - set QP = 4 (lossless setting)

  - conduct shift operation to remove the floating point precision

Figure 92 shows the fixed-point RAHT process and Figure 93 shows the implementation of integer Haar transform on top of fixed-point RAHT. In Figure 93, the yellow block and blue sentences are related to additional functions to introduce lossless coding with integer Haar transform.

Figure 92: Fixed-point RAHT and prediction process overview.

Figure 93: Integer Haar transform on top of fixed-point RAHT process.

## Cross-Attribute Prediction [136][137]

An attribute coding method is proposed [137] that uses the correlation between different attribute types (e.g., color and reflectance) based on the LoD structure for coding Am-fused sequences.

(1) First, a flag is signalled to indicate whether cross-attribute prediction is enabled. If so, a bit is further signalled to indicate the prediction relationship between two attributes. When it is true, the encoded reflectance information is used to assist the color prediction process; otherwise, the encoded color information is used to predict the reflectance.

(2) When coding the second attribute, for each point to be coded, the three predictors determined according to the LoD structure is reordered based on a so-called overall distance between the point and its three predictors. The overall distance is defined as the weighted combination taking into account both the geometry information and the first attribute information that have been coded. Specifically, the overall distance is calculated as:

overallDis = geomW·geomDis + attrW·attrDis

where attrDis is the absolute difference of the already-encoded attribute between the current point and its predictors, geomDis is the L1 distance between the point and its predictors, geomW and attrW represent the weight of the geometry distance and attribute difference respectively. In our current implementation, geomW is set to 1, attrW is determined as:

attrW = lambda·maxGeom / maxAttr

where maxGeom is the summation of the length, width and height of the slice bounding box, maxAttr is the possible maximum value of the already-encoded attribute, and lambda is a parameter balancing the importance of geometry and attribute.

Since a larger attribute quantization parameter results in more smoothed attribute, and thus less impact on the predictor reorder, lambda is simply treated as a function of attribute quantization parameter (denoted as AttrQP):

Lambda = -k·AttrQP + b

When using reflectance to predict color, k= 67>>15, b = 4011>>15; when using color to predict reflectance, k = 33>>12, b=1939>>12.

(3) The three predictors are sorted using the overall distance. The weight of each predictor is updated according to its overall distance. The following prediction and entropy coding modules remain the same as current G-PCC.

## Extend Context for Attribute Coefficient Entropy Coding [139]

In current G-PCC, the number of contexts for attribute coefficient is 3. However, for coding of

16-bit attribute it is proposed to increase the number of contexts from 3 to 12.

## Inter-Chroma Channel Attribute Prediction for RAHT [140]

It is proposed to exploit correlations between chroma channels when performing RAHT in TMC13 and GeS-TM.

Let CoeffCb and CoeffCr denote the RAHT coefficients of chroma Cb and Cr channels. The relationship between the RAHT transform coefficients of each 2*2*2 block (i.e., up to seven CoeffCb and CoeffCr) is simply modelled by the following linear function:

CoeffCr≅s∙CoeffCb

where s is a scalar determined per node according to previously M (fixed as 128 in the method) coded Cb and Cr coefficient of the same layer. Specifically, for the k-th node in level lvl, the fitting parameter slvl,k is calculated as:

slvl,k= ( i=max(0,k-1-M)k-1coeffCbilvl×coeffCrilvl)≪2i=max(0,k-1-M)k-1coeffCbi lvl2

where coeffCbilvl  and coeffCrilvl denote the reconstruction value of previously coded Cb and Cr in the same level. The value of s is clipped to the range of [-4, 4]. It should be noted that there is no need to signal s in the bitstream since it can be calculated from the coded information.

After the linear modelling of CoeffCr using CoeffCb, the coding of CoeffCr is replaced by coding its residual:

CoeffCr'=CoeffCr-s∙CoeffCbrecon≫2

## Restriction of Scale Values Used in Spherical Coordinate Conversion [75]

It is proposed to impose a restriction on the spherical coordinate conversion scale values to be powers of 2 (or reciprocals of powers of 2). This proposal is reported to reduce the runtime complexity of spherical coordinate conversion in some cases without affecting the coding efficiency.

## Sample Domain Prediction for RAHT [142][143] (TMC13, GeS-TM)

In G-PCC, prediction is performed in transform domain for RAHT attribute coding. This has the following problems: i) applying forward and inverse RAHT introduces distortion for prediction signal due to fixed-point computation and ii) leads to additional computational complexity. An investigation on sample domain prediction for TMC13 and GeS-TM reported complexity reduction with minor coding gain. Additionally, sample domain prediction opens door to skip the transform process altogether at the decoder, when all the AC residues of RAHT node are quantized to zero, leading to additional complexity.

The details of sample domain prediction are as follows:

Notation: X is original signal in sample domain, T, the RAHT transform, and Y is the transform signal. X' and X are prediction and reconstruction signals respectively.

At the encoder, residue (R) is computed as,

Ry=T(X-X')

and quantized to get  Ry, the quantized residue in RAHT domain.

At the decoder, sample domain reconstruction is obtained as,

X=X'+ T'(Ry)

It is claimed that performing prediction in sample domain is beneficial since i) it avoids precision loss and preserves all the information in the prediction signal and ii) it eliminates the need for forward RAHT at the decoder thereby achieving complexity reduction.

## Improved Intra-attribute Coding Method for RAHT [144]

When the current RAHT coding layer is eligible for intra-prediction, an additional non-prediction choice is proposed to code the current RAHT coding layer. A flag is coded to indicate if the current RAHT coding layer is coded using intra-prediction or non-prediction. On the encoder side, RDO is used to adaptively select the best coding mode.

In the current G-PCC, the coding follows a top-down scan order along the octree. When the current frame is eligible for intra-prediction, 19 possible neighbor parent nodes, and advanced coded neighbor nodes are used to form the prediction value for the target nodes (child nodes) of the center node.

Figure 94: Overview of the RAHT intra-prediction/non-prediction.

It is proposed to add another coding mode, non-prediction if the current RAHT coding layer is eligible for intra-prediction. Therefore, in the current intra-RAHT transform coding, the current RAHT node may be coded with either intra-prediction or non-prediction. As shown in Figure 94, a coding flag is added to indicate the intra-prediction or non-prediction mode for the current RAHT coding layer. As a result, all nodes in the current RAHT coding layer may be coded as intra-prediction or non-prediction. The node in the root level is not needed to select intra-prediction or non-prediction. On the encoder side, RDO is used to adaptively select the best coding mode.

The coding cost calculation formula is as follows:

RDCost=Rate+λ×D

where Rate is equal to the number of coding bits for the selected coding mode and D is defined as the sum of absolute differences (SAD) between the original signal and corresponding prediction, the value of λ set as follows:

λ=2QP-43×25         numAttrs==12QP-43×35          numAttrs==3

where QP is the quantization value. The codeMode is determined as follows and coded into the bitstream.

codeMode=1            intraPredCost≤nonPredCost0            intraPredCost>nonPredCost

## Inter-chroma Channel Prediction for RAHT [145] (GeS-TM)

To improve the attribute coding performance, inter-channel prediction technologies have been developed for LoD-based predlift coding scheme. In this proposal, correlations between chroma channels are explored when performing RAHT.

Let CoeffCb and CoeffCr denote the RAHT coefficients of chroma Cb and Cr channels. The relationship between the RAHT transform coefficients of each 2*2*2 block (i.e., up to seven CoeffCb and CoeffCr) is modelled by the following linear function:

CoeffCr≅s∙CoeffCb

where s is a scalar determined per node according to previously M (fixed as 128 in the method) coded Cb and Cr coefficient of the same layer. Specifically, for the k-th node in level lvl, the fitting parameter slvl,k is calculated as:

slvl,k=  i=max0,k-1-Mk-1coeffCbilvl×coeffCrilvl≪4i=max(0,k-1-M)k-1coeffCbi lvl2

where coeffCbilvl  and coeffCrilvl denote the reconstruction value of previously coded Cb and Cr in the same level. s is computed using approximate division with rounding and is clipped to the range of [-16, 16]. It should be noted that there is no need to signal s in the bitstream.

After the linear modelling of CoeffCr using CoeffCb, the coding of CoeffCr is replaced by coding its residual:

CoeffCr'=CoeffCr-s∙CoeffCbrecon≫4

## Context Reduction for RAHT Entropy Coding [146] (GeS-TM)

It is proposed to remove Size context since it is never used, which can reduce the number of contexts to 81 without impact on performance.

## RAHT Prediction Mode Signalling [147] (GeS-TM)

In the current RAHT coding in GeS-TM, the RAHT transform layers are classified into three groups, which are upper layers, middle layers and lower layers. Prediction strategies are different for different layer groups. Additionally, specific layers in the middle and lower layer are allowed for average prediction. One of three prediction modes, namely the orgCodingMethod, Inter-layerCodingMethod and Intra-layerCodingMethod, will be selected for each transform layer by an RDO method and the prediction mode with the minimum R-D cost will be signaled. Details of the prediction strategy are shown in Table below.

| Layer Prediction method for every node Prediction mode |  | orgCodingMethod | Inter-layerCodingMethod | Intra-layerCodingMethod |
| --- | --- | --- | --- | --- |
| Upper Layer |  | Inter->intra->null | Inter->intra->null | intra->null |
| Middle Layer | Disable average prediction | RDO among inter, intra and null | Inter->intra->null | intra->null |
|  | Enable average prediction | RDO among average, intra and null | average->intra->null | intra->null |
| Lower Layer | Enable average prediction | average-> null | average->intra->null | intra->null |
|  | Disable average prediction | intra->null | Inter->intra->null | intra->null |

First, one flag named layer_based_pred_mode_flag will be signalled, which indicates whether it is a layer-based prediction mode. Second, if the prediction mode is layer-based, i.e., layer_based_pred_mode_flag=1, another flag named layer_pred_mode will be signalled additionally, which indicates whether it is Inter-layerCodingMethod.

From the Table, it can be seen that the current prediction mode determination flow has two redundant cases:

First, when the transform layer is in the upper layer, the prediction method of orgCodingMethod and Inter-layerCodingMethod is the same. Specifically,

- if inter prediction is enabled for the current node, use inter prediction;

- Otherwise, if intra prediction is enabled, use intra prediction;

- Otherwise, use null prediction.

Second, when the transform layer is in the lower layer, the prediction method of orgCodingMethod and Intra-layerCodingMethod is the same, as highlighted in yellow. Specifically,

- if intra prediction is enabled for the current node, use intra prediction;

- Otherwise, use null prediction.

In the above two cases, the highlighted prediction modes have the same prediction results and the same RD cost, resulting in:

- signalling redundancy in prediction mode, and

- calculation redundancy in RD cost.

Therefore, it is proposed to remove the above redundancy. When the transform layer is in the upper layer, only calculate the cost for one of two prediction modes: orgCodingMethod and Inter-layerCodingMethod. Then,

- if the cost is less than the cost of Intra-layerCodingMethod, set layer_based_pred_mode_flag as 0;

- Otherwise, set layer_based_pred_mode_flag as 1, and there is no need to signal layer_pred_mode anymore.

When the transform layer is in the lower layer with average prediction disabled, only calculate the cost for one of two prediction modes: orgCodingMethod and Intra-layerCodingMethod. Then

- if the cost is less than the cost of Inter-layerCodingMethod, set layer_based_pred_mode_flag as 0;

- Otherwise, set layer_based_pred_mode_flag as 1, and there is no need to signal layer_pred_mode anymore.

The modified decision flow is given as below:

## Canonical Geometry Order-based Periodic Subsampling Method for LoD Generation [149]

The proposed LoD subsampling method samples points using sampling period in the canonical geometry order (saved before the Morton sorting process) for PredGeom.

In G-PCC Ed.1, the reconstructed position list from the geometry encoder/decoder is sorted by Morton code, and the sorted list is sampled using periodic sampling for LOD generation. However, since different parts of the sorted points may have different laserIDs, the sampling can be non-uniform as illustrated in Figure 95. A non-uniformly sampled LOD structure may decrease the prediction performance for the PredLift attribute coding.

Figure 95: An example of Morton subsampling results (green: sampled points).

To address this issue, a sampling method based on canonical order is proposed. The proposed method uses canonical order indices instead of Morton code order indices for LoD sampling without the need for reordering. The proposed method is applied when canonical_lod_sampling_enabled is true. The canonical_lod_sampling_enabled flag is implicitly set to true when geom_tree_type is equal to 1 (predictive tree) and attr_canonical_order_enabled is equal to false in both the encoder and decoder.

Figure 96 shows a comparison of subsampled results using periodic and the proposed sampling method. As illustrated, the sampled points (shown in green) are more uniformly distributed using our proposed method compared to periodic sampling.

Figure 96: Comparison of subsampled results using Morton and the proposed subsampling.

## RAHT Code Optimizations [150] (GeS-TM)

Faster compute

- replaced branching by arithmetic

- faster sqrt, isqrt

- faster types (int or int64 for local arithmetic)

- removed/replaced slow functions/operations  (like ilog2)

- templatization for haar/raht and numAtrr

- removed FixedPoint and VecAttr

- stop computing things not needed by the decoder

- avoid recomputing

Compact memory

- changed types to smaller types

- struct RAHT node decoder is lighter than struct encoder

- replaced iterators (64 bits pointers on 64 bits architectures) by indices (32 bits)

Minimize memory traffic

- removed copies, replaced by references

- struct decoder lighter than struct encoder

- local determination of parents’ modes

- removes a pass in descending tree

- well-sized and timely memory reserve

## Cross-component Residual Prediction [151] (GeS-TM)

Luma to chroma channel prediction is proposed where both Cb and Cr residues are predicted from luma residue (ry) as follows:

rcb=αcbry ,  rcr=αcrry

where, rcb and rcr are the predicted chroma residue values and αcb and αcr are the prediction coefficients.

# TriSoup

## Alternative Method for Determining Projection Plane [152][153]

In Trisoup, the projection plane is determined by computing the variance of vertices positions for each axis. The proposed method uses maximum value and minimum value of vertices positions for each axis instead of the variance.

The proposed method is illustrated in the following code:

// Alternative method to determin dominant axis.

Vec3<int32_t> min_pos = leafVertices[0].pos;

Vec3<int32_t> max_pos = leafVertices[0].pos;

for (int j = 1; j < leafVertices.size(); j++) {

for (int axis_id = 0; axis_id < 3; axis_id++) {

if (leafVertices[j].pos[axis_id] > max_pos[axis_id]){

max_pos[axis_id] = leafVertices[j].pos[axis_id];

} else if (leafVertices[j].pos[axis_id] < min_pos[axis_id]){

min_pos[axis_id] = leafVertices[j].pos[axis_id];

}

}

}

Vec3<int32_t> diff_max_min = max_pos - min_pos;

int32_t min_diff = diff_max_min[0];

int32_t dominantAxis = 0;

for (int axis_id = 1; axis_id < 3; axis_id++) {

if (diff_max_min[axis_id] < min_diff){

min_diff = diff_max_min[axis_id];

dominantAxis = axis_id;

}

}

## Refinement of Trisoup Projection Plane Determination [154]

Visual “holes“ on reconstructed point cloud were observed. The cause of the problem is the projection plane determination, which is a part of Trsoup decoding process. Therefore, the following method proposes a refined projection plane determination process.

Figure 97 shows the current projection plane determination process in G-PCC Ed.1. The current implementation calculates the difference between max coordinate value and min coordinate value accroding to each axis, and then the projection plane is determined by ignoring an axis which has the smallest difference. In other words, the projection plane is defined by axes which have larger difference. In the example, two or more axes have the minimum difference value, projection plane is determined by ignoring an axis according to predefined priority as x  -> y -> z. The current implementation does not work well in the case that two or more axes have the minimum difference.

Figure 98 shows an example case that the current implementation does not work. In this case, differences between the max coordinate value and the min coordinate value for x, y, and z axis are identical (=N). In this case, projection plane is determined by ignoring x axis (i.e., y-z plane). However, the x-y plane or x-z plane is better than y-z plane as projection plane. As written in above, the current implementation sometimes chooses unsuitable projection plane and it is the cause of malformed node and holes.

The problem of the current implementation is that it evaluates only 1-dimentional spread of the vertices. However, desirable behavior of the process is that the process evaluates 2-dimensional spread of the vertices.

Figure 97: The current projection plane determination process.

Figure 98: An example case that the current projection plane determination does not work well.

The proposed method chooses a projection plane which has the largest area of polygon defined by projected vertices. The area can be calculated as the sum of small triangles as shown in Figure 99.

Figure 99: Proposed method.

## Improved TriSoup [155][156][157][158][159][160][161][162]

Many changes are proposed to the so-called TriSoup technology. These changes are not independent from each other and are presented in seven input contributions:

- m59288 Part 1 - Improving TriSoup:  summary, results and perspective

- m59289 Part 2 - Fixes and simplifications

- m59290 Part 3 - Adding a centroid vertex

- m59291 Part 4 - Vertex Quantization

- m59292 Part 5 - Improved rendering from triangles

- m59293 Part 6 - Compression of vertex presence flag and vertex position

- m59294 Part 7 - Latest improvements

Description needed

## Adaptive Halo Improvement for Voxelization [163]

It is proposed to create an adaptive halo around the TriSoup triangles by making the halo parameter ε depend on the sampling distance of point cloud data. By doing so, the sizes of the triangles are adjusted with point sampling distance of different point cloud data such that ray tracing will intersect the adjusted triangles and miss less points.

The proposed adaptive halo parameter εa>0 can be obtained by εa=ε*d, where the sampling distance can be estimated according to the total number of input points, the number of leaf nodes and the size N of leaf node before the voxelization process, and it can also be obtained by looping method to select a best sampling distance during the voxelization process.

After getting the triangles by the determined vertices for each leaf node, the proposed method also uses the Möller-Trumbore algorithm to get decoded points from the obtained triangles. In the Möller-Trumbore algorithm to determine each intersection point, the proposed adaptive halo parameter εa is used in the relaxing condition. The calculated values of u,v,w must satisfy the conditions u,v,w≥-εa. Other methods are the same as those in latest Trisoup coding method.

The halo tool can be enabled on a per sequence basis and in addition, the trisoup_adaptive_halo_flag is introduced in gps. At the encoder, if the flag is true, it uses adaptive halo method to extend triangles; otherwise, it uses fixed halo method in current Trisoup coding to extend triangles, and the flag is coded into bitstream at encoder. At decoder, it decodes the flag from the bitstream, and if the flag is true, it uses the adaptive halo method to extend triangles; otherwise, it uses fixed halo method in current trisoup coding to extend triangles, and then to reconstruct point cloud by ray tracing method.

## Non-Cubic TriSoup [166][167]

In point clouds processed by TriSoup on TMC13v19, there may be visual gaps on the slice boundaries. This is because while the leaf node of TriSoup is limited to a cube, there is no restriction on the origin and width of the slice bounding box, so it was impossible to match the slice boundary with the leaf node boundary. This caused a visual gap because vertices could not be created on the slice boundary.

Figure 100: Visual gaps of reconstructed point clouds.

This method resolves visual gaps between slices by making the leaf nodes at the end of a slice non-cubic, thereby creating vertices at the boundary of the slice. This eliminates the need to consider slice partitioning.

Figure 101: Non-cubic node.

## Voxelization of TriSoup Triangles [168]

The current overall process of the voxelization of triangles follows these steps for each TriSoup node:

- voxelization of TriSoup vertices

- voxelization of the unique TriSoup centroid

- voxelization of each TriSoup triangle belonging to the TriSoup node

  - by ray launch along two axes the most perpendicular to the triangle

  - using halo

  - using fine ray launch

- eliminating duplicated voxels

The current voxelization process is known to have several drawbacks. Firstly, each TriSoup vertex is voxelized up to 4 times. This increases the number of rendered points and consequently it increases the attribute bitrate. It also leads to visual problems when TriSoup vertices are quantized at lower bit rates. Secondly, voxelization parameters are fuzzy. They have been set around an overall sweet spot but have led to big gains on some sequences, loss on others. It is still an open question what the optimal halo parameter is. It seems to be largely content and rate point dependent. Also, the optimal distance of fine ray launch seems to be 1/8. It is not understood why. Thirdly, fine ray launch has heavily increased the complexity of the voxelization process. It is thus proposed to modify to voxelization process to tackle the problems above.

Further description needed

## Inter TriSoup Based on Motion Compensated Point Cloud [169]

The inter codec of EE13.60 is based on the merge of the historical inter ETM and the current GPCC TMv20. A 3D motion field is obtained from the octree coding method, and the reference point cloud is compensated to obtain inter contextual information for coding the octree. However, no usage of the reference and/or the compensated point cloud has been attempted yet to improve the coding of TriSoup information. It is thus proposed to use the compensated point cloud to build contextual information that drives the entropy coding of presence and position of TriSoup vertices as well as the value the centroid residuals.

Further description needed

## On TriSoup Voxelization [170]

Description requested

## Centroid Vertex Positioning [171]

Modified method is proposed to improve the determination of the centroid vertex in the TriSoup process.

Description requested

## Raster Scan Order for Octree Coding (GeS-TM) [172][173]

The current design of G-PCC uses Morton code ordering of the node positions for coding the occupancy of every node at a given depth in octree geometry coding, which is a different ordering than the ordering used for the edges of the octree leaf nodes within TriSoup. It requires some reordering between the octree layer and the TriSoup layer. The ordering used in TriSoup is lexicographic order of coordinates (a.k.a “raster scan” order). Therefore, it is proposed to replace the Morton coding order of nodes occupancy by a lexicographic order to unify octree and TriSoup processing order.

The proposed lexicographic order is not a fully lexicographic order in terms of the order of the coded occupancy bits. It is a lexicographic order in terms of coding order of the parents’ nodes. The child occupancy of each node is coded in Morton order, but the corresponding child nodes are processed in lexicographic order at next depth. Thus, most of the existing code/spec for coding the occupancy bits of a given node can be kept. This process is illustrated in Figure 102. For simplicity the illustration is made on a dense point cloud that would be made of fully occupied nodes and is described below.

Without considering QTBT for the sake of simplicity, at depth d, each node with coordinates (x, y, z) may have up to 8 child nodes with 1/8th of the volume their parent node, and with coordinates:

- (x, y, z) for child 0,

- (x, y, z+2d-1) for child 1,

- (x, y+2d-1, z) for child 2,

- (x, y+2d-1, z+2d-1) for child 3,

- (x+2d-1, y, z) for child 4,

- (x+2d-1, y, z+2d-1) for child 5,

- (x+2d-1, y+2d-1, z) for child 6, and

- (x+2d-1, y+2d-1, z+2d-1) for child 7.

In the following, we consider:

- a tube of nodes as being made of the nodes with same x and y, and

- a slice of nodes as being made of the nodes with same x.

Figure 102: Raster scan ordered parents nodes with 8 child occupancy bits coding and raster scan ordering of the child nodes.

In current implementation, the 8 children occupancy is coded for every parent node (Figure 102-(a)) in a same tube of parent nodes (Figure 102-(b)). These nodes are successive nodes in a lexicographic ordered buffer of nodes for the current depth of the tree. For each one of these nodes, the occupied children belonging to tube 0 (Figure 102-(d)) of children nodes are appended at the end of a lexicographic ordered buffer of nodes for the next depth of the tree, to obtain the tube 0 of child nodes in Figure 102-(e). Then, all the same nodes belonging to the same tube of parent nodes are processed a second time to append the children belonging to tube 1 (Figure 102-(d)) to the buffer for the next depth, and to obtain the tube 1 of child nodes in Figure 102-(e). This process is repeated for every tube of parent nodes belonging to a same slice of parent nodes (Figure 102-(c)) to code their occupancy and obtain the slice 0 child nodes in the buffer of nodes for next depth. Then, all the same nodes belonging to the same slice of parent nodes are processed a third and a fourth time to append the child nodes belonging to each one of tube 2 and then tube 3 (in same order as for tube 0 and 1). The buffer of child nodes then becomes the buffer of parent nodes for the next depth, and it is ordered in lexicographic order by construction.

One advantage of using lexicographic order is shown in Figure 103 illustrating an example of the occupancy map (or causal neighborhood) of child occupancy in lexicographic order. For every parent node, the same causal neighborhood will be available.

Figure 103: Occupancy map (white cubes) known from previously encoded or decoded occupancy bits when encoding or decoding occupancy bit of a child node (striped cube) using proposed lexicographic order.

Also, to access occupancy of neighboring nodes, the atlas is not needed anymore. It is possible to simply maintain a table of pointers in the buffer of child and/or parent nodes to access the neighboring node’s information directly. This table can simply be maintained by incrementing pointers in the buffer, and the overall complexity for maintaining these pointers is O(N) for the N nodes of a given depth (average complexity for maintaining a pointers to get neighbor occupancy of one node is Θ(1)).

## TriSoup Node Grid Alignment (GeS-TM) [174][175]

Non-alignment of TriSoup node grid impacts quality of inter predictor and is non-suited for Skip mode. It is proposed to ensure normatively that skip mode is not used without grid alignment.

## Motion-Compensated Inter-Frame Attribute Coding Scheme for Dynamic Solid Point Clouds (GeS-TM) [177][178]

This contribution proposes a coding scheme with inter-frame prediction based on local motion compensation for RAHT attributes encoding.

### Overview

The proposed scheme is built on top of the current intra-frame RAHT encoding scheme in GeS-TM. It leverages the local motion field already present in the geometry encoder to generate a temporally predicted point cloud frame attribute component, thus providing additional inter-frame predictors to the hierarchical RAHT encoding process.

The best prediction mode for each node is decided among three candidates: no prediction, intra-frame prediction and inter-frame prediction. The choice is made on a rate-distortion criterion balancing the minimization of the prediction error and the coding cost of the residues and prediction modes (as prediction modes are now arithmetically encoded and transmitted to the decoder). This prediction decision process is repeated at each resolution level except at the last levels of highest resolutions, where a simple inference is performed at both encoder and decoder, to avoid a prohibitive transmission cost of prediction modes. The level at each we stop encoding the prediction mode and start inferring is a parameter chosen by the user.

Figure 104: Overview of the attribute encoder.

In Figure 104 a block-scheme of the encoder is depicted, with modified or additional processing blocks and new data flows colored green.

### Motion Compensation

The block-matching based local motion estimation already implemented in the geometry encoder of the GeS-TM is re-used as it is. The associated motion compensation module is modified to add to the output prediction point cloud frame PCtp and attribute component Atp. A 32-frame long GOP structure is used, with 31 frames inbetween two successive intra-coded frames being encoded with the proposed scheme with inter-prediction.

### Prediction Mode Decision

Three prediction modes are in competition for encoding a RAHT node:

- Null mode: direct coding of RAHT coefficients

- Intra mode: intra-frame transform domain RAHT prediction

- Inter mode: attribute values at same location of current node in temporally predicted point cloud (averaged in accordance with the size of the node at current level)

The decision is based on a classical rate-distortion criterion, by minimizing cost function MSE + λRate, where MSE is the mean squared error of the reconstructed attributes and Rate is the average number of bits per child to encode the difference of quantized coefficients (the prediction residue) and to signal the prediction mode.

### Prediction Mode Signalling

The choice of prediction mode is modelled using the binary tree depicted in Figure 105 where each leaf corresponds to one of the 3 prediction modes. At each tree node the encoder passes through the decision process, the followed direction is signalled with a binary flag that is encoded using binary arithmetic coding with a given probability for the flag to be true or false. The probabilities of nodes 1 and 2, isNull[ctx] and isIntra[ctx] respectively, are adaptive and depend on a context ctx. They are updated on-the-fly during encoding process.

Figure 105: Binary tree for prediction mode decision.

There are in total 216 such context by combining the following set of information available at both the encoder and decoder:

- enabling intra prediction flag (from current RAHT encoding process),

- availability of temporal predictors for all the children,

- prediction mode used by parent,

- majority prediction mode among neighbouring uncles and already decoded cousins,

- number of children,

- MSE between the inter-predicted and parent node DC value compared to a threshold.

When the enabling intra prediction flag is false and not all node children have a temporal prediction, no binary decision is made and no prediction is performed.

In [178] the number of contexts used to encode the prediction mode was reduced from 216 to 108.

### Prediction Mode Inference

The prediction mode is not signalled for the highest resolution levels close to the leaf nodes, which would become prohibitive in terms of transmission cost, but inferred from available information at both encoder and decoder. An example of such signalling vs. inference split is illustrated in Figure 106.

Figure 106: Prediction mode signalling vs. inference at last 2 levels.

The enabling intra prediction flag already implemented within the current RAHT process is used to decide between the Null and Intra prediction modes (no Inter prediction is performed at those highest resolution levels).

## Harmonized Inter Prediction for RAHT (GeS-TM) [179]

Below figure shows the flow chart of harmonized prediction method for RAHT layer.

Figure 107: Harmonized method.

Description requested.

## Face Vertex for TriSoup [180][181]

### Problem Description

In the current Trisoup, when the undulations of the original point cloud surface exist continuously as ridges across the nodes, there is a problem that the ridges cannot be represented at the node boundaries and dents are occurred between the nodes. From

Figure 108 and Figure 109, there are examples of the point clouds reconstructed by current TriSoup in TMC13v22. In Figure 108, the contour of leg is not represented causing unevenness, and in Figure 109 the edges of the wall are uneven.

Figure 108: Reconstructed point cloud by TMC13v22.

Figure 109: Reconstructed point cloud by TMC13v22.

This is because the current TriSoup cannot create vertices on the face of the node, as shown in Figure 110, when the ridge line of the original point cloud surface passes through the face of the node, the centroid is reconstructed at the ridge line position correctly, on the other hand, on the node surface, there is no method to reconstruct the ridge line shape, so original point cloud surface cannot be reconstructed, this causes the volume of the boundary of the node to be missing, resulting in such reconstructed surfaces.

Figure 110: Mechanism of surface unevenness.

### Proposed Method

To solve this problem, a method of vertex creation, transmission, and reconstruction is proposed that can reconstruct the ridge shape between nodes. This method is called “face vertex”.

### Generation of Face Vertex

The connection between centroids with neighbouring nodes is determined by encoder.

To make a judgment without overlapping between faces, the face shared with the three-neighbouring node with the larger coordinate is instantiated. For each of instantiated faces, if the weight of the points near the intersection of the line segment connecting the centroids and this face is more than a certain value, it is judged that the face vertex is generated on this face. The result of this judgment is stored to each face instance as boolean.

Figure 111: Instantiation of faces.

Figure 112: Generation of face vertex.

### Transmission Condition of Face Values

To reconstruct TriSoup surface at the decoder, the arithmetically encoded information of octree, edge vertex, centroid, and face vertex is transmitted. When transmitting each face values from encoder, the number of faces to be transmitted is reduced by applying some conditions by decoder-known information, by determining whether a face vertex can be generated on the face from geometrical relationship between the edge vertices and centroids. This eligibility judgment is made for each face instance based on the AND of the following five conditions.

These conditions are shared with the decoder:

1. The current node has a refined centroid.

2. The current node has a neighboring node.

3. The neighboring node has a refined centroid.

4. The number of edge vertices on the shared face is two or three.

5. The reconstructed surface inflates by the face vertex. This condition is determined by the signs of the inner products between three vectors. (Figure 113)

Figure 113: Relationships among nodes, vertices, and the face in the fifth condition.

In practice, for the faces that meet these five conditions, as described above, the boolean of the face is determined to be true or false based on the point cloud distribution as described in Figure 112. These face values are arithmetically encoded and transmitted to the decoder.

### Reconstruction

The decoder receives information of the faces that match the above five conditions in addition to octree-nodes, edge vertices, and centroids. Next, the decoder generates face instances for each neighbouring node as in Figure 112 and it determines the five conditions for each face in the same way as the encoder and obtains the boolean for the faces that match those conditions from the arithmetic decoder.

Once the whole face values are determined, for each face that has a “true” value, the intersection of the face and the line segment between the two centroids at both ends is reconstructed as a face vertex. To reconstruct the trisoup surface, for each node, the edge vertices and face vertices are ordered counterclockwise around the centroid and the triangles are reconstructed, then point cloud is reconstructed by raytracing.

Figure 114: flowchart of determination for transmission of faces and reconstruction with face vertex.

### Syntax

The function on/off flag of this method is defined and stored in Geometry Data Unit Header:

| geometry_parameter_set( ) { | Descriptor |
| --- | --- |
| gps_geom_parameter_set_id | u(4) |
| gps_seq_parameter_set_id | u(4) |
| slice_geom_origin_scale_present | u(1) |
| if( ¬slice_geom_origin_scale_present ) |  |
| gps_geom_origin_log2_scale | ue(v) |
| ： |  |
| trisoup_enabled_flag | ue(v) |
| ： |  |
| if( trisoup_enabled_flag ) { |  |
| If(trisoup_centroid_vertex_residual_flag ) { |  |
| trisoup_face_vertex_flag | u(1) |
| } |  |
| } |  |

Geometry Parameter Set semantics

trisoup_face_vertex_flag  specifies whether the face vertex method is performed.

Syntax changes on Geometry Data Unit Data:

| geometry_data_unit_data( ) { | Descriptor |
| --- | --- |
| : |  |
| : |  |
| if( ! ( partitionSkip & 4 ) ) |  |
| depthS = depthS + 1 |  |
| if( ! ( partitionSkip & 2 ) ) |  |
| depthT = depthT + 1 |  |
| if( ! ( partitionSkip & 1 ) ) |  |
| depthV = depthV + 1 |  |
| } |  |
| if( log2_trisoup_node_size > 0 ) |  |
| geometry_trisoup_data( ) |  |
| } |  |

| geometry_trisoup_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= num_unique_segments_minus1; i++ ){ |  |
| segment_indicator[ i ] | ae(v) |
| if( segment_indicator[ i ] ) |  |
| vertex_position[ i ] | ae(v) |
| } |  |
| for( i = 0; i < num_leaf_nodes; i++ ){ |  |
| if ( CentroidEligibleNode[i] ) |  |
| quantized_drift | ae(v) |
| (tentative) } |  |
| if( trisoup_centroid_vertex_residual_flag && trisoup_face_vertex_flag ) { |  |
| for( i = 0; i < num_leaf_nodes; i++ ){ |  |
| for( j = 0; j < 3; j++ ){ |  |
| if( FaceEligible[i*3+j] ){ |  |
| has_face_vretex | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Geometry data unit data semantics

num_leaf_nodes specifies the number of leaf nodes.

The eligibility state of all face instances are updated at the start of generating face vertices.

FaceEligible[k] means the eligibility of k-th face instance, and it is specified by the AND of the five conditions described before.

## Inter Skip Mode (GeS-TM) [182]

A new inter coding mode for the GeS-TM is proposed as a first step toward a “skip mode” for the coding of the geometry of moving point clouds. It is based on the colocated TriSoup information from a reference point cloud to code a current TriSoup information in a current frame.

Additional description requested.

## RAHT Mode Selection (GeS-TM) [183]

In this contribution, the RAHT transform layers are classified as upper layers, middle layers, and lower layers. Different RAHT prediction modes are determined for nodes in different layers. The decision flow is shown in Figure 115. Two parameters, namely

- rahtPredUpperLayer indicating the number of layers from the root, (For example, rahtPredUpperLayer = 3 indicates that the first three layers are upper layer) and

- rahtPredLowerLayer indicating the number of layers from the leaf. (For example, rahtPredLowerLayer = 2 indicates that the last two layers are lower layer)

are used for RAHT layer classification. Note that the summation of the two parameters should not be larger than the maximum number of layers.

Figure 115: The proposed prediction mode decision flow of RAHT.

## Face Vertex for GeS-TM [185]

Face Vertex for TriSoup in TMC13 is described in section 10.14. These are the differences for GeS-TM:

- Neighbouring node search

- Inter prediction

Because raster scan order is adopted in GeS-TM, the already-coded neighbouring node index is known. Thus, neighbouring node search is not needed. In TMC13, the face shared with the three-neighbouring node with the larger coordinate is evaluated whether it has a face vertex, while in GeS-TM, the face shared with the three-neighbouring node with the smaller coordinate is evaluated. The probability of each eligible face value sufficiently high to saturate the arithmetic coding performance. The inter prediction of face vertex was evaluated, but compression ratio was degraded. Therefore, inter prediction of face vertex is omitted.

## Initialization of Dynamic OBUF LUT for Octree Coding (TriSoup in TMC13) [43]

In TMC13, dynamic OBUF is used to select more efficient binary coders for coding occupancy bits of octree node, and the inputs to OBUF LUT are two contextual information (ctx1,ctx2), which are obtained by dividing the context information of neighbour node configurations into 2 parts (ctx1,ctx2), wherein the ctx1 is a primary part of the context information, and the ctx2 is the secondary part of the context information. And after coding an occupancy bit the LUT will be updated.

In current TMC13 octree coding, the dynamic OBUF LUT of size ctx1 is initialized to be 127 for all elements, which makes the LUT point to binary coders with probability 0.5 at initial state of the OBUF instance for octree coding. However, for occupancy bits coding of non-sparse octree nodes, there are some prior information of ctx1 information that can be used to help LUT to update to converge efficiently, and a re-initialization of dynamic OBUF LUT for occupancy coding of non-sparse octree nodes is needed, and this is already done in Ges-TM.

It is proposed to add a re-initialization process to dynamic OBUF LUT and set an initialization table for OBUF LUT used in coding each occupancy bit referring to Ges-TM. Examples of the initialization tables for coding b0 and b2 are shown below:

## Fix for Visual Gap at Slice Boundary [186]

It was observed that there are still visual gaps on the slice boundary under low rates as shown in Figure 118. Two reasons were identified that result in visual gaps on the slice boundary:

- One reason is slice origin determination in slice partitioning. In current G-PCC, the point cloud will be divided into slices aligned with the TriSoup boundary. Then compute to get the minimum coordinate in all points to obtain the slice origin for each slice. For each slice, the TriSoup node will be formed according to its slice origin. The TriSoup nodes in different slices may not still be aligned because the node for each slice will be formed according to its slice origin but the origins of different slices may not be aligned. The adjacent two edges located on different sides of the slice boundary may have different vertices because different points are used to compute their vertex positions as shown in Figure 116.

Slice boundary

Figure 116: Problem caused by misalignment of slice origin.

- The other reason resulting in visual gap comes from slice padding in Overlapping Slices. The No-Cubic technology will resize the TriSoup nodes, including padding nodes from neighbouring slice, on slice boundary according to slice bounding box. The padding node will cause two problems after being resized. The first problem is shown in the left part of  Figure 117. The edge A and edge B will overlap after the padding node is resized, so the points in padding node will be counted twice. It will result in inconsistent points used to calculate positions of vertex located on different sides of the slice boundary. The second problem is shown in the right part of Figure 117. Some points will be located outside the padding node after being resized, but they are still used to compute the edge vertex position. This will also cause the vertex lies outside the edge, leading to program crashes.

Slice bounding box

Edge A overlap with edge B after resizing

Edge A

Edge B

Padding node

Slice bounding box

Anomalous points

Padding node

Figure 117: Problems caused by slice padding and No-Cubic technology.

tmc13v24rc1, R01

tmc13v24rc1, R02

tmc13v24rc1, R03

tmc13v24rc1, R04

Figure 118: Reconstructed point cloud by tmc13v24rc1(basketball_player_vox11_00000200).

The proposed fix consists of two parts:

- Align the origins of different slices based on TriSoup node boundary

sliceOrigin=sliceOrigin-sliceOrigin% trisoupNodeSize

- Fix padding problem

  - Count the points in the padding node only once

  - Remove the anomalous points in padding node after being resized

Slice bounding box

Only consider edge B after resizing

Edge A

Edge B

Padding node

Slice bounding box

Anomalous points

Padding node

Figure 119: Proposed fixes.

The signalling bits of the slice origins can be saved because of the slice origins have been aligned according to TriSoup node boundary. In current G-PCC, the slice origin will be scaled by geomBoxLog2Scale before being signalled. We can revise the geomBoxLog2Scale according to the trisoupNodeSizeLog2 to save the signalling bits of the slice origins.

geomBoxLog2Scale=max⁡(geomBoxLog2Scale, trisoupNodeSizeLog2)

The subjective quality is shown in Figure 120.

tmc13v24rc1, R01

tmc13v24rc1, R02

Proposed, R01

Proposed, R02

Figure 120: Reconstructed point cloud by proposed method (basketball_player_vox11_00000200).

There are differences of reconstructed surfaces between anchor and proposed method inside slices, as shown the regions covered by the yellow circles. This is because the shift of the slice origin, which causes the TriSoup nodes to cover different regions.

## Reconstructing Non-Closed Surfaces in TriSoup Nodes [187] (TMC13 and GeS-TM)

The problem addressed in this proposal is that non-closed surfaces, that is surfaces that only partially intersect the node, are not reconstructed. In Figure 121 for example, there is a surface that is only partially intersecting the node and hence, only two edge-vertices are detected in the encoder. As a result of this, nothing within this node will be reconstructed.

Figure 121: An example of a non-closed surface that is only partially intersecting a node, resulting in there only being two edge-vertices.

The intuitive way of representing that surface using existing vertex-types is illustrated in Figure 122. This is also how the representation is implemented in the proposed solution. However, to represent a surface like this without introducing a new syntax, certain tools must be modified to fit a new reconstruction process, for nodes with only two edge-vertices.

Figure 122: An example of how to represent a non-closed surface using edge-, centroid- and face-vertices.

## Improvement of TriSoup Surface Reconstruction [188] (TMC13 and GeS-TM)

In the current TriSoup design, all original points that are within a predefined distance to one edge can produce one vertex on that edge. If the surface passes through a corner of the node, as shown in Figure 123 (a), it may result in more than one vertices (e.g., V1, V2, and V3) generated for the three edges intersecting that corner. Once these vertices are connected to the centroid vertex (e.g., Vc) to reconstruct the surface, an undesired fold (e.g., VcV2 as shown in Figure 123 (b)) is produced in the surface. This leads to significant visual artifacts in the reconstructed point cloud.

Figure 123: (a) original points and the generated vertices for one Trisoup node; (b) triangles reconstructed by connecting vertices; (c) reconstructed point cloud.

To remove this visual artifact in that case, one vertex instead of multiple vertices should be generated. A method to detect such case is therefore proposed, together with a method to determine the position of the modified vertex.

## Triangle Voxelization Process Simplification [189] (TMC13 and GeS-TM)

In current rendering method of TriSoup, ray tracing along one of the three axis is performed to voxelize the triangle. The well known Möller-Trumbore algorithm is used to perform the ray tracing, which consists of many cross products and dot products. It is proposed to determine whether the ray and the triangle intersect in 2D projection plane instead of 3D space, which converts 3D products into 2D products. Secondly, the calculation of the intersection point coordinate is simplified.

## Versatile Geometry Quantization of TriSoup [190] (GeS-TM)

The TriSoup node size is a power of two and is signalled in the HLS by trisoupNodeSizeLog2. In the CTC, the size is 4x4x4 for r04, 8x8x8 for r03, 16x6x16 for r02 and 32x32x32 for r01. This size must be uniform for all TriSoup nodes of a slice. The size of the vertex quantization interval on edges of TriSoup nodes is also a power of two that is signalled by trisoupQuantizationBits. It is proposed to provide more versatility to the quantization of triangle vertices by introducing a TriSoup QP parameter trisoupQuantizationQP that replaces number of bits trisoupQuantizationBits. The size of quantization intervals is obtained based on TriSoup QP that may not be a power of two.

## TriSoup Node Sizes Non-powers of Two [191] (GeS-TM)

It is proposed to extend the possible TriSoup node sizes to any integer greater than or equal to 4. A technique named “dicing” of point cloud is used to obtain a simple implementation of this extension without significant increase in complexity.

Additional description requested.

## On the OBUF Scheme for TriSoup Vertex Presence Flag [192] (GeS-TM)

The current TriSoup vertex coding pipeline in GeS-TM is illustrated in Figure 124. In the process of coding the vertex presence flag for each unique edge, the inter-frame information and the intra-frame information are jointly fed into the dynamic OBUF. Then, an appropriate coder is derived from three groups of coders, as shown in Figure 125.

Figure 124: Flowchart of TriSoup vertex encoding process.

Figure 125: OBUF scheme of TriSoup vertex presence flag.

In this proposal, the inter and intra contexts are reorganized in a different way as shown in Figure 126. Compared with the existing scheme in GeS-TM, following modifications are made:

- Eliminating the two inter bits (representing three states: NoPred, Pred0 and Pred1) among the secondary information bits for TriSoup inter prediction. As a result, the number of inter states is reduced from 224 to 222 for coding each vertex presence flag.

- Feed the three groups of state to seven coder groups according to the inter-frame information (i.e., NoPred, Pred0 and Pred1).

Figure 126: Flowchart of combining inter and intra contexts.

## Speedup of Triangle Soup Ray Tracing [193] (TMC13)

The speedup of the ray tracing is achieved using two methods. Firstly, the intersection condition is computed in 2D projection plane instead of 3D space. In the calculation process of Möller-Trumbore, u, v and w will be compared with halo parameter ε to judge whether the ray and the triangle intersect. Their calculation consists of many 3D cross products and dot products. It is determined whether the ray and the triangle intersect in 2D projection plane instead of 3D space. Firstly, the halo parameters ε will be converted from 3D to 2D:

ε2D=ε×S∆A'B'C'

(A,w)

(B,u)

(C,v)

Pstart

Pint

bounding box

A'

C'

B'

Figure 127: Halo in projection plane.

Then it is detemined whether the ray and the triangle intersect in projection plane. The “inside-outside” technique judging whether a point is inside a triangle will be used:

PstartA'×A'B'<ε2D

PstartB'×B'C'<ε2D

PstartA'×C'A'<ε2D

Then the 3D vector dot products and cross products will be converted into simpler 2D vector cross product.

| Operation | Addition and subtraction | Multiplication | Division | Comparison |
| --- | --- | --- | --- | --- |
| 3D (G-PCC) | 7 | 9 | 2 | 3 |
| 2D (Proposed) | 1 or 2 or 3 | 2 or 4 or 6 | 0 | 1 or 2 or 3 |

Secondly, the calculation of the intersection point coordinate is simplified. tx,y=e2∙s×e1a= c0x+ c1y+c2, is a linear function of projection plane coordinate x,y, where c0, c1 and c2 are the constant coefficients just related with triangle, which can be precomputed. Then tx,y by tx,y= tx-1,y + c0, tx,y= tx,y-1 + c1 or tx,y=c0x+ c1y+c2. Specifically, tx,y can be calculated as follows:

- If tx-1,y has been calculated, tx,y= tx-1,y + c0;

- Otherwise, if tx,y-1 has been calculated, tx,y= tx,y-1 + c1

- Otherwise, tx,y=c0x+ c1y+c2

Then the 3D vector dot product and cross product will be converted into simpler scalar addition and multiplication.

| Operation | Addition and subtraction | Multiplication | Division | Comparison |
| --- | --- | --- | --- | --- |
| G-PCC | 6 | 9 | 1 | 0 |
| Proposed | 1 or 2 | 0 or 2 | 0 | 1 or 2 |

## Local Quality Units for GeS-TM [194] (GeS-TM)

Quality Units have been implemented to tune locally the geometry Quality Parameter for TriSoup.

Additional description requested

## Memory Reduction of Trisoup Vertex Position Coding Using OBUF [195][196] (GeS-TM)

In Figure 128 the existing entropy coding pipeline of two position bits using dynamic OBUF (first bit on the left, second bit on the right) is illustrated.

Figure 128: OBUF Scheme of Trisoup vertex position bits.

In the proposed scheme, the inter and intra contexts are reorganized in a different way as shown in Figure 129 (first bit on the left, and second bit on the right).

Figure 129: Flowchart of proposed scheme.

Compared with the existing scheme, following modifications are made:

- Eliminating the two inter bits (representing three states: NoPred, Pred0 and Pred1) among the secondary information bits for Trisoup inter prediction. As a result, the number of inter states is reduced from 223 to 221 for coding the first bit of vertex position and from 224 to 222 for coding the second bit of vertex position.

- Feed the five groups of state to thirteen coder groups for the first bit and feed the three groups of state to seven coder groups for the second bit according to the inter-frame information (i.e., NoPred, Pred0 and Pred1).

Figure 130: Composition of the inter context.

Additional memory reduction is proposed in [196].

# Entropy Coding

## Simplification of Bypass Mode [197]

It is proposed to use different functions for encoding/decoding of bypass and context bins in TMC13. These minor changes allow for avoiding probability update, interval (range) division and renormalization, and memory storage for probability in the bypass coding mode. The goal of this proposal is to simplify the encoding and decoding of bypass bins in TMC13.

In the current TMC13, the encoding of bypass bins is implemented using the same function that is used for context bins with probability p=½. Current s/w uses two bytes for the integer representation of probability P; the relation between P and p is given by P216=p. It means that for probability p=½ it is necessary to set P=215. Thus, for encoding/decoding of bypass and context bins the same functions are used. In this case for bypass mode following actions are performed:

- Storage and up-date of probability

- Division and renormalization of interval (range)

Note that for interval subdivision first division by two is performed and if Range becomes less than the legal diapason, then the renormalization operation is required (multiplication by two). However, if the range is odd then (range>>1)<<1) is not the same as the old value of the Range that was before division and renormalization. That is the main reason why the proposed solution has a tiny difference from the current. To avoid unnecessary storage and update of probability, and to skip subdivision and renormalization we propose to use in TMC13 s/w following coding scheme for bypass bin. Besides this coding scheme allows the encoding/decoding of several bypass bins simultaneously. Note that a similar implementation of bypass mode is already used in many reference s/w such as VTM, HM, etc.

Encoder:

The encoder operates with three variables: Low is the left boundary of the interval, Range is the current length of the interval and Counter is a value that increases after each renormalization. In the current version of s/w renormalization is performed by consequent multiplication by two and every time counter increases by 1. When the counter reaches 8 we can write a byte and set the counter as zero. It is also necessary to control carry propagation. Thus, when the bypass bin is encoded Low should be doubled, the Range should stay as it is and the counter should be increased by 1. If the encoded bin is “one” Low should be increased by Range. Control of carry propagation and writing byte to bitstream are unchanged. This division and renormalization of Range are omitted and storage and update of probability are not needed.

Figure 131: Flowchart of bypass bin encoding.

Decoder:

The decoder operates with three variables: Code_minus_Low  is the current code minus the left boundary of the interval, Range is the current length of the interval and Counter is a value that decreases after each renormalization. In the current version of s/w, renormalization is performed by consequent multiplication by two and every time counter decreases by 1. When the counter reaches 0 we can read the byte and set the counter as 8. Thus when the bypass bin is decoded Code_minus_Low  should be doubled,  Range should stay as it is and the counter should be decreased by 1. If  Code_minus_Low >= Range the value “1” should be decoded and Code_minus_Low should be decreased by Range, otherwise, the value “0” should be decoded.

Figure 132: Flowchart of bypass bin decoding.

# Fine-Granularity Slicing Using Layer-Group Structure

When fine-granularity slicing is enabled, a G-PCC bitstream is segmented into multiple sub-bitstreams. For effective use of the layering structure of G-PCC, each slice contains coded data of partial coding layers or partial regions. When the slice segmentation is paired with the coding layer structure, scalable transmission or spatial random access use cases could be supported in an efficient manner.

## Layer Group-Based Slice Segmentation [198]-[204]

In fine-granularity slicing, each slice segment contains the coded data from a layer-group which is defined as follows [198].

“A layer-group is defined as a group of consecutive tree layers whose start and the end depth could be any number in tree depth, and the start shall be smaller than the end. The order of the coded data in the slice segments shall be the same as the order of the coded data in the single slice.”

For example, consider a geometry coding layer structure with 8 layers as illustrated in Figure 133(a). In this example, there are three layer-groups and each layer-group is matched with different slices: layer-group 1 for coding layers from 0 to 4, layer-group 2 for coding layer 5, and layer-group 3 for coding layers 6 and 7. When the first two slices are transmitted or selected, the decoded output will be the partial layer of 0 to 5 as illustrated in Figure 133(b). By using slices with the layer-group structure, partial decoding of the coding layers could be supported without access to the entire bitstream.

In addition to the layer-group structure, division of a layer-group into multiple subgroups is also used in the consideration of the spatial random access use case [199].

“A subgroup is a subset of a layer-group where points of a subgroup are adjacent to each other. Subgroups of a layer-group are exclusive to each other and the set of points in the subgroups of a layer-group is identical to the set of points in the layer-group.”

Figure 133: Partial decoding process using layer-group based geometry slices. (a) Encoder: Three slices generated by the layer-group structure. (b) Decoder: partially decoded output by using 2 slices.

As points in each subgroup are bounded in a spatial region, the boundary of a subgroup could be described by the subgroup bounding box information [200]. By using the spatial information, the layer-group and subgroup structure could support accessing the region of interest (ROI) efficiently by selecting slices that cover the ROI.

In Figure 134, an example of subgroup structure and corresponding bounding box is described. In the figure, layer-group 2 and 3 of the previous example are divided into 2 and 4 sub-groups and contained in different slices. Given the slices of layer-group and sub-groups with its bound box information, the spatial access could be performed by 1) comparing each slice’s bounding box with the ROI, 2) selecting the slices whose subgroup bounding box is overlapped with ROI, and then 3) decoding the selected slices. When the ROI is considered in the region 3-3, slices 1, 3, and 6 are selected as the subgroup bounding box of the layer-group 1, subgroup 2-2, and 3-3 covers the ROI region. Note that, it is assumed that there is no dependency between subgroups in the same layer-group for effective spatial access. In case of the live streaming or low latency use case, the selection and decoding could be performed at the time of receiving each slice segments which will improve time efficiency.

Figure 134: Partial decoding process using layer-group and subgroup structure of (a) layer-group and subgroup structure and (b) subgroup bounding box information.

Based on the scalability and spatial access capabilities, the layer-group slicing provides efficient access to large-scale point cloud data or high-density point cloud data [201][202]. Due to the large number of points and the size of the data, rendering or displaying such content would take a significant amount of time. As an alternative approach, the level of detail could be adjusted based on the viewer’s interest. For example, structural or global area information is more important than local details if the viewer is far away from the scene or object. On the other hand, when the viewer gets close to a specific area or object, detailed information about the area of interest is needed. Using the adaptive method, the renderer can present data of sufficient quality to the viewer in an efficient manner. Figure 135 shows examples of incremental detail changes for three levels of viewing distance, where the viewing distance is changed based on the ROI.

1) High-level view (coarse detail),

2) Mid-level view (medium-level detail),

3) Low-level view (fine-grained detail).

Figure 135: Example of multi-resolution multi-size ROI.

The multi-resolution ROIs could be supported, when layer-group slicing is used in the generation of G-PCC bitstream. In Figure 136, the encoder generates bitstream slices of octree layer-groups or spatial subgroups of each layer-groups. Upon request, a slice matching the ROI of each resolution is selected and transmitted. The overall bitstream size is smaller than the tile-based approaches because the bitstream contains no details other than the requested ROI. At the receiver, the decoder can combine the slices to produce three outputs: 1) high-level view output comes from layer-group slice 1, 2) mid-level view output comes from layer-group slice 1 and selected subgroups of layer-group 2, and then 3) the low-level view of fine detail output comes out of layer-group 1 and selected subgroups of layer-group 2 and 3. As the outputs could be generated progressively, the receiver could provide viewing experience like zooming from high-level view to low-level view with a gradual increase in resolution.

Figure 136: Scalable transmission based on the layer-group slicing.

The parameters which describe layer-group structure are signaled in different levels [203]. Common structural information is described in SPS and the details of each layer-group or subgroup are signaled in slice header. Also, layer-group structure inventory and dependent slice header is introduced to describe overall layer-group structure. The definition of the G-PCC slices and the signalling method of the fine-granularity slicing is given as in the following.

1) Definition of G-PCC slices.

A. Slice: A set of points coded by one independent fine-granularity slice and zero or more dependent fine-granularity slices.

B. Dependent fine-granularity slice: A data unit of a slice that depends upon a previous data unit within the same slice.

C. Independent fine-granularity slice: The first [geometry] data unit of a slice.

2) Fine-granularity slice is enabled in SPS.

3) Essential information to decode dependent fine-granularity slice is delivered by the dependent data unit header, including context inheritance and per-slice bounding box.

4) Define layer-group structure inventory to describe the relationship between fine-granularity slices [204].

## Geometry Slicing

### Coding Process

#### Encoder Process [205]

The input to the layer-group slicing encoder is point cloud data and the parameters which describe the structural information of the layer-group slicing. In the beginning of each tree depth, the layer-group of the target tree depth is determined by using the layer-group structure parameters. With the layer-group index, the subgroup index of each node is determined by using subgroup bounding box. When a subgroup is changed for a node, the context state and buffers used by the previous subgroup encoder is saved and those of the current subgroup encoder is loaded. By using separate encoders for each subgroup, the context state could be continued within the subgroup. Also, to limit the neighbor nodes to be in the same subgroup with the current node, the range of the geometry occupancy atlas is updated by considering the subgroup boundary on top of the atlas boundary. Both methods make each coded bitstream to be decodable independently without node information of the neighbor subgroups. This process is recursively conducted for all nodes in all tree depths. When the end of the nodes of the target tree depth is reached, fine-granularity slices (FGS) are generated which are one-to-one matched with the subgroups of each layer-group.

In the following, the techniques used for the independent decoding is addressed with pseudo codes.

Figure 137: Encoder process for layer-group slicing.

##### Save and Load Encoder (with layer-group and subgroup determination)

As mentioned in the encoder process, save and load the encoder state is necessary to guarantee the independent decoding of each subgroup. To provide flexible subgroup partitioning, this process is conducted for each node of a tree layer.

For each tree layer, the layer-group of the tree layer is determined and fixed for all tree depths within the layer-group. As a layer-group is a set of consecutive tree layers, the layer-group index is changed at the beginning of the layer-group. With the determined layer-group, the subgroup is determined. As the subgroups is a group of nodes bounded in the subgroup bounding box, the subgroup of a node is found by comparing the node position and the subgroup bounding box. Whenever the subgroup or layer-group is changed, the encoder state of the previous subgroup is saved for future use and the encoder state of the current subgroup is loaded for continuous encoding.

for (depth = 0; depth < maxDepth; depth++) { // determine layer-group index if (depth == 0) { curLayerGroupId = 0; curSubgroupId = 0; load current context state; sum_layers = numLayersPerLayerGroup[curLayerGroupId]; } else if (depth == sum_layers) { prevLayerGroupId = curLayerGroupId++; prevSubgroupId = curSubgroupId; curSubgroupId = 0; save previous encoder state; load current encoder state; sum_layers += numLayersPerLayerGroup[curLayerGroupId]; } else if (numSubgroupsMinus1[curLayerGroupId] > 0) { prevSubgroupId = curSubgroupId; curSubgroupId = 0; save previous encoder state; reload current encoder state; } for (all nodes in curLayerGroupId) { // determine subgroup index if (!(nodePos >= bbox_min && nodePos < bbox_max)) { for (i = 0; i<=numSubgroupsMinus1[curLayerGroupId]; i++) { if (nodePos >= bbox_min[i] && nodePos < bbox_max[i]){ prevSubgroupId = curSubgroupId; curSubgroupId = i; // save and load encoder save previous encoder state; if (first node of the current subgroup) load reference encoder state; else reload current encoder state; break; } } } } }

##### Update Geometry Occupancy Atlas

To consider the subgroup boundary in the geometry occupancy atlas, _maxRange and _minRange are defined in the class MortonMap3D. When the atlas is in the subgroup, the minimum and the maximum ranges are set as 0 and the length of an edge of the cube, respectively. When the minimum boundary of the subgroup is larger than the atlas minimum, _minRange is set as the minimum of the subgroup boundary. When the maximum boundary of the subgroup is smaller than the atlas maximum, _maxRange is set as the maximum of the subgroup boundary. By using the range, the part of the atlas that is overlapped with the subgroup bounding box is considered as active and the nodes in the active region shall be used as the neighbors. This guarantees FGS to be decoded without nodes in the neighbor subgroups.

class MortonMap3D { setRange( ) { for (m = 0; m < 3; m++) { // _maxRange if (bboxMax < atlasOrigin + _cubeSize) _maxRange[m] = bboxMax - atlasOrigin; else _maxRange[m] = _cubeSize; // _minRange if (bboxMin > atlasOrigin) _minRange[m] = bboxMin - atlasOrigin; else _minRange[m] = 0; } } }

#### Decoder Process [205]

The decoding process of the layer-group slicing reference SW for the first fine-granularity slice (FGS) is the same with the conventional decoding of the geometry slice: parsing parameter set, parsing data unit header and decoding data unit. When layer-group slicing is enabled, the following dependent geometry data unit are considered as FGS for the first slice if they have same slice_id. Considering the context reference and node inheritance between parent and child subgroups, the order of the FGSs are assumed to be arranged by the layer_group_id and subgroup_id in the ascending order.

Before decoding the dependent data unit, the context state, output nodes, and layer-group parameters of the previous slice are saved to the buffer for followed slices. After parsing dependent data unit header, parent of the current subgroup is detected by finding a subgroup whose subgroup bounding box is a superset of the current subgroup bounding box. When the parent subgroup is determined, parent nodes of the current dependent data unit are selected. By using the selected nodes as the initial nodes of the decoding process, the current dependent data unit is decoded until the tree layer covered by the current layer-group. The decoding process of the dependent data unit is conducted recursively until the end of the geometry bitstream.

In the following, the additional process to find the parent subgroup and the parent nodes will be addressed in more details.

Figure 138: Decoding process of layer-group slicing enabled case.

##### Parent Subgroup Detection

In the decoding of the fine granularity slices, the nodes of the parent subgroup are used as the input to the child subgroups to provide continuous decoding in the layer-group boundaries. As this is due to the hierarchical structure of the layer-group slicing, it could be induced by using the parent-child spatial relationship: a child subgroup is a subset of a parent subgroup where the bounding box of a child subgroup is spatially exclusive to the bounding boxes of the other child subgroups within a same layer-group.

Based on the relationship, a slice with the parent subgroup could be detected by finding spatial superset of the current slice. By using subgroup_bbox_origin and subgroup_bbox_size signaled in the data unit header, the parent subgroup could be found by comparing it with the bounding box information of the subgroups in the previous layer-group level.

parentLayerGroup = curLayerGroup - 1; for (i = 0; i < numSubgroups[parentLayerGroup]; i++) { if (_bboxMin[parentLayerGroup][i] <= curBboxMin && _bboxMax[parentLayerGroup][i] > curBboxMin) { parentSubgroup = i; break; } }

##### Input Parent Node Selection

In the decoding of dependent slices, the output nodes of the parent subgroup are used as an input to the decoding of the child subgroup. When the subgroup bounding boxes of the parent and the child subgroups are identical, all the nodes produced by the parent subgroup are used. On the other hand, when the subgroup bounding box of the child subgroup is a subset of the parent subgroup, decoders need to select actual parent nodes. To find the parent nodes, each node in the parent subgroup is compared with the bounding box boundary of the child subgroup.

for (node = inNodes.begin(); node != inNodes.end(); node++) { if (node.Pos >= bbox_min && node.Pos < bbox_max) fifo.emplace_back(node); else continue; }

### Syntax and Semantics

The syntax elements of the geometry fine granularity slices are given in the following tables where the new syntax elements are highlighted in yellow [205].

#### Sequence Parameter Set Syntax and Semantics

Table 11: Sequence parameter set syntax.

| seq_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| layer_group_enabled_flag | u(1) |
| if(layer_group_enabled_flag) { |  |
| num_layer_groups_minus1 | u(8) |
| for(i=0; i<=num_layer_groups_minus1; i++) { |  |
| layer_group_id[i] | u(8) |
| num_layers_minus1[i] | u(8) |
| subgroup_enabled_flag[i] | u(1) |
| } |  |
| if(subgroup_enabled_flags( ) ) { |  |
| subgroup_bbox_origin_bits_minus1 | ue(v) |
| subgroup_bbox_size_bits_minus1 | ue(v) |
| } |  |
| } |  |
| while( more_data_in_data_unit( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

layer_group_enabled_flag is equal to 1 specifies that the geometry bitstream of a slice is contained in multiple slices which is matched to a group of coding layers or its subgroup. layer_group_enabled_flag is equal to 0 specifies that the geometry bitstream is contaned in a single slice.

num_layer_groups_minus1 plus 1 specifies the number of layer-groups where the layer-group represents group of consecutive tree layers that are part of the geometry coding tree structure. num_layer_groups_minus1 shall be in the range of 0 to number of coding tree layers.

layer_group_id specifies the indicator of a layer-group of a slice. The range of layer_group_id shall be in the 0 to num_layer_groups_minus1.

num_layers_minus1 plus 1 specifies the number of coding layers contained in the i-th layer-group. The total number of layer-groups could be derived by adding all (num_layers_minus1[i] + 1) for i equal to 0 to num_layer_groups_minus1.

subgroup_enabled_flag is equal to 1 specifies the i-th layer-group is divided into two or more subgroups where the set of points in the subgroups of a layer-group is identical to the set of points in the layer-group. When subgroup_enabled_flag of the i-th layer-group is equal to 1, subgroup_enabled_flag of the j-th layer-group shall be equal to 1 when j is larger than or equal to i. subgroup_enabled_flag is equal to 0 specifies that the current layer-group is not sub-divided into multiple subgroups and contained in a single slice.

subgroup_bbox_origin_bits_minus1 plus 1 is the length in bits of the syntax elements subgroup_ bbox_origin.

subgroup_bbox_size_bits_minus1 plus 1 is the length in bits of the syntax elements subgroup_ bbox_size.

#### Dependent Geometry Data Unit Header Syntax and Semantics

Table 12: Dependent geometry data unit header syntax.

| dependent_geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| dgsh_geometry_parameter_set_id | u(4) |
| dgsh_slice_id | ue(v) |
| layer_group_id | u(8) |
| if(subgroup_enabled_flag[layer_group_id]) { |  |
| subgroup_id | u(8) |
| for(i=0; i<3; i++) |  |
| subgroup_bbox_origin[i] | u(v) |
| for(i=0; i<3; i++) |  |
| subgroup_bbox_size[i] | u(v) |
| } |  |
| ref_layer_group_id | u(8) |
| if(subgroup_enabled_flag[layer_group_id]) |  |
| ref_subgroup_id | u(8) |
| byte_alignment( ) |  |
| } |  |

dgdu_geometry_parameter_set_id specifies the active GPS indicated by gps_geom_parameter_ set_id. The value of dgdu_geometry_parameter_set_id shall be the same with the value of gdu_geometry_parameter_set_id of the corresponding slice.

dgdu_slice_id specifies the slice where the current dependent geometry data unit belongs.

layer_group_id specifies the indicator of a layer-group of a slice. The range of layer_group_id shall be in the range of 0 to num_layer_groups_minus1.  When not present, layer_group_id is inferred to be 0.

subgroup_id specifies the indicator of the subgroup of the layer-group referred by layer_group_id. The range of subgroup_id shall be in the range of 0 to num_subgroups_minus1[layer_group_id] where the subgroup_id represent the order of the slices in the same layer_group_id. When not present, subgroup_id is inferred to be 0.

subgroup_bbox_origin specifies the position of the origin of the subgroup bounding box of the i-th subgroup indicated by subgroup_id of the j-th layer-group indicated by layer_group_id.

subgroup_bbox_size specifies the size of the subgroup bounding box of the i-th subgroup indicated by subgroup_id of the j-th layer-group indicated by layer_group_id.

A bounding box of points in a subgroup is described by subgroup_bbox_origin and subgroup_bbox_size. A region in the bounding box of the i-th subgroup shall not be overlapped with the bounding box of the j-th subgroup when i and j are not identical.

ref_layer_group_id specifies the indicator of the layer-group identifier of the context reference of the current dependent data unit. The range of the ref_layer_group_id shall be in the range of 0 to the layer_group_id of the current dependent data unit.

ref_subgroup_id specifies the indicator of the reference subgroup of the layer-group indicated by ref_layer_group_id. The range of the ref_subgroup_id shall be in the range of 0 to num_subgroup_id_ minus1 of the layer-group indicated by ref_layer_group_id.

context_reference_indication_flag equal to 1 indicates that the context state of the current dependent data unit will be inherited to one or more followed dependent data units. context_reference_ indication_flag equal to 0 indicate the context state of the current dependent slice will not be inherited to the followed dependent slice. When not present, context_reference_indication_flag of a data unit is inferred to be 1.

Decoders could manage the context buffer by using context_reference_indication_flag. When context_reference_indication_flag equal to 1, the context state of the current dependent slice is saved to the context buffer at the end of the decoding of the current data unit. When context_reference_ indication_flag equal to 0, the context state of the current dependent slice is not saved to the context buffer.

#### Layer-Group Structure Inventory Syntax and Semantics

Table 13: Layer-group structure inventory syntax.

| layer_group_struture_inventory( ) { | Descriptor |
| --- | --- |
| lgsi_seq_parameter_set_id | u(4) |
| lgsi_frame_ctr_lsb_bits | u(5) |
| lgsi_frame_ctr_lsb | u(v) |
| lgsi_num_slice_ids_minus1 | u(8) |
| if(lgsi_num_slice_ids_minus1 >= 0 ) { |  |
| for(sId=0; sId<=lgsi_num_slice_ids_minus1; sId++){ |  |
| lgsi_slice_id[sId] | ue(v) |
| lgsi_num_layer_groups_minus1[sId] | u(8) |
| lgsi_subgroup_bbox_origin_bits_minus1[sId] | ue(v) |
| lgsi_subgroup_bbox_size_bits_minus1[sId] | ue(v) |
| for(i=0; i<=lgsi_num_layer_groups_minus1[sId]; i++){ |  |
| lgsi_layer_group_id[sId][i] | u(8) |
| lgsi_num_layers_minus1[sId][i] | u(8) |
| lgsi_num_subgroups_minus1[sId][i] | u(16) |
| for(j=0; j<=lgsi_num_subgroups_minus1[sId][i]; j++){ |  |
| lgsi_subgroup_id[sId][i][j] | u(16) |
| lgsi_parent_subgroup_id[sId][i][j] | u(16) |
| for(k=0; k<3; k++) |  |
| lgsi_subgroup_bbox_origin[sId][i][j][k] | u(v) |
| for(k=0; k<3; k++) |  |
| lgsi_subgroup_bbox_size[sId][i][j][k] | u(v) |
| } } } } |  |
| lgsi_origin_bits_minus1 | ue(v) |
| for(k=0; k<3; k++) |  |
| lgsi_origin_xyz[k] | se(v) |
| lgsi_origin_log2_scale | ue(v) |
| byte_alignment( ) |  |
| } |  |

lgsi_seq_parameter_set_id specifies a value of sps_seq_parameter_set_id.  It is a requirement of bitstream conformance that lgsi_seq_parameter_set_id is equal to 0.

lgsi_frame_ctr_lsb_bits specifies the length in bits of the lgsi_frame_ctr_lsb syntax element.

lgsi_frame_ctr_lsb specifies the lgsi_frame_ctr_lsb_bits least significant bits of FrameCtr from which the group structure inventory is valid. A layer-group structure inventory remains valid until it is replaced by another layer-group structure inventory.

lgsi_num_slice_ids_minus1 plus 1 specifies the number of slices present in the layer-group structure inventory.

lgsi_slice_id specifies the slice id of the sid-th slice within the layer-group structure inventory. It is a requirement of bitstream conformance that all values of lgsi_slice_id are unique within a layer-group structure inventory.

lgsi_num_layer_groups_minus1 plus 1 specifies the number of layer-group.

lasi_subgroup_bbox_origin_bits_minus1 plus 1 is the length in bits of the syntax elements lgsi_ subgroup_bbox_origin.

lgsi_subgroup_bbox_size_bits_minus1 plus 1 is the length in bits of the syntax elements lgsi_ subgroup_bbox_size.

lgsi_layer_group_id specifies the indicator of a layer-group. The range of lgsi_layer_group_id shall be in the range of 0 to lgsi_num_layer_groups_minus1.

lgsi_num_layers_minus1 plus 1 specifies the number of coded layer in the slice of i-th layer-group in the sid-th slice. The total number of the coded layer required for decoding n-th layer-group is equal to sum of lgsi_num_layers_minus1[sid][i] + 1 for i equal 0 to n.

lgsi_num_subgroups_minus1 plus 1 specifies the number of sub-groups in the i-th layer-group in the sid-th slice.

lgsi_subgroup_id specifies the indicator of a layer-group. The range of lgsi_subgroup_id shall be in 0 to lgsi_num_subgroups_minus1.

lgsi_parent_subgroup_id specifies the indicator of a sub-group in the layer-group indicated by lgsi_subgroup_id. The range of lgsi_parent_subgroup_id shall be in 0 to gi_num_subgroups_minus1 in the layer-group indicated by lgsi_subgroup_id.

lgsi_subgroup_bbox_origin specifies the origin of the subgroup bounding box of the subgroup indicated by lgsi_subgroup_id of the layer-group indicated by lgsi_layer_group_id.

lgsi_subgroup_bbox_size specifies the size of the subgroup bounding box of the subgroup indicated by lgsi_subgroup_id of the layer-group indicated by lgsi_layer_group_id.

lgsi_origin_bits_minus1 plus 1 specifies the length in bits of the lgsi_origin_xyz syntax elements.

lgsi_origin_xyz indicate the origin of all of the partitions. The value of lgsi_origin_xyz[ k ] should be equal to sps_bounding_box_offset[ k ].

lgsi_origin_log2_scale indicates a scaling factor to scale components of lgsi_origin_xyz.  The value of lgsi_origin_log2_scale should be equal to sps_bounding_box_offset_log2_scale.

### Context Buffer Management

#### Context Reference Indication [206]

In fine granularity slicing, context inheritance between slices is used to alleviate the coding loss caused by the discontinuity between adjacent nodes or coding layers. When a bitstream in a slice is decoded, the context state is saved to the context buffer as shown in Figure 139(a). When context inheritance is used, the context states of the following slices are initialized by one of the saved states of the previous slices indicated by ref_layer_group_id and ref_subgroup_id (Figure 139(b)). Therefore, for the context state of the slice belonging to the last layer-group, it is initialized by the saved context state of the previous slices as shown in Figure 139(c). However, decoders could determine to not save the context of FGS N+1 to 2N when there is a constraint of not referencing to the subgroups in the same layer-group. With the prior information, a smart decoder could save context buffer depending on its usage by the subsequent subgroups.

Figure 139: Context buffer update (parent subgroup referencing).

However, when flexible context reference is allowed, there are context states that are not used by the following slices. For example, consider the root layer-group referencing where all the context states of the dependent slices are initialized by the context state saved from the first slice. As decoders do not know the overall referencing structure and there is possibility of using the current context in the following slices. However, as expected, the saved context states (1, 0) to (1, N-1) are not used by any slices. In this example, the inefficiency comes from lack of information in the decoder side.

Figure 140: Context buffer update (root layer-group referencing)

To improve the context buffer management efficiency, a new signal is introduced to help decoders to indicate the usage of the current context in the followed slices and determine whether to save the context. In Figure 141(b), the use case of the new signal in the buffer management is illustrated. Compared to the previous example without any information, saving context state to the context buffer is determined by the context reference indicator, called context_reference_indication_flag. When context reference indicator is enabled, decoder saves the current context state to the context buffer to allow use the context state by the followed slices. On the other hand, when context reference indicator is disabled, decoders do not save the current context state to the context buffer. Comparing the Figure 141(c) and Figure 140(c), the context buffer for the root-layer referencing case have only one context compared to context buffer for the parent subgroup referencing case.

Figure 141: Buffer management by using context reference indicator.

Based on the context memory management mechanism, a new signal context_reference_indication_flag is introduced in the dependent geometry data unit header as follows.

Table 14: Updates in the dependent geometry data unit header syntax.

| dependent_geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(subgroup_enabled_flag[layer_group_id]) |  |
| ref_subgroup_id | u(16) |
| context_reference_indication_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

context_reference_indication_flag equal to 1 indicates that the context state of the current dependent slice will be inherited to one or more followed dependent slices. context_reference_indication_flag equal to 0 indicates the context state of the current dependent slice will not be inherited to the followed dependent slice.

Decoders could manage the context buffer by using context_reference_indication_flag. When context_ reference_indication_flag equal to 1, the context state of the current dependent slice is saved to the context buffer at the end of the decoding. When context_reference_indication_flag equal to 0, the context state of the current dependent slice is not saved to the context buffer.

#### Releasing Context Reference [207]

By using context reference indication flag, total size of the context buffer could be reduced by storing the context states that are known to be used by the following slices. However, there is a limitation that decoders cannot know the release timing of each saved context. To resolve the issue, it was proposed to signal the number of subgroups which reference the current subgroup.

Figure 142: Releasing context buffer.

In the figure, the method of releasing context buffer by using the proposed signal is illustrated. In the figure, context buffer has two additional columns to indicate the number of the subsequent subgroups referencing to the current subgroup and to count the number of the subgroups which already use the context state in the subgroup decoding.

When the decoding of FGS2 is finished, the output context state (1, 0) is saved to the context buffer and the number of subsequent subgroups is saved. When the next slice, i.e., FGS N+2, is decoded, the context state is initialized by the context state (1, 1) and the corresponding counter in the context buffer is increased by 1. Since the number in the counter and the saved value of the number of subsequent subgroups is identical, the context state (1, 1) could be released. In this example, the maximum number of the context state in the context buffer is 2 which is the number of layer-group minus 1.

Based on the proposed context memory management mechanism, it is proposed to define a new signal num_subsequent_subgroups in the data unit headers as follows.

Table 15: Updates in the geometry data unit header syntax.

| geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if(sps.layer_group_slicing_flag) |  |
| num_subsequent_subgroups | u(8) |
| byte_alignment( ) |  |
| } |  |

Table 16: Updates in the dependent geometry data unit header syntax.

| dependent_geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| … |  |
| context_reference_indication_flag | u(1) |
| if(context_reference_indication_flag) |  |
| num_subsequent_subgroups | u(8) |
| byte_alignment( ) |  |
| } |  |

num_subsequent_subgroups specifies the number of the subsequent dependent data units which references the current data unit or dependent data unit.

### Managing Number of Points Per Slice [208]

In the layer-group slicing, the subgroup is divided by a parameter subgroupSize, which determines the length of an edge of a cubic subgroup bounding box. As a bounding box of a content is divided with uniform distance in all axis, the cubic subgroup could be considered as a unit for spatial random access.

However, this simple approach cannot consider the number of points in a subgroup and therefore the number of points in a subgroup might exceed the limitation used in the TMC13. As the limitation is used to guarantee the receiver’s performance, it is necessary to make subgroups to have points under the limitation.

For this purpose, one could use smaller subgroupSize, but this might increase the number of subgroups which might cause inefficiency by making more discontinuities in between the adjacent nodes and by adding headers and parameters in the inventory. Also, some constraint on the number of subgroups might be needed in consideration of the decoder capability. Therefore, the following constraints are considered to generate a subgroup.

1)	Number of points in a subgroup is lower than the level limitation

2)	Less number of additional subgroups

Considering the constraints, step-by-step subgroup splitting method is introduced where the method splits a subgroup with one direction and check the number of points in each sub-division at each step. This early termination could minimize the number of additional subgroups.

Figure 143: Subgroup division process.

In Figure 143, an example of the subgroup split is shown, where one of the division direction is selected in each step. In the first step, the number of the points in a subgroup bounding box is calculated and divided into two sub-blocks when the number of points is larger than the level limitation. In the determination of the division direction, the cost of each division is calculated and compared. To make less subgroup division, the cost of the division is considered as the difference between the number of point at each sub-divisions A and B.

Cost = | number of points in the sub-division A – number of points in the sub-division B |

By comparing the three costs of dividing across x, y, and z axis, the axis which have lowest cost is selected as the subgroup division. In the example in the figure, y direction is selected.

When the number of points in the sub-division A and B are below the level limit, the process will be terminated. On the other hand, if one or two subgroups do not satisfy the constraint 1, the subgroup bounding box is divided by the direction selected from the remaining of the previous step by the same cost function. After the second division, the number of points in each sub-division is estimated and compared with the level limit. Based on the result, the splitting could be terminated in the second step or additional division will be conducted.

When the number of points in the sub-division A or B is above the level limit, the third subgroup division will be conducted. As there is only one division direction remained, subgroup division is performed without selection.

The final output of the subgroup split is all the sub-divided bounding boxes of each step whose number of points in the bounding box is below the level limitation.

### Adaptive Planar Eligibility Based on Density [209]

In TMC13, the planar eligibility is determined for each tree depth based on the density as follows.

realDensityi+1=numPoints-numPointsCodedByIdcminumSubnodesi

where numPoints indicates the number of points in a point cloud, numPointsCodedById dcmi is the number of points that are coded using IDCM from root node layer to ith octree layer, numSubnodesi is the number of occupied sub-nodes generated by the nodes in ith octree layer, and realDensityi+1 is the real density of points for i+1th octree layer.

Before the coding process of i+1th octree layer, the eligibility of all the nodes to xyz-planar mode in i+1th octree layer is determined as follows:

planarEligibleKOctreeDepthi+1=1,       realDensityi+1<th0,                           otherwise

where th is a predefined threshold, whose typical value is set to 1.3.

When the above density is used in layer-group slicing, the density could be estimated adaptive to subgroup density, which is defined as follows.

realDensityi+1(s)=numPoints(s)-numPointsCodedByIdcmi(s)numSubnodesi(s)

where numPoints(s) indicates the number of points of the subgroup s, numPointsCoded ByIdcmi(s) is the number of points that are coded using IDCM from root node layer to ith octree layer and within the subgroup boundary of the subgroup s, numSubnodesi (s) is the number of child nodes generated by the nodes in  the i-th octree layer in the subgroup s, and realDensityi+1(s) is the real density of points for the (i+1)-th octree layer of subgroup s.

Before the coding process of the (i+1)-th octree layer, the eligibility of all the nodes to xyz-planar mode in the (i+1)-th octree layer of the subgroup s is determined as follows:

planarEligibleKOctreeDepthi+1(s)=1,       realDensityi+1(s)<th0,                                 otherwise

where th is a predefined threshold, whose typical value is set to 1.3.

The problem of using the subgroup adaptive planar eligibility based on density is that the number of points of each subgroup cannot be known in the decoder until the subgroups belong to the last layer-group are delivered. To match the encoder and decoder, it is proposed to signal planar_eligible_ kOctree_depth of each depth in the data unit header of geometry data unit and dependent geometry data units.

To match the subgroup adaptive planar eligibility flags in the encoder and decoder, the following syntax and semantics are proposed to be updated in both geometry data unit header and dependent geometry data unit header.

Table 17: Updates in geometry data unit header syntax.

| geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| checkPlanarEligibilityBasedOnOctreeDepth = geom_planar_mode_enabled_flag && geom_octree_depth_planar_eligibiity_enabled_flag && ! geom_angular_mode_enabled_flag |  |
| if(layer_group_enabled_flag && checkPlanarEligibilityBasedOnOctreeDepth) { |  |
| for(i = 0; i <= num_layers_minus1[0]; i++) |  |
| planar_eligible_kOctree_depth[i] | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Table 18: Updates in dependent geometry data unit header syntax.

| dependent_geometry_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if(layer_group_enabled_flag && checkPlanarEligibilityBasedOnOctreeDepth) { |  |
| for(i = 0; i <= num_layers_minus1[layer_group_id]; i++) |  |
| planar_eligible_kOctree_depth[i] | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

planar_eligible_kOctree_depth equal to 1 indicate the planar eligibility flag is enabled for the i-th depth of the current subgroup. planar_eligible_kOctree_depth equal to 0 indicate the planar eligibility flag is disabled for the i-th depth of the current subgroup.

### Occupancy Tree Depth Signalling [216]

Fine granularity slicing could be used to support multiple decoders with different capabilities due to the memory, clock speed, display device, etc. It is proposed to add a signal in the sequence parameter set level to indicate the depth of encoded occupancy tree explicitly. Since it is used in the decoding process of the fine granularity slices, it is proposed to not change this signal in the generation of parameter sets in supporting different capability of decoders. In addition to that, in the geometry data unit header, it is also proposed to describe the actual depth of coded occupancy tree of a geometry data unit. This prevents mismatch of information when only the first fine granularity slice is delivered to the decoders.

### Direct Mode Eligibility Condition on Layer-group Boundary [217]

When there is a disconnection due to the layer-group of fine granularity slices, the idcm eligibility cannot be determined in the start depth of a child subgroup since there are loss of those information from parent and grandparent.

To provide continuity in the layer-group boundary in terms of direct mode eligibility, it is proposed to save OccNeighPatEq0 and OccNodeChildCnt at the end of subgroup coding and restore it at the start of the child subgroup coding.

### Improved Context Buffer Management [218]

In layer-group slicing, context inheritance between fine granularity slices is used to alleviate the coding loss caused by the discontinuity between adjacent nodes or coding layers. However, as the number of slices or layer-groups increases, a burden on the decoder increases as the number of context states in memory increases.

To help decoders to manage the context buffer, context_reference_indication_flag is signaled to indicate future use of the current context by the subsequent slices. By using this, total size of the context buffer could be reduced by storing the context states that are known to be used by the following slices.

In addition to that, the saved context states are released by using num_subsequent_subgroups, which informs the number of subgroups refer to the context state of the current subgroup. However, when partial layer or partial region of the bitstream is decoded, the saved contexts might not be released due to the subsequent subgroup over the output layer-group or out of the region of interest (ROI). As result, the number of the coded subsequent subgroups cannot reach to the number signaled by num_subsequent_subgroups and the unnecessary context states are remained in the decoder memory.

For partial decoding of tree depth, it is proposed to signal the number of subsequent subgroups per layer-group which extends the existing mechanism in TMC13. For partial decoding of spatial region and release of parent node, it is shown that decoders could release them without other additional signals.

### Syntax Refinement for Fine Granularity Slice [219]

It is proposed to modify the syntax elements implemented in the current TMC13 to clarify the parent-child relationship and the context referencing relationship for all data units in the tree, starting from the root-most data unit. This allows the decoder to identify all the necessary data units at the start of decoding and to accurately determine when resources become unnecessary. This applies equally to full decoding, scalable decoding, and partial decoding.

## Attribute Slicing

### Coding Process [210][211][212]

#### Encoder Process

The attribute slicing encoder is based on partial LoD generation for each subgroup. This method consumes less memory because it does not need to generate LoDs for the entire point cloud. On the other hand, it requires the process of assigning the values of the corresponding attributes to the nodes of each intermediate layer group before encoding begins.

Geometry Encoding: 1. Perform octree geometry encoding for each subgroup (data unit): a. Encode a partial octree. b. For intermediate layer-groups, store the child indexes for each node. 2. For nodes in intermediate layer-groups, compute a sampled-index which indicates points in the corresponding LoD using small LoD generation using the stored child indexes. Recoloring: - Recolor the encoded points. Attribute Encoding: Conduct attribute encoding for each subgroup: a. QP/Weight adjustment: i. For C1/C2, QP delta adaptation ii. For CY, Weight adjustment b. For intermediate layer-groups: i. Assign an attribute value to each intermediate node, derived from the recoloured points indicated by the sampled-index. c. Generate a partial LoD for the current subgroup using intermediate nodes and IDCM points which are encoded in the current subgroup. i. Reverse the sample direction of octree subsamples for odd and even levels as in the case of scalable coding d. Based on the partial LoD, encode attribute values: i. At the top level of LoD, predict attribute values for each point using its corresponding parent node. ii. At other levels, use points in the LoD for predicting attribute values.

Figure 144: Sample and store the index of corresponding points based on the small LoD of children points as sampled-index for each intermediate node in geometry octree coding.

#### Decoder Process

Geometry Decoding: 1. Perform octree geometry decoding for each subgroup (data unit): a. Decode a partial octree. Attribute Decoding: 1. Conduct attribute decoding for each subgroup: a. Generate a partial LoD for the current subgroup using intermediate nodes and IDCM points which are encoded in the current subgroup. i. Reverse the sample direction of octree subsamples for odd and even levels as in the case of scalable coding b. Based on the partial LoD, decode attribute values: i. At the top level of LoD, predict attribute values for each point using its corresponding parent node. ii. At other levels, use points in the LoD for predicting attribute values.

### Syntax and Semantics

#### Attribute Parameter Set Syntax and Semantics

Table 19: Updates in attribute parameter set data unit syntax.

| attribute_parameter_set ( ) { | Descriptor |
| --- | --- |
| … |  |
| aps_extension_present | u(1) |
| if( aps_extension_present ) |  |
| layer_group_aligned_flag | u(1) |
| while( more_data_in_data_unit( ) ) |  |
| aps_extension_data | u(1) |
| byte_alignment( ) |  |
| } |  |

layer_group_aligned_flag equal to 1 specifies that the attribute bitstream contains at least one independent attribute data unit and dependent data units and each unit contains attributes values corresponding to each individual decoded points of the independent / dependent geometry data units. When layer_group_aligned_flag is equal to 0, the attribute bitstream contains only independent attribute data units contains attribute values corresponding to all united decoded points of the independent / dependent data units.

#### Dependent Attribute Data Unit Header Syntax and Semantics

Table 20: Updates in dependent attribute data unit header syntax.

| dependent_attribute_data_unit_header ( ) { | Descriptor |
| --- | --- |
| dadu_attr_parameter_set_id | u(4) |
| dadu_sps_attr_idx | ue(v) |
| dadu_slice_id | ue(v) |
| dadu_layer_group_id | un(8) |
| if(subgroup_enabled_flag [dadu_layer_group_id]) |  |
| dadu_subgroup_id | un(16) |
| if( last_comp_pred_enabled && AttrDim == 3 ) |  |
| for( i = 0; i ≤ num_layers_minus1[dadu_layer_group_id]+1; i++ ) |  |
| dadu_last_comp_pred_coeff_diff[ i ] | se(v) |
| if( inter_comp_pred_enabled ) |  |
| for( i = 0; i ≤ num_layers_minus1[dadu_layer_group_id]+1; i++ ) |  |
| for( c = 1; c < AttrDim; c++) |  |
| dadu_inter_comp_pred_coeff_diff[ i ][ c ] | se(v) |
| if( attr_qp_offsets_present ) |  |
| for( ps = 0; ps < Min( 2, AttrDim ); ps++) |  |
| dadu_attr_qp_offset[ ps ] | se(v) |
| dadu_attr_qp_layers_present | u(1) |
| if( dadu_attr_qp_layers_present ) { |  |
| dadu_attr_qp_layer_cnt_minus1 | ue(v) |
| for( dpth = 0; dpth ≤ dadu_attr_qp_layer_cnt_minus1; dpth++ ) |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++ ) |  |
| dadu_attr_qp_layer_offset[ dpth ][ qc ] | se(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

dadu_attr_parameter_set_id specifies the active APS by its aps_attr_parameter_set_id.

dadu_sps_attr_idx identifies the coded attribute by its index into the active SPS attribute list.

dadu_slice_id specifies the slice where the current dependent attribute data unit belongs.

dadu_layer_group_id specifies the indicator of a layer-group. The range of dadu_layer_group_id shall be in 0 to num_layer_groups_minus.

dadu_subgroup_id specifies the indicator of a subgroup. The range of dadu_subgroup_id shall be in 0 to num_subgroups_minus1[layer_group_id]. When not present, dadu_subgroup_id is inferred to be 0.

dadu_last_comp_pred_coeff_diff[ i ] specifies the scale factor applied to the second coefficient component of a multi-component attribute to predict the last coefficient component. Each scale factor applies to coefficients in a single transform level i, proceeding from the first to the last.

dadu_inter_comp_pred_coeff_diff[ i ][ c ] specifies scale factor applied to the first coefficient component of a multi-component attribute to predict the c-th coefficient component. Each scale factor applies to coefficients in a single transform level i, proceeding from the first to the last.

dadu_attr_qp_offset specifies whether per data unit QP offsets, dadu_attr_qp_offset[ c ], are present in the DADU header.

dadu_attr_qp_layers_present specifies whether (when 1) or not (when 0) per-transform-layer QP offsets are present in the DADU.

dadu_attr_qp_layer_cnt_minus1 plus 1 specifies, when present, the number of levels in the LoD hierarchy or RAHT tree for which QP offsets are signaled.

dadu_attr_qp_layer_offset[ dpth ][ qc ] specifies QP offsets used for the primary (qc = 0) and any secondary (qc = 1) attribute components.  Each offset applies to transform coefficients at depth dpth of the LoD hierarchy or RAHT tree.  If the LoD hierarchy or RAHT tree has a greater number of levels than dadu_attr_qp_layer_cnt_minus1 + 1, dadu_attr_qp_layer_offset[dadu_ attr_qp_layer_cnt_minus1 ][ qc ] also specifies the QP offsets for transform coefficients at a depth greater than dadu_attr_qp_layer_cnt_minus1.

NOTE: Dependent attribute data unit header has no IDs to indicate its parent data unit. They can be inherited from its corresponding dependent geometry data unit header.

### Context Reference Signalling [213]

In the signalling of the attribute context reference, it is proposed to provide options of explicit and implicit signalling to provide general solution for future applications.

In the implicit signalling case, the context reference of the current attribute FGS is inherited from the context reference of the corresponding geometry FGS whose layer_group_id and the subgroup_id is identical to those ids in the dependent attribute data unit header. In other words, the ref_layer_group_id and ref_subgroup_id of an attribute FSG are induced by the ref_layer_group_id and ref_subgroup_id of a geometry FGS with layer_group_id and the subgroup_id signaled in the attribute FGS.

To find reference of attr-FGS A (layer_group_id = a, subgroup_id = a’) - Find corresponding geometry, geom-FGS A, with layer_group_id = a and subgroup_id = a’ - Find the reference ids of geom-FGS A, ref_layer_group_id = b, ref_subgroup_id = b’, from the dependent geometry data unit header - Find the attr-FGS B, the reference of attr-FGS A, whose layer_group_id and the subgroup_id are identical to ref_layer_group_id = b, ref_subgroup_id = b’

When the reference attribute FGS is not present, the context reference indicator ref_layer_group_id and ref_subgroup_id of the current attribute FGS is deduced from the ref_layer_group_id and ref_subgroup_id of the reference geometry FGS of a geometry FGS that is corresponding to the current attribute FGS.

When reference of attr-FGS B is missing, additional process is conducted - Find geom-FGS B, with layer_group_id = b and subgroup_id = b’ - Find the reference ids of geom-FGS B, ref_layer_group_id = c, ref_subgroup_id = c’, from the dependent geometry data unit header. - Find the attr-FGS C, the reference of attr-FGS B, by using the same reference ids ref_layer_group_id = c, ref_subgroup_id = c’

In the explicit indication of the context reference, the reference indicator is signaled in the dependent attribute data unit header. When aps_attr_ref_id_present_flag is enabled, ref_layer_group_id and ref_subgroup_id are signaled in the dependent attribute data unit.

Table 21: Updates in the attribute parameter set.

| attr_parameter_set( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| aps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| if(layer_group_enabled_flag) { |  |
| aps_attr_ref_id_present_flag | u(1) |
| } |  |
| …… |  |

Table 22: Updates in the dependent attribute data unit header.

| dependent_attribute_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if(aps_attr_ref_id_present_flag) |  |
| attr_ref_layer_group_id | u(8) |
| if(subgroup_enabled_flag[layer_group_id]) { |  |
| attr_ref_subgroup_id | u(16) |
| attr_context_reference_indication_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

aps_attr_ref_id_present_flag equal to 1 indicate that the context reference of the attribute slices will be indicated by the attr_ref_layer_group_id and attr_ref_subgroup_id and the context state usage in the followed attribute slices will be indicated by the attr_context_reference_indication_flag in the dependent attribute data unit header. When aps_attr_ref_id_present_flag is equal to 0, the context reference and the context state usage will be inherited from the geometry slice that is corresponding to the layer_group_id and subgroup_id of the current dependent attribute slice.

attr_ref_layer_group_id specifies the indicator of the layer-group identifier of the context reference of the current dependent attribute data unit. The range of the attr_ref_layer_group_id shall be in the range of 0 to the attr_layer_group_id of the current dependent attribute data unit.

attr_ref_subgroup_id specifies the indicator of the reference subgroup of the layer-group indicated by attr_ref_layer_group_id. The range of the attr_ref_subgroup_id shall be in the range of 0 to num_subgroup_id_minus1 of the layer-group indicated by rattr_ef_layer_group_id.

attr_context_reference_indication_flag equal to 1 indicate that the context state of the current dependent attribute slice will be inherited to one or more followed dependent attribute slices. attr_context_reference_indication_flag equal to 0 indicate the context state of the current dependent slice will not be inherited to the followed dependent slice.

Decoders could manage the context buffer by using attr_context_reference_indication_flag. When attr_context_reference_indication_flag equal to 1, the context state of the current dependent slice is saved to the context buffer at the end of the decoding. When attr_context_reference_indication_flag equal to 0, the context state of the current dependent slice is not saved to the context buffer.

### Subgroup Quantization Weight [214]

In the predicting transform, the quantization weights are used to give credits to the nodes used in the prediction. In G-PCCv1, the weights are derived by the sum of weights of the related nodes which reference the current node as a neighbor node. Following the concept, the weight of a node in a subgroup is calculated with the following constraint.

- Subgroup quantization weight: sum of weights of the related nodes which reference the current node as the neighbor node where the current node and the neighbor nodes are in the same subgroup.

This approach guarantees the decoder to produce the subgroup weight that is identical to the weight in the encoder for both full FGSs and missing FGSs. However, the weights might be much smaller than the expected weight and the difference is getting worse for higher LoDs. To alleviate the difference, subgroup weight adjustment is proposed for each subgroup and each node.

- Subgroup quantization weight adjustment :

weight(m) += alpha * numRefNodes(m) / maxNumRefNodes + beta

where weight(m) is the subgroup weight of the m-th node, numRefNodes(m) is the number of related nodes of the m-th node, and maxNumRefNodes is the maximum number of related nodes in the current subgroup.

In the encoder, the weight adjustment coefficients alpha and beta could be determined by the maximum value of out-of-subgroup weights subtracted by beta and by the average of out-of-subgroup weights of the subgroup leaf nodes, respectively [214]. Alternatively, the parameters could be generated by Least Square Method [215].

Table 23: Updates in the attribute data unit header.

| attribute_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if(layer_group_enabled_flag) { |  |
| subgroup_weight_adj_coeff_a_bits_minus1 | ue(v) |
| subgroup_weight_adj_coeff_b_bits_minus1 | ue(v) |
| subgroup_weight_adj_coeff_a | ue(v) |
| subgroup_weight_adj_coeff_b | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

subgroup_weight_adj_coeff_a_bits_minus1 and subgroup_weight_adj_coeff_b_bits_minus1 plus 1 indicate the size of the subgroup_weight_adj_coeff_a and subgroup_weight_adj_coeff_b, respectively.

subgroup_weight_adj_coeff_a and subgroup_weight_adj_coeff_b indicate the coefficients to adjust the weights in the current subgroup.

Table 24: Updates in the dependent attribute data unit header.

| dependent_attribute_data_unit_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| subgroup_weight_adjustment_enabled_flag | u(1) |
| if(subgroup_weight_adjustment_enabled_flag) { |  |
| subgroup_weight_adj_coeff_a | ue(v) |
| subgroup_weight_adj_coeff_b | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# References

## Predictive Geometry Coding

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][New] Improved Quantization of Azimuthal Angle in Predictive Geometry Coding, ISO/IEC JTC1/SC29/WG7 m55979, January 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][EE13.51] Report on Predictive Geometry Improvement, ISO/IEC JTC1/SC29/WG7 m56482, April 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][New]EE13.51 related - Improved Coding of Azimuthal Angle Residual, ISO/IEC JTC1/SC29/WG7 m56741, April 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][New]EE13.51 related - Improved Coding of The Number of Azimuthal Angle Steps, ISO/IEC JTC1/SC29/WG7 m56742, April 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][New]EE13.51 related - Improved Radius Residual Sign Coding and Improved Points Ordering, ISO/IEC JTC1/SC29/WG7 m56743, April 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][New]EE13.51 related - Improved List of Predictors, ISO/IEC JTC1/SC29/WG7 m56744, April 2021.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][EE13.51] Report on Predictive Geometry improvement, ISO/IEC JTC1/SC29/WG7 m57092, July 2021.

- S. Gao, S. Lasserre, J. Taquet, M.-L. Champel, [GPCC][EE13.51 related] Improved Coding of the Magnitude of Radius Residual, ISO/IEC JTC1/SC29/WG7 m58719, Jan. 2022.

- S. Gao, J. Taquet, S. Lasserre, [GPCC][EE13.51] Report on predictive geometry improvement, ISO/IEC JTC1/SC29/WG7 m59297, Apr. 2022.

- S. Gao, J. Taquet, S. Lasserre, M.-L. Champel, [G-PCC][EE13.51] Test 1 Report on improving lossy predictive tree by disabling cartesian residual coding, ISO/IEC JTC1/SC29/WG7 m59989, July 2022.

- S. Gao, J. Taquet, S. Lasserre, M.-L. Champel, [G-PCC][EE13.51 related] Improving lossy predictive tree by disabling cartesian residual coding, ISO/IEC JTC1/SC29 WG7 m59483, April 2022.

- J. Taquet, S. Lasserre, S. Gao, M.-L. Champel, [G-PCC][EE13.51 related] Alignment of previous adoptions with specification conventions and minor corrections, ISO/IEC JTC1/SC29 WG7 m60150, July 2022.

- H. Le Floch, N. Ouedraogo, F. Tannhauser, P. Onno, [GPCC] Truncated Unary for predIdx, ISO/IEC JTC1/SC29 WG7 m60236, July 2022.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][New] Removing parsing dependence in predictive geometry coding, ISO/IEC JTC1/SC29 WG7 m63252, Apr. 2023.

- Y. Hanaoka, C. Nakatsuka, K. Unno, K. Nonaka, [G-PCC][EE13.51]Test5: Improved radius residual coding by the threshold of the number of azimuthal angle steps, ISO/IEC JTC1/SC29 WG7 m64086, July 2023.

- Y. Hanaoka, C. Nakatsuka, K. Unno, K. Nonaka, [G-PCC][EE13.51-related][New] Improved radius residual coding by the threshold of the number of azimuthal angle steps, ISO/IEC JTC 1/SC 29/WG 7 m62988, Apr. 2023.

## Octree Geometry Coding

- S. Lasserre, S. Gao, [G-PCC][new] Improvement of the angular mode in IDCM, ISO/IEC JTC1/SC29/WG7 m56471, April 2021.

- S. Gao, S. Lasserre, J. Taquet, [G-PCC][New] Improvement of the azimuthal IDCM coding in TMC3, ISO/IEC JTC1/SC29/WG7 m56472, April 2021.

- S. Gao, S. Lasserre, J. Taquet, [G-PCC][New] Context enhancement of the azimuthal IDCM coding in TMC3, ISO/IEC JTC1/SC29/WG7 m56473, April 2021.

- S. Gao, S. Lasserre, J. Taquet, M.-L. Champel, [G-PCC][EE13.51] Report on Octree Geometry improvement, ISO/IEC JTC1/SC29/WG7 m57093, July 2021.

- W. Zhang, T. Tian, F. Yang, M.-L. Champel, S. Gao, [G-PCC] [New] On planar flag signalling, ISO/IEC JTC1/SC29/WG7 m58237, Online, Oct. 2021.

- W. Zhang, T. Tian, F. Yang, M.-L. Champel, S. Gao, [G-PCC] EE13.57 Test 1 Report on planar flag signalling, ISO/IEC JTC1/SC29/WG07 m58808, Jan. 2022.

- L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz, [G-PCC][New proposal] Disabling planar mode for IDCM nodes, ISO/IEC JTC1/SC29/WG7 m58129, Oct. 2021.

- L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz, [G-PCC] EE13.57 Test 2: Disable planar mode for IDCM coded nodes when the angular information is available, ISO/IEC JTC1/SC29/WG7 m58964, Jan. 2022.

- S. Lasserre, J. Taquet, [GPCC] Report of EE 13.58 on neighbour-based occupancy of octree, ISO/IEC JTC1/SC29/WG7 m59264, Apr. 2022.

- S. Lasserre, J. Taquet, [GPCC][EE 13.58] Code and documentation for new octree based on dynamic OBUF, ISO/IEC JTC1/SC29/WG7 m59263, Apr. 2022.

- S. Lasserre, [GPCC][new] On further improving the coding of neighbour-based occupancy of octree, ISO/IEC JTC1/SC29/WG7 m58558, Jan. 2022.

- S. Lasserre, [GPCC][new] On improving the OBUF scheme: dynamic OBUF, ISO/IEC JTC1/SC29/WG7 m58559, Jan. 2022.

- Z. Wang, S. Wan, [G-PCC][EE 13.57] Test 1 Report on planar mode, ISO/IEC JTC1/SC29/WG7 m59533, Apr. 2022.

- Z. Wang, S. Wan, [G-PCC] [New Proposal] On improving the eligibility of planar mode, ISO/IEC JTC1/SC29/WG7 m59001, Jan. 2022.

- S. Lasserre, [GPCC][EE13.58 related] On removing the Z coding path in the new octree framework, ISO/IEC JTC1/SC29/WG7 m59975, July 2022.

- S. Lasserre, J. Taquet, [GPCC] Report for EE 13.58 on neighbor-based occupancy of octree, ISO/IEC JTC1/SC29/WG7 m60655, Oct. 2022.

- S. Lasserre, [GPCC][EE13.58 related] On improving and reorganizing the contextual neighbour-based information in new octree, ISO/IEC JTC1/SC29/WG7 m59976, July 2022.

- S. Lasserre, [GPCC] A common dynamic OBUF class for octree and TriSoup, ISO/IEC JTC1/SC29/WG7 m60697, Oct. 2022.

- S. Lasserre, [GPCC][new] On low memory footprint dynamic OBUF, ISO/IEC JTC1/SC29/WG7 m61583, Jan. 2023.

- T. Tian, S. Hao, Z. Wang, S. Wan, W. Zhang, F. Yang, [G-PCC][EE13.50] Test 2: initialization of dynamic OBUF, ISO/IEC JTC1/SC29/WG7 m61584, Jan. 2023.

- S. Hao, S. Wan, T. Tian, W. Zhang, F. Yang, [G-PCC] [EE13.67] Report on setting probability bounds of true coders in Dynamic OBUF, ISO/IEC JTC1/SC29/WG7 m62547, Apr. 2023.

- S. Hao, S. Wan, T. Tian, W. Zhang, F. Yang, [G-PCC] [New Proposal] On setting adaptive boundaries for probability updating of binary coders in OBUF, ISO/IEC JTC1/SC29/WG7 m61593, Jan. 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, C. Ma, H. Wei, D. Wang, [G-PCC][EE 13.57 Test3] Report on improved planar mode for octree-based G-PCC, ISO/IEC JTC1/SC29/WG7 m62669, Apr. 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, C. Ma, H. Wei, D. Wang, [G-PCC][EE 13.57 related] Improved planar mode for octree-based GPCC, ISO/IEC JTC1/SC29/WG7 m61899, Jan. 2023.

- W. Wang, Y. Xu, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC][EE13.65] Report on Potential Improvements of Octree-coding for Spin-LIDAR Sequences, ISO/IEC JTC1/SC29/WG7 m62761, Apr. 2023.

- W. Wang, Y. Xu, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC][New proposal] Z coordinates Improvement for spin-LIDAR sequences, ISO/IEC JTC1/SC29/WG7 m61917, Jan. 2023.

- Shuo Gao, Champel Mary-Luc, [G-PCC][new] Improve dynamic OBUF LUT for octree coding, ISO/IEC JTC1/SC29/WG7 m66398, Jan. 2024.

## Inter Prediction for Geometry Coding

- L. Pham Van, K. Cao, A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz, [G-PCC][EE13.2-related] Improved Global Motion Estimation for G-PCC, ISO/IEC JTC1/SC29/WG7 m56113, January 2021.

- L. Pham Van, A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz, [G-PCC] EE13.2 Test 1.1: Global motion using two-threshold classification, ISO/IEC JTC1/SC29/WG7 m56818, April 2021.

- Exploratory model for inter-prediction in G-PCC, ISO/IEC JTC1/SC29 WG11 N18096, Macau, CN, October 2018.

- A. K. Ramasubramonian, B. Ray, L. Pham Van, G. Van der Auwera, M. Karczewicz, [G-PCC][New] Inter prediction with predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m56117, January 2021.

- A. K. Ramasubramonian, L. Pham Van, G. Van der Auwera, M. Karczewicz, 	[G-PCC] EE13.2 report on inter prediction, Test 2, ISO/IEC JTC1/SC29/WG7 m56839, April 2021.

- A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz, [G-PCC][EE13.2-related] Additional results for inter prediction for predictive geometry, ISO/IEC JTC1/SC29/WG7 m56841, April 2021.

- Y.-Z. Xu, L. Zhang, K. Zhang, [G-PCC][EE13.2 related][New Proposal] Encoding complexity reduction for inter-EM, ISO/IEC JTC1/SC29/WG7 m57320, July 2021.

- L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz, [G-PCC][EE13.2 Test 1] InterEM with planar mode enabled, ISO/IEC JTC1/SC29/WG7 m57286, July 2021.

- J. Kim, K. Kim, J. Y. Lee, H. Kwon, J. Seo, [G-PCC][EM related] Report of EE13.2 on inter prediction test 2, ISO/IEC JTC1/SC29/WG7 m57485, July 2021.

- Y. Park, H. Oh, S. Lee, H. Hur, [G-PCC][New] Dependent entropy frame coding in InterEM, ISO/IEC JTC1/SC29/WG7 m56590, April 2021.

- Y. Park, H. Hur, [G-PCC][EE13.2 Test 3] Report on dependent entropy frame coding, ISO/IEC JTC1/SC29/WG7 m57116, July 2021.

- K. L. Loi, T. Nishi, T. Sugio, [G-PCC][New]Inter Prediction for Improved Quantization of Azimuthal Angle in Predictive Geometry Coding, ISO/IEC JTC1/SC29/WG7 m57351, July 2021.

- A. K. Ramasubramonian, L. Pham Van, G. Van der Auwera, M. Karczewicz, [G-PCC][New proposal] Improvements to inter prediction using predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m57299, July 2021.

- EE13.2 Test 1: InterEM version 3 with angular mode enabled, ISO/IEC JTC1/SC29/WG7 m58126, Oct. 2021.

- W. Wang, Y.-Z. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New Proposal] Suggested fix for global motion matrix and segment thresholds coding, ISO/IEC JTC1/SC29/WG7 m58019, Oct. 2021.

- A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz, [G-PCC][New proposal] Results on inter prediction for predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m59650, Apr. 2022.

- L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz, [G-PCC][Inter-EM] Inter prediction for octree geometry coding: evaluation and refinements, ISO/IEC JTC1/SC29/WG7 m59619, Apr. 2022.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC] EE13.2 Test 6 Report on inter prediction with dynamic OBUF scheme, ISO/IEC JTC1/SC29/WG7 m60357, July 2022.

- H. Hur, [G-PCC][EE13.2] Report on Unified Partition Method for Motion Compensation (Test7), ISO/IEC JTC1/SC29/WG7 m60189, July 2022.

- R. Hooda, A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][New proposal] On conversion of cartesian to spherical coordinates in inter prediction, ISO/IEC JTC1/SC29/WG7 m60358, July 2022.

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE 13.2 Test2] Report on Bi-direction Inter Prediction, ISO/IEC JTC1/SC29/WG7 m61084, Oct. 2022.

- Y.-Z. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New Proposal] Bi-direction Inter Prediction for G-PCC, ISO/IEC JTC1/SC29/WG7 m58013, Oct. 2021.

- K. L. Loi, T. Nishi, T. Sugio, [G-PCC][EE13.2] Report on Inter Prediction Test 8 for Predictive Geometry, ISO/IEC JTC1/SC29/WG7 m61586, Jan. 2023.

- K. L. Loi, T. Nishi, T. Sugio, [G-PCC][New] Inter Prediction for Predictive Geometry used in conjunction with Additional Global Motion Compensated Reference Frame, ISO/IEC JTC1/SC29/WG7 m61019, Oct. 2022.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC] Thoughts on EE13.2 Test 8 - inter prediction in predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m62218, Jan. 2023.

- K. L. Loi, T. Nishi, T. Sugio, [G-PCC][EE13.2] Report on Inter Prediction Test 6 for Predictive Geometry, ISO/IEC JTC1/SC29/WG7 m62512, Apr. 2023.

- S. Hao, S. Wan, T. Tian, W. Zhang, F. Yang, [G-PCC][New proposal] Modification on inter entropy continuation, ISO/IEC JTC1/SC29/WG7 m63162, Apr. 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, C. Ma, H. Wei, D. Wang, [G-PCC][EE 13.64] Report on IDCM coding for inter octree-based G-PCC, ISO/IEC JTC1/SC29/WG7 m64024, July 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, C. Ma, H. Wei, D. Wang, [G-PCC][EE 13.57 related] Improved IDCM coding of inter octree-based GPCC, ISO/IEC JTC1/SC29/WG7 m61900, Jan. 2023.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][New] Fixes and improvements to G-PCC, ISO/IEC JTC1/SC29/WG7 m65409, October 2023.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][EE13.2] Test 5 on inter prediction buffers in predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m66466, Jan. 2024.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC] Improvements to G-PCC Ed. 2, ISO/IEC JTC1/SC29/WG7 m66564, Jan. 2024.

- Y. Yu, H. Yu, Z. Sun, V. Zakharchenko, D. Wang,	[G-PCC][New proposal] On reference frame for inter prediction, ISO/IEC JTC1/SC29/WG7 m67873, Apr. 2024.

- S. Lasserre, J. Taquet, [GPCC][EE13.60 Test2b] Improved inter predictive information for occupancy tree, ISO/IEC JTC1/SC29/WG7 m67004, Apr. 2024. (GeS-TM)

- R. Huang, W. Zhang, S. Hao, S. Wan, F. Yang, [G-PCC][EE13.60 Test2a] On the inter context design of the octree coding for GeS-TM, ISO/IEC JTC1/SC29/WG7 m67540, Apr. 2024. (GeS-TM)

- H. Hur, J. Lee, [G-PCC][EE13.2] Report on Inter Prediction (Test5), ISO/IEC JTC1/SC29/WG7 m68325, July 2024.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC] On inter prediction syntax elements in G-PCC, ISO/IEC JTC1/SC29/WG7 m69133, July 2024.

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][EE13.2] Test 3 on inter prediction buffers in predictive geometry coding, ISO/IEC JTC1/SC29/WG7 m69040, July 2024.

## Inter Prediction for Attribute Coding

- Y.-Z. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New Proposal] Bi-direction Inter Prediction for G-PCC, ISO/IEC JTC1/SC29/WG7 m58013, Oct. 2021. (attribute coding part)

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE 13.2 Test 6] Report on Bi-direction Inter Prediction, ISO/IEC JTC1/SC29/WG7 m58889, Jan. 2022. (attribute coding part)

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE 13.2 Test 3] Report on Attribute Inter Prediction for Predictive Geometry Coding, ISO/IEC JTC1/SC29/WG7 m60202, July 2022.

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New proposal] Attribute Inter Prediction for Predictive Geometry Coding Mode in Inter-EM, ISO/IEC JTC1/SC29/WG7 m59612, Apr. 2022.

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE13.2 Test 6] Report on Inter Prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29/WG7 m61886, Jan. 2023.

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New proposal] Inter-Prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29/WG7 m61083, Oct. 2022.

- H. Hur, Y. Park, [G-PCC][EE13.2] Report on inter prediction (Test7), ISO/IEC JTC1/SC29/WG7 m62704, Apr. 2023.

- H. Hur, Y. Park, [G-PCC][EE13.2-related] on dependent entropy frame coding for attribute, ISO/IEC JTC1/SC29/WG7 m61916, Jan. 2023.

- A. K. Ramasubramonian, [G-PCC] Cross-check of EE13.2 Test 7 on dependent frame entropy coding for attributes, ISO/IEC JTC1/SC29/WG7 m63493, Apr. 2023 (fixes).

- Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2] Report on RAHT Inter Prediction (Test5.1), ISO/IEC JTC1/SC29/WG7 m62706, Apr. 2023.

- Y.-Z. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New proposal] Improvements to Inter-Prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29/WG7 m61888, January 2023.

- N. Iguchi, T. Sugio, [G-PCC][EE13.2] Report on Inter prediction for RAHT attribute coding (Test5.2), ISO/IEC JTC1/SC29/WG7 m62760, Apr. 2023.

- N. Iguchi, T. Sugio, [G-PCC][EE13.2Test6 related][New] Coefficient Inter Prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29/WG7 m61945, Jan. 2023.

- Y. Xu, W. Wang, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New proposal] Improvements to Attribute Inter-Prediction With Slices Partition, ISO/IEC JTC1/SC29/WG7 m62710, Apr. 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, D. Wang, [G-PCC][EE13.2 related][New proposal] Attribute Inter Prediction for predLifting transform coding, ISO/IEC JTC1/SC29/WG7 m63014, Apr. 2023.

- Y.-Z. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2 Test2] Report on bi-prediction under predictive geometry coding mode, ISO/IEC JTC1/SC29/WG7 m64045, July 2023.

- Y.-Z. Xu, W. Wang, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC][EE13.2 Test6] Report on improvements to RAHT prediction, ISO/IEC JTC1/SC29/WG7 m64044, July 2023.

- Y.-Z. Xu, W. Wang, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC][EE13.2 related][New proposal] Improvements to RAHT Prediction, ISO/IEC JTC1/SC29/WG7 m62712, April 2023.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.68 Test2] Report on RAHT Temporal Filtering, ISO/IEC JTC1/SC29/WG7 m64020, July 2023.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][New Proposal] Temporal Filtering for RAHT Coding, ISO/IEC JTC1/SC29/WG7 m63003, April 2023.

- Z. Sun, Y. Yu, V. Zakharchenko, H. Yu, D. Wang, [G-PCC][EE 13.2] Report on inter-prediction for RAHT transform coding, ISO/IEC JTC1/SC29/WG7 m64026, July 2023.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [GPCC][New] On Prediction Mode Selection for RAHT and Harmonization with Temporal Filtering, ISO/IEC JTC1/SC29/WG7 m65008, October 2023.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2 Test3] Report on RAHT Inter-Eligibility, ISO/IEC JTC1/SC29/WG7 m64795, October 2023.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][New] Eligibility Criterion for Inter-RAHT coding, ISO/IEC JTC1/SC29/WG7 m64303, July 2023.

- Y. Hanaoka, K. Unno, K. Nonaka, [G-PCC][New]: Improvements on RAHT inter prediction, ISO/IEC JTC1/SC29/WG7 m66393, ISO/IEC JTC1/SC29/WG7 m66393, Jan. 2024.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2-Related][New] Resampling for RAHT Inter-Prediction, ISO/IEC JTC1/SC29/WG7 m67095, Apr. 2024.

- Y. Xu, K. Zhang, L. Zhang, [G-PCC][New Proposal] Improvements to the RAHT inter prediction, ISO/IEC JTC1/SC29/WG7 m67447, Apr. 2024.

- Z. Sun, Y. Yu, H. Yu, D. Wang, [G-PCC][EE13.2-Test 5] Report global motion-compensated inter-prediction for attribute coding, ISO/IEC JTC1/SC29/WG7 m67568, Apr. 2024.

- Z. Sun, Y. Yu, H. Yu, D. Wang, [G-PCC][EE 13.2-Test1] Report on Bi-directional inter-prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29/WG7 m67562, Apr. 2024.

- Z. Sun, Y. Yu, H. Yu, D. Wang, [G-PCC][EE13.2-Test2] Report on inter-prediction for RAHT attribute coding, ISO/IEC JTC1/SC29/WG7 m67564, Apr. 2024.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.2-Test7] Report on Reference Node Resampling for lossless RAHT, ISO/IEC JTC1/SC29/WG7 m68282, July 2024.

- Y. Xu, K. Zhang, L. Zhang, [G-PCC][new] Improvements to the syntax elements transmission, ISO/IEC JTC1/SC29/WG7 m68407, July 2024.

- Z. Sun, Y. Yu, H. Yu, D. Wang, [G-PCC][EE13.2-Test 4] Report global motion-compensated inter-prediction for attribute coding, ISO/IEC JTC1/SC29/WG7 m68646, July 2024. (clipping aspect)

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC] Improvements to TMC13/G-PCC, ISO/IEC JTC1/SC29/WG7 m69041, July 2024.

- J. Taquet, S. Lasserre, [G-PCC][EE13.60 related] On allowing motion compensated attributes projection without using dual motion for octree, ISO/IEC JTC1/SC29/WG7 m68327, July 2024. (GeS-TM)

- C. Ma, Y. Yu, H. Yu, D. Wang, [G-PCC][EE13.60-Test5]Report on a method to enhance inter RAHT average prediction, ISO/IEC JTC1/SC29/WG7 m68397, July 2024. (GeS-TM)

## Attribute Coding

- H. Yuan, X. Wang, G. Sun, L. Wang, M. Li, [G-PCC][New] Adaptive Quantization for LoD-based Attribute Predicting Transform Coding, ISO/IEC JTC1/SC29/WG7 m55860, January 2021.

- H. Yuan, X. Wang, G. Sun, L. Wang, M. Li, [G-PCC][EE13.49] Report on Adaptive Quantization for LoD Attributes, ISO/IEC JTC1/SC29/WG7 m56650, April 2021.

- W. Zhang, T. Tian, L. Yang, F. Yang, M.-L. Champel, S. Gao, [G-PCC][EE13.49] Report on Attribute LoD Prediction, ISO/IEC JTC1/SC29/WG7 m58236, Oct. 2021.

- W. Zhang, L. Yang, F. Yang, T. Tian, M-L. Champel, S. Gao, [G-PCC][New]Neighbor Search Improvements for Attribute LoD Prediction, ISO/IEC JTC1/SC29/WG07 m57324, July 2021.

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.59] Report on Low Latency Attribute Coding and Slicing, ISO/IEC JTC1/SC29/WG07 m60205, July 2021.

- W. Wang, Y. Xu, K. Zhang, L. Zhang, “[G-PCC][New Proposal] Flexibility Enhancement of Low Latency Attribute Coding inside Slice”, ISO/IEC JTC1/SC29/WG7 m59615, April 2022.

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.61] Report on Extended Transform Domain, ISO/IEC JTC1/SC29/WG7 m61090, Oct. 2022.

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][New Proposal] Sub-Node-based Prediction in Transform Domain for RAHT, ISO/IEC JTC1/SC29/WG7 m60203, July 2022.

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.61 related][New Proposal] Complexity Reduction for RAHT, ISO/IEC JTC1/SC29/WG7 m61089, Oct. 2022.

- W. Zhang, Z. Liu, J. Lu, F. Yang, M.-L. Champel, S. Gao, [G-PCC][New] On disabling Transform Domain Prediction of RAHT, ISO/IEC JTC1/SC29/WG7 m61015, Oct. 2022. (see common solution with m61089)

- A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz, [G-PCC][New proposal] Fixes and improvements of G-PCC, ISO/IEC JTC1/SC29/WG7 m61223, Oct. 2022.

- G. Sandri, F. Thudor, B. Chupeau, [G-PCC][EE 13.61] On RAHT reconstruction buffer, ISO/IEC JTC1/SC29/WG7 m61569, Jan. 2023.

- G. Sandri, F. Thudor, B. Chupeau, On RAHT reconstruction buffer, ISO/IEC JTC1/SC29/WG7 m61151, Oct. 2022.

- B. Vishwanath, Y. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC][EE 13.63] Report: On RAHT Coefficient Quantization, ISO/IEC JTC1/SC29/WG7 m61829, Jan. 2023.

- B. Vishwanath, Y. Xu, W. Wang, K. Zhang, L. Zhang, [G-PCC] [New Proposal] On RAHT Coefficient Quantization, ISO/IEC JTC1/SC29/WG7 m61159, Oct. 2022.

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][New] A simplification and a fix for RAHT coding, ISO/IEC JTC1/SC29/WG7 m63002, Apr. 2023 (fix).

- C. D. Han, T. Sugio, N. Iguchi, [G-PCC][EE13.13] Report on Lossless extension to RAHT, ISO/IEC JTC1/SC29/WG7 m64118, July 2023.

- C. D. Han, T. Sugio, N. Iguchi, [G-PCC] [New] Lossless extension to RAHT, ISO/IEC JTC1/SC29 WG7 Doc. m63136, April 2023.

- W. Zhang, D. Wang, Y. Zhang, F. Yang, N. Wang, Z. Lv, [G-PCC] [EE 13.69] Report on cross-attribute prediction, ISO/IEC JTC1/SC29 WG7 Doc. m65087, October 2023.

- W. Zhang, D. Wang, F. Yang, N. Wang, Z. Lv, [G-PCC][New] Improved LoD-based attribute coding using cross-type attribute correlation, ISO/IEC JTC1/SC29 WG7 Doc. m64113, July 2023.

- Y. Xu, B. Vishwanath, K. Zhang, L. Zhang, [G-PCC] Fix and improvement to some HLS signalling, ISO/IEC JTC1/SC29 WG7 Doc. m65058, October 2023.

- N. Iguchi, T. Sugio, [G-PCC][new] Improve for high bit-depth attribute coding, ISO/IEC JTC1/SC29 WG7 Doc. m65387, October 2023.

- W. Zhang, X. Liu, F. Yang, Z. Lv, [G-PCC][EE13.69] Inter-chroma channel attribute prediction for RAHT, ISO/IEC JTC1/SC29 WG7 Doc. m66183, Jan. 2024.

- K. Unno, Y. Hanaoka, K. Nonaka, K. Kawamura, [G-PCC][New] Improvement on spec for RAHT intra neighbor search, ISO/IEC JTC1/SC29 WG7 m66363, Jan. 2024.

- B. Vishwanath, W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][13.60-Test2] Report on Sample Domain Prediction for RAHT, ISO/IEC JTC1/SC29 WG7 m66152, Jan. 2024. (GeS-TM)

- B. Vishwanath, W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.69-Test2] Report on Sample Domain Prediction for RAHT, ISO/IEC JTC1/SC29 WG7 m67094, Apr. 2024. (TMC13)

- Z. Sun, Y. Yu, H. Yu, D. Wang, [G-PCC][EE13.61-Test2] Report on Intra-Prediction for RAHT Attribute Coding, ISO/IEC JTC1/SC29 WG7 m67565, Apr. 2024.

- W. Zhang, X. Liu, F. Yang, Z. Lv, [G-PCC][EE13.69 Test 1] Inter-chroma channel prediction for RAHT, ISO/IEC JTC1/SC29 WG7 m67541, Apr. 2024. (GeS-TM)

- W. Zhang, X. Liu, F. Yang, Z. Lv, [G-PCC][EE13.60-related][New] Context reduction for RAHT entropy coding, ISO/IEC JTC1/SC29 WG7 m67542, Apr. 2024. (GeS-TM)

- W. Zhang, J. Wang, X. Liu, Z. Lv, [G-PCC][EE13.60-related][New] On RAHT prediction mode signalling, ISO/IEC JTC1/SC29 WG7 m67628, Apr. 2024. (GeS-TM)

- J. Wang, X. Liu, W. Zhang, F. Yang, Z. Lv, [G-PCC][EE13.60-related][New] Fixes and Improvements to RAHT, ISO/IEC JTC1/SC29 WG7 m67627, Apr. 2024. (TMC13 and GeS-TM)

- J. Lee, H.-M. Oh, H. Hur, M. Lee, [G-PCC][EE13.69-Test3] Report on the canonical geometry order-based periodic subsampling method for LOD generation, ISO/IEC JTC1/SC29 WG7 m68770, July 2024.

- S. Lasserre, J. Taquet, [GPCC][GeSTM] On RAHT optimization for GeSTMv6-rc2, ISO/IEC JTC1/SC29 WG7 m68253, July 2024. (GeS-TM, TMC13)

- B. Vishwanath, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.69-Test1] Report on Cross Component Residual Prediction, ISO/IEC JTC1/SC29 WG7 m68283, July 2024. (GeS-TM)

## TriSoup Coding

- K. Unno, K. Kawamura, [G-PCC][New], Simplification of determining process of projection plane in Trisoup, ISO/IEC JTC1/SC29/WG7 m55952, January 2021.

- K. Unno, K. Kawamura, [G-PCC][EE13.50] Report on Trisoup, ISO/IEC JTC1/SC29/WG7 m56549, April 2021.

- K. Unno, K. Kawamura, [G-PCC][EE13.50 related][New Proposal] Refinement of Trisoup projection plane determination for improving subjective quality, ISO/IEC JTC1/SC29/WG7 m58775, Jan. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 1 Improving TriSoup summary, results and perspective, ISO/IEC JTC1/SC29/WG7 m59288, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 2 Fixes and simplifications, ISO/IEC JTC1/SC29/WG7 m59289, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 3 Adding a residual for the centroid vertex, ISO/IEC JTC1/SC29/WG7 m59290, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 4 Vertex Quantization, ISO/IEC JTC1/SC29/WG7 m59291, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 5 Improved rendering from triangles, ISO/IEC JTC1/SC29/WG7 m59292, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 6 Compression of vertex presence flag and vertex position, ISO/IEC JTC1/SC29/WG7 m59293, Apr. 2022.

- S. Lasserre, [GPCC][TriSoup] Part 7 Latest improvements, ISO/IEC JTC1/SC29/WG7 m59294, Apr. 2022.

- S. Lasserre, [GPCC][EE 13.50 Test 1] Report on improved TriSoup, ISO/IEC JTC1/SC29/WG7 m59973, July 2022.

- S. Gao, J. Taquet, M.-L. Champel, [G-PCC][EE13.50] Test 1 Improvement of voxelization of trisoup model, ISO/IEC JTC1/SC29/WG7 m60797, Oct. 2022.

- J. Taquet, S. Gao, [G-PCC][EE13.50 related] Fixup for trisoup vertices quantization, ISO/IEC JTC1/SC29/WG7 m61436, Oct. 2022.

- S. Lasserre, [GPCC][EE13.50] Report on enhanced edge neighborhood for vertex prediction in TriSoup, ISO/IEC JTC1/SC29/WG7 m61565, Jan. 2023.

- A. Ito, N. Iguchi, T. Sugio, [G-PCC][EE13.50] Test 4: Non-cubic method on Trisoup, ISO/IEC JTC1/SC29/WG7 m61577, Jan. 2023.

- A. Ito, N. Iguchi, T. Sugio, [G-PCC][New] Proposal on Non-Cubic node method for Trisoup, ISO/IEC JTC1/SC29/WG7 m61116, Oct. 2022.

- S. Lasserre, [GPCC][new] On the voxelization of TriSoup triangles, ISO/IEC JTC1/SC29/WG7 m61982, Jan. 2023.

- S. Lasserre, [GPCC][EE13.60] An inter TriSoup scheme based on motion compensated point clouds, ISO/IEC JTC1/SC29/WG7 m61582, Jan. 2023.

- S. Lasserre, [GPCC][EE13.50] Report on the voxelization of TriSoup, ISO/IEC JTC1/SC29/WG7 m62521, Apr. 2023.

- S. Lasserre, [GPCC][EE13.60][EE13.50 related] On optimal centroid vertex positioning in TriSoup, ISO/IEC JTC1/SC29/WG7 m62526, Apr. 2023.

- J. Taquet, S. Gao, [G-PCC][EE13.66] Report on raster scan order for Octree coding, ISO/IEC JTC1/SC29/WG7 m62531, Apr. 2023. (GeS-TM)

- J. Taquet, S. Gao, [G-PCC][New] On raster scan order for Octree coding, ISO/IEC JTC1/SC29/WG7 m62004, Jan. 2023. (GeS-TM)

- J. Taquet, S. Lasserre, [GPCC][EE13.60] Report for EE 13.60 Test 1 on grid alignment, ISO/IEC JTC1/SC29/WG7 m63660, July 2023. (GeS-TM)

- S. Lasserre, [GPCC][EE13.60] On the grid of TriSoup nodes, ISO/IEC JTC1/SC29 WG7 m62527, April 2023. (GeS-TM)

- S. Lasserre, J. Taquet, [GPCC] Report on new architecture for GeS TM v2.0-rc1 and related integrations, ISO/IEC JTC1/SC29 WG7 m63828, July 2023. (GeS-TM)

- G. Sandri, B. Chupeau, F. Thudor, [G-PCC][EE13.60] Report on Test 4 on motion-compensated inter-frame attribute coding scheme for dynamic solid point clouds, ISO/IEC JTC1/SC29 WG7 m64005, July 2023. (GeS-TM)

- G. Sandri, F. Thudor, B. Chupeau, [G-PCC][EE13.60] A motion-compensated inter-frame attribute coding scheme for dynamic solid point clouds, ISO/IEC JTC1/SC29 WG7 m62583, April 2023. (GeS-TM)

- N. Iguchi, T. Sugio, [G-PCC][EE13.60] Report on Test5 : Inter-RAHT prediction for GeS-TM, ISO/IEC JTC1/SC29/WG7 m64218, July 2023. (GeS-TM)

- A. Ito, N. Iguchi, T. Sugio, [G-PCC][EE13.50 Test1] Face vertex for Trisoup, ISO/IEC JTC1/SC29/WG7 m63707, July 2023.

- A. Ito, N. Iguchi, T. Sugio, [G-PCC][New] Face vertex for Trisoup, ISO/IEC JTC1/SC29/WG7 m62563, April 2023.

- S. Lasserre, J. Taquet, [GPCC][EE13.60] Report for EE 13.60 Test 2 on inter skip mode, ISO/IEC JTC1/SC29/WG7 m63661, July 2023. (GeS-TM)

- W. Zhang, J. Wang, X. Liu, F. Yang, N. Wang, Z. Lv, [G-PCC][EE 13.60 Test 4-related] On RAHT prediction mode selection, ISO/IEC JTC1/SC29/WG7 m64112, July 2023. (GeS-TM)

- S. Lasserre, J. Taquet, [GPCC][CTC/EE13.60 related] On activation of TriSoup centroid vertices, ISO/IEC JTC1/SC29/WG7 m64912, October 2023. (GeS-TM)

- A. Ito, N. Iguchi, T. Sugio, [G-PCC][EE13.60] Test5 Face vertex for Trisoup on GeS-TM, ISO/IEC JTC1/SC29/WG7 m65235, October 2023. (GeS-TM)

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.71 Test1] Report on Visual Quality Improvement, ISO/IEC JTC1/SC29/WG7 m66493, Jan. 2024.

- N. Svensson, [G-PCC][EE13.50][Test 2] Reconstructing Non-Closed Surfaces in Trisoup Nodes, ISO/IEC JTC1/SC29/WG7 m67062, Apr. 2024. (TMC13 and GeS-TM)

- W. Zhang, K. Yao, F. Yang, S. Gao, M.-L. Champel, [G-PCC][EE13.71 Test3] An improvement of Trisoup Surface Reconstruction, ISO/IEC JTC1/SC29/WG7 m67539, Apr. 2024. (TMC13 and GeS-TM)

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.50 Test1] Report on Triangle Soup, ISO/IEC JTC1/SC29/WG7 m67834, Apr. 2024. (TMC13 and GeS-TM)

- S. Lasserre, J. Taquet, [GPCC][EE13.72] On versatile geometry quantization of TriSoup for GeSTM, ISO/IEC JTC1/SC29/WG7 m67002, Apr. 2024. (GeS-TM)

- S. Lasserre, J. Taquet, [GPCC][EE13.72] On node sizes not powers of two for GeSTM, ISO/IEC JTC1/SC29/WG7 m67015, Apr. 2024. (GeS-TM)

- R. Huang, W. Zhang, F. Yang, [G-PCC][EE13.60-related][New] On the OBUF scheme for Trisoup vertex presence flag, ISO/IEC JTC1/SC29/WG7 m67544, Apr. 2024. (GeS-TM)

- W. Wang, Y. Xu, K. Zhang, L. Zhang, [G-PCC][EE13.50 Test1] Report on Triangle Soup, ISO/IEC JTC1/SC29/WG7 m68929, July 2024. (TMC13)

- S. Lasserre, J. Taquet, [GPCC][EE13.72] On local quality units for GeSTM, ISO/IEC JTC1/SC29/WG7 m68255, July 2024. (GeS-TM)

- R. Huang, F. Yang, W. Li, Z. Yu, B. Li, [G-PCC][EE13.60 Test 1] On the OBUF Scheme for Trisoup vertex position coding, ISO/IEC JTC1/SC29/WG7 m68285, July 2024. (GeS-TM)

- S. Lasserre, [GPCC][EE13.60] Crosscheck of m68285 for EE13.60 Test1, ISO/IEC JTC1/SC29/WG7 m68350, July 2024. (GeS-TM)

## Entropy Coding

- A. Alshin, H. Hur, V. Shepelev, [G-PCC][New] Simplification of Bypass Mode in TMC13, ISO/IEC JTC1/SC29/WG7 m62220, Jan. 2023.

## Fine Granularity Slicing

- H.-M. Oh, [G-PCC][New Proposal] Layer structure based PCC slice segmentation, ISO/IEC JTC1/SC29 WG11 m49198, Gothenburg, July 2019.

- H.-M. Oh, H. Hur, S. Oh, [G-PCC] Slice segmentation considering coding layers, ISO/IEC JTC1/SC29 WG7 m55350, Online, Oct. 2020.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44] Slice segmentation for spatial access, ISO/IEC JTC1/SC29 WG7 m55996, Online, Jan. 2021.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44] Report on granularity slices, ISO/IEC JTC1/SC29/WG07 m56750, Apr. 2021.

- H.-M. Oh, H. Hur, S. Lee, [G-PCC][EE13.44] Report on fine granularity slices using layer-group structure, ISO/IEC JTC1/SC29/WG07 m57350, July 2021.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44] Report on fine granularity slices using layer-group structure, ISO/IEC JTC1/SC29/WG07 m58119, Oct. 2021.

- S. Sugimoto, T. Miyazawa, Y. Hiwasaki, [G-PCC][EE13.44]Report on granularity slicing and group structure inventory, ISO/IEC JTC1/SC29/WG07 m57112, July 2021.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44] Report on fine granularity slices using layer-group structure, ISO/IEC JTC1/SC29/WG07 m58829, Jan. 2022.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44 related] On context buffer management, ISO/IEC JTC1/SC29/WG07 m58124, Oct. 2021.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44 related] On context buffer management, ISO/IEC JTC1/SC29/WG7 m64265, July 2023

- H.-M. Oh, H. Hur, [G-PCC][EE13.44 related] On number of points limitation, ISO/IEC JTC1/SC29/WG07 m58125, Oct. 2021.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44-related] On EE reference software of geometry layer-group slicing for TMC13v21, ISO/IEC JTC1/SC29/WG7 m63583, Apr. 2023.

- S. Sugimoto, [G-PCC][EE13.44]Attribute coding on fine granularity slicing, ISO/IEC JTC1/SC29/WG07 m59572, April 2022.

- S. Sugimoto, [G-PCC][EE13.44]Attribute coding on fine granularity slicing, ISO/IEC JTC1/SC29/WG07 m61009, October 2022.

- S. Sugimoto, [G-PCC][EE13.44]Attribute coding on fine granularity slicing, ISO/IEC JTC1/SC29 WG11 m65427, October, 2023

- H.-M. Oh, H. Hur, [G-PCC][EE13.44 related] Context reference signaling of attribute layer-group slicing, ISO/IEC JTC1/SC29 WG11 m59800, April 2022

- H.-M. Oh, H. Hur, [G-PCC][EE13.44] Report on attribute layer-group slicing, ISO/IEC JTC1/SC29 WG11 m60273, July 2022.

- S. Sugimoto, [G-PCC][EE13.44]Attribute coding on fine granularity slicing, ISO/IEC JTC1/SC29/WG07 m63107, April 2023.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44][Test 2] Report on occupancy tree depth signaling, ISO/IEC JTC1/SC29/WG07 m68970, July 2024.

- H.-M. Oh, H. Hur, S. Sugimoto, [G-PCC][EE13.44-related] On direct node eligibility condition on layer-group boundary, ISO/IEC JTC1/SC29/WG07 m68975, July 2024.

- H.-M. Oh, H. Hur, [G-PCC][EE13.44][Test 1-1] Report on improved context buffer management, ISO/IEC JTC1/SC29/WG07 m68969, July 2024.

- S. Sugimoto, [G-PCC][EE13.44][Test 1-2] Syntax refinement for fine granularity slice, ISO/IEC JTC1/SC29/WG07 m68979, July 2024.