INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG03 MPEG2025/N24424

April 2025, Online

| Source | Systems |
| --- | --- |
| Status | Working Draft |
| Title | WD of ISO/IEC TR 23009-7 Delivery of CMAF content with DASH |
| Editor | Thomas Stockhammer |

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	v

1	Scope	1

2	Normative references	1

3	Definitions	1

3.1	Terms and definitions	1

Refer to definitions in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions.	1

3.2	Abbreviated terms	1

Refer to abbreviated terms in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions in addition to the above.	1

4	Document Organization	1

5	Overview	2

6	Background	2

6.1	Key Concepts in DASH	2

6.2	Key Concepts in CMAF	4

7	Operational Assumptions	6

8	General mapping considerations	6

8.1	Basic Mapping	6

8.2	On-demand and live content considerations	8

8.3	MPD parameters considerations	8

8.4	DASH Packager Guidance using ISOBMFF KindBox	14

8.4.1	General	14

8.4.2	DASH Descriptors and other child elements	14

8.4.3	DASH Attributes	15

8.5	Segment addressing schemes	17

9	Mapping Media Profiles to DASH Adaptation Sets and Segments	17

9.1	General Considerations	17

9.2	Mapping of a CMAF Presentation Profile	17

9.2.1	Sample entry	18

9.3	Video	18

9.3.1	AVC	18

9.3.2	HEVC	18

9.3.3	SHVC	18

9.4	Audio	18

9.4.1	HE-AAC single bitrate	18

9.4.2	HE-AAC multi bitrate	18

9.5	Subtitles	18

10	Mapping of Encrypted Tracks	18

11	Mapping of CMAF Segment Mode	18

11.1.1	Basic Mapping	18

11.1.2	Switching and Random Access Signalling	18

11.1.3	Low-Latency Mode	18

11.1.4	Events and other delivery boxes	18

11.1.5	Compatibility Considerations	18

12	Mapping of CMAF Track File Mode	18

12.1.1	Basic Mapping	18

12.1.2	Segment Index for CMAF Fragment addressing	18

12.1.3	Events and other delivery boxes	18

12.1.4	Compatibility Considerations	18

13	Mapping of CMAF Chunk Mode	18

13.1.1	Basic Mapping	18

13.1.2	Switching and Random Access Signalling	18

13.1.3	Segment Index for CMAF Fragment addressing	18

13.1.4	Events and other delivery boxes	18

13.1.5	Compatibility Considerations	18

14	Recommendation for External Interoperability Points	19

- Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

ISO/IEC 23009-1 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

ISO/IEC 23009 consists of the following parts, under the general title Information technology — Dynamic adaptive streaming over HTTP (DASH):

- Part 1: Media presentation description and segment formats

- 	Part 2: Conformance and reference software

- 	Part 3: Implementation guidelines [Technical Report]

- 	Part 4: Segment encryption and authentication

- 	Part 5: Server and network assisted DASH

- 	Part 6: DASH with server push and websocket sub protocol

- 	Part 7: Delivery of CMAF content with DASH

Error: Reference source not found

# Scope

This part of Error: Reference source not found recommends best practises for delivery of CMAF content (ISO/IEC 23000-19) with the DASH standard.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

| [CMAF] | Study of DIS of ISO/IEC 23009-1, Information technology — Common Media Application Format for Segmented Media |
| --- | --- |
| [DASH1] | ISO/IEC 23009-1, Information technology — Dynamic adaptive streaming over HTTP-Part: Media presentation description and segment formats |

# Definitions

## Terms and definitions

## Refer to definitions in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions.

## Abbreviated terms

For the purposes of this International Standard, the following abbreviated terms apply.

CMAF	Common Media Application Format

DRM	Digital Rights Management

ISOBMFF	ISO Base Media File Format (as specified by ISO/IEC 14496-12)

## Refer to abbreviated terms in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions in addition to the above.

# Document Organization

TBD

# Overview

This Standard defines guidelines for delivering content generated based on the CMAF specification (ISO/IEC 23000-19) using DASH specification (ISO/IEC 23009-1).

The CMAF specification defines an encoding format for the content. Each media component of the content may be encoded in multiple tracks, grouped in one or more CMAF Switching Sets. However there is no description of how these tracks are related, and how various media components should be delivered and played.

The DASH specification defines segment formats for media content. But it also defines a manifest, Media Presentation Description (MPD) which expresses the relationship of tracks and segments as well as how they are identified as URI resources.

While CMAF delivery entities can be identical to DASH segments, there are multiple ways to package them and/or identified them as resources and described by a MPD. These guidelines recommend some of the most popular delivery schemes for such mapping and delivery.

# Background

## Key Concepts in DASH

DASH defines a manifest format using an XML schema and semantics, as well as constraints on DASH segments that can be addressed for delivery, and synchronized for presentation.

The collection of encoded and deliverable versions of media content and the appropriate description of these form a Media Presentation. Media content is composed of a single or multiple contiguous media content periods in time. Content in different media content periods may be completely independent or certain periods of a Media Presentation may belong to the same Asset, for example a Media Presentation is a collection of main program composed of multiple periods, each assigned to the same Asset, and interleaved with inserted advertisement periods. Each media content period is composed of one or multiple media content components, for example audio components in various languages, different video components providing different views of the same program, subtitles in different language and others. Each media content component has an assigned media content component type, for example audio or video. Same asset over multiple periods may be identified by a DASH descriptor enabling DASH clients to maintain the continuity across periods’ boundaries. Furthermore, sub-assets composing the same asset may also identified using a similar method. For instance, if an asset is composed of multiple video components, sub-assets enable selecting the previously selected video component after an ad insertion.

Each media content component may have several encoded versions, referred to as media streams. Each media stream inherits the properties of the media content, the media content period, the media content component from which it was encoded and in addition it gets assigned the properties of the encoding process such as sub-sampling, codec parameters, encoding bitrate, etc. This describing metadata is relevant for static and dynamic selection of media content components and media streams.

Figure 3 — DASH High-Level Data Model

DASH is based on a hierarchical data model aligned with the presentation in Figure 3. A DASH Media Presentation is described by a Media Presentation Description document. This describes the sequence of Periods (see 5.3.2,[DASH1]) in time that make up the Media Presentation. A Period typically represents a media content period during which a consistent set of encoded versions of the media content is available i.e. the set of available bitrates, languages, captions, subtitles etc. does not change during a Period.

Within a Period, material is arranged into Adaptation Sets (see 5.3.3, [DASH1]). An Adaptation Set represents a set of interchangeable encoded versions of one or several media content components (see 5.3.4/[DASH1]). For example, there may be one Adaptation Set for the main video component and a separate one for the main audio component. If there is other material available, for example captions or audio descriptions, then these may each have a separate Adaptation Set. Material may also be provided in multiplexed form, in which case interchangeable versions of the multiplex may be described as a single Adaptation Set, for example an Adaptation Set containing both the main audio and main video for a Period. Each of the multiplexed components may be described individually by a media content component description.

An Adaptation Set contains a set of Representations (see 5.3.5, DASH1). A Representation describes a deliverable encoded version of one or several media content components. A Representation includes one or more media streams (one for each media content component in the multiplex). Any single Representation within an Adaptation Set is sufficient to render the contained media content components. By collecting different Representations in one Adaptation Set, the Media Presentation author expresses that the Representations represent perceptually equivalent content. Typically, this means that clients may switch dynamically from Representation to Representation within an Adaptation Set in order to adapt to network conditions or other factors. Switching refers to the presentation of decoded data up to a certain time t, and presentation of decoded data of another Representation from time t onwards. If Representations are included in one Adaptation Set, and the client switches properly, the Media Presentation is expected to be perceived seamless across the switch. Clients may ignore Representations that rely on codecs or other rendering technologies they do not support or that are otherwise unsuitable.

Within a Representation, the content may be divided in time into Segments (see 5.3.9 and 6, [DASH1]) for proper accessibility and delivery. In order to access a Segment, a URL is provided for each Segment. Consequently, a Segment is the largest unit of data that can be retrieved with a single HTTP request.

Segments may be further subdivided into Subsegments each of which contains a whole number of complete access units. There may also be media-format-specific restrictions on Subsegment boundaries, for example in the ISO Base Media File Format a Subsegment must contain a whole number of complete movie fragments. If a Segment is divided into Subsegments they are described by a compact Segment index, which provides the presentation time range in the Representation and corresponding byte range in the Segment occupied by each Subsegment. Clients may download this index in advance and then issue requests for individual Subsegments.

Clients may switch from Representation to Representation within an Adaptation Set at any point in the media. However, switching at arbitrary positions may be complex because of coding dependencies within Representations and other factors, potentially requiring parallel download and decoding in the DASH client. It is also desirable to avoid download of 'overlapping' data i.e. media for the same time period from multiple Representations. Usually, switching is simplest at a Stream Access Point (SAP) in the new stream. In order to formalize requirements related to switching DASH defines a codec-independent concept of Stream Access Points and identifies various types of Stream Access Points.

Segmentation and Subsegmentation may be performed in ways that make switching simpler. For example, in the very simplest cases each Segment or Subsegment begins with a SAP and the boundaries of Segments or Subsegments are aligned across the Representations of one Adaptation Set. In this case, switching Representation involves playing to the end of a (Sub)Segment of one Representation and then playing from the beginning of the next (Sub)Segment of the new Representation. The Media Presentation Description and Segment Index provide various indications, which describe properties of the Representations that may make switching simpler. Profiles of this specification may then require these indicators to be set in certain ways, making implementation of clients for those profiles simpler at the cost of requiring the media data to obey the indicated constraints.

## Key Concepts in CMAF

CMAF provides the following concepts:

- CMAF specifies CMAF Tracks containing CMAF Fragments conforming to Media Profiles that can be combined to form CMAF Presentations.  CMAF Tracks can exist as live streams as well as stored files.

- CMAF specifies CMAF Presentations that are encoded conforming to a hypothetical late binding application model that can select multiple independent CMAF Tracks and simultaneously decode and synchronize CMAF Fragments to result in a multimedia presentation.

- CMAF specifies CMAF Selection Set encoding constraints for synchronized alternative content that can be selected based on device compatibility and user preference, e.g. alternative languages or codecs.

- CMAF specifies CMAF Switching Set encoding constraints for alternative encodings of the same content at different bitrates. CMAF Tracks in a CMAF Switching Set can be dynamically switched and seamlessly spliced by sequencing CMAF Fragments from different CMAF Tracks through a single decoder.

- CMAF Tracks in CMAF Aligned Switching Sets are alternative encodings of the same content with time-aligned CMAF Fragments, but the CMAF Tracks do not conform to the same CMAF Switching Set constraints, e.g. they may be encoded with different codecs, encryption, etc. Track Switching between CMAF Synchronized Switching Sets might be seamless on some Players, but not all Players.

- CMAF specifies CMAF Addressable Media Objects for storage and delivery and identification as Resources by external specifications, such as adaptive streaming protocols.  CMAF Addressable Media Objects are CMAF Headers, CMAF Track Files, CMAF Segments, and CMAF Chunks.

- CMAF specifies CMAF Media Profiles and file brands for widely used audio, video, and subtitle formats. Each CMAF Media Profile specifies constraints on codecs, samples, CMAF Fragments, and CMAF Tracks to identify encoder/decoder interoperability, and optional functionality including random access and seamless adaptive switching specified as CMAF Switching Set constraints. CMAF specifies general requirements and registration for CMAF Media Profiles and brands to enable other specifications to define CMAF Media Profiles that conform to the general CMAF requirements.

- CMAF specifies a CMAF Presentation Profile that conditionally requires CMAF Tracks and Media Profiles in conforming CMAF Presentations to enable most media playback devices to render a CMAF Presentation.  Additional CMAF Tracks and CMAF Media Profiles can be optionally included in a CMAF Presentation conforming to a Presentation Profile.  Other CMAF Presentation Profiles can be defined by other specifications that define different conditional requirements on CMAF Tracks and CMAF Media Profiles intended for specific applications.

Figure 1 CMAF data object model

# Operational Assumptions

TBD

# General mapping considerations

In the mapping considerations, it needs to be considered that first, CMAF conformance is not signalled. Secondly, there is not a required one-to one mapping of CMAF formats to DASH, and the same CMAF content can be mapped to DASH content in various ways depending on the application and delivery protocol.

There are also three levels of mapping that need to be considered:

- Media Profile related issues

- CMAF Header and Segment related issues

- Mapping of CMAF addressable resources to DASH Segments. Depending on the addressable media objects, different DASH profiles may be used.

## Basic Mapping

Table 1 provides a basic mapping for CMAF data objects to DASH data objects.

Table 1 – Mapping of CMAF objects to DASH object models

| CMAF Entity | DASH Entity |
| --- | --- |
| CMAF Presentation | One or multiple Periods in a Media Presentation |
| CMAF Selection Set | A group of Adaptation Sets with the same value of @group |
| CMAF Aligned Switching Set | Multiple Adaptation Sets with possible cross switching |
| CMAF Switching Set | One or more Adaptation Sets |
| Single initialization Switching Set | Multiple Adaptation Sets with corss switching @bitstreamSwitching =True* |
| CMAF Track File | On Demand profile segment* |
| CMAF Segment | Live profile segment* |
| CMAF Chunk | Broadcast TV profile segment* |
| CMAF Header | Initialization Segment* |
| CMAF Fragment | Live profile segment |

* With additional constraints

As is shown in Table 1:

- CMAF Presentation may be described by a DASH Media Presentation Description (MPD). CMAF only defines one set of synchronized CMAF Tracks as a Presentation. DASH can describe a CMAF Presentation as one DASH period or multiple DASH periods. A DASH MPD can describe multiple CMAF Presentations as a sequence of DASH periods. The following MPD parameters need to be set:

  - @profile: should be set to an appropriate value. See xxxxx

  - @minBufferTime: should be set to the nominal duration of a CMAF fragment.

- A CMAF Selection Set contains one or more synchronized Switching Sets of the same media type (audio, video, or subtitles), containing alternative content or encoding for the same Presentation over the same presentation time. Each Selection Set can be represented as a DASH Group. The @group attribute on an AdaptationSet element identifies adaptation sets that describe CMAF Switching Sets in a CMAF Selection Set identified by that @group value.

- A CMAF Switching Set can be represented by a DASH Adaptation Set in the MPD. CMAF Switching sets are more constrained than the DASH Adaptation Sets. Note that a Switching Set may be divided into multiple Adaptation Sets, e.g. HD and SD Adaptation Sets. In this mapping, the following adaptation set’s parameters should be set:

  - @mimeType: set to the media type and using mp4 and adding the brand of the media profile.

  - @codecs: set to the codecs parameter of the sample entry.

- A CMAF Single Initialized Switching Set can be represented by a DASH Adaptation Set with @bitstreamSwitching=True. The DASH player may switch between representations without downloading and using the initialization segment at the switching point for seamless switching. In addition, an AdaptationSet element can include a Descriptor whose schemeIdUri identifies the Media Profile specific encoding constraints and processing model for the CMAF Switching Set referenced. For instance, it can tell a player if it needs to process the CMAF Header only once, or on each Representation switch. More complicated Media Profiles may have more complicated processing rules that allow switching on predicted frames, using additional layers, etc.

- A CMAF Track can be represented as a DASH Representation with single segment. The DASH Extended On Demand profile uses such representation. In this case, the entire CMAF Track is addressed with single URL. A separate segment index file may also be provided for providing indexing information of the segment.

- A CMAF Segment can be represented as a DASH Live profile’s segment. Note that CMAF segment has additional constraints. Any of DASH addressing schemes can be used for these segments.

- A CMAF Low Latency Chunk can be represented as a DASH’s Broadcast TV profile segment format. The location of switching points can be signaled using DASH’s Switching element. Additional random access point can be inserted during encoding of some of CMAF Fragments and be signaled using RandomAccess element in MPD.

- A CMAF Header can be represented as a DASH Initialization Segment.

## On-demand and live content considerations

In addition, different mapping needs to be considered for the following cases:

- On-demand content: the CMAF content is available in complete i.e. the entire presentation was previously encoded. For such content, a static type MPD can be used. The @availabilityStartTime and @availabilityEndTime attributes may be used if the content is available and/or only offered during a wall clock time interval. All DASH’s segment addressing schemes can be used for CMAF Resource addressing. Other DASH features, such as multiple periods, multiple BaseURLs, various descriptors can be used with CMAF content.

- Live content: the CMAF content is generated on the fly, i.e. the content is encoded in real time. In this case, a dynamic type MPD is used for DASH delivery. The attributes such as @availabilityStartTime, @mediaPresentationDuration, @timeShiftBufferDepth and @minBufferTime are set according to availability of each segment at the media server. Other dynamic type MPD’s attributes, such as MPD minimum update period or MPD validity expiration inband events may be used for MPD updates. Note that none of above information are present in CMAF segments or tracks and the author of the content need to present them along with CMAF content for MPD authoring. Other DASH features, such as multiple periods, multiple BaseURLs, various descriptors, and remote elements can be used with CMAF content. In this case, two modes can be considered:

  - Segment mode: The encoder generates a full Segment

  - Chunk mode: The encoder generates a chunked mode

## MPD parameters considerations

Table 2 provides consideration for various MPD level parameters. Note that some of the values may be dependent to specific CMAF track parameters.

Table 2 MPD level parameters considerations

| Attribute or Element | Use | Proposed setting based on CMAF |
| --- | --- | --- |
| @profiles | O | If CMAF content is segmented, use live or extended live profile, depends also on the presence and absence of Inband Event Streams. If CMAF content is in form of Track files, use on-demand or extended on-demand. A multi period presentation with mix on-demand and live content, may use extended main profile. |
| @type | OD | To be set depending on the availability of the content. If the entire CMAF content available, type ‘static’ is recommended. Otherwise, it must be set to ‘dynamic’ |
| @availabilityStartTime | CM | To be set depending on the availability time of the content. |
| - @publishTime | - OD | No direct depenencies to CMAF. |
| @availabilityEndTime | O | To be set depending on the availability time of the content. |
| @mediaPresentationDuration | O | The CMAF Presentation duration if it is known |
| @minimumUpdatePeriod | O | For live content with dynamic MPD. |
| @minBufferTime | M | To be defined based on CMAF segment durations |
| @timeShiftBufferDepth | O | To be set based on content avaialibiity. |
| @suggestedPresentationDelay | O | To be set by the use case requirements |
| @maxSegmentDuration | O | Maximum duration of a CMAF segment |
| @maxSubsegmentDuration | O | n/a |
| ProgramInformation | 0…N | Content Property, |
| BaseURL | 0…N | outside of CMAF scope. |
| Location | 0…N | outside of CMAF scope. |
| Period | 1…N | define one per CMAF Presentation. |
| Metrics | 0 ... N | outside of CMAF scope. |
| EssentialProperty | 0 … N | outside of CMAF scope. |
| SupplementalProperty | 0 … N | outside of CMAF scope. |
| UTCTiming | 0 ... N | outside of CMAF scope. |

Table 3 Period level parameters considerations

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  |  | @start | O | To be set based on CMAF Presentation timing and according to [DASH1] requirements. |
|  |  | @duration | O | To be set based on CMAF Presentation timing and according to [DASH1] requirements. |
|  |  | @bitstreamSwitching | OD | Set to ‘true’ if all CMAF switching sets are Single Initialization in this period. |
|  |  | BaseURL | 0…N | Delivery property. To be defined independent of CMAF. |
|  |  | SegmentBase | 0...1 | Delivery property. To be defined independent of CMAF. |
|  |  | SegmentList | 0...1 | Delivery property. To be defined independent of CMAF. |
|  |  | SegmentTemplate | 0...1 | Delivery property. To be defined independent of CMAF. |
|  |  | - AssetIdentifier | - 0...1 | - Content property. Not defined by CMAF. |
|  |  | EventStream | - 0...N | - Out of CMAF scope |
|  |  | AdaptationSet | 0...N | Entire or part of a CMAF Switching Set. |
|  |  | Subset | 0...N | n/a |
|  |  | SupplementalProperty | 0...N | outside of CMAF scope. |
|  |  | EmptyAdaptationSet | 0...N | Delivery property. To be defined independent of CMAF. |
|  |  | - GroupLabel | - 0 … N | outside of CMAF scope. |
|  |  | - Preselection | - 0 … N | one instance may be present for each ‘prsl’ box present in group lists box in meta box on file level otherwise outside of CMAF scope. |

Table 4 Adaptation Set level parameters considerations

| Element or Attribute Name |  |  |  |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | @group |  | O | set to a unique number for each Selection Set. |
|  |  |  |  |  |  | @lang |  | O | according to the corresponding CMAF track’s language field and the ExtendedLanguageBox as defined in [ISOBMFF] 8.4.6. |
|  |  |  |  |  |  | @contentType |  | O | according to the content type of the corresponding CMAF track. |
|  |  |  |  |  |  | @par |  | O | to be set consistent to @width, @height and @sar. |
|  |  |  |  |  |  | @minBandwidth |  | O | the minimum @bandwidth value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @maxBandwidth |  | O | the maximum @bandwidth value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @minWidth |  | O | the minimum @width value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @maxWidth |  | O | the maximum @width value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @minHeight |  | O | the minimum @height value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @maxHeight |  | O | the maximum @height value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @minFrameRate |  | O | the minimum @framerate value in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @maxFrameRate |  | O | the maximum @framerate in all CMAF tracks in the corresponding Switching Set. |
|  |  |  |  |  |  | @segmentAlignment |  | OD | - set to ‘true’ o if a CMAF Switching Set is offered in the MPD in segment mode |
|  |  |  |  |  |  | @bitstreamSwitching |  | O | set to ‘true’ if the corresponding CMAF tracks have Single Initialization. |
|  |  |  |  |  |  | @subsegmentAlignment |  | OD | - set to ‘true’ if a CMAF Switching Set is offered in the MPD in track mode, |
|  |  |  |  |  |  | @subsegmentStartsWithSAP |  | OD | - defined only if that each Subsegment with SAP_type greater than 0 starts with a SAP of type less than or equal to the value of @subsegmentStartsWithSAP. |
|  |  |  |  |  |  |  | AudioChannelConfiguration | 0 … N | - may be set based on the ChannelLayoutBox of the corresponding CMAF Tracks. |
|  |  |  |  |  |  | Accessibility |  | 0 … N | - may be set based on the KindBox of the corresponding CMAF Tracks. |
|  |  |  |  |  |  | Role |  | 0 … N | - may be set based on the KindBox of the corresponding CMAF Tracks. |
|  |  |  |  |  |  | Rating |  | 0 … N | - out of CMAP scope. |
|  |  |  |  |  |  | Viewpoint |  | 0 … N | - out of CMAP scope. |
|  |  |  |  |  |  | ContentComponent |  | 0…N | - n/a |
|  |  |  |  |  |  | BaseURL |  | 0…N | out of CMAP scope. |
|  |  |  |  |  |  | SegmentBase |  | 0...1 | out of CMAP scope. |
|  |  |  |  |  |  | SegmentList |  | 0...1 | out of CMAP scope. |
|  |  |  |  |  |  | SegmentTemplate |  | 0...1 | out of CMAP scope. |
|  |  |  |  |  |  | Representation |  | 0 … N | specifies a Representation based on each CMAF track. |

Table 5 Representation level parameters considerations

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  |  | @bandwidth | M | to set based on the corresponding CMAF track average bandwidth and @minBufferTime. |
|  |  |  | @qualityRanking | O | out of CMAP scope. |
|  |  |  | @dependencyId | O | to set for enhanced scalable CMAF tracks. |
|  |  |  | @associationId | O | out of CMAP scope. |
|  |  |  | @associationType | O | out of CMAP scope. |
|  |  |  | @mediaStreamStructureId | O | n/a |
|  |  |  | CommonAttributesElements | - | Common Attributes and Elements (attributes and elements from base type RepresentationBaseType), for more details see Table 6. |
|  |  |  | BaseURL | 0…N | out of CMAP scope. |
|  |  |  | SubRepresentation | 0 … N | n/a |
|  |  |  | SegmentBase | 0...1 | out of CMAP scope. |
|  |  |  | SegmentList | 0 ... 1 | out of CMAP scope. |
|  |  |  | SegmentTemplate | 0 ... 1 | out of CMAP scope. |

Table 6 Common parameters considerations

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  |  | @profiles | O | may be set depending on CMAF tracks, segments or chunks configuration according to Section 11. |
|  |  |  | @width | O | may be set to width field value of the corresponding CMAF track’s SampleDescriptionBox. |
|  |  |  | @height | O | may be set to height field value of the corresponding CMAF track’s SampleDescriptionBox. |
|  |  |  | @sar | O | may be set to height field value of the corresponding CMAF track’s SampleDescriptionBox. |
|  |  |  | @frameRate | O | TBD |
|  |  |  | @audioSamplingRate | O | may be set to sampling_rate field value of the corresponding CMAF track’s SamplingRateBox. |
|  |  |  | @mimeType | M | To be set according to the media type. |
|  |  |  | @segmentProfiles | O | TBD |
|  |  |  | @codecs | O | may be set according to [DASH1]. |
|  |  |  | @maximumSAPPeriod | O | may be set according to [DASH1]. |
|  |  |  | @startWithSAP | O | to be set to 1 or 2 depending on the SAP type. |
|  |  |  | @maxPlayoutRate | O | may be set according to [DASH1]. |
|  |  |  | @codingDependency | O | may be set according to [DASH1]. |
|  |  |  | @scanType | O | may be set to "progressive". |
|  |  |  | @selectionPriority | OD default=1 | May be set, if there is a preference by the content author |
|  |  |  | @tag | O | may be set according to [DASH1]. |
|  |  |  | FramePacking | 0 … N | n/a |
|  |  |  | AudioChannelConfiguration | 0 … N | may be set according to [DASH1]. |
|  |  |  | ContentProtection | 0 … N | may be set according to [DASH1]. |
|  |  |  | EssentialProperty | 0 … N | may be set according to [DASH1]. |
|  |  |  | SupplementalProperty | 0 … N | may be set according to [DASH1]. |
|  |  |  | - InbandEventStream | - 0 … N | - may be set according to [DASH1] and the existence of inband events in CMAF tracks. |
|  |  |  | - Switching | - 0 … N | - TBD |
|  |  |  | - RandomAccess | - 0 … N | - TBD |
|  |  |  | - GroupLabel | - 0 … N | may be set according to [DASH1]. |
|  |  |  | - Label | - 0 … N | May be used, but outside the scope of CMAF to DASH mapping |

Table 7 Preselection level parameters considerations

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  |  | @preselectionComponents | M | If the CMAF content is comprised of one CMAF track, set this attribute to the value of the AdaptationSet@id of the comprising CMAF track. If the Preselection requires multiple CMAF tracks, add the AdaptationSet@id of all contributing CMAF tracks as whitespace-separated list to this attribute. In this case, the first element in this list shall refer to the id of that AdaptationSet containing all initialization data. Beforementioned requirements are fulfilled if the order of all AdaptationSet@id in the @preselectionComponents attribute match exactly the order of the corresponding entity_ids in the PreselectionGroupBox. |
|  |  |  | @lang | O | according to the corresponding ExtendedLanguageBox in the PreselectionGroupBox, if present. |
|  |  |  | @order | OD | shall be set based on the information in the interleaving_tag of the PreselectionGroupBox. may be absent for Preselections comprised of only one CMAF track or when the interleaving_tag in the PreselectionGroupBox is absent. |
|  |  |  | @tag | O | shall be set to the value of the preselection_tag field value of the corresponding main CMAF track’s PreseletionGroupBox, if present Otherwise: do not set |
|  |  |  | @selectionPriority | O | may be set to the preselection_priority element |
|  |  |  | CommonAttributesElements | - | Common Attributes and Elements (attributes and elements from base type RepresentationBaseType), for more details see Table 6. |
|  |  |  | Accessibility | 0…N | out of CMAF scope. |
|  |  |  | Role | 0 … N | may be set based on the KindBox of the corresponding PreselectionGroupBox. |
|  |  |  | Rating | 0...1 | out of CMAF scope. |
|  |  |  | Viewpoint | 0 ... 1 | out of CMAF scope. |

## Preselection Considerations

If a CMAF Header contains PreselectionGroupBoxes, each instance thereof is assigned to one Preselection element with the following guidelines to be considered:

- The value of the group_id may be assigned to the Preselection@id attribute. (optional)

- The tracks referenced through the entity_id in the PreselectionGroupBox shall be assigned to the Preselection@preselctionComponents attribute with each value of an entity_id being represented by the corresponding AdaptationSet@id value. The order of the entries in the @preselectionComponents attribute shall be exactly the same as in the loop of entity_ids.

- The value of the preselection_tag, if present, shall be assigned to the Preselection@tag attribute. Otherwise, this attribute may be absent.

- The value of the preselection_priority, if present, shall be assigned to the Preselection@selectionPriority attribute. Otherwise, this attribute may be absent.

- The value of the interleaving_tag, if present, shall be used to determine the Preselection@order attribute. If the interleaving_tag, the @order shall be either set to “undefined” or be absent.

- If additional boxes are present in a PreselectionGroupBox, the following guidelines should be followed:

  - An ExtendedLanguage box is assigned to the Preselection@lang attribute

  - KindBoxes are converted to Role and Accessibility descriptors or other attributes and elements as specified in clause 8.5.

  - Any ChannelLayoutBox or AudioRenderingIndicationBox is converted to an AudioChannelConfiguration descriptor.

- All DASH AdaptationSets comprising any Preselection should get a SupplementalProperty descriptor assigned. The @schemeIdUri of this descriptor shall be “urn:mpeg:dash:preselection:2016”, the value may be absent. If the AdaptationSet is never referenced as the main AdaptationSet (i.e. is never the first entry in the list of AdaptationSet@id values in the Preselection@preselectionComponents list), an EssentialProperty descriptor shall be instead.

Possible ToDos:

Use cases for Preselection elements:

- NGA

- Layered Coding: Preselection@codecs

- Supplementary video services and the supplementary video descriptor ([DASH], clause 5.8.5.16)

## DASH Packager Guidance using ISOBMFF KindBox

### General

This clause defines schema for use with the ISOBMFF KindBox for signalling DASH child elements and attributes. Such information provides guidance to DASH packagers when generating the DASH manifest based on ISOBMFF source files.

The presence of a KindBox with one of the identifiers defined by this clause in the schemeURI field shall indicate to a DASH packager to add respective information into the DASH manifest as defined for that scheme.

When information is targeting Representation elements, the KindBox shall be present in the MediaBox of the respective track, for AdaptationSet elements in the SwitchingGroupBox, for Preselections in the PreselectionGroupBox and for Periods in the MovieBox.

Since a KindBox with its schemeURI set to one of the identifiers defined by this clause is intended to guide DASH packagers, players are expected to ignore such a box. Furthermore, such a KindBox should get removed when a creating DASH segments. KindBoxes containing other, possibly unknown settings in their schemeUri field shall not get removed from segment unless specified differently for that scheme.

NOTE: Other applications, such as storage formats according to 23009-9 (DASH REaP) define different schema used with the schemeURI field for similar purposes.

If the ISOBMFF specification or any other standard defines dedicated means for signaling properties other than utilizing the KindBox as documented in this subclause, content creators should prefer such signaling over the use of the KindBox. An example therefore is the ExtendedLanguageBox to indicate the media’s language. The content of this box can get translated into the @lang attribute.

On the other hand, when both types of signaling are present in an incoming ISOBMFF file, a DASH packager should prioritize the KindBox with a scheme from this clause since this indicates that the content provider wants the DASH manifest to intentionally deviate from the signaling based on those elements.

### DASH Descriptors and other child elements

#### Definition

An ISOBMFF KindBox may get used to indicate to the DASH packager to add additional XML child elements to the parent element. Such child elements may possible be, but are not limited to elements derived from the DescriptorType class as defined in clause 5.8 of this document.

If used for this purpose, the KindBox shall carry the following information:

•	The schemeURI field shall be set to “urn:mpeg:dash:descriptor:2025” (without the DQUOTE characters).

•	The value field carries the XML snippet of the intended child element.

The syntax for the value field shall be formatted according to the subsequent ABNF notation according to IEFT RFC 5234 for the KINDBOX-XMLCHILD-VALUE:

KINDBOX-XMLCHILD-VALUE = XML-SNIPPED XML-SNIPPED = *VCHAR ; XML snippet as defined in this document

NOTE: This ABNF definition does not contain the mandatory null byte termination required by the utf8string field.

#### Examples

The following XML snippets show use cases where the DescriptorType definition got amended compared to the base definition.

- Descriptor extended by child element:

<SupplementalProperty schemeIdUri="urn:mpeg:dash:supv:2022"> <SupVideoInfo processingInfo="1 3 5"/> </SupplementalProperty>

- Descriptor extended by additional attributes:

<SupplementalProperty schemeIdUri="urn:dvb:dash:fontdownload:2014" value="1" dvb:url="https://example.com/fonts/SubtitleDisplay.woff" dvb:fontFamily="SubtitleDisplay" dvb:mimeType="application/font-woff"/>

Source: https://dashif.org/dash.js/pages/advanced/subtitles-and-captions/dvb-font-downloading.html

### DASH Attributes

#### Definition

An ISOBMFF KindBox may get used to indicate to the DASH packager to add additional attributes to the parent element.

If used for this purpose, the KindBox shall carry the following information:

•	The schemeURI field shall be set to “urn:mpeg:dash:attribute:2025” (without the DQUOTE characters).

•	The content of the value field in the KindBox provides a whitespace separated list of attribute names, optionally including their assigned values.

The syntax for the value field shall be formatted according to the subsequent ABNF notation according to IEFT RFC 5234 for the KINDBOX-ATT-VALUE:

KINDBOX-ATT-VALUE = nameValuePair *( SP nameValuePair) nameValuePair = AttName [ "=" DQUOTE ValueString DQUOTE] AttName = 1*VCHAR ; name of XML attribute ValueString = 1*VCHAR ; as defined for the ; respective attribute

NOTE: This ABNF definition does not contain the mandatory null byte termination required by the utf8string field.

#### Example

The following example shows how a selectionPriority attribute, intended to get assigned to a Preselection element, gets represented in ISOBMFF:

Since the ‘prsl’ grouping_type relates to the DASH Preselection element, this KindBox is expected to yield the following XML snippet:

<Preselection selectionPriority="5"/>

[Ed.Note] Related input contribution m69412 proposed this text as Annex to 23009-1. Additionally, an update to table 2, clause 4.7 was suggested as this table as shown below acts as a registry for the “urn:mpeg:dash:…” namespace. We need to consider whether we can reuse this namepace here as well.

Update Table 2 in clause 4.7 “Schemes” with the following lines:

| Scheme Identifier | Clause in this document | Informative description |
| --- | --- | --- |
| urn:mpeg:dash:attribute:2025 | N.2.3 | DASH packager signaling for attributes |
| urn:mpeg:dash:descriptor:2025 | N.2.2 | DASH packager signaling for child elements |

## Segment addressing schemes

To each CMAF addressable object a resource identifier is assigned. The resource identifier should be added such that templating can be used.

  - CMAF Header is assigned to the Initialization Segment

  - CMAF media segments are assigned to DASH Media Segments in the order of decoding using the appropriate numbering scheme.

# Mapping Media Profiles to DASH Adaptation Sets and Segments

## General Considerations

The mapping to DASH Segment formats SHOULD be done by using one of the CMAF Resources

- CMAF Single Fragment Segment Mode is mapped to either the ISO Base media file format live profile as defined in clause 8.4 of [Error: Reference source not found1] or the ISO Base media file format extended live profile as defined in clause 8.9 of [Error: Reference source not found1].

- CMAF Multiple Fragment Segment Mode is mapped to either the ISO Base media file format live profile as defined in clause 8.4 of [Error: Reference source not found1] or the ISO Base media file format extended live profile as defined in clause 8.9 of [Error: Reference source not found1].

- CMAF Chunk Mode is mapped to Broadcast TV profile as defined in clause 8.11 of [Error: Reference source not found1].

- CMAF Track File Mode is mapped to either the ISO Base media file format On Demand profile as defined in clause 8.3 of [Error: Reference source not found1] or the Extended ISO Base media file format On Demand profile as defined in clause 8.8 of [Error: Reference source not found1].

## Mapping of a CMAF Presentation Profile

If the CMAF Presentation Author wants to map a CMAF Presentation to a DASH Media using the technologies for a CMAF Presentation Profile may be offered using a dedicated profile. The profile is signaled as “urn:mpeg:dash:<dash-profile>:cmaf:<presentation-profile>” with <dash-profile> to be set according to Table Y and the presentation profile set to the 4cc as defined for the presentation profile.

- The Media Presentation conforms to the dash-profile as indicated in Table 8.

- If containing video, the MPD SHOULD contain at least one Adaptation Set following the DASH recommendations as defined in 8.1 and the Adaptation Set conforms to the DASH profile as indicated by dash-profile.

- If containing audio, the MPD SHOULD contain at least one Adaptation Set following the DASH recommendations as defined in 8.1 and the Adaptation Set conforms to the DASH profile as indicated by dash-profile.

- If containing subtitle, the MPD SHOULD contain at least one Adaptation Set following the DASH recommendations as defined in 8.1 and the Adaptation Set conforms to the DASH profile as indicated by dash-profile.

Table 8 Template for conformance signalling of DASH Profiles

| <dash-profile> | DASH Profile conformance |
| --- | --- |
| OnDemand | the ISO Base media file format On Demand profile as defined in clause 8.3 of [DASH1] |
| Live | ISO Base media file format live profile as defined in clause 8.4 of [DASH1] |
| Extended-OnDemand | Extended ISO Base media file format On Demand profile as defined in clause 8.8 of [DASH1] |
| Extended-Live | ISO Base media file format extended live profile as defined in clause 8.9 of [DASH1] |
| Broadcast | Broadcast TV profile as defined in clause 8.11 of [DASH1] |

### Sample entry

## Video

### AVC

### HEVC

### SHVC

## Audio

### HE-AAC single bitrate

### HE-AAC multi bitrate

## Subtitles

# Mapping of Encrypted Tracks

# Mapping of CMAF Segment Mode

### Basic Mapping

### Switching and Random Access Signalling

### Low-Latency Mode

### Events and other delivery boxes

### Compatibility Considerations

# Mapping of CMAF Track File Mode

### Basic Mapping

### Segment Index for CMAF Fragment addressing

### Events and other delivery boxes

### Compatibility Considerations

# Mapping of CMAF Chunk Mode

### Basic Mapping

### Switching and Random Access Signalling

### Segment Index for CMAF Fragment addressing

### Events and other delivery boxes

### Compatibility Considerations

# Recommendation for External Interoperability Points

[Ed.Note] Adapt this name according to any potential future changes of 14496-12, 8th edition, Amd.1, clause 8.15.4.2.