## INTERNATIONAL ORGANIZATION FOR STANDARDIZATION ORGANISATION INTERNATIONALE DE NORMALISATION ISO/IEC JTC 1/SC 29/WG 5 MPEG JOINT VIDEO EXPERTS TEAM WITH ITU-T SG21

### ISO/IEC JTC 1 / SC 29 / WG 5 N 367

### Daejeon, KR, 26 June – 4 July 2025

Source:; Convenor (Jens-Rainer Ohm). Title:; Joint Call for Evidence on video compression with capability beyond VVC. Type:; General. Subtype:; Other. Status:; Approved. Date:; 2025-07-04. Expected Action:; Info. Action due date:; N/A. Pages:; 15 (not including this cover page). Email of convenor:; ohm @ ient . rwth-aachen . de. Committee URL:; https://sd.iso.org/documents/ui/#!/browse/iso/iso-iec-jtc-1/iso-iec-jtc-1-sc-29/iso-iec-jtc-1-sc-29-wg-5.

| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 39th Meeting, Daejeon, KR, 26 June – 4 July 2025 | Document: JVET-AM2026-v1 |
| --- | --- |

| Title: | Joint Call for Evidence on video compression with capability beyond VVC |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Text of CfE |  |  |
| Authors: | Jens-Rainer Ohm Mathias Wien Frank Bossen | Email: | ohm@ient.rwth-aachen.de Mathias.Wien@lfb.rwth.aachen.de frank@bossentech.com |
| Source: | Editors |  |  |

_____________________________

# Introduction

The JVET has been tasked by its ITU-T and ISO/IEC parent bodies to study the potential for standardization of video coding technology with capability exceeding that of VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3) in terms of compression, encoder/decoder implementability, applicability to a variety of content, and other features such as transmission latency/robustness, scalability, etc. Technology of interest could take the form of extending an existing standard, or might require definition of an entirely new standard.

Example sources include camera-captured content, computer-generated content, user-generated content, and high dynamic range content, while example applications include broadcast (both live or pre-authored content), real-time video conferencing, video chat, on-demand viewing, gaming, video upload, storage-based media replay, and surveillance with fixed or moving cameras [2].

This Call for Evidence (CfE) is being issued as part of this study. The CfE requests information regarding video compression technology that has compression performance or additional functionality beyond that of VVC, where the trade-off in terms of implementation cost is also an important criterion.

As it is recognized that in a growing number of applications practically fast encoding is required, information on this aspect is also separately requested in the context of the CfE. As it cannot be expected that encoding with lower run time comes without a penalty in compression, this will be tested separately.

Responses to the CfE will be evaluated at the 40th JVET meeting during 3-12 October 2025, as further described below.

Depending on the result of the evaluation, a Call for Proposals (CfP) may be issued in preparation for starting a formal standardization project.

Companies and organizations who have developed compression technology that they believe to be superior to the Main 10 Profile of the VVC standard regarding the aforementioned aspects are kindly invited to bring such information to the JVET in response to this CfE. Additionally, contributions are also welcome regarding technology that better supports newly emerging application areas of video coding by additional functionality (see section 5 of this document).

The basic test case for providing evidence on improved compression capability is described in section 2. It is encouraged but not required to provide a detailed description of a technology submitted for testing in the context of the CfE. To perform a rough assessment of implementation complexity, encoding and decoding run times shall be reported relative to VTM anchors. It is preferred to receive submissions via bitstreams and decoder executables (preferably runnable under Ubuntu 24.04) which will be used only to generate the reconstructed video sequences in the subjective viewing described in section 4.

For the additional test case of providing evidence about fast encoding capability described in section 3, it is encouraged but not required to submit an encoder executable which runs in a single-thread processing environment for demonstration purposes. The information about encoding and decoding run time relative to the VTM anchors, measured using the same simulation environment, must be reported. Information about multi-threading capability and runtime reduction achievable by multi-thread processing is encouraged to be reported with necessary explanation (such as number of threads, description of parallelized modules).

Beyond objective quality criteria such as PSNR and SSIM, the performance of submissions will be assessed based on expert viewing tests conducted during the October 2025 meeting. Rate points to be tested have been determined on a per-sequence basis to cover a wide range of visual qualities. For this purpose, encodings using VTM reference software [3] as well as the ECM software [4] developed by JVET within an exploration effort have been used. Rate-matched encoded bitstreams of both VTM and ECM and variants with runtime-constrained encoding will also be included in the CfE tests, obeying the same coding conditions as the submissions.

Results of the CfE will be summarized in a report output from the 40th JVET meeting. Result data of a submission will not be included in the report if the submitter expresses such a preference to the JVET chair. For submissions that are included, results will be anonymized on a best effort basis.

## Timeline

- Call for Evidence issued: 2025-07-04

- VTM anchors, additional runtime-constrained VTM encodings, and VTM encoder configuration information available: 2025-07-31

- ECM encodings and ECM encoder configuration information available: 2025-08-31

- Registration of a response (see Section 6): 2025-09-05

- Materials for preparation of visual testing available (bitstreams and decoder): 2025-09-19

- Submission of contributions (descriptive document): 2025-09-26

- Evaluation of responses: 3–12 October 2025 at the 40th JVET meeting (expected to be attended by submitters)

- Depending on the outcome of the Call for Evidence, a Draft Call for Proposals may be issued by the end of the October 2025 meeting.

## VVC anchors and ECM encoding cases

VVC anchors are generated using the VTM 23.11 software package. ECM encodings are generated using the ECM 16.1 software package. A static quantization parameter (QP) setting is applied, where a one-time change of the quantization parameter from value QP to value QP+1 is used in order to meet the defined target bit rates. The quantization parameter settings can be found in the encoder configuration information.

# Test cases for evidence on improved compression

The following categories are considered as test cases. Each category name includes a characterization of the content type, the suggested encoder configuration and an indication of the resolution.

- SDR RA UHD/4K: Representing the use case of distribution of standard dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

- SDR RA HD: Representing the use case of distribution of standard dynamic range HD video content e.g. in a streaming scenario, using a random-access configuration.

- SDR LB HD: Representing the use case of conversational and other low delay applications at HD resolution, correspondingly using a low-delay configuration.

- HDR RA 4K: Representing the use case of distribution of high dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

- HDR RA Cropped 8K: Representing the use case of distribution of high dynamic range 8K video content e.g. in a streaming scenario, using a random-access configuration. In order to reduce the encoding workload for assessment of this category and allow investigation on 4K displays, cropped regions of 3840×2160 resolution are used.

- Gaming LB HD: Representing the use case of online gaming with a low-delay configuration.

- UGC RA: Representing the use case of user generated content at 1080×1920 or 1920x1080 resolution using a random-access configuration.

Submitters are encouraged (but not required) to submit results for all test cases. However, submitters are required to provide results for all sequences in a given test case.

## General coding conditions

The following rules apply:

- Encoded bitstreams shall not exceed the target bit rates defined for each of the test cases.

- Sequences are expected to be encoded at full input resolution for all rate points. If coding at reduced resolution is part of the algorithmic concept, it shall be described.

- Quantization settings should be kept static. When a change of quantization is used it shall be described.

- Notwithstanding the previous rule, a one-time change of the quantization settings to meet the target bit rate is allowed and must be documented.

- Optimizing encoding parameters using non-automatic means as well as optimizing encoding parameters on a per sequence basis within a category is discouraged.  When any such optimization is employed, it shall be described.

- No part of test sequences shall be used as a training set or part of a training set for training entropy coding tables, VQ codebooks, transforms, predictors, filters, neural network models, etc.

- Use of preprocessing and/or postprocessing shall be described.

- Use of perceptual optimization shall be described.

- Use of multi-pass encoding shall be described.

## SDR RA UHD/4K

### Sequence formats and target bit rates

All sequences have a picture size of 3840×2160, chroma format 4:2:0 YCbCr according to ITU-R BT.2020, and bit depth 10. Further information can be found in Table 1.

Table 1: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRU1 | CrowdRun | 10 | 500 | 50 | 17282413c9a523a02e0c8463c097221f |
| SRU2 | DrivingPOV3 | 10 | 600 | 60 | e81b65724c4235128b2749ccb3b0fb4a |
| SRU3 | FireDance | 10 | 250 | 25 | bfcfc333924835aaf4ed89efa8428a36 |
| SRU4 | HallwayScene | 10 | 250 | 25 | b0c8718579998060e375fcdb77243013 |

Target bit rates for the encoding of the sequences are listed in Table 2.

Table 2: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| SRU1 | CrowdRun | 700 | 1500 | 3200 | 7000 |
| SRU2 | DrivingPOV3 | 300 | 600 | 1200 | 2400 |
| SRU3 | FireDance | 300 | 700 | 1300 | 2500 |
| SRU4 | HallwayScene | 100 | 250 | 500 | 1000 |

### Coding conditions for VVC anchors

The general description in section 2.1 applies.

Specifically, in this test case, a random-access scenario (RA) is used for evaluation and follows the JVET common test conditions and software reference configurations [5]. The intra refresh period is dependent on the frame rate of the source: a value 32 shall be used for sequences with a frame rate equal to 24fps, 25fps or 30fps, and a value 64 for 50fps and 60fps.

### Coding conditions for submissions

Submissions to the Call for Evidence shall obey the following rules:

- The general rules as described in section 2.1.

- Allow for random access at intervals not larger than the intra refresh period of the respective anchor.

## SDR RA HD

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080, chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 3.

Table 3: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRH1 | DucksTakeOff | 8 | 500 | 50 | 3504582275e36e310fae72ffa6a0f8aa |
| SRH2 | DrivingPOV4 | 10 | 600 | 60 | 4724c33b6b27669a406587bdb8d43dc8 |
| SRH3 | Seeking | 8 | 500 | 50 | 0f26ad79d085895299860530e145a8c9 |
| SRH4 | Umbrella | 8 | 500 | 50 | 87572379ef39bedf55f9c9d8152086e7 |

Target bit rates for the encoding of the sequences are listed in Table 4.

Table 4: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| SRH1 | DucksTakeOff | 300 | 900 | 2400 | 4000 |
| SRH2 | DrivingPOV4 | 100 | 250 | 550 | 1400 |
| SRH3 | Seeking | 200 | 400 | 800 | 1600 |
| SRH4 | Umbrella | 300 | 600 | 1400 | 3500 |

### Coding conditions for VVC anchors

The description in Section 2.2.2 applies.

### Coding conditions for submissions

The description in Section 2.2.3 applies.

## SDR LB HD

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 5.

Table 5: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SLH1 | Beatriz L | 8 | 500 | 50 | fe74cd5046fa033b4f743f42b29e69cd |
| SLH2 | GregoryCactus2 P | 10 | 300 | 30 | 4fb703c30a3fb06bacef461c69908b93 |
| SLH3 | GregoryScarf2 P | 10 | 300 | 30 | 7608f5bf81b4f3f11d0c7a489a5ff26c |
| SLH4 | OfficeWalkAtWall L | 8 | 300 | 30 | 529c15491ea8e1eb0320244a6ff902bb |

Target bit rates for the encoding of the sequences are listed in Table 6.

Table 6: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| SLH1 | Beatriz | 50 | 100 | 200 | 400 |
| SLH2 | GregoryCactus2 | 200 | 600 | 1500 | 4000 |
| SLH3 | GregoryScarf2 | 200 | 600 | 1800 | 5000 |
| SLH4 | OfficeWalkAtWall | 50 | 150 | 350 | 900 |

### Coding conditions for VVC anchors

The general description in section 2.1 applies.

Specifically, in this test case, a low-delay scenario with B pictures (LB) is used for evaluation and follows the JVET common test conditions and software reference configurations [5]. No picture reordering is applied between decoder processing and output.

### Coding conditions for submissions

Submissions to the Call for Evidence shall obey the following rules:

- The general rules as described in section 2.1.

- Overall structural delay shall not be larger than that of the respective anchor.

## HDR RA 4K

### Sequence formats and frame rates

All sequences have a picture size of 3840×2160, chroma format 4:2:0 YCbCr, bit depth 10bit, according to ITU-R BT.2100, with the applicable transfer function indicated. Further information can be found in Table 7.

Table 7: Test sequences

| Sequence ID (SID) | Sequence name | Transfer fct. | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| HRU1 | AMS06 | HLG10 | 600 | 60 | d1f11c771febbb8a2bbb7faadc13cbbb |
| HRU2 | MeridianHDR2 | P3 PQ 4000nits | 600 | 60 | 9e889c7d78b1b5a0fdebb88871a362dd |
| HRU3 | NeptuneFountain3R1 | HDR10 PQ | 600 | 60 | 4d7aa4113ddbf6de8a3a9856168f6d5c |
| HRU4 | SparksWelding | HDR10 PQ 1000nits | 600 | 60 | 7e0c6bd867c370dde9b11c76ed93409f |

Target bit rates for the encoding of the sequences are listed in Table 8.

Table 8: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| HRU1 | AMS06 | 600 | 1300 | 3500 | 8000 |
| HRU2 | MeridianHDR2 | 150 | 300 | 600 | 1200 |
| HRU3 | NeptuneFountain3R1 | 350 | 850 | 2000 | 5000 |
| HRU4 | SparksWelding | 400 | 1000 | 2600 | 7000 |

### Coding conditions for VVC anchors

In this test case, the random-access scenario is used for evaluation. The description in Section 2.2.2 applies, with the common testing conditions for HDR/WCG content being used [6].

### Coding conditions for submissions

The description in Section 2.2.3 applies.

## HDR RA Cropped 8K

### Sequence formats and frame rates

All sequences have been cropped from 8K resolution to a picture size of 3840×2160, chroma format 4:2:0 YCbCr, bit depth 10bit, according to ITU-R BT.2100, with the applicable transfer function indicated. Further information can be found in Table 9.

Table 9: Test sequences

| SID | Sequence name | Transfer fct. | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| HRC1 | ChandelierCropBR | HDR10 PQ | 360 | 60 | bb42a5d627330c652f19aca4b184b47f |
| HRC2 | FashionLadyCrop1 | HDR10 PQ | 380 | 60 | d8546edaa260468f027c0e1c28d422b0 |
| HRC3 | WaterfallForest | HLG10 | 500 | 50 | 54abb97e78a7255885fc283d02f9c964 |
| HRC4 | WomenFootball | HLG10 | 500 | 50 | 94bb484ab23ac900d1f417c52972535f |

Target bit rates for the encoding of the sequences are listed in Table 10.

Table 10: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| HRC1 | ChandelierCropBR | 300 | 650 | 1300 | 2800 |
| HRC2 | FashionLadyCrop1 | 200 | 500 | 1000 | 2000 |
| HRC3 | WaterfallForest | 1000 | 2500 | 6000 | 14000 |
| HRC4 | WomenFootball | 300 | 600 | 1100 | 2000 |

### Coding conditions for VVC anchors

The description in Section 2.5.2 applies.

### Coding conditions for submissions

The description in Section 2.5.3 applies.

## Gaming LB HD

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080, chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 11.

Table 11: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| GLH1 | DOTA2s360 | 8 | 550 | 60 | 999ae67b022892a7b70ab4dcc70f1043 |
| GLH2 | GTAVs090 | 8 | 600 | 60 | 43643d686a91a51758c63346fab7eca3 |
| GLH3 | Level1 | 10 | 600 | 60 | 87c7055771366d16b0d69cb56f6c66fb |
| GLH4 | Minecraft | 8 | 600 | 60 | 4f1055d558284e789fcae7686f3419bc |

Target bit rates for the encoding of the sequences are listed in Table 12.

Table 12: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| GLH1 | DOTA2s360 | 180 | 300 | 550 | 1000 |
| GLH2 | GTAVs090 | 400 | 900 | 2000 | 3600 |
| GLH3 | Level1 | 400 | 1000 | 2000 | 4000 |
| GLH4 | Minecraft | 300 | 600 | 1200 | 2400 |

### Coding conditions for VVC

The general description in section 2.1 applies.

Specifically, in this test case, a low-delay scenario with B pictures (LB) is used for evaluation and follows the JVET common test conditions and software reference configurations [5]. No picture reordering is applied between decoder processing and output.

### Coding conditions for submissions

The description in Section 2.4.3 applies.

## UGC RA

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 13.

Table 13: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| URH1 | Camellia P | 8 | 600 | 60 | 6e5413666e42d3c24db9b81b5db641ff |
| URH2 | Hobby-w5xz-backpack P | 8 | 240 | 24 | b8ee8e98b5d34afcfc24a9d02d8f3a43 |
| URH3 | NightLandscapeRunning L | 8 | 300 | 30 | 8eed52f009ed4ca2d31abe9e61b30432 |
| URH4 | Sports-76a2-iceball L | 8 | 600 | 60 | 3030f3760f34ce97e67d7e1a02eedd3b |
| URH5 | VerticalVideo-3709-snow P | 8 | 300 | 30 | 3db32dc2c295dc25622ebc516420c63b |
| URH6 | VerticalVideo-3d96-walk P | 8 | 300 | 30 | 5eb532977244f8be48591bf06592579a |

Target bit rates for the encoding of the sequences are listed in Table 14.

Table 14: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| URH1 | Camellia | 250 | 500 | 1000 | 2000 |
| URH2 | Hobby-w5xz-backpack | 90 | 160 | 280 | 500 |
| URH3 | NightLandscapeRunning | 100 | 250 | 600 | 1200 |
| URH4 | Sports-76a2-iceball | 80 | 160 | 250 | 400 |
| URH5 | VerticalVideo-3709-snow | 80 | 160 | 300 | 500 |
| URH6 | VerticalVideo-3d96-walk | 80 | 160 | 300 | 600 |

### Coding conditions for VVC anchors

The description in Section 2.2.2 applies.

### Coding conditions for submissions

The description in Section 2.2.3 applies.

# Test cases for evidence on improved compression with runtime-constrained encoding

The same categories as listed in section 2 are considered as test cases.

Submitters are strongly encouraged (but not required) to submit results for all test cases. However, submitters are required to provide results for all sequences in a given test case.

It is to be noted that, due to the possible large amount of complexity/rate combinations, it is likely that not all points can be included in viewing tests. This can only be determined after the number of submissions is known. Complexity/rate combinations for which no visual test is performed will be assessed using objective metrics.

## Sequence formats and frame rates

For each category, sequence formats and frame rates are identical to those defined in Section 2.

## Coding conditions for anchors

For each category, the coding conditions for the anchor are identical to those defined in the corresponding part of section 2 except as noted below.

For VVC, bitstreams are generated for the following configurations documented in [5]:

- Default (equivalent to the one used in Section 2)

- High performance (encoding times are approximately 2 times those of the default)

- Reduced encoding time (3 variants, encoding times are approximately 0.2 to 0.75 times those of the default)

VTM with default configurations is the anchor with respect to which relative compression performance and run times are calculated.

## Coding conditions for submissions

Submissions to the Call for Evidence shall obey the following rules:

- For each category, the rules defined in the corresponding part of section 2 apply.

- Encoder run times must be in the range of targets defined below, and should match the runtime of targets as closely as possible.

- No target is defined for decoder run times. However, decoder run times are to be reported and may be considered in the evaluation of this Call.

- Code optimization beyond what is typically done for reference software such as VTM is discouraged. The level of code optimization shall be described.

Three encoder run time targets are defined as:

- 5 times (500%) the encoder run time of the VTM anchor

- Same (100%) as the encoder run time of the VTM anchor

- 0.2 times (20%) the encoder run time of the VTM anchor

For each bitstream, run time is defined to be the total processing time to generate the bitstream and includes processing time for any pre-processing operations. Where multithreading is used, the run time is the sum of the run times of each thread. Design parts affected by multithreading must be described.

The run time target is defined for the aggregate run time across all bitstreams. It is not required to meet to the target run time for each bitstream. It is discouraged to tune encoder configurations for each bitstream such as to meet the target run time for every bitstream.

For the purpose of matching run time targets, the aggregate run time across multiple sequences and rate points shall be computed as follows: 1) run times for all rate points are summed for each sequence; 2) the geometric mean is computed across all sequences; 3) the ratio to anchor runtime is computed. Alternate ways to aggregate run times may be additionally used when evaluating responses. For example, run times may be aggregated across a single rate point (e.g., the highest rate point).

Matching the run time targets exactly is not necessary. Run times should be sufficiently close to the target to be able to plot an encoder run time vs compression performance curve for each submission and have this curve cover a range similar to the one spanned by the three run time targets defined above.

It is strongly encouraged to include a set of bitstreams and results for each of the three run time targets in a response. Additional run time vs compression trade-offs may be included in a response such as to have a more densely sampled curve. For example, results for run times approximately 0.5 times or 2 times those of the VTM anchor may be included.

# Evaluation Methodology

Evaluation of the submissions in response to the Call for Evidence will be performed at the October 2025 JVET meeting in Geneva, CH.

Respondents are asked to make submissions preferably by including bitstreams and decoders, and, as much as possible, documentation of the compression technology. Submissions must provide bitstreams for all sequences for all test cases submitted for evaluation of the response, and the decoder must be capable of decoding the bitstreams and storing the decoded data in 10-bit YUV 4:2:0 uncompressed format as needed for the subjective evaluation. For evaluation by objective metrics, CSV files containing the values listed in annex B shall be provided.

The subjective assessment of the received submissions will be done by an expert panel either before or during the October meeting. The ACR-HR method will be used, as described in Recommendation ITU-T P.910. A panel of at least 15 experts will be selected among the available volunteers to participate in the evaluations.

In order to complete the evaluation by the end of the October meeting, it may be necessary to select a subset of representative video clips for the visual testing. This will be determined by the JVET experts to properly represent as many compression use cases as possible. Evaluation based on objective metrics will be done for all submitted test cases.

# Evidence on other functionality

Companies and organizations who have developed compression technology that provides additional functionality with a better compression trade-off than existing standards are also invited to submit such information in the context of this Call for Evidence. Proposals on how the benefit of such functionality could be assessed in the context of a Call for Proposals or during the development of a video compression standard would also be welcome. For examples of such additional functionality, you may refer to [2].

# Logistics

Prospective contributors of responses to the Call for Evidence should contact the following persons:

Jens-Rainer Ohm (JVET chair)

RWTH Aachen University, Institute of Communications Engineering

Melatener Str. 23, 52074 Aachen, Germany

Tel. +49-241-8027671, email ohm@ient.rwth-aachen.de

Mathias Wien (JVET test coordinator)

RWTH Aachen University, Institute of Imaging and Computer Vision

Kopernikusstr. 16, 52074 Aachen, Germany

Tel. +49-241-8027867, email wien@lfb.rwth-aachen.de

Registration of a response shall be made by submitting the registration form in Annex C to the persons above on or before September 5, 2025. However, interested parties are kindly invited to express their intent as early as possible.

Details on how to format and submit documents, bitstreams, and other required data will be communicated directly to those who register a response. Additionally, the JVET chair will provide assistance to submitters from outside JVET in order for them to attend the JVET meeting.

Test sequences, anchors, and configuration information will also be made available by contacting the persons above (see section 1.1 for availability dates).

# References

- ITU-T, Working practices using objective metrics for evaluation of video coding efficiency experiments, Doc. HSTP-VID-WPOM, 2020.

- “Draft of use cases and requirements for potential next-generation video coding standard beyond VVC capability”, ISO/IEC SC 29/WG 2 document N 448 and ITU-T Q6/21 document VCEG-BX25, April 2025

- “Algorithm description for Versatile Video Coding and Test Model 22 (VTM 22)”, Joint Video Experts Team (JVET), 34th Meeting, Rennes, April 2024, Doc. JVET-AH2002.

- “Algorithm description of Enhanced Compression Model 16 (ECM 16)”, Joint Video Experts Team (JVET), 37th Meeting, Geneva, January 2025, Doc. JVET-AK2025.

- “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, Joint Video Experts Team (JVET), 38th Meeting, by Teleconference, March/April 2025, Doc. JVET-AL2010.

- “VTM and HM common test conditions and evaluation procedures for HDR/WCG video”, Joint Video Experts Team (JVET), 29th Meeting, by Teleconference, January 2023, Doc. JVET-AC2011.

Annex A
Using HDRTools to compute distortion metrics

This annex describes the usage of HDRTools to compute distortion metrics. The HDRTools software package can be downloaded from https://gitlab.com/standards/HDRTools. Building instructions are provided in the README.md file included in the HDRTools software package.

Version 0.25 of the HDRTools software should be used. Note that Version 0.25 of the HDRMetrics utility may produce different results on different platforms when non-integer frame rates are provided. To ensure consistent results of the HDRMetrics utility across different platforms, use of an integer frame rate input parameter is recommended. The value of the frame rate parameter should not affect results of HDRMetrics computation.

To compute the PSNR, wPSNR, and MS-SSIM metrics using the HDRMetrics utility, the following command is used from the root HDRTools directory:

build/bin/HDRMetrics \
-f cfg/HDRMetricYUV.cfg \
-p Input0File=<original.yuv> \
-p Input1File=<decoded.yuv> \
-p Input0Height=<height> \
-p Input0Width=<width> \
-p Input1Height=<height> \
-p Input1Width=<width> \

-p Input0BitDepthCmp0=<bitdepth of original.yuv> \
-p Input0BitDepthCmp1=<bitdepth of original.yuv> \
-p Input0BitDepthCmp2=<bitdepth of original.yuv> \
-p Input1BitDepthCmp0=10 \
-p Input1BitDepthCmp1=10 \
-p Input1BitDepthCmp2=10 \
-p NumberOfFrames=<num_frames> \
-p EnableWTPSNR=1 \
-p EnableJVETPSNR=1 \

-p EnableJVETMSSSIM=1 \
-p WeightTableFile=cfg/hdrTable.txt \
-p Input0Rate=60 \
-p Input1Rate=60

The distortion values that are to be reported (see Annex B) are listed on the output line starting with “D_Avg”.

Annex B
Data submission format

Data should be submitted in CSV format. For each sequence and each rate point, the following information should be provided, as applicable. When submitting data for runtime-constrained encoding as per section 3, a separate CSV file shall be provided for each target run time.

| Column name | Description |
| --- | --- |
| Sequence | Sequence name, e.g., SRU1 |
| Rate point | Rate point, e.g., R1 |
| Bitrate | Bitrate in kbit/s |
| PSNR Y | PSNR metric for Y component (See Annex A and [1]) |
| PSNR U | PSNR metric for U component (See Annex A and [1]) |
| PSNR V | PSNR metric for V component (See Annex A and [1]) |
| SSIM Y | MS-SSIM metric for Y component (See Annex A) |
| SSIM U | MS-SSIM metric for U component (See Annex A) |
| SSIM V | MS-SSIM metric for V component (See Annex A) |
| WPSNR Y | Weighted PSNR metric for Y component (See Annex A). To be reported for HRC and HRU categories only. |
| WPSNR U | Weighted PSNR metric for U component (See Annex A). To be reported for HRC and HRU categories only. |
| WPSNR V | Weighted PSNR metric for V component (See Annex A). To be reported for HRC and HRU categories only. |
| EncT | Encoder run time in seconds as measured on CPU in single-threaded mode |
| DecT | Decoder run time in seconds as measured on CPU in single-threaded mode |
| relative EncT | Encoder run time ratio to VTM anchor(e.g., 1.0 if runtime is equal to anchor) |
| relative DecT | Decoder run time ratio to VTM anchor |
| peak Enc Memory | Peak memory use of encoder in KB (see getProcStatusValue("VmPeak:") in VTM software) |
| peak Dec Memory | Peak memory use of decoder in KB |
| MD5sum | MD5 checksum of the decoded YUV file |

The first line in the CSV file should contain the name of the columns. For example:

Sequence;Rate point;bitrate;PSNR Y;PSNR U;PSNR V;SSIM Y;SSIM U;SSIM V;WPSNR Y;WPSNR U;WPSNR V;EncT;DecT;relative EncT;relative DecT;peak Enc Memory;peak Dec Memory;MD5sum

# Annex C
Registration Form

Registration of response to the Joint Call for Evidence on video compression with capability beyond VVC

| Organization: |  |
| --- | --- |
| Contact person: |  |
| Contact email: |  |

## Test cases for evidence on improved compression

| Category | Submission intended |
| --- | --- |
| SDR RA UHD/4K | ☐yes ☐ no |
| SDR RA HD | ☐yes ☐ no |
| SDR LB HD | ☐yes ☐ no |
| HDR RA 4K | ☐yes ☐ no |
| HDR RA Cropped 8K | ☐yes ☐ no |
| Gaming LB HD | ☐yes ☐ no |
| UGC RA | ☐yes ☐ no |

## Test cases for evidence on improved compression with runtime-constrained encoding

Intended number of runtime targets: ____

| Category | Submission intended |
| --- | --- |
| SDR RA UHD/4K | ☐yes ☐ no |
| SDR RA HD | ☐yes ☐ no |
| SDR LB HD | ☐yes ☐ no |
| HDR RA 4K | ☐yes ☐ no |
| HDR RA Cropped 8K | ☐yes ☐ no |
| Gaming LB HD | ☐yes ☐ no |
| UGC RA | ☐yes ☐ no |

☐ Submission includes decoder executables runnable under Ubuntu 24.04 and bitstreams.

☐ Submission includes additional functionality according to Section 5. If box is ticked, please provide below a brief description of the intended visual test (informal viewing, formal expert viewing, recommended viewing setup, etc.):

_______________________________

Configurations for high performance and reduced encoding time are not described for HDR test cases in [6], but can be configured by using equivalent settings as in [5].

Though it is not mandatory to describe the underlying technology in detail, the description should allow an assessment to understand its relevance for prospective standardization, e.g. in terms of processing complexity, memory usage, encoding/decoding delay, relation with existing video compression technology, etc.

Registration by 2025-09-05 is required only if visual demonstration about the benefit of the functionality is planned – otherwise, only submit your descriptive document as input to the JVET meeting by 2025-09-26.