# ISO/IEC JTC 1/SC 29/WG 03 N1704

ISO/IEC JTC 1/SC 29/WG 03

MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Exploration on media authenticity and provenance indication with the MPEG Systems technologies

Status:	Approved

Date of document:	2026-01-30

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	33 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:                https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N 1704

Jan 2026, Online

| Source | MPEG Systems working group |
| --- | --- |
| Status | Output |
| Title | Exploration on media authenticity and provenance indication with the MPEG Systems technologies |
| Serial Number | 25893 |

Table of Contents

1	Introduction	4

2	Background	4

3	System level requirements on technologies for multimedia authentication	4

4	Definitions	5

5	Scope of work	6

6	Producers and receivers	6

7	Use cases	6

7.1	Misinformation and disinformation	6

7.1.1	Deepfake detection by authenticity verification	6

7.1.2	Manipulated media	7

7.1.3	Virtual meeting authentication	7

7.1.4	Impersonation using video and audio	7

7.1.5	Fake news with synthesized audio	7

7.2	Media forgery	7

7.2.1	Insurance fraud	7

7.2.2	Surveillance footage integrity	7

7.2.3	Audio recordings in investigations	7

7.3	Media creation	7

7.3.1	Authenticity & integrity for media	7

7.3.2	AI-generated content (AIGC)	8

7.4	Media modification	8

7.4.1	Video and audio editing software	8

7.4.2	Media adaptation during transport	8

7.4.3	Content splicing and ad-insertion	8

7.4.4	Video and audio transcoding for streaming	8

7.4.5	Movie and audio preservation	8

7.5	Media augmentation and extension	8

7.5.1	Media composition	8

7.5.2	Identifying augmented reality (AR) and virtual reality (VR) components	8

7.6	General characteristics of the use cases	9

8	System level requirements	9

8.1	General aspects	9

8.2	System level specific requirements	9

9	Gap Analysis	11

9.1.1	m73494 Analysis of systems media authenticity requirements based on MPEG-7 and MPEG-21	11

9.1.2	m73485 Gap analysis of ISOBMFF and DASH to support media authentication requirements	11

9.1.3	m73486 Gap analysis of C2PA to support media authentication requirements in ISOBMFF and DASH	11

9.2	m73486 Gap analysis of C2PA to support media authentication requirements in ISOBMFF and DASH	11

10	Workflows (based on m74553 and m7440)	11

10.1	Streaming Workflow (m74553)	11

10.2	File-based workflow (m74553)	13

10.3	Cross authentication in the file-based delivery workflow (m74440)	14

10.4	Content properties (m74553)	15

10.5	Authentication properties (m74553)	15

10.6	Interfaces (m74553)	16

10.7	Authentication functions (m74553)	16

10.7.1	Authentication author	16

10.7.2	Authentication verifier	17

10.8	Interfaces’ features (m74553)	17

10.8.1	AIF-1a	17

10.8.2	AIF-1b	17

10.8.3	AIF-1c	17

10.8.4	AIF-2a	17

10.8.5	AIF-2b	17

10.8.6	AIF-2c	18

10.8.7	AIF-3c	18

11	Potential work items (m74553)	18

11.1	ISOBMFF	18

11.1.1	System-level authentication information in tracks/track fragments	18

11.2	Elementary authentication information in tracks	19

11.3	DASH	19

11.3.1	Signaling at the manifest	19

12	Proposed candidate solutions	19

12.1	Elementary Stream Authentication in ISOBMFF (m75633)	19

12.1.1	Introduction	19

12.1.2	Rational	19

12.1.3	Digitally Signed Content SEIs	19

12.1.4	Elementary stream authentication signalling	20

12.1.5	Design principles	20

12.1.6	Syntax	20

12.1.7	Semantics	21

12.1.8	Inclusion in ISOBMFF	22

12.1.9	Characteristics	22

12.1.10	Applications	22

12.2	Generic Authentication in ISOBMFF (m75709)	22

12.2.1	Introduction	22

12.2.2	Motivation	22

12.2.3	Use cases	23

12.2.4	Background	23

12.2.5	Rational	23

12.2.6	Generic authentication signalling	23

12.2.7	Inclusion in ISOBMFF	25

12.2.8	Characteristics	25

12.2.9	Use of C2PA with AuthBox	25

12.2.10	Use of ElementaryAuthBox with AuthBox	26

12.2.11	Applications	26

12.2.12	Authentication group box	27

12.3	Media cross authentication signaling in ISOBMFF (m74440, m75550)	27

12.3.1	Introduction	27

12.3.2	Media cross-authentication for ISOBMFF-based container	28

12.3.3	Authentic Entity Group Box	28

12.4	Signaling in MPEG-2  TS (m74512)	32

13	Discussions at MPEG152	32

14	Recommendations	32

15	References	32

# Introduction

This document defines the scope of a WG03 exploration on media authentication and AI-generated/altered content marking in the MPEG Systems standards.

Media authentication, i.e. verification of the truthfulness of media content, through various means has been the subject of discussion, especially in the light of AI generated and altered content. Various news incidences involving the deepfakes, misinformation and disinformation about politics and elections, and public figures, and even in other popular subjects caused the legal authorities in various countries looking into legal aspects of media authentication and the need to require the means of authentication of a publish content for some applications and distribution platforms. While those activities are moving forward and their impact will be anticipated, the technical means enabling media authentication are highly desired to implement those legal requirements as well as the market needs.

# Background

The need for media authentication and AI generated/altered content marking has been raising recently. Legislative works have been started in several jurisdictions that attempt to provide requirements for such solutions.

The recent standard activities such as C2PA and JPEG TRUST  address some of the needed functionalities for certain media authentication use cases. However, the ranges of use cases and application are vast. The extend of the use cases and application, as well as various delivery schemes are outline in WG02-NXXXX.

In the technical field, the standard organizations and consortia have been recently investigated or started development of standards around media authentication. In MPEG, WG3 with the exploration on marking AI generated/altered media (MDS24619_WG03_N01411), and JVET/WG5 with digital signed SEI messages have been working on this topic (JVET-AK2006). Recently, WG02 completed the requirements work on the media authentication (WG02_N0427), with the goal of creating an umbrella media authentication requirement document for the entire MPEG community. This document defines the main use cases and the technical requirements. Futhermore, contribution m71392 proposed a subset of the WG02_N00413 requirements for the MPEG system standards and contribution m71125 proposed a method for the use of C2PA with MPEG DASH to address some of the requirements.

# System level requirements on technologies for multimedia authentication

Multimedia content, including text, images, audio, and video, is integral to modern life.  Its reliable exchange is supported globally by ISO/IEC JTC1/SC 29 standards. However, its trustworthy usability is increasingly challenged by sophisticated forgeries and manipulations like AI-driven deepfakes. Since such techniques blur the lines between reality and fabrication, they raise significant cybersecurity and trust issues within digital ecosystems.

It is observed that for some of the ISO/IEC JTC1/SC 29 standards like MPEG-D USAC or VVC, no mechanisms for trustworthy verification exist which, on the one hand, can be realized in an independent and self-contained way within these standards but which, on the other hand, can be combined with each other and with other layers of the MPEG-ecosystem. While the first aspect is crucial for applications like independent streaming, the second aspect is important to prevent forgeries via fake combinations of individually verified media assets.

In parallel, the rapid evolution of artificial intelligence (AI) technologies—particularly in large language models, image and video generation, and multimodal systems—has significantly transformed industries like entertainment and healthcare by enhancing productivity and creativity. However, this progress has also led to ethical concerns regarding the proliferation of deceptive content. The growing prevalence of AI-generated content underscores the urgent need for novel verification technologies to maintain media authenticity and prevent malicious usage, restoring trust in digital sources.

This document captures the subset of requirements defined in WG02_N042 as the system level requirements for the media authentication.

# Definitions

This section provides the definition of terms used in this document.

| Media types | Types of media including 2-D and multi-component video and visual (such as graphics and animation), audio, closed caption, and haptics. |
| --- | --- |
| Media asset | A media content that may consist of one or more media types, and one or more tracks of each type, which together constitute a media presentation. |
| Media asset metadata | The portion of a media asset that provides information about the media asset or its content that is not directly used for rendering the media presentation, such as location, creator, annotations or IPR information [2]. |
| Media authentication | Process or action of proving or showing a media asset being true, genuine, or valid. |
| Media authenticity | The truthfulness and reliability of media content, ensuring that it accurately represents the claimed information or events. |
| Media integrity | The preservation of media content in its original state without any unauthorized modifications or corruption during storage or transmission. |
| Producer | An entity, person or organization that produces, edits, revises or manipulates a media asset. |
| Receiver | An entity, person or organization that receives, retrieves, or streams the content for consumption as the end user or for the purpose of producing another content. |
| Deepfake | A media (typically of a person or persons) which has been digitally altered, typically to be used maliciously or spread false information. |
| Misinformation | Information that is based on reality, used to inflict harm on a person, social group, organization or country [1]. |
| Disinformation | Information that is false and deliberately created to harm a person, social group, organization or country [1]. |
| Media forgery | The intentional manipulation, alteration, or creation of digital media—such as images, videos, or audio—with the purpose of misleading or deceiving viewers about the authenticity or truthfulness of the content. |
| Trust verification | The process of verifying that the media is produced by a trusted producer. |

# Scope of work

The scope of this work is the development and/or enhancement of the specification(s) for the inclusion of necessary metadata in the system layer streams, for enabling the authentication of the corresponding media assets. During the development of such standards, MPEG Systems may leverage the existing specifications, when they satisfy parts of the requirements, and develop new metadata formats and procedures to fulfill other requirements for addressing the use cases defined in this document.

# Producers and receivers

Before discussing the use cases, it is important to note to categorize the producers and receivers of the content that is the subject of media authentication.

The producers of the content can be categorized into two major groups:

- Professional producers (PP): The professional organizations and entities that produce the content, such as news agencies and TV networks.

- General producers (GP): Individual users that capture or produce content, for instance the social network subscribers who post media content on their account on any social network.

Similarly, the receivers of the media can be grouped into two main categories:

- General Receiver (GR): A public user usually consumes the media content and is interested in verifying the authenticity of the content. The main task is the authentication and the alternation and editing details of the content is usually not a concern for such user.

- Professional Receiver (PR): A professional user, such as a journalist, news agency or authority usually has an interest in the origin of the content and the various steps of alternation, editing and manipulation the content went through, and the sources who perform such alternation and editing on the content.

# Use cases

One of the key objectives of this document is to identify use cases that fall under the scope of multimedia authentication and to identify needs for standardization.

The following use cases have been identified as relevant.

Misinformation and disinformation

Deepfake detection by authenticity verification

The transfer operation requires confirmation of the identity of the content provider (e.g. in order to determine whether, when a famous person is making questionable claims, the media is genuine and not a deepfake). Generated by PP/GP and used by GR and PR users.

Manipulated media

A GR user wants to confirm whether media, generated by a PP, has been altered when published (e.g., to confirm the authenticity of a news video that shows an important event or to verify whether an audio attached to a video of some individual actually corresponds to that video and not to an older video of the same individual).

Virtual meeting authentication

In business or legal virtual meetings, participants want to ensure that none of the voices have been altered or synthesized to impersonate key decision-makers.

Impersonation using video and audio

Public figures, celebrities, or business executives are often targeted by fake videos and voiceovers to damage their reputations or impersonate them for malicious purposes. Identifying forged videos or synthesized voices is critical for protecting personal and professional identities. (e.g., A celebrity’s voice is faked in an online interview to make controversial statements. The celebrity needs tools to verify the authenticity of the media to refute the forgery.)

Fake news with synthesized audio

In media reports, audio can be synthetically generated to fake statements from public figures. Verification of audio source and authenticity is essential to prevent the spread of fake news. (e.g., An audio clip allegedly featuring a famous actor making offensive comments circulates online. Media outlets need to verify the audio’s origin to confirm whether it was artificially generated.)

Media forgery

Insurance fraud

An insurance company (PR) wants to confirm the authenticity of some media during a claim process (e.g., a video of a car accident), for the content produced by a GP.

Surveillance footage integrity

Security teams and law enforcement (PR) must validate that surveillance camera footage (PP/GP) has not been tampered with when used in investigations of criminal activities.

Audio recordings in investigations

Investigators (PR) want to confirm that audio recordings (PP/GP) used in criminal cases (e.g., wiretaps or interviews) have not been edited or distorted to manipulate the context of the conversation.

Media creation

Authenticity & integrity for media

Media organizations (PR) want to confirm the authenticity of the received media assets from PP/GPs.

AI-generated content (AIGC)

With the rise of AI-generated videos and synthesized audio, receivers (GR) and professionals (PR) alike need to know whether media was created or modified using AI software (PP/GP). Identifying AI-generated content is crucial for transparency. (e.g., Viewers want to verify whether a popular video clip was created using AI or is a genuine, human-created piece of content.)

Media modification

Video and audio editing software

The extent of editing on video and audio content should be made transparent for the receiver (PR/GR). For example, professional editing tools (PP) used to enhance or alter content should label post-processing details to indicate authenticity. (e.g., A film editor needs to mark the sections of a movie trailer that have been color-corrected and where the audio has been pitch-adjusted for effect.)

Media adaptation during transport

The throughput of the network might change over time and some techniques may be applied to adapt it to the network conditions while still being able to authenticate the media (e.g., for scalable media codecs some layers might be dropped, or a media stream could be transcoded.) Applicable to both GR and PR.

Content splicing and ad-insertion

The media might be spliced (e.g. by inserting the advertisement in the main content or added graphics and animation.) The various parts of the original media need to be verifiable for authenticity. Applicable to both GR and PR.

Video and audio transcoding for streaming

Audio and video transcoding during streaming processes must retain authenticity verification, ensuring that the content hasn’t been altered beyond codec adjustments. (e.g., An online streaming platform needs to verify that the transcoding of audio tracks to lower bitrates for mobile devices has not altered the content’s original integrity.)

Movie and audio preservation

Compression, transcoding, and other preservation processes applied to classic films and audio recordings must be documented to maintain transparency. This ensures that the archived content can be trusted as faithful to the original. (e.g., A film archive wants to keep track of the transcoding steps applied to preserve an old movie for modern viewing formats, while preserving its original quality.)

Media augmentation and extension

Media composition

Judges and police (PR) want to know whether media evidence (GP and/or PP) is synthesized when adjudicating cases (e.g., when an edited video is composed of several ones).

Identifying augmented reality (AR) and virtual reality (VR) components

In AR and VR content, both video and audio elements may be augmented or generated in real time. Ensuring that these augmented components are clearly distinguished from real-world captures is important for maintaining authenticity.

General characteristics of the use cases

As can be seen from the described use cases, media authentication typically invokes several media components.

In a typical use case, a content provider wants to distribute a compressed version of a multimedia asset that consists of a combination of one or more video sequences, multiple audio streams (in different languages) and multiple caption streams for the respective audio streams. The content is generated and distributed on the fly, for example in a live streaming scenario. Moreover, users across the world may want to consume different versions of the content. For example, some users may want to consume an individual audio stream or the video stream only or they may want to consume specific versions of such media content depending on their available bandwidth, as, for instance, the video at a particular resolution or audio at a particular language or bitrate. As such, joint authentication should be possible for such media combinations.

Furthermore, as discussed in the use cases above, only parts or substreams of the original video or audio streams might be received depending on the available bandwidth, for example by invoking a scalability functionality of the underlying codec.

In summary, it is the intention of the content provider and of all users that the content provider can be trustworthily identified as the originator of each distributed substream of the media asset and that if multiple substreams are jointly consumed, users can verify their coherence, in order to for example guarantee that an audio stream was indeed intended to be associated to a video stream by the content provider.

Note: To address the above use cases, the standard requires carriage of embedded data in the media stream(s). Therefore, discovering if a media asset is fake or not, without carriage of the required embedded data is out of the scope of the solutions.

# System level requirements

General aspects

The requirements listed fall within the scope of metadata embedding and referencing for verifying the media authenticity.

Solutions need to fulfill generic requirements for media streaming, such as dealing with random access into the streams, dynamic adaptive switching and content splicing.

In order to meet the requirements listed below for a single compressed version of the multimedia asset, it is desirable that the underlying compression standards like MPEG-D USAC or VVC both support a trustworthy verification mechanism that can be carried out independently (supporting important functionalities of the standards) but at the same time link such a verification mechanism to a joint verification method for substreams associated to different media types/codecs. Such a joint verification mechanism may be supported by the relevant media data at the system level. Therefore, requirements are developed that require solutions to be transported in media bitstreams and system streams to enable media authentication.

System level specific requirements

Based on the identified use cases, the following requirements have been identified.

- The solution shall support the verification of media content encapulated in ISOBMFF or delivered with MPEG DASH.

- The solutions shall provide means to embed authenticity information into media assets.

  - A media asset may consist of several media types and each media type may have one or more media tracks and potentially along with the associated DASH manifest. One or more media types also may be encapsulated in one media track. The solutions shall be flexible enough to embed such information at the elementary streams, as well as the system layer(s) to signal the cross-authentication of one or more elementary streams.

  - Trust verification of several media types in separate tracks/streams shall ensure that the correlation and synchronization between different streams can be verified.

- The solution shall support digital signing of media assets, metadata and content by producer(s) to establish authenticity and integrity of the content by:

  - Supporting various hashing methods.

  - Supporting verification of each media component by information embedded in the media component itself.

  - Supporting joint verification of more than one media component associated with each other.

- The solution shall support the carriage of the provenance information and/or the binding data to the provenance information with the media content. The solution shall be flexible enough to support various provenance schemes (e.g. schemas required by different regulations).

- The solutions shall provide means to allow for random access into the media tracks/streams while, at the same time, allowing for verification of temporal coherence between random access segments.

- The solution shall provide means to allow the dynamic switching of the media tracks/streams, when applicable, e.g. in the case of streaming, switching between different representations of the same media component shall allow the verification of the media component as a single part of a media asset.

- The solution shall provide the possibility for the verification of different time intervals of the media assets, i.e. the authentication of each time interval of a media asset may need to be done independently of the other periods of the media.

- The solution shall allow the verification of media assets that are captured and delivered in real time (i.e. live) as well as for later delivery (i.e. on-demand). It should offer configurable latency for the authentication process. The viewer shall be able to verify the content as being delivered in real time regardless of whether it was produced in real time or post-processed after production.

- The solution shall allow the verification of the content distributed via broadcast or other means of one-way distribution where authentication information (e.g. keys) are exchanged via side channels (e.g. using secure on-device storage or other 
“offline” methods).

- The solution shall provide means to modify specific parts of the media streams while still allowing verification of authenticity for media assets by:

  - Supporting scalability when codecs allow for it.

  - Support multiple component coding (e.g. multi-view or depth-map, texture, geometry, etc.)

  - Supporting authoring of secondary information and metadata (such as MPEG-H 3D audio user interaction and DRC metadata) in a downstream device.

  - Support for media assets that are stored in single or multiple files, where applicable.

  - Support of late binding between different media components or media objects.

- The solution should, as much as possible, provide some means of robustness against loss such as video frame loss and/or audio dropouts, allowing partial verification of more important parts in cases of partial loss.

- The solution should provide means to verify different substreams of a media track/stream with different verification and/or hashing methods, where applicable.

- The solution shall provide means to embed authenticity information into media tracks and/or associated DASH manifestsin a way that does not have a significant impact on the bitrate (average, peak) of the media bitstream as much as possible.

# Gap Analysis

The following is a summary of contributions discussed at MPEG#151, Daejeon, meeting.

m73494 Analysis of systems media authenticity requirements based on MPEG-7 and MPEG-21

MPEG 7 and MPEG 21 need extensions to be useful for media authentication and since they are not widely adopted, it is recommended to start an activity to extend ISOBMFF and DASH independent from them. The assumed scope is not to create a new provenance format but support of carrying provenance information in those standards.

m73485 Gap analysis of ISOBMFF and DASH to support media authentication requirements

Several gaps are identified for ISOBMFF and DASH by going through the requirements and discussing what needs to be used in ISOBMFF and/or DASH and what is missing.

m73486 Gap analysis of C2PA to support media authentication requirements in ISOBMFF and DASH

A gap analysis of using of C2PA with ISOBMFF and DASH is provided. There were different views at the meeting on  how C2PA can be used with ISOBMFF and DASH.

m73486 Gap analysis of C2PA to support media authentication requirements in ISOBMFF and DASH

C2PA for live streaming has challanges for temporal binding of segments. A solution is proposed based on DSC VSEI and to carry the information at the system layer.

# Workflows (based on m74553 and m7440)

Streaming Workflow (m74553)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/32

Figure 1 defines a general end-to-end workflow for media authentication of media streaming applications.

Figure 1: The media authentication workflow for streaming

In this figure, the workflow consists of:

- Media is captured by the device. Raw authentication information is provided to the authentication creator (AIF-1a).

- The content is edited, manipulated and mixed with prestored and/or generated content. The provenance information is updated (AIF-1b)

- Media is encoded and packaged. The authentication creator runs algorithms on the data (AIF-1c), interacts with the certificate infrastructure if needed (AIF-4a) and includes the authentication metadata in encoded and packaged data (AIF-1c). Optionally, it may include storing some data in the cloud (AIF-4a).

- Media is provided to the content /service provider.

  - The content received may be verified by the content/service provider (AIF-2a).

  - The authentication metadata may be removed/redacted by the content/service provider.

- The content is edited and revised. It may be manipulated, enhanced or revised and/or combined with other pre-stored or generated content. The provenance information is provided to the authentication manipulation/authoring module along with the provenance information provided at the contribution (AIF-2a). The new parts of the content may have their own provenance information.

- Additional authentication metadata is included during the media packaging and manifest generation (AIF-2b/2c).

- The media presentation may be repackaged at the network or client side or transmitted by a transmitter. Such operations maintain the authentication metadata for the part of the media originated from the service provider.

  - The inserted content (such as inserted ads) should not impact the authentication metadata nor the verification of the main content.

  - Each inserted content may add its own authentication information.

  - If the inserted content causes failure in the verification of the main content, it should either

    - Remove signaling that the manifest to avoid confusing the player, or

    - Add the new authentication metadata for the combined content.

- The player/receiver uses the verification module (AIF-3) for media verification through AIF-5.

  - The player may use the manifest to select the media that can be verified.

  - The verification may occur during the playback of media.

File-based workflow (m74553)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/32

Figure 2 describes the workflow for file-based transfer applications.

Figure 2: The media authentication workflow for file-based transfer

In this figure, the workflow consists of:

- Media is captured by the device. Raw authentication information is provided to the authentication creator (AIF-1a).

- The content is edited, manipulated and mixed with prestored and/or generated content. The provenance information is updated (AIF-1b)

- Media is encoded and packaged into one or multiple files. The authentication creator runs algorithms on the data (AIF-1c), interacts with the certificate infrastructure if needed (AIF-4a) and includes the authentication metadata in encoded and packaged data (AIF-1c). Optionally, it may include storing some data in the cloud (AIF-4a).

- Media is provided to the content /service provider.

  - The content received may be verified by the content/service provider (AIF-2a).

  - The authentication metadata may be removed/redacted by the content/service provider.

- The content is edited and revised. It may be manipulated, enhanced or revised and/or combined with other pre-stored or generated content. The provenance information is provided to the authentication manipulation/authoring module along with the provenance information provided at the contribution (AIF-2a). The new parts of the content may have their own provenance information.

- The media presentation may be repackaged at the network or client side or transmitted by a transmitter. Such operations maintain the authentication metadata for the part of the media originated from the service provider.

  - The inserted content (such as inserted ads) should not impact the authentication metadata nor the verification of the main content.

  - Each inserted content may add its own authentication information.

  - If the inserted content causes failure in the verification of the main content, it should either

    - Remove signalling that the manifest to avoid confusing the player, or

    - Add the new authentication metadata for the combined content.

- The player uses the verification module (AIF-3) for media verification through AIF-5.

  - The verification may occur during the playback of media.

The media authentication interfaces are a subset of the ones identified for the streaming workflow. All interfaces are the same except AIF-2c, which does not exist in this case.

Cross authentication in the file-based delivery workflow (m74440)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/34

Figure 3 is the file-based workflow (originally presented in the contribution m73957 [7]) modified here to highlight where cross-authentication (X-auth) mechanisms may be applied.

Figure 3 File-based workflow with cross authentication (X-Auth) of different Media types

AIF-5

On the contributor side, media components possibly with different media types (e.g. video, audio tracks, subtitles, etc.) of a media asset are captured (original content), retrieved or generated. Each component is authenticated individually through the AIF-1a and AIF1b interfaces. Once authenticated, all media components with different media types building the media asset are packaged with cross-authentication performed through the AIF-1c interface. The resulting packaged file is then made available to the service provider. There can multiple contributors providing media components with different variants or different media types to the Service provider e.g. different audio tracks, etc.

On the service provider side, the packaged file can be edited by modifying existing media types, reusing prestored content or incorporating new ones (possibly from multiple contributors). Similar to contributors, the service provider must ensure cross-authentication of the new packaged file, handled through the AIF-2a and AIF-2b interfaces.

Content properties (m74553)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/32

The following content properties are considered:

- The content may be produced for on-demand, live or low-latency live consumption.

- The content may be produced as single or multi-period content by the contributor and further be spliced to multi-period content by the provider.

- Time-based events may be added by the contributor or service provider to the content.

- The content may be manipulated by the provider or by the client to be spliced with other content (such as ads). Use cases such as DASH MPD insert event, with dynamic ad duration and early termination, are considered.

Authentication properties (m74553)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/32

The following authentication properties are considered:

- The authentication metadata provides information to verify the content for specific verification periods, i.e. each time interval of the content has authentication metadata.

  - Since the content may be produced on-demand, live or in the low-latency case, the media verification periods are defined to enable each use case accordingly.

  - If authentication verification requires insertion of period transition (i.e. changing the period structure), the resulting content shall conform to the DASH standard, including not losing the ad insertion opportunities.

  - Adding ad insertion opportunities shall still allow verification of the content for each verification period intended by the service provider.

  - A pass-through verifiable content is only possible if the content is contained in one or more periods, marked as “unchangeable”.

  - Cross-media authentication, i.e. authentication between different tracks or representation of media components, needs to be addressed.

- The manifest and its updates may provide:

  - High-level information about the existence of authentication metadata and their format, and possible playback constraints.

  - The cross-media authentication information, i.e. which tracks are expected to be authenticated together.

Interfaces (m74553)

The following common interfaces are identified in the above workflows:

- AIF-1a: capture provenance data

- AIF-1b: authentication data created during editing

- AIF-1c: authentication data embedding in encoding/packaging

- AIF-2a: provenance data update during authoring

- AIF-2b: authentication data embedding in the media content

- AIF-2c: authentication data embedding in manifest/packaging

- AIF-3: extraction of authentication data for verification

- AIF-4a/4b: auth. author cloud interface

- AIF-5: auth. verifier cloud interface

Authentication functions (m74553)

Authentication author

- Validate the previously authenticated media

- Receive capturing data and create provenance information

- Create and update the provenance information

- Calculation of hashes and signatures.

Authentication verifier

- Verification of the previously authenticated media

Interfaces’ features (m74553)

AIF-1a

#### Capture-> Auth. Author

- The capturing provides the authentication authoring module with the characteristics of the capture information, such as time, location, and format. This interface is out of the scope of MPEG ISOBMFF and DASH work.

AIF-1b

#### Editing-> Auth. Author

- Provenance information of the generated/prestored content along with its timing information

AIF-1c

#### Encoder/Packager-> Auth. Author

- Encoded media tracks

#### Auth. Author-> Encoder/Packager

- Provenance information packaged in ISOBMFF boxes

- Signaling authentication in various tracks/track fragments

- Signaling authentication information in the DASH manifest

AIF-2a

#### Encoder/Packager-> Auth. Author

- Authentication information, including provenance information of

  - Received content

  - Generated/Prestored content

#### Auth. Author-> Encoder/Packager

- Verification results of the received content

AIF-2b

#### Encoder/Packager-> Auth. Author

- Encoded media tracks

#### Auth. Author-> Encoder/Packager

- Provenance information packaged in ISOBMFF boxes

- Signatures

- Signalling of the authentication in various tracks/track fragments

AIF-2c

#### Packager-> Auth. Author

- Packaging information

#### Auth. Author-> Encoder/Packager

- Signaling of the authentication information for the DASH manifest

AIF-3c

#### DASH Access client-> Auth. Verifier:

- Authentication info at manifest for selection

- Media tracks/segments for verification

#### Auth. Verifier-> DASH Access client

- Selection of the adaptation sets that can be verified

- Verification results

# Potential work items (m74553)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/32

ISOBMFF

System-level authentication information in tracks/track fragments

- There is no standard mechanism to include the authentication metadata for each track/track fragment and cross authentication info (between tracks) in ISOBMFF.

- Currently C2PA uses uudi/udta to include c2pa information (as a JUMBF box). In this case, the cross-authentication information exists in the C2PA manifest. Using the ISOBMFF dedicated first-class citizen box to signal the media authentication scheme and its parameters (including C2PA) is a more preferred solution since the discovery and playback of that box would be more universally supported. Such signaling should cover multi-rate encoding of the same media component as well as the cross media component authentication information.

- Adding a signaling mechanism at the file format level is useful to indicate the authentication information of various tracks and how they are related to each other, enabling a parser to identify the correlation between tracks before invoking the authentication verification and simplify AIF-3.

Elementary authentication information in tracks

- There is no standard mechanism to signal the existence of authentication and its characteristics of elementary streams at the track level.

- There is no mechanism of signaling cross-track authentication for elementary authentication information.

- There is no mechanism to tie the elementary stream authentication to the system layer authentication.

DASH

Signaling at the manifest

- The DASH standard does not support a standard way to signal the existence of the authentication information in the manifest. The signaling can be used for track selection as well as to simplify the interaction between the DASH access player and authentication verifier. The signaling can include signaling of the scheme and exposing high-level information of the scheme, as well as the relationship between the media authentication of various adaptation sets.

# Proposed candidate solutions

Elementary Stream Authentication in ISOBMFF (m75633)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/44

Introduction

This section proposes an elementary stream authentication box for signaling existence of the elementary stream authentication in ISOBMFF.

Rational

Version 4 of VSEI () defines Digitally Signed Content SEIs enabling signaling authentication with encoded video bit streams. Similarly, the new amendments in various audio codecs (AAC, USAC and MPEG-H) allows the same functionality.

The existence of media authentication metadata at elementary stream can be exposed at the system layer. The value is that a file format parser can discover the existence and high-level characteristic of the metadata in track and expose that to the application for the purpose of selection.

Digitally Signed Content SEIs

In this section, we review the Digitally Signed Content (DSC) SEIs as an example of the elementary stream authentication.

The VSEI V4 states:

“The digitally signed content initialization (DSCI) SEI message, digitally signed content selection (DSCS) SEI message, and digitally signed content verification (DSCV) SEI message provide a mechanism for verifying that the coded video has been produced by a content provider that identifies itself via the digital certificate that is referenced in the DSCI SEI message. These three SEI messages are collectively referred to as the digitally signed content (DSC) SEI messages.”

The DSC SEIs defines the following parameters:

- Id for identification of various DSC SEIs.

- Used hashing method.

- Key retrieval mode, for retrieval of the key where is a C2PA manifest or a certificate.

- Key source URI, the location/index of the key.

- Optional key register index in certificate registry.

- Optional content UUID.

- Start flag, signaling the signed start point.

- End flag, signaling the signed end point.

Elementary stream authentication signalling

If an elementary stream authentication exists (such as VSEI DSC SEI in video streams or audio authentication metadata in audio streams), we propose to include the high-level information of such schemes in a media authentication box.

Design principles

If elementary stream authentication exists (such as VSEI DSC SEI in video streams or audio authentication metadata in audio streams, the high-level information of such schemes is included in a media authentication box. This allows a file reader to signal authentication metadata to the application, and the application can verify if they are supported or not.

These characteristics include:

- Description of used scheme and profile

- If a fragment is start or end of an independent verification period

- Out-of-band dependent information (information that authentication of this file, track, or track fragment is dependent on), that may exist in an outside resource, another file, another track of the same file, or another fragment.

- Associated files, tracks, or track fragments that are part of the overall authentication of the media presentation.

- Inclusion of specific authentication scheme metadata, such as elementary stream authentication or C2PA.

- Ability to sign the entire box, therefore protecting it against any tampering.

Syntax

aligned(8) class ElementaryAuthBox extends FullBox('eaut', version = 0, 0) { utf8string schemeURI; utf8string profile; bit(1) signed_content_start; bit(1) signed_content_end; bit(1) hash_flag; bit(1) key_retrieval_mode_flag; bit(1) content_id_flag; bit(1) dependency_flag; bit(1) association_flag; bit(1) signed_flag; if (hash_flag == 1) unsigned int(8) hash_method; if (key_retrieval_mode_flag == 1){ bit(1) key_retrieval_mode; unsigned int(10) key_register_indx; bit(4) reserved; utf8string keysourceURI; } if (content_id_flag == 1) unsigned int(8)[16] content_uuid; if (depencency_flag == 1) unsigned int(8)[16] dependent_uuid; if (association_flag == 1) unsigned int(8)[16] association_uuid; if (signed_flag == 1){ unsigned int(8) signature_hash_method; utf8string signature_keysourceURI; unsigned int(10) signature_key_register_indx; bit(6) reserved; unsigned int(24) signature_length; for (i=0; i<signature_length, i++) unsigned int(4) signature[i];

Semantics

schemeURI declares the identifier of the authentication scheme.

profile is the profile of the declared scheme that the elementary scheme conforms to.

signed_content_start value 1 indicates the intended beginning of the signed content, i.e. the content can be verified independent from previous media fragments.

signed_content_end value 1 indicates the intended end of the signed content, i.e. the content can be verified independent from the next media fragments.

hash_flag value 1 indicates that the hash algorithm is fixed for the entire track/fragment and defined by hash_method.

key_retrieval_mode_flag value 1 indicates that the key retrieval mode is fixed for the entire track/fragment and defined by key_retrieval_mode, and the corresponding URI is defined by keysourceURI, and if an index exists, by key_register_idx, respectively.

content_id_flag value 1 indicates inclusion of the content_id field.

dependency_flag value 1 indicates inclusion of the dependency_id field.

association_flag value 1 indicates inclusion of the association_id field.

signed_flag value 1 indicates the inclusion of the signature.

hash_method indicates the hash algorithm used to calculate the message digest. The values are defined according to ISO/IEC 23002-7 Table 34.

key_register_indx defines the index that specifies the certificate of the content provider, in the certificate register indicated by key_source_uri. The valid range is 0 to 1024, inclusive. The value 1024 means that the index is not defined.

keysourceURI defines the key source URI for key retrieval.

key_retrieval_mode defines the mode of key retrieval. The values are defined by the authentication scheme and profile signalled in schemeURI and profile, respectively.

content_uuid specifies the content identifier UUID.

dependency_uuid specifies the content_uuid value of the authentication box that this box's authentication metadata is dependent on.

association_uuid specifies a UUID value of this box and all authentication boxes of the associated entities. The associated entities are the entities in which their authentication is expected to be jointly verified.

signature_hash_method indicates the hash algorithm used to calculate this box’s digest. The values are defined according to ISO/IEC 23002-7 Table 34.

signature_key_register_indx defines the index that specifies the certificate of the content provider for verifying the signature of this box, in the certificate register indicated by key_source_uri. The valid range is 0 to 1024, inclusive. The value 1024 means that the index is not defined.

signature_keysourceURI defines the key source URI for key retrieval for signature verification of this box.

signature_length defines the length of the signature.

signature includes the [i] element of the signature. The signature is calculated from the entire box’s digest except signature_length and signature fields using the hash algorithm defined in signature_hash_method.

Inclusion in ISOBMFF

The defined elementaryAuthBox can be included in several places:

- For track level (inside trak)

- For the fragments:

  - Inside the moov box for initialization

  - Inside each media fragment in one of the following ways:

    - Top-level box after styp box

Characteristics

- The box can be used for signalling of the elementary stream, in a very light way, with no parameter signalling, or it can be used, including various parameters that stay constant for the duration of the track/fragment.

- The box can be used to indicate the dependency of the authentication metadata at the elementary stream level to other resources, including authentication metadata in other resources, for instance, another file, track or track fragment.

- The box can be used to indicate the associated files, tracks or fragments for authentication purposes, i.e. the authenticity of those resources needs to be verified together.

- Finally, the information in the box can optionally be signed and therefore verified by the player.

Applications

#### Track Level

In the case of file playback, the file parser can retrieve the information from the track before the playback of the file and see if the media player supports the authentication method/scheme. It can also be used for track selection, i.e. if there are multiple tracks with different elementary stream authentication, the player can choose the tracks whose authentication verification is supported by its media player.

#### Fragment level

In the case of a fragment playback, the file parser can parse the initialization segment to see the general characteristics of the track authentication, and then, when it parses a specific fragment, it can parse further information.

Generic Authentication in ISOBMFF (m75709)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/46

Introduction

This section proposes a generic authentication box for signaling any authentication scheme in ISOBMFF.

Motivation

Media authentication has become an important feature in media delivery and playback. The media content carries metadata that enables verification of the content's authenticity. MPEG Systems has an exploration activity in this area (WG03N1621). A gap analysis has been performed as well as defining the media authentication workflows in media contribution, delivery and playback. The conclusion so far is that the ISOBMFF and DASH standards need to address a better binding and annotation of the media authentication metadata when it exists in a media presentation.

Use cases

The following use cases are considered for ISOBMFF:

- A track carries authentication metadata for the entire track.

- A track fragment carries authentication metadata for the track fragment.

- A set of tracks has metadata that defines the authentication of the tracks together.

- A track carries the metadata signalling that its elementary stream has media authentication.

- In any case above, one or more sets of media authentication metadata, i.e. using different media authentication schemes, may exist.

Background

C2PA specification defines a UUID box to include its media authentication in ISOBMFF. The C2PA metadata is usually included in ISOBMFF either at the file level or at the fragment level. C2PA does not cover the following cases:

- Signalling the existence of authentication metadata at the elementary stream level

- Signalling of the cross-track authentication without parsing the C2PA manifest, if the tracks are not in the same file.

Recently, the VSEI (ISO/IEC23002-7) developed the Digitally Signed Content (DSC) SEIs that enable authenticating video elementary streams. Similarly, the AAC, USAC and MPEG-H audio amendments are under development for signalling authentication metadata with audio elementary streams.

Therefore, existence of different media authentication metadata schemes in a media presentation in many cases might be unavoidable. If ISOBMFF has a standard mechanism for discovery and carriage of such data, file parsers can discover various media authentication metadata more easily and efficiently.

Rational

A single location in ISOBMFF for media authentication data has several benefits:

- All file format parsers know where to find such information.

- If a content has media authentication information, in any schemes, can be discovered easily by an editor, and the information can be read and revised.

- Multiple authentication schemes, or multiple instances of an authentication scheme can be stored in the same location and easily be discovered, including system layer authentication as well as elementary level authentication.

- Cross track/fragment verification, i.e. grouping of the resources for verification, can be signaled independent than authentication scheme, and therefore simplifies discovery of dependencies of tracks/fragments for the application, even before running the verification process.

Generic authentication signalling

We also define a generic authentication box, which can signal the existence of any authentication scheme along with the corresponding authentication metadata.

#### Syntax

aligned(8) class AuthBox extends FullBox('auth', version = 0, 0)

{

utf8string schemeURI;

utf8string profile;

bit(1) content_id_flag;

bit(1) dependency_flag;

bit(1) association_flag;

bit(1) label_flag;

bit(1) signed_flag;

bit(3) reserved;

if (content_id_flag == 1)

unsigned int(8)[16] content_uuid;

if (depencency_flag == 1)

unsigned int(8)[16] dependent_uuid;

if (association_flag == 1)

unsigned int(8)[16] association_uuid;

if (label_flag == 1)

utf8string label;

if (signed_flag == 1){

unsigned int(8) signature_hash_method;

utf8string signature_keysourceURI;

unsigned int(10) signature_key_register_indx;

bit(6) reserved;

unsigned int(24) signature_length;

for (i=0; i<signature_length, i++)

unsigned int(4) signature[i];

}

// additional authentication metadata

}

#### Semantics

schemeURI declares the identifier of the authentication scheme.

profile is the profile of the declared scheme that the elementary scheme conforms to.

content_id_flag value 1 indicates inclusion of the content_id field.

dependency_flag value 1 indicates inclusion of the dependency_id field.

association_flag value 1 indicates inclusion of the association_id field.

label_flag value 1 indicates inclusion of the label field.

content_uuid specifies the content identifier UUID.

dependency_uuid specifies the content_uuid value of the authentication box that this box's authentication metadata is dependent on.

association_uuid specifies a UUID value of this box and all authentication boxes of the associated entities. The associated entities are the entities in which their authentication is expected to be jointly verified.

label specifies a label for this content. This value shall be unique in a file.

signature_hash_method indicates the hash algorithm used to calculate this box’s digest. The values are defined according to ISO/IEC 23002-7 Table 34.

signature_key_register_indx defines the index that specifies the certificate of the content provider for verifying the signature of this box, in the certificate register indicated by key_source_uri. The valid range is 0 to 1024, inclusive. The value 1024 means that the index is not defined.

signature_keysourceURI defines the key source URI for key retrieval for signature verification of this box.

signature_length defines the length of the signature.

signature includes the [i] element of the signature. The signature is calculated from the entire box’s digest except signature_length and signature fields using the hash algorithm defined in signature_hash_method.

Inclusion in ISOBMFF

One or more of the defined AuthBox can be included in several places:

- For the track level, inside trak

- For the fragments:

  - Inside the moov box for initialization

  - Inside each media fragment after the styp box

Characteristics

- The box can be used for signalling any authentication scheme.

- Multiple authentication methods can coexist for a file/track/fragment, each defined by one AuthBox.

- The Authbox can be used to indicate the dependency of the authentication metadata on other resources, including authentication metadata in other resources, for instance, another file, track or track fragment.

- The box can be used to indicate the associated files, tracks or fragments for authentication purposes, i.e. the authenticity of those resources needs to be verified together.

- Finally, the information in the box can optionally be signed and therefore verified by the player.

- In the case of using AuthBox, the ElementaryAuthBox (m75633) can be simplified by removing the following fields, since those fields are also defined in AuthBox:

  - schemeURI declares the identifier of the authentication scheme.

  - profile is the profile of the declared scheme that the elementary scheme conforms to.

  - content_id_flag value 1 indicates inclusion of the content_id field.

  - dependency_flag value 1 indicates inclusion of the dependency_id field.

  - association_flag value 1 indicates inclusion of the association_id field.

  - signed_flag value 1 indicates the inclusion of the signature.

  - content_uuid specifies the content identifier UUID.

  - dependency_uuid specifies the content_uuid value of the authentication box that this box's authentication metadata is dependent on.

  - association_uuid specifies a UUID value of this box and all authentication boxes of the associated entities. The associated entities are the entities in which their authentication is expected to be jointly verified.

  - signature_hash_method indicates the hash algorithm is used to calculate this box’s digest. The values are defined according to ISO/IEC 23002-7 Table 34.

  - signature_key_register_indx defines the index that specifies the certificate of the content provider for verifying the signature of this box, in the certificate register indicated by key_source_uri. The valid range is 0 to 1024, inclusive. The value 1024 means that the index is not defined.

  - signature_keysourceURI defines the key source URI for key retrieval for signature verification of this box.

  - signature_length defines the length of the signature.

  - signature includes the [i] element of the signature. The signature is calculated from the entire box’s digest except signature_length and signature fields using the hash algorithm defined in signature_hash_method.

Use of C2PA with AuthBox

The C2PA metadata can be included in one of the following ways in AuthBox:

- Least modification: C2PA in JUMBF box inside UUID box inside the AuthBox. In this case, the following fields are set to 0:

  - content_id_flag, dependency_flag, association_flag, label_flag, sign_flag

- Integrated at JUMBF: in this case, C2PA in a JUMBF box is included in the AuthBox and then:

  - The following flags are set to 0: dependency_flag, association_flag,label_flag, sign_flag

  - The content_id_flag flag is set to 1, and the content_uuid is set to the corresponding uuid values in C2PA UUID boxes.

- Integrated at C2PA: in this case C2PA is included in the AuthBox, and then:

  - The following flag is optionally set to 0: sign_flag

  - The content_id_flag flag is set to 1, and the content_uuid is set to the corresponding uuid values in C2PA UUID boxes.

  - The dependency_id is set to the corresponding value in the corresponding JUMBF box id field.

  - The label is set to the corresponding value in the JUMBF box.

Use of ElementaryAuthBox with AuthBox

If the Authbox is included in a file or track, a SimplifiedElementraryAuthBox can be included in the authbox, since some of the fields become redundant. Then the SimplifiedElementaryAuthbox is defined as:

aligned(8) class SimplifiedElementaryAuthBox extends FullBox('seau', version = 0, 0)

{

bit(1) signed_content_start;

bit(1) signed_content_end;

bit(1) hash_flag;

bit(1) key_retrieval_mode_flag;

bit(4) reserved;

if (hash_flag == 1)

unsigned int(8) hash_method;

if (key_retrieval_mode_flag == 1){

bit(1) key_retrieval_mode;

unsigned int(10) key_register_indx;

bit(5) reserved;

utf8string keysourceURI;

}

}

Applications

#### Track Level

In the case of file playback, the file parser can retrieve the information from the track before the playback of the file and see if the media player supports the authentication method/scheme. It can also be used for track selection, i.e. if there are multiple tracks with different elementary stream authentication, the player can choose the tracks whose authentication verification is supported by its media player. Further, the file format parser can see which tracks are dependent or associated with authentication and expose that information to the application. The value of a high-level authentication box is that the box is found quickly and parsed efficiently. Also, since more than one box is allowed, multiple authentication schemes can be signalled using the same box structure. Even dependent or associated authentication schemes can be signalled.

#### Fragment level

In the case of a fragment playback, the file parser can parse the initialisation segment to see the general characteristics of the track authentication, and then, when it parses a specific fragment, it can parse further information.  The dependent and association flags allow signalling if a fragment is dependent on another fragment or associated with other fragments (in the same track or other tracks). Also, since more than one box is allowed, multiple authentication schemes can be signalled using the same box structure. Even dependent or associated authentication schemes can be signalled.

Authentication group box

We also describe an alternative method of grouping the track to include the authentication metadata for them using the ISOBMFF entity to group box.

#### Entity to Group box for grouping

The current ISOBMFF definition of the entity to group box is:

aligned(8) class EntityToGroupBox(grouping_type, version, flags)

extends FullBox(grouping_type, version, flags)

{

unsigned int(32) group_id;

unsigned int(32) num_entities_in_group;

for (i=0; i<num_entities_in_group; i++) {

unsigned int(32) entity_id;

}

// the remaining data may be specified for a particular grouping_type

}

#### Authentication group box

##### Definition

| Box Type: | 'augb' |
| --- | --- |
| Container: | GroupsListBox in a MetaBox on the file level |
| Mandatory: | No |
| Quantity: | Zero or more |

A group of tracks that needs to be authenticated together is represented by an AuthenticationGroupBox. This box contains information on the authentication data for this group of tracks, along with one or more Auth Boxes.

##### Syntax

aligned(8) class AuthenticationGroupBox

extends EntityToGroupBox('augb', version=0, flags)

{

Box AuthBox[]; // Boxes describing the authentication

}

##### Semantics

AuthBox defines the authentication information for the entity group.

Media cross authentication signaling in ISOBMFF (m74440, m75550)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/34

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/42

Introduction

Media cross-authentication is essential to ensure that the relationships between tracks can be verified and trusted. As media experiences increasingly rely on synchronized combinations of heterogeneous elements (such as video, audio, subtitles, and auxiliary or immersive components) it becomes critical to authenticate not only each individual timed media or non-timed media but also their interdependencies. In the context of ISOBMFF, ensuring cross-media authentication is a requirement for maintaining consistent and reliable media authenticity.

Contribution m74440 [1], presented at the Geneva MPEG#152 meeting, addressed this need by proposing the use of EntityToGroupBox to carry media cross-authentication information in an ISOBMFF file. This contribution was accepted, and its core concepts were incorporated into the corresponding exploration document (MDS25605_WG03_N01631).

Furthermore, during the Ad-hoc meeting of December 5th (Media Auth Adhoc Dec 5 (#40)), the group issued the following recommendations:

“Recommendations:

- Start the technical activity for ISOBMFF support of media authentication (system level authentication, cross track authentication and signalling the existence of elementary stream authentication)

- Collaborate with C2PA and other relevant organizations on the technical progress.”

In alignment with these directions, the intention of the present contribution is to provide a detailed technical proposal on how EntityToGroupBox can be extended to ensure consistent media cross-authentication.

Media cross-authentication for ISOBMFF-based container

Within the ISO Base Media File Format (ISO/IEC 14496-12), the EntityToGroupBox provides a generic mechanism to describe sets of items and/or tracks together with their associated grouping parameters.

The EntityToGroupBox is defined as follows:

aligned(8) class EntityToGroupBox(grouping_type, version, flags)

extends FullBox(grouping_type, version, flags) {

unsigned int(32) group_id;

unsigned int(32) num_entities_in_group;

for (i = 0; i < num_entities_in_group; i++) {

unsigned int(32) entity_id;

}

// additional data may be specified depending on grouping_type

}

While the existing EntityToGroupBox provides a generic mechanism for grouping entities, it does not natively support the inclusion of data required for media cross-authentication.

This structure could be extended with a new grouping_type, a four-character code such as "gaut" to express media cross-authentication relationships in a consistent and interoperable manner.

Authentic Entity Group Box

#### Definition

| Box Type: | ‘gaut’ |
| --- | --- |
| Container: | GroupsListBox in a MetaBox on file level |
| Mandatory: | No |
| Quantity: | Zero or more |

A new box AuthenticEntityToGroupBox is proposed as an extension of the EntityToGroupBox. This new structure enriches the grouping mechanism with authentication-specific information.

Each entity is authenticated independently and contains information needed to proceed with the authentication of the entity.

The “entity authenticity part” in the AuthenticEntityToGroupBox references this information for each entity of the group.

Two modes are supported for conveying entity authentication information:

- External authenticity mode, in which a URI references an external container (e.g. a C2PA manifest) that provides authentication metadata

- Embedded authenticity mode, in which digest information is specified: a digest value, the algorithm used to compute it, and a list of boxes for that entity over which the digest value is computed. As examples of these boxes:

- The track (‘taut’) or sample (‘saut’) boxes as proposed in [2]).

- The ‘uuid’ boxes as specified in the C2PA specifications.

- The ‘seii’ box (SEI information box as specified in the MPEG 14496-15 specifications [3]), that lists the SEI messages that must be handled.

The “signature part” in the AuthenticEntityToGroupBox is intended to protect the integrity of the relationships between entities listed in the box. It supports two modes for conveying the box signature information:

- External signature mode, in which a URI references an external container providing signature information such as a C2PA manifest

- Embedded signature mode, in which the signature metadata and signature value are carried directly within the box.

#### Syntax

The new AuthenticEntityGroupBox box is defined as following:

aligned(8) class AuthenticEntityToGroupBox extends EntityToGroupBox(’gaut’, version, flags)

{

// Entity authenticity part

for (i=0; i<num_entities_in_group; i++)

{

bit(1) entity_authenticity_mode;

if (entity_authenticity_mode == 0)

{

utf8string entity_uri;

utf8string entity_auth_scheme;

}

else if (entity_authenticity_mode == 1)

{

utf8string       entity_digest_alg;

unsigned int(8)  entity_digest_length;

unsigned int(8)  entity_digest[ entity_digest_length ];

unsigned int(8)  num_authentic_boxes;

for (j=0; j<num_authentic_boxes; j++)

{

unsigned int(32) authentic_box_code;

}

}

}

// Signature part

bit(1)	signature_mode;

if (signature_mode == 0)

{

utf8string sign_uri;

}

else if (signature_mode == 1)

{

utf8string      sign_alg;

unsigned int(8) cert_value[];

utf8string      expiration_time;

unsigned int(8) sign_value[];

}

}

#### Semantics

Entity authenticity part:

A first part of the AuthenticEntityGroupBox is related to the authenticity information of each entity. The entity authentication information in the AuthenticEntityToGroupBox shall apply, in order, to the list of entity identifiers inherited from the EntityToGroupBox.

entity_authenticity_mode indicates the type of authenticity information for the entity. If set to 0, it means that the entity is authenticated by external means. Otherwise, it means that information for authenticity is provided inside the entity.

If entity_authenticity_mode is 0 (“external authenticity mode”), the entity_uri field references an external container (e.g. a C2PA manifest) that specifies the authenticity information. Optionally, an additional entity_auth_scheme string parameter specifies the authentication scheme of the external manifest.

If entity_authenticity_mode is 1 (“embedded authenticity mode”), the authenticity information is given in the AuthenticEntityGroupBox box and are related to ISOBMFF boxes:

- entity_digest_alg identifies the cryptographic algorithm used to generate the entity_digest, for example " SHA-256".

- entity_digest_length indicates the length of the digest contained in entity_digest.

- entity_digest is a cryptographic digest computed over the authenticated data associated with the entity identified by the corresponding entity identifier inherited from the EntityToGroupBox. The digest shall be computed by applying the cryptographic hash function identified by entity_digest_alg to the concatenation of the canonical ISOBMFF serialization of the boxes related to entity authentication listed in the box list block, in the order in which they appear in the file.

- num_authentic_boxes is a strictly positive number of boxes associated with the entity for the digest computation.

- authentic_box_code: FourCC (4-character code) identifying a box associated with the entity for authentication.

Signature part:

The signature part of the AuthenticEntityGroupBox is related to the integrity of the entire box.

signature_mode indicates the verification method. If set to 0, use the sign_uri to reference an external source of signature information. If set to 1, the signature information block should be used.

sign_uri identifies reference to external container that provides authentication signature information, like a C2PA manifest. This external container shall contain authenticity information for the AuthenticEntityGroupBox.

sign_alg identifies the cryptographic algorithm used to generate the signature, for example "ES256" (Elliptic Curve Digital Signature Algorithm with SHA-256) or “RSA_SHA1” (signature algorithm).

cert_value[] contains the certificate data, e.g. the content of an X.509 certificate that contains the public key used to verify the signature.

expiration_time indicates when the signature is expired, expressed in a precise timestamp format (e.g., "2023-02-12T18:40:19+00:00").

sign_value[] carries the digital signature computed over the serialized content of the AuthenticEntityToGroupBox excluding the sign_value field itself, with all remaining fields included in their canonical ISOBMFF serialization order. The length of sign_value is not explicitly signalled, as it can be inferred from the signature algorithm specified in sign_alg.

#### Processing steps

To perform the media cross-authentication in an ISOBMFF file, a verifier should parse the file and verify the authenticity of each AuthenticEntityToGroupBox. This verification can be performed entirely before the playing of the media or progressively during the reception of the data.

For each AuthenticEntityToGroupBox, the verifier first validates the signature protecting the box in order to establish its integrity.

- If sign_uri is referenced in the signature part of the ADU, the verification must follow the specifications of the external container.

- If no sign_uri is present, the verifier first checks the sign_alg field to determine which cryptographic method should be applied (e.g., ES256). The public key is retrieved from the cert_value field. Next, using the public key with the algorithm indicated by sign_alg, the verifier recomputes the signature of the AuthenticEntityToGroupBox, excluding the sign_value[] and compares it against the sign_value[]. Additionally, the expiration_time must be checked versus the current time to confirm that the signature has not expired.

When the signature check is satisfied, the verifier can check the authenticity information of each entity, thus ensuring trust in the combination of entities:

- If the entity_authenticity_mode field is 0, it indicates the presence of an external manifest, the verifier checks the authenticity information according to this manifest and if present, the value of entity_auth_scheme.

- If the entity_authenticity_mode field is 1, it indicates the presence of internal authenticity information, a digest value is verified by using the entity_digest_alg, over the data identified by the authentic boxes or the authentic ranges specified for the entity.

- If the check fails for one entity, the verifier can choose to stop the process and to warn the user that the media cross-authentication failed. Or the verifier can  choose to continue the check of the other entities.

Finally, the verifier can check the authenticity of each entity referenced in the AuthenticEntityToGroupBox according to their own security attributes.

If desired, the verifier can only check the entities that will be played. For instance, the AuthenticEntityToGroupBox can group a video entity and multiple audio entities corresponding to different languages (and optional multiple subtitle entities). Since only one audio entity (and optionally one subtitle entity) is selected for playback, the verifier can restrict its verification to those selected entities, while the remaining entities in the group do not require verification.

The result of the media cross-authentication check can be indicated (a sign or a text on a display, a sound…) and if the check failed, the player could decide to stop or continue the playback.

Signaling in MPEG-2  TS (m74512)

https://git.mpeg.expert/MPEG/Systems/explorations/-/issues/36

M74512 proposes a dedicated descriptor to signal information regarding authenticity for a program and its elementary streams within the Program Specific Information (PSI) that is part of a Transport Stream according to ISO/IEC 13818-1 MPEG-2 systems. It suggests specifying a descriptor including fields that support the joint evaluation of authenticity of different elementary streams, i.e. the detection if an authenticated elementary stream was changed or if there is a mismatch between different elementary streams.

# Discussions at MPEG152

- There are existing tools that can be used, such as the signaling SEI prefixes.

- Specific mechanism may not be an issue, but finding the information in the system layer is important.

- Understanding the granularity and sequencing of data operation in this context would provide more insights for the design goals.

# Recommendations

The following steps are recommended:

- Investigate if each of the following standards requires a generic solution for carriage and signaling of the media authentication and provenance metadata

  - ISOBMFF/CMAF

  - DASH

  - MPEG 2TS

- Prepare a half day brainstorming session at the next MPEG Systems meeting and invite external experts if needed

- Collaborate with the relevant organizations such as C2PA, EBU and SVTA/DASH-IF on the media authentication

# References

- UNESCO: https://en.unesco.org/fightfakenews

- ISO/IEC JTC 1/SC 29/WG1, “Use cases and requirements for JPEG Trust”, N100886, Sapporo, 13-19 July 2024.

- C2PA Technical Specification, https://c2pa.org/specifications/specifications/2.1/specs/C2PA_Specification.html

- JPEG Trust, https://jpeg.org/jpegtrust/documentation.html