INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG03 MPEG2025/N25895

January 2026, Online

| Source | Systems |
| --- | --- |
| Status | Working Draft |
| Title | WD of ISO/IEC TR 23009-7 Delivery of CMAF content with DASH |
| Editor | Thomas Stockhammer, Stephan Schreiner |

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	iv

1	Scope	1

2	Normative references	1

3	Definitions	1

3.1	Terms and definitions	1

Refer to definitions in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions.	1

3.2	Abbreviated terms	1

4	Document Organization	1

5	Overview	2

6	Background	2

6.1	Key Concepts in DASH	2

6.2	Key Concepts in CMAF	4

7	DASH Profiles for CMAF Content	6

7.1	General	6

7.2	Open Issues	6

8	DASH Media Presentation Generation from a CMAF Content	6

8.1	General	6

8.2	Preselection Considerations	6

8.3	DASH Packager Guidance using ISOBMFF KindBox	7

8.3.1	General	7

8.3.2	DASH Descriptors and other child elements	7

8.3.3	DASH Attributes	8

- Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

ISO/IEC 23009-1 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

ISO/IEC 23009 consists of the following parts, under the general title Information technology — Dynamic adaptive streaming over HTTP (DASH):

- Part 1: Media presentation description and segment formats

- 	Part 2: Conformance and reference software

- 	Part 3: Implementation guidelines [Technical Report]

- 	Part 4: Segment encryption and authentication

- 	Part 5: Server and network assisted DASH

- 	Part 6: DASH with server push and websocket sub protocol

- 	Part 7: Delivery of CMAF content with DASH

Error: Reference source not found

# Scope

This part of Error: Reference source not found recommends best practises for delivery of CMAF content (ISO/IEC 23000-19) with the DASH standard.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format

ISO/IEC 23000-19, Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media

ISO/IEC 23009-1, Information technology — Dynamic adaptive streaming over HTTP-Part: Media presentation description and segment formats

# Definitions

## Terms and definitions

## Refer to definitions in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions.

## Abbreviated terms

For the purposes of this International Standard, the following abbreviated terms apply.

CMAF	Common Media Application Format (as specified by ISO/IEC 23000-19)

DRM	Digital Rights Management

ISOBMFF	ISO Base Media File Format (as specified by ISO/IEC 14496-12)

Refer to abbreviated terms in ISO/IEC 23000-19 and ISO/IEC 23009-1 for definitions in addition to the above.

# Document Organization

TBD

# Overview

This report defines guidelines for delivering content generated based on the CMAF as defined in ISO/IEC 23000-19 using DASH as defined in ISO/IEC 23009-1.

The CMAF specification defines an encoding format for the content. Each media component of the content may be encoded in multiple tracks, grouped in one or more CMAF Switching Sets. However, there is no description of how these tracks are related, and how various media components should be delivered and played.

The DASH specification defines segment formats for media content. But it also defines a manifest, Media Presentation Description (MPD) which expresses the relationship of tracks and segments as well as how they are identified as URI resources.

While CMAF delivery entities can be identical to DASH segments, there are multiple ways to package them and/or identified them as resources and described by a MPD. These guidelines recommend some of the most popular delivery schemes for such mapping and delivery.

# Background

## Key Concepts in DASH

DASH defines a manifest format using an XML schema and semantics, as well as constraints on DASH segments that can be addressed for delivery, and synchronized for presentation.

The collection of encoded and deliverable versions of media content and the appropriate description of these form a Media Presentation. Media content is composed of a single or multiple contiguous media content periods in time. Content in different media content periods may be completely independent or certain periods of a Media Presentation may belong to the same Asset, for example a Media Presentation is a collection of main program composed of multiple periods, each assigned to the same Asset, and interleaved with inserted advertisement periods. Each media content period is composed of one or multiple media content components, for example audio components in various languages, different video components providing different views of the same program, subtitles in different language and others. Each media content component has an assigned media content component type, for example audio or video. Same asset over multiple periods may be identified by a DASH descriptor enabling DASH clients to maintain the continuity across periods’ boundaries. Furthermore, sub-assets composing the same asset may also identified using a similar method. For instance, if an asset is composed of multiple video components, sub-assets enable selecting the previously selected video component after an ad insertion.

Each media content component may have several encoded versions, referred to as media streams. Each media stream inherits the properties of the media content, the media content period, the media content component from which it was encoded and in addition it gets assigned the properties of the encoding process such as sub-sampling, codec parameters, encoding bitrate, etc. This describing metadata is relevant for static and dynamic selection of media content components and media streams.

Figure 3 — DASH High-Level Data Model

DASH is based on a hierarchical data model aligned with the presentation in Figure 3. A DASH Media Presentation is described by a Media Presentation Description document. This describes the sequence of Periods (see 5.3.2 of ISO/IEC 23009-1) in time that make up the Media Presentation. A Period typically represents a media content period during which a consistent set of encoded versions of the media content is available i.e. the set of available bitrates, languages, captions, subtitles etc. does not change during a Period.

Within a Period, material is arranged into Adaptation Sets (see 5.3.3 of ISO/IEC 23009-1). An Adaptation Set represents a set of interchangeable encoded versions of one or several media content components (see 5.3.4 of ISO/IEC 23009-1). For example, there may be one Adaptation Set for the main video component and a separate one for the main audio component. If there is other material available, for example captions or audio descriptions, then these may each have a separate Adaptation Set. Material may also be provided in multiplexed form, in which case interchangeable versions of the multiplex may be described as a single Adaptation Set, for example an Adaptation Set containing both the main audio and main video for a Period. Each of the multiplexed components may be described individually by a media content component description.

An Adaptation Set contains a set of Representations (see 5.3.5 of ISO/IEC 23009-1). A Representation describes a deliverable encoded version of one or several media content components. A Representation includes one or more media streams (one for each media content component in the multiplex). Any single Representation within an Adaptation Set is sufficient to render the contained media content components. By collecting different Representations in one Adaptation Set, the Media Presentation author expresses that the Representations represent perceptually equivalent content. Typically, this means that clients may switch dynamically from Representation to Representation within an Adaptation Set in order to adapt to network conditions or other factors. Switching refers to the presentation of decoded data up to a certain time t, and presentation of decoded data of another Representation from time t onwards. If Representations are included in one Adaptation Set, and the client switches properly, the Media Presentation is expected to be perceived seamless across the switch. Clients may ignore Representations that rely on codecs or other rendering technologies they do not support or that are otherwise unsuitable.

Within a Representation, the content may be divided in time into Segments (see 5.3.9 and 6 of ISO/IEC 23009-1) for proper accessibility and delivery. In order to access a Segment, a URL is provided for each Segment. Consequently, a Segment is the largest unit of data that can be retrieved with a single HTTP request.

Segments may be further subdivided into Subsegments each of which contains a whole number of complete access units. There may also be media-format-specific restrictions on Subsegment boundaries, for example in the ISO Base Media File Format a Subsegment must contain a whole number of complete movie fragments. If a Segment is divided into Subsegments they are described by a compact Segment index, which provides the presentation time range in the Representation and corresponding byte range in the Segment occupied by each Subsegment. Clients may download this index in advance and then issue requests for individual Subsegments.

Clients may switch from Representation to Representation within an Adaptation Set at any point in the media. However, switching at arbitrary positions may be complex because of coding dependencies within Representations and other factors, potentially requiring parallel download and decoding in the DASH client. It is also desirable to avoid download of 'overlapping' data i.e. media for the same time period from multiple Representations. Usually, switching is simplest at a Stream Access Point (SAP) in the new stream. In order to formalize requirements related to switching DASH defines a codec-independent concept of Stream Access Points and identifies various types of Stream Access Points.

Segmentation and Subsegmentation may be performed in ways that make switching simpler. For example, in the very simplest cases each Segment or Subsegment begins with a SAP and the boundaries of Segments or Subsegments are aligned across the Representations of one Adaptation Set. In this case, switching Representation involves playing to the end of a (Sub)Segment of one Representation and then playing from the beginning of the next (Sub)Segment of the new Representation. The Media Presentation Description and Segment Index provide various indications, which describe properties of the Representations that may make switching simpler. Profiles of this specification may then require these indicators to be set in certain ways, making implementation of clients for those profiles simpler at the cost of requiring the media data to obey the indicated constraints.

## Key Concepts in CMAF

CMAF provides the following concepts:

- CMAF specifies CMAF Tracks containing CMAF Fragments conforming to Media Profiles that can be combined to form CMAF Presentations.  CMAF Tracks can exist as live streams as well as stored files.

- CMAF specifies CMAF Presentations that are encoded conforming to a hypothetical late binding application model that can select multiple independent CMAF Tracks and simultaneously decode and synchronize CMAF Fragments to result in a multimedia presentation.

- CMAF specifies CMAF Selection Set encoding constraints for synchronized alternative content that can be selected based on device compatibility and user preference, e.g. alternative languages or codecs.

- CMAF specifies CMAF Switching Set encoding constraints for alternative encodings of the same content at different bitrates. CMAF Tracks in a CMAF Switching Set can be dynamically switched and seamlessly spliced by sequencing CMAF Fragments from different CMAF Tracks through a single decoder.

- CMAF Tracks in CMAF Aligned Switching Sets are alternative encodings of the same content with time-aligned CMAF Fragments, but the CMAF Tracks do not conform to the same CMAF Switching Set constraints, e.g. they may be encoded with different codecs, encryption, etc. Track Switching between CMAF Synchronized Switching Sets might be seamless on some Players, but not all Players.

- CMAF specifies CMAF Addressable Media Objects for storage and delivery and identification as Resources by external specifications, such as adaptive streaming protocols.  CMAF Addressable Media Objects are CMAF Headers, CMAF Track Files, CMAF Segments, and CMAF Chunks.

- CMAF specifies CMAF Media Profiles and file brands for widely used audio, video, and subtitle formats. Each CMAF Media Profile specifies constraints on codecs, samples, CMAF Fragments, and CMAF Tracks to identify encoder/decoder interoperability, and optional functionality including random access and seamless adaptive switching specified as CMAF Switching Set constraints. CMAF specifies general requirements and registration for CMAF Media Profiles and brands to enable other specifications to define CMAF Media Profiles that conform to the general CMAF requirements.

- CMAF specifies a CMAF Presentation Profile that conditionally requires CMAF Tracks and Media Profiles in conforming CMAF Presentations to enable most media playback devices to render a CMAF Presentation.  Additional CMAF Tracks and CMAF Media Profiles can be optionally included in a CMAF Presentation conforming to a Presentation Profile.  Other CMAF Presentation Profiles can be defined by other specifications that define different conditional requirements on CMAF Tracks and CMAF Media Profiles intended for specific applications.

Figure 1 CMAF data object model

# DASH Profiles for CMAF Content

## General

ISO/IEC 23001-9 defines profiles to carry CMAF content in DASH Media Presentations. The core aspects of the mapping includes mapping of

- Timing Model

- CMAF Structures to DASH MPDs

- Mapping of CMAF Addressable Objects to DASH Segments

- Mapping of Media, Encryption and CMAF Header Information to MPD

The profile is based on the 3rd edition of ISO/IEC 23000-19 and also assumes ISO/IEC 14496-12 based on 'iso9'.

## Open Issues

The profile does currently not define any aspects related to generate Preselections, for example for multi-codec tracks.

# DASH Media Presentation Generation from a CMAF Content

## General

This clause addresses the case that CMAF content is available, and a packager is expected to generate a conforming DASH MPD.

In general, a DASH Packager is recommended to follow the instructions for the DASH profile for CMAF content as defined in ISO/IEC 23009-1. This mapping is considered sufficient, but is not necessary.

In addition to the mapping, this clause includes additional considerations for generating an MPD.

## Preselection Considerations

If a CMAF Principal Header contains PreselectionGroupBoxes, each instance thereof is assigned to one Preselection element with the following guidelines to be considered:

- The value of the group_id is assigned to the Preselection@id attribute. (optional)

- The tracks referenced through the entity_id in the PreselectionGroupBox are assigned to the Preselection@preselctionComponents attribute with each value of an entity_id being represented by the corresponding AdaptationSet@id value. The order of the entries in the @preselectionComponents attribute is exactly the same as in the loop of entity_ids.

- The value of the preselection_tag, if present, is assigned to the Preselection@tag attribute. Otherwise, this attribute is absent.

- The value of the preselection_priority, if present, is assigned to the Preselection@selectionPriority attribute. Otherwise, this attribute may be absent.

- The value of the interleaving_tag, if present, is used to determine the Preselection@order attribute. If the interleaving_tag, the @order is either set to "undefined" or is absent.

- If additional boxes are present in a PreselectionGroupBox, the following guidelines should be followed:

  - An ExtendedLanguage box is assigned to the Preselection@lang attribute

  - KindBoxes are converted to Role and Accessibility descriptors or other attributes and elements as specified in clause 8.3.

  - Any ChannelLayoutBox or AudioRenderingIndicationBox is converted to an AudioChannelConfiguration descriptor.

- All DASH AdaptationSets comprising any Preselection optionally get a SupplementalProperty descriptor assigned. The @schemeIdUri of this descriptor is "urn:mpeg:dash:preselection:2016”, with the value being absent. If the AdaptationSet is never referenced as the main AdaptationSet (i.e. is never the first entry in the list of AdaptationSet@id values in the Preselection@preselectionComponents list), an EssentialProperty descriptor is used instead.

## DASH Packager Guidance using ISOBMFF KindBox

### General

This clause defines schema for use with the ISOBMFF KindBox for signalling DASH child elements and attributes. Such information provides guidance to DASH packagers when generating the DASH manifest based on ISOBMFF source files.

The presence of a KindBox with one of the identifiers defined by this clause in the schemeURI field indicates to a DASH packager to add respective information into the DASH manifest as defined for that scheme.

When information is targeting Representation elements, the KindBox is present in the MediaBox of the respective track, for AdaptationSet elements in the SwitchingGroupBox, for Preselections in the PreselectionGroupBox and for Periods in the MovieBox.

Since a KindBox with its schemeURI set to one of the identifiers defined by this clause is intended to guide DASH packagers, players are expected to ignore such a box. Furthermore, such a KindBox should get removed when a creating DASH segments. KindBoxes containing other, possibly unknown settings in their schemeUri field do not get removed from segment unless specified differently for that scheme.

NOTE: Other applications, such as storage formats according to 23009-9 (DASH REaP) define different schema used with the schemeURI field for similar purposes.

If the ISOBMFF specification or any other standard defines dedicated means for signaling properties other than utilizing the KindBox as documented in this subclause, content creators should prefer such signaling over the use of the KindBox. An example therefore is the ExtendedLanguageBox to indicate the media’s language. The content of this box can get translated into the @lang attribute.

On the other hand, when both types of signaling are present in an incoming ISOBMFF file, a DASH packager should prioritize the KindBox with a scheme from this clause since this indicates that the content provider wants the DASH manifest to intentionally deviate from the signaling based on those elements.

### DASH Descriptors and other child elements

#### Definition

An ISOBMFF KindBox may get used to indicate to the DASH packager to add additional XML child elements to the parent element. Such child elements may possible be, but are not limited to elements derived from the DescriptorType class.

If used for this purpose, the KindBox carries the following information:

- The schemeURI field is set to “urn:mpeg:dash:descriptor:2025” (without the DQUOTE characters).

- The value field carries the XML snippet of the intended child element.

The syntax for the value field is formatted according to the subsequent ABNF notation according to IEFT RFC 5234 for the KINDBOX-XMLCHILD-VALUE:

KINDBOX-XMLCHILD-VALUE = XML-SNIPPED XML-SNIPPED = *VCHAR ; XML snippet as defined in 23009-1

NOTE: This ABNF definition does not contain the mandatory null byte termination required by the utf8string field.

#### Examples

The following XML snippets show use cases where the DescriptorType definition got amended compared to the base definition.

- Descriptor extended by child element:

<SupplementalProperty schemeIdUri="urn:mpeg:dash:supv:2022"> <SupVideoInfo processingInfo="1 3 5"/> </SupplementalProperty>

- Descriptor extended by additional attributes:

<SupplementalProperty schemeIdUri="urn:dvb:dash:fontdownload:2014" value="1" dvb:url="https://example.com/fonts/SubtitleDisplay.woff" dvb:fontFamily="SubtitleDisplay" dvb:mimeType="application/font-woff"/>

Source: https://dashif.org/dash.js/pages/advanced/subtitles-and-captions/dvb-font-downloading.html

### DASH Attributes

#### Definition

An ISOBMFF KindBox may get used to indicate to the DASH packager to add additional attributes to the parent element.

- If used for this purpose, the KindBox carries the following information:

- The schemeURI field is set to “urn:mpeg:dash:attribute:2025” (without the DQUOTE characters).

- The content of the value field in the KindBox provides a whitespace separated list of attribute names, optionally including their assigned values.

The syntax for the value field is formatted according to the subsequent ABNF notation according to IEFT RFC 5234 for the KINDBOX-ATT-VALUE:

KINDBOX-ATT-VALUE = nameValuePair *( SP nameValuePair) nameValuePair = AttName [ "=" DQUOTE ValueString DQUOTE] AttName = 1*VCHAR ; name of XML attribute ValueString = 1*VCHAR ; as defined for the ; respective attribute

NOTE: This ABNF definition does not contain the mandatory null byte termination required by the utf8string field.

#### Example

The following example shows how a selectionPriority attribute, intended to get assigned to a Preselection element, gets represented in ISOBMFF:

Since the ‘prsl’ grouping_type relates to the DASH Preselection element, this KindBox is expected to yield the following XML snippet:

<Preselection selectionPriority="5"/>