ISO/IEC AWI 14496-34(en)

Second edition

Date: 2026-02-04

Information technology — Coding of audio-visual objects —

Part 34:
Syntactic description language

Technologies de l'information — Codage des objets audiovisuels —

Partie 34: Langage de description syntaxique

© ISO 2026

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: + 41 22 749 01 11

E-mail: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	v

Introduction	vi

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

4	Notation	3

4.1	General	3

4.2	Rule formatting	3

4.3	Rule tokens	4

4.4	Bitstream examples	4

5	Language concepts and lexical elements	5

5.1	Character set	5

5.2	Whitespace	5

5.3	Case sensitivity	6

5.4	Comments	6

5.5	Identifiers	6

5.6	Punctuators	6

5.7	Keywords	7

5.8	Operators	8

5.9	Expressions and evaluation	10

5.10	Statements	11

5.11	Built-in operators	11

5.12	Numbers	14

5.13	Logic values	15

5.14	Binary literal values	15

5.15	Hexadecimal literal values	16

5.16	Multiple character literal values	16

5.17	Integer, decimal and floating-point literal values	17

5.18	String literal values	18

5.19	Scope	19

5.20	SDL specification limits	21

6	Elementary data types	22

6.1	General	22

6.2	Constant-length direct representation bit fields	22

6.3	Variable length direct representation bit fields	26

6.4	Constant-length indirect representation bit fields	27

6.5	Variable length indirect representation bit fields	30

6.6	Variable length strings	31

7	Composite data types	32

7.1	Classes	32

7.2	Base and derived classes	33

7.3	Abstract classes	34

7.4	Polymorphism in class declaration	35

7.5	Expandable classes	38

7.6	Parameter types	39

7.7	Arrays	44

8	Computed variables	50

8.1	General	50

8.2	Elementary data types	50

8.3	Arrays	51

8.4	Multi-dimensional arrays	52

9	Syntactic flow control	53

9.1	General	53

9.2	Conditionals	53

9.3	Loops	57

Annex A (informative)  SDL user guide	61

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives or www.iec.ch/members_experts/refdocs).

ISO and IEC draw attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO and IEC take no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO and IEC had not received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents and https://patents.iec.ch. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see www.iso.org/iso/foreword.html. In the IEC, see www.iec.ch/understanding-standards.

This document was prepared by Joint Technical Committee JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO 14496 series can be found on the ISO and IEC websites.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html and www.iec.ch/national-committees.

The second edition comprises the following changes:

TODO

Introduction

This document describes the mechanism with which bitstream syntax is documented in several standard parts such as in ISO/IEC 14496 or ISO/IEC 23000. This mechanism is called the Syntactic Description Language (SDL) and is documented here in the form of syntactic and semantic rules.

The SDL builds on concepts defined in the C-like syntax used in ISO/IEC 11172-1 and ISO/IEC 13818-1 to define an extensible framework for describing bitstreams. This framework is inspired by class typing system concepts from the C++ and Java programming languages. SDL specialises this class typing system concept by providing facilities for defining bitstream-level quantities, and how they should be parsed.

The SDL syntax was initially defined in an early edition of ISO/IEC 14496-1 and this document is largely based on that original syntax. Now elevated to a standalone part, the SDL syntax specification is modified and enhanced to remove ambiguity and minimize undefined behavior.

In scenarios where the usage or interpretation of the SDL are ambiguous or undefined, this document attempts to specify whether such a scenario is considered an invalid SDL specification or will result in undefined behaviour.

Information technology — Coding of audio-visual objects —

Part 34:
Syntactic description language

# Scope

This document specifies a Syntactic Description Language (SDL) for describing the structure of binary data. It covers the representation of an SDL specification in plain text, the syntax of the SDL and the semantic rules of the SDL. The storage of an SDL specification in a text file is not specified in this document. A computer program implementation or a standard containing an SDL specification may choose to specify such details.

Lexical elements of the SDL are described first, followed by elementary and composite type constructs to specify bitstreams. Finally, support for general purpose computation and syntactic control flow are addressed. Example SDL specification fragments and corresponding bitstreams are provided to clarify various concepts.

NOTE	While the SDL borrows from and contains some aspects of a general-purpose programming language, it is not intended, nor is it suitable, to be used for such a purpose. This is reflected in the fact that many concepts related to general-purpose programming languages are not addressed in this document. Examples of concepts considered irrelevant to the SDL and therefore not addressed in this document include compilation, execution, input/output, execution environment and machine architecture.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

IEEE Std 754-2019, IEEE Standard for Floating-Point Arithmetic

IETF RFC 4648, The Base16, Base32, and Base64 Data Encodings

ISO/IEC 10646, Information technology — Universal coded character set (UCS)

# Terms and definitions

For the purposes of this document, the following terms, abbreviations and definitions apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

ISO Online browsing platform: available at https://www.iso.org/obp

IEC Electropedia: available at https://www.electropedia.org/

fixed length code

FLC

constant-length direct representation bit fields

syntactic description language

SDL

language defined by this document that allows the description of a bitstream’s syntax

composite type

variable type which consists of an aggregated structure of one or more elements

EXAMPLE	Arrays, classes and maps.

computed variable

variable whose value is only stored in the parsing context (3.12)

declaration

sequential unit of tokens used to define the ‘shape’ of a variable value without causing data to be consumed from a bitstream or stored in the parsing context (3.12)

EXAMPLE	Class and map declarations.

definition

sequential unit of tokens used to create a parsed variable (3.11) or computed variable (3.4) and its value, potentially causing data to be consumed from a bitstream or stored in the parsing context (3.12)

EXAMPLE	A parsed class variable definition or a computed integer variable definition.

elementary type

individual numeric value types

EXAMPLE	Bits, integers and floats.

invalid SDL specification

SDL specification (3.13) that is syntactically valid but is nonetheless considered invalid due to either semantic rules or the fact that such an SDL specification (3.13) would be impossible to interpret or process

literal

lexical representation of a value in SDL syntax

EXAMPLE	"hello" or 1.2

literal value

value (3.17) of a literal (3.9) stored in the parsing context (3.12)

EXAMPLE	int i = 2; The text 2 is an integer literal and the value stored in the parsing context for i is the literal value 2.

parsed variable

variable whose value (3.17) is initially stored in a bitstream and once parsed is also stored in the parsing context (3.12)

parsing context

abstract concept of a volatile storage area for variable values

Note 1 to entry: A computer program implementation would use computer memory for this and may choose to impose constraints such as storage size.

SDL specification

formal description of a binary bitstream using the syntactic description language (3.2)

universal coded character set

UCS

standard set of characters defined by ISO/IEC 10646

undefined behaviour

behaviour resulting in an undefined outcome based on the interpretation or processing of an SDL specification (3.13)

Note 1 to entry: Certain scenarios in this document are identified as resulting in undefined behaviour.

Note 2 to entry: A computer program implementation or a standard presenting an SDL specification may choose to define expected behaviour in these scenarios.

unspecified constraint

constraint or limit on an SDL specification (3.13) which would need to be defined for it to be viably processed in some manner by a computer program

Note 1 to entry: As the scope of the SDL does not cover such processing, where appropriate such potential constraints are identified and explicitly left unspecified.

Note 2 to entry: A computer program implementation or a standard containing an SDL specification may choose to specify such constraints.

value

data stored in the parsing context (3.12) which may originate from a literal (3.9) or an evaluated expression

# Notation

## General

The body of this document outlines the concepts, lexical elements, syntax and semantics. Informal grammar rules are used throughout to introduce the syntax.

## Rule formatting

The following formatting is used when presenting the informal grammar rules.

Keywords, punctuators and operators are formatted using a bold font, e.g. keyword.

Constructs referenced across informal grammar rules are formatted using a bold, italic font, e.g. construct.

Rule tokens used to express the informal grammar rules (defined in 4.3) are formatted using a non-italic, non-bold font, e.g. ‘[’ and ‘]’.

Identifiers and values are formatted using an italic font, e.g. identifier.

## Rule tokens

The following tokens are used in the informal grammar rules.

An optional element is indicated by surrounding it with ‘[’ and ‘]’ e.g. [optional_element].

Alternative elements are indicated by separating each alternative by ‘|’ e.g. element_1| element_2.

An unspecified sequence of one or more elements is indicated by ‘…’ e.g. element;… .

## Bitstream examples

This document provides encoded bitstream examples demonstrating the behaviour of various SDL constructs. The bitstream examples are to be read left to right, top to bottom. Line breaks are irrelevant and are used purely to allow the annotation of the bitstream. Annotations appear after a comment marker ‘//’. For example, the following indicates the 32-bit value 0xFF11 encoded in a binary bitstream:

EXAMPLE 1

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 // first two bytes
// blank lines are irrelevant
 
0 0 0 0 0 0 0 1    // third byte
 
0 0 0 0    // another 4 bits
0 0 0 1    // last 4 bits

Examples always have a starting parse position. If parsing has just started, the example's starting parse position will be before the first bit in the bitstream. If parsing has already commenced, the example’s starting parse position will be after the last parsed bit. In both cases the example's starting parse position is before the first bit that is relevant to the example. To emphasize the fact that the example may be part of a larger bitstream, examples will always be preceded and succeeded by ‘...’. As an example:

EXAMPLE 2

...        // preceding bits not relevant to example
0 0 0 0    // first 4 bits
0 0 0 1    // last 4 bits
...        // succeeding bits not relevant to example

Where skipping an indeterminate number of bits to achieve alignment is part of the example it is indicated as follows:

EXAMPLE 3

...
<skip 0..7 bits>  // aligned(8)
0 0 0 0           // first 4 bits
0 0 0 1           // last 4 bits
...

# Language concepts and lexical elements

## Character set

The basic character set used to write an SDL specification consists of:

The 26 uppercase letters of the Latin alphabet:

A B C D E F G H I J K L M N O P Q R S T U V W X Y Z

The 26 lowercase letters of the Latin alphabet:

a b c d e f g h i j k l m n o p q r s t u v w x y z

The 10 decimal digits:

0 1 2 3 4 5 6 7 8 9

The following 33 graphic characters:

! " # % & ' ( ) * + , - . / : ; < = > ? [ \ ] ^ _ { | } ~ $ @ ` <space>

where <space> represents the character with UCS code point U+0020.

In certain string literal values and comments, an SDL specification may contain UCS characters (as defined in ISO/IEC 10646) with the exception of the ZERO WIDTH NO-BREAK SPACE character (U+FEFF), non-printable characters (U+0000 to U+001F inclusive) and characters causing line feeds (U+0085, U+2028, U+2029). To represent these excluded UCS characters, a code point escape sequence may be used (discussed in 5.18). As an example, a string literal consisting of the ZERO WIDTH NO-BREAK SPACE character would be represented as "\uFEFF".

NOTE 1	UCS string literal values are defined in 5.18.

NOTE 2	Comments are discussed in 5.4.

In multiple character literal values, an SDL specification may contain characters from the range U+0020 to U+007E, inclusive, of ISO/IEC 10646.

NOTE 3	Multiple character literal values are discussed in 5.16.

## Whitespace

One or more space, tab or carriage return characters constitute a whitespace and serve to demarcate tokens in an SDL specification. Whitespace is required wherever parsing of tokens in an SDL specification would be ambiguous. For example, the following are all valid and equivalent:

EXAMPLE 1

int ( 10 ) i = -1 .. 3;
int(10)i=-1..3;
int( 10) i= - 1 ..3;

For example, the following is invalid as tokenization of unsigned int is not possible:

EXAMPLE 2

unsignedint i; // missing whitespace causing token ambiguity

Indentation using whitespace is used only for assisting in the readability of an SDL specification and has no effect on its syntactic or semantic meaning.

## Case sensitivity

The SDL is case sensitive.

## Comments

Comments start by // and continue to the end of the current line. A comment may be preceded by any SDL syntax between the start of the line and the start of the comment. Comment text may contain SDL compatible UCS characters (as defined in 5.1). If UCS characters outside of the SDL compatible UCS characters are required, a UCS code point escape sequence may be used (discussed in 5.18). When parsing tokens in an SDL specification, the comment start characters // and the remainder of the line are ignored.

Rule S.1: Comments

// This is a comment.

## Identifiers

Identifiers may be comprised of a mixture of upper- and lower-case Latin alphabetic characters (as defined in 5.1), digits (as defined in 5.1) and underscore (e.g. ‘_’). An identifier shall include at least one alphabetic character. For example, the following are all valid identifiers:

EXAMPLE 1

myVar
My_Var
My2ndVar
2D_Region[2]
_2d_region

An identifier shall not match in a case-sensitive way any SDL syntax items such as keywords, binary, hexadecimal and string literal prefixes. For example, the following identifiers are all invalid:

EXAMPLE 2

u   // conflict with UCS string literal prefix
0b  // conflict with binary literal value prefix
MyClass map // conflict with keyword "map"
1e2 // conflict with floating point literal value
2_2 // identifier shall contain at least one alphabetic character

An identifier should not match in a case-insensitive way any SDL syntax items such as keywords, binary, hexadecimal and string literal. For example:

Break // valid but not recommended identifier as it may be confused with the keyword break

NOTE	It is possible that some automated tools will generate source code written in a diverse range of programming languages based on SDL specifications. When defining identifiers in an SDL, it is therefore useful to avoid defining identifiers conflicting with specific rules of certain programming languages, e.g. identifiers starting with digits or underscores as these identifiers can be illegal or have specific meaning in some programming languages, or identifiers conflicting with certain programming language keywords, e.g. public.

## Punctuators

The following punctuator tokens are defined in the SDL:

(   open parenthesis

)   close parenthesis

{   open brace

}   close brace

[   open bracket

]   close bracket

:   colon

;   semicolon

,   comma

"   double quote

'   single quote

## Keywords

The following keyword tokens are defined in the SDL:

abstract

aligned

base64string

bit

break

case

class

computed

const

default

do

else

expandable

extends

float

for

if

int

legacy

lengthof

map

reserved

switch

unsigned utf16string

utf8string

utf8list

utfstring

while

## Operators

### List of operators

The following arithmetic, logical and language specific operators are defined in the SDL:

| . | class member access, e.g. a.b |
| --- | --- |
| [] | array element access, e.g. a[b] |
| lengthof() | parsed bit length, e.g. lengthof(a) |
| ++ | postfix increment, e.g. a++ |
| -- | postfix decrement, e.g. a-- |
| + | (unary) plus, e.g. +a |
| - | (unary) negation, e.g. -a |
| ~ | (unary) bitwise negation, e.g. ~a |
| * | multiplication |
| / | division |
| % | modulus |
| + | addition |
| - | subtraction |
| << | bitwise shift left |
| >> | bitwise shift right |
| < | relational less than |
| <= | relational less than or equal |
| > | relational greater than |
| >= | relational greater than or equal |
| == | relational equal |
| != | relational not equal |
| & | bitwise AND |
| ¦ | bitwise OR |
| ^ | bitwise XOR |
| && | logical AND |
| ¦¦ | logical OR |
| ! | logical NOT |
| = | assignment (discussed further in 5.9) |

### Operator precedence and associativity

The operators are presented below in order of descending precedence. Operators appearing on an earlier line have higher precedence. Operators appearing on the same line have equal precedence. In general, operators with equal precedence have left-to-right associativity. However, some operators have right-to-left associativity and these are indicated below:

[] .

! ~ ++ -- +(unary) -(unary) lengthof() (right-to-left associativity)

* / %

+ -

<< >>

< <= > >=

== !=

&

^

¦

&&

¦¦

= (right-to-left associativity)

A pair of matching parentheses (e.g. ‘(’ and ‘)’) may be used to group operations to apply specific precedence and associativity which differ from the default. Parentheses may be used to group more than one operator. Parentheses may be nested. For example:

EXAMPLE

7 * 2 - 4 + 2     // equals 12 
7 * (2 – (4 + 2)) // equals -28

### Specific operator behaviour

The array element access operator (i.e. ‘[]’) shall be used with positive integer values. The behaviour of the array element access operator when the operand specifies an element index equal to or greater than the size of  the array is not defined by the SDL which can lead to undefined behaviour.

The modulus operator (i.e. ‘%’) shall not be used with float value operands. The sign of the result for the modulus operator with negative operands is not defined by the SDL which can lead to undefined behaviour. The result of the modulus operator where the value of the second operand is zero is not defined by the SDL which can lead to undefined behaviour.

The division operator (i.e. ‘/’) applied to integer values truncates the result meaning any fractional part of the value is discarded. The direction of truncation for integer division with negative operands is not defined by the SDL which can lead to undefined behaviour. The value of division where the value of the second operand is zero is not defined by the SDL which can lead to undefined behaviour.

The left shift operator (i.e. ‘<<’) fills vacated bits with zero.

The right shift operator (i.e. ‘>>’) applied to an unsigned int value fills vacated bits with zero. The behaviour of the right shift operator applied to a signed int value is not defined by the SDL which can lead to undefined behaviour.

Usage of the equality (i.e. ‘==’) and inequality operators (i.e. ‘<’, ‘<=’, '>’, ‘>=’ and ‘!=’) with operands of differing types involves number value coercion (discussed in 5.12.4).

## Expressions and evaluation

An expression is a sequence of operators and operands which are evaluated to produce a result which is either a value or a reference to a variable. The values of the operands are evaluated first before evaluating the result of the operator.

For the postfix operators (i.e. ‘++’ and ‘--’), the value of the operand is used in the expression before the result of the operator is evaluated. For example:

EXAMPLE 1

i = 1;
j = i++; // j now equals 1 and i equals 2

If a postfix operator (i.e. ‘++’ or ‘--’) appears on the left-hand side of either an assignment or comparison operator another postfix operator (i.e. either ‘++’ or ‘--’) shall not appear on the right-hand side of the same assignment or comparison operator.

The assignment operator (i.e. ‘=’) shall only appear once in an expression. For example, the following is invalid:

EXAMPLE 2

foo1 = foo2 = 1;   // illegal usage of assignment more than once
                   // in an expression

For the assignment operator (i.e. ‘=’), the evaluation order of operands is left to right. For example:

EXAMPLE 3

i = 0;
a[i] = i++; // i is used as an index in the array before it is
            // incremented
// note that i is used as the right-hand value before the postfix
// increment therefore after this expression: a[0] == 0 and i == 1.

For the logical operators (i.e. ‘&&’ and ‘||’), the evaluation order of operands is left-to-right and evaluation will stop as soon as the logical result value is determined. For example:

EXAMPLE 4

i = 0;
if (i == 0 || i++) { // i++ is not evaluated
  j = i;             // j will be equal to 0
}

For all binary operators except assignment, coercion is performed implicitly to ensure both operands are of a common type and the common type is then used for the value of the expression using the operator. For the assignment operator coercion is performed implicitly to force the right operand value to be the type of the left operand. Numeric value coercion is discussed in 5.12.4. Logic value coercion is discussed in 5.13.

## Statements

A statement consists of an expression followed by a semicolon. By surrounding a number of statements with a pair of matching braces ({ and }), the statements are grouped into a block which is itself treated as a single statement. Such a block is not followed by a semicolon.

Statements are evaluated in sequential order except for scenarios involving syntactic flow control (discussed in Clause 9).

## Built-in operators

This lengthof() operator returns the length, in bits, of the parsed variable contained in parentheses. The length is the number of bits that was most recently used to parse the variable value. A return value of 0 means that no bits were parsed for this variable, e.g. all parsed variable members of a class were contained in conditional clauses whose conditions were not met. The lengthof() operator shall not be used with a computed variable.

Rule O.1: lengthof() Operator

lengthof(parsed_variable)

EXAMPLE 1

int(3) foo;   // lengthof(foo) == 3
 
class A {           // classes are defined later in this document
  int (5) b;        // parsed variable
  computed int c;   // computed variable
}
 
A a;                 // lengthof(a) == 5
 
class Point {        // classes are defined later on in this document
  computed int x;
  computed int y;
}
 
map points (Point) {  // maps are defined later on in this document
  0b00, {100, 100},
  0b01, {200, 200}
}
 
Point(points) p;       // lengthof(p) == 2
 
utf8string myString;   // if myString == "hello" then
                       // lengthof(myString) == 48
                       // i.e. 5 characters + null termination
 
computed int i = 5;
computed int j = lengthof(i) // illegal usage of computed variable
computed int k = lengthof(5) // illegal usage of literal value
 
unsigned int(8)* m;   // lookahead is defined later in this document
                      // lengthof(m) == 8  despite the lookahead
 
class A {
  unsigned int(4) value1;
}
 
class B extends A {   // derived classes are defined later
                      // in this document
  unsigned int(4) value2;
}
 
B b;
 
// lengthof(b) == 8
// lengthof(b.value2) == 4  legal, expression result is a parsed value
// lengthof(b.value2 * 2)   illegal, expression result is
// a computed value
 
unsigned int(4) a[5]; // arrays are defined later in this document
 
// lengthof(a) == 20
// lengthof(a[0]) == 4 legal, expression result is a parsed value
// lengthof(a[0] * 2)  illegal, expression result is a computed value
class Conditional {
  unsigned int(3) foo;
  bit(1) bar_flag;
  if (bar_flag) {   // conditionals are defined later in this document
    unsigned int(8) bar;
  }
}
Conditional c;
// if c.bar_flag == 0, lengthof(c) == 4
// if c.bar_flag == 1, lengthof(c) == 12
 
 
class Looped {
  for (computed int i = 0; i < 2; i++) {
    int(8) offset[[i]];
  }
}
 
Looped looped; // lengthof(looped) == 16

Alignment attributes of parsed variables do not affect the value returned by the lengthof() operator as any skipped bits in the bitstream are not used to store the value of the variable. If the parsed variable is a class that contains members which have alignment attributes then these will affect the value returned by the lengthof()for the containing class.

The result of the lengthof() operator invoked on a class with an encoded class_id value shall include the number of bits used to encode the class_id value as well as the member elements of the class. It shall not include any bits skipped if the class is declared as aligned.

The result of the lengthof() operator invoked on an expandable class shall include the number of bits used to encode an optional class_id value, the sizeOfInstance value, the member elements of the class and any padding bits required at the end of the class to ensure it is an integer number of bytes in size. It shall not include any bits skipped if the class is declared as aligned.

EXAMPLE 2

int (3) foo1;               // lengthof(foo1) == 3
aligned(16) int (3) foo2;   // lengthof(foo2) == 3

The range operator defines the inclusive range of numbers from the specified min_value up to the specified max_value. The min_value and max_value shall be of the same type and the max_value shall be greater than or equal to the min_value.

Rule O.2: Range operator

min_value..max_value

The following are all valid and unique range expressions:

EXAMPLE 3

-1..3
0b01..0b11
MinID..MaxID
0x01 .. MaxID
-1e5 .. 1e5

A period (i.e., ‘.’) is used to access members of a class variable.

Rule O.3: Class member access operator

class_variable_identifier.member_identifier …

The operator may be repeated if the member is itself a class variable, e.g.:

EXAMPLE 4

level1.level2.level3

An expression may be used for the class_variable_identifier e.g.

EXAMPLE 5

myClassArray[2l].level2.level3

Rule O.4: Array element access operator

array_variable_identifier[index_expression] …

The operator may be repeated if the array element is itself an array variable, e.g.:

EXAMPLE 6

multiDimArray[2][i * 2]

An expression may be used for the array_variable_identifier, e.g.

EXAMPLE 7

parentClass.memberVariable.multiDimArray[2][i * 2]

## Numbers

### Endianness

All quantities shall be represented in the bitstream with the most significant byte first, and the most significant bit first.

For the purposes of expression evaluation, the storage of number values in the parsing context (for both parsed and computed variables) is also assumed to be with the most significant byte first, and the most significant bit first.

### Representation of numbers

The length of an unsigned or signed integer value is the number of bits used to represent the value in the bitstream. For a signed integer, one bit of the length is used to represent the sign.  Signed integer values shall be represented using the two’s-complement representation.

The values of float variables shall be represented in the bitstream using the interchange format for binary floating-point numbers defined in IEEE Std 754-2019 for the corresponding defined bit length.

Although the SDL supports signed zero for floating point literals, the interpretation and use of such values is undefined behaviour. Scenarios in which negative zero values are generated and how they are coerced is also undefined behaviour.

### Number limits

Parsed variables have defined width attributes which explicitly define the maximum and minimum (for signed numbers) limits of values which can be found in encoded bitstreams.

Computed variables values (which are stored only in the parsing context) and parsed variable values once parsed from the bitstream (which are then stored in the parsing context) do not have constraints on the storage width and therefore do not have number limits explicitly defined by the SDL. Care should be taken when creating SDL specifications to avoid undefined behaviour. As an example:

EXAMPLE

computed bit b = 0;
b++; // b is now equal to 1
b++; // undefined as to whether this causes a processing error or
     // b wraps to 0 or increments to 2

A computer program implementation or a standard containing an SDL specification may choose to specify such details.

### Value coercion

Numeric value coercion occurs when the type of one value must be converted to the type of another value. There are two forms of numeric value coercion in the SDL.

The first form of numeric value is conversion of two numeric types to a common type which is necessary when evaluating the result of a binary operator whose operands are of different types. Common value coercion shall occur in the following sequence:

If either value is floating point, the other is converted to floating point. When an integer or decimal value is converted to floating point, and the value is not exactly representable, then the result shall be the next lower representable value.

Otherwise, if either value is decimal, the other is converted to decimal.

Otherwise, the values are integers.

As the SDL does not specify constraints on the storage width of number values stored in the parsing context it is expected that all number values will always be of sufficient width to accommodate common value coercion and there is no need to define any behaviour when converting between signed and unsigned types.

The second form of numeric value coercion is forced conversion of one numeric type to another which is necessary when passing parameter values as part of a class definition (discussed in 7.1), when specifying a value for use with the array element access operator, when defining the dimensions of arrays (discussed in 7.7) and when defining partial arrays (discussed in 7.7.4). Forced value coercion shall occur as follows.

If an integer or decimal value is converted to floating point, and the value is not exactly representable, then the result shall be the next lower representable value.

If a floating point or decimal value is converted to an integer value the fractional part of the number is discarded. Conversion from a negative floating point or decimal value to an unsigned integer value is not defined by the SDL which may lead to undefined behaviour.

As the SDL does not specify constraints on the storage width of number values stored in the parsing context but it does specify the width of parsed variables, an SDL specification shall not use a literal value to initialise an elementary type parsed variable that is outside of the number limit range of a parsed variable type.

Apart from value coercion between numeric types, the SDL also supports explicit behaviour for logic value coercion (discussed in 5.13).

## Logic values

Numeric values or expression results may be used as logical Boolean values when evaluating expressions involving logical operators and when evaluating conditions used within syntactic flow control statements. To accommodate this, a numeric value of zero is coerced to a logic value of false when required and a non-zero value is coerced to a logic value of true. Coercion of Boolean values to numeric values is not supported.

## Binary literal values

To designate binary values, the ‘0b’ prefix shall be used followed by a series of one or more ‘0’ and ‘1’ characters.

Rule S.1: Binary literal value

0bbinary_value

A period (‘.’) may be optionally placed after every four digits for readability.

EXAMPLE 1

0b00100101
0b0010.0101

As the SDL is case sensitive, usage of a capital ‘B’ in the binary literal prefix is invalid. For example, the following is invalid:

EXAMPLE 2

0B00100101 // not a binary literal as uppercase B has been used

Binary literal values are treated as unsigned integer values when performing numeric value coercion.

## Hexadecimal literal values

To designate hexadecimal values, the prefix ‘0x’ shall be used followed by a series of characters in the range ‘A’ to ‘F’ and digits in the range ‘0’ to ‘9’.

Rule S.2: Hexadecimal literal value

0xhexadecimal_value

A period (i.e., ‘.’) may be optionally placed after every four characters for readability:

EXAMPLE 1

0xCAFEBEEF
0xCAFE.BEEF

As the SDL is case sensitive, usage of a capital ‘X’ in the hexadecimal literal prefix is invalid. For example, the following are invalid:

EXAMPLE 2

0XCAFEBEEF // not a hexadecimal literal as uppercase X has been used
0xCaFE     // invalid hexadecimal literal as lowercase a has been used

NOTE	0x0F is equivalent to 0b0000.1111.

Hexadecimal literal values are treated as unsigned integer values when performing numeric value coercion.

## Multiple character literal values

Multiple character literal values can be defined as a sequence of one or more characters from the range 0x20 to 0x7E, inclusive, of ISO/IEC 10646 enclosed in single quotation marks (e.g. ' and ').

Rule S.3: Multiple character literal value

'character_sequence'

EXAMPLE 1	'pasp'

Multiple character literal values are treated as unsigned integer values when performing numeric value coercion. Each character in the literal is treated as a single byte value (in the range 0x20 to 0x7E, inclusive) and used left to right. As an example:

EXAMPLE 2

'pasp' // equals (0x70 << 24) + (0x61 << 16) + (0x73 << 8) + 0x70 = 0x70617370

To allow the usage of the single quote character (i.e. ') as a character within a pair of single quotation marks (which are used to demarcate the multiple character literal value) the escape sequence \' is defined to mean the character '. For example:

EXAMPLE 3

'AB\'CD' // to be interpreted as: AB'CD

To accommodate the usage of the character sequence \' for single quote, the escape sequence \\ is also defined to mean the character \. For example:

EXAMPLE 4

'\\\\'  // to be interpreted as: \\
'\\\''  // to be interpreted as: \'

To aid in visual formatting of an SDL specification, adjacent multiple character literal tokens may be used. These shall be considered as concatenated values. For example, the following are equivalent:

EXAMPLE 5

'AB CD'
'AB'  ' CD'

Multiple character literal tokens on sequential lines shall also be considered concatenated. For example, the following is equivalent to the previous examples:

EXAMPLE 6

'AB'
' CD'

## Integer, decimal and floating-point literal values

Integer literal values shall be defined using digits and an optional signed unary operator (i.e. ‘+’, ‘-'). The following are valid integer literal values:

EXAMPLE 1

200
+200
-200
0

The following integer literal values are invalid:

EXAMPLE 2

-0   // signed zero is illegal
+0   // signed zero is illegal
002  // leading zeroes are illegal

Decimal literal values shall be defined using digits, an optional signed unary operator (i.e. ‘+’, ‘-'), a decimal separator (i.e. ‘.’) and at least one digit following the decimal separator. The following are valid decimal values:

EXAMPLE 3

58.0
58.3
0.4
+2.3
-2.3000
+0.2

The following decimal literal values are invalid:

EXAMPLE 4

-0.0   // signed zero is illegal
002.3  // leading zeroes are illegal
3.     // missing digit after decimal separator

Floating-point literal values shall be defined using an integer or decimal significand, an exponent indicator (i.e. ‘e’) and an integer exponent. The following are valid floating-point values:

EXAMPLE 5

123e67
123.456e-67
0.4e-67
+0.2e2
-4e1
-0e0 // negative zero
+0e0 // positive zero

The following floating-point literal values are invalid:

EXAMPLE 6

123E67   // uppercase E is illegal
00123e67 // leading zeroes in significand are illegal
123e067  // leading zeroes in exponent are illegal

## String literal values

String literal values shall be defined as a sequence of characters enclosed in double quotation marks (e.g. between ‘"’ and ‘"’) with an optional prefix u indicating that the string can contain SDL compatible UCS characters (as defined in 5.1). Lack of the prefix indicates usage of the basic SDL character set.

Basic string literals may contain the space character but shall not contain other white space characters such as tab or carriage return. For UCS string literals, if UCS characters outside of the SDL compatible UCS characters are required, a code point escape sequence shall be used (discussed further below).

The following are valid string literal values:

EXAMPLE 1

"hello world    " // basic character set
u"hello world §"  // UCS characters

The following are invalid string literal values:

EXAMPLE 2

"hello            // illegal carriage return character within...
world"            // ... string literal value
"hello world §"   // illegal UCS character in basic string literal

To allow the usage of the double quote character (i.e. ‘"’) as a string literal value within a pair of double quotation marks (which are used to demarcate the string literal), the escape sequence \" is defined to mean the character ". For example:

EXAMPLE 3

"It is \"good\"" // to be interpreted as: It is "good"

UCS code point escape sequences may be used in UCS string literals to designate characters that are not in the basic character set, are not printable or where any possible ambiguity of characters may exist, e.g. MICRO SIGN (UCS code point U+00B5) vs GREEK SMALL LETTER MU (UCS code point U+03BC). A universal character code point shall be defined using 4 uppercase hexadecimal characters prefixed with \u or 8 uppercase hexadecimal characters prefixed with \U. The hexadecimal value is the code point of the desired character as specified by ISO/IEC 10646. The following are valid string literal values:

EXAMPLE 4

u"\u1234"      // to be interpreted as: ሴ 
u"\U00001234"  // to be interpreted as: ሴ

To accommodate the usage of the character sequences \u and \U for universal character names and \" for double quote, the escape sequence \\ is also defined to mean the character \. For example:

EXAMPLE 5

u"\\u1234"   // to be interpreted as: \u1234
u"\\\\"      // to be interpreted as: \\
"\\\""       // to be interpreted as: \"

Further escape character sequences such as \n and \t (which other computer languages may ascribe special meaning to) are legal within string literal values but have no special meaning. A computer program implementation or a standard containing an SDL specification may choose to ascribe such meanings to such character sequences.

To aid in visual formatting of an SDL specification, adjacent string literals may be used. These shall be considered as concatenated values. For example, the following are equivalent:

EXAMPLE 6

"hello world"
"hello"  " world"

String literals on sequential lines shall also be considered concatenated. For example, the following is equivalent to the previous examples:

EXAMPLE 7

"hello"
" world"

Non-compatible string literals shall not be placed adjacent to each other. For example, the following is invalid:

EXAMPLE 8

u"hello" "world"  // illegal concatenation of UCS and basic character values

## Scope

### General

An identifier can denote a variable, a member of a map, a member of a class or a class parameter. The same identifier can denote different entities at different points in an SDL specification.

The scope of an identifier is the part of an SDL specification within which the identifier is visible and may be referenced. An identifier shall only be defined once within a single scope.  If an identifier defined in an outer scope is defined again within a nested inner scope, then within the inner scope, references to the identifier will refer to the inner scope variable definition and the outer scope variable definition is not visible.

There are three kinds of scope: global, class and block.

If the declaration of an identifier appears outside of any block or parameter list, the identifier has global scope. The scope of a global identifier commences at the point it is declared or defined and terminates at the end of the SDL specification. The only items that may be present in global scope are:

constant computed elementary variable definitions (defined in 8.2)

constant computed array definitions (defined in 8.3 )

map declarations (defined in Clause 3)

class declarations (defined in 7.1)

Class declarations and map declarations can only be made at global scope.

A standard presenting an SDL specification which consists of global scope elements introduced in separate document sections or which refers to global scope elements in another SDL specification should define details on the interpretation of element ordering. A standard presenting an SDL specification should also define the top level (or entry point) element that a parser should expect when commencing to parse a bitstream.

If the identifier appears within the list of parameters for a class declaration or within the top-level block declaring the class, the identifier has class scope. It is accessible anywhere within the block which declares the class. Class scope variables can be accessed outside of the class using the class member access operator (i.e. ‘.’).

An identifier declared within a block (introduced by the character '{' and exited by a matching paired character '}' has block scope. It is accessible from the point it is declared until the end of the block.

The SDL also defines two specific scope behaviors for parsed and computed variables which differ from other well-known programming languages. These are presented in the following two clauses.

### Parsed variable scope

Parsed variables (defined in Clause 6 and Clause 7) defined within a class declaration, regardless of nested block scopes or conditional branch (defined in 9.2) scopes have class scope, i.e., they are available as class member variables. Parsed elementary data type variables with definitions that fall outside the flow of parsing shall have an initial value of 0.

NOTE	This scoping behaviour differs from other well-known programming languages.

For example:

EXAMPLE

class A {
  // class scope starts...
  // foo1 is not yet defined
  int (5) foo1;    // parsed variable
  // foo1 is now defined and accessible in class scope
  {
    // nested block scope starts...
    // foo1 can be referenced here due to both class scope 
    // and outer block scope
    // foo2 is not yet defined
    int (5) foo2;  // parsed variable
    // foo2 is now defined and accessible in block scope AND class scope
  }
  // foo2 can be referenced here as it has class scope
  if (foo1 == 0) {
    // conditional block scope starts...
    int (5) foo3;  // parsed variable
    // foo3 is now defined and accessible in block scope AND class scope
  }
  // foo3 can be referenced here as it has class scope even if foo1 != 0
}
 
A a;
 
// a.foo1 is accessible
// a.foo2 is accessible
// a.foo3 is accessible, value is 0 if foo1 != 0

### Computed variable scope

In general, computed variables (defined in Clause 8), have block scope. The exception to this is computed variables defined in the top-level scope of a class which have class scope, i.e. they are available as class member variables.

For example:

EXAMPLE

class B {
  // class scope starts...
  // bar1 is not yet defined
  computed int bar1;    // computed variable
  // bar1 is now defined and accessible in class scope
  {
    // nested block scope starts...
    // bar1 can be referenced here due to both class scope
    // and outer block scope
    // bar2 is not yet defined
    computed int bar2;  // computed variable
    // bar2 is now defined and only accessible in block scope
  }
  if (bar1 == 0) {
    // conditional block scope starts...
    computed int bar3;  // computed variable
    // bar3 is now defined and only accessible in block scope
  }
}
 
B b;
 
// b.bar1 is accessible
// b.bar2 is NOT accessible
// b.bar3 is NOT accessible

## SDL specification limits

As this document does not cover parsing or translation of SDL specifications, the following are identified as unspecified constraints on an SDL specification:

maximum levels of block nesting;

maximum levels of expression nesting;

maximum levels of class nesting;

maximum levels of map nesting;

maximum number of identifiers;

maximum number of class members;

maximum number of labelled statements in a switch clause;

maximum number of characters in a string literal;

maximum number of parameters in a class parameter list;

maximum character length of a multiple character numeric literal;

maximum number of concatenated adjacent multiple character numeric literals;

maximum character length of a string literal value;

maximum number of concatenated adjacent string literal values;

maximum number of dimensions in a multi-dimensional array;

maximum number of elements in each dimension of an array.

# Elementary data types

## General

The SDL defines the following elementary data types as listed below and described in more detail in the subsequent subclauses.

- Constant-length direct representation bit fields or Fixed Length Codes (FLCs). These describe the encoded value exactly as it is to be used by the appropriate decoding process (see 6.2).

- Variable length direct representation bit fields, or parametric FLCs. These are FLCs for which the actual length is determined by the context of the bitstream (e.g. the value of another parameter) (see 6.3).

- Constant-length indirect representation bit fields. These require an extra lookup using an encoded value FLC into an appropriate table or variable to obtain the desired value or set of values (see 6.4).

- Variable-length indirect representation bit fields (e.g., Huffman codes). These require an extra lookup using an encoded value parametric FLC into an appropriate table or variable to obtain the desired value or set of values (see 6.5).

- Variable length strings. These represent a series of characters (see 6.6).

## Constant-length direct representation bit fields

Constant-length direct representation bit fields, or FLCs, which are parsed variable definitions shall be represented as:

Rule E.1: Elementary data types

[const] [aligned[(modifier)]] type(length) identifier[=value];

### Alignment

The keyword aligned indicates that the data is aligned on a byte boundary.

For example, a 16‑bit value aligned on a byte boundary:

EXAMPLE 1

aligned bit(16) foo;

An example bitstream for this would be:

EXAMPLE 2

...
<skip 0..7 bits>                  // byte aligned
0 0 0 1 0 0 1 0 0 1 0 0 1 0 0 0   // foo = 0x1248
...

An optional numeric modifier attribute may be used to signify alignment on boundaries other than byte. Allowed values are 8, 16, 32, 64, and 128. Any skipped bits due to alignment shall have the value ‘0’.

For example, an 8‑bit value aligned on a 2‑byte boundary:

EXAMPLE 3

aligned(16) bit(8) foo;

An example bitstream for this would be:

EXAMPLE 4

...
<skip 0..15 bits> // 2-byte aligned
0 0 0 1 0 0 1 0   // foo = 0x12
...

### Type

The grammar rule construct type comprises the following: int for signed integer, unsigned int for unsigned integer, float for floating point, and bit for raw binary data.

### Length

The length attribute indicates the length of the element value in bits, as it is stored in the bitstream. When the grammar rule construct type is float, then the length attribute shall be equal to 16, 32, 64, 128 or 256.

NOTE	As some of those bit depths may be uncommon for floats, the expected environment implementing a given SDL specification must support the chosen bit depth.

Refer to 5.12.3 for a discussion on the maximum value of length.

For example, a 5‑bit unsigned integer that is parsed would be represented as:

EXAMPLE 1

unsigned int(5) parsed_variable;

An example bitstream for this would be:

EXAMPLE 2

...
1 0 0 1 0 // parsed_variable = 0x12
...

### Value

The value attribute shall be present only when the value is constrained to a single value (e.g. start codes) or a range of values (i.e., ‘0x01..0xAF’). A variable or expression may be used when defining a single value or range of values for the value attribute. For example:

EXAMPLE

unsigned int(8) min = 0..4;
unsigned int(8) max = 4..7;	
unsigned int(8) foo = min .. max - 1;

The single value or range of values defined shall be valid for the defined variable type.

Refer to 5.12.3 for a discussion on the limits of value.

Refer to 5.12.4 for a discussion on the coercion of value.

Parsed elementary data type variables with definitions that fall outside the flow of parsing shall have an initial value of 0.

### Constants

Constants shall be defined using the keyword const.

EXAMPLE 1

const int(8) SOME_VALUE;       // the parsed value will be immutable
const bit(2) BIT_PATTERN=0b01; // the parsed value shall be 0b01 and
                               // will be immutable

An example bitstream for this would be:

EXAMPLE 2

...
0 0 0 1 0 0 1 0 // SOME_VALUE = 0x12 and is now immutable
0 1             // BIT_PATTERN = 0x1 (error if bitstream not "0 1")
                // and is now immutable
...

A const variable value shall not be modified. For example, the following is invalid:

EXAMPLE 3

const int(8) PRECISION = 10;
precision++; // mutating a constant value is illegal

### Look-ahead

In some scenarios, it can be desirable to examine the immediately following bits in the bitstream, without consuming these bits. To support this behaviour, a ‘*’ character shall be placed after the parse length parentheses to modify the parse size semantics.

Rule E.2: Look-ahead parsing

[const] [aligned[(modifier)]] type(length)* identifier [=value];

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword const has the same meaning as in 6.2.5.

For example, the value of the next 8 bits in the bitstream can be read as an unsigned integer without advancing the current position in the bitstream using the following representation:

EXAMPLE 1

unsigned int(8)* next_byte;
 
// check if high-bit set
if (next_byte & 0x80) {
  // if so parse as a 16-bit value
  unsigned int(16) value;
}
else {
  // if not parse as an 8-bit value
  unsigned int(8) value;
}

NOTE	The conditional if-then-else construct is defined in 9.2.

An example bitstream for this would be:

EXAMPLE 2

...
                                // next_byte = 0x87
1 0 0 0 0 1 1 1 0 1 0 0 1 0 1 0 // value = 0x8746
...

### Legacy

In some scenarios, a variable needs to be declared for legacy reasons but is no longer used in practice. To declare such a variable, the keyword legacy shall be used.

Rule E.3: Legacy keyword

legacy [const] [aligned[(modifier)]] type(length) identifier [=value];

NOTE	The look-ahead function defined in 6.2.6 is not supported for the legacy keyword.

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword const has the same meaning as in 6.2.5.

When the optional value attribute is present, it shall be a single value and not a range of values. The value is merely an indication of the possible value encoded in the bitstream but is not a restriction as in the case where the keyword legacy is absent.

For example, the following class definition (classes are defined in 7.1) defines the second and third member variables for legacy reasons but their values have no meaning:

EXAMPLE

class WidelyUsedClass {
  int(8) important_1;
  legacy int(8) legacy_1;          // value should be ignored
  legacy int(8) legacy_2 = 0x10;   // value should be ignored,
                                   // might not be 0x10
  int(8) important_2;
}

### Reserved

In some scenarios, the semantics of a variable’s value is exclusively defined by the SDL specification declaring it even though the values are left unspecified. To declare such a variable, the keyword reserved shall be used.

Rule E.4: Reserved keyword

reserved [const] [aligned[(modifier)]] type(length) identifier [=value];

NOTE	The look-ahead function defined in 6.2.6 is not supported for the reserved keyword.

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword const has the same meaning as in 6.2.5.

When the optional value attribute is present, it is merely an indication of the possible value encoded in the bitstream but is not a restriction as in the case where the keyword reserved is absent.

For example, the following class definition (classes are defined in 7.1) defines the first member and reserves the second and third members for future potential use:

EXAMPLE

class PotentiallyUsefulClass {
  int(8) important;
  reserved int(8) unused_1;                  // value should be ignored
  reserved int(8) unused_2 = 0x00..0xF0;     // value should be ignored,
                                             // might not be in range
}

## Variable length direct representation bit fields

The case of variable length direct representation bit fields, or parametric FLCs is covered by Rule E.1, by allowing the length attribute to be a parsed variable included in the bitstream, a computed variable, or an expression involving such variables. In case the length attribute is determined by an expression, the result of this expression shall be a strictly positive integer value.

For example, in the following representation, the size of the variable DC is determined by the 3‑bit unsigned integer value precision:

EXAMPLE 1

unsigned int(3) precision;
int(precision) DC;

An example bitstream for this would be:

EXAMPLE 2

...
1 0 1      // precision = 5
1 1 1 0 1  // DC = -3
...

## Constant-length indirect representation bit fields

Indirect representation indicates that the actual value of the element at hand is indirectly specified by the bitstream using a table or map structure. In other words, the value extracted from the bitstream, which is an FLC, is an index to a map from which the final desired value is extracted. This indirection is expressed by defining the map itself:

Rule E.3: Maps

map map_identifier(output_type) {
  index,
  {value[, …]}
  [,…]
}

The content of the map is declared as a set of pairs of input index values and output values. The index values shall be unique.

NOTE	The syntax for maps does not allow to specify the alignment of map index values. Achieving such alignment is the responsibility of the SDL definition author.

The input type of a map (the index specified in the first column) shall always be bit and therefore index values always expressed as binary literal values. For the case of constant-length indirect representation bit fields, these binary literal values shall all be the same length. The length in bits of the index is the number of bits read from the bitstream.

The output_type of a map shall be either a type or a defined class identifier (classes are defined in 7.1).

Output values used to populate the output_type shall be specified as aggregates surrounded by curly braces ( ‘{’ and ‘}’). Output values shall only be numeric literal values or constants (with the exception of parsed values used within variable length indirect representation bit fields as defined in 6.5).

For example, a map that relates an input binary value to an output populated YUVblocks class (classes are defined in 7.1) can be defined using the following representation:

EXAMPLE 1

class YUVblocks {// classes are defined later on in this document
  computed int Yblocks; // computed variables are defined later
                        // in this document
  computed int Ublocks;
  computed int Vblocks;
}
 
// a table relating chroma formats to the number
// of blocks per signal component
map blocks_per_component (YUVblocks) {
  0b00, {4, 1, 1},
  0b01, {4, 2, 2},
  0b10, {4, 4, 4}
}

In the above example the class YUVblocks contains only computed member variables (defined in 8). Although the SDL supports using a declared class containing parsed member variables as the output_type of a map, such usage will result in undefined behaviour.

As another example, a map that relates an input binary value to an output populated int value can be declared using the following representation:

EXAMPLE 2

map offsets (int) {
  0b00, {1024},
  0b01, {2048},
  0b10, {4096}
}

As class declarations may include nested hierarchies, the aggregate output values may also include nesting. As an example:

EXAMPLE 3

class Foo {
  computed int f1;
  computed int f2;
}
 
class Bar {
  computed int b1;
  Foo foo1;
}
 
map barMap (Bar) {
  0b00, {0, { 1, 1} },
  0b01, {4, { 2, 2} },
  0b10, {4, { 4, 4} }
}

The type and number of constituent values within the aggregate output values of the map declaration shall match the corresponding constituent type within the map’s output_type. For example, the following is invalid:

EXAMPLE 4

map illegal (int) {
  0b00, {1.1},    // illegal float value for an integer output
  0b01, {2, 2},   // illegal multiple values for a single value output
  0b10, {}        // illegal missing value
}

The next rule describes the usage of map declarations to define variables.

Rule E.4: Mapped data types

[legacy|reserved] output_type<map_identifier> map_variable_identifier;

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The output_type of the variable shall be identical to the output_type declared for the map.

For example, the following makes use of the blocks_per_component map declared above which has a YUVBlocks classoutput_type:

EXAMPLE 5

YUVblocks<blocks_per_component> chroma_format;
 
if (chroma_format.Ublocks != 4) {
  unsigned int(8) u_width;
  unsigned int(8) u_height;
}

An example bitstream for this would be:

EXAMPLE 6

...
0 1              // input 0b01 maps to
                 // chroma_format = { 
                 //   Yblocks = 4, Ublocks = 2, Vblocks = 2
                 // }
0 0 0 1 0 0 0 0  // u_width = 16
0 0 0 1 0 0 0 0  // u_height = 16
...

As another example, the following makes use of the offsets map declared above which has an intoutput_type:

EXAMPLE 7

int<offsets> index_offset;
 
if (index_offset == 2048) {
  unsigned int(6) foo;
}

An example bitstream for this would be:

EXAMPLE 8

...
0 1          // input 0b01 maps to 
             // index_offset = 2048
0 1 0 0 0 0  // foo = 16
...

## Variable length indirect representation bit fields

For a variable length element utilizing a Huffman or variable length code table, an identical SDL specification to the fixed length case shall be used.

The only difference is that the indices of the map are now of variable length and shall unambiguously identify each output value in addition of being unique (e.g. leaf values of a binary tree).

EXAMPLE 1

class val {
  computed unsigned int foo;
  computed int bar;
}
 
map sample_vlc_map (val) {
  0b0000.1,   {0, 5},
  0b0000.01,   {1, -14}
}

Due to the large number of possible entries in variable length code tables, it can be inefficient to keep using variable length codewords for all possible values. Because of this, they are often partially defined. This necessitates the use of escape codes, that signal the subsequent use of a fixed-length (or even variable length) value representation. To support this, parsed variable type declarations may be used for map output values.

In this case, the type of the output value within the map declaration shall match the type associated with the map’s output_type.

Rule E.5: Maps with escape codes

map map_identifier(output_type) {
  index, {value | type(length)[, …]}[,
   …]
}

In the following example, when the codeword 0b0000.0000.1 is encountered in the bitstream, then the value ‘5’ is assigned to the first element of the output value (i.e. val.foo). The following 32 bits are then parsed and assigned as the value of the second element of the output value (i.e. val.bar).

EXAMPLE 2

class Val {
  computed unsigned int foo;
  computed int bar;
}
 
map sample_map_with_esc (Val) {
  0b0000.001,      {0, 5},
  0b0000.0001,     {1, -14},
  0b0000.0000.1,   {5, int(6)},
  0b0000.0000.0,   {0, -20}
}
 
Val<sample_map_with_esc> myVal;

An example bitstream for this would be:

EXAMPLE 3

...
0 0 0 0 0 0 0 0 1 // input 0b0000.0000.1 maps to
                  // myVal = { 
                  //   foo = 5,
0 1 0 0 0 0       //   bar = 16
                  // }
...

NOTE	In case more than one element utilizes a parsed type declaration, the order in which elements are parsed determines the extracted values.

## Variable length strings

A variable length parsed variable string definition shall be represented as:

Rule E.6: String data types

[legacy|reserved] [const] [aligned[(modifier)]] string_type string_identifier [=[u]"string_value"];

The string_type shall be any of the following: utf8string, utf16string,utfstring, utf8list, base64string. The format of those string types shall be as described in Table 1. In these descriptions, null-terminated means that the last character of a string is a UCS NUL character, and hence an empty string is represented by a single UCS NUL character. In addition, space-separated means that a SPACE character whose UCS code point is U+0020 is used as string separator. In a utf8list string, there shall not be any leading or trailing space character nor two consecutive space characters.

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword const has the same meaning as in 6.2.5.

The keyword aligned and its modifier have the same meaning as in 6.2.1.

Table 1 — String data type formats

| Name | Format |
| --- | --- |
| utf8string | UTF-8 encoded string as defined in ISO/IEC 10646, null-terminated. |
| utf16string | UTF-16 encoded string as defined in ISO/IEC 10646, null-terminated. The sequence of bytes shall start with codepoint U+FEFF (ZERO WIDTH NO-BREAK SPACE) and the null termination shall be 2 bytes set to 0. |
| utfstring | UTF-8 or UTF-16 encoded string as defined in ISO/IEC 10646, null-terminated. If UTF-16 is used, the sequence of bytes shall start with codepoint U+FEFF (ZERO WIDTH NO-BREAK SPACE) and the null termination shall be 2 bytes set to 0. |
| utf8list | null-terminated list of space-separated UTF-8 encoded strings as defined in ISO/IEC 10646 |
| base64string | null-terminated base64 compliant encoded data as defined in clause 4 of RFC 4648 |

The encoding of a utfstring value into a bitstream may be performed using either UTF-8 or UTF-16 and the encoding to use is not defined by the SDL which can lead to undefined behaviour. A computer program implementation or a standard presenting an SDL specification may choose to define expected behaviour in this scenario.

EXAMPLE

aligned(8) utf8string message;
base64string encoded_data;

### String value

The string_value attribute shall represent a string literal. The string_value shall not include a null termination character nor codepoint U+FEFF (ZERO WIDTH NO-BREAK SPACE) which may be present in the encoded bitstream. When a string literal is present, the character set of the string literal (basic or UCS indicated by the absence or presence of a u prefix respectively) shall be compatible with the string_type of the variable:

a string literal prefix u with utf8string, utf16string,utfstring and utf8list variables.

no string literal prefix with base64string variables.

EXAMPLE 1

utfstring code = u"this is a code";
utf8list interesting_list = u"apple orange cherry";
utf8string mot = u"cœur";
base64string = "aGVsbG8K"

Parsed string variables with definitions that fall outside the flow of parsing shall have an initial value of an empty string (e.g. ‘""’).

A string_value shall not conflict with the string literal prefix or the string_type. For example, the following is invalid:

EXAMPLE 2

base64string = "inval?id"; // illegal character in a base64 string

A computer program implementation of the SDL or a parser for an SDL defined bitstream shall not perform any form of UCS character normalization, composition or decomposition. The exact ordered set of code points stored in the bitstream shall be returned by a parser, even if the encoding of the data is changed when stored in the parsing context e.g., UTF-8 to UTF-16, UTF-16 Big Endian to UTF-16 Little Endian. For example, if the bitstream contains the codepoint U+00B5 (MICRO SIGN), a parser shall not canonically normalize that codepoint to U+03BC (GREEK SMALL LETTER MU).

# Composite data types

## Classes

Classes are the mechanism with which definitions of composite types is performed. Their syntax is as follows.

Rule C.1: Classes

[aligned[(modifier)]] class class_identifier {
  [element; …]
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The element entries within the curly braces (“{“ and “}”) are the definitions of the contained elementary data types (as defined in Clause 6), composite data types (as defined in Clause 7) or syntactic flow control elements (as defined in Clause 9). The order of declaration of the elements is the same order in which the elements appear in the bitstream. Classes may also be encapsulated within other classes. In this case, the element in Rule C.1 is a class itself. A class declaration shall not recursively reference itself.

NOTE	As alignment is performed by advancing the current position in the bitstream to be before the next encoded value, it is somewhat redundant but not illegal to declare a class as aligned, and to declare the first element of the class as aligned. In this case, the alignment before the first element’s encoded value in the bitstream is performed using the largest of the two alignment declarations.

The next rule describes the use of a declared class.

Rule C.2: Class data types

[legacy] class_identifier class_variable_identifier;

The keyword legacy has the same meaning as in 6.2.7.

A particular member element declared in a class variable can be accessed (considering the scoping rules defined in 5.19) using the class member access operator (“.”).

There is no ability to nest class declarations inline. For example this is invalid:

EXAMPLE 1

class Foo {
  class Bar { // invalid nested class declaration
    unsigned int(8) a;
  }
}

A correct declaration syntax for this would be:

EXAMPLE 2

class Bar {
  unsigned int(8) a;
}

class Foo {
  Bar b;
}

## Base and derived classes

The optional keyword extends followed by the base_class attribute specifies that the class is a derived class and that it derives from another class of type base_class called the base class. The base class shall be either a class (as defined in 7.1), a derived class (as defined in this subclause) or an abstract class (as defined in 7.3). Derivation implies that all elements present in the base class can also be accessed in the derived class, and that, in the bitstream, all such information precedes any additional elements specified in the derived class.

Rule C.3: Derived classes

[aligned[(modifier)]] class class_identifier [extends base_class] {
  [element; …]
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

A derived class shall not differ in declared alignment from the base class.

In the following example, myBar.b immediately precedes myBar.c in the bitstream:

EXAMPLE 1

class Foo {
  int(3) a;
}
 
class Bar extends Foo {
  int(a) b;      // base class element a is accessible
  int(10) c;
}
 
Bar bar;
 
// bar.b from derived class is accessible 
// bar.a from base class is accessible

An example bitstream for this would be:

EXAMPLE 2

...
                    // bar = {
1 0 1               //   a = 5,
0 0 1 0 0           //   b = 4,
0 0 0 0 0 0 1 0 0 0 //   c = 8,
                    // } 
...

## Abstract classes

The optional keyword abstract specifies that the class is an abstract class. Only non-abstract classes, possibly derived from a base class, shall be present in the bitstream.

Rule C.4: Abstract classes

[aligned[(modifier)]] abstract class class_identifier [extends base_class] {
  [element;…]
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword extends and its base_class have the same meaning as in 7.2.

An abstract class may derive from another abstract class. An abstract class may also derive from a non-abstract class.

EXAMPLE 1

abstract class Shape {
}
 
class Circle extends Shape {
  unsigned int(16) radius;
}
 
class Rectangle extends Shape {
  unsigned int(16) width;
  unsigned int(16) height;
}
 
// only Circle and Rectangle classes can be present
class Example {
  Circle c;
  Rectangle r;
}
 
Example myExample;

An example bitstream for this would be:

EXAMPLE 2

...
                // myExample = {
                //   c = {
0 0 0 0 0 0 0 0
0 0 0 0 0 0 1 0 //     radius = 2
                //   }, 
                //   r = {
0 0 0 0 0 0 0 0 
0 0 0 0 0 1 0 0 //     width = 4,
0 0 0 0 0 0 0 0
0 0 0 0 1 0 0 0 //     height = 8
                //   }
                // } 
...

## Polymorphism in class declaration

### General

If the bit keyword is added to a class definition, a derived class may appear at any point in the bitstream where its base class is used in the syntax, hence allowing polymorphism in the SDL syntax.

Rule C.5: Class polymorphism

[aligned[(modifier)]] [abstract] class class_identifier [extends base_class] [: bit(length)[class_id_identifier=]class_id|id_range|extended_id_range]{   [element; …]}
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword abstract has the same meaning as in 7.3.

The keyword extends and its base_class have the same meaning as in 7.2.

The class_id value is the key demultiplexing entity which is present in the bitstream before any class member variable of the class. This allows differentiation between base and derived classes when parsing. The length of the class_id in bits is given by the length attribute following the bit keyword. The optional attribute class_id_identifier allows to access this class_id value from within the class. If the class is aligned, the alignment occurs in the bitstream before the encoded class_id.

When the bit keyword is declared, all derived class class_id attributes shall declare the same length attribute value as the base class.

When a class_id_identifier is declared, all derived classes shall declare the same class_id_identifier. If a class_id_identifier is not declared, all derived classes will also not declare one.

The actual class to be parsed is determined as follows:

The base class declaration shall assign a constant value or a range of values to class_id.

Each derived class declaration shall assign a constant value to class_id.

Derived classes may use the same class_id value as the base class. In that case, classes can only be discriminated through contextual information such as the value of a member variable from the base class.

EXAMPLE 1

class Foo : bit(2) id = 0 {
  // note that as "id" is declared it is accessible within this class
  // as a constant value and lengthof(id) will return 2
  int(5) a; // this a is preceded by the 2 bits of id    
}
 
class Foo1 extends Foo : bit(2) id = 1 {
  int(3) b; // this b is preceded by the 5 bits of a    
}
 
class Foo2 extends Foo : bit(2) id = 2 {
  int(4) c; // this c is preceded by the 5 bits of a
}
 
class Example {
  Foo f;   // may be Foo, Foo1 or Foo2
}
 
Example myExample;
 
// myExample.f.id will equal 0, 1 or 2

An example bitstream for this would be:

EXAMPLE 2

...
                // myExample = {
                //   f = {
1 0             //     id = 2,
0 0 0 0 1       //     a = 1,
0 0 1 1         //     c = 3
                //   }
                // }
...

As an alternative to a single class_id value, it is also possible to declare:

an id_range attribute which is a range of numerical values specified as start_id..end_id, inclusive of both bounds

an extended_id_range attribute which is a combination of id_range and class_id  specified as a comma-separated list of class_id values and id_range values e.g. myId=0x01,0x02,0x10..Ox1F.

In such cases:

The base class declaration shall declare a range of valid values via an id_range or extended_id_range.

Each derived class declaration shall assign a constant value or a range of valid values via a class_id value or an id_range or extended_id_range. This value or range of values shall correspond to legal values declared for the base class.

EXAMPLE 3

class Foo : bit(5) id = 1,10..20 {
  int(5) a;
}
 
class Foo1 extends Foo : bit(2) id = 10 {
  int(5) b; // this b is preceded by the 5 bits of a
}

It is possible that an SDL specification does not provide class declarations for all valid class_id values.  For example:

EXAMPLE 4

class Foo : bit(3) = 0,5,6..7 {
}
 
class Foo0 : bit(3) = 0 {
}
 
class Foo5 : bit(3) = 5 {
}

In the example above as there are no class declarations for the class_id values of 6 and 7, this can result in undefined behaviour. The behaviour shall be specified by the standard presenting the SDL specification.

NOTE	For example, the standard presenting the SDL specification can state that a class declared using an id_range or extended_id_range is effectively abstract, i.e. class_id values encountered in the bitstream are values appearing as single class_id values in derived class declarations. Alternatively, the standard can state that any class_id encountered in the bitstream which is not associated with a derived class declaration but which is valid according to the base class declaration results in the base class being used for parsing.

In the example above, if a bitstream is encoded using a class_id value of 4 this will also result in undefined behaviour and shall be addressed by the standard presenting the SDL specification.

### Polymorphism for abstract classes

Since abstract classes are not present in a bitstream, this means that the derived classes of an abstract class may use the entire range of class_id values available. For the abstract base class, the class_id shall be specified as 0 or alternatively an id_range or extended_id_range shall be specified.

EXAMPLE

abstract class Foo : bit(1) id = 0 { // the value 0 is not really used
}
 
// derived classes are free to use the entire range of ids,
// in this case 0..1
class Foo0 extends Foo : bit(1) id = 0 {
  int(5) a;
}
 
class Foo1 extends Foo : bit(1) id = 1 {
  int(10) b;
}
 
class Example {
  Foo f;   // can only be Foo0 or Foo1, not Foo
}

## Expandable classes

When the expandable keyword is used in the class declaration, it indicates that the class may contain implicit arrays or undefined trailing data, called the “expansion”.

Rule C.6: Expandable classes

[aligned[(modifier)]] expandable[(max_class_size)]
  [abstract] class class_identifier
    [extends base_class] [:bit(length)[class_id_identifier=]class_id|id_range|extended_id_range]{
  [element; …]
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword abstract has the same meaning as in 7.3.

The keyword extends and its base_class have the same meaning as in 7.2.

The keyword bit and its length have the same meaning as in 7.4.1.

Expandable classes can be used for classes that are required to support future compatible extensions or that include private data. For example, a legacy device can decode an expandable class up to the last parsed variable appearing in the version of the class declaration that the device is aware of and can skip the unknown class data following the last known variable.

To this end, an expandable class explicitly encodes its own size in bytes in the bitstream. The size precedes any parsed variables of the class and its variable-length encoding is defined below using the SDL convention:

computed int sizeOfInstance = 0;
bit(1) nextByte;
bit(7) sizeByte;
sizeOfInstance = sizeByte;
 
while (nextByte) {
  bit(1) nextByte;
  bit(7) sizeByte;
  sizeOfInstance = sizeOfInstance << 7 | sizeByte;
}

NOTE	By definition, the encoding of sizeOfInstance is always an integer number of bytes in size.

The size information is implicitly accessible within the class as the member variable sizeOfInstance whenever a class is made expandable.

If the expandable keyword has a maxClassSize attribute, then this indicates the maximum permissible size of this class in bytes, i.e. a maximum permissible value for sizeOfInstance.

An expandable class may derive from another expandable class. An expandable class may also derive from a non-expandable class. A non-expandable class shall not derive from an expandable class.

As the sizeOfInstance value encodes the class size in bytes, an expandable class should be declared in a way to ensure that its size is always an integer number of bytes. If this is not the case, padding bits shall be present in the bitstream at the end of the last encoded parsed member variable. These padding bits shall be included when determining the sizeOfInstance value.

If the class declaration uses the bit keyword (defined in 7.4.1) indicating the presence of an encoded class_id, the encoding of this value shall precede the size encoding. If the class is aligned, the alignment occurs in the bitstream before the encoded class_id and size information. The sizeOfInstance value shall not include the number of bytes needed for the size encoding, the bits skipped to achieve alignment nor the class_id encoding.

Anywhere in the bitstream where a set of expandable classes with class_id values is expected, it is permissible to intersperse expandable classes with unknown class_id values. These classes shall be skipped, using the size information.

The following example declares an expandable class with a maximum size of 120 bytes:

EXAMPLE 1

aligned expandable(120) class Example {
  // note that as the class is expandable "sizeOfInstance" is
  // accessible within this
  // class as a constant value and lengthof(sizeOfInstance)
  // will return a multiple of 8
  int(3) a;
  // 5 padding bits will follow in the bitstream to ensure the class
  // size is an integer number of bytes
}
 
Example myExample;
 
// myExample.sizeOfInstance == 1

An example bitstream for this would be:

EXAMPLE 2

...
<skip 0..7 bits>  // aligned
0                 // nextByte = 0
0 0 0 0 0 0 0 1   // sizeByte = 1
                  // myExample = {
                  //   sizeOfInstance = 1,
0 0 1             //   a = 1  
0 0 0 0 0         //   5 padding bits to ensure
                  //   myExample.sizeOfInstance == 1 byte
                  // }
...

## Parameter types

Classes may be declared with parameters. This addresses cases where the data structure of the class depends on referencing values of one or more other parsed items. Parameter types provide placeholders for such references, in the same way as the arguments in a C function declaration. The syntax of a class declaration including parameter functionality is as follows.

Rule C.7: Class parameter types

[aligned[(modifier)]] [expandable[(max_class_size)]]
  [abstract] class class_identifier[(parameter_list)]
    [extends base_class[(parameter_value[, parameter_value]…)]] [: bit(length)[class_id_identifier=]class_id|id_range|extended_id_range]{
  [element;…]
}

The keyword aligned and its modifier have the same meaning as in 6.2.1.

The keyword expandable and its max_class_size have the same meaning as in 7.5.

The keyword abstract has the same meaning as in 7.3.

The keyword extends and its base_class have the same meaning as in 7.2.

The keyword bit and its length have the same meaning as in 7.4.1.

The parameter_list shall be a list of type or class identifiers and variable identifier pairs separated by commas. Usage of a string_type in a parameter_list is not supported.

The comma separated list of parameter_value entries is defined below.

A class that uses parameters is dependent on the values in its parameter list. When populating the member values of such a class, the parameters shall be already be populated with values. The value of any parsed variable or computed variable (defined in 8) accessible within the current scope, may be passed as a parameter.

The next rule describes the use of a declared class which uses parameters.

Rule C.8: Parameterized class data types

class_identifier class_variable_identifier(parameter_value [, parameter_value]…);

The comma separated list of parameter_value entries shall be either expressions (e.g. i+a[0]) or constant values (e.g. 0b01) which match the number and type of the parameters specified in the corresponding class declaration parameter_list. The evaluation order of parameter_value values shall be from left to right.

Parameter values used to populate a class variable shall match the number and type of the parameters specified in the parameter list. For example, the following is invalid:

EXAMPLE 1

class A (int i, int j) {
  unsigned int(i) foo;
  unsigned int(j) bar;
}
 
class B {
  int(2) i;
  A a1(i);   // illegal mismatch of parameter count
}

In the following example, class B declares two parameters a and i and the values are populated within class C when parsing the bitstream:

EXAMPLE 2

class A {
  unsigned int(4) format;
}
 
class B (A a, int i) {      // B declares two parameters
  unsigned int(i) foo;
  if (a.format == 2) {
    int(4) bar;
  }
}
 
class C {
  int(2) i;
  A a;
  B b(a, i); // two expressions used as parameters values
}
 
C c;

An example bitstream for this would be:

EXAMPLE 3

...
         // c = {   
1 1      //   i = 3,
         //   a = {
0 0 1 0  //     format = 2
         //   },
         //   b = {
1 0 1    //     foo = 5,
0 1 1 1  //     bar = 7
         // }
...

In the following example, class A declares one parameter i and the value is populated during the declaration of class B:

EXAMPLE 4

class A (int i) {           // A declares one parameter
  unsigned int(4) id = i;   // parsed value for id must equal
                            // the value of i
  unsigned int(4) value1;
}
 
class B extends A (3) {      // constant used as parameter value
  unsigned int(4) value2;
}
B b;

An example bitstream for this would be:

EXAMPLE 5

...
         // b = {   
1 1      //   id = 3,
0 0 1 0  //   value1 = 2,
0 1 0 1  //   value2 = 5,
         // }
...

In the following example, class A declares two parameters i and j and class B also declares two parameters x and y. The class B parameters are used in expressions as parameter values to the base class A. The value of x is populated during the declaration of class C and the value of z is populated during the definition of the class variable c:

EXAMPLE 6

class A (int i, int j) {    // A declares two parameters
  unsigned int(i) id = j;   // width of id is i
                            // and the value must equal j
}
 
class B (int x, int y) extends A (x, y) { // parameters used in 
                                          // value expressions
  unsigned int(4) value1;
}
 
class C (int z) extends B (4, z) {   // constant and expression used
                                     // as parameter values
  unsigned int(4) value2;
}
 
C c(3);  // one parameter value provided

An example bitstream for this would be:

EXAMPLE 7

...
         // c = {   
0 0 1 1  //   id = 3, (width of id is 4, id must equal 3)
0 0 1 0  //   value1 = 2,
0 1 0 1  //   value2 = 5,
         // }
...

In the following example, class A declares one parameter i and class B declares one parameter j whilst class C declares two parameters x and y. The class C parameters are used in expressions as parameter values to both define its base class A and its member class B:

EXAMPLE 8

class A (int i) {   // A declares one parameter
  unsigned int(i) value1; // width of value1 is i
}
 
class B (int j) {   // B declares one parameter
  unsigned int(j) value2; // width of value2 is j
}
 
class C (int x, int y) extends A (x) { // one parameter value
                                       // used in expression
  B b(y); // one parameter value used in expression
}
 
C c(4, 2); // two constants used as parameter values

An example bitstream for this would be:

EXAMPLE 9

...
         // c = {   
0 1 0 1  //   value1 = 5,
         //   b = {
1 1      //     value2 = 3
         //   }
         // }
...

In the following example, class A declares one parameter i and class B declares one parameter j whilst class C declares no parameters. An expression is used during the definition of the class variable b (e.g. c.value3) and another expression is used during the declaration of class B (e.g. j * 2):

EXAMPLE 10

class A (int i) {           // A declares one parameter
  unsigned int(i) value1;   // width of value1 is i
}
 
class B (int j) extends A (j * 2) {  // parameter value j propagated
                                     // in an expression
  unsigned int(4) value2;
}
 
class C {
  unsigned int(4) value3;
}
 
C c;
B b(c.value3);  // expression used as parameter value

An example bitstream for this would be:

EXAMPLE 11

...
         // c = {   
0 0 0 1  //   value3 = 1
         // }
         // b = {
1 1      //   value1 = 3 (width of value1 is c.value3 * 2 = 2)
0 1 0 1  //   value2 = 5
         // }
...

Values are copied (including a deep copy of all values for nested class structures) when provided as parameter values to a class meaning that any changes to parameter values by the class making use of them are not visible externally. As an example:

EXAMPLE 12

class A {
  int(2) i;
}
 
class B (A a) {
  int(2) j;
  a.i++;
  int k = a.i;
}
 
A a;
// assume a.i == 2 here
 
B b(a);
// a.i == 2 here as changes within b to a.i were on a deep copy of a
// b.k == 3 here as b.k was initialised using the modified deep copy of a

An example bitstream for this would be:

EXAMPLE 13

...
         // a = {   
1 0      //   i = 2,
         // },
         // b = {
1 1      //   j = 3
         // }
         // a.i == 2 here
         // b.k == 3 here
...

## Arrays

### General

Parsed array variables shall be defined using square brackets. The array declaration is applicable to both elementary types and classes.

Rule A.1: Arrays

[legacy|reserved] [aligned[(modifier)]]element_type array_identifier[length] [=value];

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword aligned and its modifier are discussed in 7.7.2.

The element_type of an array shall be either a parsed type declaration (e.g. an elementary type with a length attribute, e.g. ‘int(2)’) or a defined class identifier.

The attribute length specifies the capacity of the array. The length value may be an expression. The length value shall be an integer value of 0 or greater. The maximum length value is identified as an unspecified constraint.

The value attribute shall be present only when the parsed array element values are to be constrained. The values should be initialized with an array of literal values. For example:

EXAMPLE 1

int(8) a[4] = [ 0..2, 1, 2, 3 ];

Array initialization for arrays where element_type is a defined class identifier is not supported.

In the following example, a is an array of 5 elements, each of which is represented using 4 bits in the bitstream and interpreted as an unsigned integer:

EXAMPLE 2

unsigned int(4) a[5];

An example bitstream for this would be:

EXAMPLE 3

...
         // a = [   
0 0 0 1  //   1,
0 0 1 0  //   2,
0 0 1 1  //   3,
0 1 0 0  //   4,
0 1 0 1  //   5
         // ]
...

In the following example, the length of c depends on the actual value of b:

EXAMPLE 4

int(10) b;
int(2) c[b];

An example bitstream for this would be:

EXAMPLE 5

...
0 0 0 0 0 0 0 0 1 0 // b = 2
                    // c = [
0 1                 //   1,
2 0                 //   2
                    // ]
...

Individual values of an array are accessed using the array element access operator as described in 5.11. For example:

EXAMPLE 6

int(4) a[5];
computed int b = a[0];   // access the value of the first
                         // element in array a
a[1] = 10;               // set the value of the second element in array a

NOTE	In the example above b is a computed variable as defined in Clause 8.

### Alignment

The keyword aligned indicates that the start of the array data is aligned on a byte boundary.

For example, an array where the first item is aligned on a byte boundary and the individual items are packed with no skipped bits for alignment:

EXAMPLE 1

aligned bit(5) foo[7];

An example bitstream for this would be:

EXAMPLE 2

...
<skip 0..7 bits> // aligned
                 // foo = [
0 0 0 0 1        //   1,
0 0 0 1 0        //   2,
0 0 0 1 1        //   3,
0 0 1 0 0        //   4,
0 0 1 0 1        //   5,
0 0 1 1 0        //   6,
0 0 1 1 1        //   7
                 // ]
...

An optional numeric modifier attribute may be used to signify alignment on boundaries other than byte. Allowed values are 8, 16, 32, 64, and 128. Any skipped bits due to alignment shall have the value ‘0’.

### Multi-dimensional arrays

Multi-dimensional arrays are also supported. The parsing order from the bitstream corresponds to scanning the array by incrementing first the right-most index of the array, then the second right-most, and so on.

Rule A.2: Multi-dimensional arrays

[legacy|reserved] [aligned[(modifier)]]element_type array_identifier[length][length]… [=value];

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword aligned and its modifier have the same meaning as in 7.7.2.

The grammar rule construct element_type has the same meaning as in 7.7.

The value attribute has the same meaning as in 7.7.1

In the following example, a is an array of 2 elements, each of which is represented as an array of 3 elements using 4 bits in the bitstream and interpreted as an unsigned integer.

EXAMPLE 1

unsigned int(4) a[2][3] = [[0..1, 2, 3], [4..5, 5..7, 6]];

An example bitstream for this would be:

EXAMPLE 2

...
                         // a = [
0 0 0 1 0 0 1 0 0 0 1 1  //   [ 1, 2, 3 ],
0 1 0 0 0 1 0 1 1 1 0 0  //   [ 4, 5, 6 ]
                         // ]
...

The maximum number of dimensions for an array is identified as an unspecified constraint.

Individual values of a multi-dimensional array are accessed using a sequence of array element access operators as described in 5.11 where the number of operators matches the dimensionality of the array. For example:

EXAMPLE 3

int(4) a[5][7];
computed int b = a[1][2]; // access the value in the 2nd row
                          // and 3rd column of array a
a[2][3] = 10;             // set the value in the 3rd row
                          // and 4th column of array a

### Partial arrays

In several situations, it is desirable to load the values of an array one by one, to check for a terminating or other condition. For this purpose, an extended array definition shall be used in which individual elements of the array may be accessed for population i.e. allowing the definition of dynamically sized sparse arrays.

Rule A.3: Partial arrays

[legacy|reserved] [aligned[(modifier)]]element_type array_identifier[[index]];

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword aligned and its modifier have the same meaning as in 7.7.2.

The grammar rule construct element_type has the same meaning as in 7.7.

Here index is the index of the element of the array that is defined. Index values shall start at the value ‘0’.  The index value may be an expression. Several such partial array definitions may appear in an SDL specification, but they shall all agree on the typespec declaration.

NOTE	a[5] specifies an array of five elements, whereas a[[5]] specifies the 6th element of an array which contains at least six elements.

Non-populated elements of a partial array shall not be accessed.

Multi-dimensional partial arrays are also supported.

Partial array initialization with literals is not supported.

Rule A.4: Partial multi-dimensional arrays

[legacy|reserved] [aligned[(modifier)]]element_type array_identifier[[length]][[length]]…;

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword aligned and its modifier have the same meaning as in 7.7.2.

The grammar rule construct element_type has the same meaning as in 7.7.

The following example indicates the element a(5, 3) of the array i.e. the element in the 6th row (row index 5) and the 4th column (column index 3):

EXAMPLE 1

int(4) a[[3]][[5]];

An example bitstream for this would be:

EXAMPLE 2

...
        // a = [
        //   [  ,  ,  ,   ],
        //   [  ,  ,  ,   ],
        //   [  ,  ,  ,   ],
        //   [  ,  ,  ,   ],
        //   [  ,  ,  ,   ],
0 1 0 1 //   [  ,  ,  , 5 ]
        // ]
...

The following example indicates the entire 6th row (row index 5) of the array which contains 3 elements each of which is a 4-bit integer value:

EXAMPLE 3

int(4) a[[5]][3];

An example bitstream for this would be:

EXAMPLE 4

...
                        // a = [
                        //   [  ,  ,   ],
                        //   [  ,  ,   ],
                        //   [  ,  ,   ],
                        //   [  ,  ,   ],
                        //   [  ,  ,   ],
0 0 0 1 0 0 1 1 0 1 0 1 //   [ 1, 3, 5 ]
                        // ]
...

The following example indicates the entire 4th column (column index 3) of the array which contains 5 rows:

EXAMPLE 5

int(4) a[5][[3]];

An example bitstream for this would be:

EXAMPLE 6

...
        // a = [
0 0 0 1 //   [  ,  ,  , 1 ],
0 0 1 0 //   [  ,  ,  , 2 ],
0 0 1 1 //   [  ,  ,  , 3 ],
0 1 0 0 //   [  ,  ,  , 4 ],
0 1 0 1 //   [  ,  ,  , 5 ]
        // ]
...

In the following example, the entries of the one-dimensional array wordLength are populated in a sparse manner within the for-loop (for-loops are presented in 9.3). On the subsequent line, these values are then accessed to define and populate the two-dimensional array words (again in a sparse manner for one of the dimensions):

EXAMPLE 7

unsigned int(8) wordCount;
computed int i;
for (i = 0; i < wordCount; i++) {
  // sparse array population
  unsigned int(8) wordLength[[i]];
  // first dimension sparse array population,
  // second dimension fully populated with a size defined by
  // wordLength[i]
  bit(8) words[[i]][wordLength[i]];
}

An example bitstream for this would be:

EXAMPLE 8

...
0 0 0 0 0 0 1 1 // wordCount = 3
0 0 0 0 0 0 1 0 // wordLength[0] = 2
                // words[0] = [
0 0 0 0 0 1 1 1 //   7,
0 0 0 0 1 0 0 0 //   8
                // ]
0 0 0 0 0 0 1 1 // wordLength[1] = 3
                // words[1] = [
0 0 0 0 0 0 0 1 //   1,
0 0 0 0 0 0 0 1 //   1,
0 0 0 0 0 0 0 1 //   1,
                // ]
0 0 0 0 0 0 1 0 // wordLength[2] = 1
                // words[2] = [
0 0 0 0 0 1 1 0 //   6
                // ]
...

### Implicit arrays

An array with an implicit length is an implicit array. It is indicated by an array definition without a length attribute. To limit the possible minimum and maximum implicit length of the array, an optional range attribute may be used.

Rule A.5: Implicit arrays

[legacy|reserved] [aligned[(modifier)]]class_identifier array_identifier[[range]];

The keyword legacy has the same meaning as in 6.2.7.

The keyword reserved has the same meaning as in 6.2.8.

The keyword aligned and its modifier have the same meaning as in 7.7.2.

The optional range attribute may be specified as min..max, inclusive of both bounds.

Multi-dimensional implicit arrays are not supported.

Implicit array initialization with literals is not supported.

When a series of polymorphic classes is present in the bitstream, it can be represented as an array of the same type as that of the base class.

For example, assume that a set of polymorphic classes is declared, derived from the base class Foo (which might or might not be abstract):

EXAMPLE 1

class Foo : bit(16) id = 0 {
  int(5) a;
}

For an array of such objects, it is possible to implicitly determine the length by examining the validity of the class_id of the class. Objects are inserted in the array as long as the class_id can be properly resolved to one of the class_id values defined in the base (if not abstract) or its derived classes.

NOTE	When an implicit array is followed by further parsable data, there is a possibility that the following encoded bits match the value of a class class_id. In this case, it is the responsibility of the SDL definition author to take care of preventing this possible ambiguity in the bitstream by a mechanism such as emulation prevention bytes or reserved value markers.

In the following example, the number of elements is implicitly obtained via class_idresolution:

EXAMPLE 2

Foo f[];   // length implicitly obtained via class_id resolution

In the following example, f has at least 1 and at most 255 elements:

EXAMPLE 3

Foo f[1 .. 255];   // at least 1, at most 255 elements

# Computed variables

## General

To accommodate complex syntactic constructs, in which context information cannot be directly obtained from the bitstream but only as a result of a computation, computed variables are defined.

Computed variables may be used in expressions and conditions in the same way as parsed variables. In the following example, the number of non-zero elements of an array is computed.

EXAMPLE

int(4) myArray[100]; // parsed array of 100 integers, each 4 bits in size
computed int i;      // computed integer variable
computed int n = 0;  // computed integer variable
for (i = 0; i < 100; i++) {
  if (myArray[i] != 0) {
    n++;
  }
}
 
// parse as many coefficients as there are non-zero elements in myArray
int(16) coefficients[n];

## Elementary data types

Computed elementary data type variable definitions are distinguished from parsed variable definitions due to the presence of a computed keyword and the absence of a length attribute.

Rule CV.1: Elementary data types

computed[const] computed_type identifier[=value];

The keyword const has the same meaning as in 6.2.5.

For example, an unsigned integer computed variable is represented as:

EXAMPLE

computed unsigned int foo;

The result of accessing the value of a computed variable which has not yet been initialised with a value via the assignment operator (e.g. ‘=’) is not defined by the SDL and can lead to undefined behavior.

### Type

The grammar rule construct computed_type comprises the following: int for signed integer, unsigned int for unsigned integer, float for floating point, and bool for logic values.

### Value

The value attribute shall be present when the variable is initialised with a value. A variable or expression may be used when defining a value.

For example:

EXAMPLE

computed unsigned int initial = 10;
computed unsigned int foo = initial;

Unlike a parsed variable, a range of values shall not be used when declaring a computed variable.

Refer to 5.12.3 for a discussion on the limits of value.

Refer to 5.12.4 for a discussion on the coercion of value.

## Arrays

Computed array variables shall be defined using square brackets. The array definition for computed variables is only applicable to elementary types.

Rule CV.2: Arrays

computed [const] computed_type array_identifier[length] [=value];

Constant arrays shall be defined using the keyword const. A const variable array element value shall not be modified.

The attribute length has the same meaning as in 7.7 and the grammar rule computed_type is as previously defined in 8.2.1.

The value attribute shall be when the initial array elements values are to be defined. The values should be initialized with an array of literal values. For example:

EXAMPLE 1

computed int a[4] = [ 0, 1, 2, 3 ];

In the following example, a is an array of 5 elements, each of which is an unsigned integer:

EXAMPLE 2

computed unsigned int a[5];

In the following example, the length of c depends on the value of the parsed variable b:

EXAMPLE 3

int(10) b;
computed int c[b];

Individual values of an array are accessed using the array element access operator as described in 5.11. For example:

EXAMPLE 4

computed int a[5];
a[0] = 9;                // set the value of the 1st element in array a
computed int b = a[0];   // access the value of the 1st element
                         // in array a

## Multi-dimensional arrays

Computed multi-dimensional arrays are also supported.

Rule CV.3: Multi-dimensional arrays

computed computed_type array_identifier[length][length]… [=value];

The attribute length has the same meaning as in 7.7, the grammar rule construct computed_type is as previously defined in 8.2.1 and the value attribute is as previously defined in 8.3 .

In the following example, a is an array of 2 rows, each of which is represented as an array of 3 unsigned integer values:

EXAMPLE 1

computed unsigned int a[2][3] = [[0, 1, 2], [3, 4, 5]];

The maximum number of dimensions for an array is identified as an unspecified constraint.

Individual values of a multi-dimensional array are accessed using a sequence of array element access operators as described in 5.11 where the number of operators matches the dimensionality of the array. For example:

EXAMPLE 2

computed unsigned int a[1][2];
computed int b = a[1][2]; // access the value in the 2nd row and
                          // 3rd column of array a
a[2][3] = 10;             // set the value in the 3rd row and
                          // 4th column of array a

# Syntactic flow control

## General

Syntactic flow control provides constructs that allow conditional parsing, depending on context, as well as repetitive parsing.

## Conditionals

The if-then-else construct is used for testing conditions.

Rule FC.1: Flow control using if-then-else

if (condition) [{]
  …
[}] [else if (condition) [{]
  …
[}]] [else [{]
  …
[}]]

Each condition is evaluated sequentially in order until a condition evaluates to a logic value of true. When this occurs, the conditional clause associated with it is evaluated and then the remainder of the construct shall be skipped. A final else statement without a condition may be used for a default case where none of the explicit conditions are satisfied.

Each conditional clause shall have at least one statement. If a conditional clause contains a single statement, then usage of braces ‘{’ and ‘}’ is optional (they are implicit), but if multiple statements are present then braces shall be present. For example:

EXAMPLE 1

if (condition)
  int(8) a;
else {
  int(8) b;
  int(8) c;
}

Because an else statement is optional, there is potential for ambiguity when an else is omitted within a nested if-then-else construct. In this case, the else statement shall be associated with the closest previous if statement that did not have an if. For example:

EXAMPLE 2

if (condition1)
  if (condition2)
  int(8) a;
else // associated with "if (condition2)"
  int(8) b;

NOTE 1	Usage of the keyword break to exit a clause within an if-then-else construct is not supported.

In the following example, the presence of the parsed variable bar is determined by the bar_flag:

EXAMPLE 3

class conditional_class1 {
  unsigned int(3) foo;
  bit(1) bar_flag;
  if (bar_flag) {
    unsigned int(8) bar;
  }
  unsigned int(4) more_foo;
}

conditional_class1 myExample1;

An example bitstream for this would be:

EXAMPLE 4

...
                // myExample1 = {
0 0 1           //   foo = 1,
1               //   bar_flag = 1,
0 0 0 1 0 0 0 0 //   bar = 16,
0 1 0 0         //   more_foo = 4
                // }
...

NOTE 2	The use of bar_flag necessitates its definition before the conditional is encountered.

A parsed variable may be defined more than once across conditional branches if the defined type is identical (the length attribute may differ). In the following example, two different representations for the parsed variable bar are defined, depending on the value of bar_flag.

EXAMPLE 5

class conditional_class2 {
  unsigned int(3) foo;
  bit(1) bar_flag;
  if (bar_flag) {
     unsigned int(8) bar;
  }
  else {
    unsigned int(4) bar;
    int(8) optional_foo;
  }
  unsigned int(4) more_foo;
}

conditional_class2 myExample2;

An example bitstream for this would be:

EXAMPLE 6

...
                // myExample2 = {
0 0 1           //   foo = 1,
0               //   bar_flag = 0,
0 1 0 0         //   bar = 4,
0 0 0 0 0 1 1 1 //   optional_foo = 7
0 1 0 0         //   more_foo = 4
                // }
...

As presented in 5.19.2, parsed variables defined within a class declaration, regardless of conditional branch scopes have class scope, i.e. they are available as class member variables. Using the above class declaration, the following example clarifies this further:

EXAMPLE 7

conditional_class2 c;
 
// c.foo, c.bar_flag, c.bar, c.optional_foo and c.more_foo
// are always all accessible
// if c.bar_flag == 1 then:
//   c.bar is an 8-bit unsigned integer with a value
//   parsed from the bitstream
//   c.optional_foo is an 8-bit integer with no parsed value
//   and a default value of 0
// if c.bar_flag == 0 then:
//   c.bar is a 4-bit unsigned integer with a value parsed
//   from the bitstream
//   c.optional_foo is an 8-bit integer with a value parsed
//   from the bitstream

The if-then-else clauses shall not define duplicate class member variables being declared simultaneously. For example, the following is an invalid SDL specification as it allows for the member variable foo to be declared more than once simultaneously:

EXAMPLE 8

class invalid_class {
  int(2) bar;
  if (bar > 0) {
    unsigned int(8) foo;   // first declaration of foo if bar > 0
  }
  if (bar > 1) {
    unsigned int(4) foo; // second declaration of foo which will conflict
                         // with the first declaration if bar > 1
   }
}

To facilitate cascades of if-then-else constructs, the switch syntax may be used.

Rule FC.2: Flow control using switch

switch (condition) {
  [caselabel:
    […]      
    [break;]]
  …
  [default:
    […]]
}

The condition in a switch statement shall evaluate to an integer value. Each label shall be an integer constant expression and each label in the same switch statement shall be unique.

EXAMPLE 9

unsigned int(32) code;
switch (code) {
  case 0:
    Foo f;
    break;
 
  case 1:
    Bar b;
    break;
 
  default:
    Moo m;
}

If the break keyword is not encountered, then the flow shall continue onto the next labelled clause. The case and the default clauses may be optionally placed in braces ‘{’ and ‘}’ and this has no effect on the conditional flow. For example:

EXAMPLE 10

unsigned int(32) code;
switch (code) {
  case 0:
    // flow through to case 1
 
  case 1: {
      Foo f1;
    } // brace scope has no effect
    Foo f2;
    // flow through to case 2
 
  case 2: {
      Bar b1;
      Bar b2;
      break;
    } // brace scope has no effect
    // no flow through to default
  default:
    Moo m;
}

Unlike other languages, variables can be defined within switch clauses without defining a new scope. However computed variables may not be defined more than once without introducing a new scope. For example:

EXAMPLE 11

unsigned int(32) code;
switch (code) {
  case 0:
    computed int i; // legal
    // flow through to case 1
  case 1: {
    computed int j; // legal
    // illegal to redefine i at this point
    {
        computed int i; // legal redefinition of i in new scope
    }
}

As with if-then-else construct, parsed variables may be defined more than once across conditional branches in a switch statement if the defined type is identical (the length attribute may differ). Additionally, switch clauses shall not define duplicate class member variables being declared simultaneously. For example, the following is invalid:

EXAMPLE 12

unsigned int(32) code;
switch (code) {
  case 0:
    Foo f1;
    // flow through to case 1
  case 1: {
    Foo f1; // invalid duplicate definition
}

## Loops

Context-sensitive constructs are also provided for iterative parsing. These constructs imply the repetitive use of the same syntax to parse the bitstream, until some condition is met.

In the for-loop syntax, expression1, expression2 and expression3 each constitute a single statement. If specified, expression1 shall be either a computed variable definition with an assigned value or a value assignment and shall be evaluated prior to starting the repetitions. Then if expression2 is not specified, or it is specified and it evaluates to a logic value of true, the statements within the scope of the for-loop shall be processed, followed by the evaluated of expression3 if it is specified. The process shall repeat until expression2 evaluates to a logic value of false.

Rule FC.3: Flow control using for

for ([expression1]; [expression2]; [expression3]) [{]
  …
[}]

The body of the for-loop shall specify at least one statement. If a single statement is specified, then usage of braces ‘{’ and ‘}’ is optional (they are implicit), but if multiple statements are present then braces shall be present. For example:

EXAMPLE 1

computed int i;
computed int n = 0;
 
for (i = 0; i < 10; i++) {
  n++;
  n--;
}
// n == 0 here
for (i = 0; i < 10; i++)
  n++;
  n--; // this is outside of the for loop
// n == 9 here

NOTE 1	Usage of the keyword break to exit a for-loop construct is not supported.

Computed variables may be defined within the body of a loop as they are block scoped. The following example shows a computed variable defined within the scope of the for-loop construct:

EXAMPLE 2

computed int n = 0;
 
for (computed int i = 0; i < 10; i++) {
  n++;
  // i is accessible here
}
// i is not accessible here

Elementary data type parsed variables, class variables, string parsed variables and array parsed variables may be defined within the body of a loop, however it is considered an error condition if the parsing control flow results in the redefinition of such class scoped member elements. For example:

EXAMPLE 3

class Looped {
  int(4) count;
  computed int i;
  for (i = 0; i < count; i++) {
    int(8) offset;    // if count > 1 an error condition will occur as
                      // offset is redefined
    int(4) vertex[2]; // if count > 1 an error condition will occur as
                      // vertex is redefined
  }
}
 
Looped looped;

Such a scenario is considered an invalid SDL specification.

If such loop-based variable population is required, it is preferred to use partial arrays. Partial array parsed variables may be defined within the body of a loop as effectively these are accessing specific elements within a single defined array. For example:

EXAMPLE 4

class Looped {
  int(4) count;
  computed int i;
  for (i = 0; i < count; i++) {
    int(8) offsets[[i]];
    int(4) vertices[[i]][2];
  }
}
 
Looped looped;
 
// here, looped.offsets is an array with a size of looped.count
// here, looped.vertices is a 2-dimensional array with sizes
// of looped.count,2

An example bitstream for this would be:

EXAMPLE 5

...
                // looped = {
0 0 1 0         //   count = 2,
0 0 0 0 0 1 1 0 //   offsets[0] = 6
0 0 0 1 0 0 1 0 //   vertices[0] = [1, 2]
0 0 0 0 0 0 1 0 //   offsets[1] = 2
0 0 1 1 0 1 0 0 //   vertices[1] = [3, 4]
                // }
 
// here looped.offsets == [6, 2]
// here looped.vertices == [[1, 2], [3, 4]]
...

If there is no need to parse other items in the loop nor perform further logic, a simpler approach for the above scenario is to declare the class Looped as follows:

EXAMPLE 6

class Looped {
  int(4) count;
  int(8) offsets[count];
  int(4) vertices[count][2];
}

An example bitstream for this would be:

EXAMPLE 7

...
                // looped = {
0 0 1 0         //   count = 2,
0 0 0 0 0 1 1 0 //   offsets[0] = 6
0 0 0 0 0 0 1 0 //   offsets[1] = 2
0 0 0 1 0 0 1 0 //   vertices[0] = [1, 2]
0 0 1 1 0 1 0 0 //   vertices[1] = [3, 4]
                // }
 
// here looped.offsets == [6, 2]
// here looped.vertices == [[1, 2], [3, 4]]
...

In the following example, as there is a conditional clause there is no possibility that the variables will be redefined and therefore no possibility this will result in an error scenario:

EXAMPLE 8

class Looped {
  int(4) count;
  computed int i;
  for (i = 0; i < count; i++) {
     if (i == 0) {
       int(8) offset0;
     }
     if (i == 1) {
       int(8) offset1;
     }
  }
}
 
unsigned int(8) count; // the only value values for count here are 0 or 1
                       // anything else would/should cause
                       // parsing to fail...
 
for (int i = 0; i < count; i++) {
  int(8) offset1;      // ... as this can only be declared 0 or 1 times
}
if (i == 0) {
  int(8) offset2;
}
 
offset2 == 0

In the do-while-loop syntax, the block of statements shall be evaluated at least once and until condition evaluates to a logical value of false.

Rule FC.4: Flow control using do

do { 
  …
} while(condition);

The body of the do-while-loop shall specify at least one statement. Braces ‘{’ and ‘}’ shall always be present.

EXAMPLE 9

computed int i = 10;
do {
  i--;
} while (i > 0);

NOTE 2	Usage of the keyword break to exit a do-while-loop construct is not supported.

In the while-loop syntax, the loop shall be evaluated zero or more times, as long as condition evaluates to a logic value of true.

Rule FC.5: Flow control using while

while (condition)
  …

The body of the while-loop shall specify at least one statement. If a single statement is specified, then usage of braces ‘{’ and ‘}’ is optional (they are implicit), but if multiple statements are present then braces shall be present. For example:

EXAMPLE 10

computed int i = 10;
 
while (i > 0) {
  i++;
  i = i - 2;
}
// i < 0 here
 
i = 10
while (i > 0)
  i++;
  i = i - 2; // this is outside of the while loop and is never reached

NOTE 3	Usage of the keyword break to exit a while-loop construct is not supported.

(informative)

SDL user guide

Getting started

In this annex, some basic concepts are presented to describe a binary structure using SDL. For illustration purposed, the example of the MPEG-2 transport packet is used to demonstrate a possible SDL specification to describe it. The binary structure of a transport packet according to ISO/IEC 13818-1 is described in Table A.1.

Table A.1 — ISO/IEC 13818-1 transport packet binary structure

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| transport_packet(){ |  |  |
| sync_byte | 8 | bslbf |
| transport_error_indicator | 1 | bslbf |
| payload_unit_start_indicator | 1 | bslbf |
| transport_priority | 1 | bslbf |
| PID | 13 | uimsbf |
| transport_scrambling_control | 2 | bslbf |
| adaptation_field_control | 2 | bslbf |
| continuity_counter | 4 | uimsbf |
| if(adaptation_field_control = = '10' ¦¦ adaptation_field_control = = '11'){ |  |  |
| adaptation_field() |  |  |
| } |  |  |
| if(adaptation_field_control = = '01' ¦¦ adaptation_field_control = = '11') { |  |  |
| for (i = 0; i < N; i++){ |  |  |
| data_byte | 8 | bslbf |
| } |  |  |
| } |  |  |
| } |  |  |

As shown in Table A.1, a “transport_packet” is declared as a logical structure which can be encoded in a bitstream. A convenient way to represent the same logical structure in SDL is to declare a class. The minimum declaration for a class is an identifier and an empty body:

class transport_packet {
  // to be defined
}

The body of the class corresponds to the representation of the sequence of bits to be described. Based on Table A.1, it can be observed that the first byte of a transport packet is the “sync_byte”. Let be transport_packet the name of a new class containing this first byte:

class transport_packet {
  unsigned int(8)   sync_byte;
}

This first variable is defined as unsigned integer and its identifier is sync_byte. Since its length in the bitstream is one byte, the number 8 is passed in parenthesis as the length attribute of this variable. As the ISO/IEC 13818-1 standard requires the “sync_byte” to be equal to 0x47, it can be further restricted that this variable will always be equal to this value:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
}

The “sync_byte” is followed by three 1-bit flag elements: “transport_error_indicator”, “payload_unit_start_indicator”, and “transport_priority”. SDL syntax allows such raw binary data elements to be represented using the keyword bit (alternatively, unsigned int(1) is also possible):

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
}

Following the flags is the packet identifier (“PID”) which is a 13-bit field. Since this variable represents a number (PID has well-defined hexadecimal values), it can be defined as an unsigned integer:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
}

After the PID comes two 2-bit control elements whose values are defined as bit strings, e.g. “00 = Not scrambled”, “01 = User-defined”, etc. Those variables can also be defined as bit variables:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
}

At this point, the non-variable part of the transport packet is declared. As shown in Table A.1, the remainder of the packet depends on the value of the “adaptation_field_control” element. To represent this, the use of an if construct is appropriate:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
 
  if (adaptation_field_control == 0b10 ||
    adaptation_field_control == 0b11){
    // to be defined
  }
 
 if (adaptation_field_control == 0b01 ||
   adaptation_field_control == 0b11) {
   // to be defined
  }
}

As can be seen, SDL allows to test the value of a variable using the relational equal == operator. Here since they are bit variables, the binary value literal representation with prefix ‘0b’ is used. Finally, the logical combination of tests is achieved using the logical OR operator ‘||’.

All that remains to specify is the two clauses of the if constructs. In the first one, a logical structure “adaptation_field()” is present. To represent this, a previously declared class adaptation_field is assumed and a variable of this type is declared:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
 
  if (adaptation_field_control == 0b10 ||
    adaptation_field_control == 0b11) {
    adaptation_field data;
  }
 
  if (adaptation_field_control == 0b01 ||
    adaptation_field_control == 0b11) {
    // to be defined
   }
}

NOTE	It is not needed to specify how large this data field is as long as the declaration of the adaptation_field class determines it.

To complete the transport packet, it is needed specify the clause of the second if construct. The ISO/IEC 13818-1 standard defines this part of the transport packet as a loop over N-1 elements corresponding to a sequence of N-1 bytes. The SDL author can be tempted to use a for-loop:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
 
  if (adaptation_field_control == 0b10 ||
    adaptation_field_control == 0b11) {
    adaptation_field data;
  }
 
  if (adaptation_field_control == 0b01 ||
    adaptation_field_control == 0b11) {
    for (int i = 0; i < N; i++) {
      bit(8) data_byte;
    }
  }
}

However, this declaration is an invalid SDL specification since the same parsed variable definition data_byte is repeated multiple times. To specify such a sequence of bytes, it is possible to declare an array of N-1 elements instead of using a for-loop:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
 
  if (adaptation_field_control == 0b10 ||
    adaptation_field_control == 0b11) {
    adaptation_field data;
  }
 
  if (adaptation_field_control == 0b01 ||
    adaptation_field_control == 0b11) {
    bit(8) data_byte[N-1];
  }
}

The SDL class declaration of transport_packet is almost complete. There is still an unknown variable, N, in use. The ISO/IEC 13818-1 standard defines N as “specified by 184 minus the number of bytes in the adaptation_field()”. The variable N is thus not written in the bitstream but computed based on the context of the class at this point of the binary structure. For this, SDL provides computed variables which can hold any value for the purpose of declaring SDL elements. Let be N a computed variable defined as an unsigned integer and let be 184 its initial value:

class transport_packet {
  unsigned int(8)   sync_byte = 0x47;
  bit(1)            transport_error_indicator;
  bit(1)            payload_unit_start_indicator;
  bit(1)            transport_priority;
  unsigned int(13)  PID;
  bit(2)            transport_scrambling_control;
  bit(2)            adaptation_field_control;
 
  computed unsigned int N = 184;
 
  if (adaptation_field_control == 0b10 ||
    adaptation_field_control == 0b11) {
    adaptation_field data;
    N = N – 1 – data.adaptation_field_length;
  }
 
  if (adaptation_field_control == 0b01 ||
    adaptation_field_control == 0b11) {
    bit(8) data_byte[N];
  }
}

In the class adaption_field, it is assumed that the variable adaptation_field_length exists which is defined in the ISO/IEC 13818-1 standard as “an 8-bit field specifying the number of bytes in the adaptation_field immediately following the adaptation_field_length”. Therefore, this variable can be accessed with the SDL class member access operator ‘.’ and calculate the size of the array.

The syntax of the MPEG-2 transport packet as an SDL class declaration is now complete. To finalise the SDL specification, the bitstream can be defined as an endless sequence of transport_packet classes.