# C:\Users\shinji_w\Desktop\ISO-IEC_23008-1_(E).docFINAL DRAFT INTERNATIONAL STANDARD© ISO/IEC 2015 – All rights reservedISO/IEC  23001-11 63Part 11: Green MetadataInformation technology — Green ViÉlément introductif — Élément central — Partie 1: Titre de la partieInformation technology — MPEG Systems Technologies  — Part 11: Energy-Efficient Media Consumption (Green Metadata)E2013-04-26(40) EnquiryISO/IECISO/IEC J   International Standard 2013ISO/IEC 23008-ISO/IEC 23008-1Information technology11291 2見出し 2見出し 1    02 STD Version 2.1c240   4            ISO/IEC JTC 1/SC 29

Date: 2026-02-20

ISO/IEC  23001-11

ISO/IEC JTC 1/SC 29/WG 11

Secretariat:

Information technology — MPEG Systems Technologies  — Part 11: Energy-Efficient Media Consumption (Green Metadata)

Élément introductif — Élément central — Partie 1: Titre de la partie

Contents	Page

Foreword	8

Introduction	9

1	Scope	1

2	Normative references	1

3	Terms, definitions, symbols, abbreviated terms	1

3.1	Terms and definitions	1

3.2	Symbols and abbreviated terms	3

4	Conventions	4

4.1	Arithmetic operators	4

4.2	Relational operators	4

4.3	Bit-wise operators	5

4.4	Assignment operators	5

4.5	Range notation	5

4.6	Mathematical functions	5

4.7	Specification of syntax functions and descriptors	6

5	Functional architecture	6

5.1	Description of the functional architecture	6

5.2	Definition of components in the functional architecture	7

6	Decoder power reduction	8

6.1	General	8

6.2	Complexity metrics for decoder-power reduction	8

6.3	Interactive signalling for remote decoder-power reduction	40

7	Display power reduction using display adaptation	45

7.1	General	45

7.2	Syntax	45

7.3	Signalling	49

7.4	Semantics	51

8	Energy-efficient media selection	68

8.1	General	68

8.2	Syntax	68

8.3	Signalling	68

8.4	Semantics	70

9	Metrics for quality recovery after low-power encoding	71

9.1	General	71

9.2	Syntax	71

9.3	Signalling	71

9.4	Semantics	72

10	Conformance and reference software	74

Annex A (normative)  Supplemental Enhancement Information (SEI) syntax	75

A.1	Syntax and semantics of green metadata SEI message carried in AVC NAL units	75

A.1.1	Syntax	75

A.1.2	Semantics	77

A.2	Syntax and semantics of green metadata SEI message carried in HEVC NAL units	77

A.2.1	Syntax	77

A.2.2	Semantics	80

A.3	Syntax and semantics of green metadata SEI message carried in VVC NAL units	80

A.3.1	Syntax	81

A.3.2	Semantics	84

Annex B (informative)  Implementation guidelines for the usage of green metadata	85

B.1	Codec dynamic voltage frequency scaling for decoder-power reduction	85

B.1.1	General	85

B.1.2	Derivation of the complexity metrics	85

B.1.2.1	Deriving the worst-case, largest value for NmaxNumSixTapFiltPic(i)	85

B.1.2.2	Deriving the worst-case, largest value for NmaxAlphaPointDbfsPic(i)	88

B.1.3	Example usage of C-DVFS metadata	90

B.2	Display adaptation	92

B.2.1	General	92

B.2.2	Example usage of display-adaptation metadata	92

B.2.2.1	Example usage of display-adaptation metadata for contrast enhancement	92

B.2.2.2	Preventing flicker arising from control latency	94

B.2.2.3	Metadata for DA on displays with control-frequency limitations	94

B.2.2.4	DA metadata to prevent flicker from large variations	95

B.3	Void 	97

B.4	Interactive signalling for remote decoder-power reduction	118

B.4.1	General	118

B.4.2	Decoding operations reduction request computation and transmission	118

B.4.3	Use of decoding operations reduction request (DOR-Req) and decoding operation response (DOR-Resp)	119

B.5	Cross-segment decoding for quality recovery after low-power encoding	121

B.5.1	General	121

B.5.2	Green metadata Usage	121

B.6	Interactive signalling for display power reduction through the use of attenuation maps	123

B.6.1	General	123

B.6.2	Display attenuation map power reduction request (DAMPR-Req) and display attenuation map power reduction response (DAMPR-Resp).	123

B.6.3	Use of the display attenuation map power reduction request	124

B.6.4	Display power reduction attenuated video request (DPRAV-Req) and display power reduction attenuated video response (DPRAV-Resp).	124

B.6.5 Use of the display power reduction attenuated video request	127

Annex C (normative)  Conformance and reference software	129

C.1	Complexity metrics for decoder-power reduction	129

C.1.1	Conformance test vectors	129

C.1.2	Reference software	130

C.2	Void	130

C.3	Void	131

C.4	Metrics for quality recovery after low-power encoding	132

C.4.1	Conformance test vectors	132

C.4.2	Reference software	133

Annex D (informative)  Objective distortion metrics	134

D.1	General	134

D.2	PSNR	134

D.3	wPSNR	134

D.4	WS-PSNR	136

D.4.1	WS-PSNR calculation for equi-rectangular projection (ERP) format with optional padding	136

D.4.2	WS-PSNR calculation for cubemap projection (CMP) and hemisphere cubemap projection (HCMP) formats	137

D.5	SSIM	137

Bibliography	139

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives or www.iec.ch/members_experts/refdocs).

ISO and IEC draw attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO and IEC take no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO and IEC had not received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents and https://patents.iec.ch. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see www.iso.org/iso/foreword.html. In the IEC, see www.iec.ch/understanding-standards.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO/IEC 23001 series can be found on the ISO and IEC websites.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html and www.iec.ch/national-committees.

Introduction

This document specifies the metadata (green metadata) that facilitates reduction of energy usage during media consumption as follows:

—	the format of the metadata that enables reduced decoder power consumption;

—	the format of the metadata that enables reduced display power consumption;

—	the format of the metadata that enables media selection for joint decoder and display power reduction;

—	the format of the metadata that enables quality recovery after low-power encoding.

This metadata facilitates reduced energy usage during media consumption without any degradation in the quality of experience (QoE). However, it is also possible to use this metadata to get larger energy savings, but at the expense of some QoE degradation.

The metadata for energy-efficient decoding specifies two sets of information: complexity metrics (CM) metadata and decoding operation reduction request (DOR-Req) and response (DOR-Resp) metadata. A decoder uses CM metadata to vary operating frequency and thus reduce decoder power consumption. In a point-to-point video conferencing application, the remote encoder uses the DOR-Req metadata to modify the decoding complexity of the bitstream and thus reduce local decoder power consumption. The remote encoder uses the DOR-Resp metadata to acknowledge for the request and indicate how it has decided to answer.

The metadata for energy-efficient encoding specifies quality metrics that are used by a decoder to reduce the quality loss from low-power encoding.

The metadata for energy-efficient presentation specifies Attenuation Map Information (AMI) metadata, RGB-component statistics and quality levels. A presentation subsystem uses this metadata to reduce power by modifying the content based on attenuation map and/or adjusting display parameters, based on the statistics, to provide a desired quality level from those provided in the metadata.

In a point-to-point video conferencing application, two types of interactive signalling mechanisms exist to reduce the energy consumption of the display. In a first type, a decoder can use the display attenuation map power reduction request (DAMPR-Req) message to request for transmission of display attenuation maps, that will be applied to the decoded content, to adapt the amount of light emitted by the display,  thus reducing the display energy consumption. The remote encoder uses the display attenuation map power reduction response (DAMPR-Resp) message, to acknowledge reception of the request and to indicate how it decides to answer. In a second type, a decoder can use the display power reduction attenuated video request (DPRAV-Req), to request the remote encoder to generate a given display attenuation map and apply it on the base video to generate an attenuated video. This attenuated video shall then be encoded and transmitted up to the decoder. The remote encoder uses the display power reduction attenuated video response (DPRAV-Resp) message, to acknowledge reception of the request and to indicate how it decides to answer.

The metadata for energy-efficient media selection specifies DOR-Req parameters, RGB-component statistics and quality levels. The client in an adaptive streaming session uses this metadata to determine decoder and display power-saving characteristics of available video representations and to select the representation with the optimal quality for a given power-saving.

Information technology — MPEG Systems Technologies  — Part 11: Energy-Efficient Media Consumption (Green Metadata)

# Scope

This document specifies metadata for energy-efficient decoding, encoding, presentation, and selection of media.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 14496-10, Information technology — Coding of audio-visual objects — Part 10: Advanced Video Coding

ISO/IEC 23008-2, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding

ISO/IEC 23009-1, Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats

ISO/IEC 23090-3, Information technology — Coded representation of immersive media — Part 3: Versatile video coding

# Terms, definitions, symbols, abbreviated terms

## Terms and definitions

For the purposes of this document, the terms and definitions given in ISO/IEC 14496-10, ISO/IEC 23008-2, ISO/IEC 23009-1, ISO/IEC 23090-3 and the following apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at https://www.iso.org/obp

—	IEC Electropedia: available at https://www.electropedia.org/

3.1.1

alpha-point deblocking instance

APDI

single filtering operation that produces either a single, filtered output p'0 or a single, filtered output q'0, where p'0 and q'0 are filtered samples across a 4x4 block edge

3.1.2

deblocking filtering instance

single filtering operation that produces either a single, filtered output p' or a single, filtered output q', where p' and q' are filtered samples across an 8x8 and 4x4 block edge for HEVC and VVC, respectively

3.1.3

decoding process

process that reads a bitstream and derives decoded pictures from it

Note 1 to entry: The decoding process is specified in ISO/IEC 14496-10, ISO/IEC 23008-2 or ISO/IEC 23090-3.

3.1.4

encoding process

process that produces a bitstream

Note 1 to entry: The bitstream shall conform to ISO/IEC 14496-10, ISO/IEC 23008-2 or ISO/IEC 23090-3.

3.1.5

no-quality-loss operating point

NQLOP

metadata-enabled operating point associated with the largest display-power reduction that can be achieved without any quality loss (infinite PSNR)

3.1.6

non-zero block

block containing at least one non-zero transform coefficient

3.1.7

peak signal

maximum permissible RGB component in a reconstructed frame

Note 1 to entry: For N-bit video, peak signal is (2N – 1).

3.1.8

period

interval over which complexity-metrics metadata are applicable

3.1.9

pixel

smallest addressable element in an all-points addressable display device

3.1.10

reconstructed frames

frames obtained after applying RGB colour-space conversion and cropping to the specific decoded picture or pictures for which display power-reduction metadata are applicable

3.1.11

RGB colour space

colour space based on the red, green, and blue colour primaries

3.1.12

RGB component

single sample representing one of the three primary colours of the RGB colour space [3.1.11]

3.1.13

six-tap filtering

STF

single application of the 6-tap filter to generate a single filtered sample for fractional positions using the samples at integer-sample positions

## Symbols and abbreviated terms

| + | Addition |
| --- | --- |
| - | subtraction (as a two-argument operator) or negation (as a unary prefix operator) |
| * | Multiplication |
| / | integer division with truncation of the result toward zero. For example, 7 / 4 and −7 / −4 are truncated to 1 and −7 / 4 and 7 / −4 are truncated to −1 |
| ÷ | division in mathematical equations where no truncation or rounding is intended |

| AMI APDI | attenuation map information alpha-point deblocking instance |
| --- | --- |
| ASIC | application specific integrated circuit |
| AVC | advanced video coding – ISO/IEC 14496-10 |
| CM | complexity metric |
| CMOS | complementary metal oxide semiconductor |
| CMP | cubemap projection format |
| CPU | central processing Unit |
| DASH | dynamic adaptive streaming over HTTP |
| DA | display adaptation |
| DAMPR-Req | display attenuation map power reduction request |
| DAMPR-Resp | display attenuation map power reduction response |
| DPRAV-Req | display power reduction attenuated video request |
| DPRAV-Resp | display power reduction attenuated video response |
| DVFS | dynamic voltage frequency scaling |
| ERP | equi-rectangular projection format |
| Fps | frames per second |
| FS | fresh start |
| GP | good picture |
| HEVC | high efficiency video coding – ISO/IEC 23008-2 |
| HCMP | hemisphere cubemap projection format |
| Mbps | mega bits per second |
| MPD | media presentation description |
| MSD | mean square difference |
| MV | motion vector |
| NQLOP | no-quality-loss operating point |
| picAMI | attenuation map sample values coded in auxiliary pictures of type AUX_ALPHA |
| PSNR | peak signal-to-noise ratio |
| QoE | quality of experience |
| RBLL | remaining battery life level |
| RGB | red, green, blue |
| SEI | supplemental enhancement information |
| SP | start picture |
| STF | six-tap filtering |
| SSIM | structural similarity index measure |
| VVC | versatile video coding – ISO/IEC 23090-3 |
| XSD | cross-segment decoding |
| wPSNR | weighted peak signal-to-noise ratio |
| WS-PSNR | weighted to spherically uniform peak signal-to-noise ratio |

# Conventions

## Arithmetic operators

| xy | exponentiation |
| --- | --- |
| x/y | division where no truncation or rounding is intended |
|  | division where no truncation or rounding is intended |
| i = xyf( i ) | summation of with i taking all integer values from x up to and including y |
| p in Byf( p ) | summation of f(p) with p taking all integer location values in a block B in a picture |
| x % y | Modulus. Remainder of x divided by y, defined only for integers x and y with x >= 0 and y > 0 |

## Relational operators

>	greater than

>=	greater than or equal to

<	less than

<=	less than or equal to

= =	equal to

!=	not equal to

When a relational operator is applied to a syntax element or variable that has been assigned the value"na" (not applicable), the value "na" is treated as a distinct value for the syntax element or variable. The value "na" is considered not to be equal to any other value.

## Bit-wise operators

x  >>  y	arithmetic right shift of a two's complement integer representation of x by y binary digits

This function is defined only for non-negative integer values of y. Bits shifted into the most significant bits (MSBs) as a result of the right shift have a value equal to the MSB of x prior to the shift operation.

x  <<  y	arithmetic left shift of a two's complement integer representation of x by y binary digits

This function is defined only for non-negative integer values of y. Bits shifted into the least significant bits (LSBs) as a result of the left shift have a value equal to 0.

## Assignment operators

=	assignment operator

++	increment, i.e., x++ is equivalent to x = x + 1; when used in an array index, evaluates to the value of the variable prior to the increment operation

− −	decrement, i.e., x− − is equivalent to x = x–− 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation

+=	increment by amount specified, i.e., x  +=  3 is equivalent to x = x + 3, and x  +=  (−3) is equivalent to x = x + (−3)

−=	decrement by amount specified, i.e., x  −=  3 is equivalent to x = x–− 3, and x  −=  (−3) is equivalent to x = x− (−3)

## Range notation

| x = y..z | x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than or equal to y |
| --- | --- |

## Mathematical functions

Mathematical functions are defined as follows:

(4-1)

(4-2)

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise	(4-3)

Floor(x) is the greatest integer less than or equal to x	(4-4)

Log10(x) returns the base-10 logarithm of x	(4-5)

Round(x) = Sign(x) * Floor(Abs(x) + 0.5)	(4-6)

(4-7)

xy specifies x to the power of y	(4-8)

power(x, y) specifies x to the power of y	(4-9)

## Specification of syntax functions and descriptors

The following function is used in the specification of the syntax:

read_bits( n ) reads the next n bits from the bitstream and advances the bitstream pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the bitstream pointer.

The following descriptors specify the parsing process of each syntax element:

- u(n): unsigned integer using n bits. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

- s(n): signed integer using n bits. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a two's complement integer representation with most significant bit written first.

# Functional architecture

## Description of the functional architecture

Figure 1 shows the functional architecture utilizing green metadata. The media pre-processor is applied to analyse and to filter the content source and a video encoder is used to encode the content to a bitstream for delivery. The bitstream is delivered to the receiver and decoded by a video decoder with the output rendered on a presentation subsystem that implements a display process.

Figure 1 — Functional architecture

The green metadata is extracted from either the media encoder or the media pre-processor. In both cases, the green metadata is multiplexed or encapsulated in the conformant bitstream. Such green metadata is used at the receiver to reduce the power consumption for video decoding and presentation. The bitstream is packetized and delivered to the receiver for decoding and presentation. At the receiver, the metadata extractor processes the packets and sends the green metadata to a power optimization module for efficient power control. For instance, the power optimization module interprets the green metadata and then applies appropriate operations to reduce the video decoder’s power consumption when decoding the video and to reduce the presentation subsystem’s power consumption when rendering the video. In addition, the power-optimization module can collect receiver information, such as remaining battery capacity, and send it to the transmitter as green feedback to adapt the encoder operations for power-consumption reduction.

The normative aspect of this document is limited to the green metadata and green feedback in Figure 1.

## Definition of components in the functional architecture

green metadata generator

—	Generates metadata from either the video encoder or the content pre-processor.

green metadata extractor

—	Interprets the bitstream syntax information and sends it to the power optimization module in the receiver.

green feedback generator

—	Generates feedback information for the transmitter.

—	Communicates with the transmitter through a feedback channel, if available, for energy-efficient processing.

green feedback extractor

—	Receives the feedback from the receiver and sends it to the power optimization module in the transmitter.

power optimization module in the transmitter

—	Collects platform statistics such as the remaining battery capacity of the device in which the transmitter resides.

—	Controls the operation of the green metadata generator, video encoder and content pre-processor.

—	Processes green feedback.

power optimization module in the receiver

—	Processes the green-metadata information and applies appropriate operations for power-consumption control.

—	Collects platform statistics such as remaining battery capacity of the device in which the receiver resides.

—	Sends requests to green feedback generator.

# Decoder power reduction

## General

Energy-efficient decoding is achieved with two types of metadata: complexity metrics (CMs) metadata and decoding operation reduction request (DOR-Req) metadata. A decoder may use CMs metadata to vary operating frequency and thus reduce decoder power consumption. In a point-to-point video conferencing application, the remote encoder may use the DOR-Req metadata to modify the decoding complexity of the bitstream and thus reduce local decoder power consumption.

## Complexity metrics for decoder-power reduction

### General

With respect to the functional architecture in Figure 1, the green-metadata generator provides CMs that indicate the picture-decoding complexity of an AVC, HEVC, VVC, or EVC bitstream to the decoder.

### Syntax

The syntax for the AVC CMs is described in Table 1.

Table 1 — syntax for the AVC CMs

|  | Descriptor |
| --- | --- |
| period_type | u(8) |
| if( ( period_type = = 2 ) ¦¦ ( period_type = = 7 ) ) |  |
| num_seconds | u(16) |
| else if( ( period_type = = 3 ) ¦¦ ( period_type = = 8 ) ) |  |
| num_pictures | u(16) |
| if( period_type = = 8 ) { |  |
| temporal_map | u(8) |
| for( t = 0; t < 8; t++ ) |  |
| if( ( temporal_map >> t ) % 2 = = 1 ) |  |
| num_pictures_in_temporal_layers[ t ] | u(16) |
| } |  |
| if( period_type <= 3 ) { |  |
| portion_non_zero_8x8_blocks | u(8) |
| portion_intra_predicted_macroblocks | u(8) |
| portion_six_tap_filterings | u(8) |
| portion_alpha_point_deblocking_instances | u(8) |
| } |  |
| else if( period_type = = 4 ) { |  |
| for( i = 0; i <= num_slice_groups_minus1; i++ ) |  |
| num_slices_minus1[ i ] | u(16) |
| for( i = 0; i <= num_slice_groups_minus1; i++ ) |  |
| for( j = 0; j <= num_slices_minus1[ i ]; j++ ) { |  |
| first_mb_in_slice[ i ][ j ] | u(16) |
| portion_non_zero_8x8_blocks[ i ][ j ] | u(8) |
| portion_intra_predicted_macroblocks[ i ][ j ] | u(8) |
| portion_six_tap_filterings[ i ][ j ] | u(8) |
| portion_alpha_point_deblocking_instances[ i ][ j ] | u(8) |
| } |  |
| } |  |
| else if( ( period_type >= 5 ) && ( period_type <= 8 ) ) { |  |
| num_layers_minus1 | u(16) |
| for( l = 0; l <= num_layers_minus1; l++ ) { |  |
| picture_parameter_set_id[ l ] | u(8) |
| priority_id[ l ] | u(6) |
| dependency_id[ l ] | u(3) |
| quality_id[ l ] | u(4) |
| temporal_id[ l ] | u(3) |
| portion_non_zero_8x8_blocks[ l ] | u(8) |
| portion_intra_predicted_macroblocks[ l ] | u(8) |
| portion_six_tap_filterings[ l ] | u(8) |
| portion_alpha_point_deblocking_instances[ l ] | u(8) |
| } |  |
| } |  |

The syntax for the HEVC CMs is described in Table 2.

Table 2 — syntax for the HEVC CMs

|  | Descriptor |
| --- | --- |
| period_type | u(8) |
| if( period_type = = 2 ) |  |
| num_seconds | u(16) |
| else if( period_type = = 3 ) |  |
| num_pictures | u(16) |
| if( period_type <= 3 ){ |  |
| portion_non_zero_blocks_area | u(8) |
| if( portion_non_zero_blocks_area != 0 ) { |  |
| portion_8x8_blocks_in_non_zero_area | u(8) |
| portion_16x16_blocks_in_non_zero_area | u(8) |
| portion_32x32_blocks_in_non_zero_area | u(8) |
| } |  |
| portion_intra_predicted_blocks_area | u(8) |
| if( portion_intra_predicted_blocks_area = = 255 ) { |  |
| portion_planar_blocks_in_intra_area | u(8) |
| portion_dc_blocks_in_intra_area | u(8) |
| portion_angular_hv_blocks_in_intra_area | u(8) |
| } |  |
| else { |  |
| portion_blocks_a_c_d_n_filterings | u(8) |
| portion_blocks_h_b_filterings | u(8) |
| portion_blocks_f_i_k_q_filterings | u(8) |
| portion_blocks_j_filterings | u(8) |
| portion_blocks_e_g_p_r_filterings | u(8) |
| } |  |
| portion_deblocking_instances | u(8) |
| } |  |
| else if( period_type = = 4 ) { |  |
| max_num_slices_tiles_minus1 | u(16) |
| for( t = 0; t <= max_num_slices_tiles_minus1; t++ ) { |  |
| first_ctb_in_slice_or_tile[ t ] | u(16) |
| portion_non_zero_blocks_area[ t ] | u(8) |
| if( portion_non_zero_blocks_area[ t ] != 0 ) { |  |
| portion_8x8_blocks_in_non_zero_area[ t ] | u(8) |
| portion_16x16_blocks_in_non_zero_area[ t ] | u(8) |
| portion_32x32_blocks_in_non_zero_area[ t ] | u(8) |
| } |  |
| portion_intra_predicted_blocks_area[ t ] | u(8) |
| if( portion_intra_predicted_blocks_area[ t ] = = 255 ) { |  |
| portion_planar_blocks_in_intra_area[ t ] | u(8) |
| portion_dc_blocks_in_intra_area[ t ] | u(8) |
| portion_angular_hv_blocks_in_intra_area[ t ] | u(8) |
| } |  |
| else { |  |
| portion_blocks_a_c_d_n_filterings[ t ] | u(8) |
| portion_blocks_h_b_filterings[ t ] | u(8) |
| portion_blocks_f_i_k_q_filterings[ t ] | u(8) |
| portion_blocks_j_filterings[ t ] | u(8) |
| portion_blocks_e_g_p_r_filterings[ t ] | u(8) |
| } |  |
| portion_deblocking_instances[ t ] | u(8) |
| } |  |
| } |  |

The syntax for the VVC CMs is described in Table 3.

Table 3 — syntax for the VVC CMs

|  | Descriptor |
| --- | --- |
| period_type | u(4) |
| granularity_type | u(3) |
| extended_representation_flag | u(1) |
| if( period_type = = 2 ) |  |
| num_seconds | u(16) |
| else if( period_type = = 3 ) |  |
| num_pictures | u(16) |
| if( granularity_type = = 0 ) { |  |
| portion_non_zero_blocks_area | u(8) |
| portion_non_zero_transform_coefficients_area | u(8) |
| portion_intra_predicted_blocks_area | u(8) |
| portion_deblocking_instances | u(8) |
| portion_alf_filtered_blocks | u(8) |
| if( extended_representation_flag ) { |  |
| if( portion_non_zero_blocks_area != 0 ) { |  |
| portion_non_zero_4_8_16_blocks_area | u(8) |
| portion_non_zero_32_64_128_blocks_area | u(8) |
| portion_non_zero_256_512_1024_blocks_area | u(8) |
| portion_non_zero_2048_4096_blocks_area | u(8) |
| } |  |
| if( portion_intra_predicted_blocks_area < 255 ) { |  |
| portion_bi_and_gpm_predicted_blocks_area | u(8) |
| portion_bdof_blocks_area | u(8) |
| } |  |
| portion_sao_filtered_blocks | u(8) |
| } |  |
| } |  |
| else if( granularity_type <= 3 ) { |  |
| max_num_segments_minus1 | u(16) |
| for( t = 0; t <= max_num_segments_minus1; t++ ) { |  |
| segment_address[ t ] | u(16) |
| portion_non_zero_blocks_area[ t ] | u(8) |
| portion_non_zero_transform_coefficients_area[ t ] | u(8) |
| portion_intra_predicted_blocks_area[ t ] | u(8) |
| portion_deblocking_instances[ t ] | u(8) |
| portion_alf_filtered_blocks[ t ] | u(8) |
| if( extended_representation_flag ) { |  |
| if( portion_non_zero_blocks_area[ t ] != 0 ) { |  |
| portion_non_zero_4_8_16_blocks_area[ t ] | u(8) |
| portion_non_zero_32_64_128_blocks_area[ t ] | u(8) |
| portion_non_zero_256_512_1024_blocks_area[ t ] | u(8) |
| portion_non_zero_2048_4096_blocks_area[ t ] | u(8) |
| } |  |
| if( portion_intra_predicted_blocks_area[ t ] < 255 ) { |  |
| portion_bi_and_gpm_predicted_blocks_area[ t ] | u(8) |
| portion_bdof_blocks_area[ t ] | u(8) |
| } |  |
| portion_sao_filtered_blocks[ t ] | u(8) |
| } |  |
| } |  |
| } |  |

The syntax for the EVC CMs is described in Table 4:

Table 4 — syntax for the EVC CMs

|  | Descriptor |
| --- | --- |
| period_type | u(8) |
| if( profile_idc = = 0 ) { |  |
| if( ( period_type = = 0 ) ¦¦ ( period_type == 2 ) ) { |  |
| num_non_zero_4_cus | uk(v) |
| num_non_zero_8_cus | uk(v) |
| num_non_zero_16_cus | uk(v) |
| num_non_zero_32_cus | uk(v) |
| num_non_zero_64_cus | uk(v) |
| portion_fractional_prediction_sample | u(8) |
| } else if( ( period_type = = 1 ) ¦¦ ( period_type = = 3) ) { |  |
| num_count | u(16) |
| for( t = 0; t < num_count; t++ ) { |  |
| num_non_zero_4_cus [t] | uk(v) |
| num _non_zero_8_cus [t] | uk(v) |
| num _non_zero_16_cus [t] | uk(v) |
| num _non_zero_32_cus [t] | uk(v) |
| num _non_zero_64_cus [t] | uk(v) |
| portion_fractional_prediction_sample [t] | u(8) |
| } |  |
| } |  |
| else if( profile_idc = = 1 ) { |  |
| if( ( period_type = = 0 ) ¦¦ ( period_type = = 2 ) ) { |  |
| num_non_zero_samples | uk(v) |
| num_affine_samples | uk(v) |
| num_dmvr_samples | uk(v) |
| num_alf_samples | uk(v) |
| num_deblocking_filter_samples | uk(v) |
| num_htdf_samples | uk(v) |
| } else if( ( period_type = = 1 ) ¦¦ ( period_type = = 3) ) { |  |
| num_count | u(8) |
| for( t = 0; t < num_count; t++) { |  |
| num_non_zero_samples [t] | uk(v) |
| num__samples [t] | uk(v) |
| num_dmvr_samples [t] | uk(v) |
| num_alf_samples [t] | uk(v) |
| num_deblocking_filter_samples [t] | uk(v ) |
| num_htdf_samples [t] | uk(v ) |
| } |  |
| } |  |
| } |  |

### Signalling

SEI messages can be used to signal green metadata in an AVC, HEVC or VVC stream. The green metadata SEI message payload type is specified in ISO/IEC 14496-10, ISO/IEC 23008-2, and ISO/IEC 23090-3. The complete syntax of the green metadata SEI message payload is specified in Annex A.

The message containing the CMs is transmitted at the start of an upcoming period. The next message containing CMs is transmitted at the start of the next upcoming period. Therefore, when the upcoming period is a picture or the interval up to the next I-slice, a message is transmitted for each picture or interval, respectively. However, when the upcoming period is a specified time interval or a specified number of pictures, the associated message is transmitted with the first picture in the time interval or with the first picture in the specified number of pictures.

### Semantics

#### AVC semantics

The semantics of various terms are defined below.

period_type specifies the type of upcoming period over which the complexity metrics are applicable and is defined in the Table 5.

Table 5 — specification of period_type for AVC

| Value | Description |
| --- | --- |
| 0x00 | complexity metrics are applicable to a single picture |
| 0x01 | complexity metrics are applicable to all pictures in decoding order, up to (but not including) the picture containing the next I slice |
| 0x02 | complexity metrics are applicable over a specified time interval in seconds |
| 0x03 | complexity metrics are applicable over a specified number of pictures counted in decoding order |
| 0x04 | complexity metrics are applicable to a single picture with slice granularity |
| 0x05 | complexity metrics are applicable to a single picture with scalable layer granularity |
| 0x06 | complexity metrics are applicable to all pictures in decoding order, up to (but not including) the picture containing the next I slice in the base layer with scalable layer granularity |
| 0x07 | complexity metrics are applicable over a specified time interval in seconds with scalable layer granularity |
| 0x08 | complexity metrics are applicable over a specified number of pictures counted in decoding order with scalable layer granularity |
| 0x09–0xFF | user-defined |

num_seconds indicates the number of seconds over which the complexity metrics are applicable when period_type is 2 or 7.

num_pictures indicates the number of pictures, counted in decoding order, over which the complexity metrics are applicable when period_type is 3 or 8. When period_type is 8, this is a default number of pictures for each temporal layer, which can be overridden using temporal_map flags.

NpicsInPeriod specifies the number of pictures in the specified period. When period_type is 0 or 4, then NpicsInPeriod is 1. When period_type is 1, then NpicsInPeriod is determined by counting the pictures in decoding order up to (but not including) the one containing the next I slice. When period_type is 2, then NpicsInPeriod is determined from the frame rate. When period_type is 3, then NpicsInPeriod is equal to num_pictures.

NmbsInPeriod specifies the total number of macroblocks that are coded in the specified period. It is determined by the following computation:

NmbsInPeriod=n=1NpicsInPeriodNmbsInPic(n)

(6-1)

where NmbsInPic(n) is set to the value of the AVC variable PicSizeInMbs for the nth picture within the specified period, where 1 <= n <= NpicsInPeriod.

temporal_map indicates which temporal layer has a different number of pictures from num_pictures in the specified period, when period_type is 8.

num_pictures_in_temporal_layers[ t ] indicates the number of pictures in the specified period for the tth temporal layer when period_type is 8. When not present, it is equal to num_pictures.

NpicsInPeriodForTempLayer[ t ] specifies the number of pictures in the specified period for the tth temporal layer. When period_type is 5 then NpicsInPeriodForTempLayer[ t ] is 1. When period_type is 6, then NpicsInPeriodForTempLayer[ t ] is determined by counting the pictures associated to  the tth temporal layer in decoding order up to (but not including) the one containing the next I slice. When period_type is 7, then NpicsInPeriodForTempLayer[ t ] is determined from the frame rate associated to the tth temporal layer. When period_type is 8, then NpicsInPeriodForTempLayer[ t ] is equal to num_pictures_in_temporal_layers[ t ].

portion_non_zero_8x8_blocks indicates the portion of 8x8 blocks with non-zero transform coefficients values in the specified period and is set equal to PnonZero8x8Blks defined as follows:

PnonZero8x8Blks=FloorNnonZero8x8Blks 4 * NmbsInPeriod*255	(6-2)

where NnonZero8x8Blks is the number of 8x8 blocks with non-zero transform coefficients values in the specified period. NnonZero8x8Blks is derived from portion_non_zero_8x8_blocks and NmbsInPeriod in the decoder.

portion_intra_predicted_macroblocks indicates the portion of intra-predicted macroblocks in the specified period and is set equal to PintraMbs defined as follows:

PintraMbs=FloorNintraMbs NmbsInPeriod *255	 (6-3)

where NintraMbs is the number of intra-predicted macroblocks in the specified period. NintraMbs is derived from portion_intra_predicted_macroblocks and NmbsInPeriod in the decoder.

portion_six_tap_filterings indicates the portion of 6-tap filterings (STFs) in the specified period and is set equal to PsixTapFilt defined as follows:

PsixTapFilt=FloorNsixTapFilt NmaxSixTapFiltInPeriod*255	(6-4)

where NmaxSixTapFiltInPeriod is the maximum number of STFs that can occur within the specified period and is derived from NmbsInPeriod variable as

NmaxSixTapFiltInPeriod = ( 1664 * NmbsInPeriod )	(6-5)

and NsixTapFilt is the number of 6-tap filterings (STFs) within the specified period. Guidance for the counting of NsixTapFilt can be found in Annex B. NsixTapFilt is derived from portion_six_tap_filterings and NmaxSixTapFiltInPeriod in the decoder.

portion_alpha_point_deblocking_instances indicates the portion of alpha-point deblocking instances (APDIs) in the specified period and is set equal to PalphaPtDbfs defined as follows:

PalphaPtDbfs=FloorNalphaPtDbfs NmaxAlphaPtDbfsInPeriod *255	(6-6)

NmaxAlphaPtDbfsInPeriod is the maximum number of APDIs that can occur within the specified period and is derived from NmbsInPeriod and SchrMult variables as

NmaxAlphaPtDbfsInPeriod =128 * SchrMult* NmbsInPeriod	(6-7)

SchrMult depends on the AVC variables separate_colour_plane_flag and chroma_format_idc as shown in the Table 6.

Table 6 — specification of SchrMult for AVC

| SchrMult | separate_colour_plane_flag | chroma_format_idc | Comment |
| --- | --- | --- | --- |
| 1 | 0 | 0 | monochrome |
| 1.5 | 0 | 1 | 4:2:0 sampling |
| 2 | 0 | 2 | 4:2:2 sampling |
| 3 | 0 | 3 | 4:4:4 sampling |
| 3 | 1 | any value | separate colour plane |

NalphaPtDbfs is the number of APDIs in the specified period. Using the notation in ISO/IEC 14496-10, this is equivalent to the total number of filtering operations applied to produce filtered samples of the type p'0 or q'0, in the specified period. NalphaPtDbfs is derived from portion_alpha_point_deblocking_instances and NmaxAlphaPtDbfsInPeriod in the decoder.

num_slices_minus1 plus 1 indicates the number of slices per slice_group in the picture.

first_mb_in_slice[ i ][ j ] indicates the first macroblock number in the slice[ i ][ j ].

NmbsInSlice[ i ][ j ] is the total number of macroblocks that are coded in the slice[ i ][ j ] and is determined by the following computation:

- if num_slice_groups_minus1 is equal to 0, the following process described in pseudo-code applies.

if (j<num_slices_minus1[0])

NmbsInSlice[0][j] = first_mb_in_slice[0][j+1] – first_mb_in_slice[0][j]

else

NmbsInSlice[0][j] = PicSizeInMbs – first_mb_in_slice[0][j]	 (6-8)

- otherwise (num_slice_groups_minus1 is not equal to 0), and after derivation of the macroblock to slice group map (MbToSliceGroupMap) as specified in ISO/IEC 14496-10:2019, 8.2.2.8, the following process described in pseudo-code applies:

k=0;

if (j<num_slices_minus1[i])

for ( n=first_mb_in_slice[i][j]; n<first_mb_in_slice[i][j+1]; n++ )

if ( MbToSliceGroupMap[first_mb_in_slice[i][j]] = = MbToSliceGroupMap[n] )

k++;

NmbsInSlice[i][j] = k;

else

for ( n=first_mb_in_slice[i][j]; n<PicSizeInMbs; n++ )

if ( MbToSliceGroupMap[first_mb_in_slice[i][j]] = = MbToSliceGroupMap[n] )

k++;

NmbsInSlice[i][j] = k; 	 (6-9)

portion_non_zero_8x8_blocks[ i ][ j ] indicates the portion of 8x8 blocks with non-zero transform coefficients values in the slice[ i ][ j ] and is set equal to PnonZero8x8Blks[ i ][ j ] defined as follows:

PnonZero8x8Blks[ i ][ j ]=FloorNnonZero8x8Blks i  j  4 * NmbsInSlice[ i ][ j ]*255	(6-10)

where NnonZero8x8Blks[ i ][ j ] is the number of 8x8 blocks with non-zero transform coefficients values in the slice[ i ][ j ]. NnonZero8x8Blks[ i ][ j ] is derived from portion_non_zero_8x8_blocks[ i ][ j ] and NmbsInSlice[ i ][ j ] in the Decoder.

portion_intra_predicted_macroblocks[ i ][ j ] indicates the portion of macroblocks using intra prediction modes in the slice[ i ][ j ] and is set equal to PintraMbs[ i ][ j ] defined as follows:

PintraMbs[ i ][ j ]=FloorNintraMbs[ i ][ j ]NmbsInSlice[ i ][ j ]*255	(6-11)

where NintraMbs[ i ][ j ] is the number of macroblocks using intra prediction modes in the slice[ i ][ j ]. NintraMbs[ i ][ j ] is derived from portion_intra_predicted_macroblocks[ i ][ j ] and NmbsInSlice[ i ][ j ] in the decoder.

portion_six_tap_filterings[ i ][ j ] indicates the portion of 6-tap filterings (STFs) in the specified slice[ i ][ j ] and is set equal to PsixTapFilt[ i ][ j ] defined as follows:

PsixTapFilt[ i ][ j ]=FloorNsixTapFilt[ i ][ j ]NmaxSixTapFiltInSlice[ i ][ j ]*255	(6-12)

where NmaxSixTapFiltInSlice[ i ][ j ] is the maximum number of STFs that can occur in the slice[ i ][ j ] and is derived from NmbsInSlice[ i ][ j ] variable as

NmaxSixTapFiltInSlice[ i ][ j ] = 1664 * NmbsInSlice[ i ][ j ]	(6-13)

and NsixTapFilt[ i ][ j ] is the number of 6-tap filterings (STFs) within the slice[ i ][ j ]. Guidance for the counting of NsixTapFilt[ i ][ j ]  can be found in Annex B. NsixTapFilt[ i ][ j ] is derived from portion_six_tap_filterings[ i ][ j ] and NmaxSixTapFiltInslice[ i ][ j ] in the decoder.

portion_alpha_point_deblocking_instances[ i ][ j ] indicates the portion of alpha-point deblocking instances (APDIs) in the specified slice[ i ][ j ] and is set equal to PalphaPtDbfs[ i ][ j ] defined as follows:

PalphaPtDbfs[ i ][ j ]= FloorNalphaPtDbfs[ i ][ j ]NmaxAlphaPtDbfsInSlice[ i ][ j ]*255	(6-14)

where NmaxAlphaPtDbfsInSlice[ i ][ j ] is the maximum number of APDIs that can occur in the slice[ i ][ j ] and is derived from NmbsInSlice[ i ][ j ] and SchrMult variables as

NmaxAlphaPtDbfsInSlice[ i ][ j ] = 128 * SchrMult * NmbsInSlice[ i ][ j ]	 (6-15)

and NalphaPtDbfs[ i ][ j ] is the number of alpha-point deblocking instances (APDIs) in slice[ i ][ j ]. NalphaPtDbfs[ i ][ j ] is derived from portion_alpha_point_deblocking_instances[ i ][ j ] and NmaxAlphaPtDbfsInSlice[ i ][ j ] in the decoder.

num_layers_minus1 plus 1 indicates the number of scalable layers in the associated picture or in the specified period.

pic_parameter_set_id[ l ] indicates the picture parameter set in use for the lth scalable layer. The value of pic_parameter_set_id[ l ] shall be in the range of 0 to 255, inclusive (as specified in  ISO/IEC 14496-10:2019, 7.4.3).

priority_id[ l ] indicates a priority identifier for the NAL unit in the lth scalable layer. The value of priority_id[ l ] shall be in the range of 0 to 63, inclusive (as specified in ISO/IEC 14496-10:2019, G.7.3.1.1).

dependency_id[ l ] indicates a dependency identifier for the NAL unit in the lth scalable layer. The value of dependency_id[ l ] shall be in the range of 0 to 7, inclusive (as specified in ISO/IEC 14496-10:2019, G.7.3.1.1).

quality_id[ l ] indicates a quality identifier for the NAL unit in the lth scalable layer. The value of quality_id[ l ] shall be in the range of 0 to 15, inclusive (as specified in ISO/IEC 14496-10:2019, G.7.3.1.1).

temporal_id[ l ] indicates a temporal identifier for the NAL unit in the lth scalable layer. The value of temporal_id[ l ] shall be in the range of 0 to 7, inclusive (as specified in ISO/IEC 14496-10:2019, G.7.3.1.1).

portion_non_zero_8x8_blocks[ l ] indicates the portion of 8x8 blocks with non-zero transform coefficients values in the lth scalable layer and is set equal to PnonZero8x8Blks[ i ][ j ] defined as follows:

PnonZero8x8Blks[ l ]=FloorNnonZero8x8Blks[ l ]4 * NmbsInLayerInPeriod[ l ]*255	(6-16)

NmbsInLayerInPeriod[ l ] is the total number of macroblocks in the lth  scalable layer in the specified period and is derived from NmbsInLayerInPeriod[ l ] and NpicsInPeriodForTempLayer[ temporal_id[ l ] ] as

NmbsInLayerInPeriod[ l ] =  NmbsInLayer[ l ]* NpicsInPeriodForTempLayer[ temporal_id l  ] 	(6-17)

where NmbsInLayer[ l ] is the total number of macroblocks in the lth scalable layer and determined after derivation of the number of macroblocks associated with pic_parameter_set_id[ l ],  as specified in ISO/IEC 14496-10:2019, 7.4.3.

NnonZero8x8Blks[ l ] is the number of 8x8 blocks with non-zero transform coefficients values in the lth scalable layer in the specified period. It is derived from portion_non_zero_8x8_blocks[ l ] and NmbsInLayerInPeriod[ l ] in the decoder.

portion_intra_predicted_macroblocks[ l ] indicates the portion of macroblocks using intra prediction modes in the lth scalable layer and is set equal to PintraMbs[ i ][ j ] defined as follows:

PintraMbs[ l ]=FloorNintraMbs[ l ]NmbsInLayerInPeriod[ l ]*255	(6-18)

NintraMbs[ l ] is the number of macroblocks using intra prediction modes in the lth  scalable layer in the specified period. It is derived from portion_intra_predicted_macroblocks[ l ] And NmbsInLayerInPeriod[ l ] in the decoder.

portion_six_tap_filterings[ l ] indicates the portion of 6-tap filterings (STFs) in the specified lth scalable layer in the specified period and is set equal to PsixTapFilt[ i ][ j ] defined as follows:

PsixTapFilt[ l ]=FloorNsixTapFilter[ l ]NmaxSixTapFiltInLayerInPeriod[ l ]*255	(6-19)

NmaxSixTapFiltInLayerInPeriod[ l ] is the maximum number of STFs that can occur in the lth scalable layer in the specified period and is derived from NmbsInLayerInPeriod[ l ] variable as

NmaxSixTapFiltInLayerInPeriod[ l ] = 1664 * NmbsInLayerInPeriod[ l ]	(6-20)

NsixTapFilt[ l ] is the number of 6-tap filterings (STFs) within the lth scalable layer in the specified period. Guidance for the counting of NsixTapFilt[ l ] can be found in Annex B. It is derived from portion_six_tap_filterings[ l ] and NmaxSixTapFiltInLayerInPeriod[ l ] in the decoder.

portion_alpha_point_deblocking_instances[ l ] indicates the portion of alpha-point deblocking instances (APDIs) in the specified lth scalable layer in the specified period and is set equal to PalphaPtDbfs[ i ][ j ] defined as follows:

PalphaPtDbfs[ l ]=FloorNalphaPtDbfs [ l ]NmaxAlphaPtDbfsInLayerInPeriod[ l ]*255	(6-21)

NmaxAlphaPtDbfsInLayerInPeriod[ l ] is the maximum number of APDIs that can occur in the lth scalable layer in the specified period and is derived from NmbsInLayerInPeriod[ l ] and SchrMult variables as

NmaxAlphaPtDbfsInLayerInPeriod[ l ] = 128 * SchrMult * NmbsInLayerInPeriod[ l ]	(6-22)

NalphaPtDbfs[ l ] is the number of alpha-point deblocking instances (APDIs) in the lth scalable layer in the specified period. It is derived from portion_alpha_point_deblocking_instances[ l ] and NmaxAlphaPtDbfsInLayerInPeriod[ l ] in the decoder.

#### HEVC semantics

The semantics of various terms are defined below.

period_type specifies the type of upcoming period over which the complexity metrics are applicable and is defined in the Table 7.

Table 7 — specification of period_type for HEVC

| Value | Description |
| --- | --- |
| 0x00 | complexity metrics are applicable to a single picture |
| 0x01 | complexity metrics are applicable to all pictures in decoding order, up to (but not including) the picture containing the next I slice |
| 0x02 | complexity metrics are applicable over a specified time interval in seconds |
| 0x03 | complexity metrics are applicable over a specified number of pictures counted in decoding order |
| 0x04 | complexity metrics are applicable to a single picture with slice or tile granularity |
| 0x05-0xFF | reserved |

num_seconds indicates the number of seconds over which the complexity metrics are applicable when period_type is 2.

num_pictures indicates the number of pictures, counted in decoding order, over which the complexity metrics are applicable when period_type is 3.

NpicsInPeriod is the number of pictures in the specified period. When period_type is 0, then NpicsInPeriod is 1. When period_type is 1, then NpicsInPeriod is determined by counting the pictures in decoding order up to (but not including) the one containing the next I slice. When period_type is 2, then NpicsInPeriod is determined from the frame rate. When period_type is 3, then NpicsInPeriod is equal to num_pictures.

N4x4BlksInPeriod is the total number of 4x4 blocks that are coded in the specified period.

It is determined by the following computation:

N4x4BlksInPeriod=n=1NpicsInPeriodN4x4BlksPic(n)

(6-23)

where N4x4BlksPic(n) is derived for the nth picture within the specified period, with 1 <= n <= NpicsInPeriod , from HEVC variables CtbLog2SizeY and PicSizeInCtbsY as follows:

N4x4BlksPic(n) = SpicInCtb * Nctbs	(6-24)

where

- Nctbs is set equal to ( 1 << ( CtbLog2SizeY – 2 ) )2

- SpicInCtb is set equal to PicSizeInCtbsY.

portion_non_zero_blocks_area indicates the portion of area covered by blocks with non-zero transform coefficients values, in the pictures of the specified period, using a 4x4 blocks granularity and is set equal to PnonZeroBlksArea defined as follows:

PnonZeroBlksArea=FloorNnonZeroBlks N4x4BlksInPeriod *255 	(6-25)

where NnonZeroBlks is the number of blocks with non-zero transform coefficients values in the specified period using 4x4 granularity. At the encoder side, NnonZeroBlks is computed as follows:

NnonZeroBlks =NnonZero4x4Blks+4*NnonZero8x8Blks

+16*NnonZero16x16Blks+64*NnonZero32x32Blks	(6-26)

where NnonZero4x4Blks, NnonZero8x8Blks, NnonZero16x16Blks, NnonZero32x32Blks are the number of 4x4, 8x8, 16x16 and 32x32 blocks with non-zero transform coefficients values, respectively, in the specified period.

NnonZeroBlks is derived from portion_non_zero_blocks_area and N4x4BlksInPeriod in the decoder.

portion_8x8_blocks_in_non_zero_area indicates the portion of 8x8 blocks area in the non-zero area in the specified period and is set equal to PnonZero8x8Blks defined as follows:

PnonZero8x8Blks=Floor4 * NnonZero8x8BlksNnonZeroBlks *255	(6-27)

When not present, it is set equal to 0.

NnonZero8x8Blks is the number of 8x8 blocks with non-zero transform coefficients values in the specified period. It is derived from portion_8x8_blocks_in_Non_zero_area and NnonZeroBlks in the decoder.

portion_16x16_blocks_in_non_zero_area indicates the portion of 16x16 blocks area in the non-zero area in the specified period and is set equal to PnonZero16x16Blks defined as follows:

PnonZero16x16Blks=Floor16 * NnonZero16x16BlksNnonZeroBlks *255	(6-28)

When not present, is equal to 0.

NnonZero16x16Blks is the number of 16x16 blocks with non-zero transform coefficients values in the specified period. It is derived from portion_16x16_blocks_in_Non_zero_area and NnonZeroBlks in the decoder.

portion_32x32_blocks_in_non_zero_area indicates the portion of 32x32 blocks area in the non- zero area in the specified period and is set equal to PnonZero32x32Blks defined as follows:

PnonZero32x32Blks=Floor64 * NnonZero32x32BlksNnonZeroBlks *255	(6-29)

When not present, it is set equal to 0.

NnonZero32x32Blks is the number of 32x32 blocks with non-zero transform coefficients values in the specified period. It is derived from portion_32x32_blocks_in_non_zero_area and NnonZeroBlks in the decoder.

NnonZero4x4Blks is the number of 4x4 blocks with non-zero transform coefficients values in the specified period. NnonZero4x4Blks is derived from NnonZeroBlks, NnonZero8x8Blks, NnonZero16x16Blks and NnonZero32x32Blks as follows in the decoder:

NnonZero4x4Blks=NnonZeroBlks-4*NnonZero8x8Blks

-16*NnonZero16x16Blks-64*NnonZero32x32Blks	(6-30)

portion_intra_predicted_blocks_area indicates the portion of area covered by intra predicted blocks in the pictures of the specified period using 4x4 granularity and is set equal to PintraBlks defined as follows:

PintraBlks=Floor4 * NintraBlksN4x4BlksInPeriod *255	(6-31)

NintraBlks is the number of intra predicted blocks in the specified period using 8x8 granularity. At the encoder side, it is computed as follows:

NintraBlks =Nintra8x8Blks+4*Nintra16x16Blks+16*Nintra32x32Blks

+ 64*Nintra64x64Blks 	(6-32)

where Nintra8x8Blks, Nintra16x16Blks, Nintra32x32Blks and Nintra64x64Blks are the number of intra predicted 8x8, 16x16, 32x32 and 64x64 blocks respectively, in the specified period.

NintraBlks is derived from portion_intra_predicted_blocks_area and N4x4BlksInPeriod in the decoder.

portion_planar_blocks_in _intra_area indicates the portion of planar blocks area in the intra predicted area in the specified period and is set equal to PplanarBlksInIntra defined as follows:

PplanarBlksInIntra=FloorNplanarBlks4 * NintraBlks*255	(6-33)

When not present, it is set equal to 0.

NplanarBlks is the number of intra planar predicted blocks in the specified period using 4x4 granularity.  At the encoder side, it is computed as follows:

NplanarBlks=Nplanar4x4Blks+4*Nplanar8x8Blks+16*Nplanar16x16Blks

+ 64*Nplanar32x32Blks+256*Nplanar64x64Blks	  (6-34)

where Nplanar4x4Blks, Nplanar8x8Blks, Nplanar16x16Blks, Nplanar32x32Blks and Nplanar64x64Blks are the number of intra planar predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks respectively, in the specified period.

NplanarBlks is derived from portion_planar_blocks_in_intra_area and NintraBlks in the decoder.

portion_dc_blocks_in_intra_area indicates the portion of DC blocks area in the intra predicted area in the specified period and is set equal to PDCBlksInIntra defined as follows:

PDCBlksInIntra=FloorNDCBlks4 * NintraBlks*255	(6-35)

When not present, it is set equal to 0.

NDCBlks is the number of intra DC predicted blocks in the specified period using 4x4 granularity. At the encoder side, it is computed as follows:

NDCBlks=NDC4x4Blks+4*NDC8x8Blks+16*NDC16x16Blks

+ 64*NDC32x32Blks+256*NDC64x64Blks  	(6-36)

where NDC4x4Blks, NDC8x8Blks, NDC16x16Blks, NDC32x32Blks and NDC64x64Blks are the number of intra DC predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks respectively, in the specified period.

NDCBlks is derived from portion_dc_blocks_in_intra_area and NintraBlks in the decoder.

portion_angular_hv_blocks_in_intra_area indicates the portion of angular horizontal or vertical blocks area in the intra predicted area in the specified period and is set equal to PHVBlksInIntra defined as follows:

PHVBlksInIntra=FloorNHVBlks4 * NintraBlks*255	(6-37)

When not present, it is set equal to 0.

NangularHVBlks is the number of intra angular horizontally or vertically predicted blocks in the specified period using 4x4 granularity. At the encoder side, it is computed as follows:

NangularHVBlks=NangularHV4x4Blks+4*NangularHV8x8Blks+16*NangularHV16x16Blks

+ 64*NangularHV32x32Blks+256*NangularHV64x64Blks 	(6-38)

where NangularHV4x4Blks, NangularHV8x8Blks, NangularHV16x16Blks, NangularHV32x32Blks and NangularHV64x64Blks are the number of intra angular horizontally or vertically predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks respectively, in the specified period.

NangularHVBlks is derived from portion_angular_hv_blocks_in_intra_area and NintraBlks in the decoder.

portion_blocks_a_c_d_n_filterings indicates the portion of prediction blocks whose luma samples position are located in sub-sample position a, c, d or n, as defined in Annex B, in the specified period and is set equal to PacdnFiltBlks defined as follows:

PacdnFiltBlks=FloorNacdnFiltBlksN4x4BlksInPeriod*255	(6-39)

When not present, it is set equal to 0.

NacdnFiltBlks is the number of prediction blocks whose luma samples position are located in sub-sample position a, c, d or n, as defined in Annex B, in the specified period. It is derived from portion_blocks_a_c_d_n_filterings and N4x4BlksInPeriod in the decoder.

portion_blocks_h_b_filterings indicates the portion of prediction blocks whose luma samples position are located in sub-sample position h or b, as defined in Annex B, in the specified period and is set equal to PhbFiltBlks defined as follows:

PhbFiltBlks=FloorNhbFiltBlksN4x4BlksInPeriod*255	(6-40)

When not present, it is set equal to 0.

NhbFiltBlks is the number of prediction blocks whose luma samples position are located in sub-sample position h or b, as defined in Annex B, in the specified period.

It is derived from portion_blocks_h_b_filterings and N4x4BlksInPeriod in the decoder.

portion_blocks_f_i_k_q_filterings indicates the portion of prediction blocks whose luma samples position are located in sub-sample position f, i, k or q, as defined in Annex B, in the specified period and is set equal to PfikqFiltBlks defined as follows:

PfikqFilt=FloorNfikqFiltBlksN4x4BlksInPeriod*255	(6-41)

When not present, it is set equal to 0.

NfikqFiltBlks is the number of prediction blocks whose luma samples position are located in sub-sample position f, i, k or q as defined in Annex B, in the specified period.

It is derived from portion_blocks_f_i_k_q_Filterings and N4x4BlksInPeriod in the decoder.

portion_blocks_j_filterings indicates the portion of prediction blocks whose luma samples position are located in sub-sample position j, as defined in Annex B, in the specified period and is set equal to PjFiltBlks defined as follows:

PjFiltBlks=FloorNjFiltBlksN4x4BlksInPeriod*255	(6-42)

When not present, it is set equal to 0.

NjFiltBlks is the number of prediction blocks whose luma samples position are located in sub-sample position j, as defined in Annex B, in the specified period.

It is derived from portion_blocks_j_Filterings and N4x4BlksInPeriod in the decoder.

portion_blocks_e_g_p_r_filterings indicates the portion of prediction blocks whose luma blocks position are located in sub-sample position e, g, p or r, as defined in Annex B, in the specified period and is set equal to PegprFiltBlks defined as follows:

PegprFiltBlks=FloorNegprFiltBlksN4x4BlksInPeriod*255	(6-43)

When not present, it is set equal to 0.

NegprFiltBlks is the number of prediction blocks whose luma samples position are located in sub-sample position e, g, p or r, as defined in Annex B, in the specified period.

It is derived from portion_blocks_e_g_p_r_Filterings and N4x4BlksInPeriod in the decoder.

portion_deblocking_instances indicates the portion of deblocking filtering instances in the specified period and is set equal to PdbfInstances defined as follows:

PdbfInstances=FloorNdbfInstances4 * SchrMult * N4x4BlksInPeriod*255	(6-44)

SchrMult depends on the HEVC variables separate_colour_plane_flag and chroma_format_idc as shown in the Table 8.

Table 8 — specification of SchrMult for HEVC

| SchrMult | separate_colour_plane_flag | chroma_format_idc | Comment |
| --- | --- | --- | --- |
| 1 | 0 | 0 | monochrome |
| 1.5 | 0 | 1 | 4:2:0 sampling |
| 2 | 0 | 2 | 4:2:2 sampling |
| 3 | 0 | 3 | 4:4:4 sampling |
| 3 | 1 | 3 | separate colour plane |

NdbfInstances is the number of deblocking filtering instances in the specified period. It is derived from portion_deblocking_instances, N4x4BlksInPeriod and SchrMult in the decoder.

max_num_slices_tiles_minus1 specifies the maximum number between the number of slices and the number of tiles in the associated picture.

first_ctb_in_slice_or_tile[ t ] specifies the first Coding Tree Block (CTB) number in slice[ t ] or tile[ t ] in raster scan order.

N4x4BlksInSliceOrTile[ t ] is the total number of 4x4 blocks in the slice[ t ] or tile[ t ] and is determined, after derivation of the Coding tree block raster and tile scanning conversion process (CtbAddrRsToTs) as specified in ISO/IEC 23008-2:2017, 6.5.1, by the following computation:

- The value IfirstCtb is set equal to CtbAddrRsToTs[ first_ctb_in_slice_or_tile[ t ] ].

- If t is lower than num_max_slices_tiles_minus1, the value IlastCtb is set equal to CtbAddrRsToTs[ first_ctb_in_slice_or_tile[t+1] ].

- Otherwise (t equal to num_max_slices_tiles_minus1), the value IlastCtb is set equal to CtbAddrRsToTs[ PicSizeInCtbsY ].

- N4x4BlksInSliceOrTile[ t ] is derived as follows:

N4x4BlksInSliceOrTile[ t ] = ( IlastCtb – IfirstCtb ) * Nctbs	(6-45)

portion_non_zero_blocks_area[ t ] indicates the portion of area covered by blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] using a 4x4 blocks granularity and is set equal to PnonZeroBlksAreaInSliceOrTile[ t ] defined as follows:

PnonZeroBlksAreaInSliceOrTile[ t ]=FloorNnonZeroBlksInSliceOrTile[ t ]N4x4BlksInSliceOrTile[ t ]*255 	(6-46)

NnonZeroBlksInSliceOrTile[ t ] is the number of blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] using 4x4 granularity. At the encoder side, it is computed as follows:

NnonZeroBlksInSliceOrTilet= NnonZero4x4BlksInSliceOrTile t

+ 4*NnonZero8x8BlksInSliceOrTile t + 16*NnonZero16x16BlksInSliceOrTile[ t ]

+64*NnonZero32x32BlksInSliceOrTile[ t ]	(6-47)

where NnonZero4x4BlksInSliceOrTile[ t ], NnonZero8x8BlksInSliceOrTile[ t ], NnonZero16x16BlksInSliceOrTile[ t ], NnonZero32x32BlksInSliceOrTile[ t ] are  the number of non-zero 4x4, 8x8, 16x16, 32x32 blocks in the slice[ t ] or tile[ t ] respectively.

NnonZeroBlksInSliceOrTile[ t ] is derived from portion_non_zero_blocks_area[ t ] And N4x4BlksInSliceOrTile[ t ] in the decoder.

portion_8x8_blocks_in_non_zero_area[ t ] indicates the portion of 8x8 blocks area in the non-zero area in the slice[ t ] or tile[ t ] and is set equal to PnonZero8x8BlksInSliceOrTile[ t ] defined as follows:

PnonZero8x8BlksInSliceOrTile[ t ]=Floor4 * NnonZero8x8BlksInSliceOrTile[ t ]NnonZeroBlksInSliceOrTile[ t ]*255	(6-48)

When not present, it is set equal to 0.

NnonZero8x8BlksInSliceOrTile[ t ] is the number of 8x8 blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ]. It is derived from portion_8x8_blocks_in_non_zero_area[ t ] and NnonZeroBlksInSliceOrTile[ t ] in the decoder.

portion_16x16_blocks_in_non_zero_area[ t ] indicates the portion of 16x16 blocks area in the non-zero area in the slice[ t ] or tile[ t ] and is set equal to PnonZero16x16BlksInSliceOrTile[ t ] defined as follows:

PnonZero16x16BlksInSliceOrTile[ t ]=Floor16 * NnonZero16x16BlksInSliceOrTile[ t ]NnonZeroBlksInSliceOrTile[ t ]*255	(6-49)

When not present, it is set equal to 0.

NnonZero16x16BlksInSliceOrTile[ t ] is the number of 16x16 blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ]. It is derived from portion_16x16_blocks_in_non_zero_area[ t ] and NnonZeroBlksInSliceOrTile[ t ] in the decoder.

portion_32x32_blocks_in_non_zero_area[ t ] indicates the portion of 32x32 blocks area in the non- zero area in the slice[ t ] or tile[ t ] and is set equal to PnonZero32x32BlksInSliceOrTile[ t ] defined as follows:

PnonZero32x32BlksInSliceOrTile[ t ]=Floor64 * NnonZero32x32BlksInSliceOrTile[ t ]NnonZeroBlksInSliceOrTile[ t ]*255	(6-50)

When not present, it is set equal to 0.

NnonZero32x32BlksInSliceOrTile[ t ] is the number of 32x32 blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ]. It is derived from portion_32x32_blocks_in_non_zero_area[ t ] and NnonZeroBlksInSliceOrTile[ t ] in the decoder.

NnonZero4x4BlksInSliceOrTile[ t ] is the number of 4x4 blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ]. It is derived from NnonZeroBlksInSliceOrTile[ t ],  NnonZero8x8BlksInSliceOrTile[ t ], NnonZero16x16BlksInSliceOrTile[ t ] and NnonZero32x32BlksInSliceOrTile[ t ] as follows in the decoder:

NnonZero4x4BlksInSliceOrTile t =NnonZeroBlksInSliceOrTile t

- 4*NnonZero8x8BlksInSliceOrTile t -16*NnonZero16x16BlksInSliceOrTile[ t ]

- 64*NnonZero32x32BlksInSliceOrTile[ t ]	(6-51)

portion_intra_predicted_blocks_area[ t ] indicates the portion of area covered by intra predicted blocks in the slice[ t ] or tile[ t ] using 8x8 granularity and is set equal to PintraBlks[ t ] defined as follows:

PintraBlks[ t ]=Floor4 * NintraBlksInSliceOrTile[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-52)

NintraBlksInSliceOrTile[ t ] is the number of intra predicted blocks using 8x8 granularity in the slice[ t ] or tile[ t ]. At the encoder side, it is computed as follows:

NintraBlksInSliceOrTile t =Nintra8x8BlksInSliceOrTile t

+ 4*Nintra16x16BlksInSliceOrTile t  + 16*Nintra32x32BlksInSliceOrTile t

+ 64*Nintra64x64BlksInSliceOrTile[ t ]	(6-53)

where Nintra8x8BlksInSliceOrTile[ t ], Nintra16x16BlksInSliceOrTile[ t ], Nintra32x32BlksInSliceOrTiles[ t ] and Nintra64x64BlksInSliceOrTile[ t ] are  the number of intra predicted 8x8, 16x16, 32x32 and 64x64 blocks in the slice[ t ] or tile[ t ] respectively.

NintraBlksInSliceOrTile[ t ] is derived from portion_intra_predicted_blocks_area[ t ] and N4x4BlksInSliceOrTile[ t ] in the decoder.

portion_planar_blocks_in_intra_area[ t ] indicates the portion of planar blocks in the intra predicted area the slice[ t ] or tile[ t ] and is set equal to PplanarBlksInIntra[ t ] defined as follows:

PplanarBlksInIntra[ t ]=FloorNplanarBlksInSliceOrTile[ t ]4 * NintraBlksInSliceOrTile[ t ]*255	(6-54)

When not present, it is set equal to 0.

NplanarBlksInSliceOrTile[ t ] is the number of intra planar predicted blocks in the slice[ t ] or tile[ t ] using 4x4 granularity. At the encoder side, it is computed as follows:

NplanarBlksInSliceOrTile t =Nplanar4x4Blks t

+ 4*Nplanar8x8Blks t +16*Nplanar16x16Blks t

+ 64*Nplanar32x32Blks t +256*Nplanar64x64Blks t 	(6-55)

where Nplanar4x4Blks[ t ], Nplanar8x8Blks[ t ], Nplanar16x16Blks[ t ], Nplanar32x32Blks[ t ] and Nplanar64x64Blks[ t ] are  the number of intra planar predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks in the slice[ t ] or tile[ t ] respectively.

NplanarBlksInSliceOrTile[ t ] is derived from portion_planar_blocks_in_intra_area[ t ] and NintraBlksInSliceOrTile[ t ] in the decoder.

portion_dc_blocks_in_intra_area[ t ] indicates the portion of DC blocks in the intra predicted area in the slice[ t ] or tile[ t ] and is set equal to PDCBlksInIntra[ t ] defined as follows:

PDCBlksInIntra[ t ]=FloorNDCBlksInSliceOrTile[ t ]4 * NintraBlksInSliceOrTile[ t ]*255	(6-56)

When not present, it is set equal to 0.

NDCBlksInSliceOrTile[ t ] is the number of intra DC predicted blocks in the slice[ t ] or tile[ t ] using 4x4 granularity. At the encoder side, it is computed as follows:

NDCBlksInSliceOrTile t =NDC4x4Blks t +4*NDC8x8Blks t

+ 16*NDC16x16Blks t +64*NDC32x32Blks[ t ]

+ 256*NDC64x64Blks[ t ]  	(6-57)

where NDC4x4Blks[ t ], NDC8x8Blks[ t ], NDC16x16Blks[ t ], NDC32x32Blks[ t ] and NDC64x64Blks[ t ] are  the number of intra DC predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks in the slice[ t ] or tile[ t ] respectively.

NDCBlksInSliceOrTile[ t ] is derived from portion_dc_blocks_in_intra_area[ t ] and NintraBlksInSliceOrTile[ t ] in the decoder.

portion_angular_hv_blocks_in_intra_area[ t ] indicates the portion of angular horizontal or vertical blocks in the intra predicted area in the slice[ t ] or tile[ t ] and is set equal to PHVBlksInIntra[ t ] defined as follows:

PHVBlksInIntra[ t ]=FloorNHVBlksInSliceOrTile[ t ]4 * NintraBlksInSliceOrTile[ t ]*255	(6-58)

When not present, it is set equal to 0.

NHVBlksInSliceOrTile[ t ] is the number of intra angular horizontally or vertically predicted blocks in the slice[ t ] or tile[ t ] using 4x4 granularity. At the encoder side, it is computed as follows:

NHVBlksInSliceOrTile t =NHV4x4Blks t +4*NHV8x8Blks t

+ 16*NHV16x16Blks t +64*NHV32x32Blks t +

+ 256*NHV64x64Blks[ t ] 	 (6-59)

where NHV4x4Blks[ t ], NHV8x8Blks[ t ], NHV16x16Blks[ t ], NHV32x32Blks[ t ] and NHV64x64Blks[ t ] are  the number of intra angular horizontally or vertically predicted 4x4, 8x8, 16x16, 32x32 and 64x64 blocks in the slice[ t ] or tile[ t ] respectively.

NHVBlksInSliceOrTile[ t ] is derived from portion_angular_hv_blocks_in_intra_area[ t ] and NintraBlksInSliceOrTile[ t ] in the decoder.

portion_blocks_a_c_d_n_filterings[ t ] indicates the portion of prediction blocks whose luma samples position are located in sub-sample position a, c, d or n, as defined in Annex B, in the slice[ t ] or tile[ t ]. When not present, is equal to 0. portion_blocks_a_c_d_n_filterings[ t ] is set equal to PacdnFiltBlks[ t ] defined as follows:

PacdnFiltBlks[ t ]=FloorNacdnFiltBlks[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-60)

NacdnFiltBlks[ t ] is the number of prediction blocks whose luma samples position are located in sub-sample position a, c, d or n, as defined in Annex B, in the slice[ t ] or tile[ t ]. It is derived from portion_blocks_a_c_d_n_filterings[ t ] and N4x4BlksInSliceOrTile[ t ] in the decoder.

portion_blocks_h_b_filterings[ t ] indicates the portion of prediction blocks whose luma samples position are located in sub-sample position h or b, as defined in Annex B, in the slice[ t ] or tile[ t ]. When not present, is equal to 0. portion_blocks_h_b_filterings[ t ] is set equal to PhbFiltBlks[ t ] defined as follows:

PhbFiltBlks[ t ]=FloorNhbFiltBlks[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-61)

NhbFiltBlks[ t ] is the number of prediction blocks whose luma samples position are located in sub-sample position h or b, as defined in Annex B, in the slice[ t ] or tile[ t ]. It is derived from portion_blocks_h_b_filterings[ t ] And N4x4BlksInSliceOrTile[ t ]  in the decoder.

portion_blocks_f_i_k_q_filterings[ t ] indicates the portion of prediction blocks whose luma samples position are located in sub-sample position f, i, k or q, as defined in Annex B, in the slice[ t ] or tile[ t ]. When not present, is equal to 0. portion_blocks_f_i_k_q_filterings[ t ] is set equal to PfikqFiltBlks[ t ] defined as follows:

PfikqFiltBlks[ t ]=FloorNfikqFiltBlks[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-62)

NfikqFiltBlks[ t ] is the number of prediction blocks whose luma samples position are located in sub-sample position f, i, k or q, as defined in Annex B, in the slice[ t ] or tile[ t ]. It is derived from portion_blocks_f_i_k_q_filterings[ t ] And N4x4BlksInSliceOrTile[ t ]  in the decoder.

portion_blocks_j_filterings[ t ] indicates the portion of prediction blocks whose luma samples position are located in sub-sample position j, as defined in Annex B, in the slice[ t ] or tile[ t ]. When not present, is equal to 0. portion_blocks_j_filterings[ t ] is set equal to PjFiltBlks[ t ] defined as follows:

PjFiltBlks[ t ]=FloorNjFiltBlks[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-63)

NjFiltBlks[ t ] is the number of prediction blocks whose luma samples position are located in sub-sample position j, as defined in Annex B, in the slice[ t ] or tile[ t ]. It is derived from portion_blocks_j_filterings[ t ] And N4x4BlksInSliceOrTile[ t ]  in the decoder.

portion_blocks_e_g_p_r_filterings[ t ] indicates the portion of prediction blocks whose luma samples position are located in sub-sample position e, g, p or r, as defined in Annex B, in the slice[ t ] or tile[ t ]. When not present, is equal to 0. portion_blocks_ e_g_p_r_filterings[ t ] is set equal to PegprFiltBlks[ t ] defined as follows:

PegprFiltBlks[ t ]=FloorNegprFiltBlks[ t ]N4x4BlksInSliceOrTile[ t ]*255	(6-64)

NegprFiltBlks[ t ] is the number of prediction blocks whose luma samples position are located in sub-sample position e, g, p or r, as defined in Annex B, in the slice[ t ] or tile[ t ]. It is derived from portion_blocks_e_g_p_r_filterings[ t ] And N4x4BlksInSliceOrTile[ t ]  in the decoder.

portion_deblocking_instances[ t ] indicates the portion of deblocking filtering instances in the slice[ t ] or tile[ t ]. portion_deblocking_instances[ t ] is set equal to PdbfInstances[ t ] defined as follows:

PdbfInstances[ t ]=FloorNdbfInstances[ t ]4*SchrMult *N4x4BlksInSliceOrTile[ t ]*255

(6-65)

NdbfInstances[ t ] is the number of deblocking filtering instances in the slice[ t ] or tile[ t ]. It is derived from portion_deblocking_instances[ t ], N4x4BlksInSliceOrTile[ t ] and SchrMult in the decoder.

#### VVC semantics

The semantics of various terms are defined below.

period_type specifies the type of upcoming period over which the complexity metrics are applicable and is defined in the Table 9.

Table 9 — specification of period_type for VVC

| Value | Description |
| --- | --- |
| 0x0 | complexity metrics are applicable to a single picture |
| 0x1 | complexity metrics are applicable to all pictures in decoding order, up to (but not including) the picture containing the next I slice |
| 0x2 | complexity metrics are applicable to all pictures over a specified time interval in seconds |
| 0x3 | complexity metrics are applicable over a specified number of pictures counted in decoding order |
| 0x4-0xF | user-defined |

granularity_type indicates the type of granularity which the complexity metrics are applicable and is defined in the Table 10.

Table 10 — specification of granularity_type for VVC

| Value | Description |
| --- | --- |
| 0x0 | complexity metrics are applicable to picture granularity |
| 0x1 | complexity metrics are applicable to sub-picture granularity |
| 0x2 | complexity metrics are applicable to slice granularity |
| 0x3 | complexity metrics are applicable to tile granularity |
| 0x4-0x7 | user-defined |

extended_representation_flag equal to 1 indicates that the syntax elements portion_non_zero_4_8_16_blocks_area, portion_non_zero_32_64_128_blocks_area, portion_non_zero_256_512_1024_blocks_area, portion_non_zero_2048_4096_blocks_area, portion_bi_and_gpm_predicted_blocks_area, portion_bdof_blocks_area, portion_sao_filtered_blocks, portion_non_zero_4_8_16_blocks_area[ t ], portion_non_zero_32_64_128_blocks_area[ t ], portion_non_zero_256_512_1024_blocks_area[ t ], portion_non_zero_2048_4096_blocks_area[ t ], portion_bi_and_gpm_predicted_blocks_area[ t ], portion_bdof_blocks_area[ t ] and portion_sao_filtered_blocks[ t ] may be present. extended_representation_flag equal to 0 indicates that these syntax elements are not present.

num_seconds indicates the number of seconds over which the complexity metrics are applicable when period_type is 2.

num_pictures indicates the number of pictures, counted in decoding order, over which the complexity metrics are applicable when period_type is 3.

NpicsInPeriod indicates the number of pictures in the specified period. When period_type is 0, then NpicsInPeriod is 1. When period_type is 1, then NpicsInPeriod is determined by counting the pictures in decoding order up to (but not including) the one containing the next I slice. When period_type is 2, then NpicsInPeriod is determined from the frame rate. When period_type is 3, then NpicsInPeriod is equal to num_pictures.

N4SampleBlksInPeriod is the total number of 4-samples luma and chroma blocks that are coded in the specified period.

It is determined by the following computation:

N4SampleBlksInPeriod=n=1NpicsInPeriodN4SampleBlksPic(n)

(6-66)

where N4SampleBlksPic(n) is derived for the nth picture within the specified period 1 <= n <= NpicsInPeriod from VVC variables PicSizeInCtbsY and CtbLog2SizeY specified for the decoding process of the nth picture within the specified period, as follows:

N4SampleBlksPic(n) = SchrMult * SpicInCtb * Nctbs	(6-67)

where

—	Nctbs is set equal to ( 1 << ( CtbLog2SizeY – 1 ) )2

—	SpicInCtb is set equal to PicSizeInCtbsY

—	SchrMult depends on the VVC variable sps_chroma_format_idc as shown in the Table 11

Table 11 — specification of SchrMult for VVC

| SchrMult | sps_chroma_format_idc | Comment |
| --- | --- | --- |
| 1 | 0 | monochrome |
| 1.5 | 1 | 4:2:0 sampling |
| 2 | 2 | 4:2:2 sampling |
| 3 | 3 | 4:4:4 sampling |

portion_non_zero_blocks_area indicates the portion of area covered by blocks with non-zero transform coefficients values, in the pictures of the specified period, using 4-samples block granularity and is set equal to PnonZeroBlksArea defined as follows:

PnonZeroBlksArea=FloorNnonZeroBlksN4SampleBlksInPeriod*255	(6-68)

where NnonZeroBlks is the number of blocks with non-zero transform coefficients values in the specified period using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZeroBlks= X=4,8,16,32,64,128,256,512,1024,2048,4096X4*NnonZeroBlks_X

(6-69)

where NnonZeroBlks_X is the number of blocks with non-zero transform coefficients values, for transform blocks with number of samples X=4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, respectively, in the specified period.

NnonZeroBlks is derived from portion_non_zero_blocks_area and N4SampleBlksInPeriod in the decoder.

portion_non_zero_4_8_16_blocks_area indicates the portion of 4-, 8- and 16-samples blocks area in the non-zero area in the specified period and is set equal to PnonZero4_8_16_Blks defined as follows:

PnonZero4_8_16_Blks=FloorNnonZero4_8_16_BlksNnonZeroBlks*255	(6-70)

When not present, portion_non_zero_4_8_16_blocks_area is set equal to 0.

NnonZero4_8_16_Blks is the number of 4-, 8- and 16-samples transform blocks with non-zero transform coefficients values in the specified period using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero4_8_16_Blks= X=4,8,16X4*NnonZeroBlks_X

(6-71)

NnonZero4_8_16_Blks is derived from portion_non_zero_4_8_16_blocks_area and NnonZeroBlks in the decoder.

portion_non_zero_32_64_128_blocks_area indicates the portion of 32-, 64- and 128-samples blocks area in the non-zero area in the specified period and is set equal to PnonZero32_64_128_Blks defined as follows:

PnonZero32_64_128_Blks =FloorNnonZero32_64_128_BlksNnonZeroBlks*255	(6-72)

When not present, portion_non_zero_32_64_128_blocks_area is set equal to 0.

NnonZero32_64_128_Blks is the number of 32-, 64- and 128-samples transform blocks with non-zero transform coefficients values in the specified period using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero32_64_128_Blks=X=32,64,128X4*NnonZeroBlks_X

(6-73)

NnonZero32_64_128_Blks is derived from portion_non_zero_32_64_128_blocks_area and NnonZeroBlks in the decoder.

portion_non_zero_256_512_1024_blocks_area indicates the portion of 256-, 512- and 1024-samples blocks area in the non-zero area in the specified period and is set equal to PnonZero256_512_1024_Blks defined as follows:

PnonZero256_512_1024_Blks =FloorNnonZero256_512_1024_BlksNnonZeroBlks*255	(6-74)

When not present, portion_non_zero_256_512_1024_blocks_area is set equal to 0.

NnonZero256_512_1024_Blks is the number of 256-, 512- and 1024-samples transform blocks with non-zero transform coefficients values in the specified period using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero256_512_1024_Blks=X=256,512,1024X4*NnonZeroBlks_X

(6-75)

NnonZero256_512_1024_Blks is derived from portion_non_zero_256_512_1024_blocks_area and NnonZeroBlks in the decoder.

portion_non_zero_2048_4096_blocks_area indicates the portion of 2048- and 4096-samples blocks area in the non-zero area in the specified period and is set equal to PnonZero2018_4096_Blks defined as follows:

PnonZero2048_4096_Blks =FloorNnonZero2048_4096_BlksNnonZeroBlks*255	(6-76)

When not present, portion_non_zero_2048_4096_blocks_area is set equal to 0.

NnonZero2048_4096_Blks is the number of 2048- and 4096-samples transform blocks with non-zero transform coefficients values in the specified period using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero2048_4096_Blks= X=2048,4096X4*NnonZeroBlks_X

(6-77)

NnonZero2048_4096_Blks is derived from portion_non_zero_2048_4096_blocks_area and NnonZeroBlks in the decoder.

portion_non_zero_transform_coefficients_area indicates the portion of area covered by non-zero transform coefficients in non-zero transform blocks in the pictures of the specified period, using 4-samples block granularity and is set equal to PnonZeroCoefsArea defined as follows:

PnonZeroCoefsArea =FloorNnonZeroTransformCoefs 4 * NnonZeroBlks*255	(6-78)

NnonZeroTransformCoefs is the area covered by non-zero transform coefficients in non-zero transform blocks in the specified period using 4-samples block granularity.

NnonZeroTransformCoefs is derived from portion_non_zero_transform_coefficients_area and NnonZeroBlks in the decoder.

portion_intra_predicted_blocks_area indicates the portion of area covered by intra predicted blocks in the pictures of the specified period using 4-samples block granularity and is set equal to PintraPredBlks defined as follows:

PintraPredBlks=FloorNintraPredBlksN4SampleBlksInPeriod*255	(6-79)

NintraPredBlks is the number of intra predicted blocks in the specified period using 4-samples block granularity.  At the encoder side, it is computed as follows:

NintraPredBlks=X=4,8,16,32,64,128,256,512,1024,2048,4096X4*NintraPredBlks_X

(6-80)

where NintraPredBlks_X  is the number of blocks using intra prediction, for blocks with number of samples X=4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, in the specified period.

NintraPredBlks is derived from portion_intra_predicted_blocks_area and N4SampleBlksInPeriod in the decoder.

portion_bi_and_gpm_predicted_blocks_area indicates the portion of area covered by inter bi-predicted or GPM-predicted blocks in the pictures of the specified period using 4-samples block granularity and is set equal to PbiGpmPredBlks defined as follows:

PbiGpmBlks=FloorNbiAndGpmPredBlksN4SampleBlksInPeriod*255	(6-81)

NbiAndGpmPredBlks is the number of inter bi-predicted and GPM-predicted blocks in the specified period using 4-samples block granularity.  At the encoder side, it is computed as follows:

NbiAndGpmPredBlks = X=4,8,16,32,64,128,256,512,1024,2048,4096X4*NbiAndGpmPredBlks_X

(6-82)

Where NbiAndGpmPredBlks_X are the number of blocks using inter bi-prediction or GPM prediction, for blocks with number of samples X=4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, in the specified period.

NbiAndGpmPredBlks is derived from portion_bi_and_gpm_predicted_blocks_area and N4SampleBlksInPeriod in the decoder.

portion_bdof_blocks_area indicates the portion of area covered by inter blocks using BDOF in the pictures of the specified period using 4-samples block granularity and is set equal to PbdofBlks defined as follows:

PbdofBlks=FloorNbdofBlksN4SampleBlksInPeriod*255	(6-83)

NbdofBlks is the number of inter blocks using BDOF in the specified period using 4-samples block granularity.  At the encoder side, it is computed as follows:

NbdofBlks=X=256,512,1024,2048,4096X/4*NbdofBlks_X

(6-84)

Where NbdofBlks_X are the number of inter-coded blocks using BDOF, for blocks with number of samples X=256, 512, 1024, 2048, 4096, in the specified period.

NbdofBlks is derived from portion_bdof_blocks_area and N4SampleBlksInPeriod in the decoder.

portion_deblocking_instances indicates the portion of deblocking filtering instances in the specified period and is set equal to PdbfInstances defined as follows:

PdbfInstances=FloorNdbfInstances4*N4SampleBlksInPeriod*255	(6-85)

NdbfInstances is the number of deblocking filtering instances in the specified period. It is derived from portion_deblocking_instances, and N4SampleBlksInPeriod in the decoder.

portion_sao_filtered_blocks indicates the portion of SAO filtered blocks in the specified period using 4-samples block granularity.  At the encoder side, it is set equal to PsaoBlks computed as follows:

PsaoBlks=FloorNsaoFilteredBlksN4SampleBlksInPeriod*255	(6-86)

NsaoFilteredBlks is the number of SAO filtered blocks in the specified period using 4-samples block granularity. It is derived from portion_sao_filtered_blocks, N4SampleBlksInPeriod in the decoder.

portion_alf_filtered_blocks indicates the portion of ALF filtered blocks in the specified period using 4-samples block granularity.  At the encoder side, it is set equal to PalfBlks computed as follows:

PalfBlks=FloorNalfFilteredBlksN4SampleBlksInPeriod*255	(6-87)

NalfFilteredBlks is the number of ALF filtered blocks in the specified period using 4-samples block granularity. It is derived from portion_alf_filtered_Blocks and N4SampleBlksInPeriod in the decoder.

max_num_segments_minus1 indicates the number of subpictures, slices or tiles in the associated picture.

segment_address[ t ] indicates the address of the tth segment. When granularity_type is equal to 1, segment_address[ t ] indicates the subpicture ID of the tth subpicture subpicture[ t ]. When granularity_type is equal to 2 or 3, segment_address[ t ] indicates the picture raster scan address of the first coding tree block (CTB) number in slice[ t ] or tile[ t ].

N4SampleBlksInSegment[ t ] is the total number of 4-samples luma and chroma blocks in the slice[ t ] or tile[ t ] or subpicture[ t ]. N4SampleBlksInSegment[ t ] is determined by the following computation.

—	If granularity_type is equal to 1, N4SampleBlksInSegment[ t ] is derived as follows from VVC variables sps_subpic_id, sps_subpic_width_minus1, sps_subpic_height_minus1 and CtbLog2SizeY specified in ISO/IEC 23090-3

  - s is defined as the index value such that sps_subpic_id[ s ] is equal to the subpicture ID segment_address[ t ].

  - Wsubpic is set equal to ( 1 + sps_subpic_width_minus1[ s ] ) << ( CtbLog2SizeY – 1 ).

  - Hsubpic is set equal to ( 1 + sps_subpic_height_minus1[ s ] ) << ( CtbLog2SizeY – 1 ).

  - N4SampleBlksInSegment[ t ] is set equal to ( SchrMult * Wsubpic * Hsubpic ).

- if granularity_type is equal to 2, N4SampleBlksInSegment[ t ] is derived as follows from VVC variables NumCtusInSlice and CtbLog2SizeY specified in ISO/IEC 23090-3:

- N4SampleBlksInSegment[ t ] is set equal to SchrMult * ( ( NumCtusInSlice[ t ] ) << ( CtbLog2SizeY – 1) )

- Otherwise, if granularity_type is equal to 3, N4SampleBlksInSegment[ t ] is derived as follows from VVC variables ctbToTileColIdx, ctbToTileRowIdx, ColWidthVal and RowHeightVal specified in ISO/IEC 23090-3:

- ctbAddrX is set equal to segment_address[ t ].

- tileColIdx is set equal to ctbToTileColIdx[ ctbAddrX ].

- tileRowIdx is set equal to ctbToTileRowIdx[ ctbAddrX ].

- Wtile is set equal to ColWidthVal[ tileColIdx ] << ( CtbLog2SizeY – 1 ).

- Htile is set equal to RowHeightVal[ tileRowIdx ] << ( CtbLog2SizeY – 1 ).

- N4SampleBlksInSegment[ t ] is set equal to ( SchrMult * Wtile * Htile ).

portion_non_zero_blocks_area[ t ] indicates the portion of area covered by blocks with non-zero transform coefficients values, in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZeroBlksArea[ t ] defined as follows:

PnonZeroBlksArea[ t ]=FloorNnonZeroBlksInSegment[ t ]N4SampleBlksInSegment[ t ]*255 	(6-88)

where NnonZeroBlksInSegment[ t ] is the number of blocks with non-zero transform coefficients values, in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZeroBlksInSegment[ t ]= X=4,8,16,32,64,128,256,512,1024,2048,4096X/4*NnonZeroBlksInSegment_X[ t ]

(6-89)

where NnonZeroBlksInSegment_X[ t ] is the number of blocks with non-zero transform coefficients values, in the slice[ t ] or tile[ t ] or subpicture[ t ], for blocks with number of samples X=4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, respectively.

NnonZeroBlksInSegment[ t ] is derived from portion_non_zero_blocks_area[ t ] and N4SampleBlksInSegment[ t ] in the decoder.

portion_non_zero_4_8_16_blocks_area[ t ] indicates the portion of 4-, 8- and 16-samples blocks area in the non-zero area in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZero4_8_16_Blks[ t ] defined as follows:

PnonZero4_8_16_Blks[ t ] =FloorNnonZero4_8_16_BlksInSegment[ t ]NnonZeroBlksInSegment[ t ]*255	(6-90)

When not present, portion_non_zero_4_8_16_blocks_area[ t ] is set equal to 0.

NnonZero4_8_16_BlksInSegment[ t ] is the number of 4-, 8- and 16-samples blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero4_8_16_BlksInSegment[ t ] = X=4,8,16X4*NnonZeroBlksInSegment_X[ t ]

(6-91)

NnonZero4_8_16_BlksInSegment[ t ] is derived from portion_non_zero_4_8_16_blocks_area[ t ] and NnonZeroBlksInSegment[ t ] in the decoder.

portion_non_zero_32_64_128_blocks_area[ t ] indicates the portion of 32-, 64- and 128-samples blocks area in the non-zero area in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZero32_64_128_Blks[ t ] defined as follows:

PnonZero32_64_128_Blks[ t ] =FloorNnonZero32_64_128_BlksInSegment[ t ]NnonZeroBlksInSegment[ t ]*255	(6-92)

When not present, portion_non_zero_32_64_128_blocks_area[ t ] is set equal to 0.

NnonZero32_64_128_BlksInSegment[ t ] is the number of 32-, 64- and 128-samples blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero32_64_128_BlksInSegment[ t ] = X=32,64,128X4*NnonZeroBlksInSegment_X[ t ]

(6-93)

NnonZero32_64_128_BlksInSegment[ t ] is derived from portion_non_zero_32_64_128_blocks_area[ t ] and NnonZeroBlksInSegment[ t ] in the decoder.

portion_non_zero_256_512_1024_blocks_area[ t ] indicates the portion of 256-, 512- and 1024-samples blocks area in the non-zero area in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZero256_512_1024_Blks[ t ] defined as follows:

PnonZero256_512_1024_Blks[ t ] =FloorNnonZero256_512_1024_BlksInSegment[ t ]NnonZeroBlksInSegment[ t ]*255	(6-94)

When not present, portion_non_zero_256_512_1024_blocks_area[ t ] is set equal to 0.

NnonZero256_512_1024_BlksInSegment[ t ] is the number of 256-, 512- and 1024-samples blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero256_512_1024_BlksInSegment[ t ] = X=256,512,1024X4*NnonZeroBlksInSegment_X[ t ]

(6-95)

NnonZero256_512_1024_BlksInSegment[ t ] is derived from portion_non_zero_256_512_1024_blocks_area[ t ] and NnonZeroBlksInSegment[ t ] in the decoder.

portion_non_zero_2048_4096_blocks_area[ t ] indicates the portion of 2048- and 4096-samples blocks area in the non-zero area in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZero2048_4096_Blks[ t ] defined as follows:

PnonZero2048_4096_Blks[ t ] =FloorNnonZero2048_4096_BlksInSegment[ t ]NnonZeroBlksInSegment[ t ]*255	(6-96)

When not present, portion_non_zero_2048_4096_blocks_area[ t ] is set equal to 0.

NnonZero2048_4096_BlksInSegment[ t ] is the number of 2048- and 4096-samples blocks with non-zero transform coefficients values in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. At the encoder side, it is computed as follows:

NnonZero2048_4096_BlksInSegment[ t ] = X=2048,4096X4*NnonZeroBlksInSegment_X[ t ]

(6-97)

NnonZero2048_4096_BlksInSegment[ t ] is derived from portion_non_zero_2048_4096_blocks_area[ t ] and NnonZeroBlksInSegment[ t ] in the decoder.

portion_non_zero_transform_coefficients_area[ t ] indicates the portion of area covered by non-zero transform coefficients in non-zero transform blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PnonZeroCoefsArea[ t ] defined as follows:

PnonZeroCoefsArea=FloorNnonZeroTransformCoefs[ t ]4 * NnonZeroBlksInSegment[ t ]*255	(6-98)

NnonZeroTransformCoefs[ t ] is the area covered by non-zero transform coefficients in non-zero blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.

NnonZeroTransformCoefs[ t ] is derived from portion_non_zero_transform_coefficients_area[ t ] and NnonZeroBlksInSegment[ t ] in the decoder.

portion_intra_predicted_blocks_area[ t ] indicates the portion of area covered by intra predicted blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PintraPredBlks[ t ] defined as follows:

PintraPredBlks[ t ]=FloorNintraPredBlks[ t ]N4SampleBlksInSegment[ t ]*255	(6-99)

NintraPredBlks[ t ] is the number of intra predicted blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.  At the encoder side, it is computed as follows:

NintraPredBlks[ t ]= X=4,8,16,32,64,128,256,512,1024,2048,4096X4*NintraPredBlks_X[ t ]

(6-100)

Where NintraPredBlks_X[ t ] is the number of blocks using intra prediction, for blocks with number of samples X=4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, in the slice[ t ] or tile[ t ] or subpicture[ t ].

NintraPredBlks[ t ] is derived from portion_intra_predicted_blocks_area[ t ] and N4SampleBlksInSegment[ t ] in the decoder.

portion_bi_and_gpm_predicted_blocks_area[ t ] indicates the portion of area covered by inter bi-predicted or GPM-predicted blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PbiGpmPredBlks[ t ] defined as follows:

PbiGpmPredBlks[ t ]=FloorNbiAndGpmPredBlks[ t ]N4SampleBlksInSegment[ t ]*255	(6-101)

NbiAndGpmPredBlks[ t ] is the number of inter bi-predicted and GPM-predicted blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.  At the encoder side, it is computed as follows:

NbiAndGpmPredBlks[ t ] = X=16,32,64,128,256,512,1024,2048,4096X4*NbiAndGpmPredBlks_X[ t ]

(6-102)

Where NbiAndGpmPredBlks_X[ t ] are the number of blocks using inter bi-predicted prediction, for blocks with number of samples X=16, 32, 64, 128, 256, 512, 1024, 2048, 4096, in the slice[ t ] or tile[ t ] or subpicture[ t ].

NbiAndGpmPredBlks[ t ] is derived from portion_bi_and_gpm_predicted_blocks_area[ t ] and N4SampleBlksInSegment[ t ] in the decoder.

portion_bdof_blocks_area[ t ] indicates the portion of area covered by inter blocks using BDOF in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PbdofBlks[ t ] defined as follows:

PbdofBlks[ t ]=FloorNbdofBlks[ t ]N4SampleBlksInSegment[ t ]*255	(6-103)

NbdofBlks[ t ] is the number of inter blocks using BDOF in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.  At the encoder side, it is computed as follows:

NbdofBlks  t = X=256,512,1024,2048,4096X4*NbdofBlks_X

(6-104)

Where NbdofBlks_X[ t ] are the number of blocks using inter blocks using BDOF, for blocks with number of samples X=256, 512, 1024, 2048, 4096, in the slice[ t ] or tile[ t ] or subpicture[ t ].

NbdofBlks[ t ] is derived from portion_bdof_blocks_area[ t ] and N4SampleBlksInSegment[ t ] in the decoder.

portion_deblocking_instances[ t ] indicates the portion of deblocking filtering instances in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity and is set equal to PdbfInstances[ t ] defined as follows:

PdbfInstances[ t ]=FloorNdbfInstances[ t ]4*N4SampleBlksInSegment[ t ]*255	(6-105)

NdbfInstances[ t ] is the number of deblocking filtering instances in the specified period. It is derived from portion_deblocking_instances[ t ] and N4SampleBlksInSegment[ t ] in The slice[ t ] or tile[ t ] or subpicture[ t ].

portion_sao_filtered_blocks[ t ] indicates the portion of SAO filtered blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.  At the encoder side, it is set equal to PsaoBlks[ t ] computed as follows:

PsaoBlks[ t ] =FloorNsaoFilteredBlks[ t ]N4SampleBlksInSegment[ t ]*255	(6-106)

NsaoFilteredBlks[ t ] is the number of SAO filtered blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. It is derived from portion_sao_filtered_blocks[ T ], N4SampleBlksInSegment[ t ] in the decoder.

portion_alf_filtered_blocks[ t ] indicates the portion of ALF filtered blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity.  At the encoder side, it is set equal to PalfBlks[ t ]  computed as follows:

PalfBlks[ t ] =FloorNalfFilteredBlks[ t ]N4SampleBlksInSegment[ t ]*255	(6-107)

NalfFilteredBlks[ t ] is the number of ALF filtered blocks in the slice[ t ] or tile[ t ] or subpicture[ t ], using 4-samples block granularity. It is derived from portion_alf_filtered_blocks[ t ], N4SampleBlksInSegment[ t ] in the decoder.

#### EVC Semantics

##### General

As EVC Baseline profile and Main profile share almost no tools and the methods used for partition of the pictures are not same, the profile the CVS conforming to is used to decide the set of syntax elements to describe the complexity metrics to be applied to each CVS. In addition, As the largest size of picture is indicated by the level the CVS is conformed to, the length of the syntax elements indicating the number of pixels and coding units are decided by the level. In addition, the width and height of the coding units are also considered when the length of the syntax elements indicating the number of coding units for the CVS conforming to the baseline profile is decided.

##### Variable length syntax element for EVC Semantics

The maximum number of pixels and coding units depend on the size of the picture the complexity metric is applied to. As the largest size of picture is indicated by the level the CVS is conformed to the length of the syntax elements indicating the number of pixels and coding units are decided by the levels. In addition, the width and height of the coding units are also considered when the length of the syntax elements indicating the number of coding units for the CVS conforming to the baseline profile as the width and height of the coding units get larger than the maximum number of coding units get smaller.

– uk(v): the field is unsigned integer and the length is decided by the value of the level_idc field in the SPS used by the CVS this SEI message is applied to and the value of k assigned to each field based on the size of the units counted. The length of the field according to each value of both level_idc and k is shown in the Table 12.

Table 12 — length of uk(v)

| level | value of level_idc | length of the field |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | k==1 | k==4 | k==8 | k==16 | k==32 | k==64 |
| 1 | 30 | 16 | 16 | 16 | 8 | 8 | 8 |
| 2 | 60 | 24 | 16 | 16 | 16 | 8 | 8 |
| 2.1 | 63 | 24 | 16 | 16 | 16 | 8 | 8 |
| 3 | 90 | 24 | 16 | 16 | 16 | 16 | 8 |
| 3.1 | 93 | 24 | 16 | 16 | 16 | 16 | 8 |
| 4 | 120 | 24 | 24 | 16 | 16 | 16 | 16 |
| 4.1 | 123 | 24 | 24 | 16 | 16 | 16 | 16 |
| 5 | 150 | 24 | 24 | 24 | 16 | 16 | 16 |
| 5.1 | 153 | 24 | 24 | 24 | 16 | 16 | 16 |
| 5.2 | 156 | 24 | 24 | 24 | 16 | 16 | 16 |
| 6 | 180 | 32 | 24 | 24 | 24 | 16 | 16 |
| 6.1 | 183 | 32 | 24 | 24 | 24 | 16 | 16 |
| 6.2 | 186 | 32 | 24 | 24 | 24 | 16 | 16 |

##### Semantics of EVC CM

profile_idc indicates the profile of the CVS this SEI message is associated with.

period_type specifies the type of upcoming period over which the complexity metrics are applicable and is defined in the Table 13.

Table 13 — specification of period_type for EVC

| Value | Description |
| --- | --- |
| 0x0 | complexity metrics are applicable to a single picture |
| 0x1 | complexity metrics are applicable over a specified number of pictures counted in decoding order |
| 0x2 | complexity metrics are applicable to a single slice |
| 0x3 | complexity metrics are applicable to a specified number of slices counted in decoding order |
| 0x4-0xF | user-defined |

num_non_zero_4_cus indicates the number of coding units whose width and height are not greater than 4 samples and have non-zero transform coefficients values in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 4.

num_non_zero_8_cus indicates the number of coding units whose width and height are 8 samples and have non-zero transform coefficients values in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 8.

num_non_zero_16_cus indicates the number of coding units whose width and height are 16 samples and have non-zero transform coefficients values in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 16.

num_non_zero_32_cus indicates the number of coding units whose width and height are 32 samples and have non-zero transform coefficients values in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field applied to and the value of the k is equal to 32.

num_non_zero_64_cus indicates the number of coding units whose width and height are 32 samples and have non-zero transform coefficients values in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 64.

portion_fractional_prediction_sample indicates the portion of the samples requires fractional sample interpolation process applied in the period complexity metrics is applied to. It is defined as follows:

portion_fractional_prediction_sample=Floornumber of samples requiring fractional sample interpolation process total number of samples in the period*255	(6-108)

num_count indicates the number of pictures or slices over which the complexity metrics is applicable when period_type is 1, 3, or 5.

When period_type is 1, then num_count indicates the number of pictures in decoding order. When period_type is 3, then num_count indicates the number of slices in decoding order.

num_non_zero_4_cus [t] indicates the number of coding units whose width and height are not greater than 4 samples and have non-zero transform coefficients values in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to with and the value of the k is equal to 4. When period_type is 1, the period this metrics applied is tth picture in decoding order. When period_type is 3, the period this metrics applied is tth slice in decoding order.

num_non_zero_8_cus [t] indicates the number of coding units whose width and height are 8 samples and have non-zero transform coefficients values in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 8. When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_non_zero_16_cus [t] indicates the number of coding units whose width and height are 16 samples and have non-zero transform coefficients values in the period (t+1)th complexity metrics is applied to . The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to with and the value of the k is equal to 16. When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_non_zero_32_cus [t] indicates the number of coding units whose width and height are 32 samples and have non-zero transform coefficients values in the period (t+1)th complexity metrics is applied to . The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 32. When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_non_zero_64_cus [t] indicates the number of coding units whose width and height are 64 samples and have non-zero transform coefficients values in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 464 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

portion_fractional_prediction_sample [t] indicates the portion of the samples requires fractional sample interpolation process applied to in the period (t+1)th complexity metrics is applied. When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_non_zero_samples indicates the number of non-zero transform coefficients in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_affine_samples indicates the number of pixels affine model based motion compensation is applied to in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_dmvr_samples indicates the number of pixels decoder-side motion vector refinement is applied to in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_alf_samples indicates the number of pixels adaptive loop filter is applied to in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_deblocking_filter_samples indicates the number of pixels deblocking filter is applied to in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_htdf_samples indicates the number of pixels hadamard transform domain filter is applied to in the period complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS applied the CVS the SEI message this field is associated with and the value of the k is equal to 1.

num_non_zero_samples [t] indicates the number of non-zero transform coefficients in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1. When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_affine_samples [t] indicates the number of pixels affine model based motion compensation is applied to in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_dmvr_samples [t] indicates the number of pixels decoder-side motion vector refinement is applied to in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_alf_samples [t] indicates the number of pixels adaptive loop filter is applied to in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_deblocking_filter_samples [t] indicates the number of pixels deblocking filter is applied to in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

num_htdf_samples [t] indicates the number of pixels Hadamard transform domain filter is applied to in the period (t+1)th complexity metrics is applied to. The length of this field is decided by the value of the level_idc field in the SPS used by the CVS the SEI message this field is applied to and the value of the k is equal to 1 . When period_type is 1, the period this metrics applied is (t+1)th picture in decoding order. When period_type is 3, the period this metrics applied is (t+1)th slice in decoding order.

## Interactive signalling for remote decoder-power reduction

### General

For point-to-point video conferencing, each device contains a transmitter and a receiver. A local device sends metadata that instructs the remote device to modify the decoding complexity of the bitstream and thus reduce local decoder-power consumption.

### Syntax

The syntax for interactive signalling for remote decoder-power reduction is described in Table 14.

Table 14 — syntax for interactive signalling for remote decoder-power reduction

|  | Descriptor |
| --- | --- |
| dec_pow_reduction_type | u(2) |
| if( dec_pow_reduction_type = = 0 ) |  |
| dec_ops_reduction_req | s(6) |
| else if( dec_pow_reduction_type = = 1 ) { |  |
| disable_loop_filters | u(1) |
| disable_bi_prediction | u(1) |
| disable_intra_in_B | u(1) |
| disable_fracpel_filtering | u(1) |
| user_defined_req | u(2) |
| } |  |
| else if( dec_pow_reduction_type = = 2 ) { |  |
| pic_width_in_luma_samples | u(14) |
| pic_height_in_luma_samples | u(14) |
| frames_per_second | u(10) |
| } |  |
| else if( dec_pow_reduction_type = = 3 ) { |  |
| dec_pow_reduction_extension_type | u(2) |
| if( dec_pow_reduction_extension_type == 0 ) { |  |
| nb_dec_pow_reduction_type_req | u(2) |
| for( i = 0 ; i < nb_dec_pow_reduction_type_req; i++ ) |  |
| dec_pow_reduction_type_req_id[ i ] | u(2) |
| } |  |
| } |  |

The transmitter then uses the message format described in Table 15 to acknowledge the request of the decoding operation reduction from the receiver:

Table 15 — syntax for interactive signalling from the transmitter to acknowledge remote decoder-power reduction

|  | Descriptor |
| --- | --- |
| dec_pow_reduction_type_resp | u(2) |
| if( dec_pow_reduction_type_resp = = 0 ) |  |
| dec_ops_reduction_resp | s(6) |
| else if( dec_pow_reduction_type_resp = = 1 ) { |  |
| disabled_loop_filters_resp | u(1) |
| disabled_bi_prediction_resp | u(1) |
| disabled_intra_in_B_resp | u(1) |
| disabled_fracpel_filtering_resp | u(1) |
| user_defined_resp | u(2) |
| } |  |
| else if( dec_pow_reduction_type_resp = = 2 ) { |  |
| pic_width_in_luma_samples_resp | u(14) |
| pic_height_in_luma_samples_resp | u(14) |
| frames_per_second_resp | u(10) |
| } |  |
| else if( dec_pow_reduction_type_resp = = 3 ) { |  |
| dec_pow_reduction_extension_type_resp | u(2) |
| if( dec_pow_reduction_extension_type_resp = = 0 ) { |  |
| nb_dec_pow_reduction_type_resp | u(2) |
| if( nb_dec_pow_reduction_type_resp = = 0 ) { |  |
| pic_width_in_luma_samples_resp |  |
| pic_height_in_luma_samples_resp |  |
| frames_per_second_resp |  |
| } |  |
| else |  |
| for( i = 0 ; i < nb_dec_pow_reduction_type_resp; i++ ) |  |
| dec_pow_reduction_type_resp_id[ i ] | u(2) |
| } |  |
| } |  |

### Signalling

The transmitter in each device sends a decoding operation reduction request (DOR-Req) message to the attention of the remote encoder. In a first mode (dec_pow_reduction_type equal to 0), this message requests the remote encoder to adjust its encoding parameters so that ideally, when the local decoder decodes the bitstream, the power saving of the local decoder matches the power saving implied by then DOR-Req message. In a second mode (dec_pow_reduction_type equal to 1), this message requests the remote encoder to disable coding tools so that, when the local decoder decodes the bitstream, the power consumption of the local decoder is decreased. In a third mode (dec_pow_reduction_type equal to 2), this message requests the remote encoder to adjust the picture resolution and video frame rate so that, when the local decoder decodes the bitstream, the power consumption of the local decoder is decreased.  In a fourth mode (dec_pow_reduction_type equal to 3), this message indicates that some other extended decoding operation reduction types are requested by the remote encoder.

In a first extension of the decoding operation reduction types, the message requests a global or partial cancellation of the last decoding operation reduction requests of specific types at the transmitter side. In this case, the encoder stops the corresponding changes previously enabled in its coding process. In case a global cancellation is requested, the transmitter goes back to the nominal mode, where no change in local decoding operations compared to the start of the video session is applied.

In response to a decoding operation reduction request (DOR-Req), the remote encoder sends a decoding operation reduction response (DOR-Resp) message to the remote decoder. In a first mode (dec_pow_reduction_type_resp equal to 0), it acknowledges that it has proceeded to an adjustment of its encoding parameters to save power while decoding. In a second mode (dec_pow_reduction_type_resp equal to 1), this message acknowledges that the encoder has disabled some coding tools. The list of coding tools it has accepted to disable is then sent in the message. In a third mode (dec_pow_reduction_type_resp equal to 2), it acknowledges that it has adjusted the picture resolution and video frame rate. In a fourth mode (dec_pow_reduction_type_resp equal to 3), this message indicates that some other extended decoding operation reduction types were acknowledged.

In a first extension of the decoding operation reduction types, the remote encoder acknowledges the list of types of decoding operation reduction requests it has accepted to cancel.

### Semantics

#### Decoding operation reduction request semantics

dec_pow_reduction_type indicates the type of the decoder power reduction method which is requested by the receiver. The type is indicated by an unsigned integer. The types are explained in Table 16.

Table 16 — definition of dec_pow_reduction_type

| dec_pow_reduction_type | Definition |
| --- | --- |
| 0 | Decoder operations reduction |
| 1 | Coding tool configuration |
| 2 | Spatial and temporal scaling |
| 3 | Extension of the request types |

dec_ops_reduction_req indicates the requested variation of local decoding operations relative to the local decoding operations since the last dec_ops_reduction_req was sent to the transmitter, or since the start of the video session, if no earlier dec_ops_reduction_req was sent. dec_ops_reduction_req is an integer in the interval [-31, 32]. When not present, dec_ops_reduction_req is set equal to 0.

PDecOpsReductionReq is derived by dec_ops_reduction_req and indicates the requested percentage change of local decoding operations by

PDecOpsReductionReq=2*dreq   	(6-109)

where dreq is set equal to dec_ops_reduction_req.

A negative percentage means a decrease of decoding operations. PDecOpsReductionReq is an integer in the interval [-62, 64] in steps of two.

disable_loop filters equal to 1 indicates that loop filters are requested to be disabled, disable_loop_filters equal to 0 specifies that loop filters are requested to be enabled. Loop filters include, upon availability, the deblocking filter, sample Adaptive offset, and the adaptive loop filter.

disable_bi_prediction equal to 1 indicates bi-prediction is requested to be disabled in B slices. disable_bi_prediction equal to 0 indicates bi-prediction is requested to be enabled in B slices.

disable_intra_in_B equal to 1 indicates intra prediction is requested to be disabled in B slices. disable_intra_in_B equal to 0 indicates intra prediction is requested to be enabled in B slices.

disable_fracpel_filtering equal to 1 indicates fractional pel filtering is requested to be disabled in P slices or B slices. disable_fracpel_filtering equal to 0 indicates fractional pel filtering is requested to be enabled in P slices or B slices.

user_defined_req indicates a request To enable or disable user-defined coding tools.

pic_width_in_luma_samples indicates the requested picture width in the units of luma samples.

pic_height_in_luma_samples indicates the requested picture height in the units of luma samples.

frames_per_second indicates the requested frame rate.

dec_pow_reduction_extension_type indicates other types of requests for decoding operation reduction from the remote decoder.

When dec_pow_reduction_extension_type equals 0, it indicates that a global or partial cancellation of the last decoding operation reduction requests of specific types at the transmitter side is requested. In this case, the encoder is requested to stop the corresponding changes previously enabled in its coding process. In case a global cancellation is requested, the transmitter is requested to go back to the nominal mode, where no change in local decoding operations compared to the start of the video session is applied.

nb_dec_pow_reduction_type_req indicates the number of requested types of request the decoder requests to cancel. nb_dec_pow_reduction_type_req shall be in the range 0 to 3.

dec_pow_reduction_type_req_id[ i ] indicates that the decoder requests to cancel the last request of corresponding type. dec_pow_reduction_type_req_id[ i ] shall not be equal to dec_pow_reduction_type_req_id[ j ], for any j in 0..nb_dec_pow_reduction_type_req not equal to i.

#### Decoding operation reduction response semantics

dec_pow_reduction_type_resp indicates the type of the decoder power reduction method the encoder acknowledges to have received and accepts to apply. The type is indicated by an unsigned integer. The types are explained in Table 17.

Table 17 — definition of dec_pow_reduction_type_resp

| dec_pow_reduction_type_resp | Definition |
| --- | --- |
| 0 | Decoder operations reduction |
| 1 | Coding tool configuration |
| 2 | Spatial and temporal scaling |
| 3 | Extension of the response types |

dec_ops_reduction_resp indicates the variation of local decoding operations relative to the local decoding operations since the last dec_ops_reduction_req was sent to the transmitter, or since the start of the video session, if no earlier dec_ops_reduction_req was sent, the encoder accepts to set in place. dec_ops_reduction_resp is an integer in the interval [-31, 32]. When not present, dec_ops_reduction_resp is set equal to 0.

PDecOpsReductionReq is derived by dec_ops_reduction_resp and indicates the requested percentage change of local decoding operations by

PDecOpsReductionReq=2∗dresp

(6108)

where dresp is set equal to dec_ops_reduction_resp.

A negative percentage means a decrease of decoding operations. PDecOpsReductionReq is an integer in the interval [-62, 64] in steps of two.

disabled_loop filters_resp equal to 1 indicates that loop filters were disabled, disabled_loop_filters_resp equal to 0 specifies that the encoder does not accept to disable loop filters. Loop filters include, upon availability, the deblocking filter, sample Adaptive offset, and the adaptive loop filter.

disabled_bi_prediction_resp equal to 1 indicates bi-prediction is disabled in B slices. disabled_bi_prediction_resp equal to 0 indicates that the encoder does not accept to disable bi-prediction in B slices.

disabled_intra_in_B_resp equal to 1 indicates intra prediction is disabled in B slices. disable_intra_in_B_resp equal to 0 indicates that the encoder does not accept to disable intra prediction in B slices.

disabled_fracpel_filtering_resp equal to 1 indicates fractional pel filtering is disabled in P slices or B slices. disabled_fracpel_filtering_resp equal to 0 indicates that the encoder does not accept to disable fractional pel filtering in P slices or B slices.

user_defined_resp indicates whether or not the encoder accepts to enable or disable user-defined coding tools.

pic_width_in_luma_samples_resp indicates the picture width in the units of luma samples that the encoder accepts to encode.

pic_height_in_luma_samples_resp indicates the picture height in the units of luma samples samples that the encoder accepts to encode.

frames_per_second_resp indicates the frame rate samples that the encoder accepts to produce.

dec_pow_reduction_extension_type_resp indicates other types of requests for decoding operation reduction acknowledged by the remote encoder.

When dec_pow_reduction_extension_type_resp equals 0, it indicates that a global or partial cancellation of the last decoding operation reduction requests of specific types was accepted at the transmitter side. In this case, the encoder stops the corresponding changes previously enabled in its coding process. In case a global cancellation was accepted, the transmitter goes back to the nominal mode, where no change in local decoding operations compared to the start of the video session is applied.

nb_dec_pow_reduction_type_resp indicates the number of requested types the encoder accepts to cancel. When nb_dec_pow_reduction_type_resp equal to 0, it indicates that a global cancellation of all previous decoding operation reductions was accepted by the encoder. nb_dec_pow_reduction_type_resp shall be in the range 0 to 3.

dec_pow_reduction_type_resp_id[ i ] indicates that the encoder accepts to cancel the last request of corresponding type. dec_pow_reduction_type_resp_id[ i ] shall not be equal to dec_pow_reduction_type_resp_id[ j ], for any j in 0..nb_dec_pow_reduction_type_resp not equal to i.

# Display power reduction using display adaptation

## General

With respect to the functional architecture, display adaptation (DA) provides green metadata comprised of Attenuation Map Information (AMI), RGB-component statistics and quality indicators. The attenuation Map Information (AMI) metadata indicate how to use attenuation maps carried as auxiliary pictures of type AUX_ALPHA for display adaptation. The attenuation maps and their related Attenuation Map Information as well as RGB-component statistics are used to set display controls in the presentation subsystem so that desired quality levels and corresponding display power reductions are attained.

In a point-to-point video conferencing application, the display adaptation can be performed with or without an SEI message.

## Syntax

### Syntax for systems without a signalling mechanism from the receiver to the transmitter

#### Syntax to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and using SEI messages

The syntax for the AMI metadata is described in Table 18. This syntax is the same for AVC, HEVC and VVC.

Table 18— syntax for the AMI metadata

|  | Descriptor |
| --- | --- |
| ami_flags | u(8) |
| if( ( ami_flags & 0x01 ) = = 0 ) { |  |
| ami_display_model | u(4) |
| if( ami_flags & 0x04 > 0 ) |  |
| ami_map_approximation_model | u(4) |
| ami_map_number | u(3) |
| for( i = 0; i < ami_map_number; i++ ) { |  |
| ami_layer_id[ i ] | u(8) |
| ami_ols_number[ i ] | u(4) |
| for( j = 0; j < ami_ols_number[ i ]; j++ ) |  |
| ami_ols_id[ i ][ j ] | u(8) |
| ami_energy_reduction_rate[ i ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ i ] | u(3) |
| ami_video_quality_level[ i ] | u(16) |
| } |  |
| ami_max_value[ i ] | u(8) |
| if( ( ( ami_flags & 0x02 ) = = 0 ) ¦¦ ( i = = 0 ) ) { |  |
| ami_attenuation_use_idc[ i ] | u(4) |
| ami_attenuation_comp_idc[ i ] | u(4) |
| if( ami_flags & 0x08 > 0 ) { |  |
| ami_preprocessing_flag[ i ] | u(1) |
| if( ami_preprocessing_flag[ i ] ) |  |
| ami_preprocessing_type_idc[ i ] | u(2) |
| ami_preprocessing_scale_idc[ i ] | u(3) |
| } |  |
| if( ami_flags & 0x10 > 0 ) |  |
| ami_backlight_scaling_idc[ i ] | u(4) |
| } |  |
| } |  |
| if( ami_map_number = = 0 ) { |  |
| ami_energy_reduction_rate[ 0 ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ 0 ] | u(3) |
| ami_video_quality_level[ 0 ] | u(16) |
| } |  |
| } |  |
| } |  |

#### Syntax to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and not using SEI messages

The message format used to send metadata from the transmitter to the receiver is described in Table 19.

Table 19 — syntax for display power reduction

|  | Descriptor |
| --- | --- |
| num_constant_backlight_voltage_time_intervals | u(2) |
| num_max_variations | u(2) |
| num_quality_levels | u(4) |
| for( j = 0; j < num_max_variations; j++ ) |  |
| max_variation[ j ] | u(8) |
| for( k = 0; k < num_constant_backlight_voltage_time_intervals; k++ ) { |  |
| constant_backlight_voltage_time_interval[ k ] | u(16) |
| for( j = 0; j < num_max_variations; j++ ) { |  |
| lower_bound[ k ][ j ] | u(8) |
| if( lower_bound[ k ][ j ] > 0 ) |  |
| upper_bound[ k ][ j ] | u(8) |
| rgb_component_for_infinite_psnr[ k ][ j ] | u(8) |
| for( i = 1; i <= num_quality_levels; i++) { |  |
| max_rgb_component[ k ][ j ][ i ] | u(8) |
| scaled_psnr_rgb[ k ][ j ][ i ] | u(8) |
| } |  |
| } |  |
| } |  |

### Syntax for systems with a signalling mechanism from the receiver to the transmitter

#### Syntax to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and using SEI messages

##### First mode of use: application of attenuation maps at the receiver

The receiver first uses the message format to signal information related to display adaptation through the use of attenuation maps described in Table 20:

Table 20 — syntax from receiver for display adaptation through the use of attenuation maps (DAMPR-Req)

|  | Descriptor |
| --- | --- |
| ami_cancel_flag_req | u(1) |
| if( !ami_cancel_flag_req ) { |  |
| ami_display_model_cap | u(4) |
| ami_attenuation_use_cap | u(8) |
| ami_attenuation_comp_cap | u(8) |
| ami_preprocessing_type_cap | u(8) |
| ami_map_approximation_model_cap | u(8) |
| } |  |
| ami_map_number_req | u(3) |
| for( i = 1; i <= ami_map_number_req; i++ ) { |  |
| ami_energy_reduction_rate_req [ i ] | u(5) |
| ami_video_quality_metric_type_req [ i ] | u(3) |
| ami_video_quality_level_req [ i ] | u(16) |
| } |  |

The transmitter then uses the message format to signal metadata to the receiver described in Table 21:

Table 21 — syntax from transmitter to receiver for display adaption through the use of attenuation maps (DAMPR-Resp)

|  | Descriptor |
| --- | --- |
| ami_map_number_resp | u(3) |
| for( i = 1; i <= ami_map_number_resp; i++ ) { |  |
| ami_energy_reduction_rate_resp[ i ] | u(5) |
| ami_video_quality_metric_type_resp [ i ] | u(3) |
| ami_video_quality_level_resp [ i ] | u(16) |
| } |  |

##### Second mode of use: transmission of an attenuated video

The receiver first uses the message format to signal information related to display adaptation through the use of attenuation maps described in Table 22.

Table 22 — syntax for the display power reduction attenuated video request (DPRAV-Req) message from the receiver

|  | Descriptor |
| --- | --- |
| ami_cancel_flag | u(1) |
| ami_display_model_cap | u(4) |
| ami_attenuation_comp_cap | u(8) |
| ami_energy_reduction_rate_req | u(5) |
| ami_video_quality_metric_type_req | u(3) |
| ami_video_quality_level_req | u(16) |

The transmitter then uses the message format to signal metadata to the receiver described in Table 23.

Table 23 — syntax for the display power reduction attenuated video reduction response (DPRAV-Resp) message from the transmitter.

|  | Descriptor |
| --- | --- |
| ami_energy_reduction_rate_resp | u(5) |
| ami_video_quality_metric_type_resp | u(3) |
| ami_video_quality_level_resp | u(16) |

#### Syntax to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and not using SEI messages

The receiver first uses the message format to signal information to the transmitter described in Table 24.

Table 24 — syntax from receiver for display power reduction

|  | Descriptor |
| --- | --- |
| constant_backlight_voltage_time_interval | u(16) |
| max_variation | u(8) |

The transmitter then uses the message format to signal metadata to the receiver described in Table 25.

Table 25 — syntax to receiver for display power reduction

|  | Descriptor |
| --- | --- |
| num_quality_levels | u(4) |
| lower_bound | u(8) |
| if (lower_bound > 0) |  |
| upper_bound | u(8) |
| rgb_component_for_infinite_psnr | u(8) |
| for(i = 1; i <= num_quality_levels; i++ ) { |  |
| max_rgb_component[ i ] | u(8) |
| scaled_psnr_rgb[ i ] | u(8) |
| } |  |

## Signalling

### Signalling for systems without a signalling mechanism from the receiver to the transmitter

#### Signalling to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and using SEI messages

SEI messages can be used to signal green metadata in an AVC, HEVC or VVC stream.

The green metadata SEI message payload type is specified in ISO/IEC 14496-10, ISO/IEC 23008-2, and ISO/IEC 23090-3.

Attenuation Map Information (AMI) metadata describing how to use attenuation maps are carried through SEI message from the transmitter to the receiver. The Attenuation Maps are carried as auxiliary pictures of type AUX_ALPHA for display adaptation, with the flag alpha_channel_use_idc equal to 3.

The complete syntax of the green metadata SEI message payload, including the Attenuation Map Information, is specified in Annex A.

The SEI message containing the AMI metadata is transmitted at the start of an upcoming period. The next message containing AMI metadata is transmitted at the start of the next upcoming period. Therefore, when the upcoming period is a picture or the interval up to the next I-slice, a message is transmitted for each picture or interval, respectively. However, when the upcoming period is a specified time interval or a specified number of pictures, the associated message is transmitted with the first picture in the time interval or with the first picture in the specified number of pictures.

#### Signalling to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and not using SEI messages

Green metadata can be carried as specified in ISO/IEC 13818-1[1] or it can be carried in metadata tracks within the ISO base media file format (ISO/IEC 14496-12 [2]), as specified in ISO/IEC 23001-19. Using the format in 7.2.1.2, the transmitter sends a message to the receiver. The DA metadata is applicable to the presentation subsystem until the next message containing DA metadata arrives.

### Signalling for systems with a signalling mechanism from the receiver to the transmitter

#### Signalling to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and using SEI messages

##### First mode of use: application of attenuation maps at the receiver

The receiver in each device sends a display attenuation map power reduction request (DAMPR-Req) message to the attention of the transmitter. This message allows the receiver to request some attenuation maps with some specific energy reduction rates to the transmitter, and to specify the transmitter with specific information on its capacities to apply attenuation maps, e.g., its display type, supported processings to apply on the requested attenuation maps and supported types of attenuation maps.

In response to the DAMPR-Req from the receiver, the transmitter sends a display attenuation map power reduction response (DAMPR-Resp) message. In a first mode, this message indicates the number of attenuation maps it accepts to produce, and the information of energy reduction rate expected when applying them on the original video. In a second mode, when this message is used to acknowledge a DAMPR-Req with a cancel flag:

- it is empty or the parameter ami_map_resp_number is set to 0 for a global cancellation of the attenuation maps

- it contains the list of the remaining attenuation maps with their related energy reduction rate.

##### Second mode of use: transmission of an attenuated video

The receiver sends a display power reduction attenuated video request (DPRAV-Req) message to request the transmitter to apply a given display attenuation map corresponding to a given display energy reduction rate and a given quality level, on the original video before encoding and transmitting it in order to reduce its energy consumption while rendering the video on its display.

The transmitter sends a display power reduction attenuated video response (DPRAV-Resp) message to indicate the expected display energy reduction rate and the corresponding level of the video quality metric selected by the receiver when the attenuated video is rendered on the receiver display.

When this message is used to acknowledge a request with a cancel flag, it is empty.

#### Signalling to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and not using SEI messages

Using the first message format described in 7.2.2.2, the receiver first signals constant_backlight_voltage_time_interval and max_variation to the transmitter. The transmitter then uses the second message format in 7.2.2.2 to send a message to the receiver. The DA metadata is applicable to the presentation subsystem until the next message containing DA metadata arrives.

## Semantics

### Semantics for systems without a signalling mechanism from the receiver to the transmitter

#### Semantics to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and using SEI messages

The semantics of various terms are defined below.

Note: In a preferred mode, the attenuation map shall be applied on the sample values of the decoded primary picture(s), i.e., just after the decoding process.

The Attenuation Map Information (AMI) metadata provide information about the interpretation of the attenuation map sample values coded in auxiliary pictures of type AUX_ALPHA (in the following picAMI) and the post-processing intended to be applied to the one or more associated primary pictures of the CVS.

Note: The association of auxiliary pictures to primary pictures are specified in the SDI SEI message (see ISO/IEC 23002-7).

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with a green metadata SEI message.

When an access unit (AU) contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and a green metadata SEI message, the SDI SEI message shall precede the green metadata SEI message in decoding order.

When an AU contains a picAMI in a layer, with nuh_layer_id equal to nuhLayerIdAMI, that is indicated as an Alpha Map auxiliary layer by an SDI SEI message, the attenuation map sample values of picAMI persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdAMI is output.

–	A CLVS containing the auxiliary picture picAMI ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdAMI ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the green metadata SEI message applies.

ami_flags is a bit field mask which groups several flags, that allow to reduce the size of the AMI metadata when some parameters are not needed to apply the attenuation maps. These flags are described in Table 26.

bit 0: when not present, indicates that the SEI message cancels the persistence of any previous Attenuation Map Information SEI message in output order. When present, indicates that Attenuation Map Information follows. Corresponds to an ami_cancel_flag.

bit 1: indicates whether the following Attenuation Map Information is defined globally for all the listed maps. When not present, indicates that ami_attenuation_use_idc[ i ], ami_attenuation_comp_idc[ i ], ami_preprocessing_flag[ i ], ami_preprocessing_type_idc[ i ], ami_preprocessing_scale_idc[ i ], ami_backlight_scaling_idc[ i ] for i=0.. ami_map_number, shall be present. When present, indicates that only ami_attenuation_use_idc[ 0 ], ami_attenuation_comp_idc[ 0 ], ami_preprocessing_flag[ 0 ], ami_preprocessing_type_idc[ 0 ], ami_preprocessing_scale_idc[ 0 ], ami_backlight_scaling_idc[ 0 ] shall be present. Corresponds to an ami_global_flag.

bit 2: indicates whether the listed attenuation maps can be used to approximate other attenuation maps for other reduction rates. Corresponds to an ami_approximate_flag.

bit 3: indicates whether some preprocessing is required to use the attenuation maps. Corresponds to an ami_preprocessing_global_flag.

bit 4: indicates whether backlight scaling is required and, in this case, the field ami_backlight_scaling_idc specifies the process to compute the scaling factor of the backlight of transmissive pixel displays, derived from the attenuation map sample values of the decoded auxiliary picture of index i. Corresponds to an ami_backlightscaling_flag.

bit 5: indicates whether video quality information is present in the message. Corresponds to an ami_video_quality_flag.

bits 6-7: bit reserved for future use.

Table 26 — description of ami_flags

| ami_flags | Interpretation of attenuation map-related flags |
| --- | --- |
| 0 | ami_cancel_flag |
| 1 | ami_global_flag |
| 2 | ami_approximate_flag |
| 3 | ami_preprocessing_global_flag |
| 4 | ami_backlightscaling_flag |
| 5 | ami_video_quality_flag |
| 6..7 | Reserved for future use |

ami_map_number specifies the number of auxiliary pictures of type AUX_ALPHA in the CVS. This field is set to 0 when the SEI is distributed with an attenuated video within the CLVS.

ami_display_model is a bit field mask which indicates the display models on which the attenuation map sample values of the auxiliary picture picAMI may be used. Values of the bit field mask are described in Table 27.

Table 27 — interpretation of the bits of ami_display_model

| ami_display_model | Display model |
| --- | --- |
| 0 | Backlit pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

For example, ami_display_model=11 means the Attenuation Map Information can be used for both “backlit” and “emissive” display models.

ami_map_approximation_model specifies the model used to extrapolate a set of received attenuation map sample values(s) from a set of decoded auxiliary picture(s) with individual energy reduction rate(s) to another set of attenuation map sample values with a different energy reduction rate. Values of the ami_map_approximation_model are described in Table 28.

ami_map_approximation_model equal to 0 specifies that a linear scaling of the attenuation map sample values of the provided auxiliary picture given its respective ami_energy_reduction_rate should be considered to obtain corresponding attenuation map sample values for another energy reduction rate. In case several auxiliary pictures picAMI are provided, the auxiliary picture with the lowest ami_energy_reduction_rate is used for the linear scaling. This is the preferred type.

ami_map_approximation_model equal to 1 specifies that a bilinear interpolation between the attenuation map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding attenuation map sample values for another energy reduction rate.

ami_map_approximation_model equal to 2 specifies that an interpolation of type Lanczos between the attenuation map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding attenuation map sample values for another energy reduction rate.

ami_map_approximation_model equal to 3 specifies that an interpolation of type bicubic between the attenuation map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding attenuation map sample values for another energy reduction rate.

ami_map_approximation_model equal to 4 specifies that a proprietary user defined process should be used to infer corresponding attenuation map sample values for another energy reduction rate from the attenuation map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate.

Table 28 — interpretation of ami_map_approximation_model

| ami_map_approximation_model | Attenuation map interpolation process |
| --- | --- |
| 0 | Linear scaling |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | Bicubic interpolation |
| 4 | User defined |
| 5..15 | Reserved for future uses |

ami_attenuation_use_idc[ i ] specifies the use of the attenuation map sample values of the decoded auxiliary picture of index i. Values of ami_attenuation_use_idc[ i ] are described in Table 29.

ami_attenuation_use_idc[ i ] equal to 0 specifies that the attenuation map sample values of the decoded auxiliary picture should be subtracted from one or more associated primary picture decoded sample(s) before displayed on screen. This is the preferred type.

ami_attenuation_use_idc[ i ] equal to 1 specifies that the attenuation map sample values of the decoded auxiliary picture should be multiplied by one or more associated primary picture decoded sample(s) before displayed on screen.

ami_attenuation_use_idc[ i ] equal to 2 specifies that the attenuation map sample values of the decoded auxiliary picture should be used according to a proprietary user defined process to modify the one or more associated primary picture decoded sample(s) before displayed on screen.

Table 29 — interpretation of ami_attenuation_use_idc[ i ]

| ami_attenuation_use_idc[ i ] | Process to apply on associated primary picture decoded samples |
| --- | --- |
| 0 | Subtraction |
| 1 | Multiplication |
| 2 | User defined |
| 3..15 | Reserved for future uses |

ami_attenuation_comp_idc[ i ] specifies on which colour component(s) of the associated primary picture(s) decoded samples the decoded auxiliary picture of type AUX_ALPHA of index i should be applied using the process defined by ami_attenuation_use_idc[ i ]. Values of ami_attenuation_comp_idc[ i ] are described in Table 30.

ami_attenuation_comp_idc[ i ] equal to 0 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the luma component of the associated primary picture(s) decoded samples. This is the preferred type.

ami_attenuation_comp_idc[ i ] equal to 1 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the luma component and the chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 2 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the RGB components (after YUV to RGB conversion) of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 3 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the first component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 4 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the second component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 5 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the third component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 6 specifies that the mapping between the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i and the components of which to apply the decoded auxiliary picture of type AUX_ALPHA of index i corresponds to some proprietary user-defined process.

Table 30 — interpretation of ami_attenuation_comp_idc[ i ]

| ami_attenuation_comp_idc[ i ] | Mapping between components of the attenuation map and primary picture components on which to apply the attenuation map |
| --- | --- |
| 0 | Luma component in the picAMI applied to the luma component of the associated primary picture |
| 1 | Luma component in the picAMI applied to the luma and chroma components of the associated primary picture |
| 2 | Luma component in the picAMI applied to the three RGB components of the associated primary picture |
| 3 | Luma component in the picAMI applied to the first component of the associated primary picture |
| 4 | Luma component in the picAMI applied to the second component of the associated primary picture |
| 5 | Luma component in the picAMI applied to the third component of the associated primary picture |
| 6 | User defined |
| 7..15 | Reserved for future uses |

ami_backlight_scaling_idc[ i ] specifies the process to compute the scaling factor of the backlight of transmissive pixel displays, derived from the attenuation map sample values of the decoded auxiliary picture of index i. Values of ami_backlight_scaling_idc[ i ] are described in Table 31.

ami_backlight_scaling_idc[ i ] equal to 0 specifies that the scaling to apply to the backlight of the display is computed as the ratio between the maximal values of the associated primary picture decoded samples after and before applying the attenuation map sample values of the decoded auxiliary picture of index i. The associated primary picture decoded sample(s) on which the attenuation map sample values of the decoded auxiliary picture of index i are applied are further rescaled to their maximal value before the application of the attenuation map sample values of the decoded auxiliary picture of index i. This is the preferred type.

ami_backlight_scaling_idc[ i ] equal to 1 specifies that the scaling to apply to the backlight of the display is determined according to a proprietary user defined process derived from the attenuation map sample values of the decoded auxiliary picture of index i.

Table 31 — interpretation of ami_backlight_scaling_idc[ i ]

| ami_backlight_scaling_idc[ i ] | Backlight scaling processing type |
| --- | --- |
| 0 | Scaling by the ratio between maximum values before and after the use of the attenuation map |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_preprocessing_flag[ i ] specifies whether some pre-upsampling is to be used on the attenuation map sample values of the decoded auxiliary picture of index i. In that case it is supposed that the auxiliary coded picture(s) and the primary coded picture have different sizes.

ami_preprocessing_type_idc[ i ] specifies the recommended type of the interpolation (e.g., bicubic) used to resample the attenuation map sample values of the decoded auxiliary picture of index i at the same resolution as the associated decoded picture. Values of ami_preprocessing_type_idc[ i ] are described in Table 32.

ami_preprocessing_type_idc[ i ] equal to 0 specifies that an interpolation of type bicubic between the attenuation map sample values of the provided auxiliary picture of index i should be considered to obtain the attenuation map sample values to apply to the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_type_idc[ i ] equal to 1 specifies that a bilinear interpolation between the attenuation map sample values of the provided auxiliary picture of index i should be considered to obtain the attenuation map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 2 specifies that an interpolation of type Lanczos between the attenuation map sample values of the provided auxiliary picture of index i should be considered to obtain the attenuation map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 3 specifies that a proprietary user defined process should be used to pre-upsample the attenuation map sample values of the provided auxiliary picture of index i to obtain the attenuation map sample values to apply to the sample values of the decoded picture.

Table 32 — interpretation of ami_preprocessing_type_idc[ i ]

| ami_preprocessing_type_idc[ i ] | Attenuation map preprocessing type |
| --- | --- |
| 0 | Bicubic interpolation |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | User defined |
| 4..15 | Reserved for future uses |

ami_preprocessing_scale_idc[ i ] specifies which scaling should be applied to the attenuation map of index i to obtain the attenuation map sample values before applying them on the sample values of the decoded picture. Values of ami_preprocessing_scale_idc[ i ] are described in Table 33.

ami_preprocessing_scale_idc[ i ] equal 0 specifies that a scaling of 1255 should be applied to the attenuation map of index i to obtain the attenuation map sample values before applying them on the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_scale_idc[ i ] equal 1 specifies that a proprietary user defined scaling should be applied to the attenuation map of index i to obtain the attenuation map sample values before applying them on the sample values of the decoded picture.

Table 33 — interpretation of ami_preprocessing_scale_idc[ i ]

| ami_preprocessing_scale_idc[ i ] | Attenuation map scaling preprocess |
| --- | --- |
| 0 | Scaling of 1255 |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_layer_id[ i ] specifies the identifier of the decoded layer for the attenuation map of index i.

ami_ols_number[ i ] specifies the number of Output Layer Sets to which the attenuation map of index i belongs.

ami_ols_id[ i ][ j ] specifies the identifier of the Output Layer Set of index j for the attenuation map of index i. This identifier shall be used to select the Output Layer Set to output both the primary decoded picture and the Attenuation Map of index i.

ami_energy_reduction_rate[ i ] indicates the expected energy saving rate when the video is displayed after applying the attenuation map sample values of the decoded auxiliary picture of index i on the sample values of the decoded picture.

ami_max_value[ i ] indicates the maximum value of the attenuation map of index i. Such a maximal value can be optionally used to further adjust the dynamic of the encoded attenuation map in the scaling process.

ami_video_quality_metric_type[ i ] indicates the quality metric which was considered by the transmitter to inform the receiver of the reduction of the video quality after applying the encoded attenuation map. List of quality metrics is available in Table 34.

Table 34 — interpretation of ami_video_quality_metric_type[ i ]

| ami_video_quality_metric_type[ i ] | Metric to assess the video quality of the video after applying the attenuation map |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04..0x07 | Reserved for future metrics |

ami_video_quality_level[ i ] is a 16-bit unsigned integer interpreted as a floating-point VMetric value as follows, with m set equal to ami_video_quality_level[ i ]:

VMetric=m100

which indicates the expected video quality when the decoded picture is rendered on the receiver display after application of the attenuation map either at the transmitter (in this case, ami_map_number is equal to 0) or at the receiver.

#### Semantics to transmit DA metadata without a signalling mechanism from the receiver to the transmitter and not using SEI messages

num_constant_backlight_voltage_time_intervals indicates the number of constant backlight/voltage time intervals for which metadata is provided in the bitstream.

num_max_variations indicates the number of maximum variations for which metadata is provided in the bitstream.

num_quality_levels indicates the number of quality levels that are enabled by the metadata, excluding the NQLOP.

max_variation[ j ] indicates the maximal change between backlight values of two successive frames relative to the backlight value of the earlier frame. The backlight value for a frame is the value of VBacklightScalingFactor[ k ][ j ][ i ] for that frame. VBacklightScalingFactor[ k ][ j ][ i ] is derived from max_rgb_component[ k ][ j ][ i ] and the peak signal variable PS , as (max_rgb_component[ k ][ j ][ i ] / PS) for the kth constant_backlight_voltage_time_interval, jth max_variation and ith quality level.

max_variation is in the range [0.001, 0.1] and is normalized to one byte by rounding after multiplying by 2 048. This is the jth maximal backlight change for which metadata is provided in the bitstream, where 0 <= j < num_max_variations.

constant_backlight_voltage_time_interval[ k ] indicates the minimum time interval, in milliseconds, that shall elapse before the backlight can be updated after the last backlight update. This is the kth minimum time interval for which metadata is provided in the bitstream, where 0 <= k < num_constant_backlight_voltage_time_intervals.

lower_bound[ k ][ j ] indicates if lower_bound[ k ][ j ] is greater than zero, then metadata for contrast enhancement is available at the lowest quality level, for the kth constant_backlight_voltage_time_interval and jth max_variation. If lower_bound[ k ][ j ] = 0, then contrast-enhancement metadata is unavailable.

upper_bound[ k ][ j ] indicates for the kth constant_backlight_voltage_time_interval and jth max_variation, if lower_bound[ k ][ j ] is greater than zero, then contrast enhancement is performed as follows: All RGB components of reconstructed frames that are less than or equal to lower_bound[ k ][ j ] are set to zero and all RGB components that are greater than or equal to upper_bound[ k ][ j ] are saturated to PS. The RGB components in the range (lower_bound[ k ][ j ], upper_bound[ k ][ j ]) are mapped linearly onto the range (0, PS).

rgb_component_for_infinite_psnr[ k ][ j ] indicates for the kth constant_backlight_voltage_time_interval and jth max_variation, the largest RGB component in the reconstructed frames. Therefore, the scaled frames FScaledFrames[ k ][ j ][ 0 ] are identical to the reconstructed frames. The rgb_component_for_infinite_psnr[ k ][ j ] defines a no-quality-loss operating point (NQLOP) and consequently FScaledFrames[ k ][ j ][ 0 ] have a PSNR of infinity relative to the reconstructed frames.

max_rgb_component[ k ][ j ][ i ] indicates for the kth constant_backlight_voltage_time_interval, jth max_variation and ith quality level, the maximum RGB component that is retained in the frames, where 1 <= i <= num_quality_levels.

Note that max_rgb_component[ k ][ j ][ 0 ] = rgb_component_for_infinite_psnr[ k ][ j ].

scaled_psnr_rgb[ k ][ j ][ i ] indicates the PSNR of FScaledFrames[ k ][ j ][ i ] relative to the reconstructed frames. FScaledFrames[ k ][ j ][ i ] are for the kth constant_backlight_voltage_time_interval, jth max_variation and ith quality level, the frames obtained from the reconstructed frames by saturating to max_rgb_component[ k ][ j ][ i ] all RGB components that are greater than max_rgb_component[ k ][ j ][ i ], where 0 <= i <= num_quality_levels.

scaled_psnr_rgb[ k ][ j ][ i ] is set equal to the PSNR value vPSNR, defined as follows for 0 < i <= num_quality_levels:

vPSNR=ClipRound10Log10PS 2* w* h* Ncolour * Nframesn=1Nframesc=1Ncolourl=Xs+1PSNc,nl* l-Xs 2                 (7-1)

where

- w is the width of a video frame.

- h is the height of a video frame.

- Ncolour is the number of colour channels. For RGB colourspace, Ncolour = 3.

- Nframes is the number of frames in the reconstructed frames.

- Nc,n ( l ) is the number of RGB components samples that are set to l in the nth frame of colour-channel c in reconstructed frames.

- Xs   is max_rgb_component[ k ][ j ][ i ].

Note that scaled_psnr_rgb[ k ][ j ][ 0 ] is associated with the NQLOP. It is not transmitted but understood to be mathematically infinite.

### Semantics for systems with a signalling mechanism from the receiver to the transmitter

#### Semantics to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and using SEI messages

##### Display attenuation map power reduction request semantics

ami_cancel_flag_req equal to 1 indicates that the receiver requests to cancel the transmission of the attenuation maps of energy reduction rates listed in the message and the related information in the Green Metadata SEI message.

ami_display_model_cap is a bit field mask which indicates the display models of the receiver on which the attenuation map sample values may be used. Values of the bit field mask are described in Table 35.

Table 35 — interpretation of the bits of ami_display_model_cap

| ami_display_model_cap | Display model |
| --- | --- |
| 0 | Transmissive pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

ami_attenuation_use_cap is a bit field mask which specifies the different processes supported by the receiver to apply the attenuation map sample values. Values of the bit field mask are given in Table 36.

bit 0 set to 1 indicates that subtraction is supported.

bit 1 set to 1 indicates that multiplication is supported.

bits 2..7 are reserved for future processes.

Table 36 — interpretation of ami_attenuation_use_cap

| ami_attenuation_use_cap | Process to apply the attenuation map samples on associated primary picture decoded samples |
| --- | --- |
| 0 | Subtraction |
| 1 | Multiplication |
| 2..7 | User defined |

ami_attenuation_comp_cap is a bit field mask which indicates on which pixel component(s) the receiver accepts to apply an attenuation map. Values of ami_attenuation_map_comp_cap are given in Table 37.

bit 0 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the luma component of the primary picture.

bit 1 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the luma component and the chroma components of the primary picture.

bit 2 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the RGB components (after YUV to RGB conversion) of the primary picture.

bit 3 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the first component of the primary picture.

bit 4 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the second component of the primary picture.

bit 5 set to 1 indicates that the receiver supports to apply the luma component of the attenuation map to the third component of the primary picture.

bits 6-7 are reserved for future uses.

Table 37 — interpretation of ami_attenuation_comp_cap

| ami_attenuation_comp_cap | Mapping between the luma component of the attenuation map and primary picture components on which to apply the attenuation map supported by the receiver |
| --- | --- |
| 0 | Luma component of the map applied to the luma component of the primary picture |
| 1 | Luma component of the map applied to the luma and chroma components of the primary picture |
| 2 | Luma component of the map applied to the three RGB components of the primary picture |
| 3 | Luma component of the map applied to the first component of the primary picture |
| 4 | Luma component of the map applied to the second component of the primary picture |
| 5 | Luma component of the map applied to third component of the primary picture |
| 6..7 | Future uses |

ami_preprocessing_type_cap is a bit field mask which indicates which pre-processings of the attenuation map are supported by the receiver. Values of ami_preprocessing_type_cap are given in Table 38.

bit 0 set to 1 indicates that the receiver supports scaling.

bit 1 set to 1 indicates that the receiver supports a pre-upsampling of type bicubic.

bit 2 set to 1 indicates that the receiver supports a pre-upsampling of type bilinear interpolation.

bit 3 set to 1 indicates that the receiver supports a pre-upsampling of type Lanczos interpolation.

bits 4..7 are reserved for future pre-processing types

Table 38 — interpretation of ami_preprocessing_type_cap

| ami_preprocessing_type_cap | Attenuation map preprocessing type |
| --- | --- |
| 0 | Scaling pre-processing |
| 1 | Bicubic pre-upsampling |
| 2 | Bilinear pre-upsampling |
| 3 | Lanczos pre-upsampling |
| 4..7 | Reserved for future pre-processing types |

ami_map_approximation_model_cap is a bit field mask specifying the models supported by the receiver to extrapolate the set of sample values of the requested attenuation maps with individual energy reduction rate(s) to another set of attenuation map sample values with a different energy reduction rate. Values of ami_map_approximation_model_cap are given in Table 39.

bit 0 set to 1 indicates that linear scaling is supported.

bit 1 set to 1 indicates that bilinear interpolation is supported.

bit 2 set to 1 indicates that Lanczos interpolation is supported.

bit 3 set to 1 indicates that bicubic interpolation is supported.

bits 4..7 are reserved for future approximation models

Table 39 — interpretation of ami_map_approximation_model_cap

| ami_map_approximation_model_cap | Attenuation map interpolation process |
| --- | --- |
| 0 | Linear scaling |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | Bicubic interpolation |
| 4..7 | Reserved for future uses |

ami_map_number_req indicates the number of attenuation maps that are requested by the receiver.

ami_energy_reduction_rate_req[ i ] indicates the energy reduction rate the receiver expects when the attenuation map of index i is applied on the primary picture. An example unit for the ami_energy_reduction_rate is a percentage or a value in Watts.

ami_video_quality_metric_type_req[ i ] indicates the quality metric to be considered by the transmitter when it performs the balancing between the metric’s value and the energy reduction rate for the generation of the display attenuation map of index i. List of quality metrics is given in Table 40.

Table 40 - interpretation of ami_video_quality_metric_type_req

| ami_video_quality_metric_type_req | Metric to assess the video quality of the displayed video after application of the attenuation map. |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04..0x07 | Reserved for future metrics |

ami_video_quality_level_req[ i ] is a 16-bit unsigned integer interpreted as a floating-point VMetric value as follows, with m set equal to ami_video_quality_level_req[ i ]: VMetric=m100

which indicates the value of the requested quality metric that the receiver expects when the display attenuation map of index i is applied on the primary picture. If the receiver does not want to specify an expected video quality, the ami_video_quality_level_req[ i ] shall be set to 0 and, at the transmitter side, the ami_video_quality_level_req[ i ] value shall not be interpreted.

Note: the transmitter is expected to find a good balancing between ami_video_quality_level_req[ i ] and ami_energy_reduction_rate_req[ i ] values requested by the receiver. When this balance cannot be reached, it is recommended that it should compute the display attenuation map taking only into account the energy reduction rate.

##### Display attenuation map power reduction response semantics

ami_map_number_resp indicates the number of transmitted attenuation maps. When set to 0, it indicates that the transmitter acknowledges reception of a global cancellation request of all attenuation maps from the receiver.

ami_energy_reduction_rate_resp[ i ] indicates the energy reduction rate the receiver can expect when the attenuation map of index i is applied on the primary picture. An example unit for the ami_energy_reduction_rate is a percentage or a value in Watts.

ami_video_quality_metric_type_resp[ i ] indicates the quality metric which was considered by the transmitter when it performed the balancing between the metric’s value and the energy reduction rate for the generation of the display attenuation map of index i. List of quality metrics is given in Table 41.

Table 41 — interpretation of  ami_video_quality_metric_type

| ami_video_quality_metric_type_resp | Metric to assess the video quality of the displayed video after application of the attenuation map |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04..0x07 | Reserved for future metrics |

ami_video_quality_level_resp[ i ] is a 16-bit unsigned integer interpreted as a floating-point VMetric value as follows, with m set equal to ami_video_quality_level_resp[ i ]:

VMetric=m100

which indicates the value of the quality metric, previously requested, when the decoded picture is rendered on the receiver display after application of the display attenuation map of index i. If m=0, the VMetric shall be considered as not computed and provided by the transmitter.

##### Display power reduction attenuated video request semantics

ami_cancel_flag equal to 1 indicates that the receiver requests to cancel the transmission of the attenuated video content and the related information in the green metadata SEI message.

ami_display_model_cap is a bit field mask which indicates the display models of the receiver on which the attenuated video content will be rendered. Values of the bit field mask are described in Table 42.

Table 42 — interpretation of ami_display_model_cap

| ami_display_model_cap | Display model |
| --- | --- |
| 0 | Transmissive pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

ami_attenuation_comp_cap is a bit field which allows the receiver to indicate both which component(s) of the attenuation map exists, and the component(s) of the original video on which it accepts that the transmitter applies the components of the attenuation map. Values of ami_attenuation_comp_cap are given in Table 43.

bit 0 set to 1 indicates that the receiver accepts that the transmitter builds an attenuation map of one only component and that it applies this component on the luma component of the primary picture.

bit 1 set to 1 indicates that the receiver accepts that the transmitter builds an attenuation map of one only component and that it applies this component to the luma component and the chroma components of the primary picture.

bit 2 set to 1 indicates that the receiver accepts that the transmitter builds an attenuation map of one only component and that it applies this component to the RGB components (after YUV to RGB conversion) of the primary picture.

bit 3 set to 1 indicates that the receiver accepts that the transmitter builds an attenuation map of two components and that it applies the first component to the luma component and the second component to both chroma components of the primary picture.

bit 4 set to 1 indicates that the receiver accepts that the transmitter applies a per component attenuation map respectively to the luma and chroma components of the primary picture.

bit 5 set to 1 indicates that the receiver accepts that the transmitter applies a per component attenuation map to the RGB components (after YUV to RGB conversion) of the primary picture.

bit 6-7 are reserved for future uses.

Table 43 — interpretation of ami_attenuation_comp_cap

| ami_attenuation_comp_cap | Mapping between components of the attenuation map and primary picture components on which to apply the attenuation map by the transmitter |
| --- | --- |
| 0 | One component of the map applied to the luma component of the primary picture |
| 1 | One component of the map applied to the luma and chroma components of the primary picture |
| 2 | One component in the map applied to the three RGB components of the primary picture |
| 3 | Two components of the map applied as: - First component to luma component - Second component to the chroma components of the primary picture |
| 4 | Three components of the map applied to resp. luma and chroma components of the associated primary picture |
| 5 | Three components of the map applied to resp. the RGB components of the associated primary picture |
| 6..7 | Future uses |

ami_energy_reduction_rate_req indicates the energy reduction rate the receiver expects when the attenuation map is applied on the primary picture. The unit of the ami_energy_reduction_rate is a percentage.

ami_video_quality_metric_type_req indicates the quality metric to be considered by the transmitter when it performs the balancing between the metric’s value and the energy reduction rate for the generation of the attenuation map. List of quality metrics is given in Table 44.

Table 44 - interpretation of ami_video_quality_metric_type_req

| ami_video_quality_metric_type_req | Metric to assess the video quality of the displayed video after application of the attenuation map |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04..0x07 | Reserved for future metrics |

ami_video_quality_level_req is a 16-bit unsigned integer interpreted as a floating-point VMetric value as follows, with m set equal to ami_video_quality_level_req:

VMetric=m100

which indicates the value of the requested quality metric that the receiver requests when the display attenuation map is applied on the primary picture. If the receiver does not want to specify an expected video quality, the ami_video_quality_level_req shall be set to 0 and, at the transmitter side, the ami_video_quality_metric_type_req value shall not be interpreted.

Note: the transmitter is expected to find a good balancing between ami_video_quality_level_req and ami_energy_reduction_rate_req values requested by the receiver. When this balance cannot be reached, it is recommended that it should compute the display attenuation map taking only into account the energy reduction rate.

##### Display power reduction attenuated video response semantics

ami_energy_reduction_rate_resp indicates the energy reduction rate in percentage, the receiver can expect when the attenuated video is rendered on its display.

ami_video_quality_metric_type_resp indicates the quality metric which was considered by the transmitter when it performed the balancing between the value of this metric and the energy reduction rate for the generation of the attenuation map. List of quality metrics is given in Table 45.

Table 45 — interpretation of ami_video_quality_metric_type_resp

| ami_video_quality_metric_type_resp | Metric to assess the video quality of the displayed video after application of the attenuation map |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04..0x07 | Reserved for future metrics |

ami_video_quality_level_resp is a 16-bit unsigned integer interpreted as a floating-point VMetric value as follows, with m set equal to ami_video_quality_level_resp:

VMetric=m100

which indicates the value of the quality metric, previously requested, when the decoded attenuated video picture is rendered on the receiver display. If m=0, the VMetric shall be considered as not computed and provided by the transmitter.

#### Semantics to transmit DA metadata with a signalling mechanism from the receiver to the transmitter and not using SEI messages

The semantics is the same as the one described in section 7.4.1.2.

# Energy-efficient media selection

## General

The green metadata specified in this clause can enable a client in an adaptive streaming session, such as DASH, to determine decoder and display power-saving characteristics of available video representations and to select the representation with the optimal quality for a given power-saving.

Two types of green metadata are defined as follows:

—	decoder-power indication metadata gives the potential decoder power saving of each available representation of a video Segment (as defined in ISO/IEC 23009-1:2019, 3.1.39);

—	display-power indication metadata gives the maximum potential display power saving of a video Segment for a specified number of quality levels. This metadata is computed without any constraint on the maximal backlight change between two successive frames and with no practical restriction on the minimum time interval between backlight updates. Therefore, using the semantics of 7.4, the metadata is produced with the assumptions that max_variation is mathematically infinite and that constant_backlight_voltage_time_interval is less than or equal to the interval between two successive frames.

## Syntax

The decoder-power indication metadata is a pair of decoder operations reduction ratios. The syntax is described in Table 46

Table 46  — syntax for decoder-power indication

|  | Descriptor |
| --- | --- |
| dec_ops_reduction_ratio_from_max | u(8) |
| dec_ops_reduction_ratio_from_prev | s(16) |

The display-power indication metadata contains a list of ms_num_quality_levels pairs. The syntax is described in Table 47.

Table 47   — syntax for display-power indication

|  | Descriptor |
| --- | --- |
| ms_num_quality_levels | u(4) |
| ms_rgb_component_for_infinite_psnr | u(8) |
| for( i = 1; i <= ms_num_quality_levels; i++ ) { |  |
| ms_max_rgb_component[ i ] | u(8) |
| ms_scaled_psnr_rgb[ i ] | u(8) |
| } |  |

## Signalling

Green metadata may be carried in metadata tracks within the ISO base media file format (ISO/IEC 14496-12 [2]), or using specific adaptation set within the MPD in the context of DASH delivery.

In both cases, carriage of green metadata is specified in ISO/IEC 23001-19.

## Semantics

### Decoder-power indication metadata semantics

dec_ops_reduction_ratio_from_max(i) indicates the percentage by which decoding operations are reduced in the ith representation compared to the most demanding representation of the current video Segment. dec_ops_reduction_ratio_from_max(i) is set equal to dopsReducRatioFromMax(i) derived as follows:

dopsReducRatioFromMax(i)=FloorNMaxNumDecOps- NDecOps(i)NMaxNumDecOps*100	(8-1)

NmaxNumDecOps is the estimated number of decoding operations required for the most demanding representation of the current video Segment.

NDecOps(i) is the estimated number of decoding operations required for the ith representation of the current video Segment.

dec_ops_reduction_ratio_from_prev(i) indicates the percentage by which decoding operations are reduced in the current video Segment compared to the previous video Segment for the ith representation in a given Period (as defined in ISO/IEC 23009-1:2019, 3.1.34). A negative value means an increase in decoding operations. dec_ops_reduction_ratio_from_prev(i) is set equal to dopsReducRatioFromPrev(i) derived as follows:

dopsReducRatioFromPrev(i)=FloorNPrevDecOps(i) - NDecOps(i)NDecOps(i)*  100	(8-2)

If the current video Segment is the first Segment of a Period, then dec_ops_reduction_ratio_from_prev(i) is set equal to 0.

NPrevDecOps(i) is the estimated number of decoding operations required for the ith representation of the previous video Segment in a given Period. If the current video Segment is the first Segment of a Period, then NPrevDecOps(i) = NDecOps(i).

### Display-power indication metadata semantics

ms_num_quality_levels indicates the number of quality levels that are enabled by the metadata.

ms_rgb_component_for_infinite_psnr indicates the average, over the N reconstructed frames of the video Segment, of the largest RGB component in each of the reconstructed frames.

ms_max_rgb_component[ i ] indicates for the ith quality level (1 <= i <= num_quality_levels), the average, over the N reconstructed frames of the video Segment, of the maximum RGB component that is retained in each of the reconstructed frames.

Note that ms_max_rgb_component[ 0 ] = ms_rgb_component_for_infinite_psnr.

ms_scaled_psnr_rgb[ i ] indicates for the ith quality level (1 <= i <= num_quality_levels), the average, over the N reconstructed frames in the video Segment, of scaled_psnr_rgb[ i ] computed for each frame as defined in 7.4, with Nframes = 1. Note that ms_scaled_psnr_rgb[ 0 ] is associated with the NQLOP. It is not transmitted, but understood to be mathematically infinite.

# Metrics for quality recovery after low-power encoding

## General

An encoder can achieve power reduction by encoding alternating high-quality and low-quality Segments, in a segmented delivery mechanism such as DASH. The power reduction occurs because low-complexity encoding mechanisms are used to produce the low-quality Segments. A metric describing the quality of the associated picture or subpicture is delivered as metadata to the decoder. The metric is utilized, by the decoder, in conjunction with the associated frame or subpicture of the prior high-quality Segment to enhance the quality of the low-quality Segment and, thereby, ameliorate any negative visual impact. Annex B describes in detail how cross-segment decoding may be used to improve the quality of the low-quality Segments.

## Syntax

### AVC and HEVC syntax

For AVC and HEVC bitstreams, the encoder embeds the metadata message in the last picture of each Segment using the syntax of Table 48.

Table 48 — syntax for quality metrics for AVC and HEVC

|  | Descriptor |
| --- | --- |
| xsd_metric_type | u(8) |
| xsd_metric_value | u(16) |

### VVC syntax

For VVC bitstreams, the encoder embeds the metadata message in the associated picture using the syntax of Table 49

Table 49 — syntax for quality metrics for VVC

|  | Descriptor |
| --- | --- |
| xsd_subpic_number_minus1 | u(16) |
| xsd_subpic_id[ i ] | u(16) |
| xsd_metric_number_minus1[ i ] | u(8) |
| xsd_metric_type[ i ][ j ] | u(8) |
| xsd_metric_value[ i ][ j ] | u(16) |

## Signalling

SEI messages can be used to signal green metadata in an AVC, HEVC or VVC bitstream. The green metadata SEI message payload type for AVC is specified in ISO/IEC 14496-10. The green metadata SEI message payload type for HEVC is specified in International Standard ISO/IEC 23008-2. The green metadata SEI message payload type for VVC is specified in International Standard ISO/IEC 23090-3.

The SEI message for green metadata can be used to signal the preceding message as explained in Annex A.

## Semantics

### AVC and HEVC Semantics

xsd_metric_type indicates the type of the objective quality metric as shown in the Table 50. PSNR, as defined in ISO/IEC 23001-10 [3], is the only type currently supported. A definition of PSNR is provided in Annex D.

Table 50 — specification of xsd_metric_type for AVC and HEVC

| Value | Description |
| --- | --- |
| 0x00 | PSNR |
| 0x01–0xFF | User-defined |

xsd_metric_value contains the metric value of the last picture of the Segment. When xsd_metric_type is 0, then the stored 16-bit unsigned integer xsd_metric_value, is interpreted as a floating-point VPSNR value (in dB) as follows, with m set equal to xsd_metric_value:

VPSNR=m100	(9.1)

### VVC Semantics

xsd_subpic_number_minus1 plus 1 indicates the number of subpictures associated with the metrics specified in the SEI message. The value of xsd_subpics_number_minus1[ i ] shall be in the range of 0 to MaxSlicesPerAu – 1, inclusive, where MaxSlicesPerAu is defined in ISO/IEC 23090-3.

xsd_subpic_id[ i ] indicates the subpicture ID of the ith subpicture.

xsd_metric_number_minus1[ i ] plus 1 indicates the number of objective quality metrics associated with the ith subpicture.

xsd_metric_type[ i ][ j ] indicates the type of the jth objective quality metric associated with the ith subpicture as shown in Table 51. PSNR, wPSNR, WS-PSNR and SSIM are the only types currently supported. Definitions of PSNR, wPSNR, WS-PSNR and SSIM are provided in Annex D.

Table 51 — specification of xsd_metric_type for VVC

| Value | Description |
| --- | --- |
| 0x00 | PSNR |
| 0x01 | SSIM |
| 0x02 | wPSNR |
| 0x03 | WS-PSNR |
| 0x04-0xFF | User-defined |

xsd_metric_value[ i ][ j ] contains the value of the jth objective quality metric associated with the ith subpicture. When xsd_subpic_number_minus1 is equal to 0, xsd_metric_value[ 0 ][ j ] contains the value of jth objective quality metric of the associated picture.

When xsd_metric_type[ i ][ j ] is 0, then the stored 16-bit unsigned integer xsd_metric_value[ i ][ j ], is interpreted as a floating-point VPSNR value (in dB) as follows, with m set equal to xsd_metric_value[ i ][ j ]:

VPSNR  =m100	(9.2)

When xsd_metric_type[ i ] is 1, then the stored 16-bit unsigned integer xsd_metric_value[ i ][ j ], is interpreted as a floating-point VSSIM value as follows, with m set equal to xsd_metric_value[ i ][ j ]:

VSSIM=m100	(9.3)

When xsd_metric_type[ i ] is 2, then the stored 16-bit unsigned integer xsd_metric_value[ i ][ j ], is interpreted as a floating-point VwPSNR value (in dB) as follows, with m set equal to xsd_metric_value[ i ][ j ]:

VwPSNR =m100	(9.4)

When xsd_metric_type[ i ] is 3, then the stored 16-bit unsigned integer xsd_metric_value[ i ][ j ], is interpreted as a floating-point VWS-PSNR value (in dB) as follows, with m set equal to xsd_metric_value[ i ][ j ]:

VWS-PSNR=m100	(9.5)

# Conformance and reference software

Conformance and reference software for green metadata shall be used as specified in Annex C.

(normative)

Supplemental Enhancement Information (SEI) syntax

Syntax and semantics of green metadata SEI message carried in AVC NAL units

This clause describes the payload syntax and semantics if payloadType 56 appears in an AVC NAL unit with nal_unit_type set to 6.

Syntax

| green_metadata(payload_size) { | Descriptor |
| --- | --- |
| green_metadata_type | u(8) |
| switch( green_metadata_type ) { |  |
| case 0: |  |
| period_type | u(8) |
| if( ( period_type = = 2 ) ¦¦ ( period_type = = 7 ) ) |  |
| num_seconds | u(16) |
| else if( ( period_type = = 3 ) ¦¦ ( period_type = = 8 ) ) |  |
| num_pictures | u(16) |
| if( period_type = = 8 ) { |  |
| temporal_map | u(8) |
| for( t = 0; t < 8; t++ ) |  |
| if( ( temporal_map >> t ) % 2 = = 1 ) |  |
| num_pictures_in_temporal_layers[ t ] | u(16) |
| } |  |
| if( period_type <= 3) { |  |
| portion_non_zero_8x8_blocks | u(8) |
| portion_intra_predicted_macroblocks | u(8) |
| portion_six_tap_filterings | u(8) |
| portion_alpha_point_deblocking_instances | u(8) |
| } |  |
| else if( period_type = = 4 ) { |  |
| for( i = 0; i <= num_slice_groups_minus1; i++ ) |  |
| num_slices_minus1[ i ] | u(16) |
| for( i = 0; i <= num_slice_groups_minus1; i++ ) |  |
| for( j = 0; j <= num_slices_minus1[ i ]; j++ ) { |  |
| first_mb_in_slice[ i ][ j ] | u(16) |
| portion_non_zero_8x8_blocks[ i ][ j ] | u(8) |
| portion_intra_predicted_macroblocks[ i ][ j ] | u(8) |
| portion_six_tap_filterings[ i ][ j ] | u(8) |
| portion_alpha_point_deblocking_instances[ i ][ j ] | u(8) |
| } |  |
| } |  |
| else if( ( period_type >= 5) && ( period_type <= 8) ) { |  |
| num_layers_minus1 | u(16) |
| for( l = 0; l <= num_layers_minus1; l++ ) { |  |
| picture_parameter_set_id[ l ] | u(8) |
| priority_id[ l ] | u(6) |
| dependency_id[ l ] | u(3) |
| quality_id[ l ] | u(4) |
| temporal_id[ l ] | u(3) |
| portion_non_zero_8x8_blocks[ l ] | u(8) |
| portion_intra_predicted_macroblocks[ l ] | u(8) |
| portion_six_tap_filterings[ l ] | u(8) |
| portion_alpha_point_deblocking_instances[ l ] | u(8) |
| } |  |
| } |  |
| break; |  |
| case 1: |  |
| xsd_metric_type | u(8) |
| xsd_metric_value | u(16) |
| break; |  |
| case 2: |  |
| ami_flags | u(8) |
| if( ( ami_flags & 0x01 ) = = 0 ) { |  |
| ami_display_model | u(4) |
| if( ami_flags & 0x04 > 0 ) |  |
| ami_map_approximation_model | u(4) |
| ami_map_number | u(3) |
| for( i = 0; i < ami_map_number; i++ ) { |  |
| ami_layer_id[ i ] | u(8) |
| ami_ols_number[ i ] | u(4) |
| for( j = 0; j < ami_ols_number[ i ]; j++ ) |  |
| ami_ols_id[ i ][ j ] | u(8) |
| ami_energy_reduction_rate[ i ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ i ] | u(3) |
| ami_video_quality_level[ i ] | u(16) |
| } |  |
| ami_max_value[ i ] | u(8) |
| if( ( ( ami_flags & 0x02 ) = = 0 ) ¦¦ ( i = = 0 ) ) { |  |
| ami_attenuation_use_idc[ i ] | u(4) |
| ami_attenuation_comp_idc[ i ] | u(4) |
| if( ami_flags & 0x08 > 0 ) { |  |
| ami_preprocessing_flag[ i ] | u(1) |
| if( ami_preprocessing_flag[ i ] ) |  |
| ami_preprocessing_type_idc[ i ] | u(2) |
| ami_preprocessing_scale_idc[ i ] | u(3) |
| } |  |
| if( ami_flags & 0x10 > 0 ) |  |
| ami_backlight_scaling_idc[ i ] | u(4) |
| } |  |
| } |  |
| if( ami_map_number = = 0 ) { |  |
| ami_energy_reduction_rate[ 0 ] | u(5) |
| if( ami_flags && 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ 0 ] | u(3) |
| ami_video_quality_level[ 0 ] | u(16) |
| } |  |
| } |  |
| } |  |
| break; |  |
| default: |  |
| } |  |
| } |  |

Semantics

green_metadata_type specifies the type of metadata that is present in the SEI message. If green_metadata_type is 0, then complexity metrics are present. If green_metadata_type is 1, then metadata enabling quality recovery after low-power encoding is present. If green_metadata_type is 2, then metadata enabling the use of attenuation maps for display adaptation is present. Other values of green_metadata_type are reserved for future use by ISO/IEC.

Syntax and semantics of green metadata SEI message carried in HEVC NAL units

This clause describes the payload syntax and semantics if payloadType 56 appears in an HEVC NAL unit with nal_unit_type set to PREFIX_SEI_NUT.

Syntax

| green_metadata( payload_size ) { | Descriptor |
| --- | --- |
| green_metadata_type | u(8) |
| switch( green_metadata_type ) { |  |  |
| case 0: |  |  |
| period_type | u(8) |
| if( period_type = = 2 ) |  |
| num_seconds | u(16) |
| else if( period_type = = 3 ) |  |
| num_pictures | u(16) |
| if( period_type <= 3 ) { |  |
| portion_non_zero_blocks_area | u(8) |
| if( portion_non_zero_blocks_area != 0 ) { |  |
| portion_8x8_blocks_in_non_zero_area | u(8) |
| portion_16x16_blocks_in_non_zero_area | u(8) |
| portion_32x32_blocks_in_non_zero_area | u(8) |
| } |  |
| portion_intra_predicted_blocks_area | u(8) |
| if( portion_intra_predicted_blocks_area = = 255 ) { |  |
| portion_planar_blocks_in_intra_area | u(8) |
| portion_dc_blocks_in_intra_area | u(8) |
| portion_angular_hv_blocks_in_intra_area | u(8) |
| } |  |
| else { |  |
| portion_blocks_a_c_d_n_filterings | u(8) |
| portion_blocks_h_b_filterings | u(8) |
| portion_blocks_f_i_k_q_filterings | u(8) |
| portion_blocks_j_filterings | u(8) |
| portion_blocks_e_g_p_r_filterings | u(8) |
| } |  |
| portion_deblocking_instances | u(8) |
| } |  |
| else if( period_type = = 4 ) { |  |
| max_num_slices_tiles_minus1 | u(16) |
| for( t = 0; t <= max_num_slices_tiles_minus1; t++ ) { |  |
| first_ctb_in_slice_or_tile[ t ] | u(16) |
| portion_non_zero_blocks_area[ t ] | u(8) |
| if( portion_non_zero_blocks_area[ t ] != 0 ) { |  |
| portion_8x8_blocks_in_non_zero_area[ t ] | u(8) |
| portion_16x16_blocks_in_non_zero_area[ t ] | u(8) |
| portion_32x32_blocks_in_non_zero_area[ t ] | u(8) |
| } |  |
| portion_intra_predicted_blocks_area[ t ] | u(8) |
| if( portion_intra_predicted_blocks_area[ t ] = = 255 ) { |  |
| portion_planar_blocks_in_intra_area[ t ] | u(8) |
| portion_dc_blocks_in_intra_area[ t ] | u(8) |
| portion_angular_hv_blocks_in_intra_area[ t ] | u(8) |
| } |  |
| else { |  |
| portion_blocks_a_c_d_n_filterings[ t ] | u(8) |
| portion_blocks_h_b_filterings[ t ] | u(8) |
| portion_blocks_f_i_k_q_filterings[ t ] | u(8) |
| portion_blocks_j_filterings[ t ] | u(8) |
| portion_blocks_e_g_p_r_filterings[ t ] | u(8) |
| } |  |
| portion_deblocking_instances[ t ] | u(8) |
| } |  |
| } |  |
| break; |  |
| case 1: |  |
| xsd_metric_type | u(8) |
| xsd_metric_value | u(16) |
| break; |  |
| case 2: |  |
| ami_flags | u(8) |
| if( ( ami_flags & 0x01 ) = = 0 ) { |  |
| ami_display_model | u(4) |
| if( ami_flags & 0x04 > 0 ) |  |
| ami_map_approximation_model | u(4) |
| ami_map_number | u(3) |
| for( i = 0; i < ami_map_number; i++ ) { |  |
| ami_layer_id[ i ] | u(8) |
| ami_ols_number[ i ] | u(4) |
| for( j = 0; j <ami_ols_number[ i ]; j++ ) |  |
| ami_ols_id[ i ][ j ] | u(8) |
| ami_energy_reduction_rate[ i ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ i ] | u(3) |
| ami_video_quality_level[ i ] | u(16) |
| } |  |
| ami_max_value[ i ] | u(8) |
| if( ( ( ami_flags & 0x02 ) = = 0 ) ¦¦ ( i = = 0 ) ) { |  |
| ami_attenuation_use_idc[ i ] | u(4) |
| ami_attenuation_comp_idc[ i ] | u(4) |
| if( ami_flags & 0x08 > 0 ) { |  |
| ami_preprocessing_flag[ i ] | u(1) |
| if( ami_preprocessing_flag[ i ] ) |  |
| ami_preprocessing_type_idc[ i ] | u(2) |
| ami_preprocessing_scale_idc[ i ] | u(3) |
| } |  |
| if( ami_flags & 0x10 > 0 ) |  |
| ami_backlight_scaling_idc[ i ] | u(4) |
| } |  |
| } |  |
| if( ami_map_number = = 0 ) { |  |
| ami_energy_reduction_rate[ 0 ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ 0 ] | u(3) |
| ami_video_quality_level[ 0 ] | u(16) |
| } |  |
| } |  |
| } |  |
| break; |  |
| default: |  |
| } |  |
| } |  |

Semantics

green_metadata_type specifies the type of metadata that is present in the SEI message. If green_metadata_type is 0, then complexity metrics are present. If green_metadata_type is 1, then metadata enabling quality recovery after low-power encoding is present. If green_metadata_type is 2, then metadata enabling the use of attenuation maps for display adaptation is present. Other values of green_metadata_type are reserved for future use by ISO/IEC.

Syntax and semantics of green metadata SEI message carried in VVC NAL units

This clause describes the payload syntax and semantics if payloadType 56 appears in a VVC NAL unit with nal_unit_type set to PREFIX_SEI_NUT.

Syntax

| green_metadata( payload_size ) { | Descriptor |
| --- | --- |
| green_metadata_type | u(8) |
| switch( green_metadata_type ) { |  |
| case 0: |  |
| period_type | u(4) |
| granularity_type | u(3) |
| extended_representation_flag | u(1) |
| if( period_type = = 2 ) |  |
| num_seconds | u(16) |
| else if( period_type = = 3 ) |  |
| num_pictures | u(16) |
| if( granularity_type = = 0 ) { |  |
| portion_non_zero_blocks_area | u(8) |
| portion_non_zero_transform_coefficients_area | u(8) |
| portion_intra_predicted_blocks_area | u(8) |
| portion_deblocking_instances | u(8) |
| portion_alf_instances | u(8) |
| if( extended_representation_flag ) { |  |
| if( portion_non_zero_blocks_area != 0 ) { |  |
| portion_non_zero_4_8_16_blocks_area | u(8) |
| portion_non_zero_32_64_128_blocks_area | u(8) |
| portion_non_zero_256_512_1024_blocks_area | u(8) |
| portion_non_zero_2048_4096_blocks_area | u(8) |
| } |  |
| if( portion_intra_predicted_blocks_area < 255 ) { |  |
| portion_bi_and_gpm_predicted_blocks_area | u(8) |
| portion_bdof_blocks_area | u(8) |
| } |  |
| portion_sao_instances | u(8) |
| } |  |
| } |  |
| else if( granularity_type <= 3 ) { |  |
| max_num_segments_minus1 | u(16) |
| for( t = 0; t <= max_num_segments_minus1; t++ ) { |  |
| segment_address[ t ] | u(16) |
| portion_non_zero_blocks_area[ t ] | u(8) |
| portion_non_zero_transform_coefficients_area[ t ] | u(8) |
| portion_intra_predicted_blocks_area[ t ] | u(8) |
| portion_deblocking_instances[ t ] | u(8) |
| portion_alf_filtered_blocks[ t ] | u(8) |
| if( extended_representation_flag ) { |  |
| if( portion_non_zero_blocks_area[ t ] != 0 ) { |  |
| portion_non_zero_4_8_16_blocks_area[ t ] | u(8) |
| portion_non_zero_32_64_128_blocks_area[ t ] | u(8) |
| portion_non_zero_256_512_1024_blocks_area[ t ] | u(8) |
| portion_non_zero_2048_4096_blocks_area[ t ] | u(8) |
| } |  |
| if( portion_intra_predicted_blocks_area[ t ] < 255 ) { |  |
| portion_bi_predicted_blocks_area[ t ] | u(8) |
| portion_bdof_block_area[ t ] | u(8) |
| } |  |
| portion_sao_filtered_blocks[ t ] | u(8) |
| } |  |
| } |  |
| } |  |
| break; |  |
| case 1: |  |
| xsd_subpic_number_minus1 | u(16) |
| for( i = 0; i <= xsd_subpic_number_minus1; i++ ) { |  |
| xsd_subpic_idc[ i ]_ | u(16) |
| xsd_metric_number_minus1[ i ] | u(8) |
| for( j = 0; j <= xsd_metric_number_minus1[ i ]; j++ ) { |  |
| xsd_metric_type[ i ][ j ] | u(8) |
| xsd_metric_value[ i ][ j ] | u(16) |
| } |  |
| } |  |
| break; |  |
| case 2: |  |
| ami_flags | u(8) |
| if( ( ami_flags & 0x01 ) = = 0 ) { |  |
| ami_display_model | u(4) |
| if( ami_flags & 0x04 > 0 ) |  |
| ami_map_approximation_model | u(4) |
| ami_map_number | u(3) |
| for( i = 0; i < ami_map_number; i++ ) { |  |
| ami_layer_id[ i ] | u(8) |
| ami_ols_number[ i ] | u(4) |
| for( j = 0; j < ami_ols_number[ i ]; j++ ) |  |
| ami_ols_id[ i ][ j ] | u(8) |
| ami_energy_reduction_rate[ i ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ i ] | u(3) |
| ami_video_quality_level[ i ] | u(16) |
| } |  |
| ami_max_value[ i ] | u(8) |
| if( ( ( ami_flags & 0x02 ) = = 0 ) ¦¦ ( i == 0 ) ) { |  |
| ami_attenuation_use_idc[ i ] | u(4) |
| ami_attenuation_comp_idc[ i ] | u(4) |
| if( ami_flags & 0x08 > 0 ) { |  |
| ami_preprocessing_flag[ i ] | u(1) |
| if( ami_preprocessing_flag[ i ] ) |  |
| ami_preprocessing_type_idc[ i ] | u(2) |
| ami_preprocessing_scale_idc[ i ] | u(3) |
| } |  |
| if( ami_flags & 0x10 > 0 ) |  |
| ami_backlight_scaling_idc[ i ] | u(4) |
| } |  |
| } |  |
| if( ami_map_number == 0 ) { |  |
| ami_energy_reduction_rate[ 0 ] | u(5) |
| if( ami_flags & 0x20 > 0 ) { |  |
| ami_video_quality_metric_type[ 0 ] | u(3) |
| ami_video_quality_level[ 0 ] | u(16) |
| } |  |
| } |  |
| } |  |
| break; |  |
| default: |  |
| } |  |
| } |  |

Semantics

green_metadata_type specifies the type of metadata that is present in the SEI message. If green_metadata_type is 0, then complexity metrics are present. If green_metadata_type is 1, then metadata enabling quality recovery after low-power encoding is present. If green_metadata_type is 2, then metadata enabling the use of attenuation maps of display adaptation is present. Other values of green_metadata_type are reserved for future use by ISO/IEC.

(informative)

Implementation guidelines for the usage of green metadata

Codec dynamic voltage frequency scaling for decoder-power reduction

General

Codec Dynamic Voltage Frequency Scaling (C-DVFS) uses the DVFS technique to scale the voltage and operating frequency of the CPU to achieve power savings while decoding a bitstream. Typically the dynamic power consumption of a CMOS circuit increases monotonically with the operating frequency. The power-optimization module at the receiver extracts the complexity metrics (CMs) metadata that indicates picture-decoding complexity. It uses these CMs to determine and set the optimum operating voltage and frequency of the CPU so that video pictures are correctly decoded with minimal power consumption. By embedding these CMs as metadata into the bitstream at the encoder, C-DVFS enabled receivers achieve power reduction.

Derivation of the complexity metrics

5.2.2 specifies CMs associated to AVC: portion_non_zero_8x8_blocks, portion_intra_predicted_macroblocks, portion_six_tap_filterings and portion_alpha_point_deblocking_instances. The computation of the first two CMs, as explained in 6.2.4, is straightforward. However, computation of portion_six_tap_filterings and portion_alpha_point_deblocking_instances is more involved. To provide a better understanding of these two CMs, the next two subclauses describe how NmaxNumSixTapFiltPic(i) and NmaxAlphaPointDbfsPic(i) are derived.

5.2.2 specifies CMs associated to HEVC: portion_blocks_a_c_d_n_filterings, portion_blocks_h_b_filterings, portion_blocks_f_i_k_q_filterings, portion_blocks_j_filterings and portion_blocks_e_g_p_r_filterings. To provide a better understanding of these five CMs, the different sub-sample position a, b, c, d, e, f, g, h, i, j, k, n, p, q and r, are represented in Figure B.1, where upper-case letters represent integer samples and lower-case letters represent sub-sample positions derived.

5.2.2 specifies CMs associated to VVC: portion_non_zero_blocks_area, portion_non_zero_4_8_16_blocks_area, portion_non_zero_32_64_128_blocks_area,  portion_non_zero_256_512_1024_blocks_area, portion_non_zero_2048_4096_blocks_area, portion_non_zero_transform_coefficients_area, portion_intra_predicted_blocks_area, portion_bi_and_gpm_predicted_blocks_area, portion_bdof_blocks_area, portion_deblocking_instances, portion_sao_filtered_blocks, portion_alf_filtered_blocks.

Deriving the worst-case, largest value for NmaxNumSixTapFiltPic(i)

To determine NmaxNumSixTapFiltPic(i), the following terms, as defined in ISO/IEC 14496-10, are referenced: motion vector, PicSizeInMbs, reference picture list. At the decoder, the worst-case, largest number of 6-tap filterings (STFs) occurs in a picture when all partitions consist of 4x4 blocks that are interpolated. The 4x4 blocks produce the largest number of STFs because the overhead from interpolating samples that are outside the block is larger for 4x4 blocks than for 8x8 blocks as explained below.

In Figure B.1, upper-case letters represent integer samples and lower-case letters represent fractional sample positions. Subscripts are used to indicate the integer sample that is associated with a fractional sample position. The subsequent analysis is for the worst-case largest number of STFs for the interpolation of the 4x4-block consisting of samples G, H, I, J, M, N, P, Q, R, S, V, W, T, U, X, Y. This interpolation shall be performed when a motion vector (MV) points to one of the following fractional-sample positions: aG, bG, cG, dG, eG, fG, gG, hG, iG, jG, kG, nG, pG, qG, rG. If the MV points to aG, then the decoder shall compute aG and the 15 points (aH, aI, ...) that have the same respective relative locations to H, I, J, M, N, P, Q, R, S, V, W, T, U, X, Y that aG has to G. Similarly, the decoder shall compute 16 points for each of the other fractional-sample positions (bG, cG, ..., rG) that the MV can point to. To determine the worst-case largest number of STFs for the interpolation of the 4x4 block, here is a count of the STFs required for each fractional-sample position that the MV can point to.

Figure B.1 — Quarter-sample interpolation of the 4x4-block consisting of samples G, H, I, J, M, N, P, Q, R, S, V, W, T, U, X, Y

- If the MV points to bG, then to interpolate bG, the decoder shall apply 1 STF to E, F, G, H, I, J which are already available as integer samples. So 16 STFs are needed to compute bG, ..., bY for the 4x4 block.

- If the MV points to hG, then to interpolate hG, the decoder shall apply 1 STF to A, C, G, M, R, T which are already available as integer samples. So 16 STFs are needed to compute hG, ..., hY for the 4x4 block.

- If the MV points to jG, then to interpolate jG, the decoder shall apply 6 STFs to compute aa, bb, bG, sM, gg, hh because these are unavailable. Next, 1 STF is needed to compute jG from aa, bb, bG, sM, gg, hh. So 7 STFs are required for jG.

- To get jM, the decoder needs bb, bG, sM, gg, hh, ii. Only ii is unavailable. So 2 STFs are needed for jM (one for ii and one for jM).

- To get jR, the decoder needs 2 STFs (one for jj and one for jR).

- To get jT, the decoder needs 2 STFs (one for kk and one for jT).

- Therefore, for jG, jM, jR and jT, the decoder needs 7 + 2 + 2 + 2 = 13 STFs. Since the computation is identical for each of the four columns GMRT, HNSU, IPVX and JQWY, the decoder needs 13 * 4 = 52 STFs to compute jG, ... jY for the 4x4 block.

- If the MV points to aG, then to interpolate aG, the decoder needs 1 STF to get bG (from (a)) and therefore 16 STFs to compute aG, ..., aY for the 4x4 block.

- If the MV points to cG, then to interpolate cG, the decoder needs 1 STF to get bG (from (a)) and therefore 16 STFs to compute cG, ..., cY for the 4x4 block.

- If the MV points to dG, then to interpolate dG, the decoder needs 1 STF to get hG (from (b)) and therefore 16 STFs to compute dG, ..., dY for the 4x4 block.

- If the MV points to nG, then to interpolate nG, the decoder needs 1 STF to get hG (from (b)) and therefore 16 STFs to compute nG, ..., nY for the 4x4 block.

- If the MV points to fG, then to interpolate fG, the decoder needs 7 STFs to get jG (from (c)). Note that bG is included in these 7 STFs. Therefore, from (c), 52 STFs are required to compute fG, ... fY for the 4x4 block.

- If the MV points to iG, then to interpolate iG, the decoder needs 7 STFs to get jG. Note that hG is computed by one of these 7 STFs. Therefore, 52 STFs are required to compute iG, ... iY for the 4x4 block. For this analysis, the row jG, jH, jI, jJ is computed first (to obtain hG) and then this process is repeated for the other 3 rows (MNPQ, RSVW, TUXY) in the 4x4 block. Previously, in (c), column GMRT was analysed first and the analysis was then repeated for the other 3 columns (HNSU, IPVX, JQWY).

- If the MV points to kG, then to interpolate kG, the decoder needs 7 STFs to get jG. Note that mG is computed by one of these 7 STFs. Therefore, 52 STFs are required to compute kG, ... kY for the 4x4 block.

- If the MV points to qG, then to interpolate qG, the decoder needs 7 STFs to get jG. Note that sG is computed by one of these 7 STFs. Therefore, 52 STFs are required to compute qG, ... qY for the 4x4 block.

- If the MV points to eG, then to interpolate eG, the decoder needs 2 STFs to get bG and hG (from (a), (b)). Therefore 32 STFs are needed to compute eG, ..., eY for the 4x4 block.

- If the MV points to gG, then to interpolate gG, the decoder needs 2 STFs to get bG and mH. Therefore, 32 STFs are needed to compute gG, ..., gY for the 4x4 block.

- If the MV points to pG, then to interpolate pG, the decoder needs 2 STFs to get hG and sG. Therefore, 32 STFs are needed to compute pG, ..., pY for the 4x4 block.

- If the MV points to rG, then to interpolate rG, the decoder needs 2 STFs to get mG and sG. Therefore, 32 STFs are needed to compute rG, ..., rY for the 4x4 block.

From (a),…,(n), the worst-case, largest number of STFs is 52, when the MV points to jG, fG, iG, kG or qG. Since the overhead of filtering samples outside the block is smaller for larger block sizes, the worst case STFs is when all partitions are 4x4 blocks and two MVs are used for each block (one from each reference picture list). In this case, the worst-case, largest number of STFs in a picture is derived based on the following pseudo-code:

NMaxNumSixTapFiltPic(i) = (worst-case number of STFs in a 4x4 block)

* (worst-case number of reference picture lists)

* (PicSizeInMbs) * (number of 4x4 luma blocks in a macroblock)

= 52 * 2 * PicSizeInMbs * 16

= 1664 * PicSizeInMbs	(B-1)

Deriving the worst-case, largest value for NmaxAlphaPointDbfsPic(i)

To determine NmaxAlphaPointDbfsPic(i), the following analysis determines the worst-case, largest number of alpha-point deblocking instances (APDIs) that can occur when deblocking a picture at the decoder. The following terms, as defined in ISO/IEC 14496-10, are referenced: raster scan, PicSizeInMbs.

Consider a macroblock containing a 16x16 luma block in which the samples have been numbered in raster-scan order as shown in Figure B.2. Upper-case roman numerals are used to reference columns of samples and lower-case roman numerals are used to reference rows of samples. For example, column IV refers to the column of samples 4, 20, ... 244 and row xiii refers to the row of samples 193, 194, ..., 208. edges are indicated by an ordered pair that specifies the columns or rows on either side of the edge. For example, edge (IV, V) refers to the vertical edge between columns IV and V. Similarly, edge (xii, xiii) indicates the horizontal edge between rows xii and xiii. Note that the leftmost vertical edge and the topmost horizontal edge are denoted by (0, I) and (0, i) respectively.

The maximum number of APDIs occurs when the 4x4 transform is used on each block and a single APDI occurs in every set of eight samples across a 4x4 block horizontal or vertical edge denoted as pi and qi with i = 0..3 as shown in Figure 8-11 of ISO/IEC 14496-10.

For the macroblock in Figure B.2, the vertical edges (0, I), (IV, V), (VIII, IX) and (XII, XIII) are filtered first. Then the horizontal edges (0,i), (iv, v), (viii, ix) and (xii, xiii) are filtered. Now, when vertical edge (0, I) is filtered, in the worst-case, an APDI occurs on each row of the edge because the q0 samples 1, 17, ... 241 will all be APDIs. Therefore, 16 APDIs occur in vertical edge (0, I). Similarly, when vertical edge (IV, V) is filtered, there are also 16 APDIs corresponding to the 16 (p0, q0) sample pairs (20, 21), (36, 37), ... (244, 245). Thus, there are 16*4 = 64 APDIs from vertical-edge filtering. After horizontal-edge filtering, there are an additional 64 APDIs because each horizontal edge contributes 16 APDIs. For example, horizontal edge (viii, ix) contributes the 16 APDIs corresponding to the (p0, q0) sample pairs (113, 129), (114, 130), ..., (128, 144). Hence, in the worst-case, deblocking the luma block in a macroblock produces 128 APDIs.

Next, consider the two chroma blocks corresponding to the luma block in the macroblock. The worst-case number of APDIs is determined by the chroma sampling relative to the luma sampling.

Figure B.2 — 16x16 luma block. Upper-case roman numerals reference columns of samples and lower-case roman numerals reference rows of samples

- For each chroma block in 4:2:0 format, two vertical edges and two horizontal edges are filtered. Each edge contributes 8 APDIs, in the worst-case. So, 8*4*2 = 64 APDIs are produced by worst-case deblocking of the two chroma blocks.

- For 4:2:2 format, two vertical edges and four horizontal edges are filtered. Each vertical edge contributes 16 APDIs and each horizontal edge contributes 8 APDIs. So, 2*(2*16 + 4*8) = 128 APDIs are produced by worst-case deblocking of the two chroma blocks.

- For 4:4:4 format, the worst-case analysis for each chroma block is identical to that of the 16x16 luma block. Therefore, 256 APDIs are produced by worst-case deblocking of the two chroma blocks.

- Finally, for separate colour planes, the worst-case analysis of a 16x16 block is identical to that of 16x16 luma block.

To conclude, since each picture has PicSizeInMbs macroblocks, the worst-case number of APDIs per picture, is derived based on the following pseudo-code:

NmaxAlphaPointDbfsPic(i)= PicSizeInMbs * (128 + 64) = 192 * PicSizeInMbs, for 4:2:0,

= PicSizeInMbs * (128 + 128) = 256 * PicSizeInMbs, for 4:2:2,

= PicSizeInMbs * (128 + 256) = 384 * PicSizeInMbs, for 4:4:4,

= 128 * PicSizeInMbs, for a single colour plane.	(B-2)

Example usage of C-DVFS metadata

C-DVFS metadata may be signalled at a slice, layer, picture, group of pictures, or scene level and can therefore be adapted to application requirements. Signalling may be done with SEI messages. With SEI-message signalling, each time the SEI message is encountered by the decoder, a new upcoming period begins. The value period_type indicates whether the new upcoming period is a single picture, a single group of pictures, or a time interval (specified in seconds or number of pictures). Figure B.3 shows an example process for metadata extraction, complexity prediction, DVFS control-parameter determination and decoding under DVFS control. As an example, assume that the upcoming period is a single picture. Then, the SEI message is parsed to obtain portion_non_zero_8x8_blocks, portion_intra_predicted_macroblocks, portion_six_tap_filterings and portion_num_alpha_point_deblocking_instances. From these portion values and the corresponding worst-case instances the four CMs are derived: num_non_zero_8x8_blocks (nnz), num_intra_predicted_macroblocks (nintra), num_six_tap_filterings (nsix), and num_alpha_point_deblocking_instances (nα). Once the complexity parameters are derived, the total picture complexity (Cpict) is estimated or predicted according to Formula B-3:

Cpict=kinit*nMB+kbit*nbit+knz*nnz+kintra*nintra+ksix*nsix+k∝*n∝	(B-3)

where Cpict is the total picture complexity. The total number of macroblocks per picture (nMB) and the number of bits per picture (nbit) can be easily obtained after de-packetizing the encapsulated packets and parsing the sequence parameter set. Constants kinit, kbit, knz, kintra, ksix, and kα are unit-complexity constants for performing macroblock initialization (including parsed data filling and prefetching), single-bit parsing, non-zero block transform and quantization, intra-block prediction, inter-block six-tap filtering, and deblocking alpha-points filtering, respectively. Note that knz, kintra, and ksix are fixed constants for a typical platform, while kinit , kbit , and kα can be accurately estimated using a linear predictor from a previous decoded picture.

Once the picture complexity is determined, the decoder applies DVFS to determine a suitable clock frequency and supply voltage for the decoder. Then, the decoder can decode the video picture at the appropriate clock frequency and supply voltage.

The DVFS-enabling SEI message can be inserted into the bitstream on a slice-by-slice, layer-by-layer, picture-by-picture, scene-by-scene, or even time-interval-by-time-interval basis, depending on the underlying application. Therefore, the SEI message can be inserted once at the start of each picture, scene, or time interval. A scene-interval or time-interval inserted message requires less overhead than a picture-level inserted message. For processors that do not support high-frequency DVFS (e.g. adapting at 33 ms for 30Hz video playback), setting period_type to an interval is preferable to setting period_type to a picture. Once all complexity metrics are obtained from the SEI message, the decoder estimates the complexity for the next slice, layer, picture, group of pictures, or time interval as indicated by period_type. This complexity is then used to adjust the voltage and frequency for the upcoming period.

In a hardware (ASIC) implementation, instead of deriving decoding complexity and using it directly to control a single clock frequency in a DVFS scheme, the ASIC can be designed so that it includes several distinct clock domains, each of which corresponds to one of the terms in Formula B-3. Greater power reduction can be obtained by using such a flexible ASIC with distinct clock domains. For example, six clock domains in the ASIC can control the following six sections of the ASIC: macroblock initialization, bit parsing, transform and quantization, intra-block prediction, interpolation, and deblocking. To achieve fine-grained DVFS adjustments, the clock frequencies in each domain may be varied in proportion to the corresponding term in Formula B-3. Accordingly, the preceding clock domains can have instantaneous clock frequencies that are respectively proportional to the following terms: kinit * nMB, kbit * nbit, knz * nnz, kintra * nintra, ksix * nsix, and kα * nα.

Figure B.3 — Example of parsing, complexity prediction, and DVFS control

Display adaptation

General

Display adaptation (DA) achieves power savings by scaling up the RGB components in the reconstructed frames while reducing the backlight or voltage proportionally. The decreased backlight or voltage reduces display power consumption while still producing the same perceived display. The metadata in 7.2.1 may be stored using the file format specified in ISO/IEC 23001-10 or the metadata may be carried by MPEG-2 systems as specified in ISO/IEC 13818-1 [1].

Example usage of display-adaptation metadata

The metadata scaled_psnr_rgb[ i ] indicates the PSNR for the ith quality level. At the transmitter, reconstructed frames are available within the encoder and FScaledFrames[ i ] is estimated by saturating all RGB components of reconstructed frames to max_rgb_component[ i ]. The FScaledFrames[ i ] thus obtained are what would be perceived at the display after the receiver scales the RGB components of reconstructed frames by (PS / max_rgb_component[ i ]), PS being the peak signal and then applies the backlight scaling factor, b = (max_rgb_component[ i ] / PS) to the LCD backlight. scaled_psnr_rgb[ i ] is computed at the transmitter using PS and by assuming that the noise is the difference between FScaledFrames[ i ] and reconstructed frames accumulated over R, G and B components, as explained in 7.4.

The receiver examines the (num_quality_levels + 1) pairs of metadata and selects the pair (max_rgb_component[iSelected], scaled_psnr_rgb[iSelected]) for which scaled_psnr_rgb[iSelected] is an acceptable quality level. Then, the receiver derives DA scaling factors from max_rgb_component[iSelected]. Finally, the display scales the RGB components of reconstructed frames by PS / max_rgb_component[iSelected] and it scales the backlight or voltage level by max_rgb_component[iSelected] / PS. After backlight scaling, the displayed pixels are perceived as FScaledFrames[iSelected]. The metadata clearly enables a trade-off between quality (PSNR) and power reduction (backlight scaling factor).

The following power-saving protocol can be implemented in a mobile device. The user specifies a list of N acceptable PSNR quality levels Q[1], …, Q[ n ], where Q[1] > Q[2] > …> Q[ n ] and a list of Remaining Battery Life Levels (RBLLs) RBLL[1], …, RBLL[ n ] so that RBLL[1] > RBLL[2] > … > RBLL[ n ]. For example, consider N = 3 and Q[1] = 40, Q[2] = 35, Q[3] = 25 with RBLL[1] = 70%, RBLL[2] = 40% and RBLL[3] = 0%. When the user watches a video, the device monitors the actual RBLL, denoted RBLLactual, of the device and selects RBLL[iSelected ] so that RBLL[iSelected – 1] > RBLLactual > RBLL[iSelected], where RBLL[ 0 ] = 100%. For each frame to be displayed, the device examines the display-adaptation metadata and selects the pair indexed by jSelected for which Q[iSelected – 1] > scaled_psnr_rgb[jSelected] > Q[iSelected], where Q[ 0 ] = infinity. The metadata max_rgb_component[jSelected] is then used to determine display-adaptation scaling parameters. Thus, the device implements a protocol that strikes a balance between perceived quality and power-saving. The balance is tilted toward quality when the RBLL is high but shifts toward power saving as the battery is depleted.

Example usage of display-adaptation metadata for contrast enhancement

At low quality levels, contrast enhancement significantly improves perceived visual quality, especially for bright content. To enhance contrast at the lowest quality level associated with the backlight scaling factor b = (max_rgb_component[num_quality_levels] / PS) the receiver first examines lower_bound. If it is greater than zero, then contrast enhancement metadata is available and the receiver stores upper_bound. The presentation subsystem performs contrast enhancement by setting the backlight scaling factor to b = (max_rgb_component[num_quality_levels] / PS), and for each RGB component, x, of reconstructed frames, the scaling to S(x) is performed using the following pseudo-code:

S(x) = 0,                                             for x in [0, lower_bound],

= PS *(x–lower_bound)/(upper_bound–lower_bound)  for x in (lower_bound, upper_bound),

= PS                                             for x in [upper_bound, PS]

Observe that the interval (lower_bound, upper_bound) is mapped to the interval (0, PS). Then, after applying the backlight scaling factor, b, to the display, the interval (lower_bound, upper_bound) is perceived visually as the interval (0, b * PS). Therefore, for RGB components within the interval (lower_bound, upper_bound), the perceived contrast enhancement is proportional to ( b * PS / (upper_bound – lower_bound) ). This expression simplifies to b / (upper_bound – lower_bound), because PS is a constant. For RGB components within the intervals [0, lower_bound] and [upper_bound, PS], all contrast is lost because these intervals are mapped to 0 and PS, respectively.

From the preceding observation, it is clear that the contrast is maximized by determining lower_bound and upper_bound so that the majority of RGB components lie within the interval (lower_bound, upper_bound). Therefore, the optimal contrast-enhancement metadata is computed by the following process, at the transmitter. First, determine the VBacklightScalingFactor corresponding to the lowest quality level as b = max_rgb_component[num_quality_levels] / PS. Then, invoke the following pseudocode function get_contrast_metadata() to determine lower_bound and upper_bound.

// Given RGB components, x, of reconstructed frames with cumulative distribution function,

// C(x), the function get_contrast_metadata() returns lower_bound and upper_bound.

[lower_bound, upper_bound] = get_contrast_metadata(C(x)) {

// C(x): Cumulative distribution function of RGB components of reconstructed frames.

max_enhancement = 0;
  for (lower_bound = 0; lower_bound < PS; lower_bound++){

for (upper_bound = lower_bound; upper_bound < PS; upper_bound++){

enhancement = (C(upper_bound) – C(lower_bound)) / (upper_bound – lower_bound)

if (enhancement > max_enhancement) {

max_enhancement = enhancement;

best_lower_bound = lower_bound;

best_upper_bound = upper_bound;

}

}

}

return (best_lower_bound, best_upper_bound);

}

Although the metadata computed by get_contrast_metadata() is optimal for each frame, flicker artefacts may occur when the video is viewed due to large differences between lower_bound (or upper_bound) settings on successive video frames. To avoid such flicker, the lower_bound and upper_bound metadata should be smoothed temporally using the pseudo-code function smooth_contrast_metadata() shown below.

// Given a video sequence with frameNum in [1,…,N], first smooth the lower bounds by

// applying the function recursively to all frames by issuing

//   smooth_contrast_metadata(LowerBounds,1),

//   …

//   smooth_contrast_metadata(LowerBounds,N)

// Then smooth the upper bounds by issuing

//   smooth_contrast_metadata(UpperBounds,1),

//   …

//   smooth_contrast_metadata(UpperBounds,N)

// where

//   LowerBounds: vector of lower_bound metadata for the N frames

//   UpperBounds: vector of upper_bound metadata for the N frames

void smooth_contrast_metadata(Vector, frameNum) {

// Vector: vector of metadata to be smoothed

// frameNum: current frame number

cur = Vector[frameNum]

prev = Vector[frameNum – 1]

if Abs((cur – prev) / prev) > Threshold { // Check whether the metadata variation between

// successive frames exceeds the threshold.

if (cur < prev) { // if the current frame’s metadata are lower than the previous frame’s

// metadata, then increase the current frame’s metadata so that it

// reaches the acceptable threshold.

Vector[frameNum] = prev * (1 – Threshold)

} else { // increase the previous frame’s metadata so that it reaches the acceptable

// threshold. Then adjust the metadata for all preceding frames.

Vector[frameNum – 1] = cur / (1 + Threshold)

smooth_contrast_metadata(Vector, frameNum – 1)

}

}

}

The value of Threshold is display independent and can be set to 0.015, which corresponds to a 1.5% metadata variation between successive frames.

Preventing flicker arising from control latency

If DA metadata were unavailable, then to implement DA, the display would have to estimate max_rgb_component[ i ] and immediately adjust the backlight (or voltage). This is impossible in most practical implementations because there is a significant latency of D milliseconds between the instant when the backlight scaling control is applied and the instant when the backlight actually changes, in response to the control. If D is sufficiently large, then the backlight values are not synchronized with the displayed frames and flickering is visible. Fortunately, DA metadata eliminates this flickering. Because the receiver obtains the metadata in advance, the backlight scaling factor can be applied D milliseconds ahead of the video frame with which that scaling factor is associated. Therefore, by transmitting metadata, the latency issue is solved and the backlight scaling factor is set appropriately for each frame. This avoids flicker from backlight changes during video display.

Metadata for DA on displays with control-frequency limitations

Besides eliminating flicker arising from backlight-control latency, DA metadata can also enable DA to be applied to displays in which the backlight (or voltage) cannot be changed frequently. For such displays, once the backlight has been updated it shall retain its value for a time interval that spans the duration of some number of successive frames. After the time interval has elapsed, the backlight may be updated again. DA metadata allows the backlight to be set appropriately for the specified time interval so that maximal power reduction and minimal RGB-component saturation occurs. This appropriate backlight value is determined by aggregating the RGB component histograms in all successive frames in each time interval over which the backlight shall remain constant. The aggregated histograms are then used to derive DA metadata, as explained in preceding subclauses. To enable this mode of operation, the receiver shall signal to the transmitter, constant_backlight_voltage_time_interval, the time interval over which the backlight (or voltage) shall remain constant. Alternatively, the transmitter may assume a reasonable value for constant backlight voltage time interval.

On currently available displays, setting constant_backlight_voltage_time_interval to 100 milliseconds is sufficient to prevent flicker. Therefore, setting num_constant_backlight_voltage_time_intervals = 1 and constant_backlight_voltage_time_interval[ 0 ] = 100 is sufficient to prevent flicker arising from control-frequency limitations. However, in the future, a new display technology with constant_backlight_voltage_time_interval significantly different from 100 milliseconds may be invented. During the transition period from the current display technology to the new display technology, two types of displays are widely used and it is necessary to set num_constant_backlight_voltage_time_intervals = 2, to support both display types. The preceding mode of operation assumes that a signalling mechanism from the receiver to the transmitter does not exist.

However, if such a signalling mechanism does exist, then the receiver can explicitly signal constant_backlight_voltage_time_interval to the transmitter as explained in 7.2.2 and 7.3.2. If the transmitter is additionally capable of re-computing the display adaptation metadata to be consistent with the signalled constant_backlight_voltage_time_interval, then the re-computed metadata can subsequently be provided to the receiver.

DA metadata to prevent flicker from large variations

On some platforms, besides the flicker that arises from control latency and control-frequency limitations, flicker can also occur due to a large difference between the backlight (or voltage) settings (defined as VBacklightScalingFactor in 7.4) of successive video frames. To avoid such flicker, a transmitter may use the function adjust_backlight() to adjust the backlight setting of each frame. Specifically, if the relative backlight variation between a frame and its predecessor is larger than a threshold, then the backlight values of all preceding frames shall be adjusted. This adjustment is done at the transmitter after metadata has been computed using one of the methods described in the preceding subclauses.

For example, for a targeted quality level, the transmitter would estimate max_rgb_component and the corresponding VBacklightScalingFactor for each of N frames. Given max_variation (normalized to 255), the transmitter applies adjust_backlight() with the specified max_variation threshold computed as the floating-point number (max_variation/2048). This function adjusts the vector of VBacklightScalingFactor values for the N frames so that the relative backlight variation between successive frames is less than max_variation. After the backlight values have been adjusted, the DA metadata is modified, if necessary, to be consistent with the adjusted backlight values.

// Given a video sequence with frameNum in [1,…,N], apply the function recursively

// to all frames by issuing adjust_backlight(Backlights,1,max_variation),

// …

// adjust_backlight(Backlights,N,max_variation)

void adjust_backlight(Backlights, frameNum, max_variation) {

// Backlights: vector of VBacklightScalingFactor values

// frameNum: current frame number

// max_variation: maximum permissible backlight variation between two

// consecutive backlight values

cur = Backlights[frameNum]
  prev = Backlights[frameNum – 1]

if Abs((cur – prev) / prev) > max_variation { // Check whether the backlight

// variation between successive frames exceeds the threshold.

if (cur < prev) { // if the current frame’s backlight is lower than the previous

//frame’s backlight, then increase the current frame’s

// backlight so that it reaches the acceptable threshold.

Backlights[frameNum] = prev * (1 – max_variation)

} else { // increase the previous frame’s backlight so that it reaches the

// acceptable threshold. Then adjust the backlights for all preceding

// frames.

Backlights[frameNum – 1] = cur / (1 + max_variation)

adjust_backlight(Backlights, frameNum – 1, max_variation)

}

}

For a given display, large values of max_variation induce more flicker but also save more power. Therefore, the selected value of max_variation is a compromise between flicker reduction and power saving. The max_variation metadata guarantees that the receiver does not experience flicker because the backlights are adjusted specifically for the receiver’s display.

On currently available displays, setting max_variation = 0.015*2 048 is sufficient to prevent flicker. Therefore, setting num_max_variations = 1 and max_variation = 0.015*2 048 is sufficient to prevent flicker arising from control-frequency limitations. However, in the future, a new display technology with max_variation significantly different from 0.015*2 048 may be invented. During the transition period from the current display technology to the new display technology, two types of displays are widely used and it is necessary to set num_max_variations = 2, to support both display types. The preceding mode of operation assumes that a signalling mechanism from the receiver to the transmitter does not exist.

However, if such a signalling mechanism does exist, then the receiver can explicitly signal max_variation to the transmitter as explained in 7.3.2. If the transmitter is additionally capable of re-computing the display adaptation metadata to be consistent with the signalled max_variation, then the re-computed metadata can subsequently be provided to the receiver.

Void

Interactive signalling for remote decoder-power reduction

General

This clause explains how interactive green metadata can be computed at the decoder and how it can be used at the encoder.

Decoding operations reduction request computation and transmission

A terminal can display the expected remaining usage duration based on current battery level and the energy consumption of the current video it is decoding. The user can then act on its terminal to increase this usage duration, which are translated into a power-consumption saving ratio.

This power-consumption saving ratio cannot be sent as is to the remote device because the relationship between power consumption and processing cycles of a processor is not linear and is processor/device dependent.

Using the mapping between power consumption and power supply voltage and the mapping between power supply voltage and processing frequency of processors or devices, the terminal can translate the power-consumption saving ratio into decoding operations reduction request (DOR-Req) metadata (step 1 in Figure B.4).

The DOR-Req metadata are sent as green feedback by the device in an out-of-band message (step 2 in Figure B.4).

Figure B.4 — Production of the DOR-Req message

Use of decoding operations reduction request (DOR-Req) and decoding operation response (DOR-Resp)

The decoding operations reduction request (DOR-Req) metadata are extracted in the remote device and presented to the power optimization module (step 3 in Figure B.5) which translates this request into a configuration of the encoder (step 4 in Figure B.5), so that it can produce a stream which complies to the DOR-Req (step 5 in Figure B.5).

Figure B.5 — Usage of the DOR-Req message

Thus, each encoder can adapt the complexity of the encoded stream as a function of the battery level of the other device communicating with it.

The strategy used by the encoder to reduce complexity is non-normative. A gradual action can be used to find the best compromise: decoding complexity vs. perceived quality. For example, when the first DOR-Req mode is used (corresponding to dec_pow_reduction_type equal to 0), the gradual action is controlled through the monitoring of the DOR-Req parameter dec_ops_reduction_req. When the DOR-Req parameter dec_ops_reduction_req becomes negative, the encoder knows that it can gradually increase the complexity to reverse its previous actions.

If the two devices are equipped with batteries, the best strategy can be defined by considering the power-saving requests of both devices as shown in Figure B.6.

Figure B.6 — Using local and remote information in the power-optimizer module

This technology achieves maximum power saving in association with C-DVFS technology. As shown in Figure B.5, the DOR-Req messages are directed at remote encoders. However, if these remote encoders produce C-DVFS SEI messages in association with the video stream, then the local decoders can use the C-DVFS SEI message for the maximum power saving achieved from the change initiated by the DOR-Req message.

In response to a DOR-Req message, a decoding operations reduction response (DOR-Resp) mechanism can also be used by the remote device to notify immediately the receiver without needing to get the information from the bistream (i.e., through the corresponding SEI message). The DOR-Resp message provides information whether the remote encoders partially or globally accept the received request., The remote encoder can also decide to reply to the request with different changes in its coding process, than the ones requested by the decoder. Thus, it further allows the receiver to further adapt its power optimization behavior according to this response.

When a DOR-Resp mechanism is used to acknowledge a request of cancellation, with a list of requested types, it contains the list of types of the requests the encoder has accepted to perform. In this case, the last requests of types corresponding to the list are cancelled.

When nb_dec_pow_reduction_type_resp of the DOR-Resp message equal 0, the message is used to acknowledge a global cancellation of all previous decoding operation reduction requests, independently of their type. In this case, the information of image resolution and frame rate are sent to the decoder.

Cross-segment decoding for quality recovery after low-power encoding

General

An encoder can achieve power reduction by encoding alternate high-quality and low-quality Segments, in a segmented delivery mechanism such as DASH. The power reduction occurs because low-complexity encoding mechanisms (fewer encoding modes, fewer reference pictures, smaller search ranges, etc.) are used to produce the low-quality Segments. A metric describing the quality of the last picture of each Segment is delivered as metadata to the decoder. This clause describes how cross-segment decoding can be used to improve the quality of the low-quality Segments.

A cross-segment decoder utilizes quality metrics contained in the high-quality Segments (from high complexity encoding) to enhance decoding of the low-quality Segments (from low complexity encoding), producing a visual experience with significantly higher QoE, but with reduced average encoding complexity (and therefore reduced encoding power consumption).

Note that the decoding complexity for the first picture in the low-quality Segment is increased, while the decoding complexity for the other pictures remains the same as for regular decoders.

Green metadata Usage

At the transmitter, the encoder records the quality metric of the last picture of each Segment using xsd_metric_type and xsd_metric_value. The XSD-enabled decoder, when it receives the metric data, uses the metrics to determine if it executes an enhancement algorithm. If the metric indicates that the last picture of the previous Segment is of better quality than the first picture of the new Segment, then it uses the last picture of the previous Segment to enhance the first picture of the new Segment as described below.

The XSD algorithm applies to the transition from a Segment with higher video quality to a temporally neighbouring Segment with poorer quality that is encoded independently of the higher quality Segment. The last picture (in display order) in the higher quality Segment is the “good picture” (GP). The first IDR picture of the poor quality Segment is the “start picture” (SP). The output from the current algorithm is the “fresh start” (FS). Note that the SP as an IDR picture was encoded without referencing the GP or any other pictures in the higher quality Segment. The goal of the enhancement algorithm is to use information contained in the GP to improve the quality of the decoded SP to get an improved reference picture, FS, for subsequent pictures in the low quality Segment.

Depending on the level of motion for different spatial regions of the SP, two enhancement methods are used by the decoder, one for relatively low-motion areas, the other for the higher-motion areas. For both algorithms, the decoder looks for matches between areas in the decoded GP and the SP, as determined by a distortion metric and a threshold calculated by the decoder.

NOTE	MSD = Mean Square Difference in the following algorithm description.

// To estimate MVs, different algorithms can be used. Square Diamond Search is described below.

for each block B in SP

set the centre to B.

calculate the SAD between B and the co-located block B' in GP.

repeat
      calculate the SAD between B and the block in the up left, up right,

down left, down right of B' in GP.

select the block leading to the minimum SAD as the next centre.

set B' as last centre

until the centre = last centre;

repeat
      calculate the SAD between B and the block in the left, right, up, down of B' in GP.

select the block leading to the minimum SAD as the next centre.

set B' as last centre

until the centre = last centre

Set C' to the centre in GP.

Set MV(B) to the motion vector from C' to B

// Enhance the new SP

calculate the average Sum of Absolute Difference (AvgSAD) of the MVs.

calculate TMSD = 0.775 * e0.4306 * AvgSAD + 132.4.

for each 16 x16 patch P in SP

// QP is average quantization parameter in frame.

// width is the width of the frame.

// len(MV(P)) is the number of bits to code MV(P).

if (len(MV(P)) < width * QP /30000) // Use low-motion enhancement method

calculate the MSD between P and the co-located patch P' in GP

if (MSD < = TMSD)

copy P' to P;

else // P is high motion – use high-motion enhancement method

for each 4x4 block B in P

good_mv = 0;

for each of 8 mv near MV(B)

if mv = MV(B)

good_mv++;
             if (good_mv >= 5)

calculate the MSD between B and co-located B' in GP referenced by MV(B)

if (MSD <= TMSD)

copy B' to B;

After the FS-Picture Generation algorithm is applied, decoding of subsequent pictures is done as usual.

Interactive signalling for display power reduction through the use of attenuation maps

General

This clause explains how a receiver device can request attenuation maps with specific energy reduction rates while also giving information on which processes it is capable to apply on the attenuation maps. It also explains how the transmitter can respond to the receiver’s request.

Display attenuation map power reduction request (DAMPR-Req) and display attenuation map power reduction response (DAMPR-Resp).

A use case example where a receiver is notified of a low level battery is shown in Figure B.7 4.

In this use case example, the transmitter has announced the service#1 as a green profile service in step1.

Figure B.7 - display attenuation map power reduction request when a low level of battery is raised

In step 2, the user selects the service#1 from their service list, and its energy aware application requests for the service.

In step 3, the reception of the video data is started.

In step 4, as an example, a low battery level is raised, and the receiver decides to initiate the process of attenuation map display adaptation to reduce its energy consumption.

In step 5, the display attenuation map power reduction request (DAMPR-Req) is sent from the receiver to the transmitter with a list of energy reduction rates, and possibly with its capabilities to use and apply Attenuation Maps. The transmitter sends a display attenuation map power reduction response (DAMPR-Resp) to acknowledge reception of the receiver’s request, which contains a list of the attenuation maps corresponding to the request, that will be sent.

In step 6, attenuation maps and green metadata SEI are inserted in the bitstream and the display adaptation process is performed to reduce energy consumption of the receiver.

In step 7, the receiver decides to stop the display adaptation process and sends a display attenuation map power reduction request with a cancel flag.

When the transmitter has stopped the production of the attenuation map (s) and the related AMI metadata, it sends the display attenuation map power reduction response (DAMPR-Resp) with an acknowledgement of the cancel request.

Use of the display attenuation map power reduction request

On reception by the transmitter of a display attenuation map power reduction request, the transmitter will compute attenuation maps corresponding to the requested energy reduction rates and compatible with the capabilities of the receiver as per their use and application on the main video bit stream (step 5).

For example, if the receiver requested two attenuation maps for the energy reduction rates 10% and 20% and specifies that it can only apply them through multiplication of the attenuation map samples values with the sample values of the luminance component of the main decoded picture, the transmitter may compute and transmit two attenuation maps for 10% and 20%, that should be applied by multiplication of their sample values with the luminance component of the main decoded picture.

This will enable a saving of bandwidth by sending only specific attenuation maps as requested by the receiver.

The strategy used by the encoder to compute the attenuation maps is non normative. In the above use case, the transmitter may only transmit one single attenuation map which corresponds to the receiver’s request, for example for the energy reduction rate 10%, in case it is not capable of computing an attenuation map with the energy reduction rate of 20%. In this case, it will send a display attenuation map power reduction response (DAMPR-Resp) with information on only one attenuation map for 10%.

    - B.6.4	Display power reduction attenuated video request (DPRAV-Req) and display power reduction attenuated video response (DPRAV-Resp).

Figure B.8 - display attenuation map power reduction request when a low level of battery is raised

Figure B.8 describes a use case example where the receiver has been notified by a low level of battery.

In step 1, the transmitter announces the service#1 as a green profile service.

In step 2, the user selects the service#1 from his service list, the energy aware application request for the service and in step 3, the reception of the video data is started.

In step 4, as an example, a low battery level is raised and the receiver decides to initiate the process of display adaptation to reduce its energy consumption.

In step 5, the display power reduction attenuated video request is sent from the receiver to the transmitter with an energy reduction rate and/or a video quality reduction expected value(s).

In step 6, a display attenuation map is produced and applied to the video content. The attenuated video content is then inserted in the bitstream with the green metadata SEI. The bitstream is received by the receiver and the video content is rendered on the display.

In step 7, the receiver decides to stop the attenuated video process and send a display power reduction attenuated video request with a cancel request.

- B.6.5 Use of the display power reduction attenuated video request

Figure B.9 - display power request attenuated video request when a low battery level is raised

On reception by the transmitter of a display power reduction attenuated video request, the transmitter will compute the display attenuation map corresponding to the requested energy reduction rate and quality level, compatible with the capabilities of the receiver as per their use and application on the main video bit stream (step 5 of Figure B.9).

For example, if the receiver requests for an attenuated video with the energy reduction rate 20% and specifies that it can only use the attenuated video on an emissive display, the transmitter may compute a display attenuation map for 20% and dedicated to emissive displays, apply it on the base video to generate an attenuated video. This attenuated video may then be encoded and transmitted up to the decoder.

This will enable a saving of bandwidth by directly sending an attenuated video, without the need to also send display attenuation maps.

The strategy used by the encoder to compute the display attenuation maps is non normative.

In the above use case, the transmitter may transmit an attenuated video with an energy reduction rate that does not correspond to the receiver’s request, for example for the energy reduction rate 10%, in case it is not capable of computing a display attenuation map with the energy reduction rate of 20%. In this case, it will send a display power reduction attenuated video response (DPRAV-Resp) with information on an energy reduction rate of 10%.

(normative)

Conformance and reference software

Complexity metrics for decoder-power reduction

Conformance test vectors

Table C.1 describes the two 4:2:0 8 bit per sample AVC conformance bitstreams with embedded green metadata SEI message available at http://standards.iso.org/iso-iec/23001/-11/ed-3/en.

Table C.1 – AVC conformance bitstreams with complexity metrics

| name | resolution /Frame rate (Fps) | RAP period (number of frames) | bitrate (Mbps) | green metadata SEI period (number of frames) | profile | expected value |
| --- | --- | --- | --- | --- | --- | --- |
| mobcal_480p50_AVC_HP.bin | 704x480p@50 | 15 | 2.798 | 1 | High | mobcal_480p50_AVC_HP.txt |
| stockholm_720p5994_AVC_HP.bin | 1280x720p@59.94 | 15 | 7.205 | 1 | High | stockholm_720p5994_AVC_HP.txt |

To verify conformance of a software implementation of green metadata SEI message parsing, the conformance streams shall be used to check that extracted values match expected values given in the side text files provided with the conformance streams.

Table C.2 describes four 4:2:0 10 bit per sample VVC conformance bitstreams with embedded green metadata SEI message available at http://standards.iso.org/iso-iec/23001/-11/ed-3/en.

Table C.2 – VVC conformance bitstreams with complexity metrics

| name | resolution /Frame rate (Fps) | RAP period (number of frames) | bitrate (Mbps) | green metadata SEI period (number of frames) | profile | expected value |
| --- | --- | --- | --- | --- | --- | --- |
| blowingBubbles_testCMs_ext.vvc | 416x240@50 | 50 | 1.561 | 1 | Main10 | blowingBubbles_testCMs_ext.txt |
| blowingBubbles_testCMs_reduced.vvc | 416x240@50 | 50 | 1.561 | 1 | Main10 | blowingBubbles_testCMs_reduced.txt |
| cactus_testCMs_ext.txt | 1920x1080p@50 | 32 | 1.074 | 1 | Main10 | cactus_testCMs_ext.vvc |
| cactus_testCMs_reduced.txt | 1920x1080p@50 | 32 | 1.074 | 1 | Main10 | cactus_testCMs_reduced.vvc |

To verify conformance of a software implementation of VVC green metadata SEI message parsing, the conformance streams shall be used to check that extracted values match expected values given in the side text files provided with the conformance streams.

Reference software

Reference decoder software provided in ISO/IEC 14496-5 [6] or Rec. ITU-T H.264.2 integrates a green metadata SEI message parser for AVC bitstreams, which extracts and displays SEI messages from conformance and test bitstreams.

To enable the green metadata SEI message parser, the source code shall be compiled with the macro #define PRINT_GREEN_METADATA_INFO.

To verify conformance of a test green metadata SEI message generated from a video in a test AVC bitstream, the reference decoder software provided in ISO/IEC 14496-5 or Rec. ITU-T H.264.2 shall be used to extract the test SEI message from the test bitstream and then to check the message for syntactic correctness and valid ranges.

Reference decoder software provided in ISO/IEC 23090-16 [7] integrates a green metadata SEI message parser for VVC bitstreams, which extracts and displays SEI messages from conformance and test bitstreams.

To verify conformance of a test green metadata SEI message generated from a video in a test VVC bitstream, the reference decoder software provided in ISO/IEC 23090-16 shall be used to extract the test SEI message from the test bitstream and then to check the message for syntactic correctness and valid ranges.

Void

Void

Metrics for quality recovery after low-power encoding

Conformance test vectors

Table C.3 describes 4:2:0 8 bits AVC conformance bitstream with embedded green metadata SEI message available at http://standards.iso.org/iso-iec/23001/-11/ed-3/en.

Table C.3 – AVC conformance bitstreams with quality metrics

| name | resolution / frame rate (Fps) | bitrate (Mbps) | profile | expected value |
| --- | --- | --- | --- | --- |
| crowdrun_1080p50_AVC_HP.bin | 1920x1080p@50 | 10 | High | 28.70 |

The bitstream embeds for the last frame a SEI message that contains PSNR value of this frame, as described in this document.

To verify conformance of a software implementation of green metadata SEI message parsing, the conformance AVC stream shall be used to check that the extracted PSNR value matches the expected value given in the side text file provided with the conformance stream.

Table C.4 describes 4:2:0 8 bit per sample HEVC conformance bitstream with embedded green metadata SEI message available at http://standards.iso.org/iso-iec/23001/-11/ed-3/en.

Table C.4 – HEVC conformance bitstreams with quality metrics

| name | resolution / frame rate (Fps) | bitrate (Mbps) | profile | expected value |
| --- | --- | --- | --- | --- |
| kimono_1080p24_HEVC_MP.bin | 1920x1080p@24 | 1 | Main | 37.99 |

The bitstream embeds for the last frame a SEI message that contains the PSNR value of this frame as described in this document.

To verify conformance of a software implementation of green metadata SEI message parsing, the conformance HEVC stream shall be parsed to check that the PSNR extracted value matches the expected value given in the side text file provided with the conformance stream.

Table C.5 describes 4:2:0 10 bit per sample VVC conformance bitstreams with embedded green metadata SEI message available at http://standards.iso.org/iso-iec/23001/-11/ed-3/en.

Table C.5 – VVC conformance bitstreams with quality metrics

| name | resolution / frame rate (Fps) | bitrate (Mbps) | profile | expected value (PSNR) | expected Value (SSIM) |
| --- | --- | --- | --- | --- | --- |
| blowingBubbles_testQMs.vvc | 416x240@50 | 6.264 | Main10 | frame 0: 42.60 frame 1: 39.61 | - |
| cactus_testQMs.vvc | 1920x1080p@50 | 9.271 | Main10 | frame 0: 35.03 frame 1: 34.09 | frame 0: 0.97 frame 1: 0.97 |

The bitstreams embed for frames 0 and 1 a SEI message that contains the PSNR (blowingBubbles_testQMs.vvc) and the PSNR and the SSIM (cactus_testQMs.vvc) of two frames as described in this document.

To verify conformance of a software implementation of green metadata SEI message parsing, the conformance VVC streams shall be parsed to check that the PSNR and, when relevant, SSIM extracted values match the expected values given in the side text files provided with the conformance streams.

Reference software

Reference decoder software provided in ISO/IEC 14496-5 or Rec. ITU-T H.264.2 integrates a green metadata SEI message parser, which extracts and displays SEI messages from conformance and test AVC bitstreams.

To enable the green metadata SEI message parser, the source code shall be compiled with the macro #define PRINT_GREEN_METADATA_INFO.

To verify conformance of a test green metadata SEI message generated from a video in a test AVC bitstream, the reference software provided in ISO/IEC 14496-5 or Rec. ITU-T H.264.2 shall be used to extract the test SEI message from the test bitstream and then to check the message for syntactic correctness and valid ranges.

Reference decoder software provided in ISO/IEC 23008-5 [5] integrates a green metadata SEI message parser, which extracts and displays SEI messages from conformance and test HEVC bitstreams.

To verify conformance of a test HEVC green metadata SEI message generated from a video in a test HEVC bitstream, the reference software provided in ISO/IEC 23008-5 shall be used to extract the test SEI message from the test bitstream and then to check the message for syntactic correctness and valid ranges.

Reference decoder software provided in ISO/IEC 23090-16 integrates a green metadata SEI message parser, which extracts and displays SEI messages from conformance and test VVC bitstreams.

To verify conformance of a test green metadata SEI message generated from a video in a test VVC bitstream, the reference software provided in ISO/IEC 23090-16 shall be used to extract the test SEI message from the test bitstream and then to check the message for syntactic correctness and valid ranges.

(informative)

Objective distortion metrics

General

This annex provides a description of the processes for computing objective distortion metrics between samples of a reference picture and samples of a test picture, the reference and test pictures being made of 1 or 3 colour planes (e.g. YCbCr or RGB).

PSNR

The PSNR metric is calculated from the mean squared error of the sample values. PSNR is calculated individually for a single colour channel. A definition of PSNR is provided in ISO/IEC 23001-10.

The inputs of PSNR metric estimation process are:

—	the reference samples, xref c [ p ] with c being the index of the colour plane for which the metric is computed and and p being the samples relative location in the picture.

—	the test samples, xtest c [ p ] with c being the index of the colour plane for which the metric is computed and p being the samples relative location in the picture.

—	the bit depth B of the samples of the colour channel c.

The output of this process is:

—	the PSNR metric VPSNR.

A description of the metric derivation process follows.

The PSNR metric VPSNR is calculated as:

VPSNR=10*Log10X2vMSE

where X is the maximum sample value for the specific bit depth B derived as ( ( 1 << B ) – 1 ) and VMSE is given as:

VMSE= 1Ncp(xref c [ p ] -xtest c [ p ] )2

where Nc is the number of samples in the considered colour plane of index c.

NOTE: In this document, the PSNR metric is derived for channel c = 0.

wPSNR

The wPSNR metric, or weighted PSNR metric, is calculated from a weighted mean squared error of the sample values, that can be used as distortion metric of Rec. BT.2100 PQ video content. wPSNR is calculated individually for a single luma (c = 0) or chroma (c = 1 or 2) channel. A definition of wPSNR is provided in document ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-V2011.

The inputs of wPSNR metric estimation process are:

—	the reference luma samples, xref 0 [ p ] with p being the samples relative location in the picture.

—	the reference samples, xref c [ p ] with c being the index of the colour plane for which the metric is computed and and p being the samples relative location in the picture.

—	the test samples, xtest c [ p ] with c being the index of the colour plane for which the metric is computed and p being the samples relative location in the picture.

—	the bit depth B of the samples of the colour channel c.

—	the bit depth B0 of the samples of the luma colour channel c = 0.

The output of this process is:

—	the wPSNR metric vwPSNR.

A description of the metric derivation process follows.

The wPSNR metric VwPSNR is calculated as:

VwPSNR=10*Log10X2wMSE

where X is the maximum sample value for the specific bit depth B derived as ( ( 1 << B ) – 1 ) and wMSE is given as

wMSE=1Ncp in picturewxref 0 [ p ]*(xref c [ p ] -xtest c [ p ] )2

where Nc is the number of samples in the considered colour plane of index c, wxref 0 [ p ] is a weight that is a function of the luma sample value xref 0 [ p ] at relative location p in the picture.

The weight wxref 0 [ p ] is computed as follows:

—	the value XL is derived as follows.

—	if B0 is greater than or equal to 10, XL is set equal to (xref 0  p ≫(B0 -10)).

—	otherwise (B0 is lower than 10), XL is set equal to (xref 0  p ≫(10- B0)).

—	the value YP is set equal to Clip3(–3, 6, 0.015 * XL – 1.5 – 6).

—	wxref 0 [ p ]  is set equal to power(2.0, YP ÷ 3.0).

NOTE: In this document, the wPSNR metric is derived for channel c = 0.

WS-PSNR

The WS-PSNR metric, or weighted to spherically uniform PSNR metric, is calculated from a weighted mean squared error of the sample values, that can be used as distortion metric of 360° video content. WS-PSNR calculates PSNR using all image samples on the 2D projection plane. The distortion at each position p is weighted by the spherical area covered by that sample position. WS-PSNR is calculated individually for a single luma (c = 0) or chroma (c = 1 or 2) channel. A definition of WS-PSNR is provided in document ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-T2004.

The inputs of this process are:

—	the reference samples, xref c [ p ] with c being the index of the colour plane for which the metric is computed and p being the samples relative location in the picture.

—	the test samples, xtest c [ p ] with c being the index of the colour plane for which the metric is computed and p being the samples relative location in the picture.

—	the bit depth B of the samples of the colour channel c.

The output of this process is:

—	the WS-PSNR metric VWS-PSNR.

A description of the metric derivation process follows.

The WS-PSNR metric VWS-PSNR is calculated as:

VWS-PSNR =10*Log10X2vWS-MSE

where X is the maximum sample value for the specific bit depth B derived as ( ( 1 << B ) – 1 ) and 
VWS-MSE is given as

VWS-MSE= 1Ncp in picturewsp*(xref c [ p ] -xtest c [ p ] )2

where Nc is the number of samples in the considered colour plane of index c, and wsp is a weight that is a function of the relative location p in the picture.

Since WS-PSNR is calculated based on the projection plane, different weights are derived for different projection formats. Weight derivation for different projection formats is discussed below.

NOTE: In this document, the WS-PSNR metric is derived for channel c = 0.

WS-PSNR calculation for equi-rectangular projection (ERP) format with optional padding

For an M*N image in the ERP format, the weight wsp at relative position p=( i , j ) in the picture is calculated as:

wsp=cosj+0.5-N2πN

WS-PSNR calculation for cubemap projection (CMP) and hemisphere cubemap projection (HCMP) formats

For an image in CMP format, weight distributions on all faces are the same. Therefore, only weights in one face are derived. For relative position p=(i,j) on a CMP face with resolution A*A, the weight wsp is calculated as:

wsp=1+d2i,jr2-(3÷2)

where r=A2 is the radius, and d2i,j=(i+0.5-A/2)2+(j+0.5-A/2)2 is the squared distance between (i,j) and the center of the face.

The weights for HCMP are based on those used for CMP, but only consider the active full face and four half-faces.

SSIM

SSIM is calculated individually for a single colour channel.

The inputs of SSIM metric estimation process are:

—	the reference samples, xref c [ p ] with c being the index of the colour plane for which the metric is computed and and p being the samples relative location in the picture.

—	the test samples, xtest c [ p ] with c being the index of the colour plane for which the metric is computed and p being the samples relative location in the picture.

—	the bit depth B of the samples of the colour channel c.

—	the width W and height H of the colour channel c.

The output of this process is:

—	the SSIM metric VSSIM.

A description of the metric derivation process follows.

The SSIM metric VSSIM is calculated as:

VSSIM= 1(W-S-1)(H-S-1)i=0W-S-1j=0H-S-1VSSIM(i,j)

where S=8 is the block width and height used to compute VSSIM(p), and VSSIM(p) is computed as follows for the relative location p=(i,j) in the picture as follows:

VSSIMp=2*μrefp*μtestp+c1*(2*σref,test(p)+c2)μrefp2+μtestp2+c1*(σref(p)2+σtest(p)2+c2)

where c1 = (k1 * X)2, c2 = (k2 * X)2, k1 = 0.01, k2 = 0.03, X = ( ( 1 << B ) – 1 ),

and where μref(p), μtest(p), σref(p), σref(p) and σref,test(p) are defined as follows:

—	μsig(p) denotes the average value of reference samples xref c [ r ], for r in the 8x8 window with top left pixel positioned at relative location p=(i,j) in the picture,

—	μtest(p) denotes the average value of test samples xtest c [ r ], for r in the 8x8 window with top left pixel positioned at relative location p=(i,j) in the picture,

—	σref(p) denotes the variance value of reference samples xref c [ r ], for r in the 8x8 window with top left pixel positioned at relative location p=(i,j) in the picture,

—	σtest(p) denotes the variance value of test samples xtest c [ r ], for r in the 8x8 window with top left pixel positioned at relative location p=(i,j) in the picture,

—	σref,test(p) denotes the covariance value of reference samples xref c [ r ] and test samples xtest c [ r ], for r in the 8x8 window with top left pixel positioned at relative location p=(i,j) in the picture.

μsig(p) is derived as follows, for p=(i,j) and for sig=ref or test:

—	the parameter μ is set equal to 0

—	for k=0..(S – 1) and l=0..(S – 1) the following applies:

μ =  μ+ xsig c [ i+k ][ j+l ]

—	μsigp is set equal to μ÷(S*S)

σsig(p) is derived as follows, for p=(i,j) and for sig=ref or test:

—	the parameter μ2 is set equal to 0

—	for k=0..(S – 1) and l=0..(S – 1) the following applies:

μ2= μ2+ xsig c  i+k  j+l *xsig c [ i+k ][ j+l ]

—	σsigp is set equal to (μ2÷S*S- μsigp*μsigp)

σref,test(p) is derived as follows, for p=(i,j):

—	the parameter μcross is set equal to 0

—	for k=0..(S – 1) and l=0..(S – 1) the following applies:

μcross =  μcross+ xref c  i+k  j+l *xtest c [ i+k ][ j+l ]

—	σref,test(p)  is set equal to (μcross÷S*S- μrefp*μtestp)

NOTE: In this document, the SSIM metric is derived for channel c = 0.

Bibliography

[1]	ISO/IEC 13818-1, Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems

[2]	ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format

[3]	ISO/IEC 23001-10, Information technology — MPEG systems technologies — Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File

[4]	ISO/IEC/TR 23009-3, Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 3: Implementation guidelines

[5]	ISO/IEC 23008-5, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 5: Reference software for high efficiency video coding

[6]	ISO/IEC 14496-5, Information technology — Coding of audio-visual objects — Part 5: Reference software | Rec. ITU-T H.264.2: Reference software for ITU-T H.264 advanced video coding

[7]	ISO/IEC 23090-16, Information technology — Coded representation of immersive media — Part 16: Reference software for versatile video coding