ISO/IEC JTC 1/SC 29/WG 03 N1752

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Exploration on Systems technologies for coding for machine standards

Status:	Approved

Date of document:	2026-02-09

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	9 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N1752

January 2026, online

| Title | Exploration on Systems technologies for coding for machine standards |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 25953 |

Summary

During the MPEG 153rd meeting, a first proposal was provided on systems technologies for coding for machine standards, focusing on the carriage of VCM NAL unit structured video in the ISO Base Media File Format (ISOBMFF). This document summarizes the outcome of that work and reflects key questions raised during subsequent MPEG discussions, including the use of multiple elementary streams for VCM and the placement of inner codec configuration information.

This exploration provides background information on system-level considerations for VCM bitstream carriage, analyses ideas around the definition and placement of configuration and metadata and discusses open issues and potential directions for further study.

1	Introduction

With the establishment of the DIS for Video Coding for Machines (VCM), the definition of system-level mechanisms for storing and conveying VCM bitstreams has become necessary for practical deployment. In particular, compressed VCM bitstreams are expected to be used within existing media container and delivery frameworks.

Contribution m75471 addressed the carriage of VCM NAL unit structured video in the ISO Base Media File Format. It defined the elementary stream structure, the mapping of NAL units to samples, and a VCM-specific decoder configuration record, while maintaining alignment with existing ISOBMFF carriage mechanisms for NAL unit structured video.

2	Overview: Carriage of NAL unit structured video in the ISOBMFF for VCM

X VCM elementary streams and sample definitions

X.1 Overview

The Video Coding for Machines (VCM) standard (ISO/IEC 23888-2) offers increased coding efficiency and many features for the systems that use it. To enable the best visibility of, and access to, those features, and to enhance the opportunities for the interchange and interoperability of media. This clause of ISO/IEC 14996-15 specifies the storage format for single-layer VCM (ISO/IEC 23888-2) video streams.

The storage of VCM content uses the existing capabilities of the ISO base media file format but also defines extensions to support the following features of the VCM codec.

- The video, sequence, and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded video data. In VCM, a picture that contains coded video data(VCM_CVD) references the picture parameter set(VCM_RSD_PRD) containing its decoding parameters. When a picture does not contain coded video data, restoration information for the picture is conveyed solely in a temporal synthesis picture data(TSPD) NAL unit. In turn, picture-level restoration and temporal synthesis information (conveyed in VCM_RSD_PRD and TSPD NAL units)  references a sequence parameter set that contains sequence level decoding parameter information, and the sequence parameter set references a video parameter set that contains global decoding parameter information.

X.2 Elementary stream structure

Two types of elementary streams are defined for storing VCM content:

- Video elementary stream that does not contain any parameter sets; all parameter sets are stored in a sample entry or sample entries;

- Video and parameter set elementary stream that may contain parameter sets, and may also have parameter sets stored in their sample entry or sample entries;

X.3 Sample and configuration definition

X.3.1 Canonical order and restrictions

The canonical stream format is a VCM elementary stream that satisfies the following conditions in addition to the general conditions in 4.2.2:

NOTE 	The general conditions referenced in 4.2.2 are defined in Clause 4 (General definitions).

—	All VCL NAL units for a single picture shall be contained within the sample whose decoding time and composition time are those of the picture.

- A sample may contain either coded video data NAL units or temporal synthesis picture data NAL units, according to the VCM bitstream syntax

—	Access unit delimiter NAL units: The constraints obeyed by access unit delimiter NAL units are defined in ISO/IEC 23888-2.

—	Parameter sets: For a VCM video stream to which a particular sample entry applies, VCM parameter set NAL units, including the VCM video parameter set (VCM_VPS) and VCM restoration data NAL(VCM_RSD) units carrying sequence-level or picture-level parameter information, shall be stored only in the sample entry when the sample entry name is 'vcm1', and may be stored in both the sample entry and the samples when the sample entry name is 'vcmi'.

NOTE 1	A parameter set NAL unit to be used for decoding or restoration of an output picture shall be conveyed prior to the sample associated with that output picture or within the sample itself.

NOTE 2	Storing VCM parameter set NAL units in the sample entry of a VCM video stream provides a simple and static method for supplying parameter information. Storing VCM parameter set NAL units in samples allows greater flexibility, including updates of parameter information using the same identifier or the introduction of additional parameter sets. A VCM decoder initializes using the parameter sets present in the sample entry and may subsequently update its state using parameter sets encountered in the stream, starting from any sample marked as a sync sample. When the sample entry changes, the decoder re-initializes using the parameter sets contained in the new sample entry.

— SEI messages: SEI message NAL units of a declarative nature may be stored in the sample entry. There is no restriction on the presence or removal of such SEI message NAL units in samples.

— Filler data: Video data is naturally represented as variable bit rate in the file format and should be filled for transmission if needed. Filler Data NAL units and Filler Data SEI messages shall not be present in the stored stream when the sample entry does not permit in-stream parameter sets.

NOTE 3	The removal or addition of Filler Data NAL units, start codes, SEI messages or Filler Data SEI messages can change the bitstream characteristics with respect to conformance with the VCM HRD model when operating the HRD in CBR mode as specified in ISO/IEC 23888-2 Annex C.

X.3.2 Decoder configuration information

X.3.2.1 VCM decoder configuration record

X.3.2.1.1 Definition

X.3.2.1.2 Syntax

aligned(8) class VCMDecoderConfigurationRecord {
	unsigned int(8) configurationVersion = 1;
	unsigned int(1) general_tier_flag;
	unsigned int(7) general_profile_codec_group_idc;
	unsigned int(16) inner_codec_config_length;
	bit(8*inner_codec_config_length) inner_codec_config_bytes;
	unsigned int(8) general_profile_restoration_idc;
	unsigned int(8) general_level_idc;
	unsigned int(16) avgFrameRate;
	unsigned int(1) constantFrameRate;
	bit(5) reserved = '11111'b;
	unsigned int(2) lengthSizeMinusOne; 
	unsigned int(8) numOfArrays;
	for (j=0; j < numOfArrays; j++) {
		unsigned int(1) array_completeness;
		bit(1) reserved = 0;
		unsigned int(6) NAL_unit_type;
		unsigned int(16) numNalus;
		for (i=0; i< numNalus; i++) {
			unsigned int(16) nalUnitLength;
			bit(8*nalUnitLength) nalUnit;
		}
	}
}

X.3.2.1.3 Semantics

general_tier_flag, general_profile_codec_group_idc, general_profile_restoration_idc, general_level_idc, contain the matching values for the fields general_tier_flag, general_profile_codec_group_idc, general_profile_restoration_idc, and general_level_idc as defined in ISO/IEC 23888-2, for the stream to which this configuration record applies.

inner_codec_config_length specifies the length, in bytes, of the inner codec initialization information carried in the field inner_codec_config_bytes. The value of inner_codec_config_length shall be greater than zero when initialization information for an inner codec is present, and shall be equal to zero when no inner codec initialization information is carried in this configuration record.

inner_codec_config_bytes indicates the inner codec initialization information included in the configuration record for the stream to which this configuration record applies. The value of inner_codec_config_bytes represents a byte sequence of length inner_codec_length bytes (8 × inner_codec_length bits), corresponding to the complete decoder configuration information of the indicated inner codec.

The contents of inner_codec_config_bytes shall be conveyed in a codec-specific format and shall be equivalent in size and content to the decoder configuration record defined for the corresponding video coding standard. The information carried in inner_codec_config_bytes shall be sufficient to initialize the inner codec decoder prior to decoding any coded video data in the VCM bitstream.
NOTE 4	The field inner_codec_config_bytes is used to carry, in encapsulated form, the decoder initialization information of an inner codec operating within the VCM decoding process. The inner codec corresponds to a video coding standard defined outside this specification.

avgFrameRate gives the average frame rate in units of frames/(256 seconds), for the stream to which this configuration record applies. Value 0 indicates an unspecified average frame rate.

constantFrameRate equal to 1 indicates that the stream to which this configuration record applies is of constant frame rate. Value 0 indicates that the output frame rate may vary. (Value 0 indicates that the stream may or may not be of constant frame rate.)

lengthSizeMinusOne plus 1 indicates the length in bytes of the NALUnitLength field in an VCM video sample in the stream to which this configuration record applies. For example, a size of one byte is indicated with a value of 0. The value of this field shall be one of 0, 1, or 3 corresponding to a length encoded with 1, 2, or 4 bytes, respectively.

numArrays indicates the number of arrays of NAL units of the indicated type(s).

array_completeness when equal to 1 indicates that all NAL units of the given type are in the following array and none are in the stream; when equal to 0 indicates that additional NAL units of the indicated type may be in the stream; the permitted values are constrained by the sample entry name.

NAL_unit_type indicates the type of the NAL units in the following array (which shall be all of that type); it takes a value as defined in ISO/IEC 23888-2; it is restricted to take one of the values indicating a VCM_VPS, VCM_RSD_SRD, VCM_SRD_PRD, VCM_TSPD, VCM_RSD_SEI message NAL unit.

numNalus indicates the number of NAL units of the indicated type included in the configuration record for the stream to which this configuration record applies. The SEI array shall only contain SEI messages of a 'declarative' nature, that is, those that provide information about the stream as a whole. An example of such an SEI could be a user-data SEI.

nalUnitLength indicates the length in bytes of the NAL unit.

nalUnit contains an VCM_RSD_SRD, VCM_RSD_PRD, VCM_TSPD, VCM_VPS or declarative SEI NAL unit, as specified in ISO/IEC 23888-2.

X.4 Derivation from the ISO base media file format

X.4.1 VCM stream definition

X.4.1.1 Sample entry name and format

X.4.1.1.1 Definition

Sample Entry and Box Types:	'vcm1', 'vcmi', 'vcmC'
Container:	Sample Table Box ('stbl')
Mandatory:	An 'vcm1' or 'vcmi' sample entry is mandatory
Quantity:	One or more sample entries may be present

Box Type:	'vcmC'
Container:	VCM Sample Entry ('vcm1' or 'vcmi')
Mandatory:	Yes 
Quantity:	One

A VCM visual sample entry shall contain a VCM Configuration Box, as defined below. This box includes a VCMDecoderConfigurationRecord, as defined in X.3.2.1.

An optional BitRateBox may be present in the VCM visual sample entry to signal bit rate information of the VCM video stream. Extension descriptors that may be inserted into the Elementary Stream Descriptor, when used in MPEG-4 systems, may also be present.

Multiple VCM visual sample entries may be used, as permitted by ISO/IEC 14496-12, to indicate sections of video that use different configurations or parameter sets.

When the sample entry name is 'vcm1' or 'vcmi', the stream to which this sample entry applies shall be a compliant VCM bitstream, as viewed by a VCM decoder operating under the configuration (including profile, tier, and level) given in the associated VCMConfigurationBox.

When the sample entry name is 'vcm1', the value of array_completeness shall be equal to 1 for the arrays of all types of parameter set NAL units present in the VCM Configuration Box.

X.4.1.1.2 Syntax

class VcmConfigurationBox extends Box('vcmC') {
	VcmDecoderConfigurationRecord() VCMConfig;
}

class VcmSampleEntry() extends VisualSampleEntry ('vcm1' or 'vcmi'){
	VcmConfigurationBox	config;
	MPEG4ExtensionDescriptorsBox ();	// optional
}

X.4.1.1.3 Semantics

Compressorname in the base class VisualSampleEntry indicates the name of the compressor used with the value "\012VCM Coding" being recommended (\012 is 10, the length of the string in bytes).

VcmDecoderConfigurationRecord is defined in ‎X.3.3.6

3	Discussion during MPEG 153rd

During MPEG 153rd, Online and gitlab based discussions happened. The discussions are here.

A summary of the questions is as follows:

- Why two elementary streams for VCM?

Comment:

The two elementary stream structures for VCM is charted from the existing HEVC carriage model in ISOBMFF and is adopted to maintain consistency with established NAL unit structured video standards.

VCM introduces multiple types of NAL units, including restoration data and temporal synthesis picture data, which require clear rules for conveying infrequently changing parameters and picture-associated information. The use of two elementary stream structures provides flexibility in signaling parameter sets while preserving a canonical stream format suitable for system-level processing.

For each VCM NAL unit type, it is necessary to clearly specify which information is conveyed as parameter sets and which is not, and whether the NAL units are permitted in the sample entry, in samples, or in both.

The handling, placement, and constraints of the VCM_EoB NAL unit need to be clearly stated in the carriage specification to ensure consistent interpretation across storage and delivery systems.

- Is the inner_codec_config placement in VCMDecoderConfigurationRecord correct?

Comment:

The placement of inner_codec_config in the VCMDecoderConfigurationRecord was discussed to ensure correct signaling and interoperability with existing ISOBMFF-based systems.

It was advised to define a table of supported inner codecs, including explicit codec indication and the corresponding sample entry point, in order to clearly identify how the inner codec configuration is interpreted.

Based on the discussion, the exploration document will be updated to better clarify the signaling and usage of inner_codec_config.

To formalize this potential work in MPEG, a tentative Working Draft (WD) was issued at MPEG #148 and is attached to this exploration document.

3.1	Potential Work in MPEG

Further work in MPEG is required to define consistent carriage and transmission mechanisms for VCM bitstreams across different delivery environments. In particular, the definition of VCM bitstream carriage should consider ISO/IEC 14496-12, ISO/IEC 14496-15, and ISO/IEC 13818-1 in order to ensure interoperable storage, multiplexing, timing, and synchronized delivery of VCM elementary streams. Additional clarification may also be required for the signaling of VCM-specific NAL units and decoder configuration information at the system level.

3.2	Next steps

As a next step, the current exploration may be refined based on the discussions held in the MPEG 153rd meeting and used as input for further system-level studies. Subject to interest within MPEG, the work may be continued for the development of the carriage and transmission of VCM bitstreams.

4	References

[1] ISO/IEC 14496-15:2024, Information technology - Coding of audio-visual objects - Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format.

[2] ISO/IEC 14496-12:2022, Information technology - Coding of audio-visual objects - Part 12: ISO base media file format.

[3] ISO/IEC 13818-1:2023, Information technology – Generic coding of moving pictures and associated audio information - Part 1: Systems.

[4] ISO/IEC JTC 1/SC 29/WG 04, “Text of ISO/IEC DIS 23888-2 Video coding for machines,” WG04N00793, Online, January 2025