ISO/IEC JTC 1/SC 29/WG 04 N0771

ISO/IEC JTC 1/SC 29/WG 04
MPEG Video Coding 
Convenorship: CN

Document type:	Output Document

Title:	Common test conditions for lenslet video coding

Status:	Approved

Date of document:	2026-03-10

Source:	ISO/IEC JTC 1/SC 29/WG 04

Expected action:	                 None

Action due date:	                 None

No. of pages:	9 (without cover page)

Email of Convenor:	yul@zju.edu.cn

Committee URL:	https://sd.iso.org/documents/ui/#!/browse/iso/iso-iec-jtc-1/iso-iec-jtc-1-sc-29/iso-iec-jtc-1-sc-29-wg-4

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 04 MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 04 N0771

January 2026, Online

| Title | Common test conditions for lenslet video coding |
| --- | --- |
| Source | WG 04, MPEG Video Coding |
| Status | Approved |
| Serial Number | 25966 |
| Authors | Lili Zhao (China Mobile, CN), Xin Jin (Tsinghua SIGS, CN) Mehrdad Teratani (Nagoya University, JP) |

# Abstract

This document provides the common test conditions (CTCs) for lenslet video coding (LVC) to be conducted after the 152nd MPEG meeting.

- Introduction

This document updates common test conditions for lenslet video coding (LVC). Input contributions should include a set of results as complete as possible, applicable to the proposal. Results should be reported using the attached Excel sheets.

- Paradigm

According to the requirements for LVC, the LVC activities are foreseen to be followed in two phases. This is the first phase, where the focus will be on the technologies, namely Codec Agnostic LVC Tools (hereafter LVC tools), compatible with Versatile Video Coding (VVC) [1]. These technologies achieve higher compression efficiency for lenslet video and ensure that at least a sub-bitstream remains decodable by the VVC decoder. The encoding and decoding process for LVC is described in Figure 1.

Figure 1. Common test procedure for lenslet video coding in phase one.

Note: The input lenslet video can be either captured by plenoptic 1.0 or 2.0 cameras, or synthetically generated from the content captured by camera array.

- Test Sequences

Nine sequences are included in the CTC, listed in Table 1. All test sequences are available in the MPEG Content repository accessible under the following URLs (MPEG password is required). Some sample central views are shown in Annex B.

Table 1. Sequence Specifications and Download Links

| Sequence name | Resolution | Frames | Frame rate | Chroma format | Bit depth | Download Links |
| --- | --- | --- | --- | --- | --- | --- |
| Origami | 2048×2048 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Origami.zip |
| Fujita2 | 2048×2048 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Fujita2.zip |
| TempleBoatGiantR32 | 6464×4852 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/TempleBoatGiantR32.zip |
| Boxer-IrishMan-Gladiator2 | 3840×2160 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Boxer-IrishMan-Gladiator2.zip |
| Boys2 | 3976×2956 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Boys2.zip |
| Matryoshka | 4040×3064 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Matryoshka.zip |
| Motherboard2 | 4036×3064 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/Motherboard2.zip |
| HandTools | 4036×3064 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/HandTools.zip |
| MiniGarden2 | 4036×3064 | 0-299 | 30 | 4:2:0 | 8 | https://content.mpeg.expert/data/CfP/LVC/Sequences/MiniGarden2.zip |

Note:  Downloaded test material should be verified with MD5 checksums (https://content.mpeg.expert/data/CfP/LVC/Sequences/Readme.pdf).

- Lenslet Video Test Model (LVTM)

Figure 2. Architecture for the LVTM v0.1

- LVTMv0.1

In September, LVTM v0.1 is released. The tag “v0.1” is available from https://git.mpeg.expert/MPEG/Video/lvc/rs/lvtm. Further software documentation and usage description is available [2].

In LVTM, the used VTM-11.0 operates in Random Access mode. Note that the configuration files of VVC and MCA per sequence are provided at https://git.mpeg.expert/MPEG/Video/lvc/rs/lvtm/-/tree/v0.1/config?ref_type=tags.

- RLC-4.0

The used RLC-4.0 is provided at https://content.mpeg.expert/data/CfP/LVC/ConfigFiles/Executable/.

- Test conditions

  - 5.1 Core Experiments (CEs)

For each CE[3], the first 64 frames of each test sequence (shown in Table 1) are coded. The coding parameter setting follows the description of section 5.2.

  - 5.2 Coding parameters

The coding parameters for each test point are available in the /config directory of the provided LVTM.

- QP Setting for the LVTM anchor

The LVTM anchor works in the QPs are listed in Table 2. The anchor results can be found in the attached datasheet “DataSheet_lvtm_v0.1.xlsm”.

Table 2. The QPs setting for the LVTM anchor.

| Sequences | QP1 | QP2 | QP3 | QP4 |
| --- | --- | --- | --- | --- |
| Origami | 32 | 37 | 40 | 44 |
| Fujita2 | 32 | 37 | 41 | 45 |
| TempleBoatGiantR32 | 37 | 41 | 45 | 49 |
| Boxer-IrishMan-Gladiator2 | 37 | 41 | 45 | 49 |
| Boys2 | 32 | 37 | 41 | 45 |
| Matryoshka | 37 | 41 | 46 | 50 |
| Motherboard2 | 37 | 40 | 45 | 49 |
| HandTools | 39 | 45 | 49 | 52 |
| MiniGarden2 | 37 | 44 | 48 | 52 |

- Camera type parameter

The parameter Type represents the camera type: Raytrix (0) and TSPC(1), where is it set according to the Table 3.

Table 3. The illustration on type parameters

| Test material filename | Type |
| --- | --- |
| Origami | 0 |
| Fujita2 | 0 |
| TempleBoatGiantR32 | 0 |
| Boxer-IrishMan-Gladiator2 | 0 |
| Boys2 | 1 |
| Matryoshka | 1 |
| Motherboard2 | 1 |
| HandTools | 1 |
| MiniGarden2 | 1 |
| Origami | 0 |
| Fujita2 | 0 |
| TempleBoatGiantR32 | 0 |

- Patch size parameter

At 152nd meeting, this optimized patch size configuration was formally adopted as the LVTM anchor [4]. In <sequence>_enc.cfg and <sequence>_dec.cfg, the “patch” is defined as Table 4.

Table 4. The patch size of the LVTM anchor [4]

| Sequences | Patch Size |
| --- | --- |
| Origami | 14 |
| Fujita2 | 14 |
| TempleBoatGiantR32 | 22 |
| Boxer-IrishMan-Gladiator2 | 24 |
| Boys2 | 48 |
| Matryoshka | 48 |
| Motherboard2 | 48 |
| HandTools | 44 |
| MiniGarden2 | 44 |

- Chroma upsampling parameter

At 153rd meeting, this optimized chroma upsampling configuration was formally adopted as the LVTM anchor. In <sequence>_enc.cfg and <sequence>_dec.cfg, the “u_filter” and “v_filter” is defined as Table 5.

Table 5. The chroma upsampling configuration of the LVTM anchor

| Sequences | U_filter | V_filter |
| --- | --- | --- |
| Origami | 1 | 1 |
| Fujita2 | 1 | 1 |
| TempleBoatGiantR32 | 4 | 16 |
| Boxer-IrishMan-Gladiator2 | 4 | 16 |
| Boys2 | 4 | 16 |
| Matryoshka | 4 | 4 |
| Motherboard2 | 16 | 4 |
| HandTools | 4 | 16 |
| MiniGarden2 | 16 | 16 |

- Edge pixel prediction vectors parameter

At 153rd meeting, this optimized edge pixel prediction vectors configuration was formally adopted as the LVTM anchor. In <sequence>_enc.cfg and <sequence>_dec.cfg, the six vectors parameter is defined as Table 6.

Table 6. The edge pixel prediction vector configuration of the LVTM anchor

| Sequence | Vector 1 | Vector 2 | Vector 3 | Vector 4 | Vector 5 | Vector 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Origami | (8, 14) | (-8, 14) | (-16, 0) | (-8, -14) | (8, -14) | (16, 0) |
| Fujita2 | (9, 16) | (-9, 16) | (-18, -1) | (-8, -15) | (9, -16) | (17, 0) |
| TempleBoatGiantR32 | (12, 21) | (-14, 24) | (-24, 0) | (-14, -21) | (13, -23) | (24, -1) |
| Boxer-IrishMan-Gladiator2 | (13, 23) | (-13, 23) | (-26, 0) | (-13, -23) | (13, -23) | (26, 0) |
| Boys2 | (-44, -75) | (44, -75) | (88, 0) | (44, 75) | (-42, 76) | (-88, -1) |
| Matryoshka | (-44, -78) | (46, -78) | (84, -2) | (46, 78) | (-44, 78) | (-88, -2) |
| Motherboard2 | (-44, -78) | (45, -78) | (88, 1) | (45, 78) | (-44, 78) | (-88, 0) |
| HandTools | (-44, -77) | (45, -76) | (88, -1) | (44, 78) | (-44, 76) | (-88, 0) |
| MiniGarden2 | (-44, -77) | (44, -75) | (88, -1) | (43, 78) | (-44, 74) | (-88, 1) |

- The resolution of the inputs for VVC

The resolution of the sequences after MCA processing has changed. In <sequence>_vvc.cfg, the “SourceWidth” and “SourceHeight” are defined as Table 7.

Table 7. The resolution of the sequences after MCA

| Sequences | Resolution |
| --- | --- |
| Origami | 1616×1408 |
| Fujita2 | 1616×1408 |
| TempleBoatGiantR32 | 4498×3900 |
| Boxer-IrishMan-Gladiator2 | 2616×1680 |
| Boys2 | 3250×2100 |
| Matryoshka | 3300×2150 |
| Motherboard2 | 3300×2150 |
| HandTools | 3300×2150 |
| MiniGarden2 | 3300×2150 |

- Reporting

Proposals will be evaluated based on bit rate, objective quality metrics, and complexity. The Excel sheet is provided together with this document to report the results. Proponents are expected to deliver a full set of results for each targeted testing condition.

  - 6.1 Bit rate reporting

The bitrate is calculated for the corresponding LVC bitstream. Then, two BD-rate [5] results, for decoded lenslet video and rendered multiview video, can be calculated using the datasheet provided at https://content.mpeg.expert/data/CfP/LVC/DataSheet/DataSheet.xlsm.

## 6.2 Distortion reporting

Lenslet-PSNR and Multiview-PSNR are measured for lenslet video and for the multiview video rendered by RLC-4.0 at each test point, respectively:

•  Lenslet-PSNR: PSNR of lenslet video in YUV 4:2:0 (hereafter Lenslet-PSNR) at each rate point

• Multiview-PSNR: PSNR averaged over the rendered multiview videos in YUV 4:2:0 (hereafter Multiview-PSNR) at each rate point

The Lenslet-PSNR and Multiview-PSNR calculation method is defined in Annex A.1. An example of rendering and calculating Multiview-PSNR for one dataset is provided in Annex A.2.

## 6.3 Complexity reporting

The encoder and decoder runtimes shall be reported relative to the unmodified test model runtimes on the same system. In addition, contributions should contain the following aspects regarding codec complexity:

- execution time with respect to anchor software

- suitability of the hardware implementation, i.e., avoid floating points, memory usage, potential for parallelization

- Cross-check criteria

When a technology is proposed in LVC, it shall pass a successful cross-check by an independent expert not affiliated with the proponent for consideration of adoption. The cross checker shall report bitrate, performance, encoding and decoding runtimes.

Coordinator:

Lili Zhao, zhaoliliyjy@chinamobile.com

References

[1]  ITU-T H.266/ISO/IEC 23090-3, “Versatile Video Coding (VVC)”.

[2] MDS25968_WG04_N00773, Software manual of LVTM, ISO/IEC JTC1/SC29/WG04, Online, January, 2026.

[3] MDS25967_WG04_N00772, Descriptions of core experiments on lenslet video coding, ISO/IEC JTC1/SC29/WG04, Online, January, 2026.

[4] m74604, [LVC] Report on LVTM performance evaluation and anchor results, ISO/IEC JTC1/SC29/WG04, Geneva, Switzerland, October, 2025.

[5]  ITU-T HSTP-VID-WPOM - Working practices using objective metrics for evaluation of video coding efficiency experiments, 2020, http://handle.itu.int/11.1002/pub/8160e8da-en.

Annex A – Objective Evaluation Metrics

The evaluation procedure is illustrated in Figure A-1. Figure A-2 provides details of the Multiview Renderer module of Figure A-1.

Figure A-1. Evaluation procedure for objective and subjective tests.

Figure A-2. Multiview Renderer module of Figure3.

A.1. Objective Test method: BD rate

Objective evaluation is conducted in lenslet video and multiview video using BD rate [3] criteria.

The multiview video needs to be rendered from the uncompressed/decompressed lenslet video using RLC-4.0.

The PSNR of a lenslet video frame/view I (in dB) is defined as

PSNR dB=10log10MAXI2MSE,

where MAXI is the maximum possible pixel value of the image I. When the pixels are represented using 8 bits per sample, MAXI is 255. For detail of MSE calculation, refer to Eq. (1) in reference [3].

PSNR is measured in YUV 4:2:0 that includes Y-PSNR, U-PSNR, and V-PSNR.

Lenslet-PSNR is the average PSNR over all frames of a lenslet video.

Multiview-PSNR is the average PSNR over all rendered view videos of a lenslet video.

A.2. Example of rendering and calculating Multiview-PSNR

25 views per test point are rendered by Reference Lenslet content Converter (RLC-4.0, available at https://content.mpeg.expert/data/CfP/LVC/ConfigFiles/Executable/) for performance evaluation. The configuration files for RLC include <sequence name>_param.cfg and <sequence name>_calib.xml, which are available at https://content.mpeg.expert/data/CfP/LVC/ConfigFiles/Configs/. Calibration files <sequence name>_calib.xml are provided for test sequences.

This is an example of the evaluation procedure for Boys2 sequence. Command line files for all the test sequences and bitrates are provided on the server (https://content.mpeg.expert/data/CfP/LVC/ConfigFiles/Rendering_Command_Lines.sh). These were used for anchor generation.

From Lenslet Video to Input of RLC

ffmpeg -s 3976x2956 -pix_fmt yuv420p -i Boys2_3976x2956_300frames_8bit_yuv420.yuv -start_number 0 Boys2/Image%03d.png -y

ffmpeg version: 7.0.1-static https://johnvansickle.com/ffmpeg/

built with gcc 8 (Debian 8.3.0-6), BT.601.

RLC Execution

RLC40 <sequence name>_Param.cfg

Boys2_Param.cfg:

method			2

viewNum			5

Calibration_xml		Boys2/Boys2_calib.xml

RawImage_Path		Boys2/Image%03d.png

Output_Path		Boys2/Frame#%03d

start_frame		0

end_frame			299

height			2956

width			3976

upsample			2

psizeInflate		2.598076211353316

maxPsize			0.3

patternSize		0.325

psizeShortcutThreshold	4

From RLC output to multiview Video

ffmpeg -start_number 0 -i Boys2/Frame#%03d/image_001.png -vf format=yuv420p -frames:v 300 Boys2_1.yuv -y

...

ffmpeg -start_number 0 -i Boys2/Frame#%03d/image_025.png -vf format=yuv420p -frames:v 300 Boys2_25.yuv -y

Multiview-PSNR calculation

For two YUV sequences from the same perspective, the Multiview-PSNR is calculated with the following instruction.

ffmpeg -s 1098x800 -pix_fmt yuv420p -i Boys2_1.yuv -s 1098x800 -pix_fmt yuv420p -i Boys2_1_qp32.yuv -lavfi psnr -f null -

...

ffmpeg -s 1098x800 -pix_fmt yuv420p -i Boys2_25.yuv -s 1098x800 -pix_fmt yuv420p -i Boys2_25_qp32.yuv -lavfi psnr -f null -

The average of the 25 views is then calculated.

Annex B – Description of test sequences

Table B-1. Sequences sample central views

| Origami | Fujita2 | TempleBoatGiantR32 |
| --- | --- | --- |
| Boxer-IrishMan-Gladiator2 | Boys2 | Matryoshka |
| Motherboard2 | HandTools | MiniGarden2 |