# ISO/IEC JTC 1/SC 29/WG 04 N0776

ISO/IEC JTC 1/SC 29/WG 04
MPEG Video Coding 
Convenorship: CN

ISO/IEC JTC 1/SC 29/WG 04
MPEG Video Coding 
Convenorship: CN

Document type:	Output Document

Title:	Manual of RLC5.0

Status:	Approved

Date of document:	2026-01-31

Source:	ISO/IEC JTC 1/SC 29/WG 04

# Expected action:	None

# Action due date:	None

No. of pages:	4 (without cover page)

Email of Convenor:	yul@zju.edu.cn

Committee URL:	https://sd.iso.org/documents/ui/#!/browse/iso/iso-iec-jtc-1/iso-iec-jtc-1-sc-29/iso-iec-jtc-1-sc-29-wg-4

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 04 MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 04 N 0776

January 2026, Online

| Title | Manual of RLC 5.0 |
| --- | --- |
| Source | WG 04, MPEG Video Coding |
| Status | Approved |
| Serial Number Editor | 25971 Jonghoon Yim, Byeungwoo Jeon, Xin Jin, Mehrdad Teratani |

# Abstract

This document is the manual for the Reference Lenslet Content Converter (RLC-5.0) [2, 3], which converts lenslet video captured by a Plenoptic Camera 2.0 into multiview video. It describes the updates introduced in RLC-5.0 compared to version 4.0, as well as instructions for its use.

# Introduction

The Reference Lenslet Converter (RLC) is designed to convert lenslet data captured by Plenoptic Camera 2.0 (focused plenoptic) cameras into multiview format. In MPEG Lenslet Video Coding (LVC) activities, two types of plenoptic cameras are commonly used: multi-focused plenoptic cameras manufactured by Raytrix [5] (e.g., R5, R8, and R32) and the Tsinghua Single-focused Plenoptic Camera (TSPC), which is internally developed by Tsinghua University. The RLC can handle data from both multi-focused and single-focused plenoptic cameras.

Figure 1.  An example of the lenslet video captured by TSPC (single-focused).

Figure 2.  An example of lenslet video captured by Raytrix R5 camera (multi-focused).

To reduce the number of color space conversion steps indicated in the top diagram in Figure 3, RLC-5.0 was developed to directly accept input data in the YCbCr 4:2:0 color space and to perform patch size estimation and patch stitching for view rendering—identical to those used in RLC-4.0—without requiring any additional color space conversion.

Figure 3.  Diagrams comparing the rendering pipeline. (Top) Rendering workflow for RLC version 4.0 and earlier. (Bottom) Proposed rendering workflow for RLC 5.0.

RLC-5.0 internally operates on the YCbCr 4:2:0 color space. However, both the input and output file formats may be either YCbCr 4:2:0–formatted .yuv files or RGB-formatted .png files. If either the input or output file format is .png, RLC performs the color space conversion internally.

Figure 4. A Diagram illustrating the possible input and output format with RLC 5.0.

# Usage of RLC 5.0 software.

RLC-5.0 is available at [1]. The package consists of three subdirectories:

- CMakeLists_txt for Linux

- CMakeLists_txt for Windows, and

- Source Code

The source code required to build the program, along with the param.cfg file used to configure rendering parameters, is located in directory ③. To build the RLC-5.0 project, a CMakeLists.txt file is required. Depending on the target operating system, select the appropriate CMakeLists.txt file from directory ① or ② and place it in the same directory as the contents of directory ③ before building the project.

When it is necessary to define new preprocessors during code editing, add #include <TypeDef.h> at the top of the corresponding .hpp or .h file. You can then define the new preprocessors in TypeDef.h and reuse them across multiple source files in different locations.

When editing the code in Microsoft Visual Studio on Windows, you can use the Shift + F12 key to conveniently navigate to all locations in the solution where a given preprocessor is referenced.

The textbox below shows the contents of the configuration file required to run the RLC.

method 2 viewNum 5 Calibration_xml ./Z_TestMaterial/Boys2_calib.xml RawImage_Path ./Boys2_3976x2956_300frames_8bit_yuv420.yuv Output_Path ./Z_TestMaterial/Boys2_Original/ start_frame 0 end_frame 299 height 2956 width 3976 upsample 2 psizeInflate 2.598076211353316 maxPsize 0.3 patternSize 0.325 psizeShortcutThreshold 4 Output_format yuv

The .cfg file format remains identical to that used in RLC-4.0, as described in [6]. In RLC-5.0, the input file format (.yuv or .png) is inferred from the file extension specified in the RawImage_Path field.

A new field, Output_format, is appended to the end of the configuration file and should be set to either yuv or png. Do not place empty lines before or after this field, as doing so will cause a runtime error.

After building the RLC software, you can launch the program using the following command:

RLC50_SOFTWARE param.cfg

Coordination:

Jonghoon Yim, yjh6522@skku.edu

Zhuo Tan, tz23@mails.tsinghua.edu.cn

Byeungwoo Jeon, bjeon@skku.edu

Xin Jin, jin.xin@sz.tsinghua.edu.cn

Mehrdad Teratani, teratani.mehrdad.v5@f.mail.nagoya-u.ac.jp

Reference

[1] (online) https://git.mpeg.expert/MPEG/Video/lvc/rs/EE/m72566-rlc-ycbcr

[2] Jonghoon Yim, et al., “[LVC] Enhancing Rendering Capability of Reference Lenslet Converter (RLC) in YCbCr Color Space without Downsampling Chroma Channel,” ISO/IEC JTC 1/SC 29/WG 04, m72566, online, March 2025.

[3] Jonghoon Yim, et al., “[LVC] Cross-check results of the Updated Reference Lenslet Converter (RLC) Software,” ISO/IEC JTC 1/SC 29/WG 04, m72566, online, January 2026.

[4] Zhuo Tan, et al., “[LVC] Release of Reference Lenslet content Converter (RLC4.0) for Lenslet Video Coding Activities,” ISO/IEC JTC 1/SC 29/WG 04, m70094, Kemer, Türkiye, November 2024.

[5] (online) https://raytrix.de

[6] (online) https://content.mpeg.expert/data/CfP/LVC/ConfigFiles/Configs/