ISO/IEC JTC 1/SC 29/AG 03 N232

ISO/IEC JTC 1/SC 29/AG 03
MPEG Liaison and Communication 
Convenorship: KR

Document type:	Output Document

Title:	White Paper on Haptics

Status:	Approved

Date of document:	2026-02-09

Source:	ISO/IEC JTC 1/SC 29/AG 03

Expected action:	None

Action due date:	None

No. of pages:	19 (with cover page)

Email of Convenor:	kyuheonkim@khu.ac.kr

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29ag3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/AG 03 MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/AG 03 N232

January 2026, Online

| Title | White Paper on Haptics |
| --- | --- |
| Source | AG 03, MPEG Liaison and communication |
| Status | Approved |
| Serial Number | 25993 |
| Authors | Philippe Guillotel (philippe.guillotel@interdigital.com) Quentin Galvane (Interdigital), Lars Nockenberg (TUM), Eric Vezzoli (Interhaptics), Ahmed Hamza (InterDigital), Akihiro Nomoto (Sony), Wang Huan (Vivo), Ahmed Farooq (TAU), Devbrat Anuragi (TAU), Roope Raisamo (Nokia) |

Summary

This contribution includes the Haptic white paper describing the MPEG Haptics standard phase 1, e.g. ISO/IEC 23090-31 specification. It includes also considerations of transport as specified in associated systems specification ISO/IEC 23090-32 and IETF RTP RFC 9695.

Version revised according to AG3 requests.

White Paper MPEG Haptics

ISO/IEC 23090-31:2025

WG7 01/06/2025

Contents

1	Introduction	3

2	MPEG-I Haptics overview	3

2.1	Standard concepts, architecture	3

2.2	Encoding Haptic Effects	6

2.2.1	Reference Software Architecture	6

2.2.2	Reference and Conformance Software Usage	7

2.2.3	Conformance	8

2.2.4	Datasets	8

2.3	Profiles and levels	9

2.4	Performances	10

3	Transport of Haptics	11

3.1	MIHS Structure	11

3.2	Storage of MPEG-I Haptics in ISOBMFF	12

3.3	Streaming MPEG-I Haptics with MPEG-DASH	13

3.4	Real-time delivery of MPEG-I Haptics	15

4	Conclusion	16

5	References	17

Introduction

Haptics refers to the science and technology of transmitting and understanding information through touch, encompassing the generation, manipulation, and perception of tactile sensations, forces, and motions. In the context of immersive media, haptics involves encoding and decoding these diverse touch-related data to enrich user experiences, allowing for the integration of physical feedback into immersive virtual environments.

The MPEG haptics coding standard aims to standardize the representation and transmission of haptic data across the vibrotactile and kinesthetic modalities as a new media, enabling interoperable and high-quality haptic interactions across various devices and applications. The first version (phase 1) of the standard has been published in January 2025. The codec architecture is flexible and generic to fulfill current and future applications and services, including encoding of haptic effects using only a vectorial (or parametric) representation for very low bit-rate applications and simple synthetic effects, as well as a high-quality encoding using an additional wavelet encoding for higher bitrates and real signals. Mixing both representations in the same stream is also possible.

This white paper provides an overview of the standard with the associated codec architecture, as well as considerations of transport using MPEG ISOBMFF file format, MPEG MP4/DASH protocol and IETF RTP protocol. An overview of the standard, partly integrated into this document, is available as an IEEE ToH publication (Guillotel, et al., 2025), and a webpage MPEG-I Part 31 — Haptics Coding.

MPEG-I Haptics overview

Standard concepts, architecture

The MPEG haptic standard (reference ISO/IEC 23090-31 (23090-31:2025)) was designed to support the encoding of both descriptive (parametric) haptic data and PCM haptic signals (time-sampled 1D signals). The descriptive input formats supported are the defined MPEG HJIF (Haptic JSON-based Interchange Format) format (the encoded MPEG format which can also be ingested) and two proprietary input formats (IVS and AHAP).

This hybrid coding is based on a hierarchical data structure, shared by the two formats defined in the specification, namely the HJIF (Haptic JSON Interchange Format) file format and the packetized MIHS (MPEG-I Haptic Streaming) format. The global architecture of the codec is depicted in Figure 1. It allows the use of different coding methods for different purposes, from low frequency effects to complex signals with high accuracy.

Figure 1: MPEG Haptics architecture (encoder and synthesizer are not normative; bitstreams and decoder are normative).

HJIF is a readable text format mostly suited for editing effects or to transmit simple haptic commands to a receiver (i.e. mobile communication). hmpg is the binary version of HJIF, suitable for media streaming and broadcasting applications (registered as a subtype under the “haptics” top-level media type in IETF RFC 9695).

.ivs, .ahap and .hjif are the three parametric input formats currently supported. A parametric format consists of a set of primitives referenced with some timing information to describe haptic effects rendered at a given time during the playback of the experience.

.wav is the input format for PCM data, as specified for waveform audio files (Waveform Audio File Format — Wikipédia). This type of haptic corresponds to a series of samples encoded with 8 to 32 bits integer at a given sampling frequency (similar to an audio file).

.ohm is an optional file describing the input information to be encoded.

The choice of the input format and encoding method is defined by the application developer. Typically, parametric might be used for low bit-rate mobile applications, while PCM would be used for continuous signals distribution.

Depending on the format, the receiver decodes the binary stream or the JSON readable format. The decoded data or original descriptive information is then interpreted by the rendering software to synthesize the original signal and convert it to device APIs, such as .wav files (series of PCM coded samples).

The renderer is not specified and is designed by the manufacturer. The device adaptation is also part of the manufacturer-specific device drivers.

Data Coding Structure

The coded representation of haptics defined in the standard is based on the hierarchical data structure and shared by the two output formats HJIF and MIHS. This hierarchical construction organizes haptic data through five layers:

- Experience: Root of the data structure, it contains the complete haptic experience through one or more haptic perceptions.

- Perception: A perception defines the haptic signal for a given modality like vibrotactile, force, temperature, etc. Several perceptions may be encoded in the same stream/file and associated with a different type of end user device.

- Channels: Perception may contain several channels. The different channels of perception encode data of the same modality, but they may be associated with different actuators located in different places on the body like a haptic suit.

- Bands: The data contained in a channel is encoded through one or several frequency bands, each defining the data in each frequency range. The standard defines four types of bands with different data representations.

- Effect: Each band contains one or several haptic events called effects. Effects are encoded through keyframes for descriptive content or binary wavelet streams for encoded PCM data.

Each level of the hierarchical structure contains metadata describing the corresponding data, providing information to configure the encoder or describing the encoded data for the rendering and synthesizer, as described below:

• Experience metadata

  - Global information (date, version)

  - A list of avatars representing the user. To allow mapping of an effect at a location on the user’s body.

  - A list of haptic perceptions.

• Perception metadata

  - Information on the modality (see Table 1) and

  - associated avatar.

  - A list of reference devices (including capabilities of

  - the target actuators).

  - A library of effects (for repeated effects).

  - A list of haptic channels.

| Modality | Perception unit | SI unit | Temporal (T)/Spatial(S) |
| --- | --- | --- | --- |
| Pressure | Pa | s | T |
| Acceleration | m/sˆ2 | s | T |
| Velocity | m/s | s | T |
| Position | m | s | T |
| Temperature | °C | s | T |
| Vibrotactile | Normalized to [−1,1] | s | T |
| Water | mˆ3 | s | T |
| Wind | m/s | s | T |
| Force | N | s | T |
| Electrotactile | Normalized to [−1,1] | s | T |
| Vibrotactile Texture | Normalized to [−1,1] | m | S |
| Stiffness | N/m | m | S |
| Friction | Normalized to [−1,1] | m | S |
| Humidity | Normalized to -1/1 | s | T |
| User-defined Temporal | Normalized to [−1,1] | s | T |
| User-defined Spatial | Normalized to [−1,1] | m | S |

Table 1 – Supported haptic modalities.

• Channel metadata

  - Information on the channel (gain, mixing weight, body part, reference device).

  - A list of haptic bands in each frequency range.

• Data

  - Band data: Contains information about the band (frequency range) and type (curve, transient, VectorialWave, WaveletWave), and the list of haptic effects data.

  - Effect data: The encoded data for each band and channels. This is the encoded data information.

Encoding Haptic Effects

The different input formats, PCM (.wav) and descriptive formats (.ivs, .ahap, .hjif) have distinct encoding processes. The elementary encoded information is an effect with additional information related to the rendering, included in the metadata. The main rendering information includes the reference device (characteristics of the targeted device) and the body mapping (localization on the user’s body). The encoding format supports the description of the effect or a reference to a pre-designed effect (through a library of effects). Effects can also be combined.

Various encoding modes can be used and combined. The choice of the most efficient method is let to the application designer, depending on the type of signals, devices and network. A reference software implementing this codec for ISO/IEC 23090-31 was made publicly available by MPEG: https://github.com/MPEGGroup/HapticReferenceSoftware, to serve as an example. It supports all the formats and tools, but only one encoding algorithm. More efficient and complex algorithms can be developed.

In addition, ISO/IEC 23090-33 provides detailed explanations on this conformance and reference software, including the architecture, the usage as well as a description of the conformance checking process and associated datasets.

Reference Software Architecture

Figure 2 illustrates the overall architecture of the implemented reference codec. The codec can process both waveform PCM signals (WAV) and descriptive haptic files such as AHAP, IVS or HJIF.

Metadata information is provided to the codec through optional OHM input files.

The encoder can output two types of formats: an interchange file format (.hjif) encoded in JSON, and a binary encoded streaming format (MIHS) that can also be stored as a binary file (.hmpg). The two formats have complementary purposes, and a lossy one-to-one conversion can be operated between them.

The decoder takes as input a binary .hmpg file (the MIHS bitstream) or a .hjif file and either outputs a .hjif file or directly synthesizes the haptic data.

The synthesizer allows to render haptic data directly from a “.hjif” input file or a decoded MIHS stream into a PCM output file if it does not contain wavelet encoded data. If the input file or stream contains wavelet data, it first goes through a wavelet decoder before synthesis.

Figure 2: Overview of the reference and conformance software with conformance points

For parametric input signals, transcoding is used towards the MPEG .hjif format (if the input is not .hjif), which is then transmitted as a .hjif file format, or binary encoded in a .hmpg file or stream.

For PCM signals (.wav), a 2 layers coding algorithm has been implemented to demonstrate both type of coding methods (see section 3). The input signal is thus first decomposed into two frequency bands (Low and High). The low frequency band is encoded using a Keyframe description and the high frequency band is encoded using a wavelet decomposition.  The coded data are then encapsulated in a .hjif file format, or binary encoded in a .hmpg file or stream.

Note: The information provided in this section is mainly focused on the reference software. It thus corresponds to one possible implementation. The reader should keep in mind that there are many other ways to do encoding and thus decoding to adapt to the targeted application.

Reference and Conformance Software Usage

The Reference and Conformance software includes 4 executables modules:

- Encoder: Encodes PCM or parametric data into HJIF files or MIHS streams. It takes as input either a .ahap file (apple proprietary format), a .ivs file (immersion proprietary format), a .hjif file (the considered MPEG format), a .wav file (containing PCM data) or a “.ohm” metadata file with companion files (.ahap, .ivs, or .wav files)

- Decoder: Decodes a .hmpg file (MIHS bitstream) or a .hjif file and either outputs a .hjif file or feed the decoded data to the Synthesizer.

- Synthesizer: Synthetizes haptic data into PCM data. It takes as input a .hjif file or a .hmpg file containing the MIHS bitstream. If the file contains encoded data (.hmpg file or .hjif file with wavelet bands), it calls the Decoder to decode the data. The Synthesizer then renders the decoded haptic data into a PCM output file.

- Conformance: Validates that input files are conformant to ISO/IEC 23090-31. Conformance checks are detailed in section 2.2.3. It can be used to test the conformance of a HJIF file, to test the conformance of a MIHS stream or to check the conformance and compare two input files (HJIF or MIHS).

Detailed usage of these executables is provided in the readme file of the reference software as well as in ISO/IEC 23090-33.

Conformance

ISO/IEC 23090-33 specifies tests for verifying the conformance of bitstreams as well as decoders. Two levels of conformance have been defined. These tests were implemented through four types of conformance checks: HJIF checks, MIHS checks, compatibility checks, and decoder tests.

The conformance of an HJIF input document is verified in two steps: Step1. Schema verification using the JSON schemas provided with ISO/IEC 23090-31. The list of constraints checked by the schema verifier is detailed in Annex D of ISO/IEC 23090-33. Step 2. Semantic verification checking a set of rules extracted from the constraints specified in ISO/IEC 23090-31. The complete list of rules is detailed in Annex E of ISO/IEC 23090-33.

The conformance of a MIHS input stream is verified in a single step: The haptics MIHS bitstream verifier decodes each MIHS unit and packets and checks whether the syntax and conditions of the bitstream are valid according to the specification ISO/IEC 23090-31. The complete list of tested constraints is detailed in Annex F of ISO/IEC 23090-33. The verifier outputs a conformance report.

The conformance tools implement compatibility checks warning the user that some properties may be affected by the binary encoding. The complete list of properties with their relative limitations is provided in ISO/IEC 23090-33. The output of the compatibility verifier is a report with the list of all the constraints to be addressed to perform a binary encoding of the HJIF file.

Decoder conformance is tested through the following 4 steps: 1) A HJIF reference file is encoded into a MIHS stream. 2) The MIHS stream is fed to the decoder. 3) The output of the decoder is provided to a HJIF comparator along with the HJIF reference file corresponding to the MIHS reference stream. 4) The Conformance comparator compares the items in the two HJIF files. If the files are different, the Conformance comparator outputs an error.

Datasets

To evaluate the reference and conformance software, MPEG built two separate datasets:

- Reference test content: This dataset is used to evaluate the encoder. It is comprised of three subsets for training, evaluation and test, each containing test files for short vibrotactile signals, long vibrotactile signals, and kinesthetic signals. This dataset contains WAV files, IVS files, and AHAP files, along with their corresponding OHM metadata files. The reference software is provided with a python script used to evaluate the reference software based on this dataset. The script encodes (at different bitrates), decodes, and synthetizes every reference file and outputs a detailed report of the performance of the evaluated codec.

- Conformance dataset: This dataset is used to ensure that the codec is able to detect non-conformance files. It is composed of a set of non-conformant files violating the conformance constraints. It contains files for testing the HJIF schema verifier, the HJIF semantic verifier, the MIHS verifier and the compatibility verifier. A set of conformant files extracted from the Reference test content dataset are also used for testing the conformance of the decoder as illustrated in Figure 6. A dedicated python script testing all non-conformant files with expected output is available with reference and conformance software. It ensures that the decoder identifies all conformance issues defined in ISO/IEC 23090-33.

The complete list of files constituting these two datasets is provided in the annexes of ISO/IEC 23090-33 (23090-33:2025).

Profiles and levels

In order to simplify implementations (hardware and software), each profile restricts the number of tools used, and the associated levels define some maximal values regarding some of the encoding features (cf. Table 2).

The Simple Parametric Profile is developed for the existing set of haptics interface and peripherals existing on the market, such as mobile phones or game controllers. It restricts the encoding to parametric haptic descriptions, using Transient bands, Curve bands, and Vectorial wave (no wavelet encoding) encoding tools.

The Main profile targets advanced platforms such as simulators or motion platforms. It allows scalable applications and very high fidelity encoding. It includes all tools.

Levels are associated to the profiles to limit decoders’ complexity, by restricting the number of bands, channels and perceptions.

| Profile | Level number | Number of channels | Number of bands | Perceptions | Band types | Effect types | Timescale | MIHS Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Simple Parametric | 1 | 1-127 | 1-7 | Force Vibrotactile Stiffness Vibrotactile Texture | Transient Curve Vectorial | no composite | 1000 | First MIHS data unit contains all spatial effects. |
|  | 2 | 1-65535 | 1-63 | All | Transient Curve Vectorial | no composite | 1000 | First MIHS data unit contains all spatial effects. |
| Main | 1 | 1-127 | 1-7 | All | All | All | 1000 for transient, curve and vectorial Any for wavelet bands | No restiction |
|  | 2 | 1-65535 | 1-63 | All | All | All | 1000 for transient, curve and vectorial Any for wavelet bands | No restiction |

Table 2: Profiles and levels definitions

Performances

Both objective and subjective evaluations of the reference implementation of the MPEG Haptics codec (section 2.2.1) have been performed. The results reported here have been done with the MPEG software reference model version 3 (CRM3). It only provides a rough indication of the performance, since it was not optimized for a specific application or platform.

As parametric is not compressed, only the distribution of the original data rate is provided in Figure 7 left. From this representative set of streams, it can be concluded that most content (from a set of 43 relevant files) can be encoded with a bit-rate lower than 8kbps.

For PCM input data, depending on the encoding scheme used and the number of layers, different bitrate / distortion performances are achieved (cf. Figure 7 right). Three versions of the codec have been tested, i) C2V a version using only vectorial coding, ii) C2W using only wavelet coding and iii) C2VWR using both vectorial and wavelet encoding. As expected, the wavelet encoding (C2W) provides a more accurate reconstruction of the signal, especially at high bitrates. On the other hand, vectorial coding (C2V) allows very low bitrates with reasonable quality and reduced computational complexity. The 2-layer approach (C2VWR) mixing Vectorial coding for the lower layer (LF) and wavelet coding for the higher layer (HF), provides a compromise between the advantages of both solutions, since errors in the low-frequency (LF) coding are compensated by the wavelet coding (HF) when the target bit-rate provides room for higher reconstruction fidelity. An advantage of this scalable approach is that one could just decode the LF band to get a low frequency approximation of the signal or the two layers for a more complete signal reconstruction.

Figure 7: MPEG Haptics objective performances (left: parametric bit-rates distribution, right: PCM PSNR).

An additional subjective evaluation, with 35 subjects and 3 different labs, was performed, using the MUSHRA methodology (cf. Figure 8). The subjective results mainly confirm the objective results. C2W is performing better for most bitrates (except at low bitrates due to a less accurate rate control). C2V generally performs lower than the other methods. C2VWR is overall a good compromise between the other two systems (taking the best of each) except for 2 kbps, where it achieved the best score due to higher total bitrate. Since the perceptual quality is high for all systems at medium and high bitrates, the differences in scores are lower.

Figure 8: MPEG Haptics subjective performances for three bitrates (HR: Hidden Reference, V: Vectorial (C2V), W: Wavelet (C2W), VWR: Vectorial + Wavelet + Residual (C2VWR)).

Transport of Haptics

MIHS Structure

The MIHS (MPEG-I Haptic Stream) distribution format is a packetized binary stream stored as a binary file (.hmpg) or directly streamed using DASH or RTP protocols. The format is designed to be easily encapsulated by any network protocol. The packets can be of variable duration and include two levels of packetization as shown in Figure 12. The two levels of packetization are:

- MIHS unit that covers a duration of time and includes zero or more MIHS packets.

- MIHS packet that includes metadata or haptic effect data.

Figure 13: MIHS packetization.

Each MIHS unit covers a duration of haptic presentation time defined by a timestamp and a duration. Consecutive MIHS units are either temporally aligned, sharing the same timestamp and duration, or in sequence, i.e., a MIHS Unit starts at the end of the previous MIHS unit.

An MIHS unit comprises the unit header and MIHS packets,

- Initialization which contains one MIHS initialization timing packet and may include one or more metadata MIHS packets. This unit starts a new time anchor in haptics presentation time by setting the timestamp and may also set or change the timescale. The duration of an initialization unit is zero.

- Temporal which contains one or more MIHS packets with temporal haptic effects. The duration of a temporal unit is a positive number.

- Spatial which contains one or more MIHS packets with spatial haptic effects. The duration of a spatial unit is zero.

- Silent which indicates that there is no effect that starts during this time interval. This unit does not include any MIHS packets other than a timing packet. The duration of a silent unit is a positive number.

Storage of MPEG-I Haptics in ISOBMFF

In addition to the MPEG Haptics coding standard, MPEG has also developed the ISO/IEC 23090-32 (Carriage of Haptics Data) specification (23090-32:2025) to enable the storage and delivery of haptics media conformant to ISO/IEC 23090-31. The specification defines how coded MPEG-I Haptics content may be stored in an ISO base media file format (ISOBMFF) (14496-12:2022) container, providing the ability to multiplex haptics media with other types of media such as audio and video.

The specification introduces two methods for storing MPEG Haptics bitstreams in ISOBMFF: single-track storage and multi-track storage. Single-track storage enables direct encapsulation of an MPEG Haptics MIHS bitstream in an ISOBMFF container, albeit with limited flexibility when it comes selective extraction from the haptics experience’s components. Multi-track storage enables encapsulating some components of the haptics experience, more specifically a set of bands associated with a certain channel of a perception in the MPEG Haptics bitstream in a separate ISOBMFF track. This enables a player to selectively extract and load data for one or more channels for a set of perceptions within the haptics experience without having to de-multiplexing the bitstream. The ISO/IEC 23090-32 specification defines two brands that signal to the player that an ISOBMFF container includes MPEG Haptics media and which storage method (e.g., single track vs. multi-track) was used to encapsulate it. The two main brands defined in the specification are:

- ‘mih1’: which signals encapsulation using only an MIHS track that carries all the MIHS units of the haptics experience, including all perceptions and their respective channels.

- ‘mhb1’: which signals encapsulation using one MIHS track that reference one more MIHS band tracks that carry MIHS units for one or more bands of one of the channels from the MIHS bitstream.

Regardless of the carriage mode used, the MIHS track is required to include a configuration structure (MIHSConfigurationBox), holds the metadata and effect library packets needed to initialize decoding of the carried MIHS samples (reflecting one or more MIHS initialization units), and a haptic experience description structure (HapticExperienceDescriptionBox), which contains the experience/perception/channel/band description hierarchy.

Timing packets from the MIHS bitstream may be omitted because timing is handled by standard ISOBMFF timing boxes.

Streaming MPEG-I Haptics with MPEG-DASH

ISO/IEC 23090-32 also enables HTTP adaptive streaming of MPEG-I Haptics using MPEG-DASH (23009-1:2022) by defining how haptic tracks stored in ISOBMFF are segmented and signaled in DASH manifests, allowing DASH clients to discover, select, and adapt haptic representations in coordination with audio and video streams.

The specification defines a set of normative MPEG-DASH MPD signaling rules for haptics, centered around how to identify haptics tracks/representations, how to signal random access and switching constraints, how to associate haptics with other media, and (for multi-track experiences) how you express dependencies and experience/band structure.

To signal a MIHS (haptics) track in the MPD, the presence of @codecs="mih1" and @mimeType="haptics/mp4" on either the AdaptationSet or the Representation is required. For basic MIHS track delivery, it is required that the first sample of each segment (live profile) or subsegment (on-demand profile) is a sync sample and that @startWithSAP="1" for live and @subsegmentstartWithSAP="1" for on-demand. For random access and buffering efficiency, it is recommended that haptics segment/subsegment durations be approximately the same as audio/video segment durations.

When multiple MIHS tracks are offered, the MPD rules depend on the intended selection behavior. For alternative tracks that should not be rendered together, the MPD should use one Representation in one AdaptationSet per MIHS track and set AdaptationSet@group to the same value across the alternatives.

For multi-track haptics experiences (i.e., a MIHS track plus one or more band tracks), the MPD represents each track (including the main track) as its own AdaptationSet, with the main referred to as the Haptics Experience Adaptation Set and the others as Haptic Bands Adaptation Sets. The Experience Adaptation Set uses @codecs="mih1", while each Bands Adaptation Set uses @codecs="mhb1", and the @mimeType attribute for all AdaptationSets of a haptics experience is set to "haptics/mp4". Each band Representation then signals dependency on the experience Representation using @dependencyId set to the experience Representation’s @id.

To describe the experience and band structure at the MPD level, the specification defines a HapticsExperience DASH descriptor carried as an EssentialProperty with a common @id value across all AdaptationSets that belong to the same haptics experience. Within that descriptor, the hapticsExperience element is only allowed when the enclosing AdaptationSet has @codecs="mih1" and must not conflict with the in-file HapticExperienceDescriptionBox, while hapticsBandsDescription is only allowed when @codecs="mhb1" and must not conflict with the in-file HapticBandConfigurationBox. This descriptor enables a DASH player to identify the characteristics of the haptics signals carried by each Adaptation Set and/or Representation associated with the haptics experience in the MPD and to make selective adaptation decisions accordingly.

A basic, simplified MPD example for the multi-track haptics experience case (one Haptics Experience Adaptation Set plus two Haptics Bands Adaptation Sets) is shown below.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns="urn:mpeg:dash:schema:mpd:2011" xmlns:haptics="urn:mpeg:mpegI:haptics:2025" type="static" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011" minBufferTime="PT1.5S" mediaPresentationDuration="PT60S"> <Period> <!-- Haptics Experience (base track) --> <AdaptationSet id="he1" mimeType="haptic/mp4" codecs="mih1"> <BaseURL>haptics/</BaseURL> <SegmentTemplate timescale="90000" media="$id$/$Time$.mp4s"> <SegmentTimeline> <S t="0" d="180180" r="29"/> </SegmentTimeline> </SegmentTemplate> <Representation id="h1" bandwidth="100"> <EssentialProperty schemeIdUri="urn:mpeg:mpegI:haptics:2025"> <haptics:hapticsExperience edition="1" profile="xx" level="1"> <haptics:hapticsPerception id="1" type="1"/> <haptics:hapticsChannel id="1" perceptionId="1"/> <haptics:hapticsChannel id="2" perceptionId="1"/> </haptics:hapticsExperience> </EssentialProperty> </Representation> </AdaptationSet> <!-- Haptics Bands for Channel 1 (band track) --> <AdaptationSet id="hc1" mimeType="haptic/mp4" codecs="mhb1"> <BaseURL>haptics/</BaseURL> <SegmentTemplate timescale="90000" media="$id$/$Time$.mp4s"> <SegmentTimeline> <S t="0" d="180180" r="29"/> </SegmentTimeline> </SegmentTemplate> <Representation id="hbc1" bandwidth="500" dependencyId="h1"> <EssentialProperty schemeIdUri="urn:mpeg:mpegI:haptics:2025"> <haptics:hapticsBandsDescription perceptionId="1" channelId="1"/> </EssentialProperty> </Representation> </AdaptationSet> <!-- Haptics Bands for Channel 2 (band track) --> <AdaptationSet id="hc2" mimeType="haptic/mp4" codecs="mhb1"> <BaseURL>haptics/</BaseURL> <SegmentTemplate timescale="90000" media="$id$/$Time$.mp4s"> <SegmentTimeline> <S t="0" d="180180" r="29"/> </SegmentTimeline> </SegmentTemplate> <Representation id="hbc2" bandwidth="500" dependencyId="h1"> <EssentialProperty schemeIdUri="urn:mpeg:mpegI:haptics:2025"> <haptics:hapticsBandsDescription perceptionId="1" channelId="2"/> </EssentialProperty> </Representation> </AdaptationSet> </Period> </MPD>

This brings haptics into established MP4/DASH delivery workflows used for large-scale streaming in both on-demand and live streaming services.

Real-time delivery of MPEG-I Haptics

In addition to the support for on-demand and live streaming of MPEG Haptics media, as defined in ISO/IEC 23090-32, a Real-time Transport Protocol (RTP) payload format for MPEG Haptics for real-time delivery and communication use cases is currently under development within the AVTCORE group of the Internet Engineering Task Force (IETF) in (IETF Haptics Payload)  as a Proposed Standard  and is expected to be published early 2026. In addition, the IETF RFC 9695 (RFC 9695) registers and documents the 'haptics' top-level media type, under which subtypes for representation formats for haptics may be registered.

The payload format specifies how MIHS units are mapped into RTP packets, including rules for aggregation of multiple units into a single packet and fragmentation of large units across multiple packets. These mechanisms allow the payload format to adapt to varying network conditions and MTU sizes. The proposed Internet-Draft introduces a payload header structure that enables the receiver to identify MIHS boundaries and correctly reassemble fragmented units. It also defines how codec configuration information is conveyed, ensuring that receivers can correctly initialize the haptics decoder before or during a session.

To enable interoperable session setup, the draft also defines SDP (Session Description Protocol) signaling for MPEG haptics. This includes registration and use of the top-level haptics media type, along with media subtype and format parameters that describe the MPEG-I haptics codec configuration, clock rate, and other transport-relevant attributes. This signaling allows MPEG haptics streams to be negotiated alongside audio and video in RTP-based systems such as WebRTC.

Conclusion

The MPEG standard is flexible to accommodate main haptic modalities and formats. For instance, one may encode haptic effects using only a vectorial representation for very low bit-rate applications and simple synthetic effects, while a higher quality encoding can be achieved using an additional wavelet encoding band (or even using only wavelet encoding), for higher bitrates and real signal recordings. In addition, the two defined profiles allow for the deployment of simple systems using only parametric haptics or more advanced systems for High-fidelity haptics. The choice is let to the application standard or system.

The format also includes signaling for the localization of the haptic signal on the user and for the reference device used during the design process. This is particularly useful for applications to adapt the designed experience to the available user setup configuration.

Two different output formats are defined, a readable interchange file format based on JSON (HJIF) particularly suitable for creation and editing of haptic experiences, and a binary version format ready for storage or streaming (MIHS).

From the evaluations performed during the development process, the following bit-rate ranges per channel can be recommended depending on the expected quality level:

• 2-4 kbps: perceptible distortion

• 8-16 kbps: some distortions, but not annoying

• 32-64 kbps: no perceptible distortion

• ≥ 64 kbps: perceptually lossless.

Transport of this haptic data has been specified into global distribution systems. The MPEG Systems Working Group (ISO/IEC JTC1/SC 29/WG3) has defined an ISOBMFF (ISO Base Media File Format) binding for the MIHS distribution format and DASH signaling for supporting the streaming of Haptics along other multimedia content in a dedicated standard for carriage of haptics data (ISO/IEC 23090-32). In addition, IETF defined haptics as a new top level media type, at the same level as audio, video, image, etc., in RFC 9695 with the two MPEG haptic formats ‘hjif’ and ‘hmpg’ registered as sub-types under the ‘haptics’ top-level media type. An internet-draft to specify transport of MPEG Haptics data into RTP packets is also being developed within the AVTCORE workgroup and is expected to be finalized in 2026. These will facilitate adoption and deployment of Haptics for multimedia and communication applications.

While this first version of the MPEG standard (considering the coding of time-dependent haptic signals) is now ready for deployment, a new phase has been started in MPEG to extend it to Haptic interactions, avatars and 3D objects typically needed in Extended Reality (XR) applications and the forthcoming metaverse. Phase 2 will also extend the coverage to new haptic modalities, such as thermal and electrotactile, as well as provide more advanced methods for haptic mediation and combination of different haptic signals.

References

(Guillotel, et al., 2025) Guillotel, P., Muthusamy, Y., Galvane, Q., Vezzoli, E., Nockenberg, L., Sodagar, I., . . . Gueorguiev, D. (2025). Adding Touch to Immersive Media: An Overview of the MPEG Haptics Coding Standard. IEEE Transactions on Haptics, 1-15.

(23090-:2025) - Information technology — Coded representation of immersive media Part 31: Haptics coding. ISO/IEC 23090-31:2025 - Information technology — Coded representation of immersive media — Part 31: Haptics coding

(23090-33:2025) - Information technology — Coded representation of immersive media Part 33: Conformance and reference software for haptics coding. ISO/IEC 23090-33:2025 - Information technology — Coded representation of immersive media — Part 33: Conformance and reference software for haptics coding

(23090-32:2025) - Information technology — Coded representation of immersive media — Part 32: Carriage of haptics data

(14496-12:2022) - Information technology — Coding of audio-visual objects — Part 12: ISO base media file format

(23009-1:2022) - Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats

(IETF Haptics Payload) – Yang, H.S. and de Foy, X., “RTP Payload Format for Haptics”, Online: https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-haptics/

(IETF RFC 9695) – Muthusamy, Y. K. and Ullrich, C., “The 'haptics' Top-Level Media Type”, RFC 9695, March 2025

IVS registered as a subtype under the “haptics” top-level media type in IETF RFC 9695 by IANA. RFC 9695: The 'haptics' Top-Level Media Type.

AHAP (Apple Haptic Audio Pattern) is Apple format for Haptics. Core Haptics | Apple Developer Documentation.

RFC 9695: The 'haptics' Top-Level Media Type