# a                                        ISO/IEC JTC 1/SC 29/WG 04 N789

ISO/IEC JTC 1/SC 29/WG 04
MPEG Video Coding 
Convenorship: CN

Document type:	Output Document

Title:	Common test conditions for video coding for machines

Status:	Approved

Date of document:	2026-03-06

Source:	ISO/IEC JTC 1/SC 29/WG 04

Expected action:	None

Action due date:	None

No. of pages:	18 (without cover page)

Email of Convenor:	yul@zju.edu.cn

Committee URL:	https://sd.iso.org/documents/ui/#!/browse/iso/iso-iec-jtc-1/iso-iec-jtc-1-sc-29/iso-iec-jtc-1-sc-29-wg-4

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 04 MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 04 N789

January 2026, Online

| Title | Common test conditions for video coding for machines |
| --- | --- |
| Source | WG 04, MPEG Video Coding |
| Status | Approved |
| Serial Number | 26007 |
| Authors | Shan Liu, Chris Rosewarne, Olgierd Stankiewicz |

# Abstract

This document describes common test conditions (CTC), reference software and configurations, anchor generation, evaluation framework, reporting and other information to be used for experimenting and evaluating technologies for video coding for machines (VCM). These common test conditions are recommended for use in technical contributions to the 154th and following MPEG meetings, as applicable.

- Introduction

Common test conditions (CTC) are desirable for conducting experiments in a well-defined environment followed by the comparison of the outcome of experiments. This document defines such common test conditions for experimenting and evaluating technologies for video coding for machines (VCM).

A set of test conditions (or configurations) are defined to reflect random-access, low-delay and intra-only settings for both inner and end-to-end codecs, as follows:

- Random access for inner (RA_inner)

- Random access for end-to-end (RA_e2e)

- Low delay for inner (LD_inner)

- Low delay for end-to-end (LD_e2e)

- All intra for inner (AI_inner)

- All intra for end-to-end (AI_e2e)

The reporting of results for each configuration is either mandatory or optional, dependent on the dataset, the machine task, etc. Detailed information about mandatory and optional configurations and results are provided in Section 2.

Evaluation scripts can be found at https://git.mpeg.expert/MPEG/Video/VCM/vcm-ctc. Access right to the repository is granted to registered experts in the MPEG WG 4.

The following sections define test sequences, anchors, test and training conditions, evaluation metrics and reporting. Anyone bringing input contributions for WG 4 VCM AHG to discuss shall provide a set of results that is as complete as possible and uses the CTC that apply to the proposal.

- Test datasets

Three video datasets, i.e., SFU-HW-objects-v3.2 dataset (referred to as SFU-HW in this document), Tencent Video dataset (referred as TVD in this document) and PandaSet dataset (referred as Panda), are included in the CTC. Reporting results on SFU-HW and TVD, except class O of the SFU-HW dataset, for RA_inner, RA_e2e, LD_inner, LD_e2e, AI_inner, AI_e2e configurations are mandatory. Reporting results class O of the SFU-HW dataset and on PandaSet dataset for all configurations are optional. In addition, three image datasets are included in the CTC and can be tested using AI_inner configuration. Reporting results on the image datasets is optional.

The relevant files and data corresponding to the test materials have been uploaded to the MPEG Expert area (https://content.mpeg.expert/data/), with access instructions as follows.

- Log in using username = mpeg

- The password is the same as for the MPEG Document Management System

- Subdirectories: MPEG-AI/VCM

- VCM path: https://content.mpeg.expert/data/MPEG-AI/VCM/

## SFU-HW Dataset

The SFU-HW-objects-v3.2 dataset (referred to as SFU-HW in this document) is a video dataset consisting of 14 sequences which are known from previous standardization efforts in JCT-VC and JVET. The sequences can be found on ftp://hevc@mpeg.tnt.uni-hannover.de. The annotations are available at https://data.mendeley.com/datasets/367kty6nw7/1. Specific license information is provided with each test sequence.

Table 1 provides the detailed information of each sequence from the SFU-HW dataset to be used in this CTC. Only a subset of each video sequence needs to be encoded. The first n frames of each sequence are skipped. The exact number n frames to skip and the number of frames to be coded can be found in Table 1. The n value in the table can be directly applied to VTM encoder [1] by using the FrameSkip parameter. The MD5 checksums for the 14 sequences in SFU-HW dataset are provided in Table 2.

Table 1. Test sequences in SFU-HW dataset

| Class | Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | RA/LD/AI _inner/e2e |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 8 | 117 | 33 | M |
| B | ParkScene | 240 | 24 | 8 | 207 | 33 | M |
| B | BasketballDrive | 500 | 50 | 8 | 403 | 97 | M |
| B | BQTerrace | 600 | 60 | 8 | 471 | 129 | M |
| C | RaceHorsesC | 300 | 30 | 8 | 235 | 65 | M |
| C | BQMall | 600 | 60 | 8 | 471 | 129 | M |
| C | PartyScene | 500 | 50 | 8 | 403 | 97 | M |
| C | BasketballDrill | 500 | 50 | 8 | 403 | 97 | M |
| D | RaceHorsesD | 300 | 30 | 8 | 235 | 65 | M |
| D | BQSquare | 600 | 60 | 8 | 471 | 129 | M |
| D | BlowingBubbles | 500 | 50 | 8 | 403 | 97 | M |
| D | BasketballPass | 500 | 50 | 8 | 403 | 97 | M |
| O | Kimono | 240 | 24 | 8 | 207 | 33 | O |
| O | Cactus | 500 | 50 | 8 | 403 | 97 | O |

Note: M – mandatory; O – optional.

Table 2. MD5 checksum of the test sequences in SFU-HW dataset

| Class | Sequence name | MD5 checksum |
| --- | --- | --- |
| A | Traffic | 4f03a86b03b47fc821acffb8baea56f6 |
| B | ParkScene | b7ada0912d693304165254177d08343d |
| B | BasketballDrive | d38951ad478b34cf988d55f9f1bf60ee |
| B | BQTerrace | efde9ce4197dd0b3e777ad32b24959cc |
| C | RaceHorsesC | 0a351df99f22d837bc528bd4901c6968 |
| C | BQMall | f889efea02b0c9a7d174b0f7a99cb51b |
| C | PartyScene | 4766c455665b6d228a6390e3d3ff2647 |
| C | BasketballDrill | bd215136fed04067d82c10b2e49b2c7c |
| D | RaceHorsesD | 290a63e86213abc4459fce1dbd39edbe |
| D | BQSquare | 713ef64958345859b9bae986c3a3f763 |
| D | BlowingBubbles | 50a520722f0e906b7884b6b9fea48699 |
| D | BasketballPass | bfd9abbdc677790130dc4023b4e409f0 |
| O | Kimono | 4a83005bc719012ac148dd3898e5e4ed |
| O | Cactus | 3fddb71486f209f1eb8020a0880ddf82 |

Overall BD-rate performance of SFU-HW video content is calculated and reported on a per-dataset basis and the reporting template contains assigned fields and auto-functions to facilitate this purpose. Note that class O (Cactus and Kimono) is not included in calculating the overall results.

## Tencent Video Dataset (TVD)

The Tencent Video Dataset (TVD) is a video dataset consisting of 7 sequences in 1920x1080 resolution used for object tracking. Detailed information about these sequences is provided in Table 3. The dataset with corresponding annotations is available at [2] https://multimedia.tencent.com/resources/tvd.

Table 3. Test sequences in TVD

| Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | RA/LD/AI _inner/e2e |
| --- | --- | --- | --- | --- | --- | --- |
| TVD-01-1 | 3000 | 50 | 8 | 1500 | 500 | M |
| TVD-01-2 | 3000 | 50 | 8 | 2000 | 500 | M |
| TVD-01-3 | 3000 | 50 | 8 | 2500 | 500 | M |
| TVD-02-1 | 636 | 50 | 10 | 0 | 636 | M |
| TVD-03-1 | 2334 | 50 | 10 | 0 | 500 | M |
| TVD-03-2 | 2334 | 50 | 10 | 500 | 500 | M |
| TVD-03-3 | 2334 | 50 | 10 | 1000 | 500 | M |

Table 4. MD5 checksum of the test sequences in TVD

| Sequence name | MD5 checksum |
| --- | --- |
| TVD-01 | 1dddac6c82e5c8e59f06d283458e2db7 |
| TVD-02 | aad63df298fa6401c16a36ede61e9798 |
| TVD-03 | 9aa26e98ac34e7da9712c3ed4677da4b |

As the original sequences are available in .mp4 format, they shall be converted to YUV420p using FFmpeg [3]:

ffmpeg -i {input.mp4} {output.yuv}

The MD5 checksums for TVD sequences in YUV420 format are provided in Table 4.

## PandaSet Dataset (optional)

The PandaSet dataset is a video dataset consisting of 106 sequences in 1920x1080 resolution used for semantic segmentation.  Among the total 106 sequences, 73 are selected to be included in the CTC to provide optional test results. They are grouped into six classes, as shown in Table 5.

Table 5. Test sequences in Panda dataset

| Class | Sequence ID | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | RA/LD/AI _inner/e2e |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 57, 58, 69, 70, 72, 73, 77 | 80 | 10 | 8 | 0 | 0 | O |
| B1 | 3, 11, 16, 17, 21, 23, 27, 29, 30, 33, 35, 37, 39, 43, 53, 56, 97 | 80 | 10 | 8 | 0 | 0 | O |
| C1 | 88, 89, 90, 95, 109, 112, 113, 115, 117, 119, 122, 124 | 80 | 10 | 8 | 0 | 0 | O |
| A2 | 64, 65, 66, 67, 71, 78, 149 | 80 | 10 | 8 | 0 | 0 | O |
| B2 | 1, 5, 13, 15, 19, 24, 28, 32, 34, 38, 40, 41, 42, 44, 46, 52, 54, 139 | 80 | 10 | 8 | 0 | 0 | O |
| C2 | 80, 84, 94, 101, 102, 103, 105, 106, 110, 116, 123, 158 | 80 | 10 | 8 | 0 | 0 | O |

More details about PandaSet dataset are available at https://pandaset.org/. The dataset with annotation can be downloaded from https://content.mpeg.expert/data/MPEG-AI/VCM/Pandaset/.

The MD5 checksums of the Panda sequences in YUV420 format are provided in Table 6.

Table 6. MD5 checksum of the test sequences in Panda dataset

| Class | Sequence name | MD5 checksum |
| --- | --- | --- |
| A1 | 57 | 9130490082689713a69e575a80fb4ad6 |
| A1 | 58 | 1440c40a45549a2f445e7f77d68ade95 |
| A1 | 69 | 3910301cc88baf0784a857ab7311dfc7 |
| A1 | 70 | f2010059d5c7cc6994f6fe57f6c0b3b8 |
| A1 | 72 | 680260f1c9b02008e274ce95f9b695bb |
| A1 | 73 | 17901de39c20e19997428387797a9161 |
| A1 | 77 | 15fb8f97f221ad0d09ef712ba98f3b32 |
| B1 | 3 | b9adefa4b91950dff73d68e3084f0087 |
| B1 | 11 | 30d36645cd4f91392cfa6b2af591efe5 |
| B1 | 16 | 59deb96e6d0384d10afaa5ce10a90e26 |
| B1 | 17 | 206ad92bdfaf0cfa78825500b239908f |
| B1 | 21 | ec53bde219de6dc4561829654365cc42 |
| B1 | 23 | 0212ceac5619c8fd5ea83d39072fcd2f |
| B1 | 27 | 63b21eb8e620e19e61397321a7cc8023 |
| B1 | 29 | b97e9951a61340441d1ee7e69d420b9e |
| B1 | 30 | f0df4c2f6bb9101a34c74a738ad6d2bb |
| B1 | 33 | 5854fb8637c1712376722531ba3e44ee |
| B1 | 35 | 1ffc71a4adbad0c6404f9feb07703943 |
| B1 | 37 | 8dccf94cab1f9a1540e8a58dd1b6a3b9 |
| B1 | 39 | 2c17b499c276424ff661531e4f624208 |
| B1 | 43 | 622b88fc748ad35775c9dd11b912e1de |
| B1 | 53 | ec850c3929eb14e93819211a1a4bec25 |
| B1 | 56 | 4f90b8bcb8201e9c15d3913c078f125b |
| B1 | 97 | 3d2e46d5522ea4f15ff630435177a30d |
| C1 | 88 | 64ac63da2e4ad4e2138385ffe79eceac |
| C1 | 89 | 2c157de05be51e95a534b711f80f6ef5 |
| C1 | 90 | d10c40118ac9cb5aab853788f8bfefef |
| C1 | 95 | 534cb091fa6355c02239bbea7e055851 |
| C1 | 109 | 51bbec10cdbf61e9ae0136d4c3d76103 |
| C1 | 112 | 5f267a5086c69cd8580f5cccb7475ca4 |
| C1 | 113 | a09b649c860fc3f905e1ecead478af27 |
| C1 | 115 | 3fb89c43967c5a1faf1952e2b88f734c |
| C1 | 117 | 0adf67cd7d0c77961f57aae4dd528856 |
| C1 | 119 | a1ba6f0ef28ab8ad80e05676be07224b |
| C1 | 122 | b2b614dead35d3fbfa50b97530deee1a |
| C1 | 124 | 3ae37079ec6e32deb138dae6d3ad7832 |
| A2 | 64 | 840e927178cf14eff1a6b1a4eaf3a466 |
| A2 | 65 | 12075d8f069d017d7dd4c4358d8ef0fd |
| A2 | 66 | c582ba5619715acc9aead8487ee13ba7 |
| A2 | 67 | 404f8281d8b41a9eb454e54bf0491c51 |
| A2 | 71 | 87a48d966c8ba0a4e9ac24b647c0e9be |
| A2 | 78 | 11d3e7d488907424bc4cdd5bd2f306a5 |
| A2 | 149 | aac60318588523e07778107c8700bc3c |
| B2 | 1 | 5fc6b2c96a16d37a4094856e2f84a83b |
| B2 | 5 | b472fd9af8cec51457cddc3c233c00b0 |
| B2 | 13 | 0e74b2133e0cec1d79f75e6473fdaa9f |
| B2 | 15 | f22bcb4cca87f4efc11ff2227cf89790 |
| B2 | 19 | 9d7726b0f7ab826627b6dfccc4179212 |
| B2 | 24 | 5400c8c7e4be2d9d6d6bfe22dde300fa |
| B2 | 28 | f89cb49f08f27db2df2cd7fc9018eb71 |
| B2 | 32 | 7ced67a4ca3d801866d19d71443d3331 |
| B2 | 34 | a871e52e0038da9c7abd4a92214b504d |
| B2 | 38 | ac28704f6f74063f9e0a79591d4e636f |
| B2 | 40 | 20936234433ab631bed08ed5a6909c6a |
| B2 | 41 | 889c2719182256259fd67e49adb63e04 |
| B2 | 42 | 13965016f866c14bc3992acc8fcd69e8 |
| B2 | 44 | 932aa68b07ca51bcfdca7918a5da6267 |
| B2 | 46 | 176236bc25abc2333f9223f6cf36973a |
| B2 | 52 | a4ce077c7782b6a0dd74d036624868b4 |
| B2 | 54 | 41b24a74ac7444b8d7bd4b9ec4c7b6c4 |
| B2 | 139 | 7e802b6f216cd872e80d40bb25086b96 |
| C2 | 80 | cfb25bb1b0e21e58e8ed279cfd3b6f2a |
| C2 | 84 | f806095ed1dea3fe04c4cae9f4c9b4fc |
| C2 | 94 | 763a5f1218685d9ad3b1a55f44b6ee75 |
| C2 | 101 | b984600ebf0877b43a36266614b0967e |
| C2 | 102 | 775135212b887e48cb25e08bb4261a30 |
| C2 | 103 | e9aa3f4b3624152dff19451964e70d29 |
| C2 | 105 | 7804195c442963b721ea7e7ece7dcdcb |
| C2 | 106 | 38696f8e3a440f926742f8d1b84543c9 |
| C2 | 110 | 964c7860cc1a495c80c85a1cba0262bc |
| C2 | 116 | a2f5af8f1f67d01603f2775da64e2958 |
| C2 | 123 | 2b97973e20f1a77b00d87e78bed3316d |
| C2 | 158 | d26de8bfbb63aeed3529abf6e80fd138 |

## Image Datasets (optional)

In addition, three image datasets are included to provide optional information using all-intra configuration.

## Tencent Video Dataset (Image)

The Tencent Video Dataset (Image), or in short TVD-I, is an image dataset of 166 images of 1920x1080 resolution that have annotations for object detection and instance segmentation. The dataset with corresponding annotations is available at [2] https://multimedia.tencent.com/resources/tvd.

## OpenImages v6

The OpenImages dataset consists of around 9 million images. A subset of the validation set of its version 6 containing 5000 images are selected for testing object detection in this activity. The dataset with corresponding annotations is available at https://storage.googleapis.com/openimages/web/index.html. The list of images that are selected can be found in the script package included in [4].

## FLIR

The FLIR dataset used in the VCM group is a dataset consisting of 300 infrared images. The images, annotations and the fine-tuned model for thermal images can be found on the MPEG content repository (https://content.mpeg.expert/data/).

- Anchor

## Anchor software

VCM-RS release v1.2 is used for the compression anchors for the VCM experiments. The VCM-RS is available at

https://git.mpeg.expert/MPEG/Video/VCM/VCM-RS.

Access right to the repository is granted to registered experts in the MPEG WG 4.

## Anchor configuration

The VCM-RS contains placeholders for some pre-inner codec and post-inner codec modules, as well as a neural-network based inner codec as an alternative for the inner codec module. These placeholders are by-passed when generating anchors. VTM 20.0 [1] is included in VCM-RS v1.2 and used as the basis for inner codec in VCM-RS.

Six configurations are defined for encoding video sequences with different GOP and latency structures using the VCM-RS as follows:

- Random access for inner (RA_inner)

- Random access for end-to-end (RA_e2e)

- Low delay for inner (LD_inner)

- Low delay for end-to-end (LD_e2e)

- All intra for inner (AI_inner)

- All intra for end-to-end (AI_e2e)

Configuration files corresponding to the above configurations are provided together with VCM-RS for experiments and evaluations. The following parameters may be changed for generating compressed data for each test point:

- input directory to specify the directory where input image files are stored or where the frames of an input video are stored

- output_dir to specify the output directory where the bitstream files and reconstructed images or frames are stored

- directory_as_video to specify the directory containing frames of an input video

- FrameRate to reflect the frame rate of a given video sequence

- FrameSkip to reflect the number of frames to be skipped

- FramesToBeEncoded to reflect the frame count of a given video sequence to be encoded

- IntraPeriod to reflect the intra refresh period in the random-access test cases. The intra refresh period is dependent on the frame rate of the source and the GOP size in use: a value 32 shall be used for sequences with a frame rate equal to 20fps, 24fps, 25fps and 30fps, 64 for 50fps, and 60fps, and 96 for 100fps.

- quality to reflect the quantization parameter value, corresponding to the quantization parameter (QP) value used in VVC

- Configuration to reflect the GOP and latency structure setting used for encoding

Note that in this CTC, GOP size for RA configuration is determined by the value of IntraPeriod. When IntraPeriod is greater than or equal to 16, GOP is set to 16, otherwise it is set to 8. Coding tool configurations are illustrated in Table 7. Note that when Adaptive Temporal Resampling is turned on, the GOP size can be changed.

Table 7 Tool settings for inner and end-to-end configurations

| Tool configurations | RA |  |  | LD |  | AI |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA_inner |  | RA_e2e | LD_inner | LD_e2e | AI_inner | AI_e2e |
| Temporal Resampling | On |  |  |  |  |  | Off |
| Adaptive Temp Resampling | 1 | 0 |  | 1 | 0 | 1 | 0 |
| Adaptive Temp Method | Resample based detection | N/A |  | Resample based detection | N/A | Resample based tracking | N/A |
| SceneChangeHandling | Off |  |  |  |  |  |  |
| ROI | On |  |  |  |  |  |  |
| RoIAccumulationPeriod | Intra Period |  |  | 32 | 1 | 1 |  |
| RoIAccumulationWindowExtension | 0 |  |  | 0 | 32 | 0 |  |
| RoIRetargetingMode | sequence |  | dynamic+pad | sequence | dynamic+pad | sequence | dynamic+fit |
| RoIRetargetingMaxNumRoIs | 3 |  |  | 1 |  | 11 |  |
| RoIAdaptiveMarginDilation | 0 |  |  | 0 |  | 0 | 1 |
| RoIAdaptivePreMergeScaling | 1 |  |  |  |  |  | 0 |
| RoIInflation | auto |  |  |  |  |  |  |
| Dynamic Colourization | On |  |  |  |  |  |  |
| Spatial Resampling | On |  |  |  |  |  |  |
| Bit Truncation and Restoration | On |  |  |  |  |  |  |
| Luma Enhancement | On |  |  |  |  |  |  |
| NN-based In-loop Filter | Off |  |  |  |  |  |  |
| GOP structure | “Random Access” |  |  | “Low delay” |  | “All Intra” |  |

In addition, RoIBackgroundFilter is set equal to 0.10 (10%) and RoIRetargetingBackgroundScale is set to 13, for the optional semantic segmentation machine task (PandaSet).

## Anchor generation

The following input arguments shall be specified according to the CTC when using VTM 20.0 as the inner codec of VCM-RS v1.3.

--FrameRate <frame rate> \

--IntraPeriod <intra period> \

--FrameSkip <frame skip> \

--FramesToBeEncoded <number of frames to be encoded> \

--quality <quality> \

--output_dir <output directory> \

<input yuv file>

Six equally spaced QP values with an interval of 4 are used to generate the anchor results for SFU-HW dataset and TVD. For example, QP1=QP0+4, QP2=QP1+4=QP0+8, …, QP5=QP4+4=QP0+20. The starting QPs (QP0) for SFU-HW dataset and TVD are provided in Table 8 and Table 9, respectively. For Panda dataset and Image dataset, the interval is 5. The QP values for generating the anchor results for Panda dataset can be found in Table 10, and the QP values for generating the anchor results for image datasets can be found in Table 11.

Table 8. Starting QPs (QP0) for sequences in SFU-HW dataset to generate anchor

| Class | Sequence name | RA (inner and e2e) LD (inner and e2e) | AI (inner and e2e) |
| --- | --- | --- | --- |
|  |  | QP0 | QP0 |
| A | Traffic | 32 | 26 |
| B | ParkScene | 16 | 16 |
| B | BasketballDrive | 26 | 20 |
| B | BQTerrace | 26 | 22 |
| C | BasketballDrill | 12 | 10 |
| C | BQMall | 22 | 20 |
| C | PartyScene | 18 | 16 |
| C | RaceHorsesC | 14 | 14 |
| D | BasketballPass | 10 | 10 |
| D | BQSquare | 12 | 12 |
| D | BlowingBubbles | 14 | 14 |
| D | RaceHorsesD | 14 | 12 |
| O | Kimono | 22 | 20 |
| O | Cactus | 20 | 22 |

Table 9. Starting QPs (QP0) for sequences in TVD to generate anchor

| Sequence name | RA (inner and e2e) LD (inner and e2e) | AI (inner and e2e) |
| --- | --- | --- |
|  | QP0 | QP0 |
| TVD-01-1 | 14 | 16 |
| TVD-01-2 | 16 | 16 |
| TVD-01-3 | 16 | 20 |
| TVD-02-1 | 20 | 20 |
| TVD-03-1 | 24 | 24 |
| TVD-03-2 | 26 | 24 |
| TVD-03-3 | 22 | 24 |

Table 10. QPs for Panda datasets to generate anchor

| Class | All Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | QP0 | QP1 | QP2 | QP3 | QP4 | QP5 |
| A1 | 22 | 27 | 32 | 37 | 42 | 47 |
| B1 | 22 | 27 | 32 | 37 | 42 | 47 |
| C1 | 22 | 27 | 32 | 37 | 42 | 47 |
| A2 | 22 | 27 | 32 | 37 | 42 | 47 |
| B2 | 22 | 27 | 32 | 37 | 42 | 47 |
| C2 | 22 | 27 | 32 | 37 | 42 | 47 |

Table 11. QPs for image datasets to generate anchor

| Dataset | AI Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | QP0 | QP1 | QP2 | QP3 | QP4 | QP5 |
| OpenImageV6 | 22 | 27 | 32 | 37 | 42 | 47 |
| FLIR (IR) | 22 | 27 | 32 | 37 | 42 | 47 |
| TVD-I | 22 | 27 | 32 | 37 | 42 | 47 |

- Evaluation

## Evaluation pipeline

Figure 1 and Figure 2 show the pipeline for evaluation of proposed VCM technologies for image and video datasets, respectively. These pipelines consist of the following stages:

- For video datasets, the original video (in YUV420 format) and for image datasets, images (in PNG format) are encoded by the VCM-RS encoder into bitstreams. Encoding can be conducted on one GPU, or more than one GPUs simultaneously. The scripts for enabling multi-GPU encoding are available at https://git.mpeg.expert/MPEG/Video/VCM/VCM-RS/-/blob/main/Scripts/VTM_InnerCodec/run_all.sh.

- Bitstreams are decoded by the VCM-RS decoder to produce decoded video (in YUV420 format) and images (in PNG format).

- For video datasets, the task network is performed using the decoded video after conversion from YUV420 to PNG format. For image datasets decoded PNGs are directly supplied to the task network to produce a task network output.

- A task evaluator produces a task result using the task network output and the ground truth.

- An objective visual evaluator produces Y-PSNR, U-PSNR, V-PSNR, and RGB-PSNR results using the decoded video (image) and the original video (image).

Figure 1. Pipeline of VCM evaluation framework for video datasets.

Figure 2. Pipeline of VCM evaluation framework for image datasets.

The video dataset pipeline of Fig. 1 is applied to the following datasets:

- SFU-HW Dataset

- Tencent Video Dataset (TVD)

- PandaSet

The image dataset pipeline of Fig. 2 is applied to the following datasets:

- Tencent Video Dataset (Image)

- OpenImages v6

- FLIR

Note that the SFU-HW videos are provided in YUV420 format, while the TVD videos are provided in MP4 format and require conversion to YUV420 before encoding by VCM-RS.

FFmpeg [3] should be used when format conversion is needed for coding and evaluation. Some example command lines are as follows.

- png to yuv: ffmpeg -i {input.png} -f rawvideo -pix_fmt yuv420p -dst_range 1 {output.yuv}

- yuv to png (per frame): ffmpeg -f rawvideo -s {width}x{height} -pix_fmt yuv420p10le -i {input.yuv} -vsync 1 -y -pix_fmt rgb24 {outputPngfolder}/%06d.png

## Task networks

## Object detection

The following network architecture is used for the evaluation of object detection task.

- Faster R-CNN [3] X101-FPN (part of Facebook AI Research’s Detectron2 [4])

Model parameters file is ‘model_final_68b088.pkl’ available here. Note that FLIR anchor is generated using retrained model parameters that are available in the MPEG Content Repository.

## Object tracking

The following network architecture is used for the evaluation of object tracking task.

- JDE-1088x608 [5]

Model parameters file is ‘jde.1088x608.uncertainty.pt’ available here or here.

## Semantic segmentation

The following network architecture is used for the evaluation of semantic segmentation task.

- Panoptic FPN R-101-FPN 3x (part of Facebook AI Research’s Detectron2 [4])

Model parameter file is available here.

- Test conditions

## Fixed QP

For the proposals with a quantization concept, results shall be provided using the six quantization parameter values that match to the anchor’s QP values. These values define the initial QP values that are specified as the input QP of the proposals. Adaptation of QP during compression and processing shall be described in the proposals. If the task R-D performance of a proposed method does not overlap with that of the anchor, and thus impacting the BD-rate calculation, extra QP points shall be used to generate  R-D task performance results within a range that overlaps sufficiently with the anchor.

## Bit-rate Target

For proposals that do not have a quantization concept substantially similar to the anchor, such as so-called super-resolution and/or end-to-end (E2E) architectures, results shall be provided for six rate-distortion points for each sequence corresponding to the anchor quantization values. Each of the six provided points is requested to be ±10% of the rate of the corresponding quantization parameter of the anchor.  This rate comparison shall be performed on a sequence basis.

- Training conditions

A proposal can train the compressor with various training materials. The training materials shall be publicly available with license terms that allow commercial usage or can be used by standards committees or standardization activities. No images or videos from the VCM validation dataset shall be used in training. It is required that the usage of any training materials be described in the contribution for reproducing training results.

Table 11 describes a list of recommended training sets. It is desirable that all proposals use the specific training sets.  Results using sequences not in the list of specific sequences may also be provided as supplemental information.

Table 11 Recommended source for training datasets

| Database | Location |
| --- | --- |
| OpenImageV6-train+ | https://storage.googleapis.com/openimages/web/index.html |
| TVD-train* | https://multimedia.tencent.com/resources/tvd |
| BVI-DVC dataset | https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc |

+OpenImageV6-train: the “train” subset of OpenImageV6.

*TVD-train: TVD video dataset excluding sequences listed in Appendix.

In general, if an image or video, or part of it is used for testing, it shall not be used for training.

- Reporting

## Machine task performance

- Bitrate measurement

For image datasets, bits per pixel (BPP) shall be used. For video sequences, the bitrate shall be measured in kilobits per second (kbps).

- PSNR

The Peak Signal to Noise Ratio.

- SSIM

The Structural Similarity.

- MS-SSIM

The Multi-scale Structural Similarity.

- mAP for detection and segmentation tasks

For both object detection and object segmentation, mean Average Precision (mAP) shall be used to measure the performance of the detection results. Please refer to [6] for more information about mAP.

- MOTA for tracking tasks

For the object tracking task, Multiple Object Tracking Accuracy (MOTA) shall be used to measure the tracking performance. Please refer to [6] for more information about MOTA.

- MIoU for semantic segmentation tasks

For semantic segmentation task, Mean Intersection over Union (MIoU) shall be used to measure the performance.

Assuming k is the number of categories, MIoU is calculated as

MIoU=1ki=1kPiij=1kPij+j=1kPji-Pii

where Pij refers to the number of pixels inferred from category i as category j, Pii refers to the true positive. Pij and Pji refer to the false positive and false negative, respectively.

- BD-rate machine task calculation

A cubic polynomial function-based curve fitting method [7] is applied to the mAP or MOTA score values before they are used to calculate the machine task BD-rate performance in the reporting template. This curve fitting process is automated using the scripts provided in http://mpegx.int-evry.fr/software/MPEG/Video/VCM/vcm-ctc/scripts directory. After curve fitting, the middle four QP points are used for calculating BD-rate, i.e., the lowest and the highest QPs are excluded from the BD-rate calculation.

## Runtime

Runtime including Encoding time (EncT) and Decoding time (DecT) shall be reported for complexity measurement. The proposed runtime methods for a VCM solution are:

- EncT: Time needed to convert input video or image to bitstream.

- DecT: Time needed to convert bitstream to reconstructed output video or image.

For the purpose of reporting encoding and decoding runtimes, the anchor and proposal shall be simulated on the same platform, e.g., similar CPU and GPU configuration, to have reliable time comparison. It is encouraged to report runtimes of modules, e.g., Temporal Resample, Spatial Resample, RoI, Bit-depth truncation and Inner codec, etc.

From the anchor and proposal EncT and DecT, encoder runtime ratio (EncR) and decoder runtime ratio (DecR) are computed in the result template.

## Inference information

In additional to encoding and decoding time, the information described below is required to be provided for the inference process for both encoding and decoding process.

- Network Visualization: Graphical representation of the neural network

- Param. Number: Total numbers of parameters in the neural network.

- Param. Precision: Bits for storing one parameter. Besides, using “I” for indicating the integer, and using “F” for indicating the floating number. For example, if the proposed method uses 16-bit integer to represent a parameter, you can report this information as “16 (I)”.

- MAC (Kilo): Number of multiply–accumulate operations in inference stage per pixel, where the multiply–accumulate operation is a common step that computes the product of two numbers and adds that product to an accumulator.

- Mem.T (MB): Temporary memory. It denotes the memory used to store the output feature map in each intermediate layer (forward pass). Since different size of input may influence the value, it is suggested to use 3840x2160 as the input size for unification, if there is no parallel operation. Or, if block level parallel operation is used in the proposed method, the block size can be used as the input for calculation, while the input size should be reported. For reporting Mem.T (MB) the calculation process is also suggested to be provided for crosschecking.

- Patch Size:	size of input to the neural networks during inference (patchW×patchH×patchT, e.g. 64x64x3)

## Training information

It is required to report and discuss the following information for the training process.

- Epoch:	The number of complete passes through the training data (e.g. 100)

- Batch Size:	The number of samples processed before the model is updated. (e.g. 4Kx16frames)

- Training Time: CPU and/or GPU (e.g. 48h)

- Learning Curve: Plot of the training loss and validation loss (or similar) versus the number of epochs

- Training Sets: Training sets used. If a pre-trained model is used, the source of the pre-trained model and its training sets should be reported in detail.

- Training Configuration per Rate-Distortion Point: Any changes in the requested information used to generate different rate-distortion points. Additional training information could also help to better understand the proposed neural network-based method and thus encouraged to be included in the contribution.

- Number of Iterations: number of gradient updates within an epoch

- Patch Size:	size of input to the neural networks (patchW×patchH×patchT, e.g. 64x64x3)

- Learning Rate:	The amount that the weights are updated during training (e.g. 5e-4)

- Optimizer:	The algorithm used to change the attributes of proposed neural networks (e.g. ADAM)

- Loss Function:	The function to calculate the model error during training and optimization (e.g. L1, L2, etc.)

- Preprocessing:	(e.g. preprocessing procedure, normalization, cropping method, rotation, zoom etc.)

- Cross-check criteria

When a technology is proposed in VCM, it shall pass a successful cross-check by an independent expert not affiliated with the proponent for consideration of adoption. The cross checker shall report bitrate, machine task performance, encoding decoding and runtimes.

The successful cross-check criteria are as follows:

- The bitrates reported by the proposal and cross-check report shall match exactly.

- The difference in machine task performance between results provided by the proponent and the cross-checker shall be less than 1%. For example, any machine task performance value between 9.9% and 10.1% in the cross-check report is considered successful when the reported machine task performance value in the proposal is 10%.

- Reference

- ISO/IEC JTC1/SC29/WG5, "VVC Reference Model (VTM)," https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git.

- X. Xu, S. Liu and Z. Li, "A Video Dataset for Learning-based Visual Data Compression and Analysis," in 2021 International Conference on Visual Communications and Image Processing (VCIP), Dec. 2021.

- S. Ren, K. He, R. Girshick, et al. "Faster R-CNN: Towards Real-Time Object Detection with Region Proposal Networks," IEEE Transactions on Pattern Analysis and Machine Intelligence, 2016, 39(6): 1137-1149.

- Y. Wu, A. Kirillov, F. Massa, et al. "Detectron2," https://github.com/facebookresearch/detectron2

- Z. Wang, L. Zheng, Y. Liu, et al. "Towards real-time multi-object tracking," in European Conference on Computer Vision (ECCV). 2020: 107-122.

- ISO/IEC JTC1/SC29/WG4, “Evaluation framework and methodologies for video coding for machines,” N00277, Mainz, October 2022.

- H. Wang et al., "Improvements of the BD-Rate Metrics Using Monotonic Curve-Fitting Methods," 2024 Picture Coding Symposium (PCS), Taichung, Taiwan, 2024, pp. 1-5, https://doi.org/10.1109/PCS60826.2024.10566370.

- https://git.mpeg.expert/MPEG/Video/VCM/VCM-RS.

# Appendix: TVD-train dataset

TVD-train is a training dataset consisting video sequences from TVD excluding sequences listed below:

*MovingBikes_3840x2160_50fps_8bit_420

FallingLeaves_3840x2160_50fps_8bit_420

FallenLeaves_3840x2160_50fps_8bit_420

GirlThrowingLeaves_3840x2160_50fps_8bit_420

GirlWalkingOnStreet_3840x2160_50fps_8bit_420

GirlWatchingPhone_3840x2160_50fps_8bit_420

StaticWaterAndBikes2_3840x2160_50fps_10bit_420

GirlRunningOnGrass_3840x2160_50fps_10bit_420

GirlsOnGrass1_3840x2160_50fps_10bit_420

GirlsOnGrass2_3840x2160_50fps_10bit_420

*PeopleOnGrass_3840x2160_50fps_10bit_420

*MovingBikesAndPedestrian4_3840x2160_50fps_10bit_420

BoyMakingUp1_3840x2160_50fps_10bit_420

BoyMakingUp2_3840x2160_50fps_10bit_420

BoyDressing1_3840x2160_50fps_10bit_420

BoyDressing2_3840x2160_50fps_10bit_420

BoyWithCostume_3840x2160_50fps_10bit_420

MountainsAndStairs1_3840x2160_24fps_10bit_420

MountainsAndStairs4_3840x2160_25fps_10bit_420

GirlWithTeaSet1_3840x2160_25fps_10bit_420

GirlWithTeaSet2_3840x2160_25fps_10bit_420

GirlWithTeaSet3_3840x2160_25fps_10bit_420

CableCar_3840x2160_25fps_10bit_420

HotelClerks_3840x2160_25fps_10bit_420

RestaurantWaitress1_3840x2160_25fps_10bit_420

RestaurantWaitress2_3840x2160_25fps_10bit_420

ChefCuttingUp1_3840x2160_25fps_10bit_420

ChefCuttingUp2_3840x2160_25fps_10bit_420

ChefCooking2_3840x2160_25fps_10bit_420

ChefCooking3_3840x2160_25fps_10bit_420

ChefCooking4_3840x2160_25fps_10bit_420

ChefCooking5_3840x2160_25fps_10bit_420

RawDucks_3840x2160_25fps_10bit_420

PeopleNearDesk_3840x2160_25fps_10bit_420

HotPot_3840x2160_25fps_10bit_420

RiverAndTrees_3840x2160_25fps_10bit_420

BuildingTouristAttraction2_3840x2160_25fps_10bit_420

BuildingTouristAttraction3_3840x2160_25fps_10bit_420

RoomTouristAttraction1_3840x2160_25fps_10bit_420

RoomTouristAttraction2_3840x2160_25fps_10bit_420

RoomTouristAttraction3_3840x2160_25fps_10bit_420

RoomTouristAttraction4_3840x2160_25fps_10bit_420

RoomTouristAttraction5_3840x2160_25fps_10bit_420

RoomTouristAttraction6_3840x2160_25fps_10bit_420

RoomTouristAttraction7_3840x2160_25fps_10bit_420

BlackBird_3840x2160_25fps_10bit_420

StampCarving1_3840x2160_25fps_10bit_420

StampCarving2_3840x2160_25fps_10bit_420

Weave_3840x2160_25fps_10bit_420

ManWithFilmMachine_3840x2160_25fps_10bit_420

LyingDog_3840x2160_25fps_10bit_420

OilPainting1_3840x2160_25fps_10bit_420

OilPainting2_3840x2160_25fps_10bit_420

# *These sequences are used in video task testing; while other sequences contain extracted frames used for image task testing.