INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 3

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 03 N1777

Santa Eulària, ES – April 2026

| Title | Exploration on application specific metadata definition and carriage in media AUs – Annex on WebCodecs focus approach |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 26169 |

# Introduction

The exploration on “Media type independent metadata” (N1703) has been carried out over the past two years starting from the split rendering use case and evolved to generic concept of transporting metadata for access units for various use cases.

The contribution m76690 submitted at MPEG #154 proposed a new Work Item for a generic carriage of metadata. From the discussion during the BoG session, the following conclusions were reached (#15):

Embedding metadata would be useful if such metadata needs to be extracted before the decoder and delivered to the application through standardized API.

From conventional decoder design point of view, applying such data to the decoded media data before the composition/rendering could not be feasible.

If the extracted data is simply sent to the unknown application, then it is very similar to private data. We can define an envelop framework with code assigned. Then, we need to have registration system.

It might be a way to define an application communicating with out SDM through a conceptual APIs. Then, select one API currently existing, e.g. WebCodec APIs, as a starting point then define methods of delivering metadata fed to such API.

To revise the contribution showing two interfaces, before the decoder and after the decoder considering WebCodec APIs.

Based on the conclusions above, this contribution reshaped the initial proposed scope to match the scope and goals to target the binding onto WebCodecs API.

# Background on W3C WebCodecs

## API specification

The WebCodecs specification [1] defines standardized JavaScript interfaces for encoding and decoding audio, video, and image data bitstreams. It establishes a framework that decouples media processing from specific codec implementations, allowing web application uthors to interact with various codecs.

Table 1 - Summary of W3C WebCodecs components

| Component | Description |
| --- | --- |
| VideoDecoder | Decodes video streams into frames. |
| VideoEncoder | Encodes frames into video streams. |
| AudioDecoder | Decodes audio streams into audio frames. |
| AudioEncoder | Encodes audio frames into audio streams. |
| VideoFrame | Represents a single frame of video data. |
| AudioData | Represents a single audio frame. |
| EncodedVideoChunk | Represents a chunk of encoded video data. |
| EncodedAudioChunk | Represents a chunk of encoded audio data. |
| VideoDecoderConfig | Configuration for video decoding, including codec and dimensions. |
| AudioDecoderConfig | Configuration for audio decoding, including codec and sample rate. |
| VideoEncoderConfig | Configuration for video encoding, including codec and encoding options. |
| AudioEncoderConfig | Configuration for audio encoding, including codec and encoding options. |
| WebCodecsErrorCallback | Callback for handling errors in encoding/decoding processes. |

## Current adoption in web browsers

The WebCodecs API is readily available on major web browsers implementation today as can be seen on Figure 1. Note that the support for each codec extension defined by WebCodecs may vary per web browser, device, OS, etc…

Figure 1 - Support of WebCodecs API in we browsers (2026/04/30)

# Thoughts for a MPEG Systems technologies binding on W3C WebCodecs

## General

Based on the exploration work and the discussion which happened at MPEG #154, SC29WG03 could envision starting a project to integrate MPEG Systems technologies into the W3C WebCodecs API. The goal of this binding would be mapping each W3C WebCodecs listed in onto a concept MPEG Systems technologies, e.g., access unit, elementary stream, composition unit, composition timestamp, etc., as well as mapping WebCodecs functions and API interfaces onto the MPEG Systems Decoder Model, with possible extensions when needed.

## Initial thoughts on the binding

As reminder, the MPEG-4 Part 1 System Decoder model is composed of the Decoding Buffer, Decoder, Composition Memory and Compositor as depicted in Figure 2

Figure 2 - MPEG-4 Part 1 Systems Decode Model

As part of the BoG discussion during MPEG #154, it was discussed that the extraction of access unit metadata would imply that an additional block before the decoder would perform this extraction. Then this extracted metadata would be passed on to the application as depicted in Figure 3.

Figure 3 - Metadata-aware System Decoder Model

Based on this, a crude mapping can be made using the concept listed in Table 1 which is depicted in Figure 4.

Figure 4 - Initial thoughts for WebCodecs mapping to MPEG Systems Decoder Model

## Possible deliverables

This work could result in the following deliverables:

- A new International Standard defining:

  - The mapping of WebCodecs objects, including encoded chunks, decoder configuration, decoded audio and video data and video frame metadata onto MPEG Systems concepts.

  - For video frame metadata, when carried in the bitstream, the carriage of this metadata in video elementary stream.

- What remains as open questions:

  - The WebCodecs API supports audio, video and images encoding and decoding. W3C also defines several extensions for integration of specific codecs. The codecs in scope for this possible new work would still need to be decided and the work could only target a subset of the supported MPEG codecs by WebCodecs.

  - Defining a MPEG API that can be mapped onto WebCodecs instead directly working against the WebCodecs API. This would allow other media APIs to be mapped onto MPEG Systems technologies and not only WebCodecs.

  - Creating a registration process (e.g., in MP4RA) for external organisations to register application-level metadata to be carried via video elementary stream and exposed through the metadata() function of the VideoFrame object.

# Conclusion

We recommend:

- Including sections 2 and 3 as new document as part of the output document of the current exploration.

- Inviting study of this new annex by MPEG experts for consideration as possible new work item to be discussed in MPEG #155.

# References

- WebCodecs, https://www.w3.org/TR/webcodecs/ ‚

- WebCodecs API | Can I use... Support tables for HTML5, CSS3, etc, https://caniuse.com/webcodecs