ISO/IEC JTC 1/SC 29/WG 03 N1809

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Exploration on Systems technology for AI-based media standards (SyfAI)

Status:	Approved

Date of document:	2026-05-02

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	9 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N1809

April 2026, Santa Eularia, SP

| Title | Exploration on Systems technology for AI-based media standards (SyfAI) |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Approved |
| Serial Number | 26201 |

Summary

During the MPEG 153rd meeting, exploration on systems technology for AI-based media standards work was established, especially focusing on the carriage of VCM NAL unit structured video in the ISO Base Media File Format (ISOBMFF). This document summarizes the outcome of 154th discussion on the same topic and also reflects key questions raised during subsequent MPEG discussions, including flexible AI codec model framework and carriage of compressed neural network.

This exploration provides background information on system-level considerations for VCM bitstream carriage, analyses ideas around the definition and placement of configuration and metadata and discusses open issues and potential directions for AI-based media standards as further study.

1	Introduction

With the establishment of the DIS for Video Coding for Machines (VCM), the definition of system-level mechanisms for storing and conveying VCM bitstreams has become necessary for practical deployment. In particular, compressed VCM bitstreams are expected to be used within existing media container and delivery frameworks.

Contribution m76651 addressed the overall architecture for carriage of VCM bitstream and also addressed updates on NAL unit structured video in the ISO Base Media File Format.

2	Overview: Carriage of NAL unit structured video in the ISOBMFF for VCM

X.1 Overall architecture for carriage of VCM bitstream data

Figure 1 shows a typical content flow process for VCM media.

Figure 1 — Content flow process for VCM media

A real-world scene (A) is captured by a set of cameras, a camera device with multiple lenses and sensors, or by virtual cameras. The acquisition results in source video data (B). One or multiple video frames are encoded as a VCM bitstream (Ev). One or more VCM bitstreams are then packaged into a media file for local playback (F) or a sequence of an initialization segment and media segments for streaming (Fs), according to a particular media container file format. In this document, the media container file format is the ISO Base Media File Format specified in ISO/IEC 14496-12. The file encapsulator may also include metadata into the file or the segments. The segments Fs are delivered using a delivery mechanism to a player.

The file that the file encapsulator outputs (F) is identical to the file that the file decapsulator takes as input (F'). A file decapsulator processes the file (F') or the received segments (F's) and extracts the coded bitstreams (E'v) and parses the metadata.  The VCM bitstream is then decoded into a decoded signal (B'). The decoded data (B') is rendered, and displayed onto the screen or may be consumed for machine analysis.

X VCM elementary streams and sample definitions

X.1 Overview

The Video Coding for Machines (VCM) standard (ISO/IEC 23888-2) offers increased coding efficiency and many features for the systems that use it. To enable the best visibility of, and access to, those features, and to enhance the opportunities for the interchange and interoperability of media. This clause of ISO/IEC 14996-15 specifies the storage format for single-layer VCM (ISO/IEC 23888-2) video streams.

The storage of VCM content uses the existing capabilities of the ISO base media file format but also defines extensions to support the following features of the VCM codec.

- The video, sequence, and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded video data. In VCM, a picture that contains coded video data(VCM_CVD) references the picture parameter set(VCM_RSD_PRD) containing its decoding parameters. When a picture does not contain coded video data, restoration information for the picture is conveyed solely in a temporal synthesis picture data(TSPD) NAL unit. In turn, picture-level restoration and temporal synthesis information (conveyed in VCM_RSD_PRD and TSPD NAL units)  references a sequence parameter set that contains sequence level decoding parameter information, and the sequence parameter set references a video parameter set that contains global decoding parameter information.

X.2 Elementary stream structure

Two types of elementary streams are defined for storing VCM content:

- Video elementary stream that does not contain any parameter sets; all parameter sets are stored in a sample entry or sample entries;

- Video and parameter set elementary stream that may contain parameter sets, and may also have parameter sets stored in their sample entry or sample entries;

X.3 Sample and configuration definition

X.3.1 Canonical order and restrictions

The canonical stream format is a VCM elementary stream that satisfies the following conditions in addition to the general conditions in 4.2.2:

NOTE 	The general conditions referenced in 4.2.2 are defined in Clause 4 (General definitions).

—	All VCL NAL units for a single picture shall be contained within the sample whose decoding time and composition time are those of the picture.

- A sample may contain either coded video data NAL units or temporal synthesis picture data NAL units, according to the VCM bitstream syntax

—	Access unit delimiter NAL units: The constraints obeyed by access unit delimiter NAL units are defined in ISO/IEC 23888-2.

—	Parameter sets: For a VCM video stream to which a particular sample entry applies, VCM parameter set NAL units, including the VCM video parameter set (VCM_VPS) and VCM restoration data NAL(VCM_RSD) units carrying sequence-level or picture-level parameter information, shall be stored only in the sample entry when the sample entry name is 'vcm1', and may be stored in both the sample entry and the samples when the sample entry name is 'vcmi'.

NOTE 1	A parameter set NAL unit to be used for decoding or restoration of an output picture shall be conveyed prior to the sample associated with that output picture or within the sample itself.

NOTE 2	Storing VCM parameter set NAL units in the sample entry of a VCM video stream provides a simple and static method for supplying parameter information. Storing VCM parameter set NAL units in samples allows greater flexibility, including updates of parameter information using the same identifier or the introduction of additional parameter sets. A VCM decoder initializes using the parameter sets present in the sample entry and may subsequently update its state using parameter sets encountered in the stream, starting from any sample marked as a sync sample. When the sample entry changes, the decoder re-initializes using the parameter sets contained in the new sample entry.

— SEI messages: SEI message NAL units of a declarative nature may be stored in the sample entry. There is no restriction on the presence or removal of such SEI message NAL units in samples.

— Filler data: Video data is naturally represented as variable bit rate in the file format and should be filled for transmission if needed. Filler Data NAL units and Filler Data SEI messages shall not be present in the stored stream when the sample entry does not permit in-stream parameter sets.

NOTE 3	The removal or addition of Filler Data NAL units, start codes, SEI messages or Filler Data SEI messages can change the bitstream characteristics with respect to conformance with the VCM HRD model when operating the HRD in CBR mode as specified in ISO/IEC 23888-2 Annex C.

X.3.2 Decoder configuration information

X.3.2.1 VCM decoder configuration record

X.3.2.1.1 Definition

X.3.2.1.2 Syntax

aligned(8) class VCMDecoderConfigurationRecord {
	unsigned int(8) configurationVersion = 1;
	unsigned int(1) general_tier_flag;
	unsigned int(7) general_profile_codec_group_idc;
	unsigned int(8) general_profile_restoration_idc;
	unsigned int(8) general_level_idc;
	unsigned int(16) avgFrameRate;
	unsigned int(1) constantFrameRate;
	bit(5) reserved = '11111'b;
	unsigned int(2) lengthSizeMinusOne; 
	unsigned int(8) numOfArrays;
	for (j=0; j < numOfArrays; j++) {
		unsigned int(1) array_completeness;
		bit(1) reserved = 0;
		unsigned int(6) NAL_unit_type;
		unsigned int(16) numNalus;
		for (i=0; i< numNalus; i++) {
			unsigned int(16) nalUnitLength;
			bit(8*nalUnitLength) nalUnit;
		}
	}
}

X.3.2.1.3 Semantics

general_tier_flag, general_profile_codec_group_idc, general_profile_restoration_idc, general_level_idc, contain the matching values for the fields general_tier_flag, general_profile_codec_group_idc, general_profile_restoration_idc, and general_level_idc as defined in ISO/IEC 23888-2, for the stream to which this configuration record applies.

avgFrameRate gives the average frame rate in units of frames/(256 seconds), for the stream to which this configuration record applies. Value 0 indicates an unspecified average frame rate.

constantFrameRate equal to 1 indicates that the stream to which this configuration record applies is of constant frame rate. Value 0 indicates that the output frame rate may vary. (Value 0 indicates that the stream may or may not be of constant frame rate.)

lengthSizeMinusOne plus 1 indicates the length in bytes of the NALUnitLength field in an VCM video sample in the stream to which this configuration record applies. For example, a size of one byte is indicated with a value of 0. The value of this field shall be one of 0, 1, or 3 corresponding to a length encoded with 1, 2, or 4 bytes, respectively.

numArrays indicates the number of arrays of NAL units of the indicated type(s).

array_completeness when equal to 1 indicates that all NAL units of the given type are in the following array and none are in the stream; when equal to 0 indicates that additional NAL units of the indicated type may be in the stream; the permitted values are constrained by the sample entry name.

NAL_unit_type indicates the type of the NAL units in the following array (which shall be all of that type); it takes a value as defined in ISO/IEC 23888-2; it is restricted to take one of the values indicating a VCM_VPS, VCM_RSD_SRD, VCM_SRD_PRD, VCM_TSPD, VCM_RSD_SEI message NAL unit.

numNalus indicates the number of NAL units of the indicated type included in the configuration record for the stream to which this configuration record applies. The SEI array shall only contain SEI messages of a 'declarative' nature, that is, those that provide information about the stream as a whole. An example of such an SEI could be a user-data SEI.

nalUnitLength indicates the length in bytes of the NAL unit.

nalUnit contains an VCM_RSD_SRD, VCM_RSD_PRD, VCM_TSPD, VCM_VPS or declarative SEI NAL unit, as specified in ISO/IEC 23888-2.

X.4 Derivation from the ISO base media file format

X.4.1 VCM stream definition

X.4.1.1 Sample entry name and format

X.4.1.1.1 Definition

Sample Entry and Box Types:	'vcm1', 'vcmi', 'vcmC'
Container:	Sample Table Box ('stbl')
Mandatory:	An 'vcm1' or 'vcmi' sample entry is mandatory
Quantity:	One or more sample entries may be present

Box Type:	'vcmC'
Container:	VCM Sample Entry ('vcm1' or 'vcmi')
Mandatory:	Yes 
Quantity:	One

A VCM visual sample entry shall contain a VCM Configuration Box, as defined below. This box includes a VCMDecoderConfigurationRecord, as defined in X.3.2.1.

An optional BitRateBox may be present in the VCM visual sample entry to signal bit rate information of the VCM video stream. Extension descriptors that may be inserted into the Elementary Stream Descriptor, when used in MPEG-4 systems, may also be present.

Multiple VCM visual sample entries may be used, as permitted by ISO/IEC 14496-12, to indicate sections of video that use different configurations or parameter sets.

When the sample entry name is 'vcm1' or 'vcmi', the stream to which this sample entry applies shall be a compliant VCM bitstream, as viewed by a VCM decoder operating under the configuration (including profile, tier, and level) given in the associated VCMConfigurationBox.

When the sample entry name is 'vcm1', the value of array_completeness shall be equal to 1 for the arrays of all types of parameter set NAL units present in the VCM Configuration Box.

X.4.1.1.2 Syntax

class VcmConfigurationBox extends Box('vcmC') {
	VcmDecoderConfigurationRecord() VCMConfig;
	InnerCodecConfigurationRecord() InnerCodecConfig;
}

class VcmSampleEntry() extends VisualSampleEntry ('vcm1' or 'vcmi'){
	VcmConfigurationBox	config;
	MPEG4ExtensionDescriptorsBox ();	// optional
}

X.4.1.1.3 Semantics

Compressorname in the base class VisualSampleEntry indicates the name of the compressor used with the value "\012VCM Coding" being recommended (\012 is 10, the length of the string in bytes).

VcmDecoderConfigurationRecord is defined in ‎X.3.3.6

InnerCodecConfigurationRecord is the DecoderConfigurationRecord of the inner codec supported by VCM, defined in subclause 7.4.3.2 in ISO/IEC 23888-2.

3	Discussion during MPEG 154th

During MPEG 154th , Online and gitlab based discussions happened. The discussions are here.

A summary of the questions is as follows:

- flexible AI codec model framework

To support flexible AI codecs, the proposal highlights the need for:

- Unified terminology (modules, blocks, primitives, etc.)

- System-level support, including:

  - Model discovery, validation, storage, delivery

  - Configuration update mechanisms

- Standardized signaling of:

  - Models, weights, metadata, training info

- Potential model repository infrastructure (like GitHub instead of static files)Proposed text for carriage of VCM data over MPEG2-TS.

Comments:

need to define a conceptual architecture in terms of

- when/where/how does system fetch in

draw boxes and define requirements considering following, but not limited to

- security (in terms of network, data)

- authentication

- level of flexibility

- repository

Scope of work needs to be defined

- Only for mpeg

- menifest shall include inner functionality of codec, delivery of model, capalibity, etc.

Starting point (codec)

- AI-PCC

- FCM

- in terms of interface, functionalities, format (envelope of menifest)Use case is requested for the next meeting

Recommendation: Noted. Bring architectural diagram and requirements.

- carriage of compressed neural networks

This contribution proposes starting a new MPEG standard for storage and carriage of compressed neural networks, as defined in ISO/IEC 15938‑17, motivated by emerging machine‑centric use cases that require reusable and extensible neural network assets beyond existing NAL unit–based file format dependencies.

Comments: FF is the only way of doing this? or the best way?

It is already in FF (part 15 in NAL format), however, NAL based format may not be the best way for codec agnostic perspective.

Still need more concrete justification.

Actual use case may help.

Recommendation: Noted. Bring use case for the next meeting

- Transport of VCM video over MPEG-2 Systems

Proposed text for carriage of VCM data over MPEG2-TS.

It is not clear why M2TS extension is necessary for VCM.

Recommendation: Noted. Bring use case for the next meeting

- Carriage of NALU based VCM bitstresm

Define an architecture for the carriage of VCM bistream

Update the VCMDecoderConfigurationRecord and the corresponding VcmConfigurationBox to indicate Configuration Record related to Inner codec.

Recommendation: Accepted and update the exploration document.

3.1	Potential Work in MPEG

Further work in MPEG is required to define consistent carriage and transmission mechanisms for VCM bitstreams across different delivery environments. In particular, the definition of VCM bitstream carriage should consider ISO/IEC 14496-12, ISO/IEC 14496-15, and ISO/IEC 13818-1 in order to ensure interoperable storage, multiplexing, timing, and synchronized delivery of VCM elementary streams. Additional clarification may also be required for the signaling of VCM-specific NAL units and decoder configuration information at the system level.

3.2	Next steps

As a next step, the current exploration may be refined based on the discussions held in the MPEG 154th meeting and used as input for further system-level studies. Subject to interest within MPEG, the work may be continued for the development of the carriage and transmission of VCM bitstreams.

4	References

[1] ISO/IEC 14496-15:2024, Information technology - Coding of audio-visual objects - Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format.

[2] ISO/IEC 14496-12:2022, Information technology - Coding of audio-visual objects - Part 12: ISO base media file format.

[3] ISO/IEC 13818-1:2023, Information technology – Generic coding of moving pictures and associated audio information - Part 1: Systems.

[4] ISO/IEC JTC 1/SC 29/WG 04, “Text of ISO/IEC DIS 23888-2 Video coding for machines,” WG04N00793, Online, January 2025