# ISO/IEC JTC 1/SC 29/WG 3      N01813

ISO/IEC JTC 1/SC 29/WG 3

MPEG Systems

Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Exploration of carriage of static Gaussian Splat in HEIF

Status:	Approved

Date of document:	2026-06-01

Source:	ISO/IEC JTC 1/SC 29/WG 3

Expected action:	None

Action due date:	None

No. of pages:	24- (including cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG3

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG3 N01813

April 2026, Santa Eurelia

| Title | Exploration of carriage of static Gaussian Splat in HEIF |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Editor | Emre B. Aksu (Nokia) |
| Status | Approved |
| Serial Number | 26205 |

# Summary

During MPEG#153 a first proposal was provided of carriage of static Gaussian Splat in HEIF.

During MPEG#154, several other contributions on this topic were discussed and several comments were collected. This document consolidates these contributions based on decisions and agreements made during the meeting, including open questions and discussion points.

# 1	Introduction

This exploration document provides information, technology alternatives and related discussion points about potential mechanisms for the carriage of Gaussian Splats in an ISOBMFF/HEIF compliant file.

Gaussian Splats represent a scene as a set of anisotropic 3D Gaussians with per primitive appearance terms and are increasingly used to enable photorealistic rendering from different viewpoints.

The objective is to enable interoperable storage and delivery of Gaussian Splats in the same packaging ecosystem used for still images and image sequences. A conforming file shall have a dual-use (i.e. as an image and as a Gaussian splat representation) functionality, for example, remain usable by legacy HEIF viewers by providing a conventional cover image, while Gaussian splat aware applications and decoders can access the additional representation and render a 3D experience.

The exploration targets a Gaussian Splat representation compression agnostic carriage mechanism, while taking into account different Gaussian splat representation formats.

# 2 	Existing non-ISO-defined Static Gaussian Splat representations

Table 1 provides the list of currently known static Gaussian splat representations and formats used by the industry. In addition to these formats, it should also be noted that there is ongoing standardization work within ISO and JVET for MPEG V-PCC, G-PCC and JVET vSEI based static Gaussian Splat coding and their storage formats.

Table 1 Overview of non-ISO-defined Gaussian splat representations and formats used by the industry

| Format / Specification | File Extension | Storage / Coding Mechanism | Main Properties Encoded | Typical Use | Link to specification / information |
| --- | --- | --- | --- | --- | --- |
| Gaussian PLY (3DGS) | .ply | Raw floating-point attributes in PLY | Position, scale, rotation, opacity, full SH | Training, editing, archival, mezzanine data format, research | https://paulbourke.net/dataformats/ply/ |
| SPZ (Niantic) | .spz | Fixed-point + entropy coding | Full GS attributes incl. SH | Distribution, web, mobile | https://github.com/nianticlabs/spz |
| SOG (PlayCanvas) | .sog | Image-backed container (WebP + metadata) | GS parameters + palette SH | Web delivery | https://developer.playcanvas.com/user-manual/gaussian-splatting/formats/sog/ |
| SPLAT | .splat | Flat binary, GPU-oriented layout | Quantized geometry, limited color/SH | Lightweight web viewers | https://github.com/antimatter15/splat/blob/main/README.md |
| KSPLAT | .ksplat | Blocked & spatially sorted binary chunks | Quantized GS attributes | Progressive streaming in web context | https://deepwiki.com/mkkellogg/GaussianSplats3D/3.3-splat-and-ksplat-loading |
| Qualcomm Lightweight Gaussian Coding (L-GSC) | .lgsc | Lightweight GS entropy coding | Position, scale, rotation, opacity, SH | On-device & mobile | https://github.com/qualcomm/lite-3Dgsplat-codec |
| glTF Gaussian Splatting (KHR_gaussian_splatting) | .gltf / .glb | glTF 2.0 GS extension (point primitives) | Position, scale, rotation, opacity, SH | Interchange, engines, geospatial content | https://github.com/KhronosGroup/glTF/blob/main/extensions/2.0/Khronos/KHR_gaussian_splatting/README.md |
| glTF + SPZ Compression | .glb | glTF extension referencing SPZ payloads | Compressed GS baseline | Streaming & delivery On-device & mobile | https://community.cesium.com/t/create-khr-spz-gaussian-splats-compression-tooling/41103 |

These representation formats could be grouped in 4 different categories:

- single-blob binary representations, in which the coded Gaussian Splat data is stored as one contiguous payload,

- structured binary representations, in which the coded data may be organized as blocks, chunks, or non-contiguous segments internal to the representation format,

- image-backed or image-assisted representations, in which some coded parameters are stored in image-like resources combined with metadata (such as JSON), and

- encapsulated interchange representations, in which Gaussian Splat data is carried within a higher-level scene or asset format (e.g. glTF).

Each of these categories could be considered as different mechanisms for ISOBMFF encapsulation of Gaussian splats to enable creation of a dual-use and backwards compatible ISOBMFF compliant file, such as a HEIF file that contains an image and a Gaussian splat representation.

# 2bis	Analysis of possible interaction between an external 3DGS format and an MPEG System format

| Figure 1 - Scenario 1 | Figure 2 - Scenario 2 | Figure 3 - Scenario 3 |
| --- | --- | --- |

## Scenario 1: Same-level delivery

In this scenario, the XYZ format and the proposed HEIF format operate at the same level of 3DGS file delivery. Both serve as standalone file formats for storing and delivering Gaussian Splat content. The XYZ format files can be converted to and from HEIF format. Those conversions may not be fully reversible in terms of Gaussian data and metadata preservation.

## Scenario 2: XYZ format embedded in HEIF

In this scenario, HEIF acts as a container for SOG content, preserving the SOG file structure within the HEIF container. The characteristics include HEIF acting as container for XYZ format content, XYZ format images stored as HEIF items, and other metadata stored as HEIF metadata. The use case involves packaging existing XYZ format content in HEIF for compatibility with HEIF viewers and leveraging HEIF's single-file delivery while maintaining SOG structure.

## Scenario 3: HEIF as XYZ format elements

In this scenario, HEIF files constitute parts of XYZ format content, where individual XYZ format images are stored as HEIF files. This implies HEIF files constituting parts of XYZ format content, individual images (e.g., position image) stored as HEIF files, potential efficiency gains from HEIF compression, and XYZ format structure built from HEIF elements.

# 3	General storage principles for backward compatibility and dual-use

As a result of the expert discussions after MPEG#154, the following general principals could be listed:

3.1 . Usage of brands:

One trivial mechanism for such a dual-use of an ISOBMFF compliant file is via new brand definitions. Hence, for each storage mechanism, it may make sense to define a new brand to signal the presence of a particular storage mechanism. As an example, such a brand could be used in conjunction with a HEIF-relevant brand to enable a cover image (primary image item) and a Gaussian Splat representation stored and processed together.

3.2. Referencing Gaussian Splat representation from primary image item as an entry point:

Independent of the Gaussian Splat representation item’s type, an item reference (e.g. ‘gsil’ ) from the primary image item to the Gaussian Splat representation item would be sufficient to relate a cover item to the Gaussian Splat representation.

3.3. Gaussian Splat representation item types:

In MPEG#154, it was discussed that usage of auxiliary image items as a Gaussian splat representation entry item type could bring backward compatibility issues and may also not be suitable due to the 3D nature of the Gaussian Splats. The following list provides potential representation item types that were advised and discussed:

Depending on the category of the stored Gaussian Splat representation, the “entry point” item of a Gaussian Splat Representation can be one of the following item types:

- A MIME Item of a particular content_type (e.g. ‘application/octet-stream’): This may be used for a single-blob based storage of a Gaussian Splat representation for enabling a dual-use functionality, as well as when image-like attributes are stored (e.g. ‘application/image’). Which one to use may be defined based on the particular brand.

- A dedicated item with a specific 4CC code

- Usage of already-standardized item types and carriage mechanisms, for example, for video and image items.

If multiple items are required to represent a Gaussian Splat representation, a well-defined item reference type can be used.(e.g ‘gsst’ as provided in this document) from the entry point Gaussian Splat representation item to the other items that are required for the decoding and rendering purposes.

3.3. Gaussian Splat representation format related signaling:

Since there are multiple representation formats, independent of the encoding formats of the data itself, early identification of the Gaussian Splat representation format would be needed to invoke the correct media handler early enough to handover the representation handling functionality. For this purpose, signaling a 4CC code inside a common global configuration item was discussed.

# 4	Exploration on Single blob type of Gaussian Representation Storage

## 4.1 General

In this scenario, a Gaussian Splat representation is stored as a single blob of data. The main purpose is to enable dual-use and have a cover image associated with this representation.

- A new brand, ‘gsc2’ is defined to indicate such a single-blob-based storage.

- An item reference of type ‘gsil’ is used to associate the primary mage item with the Gaussian Splat Representation Item

- The Gaussian Splat Representation item is a MIME item. The content_type of the MIME item is set based on the payload itself and not relevant.

- GaussianSplatGlobalConfigurationProperty as defined in section X.2.3.1  is associated with the Gaussian Splat representation item, with a gs_4CC field indicating the representation format

The following diagram represents the explored approach:

## Example processing model

- Since the major brand is set to ‘heic’, an image viewer sees and processes this file as a HEIF compliant image file and displays the cover image item.

- An application that can further support the compatible brand ‘gsc2’, would further process the item reference and find the Gaussian splat representation item.

- By accessing and parsing the GaussianSplatGlobalConfigurationProperty, application would read the gs_4cc field, then invoke the proper media handler and pass the item payload for further Gaussian splat decoding, formatting and rendering.

# Ed. Note: Further include text for “image-backed or image-assisted representations”, such as SOG

# 2	Overview: GS as a HEIF profile

## 2.1 General

A Gaussian Splat representation is carried as an auxiliary image item associated with a conventional cover/master image item.

## 2.2	Carriage model

The primary item is a coded image item that acts as a 2D cover image for legacy HEIF viewers.

A Gaussian Splat (GS) representation item is an auxiliary image item linked to the cover image through an item reference of type 'auxl'. The GS representation item groups one or more GS stream items in the MetaBox or in one or more tracks identified by the GS representation item.

When GS data units are carried as items, each GS stream item carries one or more attribute sets and signals the role(s) through one or more item properties, enabling selective loading and parallel decoding.

When GS data units are carried as tracks, one or more attributes may be packed into samples of an auxiliary track or into samples of a track using an image or video coding format. Each compressed GS stream item signals the compressor used through an item property using a 4CC code. The design is independent of any specific compression algorithm.

## 2.3Role Assignment

The following is an example of roles composing a GS representation (based on the INRIA format for GS [4]). The roles may be conveyed in separate stream items or may be grouped together. When multiple roles are present in a single item or track sample, one instance of the role signalling is associated for each role.

- One stream item with role 'posi' contains position data for all points.

- One stream item with role 'rotq' contains rotation quaternions for all points.

- One stream item with role 'scl3' contains per point scale values.

- One stream item with role 'opac' contains per point opacity values.

- One stream item with role 'shco' with sh_band_index equal to 0 contains spherical harmonics band 0 coefficients, also called the DC term.

- One stream item with role 'shco' with sh_band_index equal to 1 contains spherical harmonics band 1 coefficients.

- Additional stream items with role 'shco' with sh_band_index equal to 2, 3, and so on, contain higher order spherical harmonics bands when present.

## 2.4 Example File Structures

The following examples show different examples of HEIF files carrying GS representations:

The first example shows a single GS image with one 2D cover image.

FileTypeBox 'ftyp': major-brand='heic', compatible-brands='gsc1, mif1, ...' MetaBox 'meta': HandlerBox 'hdlr': 'pict' PrimaryItemBox 'pitm': item_ID=1 ItemInfoBox 'iinf': entry_count=8 1) infe: item_ID=1, item_type='hvc1' // cover image 2) infe: item_ID=10, item_type='gspl' // GS master item (non-image) 3) infe: item_ID=11, item_type='gspc' // stream: position 4) infe: item_ID=12, item_type='gspc' // stream: rotation 5) infe: item_ID=13, item_type='gspc' // stream: opacity 6) infe: item_ID=14, item_type='gspc' // stream: SH band 0 (DC) 7) infe: item_ID=15, item_type='gspc' // stream: SH band 1 8) infe: item_ID=16, item_type='gspc' // stream: SH band 2 ItemLocationBox 'iloc': item_count=8 item_ID=1, extent_offset=P0, extent_length=Q0 item_ID=10, extent_offset=P10, extent_length=Q10 item_ID=11, extent_offset=P11, extent_length=Q11 ... item_ID=16, extent_offset=P16, extent_length=Q16 ItemReferenceBox 'iref': referenceType='gisl', from_item_ID=10, to_item_ID=1 // cover links to GS referenceType='gsst', from_item_ID=10, to_item_ID=11..16 // GS master points at streams referenceType='cdsc', from_item_ID=20, to_item_ID=1 // optional camera metadata item ItemPropertiesBox 'iprp': ItemPropertyContainerBox 'ipco': 'hvcC' 'ispe' // cover config and size 'auxC' // on item 10, identifies gaussian_splats via URI 'gsgC' // GS global config (numPoints, shDegree, coord system) 'gscC' // coding method config; compressor_code='gscu' indicates uncompressed 'gsrC' (x6) // per-stream role property, one per stream item 'rref' // optional, to require processing 'gsst' references ItemPropertyAssociation 'ipma': item_ID=1: essential=1 hvcC, essential=0 ispe item_ID=10: essential=1 auxC, essential=1 gsgC, essential=1 rref item_ID=11: essential=1 gscC, essential=1 gsrC(role='posi') item_ID=12: essential=1 gscC, essential=1 gsrC(role='xfrm') item_ID=13: essential=1 gscC, essential=1 gsrC(role='opac') item_ID=14: essential=1 gscC, essential=1 gsrC(role='shb0') item_ID=15: essential=1 gscC, essential=1 gsrC(role='shb1') item_ID=16: essential=1 gscC, essential=1 gsrC(role='shb2') MediaDataBox 'mdat' (or 'idat'): payloads at P0, P10..P16

The following example depicts a single Gaussian Splat frame that is packed and encoded using a video codec in a track that is referenced from a Gaussian Splat item.

FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1'), compatible-brands='gsc1, mif1, mp42, ...' MetaBox 'meta': HandlerBox 'hdlr': 'pict' PrimaryItemBox 'pitm': item_ID=1 ItemInfoBox 'iinf': entry_count=2 1) infe: item_ID=1, item_type='hvc1' // cover image 2) infe: item_ID=10, item_type='gspl' // GS representation item (non-image) ItemLocationBox 'iloc': item_ID=1, extent_offset=P0, extent_length=Q0 item_ID=10, extent_offset=P10, extent_length=Q10 // optional bootstrap data, may be zero length ItemReferenceBox 'iref': referenceType='gisl', from_item_ID=10, to_item_ID=1 ItemPropertiesBox 'iprp': ItemPropertyContainerBox 'ipco': 'hvcC' 'ispe' // cover config and size 'auxC' // on item 10, identifies gaussian_splats via aux_type URN 'gsgC' // GS global config (numPoints, shDegree, coord system) 'gstR' // points to track_ID=100 carrying packed GS data ItemPropertyAssociation 'ipma': item_ID=1: essential=1 hvcC, essential=0 ispe item_ID=10: essential=1 auxC, essential=1 gsgC, essential=1 gstR MovieBox 'moov': trak (track_ID=100) // Packed GS attribute track, encoded as video frames mdia hdlr: 'auxv' minf.stbl stsd: sample_entry: 'hvc1' (or any video coding sample entry) auxi: aux_track_type = "urn:mpeg:gaussian_splats:2026:auxid:1" gsrm: access_unit_sample_count=7 entry[0]: role_code='posi', sample_offset=0, sample_count=1, role_offset=0, role_length=Lpos entry[1]: role_code='rotq', sample_offset=1, sample_count=1, role_offset=0, role_length=Lrot entry[2]: role_code='scl3', sample_offset=2, sample_count=1, role_offset=0, role_length=Lscl entry[3]: role_code='opac', sample_offset=3, sample_count=1, role_offset=0, role_length=Lopac entry[4]: role_code='shco', sh_band_index=0, sample_offset=4, sample_count=1, role_offset=0, role_length=Lsh0 entry[5]: role_code='shco', sh_band_index=1, sample_offset=5, sample_count=1, role_offset=0, role_length=Lsh1 entry[6]: role_code='shco', sh_band_index=2, sample_offset=6, sample_count=1, role_offset=0, role_length=Lsh2 samples: F0, F1, F2, F3, F4, F5, F6 // one access unit representing a single GS frame MediaDataBox 'mdat': cover image item data and video-coded samples for track_ID=100

The following example shows a HEIF file with multiple views of a single Gaussian Splat frame from different camera views and stored as an image sequence.

FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1') compatible-brands='gsc1, mif1, mp42, ...' (and any codec brands used by 'pict' tracks) MovieBox 'moov': TrackBox 'trak' (track_ID=1) // Viewpoint 0 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V0_0, V0_1, V0_2, ... TrackBox 'trak' (track_ID=2) // Viewpoint 1 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V1_0, V1_1, V1_2, ... TrackBox 'trak' (track_ID=3) // Viewpoint 2 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V2_0, V2_1, V2_2, ... TrackBox 'trak' (track_ID=30) // timed camera intrinsics/extrinsics for viewpoint 0 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=1 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C0_0, C0_1, C0_2, ... TrackBox 'trak' (track_ID=31) // timed camera intrinsics/extrinsics for viewpoint 1 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=2 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C1_0, C1_1, C1_2, ... TrackBox 'trak' (track_ID=32) // timed camera intrinsics/extrinsics for viewpoint 2 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=3 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C2_0, C2_1, C2_2, ... // Split-stream GS non-image tracks shared by all viewpoints TrackBox 'trak' (track_ID=10) // GS positions TrackReferenceBox 'tref': track_reference_type 'gisl' -> track_ID=1, track_ID=2, track_ID=3 MediaBox 'mdia': HandlerBox 'hdlr': 'auxv' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'gspc' AuxiliaryTypeInfoBox 'auxi': aux_track_type="urn:mpeg:gaussian_splats:2026:auxid:1" GaussianSplatCompressionConfigurationProperty 'gscC': compressor_code='gscu' (optional) GaussianSplatTrackRoleMapBox 'gsrm' (not needed when the track conveys only one role) (optional) role signalling in sample entry, e.g. role_code='posi'

# 3	Proposed Specification text

# Definitions

- Gaussian Splat: a parametric primitive representing scene radiance as an oriented anisotropic Gaussian with associated appearance parameters.

- Gaussian Splat image: an image representation that comprises a set of Gaussian Splats intended for rendering one or more 2D views of a scene.

- Gaussian Splat representation: the complete set of data needed to decode and render a Gaussian Splat image, including geometry, appearance, and any required metadata.

- Gaussian Splat representation item: a non-image item of type 'gspl' that carries or identifies a Gaussian Splat representation and that is the target of an item reference of type 'gisl' from a conventional image item, declaring the conventional image item as a 2D cover for the Gaussian Splat representation.

- Gaussian Splat representation item: an item of type 'gspl' that identifies a Gaussian Splat representation and provides references to one or more Gaussian Splat stream items and optionally to one or more Gaussian Splat data tracks.

- Gaussian Splat stream item: an item of type 'gspc' that carries a coded byte sequence corresponding to one or more Gaussian Splat streams, where the coding method is identified by an associated GaussianSplatCompressionConfigurationProperty ('gscC').

- compressor_code: a four character code that identifies the coding method used for a Gaussian Splat stream item or Gaussian Splat data track. The value 'gscu' indicates uncompressed stream data.

- Gaussian Splat data track: a track that carries coded Gaussian Splat data units, optionally using an image or video coding format, and signals the coding method using compressor_code.

- Spherical harmonics degree: the maximum spherical harmonics order represented by the spherical harmonics coefficients of a Gaussian Splat representation.

- Spherical harmonics band: a subset of spherical harmonics coefficients that share the same order and are signaled as a distinct component of the representation.

# Annex X
(normative)
Gaussian Splat Image File Format

## X.1 General

This annex derives a format to encapsulate Gaussian Splat representations in files conforming to the Image File Format specified in this document. The Gaussian Splat representation enables view synthesis and photorealistic rendering from novel viewpoints by storing a set of 3D Gaussian primitives and associated appearance parameters.

One profile is specified in this annex. The coding method used for each Gaussian Splat stream item is signalled by a 4CC compressor_code in a GaussianSplatCompressionConfigurationProperty ('gscC'), including a value that indicates uncompressed stream data.

## X.2 Gaussian Splat images and image collections

### X.2.1 General

Clause X.2 specifies requirements for files containing Gaussian Splat representation items. When a brand specified in X.4.1 is among the compatible brands of a file, the requirements specified in Clause X.2 shall be applied.

The specifications of Clause 6 apply.

### X.2.2 Gaussian Splat representation items

#### X.2.2.1 Representation item of type 'gspl'

An item of type 'gspl' is a Gaussian Splat representation item.

Each Gaussian Splat representation item of type 'gspl' shall be declared as a non- image item. The AuxiliaryTypeProperty ('auxC') shall not be associated with a Gaussian Splat representation item.

Each Gaussian Splat representation item of type 'gspl' shall be the target of an item reference of type ‘gisl’ from a conventional image item that serves as the 2D cover.

Each Gaussian Splat representation item of type 'gspl' shall group one or more item Gaussian Splat data units. The data units shall be carried either as one or more Gaussian Splat stream items referenced using an item reference of type 'gsst' as specified in X.2.2.2, or in one or more tracks identified by associated GaussianSplatTrackReferenceProperty ('gstR ') as specified in X.2.3.3, or both.

#### X.2.2.2 Gaussian Splat stream items

A Gaussian Splat stream item contains the data for one or more Gaussian Splat streams. Each role present in the stream item shall be indicated by an associated GaussianSplatRoleProperty ('gsrC') as specified in X.2.3.1. When more than one role is present, multiple 'gsrC' item properties shall be associated with the stream item, one for each role.

Gaussian Splat stream items shall be referenced by exactly one Gaussian Splat representation item of type 'gspl' using an item reference of type 'gsst'.

##### X.2.2.2.1 Stream item of type 'gspc'

An item of type 'gspc' consists of a coded representation of one or more Gaussian Splat streams. The coding method used for the stream item shall be signalled by an associated GaussianSplatCompressionConfigurationProperty ('gscC') as specified in X.2.3.2.

The decoded stream data for each role present in the stream item shall conform to the data layouts specified in X.2.5. Each role present in the stream item shall be indicated by one or more associated GaussianSplatRoleProperty ('gsrC') item properties as specified in X.2.3.1.

### X.2.3 Gaussian Splat item properties

#### X.2.3.1 Gaussian Splat global configuration property

##### X.2.3.1.1	Definition

Box type: gsgC
Property type: Descriptive item property
Container: ItemPropertyContainerBox
Mandatory (per item): Yes, for a Gaussian Splat representation item of type gspl
Quantity (per item): One for a Gaussian Splat representation item of type gspl

The GaussianSplatGlobalConfigurationProperty specifies configuration parameters that apply to the associated Gaussian Splat representation as a whole, including the number of points, spherical harmonics degree, component format for uncompressed streams, and coordinate system flags.

essential shall be equal to 1 for a gsgC item property associated with a Gaussian Splat representation item of type gspl.

##### X.2.3.1.2	Syntax

aligned(8) class GaussianSplatGlobalConfigurationProperty

extends ItemFullProperty('gsgC', version == 0, flags == 0) {

unsigned int(32) gs_4cc;

unsigned int(8)  gs_version;

unsigned int(8)  gs_flags;

reserved bit(16) reserved;

unsigned int(32) num_points;

unsigned int(8)  sh_degree;

unsigned int(8)  component_format;

unsigned int(16) reserved2;

}

##### X.2.3.1.3	Semantics

gs_4cc specifies the 4CC code of the Gaussian splat Representation Format. This 4CC may be used to invoke the proper media handler to invoke, decode and render the Gaussian splat representation.

gs_version specifies the version of the Gaussian Splat global configuration. For this version of this annex, gs_version shall be equal to 0.

gs_flags provides flags that define the coordinate system and interpretation of values in the data streams as defined in the following table.

Table X.1 gs_flags values for GaussianSplatGlobalConfigurationProperty

| Bits | Description |
| --- | --- |
| 0 | when equal to 1, the coordinate system is right handed, when equal to 0, it is left handed. |
| 1 | when equal to 1, positions and scales are expressed in meters, when equal to 0, units are unspecified. |
| 2 | when equal to 1, quaternion components are ordered (qx, qy, qz, qw), when equal to 0, ordering is (qw, qx, qy, qz). |
| 3..15 | reserved, set to 0. |

num_points specifies the number of points in the Gaussian Splat representation.

sh_degree specifies the maximum spherical harmonics degree present in the Gaussian Splat representation.

component_format specifies the component representation used for uncompressed role byte sequences, i.e. when compressor_code is equal to gscu. For this version of this annex, component_format equal to 0 indicates IEEE 754 binary32. Other values are reserved for future use.

reserved and reserved2 shall be set to 0.

#### X.2.3.2 Gaussian Splat role property

##### X.2.3.2.1 Definition

Box type: 				'gsrC'

Property type: 			Descriptive item property

Container: 			ItemPropertyContainerBox

Mandatory (per item):		Yes, for a Gaussian Splat stream item

Quantity (per item): 		One or more for a Gaussian Splat stream item

The GaussianSplatRoleProperty identifies a role present in a Gaussian Splat stream item and, when applicable, parameters such as spherical harmonics band index, stream fragmentation information, and the byte range within the decoded byte sequence corresponding to the role.

essential shall be equal to 1 for a 'gsrC' item property associated with a Gaussian Splat stream item.

##### X.2.3.2.2 Syntax

aligned(8) class GaussianSplatRoleProperty

extends ItemFullProperty('gsrC', version == 0, flags == 0){

unsigned int(32) role_code;

unsigned int(16) role_instance;

unsigned int(16) role_count;

unsigned int(16) sh_band_index;

reserved bit(16) reserved;

unsigned int(32) role_offset;

unsigned int(32) role_length;

}

##### X.2.3.2.3 Semantics

role_code specifies the role identified by this property. The role_code value shall be one of the values specified in Table X.1, unless otherwise specified by an extension of this annex.

role_instance and role_count specify fragmentation of a role across multiple stream items. When role_count is greater than 1, the complete role stream is formed by concatenating the role byte sequences of the fragments in increasing role_instance order. When role_count is equal to 1, role_instance shall be equal to 0.

sh_band_index specifies the spherical harmonics band index when role_code indicates spherical harmonics coefficients. For roles other than spherical harmonics coefficients, sh_band_index shall be set to 0xFFFF.

role_offset specifies the zero-based byte offset from the start of the decoded byte sequence of the associated stream item to the first byte of the role byte sequence identified by this property.

role_length specifies the length in bytes of the role byte sequence identified by this property.

For a given stream item, the byte ranges indicated by 'gsrC' item properties associated with the stream item shall not overlap, unless otherwise specified by an extension of this annex.

Table X.1 – role_code values for GaussianSplatRoleProperty

| role_code | Semantics |
| --- | --- |
| posi | Position of each Gaussian, expressed as three components per point. |
| rotq | Orientation of each Gaussian, expressed as four components per point (quaternion). |
| scl3 | Scale of each Gaussian, expressed as three components per point. |
| opac | Opacity of each Gaussian, expressed as one component per point. |
| shco | Spherical harmonics coefficients. The sh_band_index field indicates which band is present in the stream item. |

#### X.2.3.3 Gaussian Splat compression configuration property

##### X.2.3.3.1 Definition

Box type:	 		'gscC'

Property type: 		Descriptive item property

Container: 			ItemPropertyContainerBox

Mandatory (per item): 	Yes, for a Gaussian Splat stream item of type 'gspc'

Quantity (per item): 	One for a Gaussian Splat stream item of type 'gspc'

The GaussianSplatCompressionConfigurationProperty identifies the coding method used for an item of type 'gspc' and provides parameters required to initialize a decompressor.

essential shall be equal to 1 for a 'gscC' item property associated with a Gaussian Splat stream item of type 'gspc'.

##### X.2.3.3.3 Syntax

aligned(8) class GaussianSplatCompressionConfigurationProperty

extends ItemFullProperty('gscC', version == 0, flags == 0) {

unsigned int(32) compressor_code;

unsigned int(32) compressor_parameter_length;

unsigned int(8) compressor_parameter[compressor_parameter_length];

}

##### X.2.3.3.3 Semantics

compressor_code is a four-character code identifying the coding method used for the item of type 'gspc'. The interpretation of compressor_parameter is defined by the identified coding method. When compressor_code is equal to 'gscu', the stream item is uncompressed, compressor_parameter_length shall be equal to 0, and the decoded byte sequence shall be identical to the stored byte sequence.

#### X.2.3.4 Gaussian Splat track reference property

##### X.2.3.4.1 Definition

Box type:		'gstR'

Property type:		Descriptive item property

Container:		ItemPropertyContainerBox

Mandatory (per item):   Conditional, for a Gaussian Splat representation item of type 'gspl' that identifies one or more tracks carrying Gaussian Splat data

Quantity (per item):	One for a Gaussian Splat representation item of type 'gspl' that identifies tracks

The GaussianSplatTrackReferenceProperty identifies one or more tracks in the file that carry Gaussian Splat data units for the associated Gaussian Splat representation item.

essential shall be equal to 1 for a 'gstR' item property associated with a Gaussian Splat representation item of type 'gspl'.

##### X.2.3.4.2 Syntax

aligned(8) class GaussianSplatTrackReferenceProperty

extends ItemFullProperty('gstR', version == 0, flags == 0) {

unsigned int(16) track_count;

reserved bit(16) reserved;

unsigned int(32) track_ID[track_count];

}

##### X.2.3.4.3 Semantics

track_count specifies the number of track_ID values that follow.

track_ID identifies a track by the track_ID in the TrackHeaderBox of that track. Each referenced track shall be present in the same file and shall carry Gaussian Splat data units as specified in X.3.

When a 'gstR' item property is associated with an item of type 'gspl', the item may omit any item references of type 'gsst'. When both 'gstR' and 'gsst' are present, the identified data units shall be treated as components of the same Gaussian Splat representation.

### X.2.4 Item reference type ‘gisl’

#### X.2.4.1 General

The item reference type 'gisl' identifies a Gaussian Splat representation item of type 'gspl' associated with a conventional image item used as a 2D cover. In an item reference of type 'gisl', the from_item_ID shall identify the conventional image item that serves as the 2D cover, and the to_item_ID shall identify the Gaussian Splat representation item. A reader that does not recognize the 'gisl' reference shall ignore it. A reader that does not recognize the 'gspl' item type shall ignore the Gaussian Splat representation item.

### X.2.5 Uncompressed stream data layout

The uncompressed stream data layout is role specific and is defined by the role_code in the associated GaussianSplatRoleProperty. When a stream item contains more than one role, role_offset and role_length indicate the location of each role byte sequence within the decoded byte sequence of the stream item.

Unless otherwise stated in this annex, each component in the uncompressed stream data shall be represented as IEEE 754 binary32 in little endian byte order.

For role_code values specified in Table X.1, the following layouts apply for each point, in the order of points from 0 to num_points − 1:

- 'posi': three components (x, y, z).

- 'rotq': four components (qx, qy, qz, qw).

- 'scl3': three components (sx, sy, sz).

- 'opac': one component (opacity).

- 'shco': an array of coefficient triplets for the signalled sh_band_index. The number of coefficient triplets for a given band index shall be as specified by the spherical harmonics degree and the coding method.

### X.2.6 Item reference type 'gsst'

The item reference type 'gsst' identifies Gaussian Splat stream items that belong to a Gaussian Splat representation item of type 'gspl'.

In an item reference of type 'gsst', the from_item_ID shall identify the Gaussian Splat representation item and the to_item_ID values shall identify Gaussian Splat stream items.

## X.3 Gaussian Splat image sequences

### X.3.1 General

Gaussian Splat representations may be carried as timed sequences. The specifications of Clause 7 apply.

A timed Gaussian Splat sequence may be carried in one or more tracks. When a timed Gaussian Splat sequence is present, it is recommended to provide a viewable image sequence track and to associate the Gaussian Splat sequence to the viewable track using the mechanisms specified in Clause 7 for auxiliary tracks.

NOTE: This annex is compression agnostic. Image or video coding formats may be used to carry Gaussian Splat data units in tracks, provided that the roles and the mapping to the uncompressed stream data layouts specified in X.2.5 are signaled.

### X.3.2 Gaussian Splat data tracks

#### X.3.2.1 General

Gaussian Splat data units may be carried in a Gaussian Splat data track. A Gaussian Splat data track shall be an auxiliary image sequence track with handler type 'auxv' and shall signal its auxiliary track type using an AuxiliaryTypeInfoBox ('auxi') contained in the sample entry, as specified in Clause 7.

A Gaussian Splat data track may be associated with one or more master image sequence tracks using a track reference of type 'auxl' as specified in Clause 7. A Gaussian Splat data track may also be identified by a Gaussian Splat representation item of type 'gspl' using the GaussianSplatTrackReferenceProperty ('gstR') as specified in X.2.3.4.

A Gaussian Splat data track may convey one or more roles. When a Gaussian Splat data track conveys more than one role, or when the order of roles cannot be derived from other information, the mapping of roles to samples within a Gaussian Splat access unit shall be provided by a GaussianSplatTrackRoleMapBox ('gsrm') as specified in X.3.2.1.

#### X.3.2.1 Gaussian Splat track role map box

##### X.3.2.1.1 Definition

Box type:	'gsrm'

Container:	Sample entry box

Mandatory:	Conditional, when a Gaussian Splat data track conveys more than one role or when an explicit mapping of roles to samples is needed

Quantity:	One

The GaussianSplatTrackRoleMapBox identifies the roles conveyed by a Gaussian Splat data track and provides the mapping of each role to one or more samples within a Gaussian Splat access unit.

##### X.3.2.1.2 Syntax

aligned(8) class GaussianSplatTrackRoleMapBox

extends FullBox('gsrm', version == 0, flags == 0){

unsigned int(16) access_unit_sample_count;

unsigned int(16) entry_count;

for(i=0; i<entry_count; i++){

unsigned int(32) role_code;

unsigned int(16) sh_band_index;

reserved bit(16) reserved;

unsigned int(16) sample_offset;

unsigned int(16) sample_count;

unsigned int(32) role_offset;

unsigned int(32) role_length;

}

}

##### X.3.2.1.3 Semantics

access_unit_sample_count specifies the number of consecutive samples that form one Gaussian Splat access unit. When access_unit_sample_count is equal to 1, each sample corresponds to one access unit.

entry_count specifies the number of role mapping entries that follow.

role_code and sh_band_index specify the role and, when applicable, the spherical harmonics band index, using the same semantics as specified for GaussianSplatRoleProperty in X.2.3.2.

sample_offset specifies the zero-based index of the first sample within the access unit that contributes to the role. sample_count specifies the number of consecutive samples within the access unit that contribute to the role.

role_offset specifies the zero-based byte offset within the decoded byte sequence of each contributing sample to the first byte of the role byte sequence. role_length specifies the number of bytes contributed by each contributing sample. When sample_count is greater than 1, the complete role byte sequence is formed by concatenating role_length bytes from each contributing sample in increasing sample order, starting at role_offset.

## X.4 Gaussian Splat specific brands

### X.4.1 Gaussian Splat image and image collection brands

#### X.4.1.1 General

The brand 'gsc1' is specified in the following subclauses.

#### X.4.1.2 Requirements on files

Files shall include 'mif1' among the compatible brands and hence conform to the specifications in 10.2.1.1.

Additionally, there shall be at least one Gaussian Splat representation item of type 'gspl' present in the file that is the target of an item reference of type ‘gisl’ from a conventional image item in the same file. The Gaussian Splat representation item shall identify Gaussian Splat data units either by one or more item references of type 'gsst', or by an associated 'gstR' item property, or both.

When the 'gsc1' brand is among the compatible brands, Gaussian Splat stream items referenced by the Gaussian Splat representation item shall be of type 'gspc'. Each stream item shall have an associated 'gscC' item property as specified in X.2.3.3. The value 'gscu' of compressor_code indicates uncompressed stream data. If the Gaussian Splat representation item identifies one or more tracks using an associated 'gstR' item property, each referenced track may carry Gaussian Splat data units using an image or video coding format. When a referenced track conveys more than one role, a 'gsrm' box shall be present in the sample entry as specified in X.3.2.1.

#### X.4.1.3 Requirements on readers

The requirements on readers specified in 10.2.1.2 shall be supported.

Readers for the 'gsc1' brand shall be able to parse a Gaussian Splat representation item of type 'gspl' and the identified data units and reconstruct the uncompressed stream data for each referenced role. Readers shall be able to parse the 'gscC' property and obtain the compressor_code and decoding parameters for each stream item. When compressor_code is equal to 'gscu', no decoding is required. When compressor_code identifies a supported coding method, a reader shall be able to decode the stream item and reconstruct the uncompressed stream data. When compressor_code identifies an unsupported coding method, a reader may ignore the corresponding stream item. If a Gaussian Splat representation item identifies one or more tracks using an associated 'gstR' item property, a reader shall be able to locate the referenced track(s) and, when present, parse the 'gsrm' box in the sample entry to determine the mapping of roles to samples.

NOTE: A file including a Gaussian Splat representation is required to include a viewable master image item conforming to 'mif1'. A reader that does not support Gaussian Splat rendering can still display the master image.

# 4	Extracted 4CCs

| 4CC | Clause | First Occurrence Paragraph |
| --- | --- | --- |
| gisl | 23008-12 | A Gaussian Splat (GS) representation item is an auxiliary image item linked to the cover image through an item reference of type 'gisl'... |
| scl3 | 23008-12 | One stream item with role 'scl3' contains per point scale values. |
| gsrC | X.2.2.2 Gaussian Splat stream items | A Gaussian Splat stream item ... ('gsrC') ... |
| gspc | X.2.3.3 Semantics | compressor_code is a four character code identifying the compression method used for the item of type 'gspc'... |
| gspl | X.2.3.4 Gaussian Splat track reference property | Mandatory (per item): Conditional, for a Gaussian Splat representation item of type 'gspl'... |
| gstR | X.2.3.4 Syntax | extends ItemFullProperty('gstR', version = 0, flags = 0)... |
| gsst | X.2.3.4 Semantics | When a 'gstR' item property ... 'gsst' ... |
| auxi | X.3.2 Gaussian Splat data tracks | Gaussian Splat data units ... AuxiliaryTypeInfoBox ('auxi') ... |
| gsrm | X.3.2 Gaussian Splat data tracks | A Gaussian Splat data track ... GaussianSplatTrackRoleMapBox ('gsrm') ... |
| gsc1 | X.4.1.1 General | The brands 'gsu1' and 'gsc1' are specified... |
| gsu1 | X.4.1.1 General | The brands 'gsu1' and 'gsc1' are specified... |
| mif1 | X.4.1.2 Requirements on files | Files shall include 'mif1' among the compatible brands... |
| gspu | X.4.1.2 Requirements on files | When the 'gsu1' brand ... type 'gspu'... |
| gscC | X.4.1.2 Requirements on files | Each stream item of type 'gspc' ... 'gscC' ... |
| meta | 2.4 Examples | MetaBox 'meta': |
| hvc1 | 2.4 Examples | 1) infe: item_ID=1, item_type='hvc1'... |
| ipco | 2.4 Examples | ItemPropertyContainerBox 'ipco': |
| shb0 | 2.4 Examples | item_ID=14: essential=1 gsrC(role='shb0') |
| shb1 | 2.4 Examples | item_ID=15: essential=1 gsrC(role='shb1') |
| shb2 | 2.4 Examples | item_ID=16: essential=1 gsrC(role='shb2') |
| ftyp | 2.4 Examples | FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1')... |
| heic | 2.4 Examples | FileTypeBox 'ftyp': major-brand='heic'... |
| hdlr | 2.4 Examples | HandlerBox 'hdlr': 'pict' |
| pict | 2.4 Examples | HandlerBox 'hdlr': 'pict' |
| pitm | 2.4 Examples | PrimaryItemBox 'pitm': item_ID=1 |
| iinf | 2.4 Examples | ItemInfoBox 'iinf': entry_count=2 |
| iloc | 2.4 Examples | ItemLocationBox 'iloc': |
| iref | 2.4 Examples | ItemReferenceBox 'iref': |
| iprp | 2.4 Examples | ItemPropertiesBox 'iprp': |
| hvcC | 2.4 Examples | 'hvcC' 'ispe' // cover config and size |
| ispe | 2.4 Examples | 'hvcC' 'ispe' // cover config and size |
| auxC | 2.4 Examples | 'auxC' // identifies gaussian_splats via aux_type URN |
| gsgC | 2.4 Examples | 'gsgC' // GS global config |
| ipma | 2.4 Examples | ItemPropertyAssociation 'ipma': |
| moov | 2.4 Examples | MovieBox 'moov': |
| auxv | 2.4 Examples | hdlr: 'auxv' |
| posi | 2.4 Examples | entry[0]: role_code='posi'... |
| rotq | 2.4 Examples | entry[1]: role_code='rotq'... |
| opac | 2.4 Examples | entry[3]: role_code='opac'... |
| shco | 2.4 Examples | entry[4]: role_code='shco'... |
| mdat | 2.4 Examples | MediaDataBox 'mdat': |
| mp42 | 2.4 Examples | compatible_brands: 'mif1', 'mp42'... |
| av01 | 2.4 Examples | sample_entry: 'hvc1' (or 'av01',... |
| vvc1 | 2.4 Examples | sample_entry: 'hvc1' (or 'av01', 'vvc1',...) |

We also checked MP4RA, and none of the proposed new 4CCs highlighted in red bold above are registered in MP4RA.

We also ran the SDL through the https://mpeggroup.github.io/mpeg-sdl-editor/ validator for any bugs.

# 5 	Example

Here is a detailed example with SPZ:

FileTypeBox 'ftyp':

major_brand = 'heic'

minor_version = 0

compatible_brands[] = { 'gsc1', 'mif1', 'heic', 'mp42' }

MetaBox 'meta':

FullBox header:

version = 0

flags = 0

HandlerBox 'hdlr':

version = 0

flags = 0

pre_defined = 0

handler_type = 'pict'

reserved[3] = 0

name = "Picture handler\0"

PrimaryItemBox 'pitm':

version = 0

flags = 0

item_ID = 1

ItemInfoBox 'iinf':

version = 0

flags = 0

entry_count = 3

ItemInfoEntry 'infe' (for item_ID=1):

version = 2

flags = 0

item_ID = 1

item_protection_index = 0

item_type = 'hvc1'

item_name = "CoverImage\0"

ItemInfoEntry 'infe' (for item_ID=10):

version = 2

flags = 0

item_ID = 10

item_protection_index = 0

item_type = 'gspl'

item_name = "GaussianSplatRepresentation\0"

ItemInfoEntry 'infe' (for item_ID=11):

version = 2

flags = 0

item_ID = 11

item_protection_index = 0

item_type = 'gspc'

item_name = "GaussianSplatStream_SPZ\0"

ItemLocationBox 'iloc':

version = 0

flags = 0

offset_size = 4

length_size = 4

base_offset_size = 8

index_size = 0

item_count = 3

item_ID = 1:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00001000     // P1

extent_length = 0x0001F000     // Q1

item_ID = 10:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00000000     // P10

extent_length = 0x00000000     // Q10

item_ID = 11:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00020000     // P11

extent_length = 0x00009400     // Q11

ItemReferenceBox 'iref':

version = 0

flags = 0

SingleItemTypeReferenceBox 'auxl':

from_item_ID = 10

reference_count = 1

to_item_ID[0] = 1

SingleItemTypeReferenceBox 'gsst':

from_item_ID = 10

reference_count = 1

to_item_ID[0] = 11

ItemPropertiesBox 'iprp':

ItemPropertyContainerBox 'ipco':

property[1] = HEVCConfigurationBox 'hvcC':

configurationVersion = 1

general_profile_space = 0

general_tier_flag = 0

general_profile_idc = 1

general_level_idc = 120

nalArray[] = (HEVC config payload)

property[2] = ImageSpatialExtentsProperty 'ispe':

version = 0

flags = 0

image_width = 1920

image_height = 1080

property[3] = AuxiliaryTypeProperty 'auxC':

version = 0

flags = 0

aux_type = "urn:mpeg:gaussian_splats:2026:auxid:1\0"

aux_subtype = GaussianSplatAuxConfigSubType:

gs_version = 0

gs_flags = 0x0001                 // bit0 = right-handed coordinate system

reserved = 0

num_points = 1000                 // 0x000003E8

sh_degree = 2

component_format = 0              // float32

reserved2 = 0

property[4] = GaussianSplatGlobalConfigurationProperty 'gsgC':

version = 0

flags = 0

num_points = 1000

sh_degree = 2

component_format = 0

coord_system = 1                    // 1 = right-handed

reserved = 0

property[5] = GaussianSplatCompressionConfigurationProperty 'gscC':

version = 0

flags = 0

compressor_code = 'spz '            // 0x73707A20

compressor_parameter_length = 8

compressor_parameter[8] = {

0x00,                             // cfg_version = 0

0x03,                             // spz_version_policy = require v3

0x02,                             // input_coord_system = RDF

0x01,                             // output_coord_system = RUB

0x01,                             // flags: bit0 require_magic = 1

0x00, 0x00, 0x00                  // reserved

}

property[6] = GaussianSplatRoleProperty 'gsrC' for role posi:

version = 0

flags = 0

role_code = 'posi'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 0

role_length = 12000                 // 1000 points * 3 float32

property[7] = GaussianSplatRoleProperty 'gsrC' for role rotq:

version = 0

flags = 0

role_code = 'rotq'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 12000

role_length = 16000                 // 1000 * 4 float32

property[8] = GaussianSplatRoleProperty 'gsrC' for role scl3:

version = 0

flags = 0

role_code = 'scl3'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 28000

role_length = 12000                 // 1000 * 3 float32

property[9] = GaussianSplatRoleProperty 'gsrC' for role opac:

version = 0

flags = 0

role_code = 'opac'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 40000

role_length = 4000                  // 1000 * 1 float32

property[10] = GaussianSplatRoleProperty 'gsrC' for role shco band 0:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 0

reserved = 0

role_offset = 44000

role_length = 12000                 // band0 has 1 coeff triplet per point, 3 float32 per triplet

property[11] = GaussianSplatRoleProperty 'gsrC' for role shco band 1:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 1

reserved = 0

role_offset = 56000

role_length = 36000                 // band1 has 3 coeff triplets per point

property[12] = GaussianSplatRoleProperty 'gsrC' for role shco band 2:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 2

reserved = 0

role_offset = 92000

role_length = 60000                 // band2 has 5 coeff triplets per point

ItemPropertyAssociation 'ipma':

version = 0

flags = 0

entry_count = 3

entry for item_ID=1:

association_count = 2

assoc[0]:

essential = 1

property_index = 1    // hvcC

assoc[1]:

essential = 0

property_index = 2    // ispe

entry for item_ID=10:

association_count = 2

assoc[0]:

essential = 1

property_index = 3    // auxC

assoc[1]:

essential = 1

property_index = 4    // gspC

entry for item_ID=11:

association_count = 8

assoc[0]:

essential = 1

property_index = 5    // gscC compressor_code='spz '

assoc[1]:

essential = 1

property_index = 6    // gsrC posi

assoc[2]:

essential = 1

property_index = 7    // gsrC rotq

assoc[3]:

essential = 1

property_index = 8    // gsrC scl3

assoc[4]:

essential = 1

property_index = 9    // gsrC opac

assoc[5]:

essential = 1

property_index = 10   // gsrC shco band0

assoc[6]:

essential = 1

property_index = 11   // gsrC shco band1

assoc[7]:

essential = 1

property_index = 12   // gsrC shco band2

MediaDataBox 'mdat':

at file offset 0x00001000:

item_ID=1 bytes, length 0x0001F000

at file offset 0x00020000:

item_ID=11 bytes, length 0x00009400, contents:

gzip stream that is an SPZ bitstream

SPZ header and payload are opaque to HEIF

decompression produces a decoded byte sequence of length 152000 bytes

the role_offset and role_length in gsrC point into that decoded sequence

# 6	Discussion during MPEG meetings:

# 6.1 MPEG#153

During MPEG#153, online and gitlab based discussions happened. The gitlab discussions are here.

A summary of the questions are as follows

## A. Conceptual Scope & Placement

- Should GS be part of HEIF or be restricted to volumetric media standards (23090-10 / 18)?

- Does treating GS as an “image” stretch HEIF beyond its intended scope?

- If both domains support GS, how do we avoid fragmentation and ensure interoperability?

## B. Technical Model & Semantics

- Should GS representations really be modeled as auxiliary image items?

  - If so, what are the semantics of image dimensions (ispe) for 3D content?

- Is the use of role_offset and role_length appropriate or overly constraining?

  - Does it assume too much about the decoded GS structure?

- Should track referencing (gstR) be permitted within image items, or is this mixing incompatible mechanisms?

## C. Compatibility with Existing Codecs

- How to support GS codecs that internally use video/image coding formats (e.g., SOG) without duplicating existing signaling?

- Should new 4CCs be defined for codecs that already have registered sample entries?

## D. Real-World Deployment Concerns

- Are current GS codecs mature enough to define a generic long-term container?

- Do GS formats share enough structural commonality to justify a unified HEIF-based framework?

## E. Spec Evolution & Clarity

- Should the definitions of gspl, gspc, gscC, gsrC, etc., be simplified or aligned more closely with ISOBMFF semantics?

- Is more alignment needed with existing HEIF “image item” and “coded image” concepts?

# 6.2 MPEG#154

During MPEG#154, several comments were received in the following Jira issues:

- m76764 [ISOBMFF] GS in HEIF: Identified Issues and proposed specification changes (#437) · Issues · MPEG / Systems / File Format / ISOBMFF · GitLab

- m76763 [ISOBMFF] Implementation Report and Usage Guide for Gaussian Splats in HEIF (#436) · Issues · MPEG / Systems / File Format / ISOBMFF · GitLab

- m76687 [WG3][Explorations] Possible interactions with HEIF and other GS content formats (#228) · Issues · MPEG / Systems / File Format / HEIF · GitLab

- m76612 [WG3][Explorations] On Exploration of carriage of static Gaussian Splat in HEIF (#225) · Issues · MPEG / Systems / File Format / HEIF · GitLab

Based on the discussions:

- Storage of existing gaussian splat representations as-is is added to the exploration document

- SOG-like 3DGS representations are added to the exploration document

- Usage of auxiliary images as gaussian splat representations is discouraged and current exploration document is updated accordingly

# 7	References

[1]	ISO/IEC 23008-12:2024, Information technology - High efficiency coding and media delivery in heterogeneous environments - Part 12: Image File Format.

[2]	ISO/IEC 14496-12:2022, Information technology - Coding of audio-visual objects - Part 12: ISO base media file format.

[3]	RFC 3986, Uniform Resource Identifier (URI): Generic Syntax.

[4]	Kerbl et al., “3D Gaussian Splatting for Real-Time Radiance Field Rendering”

[5] 	m76612, “[WG3][Explorations] On Exploration of carriage of static Gaussian Splat in HEIF”, MPEG#154

[6] 	m76687, “[WG3][Explorations] Possible interactions with HEIF and other GS content formats”, MPEG#154

[7] 	m76612, “On Exploration of carriage of static Gaussian Splat in HEIF”, MPEG#154

[8] 	m76687, “Possible interactions with HEIF and other GS content formats”, MPEG#154

[9] 	m76764, “GS in HEIF: Identified Issues and proposed specification changes”, MPEG#154

[10] 	m76763, “ Implementation Report and Usage Guide for Gaussian Splats in HEIF”, MPEG#154