ISO/IEC JTC 1/SC 29/AG 5 N199

ISO/IEC JTC 1/SC 29/AG 5

MPEG Visual Quality Assessment

Convenorship: DE

Document type:	Projects

Title:	Report on dry-run expert viewing for test sequences under consideration for the JVET CfP on video compression with capability beyond VVC

Status:	Approved

Date of document:	2026-06-22

Source:	ISO/IEC JTC 1/SC 29/AG 5

Expected action:	Information

Action due date:	None

No. of pages:	13 (with cover page)

Email of Convenor:	wien@lfb.rwth-aachen.de

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29ag5

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/AG 5

MPEG VISUAL QUALITY ASSESSMENT

ISO/IEC JTC 1/SC 29/AG 5 N 199

Santa Eularia, ES, April 2026

| Title | Report on dry-run expert viewing for test sequences under consideration for the JVET CfP on video compression with capability beyond VVC |
| --- | --- |
| Source | MPEG Visual Quality Assessment |
| Editors | M. Wien, C. Lehmann, P. de Lagrange, J. Calic |
| Status | Approved |
| Serial Number | 26292 |

_____________________________

# Abstract

This document reports on the test procedure and results from the second dry-run evaluation of the test sequences under consideration for the CfP conducted at the Santa Eularia meeting, assessing their suitability and finalising the associated bit rate points. It’s content is identical to JVET-AP0285.

# Introduction

Following the expert viewing of the test sequences under consideration for the CfP at the AGH17 meeting in Aachen, a second dry-run test was conducted at the Santa Eularia meeting, evaluating the suitability of candidate test sequences and associated final rate points for subjective evaluation. The Degradation Category Rating (DCR) protocol was selected for the subjective evaluation at this meeting [1][2]. This document reports the test procedure and the results of the evaluation. A new methodology for enhanced session randomization has been trialled, bringing four different randomizations for each session.

# Test setup

## Tasks conducted

The conducted dry-run viewings were aimed at the following tasks:

- Confirming modified rate points of test sequences listed in draft CfP.

- Evaluating candidate test sequences for inclusion in the draft CfP.

- Evaluating a small set of RPR rate points (only R1 and R2 considered).

- Evaluating modified 0.2x encoder configuration for one HPQ sequence.

- Additional tests conducted were as follows:

  - Evaluation of 3 HDR PQ HD gaming sequences, one upscaled to UHD and added as supplemental test point to enlarge UHD HDR PQ session.

  - Investigation of testing across test categories with common viewing parameters and varied frame rates (e.g. SDR HD RA together with SDR HD LD, UGC, etc.). Some sequences were observed two times, in separate 50fps and 60fps sessions.

## Reviewed test sequences

Sequences from the following categories were reviewed. Each category name includes a characterization of the content type, the suggested encoder configuration and an indication of the resolution.

- SDR RA UHD/4K: Representing the use case of distribution of standard dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

  - Adjusted bitrates for SRU3 (FireDance) and SRU4 (HallwayScene) were reviewed

- SDR RA HD: Representing the use case of distribution of standard dynamic range HD video content e.g. in a streaming scenario, using a random-access configuration.

  - Adjusted bitrates for SRH2 (DrivingPOV4) and TravellerSwim candidate were reviewed

- SDR LB HD: Representing the use case of conversational and other low delay applications at HD resolution, correspondingly using a low-delay configuration.

  - Adjusted bitrates for SLH1 (Beatriz) and SLH4 (OfficeWalkAtWall) were reviewed

- HDR-PQ RA UHD: Representing the use case of distribution of high dynamic range UHD/4K/8K video content using the PQ transfer function e.g. in a streaming scenario, using a random-access configuration. In order to reduce the encoding workload for assessment of this category and allow investigation on 4K displays, cropped regions of 3840×2160 resolution are used for 4K and 8K content.

  - Adjusted/additional bitrates for HPQ2 (FashionLadyCrop1) and HPQ5 (SparksWelding), and a new RT3 configuration for HPQ3 (MeridianHDR2), were reviewed

- HDR-HLG RA UHD: Representing the use case of distribution of high dynamic range UHD/4K/8K video content using the HLG transfer function e.g. in a streaming scenario, using a random-access configuration. In order to reduce the encoding workload for assessment of this category and allow investigation on 4K displays, cropped regions of 3840×2160 resolution are used for 4K and 8K content.

  - Adjusted bitrates for SeaWalk, CrowdWalk and TowerPlants candidates were reviewed

- Gaming LB HD: Representing the use case of online gaming with a low-delay configuration.

  - Carla: adjusted bitrates with a new rendering were reviewed

  - Wukong: adjusted bitrates were reviewed

  - New HDR candidates (previously tested in SDR) were reviewed: ARGP2, DesertTown3 and SunTemple3; ARPG2 was also viewed upsampled to UHD -using the normative RPR filters- as a filler in the UHD PQ viewing session

- UGC RA: Representing the use case of user generated content at 1080×1920 or 1920×1080 resolution using a random-access configuration.

  - URH1 (Camellia): adjusted bitrates were reviewed

## General coding conditions

VVC anchors were generated using the VTM 23.14 software package. A static quantization parameter (QP) setting was applied for the generation of the anchors, though a one-time change of the quantization parameter from value QP to value QP+1 could be applied in order to meet the defined target bit rates.

## Logistics

At the meeting site, a setup with four calibrated OLED displays was employed. The playout was provided by two laptops connected to two DeckLink 4K Extreme 12G cards through Echo Express SE IIIe Thunderbolt Expansion Systems. The DeckLink cards generated HDMI output from the playout system, each feeding two TVs over an HDMI splitter. The playout laptops were equipped with SSD drives capable of stable playout of raw YUV 10-bit data at the required frame rate. The dual playout capability enabled additional randomisations of the viewing sessions, enabling four available randomisations of each session.

Table 1: Test logistics

| Test Site | Ibiza Congress Center, Santa Eularia, room C2 |
| --- | --- |
| Display, size, connection (resolution setting) | 4 × LG OLED65B56LA, 65”, HDMI (3840×2160) connected via two HDMI splitter to the video PC |
| Viewing distance | 3 viewers at 1.5H |
| Viewing angle | ±75°, 90° (at screen centre) |
| Total number of viewers | 24 (video coding experts) |

Participating experts confirmed visual acuity and normal colour vision.

## Test method and test design

As mentioned above, the Degradation Category Rating (DCR) was used for the subjective evaluation [1] of the observed sequences. The test sequences were evaluated using the 11-grade scale shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale for the DCR tests.

Each basic test cell (BTC) is structured as followed:

“Original” (1sec) – [uncompr. sequence] (10sec) – “A” (1sec) – [PVS] (10sec) – “Vote <N>” (5sec)

PVS denotes the processed video sequence under evaluation.

Note that in the given design, a BTC for the DCR tests has a duration of 27sec.

## Test sessions

A total of 7 test sessions was conducted for the evaluation using the DCR method, where sequences were grouped by their format (SDR/PQ/HLG) and frame rate (30/60 and 25/50 fps).

Each viewing session had 24 viewers separated into two groups of 12.

The sessions were designed not to exceed 23min of viewing time. The sessions were organized by characteristics of the test sequences, including separation of SDR, HDR-PQ and HDR-HL, as well as 50 and 60 Hz test sequences. An overview of the sessions is provided in the table below.

Table 2: DCR test sessions

| Session ID | Session | PVS | Duration (min) |
| --- | --- | --- | --- |
| S01 | SDR HD 50Hz | 16 | 8 |
| S02 | SDR HD 60Hz | 16 | 8 |
| S03 | PQ HD 60Hz | 22 | 11 |
| S04 | HLG UHD 50Hz | 22 | 11 |
| S05 | PQ UHD 60Hz | 21 | 10 |
| S06 | SDR UHD 50Hz | 28 | 14 |
| S07 | SDR UHD 60Hz | 26 | 13 |

All test sessions were designed inserting a stabilization phase as suggested in ITU-R BT.500-14. The test sessions included trapping BTCs where the original uncompressed sequence was shown for evaluation.

The participating experts were trained with training sessions for each resolution and container format. All test sequences under evaluation were occurring at least once in the training sessions, and a selection of rate point representing the expected impairment range was presented. Before the presentation of the training sessions, the experts were instructed on the meaning of the impairment scale of the respective protocol. Any occurring requests and questions on the test procedure or the scale were answered. For the training, the following method was used:

- Explain procedure and scoring scale.

- Run test session (6 BTCs here, covering examples of the impairment range)

- Let viewers note their scores.

- Let them read out their scores. If variation is larger than 3 on the grading scale, re-run BTC

  - Viewers shall reconsider their scoring for that sequence (not enforced to change their mind)

  - No indications where to look allowed, viewers have to make it up by themselves.

  - Answering of remaining questions, no further communication of scores.

The UHD and HD test sessions were both presented with a viewing distance of 1.5H of the UHD display for the centre seat. The HD sequences were displayed without scaling in the centre of the UHD area with a mid-gray padding around them. Thereby, an effective 3H viewing distance for the HD content was achieved.

# Results and analysis

A total of 151 PVS were evaluated in the conducted DCR tests.

## Data processing

Usual data checks were applied to the raw dataset.

As a first data processing step, the Control PVS (uncompressed sequence) was evaluated. A threshold of an opinion score OS>7 was applied, indicating that the viewer would rate the uncompressed sequence to show no or only slight impairment (scores 10, 9, 8). Very few viewers failed and were rejected (5.4%, 9 out of 168 viewers).

As a next step, the Pearson Correlation Coefficient (PCC) was evaluated for each viewer and viewers with a PCC<75% were removed (0.6%, 1 out of 168 viewers).

Finally, the non-normalized z-scores with a threshold of 2.5 were applied. This led to a small number of outliers which were removed (2.0%, 73 out of 3624 scores).

In the following sections, the measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream in each of the tested categories. The ±95% confidence intervals (CIs) for the MOS values are indicated.

## Reviewed SDR RA UHD/4K sequences

### Sequence formats and target bit rates

All sequences have a picture size of 3840×2160, chroma format 4:2:0 YCbCr according to ITU-R BT.709, and bit depth 10. Further information can be found in Table 3.

Table 3: Test sequences

| Sequence ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRU3 | FireDance | 10 | 250 | 25 | bfcfc333924835aaf4ed89efa8428a36 |
| SRU4 | HallwayScene | 10 | 250 | 25 | b0c8718579998060e375fcdb77243013 |

Target bit rates for the encoding of the sequences are listed in Table 4. Changes from JVET-AO2026 are underlined (some bitrates were increased). Only Rates 1, 2, 3, 4 were used for viewing. HallwayScene was viewed twice in two different sessions: once played at 50fps with frame doubling, and once played at 60fps as a 24fps source with frame doubling/tripling.

Table 4: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SRU3 | FireDance | 400 | 800 | 1500 | 2500 | 5000 |
| SRU4 | HallwayScene | 150 | 250 | 500 | 1000 | 2000 |

Figure 2: MOS plots for the FireDance (SRU3) and HallwayScene (SRU4) in the SDR RA UHD/4K category

## Reviewed SDR RA HD sequences

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080, chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 5.

Table 5: Test sequences

| Sequence ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRH2 | DrivingPOV4 | 10 | 600 | 60 | 4724c33b6b27669a406587bdb8d43dc8 |
| - | TravellerSwim | 10 | 500 | 50 | d79cc6457e82f28ec3397adda4bede13 |

Target bit rates for the encoding of the sequences are listed in Table 6. Changes from JVET-AO2026 and JVET-AP0046 are underlined (bitrates were increased for DrivingPOV4, and decreased for TravellerSwim). Only Rates 1, 2, 3, 4 were used for viewing.

Table 6: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SRH2 | DrivingPOV4 | 150 | 300 | 750 | 2000 | 4000 |
| - | TravellerSwim | 150 | 300 | 600 | 1200 | 2400 |

Figure 3: MOS plots for the DrivingPOV4 (SRH2) and TravellerSwim in the SDR RA HD category

## Reviewed SDR LB HD sequences

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 7.

Table 7: Test sequences

| ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SLH1 | Beatriz L | 8 | 500 | 50 | fe74cd5046fa033b4f743f42b29e69cd |
| SLH4 | OfficeWalkAtWall L | 8 | 300 | 30 | 529c15491ea8e1eb0320244a6ff902bb |

Target bit rates for the encoding of the sequences are listed in Table 8. Changes from JVET-AO2026 are underlined (bitrates were increased). Only Rates 1, 2, 3, 4 were used for viewing.

Table 8: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SLH1 | Beatriz | 70 | 140 | 280 | 550 | 1100 |
| SLH4 | OfficeWalkAtWall | 90 | 200 | 450 | 1000 | 2000 |

Figure 4: MOS plots for the Beatriz (SLH1) and OfficeWalkAtWall (SLH4) in the SDR RA HD category

## Reviewed HDR-PQ RA UHD sequences

### Sequence formats and frame rates

UHD, 4K and 8K HDR PQ test sequences with chroma format 4:2:0 YCbCr, bit depth 10bit, according to ITU-R BT.2100, with the applicable transfer function indicated. Sequences of 4K or 8K resolution have been cropped to a picture size of 3840×2160. Further information can be found in Table 9.

Table 9: Test sequences

| ID | Sequence name | Transfer fct. | Frames | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| HPQ2 | FashionLadyCrop1 | HDR10 PQ | 380 | 60 | d8546edaa260468f027c0e1c28d422b0 |
| HPQ3 | MeridianHDR2 | P3 PQ 4000nits | 600 | 60 | 9e889c7d78b1b5a0fdebb88871a362dd |
| HPQ5 | SparksWelding | HDR10 PQ 1000nits | 600 | 60 | 7e0c6bd867c370dde9b11c76ed93409f |

Target bit rates for the encoding of the sequences are listed in Table 10. Bitrate changes from JVET-AP0046 are underlined. For HPQ2, an alternative R4 was tested as proposed in [5]. For HPQ3, no bitrate changes were applied, but the updated VTM rt3 configuration from [4] was used.

Table 10: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 4b | Rate 5 |
| HPQ2 | FashionLadyCrop1 | 250 | 500 | 1000 | 2000 | 5500 | 4000 |
| HPQ3 | MeridianHDR2 | 150 | 300 | 600 | 1200 |  | 2400 |
| HPQ5 | SparksWelding | 400 | 1000 | 2500 | 6000 |  | 12000 |

Figure 5: MOS plots for the FashionLadyCrop1 (HPQ2) and MeridianHDR2 (HPQ3) and 
SparksWelding (HPQ5) in the HDR-PQ RA UHD category

## Reviewed HDR-HLG RA UHD sequences

### Sequence formats and frame rates

UHD, HDR HLG candidate test sequences with chroma format 4:2:0 YCbCr, bit depth 10bit, according to ITU-R BT.2100, with the applicable transfer function indicated. All sequences under consideration have a picture size of 3840×2160. Further information can be found in Table 11.

Table 11: Test sequences

| ID | Sequence name | Transfer fct. | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| - | SeaWalk | HLG10 | 500 | 50 | c6640bb1c3cdef1d653bd54271cfe49c |
| - | CrowdWalk | HLG10 | 500 | 50 | 9e16a8217657cb3aa6a744356ed7d994 |
| - | TowerPlants | HLG10 | 500 | 50 | 8a68e3168a2f235bb001e3bb99d4d6c4 |

Target bit rates for the encoding of the sequences are listed in Table 12. Changes from JVET-AP0046 are underlined (bitrates were increased for SeaWalk and compressed for CrowdWalk and TowerPlants). Only Rates 1, 2, 3, 4 were used for viewing.

Table 12: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| - | SeaWalk | 250 | 500 | 1000 | 2000 | 4000 |
| - | CrowdWalk | 500 | 1000 | 2000 | 4000 | 8000 |
| - | TowerPlants | 1200 | 2500 | 5000 | 12000 | 24000 |

In this category, additional test points using the RPR configuration were added in order to assess the impact of this encoder configuration on the test sequences under consideration.

Figure 6: MOS plots for the SeaWalk, CrowdWalk and TowerPlants 
in the HDR-HLG RA UHD category

## Reviewed of candidates for Gaming LB sequences

### Sequence formats and frame rates

During this review, a set of new candidate sequences in this category was assessed. The picture sizes of new candidates were 1920×1080 or 3840×2160, with chroma format 4:2:0 YCbCr according to ITU-R BT.709 or ITU-R BT.2100. Further information can be found in Table 13 and Table 14.

Table 13: UHD SDR test sequences

| ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| - | Carla3 | 8 | 600 | 60 | f19a46250aafb999b37e83f834895222 |
| - | Carla4 | 8 | 600 | 60 | 360dee13c047252af13cf1af5c7cca58 |
| - | Carla5 | 8 | 600 | 60 | 11db5b51ad88b53ab46c0d7953601e8c |
| - | Wukong1 | 10 | 600 | 60 | 5accafb1c16ecc961740cd90286ca9b8 |
| - | Wukong2 | 10 | 600 | 60 | 8da88d4168684a66623616d2d1112211 |
| - | Wukong3 | 10 | 600 | 60 | af3f9598bcfbbf76be2df7e0f30d39a6 |

Table 14: HD HDR -PQ test sequences

| ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| - | ARPG2-HDR | 10 | 600 | 60 | 6eabdb7f0827afa5b0ecb95208b069c1 |
| - | DesertTown3-HDR | 10 | 600 | 60 | 8b91f2c5630b592d8bc0e30c6e33b758 |
| - | SunTemple3-HDR | 10 | 600 | 60 | e9766660f098c63a5874605a89bf6ccf |

Target bit rates for the encoding of the sequences are listed in Table 15. Only Rates 1, 2, 3, 4 were used for viewing. The HDR sequences are new, a new rendering of Carla was used, and bitrate changes from JVET-AP0046 are underlined (general increase for Carla, decrease for Wukong1, minor adjustment for Wukong2 and some increase of lower end for Wukong3).

Table 15: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| - | Carla3 | 700 | 1600 | 3600 | 8000 | 16000 |
| - | Carla4 | 1000 | 2200 | 5000 | 11000 | 22000 |
| - | Carla5 | 1100 | 2200 | 4300 | 8500 | 17000 |
| - | Wukong1 | 800 | 1800 | 4500 | 10000 | 20000 |
| - | Wukong2 | 800 | 2000 | 4500 | 12000 | 24000 |
| - | Wukong3 | 800 | 2000 | 5000 | 15000 | 30000 |
| - | ARPG2-HDR | 200 | 400 | 700 | 1300 | 2600 |
| - | DesertTown3-HDR | 150 | 250 | 450 | 800 | 1600 |
| - | SunTemple3-HDR | 200 | 350 | 600 | 1000 | 2000 |

In this category, the following additional aspects were tested:

- Carla3 and Carla 5 were shown at 50fps instead of 60fps for both configurations (referred to as “Extra” and “RPR” in the plots).

- Wukong2 was shown once at 60fps (“Anchor” in the plot) and once at 50fps (“Extra” in the plot)

- ARPG2 was shown in a session at HD resolution (“Anchor” and “RPR”), and once upsampled to UHD resolution in the UHD HDR-PQ session (“Extra”).

Figure 7: MOS plots for the Carla 3, 4, and 5 in the Gaming LB category

Figure 8: MOS plots for Wukong 1, 2, and 3, as well as ARPG2-HDR,
DesertTown3-HDR, and SunTemple3-HDR in the Gaming LB category

## Reviewed UGC RA sequences

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 16.

Table 16: Test sequences

| ID | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| URH1 | Camellia P | 8 | 600 | 60 | 6e5413666e42d3c24db9b81b5db641ff |

Target bit rates for the encoding of the sequences are listed in Table 17. Changes from JVET-AO2026 are underlined (bitrates were increased). Only Rates 1, 2, 3, 4 were used for viewing.

This sequence was shown once in a HD 60fps session (“Anchor”) and once in a HD 50fps session (“Extra”).

Table 17: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Sequences name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| URH1 | Camellia | 200 | 400 | 800 | 1600 | 3600 |

Figure 9: MOS plots for the Camellia (URH1) in the UGC RA category

# Discussion and conclusions

The evaluation of the test sequences and rate points in this dry-run experiment is considered successful. Based on the results of the tests, it is suggested that the rate points evaluated can be used to further refine the final test set for the CfP.

For the test sequences evaluated two times at different frame rates (50fps or 60fps), consistent results were observed. It is suggested that choosing a playout at such slightly divergent frame rate would be acceptable also in the context of the CfP, if consistently applied for the anchor and proposals across test sites.

The test sequences HD HDR-PQ gaming test sequences were considered very stressful to assess for the test subjects. Based on comments from participating experts it is suggested to carefully consider inclusion of such sequences in subjective tests with expert or naïve viewers, especially in the given passive setup where the viewer cannot influence the camera motion in the scene.

# References

- Recommendation ITU-T P.910 (2023), “Subjective video quality assessment methods for multimedia applications”.

- Recommendation ITU-R BT.500-15 (2023), “Methodologies for the subjective assessment of the quality of television images”.

- “Final draft Joint Call for Proposals on Video Compression with Capability beyond VVC”, Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29, 41st Meeting, by teleconference, 14–23 January 2026, Doc: JVET-AO2026-v7.

- F. Bossen, “Improved VTM configuration for 0.2x runtime target”, Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29, 42nd Meeting, Santa Eulària, 24 April – 1 May 2026, Doc: JVET-AP0237.

- D. Li, V. Rufitskiy, A. Filippov, H. Qin, “Additional sequence evaluation results for the upcoming CfP”, Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29, 42nd Meeting, Santa Eulària, 24 April – 1 May 2026, Doc: JVET-AP0149.