ISO/IEC JTC 1/SC 29/AG 5 N201

ISO/IEC JTC 1/SC 29/AG 5

MPEG Visual Quality Assessment

Convenorship: DE

Document type:	Projects

Title:	Report of verification test on VVC multi-layer coding

Status:	Approved

Date of document:	2026-07-15

Source:	ISO/IEC JTC 1/SC 29/AG 5

Expected action:	Information

Action due date:	None

No. of pages:	21 (with cover page)

Email of Convenor:	wien@lfb.rwth-aachen.de

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29ag5

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/AG 5

MPEG VISUAL QUALITY ASSESSMENT

ISO/IEC JTC 1/SC 29/AG N201

Santa Eularia, ES – April 2026

| Title: | Report of verification tests on VVC multi-layer coding |  |  |
| --- | --- | --- | --- |
| Status: | Approved |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Olena Chubach Shunsuke Iwamura Philippe de Lagrange Mathias Wien | Tel: Email: | olena.chubach@mediatek.com iwamura.s-gc@nhk.or.jp philippe.delagrange@interdigital.com wien@lfb.rwth-aachen.de |
| Source: | Verification Test Coordinators |  |  |
| Serial no.: | 26406 |  |  |

_____________________________

# Executive Summary

This document reports the verification test results of VVC multi-layer coding. The report comprises two tests: test one on content layering features and test two on scalability performance using dual-layer coding.

The purpose of the content layering test was to showcase the content personalization feature enabled by VVC by reframing the base layer and overlaying additional content in an enhancement layer, and let the user choose to view either the base layer or the enhancement layer of their choice. In the tested configuration,  downscaling of the base layer was used in the enhancement layer to reframe the content and add L-shaped additional content extending the area around the base layer.

This capability of VVC was verified by setting up a demonstration, involving real-time multi-layer VVC decoding and output layer selection. This demonstration was run successfully in Geneva during the 31st JVET meeting.

The purpose of the scalability test was to verify that the VVC enhancement layer can provide significant quality improvement over the base layer while achieving:

- Significant coding gains compared to simulcast, allowing, e.g., reduced bandwidth for broadcasting or lower CDN cache storage requirements for streaming applications.

- Moderate losses compared to single-layer coding, allowing, e.g., delivery of a dual-layer bitstream instead of a single-layer stream at the target quality.

This capability of VVC was verified by conducting a formal visual quality assessment test using video sequences relevant for broadcast and streaming applications. Two dual-layer configurations were tested and compared with single-layer and upscaled base layer: HDR UHD with HD base layer (2x scaling ratio), and SDR HD with 720p base layer (1.5x scaling ratio). Both used approximately 25% of the total bitrate for the enhancement layer.

VTM-23.11 was used to generate the bitstreams for the scalability test. It is generally observed that the tested VTM dual-layer configuration performed on par or only slightly below the tested single-layer configuration. At the highest rate point, a slight benefit over the single-layer configuration was even observed for some test sequences. Also, the enhancement layer was verified to bring quality improvement compared to the upscaled base layer.

The test demonstrated excellent performance of the VVC Multilayer Main 10 profile and its suitability for applications such as the bitrate ladder for adaptive streaming, delivering enhanced quality while reducing edge storage requirements.

# Introduction

The main design objective of the VVC standard [1] is its high versatility combined with improved compression performance relative to its predecessor, the HEVC standard.

Verification tests of improved compression capability and support of omnidirectional video were reported in [2], [3] and [4].

One important versatile feature of VVC is multi-layer coding, supported in the VVC Multilayer Main 10 profile. Compared to scalable HEVC (SHVC), which requires an integrated upsampler to match the spatial resolutions of the base and enhancement layers, VVC natively supports multi-layer. Rather than relying on additional low-level tools, rescaling in VVC is supported by the reference picture resampling (RPR) tool included in the VVC Main 10 profile. In addition, VVC allows both, upscaling and downscaling, resulting in more flexibility for reframing. Moreover, all layers in VVC share the same decoded picture buffer (DPB), so that a single decoder instance can decode all relevant layers. The VVC Multilayer Main 10 profile supports both, scalable representations of the encoded content decoded to a single output layer, and multi-view representations, that provide multiple output layers for applications such as stereo and general multi-view content. This test report focuses on multi-layer coding with a single output layer.

VVC multilayer enables efficient transmission and storage of video content at different resolutions and quality levels for broadcasting and streaming applications (scalability), as well as content personalization through reframing and overlays (content layering).

In broadcasting, the same content is often delivered at different resolutions for home TVs and mobile devices. In streaming, video content is commonly stored at multiple resolutions and quality levels on servers. In such cases, multilayer coding can reduce bandwidth usage and storage requirements compared to encoding, storing, and transmitting each version separately.

Personalized video services have also become increasingly important. Typical examples include multilingual content and sign language. With content layering, these services can be supported directly at the video coding level, whereas previously they were handled only at the system level.

The effectiveness of content layering is demonstrated in this report through the functionality test described in Section 2, in which the main content and multiple sub-contents (enhancements) are encoded in a multi-layer bitstream and delivered to the users. By selecting the layer to decode, users select whether to view the main content or one of the sub-contents available.

The effectiveness of multilayer in terms of coding efficiency with spatial scalability, is demonstrated in this report through the formal subjective test, results for which are reported in Section 3. In this test, the VVC Multilayer Main 10 profile is compared with VVC single-layer coding and simulcast coding using the VVC Main 10 profile. The following two aspects are addressed:

- Spatial scalability: an enhancement layer (for example, UHD) is coded on top of a lower resolution base layer (for example, downscaled by 2x), while only a small portion of the bitrate is assigned to the enhancement layer. This setup targets use cases that require support for multiple resolutions, while maintaining quality and bitrate close to those of single-layer coding. The enhancement layer quality is compared with both single-layer coding and the upscaled base layer.

- Quality ladder: a range of resolutions and bitrates is used to evaluate both spatial and quality scalability. The tested resolutions range from 720p to UHD, with multiple bitrate points. The quality points are grouped into layer pairs with resolution ratios of 1.5x or 2x. This setup targets streaming applications where efficient local cache storage is important. In this case, dual-layer coding at different quality points is compared with single-layer coding.

# Content layering

This section reports the verification test results of VVC multi-layer coding according to the “Content layering” category of the verification test plan [5]. A functional test using three test sequences with sub-contents was carried out during the 31st JVET meeting using a real-time VVC decoder with multilayer coding capability, to demonstrate the effectiveness of content layering application.

This test exercises content personalization features, where the base layer is a full frame video, and the enhancement layer reframes the base video and adds overlaid elements; the user can be offered several such enhancements (also called sub-content) to choose from, according to their preferences. The test enhancements include sign language, dancing tutorials, and sport event side information with extra views.

These results have been previously released in document JVET-AF2033 [6].

## Functional test setting

### Test sequence creation

The three test sequences used in the functional test are 4K, 60Hz and filmed by NHK. Location footage, football game, and children’s program are selected as common program. For each content, sub-content that matched the context of each program was created. For location footage, the main content was down-converted and sign language commentary was inserted. For football game, another camera footage focused on the home team and away team was overlaid in a small window on top of main content. In children’s program, a dance instruction video is overlaid on the down-sized main content.

Table 1 – Test sequences for content layering

|  | Content1: Location footage Main: Report from library Sub: Main with sign-language | Content2: Football game Main: Main camera Sub1: Main with home team view Sub2: Main with away team view | Content3: Children’s program Main: Music video Sub1: Main with dance instruction by a boy Sub2: Main with dance instruction by a girl |
| --- | --- | --- | --- |
| L0 |  |  |  |
| L1 |  |  |  |
| L2 |  |  |  |

### Test bitstream preparation

VTM-18.0 with content layering functionality is used for generating the bitstream. The main content is coded as base layer bitstream. For the enhancement layer, the sub-content with alpha channel information is overlaid on top of the locally decoded picture of the base layer and used as the input signal of the enhancement layer. Since most of the area of the enhancement layer is exactly the same as the base layer, most blocks can be coded as “skipped” without RD optimization and only the overlay region is coded with RD optimization.

Figure 1 illustrates an overview of the encoding procedure for content layering. Note that the alpha channel information does not have to be transmitted to the decoder side since the input signal of the enhancement layer is the main content with overlay.

Figure 1. Content layering using multilayer coding

Figure 2 shows an alternative solution for realizing the same functionality using systems layer composition. Here, the main content, sub-content, and the alpha channel information are separately encoded as single-layer bitstreams and transmitted to the decoder (along with position information, if needed). At the decoder side, the main content and the sub-content are decoded separately and composited using the position information and the alpha channel information.

Figure 2. Content layering using system layer composition

For some test sequences (Content1 and Content3), the sub-content is not just overlaid on top of the main content. The main content is down-scaled, and L-shaped sub-content is inserted into the margin. Inter-layer prediction can be effectively performed with the scaling window functionality enabled. The down-scaling filter is the same as the filter used in RPR, so that inter-layer prediction is performed perfectly in a manner similar to the case of normal sub-content overlaying shown in Figure 1. Figure 3 illustrates an overview of the coding procedure for content layering with L-shaped sub-content.

Figure 3. Content layering with scaling window

### Encoding parameters

The following test conditions were used for the functional test.

- Rate control

  - The rate control is enabled for base and enhancement layers. The allocated bitrates for BL and EL are 20Mbps and 2Mbps, respectively.

- Coding structure: random access

  - Intra refresh at approximately 1 second intervals.

  - GOP size 32

  - Picture reordering allowed.

  - Inter-layer and intra-layer prediction allowed.

### Equipment setting

The functional tests were coordinated by the chairs of JVET AHG4 and AG5. A block diagram of the test equipment is illustrated in Figure 4. A multilayer bitstream encapsuled in CMAF over MMT is output from the streamer, and a VVC decoder supporting multilayer decodes the bitstream in real time. The brief specifications of the VVC multilayer decoder are shown in Table 2.

Viewers can select one of the main content and the available sub-contents for play out using a remote control running on a Tablet PC. The VVC multilayer real-time decoder, the CMAF/MMT streamer, and the signal converter for the functional test were provided by NHK and the 4K Monitor was provided by ITU-T SG16.

Figure 4. Equipment for the functional test

Table 2. Specifications of multilayer VVC decoder

| Profile | Main 10, Multilayer Main 10 |
| --- | --- |
| Level | 6.3 |
| Number of supported layers | 4 layers |
| Input Format | CMAF (ISO/IEC 23000-19) on MMT (ISO/IEC 23008-1) |
| Output Format | Resolution: 1080p, 2160p, 4320p Frame rate [Hz]: 59.94, 60, 119.88, 120 |

## Demonstration and discussion

Demonstration was conducted on July 19th at “M03b” in Mont-brilliant building. Approximately 20 participants attended the demonstration. A simple remote-control application running on a tablet PC was used, allowing participants to freely switch the displayed content by tapping the buttons in the application.

A question was raised about how rate control is handled. It was reported that rate control was performed independently per layer. This aspect is considered future work to ensure compliance with the bitrate limits for the overall stream.

A question was also raised about how multiple versions of such a scheme could be supported, for example, in an HTTP streaming scenario. This is considered a challenging encoding task and has not yet been investigated.

A question was further raised about how HDR metadata would be handled. In this case, the metadata would need to be updated for the each enhancement layer to reflect the parameters for the final scene composition.

Figure 5. 4K monitor and remote-control-like tablet PC

Figure 6. Demonstration of content layering

# Scalability

In this section, the results for the subjective testing according to the verification test plan JVET-AO2021 [7] for the VVC Multilayer Main 10 profile in dual-layer configurations involving both spatial and quality scalability are reported.

The test addresses two use cases relevant to multilayer coding:

- broadcasting applications: the same content is often broadcast simultaneously at different resolutions for both home TV sets and mobile devices;

- streaming applications: the same video content is stored on the server at multiple quality levels and resolutions.

In streaming applications, video quality is typically adjusted according to network conditions and device capabilities. To deliver the optimum video quality, the same video content is encoded at multiple resolutions and bitrates, and sometimes with different codecs. The resulting set of encoded video versions is referred to as a “quality ladder.”

The quality ladder is typically stored in the so-called edge storage, which acts as a local cache for adaptive content delivery. The most suitable encoded video version is then selected and delivered to each user, based on their current conditions.

Because the same content is encoded multiple times at different quality levels, multi-layer coding can help reduce broadcast bandwidth and edge storage requirements by combining several quality points into a single bitstream. At the same time, the configurations and performance of multi-layer coding can directly affect the end-user viewing experience.

## Test setup

The test focuses on dual-layer coding, in which the end-user device decodes at most one base layer and one enhancement layer. Multiple independent layer pairs may also share the same base layer.

The objective of this test is to verify that a VVC enhancement layer can provide significant quality improvement over the base layer while achieving:

- Significant coding gains compared to simulcast, such as reduced bandwidth in broadcasting or lower CDN cache storage requirements in streaming applications.

- Only moderate losses compared to single-layer coding, for example when a dual-layer bitstream is delivered instead of a single-layer stream at the target quality.

For the visual evaluation, a simplified set of configurations is selected matching the playback on a typical UHD TV commonly used in test environments. The configurations are designed to address the spatial scalability and quality ladder scenarios:

- Evaluating the contribution of the enhancement layer compared to the upscaled base layer.

- Evaluating the performance of dual-layer coding compared to single-layer coding and simulcast coding.

The following dual-layer configurations are included:

- HD base layer with UHD enhancement layer (2x scaling ratio), using HDR PQ transfer characteristics.

- 720p base layer with HD enhancement layer (1.5x scaling ratio), using SDR transfer characteristics.

- Enhancement layer bitrate set to 25% of the total bitrate, representing a moderate allocation that matches a typical step in the quality ladder.

The visual assessment includes the following test points:

- Single-layer anchor

- Upscaled base layer

- Enhancement layer

HD video is displayed without scaling and centered on a UHD TV.

The evaluation was performed by formal subjective visual testing according to the DCR / DSIS protocols [8][9].

### Test conditions and encoder configurations

Number of sequences, video resolutions and frame rates:

- Five HDR PQ sequences at UHD resolution (3840×2160), downscaled to 1920×1080 using the TAppDownConvertStatic tool from the SHM repository. The following example command line is used (relevant for UHD BT.2020-color content, using chroma location 2):

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10 0 0 0 600 -phase 0 0 0 0

- Five SDR sequences at HD resolution (1920×1080), downscaled to 1280×720 using the TAppDownConvertStatic tool from the SHM repository. The following example command lines are used:

  - For HD BT.709-color content (chroma location 0):

TAppDownConvertStatic 1920 1080 <input_HD.yuv> 1280 720 <output_720.yuv> 10 10

  - For HD BT.2020-color content (chroma location 2):

TAppDownConvertStatic 1920 1080 <input_HD.yuv> 1280 720 <output_720.yuv> 10 10 0 0 0 600 -phase 0 0 0 0

- Both 24fps and 60fps, to vary the number of frames per intra period

Rate points:

- For single-layer anchor: 3 rate points covering the MOS range from 4 to 8 or slightly higher

- Base layer QP is adjusted so that BL bitrate is the closest to 3/4 of the anchor, and enhancement layer QP is adjusted so that total bitrate is aligned with the anchor.

- Extra rate points for single-layer anchor at lower quality (targeting MOS 2) are generated to keep the usual range presented to viewers

Encoding software and scaling ratios:

- Anchors: encoded using VTM-23.11, single layer, for each resolution

- Test: encoded using VTM-23.11 in dual-layer configuration:

  - (1280×720) + HD (1.5x)

  - HD + UHD (2x)

Encoding parameters:

- Bit depth of 10 bits for all video resolutions

- Random access coding structure:

  - Intra refresh at approximately 1 second intervals.

  - GOP size 32

  - Picture reordering allowed.

  - Inter-layer and intra-layer prediction allowed. Dependent layer uses the same coding structure + same POC of base layer as additional reference.

- Other settings as in the configuration files, with the Picture Hash SEI deactivated for the measurement of bit rate

  - VTM: cfg/encoder_randomaccess_vtm.cfg, and set DMVREncMvSelect=1

  - VTM dual layer: cfg/multi-layer/two_layers_scalable.cfg, and (for layer 1 only) cfg/multi-layer/layer1_gop_randomaccess.cfg

  - Applicable HDR parameters when relevant

### Test sequences and rate points

Test sequences representing streaming use cases and containing high-resolution detail were reviewed and selected, with attention given to detail level and a balanced range of frame rates. The selected sequences are listed in Table 3 and Table 4.

The applicable QP values are reported in Table 5 and Table 6.

Table 3 –Test sequences for HDR PQ, UHD

| Test sequence & format | Thumbnail | Test sequence & format | Thumbnail |
| --- | --- | --- | --- |
| BodeMuseum (PQ) 3840×2160, 60fps, 600 frames PQ, BT.2020 color f7275fa73812e517b07fbe9e32dd7ae3 |  | H3-AMS08 3840×2160, 60fps, 600 frames PQ, BT.2020 color a9a429a6fbcaf915d9396ccb57790e40 |  |
| Meridian-Parking1 3840×2160, 60fps, 600 frames PQ, BT.2020 color 89646742683e09090734e1bb7dbf957c |  | Stem2-Alert 3840×2160, 24fps, 240 frames PQ, BT.2020 color e0a213e7c7d7149834fe6d379df27003 |  |
| Zombie-Climbing2 3840×2160, 24fps, 240 frames PQ, BT.2020 color 9fce60bf59aa0e8e073eb3a69313d0bc |  |  |  |

Table 4 –Test sequences for SDR, HD

| Test sequence & format | Thumbnail | Test sequence & format | Thumbnail |
| --- | --- | --- | --- |
| BodeMuseum (SDR) 1920x1080, 60fps, 600 frames SDR, BT.2020 color ddea5e90e9fe941f555b7ecc2949182c |  | Meridian-Smoker0 1920x1080, 60fps, 600 frames SDR, BT.709 color 2637790d450134872a9f03bacdba4502 |  |
| Procession 1920x1080, 60fps, 600 frames SDR, BT.709 color 32f66ac2979d97f26f8608374d4ffe98 |  | Stem2-Waiting1 1920x1080, 24fps, 240 frames SDR, BT.709color 719a4325320716ceee18f8b6c92e24b9 |  |
| TearsOfSteel-Discussion 1920×852, 24fps, 240 frames SDR, BT.709color 18c97060109f068d4e0c6fe5c4cac6dd |  |  |  |

Table 5 – QPs for HDR, UHD test content

| Name | VTM SL QP | VTM DL BL/EL QP |
| --- | --- | --- |
| BodeMuseum (PQ) | 22 | 16 / 27 |
|  | 26 | 20 / 30 |
|  | 30 | 24 / 33 |
|  | (38) |  |
| H3-AMS08 | 25 | 18 / 29 |
|  | 29 | 23 / 32 |
|  | 34 | 28 / 36 |
|  | (42) |  |
| Meridian-Parking1 | 26 | 21 / 29 |
|  | 30 | 25 / 33 |
|  | 34 | 29 / 36 |
|  | (42) |  |
| Stem2-Alert | 21 | 17 / 22 |
|  | 24 | 20 / 25 |
|  | 28 | 24 / 29 |
|  | (38) |  |
| Zombie-Climbing2 | 23 | 19 / 24 |
|  | 28 | 24 / 29 |
|  | 33 | 29 / 33 |
|  | (42) |  |

Table 6 – QPs for SDR, HD test content

| Name | VTM SL QP | VTM DL BL/EL QP |
| --- | --- | --- |
| BodeMuseum (SDR) | 27 | 23 / 32 |
|  | 31 | 28 / 34 |
|  | 36 | 33 / 38 |
|  | (40) |  |
| Meridian-Smoker0 | 22 | 20 / 23 |
|  | 27 | 25 / 28 |
|  | 31 | 29 / 32 |
|  | (38) |  |
| Procession | 25 | 23 / 26 |
|  | 30 | 28 / 31 |
|  | 35 | 33 / 35 |
|  | (40) |  |
| Stem2-Waiting1 | 23 | 22 / 25 |
|  | 28 | 27 / 29 |
|  | 33 | 32 / 34 |
|  | (40) |  |
| TearsOfSteel-Discussion | 22 | 19 / 26 |
|  | 28 | 26 / 30 |
|  | 34 | 32 / 35 |
|  | (38) |  |

The 4th single layer QP values (provided in brackets) were introduced to extend the quality range of the formal subjective viewing tests and reduce bias caused by the limited quality range of the actual test points. The MOS values for these points do not have corresponding dual layer configurations and therefore are not reported here.

### Logistics

RWTH Aachen University and VABTech performed the subjective visual tests. The test was independently performed in the two labs in order to achieve two independent measurements for all test points.

#### RWTH test setup

| Test Site | RWTH Aachen University |
| --- | --- |
| Display, size (resolution setting), connection | - 2× LG OLED77 (3840×2160), HDMI - 2× LG OLED55G19LA (3840×2160), HDMI The source signal was sent from the video board of the PC in parallel to the displays via an HDMI splitter. |
| Viewing distance | 3 viewers at 1.5H for the 77” displays, 2 viewers at 1.5H for the 55” displays. The HD video signal was displayed centered to the UHD screen with a mid-grey padding for the unused area. |
| Viewing angle | 90°, 70° |
| Total number of viewers | 26 (8 females, 18 males; age 18-30, 12 different nationalities), all screened for visual acuity and normal colour vision |

All test subjects were paid for their participation in the visual tests.

### VABTech test setup

| Test Site | VABTech |
| --- | --- |
| Display, size (resolution setting), connection | - 2× LG OLED65CX (3840×2160), HDMI. - Each display was connected by an HDMI cable to a different server; the two servers were identical and run the same video player SW. - All scores were collected by means of paper scoring sheets; then the scores were inserted in a Excel spreadsheet by the viewers. |
| Viewing distance | 2 viewers at 1.5H per display, the HD video signal was displayed centered to the UHD screen with a mid-grey padding for the unused area. |
| Viewing angle | 70° |
| Total number of viewers | 34 viewers (12 females, 22 males; age 18-23, all Italian), all screened for visual acuity and normal colour vision. Some of them were pre-screened for cognitive task ability |

All test subjects were paid for their participation in the visual tests.

### Test method and design

The tests were conducted according to the DCR of ITU-T P.910 using the 11-grade impairment scale shown below.

Figure 7: 11 grades impairment scale

#### RWTH Aachen University

In total, four viewing sessions were conducted, with two sessions for the UHD and two sessions for the HD test sequences. The sessions included 31-32 PVS (processed video sequences) each. The test sessions included a stabilization phase of three PVS and one PVS evaluating the uncompressed sequence to enable checking of the viewers’ attention.

The tests were performed with up to 20 viewers per viewing session, who were split into two groups of max 10 viewers for the training and the test sessions. The training as well as the tests were conducted in an interleaved fashion where one group was performing the session while the other group was resting, respectively. In each session, the viewers were provided with voting sheets, a pen and a clipboard to note down their scores. The viewers of the resting group were asked to enter their scores of the previous test sessions into a database for further processing.

At the start of the viewing session, the participants were instructed on the test. The size of the sessions and the context of evaluation of VVC single- and multi-layer coding with the example application of a streaming quality ladder was described. The test procedure and the DCR test protocol were explained by presenting a modified version of the AG 5 instructions for on-site DCR expert viewing tests [10], adapted for the use with naïve viewers. After answering potential questions from the viewers, they were shown the training session. The training sessions consisted of five PVS for the HD and UHD cases, each. The training sessions were designed to show each of the corresponding test sequences twice while covering the expected range of high to low impairments. For conduction of the training, the method used in JVET-AH2029/AG 5 N118 [11] was employed.

#### VABTech

The test was organized in four test sessions (2HD and 2UHD resolution). Each test session was designed including a three PVSs stabilization phase (whose scores were discarded) and two SRC vs SRC PVSs, to check for viewers’ behavior consistency. Each test session did not exceed 13 minutes.

An accurate training phase was run; a preliminary description of how a DSIS test session has to be run focusing on the correct timing in the scoring sheet fulfil; as all the (eventual) questions were answered, one short training test session was run selecting the widest range of quality available; as the training session was completed, the viewers were asked to read their score (one by one) for all the PVS they run. Should some weird score come out, the case was discussed by all the viewers, trying to converge to similar results.

For the actual test eight viewers were called for each time slot; they were divided in two groups of four subjects each; four subjects were running the test, while other four were loading their scores (from the paper sheets to an excel) and then resting.

A total of 34 viewers were hired for this test, 12 females, 22 males; the overall test execution was run across 5 testing days.

For some of the viewers a new cognitive effort test was done, using a subset of the Wechsler Adult Intelligence Scale IV (WAIS test).

## Test results and analysis

The acquired results were first independently and carefully processed by each of the labs with respect to outlier detection and consistency analysis.

The data of the subjective tests and of objective measurements are provided in the attached spreadsheet JVET-AP2033_AnnexMOSMetrics.xlsx.

### Data processing

#### RWTH Aachen University

As a first step, the complete set of votes from viewers was screened for the trapping sequences. In both, UHD and HD test sessions, a single viewer with a score below 8 for the original was identified. The results of the corresponding session with that trapping sequence were rejected for the affected viewer. The data was further screened using the Pearson correlation with a threshold of 75% (overall, three viewers were rejected for HD, and two for UHD), and applying non-normalized z-scores with a threshold of 2.5.

#### VABTech

At VABtech, post-test screening of the raw data produced by the viewers was done to exclude outliers and bad behaving humans.

A Pearson correlation (75%) screening was done (no viewer was excluded).

A homogeneity test of the raw scores was done: high quality  low quality, high scores,  low scores. Each nonhomogeneous case was marked; viewers with more than four non-homogeneous cases were excluded: no viewer was excluded after this analysis, even if some homogeneous cases were marked.

A further screening was done according to what is specified in Rec. BT500.

### MOS results and MOS-over-rate plots

After completion of the data acquisition, the results were processed independently in both laboratories and then analyzed. Both achieved mostly consistent results, with the test points showing the same ranking of the dual-layer and upscaled base-layer results. To accommodate all expressed viewer opinions, it was decided to merge the consolidated data of both laboratories for reporting and to compute the MOS and confidence interval over the pool of all viewers.

The plots present MOS-over-bitrate curves for the upscaled base layer, single layer, and dual layer configurations. Additionally, the curve for a simulcast configuration is shown. In this scenario, the base layer bitstream (at lower resolution) and the single layer bitstream (at target resolution) are transmitted and the target resolution is presented.

The plots in the following subsections show the resulting overall MOS scores with the 95% confidence interval (CI) marked for both, the positive and negative direction.

#### UHD HDR PQ results

Figure 8: MOS-over-bitrate plots for the UHD HDR PQ test sequences

#### HD SDR results

Figure 9: MOS-over-bitrate plots for the HD SDR test sequences

### PSNR-over-rate plots

The plots below present wPSNR-YUV-over-bitrate curves for the upscaled base layer, single layer, and dual layer, and simulcast configurations for the UHD HDR PQ test sequences and PSNR-YUV-over-bitrate curves for the upscaled base layer, single layer, and dual layer, and simulcast configurations for the HD SDR test sequences.

The plots in the following subsections show the aggregated YUV metrics computed from the Y, U, and V metric values according to the working practices using objective metrics for evaluation of video coding efficiency experiments [12].

#### UHD HDR PQ results

Figure 10: PSNR-YUV-over-bitrate plots for the UHD HDR PQ test sequences

#### HD SDR results

Figure 11: MOS-over-bitrate plots for the HD SDR test sequences

## Discussion

The testing task for this verification test is generally considered very challenging. This is partially due to the properties of the test sequences, including unsteady camera movement, scene cuts, and the impact of camera focus.

Overall, that the tested VTM reference software in dual-layer configuration performed on par with, or only slightly below the tested single-layer configuration. At the highest bitrate point, a slight benefit over the single-layer configuration can even be observed for some test sequences. In most cases, a clear MOS improvement is observed when comparing the dual-layer configurations with the upscaled base layer.

The performance of the upscaled base-layer configuration also highlights the varying properties of the content. In some cases, the single-layer and dual-layer configurations clearly outperform the upscaled base layer. In other cases, all three configurations perform comparably. In a few cases, the dual-layer configuration does not show clear benefit. This behavior may be explained by the challenging sequence content combined with potential encoder control limitations of the VTM reference software.

These observations also indicate further potential for quality ladder applications. The results suggest that combining the upscaled base layer with the refinement obtained with the dual-layer configuration within the same quality ladder can offer an effective set of operating points along the convex hull of the MOS-based rate-distortion curve.

The overall performance is summarized in the tables below  using the following color coding:

- Dark green: 	dual-layer configuration performs clearly better than upscaled base layer

- Light green: 	dual-layer configuration performs comparable to single layer and/or upscaled base layer

- Yellow: 	dual-layer configuration does not perform better than the other two configurations

Table 7 – Qualitative interpretation of the results

| UHD HDR PQ |  |  |  |
| --- | --- | --- | --- |
| Sequence | bit rates |  |  |
|  | low | mid | high |
| BodeMuseum |  |  |  |
| H3-AMS08 |  |  |  |
| Meridian-Parking1 |  |  |  |
| Stem2-Alert |  |  |  |
| Zombie-Climbing2 |  |  |  |
| HD SDR |  |  |  |
| Sequence | bit rates |  |  |
|  | low | mid | high |
| BodeMuseum |  |  |  |
| Meridian -Smoker0 |  |  |  |
| Procession |  |  |  |
| Stem2-Waiting1 |  |  |  |
| TOS-Discussion |  |  |  |

# Conclusions

The functionality test demonstrated the versatility of the VVC Multilayer profile for content layering applications. The real-time VVC decoder used in the test was capable of seamlessly switching between different content layers, providing the intended dynamic functionality described in the application scenario.

The formal subjective tests on dual-layer spatial scalability demonstrated excellent performance of the VVC Multilayer profile and its suitability for applications such as bitrate ladder for adaptive streaming. The results showed that VVC can provide operating points on the convex hull of the MOS-over-bitrate curves, delivering enhanced efficiency while reducing edge storage in the bitrate ladder application scenario.

# References

- Recommendation ITU-T H.266 | International Standard ISO/IEC 23090-3 (2022), Versatile Video Coding.

- V. Baroncini, M. Wien, “VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content,” Doc. JVET-T2020, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, online, October 2020.

- V. Baroncini, M. Wien, “VVC Verification Test Report for High Definition (HD) and 360° Standard Dynamic Range (SDR) Video Content,” Doc. JVET-V2020, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, online, April 2021.

- V. Baroncini, M. Wien, “VVC Verification Test Report for High Dynamic Range Video Content,” Doc. JVET-W2020, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, online, July 2021.

- S. Iwamura, P. de Lagrange, and M. Wien, “Verification test plan for VVC multilayer coding,” Doc. JVET-AD2021, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, Antalya, TR, April 2023.

- S. Iwamura, P. de Lagrange, and M. Wien, “Report of verification test on VVC multi-layer coding: Content layering,” Doc. JVET-AF2033, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, Hannover, DE, October 2023.

- O. Chubach, P. de Lagrange, M. Wien, “Verification test plan for VVC multilayer coding (update 8),” Doc. JVET-AO2021, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, online, January 2026.

- Recommendation ITU-T P.910 (2023), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-15 (2023), Methodologies for the subjective assessment of the quality of television images.

- ISO/IEC JTC 1/SC 29/AG 5 (Ed. M. Wien), “Instructions for on-site ACR and DCR expert viewing tests,” Doc. AG 5 N179, Geneva, Oct. 2025.

- V. Baroncini, J.-R. Ohm, M. Wien, “Visual quality comparison of ECM/VTM encoding,” Doc. JVET-AH2029, Joint Video Experts Team (JVET) of ITU-T SG21 and ISO/IEC JTC 1/SC 29, Rennes, FR, Apr. 2024.

- Technical paper ITU-T HSTP-VID-WPOM (2020), Working practices using objective metrics for evaluation of video coding efficiency experiments.

# Acknowledgements

The verification test coordinators wish to thank the organizations who contributed to and supported the VVC verification tests, particularly including NHK and ITU-T SG21, Ericsson, InterDigital, MediaTek, and Nokia.