# ISO/IEC JTC 1/SC 29/WG 03 N1820

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Technology under Consideration on ISO/IEC 23001-17

Status:	Draft

Date of document:	2026-05-01

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	5 (with cover page)

Email of Convenor:	young.L @ samsung . com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 03 MPEG SYSTEMS

ISO/IEC JTC 1/SC 29/WG 03 N1820

April 2025 – Santa Eulària, ES

| Title | Technology under Consideration on ISO/IEC 23001-17 |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Status | Draft |
| Serial Number | 26416 |

Abstract

This document collects the following topics and candidate technologies for the Carriage of uncompressed video and images in ISO Base Media File Format (ISO/IEC 23001-17).

Table of Contents

1	Introduction	2

2	Component limitation via 16-bit vs 32-bit parameter in uncC box definition	2

3	Storage location of non-uniformity corrections	2

4	Little Endian coding descriptions	3

5	Alignment of cmpd entries with the component index entries of the uncC box	3

6	Missing Value Range Signaling for Non-Unsigned-Integer Components	4

7	MPEG #154 outcome	4

# Introduction

Since 23001-17 Ed 1 was published, issues with the text have been identified for improving the clarity of feature implementation. In addition, additional features and capabilities are proposed for inclusion in a new 23001-17 Edition 2.

The following technologies under consideration are proposed for addressing in 23001-17 Ed2:

# Component limitation via 16-bit vs 32-bit parameter in uncC box definition

Description: In the declaration for the ‘uncC’ box, the component_index parameter is declared as an unsigned int (16). This limits the number of components to 65535 components. This is an error as the number of components supported in the ComponentDefinitionBox (‘cmpd’) declares the component_count parameter as an unsigned int (32). The component description extensions (Section 6) declare the component_index parameter as an unsigned int (32). These match the original intent of supporting 32-bits for the component count to support hyperspectral and other applications where the number of components may exceed 65K components.

Requirement: Support up to 32-bits worth of components in a declared image or video track.

Recommendation: Update the ‘uncC’ box to support the declaration of the component_index as an unsigned int (32).

# Storage location of non-uniformity corrections

Description: Section 6 of 23001-17 provides a number of boxes that support the storage of parameters needed for manipulating and displaying uncompressed imagery content. The majority of these boxes are small and can be easily carried as an item property or as a box inside the sample entry. The non-uniformity correction box (‘snuc’), however, contains offset array and gain arrays that are equivalent in size to the image or samples they are associated with. The ‘snuc’ box can therefore easily achieve tens of megabytes in size, which then bloats the size of the file level metabox or the movie box. Providing a mechanism to store these arrays in a media data box or in an external file will provide for a more efficient implementation in many cases. An additional need relates to the desirability of being able to store multiple gain/offset tables for an image or track. With infrared cameras, if recording activities occur over an extended time where the ambient temperature can change dramatically, such as early morning sunrise, to afternoon full heat of day, the calibration for a sensor can change dramatically. In these cases, it is beneficial to take multiple calibrations over time so corrections can best match the time of image capture.

Requirements:

- Add the ability to store the non-uniformity gain and offset tables in external files or in the mdat box

- Add the ability to store more than one table. Associate an absolute timestamp with each NUC table to enable management of multiple tables

Recommendation: Make necessary modifications to the ‘snuc’ box to address these requirements

# Little Endian coding descriptions

Description: During library development for 23001-17, questions arose w.r.t little endian coding implementation. In the next to last paragraph of 5.2.1.3, it says “If components_little_endian is 1, block_little_endian shall be 0 and component_align_size of each component shall be different from 0.” A question relates to if there is a typo here and whether “block_little_endian” should be “block_size”? Block_little_endian = 0 means it should be big endian (but we are declaring as little endian via components_little_endian=1). If components_little_endian is 1, then blocks wouldn’t be used, because the endian alignment is on the component, as opposed to a block. As a result, it seems like we would want to say block_size is 0, instead of block_little_endian is 0.

Recommendation: Improve the clarity of the language on blocking and little endian implementation.

# Alignment of cmpd entries with the component index entries of the uncC box

Description: During library development for 23001-17, question arose w.r.t. the alignment of entries in the component definition box and the indexed entries in the uncC box.

  - One interpretation is for every component in an uncC declared image or sample, there is a unique corresponding entry in the component definition box. In this case, if there are ‘n’ components declared in an uncC box, then there will be ‘n’ component entries in the ‘cmpd’ box.

  - A second interpretation is that multiple components declared in an uncC box for an image or sample can point to a single entry in the component definition box. For instance, if a hyperspectral image is declared with 1000 components and they are all considered monochrome, then all 1000 components can point to a single monochrome component type entry declared in the ‘cmpd’ box.

Related to this issue is the ability to properly associate a component index from a component description extension box defined in Section 6 to an appropriate component declared in the uncC box. Both point to an entry in the ‘cmpd’ box.

Requirement: Ensure there is a one-to-one, unique mapping to components listed in the ‘uncC’ box.

Recommendation: Improve the clarity of the language on component type assignments and component index values and ensure there is a clearly defined mapping from a specific component index in a box from Section 6 to a component index declared in the uncC box. While the ‘cmpd’ box was implemented separate from the ‘uncC’ box so it can be used for other codec allocations, consider providing a mechanism to declare the type directly in the ‘uncC’ box to reduce the total byte count since the information is effectively being written twice.

# Missing Value Range Signaling for Non-Unsigned-Integer Components

Description: A semantically correct conversion between two component types requires knowing the declared value range of both sides. The ComponentReferenceLevelBox (‘clev’) provides this only for unsigned integers - and only on one side at a time. Every conversion pair where at least one side is float (1), complex-number (2), or signed-int (3) is either undefined or does not produce the expected result.

‘clev’ does not cover the gap because floating point and complex numbers are explicitly excluded:

"Reference levels shall be ignored for non-integer component types."

- ISO/IEC 23001-17:2024, 6.1.4.1

The affine formula is not fit for signed integers. When clip_range = 0, clev defines:

- output = black_level + N * (white_level - black_level) / (2^k - 1)

where N is the raw bit pattern. For a k-bit signed integer N in [-2^(k-1), 2^(k-1)-1], negative values produce output below black_level and the maximum positive value never reaches white_level.

The level fields are signed int(32), which cannot represent float values (-1.0, 0.5, 10000.0) or the range of signed integers deeper than 32 bits.

Amendment 2 introduced signed-int (component_format = 3) with no corresponding update to clev.

Conversions are ambiguous without input and output ranges.

References:

- ISO/IEC 23001-17:2024, 5.2.1.3 Table 2 - Component formats

- ISO/IEC 23001-17:2024, 6.1.4 - ComponentReferenceLevelBox (clev)

- ISO/IEC 23001-17:2024/Amd. 2:2025, 5.2.1.3 - Addition of signed integer component format

Requirement: Ensure adequate CLEV adjustment support for non-unsigned integer pixel formats.

Recommendation: Add the ability to apply CLEV adjustments to all pixel numerical format types.

# MPEG #154 outcome

We study “m76776 - 23001-17 Observations and discussion topics”, included in the topics listed above, and develop draft text for a working draft for 23001-17 Edition 2, to be submitted to MPEG #155.