## INTERNATIONAL ORGANIZATION FOR STANDARDIZATION ORGANISATION INTERNATIONALE DE NORMALISATION ISO/IEC JTC 1/SC 29/WG 5 MPEG JOINT VIDEO EXPERTS TEAM WITH ITU-T SG21

### ISO/IEC JTC 1 / SC 29 / WG 5 N 400

### Santa Eulària, ES, 24 April – 1 May 2026

Source:; Convenor (Jens-Rainer Ohm). Title:; Joint Call for Proposals on video compression with capability beyond VVC. Type:; General. Subtype:; Information. Status:; Approved. Date:; 2026-07-15. Expected Action:; Info. Action due date:; N/A. Pages:; 23 (not including this cover page). Email of convenor:; ohm @ ient . rwth-aachen . de. Committee URL:; https://sd.iso.org/documents/ui/#!/browse/iso/iso-iec-jtc-1/iso-iec-jtc-1-sc-29/iso-iec-jtc-1-sc-29-wg-5.

| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 43rd Meeting, Geneva, CH, 7–15 July 2026 | Document: JVET-AQ2021-v1 |
| --- | --- |

| Title: | Joint Call for Proposals on Video Compression with Capability beyond VVC |  |  |
| --- | --- | --- | --- |
| Status: | Approved |  |  |
| Purpose: | Joint CfP |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Mathias Wien Frank Bossen | Email: | ohm@ient.rwth-aachen.de wien@lfb.rwth.aachen.de frank@bossentech.com |
| Source: | JVET |  |  |

_____________________________

# Introduction

This document is the Joint Call for Proposals on video coding technology with capabilities that significantly exceed those of the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3) and its current extensions. This Call for Proposals (CfP) has been issued by the Joint Video Experts Team (JVET) on behalf of its ITU-T and ISO/IEC parent bodies (ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29). The evaluation of submissions is planned to be conducted during the JVET meeting in January 2027.

# Purpose and procedure

The CfP targets a new generation of video compression technology that substantially exceeds the VVC standard in terms of compression, encoder/decoder implementability, applicability to a variety of content, and other features such as transmission latency/robustness, scalability, etc.

Example sources include camera-captured content, computer-generated content, user-generated content, and high-dynamic-range content, while example applications include broadcast (both live or pre-authored content), real-time video conferencing, video chat, on-demand viewing, gaming, video upload, storage-based media replay, and surveillance with fixed or moving cameras. More background information, as well as information about applications and requirements, is given in [1].

The CfP requests submission of video compression technology that has compression performance or additional functionality beyond that of VVC, where the trade-off in terms of implementation cost is an important criterion as well. As it is recognized that in a growing number of applications practical fast encoding is required, information on this aspect is also requested in the context of the CfP and will be evaluated separately.

Multiple test cases are defined: one test case for improved compression at large and three test cases for improved compression with runtime-constrained encoding. For each test case, test sequences are organized in test categories which represent a content type and an associated encoding configuration. Submitters are encouraged (but not required) to submit results for all test cases. However, submitters are requested to provide complete results for all categories in a given test case.

Responses to the CfP will be evaluated at the 45th JVET meeting in January 2027, as further described below.

Companies and organizations who have developed compression technology that they believe to be superior to the Main 10 Profile of the VVC standard regarding the aforementioned aspects are kindly invited to propose such technology to the JVET in response to this CfP. Additionally, contributions are also welcome regarding technology that better supports newly emerging application areas of video coding by additional functionality (see section 6 of this document).

Formal subjective tests will be conducted to evaluate the proposed compression technologies. Results of these tests will be made public (although no direct identification of the proponents will be made in the report of the results unless it is specifically requested or authorized by a proponent to be explicitly identified and a consensus is established to do so). No commitment to any course of action regarding the proposed technology can be made prior to having evaluated the results of the tests.

Descriptions of proposals shall be registered as JVET input documents to the meeting of January 2027 (see the timeline in section 3). Proponents are requested to attend this meeting to present their proposals. Further information about logistics attending the meeting can be obtained from the listed contact persons (see section 11).

# Timeline

The timeline of the Call for Proposals is as follows:

2026-05-31:	VTM anchors, runtime-constrained VTM encodings, additional VTM encodings with 
			RPR enabled, and VTM encoder configuration information available

2026-07-17:	Call for Proposals

2026-08-01: 	Formal registration period opens.

2026-09-01:	Formal registration period ends.

2026-09-07:	Final fee (see section 8) is determined and a formal offer for conducting the tests is sent by the test coordinator.

2026-10-26:	Main submission package (see description in section 9A) shall be made available at the site of the test coordinator. By this date, confirmation of the purchase order shall be received.

2026-11-02:	Subjective assessment starts (planned to be finalized by 2026-12-21).

2026-11-02:	Set of additional sequences provided to the proponents.

2026-11-30:	The main submission package is made available to cross-checkers (participation mandatory for proponents, coordinated by the JVET chair and the test coordinator).

2026-12-21:	Supplemental submission package (which includes bitstreams and associated materials for the set of additional sequences, see description in section 9B) shall be made available to the test coordinator.

2026-12-23:	The supplemental submission package is made available to cross-checkers (participation mandatory for proponents, coordinated by the JVET chair and the test coordinator; cross-check should be finalized by 2027-01-13).

2027-01-06:	Registration and submission of documents describing the proposals.

2027-01-06:	Reporting of the cross-checking results to the JVET chair and test coordinator

2027-01-13:	Summary of subjective and objective test results are made available to proponents and within JVET.

2027-01-13/22:	Evaluation of proposals at JVET meeting

Anticipated tentative timeline after CfP:

(referring to the first version of an anticipated new standard, which may be extended later in subsequent work):

2027-01	Initial test model selection process begins

2027-10	Initial test model selection completed (at latest)

2029-10	Final standard completed

# Test case on improved compression

The following categories are considered. Each category name includes a characterization of the content type, the suggested encoder configuration and an indication of the resolution.

- SDR RA UHD/4K: Representing the use case of distribution of standard dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

- SDR RA HD: Representing the use case of distribution of standard dynamic range HD video content e.g. in a streaming scenario, using a random-access configuration.

- SDR LB HD: Representing the use case of conversational and other low delay applications at HD resolution, using a low-delay configuration.

- HDR-PQ RA UHD: Representing the use case of distribution of high dynamic range UHD/4K/8K video content using the PQ transfer function e.g. in a streaming scenario, using a random-access configuration. Cropped regions of 3840×2160 resolution are used for 4K and 8K content to reduce the encoding workload for assessment of this category and allow investigation on 4K displays.

- HDR-HLG RA UHD: Representing the use case of distribution of high dynamic range UHD/4K/8K video content using the HLG transfer function e.g. in a streaming scenario, using a random-access configuration. Cropped regions of 3840×2160 resolution are used for 4K and 8K content to reduce the encoding workload for assessment of this category and allow investigation on 4K displays.

- Gaming LB HD/UHD: Representing the use case of online gaming with a low-delay configuration.

- UGC RA: Representing the use case of user-generated content at 1080×1920 or 1920×1080 resolution using a random-access configuration.

Formal subjective testing of sequences from the main submission package will be conducted only for rates 1-4. The points at higher rates will only be used to investigate the gain based on objective metrics over broader ranges of rate and quality and will also be considered to determine overall encoder and decoder run times. Depending on the number of submissions, it may be necessary to select a subset of representative video sequences for the subjective assessment if otherwise it would not be possible to complete such assessment within the timeline foreseen for the subjective tests. This subset would be selected by the JVET chair and the JVET test coordinator to represent as many test categories as possible. Assessment based on objective metrics will be done for all submitted sequences (including those in the additional set) and all rate points.

## General coding conditions

The following rules apply:

- Encoded bitstreams shall not exceed the target bit rates defined for each of the sequences.

- Sequences are expected to be encoded at full input resolution for all rate points. If coding at reduced resolution is part of the algorithmic concept, it shall be described.

- Quantization settings should be kept static. When quantization parameters or more generally rate-distortion parameters (e.g., lambda) change from frame to frame (e.g., QP offset dependent on temporal ID) or within a frame, it shall be described.

- Notwithstanding the previous rule, a small one-time change of the quantization and/or rate-distortion parameter settings to meet the target bit rate is allowed and must be documented. Only a modification towards a lower bitrate for the remaining part of the bitstream is permitted.

- Optimizing encoding parameters using non-automatic means as well as optimizing encoding parameters on a per sequence basis within a category is discouraged.  When any such optimization is employed, it shall be described.

- No part of test sequences shall be used as a training set or part of a training set for training entropy coding tables, VQ codebooks, transforms, predictors, filters, neural network models, etc. Furthermore, it is mandatory to report which material was used to train corresponding parts of the algorithm.

- The use and the compression performance impact of preprocessing and/or postprocessing shall be described. Preprocessing includes any modification of the original test sequence before being input to the encoder and any process that is required to determine any parameter that is signaled in the bitstream, except for the parameters (e.g., quantization parameters) that are derived for the purpose of rate matching. Postprocessing includes any process outside the main decoding loop that is required to produce the reconstructed YUV sequences. Encoder and decoder run times shall include time spent for preprocessing and postprocessing as applicable (e.g., time spent for postprocessing is included in the decoder run time but not the encoder run time if postprocessing is not required to produce the bitstreams).

- The use and the compression performance impact of perceptual optimization shall be described.

- The use and the compression performance impact of multi-pass encoding shall be described.

If any preprocessing, postprocessing or encoder optimization is used in a submission, it is encouraged to provide results obtained by applying equivalent methods to the anchor. Such information may be requested at the evaluation meeting in January 2027, or during subsequent investigations.

A set of additional sequences and associated target rate and quality ranges will be announced to the proponents after the submission of decoder binaries and bitstreams for the sequences listed in sections 4.2-4.8. The sequences in this set will not be included in the formal subjective evaluation conducted for the proposals. It is expected that the amount of content to be processed will be 50% or less compared to all the test sequences described in this section. The additional sequences will feature resolutions, structural delay constraints, bitrates, and content type similar to the test sequences and coding conditions described in this section. Bitstreams submitted for the additional sequences are required to be within 80-100% of the defined target bitrates. Bitstreams, information about encoder configuration settings, and results for this additional set shall be provided by 2026-12-21. Those bitstreams are expected to be encodable and decodable using the binary executables previously submitted.

For each bitstream, run time is defined to be the total processing time to generate the bitstream and includes processing time for any pre-processing operations. Where multithreading is used, the run time is the sum of the run times of each thread. Design parts affected by multithreading must be described. If segment-wise parallel encoding or decoding is used, it is recommended to report the sum of the segment run times as the total run time. Run times for both the anchor and the proposal must be measured using the same methodology.

The aggregate encoder and decoder run times across multiple sequences and all rate points (including those not used in visual testing) shall be computed as follows: 1) run times for all rate points are summed for each sequence; 2) the geometric mean is computed across all sequences; 3) the ratio to the anchor runtime is computed. Alternate ways to aggregate run times may be additionally used when evaluating responses. For example, run times may be aggregated across a single rate point (e.g., the highest rate point). Sequences from both the main and the supplemental submission packages will be used to compute the aggregate run times, either separately or in combination.

A cross-checking process will be conducted to validate both the main and supplemental submission packages. Cross-checking will be performed by other parties without identification of the respective proposing party. The cross-check is intended to verify the consistency of the submitted materials using the submitted encoders, decoders, and configuration settings, as well as the correctness of the reported metrics.

## SDR RA UHD/4K

### Sequence formats and target bit rates

All sequences have a picture size of 3840×2160, chroma format 4:2:0 YCbCr according to ITU-R BT.709, and bit depth 10. Further information can be found in Table 1.

Table 1: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRU1 | CrowdRun | 10 | 500 | 50 | 17282413c9a523a02e0c8463c097221f |
| SRU2 | DrivingPOV3 | 10 | 600 | 60 | e81b65724c4235128b2749ccb3b0fb4a |
| SRU3 | FireDance | 10 | 250 | 25 | bfcfc333924835aaf4ed89efa8428a36 |
| SRU4 | HallwayScene | 10 | 250 | 25 | b0c8718579998060e375fcdb77243013 |

Target bit rates for the encoding of the sequences are listed in Table 2.

Table 2: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SRU1 | CrowdRun | 700 | 1500 | 3200 | 7000 | 14000 |
| SRU2 | DrivingPOV3 | 300 | 600 | 1200 | 2400 | 4800 |
| SRU3 | FireDance | 400 | 800 | 1500 | 2500 | 5000 |
| SRU4 | HallwayScene | 150 | 250 | 500 | 1000 | 2000 |

### Coding conditions for VVC anchors

The general description in section 4.1 applies.

In this test category, a random-access scenario (RA) is used for evaluation. The encoder configuration follows the settings provided in the JVET common test conditions and software reference configurations [3]. The intra refresh period is dependent on the frame rate of the source: the value 32 shall be used for sequences with a frame rate equal to 24fps, 25fps or 30fps, and the value 64 for 50fps and 60fps.

### Coding conditions for submissions

Submissions to the Call for Proposals shall obey the following rules:

- The general rules as described in section 4.1.

- Allow for random access at intervals not larger than the intra refresh period of the respective anchor. The structure of the intra period shall be documented. The random access functionality shall enable decoding the bitstream after discarding any and all information before the start of the random access point.

## SDR RA HD

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080, chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 3.

Table 3: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SRH1 | DucksTakeOff | 8 | 500 | 50 | 3504582275e36e310fae72ffa6a0f8aa |
| SRH2 | TravellerSwim | 10 | 500 | 50 | d79cc6457e82f28ec3397adda4bede13 |
| SRH3 | Seeking | 8 | 500 | 50 | 0f26ad79d085895299860530e145a8c9 |
| SRH4 | Umbrella | 8 | 500 | 50 | 87572379ef39bedf55f9c9d8152086e7 |

Target bit rates for the encoding of the sequences are listed in Table 4.

Table 4: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SRH1 | DucksTakeOff | 300 | 900 | 2400 | 4000 | 8000 |
| SRH2 | TravellerSwim | 150 | 300 | 600 | 1200 | 2400 |
| SRH3 | Seeking | 200 | 400 | 800 | 1600 | 3200 |
| SRH4 | Umbrella | 300 | 600 | 1400 | 3500 | 7000 |

### Coding conditions for VVC anchors

The description in Section 4.2.2 applies.

### Coding conditions for submissions

The description in Section 4.2.3 applies.

## SDR LB HD

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 5.

Table 5: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| SLH1 | Beatriz L | 8 | 500 | 50 | fe74cd5046fa033b4f743f42b29e69cd |
| SLH2 | GregoryCactus2 P | 10 | 300 | 30 | 4fb703c30a3fb06bacef461c69908b93 |
| SLH3 | GregoryScarf2 P | 10 | 300 | 30 | 7608f5bf81b4f3f11d0c7a489a5ff26c |
| SLH4 | OfficeWalkAtWall L | 8 | 300 | 30 | 529c15491ea8e1eb0320244a6ff902bb |

Target bit rates for the encoding of the sequences are listed in Table 6.

Table 6: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| SLH1 | Beatriz | 70 | 140 | 280 | 550 | 1100 |
| SLH2 | GregoryCactus2 | 200 | 600 | 1500 | 4000 | 8000 |
| SLH3 | GregoryScarf2 | 200 | 600 | 1800 | 5000 | 10000 |
| SLH4 | OfficeWalkAtWall | 90 | 200 | 450 | 1000 | 2000 |

### Coding conditions for VVC anchors

The general description in section 4.1 applies.

In this test category, a low-delay scenario with B pictures (LB) is used for evaluation. The encoder configuration follows the settings provided in the JVET common test conditions and software reference configurations [3]. No picture reordering is applied between decoder processing and output.

### Coding conditions for submissions

Submissions to the Call for Proposals shall obey the following rules:

- The general rules as described in section 4.1.

- Overall structural delay shall not be larger than that of the respective anchor. The encoder, including any preprocessing steps, shall process pictures in their display order and not use any picture look-ahead.

## HDR-PQ RA UHD

### Sequence formats and frame rates

UHD, 4K and 8K HDR PQ test sequences with chroma format 4:2:0 YCbCr, bit depth 10 bit, according to ITU-R BT.2100, with the applicable transfer function indicated. Sequences of 4K or 8K resolution have been cropped to a picture size of 3840×2160. Further information can be found in Table 7.

Table 7: Test sequences

| Sequence ID (SID) | Sequence name | Transfer fct. | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| HPQ1 | ChandelierCropBR | HDR10 PQ | 360 | 60 | bb42a5d627330c652f19aca4b184b47f |
| HPQ2 | FashionLadyCrop1 | HDR10 PQ | 380 | 60 | d8546edaa260468f027c0e1c28d422b0 |
| HPQ3 | MeridianHDR2 | P3 PQ 4000nits | 600 | 60 | 9e889c7d78b1b5a0fdebb88871a362dd |
| HPQ4 | SparksWelding | HDR10 PQ 1000nits | 600 | 60 | 7e0c6bd867c370dde9b11c76ed93409f |

Target bit rates for the encoding of the sequences are listed in Table 8.

Table 8: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| HPQ1 | ChandelierCropBR | 300 | 650 | 1300 | 2800 | 5600 |
| HPQ2 | FashionLadyCrop1 | 250 | 650 | 1700 | 4500 | 9000 |
| HPQ3 | MeridianHDR2 | 150 | 300 | 600 | 1200 | 2400 |
| HPQ4 | SparksWelding | 400 | 1000 | 2500 | 6000 | 12000 |

### Coding conditions for VVC anchors

In this test category, the random-access scenario is used for evaluation. The description in Section 4.2.2 applies, with the encoder settings in common test conditions for HDR/WCG content being used [4].

### Coding conditions for submissions

The description in Section 4.2.3 applies.

## HDR-HLG RA UHD

### Sequence formats and frame rates

UHD, 4K and 8K HDR HLG test sequences with chroma format 4:2:0 YCbCr, bit depth 10 bit, according to ITU-R BT.2100, with the applicable transfer function indicated. Sequences of 4K or 8K resolution have been cropped to a picture size of 3840×2160. Further information can be found in Table 9.

Table 9: Test sequences

| Sequence ID (SID) | Sequence name | Transfer fct. | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| HLG1 | WaterfallForest | HLG10 | 500 | 50 | 54abb97e78a7255885fc283d02f9c964 |
| HLG2 | WomenFootball | HLG10 | 500 | 50 | 94bb484ab23ac900d1f417c52972535f |
| HLG3 | AMS06 | HLG10 | 600 | 60 | d1f11c771febbb8a2bbb7faadc13cbbb |
| HLG4 | SeaWalk | HLG10 | 500 | 50 | c6640bb1c3cdef1d653bd54271cfe49c |

Target bit rates for the encoding of the sequences are listed in Table 10.

Table 10: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| HLG1 | WaterfallForest | 1000 | 2500 | 6000 | 14000 | 28000 |
| HLG2 | WomenFootball | 300 | 600 | 1100 | 2000 | 4000 |
| HLG3 | AMS06 | 600 | 1300 | 3500 | 8000 | 16000 |
| HLG4 | SeaWalk | 200 | 400 | 800 | 1800 | 3600 |

### Coding conditions for VVC anchors

The description in Section 4.5.2 applies.

### Coding conditions for submissions

The description in Section 4.5.3 applies.

## Gaming LB HD/UHD

### Sequence formats and frame rates

All sequences have chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 11.

Table 11: Test sequences

| Sequence ID (SID) | Sequence name | Picture size | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- | --- |
| GLH1 | DOTA2s360 | 1920×1080 | 8 | 550 | 60 | 999ae67b022892a7b70ab4dcc70f1043 |
| GLH2 | GTAVs090 | 1920×1080 | 8 | 600 | 60 | 43643d686a91a51758c63346fab7eca3 |
| GLH3 | Level1 | 1920×1080 | 10 | 600 | 60 | 87c7055771366d16b0d69cb56f6c66fb |
| GLH4 | Minecraft | 1920×1080 | 8 | 600 | 60 | 4f1055d558284e789fcae7686f3419bc |
| GLU5 | Wukong2 | 3840×2160 | 10 | 600 | 60 | 8da88d4168684a66623616d2d1112211 |
| GLU6 | Carla5 | 3840×2160 | 8 | 600 | 60 | 11db5b51ad88b53ab46c0d7953601e8c |

Target bit rates for the encoding of the sequences are listed in Table 12.

Table 12: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| GLH1 | DOTA2s360 | 180 | 300 | 550 | 1000 | 2000 |
| GLH2 | GTAVs090 | 400 | 900 | 2000 | 3600 | 7200 |
| GLH3 | Level1 | 400 | 1000 | 2000 | 4000 | 8000 |
| GLH4 | Minecraft | 300 | 600 | 1200 | 2400 | 4800 |
| GLU5 | Wukong2 | 1000 | 2400 | 6000 | 14000 | 28000 |
| GLU6 | Carla5 | 1100 | 2200 | 4300 | 8500 | 17000 |

### Coding conditions for VVC anchors

The general description in section 4.1 applies.

In this test category, a low-delay scenario with B pictures (LB) is used for evaluation and follows the encoder settings provided in the JVET common test conditions and software reference configurations [3]. No picture reordering is applied between decoder processing and output.

### Coding conditions for submissions

The description in Section 4.4.3 applies.

## UGC RA

### Sequence formats and frame rates

All sequences have a picture size of 1920×1080 (landscape orientation, L) or 1080×1920 (portrait orientation P), chroma format 4:2:0 YCbCr according to ITU-R BT.709. Further information can be found in Table 13.

Table 13: Test sequences

| Sequence ID (SID) | Sequence name | Bit depth | Frame count | Frame rate | MD5Sum |
| --- | --- | --- | --- | --- | --- |
| URH1 | Camellia P | 8 | 600 | 60 | 6e5413666e42d3c24db9b81b5db641ff |
| URH2 | Hobby-w5xz-backpack P | 8 | 240 | 24 | b8ee8e98b5d34afcfc24a9d02d8f3a43 |
| URH3 | Sports-76a2-iceball L | 8 | 600 | 60 | 3030f3760f34ce97e67d7e1a02eedd3b |
| URH4 | VerticalVideo-3709-snow P | 8 | 300 | 30 | 3db32dc2c295dc25622ebc516420c63b |

Target bit rates for the encoding of the sequences are listed in Table 14.

Table 14: Target bit rates

|  |  | Target bit rates [kbit/s] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SID | Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| URH1 | Camellia | 200 | 400 | 800 | 1600 | 3200 |
| URH2 | Hobby-w5xz-backpack | 90 | 160 | 280 | 500 | 1000 |
| URH3 | Sports-76a2-iceball | 80 | 160 | 250 | 400 | 800 |
| URH4 | VerticalVideo-3709-snow | 80 | 160 | 300 | 500 | 1000 |

### Coding conditions for VVC anchors

The description in Section 4.2.2 applies.

### Coding conditions for submissions

The description in Section 4.2.3 applies.

# Test cases on improved compression with runtime-constrained encoding

The categories and sequences defined in sections 4.2 – 4.8 are considered.

Submitters are strongly encouraged (but not required) to submit results for all three test cases. However, submitters are requested to provide results for all categories in a given test case.

## Sequence formats and frame rates

For each category, sequence formats and frame rates are identical to those defined in section 4.

## Coding conditions for VVC anchors

For each category, the coding conditions for the anchor are identical to those defined in the corresponding parts of section 4, except as noted below.

For VVC, bitstreams are generated for the following VTM configurations documented in [3]:

- Default (equivalent to the one used in section 4)

- High performance (encoding times are approximately 2 times those of the default)

- Reduced encoding time (3 variants, encoding times are approximately 0.2 to 0.75 times those of the default)

VTM with default configurations is the anchor with respect to which relative compression performance and run times are calculated.

## Coding conditions for submissions

Submissions to the Call for Proposals shall obey the following rules:

- For each category, the rules defined in the corresponding part of section 4 apply.

- Encoder run times must be in the range of targets defined below and should match the runtime targets as closely as possible.

- No target is defined for decoder run times. However, decoder run times are to be reported and may be considered in the evaluation of this Call.

- Code optimization beyond what is typically done for reference software such as VTM is discouraged. The level of code optimization shall be described.

Three encoder run time targets are defined as:

- 5 times (500%) the encoder run time of the VTM anchor

- Same (100%) as the encoder run time of the VTM anchor

- 0.2 times (20%) the encoder run time of the VTM anchor

The run time target is defined for the aggregate run time across all bitstreams. It is not required to meet to the target run time for each bitstream. It is discouraged to tune encoder configurations for each bitstream such as to meet the target run time for every bitstream.

For the purpose of matching run time targets, the aggregate run time defined in section 4.1 shall be used.

Matching the run time targets exactly is not necessary. Run times should be sufficiently close to the target to be able to plot an encoder run time vs compression performance curve for each submission and have this curve cover a range similar to the one spanned by the three run time targets defined above.

It is strongly encouraged to include a set of bitstreams and results for each of the three run time targets in a response. Additional run time vs compression trade-offs may be included in a response such as to have a more densely sampled curve. For example, results for run times approximately 0.5 times or 2 times those of the VTM anchor may be included.

# Technology supporting additional functionality

Companies and organizations who have developed compression technology that provides additional functionality with a better compression trade-off than existing standards are also invited to submit such information in the context of this Call for Proposals. Proposals on how the benefit of such functionality could be assessed during the development of a video compression standard is welcome. For examples of such additional functionality, you may refer to [1].

Submissions to this section are not part of the main submission package and will not be part of the formal subjective test. Potential expert viewing or informal demonstration could be conducted at a JVET meeting. Aspects of proposals related to this section are to be submitted by the document deadline of the 45th JVET meeting and are not subject to mandatory crosscheck before the meeting.

# Test sites and delivery of test material

The material submitted for proposals will be evaluated by means of a formal subjective assessment process. The tests will be coordinated by the test coordinator and conducted at one or more test sites.

All proponents need to deliver, by the due date of 2026-10-26, an SSD to the address of the Test Coordinator (see section 11; see section 9A) for the requested contents of the SSD).

The correct reception of the material will be confirmed by the Test Coordinator. Any inconvenience caused by an unexpected delivery delay or a failure of the disk will be under the complete responsibility of the proponents. However solutions will be negotiated to ensure that the data can still be included in the test if feasible, which means that correct and complete data need to be available before the beginning of the test at the latest.

All bitstreams, the decoder executable, and the YUV files shall be accompanied by an MD5 checksum file to verify their correct storage on the disk.

Further technical details on the delivery of the coded material will be provided by the test coordinator.

# Testing fee

A testing fee will be charged for each response to this Call for Proposals that requires the formal subjective test. The sole purpose of the testing fee is to cover costs to conduct the subjective assessment which requires the hiring of test subjects. The final amount of the fee will be determined when the number of registered proposals is known. The fee will be in proportion to the amount of subjective testing conducted for all test cases submitted in a given response.  Submission of a partial test case will be considered as a full test case for determining the requested fee. A maximum of EUR 20k is expected per test case submitted for subjective evaluation.

Proponents can indicate how many of the submitted case(s) should be included in the subjective evaluation. It will be decided during the 44th JVET meeting in October 2026 which of the cases available for a given proposal will be included in the subjective test, such that the benefit in making different proposals comparable is maximized.

# Requirements for submissions

Proponents shall provide the following; incomplete proposals may not be considered:

- Main submission package to be received by 2026-10-26:

- Bitstreams for all submitted test cases.

- Binary decoder and encoder executables including relevant encoder configuration settings for all submitted test cases. A description of how to use the software (command line, configuration parameters, etc.) shall also be included. The decoder must be capable of decoding the bitstreams and storing the reconstructed data in 10-bit YUV 4:2:0 uncompressed format as needed for the subjective evaluation. Executables should be built to run on Ubuntu 24.04 (x86-64 v3). A single decoder executable shall be provided for all test cases. A decoder executable should require only two input parameters to reconstruct the YUV file from a bitstream file, for example: “<decoder_exe> -b <bitstream_filename> -o <yuv_filename>”.

- Reconstructed sequences (YUV files 4:2:0 packed uncompressed format) for all submitted test cases.

- For evaluation by objective metrics, CSV files containing the values listed in annex D.

- MD5 checksums for all submitted files. Reporting these checksums in one aggregated file is encouraged.

- Supplemental submission package to be received by 2026-12-21:

- Bitstreams and relevant encoder configuration settings for all submitted test cases.

- Reconstructed sequences (YUV files 4:2:0 packed uncompressed format) for all submitted test cases.

- For evaluation by objective metrics, CSV files containing the values listed in annex D.

- MD5 checksums for all submitted files. Reporting these checksums in one aggregated file is encouraged.

- Document to be submitted before the JVET meeting in January 2027 shall contain

- A technical description of the proposal with sufficient detail for full conceptual understanding and generation of equivalent performance results by experts and for conveying the degree of optimization required to replicate the performance. This description should include all data processing paths and individual data processing components used to generate the bitstreams. It does not need to include complete bitstream format or implementation details, although as much detail as possible is desired.

- The technical description shall also contain statements about the programming language in which the software is written, e.g. C/C++, for the use external libraries, and the platform(s) on which the binaries can be compiled.

- The technical description shall state how the proposed technology behaves in terms of random access to any picture within the sequence. For example, a description of the GOP structure and the maximum number of pictures that must be decoded to access any picture could be given.

- The technical description shall specify the expected encoding and decoding delay characteristics of the technology, including structural delay e.g. due to the amount of picture reordering and DPB buffering, the degree of picture-level multi-pass decisions and the degree by which the delay can be minimized by parallel processing.

- The technical description shall contain information suitable to assess the complexity of the implementation of the technology:

- Encoding and decoding run time relative to the VTM anchors, measured using the same simulation environment. Proponents shall provide a description of the platform and methodology used to determine the time. To help interpretation, a description of software optimizations undertaken, if any, is encouraged.

- Complexity characteristics of encoder and decoder by filling the complexity reporting template.

- Degree of capability for parallel processing.

- Furthermore, the technical description should point out any specific properties of the proposal (e.g., additional functionality such as benefit for 4:4:4 coding, error resilience, scalability).

- Optional information

Proponents are encouraged (but not required) to attach the CSV files for all submitted test cases and for all test sequences in the main and additional test sets as specified in Annex D to the report document.

Proponents are encouraged (but not required) to allow members of JVET to have access, on a temporary or permanent basis, to their encoded bitstreams and binary executables or source code.

Any additional information about the properties and performance of the algorithm or specific tools is welcome.

More information about file formats can be found in Annex A. Files of decoded sequences and bitstreams shall follow the naming conventions as specified in Annex A.

# Subsequent provision of source code and IPR considerations

Proponents are advised that, upon acceptance for further evaluation, it will be required that relevant parts of technology proposed be made available in source code format to participants in the core experiments process and for potential inclusion in the prospective standard as reference software. When a particular technology becomes a candidate for further evaluation, commitment to provide such software is a condition of participation. The software shall produce identical results to those submitted to the test. Additionally, submission of improvements (bug fixes, etc.) is certainly encouraged.

In the above, “relevant parts of technology proposed” may include training scripts or equations used to derive parameters of a tool or algorithm. It is also expected that training materials used in the preparation of a proposal would be made available for further investigation and verification in JVET, or re-training would need to be conducted using alternative material that is available to JVET.

Furthermore, proponents are advised that this Call is being made subject to the common patent policy of ITU-T/ITU-R/ISO/IEC (see http://www.itu.int/ITU-T/dbase/patent/patent-policy.html or the ISO/IEC Directives Part 1 with Consolidated JTC 1 Supplement, Appendix I) and the other established policies of the standardization organizations.

# Contacts

Prospective contributors of responses to the Call for Proposals should contact the following people:

Jens-Rainer Ohm (JVET chair)

RWTH Aachen University, Institute of Communications Engineering

Melatener Str. 23, 52074 Aachen, Germany

Tel. +49-241-8027671, email ohm@ient.rwth-aachen.de

Mathias Wien (JVET test coordinator)

RWTH Aachen University, Institute of Imaging and Computer Vision

Kopernikusstr. 16, 52074 Aachen, Germany

Tel. +49-241-8027867, email wien@lfb.rwth-aachen.de

Registration of a response shall be made by submitting the registration form in Annex E to the persons above on or before September 1, 2026. Interested parties are kindly invited to express their intent as early as possible.

Details on how to format and submit documents, bitstreams, and other required data will be communicated directly to those who register a response. Additionally, the JVET chair will provide assistance to submitters from outside JVET in order for them to attend the JVET meeting in January 2027.

Test sequences, anchors, and configuration information will also be made available by contacting the persons above (see section 3 for availability dates).

# References

- “Use cases and requirements for next-generation video coding standard beyond VVC capability”, ISO/IEC SC 29/WG 2 document N 512 and ITU-T Q6/21 document VCEG-CA20, April 2026.

- “Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)”, Joint Video Experts Team (JVET), 41st Meeting, by teleconference, January 2026, Doc. JVET-AO2002.

- “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, Joint Video Experts Team (JVET), 42nd Meeting, Santa Eularia, ES, April/May 2026, Doc. JVET-AP2010.

- “VTM and HM common test conditions and evaluation procedures for HDR/WCG video”, Joint Video Experts Team (JVET), 41st Meeting, by teleconference, January 2026, Doc. JVET-AO2011.

- ITU-T, Working practices using objective metrics for evaluation of video coding efficiency experiments, Doc. HSTP-VID-WPOM, 2020.

- International Telecommunication Union – Radiocommunication Sector, Methodologies for the subjective assessment of the quality of television images, Recommendation ITU-R BT.500-15 (available at https://www.itu.int/rec/R-REC-BT.500).

- International Telecommunication Union – Telecommunication Standardization Sector, Subjective video quality assessment methods for multimedia applications, Recommendation ITU-T P.910 (available at https://www.itu.int/rec/t-rec-p.910).

- “A Packed Planar RGB and YUV Format for Uncompressed Storage of High Dynamic Range Still Images and Videos,” https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/blob/master/doc/pyuv_format.pdf.

Distribution formats for test sequences, decoded results, and bitstreams

Distribution of original video material files containing test sequences is done using YUV files with extension “.yuv”. YUV files are header-less and uncompressed. Each frame in the file comprises an array of M×N luma (Y) samples followed by an array of (M/2)×(N/2) chroma (U) samples and by an array of (M/2)×(N/2) chroma (V) samples, where M is the width of the frame and N its height. Within each array, samples are ordered in raster-scan order starting from the top-left sample. If the bit depth is 8, each sample comprises one byte. If the bit depth is 10, each sample comprises two bytes, where the first byte represents the sample value modulo 256, and the second byte the sample value divided by 256.

VVC Anchor bitstreams are provided with extension “.bit”. Bitstream formats of proposals can be proprietary, but must contain all information necessary to decode the sequences at a given data rate (i.e., no additional parameter files shall be needed). The file extension of a proposal bitstream shall be “.bit”.

Reconstructed sequences shall be provided in the “.pyuv” format [8], with 10 bits per component for all sequences. The format output by the binary decoder executable shall be either “.yuv” or “.pyuv”, with 10 bits per component.

Filenames must follow the following format: xxxx_Pyy_Rz_Cw.eee where

- xxxx is the sequence ID, as defined in section 4

- yy is a proponent ID to be provided upon registration, the value 00 is reserved for the VVC anchor

- z is a rate target (1 to 5)

- w is a test case (0 = improved compression, 1 = runtime constraint 5x, 2 = runtime constraint 1x, 3 = runtime constraint 0.2x, 4.. = additional runtime constraints (optional))

- eee is an extension (pyuv for reconstructed packed YUV files, bit for bitstream files, csv for CSV files)

Description of testing environment and methodology

The test method adopted for this evaluation is degradation category rating (DCR) [7].

Degradation Category Rating (DCR)

The degradation category rating (DCR) method presents stimuli in pairs. The first stimulus presented in each pair is always the reference. The second stimulus is that reference stimulus after processing by the systems under test. DCR is a double stimulus method. The DCR method is also known as the double stimulus impairment scale (DSIS) method defined in ITU-R BT.500 [6].

An impairment rating scale made of 11 levels is planned to be used, ranging from “0” (severely annoying) to “10” (imperceptible).

The structure of the basic test cell (BTC) of the DCR method includes the presentation of the uncompressed reference of the video sequences followed by the Processed Video Sequence (PVS) under evaluation. Then a message displays for 5 seconds asking the viewers to vote. The presentation of the video clips is preceded by a mid-grey screen displaying e.g. “Source” for the original and “Test” for the coded version of the sequence under test for one second. During the voting time, the number of the vote is indicated on the screen.

Figure B.1 – Structure of a DCR BTC

Training and stabilization phase

The outcome of a test is highly dependent on a proper training of the test subjects.

For this purpose, each subject has to be trained by means of a short practice (training) session demonstrating the range of qualities to be expected in the test.

The stabilization phase uses the test material of a test session, e.g. three BTCs containing one sample of best quality, one of the worst qualities and one of medium quality, are presented at the beginning of the test session. In this way, the test subjects have an immediate impression of the quality range they are expected to evaluate during that session.

The scores of the stabilization phase are discarded for the evaluation.

The laboratory setup

The laboratories for subjective assessments are supposed to comply with established recommendations [6][7]. Play-out of the video sequences must be done at native resolution.

When a video sequence is shown at a resolution lower than the native resolution of the display itself, the video has to be presented in the center of the display; the active part of the display (i.e. that is actually showing the video signal) must have a dimension equal in rows and columns to the raster of the video; the remaining part of the screen has to be set to a mid-grey level (e.g., 128 in the 0-255 range). This constraint guarantees that no interpolation or distortion artefacts of the video images will be introduced.

The video play server, or the PC used to play video has to be able to support the display of both HD and UHD video formats, at 24, 25, 30, 50 and 60 frames per second, without any limitation, or without introducing any additional temporal or visual artefacts.

Viewing environment

The viewing distance is set to 1.5H, where H is equal to the height of the active part of the screen, depending on the size of the active part of the screen and its native resolution.

The test rooms are expected to be protected from external visual or audio pollution. Internal general light must be low (just enough to allow the viewing subjects to fill out the scoring sheets) and no direct light must be visible to the viewing subjects seated in front of the screen; the light behind the monitor should be dimmed to an intensity and should have a colour tuned as close as possible to D65. No other light source, in particular any light source directed to the screen or creating reflections are permitted.

Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, will be stored in a database for further processing.

For each coding condition, the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values will be computed and reported.

The MOS and CI values will be used to draw graphs. The graphs will be drawn grouping the results for each video test sequence. No graph grouping results from different video sequences will be considered.

Using HDRTools to compute distortion metrics

This annex describes the usage of HDRTools to compute distortion metrics. The HDRTools software package can be downloaded from https://gitlab.com/standards/HDRTools. Building instructions are provided in the README.md file included in the HDRTools software package.

Version 0.26 of the HDRTools software should be used. Note that Version 0.26 of the HDRMetrics utility may produce different results on different platforms when non-integer frame rates are provided. To ensure consistent results of the HDRMetrics utility across different platforms, use of an integer frame rate input parameter is recommended. The value of the frame rate parameter should not affect results of HDRMetrics computation.

To compute the PSNR, wPSNR, and MS-SSIM metrics using the HDRMetrics utility, the following command is used from the root HDRTools directory:

build/bin/HDRMetrics \
-f cfg/HDRMetricYUV.cfg \
-p Input0File=<original.yuv> \
-p Input1File=<decoded.yuv> \
-p Input0Height=<height> \
-p Input0Width=<width> \
-p Input1Height=<height> \
-p Input1Width=<width> \

-p Input0BitDepthCmp0=<bitdepth of original.yuv> \
-p Input0BitDepthCmp1=<bitdepth of original.yuv> \
-p Input0BitDepthCmp2=<bitdepth of original.yuv> \
-p Input1BitDepthCmp0=10 \
-p Input1BitDepthCmp1=10 \
-p Input1BitDepthCmp2=10 \
-p NumberOfFrames=<num_frames> \
-p EnableWTPSNR=1 \
-p EnableJVETPSNR=1 \

-p EnableJVETMSSSIM=1 \
-p WeightTableFile=cfg/hdrTable.txt \
-p Input0Rate=60 \
-p Input1Rate=60 \

-p MaxSampleValue=1023.0

The distortion values that are to be reported (see Annex D) are listed on the output line starting with “D_Avg”.

Results data submission format

Results data shall be submitted in the CSV format using the semicolon (“;”) as separator. For each sequence and each rate point, the following information should be provided, as applicable. When submitting data for runtime-constrained encoding as per section 5, a separate CSV file shall be provided for each target run time.  The precision of all reported metric values as indicated in the tables below shall be followed (these match the precision of the default output provided by HDRTools).

| Column name | Description | Decimal places |
| --- | --- | --- |
| Sequence | Sequence ID, e.g., SRU1 | na |
| Rate point | Rate point, e.g., R1 | na |
| Bitrate | Bitrate in kbit/s | 4 |
| PSNR Y | PSNR metric for Y component (See Annex C and [5]) | 3 |
| PSNR U | PSNR metric for U component (See Annex C and [5]) | 3 |
| PSNR V | PSNR metric for V component (See Annex C and [5]) | 3 |
| SSIM Y | MS-SSIM metric for Y component (See Annex C) | 5 |
| SSIM U | MS-SSIM metric for U component (See Annex C) | 5 |
| SSIM V | MS-SSIM metric for V component (See Annex C) | 5 |
| WPSNR Y | Weighted PSNR metric for Y component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |
| WPSNR U | Weighted PSNR metric for U component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |
| WPSNR V | Weighted PSNR metric for V component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |
| EncT | Encoder run time in seconds as measured on CPU in single-threaded mode (see Section 4.1) | 3 |
| DecT | Decoder run time in seconds as measured on CPU in single-threaded mode (see Section 4.1) | 3 |
| relative EncT | Encoder run time ratio to VTM anchor (e.g., 1.0000 if runtime is equal to anchor) | 4 |
| relative DecT | Decoder run time ratio to VTM anchor | 4 |
| peak Enc Memory | Peak memory use of encoder in KB (see getProcStatusValue("VmPeak:") in VTM software) | 0 |
| peak Dec Memory | Peak memory use of decoder in KB | 0 |
| MD5sum | MD5 checksum of the reconstructed YUV file (“.yuv” format) | na |

The first line in the CSV file should contain the name of the columns. For example:

Sequence;Rate point;Bitrate;PSNR Y;PSNR U;PSNR V;SSIM Y;SSIM U;SSIM V;WPSNR Y;WPSNR U;WPSNR V;EncT;DecT;relative EncT;relative DecT;peak Enc Memory;peak Dec Memory;MD5sum

Additionally, for each combination of sequence and rate point, a separate CSV file must be provided with a bit count and PSNR-Y, -U, and -V values for each frame in display order (one frame per text line, semicolon (“;”) separated values). For sequences in the HPQ category, the text file must further include wPSNR-Y, -U, and -V values for each frame. The file name of this CSV file shall match the bitstream file name (see Annex A), with the following information.

| Column name | Description | Decimal places |
| --- | --- | --- |
| Frame | Frame number, starting from zero | 0 |
| Bit count | Number of bits for encoding the frame | 0 |
| PSNR Y | PSNR metric for Y component (See Annex C and [5]) | 3 |
| PSNR U | PSNR metric for U component (See Annex C and [5]) | 3 |
| PSNR V | PSNR metric for V component (See Annex C and [5]) | 3 |
| WPSNR Y | Weighted PSNR metric for Y component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |
| WPSNR U | Weighted PSNR metric for U component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |
| WPSNR V | Weighted PSNR metric for V component (See Annex C). To be reported for sequences in the HPQ category only. | 3 |

The first line in the CSV file should contain the name of the columns. For example:

Frame;Bit count;PSNR Y;PSNR U;PSNR V;WPSNR Y;WPSNR U;WPSNR V

Registration Form

Registration of a response to the Joint Call for Proposals on video compression with capability beyond VVC

| Organization: |  |
| --- | --- |
| Contact person: |  |
| Contact email: |  |

## Test case on improved compression: ☐

## Test cases on improved compression with runtime-constrained encoding: ☐

Intended approximate runtime targets: ☐ 0.2x  ☐ 1x  ☐ 5x

Number of test cases to be included in subjective test ___

☐ Submission includes encoder and decoder executables runnable under Ubuntu 24.04 (x86-64) to decode sequences from bitstreams (if box not ticked, please contact the test coordinator about usage of a different platform).

☐ Submission includes additional functionality according to Section 6 which requires subjective evaluation (not included in the formal subjective tests).

Remarks:

If a proponent intends to submit only for a subset of categories, it is requested to inquire via the contact persons (see sec. 11).

People who formally registered will receive instructions regarding how to submit the coded materials. If material is received after the deadline, the proposal may be excluded from testing.

Contact persons will provide information about document submission process. Note that submitted documents will be made publicly available. Exceptions to public availability will be considered on a case-by-case basis upon request by the contributor.

Proponents are requested to attend this standardization meeting.

To allow a comparison against variable-resolution coding, rate-matched versions of VTM enabling its tool of GOP-adaptive reference picture resampling [2] will be included in the visual test.

If the same picture is encoded or decoded in multiple segments the time for processing such a picture may be included only once in the total run time. Application of such operation must be documented. For easier crosschecking purposes, it is recommended to report the summation of full segment run times.

The frame distance between any two consecutive random access points shall not exceed the intra refresh period.

Configurations for high performance and reduced encoding time are not described for HDR categories in [4], but can be configured by using equivalent settings as in [3].

The same material for both main and supplemental submission packages shall also be brought to the meeting in January 2027.

The test coordinator shall be informed well in advance if this is not possible.

A template providing more detail on the structure and content of the proposal description document is provided as output document JVET-AR2022. A draft of this document is available as JVET-AQ2022.

The CSV files with the results data of all proposals are collected by the test coordinator and will be used for generating the CfP summary report of subjective and objective test results (without identifying proposals). A script for the purpose of this analysis will be made available to the proponents by the test coordinator before the meeting. Proponents are encouraged to use this script if they intend to provide results in their proposal document.