ISO/IEC 23009-1:2025/AMD 1:202x(E)

ISO/IEC JTC1/SC 29

Secretariat: JISC

Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: Support for media authentication, CMCD extension, and other enhancements

WD

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

A model document of an International Standard (the Model International Standard) is available at:
https://www.iso.org/drafting-standards.html

© ISO 20XX

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

1	Clause 5.3.3, Adaptation Sets	3

2	Clause 4.7, Overview / Schemes	5

3	Clause 5, Media Presentation	5

3.1	Clause 5.3.3.1, Hierarchical data model / AdaptationSets / Overview	5

3.2	Clause 5.8.4.6, Frame Packing	5

3.3	Clause 5.8.4.7, Audio Channel Configuration	6

4	Clause 8, Adding preselection to DASH profile for CMAF content	6

4.1	clause 8.12.3.3	6

4.2	clause 8.12.4.4	6

4.3	new clause 8.12.8.1 or 8.12.4.6	6

5	Annex I, Flexible Insertion of URL Parameters	7

5.1	Clause I.1, General	7

5.2	Clause I.2.3.3, Computation of a final query string	8

6	Annex G, MPD Examples and MPD Usage	8

6.1	Overall annex G	8

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO [had/had not] received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, SC 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO/IEC 14496 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: Support for media authentication, CMCD extension, and other enhancements

# Clause 5.3.3, Adaptation Sets

Clause 5.3.3.2 Semantics

Add the following entry in Table 6 — Semantics of AdaptationSet element:

| Element or Attribute Name | Use | Description |
| --- | --- | --- |
| AdaptationSet |  | Adaptation Set description. |
| … |  |  |
| @targetScreenOrientation | O | specifies a comma-separated list of targeted screen orientations for displaying the video component of this Adaptation Set. The values are: - any - landscape - portrait - square Where “any” means that the video component is adapted to any screen orientation, “landscape”, “portrait” means that the video component is adapted to, respectively, portrait and landscape screen orientations as defined in 2.1 Screen orientation types in W3C Screen Orientation, and “square” means when the screen aspect ratio is equal to 1.0. If not present, the value is “any”. |

Clause 5.3.3.3 XML syntax

Replace Clause 5.3.3.3 XML syntax as follows:

<xs:complexType name="AdaptationSetType"> <xs:annotation> <xs:documentation xml:lang="en"> Adaptation Set </xs:documentation> </xs:annotation> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentComponent" type="ContentComponentType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="RequestParam" type="up:ExtendedUrlInfoType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="Representation" type="RepresentationType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute ref="xlink:type" fixed="simple"/> <xs:attribute ref="xlink:show" fixed="embed"/> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="group" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="RFC6838ContentTypeType"/> <xs:attribute name="par" type="RatioType"/> <xs:attribute name="minBandwidth" type="xs:unsignedInt"/> <xs:attribute name="maxBandwidth" type="xs:unsignedInt"/> <xs:attribute name="minWidth" type="xs:unsignedInt"/> <xs:attribute name="maxWidth" type="xs:unsignedInt"/> <xs:attribute name="minHeight" type="xs:unsignedInt"/> <xs:attribute name="maxHeight" type="xs:unsignedInt"/> <xs:attribute name="minFrameRate" type="FrameRateType"/> <xs:attribute name="maxFrameRate" type="FrameRateType"/> <xs:attribute name="targetScreenOrientation" type="ScreenOrientation"/> <xs:attribute name="segmentAlignment" type="xs:boolean" default="false"/> <xs:attribute name="subsegmentAlignment" type="xs:boolean" default="false"/> <xs:attribute name="subsegmentStartsWithSAP" type="SAPType" default="0"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean"/> <xs:attribute name="initializationSetRef" type="UIntVectorType"/> <xs:attribute name="initializationPrincipal" type="xs:anyURI"/> </xs:extension> </xs:complexContent> </xs:complexType> <xs:simpleType name="RatioType"> <xs:annotation> <xs:documentation xml:lang="en"> Ratio Type for sar and par </xs:documentation> </xs:annotation> <xs:restriction base="xs:string"> <xs:pattern value="[0-9]*:[0-9]*"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="FrameRateType"> <xs:annotation> <xs:documentation xml:lang="en"> Type for Frame Rate </xs:documentation> </xs:annotation> <xs:restriction base="xs:string"> <xs:pattern value="[0-9]+(/[1-9][0-9]*)?"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="ScreenOrientation"> <xs:annotation> <xs:documentation xml:lang="en"> Orientation of the targeted Sceen </xs:documentation> </xs:annotation> <xs:restriction base="xs:string"> <xs:enumeration value="any"/> <xs:enumeration value="landscape"/> <xs:enumeration value="portrait"/> <xs:enumeration value="square"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="RFC6838ContentTypeType"> <xs:annotation> <xs:documentation xml:lang="en"> Type for RFC6838 Content Type </xs:documentation> </xs:annotation> <xs:restriction base="xs:string"> <xs:enumeration value="text"/> <xs:enumeration value="image"/> <xs:enumeration value="audio"/> <xs:enumeration value="video"/> <xs:enumeration value="application"/> <xs:enumeration value="font"/> </xs:restriction> </xs:simpleType>

# Clause 4.7, Overview / Schemes

In clause 4.7, table 2, update the following line:

| urn:mpeg:dash:outputChannelPositionList:2012 | 5.8.5.4 | A list of output channel position to signal individual speaker positions as defined in ISO/IEC 23001-8. Legacy format for backward-compatibility, it is recommended to use the signalling as defined in ISO/IEC 23091-3 instead. |
| --- | --- | --- |

to:

| urn:mpeg:dash:outputChannelPositionList:2012 | 5.8.5.4 | A list of output channel position to signal individual speaker positions as defined in ISO/IEC 23091-3. Legacy @schemeIdUri format for backward-compatibility, it is recommended to use the signalling as defined in ISO/IEC 23091-3 instead. |
| --- | --- | --- |

# Clause 5, Media Presentation

## Clause 5.3.3.1, Hierarchical data model / AdaptationSets / Overview

Add a note in Clause 5.3.3.1 to alert readers to the Annex I exception:

Note: Annex I defines specific schemeIdUri values that use a different processing model where values from multiple hierarchy levels are concatenated rather than overridden. Refer to Annex I.1.X for details on this special processing model.

## Clause 5.8.4.6, Frame Packing

In clause 5.8.4.6 “Frame packing”, change both instances of:

ISO/IEC 23091-3

to

ISO/IEC 23091-2.

## Clause 5.8.4.7, Audio Channel Configuration

In clause 5.8.4.7 “Audio Channel Configuration”, change:

The descriptor may carry audio channel configuration using the URN label and values defined for ChannelConfiguration in ISO/IEC 23001-8.

NOTE 2	In addition, a scheme for audio channel configuration is also defined in subclause 5.8.5.4. This scheme is maintained for backward compatibility, but it is preferable to use the signalling as defined in ISO/IEC 23001-8.

to

The descriptor may carry audio channel configuration using the URN label and values defined for ChannelConfiguration in ISO/IEC 23091-3.

NOTE 2	In addition, a scheme for audio channel configuration is also defined in subclause 5.8.5.4. This scheme is maintained for backward compatibility, but it is preferable to use the signalling as defined in ISO/IEC 23091-3.

# Clause 8, Adding preselection to DASH profile for CMAF content

## clause 8.12.3.3

In clause 8.12.3.3 (“Mapping of CMAF Structures”), add a new bullet point:

- The mapping of PreselectionGroupBoxes from CMAF Principal Headers to DASH Preselection elements, including the conversion of contained metadata boxes to appropriate DASH descriptors and attributes.

## clause 8.12.4.4

In clause 8.12.4.4 (“Period Constraints”), add after existing constraints:

If PreselectionGroupBoxes are present in CMAF Principal Headers, the corresponding Preselection elements shall be present at the Period level and shall reference the Adaptation Sets that correspond to the CMAF Switching Sets referenced by the PreselectionGroupBoxes. Details are provided in 8.12.4.X.

## new clause 8.12.8.1 or 8.12.4.6

Add the following new clause where appropriate (e.g. as new clause 8.12.8.1 or 8.12.4.6):

If the CMAF Principal Header contains one or more PreselectionGroupBoxes as defined in ISO/IEC 14496-12, 8th edition, each instance thereof shall be mapped to a Preselection element in the MPD according to the following rules:

- Preselection@id attribute: The value of the group_id in the PreselectionGroupBox may be assigned to the Preselection@id attribute.

- Preselection@preselectionComponents attribute: The tracks referenced through the entity_id in the PreselectionGroupBox shall be assigned to the Preselection@preselectionComponents attribute, where each value of an entity_id is represented by the corresponding AdaptationSet@id value. The order of the entries in the @preselectionComponents attribute shall be the same as in the loop of entity_ids in the PreselectionGroupBox.

- Preselection@tag attribute: If the preselection_tag is present in the PreselectionGroupBox, its value shall be assigned to the Preselection@tag attribute. Otherwise, this attribute should be absent.

- Preselection@selectionPriority attribute: If the preselection_priority is present in the PreselectionGroupBox, its value should be assigned to the Preselection@selectionPriority attribute. Otherwise, this attribute may be absent.

NOTE X There are cases where the selection priority was set in the encoded bitstream on a best knowledge basis, but when multiple encodings are combined in single MPD, the direct usage of the selection priority defined in the media is not suitable in the MPD. In such cases the selection priority could be omitted or set differently.

- Preselection@order attribute: If the interleaving_tag is present in the PreselectionGroupBox, its value shall be used to determine the Preselection@order attribute. If the interleaving_tag is not present, the @order shall either be set to "undefined" or be absent.

- Additional boxes in PreselectionGroupBox: If additional boxes are present in a PreselectionGroupBox, the following mappings shall apply according to the recommendations in clause 8.12.4.3:

  - An ExtendedLanguageBox shall be mapped to the Preselection@lang attribute.

  - KindBoxes should be converted to Role and Accessibility descriptors or other attributes and elements.

  - Any ChannelLayoutBox or AudioRenderingIndicationBox shall be converted to an AudioChannelConfiguration descriptor.

- Preselection descriptor: All DASH AdaptationSets comprising any Preselection may have a SupplementalProperty descriptor assigned with @schemeIdUri set to "urn:mpeg:dash:preselection:2016" and the value being absent. If the AdaptationSet contributing to at least one Preselection is never referenced as the main AdaptationSet (i.e., is never the first entry in the list of AdaptationSet@id values in the Preselection@preselectionComponents list), an EssentialProperty descriptor shall be used instead and shall be present.

# Annex I, Flexible Insertion of URL Parameters

## Clause I.1, General

Add a new subsection immediately after the current Annex I introduction:

I.1.X Processing Model and Inheritance Behavior

The processing of descriptors defined in this Annex deviates from the general inheritance principle described in Clause 5.3.3.1.

While it is forbidden that SupplementalProperty or EssentialProperty descriptors can be present at both AdaptationSet and Representation level (and if so, that descriptor on the lower level would take precedence over the default value from the higher level), the Annex I processing model is as follows: For descriptors with schemeIdUri values defined in this Annex, values from multiple levels are concatenated rather than replaced. The concatenation order is specified for each descriptor type in the respective subsections.

This special processing model only applies to all descriptors referencing schemeIdUri values defined in this Annex, unless explicitly stated otherwise for a specific schemeIdUri.

Note: Content authors and client implementers should be aware that this processing model differs fundamentally from the general DASH inheritance mechanism and requires specific implementation logic.

## Clause I.2.3.3, Computation of a final query string

In clause I.2.3.3, replace the current text:

When two or more occurrences of URL query descriptors exist within an MPD, the final query string used at the Representation level is a concatenation of the corresponding URL query strings of the occurrences in their orders of appearance in the MPD hierarchy.

With:

Deviation from general inheritance: Unlike the general inheritance principle defined in Clause 5.3.3.1, where lower-level values override higher-level values, URL query descriptors use a concatenation model. When two or more occurrences of URL query descriptors exist within an MPD, the final query string used at the Representation level is a concatenation of the corresponding URL query strings of the occurrences in their orders of appearance in the MPD hierarchy. This means that a Representation-level descriptor does not replace or override an AdaptationSet-level descriptor; instead, both are combined.

Further, an example may be added after the existing concatenation order description:

Example: If an AdaptationSet contains a SupplementalProperty with @schemeIdUri="urn:mpeg:dash:urlparam:2016" and @value="?param1=A", and a contained Representation has a SupplementalProperty with the same schemeIdUri and @value="&param2=B", the final query string for that Representation will be "?param1=A&param2=B", not just "&param2=B" as would be the case under general inheritance rules.

# Annex G, MPD Examples and MPD Usage

## Overall annex G

In Annex G: MPD Examples and MPD Usage, replace all instances (8 in total) of

<AudioChannelConfiguration schemeIdUri="urn:mpeg:dash:23003:3:audio_channel_configuration:2011" value="X" />

to

<AudioChannelConfiguration schemeIdUri="urn:mpeg:mpegB:cicp:ChannelConfiguration" value="X" />

where X represents the existing value (used values in examples include 2 and 6).

The decision where to place the proposed text may depend on whether the new provisions amend the existing profiles in clauses 8.12.5 and 8.12.6, or whether the mapping is considered supplementary to, but not included in these profiles.