# ISO/IEC JTC 1/SC 29/WG 3      N01871

ISO/IEC JTC 1/SC 29/WG 3

MPEG Systems

Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Exploration of carriage of static Gaussian Splat in HEIF

Status:	Approved

Date of document:	2026-08-21

Source:	ISO/IEC JTC 1/SC 29/WG 3

Expected action:	None

Action due date:	None

No. of pages:	31- (including cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG3

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG3 N01871

July 2026, Geneva

| Title | Exploration of carriage of static Gaussian Splat in HEIF |
| --- | --- |
| Source | WG 03, MPEG Systems |
| Editor | Emre B. Aksu (Nokia), Imed Bouazizi (Qualcomm) |
| Status | Approved |
| Serial Number | 26540 |

# Summary

During MPEG#153 a first proposal was provided of carriage of static Gaussian Splat in HEIF.

During MPEG#154, several other contributions on this topic were discussed and several comments were collected. This document consolidates these contributions based on decisions and agreements made during the meeting, including open questions and discussion points.

During MPEG#155, m78065 provided a merged text for contributions m77779, m77791 as well as several updates based on technical comments from m77646.

# 1	Introduction

This exploration document provides information, technology alternatives and related discussion points about potential mechanisms for the carriage of Gaussian Splats in an ISOBMFF/HEIF compliant file.

Gaussian Splats represent a scene as a set of anisotropic 3D Gaussians with per primitive appearance terms and are increasingly used to enable photorealistic rendering from different viewpoints.

The objective is to enable interoperable storage and delivery of Gaussian Splats in the same packaging ecosystem used for still images and image sequences. A conforming file shall have a dual-use (i.e. as an image and as a Gaussian splat representation) functionality, for example, remain usable by legacy HEIF viewers by providing a conventional cover image, while Gaussian splat aware applications and decoders can access the additional representation and render a 3D experience.

The exploration targets a Gaussian Splat representation compression agnostic carriage mechanism, while taking into account different Gaussian splat representation formats.

# 2 	Existing non-ISO-defined Static Gaussian Splat representations

Table 1 provides the list of currently known static Gaussian splat representations and formats used by the industry. In addition to these formats, it should also be noted that there is ongoing standardization work within ISO and JVET for MPEG V-PCC, G-PCC and JVET vSEI based static Gaussian Splat coding and their storage formats.

Table 1 Overview of non-ISO-defined Gaussian splat representations and formats used by the industry

| Format / Specification | File Extension | Storage / Coding Mechanism | Main Properties Encoded | Typical Use | Link to specification / information |
| --- | --- | --- | --- | --- | --- |
| Gaussian PLY (3DGS) | .ply | Raw floating-point attributes in PLY | Position, scale, rotation, opacity, full SH | Training, editing, archival, mezzanine data format, research | https://paulbourke.net/dataformats/ply/ |
| SPZ (Niantic) | .spz | Fixed-point + entropy coding | Full GS attributes incl. SH | Distribution, web, mobile | https://github.com/nianticlabs/spz |
| SOG (PlayCanvas) | .sog | Image-backed container (WebP + metadata) | GS parameters + palette SH | Web delivery | https://developer.playcanvas.com/user-manual/gaussian-splatting/formats/sog/ |
| SPLAT | .splat | Flat binary, GPU-oriented layout | Quantized geometry, limited color/SH | Lightweight web viewers | https://github.com/antimatter15/splat/blob/main/README.md |
| KSPLAT | .ksplat | Blocked & spatially sorted binary chunks | Quantized GS attributes | Progressive streaming in web context | https://deepwiki.com/mkkellogg/GaussianSplats3D/3.3-splat-and-ksplat-loading |
| Qualcomm Lightweight Gaussian Coding (L-GSC) | .lgsc | Lightweight GS entropy coding | Position, scale, rotation, opacity, SH | On-device & mobile | https://github.com/qualcomm/lite-3Dgsplat-codec |
| glTF Gaussian Splatting (KHR_gaussian_splatting) | .gltf / .glb | glTF 2.0 GS extension (point primitives) | Position, scale, rotation, opacity, SH | Interchange, engines, geospatial content | https://github.com/KhronosGroup/glTF/blob/main/extensions/2.0/Khronos/KHR_gaussian_splatting/README.md |
| glTF + SPZ Compression | .glb | glTF extension referencing SPZ payloads | Compressed GS baseline | Streaming & delivery On-device & mobile | https://community.cesium.com/t/create-khr-spz-gaussian-splats-compression-tooling/41103 |

These representation formats could be grouped in 4 different categories:

- single-blob binary representations, in which the coded Gaussian Splat data is stored as one contiguous payload,

- structured binary representations, in which the coded data may be organized as blocks, chunks, or non-contiguous segments internal to the representation format,

- image-backed or image-assisted representations, in which some coded parameters are stored in image-like resources combined with metadata (such as JSON), and

- encapsulated interchange representations, in which Gaussian Splat data is carried within a higher-level scene or asset format (e.g. glTF).

Each of these categories could be considered as different mechanisms for ISOBMFF encapsulation of Gaussian splats to enable creation of a dual-use and backwards compatible ISOBMFF compliant file, such as a HEIF file that contains an image and a Gaussian splat representation.

# 3	Analysis of possible interaction between an external 3DGS format and an MPEG System format

| Figure 1 - Scenario 1 | Figure 3 - Scenario 3 | Figure 2 - Scenario 2 |
| --- | --- | --- |

Scenario 1: Same-level delivery

In this scenario, the XYZ format and the proposed HEIF format operate at the same level of 3DGS file delivery. Both serve as standalone file formats for storing and delivering Gaussian Splat content. The XYZ format files can be converted to and from HEIF format. Those conversions may not be fully reversible in terms of Gaussian data and metadata preservation.

Scenario 2: XYZ format embedded in HEIF

In this scenario, HEIF acts as a container for XYZ format content, preserving the XYZ format file structure within the HEIF container. The characteristics include HEIF acting as container for XYZ format content, XYZ format images stored as HEIF items, and other metadata stored as HEIF metadata. The use case involves packaging existing XYZ format content in HEIF for compatibility with HEIF viewers and leveraging HEIF's single-file delivery while maintaining XYZ format structure.

Scenario 3: HEIF as XYZ format elements

In this scenario, HEIF files constitute parts of XYZ format content, where individual XYZ format images are stored as HEIF files. This implies HEIF files constituting parts of XYZ format content, individual images (e.g., position image) stored as HEIF files, potential efficiency gains from HEIF compression, and XYZ format structure built from HEIF elements.

# 4	General storage principles for backward compatibility and dual-use

As a result of the expert discussions after MPEG#154, the following general principles could be listed:

4.1 . Usage of brands:

One trivial mechanism for such a dual-use of an ISOBMFF compliant file is via new brand definitions. Hence, for each storage mechanism, it may make sense to define a new brand to signal the presence of a particular storage mechanism. As an example, such a brand could be used in conjunction with a HEIF-relevant brand to enable a cover image (primary image item) and a Gaussian Splat representation stored and processed together.

4.2. Referencing Gaussian Splat representation from primary image item as an entry point:

Independent of the Gaussian Splat representation item’s type, an item reference (e.g. ‘gsil’ ) from the primary image item to the Gaussian Splat representation item would be sufficient to relate a cover item to the Gaussian Splat representation. In this document, the item reference type 'gsil' is specified for this purpose, with the from_item_ID identifying the cover image item.

4.3. Gaussian Splat representation item types:

In MPEG#154, it was discussed that usage of auxiliary image items as a Gaussian splat representation entry item type could bring backward compatibility issues and may also not be suitable due to the 3D nature of the Gaussian Splats. The following list provides potential representation item types that were advised and discussed:

Depending on the category of the stored Gaussian Splat representation, the “entry point” item of a Gaussian Splat Representation can be one of the following item types:

- A MIME Item of a particular content_type (e.g. ‘application/octet-stream’): This may be used for a single-blob based storage of a Gaussian Splat representation for enabling a dual-use functionality, as well as when image-like attributes are stored (e.g. ‘application/image’). Which one to use may be defined based on the particular brand.

- A dedicated item with a specific 4CC code

- Usage of already-standardized item types and carriage mechanisms, for example, for video and image items.

If multiple items are required to represent a Gaussian Splat representation, a well-defined item reference type can be used.(e.g ‘gsst’ as provided in this document) from the entry point Gaussian Splat representation item to the other items that are required for the decoding and rendering purposes. As a resolution of this discussion, this document adopts a single-entry point item type: for every storage mechanism, the entry point of a Gaussian Splat representation is a Gaussian Splat representation item of type 'gspl'. The representation format is identified by the gs_format field of the associated GaussianSplatGlobalConfigurationProperty. Item references of type 'gsil' identify the entry point from the cover image item, while item references of type 'gsst' provide composition references from the entry point item to constituent stream items. The options listed above are retained for reference.

4.4. Gaussian Splat representation format related signaling:

Since there are multiple representation formats, independent of the encoding formats of the data itself, early identification of the Gaussian Splat representation format would be needed to invoke the correct media handler early enough to handover the representation handling functionality. For this purpose, signaling a 4CC code inside a common global configuration item was discussed. The gs_format values are registered with MP4RA. The value 'gsis' is reserved to indicate the structured binary representation specified in Annex X. Format dispatch by a reader is based on gs_format.

# 5	Exploration on Single blob type of Gaussian Representation Storage

## 5.1 General

In this scenario, a Gaussian Splat representation is stored as a single blob of data. The main purpose is to enable dual-use and have a cover image associated with this representation.

- A new brand, ‘gsc2’ is defined to indicate such a single-blob-based storage.

- An item reference of type ‘gsil’ is used to associate the primary image item with the Gaussian Splat Representation Item

- The Gaussian Splat representation item is an item of type 'gspl'. The single blob binary data is stored as an item of type ‘mime’ (i.e MIME item) with a proper content_type, content_encoding values in ‘infe’ box. The contents of these values are dependent on the value of the gs_format field in the GaussianSplatGlobalConfigurationProperty and out of scope of the exploration.

- GaussianSplatGlobalConfigurationProperty as defined in section X.2.3.1  is associated with the Gaussian Splat representation item, with a gs_format field indicating the representation format

The following diagram represents the explored approach:

## 5.2 Example processing model

- Since the major brand is set to ‘heic’, an image viewer sees and processes this file as a HEIF compliant image file and displays the cover image item.

- An application that can further support the compatible brand ‘gsc2’, would further process the item reference and find the Gaussian splat representation item.

- By accessing and parsing the GaussianSplatGlobalConfigurationProperty, application would read the gs_format field, then invoke the proper media handler which further reads the MIME item and its payload for further decoding as indicated by the gs_format field of GaussianSplatGlobalConfigurationProperty.

# 6	Overview: GS stored as a structured binary representation

## 6.1 General

A Gaussian Splat representation is carried as a non-image item of type 'gspl' that is associated with a conventional cover image item through an item reference of type 'gsil'. The representation data is organized into one or more stream items or data tracks, enabling selective access to individual attributes of the representation.

## 6.2	Carriage model

The primary item is a coded image item that acts as a 2D cover image for legacy HEIF viewers.

A Gaussian Splat (GS) representation item is a non-image item of type 'gspl' that is the target of an item reference of type 'gsil' from the cover image item.

The GS representation item groups one or more GS stream items via item reference of type ‘gsst’ in the MetaBox..

When GS data units are carried as items, each GS stream item carries one or more attribute sets and signals the role(s) through one or more item properties, enabling selective loading and parallel decoding.

When GS data units are carried as tracks, one or more attributes may be packed into samples of a Gaussian Splat data track or into samples of a track using an image or video coding format. Each compressed GS stream item signals the compressor used through an item property using a 4CC code. The design is independent of any specific compression algorithm.

## 6.3Role Assignment

The following is an example of roles composing a GS representation (based on the INRIA format for GS [4]). The roles may be conveyed in separate stream items or may be grouped together. When multiple roles are present in a single item or track sample, one instance of the role signalling is associated for each role.

- One stream item with role 'posi' contains position data for all points.

- One stream item with role 'rotq' contains rotation quaternions for all points.

- One stream item with role 'scl3' contains per point scale values.

- One stream item with role 'opac' contains per point opacity values.

- One stream item with role 'shco' with sh_band_index equal to 0 contains spherical harmonics band 0 coefficients, also called the DC term.

- One stream item with role 'shco' with sh_band_index equal to 1 contains spherical harmonics band 1 coefficients.

- Additional stream items with role 'shco' with sh_band_index equal to 2, 3, and so on, contain higher order spherical harmonics bands when present.

## 6.4 Example File Structures

The following examples show different examples of HEIF files carrying GS representations:

The first example shows a single GS image with one 2D cover image.

FileTypeBox 'ftyp': major-brand='heic', compatible-brands='gsc1, mif1, ...' MetaBox 'meta': HandlerBox 'hdlr': 'pict' PrimaryItemBox 'pitm': item_ID=1 ItemInfoBox 'iinf': entry_count=8 1) infe: item_ID=1, item_type='hvc1' // cover image 2) infe: item_ID=10, item_type='gspl' // GS representation item (non-image) 3) infe: item_ID=11, item_type='gspc' // GS stream item: position 4) infe: item_ID=12, item_type='gspc' // GS stream item: rotation 5) infe: item_ID=13, item_type='gspc' // GS stream item: opacity 6) infe: item_ID=14, item_type='gspc' // GS stream item: SH band 0 (DC) 7) infe: item_ID=15, item_type='gspc' // GS stream item: SH band 1 8) infe: item_ID=16, item_type='gspc' // GS stream item: SH band 2 ItemLocationBox 'iloc': item_count=8 item_ID=1, extent_offset=P0, extent_length=Q0 item_ID=10, extent_offset=P10, extent_length=Q10 item_ID=11, extent_offset=P11, extent_length=Q11 ... item_ID=16, extent_offset=P16, extent_length=Q16 ItemReferenceBox 'iref': referenceType='gsil', from_item_ID=10, to_item_ID=1 // cover links to GS referenceType='gsst', from_item_ID=10, to_item_ID=11..16 // GS master points at streams referenceType='cdsc', from_item_ID=20, to_item_ID=1 // optional camera metadata item ItemPropertiesBox 'iprp': ItemPropertyContainerBox 'ipco': 'hvcC' 'ispe' // cover config and size 'auxC' // on item 10, identifies gaussian_splats via URI 'gsgC' // GS global config (numPoints, shDegree, coord system) 'gscC' // coding method config; compressor_code='gscu' indicates uncompressed 'gsrC' (x6) // per-stream role property, one per stream item 'rref' // optional, to require processing 'gsst' references ItemPropertyAssociation 'ipma': item_ID=1: essential=1 hvcC, essential=0 ispe item_ID=10: essential=1 auxC, essential=1 gsgC, essential=1 rref item_ID=11: essential=1 gscC, essential=1 gsrC(role='posi') item_ID=12: essential=1 gscC, essential=1 gsrC(role='xfrm') item_ID=13: essential=1 gscC, essential=1 gsrC(role='opac') item_ID=14: essential=1 gscC, essential=1 gsrC(role='shb0') item_ID=15: essential=1 gscC, essential=1 gsrC(role='shb1') item_ID=16: essential=1 gscC, essential=1 gsrC(role='shb2') MediaDataBox 'mdat' (or 'idat'): payloads at P0, P10..P16

The following example depicts a single Gaussian Splat frame that is packed and encoded using a video codec in a track that is referenced from a Gaussian Splat item.

FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1'), compatible-brands='gsc1, mif1, mp42, ...' MetaBox 'meta': HandlerBox 'hdlr': 'pict' PrimaryItemBox 'pitm': item_ID=1 ItemInfoBox 'iinf': entry_count=2 1) infe: item_ID=1, item_type='hvc1' // cover image 2) infe: item_ID=10, item_type='gspl' // GS representation item (non-image) ItemLocationBox 'iloc': item_ID=1, extent_offset=P0, extent_length=Q0 item_ID=10, extent_offset=P10, extent_length=Q10 // optional bootstrap data, may be zero length ItemReferenceBox 'iref': referenceType='gsil', from_item_ID=10, to_item_ID=1 ItemPropertiesBox 'iprp': ItemPropertyContainerBox 'ipco': 'hvcC' 'ispe' // cover config and size 'gsgC' // GS global config (numPoints, shDegree, coord system) 'gstR' // points to track_ID=100 carrying packed GS data ItemPropertyAssociation 'ipma': item_ID=1: essential=1 hvcC, essential=0 ispe item_ID=10: essential=1 auxC, essential=1 gsgC, essential=1 gstR MovieBox 'moov': trak (track_ID=100) // Packed GS attribute track, encoded as video frames mdia hdlr: 'auxv' minf.stbl stsd: sample_entry: 'hvc1' (or any video coding sample entry) auxi: aux_track_type = "urn:mpeg:gaussian_splats:2026:auxid:1" gsrm: access_unit_sample_count=7 entry[0]: role_code='posi', sample_offset=0, sample_count=1, role_offset=0, role_length=Lpos entry[1]: role_code='rotq', sample_offset=1, sample_count=1, role_offset=0, role_length=Lrot entry[2]: role_code='scl3', sample_offset=2, sample_count=1, role_offset=0, role_length=Lscl entry[3]: role_code='opac', sample_offset=3, sample_count=1, role_offset=0, role_length=Lopac entry[4]: role_code='shco', sh_band_index=0, sample_offset=4, sample_count=1, role_offset=0, role_length=Lsh0 entry[5]: role_code='shco', sh_band_index=1, sample_offset=5, sample_count=1, role_offset=0, role_length=Lsh1 entry[6]: role_code='shco', sh_band_index=2, sample_offset=6, sample_count=1, role_offset=0, role_length=Lsh2 samples: F0, F1, F2, F3, F4, F5, F6 // one access unit representing a single GS frame MediaDataBox 'mdat': cover image item data and video-coded samples for track_ID=100

The following example shows a HEIF file with multiple views of a single Gaussian Splat frame from different camera views and stored as an image sequence.

FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1') compatible-brands='gsc1, mif1, mp42, ...' (and any codec brands used by 'pict' tracks) MovieBox 'moov': TrackBox 'trak' (track_ID=1) // Viewpoint 0 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V0_0, V0_1, V0_2, ... TrackBox 'trak' (track_ID=2) // Viewpoint 1 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V1_0, V1_1, V1_2, ... TrackBox 'trak' (track_ID=3) // Viewpoint 2 2D image sequence MediaBox 'mdia': HandlerBox 'hdlr': 'pict' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'hvc1' (or 'av01', 'vvc1', etc.) samples: V2_0, V2_1, V2_2, ... TrackBox 'trak' (track_ID=30) // timed camera intrinsics/extrinsics for viewpoint 0 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=1 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C0_0, C0_1, C0_2, ... TrackBox 'trak' (track_ID=31) // timed camera intrinsics/extrinsics for viewpoint 1 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=2 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C1_0, C1_1, C1_2, ... TrackBox 'trak' (track_ID=32) // timed camera intrinsics/extrinsics for viewpoint 2 TrackReferenceBox 'tref': track_reference_type 'cdsc' -> track_ID=3 MediaBox 'mdia': HandlerBox 'hdlr': 'meta' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: (camera metadata sample entry) samples: C2_0, C2_1, C2_2, ... // Split-stream GS non-image tracks shared by all viewpoints TrackBox 'trak' (track_ID=10) // GS positions TrackReferenceBox 'tref': track_reference_type 'gsil' -> track_ID=1, track_ID=2, track_ID=3 MediaBox 'mdia': HandlerBox 'hdlr': 'auxv' MediaInformationBox 'minf': SampleTableBox 'stbl': SampleDescriptionBox 'stsd': sample_entry: 'gspc' AuxiliaryTypeInfoBox 'auxi': aux_track_type="urn:mpeg:gaussian_splats:2026:auxid:1" GaussianSplatCompressionConfigurationProperty 'gscC': compressor_code='gscu' (optional) GaussianSplatTrackRoleMapBox 'gsrm' (not needed when the track conveys only one role) (optional) role signalling in sample entry, e.g. role_code='posi'

The following example illustrates storage of SPZ compressed Gaussian splat data as a single blob data structure in a HEIF file with brand ‘gsc2’

FileTypeBox 'ftyp': major-brand='heic', compatible-brands='gsc2, mif1,...' MetaBox 'meta': HandlerBox 'hdlr': 'pict' PrimaryItemBox 'pitm': item_ID=1 ItemInfoBox 'iinf': entry_count=2 1) infe: item_ID=1, item_type='hvc1' // cover image 2) infe: item_ID=10, item_type='gspl' // GS representation item (non-image) 3) infe: item_ID=11, item_type='mime' // GS stream item or type ‘mime’ with content type ”application/octet-stream” ItemLocationBox 'iloc': item_ID=1, extent_offset=P0, extent_length=Q0 item_ID=10, extent_offset=P10, extent_length=Q10 // optional bootstrap data, may be zero length item_ID=11, extent_offset=P11, extent_length=Q11 ItemReferenceBox 'iref': referenceType='gsil', from_item_ID=10, to_item_ID=1 referenceType=’gsst’, from_item_ID=10, to_item_ID=11 ItemPropertiesBox 'iprp': ItemPropertyContainerBox 'ipco': 'hvcC' 'ispe' // cover config and size 'gsgC' // GaussianSplatGlobalConfigurationProperty (e.g. with gs_format =’spzb’, registered via MP4RA) ItemPropertyAssociation 'ipma': item_ID=1: essential=1 hvcC, essential=0 ispe item_ID=10: essential=1 auxC, essential=1 gsgC, essential=1 gstR MediaDataBox 'mdat': cover image item data and SPZ-compressed Gaussian Splat binary data

# 7 	Image-backed storage of a Gaussian Splat representation

In this storage mechanism, some or all of the parameters of a Gaussian Splat representation are carried in image-like resources, combined with additional metadata, as is the case for SOG-like representation formats. The unified design of this document applies as the following: the entry point is a Gaussian Splat representation item of type 'gspl' whose gs_format identifies the image-backed representation format; the constituent image resources are stored as image items, ‘mime’ items or other items as required by the gs_format value and referenced from the Gaussian Splat representation item using item references of type 'gsst',

A new brand (‘gsc2’) covers this storage mechanism, consistent with the brand-per-mechanism principle of clause 4.1.

# 8 	Parser behavior (informative)

This clause describes the expected processing model of readers with different capabilities. The description is informative.

## 8.1 	Brand-based capability detection

The reader parses the FileTypeBox. When neither 'gsc1' nor 'gsc2' is among the compatible brands, the file is processed as a conventional HEIF file. A legacy reader displays the primary item, which is a conventional coded image acting as the 2D cover. For such a reader, the Gaussian Splat representation item is a non-image item of an unknown type and is ignored together with the item reference of type 'gsil'. The presence of the Gaussian Splat representation does not affect legacy processing.

## 8.2 	Discovery of the Gaussian Splat representation

A Gaussian Splat aware reader parses the ItemReferenceBox and locates the item reference of type 'gsil' whose from_item_ID identifies the cover image item and whose to_item_ID identifies the Gaussian Splat representation item of type 'gspl'. The reader then collects the item properties associated with the representation item. The GaussianSplatGlobalConfigurationProperty is essential for that. A reader that does not understand this property does not process the item.

## 8.3 	Format dispatch

The reader reads the gs_format field of the GaussianSplatGlobalConfigurationProperty. When gs_format indicates an externally defined representation format, the reader proceeds as described in 8.4. When gs_format is equal to 'gsis', the reader proceeds as described in 8.5. When the gs_format value is not recognized, the reader falls back to presenting the cover image. The reader is not expected to process or reconstruct the Gaussian Splat representation. A media handler that can handle the signaled gs_format is expected to process the representation further.

## 8.4 	Single-blob processing

In this case, The single binary blob media data is stored as a single item’s payload in the file. The reader, after reading the gs_format field, dispatches the proper media handler. The media handler processes the ‘gsst’ referenced item via the reader, accesses the media data of the item (e.g. a ‘mime’ item) and does the necessary format-specific decoding and representation generation operations. All information internal to the payload, including point count and quantization parameters, is interpreted by that media handler; the sh_degree, component_format and gs_flags fields are informative in this case.

## 8.5 Structured representation processing

The reader reads num_points, sh_degree, component_format and gs_flags, which are authoritative for the structured binary representation. The reader collects the Gaussian Splat stream items through item references of type 'gsst', and Gaussian Splat data tracks through the GaussianSplatTrackReferenceProperty when present. For each stream item, the reader reads the GaussianSplatCompressionConfigurationProperty and, when compressor_code is not equal to 'gscu', initializes the corresponding decompressor with compressor_parameter and decompresses the payload to the uncompressed stream data layout of X.2.5. The reader then uses the GaussianSplatRoleProperty associations to locate the byte range of each role, to reassemble fragmented roles, and to select the subset of roles required, for example limiting the loaded spherical harmonics bands.

## 8.6 Camera parameters and rendering

The reader checks the cover image item for associated camera parameter properties as specified in X.2.7 and, for timed content, for camera metadata tracks linked through track references of type 'cdsc'. The reconstructed attribute arrays are handed to the renderer, optionally initialized with the viewpoint of the cover image.

# 9	Proposed Specification text

# Definitions

- Gaussian Splat: a parametric primitive representing scene radiance as an oriented anisotropic Gaussian with associated appearance parameters.

- Gaussian Splat representation: the complete set of data needed to decode and render a Gaussian Splat, including geometry, appearance, and any required metadata.

- Gaussian Splat representation format: the format of a Gaussian Splat representation, identified by the gs_format field of the GaussianSplatGlobalConfigurationProperty. The value 'gsis' identifies the structured binary representation specified in Annex X; other values identify externally defined representation formats registered with MP4RA.Gaussian Splat representation item: an item of type 'gspl' that carries or identifies a Gaussian Splat representation, that is the target of an item reference of type 'gsil' from a conventional image item serving as a 2D cover, and that either carries the representation payload directly or provides references to one or more Gaussian Splat stream items or Gaussian Splat data tracks.Gaussian Splat stream item: an item that carries a coded byte sequence corresponding to one or more Gaussian Splat streams, where the coding method is identified by an associated GaussianSplatCompressionConfigurationProperty ('gscC'). A Gaussian Splat stream item may be an item of type ‘gspc’, ‘mime’ or any other coded image item type as defined in HEIF specification (ISO/IEC 23008-12), depending on the nature of the Gaussian Splat representation format being stored and the gs_format field of the GaussianSplatGlobalConfigurationProperty.

- compressor_code: a four character code that identifies the coding method used for a Gaussian Splat stream item or Gaussian Splat data track. The value 'gscu' indicates uncompressed stream data.

- Gaussian Splat data track: a track that carries coded Gaussian Splat data units, optionally using an image or video coding format, and signals the coding method using compressor_code.

- Spherical harmonics degree: the maximum spherical harmonics order represented by the spherical harmonics coefficients of a Gaussian Splat representation.

- Spherical harmonics band: a subset of spherical harmonics coefficients that share the same order and are signaled as a distinct component of the representation.

# Annex X
(normative)
Storage of Gaussian Splats in HEIF

## X.1 General

This annex specifies syntax elements and brands to encapsulate Gaussian Splat representations in files conforming to the HEIF specification (ISO/IEC 23008-12) . The Gaussian Splat representation enables view synthesis and photorealistic rendering from novel viewpoints by storing a set of 3D Gaussian primitives and associated appearance parameters.

Two conformance points are specified in this annex: the brand 'gsc1' for the structured binary representation and the brand 'gsc2' for the carriage of an externally defined Gaussian Splat representation as a single binary blob or as an item-based representation structure that maps to the existing storage format of the Gaussian splat representation. The Gaussian Splat representation format is signalled by the gs_format field of the GaussianSplatGlobalConfigurationProperty ('gsgC'). For the structured binary representation, the coding method used for each Gaussian Splat stream item is signalled by a 4CC compressor_code in a GaussianSplatCompressionConfigurationProperty ('gscC'), including a value that indicates uncompressed stream data.

## X.2 Storage of Gaussian Splat representations

### X.2.1 General

Clause X.2 specifies requirements for files containing Gaussian Splat representation items. When a brand specified in X.4.1 is among the compatible brands of a file, the requirements specified in Clause X.2 shall be applied.

### X.2.2 Gaussian Splat representation items

#### X.2.2.1 Gaussian Splat Representation Item

A Gaussian Splat Representation item shall be an item of type ‘gspl’.

Each Gaussian Splat representation item shall be referenced by an  item reference of type ‘gsil’ from an image item that represents the Gaussian splat as a 2D image, such as a 2D cover. .

Gaussian Splat Representation items shall act as the entry point for the Gaussian Splat representation which is indicated by the gs_format field of the  GaussianSplatGlobalConfigurationProperty.  The actual Gaussian Splat data is carried by the referenced Gaussian Splat stream items. A minimal (e.g. 1 byte) reserved payload may be present for the Gaussian Splat Representation item in order to satisfy ItemLocationBox requirements.

Each Gaussian Splat representation item of type 'gspl' whose gs_format is equal to 'gsis' shall group one or more Gaussian Splat data units. The data units shall be carried either as one or more Gaussian Splat stream items referenced using an item reference of type 'gsst', or in one or more tracks identified by a GaussianSplatTrackReferenceProperty.

In addition to the item reference of ‘gsil’ a cover image and a Gaussian Splat Representation item may be declared as alternative representations via an ‘altr’ entity grouping. In such a case, the first item in the entity group shall be the primary image item.

#### X.2.2.2 Gaussian Splat stream items

A Gaussian Splat stream item contains the data for one or more Gaussian Splat streams. Each role present in the stream item shall be indicated by an associated GaussianSplatRoleProperty ('gsrC') as specified in X.2.3.1. When more than one role is present, multiple 'gsrC' item properties shall be associated with the stream item, one for each role. GaussianSplatRoleProperty shall be present for Gaussian Splat stream items of item_type ‘gspc’ and optional otherwise depending on the value of the gs_format field of the associated GaussianSplatGlobalConfigurationProperty.

Gaussian Splat stream items shall be referenced by exactly one Gaussian Splat representation item using an item reference of type 'gsst'.

##### X.2.2.2.1 Gaussian Splat Stream item

A Gaussian Splat Stream item shall be one of the following item types:

- An item of type ‘gspc’

- An item of type ‘mime’ (i.e. MIME item)

- A coded image item of type ‘hvc1’ of ‘vvc1’ as defined in HEIF specification (ISO/IEC 23008-12)

- An item of an item type other than the above listed item types whose interpretation is defined by the gs_format syntax element value in GaussianSplatGlobalConfigurationProperty.

Such Gaussian Splat stream items shall be processed based on the gs_format syntax element of GaussianSplatGlobalConfigurationProperty.

Gaussian Splat Stream item of type ‘mime’ may have a mime type of  ‘application/octet-stream’, ‘image/png’, ‘application/json’ or alike, that may store one or more Gaussian Splat attributes data or Gaussian Splat representation related configuration information.

Gaussian Splat Stream items which are coded image items of type ‘hvc1’ of ‘vvc1’ as defined in HEIF specification (ISO/IEC 23008-12) shall have associated decoder related configuration item properties as defined in HEIF specification (ISO/IEC 23008-12) and shall not have an associated GaussianSplatCompressionConfigurationProperty.

Gaussian Splat Stream Item of type 'gspc' consists of a coded representation of one or more Gaussian Splat streams. The coding method used for the stream item shall be signalled by an associated GaussianSplatCompressionConfigurationProperty ('gscC') as specified in X.2.3.3.

The decoded stream data for each role present in the stream item shall conform to the data layouts specified in X.2.5. Each role present in the stream item shall be indicated by one or more associated GaussianSplatRoleProperty ('gsrC') item properties as specified in X.2.3.2.

### X.2.3 Gaussian Splat item properties

#### X.2.3.1 Gaussian Splat global configuration property

##### X.2.3.1.1	Definition

Box type: gsgC
Property type: Descriptive item property
Container: ItemPropertyContainerBox
Mandatory (per item): Yes, for a Gaussian Splat representation item of type gspl
Quantity (per item): One for a Gaussian Splat representation item of type gspl

The GaussianSplatGlobalConfigurationProperty specifies configuration parameters that apply to the associated Gaussian Splat representation as a whole, including the number of points, spherical harmonics degree, component format for uncompressed streams, and coordinate system flags.

essential shall be equal to 1 for a gsgC item property associated with a Gaussian Splat representation item of type gspl.

##### X.2.3.1.2	Syntax

aligned(8) class GaussianSplatGlobalConfigurationProperty

extends ItemFullProperty('gsgC', version == 0, flags == 0) {

unsigned int(32) gs_format;

unsigned int(8)  gs_version;

unsigned int(8)  gs_flags;

unsigned int(32) num_points;

unsigned int(8)  sh_degree;

unsigned int(8)  component_format;

}

##### X.2.3.1.3	Semantics

gs_format specifies the 4CC code of the Gaussian splat Representation Format. This 4CC may be used to invoke the proper media handler to parse, decode and render the Gaussian splat representation. gs_format values other than 'gsis' are externally defined 4CCs registered with MP4RA. The value 'gsis' indicates the structured binary representation specified in this annex.

gs_version specifies the version of the Gaussian Splat global configuration. For this version of this annex, gs_version shall be equal to 0.

gs_flags provides flags that define the coordinate system and interpretation of values in the data streams as defined in the following table.

Table X.1 gs_flags values for GaussianSplatGlobalConfigurationProperty

| Bits | Description |
| --- | --- |
| 0 | when equal to 1, the coordinate system is right handed, when equal to 0, it is left handed. |
| 1 | when equal to 1, positions and scales are expressed in meters, when equal to 0, units are unspecified. |
| 2 | when equal to 1, quaternion components are ordered (qx, qy, qz, qw), when equal to 0, ordering is (qw, qx, qy, qz). |
| 3 | when bit 3 is equal to 1, the components of all roles for a given point are stored contiguously in an interleaved per-point layout. |
| 4..15 | reserved, set to 0. |

num_points specifies the number of points in the Gaussian Splat representation.

sh_degree specifies the maximum spherical harmonics degree present in the Gaussian Splat representation.

When gs_format is not equal to 'gsis', the fields, sh_degree, component_format and gs_flags values are informative and may be overridden by the corresponding information carried within the data of the Gaussian Splat representation format.

component_format  specifies the default component representation used for uncompressed role byte sequences, i.e. when compressor_code is equal to 'gscu'. component_format equal to 0 indicates IEEE 754 binary32. component_format equal to 1 indicates IEEE 754 binary16. component_format equal to 2 indicates an 8-bit unsigned normalized integer with linear mapping to the range [0, 1]. Other values are reserved for future use. When gs_format is not equal to 'gsis', component_format shall be set to 0.

#### X.2.3.2 Gaussian Splat role property

##### X.2.3.2.1 Definition

Box type: 				'gsrC'

Property type: 			Descriptive item property

Container: 			ItemPropertyContainerBox

Mandatory (per item):		Yes, for a Gaussian Splat stream item of type ‘gspc’

Quantity (per item): 		One or more for a Gaussian Splat stream item of type ‘gspc’, zero or more for other Gaussian Splat stream item types

The GaussianSplatRoleProperty identifies a role present in a Gaussian Splat stream item and, when applicable, parameters such as spherical harmonics band index, stream fragmentation information, and the byte range within the decoded byte sequence corresponding to the role.

essential shall be equal to 1 for a 'gsrC' item property associated with a Gaussian Splat stream item of type ‘gspc’.

##### X.2.3.2.2 Syntax

aligned(8) class GaussianSplatRoleProperty

extends ItemFullProperty('gsrC', version == 0, flags == 0){

unsigned int(32) role_code;

unsigned int(16) role_instance;

unsigned int(16) role_count;

unsigned int(16) sh_band_index;

if ( compressor_format == ‘gscu’) {

unsigned int(32) role_offset;

unsigned int(32) role_length;

}

}

##### X.2.3.2.3 Semantics

role_code specifies the role identified by this property. The role_code value shall be one of the values specified in Table X.1, unless otherwise specified by an extension of this annex.

role_instance and role_count specify fragmentation of a role across multiple stream items. When role_count is greater than 1, the complete role stream is formed by concatenating the role byte sequences of the fragments in increasing role_instance order. When role_count is equal to 1, role_instance shall be equal to 0.

sh_band_index specifies the spherical harmonics band index when role_code indicates spherical harmonics coefficients. For roles other than spherical harmonics coefficients, sh_band_index shall be set to 0xFFFF.

role_offset specifies the zero-based byte offset from the start of the decoded byte sequence of the associated stream item to the first byte of the role byte sequence identified by this property.

role_length specifies the length in bytes of the role byte sequence identified by this property.

For a given stream item, the byte ranges indicated by 'gsrC' item properties associated with the stream item shall not overlap, unless otherwise specified by an extension of this annex.

Table X.1 – role_code values for GaussianSplatRoleProperty

| role_code | Semantics |
| --- | --- |
| posi | Position of each Gaussian, expressed as three components per point. |
| rotq | Orientation of each Gaussian, expressed as four components per point (quaternion). |
| scl3 | Scale of each Gaussian, expressed as three components per point. |
| opac | Opacity of each Gaussian, expressed as one component per point. |
| shco | Spherical harmonics coefficients. The sh_band_index field indicates which band is present in the stream item. |

#### X.2.3.3 Gaussian Splat compression configuration property

##### X.2.3.3.1 Definition

Box type:	 		'gscC'

Property type: 		Descriptive item property

Container: 			ItemPropertyContainerBox

Mandatory (per item): 	Yes, for a Gaussian Splat stream item of type 'gspc'

Quantity (per item): 	One for a Gaussian Splat stream item of type 'gspc'

The GaussianSplatCompressionConfigurationProperty identifies the coding method used for an item of type 'gspc' and provides parameters required to initialize a decompressor.

essential shall be equal to 1 for a 'gscC' item property associated with a Gaussian Splat stream item of type 'gspc'.

##### X.2.3.3.3 Syntax

aligned(8) class GaussianSplatCompressionConfigurationProperty

extends ItemFullProperty('gscC', version == 0, flags == 0) {

unsigned int(32) compressor_code;

unsigned int(32) compressor_parameter_length;

unsigned int(8) compressor_parameter[compressor_parameter_length];

}

##### X.2.3.3.4 Semantics

compressor_code is a four-character code identifying the coding method used for the item of type 'gspc'. The interpretation of compressor_parameter is defined by the identified coding method. When compressor_code is equal to 'gscu', the stream item is uncompressed, compressor_parameter_length shall be equal to 0, and the decoded byte sequence shall be identical to the stored byte sequence.

#### X.2.3.4 Gaussian Splat track reference property

##### X.2.3.4.1 Definition

Box type:		'gstR'

Property type:		Descriptive item property

Container:		ItemPropertyContainerBox

Mandatory (per item):   Conditional, for a Gaussian Splat representation item of type 'gspl' that identifies one or more tracks carrying Gaussian Splat data

Quantity (per item):	One for a Gaussian Splat representation item of type 'gspl' that identifies tracks

The GaussianSplatTrackReferenceProperty identifies one or more tracks in the file that carry Gaussian Splat data units for the associated Gaussian Splat representation item.

essential shall be equal to 1 for a 'gstR' item property associated with a Gaussian Splat representation item of type 'gspl'.

##### X.2.3.4.2 Syntax

aligned(8) class GaussianSplatTrackReferenceProperty

extends ItemFullProperty('gstR', version == 0, flags == 0) {

unsigned int(16) track_count;

reserved bit(16) reserved;

unsigned int(32) track_ID[track_count];

}

##### X.2.3.4.3 Semantics

track_count specifies the number of track_ID values that follow.

track_ID identifies a track by the track_ID in the TrackHeaderBox of that track. Each referenced track shall be present in the same file and shall carry Gaussian Splat data units as specified in X.3.

When a 'gstR' item property is associated with an item of type 'gspl', the item may omit any item references of type 'gsst'. When both 'gstR' and 'gsst' are present, the identified data units shall be treated as components of the same Gaussian Splat representation.

### X.2.4 Item reference type ’gsil’

#### X.2.4.1 General

The item reference type ‘gsil’ identifies a Gaussian Splat representation item associated with a conventional image item used as a 2D cover. In an item reference of type ’’gsil’, the from_item_ID shall identify the conventional image item that serves as the 2D cover, and the to_item_ID shall identify the Gaussian Splat representation item. A reader that does not recognize the ‘gsil’ reference shall ignore it. A reader that does not recognize the 'gspl' item type shall ignore the Gaussian Splat representation item.

### X.2.5 Uncompressed stream data layout

The uncompressed stream data layout is role specific and is defined by the role_code in the associated GaussianSplatRoleProperty. When a stream item contains more than one role, role_offset and role_length indicate the location of each role byte sequence within the decoded byte sequence of the stream item.

Unless otherwise stated in this annex, each component in the uncompressed stream data shall be represented in the component representation indicated by component_format, in little endian byte order.

For role_code values specified in Table X.1, the following layouts apply for each point, in the order of points from 0 to num_points − 1:

- 'posi': three components (x, y, z).

- 'rotq': four components (qx, qy, qz, qw).

- 'scl3': three components (sx, sy, sz).

- 'opac': one component (opacity).

- 'shco':. an array of coefficient triplets for the signalled sh_band_index. For band index n, the number of coefficient triplets per point is equal to (2*n + 1), such that the total number of triplets per point across all bands from 0 to sh_degree is equal to (sh_degree + 1)*(sh_degree + 1).

When bit 3 of gs_flags is equal to 1, the components of all roles for a given point are stored contiguously in an interleaved per-point layout. The detailed constraints for the interleaved layout, including the role order within a point, are for further study. NOTE: The interleaved layout targets GPU-oriented single-blob representations in which all attributes of a point are fetched together.

### X.2.6 Item reference type 'gsst'

The item reference type 'gsst' identifies Gaussian Splat stream items that belong to a Gaussian Splat representation item of type 'gspl'.

In an item reference of type 'gsst', the from_item_ID shall identify the Gaussian Splat representation item and the to_item_ID values shall identify Gaussian Splat stream items.

### X.2.7 Camera parameters

A conventional image item that serves as the 2D cover of a Gaussian Splat representation may be associated with a CameraIntrinsicMatrixProperty ('cmin') and a CameraExtrinsicMatrixProperty ('cmex') as specified in ISO/IEC 23008-12. When present, these properties describe the camera model and the camera pose corresponding to the cover image, expressed in the coordinate system of the Gaussian Splat representation as indicated by gs_flags.

Additional camera metadata may be carried in a metadata item that is linked to the cover image item using an item reference of type 'cdsc'.

When multiple cover image items corresponding to different viewpoints are associated with the same Gaussian Splat representation item, each cover image item may be associated with its own camera parameter properties.

NOTE: Relating the cover image viewpoint to the coordinate system of the Gaussian Splat representation enables a reader to initialize rendering from a viewpoint that is consistent with the 2D cover image.

## X.3 Storage of Gaussian Splats as timed sequences

### X.3.1 General

Gaussian Splat representations may be carried as timed sequences. The specifications of Clause 7 apply.

A timed Gaussian Splat sequence may be carried in one or more tracks. When a timed Gaussian Splat sequence is present, it is recommended to provide a viewable image sequence track and to associate the Gaussian Splat sequence to the viewable track. using a track reference of type 'gsil'.

NOTE: This annex is compression agnostic. Image or video coding formats may be used to carry Gaussian Splat data units in tracks, provided that the roles and the mapping to the uncompressed stream data layouts specified in X.2.5 are signaled.

### X.3.2 Gaussian Splat data tracks

#### X.3.2.1 General

Gaussian Splat data units may be carried in a Gaussian Splat data track. A Gaussian Splat data track shall use a sample entry of type 'gspc'. The coding method used for the samples shall be signalled by the compressor_code in the sample entry.

A Gaussian Splat data track may be associated with one or more image sequence tracks using a track reference of type 'gsil'. When used as a track reference type, 'gsil' has semantics analogous to the item reference type 'gsil' and associates the Gaussian Splat data with the visual track(s) that provide the 2D cover. A Gaussian Splat data track may also be identified by a Gaussian Splat representation item of type 'gspl' using a GaussianSplatTrackReferenceProperty ('gstR').

Ed. Note: The handler type to be used for Gaussian Splat data tracks ('auxv', 'meta', or a dedicated handler type) is for further study.

A Gaussian Splat data track may convey one or more roles. When a Gaussian Splat data track conveys more than one role, or when the order of roles cannot be derived from other information, the mapping of roles to samples within a Gaussian Splat access unit shall be provided by a GaussianSplatTrackRoleMapBox ('gsrm') as specified in X.3.2.1.

#### X.3.2.1 Gaussian Splat track role map box

##### X.3.2.1.1 Definition

Box type:	'gsrm'

Container:	Sample entry box

Mandatory:	Conditional, when a Gaussian Splat data track conveys more than one role or when an explicit mapping of roles to samples is needed

Quantity:	One

The GaussianSplatTrackRoleMapBox identifies the roles conveyed by a Gaussian Splat data track and provides the mapping of each role to one or more samples within a Gaussian Splat access unit.

##### X.3.2.1.2 Syntax

aligned(8) class GaussianSplatTrackRoleMapBox

extends FullBox('gsrm', version == 0, flags == 0){

unsigned int(16) access_unit_sample_count;

unsigned int(16) entry_count;

for(i=0; i<entry_count; i++){

unsigned int(32) role_code;

unsigned int(16) sh_band_index;

reserved bit(16) reserved;

unsigned int(16) sample_offset;

unsigned int(16) sample_count;

unsigned int(32) role_offset;

unsigned int(32) role_length;

}

}

##### X.3.2.1.3 Semantics

access_unit_sample_count specifies the number of consecutive samples that form one Gaussian Splat access unit. When access_unit_sample_count is equal to 1, each sample corresponds to one access unit.

entry_count specifies the number of role mapping entries that follow.

role_code and sh_band_index specify the role and, when applicable, the spherical harmonics band index, using the same semantics as specified for GaussianSplatRoleProperty in X.2.3.2.

sample_offset specifies the zero-based index of the first sample within the access unit that contributes to the role. sample_count specifies the number of consecutive samples within the access unit that contribute to the role.

role_offset specifies the zero-based byte offset within the decoded byte sequence of each contributing sample to the first byte of the role byte sequence. role_length specifies the number of bytes contributed by each contributing sample. When sample_count is greater than 1, the complete role byte sequence is formed by concatenating role_length bytes from each contributing sample in increasing sample order, starting at role_offset.

## X.4 Gaussian Splat specific brands

### X.4.1 General

#### X.4.1.1 ’gsc1’ brand

The brand 'gsc1' is specified in the following subclauses.

#### X.4.1.1.1 Requirements on files

Files shall include 'mif1' among the compatible brands and hence conform to the 'mif1' structural brand as defined in HEIF specification (ISO/IEC 23008-12).

Additionally, there shall be at least one Gaussian Splat representation item of type 'gspl' present in the file that is the target of an item reference of type ‘gsil’ from a conventional image item in the same file. The Gaussian Splat representation item shall identify Gaussian Splat data units either by one or more item references of type 'gsst', or by an associated 'gstR' item property, or both.

When the 'gsc1' brand is among the compatible brands, Gaussian Splat stream items referenced by the Gaussian Splat representation item shall be of type 'gspc'. Each stream item shall have an associated 'gscC' item property as specified in X.2.3.3. The value 'gscu' of compressor_code indicates uncompressed stream data. If the Gaussian Splat representation item identifies one or more tracks using an associated 'gstR' item property, each referenced track may carry Gaussian Splat data units using an image or video coding format. When a referenced track conveys more than one role, a 'gsrm' box shall be present in the sample entry as specified in X.3.2.1.

Files conforming to the 'gsc1' brand shall set gs_format equal to 'gsis' in the GaussianSplatGlobalConfigurationProperty associated with the Gaussian Splat representation item.

#### X.4.1.1.2 Requirements on readers

The requirements on readers that are specified for the ‘mif1’ structural brand in HEIF (ISO/IEC 23008-12) shall be supported.

Readers for the 'gsc1' brand shall be able to parse a Gaussian Splat representation item of type 'gspl' and the identified data units and reconstruct the uncompressed stream data for each referenced role. Readers shall be able to parse the 'gscC' property and obtain the compressor_code and decoding parameters for each stream item. When compressor_code is equal to 'gscu', no decoding is required. When compressor_code identifies a supported coding method, a reader shall be able to decode the stream item and reconstruct the uncompressed stream data. When compressor_code identifies an unsupported coding method, a reader may ignore the corresponding stream item. If a Gaussian Splat representation item identifies one or more tracks using an associated 'gstR' item property, a reader shall be able to locate the referenced track(s) and, when present, parse the 'gsrm' box in the sample entry to determine the mapping of roles to samples.

NOTE: A file including a Gaussian Splat representation is required to include a viewable master image item conforming to 'mif1'. A reader that does not support Gaussian Splat rendering can still display the master image.

#### X.4.1.2 ‘gsc2’ brand

The brand 'gsc2' is specified in the following subclauses.

#### X.4.1.2.1 Requirements on files

Files shall include 'mif1' among the compatible brands as defined in HEIF specification (ISO/IEC 23008-12).

Additionally, there shall be at least one Gaussian Splat representation item of type ‘gspl’ present in the file that is the target of an item reference of type ‘gsil’ from a conventional image item in the same file.

When the 'gsc2' brand is among the compatible brands, Gaussian Splat stream items referenced by the Gaussian Splat representation item with item reference ‘gsst’ may be one of the following item types:

- An item of type ‘mime’ (i.e. MIME item)

- A coded image item of type ‘hvc1’ of ‘vvc1’ as defined in HEIF specification (ISO/IEC 23008-12)

- An item with an item type other than the above listed item types whose interpretation is defined by the gs_format syntax element value in  GaussianSplatGlobalConfigurationProperty.

If a Gaussian Splat stream item is a ‘mime’ item, item_name syntax element of ‘infe’ box shall contain the file name. Absolute paths, relative paths, directory components, drive letters, URI prefixes, and path separator characters (e.g. '/' and '\') shall not be present.

#### X.4.1.2.2 Requirements on readers

Readers for the 'gsc2' brand shall be able to understand ‘mif1’ brand as specified in HEIF specification (ISO/IEC 23008-12). Readers shall be able to parse a Gaussian Splat representation item and the associated GaussianSplatGlobalConfigurationProperty. When the gs_format syntax element value of the GaussianSplatGlobalConfigurationProperty  identifies a supported Gaussian Splat Representation format, a reader shall be able to parse the referenced Gaussian Splat stream items. If the gs_format syntax element does not contain a supported format value, the reader shall not further process the Gaussian Splatting stream items and fall back to the primary image item.

# 4	Extracted 4CCs

| 4CC | Clause | First Occurrence Paragraph |
| --- | --- | --- |
| gsil | X.2.2.1 | A Gaussian Splat (GS) representation item is a non-image item of type 'gspl' that is the target of an item reference of type 'gsil' from the cover image item... |
| gsrC | X.2.3.2 Gaussian Splat role property | A Gaussian Splat stream item ... ('gsrC') ... |
| gspc | X.2.2.2.1 Gaussian Splat Stream item | compressor_code is a four character code identifying the compression method used for the item of type 'gspc'... |
| gspl | X.2.2.1 Gaussian Splat Representation Item | Mandatory (per item): Conditional, for a Gaussian Splat representation item of type 'gspl'... |
| gstR | X.2.3.4 Syntax | extends ItemFullProperty('gstR', version = 0, flags = 0)... |
| gsst | X.2.3.4 Semantics | When a 'gstR' item property ... 'gsst' ... |
| gsrm | X.3.2 Gaussian Splat data tracks | A Gaussian Splat data track ... GaussianSplatTrackRoleMapBox ('gsrm') ... |
| gsc1 | X.4.1.1 General | The brands 'gsc1' and 'gsc2' are specified in the following subclauses... |
| gscu | X.2.3.3 Semantics | The value 'gscu' indicates uncompressed stream data. |
| gsis | X.2.3.1 Semantics | The value 'gsis' indicates the structured binary representation specified in this annex. |
| gsc2 | #### X.4.1.2 ‘gsc2’ brand | The brand 'gsc2' indicates the carriage of an externally defined Gaussian Splat representation formats other than ‘gsis’. |
| mif1 | X.4.1.2 Requirements on files | Files shall include 'mif1' among the compatible brands... |
| gscC | X.4.1.2 Requirements on files | Each stream item of type 'gspc' ... 'gscC' ... |
| meta | 2.4 Examples | MetaBox 'meta': |
| hvc1 | 2.4 Examples | 1) infe: item_ID=1, item_type='hvc1'... |
| ipco | 2.4 Examples | ItemPropertyContainerBox 'ipco': |
| ftyp | 2.4 Examples | FileTypeBox 'ftyp': major-brand='heic' (or 'gsc1')... |
| heic | 2.4 Examples | FileTypeBox 'ftyp': major-brand='heic'... |
| hdlr | 2.4 Examples | HandlerBox 'hdlr': 'pict' |
| pict | 2.4 Examples | HandlerBox 'hdlr': 'pict' |
| pitm | 2.4 Examples | PrimaryItemBox 'pitm': item_ID=1 |
| iinf | 2.4 Examples | ItemInfoBox 'iinf': entry_count=2 |
| iloc | 2.4 Examples | ItemLocationBox 'iloc': |
| iref | 2.4 Examples | ItemReferenceBox 'iref': |
| iprp | 2.4 Examples | ItemPropertiesBox 'iprp': |
| hvcC | 2.4 Examples | 'hvcC' 'ispe' // cover config and size |
| ispe | 2.4 Examples | 'hvcC' 'ispe' // cover config and size |
| gsgC | 2.4 Examples | 'gsgC' // GS global config |
| ipma | 2.4 Examples | ItemPropertyAssociation 'ipma': |
| moov | 2.4 Examples | MovieBox 'moov': |
| posi | 2.4 Examples | entry[0]: role_code='posi'... |
| rotq | 2.4 Examples | entry[1]: role_code='rotq'... |
| opac | 2.4 Examples | entry[3]: role_code='opac'... |
| shco | 2.4 Examples | entry[4]: role_code='shco'... |
| mdat | 2.4 Examples | MediaDataBox 'mdat': |
| mp42 | 2.4 Examples | compatible_brands: 'mif1', 'mp42'... |
| av01 | 2.4 Examples | sample_entry: 'hvc1' (or 'av01',... |
| vvc1 | 2.4 Examples | sample_entry: 'hvc1' (or 'av01', 'vvc1',...) |

We also checked MP4RA, and none of the proposed new 4CCs highlighted in red bold above are registered in MP4RA.

We also ran the SDL through the https://mpeggroup.github.io/mpeg-sdl-editor/ validator for any bugs.

# 5 	Example

Here is a detailed example with SPZ when ‘gsc1’ brand is used:

FileTypeBox 'ftyp':

major_brand = 'heic'

minor_version = 0

compatible_brands[] = { 'gsc1', 'mif1', 'heic', 'mp42' }

MetaBox 'meta':

FullBox header:

version = 0

flags = 0

HandlerBox 'hdlr':

version = 0

flags = 0

pre_defined = 0

handler_type = 'pict'

reserved[3] = 0

name = "Picture handler\0"

PrimaryItemBox 'pitm':

version = 0

flags = 0

item_ID = 1

ItemInfoBox 'iinf':

version = 0

flags = 0

entry_count = 3

ItemInfoEntry 'infe' (for item_ID=1):

version = 2

flags = 0

item_ID = 1

item_protection_index = 0

item_type = 'hvc1'

item_name = "CoverImage\0"

ItemInfoEntry 'infe' (for item_ID=10):

version = 2

flags = 0

item_ID = 10

item_protection_index = 0

item_type = 'gspl'

item_name = "GaussianSplatRepresentation\0"

ItemInfoEntry 'infe' (for item_ID=11):

version = 2

flags = 0

item_ID = 11

item_protection_index = 0

item_type = 'gspc'

item_name = "GaussianSplatStream_SPZ\0"

ItemLocationBox 'iloc':

version = 0

flags = 0

offset_size = 4

length_size = 4

base_offset_size = 8

index_size = 0

item_count = 3

item_ID = 1:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00001000     // P1

extent_length = 0x0001F000     // Q1

item_ID = 10:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00000000     // P10

extent_length = 0x00000000     // Q10

item_ID = 11:

construction_method = 0

data_reference_index = 0

base_offset = 0

extent_count = 1

extent[0]:

extent_offset = 0x00020000     // P11

extent_length = 0x00009400     // Q11

ItemReferenceBox 'iref':

version = 0

flags = 0

SingleItemTypeReferenceBox 'gsil':

from_item_ID = 10

reference_count = 1

to_item_ID[0] = 1

SingleItemTypeReferenceBox 'gsst':

from_item_ID = 10

reference_count = 1

to_item_ID[0] = 11

ItemPropertiesBox 'iprp':

ItemPropertyContainerBox 'ipco':

property[1] = HEVCConfigurationBox 'hvcC':

configurationVersion = 1

general_profile_space = 0

general_tier_flag = 0

general_profile_idc = 1

general_level_idc = 120

nalArray[] = (HEVC config payload)

property[2] = ImageSpatialExtentsProperty 'ispe':

version = 0

flags = 0

image_width = 1920

image_height = 1080

property[3] = GaussianSplatGlobalConfigurationProperty 'gsgC':

version = 0

flags = 0

gs_format = 'spz '            // 0x73707A20

num_points = 1000

sh_degree = 2

component_format = 0

coord_system = 1                    // 1 = right-handed

reserved = 0

property[4] = GaussianSplatCompressionConfigurationProperty 'gscC':

version = 0

flags = 0

compressor_parameter_length = 8

compressor_parameter[8] = {

0x00,                             // cfg_version = 0

0x03,                             // spz_version_policy = require v3

0x02,                             // input_coord_system = RDF

0x01,                             // output_coord_system = RUB

0x01,                             // flags: bit0 require_magic = 1

0x00, 0x00, 0x00                  // reserved

}

property[5] = GaussianSplatRoleProperty 'gsrC' for role posi:

version = 0

flags = 0

role_code = 'posi'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 0

role_length = 12000                 // 1000 points * 3 float32

property[6] = GaussianSplatRoleProperty 'gsrC' for role rotq:

version = 0

flags = 0

role_code = 'rotq'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 12000

role_length = 16000                 // 1000 * 4 float32

property[7] = GaussianSplatRoleProperty 'gsrC' for role scl3:

version = 0

flags = 0

role_code = 'scl3'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 28000

role_length = 12000                 // 1000 * 3 float32

property[8] = GaussianSplatRoleProperty 'gsrC' for role opac:

version = 0

flags = 0

role_code = 'opac'

role_instance = 0

role_count = 1

sh_band_index = 0xFFFF

reserved = 0

role_offset = 40000

role_length = 4000                  // 1000 * 1 float32

property[9] = GaussianSplatRoleProperty 'gsrC' for role shco band 0:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 0

reserved = 0

role_offset = 44000

role_length = 12000                 // band0 has 1 coeff triplet per point, 3 float32 per triplet

property[10] = GaussianSplatRoleProperty 'gsrC' for role shco band 1:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 1

reserved = 0

role_offset = 56000

role_length = 36000                 // band1 has 3 coeff triplets per point

property[11] = GaussianSplatRoleProperty 'gsrC' for role shco band 2:

version = 0

flags = 0

role_code = 'shco'

role_instance = 0

role_count = 1

sh_band_index = 2

reserved = 0

role_offset = 92000

role_length = 60000                 // band2 has 5 coeff triplets per point

ItemPropertyAssociation 'ipma':

version = 0

flags = 0

entry_count = 3

entry for item_ID=1:

association_count = 2

assoc[0]:

essential = 1

property_index = 1    // hvcC

assoc[1]:

essential = 0

property_index = 2    // ispe

entry for item_ID=10:

association_count = 1

assoc[0]:

essential = 1

property_index = 3    // gsgC

entry for item_ID=11:

association_count = 8

assoc[0]:

essential = 1

property_index = 4    // gscC compressor_code='spz '

assoc[1]:

essential = 1

property_index = 5    // gsrC posi

assoc[2]:

essential = 1

property_index = 6    // gsrC rotq

assoc[3]:

essential = 1

property_index = 7    // gsrC scl3

assoc[4]:

essential = 1

property_index = 8    // gsrC opac

assoc[5]:

essential = 1

property_index = 9   // gsrC shco band0

assoc[6]:

essential = 1

property_index = 10   // gsrC shco band1

assoc[7]:

essential = 1

property_index = 11   // gsrC shco band2

MediaDataBox 'mdat':

at file offset 0x00001000:

item_ID=1 bytes, length 0x0001F000

at file offset 0x00020000:

item_ID=11 bytes, length 0x00009400, contents:

gzip stream that is an SPZ bitstream

SPZ header and payload are opaque to HEIF

decompression produces a decoded byte sequence of length 152000 bytes

the role_offset and role_length in gsrC point into that decoded sequence

# 12	Discussion during MPEG meetings:

# 12.1 MPEG#153

During MPEG#153, online and gitlab based discussions happened. The gitlab discussions are here.

A summary of the questions are as follows

## A. Conceptual Scope & Placement

- Should GS be part of HEIF or be restricted to volumetric media standards (23090-10 / 18)?

- Does treating GS as an “image” stretch HEIF beyond its intended scope?

- If both domains support GS, how do we avoid fragmentation and ensure interoperability?

## B. Technical Model & Semantics

- Should GS representations really be modeled as auxiliary image items?

  - If so, what are the semantics of image dimensions (ispe) for 3D content?

- Is the use of role_offset and role_length appropriate or overly constraining?

  - Does it assume too much about the decoded GS structure?

- Should track referencing (gstR) be permitted within image items, or is this mixing incompatible mechanisms?

## C. Compatibility with Existing Codecs

- How to support GS codecs that internally use video/image coding formats (e.g., SOG) without duplicating existing signaling?

- Should new 4CCs be defined for codecs that already have registered sample entries?

## D. Real-World Deployment Concerns

- Are current GS codecs mature enough to define a generic long-term container?

- Do GS formats share enough structural commonality to justify a unified HEIF-based framework?

## E. Spec Evolution & Clarity

- Should the definitions of gspl, gspc, gscC, gsrC, etc., be simplified or aligned more closely with ISOBMFF semantics?

- Is more alignment needed with existing HEIF “image item” and “coded image” concepts?

# 12.2 MPEG#154

- During MPEG#154, several comments were received in the following Jira issues:m76764 [ISOBMFF] GS in HEIF: Identified Issues and proposed specification changes (#437) · Issues · MPEG / Systems / File Format / ISOBMFF · GitLab

- m76763 [ISOBMFF] Implementation Report and Usage Guide for Gaussian Splats in HEIF (#436) · Issues · MPEG / Systems / File Format / ISOBMFF · GitLab

- m76687 [WG3][Explorations] Possible interactions with HEIF and other GS content formats (#228) · Issues · MPEG / Systems / File Format / HEIF · GitLab

- m76612 [WG3][Explorations] On Exploration of carriage of static Gaussian Splat in HEIF (#225) · Issues · MPEG / Systems / File Format / HEIF · GitLab

Based on the discussions:

- Storage of existing gaussian splat representations as-is is added to the exploration document

- SOG-like 3DGS representations are added to the exploration document

- Usage of auxiliary images as gaussian splat representations is discouraged and current exploration document is updated accordingly

Following MPEG#154, this update unifies the storage mechanisms around the 'gspl' entry point item, normalizes the item reference type to 'gsil', renames the four-character-code field of the global configuration property to gs_format with MP4RA registration, removes the remaining auxiliary image machinery, restores the camera parameter relationships, and adds an informative parser behavior clause.

It also extends Gaussian Splat stream item’s possible item types, which are dependent on the gs_format value. Also, ‘gsc2’ brand is modified to cover gs_formats other than ‘gsis’.

# 12.3 MPEG#155

In MPEG 155, there were 3 contributions (m77791, m77779 and m77646). The text above is based on m78065. It is a merged version of m77791 and m77779, as well as inclusion of the ‘altr’ entity group and several other comments in the provided in m77646.

The relevant GitLab issues are as follows:

- m78065 Exploration on gaussian splat carriage in HEIF (#236) · Issues · MPEG / Systems / File Format / HEIF · GitLab

- m77646: Analysis and Comments on Exploration of carriage of 3DGS content in HEIF (#235) · Issues · MPEG / Systems / File Format / HEIF · GitLab

- m77779: [HEIF] On Exploration of carriage of static Gaussian Splat in HEIF (#232) · Issues · MPEG / Systems / File Format / HEIF · GitLab

- m77791: [Explorations] Updates to static GS in HEIF (#231) · Issues · MPEG / Systems / File Format / HEIF · GitLab

Changes can be summarized as follows:

1.	Gaussian Splat Representation items are defined as the entry point with item type ‘gspl’

2.	gs_4cc field is now renamed as gs_format

3.	An optional entity grouping of grouping type ‘altr’ is introduced that would contain the primary item and the Gaussian Splat Representation Item

4.	Additional Gaussian Stream item types  are introduced, as in m77779

5.	‘gsc2’ brand is updated to storage of representation other than having structured binary representations which are covered in brand ‘gsc1’.

6.	Several typo and editorial fixes

7.	Several comments are still left in the document for further considerations

Additionally, in MPEG GitLab, issue #236 captured the following comments and questions:

The following topics are still open for discussion:

- The overall placement of 3DGS carriage, including whether it belongs in HEIF or should instead be addressed within volumetric media specifications.

- The motivation and justification for defining 3DGS carriage in HEIF, particularly in view of existing approaches in G-PCC and V-PCC.

- Whether a primary item and viewable cover image should be mandatory for a 3DGS representation to ensure backward compatibility.

- The respective roles of gsil and altr, and whether both mechanisms are needed.

- The cardinality model and its impact on use cases such as multiple levels of detail (LOD) and shared representation components.

- Alignment with glTF-based ecosystems and MPEG-I Scene Description (ISO/IEC 23090-14).

- Architectural alignment and interoperability with existing MPEG and Khronos technologies.

- Consistency of examples and figures with the latest design direction.

- Remaining editorial inconsistencies and outdated references.

- “TBD” marked/commented sections in this document

# 13	References

[1]	ISO/IEC 23008-12:2024, Information technology - High efficiency coding and media delivery in heterogeneous environments - Part 12: Image File Format.

[2]	ISO/IEC 14496-12:2022, Information technology - Coding of audio-visual objects - Part 12: ISO base media file format.

[3]	RFC 3986, Uniform Resource Identifier (URI): Generic Syntax.

[4]	Kerbl et al., “3D Gaussian Splatting for Real-Time Radiance Field Rendering”

[5] 	m76612, “[WG3][Explorations] On Exploration of carriage of static Gaussian Splat in HEIF”, MPEG#154

[6] 	m76687, “[WG3][Explorations] Possible interactions with HEIF and other GS content formats”, MPEG#154

[7] 	m76612, “On Exploration of carriage of static Gaussian Splat in HEIF”, MPEG#154

[8] 	m76687, “Possible interactions with HEIF and other GS content formats”, MPEG#154

[9] 	m76764, “GS in HEIF: Identified Issues and proposed specification changes”, MPEG#154

[10] 	m76763, “ Implementation Report and Usage Guide for Gaussian Splats in HEIF”, MPEG#154

[11]       m78065, “Exploration on gaussian splat carriage in HEIF”, MPEG#155

[12] 	m77646, “Analysis and Comments on Exploration of carriage of 3DGS content in HEIF”, MPEG#155

[13] 	m77779, “[HEIF] On Exploration of carriage of static Gaussian Splat in HEIF”, MPEG#155

[14] 	m77791, “[Explorations] Updates to static GS in HEIF”, MPEG#155