# ISO/IEC JTC 1/SC 29/WG 03 N1883

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	Working Draft 1 for 23001-17 2nd Edition

Status:	Working Draft

Date of document:	2026-07-22

Source:	ISO/IEC JTC 1/SC 29/WG 03

No. of pages:	80 (with cover page)

Email of Convenor:	young.L @ samsung . com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 3

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 3 n 1883

Geneva – July 2026

| Title | [Raw] Working Draft 1 for 23001-17 2nd Edition |  |
| --- | --- | --- |
| Author | Scott Houchin |  |
| Reference | - 23001-17 Uncompressed still and motion imagery |  |
| Serial Number | 26714 |  |

The attached draft of 23001-17 2nd Edition represents the output of the July 2026 WG3 meeting.

ISO/IEC JTC 1/SC 29 /WG3 N nnnnn

Date: 2026-07-06

ISO/IEC 23001-17:202X(E)

ISO/IEC JTC 1/SC 29/WG 3

Secretariat: JISC

Information technology — MPEG Systems technologies — Part17: Carriage of uncompressed video and images in ISO Base Media File Format

Document type: Working Draft 1

Document subtype:

Document language: E

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manager of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	6

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

3.1	block	2

3.2	component	2

3.3	frame	2

3.4	interleaving	2

3.5	pixel	2

3.6	row	2

3.7	sample data	2

3.8	tile	2

3.9	uncompressed frame	2

3.10	uncompressed image	2

3.11	uncompressed video	3

3.12	TAI	3

3.13	TAI clock	3

3.14	receptor clock	3

3.15	remote clock	3

3.16	coordinated universal time, UTC	3

3.17	global positioning system, GPS	3

3.18	precision time protocol, PTP	3

3.19	network time protocol, NTP	3

3.20	SI second	4

3.21	generically-compressed,adj	4

3.22	generically-compressed item	4

3.23	generically-compressed sample	4

4	Uncompressed video and image formats	4

4.1	Overview	4

4.2	Storage in media tracks	4

4.3	Storage in image items	5

5	Uncompressed frame description	6

5.1	Component Definition	6

5.1.1	Definition	6

5.1.2	Syntax	7

5.1.3	Semantics	8

5.2	Uncompressed Frame Configuration	8

5.2.1	Definition	8

5.2.2	Syntax	22

5.2.3	Semantics	22

5.2.4	Examples (Informative)	23

5.3	Profiles for uncompressed frame configurations	29

5.3.1	Overview	29

5.3.2	Predefined configurations	30

5.4	MIME type sub-parameters	31

6	Component description extensions	32

6.1	Extensions for uncompressed video and uncompressed images	32

6.1.1	Overview	32

6.1.2	Component Palette configuration	32

6.1.3	Component Pattern Definition	33

6.1.4	Component Reference Level	35

6.1.5	Polarization Pattern Definition	37

6.1.6	Sensor Non-Uniformity Correction	39

6.1.7	Sensor Bad Pixels Map	40

6.1.8	Chroma Location	42

6.1.9	Frame Packing Information	42

6.1.10	Disparity Information	43

6.1.11	Depth Mapping Information	44

6.2	Sample group descriptions	44

6.2.1	Field Interlace Type	44

6.3	Image Item properties	45

6.3.1	Field Interlace Property	45

7	Multiple tracks and items storage	46

7.1	Overview	46

7.2	Component video track group	46

7.3	Image tiling using ISOBMFF tracks and items	46

8	Labeling of Samples and Items	47

8.1	High Precision Time Tagging	47

8.1.1	TAI clock information box	49

8.1.2	TAI timestamp packet	51

8.1.3	Sample TAI timestamps	52

8.1.4	Item TAI timestamps	52

9	Generic compression of items and sample data	53

9.1	Overview	53

9.2	Compression Configuration Box	54

9.2.1	Definition	54

9.2.2	Syntax	54

9.2.3	Semantics	55

9.3	Generically-compressed media tracks	55

9.3.1	Overview	55

9.3.2	Generically Compressed Units Info Box	56

9.4	Generically-compressed items	57

9.4.1	Overview	57

9.4.2	Generically Compressed Units Item	57

9.5	Hypothetical reconstruction model	59

10	Mapping to other ISOBMFF capabilities	60

10.1	Mapping of generic sample compression to uncompressed image and sample data	60

10.2	Combined usage of Generic Sample Compression and Common Encryption	60

11	Generic compression brands	61

11.1	Brand for generic compression of media tracks	61

11.2	Brand for generic compression of items	62

Bibliography	63

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives or www.iec.ch/members_experts/refdocs).

ISO and IEC draw attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO and IEC take no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO and IEC had not received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents and https://patents.iec.ch. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see www.iso.org/iso/foreword.html. In the IEC, see www.iec.ch/understanding-standards.

.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information Technology, Subcommittee SC 29, Coding of audio, picture, multimedia, and hypermedia.

A list of all parts in the ISO/IEC 23001 series can be found on the ISO and IEC websites.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html and www.iec.ch/national-committees.

Information technology — MPEG Systems technologies — Part 17: Carriage of uncompressed video and images in ISO Base Media File Format

# Scope

This document specifies how uncompressed 2D image and video data is carried in files in the family of standards based on the ISO base media file format (ISO/IEC 14496-12). This includes but is not limited to monochromatic data, colour data, transparency (alpha) information and depth information.

The primary goal of this document is to allow exchange of uncompressed video and image data while relying on the information set provided by the ISO base media file format, such as timing, colour space and sample aspect ratio to specify the interpretation and/or display of video and image data.

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format

ISO/IEC 23008-12, Information technology – High efficiency coding and media delivery in heterogeneous environments – Part 12: Image File Format (HEIF)

ISO/IEC 23001-16, Information technology — MPEG Systems technologies — Part16: Derived visual tracks in the ISO base media file format

IEEE 754-2008, IEEE Standard for Floating-Point Arithmetic

IETF RFC 1951, DEFLATE Compressed Data Format Specification version 1.3

IETF RFC 1950, ZLIB Compressed Data Format Specification version 3.3

IETF RFC 7932, Brotli Compressed Data Format

# Terms and definitions

For the purposes of this document, the terms and definitions given in ISO/IEC 14496-12 and the following apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

- ISO Online browsing platform: available at https://www.iso.org/obp

- IEC Electropedia: available at http://www.electropedia.org/

## block

consecutive bytes within the sample data containing one or more component values for one or more pixels and possible padding

## component

part of the image data representing a single channel (or dimension) of the image

Note to entry: In this document, a component may describe visual information such as luminance or chroma, or other information usually not intended for direct display such as depth or transparency.

## frame

two-dimensional rectangular array of pixels contained in the sample data

## interleaving

ordering of pixel or component within the sample data

## pixel

smallest element of an image, comprised of one or more components

## row

horizonal line of pixels within a frame or a tile

## sample data

payload of the media sample when the uncompressed frame is described by a media track, or payload of the item when the uncompressed frame is described by an image item

Note 1 to entry: Media sample as defined in ISO/IEC 14496-12.

Note 2 to entry: Image item as defined in ISO/IEC 23008-12.

## tile

two-dimensional rectangular array of pixels within a frame

## uncompressed frame

frame for which each value of each component is coded independently from any other component value in the same frame or any other frame

Note to entry: In this document, the uncompressed term is used with some video formats applying sub-sampling of some components for the purpose of data reduction; however, data access to each individual component for such formats is still independent from other components or frames.

## uncompressed image

single uncompressed frame stored as an image item

## uncompressed video

sequence of one or more uncompressed frames

TAI

international atomic time

high-precision continuous scale of time derived from hundreds of precise atomic clocks from around the world and maintained as closely as possible to the International System (SI) second.

Note 1 to entry: Current practice achieves a maximum deviation of approximately one second every 100 million years.

Note 2 to entry: The abbreviated term comes from the French "Temps Atomique International".

TAI clock

clock capable of synchronizing to a source of TAI time and generating TAI timestamps

receptor clock

clock located where measurements are made (e.g. local to a sensor) and capable of synchronizing to a source of time from a remote clock.

remote clock

clock capable of transmitting time over significant distances and usually highly accurate (e.g. GPS system or PTP Grand Master)

coordinated universal time, UTC

international standard for regulating clocks and time, forming a basis for civil time

Note 1 to entry: UTC is based on TAI but includes irregularly inserted leap second additions or subtractions to account for variation in the earth’s rotation.

global positioning system, GPS

satellite system providing global positioning, navigation, and timing services

Note 1 to entry: Timing services are based on TAI time.

precision time protocol, PTP

protocol for synchronizing clocks to a source of TAI time across computer networks

Note 1 to entry: IEEE 1588-2008 defines the precision time protocol.

Note 2 to entry: PTP systems can achieve measurement uncertainties below a microsecond.

network time protocol, NTP

protocol for synchronizing clocks to UTC time across computer networks

Note 1 to entry: RFC 5905 defines the Network Time Protocol.

Note 2 to entry: Systems using NTP typically achieve measurement uncertainties in the range of milliseconds.

SI second

International System of Units (SI) base unit for measuring time

## generically-compressed,adj

compressed using one of a defined set of lossless compression algorithms

Note to entry: Supported compression algorithms are specified in Table 7.

## generically-compressed item

item that has been generically-compressed

## generically-compressed sample

sample that has been generically-compressed

## auxiliary image

image that may not be intended to be displayed but provides supplemental information, such as transparency data, complementing a respective master image

NOTE1 An auxiliary image and its master image are linked using an item reference of 'auxl' from the auxiliary image to the master image.

NOTE2 A track with reference type 'auxl' can have a coding dependency; its use is clarified by specifications that use it.

NOTE3 When multiple track references describe an auxiliary video track, derived specifications can constrain or recommend which track references are used. For example, derived specifications can constrain or recommend whether to use 'vdep' or 'auxl' or both for auxiliary depth video track.

## Pre-derived coded image

###### a coded image that has been derived from one or more other images

## Pre-derived coded image

An image item including a 'base' item reference

If a coded image has been derived from others — for example, a composite HDR image derived from exposure-bracketed individual images — then it shall be linked to those images by item references of type 'base' from the coded image to all images it derives from.

NOTE As per the current HEIF document, the exact derivation process used to produce the image is not described.

## Derived visual tracks

video or picture track that contains a timed sequence of samples containing an ordered list of derivation operations

NOTE Derived visual tracks are designed to enable defining a timed sequence of visual transformation operations to be applied to input still images and/or samples of timed sequences of images in the same presentation.

## Raw image

raw imagery is a subset of uncompressed imagery where no corrections or adjustments have been made to the measured sensor component values

          - NOTE Imagery captured directly from a sensor without non-uniformity corrections, white balance, bad pixel replacement, deBayering, etc. are raw.

# Uncompressed video and image formats

## Overview

Uncompressed frames may be stored in ISO base media files as media samples of media tracks or as image items using a generic uncompressed video description defined in this document.

Media tracks, media samples and image items may be further described using the various tools defined in ISO/IEC 14496-12, such as sample group descriptions, MetaBox, metadata tracks and sample auxiliary information. User-defined components may be used to carry per-pixel specific information, either in the same sample or item as the described pixels or in a separate track or item.

The tools defined in ISO/IEC 14496-12 and ISO/IEC 23008-12 should be used whenever applicable, namely to specify pixel aspect ratio, colour information, clean aperture, content light level, mirror and rotate properties or track header matrix, etc.

An uncompressed video media sample or item consists of one uncompressed frame. Each uncompressed frame is organized as a set of one or more rectangular, non-overlapping and contiguous (without holes) areas called tiles.

ISOBMF allows constructing files referring to external data. This enables building ISOBMF files describing existing uncompressed image or video files without having to copy the media data, simplifying integration into existing workflows.

## Storage in media tracks

Uncompressed video tracks compliant to this document are tracks compliant to ISO/IEC 14496-12 that use a VisualSampleEntry with codingname equal to 'uncv', hereafter called uncompressed video sample entry.

The uncompressed video sample entry shall contain

- one UncompressedFrameConfigBox

- one ComponentDefinitionBox if the UncompressedFrameConfigBox version is not 1.

When both UncompressedFrameConfigBox and ComponentDefinitionBox are present in the sample entry, the ComponentDefinitionBox shall precede the UncompressedFrameConfigBox.

The compressorname field of an uncompressed video sample entry should be set to all 0 (empty string). The depth field of an uncompressed video sample entry shall be ignored and should be set to 0, the bit depth per component being indicated by the UncompressedFrameConfigBox .

The handler type associated with the track is usually 'vide', 'auxv' or 'pict' but derived specifications may introduce new handler types. The width and height fields of the sample entry documents the exact frame dimension, in pixels of any non-subsampled component, of any sample of the video stream that is described by this sample entry. Consequently, if the frame dimension changes within a video track, multiple sample entries shall be used.

The payload of an uncompressed video media sample consists of one uncompressed frame.

The size in bytes of an uncompressed video media sample shall be at least the size in bytes required to store all the components values documented by the uncompressed video configuration as defined in subclause 5.2.

NOTE	The sample data can be larger than the size in bytes required to store all the components values, typically to store information in the trailing data. How such additional bytes are handled by a file reader is out of scope of this document.

Each uncompressed video media sample shall be marked as a sync sample. As a consequence, the SyncSampleBox, ShadowSyncSampleBox, CompositionOffsetBox and CompositionToDecodeBox shall not be present in the track.

Media tracks containing only non-visual components should be marked as not present in the presentation, i.e. track_in_movie flag should not be set.

Media tracks containing user-defined components providing per-pixel information for pixels in another track should use a track reference of type 'cdsc' to the track they describe.

## Storage in image items

An uncompressed image compliant to this document is an image item compliant to ISO/IEC 23008-12 with the item_type 'unci'.

An uncompressed image shall be associated with:

- an UncompressedFrameConfigBox essential item property, i.e., essential shall be equal to 1 for an UncompressedFrameConfigBox item property associated with an image item of type 'unci',

- a ComponentDefinitionBox essential item property if the UncompressedFrameConfigBox version is not 1,

- an ImageSpatialExtentsProperty whose image_width and image_height fields shall document the exact frame dimension, in pixels, of the reconstructed image, i.e. the size of the image before applying any associated transformative properties.

The payload of an uncompressed image consists of one uncompressed frame.

The size in bytes of an uncompressed image item shall be at least the size in bytes required to store all the components values documented by the uncompressed video configuration as defined in subclause 5.2.

NOTE	The image item data can be larger than the size in bytes required to store all the components values, typically to store information in the trailing data. How such additional bytes are handled by a file reader is out of scope of this document.

An uncompressed image can be further documented using the various tools defined in ISO/IEC 14496-12 or ISO/IEC 23008-12, such as descriptive item properties.

Uncompressed images containing only non-visual components should be marked as hidden items, i.e. have (flags & 1) equal to 1 in their ItemInfoEntry.

Uncompressed images containing user-defined components providing per-pixel information for pixels in another item should use an item reference of type 'cdsc' to the item they describe.

If an uncompressed item is associated with an AuxiliaryTypeProperty indicating alpha (resp. depth), the uncompressed item shall have one and only one component of type alpha (resp. depth).

# Uncompressed frame description

## Component Definition

### Definition

Box Type:	'cmpd' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory: see below
Quantity:	At most one per uncompressed video sample entry or at most one associated per uncompressed image item

The ComponentDefinitionBox is used to document the types of components present in samples or items associated with this box through the sample entry or through item property association.

Components defined in the ComponentDefinitionBox are referenced by indexes in various boxes in this document. Care has to be taken while removing components from an uncompressed video or image to also remove in other boxes any reference to the removed components. There is no requirement that all components defined in the ComponentDefinitionBox are referenced by other boxes.

For all boxes referring to components defined in the ComponentDefinitionBox , the associated ComponentDefinitionBox (resp. the associated UncompressedFrameConfigBox) is defined as:

- for an uncompressed video, the ComponentDefinitionBox (resp. UncompressedFrameConfigBox) present in the same sample entry as the referring box,

- for an uncompressed image, the ComponentDefinitionBox (resp. UncompressedFrameConfigBox) associated, through ItemPropertyAssociationBox, to the same image item as the referring box.

This box shall be present if the version of the associated UncompressedFrameConfigBox is 0.

This box may be present if the version of the associated UncompressedFrameConfigBox is 1.

When the ComponentDefinitionBox is not present, the associated ComponentDefinitionBox is implicitly defined as indicated by the profile of the associated UncompressedFrameConfigBox , see subclause 5.3.

The component_index field value defined in boxes using an associated ComponentDefinitionBox indicates the index in the list of components, with value 0 indicating the first component listed in the associated ComponentDefinitionBox. The component_index field value shall be strictly less than the component_count field value of the associated ComponentDefinitionBox.

A ComponentDefinitionBox may describe two or more components with the same type (e.g. two monochrome components representing different portions of the electromagnetic spectrum), and file readers may require additional information to process the pixel data. How such information is provided to the file reader is out of scope of this document.

Table 1 – Component types

| Value | Description |
| --- | --- |
| 0 | Monochrome component |
| 1 | Luma component (Y) |
| 2 | Chroma component (Cb / U) |
| 3 | Chroma component (Cr / V) |
| 4 | Red component (R) |
| 5 | Green component (G) |
| 6 | Blue component(B) |
| 7 | Alpha/transparency component (A) |
| 8 | Depth component (D) |
| 9 | Disparity component (Disp) |
| 10 | Palette component (P) The component_format value for this component shall be 0. |
| 11 | Filter Array (FA) component such as Bayer, RGBW, etc. |
| 12 | Padded component (unused bits/bytes) |
| 13 | Cyan component (C) |
| 14 | Magenta component (M) |
| 15 | Yellow component (Y) |
| 16 | Key (black) component (K) |
| 17 | Not involved in default commodity display |
| 18 to 0x7FFF | ISO/IEC reserved |
| 0x8000 to 0xFFFF | User-defined component type(s) |

Component types reflect only a nominal characterization of the pixel data and how that pixel data should be displayed; precise bandpass limits, for example, are not implied and may differ from image to image. For some component types, some other boxes can provide additional information, such as ColourInformationBox or MasteringDisplayColourVolumeBox. For other component types, the signalling of such information is out of scope of this document.

if a ColourInformationBox is present for the media sample or item:

- if the ColourInformationBox describes components by numbers, there shall be at least as many components in the reconstructed image as indicated by the ColourInformationBox , and components are matched by indices,

- if the ColourInformationBox describes components by labels (e.g., Y, U/Cb, V/Cr or R, G, B), these components shall be present in the image after applying palette or filter array, if any.

If the bands need to be identified beyond the component type, it is recommended to use URIs in place of the enumerated component_type value.

### Syntax

aligned(8) class ComponentDefinitionBox extends Box('cmpd') {
	unsigned int(32) component_count;	
	{
		unsigned int(16) component_type;
		if (component_type>=0x8000) {
			utf8string component_type_uri;
		}
	} [component_count];
}

### Semantics

component_count indicates the number of components described in this box

component_type indicates the type of the component, as defined in Table 1

component_type_uri indicates a URI describing the user-defined component type

## Uncompressed Frame Configuration

### Definition

#### Overview

Box Type:	'uncC' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	Yes, if codingname of the sample entry is 'uncv' or if the item_type of the item is 'unci'
Quantity:	One per uncompressed video sample entry or one associated per uncompressed image item

The UncompressedFrameConfigBox describes uncompressed frames that are composed of one or more components. RGB or YUV formats are typical examples of such uncompressed frames for which each colour component is a component of the uncompressed frame. Other component types can also be described (e.g., disparity, transparency, palette). The type of each component is specified by the component_type field in the associated ComponentDefinitionBox.

Component values may be absolute values or indexes into a colour palette, with adjustable black and white reference levels. Pattern-based sensor data, such as Bayer, can be described through user-defined patterns, together with various sensor information (polarization, non-uniformity correction, broken pixels, etc.).

For each component, this box specifies the numerical format (e.g. unsigned integer, IEEE 754 binary32 floating point) and bit depth through the component_format and component_bit depth_minus_one fields.

Pixel data may be interleaved per component, per pixel, per row or per tile, as specified by the interleave_type field, with byte alignment for each row and tile specified by the fields row_align_size and tile_align_size. Pixel data may also be grouped together in blocks, typically to respect endianness constraints, as specified by the block_size, block_pad_lsb and block_little_endian fields.

Some formats, such as YUV video, do not always use the same 2D resolution for each component of the frame. This is indicated by the sampling_type field.

The UncompressedFrameConfigBox may indicate a profile, allowing faster identification of the class of video data used. Profiles are defined in subclause 5.3. Some profiles may use version 1 of the UncompressedFrameConfigBox, in which case only the profile is given and the ComponentDefinitionBox, if absent, is implicitly defined by the profile.

#### Component ordering and constraints

Each pixel in a frame is made of one or more components, where each component is assigned a type (e.g., ‘Y’, ‘U’, ‘V’), as detailed in 5.1.

The order in which components are specified in the UncompressedFrameConfigBox indicates the order in which components are placed into the sample data or within blocks, prior to blocking and endian conversion.

Some formats, such as Bayer image data, contain a 2D array of single-component values, with each individual component value assigned to a component type using a fixed pattern. Such formats are described as mono-component data with no subsampling, use a component type of 11 (‘FA’). There shall be at most one component with type 11 (‘FA’) present in the component list. There may be additional components of other types present, for example to associate an alpha component with a Bayer image. If a component with type 11 (‘FA’) is present, there shall be a ComponentPatternDefinitionBox present in the video sample entry or associated to the item to indicate the pattern of component values.

Some formats code colours according to a set of predefined colours, or palette. Such formats are described as single-component with no subsampling, use a component type of 10 (‘P’). There shall be at most one component with type 10 (‘P’) present in the component list. There may be additional components of other types present, for example to associate per-pixel alpha component with a palette image. If a component with type 10 (‘P’) is present, there shall be an ComponentPaletteBox present in the video sample entry or associated to the item to indicate the palette values.

#### Component size and numerical format

The variable component_bit_depth for a component is defined as (component_bit_depth_minus_one + 1).

For a given component, the binary representation of each value is given by component_bit_depth (the size in bits of each component value) and component_format.

The possible values for component_format field is defined in Table 2.

Table 2 - Component formats

| Value | Description |
| --- | --- |
| 0 | Component value is an unsigned integer coded on component_bit_depth bits. |
| 1 | Component value is an IEEE 754 binary float number coded on component_bit_depth bits (e.g. if component_bit_depth is 16, then the component value is coded as IEEE 754 “binary16”). For this component format, component_bit_depth values shall be 16, 32, 64, 128 or 256; other values are forbidden. |
| 2 | Component value is a complex number coded on component_bit_depth bits, where the first component_bit_depth/2 bits represent the real part and the next component_bit_depth/2 bits represent the imaginary part. Each part is coded as an IEEE 754 binary float of the size component_bit_depth/2. For this component format, component_bit_depth values shall be 32, 64, 128 or 256; other values are forbidden. |
| 3 | Component value is a signed two’s complement integer coded on component_bit_depth bits. For this component format, component_bit_depth values shall be greater than 1. |
| other values | ISO/IEC reserved for future definition |

If component_align_size is 0, the component value is coded on component_bit_depth bits exactly.

Otherwise (component_align_size is not 0), the component value is coded as a word WC of component_align_size bytes, starting on a byte boundary. This implies that some pre-alignment padding bits may be present after the previous component value stored; if such pre-alignment padding bits are present, they shall be set to 0. The least significant bit of the component value is located at the least significant bit of WC. Alignment padding bits, if present, are located at the most significant bits and shall be set to 0. If components_little_endian is 0, WC is stored as a big-endian word. Otherwise (components_little_endian is 1), WC is stored as a little-endian word.

NOTE	For example, a pixel with 10-bit unaligned (component_align_size=0) R, G and B components, followed by a 1-byte aligned 7-bit A component, stored using pixel interleaving mode, would exist in the sample data as 30 consecutive bits containing R, G and B, followed by 2 pre-alignment  padding bits for byte alignment, followed by one alignment padding bit then followed by the 7-bit A value (bringing the A value aligned on 1 byte), for a total of 5 bytes.

For each component, component_align_size shall be either 0 or such that  component_align_size*8 is greater than component_bit_depth. If components_little_endian is 1, block_little_endian shall be 0 and component_align_size of each component shall be different from 0.

Storage of aligned component values is illustrated in Figure 27.

#### Tiling

The uncompressed frame data is structured in a 2D grid of tiles, where the number of tiles in the horizontal direction (resp. vertical direction) is specified by the variable num_tile_cols_minus_one+1 (resp. num_tile_rows_minus_one+1). Tiles allow grouping together the component values of pixels close to each-other (i.e., in the same spatial region of the frame).

All tiles have the same width and height. The frame width (resp. height) shall be a multiple of num_tile_cols_minus_one+1 (resp. num_tile_rows_minus_one+1). The tile width is w/(num_tile_cols_minus_one+1), with w the frame width, and the tile height is h/(num_tile_rows_minus_one+1), with h the frame height.

If the width (resp. height) in a source image is not an integer multiple of num_tile_cols_minus_one+1 (resp. num_tile_rows_minus_one+1), an application creating the tiled frame shall pad the source image with an appropriate number of columns to the right (resp. rows to the bottom), and the original image dimension shall be documented using a CleanApertureBox for media tracks or a clean aperture transformative item property for image items. The width and height fields of the sample entry or the image_width and image_height fields of the ImageSpatialExtentsProperty shall specify the padded width and height.

Tiles shall be stored in raster-scan order: the top-left tile is stored first, followed by the tile to its right, and the first tile of a tile row is stored after the last tile of the previous tile row.

Within a tile, component values shall be stored in raster-scan order, left-to-right and top to bottom: for a given component, the value for pixel {x+1,y} is stored after (but possibly not contiguous with) the value for pixel {x,y}, and the value for pixel {x,y+1} is stored after (but possibly not contiguous with) the value for pixel {x, y}.

#### Sampling mode

##### Definition

All components in a frame either have the same dimensions or use pre-defined sampling modes, indicated by the sampling_type field. Possible values for this field are described in Table 3.

NOTE1 A sampling mode can restrict the possible interleaving modes since the number of values is not the same for each component.

NOTE2 Derived specifications can further restrict the usage of sampling modes with tiling.

Table 3 – Sampling type values

| Value | Sampling mode |
| --- | --- |
| 0 | No subsampling |
| 1 | YCbCr 4:2:2 subsampling |
| 2 | YCbCr 4:2:0 subsampling |
| 3 | YCbCr 4:1:1 subsampling |
| 4 to 0xFF | reserved |

##### No subsampling

In this mode, all components have the same width and height as the frame.

NOTE The tile width and height are not restricted. Derived specifications can further restrict this, for example to enforce width and height to be multiples of 2 in case Y, U and V components are present

##### YCbCr 4:2:2 Subsampling

This sampling mode shall only be used if the following conditions are all true:

- all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list,

- the tile width is a multiple of 2,

- the interleave_type field is set to 0 (component interleaving mode), 2 (mixed interleaving mode) or 5 (multi-Y pixel interleaving mode).

The width and height of the ‘Y’ component are the width and height of the frame. The height of the ‘U’ and ‘V’ component is the same as the height of the ‘Y’ component. The width of the ‘U’ and ‘V’ component is half the width of the ‘Y’ component. Components of other types may be present, and have the same dimension as the ‘Y’ component.

Pixels {x,y} and {x+1,y}, with x%2==0, share the same component values ‘U’ and ‘V’.

If row_align_size is not 0 and interleave_type is 0:

- row_align_size shall be a multiple of 2

If tile_align_size is not 0:

- tile_align_size shall be a multiple of 2

##### YCbCr 4:2:0 Subsampling

This sampling mode shall only be used if the following conditions are all true:

- all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list,

- both tile width and height are multiple of 2,

- the interleave_type field is set to 0 (component interleaving mode) or 2 (mixed interleaving mode).

The width and height of the ‘Y’ component are the width and height of the frame. The width of the ‘U’ and ‘V’ component is half the width of the ‘Y’ component’. The height of the ‘U’ and ‘V’ component is half the height of the ‘Y’ component. Components of other types may be present, and have the same dimension as the ‘Y’ component.

Pixels {x,y}, {x+1,y}, {x,y+1} and {x+1,y+1} with x%2==0 and y%2==0, share the same component values ‘U’ and ‘V’.

If row_align_size is not 0 and interleave_type is 0:

- row_align_size shall be a multiple of 2

If tile_align_size is not 0:

- tile_align_size shall be a multiple of 4

##### YCbCr 4:1:1 Subsampling

This sampling mode shall only be used if the following conditions are all true:

- all three component_type values ‘Y’, ‘U’ and ‘V’ are present in the component list,

- tile width is a multiple of 4,

- the interleave_type field is set to 0 (component interleaving mode), 2 (mixed interleaving mode) or 5 (Multi-Y pixel interleaving mode).

The width and height of the ‘Y’ component are the width and height of the frame. The height of the ‘U’ and ‘V’ component is the same as the height of the ‘Y’ component. The width of the ‘U’ and ‘V’ component is the width of the ‘Y’ component divided by 4. Components of other types may be present, and have the same dimension as the ‘Y’ component.

Pixels {x,y} , {x+1,y} , {x+2,y} , {x+3,y}, with x%4==0, share the same component values ‘U’ and ‘V’.

If row_align_size is not 0 and interleave_type is 0:

- row_align_size shall be a multiple of 4

If tile_align_size is not 0:

- tile_align_size shall be a multiple of 4

#### Interleaving modes

##### Definition

The interleaving mode of pixels within a tile (or within the frame if a single tile is used) is indicated by interleave_type, as defined in Table 4. The interleave_type shall apply to all listed components unless stated otherwise in the interleaving mode definition.

NOTE The interleaving describes the layout of values within the sample data. Positioning of sample data within the container file is done according to ISO/IEC 14496-12.

Table 4 – Interleaving type values

| Value | Interleaving mode |
| --- | --- |
| 0 | Component interleaving |
| 1 | Pixel interleaving |
| 2 | Mixed interleaving |
| 3 | Row interleaving |
| 4 | Tile-component interleaving |
| 5 | Multi-Y pixel interleaving |
| 6 to 0xFF | Reserved |

##### Component interleaving

For a given component, values for all pixels of a tile shall be located sequentially in the sample data. Component values shall be located in the order the components were declared.

Component interleaving mode can be used with a single tile, as illustrated in Figure 1, or with multiple tiles, as illustrated in Figure 2. In both examples, R is the first component, G is the second and B is the third.

Figure 1 – Single tile component interleaving example

Figure 2 – Multiple tiles component interleaving example

##### Pixel interleaving

For a given pixel, the component values shall be located one after the other, in the order the components are declared, with the first component of a pixel following the last component of the previous pixel.

Pixel interleaving mode shall only be used if sampling_type field value is 0, i.e. when all components are at the same resolution.

Pixel interleaving mode can be used with a single tile, as illustrated in Figure 3 or with multiple tiles, as illustrated in Figure 4.

Figure 3 – Single tile Pixel interleaving example

Figure 4 – Multiple tiles Pixel interleaving example

##### Mixed interleaving

Mixed interleaving mode shall only be used if the following conditions are all true:

- sampling_type field value is not 0,

- sampling_type value explicitly allows this interleaving mode,

- the ‘U’ and ‘V’ components are consecutive in the component list, i.e. ‘V’ component immediately follows ‘U’ component or ‘U’ component immediately follows ‘V’ component.

For all components other than ‘U’ and ‘V’, component values for all pixels shall be located as specified for component interleaving mode in the order they are declared. The ‘U’ and ‘V’ component values shall be located as specified by pixel interleaving mode; the first value for component ‘U’, if declared first, or ‘V’ otherwise, shall be located after the last value of the component preceding ‘U’ or ‘V’ if any, or first in the tile otherwise.

NOTE Mixed interleaving mode is typically used to store YUV 420 data with all Y components followed by interleaved U and V components.

Figure 5 illustrates mixed interleaving for sampling_type=2 and Figure 6 illustrates mixed interleaving for sampling_type=1.

Figure 5 – Mixed interleaving for YUV 420 example

Figure 6 – Mixed interleaving for YUV422 example

##### Row interleaving

For a given row, component values for each component shall be located sequentially, in the order the components are declared. Each non-first row in the tile shall be located after the previous row. If multiple tiles are used, the first component of the first row of a tile shall be located after the last component of the last row of the previous tile.

Row interleaving mode shall only be used if sampling_type field value is 0, i.e. when all components are at the same resolution.

Row interleaving mode can be used with a single tile, as illustrated in Figure 7, or with multiple tiles, as illustrated in Figure 8.

Figure 7 – Single tile row interleaving example

Figure 8 – Multiple tiles row interleaving example

##### Tile-component interleaving

For each component, in the order they are declared, the values for each component for all tiles shall be located sequentially, with values within the tile located per each individual tile.

Tile-component interleaving mode shall not be used if only a single tile is defined.

Tile-component interleaving mode shall only be used if sampling_type field value is 0, i.e. when all components are at the same resolution.

Tile-component interleaving mode is illustrated in Figure 9.

Figure 9 – Tile-component interleaving example

##### Multy-Y pixel interleaving

Component values shall be organized as with interleave_type=1 (pixel interleaving mode), and the list of components shall contain multiple ‘Y’ components representing the multiple ‘Y’ component values associated with a single pair of ‘U’ and ’V’ component values.

The ‘U’ and ‘V’ components shall only be listed once in the list of components.

NOTE There are no restrictions on the order of declaration of the ‘Y’ components; derived specifications can further restrict this.

Regardless of how the multiple ‘Y’ components are interleaved with the ‘U’ and ‘V’ components, the ‘Y’ component values shall be stored in left-to-right, top-to-bottom order.

If the chrominance components are subsampled according to 4:2:2 subsampling (sampling_type=1), the component list describes two horizontally consecutive pixels, and thus shall include two ‘Y’ components, representing the two ‘Y’ values associated with a single pair of ‘U’ and ‘V’ component values. For example, {‘U’, ‘Y’, ‘V’, ‘Y’} for sampling_type=1 (YUV 4:2:2) indicates that the Y value of the second pixel shall be located after the ‘V’ component value.

Figure 10 – ‘U’, ‘Y’, ‘V’, ‘Y’ 4 :2 :2 example

If the chrominance components are subsampled according to 4:1:1 subsampling, the component list describes four horizontally consecutive pixels, and thus shall include four Y components, representing the four Y values associated with a single pair of U and V component values. For example, {‘U’, ‘Y’, ‘Y’, ‘V’, ‘Y’, ‘Y’} for sampling_type=3 (YUV 4:1:1) indicates that the ‘Y’ value of the second pixel shall be located after the ‘Y’ value of the first pixel, that the ‘Y’ value of the third pixel shall be located after the ‘V’ component value and that the ‘Y’ value of the fourth pixel shall be located after the ‘Y’ value of the third pixel.

Figure 11 – ‘U’, ‘Y’, ‘Y’, ‘V’, ‘Y’, ‘Y’ 4 :1 :1 example

Multi-Y pixel interleaving mode shall not be used for other forms of chrominance subsampling.

#### Block and value alignment to byte boundaries

Padding bits can be included within the sample data to ensure that component values or pixels are aligned to specific byte boundaries. In addition, the end of rows and tiles can be aligned to specific byte boundaries. This may be to ensure individual component values or pixels are CPU addressable (e.g., not split across two 32-bit words), and rows and tiles of images are aligned to key boundaries (e.g., to memory pages or disk drive sectors).

Component values can be stored either directly in the sample data or inside fixed-size blocks. The block size in bytes is specified by the block_size field.

If the block size is 0 (blocking is not used within the sample data):

- no padding is performed between values, except potentially:

  - in-between component values if some components use a component_align_size different from 0

  - after the last component value of a pixel if pixel_size is set

  - after the last value in a row if row_align_size is set

  - after the last value in a tile if tile_align_size is set

- block_pad_lsb, block_little_endian and block_reversed shall all be 0,

- values shall be stored most-significant bit first.

Storage of component values with block_size=0 is illustrated in Figure 25.

If the block size is not 0:

- each block shall consist in exactly block_size bytes and shall contain exactly one or more component values, i.e. the bits representing a component value are always contained within a single block,

- by default, within a block, the first value of the first component of the top-left pixel of the frame shall be stored in the highest order bits of the first block (big-endian “left to right” format). The second value (for next component or next pixel of same component) shall be stored within the next set of bits in the block up until the last component value being stored in the least significant bits of the block,

- if the block size in bits is greater than the size in bits of component values present in the block, padding bits are present. When present, these padding bits shall be located either in the most or in the least significant bits,

- the block size shall be greater than or equal to the maximum value of component_align_size for each component.

If pad_unknown value is 1, the padding bits values are not restricted, otherwise the padding bits shall all be set to 0.

NOTE1 Padding bits can be more than 8 (one byte). Unrestricted padding bit values is typically used for legacy content with no restrictions on padded values. Derived specification can further restrict usage of pad_unknown.

If blocks are used, the block containing the first component of the top-left pixel of the frame shall begin at the first byte of the sample data. Otherwise (blocks are not used), the most significant bits of the first component of the top-left pixel of the frame shall be located at the most significant bits of the first byte of the sample data.

When block_pad_lsb is 0, the least significant bit of the last component value in the block shall be located at the least significant bit of the block and, consequently, any padding bits are located in the most significant bits of the block. Otherwise (block_pad_lsb is 1), the most significant bit of the first component value in the block shall be located at the most significant bit of the block and, consequently, any padding bits are located in the least significant bits of the block.

The order of storage, in the sample data, of the bytes within the block is determined by the endian storage type. Figure 12 shows a 32-bit block with bits labelled from 0 (least significant bit) to 31 (most significant bit) with the following component assignments:

Component 1: Type 4 (Red) – 8 bits

Component 2: Type 5 (Green) – 8 bits

Component 3: Type 6 (Blue) – 8 bits

Padding bits are located at the least significant bits of the block.

The least significant bit of each component is labelled ‘0’ and the most significant bit ‘7’.

Figure 12 – Assignment of 8-bit RGB components to a 32-bit block

If block_little_endian is 0 (big endian storage), the most significant byte of the block (Component 1 in Figure 12) shall be stored in the lowest storage byte location in the sample data. The remaining bytes shall be stored in increasing address spaces (Component 2 then 3 in Figure 12) and the least significant byte of the block (Padding bits in Figure 12) shall be stored in the highest address location. The big-endian storage arrangement for the component assignments of Figure 12 is shown in Figure 13.

Figure 13 – Big-endian storage of the block defined in Figure 12

If block_little_endian is 1 (little endian storage), the least significant byte of the block (Padding bits in Figure 12) shall be stored in the lowest storage byte location. The remaining bytes shall be stored in increasing address space (Component 3 then 2 in Figure 12) and the most significant byte of the block (Component 1 in Figure 12) shall be stored in the highest address location. The little-endian storage arrangement for the block in Figure 12 is shown in Figure 14.

Figure 14 – Little-endian storage of the block defined in Figure 12

If block_reversed is 0, the components order within the block shall be the component declaration order. Otherwise (block_reversed is 1), the partition of components into blocks shall remain unchanged, but the components order within each block shall be inverted, without changing the location of padding bits. block_reversed shall be 0 if block_little_endian is 0.

NOTE2 Reversing the order of components in a block is typically used with little-endian storage when multiple blocks are needed to store a single pixel value, to ensure the low bits of a block are either a continuation of the previous pixel or a start of a new one.

An example usage of block_reversed=1 is shown in Figure 23 and Figure 24. Figure 29 illustrates usage of block_reversed=1 with multi-Y interleaving mode.

If pixel_size is 0, no additional padding is present after each pixel. Otherwise, let PixelBytes be the number of bytes required to contain all component values of a single pixel, including any padding resulting from block_size or component_align_size. pixel_size shall be greater than or equal to PixelBytes, and the number of padding bytes present after the last byte of each pixel shall be pixel_size - PixelBytes. Usage of pixel_size different from 0 is illustrated in Figure 28.

NOTE3	pixel_size is typically used to ensure the start of every pixel falls on a significant architectural boundary (e.g., all pixels start on 8-byte boundaries).

pixel_size shall be 0 if interleave_type is different from 1 or 5.

If both block_size and pixel_size are non-zero, then the first component value of any pixel shall be the first value stored in the first block for this pixel. This implies that the last block of any pixel may contain additional padding bits. For example, for a 10-bit RGBA image stored using block_size=4 and pixel_size=10, the first block for each pixel will contain the R, G, and B components and 2 bits of padding while the second block for each pixel will only contain the A component value and 22 bits of padding, and two additional bytes of padding will be present after the second block.

NOTE4 Specifying pixel_size together with block_size ensures no block contains values from different pixels.

When block_size and pixel_size are both non-zero, the additional padding at the end of each pixel is not required to be a multiple of the block size.

Rows and tile rows shall be byte-aligned at the end of the row :

- if the block size is 0, padding bits until byte alignment may be present after the last component value of the last pixel of each row,

- Otherwise, the last block of each row (containing the last component value of the last pixel of each row) may contain more padding bits than previous blocks.

If row_align_size is 0, no additional padding is present at the end of rows and tile rows. Otherwise, let RowSize be the number of bytes required to contain, for a given row R:

- all values of all components of row R (including all component, block and pixel padding within and at the end of the sample data for row R) if interleave_type is 1 or 5;

- all values of pixel interleaved components (‘U’ and ‘V’) of row R if interleave_type is 2 and component type is ‘U’ or ‘V’ (including all component, block and pixel padding within and at the end of the sample data for row R);

all values of the current component for row R (including all component and block padding within the sample data for row R) otherwise.

The number of padding bytes present after the last byte of the current row shall be the smallest integer N, with N>=0, such that (RowSize+N) % row_align_size is 0.

If tile_align_size is 0, no additional padding is present at the end of tiles. Otherwise, let TileSize be the number of bytes required to contain, for a given tile T:

- all values of the current component for all rows of the tile T (including all component, block and row padding within and at the end the tile T) if interleave_type is 4,

- all values of all components for all rows of the tile T (including all component, block, pixel and row padding within and at the end of the sample data for the tile T) otherwise.

The number of padding bytes present after the last byte of the last row of the tile shall be the smallest integer N, with N>=0, such that (TileSize+N) % tile_align_size is 0.

tile_align_size shall be 0 if a single tile is used.

NOTE5 Block alignment allows groups of consecutive component values, regardless of interleaving mode, to be aligned within an easily addressable value; e.g., multiple components can be packed into 32-bit values but no component value spans two consecutive 32-bit values. For example, a 10-bit RGBA image using pixel interleaving mode with no component padding and 32-bit (4-byte) block padding on the right will produce a repeated pattern of four 32-bit blocks describing 3 pixels [{R,G,B}, {A, R, G}, {B, A, R}, {G, B, A}]. If tile (resp. row) padding is also specified to be 32-bit (4-byte) then the start the next tile (resp. row) will start at the next 4-byte boundary, regardless of whether space for the next component value was available in the previous block. For example, with 4-byte tile alignment, if the width of the tile is not a multiple of 3, the last block of the line will only contain 1 or 2 component values but all 4-bytes of the last block will be present in the sample data.

### Syntax

aligned(8) class UncompressedFrameConfigBox extends FullBox('uncC', version, 0) {

	unsigned int(32) profile;	
	if (version==1) {
	}
	else if (version==0 || version==2) {
		unsigned int(32) component_count;	
		{
			if (version==0) }
				unsigned int(16) component_index;
			} else {
				unsigned int(32) component_index;
			unsigned int(8) component_bit_depth_minus_one;
			unsigned int(8) component_format;
			unsigned int(8) component_align_size;
		} [component_count];
		unsigned int(8) sampling_type;
		unsigned int(8) interleave_type;
		unsigned int(8) block_size;
		bit(1) components_little_endian;
		bit(1) block_pad_lsb;
		bit(1) block_little_endian;
		bit(1) block_reversed;
		bit(1) pad_unknown;
		bit(3) reserved = 0;
		unsigned int(32) pixel_size;
		unsigned int(32) row_align_size;
		unsigned int(32) tile_align_size;
		unsigned int(32) num_tile_cols_minus_one;
		unsigned int(32) num_tile_rows_minus_one;
	}
}

### Semantics

profile indicates the predefined configuration for this uncompressed frame configuration, as defined in Table 5. Value 0 indicates this uncompressed frame configuration may or may not be compliant to a profile. If not 0, all remaining fields in the box shall have the values indicated in Table 5 for this profile. The four-character code 'gene' is reserved and shall be interpreted as a value of 0. If version is 1, the profile shall be a valid profile listed in Table 5 for which version=1 is allowed.

component_count indicates the number of components present in the frames described by this uncompressed frame configuration

component_index indicates the index of the component listed in the associated ComponentDefinitionBox

component_bit_depth_minus_one indicates the size in bits minus one of the values for the component

component_format indicates the format of the component, as defined in Table 2

component_align_size indicates the component byte-alignment size, as described in subclause 5.2.1.3

sampling_type indicates the sampling mode as defined in Table 3

interleave_type indicates the interleaving mode of the components, as defined in Table 4

block_size indicates the size in bytes of blocks, as described in subclause 5.2.1.7

components_little_endian indicates that components are stored as little endian, as described in subclause 5.2.1.3

block_pad_lsb indicates padding bits location, as described in subclause 5.2.1.7

block_little_endian indicates block endianness, as described in subclause 5.2.1.7

block_reversed indicates if component order is reversed within the block, as described in subclause 5.2.1.7

pad_unknown indicates that the value of padded bits in blocks or padded components is unknown

pixel_size indicates the total number of bytes (including component and block padding) used to store all components for a single pixel when component values for that pixel are interleaved, as described in subclause 5.2.1.7

row_align_size indicates the padding between rows, as described in subclause 5.2.1.7

tile_align_size indicates the padding between tiles, as described in subclause 5.2.1.7

num_tile_cols_minus_one plus one indicates the horizontal number of tiles in the frame

num_tile_rows_minus_one plus one indicates the vertical number of tiles in the frame

### Examples (Informative)

The following figures illustrate the assignment of three components to 32-bit word blocks and associated storage configurations for endian and block padding options. The least significant bit of the block or of each component is labelled ‘0’, the most significant bit is labelled ‘31’. The three components for most of the examples are defined as follows:

Component 1: Type 4 (Red) – 9 bits

Component 2: Type 5 (Green) – 10 bits

Component 3: Type 6 (Blue) – 9 bits

Figure 15 shows the assignment of the components to a 32-bit block (block_size=4) with padding at the most significant bits (block_pad_lsb=0).

Figure 15 – Example assignment of RGB to 32-bit block, padded at most significant bits

Figure 16 shows the assignment of the components to a 32-bit block (block_size=4) with padding at the least significant bits (block_pad_lsb=1).

Figure 16 – Example assignment of RGB to 32-bit block, padded at least significant bits

The remaining figures in this section show various combinations of endian, block padding, block reverse, component alignment and pixel alignment options. “Storage Byte 0” (resp. “Storage Byte 3”) in the figures is the first (resp. last) byte stored in the sample data for the illustrated pixel or block.

Figure 17 illustrates a 32-bit block stored in big endian (with block_little_endian=0) and padding bits at least significant bits (block_pad_lsb=1).

Figure 17 – Big-endian block padded at least significant bits

Figure 18 illustrates a 32-bit block stored in big endian (with block_little_endian=0) and padding bits at most significant bits (block_pad_lsb=0).

Figure 18 – Big-endian block padded at most significant bits

Figure 19 illustrates a 32-bit block stored in little endian (with block_little_endian=1) and padding bits at least significant bits (block_pad_lsb=1).

Figure 19 – Little-endian block padded at least significant bit

Figure 20 illustrates Figure 19 with the byte order in the sample data reversed to better show component alignment.

Figure 20 – Little-endian block padded at least significant bit with the storage byte order reversed

Figure 21 illustrates a 32-bit block stored in little endian (with block_little_endian=1) and padding bits at most significant bits (block_pad_lsb=0).

Figure 21 – Little-endian block padded at most significant bit

Figure 22 illustrates Figure 21 with the byte order in the sample data reversed to better show component alignment.

Figure 22 – Little-endian block padded at most significant bit with the storage byte order reversed

Figure 23 illustrates a 32-bit block stored in little endian (with block_little_endian=1), padding bits at most significant bits (block_pad_lsb=0) and reversed order of component in block (block_reversed=1).

Figure 23 – Little-endian block padded at most significant bit with reversed component order

Figure 24 illustrates Figure 23 with the byte order in the sample data reversed to better show component alignment.

Figure 24 – Little-endian block padded at most significant bit with reversed component order  with the storage byte order reversed

Figure 25 shows storage of component Red (9 bits), Green (10 bits) and Blue (9 bits) with no block and no pixel alignment.

Figure 25 - RGB – red (9 bits), green (10 bits), blue (9 bits), no block, no padding bits

Figure 26 shows storage of components Red, Green, Blue, Alpha, each on 5 bits with no block and 3 bytes per pixels (pixel_size=3).

Figure 26 – RGBA (each 5 bits), no block and 3 bytes pixel size

Figure 27 shows storage of components Red (10 bits), Green (12 bits), Blue (10 bits) with no block and component_align_size=2 for the Green component only.

Figure 27 – Red (10-bit unaligned), Green (12-bit, 2-byte alignment), Blue (10-bit unaligned), no block

Figure 28 shows storage of components Red, Green, Blue, Alpha, each on 9 bits within a 32-bit blocks.

Figure 28 – RGBA (each 9 bits), block size (4 bytes), pixel size (8 bytes)

Figure 29 shows storage of multi-Y interleaved YUV with 422 subsampling, each component coded on 10 bits, within 32-bit blocks with block_little_endian=1 and block_reversed=1, also known as ‘v210’ format.

Figure 29 – Multi-Y interleaved component 10-bit UYVY in 32 bit little endian reversed blocks with bytes shown in reverse order per 32-bit word

## Profiles for uncompressed frame configurations

### Overview

The four-character codes defined in this subclause identify common formats for which a profile is defined. This profile may be indicated in the UncompressedFrameConfigBox to simplify pixel format detection by file processors.

Profiles only provide identification of the pixel format used in uncompressed frames. Image reconstruction may need additional information, such as colour information or chroma location, present in child boxes of the sample entry of the track or associated with the image.

NOTE	Some of these profiles are introduced to document existing practices in the industry prior to the definition of this document.

### Predefined configurations

In Table 5 below, the values from the ComponentDefinitionBox and the UncompressedFrameConfigBox are listed as:

{profile, [{component_type, component_bit_depth_minus_1}], sampling_type, interleave_type}

The array of {component_type, component_bit_depth_minus_1} is given in component order as specified in the UncompressedFrameConfigBox, and the number of elements in the array gives the value for component_count.

Unless indicated otherwise, all other fields of UncompressedFrameConfigBox, including flags and version, shall have a value of 0.

If version 1 is allowed for a profile, the implicit ComponentDefinitionBox is defined by the array of component_type in their order of appearance. For example, for the field values {'rgb3', [{4,7},{5,7},{6,7}], 0, 1}, the implicit ComponentDefinitionBox has 3 components [4,5,6].

Table 5 – Predefined uncompressed frame formats

| Profile identifier | Description | Field values for UncompressedFrameConfigBox |
| --- | --- | --- |
| '2vuy' | 8 bits YUV 422 packed Cb Y0 Cr Y1 | {'2vuy', [{2,7},{1,7},{3,7},{1,7}], 1, 5} |
| 'yuv2' | 8 bits YUV 422 packed Y0 Cb Y1 Cr | {'yuv2', [{1,7},{2,7},{1,7},{3,7}], 1, 5} |
| 'yvyu' | 8 bits YUV 422 packed Y0 Cr Y1 Cb | {'yvyu', [{1,7},{3,7},{1,7},{2,7}], 1, 5} |
| 'vyuy' | 8 bits YUV 422 packed Cr Y0 Cb Y1 | {'vyuy', [{3,7},{1,7},{2,7},{1,7}], 1, 5} |
| 'yuv1' | 8 bits YUV 411 packed Y0 Y1 Cb Y2 Y3 Cr | {'yuv1', [{1,7},{1,7},{2,7},{1,7},{1,7},{3,7}], 3, 5} |
| 'v308' | 8 bits YUV 444 packed Cr Y Cb | {'v308', [{3,7},{1,7},{2,7}], 0, 1} |
| 'v408' | 8 bits YUVA 444 packed Cb Y Cr A | {'v408', [{2,7},{1,7},{3,7},{7, 7}], 0, 1} |
| 'y210' | 10 bits YUV 422 packed LE Y0 Cb Y1 Cr | {'y210', [{1,9},{2,9},{1,9},{3,9}], 1, 5} block_size shall be 2. block_little_endian shall be 1. block_pad_lsb shall be 1. |
| 'v410' | 10 bits YUV 444 packed CbYCr, 2 unused bits | {'v410', [{2,9},{1,9},{3,9}], 0, 1} block_size shall be 4. block_little_endian shall be 1. block_pad_lsb shall be 1. block_reversed shall be 1. |
| 'v210' | YUV 422 10 bits packed CbYCr | {‘v210’, [{2,9},{1,9},{3,9},{1,9}], 1, 5} block_size shall be 4. block_little_endian shall be 1. block_pad_lsb shall be 0. block_reversed shall be 1. frame width shall be a multiple of 48. row_align_size shall be 0. |
| 'rgb3' | RGB 24 bits packed | {'rgb3', [{4,7},{5,7},{6,7}], 0, 1} version shall be 0 or 1. |
| 'i420' | YUV 420 8 bits planar YCbCr | {'i420', [{1,7},{2,7},{3,7}], 2, 0} |
| 'nv12' | YUV 420 8 bits semi-planar YCbCr | {'nv12', [{1,7},{2,7},{3,7}], 2, 2} |
| 'nv21' | YUV 420 8 bits semi-planar YCrCb | {'nv21', [{1,7},{3,7},{2,7}], 2, 2} |
| 'rgba' | RGBA 32bits packed | {'rgba', [{4,7},{5,7},{6,7},{7,7}], 0, 1} version shall be 0 or 1. |
| 'abgr' | RGBA 32bits packed | {'abgr', [ {7,7}, {6,7},{5,7},{4,7}], 0, 1} version shall be 0 or 1. |
| 'yu22' | YUV 422 8 bits planar YCbCr | {'yu22', [{1,7},{2,7},{3,7}], 1, 0} |
| 'yv22' | YUV 422 8 bits planar YCrCb | {'yv22', [{1,7},{3,7},{2,7}], 1, 0} |
| 'yv20' | YUV 420 8 bits planar YCrCb | {'yu20', [{1,7},{3,7},{2,7}], 2, 0} |

## MIME type sub-parameters

When the ‘codecs’ parameter of a MIME type is used, as defined in RFC 6381, its value shall be formatted, using field values of the ComponentDefinitionBox and the UncompressedFrameConfigBox, as follows (each element in the value is separated from the previous one using a dot ‘.’):

- The first element of the value shall be set to ‘uncv’ for video tracks and to ‘unci’ for image items,

- If the profile field is not 0, an element equal to the profile four-character code string (case sensitive) is appended

- Otherwise (the profile field value is 0), the string ‘gene’ (case sensitive) is appended

- Optionally if profile field is not 0, mandatory otherwise:

  - an element equal to the sampling_type expressed in hexadecimal is appended

  - an element equal to the interleave_type expressed in hexadecimal is appended

  - an element equal to the block_size expressed in hexadecimal is appended

  - an element equal to ‘cTr’ is appended, with c the number of tile columns in hexadecimal and r the number of tile rows in hexadecimal

  - for each component, in order of appearance, listed in the UncompressedFrameConfigBox, an element equal to ‘aLb’ may be appended, with a the hexadecimal value of component_type and b the hexadecimal value of component_bit_depth. Presence of this element is not mandatory, to limit the size of the ‘codecs’ parameter for images with very high number of components.

  - finally, an element equal to ‘Nz’ may be appended, with z the number of components in the UncompressedFrameConfigBox.

Hexadecimal value shall not be prefixed with ‘0x’, and leading zeros shall be omitted.

Examples:

- For an uncompressed video with profile yvyu: codecs=uncv.yvyu

- For an uncompressed video with monochrome and alpha, same sampling (sampling_type=0), component interleaving mode (interleave_type=0) with each value stored on 10 bits with 2 bytes blocks little-endian left-padded: codecs=uncv.gene.0.0.2.1T1.0LA.7LA

- For an uncompressed video with 2 bits alpha, 10 bits R, B, G, same sampling (sampling_type=0), pixel interleaving mode (interleave_type=1) with no specific block alignment (block_size=0) and a tiling of 4 horizontal for 3 vertical tiles: codecs=uncv.gene.0.1.0.4T3.7L2.4LA.5LA.6LA

# Component description extensions

## Extensions for uncompressed video and uncompressed images

### Overview

The boxes defined in this subclause can be used to provide further information on one or more components present in the uncompressed frame. Presence of these boxes may be mandated by the presence of specific components in the uncompressed frame (e.g., presence of a palette component mandates the presence of a ComponentPaletteBox).

Each of the defined boxes can be:

- added to a video sample entry for media tracks

- added as properties associated with an image item.

The syntaxes in this section are given for a video sample entry container and the defined boxes therefore extend Box (resp. FullBox). When used in an ItemPropertyContainerBox, the same syntax applies but the defined boxes extend ItemProperty (resp. ItemFullProperty). The properties defined in the following sections are descriptive properties.

NOTE Most of the boxes defined in clause 6 can apply to both uncompressed video formats and compressed video formats. Derived specifications can allow usage of these boxes for compressed formats.

### Component Palette configuration

#### Definition

Box Type:	'cpal' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The ComponentPaletteBox allows describing image data coded through a palette.

This box shall be present if and only if there is an associated ComponentDefinitionBox present, in which there is a component defined with a component_type value of 10 (‘P’).

If ComponentPaletteBox is used as an item property, it shall be marked as essential.

The component value of each pixel gives the index in the palette, and shall be an integer between 0 and values_count-1, with value 0 indicating the first entry in the list of pixel values of the palette.

In one ComponentPaletteBox, there shall not be any listed component with:

- the same component_type as a component listed in the associated UncompressedFrameConfigBox,

- a component_type value of 11 (‘FA’).

#### Syntax

aligned(8) class ComponentPaletteBox extends FullBox('cpal', 0, 0) {
	unsigned int(16) palette_components_count;	
	{
		unsigned int(32) component_index;
		unsigned int(8) component_bit_depth_minus_one;
		unsigned int(8) component_format;
	} [palette_components_count];

	unsigned int(32) values_count;
	for (i=0; i<values_count; i++) {
		for (j=0; j<palette_components_count; j++) {
			bit(X) component_value;
			aligned(8) bit(0) unused_bits_zero;
		}
	}
}

#### Semantics

palette_components_count indicates the number of components described by the palette

component_index indicates the index of the Nth component listed in the associated ComponentDefinitionBox.

component_bit_depth_minus_one indicates the size in bits minus one of values of the Nth component

component_format indicates the format of the Nth component, as defined in Table 2

values_count indicates the number of pixel values following

component_value indicates the value of the Nth component for the given entry. This field is coded on X bits with X = component_bit_depth_minus_one + 1 of the Nth component

unused_bits_zero represents the number of unused bits following the component value to achieve byte alignment. These unused bits shall be ignored by the file reader and should be set to 0.

### Component Pattern Definition

#### Definition

Box Type:	'cpat' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The ComponentPatternDefinitionBox allows describing filter array patterns such as Bayer.

This box shall be present if and only if there is an associated ComponentDefinitionBox present, in which there is a component defined with a component_type value of 11 (‘FA’).

If ComponentDefinitionBox is used as an item property, it shall be marked as essential.

The pattern is used to assign the final component type to each value. The pattern is defined at the top-left pixel of the image, and is repeated to cover the entire image. For a pixel position {x, y} in the image, with {0,0} being the top-left pixel, the coded value for that position represents the component type given by position {width%pattern_width, height%pattern_height} in the ComponentPatternDefinitionBox, and other component values for that pixel, as defined in the ComponentPatternDefinitionBox, are not present.

The tile width (resp. height) shall be a multiple of pattern_width (resp. pattern_height).

In one ComponentPatternDefinitionBox, there shall not be a listed component with:

- the same component_type as a component listed in the associated UncompressedFrameConfigBox,

- a component_type value of 10 (‘P’).

Examples:

- a BGGR Bayer image will be represented by a single component 11 (‘FA’) and a pattern with 2 rows, 2 columns indicating components [6,5,5,4],

- A GRBG Bayer image will be represented by a single component 11 (‘FA’) and a pattern with 2 rows, 2 columns indicating components [5,4,6,5],

- a BGGR Bayer image with an alpha plane following the Bayer data will be represented by two components 11 (‘FA’) and 7 (‘A’), an interleave_type of 0 and a pattern with 2 rows, 2 columns indicating components [6,5,5,4],

- a Red-White-Green-Blue sensor image will be represented by a single component 11 (“FA”) and a pattern with 4 rows, 4 columns indicating the R, G, B and monochrome components order.

#### Syntax

aligned(8) class ComponentPatternDefinitionBox extends FullBox('cpat', 0, 0) {
	unsigned int(16) pattern_width;
	unsigned int(16) pattern_height;
	for (i=0; i< pattern_height; i++) {
		for (j=0; j< pattern_width; j++) {
			unsigned int(32) component_index;
			double(32) component_gain;
		}
	}
}

#### Semantics

pattern_width indicates the width in pixels of the pattern

pattern_height indicates the height in pixels of the pattern

component_index indicates the index of the Nth component listed in the associated ComponentDefinitionBox.

component_gain indicates a gain to be applied to the Nth component, such as for white balance correction with Bayer imagery, etc. The value shall be coded as an IEEE 754 “binary32”.

NOTE The gain for each component is determined by a user defined method for balancing the levels of all components. The application of the gain terms is also a user defined implementation. This is commonly achieved by setting the gain value to one for the component with the highest signal level and to a value greater than one for the remaining components, thereby balancing all the components. Values saturate at the maximum value of a component.

### Component Reference Level

#### Definition

Box Type:	'clev' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one

The ComponentReferenceLevelBox allows describing the minimum and maximum values for components present in the image data.

When this box is present, there shall be an associated ComponentDefinitionBox present.

When this box is absent or a component type is not listed in this box, reference white and black values for this component type are derived from the ColourInformationBox present in the sample entry of the track or associated with the image item. If the ColourInformationBox is absent, the levels for the desired component type are derived as follows:

- For components of type Y, U or V with 8 bits depth, reference black is 16 and reference white is 235

- For components of type Y, U or V with 10 bits depth, reference black is 64 and reference white is for 940

- Otherwise, reference black is 0 and reference white is maximum value for component bit depth.

When ComponentReferenceLevelBox and ColourInformationBox are both present and document reference levels for the same component types, information from the ColourInformationBox shall be used.

NOTE If the ColourInformationBox is present with unspecified matrix_coefficients and has full_range_flag set to 1, full range is assumed.

In version 0 of this box, reference levels shall be ignored for non-integer component types. Version 1 of this box defines reference levels for all component formats defined in Table 2. In version 1, the black_level and white_level fields are coded either as signed int(32) or using the component_bit_depth and component_format of the referenced component, as selected by the native_level field and defined in subclause 6.1.4.2.

In version 0 of this box, if clip_range is set to 1, black_level  indicates the minimum value and white_level indicates the maximum value for the component; readers shall clip any value less than black_level to black_level and any value greater than white_level  to white_level.  In version 1 of this box, if clip_range is set to 1, readers shall clip each stored value to the range [black_level, white_level] before display or interpretation; for component_format 2 (complex), clipping is applied independently to the real and to the imaginary part.

In version 0 of this box, if clip_range is set to 0, readers shall transform the value N coded on k bits (as read from the sample data) to the value black_level + N × (white_level –black_level) / (2k–1) before display or interpretation. In version 1 of this box, if clip_range is set to 0, readers shall transform the stored value to a signal value before display or interpretation, using the formula selected by the component_format of the referenced component, as declared in the UncompressedFrameConfigBox:

- component_format 0 (unsigned integer): for a value N coded on k bits,

output = black_level + N * (white_level – black_level) / (2k – 1). Code 0 maps to black_level and code 2k – 1 maps to white_level.

- component_format 3 (signed integer): for a value N coded on k bits,

output = black_level + (N + 2k–1) * (white_level – black_level) / 2k–1. The minimum signed code -2k–1 maps to black_level and the maximum signed code 2k–1 – 1 maps to white_level.

- component_format 1 (float): the stored IEEE 754 value is the signal value; no transform is applied. black_level and white_level declare the expected signal range of the component.

component_format 2 (complex): the formula for component_format 1 is applied independently to the real and to the imaginary part.

#### Syntax

aligned(8) class ComponentReferenceLevelBox extends FullBox('clev', version, 0) {
   unsigned int(32) level_count;
   {
      unsigned int(32) component_index;
      unsigned int(1) clip_range;

if (version == 0) {
      bits(7) reserved = 0;
      signed int(32) black_level;
      signed int(32) white_level;

} else if (version == 1) {

bits(7) reserved = 0;

ComponentValue(component_index) black_level;

ComponentValue(component_index) white_level;

}

} [level_count];

}

ComponentValue(component_index) is a value coded using the component_bit_depth (denoted k) and the component_format of the component referenced by component_index, as declared in the UncompressedFrameConfigBox. The value is byte-aligned and coded big-endian as follows:

- component_format 0 (unsigned integer) and component_format 3 (signed integer): a field of ceil(k/8) bytes containing the value; an unsigned value is zero-extended and a signed value is sign-extended (two's complement) to the field width.

- component_format 1 (float): an IEEE 754 binary value of k bits, with k in {16, 32, 64, 128, 256}.

- component_format 2 (complex): the real part followed by the imaginary part, each an IEEE 754 binary value of k/2 bits, with k in {32, 64, 128, 256}.

Each entry begins on a byte boundary, so black_level and white_level are byte-aligned.

#### Semantics

version equal to 0 indicates that black_level and white_level are coded as signed int(32). version equal to 1 indicates that black_level and white_level are coded as defined in subclause 6.1.4.2. In version 1, black_level and white_level denote the minimum and maximum of the declared value range of the component, irrespective of component format and signal polarity. The field names are retained from version 0 for continuity only; in version 1 they carry no unipolar "black” or "white" meaning and shall be read solely as the minimum and maximum of the value range.

In version 1, black_level and white_level are coded as a ComponentValue, using the component_bit_depth and component_format of the referenced component, as defined in subclause 6.1.4.2. The version selects only the coding of the two level fields; the transform or clipping applied to stored values is selected by the component_format.

For component_format 1 (float) and component_format 2 (complex), version shall be 1,

because signed int(32) cannot represent these values. version shall also be 1 whenever

a level is not representable as signed int(32), for example an unsigned level greater than

231 – 1 or any component with component_bit_depth greater than 32. version equal to

0 may be used only when both black_level and white_level are representable as signed

int(32).

NOTE: For component_format 3 (signed integer), a bipolar signal whose range is symmetric about zero is fully described by a negative black_level and a positive white_level.

component_index indicates the index of the Nth component listed in the associated ComponentDefinitionBox.

clip_range indicates if the levels indicate a clip range or an affine transformation of the Nth component values

##### Examples (Informative)

An 8-bit unsigned component whose codes [0, 255] map onto a calibrated signal range

[0, 10000], using the signed int(32) level path:

clev version 1, level_count 1

component_index = 0

clip_range = 0                /* transform to signal value */

black_level = 0x00000000      /* 0 */

white_level = 0x00002710      /* 10000 */

With clip_range equal to 0, code 0 transforms to 0 and code 255 transforms to 10000. The

signal range exceeds the 8-bit code range, so the signed int(32) path is used.

A 32-bit float component carrying calibrated radiance clipped to [0.0, 10000.0]:

clev version 1, level_count 1

component_index = 0

clip_range = 1                          /* clip to [black_level, white_level] */

native_level = 1                        /* levels coded in component format */

black_level = 0x00000000                /* 0.0f, IEEE 754 binary32 */

white_level = 0x461C4000                /* 10000.0f, IEEE 754 binary32 */

A 12-bit signed component carrying sensor difference data with codes in [-2048, 2047],

declaring a calibrated signal range of [-1000, 1000]:

clev version 1, level_count 1

component_index = 0

clip_range = 0        /* transform to signal value */

black_level = 0xFC18  /* -1000, sign-extended to the 2-byte field */

white_level = 0x03E8  /* 1000 */

With clip_range equal to 0, the minimum code -2048 transforms to -1000, code 0 transforms

to approximately 0 (near the midpoint of both ranges), and code 2047 transforms to 1000. The

level fields are 12-bit signed values stored in a ceil(12/8) = 2-byte field, sign-extended.

A complex component with bit depth 64 (each part an IEEE 754 binary32), clipped to

[0.0, 1.0] on the real part and [-1.0, 1.0] on the imaginary part:

clev version 1, level_count 1

component_index = 0

clip_range = 1                       /* clip real and imaginary

parts independently */

black_level = 0x00000000 0xBF800000  /* real:  0.0f, imag: -1.0f,

each IEEE 754 binary32 */

white_level = 0x3F800000 0x3F800000  /* real:  1.0f, imag:  1.0f,

each IEEE 754 binary32 */

A 3-component float image (R, G, B, each IEEE 754 binary32) clipped to [0.0, 1.0]:

clev version 1, level_count 3

component_index = 0

clip_range = 1

native_level = 1

black_level = 0x00000000

white_level = 0x3F800000

component_index = 1

clip_range = 1

native_level = 1

black_level = 0x00000000

white_level = 0x3F800000

component_index = 2

clip_range = 1

native_level = 1

black_level = 0x00000000

white_level = 0x3F800000

### Polarization Pattern Definition

#### Definition

Box Type:	'splz' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or more

The PolarizationPatternDefinitionBox allows describing a filter array pattern for sensors that have polarization patterns implemented on the image sensor. If the sensor also includes a colour or spectral filter, the pattern dimension is not required to match the pattern dimension given in the ComponentPatternDefinitionBox.

When this box is present, there shall be an associated ComponentDefinitionBox present.

The pattern is used to assign polarization values at the pixel level. The pattern is defined at the top-left pixel of the image, and is repeated to cover the entire image. For a pixel position {x, y} in the image, with {0,0} being the top-left pixel, the polarization value for each pixel is given by position {width%pattern_width, height%pattern_height} in the PolarizationPatternDefinitionBox.

Polarization angles are specified counter-clockwise, with value 0 indicating an orientation to the right, as illustrated in Figure 30.

The tile width (resp. height) shall be a multiple of pattern_width (resp. pattern_height).

NOTE	If both pattern_width and pattern_height are 1, this implies that a single polarization angle is given for all pixels in the image.

Multiple PolarizationPatternDefinitionBox can be specified if components of the image have different polarization filters. In this case, there shall be at most one PolarizationPatternDefinitionBox describing one given component of the image.

Figure 30 illustrates an RGGB Bayer filter array pattern superimposed on a 4x4 pixel grid (filter array pattern with values of ‘FA1’ through ‘FA16’) with a 2x2 polarization pattern with angles of 90, 45, 135, and 0 degrees.

Figure 30 – Polarization example with a filter array pattern component

#### Syntax

aligned(8) class PolarizationPatternDefinitionBox extends FullBox('splz', 0, 0) {
	unsigned int(32) component_count;
	{
		unsigned int(32) component_index;
	} [component_count]
	unsigned int(16) pattern_width;
	unsigned int(16) pattern_height;
	for (i=0; i< pattern_height; i++) {
		for (j=0; j< pattern_width; j++) {
			double(32) polarization_angle;
		}
	}
}

#### Semantics

component_count indicates the number of components to which the polarization pattern applies. If this value is 0, the polarization pattern applies to all components of the image.

component_index indicates the index of the component listed in the associated ComponentDefinitionBox.

pattern_width indicates the width in pixels of the pattern

pattern_height indicates the height in pixels of the pattern

polarization_angle indicates the polarization angle of the pixel at the defined pattern location. Value shall either be 0xFFFFFFFF, indicating that no polarization filter is present, or shall be an IEEE 754 “binary32” number, indicating the polarization angle in degrees, with a value greater than or equal to 0.0 and strictly less than 360.0.

### Sensor Non-Uniformity Correction

#### Definition

Box Type:	'snuc' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or more

The SensorNonUniformityCorrectionBox allows describing pixel specific gain and offset corrections.

When this box is present, there shall be an associated ComponentDefinitionBox present.

Whether the correction has applied or not to the values stored in the sample data is indicated by the nuc_is_applied field.

The Non-Uniform Correction (NUC) nuc_gain and nuc_offset attributes provide non-uniformity corrections for a sensor. This is commonly used with radiometric imagery. The correction equation is linear: output= nuc_gain × input + nuc_offset, where input is an input component value as stored in the file. The minimum value of output saturates at a value of 0. In situations where the same component value range is to be maintained after application of the NUCs, the maximum value of output saturates at a level defined by the number of bits in the component value, for instance, 65535 for a 16-bit component.

The NUC coefficients are assigned per pixel through the full image. The NUC gain and offset tables are aligned with the entire image, resulting in a table width equal to image_width and table height equal to image_height. For a pixel position (x, y) in the image, with (0,0) being the top-left pixel, the NUC coefficients for each pixel are given by position {x,y} in the SensorNonUniformityCorrectionBox.

Multiple SensorNonUniformityCorrectionBox can be specified if components of the image have different gains and offsets. In this case, there shall be at most one SensorNonUniformityCorrectionBox describing one given component of the image.

When version is 1, the nuc_gain and nuc_offset data are not stored within the box, but instead linked to the image item or sample as specified in clause 12.

#### Syntax

aligned(8) class SensorNonUniformityCorrectionBox
extends FullBox('snuc', 0, flags) {
	unsigned int(32) component_count;
	{
		unsigned int(32) component_index;
	} [component_count]
	unsigned int(1) nuc_is_applied;
	unsigned int(7) reserved=0;
	if (version == 0) {
		unsigned int(32) image_width;
		unsigned int(32) image_height;
		for (i=0; i< image_height; i++) {
			for (j=0; j< image_width; j++) {
				double(32) nuc_gain;
			}
		}
		for (i=0; i< image_height; i++) {
			for (j=0; j< image_width; j++) {
				double(32) nuc_offset;
				}
			}
	}

else if (version == 1) {

}

}

#### Semantics

version is an integer that specifies the version of this box. The value shall be set to 1, as the value of 0 has been deprecated.

component_count indicates the number of components to which the non uniformity correction pattern applies. If this value is 0, the non uniformity correction applies to all components of the image.

component_index indicates the index of the component listed in the associated ComponentDefinitionBox.

nuc_is_applied if 1, indicates that the corrections have been applied to the component values in the sample data. If value is 0, the component values are still raw, without NUC corrections applied.

image_width indicates the width in pixels of the NUC tables, shall be equal to image width. This field is not present when version is 1.

image_height indicates the height in pixels of the NUC tables, shall be equal to image height. This field is not present when version is 1.

nuc_gain specifies the Non-Uniform Correction (NUC) gain. The value shall be coded as an IEEE 754 “binary32”. This field is not present when version is 1.

nuc_offset specifies the Non-Uniform Correction (NUC) offset. The value shall be coded as an IEEE 754 “binary32”. This field is not present version is 1.

NOTE 2 In version 0, the nuc_gain and nuc_offset arrays are stored in the item property or sample entry box. As their content will normally be very large, their footprint is not appropriate for an item property or a sample entry box. For this reason, use of version 0 entries is deprecated and the nuc_gain and nuc_offset arrays are stored using auxiliary image items.

### Sensor Bad Pixels Map

#### Definition

Box Type:	'sbpm'
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or more

The SensorBadPixelsMapBox allows identifying bad pixels on a sensor, i.e. pixels for which at least one component value is corrupted.

When this box is present, there shall be an associated ComponentDefinitionBox present.

The box allows entire sensor rows, entire sensor columns, and individual bad pixels to be marked as bad. Pixels that are part of bad rows or bad columns may be listed as part of the list of individually bad pixels, but those pixels are not required to be listed twice.

NOTE1 For example, a sensor system can detect a small region of pixels that show a spurious response where that region intersects with a bad row. The system can report all of the pixels in that region as part of the list of independently bad pixels even if some of these pixels are reported in the bad rows.

NOTE2 If the source image has been padded with rows and/or columns for tile storage, these padded row or columns do not need to be present in the bad pixel map.

Multiple SensorBadPixelsMapBox can be specified if components of the image have different sensor defects. In this case, there shall be at most one SensorBadPixelsMapBox describing one given component of the image.

#### Syntax

aligned(8) class SensorBadPixelsMapBox extends FullBox('sbpm', 0, 0) {
	unsigned int(32) component_count;
	{
		unsigned int(32) component_index;
	} [component_count]
	unsigned int(1) correction_applied;

unsigned int(7) reserved=0;

unsigned int(32) num_bad_rows;

unsigned int(32) num_bad_cols;

unsigned int(32) num_bad_pixels;

for (i=0; i< num_bad_rows; i++) {

unsigned int(32) bad_row;

}

for (i=0; i< num_bad_cols; i++) {

unsigned int(32) bad_column;

}
	for (i=0; i< num_bad_pixels; i++) {
		unsigned int(32) bad_pixel_row;

unsigned int(32) bad_pixel_column;

}
}

#### Semantics

component_count indicates the number of components to which the bad pixel map applies. If this value is 0, the bad pixels described apply to all components of the image.

component_index indicates the 0-based index of the component listed in the associated ComponentDefinitionBox.

correction_applied if 1, indicates that component values for bad pixels in the sample data have been corrected; if 0, indicates that no correction has been applied. The method used to apply correction is out of scope of this document.

num_bad_rows the number of full rows in the image that are bad.

num_bad_columns the number of full columns in the image that are bad.

num_bad_pixels the number of individual bad pixels. This does not include bad pixels that are identified as being part of an entire bad row or bad column.

bad_row a row number for which all pixels are bad. Value 0 corresponds to the top row. The value shall be strictly less than image height.

bad_column a column number for which all pixels are bad. Value 0 corresponds to the left column. The value shall be strictly less than image width.

row the row index of the coordinate pair identifying the location of a bad pixel. Value 0 corresponds to the top row. The value shall be strictly less than image height.

column the column index of a coordinate pair identifying the location of a bad pixel. Value 0 corresponds to the left column. The value shall be strictly less than image width.

### Chroma Location

#### Definition

Box Type:	'cloc' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or associated per item

The ChromaLocationBox may be used to describe the chroma subsampling location method used for frames using YUV components. It shall not be present in the sample entry or associated with the image item for other component formats or if the sampling_type value is 0.

For YUV 4:2:2 or YUV 4:1:1 subsampling, the indicated Chroma420SampleLocType as defined in ISO/IEC 23091-2 shall have VerticalOffsetC equal to 0.

If ChromaLocationBox is not present for frames using a YUV subsampling other than YUV444, the associated Chroma420SampleLocType is 0.

If ChromaLocationBox is used as an item property, it shall be marked as essential.

If ChromaLocationBox is used with a format different from the uncompressed format described in this document, the information in ChromaLocationBox overrides any chroma subsampling location potentially present in the bitstream.

#### Syntax

aligned(8) class ChromaLocationBox extends FullBox('cloc', 0, 0) {
	unsigned int(8) chroma_location;
}

#### Semantics

chroma_location indicates a Chroma420SampleLocType value as defined in ISO/IEC 23091-2, or the value 6. Value 6 is defined, following Chroma420SampleLocType semantics as defined in ISO/IEC 23091-2, with: VerticalOffsetC=0, HorizontalOffsetC=0 for Cr and HorizontalOffsetC=1 for Cb (ex: DV PAL).

### Frame Packing Information

#### Definition

Box Type:	'fpac' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or associated per item

The FramePackingInformationBox can be used to describe how two pictures are packed into a frame for stereoscopic imagery.

If not present, each frame consists in a single picture.

If FramePackingInformationBox is used as an item property, it shall be marked as essential.

#### Syntax

class FramePackingInfoBox extends FullBox('fpac', 0, 0) {
	unsigned int(4) video_frame_packing;
	unsigned int(4) PackedContentInterpretationType
	unsigned int(1) QuincunxSamplingFlag;
	unsigned int(7) reserved = 0;
}

#### Semantics

video_frame_packing, QuincunxSamplingFlag, PackedContentInterpretationType have the same semantics as defined in ISO/IEC 23091-2.

### Disparity Information

#### Definition

Box Type:	'disi' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or associated per item

The DisparityInformationBox can be used to describe how values of a disparity map should be interpreted. If not present, the mapping from disparity value to distance in meters uses the default values (parallax_zero=2N-1, parallax_scale=256, dref=300, wref=100) as defined in ISO/IEC 23002-3.

If this box is used as an item property, it shall be marked as essential.

#### Syntax

class DisparityInformationBox extends FullBox('disi', 0, 0) {
	unsigned int(32) component_count;
	{
		unsigned int(32) component_index;
	} [component_count]
	unsigned int(16) parallax_zero;
	unsigned int(16) parallax_scale;
	unsigned int(16) dref;
	unsigned int(16) wref;
}

#### Semantics

component_count indicates the number of components to which the disparity information applies. If this value is 0, the disparity information applies to all disparity components of the image.

component_index indicates the 0-based index of the component listed in the associated ComponentDefinitionBox.

parallax_zero, parallax_zero, parallax_zero, parallax_zero: have the same semantics as defined in ISO/IEC 23002-3.

### Depth Mapping Information

#### Definition

Box Type:	'depi' 
Container:	Video sample entry, ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per video sample entry or associated per item

The DepthMappingInformationBox may be used to describe how values in a depth map are transformed into distance values. If not present, the mapping from depth value to distance values follow the default values (nknear=128, nkfar=128) as defined in ISO/IEC 23002-3.

If this box is used as an item property, it shall be marked as essential.

#### Syntax

class DepthMappingInformationBox extends FullBox('depi', 0, 0) {
	unsigned int(32) component_count;
	{
		unsigned int(32) component_index;
	} [component_count]
	unsigned int(8) nknear;
	unsigned int(8) nkfar;
}

#### Semantics

component_count indicates the number of components to which the depth mapping information. If this value is 0, the depth mapping information applies to all disparity components of the image.

component_index indicates the 0-based index of the component listed in the associated ComponentDefinitionBox.

nknear, nkfar near and far distances have the same semantics as defined in ISO/IEC 23002-3.

## Sample group descriptions

### Field Interlace Type

#### Definition

The FieldInterlaceType sample group description allows describing the field layout in a sample data in case of interlaced video content. This sample group description is identified by the 'ilce' grouping_type.

If no FieldInterlaceType sample group description is associated to a sample, whether because the sample group description is not present or because the sample is not mapped to any sample group description of this type, the sample data is assumed to be a progressive frame.

#### Syntax

aligned(8) class FieldInterlaceDescription {
	unsigned int(2) interlace_type;
	unsigned int(1) interleaved;
	unsigned int(1) first_field_type;
	unsigned int(1) bottom_first;
	bits(3) reserved=0;
}

class FieldInterlaceType extends VisualSampleGroupEntry ('ilce') {
	FieldInterlaceDescription interlace_description;
}

#### Semantics

interlace_type indicates the interlacing type. The following values are defined:

- 0: progressive frame

- 1: the sample data contains both top and bottom fields. The duration of each field is half the duration of the associated sample.

- 2: the sample data contains a single field. The video sample entry height shall be twice the number of lines in the field. The duration of the field is the duration of the associated sample.

- 3: the sample data contains both top and bottom fields, but the fields are co-incident in time (sample data is a progressive segmented frame).

interleaved if set to 1, indicates that lines of each field are interleaved in the sample data, i.e. first line of first field followed by first line of second field followed by second line of first field etc.). Otherwise (interleaved set to 0), the first line of the second field follows the last line of the first field in the sample data. This value shall be 0 for interlace_type values other than 1 and 3.

first_field_type if set to 1, indicates that the first field appearing in the sample data is the bottom field. If a single field is present in the sample data, then this field is the bottom field. This value shall be 0 if interlace_type value is 0.

bottom_first if set to 1, indicates that the first field to present is the bottom field. This value shall be 0 if interlace_type value is not 1.

## Image Item properties

### Field Interlace Property

#### Definition

Box Type:	'ilcp' 
Container:	ItemPropertyContainerBox
Mandatory:	No
Quantity:	Zero or one per item

The FieldInterlaceProperty allows describing the field layout of an image item in case of interlaced video content. In not present, the image item is assumed to be a progressive frame. If present, the property shall be marked as essential.

NOTE Rendering of images consisting in two non time-coincident fields or in a single field is implementation specific. Derived specification can further constrain usage or rendering of interleaved image items.

#### Syntax

class FieldInterlaceProperty extends ItemProperty ('ilcp') {
	FieldInterlaceDescription interlace_description;
}

#### Semantics

The semantics are identical to the FieldInterlaceType sample group description semantics, using the image item width (resp. height) instead of the video sample entry width (resp. height).

# Multiple tracks and items storage

## Overview

Multiple tracks (resp items) storage allows describing uncompressed video (resp. image)  streams whose components are in multiple tracks (resp. items). This process may be used in various cases:

- simplify editing, by modifying only one component (e.g. alpha or depth) without modifying the other components (colour)

- provide players a way to play a subpart of the components (e.g. colour) instead of failing playback due to unsupported configuration

- enable efficient disk and network usage for partial access (spatial, temporal) of a subset of the components. A typical use case is to store all or a subset of samples for the first component followed by samples of the second component etc. by using ISOBMFF sample and item data layout tools.

- use components with different spatial resolutions

## Component video track group

Video tracks containing components of the same media source may be grouped using a TrackGroupBox with the track_group_type value of 'scvg'. The pair of track_group_id and track_group_type identifies a track group within the file. The tracks that contain a particular TrackGroupTypeBox having the same value of track_group_id and track_group_type belong to the same track group.

Tracks belonging to such a group may have different resolutions and different sample entry types, but shall have:

- The same aspect ratio, as described by the width and height fields of the TrackHeaderBox,

- The same temporal layout, i.e. there shall not be any presentation time for which no sample is defined for one or more tracks of the group while a sample is defined for other tracks of the group.

NOTE The component video track group is mainly intended for storage of uncompressed video with components in different tracks, but can be used for a mix of compressed and uncompressed video (e.g. colour compressed, alpha / depth uncompressed). Derived specification can further restrict the kind of components allowed in 'scvg' track groups, the timescale used, the sample entry format, the visual dimension of the tracks, etc. The display or processing of (part of) a track group with track_group_type value of 'scvg' is application specific.

## Image tiling using ISOBMFF tracks and items

Source image data may be spatially split in different rectangular images, or tiles, each resulting tile being stored in its own track or image item rather than storing the entire image data as a single track or item.

A typical use case is to allow a temporal interleaving of tiles different from what can be achieved with tiling as defined in 5.2.1.4, for which each sample data shall contain all tiles of the associated frame.

Video tracks containing tiles of the same media source may be grouped using a TrackGroupBox with the track_group_type value of 'stvg'.

The pair of track_group_id and track_group_type identifies a track group within the file. The tracks that contain a particular TrackGroupTypeBox having the same value of track_group_id and track_group_type belong to the same track group, i.e. they contain different tiles of the same source image. Tile layouts shall be indicated using the width, height and matrix of the track header. Tracks belonging to the same track group with type 'stvg' shall not spatially overlap each other. There may be uncovered area(s) in the reconstructed frame, for example when a tile track is removed from the file. Tracks belonging to the same track group with type 'stvg' are not required to use the same sample entry type.

For image items, tile layouts can be indicated using the grid derived image item.

NOTE Derived specification can further restrict the combination of tiles within the uncompressed video tracks and multi-track tiling or the presence of holes in the reconstructed frame.

# Labeling of Samples and Items

## High Precision Time Tagging

To support applications requiring high resolution and high accuracy time labeling of media items and track samples, this clause provides a labeling method based on International Atomic Time (TAI). A TAI timestamp is a measurement of a TAI clock represented as a discrete integer number of nanoseconds since the TAI epoch of 1958-01-01T00:00:00.0. Additionally, this clause defines TAI timestamp quality status metadata included with each timestamp. Ideally, to be useful and support a broad range of use cases, an ideal timestamp labeling utility benefits from the following attributes:

- Total ordering: timestamps provide total ordering in which events occur.

- Relative differencing: timestamps support the ability to compute the difference in time between two events, in SI seconds.

- Absolute time: timestamps are in a known, universal absolute time reference, enabling correlation of information from sensors and events in different locations.

TAI timestamps are measurements from a TAI clock. TAI clocks report timestamp values relative to the TAI epoch. They provide the ability to search and discover media content based on universal, real-world time. In situations where an adjustment or correction to a timestamp is necessary to improve the accuracy, such as when synchronization to a remote clock is not available during data collection, the ability to modify values using adjustment information post collection is provided, along with a flag to indicate the inclusion of the adjustment.

TAI clocks are “receptor clocks”, typically receiving synchronization data with one or more remote clocks (e.g., GPS system and PTP Grand Master clocks) which are sources of TAI time. Different types of receptor clocks have different levels of quality when synchronizing with remote clocks; therefore, this clause defines metadata for describing the TAI clock, its capabilities, and its state when sampling a timestamp.

Each TAI timestamp is associated with the beginning of a physical measurement, such as the start of exposure for an imaging sensor. For sensors with variable timing, such as rolling shutters, the TAI timestamp is associated with the first pixel(s) initiating an exposure for a frame. The timing associated with remaining pixel exposures is dependent on the sensor architecture and is outside the scope of the metadata described in this subclause. When extracting data from an image or sample, such as the location of an object in an image, the location data inherits its time from the image frame.

The creation and recording of synthetic data, such as a simulation, is not a measurement of the physical world. The implementation of time labeling, including alignment of synthetic data to real-world data is application dependent and depends on use case needs. In these cases, labeling of the media as synthetic is outside the scope of metadata described in this subclause.

For media captured in real-time, valid TAI timestamps for the media monotonically increase; however, there are situations where the monotonic nature does not always hold, such as when losing remote to receptor clock synchronization and a resulting discontinuity upon resync occurs. Synchronization status metadata is available to indicate when a receptor clock is not in a synchronous state with the remote clock.

TAI is different from Coordinated Universal Time (UTC) because UTC includes irregularly inserted discontinuities called leap seconds. When converting TAI time to UTC, applications convert the TAI timestamp to date text form and subtract the correct leap seconds value for the date text:

UTC date-text = TAI date-text - leap seconds (based on value at time of measurement)

When converting from timestamps derived from UTC, such as Network Time Protocol (NTP), reverse the computation. To perform a conversion from UTC time to a TAI timestamp, transform the UTC time to a UTC nanosecond timestamp relative to the TAI epoch, then adjust for leap seconds:

TAI timestamp = UTC timestamp + leap seconds (based on value at time of measurement)

A caveat of using UTC-based systems for capturing timestamps is the leap-second adjustment methods employed by various implementations. In practice these methods include stopping the remote clock, slewing the clock at a slow rate, etc. which may introduce errors and compromise the ability to generate accurate time differences on occasions when leap seconds occur. For these reasons, this standard recommends sourcing time from a clock which does not adjust for leap seconds, i.e., TAI based clocks.

Table 6 provides equations to convert times from common time sources (i.e., GPS, PTP, and NTP) to TAI time. The first column of Table 6 is the name of the time source; the second column is the epoch of the time source and whether the epoch is defined in the TAI or UTC time system (for informational use only); the third column provides the equation to convert the time from the time source’s epoch offset to TAI’s epoch offset. The first step, before applying the equations in the table, is to convert the source time to nanoseconds. The GPS and PTP time standards measure time without using leap seconds, therefore their conversions are straight forward offset additions (based on their respective epoch). Vendors may implement receptor clocks with an epoch other than the base standard; as a result, writers must account for use of the vendor’s implemented epoch when performing conversions between time systems.

The NTP standard[4] measures time in UTC, with leap second offsets, so the NTP equation removes leap seconds, where the leap second value comes from a leap second table lookup. The GPS epoch is based on a specific UTC time, but GPS time does not implement leap seconds and tracks with TAI. The POSIX Time standard uses an epoch of Jan 1, 1970 (UTC), which is offset from Jan 1, 1970 (TAI) by 8.000082 seconds. The 82 microseconds are due to non-integer leap second computations used before 1972. As a result, the conversion factor for a clock using the POSIX epoch and tracking with UTC time is 378,691,208,000,082,000 nanoseconds plus the current UTC leap seconds*1,000,000,000. The conversion factor for a clock using the POSIX epoch and tracking with TAI time is 378,691,208,000,082,000 nanoseconds.

Table 6 - Timestamp conversion factors

| Timing Standard | Epoch | Conversion to TAI timestamp (nanoseconds) |
| --- | --- | --- |
| International Atomic Time (TAI) | 1958-01-01T00:00:00.0 (TAI) | N/A |
| Global Positioning System (GPS) | 1980-01-06T00:00:00.0 (UTC) | TAI timestamp = GPS timestamp + 694,656,019,000,000,000 |
| Precision Time Protocol (PTP) | 1970-01-01T00:00:00.0 (TAI) | TAI timestamp = PTP timestamp + 378,691,200,000,000,000 |
| Network Time Protocol (NTP) a | 1900-01-01T00:00:00.0 (UTC) | TAI timestamp = NTP timestamp – 1,830,297,600,000,000,000 + UTC leap seconds*1,000,000,000 UTC leap seconds comes from table lookup b using the NTP timestamp |
| a Use of clock sources with time derived from UTC may introduce errors due to implementation of leap second insertion. b The current value for leap seconds can be found on the National Institute of Standards and Technology web site at https://www.nist.gov/pml/time-and-frequency-division/time-realization/leap-seconds |  |  |

When writing a TAI timestamp to annotate an individual piece of content, the timestamp provides a record of the measurement time of the sample or item and is intended to remain with the sample or item. Subsequent media processing, like editing to change the order of the media samples, may alter the overall monotonic nature of the TAI timestamps within the structure of a track.

Two data structures support TAI timestamps:

- TAIClockInfoBox: describes the TAI clock generating timestamps.

- TAITimestampPacket: contains a timestamp and quality status for the timestamp.

NOTE	The tools defined in Clause 8 can apply to both uncompressed and compressed video samples and image items, as well as items and timed metadata samples. Derived specifications can allow usage of these boxes.

### TAI clock information box

#### Definition

Box Type: 'taic'

Container: SampleEntry, ItemPropertyContainerBox

Mandatory: Yes, when TAI_timestamp values are associated with items or samples

Quantity: One for a given item when associated with a TAI_timestamp; one in a sample entry of a track when carrying TAI_timestamp values as sample auxiliary information.

NOTE	Writers can share and associate a TAIClockInfoBox with multiple items when the source clock for the TAI_timestamp values is the same.

The TAIClockInfoBox provides metadata about a specific TAI clock referenced by one or more TAITimestampPackets. This metadata includes information about the type, quality, and status of the TAI clock. This enables readers to accurately correlate information from multiple cameras or other sensors, for example temporally aligning frames between two disparate cameras with different clock sources. For systems and situations where a TAI clock does not synchronize with a remote clock, the TAIClockInfoBox includes a clock type value to indicate this status condition. When a receptor clock is in synchronization with the remote clock the state of the receptor clock is “synchronized”. When a remote clock is unavailable the state of the receptor clock is “unsynchronized”.

When a TAI_timestamp value applies to a declared media item, the TAIClockInfoBox is a descriptive item property. When the TAI_timestamp values apply to track samples, the writer carries the TAI_timestamp values as sample auxiliary information and includes a TAIClockInfoBox in the SampleEntry. The TAIClockInfoBox shall be present in the sample entry when samples are associated with a TAI_timestamp, carried as sample auxiliary information and shall be present as an item property when a TAI_timestamp is associated with an item.

The syntax below for the TAIClockInfoBox is for a sample entry container and extends FullBox. When used in an ItemPropertyContainerBox, the same syntax applies but the defined box extends ItemFullProperty and is a descriptive property associated with items having a corresponding TAI_timestamp.

#### Syntax

aligned(8) class TAIClockInfoBox extends FullBox('taic', 0, 0) {{
   unsigned int(64) time_uncertainty;
   unsigned int(32) clock_resolution;
   signed int(32) clock_drift_rate;
   unsigned int(2) clock_type;
   unsigned int(6) reserved = 0;
}

#### Semantics

time_uncertainty is the standard deviation measurement uncertainty for the timestamp generation process, expressed as an unsigned integer number of nanoseconds. Calibration testing of a device determines the time_uncertainty value relative to the true TAI time of a measuring event. When the time_uncertainty is unknown, writers shall set it to the maximum value (0xFFFF FFFF FFFF FFFF).

clock_resolution specifies the resolution of the receptor clock in nanoseconds. For example, a microsecond clock has a clock_resolution of 1000, and a ten-millisecond resolution clock has a clock_resolution of 10,000,000. Receptor clocks with resolution finer than 1 nanosecond set their clock_resolution to zero. While the clock_resolution can differ, TAI_timestamp values are always in the 64-bit nanosecond form.

NOTE 1	Both the time_uncertainty and clock_resolution affect the statistical accuracy of the TAI_timestamp.

clock_drift_rate represents the rate of divergence of a receptor clock and its source of time when synchronization stops. For example, GPS receptor clock losing communication with remote satellite clock. When a system loses synchronization, the clock_drift_rate provides status information to help determine the level of increasing uncertainty for the TAI_timestamp measurements over time. The clock_drift_rate is in picoseconds per second and comes from either a clock’s specification sheet or laboratory testing and measurement. A positive value describes a clock drifting faster than the true time and a negative value describes a clock drifting slower than the true time. When the clock_drift_rate is unknown, writers shall set it to the maximum value (0x7FFF FFFF).

clock_type is an enumeration value indicating the synchronization capability of the clock generating the timestamps. The enumeration values are:

- Clock type is unknown.

- The clock does not synchronize to an atomic source of absolute TAI time (e.g. unsynchronized CPU clock).

- The clock can synchronize to an atomic source of absolute TAI time (e.g., synchronized GPS timing card)

- Reserved – DO NOT USE

NOTE 2	When writers generate TAI Timestamps for simulated media, clock information parameters are set to desired values based on simulation needs.

### TAI timestamp packet

#### Definition

The TAITimestampPacket supports the carriage of a TAI_timestamp value along with status information. The TAI_timestamp provides temporal context and attaches to samples and items within a file.

When writing the TAI timestamp packet, the nominal state is synchronization of the receptor and remote clocks. However, there are two other states the TAI timestamp packet can indicate during timestamp measurement: synchronization failure and timestamp generation failure.

Synchronization failure means the receptor clock is operating correctly but has lost synchronization with the remote clock, therefore the receptor clock may be measuring timestamps which are drifting away from the actual TAI time. The TAIClockInfoBox provides drift rate estimates to aid in adjusting the time by the reader (if desired).

The timestamp generation failure means the receptor clock is not reporting a measurement, the time value is unknown, and the TAITimestampPacket is written with a “best” estimate of the true TAI timestamp. At a minimum, to achieve correct ordering of successive values, writers shall produce a TAI_timestamp which is one nanosecond greater than the last TAITimestampPacket’sTAI_timestamp.

To attach TAI_timestamp values to track samples, writers include TAITimestampPackets as sample auxiliary information. To attach TAI_timestamp values to items, writers implement them as a descriptive item property and associate with the item.

#### Syntax

aligned(8) class TAITimestampPacket {
   unsigned int(64) TAI_timestamp;
   unsigned int(1) synchronization_state;
   unsigned int(1) timestamp_generation_failure;
   unsigned int(1) timestamp_is_modified;
   unsigned int(5) reserved = 0;
}

#### Semantics

TAI_timestamp is a 64-bit unsigned integer representing the number of nanoseconds since the TAI epoch of 1958-01-01T00:00:00.0. Each nanosecond is one billionth of a Standard International (SI) Second. The TAIClockInfoBox includes the uncertainty and resolution of the clock generating each TAI_timestamp.

synchronization_state indicates the remote and receptor TAI clocks are in synchronization when creating the TAI timestamp. When measuring the TAI_timestamp, if the receptor clock and a remote TAI clock are not in a state of synchronization, the synchronization_state shall be set to 0. When measuring the TAI_timestamp, if the receptor clock and a remote TAI clock are in a state of synchronization, the synchronization_state shall be set to 1. When the companion TAIClockInfoBox’s  clock_type value equals 0 or 1, the synchronization_state shall equal 0.

timestamp_generation_failure When a TAI receptor clock does not report a TAI_timestamp, the writer sets the timestamp_generation_failure bit to 1. On each successive data sample requiring the creation of a TAI_timestamp estimate, the writer shall generate a value of at least one nanosecond greater than the last prior TAI_timestamp written, providing for consistent total ordering of measured values associated with the given clock. When the timestamp generation is nominal (even if the clock is unsynchronized), the timestamp_generation_failure shall be set to 0.

timestamp_is_modified When modifying an existing TAI_timestamp (e.g., modification to better align its value to the actual measurement time), the timestamp_is_modified parameter signals this condition. When a writer modifies the TAI_timestamp (e.g., correction applied after initial recording) the timestamp_is_modified shall be set to 1. When the TAI_timestamp is the original recorded clock time, even if estimated when the timestamp_generation_failure is set to 1, the timestamp_is_modified shall be set to 0.

### Sample TAI timestamps

#### Definition

Aux Info Type: 'stai'

Container: Sample auxiliary information

Mandatory: No

Quantity: Zero, or one per sample

A timestamp_packet is a sample auxiliary information payload for each sample in a track. A writer stores the companion TAIClockInfoBox in the sample entry associated with the sample timestamps.

#### Syntax

TAITimestampPacket timestamp_packet;

#### Semantics

timestamp_packet is an instance of the TAITimestampPacket class, see subclause 8.1.2.

aux_info_type (‘saiz’ and ‘saio’ box parameter) This parameter is set to ‘stai’, which indicates sample TAI_timestamp values.

aux_info_type_parameter (‘saiz’ and ‘saio’ box parameter) This parameter is currently unused for the aux_info_type of ‘stai’. The aux_info_type_parameter is a reserved value, and all 32-bits of the unsigned word shall be set to 0.

Writers set the remaining ‘saiz’ and ‘saio’ box parameters as per ISO/IEC 14496-12 box specification.

### Item TAI timestamps

#### Definition

Box Type: 'itai'

Property Type: Descriptive item property

Container: ItemPropertyContainerBox

Mandatory: No

Quantity: Zero, or one per item

The TAITimestampBox enables associating a TAI_timestamp to a declared item. This box may be present when deriving the item contents from a measured process, such as an image item or a metadata item containing measured sensor data. This box has a required companion TAIClockInfoBox property containing TAI clock information. A TAI_timestamp and TAIClockInfoBox are two separate properties so writers can generate clock information (TAIClockInfoBox ) once and associated to multiple items with each having their own TAI_timestamp, when the clock information is the same.

Derived image items are comprised of one or more image items, which are inputs to the derivation. In cases where all input items share the same clock source and measured TAI_timestamp, the derived item may be associated with the common ‘taic’ and ‘itai’ properties. In cases where the input items contain different measurement times or different clock sources, the derived item shall not be associated with a TAITimestampBox or TAIClockInfoBox and the TAI timing information, if present, shall be associated with the individual input items of the derivation.

#### Syntax

aligned(8) class TAITimestampBox extends ItemFullProperty('itai', 0, 0) {
   TAITimestampPacket timestamp_packet;
}

#### Semantics

timestamp_packet is an instance of the TAITimestampPacket class, which is described in subclause 8.1.2.

# Generic compression of items and sample data

## Overview

Storing uncompressed item and sample data is required in many use cases, some of which are not well-supported by typical image compression algorithms (e.g. floating point or very high-bit depth imagery). Other types of sample data, such as KLV (SMPTE 336M-2007[5]), are only compressible by standard generic data compression mechanisms. The ability to compress data for any media type without compression loss is desirable for reducing storage sizes and transmission times, such as in the following scenarios.

- A data producer generates a large image (or image sequence) using 32-bit IEEE 754 binary floating point component values after calibration. The image is tiled using 1024 × 1024 tiles and each tile is independently compressed using deflate.

- A data consumer desires to load only a specific spatial region from an image or sample based on some form of chunking of the image (chunk by rectangular tiles or chunk by rows). Consumer uses offset/size information provided within the file to locate only the desired chunk(s). Each chunk is independently decompressed.

- A data consumer desires to load only a spatial region from a large tiled image, where the desired region is smaller than a region contained within a single compressed chunk. After decompressing the chunk, the order, alignment and padding of the component data is maintained, enabling the consumer to locate individual component values via calculated offsets.

- A data producer collects an image of the Earth scanning diagonally from northwest to southeast. For simpler human viewing of the image, the collected image is rotated 45 degrees to align north to up, and additional padding is added to form a tiled, rectangular image buffer. Since the four corner tiles are entirely fill data, they can more efficiently be stored than simply compressing multiple blocks of fill data.

- A data producer generates multiple blocks of metadata, encoded using Key-Length-Value encoding, and stores them as individual items. The value portion of each KLV metadata item is a set or pack containing binary metadata and is generically compressed to save space.

To decompress a complete media sample or item, the file reader locates the data as given by the sample or item’s compressed units description. The extracted compressed data is then decompressed according to the compression algorithm specified by the compression_type field in the CompressionConfigurationBox. The resultant data is formatted exactly as was specified by the underlying media format, including any padding placed at the end of the element to align the next element.

If the value of the compressed_unit_type field in the CompressionConfigurationBox is not 0, decompression of individual portions of the sample is possible. In those cases, the individual ranges specified in the GenericallyCompressedUnitsInfoBox map to the individual units as specified by the compressed_unit_type field, in the order those units would have been found in the coded data were that coded data left uncompressed as specified in Clause 5.

For media tracks, generic compression is signalled using a restricted transformation scheme as specified in subclause 9.3, and can be used in conjunction with other track transformation such as encryption or other restricted video transformation, as specified in ISOBMFF.

For items, generic compression is signalled using an essential property as specified in subclause 9.4 and can be used in conjunction with item protection as defined in ISOBMFF.

## Compression Configuration Box

### Definition

Box Type: 'cmpC'
Container: SchemeInformationBox or ItemPropertyContainerBox
Mandatory: Yes (when the SchemeType is 'gcmp')	
Quantity: One

The CompressionConfigurationBox specifies the specific data compression method used and codec-specific type of compressed units within a media sample or item data.

This box can be:

- added to a restricted video sample entry for media tracks;

- added as properties associated with an item.

The syntax in this section is given for a video sample entry container and the defined box therefore extends FullBox. When used in an ItemPropertyContainerBox, the same syntax applies but the defined boxes extends ItemFullProperty. The properties defined in the following sections are descriptive properties.

The definition of each value of compression_type specifies not only the algorithm but also the bitstream format for each compressed subsample. For example, ‘zlib’ specifies the use of the deflate algorithm as packaged in the zlib format defined by IETF RFC 1950.

Value 0 for compressed_unit_type indicates that the compressed range is always the complete media sample or item.

Derived specifications may assign compressed_unit_type values other than 0 according to the specificities of the underlying media format.

### Syntax

aligned(8) class CompressionConfigurationBox extends FullBox('cmpC', version=0, flags=0) {

unsigned int(32) compression_type;

unsigned int(8) compressed_unit_type;

}

### Semantics

compression_type is a 4CC indicating the compression mode for the sample or item. Values shall be in accordance with Table 7.

Table 7 – Compression types

| Value | Description |
| --- | --- |
| ‘defl’ | DEFLATE algorithm as defined in IETF RFC 1951 |
| ‘zlib’ | DEFLATE algorithm as packaged in the format defined by IETF RFC 1950 |
| ‘brot’ | Brotli algorithm as defined in IETF RFC 7932 |
| ‘iden’ | Identity (no compression) |

compressed_unit_type indicates the unit being compressed within a generically-compressed media sample or item. The range of bytes specified for each unit contains exactly the result of the compression algorithm applied to the identified unit.

A value of 0 means that compression is applied to the entire sample or item. Other values of the compressed_unit_type are specified in subclause 10.1 Table 8. For example, if the value of this field is 2 (units are tiles), then each tile of the item is independently accessible and decompressible.

NOTE	Depending on the compression algorithm and codestream format, a reader might be able to decompress multiple units as a single combined input buffer. For example, while the IETF RFC 1950 format for DEFLATE does include a short header at the start of each compressed unit, the decompressor does recognize that header even if it is in the middle of the compressed data buffer. In many cases, it might possible for units to be concatenated in any order and input to a single instance of the decompressor, as long as the reader knows what order the units were found in the concatenated compressed buffer and can then extract those units from the decompressed buffer. However, this capability is not guaranteed by this document.

## Generically-compressed media tracks

### Overview

Generically-compressed media tracks compliant to this specification are media tracks compliant to ISO/IEC 14496-12 that use a reserved transformation scheme of type 'gcmp', hereafter called generically-compressed sample entry. The scheme version for the 'gcmp' scheme shall be set to 1.

The SchemeInformationBox of a generically-compressed sample entry shall contain one CompressionConfigurationBox, specifying the data compression method used and the partitioning of the compression scheme over the sample data.

The GenericallyCompressedUnitsInfoBox is used to specify the location and size of each independently compressed element within the sample.

For non-fragmented movies, the sample table of a track using a generically-compressed sample entry shall contain one GenericallyCompressedUnitsInfoBox. For a fragmented movie, each TrackFragmentBox of a track using a generically-compressed sample entry shall contain one GenericallyCompressedUnitsInfoBox.

A generically-compressed sample entry does not impact the description of the media samples of the track: random access information, subsample description and sample group properties describe the media sample without the generic compression being applied.

If content encryption is applied, it shall be applied on the compressed sample. If the protection scheme uses a track transformation (e.g. Common Encryption), the sample entry shall describe these two transformations as specified in ISOBMFF.

### Generically Compressed Units Info Box

#### Definition

Box Type: 'cbri'
Container: SampleTableBox or TrackFragmentBox
Mandatory: See below	
Quantity: Zero or One

The GenericallyCompressedUnitsInfoBox describes the units of compressed data for samples of a track or track fragment.

The GenericallyCompressedUnitsInfoBox shall be present whenever the track has a sample entry using a restricted scheme of type 'gcmp'and the compressed_unit_type of the associated CompressionConfigurationBox has a value other than 0.

The sample size, as documented by the SampleSizeBox or TrackRunBox, shall be the size of the compressed material described by the GenericallyCompressedUnitsInfoBox, i.e. shall be equal to MAX(unit_offset + unit_size) of any compressed unit in the sample. If no compressed unit is present for a sample, the entire sample payload is the compressed unit, i.e. an implicit unit is defined with unit_size equal to the sample size and unit_offset equal to 0.

Each compression unit of a sample shall be compliant to the compression type indicated by the CompressionConfigurationBox of the sample entry this sample is mapped to.

All samples in the track or track fragment shall be documented in the GenericallyCompressedUnitsInfoBox. Within a sample, it is possible to mutualize identically compressed data by using the same entity offset and size as a previously described entity.

NOTE	For a given sample, the compressed unit size/offset pairs are listed in the GenericallyCompressedUnitsInfoBox in the order specified for that unit type (as specified by the compression_unit_type field in the CompressionConfigurationBox). For example, when units correspond to tiles, the unit (i.e., tile) size/offset pairs are listed in raster-scan order as specified in subclause 5.2.1.4. However, the actual data for those units within the sample data can be in any order. Derived specifications can further constrain this, for example to require that the compressed data for each compressed unit be in the same order as the units.

#### Syntax

aligned class compressed_unit_info(unit_offset_nbbits, unit_size_nbbits) {
   if (unit_offset_nbbits)
      unsigned int(unit_offset_nbbits) unit_offset;
   unsigned int(unit_size_nbbits) unit_size;
}
 
 
class GenericallyCompressedUnitsInfoBox extends FullBox('cbri', version=0, flags=0) {
   unsigned int(3) unit_offset_code;
   unsigned int(3) unit_size_code;
   bit(2) reserved = 0;
   unsigned int(32) num_samples;
 
   int unit_offset_nbbits;
   if (unit_offset_code==0)      unit_offset_nbbits=0;
   else if (unit_offset_code==1) unit_offset_nbbits=16;
   else if (unit_offset_code==2) unit_offset_nbbits=24;
   else if (unit_offset_code==3) unit_offset_nbbits=32;
   else if (unit_offset_code==4) unit_offset_nbbits=64;
 
   int unit_size_nbbits;
   if (unit_size_code==0)      unit_size_nbbits=8;
   else if (unit_size_code==1) unit_size_nbbits=16;
   else if (unit_size_code==2) unit_size_nbbits=24;
   else if (unit_size_code==3) unit_size_nbbits=32;
   else if (unit_size_code==4) unit_size_nbbits=64;
 
   for (int i = 0; i < num_samples; i++) {
      unsigned int(32) num_compressed_units;
      for (int r = 0; r < num_compressed_units; r++) {
         compressed_unit_info(
            unit_offset_nbbits,
            unit_size_nbbits
         );
      }
   }
}

#### Semantics

unit_offset_code indicates the number of bits used to describe unit offsets. A value of 0 implies that offsets are not used.

unit_size_code indicates the number of bits used to describe unit sizes.

num_samples is an integer indicating the number of samples for the current entry. This value shall not be 0.

num_compressed_units is an integer indicating the number of compressed units for samples described by the current entry. If this value is 0, this implies that a single unit covering the entire sample data is used.

NOTE	If the structure of the sample varies within the track or track segment (e.g., the dimensions or size of the sample changes), then the number of compressed units might also change depending on how elements within the sample are mapped to units.

compressed_unit_info.unit_offset is the offset in bytes to the first byte of the compressed unit from the start of the sample data as stored in the file. If unit_offset_nbits is 0, this field is not coded and takes the value:

- 0 for the first compressed unit of the sample

- Sum of all compressed_unit_info.unit_size values of all the preceding compressed units of the sample otherwise

compressed_unit_info.unit_size is size in bytes of this compressed unit. This value shall not be 0

## Generically-compressed items

### Overview

Generically-compressed items have an associated essential item property of type 'cmpC', which shall be marked as essential. When content protection is applied to a generically-compressed item, the protection shall be applied to the compressed payload. Generically-compressed items can be used as can be any other items without restriction, including as part of an image grid item.

### Generically Compressed Units Item

#### Definition

Box Type: 'icef'
Container: ItemPropertyContainerBox
Mandatory: No	
Quantity: Zero or One

The GenericallyCompressedUnitsItemInfoBox describes the units of compressed data for an item.

The GenericallyCompressedUnitsItemInfoBox shall be present for any item with an associated essential property of type 'cmpC' (CompressionConfigurationBox) for which compressed_unit_type has a value other than 0. When not present, this implies that a single unit covering the complete item data is used.

The item size in bytes is the size of the compressed material described by the GenericallyCompressedUnitsItemInfoBox, i.e. shall be equal to MAX(unit_offset + unit_size)of any compressed unit in the item. This size shall be equal to the sum of all extent_length of the extents describing this item in ItemLocationBox.

Each compressed unit of an item shall be compliant to the compression type indicated by the CompressionConfigurationBox associated to this item. The bytes for a compressed unit shall be contained in a single item extent. One item extent may however contain more than one compressed unit.

NOTE 1	A compressed unit of an item can reuse parts of the item data by pointing to the same data using the same unit_offset. Similarly, an item can reuse parts of another item using the extent construction mechanisms.

NOTE 2	For a given item, the compressed unit size/offset pairs are listed in the GenericallyCompressedUnitsItemInfoBox in the order specified for that unit type (as specified by the compression_unit_type field in the CompressionConfigurationBox). For example, when units correspond to tiles, the unit (i.e. tile) size/offset pairs are listed in raster-scan order as specified in subclause 5.2.1.4. However, the actual data for those units within the item data can be in any order. Derived specifications can further constrain this, for example to require that the compressed data for each compressed unit be in the same order as the units.

If this property is not associated with an item using generic compression, the entire item is the compressed unit.

#### Syntax

The dependent class compressed_unit_info is as defined in subclause 9.3.2.

aligned(8) class GenericallyCompressedUnitsItemInfoBox extends
ItemFullProperty('icef', version=0, flags=0) {
   unsigned int(3) unit_offset_code;
   unsigned int(3) unit_size_code;
   bit(2) reserved = 0;
 
   int unit_offset_nbbits;
   if (unit_offset_code==0)      unit_offset_nbbits=0;
   else if (unit_offset_code==1) unit_offset_nbbits=16;
   else if (unit_offset_code==2) unit_offset_nbbits=24;
   else if (unit_offset_code==3) unit_offset_nbbits=32;
   else if (unit_offset_code==4) unit_offset_nbbits=64;
 
   int unit_size_nbbits;
   if (unit_size_code==0)      unit_size_nbbits=8;
   else if (unit_size_code==1) unit_size_nbbits=16;
   else if (unit_size_code==2) unit_size_nbbits=24;
   else if (unit_size_code==3) unit_size_nbbits=32;
   else if (unit_size_code==4) unit_size_nbbits=64;
 
   unsigned int(32) num_compressed_units;
   for (int r = 0; r < num_compressed_units; r++) {
      compressed_unit_info(unit_offset_nbbits, unit_size_nbbits);
   }
}

#### Semantics

unit_offset_code indicates the number of bits used to describe unit offsets. A value of 0 implies that offsets are not used.

unit_size_code indicates the number of bits used to describe unit sizes.

num_compressed_units is an integer that indicates the number of compressed units for the item. This value shall not be 0.

compressed_unit_info.unit_offset is the offset in bytes to the first byte of the compressed unit, with value 0 being the first byte of the item payload. If unit_offset_nbits is 0, this field is not coded and takes the value:

- 0 for the first compressed unit of the item

- Sum of all unit_size values of the preceding compressed units otherwise.

compressed_unit_info.unit_size is size in bytes of this compressed unit. This value shall not be 0.

## Hypothetical reconstruction model

The reconstruction by a file reader of an entire generically compressed item or sample shall be equivalent to the result of the following process:

- Identify if generic compression is used:

  - presence of CompressionConfigurationBox associated property for an item;

  - presence of a ‘resX’ sample entry transformation with a scheme type ‘gcmp’, implying a CompressionConfigurationBox in the SchemeInformationBox.

- Identify the compression type and unit type in the CompressionConfigurationBox.

- Extract the item or sample payload as for any uncompressed item or payload, using extents for items and sample size and offsets for media tracks.

- If encryption applies to the sample or item, decrypt the entire sample or item payload as defined by the encryption scheme.

- Locate the GenericallyCompressedUnitsItemInfoBox for items, or the entry for the given sample in the GenericallyCompressedUnitsInfoBox for media tracks.

- Prepare an empty decompression buffer for the item/sample.

- For each compressed unit, in the order they are listed:

  - extract the bytes from the item or sample payload using the unit_offset and unit_size of the compressed unit;

  - decompress these bytes using the identified decompressor;

  - concatenate these bytes into the decompression buffer.

- Replace the item/sample payload with the decompression buffer and update the item/sample size as seen by the application.

Implementations can, of course, find better ways to optimize this process, especially if accessing only a subset of the compressed units is desired (e.g. individually compressed tiles), as indicated by the CompressionConfigurationBox. In such cases, an implementation can decide to (decrypt and) decompress only the bytes required for its needs.

# Mapping to other ISOBMFF capabilities

## Mapping of generic sample compression to uncompressed image and sample data

For uncompressed video sample entries or items compliant to this document, values of the compressed_unit_type field for the CompressionConfigurationBox shall be in accordance with Table 8.

Table 8 – Compressed unit types

| Value |  | Description |  | Restrictions and relationship to other fields |
| --- | --- | --- | --- | --- |
| Types applicable to all items and samples |  |  |
| 0 | the unit shall be the full item or sample as defined in subclause 9.2.3 | num_compressed_units shall be 1 |
| Image-related types |  |  |
| 1 |  | the unit shall be the full image for a given component (component-based interleave and mixed interleave) |  | num_compressed_units shall be equal to component_count in both the ComponentDefinitionBox and the UncompressedFrameConfiguration box |
| 2 |  | the unit shall be a single tile |  | If interleave_type is 4 (tile-component interleaving), then a compressed unit corresponds to one tile-component and num_compressed_units shall be equal to (num_tile_rows_minus_one + 1) × (num_tile_cols_minus_one + 1) × component_count ADD ELSE CLAUSE |
| 3 |  | the unit shall be a single row |  |  |
| 4 |  | the unit shall be a single pixel |  |  |
| other values |  | ISO/IEC reserved for future definition |  |  |
| Other types |  |  |
| other values |  | ISO/IEC reserved for future definition |  |  |

Unit ranges are specified in the same order as the units themselves. For example, the unit ranges for image tiles are in the same order as the tiles as specified in Clause 5.

NOTE	Compressed unit types are potentially applicable to other types of data besides items and samples and thus specified independently from the values for the compressed_unit_type field in the CompressionConfigurationBox in subclause 9.2.

## Combined usage of Generic Sample Compression and Common Encryption

When a media sample or item is encrypted using ISO/IEC 23001-7 Common Encryption, the encryption boundaries, i.e. the start of a CENC subsample, should be aligned with significant semantic elements of the item, e.g. start of a tile, a row, a component plane, etc. Given the uncompressed nature of the media, it is expected that most CENC subsamples will have a BytesOfClearData with value 0. Derived specifications may further restrict the mapping of these elements to CENC subsamples.

When a media sample or item is using both generic sample compression and encryption using ISO/IEC 23001-7 Common Encryption, the following apply.

- The encryption shall be applied to the generic compressed data.

- The first byte of each compressed unit shall be the first byte of a CENC subsample which shall have no decryption dependency to previous CENC subsamples. This implies that a full crypt block starts on this first byte, and that either CTR block chaining is used or that the cypher state is re-initialized at the start of the subsample (e.g. ‘cbcs’ scheme).

A media track using both generic compression and common encryption will therefore have two sample entry transformations, first for encryption then for generic compression. For a video track, this can be summarized as:

| - Sample entry type ‘encv’ |  |  |
| --- | --- | --- |
| - ProtectionSchemeInfoBox |  |  |
|  |  | - SchemeType ‘cenc’, ‘cbcs’ … |
|  |  | - SchemeInfoBox→TrackEncryptionBox (CENC specific ) |
|  |  | - OriginalFormatBox indicating ‘gcmp’ |
|  | - RestrictedSchemeInfoBox |  |
|  |  | - SchemeType ‘gcmp’ |
|  |  | - SchemeInfoBox→CompressionConfigurationBox |
|  |  | - OriginalFormatBox indicating original media format (‘uncv’ for example) |

# Generic compression brands

## Brand for generic compression of media tracks

The brand 'gcmm' shall be used to indicate compatibility with generically compressed media samples and items. The 'gcmm' brand requires support for the 'iso6' brand.

Processing of restricted sample entries (i.e. 'resv') with a scheme type of 'gcmp' is required under this brand.

Additionally, the 'gcmm' brand requires support for:

- CompressionConfigurationBox with version = 0 and flags = 0

- GenericallyCompressedUnitsInfoBox with version = 0 and flags = 0

- The compression types 'defl' and 'iden'

## Brand for generic compression of items

The brand 'gcmi' shall be used to indicate compatibility with generically compressed media samples and items. The 'gcmi' brand requires support for the 'isoa' brand.

Additionally, the 'gcmi' brand requires support for:

- CompressionConfigurationBox with version = 0 and flags = 0

- GenericallyCompressedUnitsItemInfoBox with version = 0 and flags = 0

- The compression types 'defl' and 'iden'

# Derivations

## Introduction

Text to be added

### Sensor non-uniformity correction derivation for image items

### Sensor non-uniformity correction derivation

#### Definition

An item with an item_type value of ‘dnci’ defines a derived image item whose reconstructed image includes a non-uniformity correction (NUC) of raw component value data and is formed from one base input image (e.g., a raw, uncorrected infrared image item), one nuc_gain table auxiliary input image, and one nuc_offset auxiliary input image. The base input image is uncompressed and contains an image formed directly from a sensor without calibration correction, or other manipulation, for addressing both gain and offset non-uniformity across all sensor elements that produce component values in a raw image. The nuc_gain table auxiliary input image carries the gain table and the nuc_offset table auxiliary input image carries the offset table for applying a non-uniformity correction for the base input image. An aux_type (urn:mpeg:uncompressed:2024:auxid:001) is assigned to the nuc_gain table auxiliary item via an ‘auxC’ AuxiliaryTypeProperty and an aux_type (urn:mpeg:uncompressed:2024:auxid:002) is assigned to the nuc_offset table auxiliary item via an ‘auxC’ AuxiliaryTypeProperty.

The input images are identified using the SingleItemTypeReferenceBox/ SingleItemTypeReferenceBoxLarge.  They have the type 'dimg' for this derived image item within the ItemReferenceBox. In the SingleItemTypeReferenceBox/ SingleItemTypeReferenceBoxLarge of type 'dimg', the value of from_item_ID identifies the derived image item of type 'nuci', the value of reference_count shall be equal to 3, and the values of to_item_ID identify the input images. The first input image shall be the base input image, the second input image shall be the nuc_gain table input image, and the third input image shall be the nuc_offset table input image. The nuc_gain table input image and the nuc_offset table input image shall both be uncompressed and shall have the same dimensions as the base input image, which is also uncompressed. The nuc_gain table and the nuc_offset table shall have values coded as an IEEE 754 “binary32”.

To meet the requirements for auxiliary image items, an item reference of type 'auxl' is assigned from the nuc_gain table auxiliary image item to the 'nuci' derived image item and from the nuc_offset table auxiliary image item to the 'nuci' derived image item.

Construction of the derived output image is achieved by applying gain and offset corrections from the nuc_gain table and the nuc_offset table to the base image.

Given the input parameters:

- Let InputComponentValue(x, y) be the component value intensity from a sample of a raw uncompressed source image at spatial coordinates (x, y).

- Let nuc_gain(x, y) be the component value gain factor retrieved from a nuc_gain table auxiliary image item at spatial coordinate (x,y).

- Let nuc_offset(x, y) be the component value offset factor retrieved from a nuc_offset table auxiliary image item.

Generate the derived output result:

- The derived output component values shall match the component value type (unsigned integer, etc.) and bit depth of the base input image.

- Let OutputComponentValue(x, y) be the component value intensity from the derived image with non-uniformity corrections applied via the following mathematical transformation:

For each component in an image labeled for non-uniformity correction in the derivation, the correction equation, at component value location (x,y) is:

OutputComponentValue(x, y) = nuc_gain(x,y) * InputComponentValue(x, y) + nuc_offset(x,y)

where x and y provide the (columns, rows) component value location.

For an unsigned integer component value, the minimum value of OutputComponentValue(x, y) saturates at a value of 0. Since the component value range is required to be maintained after application of the non-uniformity correction, the maximum value of OutputComponentValue(x, y) saturates at a level defined by the number of bits in the component value, for instance, 65535 for a 16-bit unsigned integer component.

The NUC coefficients are assigned per component value through the full image. The nuc_gain table auxiliary image item and nuc_offset table auxiliary image item are aligned with the entire base image, resulting in the auxiliary items having width and height equal to the base image item’s image_width and image_height. For a component value {x, y} position in the image, with {0,0} being the top-left component value, the NUC coefficients for each component value are given by position {x,y} in the nuc_gain table and nuc_offset table auxiliary image items.

A 'nuci' derived image item shall be associated with a 'snuc' item property, with the version set to 1.  This eliminates the storage of the gain and offset arrays in the 'snuc' item property (or sample entry box). The property contains an item component_index listing which defines the component(s) of the visual base input image item the non-uniformity correction(s) apply to. The remaining components are copied to the 'nuci' derived image item without modification. The property also contains a nuc_is_applied flag to indicate whether the NUC has been applied to the derived image item. This value is set to 1 when using the 'nuci' derivation. A 'snuc' item property may be associated with the base input image item with the nuc_is_applied flag set to 0, indicating non-uniformity corrections have not been applied to the raw input image.

The nuc_gain table auxiliary image item and the nuc_offset table auxiliary item shall have either one component, or a number of components equal to the component_count listed in the 'snuc' item property. When the number of components in the auxiliary items is one, the same nuc_gain table and nuc_offset table is applied to all components listed in the 'snuc' box. When the number of components in the auxiliary items is equal to the number of components defined by the component_count, the nuc_gain and nuc_offset corrections are applied to the matching component in the base image item based on the ordering of components in the 'snuc' item property component_index list.

A 'nuci' derived image item shall be associated with an 'ispe' item property. Optional item properties may include the TAI Timestamp ('itai') and Clock Information ('taic') properties. Having a timestamp associated with the NUC calibration data supports determining the level of temporal alignment of the calibration data with the image data the calibration is being applied to.

When a 'nuci' derived image item is the input to another derived image item, that derived image item shall treat the reconstructed image of the 'nuci' derived image item as if the nuc_gain and nuc_offset tables have been fully applied.

The nuc_gain table and nuc_offset table auxiliary input image items should be marked as hidden by setting (flags & 1) equal to 1 in their ItemInfoEntry declarations.

A notional block diagram for the implementation of a 'nuci' derived image item is shown in Figure 31.

Figure 31: Generating a derived image with non-uniformity corrections

NOTE 1: Backwards compatibility with parsers that do not support the non-uniformity correction derivation can be achieved by placing the base input image item and the 'nuci' derived image item in an 'altr' entity group.

Example:

This example illustrates how to store a non-uniformity corrected derived image item in a file.

FileTypeBox 'ftyp': major-brand='mif1', compatible-brands='mif1, geo1, etc.'

MetaBox 'meta': (container)

HandlerBox 'hdlr':	'pict'

PrimaryItemBox 'pitm': item_ID=1;

ItemInfoBox 'iinf': entry_count=4		
		1)	'infe': item_ID=1, item_type='nuci';	 		// Non-uniformity corrected image
		2)	'infe': item_ID=2, item_type='uncC'; 			// base image
		3)	'infe': item_ID=3 (Hidden), item_type='uncC'; 	// NUC gain table

4)	'infe': item_ID=4 (Hidden), item_type='uncC'; 	// NUC offset table

ItemLocationBox 'iloc': item_count=4
		item_ID=1, construction_method=1, extent_count=1, extent_offset=P1, extent_length=Q1;
		item_ID=2, extent_count=1, extent_offset=P2, extent_length=Q2;
		item_ID=3, extent_count=1, extent_offset=P3, extent_length=Q3;
		item_ID=4, extent_count=1, extent_offset=P4, extent_length=Q4;

ItemReferenceBox 'iref':

referenceType='dimg', from_item_ID=1, ref_count=3, 	// nuci

to_item_ID=2, 					// base

to_item_ID=3; 					// NUC gain table

to_item_ID=4; 					// NUC offset table

ItemPropertiesBox 'iprp':
		ItemPropertyContainerBox 'ipco':

Index:

// nuci derived item, item 1
		1)		'ispe'

2)		'snuc'		//sensor non-uniformity correction item property

// base uncC image, item 2

3)		'ispe'

4)		'cmpd'

5)		'uncC'

6)		'taic'		//Clock Information

7)		'itai'		//TAI Timestamp

// NUC gain table, item 3

8)		'ispe'

9)		'cmpd'

10)		'uncC'

11)		'auxC'		//AuxiliaryTypeProperty – defines type as NUC_gain table

12)		'taic'		//Clock Information

13)		'itai'		//TAI Timestamp

// NUC offset table, item 4

14)		'ispe'

15)		'cmpd'

16)		'uncC'

17)		'auxC'		//AuxiliaryTypeProperty – defines type as NUC_offset table

18)		'taic'		//Clock Information

19)		'itai'		//TAI Timestamp

ItemPropertyAssociation 'ipma': entry_count=4
			1)	item_ID=1, association_count=2 // nuci
				essential=0, property_index=1;
				essential=1, property_index=2;
			2)	item_ID=2, association_count=5 // base

essential=0, property_index=3;
				essential=1, property_index=4;
				essential=1, property_index=5;
				essential=0, property_index=6;

essential=0, property_index=7;

3)	item_ID=3, association_count=6 // NUC gain table

essential=0, property_index=8;
				essential=1, property_index=9;
				essential=1, property_index=10;
				essential=1, property_index=11;

essential=0, property_index=12;

essential=0, property_index=13;

4)	item_ID=4, association_count=6 // NUC  offset table

essential=0, property_index=14;
				essential=1, property_index=15;
				essential=1, property_index=16;
				essential=1, property_index=17;

essential=0, property_index=18;

essential=0, property_index=19;

ItemDataBox 'idat':

Non-uniformity correction derivation data block (at idat_offset P1, with length Q1)

GroupsListBox 'grpl':

EntityToGroupBox 'altr': group_id=1, num_entities_in_group=2

entity_id=1;

entity_id=2;

MediaDataBox 'mdat':
	uncC Image (at file offset P2, with length Q2)

uncC Image (at file offset P3, with length Q3)

uncC Image (at file offset P4, with length Q4)

////////////End of Example

### Sensor non-uniformity corrected pre-derived images

#### Definition

An item_type of 'uncC' is a pre-derived, uncompressed image item containing one or more components with component values modified via the application of gain and offset non-uniformity corrections. The 'uncC' image item is constructed using the following three inputs:

- a raw, uncompressed base image item,

- a nuc_gain table auxiliary image item, and

- a nuc_offset table auxiliary image item.

The process follows the steps for generating a non-uniformity correction derived image item (see Section x.x.), with the following exceptions:

- Instead of a 'dimg' item reference, a 'base' item reference, with a reference count of three, is declared from the 'uncC' pre-derived image item, and to the raw, uncompressed base image item, the nuc_gain table auxiliary image item, and the nuc_offset table auxiliary image item.

- The resulting image is a pre-derived image item instead of a derived image item.

To label the output image item as having been non-uniformity corrected, a 'snuc' item property is associated with the output image, the version is set to 1, and the nuc_is_applied flag is set to 1. To label the input image as being raw, without corrections, a 'snuc' item property is associated with the input image, with the version set to 1, and the nuc_is_applied flag set to 0.

A notional block diagram for the implementation of a pre-derived image item is shown in Figure 32.

NOTE: 	When removing (or modifying) an item that is marked as a base image of a  pre-derived'uncC' image, the associated 'base' item reference associated with the 'uncC' item needs to be removed.

Figure 32: Generating a pre-derived image with non-uniformity corrections

### Sensor non-uniformity correction derivation for derived visual tracks

#### Definition

Box Type: 'nucv'

Mandatory (per sample):	No

Quantity (per sample): 0 or 1

Inputs: (1) nuc_gain table auxiliary item, (2) nuc_offset table auxiliary item, (3) raw, uncompressed visual track

This derivation follows the process defined in ISO/IEC 23001-16 Derived visual tracks in the ISOBMFF. The number of inputs shall be equal to 3 for this derivation transformation.  A summary of the transformation is contained in Table 10.

Table 10: Inputs and Parameters for the 'nucv' derivation

| Operation | Inputs | Parameters |
| --- | --- | --- |
| Sensor non-uniformity correction | 0 | - raw, uncompressed visual track - nuc_gain table auxiliary item - nuc_offset table auxiliary item |

The inputs for this derivation transformation shall be:

- a raw, uncompressed visual track,

- a nuc_gain table auxiliary item, and

- a nuc_offset table auxiliary item

A 'snuc' box is included in the DerivedVisualSampleEntry of the track. The 'snuc' box  is configured with version set equal to 1 and nuc_is_applied is set to 1. The nuc_gain table is carried in the nuc_gain table auxiliary item and the nuc_offset table is carried in the nuc_gain table auxiliary item. Both of these auxiliary items are declared in the file level MetaBox.

The track has three track references of type 'dtrk', from the 'nucv' derived track, and (1) to an raw, uncompressed video track, (2) to a nuc_gain table auxiliary item and (3) to a nuc_offset table auxiliary item.

Construction of the derived output sample is achieved by applying gain and offset corrections from the nuc_gain table and the nuc_offset table to an input visual track sample.

Given the input parameters:

- Let InputComponentValue(x, y) be the component value intensity from a sample of a raw uncompressed source track sample at spatial coordinates (x, y).

- Let nuc_gain(x, y) be the component value gain factor retrieved from a nuc_gain table auxiliary image item at spatial coordinate (x,y).

- Let nuc_offset(x, y) be the component value offset factor retrieved from a nuc_offset table auxiliary image item.

Generate the derived output result:

- The derived output component values shall match the component value type (unsigned integer, etc.) and bit depth of the base input track sample.

- Let OutputComponentValue(x, y) be the component value intensity from the derived track sample with non-uniformity corrections applied via the following mathematical transformation:

For each component in a track sample labeled for non-uniformity correction in the derivation, the correction equation, at component value location (x,y) is:

OutputComponentValue(x, y) = nuc_gain(x,y) * InputComponentValue(x, y) + nuc_offset(x,y)

where x and y provide the (columns, rows) component value location.

The visual output of the derivation transformation contains the source uncompressed visual track with non-uniformity corrections applied. A summary of the box structure for the 'nucv' derivation is shown in Figure 33.

Figure 33: The 'nucv' derivation for track samples

#### Syntax

aligned(8) class DerivedSensorNonUniformityCorrection extends VisualDerivationBase ('nucv', flags) {

}

#### Semantics

N/A

Bibliography

[1]	ISO/IEC 23002-3, Information technology — MPEG video technologies — Part 3: Representation of auxiliary video and supplemental information

[2]	ISO/IEC 23091-2, Information technology —Coding-independent code points — Part 2: Video

[3]	IEEE 1588-2008, IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems

[4]	IETF RFC 5905, Network Time Protocol Version 4: Protocol and Algorithms Specification

[5]	SMPTE 336M-2007 (Data Encoding Protocol Using Key-Length Value)