INTERNATIONAL ORGANISATION FOR STANDARDISATION
ORGANISATION INTERNATIONALE DE NORMALISATION
ISO/IEC JTC 1/SC 29/WG 11
CODING OF MOVING PICTURES AND AUDIO

- ISO/IEC JTC 1/SC 29/WG 11 N16708
Genève, CH, January 2017

Title:		White paper on MPEG-DASH’s New Features

Source:	Communication

Editor:	Iraj  Sodagar (Microsoft, US)

MPEG-DASH’s New Features

2017/01/20

# Introduction

With the publication of 1st edition of MPEG-DASH standard (ISO/IEC 23009-1) in April 2012, the internet video streaming industry got excited about the first open international standard that could converge the industry to a single protocol for streaming.  The standard was created by a large set of leading technology companies in streaming industry, combining and creating the best and more efficient techniques under a single standard that is open and available to use by anyone. This was the first time that an open standard would enable the true convergence in streaming protocol and help services provider to reduce their copies of catalogs by using a single standard.

During the last four years, MPEG has work to enhance and extend DASH functionalities. Today MPEG-DASH has 6 different parts, some of which in their second edition. The 3rd edition of the core part will be published in the first half of 2017. This white paper describes some of the newer features of MPEG-DASH. Figure 1 list the highlighted features of each edition and various parts.

MPEG-DASH highlighted features

## Part 1: MPEG-DASH core specification

MPEG-DASH standard (ISO/IEC 23009) currently consists of six parts. Part 1 [1] defines the manifest and segment formats. This is the core part of specification. The DASH standard can be used by several delivery protocols including HTTP/1.1. This part is also codec agnostic i.e. it can be used with any media coder that can be contained in fragmented ISO Base Media File Format (ISOBMFF) [2], or MPEG-2 transport. This part defines a manifest, Media Description Presentation (MPD), which expresses the timing model of a media content. It also specifies various components of the media content and their location on internet in multiple variation for dynamic switching and bandwidth adaptation. A Representation is one variation of a media content in this collection. All Representations of the same content (differ in i.e. bit rate, resolution, frame rate) are grouped into an Adaptation Set. MPD can be very expressive for content annotation. For each media component, MPD may define its role, its accessibility rules, its viewpoints or audio channel configuration and information about its DRM and decryption scheme if it is protected. It has an update mechanism by which can add new content. This feature is important for live streaming.  MPD can support multiple CDNs and several segment addressing schemes.

Media segment format allows a media component to be streamed segment by segment (or even subsegment by subsegment). Segments have order and concatenation of them creates a complete media stream. Segments, if encoded properly, also enable seamless switching between different representations by the DASH client to adopt to the network bandwidth fluctuations.

The 1st edition of Part 1 was published in 2012. Two years later, MPEG added additional feature including event messages and asset identifiers [1]. Event messages can carry media timeline events in MPD or in-band as part of media segments. Events provide a flexible method of sending sparse information to be used by DASH client or an application using the DASH client. For instance, MPD update event informs the client to download a new update to its MPD. In other hand, SCTE35 messages can be embedded in in-band events to carry ad-insertion signaling.

Since 2014, MPEG has added several new features to DASH Part 1, which will be included in the 3rd edition , expected to be published in the first half of 2017. We list some of the newest features here.

### Spatial Relationship Description

Due to popularity of large size/multiple camera capture of the scene, delivery of large size video in one piece may not be efficient. Instead, the video can be divided to an array of tile and a client only streams the tiles that are relevant to its view. Spatial Relationship Description (SRD) tool [3] signals the coordinates and size of the video tile with respect to the coordinates of entire video in MPD.  This information enable a client to calculate the relative location of each video tile. Then the client can decide to stream a small region which may consist of a few tiles, or even entire video by streaming every single tile at the same time. SRD provide a very flexible coordinate system which allows overlaps, tiles for ROI, and tiles with different sizes. SRD is one of key technologies being consider for efficient VR streaming.

### External MPD Linking

A MPD can create a relationship between a single program/channel and a preview mosaic channel having all channels at once. By viewing the mosaic channel, the user can browse the mosaic video of all channels, decide which channel s/he wants to watch. Then Viewer picks that channel and the player starts streaming that channel in full resolution. External MPD linking [4] enables use-case. In the MPD for mosaic, each channel is represented as one empty Adaptation Set. In each channel MPD, there is an identical Adaptation Set that represent same content. The linking mechanism allows every empty Adaptation Set in mosaic MPD to be linked to the MPD of the corresponding channel.

### Period Continuity

The media time line of a media session can be broken into multiple periods. The Period element in MPD is a simple way to break the timeline to different intervals. Periods allow ad-insertion or adding new periods to a MPD of live program. Since not all periods of the same content may not be played sequentially (for example in the case of ad-insertion that the player plays one period of content, followed by one period of ad, followed by one period of the same content), DASH provides a signaling [4] to indicate whether one period is a continuation of the previous one. Period continuity signaling also indicate time-alignment of content is needed to play the content smoothly during period transition.

### MPD Chaining

Playing pre-roll ads before playing a content is one function often needed by ad-driven streaming services. In this use-case, when the user select a content, the player should play a pre-roll ad first and then continue with playback of the selected content. In this use-case, the selected content might be on-demand (pre-stored) or live (the content is produced in real time). Supporting pre-roll ads for on-demand content is easy: we can create a new MPD that has two periods: one Period corresponding to the ad followed by the Period corresponding the content. But in the case in which the main program is live, the expected behavior can be that when user tries to join the live session, the player should play the canned ad first and join the live session at the live edge.  Since the live edge can vary for each experience, there is not easy way of creating a combined MPD. MPEG developed the MPD chaining tool [5], which allows chaining two or more MPDs to each other. A player plays the first MPD (e.g. ad) and upon completion, start playing the live MPD at its live edge.

### Flexible Segment Format for Broadcast TV

This tool, also included in the amendment 4 of DASH 2nd edition [5], separates the signaling of the switching points and random access points in each stream. Switching points are used for dynamic adaptation by seamless switching cross Representations. Random access points are used to start the playback at those points, for instance because of a channel change. In basic DASH profiles, the switching points and random access points are the same making the format simpler. But in order to have frequent random access point, the segments need to have more frequent switching point which in turn results in lower compression efficiency (more IDR frames). The flexible segment format allows to separate the switching points and random access points. Therefore, the content can be encoded with a good compression efficiency, yet allowing higher number of random access point, but with lower frequency of switching points. Such content is suitable both for streaming as well as Broadcast TV (hybrid delivery).

## Part 5 Server and Network Assisted DASH

MPEG DASH clients follow a client-pull paradigm by adapting their requests based on the available bandwidth and other local resources. This has proven to be easier to deploy over CDN infrastructure than server-push technologies. However, this decentralized nature introduces new challenges when offering a consistent and higher quality of service for premium users. For instance, there are many examples of situations where quality of experience can be affected:

- Resources announced in the MPD may become outdated after a network failure or reconfiguration, resulting in misdirected and unsuccessful DASH segment requests by the client.

- A DASH client can mistakenly switch to lower quality segments when a mobile hand-over or a cache miss is interpreted as a bandwidth reduction.

- A DASH client may unnecessarily start a stream with lower quality segments, and only ramp up after bandwidth estimation is reliable.

- Multiple DASH clients may compete for a shared bandwidth, leading to unwanted mutual interactions and possibly oscillations

MPEG DASH Part 5 Server and Network-assisted DASH (SAND) [6] addresses these issues. The key features of SAND are asynchronous network-to-client and network-to-network communication of quality-related assisting information.

SAND architecture has been defined as an augmentation of the regular DASH architecture thanks to the addition of a SAND communication channel that allows SAND enabled elements to exchange SAND messages between them. These SAND messages carry relevant information such as bandwidth availability, future request of DASH clients, presence of media segments in nearby HTTP caches, etc.  In addition, the real-time and stateless design of SAND make optimization of deployments possible in a variety of environments such as home network, over-the-top, mobile networks and others.

## Part 6:    DASH with Server Push and WebSockets

Introduction and deployment of more advance protocols such as HTTP/2 and WebSocket provides the opportunity for streaming DASH content over those protocols and benefiting from their new features.  Both HTTP/2 and WebSocket allow server-initiated and client-initiated transactions, data request cancelation, and multiplexing of multiple data responses.

Part 6 of DASH [7] provides an interoperable use of server-push mode of each protocol. The specification defines a set of push directives sent by client requesting the server to push multiple segments together.  Each defined push directive defines a specific push strategy. For instance, with push-next directive the client requests the server to push the next K segments, where the K is defined in by the client. With push-time directive, the client requests the server to push any available segment of current stream for the next N seconds, while N is also defined by the client. The push-list directive means a request to the server for pushing the segments in the directive. The specification also defines push acknowledgments that are sent by server in the response of a push directive.

The main use-case for DASH with Server Push and WebSockets is the low latency live streaming. In this case, the client is streaming very close to live edge.  In order to reduce the latency, a segment should be delivered to the client in shortest time possible. In this case, if the sever has received the appropriate push directive, as soon as a segment becomes available on server, the server doesn’t need to wait for a HTTP GET request from the client and pushes it in the push channel.

## References

[1] ISO/IEC 23009-1:2014 Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats

[2] ISO/IEC 14496-12:2015 Information technology -- Coding of audio-visual objects -- Part 12: ISO base media file format

[3] ISO/IEC 23009-1:2014/Amd 2:2015 Spatial relationship description, generalized URL parameters and other extensions

[4] ISO/IEC 23009-1:2014/Amd 3:2016 Authentication, MPD linking, Callback Event, Period Continuity and other Extensions

[5] ISO/IEC 23009-1:2014/Amd 4:20xx AMENDMENT 4: Segment Independent SAP Signalling (SISSI), MPD chaining, MPD reset and other extensions

[6] ISO/IEC 23009-5 Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 5: Server and network assisted DASH (SAND)

[7] ISO/IEC DIS 23009-6 Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 6: DASH with server push and websockets