INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 N17284

October 2017, Macau, CN

| Source | Editor |
| --- | --- |
| Status | Approved |
| Title | Whitepaper: Common Encryption for ISO Base Media Files |
| Author | Kilroy Hughes |

Common Encryption for ISO Base Media File Format is specified in ISO/IEC 23001-7, currently in its 3rd  Edition.

# Overview

Common Encryption specifies elementary stream encryption and encryption parameter storage to enable a single ISO Media file to be used on a wide range of devices that support different Digital Rights Management systems (DRM) to manage keys and securely decrypt the media.

This enables a significant improvement in the efficiency in the production, storage, and distribution of multimedia content on the Internet, which previously required multiple copies of each file, one for each DRM system.

Encryption of the elementary stream enables a high level of content protection on a wide range of devices.  By not encrypting the ISO Media file container, file readers and players can process files and tracks normally until they reach the elementary stream decryption and decoding stage. Only the elementary stream decryptor and decoder must be secure. Encrypted data need not be accessed by insecure applications after being decrypted by a secure video processor.

Common Encryption uses an optimized algorithm for video samples called partial NAL encryption.  Partial NAL encryption only encrypts the video data contained in video NAL units so that sample size headers, NAL headers, video slice headers, and non-video NALs may be processed normally, prior to decryption.  Elementary streams can be edited without decryption to convert between different sample formats, such as ‘avc1’, ‘avc3’, ‘hvc1’, ‘hev1’, and to or from elementary streams containing startcodes used in MPEG-2 Transport Streams.  This allows a single Common Encrypted elementary stream to be packaged for delivery in both MPEG-4 files and MPEG-2 transport streams without decryption and re-encryption of the elementary stream.

Audio and other media types may be encrypted using full sample encryption, which encrypts all bytes in a sample.

Decryption is simplified by storing byte offsets to encrypted data so audio and video decryption can be performed without requiring that the decryptor be aware of the codec or elementary stream format. Clear and encrypted byte ranges are identified in the track metadata as “subsamples”. Subsamples allow decryption of media data stored in a Media Data Box (‘mdat’) box using the always present sample byte offsets (e.g. in the Track Run Box ‘trun’) to locate samples, and the Common Encryption subsample byte offsets to locate clear and encrypted byte ranges in each sample.

An initialization vector (IV) and subsamples for each media sample may be stored in each sample’s track or track fragment header in Sample Auxiliary Information. Appropriate location, such as in a Sample Encryption Box (‘senc’) in a movie fragment, makes each sample randomly decryptable by making the IV accessible when samples are located by their byte offset. Playback operations such as fast forward and rewind are enabled without a significant increase in processing to locate the sample, IV, and encrypted cipher blocks.  IVs and subsample ranges can be indexed using Sample Auxiliary Information Offset Box (‘saio’) and Sample Auxiliary Information Size Box (‘saiz’), which enable flexible location of the Sample Auxiliary Information.

A Key Identifier (KID) is associated with each track and sample, which allows multiple DRM systems to identify the key that was used to encrypt the sample(s).  For typical cases, a single key per track is used and identified by the default_KID field in the Track Encryption Box (‘tenc’).  Sample groups may be used to signal key changes over time, a process sometimes referred to as “key rotation” when used during video streaming. Sample groups can also indicate samples that are not encrypted, i.e. unprotected segments of an audio or video track.

License acquisition and storage are supported by the addition of an optional Protection System Specific Header Box (‘pssh’) in the Movie Box (‘moov’).  The ‘pssh’ box contains a System_ID field to indicate the protection system that it pertains to, and that protection system may interpret the data stored there to acquire a license that is usually bound to a specific user or device through a DRM key management system, which also enforces the security of authorized decryption and playback implementations.

Licenses containing protected keys may also be stored in ‘pssh’ boxes in Movie Fragment Boxes (‘moof’), as well as the Movie Box.  Key rotation may be implemented by storing “leaf” licenses in a ‘pssh’ box in each movie fragment to make the protected keys available within each movie fragment that references them by KID.  Leaf licenses may be shared by all users, but require an individualized “root” license to decrypt the leaf license.

# First Edition

Edition 1 of Common Encryption has already been adopted by many application formats specified for the delivery of premium video using AVC video compression and MPEG DASH streaming.  It defines a single encryption scheme using AES-128 counter mode cipher, and is identified by the four character code ‘cenc’.

# Second Edition

The second edition of Common Encryption is generalized to work with any NAL structured video format, including AVC, HEVC, SVC, etc.  The encryption algorithm for NAL structured video does not encrypt decoding parameter NALs, SEI Message NALs, video NAL headers, or video slice and tile headers so that they may be read by parsers and players that require video information to control decoding, scaling, synchronization, splicing, captions, etc.

Second Edition also specifies an optional encryption scheme (‘cbc1’) using AES-128 with Cipher Block Chaining mode (CBC), which uses the previous cipher block to generate the initialization vector (IV) for the next cipher block in the sample using an XOR operation.  Counter mode (CTR) generates the IV for the next cipher block by incrementing a counter that is initially set to the IV value of the sample, then incremented in sequence for each encrypted cipher block in that sample.  All encrypted cipher blocks in a sample may be concatenated and decrypted as a single sequence, regardless of NAL boundaries.  CBC mode can only operate on complete 16 byte cipher blocks, so last block alignment is used for partial NAL encryption to avoid partial cipher blocks.

Second Edition specifies a Sample Encryption Box (‘senc’) that may be used to store initialization vectors and encryption range byte offsets.  The Sample Encryption Box (‘senc’) may be stored in each track fragment in an encrypted track so that decryption information is accessible in every Segment during random access file playback, and during adaptive streaming of movie fragments.  Using ‘senc’ in each Track Fragment Header Box (‘traf’) results in each movie fragment (and DASH Media Segment) being independently decryptable because it contains the byte offset to access each sample, the byte offsets of each encrypted range within the sample, and the initialization vector necessary to decrypt the first cipher block of each sample.

Both CTR mode (‘cenc’ scheme) and CBC mode (‘cbc1’ scheme) store an IV for each sample that can be stored in each movie fragment to provide error resilience and random access without requiring download or processing of previous sample data. Randomization of the IV for each sample may provide additional security against crypto attacks.

# Second Edition Optimizations for Streaming

The Second Edition specifies XML elements and attributes that may be used to expose some Common Encryption file information in XML documents, such as a DASH Media Presentation Description (MPD) streaming manifest.  The DASH MPD Descriptor complex type allows the addition of an attribute and/or element in a declared namespace different from the DASH namespace. This extension mechanism may be used to add the cenc:default_KID attribute and the cenc:pssh element to Descriptors in DASH MPDs.

A DASH MPD containing a ContentProtection Descriptor with the schemeIdURI "urn:mpeg:dash:mp4protection:2011” notifies a DASH player that the content in an Adaptation Set is encrypted using the protection scheme indicated in the value attribute.  An attribute is defined in the Second Edition of Common Encryption that can be added to expose the default_KID stored in the Track Encryption Box (‘tenc’) in an MPD ContentProtection Descriptor element.  The default_KID field in ‘tenc’ is a big endian array of 16 bytes, and is stored in the cenc:default_KID attribute in DASH ContentProtection Descriptor elements as a UUID string.  The mp4protection scheme sets the @value attribute of the descriptor to the encryption scheme stored in the Protection Scheme Box (‘schm’) in an Adaptation Set of ISO Base Media files and Segments.  The protection schemes defined in Second Edition are ‘cenc’ and ‘cbc1’.

The cenc:default_KID attribute identifies the key and the associated content to applications that could verify or buy rights to a presentation, distinguish unique and duplicate KIDs in different Adaptation Sets, and check for the presence of a key with that KID on the device, or request a license containing that KID in some DRM system the playback application or device supports.  The following is an example, and a similar descriptor with a default_KID that may be required for some or all encrypted Adaptation Sets in a DASH MPD.

<ContentProtection schemeIdUri="urn:mpeg:dash:mp4protection:2011" 
    value="cenc" cenc:default_KID=”34e5db32-8625-47cd-ba06-68fca0655a72”/>

Second Edition also defines an element that can be used to expose the Protection System Specific Header Box (‘pssh’) normally stored in the Movie Box (‘moov’) in a file header or DASH Initialization Segment.

DASH ContentProtection Descriptor elements in an MPD may use a urn:uuid schemeIdUri to identify a specific DRM system using the SystemID value used in the Protection System Specific Header Box (‘pssh’) defined in this specification.  Each DRM system may also specify additional elements and attributes for its scheme and SystemID that can be used for license acquisition or other functions of the identified DRM system.

Exposing license acquisition information early in the MPD allows licenses to be acquired in advance of a live event for which not media id available, or before any other playback attempt.  An early license request using MPD information may avoid the delay and disappointment of downloading and attempting to play content that will then be found to be unauthorized or incompatible by a particular user.  Protection System Specific Header Information stored in the MPD may be different for different networks and service providers using the same content, and may be changed over time without the need to edit the media files.  For instance, the license server URL for one or more DRMs may be different over time, by location, by ISP, etc.

An MPD may include a ContentProtection Descriptor element with schemeIdUri of urn:uuid for each SystemID that can provide a DRM license, and include sufficient information in the descriptor to enable license acquisition.  License acquisition can be enabled by adding a cenc:pssh element to each urn:uuid scheme descriptor so that a player can find the same license acquisition information it would find in a ‘pssh’ box. An MPD will normally be processed before Media Segments are downloaded, so license acquisition information in an MPD will normally take precedence over information stored in ‘pssh’ boxes.  Note that the cenc:pssh element contains a complete ‘pssh’ box, not just the contents of the box, so that parsing will be identical from the MPD or file.

The following example shows both types of Content Protection Descriptors used together in an MPD.

The first Content Protection Descriptor with schemeIdUri of urn:mpeg:dash:mp4protection:2011 indicates that the ‘cenc’ scheme (Common Encryption CTR mode) was used to encrypt the referenced media, and the cenc:default_KID.

The second Content Protection Descriptor with schemeIdUri of urn:uuid type, indicates a hypothetical DRM system “Acme” with a SystemID represented as a UUID string, and a cenc:pssh element containing a base64 encoded ‘pssh’ box with that SystemID to provide license acquisition information in the MPD.  There may be multiple ContentProtection Descriptors with schemeIdUri of urn:uuid, but with different SystemIDs, each indicating a different DRM system that can provide a license.

<ContentProtection schemeIdUri="urn:mpeg:dash:mp4protection:2011" value="cenc" cenc:default_KID=”34e5db32-8625-47cd-ba06-68fca0655a72” /> <ContentProtection schemeIdUri=”urn:uuid:d0ee2730-09b5-459f-8452-200e52b37567” value=”Acme 2.0”> <!-- base64 encoded ‘pssh’ box with this Acme SystemID --> <cenc:pssh> YmFzZTY0IGVuY29kZWQgY29udGVudHMgb2YgkXB zc2iSIGJveCB3aXRoIHRoaXMgU3lzdGVtSUQ= </cenc:pssh> </ContentProtection>

Common Encryption has proven efficient for the delivery of video to the wide variety of the billions of devices now able to access Internet delivered video because it enables the use of a single Common Encryption elementary stream for stored files, live adaptive streaming, M2TS and ISO Media container formats, a variety of codecs and sample formats, and multiple DRM systems.

Third Edition

The third edition of Common Encryption added two pattern encryption schemes, identified as ‘cbcs’ and ‘cens’. These schemes can be used to protect encoded video by encrypting and skipping blocks in a regular pattern, e.g. encrypting one block in ten. This is sufficient to prevent decoding of compressed video, while reducing the amount of processing, battery power, etc. required to decrypt the content. Unlike the ‘cens’ scheme, the ‘cbcs’ scheme allows a constant initialization vector per track and relies on alignment with video NALs in the encryption process, but uses the same byte range signaling for protected ranges used by all Common Encryption schemes. Of the four encryption schemes defined in Common Encryption, only the ‘cbcs’ and ‘cenc’ schemes are widely deployed, and are referenced by the MPEG Common Media Application Format (CMAF).