INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2014/N15069

Geneva, CH

February 2015

| Title | Draft of White paper on MPEG Media Transport (MMT) |
| --- | --- |
| Source | Communication |
| Status | Approved |
| Editor | Kyuheon Kim, Kyungmo Park, Sunghee Hwang, Jaeyeon Song |

# Introduction

During the last two decades, MPEG has developed a number of standards for multimedia delivery. Most notably, the MPEG-2 Transport Stream (TS) has been standardized to support real-time streaming delivery, and the ISO Base Media File Format (BMFF) has been standardized for the exchange and progressive download applications. Both have been designed to provide essential information for multimedia services such as the structure of multimedia content composed of multiple audio-visual streams and the temporal relationship among those streams. MPEG-2 TS is currently being used for most real-time multimedia services including digital broadcasting and IPTV, while the ISO BMFF has been adopted as a storage format by the Third Generation Partnership Project (3GPP) and Digital Entertainment Content Ecosystem.

We are witnessing a change in the multimedia service environments due to the rapid increase of multimedia content delivery over the Internet, which introduces several new requirements for multimedia delivery. For instance, a greater demand exists for flexible and partial access to media content.

MPEG-2 TS provides efficient mechanisms to multiplex multiple audio-visual data streams into one delivery stream according to consumption order. Audio-visual data streams are packetized into small fixed-size packets and interleaved to form a single stream in the order of synchronized playback. In addition, information about the multiplexing structure is also interleaved with the data packets so that the receiving entity can efficiently identify a specific stream. Sequence numbers identify missing packets at the receiving ends, and timing information is assigned after multiplexing with the assumption that the multiplexed stream will be delivered and played in sequential order. These design principles made MPEG-2 TS a perfect solution for streaming of multimedia content to a large number of users.

However, the increase of personalized, on-demand viewing of multimedia content over the Internet has introduced a new requirement for more flexible access to the content. MPEG-2 TS cannot efficiently support personalized advertisement or audio-stream selection with a language suitable for a specific user because dynamic insertion or multiplexing of advertisements or audio streams requires demultiplexing and remultiplexing streams. The ISO BMFF has a similar challenge in that it stores metadata containing information for synchronized playback that is separately stored from the compressed media data. To support the progressive download of a file (enabling playback of the file before it is completely downloaded), the metadata can be interleaved with the media data in the order of consumption. However, it is still difficult to efficiently access a certain subset of the file, for example, to download only an audio stream with a specific language or switch from one to another during playback.

Under this change of multimedia delivery environment, MPEG started to develop a new standard “MPEG Media Transport” called as “MMT”. MMT assumes an IP network with in-network intelligent caches close to the receiving entities, which not only actively cache the contents but also adaptively packetize and push the content to the receiving entities. In order to achieve the IP friendly delivery service, MMT defines broadly three requirements as follows. Firstly, easy access to multimedia components in multimedia contents, and secondly decrease of inefficiency brought by different formats for delivery and storage, and finally easy combination for various contents of multimedia components, which are located over various cashes and storages.

MMT is composed of three different parts as a standard package such as MPEG-H Part 1 MMT, Part 10 MMT Forward Error Correction (FEC) Codes and Part 11 MMT Composition Information (CI). This paper shows the architecture of MMT and explains its relevant functionalities in MMT standard package.

Architecture

To efficiently and effectively support multimedia content delivery over the emerging Internet of the future, MMT develops technologies for three major functional areas such as encapsulation, delivery and signalling functions as being shown in Figure 1. Also, the technologies defined by MMT can be used in an IP environment.

Figure 1. Conceptual architecture of the MPEG Media Transport (MMT) standard

In the encapsulation function, encoded media is to be processed into a specified MMT format, which defines the content’s logical structure and the physical encapsulation format by using the ISO BMFF. For example, encoded multimedia component is decomposed into Assets, and individual Asset is encapsulated into a MPU file, which is newly defined in MMT based on ISO BMFF. Also, spatial and temporal relationship among multimedia components is described by HTML5 and Composition Information (CI) as being shown in Figure2. This MPU file produced by a encapsulation function is used either to store coded media data on a storage device for delivery preparation.

The delivery functional area defines the application-layer protocol that supports packetized streaming delivery of multimedia content through a heterogeneous network environment. It also defines the payload’s format to packetize encapsulated coded media data, considering the underlying delivery layer packet’s size. For example, the delivery function is to put Assets into a MMTP payload, which can be delivered by any other networks including broadcasting and IP network. In case of recovering lost MMTP packets AL-FEC is defined in MPEG-H part 12.

The signalling functional area defines message formats to manage MMT package consumption and delivery. Consumption management messages signal the MMT package’s structure, and delivery management messages signal the payload format’s structure and the protocol’s configuration. For example, the jitter can be calculated by this signalling information.

Figure 2. Functional flowchart of MMT

MMT

Content Model

MMT content is modelled by an onion-shell structure. It is composed of Media Fragment Unit (MFU), Media Processing Unit (MPU), MMT Asset and MMT Package. The relationship among these units is explained as follows; In order to produce MMT content, encoded media data is decomposed based on Access Unit (AU) or Slice. This AU or Slice data is considered as MFU, which can be independently decoded by itself. One or more than one MFUs are combined into MPU, which is structured on the basis of ISO BMFF, and is considered as a minimum unit for storage and consumption of a MMT content, and also can be independently decoded by itself. One or more than one MPU files are combined into an Asset, which is a logical unit for multimedia component elementary stream such as audio, video and data.  Individual Asset has a its own ID, and thus all the MPU files for a relevant Asset can be recognised through this ID. Finally, a MMT package can be realised by one or more than one Assets, where a MMT package is a logical unit of multimedia content such as a broadcasting program. Since a MMT package is represented by spatial and temporal relationship among Assets, and can be delivered by various networks, these relationships and delivery network information is contained in a MMT Package. As being shown in Figure 4., the spatial and temporal relationship is described by Composition Information, and network information is done by Transport Character Information (TCI).

Figure 3. MMT content model

As being explained in above, a MMT Package is a logical unit for a multimedia content, which is composed of media elementary streams and relevant data. And its basic file format for storage is specified by a combination of MPU files. Since MPU file is inherited from ISO BMFF structure, MPU file is also defined by box-based structure. MPU file format additionally defines two new boxes for MMT Package such as MPU box (‘mmpu’) and MMT Hint box (‘mmth’) as being shown in Figure 4. The ‘mmpu’ box contains a sequence number and Asset ID, which help to recognise which Asset a MPU file is belong to, and its location in Asset. The ‘mmth’ box provides the number of MFUs, media data property such as timed or non-timed data and media sample information. As being shown in Figure 4., a MPU file is inherited from ISO BMFF and thus, uses the boxes defined in ISO BMFF. A MPU file is identified by the brand name such as ‘mpuf’ in ‘ftyp’ box. Instead of ‘sidx’ box in ISO BMFF, a MPU file defines ‘mmpu’ box explained in the above. And ‘moov’ box followed by ‘mmpu’ box provides track and initial information of media data, where ‘mmth’ box newly defined for a MMT Package is added.

Figure 4. MPU structure

# Delivery

MMT defines the application-layer protocol and the payload format for efficient packetized delivery of MMT packages called as MMTP. The application-layer protocol MMT defines provides enhanced features for delivery of MMT packages compared to conventional application-layer protocols for multimedia delivery, such as calculating delivery jitter and the multiplexed delivery of multimedia components. A range of multimedia applications is expected to benefit from MMT delivery, including broadcast and multicast as well as the hybrid delivery and delivery of multiscreen presentations.

MMTP defines two different delivery types such as MPU and GFD modes. The MPU mode is designed for a real-time delivery service such as broadcasting, and the GFD mode is for a non-realtime file down load service. This delivery types and other information such as packet id, timestamp, packet sequence number and packet counter are specified in MMTP Packet header as being shown in Figure 5. The packet id tells which Asset a packet is belong to, and thus is used for multiplexing process in a server and is specified by a user on the basis of Asset ID and MMT Packet Table defined in MMT Signalling Message when a MPU file is generated. The timestamp tells the time of sending a packet based on NTP (Network Time Protocol), which can be used for calculating jitter brought by a network, and thus can manage a server buffer for a stable service. The packet sequence number tells the location of MPU file in a relevant Asset, which can be used for checking the loss of packets in a receiver.

This header information and relevant payload is combined into a MMTP packet. Especially, for the case of loosing MMTP packet header or packets, FEC information is added as a Tail of MMTP payload.

As being shown in Figure 5., the structure of MMTP payload is quite similar with RTP payload format. However, MMTP uses the same payload format regardless of media codec types as being contrast to RTP, which uses different payload formats depending on media codec types. MMT payload can contain various data such as one or more than one MPU files, fragmented MPU files, signalling messages and any other data. The types of payload data can be recognised by the DU header in the Payload.

Figure 5. MMTP packet structure

# Signalling messages

MMT signalling messages provide information for media consumption and delivery to the MMT client. These are binary formats carried by MMT payload; hence, they can support both unidirectional and bidirectional delivery environments. As being shown in Table 1., there are six MMT signalling messages such as Package Access (PA), Media Presentation Information (MPI), MMT Packet Table (MPT), Clock Relation Information (CRI), Device Capability Information (DCI) and Hypothetical Receiver Buffer Model (HRBM) defined. These messages has a table structure shown in Figure 6. Especially, PA message has the other signalling messages, and thus controls the other message handlings.

Table 1. Signalling message functions

| Message | Functions |
| --- | --- |
| PA | contains minimum information for faster access to MMT package in case of unidirectional repeated message transmission |
| MPI | contains a whole or a subset of a presentation information document such as Composition Information or HTML5. |
| MPT | contains all or a part of information required for a single Package consumption such as assets in a package |
| CRI | contains clock related information used for the mapping between an NTP timestamp and MPEG-2 System Time Clock |
| DCI | contains the required device capability information for Package consumption |
| HRBM | Provide buffer management information for a receiver |

Figure 6. MMTP packet structure

The role of these signalling messages is quite similar with PSI in MPEG-2 TS such as provision of component Asset information. However, MMT is designed for hybrid delivery network environment, and it is needed to prepare the component media to be located at various places and to be delivered over various networks. Thus, MMT signalling messages additionally provides temporal and spatial consumption information for consuming component media at the right time and place.

# Application Layer Forward Error Correction (AL-FEC)

MMT provides an AL-FEC mechanism for reliable delivery in IP network environment in MPEG-H Part 10. As being shown in Figure 7, after packetization, MMTP packets are passed to the MMT FEC scheme for protection. The MMT FEC scheme returns repair symbols with FEC payload ID. Then the repair symbols are delivered by MMT protocol with the MMTP packets. The FEC configuration information (i.e. FEC message) provides the identification of FEC encoded flow, the information specified FEC coding structure and FEC code. The FEC configuration information is delivered as AL-FEC message to MMT receiving entity for FEC operation.

Figure 7. Architecture for AL-FEC in MMT

MMT AL-FEC framework consists of FEC coding structure, Encoding symbol format, FEC source or repair packet format, FEC signalling and FEC message.

- FEC coding structure: it provides two kinds of FEC coding structures for multi-level construction of MMTP packets for layered or non-layered media data for an appropriate level of protection each Asset in an FEC source flow. As being shown in Figure 8 & 9, respectively Two-stage FEC supports two different QoSs (e.g. low latency for AV streaming and high robustness for File download) in a single FEC encoded flow and LA-FEC supports multiple protection of SVC contents.

Figure 8. Two-stages FEC coding structure                                      Figure 9. LA-FEC coding structure

- Encoding symbol format: it provides a format of an encoding symbol block which consists of a source symbol block and a repair symbol block. A source symbol block is generated from source packet block (i.e. a certain number of MMTP packets) according to one of three source symbol block generation (SSBG) modes. A repair symbol block is generated from the source symbol block by a given FEC code.

- FEC source or repair packet format: it provides FEC source packet and FEC repair packet to be delivered after FEC protection. In packet switching network environment delivering order of packets may not same as its receiving order, so MMTP packet after FEC protection (i.e. FEC source or repair packet) includes FEC payload ID to identify its location in encoding symbol block.

- FEC signalling: it provides interoperable FEC configuration information between MMT sending and receiving entities such as FEC message and FEC payload ID. FEC payload ID identifies the boundary of its associated encoding symbol block and the location of its MMTP packet within the encoding symbol block.

FEC message: it provides the configuration of FEC protected flows such as the configuration of FEC source and repair flows, FEC code parameters, FEC coding structure, SSBG mode, FEC protection window etc.. Especially, the configurations of FEC source and repair flows are indicated by the list of packet_ids which each packet_id is carried in MMTP packet header. So, any combination of packet_ids can be protected by FEC regardless of its data type. For example, when audio Asset, video Asset and signalling message are delivered. All of three or any two among three or any one can be protected by FEC.

# Composition Information

Composition Information in MMT (MMT-CI) is to provide information of how to show a MMT Package in terms of both spatial and temporal relationships among MMT Assets in a MMT Package, and thus, specifies how to describe and consume a MMT Package in terms of Assets relationships. The information about a MMT Package and the composition of MMT Assets in the MMT Package are described in MMT-CI, which is specified in MPEG-H Part 11. As an initial presentation of an MMT Package, the spatial relationship among Assets in the Package is described by HTML5. Also, Assets in the MMT Package may be delivered earlier or later in accordance with timing or event, and presented in the certain region. This temporal relationship is described by MMT-CI, which is based on XML language.

Since the temporal change of Assets such as video and audio elements defined in HTML5 is described by CI, Figure 10 shows the correspondence between HTML5 and CI. As being shown in Figure 7., the spatial location of Assets such as video, audio and image is defined by the div element in HTML5, and its temporal changed location of the Assets by divLocation element in CI. Thus, the temporal changes of Assets can be realised without reloading another HTML5 document. Also, the presentation time of their Assets are described by MediaSync element in CI.

Figure 10. Conceptual structure of the HTML5 and CI Document

Also, CI describes an MMT Package in terms of Area and View as shown in Figure 11.

Figure 11. Concept of View & Area

By using HTML5 and CI a temporal changes of an MMT Package can be realized as being shown in Figure 12. An initial presentation of an MMT Package is described by HTML5. The received HTML5 is parsed and produces a DOM tree, which shows Areas and its relevant Assets. The CI file is sequentially received when the spatial location of Areas or Assets in Areas are changed from the initial presentation. The CI file is also parsed into a DOM tree using XML Parser. And it can update the DOM nodes/elements to be saved in memory. The DOM nodes/elements are referenced using their reference identifiers such as refDiv element. The details of the DOM tree structure parsed from the CI file is explained in Figure 13.

Figure 12. CI Processing engine

As being explained in above, a MMT Package is a logical unit for a multimedia content, which is composed of media elementary streams and relevant data. And its basic file format for storage is specified by a combination of MPU files. Since MPU file is inherited from ISO BMFF structure, MPU file is also defined by box-based structure. MPU file format additionally defines two new boxes for MMT Package such as MPU box (‘mmpu’) and MMT Hint box (‘mmth’) as being shown in Figure 4. The ‘mmpu’ box contains a sequence number and Asset ID, which help to recognise which Asset a MPU file is belong to, and its location in Asset. The ‘mmth’ box provides the number of MFUs, media data property such as timed or non-timed data and media sample information. As being shown in Figure 4., a MPU file is inherited from ISO BMFF and thus, uses the boxes defined in ISO BMFF. A MPU file is identified by the brand name such as ‘mpuf’ in ‘ftyp’ box. Instead of ‘sidx’ box in ISO BMFF, a MPU file defines ‘mmpu’ box explained in the above. And ‘moov’ box followed by ‘mmpu’ box provides track and initial information of media data, where ‘mmth’ box newly defined for a MMT Package is added.

Figure 13. Example of  DOM Tree MPU structure in CI file

Also, the CI can be used when a View is changed in terms of Areas as being shown in Figure 14.

Figure 14. Example of the View-Change

In conclusion, MMT can make a spatial layout of Assets by using HTML 5 as an initial step for rendering, and use CI specification for temporal layout change of Assets. ructure

# References

[1].	ISO/IEC 13818-1, Information Technology – Generic Coding of Moving Pictures and Associated Audio Information: Part 1 Systems, Int’l Organization for Standardization, 2007.

[2].	ISO/IEC 14496-12, Information Technology — Coding of Audio-Visual Objects — Part 12 ISO Base Media File Format, Int’l Organization for Standardization, 2012.

[3].	I. Sodagar, “The MPEG-DASH Standard,” IEEE Multimedia, vol. 18, no. 4, 2011, pp. 62–67.

[4].	V. Jacobson and et. al., “Networking Named Content,” Comm. ACM, vol. 55, no. 1, 2012, pp. 117–124.

[5].	ISO/IEC JTC 1/SC 29/WG 11 MPEG, Study of ISO/IEC CD 23008-1 MPEG Media Transport, ISO/IEC JTC 1/SC 29/WG 11 N13089, MPEG, 2012; http://mpeg.chiariglione.org/working_documents/mpeg-h/mmt/mmt.zip.

[6]. A. Detti, M. Pomposini, N. Blefari-Melazzi, S. Salsano, “Supporting the Web with an Information Centric Network that Routes by Name”, Elsevier  Computer Networks, vol. 56, Issue 17,  p. 3705–3722

|  | Input |  |  | Output |  | Camera Calibration Registration |  | View Synthesis on Source Format |  | Distribution format |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application | Sparse in | Dense in | Topology in | Sparse out | Dense out | Once Offline | Per frame Online | Depth Map | Ray Space | Codec |
| 3DV = Desktop Auto-Stereoscopic Display | 2 or 3 |  | Linear | < 10 |  | X |  | X |  | 3DV Compatible |
| I) Handheld Camera Free Navigation |  | > 32 | Free |  | > 32 |  | X | X | X |  |
| II)Sports Free Navigation (fixed cams) | < 10 |  | 4xLin.& Cylindrical |  | > 32 | X |  | X |  |  |
| III) Super-MultiView Display HPO |  | > 32 | 1D Linear |  | > 100 to 300 < 1◦ per view | X |  | X | X |  |
| IV) Integral Photography, Full Parallax |  | > 50x50 | 2D array |  | > 50x50 | X |  |  | X |  |